| 3GPP TS 38.151 V17.10.0 (2024-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements for NR UEs (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	General	9

4.1	Relationship between minimum requirements and test requirements	9

4.2	Applicability of minimum requirements	9

4.3	Applicability rules for testing of SA and NSA UEs	9

5	Frequency bands	10

5.1	General	10

5.2	Operating bands	10

6	FR1 MIMO OTA requirements	13

6.1	General	13

6.1.1	Definition of MIMO throughput	13

6.1.2	Total Radiated Multi-antenna Sensitivity (TRMS)	13

6.2	Minimum requirement	14

7	FR2 MIMO OTA requirements	14

7.1	General	14

7.1.1	MIMO Average Spherical Coverage (MASC)	14

7.2	Minimum requirement	15

Annex A (normative): <FR1 Test methodology>	16

A.1	General	16

A.2	Multi-Probe Anechoic Chamber (MPAC)	16

A.2.1	System setup	16

A.2.2	Calibration procedure	17

A.2.3	Test procedure	17

A.2.4	Minimum Range Length	18

A.2.5	Preliminary MU budget of FR1 MPAC system	18

A.3	EUT positioning	19

Annex B (normative): <FR2 Test methodology>	22

B.1	General	22

B.2	FR2 3D Multi-Probe Anechoic Chamber (3D-MPAC)	22

B.2.1	System setup	22

B.2.2	Calibration procedure	24

B.2.3	Test procedure	24

B.2.4	Minimum Range Length	26

B.2.5	Preliminary MU of FR2 3D-MPAC system	27

B.2.6	Sample Device Orientations for Selected Test Points	28

B.3	EUT positioning	31

Annex C (normative): <FR1 Channel models and Validation procedure>	32

C.1	FR1 Channel models	32

C.2	FR1 Base Station beam configuration	33

C.3	FR1 Channel model validation	34

C.3.1	General	34

C.3.2	Power Delay Profile (PDP)	35

C.3.3	Doppler/Temporal correlation	38

C.3.4	Spatial correlation	41

C.3.5	Cross-polarization	48

C.3.6	Power validation	51

C.4	Validation Pass/fail limit	52

C.4.1	General	52

C.4.2	Pass/Fail Criteria of PDP	52

C.4.3	Pass/Fail Criteria of Doppler/Temporal correlation	52

C.4.4	Pass/Fail Criteria of Spatial correlation	54

C.4.5	Pass/Fail Criteria of Cross-polarization	57

C.4.6	Pass/Fail Criteria of Power validation	57

Annex D (normative): <FR2 Channel models and Validation procedure >	58

D.1	FR2 Channel models	58

D.2	FR2 Base Station beam configuration	58

D.3	FR2 Channel model validation	59

D.3.1	General	59

D.3.2	FR2 Power Delay Profile (PDP)	60

D.3.3	FR2 Doppler/Temporal correlation	61

D.3.4	FR2 PAS similarity percentage (PSP)	63

D.3.5	FR2 Cross-polarization	67

D.3.6	FR2 Power validation	68

D.4	Validation Pass/fail limit	69

D.4.1	General	69

D.4.2	Pass/Fail Criteria of PDP	69

D.4.3	Pass/Fail Criteria of Doppler/Temporal correlation	69

D.4.4	Pass/Fail Criteria of PSP	70

D.4.5	Pass/Fail Criteria of Cross-polarization	71

D.4.6	Pass/Fail Criteria of Power validation	71

Annex E (normative): <gNB configurations>	72

E.1	FR1 gNB configurations	72

E.2	FR2 gNB configurations	76

Annex F (normative): <Environmental requirements>	81

F.1	Scope	81

F.2	Ambient temperature	81

F.3	Operating voltage	81

Annex G (informative): Change history	82

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document establishes the Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements for NR UEs operating on frequency Range 1 and frequency rang 2, for NR standalone (SA) and NR non-standalone (NSA) operation mode. The corresponding test methodologies are also presented in the Annex of this Technical Specification.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 38.827: “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”.

[3]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone"

[4]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone"

[5]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios"

[6]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception"

[7]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment"

[8]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz"

[9]	F. Zhang, L. Hentilä, P. Kyösti and W. Fan, "Millimeter-wave New Radio Test Zone Validation for MIMO Over-the-air Testing," in IEEE Transactions on Antennas and Propagation, doi: 10.1109/TAP.2021.3111326.

[10]	3GPP TS 38.101-4: "NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements"

[11]	3GPP TS 38.551: "NR; User Equipment (UE) Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance; Conformance testing"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

FS: UE used in a free space configuration.

Handheld UE: A UE intended to be used in hand held scenario.

MIMO Average Spherical Coverage: An averaged sensitivity of best 18 FR2 MIMO OTA sensitivity values within the 3D sphere with constant-density points for PC3 device.

Primary mechanical mode: The mode that is most often used for a specific user scenario. Every terminal has at least one primary mechanical mode, if multiple modes are supported, different primary mechanical modes may be applicable for different user scenarios, e.g., different primary mechanical modes for Free Space and Hand phantom usage for the same UE.

PSP (PAS Similarity Percentage):  The similarity of the PAS produced by the OTA system and the reference PAS, which is presented by the Total Variation Distance (TVD) of power angular spectrum (PAS). PSP is defined as (1-TVD)*100%. PSP=100% denotes full similarity and PSP=0% denotes full dissimilarity.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

PRS-EPRE-MAX	Maximum downlink RS-EPRE

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AOA	Azimuth angle Of Arrival

AOD	Azimuth angle Of Departure

BS	Base Station

CDL	Clustered Delay Line

CW	Continuous Wave

DML	Data Mode Landscape

DMP	Data Mode Portrait

DMSU	Data Mode Screen Up

DUT	Device Under Test

EUT	Equipment Under Test

FR1	Frequency Range 1

FR2	Frequency Range 2

FS	Free Space

MASC	MIMO Average Spherical Coverage

MIMO	Multiple Input Multiple Output

MPAC	Multi-Probe Anechoic Chamber

NR	New Radio

NSA	Non-Standalone, a mode of operation where operation of an other radio is assisted with an other radio

OTA	Over The Air

PAS	Power Angular Spectrum

PDP	Power Delay Profile

PSP	PAS Similarity Percentage

RS-EPRE	Reference Signal-Energy Per Resource Element

SS 	System Simulator

SSS	Secondary Synchronization Signal

TRMS	Total Radiated Multi-antenna Sensitivity

UE	User Equipment

UMa	Urban Macro

UMi	Urban Micro

XPR	Cross-Polarization Ratio

ZOA	Zenith angle Of Arrival

ZOD	Zenith angle Of Departure

ZSA	Zenith angle Spread of Arrival

ZSD	Zenith angle Spread of Departure

# 4	General

## 4.1	Relationship between minimum requirements and test requirements

The Minimum Requirements given in this specification make no allowance for measurement uncertainty. The test specification in RAN5 will define test tolerances for FR1 and FR2 MIMO OTA. The test tolerances are used to relax the minimum requirements in this specification to create test requirements.

## 4.2	Applicability of minimum requirements

The MIMO OTA minimum requirements apply only to the primary mechanical mode of UE which is declared by the manufacturer if the UE can support multiple mechanical modes.

The minimum requirements apply only to the UE under normal environmental conditions specified in Annex F.

## 4.3	Applicability rules for testing of SA and NSA UEs

The applicability and test coverage rules for Non-Standalone (NSA) only capable UEs shall include the following:

-	For FR1 NSA (EN-DC) only capable UEs, testing is not required.

-	For FR2 NSA (EN-DC) only capable UEs, for each FR2 NR band supported by the device, test the UE in EN-DC mode using any one example configuration containing that NR band or configuration declaration decision tree as per recommended MIMO OTA test procedures in this specification.

The applicability and test coverage rules for Standalone (SA) and NSA (EN-DC) capable UEs shall include the following:

-	For FR1 UEs, for each NR band in a device, test the UE in Standalone Mode as per the TRMS test procedures in this specification. This shall also fulfil coverage for all EN-DC minimum performance requirements for that NR band and need not be retested in EN-DC mode.

-	For FR2 UEs, for each FR2 NR band supported by the device, test the UE in any of SA modes including FR2 only mode, FR1+FR2 NR-DC mode and FR1+FR2 NR-CA mode using any one example configuration containing that NR band. This shall fulfil coverage for FR2 MIMO OTA requirements for that NR band and need not be retested in EN-DC mode.

# 5	Frequency bands

## 5.1	General

NR MIMO OTA Requirements are defined separately for different frequency ranges (FR). The frequency ranges in which NR can operate according to this version of the specification are identified as described in Table 5.1-1.

Table 5.1-1: Definition of frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1 | 410 MHz – 7125 MHz |
| FR2 | 24250 MHz – 52600 MHz |

The present specification covers both FR1 and FR2 operating bands. For FR2, only FR2-1 bands are applicable.

## 5.2	Operating bands

NR is designed to operate in FR1 operating bands defined in TS 38.101-1 [3] and FR2 operating bands defined in TS 38.101-2 [4]. NSA band combinations are defined in TS 38.101-3 [5]. E-UTRA is designed to operate in operating bands defined in TS 36.101 [6].

For FR2 EN-DC capable UEs, principle of EN-DC band combinations selection for FR2 MIMO OTA testing is as following:

1)	Focus on the performance of the NR carrier and do not consider multiple permutations between different LTE bands and NR band under test, i.e., for each NR band, only select one EN-DC band combination.

2)	For UE supporting multiple EN-DC band combinations for the same NR band, consider only those EN-DC configurations which have no MSD impact on either LTE or NR.

Table 5.2-1: Measurement parameters for example inter-band EN-DC band combinations (LTE + FR2, two bands)

| EN-DC configuration | E-UTRA configurations | NR FR2 configurations |
| --- | --- | --- |
| DC_66A_n261A | Mid channel | Mid channel |

Table 5.2-2: Measurement parameters for example inter-band NR-DC band combinations (FR1 + FR2, two bands)

| NR-DC configuration | NR FR1 configurations | NR FR2 configurations |
| --- | --- | --- |
| DC_n66A_n261A | Mid channel | Mid channel |

Table 5.2-3: Measurement parameters for example inter-band NR-CA band combinations (FR1 + FR2, two bands)

| NR-CA configuration | NR FR1 configurations | NR FR2 configurations |
| --- | --- | --- |
| CA-n66A_n261A | Mid channel | Mid channel |

With the above basic principle and example band combination, the selection logic for testing is defined by the decision trees shown in Figure 5.2-1 and Figure 5.2-2.

Figure 5.2-1: Decision tree for FR1 MIMO OTA testing

Figure 5.2-2: Decision tree for FR2 MIMO OTA testing

# 6	FR1 MIMO OTA requirements

## 6.1	General

### 6.1.1	Definition of MIMO throughput

The MIMO throughput is defined here as the time-averaged number of correctly received transport blocks in a communication system running an application, where a Transport Block is defined in the reference measurement channel. From OTA perspective, this is also called MIMO OTA throughput. It will be used as the baseline figure of merit for FR1 and FR2 MIMO OTA testing.

The MIMO OTA throughput is measured at the top of physical layer of NR system under the use of FRC, the SS transmit fixed-size payload bits to the DUT. The DUT signals back either ACK or NACK to the SS. The SS then records the following:

-	Number of ACKs,

-	Number of NACKs, and

-	Number of DTX slots

Hence the MIMO (OTA) throughput can be calculated as

Where Transmitted TBS is the Transport Block Size transmitted by the SS, which is fixed for an FRC during the measurement period. MeasurementTime is the total composed of successful slots (ACK), unsuccessful slots (NACK) and DTX-symbols.

The time-averaging is to be taken over a time period sufficiently long to average out the variations due to the fading channel. Therefore, this is also called the average MIMO OTA throughput. The throughput should be measured at a time when eventual start-up transients in the system have evanesced.

### 6.1.2	Total Radiated Multi-antenna Sensitivity (TRMS)

The average TRMS of free space data mode portrait (FS DMP), free space data mode landscape (FS DML), and free space data mode screen up (FS DMSU), is defined as the FR1 MIMO OTA requirement. The averaging shall be done in linear scale for the TRMS results at these DUT positions, according to the formula:

where

Such that MODE is one of {FS_DMP, FS_DML, FS_DMSU}, and {PMODE,70,0, …, PMODE,70,11} are the measured sensitivity values at each azimuth position at the 70% throughput outage.

If 1 azimuth position does not result in a defined measured sensitivity at 70% throughput, SMODE,70 is calculated using the 11 measured sensitivities and the maximum downlink RS-EPRE PRS-EPRE-MAX (substitution approach) for the one missing result. PRS-EPRE-MAX is the maximum downlink RS-EPRE supported by the test system, and is defined as -80dBm/15kHz (or equivalent -77dBm/30kHz) for FR1 MIMO OTA.

The TRMS shall be measured at the mid channel as specified in TS 38.508-1 subclause 4.3.1 [7]. The average TRMS shall be lower than the average TRMS requirements specified in Clause 6.2.

The additional criterion in azimuthal orientations shall be met:

-	The EUT must meet 70% throughput in 11 of total 12 azimuthal orientations. If the EUT fails to meet this criterion even under maximum downlink power condition (i.e. PRS-EPRE-MAX), the EUT shall fail the FR1 MIMO OTA test.

-	The EUT must meet 90% throughput in 10 of total 12 azimuthal orientations. If the EUT fails to meet this criterion even under maximum downlink power condition (i.e. PRS-EPRE-MAX), the EUT shall fail the FR1 MIMO OTA test.

## 6.2	Minimum requirement

FR1 TRMS minimum performance requirements for NR handheld UEs operating on SA mode in free space and the primary mechanical mode for 70% DL throughput with the corresponding measurement configurations (i.e., channel model and gNB configuration) specified in Annex C.1 and Annex E.1 are defined in Table 6.2-1.

Table 6.2-1: FR1 TRMS minimum performance requirements for NR handheld UEs operating on SA mode in free space and the primary mechanical mode

| NR bands | Bandwidth (MHz) | MIMO layer | Channel model | Reference channel | TRMSaverage,70 |
| --- | --- | --- | --- | --- | --- |
| n28 | 10 | 2x2 | FR1 UMi CDL-C | R.PDSCH.1-3.1 FDD | TBD dBm/15kHz |
| n41 | 40 | 4x4 | FR1 UMa CDL-C | R.PDSCH.2-2.4 TDD | -93.3 dBm/30kHz |
| n78 | 40 | 4x4 | FR1 UMa CDL-C | R.PDSCH.2-2.4 TDD | -94.8 dBm/30kHz |
| n79 | 40 | 4x4 | FR1 UMa CDL-C | R.PDSCH.2-2.4 TDD | TBD dBm/30kHz |

# 7	FR2 MIMO OTA requirements

## 7.1	General

### 7.1.1	MIMO Average Spherical Coverage (MASC)

The MIMO Average Spherical Coverage (MASC) is the Figure of Merit of FR2 MIMO OTA requirement. FR2 MIMO OTA is measured with 36 constant-density points within the 3D sphere. The MASC is determined by the averaging of the best 18 sensitivity values for power class 3 UE. The averaging shall be done in linear scale for the MASC result according to the formula:

MASC70=10log18110P70,110+110P70,210+⋅⋅⋅+110P70,1810

Such that {P70,1, …, P70,18} are the best 18 sensitivity values from all the 36 constant density measurement points, as defined in Annex B.2.3.

The MASC shall be measured at the mid channel as specified in TS 38.508-1 subclause 4.3.1 [7]. The MASC shall be lower than the requirements specified in Clause 7.2.

For FR2 MIMO OTA, PRS-EPRE-MAX, i.e., the maximum downlink RS-EPRE supported by the test system, is defined as [-79.1dBm/120kHz].

If the number of test points where the UE can meet 70% maximum throughput outage even under maximum downlink power condition (i.e., [-79.1dBm/120kHz]) is less than [18], then UE fails the test.

Other criteria for FR2 are FFS.

## 7.2	Minimum requirement

FR2 MASC minimum performance requirements for power class 3 NR handheld UEs in free space and the primary mechanical mode for averaging of the best 18 sensitivity values for 70% DL throughput with the corresponding measurement configurations (i.e., channel model and gNB configuration) specified in Annex D.1 and Annex E.2 are defined in Table 7.2-1.

Table 7.2-1: FR2 MASC minimum performance requirements for NR handheld UEs in free space and the primary mechanical mode

| NR bands | Bandwidth (MHz) | MIMO layer | Channel model | Reference channel | MASC70 [dBm/120kHz] |
| --- | --- | --- | --- | --- | --- |
| n257 | 100 | 2x2 | FR2 UMi CDL-C | R.PDSCH.5-2.2 TDD | TBD |
| n258 | 100 | 2x2 | FR2 UMi CDL-C | R.PDSCH.5-2.2 TDD | TBD |
| n260 | 100 | 2x2 | FR2 UMi CDL-C | R.PDSCH.5-2.2 TDD | TBD |
| n261 | 100 | 2x2 | FR2 UMi CDL-C | R.PDSCH.5-2.2 TDD | TBD |

Annex A (normative):
<FR1 Test methodology>

# A.1	General

FR1 MIMO OTA requirement testing is based on UE-noise limited environmental condition, i.e., UE throughput characterized as a function of signal power incident to the DUT antennas.

The minimum test zone size for FR1 MIMO OTA test methods is 20cm. “Black-box” testing approach is adopted for NR MIMO OTA testing, the physical centre of the EUT shall be placed in the centre of the test zone, the EUT shall be completely contained within the minimum test zone size.

FR1 MIMO OTA requirement testing should be performed under primary mechanical mode. The primary mechanical mode for devices having multiple mechanical modes shall be declared by the manufacturers. Single primary mechanical mode for each device should be declared for MIMO OTA conformance testing.

# A.2	Multi-Probe Anechoic Chamber (MPAC)

## A.2.1	System setup

MPAC test method is the reference methodology for FR NR MIMO OTA testing. By arranging an array of antennas around the Equipment Under Test (EUT), a spatial distribution of angles of arrival in MPAC system may be simulated to expose the EUT to a near field environment that appears to have originated from a complex multipath far field environment.

As illustrated schematically in Figure A.2.1-1, signals propagate from the base station/communication tester to the EUT through a simulated multipath environment known as a spatial channel model, where appropriate channel impairments such as Doppler and fading are applied to each path prior to injecting all of the directional signals into the chamber simultaneously through the antenna array. The resulting field distribution in the test zone is then integrated by the EUT antenna(s) and processed by the receiver(s) just as it would do so in any non-simulated multipath environment. MPAC system with 16 uniformly-spaced dual-polarized probes is permitted for NR FR1 MIMO OTA testing.

Figure A.2.1-1: MPAC system layout for NR FR1 MIMO OTA testing

## A.2.2	Calibration procedure

The system needs to be calibrated by using a reference calibration antenna with known gain values in order to ensure that the downlink signal power is correct. In non-standalone (NSA) mode, the LTE link antenna provides a stable LTE signal without precise path loss or polarization control.

Unlike traditional TRP/TRS testing where the path loss corrections can all be applied as a post processing step to the measured data, the path loss for each probe in the MPAC system must be balanced at test time in order to generate the desired channel model environment within the test zone of the chamber. The imbalance of each path during testing would result in an alteration of the angular dependence of the channel model (i.e. varied characteristics of generated channel model) within the test zone of the chamber.

1.	Place a vertical reference dipole in the centre of the test zone, connected to a VNA port, with the other VNA port connected to the input of the channel emulator unit.

2.	Configure the channel emulator for bypass mode.

3.	Measure the response of each path from each vertical polarization probe to the reference antenna in the centre of test zone.

4.	Adjust the power on all vertical polarization branches of the channel emulator so that the powers received at the centre are equal.

5.	Repeat the steps 1 to 4 with the magnetic loop or horizontally polarized reference dipole instead, and adjust the horizontal polarization branches of the channel emulator.

6.	The worst-case path loss becomes the reference path loss of the entire system, this loss is used to compute the power in the centre of the test zone relative to the output power of the Base Station simulator. Besides, based on the reference path loss, the relative offset of each path loss shall be corrected.

Note 1:	Calibration based on other antennas, e.g., horn antennas is not precluded.

Note 2:	Calibrations based on other methods are not precluded if the following conditions are met:

-	A correction is applied to normalize the response from all probe elements including the vertical and horizontal responses.

-	The path loss (including over the air, amplifier gain) from the Base Station simulator to the centre of the test zone is determined using a gain reference antenna for at least one normalized probe.

-	All measurement uncertainty contributions related to the calibration method have been accounted for in the measurement uncertainty budget.

## A.2.3	Test procedure

Before throughput testing, the initial conditions shall be confirmed to reach the correct measurement state for each test case.

1.	Ensure environmental requirements of Annex F are met.

2.	Configure the test system according to Annex C, D and E for the applicable test case.

3.	Verify the implementation of the channel model as specified in Annex C.3.

4.	Position the UE in the chamber according to Annex A.3.

5.	Power on the UE.

6.	Set up the connection.

Note:	For step 3, the verification of the channel model implementation is usually performed once for each channel model as part of the laboratory accreditation process, and will remain valid as long as the setup and instruments remain unchanged. Otherwise the channel model validation may need to be performed prior to starting each throughput test.

For throughput testing, the following steps shall be followed in order to evaluate NR MIMO OTA performance of the DUT:

1.	Measure MIMO OTA throughput from one measurement point, the maximum downlink power PRS-EPRE-MAX is defined in Clause 6.1.2. MIMO OTA throughput is the minimum downlink signal power resulting in a pre-defined throughput value, i.e., 70% and 90% of the maximum theoretical throughput.  The downlink signal power step size shall be no more than 0.5 dB when RF power level is near the NR MIMO sensitivity level.

2.	Rotate the UE around vertical axis of the test system by 30 degrees and repeat from step 1 until one complete rotation has been measured i.e. 12 different UE azimuth rotations.

3.	Repeat the test from step 1 for each specified device orientation. A list of orientations is given in Annex A.3.

4.	The postprocessing method to calculate the average MIMO Throughput is defined in Clause 6.

Note:	For step 1 of throughput testing, the measurement is not needed to start from maximum downlink power each time. To save testing time, the starting downlink power can be set as a proper value (lower than maximum downlink power supported by test system) as long as all the throughput curve curves at 12 different UE azimuth rotations can reach at least 90% of the maximum theoretical throughput.

## A.2.4	Minimum Range Length

The minimum range length of FR1 MPAC system is defined as the distance from the centre of the test zone to the aperture of the measurement probes/antennas, as illustrated in Figure A.2.4-1.

Figure A.2.4-1: Illustration of range length definition of FR1 MPAC

The minimum range length for NR FR1 MPAC OTA systems with 20cm test zone size is 1.2m. While for MPAC systems, the far-field requirements do not have to apply, it was shown that the spatial correlation can be impacted significantly for distances below 1.2m.

## A.2.5	Preliminary MU budget of FR1 MPAC system

This clause defines the measurement uncertainty (MU) budget for FR1 MPAC system, as shown in Table A.2.5-1.

Table A.2.5-1: Measurement uncertainty budget for FR1 MPAC system

| UID | Description of uncertainty contribution | Example value (410 MHz <f ≤ 3 GHz) [dB] | Example value (3 GHz < f ≤ 7.125GHz) [dB] | Distribution of the probability | Std Uncertainty (410 MHz < f ≤ 3 GHz) [dB] | Std Uncertainty (3 GHz < f ≤ 7.125GHz) [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |
| 1 | Mismatch for measurement process | 0 | 0 | U-Shaped | 0 | 0 |
| 2 | Measure distance uncertainty | 0 | 0 | Normal | 0 | 0 |
| 3 | Quality of quiet zone | 0.5 | 0.5 | Actual | 0.5 | 0.5 |
| 4 | Base Station simulator | 1.3 | 1.3 | Normal | 0.65 | 0.65 |
| 5 | Channel Emulator - absolute output power - output signal stability - output stability with temperature | 1.5 0.5 0.4 | 1.5 0.5 0.4 | Actual (Normal - power; rect-stability) | 0.84 | 0.84 |
| 6 | Amplifier uncertainties | 0.7 | 0.7 | Rectangular | 0.40 | 0.40 |
| 7 | Random uncertainty | 0.4 | 0.4 | Normal | 0.20 | 0.20 |
| 8 | Throughput measurement: output level step resolution | 0 | 0 | Rectangular | 0 | 0 |
| 9 | Signal flatness | 0 | 0 | Normal | 0 | 0 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |
| 10 | Mismatch for calibration process - loopback cable path - system input path - reference antenna | 0.2 | 0.2 | U-Shaped | 0.14 | 0.14 |
| 11 | Reference antenna positioning misalignment | 0 | 0 | Normal | 0 | 0 |
| 12 | Quality of quiet zone | 0.5 | 0.5 | Rectangular | 0.29 | 0.20 |
| 13 | Total uncertainty of the Network Analyzer | 0.2 | 0.5 | Normal | 0.10 | 0.25 |
| 14 | Uncertainty of an absolute gain of the calibration antenna | 0.58 | 0.58 | Normal | 0.29 | 0.29 |
| 15 | Offset of the Phase Center of the Reference Antenna | 0 | 0 | Normal | 0 | 0 |
| Total Expanded Uncertainty, U, with 95 % Confidence Interval |  |  |  |  | 2.61 | 2.65 |

The detailed descriptions of each measurement uncertainty contributor are defined in Annex B. 2 in [11].

# A.3	EUT positioning

This Clause defines the measurement coordinate system for the NR MIMO OTA.

There are three sets of coordinate systems used for FR1 MIMO OTA measurement. They are aligned with the following instructions to avoid misalignment and ambiguity.

The coordinate system for UE positioning is the guidance to specify the UE orientation in Annex A of 3GPP TR 38.827. The default position of DUT aligned to coordinate system is illustrated in Figure A.3-1. For convenience, name the coordinate axis of DUT coordinate system as xDUT, yDUT and zDUT.

Figure A.3-1: Illustration of DUT coordinate system

The coordinate system for channel model, which is used to specify 2x2 and 4x4 channel models for FR1 MIMO OTA, is defined in Clause 7 of 3GPP TR 38.901. With similar approach, name the coordinate axis of channel model coordinate system as xCM, yCM and zCM. The channel model coordinate system is illustrated in Figure A.3-2.

Figure A.3-2: Illustration of channel model coordinate system

The coordinate system of anechoic chamber is also applied where the 16 measurement probes are evenly located in the azimuth plane. Considering chamber implementation flexibility, there are two possible probe placements together with chamber coordinate system, shown in Figure A.3-3. Naming the coordinate axis of chamber coordinate system as xchamber and ychamber.

Figure A.3-3: Illustration of AC chamber coordinate system

In order to avoid misalignment and ambiguity for FR1 MIMO OTA measurement, the aligned coordinate system is defined. Align xDUT , xCM and xchamber  together as xreference, align yDUT , yCM and ychamber  together as yreference and align zDUT and zCM together as zreference. The aligned coordinate system is illustrated in Figure A.3-4 where only one chamber implementation is shown below for instance.

Figure A.3-4: Illustration of the aligned coordinate system

For FR1 MIMO OTA, the DUT shall be tested under Free Space Data Mode Portrait (FS DMP), Free Space Data Mode Landscape (FS DML), and Free Space Data Mode Screen Up (FS DMSU), the DUT azimuthal rotation shall be performed over 360 degrees per orientation in 30 degree steps (12 total positions).

Annex B (normative):
<FR2 Test methodology>

# B.1	General

FR2 MIMO OTA requirement testing is based on UE-noise limited environmental condition, i.e., UE throughput characterized as a function of signal power incident to the DUT antennas.

The minimum test zone size for FR2 MIMO OTA 3D-MPAC system is 20cm. “Black-box” testing approach is adopted for NR MIMO OTA testing, the physical centre of the EUT shall be placed in the centre of the test zone, the EUT shall be completely contained within the minimum test zone size.

FR2 MIMO OTA requirement testing should be performed under primary mechanical mode. The primary mechanical mode for devices having multiple mechanical modes shall be declared by the manufacturers. Single primary mechanical mode for each device should be declared for MIMO OTA conformance testing.

# B.2	FR2 3D Multi-Probe Anechoic Chamber (3D-MPAC)

## B.2.1	System setup

The 3D MPAC test method is the reference methodology for FR2 NR MIMO OTA testing. By arranging an array of antennas around the Equipment Under Test (EUT), a spatial distribution of angles of arrival in the 3D MPAC system may be simulated to expose the EUT to a near field environment that appears to have originated from a complex multipath far field environment.

As illustrated schematically in Figure B.2.1-1, signals propagate from the base station/communication tester to the EUT through a simulated multipath environment known as a spatial channel model, where appropriate channel impairments such as Doppler and fading are applied to each path prior to injecting all of the directional signals into the chamber simultaneously through the probe array. The resulting field distribution in the test zone is then integrated by the EUT antenna(s) and processed by the receiver(s) just as it would do so in any non-simulated multipath environment. The 3D MPAC system with 6 dual-polarized probes (illustrated with black dots in Figure B.2.1-1) placed on a sector with minimum radius of 0.75m from the centre of the test zone is permitted for NR FR2 MIMO OTA testing.

Figure B.2.1-1: 3D MPAC system layout for NR FR2 MIMO OTA testing

The exact probe locations with respect to the OTA test system coordinate system are tabulated in Table B.2.1-1.

Table B.2.1-1. FR2 3D MPAC Probe Locations in OTA test system coordinate system

| Probe Number | Theta [deg] | Phi [deg] |
| --- | --- | --- |
| 1 | 0.0 | 0.0 |
| 2 | 11.2 | 116.7 |
| 3 | 20.6 | -104.3 |
| 4 | 20.6 | 104.3 |
| 5 | 20.6 | 75.7 |
| 6 | 30.0 | 90.0 |

The 3D MPAC probes in Table B.2.1-1 can be implemented using conventional millimetre-wave probes as well as IFF-based probes as long as the same probe configuration and same number of probes is used.

The channel model parameters and probe locations for channel model implementation are defined in a channel model coordinate system, which is illustrated in figure B.2.1-2. The channel model coordinate axes xCM, yCM, and zCM correspond to the OTA test system coordinate axes z, y, and -x, respectively.

Figure B.2.1-2: Channel Model Coordinate Axes in FR2 3D-MPAC system

The probe locations with respect to channel model coordinate axes are tabulated in table B.2.1-2.

Table B.2.1-2. FR2 3D MPAC Probe Locations in Channel Model Coordinate System

| Probe Number | Theta [deg] | Phi [deg] |
| --- | --- | --- |
| 1 | 90 | 0 |
| 2 | 85 | 10 |
| 3 | 85 | -20 |
| 4 | 85 | 20 |
| 5 | 95 | 20 |
| 6 | 90 | 30 |

The channel model rotations assumed for this probe configuration are tabulated in Table B.2.1-3.

Table B.2.1-3. Channel Model Rotations

| UMi CDL-C |  |
| --- | --- |
| Phi [deg] | Theta [deg] |
| 32 | 15.0 |

This channel model rotation assumes the relative orientations of BS and UE antennas displayed in Figure B.2.1-3, i.e., the DUT antenna is pointed towards the BS in channel model coordinate system.

In order to avoid positioning ambiguities, the turntable implementing the rotation in  shall match the intended DUT  for P0 Orientation 1 without the re-positioning approach, as defined in Annex A.3 in [2], applied. With the re-positioning approach applied, the relative orientation between the DUT and the probes for P0 Orientation 2 shall be the same the relative orientation between DUT and probes as for P0 Orientation 1.

Figure B.2.1-3: Relative orientations of BS and UE antennas.

Since the test points are uniformly spaced in 3D already, Table B.2.3-1, there is no need to adjust/rotate the DUT rotations by the channel model rotations.

## B.2.2	Calibration procedure

The system needs to be calibrated by using a reference calibration antenna with known gain values in order to ensure that the downlink signal power is correct. In non-standalone (NSA) mode, the LTE link antenna provides a stable LTE signal without precise path loss or polarization control.

The path loss for each probe in the 3D MPAC system must be calibrated at test time in order to generate the desired channel model environment within the test zone of the chamber. The imbalance of each path during testing would result in an alteration of the angular dependence of the channel model (i.e. varied characteristics of generated channel model) within the test zone of the chamber.

For the calibration measurement, the reference antenna is placed in the centre of the quiet zone, connected to a VNA port, with the other VNA port connected to the input of the channel emulator unit as illustrated schematically in Figure D.3.2-1. For each probe antenna, the reference antenna needs to be aligned in polarization, i.e.,  or , and direction with the probe antenna that corresponds to the respective path to be calibrated. For each calibration measurement, the channel emulator needs to be configured in bypass mode. The calibration process determines the composite loss, Lpath,pol, of the entire receiver chain path gains (measurement antenna, amplification) and losses (switches, combiners, cables, path loss, etc.). The calibration measurement is repeated for each measurement path (two orthogonal polarizations and each signal path).

## B.2.3	Test procedure

Before throughput testing, the initial conditions shall be confirmed to reach the correct measurement state for each test case.

1.	Ensure environmental requirements of Annex F are met.

2.	Configure the test system according to Annex E.2 and Annex D.1 for the applicable test case.

3.	Verify the implementation of the channel model as specified in Annex D.3.

4.	Position the UE in the chamber according to Annex B.3.

5.	Power on the UE.

6.	Set up the connection.

Note:	For step 3, the verification of the channel model implementation is usually performed once for each channel model as part of the laboratory accreditation process, and will remain valid as long as the setup and instruments remain unchanged. Otherwise the channel model validation may need to be performed prior to starting each throughput test.

For throughput testing, the following steps shall be followed in order to evaluate FR2 MIMO OTA performance of the DUT:

1.	Position the DUT in the default P0 alignment option (Orientation 1), as defined in Annex A.3 in TR38.827 [2].

2.	Measure MIMO OTA throughput, the maximum downlink power is TBD. MIMO OTA throughput is the minimum downlink signal power resulting in a pre-defined throughput value (70%) of the maximum theoretical throughput.  The downlink signal power step size shall be no more than 0.5 dB when RF power level is near the NR MIMO sensitivity level.

3.	Rotate the UE to the next test point. Table B.2.3-1 lists 36 evenly spaced test points determined using the charged particle approach and with test point #1 centred at (0,0).

4.	Repeat the test from step 2 for each specified test point. If the re-positioning concept is applied, the device needs to be positioned in P0 Orientation 2 (either option 1 or option 2).

5.	The postprocessing method and the performance metric are defined in Clause 7.1.

Note:	For step 2 of throughput testing, the measurement is not needed to start from maximum downlink power each time. To save testing time, the starting downlink power can be set as a proper value (lower than maximum downlink power supported by test system) as long as the throughput curve can reach at least 70% of the maximum theoretical throughput.

Table B.2.3-1. Evenly spaced FR2 test points with a constant density

| Test Point Number | Theta [deg] | Phi [deg] |
| --- | --- | --- |
| 1 | 0.0 | 0.0 |
| 2 | 33.5 | 139.7 |
| 3 | 33.9 | 49.7 |
| 4 | 35.5 | -142.9 |
| 5 | 35.5 | -76.9 |
| 6 | 37.6 | -17.2 |
| 7 | 52.3 | 94.7 |
| 8 | 56.9 | 175.7 |
| 9 | 62.5 | 20.4 |
| 10 | 63.7 | -99.8 |
| 11 | 67.1 | -55.0 |
| 12 | 69.3 | -139.5 |
| 13 | 69.5 | 130.1 |
| 14 | 70.3 | 60.8 |
| 15 | 72.1 | -16.2 |
| 16 | 88.7 | -167.5 |
| 17 | 88.7 | 98.5 |
| 18 | 89.3 | 157.0 |
| 19 | 93.9 | -78.9 |
| 20 | 94.6 | 31.6 |
| 21 | 95.3 | -115.6 |
| 22 | 99.6 | -38.3 |
| 23 | 103.8 | -1.1 |
| 24 | 104.4 | 66.3 |
| 25 | 110.1 | 127.5 |
| 26 | 115.1 | -145.6 |
| 27 | 120.8 | 171.9 |
| 28 | 125.3 | -60.7 |
| 29 | 128.2 | -104.1 |
| 30 | 128.8 | 91.3 |
| 31 | 129.9 | 35.8 |
| 32 | 136.0 | -13.4 |
| 33 | 145.8 | 138.1 |
| 34 | 150.2 | -153.3 |
| 35 | 160.6 | -67.4 |
| 36 | 161.7 | 59.1 |

## B.2.4	Minimum Range Length

The minimum range length of FR2 MPAC system is defined as the distance from the centre of the test zone to the aperture of the measurement probes/antennas, as illustrated in Figure B.2.4-1.

Figure B.2.4-1: Illustration of range length definition of FR2 3D-MPAC

The minimum range length for NR FR2 MPAC OTA systems with 20cm test zone size is 0.75m. It was shown that the PSP can be reduced significantly for distances below 0.75m.

## B.2.5	Preliminary MU of FR2 3D-MPAC system

This clause defines the Preliminary measurement uncertainty (MU) budget for FR2 3D-MPAC system, as shown in Table B.2.5-1.

Table B.2.5-1: Preliminary measurement uncertainty budget for FR2 3D-MPAC system

| UID | Description of uncertainty contribution | Example value (26.5GHz≤f≤29.5GHz) | Example value (37GHz ≤f≤40GHz) | Distribution of the probability | Std Uncertainty (26.5GHz≤f≤29.5GHz) [dB] | Std Uncertainty (37GHz ≤f≤40GHz) [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |
| 1 | Mismatch for measurement process | 1.30 | TBD | Actual | 1.30 | TBD |
| 2 | Measure distance uncertainty | 0.00 | TBD | Rectangular | 0.00 | TBD |
| 3 | Quality of quiet zone | 0.6 | TBD | Actual | 0.6 | TBD |
| 4 | Base Station simulator | 2.9 | TBD | Normal | 1.45 | TBD |
| 5 | Channel Emulator -absolute value -stability -linearity | 1.5dB 0.5dB 0.4dB | TBD | Actual (normal- power; rect-stability) | 0.84 | TBD |
| 6 | Amplifier uncertainties | 2.10 | TBD | Normal | 1.05 | TBD |
| 7 | Random uncertainty | 0.50 | TBD | Normal | 0.25 | TBD |
| 8 | Throughput measurement: output level step resolution | 0.25 | TBD | Rectangular | 0.14 | TBD |
| 9 | DUT sensitivity drift | 0.2 | 0.2 | Rectangular | 0.12 | TBD |
| 10 | Signal flatness | 0.00 | 0.00 | Normal | 0.00 | TBD |
| Stage 1: Calibration measurement |  |  |  |  |  |  |
| 11 | Mismatch for calibration process - loopback cable path - system input path - reference antenna | 0.00 | TBD | U-Shaped | 0.00 | TBD |
| 12 | Reference antenna positioning misalignment | 0.01 | TBD | Rectangular | 0.01 | TBD |
| 13 | Quality of quiet zone | 0.60 | TBD | Actual | 0.60 | TBD |
| 14 | Total uncertainty of the Network Analyzer | 0.73 | TBD | Normal | 0.37 | TBD |
| 15 | Uncertainty of an absolute gain of the calibration antenna | 0.60 | TBD | Normal | 0.30 | TBD |
| 16 | Offset of the Phase Center of the Reference Antenna | 0.00 | TBD | Rectangular | 0.00 | TBD |
| Total Expanded Uncertainty, U, with 95% Confidence Interval |  |  |  |  | 5.05 | TBD |

## B.2.6	Sample Device Orientations for Selected Test Points

Tables B.2.6-1 and B.2.6-2 include illustrations of device/positioner/probe configurations & orientations based on a sample MIMO OTA test system for various test points for Theta < 90° (Table B.2.6-1) and Theta > 90° (Table B.2.6-2), respectively.

Table B.2.6-1. Visualization of DUT Orientation of test points with Theta < 90°.

| Test Point Number | Theta [deg] | Phi [deg] | Sample System/DUT Illustration with P0 – Orientation 1 |
| --- | --- | --- | --- |
| 1 | 0.0 | 0.0 |  |
| 2 | 33.5 | 139.7 |  |
| 3 | 33.9 | 49.7 |  |
| … |  |  |  |
| 18 | 89.3 | 157.0 |  |

The illustrations in Table B.2.6-2 with Theta > 90° include the device and positioner orientations with the device placed in the P0 alignment option with Orientation 1 as well as Orientation 2 (Option 1 or Option 2) with the re-positioning approach as defined in Annex A.3 in [2].

Table B.2.6-2. Visualization of DUT Orientation of test points with Theta > 90°.

| Test Point Number | Theta [deg] | Phi [deg] | Sample System/DUT Illustration with P0 – Orientation 1 | Sample System/DUT Illustration with P0 – Orientation 2 |
| --- | --- | --- | --- | --- |
| 19 | 93.9 | -78.9 |  |  |
| … |  |  |  |  |
| 35 | 160.6 | -67.4 |  |  |
| 36 | 161.7 | 59.1 |  |  |

# B.3	EUT positioning

This Clause defines the measurement coordinate system for the NR MIMO OTA.

For FR2 MIMO OTA, the DUT shall be tested using a 3D scan. With the DUT positioned in the default P0 alignment option, as defined in Annex A.3 in [2], measurements on 36 evenly spaced test points with a constant density shall be performed.

Annex C (normative):
<FR1 Channel models and Validation procedure>

# C.1	FR1 Channel models

The following channel models are required for FR1 MIMO OTA measurement.

The generic models are Table C.1-1 FR1 UMi CDL-C and Table C.1-2 FR1 UMa CDL-C, which do not include base station antenna filtering. UMi CDL-C and UMa CDL-C are selected to define 2x2 and 4x4 MIMO OTA requirements, respectively.

Therefore, in addition, the BS beam filtering effect defined in Annex C.2 also apply when emulating the channel models.

Table C.1-1: Channel model parameters for UMi CDL-C

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -36.1891 | -122.2815 | 98.9242 | 90 |
| 2 | 20.99 | -1.25 | -21.5937 | 125.831 | 99.1915 | 90 |
| 3 | 22.19 | -3.4684 | -21.5937 | 125.831 | 99.1915 | 90 |
| 4 | 23.29 | -5.2294 | -21.5937 | 125.831 | 99.1915 | 90 |
| 5 | 21.76 | -2.5215 | -32.5709 | -143.6126 | 99.5732 | 90 |
| 6 | 63.66 | 0 | -7.4275 | 166.4003 | 99.306 | 90 |
| 7 | 64.48 | -2.2185 | -7.4275 | 166.4003 | 99.306 | 90 |
| 8 | 65.6 | -3.9794 | -7.4275 | 166.4003 | 99.306 | 90 |
| 9 | 65.84 | -7.4215 | 37.2175 | 73.8315 | 100.4513 | 90 |
| 10 | 79.35 | -7.1215 | -47.1664 | 82.7664 | 98.5616 | 90 |
| 11 | 82.13 | -10.7215 | 41.5716 | -79.6999 | 100.6231 | 90 |
| 12 | 93.36 | -11.1215 | -67.1585 | 66.9895 | 98.218 | 90 |
| 13 | 122.85 | -5.1215 | -41.5244 | 84.0543 | 100.165 | 90 |
| 14 | 130.83 | -6.8215 | -47.0437 | -96.2818 | 100.2604 | 90 |
| 15 | 217.04 | -8.7215 | -55.7519 | 94.8406 | 98.1225 | 90 |
| 16 | 271.05 | -13.2215 | 55.3698 | 53.9494 | 100.2604 | 90 |
| 17 | 425.89 | -13.9215 | 53.2234 | 16.0364 | 98.4852 | 90 |
| 18 | 460.03 | -13.9215 | 46.8456 | 32.2963 | 98.1416 | 90 |
| 19 | 549.02 | -15.8215 | -70.1021 | 18.2098 | 97.9698 | 90 |
| 20 | 560.77 | -17.1215 | 48.9306 | 37.0455 | 100.7376 | 90 |
| 21 | 630.65 | -16.0215 | 49.6052 | 33.7452 | 98.1225 | 90 |
| 22 | 663.74 | -15.7215 | 57.7615 | 29.801 | 98.1034 | 90 |
| 23 | 704.27 | -21.6215 | 65.6725 | 11.6092 | 100.4513 | 90 |
| 24 | 865.23 | -22.8215 | -83.5324 | 56.2837 | 100.9476 | 90 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.2265 | 12.0742 | 0.5726 | 0 | 7 |  |

Table C.1-2: Channel model parameters for UMa CDL-C

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -37.4195 | -96.4031 | 96.7645 | 90 |
| 2 | 76.6135 | -1.25 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 3 | 80.9935 | -3.4684 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 4 | 85.0085 | -5.2294 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 5 | 79.424 | -2.5215 | -33.5316 | -124.0196 | 100.8594 | 90 |
| 6 | 232.359 | 0 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 7 | 235.352 | -2.2185 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 8 | 239.44 | -3.9794 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 9 | 240.316 | -7.4215 | 41.4581 | 51.4188 | 106.3995 | 90 |
| 10 | 289.6275 | -7.1215 | -49.2149 | 62.9864 | 94.4761 | 90 |
| 11 | 299.7745 | -10.7215 | 46.1367 | -41.2744 | 107.4834 | 90 |
| 12 | 340.764 | -11.1215 | -70.697 | 42.5606 | 92.3083 | 90 |
| 13 | 448.4025 | -5.1215 | -43.1524 | 64.6538 | 104.5929 | 90 |
| 14 | 477.5295 | -6.8215 | -49.0831 | -62.7423 | 105.1951 | 90 |
| 15 | 792.196 | -8.7215 | -58.4403 | 78.6184 | 91.7061 | 90 |
| 16 | 989.3325 | -13.2215 | 60.9633 | 25.6781 | 105.1951 | 90 |
| 17 | 1554.4985 | -13.9215 | 58.6569 | -23.4063 | 93.9944 | 90 |
| 18 | 1679.1095 | -13.9215 | 51.8037 | -2.3553 | 91.8265 | 90 |
| 19 | 2003.923 | -15.8215 | -73.86 | -20.5926 | 90.7426 | 90 |
| 20 | 2046.8105 | -17.1215 | 54.0442 | 3.7933 | 108.2061 | 90 |
| 21 | 2301.8725 | -16.0215 | 54.7691 | -0.4794 | 91.7061 | 90 |
| 22 | 2422.651 | -15.7215 | 63.5332 | -5.5859 | 91.5856 | 90 |
| 23 | 2570.5855 | -21.6215 | 72.0338 | -29.1381 | 106.3995 | 90 |
| 24 | 3158.0895 | -22.8215 | -88.2912 | 28.7003 | 109.5309 | 90 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.3179 | 15.632 | 3.6131 | 0 | 7 |  |

# C.2	FR1 Base Station beam configuration

The emulated BS beam configuration to be used for all emulation of channel models defined in Annex C.1 is specified in this clause.

The Base Station beam configuration includes basic antenna parameters and beamforming characteristic. The basic BS antenna parameters is defined in Table C.2-1.

Table C.2-1: BS Antenna Parameters

| Parameter description | Symbol | Parameter value |  |
| --- | --- | --- | --- |
|  |  | FR1 ≤2.5GHz | FR1 >2.5GHz |
| Antenna panels in vertical dimension | Mg | 1 | 1 |
| Antenna panels in horizontal dimension | Ng | 1 | 1 |
| Elements per panel in vertical dimension | Me | 4 | 8 |
| Elements per panel in horizontal dimension | Ne | 8 | 8 |
| Number of polarizations per panel | P | 2 | 2 |
| Element spacing in horizontal dimension () | dH | 0.5 | 0.5 |
| Element spacing in vertical dimension () | dV | 0.5 | 0.5 |

Antenna element radiation patterns, including orientation of the element main polarization components as well as orientation of the antenna array are as in the example pattern in Table 7.3-1 of TR38.901. The antenna element has ±45 polarization components and the radiation pattern parameters are 3dB = 65, 3dB = 65, Amax = 30dB, SLAv = 30dB, GE,max =8 dBi.

The beamforming characteristic of the FR1 BS pattern is defined as follow:

-	A code book of 60 fixed beams is constructed to a grid of five elevation angles from –20 to +20 with 10 steps and 12 azimuth angles from –80 to +80 with ~15 steps；

-	For 4x4 MIMO OTA, two strongest transmitting beams are selected from the pre-defined beam grid based on their proximity to the strong clusters of each FR1 channel model. These beams should have different azimuth directions and can provide the highest receive power for UE.

-	For 2x2 MIMO OTA, 1 strongest transmitting beam is selected from the pre-defined beam grid which provides the highest received power for UE based on the FR1 channel model.

-	Beam directions for channels model given in Annex C.1 are:

-	For UMa CDL-C, the beam directions are:

-	Strongest beam: AoD: -7.27°, ZoD: 100°

-	2nd strongest beam: AoD: -21.82°, ZoD: 100°

-	For UMi CDL-C, the strongest beam direction is: AoD: -7.27°, ZoD: 100°.

# C.3	FR1 Channel model validation

## C.3.1	General

This clause describes the MIMO OTA validation measurements, in order to ensure that the channel models are correctly implemented and hence capable of generating the propagation environment, as described by the model, within the test zone.

The following measurements shall be done for FR1 channel model validation:

-	Power Delay Profile (PDP)

-	Doppler/Temporal correlation

-	Spatial correlation

-	Cross-polarization

-	Power validation

Frequencies to be used to test for channel model validation:

Table C.3.1-1: Frequencies for PDP, Doppler, Spatial correlation, and Cross-polarization validation

| NR FR1 Bands | Range | Test frequency (MHz) |
| --- | --- | --- |
| n71 | Low | 617MHz |
| n12, n17, n29, n14, n28 |  | 722MHz |
| n5, n8, n18, n20 |  | 836.5MHz |
| n50, n51, n74 | Mid | 1575.42MHz |
| n3, n2, n25, n39 |  | 1880MHz |
| n1, n34, n65 |  | 2132.5MHz |
| n7, n30, n41, n40, n38, [n90] |  | 2450MHz |
| n77,n78 | High | 3600MHz |
| n79 |  | [4700MHz] |

Table C.3.1-2: Frequencies for Power validation

| NR FR1 Bands | Range | Test frequency (centre frequency of each band) |
| --- | --- | --- |
| n71 | Low | n71 |
| n12, n17, n29, n14, n28 |  | n28 |
| n5, n8, n18, n20 |  | n8 |
| n50, n51, n74 | Mid | n51 |
| n3, n2, n25, n39 |  | n3 |
| n1, n34, n65 |  | n1 |
| n7, n30, n41, n40, n38, [n90] |  | n41 |
| n77, n78 | High | n78 |
| n79 |  | n79 |

## C.3.2	Power Delay Profile (PDP)

This measurement checks that the resulting power delay profile (PDP) is in-line with the PDP defined for the channel model. For PDP validation measurement, only Vertical validation is required.

The PDP measurement is performed with a Vector Network Analyser (VNA). An example setup for PDP measurement is shown in Figure C.3.2-1. VNA transmits frequency sweep signals thorough the NR MIMO OTA test system. A reference antenna (i.e dipole antenna), within the centre of the test zone, receives the signal and VNA analyses the frequency response of the system. A number of traces (frequency responses) are measured and recorded by VNA and analysed by a post processing SW, e.g., Matlab. Special care has to be taken into account to keep the fading conditions unchanged, i.e. frozen, during the short period of time of a single trace measurement. The fading may proceed only in between traces.

Figure C.3.2-1: Setup for PDP measurements

Step the emulation and store traces from VNA, i.e., run the emulation to CIR number 1, pause, measure VNA trace, run the emulation to CIR number 10, pause, measure VNA trace. Continue until 1000 VNA traces are measured.

VNA settings for PDP measurements are presented in Table C.3.2-1.

Table C.3.2-1: VNA settings for PDP measurements

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table C.3.1-1 |
| Span | MHz | 200 |
| Number of traces |  | 1000 |
| Number of points |  | 1101 |
| Averaging |  | 1 |

Channel model specification for PDP measurements is presented in Table C.3.2-2.

Table C.3.2-2: Channel model specification for PDP measurements

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table C.3.1-1 |
| Distance between traces in channel model | wavelength (Note) | > 2 |
| Channel model |  | As specified in Annex C.1 |
| NOTE: Time [s] = distance [] / MS speed [/s] MS speed [/s] = MS speed [m/s] / Speed of light [m/s] * Centre frequency [Hz] |  |  |

Method of measurement result analysis:

Measured VNA traces (frequency responses H(t,f)) are saved into a hard drive. The data is read into, e.g., Matlab. 
The analysis is performed by taking the Fourier transform of each FR. The resulting impulse responses h(t,) are averaged in power over time:

Finally the resulting PDP is shifted in delay, such that the first tap is on delay zero.

Beam-Specific Block Diagram

It is assumed that the beams are mapped to the inputs of the channel emulator as follows:

-	Beam 1: Input 1 and Input 2

-	Beam 2: Input 3 and Input 4 (CDL-C UMa only)

Figure C.3.2-2: Setup for Beam-Specific PDP measurements (Beam 1)

Figure C.3.2-3: Setup for Beam-Specific PDP measurements (Beam 2 CDL-C UMa only)

The detailed PDP reference value for CDL-C UMa and CDL-C UMi validation are defined in the following tables:

Table C.3.2-3: PDP Targets for CDL-C UMa beam 1 at ≤ 2.5 GHz

| Combined Clusters index | Delay(ns) | Power(dB) |
| --- | --- | --- |
| 1 | 0 | -34.3 |
| 2-5 | 80 | -19.5 |
| 6-8 | 235 | 0.0 |
| 9-10 | 290 | -33.0 |
| 11 | 450 | -35.8 |
| 12 | 480 | -34.0 |

Table C.3.2-4: PDP Targets for CDL-C UMa beam 2 at ≤ 2.5 GHz

| Combined Clusters index | Delay(ns) | Power(dB) |
| --- | --- | --- |
| 1 | 0 | -27.9 |
| 2-5 | 80 | 0.0 |
| 6-8 | 235 | -18.4 |
| 9-10 | 290 | -27.8 |
| 11 | 450 | -27.9 |
| 12 | 480 | -28.0 |

Table C.3.2-5: PDP Targets for CDL-C UMa beam 1 at > 2.5 GHz

| Combined Clusters index | Delay(ns) | Power(dB) |
| --- | --- | --- |
| 1 | 0 | -34.2 |
| 2-5 | 80 | -19.3 |
| 6-8 | 235 | 0.0 |
| 9 | 290 | -34.7 |
| 10 | 450 | -35.8 |
| 11 | 480 | -34.7 |

Table C.3.2-6: PDP Targets for CDL-C UMa beam 2 at > 2.5 GHz

| Combined Clusters index | delay(ns) | power(dB) |
| --- | --- | --- |
| 1 | 0 | -27.8 |
| 2-5 | 80 | 0.0 |
| 6-8 | 235 | -18.3 |
| 9-10 | 290 | -28.9 |
| 11 | 450 | -28.1 |
| 12 | 480 | -28.8 |

Table C.3.2-7: PDP Targets for CDL-C UMi at ≤ 2.5 GHz and > 2.5 GHz

| Combined Clusters index | Delay(ns) | Power(dB) |
| --- | --- | --- |
| 1 | 0 | -30.7 |
| 2-5 | 20 | -19.2 |
| 6-10 | 65 | 0 |
| 11-12 | 130 | -31.4 |

## C.3.3	Doppler/Temporal correlation

This measurement checks the Doppler/temporal correlation. For Doppler/Temporal correlation validation measurement, only Vertical validation is required.

The Doppler spectrum is measured with a spectrum analyser as shown in Figure C.3.3-1. In this case a signal generator transmits CW signal through the NR MIMO OTA test system. The signal is received by a test antenna within the test area. Finally, the signal is analysed by a spectrum analyser and the measured spectrum is compared to the target spectrum. This setup can be used to measure Doppler Spectrum of the Channel models defined in Annex C.1.

Method of measurement:

Figure C.3.3-1: Setup for Doppler measurements

Sine wave (CW, carrier wave) signal is transmitted from the signal generator. The signal is connected from the signal generator to fading emulator via cables. The fading emulator output signals are connected to power amplifier boxes via cables. The amplified signals are then transferred via cables to the probe antennas. The probe antennas radiate the signals over the air to the test antenna. The Doppler spectrum is measured by the spectrum analyser and the trace is saved.

Signal generator settings for Doppler/Temporal correlation measurements are presented in Table C.3.3-1.

Table C.3.3-1: Signal generator settings for Doppler/Temporal correlation measurements

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table C.3.1-1 |
| Modulation |  | OFF |

Spectrum analyser settings for Doppler/Temporal correlation measurements are presented in Table C.3.3-2.

Table C.3.3-2: Spectrum analyser settings for Doppler/Temporal correlation measurements

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table C.3.1-1 |
| Minimum Span | Hz | 4 kHz |
| RBW | Hz | 1 |
| VBW | Hz | 1 |
| Number of points |  | 16001 |
| Averaging |  | 100 |

Channel model specification for Doppler/Temporal correlation measurements is presented in Table C.3.3-3.

Table C.3.3-3: Channel model specification for Doppler/Temporal correlation measurements

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table C.3.1-1 |
| Channel model |  | As specified in Annex C.1 |
| Mobile speed | km/h | 100 |

Method of measurement result analysis: Measurement data file (Doppler power spectrum) is saved into hard drive. The data is read into, e.g., Matlab. The analysis is performed by taking the Fourier transformation of the Doppler spectrum. The resulting temporal correlation function   is normalized such that max(abs(Rt(∆t)))=1. Then the function values left from the maximum i.e., the negative lags are cut out. Further on the function values after five periods are cut out.

Time Domain Alternate Method

Time domain techniques can also be used to validate the tempoal correlation. The temporal correlation validation measurement setup is illustrated in Figure C.3.3-2. In this case a Signal generator transmits a CW signal through the MIMO test system. The signal is received by a test antenna within the test area. Finally, the signal is collected by a signal analyser and the measured signal is stored as IQ data format for postprocessing.

Figure C.3.3-2: Setup for Doppler measurements based on time domain technique

The time domain doppler spectrum is measured by the signal analyser and the trace in IQ format is saved. Follow the same procedure to post process the data and calculate the temporal correlation curve. Data recording is synchronized with the channel emulator trigger. Depending on CE implementation, the trigger direction between SAN and CE needs to be adjusted, i.e., from SAN to CE or from CE to SAN.

The settings for the signal analyser are presented in Table C.3.3-4.

Table C.3.3-4: Signal Analyser Settings

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table C.3.1-1 |
| Sampling | Hz | At least 15 times bigger than the max Doppler spread (fd=v/λ) |
| Observation time | s | At least 16s. Channel Model length should be the same or greater than the observation time. |

Beam-Specific Block Diagram

It is assumed that the beams are mapped to the inputs of the channel emulator as follows:

-	Beam 1: Input 1 and Input 2

-	Beam 2: Input 3 and Input 4 (CDL-C UMa only)

Figure C.3.3-3: Setup for Beam-Specific Doppler measurements (Beam 1)

Figure C.3.3-4: Setup for Beam-Specific Doppler measurements (Beam 2 CDL-C UMa only)

The detailed Temporal correlation reference value for CDL-C UMa and CDL-C UMi channel model validation is defined in Table C.3.3-5.

Table C.3.3-5: Autocorrelation Targets

| Lambda Separation | CDL-C UMa beam 1 at ≤ 2.5 GHz | CDL-C UMa beam 2 at ≤ 2.5 GHz | CDL-C UMa beam 1 at > 2.5 GHz | CDL-C UMa beam 2 at > 2.5 GHz | CDL-C UMi beam 1 at ≤ 2.5 GHz | CDL-C UMi beam 1 at > 2.5 GHz |
| --- | --- | --- | --- | --- | --- | --- |
| 0.0 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 0.1 | 0.986 | 0.974 | 0.985 | 0.973 | 0.995 | 0.995 |
| 0.2 | 0.945 | 0.907 | 0.942 | 0.904 | 0.982 | 0.982 |
| 0.3 | 0.882 | 0.832 | 0.874 | 0.825 | 0.962 | 0.961 |
| 0.4 | 0.801 | 0.776 | 0.787 | 0.765 | 0.936 | 0.935 |
| 0.5 | 0.709 | 0.738 | 0.689 | 0.723 | 0.906 | 0.905 |
| 0.6 | 0.613 | 0.695 | 0.586 | 0.675 | 0.872 | 0.871 |
| 0.7 | 0.518 | 0.623 | 0.486 | 0.599 | 0.834 | 0.834 |
| 0.8 | 0.430 | 0.525 | 0.394 | 0.496 | 0.793 | 0.793 |
| 0.9 | 0.353 | 0.426 | 0.315 | 0.391 | 0.750 | 0.749 |
| 1.0 | 0.289 | 0.360 | 0.252 | 0.319 | 0.705 | 0.704 |
| 1.1 | 0.240 | 0.335 | 0.206 | 0.290 | 0.659 | 0.658 |
| 1.2 | 0.204 | 0.320 | 0.174 | 0.273 | 0.614 | 0.612 |
| 1.3 | 0.181 | 0.287 | 0.154 | 0.239 | 0.569 | 0.568 |
| 1.4 | 0.167 | 0.233 | 0.143 | 0.185 | 0.527 | 0.525 |
| 1.5 | 0.159 | 0.176 | 0.137 | 0.129 | 0.487 | 0.485 |
| 1.6 | 0.155 | 0.141 | 0.135 | 0.096 | 0.450 | 0.448 |
| 1.7 | 0.153 | 0.135 | 0.134 | 0.092 | 0.417 | 0.415 |
| 1.8 | 0.150 | 0.137 | 0.134 | 0.095 | 0.387 | 0.385 |
| 1.9 | 0.144 | 0.132 | 0.130 | 0.093 | 0.361 | 0.358 |
| 2.0 | 0.135 | 0.117 | 0.122 | 0.089 | 0.337 | 0.335 |
| 2.1 | 0.121 | 0.097 | 0.109 | 0.086 | 0.316 | 0.313 |
| 2.2 | 0.105 | 0.076 | 0.090 | 0.076 | 0.296 | 0.293 |
| 2.3 | 0.085 | 0.062 | 0.069 | 0.064 | 0.277 | 0.274 |
| 2.4 | 0.065 | 0.071 | 0.047 | 0.067 | 0.258 | 0.255 |
| 2.5 | 0.048 | 0.090 | 0.031 | 0.088 | 0.239 | 0.236 |
| 2.6 | 0.039 | 0.099 | 0.033 | 0.103 | 0.219 | 0.216 |
| 2.7 | 0.038 | 0.088 | 0.046 | 0.099 | 0.198 | 0.195 |
| 2.8 | 0.042 | 0.058 | 0.057 | 0.073 | 0.178 | 0.175 |
| 2.9 | 0.043 | 0.037 | 0.062 | 0.038 | 0.158 | 0.154 |
| 3.0 | 0.041 | 0.067 | 0.060 | 0.045 | 0.138 | 0.135 |
| 3.1 | 0.037 | 0.103 | 0.050 | 0.080 | 0.120 | 0.116 |
| 3.2 | 0.036 | 0.120 | 0.036 | 0.100 | 0.103 | 0.100 |
| 3.3 | 0.044 | 0.115 | 0.019 | 0.099 | 0.089 | 0.085 |
| 3.4 | 0.056 | 0.097 | 0.010 | 0.081 | 0.076 | 0.073 |
| 3.5 | 0.068 | 0.082 | 0.019 | 0.061 | 0.066 | 0.063 |
| 3.6 | 0.075 | 0.083 | 0.029 | 0.053 | 0.057 | 0.055 |
| 3.7 | 0.076 | 0.090 | 0.034 | 0.060 | 0.051 | 0.049 |
| 3.8 | 0.068 | 0.089 | 0.036 | 0.073 | 0.046 | 0.044 |
| 3.9 | 0.051 | 0.079 | 0.044 | 0.091 | 0.042 | 0.041 |
| 4.0 | 0.027 | 0.068 | 0.062 | 0.111 | 0.039 | 0.038 |
| 4.1 | 0.007 | 0.063 | 0.090 | 0.127 | 0.037 | 0.035 |
| 4.2 | 0.036 | 0.062 | 0.123 | 0.133 | 0.036 | 0.034 |
| 4.3 | 0.067 | 0.057 | 0.155 | 0.129 | 0.038 | 0.036 |
| 4.4 | 0.093 | 0.052 | 0.182 | 0.126 | 0.043 | 0.040 |
| 4.5 | 0.111 | 0.055 | 0.200 | 0.131 | 0.051 | 0.048 |
| 4.6 | 0.119 | 0.063 | 0.207 | 0.139 | 0.061 | 0.058 |
| 4.7 | 0.116 | 0.066 | 0.200 | 0.138 | 0.073 | 0.070 |
| 4.8 | 0.101 | 0.058 | 0.180 | 0.117 | 0.085 | 0.082 |
| 4.9 | 0.078 | 0.047 | 0.149 | 0.079 | 0.096 | 0.093 |
| 5.0 | 0.051 | 0.048 | 0.110 | 0.034 | 0.107 | 0.104 |

## C.3.4	Spatial correlation

This measurement checks whether the measured correlation curve follows the theoretical curve. For spatial correlation validation measurement, only Vertical validation measurement is required. Spatial correlation validation is only adopted for FR1 MIMO OTA.

The spatial correlation validation measurement setup is illustrated in Figure C.3.4-1. The network analyser transmits signals through the fading emulator and probes. The 16 probes radiate the signals within the anechoic chamber and a receiving test antenna is placed within the test zone. The test antenna is attached to a positioner that can move the antenna to pre-defined spatial locations on a fixed radius from the centre of the quiet zone. The received signal is measured with the network analyser.

The measurement and analysis procedure are as follows:

Set the target channel model to fading emulator.

1. For each position of the test antenna in the test zone, step & pause the emulator to different time instances. Measure the frequency responses  for all stepped channel snapshots , where the interval between frequency and time samples is  and , respectively. The number of channel snapshots N and frequency samples M should be sufficiently high so that the matrix can be estimated reliably.

2. Move the measurement antenna with a positioner to another location k and repeat step 2 to record frequency responses  of all stepped channel snapshots.

3. Repeat step 3 to record frequency responses at all  spatial sample points.

4. Stack measured time and frequency samples to a vector and calculate correlation between the first spatial sample point (i.e. k=1) and other spatial points

5.

6. Take the theoretical reference spatial correlation of the corresponding spatial sample points. Plot both the measured and theoretical curves.

7. Calculate the weighted RMS correlation error between the measured and the reference.

Figure C.3.4-1: Configuration for spatial correlation validation

Beam-Specific Block Diagram

It is assumed that the beams are mapped to the inputs of the channel emulator as follows:

-	Beam 1: Input 1 and Input 2

-	Beam 2: Input 3 and Input 4 (CDL-C UMa only)

Figure C.3.4-2: Configuration for spatial correlation validation (CDL-C UMi)

Figure C.3.4-3: Configuration for spatial correlation validation (CDL-C UMa)

Time and frequency samples

The number of temporal snapshots N and frequency samples M is shown in Table C.3.4-1. The channel model specification is presented in Table C.3.4-2.

Table C.3.4-1: VNA settings for spatial correlation

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table C.3.1-1 |
| Span | MHz | 0 (Note 2) |
| RF output level | dBm | -15 |
| Number of traces |  | 1000 |
| Distance between traces in channel model | Wavelength (Note 1) | > 2 |
| Number of points |  | 1 (or the smallest possible) (Note 2) |
| Averaging |  | 1 |
| NOTE1: Time in seconds = distance [] / MS speed [/s] MS speed [/s] = MS speed [m /s] / Speed of light [m/s] * Center frequency [Hz] NOTE 2: Span and number of points may be increased to estimate reliably |  |  |

Table C.3.4-2: Channel model specification

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table C.3.1-1 |
| Channel model samples | Wavelength | > 2000 |
| Channel model |  | As specified in Annex C.1 |
| Mobile speed | km/h | 30 |

Spatial samples

The spatial samples for the correlation validation measurement are on the circumference of the quiet zone, as illustrated in Figure C.3.4-4. The test zone is a circle with 20 cm diameter in the horizontal plane. The reference point (denoted by a red marker) is in AoA 270. The mean AoAs of the CDL-C UMi and CDL-C UMa models are slightly different, but the underlying geometry for the CDL model indicates that the mean AoA (or assumed LoS direction) of the model is 180°. The reference point orientation of the validation measurement is proposed to be with 90° offset to the channel model reference AoA to enable accurate sampling of the main lobe of the spatial correlation curve. The reference point orientation must be defined in the channel model coordinate system instead of the chamber/probe coordinate system to enable optimization of OTA model implementation to achieve better alignment with the cluster AoAs and probe directions. In order to have spatial samples that yield reasonable measurement times and adequately capture the main lobe of the correlation curve, a non-uniform sampling is used where the first quadrant i.e., 270-180, is sampled with dense sampling compared to the rest of the circle. The spacing of the spatial samples is summarized in Table C.3.4-3 for test frequencies less than 1800 MHz and equal to or greater than 1800 MHz.

Table C.3.4-3: Spacing of Spatial Samples

| Test Frequencies [MHz] | First quadrant of test zone circumference (270o-180o) | Remaining quadrants |
| --- | --- | --- |
| 617, 722, 836.5 1575.42 | /15 | /4 |
| 1800, 2132.50, 2450, 3600, 4700 | /10 | /2 |

Figure C.3.4-4: Test zone interpretation with Angle of Arrival reference orientation

Figure C.3.4-5: Spatial sampling for spatial correlation validation measurement for test frequencies less than and equal to or greater than 1800 MHz: 617 MHz spatial sampling (left) and 4700 MHz spatial sampling (right).

Reference Spatial Correlation Curves

The spatial correlation validation reference curves are tabulated in tables C.3.4-4 and C.3.4-5 for CDL-C UMi and CDL-C UMa, respectively, for a vertically polarized MPAC OTA setup with 16 uniformly spaced probes.

Table C.3.4-4: Spatial correlation reference curves for CDL-C UMi model for a vertically polarized MPAC OTA setup with 16 uniformly spaced probes at FR1 test frequencies

| Azim [] | ¦¦ beam 1 | Azim [] | ¦¦ beam 1 | Azim [] | ¦¦ beam 1 | Azim [] | ¦¦ beam 1 | Azim [] | ¦¦ beam 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 617 MHz |  | 722 MHz |  | 836.5 MHz |  | 1575.42 MHz |  | 1800 MHz |  |
| 270.0 | 1.00 | 270.0 | 1.00 | 270.0 | 1.00 | 270.0 | 1.00 | 270.0 | 1.00 |
| 251.4 | 1.00 | 254.1 | 1.00 | 256.3 | 1.00 | 262.7 | 1.00 | 260.9 | 1.00 |
| 232.9 | 1.00 | 238.3 | 1.00 | 242.6 | 1.00 | 255.5 | 1.00 | 251.7 | 1.00 |
| 214.3 | 0.99 | 222.4 | 1.00 | 228.9 | 1.00 | 248.2 | 1.00 | 242.6 | 0.99 |
| 195.8 | 0.99 | 206.6 | 0.99 | 215.2 | 0.99 | 240.9 | 0.99 | 233.5 | 0.99 |
| 110.4 | 0.87 | 190.7 | 0.98 | 201.6 | 0.98 | 233.7 | 0.99 | 224.3 | 0.98 |
| 40.8 | 0.87 | 120.5 | 0.84 | 187.9 | 0.96 | 226.4 | 0.99 | 215.2 | 0.97 |
| 331.2 | 0.98 | 61.1 | 0.80 | 128.7 | 0.82 | 219.1 | 0.98 | 206.0 | 0.95 |
|  |  | 1.6 | 0.91 | 77.3 | 0.73 | 211.9 | 0.97 | 196.9 | 0.92 |
|  |  | 302.1 | 0.99 | 26.0 | 0.81 | 204.6 | 0.96 | 187.8 | 0.87 |
|  |  |  |  | 334.7 | 0.95 | 197.3 | 0.94 | 134.3 | 0.39 |
|  |  |  |  | 283.3 | 1.00 | 190.0 | 0.91 | 88.6 | 0.15 |
|  |  |  |  |  |  | 182.8 | 0.87 | 43.0 | 0.24 |
|  |  |  |  |  |  | 152.7 | 0.66 | 357.3 | 0.62 |
|  |  |  |  |  |  | 125.5 | 0.44 | 311.6 | 0.94 |
|  |  |  |  |  |  | 98.2 | 0.30 |  |  |
|  |  |  |  |  |  | 71.0 | 0.28 |  |  |
|  |  |  |  |  |  | 43.7 | 0.37 |  |  |
|  |  |  |  |  |  | 16.5 | 0.54 |  |  |
|  |  |  |  |  |  | 349.2 | 0.75 |  |  |
|  |  |  |  |  |  | 321.9 | 0.91 |  |  |
|  |  |  |  |  |  | 294.7 | 0.99 |  |  |
| Azim [] | ¦¦ beam 1 | Azim [] | ¦¦ beam 1 | Azim [] | ¦¦ beam 1 | Azim [] | ¦¦ beam 1 |
| 2132.5 MHz |  | 2450 MHz |  | 3600 MHz |  | 4700 MHz |  |
| 270.0 | 1.00 | 270.0 | 1.00 | 270.0 | 1.00 | 270.0 | 1.00 |
| 261.9 | 1.00 | 263.0 | 1.00 | 265.2 | 1.00 | 266.3 | 1.00 |
| 253.9 | 1.00 | 256.0 | 1.00 | 260.5 | 1.00 | 262.7 | 1.00 |
| 245.8 | 0.99 | 249.0 | 0.99 | 255.7 | 0.99 | 259.0 | 0.99 |
| 237.8 | 0.99 | 242.0 | 0.99 | 250.9 | 0.99 | 255.4 | 0.99 |
| 229.7 | 0.98 | 234.9 | 0.99 | 246.1 | 0.99 | 251.7 | 0.99 |
| 221.7 | 0.97 | 227.9 | 0.98 | 241.4 | 0.98 | 248.1 | 0.98 |
| 213.6 | 0.96 | 220.9 | 0.97 | 236.6 | 0.98 | 244.4 | 0.98 |
| 205.6 | 0.93 | 213.9 | 0.95 | 231.8 | 0.97 | 240.8 | 0.98 |
| 197.5 | 0.89 | 206.9 | 0.92 | 227.1 | 0.97 | 237.1 | 0.97 |
| 189.5 | 0.84 | 199.9 | 0.88 | 222.3 | 0.95 | 233.5 | 0.97 |
| 181.4 | 0.77 | 192.9 | 0.83 | 217.5 | 0.93 | 229.8 | 0.96 |
| 139.7 | 0.27 | 185.9 | 0.76 | 212.7 | 0.90 | 226.1 | 0.95 |
| 99.5 | 0.14 | 144.9 | 0.19 | 208.0 | 0.86 | 222.5 | 0.93 |
| 59.2 | 0.14 | 109.9 | 0.26 | 203.2 | 0.81 | 218.8 | 0.91 |
| 18.9 | 0.26 | 74.8 | 0.37 | 198.4 | 0.75 | 215.2 | 0.87 |
| 338.6 | 0.71 | 39.8 | 0.19 | 193.7 | 0.68 | 211.5 | 0.83 |
| 298.4 | 0.97 | 4.7 | 0.29 | 188.9 | 0.59 | 207.9 | 0.78 |
|  |  | 329.7 | 0.74 | 184.1 | 0.49 | 204.2 | 0.72 |
|  |  | 294.6 | 0.97 | 156.1 | 0.23 | 200.6 | 0.64 |
|  |  |  |  | 132.3 | 0.62 | 196.9 | 0.56 |
|  |  |  |  | 108.4 | 0.85 | 193.3 | 0.47 |
|  |  |  |  | 84.6 | 0.93 | 189.6 | 0.37 |
|  |  |  |  | 60.7 | 0.92 | 185.9 | 0.27 |
|  |  |  |  | 36.9 | 0.79 | 182.3 | 0.18 |
|  |  |  |  | 13.0 | 0.42 | 161.7 | 0.51 |
|  |  |  |  | 349.1 | 0.15 | 143.5 | 0.83 |
|  |  |  |  | 325.3 | 0.60 | 125.2 | 0.95 |
|  |  |  |  | 301.4 | 0.90 | 106.9 | 0.89 |
|  |  |  |  | 277.6 | 1.00 | 88.6 | 0.80 |
|  |  |  |  |  |  | 70.4 | 0.78 |
|  |  |  |  |  |  | 52.1 | 0.88 |
|  |  |  |  |  |  | 33.8 | 0.98 |
|  |  |  |  |  |  | 15.5 | 0.91 |
|  |  |  |  |  |  | 357.3 | 0.53 |
|  |  |  |  |  |  | 339.0 | 0.09 |
|  |  |  |  |  |  | 320.7 | 0.50 |
|  |  |  |  |  |  | 302.4 | 0.82 |
|  |  |  |  |  |  | 284.2 | 0.97 |

Table C.3.4-5: Spatial correlation reference curves for CDL-C UMa model for a vertically polarized MPAC OTA setup with 16 uniformly spaced probes at FR1 test frequencies

| Azim [] | ¦¦ comb | Azim [] | ¦¦ comb | Azim [] | ¦¦ comb | Azim [] | ¦¦ comb | Azim [] | ¦¦ comb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 617 MHz |  | 722 MHz |  | 836.5 MHz |  | 1575.42 MHz |  | 1800 MHz |  |
| 270.0 | 1.00 | 270.0 | 1.00 | 270.0 | 1.00 | 270.0 | 1.00 | 270.0 | 1.00 |
| 251.4 | 0.99 | 254.1 | 0.99 | 256.3 | 0.99 | 262.7 | 0.99 | 260.9 | 0.99 |
| 232.9 | 0.99 | 238.3 | 0.98 | 242.6 | 0.98 | 255.5 | 0.98 | 251.7 | 0.96 |
| 214.3 | 0.98 | 222.4 | 0.97 | 228.9 | 0.97 | 248.2 | 0.96 | 242.6 | 0.93 |
| 195.8 | 0.96 | 206.6 | 0.96 | 215.2 | 0.96 | 240.9 | 0.94 | 233.5 | 0.90 |
| 110.4 | 0.61 | 190.7 | 0.94 | 201.6 | 0.95 | 233.7 | 0.92 | 224.3 | 0.89 |
| 40.8 | 0.47 | 120.5 | 0.58 | 187.9 | 0.92 | 226.4 | 0.91 | 215.2 | 0.88 |
| 331.2 | 0.85 | 61.1 | 0.30 | 128.7 | 0.56 | 219.1 | 0.90 | 206.0 | 0.87 |
|  |  | 1.6 | 0.56 | 77.3 | 0.19 | 211.9 | 0.89 | 196.9 | 0.84 |
|  |  | 302.1 | 0.95 | 26.0 | 0.27 | 204.6 | 0.88 | 187.8 | 0.79 |
|  |  |  |  | 334.7 | 0.70 | 197.3 | 0.87 | 134.3 | 0.16 |
|  |  |  |  | 283.3 | 0.99 | 190.0 | 0.84 | 88.6 | 0.30 |
|  |  |  |  |  |  | 182.8 | 0.79 | 43.0 | 0.22 |
|  |  |  |  |  |  | 152.7 | 0.42 | 357.3 | 0.36 |
|  |  |  |  |  |  | 125.5 | 0.13 | 311.6 | 0.57 |
|  |  |  |  |  |  | 98.2 | 0.30 |  |  |
|  |  |  |  |  |  | 71.0 | 0.31 |  |  |
|  |  |  |  |  |  | 43.7 | 0.29 |  |  |
|  |  |  |  |  |  | 16.5 | 0.33 |  |  |
|  |  |  |  |  |  | 349.2 | 0.29 |  |  |
|  |  |  |  |  |  | 321.9 | 0.48 |  |  |
|  |  |  |  |  |  | 294.7 | 0.88 |  |  |

| Azim [] | ¦¦ comb | Azim [] | ¦¦ comb | Azim [] | ¦¦ comb | Azim [] | ¦¦ comb |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2132.5 MHz |  | 2450 MHz |  | 3600 MHz |  | 4700 MHz |  |
| 270.0 | 1.00 | 270.0 | 1.00 | 270.0 | 1.00 | 270.0 | 1.00 |
| 261.9 | 0.99 | 263.0 | 0.99 | 265.2 | 0.98 | 266.3 | 0.98 |
| 253.9 | 0.95 | 256.0 | 0.95 | 260.5 | 0.95 | 262.7 | 0.94 |
| 245.8 | 0.92 | 249.0 | 0.91 | 255.7 | 0.90 | 259.0 | 0.89 |
| 237.8 | 0.89 | 242.0 | 0.87 | 250.9 | 0.84 | 255.4 | 0.83 |
| 229.7 | 0.86 | 234.9 | 0.85 | 246.1 | 0.80 | 251.7 | 0.78 |
| 221.7 | 0.85 | 227.9 | 0.83 | 241.4 | 0.77 | 248.1 | 0.73 |
| 213.6 | 0.85 | 220.9 | 0.82 | 236.6 | 0.75 | 244.4 | 0.70 |
| 205.6 | 0.83 | 213.9 | 0.82 | 231.8 | 0.73 | 240.8 | 0.68 |
| 197.5 | 0.80 | 206.9 | 0.80 | 227.1 | 0.72 | 237.1 | 0.66 |
| 189.5 | 0.75 | 199.9 | 0.77 | 222.3 | 0.71 | 233.5 | 0.65 |
| 181.4 | 0.67 | 192.9 | 0.73 | 217.5 | 0.70 | 229.8 | 0.64 |
| 139.7 | 0.22 | 185.9 | 0.66 | 212.7 | 0.69 | 226.1 | 0.63 |
| 99.5 | 0.24 | 144.9 | 0.26 | 208.0 | 0.67 | 222.5 | 0.62 |
| 59.2 | 0.03 | 109.9 | 0.23 | 203.2 | 0.64 | 218.8 | 0.61 |
| 18.9 | 0.16 | 74.8 | 0.19 | 198.4 | 0.61 | 215.2 | 0.60 |
| 338.6 | 0.37 | 39.8 | 0.13 | 193.7 | 0.56 | 211.5 | 0.59 |
| 298.4 | 0.73 | 4.7 | 0.15 | 188.9 | 0.49 | 207.9 | 0.57 |
|  |  | 329.7 | 0.38 | 184.1 | 0.41 | 204.2 | 0.55 |
|  |  | 294.6 | 0.74 | 156.1 | 0.42 | 200.6 | 0.52 |
|  |  |  |  | 132.3 | 0.19 | 196.9 | 0.48 |
|  |  |  |  | 108.4 | 0.64 | 193.3 | 0.42 |
|  |  |  |  | 84.6 | 0.47 | 189.6 | 0.35 |
|  |  |  |  | 60.7 | 0.44 | 185.9 | 0.26 |
|  |  |  |  | 36.9 | 0.28 | 182.3 | 0.18 |
|  |  |  |  | 13.0 | 0.16 | 161.7 | 0.59 |
|  |  |  |  | 349.1 | 0.16 | 143.5 | 0.26 |
|  |  |  |  | 325.3 | 0.41 | 125.2 | 0.79 |
|  |  |  |  | 301.4 | 0.40 | 106.9 | 0.43 |
|  |  |  |  | 277.6 | 0.95 | 88.6 | 0.68 |
|  |  |  |  |  |  | 70.4 | 0.63 |
|  |  |  |  |  |  | 52.1 | 0.75 |
|  |  |  |  |  |  | 33.8 | 0.87 |
|  |  |  |  |  |  | 15.5 | 0.67 |
|  |  |  |  |  |  | 357.3 | 0.09 |
|  |  |  |  |  |  | 339.0 | 0.25 |
|  |  |  |  |  |  | 320.7 | 0.32 |
|  |  |  |  |  |  | 302.4 | 0.42 |
|  |  |  |  |  |  | 284.2 | 0.73 |

Time Domain Alternative Method:

Time domain techniques can also be used to validate the spatial correlation. The spatial correlation validation measurement setup is illustrated in Figure C.3.4-6. In this case a Signal generator transmits a CW signal through the MIMO test system. The signal is received by a test antenna within the test area. Finally, the signal is collected by a signal analyser and the measured signal is stored for postprocessing.

Figure C.3.4-6: Configuration for spatial correlation validation based on time domain techniques

For each spatial point, the channel emulator should issue a trigger signal each time fading is started. For each point collect a time domain trace with the signal analyser, when done, stop fading. Data recording is synchronized with the channel emulator trigger. Depending on CE implementation, the trigger direction between SAN and CE needs to be adjusted, i.e., from SAN to CE or from CE to SAN.

Follow the same procedure to postprocess the data and calcalate the spatial correlation by setting m to 1. The settings for the Signal Generator and Signal Analyser are in Table C.3.4-6 and C.3.4-7 respectively.

Table C.3.4-6: Signal Generator Settings

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table C.3.1-1 |
| Output power | dBm | Function of the CE. Sufficiently above Noise Floor |

Table C.3.4-7: Signal Analyser Settings

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table C.3.1-1 |
| Sampling | Hz | At least 15 times bigger than the max Doppler spread (fd=v/λ) |
| Observation time | s | At least 16s. Channel Model length should be the same or greater than the observation time. |

Beam-Simultaneous Block Diagram

It is assumed that the beams are mapped to the inputs of the channel emulator as follows:

-	Beam 1: Input 1 and Input 2

-	Beam 2: Input 3 and Input 4 (CDL-C UMa only)

Figure C.3.4-7: Configuration for spatial correlation validation based on time domain techniques (CDL-C UMi)

Figure C.3.4-8: Configuration for spatial correlation validation based on time domain techniques (CDL-C UMa)

## C.3.5	Cross-polarization

This measurement checks how well the measured vertically or horizontally polarized power levels follow expected values. The test setup for cross-polarization is the same as PDP validation in Figure C.3.2-1.

Method of measurement: 		Step the emulation and store traces from VNA.

VNA settings for cross-polarization measurements are presented in Table C.3.5-1.

Table C.3.5-1: VNA settings for cross-polarization measurements

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink Centre Frequency in Table C.3.1-1 |
| Span | MHz | 40 |
| Number of traces |  | 1000 |
| Number of points |  | 802 |
| Averaging |  | 1 |

Channel model specification for cross-polarization measurements is presented in Table C.3.5-2.

Table C.3.5-2: Channel model specification for cross-polarization measurements

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table C.3.1-1 |
| Distance between traces in channel model | wavelength (Note) | > 2 |
| Channel model |  | As specified in Annex C.1 |
| Mobile speed (** | km/h | 30 |
| NOTE: Time [s] = distance [] / MS speed [/s] MS speed [/s] = MS speed [m /s] / Speed of light [m/s] * Centre frequency [Hz] (** The mobile speed is valid for the Time Domain Alternative method only |  |  |

Measurement Procedure:

Step the emulation and store traces from VNA. i.e., run the emulation to CIR number 1, pause, measure VNA trace, run the emulation to CIR number 10, pause, measure VNA trace. Continue until 1000 VNA traces are measured.  Ensure that the phase relationship at the inputs of the channel emulator do not produce destructive interference.

a.	Use a vertically polarized sleeve dipole to measure the V component.

b.	Use a horizontally polarized (vertically oriented) magnetic loop dipole, or a horizontally polarized sleeve dipole measured in four orthogonal horizontal positions and summed to measure the H component.

Method of measurement result analysis:

Measured VNA traces (frequency responses HV(t,f) and HHt,f are saved into a hard drive. The data is read into, e.g., Matlab. The frequency responses are averaged in power over time and frequency and the V/H ratio calculated as follows:

PV=tfHV(t,f)2

PH=tfHH(t,f)2

PV/H[dB]=10log10PVPH

Beam-Specific Block Diagram

It is assumed that the beams are mapped to the inputs of the channel emulator as follows:

-	Beam 1: Input 1 and Input 2

-	Beam 2: Input 3 and Input 4 (CDL-C UMa only)

Figure C.3.5-1: Setup for Beam-Specific V/H measurements (Beam 1)

Figure C.3.5-2: Setup for Beam-Specific V/H measurements (Beam 2 CDL-C UMa only)

Time Domain Alternative Method:

The power in the Vertical and Horizontal polarizations can also be measured in time domain. The measurement setup for Beam-Specific are presented in Figures C.3.5-3, and C.3.5-4.

Figure C.3.5-3: Setup for Beam-Specific V/H measurements (Beam 1)

Figure C.3.5-4: Setup for Beam-Specific V/H measurements (Beam 2 CDL-C UMa only)

The instruments settings are the same as those in tables C.3.4-6 and C.3.4-7. The measurement analysis is the same as that of the frequency domain method setting the summation over f to a single point.

The reference V/H-ratios for CDL-C UMa and CDL-C UMi channel model validation are defined in tables C.3.5-3 and C.3.5-4, respectively.

Table C.3.5-3: Reference V/H-ratios for CDL-C UMa

| UMa C, fc ≤ 2.5 GHz | Beam 1 | Input 1+2: V/H = 0 dB |
| --- | --- | --- |
|  | Beam 2 | Input 3+4: V/H = 0 dB |
| UMa C, fc > 2.5 GHz | Beam 1 | Input 1+2: V/H = 0 dB |
|  | Beam 2 | Input 3+4: V/H = 0 dB |

Table C.3.5-4: Reference V/H-ratios for CDL-C UMi

| UMi C, fc ≤ 2.5 GHz | Beam 1 | Inputs 1+2: V/H = 0 dB |
| --- | --- | --- |
| UMi C, fc > 2.5 GHz | Beam 1 | Inputs 1+2: V/H = 0 dB |

## C.3.6	Power validation

This measurement checks the total power in the centre of the test zone. The power validation is measured with a spectrum analyser as shown in Figure C.3.6-1.

Figure C.3.6-1: Setup for power validation measurements

Spectrum analyser settings for power validation measurements are presented in Table C.3.6-1.

Table C.3.6-1: Spectrum analyser settings for power validation measurements

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table C.3.1-2 |
| Integrated Channel Span | Hz | 40MHz |
| RBW | Hz | 30 kHz |
| VBW | Hz | ≥10MHz |
| Number of points |  | ≥400 |
| Averaging |  | ≥100 |
| Detector |  | RMS |

Measurement Procedure:

1.	Place a vertical reference dipole in the centre of the test zone connected to a spectrum analyser (or power meter) via a cable.

2.	Record the cable and reference dipole gains.

3.	Load the target channel model into the channel emulator and play the model.

4.	Start the NR FR1 signalling in the base station emulator with the required parameter identical to the measurements conditions.

5.	Average the power received by the spectrum analyser for a sufficient amount of time to account for the fading channel – one full channel simulation might be unnecessary. The possibility to use an equipment capable to decode the NR signal for EPRE Power Validation is not precluded.

6.	Repeat steps 1 to 4 with a magnetic loop for the horizontal polarization, or a horizontally polarized sleeve dipole measured in at least four orthogonal horizontal positions and average the summed orientations to get the H component.

7.	Calculate the total power received at the test area as the sum of the power in the two polarizations.

Note:	in step 6, if horizontally polarized sleeve dipole is used, the reference gain correction should be the average of the theta gain pattern cut of the dipole. Besides, more horizontal positions for averaging will improve the measurement accuracy but increase the total measurement time.

The detailed power validation setup for CDL-C UMi and CDL-C UMa channel models are illustrated in Figure C.3.6-2 and Figure C.3.6-3.

Figure C.3.6-2: Setup for power validation measurements for CDL-C UMi

Figure C.3.6-3: Setup for power validation measurements for CDL-C UMa

# C.4	Validation Pass/fail limit

## C.4.1	General

This clause defines the pass/fail limit of FR1 MPAC system for FR1 channel model validation.

## C.4.2	Pass/Fail Criteria of PDP

This clause defines the pass/fail criteria of PDP, this pass/fail limits apply for all FR1 frequency bands, for both combined and individual beams.

The detailed pass/fail limits for each cluster of CDL-C UMa and CDL-C UMi are defined in Table C.4.2-1.

Table C.4.2-1: PDP pass/fail limits for CDL-C UMa and CDL-C UMi channel model validation

|  | Power Tolerance | Delay Tolerance |
| --- | --- | --- |
| Paths from 0dB to 10dB | ±1dB | ±6ns |
| Paths from 10dB to 20dB | ±2.5dB | ±6ns |
| Paths from 20dB to 30dB | ±5dB | ±6ns |
| Paths from 30dB to 40dB | ±10dB | ±6ns |

## C.4.3	Pass/Fail Criteria of Doppler/Temporal correlation

This clause defines the pass/fail criteria of doppler/temporal correlation, this pass/fail limits apply for all channel models in all FR1 frequency bands, for both combined and individual beams.

The pass/fail limits for theoretical temporal correlation defined in Clause C.3.3 above 0.3 are formed as bands of ±0.1 of correlation capped at 1 at the high end. Additionally, when the theoretical temporal correlation drops below 0.3, the limits are formed at bands of ±0.3 of correlation capped at 0 at the low end.

For the detailed pass/fail limits, the values are defined in the Table C.4.3-1.

Table C.4.3-1: pass/fail limits for temporal correlation

| CDL-C UMa beam 1 at ≤ 2.5 GHz |  | CDL-C UMa beam 2 at ≤ 2.5 GHz |  | CDL-C UMa beam 1 at > 2.5 GHz |  | CDL-C UMa beam 2 at > 2.5 GHz |  | CDL-C UMi beam 1 at ≤ 2.5 GHz |  | CDL-C UMi beam 1 at > 2.5 GHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lower | Upper | Lower | Upper | Lower | Upper | Lower | Upper | Lower | Upper | Lower | Upper |
| 0.9 | 1 | 0.9 | 1 | 0.9 | 1 | 0.9 | 1 | 0.9 | 1 | 0.9 | 1 |
| 0.886 | 1 | 0.874 | 1 | 0.885 | 1 | 0.873 | 1 | 0.895 | 1 | 0.895 | 1 |
| 0.845 | 1 | 0.807 | 1 | 0.842 | 1 | 0.804 | 1 | 0.882 | 1 | 0.882 | 1 |
| 0.782 | 0.982 | 0.732 | 0.932 | 0.774 | 0.974 | 0.725 | 0.925 | 0.862 | 1 | 0.861 | 1 |
| 0.701 | 0.901 | 0.676 | 0.876 | 0.687 | 0.887 | 0.665 | 0.865 | 0.836 | 1 | 0.835 | 1 |
| 0.609 | 0.809 | 0.638 | 0.838 | 0.589 | 0.789 | 0.623 | 0.823 | 0.806 | 1 | 0.805 | 1 |
| 0.513 | 0.713 | 0.595 | 0.795 | 0.486 | 0.686 | 0.575 | 0.775 | 0.772 | 0.972 | 0.771 | 0.971 |
| 0.418 | 0.618 | 0.523 | 0.723 | 0.386 | 0.586 | 0.499 | 0.699 | 0.734 | 0.934 | 0.734 | 0.934 |
| 0.33 | 0.53 | 0.425 | 0.625 | 0.294 | 0.494 | 0.396 | 0.596 | 0.693 | 0.893 | 0.693 | 0.893 |
| 0.253 | 0.453 | 0.326 | 0.526 | 0.215 | 0.415 | 0.291 | 0.491 | 0.65 | 0.85 | 0.649 | 0.849 |
| 0.189 | 0.389 | 0.26 | 0.46 | 0.152 | 0.352 | 0.219 | 0.419 | 0.605 | 0.805 | 0.604 | 0.804 |
| 0.14 | 0.34 | 0.235 | 0.435 | 0.106 | 0.306 | 0.19 | 0.39 | 0.559 | 0.759 | 0.558 | 0.758 |
| 0.104 | 0.304 | 0.22 | 0.42 | 0 | 0.3 | 0.173 | 0.373 | 0.514 | 0.714 | 0.512 | 0.712 |
| 0 | 0.3 | 0.187 | 0.387 | 0 | 0.3 | 0.139 | 0.339 | 0.469 | 0.669 | 0.468 | 0.668 |
| 0 | 0.3 | 0.133 | 0.333 | 0 | 0.3 | 0 | 0.3 | 0.427 | 0.627 | 0.425 | 0.625 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0.387 | 0.587 | 0.385 | 0.585 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0.35 | 0.55 | 0.348 | 0.548 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0.317 | 0.517 | 0.315 | 0.515 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0.287 | 0.487 | 0.285 | 0.485 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0.261 | 0.461 | 0.258 | 0.458 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0.237 | 0.437 | 0.235 | 0.435 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0.216 | 0.416 | 0.213 | 0.413 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0.196 | 0.396 | 0.193 | 0.393 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0.177 | 0.377 | 0.174 | 0.374 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0.158 | 0.358 | 0.155 | 0.355 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0.139 | 0.339 | 0.136 | 0.336 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0.119 | 0.319 | 0.116 | 0.316 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 |
| 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 | 0 | 0.3 |

Based on the values defined in Table C.4.3-1, Figure C.4.3-1 shows the pass/fail and reference curve of temporal correlation.

Figure C.4.3-1: Pass/fail limits and targets of Temporal correlation for CDL-C UMa and CDL-C UMi channel model: red curve (reference), blue (upper limit) and green (lower limit)

## C.4.4	Pass/Fail Criteria of Spatial correlation

This clause defines the pass/fail criteria of spatial correlation, this general pass/fail limits principle apply for all channel models in all FR1 frequency bands, for both combined and individual beams.

The pass/fail limits for spatial correlation are formed as bands of ±10% of correlation capped at 100% for the upper limit for target correlation defined in clause C.3.4 of 35% (for CDL-C UMa @3600MHz, this value is 65%) and above. For target correlations below 35% (for CDL-C UMa @3600MHz, this value is 65%), the band is widened to ±20% capped at 0%.

For the detailed pass/fail limits, the values are defined in the Table C.4.4-1 and Table C.4.4-2, for CDL-C UMi and CDL-C UMa channel model, respectively.

Table C.4.4-1: Spatial correlation pass/fail limits for CDL-C UMi channel model

| Lower | Upper | Lower | Upper | Lower | Upper | Lower | Upper | Lower | Upper |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 617 MHz |  | 722 MHz |  | 836.5 MHz |  | 1575.42 MHz |  | 1880 MHz |  |
| 0.90 | 1.00 | 0.90 | 1.00 | 0.90 | 1.00 | 0.90 | 1.00 | 0.90 | 1.00 |
| 0.90 | 1.00 | 0.90 | 1.00 | 0.90 | 1.00 | 0.90 | 1.00 | 0.90 | 1.00 |
| 0.90 | 1.00 | 0.90 | 1.00 | 0.90 | 1.00 | 0.90 | 1.00 | 0.90 | 1.00 |
| 0.90 | 1.00 | 0.90 | 1.00 | 0.90 | 1.00 | 0.90 | 1.00 | 0.89 | 1.00 |
| 0.89 | 1.00 | 0.89 | 1.00 | 0.89 | 1.00 | 0.89 | 1.00 | 0.89 | 1.00 |
| 0.77 | 0.97 | 0.88 | 1.00 | 0.88 | 1.00 | 0.89 | 1.00 | 0.88 | 1.00 |
| 0.77 | 0.97 | 0.74 | 0.94 | 0.86 | 1.00 | 0.89 | 1.00 | 0.87 | 1.00 |
| 0.88 | 1 | 0.70 | 0.90 | 0.72 | 0.92 | 0.88 | 1.00 | 0.85 | 1.00 |
|  |  | 0.81 | 1.00 | 0.63 | 0.83 | 0.87 | 1.00 | 0.82 | 1.00 |
|  |  | 0.89 | 1.00 | 0.71 | 0.91 | 0.86 | 1.00 | 0.77 | 0.97 |
|  |  |  |  | 0.85 | 1.00 | 0.84 | 1.00 | 0.29 | 0.49 |
|  |  |  |  | 0.90 | 1.00 | 0.81 | 1.00 | 0.00 | 0.35 |
|  |  |  |  |  |  | 0.77 | 0.97 | 0.04 | 0.44 |
|  |  |  |  |  |  | 0.56 | 0.76 | 0.52 | 0.72 |
|  |  |  |  |  |  | 0.34 | 0.54 | 0.84 | 1.00 |
|  |  |  |  |  |  | 0.10 | 0.50 |  |  |
|  |  |  |  |  |  | 0.08 | 0.48 |  |  |
|  |  |  |  |  |  | 0.27 | 0.47 |  |  |
|  |  |  |  |  |  | 0.44 | 0.64 |  |  |
|  |  |  |  |  |  | 0.65 | 0.85 |  |  |
|  |  |  |  |  |  | 0.81 | 1.00 |  |  |
|  |  |  |  |  |  | 0.89 | 1.00 |  |  |
| Lower | Upper | Lower | Upper | Lower | Upper | Lower | Upper |
| 2132.5 MHz |  | 2450 MHz |  | 3600 MHz |  | 4700 MHz |  |
| 0.90 | 1.00 | 0.90 | 1.00 | 0.90 | 1.00 | 0.90 | 1.00 |
| 0.90 | 1.00 | 0.90 | 1.00 | 0.90 | 1.00 | 0.90 | 1.00 |
| 0.90 | 1.00 | 0.90 | 1.00 | 0.90 | 1.00 | 0.90 | 1.00 |
| 0.89 | 1.00 | 0.89 | 1.00 | 0.89 | 1.00 | 0.89 | 1.00 |
| 0.89 | 1.00 | 0.89 | 1.00 | 0.89 | 1.00 | 0.89 | 1.00 |
| 0.88 | 1.00 | 0.89 | 1.00 | 0.89 | 1.00 | 0.89 | 1.00 |
| 0.87 | 1.00 | 0.88 | 1.00 | 0.88 | 1.00 | 0.88 | 1.00 |
| 0.86 | 1.00 | 0.87 | 1.00 | 0.88 | 1.00 | 0.88 | 1.00 |
| 0.83 | 1.00 | 0.85 | 1.00 | 0.87 | 1.00 | 0.88 | 1.00 |
| 0.79 | 0.99 | 0.82 | 1.00 | 0.87 | 1.00 | 0.87 | 1.00 |
| 0.74 | 0.94 | 0.78 | 0.98 | 0.85 | 1.00 | 0.87 | 1.00 |
| 0.67 | 0.87 | 0.73 | 0.93 | 0.83 | 1.00 | 0.86 | 1.00 |
| 0.07 | 0.47 | 0.66 | 0.86 | 0.80 | 1.00 | 0.85 | 1.00 |
| 0 | 0.34 | 0.00 | 0.39 | 0.76 | 0.96 | 0.83 | 1.00 |
| 0 | 0.34 | 0.06 | 0.46 | 0.71 | 0.91 | 0.81 | 1.00 |
| 0.06 | 0.46 | 0.27 | 0.47 | 0.65 | 0.85 | 0.77 | 0.97 |
| 0.61 | 0.81 | 0.00 | 0.39 | 0.58 | 0.78 | 0.73 | 0.93 |
| 0.87 | 1.00 | 0.09 | 0.49 | 0.49 | 0.69 | 0.68 | 0.88 |
|  |  | 0.64 | 0.84 | 0.39 | 0.59 | 0.62 | 0.82 |
|  |  | 0.87 | 1.00 | 0.03 | 0.43 | 0.54 | 0.74 |
|  |  |  |  | 0.52 | 0.72 | 0.46 | 0.66 |
|  |  |  |  | 0.75 | 0.95 | 0.37 | 0.57 |
|  |  |  |  | 0.83 | 1.00 | 0.27 | 0.47 |
|  |  |  |  | 0.82 | 1.00 | 0.07 | 0.47 |
|  |  |  |  | 0.69 | 0.89 | 0.00 | 0.38 |
|  |  |  |  | 0.32 | 0.52 | 0.41 | 0.61 |
|  |  |  |  | 0.00 | 0.35 | 0.73 | 0.93 |
|  |  |  |  | 0.50 | 0.70 | 0.85 | 1.00 |
|  |  |  |  | 0.80 | 1.00 | 0.79 | 0.99 |
|  |  |  |  | 0.90 | 1.00 | 0.70 | 0.90 |
|  |  |  |  |  |  | 0.68 | 0.88 |
|  |  |  |  |  |  | 0.78 | 0.98 |
|  |  |  |  |  |  | 0.88 | 1.00 |
|  |  |  |  |  |  | 0.81 | 1.00 |
|  |  |  |  |  |  | 0.43 | 0.63 |
|  |  |  |  |  |  | 0.00 | 0.29 |
|  |  |  |  |  |  | 0.40 | 0.60 |
|  |  |  |  |  |  | 0.72 | 0.92 |
|  |  |  |  |  |  | 0.87 | 1.00 |

Table C.4.4-2: Spatial correlation pass/fail limits for CDL-C UMa channel model

| Lower | Upper | Lower | Upper | Lower | Upper | Lower | Upper | Lower | Upper |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 617 MHz |  | 722 MHz |  | 836.5 MHz |  | 1575.42 MHz |  | 1800 MHz |  |
| 0.90 | 1.00 | 0.90 | 1.00 | 0.90 | 1.00 | 0.90 | 1.00 | 0.90 | 1.00 |
| 0.89 | 1.00 | 0.89 | 1.00 | 0.89 | 1.00 | 0.89 | 1.00 | 0.89 | 1.00 |
| 0.89 | 1.00 | 0.88 | 1.00 | 0.88 | 1.00 | 0.88 | 1.00 | 0.86 | 1.00 |
| 0.88 | 1.00 | 0.87 | 1.00 | 0.87 | 1.00 | 0.86 | 1.00 | 0.83 | 1.00 |
| 0.86 | 1.00 | 0.86 | 1.00 | 0.86 | 1.00 | 0.84 | 1.00 | 0.80 | 1.00 |
| 0.51 | 0.71 | 0.84 | 1.00 | 0.85 | 1.00 | 0.82 | 1.00 | 0.79 | 0.99 |
| 0.37 | 0.57 | 0.48 | 0.68 | 0.82 | 1.00 | 0.81 | 1.00 | 0.78 | 0.98 |
| 0.75 | 0.95 | 0.10 | 0.50 | 0.46 | 0.66 | 0.80 | 1.00 | 0.77 | 0.97 |
|  |  | 0.46 | 0.66 | 0 | 0.39 | 0.79 | 0.99 | 0.74 | 0.94 |
|  |  | 0.85 | 1.00 | 0.07 | 0.47 | 0.78 | 0.98 | 0.69 | 0.89 |
|  |  |  |  | 0.60 | 0.80 | 0.77 | 0.97 | 0.00 | 0.36 |
|  |  |  |  | 0.89 | 1.00 | 0.74 | 0.94 | 0.10 | 0.50 |
|  |  |  |  |  |  | 0.69 | 0.89 | 0.02 | 0.42 |
|  |  |  |  |  |  | 0.32 | 0.52 | 0.26 | 0.46 |
|  |  |  |  |  |  | 0.00 | 0.33 | 0.47 | 0.67 |
|  |  |  |  |  |  | 0.10 | 0.50 |  |  |
|  |  |  |  |  |  | 0.11 | 0.51 |  |  |
|  |  |  |  |  |  | 0.09 | 0.49 |  |  |
|  |  |  |  |  |  | 0.13 | 0.53 |  |  |
|  |  |  |  |  |  | 0.09 | 0.49 |  |  |
|  |  |  |  |  |  | 0.38 | 0.58 |  |  |
|  |  |  |  |  |  | 0.78 | 0.98 |  |  |
| Lower | Upper | Lower | Upper | Lower | Upper | Lower | Upper |
| 2132.5 MHz |  | 2450 MHz |  | 3600 MHz |  | 4700 MHz |  |
| 0.90 | 1.00 | 0.90 | 1.00 | 0.90 | 1.00 | 0.90 | 1.00 |
| 0.89 | 1.00 | 0.89 | 1.00 | 0.88 | 1.00 | 0.88 | 1.00 |
| 0.85 | 1.00 | 0.85 | 1.00 | 0.85 | 1.00 | 0.84 | 1.00 |
| 0.82 | 1.00 | 0.81 | 1.00 | 0.80 | 1.00 | 0.79 | 0.99 |
| 0.79 | 0.99 | 0.77 | 0.97 | 0.74 | 0.94 | 0.73 | 0.93 |
| 0.76 | 0.96 | 0.75 | 0.95 | 0.70 | 0.90 | 0.68 | 0.88 |
| 0.75 | 0.95 | 0.73 | 0.93 | 0.67 | 0.87 | 0.63 | 0.83 |
| 0.75 | 0.95 | 0.72 | 0.92 | 0.65 | 0.85 | 0.60 | 0.80 |
| 0.73 | 0.93 | 0.72 | 0.92 | 0.63 | 0.83 | 0.58 | 0.78 |
| 0.7 | 0.9 | 0.70 | 0.90 | 0.62 | 0.82 | 0.56 | 0.76 |
| 0.65 | 0.85 | 0.67 | 0.87 | 0.61 | 0.81 | 0.55 | 0.75 |
| 0.57 | 0.77 | 0.63 | 0.83 | 0.60 | 0.80 | 0.54 | 0.74 |
| 0.02 | 0.42 | 0.56 | 0.76 | 0.59 | 0.79 | 0.53 | 0.73 |
| 0.04 | 0.44 | 0.06 | 0.46 | 0.57 | 0.77 | 0.52 | 0.72 |
| 0 | 0.23 | 0.03 | 0.43 | 0.44 | 0.84 | 0.51 | 0.71 |
| 0 | 0.36 | 0.00 | 0.39 | 0.41 | 0.81 | 0.50 | 0.70 |
| 0.27 | 0.47 | 0.00 | 0.33 | 0.36 | 0.76 | 0.49 | 0.69 |
| 0.63 | 0.83 | 0.00 | 0.35 | 0.29 | 0.69 | 0.47 | 0.67 |
|  |  | 0.28 | 0.48 | 0.21 | 0.61 | 0.45 | 0.65 |
|  |  | 0.64 | 0.84 | 0.22 | 0.62 | 0.42 | 0.62 |
|  |  |  |  | 0.00 | 0.39 | 0.38 | 0.58 |
|  |  |  |  | 0.44 | 0.84 | 0.32 | 0.52 |
|  |  |  |  | 0.27 | 0.67 | 0.15 | 0.55 |
|  |  |  |  | 0.24 | 0.64 | 0.06 | 0.46 |
|  |  |  |  | 0.08 | 0.48 | 0.00 | 0.38 |
|  |  |  |  | 0.00 | 0.36 | 0.49 | 0.69 |
|  |  |  |  | 0.00 | 0.36 | 0.06 | 0.46 |
|  |  |  |  | 0.21 | 0.61 | 0.69 | 0.89 |
|  |  |  |  | 0.20 | 0.60 | 0.33 | 0.53 |
|  |  |  |  | 0.85 | 1.00 | 0.58 | 0.78 |
|  |  |  |  |  |  | 0.53 | 0.73 |
|  |  |  |  |  |  | 0.65 | 0.85 |
|  |  |  |  |  |  | 0.77 | 0.97 |
|  |  |  |  |  |  | 0.57 | 0.77 |
|  |  |  |  |  |  | 0.00 | 0.29 |
|  |  |  |  |  |  | 0.05 | 0.45 |
|  |  |  |  |  |  | 0.12 | 0.52 |
|  |  |  |  |  |  | 0.32 | 0.52 |
|  |  |  |  |  |  | 0.63 | 0.83 |

## C.4.5	Pass/Fail Criteria of Cross-polarization

This clause defines the pass/fail criteria of cross-polarization, this pass/fail limits apply for all channel models in all FR1 frequency bands, for both combined and individual beams.

The cross-polarization ratio pass/fail limit is specified as ±1dB.

## C.4.6	Pass/Fail Criteria of Power validation

This clause defines the pass/fail criteria of power validation, this pass/fail limit applies to all channel models in all FR1 frequency bands.

The power validation pass/fail limit is specified as ± 1.5 dB.

Annex D (normative):
<FR2 Channel models and Validation procedure >

# D.1	FR2 Channel models

The following channel model is required for FR2 MIMO OTA measurement.

The generic model is Table D.1-1 FR2 UMi CDL-C, which does not include base station antenna filtering. Therefore, in addition, the BS beam filtering effect defined in Annex D.2 also apply when emulating the channel models.

Table D.1-1: Channel model parameters for UMi CDL-C at 28 GHz

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -30.4353 | -134.4434 | 98.9242 | 83.3318 |
| 2 | 12.594 | -1.25 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 3 | 13.314 | -3.4684 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 4 | 13.974 | -5.2294 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 5 | 13.056 | -2.5215 | -28.0782 | -152.8206 | 99.5732 | 71.1282 |
| 6 | 38.196 | 0 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 7 | 38.688 | -2.2185 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 8 | 39.36 | -3.9794 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 9 | 39.504 | -7.4215 | 17.3861 | 84.3647 | 100.4513 | 69.2454 |
| 10 | 47.61 | -7.1215 | -37.5865 | 92.0623 | 98.5616 | 66.7349 |
| 11 | 49.278 | -10.7215 | 20.2226 | -97.7585 | 100.6231 | 72.0348 |
| 12 | 56.016 | -11.1215 | -50.6106 | 78.4702 | 98.218 | 64.4337 |
| 13 | 73.71 | -5.1215 | -33.911 | 93.1719 | 100.165 | 85.4238 |
| 14 | 78.498 | -6.8215 | -37.5066 | -112.0441 | 100.2604 | 64.1548 |
| 15 | 130.224 | -8.7215 | -43.1797 | 102.4645 | 98.1225 | 64.7824 |
| 16 | 162.63 | -13.2215 | 29.2116 | 67.2359 | 100.2604 | 92.467 |
| 17 | 255.534 | -13.9215 | 27.8133 | 34.5731 | 98.4852 | 65.6889 |
| 18 | 276.018 | -13.9215 | 23.6584 | 48.5813 | 98.1416 | 68.7572 |
| 19 | 329.412 | -15.8215 | -52.5282 | 36.4455 | 97.9698 | 59.1339 |
| 20 | 336.462 | -17.1215 | 25.0168 | 52.6729 | 100.7376 | 65.3402 |
| 21 | 378.39 | -16.0215 | 25.4562 | 49.8296 | 98.1225 | 58.4365 |
| 22 | 398.244 | -15.7215 | 30.7697 | 46.4316 | 98.1034 | 65.2705 |
| 23 | 422.562 | -21.6215 | 35.9234 | 30.759 | 100.4513 | 62.6903 |
| 24 | 519.138 | -22.8215 | -61.2775 | 69.2469 | 100.9476 | 61.993 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 4.8814 | 7 |  |

# D.2	FR2 Base Station beam configuration

The emulated BS beam configuration to be used for emulation of channel model defined in Annex D.1 is specified in this clause.

The Base Station beam configuration includes basic antenna parameters and beamforming characteristic. The basic BS antenna parameters is defined in Table D.2-1.

Table D.2-1: FR2 BS Antenna Parameters

| Parameter description | Symbol | Parameter value |
| --- | --- | --- |
|  |  | FR2 |
| Antenna panels in vertical dimension | Mg | 1 |
| Antenna panels in horizontal dimension | Ng | 1 |
| Elements per panel in vertical dimension | Me | 8 |
| Elements per panel in horizontal dimension | Ne | 16 |
| Number of polarizations per panel | P | 2 |
| Element spacing in horizontal dimension () | dH | 0.5 |
| Element spacing in vertical dimension () | dV | 0.5 |

Antenna element radiation patterns, including orientation of the element main polarization components as well as orientation of the antenna array are as in the example pattern in Table 7.3-1 of TR 38.901. The antenna element has ±45 polarization components and the radiation pattern parameters are 3dB = 65, 3dB = 65, Amax = 30dB, SLAv = 30dB, GE,max =8 dBi.

The beamforming characteristic of the FR2 BS pattern is defined as follow:

-	A code book of 128 fixed beams is constructed to a grid of eight elevation angles from –25 to +25 with ~7.1 step size and 16 azimuth angles from –60 to +60 with 8 step size；

-	1 strongest transmitting beam is generated from BS, the direction of this beam towards the strongest cluster of the FR2 channel model. Specifically, the direction in UMi CDL-C model is: AoD: -12o, ZoD: 100.71.

# D.3	FR2 Channel model validation

## D.3.1	General

This clause describes the FR2 MIMO OTA validation measurements, in order to ensure that the channel models are correctly implemented and hence capable of generating the propagation environment, as described by the model, within the test zone of the 3D-MPAC system.

The following measurements shall be done for FR2 channel model validation:

-	Power Delay Profile (PDP)

-	Doppler/Temporal correlation

-	PAS similarity percentage (PSP)

-	Cross-polarization

-	Power validation

Frequencies to be used to test for FR2 channel model validation:

Table D.3.1-1: FR2 Channel model validation frequencies

| NR FR2 Bands | Range | Test Frequency (MHz) |
| --- | --- | --- |
| n257 | Low | 27750 |
| n260 | High | 38500 |
| n258 | Low | 25875 |
| n261 | Low | 27925 |

## D.3.2	FR2 Power Delay Profile (PDP)

This measurement checks that the resulting power delay profile (PDP) is in-line with the PDP defined for the channel model. For PDP validation measurement, only Vertical validation is required.

The PDP measurement is performed with a Vector Network Analyser (VNA). An example setup for PDP measurement is shown in Figure D.3.2-1. VNA transmits frequency sweep signals thorough the NR MIMO OTA test system. A reference antenna, within the centre of the test zone, receives the signal and VNA analyses the frequency response of the system. A number of traces (frequency responses) are measured and recorded by VNA and analysed by a post processing SW, e.g., Matlab. Special care has to be taken into account to keep the fading conditions unchanged, i.e. frozen, during the short period of time of a single trace measurement. The fading may proceed only in between traces.

Figure D.3.2-1: Setup for PDP measurements (FR2)

Step the emulation and store traces from VNA. I.e. run the emulation to CIR number 1, pause, measure VNA trace, run the emulation to CIR number 10, pause, measure VNA trace. Continue until 1000 VNA traces are measured.

VNA settings:

Table D.3.2-1: VNA settings for FR2 PDP measurements

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table D.3.1-1 |
| Span | MHz | 200 |
| Number of traces |  | 1000 |
| Number of points |  | 1101 |
| Averaging |  | 1 |

Channel model specification:

Table D.3.2-2: Channel model specification for FR2 PDP measurements

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table D.3.1-1 |
| Distance between traces in channel model | wavelength (Note) | > 2 |
| Channel model |  | As specified in Annex D.1 |
| NOTE: Time [s] = distance [] / MS speed [/s] MS speed [/s] = MS speed [m/s] / Speed of light [m/s] * Centre frequency [Hz] |  |  |

Method of measurement result analysis:

Measured VNA traces (frequency responses H(t,f)) are saved into a hard drive. The data is read into, e.g., Matlab. 
The analysis is performed by taking the Fourier transform of each trace. The resulting impulse responses h(t,) are averaged in power over time:

Finally, the resulting PDP is shifted in delay, such that the first tap is on delay zero.

The detailed PDP reference value for FR2 CDL-C UMi validation are defined in the following table:

Table D.3.2-3: PDP Targets for FR2 CDL-C UMi

| Combined Clusters index | Delay(ns) | Power(dB) |
| --- | --- | --- |
| 1 | 0 | -27.8 |
| 2-5 | 15 | -18.3 |
| 6-11 | 40 | 0.0 |
| 13-14 | 75 | -31.2 |

## D.3.3	FR2 Doppler/Temporal correlation

This measurement checks the Doppler/temporal correlation. For Doppler/Temporal correlation validation measurement, only Vertical validation is required.

The Doppler spectrum is measured with a spectrum analyser as shown in Figure D.3.3-1. In this case a signal generator transmits CW signal through the NR MIMO OTA test system. The signal is received by a test antenna within the test area. Finally, the signal is analysed by a spectrum analyser and the measured spectrum is compared to the target spectrum. This setup can be used to measure Doppler Spectrum of the Channel models defined in Annex D.2.

Method of measurement:

Figure D.3.3-1: Setup for FR2 Doppler measurements

Sine wave (CW, carrier wave) signal is transmitted from the signal generator. The signal is connected from the signal generator to fading emulator via cables. The fading emulator output signals are connected to frequency converter and power amplifier boxes via cables. The amplified signals are then transferred via cables to the probe antennas. The probe antennas radiate the signals over the air to the test antenna The Doppler spectrum is measured by the spectrum analyser and the trace is saved.

Signal generator settings:

Table D.3.3-1: Signal generator settings for FR2 Doppler/Temporal correlation measurements

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table D.3.1-1 |
| Modulation |  | OFF |

Spectrum analyser settings:

Table D.3.3-2: Spectrum analyser settings for FR2 Doppler/Temporal correlation measurements

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table D.3.1-1 |
| Minimum Span | Hz | 4 kHz |
| RBW | Hz | 1 |
| VBW | Hz | 1 |
| Number of points |  | 16002 |
| Averaging |  | 100 |

Channel model specification:

Table D.3.3-3: Channel model specification for FR2 Doppler/Temporal correlation measurements

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table D.3.1-1 |
| Channel model |  | As specified in Annex D.1 |
| Mobile speed | km/h | 12 |

Method of measurement result analysis: Measurement data file (Doppler power spectrum) is saved into hard drive. The data is read into, e.g., Matlab. The analysis is performed by taking the Fourier transformation of the Doppler spectrum. The resulting temporal correlation function   is normalized such that max(abs(Rt(∆t)))=1. Then the function values left from the maximum is cut out. Further on the function values after, e.g. seven periods is cut out.

The detailed Temporal correlation reference value for FR2 CDL-C UMi channel model validation is defined is table D.3.3-4.

Table D.3.3-4: Temporal correlation Targets

| Distance [λ] | X2V Corr. | Distance [λ] | X2V Corr. |
| --- | --- | --- | --- |
| 0 | 1.0000 | 2.5 | 0.1769 |
| 0.1 | 0.9929 | 2.6 | 0.1717 |
| 0.2 | 0.9717 | 2.7 | 0.1649 |
| 0.3 | 0.9379 | 2.8 | 0.1564 |
| 0.4 | 0.8937 | 2.9 | 0.1456 |
| 0.5 | 0.8414 | 3 | 0.1327 |
| 0.6 | 0.7834 | 3.1 | 0.1177 |
| 0.7 | 0.7223 | 3.2 | 0.1011 |
| 0.8 | 0.6601 | 3.3 | 0.0829 |
| 0.9 | 0.5986 | 3.4 | 0.0638 |
| 1 | 0.5387 | 3.5 | 0.0449 |
| 1.1 | 0.4817 | 3.6 | 0.0272 |
| 1.2 | 0.4284 | 3.7 | 0.0121 |
| 1.3 | 0.3796 | 3.8 | 0.0023 |
| 1.4 | 0.3362 | 3.9 | 0.0079 |
| 1.5 | 0.2984 | 4 | 0.0104 |
| 1.6 | 0.2667 | 4.1 | 0.0083 |
| 1.7 | 0.2416 | 4.2 | 0.0026 |
| 1.8 | 0.2221 | 4.3 | 0.0095 |
| 1.9 | 0.2081 | 4.4 | 0.0235 |
| 2 | 0.1987 | 4.5 | 0.0397 |
| 2.1 | 0.1921 | 4.6 | 0.0572 |
| 2.2 | 0.1879 | 4.7 | 0.0738 |
| 2.3 | 0.1844 | 4.8 | 0.0890 |
| 2.4 | 0.1812 | 4.9 | 0.1018 |
|  |  | 5 | 0.1109 |

Time Domain Alternate Method

Time domain techniques can also be used to validate the tempoal correlation. The temporal correlation validation measurement setup is illustrated in Figure D.3.3-2. In this case a Signal generator transmits a CW signal through the MIMO test system. The signal is received by a test antenna within the test area. Finally, the signal is collected by a signal analyser and the measured signal is stored as IQ data format for postprocessing. Depending on CE implementation, the trigger direction between spectrum analyser (SAN) and CE needs to be adjusted, i.e., from SAN to CE or from CE to SAN.

Figure D.3.3-2: Setup for Doppler measurements based on time domain technique

The time domain doppler spectrum is measured by the signal analyser and the trace in IQ format is saved. Follow the same procedure to post process the data and calculate the temporal correlation curve. Data recording is synchronized with the channel emulator trigger.

The settings for the signal analyser are in Table D.3.3-5:

Table D.3.3-5: Signal Analyser Settings

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table D.3.1-1 |
| Sampling | Hz | At least 15 times bigger than the max Doppler spread (fd=v/λ) |
| Observation time | s | At least 16s. Channel Model length should be the same or greater than the observation time. |

## D.3.4	FR2 PAS similarity percentage (PSP)

The PSP validation measurements aim at evaluating PAS similarity percentage (PSP), which is one of the validation metrics for characterizing FR2 channel model under test in the quite zone of 3D-MPAC. For PSP validation measurement, only vertical polarization validation is required.

The measurement array is essentially a virtual array configuration realized in 3D-MPAC through a -θ positioning system. The measurement array is a semi-circle and sectored array configuration illustrated in Figure D.3.4-1 where complex channel frequency response is measured at each antenna location 0.5 λ apart using a vector network analyser (VNA) setup.  The vertical sectors of the measurement array are limited to 60 (±30) and the horizontal sector to 180 (±90) with the broad side direction points towards the probes. Depending of the turntable architecture/implementation, the virtual array configuration for the PSP validation is composed of two alternative semi-circle arrangements (1 x horizontal and either 2 x crossed vertical or 2 x parallel vertical). The radius of the array element locations with respect to the centre of the test zone is 5 cm, which is equivalent to the half of the test zone radius at 28 GHz. For different frequency bands, the radius of the measurement array sectored semi-circles remains fixed at 5 cm while the spatial sampling of the array varies. This measurement validates the proper angular behaviour in the test zone.

Figure D.3.4-1: Semi-circle measurement array configurations with K = 37 elements (at 28 GHz). On the left with two crossed vertical sectors, on the right with two parallel vertical sectors.

Figure D.3.4-2: Setup for FR2 PSP validation measurements

The PSP validation is measured with a vector network analyser as shown in Figure D.3.4-2 illustrating the PSP measurement setup. Port 1 of the VNA transmits signals through the fading emulator and radiate them through L probes within the anechoic chamber. The radiated signals are then received at the test antenna that is positioned inside the test zone. The test antenna is mounted on a -θ positioner which is capable of moving the antenna to pre-defined spatial locations on a fixed radius from the centre of the quiet zone according the measurement array configuration. Finally, the signal is received at port 2 of the VNA. The most suitable approach for the PSP validation is based on an omnidirectional antenna (omnidirectional pattern in AZ and wide BW in EL) as the test can be automated easily. Alternatively, a directional antenna could be used but requires frequent re-positioning.

The measurement and analysis procedure are given as follows:

1.	Set the target channel model in the Channel Emulator.

2.	For each position of the test antenna on the measurement array configuration in the test zone, step & pause the emulator to different time instances. Measure the complex frequency responses Hf,t=Hm∆f,n∆T, m=0,..., M-1 for all stepped channel snapshots n=0,..., N-1 , where the interval between frequency and time samples is ∆f and ∆T, respectively. The number of channel snapshots N and frequency samples M.

3.	Move the measurement antenna with a positioner to another location k and repeat step 2 to record frequency responses Hk(m∆f,n∆T)   of all stepped channel snapshots.

4.	Repeat step 3 to record frequency responses at all k=0,..., K  spatial sample points.

5.	Estimate the measured PAS through the following two- stage processing:

a.		In the first stage, calculate the discrete azimuth and elevation angles (DoA) for the measurement array configuration by applying the MUSIC algorithm. Estimate the powers from the DoA and auto-covariance matrix of the received signal acquired through VNA complex frequency response data.

i) Compose an estimate of the covariance matrix RH∈CK×K. The entry on the kth row and k'th column of RH is

RHk,k'=n=1Nm=1MHkm∆f,n∆T Hk'm∆f,n∆T,

where  is the complex conjugate operator.

ii) Compose pseudo-PAS of the chamber environment and channel model using the MUSIC algorithm. The intention of this step is to estimate DoAs of signals radiated by probes. Perform first the eigen decomposition of RH and pick the noise-subspace matrix V. The matrix V is obtained from the eigen decomposition by removing eigenvectors, i.e. columns, that correspond to X strongest eigenvalues, where X is the number of active probes in the MPAC setup. The pseudo-PAS is [9]

PMUSICβ=1ωβ VVH ωHβ ,

where H is the matrix Hermitean operator and the near-field array factor of the virtual array composed by K spatial measurement antenna locations is

ωβ=λ04πrk-R uβexpj2πrk-R uβλ0,

where λ0 is the wavelength at the carrier centre frequency,  is the norm of a vector, rk is a location vector of the kth virtual array element, uβ is the unit vector to direction β, and R is the a priori known approximate range length [9]. Both vectors rk and uβ are defined with respect to same origin, which is the centre of the test zone.

iii) Find local maxima of PMUSICβ and pick directions βx, x=1,…,X of the X  highest peaks.

iv) Perform Bartlett beamforming as defined in step 6 with the steering vector ωβx (instead of a(β)) to the X directions identified in iii). The output is X power estimates Pestβx.

b.	In the second stage, use the angle and power estimates, i.e. the discrete PAS of X azimuth and elevation directions and power values in conjunction with a 4x4 DUT sampling array for beamforming with the conventional Bartlett beamformer to estimate the “measured PAS seen by DUT” for PSP calculation. This is

Poβ=aHβx=1XaβxPestβxaHβxaβ,

where a(β) is the array steering vector of the 4x4 DUT array.

6.	Evaluate the reference OTA PAS for the 4x4 DUT array by applying the conventional Bartlett beamformer. The theoretical PAS as reference is calculated for the ideal channel model as

Prβ=aHβ ∮a(β) P(β)aH(β)dβ aβ,

where a(β) is the array steering vector of the 4x4 DUT array and P(β) is the PAS of the reference channel model.

7.	Calculate total variation distance (Dp) from the reference and measured PAS. Mathematically,

8.	Calculate PSP values as PSP = (1-Dp) x 100%.

VNA settings:

Table D.3.4-1: VNA settings for FR2 PSP measurements

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table D.3.1-1 |
| Span | MHz | 0, or the minimum (Note 1) |
| Number of traces |  | 1000 |
| Number of points |  | 1 (Note 1) |
| NOTE 1: Span and number of points may be increased to estimate reliably. |  |  |

Channel model specification:

Table D.3.4-2: Channel model specification for FR2 PSP measurements

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table D.3.1-1 |
| Distance between traces in channel model | wavelength (Note) | > 2 |
| Channel model |  | As specified in Annex D.1 |
| NOTE: Time [s] = distance [] / MS speed [/s] MS speed [/s] = MS speed [m/s] / Speed of light [m/s] * Centre frequency [Hz] |  |  |

Time Domain Alternative Method:

PSP validation can also be implemented using time-domain techniques using the testing setup presented in Figure D.3.4-3. The VNA is substituted by a signal generator, and a signal analyser. Depending on CE implementation, the trigger direction between SAN and CE needs to be adjusted, i.e., from SAN to CE or from CE to SAN.

Figure D.3.4-3: Setup for FR2 PSP validation measurements based on time domain

Table D.3.4-3: Signal Generator Settings for FR2 PSP measurements based on time domain

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table D.3.1-1 |
| Output power | dBm | Function of the CE. Sufficiently above Noise Floor |

Table D.3.4-4: Signal Analyser Settings for FR2 PSP measurements based on time domain

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table D.3.1-1 |
| Sampling | Hz | At least 10 times bigger than the max Doppler spread (fd=v/λ) |
| Observation time | s | At least 32s |

The measurement and analysis procedure are given as follows:

Follow the same procedure as before, but M is set to 1. The Channel Emulator is not stepped, but it is allowed to play in free run mode for each of the K spatial points.

## D.3.5	FR2 Cross-polarization

This measurement checks how well the measured vertically or horizontally polarized power levels follow expected values. The test setup for cross-polarization is the same as PDP validation in Figure D.3.2-1.

Method of measurement: 		Step the emulation and store traces from VNA.

VNA settings:

Table D.3.5-1: VNA settings for FR2 cross-polarization

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink Centre Frequency in Table D.3.1-1 |
| Span | MHz | 40 |
| Number of traces |  | 1000 |
| Number of points |  | 802 |
| Averaging |  | 1 |

Channel model specification:

Table D.3.5-2: Channel model specification for FR2 cross-polarization.

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table D.3.1-1 |
| Distance between traces in channel model | wavelength (Note) | > 2 |
| Channel model |  | As specified in Annex D.1 |
| Mobile speed | km/h | 30 |
| NOTE: Time [s] = distance [] / MS speed [/s] MS speed [/s] = MS speed [m /s] / Speed of light [m/s] * Centre frequency [Hz] |  |  |

Measurement Procedure:

1.	Play or step through the channel model listed in Annex D.1.

2.	Measure the absolute power received at the centre of the test zone, averaged over a statistically significant number of fades.

a.	Perform separate V-polarization and H-polarization measurements sequentially, e.g., by rotating a single-polarized horn antenna accordingly or with a use of a dual-polarized horn where the un-used polarization is terminated.

b.	Measure input 1+2 first and then enable input at a time to measure input 1 and 2 separately

c.	Disable all CE outputs and enable one output at a time. Get the VNA trace pointing the measurement horn antenna towards the active probe. Record frequency sweeps through the channel model. Repeat for all the probes.

3.	Calculate the V/H ratio.

-	Measure VNA traces (frequency responses over time and probes HV(t,f,kV) and HHt,f,kH are saved into a hard drive. The data is read into, e.g., Matlab. The frequency responses are summed in power over time t, frequency f  and probes k and the V/H ratio calculated as follows:

PV=tfkVHV(t,f,kV)2

PH=tfkHHH(t,f,kH)2

PV/H[dB]=10log10PVPH

4.	Compare it with the theory value.

Reference target for FR2 CDL-C UMi:

-	V/H FR2 CDL-C-UMi, X2+, fc=28 GHz

-	Beam 1, Input 1:  V/H = -0.45 dB

-	Beam 1, Input 2:  V/H = 0.49 dB

-	Beam 1, Input 1+2:  V/H = 0 dB

## D.3.6	FR2 Power validation

This measurement checks the total power in the centre of the test zone. The power validation is measured with a spectrum analyser as shown in Figure D.3.6-1.

Figure D.3.6-1: Setup for FR2 power validation measurements

Spectrum analyser settings:

Table D.3.6-1: Spectrum analyser settings for FR2 power validation measurements

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table D.3.1-1 |
| Integrated Channel Span | Hz | 20MHz |
| RBW | Hz | 30 kHz |
| VBW | Hz | ≥10MHz |
| Number of points |  | ≥400 |
| Averaging |  | ≥100 |
| Detector |  | RMS |

Measurement Procedure:

1.	Place a horn antenna in the centre of the test zone connected to a spectrum analyser (or power meter) via a cable. Point the horn to the first probe and initially select the V-polarization.

2.	Record the cable and horn antenna gains.

3.	Load the target channel model into the channel emulator.

4.	Start the NR FR2 signalling in the base station emulator with the required parameter identical to the measurements conditions.

5.	Disable all CE outputs and enable only the CE output that points towards the active probe with the polarization matching the polarization of the reference horn antenna.

6.	Average the power received by the spectrum analyser to get ≥ 100 averages to account for the fading channel.

7.	Repeat steps 5 to 6 for all six probe directions and perform separate V and H polarization measurements while matching the polarization of the horn antenna in the centre of the test zone with the polarization of the probe.

8.	Calculate the total power received at the test area as the sum of the power over k=6 probes in the two polarizations as follows:

PV=10log10kV10PkV[dB]10

PH=10log10kH10PkH[dB]10

Ptotal[dB]=10log10kV10PkV[dB]10+kH10PkH[dB]10

# D.4	Validation Pass/fail limit

## D.4.1	General

This clause defines the pass/fail limit of FR2 3D-MPAC system for FR2 channel model validation.

## D.4.2	Pass/Fail Criteria of PDP

This clause defines the pass/fail criteria of PDP, this pass/fail limits apply for all FR2 frequency bands.

The detailed pass/fail limits for each cluster of FR2 CDL-C UMi are defined in Table D.4.2-1.

Table D.4.2-1: PDP pass/fail limits for FR2 CDL-C UMi channel model validation

|  | Power Tolerance | Delay Tolerance |
| --- | --- | --- |
| Paths from 0dB to 10dB | ±1dB | ±6ns |
| Paths from 10dB to 30dB | ±5dB | ±6ns |
| Paths from 30dB to 40dB | ±10dB | ±6ns |

## D.4.3	Pass/Fail Criteria of Doppler/Temporal correlation

This clause defines the pass/fail criteria of doppler/temporal correlation validation, this pass/fail limits apply for all FR2 frequency bands.

The pass/fail limits for theoretical temporal correlation defined in Clause D.3.3 above 0.3 are formed as bands of ±0.1 of correlation capped at 1 at the high end. Additionally, when the theoretical temporal correlation drops below 0.3, the limits are formed at bands of ±0.3 of correlation capped at 0 at the low end.

Table D.4.3-1: pass/fail limits for temporal correlation

| Distance [λ] | X2V Corr. |  | Distance [λ] | X2V Corr. |  |
| --- | --- | --- | --- | --- | --- |
|  | Lower | Upper |  | Lower | Upper |
| 0 | 0.9000 | 1.0000 | 2.5 | 0.0000 | 0.3000 |
| 0.1 | 0.8929 | 1.0000 | 2.6 | 0.0000 | 0.3000 |
| 0.2 | 0.8717 | 1.0000 | 2.7 | 0.0000 | 0.3000 |
| 0.3 | 0.8379 | 1.0000 | 2.8 | 0.0000 | 0.3000 |
| 0.4 | 0.7937 | 0.9937 | 2.9 | 0.0000 | 0.3000 |
| 0.5 | 0.7414 | 0.9414 | 3 | 0.0000 | 0.3000 |
| 0.6 | 0.6834 | 0.8834 | 3.1 | 0.0000 | 0.3000 |
| 0.7 | 0.6223 | 0.8223 | 3.2 | 0.0000 | 0.3000 |
| 0.8 | 0.5601 | 0.7601 | 3.3 | 0.0000 | 0.3000 |
| 0.9 | 0.4986 | 0.6986 | 3.4 | 0.0000 | 0.3000 |
| 1 | 0.4387 | 0.6387 | 3.5 | 0.0000 | 0.3000 |
| 1.1 | 0.3817 | 0.5817 | 3.6 | 0.0000 | 0.3000 |
| 1.2 | 0.3284 | 0.5284 | 3.7 | 0.0000 | 0.3000 |
| 1.3 | 0.2796 | 0.4796 | 3.8 | 0.0000 | 0.3000 |
| 1.4 | 0.2362 | 0.4362 | 3.9 | 0.0000 | 0.3000 |
| 1.5 | 0.1984 | 0.3984 | 4 | 0.0000 | 0.3000 |
| 1.6 | 0.1667 | 0.3667 | 4.1 | 0.0000 | 0.3000 |
| 1.7 | 0.1416 | 0.3416 | 4.2 | 0.0000 | 0.3000 |
| 1.8 | 0.1221 | 0.3221 | 4.3 | 0.0000 | 0.3000 |
| 1.9 | 0.1081 | 0.3081 | 4.4 | 0.0000 | 0.3000 |
| 2 | 0.0000 | 0.3000 | 4.5 | 0.0000 | 0.3000 |
| 2.1 | 0.0000 | 0.3000 | 4.6 | 0.0000 | 0.3000 |
| 2.2 | 0.0000 | 0.3000 | 4.7 | 0.0000 | 0.3000 |
| 2.3 | 0.0000 | 0.3000 | 4.8 | 0.0000 | 0.3000 |
| 2.4 | 0.0000 | 0.3000 | 4.9 | 0.0000 | 0.3000 |
|  |  |  | 5 | 0.0000 | 0.3000 |

Based on the value defined in Table D.4.3-1, Figure D.4.3-1 shows the pass/fail and reference curve of temporal correlation.

Figure D.4.3-1: Pass/fail limits and targets of Temporal correlation for CDL-C UMi channel model: red curve (reference), blue (upper limit) and green (lower limit)

## D.4.4	Pass/Fail Criteria of PSP

This clause defines the pass/fail criteria of PSP, this general pass/fail limits principle apply for all FR2 frequency bands.

The PSP pass/fail limit is specified as 84%.

## D.4.5	Pass/Fail Criteria of Cross-polarization

This clause defines the pass/fail criteria of cross-polarization, this pass/fail limits apply for all FR2 frequency bands.

The cross-polarization ratio pass/fail limit is specified as ±1.5 dB.

## D.4.6	Pass/Fail Criteria of Power validation

This clause defines the pass/fail criteria of power validation, this pass/fail limits apply for all FR2 bands.

The power validation pass/fail limit is specified as 1.5 dB.

Annex E (normative):
<gNB configurations>

# E.1	FR1 gNB configurations

The gNodeB emulator parameters shall be set according to Table E.1-1 for FR1 common parameters, Table E.1-2 for FR1 FDD 2x2 test parameters, Table E.1-3 for FR1 TDD 2x2 test parameters, Table E.1-4 for FR1 FDD 4x4 test parameters, and Table E.1-5 for FR1 TDD 4x4 test parameters.

Table E.1-1: FR1 Common test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| Carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 2) |  | RBs | 0 |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
| DL BWP configuration #1 | Cyclic prefix |  |  | Normal |
|  | RB offset |  | RBs | 0 |
|  | Number of contiguous PRB |  | PRBs | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-1 for tested channel bandwidth and subcarrier spacing |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
|  | First DMRS position for Type A PDSCH mapping |  |  | 2 |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |
|  | Symbols with PDCCH |  | Symbols | 0, 1 |
|  | Number of PRBs in CORESET |  |  | Table 5.2-2 of TS 38.101-4 for tested channel bandwidth and subcarrier spacing |
|  | Number of PDCCH candidates and aggregation levels |  |  | 1/AL8 |
|  | CCE-to-REG mapping type |  |  | Non-interleaved |
|  | DCI format |  |  | 1_1 |
|  | TCI state |  |  | TCI state #1 |
| Cross carrier scheduling |  |  |  | Not configured |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resource 1,2,3,4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset |  | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS |  |  | k0 = 0 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | Same as number of transmit antenna |
|  | CDM Type |  |  | ‘FD-CDM2’ |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  | Slots | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS |  |  | k0 = 4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | ‘FD-CDM2’ |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  | Slots | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
| PDSCH DMRS configuration | Antenna ports indexes |  |  | {1000, 1001} for Rank 2 tests {1000-1003} for Rank 4 tests |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 1 for Rank 2 tests 2 for Rank 4 tests |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from ‘CSI-RS for tracking’ configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| PT-RS configuration |  |  |  | PT-RS is not configured |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  | 1 |
| Maximum number of HARQ transmission |  |  |  | 1 |
| HARQ ACK/NACK bundling |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  | N.A |
| Precoding configuration |  |  |  | SP Type I, Random per slot with PRB bundling granularity |
| Symbols for all unused REs |  |  |  | OCNG Annex A.5 of TS 38.101-4 |
| Minimum Number of Slots per Stream |  |  |  | 20000 for 15kHz SCS 40000 for 30kHz SCS (Note 3) |
| Transmit Power Control |  |  | dBm | 13 |
| DL power level (RS EPRE of SSS) |  |  | dBm / SCS | Set at gNodeB simulator with correction from calibration |
| EPRE ratio of PDSCH to SSS |  |  | dB | 0 |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission. Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 for tested channel bandwidth and subcarrier spacing. Note 3: For FR1 MIMO OTA test lab alignments and FR1 MIMO OTA UE performance requirements, the following values can be used: For FR1 bands >1GHz: 20k for 30kHz SCS, 10k for 15kHz SCS; For FR1 bands <1GHz: 10k for 15kHz SCS; |  |  |  |  |

Table E.1-2: Test parameters for FR1 FDD 2x2

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Reference channel |  |  | R.PDSCH.1-3.1 FDD (Note 1) |
| Bandwidth |  | MHz | 10 |
| SCS |  | kHz | 15 |
| Modulation DL |  |  | 64QAM |
| Modulation UL |  |  | QPSK |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 20 |
|  | CSI-RS offset | Slots | Table 8.2-1. |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |
| Note 1: “R.PDSCH.1-3.1 FDD” is defined in Table A.3.2.1.1-3 of TS 38.101-4 |  |  |  |

Table E.1-3: Test parameters for FR1 TDD 2x2

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Reference channel |  |  | R.PDSCH.2-3.1 TDD (Note 1) |
| Bandwidth |  | MHz | 40 |
| SCS |  | kHz | 30 |
| Modulation DL |  |  | 64QAM |
| Modulation UL |  |  | QPSK |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | Table 8.2-1. |
|  | CSI-RS periodicity | Slots | 40 |
|  | CSI-RS offset | Slots | Table 8.2-1. |
| Number of HARQ Processes |  |  | 8 |
| TDD UL-DL pattern |  |  | FR1.30-1 (Note 2) |
| Note 1: “R.PDSCH.2-3.1 TDD” is defined in Table A.3.2.2.2-3 of TS 38.101-4 Note 2: “FR1.30-1” is defined in Annex A.1.2 of TS 38.101-4 |  |  |  |

Table E.1-4: Test parameters for FR1 FDD 4x4

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Reference channel |  |  | R.PDSCH.1-2.4 FDD (Note 1) |
| Bandwidth |  | MHz | 10 |
| SCS |  | kHz | 15 |
| Modulation DL |  |  | 16QAM |
| Modulation UL |  |  | QPSK |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 20 |
|  | CSI-RS offset | Slots | Table 8.2-1. |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |
| Note 1: “R.PDSCH.1-2.4 FDD” is defined in Table A.3.2.1.1-2 of TS 38.101-4 |  |  |  |

Table E.1-5: Test parameters for FR1 TDD 4x4

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Reference channel |  |  | R.PDSCH.2-2.4 TDD (Note 1) |
| Bandwidth |  | MHz | 40 |
| SCS |  | kHz | 30 |
| Modulation DL |  |  | 16QAM |
| Modulation UL |  |  | QPSK |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | Table 8.2-1. |
|  | CSI-RS periodicity | Slots | 40. |
|  | CSI-RS offset | Slots | Table 8.2-1. |
| Number of HARQ Processes |  |  | 8 |
| TDD UL-DL pattern |  |  | FR1.30-1 (Note 2) |
| Note 1: “R.PDSCH.2-2.4 TDD” is defined in Table A.3.2.2.2-2 of TS 38.101-4 Note 2: “FR1.30-1” is defined in Annex A.1.2 of TS 38.101-4 |  |  |  |

# E.2	FR2 gNB configurations

The gNodeB emulator parameters for FR2 MIMO OTA testing shall be set according to Table E.2-1 for FR2 common parameters and Table E.2-2 for FR2 TDD 2x2 test parameters.

Table E.2-1: FR2 Common test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| PTRS epre-Ratio |  |  |  | 0 |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 2) |  | RBs | 0 |
|  | Subcarrier spacing |  | kHz | 120 |
| DL BWP configuration #1 | Cyclic prefix |  |  | Normal |
|  | RB offset |  | RBs | 0 |
|  | Number of contiguous PRB |  | PRBs | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-2 for tested channel bandwidth and subcarrier spacing |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | 1 |
|  | SSB periodicity |  | ms | 20 |
|  | First DMRS position for Type A PDSCH mapping |  |  | 2 |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |
|  | Symbols with PDCCH |  |  | 0 |
|  | Number of PRBs in CORESET |  |  | Table 7.2-2 of TS 38.101-4 for tested channel bandwidth and subcarrier spacing |
|  | Number of PDCCH candidates and aggregation levels |  |  | 1/AL8 |
|  | CCE-to-REG mapping type |  |  | Non-interleaved |
|  | DCI format |  |  | 1_1 |
|  | TCI state |  |  | TCI state #1 |
| Cross carrier scheduling |  |  |  | Not configured |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 0 for CSI-RS resource 1,2,3,4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 6 for CSI-RS resource 1 and 3 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity |  | Slots | 120 kHz SCS: 160 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset |  | Slots | 120 kHz SCS: 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 0 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 12 |
|  | Number of CSI-RS ports (X) |  |  | 2 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  | Slots | 120 kHz SCS: 160 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 12 |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  | Slots | 120 kHz SCS: 160 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
| CSI-RS for beam refinement | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resource 1,2 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2 |
|  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2 |
|  | CSI-RS periodicity |  | Slots | 60 kHz SCS: 80 for CSI-RS resource 1,2 120 kHz SCS: 160 for CSI-RS resource 1,2 |
|  | CSI-RS offset |  | Slots | 0 for CSI-RS resource 1,2 |
|  | QCL info |  |  | TCI state #1 |
| PDSCH DMRS configuration | Antenna ports indexes |  |  | {1000} for Rank 1 tests {1000, 1001} for Rank 2 tests |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 1 |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type D |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from ‘CSI-RS for tracking’ configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 1 from ‘CSI-RS for tracking’ configuration |
|  |  | QCL Type |  | Type D |
| PTRS configuration | Frequency density (KPT-RS) |  |  | 2 |
|  | Time density (LPT-RS) |  |  | 1 |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  | 1 |
| Maximum number of HARQ transmission |  |  |  | 1 |
| HARQ ACK/NACK bundling |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |
| Precoding configuration |  |  |  | SP Type I, Random per slot with PRB bundling granularity |
| Symbols for all unused Res |  |  |  | OCNG in Annex A.5 of TS 38.101-4 |
| Minimum Number of Slots per Stream |  |  |  | 20000 for FR2 UMi CDL-C |
| Transmit Power Control |  |  | dBm | 13 dBm |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission. Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-2 for tested channel bandwidth and subcarrier spacing. |  |  |  |  |

Table E.2-2: Test parameters for FR2 TDD 2x2

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Reference channel |  |  | R.PDSCH.5-2.2 TDD (Note 1) |
| Bandwidth |  | MHz | 100 |
| SCS |  | kHz | 120 |
| Modulation DL |  |  | 16QAM |
| Modulation UL |  |  | QPSK |
| Active DL BWP index |  |  | 1 |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Table E.2-1 |
|  | CSI-RS offset | Slots | Table E.2-1 |
| PDCCH configuration | Number of PDCCH candidates and aggregation levels |  | 1/AL8 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | Specific to each Reference channel as defined in A.3.2.2 of TS 38.101-4 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | WB for Test 1-1, 2 for other tests |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| TDD UL-DL pattern |  |  | FR2.120-1 (Note2) |
| Note 1: “R.PDSCH.5-2.2 TDD” is defined in Table A.3.2.2.5-2 of TS 38.101-4 Note 2: “FR2.120-1” is defined in Annex A.1.3 of TS 38.101-4 |  |  |  |

Annex F (normative):
<Environmental requirements>

# F.1	Scope

The requirements in this clause apply to all types of UE(s) in FR1 and FR2.

# F.2	Ambient temperature

All the MIMO OTA requirements are applicable in room temperature e.g. 25C.

# F.3	Operating voltage

For FR1 MIMO OTA, all test cases shall be performed in the normal voltage condition with the DUT operated in stand-alone battery powered mode. No extreme voltage testing is required. It is recommended to start testing with a fully charged battery and conclude and/or pause testing before the battery has completely lost its charge.

For FR2 MIMO OTA, all test cases shall be performed in the normal voltage condition with the DUT operated in stand-alone battery powered mode or external power source. No extreme voltage testing is required. It shall be demonstrated that the impact of external power source to device performance is negligible comparing to stand-alone battery powered mode. It is recommended to start testing with a fully charged battery and conclude and/or pause testing before the battery has completely lost its charge.

Annex G (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-08 | RAN4#96-e | R4-2012709 |  |  |  | Initial Skeleton | 0.0.1 |
| 2020-11 | RAN4#97-e | R4-2016216 |  |  |  | R4-2016218, TP to TS 38.151 v0.0.1 on general part R4-2016222, TP to TS 38.151 v0.0.1 on FR1 test system for requirements R4-2017584, TP to TS 38.151 v0.0.1 on FR1 Channel model and RMC | 0.1.0 |
| 2021-02 | RAN4#98-e | R4-2101822 |  |  |  | R4-2103969 TP to TS38.151 v0.1.0 on FR2 Channel model and RMC R4-2103970 TP to TS38.151 v0.1.0 on FR2 test system for requirements R4-2103971 TP to TS38.151 v0.1.0 on Performance metrics for NR MIMO OTA requirements | 0.2.0 |
| 2021-04 | RAN4#98-bis-e | R4-2104512 |  |  |  | R4-2106094 TP to TS38.151 v0.2.0 on FR1 Channel model R4-2104511 TP to TS38.151 v0.2.0 on FR1 Calibration and Test procedure R4-2106093 TP to TS38.151: revision on MIMO Average Spherical Coverage | 0.3.0 |
| 2021-05 | RAN4#99-e | R4-2108094 |  |  |  | R4-2108614 TP to TS38.151 v0.3.0 on FR1 TRMS R4-2109660 TP to TS38.151 v0.3.0 on CDL-C UMi channel model R4-2108615 TP to TS38.151 v0.3.0 on Power validation | 0.4.0 |
| 2021-08 | RAN4#100-e | R4-2112976 |  |  |  | R4-2112977 Rapporteur input to TS38.151 R4-2112979 TP to TS38.151 on Minimum Number of Slots and Power Control R4-2115757 TP on Channel Model and DUT Positioning Clarifications R4-2115811 TP to TS38.151 on FR2 PAS similarity percentage | 0.5.0 |
| 2021-11 | RAN4#101-e | R4-2118299 |  |  |  | R4-2118302 TP to TS 38.151 on updated structure R4-2119377 Clarification of NF Compensation for FR2 PSP | 0.6.0 |
| 2022-01 | RAN4#101-bis-e | R4-2200966 |  |  |  | R4-2201920 Illustration of Device Orientations for Select Test Points R4-2200780 TP on TS 38.151 for test parameters of FR2 performance R4-2200967 TP to TS38.151 on FR2 maximum downlink power and test procedure R4-2203064 TP to TS38.151 on FR1 Spatial Channel Model Validation | 0.7.0 |
| 2022-03 | RAN4#102-e | R4-2204945 |  |  |  | R4-2204572 TP to TS 38.151 on FR1 MIMO OTA test parameter R4-2207306 TP to TS38.151 on FR1 MPAC MU budget R4-2207307 TP to TS38.151 on channel model validation limits R4-2207308 TP to TS38.151 on FR1 Temporal Correlation Validation – Time domain technique | 0.8.0 |
| 2022-03 | RAN#95-e | RP-220793 |  |  |  | For one-step approval in RAN Plenary | 1.0.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-03 | RAN#95 |  |  |  |  | Approved by plenary – Rel-17 spec under change control | 17.0.0 |
| 2022-06 | RAN#96 | RP-221662 | 0001 |  | F | BIG CR to 38.151 | 17.1.0 |
| 2022-09 | RAN#97 | RP-222039 | 0003 |  | B | Big CR to 38.151: Introduction MIMO OTA performance requirements (Rel-17, CAT B) | 17.2.0 |
| 2022-12 | RAN#98-e | RP-223310 | 0004 |  | F | CR to TS 38.151 on FR2 MU | 17.3.0 |
| 2023-06 | RAN#100 | RP-231342 | 0005 | 1 | F | CR to TS38.151 on FR1 power validation pass fail limit | 17.4.0 |
| 2023-06 | RAN#100 | RP-231342 | 0006 |  | F | CR to TS 38.151 on FR1 spatial correlation pass/fail limits | 17.4.0 |
| 2023-06 | RAN#100 | RP-231342 | 0007 |  | F | CR on TS 38.151 for clarifications on FR1 channel model parameters | 17.4.0 |
| 2023-06 | RAN#100 | RP-231342 | 0008 | 1 | F | Update on PDP Targets for FR2 CDL-C channel model | 17.4.0 |
| 2023-06 | RAN#100 | RP-231342 | 0009 | 1 | F | Update on TCF Test Methodology for FR2 CDL-C channel model | 17.4.0 |
| 2023-06 | RAN#100 | RP-231342 | 0012 | 1 | F | On TS 38.151 Annex C editorial updates | 17.4.0 |
| 2023-09 | RAN#101 | RP-232517 | 0016 | 1 | F | On Test Methodology for FR2 Channel Model Power Validation NOTE: This CR was not implemented. Author allocated it for Rel-18 in 3GU and not Rel-17. | 17.5.0 |
| 2023-09 | RAN#101 | RP-232492 | 0017 |  | F | CR to TS 38.151 on Definitions of terms | 17.5.0 |
| 2023-09 | RAN#101 | RP-232517 | 0018 | 1 | F | FR2 CM Validation Corrections | 17.5.0 |
| 2023-12 | RAN#102 | RP-233365 | 0019 | 1 | F | Updates to FR1 Channel model validation | 17.6.0 |
| 2023-12 | RAN#102 | RP-233348 | 0020 |  | F | CR to update preliminary FR2 MU | 17.6.0 |
| 2023-12 | RAN#102 | RP-233348 | 0021 | 1 | F | CR to TS 38.151 on test configurations | 17.6.0 |
| 2023-12 | RAN#102 | RP-233348 | 0022 | 1 | F | CR on introduction of applicability rules for MIMO OTA requirements | 17.6.0 |
| 2023-12 | RAN#102 | RP-233348 | 0023 |  | F | CR to TS 38.151 on FR2 channel model validation pass/fail limits | 17.6.0 |
| 2024-03 | RAN#103 | RP-240561 | 0024 | 1 | F | CR to TS 38.151 on FR1 MIMO OTA spatial correlation validation pass/fail limits | 17.7.0 |
| 2024-03 | RAN#103 | RP-240608 | 0025 | 1 | F | CR to 38.151 on FR1 MIMO OTA MU | 17.7.0 |
| 2024-03 | RAN#103 | RP-240608 | 0027 |  | F | Update to FR1 Channel model validation | 17.7.0 |
| 2024-03 | RAN#103 | RP-240561 | 0029 |  | F | (NR_MIMO_OTA-Core) Correction on the example inter-band band combination tables | 17.7.0 |
| 2024-06 | RAN#104 | RP-241486 | 0036 |  | F | Update to FR1 Calibration Procedure | 17.8.0 |
| 2024-06 | RAN#104 | RP-241399 | 0039 |  | F | CR for 38151 on UE positioning for FR1 MIMO OTA | 17.8.0 |
| 2024-09 | RAN#105 | RP-242169 | 0041 |  | F | Autocorrelation Channel Model Speed Correction | 17.9.0 |
| 2024-09 | RAN#105 | RP-242169 | 0043 |  | F | Clarification of voltage environmental requirement | 17.9.0 |
| 2024-12 | RAN#106 | RP-243050 | 0046 |  | F | Correction of FR2 CM Validation Setups | 17.10.0 |
| 2024-12 | RAN#106 | RP-243050 | 0048 |  | F | Correction of FR1 CM Validation Setups | 17.10.0 |