# 6	Transmitter characteristics

## 6.0	General test requirements for ΔTIB,c

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero.

In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

## 6.1	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed.

For UEs that do not indicate IE dualPA-Architecture, transmitter requirements for CA operation apply only when the DMRS initialization parameters (including the case when the UE applies cell ID as DMRS scrambling ID) are different across all CCs. The UE may use higher MPR values outside this limitation.

Transmitter requirements for UL MIMO operation apply when the UE transmits on 2 ports on the same CDM group. The UE may use higher MPR values outside this limitation.

Unless otherwise stated, Channel Bandwidth shall be prioritized in the selecting of test points. Subcarrier spacing shall be selected after Test Channel Bandwidth is selected.

The transmitter requirements other than UL MIMO (suffix D), Tx Diversity (suffix G) and Carrier Aggregation with UL MIMO (suffix H) shall be measured on a single UL antenna connector for each configured UL band. In case of UL CA or NR-DC (suffix A, suffix B) the requirements shall be measured on a single UL antenna connector for each UL band. In case the UE supports dualPA-Architecture according to the corresponding IE, the requirements are measured on the corresponding antenna connector for each carrier, except for the Output RF spectrum emissions (clause 6.5A) which are measured across all antenna connectors for a given band.

Uplink RB allocations given in Table 6.1-1 are used throughout this section, unless otherwise stated by the test case.

Table 6.1-1: Common uplink configuration

| Channel Bandwidth | SCS(kHz) | OFDM | RB allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Edge_Full_Left (Note 2) | Edge_Full_Right (Note 2) | Edge_1RB_Left | Edge_1RB_Right | Outer_Full | Inner_Full | Inner_1RB_Left | Inner_1RB_Right |
| 3MHz | 15 | DFT-s | 2@0 | 2@13 | 1@0 | 1@14 | 15@0 | 6@4 | 1@1 | 1@13 |
|  |  | CP | 2@0 | 2@13 | 1@0 | 1@14 | 15@0 | 7@4 | 1@1 | 1@13 |
| 5MHz | 15 | DFT-s | 2@0 | 2@23 | 1@0 | 1@24 | 25@0 | 12@6 | 1@1 | 1@23 |
|  |  | CP | 2@0 | 2@23 | 1@0 | 1@24 | 25@0 | 13@6 | 1@1 | 1@23 |
|  | 30 | DFT-s | 2@0 | 2@9 | 1@0 | 1@10 | 10@0 | 5@21 | 1@1 | 1@9 |
|  |  | CP | 2@0 | 2@9 | 1@0 | 1@10 | 11@0 | 5@21 | 1@1 | 1@9 |
|  | 60 | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | CP | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 10MHz | 15 | DFT-s | 2@0 | 2@50 | 1@0 | 1@51 | 50@0 | 25@12 | 1@1 | 1@50 |
|  |  | CP | 2@0 | 2@50 | 1@0 | 1@51 | 52@0 | 26@13 | 1@1 | 1@50 |
|  | 30 | DFT-s | 2@0 | 2@22 | 1@0 | 1@23 | 24@0 | 12@6 | 1@1 | 1@22 |
|  |  | CP | 2@0 | 2@22 | 1@0 | 1@23 | 24@0 | 12@6 | 1@1 | 1@22 |
|  | 60 | DFT-s | 2@0 | 2@9 | 1@0 | 1@10 | 10@0 | 5@21 | 1@1 | 1@9 |
|  |  | CP | 2@0 | 2@9 | 1@0 | 1@10 | 11@0 | 5@21 | 1@1 | 1@9 |
| 15MHz | 15 | DFT-s | 2@0 | 2@77 | 1@0 | 1@78 | 75@0 | 36@181 | 1@1 | 1@77 |
|  |  | CP | 2@0 | 2@77 | 1@0 | 1@78 | 79@0 | 39@191 | 1@1 | 1@77 |
|  | 30 | DFT-s | 2@0 | 2@36 | 1@0 | 1@37 | 36@0 | 18@9 | 1@1 | 1@36 |
|  |  | CP | 2@0 | 2@36 | 1@0 | 1@37 | 38@0 | 19@9 | 1@1 | 1@36 |
|  | 60 | DFT-s | 2@0 | 2@16 | 1@0 | 1@17 | 18@0 | 9@4 | 1@1 | 1@16 |
|  |  | CP | 2@0 | 2@16 | 1@0 | 1@17 | 18@0 | 9@4 | 1@1 | 1@16 |
| 20MHz | 15 | DFT-s | 2@0 | 2@104 | 1@0 | 1@105 | 100@0 | 50@25 | 1@1 | 1@104 |
|  |  | CP | 2@0 | 2@104 | 1@0 | 1@105 | 106@0 | 53@26 | 1@1 | 1@104 |
|  | 30 | DFT-s | 2@0 | 2@49 | 1@0 | 1@50 | 50@0 | 25@12 | 1@1 | 1@49 |
|  |  | CP | 2@0 | 2@49 | 1@0 | 1@50 | 51@0 | 25@121 | 1@1 | 1@49 |
|  | 60 | DFT-s | 2@0 | 2@22 | 1@0 | 1@23 | 24@0 | 12@6 | 1@1 | 1@22 |
|  |  | CP | 2@0 | 2@22 | 1@0 | 1@23 | 24@0 | 12@6 | 1@1 | 1@22 |
| 25MHz | 15 | DFT-s | 2@0 | 2@131 | 1@0 | 1@132 | 128@0 | 64@32 | 1@1 | 1@131 |
|  |  | CP | 2@0 | 2@131 | 1@0 | 1@132 | 133@0 | 67@33 | 1@1 | 1@131 |
|  | 30 | DFT-s | 2@0 | 2@63 | 1@0 | 1@64 | 64@0 | 32@16 | 1@1 | 1@63 |
|  |  | CP | 2@0 | 2@63 | 1@0 | 1@64 | 65@0 | 33@16 | 1@1 | 1@63 |
|  | 60 | DFT-s | 2@0 | 2@29 | 1@0 | 1@30 | 30@0 | 15@71 | 1@1 | 1@29 |
|  |  | CP | 2@0 | 2@29 | 1@0 | 1@30 | 31@0 | 15@71 | 1@1 | 1@29 |
| 30MHz | 15 | DFT-s | 2@0 | 2@158 | 1@0 | 1@159 | 160@0 | 80@40 | 1@1 | 1@158 |
|  |  | CP | 2@0 | 2@158 | 1@0 | 1@159 | 160@0 | 80@40 | 1@1 | 1@158 |
|  | 30 | DFT-s | 2@0 | 2@76 | 1@0 | 1@77 | 75@0 | 36@18 | 1@1 | 1@76 |
|  |  | CP | 2@0 | 2@76 | 1@0 | 1@77 | 78@0 | 39@19 | 1@1 | 1@76 |
|  | 60 | DFT-s | 2@0 | 2@36 | 1@0 | 1@37 | 36@0 | 18@9 | 1@1 | 1@36 |
|  |  | CP | 2@0 | 2@36 | 1@0 | 1@37 | 38@0 | 19@9 | 1@1 | 1@36 |
| 35MHz | 15 | DFT-s | 2@0 | 2@186 | 1@0 | 1@187 | 180@0 | 90@45 | 1@1 | 1@186 |
|  |  | CP | 2@0 | 2@186 | 1@0 | 1@187 | 188@0 | 94@47 | 1@1 | 1@186 |
|  | 30 | DFT-s | 2@0 | 2@90 | 1@0 | 1@91 | 90@0 | 45@22 | 1@1 | 1@90 |
|  |  | CP | 2@0 | 2@90 | 1@0 | 1@91 | 92@0 | 46@23 | 1@1 | 1@90 |
|  | 60 | DFT-s | 2@0 | 2@42 | 1@0 | 1@43 | 40@0 | 20@10 | 1@1 | 1@42 |
|  |  | CP | 2@0 | 2@42 | 1@0 | 1@43 | 44@0 | 22@11 | 1@1 | 1@42 |
| 40MHz | 15 | DFT-s | 2@0 | 2@214 | 1@0 | 1@215 | 216@0 | 108@54 | 1@1 | 1@214 |
|  |  | CP | 2@0 | 2@214 | 1@0 | 1@215 | 216@0 | 108@54 | 1@1 | 1@214 |
|  | 30 | DFT-s | 2@0 | 2@104 | 1@0 | 1@105 | 100@0 | 50@25 | 1@1 | 1@104 |
|  |  | CP | 2@0 | 2@104 | 1@0 | 1@105 | 106@0 | 53@26 | 1@1 | 1@104 |
|  | 60 | DFT-s | 2@0 | 2@49 | 1@0 | 1@50 | 50@0 | 25@12 | 1@1 | 1@49 |
|  |  | CP | 2@0 | 2@49 | 1@0 | 1@50 | 51@0 | 25@121 | 1@1 | 1@49 |
| 45MHz | 15 | DFT-s | 2@0 | 2@240 | 1@0 | 1@241 | 240@0 | 120@60 | 1@1 | 1@240 |
|  |  | CP | 2@0 | 2@240 | 1@0 | 1@241 | 242@0 | 121@60 | 1@1 | 1@240 |
|  | 30 | DFT-s | 2@0 | 2@117 | 1@0 | 1@118 | 108@0 | 54@271 | 1@1 | 1@117 |
|  |  | CP | 2@0 | 2@117 | 1@0 | 1@118 | 119@0 | 59@291 | 1@1 | 1@117 |
|  | 60 | DFT-s | 2@0 | 2@56 | 1@0 | 1@57 | 54@0 | 27@13 | 1@1 | 1@56 |
|  |  | CP | 2@0 | 2@56 | 1@0 | 1@57 | 58@0 | 29@14 | 1@1 | 1@56 |

| Channel Bandwidth | SCS(kHz) | OFDM | RB allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Edge_Full_Left (Note 2) | Edge_Full_Right (Note 2) | Edge_1RB_Left | Edge_1RB_Right | Outer_Full | Inner_Full | Inner_1RB_Left | Inner_1RB_Right |
| 50MHz | 15 | DFT-s | 2@0 | 2@268 | 1@0 | 1@269 | 270@0 | 135@67 | 1@1 | 1@268 |
|  |  | CP | 2@0 | 2@268 | 1@0 | 1@269 | 270@0 | 135@67 | 1@1 | 1@268 |
|  | 30 | DFT-s | 2@0 | 2@131 | 1@0 | 1@132 | 128@0 | 64@32 | 1@1 | 1@131 |
|  |  | CP | 2@0 | 2@131 | 1@0 | 1@132 | 133@0 | 67@33 | 1@1 | 1@131 |
|  | 60 | DFT-s | 2@0 | 2@63 | 1@0 | 1@64 | 64@0 | 32@16 | 1@1 | 1@63 |
|  |  | CP | 2@0 | 2@63 | 1@0 | 1@64 | 65@0 | 33@16 | 1@1 | 1@63 |
| 60MHz | 15 | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | CP | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 30 | DFT-s | 2@0 | 2@160 | 1@0 | 1@161 | 162@0 | 81@40 | 1@1 | 1@160 |
|  |  | CP | 2@0 | 2@160 | 1@0 | 1@161 | 162@0 | 81@40 | 1@1 | 1@160 |
|  | 60 | DFT-s | 2@0 | 2@77 | 1@0 | 1@78 | 75@0 | 36@181 | 1@1 | 1@77 |
|  |  | CP | 2@0 | 2@77 | 1@0 | 1@78 | 79@0 | 39@191 | 1@1 | 1@77 |
| 70MHz | 15 | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | CP | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 30 | DFT-s | 2@0 | 2@187 | 1@0 | 1@188 | 180@0 | 90@45 | 1@1 | 1@187 |
|  |  | CP | 2@0 | 2@187 | 1@0 | 1@188 | 189@0 | 95@47 | 1@1 | 1@187 |
|  | 60 | DFT-s | 2@0 | 2@91 | 1@0 | 1@92 | 90@0 | 45@22 | 1@1 | 1@91 |
|  |  | CP | 2@0 | 2@91 | 1@0 | 1@92 | 93@0 | 47@23 | 1@1 | 1@91 |
| 80MHz | 15 | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | CP | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 30 | DFT-s | 2@0 | 2@215 | 1@0 | 1@216 | 216@0 | 108@54 | 1@1 | 1@215 |
|  |  | CP | 2@0 | 2@215 | 1@0 | 1@216 | 217@0 | 109@54 | 1@1 | 1@215 |
|  | 60 | DFT-s | 2@0 | 2@105 | 1@0 | 1@106 | 100@0 | 50@251 | 1@1 | 1@105 |
|  |  | CP | 2@0 | 2@105 | 1@0 | 1@106 | 107@0 | 53@261 | 1@1 | 1@105 |
| 90MHz | 15 | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | CP | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 30 | DFT-s | 2@0 | 2@243 | 1@0 | 1@244 | 243@0 | 120@60 | 1@1 | 1@243 |
|  |  | CP | 2@0 | 2@243 | 1@0 | 1@244 | 245@0 | 123@61 | 1@1 | 1@243 |
|  | 60 | DFT-s | 2@0 | 2@119 | 1@0 | 1@120 | 120@0 | 60@30 | 1@1 | 1@119 |
|  |  | CP | 2@0 | 2@119 | 1@0 | 1@120 | 121@0 | 61@30 | 1@1 | 1@119 |
| 100MHz | 15 | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | CP | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 30 | DFT-s | 2@0 | 2@271 | 1@0 | 1@272 | 270@0 | 135@67 | 1@1 | 1@271 |
|  |  | CP | 2@0 | 2@271 | 1@0 | 1@272 | 273@0 | 137@68 | 1@1 | 1@271 |
|  | 60 | DFT-s | 2@0 | 2@133 | 1@0 | 1@134 | 135@0 | 64@32 | 1@1 | 1@133 |
|  |  | CP | 2@0 | 2@133 | 1@0 | 1@134 | 135@0 | 67@331 | 1@1 | 1@133 |
| Note 1: The allocated RB number LCRB is ceil(NRB/2) -1 in order to meet Inner RB allocation definition (RBStart,Low ≤ RBStart ≤ RBStart,High) described in subclause 6.2.2 of TS 38.101-1 [2]. Note 2: For power class 1.5, Edge_Full_Left is defined as 4 RBs allocated at the left edge of the transmission bandwidth (4@0), and Edge_Full_Right is defined as 4 RBs allocated at the right edge of the transmission bandwidth (4@NRB-4). |  |  |  |  |  |  |  |  |  |  |

## 6.1A	General

Editor’s note:

Uplink RB allocations for intra-band UL CA with mixed numerology is FFS

The minimum requirements for band combinations including Band n41 also apply for the corresponding band combinations with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said band combinations with Band n90 are not listed in the tables below but are covered by this specification.

Uplink RB allocations for intra-band CA given in Table 6.1A-1a are used throughout this section, unless otherwise stated by the test case.

Table 6.1A-1a: Common uplink configuration for intra-band contiguous 2UL CA (contiguous RB allocation)

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | 5+15 | CP | 52 | 0@0 | 52@1 | 104 | 25@0 | 79@0 |
|  |  |  | DFT-s | 50 | 0@0 | 50@0 | 100 | 25@0 | 75@0 |
| 20 |  | 10+10 | CP | 52 | 26@26 | 26@0 | 104 | 52@0 | 52@0 |
|  |  |  | DFT-s | 50 | 25@27 | 25@0 | 100 | 50@2 | 50@0 |
|  |  | 15+5 | CP | 52 | 52@26 | 0@0 | 104 | 79@0 | 25@0 |
|  |  |  | DFT-s | 50 | 50@29 | 0@0 | 100 | 75@4 | 25@0 |
|  | 15 | 5+20 | CP | 654 | 0@0 | 65@7 | 131 | 25@0 | 106@0 |
|  |  |  | DFT-s | 64 | 0@0 | 64@7 | 125 | 25@0 | 100@0 |
|  |  | 10+15 | CP | 654 | 20@32 | 45@0 | 131 | 52@0 | 79@0 |
|  |  |  | DFT-s | 65 | 20@32 | 45@0 | 125 | 50@2 | 75@0 |
|  |  | 15+10 | CP | 654 | 47@32 | 18@0 | 131 | 79@0 | 52@0 |
|  |  |  | DFT-s | 65 | 45@34 | 20@0 | 125 | 75@4 | 50@0 |
|  |  | 20+5 | CP | 654 | 65@32 | 0@0 | 131 | 106@0 | 25@0 |
| 25 |  |  | DFT-s | 64 | 64@35 | 0@0 | 125 | 100@6 | 25@0 |
|  | 30 | 10+15 | CP | 604 | 9@15 | 21@0 | 124 | 24@0 | 38@0 |
|  |  |  | DFT-s | 58 | 9@15 | 20@0 | 120 | 24@0 | 36@0 |
|  |  | 15+10 | CP | 604 | 23@15 | 7@0 | 124 | 38@0 | 24@0 |
|  |  |  | DFT-s | 58 | 20@18 | 9@0 | 120 | 36@2 | 24@0 |
|  | 15 | 10+20 | CP | 79 | 13@39 | 66@0 | 158 | 52@0 | 106@0 |
|  |  |  | DFT-s | 76 | 12@40 | 64@0 | 150 | 50@2 | 100@0 |
|  |  | 15+15 | CP | 79 | 40@39 | 39@0 | 158 | 79@0 | 79@0 |
|  |  |  | DFT-s | 76 | 40@39 | 36@0 | 150 | 75@4 | 75@0 |
|  |  | 20+10 | CP | 79 | 67@39 | 12@0 | 158 | 106@0 | 52@0 |
|  |  |  | DFT-s | 76 | 64@42 | 12@0 | 150 | 100@6 | 50@0 |
|  | 30 | 10+20 | CP | 74 | 5@19 | 32@0 | 150 | 24@0 | 51@0 |
|  |  |  | DFT-s | 74 | 5@19 | 32@0 | 148 | 24@0 | 50@0 |
| 30 |  | 15+15 | CP | 76 | 19@19 | 19@0 | 152 | 38@0 | 38@0 |
|  |  |  | DFT-s | 72 | 20@18 | 16@0 | 144 | 36@2 | 36@0 |
|  |  | 20+10 | CP | 74 | 32@19 | 5@0 | 150 | 51@0 | 24@0 |
|  |  |  | DFT-s | 74 | 32@19 | 5@0 | 148 | 50@1 | 24@0 |
|  | 15 | 15+20 | CP | 93 | 33@46 | 60@0 | 185 | 79@0 | 106@0 |
|  |  |  | DFT-s | 92 | 32@47 | 60@0 | 175 | 75@4 | 100@0 |
|  |  | 20+15 | CP | 93 | 60@46 | 33@0 | 185 | 106@0 | 79@0 |
|  |  |  | DFT-s | 92 | 60@46 | 32@0 | 175 | 100@6 | 75@0 |
|  | 30 | 15+20 | CP | 88 | 16@22 | 28@0 | 178 | 38@0 | 51@0 |
|  |  |  | DFT-s | 86 | 16@22 | 27@0 | 172 | 36@2 | 50@0 |
| 35 |  | 20+15 | CP | 88 | 29@22 | 15@0 | 178 | 51@0 | 38@0 |
|  |  |  | DFT-s | 86 | 27@24 | 16@0 | 172 | 50@1 | 36@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | 10+30 | CP | 106 | 0@0 | 106@1 | 212 | 52@0 | 160@0 |
|  |  |  | DFT-s | 100 | 0@52 | 100@0 | 210 | 50@2 | 160@0 |
| 40 |  | 20+20 | CP | 106 | 53@53 | 53@0 | 212 | 106@0 | 106@0 |
|  |  |  | DFT-s | 104 | 54@52 | 50@0 | 200 | 100@6 | 100@0 |
|  |  | 30+10 | CP | 106 | 106@53 | 0@0 | 212 | 160@0 | 52@0 |
|  |  |  | DFT-s | 100 | 100@60 | 0@0 | 210 | 160@0 | 50@0 |
|  | 30 | 10+30 | CP | 1004 | 0@0 | 50@1 | 204 | 24@0 | 78@0 |
|  |  |  | DFT-s | 100 | 0@0 | 50@1 | 198 | 24@0 | 75@0 |
|  |  | 20+20 | CP | 1004 | 26@25 | 24@0 | 204 | 51@0 | 51@0 |
|  |  |  | DFT-s | 100 | 25@26 | 25@0 | 200 | 50@1 | 50@0 |
|  |  | 30+10 | CP | 1004 | 50@25 | 0@0 | 204 | 78@0 | 24@0 |
|  |  |  | DFT-s | 100 | 50@27 | 0@0 | 198 | 75@3 | 24@0 |
|  | 15 | 15+30 | CP | 1064 | 26@53 | 80@0 | 239 | 79@0 | 160@0 |
|  |  |  | DFT-s | 118 | 18@61 | 100@0 | 235 | 75@4 | 160@0 |
| 45 | 30 | 15+30 | CP | 116 | 9@29 | 49@0 | 232 | 38@0 | 78@0 |
|  |  |  | DFT-s | 114 | 9@29 | 48@0 | 222 | 36@2 | 75@0 |
|  | 15 | 10+40 | CP | 134 | 0@0 | 134@15 | 268 | 52@0 | 216@0 |
|  |  |  | DFT-s | 128 | 0@0 | 128@12 | 266 | 50@2 | 216@0 |
|  |  | 20+30 | CP | 133 | 40@66 | 93@0 | 266 | 106@0 | 160@0 |
|  |  |  | DFT-s | 130 | 40@66 | 90@0 | 260 | 100@6 | 160@0 |
|  |  | 30+20 | CP | 133 | 94@66 | 39@0 | 266 | 160@0 | 106@0 |
|  |  |  | DFT-s | 130 | 90@70 | 40@0 | 260 | 160@0 | 100@0 |
| 50 |  | 40+10 | CP | 134 | 134@67 | 0@0 | 268 | 216@0 | 52@0 |
|  |  |  | DFT-s | 128 | 128@76 | 0@0 | 266 | 216@0 | 50@0 |
|  | 30 | 10+40 | CP | 1284 | 0@0 | 64@8 | 260 | 24@0 | 106@0 |
|  |  |  | DFT-s | 128 | 0@0 | 64@8 | 248 | 24@0 | 100@0 |
|  |  | 20+30 | CP | 128 | 19@32 | 45@0 | 258 | 51@0 | 78@0 |
|  |  |  | DFT-s | 126 | 18@33 | 45@0 | 250 | 50@1 | 75@0 |
|  |  | 30+20 | CP | 128 | 46@32 | 18@0 | 258 | 78@0 | 51@0 |
|  |  |  | DFT-s | 126 | 45@33 | 18@0 | 250 | 75@3 | 50@0 |
|  |  | 40+10 | CP | 1284 | 64@32 | 0@0 | 260 | 106@0 | 24@0 |
|  |  |  | DFT-s | 128 | 64@34 | 0@0 | 248 | 100@6 | 24@0 |
|  | 15 | 15+40 | CP | 1474 | 6@73 | 141@0 | 295 | 79@0 | 216@0 |
|  |  |  | DFT-s | 143 | 8@71 | 135@0 | 291 | 75@4 | 216@0 |
| 55 | 30 | 15+40 | CP | 144 | 2@36 | 70@0 | 288 | 38@0 | 106@0 |
|  |  |  | DFT-s | 136 | 4@34 | 64@0 | 272 | 36@2 | 100@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | 10+50 | CP | 161 | 0@0 | 161@28 | 322 | 52@0 | 270@0 |
|  |  |  | DFT-s | 160 | 0@0 | 160@28 | 320 | 50@2 | 270@0 |
|  |  | 20+40 | CP | 161 | 26@80 | 135@0 | 322 | 106@0 | 216@0 |
|  |  |  | DFT-s | 160 | 25@81 | 135@0 | 316 | 100@6 | 216@0 |
|  |  | 30+30 | CP | 160 | 80@80 | 80@0 | 320 | 160@0 | 160@0 |
|  |  |  | DFT-s | 160 | 80@80 | 80@0 | 320 | 160@0 | 160@0 |
|  |  | 40+20 | CP | 161 | 136@80 | 25@0 | 322 | 216@0 | 106@0 |
|  |  |  | DFT-s | 160 | 135@81 | 25@0 | 316 | 216@0 | 100@0 |
|  |  | 50+10 | CP | 161 | 161@80 | 0@0 | 322 | 270@0 | 52@0 |
|  |  |  | DFT-s | 160 | 160@82 | 0@0 | 320 | 270@0 | 50@0 |
|  | 30 | 10+50 | CP | 156 | 0@0 | 78@15 | 314 | 24@0 | 133@0 |
|  |  |  | DFT-s | 150 | 0@0 | 75@14 | 304 | 24@0 | 128@0 |
| 60 |  | 20+40 | CP | 156 | 12@39 | 66@0 | 314 | 51@0 | 106@0 |
|  |  |  | DFT-s | 152 | 12@39 | 64@0 | 300 | 50@1 | 100@0 |
|  |  | 30+30 | CP | 156 | 39@39 | 39@0 | 312 | 78@0 | 78@0 |
|  |  |  | DFT-s | 152 | 40@38 | 36@0 | 300 | 75@3 | 75@0 |
|  |  | 40+20 | CP | 156 | 67@39 | 11@0 | 314 | 106@0 | 51@0 |
|  |  |  | DFT-s | 152 | 64@42 | 12@0 | 300 | 100@6 | 50@0 |
|  |  | 50+10 | CP | 156 | 78@39 | 0@0 | 314 | 133@0 | 24@0 |
|  |  |  | DFT-s | 150 | 75@44 | 0@0 | 304 | 128@5 | 24@0 |
|  | 15 | 20+50 | CP | 188 | 12@94 | 176@0 | 376 | 106@0 | 270@0 |
|  |  |  | DFT-s | 181 | 1@105 | 180@0 | 370 | 100@6 | 270@0 |
|  |  | 30+40 | CP | 188 | 66@94 | 122@0 | 376 | 160@0 | 216@0 |
|  |  |  | DFT-s | 184 | 64@96 | 120@0 | 376 | 160@0 | 216@0 |
|  |  | 40+30 | CP | 188 | 122@94 | 66@0 | 376 | 216@0 | 160@0 |
|  |  |  | DFT-s | 184 | 120@96 | 64@0 | 376 | 216@0 | 160@0 |
|  |  | 50+20 | CP | 188 | 176@94 | 12@0 | 376 | 270@0 | 106@0 |
|  |  |  | DFT-s | 181 | 180@90 | 1@0 | 370 | 270@0 | 100@0 |
|  | 30 | 20+50 | CP | 184 | 5@46 | 87@0 | 368 | 51@0 | 133@0 |
|  |  |  | DFT-s | 174 | 6@45 | 81@0 | 356 | 50@1 | 128@0 |
|  |  | 30+40 | CP | 184 | 32@46 | 60@0 | 368 | 78@0 | 106@0 |
| 70 |  |  | DFT-s | 184 | 32@46 | 60@0 | 350 | 75@3 | 100@0 |
|  |  | 40+30 | CP | 184 | 60@46 | 32@0 | 368 | 106@0 | 78@0 |
|  |  |  | DFT-s | 184 | 60@46 | 32@0 | 350 | 100@6 | 75@0 |
|  |  | 50+20 | CP | 184 | 87@46 | 5@0 | 368 | 133@0 | 51@0 |
|  |  |  | DFT-s | 174 | 81@52 | 6@0 | 356 | 128@5 | 50@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | 30+50 | CP | 215 | 53@107 | 162@0 | 430 | 160@0 | 270@0 |
|  |  |  | DFT-s | 210 | 50@110 | 160@0 | 430 | 160@0 | 270@0 |
|  |  | 40+40 | CP | 216 | 108@108 | 108@0 | 432 | 216@0 | 216@0 |
|  |  |  | DFT-s | 216 | 108@108 | 108@0 | 432 | 216@0 | 216@0 |
|  |  | 50+30 | CP | 215 | 163@107 | 52@0 | 430 | 270@0 | 160@0 |
|  |  |  | DFT-s | 210 | 162@108 | 48@0 | 430 | 270@0 | 160@0 |
|  | 30 | 30+50 | CP | 210 | 25@53 | 80@0 | 422 | 78@0 | 133@0 |
| 80 |  |  | DFT-s | 210 | 25@53 | 80@0 | 406 | 75@3 | 128@0 |
|  |  | 40+40 | CP | 212 | 53@53 | 53@0 | 424 | 106@0 | 106@0 |
|  |  |  | DFT-s | 208 | 54@52 | 50@0 | 400 | 100@6 | 100@0 |
|  |  | 50+30 | CP | 210 | 80@53 | 25@0 | 422 | 133@0 | 78@0 |
|  |  |  | DFT-s | 210 | 80@53 | 25@0 | 406 | 128@5 | 75@0 |
|  | 15 | 40+50 | CP | 243 | 95@121 | 148@0 | 486 | 216@0 | 270@0 |
| 90 |  |  | DFT-s | 240 | 96@120 | 144@0 | 486 | 216@0 | 270@0 |
|  |  | 50+40 | CP | 243 | 149@121 | 94@0 | 486 | 270@0 | 216@0 |
|  |  |  | DFT-s | 240 | 150@120 | 90@0 | 486 | 270@0 | 216@0 |
|  | 30 | 40+50 | CP | 238 | 46@60 | 73@0 | 478 | 106@0 | 133@0 |
|  |  |  | DFT-s | 234 | 45@61 | 72@0 | 456 | 100@6 | 128@0 |
|  |  | 50+40 | CP | 238 | 73@60 | 46@0 | 478 | 133@0 | 106@0 |
|  |  |  | DFT-s | 234 | 72@61 | 45@0 | 456 | 128@5 | 100@0 |
|  | 15 | 50+50 | CP | 270 | 135@135 | 135@0 | 540 | 270@0 | 270@0 |
|  |  |  | DFT-s | 270 | 135@135 | 135@0 | 540 | 270@0 | 270@0 |
| 100 | 30 | 50+50 | CP | 2644 | 67@66 | 65@0 | 532 | 133@0 | 133@0 |
|  |  |  | DFT-s | 256 | 64@69 | 64@0 | 512 | 128@5 | 128@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 15+90 | CP | 282 | 0@0 | 141@33 | 566 | 38@0 | 245@0 |
|  |  |  | DFT-s | 270 | 0@0 | 135@30 | 558 | 36@2 | 243@0 |
| 105 |  | 25+80 | CP | 2804 | 0@0 | 140@5 | 564 | 65@0 | 217@0 |
|  |  |  | DFT-s | 270 | 0@0 | 135@3 | 560 | 64@1 | 216@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 10+100 | CP | 296 | 0@0 | 148@50 | 594 | 24@0 | 273@0 |
|  |  |  | DFT-s | 288 | 0@0 | 144@48 | 588 | 24@0 | 270@0 |
|  |  | 20+90 | CP | 296 | 0@0 | 148@23 | 592 | 51@0 | 245@0 |
|  |  |  | DFT-s | 288 | 0@0 | 144@21 | 586 | 50@1 | 243@0 |
|  |  | 30+80 | CP | 294 | 4@74 | 143@0 | 590 | 78@0 | 217@0 |
|  |  |  | DFT-s | 294 | 3@74 | 144@0 | 582 | 75@3 | 216@0 |
|  |  | 40+70 | CP | 294 | 32@74 | 115@0 | 590 | 106@0 | 189@0 |
|  |  |  | DFT-s | 280 | 32@74 | 108@0 | 560 | 100@6 | 180@0 |
|  |  | 50+60 | CP | 294 | 59@74 | 88@0 | 590 | 133@0 | 162@0 |
|  |  |  | DFT-s | 282 | 60@73 | 81@0 | 580 | 128@5 | 162@0 |
|  |  | 60+50 | CP | 294 | 88@74 | 59@0 | 590 | 162@0 | 133@0 |
|  |  |  | DFT-s | 282 | 81@81 | 60@0 | 580 | 162@0 | 128@0 |
|  |  | 70+40 | CP | 294 | 115@74 | 32@0 | 590 | 189@0 | 106@0 |
|  |  |  | DFT-s | 294 | 108@74 | 32@0 | 560 | 180@9 | 100@0 |
|  |  | 80+30 | CP | 294 | 143@74 | 4@0 | 590 | 217@0 | 78@0 |
|  |  |  | DFT-s | 292 | 144@73 | 2@0 | 582 | 216@1 | 75@0 |
| 110 |  | 90+20 | CP | 296 | 148@74 | 0@0 | 592 | 245@0 | 51@0 |
|  |  |  | DFT-s | 288 | 144@80 | 0@0 | 586 | 243@2 | 50@0 |
|  |  | 100+10 | CP | 296 | 148@74 | 0@0 | 594 | 273@0 | 24@0 |
|  |  |  | DFT-s | 288 | 144@81 | 0@0 | 588 | 270@3 | 24@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 15+100 | CP | 310 | 0@0 | 155@40 | 622 | 38@0 | 273@0 |
|  |  |  | DFT-s | 300 | 0@0 | 150@37 | 612 | 36@2 | 270@0 |
|  |  | 25+90 | CP | 3084 | 0@0 | 154@12 | 620 | 65@0 | 245@0 |
|  |  |  | DFT-s | 300 | 0@0 | 150@10 | 614 | 64@1 | 243@0 |
|  |  | 90+25 | CP | 3084 | 154@77 | 0@0 | 620 | 245@0 | 65@0 |
| 115 |  |  | DFT-s | 300 | 150@85 | 0@0 | 614 | 243@2 | 64@0 |
|  |  | 100+15 | CP | 310 | 155@78 | 0@0 | 622 | 273@0 | 38@0 |
|  |  |  | DFT-s | 300 | 150@86 | 0@0 | 612 | 270@3 | 36@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 20+100 | CP | 324 | 0@0 | 162@30 | 648 | 51@0 | 273@0 |
|  |  |  | DFT-s | 324 | 0@0 | 162@30 | 640 | 50@1 | 270@0 |
|  |  | 30+90 | CP | 322 | 0@0 | 161@3 | 646 | 78@0 | 245@0 |
|  |  |  | DFT-s | 320 | 0@0 | 160@2 | 636 | 75@3 | 243@0 |
|  |  | 40+80 | CP | 322 | 25@81 | 136@0 | 646 | 106@0 | 217@0 |
|  |  |  | DFT-s | 320 | 25@81 | 135@0 | 632 | 100@6 | 216@0 |
|  |  | 50+70 | CP | 3204 | 53@80 | 107@0 | 644 | 133@0 | 189@0 |
|  |  |  | DFT-s | 320 | 50@81 | 108@0 | 616 | 128@5 | 180@0 |
|  |  | 60+60 | CP | 324 | 81@81 | 81@0 | 648 | 162@0 | 162@0 |
|  |  |  | DFT-s | 324 | 81@81 | 81@0 | 648 | 162@0 | 162@0 |
|  |  | 70+50 | CP | 3204 | 109@80 | 51@0 | 644 | 189@0 | 133@0 |
|  |  |  | DFT-s | 316 | 108@81 | 50@0 | 616 | 180@9 | 128@0 |
|  |  | 80+40 | CP | 322 | 136@81 | 25@0 | 646 | 217@0 | 106@0 |
|  |  |  | DFT-s | 318 | 135@82 | 24@0 | 632 | 216@1 | 100@0 |
|  |  | 90+30 | CP | 322 | 161@81 | 0@0 | 646 | 245@0 | 78@0 |
|  |  |  | DFT-s | 320 | 160@83 | 0@0 | 636 | 243@2 | 75@0 |
| 120 |  | 100+20 | CP | 324 | 162@81 | 0@0 | 648 | 273@0 | 51@0 |
|  |  |  | DFT-s | 324 | 162@81 | 0@0 | 640 | 270@3 | 50@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 25+100 | CP | 3364 | 0@0 | 168@19 | 676 | 65@0 | 273@0 |
|  |  |  | DFT-s | 324 | 0@0 | 162@16 | 668 | 64@1 | 270@0 |
|  |  | 100+25 | CP | 3364 | 168@84 | 0@0 | 676 | 273@0 | 65@0 |
| 125 |  |  | DFT-s | 324 | 162@95 | 0@0 | 668 | 270@3 | 64@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 30+100 | CP | 350 | 0@0 | 175@10 | 702 | 78@0 | 273@0 |
|  |  |  | DFT-s | 324 | 0@0 | 162@3 | 690 | 75@3 | 270@0 |
|  |  | 40+90 | CP | 350 | 18@88 | 157@0 | 702 | 106@0 | 245@0 |
|  |  |  | DFT-s | 340 | 20@86 | 150@0 | 686 | 100@6 | 243@0 |
|  |  | 50+80 | CP | 3484 | 46@87 | 128@0 | 700 | 133@0 | 217@0 |
|  |  |  | DFT-s | 346 | 45@88 | 128@0 | 688 | 128@5 | 216@0 |
|  |  | 60+70 | CP | 350 | 74@88 | 101@0 | 702 | 162@0 | 189@0 |
|  |  |  | DFT-s | 348 | 72@88 | 100@0 | 684 | 162@0 | 180@0 |
|  |  | 70+60 | CP | 350 | 101@88 | 74@0 | 702 | 189@0 | 162@0 |
|  |  |  | DFT-s | 346 | 100@88 | 72@0 | 684 | 180@9 | 162@0 |
|  |  | 80+50 | CP | 3484 | 130@87 | 44@0 | 700 | 217@0 | 133@0 |
|  |  |  | DFT-s | 346 | 128@89 | 45@0 | 688 | 216@1 | 128@0 |
| 130 |  | 90+40 | CP | 350 | 157@88 | 18@0 | 702 | 245@0 | 106@0 |
|  |  |  | DFT-s | 340 | 160@85 | 10@0 | 686 | 243@2 | 100@0 |
|  |  | 100+30 | CP | 350 | 175@88 | 0@0 | 702 | 273@0 | 78@0 |
|  |  |  | DFT-s | 324 | 162@108 | 0@0 | 690 | 270@3 | 75@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 40+100 | CP | 378 | 11@95 | 178@0 | 758 | 106@0 | 273@0 |
|  |  |  | DFT-s | 372 | 6@100 | 180@0 | 740 | 100@6 | 270@0 |
|  |  | 50+90 | CP | 3764 | 39@94 | 149@0 | 756 | 133@0 | 245@0 |
|  |  |  | DFT-s | 372 | 36@97 | 150@0 | 742 | 128@5 | 243@0 |
|  |  | 60+80 | CP | 378 | 67@95 | 122@0 | 758 | 162@0 | 217@0 |
|  |  |  | DFT-s | 368 | 64@98 | 120@0 | 756 | 162@0 | 216@0 |
|  |  | 70+70 | CP | 3764 | 95@94 | 93@0 | 756 | 189@0 | 189@0 |
|  |  |  | DFT-s | 368 | 90@95 | 90@0 | 720 | 180@9 | 180@0 |
|  |  | 80+60 | CP | 378 | 122@95 | 67@0 | 758 | 217@0 | 162@0 |
|  |  |  | DFT-s | 368 | 120@97 | 64@0 | 756 | 216@1 | 162@0 |
|  |  | 90+50 | CP | 3764 | 151@94 | 37@0 | 756 | 245@0 | 133@0 |
| 140 |  |  | DFT-s | 372 | 150@95 | 36@0 | 742 | 243@2 | 128@0 |
|  |  | 100+40 | CP | 378 | 178@95 | 11@0 | 758 | 273@0 | 106@0 |
|  |  |  | DFT-s | 372 | 180@93 | 6@0 | 740 | 270@3 | 100@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 50+100 | CP | 4044 | 32@101 | 170@0 | 812 | 133@0 | 273@0 |
|  |  |  | DFT-s | 388 | 32@101 | 162@0 | 796 | 128@5 | 270@0 |
|  |  | 60+90 | CP | 406 | 60@102 | 143@0 | 814 | 162@0 | 245@0 |
|  |  |  | DFT-s | 396 | 54@108 | 144@0 | 810 | 162@0 | 243@0 |
|  |  | 70+80 | CP | 4044 | 88@101 | 114@0 | 812 | 189@0 | 217@0 |
|  |  |  | DFT-s | 374 | 81@102 | 100@0 | 792 | 180@9 | 216@0 |
|  |  | 80+70 | CP | 4044 | 116@101 | 86@0 | 812 | 217@0 | 189@0 |
|  |  |  | DFT-s | 392 | 100@102 | 81@0 | 792 | 216@1 | 180@0 |
|  |  | 90+60 | CP | 406 | 143@102 | 60@0 | 814 | 245@0 | 162@0 |
|  |  |  | DFT-s | 396 | 144@101 | 54@0 | 810 | 243@2 | 162@0 |
|  |  | 100+50 | CP | 4044 | 172@101 | 30@0 | 812 | 273@0 | 133@0 |
| 150 |  |  | DFT-s | 388 | 162@111 | 32@0 | 796 | 270@3 | 128@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 60+100 | CP | 434 | 53@109 | 164@0 | 870 | 162@0 | 273@0 |
|  |  |  | DFT-s | 432 | 54@108 | 162@0 | 864 | 162@0 | 270@0 |
|  |  | 70+90 | CP | 4324 | 81@108 | 135@0 | 868 | 189@0 | 245@0 |
|  |  |  | DFT-s | 430 | 80@109 | 135@0 | 846 | 180@9 | 243@0 |
|  |  | 80+80 | CP | 4324 | 109@108 | 107@0 | 864 | 216@1 | 216@0 |
|  |  |  | DFT-s | 432 | 108@109 | 108@0 | 864 | 216@1 | 216@0 |
|  |  | 90+70 | CP | 4324 | 137@108 | 79@0 | 868 | 245@0 | 189@0 |
|  |  |  | DFT-s | 432 | 135@109 | 80@0 | 846 | 243@2 | 180@0 |
| 160 |  | 100+60 | CP | 434 | 164@109 | 53@0 | 870 | 273@0 | 162@0 |
|  |  |  | DFT-s | 432 | 162@111 | 54@0 | 864 | 270@3 | 162@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 70+100 | CP | 4604 | 74@115 | 156@0 | 924 | 189@0 | 273@0 |
|  |  |  | DFT-s | 446 | 72@116 | 150@0 | 900 | 180@9 | 270@0 |
|  |  | 80+90 | CP | 4604 | 102@115 | 128@0 | 924 | 217@0 | 245@0 |
|  |  |  | DFT-s | 456 | 100@117 | 128@0 | 918 | 216@1 | 243@0 |
|  |  | 90+80 | CP | 4604 | 130@115 | 100@0 | 924 | 245@0 | 217@0 |
|  |  |  | DFT-s | 456 | 128@117 | 100@0 | 918 | 243@2 | 216@0 |
|  |  | 100+70 | CP | 4604 | 158@115 | 72@0 | 924 | 273@0 | 189@0 |
| 170 |  |  | DFT-s | 458 | 150@116 | 72@0 | 900 | 270@3 | 180@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 80+100 | CP | 4884 | 95@122 | 149@0 | 980 | 217@0 | 273@0 |
|  |  |  | DFT-s | 480 | 96@121 | 144@0 | 972 | 216@1 | 270@0 |
|  |  | 90+90 | CP | 4884 | 123@122 | 121@0 | 980 | 245@0 | 245@0 |
|  |  |  | DFT-s | 480 | 120@125 | 120@0 | 972 | 243@2 | 243@0 |
|  |  | 100+80 | CP | 4884 | 151@122 | 93@0 | 980 | 273@0 | 217@0 |
| 180 |  |  | DFT-s | 480 | 150@123 | 90@0 | 972 | 270@3 | 216@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 90+100 | CP | 516 | 116@129 | 142@0 | 1036 | 245@0 | 273@0 |
|  |  |  | DFT-s | 504 | 108@137 | 144@0 | 1026 | 243@2 | 270@0 |
|  |  | 100+90 | CP | 5164 | 144@129 | 114@0 | 1036 | 273@0 | 245@0 |
| 190 |  |  | DFT-s | 504 | 144@129 | 108@0 | 1026 | 270@3 | 243@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 100+100 | CP | 544 | 137@136 | 135@0 | 1092 | 273@0 | 273@0 |
| 200 |  |  | DFT-s | 540 | 135@138 | 135@0 | 1080 | 270@3 | 270@0 |
| NOTE 1: BWchannel_CA is defined in clause 5.3A.3 NOTE 2: NRB_alloc for intra-band contiguous UL CA is defined in 6.2A.2.0 NOTE 3: Void NOTE 4: The allocated RB number NRB_alloc is ceil(NRB_agg/2) -1∙2^μ in order to meet Inner RB allocation definition (RBStart,Low ≤ RBStart ≤ RBStart,High) described in subclause 6.2A.2.0. |  |  |  |  |  |  |  |  |  |

Table 6.1A-1b: Common uplink configuration for intra-band contiguous 2UL CA (non-contiguous RB allocation)

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | RB allocation (Inner) |  |  | RB allocation (Outer1) |  |  | RB allocation (Outer2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
| 20 | 15 | 5+15 | 30 | 1@23 | 1@29 | 66 | 1@17 | 1@57 | 104 | 1@0 | 1@78 |
|  |  | 10+10 | 34 | 1@35 | 1@16 | 63 | 1@20 | 1@30 | 104 | 1@0 | 1@51 |
|  |  | 15+5 | 37 | 1@52 | 1@9 | 66 | 1@37 | 1@23 | 104 | 1@0 | 1@24 |
| 25 | 15 | 5+20 | 34 | 1@23 | 1@33 | 85 | 1@21 | 1@80 | 131 | 1@0 | 1@105 |
|  |  | 10+15 | 43 | 1@44 | 1@34 | 80 | 1@26 | 1@53 | 131 | 1@0 | 1@78 |
|  |  | 15+10 | 43 | 1@43 | 1@6 | 80 | 1@25 | 1@25 | 131 | 1@0 | 1@51 |
|  |  | 20+5 | 48 | 1@67 | 1@8 | 85 | 1@49 | 1@24 | 131 | 1@0 | 1@24 |
|  | 30 | 10+15 | 40 | 1@21 | 1@16 | 76 | 1@12 | 1@25 | 124 | 1@0 | 1@37 |
|  |  | 15+10 | 40 | 1@21 | 1@2 | 76 | 1@12 | 1@11 | 124 | 1@0 | 1@23 |
| 30 | 15 | 10+20 | 54 | 1@51 | 1@52 | 98 | 1@28 | 1@73 | 158 | 1@0 | 1@105 |
|  |  | 15+15 | 51 | 1@53 | 1@24 | 95 | 1@30 | 1@45 | 158 | 1@0 | 1@78 |
|  |  | 20+10 | 54 | 1@53 | 1@0 | 99 | 1@31 | 1@23 | 158 | 1@0 | 1@51 |
|  | 30 | 10+20 | 48 | 1@23 | 1@23 | 94 | 1@13 | 1@35 | 150 | 1@0 | 1@50 |
|  |  | 15+15 | 46 | 1@25 | 1@9 | 92 | 1@15 | 1@22 | 152 | 1@0 | 1@37 |
|  |  | 20+10 | 46 | 1@29 | 1@0 | 94 | 1@14 | 1@9 | 150 | 1@0 | 1@23 |
| 35 | 15 | 15+20 | 62 | 1@60 | 1@42 | 114 | 1@35 | 1@69 | 185 | 1@0 | 1@105 |
|  |  | 20+15 | 62 | 1@61 | 1@16 | 113 | 1@35 | 1@41 | 185 | 1@0 | 1@78 |
|  | 30 | 15+20 | 58 | 1@29 | 1@19 | 110 | 1@17 | 1@33 | 178 | 1@0 | 1@50 |
|  |  | 20+15 | 58 | 1@30 | 1@7 | 110 | 1@17 | 1@20 | 178 | 1@0 | 1@37 |
| 40 | 15 | 10+30 | 53 | 1@51 | 1@52 | 130 | 1@40 | 1@117 | 212 | 1@0 | 1@159 |
|  |  | 20+20 | 70 | 1@71 | 1@34 | 129 | 1@42 | 1@64 | 212 | 1@0 | 1@105 |
|  |  | 30+10 | 52 | 1@110 | 1@1 | 130 | 1@40 | 1@9 | 212 | 1@0 | 1@51 |
|  | 30 | 10+30 | 66 | 1@23 | 1@42 | 126 | 1@19 | 1@57 | 204 | 1@0 | 1@77 |
|  |  | 20+20 | 64 | 1@34 | 1@14 | 124 | 1@19 | 1@29 | 204 | 1@0 | 1@50 |
|  |  | 30+10 | 46 | 1@56 | 1@0 | 126 | 1@19 | 1@3 | 204 | 1@0 | 1@23 |
| 45 | 15 | 15+30 | 79 | 1@78 | 1@78 | 146 | 1@46 | 1@112 | 239 | 1@0 | 1@159 |
|  | 30 | 15+30 | 74 | 1@37 | 1@36 | 142 | 1@22 | 1@54 | 232 | 1@0 | 1@77 |
| 50 | 15 | 10+40 | 57 | 1@51 | 1@56 | 165 | 1@50 | 1@162 | 268 | 1@0 | 1@215 |
|  |  | 20+30 | 87 | 1@89 | 1@69 | 161 | 1@53 | 1@107 | 266 | 1@0 | 1@159 |
|  |  | 30+20 | 87 | 1@88 | 1@14 | 161 | 1@52 | 1@52 | 266 | 1@0 | 1@105 |
|  |  | 40+10 | 55 | 1@162 | 1@0 | 165 | 1@52 | 1@0 | 268 | 1@0 | 1@51 |
|  | 30 | 10+40 | 50 | 1@23 | 1@24 | 160 | 1@23 | 1@78 | 260 | 1@0 | 1@105 |
|  |  | 20+30 | 82 | 1@43 | 1@32 | 156 | 1@25 | 1@51 | 258 | 1@0 | 1@77 |
|  |  | 30+20 | 82 | 1@43 | 1@5 | 156 | 1@24 | 1@23 | 258 | 1@0 | 1@50 |
|  |  | 40+10 | 48 | 1@83 | 1@0 | 160 | 1@27 | 1@0 | 260 | 1@0 | 1@23 |
| 55 | 15 | 15+40 | 84 | 1@78 | 1@83 | 182 | 1@55 | 1@157 | 295 | 1@0 | 1@215 |
|  | 30 | 15+40 | 78 | 1@37 | 1@38 | 178 | 1@27 | 1@77 | 288 | 1@0 | 1@105 |
| 60 | 15 | 10+50 | 108 | 1@51 | 1@54 | 197 | 1@51 | 1@205 | 322 | 1@0 | 1@269 |
|  |  | 20+40 | 107 | 1@105 | 1@106 | 195 | 1@61 | 1@149 | 322 | 1@0 | 1@215 |
|  |  | 30+30 | 104 | 1@107 | 1@50 | 193 | 1@63 | 1@95 | 320 | 1@0 | 1@159 |
|  |  | 40+20 | 105 | 1@112 | 1@0 | 195 | 1@62 | 1@40 | 322 | 1@0 | 1@105 |
|  |  | 50+10 | 52 | 1@220 | 1@0 | 192 | 1@79 | 1@0 | 322 | 1@0 | 1@51 |
|  | 30 | 10+50 | 48 | 1@23 | 1@23 | 192 | 1@23 | 1@100 | 314 | 1@0 | 1@132 |
|  |  | 20+40 | 100 | 1@50 | 1@49 | 190 | 1@29 | 1@72 | 314 | 1@0 | 1@105 |
|  |  | 30+30 | 98 | 1@52 | 1@22 | 188 | 1@30 | 1@45 | 312 | 1@0 | 1@77 |
|  |  | 40+20 | 96 | 1@59 | 1@0 | 190 | 1@29 | 1@17 | 314 | 1@0 | 1@50 |
|  |  | 50+10 | 44 | 1@112 | 1@0 | 186 | 1@41 | 1@0 | 314 | 1@0 | 1@23 |
| 70 | 15 | 20+50 | 107 | 1@105 | 1@106 | 229 | 1@73 | 1@195 | 376 | 1@0 | 1@269 |
|  |  | 30+40 | 126 | 1@121 | 1@89 | 229 | 1@72 | 1@140 | 376 | 1@0 | 1@215 |
|  |  | 40+30 | 126 | 1@126 | 1@35 | 229 | 1@74 | 1@86 | 376 | 1@0 | 1@159 |
|  |  | 50+20 | 106 | 1@165 | 1@0 | 229 | 1@73 | 1@31 | 376 | 1@0 | 1@105 |
|  | 30 | 20+50 | 100 | 1@50 | 1@49 | 224 | 1@35 | 1@95 | 368 | 1@0 | 1@132 |
|  |  | 30+40 | 122 | 1@61 | 1@43 | 224 | 1@34 | 1@67 | 368 | 1@0 | 1@105 |
|  |  | 40+30 | 122 | 1@62 | 1@16 | 224 | 1@35 | 1@40 | 368 | 1@0 | 1@77 |
|  |  | 50+20 | 98 | 1@85 | 1@0 | 224 | 1@35 | 1@13 | 368 | 1@0 | 1@50 |
| 80 | 15 | 30+50 | 143 | 1@143 | 1@125 | 261 | 1@83 | 1@183 | 430 | 1@0 | 1@269 |
|  |  | 40+40 | 142 | 1@144 | 1@69 | 260 | 1@84 | 1@127 | 432 | 1@0 | 1@215 |
|  |  | 50+30 | 143 | 1@144 | 1@16 | 261 | 1@84 | 1@74 | 430 | 1@0 | 1@159 |
|  | 30 | 30+50 | 138 | 1@70 | 1@60 | 256 | 1@40 | 1@89 | 422 | 1@0 | 1@132 |
|  |  | 40+40 | 136 | 1@70 | 1@31 | 256 | 1@41 | 1@62 | 424 | 1@0 | 1@105 |
|  |  | 50+30 | 138 | 1@71 | 1@6 | 256 | 1@40 | 1@34 | 422 | 1@0 | 1@77 |
| 90 | 15 | 40+50 | 161 | 1@163 | 1@107 | 294 | 1@96 | 1@173 | 486 | 1@0 | 1@269 |
|  |  | 50+40 | 161 | 1@162 | 1@52 | 294 | 1@95 | 1@118 | 486 | 1@0 | 1@215 |
|  | 30 | 40+50 | 156 | 1@80 | 1@51 | 290 | 1@47 | 1@85 | 478 | 1@0 | 1@132 |
|  |  | 50+40 | 156 | 1@80 | 1@24 | 290 | 1@47 | 1@58 | 478 | 1@0 | 1@105 |
| 100 | 15 | 50+50 | 178 | 1@180 | 1@87 | 326 | 1@106 | 1@161 | 540 | 1@0 | 1@269 |
|  | 30 | 20+80 | 98 | 1@50 | 1@48 | 324 | 1@50 | 1@163 | 536 | 1@0 | 1@216 |
|  |  | 50+50 | 172 | 1@88 | 1@40 | 322 | 1@52 | 1@79 | 532 | 1@0 | 1@132 |
|  |  | 80+20 | 94 | 1@171 | 1@0 | 320 | 1@58 | 1@0 | 536 | 1@0 | 1@50 |
| 105 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 15+90 | 76 | 1@37 | 1@37 | 344 | 1@37 | 1@188 | 566 | 1@0 | 1@244 |
|  |  | 25+80 | 126 | 1@64 | 1@61 | 340 | 1@54 | 1@158 | 564 | 1@0 | 1@216 |
| 110 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 10+100 | 48 | 1@23 | 1@23 | 360 | 1@23 | 1@213 | 594 | 1@0 | 1@272 |
|  |  | 20+90 | 96 | 1@50 | 1@46 | 356 | 1@50 | 1@183 | 592 | 1@0 | 1@244 |
|  |  | 30+80 | 154 | 1@77 | 1@76 | 358 | 1@58 | 1@158 | 590 | 1@0 | 1@216 |
|  |  | 40+70 | 194 | 1@99 | 1@89 | 358 | 1@59 | 1@131 | 590 | 1@0 | 1@188 |
|  |  | 50+60 | 192 | 1@98 | 1@60 | 356 | 1@58 | 1@102 | 590 | 1@0 | 1@161 |
|  |  | 60+50 | 192 | 1@99 | 1@32 | 356 | 1@59 | 1@74 | 590 | 1@0 | 1@132 |
|  |  | 70+40 | 194 | 1@98 | 1@5 | 358 | 1@58 | 1@47 | 590 | 1@0 | 1@105 |
|  |  | 80+30 | 152 | 1@142 | 1@0 | 358 | 1@59 | 1@20 | 590 | 1@0 | 1@77 |
|  |  | 90+20 | 95 | 1@199 | 1@0 | 348 | 1@72 | 1@0 | 592 | 1@0 | 1@50 |
|  |  | 100+10 | 52 | 1@248 | 1@0 | 324 | 1@112 | 1@0 | 594 | 1@0 | 1@23 |
| 115 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 15+100 | 76 | 1@37 | 1@37 | 376 | 1@37 | 1@209 | 622 | 1@0 | 1@272 |
|  |  | 25+90 | 127 | 1@64 | 1@62 | 374 | 1@61 | 1@182 | 620 | 1@0 | 1@244 |
|  |  | 90+25 | 122 | 1@185 | 1@0 | 374 | 1@61 | 1@2 | 620 | 1@0 | 1@64 |
|  |  | 100+15 | 74 | 1@237 | 1@0 | 354 | 1@97 | 1@0 | 622 | 1@0 | 1@37 |
| 120 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 20+100 | 98 | 1@50 | 1@47 | 392 | 1@50 | 1@208 | 648 | 1@0 | 1@272 |
|  |  | 30+90 | 154 | 1@77 | 1@76 | 390 | 1@63 | 1@179 | 646 | 1@0 | 1@244 |
|  |  | 40+80 | 208 | 1@105 | 1@103 | 390 | 1@63 | 1@151 | 646 | 1@0 | 1@216 |
|  |  | 50+70 | 210 | 1@107 | 1@78 | 390 | 1@64 | 1@125 | 644 | 1@0 | 1@188 |
|  |  | 60+60 | 210 | 1@107 | 1@49 | 390 | 1@64 | 1@96 | 648 | 1@0 | 1@161 |
|  |  | 70+50 | 210 | 1@107 | 1@22 | 390 | 1@64 | 1@69 | 644 | 1@0 | 1@132 |
|  |  | 80+40 | 206 | 1@115 | 1@0 | 390 | 1@63 | 1@40 | 646 | 1@0 | 1@105 |
|  |  | 90+30 | 152 | 1@170 | 1@0 | 390 | 1@63 | 1@12 | 646 | 1@0 | 1@77 |
|  |  | 100+20 | 94 | 1@227 | 1@0 | 378 | 1@85 | 1@0 | 648 | 1@0 | 1@50 |
|  | 15 | N/A |  |  |  |  |  |  |  |  |  |
| 125 | 30 | 25+100 | 126 | 1@64 | 1@62 | 406 | 1@64 | 1@203 | 676 | 1@0 | 1@272 |
|  |  | 100+25 | 122 | 1@213 | 1@0 | 404 | 1@72 | 1@0 | 676 | 1@0 | 1@64 |
| 130 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 30+100 | 152 | 1@77 | 1@75 | 424 | 1@70 | 1@203 | 702 | 1@0 | 1@272 |
|  |  | 40+90 | 208 | 1@105 | 1@103 | 424 | 1@70 | 1@175 | 702 | 1@0 | 1@244 |
|  |  | 50+80 | 228 | 1@116 | 1@96 | 422 | 1@68 | 1@145 | 700 | 1@0 | 1@216 |
|  |  | 60+70 | 230 | 1@117 | 1@69 | 424 | 1@70 | 1@119 | 702 | 1@0 | 1@188 |
|  |  | 70+60 | 230 | 1@117 | 1@42 | 424 | 1@70 | 1@92 | 702 | 1@0 | 1@161 |
|  |  | 80+50 | 228 | 1@116 | 1@12 | 422 | 1@68 | 1@61 | 700 | 1@0 | 1@132 |
|  |  | 90+40 | 206 | 1@143 | 1@0 | 424 | 1@69 | 1@35 | 702 | 1@0 | 1@105 |
|  |  | 100+30 | 150 | 1@199 | 1@0 | 424 | 1@70 | 1@8 | 702 | 1@0 | 1@77 |
| 140 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 40+100 | 210 | 1@105 | 1@104 | 458 | 1@75 | 1@197 | 758 | 1@0 | 1@272 |
|  |  | 50+90 | 248 | 1@125 | 1@115 | 456 | 1@73 | 1@167 | 756 | 1@0 | 1@244 |
|  |  | 60+80 | 250 | 1@127 | 1@89 | 456 | 1@75 | 1@140 | 758 | 1@0 | 1@216 |
|  |  | 70+70 | 246 | 1@125 | 1@58 | 456 | 1@75 | 1@113 | 756 | 1@0 | 1@188 |
|  |  | 80+60 | 248 | 1@126 | 1@32 | 456 | 1@74 | 1@84 | 758 | 1@0 | 1@161 |
|  |  | 90+50 | 248 | 1@125 | 1@3 | 456 | 1@73 | 1@55 | 756 | 1@0 | 1@132 |
|  |  | 100+40 | 208 | 1@170 | 1@0 | 458 | 1@74 | 1@29 | 758 | 1@0 | 1@105 |
| 150 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 50+100 | 262 | 1@132 | 1@130 | 490 | 1@80 | 1@191 | 812 | 1@0 | 1@272 |
|  |  | 60+90 | 268 | 1@136 | 1@107 | 490 | 1@80 | 1@162 | 814 | 1@0 | 1@244 |
|  |  | 70+80 | 266 | 1@135 | 1@78 | 490 | 1@80 | 1@135 | 812 | 1@0 | 1@216 |
|  |  | 80+70 | 266 | 1@135 | 1@50 | 490 | 1@80 | 1@107 | 812 | 1@0 | 1@188 |
|  |  | 90+60 | 268 | 1@135 | 1@23 | 490 | 1@79 | 1@78 | 814 | 1@0 | 1@161 |
|  |  | 100+50 | 262 | 1@143 | 1@0 | 490 | 1@80 | 1@51 | 812 | 1@0 | 1@132 |
| 160 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 60+100 | 288 | 1@145 | 1@125 | 524 | 1@86 | 1@185 | 870 | 1@0 | 1@272 |
|  |  | 70+90 | 286 | 1@145 | 1@98 | 522 | 1@85 | 1@156 | 868 | 1@0 | 1@244 |
|  |  | 80+80 | 284 | 1@144 | 1@68 | 522 | 1@85 | 1@128 | 868 | 1@0 | 1@216 |
|  |  | 90+70 | 286 | 1@145 | 1@42 | 522 | 1@85 | 1@100 | 868 | 1@0 | 1@188 |
|  |  | 100+60 | 288 | 1@145 | 1@15 | 524 | 1@86 | 1@74 | 870 | 1@0 | 1@161 |
| 170 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 70+100 | 304 | 1@153 | 1@115 | 558 | 1@92 | 1@181 | 924 | 1@0 | 1@272 |
|  |  | 80+90 | 304 | 1@154 | 1@88 | 556 | 1@91 | 1@151 | 924 | 1@0 | 1@244 |
|  |  | 90+80 | 304 | 1@154 | 1@60 | 556 | 1@91 | 1@123 | 924 | 1@0 | 1@216 |
|  |  | 100+70 | 304 | 1@153 | 1@31 | 558 | 1@92 | 1@97 | 924 | 1@0 | 1@188 |
| 180 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 80+100 | 322 | 1@163 | 1@106 | 590 | 1@96 | 1@173 | 980 | 1@0 | 1@272 |
|  |  | 90+90 | 322 | 1@164 | 1@79 | 588 | 1@96 | 1@144 | 980 | 1@0 | 1@244 |
|  |  | 100+80 | 324 | 1@164 | 1@52 | 590 | 1@96 | 1@117 | 980 | 1@0 | 1@216 |
| 190 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 90+100 | 342 | 1@173 | 1@98 | 624 | 1@103 | 1@169 | 1036 | 1@0 | 1@272 |
|  |  | 100+90 | 342 | 1@173 | 1@70 | 624 | 1@103 | 1@141 | 1036 | 1@0 | 1@244 |
| 200 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 100+100 | 360 | 1@182 | 1@88 | 656 | 1@108 | 1@162 | 1092 | 1@0 | 1@272 |
| NOTE 1: BWchannel_CA is defined in clause 5.3A.3 NOTE 2: NRB_alloc for intra-band contiguous UL CA is defined in 6.2A.2.0 |  |  |  |  |  |  |  |  |  |  |  |

## 6.1E	General

Sidelink RB allocations given in Table 6.1E-1 are used for V2X test cases, unless otherwise stated by the test case.

Table 6.1E-1: Sidelink configuration for PSSCH/PSCCH

| BWchannel (MHz) | SCS (kHz) | Parameters | RB allocation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer_Full | Outer_10RB_Left | Outer_10RB_Right | Inner_Full |
| 10MHz | 15 | PSSCH allocation | 50@0 | 10@0 | 10@42 | 25@12 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@42 | 10@12 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 42 | 12 |
|  |  | sl-SubchannelSize-r16 | 10 | 10 | 10 | 25 |
|  |  | sl-NumSubchannel-r16 | 5 | 1 | 1 | 1 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
|  | 30 | PSSCH allocation | 24@0 | 10@0 | 10@14 | 12@6 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@14 | 10@6 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 14 | 6 |
|  |  | sl-SubchannelSize-r16 | 12 | 10 | 10 | 12 |
|  |  | sl-NumSubchannel-r16 | 2 | 1 | 1 | 1 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
|  | 60 | PSSCH allocation | 10@0 | 10@0 | 10@1 | N/A |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@1 | N/A |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 1 | N/A |
|  |  | sl-SubchannelSize-r16 | 10 | 10 | 10 | N/A |
|  |  | sl-NumSubchannel-r16 | 1 | 1 | 1 | N/A |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
| 20MHz | 15 | PSSCH allocation | 105@0 | 10@0 | 10@96 | 50@25 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@96 | 10@25 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 96 | 25 |
|  |  | sl-SubchannelSize-r16 | 15 | 10 | 10 | 10 |
|  |  | sl-NumSubchannel-r16 | 7 | 1 | 1 | 5 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
|  | 30 | PSSCH allocation | 50@0 | 10@0 | 10@41 | 25@12 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@41 | 10@12 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 41 | 12 |
|  |  | sl-SubchannelSize-r16 | 10 | 10 | 10 | 25 |
|  |  | sl-NumSubchannel-r16 | 5 | 1 | 1 | 1 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
|  | 60 | PSSCH allocation | 24@0 | 10@0 | 10@14 | 12@6 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@14 | 10@6 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 14 | 6 |
|  |  | sl-SubchannelSize-r16 | 12 | 10 | 10 | 12 |
|  |  | sl-NumSubchannel-r16 | 2 | 1 | 1 | 1 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |

| BWchannel (MHz) | SCS (kHz) | Parameters | RB allocation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer_Full | Outer_10RB_Left | Outer_10RB_Right | Inner_Full |
| 30MHz | 15 | PSSCH allocation | 160@0 | 10@0 | 10@150 | 80@40 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@150 | 10@40 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 150 | 40 |
|  |  | sl-SubchannelSize-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-NumSubchannel-r16 | 16 | 1 | 1 | 8 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
|  | 30 | PSSCH allocation | 75@0 | 10@0 | 10@68 | 36@18 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@68 | 10@18 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 68 | 18 |
|  |  | sl-SubchannelSize-r16 | 15 | 10 | 10 | 12 |
|  |  | sl-NumSubchannel-r16 | 5 | 1 | 1 | 3 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
|  | 60 | PSSCH allocation | 36@0 | 10@0 | 10@28 | 15@7 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@28 | 10@7 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 28 | 7 |
|  |  | sl-SubchannelSize-r16 | 12 | 10 | 10 | 15 |
|  |  | sl-NumSubchannel-r16 | 3 | 1 | 1 | 1 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
| 40MHz | 15 | PSSCH allocation | 216@0 | 10@0 | 10@206 | 105@52 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@206 | 10@52 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 206 | 52 |
|  |  | sl-SubchannelSize-r16 | 12 | 10 | 10 | 15 |
|  |  | sl-NumSubchannel-r16 | 18 | 1 | 1 | 7 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
|  | 30 | PSSCH allocation | 105@0 | 10@0 | 10@96 | 50@25 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@96 | 10@25 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 96 | 25 |
|  |  | sl-SubchannelSize-r16 | 15 | 10 | 10 | 10 |
|  |  | sl-NumSubchannel-r16 | 7 | 1 | 1 | 5 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
|  | 60 | PSSCH allocation | 50@0 | 10@0 | 10@41 | 25@12 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@41 | 10@12 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 41 | 12 |
|  |  | sl-SubchannelSize-r16 | 10 | 10 | 10 | 25 |
|  |  | sl-NumSubchannel-r16 | 5 | 1 | 1 | 1 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |

Table 6.1E-2: Sidelink configuration for PSFCH

| BWchannel (MHz) | SCS (kHz) | Parameters | RB allocation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PSFCH_1RB_Left | PSFCH_1RB_Right | PSFCH_2RB_Left | PSFCH_2RB_Right | PSFCH_Max_Gap |  |
| 10MHz | 15 | PSFCH allocation | 1@0 | 1@51 | 2@0 | 2@50 | 1@0 | 1@49 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 2 | 0 | 2 | 0 |  |
|  |  | sl-SubchannelSize-r16 | 25 | 25 | 25 | 25 | 25 |  |
|  |  | sl-NumSubchannel-r16 | 2 | 2 | 2 | 2 | 2 |  |
|  |  | sl-PSFCH-RB-Set-r16 | Bitstring of length 50, The leftmost 4 bits and rightmost 4 bits are set to "1" and the rest are set to "0" |  |  |  |  |  |
|  | 30 | PSFCH allocation | 1@0 | 1@23 | 2@0 | 2@22 | 1@0 | 1@23 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 0 | 0 | 0 |  |
|  |  | sl-SubchannelSize-r16 | 12 | 12 | 12 | 12 | 12 |  |
|  |  | sl-NumSubchannel-r16 | 2 | 2 | 2 | 2 | 2 |  |
|  |  | sl-PSFCH-RB-Set-r16 | Bitstring of length 24, The leftmost 4 bits and rightmost 4 bits are set to "1" and the rest are set to "0" |  |  |  |  |  |
|  | 60 | PSFCH allocation | 10@0 | 1@10 | 2@0 | 2@9 | 1@0 | 1@9 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 1 | 0 | 1 | 0 |  |
|  |  | sl-SubchannelSize-r16 | 10 | 10 | 10 | 10 | 10 |  |
|  |  | sl-NumSubchannel-r16 | 1 | 1 | 1 | 1 | 1 |  |
|  |  | sl-PSFCH-RB-Set-r16 | Bitstring of length 10, The leftmost 2 bits and rightmost 2 bits are set to "1" and the rest are set to "0" |  |  |  |  |  |
| 20MHz | 15 | PSFCH allocation | 1@0 | 1@105 | 2@0 | 2@104 | 1@0 | 1@104 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 1 | 0 | 1 | 0 |  |
|  |  | sl-SubchannelSize-r16 | 15 | 15 | 15 | 15 | 15 |  |
|  |  | sl-NumSubchannel-r16 | 7 | 7 | 7 | 7 | 7 |  |
|  |  | sl-PSFCH-RB-Set-r16 | Bitstring of length 105, The leftmost 14 bits and rightmost 14 bits are set to "1" and the rest are set to "0" |  |  |  |  |  |
|  | 30 | PSFCH allocation | 1@0 | 1@50 | 2@0 | 2@49 | 1@0 | 1@49 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 1 | 0 | 1 | 0 |  |
|  |  | sl-SubchannelSize-r16 | 25 | 25 | 25 | 25 | 25 |  |
|  |  | sl-NumSubchannel-r16 | 2 | 2 | 2 | 2 | 2 |  |
|  |  | sl-PSFCH-RB-Set-r16 | Bitstring of length 50, The leftmost 4 bits and rightmost 4 bits are set to "1" and the rest are set to "0" |  |  |  |  |  |
|  | 60 | PSFCH allocation | 1@0 | 1@23 | 2@0 | 2@22 | 1@0 | 1@23 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 0 | 0 | 0 |  |
|  |  | sl-SubchannelSize-r16 | 12 | 12 | 12 | 12 | 12 |  |
|  |  | sl-NumSubchannel-r16 | 2 | 2 | 2 | 2 | 2 |  |
|  |  | sl-PSFCH-RB-Set-r16 | Bitstring of length 24, The leftmost 4 bits and rightmost 4 bits are set to "1" and the rest are set to "0" |  |  |  |  |  |
| 30MHz | 15 | PSFCH allocation | 1@0 | 1@159 | 2@0 | 2@158 | 1@0 | 1@159 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 0 | 0 | 0 |  |
|  |  | sl-SubchannelSize-r16 | 20 | 20 | 20 | 20 | 20 |  |
|  |  | sl-NumSubchannel-r16 | 8 | 8 | 8 | 8 | 8 |  |
|  |  | sl-PSFCH-RB-Set-r16 | Bitstring of length 160, The leftmost 16 bits and rightmost 16 bits are set to "1" and the rest are set to "0" |  |  |  |  |  |
|  | 30 | PSFCH allocation | 1@0 | 1@77 | 2@0 | 2@76 | 1@0 | 1@74 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 3 | 0 | 3 | 0 |  |
|  |  | sl-SubchannelSize-r16 | 25 | 25 | 25 | 25 | 25 |  |
|  |  | sl-NumSubchannel-r16 | 3 | 3 | 3 | 3 | 3 |  |
|  |  | sl-PSFCH-RB-Set-r16 | Bitstring of length 75, The leftmost 6 bits and rightmost 6 bits are set to "1" and the rest are set to "0" |  |  |  |  |  |
|  | 60 | PSFCH allocation | 1@0 | 1@37 | 2@0 | 2@36 | 1@0 | 1@35 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 2 | 0 | 2 | 0 |  |
|  |  | sl-SubchannelSize-r16 | 12 | 12 | 12 | 12 | 12 |  |
|  |  | sl-NumSubchannel-r16 | 3 | 3 | 3 | 3 | 3 |  |
|  |  | sl-PSFCH-RB-Set-r16 | Bitstring of length 36, The leftmost 6 bits and rightmost 6 bits are set to "1" and the rest are set to "0" |  |  |  |  |  |

| BWchannel (MHz) | SCS (kHz) | Parameters | RB allocation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PSFCH_1RB_Left | PSFCH_1RB_Right | PSFCH_2RB_Left | PSFCH_2RB_Right | PSFCH_Max_Gap |  |
| 40MHz | 15 | PSFCH allocation | 1@0 | 1@215 | 2@0 | 2@214 | 1@0 | 1@215 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 0 | 0 | 0 |  |
|  |  | sl-SubchannelSize-r16 | 12 | 12 | 12 | 12 | 12 |  |
|  |  | sl-NumSubchannel-r16 | 18 | 18 | 18 | 18 | 18 |  |
|  |  | sl-PSFCH-RB-Set-r16 | Bitstring of length 216, The leftmost 36 bits and rightmost 36 bits are set to "1" and the rest are set to "0" |  |  |  |  |  |
|  | 30 | PSFCH allocation | 1@0 | 1@105 | 2@0 | 2@104 | 1@0 | 1@104 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 1 | 0 | 1 | 0 |  |
|  |  | sl-SubchannelSize-r16 | 15 | 15 | 15 | 15 | 15 |  |
|  |  | sl-NumSubchannel-r16 | 7 | 7 | 7 | 7 | 7 |  |
|  |  | sl-PSFCH-RB-Set-r16 | Bitstring of length 105, The leftmost 14 bits and rightmost 14 bits are set to "1" and the rest are set to "0" |  |  |  |  |  |
|  | 60 | PSFCH allocation | 1@0 | 1@50 | 2@0 | 2@49 | 1@0 | 1@49 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 1 | 0 | 1 | 0 |  |
|  |  | sl-SubchannelSize-r16 | 25 | 25 | 25 | 25 | 25 |  |
|  |  | sl-NumSubchannel-r16 | 2 | 2 | 2 | 2 | 2 |  |
|  |  | sl-PSFCH-RB-Set-r16 | Bitstring of length 50, The leftmost 4 bits and rightmost 4 bits are set to "1" and the rest are set to "0" |  |  |  |  |  |

Table 6.1E-2: Sidelink configuration for S-SSB (PSBCH)

| BWchannel (MHz) | SCS (kHz) | RB allocation |  |  |
| --- | --- | --- | --- | --- |
|  |  | S-SSB_Low | S-SSB_Mid | S-SSB_High |
| 10MHz | 15 | 11@0 | 11@21 | 11@41 |
|  | 30 | 11@0 | 11@7 | 11@13 |
|  | 60 | 11@0 | 11@0 | 11@0 |
| 20MHz | 15 | 11@0 | 11@48 | 11@95 |
|  | 30 | 11@0 | 11@20 | 11@40 |
|  | 60 | 11@0 | 11@7 | 11@13 |
| 30MHz | 15 | 11@0 | 11@75 | 11@149 |
|  | 30 | 11@0 | 11@34 | 11@67 |
|  | 60 | 11@0 | 11@14 | 11@27 |
| 40MHz | 15 | 11@0 | 11@103 | 11@205 |
|  | 30 | 11@0 | 11@48 | 11@95 |
|  | 60 | 11@0 | 11@20 | 11@40 |
| NOTE: sl-AbsoluteFrequencySSB-r16 for each S-SSB allocation is specified in TS 38.508-1 [5] 4.3.1.8. |  |  |  |  |

## 6.1F	General

For wideband operations, the minimum requirements for the transmitter characteristics are specified for transmissions on one scheduled RB set or ≥ 1 scheduled contiguous RB set(s) within the UE channel. The requirements apply with configured UL intra-cell guard bands of non-zero size according to Table 5.3.3-2, with the union of the scheduled RB sets and the intra-cell guard bands.

Unless stated otherwise, when a clause is not present for shared spectrum channel access, the general requirements and the additional clause requirements (suffices A, B, D) in clause 6 apply.

Table 6.1F-1: Common uplink configuration for shared spectrum access

| Channel Bandwidth | SCS (kHz) | OFDM | RB allocation |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Full | [Partial_Left]2 | [Partial_Right]3 |
| 20MHz | 15 | DFT-s | 100@0 | N/A1 | N/A1 |
|  |  | CP | 106@0 | 11 | 11 |
|  | 30 | DFT-s | 50@0 | N/A1 | N/A1 |
|  |  | CP | 51@0 | 11 | 11 |
| 40MHz | 15 | DFT-s | 216@0 | N/A1 | N/A1 |
|  |  | CP | 216@0 | 22 | 22 |
|  | 30 | DFT-s | 100@0 | N/A1 | N/A1 |
|  |  | CP | 106@0 | 22 | 22 |
| 60MHz | 15 | DFT-s | N/A | N/A1 | N/A1 |
|  |  | CP | N/A | N/A | N/A |
|  | 30 | DFT-s | 162@0 | N/A1 | N/A1 |
|  |  | CP | 162@0 | 33 | 33 |
| 80MHz | 15 | DFT-s | N/A | N/A1 | N/A1 |
|  |  | CP | N/A | N/A | N/A |
|  | 30 | DFT-s | 216@0 | N/A1 | N/A1 |
|  |  | CP | 217@0 | 44 | 44 |
| Note 1: Partial RB allocation consists of non-contiguous RB allocation in one or several interlaces and therefore DFT-s-OFDM is not applicable. Note 2: A set of distributed RBs over the channel bandwidth for one Interlace only. The lowest RB index in the channel bandwidth is part of the distributed RB set. Note 3: A set of distributed RBs over the channel bandwidth for one Interlace only. The highest RB index in the channel bandwidth is part of the distributed RB set. |  |  |  |  |  |

## 6.1I	General

Uplink RB allocations for eRedCap UE not supporting IE eRedCapNotReducedBB-BW-r18 given in Table 6.1I-1 are used throughout this section, unless otherwise stated by the test case.

Table 6.1I-1: Common uplink configuration for eRedCap UE not supporting IE eRedCapNotReducedBB-BW-r18

| Channel Bandwidth | SCS(kHz) | OFDM | RB allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Edge_Full_Left (Note 2) | Edge_Full_Right (Note 2) | Edge_1RB_Left | Edge_1RB_Right | Outer_Full | Inner_Full | Inner_1RB_Left | Inner_1RB_Right |
| 5MHz | 15 | DFT-s | 2@0 | 2@23 | 1@0 | 1@24 | 25@0 | 12@6 | 1@1 | 1@23 |
|  |  | CP | 2@0 | 2@23 | 1@0 | 1@24 | 25@0 | 13@6 | 1@1 | 1@23 |
|  | 30 | DFT-s | 2@0 | 2@9 | 1@0 | 1@10 | 10@0 | 5@21 | 1@1 | 1@9 |
|  |  | CP | 2@0 | 2@9 | 1@0 | 1@10 | 11@0 | 5@21 | 1@1 | 1@9 |
| 10MHz | 15 | DFT-s | 2@0 | 2@50 | 1@0 | 1@51 | 25@0 | 25@12 | 1@1 | 1@50 |
|  |  | CP | 2@0 | 2@50 | 1@0 | 1@51 | 25@0 | 25@12 | 1@1 | 1@50 |
|  | 30 | DFT-s | 2@0 | 2@22 | 1@0 | 1@23 | 12@0 | 12@6 | 1@1 | 1@22 |
|  |  | CP | 2@0 | 2@22 | 1@0 | 1@23 | 12@0 | 12@6 | 1@1 | 1@22 |
| 15MHz | 15 | DFT-s | 2@0 | 2@77 | 1@0 | 1@78 | 25@0 | 25@12 | 1@1 | 1@77 |
|  |  | CP | 2@0 | 2@77 | 1@0 | 1@78 | 25@0 | 25@12 | 1@1 | 1@77 |
|  | 30 | DFT-s | 2@0 | 2@36 | 1@0 | 1@37 | 12@0 | 12@6 | 1@1 | 1@36 |
|  |  | CP | 2@0 | 2@36 | 1@0 | 1@37 | 12@0 | 12@6 | 1@1 | 1@36 |
| 20MHz | 15 | DFT-s | 2@0 | 2@104 | 1@0 | 1@105 | 25@0 | 25@12 | 1@1 | 1@104 |
|  |  | CP | 2@0 | 2@104 | 1@0 | 1@105 | 25@0 | 25@12 | 1@1 | 1@104 |
|  | 30 | DFT-s | 2@0 | 2@49 | 1@0 | 1@50 | 12@0 | 12@6 | 1@1 | 1@49 |
|  |  | CP | 2@0 | 2@49 | 1@0 | 1@50 | 12@0 | 12@6 | 1@1 | 1@49 |
| Note 1: The allocated RB number LCRB is ceil(NRB/2) -1 in order to meet Inner RB allocation definition (RBStart,Low ≤ RBStart ≤ RBStart,High) described in subclause 6.2.2 of TS 38.101-1 [2]. Note 2: For power class 1.5, Edge_Full_Left is defined as 4 RBs allocated at the left edge of the transmission bandwidth (4@0), and Edge_Full_Right is defined as 4 RBs allocated at the right edge of the transmission bandwidth (4@NRB-4). |  |  |  |  |  |  |  |  |  |  |

## 6.1J	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector(s) of the ATG UE with one or multiple omni-directional antenna(s) or at the transceiver array boundary (TAB) connectors of the ATG UE with the antenna array. The definition about transceiver array boundary (TAB) is specified in clause 4.3.2 of TS 38.104 [27].

## 6.2	Transmitter power

### 6.2.1	UE maximum output power

6.2.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2.1.2	Test applicability

This test case applies to all types of NR Power Class 1 UE release 15 and forward.

This test case applies to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity and don't support Redcap.

6.2.1.3	Minimum conformance requirements

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.1.3-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  | 26 | +2/-3 | 23 | ± 2 |
| n2 |  |  | 26 | +2/-33 | 23 | ± 23 |
| n3 |  |  | 26 | +2/-33 | 23 | ± 23 |
| n5 |  |  |  |  | 23 | ± 2 |
| n7 | 316 | +2/-3 | 26 | +2/-33 | 23 | ± 23 |
| n8 |  |  | 26 | +2/-33 | 23 | ± 23 |
| n12 |  |  |  |  | 23 | ± 23 |
| n13 |  |  |  |  | 23 | ±2 |
| n14 | 316 | +2/-3 | 26 | +2/-3 | 23 | ± 2 |
| n20 |  |  |  |  | 23 | ± 23 |
| n24 |  |  |  |  | 23 | +2/-33 |
| n25 | 316 | +2/-3 |  |  | 23 | ± 23 |
| n26 |  |  |  |  | 23 | ± 23 |
| n28 |  |  |  |  | 23 | +2/-2.5 |
| n30 |  |  |  |  | 23 | ± 2 |
| n31 |  |  |  |  | 23 | ±2 |
| n34 |  |  | 26 | +2/-3 | 23 | ± 2 |
| n38 |  |  |  |  | 23 | ± 2 |
| n39 |  |  | 26 | +2/-3 | 23 | ± 2 |
| n40 | 316 | +2/-3 | 26 | +2/-3 | 23 | ± 2 |
| n41 | 316 | +2/-3 | 26 | +2/-33 | 23 | ± 23 |
| n48 |  |  |  |  | 23 | +2/-3 |
| n50 |  |  |  |  | 23 | ± 2 |
| n51 |  |  |  |  | 23 | ± 2 |
| n53 |  |  |  |  | 23 | ±2 |
| n54 |  |  |  |  | 23 | ±2 |
| n65 |  |  |  |  | 23 | ±2 |
| n66 | 316 | +2/-3 | 26 | +2/-3 | 23 | ± 2 |
| n70 |  |  |  |  | 23 | ± 2 |
| n71 | 316 | +2/-3 |  |  | 23 | +2/-2.5 |
| n72 |  |  |  |  | 23 | ±2 |
| n74 |  |  |  |  | 23 | ± 2 |
| n77 | 316 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n78 | 316 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n79 |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n80 |  |  |  |  | 23 | ± 2 |
| n81 |  |  |  |  | 23 | ± 2 |
| n82 |  |  |  |  | 23 | ± 2 |
| n83 |  |  |  |  | 23 | + 2/-2.5 |
| n84 |  |  |  |  | 23 | ± 2 |
| n85 | 316 | +2/-3 |  |  | 23 | ±23 |
| n86 |  |  |  |  | 23 | ± 2 |
| n91 |  |  |  |  | 23 | ±23, 4 |
| n92 |  |  |  |  | 23 | ±23, 4 |
| n93 |  |  |  |  | 23 | ±23, 4 |
| n94 |  |  |  |  | 23 | ±23, 4 |
| n95 |  |  |  |  | 23 | ±2 |
| n97 |  |  |  |  | 23 | ±2 |
| n98 |  |  | 26 | +2/-3 | 23 | ±2 |
| n99 |  |  |  |  | 23 | +2/-33 |
| n100 | 316 | +2/-3 |  |  | 23 | ±2 |
| n101 | 316 | +2/-3 |  |  | 23 | ±2 |
| n105 |  |  |  |  | 23 | +2/-2.5 |
| n106 |  |  |  |  | 23 | ±2 |
| n109 |  |  |  |  | 23 | ±23, 4 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Power class 3 is default power class unless otherwise stated. NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. NOTE 5: FFS NOTE 6: Generally, PC1 UE is not targeted for smartphone form factor. The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. |  |  |  |  |  |  |

If a UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

- if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or

- if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.331 [6] (The exact evaluation period is no less than one radio frame); or

-	if the IE P-Max as defined in TS 38.331 [6] is provided and set to the maximum output power of the default power class or lower;

-	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in sub-clause 6.2.4;

- else if the UE does not support a power class with higher maximum output power than PC2; or

-	if the IE P-Max as defined in TS 38.331 [6] is provided and set to the maximum output power of the power class 2 or lower;

-	shall apply all requirements for power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2.4;

-	else shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2.4;

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.1.

6.2.1.4	Test description

6.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest (NOTE 3) |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for maximum output | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | power test case | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Left |
| 3 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Right |
| 4 |  | DFT-s-OFDM QPSK | Inner Full |
| 5 |  | DFT-s-OFDM QPSK | Inner 1RB Left |
| 6 |  | DFT-s-OFDM QPSK | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For band n28, the Highest test channel bandwidth is replaced by 20MHz due to MPR is always larger than 0dB for 30MHz bandwidth. NOTE 4: For P-Max test (Step 4 in Section 6.2.1.4.2) and maxUplinkDutyCycle test (Step 5 in Section 6.2.1.4.2), only “Normal” test environment, “Low range” test frequencies, “Lowest” Test Channel Bandwidth, “Lowest” test SCS and “Inner Full” RB allocation to be used. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2.1.4.3.

6.2.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbols with transient periods are not under test.

4.	For UEs supporting Power Class 2 or Power Class 1, repeat steps 1~3 on the applicable bands with message exception of P-Max defined in Table 6.2.1.4.3-2.

5.	For TDD UEs supporting Power Class 2 with UE capability maxUplinkDutyCycle-PC2-FR1, repeat steps 1~3 on the applicable bands with message exception of TDD UL-DL pattern defined in Table 6.2.1.4.3-4 and Table 6.2.1.4.3-5 which the UplinkDutyCycle is closest less than or equal to the maxUplinkDutyCycle-PC2-FR1 reported by UE. Before Step 3, wait for at least 10ms for the UE to complete the evaluation period.

6.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions.

Table 6.2.1.4.3-1: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.2.1.4.3-2: P-Max (Step 4)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 | PC2 UE or PC1 UE will fallback to PC3 UE with P-Max=23 | PC2 UE or PC1 UE |

Table 6.2.1.4.3-3: Void

Table 6.2.1.4.3-4: TDD UL-DL pattern for SCS 15 KHz (UplinkDutyCycle=60%)

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| TDD Slot Configuration pattern (Note 1) |  |  | DSUUU |  |
| Special Slot Configuration (Note 2) |  |  | 10D+2G+2U |  |
| UL-DL configuration (tdd-UL-DL-ConfigurationCommon) | referenceSubcarrierSpacing | kHz | 15 |  |
|  | dl-UL-TransmissionPeriodicity | ms | 5 |  |
|  | nrofDownlinkSlots |  | 1 |  |
|  | nrofDownlinkSymbols |  | 10 |  |
|  | nrofUplinkSlot |  | 3 |  |
|  | nrofUplinkSymbols |  | 2 |  |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | [4] if mod(i,5) = 0 |  |
| PUCCH resource indicator |  |  | PUCCH-ResourceId[8] = 7 in pucch-ResourceSetID[1] as defined in TS 38.508-1 [5] Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213) | ‘111’B value in binary |
| SS/PBCH block index |  |  | 0 | SSB#0 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,9} Note 4: There shall be no PUSCH or PUCCH or SRS transmitted in slot 1 and slot 6 to meet the specific UplinkDutyCycle. |  |  |  |  |

Table 6.2.1.4.3-5: TDD UL-DL pattern for SCS 30 KHz (UplinkDutyCycle=60%)

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| TDD Slot Configuration pattern (Note 1) |  |  | DSUUU |  |
| Special Slot Configuration (Note 2) |  |  | 10D+2G+2U |  |
| UL-DL configuration (tdd-UL-DL-ConfigurationCommon) | referenceSubcarrierSpacing | kHz | 30 |  |
|  | dl-UL-TransmissionPeriodicity | ms | 2.5 single |  |
|  | nrofDownlinkSlots |  | 1 |  |
|  | nrofDownlinkSymbols |  | 10 |  |
|  | nrofUplinkSlot |  | 3 |  |
|  | nrofUplinkSymbols |  | 2 |  |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | [4] if mod(i,5) = 0 |  |
| PUCCH resource indicator |  |  | PUCCH-ResourceId[8] = 7 in pucch-ResourceSetID[1] as defined in TS 38.508-1 [5] Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213) | ‘111’B value in binary |
| SS/PBCH block index |  |  | 0 | SSB#0 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,19} Note 4: There shall be no PUSCH or PUCCH or SRS transmitted in slot 1, slot 6, slot 11 and slot 16 to meet the specific UplinkDutyCycle. |  |  |  |  |

6.2.1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-1 for Power Class 3, Table 6.2.1.5-2 for Power Class 2, Table 6.2.1.5-2a for Power Class 1.

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-1.

The maximum output power, derived in step 5 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-2 for Power Class 2 as defined in TS 38.306.

Table 6.2.1.5-1: Maximum Output Power test requirement for Power Class 3

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | ±(2+TT) |
| n2 |  |  |  |  | 23 | ±(23+TT) |
| n3 |  |  |  |  | 23 | ±(23+TT) |
| n5 |  |  |  |  | 23 | ±(2+TT) |
| n7 |  |  |  |  | 23 | ±(23+TT) |
| n8 |  |  |  |  | 23 | ±(23+TT) |
| n12 |  |  |  |  | 23 | ±(23+TT) |
| n13 |  |  |  |  | 23 | ±(2+TT) |
| n14 |  |  |  |  | 23 | ±(2+TT) |
| n20 |  |  |  |  | 23 | ±(23+TT) |
| n24 |  |  |  |  | 23 | +2+TT/-3.03-TT |
| n25 |  |  |  |  | 23 | ±(23+TT) |
| n26 |  |  |  |  | 23 | ±(23+TT) |
| n28 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n30 |  |  |  |  | 23 | ±(2+TT) |
| n31 |  |  |  |  | 23 | ±(2+TT) |
| n34 |  |  |  |  | 23 | ±(2+TT) |
| n38 |  |  |  |  | 23 | ±(2+TT) |
| n39 |  |  |  |  | 23 | ±(2+TT) |
| n40 |  |  |  |  | 23 | ±(2+TT) |
| n41 |  |  |  |  | 23 | ±(23+TT) |
| n48 |  |  |  |  | 23 | +2+TT/-3-TT |
| n50 |  |  |  |  | 23 | ±(2+TT) |
| n51 |  |  |  |  | 23 | ±(2+TT) |
| n53 |  |  |  |  | 23 | ±(2+TT) |
| n54 |  |  |  |  | 23 | ±(2+TT) |
| n65 |  |  |  |  | 23 | ±(2+TT) |
| n66 |  |  |  |  | 23 | ±(2+TT) |
| n70 |  |  |  |  | 23 | ±(2+TT) |
| n71 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n72 |  |  |  |  | 23 | ±(2+TT) |
| n74 |  |  |  |  | 23 | ±(2+TT) |
| n77 |  |  |  |  | 23 | +2+TT/-3-TT |
| n78 |  |  |  |  | 23 | +2+TT/-3-TT |
| n79 |  |  |  |  | 23 | +2+TT/-3-TT |
| n85 |  |  |  |  | 23 | ±(23+TT) |
| n91 |  |  |  |  | 23 | ±(23, 5+TT) |
| n92 |  |  |  |  | 23 | ±(23, 5+TT) |
| n93 |  |  |  |  | 23 | ±(23, 5+TT) |
| n94 |  |  |  |  | 23 | ±(23, 5+TT) |
| n100 |  |  |  |  | 23 | ±(2+TT) |
| n101 |  |  |  |  | 23 | ±(2+TT) |
| n105 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n106 |  |  |  |  | 23 | ±(2+TT) |
| n109 |  |  |  |  | 23 | ±(2+TT) |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3 NOTE 5: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. |  |  |  |  |  |  |

Table 6.2.1.5-2: Maximum Output Power test requirement for Power Class 2

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  | 26 | +2+TT/-3-TT |  |  |
| n2 |  |  | 26 | +2+TT/-3-TT |  |  |
| n3 |  |  | 26 | +2+TT/-33-TT |  |  |
| n7 |  |  | 26 | +2+TT/-33-TT |  |  |
| n8 |  |  | 26 | +2+TT/-33-TT |  |  |
| n14 |  |  | 26 | +2+TT/-3-TT |  |  |
| n34 |  |  | 26 | +2+TT/-3-TT |  |  |
| n39 |  |  | 26 | +2+TT/-3-TT |  |  |
| n40 |  |  | 26 | +2+TT/-3-TT | n40 |  |
| n41 |  |  | 26 | +2+TT/-33-TT |  |  |
| n66 |  |  | 26 | +2+TT/-3-TT |  |  |
| n77 |  |  | 26 | +2+TT/-3-TT |  |  |
| n78 |  |  | 26 | +2+TT/-3-TT |  |  |
| n79 |  |  | 26 | +2+TT/-3-TT |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3 |  |  |  |  |  |  |

Table 6.2.1.5-2a: Maximum Output Power test requirement for Power Class 1

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n75 | 31 | +2+TT/-3-TT |  |  |  |  |
| n145 | 31 | +2+TT/-3-TT |  |  |  |  |
| n255 | 31 | +2+TT/-3-TT |  |  |  |  |
| n405 | 31 | +2+TT/-3-TT |  |  |  |  |
| n415 | 31 | +2+TT/-3-TT |  |  |  |  |
| n665 | 31 | +2+TT/-3-TT |  |  |  |  |
| n715 | 31 | +2+TT/-3-TT |  |  |  |  |
| n775 | 31 | +2+TT/-3-TT |  |  |  |  |
| n785 | 31 | +2+TT/-3-TT |  |  |  |  |
| n855 | 31 | +2+TT/-3-TT |  |  |  |  |
| n1005 | 31 | +2+TT/-3-TT |  |  |  |  |
| n1015 | 31 | +2+TT/-3-TT |  |  |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3. NOTE 5: Generally, PC1 UEis not targeted for smartphone form factor. The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. |  |  |  |  |  |  |

Table 6.2.1.5-2b: Void

Table 6.2.1.5-3: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

### 6.2.2	UE maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Message contents for R18 coverage enhancement are FFS due to the relative IEs not introduced into TS 38.331 yet.

- The test is incomplete for R18 coverage enhancement and the following test IDs cannot be tested:
Test IDs 40, 41, 42 in Tables 6.2.2.4.1-1 and 6.2.2.5-1
Test IDs 33 and 34 in Tables 6.2.2.4.1-2 and 6.2.2.5-4

6.2.2.1	Test purpose

The number of RB identified in Table 6.2.2.3-1 is based on meeting the requirements for adjacent channel leakage ratio and the maximum power reduction (MPR) due to Cubic Metric (CM).

6.2.2.2	Test applicability

This test case applies to all types of NR Power Class 1 UE release 15 and forward.

This test case applies to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5.2.4.1 is executed.

6.2.2.3	Minimum conformance requirements

UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE power class 2 and 3 and UE power class 1, the allowed maximum power reduction (MPR) is defined in Table 6.2.2.3-2, Table 6.2.2.3-1, Table 6.2.2.3-4b and Table 6.2.2.3-5, respectively for channel bandwidths ≤ 100 MHz.

If the relative channel bandwidth ≤ 4% for TDD bands or ≤ 3% for FDD band, the ∆MPR is set to zero.

If the relative channel bandwidth is larger than 4% for TDD bands or 3% for FDD bands, the ∆MPR is defined in Table 6.2.2.3-3.

Where relative channel bandwidth = 2*BWChannel / (FUL_low + FUL_high).

The allowed MPR for SRS, PUCCH formats 0, 1, 3 and 4, and PRACH shall be as specified for QPSK modulated DFT-s-OFDM of equivalent RB allocation. The allowed MPR for PUCCH format 2 shall be as specified for QPSK modulated CP-OFDM of equivalent RB allocation.

Table 6.2.2.3-1: Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52,3 | ≤ 0.52 | 02 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52,3 | 02 | 02,4 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0dB MPR is 26dBm. NOTE 2: Applicable for conditions where Note 1 does not apply. NOTE 3: For 3 MHz channel bandwidth the Pi/2 BPSK edge allocation MPR is 1 dB. NOTE 4: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostPi2BPSK-r18 is set to 1, the reference power is increased by [ΔPPowerBoost - ΔPPowerClass] NOTE 5: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostQPSK-r18 is set to 1, the reference power is increased by [ΔPPowerBoost - ΔPPowerClass]. |  |  |  |  |

Table 6.2.2.3-2: Maximum power reduction (MPR) for power class 2

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 0.5 | 0 |
|  | QPSK | ≤ 3.5 | ≤ 1 | 0 |
|  | 16 QAM | ≤ 3.5 | ≤ 2 | ≤ 1 |
|  | 64 QAM | ≤ 3.5 | ≤ 2.5 |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 3 | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostPi2BPSK-r18 is set to 1. The reference power is increased by [ΔPPowerBoost - ΔPPowerClass] NOTE 2: Applicable for a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostQPSK-r18 is set to 1. The reference power is increased by [ΔPPowerBoost - ΔPPowerClass]. |  |  |  |  |

Table 6.2.2.3-3: ∆MPR

| NR Band | Power class | Channel bandwidth | ∆MPR (dB) |
| --- | --- | --- | --- |
| n28 and n83 | Power class 3 | 30 MHz | 0.5 |
| n40 and n97 | Power class 3 and power class 2 | 100 MHz | 1 |

Table 6.2.2.3-4: Void

Table 6.2.2.3-4a: Void

Table 6.2.2.3-4b: Maximum power reduction (MPR) for power class 1 for bands other than Band n14

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | (NOTE 1) | ≤ 0.5 | 0 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | (NOTE 1) | 0 | 0 |
|  | QPSK | (NOTE 1) | ≤ 1 | 0 |
|  | 16 QAM | (NOTE 1) | ≤ 2 | ≤ 1 |
|  | 64 QAM | (NOTE 1) | ≤ 2.5 |  |
|  | 256 QAM | (NOTE 1) | ≤ 4.5 |  |
| CP-OFDM | QPSK | (NOTE 1) | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | (NOTE 1) | ≤ 3 | ≤ 2 |
|  | 64 QAM | (NOTE 1) | ≤ 3.5 |  |
|  | 256 QAM | (NOTE 1) | ≤ 6.5 |  |
| NOTE 1: MPR for all modulations for Edge RB allocation is defined as following for two distinguished channel bandwidths groups as: Within the <50MHz channel bandwidth group: MPR= CEIL7.2 dB- 6 dB∙ CBW100 MHz, 0.5 dB Within the ≥50MHz channel bandwidth group: MPR= CEIL5.35 dB+3.15 dB∙ CBW100 MHz, 0.5 dB where CEIL(x,0.5 dB) means rounding x upwards to the closest multiple of 0.5 dB. |  |  |  |  |

Table 6.2.2.3-5: Maximum power reduction (MPR) for power class 1 for Band n14

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 0.5 | ≤ 0.5 | 0 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.5 | 0 | 0 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met:

RBStart,Low ≤ RBStart ≤ RBStart,High, and

LCRB ≤ ceil(NRB/2)

where ceil(x) is the smallest integer greater than or equal to x.

An Edge RB allocation is the one for which the RB(s) is (are) allocated at the lowermost or uppermost edge of the channel with LCRB ≤ 2 RBs.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation or Edge RB allocation.

When ΔPPowerBoost is a positive value and the UE supports powerBoosting-pi2BPSK-QPSK-r18

-	An enhanced power inner allocation region within the inner region is defined so any waveform it contains satisfies the following condition:

RBStart,Low + P1  ≤  RBStart  ≤  RBStart,High - P1

Where P1 = MIN{12,CEIL(2+NRB/25)}

-	When a UE that supports powerBoosting-pi2BPSK-QPSK-r18 but does not support powerBoosting-pi2BPSK-QPSK-Modified-r18, an RB allocation that belongs to the inner region but is outside the enhanced power inner region, the applicable MPR from table 6.2.2-x is increased by the value of ΔPPowerBoost.

If CP-OFDM allocation satisfies following conditions, it is considered as almost contiguous allocation

NRB_gap / (NRB_alloc + NRB_gap) ≤ 0.25

and NRB_alloc + NRB_gap is larger than 106, 51 or 24 RBs for 15 kHz, 30 kHz or 60 kHz SCS respectively where NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs. The size and location of allocated and unallocated RBs are restricted by RBG parameters specified in sub-clause 6.1.2.2 of TS 38.214 [12]. For UE that indicates support for almostContiguousCP-OFDM-UL, the almost contiguous signals in power class 2 and 3, the allowed maximum power reduction defined in Table 6.2.2.3-2 and Table 6.2.2.3-1 are increased by

CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB,

where CEIL{x,0.5} means x rounding upwards to closest 0.5dB. The parameters of RBStart,Low and RBStart,High to specify valid RB allocation ranges for Outer and Inner RB allocations are defined as following:

RBStart,Low = max(1, floor((NRB_alloc + NRB_gap)/2))

RBStart,High = NRB – RBStart,Low – NRB_alloc –NRB_gap

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2.4 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.2.

6.2.2.4	Test description

6.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2.2.4.1-1, Table 6.2.2.4.1-2, Table 6.2.2.4.1-2a, Table 6.2.2.4.1-2c and Table 6.2.2.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.2.4.1-1: Test Configuration Table for power class 3 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 13 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 23 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 33 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 43 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 54 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 64 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 74 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 84 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 10 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 13 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 14 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 15 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 16 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 17 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 18 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 19 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 20 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 21 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 22 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 23 | Default |  | CP-OFDM QPSK | Inner Full |
| 24 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 25 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM QPSK | Outer Full |
| 27 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 28 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 29 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 30 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 31 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 32 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 33 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 34 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 35 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 36 | Default |  | CP-OFDM 256 QAM | Outer Full |
| 374,5 | Low |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 384,5 | High |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 394,5 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| 406 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 417 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 426 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. NOTE 5: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. NOTE 6: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostPi2BPSK-r18 is set to 1, the reference power is increased by [ΔPPowerBoost - ΔPPowerClass]. NOTE 7: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostQPSK-r18 is set to 1, the reference power is increased by [ΔPPowerBoost - ΔPPowerClass]. |  |  |  |  |

Table 6.2.2.4.1-1a: Void

Table 6.2.2.4.1-2: Test Configuration Table for power class 2 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| 333 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 344 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: Applicable for a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostPi2BPSK-r18 is set to 1. The reference power is increased by [ΔPPowerBoost - ΔPPowerClass] NOTE 4: Applicable for a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostQPSK-r18 is set to 1. The reference power is increased by [ΔPPowerBoost - ΔPPowerClass]. |  |  |  |  |

Table 6.2.2.4.1-2a: Test Configuration Table for power class 1 for Band n14 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| 333 | Low |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 343 | High |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 353 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for Ues which supports Pi/2 BPSK in FR1. NOTE 3: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. |  |  |  |  |

Table 6.2.2.4.1-2b: Void

Table 6.2.2.4.1-2c: Test Configuration Table for power class 1 for bands other than Band n14 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest for all RB allocations Additionally, for edge RB allocations, the maximum channel bandwidth <50MHz and the minimum channel bandwidth ≥50MHz that are supported by the UE (NOTE 4) |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| 333 | Low |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 343 | High |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 353 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for Ues which supports Pi/2 BPSK in FR1. NOTE 3: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. NOTE 4: The maximum channel bandwidth <50MHz and the minimum channel bandwidth ≥50MHz are tested only with the highest order modulation supported by the UE. |  |  |  |  |

Table 6.2.2.4.1-3: Test Configuration Table for power class 2&3 (almost contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Default |  | CP-OFDM QPSK | Outer Full |
| 3 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 4 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 5 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 6 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Tables 6.2.2.4.1-4, 6.2.2.4.1-4a and 6.2.2.4.1-4b. NOTE 2: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |

Table 6.2.2.4.1-4: Uplink configuration for almost contiguous allocation for low frequency range

| Channel Bandwidth(MHz) | SCS(kHz) | OFDM | Outer Full |  | Inner Full |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cluster1 RB allocations | Cluster2 RB allocations | Cluster1 RB allocations | Cluster2 RB allocations |
|  |  |  | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) |
| 25 | 15 | CP | 48@0 | 53@80 | N/A | N/A |
|  | 30 | CP | 24@0 | 25@40 | N/A | N/A |
|  | 60 | CP | 12@0 | 13@18 | N/A | N/A |
| 30 | 15 | CP | 64@0 | 64@96 | N/A | N/A |
|  | 30 | CP | 32@0 | 30@48 | N/A | N/A |
|  | 60 | CP | 16@0 | 14@24 | N/A | N/A |
| 35 | 15 | CP | 80@0 | 76@112 | N/A | N/A |
|  | 30 | CP | 40@0 | 36@56 | N/A | N/A |
|  | 60 | CP | 20@0 | 16@28 | N/A | N/A |
| 40 | 15 | CP | 80@0 | 88@128 | N/A | N/A |
|  | 30 | CP | 40@0 | 42@64 | N/A | N/A |
|  | 60 | CP | 20@0 | 19@32 | 12@12 | 8@28 |
| 45 | 15 | CP | 80@0 | 114@128 | 48@64 | 48@128 |
|  | 30 | CP | 40@0 | 55@64 | 24@32 | 24@64 |
|  | 60 | CP | 20@0 | 26@32 | 12@16 | 12@32 |
| 50 | 15 | CP | 96@0 | 110@160 | 48@64 | 48@144 |
|  | 30 | CP | 48@0 | 53@80 | 24@32 | 24@72 |
|  | 60 | CP | 24@0 | 25@40 | 12@16 | 12@36 |
| 60 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 64@0 | 66@96 | 32@32 | 16@80 |
|  | 60 | CP | 32@0 | 31@48 | 16@16 | 8@40 |
| 70 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 80@0 | 77@112 | 32@32 | 16@80 |
|  | 60 | CP | 40@0 | 37@56 | 16@16 | 8@40 |
| 80 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 80@0 | 89@128 | 32@32 | 16@80 |
|  | 60 | CP | 40@0 | 43@64 | 16@16 | 8@40 |
| 90 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 96@0 | 101@144 | 32@32 | 16@80 |
|  | 60 | CP | 48@0 | 49@72 | 16@16 | 8@40 |
| 100 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 112@0 | 97@176 | 48@64 | 48@144 |
|  | 60 | CP | 48@0 | 55@80 | 24@32 | 24@72 |

Table 6.2.2.4.1-4a: Uplink configuration for almost contiguous allocation for TDD high frequency range

| Channel Bandwidth(MHz) | SCS(kHz) | OFDM | Outer Full |  | Inner Full |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cluster1 RB allocations | Cluster2 RB allocations | Cluster1 RB allocations | Cluster2 RB allocations |
|  |  |  | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) |
| 25 | 15 | CP | 48@0 | 53@80 | N/A | N/A |
|  | 30 | CP | 24@0 | 25@40 | N/A | N/A |
|  | 60 | CP | 12@0 | 13@18 | N/A | N/A |
| 30 | 15 | CP | 56@0 | 64@88 | N/A | N/A |
|  | 30 | CP | 32@0 | 30@48 | N/A | N/A |
|  | 60 | CP | 16@0 | 14@24 | N/A | N/A |
| 35 | 15 | CP | 72@0 | 84@104 | N/A | N/A |
|  | 30 | CP | 40@0 | 36@56 | N/A | N/A |
|  | 60 | CP | 20@0 | 16@28 | N/A | N/A |
| 40 | 15 | CP | 72@0 | 96@120 | N/A | N/A |
|  | 30 | CP | 40@0 | 42@64 | N/A | N/A |
|  | 60 | CP | 20@0 | 19@32 | 12@12 | 8@28 |
| 45 | 15 | CP | 72@0 | 122@120 | 48@56 | 48@120 |
|  | 30 | CP | 40@0 | 55@64 | 24@32 | 24@64 |
|  | 60 | CP | 20@0 | 26@32 | 12@16 | 12@32 |
| 50 | 15 | CP | 88@0 | 112@152 | 48@72 | 48@152 |
|  | 30 | CP | 48@0 | 53@80 | 24@32 | 24@72 |
|  | 60 | CP | 24@0 | 25@40 | 12@16 | 12@36 |
| 60 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 56@0 | 74@88 | 32@40 | 16@88 |
|  | 60 | CP | 32@0 | 31@48 | 16@16 | 8@40 |
| 70 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 72@0 | 80@104 | 32@40 | 16@88 |
|  | 60 | CP | 40@0 | 37@56 | 16@16 | 8@40 |
| 80 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 72@0 | 96@120 | 32@40 | 16@88 |
|  | 60 | CP | 40@0 | 43@64 | 16@16 | 8@40 |
| 90 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 88@0 | 109@136 | 32@40 | 16@88 |
|  | 60 | CP | 48@0 | 49@72 | 16@16 | 8@40 |
| 100 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 104@0 | 105@168 | 48@72 | 48@152 |
|  | 60 | CP | 48@0 | 55@80 | 24@32 | 24@72 |

Table 6.2.2.4.1-4b: Uplink configuration for almost contiguous allocation for FDD high frequency range

| Channel Bandwidth(MHz) | SCS(kHz) | OFDM | Outer Full |  | Inner Full |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cluster1 RB allocations | Cluster2 RB allocations | Cluster1 RB allocations | Cluster2 RB allocations |
|  |  |  | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) |
| 25 | 15 | CP | 42@0 | 56@74 | N/A | N/A |
|  | 30 | CP | 22@0 | 27@38 | N/A | N/A |
|  | 60 | CP | 12@0 | 13@18 | N/A | N/A |
| 30 | 15 | CP | 58@0 | 64@90 | N/A | N/A |
|  | 30 | CP | 26@0 | 32@42 | N/A | N/A |
|  | 60 | CP | 14@0 | 16@22 | N/A | N/A |
| 35 | 15 | CP | 74@0 | 82@106 | N/A | N/A |
|  | 30 | CP | 42@0 | 34@58 | N/A | N/A |
|  | 60 | CP | 22@0 | 18@26 | N/A | N/A |
| 40 | 15 | CP | 74@0 | 94@122 | N/A | N/A |
|  | 30 | CP | 34@0 | 48@58 | N/A | N/A |
|  | 60 | CP | 18@0 | 20@30 | 12@14 | 8@30 |
| 45 | 15 | CP | 74@0 | 120@122 | 48@58 | 48@122 |
|  | 30 | CP | 34@0 | 56@58 | 24@34 | 24@66 |
|  | 60 | CP | 18@0 | 28@30 | 12@14 | 12@30 |
| 50 | 15 | CP | 90@0 | 112@154 | 48@74 | 48@154 |
|  | 30 | CP | 42@0 | 56@74 | 24@34 | 24@74 |
|  | 60 | CP | 22@0 | 27@38 | N/A | N/A |
| 60 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 58@0 | 72@90 | 32@42 | 16@90 |
|  | 60 | CP | 26@0 | 32@42 | 16@18 | 8@42 |
| 70 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 74@0 | 80@106 | 32@42 | 16@90 |
|  | 60 | CP | 34@0 | 40@50 | 16@18 | 8@42 |
| 80 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 74@0 | 95@122 | 32@42 | 16@90 |
|  | 60 | CP | 34@0 | 48@58 | 16@18 | 8@42 |
| 90 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 90@0 | 107@138 | 32@42 | 16@90 |
|  | 60 | CP | 42@0 | 55@66 | 16@18 | 8@42 |
| 100 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 106@0 | 103@170 | 48@74 | 48@154 |
|  | 60 | CP | 42@0 | 56@74 | 24@34 | 24@74 |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.2.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2.2.4.1-1, Table 6.2.2.4.1-2, Table 6.2.2.4.1-2a, Table 6.2.2.4.1-2c and Table 6.2.2.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2.2.4.3.

6.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.2.4.1-1, Table 6.2.2.4.1-2, Table 6.2.2.4.1-2a, Table 6.2.2.4.1-2c and Table 6.2.2.4.1-3. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

4.	For UEs supporting Power Class 1 and Power Class 2, repeat steps 1~3 for Test ID 22 and 36 in Table 6.2.2.4.1-1 on the applicable bands with message exception of P-Max defined in Table 6.2.2.4.3-2.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2.2.4.1-1, Table 6.2.2.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions:

Table 6.2.2.4.3-1: PUSCH-Config

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-118 PUSCH-Config |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  | Almost contiguous allocation |
|  | resourceAllocationType1 |  | Contiguous allocation |
| } |  |  |  |

Table 6.2.2.4.3-2: P-Max (Step 4)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  | PC2 UE or PC1 UE |

Table 6.2.2.4.3-2a: Void

Table 6.2.2.4.3-3: DMRS-UplinkConfig (Test ID 37 - 39 in Table 6.2.2.4.1-1)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-51 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-UplinkConfig ::= SEQUENCE { |  |  |  |
| transformPrecodingEnabled SEQUENCE { |  |  |  |
| dmrs-UplinkTransformPrecoding-r16 SEQUENCE { |  |  |  |
| pi2BPSK-ScramblingID0 | Not present |  |  |
| pi2BPSK-ScramblingID1 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.2.4.3-4: ServingCellConfig

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| powerBoostPi2BPSK | 1 |  | Test IDs where NOTE 3 in Table 6.2.2.4.1-1 applies. |
|  | 0 |  | Test IDs where NOTE 4 in Table 6.2.2.4.1-1 applies. |
| } |  |  |  |
| } |  |  |  |

6.2.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1 to Table 6.2.2.5-9a.

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1,4 Table 6.2.2.5-r.

Table 6.2.2.5-1: UE Power Class test requirements(for Bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n30, n31, n34, n38, n39, n40, n41, n50, n51, n53, n54, n65, n66, n70, n72, n74, n85, n91, n92, n93, n94, n100, n101, n106, n109) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | ΔPPowerBoost (dB) | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 0.2 | 0 | （1.52） | 0 | 25.8 | （24.32） | 2.0 |  | 2 | 28.0 + TT | 23.8 - TT | （22.3 - TT2） |
| 2 | 23 | -3 | 3.5 | 0 | （1.52） | 0 | 22.5 | （21.02） | 2.0 |  | 2 | 28.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 3 | 23 | -3 | 3.5 | 0 | （1.52） | 0 | 22.5 | （21.02） | 2.0 |  | 2 | 28.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 4 | 23 | -3 | 1.2 | 0 | （1.52） | 0 | 24.8 | （23.32） | 2.0 |  | 2 | 28.0 + TT | 22.8 - TT | （21.3 - TT2） |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 0 | 23.0 | （21.52） | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT4 | （19.5 - TT2） |
| 6 | 23 | 0 | 0.55 | 0 | （1.52） | 0 | 22.5 | （21.02） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 7 | 23 | 0 | 0.55 | 0 | （1.52） | 0 | 22.5 | （21.02） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 8 | 23 | 0 | 0.5 | 0 | （1.52） | 0 | 22.5 | （21.02） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 9 | 23 | 0 | 0 | 0 | （1.52） | 0 | 23.0 | （21.52） | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT4 | （19.5 - TT2） |
| 10 | 23 | 0 | 1 | 0 | （1.52） | 0 | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 11 | 23 | 0 | 1 | 0 | （1.52） | 0 | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 12 | 23 | 0 | 1 | 0 | （1.52） | 0 | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 13 | 23 | 0 | 1 | 0 | （1.52） | 0 | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 14 | 23 | 0 | 2 | 0 | （1.52） | 0 | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 15 | 23 | 0 | 2 | 0 | （1.52） | 0 | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 16 | 23 | 0 | 2 | 0 | （1.52） | 0 | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 17 | 23 | 0 | 2.5 | 0 | （1.52） | 0 | 20.5 | （19.02） | 2.5 | （3.52） | 2 | 25.0 + TT | 18.0 - TT | （15.5 - TT2） |
| 18 | 23 | 0 | 2.5 | 0 | （1.52） | 0 | 20.5 | （19.02） | 2.5 | （3.52） | 2 | 25.0 + TT | 18.0 - TT | （15.5 - TT2） |
| 19 | 23 | 0 | 2.5 | 0 | （1.52） | 0 | 20.5 | （19.02） | 2.5 | （3.52） | 2 | 25.0 + TT | 18.0 - TT | （15.5 - TT2） |
| 20 | 23 | 0 | 4.5 | 0 | （1.52） | 0 | 18.5 | （17.02） | 4.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 21 | 23 | 0 | 4.5 | 0 | （1.52） | 0 | 18.5 | （17.02） | 4.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 22 | 23 | 0 | 4.5 | 0 | （1.52） | 0 | 18.5 | （17.02） | 4.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 23 | 23 | 0 | 1.5 | 0 | （1.52） | 0 | 21.5 | （20.02） | 2.0 | （2.52） | 2 | 25.0 + TT | 19.5 - TT | （17.5 - TT2） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 0 | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 0 | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 0 | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 27 | 23 | 0 | 2 | 0 | （1.52） | 0 | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 28 | 23 | 0 | 3 | 0 | （1.52） | 0 | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 29 | 23 | 0 | 3 | 0 | （1.52） | 0 | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 30 | 23 | 0 | 3 | 0 | （1.52） | 0 | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 31 | 23 | 0 | 3.5 | 0 | （1.52） | 0 | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 32 | 23 | 0 | 3.5 | 0 | （1.52） | 0 | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 33 | 23 | 0 | 3.5 | 0 | （1.52） | 0 | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 34 | 23 | 0 | 6.5 | 0 | （1.52） | 0 | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 35 | 23 | 0 | 6.5 | 0 | （1.52） | 0 | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | ΔPPowerBoost (dB) | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36 | 23 | 0 | 6.5 | 0 | （1.52） | 0 | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 37 | 23 | 0 | 0.5 | 0 | （1.52） | 0 | 22.5 | （212） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 38 | 23 | 0 | 0.5 | 0 | （1.52） | 0 | 22.5 | （212） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | (19.0 - TT2) |
| 39 | 23 | 0 | 0 | 0 | （1.52） | 0 | 23 | （21.52） | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT4 | （19.5 - TT2） |
| 406 | 23 | 0 | 0 | 0 | （1.52） | 1 | 24.0 | （22.52） | 2.0 |  | 2 | 26.0 + TT | 22.0 - TT4 | （20.5 - TT2） |
| 417 | 23 | 0 | 0 | 0 | （1.52） | 1 | 24.0 | （22.52） | 2.0 |  | 2 | 26.0 + TT | 22.0 - TT4 | （20.5 - TT2） |
| 426 | 23 | 0 | 0 | 0 | （1.52） | 1 | 24.0 | （22.52） | 2.0 |  | 2 | 26.0 + TT | 22.0 - TT4 | （20.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n12, n20, n25 n26, n41, n109 transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 4: For bands n91, n92, n93, n94 and n109 the maximum power reduction requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. NOTE 5: For 3 MHz channel bandwidth the Pi/2 BPSK edge allocation MPR is 1 dB. NOTE 6: Applicable for a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostPi2BPSK-r18 is set to 1. The reference power is increased by [ΔPPowerBoost - ΔPPowerClass]. NOTE 7: Applicable for a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostQPSK-r18 is set to 1. The reference power is increased by [ΔPPowerBoost - ΔPPowerClass]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-1a: UE Power Class test requirements Band 24 for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | 25.0 + TT | 20.0 - TT | （18.5 - TT2） |
| 6 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 7 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 8 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 9 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | 25.0 + TT | 20.0 - TT | （18.5 - TT2） |
| 10 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （17.5 - TT2） |
| 11 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （17.5 - TT2） |
| 12 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （17.5 - TT2） |
| 13 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （17.5 - TT2） |
| 14 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （16.0 - TT2） |
| 15 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （16.0 - TT2） |
| 16 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （16.0 - TT2） |
| 17 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | 25.0 + TT | 17.5 – TT | （15.5 - TT2） |
| 18 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | 25.0 + TT | 17.5 - TT | （15.5 - TT2） |
| 19 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | 25.0 + TT | 17.5 - TT | （15.5 - TT2） |
| 20 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 21 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 22 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 23 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | 25.0 + TT | 18.5 - TT | （17.0 - TT2） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 27 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （16.0 - TT2） |
| 28 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 – TT | （14.5 - TT2） |
| 29 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 – TT | （14.5 - TT2） |
| 30 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 31 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 32 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 33 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 34 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 35 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 36 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 37 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 38 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 39 | 23 | 0 | 0 | 0 | （1.52） | 23 | （21.52） | 2.0 |  | 3 | 25.0 + TT | 20.0 - TT | （18.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-2: UE Power Class test requirements (for Bands n28 with channel bandwidth other than 30MHz, n71, n105) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2.5 | 25.0 + TT | 20.5 - TT |
| 6 | 23 | 0 | 0.53 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 7 | 23 | 0 | 0.53 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 8 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 9 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2.5 | 25.0 + TT | 20.5 - TT |
| 10 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 11 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 12 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 13 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 14 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 15 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 16 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 17 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 18 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 19 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 20 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 21 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 22 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 23 | 23 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2.5 | 25.0 + TT | 19.0 - TT |
| 24 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 25 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 26 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 27 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 28 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 29 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 30 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 31 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 32 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 33 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 34 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 35 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 36 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 37 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 38 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 39 | 23 | 0 | 0 | 0 | 23 | 2.0 | 2.5 | 25.0 + TT | 20.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 3: For 3 MHz channel bandwidth the Pi/2 BPSK edge allocation MPR is 1 dB |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-2a: UE Power Class test requirements (for Band n28 with 30MHz channel bandwidth) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0.5 | 0 | 22.5 | 2 | 2.5 | 25.0 + TT | 20-TT |
| 6 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 7 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 8 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 9 | 23 | 0 | 0.5 | 0 | 22.5 | 2 | 2.5 | 25.0 + TT | 20-TT |
| 10 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 11 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 12 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 13 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 14 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 15 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 16 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 17 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 18 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 19 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 20 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 21 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 22 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 23 | 23 | 0 | 2 | 0 | 21 | 2 | 2.5 | 25.0 + TT | 18.5-TT |
| 24 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 25 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 26 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 27 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 28 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 29 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 30 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 31 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 32 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 33 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 34 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 35 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 36 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 37 | 23 | 0 | 1 | 0 | 22 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 38 | 23 | 0 | 1 | 0 | 22 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 39 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-3: UE Power Class test requirements (for Bands n48, n77, n78, n79) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 0.2 | 0 | 25.8 | 2.0 | 3 | 28.0 + TT | 22.8 - TT |
| 2 | 23 | -3 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28.0 + TT | 19.5 - TT |
| 3 | 23 | -3 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28.0 + TT | 19.5 - TT |
| 4 | 23 | -3 | 1.2 | 0 | 24.8 | 2.0 | 3 | 28.0 + TT | 21.8 - TT |
| 5 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 3 | 25.0 + TT | 20.0 - TT |
| 6 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 7 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 8 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 9 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 3 | 25.0 + TT | 20.0 - TT |
| 10 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 11 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 12 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 13 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 14 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 15 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 16 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 17 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 - TT |
| 18 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 - TT |
| 19 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 - TT |
| 20 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25.0 + TT | 14.5 - TT |
| 21 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25.0 + TT | 14.5 - TT |
| 22 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25.0 + TT | 14.5 - TT |
| 23 | 23 | 0 | 1.5 | 0 | 21.5 | 2.0 | 3 | 25.0 + TT | 18.5 - TT |
| 24 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 25 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 26 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 27 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 28 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 29 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 30 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 31 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16.0 - TT |
| 32 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16.0 - TT |
| 33 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16.0 - TT |
| 34 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25.0 + TT | 11.5 - TT |
| 35 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25.0 + TT | 11.5 - TT |
| 36 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25.0 + TT | 11.5 - TT |
| 37 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 38 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 39 | 23 | 0 | 0 | 0 | 23 | 2.0 | 3 | 25.0 + TT | 20.0 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-4: UE Power Class test requirements (for Bands n1, n3, n34, n39, n40, n41, n77, n78, n79) for Power Class 2 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | ΔPPowerBoost (dB) | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 0 | 0 | （1.52） | 0 | 26.0 | （24.52） | 2.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.5 - TT2） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 0 | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 0 | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0- TT2） |
| 4 | 26 | 0 | 0.5 | 0 | （1.52） | 0 | 25.5 | （24.02） | 2.0 |  | 3 | 28.0 + TT | 22.5 - TT | （21.0 - TT2） |
| 5 | 26 | 0 | 0 | 0 | （1.52） | 0 | 26.0 | （24.52） | 2.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.5 - TT2） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 0 | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 0 | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 8 | 26 | 0 | 1 | 0 | （1.52） | 0 | 25.0 | （23.52） | 2.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.5 - TT2） |
| 9 | 26 | 0 | 1 | 0 | （1.52） | 0 | 25.0 | （23.52） | 2.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.5 - TT2） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 0 | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 0 | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 12 | 26 | 0 | 2 | 0 | （1.52） | 0 | 24.0 | （22.52） | 2.0 |  | 3 | 28.0 + TT | 21.0 - TT | （19.5 - TT2） |
| 13 | 26 | 0 | 3.5 | 0 | （1.52） | 0 | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 14 | 26 | 0 | 3.5 | 0 | （1.52） | 0 | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0- TT2） |
| 15 | 26 | 0 | 2.5 | 0 | （1.52） | 0 | 23.5 | （22.02） | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 16 | 26 | 0 | 4.5 | 0 | （1.52） | 0 | 21.5 | （20.02） | 2.0 | （2.52） | 3 | 28.0 + TT | 18.5 - TT | （17.0 - TT2） |
| 17 | 26 | 0 | 4.5 | 0 | （1.52） | 0 | 21.5 | （20.02） | 2.0 | （2.52） | 3 | 28.0 + TT | 18.5 - TT | （17.0 - TT2） |
| 18 | 26 | 0 | 4.5 | 0 | （1.52） | 0 | 21.5 | （20.02） | 2.0 | （2.52） | 3 | 28.0 + TT | 18.5 - TT | （17.0 - TT2） |
| 19 | 26 | 0 | 1.5 | 0 | （1.52） | 0 | 24.5 | （23.02） | 2.0 |  | 3 | 28.0 + TT | 21.5 - TT | （20.0 - TT2） |
| 20 | 26 | 0 | 3.5 | 0 | （1.52） | 0 | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 21 | 26 | 0 | 3.5 | 0 | （1.52） | 0 | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 22 | 26 | 0 | 3 | 0 | （1.52） | 0 | 23.0 | （21.52） | 2.0 |  | 3 | 28.0 + TT | 20.0 - TT | （18.5 - TT2） |
| 23 | 26 | 0 | 2 | 0 | （1.52） | 0 | 24.0 | （22.52） | 2.0 |  | 3 | 28.0 + TT | 21.0 - TT | （19.5 - TT2） |
| 24 | 26 | 0 | 3.5 | 0 | （1.52） | 0 | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 25 | 26 | 0 | 3.5 | 0 | （1.52） | 0 | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 26 | 26 | 0 | 3 | 0 | （1.52） | 0 | 23.0 | （21.52） | 2.0 |  | 3 | 28.0 + TT | 20.0 - TT | （18.5 - TT2） |
| 27 | 26 | 0 | 3.5 | 0 | （1.52） | 0 | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 28 | 26 | 0 | 3.5 | 0 | （1.52） | 0 | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 29 | 26 | 0 | 3.5 | 0 | （1.52） | 0 | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 30 | 26 | 0 | 6.5 | 0 | （1.52） | 0 | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 28.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 31 | 26 | 0 | 6.5 | 0 | （1.52） | 0 | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 28.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 32 | 26 | 0 | 6.5 | 0 | （1.52） | 0 | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 28.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 334 | 26 | 0 | 0 | 0 | （1.52） | 0.5 | 26.5 | （25.02） | 2.0 |  | 3 | 28.5 + TT | 23.5 - TT | （22.0 - TT2） |
| 345 | 26 | 0 | 0 | 0 | （1.52） | 0.5 | 26.5 | （25.02） | 2.0 |  | 3 | 28.5 + TT | 23.5 - TT | （22.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n3 and n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 4: Applicable for a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostPi2BPSK-r18 is set to 1. The reference power is increased by [ΔPPowerBoost - ΔPPowerClass]. NOTE 5: Applicable for a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostQPSK-r18 is set to 1. The reference power is increased by [ΔPPowerBoost - ΔPPowerClass]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-4a: UE Power Class test requirements for Band n14 for Power Class 1 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 31 | 0 | 0 | 0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 — TT |
| 2 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 3 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 4 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 5 | 31 | 0 | 0 | 0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 — TT |
| 6 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 7 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 8 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 9 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 10 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 11 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 12 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 13 | 31 | 0 | 2.5 | 0 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 — TT |
| 14 | 31 | 0 | 2.5 | 0 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 — TT |
| 15 | 31 | 0 | 2.5 | 0 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 — TT |
| 16 | 31 | 0 | 4.5 | 0 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 — TT |
| 17 | 31 | 0 | 4.5 | 0 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 — TT |
| 18 | 31 | 0 | 4.5 | 0 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 — TT |
| 19 | 31 | 0 | 1.5 | 0 | 29.5 | 2.0 | 3 | 33.0 + TT | 26.5 — TT |
| 20 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 21 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 22 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 23 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 24 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 25 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 26 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 27 | 31 | 0 | 3.5 | 0 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 — TT |
| 28 | 31 | 0 | 3.5 | 0 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 — TT |
| 29 | 31 | 0 | 3.5 | 0 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 — TT |
| 30 | 31 | 0 | 6.5 | 0 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 — TT |
| 31 | 31 | 0 | 6.5 | 0 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 — TT |
| 32 | 31 | 0 | 6.5 | 0 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 — TT |
| 33 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 34 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 35 | 31 | 0 | 0 | 0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 — TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-4b: Void

Table 6.2.2.5-4c: UE Power Class test requirements for bands other than Band n14 (bands n7, n78) (bands n41, n100 and n101) for Power Class 1 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) |  | ΔTC,c (dB) | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | <50 MHz CBW | ≥50 MHz CBW |  | <50 MHz CBW | ≥50 MHz CBW |  |  |  | <50 MHz CBW | ≥50 MHz CBW |
| 1 | 31 | 0 | 0 | 0 | 0 | 31.0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 - TT | 28.0 - TT |
| 2 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 3 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 4 | 31 | 0 | 0.5 | 0.5 | 0 | 30.5 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 - TT | 27.5 - TT |
| 5 | 31 | 0 | 0 | 0 | 0 | 31.0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 - TT | 28.0 - TT |
| 6 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 7 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 8 | 31 | 0 | 1 | 1 | 0 | 30.0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 - TT | 27.0 - TT |
| 9 | 31 | 0 | 1 | 1 | 0 | 30.0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 - TT | 27.0 - TT |
| 10 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 11 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 12 | 31 | 0 | 2 | 2 | 0 | 29.0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 - TT | 26.0 - TT |
| 13 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 14 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 15 | 31 | 0 | 2.5 | 2.5 | 0 | 28.5 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 - TT | 25.5 - TT |
| 16 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 17 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 18 | 31 | 0 | 4.5 | 4.5 | 0 | 26.5 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 - TT | 23.5 - TT |
| 19 | 31 | 0 | 1.5 | 1.5 | 0 | 29.5 | 29.5 | 2.0 | 3 | 33.0 + TT | 26.5 - TT | 26.5 - TT |
| 20 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 21 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 22 | 31 | 0 | 3 | 3 | 0 | 28.0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 - TT | 25.0 - TT |
| 23 | 31 | 0 | 2 | 2 | 0 | 29.0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 - TT | 26.0 - TT |
| 24 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 25 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 26 | 31 | 0 | 3 | 3 | 0 | 28.0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 - TT | 25.0 - TT |
| 27 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 28 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 29 | 31 | 0 | 3.5 | 3.5 | 0 | 27.5 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 - TT | 24.5 - TT |
| 30 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 31 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 32 | 31 | 0 | 6.5 | 6.5 | 0 | 24.5 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 - TT | 21.5 - TT |
| 33 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 34 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 35 | 31 | 0 | 0 | 0 | 0 | 31.0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 - TT | 28.0 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-5: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2.2.5-6: UE Power Class test requirements (for Bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n30, n31, n34, n38, n39, n40, n41, n50, n51, n53, n54, n65, n66, n70, n72, n74, n85, n91, n92, n93, n94, n100, n101, n106, n109) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 23 | 0 | 1.5 | 1.5 | 0 | 1.52 | 20 | 18.52 | 2.5 | 42 | 2 | 25.0 + TT | 17.5 - TT | 14.5 - TT2 |
| 15 | 23 | 0 | 1.5 | 1 | 0 | 1.52 | 20.5 | 192 | 2.5 | 3.52 | 2 | 25.0 + TT | 18 - TT | 15.5 - TT2 |
| 26 | 23 | 0 | 3 | 1.5 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 2 | 25.0 + TT | 14.5 - TT | 12 - TT2 |
| 27 | 23 | 0 | 3 | 1 | 0 | 1.52 | 19 | 17.52 | 3.5 | 52 | 2 | 25.0 + TT | 15.5 - TT | 12.5 - TT2 |
| 34 | 23 | 0 | 2 | 1.5 | 0 | 1.52 | 19.5 | 182 | 3.5 | 42 | 2 | 25.0 + TT | 16 - TT | 14 - TT2 |
| 35 | 23 | 0 | 2 | 1 | 0 | 1.52 | 20 | 18.52 | 2.5 | 42 | 2 | 25.0 + TT | 17.5 - TT | 14.5 - TT2 |
| 46 | 23 | 0 | 3 | 1.5 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 2 | 25.0 + TT | 14.5 - TT | 12 - TT2 |
| 47 | 23 | 0 | 3 | 1 | 0 | 1.52 | 19 | 17.52 | 3.5 | 52 | 2 | 25.0 + TT | 15.5 - TT | 12.5 - TT2 |
| 56 | 23 | 0 | 3.5 | 1.5 | 0 | 1.52 | 18 | 16.52 | 4 | 52 | 2 | 25.0 + TT | 14 - TT | 11.5 - TT2 |
| 57 | 23 | 0 | 3.5 | 1 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 2 | 25.0 + TT | 14.5 - TT | 12 - TT2 |
| 66 | 23 | 0 | 6.5 | 1.5 | 0 | 1.52 | 15 | 13.52 | 5 | 52 | 2 | 25.0 + TT | 10 - TT | 8.5- TT2 |
| 67 | 23 | 0 | 6.5 | 1 | 0 | 1.52 | 15.5 | 142 | 5 | 52 | 2 | 25.0 + TT | 10.5 - TT | 9 - TT2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n12, n20, n25, n26, n41, n109 transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 4: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 5: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 6: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-6a: Void

Table 6.2.2.5-7: UE Power Class test requirements (for Bands n28 with channel bandwidth other than 30MHz, n71, n105) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | 23 | 0 | 1.5 | 1.5 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 14 | 23 | 0 | 1.5 | 1 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18 - TT |
| 25 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 26 | 23 | 0 | 3 | 1 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5 - TT |
| 33 | 23 | 0 | 2 | 1.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16 - TT |
| 34 | 23 | 0 | 2 | 1 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 45 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 46 | 23 | 0 | 3 | 1 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5 - TT |
| 55 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14 - TT |
| 56 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 65 | 23 | 0 | 6.5 | 1.5 | 0 | 15 | 5 | 2.5 | 25.0 + TT | 10 - TT |
| 66 | 23 | 0 | 6.5 | 1 | 0 | 15.5 | 5 | 2.5 | 25.0 + TT | 10.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 3: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 4: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 5: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 6: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-7a: Void

Table 6.2.2.5-7b: UE Power Class test requirements (for Band n28 with 30MHz channel bandwidth) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14 - TT |
| 24 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 43 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14 - TT |
| 44 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 53 | 23 | 0 | 4 | 1.5 | 0 | 17.5 | 5 | 2.5 | 25.0 + TT | 12.5 - TT |
| 54 | 23 | 0 | 4 | 1 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14 - TT |
| 63 | 23 | 0 | 7 | 1.5 | 0 | 14.5 | 5 | 2.5 | 25.0 + TT | 9.5 - TT |
| 64 | 23 | 0 | 7 | 1 | 0 | 15 | 5 | 2.5 | 25.0 + TT | 10 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 3: Applicable for CBW=30MHz when SCS=60kHz. NOTE 4: Applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz when SCS=30kHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-8: UE Power Class test requirements (for Bands n48, n77, n78, n79) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | 23 | 0 | 1.5 | 1.5 | 0 | 20 | 2.5 | 3 | 25.0 + TT | 17 - TT |
| 14 | 23 | 0 | 1.5 | 1 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 - TT |
| 25 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 4 | 3 | 25.0 + TT | 14.5 - TT |
| 26 | 23 | 0 | 3 | 1 | 0 | 19 | 3.5 | 3 | 25.0 + TT | 15.5 - TT |
| 33 | 23 | 0 | 2 | 1.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16 - TT |
| 34 | 23 | 0 | 2 | 1 | 0 | 20 | 2.5 | 3 | 25.0 + TT | 17 - TT |
| 45 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 4 | 3 | 25.0 + TT | 14.5 - TT |
| 46 | 23 | 0 | 3 | 1 | 0 | 19 | 3.5 | 3 | 25.0 + TT | 15.5 - TT |
| 55 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 4 | 3 | 25.0 + TT | 14 - TT |
| 56 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 4 | 3 | 25.0 + TT | 14.5 - TT |
| 65 | 23 | 0 | 6.5 | 1.5 | 0 | 15 | 5 | 3 | 25.0 + TT | 10 - TT |
| 66 | 23 | 0 | 6.5 | 1 | 0 | 15.5 | 5 | 3 | 25.0 + TT | 10.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 3: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 4: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 5: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 6: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-8a: Void

Table 6.2.2.5-9: UE Power Class test requirements (for Bands n1, n3, n34, n39, n40, n41, n77, n78, n79) for Power Class 2 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 26 | 0 | 1.5 | 1.5 | 0 | 1.52 | 23 | 21.52 | 2 | 22 | 3 | 28.0 + TT | 20 - TT | 18 - TT2 |
| 15 | 26 | 0 | 1.5 | 1 | 0 | 1.52 | 23.5 | 222 | 2 | 22 | 3 | 28.0 + TT | 20.5 - TT | 19.0 - TT2 |
| 26 | 26 | 0 | 3 | 1.5 | 0 | 1.52 | 21.5 | 202 | 2 | 2.52 | 3 | 28.0 + TT | 18.5 - TT | 17.0 - TT2 |
| 27 | 26 | 0 | 3 | 1 | 0 | 1.52 | 22 | 20.52 | 2 | 2.52 | 3 | 28.0 + TT | 19 - TT | 17.5 - TT2 |
| 34 | 26 | 0 | 2 | 1.5 | 0 | 1.52 | 22.5 | 212 | 2 | 22 | 3 | 28.0 + TT | 19.5 - TT | 18.0 - TT2 |
| 35 | 26 | 0 | 2 | 1 | 0 | 1.52 | 23 | 21.52 | 2 | 22 | 3 | 28.0 + TT | 20 - TT | 18.5 - TT2 |
| 46 | 26 | 0 | 3 | 1.5 | 0 | 1.52 | 21.5 | 202 | 2 | 2.52 | 3 | 28.0 + TT | 18.5 - TT | 17.0 - TT2 |
| 47 | 26 | 0 | 3 | 1 | 0 | 1.52 | 22 | 20.52 | 2 | 2.52 | 3 | 28.0 + TT | 19 - TT | 17.5 - TT2 |
| 56 | 26 | 0 | 3.5 | 1.5 | 0 | 1.52 | 21 | 19.52 | 2 | 3.52 | 3 | 28.0 + TT | 18 - TT | 16.0 - TT2 |
| 57 | 26 | 0 | 3.5 | 1 | 0 | 1.52 | 21.5 | 202 | 2 | 2.52 | 3 | 28.0 + TT | 18.5 - TT | 17.0 - TT2 |
| 66 | 26 | 0 | 6.5 | 1.5 | 0 | 1.52 | 18 | 16.52 | 4 | 52 | 3 | 28.0 + TT | 14 - TT | 11.5 - TT2 |
| 67 | 26 | 0 | 6.5 | 1 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 3 | 28.0 + TT | 14.5 - TT | 12.0 - TT2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n3 and n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 4: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 5: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 6: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 8: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-9a: Void

Table 6.2.2.5-10: UE Power Class test requirements (for Band n40 with 100MHz channel bandwidth) for Power Class 2 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 1 | 0 | 25.0 | 2.0 | 3 | 28.0 + TT | 22.0 - TT |
| 2 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 3 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 4 | 26 | 0 | 1.5 | 0 | 24.5 | 2.0 | 3 | 28.0 + TT | 21.5 - TT |
| 5 | 26 | 0 | 1 | 0 | 25.0 | 2.0 | 3 | 28.0 + TT | 22.0 - TT |
| 6 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 7 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 8 | 26 | 0 | 2 | 0 | 24.0 | 2.0 | 3 | 28.0 + TT | 21.0 - TT |
| 9 | 26 | 0 | 2 | 0 | 24.0 | 2.0 | 3 | 28.0 + TT | 21.0 - TT |
| 10 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 11 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 12 | 26 | 0 | 3 | 0 | 23.0 | 2.0 | 3 | 28.0 + TT | 20.0 - TT |
| 13 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 14 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 15 | 26 | 0 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28.0 + TT | 19.5 - TT |
| 16 | 26 | 0 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28.0 + TT | 17.5 - TT |
| 17 | 26 | 0 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28.0 + TT | 17.5 - TT |
| 18 | 26 | 0 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28.0 + TT | 17.5 - TT |
| 19 | 26 | 0 | 2.5 | 0 | 23.5 | 2.0 | 3 | 28.0 + TT | 20.5 - TT |
| 20 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 21 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 22 | 26 | 0 | 4 | 0 | 22.0 | 2.0 | 3 | 28.0 + TT | 19.0 - TT |
| 23 | 26 | 0 | 3 | 0 | 23.0 | 2.0 | 3 | 28.0 + TT | 20.0 - TT |
| 24 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 25 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 26 | 26 | 0 | 4 | 0 | 22.0 | 2.0 | 3 | 28.0 + TT | 19.0 - TT |
| 27 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 28 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 29 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 30 | 26 | 0 | 7.5 | 0 | 18.5 | 4.0 | 3 | 28.0 + TT | 14.5 - TT |
| 31 | 26 | 0 | 7.5 | 0 | 18.5 | 4.0 | 3 | 28.0 + TT | 14.5 - TT |
| 32 | 26 | 0 | 7.5 | 0 | 18.5 | 4.0 | 3 | 28.0 + TT | 14.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-11: UE Power Class test requirements (for Band n40 with 100MHz channel bandwidth) for Power Class 2 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 2.5 | 1.5 | 0 | 22 | 2 | 3 | 28.0 + TT | 19 - TT |
| 2 | 26 | 0 | 4 | 1.5 | 0 | 20.5 | 2.5 | 3 | 28.0 + TT | 17.5 - TT |
| 3 | 26 | 0 | 3 | 1.5 | 0 | 21.5 | 2 | 3 | 28.0 + TT | 18.5 - TT |
| 4 | 26 | 0 | 4 | 1.5 | 0 | 20.5 | 2.5 | 3 | 28.0 + TT | 17.5 - TT |
| 5 | 26 | 0 | 4.5 | 1.5 | 0 | 20 | 2.5 | 3 | 28.0 + TT | 17 - TT |
| 6 | 26 | 0 | 7.5 | 1.5 | 0 | 17 | 5 | 3 | 28.0 + TT | 12 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 3: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |  |  |  |  |  |  |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

### 6.2.3	UE additional maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- When IE powerBoostPi2BPSK-r18 or powerBoostQPSK-r18 is enabled, A-MPR, if larger than zero, FFS on the impacts to the test point selection and test requirement due to ΔPPowerBoost.

- Tests for network signalling values NS_40, NS_09 not complete.

- The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_07, NS_44, NS_46, NS_47, NS_48, and NS_49 to all types of NR Power Class 2 and 3 UE release 16 forward, and UE release 15 if the corresponding channel bandwidths are supported.

6.2.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2. Unless stated otherwise, Edge RB allocations get the same A-MPR as Outer RB allocations. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

6.2.3.2	Test applicability

The requirements of this test apply in test case 6.5.2.3 Additional Spectrum Emission mask for network signalling values NS_03, NS_03U, NS_04, NS_06, NS_07, NS_27 and NS_35 to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity and NR Power Class 1 UE release 15 and forward.

The requirements of this test apply in test case 6.5.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, NS_43U and NS_100 to all types of NR Power Class 3 UE release 15 and forward that don’t support Tx diversity.

The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_04, NS_05, NS_05U, NS_07, NS_12, NS_13, NS_14, NS_15, NS_17, NS_18, NS_21, NS_24, NS_27, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_43, NS_43U, NS_44, NS_45, NS_48, NS_49, NS_50 and NS_56 to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity.

NOTE:	Test execution is not necessary if 6.5.2.3, 6.5.2.4.2 and 6.5.3.3 are executed.

6.2.3.3	Minimum conformance requirements

6.2.3.3.1	General

Table 6.2.3.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. In case of a power class 3 UE, when IE powerBoostPi2BPSK is set to 1, power class 2 A-MPR values apply. When IE powerBoostPi2BPSK-r18 or powerBoostQPSK-r18 is enabled, A-MPR, if larger than zero, is increased by ΔPPowerBoost. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.3.1-1A.

For almost contiguous allocations in CP-OFDM waveforms in power class 3, the allowed A-MPR defined in clause 6.2.3 is increased by CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB, where CEIL{x, 0.5} means x rounding upwards to closest 0.5dB, NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs, and the parameter LCRB is replaced by NRB_alloc + NRB_gap in specifying the RB allocation regions.

Unless otherwise specified, pi/2 BPSK in following A-MPR tables refers to both variants of pi/2 BPSK referenced in 6.2.2 Table 6.2.2.3-1.

Table 6.2.3.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 (NOTE 8) | 3, 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_03 | 6.5.2.3.3.3 | n2, n25, n66, n70, n86 |  |  | Clause 6.2.3.3.7 |
| NS_03U | 6.5.2.3.3.3, 6.5.2.4.2.3 | n2, n25, n66, n86 (NOTE 1) |  |  | Clause 6.2.3.3.7 |
| NS_04 | 6.5.2.3.3.2, 6.5.3.3.3.1 | n41 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | Clause 6.2.3.3.2 |
| NS_05 | 6.5.3.3.3.4 | n1, n65, n84 (NOTE 1) | 5, 10, 15, 20 (Note 2) |  | Clause 6.2.3.3.4 (NOTE 7) |
| NS_05U | 6.5.3.3.3.4, 6.5.2.4.2.3 | n1, n65, n84 | 5, 10, 15, 20 |  | Clause 6.2.3.3.4 (NOTE 7) |
| NS_06 | 6.5.2.3.3.4 | n1213, n85 | 3, 5, 10, 15 |  | Clause 6.2.3.3.3212 |
|  |  | n14 | 5, 10 |  |  |
| NS_07 | 6.5.2.3.3.4, 6.5.3.3.5.26 | n13 | 5,10 | Table 6.2.3.3.29-1 | Table 6.2.3.3.29-2 |
| NS_10 |  | n20 | 15, 20 | Table 6.2.3.3.3-1 | Table 6.2.3.3.3-1 |
| NS_12 | 6.5.3.3.17 | n26 | 3, 5, 10 | Table 6.2.3.3.21-1 | Table 6.2.3.3.21-2 |
| NS_13 | 6.5.3.3.18 | n26 | 3, 5 | Table 6.2.3.3.22-1 | Table 6.2.3.3.22-2 |
| NS_14 | 6.5.3.3.19 | n26 | 10, 15, 20 | Table 6.2.3.3.23-1 | Table 6.2.3.3.23-2 |
| NS_15 | 6.5.3.3.20 | n26 | 3, 5, 10, 15, 20 | Table 6.2.3.3.24-1 | Table 6.2.3.3.24-2 |
| NS_17 | 6.5.3.3.3.2 | n28, n8313 | 3, 5, 10 | Table 5.3.2-1 | N/A |
| NS_18 | 6.5.3.3.3.3 | n28, n8313, n109 | 3, 5 |  | Table 6.2.3.3.13-1, A1 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.3.13-1, A2 |
|  |  |  | 25, 30 |  | Table 6.2.3.3.13-1, A3, A4, A5 |
| NS_21 | 6.5.2.3.3.9, 6.5.3.3.3.12 | n30 | 5,10 |  | Clause 6.2.3.3.14 |
| NS_24 | 6.5.3.3.3.13 | n65 (Note 4) | 5, 10, 15, 20 | Table 6.2.3.3.15-1 | Subclause 6.2.3.3.15 |
| NS_27 | 6.5.2.3.3.8, 6.5.3.3.3.14 | n48 | 5, 10, 15, 20, 30, 40 | Table 6.2.3.3.16-1 | Table 6.2.3.3.16-2 |
| NS_35 | 6.5.2.3.3.1 | n71 | 5, 10, 15, 20, 25, 30, 35 | Table 5.3.2-1 | Clause 6.2.3.3.3111 |
| NS_37 | 6.5.3.3.3.6 | n74 (Note 3) | 10, 15 | Table 6.2.3.3.8-1 | Table 6.2.3.3.8-1 |
| NS_38 | 6.5.3.3.3.7 | n74 | 5, 10, 15, 20 | Table 6.2.3.3.9-1 | Table 6.2.3.3.9-1 |
| NS_39 | 6.5.3.3.3.8 | n74 | 10, 15, 20 | Table 6.2.3.3.10-1 | Table 6.2.3.3.10-1 |
| NS_40 | 6.5.3.3.3.9 | n51 | 5 |  | Table 6.2.3.3.5-1 |
| NS_41 | 6.5.3.3.3.10 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.3.11-1 |
| NS_42 | 6.5.3.3.3.11 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.3.12-1 |
| NS_43 | 6.5.3.3.3.5 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.3.6 |
| NS_43U | 6.5.3.3.3.5, 6.5.2.4.2.3 | n8, n81 (NOTE 1) | 5, 10, 15 |  | Clause 6.2.3.3.6 |
| NS_44 | 6.5.3.3.24 | n38 | 25,30,40 | Table 6.2.3.3.20-1 | Table 6.2.3.3.20-1 |
| NS_45 | 6.5.3.3.3.21 | n53 | 5, 10 |  | Clause 6.2.3.3.25 |
| NS_46 | 6.5.3.3.3.25 | n7 | 10, 15, 20, 25, 30, 35, 40, 50 | Table 6.2.3.3.17-1 Table 6.2.3.3.17-311 Table 6.2.3.3.17-5 | Table 6.2.3.3.17-2 Table 6.2.3.3.17-411 Table 6.2.3.3.17-6 |
| NS_47 | 6.5.3.3.3.15 | n41 (Note 5) | 30 | Table 6.2.3.3.18-1 Table 6.2.3.3.18-3 | Table 6.2.3.3.18-2 Table 6.2.3.3.18-4 |
| NS_48 | 6.5.3.3.3.22 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.3.26-1, Table 6.2.3.3.26-3 | Table 6.2.3.3.26-2, Table 6.2.3.3.26-4 (NOTE 7) |
| NS_49 | 6.5.3.3.3.23 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.3.27-1 Table 6.2.3.3.27-3 | Table 6.2.3.3.27-2, Table 6.2.3.3.27-4 (NOTE 7) |
| NS_50 | 6.5.3.3.3.16 | n39 and n98 | 10, 15, 20, 25, 30, 35, 40 |  | Clause 6.2.3.3.19 |
| NS_55 | NOTE 6 | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | N/A |
| NS_56 | 6.5.3.3.3.27 | n24, n99 | 5, 10 | Table 6.2.3.3.30 | Clause 6.2.3.3.30 |
| NS_62 | 6.5.3.3.3.28 | n54 | 5 |  | N/A |
| NS_100 | 6.5.2.4.2.3 | n1, n2, n3, n5, n8, n25, n26, n66, n80, n81, n84, n86 (Note 1) |  |  | Table 6.2.3.3.1-2 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: No A-MPR is applied for 5 MHz BWChannel where the upper channel edge is ≥ 1930 MHz, 10 MHz BWChannel where the upper channel edge is ≥ 1950 MHz and 15 MHz BWChannel where the upper channel edge is ≥ 1955 MHz and 20 MHz BWChannel where the upper channel edge is ≥ 1970 MHz. NOTE 3: Applicable when the NR carrier is within 1447.9 – 1462.9 MHz. NOTE 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980 MHz. NOTE 5: Applicable when the NR carrier is within 2545 – 2575 MHz. NOTE 6: This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements. NOTE 7: The 1Tx architecture is assumed. For power class 2 UE indicating txDiversity-r16 [TS 38.306], the additional relaxation of [2] dB is applicable. NOTE 8: The NS_01 label with the field additionalPmax [6] absent is default for all NR bands. NOTE 9: Void. NOTE 10: FFS NOTE 11: Applicable only for power class 1 operation. NOTE 12: Applicable only for power class 1 operation on band n85. NOTE 13: 3 MHz channel bandwidth is not applicable. |  |  |  |  |  |

Table 6.2.3.3.1-1A: Mapping of Network Signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U | NS_48 | NS_49 |  |  |
| n2 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n3 | NS_01 | NS_100 |  |  |  |  |  |  |
| n5 | NS_01 | NS_100 |  |  |  |  |  |  |
| n7 | NS_01 | NS_46 |  |  |  |  |  |  |
| n8 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n12 | NS_01 | NS_06 |  |  |  |  |  |  |
| n13 | NS_01 | NS_06 | NS_07 |  |  |  |  |  |
| n14 | NS_01 | NS_06 |  |  |  |  |  |  |
| n20 | NS_01 | Void | NS_10 |  |  |  |  |  |
| n24 | NS_01 | NS_56 |  |  |  |  |  |  |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n26 | NS_01 | NS_100 | NS_12 | NS_13 | NS_14 | NS_15 |  |  |
| n28 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n30 | NS_01 | NS_21 |  |  |  |  |  |  |
| n31 | NS_01 |  |  |  |  |  |  |  |
| n34 | NS_01 |  |  |  |  |  |  |  |
| n38 | NS_01 | NS_44 |  |  |  |  |  |  |
| n39 | NS_01 | NS_50 |  |  |  |  |  |  |
| n40 | NS_01 |  |  |  |  |  |  |  |
| n41 | NS_01 | NS_04 | NS_47 |  |  |  |  |  |
| n48 | NS_01 | NS_27 |  |  |  |  |  |  |
| n50 | NS_01 | NS_41 | NS_42 |  |  |  |  |  |
| n51 | NS_01 | NS_40 |  |  |  |  |  |  |
| n53 | NS_01 | NS_45 |  |  |  |  |  |  |
| n54 | NS_01 | NS_62 |  |  |  |  |  |  |
| n65 | NS_01 | NS_24 | NS_100 | NS_05 | NS_05U | NS_51 |  |  |
| n66 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n70 | NS_01 | NS_03 |  |  |  |  |  |  |
| n71 | NS_01 | NS_35 |  |  |  |  |  |  |
| n72 | NS_01 |  |  |  |  |  |  |  |
| n74 | NS_01 | NS_37 | NS_38 | NS_39 |  |  |  |  |
| n77 | NS_01 | NS_55 |  |  |  |  |  |  |
| n78 | NS_01 |  |  |  |  |  |  |  |
| n79 | NS_01 |  |  |  |  |  |  |  |
| n80 | NS_01 | NS_100 |  |  |  |  |  |  |
| n81 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n82 | NS_01 | Void |  |  |  |  |  |  |
| n83 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n84 | NS_01 | NS_100 | NS_05 | NS_05U |  |  |  |  |
| n85 | NS_01 | NS_06 |  |  |  |  |  |  |
| n86 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n91 | NS_01 |  |  |  |  |  |  |  |
| n92 | NS_01 |  |  |  |  |  |  |  |
| n93 | NS_01 |  |  |  |  |  |  |  |
| n94 | NS_01 |  |  |  |  |  |  |  |
| n95 | NS_01 |  |  |  |  |  |  |  |
| n98 | NS_01 | NS_50 |  |  |  |  |  |  |
| n99 | NS_01 | NS_56 |  |  |  |  |  |  |
| n100 | NS_01 |  |  |  |  |  |  |  |
| n101 | NS_01 |  |  |  |  |  |  |  |
| n105 | NS_01 |  |  |  |  |  |  |  |
| n106 | NS_01 |  |  |  |  |  |  |  |
| n109 | NS_01 | NS_18 |  |  |  |  |  |  |
| NOTE 1: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [6]. |  |  |  |  |  |  |  |  |

Table 6.2.3.3.1-2: A-MPR for NS_100 (UTRA protection) (Power Class 3 and Power Class 2)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 4 |
|  | 16 QAM | ≤ 4 |
|  | 64 QAM | ≤ 4 |
|  | 256 QAM | ≤ 6.5 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.1.

6.2.3.3.2	A-MPR for NS_04

For NS_04, A-MPR is not added to MPR. Also, when NS_04 is signalled, MPR shall be set to zero in the PCMAX equations to avoid double-counting MPR.

For power Class 1.5, 2 and 3, allowed maximum power reduction is defined as A-MPR=max(MPR, A-MPR'),

Note that A-MPR'=0 dB means only MPR is applied,

where A-MPR' is defined as

if RBstart ≤ fstart,max,IMD3 / (12SCS) and LCRB ≤ AWmax,IMD3 / (12SCS) and FC - BWChannel/2 < FUL_low + offsetIMD3,
then

the A-MPR' is defined according to Table 6.2.3.3.2-2 PC3_A2 relative to 23 dBm for power class 3, PC2 A4 relative to 26 dBm for power class 2, and PC1.5_A6 relative to 29 dBm for power class 1.5,

else,

if RBstart ≤ LCRB/2 + start / (12SCS) and LCRB ≤ AWmax,regrowth / (12SCS) and FC - BWChannel/2 < FUL_low + offsetregrowth,
then

the A-MPR' is defined according to Table 6.2.3.3.2-2 PC3_A1 relative to 23 dBm for power class 3, PC2 A3 relative to 26 dBm for power class 2, and PC1.5_A5 relative to 29 dBm for power class 1.5,

else

A-MPR' = 0 dB and apply MPR.

With the parameters defined in Table 6.2.3.3.2-1.

Table 6.2.3.3.2-1: Parameters for region edges and frequency offsets (Power Class 1.5, 2 and 3)

| Parameter | Symbol | Value |  | Related condition |
| --- | --- | --- | --- | --- |
|  |  | CP-OFDM | DFT-s-OFDM |  |
| Max allocation start in IMD3 region | fstart,max,IMD3 | 0.33 BWChannel |  | RBstart ≤ fstart,max,IMD3 / (12SCS) |
| Max allocation BW in IMD3 region | AWmax,IMD3 | 4 MHz |  | LCRB ≤ AWmax,IMD3 / (12SCS) |
| Freq. offset required to avoid A-MPR in IMD3 region | offsetIMD3 | BWChannel – 6 MHz |  | FC - BWChannel/2 ≥ FUL_low + offsetIMD3 |
| Right edge of regrowth region | start | 0.08 BWChannel |  | RBstart ≤ LCRB/2 + start / (12SCS) |
| Max allocation BW in regrowth region | AWmax,regrowth | 100 MHz |  | LCRB ≤ Min(LCRB,Max, AWmax,regrowth / (12SCS)) |
| Freq. offset required to avoid A-MPR in regrowth region | offsetregrowth | Max (10 MHz, 0.25* BWChannel MHz) | Max (10 MHz, 0.45* BWChannel MHz) | FC - BWChannel/2 ≥ FUL_low + offsetregrowth |

Table 6.2.3.3.2-2: A-MPR' values Access (Power Class 1.5, 2 and 3)

| Modulation/Waveform |  | A-MPR' (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PC3_A1 | PC3_A2 | PC2_A3 | PC2_A4 | PC1.5_A51 | PC1.5_A61 |
| DFT-s-OFDM | Pi/2-BPSK | ≤ 3.5 | ≤ 3.5 | ≤ 3.5 | ≤ 5.5 | ≤ 5 | ≤ 7 |
|  | QPSK | ≤ 4 | ≤ 4 | ≤ 4.5 | ≤ 6 | ≤ 6 | ≤ 7.5 |
|  | 16-QAM | ≤ 4 | ≤ 4 | ≤ 5 | ≤ 6 | ≤ 6.5 | ≤ 7.5 |
|  | 64-QAM | ≤ 4 | ≤ 4.5 | ≤ 5 | ≤ 6.5 | ≤ 6.5 | ≤ 8 |
|  | 256-QAM | ≤ 4.5 | ≤ 6 | ≤ 6.5 | ≤ 8 | ≤ 8 | ≤ 9.5 |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 16-QAM | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 64-QAM | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 256-QAM | ≤ 6.5 | ≤ 8 | ≤ 7.5 | ≤ 10 | ≤ 9 | ≤ 11.5 |
| NOTE 1: PC1.5 assumes dual Tx. |  |  |  |  |  |  |  |

For Power Class 1, NS_04 A-MPR is defined as 
A-MPR = max(MPR, A-MPRregrowth, A-MPRIMD3, A-MPRCIM3, A-MPRedge).

A-MPRregrowth is obtained from Table 6.2.3.3.2-3 in terms of total guard bandwidth (TGBW). The TGBW is defined as the frequency distance between the RB allocation and the additional spurious emission limit defined in Table 6.5.3.3.3.1-1, i.e.,

TGBW=falloc,low-2496 MHz,

where

falloc,low=FC-BWChannel2+BWGB+RBstart∙12 SCS

is the lower edge frequency of the RB allocation, FC is the channel centre frequency, BWChannel is the channel bandwidth, and BWGB is the minimum guard bandwidth defined in Table 5.3.3-1.

Table 6.2.3.3.2-3: A-MPRregrowth for NS_04 (Power Class 1)

| TGBW range | A-MPRregrowth *(dB) |
| --- | --- |
| G0dB(BWalloc) TGBW | 0 |
| G1dB(BWalloc) TGBW < G0dB(BWalloc) | 1 |
| G2dB(BWalloc) TGBW < G1dB(BWalloc) | 2 |
| G3dB(BWalloc) TGBW < G2dB(BWalloc) | 3 |
| G4dB(BWalloc) TGBW < G3dB(BWalloc) | 4 |
| G5dB(BWalloc) TGBW < G4dB(BWalloc) | 5 |
| TGBW < G5dB(BWalloc) | 6 |

Each function GA(BWalloc) defines the required minimum total guard bandwidth for A-MPR value A and is defined as

GA(BWalloc)=max⁡(0,  C2BWalloc100MHz2+C1BWalloc100MHz+C0),

where BWalloc=LCRB∙12 SCS is the allocation bandwidth, and C2, C1, and C0 are obtained from Table 6.2.3.3.2-4 for each combination of waveform, modulation, and back-off value A.

Table 6.2.3.3.2-4: Polynomial coefficients for determining the required 
total guard bandwidth for each value of A-MPRregrowth (Power Class 1)

| Waveform | Modulation | Back-off value A [dB] / Polynomial coefficients C2, C1, C0 [MHz] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| DFT-s-OFDM | Pi/2-BPSK | -3, 40, 10 | -91, 98, 0 | -248, 143, -9 | -2148, 604, -40 | 0,0,0 | N/A | N/A |
|  | QPSK | -27, 71, 1 | -49, 73, 0 | -76, 76, -3 | -108, 62, -3 | 0,0,0 | N/A | N/A |
|  | 16-QAM | -34, 81, -1 (NOTE 1) | -57, 92, -5 | -69, 82, -5 | -119, 79, -5 | 0,0,0 | N/A | N/A |
|  | 64-QAM | -23, 74, -1 (NOTE 1) | -61, 91, -4 (NOTE 1) | -84, 92, -5 | -116, 80, -5 | 0,0,0 | N/A | N/A |
|  | 256-QAM | 0, 0, 0 (NOTE 2) | N/A | N/A | N/A | N/A | N/A | N/A |
| CP-OFDM | QPSK | -32, 81, -2 (NOTE 1) | -36, 77, -3 | -40, 70, -3 | -52, 67, -3 | -67, 60, -3 | -101, 56, -4 | 0,0,0 |
|  | 16-QAM | -31, 80, -2 (NOTE 1) | -32, 72, -2 (NOTE 1) | -36, 66, -2 | -48, 62, -2 | -66, 58, -2 | -108, 60, -4 | 0,0,0 |
|  | 64-QAM | -28, 78, -2 (NOTE 1) | -33, 73, -2 (NOTE 1) | -36, 66, -2 (NOTE 1) | -48, 62, -2 | -65, 58, -2 | -124, 67, -5 | 0,0,0 |
|  | 256-QAM | 0, 0, 0 (NOTE 2) | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 1: Back-off value A is less than floor(MPR), thus this polynomial is irrelevant. NOTE 2: For 256-QAM, always A-MPRregrowth = 0. |  |  |  |  |  |  |  |  |

For both OFDM and DFT-S-OFDM, A-MPRIMD3 = max0,  min3,  4-2.77 BWalloc1 MHz dB 
if  3falloc,low-2FC  2490.5 MHz; otherwise, A-MPRIMD3 = 0 dB.

For OFDM, A-MPRCIM3 = max0,  min11,  12-2 BWalloc1 MHz dB if  4 FC-3 falloc,high   2490.5 MHz; 
otherwise, A-MPRCIM3 = 0.

For DFT-S-OFDM, A-MPRCIM3 = 3 dB if BWalloc 1.08 MHz and 4 FC-3 falloc,high   2490.5 MHz; 
otherwise, A-MPRCIM3 = 0.

Here, falloc,high=falloc,low+BWalloc is the upper edge frequency of the RB allocation.

For both OFDM and DFT-S-OFDM, if RBstart=0, LCRB=1,  and FC-BWChannel2-2496 MHz<360 kHz, A-MPRedge is defined in Table 6.2.3.3.2-5. Otherwise, A-MPRedge = 0 dB.

Table 6.2.3.3.2-5: A-MPRedge for NS_04  (Power Class 1)

| Waveform | BWChannel [MHz] | BWalloc [MHz] | A-MPR [dB] |
| --- | --- | --- | --- |
| DFT-s-OFDM | < 40 | N/A | 0 |
|  | 40…90 | -9.8CBW100MHz2+24.1CBW100MHz-6.3 | 3.5 |
|  | 100 |  | 11 |
| OFDM | < 90 | N/A | 0 |
|  | 90 | 6.5 | 3.5 |
|  | 100 | 8 | 11 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.2.

6.2.3.3.3	A-MPR for NS_10

Table 6.2.3.3.3-1: A-MPR for NS_10

| Channel bandwidth [MHz] | Parameters | Region A |
| --- | --- | --- |
| 15 | RBstart | 0 – 10 |
|  | LCRB (RBs) | 1 –20 |
|  | A (dB) | ≤ 36 |
| 20 | RBstart | 0 – 15 |
|  | LCRB (RBs) | 1 – 20 |
|  | A (dB) | ≤ 66 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects Region A, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping which intersects Region A, notes 1 and 2 apply on a Tno_hopping basis. NOTE 4: For intra-subframe frequency hopping which intersect Region A, the larger A-MPR value may be applied for both slots in the subframe. For intra-slot frequency hopping which intersects Region A, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping which intersects Region A, the larger A-MPR value may be applied for the subslot. NOTE 5: The A-MPR for DFT-s-OFDM is the total backoff and is obtained by taking the maximum value of MPR + A-MPR specified in Table 6.2.3.3.1-1 and Table 6.2.4-1 in TS 36.521-1 [21] and A value specified in Table 6.2.3.3.3-1. NOTE 6: The A-MPR for CP-OFDM is the total backoff and is obtained by adding the A value in Table 6.2.3.3.3-1 to the corresponding MPR specified in Table 6.2.2.3-1. |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.3.

6.2.3.3.4	A-MPR for NS_05 and NS_05U

Table 6.2.3.3.4-1: A-MPR regions for NS_05 and NS_05U (Power Class 3)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 | 1922.5 ≤ FC < 1927.5 | <1.62MHz/12/SCS | >2.52MHz/12/SCS | A3 |  |  |  |  |  |  |
| 10 | 1925 ≤ FC < 1935 | <1.62MHz/12/SCS | >0 | A1 | >1.62MHz/12/SCS ≤3.60MHz/12/SCS | >5.4MHz/12/SCS | A7 | ≥7.2MHz/12/SCS | ≤1.08MHz/12/SCS | A2 |
| 10 | 1935 ≤ FC < 1945 |  | >4.5MHz/12/SCS | A4 |  |  |  |  |  |  |
| 15 | 1927.5 ≤ FC < 1932.5 | <3.24MHz/12/SCS | >0 | A1 | >3.24MHz/12/SCS ≤5.40MHz/12/SCS | >8.1MHz/12/SCS | A7 | ≥10.08MHz/12/SCS | ≤1.08MHz/12/SCS | A2 |
| 15 | 1932.5 ≤ FC < 1942.5 | <1.62MHz/12/SCS | >0 | A1 |  |  |  | ≥12.24 MHz/12/SCS | ≤1.08MHz/12/SCS | A2 |
| 15 | 1942.5 ≤ FC < 1947.5 |  | >7.2MHz/12/SCS | A5 |  |  |  |  |  |  |
| 20 | 1930 ≤ FC < 1950 | <4.86MHz/12/SCS | >0 | A1 | >4.86MHz/12/SCS ≤7.20MHz/12/SCS | >9.0MHz/12/SCS | A7 | ≥13.68 MHz/12/SCS | ≤1.08MHz/12/SCS | A2 |
| 20 | 1950 ≤ FC <1960 |  | >9.0MHz/12/SCS | A6 |  |  |  |  |  |  |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.3.4-2, 6.2.3.3.4-3 and 6.2.3.3.4-4. NOTE 2: Void. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.3.4-2: A-MPR for NS_05 and NS_05U (Power Class 3)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 10 | ≤ 5 | ≤ 4 | N/A |
|  | QPSK | ≤ 10 | ≤ 5 | ≤ 4.5 |  |
|  | 16 QAM | ≤ 10 | ≤ 5 | ≤ 6 |  |
|  | 64 QAM | ≤ 11 | ≤ 5 | ≤ 6 |  |
|  | 256 QAM | ≤ 13 | ≤ 5 | ≤ 7 |  |
| CP-OFDM | QPSK | ≤ 10 | ≤ 5 | ≤ 7.5 |  |
|  | 16 QAM | ≤ 10 | ≤ 5 | ≤ 7.5 |  |
|  | 64 QAM | ≤ 11 | ≤ 5 | ≤ 8 |  |
|  | 256 QAM | ≤ 13 |  | ≤ 10 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |

Table 6.2.3.3.4-3: A-MPR for NS_05 (Power Class 3)

| Modulation/Waveform |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |  | A7 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1 | N/A | ≤ 1 | N/A | ≤ 1 | N/A | ≤ 6 |
|  | QPSK |  |  | ≤ 1.5 |  | ≤ 1.5 |  | ≤ 6 |
|  | 16 QAM |  |  |  |  |  |  | ≤ 6 |
|  | 64 QAM |  |  |  |  |  |  | ≤ 6 |
|  | 256 QAM |  |  |  |  |  |  | ≤ 6 |
| CP-OFDM | QPSK | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 6 |
|  | 16 QAM | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 6 |
|  | 64 QAM |  |  |  |  |  |  | ≤ 6 |
|  | 256 QAM |  |  |  |  |  |  | ≤ 6 |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |  |

Table 6.2.3.3.4-4: A-MPR for modulation and waveform type for NS_05U (Power Class 3)

| Modulation/Waveform |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |  | A7 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 | N/A | ≤ 2 | N/A | ≤ 2 | N/A | ≤ 6 |
|  | QPSK | ≤ 2 |  | ≤ 2 |  | ≤ 2 |  | ≤ 6 |
|  | 16 QAM | ≤ 2.5 |  | ≤ 2.5 |  | ≤ 2.5 |  | ≤ 6 |
|  | 64 QAM | ≤ 3 |  | ≤ 3 |  | ≤ 3 |  | ≤ 6 |
|  | 256 QAM | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 6 |
| CP-OFDM | QPSK | ≤ 4 |  | ≤ 4 |  | ≤ 4 |  | ≤ 6 |
|  | 16 QAM | ≤ 4 |  | ≤ 4 |  | ≤ 4 |  | ≤ 6 |
|  | 64 QAM | ≤ 4 |  | ≤ 4 |  | ≤ 4 |  | ≤ 6 |
|  | 256 QAM | ≤ 6.5 |  | ≤ 6.5 |  | ≤ 6.5 |  | ≤ 6.5 |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |  |

Table 6.2.3.3.4-5: A-MPR regions for NS_05 and NS_05U (Power Class 2)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 | 1922.5 ≤ FC < 1927.5 | < 1.98 MHz/12/SCS | > 1.44 MHz/12/SCS | A3 | < 0.72 MHz/12/SCS | ≤ 1.44 MHz/12/SCS | A4 |  |  |  |
| 10 | 1925 ≤ FC < 1935 | < 1.98 MHz/12/SCS | > 0 | A1 | ≥ 1.98 MHz/12/SCS | > max(0, RBstart-1.08 MHz/12/SCS) | A7 | ≥ 7.2 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A1 |
|  |  |  |  |  | ≥ 1.98, ≤2.7 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A8 |  |  |  |
| 10 | 1935 ≤ FC < 1945 |  | > 3.96 MHz/12/SCS | A4 |  |  |  |  |  |  |
| 15 | 1927.5 ≤ FC < 1932.5 | < 3.6 MHz/12/SCS | > 0 | A1 | ≥ 3.6 MHz/12/SCS | > max(0, RBstart-1.8 MHz/12/SCS) | A7 | ≥ 10.08 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A1 |
|  |  |  |  |  | ≥ 3.6, ≤4.68 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A8 |  |  |  |
| 15 | 1932.5 ≤ FC < 1942.5 | < 1.98 MHz/12/SCS | > 0 | A1 | ≥ 1.98 MHz/12/SCS | > max(0, RBstart+1.08 MHz/12/SCS) | A7 | ≥ 12.24 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A1 |
| 15 | 1942.5 ≤ FC < 1947.5 |  | > 5.04 MHz/12/SCS | A5 |  |  |  |  |  |  |
| 20 | 1930 ≤ FC < 1950 | < 5.04 MHz/12/SCS | > 0 | A1 | ≥ 5.04 MHz/12/SCS | > max(0, RBstart-3.6 MHz/12/SCS) | A7 | ≥ 13.68 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A1 |
|  |  |  |  |  | ≥ 5.04, ≤6.66 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A8 |  |  |  |
| 20 | 1950 ≤ FC < 1960 |  | > 9.0 MHz/12/SCS | A6 |  |  |  |  |  |  |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.3.4-6 and 6.2.3.3.4-7. NOTE 2: Void |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.3.4-6: A-MPR for NS_05 and NS_05U (Power Class 2)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [13] | ≤ 6 | ≤ 6.5 | ≤ 2 |
|  | QPSK | ≤ [13] | ≤ 6 | ≤ 7 | ≤ 2 |
|  | 16 QAM | ≤ [13] | ≤ 6 | ≤ 8.5 | ≤ 2 |
|  | 64 QAM | ≤ [14] | ≤ 6 | ≤ 9 | ≤ 2 |
|  | 256 QAM | ≤ [15] | ≤ 6 | ≤ 9.5 |  |
| CP-OFDM | QPSK | ≤ [13] | ≤ 6 | ≤ 10 | ≤ 4 |
|  | 16 QAM | ≤ [13] | ≤ 6 | ≤ 10 | ≤ 4 |
|  | 64 QAM | ≤ [14] | ≤ 6 | ≤ 10 | ≤ 4 |
|  | 256 QAM | ≤ [16] |  | ≤ 10 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |

Table 6.2.3.3.4-7: A-MPR for NS_05 and NS_05U (Power Class 2)

| Modulation/Waveform |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |  | A7 (dB) | A8 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3 | N/A | ≤ 2 |  | ≤ 2 | N/A | ≤ 8 | ≤ 3.5 |
|  | QPSK | ≤ 3 |  | ≤ 2 |  | ≤ 2 |  | ≤ 8 | ≤ 3.5 |
|  | 16 QAM | ≤ 3.5 |  | ≤ 2.5 |  | ≤ 2 |  | ≤ 8 | ≤ 3.5 |
|  | 64 QAM | ≤ 3.5 |  | ≤ 2.5 |  |  |  | ≤ 8 | ≤ 3.5 |
|  | 256 QAM |  |  |  |  |  |  | ≤ 8 |  |
| CP-OFDM | QPSK | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 4 |  | ≤ 8.5 | ≤ 3.5 |
|  | 16 QAM | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 4 |  | ≤ 8.5 | ≤ 3.5 |
|  | 64 QAM | ≤ 5 |  | ≤ 5 | ≤ 5 | ≤ 4 |  | ≤ 8.5 |  |
|  | 256 QAM |  |  |  |  |  |  | ≤ 8.5 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.4.

6.2.3.3.5	A-MPR for NS_40

Table 6.2.3.3.5-1: A-MPR for NS_40

| Modulation/Waveform |  | A (dB) |  |
| --- | --- | --- | --- |
|  |  | Channel bandwidth: 5 MHz |  |
|  |  | Outer | Inner |
| DFT-s-OFDM | QPSK | ≤ 15.5 | ≤ 12 |
|  | 16 QAM | ≤ 14.5 | ≤ 11 |
|  | 64 QAM | ≤ 14.5 | ≤ 10 |
|  | 256 QAM | ≤ 12.5 | ≤ 7.5 |
| CP-OFDM | QPSK | ≤ 14.5 | ≤ 10 |
|  | 16 QAM | ≤ 14.5 | ≤ 10 |
|  | 64 QAM | ≤ 14 | ≤ 8 |
|  | 256 QAM | ≤ 11 | ≤ 5.5 |
| NOTE 1: The A-MPR for NS_40 is the total backoff and is obtained by taking the maximum value of MPR + A-MPR specified in Table 6.2.3.3.1-1 and Table 6.2.4-30a in TS 36.101 and MPR+A specified in Table 6.2.2.3-1 and Table 6.2.3.3.5-1. |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.5.

6.2.3.3.6	A-MPR for NS_43 and NS_43U

Table 6.2.3.3.6-1: A-MPR regions for NS_43 (Power class 3 and 2)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 MHz | 902.5 ≤ FC < 912.5 |  | > 2.7 MHz/12/SCS | A1 |  |  |  |
| 10 MHz | FC = 910 |  | > 7.2 MHz/12/SCS | A2 |  | > 5.4 MHz/12/SCS | A4 |
|  |  |  | > 8.1 MHz/12/SCS | A3 |  | > 7.2 MHz/12/SCS | A5 |
| 15 MHz | FC = 907.5 | < 1.8 MHz /12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | ≥ 7.2 MHz/12/SCS | A6 |
|  |  | > 12.24 MHz/12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | < 7.2 MHz/12/SCS | A7 |
|  |  |  |  |  | ≥ 6.12 MHz/12/SCS ≤ 7.2 MHz/12/SCS | > 5.4 MHz/12/SCS | A7 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.3.6-2. NOTE 2: 15 kHz SCS unless otherwise stated NOTE 3: Void |  |  |  |  |  |  |  |

Table 6.2.3.3.6-2: A-MPR for NS_43 (Power class 3)

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) |  | A3 (dB) |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer / Inner |
| DFT-s-OFDM | Pi/2 BPSK |  | N/A | ≤ 1.5 | N/A |  | N/A |  | N/A |  | N/A | ≤ 9 |
|  | QPSK | ≤ 2 |  |  |  |  |  | ≤ 2.5 |  |  |  | ≤ 9 |
|  | 16 QAM |  |  |  |  |  |  |  |  | ≤ 2.5 |  | ≤ 9 |
|  | 64 QAM |  |  |  |  | ≤ 2.5 |  |  |  |  |  | ≤ 9 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 9 |
| CP-OFDM | QPSK | ≤ 3.5 |  |  |  |  |  |  |  | ≤ 4 |  | ≤ 9 |
|  | 16 QAM | ≤ 3.5 |  |  |  |  |  |  |  | ≤ 4 |  | ≤ 9 |
|  | 64 QAM |  |  |  |  | ≤ 4 |  |  |  |  |  | ≤ 9 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 9 |

Table 6.2.3.3.6-2a: A-MPR for NS_43 (Power Class 2)

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) |  | A3 (dB) |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) | A7 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer / Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1 | N/A | ≤ 1.5 | N/A |  |  |  | N/A |  | N/A | ≤ 12 | ≤ 4 | 3 |
|  | QPSK | ≤ 3.5 |  |  |  |  |  | ≤ 3.5 |  | ≤ 5.5 |  | ≤ 12 | ≤ 4 | 3 |
|  | 16 QAM | ≤ 3.5 |  |  |  |  |  | ≤ 3.5 |  | ≤ 5.5 |  | ≤ 12 | ≤ 5 | 4 |
|  | 64 QAM | ≤ 3.5 |  | ≤ 4.5 |  | ≤ 5 |  |  |  |  |  | ≤ 12 | ≤ 5.5 | 5.5 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 12 | ≤ 6.5 | 5.5 |
| CP-OFDM | QPSK | ≤ 4 |  |  |  |  |  | ≤ 4 |  | ≤ 6 |  | ≤ 12 | ≤ 6 | 4.5 |
|  | 16 QAM | ≤ 4 |  |  |  |  |  | ≤ 4.5 |  | ≤ 6 |  | ≤ 12 | ≤ 6 | 5 |
|  | 64 QAM | ≤ 4 |  | ≤ 5 |  | ≤ 6.5 |  |  |  |  |  | ≤ 12 | ≤ 6.5 | 6 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 12 | ≤ 7 | 7 |

Table 6.2.3.3.6-3: Void

When NS_43U is signalled for 5 and 10 MHz channel bandwidths A-MPR is defined in Table 6.2.3.3.1-2 except for DFT-s-OFDM QPSK when LCRB > 5.4 MHz/12/SCS the A-MPR is 2.5 dB. For 15 MHz channel bandwidth Table 6.2.3.3.6-4 applies.

Table 6.2.3.3.6-4: A-MPR for NS_43U

| Modulation/Waveform |  | 15 MHz |
| --- | --- | --- |
|  |  | Outer / Inner (dB) |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 9 |
|  | QPSK | ≤ 9 |
|  | 16 QAM | ≤ 9 |
|  | 64 QAM | ≤ 9 |
|  | 256 QAM | ≤ 9 |
| CP-OFDM | QPSK | ≤ 9 |
|  | 16 QAM | ≤ 9 |
|  | 64 QAM | ≤ 9 |
|  | 256 QAM | ≤ 9 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.6.

6.2.3.3.7	A-MPR for NS_03 and NS_03U

Table 6.2.3.3.7-1: A-MPR for NS_03

| Modulation/Waveform |  | Outer (dB) | Inner (dB) |
| --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | ≤ 1.5 | N/A |
|  | QPSK | ≤ 2 |  |
|  | 16 QAM | ≤ 3 |  |
|  | 64 QAM | ≤ 3.5 |  |
|  | 256 QAM | ≤ 5.5 |  |
| CP-OFDM | QPSK | ≤ 4 |  |
|  | 16 QAM | ≤ 4 |  |
|  | 64 QAM | ≤ 4.5 |  |
|  | 256 QAM | ≤ 7.5 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |

In case UE operates in a band where NS_03U applies and it receives additionalSpectrumEmission value of 3 then A-MPR values specified in Table 6.2.3.3.7-1 apply with an exception that DFT-s-OFDM Pi/2 BPSK A-MPR is 2 dB.

6.2.3.3.8	A-MPR for NS_37

Table 6.2.3.3.8-1: A-MPR regions for B11/B21 protection (NS_37) (1447.9 - 1462.9 MHz)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A (Outer/Inner) |  |  | Region B (Outer/Inner) |  |  | Region C (Outer/Inner) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 10 | 1452.9 < FC ≤1457.9 | ≥0 | > 7.2 MHz/12/SCS | ≤ A1 | N/A | N/A | N/A | N/A | N/A | N/A |
| 15 | FC =1455.4 | ≥0 | > 9.9 MHz/12/SCS | ≤ A1 | < 0.54 MHz/12/SCS | < 1.08 MHz/12/SCS | ≤ A2 | > 13.86 MHz/12/SCS | < 1.08 MHz/12/SCS | ≤ A2 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.3.8-2. NOTE 2: Void NOTE 3: Void NOTE 4: No A-MPR for SCS = 60 kHz for region B and C only. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.3.8-2: A-MPR for NS_37

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) |
| --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1 | N/A | ≤ 3 |
|  | QPSK | ≤ 1.5 |  | ≤ 3 |
|  | 16 QAM | ≤ 2.5 |  | ≤ 3 |
|  | 64 QAM | ≤ 3 |  | ≤ 3 |
|  | 256 QAM |  |  |  |
| CP-OFDM | QPSK | ≤ 3.5 |  | ≤ 3 |
|  | 16 QAM | ≤ 3.5 |  | ≤ 3 |
|  | 64 QAM |  |  |  |
|  | 256 QAM |  |  |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.8.

6.2.3.3.9	A-MPR for NS_38

Table 6.2.3.3.9-1: A-MPR for EESS (NS_38) Protection (1430 – 1470 MHz)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A Outer/Inner |  |  | Region B Outer/Inner |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR (dB) | RBstart+LCRB | A-MPR (dB) |
| 5 | 1432.5 ≤ FC < 14347.5 | ≤ -1.8 MHz/12/SCS + LCRB /2 | > 3.6 MHz/12/SCS | ≤ 7 | ≤ 2.16 MHz/12/SCS | ≤ 5.5 |
| 10 | 1435 ≤ FC < 1442 | ≤ -1.8 MHz/12/SCS + LCRB/2 | > 3.6 MHz/12/SCS | ≤ 12 | ≤ 2.16 MHz/12/SCS | ≤ 9 |
| 15 | 1437.5 ≤ FC < 1447.5 | ≤ -1.8 MHz/12/SCS + LCRB/2 | > 3.6 MHz/12/SCS | ≤ 13 | ≤ 3.6 MHz/12/SCS | ≤ 10 |
| 20 | 1440 ≤ FC < 1450 | ≤ -1.8 MHz/12/SCS + LCRB/2 | > 3.6 MHz/12/SCS | ≤ 13 | ≤ 5.4 MHz/12/SCS | ≤ 10 |
| NOTE 1 - 4: Void |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.9.

6.2.3.3.10	A-MPR for NS_39

Table 6.2.3.3.10-1: A-MPR for own RX (NS_39) Protection (1440 – 1470 MHz)

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A (Outer/Inner) |  |
| --- | --- | --- | --- |
|  |  | RBstart+LCRB | A-MPR (dB) |
| 10 | 1460 < FC ≤1465 | > 7.9 MHz/12/SCS | ≤ 6 |
| 15 | 1452.5 < FC ≤ 1462.5 | > 11.2 MHz/12/SCS | ≤ 6 |
| 20 | 1450 < FC ≤ 1460 | > 14.4 MHz/12/SCS | ≤ 6 |
| NOTE 1 - 4: Void |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.10.

6.2.3.3.11	A-MPR for NS_41

Table 6.2.3.3.11-1: A-MPR for NS_41

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A (Outer/Inner) |  |  | Region B (Outer/Inner) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR (dB) | RBstart+LCRB | A-MPR (dB) |
| 5 | - | - | - | - | - | - |
| 10 | 1437 ≤ FC < 1442 | ≤ -4.5 MHz/12/SCS + LCRB | > 4.5 MHz/12/SCS | ≤ 9 | < 1.8MHz/12/SCS | ≤ 9 |
| 15 | 1439.5 ≤ FC < 1447.5 | ≤ -5.4 MHz/12/SCS + LCRB | > 5.4 MHz/12/SCS | ≤ 11 | < 3.42 MHz/12/SCS | ≤ 9 |
| 20 | 1442 ≤ FC < 1450 | ≤ -5.4 MHz/12/SCS + LCRB | > 5.4 MHz/12/SCS | ≤ 12 | < 5.04 MHz/12/SCS | ≤ 9 |
| 40 | 1452 ≤ FC < 1497 | ≤ -7.2 MHz/12/SCS + LCRB | > 7.2 MHz/12/SCS | ≤ 13.5 | < 11.7 MHz/12/SCS | ≤ 13.5 |
| 50 | 1457 ≤ FC < 1492 | ≤ -7.2 MHz/12/SCS + LCRB | > 7.2 MHz/12/SCS | ≤ 13.5 | < 15.12 MHz/12/SCS | ≤ 13.5 |
| 60 | 1462 ≤ FC < 1487 | ≤ -7.2 MHz/12/SCS + LCRB | > 7.2 MHz/12/SCS | ≤ 13.5 | < 18.72 MHz/12/SCS | ≤ 13.5 |
| NOTE 1 - 4: Void |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.11.

6.2.3.3.12	A-MPR for NS_42

Table 6.2.3.3.12-1: A-MPR for NS_42

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  | Region B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart+LCRB | A-MPR Outer/Inner (dB) | RBstart | RBstart+LCRB | A-MPR Inner (dB) | A-MPR Outer (dB) |
| 5 | 1512 ≤ FC ≤ 1514.5 | >3.1 MHz / 12 / SCS | ≤ 7 | <0.9 MHz / 12 / SCS | ≤ 3.1 MHz / 12 / SCS | ≤ 1.5 | ≤ 4 |
| 10 | 1497 ≤ FC ≤ 1512 | >6.2 MHz / 12 / SCS | ≤ 8 | <0.9 MHz / 12 / SCS | ≤ 6.2 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 15 | 1502 FC ≤ 1509.5 | >9.3 MHz / 12 / SCS | ≤ 8 | <3.06 MHz / 12 / SCS | ≤ 9.3 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 20 | 1497 ≤ FC ≤ 1507 | >12.4 MHz / 12 / SCS | ≤ 8 | <4.5 MHz / 12 / SCS | ≤ 12.4 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 40 | 1477 ≤ FC ≤ 1497 | >24.8 MHz / 12 / SCS | ≤ 8 | <5.4 MHz / 12 / SCS | ≤ 24.8 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 50 | 1467 ≤ FC ≤ 1492 | >31 MHz / 12 / SCS | ≤ 8 | <7.2 MHz / 12 / SCS | ≤ 31 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 60 | 1462 ≤ FC ≤ 1487 | >37.2 MHz / 12 / SCS | ≤ 8 | <7.2 MHz / 12 / SCS | ≤ 37.2 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| NOTE 1 - 5: Void |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.12.

6.2.3.3.13	A-MPR for NS_18

Table 6.2.3.3.13-0: Band n28, n83 and n109 25, 30MHz A-MPR regions for NS_18

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | 703~733 | >(LCRB*12*SCS)/2+3.6 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+3.6 | ≥5.4 | A4 |
|  |  | ≤6.3 | <5.4 | A5 |
| 30 | 703~733 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |

Table 6.2.3.3.13-1: A-MPR for NS_18

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 | N/A | ≤ 5 | 3 | 8 | 3 |
|  | QPSK | ≤ 2 |  | ≤ 5 | 3 | 8 | 3 |
|  | 16 QAM | ≤ 3 |  | ≤ 6 | 3 | 8 | 3 |
|  | 64 QAM | ≤ 4 |  | ≤ 7 | 3 | 8 | 4.5 |
|  | 256 QAM | ≤ 6 |  | ≤ 9 | 3 | 8 | 5.5 |
| CP-OFDM | QPSK | ≤ 5 |  | ≤ 6.5 | 4.5 | 9.5 | 5 |
|  | 16 QAM | ≤ 5 |  | ≤ 7 | 4.5 | 9.5 | 5 |
|  | 64 QAM | ≤ 5.5 |  | ≤ 8.5 | 4.5 | 9.5 | 5.5 |
|  | 256 QAM | ≤ 8.5 |  | ≤ 11.5 | 4.5 | 9.5 | 7.5 |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.13.

6.2.3.3.14	A-MPR for NS_21

Table 6.2.3.3.14-1: A-MPR for "NS_21"

| Channel Bandwidth (MHz) | Modulation/Waveform |  | Region A1a RBstart ≤ 1.44MHz/12/SCS LCRB ≤ [0.54] MHz/12/SCS | Region A1b RBstart ≤ 1.44MHz/12/SCS LCRB > [0.54] MHz/12/SCS LCRB ≤ 2.16MHz/12/SCS | Region A2 LCRB > 5.4MHz/12/SCS | Region A3b RBend ≥ 7.74MHz/12/SCS LCRB > [0.54] MHz/12/SCS LCRB ≤ 2.16MHz/12/SCS | Region A3a RBend ≥ 7.74MHz/12/SCS LCRB ≤ [0.54] MHz/12/SCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer/Inner |  | Outer | Outer/Inner |  |
| 10 | DFT-s-OFDM | PI/2 BPSK | 6 | 3 | 4 | 3 | 6 |
|  |  | QPSK | 6 | 3 | 4 | 3 | 6 |
|  |  | 16 QAM | 6 | 3 | 4 | 3 | 6 |
|  |  | 64 QAM | 6 | 3 | 4 | 3 | 6 |
|  |  | 256 QAM | 6 | 3 | 4 | 3 | 6 |
|  | CP-OFDM | QPSK | 6 | 4 | 5.5 | 4 | 6 |
|  |  | 16 QAM | 6 | 4 | 5.5 | 4 | 6 |
|  |  | 64 QAM | 6 | 4 | 5.5 | 4 | 6 |
|  |  | 256 QAM | 6 | 4 | 5.5 | 4 | 6 |

Table 6.2.3.14-2: A-MPR for "NS_21" (applicable to UEs Release 17 and forward indicating modifiedMPR-Behavior)

| Channel Bandwidth (MHz) | Modulation/Waveform |  | Region A1a RBstart ≤ 0.36MHz/12/SCS LCRB ≤ 0.54 MHz/12/SCS | Region A1b RBstart ≤ 0.36MHz/12/SCS LCRB > 0.54 MHz/12/SCS LCRB ≤ 2.52MHz/12/SCS | Region A2 LCRB > 2.52MHz/12/SCS | Region A3b RBend ≥ 3.96MHz/12/SCS LCRB > 0.54 MHz/12/SCS LCRB ≤ 2.52MHz/12/SCS | Region A3a RBend ≥ 3.96MHz/12/SCS LCRB ≤ 0.54 MHz/12/SCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer/Inner |  | Outer | Outer/Inner |  |
| 5 | DFT-s-OFDM | PI/2 BPSK | ≤ 4.0 | ≤ 2.0 | ≤ 1.5 | 2.0 | 4.0 |
|  |  | QPSK | ≤ 4.5 | ≤ 2.5 | ≤ 2.0 | ≤ 2.5 | ≤ 4.5 |
|  |  | 16 QAM | ≤ 4.5 | ≤ 2.5 | ≤ 2.5 | ≤ 2.5 | ≤ 4.5 |
|  |  | 64 QAM | ≤ 4.5 | ≤ 2.5 | ≤ 2.5 | ≤ 2.5 | ≤ 4.5 |
|  |  | 256 QAM | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 |
|  | CP-OFDM | QPSK | ≤ 4.5 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 4.5 |
|  |  | 16 QAM | ≤ 4.5 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 4.5 |
|  |  | 64 QAM | ≤ 4.5 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 4.5 |
|  |  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.14.

6.2.3.3.15	A-MPR for NS_24

Table 6.2.3.3.15-1: A-MPR for NS_24

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 5MHz | Fc=1992.5 |  | >3.24 | A7 |  |  |  |  |  |  |
| 5MHz | Fc=1997.5 |  | >3.24 | A4 |  |  |  |  |  |  |
| 5MHz | Fc=2002.5 |  | >2.16 | A1 | >3.78 | ≤1.98 | A2 | ≤3.6 | ≤1.98 | A3 |
| 10MHz | Fc=1985 | >5.4 |  | A4 |  |  |  |  |  |  |
| 10MHz | Fc=1995 |  | >4.5 | A1 | >7.56 | ≤4.32 | A2 | ≤7.38 | ≤4.32 | A3 |
| 10MHz | Fc=2000 | >6.84 |  | A5 | <2.88 |  | A5 | ≥3.06 ≤6.66 | >1.44 | A6 |
| 15MHz | Fc=1987.5 |  | >7.02 | A1 | >11.52 | ≤6.84 | A2 | ≤11.34 | ≤6.84 | A3 |
| 15MHz | Fc=1997.5 | >9.36 |  | A5 | <3.6 |  | A5 | ≥3.78 ≤9.18 | >1.44 | A6 |
| 20MHz | Fc=1990 | >13.5 |  | A5 | <4.5 |  | A5 | ≥4.68 ≤13.32 | >2.16 | A6 |
| 20MHz | Fc=1995 | >12.6 |  | A5 | <5.4 |  | A5 | ≥5.58 ≤12.42 | >1.44 | A6 |
| NOTE 1: The A-MPR values are listed in Table 6.2.3.3.15-2. NOTE 2: For any undefined region, MPR applies |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.3.15-2: A-MPR for modulation and waveform type

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 18 | ≤ 10 | ≤ 3.5 |
| DFT-s-OFDM QPSK | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 18 | ≤ 10 | ≤ 3.5 |
| DFT-s-OFDM 16 QAM | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 18 | ≤ 10 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 10 | ≤ 3.5 |
| DFT-s-OFDM 256 QAM | ≤ 11 | ≤ 5 |  | ≤ 8.5 | ≤ 20 | ≤ 10 |  |
| CP-OFDM QPSK | ≤ 13 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 12 | ≤ 5.5 |
| CP-OFDM 16 QAM | ≤ 13 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 12 | ≤ 5.5 |
| CP-OFDM 64 QAM | ≤ 13 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 12 | ≤ 5.5 |
| CP-OFDM 256 QAM | ≤ 13 | ≤ 6.5 |  | ≤ 8.5 | ≤ 20 | ≤ 12 |  |
| NOTE 1: The backoff applied is max(MPR, A-MPR) where MPR is defined in Table 6.2.2.3-1 NOTE 2: Outer and inner allocations are defined in clause 6.1 |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.15.

6.2.3.3.16	A-MPR for NS_27

Table 6.2.3.3.16-1: A-MPR for NS_27

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  |  | Region B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS | RBend*12*SCS | LCRB*12*SCS | A-MPR | LCRB*12*SCS | A-MPR |
| 15 MHz | 3557.5 ≤ FC < 3562.5 | <1.8 MHz |  |  | A3 | ≥10.8 MHz | A3 |
|  | 3687.5 < FC ≤ 3692.5 | >11.52 MHz |  |  |  |  |  |
| 15 MHz | 3562.5 ≤ FC < 3567.5 | ≤1.08 MHz |  | <1.44 MHz | A4 | ≥11.52 MHz | 2 |
|  | 3682.5 < FC ≤ 3687.5 |  | ≥13.22 MHz |  |  |  |  |
| 20 MHz | 3560 ≤ FC < 3570 | <3.6 MHz |  |  | A5 | ≥10.8 MHz | A5 |
|  | 3680 < FC ≤ 3690 | >12.96 MHz |  |  |  |  |  |
| 20 MHz | 3570 ≤ FC < 3580 | ≤2.16 MHz |  | <1.44 MHz | A6 | ≥14.4 MHz | 2 |
|  | 3670 < FC ≤ 3680 |  | ≥16.92 |  |  |  |  |
| 30 MHz | 3565 ≤ FC < 3585 | < 7.38MHz |  |  | A7 |  |  |
|  |  | ≥ 7.38MHz ≤24.48MHz |  | ≥ 15.3MHz | A2 |  |  |
|  |  |  |  | < 15.3 MHz | A1 |  |  |
|  |  | ≥24.48MHz |  | < 2.7 MHz | A7 |  |  |
|  | 3665 < FC ≤ 3685 |  | > 19.44 MHz |  | A7 |  |  |
|  |  |  | ≤19.44MHz ≥3.24 MHz | ≥ 15.3 MHz | A2 |  |  |
|  |  |  |  | < 15.3 MHz | A1 |  |  |
|  |  |  | <3.24 MHz | < 2.7MHz | A7 |  |  |
|  | 3585 ≤ FC ≤ 3665 | ≤[3.96] MHz |  | < 1.44MHz | A8 | ≥19.44 MHz | 4 |
|  |  |  | ≥24.48MHz |  | A8 |  |  |
| 40 MHz | 3570 ≤ FC < 3600 | <11.34 MHz |  |  | A7 |  |  |
|  |  | ≥11.34 MHz, ≤31.0 MHz |  | ≥18 MHz | A2 |  |  |
|  |  |  |  | <18 MHz | A1 |  |  |
|  |  | >31.0 MHz |  | <3.6 MHz | A7 |  |  |
|  | 3650 < FC ≤ 3680 |  | >24.48 MHz |  | A7 |  |  |
|  |  |  | ≤24.48 MHz, ≥6.48 MHz | ≥18 MHz | A2 |  |  |
|  |  |  |  | <18 MHz | A1 |  |  |
|  |  |  | <6.48 MHz | <3.6 MHz | A7 |  |  |
| 40 MHz | 3600 ≤ FC ≤ 3650 | ≤6.12 MHz |  | <1.44 MHz | A8 | >20 MHz | 4.5 |
|  |  |  | ≥ 32.76 |  |  |  |  |

Table 6.2.3.3.16-2: A-MPR for modulation and waveform type

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 4.5 | 6 | 4 | 4 | 4 | 4 | 10.5 | 4 |
|  | QPSK | 4.5 | 6 | 4 | 4 | 4 | 4 | 10.5 | 4 |
|  | 16 QAM | 4.5 | 6 | 5 | 4 | 5 | 4 | 11 | 4 |
|  | 64 QAM | 4.5 | 6 | 5 | 4 | 5 | 4 | 11 | 4 |
|  | 256 QAM |  | 6 |  |  |  |  | 11 |  |
| CP-OFDM | QPSK | 5.5 | 7 | 6 | 4 | 6 | 4 | 11.5 | 4 |
|  | 16 QAM | 5.5 | 7 | 6 | 4 | 6 | 4 | 11.5 | 4 |
|  | 64 QAM | 5.5 | 7 | 6 | 4 | 6 | 4 | 11.5 | 4 |
|  | 256 QAM |  | 7 |  |  |  |  | 11.5 |  |
| NOTE 1: The backoff applied is max (MPR, A-MPR) where MPR is defined in Table 6.2.2.3-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2.3 |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.16.

6.2.3.3.17	A-MPR for NS_46

Table 6.2.3.3.17-1: A-MPR regions for NS_46 (Power class 3)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 2534.5 ≤ FC ≤ 2557.5 |  | Note 1 | A3 |
| 30 MHz | 2515 ≤ FC ≤ 2555 | ≥0, <1.44 | >0 | A4 |
|  |  | ≥1.44, <13.5 | >max (0, 12*SCS*RBend -1.8) | A5 |
|  |  | ≥13.5, <19.8 | >11.52 | A6 |
|  |  | ≥19.8, <25.92 | >6.3 | A7 |
|  |  | ≥25.92 | >0 | A8 |
| 40 MHz | 2520 ≤ FC ≤ 2550 | ≥0, <4.14 | >0 | A4 |
|  |  | ≥4.14, <18 | >max (0, 12*SCS*RBend - 4.5) | A5 |
|  |  | ≥18, <25.74 | >13.5 | A6 |
|  |  | ≥25.74, <32.4 | >12.6 | A7 |
|  |  | ≥32.4 | >0 | A8 |
| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥0, <9 | >0 | A4 |
|  |  | ≥9, <21.6 | >max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥21.6, <31.5 | >18 | A6 |
|  |  | ≥31.5, <39.6 | >16.2 | A7 |
|  |  | ≥39.6 | >0 | A8 |
| NOTE 1: > 9.72 MHz for DFT-s-OFDM, > 16.02 MHz for CP-OFDM. |  |  |  |  |

Table 6.2.3.3.17-2: A-MPR for NS_46 (Power class 3)

| Modulation/Waveform |  | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | QPSK | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | 16 QAM | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | 64 QAM | 4.5 | 5 |  | 3.5 | 6 | 10 |
|  | 256 QAM |  |  |  |  | 6 | 10 |
| CP-OFDM | QPSK | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 16 QAM | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 64 QAM | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 256 QAM | 6 |  |  |  | 7 | 11 |

Table 6.2.3.3.17-3: A-MPR regions for NS_46 (Power class 1)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 15 MHz | Fc > 2559.5 | ≥ 11.8 | ≥ -1.8*12*SCS*RBend + 32.5 | A1 |
|  |  | ≥ 13.3 | ≤ 1.62 | A2 |
| 20 MHz | Fc > 2547.5 | ≥ 15.3 | ≥ -1.5*12*SCS*RBend + 36.5 | A1 |
|  |  | ≥ 15 | ≤ 5.7 | A2 |
| 25 MHz | Fc > 2547.5 | ≥ 17 | ≥ -1.5*12*SCS*RBend + 43.5 | A3 |
|  |  | ≥ 18.5 | ≤ 6.7 | A2 |
|  | 2535.5 < Fc ≤ 2547.5 | ≥ 21.6 | ≥ 21.6 | A5 |
|  |  | ≥ 20.5 | ≤ 5.6 | A2 |
| 30 MHz | 2523 < Fc ≤ 2545 | ≥ 22.5 | ≥ -0.7*12*SCS*RBend + 41.0 | A5 |
|  |  | ≥ 23.4 | ≤ 7.2 | A2 |
|  | Fc > 2545 | ≥ 20.0 | ≥ -1.5*12*SCS*RBend + 50.5 | A8 |
|  |  | ≥ 21.6 | ≤ 7.2 | A9 |
|  |  | ≤ 9.6 | ≥ 1.1*12*SCS*RBend - 1.5 | A7 |
| 35 MHz | Fc ≤ 2532.5 | ≥ 28 | ≤ 7.2 | A2 |
|  | 2532.5 < Fc ≤ 2542.5 | ≥ 27.7 | ≥ -1.2*12*SCS*RBend + 59.4 | A4 |
|  |  | ≥ 24.5 | ≤ 7.2 | A2 |
|  | Fc > 2542.5 | ≤ 18.4 | ≥ 1.15*12*SCS*RBend – 3.4 | A7 |
|  |  | ≥ 21.6 | ≥ 9, ≥ -1.2*12*SCS*RBend + 47 | A8 |
|  |  | ≥ 23 | < 9 | A9 |
| 40 MHz | Fc > 2540 | ≤ 24 | ≥ 1.15*12*SCS*RBend - 6 | A7 |
|  |  | > 24, < 35 | ≥ -0.75*12*SCS*RBend + 40.5 | A9 |
|  |  | ≥ 34 | > 7.2 | A9 |
|  |  | ≥ 27 | > 2.2, ≤ 7.2 | A8 |
|  |  | ≥ 27 | ≤ 2.2 | A9 |
|  | 2530 < Fc ≤ 2540 | ≤ 10.8 | ≥ 1.2*12*SCS*RBend – 2 | A7 |
|  |  | ≥ 29 | ≤ 7.2 | A9 |
|  |  | ≥ 28 | ≥ -1*12*SCS*RBend + 56 | A10 |
|  | Fc ≤ 2530 | ≥ 30 | ≤ 7.2 | A2 |
| 50 MHz | Fc > 2535 | ≤ 19 | ≥ 1.17*12*SCS*RBend – 10 | A7 |
|  |  | > 19, ≤ 29.5 | ≥ 1.17*12*SCS*RBend – 10 | A11 |
|  |  | > 29.5 | > 12.8, ≥ -1.15*12*SCS*RBend + 60 | A9 |
|  |  | > 41 | > 7.2, ≤ 12.8 | A12 |
|  |  | ≥ 33 | > 2.2, ≤ 7.2 | A8 |
|  |  | ≥ 33 | ≤ 2.2 | A9 |
|  | 2525 < Fc ≤ 2535 | ≤ 14 | ≥ 1.17*12*SCS*RBend – 6 | A7 |
|  |  | > 14, ≤ 26 | ≥ 1.17*12*SCS*RBend – 6 | A11 |
|  |  | ≥ 34 | > 22.5, ≥ -1.25*12*SCS*RBend + 77.5 | A8 |
|  |  | ≥ 43 | > 7.2, ≤ 22.5 | A12 |
|  |  | ≥ 35 | > 2.2, ≤ 7.2 | A8 |
|  |  | ≥ 35 | ≤ 2.2 | A9 |
|  | Fc = 2525 | ≥ 47 | ≥ -1*12*SCS*RBend + 94 | A13 |
|  |  | ≤ 9 | ≥ 1.14*12*SCS*RBend – 2 | A7 |
|  |  | ≥ 37.4 | > 2.2, ≤ 7.6 | A8 |
|  |  | ≥ 37.4 | ≤ 2.2 | A9 |

Table 6.2.3.3.17-4: A-MPR regions for NS_46 (Power class 1)

| Modulation/ Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 | A9 | A10 | A11 | A12 | A13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 2 |  | 2 | 1 | 1 | 3 | 9.5 | 5 | 6 | 4 |  | 4 | 2.5 |
|  | QPSK | 3 | 4 | 3 | 2 | 2 | 6 | 11 | 6 | 6 | 5 |  | 4 | 4 |
|  | 16 QAM | 3 | 6.5 | 3 |  |  | 6 | 12 | 6 | 6.5 | 5 |  | 4 | 4 |
|  | 64 QAM | 3 | 7 | 3 |  |  | 6 | 12.5 | 6 | 7 | 5 |  | 4 | 4 |
|  | 256 QAM |  | 7 |  |  |  | 6 | 12.5 | 6 | 7 | 5 |  |  |  |
| CP-OFDM | QPSK | 4 | 12.5 | 6 | 4 |  | 6.5 | 15 | 9 | 12.5 | 5.5 | 10 | 5 | 4 |
|  | 16 QAM | 4 | 12.5 | 6 | 4 |  | 6.5 | 15 | 9 | 13 | 5.5 | 10 | 5 | 4 |
|  | 64 QAM | 4 | 13 | 6 |  |  | 6.5 | 15 | 9 | 13 | 5.5 | 10 | 5 | 4 |
|  | 256 QAM |  | 13 |  |  |  |  | 15 | 9 | 13 |  | 10 |  |  |

Table 6.2.3.3.17-5: A-MPR regions for NS_46 (power class 2)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 2563 ≤ FC ≤ 2565 | ≥ 7.92 | ≥ 5.4 | A6 |
|  |  | ≥ 7.92 | < 2.16 | A5 |
| 15 MHz | 2560.5 ≤ FC ≤ 2562.5 | ≥ 11.16 | ≥ 7.2 | A6 |
|  |  | ≥ 11.16 | < 7.2 | A5 |
| 20 MHz | 2552 ≤ FC ≤ 2560 | ≥12.6, < 15.3 | > 10.8 | A6 |
|  |  | ≥ 15.3 | > 8.64 | A9 |
|  |  | ≥ 14.4 | ≤ 8.64 | A10 |
| 25 MHz | 2534.5 ≤ FC ≤ 2557.5 | < 14.4 | >max (0, 12*SCS*RBend - 2.7) | A5 |
|  |  | ≥ 14.4, <18.9 | > 12.6 | A6 |
|  |  | ≥ 18.9 | > 16.02 | A9 |
|  |  | ≥ 18.9 | ≤ 16.02, ≥ 9.72 | A6 |
|  |  | ≥ 17.64 | < 9.72 | A10 |
| 30 MHz | 2515 ≤ FC ≤ 2555 | ≥ 0, < 1.44 | > 0 | A4 |
|  |  | ≥ 1.44, < 13.5 | >max (0, 12*SCS*RBend -1.8) | A5 |
|  |  | ≥ 13.5, < 19.8 | > 11.52 | A6 |
|  |  | ≥ 19.8, < 25.92 | > 6.3 | A7 |
|  |  | ≥ 25.92 | > 0 | A8 |
|  |  | > 20.7 , < 25.92 | ≤ 6.3 | A10 |
| 35 MHz | 2517.5 ≤ FC ≤ 2552.5 | ≥0, <3.42 | > 0 | A4 |
|  |  | ≥3.42, <15.84 | > max (0, 12*SCS*RBend - 3.06) | A5 |
|  |  | ≥15.84, <22.68 | > 12.6 | A6 |
|  |  | ≥22.68, <28.8 | > 9.0 | A7 |
|  |  | ≥28.8 | > 0 | A8 |
|  |  | > 24.3 , < 28.8 | ≤ 9.0 | A10 |
| 40 MHz | 2520 ≤ FC ≤ 2550 | ≥0, < 4.5 | > 0 | A4 |
|  |  | ≥ 4.14, < 5.04 | > max (0, 12*SCS*RBend - 4.5) | A11 |
|  |  | ≥ 5.04, < 18 | > max (0, 12*SCS*RBend - 4.5) | A5 |
|  |  | ≥ 18, < 25.74 | > 13.5 | A6 |
|  |  | ≥ 25.74, < 32.4 | > 12.6 | A7 |
|  |  | ≥ 32.4 | > 0 | A8 |
|  |  | > 27.9 , < 32.4 | ≤ 12.6 | A10 |
| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥ 0, < 9 | > 0 | A4 |
|  |  | ≥ 9, < 21.6 | > max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥ 21.6, < 31.5 | > 18 | A6 |
|  |  | ≥ 31.5, < 39.6 | > 16.2 | A7 |
|  |  | ≥ 39.6 | > 0 | A8 |
|  |  | > 33.84, < 39.6 | ≤ 16.2 | A10 |

Table 6.2.3.3.17-6: A-MPR for NS_46 (power class 2)

| Modulation/Waveform |  | A4 | A5 | A6 | A7 | A8 | A9 | A10 | A11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner |
| DFT-s-OFDM | PI/2 BPSK | 6.5 | 2.5 | 3.5 | 7 | 11 | 6.5 | 3.0 | 5.0 |
|  | QPSK | 6.5 | 2.5 | 3.5 | 7 | 11 | 6.5 | 3.0 | 5.0 |
|  | 16 QAM | 6.5 | 2.5 | 3.5 | 7 | 11 | 6.5 | 3.5 | 5.0 |
|  | 64 QAM | 6.5 | 2.5 | 3.5 | 7 | 11 | 6.5 | 3.5 | 5.0 |
|  | 256 QAM | 6.5 |  |  | 7 | 11 | 6.5 |  | 5.0 |
| CP-OFDM | QPSK | 6.5 | 3.5 | 5.5 | 8 | 12 | 8 | 4.5 | 6.5 |
|  | 16 QAM | 6.5 | 3.5 | 5.5 | 8 | 12 | 8 | 4.5 | 6.5 |
|  | 64 QAM | 6.5 | 3.5 | 5.5 | 8 | 12 | 8 | 4.5 | 6.5 |
|  | 256 QAM | 6.5 |  |  | 8 | 12 | 8 |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.17.

6.2.3.3.18	A-MPR for NS_47

Table 6.2.3.3.18-1: A-MPR regions for NS_47 (Power Class 2 and 3)

| Channel Bandwidth, (MHz) | Carrier Centre Frequency, Fc, (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 30MHz | Fc=2560-2560.020 | ≤5.04 | ≤1.44 | A1 |
|  |  | >5.04, ≤9.6 | ≤1.44 | A2 |
|  |  | >24.48 | ≤1.44 | A3 |
|  |  | ≤9.6 | >21 | A2 |
|  |  |  | >14.4, <21 | A4 |
|  |  | ≤6.12 | >10, ≤14.4 | A4 |
|  |  |  | >1.44, <10 | A2 |
| NOTE: The A-MPR values are listed in Table 6.2.3.3.18-2. |  |  |  |  |

Table 6.2.3.3.18-2: A-MPR for modulation and waveform type (Power Class 2 and 3)

| Modulation/Waveform | A1(dB) |  | A2(dB) |  | A3(dB) |  | A4(dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC3 | PC2 | PC3 | PC2 | PC3 | PC2 | PC3 | PC2 |
|  | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 7 | ≤ 10 | ≤ 5.5 | ≤ 8.5 | ≤ 2 | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM QPSK | ≤ 7 | ≤ 10 | ≤ 5.5 | ≤ 8.5 | ≤ 2 | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM 16 QAM | ≤ 7 | ≤ 10 | ≤ 5.5 | ≤ 8.5 |  | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM 64 QAM | ≤ 7 | ≤ 10 | ≤ 6 | ≤ 8.5 |  | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM 256 QAM | ≤ 7 | ≤ 10 | ≤ 6 | ≤ 8.5 |  | ≤ 5 |  | ≤ 6 |
| CP-OFDM QPSK | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  | ≤ 5 | ≤ 4 | ≤ 7 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  | ≤ 5 | ≤ 4 | ≤ 7 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  | ≤ 5 |  | ≤ 7 |
| CP-OFDM 256 QAM | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  |  |  | ≤ 7 |

Table 6.2.3.3.18-3: A-MPR regions and types for NS_47 (Power Class 1.5)

| Channel Bandwidth, (MHz) | Carrier Centre Frequency, Fc, (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 30MHz | Fc=2560-2560.020 | ≤5.04 | ≤1.44 | A1 |
|  |  | >5.04, ≤9.6 | ≤1.44 | A2 |
|  |  | >24.48 | ≤1.44 | A3 |
|  |  | ≤9.6 | >21 | A2 |
|  |  |  | >14.4, <21 | A4 |
|  |  | >6.12, ≤7.92 | >10, ≤14.4 | A5 |
|  |  | ≤6.12 | >10, ≤14.4 | A4 |
|  |  |  | >1.44, <10 | A2 |
| NOTE: The A-MPR values are listed in Table 6.2.3.3.18-4. |  |  |  |  |

Table 6.2.3.3.18-4: A-MPR for NS_47 (Power Class 1.5)

| Modulation/Waveform |  | A1(dB) | A2(dB) | A3(dB) | A4(dB) | A5(dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤ 13 | ≤ 11 | ≤ 8 | ≤ 8.5 | ≤ 3 |
|  | QPSK | ≤ 13 | ≤ 11 | ≤ 8 | ≤ 8.5 | ≤ 3 |
|  | 16 QAM | ≤ 13 | ≤ 11 | ≤ 8 | ≤ 8.5 | ≤ 3 |
|  | 64 QAM | ≤ 13 | ≤ 11 | ≤ 8 | ≤ 8.5 |  |
|  | 256 QAM | ≤ 13 | ≤ 11 | ≤ 8 | ≤ 8.5 |  |
| CP-OFDM | QPSK | ≤ 13 | ≤ 12.5 | ≤ 8 | ≤ 9.5 | ≤ 4 |
|  | 16 QAM | ≤ 13 | ≤ 12.5 | ≤ 8 | ≤ 9.5 | ≤ 4 |
|  | 64 QAM | ≤ 13 | ≤ 12.5 | ≤ 8 | ≤ 9.5 |  |
|  | 256 QAM | ≤ 13 | ≤ 12.5 | ≤ 8 | ≤ 9.5 |  |
| NOTE 1: PC1.5 assumes dual Tx. |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.18.

6.2.3.3.19	A-MPR for NS_50

Table 6.2.3.3.19-1: A-MPR regions for NS_50 (Power Class 3)

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 25 MHz | ≤ LCRB*12*SCS - 5 | > 5 | A7 |
|  | ≤ 6.48 | ≤ 1.44 | A8 |
|  |  | ≤ 3.6 | A9 |
| 30 MHz | ≤ LCRB*12*SCS - 5 | > 5 | A7 |
|  | ≤ 8.64 | ≤ 1.44 | A8 |
|  |  | ≤ 3.6 | A9 |
| 35 MHz | ≤ 1.62 | > 0 | A1 |
|  | > 1.62, ≤ 9.36 | ≤ 9.0 | A3 |
|  | > 1.62, ≤ 13.68 | > 9.0 | A2 |
|  | > 13.68, ≤ 30.6 | > max (29.7 –RBstart *12*SCS, 0) | A6 |
|  | > 30.6 | > 0 | A5 |
| 40 MHz | ≤ 4.32 | > 0 | A1 |
|  | > 4.32, ≤ 10.44 | ≤ 10.8 | A3 |
|  | > 4.32, ≤ 18 | > 10.8 | A2 |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A6 |
|  | > 31.68 | > 0 | A5 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.3.19-2. |  |  |  |

Table 6.2.3.3.19-2: A-MPR for NS_50 (Power Class 3)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A5 (dB) | A6 (dB) | A7 (dB) | A8 (dB) | A9 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 | ≤ 2 | ≤ 4 | ≤ 2 |  |
|  | QPSK | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 | ≤ 2 | ≤ 5 | ≤ 2 |  |
|  | 16 QAM | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 | ≤ 2 | ≤ 5 | ≤ 2.5 |  |
|  | 64 QAM | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 |  | ≤ 5 |  |  |
|  | 256 QAM | ≤ 11 | ≤ 7 |  | ≤ 5 |  | ≤ 5 |  |  |
| CP-OFDM | QPSK | ≤ 12 | ≤ 8 | ≤ 4.5 | ≤ 5 | ≤ 3.5 | ≤ 6.5 |  | ≤ 3.0 |
|  | 16 QAM | ≤ 12 | ≤ 8 | ≤ 4.5 | ≤ 5 | ≤ 3.5 | ≤ 6.5 |  | ≤ 3.0 |
|  | 64 QAM | ≤ 12 | ≤ 8 | ≤ 4.5 | ≤ 5 |  | ≤ 6.5 |  |  |
|  | 256 QAM | ≤ 12 | ≤ 8 |  |  |  | ≤ 6.5 |  |  |

Table 6.2.3.3.19-3: A-MPR regions for NS_50 (Power Class 2)

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 10 MHz | ≤ 1.44 | < 1.44 | A5 |
|  | ≤ 1.8 | ≥ 2.7+2* RBstart*12*SCS | A4 |
|  | >1.8 | ≥ 8.1- RBstart*12*SCS | A4 |
| 15 MHz | ≤ 2.88 | < 2.7 | A5 |
|  | ≤ 3.24 | ≥ 2.7+2* RBstart*12*SCS | A3 |
|  | >3.24 | ≥ 12.42- RBstart*12*SCS | A4 |
| 20 MHz | ≤ 4.32 | < 3.6 | A5 |
|  | ≤ 4.5 | ≥ 3.6+2* RBstart*12*SCS | A3 |
|  | >4.5 | ≥ 17.1- RBstart*12*SCS | A4 |
| 25 MHz | ≤ LCRB*12*SCS – 5 | > 5 | A2 |
|  | ≤ 6.48 | ≤ 1.44 | A5 |
|  | > 8.28 | > max (21.6 – RBstart*12*SCS, 0), <RBstart*12*SCS+5 | A4 |
|  | >1.8, ≤6.48 | > 1.44, ≤ 3.6 | A6 |
|  | > LCRB *12*SCS – 5, ≤ 1.8 | > 1.44 | A4 |
| 30 MHz | ≤ LCRB*12*SCS – 5 | >5 | A2 |
|  | ≤ 7.56 | ≤ 1.44 | A5 |
|  | >1.8, ≤7.56 | > 1.44, ≤ 3.6 | A6 |
|  | ≤ 1.8 | >1.44, <RBstart*12*SCS+5 | A4 |
|  | > 10.8 | > max (26.64 – RBstart*12*SCS, 0), <RBstart*12*SCS+5 | A4 |
| 35 MHz | ≤ 1.62 | > 0 | A1 |
|  | > 1.62, ≤ 9.36 | ≤ 9.0 | A3 |
|  | > 1.62, ≤ 13.68 | > 9.0 | A8 |
|  | > 13.68, ≤ 30.6 | > max (29.7 –RBstart *12*SCS, 0) | A4 |
|  | > 30.6 | > 0 | A9 |
| 40 MHz | ≤ 4.32 | > 0 | A1 |
|  | > 4.32 | > RBstart*12*SCS + 11.88 | A8 |
|  | > 4.32, ≤ 12.96 | ≤ 10.8 | A3 |
|  | > 4.32, ≤ 18 | > 10.8, <= RBstart*12*SCS + 11.88 | A7 |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A4 |
|  | > 31.68 | > 0 | A9 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.3.19-4. |  |  |  |

Table 6.2.3.3.19-4: A-MPR for NS_50 (Power Class 2)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) | A7 (dB) | A8 (dB) | A9 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer /Inner | Outer /Inner | Outer /Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 11.5 | ≤4.5 | ≤ 4 | ≤ 2.5 | ≤ 4 | ≤ 1 | ≤ 3.5 | ≤ 10 | ≤ 8 |
|  | QPSK | ≤ 11.5 | ≤ 5.5 | ≤ 4 | ≤ 2.5 | ≤ 4 | ≤ 1 | ≤ 3.5 | ≤ 10 | ≤ 8 |
|  | 16 QAM | ≤ 11.5 | ≤ 5.5 | ≤ 4 | ≤ 2.5 | ≤ 4 | ≤ 1.5 | ≤ 3.5 | ≤ 10 | ≤ 8 |
|  | 64 QAM | ≤ 11.5 | ≤ 5.5 | ≤ 4 |  | ≤ 4 |  |  | ≤ 10 | ≤ 8 |
|  | 256 QAM | ≤ 11.5 | ≤ 5.5 |  |  |  |  |  | ≤ 10 | ≤ 8 |
| CP-OFDM | QPSK | ≤ 12.5 | ≤ 7 | ≤ 5.5 | ≤ 4 | ≤ 4 | ≤ 2 | ≤ 5 | ≤ 11 | ≤ 8 |
|  | 16 QAM | ≤ 12.5 | ≤ 7 | ≤ 5.5 | ≤ 4 | ≤ 4 |  | ≤ 5 | ≤ 11 | ≤ 8 |
|  | 64 QAM | ≤ 12.5 | ≤ 7 | ≤ 5.5 | ≤ 4 | ≤ 4 |  | ≤ 5 | ≤ 11 | ≤ 8 |
|  | 256 QAM | ≤ 12.5 | ≤ 7 |  |  |  |  |  | ≤ 11 | ≤ 8 |

6.2.3.3.20	A-MPR for NS_44

Table 6.2.3.3.20-1: A-MPR regions for NS_44

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 2582.5≤ FC ≤ 2602.5 | <18.0 | >max(0, 12*SCS* RBend - 3.6) | A3 |
|  |  | ≥18.0 | <7.2 | A3 |
|  |  | ≥18.0 | ≥7.2 | A6 |
| 30 MHz | 2585 ≤ FC ≤ 2600 | <21.6 | >max(0, 12*SCS* RBend - 3.6) | A3 |
|  |  | ≥21.6 | <12.6 | A3 |
|  |  | ≥21.6 | ≥12.6 | A6 |
| 40 MHz | 2590 ≤ FC ≤ 2595 | ≥0, <2.88 | >0 | A1 |
|  |  | ≥2.88, <14.4 | >max (0, 12*SCS*RBend - 3.6) | A2 |
|  |  | ≥14.4, <23.4 | >10.8 | A3 |
|  |  | ≥23.4, <32.4 | >16.2 | A4 |
|  |  | ≥32.4 | >0 | A5 |

Table 6.2.3.3.20-2: A-MPR for NS_44

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 5 | 2 | 3 | 7 | 12 | 4 |
|  | QPSK | 5 | 2 | 3 | 7 | 12 | 4 |
|  | 16 QAM | 5 | 2 | 3 | 7 | 12 | 4 |
|  | 64 QAM | 5 |  | 3 | 7 | 12 | 4 |
|  | 256 QAM | 5 |  |  | 7 | 12 |  |
| CP-OFDM | QPSK | 5 | 4 | 5 | 8 | 12 | 6 |
|  | 16 QAM | 5 | 4 | 5 | 8 | 12 | 6 |
|  | 64 QAM | 5 | 4 | 5 | 8 | 12 | 6 |
|  | 256 QAM |  |  |  | 8 | 12 |  |

6.2.3.3.21	A-MPR for NS_12

Table 6.2.3.3.21-1: A-MPR regions for NS_12

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 815.5 ≤ Fc < 817.5 | ≤0.9 | >0 | A2 |
| 5MHz |  | ≤1.8 | >0 | A1 |
| 10MHz |  | ≤3.6 | >0 | A1 |

Table 6.2.3.3.21-2: A-MPR for NS_12

| Modulation/Waveform | A1 | A2 |
| --- | --- | --- |
|  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 4.5 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 | ≤ 4.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | ≤ 5 |
| CP-OFDM QPSK | ≤ 7 | ≤ 6 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 6 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 9.5 |  |

6.2.3.3.22	A-MPR for NS_13

Table 6.2.3.3.22-1: A-MPR regions for NS_13

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 818.5 ≤ Fc < 820 | ≤0.54 | >0 | A4 |
| 5MHz | 819.5 ≤ Fc < 821.5 | ≤1.44 | <1.08 | A1 |
|  |  | ≤1.44 | ≥1.08 | A2 |
| 5MHz | Fc ≥ 821.5 | ≤0.54 | <1.08 | A1 |
|  |  |  | ≥3.24 | A3 |

Table 6.2.3.3.22-2: A-MPR for NS_13

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 6 |  |  |
| CP-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 |  |  |

6.2.3.3.23	A-MPR for NS_14

Table 6.2.3.3.23-1: A-MPR regions for NS_14

| Channel BW | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 10MHz | ≤0.18 | <1.08 | A1 |
|  | ≥0 | ≥9 | A2 |
| 15MHz | ≤1.8 | <1.8 | A1 |
|  | ≥0 | ≥9 | A2 |
| 20MHz | ≤3.42 | <1.8 | A3 |
|  | ≥0 | ≥9 | A2 |

Table 6.2.3.3.23-2: A-MPR for NS_14

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3 | ≤ 2 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 3 | ≤ 2 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 3 | ≤ 2 | ≤ 3 |
| DFT-s-OFDM 64 QAM | ≤ 3 |  | ≤ 3 |
| DFT-s-OFDM 256 QAM |  |  | ≤ 8 |
| CP-OFDM QPSK | ≤ 5 | ≤ 4 | ≤ 5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 4 | ≤ 5 |
| CP-OFDM 64 QAM | ≤ 6 |  | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 8 |  | ≤ 8 |

6.2.3.3.24	A-MPR for NS_15

Table 6.2.3.3.24-1: A-MPR regions for NS_15

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 844.5 < Fc ≤ 847.5 | ≥1.44 | >0 | A5 |
|  |  | ≤ 0.18 | ≤ 0.36 | A6 |
| 5MHz | 840.5 < Fc ≤ 846.5 | ≥3.24 | >0 | A1 |
|  |  | <3.24, ≥2.52 | ≥1.44 | A2 |
|  |  | <0.9 | ≤0.36 | A3 |
| 10MHz | 840 < Fc ≤ 844 | ≥5.76 | >1.08 | A1 |
|  |  | ≥5.76 | ≤1.08 | A4 |
|  |  | <5.76, ≥4.14 | ≥2.7 | A2 |
|  |  | <2.52 | ≤0.36 | A3 |
|  | 835 < Fc ≤ 840 | ≥7.2 | >0 | A1 |
|  |  | <7.2, ≥5.22 | ≥4.32 | A2 |
|  |  | <1.08 | ≤0.36 | A3 |
| 15MHz | 837.5 < Fc ≤ 841.5 | ≥9.36 | >1.08 | A1 |
|  |  | ≥9.36 | ≤1.08 | A4 |
|  |  | <9.36, ≥4.68 | ≥3.6 | A2 |
|  |  | <3.96 | ≤0.36 | A3 |
|  | 831.5 < Fc ≤ 837.5 | ≥10.8 | >1.08 | A1 |
|  |  | ≥10.8 | ≤1.08 | A4 |
|  |  | <10.8, ≥6.48 | ≥3.6 | A2 |
|  |  | <2.7 | ≤0.36 | A3 |
|  | Fc ≤ 831.5 | ≥13.14 | >0 | A1 |
|  |  | <13.14, ≥7.92 | ≥3.6 | A2 |
|  |  | <0.72 | ≤0.36 | A3 |
| 20MHz | 835 < Fc ≤ 839 | ≥12.24 | >1.08 | A1 |
|  |  | ≥12.24 | ≤1.08 | A4 |
|  |  | <12.24, ≥8.46 | ≥5.4 | A2 |
|  |  | <5.58 | ≤0.36 | A3 |
|  | Fc ≤ 835 | ≥13.68 | >1.08 | A1 |
|  |  | ≥13.68 | ≤1.08 | A4 |
|  |  | <13.68, ≥8.46 | ≥5.4 | A2 |
|  |  | <4.32 | ≤0.36 | A3 |

Table 6.2.3.3.24-2: A-MPR for NS_15

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Edge/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5 | 1 |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 1.5 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 2 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 |  |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 | ≤ 13.5 | ≤ 6 |  |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 |  |
| CP-OFDM 256 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 9 | ≤ 13.5 | ≤ 7.5 |  |

6.2.3.3.25	A-MPR for NS_45

Table 6.2.3.3.25-1: A-MPR for NS_45

| Modulation/Waveform |  | Outer |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1.5 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |

6.2.3.3.26	A-MPR for NS_48

Table 6.2.3.3.26-1: A-MPR regions for NS_48 (Power Class 3)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 1932.5≤ FC ≤ 1967.5 | ≥0 | ≥9.72 | A3 |
|  |  | ≥18.72 | <1.<1.08 | A3 |
| 30 MHz | 1935 ≤ FC ≤ 1965 | ≥0 | ≥13.5 | A3 |
|  |  | ≥21.6 | <1.08<1.08 | A5 |
| 40 MHz | 1940 ≤ FC ≤ 1960 | ≥0, <2.88 | ≥0 | A2 |
|  |  | ≥2.88, <17.1 | ≥max (0, 12*SCS*RBend - 3.6) | A3 |
|  |  | ≥17.1, <27.36 | ≥13.5 | A4 |
|  |  | ≥27.36, <34.56 | ≥13.5 | A2 |
|  |  | ≥27.36, <34.56 | <1.08 | A3 |
|  |  | ≥34.56 | ≥0 | A1 |
| 45 MHz | 1942.5 ≤ FC ≤ 1957.5 | ≥0, <4.86 | >0 | A2 |
|  |  | ≥4.86, <19 | ≥max (0, 12*SCS*RBend - 3.6) | A4 |
|  |  | ≥19, <37.44 | ≥15.4 | A2 |
|  |  | ≥30.96, <37.44 | <1.08 | A5 |
|  |  | ≥37.44 | >0 | A1 |
| 50 MHz | 1945 ≤ FC ≤ 1955 | ≥0, <6.12 | >0 | A2 |
|  |  | ≥6.12, <20.7 | ≥max (0, 12*SCS*RBend - 3.6) | A4 |
|  |  | ≥20.7, <41.04 | ≥17.1 | A2 |
|  |  | ≥33.84, <41.04 | <1.08 | A5 |
|  |  | ≥41.04 | >0 | A1 |

Table 6.2.3.3.26-2: A-MPR for NS_48 (Power Class 3)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | QPSK | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | 16 QAM | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | 64 QAM | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | 256 QAM | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
| CP-OFDM | QPSK | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |
|  | 16 QAM | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |
|  | 64 QAM | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |
|  | 256 QAM | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |

Table 6.2.3.3.26-3: A-MPR regions for NS_48 (Power Class 2)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 1925≤ FC ≤ 1975 | ≥0 | ≥8.1 | A6 |
|  |  | <1.8 | ≥0 | A6 |
| 15 MHz | 1927.5≤ FC ≤ 1972.5 | ≥0 | ≥9 | A6 |
|  |  | ≥0 | ≥max (0,12*SCS* RBend - 2.88) <9 | A6 |
| 20 MHz | 1930≤ FC ≤ 1970 | ≥0 | ≥9.72 | A4 |
|  |  | ≥0 | ≥max (0,12*SCS* RBend - 3.6) <9.72 | A6 |
| 25 MHz | 1932.5≤ FC ≤ 1967.5 | ≥0 | ≥9.72 | A4 |
|  |  | ≥18.72 | <1.08 | A3 |
|  |  | ≥0 | ≥max (0, 12*SCS*RBend - 1.08) < 9.72 | A4 |
| 30 MHz | 1935 ≤ FC ≤ 1965 | ≥0 | ≥12.96 | A4 |
|  |  | ≥21.6 | <1.08 | A5 |
|  |  | ≥0 | ≥max (0, 12*SCS*RBend - 3.6) < 12.96 | A4 |
| 40 MHz | 1940 ≤ FC ≤ 1960 | ≥0, <2.88 | ≥0 | A1 |
|  |  | ≥2.88, <17.1 | ≥max (0, 12*SCS*RBend - 4.68) | A3 |
|  |  | ≥17.1, <27.36 | ≥12.96 | A4 |
|  |  | ≥27.36, <34.56 | ≥12.96 | A2 |
|  |  | ≥27.36, <34.56 | <1.08 | A3 |
|  |  | ≥34.56 | ≥0 | A1 |
| 45 MHz | 1942.5 ≤ FC ≤ 1957.5 | ≥0, <5.22 | >0 | A1 |
|  |  | ≥5.22, <19 | ≥max (0, 12*SCS*RBend - 5.4) | A4 |
|  |  | ≥19, <37.44 | ≥14.04 | A2 |
|  |  | ≥30.96, <37.44 | <1.08 | A5 |
|  |  | ≥37.44 | >0 | A1 |
| 50 MHz | 1945 ≤ FC ≤ 1955 | ≥0, <7.2 | >0 | A1 |
|  |  | ≥7.2, <20.7 | ≥max (0, 12*SCS*RBend – 5.4) | A4 |
|  |  | ≥20.7, <41.04 | ≥15.12 | A2 |
|  |  | ≥33.84, <41.04 | <1.08 | A5 |
|  |  | ≥41.04 | >0 | A1 |

Table 6.2.3.3.26-4: A-MPR for NS_48 (Power Class 2)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤[12] | ≤8.0 | ≤4.0 | ≤4.5 | ≤6.0 | ≤3.0 |
|  | QPSK | ≤[12] | ≤8.0 | ≤4.0 | ≤4.5 | ≤6.0 | ≤3.0 |
|  | 16 QAM | ≤[12] | ≤8.0 | ≤4.0 | ≤4.5 | ≤6.0 | ≤3.0 |
|  | 64 QAM | ≤[12] | ≤8.0 | ≤4.0 | ≤4.5 | ≤6.0 | ≤3.0 |
|  | 256 QAM | ≤[12] | ≤8.0 | ≤4.0 | ≤4.5 | ≤6.0 | ≤3.0 |
| CP-OFDM | QPSK | ≤[12.5] | ≤9.0 | ≤5.5 | ≤6.0 | ≤6.0 | ≤4.5 |
|  | 16 QAM | ≤[12.5] | ≤9.0 | ≤5.5 | ≤6.0 | ≤6.0 | ≤4.5 |
|  | 64 QAM | ≤[12.5] | ≤9.0 | ≤5.5 | ≤6.0 | ≤6.0 | ≤4.5 |
|  | 256 QAM | ≤[12.5] | ≤9.0 | ≤5.5 | ≤6.0 | ≤6.0 | ≤4.5 |

6.2.3.3.27	A-MPR for NS_49

Table 6.2.3.3.27-1: A-MPR regions for NS_49

| Channel Bandwidth (MHz) | Carrier Center Frequency, Fc (MHz) | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) |  |
| 25 MHz | 1932.5≤ FC ≤ 1967.5 | ≥0 | ≥9.72 | A3 |
|  |  | ≥18.72 | <1.08 | A3 |
|  |  | ≤3.96 | <1.08 | A3 |
| 30 MHz | 1935 ≤ FC ≤ 1965 | ≥0, <3.6 | ≥0 | A1 |
|  |  | ≥3.6, <6.48 | ≥0 | A5 |
|  |  | ≥6.48, <14.4 | ≥max (0,12*SCS* RBend - 3.6) | A3 |
|  |  | ≥14.4, <21.6 | ≥10.8 | A4 |
|  |  | ≥21.6 | ≥10.8 | A2 |
|  |  | ≥21.6 | <1.08 | A5 |
| 40 MHz | 1940 ≤ FC ≤ 1960 | ≥0, <7.2 | ≥0 | A1 |
|  |  | ≥7.2, <10.44 | <1.08 | A5 |
|  |  | ≥7.2, <18 | ≥max (0, 12*SCS*RBend - 3.6) | A4 |
|  |  | ≥18, <34.56 | ≥14.4, <28.8 | A2 |
|  |  | ≥27.36, <34.56 | <1.08 | A5 |
|  |  | <34.56 | ≥28.8 | A1 |
|  |  | ≥34.56 | ≥0 | A1 |
|  |  | ≥6.12, <12.42 | < min [1.08, max(0,12*SCS* RBend-6.12)] | A5 |
|  |  | ≥30.76, <36.72 | <1.08 | A5 |
| 45 MHz | 1942.5 ≤ FC ≤ 1957.5 | <36.72 | ≥16.2, <max (0, 12*SCS*RBend – 6.12) | A2 |
|  |  | <36.72 | ≥max (0, 12*SCS*RBend – 6.12) | A1 |
|  |  | ≥36.72 | >0 | A1 |
| 50 MHz | 1945 ≤ FC ≤ 1955 | ≥7.74, <14.4 | < min (1.08, max(0,12*SCS* RBend-7.74)) | A5 |
|  |  | ≥36, <39.6 | <1.08 | A5 |
|  |  | <39.6 | ≥18, <max (0, 12*SCS*RBend – 7.74) | A2 |
|  |  | <39.6 | ≥max (0, 12*SCS*RBend – 7.74) | A1 |
|  |  | ≥39.6 | >0 | A1 |

Table 6.2.3.3.27-2: A-MPR for NS_49

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | QPSK | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | 16 QAM | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | 64 QAM | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | 256 QAM | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
| CP-OFDM | QPSK | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |
|  | 16 QAM | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |
|  | 64 QAM | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |
|  | 256 QAM | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |

Table 6.2.3.3.27-3: A-MPR regions for NS_49 (Power Class 2)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 1925≤ FC ≤ 1975 | ≥0 | ≥8.1 | A3 |
|  |  | <1.8 | ≥0 | A3 |
| 15 MHz | 1927.5≤ FC ≤ 1972.5 | ≥0 | ≥9 | A3 |
|  |  | ≥0 | ≥max (0,12*SCS* RBend - 2.88) <9 | A3 |
| 20 MHz | 1930≤ FC ≤ 1970 | ≥0 | ≥9.72 | A4 |
|  |  | ≥0 | ≥max (0,12*SCS* RBend - 3.6) <9.72 | A3 |
| 25 MHz | 1932.5≤ FC ≤ 1967.5 | ≥0 | ≥7.92 | A4 |
|  |  | ≥18.72 | <1.08 | A3 |
|  |  | ≤6.48 | <3.6 | A3 |
| 30 MHz | 1935 ≤ FC ≤ 1965 | ≥0, <3.6 | ≥0 | A1 |
|  |  | ≥3.6, <7.92 | ≥0 | A5 |
|  |  | ≥7.92, <14.4 | ≥max (0,12*SCS* RBend - 4.32) | A3 |
|  |  | ≥14.4, <21.6 | ≥10.44 | A4 |
|  |  | ≥21.6 | ≥10.44 | A2 |
|  |  | ≥21.6 | <1.8 | A1 |
| 40 MHz | 1940 ≤ FC ≤ 1960 | ≥0, <9 | ≥0 | A1 |
|  |  | ≥9, <11.52 | <3.06 | A5 |
|  |  | ≥9, <18 | ≥max (3.06, 12*SCS*RBend – 6.48) | A4 |
|  |  | ≥18, <34.56 | ≥11.16, <27 | A2 |
|  |  | ≥27.36, <34.56 | <1.08 | A5 |
|  |  | <34.56 | ≥27 | A1 |
|  |  | ≥34.56 | ≥0 | A1 |
|  |  | ≥7.92, <12.42 | < min [1.08, max(0,12*SCS* RBend-7.92)] | A5 |
|  |  | ≥30.76, <36.72 | <1.08 | A5 |
| 45 MHz | 1942.5 ≤ FC ≤ 1957.5 | <36.72 | ≥12.24, <max (0, 12*SCS*RBend – 7.92) | A2 |
|  |  | <36.72 | ≥max (0, 12*SCS*RBend – 7.92) | A1 |
|  |  | ≥36.72 | >0 | A1 |
| 50 MHz | 1945 ≤ FC ≤ 1955 | ≥10.08, <14.4 | < min [1.08, max(0,12*SCS* RBend-10.08)] | A5 |
|  |  | ≥36, <39.6 | <1.08 | A5 |
|  |  | <39.6 | ≥13.68, <max (0, 12*SCS*RBend – 10.08) | A2 |
|  |  | <39.6 | ≥max (0, 12*SCS*RBend – 10.08) | A1 |
|  |  | ≥39.6 | >0 | A1 |

Table 6.2.3.3.27-4: A-MPR for NS_49 (Power Class 2)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤[12.5] | ≤7 | ≤4 | ≤5.5 | ≤6 |
|  | QPSK | ≤[12.5] | ≤7 | ≤4 | ≤5.5 | ≤6 |
|  | 16 QAM | ≤[12.5] | ≤7 | ≤4 | ≤5.5 | ≤6 |
|  | 64 QAM | ≤[12.5] | ≤7 | ≤4 | ≤5.5 | ≤6 |
|  | 256 QAM | ≤[12.5] | ≤7 | ≤4 | ≤5.5 | ≤6 |
| CP-OFDM | QPSK | ≤[13.5] | ≤8 | ≤5.5 | ≤7.5 | ≤6 |
|  | 16 QAM | ≤[13.5] | ≤8 | ≤5.5 | ≤7.5 | ≤6 |
|  | 64 QAM | ≤[13.5] | ≤8 | ≤5.5 | ≤7.5 | ≤6 |
|  | 256 QAM | ≤[13.5] | ≤8 | ≤5.5 | ≤7.5 | ≤6 |

6.2.3.3.28	A-MPR for NS_51

Table 6.2.3.3.28: void

6.2.3.3.29	A-MPR for NS_07

Table 6.2.3.3.29-1: A-MPR regions for NS_07

| Channel Bandwidth, MHz | Carrier Frequency, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 5 MHz | 782 ≤ Fc ≤ 784.5 | ≥0 | ≥ 1.8 | A3 |
| 5 MHz | 779.5 ≤ Fc < 782 | ≤ 0.9 | ≥ 0 | A1 |
|  |  | > 0.9, ≤ 1.26 | ≥1.26 | A2 |
|  |  | > 1.26, ≤ 3.42 | ≥ 1.8 | A3 |
|  |  | >3.42 | ≤ 0.36 | A4 |
| 10 MHz | Fc = 782 | ≤ 2.34 | ≥ 0 | A1 |
|  |  | >2.34, ≤ 3.24 | ≥ 1.44 | A2 |
|  |  | > 3.24, ≤ 6.48 | ≥ 3.24 | A3 |
|  |  | > 6.48 | ≤ 0.36 | A4 |

Table 6.2.3.3.29-2: A-MPR for NS_07

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | 12 | 9 | 6 | 3 |
| DFT-s-OFDM QPSK | 12 | 9 | 6 | 3 |
| DFT-s-OFDM 16 QAM | 12 | 9 | 6 | 3 |
| DFT-s-OFDM 64 QAM | 12 | 9 | 6 | 3 |
| DFT-s-OFDM 256 QAM | 12 | 9 | 6 | 3 |
| CP-OFDM QPSK | 14 | 10 | 7 | 3 |
| CP-OFDM 16 QAM | 14 | 10 | 7 | 3 |
| CP-OFDM 64 QAM | 14 | 10 | 7 | 3 |
| CP-OFDM 256 QAM | 14 | 10 | 7 | 3 |

6.2.3.3.30	A-MPR for NS_56

For 5 MHz channel centred on frequencies (FC) = 1630.0, 1630.3 MHz, A-MPR is defined as

if RBstart <= ceil{3/SCS/15 kHz)}and LCRB <= ceil{17/SCS/15 kHz)},

then

the A-MPR = 14 dB for SCS = 15 kHz and AMPR = 8 dB for SCS >= 30 kHz,

else,

if RBstart <= ceil{3/(SCS/15 kHz)} and LCRB > ceil{17/(SCS/15 kHz)},

then

the A-MPR = 6 dB,

else,

if RBstart <= ceil{8/(SCS/15 kHz)},

then

the A-MPR = 4 dB.

For 5 MHz channel centred on frequencies (Fc) = 1635.0, 1649.0, 1654.0 MHz, no A-MPR is needed.

For Channel 10 MHz with centre frequency of 1632.5 MHz, A-MPR is defined as

if RBstart < ceil{3/(SCS/15 kHz)} and LCRB <= ceil{8/(SCS/15 kHz)},

then

the A-MPR = 12 dB for SCS = 15 kHz and AMPR = 8 dB for SCS >= 30 kHz,

else,

if RBstart < ceil{9/(SCS/15 kHz)}, and LCRB > ceil{8/(SCS/15 kHz)},

then

the A-MPR = 8 dB,

else,

if RBstart <= ceil{18/(SCS/15 kHz)},

then

the A-MPR = 6 dB,

else,

if RBstart >= floor{40/(SCS/15 kHz)}], and LCRB <= ceil{7/(SCS/15 kHz)},

then

the A-MPR = 5 dB,

else,

if RBstart >= floor{40/(SCS/15 kHz)} and LCRB > ceil{7/(SCS/15 kHz)},

then

the A-MPR = 3 dB,

else,

if RBstart >= floor{35/(SCS/15 kHz)} and LCRB <= ceil{7/(SCS/15 kHz)},

then

the A-MPR = 4 dB,

else,

if RBstart >= floor{35/(SCS/15 kHz)} and LCRB > ceil{7/(SCS/15 kHz)},

then

the A-MPR = 2 dB.

For 10 MHz channel centered on frequency of 1651.5 MHz, no A-MPR is needed.

6.2.3.3.31	A-MPR for NS_35

For power class 1 operation A-MPR = 8.5 dB if

( LCRB ≤ 0.20 ∙ NRB and ( RBstart = 0 or RBstart + LCRB = NRB ) )

or

( LCRB = 1 and 5 ∙ | RBstart + 0.5 – NRB / 2 | ∙ 12 ∙ SCS ≥ 1.5 ∙ CBW + 5 MHz ).

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.31.

6.2.3.3.32	A-MPR for NS_06

For power class 1 operation on band n85 A-MPR = 8.5 dB if

( LCRB ≤ 0.20 ∙ NRB and ( RBstart = 0 or RBstart + LCRB = NRB ) )

or

( LCRB = 1 and 5 ∙ | RBstart + 0.5 – NRB / 2 | ∙ 12 ∙ SCS ≥ 1.5 ∙ CBW + 5 MHz ).

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.32.

6.2.3.4	Test description

6.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in tables 6.2.3.4.1-1 to 6.2.3.4.1-30. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Network signalling value NS_10 is not tested. The relaxation specified under NS_10 is for the purpose to control / mitigate self-desensitization of the UE’s own receiver, but no corresponding REFSENS requirements are specified. There is no need to test A-MPR requirements for its own.

Table 6.2.3.4.1-1: Test Configuration table for NS_06 and NS_35

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_06 and NS_35 |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq | ChBw | SCS | N/A for A-MPR testing. | Modulation (NOTE 2) |  | RB allocation (NOTE 1) |
| 1 | Low | Default | Default |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 3 | Default |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 4 | Low |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Right |
| 6 | Default |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 7 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 8 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Right |
| 9 | Default |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 10 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Right |
| 12 | Default |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 13 | Low |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 14 | High |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Right |
| 15 | Default |  |  |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 16 | Low |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 17 | High |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 18 | Default |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 19 | Low |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 20 | High |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 21 | Default |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 22 | Low |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 23 | High |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Right |
| 24 | Default |  |  |  | CP-OFDM 64 QAM |  | Outer Full |
| 25 | Low |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 26 | High |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Right |
| 27 | Default |  |  |  | CP-OFDM 256 QAM |  | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Void NOTE 4: This configuration is only applicable for NS_06 power class 1 UE. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-2: Test Configuration table for NS_04 (Power Class 1.5, 2 and 3)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | (See Freq column) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_04 |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq |  |  | N/A for A-MPR testing | Modulation (NOTE 2) |  | RB allocation (NOTE 1) |
| 1 (Note 3) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 2 (Note 3, 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 3 (Note 3) | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 4 (Note 3) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 5 (Note 3) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 6 (Note 3) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 7 (Note 4) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 8 (Note 4, 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 9 (Note 4) | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 10 (Note 4) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 11 (Note 4) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 12 (Note 4) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 13 | Low |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 14 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 15 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 16 | Low |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 17 | High |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Right |
| 18 | High |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 19 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 20 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 21 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 22 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 23 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Right |
| 24 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 25 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 26 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 27 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 28 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 29 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Right |
| 30 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 31 | Low |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 32 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 33 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 34 | Low |  |  |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 35 | High |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Right |
| 36 | High |  |  |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 37 | Low |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 38 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 39 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 40 | Low |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 41 | High |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 42 | High |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 43 | Low |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 44 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 45 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 46 | Low |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 47 | High |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 48 | High |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 49 | Low |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |

| A-MPR test parameters for NS_04 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |
| Test ID | Freq |  |  | N/A for A-MPR testing | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 50 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 51 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 52 | Low |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 53 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 54 | High |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 55 | Low |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 56 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 57 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 58 | Low |  |  |  | CP-OFDM 256 QAM | Outer Full |
| 59 | High |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 60 | High |  |  |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for band n41. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n41, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n41. NOTE 5: Only applicable for 10 MHz and 15 MHz channel bandwidth |  |  |  |  |  |  |

Table 6.2.3.4.1-2a: Additional test frequencies for NS_04 (SCS=15 kHz, ΔFRaster = 15 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Same values as for Low range of n41 CBW 5MHz in TS 38.508-1 [5] Table 4.3.1.1.1.41-1 SCS 15 kHz and ΔFRaster 15 kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | 52 |  |  | 2505 | 501000 | 2500.32 | 500064 | 0 | 15 | 6255 | 500430 | 2 | 0 | 0 (0) | 0 |
| 15 | 79 |  |  | 2512.5 | 502500 | 2505.39 | 501078 | 0 | 15 | 6270 | 501630 | 4 | 1 | 2 (4) | 5 |

zz

Table 6.2.3.4.1-2b: Additional test frequencies for NS_04 (SCS=30 kHz, ΔFRaster = 30 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 |  |  | 2505 | 501000 | 2500.68 | 500136 | 0 | 30 | 6261 | 500910 | 18 | 0 | 0 (0) | 0 |
| 15 | 38 |  |  | 2512.5 | 502500 | 2505.66 | 501132 | 0 | 30 | 6273 | 501870 | 6 | 0 | 0 (0) | 0 |

Table 6.2.3.4.1-2c: Additional test frequencies for NS_04 (SCS=60 kHz, ΔFRaster = 15 kHz, without CORESET#0)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 |  |  | 2505 | 501000 | 2501.04 | 500208 | 0 | 15 | - | 500568 |
| 15 | 18 |  |  | 2512.5 | 502500 | 2506.02 | 501204 | 0 | 15 | - | 501564 |

Table 6.2.3.4.1-2d: Additional test frequencies for NS_04 (SCS=15 kHz, ΔFRaster = 15 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 |  |  | 2508.5 | 501700 | 2506.25 | 501250 | 0 | 15 | 6269 | 501610 | 0 | 0 | 0 (0) | 0 |
| 10 | 52 |  |  | 2511 | 502200 | 2506.32 | 501264 | 0 | 15 | 6270 | 501630 | 2 | 0 | 0 (0) | 0 |
| 15 | 79 |  |  | 2513.505 | 502701 | 2506.395 | 501279 | 0 | 15 | 6273 | 501870 | 5 | 2 | 2 (4) | 6 |
| 20 | 106 |  |  | 2520 | 504000 | 2510.46 | 502092 | 0 | 15 | 6282 | 502590 | 10 | 1 | 1 (2) | 3 |
| 30 | 160 |  |  | 2535 | 507000 | 2520.6 | 504120 | 0 | 15 | 6306 | 504510 | 10 | 0 | 0 (0) | 0 |
| 40 | 216 |  |  | 2550 | 510000 | 2530.56 | 506112 | 0 | 15 | 6333 | 506670 | 6 | 1 | 2 (4) | 5 |
| 50 | 270 |  |  | 2565 | 513000 | 2540.7 | 508140 | 0 | 15 | 6357 | 508590 | 6 | 0 | 1 (2) | 2 |

Table 6.2.3.4.1-2e: Additional test frequencies for NS_04 (SCS=30 kHz, ΔFRaster = 30 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 |  |  | 2511 | 502200 | 2506.68 | 501336 | 0 | 30 | 6276 | 502110 | 18 | 0 | 0 (0) | 0 |
| 15 | 38 |  |  | 2513.52 | 502704 | 2506.68 | 501336 | 0 | 30 | 6276 | 502110 | 18 | 0 | 0 (0) | 0 |
| 20 | 51 |  |  | 2520 | 504000 | 2510.82 | 502164 | 0 | 30 | 6288 | 503070 | 14 | 0 | 2 (2) | 4 |
| 30 | 78 |  |  | 2535 | 507000 | 2520.96 | 504192 | 0 | 30 | 6312 | 504990 | 2 | 0 | 1 (1) | 2 |
| 40 | 106 |  |  | 2550 | 510000 | 2530.92 | 506184 | 0 | 30 | 6336 | 506910 | 2 | 0 | 0 (0) | 0 |
| 50 | 133 |  |  | 2565 | 513000 | 2541.06 | 508212 | 0 | 30 | 6363 | 509070 | 22 | 0 | 1 (1) | 2 |
| 60 | 162 |  |  | 2580 | 516000 | 2550.84 | 510168 | 0 | 30 | 6387 | 510990 | 10 | 0 | 1 (1) | 2 |
| 70 | 189 |  |  | 2595 | 519000 | 2560.98 | 512196 | 0 | 30 | 6414 | 513150 | 6 | 0 | 3 (3) | 6 |
| 80 | 217 |  |  | 2610 | 522000 | 2570.94 | 514188 | 0 | 30 | 6438 | 515070 | 6 | 0 | 2 (2) | 4 |
| 90 | 245 |  |  | 2625 | 525000 | 2580.9 | 516180 | 0 | 30 | 6462 | 516990 | 6 | 0 | 1 (1) | 2 |
| 100 | 273 |  |  | 2640 | 528000 | 2590.86 | 518172 | 0 | 30 | 6486 | 518910 | 6 | 0 | 0 (0) | 0 |

Table 6.2.3.4.1-2f: Additional test frequencies for NS_04 (SCS=60 kHz, ΔFRaster = 15 kHz, without CORESET#0)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 |  |  | 2511 | 502200 | 2507.04 | 501408 | 0 | 15 | - | 501768 |
| 15 | 18 |  |  | 2513.52 | 502704 | 2507.04 | 501408 | 0 | 15 | - | 501768 |
| 20 | 24 |  |  | 2520 | 504000 | 2511.36 | 502272 | 0 | 15 | - | 502632 |
| 30 | 38 |  |  | 2535 | 507000 | 2521.32 | 504264 | 0 | 15 | - | 504624 |
| 40 | 51 |  |  | 2550 | 510000 | 2531.64 | 506328 | 0 | 15 | - | 506688 |
| 50 | 65 |  |  | 2550 | 510000 | 2526.6 | 505320 | 0 | 15 | - | 505680 |
| 60 | 79 |  |  | 2580 | 516000 | 2551.56 | 510312 | 0 | 15 | - | 510672 |
| 70 | 93 |  |  | 2595 | 519000 | 2561.52 | 512304 | 0 | 15 | - | 512664 |
| 80 | 107 |  |  | 2610 | 522000 | 2571.48 | 514296 | 0 | 15 | - | 514656 |
| 90 | 121 |  |  | 2625 | 525000 | 2581.44 | 516288 | 0 | 15 | - | 516648 |
| 100 | 135 |  |  | 2640 | 528000 | 2591.4 | 518280 | 0 | 15 | - | 518640 |

Table 6.2.3.4.1-2g: Test Configuration table for NS_04 (Power Class 1)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Low, High |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Lowest, Highest |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |
| A-MPR test parameters for NS_04 for PC1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq | Channel BW |  | SCS | N/A for A-MPR testing | Modulation (NOTE 2) | RB allocation (NOTE 1) |  |
| 1 (Note 3) | Low | Default |  | Default |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Left |  |
| 2 (Note 3) | Low | Default |  | Default |  | DFT-s-OFDM PI/2 BPSK | Inner_1RB_Left |  |
| 3 (Note 3) | High | Default |  | Default |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Right |  |
| 4 (Note 3) | High | Default |  | Default |  | DFT-s-OFDM PI/2 BPSK | Outer Full |  |
| 5 (Note 3, 4) | Low | 10 MHz |  | Default |  | DFT-s-OFDM PI/2 BPSK | Outer Full |  |
| 6 (Note 3, 4) | Low | 100 MHz |  | 30 kHz |  | DFT-s-OFDM PI/2 BPSK | 135@33 |  |
| 7 (Note 3, 4) | Low | 100 MHz |  | 60 kHz |  | DFT-s-OFDM PI/2 BPSK | 64@16 |  |
| 8 | Low | Default |  | Default |  | DFT-s-OFDM QPSK | Edge_1RB_Left |  |
| 9 | Low | Default |  | Default |  | DFT-s-OFDM QPSK | Inner_1RB_Left |  |
| 10 | High | Default |  | Default |  | DFT-s-OFDM QPSK | Edge_1RB_Right |  |
| 11 | High | Default |  | Default |  | DFT-s-OFDM QPSK | Outer Full |  |
| 12 (Note 4) | Low | 10 MHz |  | Default |  | DFT-s-OFDM QPSK | Outer Full |  |
| 13 (Note 4) | Low | 100 MHz |  | 30 kHz |  | DFT-s-OFDM QPSK | 135@33 |  |
| 14 (Note 4) | Low | 100 MHz |  | 60 kHz |  | DFT-s-OFDM QPSK | 64@16 |  |
| 15 | Low | Default |  | Default |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |  |
| 16 | Low | Default |  | Default |  | DFT-s-OFDM 16 QAM | Inner_1RB_Left |  |
| 17 | High | Default |  | Default |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |  |
| 18 | High | 100 MHz |  | 30 kHz |  | DFT-s-OFDM 16 QAM | 50@200 |  |
| 19 (Note 4) | Low | 10 MHz |  | Default |  | DFT-s-OFDM 16 QAM | Outer Full |  |
| 20 (Note 4) | Low | 100 MHz |  | 30 kHz |  | DFT-s-OFDM 16 QAM | 135@33 |  |
| 21 (Note 4) | Low | 100 MHz |  | 60 kHz |  | DFT-s-OFDM 16 QAM | 64@16 |  |
| 22 | Low | Default |  | Default |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |  |
| 23 | Low | Default |  | Default |  | DFT-s-OFDM 64 QAM | Inner_1RB_Left |  |
| 24 | High | Default |  | Default |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |  |
| 25 | High | 100 MHz |  | 30 kHz |  | DFT-s-OFDM 16 QAM | 50@200 |  |
| 26 (Note 4) | Low | 10 MHz |  | Default |  | DFT-s-OFDM 64 QAM | Outer Full |  |
| 27 (Note 4) | Low | 100 MHz |  | 30 kHz |  | DFT-s-OFDM 64 QAM | 135@33 |  |
| 28 (Note 4) | Low | 100 MHz |  | 60 kHz |  | DFT-s-OFDM 64 QAM | 64@16 |  |
| 29 | Low | Default |  | Default |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |  |
| 30 | Low | Default |  | Default |  | DFT-s-OFDM 256 QAM | Outer Full |  |
| 31 | High | Default |  | Default |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |  |
| 32 | High | Default |  | Default |  | DFT-s-OFDM 256 QAM | Outer Full |  |
| 33 (Note 4) | Low | 10 MHz |  | Default |  | DFT-s-OFDM 256 QAM | Outer Full |  |
| 34 (Note 4) | Low | 100 MHz |  | 30 kHz |  | DFT-s-OFDM 256 QAM | 135@33 |  |
| 35 (Note 4) | Low | 100 MHz |  | 60 kHz |  | DFT-s-OFDM 256 QAM | 64@16 |  |
| 36 | Low | Default |  | Default |  | CP-OFDM QPSK | Edge_1RB_Left |  |
| 37 | Low | Default |  | Default |  | CP-OFDM QPSK | Outer Full |  |
| 38 | High | 100 MHz |  | 30 kHz |  | CP-OFDM 16 QAM | 50@200 |  |
| 39 (Note 4) | Low | 10 MHz |  | Default |  | CP-OFDM QPSK | Outer Full |  |
| 40 (Note 4) | Low | 100 MHz |  | 30 kHz |  | CP-OFDM QPSK | 135@33 |  |
| 41 (Note 4) | Low | 100 MHz |  | 60 kHz |  | CP-OFDM QPSK | 64@16 |  |
| 42 | Low | Default |  | Default |  | CP-OFDM 16 QAM | Edge_1RB_Left |  |
| 43 | Low | Default |  | Default |  | CP-OFDM 16 QAM | Outer Full |  |
| 44 | High | 100 MHz |  | 30 kHz |  | CP-OFDM 16 QAM | 50@200 |  |
| 45 (Note 4) | Low | 10 MHz |  | Default |  | CP-OFDM 16 QAM | Outer Full |  |
| 46 (Note 4) | Low | 100 MHz |  | 30 kHz |  | CP-OFDM 16 QAM | 135@33 |  |
| 47 (Note 4) | Low | 100 MHz |  | 60 kHz |  | CP-OFDM 16 QAM | 64@16 |  |
| 48 | Low | Default |  | Default |  | CP-OFDM 64 QAM | Edge_1RB_Left |  |
| 49 | Low | Default |  | Default |  | CP-OFDM 64 QAM | Outer Full |  |
| 50 | High | 100 MHz |  | 30 kHz |  | CP-OFDM 16 QAM | 50@200 |  |
| 51 (Note 4) | Low | 10 MHz |  | Default |  | CP-OFDM 64 QAM | Outer Full |  |
| 52 (Note 4) | Low | 100 MHz |  | 30 kHz |  | CP-OFDM 64 QAM | 135@33 |  |
| 53 (Note 4) | Low | 100 MHz |  | 60 kHz |  | CP-OFDM 64 QAM | 64@16 |  |
| 54 | Low | Default |  | Default |  | CP-OFDM 256 QAM | Edge_1RB_Left |  |
| 55 | Low | Default |  | Default |  | CP-OFDM 256 QAM | Outer Full |  |
| 56 | High | Default |  | Default |  | CP-OFDM 256 QAM | 50@200 |  |
| 57 (Note 4) | Low | 10 MHz |  | Default |  | CP-OFDM 256 QAM | Outer Full |  |
| 58 (Note 4) | Low | 100 MHz |  | 30 kHz |  | CP-OFDM 256 QAM | 135@33 |  |
| 59 (Note 4) | Low | 100 MHz |  | 60 kHz |  | CP-OFDM 256 QAM | 64@16 |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for band n41. NOTE 4: Applicable for requirement in regrowth region A-MPRregrowth |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-3: Test Configuration table for NS_03, NS_03U and NS_100

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |
| A-MPR test parameters for NS_03, NS_03U and NS_100 |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
|  |  |  |  | N/A for A-MPR test cases | DFT-s-OFDM |  |  |
| 1 | Low | Default | Default |  |  | PI/2 BPSK | Edge_1RB_Left |
| 2 | High | Default | Default |  |  | PI/2 BPSK | Edge_1RB_Right |
| 3 | Default | Default | Default |  |  | PI/2 BPSK | Outer_Full |
| 4 | Low | Default | Default |  |  | QPSK | Edge_1RB_Left |
| 5 | High | Default | Default |  |  | QPSK | Edge_1RB_Right |
| 6 | Default | Default | Default |  |  | QPSK | Outer_Full |
| 7 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |
| 8 | High | Default | Default |  |  | 16 QAM | Edge_1RB_Right |
| 9 | Default | Default | Default |  |  | 16 QAM | Outer_Full |
| 10 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |
| 11 | High | Default | Default |  |  | 64 QAM | Edge_1RB_Right |
| 12 | Default | Default | Default |  |  | 64 QAM | Outer_Full |
| 13 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |
| 14 | High | Default | Default |  |  | 256 QAM | Edge_1RB_Right |
| 15 | Default | Default | Default |  |  | 256 QAM | Outer_Full |
| 16 | Low | Default | Default |  | CP-OFDM | QPSK | Edge_1RB_Left |
| 17 | High | Default | Default |  |  | QPSK | Edge_1RB_Right |
| 18 | Default | Default | Default |  |  | QPSK | Outer_Full |
| 19 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |
| 20 | High | Default | Default |  |  | 16 QAM | Edge_1RB_Right |
| 21 | Default | Default | Default |  |  | 16 QAM | Outer_Full |
| 22 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |
| 23 | High | Default | Default |  |  | 64 QAM | Edge_1RB_Right |
| 24 | Default | Default | Default |  |  | 64 QAM | Outer_Full |
| 25 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |
| 26 | High | Default | Default |  |  | 256 QAM | Edge_1RB_Right |
| 27 | Default | Default | Default |  |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Void. NOTE 4: Void |  |  |  |  |  |  |  |

Table 6.2.3.4.1-4: Test Configuration table for NS_05

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest unless otherwise specified in test parameters. |  |  |  |  |
| A-MPR test parameters for NS_05 |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |  |
| 1 | 1922.5 | 5 | 15 | N/A for A-MPR testing | A3 | DFT-s-OFDM | PI/2 BPSK |  | Outer_Full |  |  |  |
| 2 | 1925 | 10 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 3 | 1925 | 10 | Default |  | A7 |  | PI/2 BPSK |  | 40@10 | 18@5 | 8@3 |  |
| 4 | 1925 | 10 | Default |  | A2 |  | PI/2 BPSK |  | 6@40 | 3@20 | 1@10 |  |
| 5 | 1935 | 10 | Default |  | A4 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 6 | 1927.5 | 15 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 7 | 1927.5 | 15 | Default |  | A7 |  | PI/2 BPSK |  | 60@19 | 27@10 | 12@5 |  |
| 8 | 1927.5 | 15 | Default |  | A2 |  | PI/2 BPSK |  | 6@56 | 3@28 | 1@14 |  |
| 9 | 1932.5 | 15 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 10 | 1932.5 | 15 | Default |  | A2 |  | PI/2 BPSK |  | 6@68 | 3@34 | 1@17 |  |
| 11 | 1942.5 | 15 | Default |  | A5 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 12 | 1930 | 20 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 13 | 1930 | 20 | Default |  | A7 |  | PI/2 BPSK |  | 72@28 | 36@14 | 16@7 |  |
| 14 | 1930 | 20 | Default |  | A2 |  | PI/2 BPSK |  | 6@76 | 3@38 | 1@19 |  |
| 15 | 1950 | 20 | Default |  | A6 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 16 | 1922.5 | 5 | 15 |  | A3 |  | QPSK |  | Outer_Full |  |  |  |
| 17 | 1925 | 10 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 18 | 1925 | 10 | Default |  | A7 |  | QPSK |  | 40@10 | 18@5 |  | 8@3 |
| 19 | 1925 | 10 | Default |  | A2 |  | QPSK |  | 6@40 | 3@20 |  | 1@10 |
| 20 | 1927.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 21 | 1927.5 | 15 | Default |  | A7 |  | QPSK |  | 60@19 | 27@10 |  | 12@5 |
| 22 | 1927.5 | 15 | Default |  | A2 |  | QPSK |  | 6@56 | 3@28 |  | 1@14 |
| 23 | 1932.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 24 | 1932.5 | 15 | Default |  | A2 |  | QPSK |  | 6@68 | 3@34 |  | 1@17 |
| 25 | 1942.5 | 15 | Default |  | A5 |  | QPSK |  | Outer_Full |  |  |  |
| 26 | 1930 | 20 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 27 | 1930 | 20 | Default |  | A7 |  | QPSK |  | 72@28 | 36@14 |  | 16@7 |
| 28 | 1930 | 20 | Default |  | A2 |  | QPSK |  | 6@76 | 3@38 |  | 1@19 |
| 29 | 1950 | 20 | Default |  | A6 |  | QPSK |  | Outer_Full |  |  |  |
| 30 | 1922.5 | 5 | 15 |  | A3 |  | 16 QAM |  | Outer_Full |  |  |  |
| 31 | 1925 | 10 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 32 | 1925 | 10 | Default |  | A7 |  | 16 QAM |  | 40@10 | 18@5 |  | 8@3 |
| 33 | 1925 | 10 | Default |  | A2 |  | 16 QAM |  | 6@40 | 3@20 |  | 1@10 |
| 34 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 35 | 1927.5 | 15 | Default |  | A7 |  | 16 QAM |  | 60@19 | 27@10 |  | 12@5 |
| 36 | 1927.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@56 | 3@28 |  | 1@14 |
| 37 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 38 | 1932.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@68 | 3@34 |  | 1@17 |
| 39 | 1930 | 20 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 40 | 1930 | 20 | Default |  | A7 |  | 16 QAM |  | 72@28 | 36@14 |  | 16@7 |
| 41 | 1930 | 20 | Default |  | A2 |  | 16 QAM |  | 6@76 | 3@38 |  | 1@19 |
| 42 | 1922.5 | 5 | 15 |  | A3 |  | 64 QAM |  | Outer_Full |  |  |  |
| 43 | 1925 | 10 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |
| 44 | 1925 | 10 | Default |  | A7 |  | 64 QAM |  | 40@10 | 18@5 |  | 8@3 |
| 45 | 1925 | 10 | Default |  | A2 |  | 64 QAM |  | 6@40 | 3@20 |  | 1@10 |
| 46 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |
| 47 | 1927.5 | 15 | Default |  | A7 |  | 64 QAM |  | 60@19 | 27@10 |  | 12@5 |
| 48 | 1927.5 | 15 | Default |  | A2 |  | 64 QAM |  | 6@56 | 3@28 |  | 1@14 |
| 49 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |
| 50 | 1932.5 | 15 | Default |  | A2 |  | 64 QAM |  | 6@68 | 3@34 |  | 1@17 |

| A-MPR test parameters for NS_05 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |  |
| 51 | 1930 | 20 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 52 | 1930 | 20 | Default |  | A7 |  | 64 QAM | 72@28 | 36@14 |  | 16@7 |
| 53 | 1930 | 20 | Default |  | A2 |  | 64 QAM | 6@76 | 3@38 |  | 1@19 |
| 54 | 1922.5 | 5 | 15 |  | A3 |  | 256 QAM | Outer_Full |  |  |  |
| 55 | 1925 | 10 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 56 | 1925 | 10 | Default |  | A7 |  | 256 QAM | 40@10 | 18@5 |  | 8@3 |
| 57 | 1925 | 10 | Default |  | A2 |  | 256 QAM | 6@40 | 3@20 |  | 1@10 |
| 58 | 1927.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 59 | 1927.5 | 15 | Default |  | A7 |  | 256 QAM | 60@19 | 27@10 |  | 12@5 |
| 60 | 1927.5 | 15 | Default |  | A2 |  | 256 QAM | 6@56 | 3@28 |  | 1@14 |
| 61 | 1932.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 62 | 1932.5 | 15 | Default |  | A2 |  | 256 QAM | 6@68 | 3@34 |  | 1@17 |
| 63 | 1930 | 20 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 64 | 1930 | 20 | Default |  | A7 |  | 256 QAM | 72@28 | 36@14 |  | 16@7 |
| 65 | 1930 | 20 | Default |  | A2 |  | 256 QAM | 6@76 | 3@38 |  | 1@19 |
| 66 | 1922.5 | 5 | 15 |  | A3 | CP-OFDM | QPSK | Outer_Full |  |  |  |
| 67 | 1925 | 10 | Default |  | A1 |  | QPSK | Outer_Full |  |  |  |
| 68 | 1925 | 10 | Default |  | A7 |  | QPSK | 42@10 | 18@5 |  | 8@3 |
| 69 | 1925 | 10 | Default |  | A2 |  | QPSK | 6@40 | 3@20 |  | 1@10 |
| 70 | 1935 | 10 | Default |  | A4 |  | QPSK | Outer_Full |  |  |  |
| 71 | 1927.5 | 15 | Default |  | A1 |  | QPSK | Outer_Full |  |  |  |
| 72 | 1927.5 | 15 | Default |  | A7 |  | QPSK | 60@19 | 28@10 |  | 12@5 |
| 73 | 1927.5 | 15 | Default |  | A2 |  | QPSK | 6@56 | 3@28 |  | 1@14 |
| 74 | 1932.5 | 15 | Default |  | A1 |  | QPSK | Outer_Full |  |  |  |
| 75 | 1932.5 | 15 | Default |  | A2 |  | QPSK | 6@68 | 3@34 |  | 1@17 |
| 76 | 1942.5 | 15 | Default |  | A5 |  | QPSK | Outer_Full |  |  |  |
| 77 | 1930 | 20 | Default |  | A1 |  | QPSK | Outer_Full |  |  |  |
| 78 | 1930 | 20 | Default |  | A7 |  | QPSK | 78@28 | 37@14 |  | 17@7 |
| 79 | 1930 | 20 | Default |  | A2 |  | QPSK | 6@76 | 3@38 |  | 1@19 |
| 80 | 1950 | 20 | Default |  | A6 |  | QPSK | Outer_Full |  |  |  |
| 81 | 1922.5 | 5 | 15 |  | A3 |  | 16 QAM | Outer_Full |  |  |  |
| 82 | 1925 | 10 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 83 | 1925 | 10 | Default |  | A7 |  | 16 QAM | 42@10 | 18@5 |  | 8@3 |
| 84 | 1925 | 10 | Default |  | A2 |  | 16 QAM | 6@40 | 3@20 |  | 1@10 |
| 85 | 1935 | 10 | Default |  | A4 |  | 16 QAM | Outer_Full |  |  |  |
| 86 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 87 | 1927.5 | 15 | Default |  | A7 |  | 16 QAM | 60@19 | 28@10 |  | 12@5 |
| 88 | 1927.5 | 15 | Default |  | A2 |  | 16 QAM | 6@56 | 3@28 |  | 1@14 |
| 89 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 90 | 1932.5 | 15 | Default |  | A2 |  | 16 QAM | 6@68 | 3@34 |  | 1@17 |
| 91 | 1942.5 | 15 | Default |  | A5 |  | 16 QAM | Outer_Full |  |  |  |
| 92 | 1930 | 20 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 93 | 1930 | 20 | Default |  | A7 |  | 16 QAM | 78@28 | 37@14 |  | 17@7 |
| 94 | 1930 | 20 | Default |  | A2 |  | 16 QAM | 6@76 | 3@38 |  | 1@19 |
| 95 | 1950 | 20 | Default |  | A6 |  | 16 QAM | Outer_Full |  |  |  |
| 96 | 1922.5 | 5 | 15 |  | A3 |  | 64 QAM | Outer_Full |  |  |  |
| 97 | 1925 | 10 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 98 | 1925 | 10 | Default |  | A7 |  | 64 QAM | 42@10 | 18@5 |  | 8@3 |
| 99 | 1925 | 10 | Default |  | A2 |  | 64 QAM | 6@40 | 3@20 |  | 1@10 |
| 100 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 101 | 1927.5 | 15 | Default |  | A7 |  | 64 QAM | 60@19 | 28@10 |  | 12@5 |
| 102 | 1927.5 | 15 | Default |  | A2 |  | 64 QAM | 6@56 | 3@28 |  | 1@14 |
| 103 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 104 | 1932.5 | 15 | Default |  | A2 |  | 64 QAM | 6@68 | 3@34 |  | 1@17 |
| 105 | 1930 | 20 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 106 | 1930 | 20 | Default |  | A7 |  | 64 QAM | 78@28 | 37@14 |  | 17@7 |
| 107 | 1930 | 20 | Default |  | A2 |  | 64 QAM | 6@76 | 3@38 |  | 1@19 |

| A-MPR test parameters for NS_05 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |  |
| 108 | 1922.5 | 5 | 15 |  | A3 |  | 256 QAM | Outer_Full |  |  |  |
| 109 | 1925 | 10 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 110 | 1925 | 10 | Default |  | A7 |  | 256 QAM | 42@10 | 18@5 |  | 8@3 |
| 111 | 1927.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 112 | 1927.5 | 15 | Default |  | A7 |  | 256 QAM | 60@19 | 28@10 |  | 12@5 |
| 113 | 1932.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 114 | 1930 | 20 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 115 | 1930 | 20 | Default |  | A7 |  | 256 QAM | 78@28 | 37@14 |  | 17@7 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-4a: Test Configuration table for NS_05 and NS_05U power class 2

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest unless otherwise specified in test parameters. |  |  |  |
| A-MPR test parameters for NS_05 and NS_05U |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | 1922.5 | 5 | 15 | N/A for A-MPR testing | A3 | DFT-s-OFDM | PI/2 BPSK |  | Outer_Full |  |  |
| 2 | 1922.5 | 5 | Default |  | A4 |  | PI/2 BPSK |  | 8@1 | 4@1 |  |
| 3 | 1925 | 10 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |
| 4 | 1925 | 10 | Default |  | A7 |  | PI/2 BPSK |  | 30@11 | 15@6 | 6@3 |
| 5 | 1925 | 10 | Default |  | A1 |  | PI/2 BPSK |  | 6@40 | 3@20 | 1@10 |
| 6 | 1925 | 10 | Default |  | A8 |  | PI/2 BPSK |  | 6@11 | 3@6 | 1@3 |
| 7 | 1935 | 10 | Default |  | A4 |  | PI/2 BPSK |  | Outer_Full |  |  |
| 8 | 1927.5 | 15 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |
| 9 | 1927.5 | 15 | Default |  | A7 |  | PI/2 BPSK |  | 54@20 | 27@10 | 12@5 |
| 10 | 1927.5 | 15 | Default |  | A1 |  | PI/2 BPSK |  | 6@56 | 3@28 | 1@14 |
| 11 | 1927.5 | 15 | Default |  | A8 |  | PI/2 BPSK |  | 6@20 | 3@10 | 1@5 |
| 12 | 1932.5 | 15 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |
| 13 | 1932.5 | 15 | Default |  | A7 |  | PI/2 BPSK |  | 64@11 | 30@6 | 15@3 |
| 14 | 1932.5 | 15 | Default |  | A1 |  | PI/2 BPSK |  | 6@68 | 3@34 | 1@17 |
| 15 | 1942.5 | 15 | Default |  | A5 |  | PI/2 BPSK |  | Outer_Full |  |  |
| 16 | 1930 | 20 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |
| 17 | 1930 | 20 | Default |  | A7 |  | PI/2 BPSK |  | 72@28 | 36@14 | 16@7 |
| 18 | 1930 | 20 | Default |  | A1 |  | PI/2 BPSK |  | 6@76 | 3@38 | 1@19 |
| 19 | 1930 | 20 | Default |  | A8 |  | PI/2 BPSK |  | 6@28 | 3@14 | 1@7 |
| 20 | 1950 | 20 | Default |  | A6 |  | PI/2 BPSK |  | Outer_Full |  |  |
| 21 | 1922.5 | 5 | 15 |  | A3 |  | QPSK |  | Outer_Full |  |  |
| 22 | 1922.5 | 5 | Default |  | A4 |  | QPSK |  | 8@1 | 4@1 |  |
| 23 | 1925 | 10 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |
| 24 | 1925 | 10 | Default |  | A7 |  | QPSK |  | 30@11 | 15@6 | 6@3 |
| 25 | 1925 | 10 | Default |  | A1 |  | QPSK |  | 6@40 | 3@20 | 1@10 |
| 26 | 1925 | 10 | Default |  | A8 |  | QPSK |  | 6@11 | 3@6 | 1@3 |
| 27 | 1935 | 10 | Default |  | A4 |  | QPSK |  | Outer_Full |  |  |
| 28 | 1927.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |
| 29 | 1927.5 | 15 | Default |  | A7 |  | QPSK |  | 54@20 | 27@10 | 12@5 |
| 30 | 1927.5 | 15 | Default |  | A1 |  | QPSK |  | 6@56 | 3@28 | 1@14 |
| 31 | 1927.5 | 15 | Default |  | A8 |  | QPSK |  | 6@20 | 3@10 | 1@5 |
| 32 | 1932.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |
| 33 | 1932.5 | 15 | Default |  | A7 |  | QPSK |  | 64@11 | 30@6 | 15@3 |
| 34 | 1932.5 | 15 | Default |  | A1 |  | QPSK |  | 6@68 | 3@34 | 1@17 |
| 35 | 1942.5 | 15 | Default |  | A5 |  | QPSK |  | Outer_Full |  |  |
| 36 | 1930 | 20 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |
| 37 | 1930 | 20 | Default |  | A7 |  | QPSK |  | 72@28 | 36@14 | 16@7 |
| 38 | 1930 | 20 | Default |  | A1 |  | QPSK |  | 6@76 | 3@38 | 1@19 |
| 39 | 1930 | 20 | Default |  | A8 |  | QPSK |  | 6@28 | 3@14 | 1@7 |
| 40 | 1950 | 20 | Default |  | A6 |  | QPSK |  | Outer_Full |  |  |
| 41 | 1922.5 | 5 | 15 |  | A3 |  | 16 QAM |  | Outer_Full |  |  |
| 42 | 1922.5 | 5 | Default |  | A4 |  | 16 QAM |  | 8@1 | 4@1 |  |
| 43 | 1925 | 10 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |
| 44 | 1925 | 10 | Default |  | A7 |  | 16 QAM |  | 30@11 | 15@6 | 6@3 |
| 45 | 1925 | 10 | Default |  | A1 |  | 16 QAM |  | 6@40 | 3@20 | 1@10 |
| 46 | 1925 | 10 | Default |  | A8 |  | 16 QAM |  | 6@11 | 3@6 | 1@3 |
| 47 | 1935 | 10 | Default |  | A4 |  | 16 QAM |  | Outer_Full |  |  |
| 48 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |
| 49 | 1927.5 | 15 | Default |  | A7 |  | 16 QAM |  | 54@20 | 27@10 | 12@5 |
| 50 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM |  | 6@56 | 3@28 | 1@14 |

| A-MPR test parameters for NS_05 and NS_05U |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 51 | 1927.5 | 15 | Default |  | A8 |  | 16 QAM | 6@20 | 3@10 | 1@5 |
| 52 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |
| 53 | 1932.5 | 15 | Default |  | A7 |  | 16 QAM | 64@11 | 30@6 | 15@3 |
| 54 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM | 6@68 | 3@34 | 1@17 |
| 55 | 1942.5 | 15 | Default |  | A5 |  | 16 QAM | Outer_Full |  |  |
| 56 | 1930 | 20 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |
| 57 | 1930 | 20 | Default |  | A7 |  | 16 QAM | 72@28 | 36@14 | 16@7 |
| 58 | 1930 | 20 | Default |  | A1 |  | 16 QAM | 6@76 | 3@38 | 1@19 |
| 59 | 1930 | 20 | Default |  | A8 |  | 16 QAM | 6@28 | 3@14 | 1@7 |
| 60 | 1950 | 20 | Default |  | A6 |  | 16 QAM | Outer_Full |  |  |
| 61 | 1922.5 | 5 | 15 |  | A3 |  | 64 QAM | Outer_Full |  |  |
| 62 | 1922.5 | 5 | Default |  | A4 |  | 64 QAM | 8@1 | 4@1 |  |
| 63 | 1925 | 10 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |
| 64 | 1925 | 10 | Default |  | A7 |  | 64 QAM | 30@11 | 15@6 | 6@3 |
| 65 | 1925 | 10 | Default |  | A1 |  | 64 QAM | 6@40 | 3@20 | 1@10 |
| 66 | 1925 | 10 | Default |  | A8 |  | 64 QAM | 6@11 | 3@6 | 1@3 |
| 67 | 1935 | 10 | Default |  | A4 |  | 64 QAM | Outer_Full |  |  |
| 68 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |
| 69 | 1927.5 | 15 | Default |  | A7 |  | 64 QAM | 54@20 | 27@10 | 12@5 |
| 70 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM | 6@56 | 3@28 | 1@14 |
| 71 | 1927.5 | 15 | Default |  | A8 |  | 64 QAM | 6@20 | 3@10 | 1@5 |
| 72 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |
| 73 | 1932.5 | 15 | Default |  | A7 |  | 64 QAM | 64@11 | 30@6 | 15@3 |
| 74 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM | 6@68 | 3@34 | 1@17 |
| 75 | 1942.5 | 15 | Default |  | A5 |  | 64 QAM | Outer_Full |  |  |
| 76 | 1930 | 20 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |
| 77 | 1930 | 20 | Default |  | A7 |  | 64 QAM | 72@28 | 36@14 | 16@7 |
| 78 | 1930 | 20 | Default |  | A1 |  | 64 QAM | 6@76 | 3@38 | 1@19 |
| 79 | 1930 | 20 | Default |  | A8 |  | 64 QAM | 6@28 | 3@14 | 1@7 |
| 80 | 1922.5 | 5 | 15 |  | A3 |  | 256 QAM | Outer_Full |  |  |
| 81 | 1925 | 10 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |
| 82 | 1925 | 10 | Default |  | A7 |  | 256 QAM | 30@11 | 15@6 | 6@3 |
| 83 | 1925 | 10 | Default |  | A1 |  | 256 QAM | 6@40 | 3@20 | 1@10 |
| 84 | 1927.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |
| 85 | 1927.5 | 15 | Default |  | A7 |  | 256 QAM | 54@20 | 27@10 | 12@5 |
| 86 | 1927.5 | 15 | Default |  | A1 |  | 256 QAM | 6@56 | 3@28 | 1@14 |
| 87 | 1932.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |
| 88 | 1932.5 | 15 | Default |  | A7 |  | 256 QAM | 64@11 | 30@6 | 15@3 |
| 89 | 1932.5 | 15 | Default |  | A1 |  | 256 QAM | 6@68 | 3@34 | 1@17 |
| 90 | 1930 | 20 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |
| 91 | 1930 | 20 | Default |  | A7 |  | 256 QAM | 72@28 | 36@14 | 16@7 |
| 92 | 1930 | 20 | Default |  | A1 |  | 256 QAM | 6@76 | 3@38 | 1@19 |
| 93 | 1922.5 | 5 | 15 |  | A3 |  | QPSK | Outer_Full |  |  |
| 94 | 1922.5 | 5 | Default |  | A4 |  | QPSK | 8@1 | 4@1 |  |
| 95 | 1925 | 10 | Default |  | A1 |  | QPSK | Outer_Full |  |  |
| 96 | 1925 | 10 | Default |  | A7 |  | QPSK | 30@11 | 15@6 | 6@3 |
| 97 | 1925 | 10 | Default |  | A1 |  | QPSK | 6@40 | 3@20 | 1@10 |
| 98 | 1925 | 10 | Default |  | A8 |  | QPSK | 6@11 | 3@6 | 1@3 |
| 99 | 1935 | 10 | Default |  | A4 |  | QPSK | Outer_Full |  |  |
| 100 | 1927.5 | 15 | Default |  | A1 |  | QPSK | Outer_Full |  |  |
| 101 | 1927.5 | 15 | Default |  | A7 |  | QPSK | 52@20 | 26@10 | 12@5 |
| 102 | 1927.5 | 15 | Default |  | A1 |  | QPSK | 6@56 | 3@28 | 1@14 |
| 103 | 1927.5 | 15 | Default |  | A8 |  | QPSK | 6@20 | 3@10 | 1@5 |
| 104 | 1932.5 | 15 | Default |  | A1 |  | QPSK | Outer_Full |  |  |
| 105 | 1932.5 | 15 | Default |  | A7 |  | QPSK | 62@11 | 30@6 | 14@3 |
| 106 | 1932.5 | 15 | Default |  | A1 |  | QPSK | 6@68 | 3@34 | 1@17 |
| 107 | 1942.5 | 15 | Default |  | A5 |  | QPSK | Outer_Full |  |  |

| A-MPR test parameters for NS_05 and NS_05U |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |  |
| 108 | 1930 | 20 | Default |  | A1 | CP-OFDM | QPSK | Outer_Full |  |  |  |
| 109 | 1930 | 20 | Default |  | A7 |  | QPSK | 72@28 | 36@14 |  | 16@7 |
| 110 | 1930 | 20 | Default |  | A1 |  | QPSK | 6@76 | 3@38 |  | 1@19 |
| 111 | 1930 | 20 | Default |  | A8 |  | QPSK | 6@28 | 3@14 |  | 1@7 |
| 112 | 1950 | 20 | Default |  | A6 |  | QPSK | Outer_Full |  |  |  |
| 113 | 1922.5 | 5 | 15 |  | A3 |  | 16 QAM | Outer_Full |  |  |  |
| 114 | 1922.5 | 5 | Default |  | A4 |  | 16 QAM | 8@1 | 4@1 |  |  |
| 115 | 1925 | 10 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 116 | 1925 | 10 | Default |  | A7 |  | 16 QAM | 30@11 | 15@6 |  | 6@3 |
| 117 | 1925 | 10 | Default |  | A1 |  | 16 QAM | 6@40 | 3@20 |  | 1@10 |
| 118 | 1925 | 10 | Default |  | A8 |  | 16 QAM | 6@11 | 3@6 |  | 1@3 |
| 119 | 1935 | 10 | Default |  | A4 |  | 16 QAM | Outer_Full |  |  |  |
| 120 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 121 | 1927.5 | 15 | Default |  | A7 |  | 16 QAM | 52@20 | 26@10 |  | 12@5 |
| 122 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM | 6@56 | 3@28 |  | 1@14 |
| 123 | 1927.5 | 15 | Default |  | A8 |  | 16 QAM | 6@20 | 3@10 |  | 1@5 |
| 124 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 125 | 1932.5 | 15 | Default |  | A7 |  | 16 QAM | 62@11 | 30@6 |  | 14@3 |
| 126 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM | 6@68 | 3@34 |  | 1@17 |
| 127 | 1942.5 | 15 | Default |  | A5 |  | 16 QAM | Outer_Full |  |  |  |
| 128 | 1930 | 20 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 129 | 1930 | 20 | Default |  | A7 |  | 16 QAM | 72@28 | 36@14 |  | 16@7 |
| 130 | 1930 | 20 | Default |  | A1 |  | 16 QAM | 6@76 | 3@38 |  | 1@19 |
| 131 | 1930 | 20 | Default |  | A8 |  | 16 QAM | 6@28 | 3@14 |  | 1@7 |
| 132 | 1950 | 20 | Default |  | A6 |  | 16 QAM | Outer_Full |  |  |  |
| 133 | 1922.5 | 5 | 15 |  | A3 |  | 64 QAM | Outer_Full |  |  |  |
| 134 | 1922.5 | 5 | Default |  | A4 |  | 64 QAM | 8@1 | 4@1 |  |  |
| 135 | 1925 | 10 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 136 | 1925 | 10 | Default |  | A7 |  | 64 QAM | 30@11 | 15@6 |  | 6@3 |
| 137 | 1925 | 10 | Default |  | A1 |  | 64 QAM | 6@40 | 3@20 |  | 1@10 |
| 138 | 1935 | 10 | Default |  | A4 |  | 64 QAM | Outer_Full |  |  |  |
| 139 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 140 | 1927.5 | 15 | Default |  | A7 |  | 64 QAM | 52@20 | 26@10 |  | 12@5 |
| 141 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM | 6@56 | 3@28 |  | 1@14 |
| 142 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 143 | 1932.5 | 15 | Default |  | A7 |  | 64 QAM | 62@11 | 30@6 |  | 14@3 |
| 144 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM | 6@68 | 3@34 |  | 1@17 |
| 145 | 1942.5 | 15 | Default |  | A5 |  | 64 QAM | Outer_Full |  |  |  |
| 146 | 1930 | 20 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 147 | 1930 | 20 | Default |  | A7 |  | 64 QAM | 72@28 | 36@14 |  | 16@7 |
| 148 | 1930 | 20 | Default |  | A1 |  | 64 QAM | 6@76 | 3@38 |  | 1@19 |
| 149 | 1950 | 20 | Default |  | A6 |  | 64 QAM | Outer_Full |  |  |  |
| 150 | 1922.5 | 5 | 15 |  | A3 |  | 256 QAM | Outer_Full |  |  |  |
| 151 | 1925 | 10 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 152 | 1925 | 10 | Default |  | A7 |  | 256 QAM | 30@11 | 15@6 |  | 6@3 |
| 153 | 1927.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 154 | 1927.5 | 15 | Default |  | A7 |  | 256 QAM | 52@20 | 26@10 |  | 12@5 |
| 155 | 1932.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 156 | 1932.5 | 15 | Default |  | A7 |  | 256 QAM | 62@11 | 30@6 |  | 14@3 |
| 157 | 1930 | 20 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 158 | 1930 | 20 | Default |  | A7 |  | 256 QAM | 72@28 | 36@14 |  | 16@7 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-5: Test Configuration table for NS_05U power class 3

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest unless otherwise specified in test parameters. |  |  |  |  |
| A-MPR test parameters for NS_05U |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |  |
| 1 | 1922.5 | 5 | 15 | N/A for A-MPR testing. | A3 | DFT-s-OFDM | PI/2 BPSK |  | Outer_Full |  |  |  |
| 2 | 1925 | 10 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 3 | 1925 | 10 | Default |  | A7 |  | PI/2 BPSK |  | 40@10 | 18@5 | 8@3 |  |
| 4 | 1925 | 10 | Default |  | A2 |  | PI/2 BPSK |  | 6@40 | 3@20 | 1@10 |  |
| 5 | 1935 | 10 | Default |  | A4 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 6 | 1927.5 | 15 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 7 | 1927.5 | 15 | Default |  | A7 |  | PI/2 BPSK |  | 60@19 | 27@10 | 12@5 |  |
| 8 | 1927.5 | 15 | Default |  | A2 |  | PI/2 BPSK |  | 6@56 | 3@28 | 1@14 |  |
| 9 | 1932.5 | 15 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 10 | 1932.5 | 15 | Default |  | A2 |  | PI/2 BPSK |  | 6@68 | 3@34 | 1@17 |  |
| 11 | 1942.5 | 15 | Default |  | A5 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 12 | 1930 | 20 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 13 | 1930 | 20 | Default |  | A7 |  | PI/2 BPSK |  | 72@28 | 36@14 | 16@7 |  |
| 14 | 1930 | 20 | Default |  | A2 |  | PI/2 BPSK |  | 6@76 | 3@38 | 1@19 |  |
| 15 | 1950 | 20 | Default |  | A6 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 16 | 1922.5 | 5 | 15 |  | A3 |  | QPSK |  | Outer_Full |  |  |  |
| 17 | 1925 | 10 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 18 | 1925 | 10 | Default |  | A7 |  | QPSK |  | 40@10 | 18@5 |  | 8@3 |
| 19 | 1925 | 10 | Default |  | A2 |  | QPSK |  | 6@40 | 3@20 |  | 1@10 |
| 20 | 1935 | 10 | Default |  | A4 |  | QPSK |  | Outer_Full |  |  |  |
| 21 | 1927.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 22 | 1927.5 | 15 | Default |  | A7 |  | QPSK |  | 60@19 | 27@10 |  | 12@5 |
| 23 | 1927.5 | 15 | Default |  | A2 |  | QPSK |  | 6@56 | 3@28 |  | 1@14 |
| 24 | 1932.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 25 | 1932.5 | 15 | Default |  | A2 |  | QPSK |  | 6@68 | 3@34 |  | 1@17 |
| 26 | 1942.5 | 15 | Default |  | A5 |  | QPSK |  | Outer_Full |  |  |  |
| 27 | 1930 | 20 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 28 | 1930 | 20 | Default |  | A7 |  | QPSK |  | 72@28 | 36@14 |  | 16@7 |
| 29 | 1930 | 20 | Default |  | A2 |  | QPSK |  | 6@76 | 3@38 |  | 1@19 |
| 30 | 1950 | 20 | Default |  | A6 |  | QPSK |  | Outer_Full |  |  |  |
| 31 | 1922.5 | 5 | 15 |  | A3 |  | 16 QAM |  | Outer_Full |  |  |  |
| 32 | 1925 | 10 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 33 | 1925 | 10 | Default |  | A7 |  | 16 QAM |  | 40@10 | 18@5 |  | 8@3 |
| 34 | 1925 | 10 | Default |  | A2 |  | 16 QAM |  | 6@40 | 3@20 |  | 1@10 |
| 35 | 1935 | 10 | Default |  | A4 |  | 16 QAM |  | Outer_Full |  |  |  |
| 36 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 37 | 1927.5 | 15 | Default |  | A7 |  | 16 QAM |  | 60@19 | 27@10 |  | 12@5 |
| 38 | 1927.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@56 | 3@28 |  | 1@14 |
| 39 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 40 | 1932.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@68 | 3@34 |  | 1@17 |
| 41 | 1942.5 | 15 | Default |  | A5 |  | 16 QAM |  | Outer_Full |  |  |  |
| 42 | 1930 | 20 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 43 | 1930 | 20 | Default |  | A7 |  | 16 QAM |  | 72@28 | 36@14 |  | 16@7 |
| 44 | 1930 | 20 | Default |  | A2 |  | 16 QAM |  | 6@76 | 3@38 |  | 1@19 |
| 45 | 1950 | 20 | Default |  | A6 |  | 16 QAM |  | Outer_Full |  |  |  |
| 46 | 1922.5 | 5 | 15 |  | A3 |  | 64 QAM |  | Outer_Full |  |  |  |
| 47 | 1925 | 10 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |
| 48 | 1925 | 10 | Default |  | A7 |  | 64 QAM |  | 40@10 | 18@5 |  | 8@3 |
| 49 | 1925 | 10 | Default |  | A2 |  | 64 QAM |  | 6@40 | 3@20 |  | 1@10 |
| 50 | 1935 | 10 | Default |  | A4 |  | 64 QAM |  | Outer_Full |  |  |  |

| A-MPR test parameters for NS_05U |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |  |
| 51 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 52 | 1927.5 | 15 | Default |  | A7 |  | 64 QAM | 60@19 | 27@10 |  | 12@5 |
| 53 | 1927.5 | 15 | Default |  | A2 |  | 64 QAM | 6@56 | 3@28 |  | 1@14 |
| 54 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 55 | 1932.5 | 15 | Default |  | A2 |  | 64 QAM | 6@68 | 3@34 |  | 1@17 |
| 56 | 1942.5 | 15 | Default |  | A5 |  | 64 QAM | Outer_Full |  |  |  |
| 57 | 1930 | 20 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 58 | 1930 | 20 | Default |  | A7 |  | 64 QAM | 72@28 | 36@14 |  | 16@7 |
| 59 | 1930 | 20 | Default |  | A2 |  | 64 QAM | 6@76 | 3@38 |  | 1@19 |
| 60 | 1950 | 20 | Default |  | A6 |  | 64 QAM | Outer_Full |  |  |  |
| 61 | 1922.5 | 5 | 15 |  | A3 |  | 256 QAM | Outer_Full |  |  |  |
| 62 | 1925 | 10 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 63 | 1925 | 10 | Default |  | A7 |  | 256 QAM | 40@10 | 18@5 |  | 8@3 |
| 64 | 1925 | 10 | Default |  | A2 |  | 256 QAM | 6@40 | 3@20 |  | 1@10 |
| 65 | 1935 | 10 | Default |  | A4 |  | 256 QAM | Outer_Full |  |  |  |
| 66 | 1927.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 67 | 1927.5 | 15 | Default |  | A7 |  | 256 QAM | 60@19 | 27@10 |  | 12@5 |
| 68 | 1927.5 | 15 | Default |  | A2 |  | 256 QAM | 6@56 | 3@28 |  | 1@14 |
| 69 | 1932.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 70 | 1932.5 | 15 | Default |  | A2 |  | 256 QAM | 6@68 | 3@34 |  | 1@17 |
| 71 | 1942.5 | 15 | Default |  | A5 |  | 256 QAM | Outer_Full |  |  |  |
| 72 | 1930 | 20 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 73 | 1930 | 20 | Default |  | A7 |  | 256 QAM | 72@28 | 36@14 |  | 16@7 |
| 74 | 1930 | 20 | Default |  | A2 |  | 256 QAM | 6@76 | 3@38 |  | 1@19 |
| 75 | 1950 | 20 | Default |  | A6 |  | 256 QAM | Outer_Full |  |  |  |
| 76 | 1922.5 | 5 | 15 |  | A3 | CP-OFDM | QPSK | Outer_Full |  |  |  |
| 77 | 1925 | 10 | Default |  | A1 |  | QPSK | Outer_Full |  |  |  |
| 78 | 1925 | 10 | Default |  | A7 |  | QPSK | 42@10 | 18@5 |  | 8@3 |
| 79 | 1925 | 10 | Default |  | A2 |  | QPSK | 6@40 | 3@20 |  | 1@10 |
| 80 | 1935 | 10 | Default |  | A4 |  | QPSK | Outer_Full |  |  |  |
| 81 | 1927.5 | 15 | Default |  | A1 |  | QPSK | Outer_Full |  |  |  |
| 82 | 1927.5 | 15 | Default |  | A7 |  | QPSK | 60@19 | 28@10 |  | 12@5 |
| 83 | 1927.5 | 15 | Default |  | A2 |  | QPSK | 6@56 | 3@28 |  | 1@14 |
| 84 | 1932.5 | 15 | Default |  | A1 |  | QPSK | Outer_Full |  |  |  |
| 85 | 1932.5 | 15 | Default |  | A2 |  | QPSK | 6@68 | 3@34 |  | 1@17 |
| 86 | 1942.5 | 15 | Default |  | A5 |  | QPSK | Outer_Full |  |  |  |
| 87 | 1930 | 20 | Default |  | A1 |  | QPSK | Outer_Full |  |  |  |
| 88 | 1930 | 20 | Default |  | A7 |  | QPSK | 78@28 | 37@14 |  | 17@7 |
| 89 | 1930 | 20 | Default |  | A2 |  | QPSK | 6@76 | 3@38 |  | 1@19 |
| 90 | 1950 | 20 | Default |  | A6 |  | QPSK | Outer_Full |  |  |  |
| 91 | 1922.5 | 5 | 15 |  | A3 |  | 16 QAM | Outer_Full |  |  |  |
| 92 | 1925 | 10 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 93 | 1925 | 10 | Default |  | A7 |  | 16 QAM | 42@10 | 18@5 |  | 8@3 |
| 94 | 1925 | 10 | Default |  | A2 |  | 16 QAM | 6@40 | 3@20 |  | 1@10 |
| 95 | 1935 | 10 | Default |  | A4 |  | 16 QAM | Outer_Full |  |  |  |
| 96 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 97 | 1927.5 | 15 | Default |  | A7 |  | 16 QAM | 60@19 | 28@10 |  | 12@5 |
| 98 | 1927.5 | 15 | Default |  | A2 |  | 16 QAM | 6@56 | 3@28 |  | 1@14 |
| 99 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 100 | 1932.5 | 15 | Default |  | A2 |  | 16 QAM | 6@68 | 3@34 |  | 1@17 |
| 101 | 1942.5 | 15 | Default |  | A5 |  | 16 QAM | Outer_Full |  |  |  |
| 102 | 1930 | 20 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 103 | 1930 | 20 | Default |  | A7 |  | 16 QAM | 78@28 | 37@14 |  | 17@7 |
| 104 | 1930 | 20 | Default |  | A2 |  | 16 QAM | 6@76 | 3@38 |  | 1@19 |
| 105 | 1950 | 20 | Default |  | A6 |  | 16 QAM | Outer_Full |  |  |  |
| 106 | 1922.5 | 5 | 15 |  | A3 |  | 64 QAM | Outer_Full |  |  |  |
| 107 | 1925 | 10 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 108 | 1925 | 10 | Default |  | A7 |  | 64 QAM | 42@10 | 18@5 |  | 8@3 |

| A-MPR test parameters for NS_05U |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |  |
| 109 | 1925 | 10 | Default |  | A2 |  | 64 QAM | 6@40 | 3@20 |  | 1@10 |
| 110 | 1935 | 10 | Default |  | A4 |  | 64 QAM | Outer_Full |  |  |  |
| 111 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 112 | 1927.5 | 15 | Default |  | A7 |  | 64 QAM | 60@19 | 28@10 |  | 12@5 |
| 113 | 1927.5 | 15 | Default |  | A2 |  | 64 QAM | 6@56 | 3@28 |  | 1@14 |
| 114 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 115 | 1932.5 | 15 | Default |  | A2 |  | 64 QAM | 6@68 | 3@34 |  | 1@17 |
| 116 | 1942.5 | 15 | Default |  | A5 |  | 64 QAM | Outer_Full |  |  |  |
| 117 | 1930 | 20 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 118 | 1930 | 20 | Default |  | A7 |  | 64 QAM | 78@28 | 37@14 |  | 17@7 |
| 119 | 1930 | 20 | Default |  | A2 |  | 64 QAM | 6@76 | 3@38 |  | 1@19 |
| 120 | 1950 | 20 | Default |  | A6 |  | 64 QAM | Outer_Full |  |  |  |
| 121 | 1922.5 | 5 | 15 |  | A3 |  | 256 QAM | Outer_Full |  |  |  |
| 122 | 1925 | 10 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 123 | 1925 | 10 | Default |  | A7 |  | 256 QAM | 42@10 | 18@5 |  | 8@3 |
| 124 | 1935 | 10 | Default |  | A4 |  | 256 QAM | Outer_Full |  |  |  |
| 125 | 1927.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 126 | 1927.5 | 15 | Default |  | A7 |  | 256 QAM | 60@19 | 28@10 |  | 12@5 |
| 127 | 1932.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 128 | 1942.5 | 15 | Default |  | A5 |  | 256 QAM | Outer_Full |  |  |  |
| 129 | 1930 | 20 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 130 | 1930 | 20 | Default |  | A7 |  | 256 QAM | 78@28 | 37@14 |  | 17@7 |
| 131 | 1950 | 20 | Default |  | A6 |  | 256 QAM | Outer_Full |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-6: Test Configuration table for NS_43 for PC3 UE

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest |  |
| A-MPR test parameters for NS_43 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | 910 | 10 | default | N/A for A-MPR | DFT-s--OFDM | PI/2 BPSK | Outer_Full (A2) |
| 2 | 907.5 | 15 |  |  |  | PI/2 BPSK | Edge_1RB_Left (A6) |
| 3 | 907.5 | 15 |  |  |  | PI/2 BPSK | Outer_Full (A6) |
| 43 | 902.5 | 5 |  |  |  | QPSK | Outer_Full (A1) |
| 5 | 910 | 10 |  |  |  | QPSK | Outer_Full (A4) |
| 6 | 907.5 | 15 |  |  |  | QPSK | Edge_1RB_Left (A6) |
| 7 | 907.5 | 15 |  |  |  | QPSK | Outer_Full (A6) |
| 8 | 910 | 10 |  |  |  | 16 QAM | Outer_Full (A5) |
| 9 | 907.5 | 15 |  |  |  | 16 QAM | Edge_1RB_Left (A6) |
| 10 | 907.5 | 15 |  |  |  | 16 QAM | Outer_Full (A6) |
| 113 | 910 | 10 |  |  |  | 64 QAM | Outer_Full (A3) |
| 12 | 907.5 | 15 |  |  |  | 64 QAM | Edge_1RB_Left (A6) |
| 13 | 907.5 | 15 |  |  |  | 64 QAM | Outer_Full (A6) |
| 14 | 907.5 | 15 |  |  |  | 256 QAM | Edge_1RB_Left (A6) |
| 15 | 907.5 | 15 |  |  |  | 256 QAM | Outer_Full (A6) |
| 16 | 902.5 | 5 |  |  | CP-OFDM | QPSK | Outer_Full (A1) |
| 17 | 910 | 10 |  |  |  | QPSK | Outer_Full (A5) |
| 18 | 907.5 | 15 |  |  |  | QPSK | Edge_1RB_Left (A6) |
| 19 | 907.5 | 15 |  |  |  | QPSK | Outer_Full (A6) |
| 20 | 902.5 | 5 |  |  |  | 16 QAM | Outer_Full (A1) |
| 21 | 910 | 10 |  |  |  | 16 QAM | Outer_Full (A5) |
| 22 | 907.5 | 15 |  |  |  | 16 QAM | Edge_1RB_Left (A6) |
| 23 | 907.5 | 15 |  |  |  | 16 QAM | Outer_Full (A6) |
| 243 | 910 | 10 |  |  |  | 64 QAM | Outer_Full (A3) |
| 25 | 907.5 | 15 |  |  |  | 64 QAM | Edge_1RB_Left (A6) |
| 26 | 907.5 | 15 |  |  |  | 64 QAM | Outer_Full (A6) |
| 27 | 907.5 | 15 |  |  |  | 256 QAM | Edge_1RB_Left (A6) |
| 28 | 907.5 | 15 |  |  |  | 256 QAM | Outer_Full (A6) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Not applicable when SCS=60kHz. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-6a: Test Configuration table for NS_43 for PC2 UE

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest |  |
| A-MPR test parameters for NS_43 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | 902.5 | 5 | Default | N/A for A-MPR | DFT-s--OFDM | Pi/2 BPSK | Outer_Full (A1) |
| 2 | 910 | 10 |  |  |  | Pi/2 BPSK | Outer_Full (A2) |
| 3 | 907.5 | 15 |  |  |  | Pi/2 BPSK | Outer_Full (A6) |
| 4 | 907.5 | 15 |  |  |  | Pi/2 BPSK | Edge_1RB_Left (A6) |
| 53 | 902.5 | 5 |  |  |  | QPSK | Outer_Full (A1) |
| 6 | 910 | 10 |  |  |  | QPSK | Outer_Full (A4) |
| 7 | 910 | 10 |  |  |  | QPSK | Outer_Full (A5) |
| 8 | 907.5 | 15 |  |  |  | QPSK | Outer_Full (A6) |
| 9 | 907.5 | 15 |  |  |  | QPSK | Edge_1RB_Left (A6) |
| 103 | 902.5 | 5 |  |  |  | 16QAM | Outer_Full (A1) |
| 11 | 910 | 10 |  |  |  | 16QAM | Outer_Full (A4) |
| 12 | 910 | 10 |  |  |  | 16QAM | Outer_Full (A5) |
| 13 | 907.5 | 15 |  |  |  | 16QAM | Outer_Full (A6) |
| 14 | 907.5 | 15 |  |  |  | 16QAM | Edge_1RB_Left (A6) |
| 153 | 902.5 | 5 |  |  |  | 64QAM | Outer_Full (A1) |
| 16 | 910 | 10 |  |  |  | 64QAM | Outer_Full (A2) |
| 173 | 910 | 10 |  |  |  | 64QAM | Outer_Full (A3) |
| 18 | 907.5 | 15 |  |  |  | 64QAM | Outer_Full (A6) |
| 19 | 907.5 | 15 |  |  |  | 64QAM | Edge_1RB_Left (A6) |
| 20 | 907.5 | 15 |  |  |  | 256QAM | Outer_Full (A6) |
| 21 | 907.5 | 15 |  |  |  | 256QAM | Edge_1RB_Left (A6) |
| 223 | 902.5 | 5 |  |  | CP-OFDM | QPSK | Outer_Full (A1) |
| 23 | 910 | 10 |  |  |  | QPSK | Outer_Full (A4) |
| 24 | 910 | 10 |  |  |  | QPSK | Outer_Full (A5) |
| 25 | 907.5 | 15 |  |  |  | QPSK | Outer_Full (A6) |
| 26 | 907.5 | 15 |  |  |  | QPSK | Edge_1RB_Left (A6) |
| 273 | 902.5 | 5 |  |  |  | 16QAM | Outer_Full (A1) |
| 28 | 910 | 10 |  |  |  | 16QAM | Outer_Full (A4) |
| 29 | 910 | 10 |  |  |  | 16QAM | Outer_Full (A5) |
| 30 | 907.5 | 15 |  |  |  | 16QAM | Outer_Full (A6) |
| 31 | 907.5 | 15 |  |  |  | 16QAM | Edge_1RB_Left (A6) |
| 323 | 902.5 | 5 |  |  |  | 64QAM | Outer_Full (A1) |
| 33 | 910 | 10 |  |  |  | 64QAM | Outer_Full (A2) |
| 343 | 910 | 10 |  |  |  | 64QAM | Outer_Full (A3) |
| 35 | 907.5 | 15 |  |  |  | 64QAM | Outer_Full (A6) |
| 36 | 907.5 | 15 |  |  |  | 64QAM | Edge_1RB_Left (A6) |
| 37 | 907.5 | 15 |  |  |  | 256QAM | Outer_Full (A6) |
| 38 | 907.5 | 15 |  |  |  | 256QAM | Edge_1RB_Left (A6) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Not applicable when SCS=60kHz |  |  |  |  |  |  |  |

Table 6.2.3.4.1-7: Test Configuration table for NS_43U

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | 15 kHz |  |
| A-MPR test parameters for NS_43U |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | 902.5 | 5 | 15 | N/A for A-MPR | DFT-s--OFDM | PI/2 BPSK | Outer_Full |
| 2 | 910 | 10 | 15 |  |  | PI/2 BPSK | Outer_Full |
| 3 | 907.5 | 15 | 15 |  |  | PI/2 BPSK | Edge_1RB_Left |
| 4 | 907.5 | 15 | 15 |  |  | PI/2 BPSK | Outer_Full |
| 5 | 902.5 | 5 | 15 |  |  | QPSK | Outer_Full |
| 6 | 910 | 10 | 15 |  |  | QPSK | Outer_Full |
| 7 | 907.5 | 15 | 15 |  |  | QPSK | Edge_1RB_Left |
| 8 | 907.5 | 15 | 15 |  |  | QPSK | Outer_Full |
| 9 | 902.5 | 5 | 15 |  |  | 16 QAM | Outer_Full |
| 10 | 910 | 10 | 15 |  |  | 16 QAM | Outer_Full |
| 11 | 907.5 | 15 | 15 |  |  | 16 QAM | Edge_1RB_Left |
| 12 | 907.5 | 15 | 15 |  |  | 16 QAM | Outer_Full |
| 13 | 902.5 | 5 | 15 |  |  | 64 QAM | Outer_Full |
| 14 | 910 | 10 | 15 |  |  | 64 QAM | Outer_Full |
| 15 | 907.5 | 15 | 15 |  |  | 64 QAM | Edge_1RB_Left |
| 16 | 907.5 | 15 | 15 |  |  | 64 QAM | Outer_Full |
| 17 | 902.5 | 5 | 15 |  |  | 256 QAM | Outer_Full |
| 18 | 910 | 10 | 15 |  |  | 256 QAM | Outer_Full |
| 19 | 907.5 | 15 | 15 |  |  | 256 QAM | Edge_1RB_Left |
| 20 | 907.5 | 15 | 15 |  |  | 256 QAM | Outer_Full |
| 21 | 902.5 | 5 | 15 |  | CP-OFDM | QPSK | Outer_Full |
| 22 | 910 | 10 | 15 |  |  | QPSK | Outer_Full |
| 23 | 907.5 | 15 | 15 |  |  | QPSK | Edge_1RB_Left |
| 24 | 907.5 | 15 | 15 |  |  | QPSK | Outer_Full |
| 25 | 902.5 | 5 | 15 |  |  | 16 QAM | Outer_Full |
| 26 | 910 | 10 | 15 |  |  | 16 QAM | Outer_Full |
| 27 | 907.5 | 15 | 15 |  |  | 16 QAM | Edge_1RB_Left |
| 28 | 907.5 | 15 | 15 |  |  | 16 QAM | Outer_Full |
| 29 | 902.5 | 5 | 15 |  |  | 64 QAM | Outer_Full |
| 30 | 910 | 10 | 15 |  |  | 64 QAM | Outer_Full |
| 31 | 907.5 | 15 | 15 |  |  | 64 QAM | Edge_1RB_Left |
| 32 | 907.5 | 15 | 15 |  |  | 64 QAM | Outer_Full |
| 33 | 902.5 | 5 | 15 |  |  | 256 QAM | Outer_Full |
| 34 | 910 | 10 | 15 |  |  | 256 QAM | Outer_Full |
| 35 | 907.5 | 15 | 15 |  |  | 256 QAM | Edge_1RB_Left |
| 36 | 907.5 | 15 | 15 |  |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-8: Test Configuration table for NS_37

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 10 MHz, 15 MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Unless otherwise specified in the SCS column select Lowest and Highest. |  |  |
| A-MPR test parameters for NS_37 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | 1457.9 | 10 | Default | N/A for A-MPR testing | DFT-s-OFDM | PI/2 BPSK (A1) | Outer_Full |  |  |
| 2 | 1455.4 | 15 | Default |  |  | PI/2 BPSK (A1) | Outer_Full |  |  |
| 3 | 1455.4 | 15 | 15, 30 |  |  | PI/2 BPSK (A2) | 5@0 | 2@0 | N/A |
| 4 | 1455.4 | 15 | 15, 30 |  |  | PI/2 BPSK (A2) | Edge_1RB_Right | Edge_1RB_Right | N/A |
| 5 | 1457.9 | 10 | Default |  |  | QPSK (A1) | Outer_Full |  |  |
| 6 | 1455.4 | 15 | Default |  |  | QPSK (A2) | Outer_Full |  |  |
| 7 | 1455.4 | 15 | 15, 30 |  |  | QPSK (A2) | 5@0 | 2@0 | N/A |
| 8 | 1455.4 | 15 | 15, 30 |  |  | QPSK (A2) | Edge_1RB_Right | Edge_1RB_Right | N/A |
| 9 | 1457.9 | 10 | Default |  |  | 16 QAM (A1) | Outer_Full |  |  |
| 10 | 1455.4 | 15 | Default |  |  | 16 QAM (A2) | Outer_Full |  |  |
| 11 | 1455.4 | 15 | 15, 30 |  |  | 16 QAM (A2) | 5@0 | 2@0 | N/A |
| 12 | 1455.4 | 15 | 15, 30 |  |  | 16 QAM (A2) | Edge_1RB_Right | Edge_1RB_Right | N/A |
| 13 | 1457.9 | 10 | Default |  |  | 64 QAM (A1) | Outer_Full |  |  |
| 14 | 1455.4 | 15 | Default |  |  | 64 QAM (A2) | Outer_Full |  |  |
| 15 | 1455.4 | 15 | 15, 30 |  |  | 64 QAM (A2) | 5@0 | 2@0 | N/A |
| 16 | 1455.4 | 15 | 15, 30 |  |  | 64 QAM (A2) | Edge_1RB_Right | Edge_1RB_Right | N/A |
| 17 | 1457.9 | 10 | Default |  | CP-OFDM | QPSK (A1) | Outer_Full |  |  |
| 18 | 1455.4 | 15 | Default |  |  | QPSK (A2) | Outer_Full |  |  |
| 19 | 1455.4 | 15 | 15, 30 |  |  | QPSK (A2) | 5@0 | 2@0 | N/A |
| 20 | 1455.4 | 15 | 15, 30 |  |  | QPSK (A2) | Edge_1RB_Right | Edge_1RB_Right | N/A |
| 21 | 1457.9 | 10 | Default |  |  | 16 QAM (A1) | Outer_Full |  |  |
| 22 | 1455.4 | 15 | Default |  |  | 16 QAM (A2) | Outer_Full |  |  |
| 23 | 1455.4 | 15 | 15, 30 |  |  | 16 QAM (A2) | 5@0 | 2@0 | N/A |
| 24 | 1455.4 | 15 | 15, 30 |  |  | 16 QAM (A2) | Edge_1RB_Right | Edge_1RB_Right | N/A |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-9: Test Configuration table for NS_38

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 10 MHz, 15 MHz and 20 MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest and Highest. |  |  |
| A-MPR test parameters for NS_38 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | 1435 | 10 | Default | N/A for A-MPR testing | DFT-s-OFDM | PI/2 BPSK | Outer_Full |  |  |
| 2 | 1435 | 10 | Default |  |  | PI/2 BPSK | 12@0 | 6@0 | 3@0 |
| 3 | 1437.5 | 15 | Default |  |  | PI/2 BPSK | Outer_Full |  |  |
| 4 | 1437.5 | 15 | Default |  |  | PI/2 BPSK | 20@0 | 10@0 | 5@0 |
| 5 | 1440 | 20 | Default |  |  | PI/2 BPSK | Outer_Full |  |  |
| 6 | 1440 | 20 | Default |  |  | PI/2 BPSK | 30@0 | 12@0 | 6@0 |
| 7 | 1435 | 10 | Default |  |  | QPSK | Outer_Full |  |  |
| 8 | 1435 | 10 | Default |  |  | QPSK | 12@0 | 6@0 | 3@0 |
| 9 | 1437.5 | 15 | Default |  |  | QPSK | Outer_Full |  |  |
| 10 | 1437.5 | 15 | Default |  |  | QPSK | 20@0 | 10@0 | 5@0 |
| 11 | 1440 | 20 | Default |  |  | QPSK | Outer_Full |  |  |
| 12 | 1440 | 20 | Default |  |  | QPSK | 30@0 | 12@0 | 6@0 |
| 13 | 1435 | 10 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 14 | 1435 | 10 | Default |  |  | 16 QAM | 12@0 | 6@0 | 3@0 |
| 15 | 1437.5 | 15 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 16 | 1437.5 | 15 | Default |  |  | 16 QAM | 20@0 | 10@0 | 5@0 |
| 17 | 1440 | 20 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 18 | 1440 | 20 | Default |  |  | 16 QAM | 30@0 | 12@0 | 6@0 |
| 19 | 1435 | 10 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 20 | 1435 | 10 | Default |  |  | 64 QAM | 12@0 | 6@0 | 3@0 |
| 21 | 1437.5 | 15 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 22 | 1437.5 | 15 | Default |  |  | 64 QAM | 20@0 | 10@0 | 5@0 |
| 23 | 1440 | 20 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 24 | 1440 | 20 | Default |  |  | 64 QAM | 30@0 | 12@0 | 6@0 |
| 25 | 1435 | 10 | Default |  |  | 256 QAM | Outer_Full |  |  |
| 26 | 1435 | 10 | Default |  |  | 256 QAM | 12@0 | 6@0 | 3@0 |
| 27 | 1437.5 | 15 | Default |  |  | 256 QAM | Outer_Full |  |  |
| 28 | 1437.5 | 15 | Default |  |  | 256 QAM | 20@0 | 10@0 | 5@0 |
| 29 | 1440 | 20 | Default |  |  | 256 QAM | Outer_Full |  |  |
| 30 | 1440 | 20 | Default |  |  | 256 QAM | 30@0 | 12@0 | 6@0 |
| 31 | 1435 | 10 | Default |  | CP-OFDM | QPSK | Outer_Full |  |  |
| 32 | 1435 | 10 | Default |  |  | QPSK | 12@0 | 6@0 | 3@0 |
| 33 | 1437.5 | 15 | Default |  |  | QPSK | Outer_Full |  |  |
| 34 | 1437.5 | 15 | Default |  |  | QPSK | 20@0 | 10@0 | 5@0 |
| 35 | 1440 | 20 | Default |  |  | QPSK | Outer_Full |  |  |
| 36 | 1440 | 20 | Default |  |  | QPSK | 30@0 | 15@0 | 7@0 |
| 37 | 1435 | 10 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 38 | 1435 | 10 | Default |  |  | 16 QAM | 12@0 | 6@0 | 3@0 |
| 39 | 1437.5 | 15 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 40 | 1437.5 | 15 | Default |  |  | 16 QAM | 20@0 | 10@0 | 5@0 |
| 41 | 1440 | 20 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 42 | 1440 | 20 | Default |  |  | 16 QAM | 30@0 | 15@0 | 7@0 |
| 43 | 1435 | 10 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 44 | 1435 | 10 | Default |  |  | 64 QAM | 12@0 | 6@0 | 3@0 |
| 45 | 1437.5 | 15 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 46 | 1437.5 | 15 | Default |  |  | 64 QAM | 20@0 | 10@0 | 5@0 |

| A-MPR test parameters for NS_38 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 47 | 1440 | 20 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 48 | 1440 | 20 | Default |  |  | 64 QAM | 30@0 | 15@0 | 7@0 |
| 49 | 1435 | 10 | Default |  |  | 256 QAM | Outer_Full |  |  |
| 50 | 1435 | 10 | Default |  |  | 256 QAM | 12@0 | 6@0 | 3@0 |
| 51 | 1437.5 | 15 | Default |  |  | 256 QAM | Outer_Full |  |  |
| 52 | 1437.5 | 15 | Default |  |  | 256 QAM | 20@0 | 10@0 | 5@0 |
| 53 | 1440 | 20 | Default |  |  | 256 QAM | Outer_Full |  |  |
| 54 | 1440 | 20 | Default |  |  | 256 QAM | 30@0 | 15@0 | 7@0 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: There is no need to test this frequency if it has already been tested in the Low Range. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-10: Test Configuration table for NS_39

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 10 MHz, 15 MHz and 20 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest and Highest. |  |
| A-MPR test parameters for NS_39 |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  |  | RB allocation (Note 1) |
| 1 | 1465 | 10 | Default | N/A for A-MPR testing | DFT-s-OFDM | PI/2 BPSK |  | Outer_Full |
| 2 | 1462.5 | 15 | Default |  |  | PI/2 BPSK |  | Outer_Full |
| 3 | 1460 | 20 | Default |  |  | PI/2 BPSK |  | Outer_Full |
| 4 | 1465 | 10 | Default |  |  | QPSK |  | Outer_Full |
| 5 | 1462.5 | 15 | Default |  |  | QPSK |  | Outer_Full |
| 6 | 1460 | 20 | Default |  |  | QPSK |  | Outer_Full |
| 7 | 1465 | 10 | Default |  |  | 16 QAM |  | Outer_Full |
| 8 | 1462.5 | 15 | Default |  |  | 16 QAM |  | Outer_Full |
| 9 | 1460 | 20 | Default |  |  | 16 QAM |  | Outer_Full |
| 10 | 1465 | 10 | Default |  |  | 64 QAM |  | Outer_Full |
| 11 | 1462.5 | 15 | Default |  |  | 64 QAM |  | Outer_Full |
| 12 | 1460 | 20 | Default |  |  | 64 QAM |  | Outer_Full |
| 13 | 1465 | 10 | Default |  |  | 256 QAM |  | Outer_Full |
| 14 | 1462.5 | 15 | Default |  |  | 256 QAM |  | Outer_Full |
| 15 | 1460 | 20 | Default |  |  | 256 QAM |  | Outer_Full |
| 16 | 1465 | 10 | Default |  | CP-OFDM | QPSK |  | Outer_Full |
| 17 | 1462.5 | 15 | Default |  |  | QPSK |  | Outer_Full |
| 18 | 1460 | 20 | Default |  |  | QPSK |  | Outer_Full |
| 19 | 1465 | 10 | Default |  |  | 16 QAM |  | Outer_Full |
| 20 | 1462.5 | 15 | Default |  |  | 16 QAM |  | Outer_Full |
| 21 | 1460 | 20 | Default |  |  | 16 QAM |  | Outer_Full |
| 22 | 1465 | 10 | Default |  |  | 64 QAM |  | Outer_Full |
| 23 | 1462.5 | 15 | Default |  |  | 64 QAM |  | Outer_Full |
| 24 | 1460 | 20 | Default |  |  | 64 QAM |  | Outer_Full |
| 25 | 1465 | 10 | Default |  |  | 256 QAM |  | Outer_Full |
| 26 | 1462.5 | 15 | Default |  |  | 256 QAM |  | Outer_Full |
| 27 | 1460 | 20 | Default |  |  | 256 QAM |  | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: There is no need to test this frequency if it has already been tested in the Low Range. |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-11: Test Configuration table for NS_18

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Low range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Use channel bandwidth as specified in test parameters |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_18 |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) | Comment |
| 1 | Low | 3MHz, 5MHz, 10MHz, 15MHz, 20MHz | Default | N/A | DFT-s-OFDM | QPSK | Edge_1RB_Left | A1, A2 |
| 2 |  |  |  |  |  | QPSK | Outer_Full | A1, A2 |
| 3 |  |  |  |  |  | 16 QAM | Edge_1RB_Left | A1, A2 |
| 4 |  |  |  |  |  | 16 QAM | Outer_Full | A1, A2 |
| 5 |  |  |  |  |  | 64 QAM | Edge_1RB_Left | A1, A2 |
| 6 |  |  |  |  |  | 64 QAM | Outer_Full | A1, A2 |
| 7 |  |  |  |  |  | 256 QAM | Edge_1RB_Left | A1, A2 |
| 8 |  |  |  |  |  | 256 QAM | Outer_Full | A1, A2 |
| 9 |  |  |  |  | CP-OFDM | QPSK | Edge_1RB_Left | A1, A2 |
| 10 |  |  |  |  |  | QPSK | Outer_Full | A1, A2 |
| 11 |  |  |  |  |  | 16 QAM | Edge_1RB_Left | A1, A2 |
| 12 |  |  |  |  |  | 16 QAM | Outer_Full | A1, A2 |
| 13 |  |  |  |  |  | 64 QAM | Edge_1RB_Left | A1, A2 |
| 14 |  |  |  |  |  | 64 QAM | Outer_Full | A1, A2 |
| 15 |  |  |  |  |  | 256 QAM | Edge_1RB_Left | A1, A2 |
| 16 |  |  |  |  |  | 256 QAM | Outer_Full | A1, A2 |
| 17 |  | 25MHz, 30MHz | 15kHz |  | DFT-s-OFDM | 256 QAM | 81@70 | A3 |
| 18 |  |  | 30kHz |  |  | 256 QAM | 40@35 | A3 |
| 19 |  |  | Default |  |  | 256 QAM | Edge_1RB_Right | A3 |
| 20 |  |  | Default |  |  | 256 QAM | Outer_Full | A4 |
| 21 |  |  | Default |  |  | 16QAM | Edge_1RB_Left | A5 |
| 22 |  |  | Default |  |  | 64QAM | Edge_1RB_Left | A5 |
| 23 |  |  | Default |  |  | 256QAM | Edge_1RB_Left | A5 |
| 24 |  |  | 15kHz |  | CP-OFDM | 256 QAM | 87@73 | A3 |
| 25 |  |  | 30kHz |  |  | 256 QAM | 42@36 | A3 |
| 26 |  |  | Default |  |  | 256 QAM | Edge_1RB_Right | A3 |
| 27 |  |  | Default |  |  | 256 QAM | Outer_Full | A4 |
| 28 |  |  | Default |  |  | 16QAM | Edge_1RB_Left | A5 |
| 29 |  |  | Default |  |  | 64QAM | Edge_1RB_Left | A5 |
| 30 |  |  | Default |  |  | 256QAM | Edge_1RB_Left | A5 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-12: Test Configuration table for NS_24

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Refer to uplink carrier centre frequency (Fc) in test parameters |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Refer to test parameters (5, 10, 15, 20 MHz) |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest |  |  |  |  |
| A-MPR test parameters for NS_24 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (NOTE 2, 3) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | Region A | Region B | Region C |
| 1-3 | 1992.5 | 5 | Default | N/A for A-MPR testing. | DFT-s-OFDM | PI/2 BPSK QPSK 16 QAM | Outer_Full | N/A | N/A |
| 4-6 | 1997.5 | 5 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | N/A | N/A |
| 7-15 | 2002.5 | 5 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 16-18 | 1985 | 10 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Edge_1RB_Right | N/A | N/A |
| 19-21 | 1985 | 10 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | N/A | N/A |
| 22-30 | 1995 | 10 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 31-39 | 2000 | 10 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Edge_1RB_Right | Edge_1RB_Left | 36@0 |
| 40-42 | 2000 | 10 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | N/A | N/A |
| 43-51 | 1987.5 | 15 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 52-60 | 1997.5 | 15 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Edge_1RB_Right | Edge_1RB_Left | 50@0 |
| 61-63 | 1997.5 | 15 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | N/A | N/A |
| 64-72 | 1990 | 20 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Edge_1RB_Right | Edge_1RB_Left | 75@0 |
| 73-75 | 1990 | 20 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | N/A | N/A |
| 76-84 | 1995 | 20 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Edge_1RB_Right | Edge_1RB_Left | 64@0 |
| 85-87 | 1995 | 20 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | N/A | N/A |
| 88 | 1992.5 | 5 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 89 | 1997.5 | 5 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 90-92 | 2002.5 | 5 | Default |  |  | 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 93 | 1985 | 10 | Default |  |  | 64 QAM | Edge_1RB_Right | N/A | N/A |
| 94 | 1985 | 10 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |

| A-MPR test parameters for NS_24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (NOTE 2, 3) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | Region A | Region B | Region C |
| 95-97 | 1995 | 10 | Default |  |  | 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 98-100 | 2000 | 10 | Default |  |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 36@0 |
| 101 | 2000 | 10 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 103-105 | 1987.5 | 15 | Default |  |  | 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 106-108 | 1997.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 50@0 |
| 109 | 1997.5 | 15 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 110-112 | 1990 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 75@0 |
| 113 | 1990 | 20 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 114-116 | 1995 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 64@0 |
| 117 | 1995 | 20 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 118 | 1992.5 | 5 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 119 | 1997.5 | 5 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 120-122 | 2002.5 | 5 | Default |  |  | 256 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 123 | 1985 | 10 | Default |  |  | 256 QAM | Edge_1RB_Right | N/A | N/A |
| 124 | 1985 | 10 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 125-127 | 1995 | 10 | Default |  |  | 256 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 128-130 | 2000 | 10 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 36@0 |
| 131 | 2000 | 10 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 133-135 | 1987.5 | 15 | Default |  |  | 256 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 136-138 | 1997.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 50@0 |
| 139 | 1997.5 | 15 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 140-142 | 1990 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 75@0 |
| 143 | 1990 | 20 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 144-146 | 1995 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 64@0 |
| 147 | 1995 | 20 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 148-150 | 1992.5 | 5 | Default |  | CP-OFDM | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 151-153 | 1997.5 | 5 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 154-162 | 2002.5 | 5 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 163-165 | 1985 | 10 | Default |  |  | QPSK 16 QAM 64 QAM | Edge_1RB_Right | N/A | N/A |
| 166-168 | 1985 | 10 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 169-177 | 1995 | 10 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |

| A-MPR test parameters for NS_24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (NOTE 2, 3) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | Region A | Region B | Region C |
| 178-186 | 2000 | 10 | Default |  |  | QPSK 16 QAM 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 37@0 |
| 187-189 | 2000 | 10 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 190-198 | 1987.5 | 15 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 199-207 | 1997.5 | 15 | Default |  |  | QPSK 16 QAM 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 51@0 |
| 208-210 | 1997.5 | 15 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 211-219 | 1990 | 20 | Default |  |  | QPSK 16 QAM 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 74@0 |
| 220-222 | 1990 | 20 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 223-231 | 1995 | 20 | Default |  |  | QPSK 16 QAM 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 69@0 |
| 232-234 | 1995 | 20 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 235 | 1992.5 | 5 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 236 | 1997.5 | 5 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 237-239 | 2002.5 | 5 | Default |  |  | 256 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 240 | 1985 | 10 | Default |  |  | 256 QAM | Edge_1RB_Right | N/A | N/A |
| 241 | 1985 | 10 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 242-244 | 1995 | 10 | Default |  |  | 256 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 245-247 | 2000 | 10 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 37@0 |
| 248 | 2000 | 10 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 250-252 | 1987.5 | 15 | Default |  |  | 256 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 253-255 | 1997.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 51@0 |
| 256 | 1997.5 | 15 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 257-259 | 1990 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 74@0 |
| 260 | 1990 | 20 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 261-263 | 1995 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 69@0 |
| 264 | 1995 | 20 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 265 | 1992.5 | 5 | Default |  |  | QPSK | Edge_1RB_Right | N/A | N/A |
| 266 | 1997.5 | 5 | Default |  |  | 16 QAM | Edge_1RB_Right | N/A | N/A |
| 267 | 1985 | 10 | Default |  |  | QPSK | Edge_1RB_Left | N/A | N/A |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: In test IDs with multiple modulations, each UL Modulation shall be tested separately against Range A, B, and C. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-13: Test Configuration table for NS_27 (contiguous allocation)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies |  |  |  |  |  |  | Refer to uplink carrier centre frequency (Fc) in test parameters. Associated parameters defined in Table 6.2.3.4.1-13a and 6.2.3.4.1-13b. |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Refer to test parameters (15, 20, 30, 40 MHz) |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest (Note 3) |  |  |  |
| A-MPR test parameters for NS_27 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | 3557.52 | 15 | Default | N/A for A-MPR testing. | DFT-s-OFDM | PI/2 BPSK |  | Edge_1RB_Left (A3) |  |  |
| 2 | 3557.52 | 15 | Default |  |  | PI/2 BPSK |  | Outer_Full (A3) |  |  |
| 3 | 3692.49 | 15 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Right (A3) |  |  |
| 4 | 3692.49 | 15 | Default |  |  | PI/2 BPSK |  | Outer_Full (A3) |  |  |
| 5 | 3562.5 | 15 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Left (A4) |  |  |
| 6 | 3562.5 | 15 | Default |  |  | PI/2 BPSK |  | Outer_Full (2) |  |  |
| 7 | 3687.48 | 15 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Right (A4) |  |  |
| 8 | 3687.48 | 15 | Default |  |  | PI/2 BPSK |  | Outer_Full (2) |  |  |
| 9 | 3560.01 | 20 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Left (A5) |  |  |
| 10 | 3560.01 | 20 | Default |  |  | PI/2 BPSK |  | Outer_Full (A5) |  |  |
| 11 | 3690 | 20 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Right (A5) |  |  |
| 12 | 3690 | 20 | Default |  |  | PI/2 BPSK |  | Outer_Full (A5) |  |  |
| 13 | 3570 | 20 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Left (A6) |  |  |
| 14 | 3570 | 20 | Default |  |  | PI/2 BPSK |  | Outer_Full (2) |  |  |
| 15 | 3679.98 | 20 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Right (A6) |  |  |
| 16 | 3679.98 | 20 | Default |  |  | PI/2 BPSK |  | Outer_Full (2) |  |  |
| 17 | 3565.02 | 30 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Left (A7) |  |  |
| 18 | 3565.02 | 30 | Default |  |  | PI/2 BPSK |  | 108@41 (A2) | 54@21 (A2) | 25@11 (A2) |
| 19 | 3565.02 | 30 | Default |  |  | PI/2 BPSK |  | 75@41 (A1) | 40@21 (A1) | 18@11 (A1) |
| 20 | 3565.02 | 30 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Right (A7) |  |  |
| 21 | 3684.99 | 30 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Right (A7) |  |  |
| 22 | 3684.99 | 30 | Default |  |  | PI/2 BPSK |  | 108@0 (A2) | 54@0 (A2) | 27@0 (A2) |
| 23 | 3684.99 | 30 | Default |  |  | PI/2 BPSK |  | 81@0 (A1) | 40@0 (A1) | 20@0 (A1) |
| 24 | 3684.99 | 30 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Left (A7) |  |  |
| 25 | 3585 | 30 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Left (A8) |  |  |
| 26 | 3585 | 30 | Default |  |  | PI/2 BPSK |  | Outer_Full (4) |  |  |
| 27 | 3664.98 | 30 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Right (A8) |  |  |
| 28 | 3664.98 | 30 | Default |  |  | PI/2 BPSK |  | Outer_Full (4) |  |  |
| 29 | 3570 | 40 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Left (A7) |  |  |
| 30 | 3570 | 40 | Default |  |  | PI/2 BPSK |  | 150@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 31 | 3570 | 40 | Default |  |  | PI/2 BPSK |  | 96@73 (A1) | 48@35 (A1) | 24@16 (A1) |
| 32 | 3570 | 40 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Right (A7) |  |  |
| 33 | 3679.98 | 40 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Right (A7) |  |  |
| 34 | 3679.98 | 40 | Default |  |  | PI/2 BPSK |  | 135@0 (A2) | 64@0 (A2) | 32@0 (A2) |
| 35 | 3679.98 | 40 | Default |  |  | PI/2 BPSK |  | 96@38 (A1) | 48@18 (A1) | 24@9 (A1) |
| 36 | 3679.98 | 40 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Left (A7) |  |  |
| 37 | 3600 | 40 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Left (A8) |  |  |
| 38 | 3600 | 40 | Default |  |  | PI/2 BPSK |  | Outer_Full (4.5) |  |  |
| 39 | 3649.98 | 40 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Right (A8) |  |  |
| 40 | 3649.98 | 40 | Default |  |  | PI/2 BPSK |  | Outer_Full (4.5) |  |  |
| 41 | 3557.52 | 15 | Default |  |  | QPSK |  | Edge_1RB_Left (A3) |  |  |
| 42 | 3557.52 | 15 | Default |  |  | QPSK |  | Outer_Full (A3) |  |  |
| 43 | 3692.49 | 15 | Default |  |  | QPSK |  | Edge_1RB_Right (A3) |  |  |
| 44 | 3692.49 | 15 | Default |  |  | QPSK |  | Outer_Full (A3) |  |  |

| A-MPR test parameters for NS_27 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 45 | 3562.5 | 15 | Default |  |  | QPSK | Edge_1RB_Left (A4) |  |  |
| 46 | 3562.5 | 15 | Default |  |  | QPSK | Outer_Full (2) |  |  |
| 47 | 3687.48 | 15 | Default |  |  | QPSK | Edge_1RB_Right (A4) |  |  |
| 48 | 3687.48 | 15 | Default |  |  | QPSK | Outer_Full (2) |  |  |
| 49 | 3560.01 | 20 | Default |  |  | QPSK | Edge_1RB_Left (A5) |  |  |
| 50 | 3560.01 | 20 | Default |  |  | QPSK | Outer_Full (A5) |  |  |
| 51 | 3690 | 20 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |
| 52 | 3690 | 20 | Default |  |  | QPSK | Outer_Full (A5) |  |  |
| 53 | 3570 | 20 | Default |  |  | QPSK | Edge_1RB_Left (A6) |  |  |
| 54 | 3570 | 20 | Default |  |  | QPSK | Outer_Full (2) |  |  |
| 55 | 3679.98 | 20 | Default |  |  | QPSK | Edge_1RB_Right (A6) |  |  |
| 56 | 3679.98 | 20 | Default |  |  | QPSK | Outer_Full (2) |  |  |
| 57 | 3565.02 | 30 | Default |  |  | QPSK | Edge_1RB_Left (A7) |  |  |
| 58 | 3565.02 | 30 | Default |  |  | QPSK | 108@41 (A2) | 54@21 (A2) | 25@11 (A2) |
| 59 | 3565.02 | 30 | Default |  |  | QPSK | 75@41 (A1) | 40@21 (A1) | 18@11 (A1) |
| 60 | 3565.02 | 30 | Default |  |  | QPSK | Edge_1RB_Right (A7) |  |  |
| 61 | 3684.99 | 30 | Default |  |  | QPSK | Edge_1RB_Right (A7) |  |  |
| 62 | 3684.99 | 30 | Default |  |  | QPSK | 108@0 (A2) | 54@0 (A2) | 27@0 (A2) |
| 63 | 3684.99 | 30 | Default |  |  | QPSK | 81@0 (A1) | 40@0 (A1) | 20@0 (A1) |
| 64 | 3684.99 | 30 | Default |  |  | QPSK | Edge_1RB_Left (A7) |  |  |
| 65 | 3585 | 30 | Default |  |  | QPSK | Edge_1RB_Left (A8) |  |  |
| 66 | 3585 | 30 | Default |  |  | QPSK | Outer_Full (4) |  |  |
| 67 | 3664.98 | 30 | Default |  |  | QPSK | Edge_1RB_Right (A8) |  |  |
| 68 | 3664.98 | 30 | Default |  |  | QPSK | Outer_Full (4) |  |  |
| 69 | 3570 | 40 | Default |  |  | QPSK | Edge_1RB_Left (A7) |  |  |
| 70 | 3570 | 40 | Default |  |  | QPSK | 150@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 71 | 3570 | 40 | Default |  |  | QPSK | 96@73 (A1) | 48@35 (A1) | 24@16 (A1) |
| 72 | 3570 | 40 | Default |  |  | QPSK | Edge_1RB_Right (A7) |  |  |
| 73 | 3679.98 | 40 | Default |  |  | QPSK | Edge_1RB_Right (A7) |  |  |
| 74 | 3679.98 | 40 | Default |  |  | QPSK | 135@0 (A2) | 64@0 (A2) | 32@0 (A2) |
| 75 | 3679.98 | 40 | Default |  |  | QPSK | 96@38 (A1) | 48@18 (A1) | 24@9 (A1) |
| 76 | 3679.98 | 40 | Default |  |  | QPSK | Edge_1RB_Left (A7) |  |  |
| 77 | 3600 | 40 | Default |  |  | QPSK | Edge_1RB_Left (A8) |  |  |
| 78 | 3600 | 40 | Default |  |  | QPSK | Outer_Full (4.5) |  |  |
| 79 | 3649.98 | 40 | Default |  |  | QPSK | Edge_1RB_Right (A8) |  |  |
| 80 | 3649.98 | 40 | Default |  |  | QPSK | Outer_Full (4.5) |  |  |
| 81 | 3557.52 | 15 | Default |  |  | 16 QAM | Edge_1RB_Left (A3) |  |  |
| 82 | 3557.52 | 15 | Default |  |  | 16 QAM | Outer_Full (A3) |  |  |
| 83 | 3692.49 | 15 | Default |  |  | 16 QAM | Edge_1RB_Right (A3) |  |  |
| 84 | 3692.49 | 15 | Default |  |  | 16 QAM | Outer_Full (A3) |  |  |
| 85 | 3562.5 | 15 | Default |  |  | 16 QAM | Edge_1RB_Left (A4) |  |  |
| 86 | 3562.5 | 15 | Default |  |  | 16 QAM | Outer_Full (2) |  |  |
| 87 | 3687.48 | 15 | Default |  |  | 16 QAM | Edge_1RB_Right (A4) |  |  |
| 88 | 3687.48 | 15 | Default |  |  | 16 QAM | Outer_Full (2) |  |  |
| 89 | 3560.01 | 20 | Default |  |  | 16 QAM | Edge_1RB_Left (A5) |  |  |
| 90 | 3560.01 | 20 | Default |  |  | 16 QAM | Outer_Full (A5) |  |  |
| 91 | 3690 | 20 | Default |  |  | 16 QAM | Edge_1RB_Right (A5) |  |  |
| 92 | 3690 | 20 | Default |  |  | 16 QAM | Outer_Full (A5) |  |  |
| 93 | 3570 | 20 | Default |  |  | 16 QAM | Edge_1RB_Left (A6) |  |  |
| 94 | 3570 | 20 | Default |  |  | 16 QAM | Outer_Full (2) |  |  |

| A-MPR test parameters for NS_27 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 95 | 3679.98 | 20 | Default |  |  | 16 QAM | Edge_1RB_Right (A6) |  |  |
| 96 | 3679.98 | 20 | Default |  |  | 16 QAM | Outer_Full (2) |  |  |
| 97 | 3565.02 | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 98 | 3565.02 | 30 | Default |  |  | 16 QAM | 108@41 (A2) | 54@21 (A2) | 25@11 (A2) |
| 99 | 3565.02 | 30 | Default |  |  | 16 QAM | 75@41 (A1) | 40@21 (A1) | 18@11 (A1) |
| 100 | 3565.02 | 30 | Default |  |  | 16 QAM | Edge_1RB_Right (A7) |  |  |
| 101 | 3684.99 | 30 | Default |  |  | 16 QAM | Edge_1RB_Right (A7) |  |  |
| 102 | 3684.99 | 30 | Default |  |  | 16 QAM | 108@0 (A2) | 54@0 (A2) | 27@0 (A2) |
| 103 | 3684.99 | 30 | Default |  |  | 16 QAM | 81@0 (A1) | 40@0 (A1) | 20@0 (A1) |
| 104 | 3684.99 | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 105 | 3585 | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A8) |  |  |
| 106 | 3585 | 30 | Default |  |  | 16 QAM | Outer_Full (4) |  |  |
| 107 | 3664.98 | 30 | Default |  |  | 16 QAM | Edge_1RB_Right (A8) |  |  |
| 108 | 3664.98 | 30 | Default |  |  | 16 QAM | Outer_Full (4) |  |  |
| 109 | 3570 | 40 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 110 | 3570 | 40 | Default |  |  | 16 QAM | 150@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 111 | 3570 | 40 | Default |  |  | 16 QAM | 96@73 (A1) | 48@35 (A1) | 24@16 (A1) |
| 112 | 3570 | 40 | Default |  |  | 16 QAM | Edge_1RB_Right (A7) |  |  |
| 113 | 3679.98 | 40 | Default |  |  | 16 QAM | Edge_1RB_Right (A7) |  |  |
| 114 | 3679.98 | 40 | Default |  |  | 16 QAM | 135@0 (A2) | 64@0 (A2) | 32@0 (A2) |
| 115 | 3679.98 | 40 | Default |  |  | 16 QAM | 96@38 (A1) | 48@18 (A1) | 24@9 (A1) |
| 116 | 3679.98 | 40 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 117 | 3600 | 40 | Default |  |  | 16 QAM | Edge_1RB_Left (A8) |  |  |
| 118 | 3600 | 40 | Default |  |  | 16 QAM | Outer_Full (4.5) |  |  |
| 119 | 3649.98 | 40 | Default |  |  | 16 QAM | Edge_1RB_Right (A8) |  |  |
| 120 | 3649.98 | 40 | Default |  |  | 16 QAM | Outer_Full (4.5) |  |  |
| 121 | 3557.52 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left (A3) |  |  |
| 122 | 3557.52 | 15 | Default |  |  | 64 QAM | Outer_Full (A3) |  |  |
| 123 | 3692.49 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right (A3) |  |  |
| 124 | 3692.49 | 15 | Default |  |  | 64 QAM | Outer_Full (A3) |  |  |
| 125 | 3562.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left (A4) |  |  |
| 126 | 3562.5 | 15 | Default |  |  | 64 QAM | Outer_Full (2) |  |  |
| 127 | 3687.48 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right (A4) |  |  |
| 128 | 3687.48 | 15 | Default |  |  | 64 QAM | Outer_Full (2) |  |  |
| 129 | 3560.01 | 20 | Default |  |  | 64 QAM | Edge_1RB_Left (A5) |  |  |
| 130 | 3560.01 | 20 | Default |  |  | 64 QAM | Outer_Full (A5) |  |  |
| 131 | 3690 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right (A5) |  |  |
| 132 | 3690 | 20 | Default |  |  | 64 QAM | Outer_Full (A5) |  |  |
| 133 | 3570 | 20 | Default |  |  | 64 QAM | Edge_1RB_Left (A6) |  |  |
| 134 | 3570 | 20 | Default |  |  | 64 QAM | Outer_Full (2) |  |  |
| 135 | 3679.98 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right (A6) |  |  |
| 136 | 3679.98 | 20 | Default |  |  | 64 QAM | Outer_Full (2) |  |  |
| 137 | 3565.02 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 138 | 3565.02 | 30 | Default |  |  | 64 QAM | 108@41 (A2) | 54@21 (A2) | 25@11 (A2) |
| 139 | 3565.02 | 30 | Default |  |  | 64 QAM | 75@41 (A1) | 40@21 (A1) | 18@11 (A1) |
| 140 | 3565.02 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 141 | 3684.99 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 142 | 3684.99 | 30 | Default |  |  | 64 QAM | 108@0 (A2) | 54@0 (A2) | 27@0 (A2) |
| 143 | 3684.99 | 30 | Default |  |  | 64 QAM | 81@0 (A1) | 40@0 (A1) | 20@0 (A1) |
| 144 | 3684.99 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 145 | 3585 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A8) |  |  |
| 146 | 3585 | 30 | Default |  |  | 64 QAM | Outer_Full (4) |  |  |
| 147 | 3664.98 | 30 | Default |  |  | 64 QAM | Edge_1RB_Right (A8) |  |  |
| 148 | 3664.98 | 30 | Default |  |  | 64 QAM | Outer_Full (4) |  |  |
| 149 | 3570 | 40 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 150 | 3570 | 40 | Default |  |  | 64 QAM | 150@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 151 | 3570 | 40 | Default |  |  | 64 QAM | 96@73 (A1) | 48@35 (A1) | 24@16 (A1) |
| 152 | 3570 | 40 | Default |  |  | 64 QAM | Edge_1RB_Right (A7) |  |  |
| 153 | 3679.98 | 40 | Default |  |  | 64 QAM | Edge_1RB_Right (A7) |  |  |
| 154 | 3679.98 | 40 | Default |  |  | 64 QAM | 135@0 (A2) | 64@0 (A2) | 32@0 (A2) |
| 155 | 3679.98 | 40 | Default |  |  | 64 QAM | 96@38 (A1) | 48@18 (A1) | 24@9 (A1) |
| 156 | 3679.98 | 40 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |

| A-MPR test parameters for NS_27 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 157 | 3600 | 40 | Default |  |  | 64 QAM | Edge_1RB_Left (A8) |  |  |
| 158 | 3600 | 40 | Default |  |  | 64 QAM | Outer_Full (4.5) |  |  |
| 159 | 3649.98 | 40 | Default |  |  | 64 QAM | Edge_1RB_Right (A8) |  |  |
| 160 | 3649.98 | 40 | Default |  |  | 64 QAM | Outer_Full (4.5) |  |  |
| 161 | 3557.52 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left (A3) |  |  |
| 162 | 3557.52 | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 163 | 3692.49 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |
| 164 | 3692.49 | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 165 | 3562.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left (A4) |  |  |
| 166 | 3562.5 | 15 | Default |  |  | 256 QAM | Outer_Full (2) |  |  |
| 167 | 3687.48 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right (A4) |  |  |
| 168 | 3687.48 | 15 | Default |  |  | 256 QAM | Outer_Full (2) |  |  |
| 169 | 3560.01 | 20 | Default |  |  | 256 QAM | Edge_1RB_Left (A5) |  |  |
| 170 | 3560.01 | 20 | Default |  |  | 256 QAM | Outer_Full (A5) |  |  |
| 171 | 3690 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right (A5) |  |  |
| 172 | 3690 | 20 | Default |  |  | 256 QAM | Outer_Full (A5) |  |  |
| 173 | 3570 | 20 | Default |  |  | 256 QAM | Edge_1RB_Left (A6) |  |  |
| 174 | 3570 | 20 | Default |  |  | 256 QAM | Outer_Full (2) |  |  |
| 175 | 3679.98 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right (A6) |  |  |
| 176 | 3679.98 | 20 | Default |  |  | 256 QAM | Outer_Full (2) |  |  |
| 177 | 3565.02 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 178 | 3565.02 | 30 | Default |  |  | 256 QAM | 108@41 (A2) | 54@21 (A2) | 25@11 (A2) |
| 179 | 3565.02 | 30 | Default |  |  | 256 QAM | 75@41 (A1) | 40@21 (A1) | 18@11 (A1) |
| 180 | 3565.02 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 181 | 3684.99 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 182 | 3684.99 | 30 | Default |  |  | 256 QAM | 108@0 (A2) | 54@0 (A2) | 27@0 (A2) |
| 183 | 3684.99 | 30 | Default |  |  | 256 QAM | 81@0 (A1) | 40@0 (A1) | 20@0 (A1) |
| 184 | 3684.99 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 185 | 3585 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A8) |  |  |
| 186 | 3585 | 30 | Default |  |  | 256 QAM | Outer_Full (4) |  |  |
| 187 | 3664.98 | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A8) |  |  |
| 188 | 3664.98 | 30 | Default |  |  | 256 QAM | Outer_Full (4) |  |  |
| 189 | 3570 | 40 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 190 | 3570 | 40 | Default |  |  | 256 QAM | 150@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 191 | 3570 | 40 | Default |  |  | 256 QAM | 96@73 (A1) | 48@35 (A1) | 24@16 (A1) |
| 192 | 3570 | 40 | Default |  |  | 256 QAM | Edge_1RB_Right (A7) |  |  |
| 193 | 3679.98 | 40 | Default |  |  | 256 QAM | Edge_1RB_Right (A7) |  |  |
| 194 | 3679.98 | 40 | Default |  |  | 256 QAM | 135@0 (A2) | 64@0 (A2) | 32@0 (A2) |
| 195 | 3679.98 | 40 | Default |  |  | 256 QAM | 96@38 (A1) | 48@18 (A1) | 24@9 (A1) |
| 196 | 3679.98 | 40 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 197 | 3600 | 40 | Default |  |  | 256 QAM | Edge_1RB_Left (A8) |  |  |
| 198 | 3600 | 40 | Default |  |  | 256 QAM | Outer_Full (4.5) |  |  |
| 199 | 3649.98 | 40 | Default |  |  | 256 QAM | Edge_1RB_Right (A8) |  |  |
| 200 | 3649.98 | 40 | Default |  |  | 256 QAM | Outer_Full (4.5) |  |  |
| 201 | 3557.52 | 15 | Default |  | CP-OFDM | QPSK | Edge_1RB_Left (A3) |  |  |
| 202 | 3557.52 | 15 | Default |  |  | QPSK | Outer_Full (A3) |  |  |
| 203 | 3692.49 | 15 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |
| 204 | 3692.49 | 15 | Default |  |  | QPSK | Outer_Full (A3) |  |  |
| 205 | 3562.5 | 15 | Default |  |  | QPSK | Edge_1RB_Left (A4) |  |  |

| A-MPR test parameters for NS_27 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 206 | 3562.5 | 15 | Default |  |  | QPSK | Outer_Full (2) |  |  |
| 207 | 3687.48 | 15 | Default |  |  | QPSK | Edge_1RB_Right (A4) |  |  |
| 208 | 3687.48 | 15 | Default |  |  | QPSK | Outer_Full (2) |  |  |
| 209 | 3560.01 | 20 | Default |  |  | QPSK | Edge_1RB_Left (A5) |  |  |
| 210 | 3560.01 | 20 | Default |  |  | QPSK | Outer_Full (A5) |  |  |
| 211 | 3690 | 20 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |
| 212 | 3690 | 20 | Default |  |  | QPSK | Outer_Full (A5) |  |  |
| 213 | 3570 | 20 | Default |  |  | QPSK | Edge_1RB_Left (A6) |  |  |
| 214 | 3570 | 20 | Default |  |  | QPSK | Outer_Full (2) |  |  |
| 215 | 3679.98 | 20 | Default |  |  | QPSK | Edge_1RB_Right (A6) |  |  |
| 216 | 3679.98 | 20 | Default |  |  | QPSK | Outer_Full (2) |  |  |
| 217 | 3565.02 | 30 | Default |  |  | QPSK | Edge_1RB_Left (A7) |  |  |
| 218 | 3565.02 | 30 | Default |  |  | QPSK | 108@41 (A2) | 54@21 (A2) | 25@11 (A2) |
| 219 | 3565.02 | 30 | Default |  |  | QPSK | 74@41 (A1) | 24@54 (A1) | 11@27 (A1) |
| 220 | 3565.02 | 30 | Default |  |  | QPSK | Edge_1RB_Left (A7) |  |  |
| 221 | 3684.99 | 30 | Default |  |  | QPSK | Edge_1RB_Left (A7) |  |  |
| 222 | 3684.99 | 30 | Default |  |  | QPSK | 108@0 (A2) | 54@0 (A2) | 27@0 (A2) |
| 223 | 3684.99 | 30 | Default |  |  | QPSK | 81@0 (A1) | 40@0 (A1) | 20@0 (A1) |
| 224 | 3684.99 | 30 | Default |  |  | QPSK | Edge_1RB_Left (A7) |  |  |
| 225 | 3585 | 30 | Default |  |  | QPSK | Edge_1RB_Left (A8) |  |  |
| 226 | 3585 | 30 | Default |  |  | QPSK | Outer_Full (4) |  |  |
| 227 | 3664.98 | 30 | Default |  |  | QPSK | Edge_1RB_Right (A8) |  |  |
| 228 | 3664.98 | 30 | Default |  |  | QPSK | Outer_Full (4) |  |  |
| 229 | 3570 | 40 | Default |  |  | QPSK | Edge_1RB_Left (A7) |  |  |
| 230 | 3570 | 40 | Default |  |  | QPSK | 153@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 231 | 3570 | 40 | Default |  |  | QPSK | 99@69 (A1) | 49@34 (A1) | 24@16 (A1) |
| 232 | 3570 | 40 | Default |  |  | QPSK | Edge_1RB_Right (A7) |  |  |
| 233 | 3679.98 | 40 | Default |  |  | QPSK | Edge_1RB_Right (A7) |  |  |
| 234 | 3679.98 | 40 | Default |  |  | QPSK | 137@0 (A2) | 68@0 (A2) | 35@0 (A2) |
| 235 | 3679.98 | 40 | Default |  |  | QPSK | 99@38 (A1) | 49@18 (A1) | 24@9 (A1) |
| 236 | 3679.98 | 40 | Default |  |  | QPSK | Edge_1RB_Left (A7) |  |  |
| 237 | 3600 | 40 | Default |  |  | QPSK | Edge_1RB_Left (A8) |  |  |
| 238 | 3600 | 40 | Default |  |  | QPSK | Outer_Full (4.5) |  |  |
| 239 | 3649.98 | 40 | Default |  |  | QPSK | Edge_1RB_Right (A8) |  |  |
| 240 | 3649.98 | 40 | Default |  |  | QPSK | Outer_Full (4.5) |  |  |
| 241 | 3557.52 | 15 | Default |  |  | 16 QAM | Edge_1RB_Left (A3) |  |  |
| 242 | 3557.52 | 15 | Default |  |  | 16 QAM | Outer_Full (A3) |  |  |
| 243 | 3692.49 | 15 | Default |  |  | 16 QAM | Edge_1RB_Right (A3) |  |  |
| 244 | 3692.49 | 15 | Default |  |  | 16 QAM | Outer_Full (A3) |  |  |
| 245 | 3562.5 | 15 | Default |  |  | 16 QAM | Edge_1RB_Left (A4) |  |  |
| 246 | 3562.5 | 15 | Default |  |  | 16 QAM | Outer_Full (2) |  |  |
| 247 | 3687.48 | 15 | Default |  |  | 16 QAM | Edge_1RB_Right (A4) |  |  |
| 248 | 3687.48 | 15 | Default |  |  | 16 QAM | Outer_Full (2) |  |  |
| 249 | 3560.01 | 20 | Default |  |  | 16 QAM | Edge_1RB_Left (A5) |  |  |
| 250 | 3560.01 | 20 | Default |  |  | 16 QAM | Outer_Full (A5) |  |  |
| 251 | 3690 | 20 | Default |  |  | 16 QAM | Edge_1RB_Right (A5) |  |  |
| 252 | 3690 | 20 | Default |  |  | 16 QAM | Outer_Full (A5) |  |  |
| 253 | 3570 | 20 | Default |  |  | 16 QAM | Edge_1RB_Left (A6) |  |  |
| 254 | 3570 | 20 | Default |  |  | 16 QAM | Outer_Full (2) |  |  |

| A-MPR test parameters for NS_27 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 255 | 3679.98 | 20 | Default |  |  | 16 QAM | Edge_1RB_Right (A6) |  |  |
| 256 | 3679.98 | 20 | Default |  |  | 16 QAM | Outer_Full (2) |  |  |
| 257 | 3565.02 | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 258 | 3565.02 | 30 | Default |  |  | 16 QAM | 108@41 (A2) | 54@21 (A2) | 25@11 (A2) |
| 259 | 3565.02 | 30 | Default |  |  | 16 QAM | 74@41 (A1) | 24@54 (A1) | 11@27 (A1) |
| 260 | 3565.02 | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 261 | 3684.99 | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 262 | 3684.99 | 30 | Default |  |  | 16 QAM | 108@0 (A2) | 54@0 (A2) | 27@0 (A2) |
| 263 | 3684.99 | 30 | Default |  |  | 16 QAM | 81@0 (A1) | 40@0 (A1) | 20@0 (A1) |
| 264 | 3684.99 | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 265 | 3585 | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A8) |  |  |
| 266 | 3585 | 30 | Default |  |  | 16 QAM | Outer_Full (4) |  |  |
| 267 | 3664.98 | 30 | Default |  |  | 16 QAM | Edge_1RB_Right (A8) |  |  |
| 268 | 3664.98 | 30 | Default |  |  | 16 QAM | Outer_Full (4) |  |  |
| 269 | 3570 | 40 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 270 | 3570 | 40 | Default |  |  | 16 QAM | 153@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 271 | 3570 | 40 | Default |  |  | 16 QAM | 99@69 (A1) | 49@34 (A1) | 24@16 (A1) |
| 272 | 3570 | 40 | Default |  |  | 16 QAM | Edge_1RB_Right (A7) |  |  |
| 273 | 3679.98 | 40 | Default |  |  | 16 QAM | Edge_1RB_Right (A7) |  |  |
| 274 | 3679.98 | 40 | Default |  |  | 16 QAM | 137@0 (A2) | 68@0 (A2) | 35@0 (A2) |
| 275 | 3679.98 | 40 | Default |  |  | 16 QAM | 99@38 (A1) | 49@18 (A1) | 24@9 (A1) |
| 276 | 3679.98 | 40 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 277 | 3600 | 40 | Default |  |  | 16 QAM | Edge_1RB_Left (A8) |  |  |
| 278 | 3600 | 40 | Default |  |  | 16 QAM | Outer_Full (4.5) |  |  |
| 279 | 3649.98 | 40 | Default |  |  | 16 QAM | Edge_1RB_Right (A8) |  |  |
| 280 | 3649.98 | 40 | Default |  |  | 16 QAM | Outer_Full (4.5) |  |  |
| 281 | 3557.52 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left (A3) |  |  |
| 282 | 3557.52 | 15 | Default |  |  | 64 QAM | Outer_Full (A3) |  |  |
| 283 | 3692.49 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right (A3) |  |  |
| 284 | 3692.49 | 15 | Default |  |  | 64 QAM | Outer_Full (A3) |  |  |
| 285 | 3562.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left (A4) |  |  |
| 286 | 3562.5 | 15 | Default |  |  | 64 QAM | Outer_Full (2) |  |  |
| 287 | 3687.48 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right (A4) |  |  |
| 288 | 3687.48 | 15 | Default |  |  | 64 QAM | Outer_Full (2) |  |  |
| 289 | 3560.01 | 20 | Default |  |  | 64 QAM | Edge_1RB_Left (A5) |  |  |
| 290 | 3560.01 | 20 | Default |  |  | 64 QAM | Outer_Full (A5) |  |  |
| 291 | 3690 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right (A5) |  |  |
| 292 | 3690 | 20 | Default |  |  | 64 QAM | Outer_Full (A5) |  |  |
| 293 | 3570 | 20 | Default |  |  | 64 QAM | Edge_1RB_Left (A6) |  |  |
| 294 | 3570 | 20 | Default |  |  | 64 QAM | Outer_Full (2) |  |  |
| 295 | 3679.98 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right (A6) |  |  |
| 296 | 3679.98 | 20 | Default |  |  | 64 QAM | Outer_Full (2) |  |  |
| 297 | 3565.02 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 298 | 3565.02 | 30 | Default |  |  | 64 QAM | 108@41 (A2) | 54@21 (A2) | 25@11 (A2) |
| 299 | 3565.02 | 30 | Default |  |  | 64 QAM | 74@41 (A1) | 24@54 (A1) | 11@27 (A1) |
| 300 | 3565.02 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 301 | 3684.99 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 302 | 3684.99 | 30 | Default |  |  | 64 QAM | 108@0 (A2) | 54@0 (A2) | 27@0 (A2) |
| 303 | 3684.99 | 30 | Default |  |  | 64 QAM | 81@0 (A1) | 40@0 (A1) | 20@0 (A1) |
| 304 | 3684.99 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 305 | 3585 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A8) |  |  |
| 306 | 3585 | 30 | Default |  |  | 64 QAM | Outer_Full (4) |  |  |
| 307 | 3664.98 | 30 | Default |  |  | 64 QAM | Edge_1RB_Right (A8) |  |  |
| 308 | 3664.98 | 30 | Default |  |  | 64 QAM | Outer_Full (4) |  |  |
| 309 | 3570 | 40 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 310 | 3570 | 40 | Default |  |  | 64 QAM | 153@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 311 | 3570 | 40 | Default |  |  | 64 QAM | 99@69 (A1) | 49@34 (A1) | 24@16 (A1) |
| 312 | 3570 | 40 | Default |  |  | 64 QAM | Edge_1RB_Right (A7) |  |  |
| 313 | 3679.98 | 40 | Default |  |  | 64 QAM | Edge_1RB_Right (A7) |  |  |
| 314 | 3679.98 | 40 | Default |  |  | 64 QAM | 137@0 (A2) | 68@0 (A2) | 35@0 (A2) |
| 315 | 3679.98 | 40 | Default |  |  | 64 QAM | 99@38 (A1) | 49@18 (A1) | 24@9 (A1) |

| A-MPR test parameters for NS_27 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 316 | 3679.98 | 40 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 317 | 3600 | 40 | Default |  |  | 64 QAM | Edge_1RB_Left (A8) |  |  |
| 318 | 3600 | 40 | Default |  |  | 64 QAM | Outer_Full (4.5) |  |  |
| 319 | 3649.98 | 40 | Default |  |  | 64 QAM | Edge_1RB_Right (A8) |  |  |
| 320 | 3649.98 | 40 | Default |  |  | 64 QAM | Outer_Full (4.5) |  |  |
| 321 | 3557.52 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left (A3) |  |  |
| 322 | 3557.52 | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 323 | 3692.49 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |
| 324 | 3692.49 | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 325 | 3562.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left (A4) |  |  |
| 326 | 3562.5 | 15 | Default |  |  | 256 QAM | Outer_Full (2) |  |  |
| 327 | 3687.48 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right (A4) |  |  |
| 328 | 3687.48 | 15 | Default |  |  | 256 QAM | Outer_Full (2) |  |  |
| 329 | 3560.01 | 20 | Default |  |  | 256 QAM | Edge_1RB_Left (A5) |  |  |
| 330 | 3560.01 | 20 | Default |  |  | 256 QAM | Outer_Full (A5) |  |  |
| 331 | 3690 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right (A5) |  |  |
| 332 | 3690 | 20 | Default |  |  | 256 QAM | Outer_Full (A5) |  |  |
| 333 | 3570 | 20 | Default |  |  | 256 QAM | Edge_1RB_Left (A6) |  |  |
| 334 | 3570 | 20 | Default |  |  | 256 QAM | Outer_Full (2) |  |  |
| 335 | 3679.98 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right (A6) |  |  |
| 336 | 3679.98 | 20 | Default |  |  | 256 QAM | Outer_Full (2) |  |  |
| 337 | 3565.02 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 338 | 3565.02 | 30 | Default |  |  | 256 QAM | 108@41 (A2) | 54@21 (A2) | 25@11 (A2) |
| 339 | 3565.02 | 30 | Default |  |  | 256 QAM | 74@41 (A1) | 24@54 (A1) | 11@27 (A1) |
| 340 | 3565.02 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 341 | 3684.99 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 342 | 3684.99 | 30 | Default |  |  | 256 QAM | 108@0 (A2) | 54@0 (A2) | 27@0 (A2) |
| 343 | 3684.99 | 30 | Default |  |  | 256 QAM | 81@0 (A1) | 40@0 (A1) | 20@0 (A1) |
| 344 | 3684.99 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 345 | 3585 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A8) |  |  |
| 346 | 3585 | 30 | Default |  |  | 256 QAM | Outer_Full (4) |  |  |
| 347 | 3664.98 | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A8) |  |  |
| 348 | 3664.98 | 30 | Default |  |  | 256 QAM | Outer_Full (4) |  |  |
| 349 | 3570 | 40 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 350 | 3570 | 40 | Default |  |  | 256 QAM | 153@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 351 | 3570 | 40 | Default |  |  | 256 QAM | 99@69 (A1) | 49@34 (A1) | 24@16 (A1) |
| 352 | 3570 | 40 | Default |  |  | 256 QAM | Edge_1RB_Right (A7) |  |  |
| 353 | 3679.98 | 40 | Default |  |  | 256 QAM | Edge_1RB_Right (A7) |  |  |
| 354 | 3679.98 | 40 | Default |  |  | 256 QAM | 137@0 (A2) | 68@0 (A2) | 35@0 (A2) |
| 355 | 3679.98 | 40 | Default |  |  | 256 QAM | 99@38 (A1) | 49@18 (A1) | 24@9 (A1) |
| 356 | 3679.98 | 40 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 357 | 3600 | 40 | Default |  |  | 256 QAM | Edge_1RB_Left (A8) |  |  |
| 358 | 3600 | 40 | Default |  |  | 256 QAM | Outer_Full (4.5) |  |  |
| 359 | 3649.98 | 40 | Default |  |  | 256 QAM | Edge_1RB_Right (A8) |  |  |
| 360 | 3649.98 | 40 | Default |  |  | 256 QAM | Outer_Full (4.5) |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For FR1 bands where highest supported SCS is 60 kHz the highest tested SCS is limited to 30 kHz as carrier with SCS=60 kHz cannot be used as PCell. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-13a: Test frequencies for NR band n48 and NS_27 (SCS=15 kHz, ΔFRaster = 15kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] | kSSB | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA(SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 79 | 3557.52 | 637168 | 3550.41 | 636694 | 0 | 30 | 7884 | 636960 | 2 | 0 | 0 (2) | 2 |
|  |  | 3562.5 | 637500 | 3555.39 | 637026 |  |  | 7888 | 637344 | 6 | 0 | 1 (6) | 6 |
|  |  | 3687.48 | 645832 | 3680.37 | 645358 |  |  | 7975 | 645696 | 2 | 2 | 1 (6) | 8 |
|  |  | 3692.49 | 646166 | 3685.38 | 645692 |  |  | 7978 | 645984 | 4 | 2 | 0 (2) | 4 |
| 20 | 106 | 3560.01 | 637334 | 3550.47 | 636698 |  |  | 7885 | 637056 | 10 | 3 | 1 (6) | 9 |
|  |  | 3570 | 638000 | 3560.46 | 637364 |  |  | 7891 | 637632 | 4 | 0 | 0 (2) | 2 |
|  |  | 3679.98 | 645332 | 3670.44 | 644696 |  |  | 7968 | 645024 | 4 | 1 | 1 (6) | 7 |
|  |  | 3690 | 646000 | 3680.46 | 645364 |  |  | 7975 | 645696 | 8 | 1 | 1 (6) | 7 |
| 30 | 160 | 3565.02 | 637668 | 3550.62 | 636708 |  |  | 7885 | 637056 | 0 | 3 | 1 (6) | 9 |
|  |  | 3585 | 639000 | 3570.6 | 638040 |  |  | 7898 | 638304 | 0 | 0 | 0 (2) | 2 |
|  |  | 3664.98 | 644332 | 3650.58 | 643372 |  |  | 7954 | 643680 | 8 | 3 | 0 (2) | 5 |
|  |  | 3684.99 | 645666 | 3670.59 | 644706 |  |  | 7968 | 645024 | 6 | 0 | 1 (6) | 6 |
| 40 | 216 | 3570 | 638000 | 3550.56 | 636704 |  |  | 7885 | 637056 | 4 | 3 | 1 (6) | 9 |
|  |  | 3600 | 640000 | 3580.56 | 638704 |  |  | 7905 | 638976 | 8 | 0 | 0 (2) | 2 |
|  |  | 3649.98 | 643332 | 3630.54 | 642036 |  |  | 7940 | 642336 | 0 | 3 | 0 (2) | 5 |
|  |  | 3679.98 | 645332 | 3660.54 | 644036 |  |  | 7961 | 644352 | 4 | 0 | 1 (6) | 6 |
| NOTE 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. NOTE 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-13b: Test frequencies for NR band n48 and NS_27 (SCS=30 kHz, ΔFRaster = 30 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] | kSSB | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA(SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 38 | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
|  |  | 3562.5 | 637500 | 3555.66 | 637044 |  |  | 7888 | 637344 | 12 | 0 | 2 (2) | 4 |
|  |  | 3687.48 | 645832 | 3680.64 | 645376 |  |  | 7975 | 645696 | 8 | 0 | 3 (3) | 6 |
|  |  | 3692.49 | 646166 | 3685.65 | 645710 |  |  | 7978 | 645984 | 10 | 0 | 1 (1) | 2 |
| 20 | 51 | 3560.01 | 637334 | 3550.83 | 636722 |  |  | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
|  |  | 3570 | 638000 | 3560.82 | 637388 |  |  | 7891 | 637632 | 4 | 0 | 0 (0) | 0 |
|  |  | 3679.98 | 645332 | 3670.8 | 644720 |  |  | 7968 | 645024 | 16 | 0 | 2 (2) | 4 |
|  |  | 3690 | 646000 | 3680.82 | 645388 |  |  | 7975 | 645696 | 20 | 0 | 2 (2) | 4 |
| 30 | 78 | 3565.02 | 637668 | 3550.98 | 636732 |  |  | 7885 | 637056 | 12 | 0 | 3 (3) | 6 |
|  |  | 3585 | 639000 | 3570.96 | 638064 |  |  | 7898 | 638304 | 0 | 0 | 0 (0) | 0 |
|  |  | 3664.98 | 644332 | 3650.94 | 643396 |  |  | 7954 | 643680 | 20 | 0 | 1 (1) | 2 |
|  |  | 3684.99 | 645666 | 3670.95 | 644730 |  |  | 7968 | 645024 | 6 | 0 | 2 (2) | 4 |
| 40 | 106 | 3570 | 638000 | 3550.92 | 636728 |  |  | 7885 | 637056 | 16 | 0 | 3 (3) | 6 |
|  |  | 3600 | 640000 | 3580.92 | 638728 |  |  | 7905 | 638976 | 8 | 0 | 0 (0) | 0 |
|  |  | 3649.98 | 643332 | 3630.9 | 642060 |  |  | 7940 | 642336 | 12 | 0 | 1 (1) | 2 |
|  |  | 3679.98 | 645332 | 3660.9 | 644060 |  |  | 7961 | 644352 | 4 | 0 | 2 (2) | 4 |
| NOTE 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. NOTE 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-14: Test Configuration table for NS_40

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest (5 MHz) |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |
| A-MPR test parameters for NS_40 |  |  |  |  |  |  |  |
| Test ID | Freq | Ch BW | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | Low | Default | Default | N/A for A-MPR testing. | DFT-s-OFDM | QPSK | Edge_1RB_Left |
| 2 | Default | Default | Default |  |  | QPSK | Inner_Full |
| 3 | Default | Default | Default |  |  | QPSK | Outer_Full |
| 4 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |
| 5 | Default | Default | Default |  |  | 16 QAM | Inner_Full |
| 6 | Default | Default | Default |  |  | 16 QAM | Outer_Full |
| 7 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |
| 8 | Default | Default | Default |  |  | 64 QAM | Inner_Full |
| 9 | Default | Default | Default |  |  | 64 QAM | Outer_Full |
| 10 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |
| 11 | Default | Default | Default |  |  | 256 QAM | Inner_Full |
| 12 | Default | Default | Default |  |  | 256 QAM | Outer_Full |
| 13 | Low | Default | Default |  | CP-OFDM | QPSK | Edge_1RB_Left |
| 14 | Default | Default | Default |  |  | QPSK | Inner_Full |
| 15 | Default | Default | Default |  |  | QPSK | Outer_Full |
| 16 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |
| 17 | Default | Default | Default |  |  | 16 QAM | Inner_Full |
| 18 | Default | Default | Default |  |  | 16 QAM | Outer_Full |
| 19 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |
| 20 | Default | Default | Default |  |  | 64 QAM | Inner_Full |
| 21 | Default | Default | Default |  |  | 64 QAM | Outer_Full |
| 22 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |
| 23 | Default | Default | Default |  |  | 256 QAM | Inner_Full |
| 24 | Default | Default | Default |  |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-15: Test Configuration table for NS_41

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | See Ch BW column |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |
| A-MPR test parameters for NS_41 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | 1437 | 10 | Default | N/A for A-MPR testing. | DFT-s-OFDM | PI/2 BPSK | Outer_Full |
| 2 | 1437 | 10 | Default |  |  | PI/2 BPSK | Edge_1RB_Left |
| 3 | 1439.5 | 15 | Default |  |  | PI/2 BPSK | Outer_Full |
| 4 | 1439.5 | 15 | Default |  |  | PI/2 BPSK | Edge_1RB_Left |
| 5 | 1442 | 20 | Default |  |  | PI/2 BPSK | Outer_Full |
| 6 | 1442 | 20 | Default |  |  | PI/2 BPSK | Edge_1RB_Left |
| 7 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | PI/2 BPSK | Outer_Full |
| 8 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | PI/2 BPSK | Edge_1RB_Left |
| 9 | 1437 | 10 | Default |  |  | QPSK | Outer_Full |
| 10 | 1437 | 10 | Default |  |  | QPSK | Edge_1RB_Left |
| 11 | 1439.5 | 15 | Default |  |  | QPSK | Outer_Full |
| 12 | 1439.5 | 15 | Default |  |  | QPSK | Edge_1RB_Left |
| 13 | 1442 | 20 | Default |  |  | QPSK | Outer_Full |
| 14 | 1442 | 20 | Default |  |  | QPSK | Edge_1RB_Left |
| 15 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | QPSK | Outer_Full |
| 16 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | QPSK | Edge_1RB_Left |
| 17 | 1437 | 10 | Default |  |  | 16 QAM | Outer_Full |
| 18 | 1437 | 10 | Default |  |  | 16 QAM | Edge_1RB_Left |
| 19 | 1439.5 | 15 | Default |  |  | 16 QAM | Outer_Full |
| 20 | 1439.5 | 15 | Default |  |  | 16 QAM | Edge_1RB_Left |
| 21 | 1442 | 20 | Default |  |  | 16 QAM | Outer_Full |
| 22 | 1442 | 20 | Default |  |  | 16 QAM | Edge_1RB_Left |
| 23 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | 16 QAM | Outer_Full |
| 24 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | 16 QAM | Edge_1RB_Left |
| 25 | 1437 | 10 | Default |  |  | 64 QAM | Outer_Full |
| 26 | 1437 | 10 | Default |  |  | 64 QAM | Edge_1RB_Left |
| 27 | 1439.5 | 15 | Default |  |  | 64 QAM | Outer_Full |
| 28 | 1439.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left |
| 29 | 1442 | 20 | Default |  |  | 64 QAM | Outer_Full |
| 30 | 1442 | 20 | Default |  |  | 64 QAM | Edge_1RB_Left |
| 31 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | 64 QAM | Outer_Full |
| 32 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | 64 QAM | Edge_1RB_Left |
| 33 | 1437 | 10 | Default |  |  | 256 QAM | Outer_Full |
| 34 | 1437 | 10 | Default |  |  | 256 QAM | Edge_1RB_Left |
| 35 | 1439.5 | 15 | Default |  |  | 256 QAM | Outer_Full |
| 36 | 1439.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left |
| 37 | 1442 | 20 | Default |  |  | 256 QAM | Outer_Full |
| 38 | 1442 | 20 | Default |  |  | 256 QAM | Edge_1RB_Left |
| 39 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | 256 QAM | Outer_Full |
| 40 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | 256 QAM | Edge_1RB_Left |
| 41 | 1437 | 10 | Default |  | CP-OFDM | QPSK | Outer_Full |
| 42 | 1437 | 10 | Default |  |  | QPSK | Edge_1RB_Left |
| 43 | 1439.5 | 15 | Default |  |  | QPSK | Outer_Full |

| A-MPR test parameters for NS_41 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 44 | 1439.5 | 15 | Default |  |  | QPSK | Edge_1RB_Left |
| 45 | 1442 | 20 | Default |  |  | QPSK | Outer_Full |
| 46 | 1442 | 20 | Default |  |  | QPSK | Edge_1RB_Left |
| 47 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | QPSK | Outer_Full |
| 48 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | QPSK | Edge_1RB_Left |
| 49 | 1437 | 10 | Default |  |  | 16 QAM | Outer_Full |
| 50 | 1437 | 10 | Default |  |  | 16 QAM | Edge_1RB_Left |
| 51 | 1439.5 | 15 | Default |  |  | 16 QAM | Outer_Full |
| 52 | 1439.5 | 15 | Default |  |  | 16 QAM | Edge_1RB_Left |
| 53 | 1442 | 20 | Default |  |  | 16 QAM | Outer_Full |
| 54 | 1442 | 20 | Default |  |  | 16 QAM | Edge_1RB_Left |
| 55 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | 16 QAM | Outer_Full |
| 56 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | 16 QAM | Edge_1RB_Left |
| 57 | 1437 | 10 | Default |  |  | 64 QAM | Outer_Full |
| 58 | 1437 | 10 | Default |  |  | 64 QAM | Edge_1RB_Left |
| 59 | 1439.5 | 15 | Default |  |  | 64 QAM | Outer_Full |
| 60 | 1439.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left |
| 61 | 1442 | 20 | Default |  |  | 64 QAM | Outer_Full |
| 62 | 1442 | 20 | Default |  |  | 64 QAM | Edge_1RB_Left |
| 63 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | 64 QAM | Outer_Full |
| 64 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | 64 QAM | Edge_1RB_Left |
| 65 | 1437 | 10 | Default |  |  | 256 QAM | Outer_Full |
| 66 | 1437 | 10 | Default |  |  | 256 QAM | Edge_1RB_Left |
| 67 | 1439.5 | 15 | Default |  |  | 256 QAM | Outer_Full |
| 68 | 1439.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left |
| 69 | 1442 | 20 | Default |  |  | 256 QAM | Outer_Full |
| 70 | 1442 | 20 | Default |  |  | 256 QAM | Edge_1RB_Left |
| 71 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | 256 QAM | Outer_Full |
| 72 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | 256 QAM | Edge_1RB_Left |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Select Fc = 1452 MHz for 40 MHz Ch BW, Fc = 1457 MHz for 50 MHz Ch BW and Fc = 1462 MHz for 60 MHz Ch BW. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-16: Test Configuration table for NS_42

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | See Ch BW column |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_42 |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  |  | RB allocation (Note 1, 3) |
| 1 | 1514.5 | 5 | Default | N/A for A-MPR testing. | DFT-s-OFDM | PI/2 BPSK |  | Outer_Full |
| 2 | 1514.5 | 5 | Default |  |  | PI/2 BPSK |  | Outer (LCRB@RBstart) |
| 3 | 1514.5 | 5 | Default |  |  | PI/2 BPSK |  | Inner (LCRB@RBstart) |
| 4 | (Note 3) | Highest (≤ 60) | Default |  |  | PI/2 BPSK |  | Outer_Full |
| 5 | (Note 3) | Highest (≤ 60) | Default |  |  | PI/2 BPSK |  | Outer (LCRB@RBstart) |
| 6 | (Note 3) | Highest (≤ 60) | Default |  |  | PI/2 BPSK |  | Inner (LCRB@RBstart) |
| 7 | 1514.5 | 5 | Default |  |  | QPSK |  | Outer_Full |
| 8 | 1514.5 | 5 | Default |  |  | QPSK |  | Outer (LCRB@RBstart) |
| 9 | 1514.5 | 5 | Default |  |  | QPSK |  | Inner (LCRB@RBstart) |
| 10 | (Note 3) | Highest (≤ 60) | Default |  |  | QPSK |  | Outer_Full |
| 11 | (Note 3) | Highest (≤ 60) | Default |  |  | QPSK |  | Outer (LCRB@RBstart) |
| 12 | (Note 3) | Highest (≤ 60) | Default |  |  | QPSK |  | Inner (LCRB@RBstart) |
| 13 | 1514.5 | 5 | Default |  |  | 16 QAM |  | Outer_Full |
| 14 | 1514.5 | 5 | Default |  |  | 16 QAM |  | Outer (LCRB@RBstart) |
| 15 | 1514.5 | 5 | Default |  |  | 16 QAM |  | Inner (LCRB@RBstart) |
| 16 | (Note 3) | Highest (≤ 60) | Default |  |  | 16 QAM |  | Outer_Full |
| 17 | (Note 3) | Highest (≤ 60) | Default |  |  | 16 QAM |  | Outer (LCRB@RBstart) |
| 18 | (Note 3) | Highest (≤ 60) | Default |  |  | 16 QAM |  | Inner (LCRB@RBstart) |
| 19 | 1514.5 | 5 | Default |  |  | 64 QAM |  | Outer_Full |
| 20 | 1514.5 | 5 | Default |  |  | 64 QAM |  | Outer (LCRB@RBstart) |
| 21 | 1514.5 | 5 | Default |  |  | 64 QAM |  | Inner (LCRB@RBstart) |
| 22 | (Note 3) | Highest (≤ 60) | Default |  |  | 64 QAM |  | Outer_Full |
| 23 | (Note 3) | Highest (≤ 60) | Default |  |  | 64 QAM |  | Outer (LCRB@RBstart) |
| 24 | (Note 3) | Highest (≤ 60) | Default |  |  | 64 QAM |  | Inner (LCRB@RBstart) |
| 25 | 1514.5 | 5 | Default |  |  | 256 QAM |  | Outer_Full |
| 26 | 1514.5 | 5 | Default |  |  | 256 QAM |  | Outer (LCRB@RBstart) |
| 27 | 1514.5 | 5 | Default |  |  | 256 QAM |  | Inner (LCRB@RBstart) |
| 28 | (Note 3) | Highest (≤ 60) | Default |  |  | 256 QAM |  | Outer_Full |
| 29 | (Note 3) | Highest (≤ 60) | Default |  |  | 256 QAM |  | Outer (LCRB@RBstart) |
| 30 | (Note 3) | Highest (≤ 60) | Default |  |  | 256 QAM |  | Inner (LCRB@RBstart) |
| 31 | 1514.5 | 5 | Default |  | CP-OFDM | QPSK |  | Outer_Full |
| 32 | 1514.5 | 5 | Default |  |  | QPSK |  | Outer (LCRB@RBstart) |
| 33 | 1514.5 | 5 | Default |  |  | QPSK |  | Inner (LCRB@RBstart) |
| 34 | (Note 3) | Highest (≤ 60) | Default |  |  | QPSK |  | Outer_Full |
| 35 | (Note 3) | Highest (≤ 60) | Default |  |  | QPSK |  | Outer (LCRB@RBstart) |
| 36 | (Note 3) | Highest (≤ 60) | Default |  |  | QPSK |  | Inner (LCRB@RBstart) |
| 37 | 1514.5 | 5 | Default |  |  | 16 QAM |  | Outer_Full |
| 38 | 1514.5 | 5 | Default |  |  | 16 QAM |  | Outer (LCRB@RBstart) |
| 39 | 1514.5 | 5 | Default |  |  | 16 QAM |  | Inner (LCRB@RBstart) |
| 40 | (Note 3) | Highest (≤ 60) | Default |  |  | 16 QAM |  | Outer_Full |
| 41 | (Note 3) | Highest (≤ 60) | Default |  |  | 16 QAM |  | Outer (LCRB@RBstart) |
| 42 | (Note 3) | Highest (≤ 60) | Default |  |  | 16 QAM |  | Inner (LCRB@RBstart) |
| 43 | 1514.5 | 5 | Default |  |  | 64 QAM |  | Outer_Full |
| 44 | 1514.5 | 5 | Default |  |  | 64 QAM |  | Outer (LCRB@RBstart) |
| 45 | 1514.5 | 5 | Default |  |  | 64 QAM |  | Inner (LCRB@RBstart) |
| 46 | (Note 3) | Highest (≤ 60) | Default |  |  | 64 QAM |  | Outer_Full |
| 47 | (Note 3) | Highest (≤ 60) | Default |  |  | 64 QAM |  | Outer (LCRB@RBstart) |
| 48 | (Note 3) | Highest (≤ 60) | Default |  |  | 64 QAM |  | Inner (LCRB@RBstart) |
| 49 | 1514.5 | 5 | Default |  |  | 256 QAM |  | Outer_Full |
| 50 | 1514.5 | 5 | Default |  |  | 256 QAM |  | Outer (LCRB@RBstart) |

| A-MPR test parameters for NS_42 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1, 3) |
| 51 | 1514.5 | 5 | Default |  |  | 256 QAM | Inner (LCRB@RBstart) |
| 52 | (Note 3) | Highest (≤ 60) | Default |  |  | 256 QAM | Outer_Full |
| 53 | (Note 3) | Highest (≤ 60) | Default |  |  | 256 QAM | Outer (LCRB@RBstart) |
| 54 | (Note 3) | Highest (≤ 60) | Default |  |  | 256 QAM | Inner (LCRB@RBstart) |
| NOTE 1: The Outer_Full RB configuration is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For 10 – 60 MHz channel bandwidth the RB allocation and test frequency are specified in Table 6.2.3.4.1-16a. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-16a: Uplink configuration for NS_42

| Ch BW (MHz) | Fc (MHz) | SCS (kHz) | OFDM | Outer LCRB@RBstart | Inner LCRB@RBstart |
| --- | --- | --- | --- | --- | --- |
| 5 | 1514.5 | 15 | DFT-s | 16@0 | 9@4 |
|  |  |  | CP | 17@0 | 9@4 |
|  |  | 30 | DFT-s | 8@0 | 5@2 |
|  |  |  | CP | 8@0 | 5@2 |
| 10 | 1512.0 | 15 | DFT-s | 32@0 | 9@4 |
|  |  |  | CP | 34@0 | 9@4 |
|  |  | 30 | DFT-s | 16@0 | 5@2 |
|  |  |  | CP | 17@0 | 5@2 |
|  |  | 60 | DFT-s | 8@0 | 3@1 |
|  |  |  | CP | 8@0 | 3@1 |
| 15 | 1509.5 | 15 | DFT-s | 50@0 | 32@16 |
|  |  |  | CP | 51@0 | 33@16 |
|  |  | 30 | DFT-s | 25@0 | 16@8 |
|  |  |  | CP | 25@0 | 17@8 |
|  |  | 60 | DFT-s | 12@0 | 9@4 |
|  |  |  | CP | 12@0 | 9@4 |
| 20 | 1507.0 | 15 | DFT-s | 64@0 | 48@24 |
|  |  |  | CP | 68@0 | 49@24 |
|  |  | 30 | DFT-s | 32@0 | 25@12 |
|  |  |  | CP | 34@0 | 25@12 |
|  |  | 60 | DFT-s | 16@0 | 12@6 |
|  |  |  | CP | 17@0 | 12@6 |
| 40 | 1497.0 | 15 | DFT-s | 135@0 | 54@29 |
|  |  |  | CP | 137@0 | 59@29 |
|  |  | 30 | DFT-s | 64@0 | 27@14 |
|  |  |  | CP | 68@0 | 29@14 |
|  |  | 60 | DFT-s | 32@0 | 15@7 |
|  |  |  | CP | 34@0 | 15@7 |
| 50 | 1492.0 | 15 | DFT-s | 162@0 | 75@39 |
|  |  |  | CP | 172@0 | 79@39 |
|  |  | 30 | DFT-s | 81@0 | 36@19 |
|  |  |  | CP | 86@0 | 39@19 |
|  |  | 60 | DFT-s | 40@0 | 18@9 |
|  |  |  | CP | 43@0 | 19@9 |
| 60 | 1487.0 | 30 | DFT-s | 100@0 | 36@19 |
|  |  |  | CP | 103@0 | 39@19 |
|  |  | 60 | DFT-s | 50@0 | 18@9 |
|  |  |  | CP | 51@0 | 19@9 |

Table 6.2.3.4.1-17: Test Configuration table for NS_47 power class 3 (contiguous allocation)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies |  |  |  |  |  |  |  | As specified in Table 6.2.3.4.1-18a and 6.2.3.4.1-18b |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 30 MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest (Note 6) |  |  |
| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 (Note 3) | Default | 30 | Default | N/A for A-MPR testing. | DFT-s-OFDM | PI/2 BPSK | Edge_1RB_Left (A1) |  |  |  |
| 2 (Note 3) | Default | 30 | Default |  |  | PI/2 BPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 3 (Note 3) | Default | 30 | Default |  |  | PI/2 BPSK | Edge_1RB_Right (A3) |  |  |  |
| 4 (Note 3) | Default | 30 | Default |  |  | PI/2 BPSK | Outer_Full (A2) |  |  |  |
| 5 (Note 3) | Default | 30 | Default |  |  | PI/2 BPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 6 (Note 3) | Default | 30 | Default |  |  | PI/2 BPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 7 (Note 3) | Default | 30 | Default |  |  | PI/2 BPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 8 (Note 4) | Default | 30 | Default |  |  | PI/2 BPSK | Edge_1RB_Left (A1) |  |  |  |
| 9 (Note 4) | Default | 30 | Default |  |  | PI/2 BPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 10 (Note 4) | Default | 30 | Default |  |  | PI/2 BPSK | Edge_1RB_Right (A3) |  |  |  |
| 11 (Note 4) | Default | 30 | Default |  |  | PI/2 BPSK | Outer_Full (A2) |  |  |  |
| 12 (Note 4) | Default | 30 | Default |  |  | PI/2 BPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 13 (Note 4) | Default | 30 | Default |  |  | PI/2 BPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 14 (Note 4) | Default | 30 | Default |  |  | PI/2 BPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 15 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Left (A1) |  |  |  |
| 16 | Default | 30 | Default |  |  | QPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 17 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 18 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 19 | Default | 30 | Default |  |  | QPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 20 | Default | 30 | Default |  |  | QPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 21 | Default | 30 | Default |  |  | QPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 22 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A1) |  |  |  |
| 23 | Default | 30 | Default |  |  | 16 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 24 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Right (A3) |  |  |  |
| 25 | Default | 30 | Default |  |  | 16 QAM | Outer_Full (A2) |  |  |  |
| 26 | Default | 30 | Default |  |  | 16 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 27 | Default | 30 | Default |  |  | 16 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 28 | Default | 30 | Default |  |  | 16 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 29 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A1) |  |  |  |
| 30 | Default | 30 | Default |  |  | 64 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 31 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Right (A3) |  |  |  |
| 32 | Default | 30 | Default |  |  | 64 QAM | Outer_Full (A2) |  |  |  |
| 33 | Default | 30 | Default |  |  | 64 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 34 | Default | 30 | Default |  |  | 64 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 35 | Default | 30 | Default |  |  | 64 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 36 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A1) |  |  |  |
| 37 | Default | 30 | Default |  |  | 256 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 38 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |  |
| 39 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 40 | Default | 30 | Default |  |  | 256 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 41 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 42 | Default | 30 | Default |  |  | 256 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 43 | Default | 30 | Default |  | CP-OFDM | QPSK | Edge_1RB_Left (A1) |  |  |  |
| 44 | Default | 30 | Default |  |  | QPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 45 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 46 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 47 | Default | 30 | Default |  |  | QPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 48 | Default | 30 | Default |  |  | QPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 49 | Default | 30 | Default |  |  | QPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 50 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A1) |  |  |  |

| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 51 | Default | 30 | Default |  |  | 16 QAM | 1@29 (A2) | 1@15 (A2) | 1@8 (A2) |
| 52 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Right (A3) |  |  |
| 53 | Default | 30 | Default |  |  | 16 QAM | Outer_Full (A2) |  |  |
| 54 | Default | 30 | Default |  |  | 16 QAM | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 55 | Default | 30 | Default |  |  | 16 QAM | 80@0 (A4) | 40@0 (A4) | 20@0 (A4) |
| 56 | Default | 30 | Default |  |  | 16 QAM | 54@0 (A2) | 27@0 (A2) | 12@0 (A2) |
| 57 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A1) |  |  |
| 58 | Default | 30 | Default |  |  | 64 QAM | 1@29 (A2) | 1@15 (A2) | 1@8 (A2) |
| 59 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Right (A3) |  |  |
| 60 | Default | 30 | Default |  |  | 64 QAM | Outer_Full (A2) |  |  |
| 61 | Default | 30 | Default |  |  | 64 QAM | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 62 | Default | 30 | Default |  |  | 64 QAM | 80@0 (A4) | 40@0 (A4) | 20@0 (A4) |
| 63 | Default | 30 | Default |  |  | 64 QAM | 54@0 (A2) | 27@0 (A2) | 12@0 (A2) |
| 64 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A1) |  |  |
| 65 | Default | 30 | Default |  |  | 256 QAM | 1@29 (A2) | 1@15 (A2) | 1@8 (A2) |
| 66 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |
| 67 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |
| 68 | Default | 30 | Default |  |  | 256 QAM | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 69 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A4) | 40@0 (A4) | 20@0 (A4) |
| 70 | Default | 30 | Default |  |  | 256 QAM | 54@0 (A2) | 27@0 (A2) | 12@0 (A2) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n41. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n41, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n41. NOTE 5: Void NOTE 6: For FR1 bands where highest supported SCS is 60 kHz the highest tested SCS is limited to 30 kHz as carrier with SCS=60 kHz cannot be used as PCell. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-17a: Test Configuration table for NS_47 power class 3 (almost contiguous allocation)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |
| Test Frequencies |  |  |  |  |  |  | As specified in Table 6.2.3.4.1-18a and 6.2.3.4.1-18b |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 30 MHz |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest (Note 3) |
| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation |  | RB allocation (Note 1) |
| 1 | Default | 30 | Default |  | CP-OFDM | QPSK | Outer_Full (A2) |
| 2 | Default | 30 | Default |  |  | 16 QAM | Outer_Full (A2) |
| 3 | Default | 30 | Default |  |  | 64 QAM | Outer_Full (A2) |
| 4 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.2.2.4.1-4, since offsetToCarrier = 0 in Tables 6.2.3.4.1-18a and 6.2.3.4.1-18b. NOTE 2: Void NOTE 3: For FR1 bands where highest supported SCS is 60 kHz the highest tested SCS is limited to 30 kHz as carrier with SCS=60 kHz cannot be used as PCell. NOTE 4: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-18: Test Configuration table for NS_47 power class 2 (contiguous allocation)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies |  |  |  |  |  |  |  | As specified in Table 6.2.3.4.1-18a and 6.2.3.4.1-18b |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 30 MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest (Note 3) |  |  |
| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 30 | Default | N/A for A-MPR testing. | DFT-s-OFDM | PI/2 BPSK | Edge_1RB_Left (A1) |  |  |  |
| 2 | Default | 30 | Default |  |  | PI/2 BPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 3 | Default | 30 | Default |  |  | PI/2 BPSK | Edge_1RB_Right (A3) |  |  |  |
| 4 | Default | 30 | Default |  |  | PI/2 BPSK | Outer_Full (A2) |  |  |  |
| 5 | Default | 30 | Default |  |  | PI/2 BPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 6 | Default | 30 | Default |  |  | PI/2 BPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 7 | Default | 30 | Default |  |  | PI/2 BPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 8 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Left (A1) |  |  |  |
| 9 | Default | 30 | Default |  |  | QPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 10 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 11 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 12 | Default | 30 | Default |  |  | QPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 13 | Default | 30 | Default |  |  | QPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 14 | Default | 30 | Default |  |  | QPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 15 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A1) |  |  |  |
| 16 | Default | 30 | Default |  |  | 16 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 17 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Right (A3) |  |  |  |
| 18 | Default | 30 | Default |  |  | 16 QAM | Outer_Full (A2) |  |  |  |
| 19 | Default | 30 | Default |  |  | 16 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 20 | Default | 30 | Default |  |  | 16 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 21 | Default | 30 | Default |  |  | 16 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 22 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A1) |  |  |  |
| 23 | Default | 30 | Default |  |  | 64 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 24 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Right (A3) |  |  |  |
| 25 | Default | 30 | Default |  |  | 64 QAM | Outer_Full (A2) |  |  |  |
| 26 | Default | 30 | Default |  |  | 64 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 27 | Default | 30 | Default |  |  | 64 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 28 | Default | 30 | Default |  |  | 64 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 29 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A1) |  |  |  |
| 30 | Default | 30 | Default |  |  | 256 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 31 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |  |
| 32 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 33 | Default | 30 | Default |  |  | 256 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 34 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 35 | Default | 30 | Default |  |  | 256 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 36 | Default | 30 | Default |  | CP-OFDM | QPSK | Edge_1RB_Left (A1) |  |  |  |
| 37 | Default | 30 | Default |  |  | QPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 38 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 39 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 40 | Default | 30 | Default |  |  | QPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 41 | Default | 30 | Default |  |  | QPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 42 | Default | 30 | Default |  |  | QPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 43 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A1) |  |  |  |
| 44 | Default | 30 | Default |  |  | 16 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 45 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Right (A3) |  |  |  |
| 46 | Default | 30 | Default |  |  | 16 QAM | Outer_Full (A2) |  |  |  |
| 47 | Default | 30 | Default |  |  | 16 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 48 | Default | 30 | Default |  |  | 16 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 49 | Default | 30 | Default |  |  | 16 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 50 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A1) |  |  |  |

| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
| 51 | Default | 30 | Default |  |  | 64 QAM | 1@29 (A2) | 1@15 (A2) | 1@8 (A2) |
| 52 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Right (A3) |  |  |
| 53 | Default | 30 | Default |  |  | 64 QAM | Outer_Full (A2) |  |  |
| 54 | Default | 30 | Default |  |  | 64 QAM | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 55 | Default | 30 | Default |  |  | 64 QAM | 80@0 (A4) | 40@0 (A4) | 20@0 (A4) |
| 56 | Default | 30 | Default |  |  | 64 QAM | 54@0 (A2) | 27@0 (A2) | 12@0 (A2) |
| 57 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A1) |  |  |
| 58 | Default | 30 | Default |  |  | 256 QAM | 1@29 (A2) | 1@15 (A2) | 1@8 (A2) |
| 59 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |
| 60 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |
| 61 | Default | 30 | Default |  |  | 256 QAM | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 62 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A4) | 40@0 (A4) | 20@0 (A4) |
| 63 | Default | 30 | Default |  |  | 256 QAM | 54@0 (A2) | 27@0 (A2) | 12@0 (A2) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For FR1 bands where highest supported SCS is 60 kHz the highest tested SCS is limited to 30 kHz as carrier with SCS=60 kHz cannot be used as PCell. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-18-1A: Test Configuration table for NS_47 power class 2 (almost contiguous allocation)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |
| Test Frequencies |  |  |  |  |  |  | As specified in Table 6.2.3.4.1-18a and 6.2.3.4.1-18b |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 30 MHz |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest (Note 3) |
| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation |  | RB allocation (Note 1) |
| 1 | Default | 30 | Default |  | CP-OFDM | QPSK | Outer_Full (A2) |
| 2 | Default | 30 | Default |  |  | 16 QAM | Outer_Full (A2) |
| 3 | Default | 30 | Default |  |  | 64 QAM | Outer_Full (A2) |
| 4 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.2.2.4.1-4, since offsetToCarrier = 0 in Tables 6.2.3.4.1-18a and 6.2.3.4.1-18b. NOTE 2: Void NOTE 3: For FR1 bands where highest supported SCS is 60 kHz the highest tested SCS is limited to 30 kHz as carrier with SCS=60 kHz cannot be used as PCell. NOTE 4: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-18a: Test frequencies for NS_47 (SCS=15 kHz, ΔFRaster = 15kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] | kSSB | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA(SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30 | 160 | 2560.005 | 512001 | 2545.605 | 509121 | 0 | 15 | 6369 | 509550 | 11 | 1 | 0 (0) | 1 |
| NOTE 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. NOTE 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-18b: Test frequencies for NS_47 (SCS=30 kHz, ΔFRaster = 30 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] | kSSB | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA(SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30 | 78 | 2560.02 | 512004 | 2545.98 | 509196 | 0 | 30 | 6375 | 510030 | 14 | 0 | 1 (1) | 2 |
| NOTE 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. NOTE 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-19: Test Configuration table for NS_48 (Power class 3)

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies |  |  |  |  |  |  | lowest and highest carrier centre frequency for each CBW in table 6.2.3.3.26-1 |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 25 MHz, 30MHz, 40MHz, 45MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_48 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 25 | Default | N/A for A-MPR testing. | DFT-s-OFDM | QPSK | Outer_Full (A3) |  |  |
| 2 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |
| 3 | Default | 30 | Default |  |  | QPSK | Outer_Full (A3) |  |  |
| 4 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |
| 5 | Default | 40 | Default |  |  | QPSK | 16@0 (A2) | 8@0 (A2) | 4@0 (A2) |
| 6 | Default | 40 | Default |  |  | QPSK | 90@0 (A3) | 48@0 (A3) | 24@0 (A3) |
| 7 | Default | 40 | Default |  |  | QPSK | 150@0 (A4) | 75@0 (A4) | 36@0 (A4) |
| 8 | Default | 40 | Default |  |  | QPSK | 192@0 (A2) | 96@0 (A2) | 48@0 (A2) |
| 9 | Default | 40 | Default |  |  | QPSK | 5@187 (A3) | 2@94 (A3) | 1@47 (A3) |
| 10 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 11 | Default | 45 | Default |  |  | QPSK | 27@0 (A2) | 12@0 (A2) | 6@0 (A2) |
| 12 | Default | 45 | Default |  |  | QPSK | 100@0 (A4) | 50@0 (A4) | 27@0 (A4) |
| 13 | Default | 45 | Default |  |  | QPSK | 200@0 (A2) | 100@0 (A2) | 50@0 (A2) |
| 14 | Default | 45 | Default |  |  | QPSK | 5@203 (A5) | 2@102 (A5) | 1@51 (A5) |
| 15 | Default | 45 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 16 | Default | 50 | Default |  |  | QPSK | 32@0 (A2) | 16@0 (A2) | 8@0 (A2) |
| 17 | Default | 50 | Default |  |  | QPSK | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 18 | Default | 50 | Default |  |  | QPSK | 225@0 (A2) | 108@0 (A2) | 54@0 (A2) |
| 19 | Default | 50 | Default |  |  | QPSK | 5@223 (A5) | 2@112 (A5) | 1@56 (A5) |
| 20 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 21 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 22 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |
| 23 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 24 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A5) |  |  |
| 25 | Default | 40 | Default |  |  | 256 QAM | 16@0 (A2) | 8@0 (A2) | 4@0 (A2) |
| 26 | Default | 40 | Default |  |  | 256 QAM | 90@0 (A3) | 48@0 (A3) | 24@0 (A3) |
| 27 | Default | 40 | Default |  |  | 256 QAM | 150@0 (A4) | 75@0 (A4) | 36@0 (A4) |
| 28 | Default | 40 | Default |  |  | 256 QAM | 192@0 (A2) | 96@0 (A2) | 48@0 (A2) |
| 29 | Default | 40 | Default |  |  | 256 QAM | 5@187 (A3) | 2@94 (A3) | 1@47 (A3) |
| 30 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 31 | Default | 45 | Default |  |  | 256 QAM | 27@0 (A2) | 12@0 (A2) | 6@0 (A2) |
| 32 | Default | 45 | Default |  |  | 256 QAM | 100@0 (A4) | 50@0 (A4) | 27@0 (A4) |
| 33 | Default | 45 | Default |  |  | 256 QAM | 200@0 (A2) | 100@0 (A2) | 50@0 (A2) |
| 34 | Default | 45 | Default |  |  | 256 QAM | 5@203 (A5) | 2@102 (A5) | 1@51 (A5) |
| 35 | Default | 45 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 36 | Default | 50 | Default |  |  | 256 QAM | 32@0 (A2) | 16@0 (A2) | 8@0 (A2) |
| 37 | Default | 50 | Default |  |  | 256 QAM | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 38 | Default | 50 | Default |  |  | 256 QAM | 225@0 (A2) | 108@0 (A2) | 54@0 (A2) |
| 39 | Default | 50 | Default |  |  | 256 QAM | 5@223 (A5) | 2@112 (A5) | 1@56 (A5) |
| 40 | Default | 50 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 41 | Default | 25 | Default |  | CP-OFDM | QPSK | Outer_Full (A3) |  |  |
| 42 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |
| 43 | Default | 30 | Default |  |  | QPSK | Outer_Full (A3) |  |  |
| 44 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |
| 45 | Default | 40 | Default |  |  | QPSK | 16@0 (A2) | 8@0 (A2) | 4@0 (A2) |
| 46 | Default | 40 | Default |  |  | QPSK | 95@0 (A3) | 48@0 (A3) | 24@0 (A3) |
| 47 | Default | 40 | Default |  |  | QPSK | 152@0 (A4) | 76@0 (A4) | 38@0 (A4) |
| 48 | Default | 40 | Default |  |  | QPSK | 192@0 (A2) | 96@0 (A2) | 48@0 (A2) |
| 49 | Default | 40 | Default |  |  | QPSK | 5@187 (A3) | 2@94 (A3) | 1@47 (A3) |
| 50 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 51 | Default | 45 | Default |  |  | QPSK | 27@0 (A2) | 13@0 (A2) | 6@0 (A2) |
| 52 | Default | 45 | Default |  |  | QPSK | 106@0 (A4) | 53@0 (A4) | 27@0 (A4) |
| 53 | Default | 45 | Default |  |  | QPSK | 208@0 (A2) | 104@0 (A2) | 52@0 (A2) |
| 54 | Default | 45 | Default |  |  | QPSK | 5@203 (A5) | 2@102 (A5) | 1@51 (A5) |
| 55 | Default | 45 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 56 | Default | 50 | Default |  |  | QPSK | 34@0 (A2) | 17@0 (A2) | 9@0 (A2) |
| 57 | Default | 50 | Default |  |  | QPSK | 115@0 (A4) | 58@0 (A4) | 29@0 (A4) |
| 58 | Default | 50 | Default |  |  | QPSK | 228@0 (A2) | 114@0 (A2) | 57@0 (A2) |
| 59 | Default | 50 | Default |  |  | QPSK | 5@223 (A5) | 2@112 (A5) | 1@56 (A5) |
| 60 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 61 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 62 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |
| 63 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 64 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A5) |  |  |
| 65 | Default | 40 | Default |  |  | 256 QAM | 16@0 (A2) | 8@0 (A2) | 4@0 (A2) |
| 66 | Default | 40 | Default |  |  | 256 QAM | 95@0 (A3) | 48@0 (A3) | 24@0 (A3) |
| 67 | Default | 40 | Default |  |  | 256 QAM | 152@0 (A4) | 76@0 (A4) | 38@0 (A4) |
| 68 | Default | 40 | Default |  |  | 256 QAM | 192@0 (A2) | 96@0 (A2) | 48@0 (A2) |
| 69 | Default | 40 | Default |  |  | 256 QAM | 5@187 (A3) | 2@94 (A3) | 1@47 (A3) |
| 70 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 71 | Default | 45 | Default |  |  | 256 QAM | 27@0 (A2) | 13@0 (A2) | 6@0 (A2) |
| 72 | Default | 45 | Default |  |  | 256 QAM | 106@0 (A4) | 53@0 (A4) | 27@0 (A4) |
| 73 | Default | 45 | Default |  |  | 256 QAM | 208@0 (A2) | 104@0 (A2) | 52@0 (A2) |
| 74 | Default | 45 | Default |  |  | 256 QAM | 5@203 (A5) | 2@102 (A5) | 1@51 (A5) |
| 75 | Default | 45 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 76 | Default | 50 | Default |  |  | 256 QAM | 34@0 (A2) | 17@0 (A2) | 9@0 (A2) |
| 77 | Default | 50 | Default |  |  | 256 QAM | 115@0 (A4) | 58@0 (A4) | 29@0 (A4) |
| 78 | Default | 50 | Default |  |  | 256 QAM | 228@0 (A2) | 114@0 (A2) | 57@0 (A2) |
| 79 | Default | 50 | Default |  |  | 256 QAM | 5@223 (A5) | 2@112 (A5) | 1@56 (A5) |
| 80 | Default | 50 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-20: Test Configuration table for NS_48 (Power class 2)

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies |  |  |  |  |  |  | lowest and highest carrier centre frequency for each CBW in table 6.2.3.3.26-1 |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 25 MHz, 30MHz, 40MHz, 45MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_48 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 10 | Default | N/A for A-MPR testing. | DFT-s-OFDM | QPSK | Outer_Full (A6) |  | - |
| 2 | Default | 10 | Default |  |  | QPSK | 8@0 (A6) | 4@0 (A6) | 2@0 (A6) |
| 3 | Default | 15 | Default |  |  | QPSK | Outer_Full (A6) |  |  |
| 4 | Default | 15 | Default |  |  | QPSK | 48@0 (A6) | 24@0 (A6) | 12@0 (A6) |
| 5 | Default | 20 | Default |  |  | QPSK | Outer_Full (A4) |  |  |
| 6 | Default | 20 | Default |  |  | QPSK | 50@0 (A6) | 25@0 (A6) | 12@0 (A6) |
| 7 | Default | 25 | Default |  |  | QPSK | Outer_Full (A4) |  |  |
| 8 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |
| 9 | Default | 25 | Default |  |  | QPSK | 50@0 (A4) | 25@0 (A4) | 12@0 (A4) |
| 10 | Default | 30 | Default |  |  | QPSK | Outer_Full (A4) |  |  |
| 11 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |
| 12 | Default | 30 | Default |  |  | QPSK | 64@0 (A4) | 32@0 (A4) | 16@0 (A4) |
| 13 | Default | 40 | Default |  |  | QPSK | 16@0 (A1) | 8@0 (A1) | 4@0 (A1) |
| 14 | Default | 40 | Default |  |  | QPSK | 90@0 (A3) | 48@0 (A3) | 24@0 (A3) |
| 15 | Default | 40 | Default |  |  | QPSK | 150@0 (A4) | 75@0 (A4) | 36@0 (A4) |
| 16 | Default | 40 | Default |  |  | QPSK | 192@0 (A2) | 96@0 (A2) | 48@0 (A2) |
| 17 | Default | 40 | Default |  |  | QPSK | 5@187 (A3) | 2@94 (A3) | 1@47 (A3) |
| 18 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 19 | Default | 45 | Default |  |  | QPSK | 27@0 (A1) | 12@0 (A1) | 6@0 (A1) |
| 20 | Default | 45 | Default |  |  | QPSK | 100@0 (A4) | 50@0 (A4) | 27@0 (A4) |
| 21 | Default | 45 | Default |  |  | QPSK | 200@0 (A2) | 100@0 (A2) | 50@0 (A2) |
| 22 | Default | 45 | Default |  |  | QPSK | 5@203 (A5) | 2@102 (A5) | 1@51 (A5) |
| 23 | Default | 45 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 24 | Default | 50 | Default |  |  | QPSK | 36@0 (A1) | 18@0 (A1) | 9@0 (A1) |
| 25 | Default | 50 | Default |  |  | QPSK | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 26 | Default | 50 | Default |  |  | QPSK | 225@0 (A2) | 108@0 (A2) | 54@0 (A2) |
| 27 | Default | 50 | Default |  |  | QPSK | 5@223 (A5) | 2@112 (A5) | 1@56 (A5) |
| 28 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 29 | Default | 10 | Default |  |  | 256QAM | Outer_Full (A6) |  | - |
| 30 | Default | 10 | Default |  |  | 256QAM | 8@0 (A6) | 4@0 (A6) | 2@0 (A6) |
| 31 | Default | 15 | Default |  |  | 256QAM | Outer_Full (A6) |  |  |
| 32 | Default | 15 | Default |  |  | 256QAM | 48@0 (A6) | 24@0 (A6) | 12@0 (A6) |
| 33 | Default | 20 | Default |  |  | 256QAM | Outer_Full (A4) |  |  |
| 34 | Default | 20 | Default |  |  | 256QAM | 50@0 (A6) | 25@0 (A6) | 12@0 (A6) |
| 35 | Default | 25 | Default |  |  | 256QAM | Outer_Full (A4) |  |  |
| 36 | Default | 25 | Default |  |  | 256QAM | Edge_1RB_Right (A3) |  |  |
| 37 | Default | 25 | Default |  |  | 256QAM | 50@0 (A4) | 25@0 (A4) | 12@0 (A4) |
| 38 | Default | 30 | Default |  |  | 256QAM | Outer_Full (A4) |  |  |
| 39 | Default | 30 | Default |  |  | 256QAM | Edge_1RB_Right (A5) |  |  |
| 40 | Default | 30 | Default |  |  | 256QAM | 64@0 (A4) | 32@0 (A4) | 16@0 (A4) |
| 41 | Default | 40 | Default |  |  | 256QAM | 16@0 (A1) | 8@0 (A1) | 4@0 (A1) |
| 42 | Default | 40 | Default |  |  | 256QAM | 90@0 (A3) | 48@0 (A3) | 24@0 (A3) |
| 43 | Default | 40 | Default |  |  | 256QAM | 150@0 (A4) | 75@0 (A4) | 36@0 (A4) |
| 44 | Default | 40 | Default |  |  | 256QAM | 192@0 (A2) | 96@0 (A2) | 48@0 (A2) |
| 45 | Default | 40 | Default |  |  | 256QAM | 5@187 (A3) | 2@94 (A3) | 1@47 (A3) |
| 46 | Default | 40 | Default |  |  | 256QAM | Outer_Full (A1) |  |  |
| 47 | Default | 45 | Default |  |  | 256QAM | 27@0 (A1) | 12@0 (A1) | 6@0 (A1) |
| 48 | Default | 45 | Default |  |  | 256QAM | 100@0 (A4) | 50@0 (A4) | 27@0 (A4) |
| 49 | Default | 45 | Default |  |  | 256QAM | 200@0 (A2) | 100@0 (A2) | 50@0 (A2) |
| 50 | Default | 45 | Default |  |  | 256QAM | 5@203 (A5) | 2@102 (A5) | 1@51 (A5) |
| 51 | Default | 45 | Default |  |  | 256QAM | Outer_Full (A1) |  |  |
| 52 | Default | 50 | Default |  |  | 256QAM | 36@0 (A1) | 18@0 (A1) | 9@0 (A1) |
| 53 | Default | 50 | Default |  |  | 256QAM | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 54 | Default | 50 | Default |  |  | 256QAM | 225@0 (A2) | 108@0 (A2) | 54@0 (A2) |
| 55 | Default | 50 | Default |  |  | 256QAM | 5@223 (A5) | 2@112 (A5) | 1@56 (A5) |
| 56 | Default | 50 | Default |  |  | 256QAM | Outer_Full (A1) |  |  |
| 57 | Default | 10 | Default |  | CP-OFDM | QPSK | Outer_Full (A6) |  | - |
| 58 | Default | 10 | Default |  |  | QPSK | 8@0 (A6) | 4@0 (A6) | 2@0 (A6) |
| 59 | Default | 15 | Default |  |  | QPSK | Outer_Full (A6) |  |  |
| 60 | Default | 15 | Default |  |  | QPSK | 48@0 (A6) | 24@0 (A6) | 12@0 (A6) |
| 61 | Default | 20 | Default |  |  | QPSK | Outer_Full (A4) |  |  |
| 62 | Default | 20 | Default |  |  | QPSK | 52@0 (A6) | 26@0 (A6) | 13@0 (A6) |
| 63 | Default | 25 | Default |  |  | QPSK | Outer_Full (A4) |  |  |
| 64 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |
| 65 | Default | 25 | Default |  |  | QPSK | 52@0 (A4) | 26@0 (A4) | 13@0 (A4) |
| 66 | Default | 30 | Default |  |  | QPSK | Outer_Full (A4) |  |  |
| 67 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |
| 68 | Default | 30 | Default |  |  | QPSK | 70@0 (A4) | 35@0 (A4) | 17@0 (A4) |
| 69 | Default | 40 | Default |  |  | QPSK | 16@0 (A1) | 8@0 (A1) | 4@0 (A1) |
| 70 | Default | 40 | Default |  |  | QPSK | 90@0 (A3) | 48@0 (A3) | 24@0 (A3) |
| 71 | Default | 40 | Default |  |  | QPSK | 150@0 (A4) | e@0 (A4) | 38@0 (A4) |
| 72 | Default | 40 | Default |  |  | QPSK | 192@0 (A2) | 96@0 (A2) | 48@0 (A2) |
| 73 | Default | 40 | Default |  |  | QPSK | 5@187 (A3) | 2@94 (A3) | 1@47 (A3) |
| 74 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 75 | Default | 45 | Default |  |  | QPSK | 28@0 (A1) | 14@0 (A1) | 7@0 (A1) |
| 76 | Default | 45 | Default |  |  | QPSK | 100@0 (A4) | 50@0 (A4) | 25@0 (A4) |
| 77 | Default | 45 | Default |  |  | QPSK | 200@0 (A2) | 100@0 (A2) | 50@0 (A2) |
| 78 | Default | 45 | Default |  |  | QPSK | 5@203 (A5) | 2@102 (A5) | 1@51 (A5) |
| 79 | Default | 45 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 80 | Default | 50 | Default |  |  | QPSK | 36@0 (A1) | 18@0 (A1) | 9@0 (A1) |
| 81 | Default | 50 | Default |  |  | QPSK | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 82 | Default | 50 | Default |  |  | QPSK | 225@0 (A2) | 108@0 (A2) | 54@0 (A2) |
| 83 | Default | 50 | Default |  |  | QPSK | 5@223 (A5) | 2@112 (A5) | 1@56 (A5) |
| 84 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 85 | Default | 10 | Default |  |  | 256QAM | Outer_Full (A6) |  | - |
| 86 | Default | 10 | Default |  |  | 256QAM | 8@0 (A6) | 4@0 (A6) | 2@0 (A6) |
| 87 | Default | 15 | Default |  |  | 256QAM | Outer_Full (A6) |  |  |
| 88 | Default | 15 | Default |  |  | 256QAM | 48@0 (A6) | 24@0 (A6) | 12@0 (A6) |
| 89 | Default | 20 | Default |  |  | 256QAM | Outer_Full (A4) |  |  |
| 90 | Default | 20 | Default |  |  | 256QAM | 52@0 (A6) | 26@0 (A6) | 13@0 (A6) |
| 91 | Default | 25 | Default |  |  | 256QAM | Outer_Full (A4) |  |  |
| 92 | Default | 25 | Default |  |  | 256QAM | Edge_1RB_Right (A3) |  |  |
| 93 | Default | 25 | Default |  |  | 256QAM | 52@0 (A4) | 26@0 (A4) | 13@0 (A4) |
| 94 | Default | 30 | Default |  |  | 256QAM | Outer_Full (A4) |  |  |
| 95 | Default | 30 | Default |  |  | 256QAM | Edge_1RB_Right (A5) |  |  |
| 96 | Default | 30 | Default |  |  | 256QAM | 70@0 (A4) | 35@0 (A4) | 17@0 (A4) |
| 97 | Default | 40 | Default |  |  | 256QAM | 16@0 (A1) | 8@0 (A1) | 4@0 (A1) |
| 98 | Default | 40 | Default |  |  | 256QAM | 90@0 (A3) | 48@0 (A3) | 24@0 (A3) |
| 99 | Default | 40 | Default |  |  | 256QAM | 150@0 (A4) | 76@0 (A4) | 38@0 (A4) |
| 100 | Default | 40 | Default |  |  | 256QAM | 192@0 (A2) | 96@0 (A2) | 48@0 (A2) |
| 101 | Default | 40 | Default |  |  | 256QAM | 5@187 (A3) | 2@94 (A3) | 1@47 (A3) |
| 102 | Default | 40 | Default |  |  | 256QAM | Outer_Full (A1) |  |  |
| 103 | Default | 45 | Default |  |  | 256QAM | 28@0 (A1) | 14@0 (A1) | 7@0 (A1) |
| 104 | Default | 45 | Default |  |  | 256QAM | 100@0 (A4) | 50@0 (A4) | 25@0 (A4) |
| 105 | Default | 45 | Default |  |  | 256QAM | 200@0 (A2) | 100@0 (A2) | 50@0 (A2) |
| 106 | Default | 45 | Default |  |  | 256QAM | 5@203 (A5) | 2@102 (A5) | 1@51 (A5) |
| 107 | Default | 45 | Default |  |  | 256QAM | Outer_Full (A1) |  |  |
| 108 | Default | 50 | Default |  |  | 256QAM | 36@0 (A1) | 18@0 (A1) | 9@0 (A1) |
| 109 | Default | 50 | Default |  |  | 256QAM | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 110 | Default | 50 | Default |  |  | 256QAM | 225@0 (A2) | 108@0 (A2) | 54@0 (A2) |
| 111 | Default | 50 | Default |  |  | 256QAM | 5@223 (A5) | 2@112 (A5) | 1@56 (A5) |
| 112 | Default | 50 | Default |  |  | 256QAM | Outer_Full (A1) |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-21: Test Configuration table for NS_12

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Low range, High range |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Refer to test parameters (3, 5, 10 MHz) |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest |  |  |  |  |
| A-MPR test parameters for NS_12 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (NOTE 2, 3) |  | RB allocation (Note 1) |  |  |
| 1 | Default | 5 | Default | N/A for A-MPR testing. | DFT-s-OFDM | QPSK | Edge_1RB_Left |  |  |
| 2 | Default | 5 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 3 | Default | 5 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 4 | Default | 5 | Default |  |  | QPSK | Outer_Full |  |  |
| 5 | Default | 5 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 6 | Default | 5 | Default |  |  | 256 QAM | Outer_Full |  |  |
| 7 | Default | 10 | Default |  |  | QPSK | Edge_1RB_Left |  |  |
| 8 | Default | 10 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 9 | Default | 10 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 10 | Default | 10 | Default |  |  | QPSK | Outer_Full |  |  |
| 11 | Default | 10 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 12 | Default | 10 | Default |  |  | 256 QAM | Outer_Full |  |  |
| 13 | Default | 5 | Default |  |  | QPSK | 12@11 |  |  |
| 14 | Default | 10 | Default |  |  | QPSK | 30@21 |  |  |
| 15 | Default | 5 | Default |  | CP-OFDM | 64 QAM | Edge_1RB_Left |  |  |
| 16 | Default | 5 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 17 | Default | 5 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 18 | Default | 5 | Default |  |  | 256 QAM | Outer_Full |  |  |
| 19 | Default | 10 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 20 | Default | 10 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 21 | Default | 10 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 22 | Default | 10 | Default |  |  | 256 QAM | Outer_Full |  |  |
| 23 | Default | 5 | Default |  |  | QPSK | 12@11 |  |  |
| 24 | Default | 10 | Default |  |  | QPSK | 30@21 |  |  |
| 25 | Default | 3 | Default |  | DFT-s-OFDM | QPSK | Edge_1RB_Left |  |  |
| 26 | Default | 3 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 27 | Default | 3 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 28 | Default | 3 | Default |  |  | QPSK | Outer_Full |  |  |
| 29 | Default | 3 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 30 | Default | 3 | Default |  |  | 256 QAM | Outer_Full |  |  |
| 31 | Default | 3 | Default |  |  | QPSK | 8@6 |  |  |
| 32 | Default | 3 | Default |  | CP-OFDM | 64 QAM | Edge_1RB_Left |  |  |
| 33 | Default | 3 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 34 | Default | 3 | Default |  |  | QPSK | 8@6 |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: In test IDs with multiple modulations, each UL Modulation shall be tested separately against RB allocation. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-22: Test Configuration table for NS_13

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Refer to uplink carrier center frequency (Fc) in test parameters |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Refer to test parameters (3, 5 MHz) |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest |  |  |  |  |
| A-MPR test parameters for NS_13 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (NOTE 2, 3) |  | RB allocation (Note 1) |  |  |
| 1 | 819.5 | 5 | Default | N/A for A-MPR testing. | DFT-s-OFDM | 16 QAM | Edge_1RB_Left |  |  |
| 2 | 819.5 | 5 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 3 | 819.5 | 5 | Default |  |  | 256QAM | Edge_1RB_Left |  |  |
| 4 | 819.5 | 5 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 5 | 819.5 | 5 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 6 | 819.5 | 5 | Default |  |  | 256QAM | Outer_Full |  |  |
| 7 | 821.5 | 5 | Default |  |  | 16 QAM | Edge_1RB_Left |  |  |
| 8 | 821.5 | 5 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 9 | 821.5 | 5 | Default |  |  | 256QAM | Edge_1RB_Left |  |  |
| 10 | High | 5 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 11 | Void |  |  |  |  |  |  |  |  |
| 12 | 819.5 | 5 | Default |  | CP-OFDM | 16 QAM | Edge_1RB_Left |  |  |
| 13 | 819.5 | 5 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 14 | 819.5 | 5 | Default |  |  | 256QAM | Edge_1RB_Left |  |  |
| 15 | 819.5 | 5 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 16 | 819.5 | 5 | Default |  |  | 256QAM | Outer_Full |  |  |
| 17 | 821.5 | 5 | Default |  |  | 16 QAM | Edge_1RB_Left |  |  |
| 18 | 821.5 | 5 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 19 | 821.5 | 5 | Default |  |  | 256QAM | Edge_1RB_Left |  |  |
| 20 | High | 5 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 21 | Void |  |  |  |  |  |  |  |  |
| 22 | 818.5 | 3 | Default |  | DFT-s-OFDM | 16 QAM | Edge_1RB_Left |  |  |
| 23 | 818.5 | 3 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 24 | 818.5 | 3 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 25 | 818.5 | 3 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 26 | 820 | 3 | Default |  |  | 16 QAM | Edge_1RB_Left |  |  |
| 27 | 820 | 3 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 28 | 818.5 | 3 | Default |  | CP-OFDM | 16 QAM | Edge_1RB_Left |  |  |
| 29 | 818.5 | 3 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 30 | 818.5 | 3 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 31 | 818.5 | 3 | Default |  |  | 16 QAM | Edge_1RB_Left |  |  |
| 32 | 820 | 3 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: In test IDs with multiple modulations, each UL Modulation shall be tested separately against RB allocation. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-23: Test Configuration table for NS_14

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Low range, High range |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Refer to test parameters (10, 15, 20 MHz) |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest |  |  |  |  |
| A-MPR test parameters for NS_14 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (NOTE 2, 3) |  | RB allocation (Note 1) |  |  |
| 1 | Default | 10 | Default | N/A for A-MPR testing. | DFT-s-OFDM | 64 QAM | Edge_1RB_Left |  |  |
| 2 | Default | 10 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 3 | Default | 15 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 4 | Default | 15 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 5 | Default | 20 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 6 | Default | 20 | Default |  |  | 256QAM | Edge_1RB_Left |  |  |
| 7 | Default | 20 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 8 | Default | 10 | Default |  |  | QPSK | 8@2 |  |  |
| 9 | Default | 15 | Default |  |  | QPSK | 8@11 |  |  |
| 10 | Default | 20 | Default |  |  | QPSK | 8@20 |  |  |
| 11 | Default | 10 | Default |  | CP-OFDM | 16 QAM | Edge_1RB_Left |  |  |
| 12 | Default | 10 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 13 | Default | 10 | Default |  |  | 256QAM | Edge_1RB_Left |  |  |
| 14 | Default | 10 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 15 | Default | 15 | Default |  |  | 16 QAM | Edge_1RB_Left |  |  |
| 16 | Default | 15 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 17 | Default | 15 | Default |  |  | 256QAM | Edge_1RB_Left |  |  |
| 18 | Default | 15 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 19 | Default | 20 | Default |  |  | 16 QAM | Edge_1RB_Left |  |  |
| 20 | Default | 20 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 21 | Default | 20 | Default |  |  | 256QAM | Edge_1RB_Left |  |  |
| 22 | Default | 20 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 23 | Default | 10 | Default |  |  | QPSK | 8@2 |  |  |
| 24 | Default | 15 | Default |  |  | QPSK | 8@11 |  |  |
| 25 | Default | 20 | Default |  |  | QPSK | 8@20 |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: In test IDs with multiple modulations, each UL Modulation shall be tested separately against RB allocation. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-24: Test Configuration table for NS_15

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Refer to uplink carrier center frequency (Fc) in test parameters |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Refer to test parameters (3, 5, 10, 15, 20 MHz) |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest |  |  |  |  |
| A-MPR test parameters for NS_15 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (NOTE 2, 3) |  | RB allocation (Note 1) |  |  |
| 1 | 846.5 | 5 | Default | N/A for A-MPR testing. | DFT-s-OFDM | 64 QAM | Edge_1RB_Right |  |  |
| 2 | 846.5 | 5 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 3 | 846.5 | 5 | Default |  |  | 64 QAM | 8@9 |  |  |
| 4 | 846.5 | 5 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 5 | 846.5 | 5 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 6 | 844 | 10 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 7 | 844 | 10 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 8 | 844 | 10 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 9 | 844 | 10 | Default |  |  | 64 QAM | 15@16 |  |  |
| 10 | 844 | 10 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 11 | 844 | 10 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 12 | 840 | 10 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 13 | 840 | 10 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 14 | 840 | 10 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 15 | 840 | 10 | Default |  |  | 64 QAM | 24@15 |  |  |
| 16 | 840 | 10 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 17 | 840 | 10 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 18 | 841.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 19 | 841.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 20 | 841.5 | 15 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 21 | 841.5 | 15 | Default |  |  | 64 QAM | 20@31 |  |  |
| 22 | 841.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 23 | 841.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 24 | 837.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 25 | 837.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 26 | 837.5 | 15 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 27 | 837.5 | 15 | Default |  |  | 64 QAM | 20@39 |  |  |
| 28 | 837.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 29 | 837.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 30 | 831.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 31 | 831.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 32 | 831.5 | 15 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 33 | 831.5 | 15 | Default |  |  | 64 QAM | 20@52 |  |  |
| 34 | 831.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 35 | 831.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 36 | 839 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 37 | 839 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 38 | 839 | 20 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 39 | 839 | 20 | Default |  |  | 64 QAM | 30@37 |  |  |
| 40 | 839 | 20 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 41 | 839 | 20 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 42 | 835 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 43 | 835 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 44 | 835 | 20 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 45 | 835 | 20 | Default |  |  | 64 QAM | 30@45 |  |  |
| 46 | 835 | 20 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 47 | 835 | 20 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 48 | 840.5 | 5 | Default |  |  | QPSK | Outer_Full |  |  |
| 49 | 835 | 10 | Default |  |  | QPSK | Outer_Full |  |  |
| 50 | 831.5 | 15 | Default |  |  | QPSK | 20@23 |  |  |
| 51 | 835 | 20 | Default |  |  | QPSK | 30@16 |  |  |

| A-MPR test parameters for NS_15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (NOTE 2, 3) |  | RB allocation (Note 1) |  |  |
| 52 | 846.5 | 5 | Default |  | CP-OFDM | 64 QAM | Edge_1RB_Right |  |  |
| 53 | 846.5 | 5 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 54 | 846.5 | 5 | Default |  |  | 64 QAM | 8@9 |  |  |
| 55 | 846.5 | 5 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 56 | 846.5 | 5 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 57 | 844 | 10 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 58 | 844 | 10 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 59 | 844 | 10 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 60 | 844 | 10 | Default |  |  | 64 QAM | 15@16 |  |  |
| 61 | 844 | 10 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 62 | 844 | 10 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 63 | 840 | 10 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 64 | 840 | 10 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 65 | 840 | 10 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 66 | 840 | 10 | Default |  |  | 64 QAM | 24@15 |  |  |
| 67 | 840 | 10 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 68 | 840 | 10 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 69 | 841.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 70 | 841.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 71 | 841.5 | 15 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 72 | 841.5 | 15 | Default |  |  | 64 QAM | 20@31 |  |  |
| 73 | 841.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 74 | 841.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 75 | 837.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 76 | 837.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 77 | 837.5 | 15 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 78 | 837.5 | 15 | Default |  |  | 64 QAM | 20@39 |  |  |
| 79 | 837.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 80 | 837.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 81 | 831.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 82 | 831.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 83 | 831.5 | 15 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 84 | 831.5 | 15 | Default |  |  | 64 QAM | 20@52 |  |  |
| 85 | 831.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 86 | 831.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 87 | 839 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 88 | 839 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 89 | 839 | 20 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 90 | 839 | 20 | Default |  |  | 64 QAM | 30@37 |  |  |
| 91 | 839 | 20 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 92 | 839 | 20 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 93 | 835 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 94 | 835 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 95 | 835 | 20 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 96 | 835 | 20 | Default |  |  | 64 QAM | 30@45 |  |  |
| 97 | 835 | 20 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 98 | 835 | 20 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 99 | 840.5 | 5 | Default |  |  | QPSK | Outer_Full |  |  |
| 100 | 835 | 10 | Default |  |  | QPSK | Outer_Full |  |  |
| 101 | 831.5 | 15 | Default |  |  | QPSK | 20@23 |  |  |
| 102 | 835 | 20 | Default |  |  | QPSK | 30@16 |  |  |
| 103 | 847.5 | 3 | Default |  | DFT-s-OFDM | 64 QAM | Edge_1RB_Right |  |  |
| 104 | 847.5 | 3 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 105 | 847.5 | 3 | Default |  |  | 16 QAM | Edge_1RB_Left |  |  |
| 106 | 844.5 | 3 | Default |  |  | QPSK | Outer_Full |  |  |
| 107 | 847.5 | 3 | Default |  | CP-OFDM | 64 QAM | Edge_1RB_Right |  |  |
| 108 | 847.5 | 3 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 109 | 847.5 | 3 | Default |  |  | 16 QAM | Edge_1RB_Left |  |  |
| 110 | 844.5 | 3 | Default |  |  | QPSK | Outer_Full |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: In test IDs with multiple modulations, each UL Modulation shall be tested separately against RB allocation. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-25: Test Configuration table for NS_46 (Power class 3)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 25 MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_46 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 |  |  |  | N/A for A-MPR testing. | DFT-s-OFDM |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |
| 3 | Default | 25 | Default |  |  | 64QAM | 60@73 (A3) |  | 30@35 (A3) | 15@16 (A3) |
| 4 | Default | 50 | Default |  |  | 64QAM | Edge_1RB_Left (A4) |  |  |  |
| 5 | Default | 50 | Default |  |  | 16QAM | 120@0 (A5) |  | 60@0 (A5) | 30@0 (A5) |
| 6 | Default | 50 | Default |  |  | 64QAM | 162@0 (A6) |  | 81@0 (A6) | 40@0 (A6) |
| 7 | Default | 50 | Default |  |  | 256QAM | 216@0 (A7) |  | 108@0 (A7) | 54@0 (A7) |
| 8 | Default | 50 | Default |  |  | 256QAM | Outer_Full (A8) |  |  |  |
| 9 |  |  |  |  | CP-OFDM |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |
| 11 | Default | 25 | Default |  |  | QPSK | 90@43 (A3) |  | 45@20 (A3) | 23@8 (A3) |
| 12 | Default | 25 | Default |  |  | 256QAM | 90@43 (A3) |  | 45@20 (A3) | 23@8 (A3) |
| 13 | Default | 50 | Default |  |  | QPSK | Edge_1RB_Left (A4) |  |  |  |
| 14 | Default | 50 | Default |  |  | 64QAM | Edge_1RB_Left (A4) |  |  |  |
| 15 | Default | 50 | Default |  |  | QPSK | 120@0 (A5) |  | 60@0 (A5) | 30@0 (A5) |
| 16 | Default | 50 | Default |  |  | 64QAM | 120@0 (A5) |  | 60@0 (A5) | 30@0 (A5) |
| 17 | Default | 50 | Default |  |  | QPSK | 175@0 (A6) |  | 88@0 (A6) | 44@0 (A6) |
| 18 | Default | 50 | Default |  |  | 64QAM | 175@0 (A6) |  | 88@0 (A6) | 44@0 (A6) |
| 19 | Default | 50 | Default |  |  | QPSK | 220@0 (A7) |  | 110@0 (A7) | 55@0 (A7) |
| 20 | Default | 50 | Default |  |  | 256QAM | 220@0 (A7) |  | 110@0 (A7) | 55@0 (A7) |
| 21 | Default | 50 | Default |  |  | QPSK | Outer_Full (A8) |  |  |  |
| 22 | Default | 50 | Default |  |  | 256QAM | Outer_Full (A8) |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-25a: Test Configuration table for NS_46 (Power class 1)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 15 MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_46 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 15 | Default | N/A for A-MPR testing. | DFT-s-OFDM | 64QAM | Outer_Full (A1) |  |  |  |
| 2 | Default | 15 | Default |  |  | 256QAM | Edge_1RB_Right (A2) |  |  |  |
| 3 | Default | 50 | Default |  |  | 256QAM | 68@35 (A7) |  | 31@19 (A7) | 17@7(A7) |
| 4 | Default | 50 | Default |  |  | N/A | N/A (A11) |  |  |  |
| 5 | Default | 50 | Default |  |  | 256QAM | 146@20 (A9) |  | 74@10 (A9) | 38@5 (A9) |
| 6 | Default | 50 | Default |  |  | 64QAM | 10@260 (A12) |  | 10@123 (A12) | 10@55 (A12) |
| 7 | Default | 50 | Default |  |  | 256QAM | 28@160 (A8) |  | 14@80 (A8) | 7@40 (A8) |
| 8 | Default | 50 | Default |  |  | 256QAM | 12@40 (A9) |  | 6@20 (A9) | 3@10 (A9) |
| 9 | Default | 50 | Default |  |  | 256QAM | 57@20 (A7) |  | 28@10 (A7) | 14@5 (A7) |
| 10 | Default | 50 | Default |  |  | N/A | N/A (A11) |  |  |  |
| 11 | Default | 50 | Default |  |  | 256QAM | 195@0 (A8) |  | 97@0 (A8) | 48@0 (A8) |
| 12 | Default | 50 | Default |  |  | 64QAM | 122@117 (A12) |  | 61@59 (A12) | 30@30 (A12) |
| 13 | Default | 50 | Default |  |  | 256QAM | 16@179 (A8) |  | 8@90 (A8) | 4@45 (A8) |
| 14 | Default | 50 | Default |  |  | 256QAM | 11@195 (A9) |  | 5@98 (A9) | 2@49 (A9) |
| 15 | Default | 50 | Default |  |  | 64QAM | 229@34 (A13) |  | 115@16 (A13) | 57@9 (A13) |
| 16 | Default | 50 | Default |  |  | 256QAM | 46@0 (A7) |  | 23@0 (A7) | 11@0 (A7) |
| 17 | Default | 50 | Default |  |  | 256QAM | 17@195 (A8) |  | 9@97 (A8) | 5@48 (A8) |
| 18 | Default | 50 | Default |  |  | 256QAM | 11@201 (A9) |  | 5@101 (A9) | 2@51 (A9) |
| 19 | Default | 15 | Default |  | CP-OFDM | 64QAM | Outer_Full (A1) |  |  |  |
| 20 | Default | 15 | Default |  |  | 256QAM | Edge_1RB_Right (A2) |  |  |  |
| 21 | Default | 50 | Default |  |  | 256QAM | 68@35 (A7) |  | 31@19 (A7) | 17@7(A7) |
| 22 | Default | 50 | Default |  |  | 256QAM | 68@90(A11) |  | 31@45(A11) | 17@18(A11) |
| 23 | Default | 50 | Default |  |  | 256QAM | 146@20 (A9) |  | 74@10 (A9) | 38@5 (A9) |
| 24 | Default | 50 | Default |  |  | 64QAM | 10@260 (A12) |  | 10@123 (A12) | 10@55 (A12) |
| 25 | Default | 50 | Default |  |  | 256QAM | 28@160 (A8) |  | 14@80 (A8) | 7@40 (A8) |
| 26 | Default | 50 | Default |  |  | 256QAM | 12@40 (A9) |  | 6@20 (A9) | 3@10 (A9) |
| 27 | Default | 50 | Default |  |  | 256QAM | 57@20 (A7) |  | 28@10 (A7) | 14@5 (A7) |
| 28 | Default | 50 | Default |  |  | 256QAM | 57@87 (A11) |  | 28@54 (A7) | 14@22 (A7) |
| 29 | Default | 50 | Default |  |  | 256QAM | 195@0 (A8) |  | 97@0 (A8) | 48@0 (A8) |
| 30 | Default | 50 | Default |  |  | 64QAM | 122@117 (A12) |  | 61@59 (A12) | 30@30 (A12) |
| 31 | Default | 50 | Default |  |  | 256QAM | 16@179 (A8) |  | 8@90 (A8) | 4@45 (A8) |
| 32 | Default | 50 | Default |  |  | 256QAM | 11@195 (A9) |  | 5@98 (A9) | 2@49 (A9) |
| 33 | Default | 50 | Default |  |  | 64QAM | 229@34 (A13) |  | 115@16 (A13) | 57@9 (A13) |
| 34 | Default | 50 | Default |  |  | 256QAM | 46@0 (A7) |  | 23@0 (A7) | 11@0 (A7) |
| 35 | Default | 50 | Default |  |  | 256QAM | 17@195 (A8) |  | 9@97 (A8) | 5@48 (A8) |
| 36 | Default | 50 | Default |  |  | 256QAM | 11@201 (A9) |  | 5@101 (A9) | 2@51 (A9) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-25b: Test Configuration table for NS_46 (Power class 2)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 15 MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_46 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 10 | Default | N/A for A-MPR testing. | DFT-s-OFDM | 64QAM | Outer_Full (A6) |  |  |  |
| 2 | Default | 10 | Default |  |  | 64QAM | Edge_1RB_Right (A5) |  |  |  |
| 3 | Default | 50 | Default |  |  | 256QAM | 1@48 (A4) |  | 1@23 (A4) | 1@11 (A4) |
| 4 | Default | 50 | Default |  |  | 64QAM | 80@37 (A5) |  | 39@20 (A5) | 19@10 (A5) |
| 5 | Default | 50 | Default |  |  | 64QAM | 101@71 (A6) |  | 51@36 (A6) | 26@17 (A6) |
| 6 | Default | 50 | Default |  |  | 256QAM | 91@127 (A7) |  | 46@63 (A7) | 23@30 (A7) |
| 7 | Default | 50 | Default |  |  | 256QAM | 1@221 (A8) |  | 1@111 (A8) | 1@56 (A8) |
| 8 | Default | 50 | Default |  |  | 64QAM | 88@112 (A10) |  | 40@60 (A10) | 20@30 (A10) |
| 9 | Default | 10 | Default |  | CP-OFDM | 64QAM | Outer_Full (A6) |  |  |  |
| 10 | Default | 10 | Default |  |  | 64QAM | Edge_1RB_Right (A5) |  |  |  |
| 11 | Default | 50 | Default |  |  | 256QAM | 1@48 (A4) |  | 1@23 (A4) | 1@11 (A4) |
| 12 | Default | 50 | Default |  |  | 64QAM | 80@37 (A5) |  | 39@20 (A5) | 19@10 (A5) |
| 13 | Default | 50 | Default |  |  | 64QAM | 101@71 (A6) |  | 51@36 (A6) | 26@17 (A6) |
| 14 | Default | 50 | Default |  |  | 256QAM | 91@127 (A7) |  | 46@63 (A7) | 23@30 (A7) |
| 15 | Default | 50 | Default |  |  | 256QAM | 1@221 (A8) |  | 1@111 (A8) | 1@56 (A8) |
| 16 | Default | 50 | Default |  |  | 64QAM | 88@112 (A10) |  | 40@60 (A10) | 20@30 (A10) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-26: Test Configuration table for NS_44

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Defined for each test ID |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Defined for each test ID |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_44 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | 2602.5 | 25 | Default | N/A | DFT-s-OFDM | 64 QAM | 100@0 (A3) |  | 50@0 (A3) | 25@0 (A3) |
| 2 | 2602.5 | 25 | Default |  |  | 64 QAM | 36@97 (A3) |  | 18@47 (A3) | 9@22 (A3) |
| 3 | 2602.5 | 25 | Default |  |  | 64 QAM | Outer_Full (A6) |  |  |  |
| 4 | 2602.5 | 25 | Default |  |  | 256 QAM | 100@0 (A3) |  | 50@0 (A3) | 25@0 (A3) |
| 5 | 2602.5 | 25 | Default |  |  | 256 QAM | 36@97 (A3) |  | 18@47 (A3) | 9@22 (A3) |
| 6 | 2602.5 | 25 | Default |  |  | 256 QAM | Outer_Full (A6) |  |  |  |
| 7 | 2600 | 30 | Default |  |  | 64 QAM | 120@0 (A3) |  | 60@0 (A3) | 30@0 (A3) |
| 8 | 2600 | 30 | Default |  |  | 64 QAM | 64@96 (A3) |  | 32@46 (A3) | 16@22 (A3) |
| 9 | 2600 | 30 | Default |  |  | 64 QAM | Outer_Full (A6) |  |  |  |
| 10 | 2600 | 30 | Default |  |  | 256 QAM | 120@0 (A3) |  | 60@0 (A3) | 30@0 (A3) |
| 11 | 2600 | 30 | Default |  |  | 256 QAM | 64@96 (A3) |  | 32@46 (A3) | 16@22 (A3) |
| 12 | 2600 | 30 | Default |  |  | 256 QAM | Outer_Full (A6) |  |  |  |
| 13 | 2595 | 40 | Default |  |  | 64 QAM | 16@0 (A1) |  | 8@0 (A1) | 4@0 (A1) |
| 14 | 2595 | 40 | Default |  |  | 64 QAM | 80@0 (A2) |  | 40@0 (A2) | 20@0 (A2) |
| 15 | 2595 | 40 | Default |  |  | 64 QAM | 128@2 (A3) |  | 64@1 (A3) | 32@1 (A3) |
| 16 | 2595 | 40 | Default |  |  | 64 QAM | 180@0 (A4) |  | 90@0 (A4) | 45@0 (A4) |
| 17 | 2595 | 40 | Default |  |  | 64 QAM | Outer_Full (A5) |  |  |  |
| 18 | 2595 | 40 | Default |  |  | 256 QAM | 16@0 (A1) |  | 8@0 (A1) | 4@0 (A1) |
| 19 | 2595 | 40 | Default |  |  | 256 QAM | 80@0 (A2) |  | 40@0 (A2) | 20@0 (A2) |
| 20 | 2595 | 40 | Default |  |  | 256 QAM | 128@2 (A3) |  | 64@1 (A3) | 32@1 (A3) |
| 21 | 2595 | 40 | Default |  |  | 256 QAM | 180@0 (A4) |  | 90@0 (A4) | 45@0 (A4) |
| 22 | 2595 | 40 | Default |  |  | 256 QAM | Outer_Full (A5) |  |  |  |
| 23 | 2602.5 | 25 | Default |  | CP-OFDM | 64 QAM | 100@0 (A3) |  | 50@0 (A3) | 25@0 (A3) |
| 24 | 2602.5 | 25 | Default |  |  | 64 QAM | 39@94 (A3) |  | 19@46 (A3) | 9@22 (A3) |
| 25 | 2602.5 | 25 | Default |  |  | 64 QAM | Outer_Full (A6) |  |  |  |
| 26 | 2602.5 | 25 | Default |  |  | 256 QAM | 100@0 (A3) |  | 50@0 (A3) | 25@0 (A3) |
| 27 | 2602.5 | 25 | Default |  |  | 256 QAM | 39@94 (A3) |  | 19@46 (A3) | 9@22 (A3) |
| 28 | 2602.5 | 25 | Default |  |  | 256 QAM | Outer_Full (A6) |  |  |  |
| 29 | 2600 | 30 | Default |  |  | 64 QAM | 120@0 (A3) |  | 60@0 (A3) | 30@0 (A3) |
| 30 | 2600 | 30 | Default |  |  | 64 QAM | 69@91 (A3) |  | 34@44 (A3) | 17@21 (A3) |
| 31 | 2600 | 30 | Default |  |  | 64 QAM | Outer_Full (A6) |  |  |  |
| 32 | 2600 | 30 | Default |  |  | 256 QAM | 120@0 (A3) |  | 60@0 (A3) | 30@0 (A3) |
| 33 | 2600 | 30 | Default |  |  | 256 QAM | 69@91 (A3) |  | 34@44 (A3) | 17@21 (A3) |
| 34 | 2600 | 30 | Default |  |  | 256 QAM | Outer_Full (A6) |  |  |  |
| 35 | 2595 | 40 | Default |  |  | 64 QAM | 16@0 (A1) |  | 8@0 (A1) | 4@0 (A1) |
| 36 | 2595 | 40 | Default |  |  | 64 QAM | 80@0 (A2) |  | 40@0 (A2) | 20@0 (A2) |
| 37 | 2595 | 40 | Default |  |  | 64 QAM | 130@0 (A3) |  | 65@0 (A3) | 33@0 (A3) |
| 38 | 2595 | 40 | Default |  |  | 64 QAM | 180@0 (A4) |  | 90@0 (A4) | 45@0 (A4) |
| 39 | 2595 | 40 | Default |  |  | 64 QAM | Outer_Full (A5) |  |  |  |

| A-MPR test parameters for NS_44 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 40 | 2595 | 40 | Default |  |  | 256 QAM | 16@0 (A1) | 8@0 (A1) | 4@0 (A1) |
| 41 | 2595 | 40 | Default |  |  | 256 QAM | 80@0 (A2) | 40@0 (A2) | 20@0 (A2) |
| 42 | 2595 | 40 | Default |  |  | 256 QAM | 130@0 (A3) | 65@0 (A3) | 33@0 (A3) |
| 43 | 2595 | 40 | Default |  |  | 256 QAM | 180@0 (A4) | 90@0 (A4) | 45@0 (A4) |
| 44 | 2595 | 40 | Default |  |  | 256 QAM | Outer_Full (A5) |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-27: Test Configuration table for NS_21

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_21 |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz |
| 1 | Low | Default | Default | N/A for A-MPR test cases | DFT-s-OFDM | PI/2 BPSK | Edge_1RB_Left |  |
| 2 | High | Default | Default |  |  | PI/2 BPSK | Edge_1RB_Right |  |
| 3 | Default | Default | Default |  |  | PI/2 BPSK | Outer_Full |  |
| 4 | Default | Default | Default |  |  | PI/2 BPSK | 4@0 | 2@0 |
| 5a | Default | 5 MHz | Default |  |  | PI/2 BPSK | 4@21 | 2@7 |
| 5b | Default | 10 MHz | Default |  |  | PI/2 BPSK | 4@48 | 2@22 |
| 6 | Low | Default | Default |  |  | QPSK | Edge_1RB_Left |  |
| 7 | High | Default | Default |  |  | QPSK | Edge_1RB_Right |  |
| 8 | Default | Default | Default |  |  | QPSK | Outer_Full |  |
| 9 | Default | Default | Default |  |  | QPSK | 4@0 | 2@0 |
| 10a | Default | 5 MHz | Default |  |  | QPSK | 4@21 | 2@7 |
| 10b | Default | 10 MHz | Default |  |  | QPSK | 4@48 | 2@22 |
| 11 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |  |
| 12 | High | Default | Default |  |  | 16 QAM | Edge_1RB_Right |  |
| 13 | Default | Default | Default |  |  | 16 QAM | Outer_Full |  |
| 14 | Default | Default | Default |  |  | 16 QAM | 4@0 | 2@0 |
| 15a | Default | 5 MHz | Default |  |  | 16 QAM | 4@21 | 2@7 |
| 15b | Default | 10 MHz | Default |  |  | 16 QAM | 4@48 | 2@22 |
| 16 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |  |
| 17 | High | Default | Default |  |  | 64 QAM | Edge_1RB_Right |  |
| 18 | Default | Default | Default |  |  | 64 QAM | Outer_Full |  |
| 19 | Default | Default | Default |  |  | 64 QAM | 4@0 | 2@0 |
| 20a | Default | 5 MHz | Default |  |  | 64 QAM | 4@21 | 2@7 |
| 20b | Default | 10 MHz | Default |  |  | 64 QAM | 4@48 | 2@22 |
| 21 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |  |
| 22 | High | Default | Default |  |  | 256 QAM | Edge_1RB_Right |  |
| 23 | Default | Default | Default |  |  | 256 QAM | Outer_Full |  |
| 24 | Default | Default | Default |  |  | 256 QAM | 4@0 | 2@0 |
| 25a | Default | 5 MHz | Default |  |  | 256 QAM | 4@21 | 2@7 |
| 25b | Default | 10 MHz | Default |  |  | 256 QAM | 4@48 | 2@22 |
| 26 | Low | Default | Default |  | CP-OFDM | QPSK | Edge_1RB_Left |  |
| 27 | High | Default | Default |  |  | QPSK | Edge_1RB_Right |  |
| 28 | Default | Default | Default |  |  | QPSK | Outer_Full |  |
| 29 | Default | Default | Default |  |  | QPSK | 4@0 | 2@0 |
| 30a | Default | 5 MHz | Default |  |  | QPSK | 4@21 | 2@7 |
| 30b | Default | 10 MHz | Default |  |  | QPSK | 4@48 | 2@22 |
| 31 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |  |
| 32 | High | Default | Default |  |  | 16 QAM | Edge_1RB_Right |  |
| 33 | Default | Default | Default |  |  | 16 QAM | Outer_Full |  |
| 34 | Default | Default | Default |  |  | 16 QAM | 4@0 | 2@0 |
| 35a | Default | 5 MHz | Default |  |  | 16 QAM | 4@21 | 2@7 |
| 35b | Default | 10 MHz | Default |  |  | 16 QAM | 4@48 | 2@22 |
| 36 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |  |
| 37 | High | Default | Default |  |  | 64 QAM | Edge_1RB_Right |  |
| 38 | Default | Default | Default |  |  | 64 QAM | Outer_Full |  |
| 39 | Default | Default | Default |  |  | 64 QAM | 4@0 | 2@0 |
| 40a | Default | 5 MHz | Default |  |  | 64 QAM | 4@21 | 2@7 |
| 40b | Default | 10 MHz | Default |  |  | 64 QAM | 4@48 | 2@22 |
| 41 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |  |
| 42 | High | Default | Default |  |  | 256 QAM | Edge_1RB_Right |  |
| 43 | Default | Default | Default |  |  | 256 QAM | Outer_Full |  |
| 44 | Default | Default | Default |  |  | 256 QAM | 4@0 | 2@0 |
| 45a | Default | 5 MHz | Default |  |  | 256 QAM | 4@21 | 2@7 |
| 45b | Default | 10 MHz | Default |  |  | 256 QAM | 4@48 | 2@22 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-28: Test Configuration table for NS_45

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest |  |
| A-MPR test parameters for NS_45 |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq | ChBw | SCS | N/A for A-MPR testing. | Modulation (NOTE 2) |  | RB allocation (NOTE 1) |
| 1 | Low | Default | Default |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 3 | Default |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 4 | Low |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Right |
| 6 | Default |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 7 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 8 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Right |
| 9 | Default |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 10 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Right |
| 12 | Default |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-29: Test Configuration table for NS_49 (power class 3)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies |  |  |  |  |  |  |  | Lowest and highest carrier centre frequency for each A-MPR requirement.in table 6.2.3.3.27-1 |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 25 MHz, 30MHz, 40MHz, 45MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 25 | Default | N/A for A-MPR testing. | DFT-s-OFDM | QPSK | Outer_Full (A3) |  |  |  |
| 2 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 3 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Left (A3) |  |  |  |
| 4 | Default | 30 | Default |  |  | QPSK | 20@0 (A1) |  | 10@0 (A1) | 5@0 (A1) |
| 5 | Default | 30 | Default |  |  | QPSK | 36@0 (A5) |  | 18@0 (A5) | 9@0 (A5) |
| 6 | Default | 30 | Default |  |  | QPSK | 80@0 (A3) |  | 40@0 (A3) | 20@0 (A3) |
| 7 | Default | 30 | Default |  |  | QPSK | 120@0 (A4) |  | 60@0 (A4) | 30@0 (A4) |
| 8 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 9 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |  |
| 10 | Default | 40 | Default |  |  | QPSK | 40@0 (A1) |  | 20@0 (A1) | 10@0 (A1) |
| 11 | Default | 40 | Default |  |  | QPSK | 5@53 (A5) |  | 2@27 (A5) | 1@14 (A5) |
| 12 | Default | 40 | Default |  |  | QPSK | 100@0 (A4) |  | 50@0 (A4) | 25@0 (A4) |
| 13 | Default | 40 | Default |  |  | QPSK | 150@33 (A2) |  | 75@17 (A2) | 36@9 (A2) |
| 14 | Default | 40 | Default |  |  | QPSK | 5@187 (A5) |  | 2@94 (A5) | 1@47 (A5) |
| 15 | Default | 40 | Default |  |  | QPSK | 192@0 (A1) |  | 96@0 (A1) | 48@0 (A1) |
| 16 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 17 | Default | 45 | Default |  |  | QPSK | 5@64 (A5) |  | 2@32 (A5) | 1@16 (A5) |
| 18 | Default | 45 | Default |  |  | QPSK | 5@199 (A5) |  | 2@100 (A5) | 1@50 (A5) |
| 19 | Default | 45 | Default |  |  | QPSK | 162@36 (A2) |  | 81@19 (A2) | 40@10 (A2) |
| 20 | Default | 45 | Default |  |  | QPSK | 200@0 (A1) |  | 100@0 (A1) | 50@0 (A1) |
| 21 | Default | 45 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 22 | Default | 50 | Default |  |  | QPSK | 5@75 (A5) |  | 2@38 (A5) | 1@19 (A5) |
| 23 | Default | 50 | Default |  |  | QPSK | 5@215 (A5) |  | 2@108 (A5) | 1@54 (A5) |
| 24 | Default | 50 | Default |  |  | QPSK | 150@45 (A2) |  | 81@23 (A2) | 40@12 (A2) |
| 25 | Default | 50 | Default |  |  | QPSK | 216@0 (A1) |  | 108@0 (A1) | 54@0 (A1) |
| 26 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 27 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 28 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |  |
| 29 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Left (A3) |  |  |  |
| 30 | Default | 30 | Default |  |  | 256 QAM | 20@0 (A1) |  | 10@0 (A1) | 5@0 (A1) |
| 31 | Default | 30 | Default |  |  | 256 QAM | 36@0 (A5) |  | 18@0 (A5) | 9@0 (A5) |
| 32 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A3) |  | 40@0 (A3) | 20@0 (A3) |
| 33 | Default | 30 | Default |  |  | 256 QAM | 120@0 (A4) |  | 60@0 (A4) | 30@0 (A4) |
| 34 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 35 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A5) |  |  |  |
| 36 | Default | 40 | Default |  |  | 256 QAM | 40@0 (A1) |  | 20@0 (A1) | 10@0 (A1) |
| 37 | Default | 40 | Default |  |  | 256 QAM | 5@53 (A5) |  | 2@27 (A5) | 1@14 (A5) |
| 38 | Default | 40 | Default |  |  | 256 QAM | 100@0 (A4) |  | 50@0 (A4) | 25@0 (A4) |
| 39 | Default | 40 | Default |  |  | 256 QAM | 150@33 (A2) |  | 75@17 (A2) | 36@9 (A2) |
| 40 | Default | 40 | Default |  |  | 256 QAM | 5@187 (A5) |  | 2@94 (A5) | 1@47 (A5) |
| 41 | Default | 40 | Default |  |  | 256 QAM | 192@0 (A1) |  | 96@0 (A1) | 48@0 (A1) |
| 42 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 43 | Default | 45 | Default |  |  | 256 QAM | 5@64 (A5) |  | 2@32 (A5) | 1@16 (A5) |
| 44 | Default | 45 | Default |  |  | 256 QAM | 5@199 (A5) |  | 2@100 (A5) | 1@50 (A5) |
| 45 | Default | 45 | Default |  |  | 256 QAM | 162@36 (A2) |  | 81@19 (A2) | 40@10 (A2) |
| 46 | Default | 45 | Default |  |  | 256 QAM | 200@0 (A1) |  | 100@0 (A1) | 50@0 (A1) |
| 47 | Default | 45 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 48 | Default | 50 | Default |  |  | 256 QAM | 5@75 (A5) |  | 2@38 (A5) | 1@19 (A5) |
| 49 | Default | 50 | Default |  |  | 256 QAM | 5@215 (A5) |  | 2@108 (A5) | 1@54 (A5) |
| 50 | Default | 50 | Default |  |  | 256 QAM | 150@45 (A2) |  | 81@23 (A2) | 40@12 (A2) |
| 51 | Default | 50 | Default |  |  | 256 QAM | 216@0 (A1) |  | 108@0 (A1) | 54@0 (A1) |
| 52 | Default | 50 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 53 | Default | 25 | Default |  | CP-OFDM | QPSK | Outer_Full (A3) |  |  |  |
| 54 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 55 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Left (A3) |  |  |  |
| 56 | Default | 30 | Default |  |  | QPSK | 20@0 (A1) |  | 10@0 (A1) | 5@0 (A1) |
| 57 | Default | 30 | Default |  |  | QPSK | 36@0 (A5) |  | 18@0 (A5) | 9@0 (A5) |
| 58 | Default | 30 | Default |  |  | QPSK | 80@0 (A3) |  | 40@0 (A3) | 20@0 (A3) |
| 59 | Default | 30 | Default |  |  | QPSK | 120@0 (A4) |  | 60@0 (A4) | 30@0 (A4) |
| 60 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 61 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |  |
| 62 | Default | 40 | Default |  |  | QPSK | 40@0 (A1) |  | 20@0 (A1) | 10@0 (A1) |
| 63 | Default | 40 | Default |  |  | QPSK | 5@53 (A5) |  | 2@27 (A5) | 1@14 (A5) |

| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 64 | Default | 40 | Default |  |  | QPSK | 100@0 (A4) | 50@0 (A4) | 25@0 (A4) |
| 65 | Default | 40 | Default |  |  | QPSK | 159@33 (A2) | 79@17 (A2) | 39@9 (A2) |
| 66 | Default | 40 | Default |  |  | QPSK | 5@187 (A5) | 2@94 (A5) | 1@47 (A5) |
| 67 | Default | 40 | Default |  |  | QPSK | 192@0 (A1) | 96@0 (A1) | 48@0 (A1) |
| 68 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 69 | Default | 45 | Default |  |  | QPSK | 5@64 (A5) | 2@32 (A5) | 1@16 (A5) |
| 70 | Default | 45 | Default |  |  | QPSK | 5@199 (A5) | 2@100 (A5) | 1@50 (A5) |
| 71 | Default | 45 | Default |  |  | QPSK | 168@36 (A2) | 83@19 (A2) | 40@10 (A2) |
| 72 | Default | 45 | Default |  |  | QPSK | 204@0 (A1) | 102@0 (A1) | 51@0 (A1) |
| 73 | Default | 45 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 74 | Default | 50 | Default |  |  | QPSK | 5@75 (A5) | 2@38 (A5) | 1@19 (A5) |
| 75 | Default | 50 | Default |  |  | QPSK | 5@215 (A5) | 2@108 (A5) | 1@54 (A5) |
| 76 | Default | 50 | Default |  |  | QPSK | 175@45 (A2) | 87@23 (A2) | 43@12 (A2) |
| 77 | Default | 50 | Default |  |  | QPSK | 220@0 (A1) | 110@0 (A1) | 55@0 (A1) |
| 78 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 79 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 80 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |
| 81 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Left (A3) |  |  |
| 82 | Default | 30 | Default |  |  | 256 QAM | 20@0 (A1) | 10@0 (A1) | 5@0 (A1) |
| 83 | Default | 30 | Default |  |  | 256 QAM | 36@0 (A5) | 18@0 (A5) | 9@0 (A5) |
| 84 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A3) | 40@0 (A3) | 20@0 (A3) |
| 85 | Default | 30 | Default |  |  | 256 QAM | 120@0 (A4) | 60@0 (A4) | 30@0 (A4) |
| 86 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |
| 87 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A5) |  |  |
| 88 | Default | 40 | Default |  |  | 256 QAM | 40@0 (A1) | 20@0 (A1) | 10@0 (A1) |
| 89 | Default | 40 | Default |  |  | 256 QAM | 5@53 (A5) | 2@27 (A5) | 1@14 (A5) |
| 90 | Default | 40 | Default |  |  | 256 QAM | 100@0 (A4) | 50@0 (A4) | 25@0 (A4) |
| 91 | Default | 40 | Default |  |  | 256 QAM | 159@33 (A2) | 79@17 (A2) | 39@9 (A2) |
| 92 | Default | 40 | Default |  |  | 256 QAM | 5@187 (A5) | 2@94 (A5) | 1@47 (A5) |
| 93 | Default | 40 | Default |  |  | 256 QAM | 192@0 (A1) | 96@0 (A1) | 48@0 (A1) |
| 94 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 95 | Default | 45 | Default |  |  | 256 QAM | 5@64 (A5) | 2@32 (A5) | 1@16 (A5) |
| 96 | Default | 45 | Default |  |  | 256 QAM | 5@199 (A5) | 2@100 (A5) | 1@50 (A5) |
| 97 | Default | 45 | Default |  |  | 256 QAM | 168@36 (A2) | 83@19 (A2) | 40@10 (A2) |
| 98 | Default | 45 | Default |  |  | 256 QAM | 204@0 (A1) | 102@0 (A1) | 51@0 (A1) |
| 99 | Default | 45 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 100 | Default | 50 | Default |  |  | 256 QAM | 5@75 (A5) | 2@38 (A5) | 1@19 (A5) |
| 101 | Default | 50 | Default |  |  | 256 QAM | 5@215 (A5) | 2@108 (A5) | 1@54 (A5) |
| 102 | Default | 50 | Default |  |  | 256 QAM | 175@45 (A2) | 87@23 (A2) | 43@12 (A2) |
| 103 | Default | 50 | Default |  |  | 256 QAM | 220@0 (A1) | 110@0 (A1) | 55@0 (A1) |
| 104 | Default | 50 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-29a: Test Configuration table for NS_49 (power class 2)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Low range, High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 10 MHz, 15 MHz, 20 MHz, 25 MHz, 30MHz, 40MHz, 45 MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 10 | Default | N/A for A-MPR testing. | DFT-s-OFDM | QPSK | Outer_Full (A3) |  |  | - |
| 2 | Default | 10 | Default |  |  | QPSK | Edge_1RB_Left (A3) |  |  |  |
| 3 | Default | 15 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |
| 4 | Default | 15 | Default |  |  | QPSK | 48@0 (A3) |  | 24@0 (A3) | 12@0 (A3) |
| 5 | Default | 20 | Default |  |  | QPSK | Outer_Full (A4) |  |  |  |
| 6 | Default | 20 | Default |  |  | QPSK | 50@0 (A3) |  | 25@0 (A3) | 12@0 (A3) |
| 7 | Default | 25 | Default |  |  | QPSK | Outer_Full (A4) |  |  |  |
| 8 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 9 | Default | 25 | Default |  |  | QPSK | 18@18 (A3) |  | 9@9 (A3) | 4@5 (A3) |
| 10 | Default | 30 | Default |  |  | QPSK | 20@0 (A1) |  | 10@0 (A1) | 5@0 (A1) |
| 11 | Default | 30 | Default |  |  | QPSK | 40@0 (A5) |  | 20@0 (A5) | 10@0 (A5) |
| 12 | Default | 30 | Default |  |  | QPSK | 80@0 (A3) |  | 40@0 (A3) | 20@0 (A3) |
| 13 | Default | 30 | Default |  |  | QPSK | 120@0 (A4) |  | 60@0 (A4) | 30@0 (A4) |
| 14 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 15 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A1) |  |  |  |
| 16 | Default | 40 | Default |  |  | QPSK | 50@0 (A1) |  | 25@0 (A1) | 12@0 (A1) |
| 17 | Default | 40 | Default |  |  | QPSK | 16@48 (A5) |  | 8@24 (A5) | 4@12 (A5) |
| 18 | Default | 40 | Default |  |  | QPSK | 100@0 (A4) |  | 50@0 (A4) | 25@0 (A4) |
| 19 | Default | 40 | Default |  |  | QPSK | 144@47 (A2) |  | 72@23 (A2) | 36@11 (A2) |
| 20 | Default | 40 | Default |  |  | QPSK | 5@187 (A5) |  | 2@94 (A5) | 1@47 (A5) |
| 21 | Default | 40 | Default |  |  | QPSK | 192@0 (A1) |  | 96@0 (A1) | 48@0 (A1) |
| 22 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 23 | Default | 45 | Default |  |  | QPSK | 5@46 (A5) |  | 2@24 (A5) | 1@13 (A5) |
| 24 | Default | 45 | Default |  |  | QPSK | 5@167 (A5) |  | 2@85 (A5) | 1@43 (A5) |
| 25 | Default | 45 | Default |  |  | QPSK | 125@46 (A2) |  | 60@26 (A2) | 30@13 (A2) |
| 26 | Default | 45 | Default |  |  | QPSK | 162@0 (A1) |  | 81@0 (A1) | 40@0 (A1) |
| 27 | Default | 45 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 28 | Default | 50 | Default |  |  | QPSK | 5@75 (A5) |  | 2@38 (A5) | 1@19 (A5) |
| 29 | Default | 50 | Default |  |  | QPSK | 5@215 (A5) |  | 2@108 (A5) | 1@54 (A5) |
| 30 | Default | 50 | Default |  |  | QPSK | 150@58 (A2) |  | 80@30 (A2) | 36@16 (A2) |
| 31 | Default | 50 | Default |  |  | QPSK | 216@0 (A1) |  | 108@0 (A1) | 54@0 (A1) |
| 32 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 33 | Default | 10 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  | - |
| 34 | Default | 10 | Default |  |  | 256 QAM | Edge_1RB_Left (A3) |  |  |  |
| 35 | Default | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 36 | Default | 15 | Default |  |  | 256 QAM | 48@0 (A3) |  | 24@0 (A3) | 12@0 (A3) |
| 37 | Default | 20 | Default |  |  | 256 QAM | Outer_Full (A4) |  |  |  |
| 38 | Default | 20 | Default |  |  | 256 QAM | 50@0 (A3) |  | 15@0 (A3) | 12@0 (A3) |
| 39 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A4) |  |  |  |
| 40 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |  |
| 41 | Default | 25 | Default |  |  | 256 QAM | 18@18 (A3) |  | 9@9 (A3) | 4@5 (A3) |
| 42 | Default | 30 | Default |  |  | 256 QAM | 20@0 (A1) |  | 10@0 (A1) | 5@0 (A1) |
| 43 | Default | 30 | Default |  |  | 256 QAM | 40@0 (A5) |  | 20@0 (A5) | 10@0 (A5) |
| 44 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A3) |  | 40@0 (A3) | 20@0 (A3) |
| 45 | Default | 30 | Default |  |  | 256 QAM | 120@0 (A4) |  | 60@0 (A4) | 30@0 (A4) |
| 46 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 47 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A1) |  |  |  |
| 48 | Default | 40 | Default |  |  | 256 QAM | 50@0 (A1) |  | 25@0 (A1) | 12@0 (A1) |
| 49 | Default | 40 | Default |  |  | 256 QAM | 16@48 (A5) |  | 8@24 (A5) | 4@12 (A5) |
| 50 | Default | 40 | Default |  |  | 256 QAM | 100@0 (A4) |  | 50@0 (A4) | 25@0 (A4) |
| 51 | Default | 40 | Default |  |  | 256 QAM | 144@47 (A2) |  | 72@23 (A2) | 36@11 (A2) |
| 52 | Default | 40 | Default |  |  | 256 QAM | 5@187 (A5) |  | 2@94 (A5) | 1@47 (A5) |
| 53 | Default | 40 | Default |  |  | 256 QAM | 192@0 (A1) |  | 96@0 (A1) | 48@0 (A1) |
| 54 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 55 | Default | 45 | Default |  |  | 256 QAM | 5@46 (A5) |  | 2@24 (A5) | 1@13 (A5) |
| 56 | Default | 45 | Default |  |  | 256 QAM | 5@167 (A5) |  | 2@85 (A5) | 1@43 (A5) |
| 57 | Default | 45 | Default |  |  | 256 QAM | 125@46 (A2) |  | 60@26 (A2) | 30@13 (A2) |
| 58 | Default | 45 | Default |  |  | 256 QAM | 162@0 (A1) |  | 81@0 (A1) | 40@0 (A1) |
| 59 | Default | 45 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 60 | Default | 50 | Default |  |  | 256 QAM | 5@75 (A5) |  | 2@38 (A5) | 1@19 (A5) |
| 61 | Default | 50 | Default |  |  | 256 QAM | 5@215 (A5) |  | 2@108 (A5) | 1@54 (A5) |
| 62 | Default | 50 | Default |  |  | 256 QAM | 150@58 (A2) |  | 80@30 (A2) | 36@16 (A2) |
| 63 | Default | 50 | Default |  |  | 256 QAM | 216@0 (A1) |  | 108@0 (A1) | 54@0 (A1) |

| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 64 | Default | 50 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 65 | Default | 10 | Default |  | CP-OFDM | QPSK | Outer_Full (A3) |  | - |
| 66 | Default | 10 | Default |  |  | QPSK | Edge_1RB_Left (A3) |  |  |
| 67 | Default | 15 | Default |  |  | QPSK | Outer_Full (A3) |  |  |
| 68 | Default | 15 | Default |  |  | QPSK | 48@0 (A3) | 24@0 (A3) | 12@0 (A3) |
| 69 | Default | 20 | Default |  |  | QPSK | Outer_Full (A4) |  |  |
| 70 | Default | 20 | Default |  |  | QPSK | 52@0 (A3) | 26@0 (A3) | 13@0 (A3) |
| 71 | Default | 25 | Default |  |  | QPSK | Outer_Full (A4) |  |  |
| 72 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |
| 73 | Default | 25 | Default |  |  | QPSK | 18@18 (A3) | 9@9 (A3) | 4@5 (A3) |
| 74 | Default | 30 | Default |  |  | QPSK | 20@0 (A1) | 10@0 (A1) | 5@0 (A1) |
| 75 | Default | 30 | Default |  |  | QPSK | 44@0 (A5) | 22@0 (A5) | 11@0 (A5) |
| 76 | Default | 30 | Default |  |  | QPSK | 80@0 (A3) | 40@0 (A3) | 20@0 (A3) |
| 77 | Default | 30 | Default |  |  | QPSK | 120@0 (A4) | 60@0 (A4) | 30@0 (A4) |
| 78 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |
| 79 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A1) |  |  |
| 80 | Default | 40 | Default |  |  | QPSK | 50@0 (A1) | 25@0 (A1) | 12@0 (A1) |
| 81 | Default | 40 | Default |  |  | QPSK | 16@48 (A5) | 8@24 (A5) | 4@12 (A5) |
| 82 | Default | 40 | Default |  |  | QPSK | 100@0 (A4) | 50@0 (A4) | 25@0 (A4) |
| 83 | Default | 40 | Default |  |  | QPSK | 144@47 (A2) | 72@23(A2) | 36@11 (A2) |
| 84 | Default | 40 | Default |  |  | QPSK | 5@187 (A5) | 2@94 (A5) | 1@47 (A5) |
| 85 | Default | 40 | Default |  |  | QPSK | 192@0 (A1) | 96@0 (A1) | 48@0 (A1) |
| 86 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 87 | Default | 45 | Default |  |  | QPSK | 5@46 (A5) | 2@24 (A5) | 1@13 (A5) |
| 88 | Default | 45 | Default |  |  | QPSK | 5@167 (A5) | 2@85 (A5) | 1@43 (A5) |
| 89 | Default | 45 | Default |  |  | QPSK | 125@46 (A2) | 62@24 (A2) | 30@13 (A2) |
| 90 | Default | 45 | Default |  |  | QPSK | 171@0 (A1) | 86@0 (A1) | 42@0 (A1) |
| 91 | Default | 45 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 92 | Default | 50 | Default |  |  | QPSK | 5@75 (A5) | 2@38 (A5) | 1@19 (A5) |
| 93 | Default | 50 | Default |  |  | QPSK | 5@215 (A5) | 2@108 (A5) | 1@54 (A5) |
| 94 | Default | 50 | Default |  |  | QPSK | 150@58 (A2) | 80@30 (A2) | 36@16 (A2) |
| 95 | Default | 50 | Default |  |  | QPSK | 216@0 (A1) | 108@0 (A1) | 54@0 (A1) |
| 96 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 97 | Default | 10 | Default |  |  | 256 QAM | Outer_Full (A3) |  | - |
| 98 | Default | 10 | Default |  |  | 256 QAM | Edge_1RB_Left (A3) |  |  |
| 99 | Default | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 100 | Default | 15 | Default |  |  | 256 QAM | 48@0 (A3) | 24@0 (A3) | 12@0 (A3) |
| 101 | Default | 20 | Default |  |  | 256 QAM | Outer_Full (A4) |  |  |
| 102 | Default | 20 | Default |  |  | 256 QAM | 52@0 (A3) | 26@0 (A3) | 13@0 (A3) |
| 103 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A4) |  |  |
| 104 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |
| 105 | Default | 25 | Default |  |  | 256 QAM | 18@18 (A3) | 9@9 (A3) | 4@5 (A3) |
| 106 | Default | 30 | Default |  |  | 256 QAM | 20@0 (A1) | 10@0 (A1) | 5@0 (A1) |
| 107 | Default | 30 | Default |  |  | 256 QAM | 44@0 (A5) | 22@0 (A5) | 11@0 (A5) |
| 108 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A3) | 40@0 (A3) | 20@0 (A3) |
| 109 | Default | 30 | Default |  |  | 256 QAM | 120@0 (A4) | 60@0 (A4) | 30@0 (A4) |
| 110 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |
| 111 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A1) |  |  |
| 112 | Default | 40 | Default |  |  | 256 QAM | 50@0 (A1) | 25@0 (A1) | 12@0 (A1) |
| 113 | Default | 40 | Default |  |  | 256 QAM | 16@48 (A5) | 8@24 (A5) | 4@12 (A5) |
| 114 | Default | 40 | Default |  |  | 256 QAM | 100@0 (A4) | 50@0 (A4) | 25@0 (A4) |
| 115 | Default | 40 | Default |  |  | 256 QAM | 144@47 (A2) | 72@23 (A2) | 36@11 (A2) |
| 116 | Default | 40 | Default |  |  | 256 QAM | 5@187 (A5) | 2@94 (A5) | 1@47 (A5) |
| 117 | Default | 40 | Default |  |  | 256 QAM | 192@0 (A1) | 96@0 (A1) | 48@0 (A1) |
| 118 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 119 | Default | 45 | Default |  |  | 256 QAM | 5@46 (A5) | 2@24 (A5) | 1@13 (A5) |
| 120 | Default | 45 | Default |  |  | 256 QAM | 5@167 (A5) | 2@85 (A5) | 1@43 (A5) |
| 121 | Default | 45 | Default |  |  | 256 QAM | 125@46 (A2) | 62@24 (A2) | 30@13 (A2) |
| 122 | Default | 45 | Default |  |  | 256 QAM | 171@0 (A1) | 86@0 (A1) | 42@0 (A1) |
| 123 | Default | 45 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 124 | Default | 50 | Default |  |  | 256 QAM | 5@75 (A5) | 2@38 (A5) | 1@19 (A5) |
| 125 | Default | 50 | Default |  |  | 256 QAM | 5@215 (A5) | 2@108 (A5) | 1@54 (A5) |
| 126 | Default | 50 | Default |  |  | 256 QAM | 150@58 (A2) | 80@30 (A2) | 36@16 (A2) |
| 127 | Default | 50 | Default |  |  | 256 QAM | 216@0 (A1) | 108@0 (A1) | 54@0 (A1) |
| 128 | Default | 50 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-30: Test Configuration table for NS_56

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier center frequency (Fc) as specified in test parameters |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz as specified in test parameters |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest unless otherwise specified in test parameters. |  |  |  |
| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | Low | 5 | 15 | N/A for A-MPR testing | 14 | DFT-s-OFDM | PI/2 BPSK |  | Edge_1RB_Left | N/A | N/A |
| 2 | Low | 5 | 30 |  | 8 |  | PI/2 BPSK |  | N/A | Edge_1RB_Left | N/A |
| 3 | Low | 5 | Default |  | 6 |  | PI/2 BPSK |  | Outer_Full | Outer_Full | N/A |
| 4 | Low | 5 | Default |  | 4 |  | PI/2 BPSK |  | 20@4 | 8@3 | N/A |
| 5 | Low | 5 | Default |  | 4 |  | PI/2 BPSK |  | 1@4 | 1@3 | N/A |
| 6 | Low | 5 | 15 |  | 14 |  | QPSK |  | Edge_1RB_Left | N/A | N/A |
| 7 | Low | 5 | 30 |  | 8 |  | QPSK |  | N/A | Edge_1RB_Left | N/A |
| 8 | Low | 5 | Default |  | 6 |  | QPSK |  | Outer_Full | Outer_Full | N/A |
| 9 | Low | 5 | Default |  | 4 |  | QPSK |  | 20@4 | 8@3 | N/A |
| 10 | Low | 5 | Default |  | 4 |  | QPSK |  | 1@4 | 1@3 | N/A |
| 11 | Low | 5 | 15 |  | 14 |  | 16 QAM |  | Edge_1RB_Left | N/A | N/A |
| 12 | Low | 5 | 30 |  | 8 |  | 16 QAM |  | N/A | Edge_1RB_Left | N/A |
| 13 | Low | 5 | Default |  | 6 |  | 16 QAM |  | Outer_Full | Outer_Full | N/A |
| 14 | Low | 5 | Default |  | 4 |  | 16 QAM |  | 20@4 | 8@3 | N/A |
| 15 | Low | 5 | Default |  | 4 |  | 16 QAM |  | 1@4 | 1@3 | N/A |
| 16 | Low | 5 | 15 |  | 14 |  | 64 QAM |  | Edge_1RB_Left | N/A | N/A |
| 17 | Low | 5 | 30 |  | 8 |  | 64 QAM |  | N/A | Edge_1RB_Left | N/A |
| 18 | Low | 5 | Default |  | 6 |  | 64 QAM |  | Outer_Full | Outer_Full | N/A |
| 19 | Low | 5 | Default |  | 4 |  | 64 QAM |  | 20@4 | 8@3 | N/A |
| 20 | Low | 5 | Default |  | 4 |  | 64 QAM |  | 1@4 | 1@3 | N/A |
| 21 | Low | 5 | 15 |  | 14 |  | 256 QAM |  | Edge_1RB_Left | N/A | N/A |
| 22 | Low | 5 | 30 |  | 8 |  | 256 QAM |  | N/A | Edge_1RB_Left | N/A |
| 23 | Low | 5 | Default |  | 6 |  | 256 QAM |  | Outer_Full | Outer_Full | N/A |
| 24 | Low | 5 | Default |  | 4 |  | 256 QAM |  | 20@4 | 8@3 | N/A |
| 25 | Low | 5 | Default |  | 4 |  | 256 QAM |  | 1@4 | 1@3 | N/A |
| 26 | Low | 10 | 15 |  | 12 |  | PI/2 BPSK |  | Edge_1RB_Left | N/A | N/A |
| 27 | Low | 10 | Highest |  | 8 |  | PI/2 BPSK |  | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 28 | Low | 10 | Default |  | 8 |  | PI/2 BPSK |  | Outer_Full | Outer_Full | Outer_Full |
| 29 | Low | 10 | Default |  | 6 |  | PI/2 BPSK |  | 1@3 | 1@2 | 1@1 |
| 30 | Low | 10 | Default |  | 6 |  | PI/2 BPSK |  | 40@9 | 18@5 | 8@2 |
| 31 | Low | 10 | Default |  | 4 |  | PI/2 BPSK |  | 1@35 | 1@17 | 1@8 |
| 32 | Low | 10 | Default |  | 2 |  | PI/2 BPSK |  | 16@35 | 5@19 | 3@8 |
| 33 | Low | 10 | Default |  | 5 |  | PI/2 BPSK |  | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 34 | Low | 10 | 15 |  | 5 |  | PI/2 BPSK |  | 1@40 | N/A | N/A |
| 35 | Low | 10 | 30 |  | 5 |  | PI/2 BPSK |  | N/A | 1@20 | N/A |
| 36 | Low | 10 | 15 |  | 3 |  | PI/2 BPSK |  | 8@44 | N/A | N/A |
| 37 | Low | 10 | 15 |  | 12 |  | QPSK |  | Edge_1RB_Left | N/A | N/A |
| 38 | Low | 10 | Highest |  | 8 |  | QPSK |  | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 39 | Low | 10 | Default |  | 8 |  | QPSK |  | Outer_Full | Outer_Full | Outer_Full |
| 40 | Low | 10 | Default |  | 6 |  | QPSK |  | 1@3 | 1@2 | 1@1 |
| 41 | Low | 10 | Default |  | 6 |  | QPSK |  | 40@9 | 18@5 | 8@2 |
| 42 | Low | 10 | Default |  | 4 |  | QPSK |  | 1@35 | 1@17 | 1@8 |
| 43 | Low | 10 | Default |  | 2 |  | QPSK |  | 16@35 | 5@19 | 3@8 |
| 44 | Low | 10 | Default |  | 5 |  | QPSK |  | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 45 | Low | 10 | 15 |  | 5 |  | QPSK |  | 1@40 | N/A | N/A |

| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 46 | Low | 10 | 30 |  | 5 |  | QPSK | N/A | 1@20 | N/A |
| 47 | Low | 10 | 15 |  | 3 |  | QPSK | 8@44 | N/A | N/A |
| 48 | Low | 10 | 15 |  | 12 |  | 16 QAM | Edge_1RB_Left | N/A | N/A |
| 49 | Low | 10 | Highest |  | 8 |  | 16 QAM | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 50 | Low | 10 | Default |  | 8 |  | 16 QAM | Outer_Full | Outer_Full | Outer_Full |
| 51 | Low | 10 | Default |  | 6 |  | 16 QAM | 1@3 | 1@2 | 1@1 |
| 52 | Low | 10 | Default |  | 6 |  | 16 QAM | 40@9 | 18@5 | 8@2 |
| 53 | Low | 10 | Default |  | 4 |  | 16 QAM | 1@35 | 1@17 | 1@8 |
| 54 | Low | 10 | Default |  | 2 |  | 16 QAM | 16@35 | 5@19 | 3@8 |
| 55 | Low | 10 | Default |  | 5 |  | 16 QAM | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 56 | Low | 10 | 15 |  | 5 |  | 16 QAM | 1@40 | N/A | N/A |
| 57 | Low | 10 | 30 |  | 5 |  | 16 QAM | N/A | 1@20 | N/A |
| 58 | Low | 10 | 15 |  | 3 |  | 16 QAM | 8@44 | N/A | N/A |
| 59 | Low | 10 | 15 |  | 12 |  | 64 QAM | Edge_1RB_Left | N/A | N/A |
| 60 | Low | 10 | Highest |  | 8 |  | 64 QAM | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 61 | Low | 10 | Default |  | 8 |  | 64 QAM | Outer_Full | Outer_Full | Outer_Full |
| 62 | Low | 10 | Default |  | 6 |  | 64 QAM | 1@3 | 1@2 | 1@1 |
| 63 | Low | 10 | Default |  | 6 |  | 64 QAM | 40@9 | 18@5 | 8@2 |
| 64 | Low | 10 | Default |  | 4 |  | 64 QAM | 1@35 | 1@17 | 1@8 |
| 65 | Low | 10 | Default |  | 2 |  | 64 QAM | 16@35 | 5@19 | 3@8 |
| 66 | Low | 10 | Default |  | 5 |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 67 | Low | 10 | 15 |  | 5 |  | 64 QAM | 1@40 | N/A | N/A |
| 68 | Low | 10 | 30 |  | 5 |  | 64 QAM | N/A | 1@20 | N/A |
| 69 | Low | 10 | 15 |  | 3 |  | 64 QAM | 8@44 | N/A | N/A |
| 70 | Low | 10 | 15 |  | 12 |  | 256 QAM | Edge_1RB_Left | N/A | N/A |
| 71 | Low | 10 | Highest |  | 8 |  | 256 QAM | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 72 | Low | 10 | Default |  | 8 |  | 256 QAM | Outer_Full | Outer_Full | Outer_Full |
| 73 | Low | 10 | Default |  | 6 |  | 256 QAM | 1@3 | 1@2 | 1@1 |
| 74 | Low | 10 | Default |  | 6 |  | 256 QAM | 40@9 | 18@5 | 8@2 |
| 75 | Low | 10 | Default |  | 4 |  | 256 QAM | 1@35 | 1@17 | 1@8 |
| 76 | Low | 10 | Default |  | 2 |  | 256 QAM | 16@35 | 5@19 | 3@8 |
| 77 | Low | 10 | Default |  | 5 |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 78 | Low | 10 | 15 |  | 5 |  | 256 QAM | 1@40 | N/A | N/A |
| 79 | Low | 10 | 30 |  | 5 |  | 256 QAM | N/A | 1@20 | N/A |
| 80 | Low | 10 | 15 |  | 3 |  | 256 QAM | 8@44 | N/A | N/A |
| 81 | Low | 5 | 15 | N/A for A-MPR testing | 14 | CP-OFDM | QPSK | Edge_1RB_Left | N/A | N/A |
| 82 | Low | 5 | 30 |  | 8 |  | QPSK | N/A | Edge_1RB_Left | N/A |
| 83 | Low | 5 | Default |  | 6 |  | QPSK | Outer_Full | Outer_Full | N/A |
| 84 | Low | 5 | Default |  | 4 |  | QPSK | 21@4 | 8@3 | N/A |
| 85 | Low | 5 | Default |  | 4 |  | QPSK | 1@4 | 1@3 | N/A |
| 86 | Low | 5 | 15 |  | 14 |  | 16 QAM | Edge_1RB_Left | N/A | N/A |
| 87 | Low | 5 | 30 |  | 8 |  | 16 QAM | N/A | Edge_1RB_Left | N/A |
| 88 | Low | 5 | Default |  | 6 |  | 16 QAM | Outer_Full | Outer_Full | N/A |
| 89 | Low | 5 | Default |  | 4 |  | 16 QAM | 21@4 | 8@3 | N/A |
| 90 | Low | 5 | Default |  | 4 |  | 16 QAM | 1@4 | 1@3 | N/A |
| 91 | Low | 5 | 15 |  | 14 |  | 64 QAM | Edge_1RB_Left | N/A | N/A |
| 92 | Low | 5 | 30 |  | 8 |  | 64 QAM | N/A | Edge_1RB_Left | N/A |
| 93 | Low | 5 | Default |  | 6 |  | 64 QAM | Outer_Full | Outer_Full | N/A |
| 94 | Low | 5 | Default |  | 4 |  | 64 QAM | 21@4 | 8@3 | N/A |
| 95 | Low | 5 | Default |  | 4 |  | 64 QAM | 1@4 | 1@3 | N/A |
| 96 | Low | 5 | 15 |  | 14 |  | 256 QAM | Edge_1RB_Left | N/A | N/A |
| 97 | Low | 5 | 30 |  | 8 |  | 256 QAM | N/A | Edge_1RB_Left | N/A |
| 98 | Low | 5 | Default |  | 6 |  | 256 QAM | Outer_Full | Outer_Full | N/A |
| 99 | Low | 5 | Default |  | 4 |  | 256 QAM | 21@4 | 8@3 | N/A |

| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 100 | Low | 5 | Default |  | 4 |  | 256 QAM | 1@4 | 1@3 | N/A |
| 101 | Low | 10 | 15 |  | 12 |  | QPSK | Edge_1RB_Left | N/A | N/A |
| 102 | Low | 10 | Highest |  | 8 |  | QPSK | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 103 | Low | 10 | Default |  | 8 |  | QPSK | Outer_Full | Outer_Full | Outer_Full |
| 104 | Low | 10 | Default |  | 6 |  | QPSK | 1@3 | 1@2 | 1@1 |
| 105 | Low | 10 | Default |  | 6 |  | QPSK | 43@9 | 19@5 | 8@2 |
| 106 | Low | 10 | Default |  | 4 |  | QPSK | 1@35 | 1@17 | 1@8 |
| 107 | Low | 10 | Default |  | 2 |  | QPSK | 17@35 | 5@19 | 3@8 |
| 108 | Low | 10 | Default |  | 5 |  | QPSK | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 109 | Low | 10 | 15 |  | 5 |  | QPSK | 1@40 | N/A | N/A |
| 110 | Low | 10 | 30 |  | 5 |  | QPSK | N/A | 1@20 | N/A |
| 111 | Low | 10 | 15 |  | 3 |  | QPSK | 8@44 | N/A | N/A |
| 112 | Low | 10 | 15 |  | 12 |  | 16 QAM | Edge_1RB_Left | N/A | N/A |
| 113 | Low | 10 | Highest |  | 8 |  | 16 QAM | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 114 | Low | 10 | Default |  | 8 |  | 16 QAM | Outer_Full | Outer_Full | Outer_Full |
| 115 | Low | 10 | Default |  | 6 |  | 16 QAM | 1@3 | 1@2 | 1@1 |
| 116 | Low | 10 | Default |  | 6 |  | 16 QAM | 43@9 | 19@5 | 8@2 |
| 117 | Low | 10 | Default |  | 4 |  | 16 QAM | 1@35 | 1@17 | 1@8 |
| 118 | Low | 10 | Default |  | 2 |  | 16 QAM | 17@35 | 5@19 | 3@8 |
| 119 | Low | 10 | Default |  | 5 |  | 16 QAM | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 120 | Low | 10 | 15 |  | 5 |  | 16 QAM | 1@40 | N/A | N/A |
| 121 | Low | 10 | 30 |  | 5 |  | 16 QAM | N/A | 1@20 | N/A |
| 122 | Low | 10 | 15 |  | 3 |  | 16 QAM | 8@44 | N/A | N/A |
| 123 | Low | 10 | 15 |  | 12 |  | 64 QAM | Edge_1RB_Left | N/A | N/A |
| 124 | Low | 10 | Highest |  | 8 |  | 64 QAM | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 125 | Low | 10 | Default |  | 8 |  | 64 QAM | Outer_Full | Outer_Full | Outer_Full |
| 126 | Low | 10 | Default |  | 6 |  | 64 QAM | 1@3 | 1@2 | 1@1 |
| 127 | Low | 10 | Default |  | 6 |  | 64 QAM | 43@9 | 19@5 | 8@2 |
| 128 | Low | 10 | Default |  | 4 |  | 64 QAM | 1@35 | 1@17 | 1@8 |
| 129 | Low | 10 | Default |  | 2 |  | 64 QAM | 17@35 | 5@19 | 3@8 |
| 130 | Low | 10 | Default |  | 5 |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 131 | Low | 10 | 15 |  | 5 |  | 64 QAM | 1@40 | N/A | N/A |
| 132 | Low | 10 | 30 |  | 5 |  | 64 QAM | N/A | 1@20 | N/A |
| 133 | Low | 10 | 15 |  | 3 |  | 64 QAM | 8@44 | N/A | N/A |
| 134 | Low | 10 | 15 |  | 12 |  | 256 QAM | Edge_1RB_Left | N/A | N/A |
| 135 | Low | 10 | Highest |  | 8 |  | 256 QAM | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 136 | Low | 10 | Default |  | 8 |  | 256 QAM | Outer_Full | Outer_Full | Outer_Full |
| 137 | Low | 10 | Default |  | 6 |  | 256 QAM | 1@3 | 1@2 | 1@1 |
| 138 | Low | 10 | Default |  | 6 |  | 256 QAM | 43@9 | 19@5 | 8@2 |
| 139 | Low | 10 | Default |  | 4 |  | 256 QAM | 1@35 | 1@17 | 1@8 |
| 140 | Low | 10 | Default |  | 2 |  | 256 QAM | 17@35 | 5@19 | 3@8 |
| 141 | Low | 10 | Default |  | 5 |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 142 | Low | 10 | 15 |  | 5 |  | 256 QAM | 1@40 | N/A | N/A |
| 143 | Low | 10 | 30 |  | 5 |  | 256 QAM | N/A | 1@20 | N/A |
| 144 | Low | 10 | 15 |  | 3 |  | 256 QAM | 8@44 | N/A | N/A |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-31: Test Configuration table for NS_07 and NS_50

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_07 and NS_50 |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz |
| 1 | Low | Default | Default | N/A for A-MPR test cases | DFT-s-OFDM | PI/2 BPSK | Edge_1RB_Left |  |
| 2 | Low | 5 MHz | Default |  |  | PI/2 BPSK | 8@6 | 4@3 |
| 3 | Low | 10 MHz | Default |  |  | PI/2 BPSK | 8@14 | 4@7 |
| 4 | Low | 5 MHz | Default |  |  | PI/2 BPSK | 10@8 | 5@4 |
| 5 | Low | 10 MHz | Default |  |  | PI/2 BPSK | 18@19 | 10@10 |
| 6 | High | 5 MHz | Default |  |  | PI/2 BPSK | 10@0 | 5@0 |
| 7 | Low | 5 MHz | Default |  |  | PI/2 BPSK | 2@20 | 1@10 |
| 8 | Low | 10 MHz | Default |  |  | PI/2 BPSK | 2@37 | 1@19 |
| 9 | Low | Default | Default |  |  | QPSK | Edge_1RB_Left |  |
| 10 | Low | 5 MHz | Default |  |  | QPSK | 8@6 | 4@3 |
| 11 | Low | 10 MHz | Default |  |  | QPSK | 8@14 | 4@7 |
| 12 | Low | 5 MHz | Default |  |  | QPSK | 10@8 | 5@4 |
| 13 | Low | 10 MHz | Default |  |  | QPSK | 18@19 | 10@10 |
| 14 | High | 5 MHz | Default |  |  | QPSK | 10@0 | 5@0 |
| 15 | Low | 5 MHz | Default |  |  | QPSK | 2@20 | 1@10 |
| 16 | Low | 10 MHz | Default |  |  | QPSK | 2@37 | 1@19 |
| 17 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |  |
| 18 | Low | 5 MHz | Default |  |  | 16 QAM | 8@6 | 4@3 |
| 19 | Low | 10 MHz | Default |  |  | 16 QAM | 8@14 | 4@7 |
| 20 | Low | 5 MHz | Default |  |  | 16 QAM | 10@8 | 5@4 |
| 21 | Low | 10 MHz | Default |  |  | 16 QAM | 18@19 | 10@10 |
| 22 | High | 5 MHz | Default |  |  | 16 QAM | 10@0 | 5@0 |
| 23 | Low | 5 MHz | Default |  |  | 16 QAM | 2@20 | 1@10 |
| 24 | Low | 10 MHz | Default |  |  | 16 QAM | 2@37 | 1@19 |
| 25 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |  |
| 26 | Low | 5 MHz | Default |  |  | 64 QAM | 8@6 | 4@3 |
| 27 | Low | 10 MHz | Default |  |  | 64 QAM | 8@14 | 4@7 |
| 28 | Low | 5 MHz | Default |  |  | 64 QAM | 10@8 | 5@4 |
| 29 | Low | 10 MHz | Default |  |  | 64 QAM | 18@19 | 10@10 |
| 30 | High | 5 MHz | Default |  |  | 64 QAM | 10@0 | 5@0 |
| 31 | Low | 5 MHz | Default |  |  | 64 QAM | 2@20 | 1@10 |
| 32 | Low | 10 MHz | Default |  |  | 64 QAM | 2@37 | 1@19 |
| 33 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |  |
| 34 | Low | 5 MHz | Default |  |  | 256 QAM | 8@6 | 4@3 |
| 35 | Low | 10 MHz | Default |  |  | 256 QAM | 8@14 | 4@7 |
| 36 | Low | 5 MHz | Default |  |  | 256 QAM | 10@8 | 5@4 |
| 37 | Low | 10 MHz | Default |  |  | 256 QAM | 18@19 | 10@10 |
| 38 | High | 5 MHz | Default |  |  | 256 QAM | 10@0 | 5@0 |
| 39 | Low | 5 MHz | Default |  |  | 256 QAM | 2@20 | 1@10 |
| 40 | Low | 10 MHz | Default |  |  | 256 QAM | 2@37 | 1@19 |
| 41 | Low | Default | Default |  | CP-OFDM | QPSK | Edge_1RB_Left |  |
| 42 | Low | 5 MHz | Default |  |  | QPSK | 7@6 | 4@3 |
| 43 | Low | 10 MHz | Default |  |  | QPSK | 8@14 | 4@7 |
| 44 | Low | 5 MHz | Default |  |  | QPSK | 10@8 | 5@4 |
| 45 | Low | 10 MHz | Default |  |  | QPSK | 18@19 | 10@10 |
| 46 | High | 5 MHz | Default |  |  | QPSK | 10@0 | 5@0 |
| 47 | Low | 5 MHz | Default |  |  | QPSK | 2@20 | 1@10 |
| 48 | Low | 10 MHz | Default |  |  | QPSK | 2@37 | 1@19 |
| 49 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |  |
| 50 | Low | 5 MHz | Default |  |  | 16 QAM | 7@6 | 4@3 |
| 51 | Low | 10 MHz | Default |  |  | 16 QAM | 8@14 | 4@7 |
| 52 | Low | 5 MHz | Default |  |  | 16 QAM | 10@8 | 5@4 |
| 53 | Low | 10 MHz | Default |  |  | 16 QAM | 18@19 | 10@10 |
| 54 | High | 5 MHz | Default |  |  | 16 QAM | 10@0 | 5@0 |
| 55 | Low | 5 MHz | Default |  |  | 16 QAM | 2@20 | 1@10 |
| 56 | Low | 10 MHz | Default |  |  | 16 QAM | 2@37 | 1@19 |
| 57 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |  |
| 58 | Low | 5 MHz | Default |  |  | 64 QAM | 7@6 | 4@3 |
| 59 | Low | 10 MHz | Default |  |  | 64 QAM | 8@14 | 4@7 |
| 60 | Low | 5 MHz | Default |  |  | 64 QAM | 10@8 | 5@4 |
| 61 | Low | 10 MHz | Default |  |  | 64 QAM | 18@19 | 10@10 |
| 62 | High | 5 MHz | Default |  |  | 64 QAM | 10@0 | 5@0 |
| 63 | Low | 5 MHz | Default |  |  | 64 QAM | 2@20 | 1@10 |
| 64 | Low | 10 MHz | Default |  |  | 64 QAM | 2@37 | 1@19 |
| 65 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |  |
| 66 | Low | 5 MHz | Default |  |  | 256 QAM | 7@6 | 4@3 |
| 67 | Low | 10 MHz | Default |  |  | 256 QAM | 8@14 | 4@7 |
| 68 | Low | 5 MHz | Default |  |  | 256 QAM | 10@8 | 5@4 |
| 69 | Low | 10 MHz | Default |  |  | 256 QAM | 18@19 | 10@10 |
| 70 | High | 5 MHz | Default |  |  | 256 QAM | 10@0 | 5@0 |
| 71 | Low | 5 MHz | Default |  |  | 256 QAM | 2@20 | 1@10 |
| 72 | Low | 10 MHz | Default |  |  | 256 QAM | 2@37 | 1@19 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-32: Test Configuration table for NS_50 (Power Class 2)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | (See Freq column for test frequency, and configuration detail specified in Table 6.2.3.4.1-32a, Table 6.2.3.4.1-32b and Table 6.2.3.4.1-32c) |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 10MHz, 15MHz, 20MHz, 25 MHz, 30MHz, 35MHz, 40MHz |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |  |
| A-MPR test parameters for NS_50 (Power Class 2) |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz |  | SCS 60 kHz |
| 1 | 1890 | 10 | Default | N/A | DFT-s-OFDM | QPSK | Edge_1RB_left (A5) |  |  |  |
| 2 | 1890 | 10 | Default |  |  | QPSK | Outer_Full (A4) |  |  |  |
| 3 | 1890 | 10 | Default |  |  | QPSK | 40@11 (A4) | 18@6 (A4) |  | / |
| 4 | 1892.5 | 15 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |
| 5 | 1892.5 | 15 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |
| 6 | 1892.5 | 15 | Default |  |  | QPSK | 60@19 (A4) | 27@10 (A4) |  | 15@5 (A4) |
| 7 | 1895 | 20 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |
| 8 | 1895 | 20 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |
| 9 | 1895 | 20 | Default |  |  | QPSK | 80@26 (A4) | 36@13 (A4) |  | 18@7 (A4) |
| 10 | 1897.5 | 25 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 11 | 1897.5 | 25 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |
| 12 | 1897.5 | 25 | Default |  |  | QPSK | 75@48 (A4) | 36@25 (A4) |  | 18@13 (A4) |
| 13 | 1897.5 | 25 | Default |  |  | QPSK | 20@11 (A6) | 10@6 (A6) |  | 5@3 (A6) |
| 14 | 1897.5 | 25 | Default |  |  | QPSK | 27@0 (A4) | 12@0 (A4) |  | 6@0 (A4) |
| 15 | 1900 | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 16 | 1900 | 30 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |
| 17 | 1900 | 30 | Default |  |  | QPSK | 20@11 (A6) | 10@6 (A6) |  | 5@3 (A6) |
| 18 | 1900 | 30 | Default |  |  | QPSK | 27@0 (A4) | 12@0 (A4) |  | 6@0 (A4) |
| 19 | 1900 | 30 | Default |  |  | QPSK | 90@63 (A4) | 45@32 (A4) |  | 24@18 (A4) |
| 20 | 1900 | 35 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 21 | 1900 | 35 | Default |  |  | QPSK | 50@10 (A3) | 25@5 (A3) |  | 12@3 (A3) |
| 22 | 1900 | 35 | Default |  |  | QPSK | 178@10 (A8) | 87@5 (A8) |  | 40@3 (A8) |
| 23 | 1900 | 35 | Default |  |  | QPSK | 111@77 (A4) | 52@39 (A4) |  | 24@20 (A4) |
| 24 | 1900 | 35 | Default |  |  | QPSK | Edge 1RB right (A9) |  |  |  |
| 25 | 1900 | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 26 | 1900 | 40 | Default |  |  | QPSK | 180@25 (A8) | 90@13 (A8) |  | 40@7 (A8) |
| 27 | 1900 | 40 | Default |  |  | QPSK | 60@25 (A3) | 30@13 (A3) |  | 15@7 (A3) |
| 28 | 1900 | 40 | Default |  |  | QPSK | 90@25 (A7) | 45@13 (A7) |  | 20@7 (A7) |
| 29 | 1900 | 40 | Default |  |  | QPSK | 108@101 (A4) | 54@51 (A4) |  | 25@26 (A4) |
| 30 | 1900 | 40 | Default |  |  | QPSK | 36@177 (A9) | 16@89 (A9) |  | 6@45 (A9) |
| 31 | 1890 | 10 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 32 | 1890 | 10 | Default |  |  | 256 QAM | Outer_Full (A4) |  |  |  |
| 33 | 1890 | 10 | Default |  |  | 256 QAM | 40@11 (A4) | 18@6 (A4) |  | / |
| 34 | 1892.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 35 | 1892.5 | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 36 | 1892.5 | 15 | Default |  |  | 256 QAM | 60@19 (A4) | 27@10 (A4) |  | 15@5 (A4) |
| 37 | 1895 | 20 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 38 | 1895 | 20 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 39 | 1895 | 20 | Default |  |  | 256 QAM | 80@26 (A4) | 36@13 (A4) |  | 18@7 (A4) |
| 40 | 1897.5 | 25 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 41 | 1897.5 | 25 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 42 | 1897.5 | 25 | Default |  |  | 256 QAM | 75@48 (A4) | 36@25 (A4) |  | 18@13 (A4) |
| 43 | 1897.5 | 25 | Default |  |  | 256 QAM | 20@11 (A6) | 10@6 (A6) |  | 5@3 (A6) |
| 44 | 1897.5 | 25 | Default |  |  | 256 QAM | 27@0 (A4) | 12@0 (A4) |  | 6@0 (A4) |
| 45 | 1900 | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 46 | 1900 | 30 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 47 | 1900 | 30 | Default |  |  | 256 QAM | 20@11 (A6) | 10@6 (A6) |  | 5@3 (A6) |
| 48 | 1900 | 30 | Default |  |  | 256 QAM | 27@0 (A4) | 12@0 (A4) |  | 6@0 (A4) |
| 49 | 1900 | 30 | Default |  |  | 256 QAM | 90@63 (A4) | 45@32 (A4) |  | 24@18 (A4) |
| 50 | 1900 | 35 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 51 | 1900 | 35 | Default |  |  | 256 QAM | 50@10 (A3) | 25@5 (A3) |  | 12@3 (A3) |
| 52 | 1900 | 35 | Default |  |  | 256 QAM | 178@10 (A8) | 87@5 (A8) |  | 40@3 (A8) |
| 53 | 1900 | 35 | Default |  |  | 256 QAM | 111@77 (A4) | 52@39 (A4) |  | 24@20 (A4) |
| 54 | 1900 | 35 | Default |  |  | 256 QAM | Edge 1RB right (A9) |  |  |  |
| 55 | Low | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 56 | Low | 40 | Default |  |  | 256 QAM | 180@25 (A8) | 90@13 (A8) |  | 40@7 (A8) |
| 57 | Low | 40 | Default |  |  | 256 QAM | 60@25 (A3) | 30@13 (A3) |  | 15@7 (A3) |
| 58 | Low | 40 | Default |  |  | 256 QAM | 90@25 (A7) | 45@13 (A7) |  | 20@7 (A7) |
| 59 | Low | 40 | Default |  |  | 256 QAM | 108@101 (A4) | 54@51 (A4) |  | 25@26 (A4) |
| 60 | Low | 40 | Default |  |  | 256 QAM | 36@177 (A9) | 16@89 (A9) |  | 6@45 (A9) |
| 61 | 1890 | 10 | Default | N/A | CP-OFDM | QPSK | Edge_1RB_left (A5) |  |  |  |
| 62 | 1890 | 10 | Default |  |  | QPSK | Outer_Full (A4) |  |  |  |
| 63 | 1890 | 10 | Default |  |  | QPSK | 41@11 (A4) | 18@6 (A4) | / |  |
| 64 | 1892.5 | 15 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |
| 65 | 1892.5 | 15 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |
| 66 | 1892.5 | 15 | Default |  |  | QPSK | 60@19 (A4) | 28@10 (A4) | 13@5 (A4) |  |
| 67 | 1895 | 20 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |
| 68 | 1895 | 20 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |
| 69 | 1895 | 20 | Default |  |  | QPSK | 80@26 (A4) | 38@13 (A4) | 17@7 (A4) |  |
| 70 | 1897.5 | 25 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 71 | 1897.5 | 25 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |
| 72 | 1897.5 | 25 | Default |  |  | QPSK | 74@47 (A4) | 37@24 (A4) | 19@13 (A4) |  |
| 73 | 1897.5 | 25 | Default |  |  | QPSK | 20@11 (A6) | 10@6 (A6) | 5@3 (A6) |  |
| 74 | 1897.5 | 25 | Default |  |  | QPSK | 27@0 (A4) | 13@0 (A4) | 6@0 (A4) |  |
| 75 | 1900 | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 76 | 1900 | 30 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |
| 77 | 1900 | 30 | Default |  |  | QPSK | 20@11 (A6) | 10@6 (A6) | 5@3 (A6) |  |
| 78 | 1900 | 30 | Default |  |  | QPSK | 27@0 (A4) | 13@0 (A4) | 6@0 (A4) |  |
| 79 | 1900 | 30 | Default |  |  | QPSK | 88@61 (A4) | 44@31 (A4) | 22@16 (A4) |  |
| 80 | 1900 | 35 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 81 | 1900 | 35 | Default |  |  | QPSK | 50@10 (A3) | 25@5 |  | 12@3 |
| 82 | 1900 | 35 | Default |  |  | QPSK | 178@10 (A8) | 87@5 (A8) |  | 40@3 (A8) |
| 83 | 1900 | 35 | Default |  |  | QPSK | 111@77 (A4) | 52@39 (A4) |  | 24@20 (A4) |
| 84 | 1900 | 35 | Default |  |  | QPSK | Edge 1RB right (A9) |  |  |  |
| 85 | 1900 | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 86 | 1900 | 40 | Default |  |  | QPSK | 191@25 (A8) | 93@13 (A8) | 44@7 (A8) |  |
| 87 | 1900 | 40 | Default |  |  | QPSK | 60@25 (A3) | 30@13 (A3) | 15@7 (A3) |  |
| 88 | 1900 | 40 | Default |  |  | QPSK | 91@25 (A7) | 46@13 (A7) | 23@7 (A7) |  |
| 89 | 1900 | 40 | Default |  |  | QPSK | 115@101 (A4) | 55@51 (A4) | 25@26 (A4) |  |
| 90 | 1900 | 40 | Default |  |  | QPSK | 39@177 (A9) | 17@89 (A9) | 6@45 (A9) |  |
| 91 | 1890 | 10 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 92 | 1890 | 10 | Default |  |  | 256 QAM | Outer_Full (A4) |  |  |  |
| 93 | 1890 | 10 | Default |  |  | 256 QAM | 41@11 (A4) | 18@6 (A4) | / |  |
| 94 | 1892.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 95 | 1892.5 | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 96 | 1892.5 | 15 | Default |  |  | 256 QAM | 60@19 (A4) | 28@10 (A4) | 13@5 (A4) |  |
| 97 | 1895 | 20 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 98 | 1895 | 20 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 99 | 1895 | 20 | Default |  |  | 256 QAM | 80@26 (A4) | 38@13 (A4) | 17@7 (A4) |  |
| 100 | 1897.5 | 25 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 101 | 1897.5 | 25 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 102 | 1897.5 | 25 | Default |  |  | 256 QAM | 74@47 (A4) | 37@24 (A4) | 19@13 (A4) |  |
| 103 | 1897.5 | 25 | Default |  |  | 256 QAM | 20@11 (A6) | 10@6 (A6) | 5@3 (A6) |  |
| 104 | 1897.5 | 25 | Default |  |  | 256 QAM | 27@0 (A4) | 13@0 (A4) | 6@0 (A4) |  |
| 105 | 1900 | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 106 | 1900 | 30 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 107 | 1900 | 30 | Default |  |  | 256 QAM | 20@11 (A6) | 10@6 (A6) | 5@3 (A6) |  |
| 108 | 1900 | 30 | Default |  |  | 256 QAM | 27@0 (A4) | 13@0 (A4) | 6@0 (A4) |  |
| 109 | 1900 | 30 | Default |  |  | 256 QAM | 88@61 (A4) | 44@31 (A4) | 22@16 (A4) |  |
| 110 | 1900 | 35 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 111 | 1900 | 35 | Default |  |  | 256 QAM | 50@10 (A3) | 25@5 (A3) | 12@3 (A3) |  |
| 112 | 1900 | 35 | Default |  |  | 256 QAM | 178@10 (A8) | 87@5 (A8) | 41@3 (A8) |  |
| 113 | 1900 | 35 | Default |  |  | 256 QAM | 111@77 (A4) | 53@39 (A4) | 24@20 (A4) |  |
| 114 | 1900 | 35 | Default |  |  | 256 QAM | Edge 1RB right (A9) |  |  |  |
| 115 | 1900 | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 116 | 1900 | 40 | Default |  |  | 256 QAM | 191@25 (A8) | 93@13 (A8) | 44@7 (A8) |  |
| 117 | 1900 | 40 | Default |  |  | 256 QAM | 60@25 (A3) | 30@13 (A3) | 15@7 (A3) |  |
| 118 | 1900 | 40 | Default |  |  | 256 QAM | 91@25 (A7) | 46@13 (A7) | 23@7 (A7) |  |
| 119 | 1900 | 40 | Default |  |  | 256 QAM | 115@101 (A4) | 55@51 (A4) | 25@26 (A4) |  |
| 120 | 1900 | 40 | Default |  |  | 256 QAM | 39@177 (A9) | 17@89 (A9) | 6@45 (A9) |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-32a: Additional test frequencies for NS_50 (SCS=15 kHz, ΔFRaster = 15 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 | DL & UL | - | 1890 | 378000 | 1885.32 | 377064 | 0 | 30 | 4725 | 378030 | 10 | 0 | 1(6) | 6 |
| 15 | 79 | DL & UL | - | 1892.5 | 378500 | 1885.39 | 377078 | 0 | 15 | 4729 | 378290 | 8 | 7 | 7(16) | 23 |
| 20 | 106 | DL & UL | - | 1895 | 379000 | 1885.46 | 377092 | 0 | 15 | 4718 | 377530 | 2 | 0 | 1(2) | 2 |
| 25 | 133 | DL & UL | - | 1897.5 | 379500 | 1885.53 | 377106 | 0 | 15 | 4743 | 379470 | 8 | 17 | 12(38) | 55 |
| 35 | 188 | DL & UL | - | 1900 | 380000 | 1883.08 | 376616 | 0 | 15 | 4750 | 379970 | 2 | 81 | 1(2) | 83 |
| 30 | 160 | DL & UL | - | 1900 | 380000 | 1885.6 | 377120 | 0 | 15 | 4729 | 378290 | 6 | 6 | 7(16) | 22 |
| 40 | 216 | DL & UL | - | 1900 | 380000 | 1880.56 | 376112 | 0 | 15 | 4729 | 378290 | 6 | 12 | 12(38) | 50 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-32b: Additional test frequencies for NS_50 (SCS=30 kHz, ΔFRaster = 30 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | DL & UL | - | 1890 | 378000 | 1885.68 | 377136 | 0 | 30 | 4725 | 378030 | 10 | 0 | 2(2) | 4 |
| 15 | 38 | DL & UL | - | 1892.5 | 378500 | 1885.66 | 377132 | 0 | 15 | 4729 | 378290 | 2 | 3 | 3(8) | 22 |
| 20 | 51 | DL & UL | - | 1895 | 379000 | 1885.82 | 377164 | 0 | 15 | 4754 | 380410 | 2 | 20 | 9(20) | 80 |
| 25 | 65 | DL & UL | - | 1897.5 | 379500 | 1885.8 | 377160 | 0 | 15 | 4743 | 379470 | 2 | 7 | 9(20) | 54 |
| 30 | 78 | DL & UL | - | 1900 | 380000 | 1885.96 | 377192 | 0 | 15 | 4729 | 378290 | 6 | 2 | 3(8) | 20 |
| 35 | 92 | DL & UL | - | 1900 | 380000 | 1883.44 | 376688 | 0 | 15 | 4750 | 379970 | 14 | 32 | 3(8) | 81 |
| 40 | 106 | DL & UL | - | 1900 | 380000 | 1880.92 | 376184 | 0 | 15 | 4729 | 378290 | 6 | 4 | 9(20) | 48 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-32c: Additional test frequencies for NS_50 (SCS=60 kHz, ΔFRaster = 15 kHz, without CORESET#0)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | DL & UL | - | 1890 | 378000 | 1886.04 | 377208 | 0 | 15 | - | 377568 |
| 15 | 18 | DL & UL | - | 1892.5 | 378500 | 1886.02 | 377204 | 0 | 15 | - | 377564 |
| 20 | 24 | DL & UL | - | 1895 | 379000 | 1886.36 | 377272 | 0 | 15 | - | 377632 |
| 25 | 31 | DL & UL | - | 1897.5 | 379500 | 1886.34 | 377268 | 0 | 15 | - | 377628 |
| 30 | 38 | DL & UL | - | 1900 | 380000 | 1886.32 | 377264 | 0 | 15 | - | 377624 |
| 35 | 44 | DL & UL | - | 1900 | 380000 | 1884.16 | 376832 | 0 | 15 |  | 377192 |
| 40 | 51 | DL & UL | - | 1900 | 380000 | 1881.64 | 376328 | 0 | 15 | - | 376688 |

Table 6.2.3.4.1-33: Test Configuration table for NS_50 (Power Class 3)

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | (See Freq column) |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 25 MHz, 30MHz, 35MHz, 40MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_50 (Power Class 3) |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | 1897.5 | 25 | Default | N/A | DFT-s-OFDM | QPSK | Outer_Full (A7) |  |  |
| 2 | 1897.5 | 25 | Default |  |  | QPSK | Edge_1RB_left (A8) |  |  |
| 3 | 1897.5 | 25 | Default |  |  | QPSK | 20@0 (A9) | 10@0 (A9) | 5@0 (A9) |
| 4 | 1900 | 30 | Default |  |  | QPSK | Outer_Full (A7) |  |  |
| 5 | 1900 | 30 | Default |  |  | QPSK | Edge_1RB_left (A8) |  |  |
| 6 | 1900 | 30 | Default |  |  | QPSK | 20@0 (A9) | 10@0 (A9) | 5@0 (A9) |
| 7 | 1900 | 35 | Default |  |  | QPSK |  |  |  |
| 8 | 1900 | 35 | Default |  |  | QPSK |  |  |  |
| 9 | 1900 | 35 | Default |  |  | QPSK |  |  |  |
| 10 | 1900 | 35 | Default |  |  | QPSK |  |  |  |
| 11 | 1900 | 35 | Default |  |  | QPSK |  |  |  |
| 12 | 1900 | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 13 | 1900 | 40 | Default |  |  | QPSK | 60@25 (A3) | 30@13 (A3) | 15@7 (A3) |
| 14 | 1900 | 40 | Default |  |  | QPSK | 180@25 (A2) | 90@13 (A2) | 40@7 (A2) |
| 15 | 1900 | 40 | Default |  |  | QPSK | 108@101 (A6) | 54@51 (A6) | 25@26 (A6) |
| 16 | 1900 | 40 | Default |  |  | QPSK | 36@177 (A5) | 16@89 (A5) | 6@45 (A5) |
| 17 | 1897.5 | 25 | Default |  |  | 256 QAM | Outer_Full (A7) |  |  |
| 18 | 1897.5 | 25 | Default |  |  | 256 QAM | Edge_1RB_left (A8) |  |  |
| 19 | 1897.5 | 25 | Default |  |  | 256 QAM | 20@0 (A9) | 10@0 (A9) | 5@0 (A9) |
| 20 | 1900 | 30 | Default |  |  | 256 QAM | Outer_Full (A7) |  |  |
| 21 | 1900 | 30 | Default |  |  | 256 QAM | Edge_1RB_left (A8) |  |  |
| 22 | 1900 | 30 | Default |  |  | 256 QAM | 20@0 (A9) | 10@0 (A9) | 5@0 (A9) |
| 23 | 1900 | 35 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 24 | 1900 | 35 | Default |  |  | QPSK | 50@10 (A3) | 25@5 (A3) | 12@3 (A3) |
| 25 | 1900 | 35 | Default |  |  | QPSK | 178@10 (A2) | 87@5 (A2) | 40@3 (A2) |
| 26 | 1900 | 35 | Default |  |  | QPSK | 111@77 (A6) | 52@39 (A6) | 24@20 (A6) |
| 27 | 1900 | 35 | Default |  |  | QPSK | Edge 1RB right (A9) |  |  |
| 28 | 1900 | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 29 | 1900 | 40 | Default |  |  | 256 QAM | 60@25 (A3) | 30@13 (A3) | 15@7 (A3) |
| 30 | 1900 | 40 | Default |  |  | 256 QAM | 180@25 (A2) | 90@13 (A2) | 40@7 (A2) |
| 31 | 1900 | 40 | Default |  |  | 256 QAM | 108@101 (A6) | 54@51 (A6) | 25@26 (A6) |
| 32 | 1900 | 40 | Default |  |  | 256 QAM | 36@177 (A5) | 16@89 (A5) | 6@45 (A5) |
| 33 | 1897.5 | 25 | Default | N/A | CP-OFDM | QPSK | Outer_Full (A7) |  |  |
| 34 | 1897.5 | 25 | Default |  |  | QPSK | Edge_1RB_left (A8) |  |  |
| 35 | 1897.5 | 25 | Default |  |  | QPSK | 20@0 (A9) | 10@0 (A9) | 5@0 (A9) |
| 36 | 1900 | 30 | Default |  |  | QPSK | Outer_Full (A7) |  |  |
| 37 | 1900 | 30 | Default |  |  | QPSK | Edge_1RB_left (A8) |  |  |
| 38 | 1900 | 30 | Default |  |  | QPSK | 20@0 (A9) | 10@0 (A9) | 5@0 (A9) |
| 39 | 1900 | 35 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 40 | 1900 | 35 | Default |  |  | QPSK | 50@10 (A3) | 25@5 (A3) | 12@3 (A3) |
| 41 | 1900 | 35 | Default |  |  | QPSK | 178@10 (A2) | 87@5 (A2) | 41@3 (A2) |
| 42 | 1900 | 35 | Default |  |  | QPSK | 111@77 (A6) | 53@39 (A6) | 24@20 (A6) |
| 43 | 1900 | 35 | Default |  |  | QPSK | Edge 1RB right (A9) |  |  |
| 44 | 1900 | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 45 | 1900 | 40 | Default |  |  | QPSK | 60@25 (A3) | 30@13 (A3) | 15@7 (A3) |
| 46 | 1900 | 40 | Default |  |  | QPSK | 191@25 (A2) | 93@13 (A2) | 44@7 (A2) |
| 47 | 1900 | 40 | Default |  |  | QPSK | 115@101 (A6) | 55@51 (A6) | 25@26 (A6) |
| 48 | 1900 | 40 | Default |  |  | QPSK | 39@177 (A5) | 17@89 (A5) | 6@45 (A5) |
| 49 | 1897.5 | 25 | Default |  |  | 256 QAM | Outer_Full (A7) |  |  |
| 50 | 1897.5 | 25 | Default |  |  | 256 QAM | Edge_1RB_left (A8) |  |  |
| 51 | 1897.5 | 25 | Default |  |  | 256 QAM | 20@0 (A9) | 10@0 (A9) | 5@0 (A9) |
| 52 | 1900 | 30 | Default |  |  | 256 QAM | Outer_Full (A7) |  |  |
| 53 | 1900 | 30 | Default |  |  | 256 QAM | Edge_1RB_left (A8) |  |  |
| 54 | 1900 | 30 | Default |  |  | 256 QAM | 20@0 (A9) | 10@0 (A9) | 5@0 (A9) |
| 55 | 1900 | 35 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 56 | 1900 | 35 | Default |  |  | 256 QAM | 50@10 (A3) | 25@5 (A3) | 12@3 (A3) |
| 57 | 1900 | 35 | Default |  |  | 256 QAM | 178@10 (A2) | 87@5 (A2) | 41@3 (A2) |
| 58 | 1900 | 35 | Default |  |  | 256 QAM | 111@77 (A6) | 53@39 (A6) | 24@20 (A6) |
| 59 | 1900 | 35 | Default |  |  | 256 QAM | Edge 1RB right (A9) |  |  |
| 60 | Low | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 61 | Low | 40 | Default |  |  | 256 QAM | 60@25 (A3) | 30@13 (A3) | 15@7 (A3) |
| 62 | Low | 40 | Default |  |  | 256 QAM | 191@25 (A2) | 93@13 (A2) | 44@7 (A2) |
| 63 | Low | 40 | Default |  |  | 256 QAM | 115@101 (A6) | 55@51 (A6) | 25@26 (A6) |
| 64 | Low | 40 | Default |  |  | 256 QAM | 39@177 (A5) | 17@89 (A5) | 6@45 (A5) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

Editor’s note: The following lines belong at the end of subclause 6.2.3.4.1. As new tables are added to this section, these lines should always follow the tables

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according Annex G.0, G.1, G.2 and G.3.0.

4.	The UL Reference Measurement channels are set according to the applicable table from Table 6.2.3.4.1-1 to Table 6.2.3.4.1-30.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2.3.4.3.

6.2.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to the applicable table from Table 6.2.3.4.1-1 to Table 6.2.3.4.1-31. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration one sub-frame (1ms). For TDD, only slots consisting of only UL symbols are under test.

4.	For network signalling value “NS_04” and UEs supporting Power Class 2, repeat steps 1~3 on the applicable bands with message exception of P-Max defined in Table 6.2.3.4.3.4-2.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2.3.4.1-1 to Table 6.2.3.4.1-20, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2.3.4.3	Message contents

6.2.3.4.3.0	General

Message contents are according to TS 38.508-1 [5] subclause 4.6.1, with the following exceptions for each network signalling value.

For almost contiguous allocation testing, message contents are according to TS 38.508-1 [5] subclause 4.6.1 with the following exceptions:

Table 6.2.3.4.3-1: PUSCH-Config

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-118 PUSCH-Config |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  |  |
| } |  |  |  |

6.2.3.4.3.1	Message contents exceptions for network signalling value “NS_03”

1.	Information element additionalSpectrumEmission is set to NS_03. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03" and NR band n2, n25, n66 and n86

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_03) |  |  |

Table 6.2.3.4.3.1-2: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03" and NR band n70

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_03) |  |  |

6.2.3.4.3.2	Message contents exceptions for network signalling value "NS_35"

1.	Information element additionalSpectrumEmission is set to NS_35. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_35" and NR band n71

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_35) |  |  |

6.2.3.4.3.3	Message contents exceptions for network signalling value "NS_03U"

1.	Information element additionalSpectrumEmission is set to NS_03U. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.3-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03U"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_03U) |  |  |

6.2.3.4.3.4	Message contents exceptions for network signalling value "NS_04"

1.	Information element additionalSpectrumEmission is set to NS_04. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.4-1: AdditionalSpectrumEmission Additional spurious emissions test requirement for "NS_04"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_04) |  |  |

Table 6.2.3.4.3.4-2: P-Max (Step 4)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  | PC2 UE |

Table 6.2.3.4.3.4-3: Void

6.2.3.4.3.5	Message contents exceptions for network signalling value "NS_05"

1.	Information element additionalSpectrumEmission is set to NS_05. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.5-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_05"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_05) |  |  |

6.2.3.4.3.6	Message contents exceptions for network signalling value "NS_05U"

1.	Information element additionalSpectrumEmission is set to NS_05U. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.6-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_05U"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_05U) |  |  |

6.2.3.4.3.7	Message contents exceptions for network signalling value "NS_06"

1.	Information element additionalSpectrumEmission is set to NS_06. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.7-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_06"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_06) |  |  |

6.2.3.4.3.8	Message contents exceptions for network signalling value "NS_08"

1.	Information element additionalSpectrumEmission is set to NS_08. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.8-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_08"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_08) |  |  |

6.2.3.4.3.9	Message contents exceptions for network signalling value "NS_08U"

1.	Information element additionalSpectrumEmission is set to NS_08U. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.9-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_08U"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_08U) |  |  |

6.2.3.4.3.10	Message contents exceptions for network signalling value "NS_10"

1.	Information element additionalSpectrumEmission is set to NS_10. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.10-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_10"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_10) |  |  |

6.2.3.4.3.11	Message contents exceptions for network signalling value "NS_17"

1.	Information element additionalSpectrumEmission is set to NS_17. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.11-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_17"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_17) |  |  |

6.2.3.4.3.12	Message contents exceptions for network signalling value "NS_18"

1.	Information element additionalSpectrumEmission is set to NS_18. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.12-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_18"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_18) |  |  |

6.2.3.4.3.13	Message contents exceptions for network signalling value "NS_37"

1.	Information element additionalSpectrumEmission is set to NS_37. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.13-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_37"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_37) |  |  |

6.2.3.4.3.14	Message contents exceptions for network signalling value "NS_38"

1.	Information element additionalSpectrumEmission is set to NS_38. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.14-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_38"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_38) |  |  |

6.2.3.4.3.15	Message contents exceptions for network signalling value "NS_39"

1.	Information element additionalSpectrumEmission is set to NS_39. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.15-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_39"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_39) |  |  |

6.2.3.4.3.16	Message contents exceptions for network signalling value "NS_40"

1.	Information element additionalSpectrumEmission is set to NS_40. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.16-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_40"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_40) |  |  |

6.2.3.4.3.17	Message contents exceptions for network signalling value "NS_41"

1.	Information element additionalSpectrumEmission is set to NS_41. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.17-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_41"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_41) |  |  |

6.2.3.4.3.18	Message contents exceptions for network signalling value "NS_42"

1.	Information element additionalSpectrumEmission is set to NS_42. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.18-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_42"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_42) |  |  |

6.2.3.4.3.19	Message contents exceptions for network signalling value "NS_100"

1.	Information element additionalSpectrumEmission is set to NS_100. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.19-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_100"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_100) |  | not for band n65 |
|  | 2 (NS_100) |  | for band n65 |

6.2.3.4.3.20	Message contents exceptions for network signalling value "NS_21"

1.	Information element additionalSpectrumEmission is set to NS_21. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.20-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_21"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_21) |  |  |

6.2.3.4.3.21	Message contents exceptions for network signalling value "NS_24"

1.	Information element additionalSpectrumEmission is set to NS_24. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.21-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_24"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_24) |  |  |

6.2.3.4.3.22	Message contents exceptions for network signalling value "NS_27"

1.	Information element additionalSpectrumEmission is set to NS_27. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.22-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_27"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_27) |  |  |

6.2.3.4.3.23	Message contents exceptions for network signalling value "NS_43"

1.	Information element additionalSpectrumEmission is set to NS_43. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.23-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_43"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_43) |  |  |

6.2.3.4.3.24	Message contents exceptions for network signalling value "NS_47"

1.	Information element additionalSpectrumEmission is set to NS_47. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.24-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_47"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_47) |  |  |

6.2.3.4.3.25	Message contents exceptions for network signalling value "NS_48"

1.	Information element additionalSpectrumEmission is set to NS_48. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.25-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_48"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 4 (NS_48) |  |  |

6.2.3.4.3.26	Message contents exceptions for network signalling value "NS_49"

1.	Information element additionalSpectrumEmission is set to NS_49. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.26-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_49"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 5 (NS_49) |  |  |

6.2.3.4.3.27	Message contents exceptions for network signalling value "NS_12"

1.	Information element additionalSpectrumEmission is set to NS_12. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.27-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_12"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_12) |  |  |

6.2.3.4.3.28	Message contents exceptions for network signalling value "NS_13"

1.	Information element additionalSpectrumEmission is set to NS_13. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.28-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_13"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_13) |  |  |

6.2.3.4.3.29	Message contents exceptions for network signalling value "NS_14"

1.	Information element additionalSpectrumEmission is set to NS_14. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.29-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_14"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 4 (NS_14) |  |  |

6.2.3.4.3.30	Message contents exceptions for network signalling value "NS_15"

1.	Information element additionalSpectrumEmission is set to NS_15. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.30-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_15"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 5 (NS_15) |  |  |

6.2.3.4.3.31	Message contents exceptions for network signalling value "NS_45"

1.	Information element additionalSpectrumEmission is set to NS_45. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

6.2.3.4.3.32	Message contents exceptions for network signalling value "NS_46"

1.	Information element additionalSpectrumEmission is set to NS_46. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.32-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_46"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_46) |  |  |

6.2.3.4.3.33	Message contents exceptions for network signalling value "NS_44"

1.	Information element additionalSpectrumEmission is set to NS_44. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.33-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_44"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_44) |  |  |

6.2.3.4.3.34	Message contents exceptions for network signalled value "NS_56"

1.	Information element additionalSpectrumEmission is set to NS_56. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.34-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_56"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_56) |  |  |

6.2.3.4.3.35	Message contents exceptions for network signalled value "NS_07"

1.	Information element additionalSpectrumEmission is set to NS_07. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.35-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_07"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_07) |  |  |

6.2.3.4.3.36	Message contents exceptions for network signalled value "NS_50"

1.	Information element additionalSpectrumEmission is set to NS_50. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.36-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_50"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_50) |  |  |

6.2.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable Table from table 6.2.3.5-1 to Table 6.2.3.5-38. The allowed A-MPR values specified in Table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.1.3-1 apply.

Table 6.2.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2.3.5-1: UE Power Class 3 test requirements (NS_35) for band n71

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 2 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 3 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 4 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 5 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 6 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 7 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 8 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 9 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 10 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 11 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 12 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 13 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 14 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 15 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 16 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 17 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 18 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 19 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 20 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 21 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 22 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 23 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 24 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 25 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 26 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 27 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

r

Table 6.2.3.5-2: UE Power Class 2 test requirements (NS_04) for band n41 (Step 3)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR’ (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 26 | 3.5 | 5.5 | 1.5 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 8 | 26 | 3.5 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 9 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 10 | 26 | 0.5 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 11 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 12 | 26 | 0.5 | 0 | 0 | 25.5 | 2.0 | 3 | 28+TT | 22.5-TT |
| 13 | 26 | 3.5 | 6 | 1.5 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 14 | 26 | 3.5 | 4.5 | 0 | 21 | 2.0 | 3 | 28+TT | 18.5-TT |
| 15 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 16 | 26 | 1 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28+TT | 18.5-TT |
| 17 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 18 | 26 | 1 | 0 | 0 | 25 | 2.0 | 3 | 28+TT | 22-TT |
| 19 | 26 | 3.5 | 6 | 1.5 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 20 | 26 | 3.5 | 5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 21 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 22 | 26 | 2 | 5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 23 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 24 | 26 | 2 | 0 | 0 | 24 | 2.0 | 3 | 28+TT | 21-TT |
| 25 | 26 | 3.5 | 6.5 | 1.5 | 18 | 4.0 | 3 | 28+TT | 14-TT |
| 26 | 26 | 3.5 | 5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 27 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 28 | 26 | 2.5 | 5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 29 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 30 | 26 | 2.5 | 0 | 0 | 23.5 | 2.0 | 3 | 28+TT | 20.5-TT |
| 31 | 26 | 4.5 | 8 | 1.5 | 16.5 | 5.0 | 3 | 28+TT | 11.5-TT |
| 32 | 26 | 4.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 33 | 26 | 4.5 | 0 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 34 | 26 | 4.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 35 | 26 | 4.5 | 0 | 1.5 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 36 | 26 | 4.5 | 0 | 0 | 21.5 | 2.0 | 3 | 28+TT | 18.5-TT |
| 37 | 26 | 3.5 | 7.5 | 1.5 | 17 | 5.0 | 3 | 28+TT | 12-TT |
| 38 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 39 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 40 | 26 | 3 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 41 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 42 | 26 | 3 | 0 | 0 | 23 | 2.0 | 3 | 28+TT | 20-TT |
| 43 | 26 | 3.5 | 7.5 | 1.5 | 17 | 5.0 | 3 | 28+TT | 12-TT |
| 44 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 45 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 46 | 26 | 3 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 47 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 48 | 26 | 3 | 0 | 0 | 23 | 2.0 | 3 | 28+TT | 20-TT |
| 49 | 26 | 3.5 | 7.5 | 1.5 | 17 | 5.0 | 3 | 28+TT | 12-TT |
| 50 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 51 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 52 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 53 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 54 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 55 | 26 | 6.5 | 10 | 1.5 | 14.5 | 5.0 | 3 | 28+TT | 9.5-TT |
| 56 | 26 | 6.5 | 7.5 | 0 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 57 | 26 | 6.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 58 | 26 | 6.5 | 7.5 | 0 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 59 | 26 | 6.5 | 0 | 1.5 | 18 | 4.0 | 3 | 28+TT | 14-TT |
| 60 | 26 | 6.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Void NOTE 3: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-3: UE Power Class 3 test requirements (NS_04) for band n41 (Step 3) and UE Power Class 2 test requirements (NS_04) for band n41 (Step 4)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR’ (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 3.5 | 5.5 | 1.5 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 2 | 23 | -3 | 3.5 | 3.5 | 0 | 22.5 | 2.0 | 2 | 28+TT | 20.5-TT |
| 3 | 23 | -3 | 3.5 | 0 | 0 | 22.5 | 2.0 | 2 | 28+TT | 20.5-TT |
| 4 | 23 | -3 | 1.2 | 3.5 | 0 | 22.5 | 2.0 | 2 | 28+TT | 20.5-TT |
| 5 | 23 | -3 | 3.5 | 0 | 1.5 | 21 | 2.0 | 2 | 28+TT | 19-TT |
| 6 | 23 | -3 | 1.2 | 0 | 0 | 24.8 | 2.0 | 2 | 28+TT | 22.8-TT |
| 7 | 23 | 0 | 0.5 | 3.5 | 1.5 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 8 | 23 | 0 | 0.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 9 | 23 | 0 | 0.5 | 0 | 0 | 22.5 | 2.0 | 2 | 25+TT | 20.5-TT |
| 10 | 23 | 0 | 0.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 11 | 23 | 0 | 0.5 | 0 | 1.5 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 12 | 23 | 0 | 0.5 | 0 | 0 | 22.5 | 2.0 | 2 | 25+TT | 20.5-TT |
| 13 | 23 | 0 | 1 | 4 | 1.5 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 14 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 15 | 23 | 0 | 1 | 0 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 16 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 17 | 23 | 0 | 1 | 0 | 1.5 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 18 | 23 | 0 | 1 | 0 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 19 | 23 | 0 | 2 | 4 | 1.5 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 20 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 21 | 23 | 0 | 2 | 0 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 22 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 23 | 23 | 0 | 2 | 0 | 1.5 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 24 | 23 | 0 | 2 | 0 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 25 | 23 | 0 | 2.5 | 4.5 | 1.5 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 26 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 27 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 28 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 29 | 23 | 0 | 2.5 | 0 | 1.5 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 30 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 31 | 23 | 0 | 4.5 | 6 | 1.5 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 32 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 33 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 34 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 35 | 23 | 0 | 4.5 | 0 | 1.5 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 36 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 37 | 23 | 0 | 3 | 5.5 | 1.5 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 38 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR’ (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 40 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 41 | 23 | 0 | 3 | 0 | 1.5 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 42 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 43 | 23 | 0 | 3 | 5.5 | 1.5 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 44 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 45 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 46 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 47 | 23 | 0 | 3 | 0 | 1.5 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 48 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 49 | 23 | 0 | 3.5 | 5.5 | 1.5 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 50 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 51 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 52 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 53 | 23 | 0 | 3.5 | 0 | 1.5 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 54 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 55 | 23 | 0 | 6.5 | 8 | 1.5 | 13.5 | 5.0 | 2 | 25+TT | 8.5-TT |
| 56 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 57 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 58 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 59 | 23 | 0 | 6.5 | 0 | 1.5 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 60 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Void NOTE 3: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-3a: UE Power Class 1 test requirements (NS_04) for band n41 (Step 3)

| Test ID | PPowerClass (dBm) | MPR (dB) |  | A-MPR (dB) |  |  |  | ΔTC,c (dB) | PCMAX,c (dBm) |  | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | <50 MHz CBW | ≥50 MHz CBW | regrowth | IMD3 | CIM3 | edge |  | <50 MHz CBW | ≥50 MHz CBW |  |  |  | <50 MHz CBW | ≥50 MHz CBW |
| 1 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 0 | 11 | 0 | 20 | 20 | 2.5 | 3 | 33+TT | 17-TT | 17-TT |
| 2 | 31 | 0.5 | 0.5 | 0 | 2 | 0 | 0 | 0 | 29 | 29 | 2.5 | 3 | 33+TT | 26-TT | 26-TT |
| 3 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 4 | 31 | 0.5 | 0.5 | 0 | 0 | 0 | 0 | 0 | 32.5 | 32.5 | 2.5 | 3 | 33+TT | 29.5-TT | 29.5-TT |
| 5 | 31 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2 | 3 | 33+TT | 24-TT | 24-TT |
| 6 | 31 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 29 | 29 | 2.5 | 3 | 33+TT | 26-TT | 26-TT |
| 7 | 31 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 29 | 29 | 2.5 | 3 | 33+TT | 26-TT | 26-TT |
| 8 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 2 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 9 | 31 | 1 | 1 | 0 | 2 | 0 | 0 | 0 | 29 | 29 | 2.5 | 3 | 33+TT | 26-TT | 26-TT |
| 10 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21-4-TT | 19.5-TT |
| 11 | 31 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 30 | 30 | 2.5 | 3 | 33+TT | 27-TT | 27-TT |
| 12 | 31 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2 | 3 | 33+TT | 24-TT | 24-TT |
| 13 | 31 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 14 | 31 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 15 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 2.5 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 16 | 31 | 2 | 2 | 0 | 2.5 | 0 | 0 | 0 | 28.5 | 28.5 | 2.5 | 3 | 33+TT | 25.5-TT | 25.5-TT |
| 17 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 2 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 18 | 31 | 2 | 2 | 0 | 0 | 2 | 0 | 0 | 29 | 29 | 2.5 | 3 | 33+TT | 26-TT | 26-TT |
| 19 | 31 | 1 | 1 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2 | 3 | 33+TT | 24-TT | 24-TT |
| 20 | 31 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 21 | 31 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 22 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 3 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 23 | 31 | 2.5 | 2.5 | 0 | 3 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 24 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 3 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 25 | 31 | 2.5 | 2.5 | 0 | 0 | 3 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 26 | 31 | 2.5 | 2.5 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2 | 3 | 33+TT | 24-TT | 24-TT |
| 27 | 31 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 28 | 31 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 29 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 30 | 31 | 4.5 | 4.5 | 0 | 0 | 0 | 0 | 0 | 26.5 | 26.5 | 2.5 | 3 | 33+TT | 23.5-TT | 23.5-TT |
| 31 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 32 | 31 | 4.5 | 4.5 | 0 | 0 | 0 | 0 | 0 | 26.5 | 26.5 | 2.5 | 3 | 33+TT | 23.5-TT | 23.5-TT |
| 33 | 31 | 4.5 | 4.5 | 0 | 0 | 0 | 0 | 0 | 26.5 | 26.5 | 2 | 3 | 33+TT | 23.5-TT | 23.5-TT |
| 34 | 31 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 31 | 31 | 2 | 3 | 33+TT | 28-TT | 28-TT |
| 35 | 31 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 31 | 31 | 2 | 3 | 33+TT | 28-TT | 28-TT |
| 36 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 0 | 11 | 0 | 20 | 20 | 2.5 | 3 | 33+TT | 17-TT | 17-TT |
| 37 | 31 | 3 | 3 | 0 | 2.5 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 38 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 10.5 | 0 | 0 | 20.5 | 20.5 | 2.5 | 3 | 33+TT | 17.5-TT | 17.5-TT |
| 39 | 31 | 3 | 3 | 6 | 0 | 0 | 0 | 0 | 25 | 25 | 2 | 3 | 33+TT | 22-TT | 22-TT |
| 40 | 31 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2.5 | 3 | 33+TT | 24-TT | 24-TT |
| 41 | 31 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2.5 | 3 | 33+TT | 24-TT | 24-TT |
| 42 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 3 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 43 | 31 | 3 | 3 | 0 | 3 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 44 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 10.5 | 0 | 0 | 20.5 | 20.5 | 2.5 | 3 | 33+TT | 17.5-TT | 17.5-TT |
| 45 | 31 | 3 | 3 | 6 | 0 | 0 | 0 | 0 | 25 | 25 | 2 | 3 | 33+TT | 22-TT | 22-TT |
| 46 | 31 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2.5 | 3 | 33+TT | 24-TT | 24-TT |
| 47 | 31 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2.5 | 3 | 33+TT | 24-TT | 24-TT |
| 48 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 49 | 31 | 3.5 | 3.5 | 0 | 0 | 0 | 0 | 0 | 27.5 | 27.5 | 2.5 | 3 | 33+TT | 24.5-TT | 24.5-TT |
| 50 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 10.5 | 0 | 0 | 20.5 | 20.5 | 2.5 | 3 | 33+TT | 17.5-TT | 17.5-TT |
| 51 | 31 | 3.5 | 3.5 | 6 | 0 | 0 | 0 | 0 | 25 | 25 | 2 | 3 | 33+TT | 22-TT | 22-TT |
| 52 | 31 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2.5 | 3 | 33+TT | 24-TT | 24-TT |
| 53 | 31 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2.5 | 3 | 33+TT | 24-TT | 24-TT |
| 54 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 55 | 31 | 6.5 | 6.5 | 0 | 0 | 0 | 0 | 0 | 24.5 | 24.5 | 2.5 | 3 | 33+TT | 21.5-TT | 21.5-TT |
| 56 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 11 | 0 | 0 | 20 | 20 | 2.5 | 3 | 33+TT | 17-TT | 17-TT |
| 57 | 31 | 6.5 | 6.5 | 0 | 0 | 0 | 0 | 0 | 24.5 | 24.5 | 2 | 3 | 33+TT | 21.5-TT | 21.5-TT |
| 58 | 31 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 31 | 31 | 2 | 3 | 33+TT | 28-TT | 28-TT |
| 59 | 31 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 31 | 31 | 2 | 3 | 33+TT | 28-TT | 28-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-4: UE Power Class 3 test requirements (NS_03 and NS_03U) for band n66, n70

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 4, 5 | NS_03, NS_03U | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 6 | NS_03, NS_03U | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7, 8 | NS_03, NS_03U | 23 | 2 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 9 | NS_03, NS_03U | 23 | 2 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 10, 11 | NS_03, NS_03U | 23 | 2.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 12 | NS_03, NS_03U | 23 | 2.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 13, 14 | NS_03, NS_03U | 23 | 4.5 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 15 | NS_03, NS_03U | 23 | 4.5 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 16, 17 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19, 20 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 21 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22, 23 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 24 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 25, 26 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| 27 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-5: UE Power Class 3 test requirements (NS_03 and NS_03U) for band n2, n25

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | NS_03 | 23 | 0.5 | 1.5 | 1.5 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 1.5 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 3 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 4, 5 | NS_03; NS_03U | 23 | 1 | 2 | 1.5 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 6 | NS_03; NS_03U | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7, 8 | NS_03; NS_03U | 23 | 2 | 3 | 1.5 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 9 | NS_03; NS_03U | 23 | 2 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 10, 11 | NS_03; NS_03U | 23 | 2.5 | 3.5 | 1.5 | 18 | 4 | 2 | 25+TT | 14-TT |
| 12 | NS_03; NS_03U | 23 | 2.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 13, 14 | NS_03; NS_03U | 23 | 4.5 | 5.5 | 1.5 | 16 | 5 | 2 | 25+TT | 11-TT |
| 15 | NS_03; NS_03U | 23 | 4.5 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 16, 17 | NS_03; NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 18 | NS_03; NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19, 20 | NS_03; NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 21 | NS_03; NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22, 23 | NS_03; NS_03U | 23 | 3.5 | 4.5 | 1.5 | 17 | 5 | 2 | 25+TT | 12-TT |
| 24 | NS_03; NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 25, 26 | NS_03; NS_03U | 23 | 6.5 | 7.5 | 1.5 | 14 | 5 | 2 | 25+TT | 9-TT |
| 27 | NS_03; NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-6: UE Power Class 3 test requirements (NS_05) for bands n1, n65

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 2 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 3 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 0.5 | 1 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 6 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 7 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 9 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 10 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 11 | 23 | 0 | 0.5 | 1 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 12 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 13 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 0.5 | 1 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 16 | 23 | 0 | 1.0 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 17 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 18 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 20 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 21 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 22 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 23 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 24 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 25 | 23 | 0 | 1.0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 26 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 27 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 28 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 29 | 23 | 0 | 1.0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 30 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 31 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 32 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 33 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 34 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 35 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 36 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 37 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 38 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 39 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 40 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 41 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 42 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 43 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 44 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 45 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 46 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 47 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 48 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 49 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 50 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 51 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 52 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 53 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 54 | 23 | 0 | 4.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 55 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 56 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 57 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 58 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 59 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 61 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 62 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 63 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 64 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 65 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 66 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 67 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 68 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 69 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 70 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 71 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 72 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 73 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 74 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 75 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 76 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 77 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 78 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 79 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 80 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 81 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 82 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 83 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 84 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 85 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 86 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 87 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 88 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 89 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 90 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 91 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 92 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 93 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 94 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 95 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 96 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 97 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 98 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 99 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 100 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 101 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 102 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 103 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 104 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 105 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 106 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 107 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 108 | 23 | 0 | 6.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 109 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 110 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 111 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 112 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 113 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 114 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 115 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-6a: UE Power Class 2 test requirements (NS_05 and NS_05U) for bands n1, n65

| Test ID | PPowerClass (dBm) | DPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 0.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 2 | 26 | 0 | 0 | 3 | 0 | 23 | 2 | 3 | 28+TT | 20-TT |
| 3 | 26 | 0 | 0.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 4 | 26 | 0 | 0.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 5 | 26 | 0 | 0 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 6 | 26 | 0 | 0 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 7 | 26 | 0 | 0.5 | 3 | 0 | 23 | 2 | 3 | 28+TT | 20-TT |
| 8 | 26 | 0 | 0.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 9 | 26 | 0 | 0.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 10 | 26 | 0 | 0 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 11 | 26 | 0 | 0 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 12 | 26 | 0 | 0.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 13 | 26 | 0 | 0.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 14 | 26 | 0 | 0 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 15 | 26 | 0 | 0.5 | 2 | 0 | 24 | 2 | 3 | 28+TT | 21-TT |
| 16 | 26 | 0 | 0.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 17 | 26 | 0 | 0.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 18 | 26 | 0 | 0 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 19 | 26 | 0 | 0 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 20 | 26 | 0 | 0.5 | 2 | 0 | 24 | 2 | 3 | 28+TT | 21-TT |
| 21 | 26 | 0 | 1 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 22 | 26 | 0 | 0 | 3 | 0 | 23 | 2 | 3 | 28+TT | 20-TT |
| 23 | 26 | 0 | 1 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 24 | 26 | 0 | 1 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 25 | 26 | 0 | 0 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 26 | 26 | 0 | 0 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 27 | 26 | 0 | 1 | 3 | 0 | 23 | 2 | 3 | 28+TT | 20-TT |
| 28 | 26 | 0 | 1 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 29 | 26 | 0 | 1 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 30 | 26 | 0 | 0 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 31 | 26 | 0 | 0 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 32 | 26 | 0 | 1 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 33 | 26 | 0 | 1 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 34 | 26 | 0 | 0 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 35 | 26 | 0 | 1 | 2 | 0 | 24 | 2 | 3 | 28+TT | 21-TT |
| 36 | 26 | 0 | 1 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 37 | 26 | 0 | 1 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 38 | 26 | 0 | 0 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 39 | 26 | 0 | 0 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 40 | 26 | 0 | 1 | 2 | 0 | 24 | 2 | 3 | 28+TT | 21-TT |
| 41 | 26 | 0 | 2 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 42 | 26 | 0 | 1 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 43 | 26 | 0 | 2 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 44 | 26 | 0 | 2 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 45 | 26 | 0 | 1 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 46 | 26 | 0 | 1 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 47 | 26 | 0 | 2 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 48 | 26 | 0 | 2 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 49 | 26 | 0 | 2 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 50 | 26 | 0 | 1 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 51 | 26 | 0 | 1 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 52 | 26 | 0 | 2 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 53 | 26 | 0 | 2 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 54 | 26 | 0 | 1 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 55 | 26 | 0 | 2 | 2.5 | 0 | 23.5 | 2 | 3 | 28+TT | 20.5-TT |
| 56 | 26 | 0 | 2 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 57 | 26 | 0 | 2 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 58 | 26 | 0 | 1 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 59 | 26 | 0 | 1 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |

| Test ID | PPowerClass (dBm) | DPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 26 | 0 | 2 | 2 | 0 | 24 | 2 | 3 | 28+TT | 21-TT |
| 61 | 26 | 0 | 2.5 | 9 | 0 | 17 | 5 | 3 | 28+TT | 12-TT |
| 62 | 26 | 0 | 2.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 63 | 26 | 0 | 2.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 64 | 26 | 0 | 2.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 65 | 26 | 0 | 2.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 66 | 26 | 0 | 2.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 67 | 26 | 0 | 2.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 68 | 26 | 0 | 2.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 69 | 26 | 0 | 2.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 70 | 26 | 0 | 2.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 71 | 26 | 0 | 2.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 72 | 26 | 0 | 2.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 73 | 26 | 0 | 2.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 74 | 26 | 0 | 2.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 75 | 26 | 0 | 2.5 | 2.5 | 0 | 23.5 | 2 | 3 | 28+TT | 20.5-TT |
| 76 | 26 | 0 | 2.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 77 | 26 | 0 | 2.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 78 | 26 | 0 | 2.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 79 | 26 | 0 | 2.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 80 | 26 | 0 | 4.5 | 9.5 | 0 | 16.5 | 5 | 3 | 28+TT | 11.5-TT |
| 81 | 26 | 0 | 4.5 | 15 | 0 | 11 | 6 | 3 | 28+TT | 5-TT |
| 82 | 26 | 0 | 4.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 83 | 26 | 0 | 4.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 84 | 26 | 0 | 4.5 | 15 | 0 | 11 | 6 | 3 | 28+TT | 5-TT |
| 85 | 26 | 0 | 4.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 86 | 26 | 0 | 4.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 87 | 26 | 0 | 4.5 | 15 | 0 | 11 | 6 | 3 | 28+TT | 5-TT |
| 88 | 26 | 0 | 4.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 89 | 26 | 0 | 4.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 90 | 26 | 0 | 4.5 | 15 | 0 | 11 | 6 | 3 | 28+TT | 5-TT |
| 91 | 26 | 0 | 4.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 92 | 26 | 0 | 4.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 93 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 94 | 26 | 0 | 1.5 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 95 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 96 | 26 | 0 | 3 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 97 | 26 | 0 | 1.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 98 | 26 | 0 | 1.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 99 | 26 | 0 | 3 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 100 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 101 | 26 | 0 | 3 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 102 | 26 | 0 | 1.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 103 | 26 | 0 | 1.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 104 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 105 | 26 | 0 | 3 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 106 | 26 | 0 | 1.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 107 | 26 | 0 | 3 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 108 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 109 | 26 | 0 | 3 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 110 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 111 | 26 | 0 | 3 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 112 | 26 | 0 | 3 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 113 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 114 | 26 | 0 | 2 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 115 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 116 | 26 | 0 | 3 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 117 | 26 | 0 | 2 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 118 | 26 | 0 | 2 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 119 | 26 | 0 | 3 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 120 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 121 | 26 | 0 | 3 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 122 | 26 | 0 | 2 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| Test ID | PPowerClass (dBm) | DPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 123 | 26 | 0 | 2 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 124 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 125 | 26 | 0 | 3 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 126 | 26 | 0 | 2 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 127 | 26 | 0 | 3 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 128 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 129 | 26 | 0 | 3 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 130 | 26 | 0 | 2 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 131 | 26 | 0 | 2 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 132 | 26 | 0 | 3 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 133 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 134 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 135 | 26 | 0 | 3.5 | 14 | 0 | 12 | 6 | 3 | 28+TT | 6-TT |
| 136 | 26 | 0 | 3.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 137 | 26 | 0 | 3.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 138 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 139 | 26 | 0 | 3.5 | 14 | 0 | 12 | 6 | 3 | 28+TT | 6-TT |
| 140 | 26 | 0 | 3.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 141 | 26 | 0 | 3.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 142 | 26 | 0 | 3.5 | 14 | 0 | 12 | 6 | 3 | 28+TT | 6-TT |
| 143 | 26 | 0 | 3.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 144 | 26 | 0 | 3.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 145 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 146 | 26 | 0 | 3.5 | 14 | 0 | 12 | 6 | 3 | 28+TT | 6-TT |
| 147 | 26 | 0 | 3.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 148 | 26 | 0 | 3.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 149 | 26 | 0 | 3.5 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 150 | 26 | 0 | 6.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 151 | 26 | 0 | 6.5 | 16 | 0 | 10 | 6 | 3 | 28+TT | 4-TT |
| 152 | 26 | 0 | 6.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 153 | 26 | 0 | 6.5 | 16 | 0 | 10 | 6 | 3 | 28+TT | 4-TT |
| 154 | 26 | 0 | 6.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 155 | 26 | 0 | 6.5 | 16 | 0 | 10 | 6 | 3 | 28+TT | 4-TT |
| 156 | 26 | 0 | 6.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 157 | 26 | 0 | 6.5 | 16 | 0 | 10 | 6 | 3 | 28+TT | 4-TT |
| 158 | 26 | 0 | 6.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-7: UE Power Class 3 test requirements (NS_05U) for bands n1, n65

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 2 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 3 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 6 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 7 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 9 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 10 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 11 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 12 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 13 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 16 | 23 | 0 | 1.0 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 17 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 18 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 20 | 23 | 0 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 21 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 22 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 23 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 24 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 25 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 26 | 23 | 0 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 27 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 28 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 30 | 23 | 0 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 31 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 32 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 33 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 34 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 35 | 23 | 0 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 36 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 37 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 38 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 39 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 40 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 41 | 23 | 0 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 42 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 43 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 44 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 45 | 23 | 0 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 46 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 47 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 48 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 49 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 50 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 51 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 52 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 53 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 54 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 55 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 56 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 57 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 58 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 59 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 61 | 23 | 0 | 4.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 62 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 63 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 64 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 65 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 66 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 67 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 68 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 69 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 70 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 71 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 72 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 73 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 74 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 75 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 76 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 77 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 78 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 79 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 80 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 81 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 82 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 83 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 84 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 85 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 86 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 87 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 88 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 89 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 90 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 91 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 92 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 93 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 94 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 95 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 96 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 97 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 98 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 99 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 100 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 101 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 102 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 103 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 104 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 105 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 106 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 107 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 108 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 109 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 110 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 111 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 112 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 113 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 114 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 115 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 116 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 117 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 118 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 119 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 120 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 121 | 23 | 0 | 6.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 122 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 123 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 124 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 125 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 126 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 127 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 128 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 129 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 130 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 131 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-8: UE Power Class 3 test requirements (NS_18)

| Test ID | ChBw (MHz) | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 3, 5 | 23 | 1 | 2 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
|  | 10, 15, 20 | 23 | 1 | 5 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
| 3, 4 | 3, 5 | 23 | 2 | 3 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
|  | 10, 15, 20 | 23 | 2 | 6 | 0 | 17 | 5 | 2.5 | 25+TT | 12-TT |
| 5, 6 | 3, 5 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 2.5 | 25+TT | 15.5-TT |
|  | 10, 15, 20 | 23 | 2.5 | 7 | 0 | 16 | 5 | 2.5 | 25+TT | 11-TT |
| 7, 8 | 3, 5 | 23 | 4.5 | 6 | 0 | 17 | 5 | 2.5 | 25+TT | 12-TT |
|  | 10, 15, 20 | 23 | 4.5 | 9 | 0 | 14 | 5 | 2.5 | 25+TT | 9-TT |
| 9, 10 | 3, 5 | 23 | 3 | 5 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
|  | 10, 15, 20 | 23 | 3 | 6.5 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 11, 12 | 3, 5 | 23 | 3 | 5 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
|  | 10, 15, 20 | 23 | 3 | 7 | 0 | 16 | 5 | 2.5 | 25+TT | 11-TT |
| 13, 14 | 3, 5 | 23 | 3.5 | 5.5 | 0 | 17.5 | 5 | 2.5 | 25+TT | 12.5-TT |
|  | 10, 15, 20 | 23 | 3.5 | 8.5 | 0 | 14.5 | 5 | 2.5 | 25+TT | 9.5-TT |
| 15, 16 | 3, 5 | 23 | 6.5 | 8.5 | 0 | 14.5 | 5 | 2.5 | 25+TT | 9.5-TT |
|  | 10, 15, 20 | 23 | 6.5 | 11.5 | 0 | 11.5 | 6 | 2.5 | 25+TT | 5.5-TT |
| 17, 18, 19 | 30 | 23 | 5 | 3 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
| 20 | 30 | 23 | 5 | 8 | 0 | 15 | 5 | 2.5 | 25+TT | 10–TT |
| 21 | 30 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 22 | 30 | 23 | 3 | 4.5 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 23 | 30 | 23 | 5 | 5.5 | 0 | 17.5 | 5 | 2.5 | 25+TT | 12.5-TT |
| 24, 25, 26 | 30 | 23 | 7 | 4.5 | 0 | 16 | 5 | 2.5 | 25+TT | 11–TT |
| 27 | 30 | 23 | 7 | 9.5 | 0 | 13.5 | 5 | 2.5 | 25+TT | 8.5–TT |
| 28 | 30 | 23 | 3.5 | 5 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
| 29 | 30 | 23 | 4 | 5.5 | 0 | 17.5 | 5 | 2.5 | 25+TT | 12.5-TT |
| 30 | 30 | 23 | 7 | 7.5 | 0 | 15.5 | 5 | 2.5 | 25+TT | 10.5-TT |

Table 6.2.3.5-10: UE Power Class 3 test requirements (NS_43)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 2 | 23 | 0.5 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 3 | 23 | 0.5 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 4 | 23 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 5 | 23 | 1.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 6 | 23 | 1.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 7 | 23 | 1.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 8 | 23 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 9 | 23 | 2.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 10 | 23 | 2.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 11 | 23 | 2.5 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 12 | 23 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 13 | 23 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 14 | 23 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 15 | 23 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 16 | 23 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 17 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 19 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 20 | 23 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 21 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 23 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 24 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 25 | 23 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 26 | 23 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 27 | 23 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 28 | 23 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-10a: UE Power Class 2 test requirements (NS_43)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0.5 | 1 | 0 | 25 | 2 | 3 | 28+TT | 22-TT |
| 2 | 26 | 0.5 | 1.5 | 0 | 24.5 | 2 | 3 | 28+TT | 21.5-TT |
| 3 | 26 | 0.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 4 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 5 | 26 | 1 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 6 | 26 | 1 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 7 | 26 | 1 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 8 | 26 | 1 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 9 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 10 | 26 | 2 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 11 | 26 | 2 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 12 | 26 | 2 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 13 | 26 | 2 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 14 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 15 | 26 | 2.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 16 | 26 | 2.5 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 17 | 26 | 2.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 18 | 26 | 2.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 19 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 20 | 26 | 4.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 21 | 26 | 4.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 22 | 26 | 3 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 23 | 26 | 3 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 24 | 26 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 25 | 26 | 3 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 26 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 27 | 26 | 3 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 28 | 26 | 3 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 29 | 26 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 30 | 26 | 3 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 31 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 32 | 26 | 3.5 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 33 | 26 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 34 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 35 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 36 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 37 | 26 | 6.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 38 | 26 | 6.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-11: UE Power Class 3 test requirements (NS_43U)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 2 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3 | 23 | 0.5 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 4 | 23 | 0.5 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 5 | 23 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 6 | 23 | 1.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 7 | 23 | 1.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 8 | 23 | 1.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 9 | 23 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 10 | 23 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 11 | 23 | 2.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 12 | 23 | 2.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 13 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 14 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 15 | 23 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 16 | 23 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 17 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 18 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 19 | 23 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 20 | 23 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 21 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 23 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 24 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 25 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 26 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 27 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 28 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 29 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 30 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 31 | 23 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 32 | 23 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 33 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 34 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 35 | 23 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 36 | 23 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-12: UE Power Class 3 test requirements (NS_100) for band n1, n5, n18, n65, n66

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 4, 5 | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 6 | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7, 8 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 9 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 10, 11 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 12 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 13, 14 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 15 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 16, 17 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19, 20 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 21 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22, 23 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 24 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 25, 26 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 27 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-13: UE Power Class 3 test requirements (NS_100) for band n2, n3, n8, n25 and n26

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 0.5 | 2 | 1.5 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 3 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 4, 5 | 23 | 1 | 2 | 1.5 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 6 | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7, 8 | 23 | 2 | 2.5 | 1.5 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 9 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 10, 11 | 23 | 2.5 | 3 | 1.5 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 12 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 13, 14 | 23 | 4.5 | 4.5 | 1.5 | 17 | 5 | 2 | 25+TT | 12-TT |
| 15 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 16, 17 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 18 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19, 20 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 21 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22, 23 | 23 | 3.5 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 24 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 25, 26 | 23 | 6.5 | 6.5 | 1.5 | 15 | 5 | 2 | 25+TT | 10-TT |
| 27 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-13a: UE Power Class 2 test requirements (NS_100) for band n8

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 26 | 3.5 | 2 | 1.5 | 21 | 2 | 3 | 28+TT | 18 |
| 3 | 26 | 0.5 | 2 | 0 | 24 | 2 | 3 | 28+TT | 21 |
| 4, 5 | 26 | 3.5 | 2 | 1.5 | 21 | 2 | 3 | 28+TT | 18 |
| 6 | 26 | 1 | 2 | 0 | 24 | 2 | 3 | 28+TT | 21 |
| 7, 8 | 26 | 3.5 | 2.5 | 1.5 | 21 | 2 | 3 | 28+TT | 18 |
| 9 | 26 | 2 | 2.5 | 0 | 23.5 | 2 | 3 | 28+TT | 20.5 |
| 10, 11 | 26 | 3.5 | 3 | 1.5 | 21 | 2 | 3 | 28+TT | 18 |
| 12 | 26 | 2.5 | 3 | 0 | 23 | 2 | 3 | 28+TT | 20 |
| 13, 14 | 26 | 4.5 | 4.5 | 1.5 | 20 | 2.5 | 3 | 28+TT | 17 |
| 15 | 26 | 4.5 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5 |
| 16, 17 | 26 | 3.5 | 4 | 1.5 | 20.5 | 2.5 | 3 | 28+TT | 17.5 |
| 18 | 26 | 1 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19 |
| 19, 20 | 26 | 3.5 | 4 | 1.5 | 20.5 | 2.5 | 3 | 28+TT | 17.5 |
| 21 | 26 | 3 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19 |
| 22, 23 | 26 | 3.5 | 4 | 1.5 | 20.5 | 2.5 | 3 | 28+TT | 17.5 |
| 24 | 26 | 3.5 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19 |
| 25, 26 | 26 | 6.5 | 6.5 | 1.5 | 18 | 4 | 3 | 28+TT | 14 |
| 27 | 26 | 6.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-14: UE Power Class 3 test requirements (NS_37)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 1.0 | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 2 | 23 | 0 | 0.5 | 1.0 | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 3 | 23 | 0 | 0.5 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 4 | 23 | 0 | 0.5 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 5 | 23 | 0 | 1.0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 6 | 23 | 0 | 1.0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 7 | 23 | 0 | 1.0 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 8 | 23 | 0 | 1.0 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 9 | 23 | 0 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 10 | 23 | 0 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 11 | 23 | 0 | 2.0 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 12 | 23 | 0 | 2.0 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 13 | 23 | 0 | 2.5 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 14 | 23 | 0 | 2.5 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 15 | 23 | 0 | 2.5 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 16 | 23 | 0 | 2.5 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 17 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 18 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 19 | 23 | 0 | 3.0 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 20 | 23 | 0 | 3.0 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 21 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 22 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 23 | 23 | 0 | 3.0 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 24 | 23 | 0 | 3.0 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-15: UE Power Class 3 test requirements (NS_38)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 2 | 23 | 0 | 0.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 3 | 23 | 0 | 0.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 4 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 5 | 23 | 0 | 0.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 6 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 7 | 23 | 0 | 1.0 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 8 | 23 | 0 | 1.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 9 | 23 | 0 | 1.0 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 10 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 11 | 23 | 0 | 1.0 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 12 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 13 | 23 | 0 | 2.0 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 14 | 23 | 0 | 2.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 15 | 23 | 0 | 2.0 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 16 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 17 | 23 | 0 | 2.0 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 18 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 19 | 23 | 0 | 2.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 20 | 23 | 0 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 21 | 23 | 0 | 2.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 22 | 23 | 0 | 2.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 23 | 23 | 0 | 2.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 24 | 23 | 0 | 2.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 25 | 23 | 0 | 4.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 26 | 23 | 0 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 27 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 28 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 29 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 30 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 31 | 23 | 0 | 3.0 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 32 | 23 | 0 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 33 | 23 | 0 | 3.0 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 34 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 35 | 23 | 0 | 3.0 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 36 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 37 | 23 | 0 | 3.0 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 38 | 23 | 0 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 39 | 23 | 0 | 3.0 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 40 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 41 | 23 | 0 | 3.0 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 42 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 43 | 23 | 0 | 3.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 44 | 23 | 0 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 45 | 23 | 0 | 3.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46 | 23 | 0 | 3.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 47 | 23 | 0 | 3.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 48 | 23 | 0 | 3.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 49 | 23 | 0 | 6.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 50 | 23 | 0 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 51 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 52 | 23 | 0 | 6.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 53 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 54 | 23 | 0 | 6.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-16: UE Power Class 3 test requirements (NS_39)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 2 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 3 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 5 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 6 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 7 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 9 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 10 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 11 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 12 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 13 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 15 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 16 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 17 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 18 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 20 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 21 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 22 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 23 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 24 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 25 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 26 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 27 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-17: UE Power Class 3 test requirements (NS_24) for n65

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-3 | 23 | N/A | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 4-6 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 7-15 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 16-TT |
| 16-18 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 19-21 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 22-30 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 31-39 | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
|  | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 40-42 | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
| 43-51 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 52-60 | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
|  | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 61-63 | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
| 64-72 | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
|  | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 73-75 | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
| 76-84 | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
|  | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 85-87 | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
| 88 | 23 | N/A | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 89 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 90-92 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 16-TT |
| 93 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 94 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 95-97 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 98-100 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 101 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 103-105 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 106-108 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 109 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 110-112 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 113 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 114-116 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 117 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 118 | 23 | 4.5 |  | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 119 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 120-122 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | 4.5 |  | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 123 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 124 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 125-127 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | 4.5 |  | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 128-130 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 131 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 133-135 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | 4.5 |  | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 136-138 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 139 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 140-142 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 143 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 144-146 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 147 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 148-150 | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 151-153 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 154-162 | 23 | N/A | 13 | 0 | 10 | 6 | 2 | 25+TT | 4-TT |
|  | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 16-TT |
| 163-165 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 166-168 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 169-177 | 23 | N/A | 13 | 0 | 10 | 6 | 2 | 25+TT | 4-TT |
|  | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 178-186 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 187-189 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 190-198 | 23 | N/A | 13 | 0 | 10 | 6 | 2 | 25+TT | 4-TT |
|  | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 199-207 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 208-210 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 211-219 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 220-222 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 223-231 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 232-234 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 235 | 23 | 6.5 |  | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 236 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 237-239 | 23 | N/A | 13 | 0 | 10 | 6 | 2 | 25+TT | 4-TT |
|  | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | 6.5 |  | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 240 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 241 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 242-244 | 23 | N/A | 13 | 0 | 10 | 6 | 2 | 25+TT | 4-TT |
|  | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | 6.5 |  | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 245-247 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 248 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 250-252 | 23 | N/A | 13 | 0 | 10 | 6 | 2 | 25+TT | 4-TT |
|  | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | 6.5 |  | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 253-255 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 256 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 257-259 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 260 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 261-263 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 264 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 265 | 23 | 1 |  | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 266 | 23 | 2 |  | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 267 | 23 | 1 |  | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-18: UE Power Class 3 test requirements (NS_27)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 2 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 3 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 4 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 5 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 6 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 7 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 8 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 9 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 10 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 11 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 12 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 13 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 14 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 15 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 16 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 17 | 23 | 0 | 0.5 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 18 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 19 | 23 | 0 | 0.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 20 | 23 | 0 | 0.5 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 21 | 23 | 0 | 0.5 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 22 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 23 | 23 | 0 | 0.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 24 | 23 | 0 | 0.5 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 25 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 26 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 27 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 28 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 29 | 23 | 0 | 0.5 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 30 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 31 | 23 | 0 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 32 | 23 | 0 | 0.5 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 33 | 23 | 0 | 0.5 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 34 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 35 | 23 | 0 | 0.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 36 | 23 | 0 | 0.5 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 37 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 38 | 23 | 0 | 0.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 39 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 40 | 23 | 0 | 0.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 41 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 42 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 43 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 44 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 45 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 46 | 23 | 0 | 1 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 47 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 48 | 23 | 0 | 1 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 49 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 50 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 51 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 52 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 53 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 54 | 23 | 0 | 1 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 55 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 56 | 23 | 0 | 1 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 57 | 23 | 0 | 1 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 58 | 23 | 0 | 1 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 59 | 23 | 0 | 1 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 60 | 23 | 0 | 1 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 61 | 23 | 0 | 1 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 62 | 23 | 0 | 1 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 63 | 23 | 0 | 1 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 64 | 23 | 0 | 1 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 65 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 66 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 67 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 68 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 69 | 23 | 0 | 1 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 70 | 23 | 0 | 1 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 71 | 23 | 0 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 72 | 23 | 0 | 1 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 73 | 23 | 0 | 1 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 74 | 23 | 0 | 1 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 75 | 23 | 0 | 1 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 76 | 23 | 0 | 1 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 77 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 78 | 23 | 0 | 1 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 79 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 80 | 23 | 0 | 1 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 81 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 82 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 83 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 84 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 85 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 86 | 23 | 0 | 2 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 87 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 88 | 23 | 0 | 2 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 89 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 90 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 91 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 92 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 93 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 94 | 23 | 0 | 2 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 95 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 96 | 23 | 0 | 2 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 97 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 98 | 23 | 0 | 2 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 99 | 23 | 0 | 2 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 100 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 101 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 102 | 23 | 0 | 2 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 103 | 23 | 0 | 2 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 104 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 105 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 106 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 107 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 108 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 109 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 110 | 23 | 0 | 2 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 111 | 23 | 0 | 1 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 112 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 113 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 114 | 23 | 0 | 2 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 115 | 23 | 0 | 2 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 116 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 117 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 118 | 23 | 0 | 2 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 119 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 120 | 23 | 0 | 2 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 121 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 122 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 123 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 124 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 125 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 126 | 23 | 0 | 2.5 | 2 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 127 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 128 | 23 | 0 | 2.5 | 2 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 129 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 130 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 131 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 132 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 133 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 134 | 23 | 0 | 2.5 | 2 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 135 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 136 | 23 | 0 | 2.5 | 2 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 137 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 138 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 139 | 23 | 0 | 2.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 140 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 141 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 142 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 143 | 23 | 0 | 2.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 144 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 145 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 146 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 147 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 148 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 149 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 150 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 151 | 23 | 0 | 2.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 152 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 153 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 154 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 155 | 23 | 0 | 2.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 156 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 157 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 158 | 23 | 0 | 2.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 159 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 160 | 23 | 0 | 2.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 161 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 162 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 163 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 164 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 165 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 166 | 23 | 0 | 4.5 | 2 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 167 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 168 | 23 | 0 | 4.5 | 2 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 169 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 170 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 171 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 172 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 173 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 174 | 23 | 0 | 4.5 | 2 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 175 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 176 | 23 | 0 | 4.5 | 2 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 177 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 178 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 179 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 180 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 181 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 182 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 183 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 184 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 185 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 186 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 187 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 188 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 189 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 190 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 191 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 192 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 193 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 194 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 195 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 196 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 197 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 198 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 199 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 200 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 201 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 202 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 203 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 204 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 205 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 206 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 207 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 208 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 209 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 210 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 211 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 212 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 213 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 214 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 215 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 216 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 217 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 218 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 219 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 220 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 221 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 222 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 223 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 224 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 225 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 226 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 227 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 228 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 229 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 230 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 231 | 23 | 0 | 1.5 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 232 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 233 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 234 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 235 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 236 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 237 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 238 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 239 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 240 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 241 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 242 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 243 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 244 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 245 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 246 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 247 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 248 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 249 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 250 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 251 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 252 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 253 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 254 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 255 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 256 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 257 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 258 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 259 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 260 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 261 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 262 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 263 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 264 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 265 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 266 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 267 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 268 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 269 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 270 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 271 | 23 | 0 | 2 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 272 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 273 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 274 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 275 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 276 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 277 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 278 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 279 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 280 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 281 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 282 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 283 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 284 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 285 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 286 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 287 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 288 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 289 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 290 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 291 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 292 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 293 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 294 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 295 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 296 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 297 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 298 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 299 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 300 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 301 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 302 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 303 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 304 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 305 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 306 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 307 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 308 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 309 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 310 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 311 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 312 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 313 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 314 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 315 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 316 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 317 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 318 | 23 | 0 | 3.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 319 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 320 | 23 | 0 | 3.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 321 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 322 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 323 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 324 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 325 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 326 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 327 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 328 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 329 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 330 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 331 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 332 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 333 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 334 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 335 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 336 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 337 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 338 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 339 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 340 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 341 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 342 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 343 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 344 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 345 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 346 | 23 | 0 | 6.5 | 4 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 347 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 348 | 23 | 0 | 6.5 | 4 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 349 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 350 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 351 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 352 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 353 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 354 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 355 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 356 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 357 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 358 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 359 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 360 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-19: UE Power Class 3 test requirements (NS_40)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1 | 15.5 | 0 | 7.5 | 7.0 | 2 | 25+TT | 0.5-TT |
| 2 | 23 | 0 | 0 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 3 | 23 | 0 | 1 | 15.5 | 0 | 7.5 | 7.0 | 2 | 25+TT | 0.5-TT |
| 4 | 23 | 0 | 2 | 14.5 | 0 | 8.5 | 6.0 | 2 | 25+TT | 2.5-TT |
| 5 | 23 | 0 | 1 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 6 | 23 | 0 | 2 | 14.5 | 0 | 8.5 | 6.0 | 2 | 25+TT | 2.5-TT |
| 7 | 23 | 0 | 2.5 | 14.5 | 0 | 8.5 | 6.0 | 2 | 25+TT | 2.5-TT |
| 8 | 23 | 0 | 2.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 9 | 23 | 0 | 2.5 | 14.5 | 0 | 8.5 | 6.0 | 2 | 25+TT | 2.5-TT |
| 10 | 23 | 0 | 4.5 | 12.5 | 0 | 10.5 | 6.0 | 2 | 25+TT | 4.5-TT |
| 11 | 23 | 0 | 4.5 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 12 | 23 | 0 | 4.5 | 12.5 | 0 | 10.5 | 6.0 | 2 | 25+TT | 4.5-TT |
| 13 | 23 | 0 | 3 | 14.5 | 0 | 8.5 | 6.0 | 2 | 25+TT | 2.5-TT |
| 14 | 23 | 0 | 1.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 15 | 23 | 0 | 3 | 14.5 | 0 | 8.5 | 6.0 | 2 | 25+TT | 2.5-TT |
| 16 | 23 | 0 | 3 | 14.5 | 0 | 8.5 | 6.0 | 2 | 25+TT | 2.5-TT |
| 17 | 23 | 0 | 2 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 18 | 23 | 0 | 3 | 14.5 | 0 | 8.5 | 6.0 | 2 | 25+TT | 2.5-TT |
| 19 | 23 | 0 | 3.5 | 14 | 0 | 9 | 6.0 | 2 | 25+TT | 3-TT |
| 20 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 21 | 23 | 0 | 3.5 | 14 | 0 | 9 | 6.0 | 2 | 25+TT | 3-TT |
| 22 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 23 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 24 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-20: UE Power Class 3 test requirements (NS_41)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 2 | 23 | 0 | 0.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 3 | 23 | 0 | 0.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 4 | 23 | 0 | 0.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 5 | 23 | 0 | 0.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 6 | 23 | 0 | 0.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 7 | 23 | 0 | 0.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 8 | 23 | 0 | 0.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 9 | 23 | 0 | 1 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 10 | 23 | 0 | 1 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 11 | 23 | 0 | 1 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 12 | 23 | 0 | 1 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 13 | 23 | 0 | 1 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 14 | 23 | 0 | 1 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 15 | 23 | 0 | 1 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 16 | 23 | 0 | 1 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 17 | 23 | 0 | 2 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 18 | 23 | 0 | 2 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 19 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 20 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 21 | 23 | 0 | 2 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 22 | 23 | 0 | 2 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 23 | 23 | 0 | 2 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 24 | 23 | 0 | 2 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 25 | 23 | 0 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 26 | 23 | 0 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 27 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 28 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 29 | 23 | 0 | 2.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 30 | 23 | 0 | 2.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 31 | 23 | 0 | 2.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 32 | 23 | 0 | 2.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 33 | 23 | 0 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 34 | 23 | 0 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 35 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 36 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 37 | 23 | 0 | 4.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 38 | 23 | 0 | 4.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 39 | 23 | 0 | 4.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 40 | 23 | 0 | 4.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 41 | 23 | 0 | 3 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 42 | 23 | 0 | 3 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 43 | 23 | 0 | 3 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 44 | 23 | 0 | 3 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 45 | 23 | 0 | 3 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 46 | 23 | 0 | 3 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 47 | 23 | 0 | 3 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 48 | 23 | 0 | 3 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 49 | 23 | 0 | 3 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 50 | 23 | 0 | 3 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 51 | 23 | 0 | 3 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 52 | 23 | 0 | 3 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 53 | 23 | 0 | 3 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 54 | 23 | 0 | 3 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 55 | 23 | 0 | 3 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 56 | 23 | 0 | 3 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 57 | 23 | 0 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 58 | 23 | 0 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 59 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 61 | 23 | 0 | 3.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 62 | 23 | 0 | 3.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 63 | 23 | 0 | 3.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 64 | 23 | 0 | 3.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 65 | 23 | 0 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 66 | 23 | 0 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 67 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 68 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 69 | 23 | 0 | 6.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 70 | 23 | 0 | 6.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 71 | 23 | 0 | 6.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 72 | 23 | 0 | 6.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-21: UE Power Class 3 test requirements (NS_42)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 2 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 3 | 23 | 0 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 4 | 23 | 0 | 0.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 5 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 6 | 23 | 0 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 7 | 23 | 0 | 1 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 8 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 9 | 23 | 0 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 10 | 23 | 0 | 1 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 11 | 23 | 0 | 1 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 12 | 23 | 0 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 13 | 23 | 0 | 2 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 14 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 15 | 23 | 0 | 1 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 16 | 23 | 0 | 2 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 17 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 18 | 23 | 0 | 1 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 19 | 23 | 0 | 2.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 20 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 21 | 23 | 0 | 2.5 | 1.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 22 | 23 | 0 | 2.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 23 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 24 | 23 | 0 | 2.5 | 1.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 25 | 23 | 0 | 4.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 26 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 27 | 23 | 0 | 4.5 | 1.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 28 | 23 | 0 | 4.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 29 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 30 | 23 | 0 | 4.5 | 1.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 31 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 32 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 33 | 23 | 0 | 1.5 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 34 | 23 | 0 | 3 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 35 | 23 | 0 | 3 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 36 | 23 | 0 | 1.5 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 37 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 38 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 39 | 23 | 0 | 2 | 1.5 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 40 | 23 | 0 | 3 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 41 | 23 | 0 | 3 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 42 | 23 | 0 | 2 | 1.5 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 43 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 44 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 45 | 23 | 0 | 3.5 | 1.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 46 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 47 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 48 | 23 | 0 | 3.5 | 1.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 49 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 50 | 23 | 0 | 6.5 | 4 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 51 | 23 | 0 | 6.5 | 1.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 52 | 23 | 0 | 6.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 53 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 54 | 23 | 0 | 6.5 | 1.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-22: UE Power Class 3 test requirements for NS_47 (contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 3.5 | 10 | 0 | 16 | 5 | 2 | 28+TT | 11-TT |
| 2 | 23 | -3 | 0.2 | 8.5 | 0 | 17.5 | 5 | 2 | 28+TT | 12.5-TT |
| 3 | 23 | -3 | 3.5 | 5 | 0 | 21 | 2 | 2 | 28+TT | 19-TT |
| 4 | 23 | -3 | 1.2 | 8.5 | 0 | 17.5 | 5 | 2 | 28+TT | 12.5-TT |
| 5 | 23 | -3 | 1.2 | 6 | 0 | 20 | 2.5 | 2 | 28+TT | 17.5-TT |
| 6 | 23 | -3 | 1.2 | 6 | 0 | 20 | 2.5 | 2 | 28+TT | 17.5-TT |
| 7 | 23 | -3 | 1.2 | 8.5 | 0 | 17.5 | 5 | 2 | 28+TT | 12.5-TT |
| 8 | 23 | 0 | 0.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 9 | 23 | 0 | 0.0 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 10 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 11 | 23 | 0 | 0.5 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 12 | 23 | 0 | 0.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 13 | 23 | 0 | 0.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 14 | 23 | 0 | 0.5 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 15 | 23 | 0 | 1.0 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 16 | 23 | 0 | 0.0 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 17 | 23 | 0 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 18 | 23 | 0 | 1.0 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 19 | 23 | 0 | 1.0 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 20 | 23 | 0 | 1.0 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 21 | 23 | 0 | 1.0 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 22 | 23 | 0 | 2.0 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 23 | 23 | 0 | 1.0 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 24 | 23 | 0 | 2.0 | 0 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 25 | 23 | 0 | 2.0 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 26 | 23 | 0 | 2.0 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 27 | 23 | 0 | 2.0 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 28 | 23 | 0 | 2.0 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 29 | 23 | 0 | 2.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 30 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 31 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 32 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 33 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 34 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 35 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 36 | 23 | 0 | 4.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 37 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 38 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 39 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 40 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 41 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 42 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 43 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 44 | 23 | 0 | 1.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 45 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 46 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 47 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 48 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 49 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 50 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 51 | 23 | 0 | 2.0 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 52 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 53 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 54 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 55 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 56 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 57 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 58 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 59 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 61 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 62 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 63 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 64 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 65 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 66 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 67 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 68 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 69 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 70 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-22a: UE Power Class 3 test requirements for NS_47 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | MPR & A-MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 7 | 1.0 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 2 | 23 | 0 | 3.0 | 7 | 1.0 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 3 | 23 | 0 | 3.5 | 7 | 1.0 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 4 | 23 | 0 | 6.5 | 7 | 1.0 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-23: UE Power Class 2 test requirements for NS_47 (contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 2 | 26 | 0 | 0.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 3 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 4 | 26 | 0 | 0.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 5 | 26 | 0 | 0.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 6 | 26 | 0 | 0.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 7 | 26 | 0 | 0.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 8 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 9 | 26 | 0 | 0.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 10 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 11 | 26 | 0 | 1.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 12 | 26 | 0 | 1.0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 13 | 26 | 0 | 1.0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 14 | 26 | 0 | 1.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 15 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 16 | 26 | 0 | 1.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 17 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 18 | 26 | 0 | 2.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 19 | 26 | 0 | 2.0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 20 | 26 | 0 | 2.0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 21 | 26 | 0 | 2.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 22 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 23 | 26 | 0 | 2.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 24 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 25 | 26 | 0 | 2.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 26 | 26 | 0 | 2.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 27 | 26 | 0 | 2.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 28 | 26 | 0 | 2.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 29 | 26 | 0 | 4.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 30 | 26 | 0 | 4.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 31 | 26 | 0 | 4.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 32 | 26 | 0 | 4.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 33 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 34 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 35 | 26 | 0 | 4.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 36 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 37 | 26 | 0 | 1.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 38 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 39 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 40 | 26 | 0 | 3 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 41 | 26 | 0 | 3 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 42 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 43 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 44 | 26 | 0 | 2.0 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 45 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 46 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 47 | 26 | 0 | 3 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 48 | 26 | 0 | 3 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 49 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 50 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 51 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 52 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 53 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 54 | 26 | 0 | 3.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 55 | 26 | 0 | 3.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 56 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 57 | 26 | 0 | 6.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 58 | 26 | 0 | 6.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 59 | 26 | 0 | 6.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 26 | 0 | 6.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 61 | 26 | 0 | 6.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 62 | 26 | 0 | 6.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 63 | 26 | 0 | 6.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-23a: UE Power Class 2 test requirements for NS_47 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 3.0 | 10 | 1.0 | 0 | 16 | 5.0 | 3 | 28+TT | 11-TT |
| 2 | 26 | 0 | 3.0 | 10 | 1.0 | 0 | 16 | 5.0 | 3 | 28+TT | 11-TT |
| 3 | 26 | 0 | 3.5 | 10 | 1.0 | 0 | 16 | 5.0 | 3 | 28+TT | 11-TT |
| 4 | 26 | 0 | 6.5 | 10 | 1.0 | 0 | 16 | 5.0 | 3 | 28+TT | 11-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-24: UE Power Class 3 test requirements for NS_48 (PC3 contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 2 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 3 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 4 | 23 | 0 | 1 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 6 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 7 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 8 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 9 | 23 | 0 | 0 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 10 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 11 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 12 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 13 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 16 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 17 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 20 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 21 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 22 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 23 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 24 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 26 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 27 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 28 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 30 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 31 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 32 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 33 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 34 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 35 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 36 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 37 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 38 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 39 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 40 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 41 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 42 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 43 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 44 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 45 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 46 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 47 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 48 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 49 | 23 | 0 | 1.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 50 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 51 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 52 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 53 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 54 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 55 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 56 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 57 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 58 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 59 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 60 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 61 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 62 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 63 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 64 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 65 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 66 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 67 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 68 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 69 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 70 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 71 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 72 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 73 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 74 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 75 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 76 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 77 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 78 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 79 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 80 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-24a: UE Power Class 2 test requirements for NS_48 (PC2 contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 1 | 3 | 0 | 23 | 2 | 3 | 28+TT | 20-TT |
| 2 | 26 | 0 | 1 | 3 | 0 | 23 | 2 | 3 | 28+TT | 20-TT |
| 3 | 26 | 0 | 1 | 3 | 0 | 23 | 2 | 3 | 28+TT | 20-TT |
| 4 | 26 | 0 | 1 | 3 | 0 | 23 | 2 | 3 | 28+TT | 20-TT |
| 5 | 26 | 0 | 1 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 6 | 26 | 0 | 1 | 3 | 0 | 23 | 2 | 3 | 28+TT | 20-TT |
| 7 | 26 | 0 | 1 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 8 | 26 | 0 | 3.5 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 9 | 26 | 0 | 1 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 10 | 26 | 0 | 1 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 11 | 26 | 0 | 3.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 12 | 26 | 0 | 1 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 13 | 26 | 0 | 1 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 14 | 26 | 0 | 1 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 15 | 26 | 0 | 1 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 16 | 26 | 0 | 1 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 17 | 26 | 0 | 0 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 18 | 26 | 0 | 1 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 19 | 26 | 0 | 1 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 20 | 26 | 0 | 1 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 21 | 26 | 0 | 1 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 22 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 23 | 26 | 0 | 1 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 24 | 26 | 0 | 1 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 25 | 26 | 0 | 1 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 26 | 26 | 0 | 1 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 27 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 28 | 26 | 0 | 1 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 29 | 26 | 0 | 4.5 | 3 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 30 | 26 | 0 | 4.5 | 3 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 31 | 26 | 0 | 4.5 | 3 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 32 | 26 | 0 | 4.5 | 3 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 33 | 26 | 0 | 4.5 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 34 | 26 | 0 | 4.5 | 3 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 35 | 26 | 0 | 4.5 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 36 | 26 | 0 | 4.5 | 4 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 37 | 26 | 0 | 4.5 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 38 | 26 | 0 | 4.5 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 39 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 40 | 26 | 0 | 4.5 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 41 | 26 | 0 | 4.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 42 | 26 | 0 | 4.5 | 4 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 43 | 26 | 0 | 4.5 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 44 | 26 | 0 | 4.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 45 | 26 | 0 | 4.5 | 4 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 46 | 26 | 0 | 4.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 47 | 26 | 0 | 4.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 48 | 26 | 0 | 4.5 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 49 | 26 | 0 | 4.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 50 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 51 | 26 | 0 | 4.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 52 | 26 | 0 | 4.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 53 | 26 | 0 | 4.5 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 54 | 26 | 0 | 4.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 55 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 56 | 26 | 0 | 4.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 57 | 26 | 0 | 3 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 58 | 26 | 0 | 3 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 59 | 26 | 0 | 3 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 60 | 26 | 0 | 3 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 61 | 26 | 0 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62 | 26 | 0 | 3 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 63 | 26 | 0 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 64 | 26 | 0 | 3.5 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 65 | 26 | 0 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 66 | 26 | 0 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 67 | 26 | 0 | 3.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 68 | 26 | 0 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 69 | 26 | 0 | 3 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 70 | 26 | 0 | 3 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 71 | 26 | 0 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 72 | 26 | 0 | 3 | 9 | 0 | 17 | 5 | 3 | 28+TT | 12-TT |
| 73 | 26 | 0 | 3 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 74 | 26 | 0 | 3 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 75 | 26 | 0 | 3 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 76 | 26 | 0 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 77 | 26 | 0 | 3 | 9 | 0 | 17 | 5 | 3 | 28+TT | 12-TT |
| 78 | 26 | 0 | 1.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 79 | 26 | 0 | 3 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 80 | 26 | 0 | 3 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 81 | 26 | 0 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 82 | 26 | 0 | 3 | 9 | 0 | 17 | 5 | 3 | 28+TT | 12-TT |
| 83 | 26 | 0 | 1.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 84 | 26 | 0 | 3 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 85 | 26 | 0 | 6.5 | 4.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 86 | 26 | 0 | 6.5 | 4.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 87 | 26 | 0 | 6.5 | 4.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 88 | 26 | 0 | 6.5 | 4.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 89 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 90 | 26 | 0 | 6.5 | 4.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 91 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 92 | 26 | 0 | 6.5 | 5.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 93 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 94 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 95 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 96 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 97 | 26 | 0 | 6.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 98 | 26 | 0 | 6.5 | 5.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 99 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 100 | 26 | 0 | 6.5 | 9 | 0 | 17 | 5 | 3 | 28+TT | 12-TT |
| 101 | 26 | 0 | 6.5 | 5.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 102 | 26 | 0 | 6.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 103 | 26 | 0 | 6.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 104 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 105 | 26 | 0 | 6.5 | 9 | 0 | 17 | 5 | 3 | 28+TT | 12-TT |
| 106 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 107 | 26 | 0 | 6.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 108 | 26 | 0 | 6.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 109 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 110 | 26 | 0 | 6.5 | 9 | 0 | 17 | 5 | 3 | 28+TT | 12-TT |
| 111 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 112 | 26 | 0 | 6.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-25: UE Power Class 3 test requirements (NS_12) for n26 low range

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | N/A | 5 | 1.5 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 2 | 23 | N/A | 5.5 | 1.5 | 16 | 5 | 2 | 25+TT | 11-TT |
| 3 | 23 | N/A | 9.5 | 1.5 | 12 | 6 | 2 | 25+TT | 6-TT |
| 4 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 6 | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 7 | 23 | N/A | 5 | 1.5 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 8 | 23 | N/A | 5.5 | 1.5 | 16 | 5 | 2 | 25+TT | 11-TT |
| 9 | 23 | N/A | 9.5 | 1.5 | 12 | 6 | 2 | 25+TT | 6-TT |
| 10 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 11 | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 12 | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 13 | 23 | 1 | N/A | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 14 | 23 | 1 | N/A | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 15 | 23 | N/A | 7 | 1.5 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 16 | 23 | N/A | 9.5 | 1.5 | 12 | 6 | 2 | 25+TT | 6-TT |
| 17 | 23 | N/A | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 18 | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 19 | 23 | N/A | 7 | 1.5 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 20 | 23 | N/A | 9.5 | 1.5 | 12 | 6 | 2 | 25+TT | 6-TT |
| 21 | 23 | N/A | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 22 | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 23 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 24 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 25 | 23 | N/A | 4.5 | 1.5 | 17 | 5 | 2 | 25+TT | 12-TT |
| 26 | 23 | N/A | 5 | 1.5 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 27 | 23 | N/A | 5 | 1.5 | 16.5 | 6 | 2 | 25+TT | 10.5-TT |
| 28 | 23 | N/A | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 29 | 23 | N/A | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 30 | 23 | N/A | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 31 | 23 | 1 | N/A | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 32 | 23 | N/A | 6 | 1.5 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| 33 | 23 | N/A | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 34 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-25b: UE Power Class 3 test requirements (NS_12) for n26 high range

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 2 | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 3 | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 4 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 6 | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 7 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 8 | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 9 | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 10 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 11 | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 12 | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 13 | 23 | 1 | N/A | 1.5 | 20.5 | 2 | 2 | 25+TT | 18.5-TT |
| 14 | 23 | 1 | N/A | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 15 | 23 | N/A | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 16 | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 17 | 23 | N/A | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 18 | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 19 | 23 | N/A | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 20 | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 21 | 23 | N/A | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 22 | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 23 | 23 | 3 | 0 | 1.5 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 24 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 25 | 23 | N/A | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 26 | 23 | N/A | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 27 | 23 | N/A | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 28 | 23 | N/A | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 29 | 23 | N/A | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 30 | 23 | N/A | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 31 | 23 | 1 | N/A | 1.5 | 20.5 | 2 | 2 | 25+TT | 18.5-TT |
| 32 | 23 | N/A | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 33 | 23 | N/A | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 34 | 23 | 3 | 0 | 1.5 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-26: UE Power Class 3 test requirements (NS_13) for n26

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | N/A | 3.5 | 1.5 | 18 | 4 | 2 | 25+TT | 14-TT |
| 2 | 23 | N/A | 4.5 | 1.5 | 17 | 5 | 2 | 25+TT | 12-TT |
| 3 | 23 | N/A | 8 | 1.5 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 4 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 6 | 23 | N/A | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 7 | 23 | N/A | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 8 | 23 | N/A | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 9 | 23 | N/A | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 10 | 23 | N/A | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 11 | Void |  |  |  |  |  |  |  |  |
| 12 | 23 | N/A | 5 | 1.5 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 13 | 23 | N/A | 6 | 1.5 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| 14 | 23 | N/A | 8 | 1.5 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 15 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 16 | 23 | N/A | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 17 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 18 | 23 | N/A | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 19 | 23 | N/A | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 20 | 23 | N/A | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 21 | Void |  |  |  |  |  |  |  |  |
| 22 | 23 | N/A | 4 | 1.5 | 17.5 | 4 | 2 | 25+TT | 13.5-TT |
| 23 | 23 | N/A | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 24 | 23 | N/A | 4 | 0 | 19 | 4 | 2 | 25+TT | 15-TT |
| 25 | 23 | N/A | 4 | 0 | 19 | 4 | 2 | 25+TT | 15-TT |
| 26 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 27 | 23 | N/A | 4 | 0 | 19 | 4 | 2 | 25+TT | 15-TT |
| 28 | 23 | N/A | 5.5 | 1.5 | 16 | 5 | 2 | 25+TT | 11-TT |
| 29 | 23 | N/A | 5.5 | 1.5 | 16 | 5 | 2 | 25+TT | 11-TT |
| 30 | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 31 | 23 | N/A | 5.5 | 0 | 17.5 | 4 | 2 | 25+TT | 13.5-TT |
| 32 | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-27a: UE Power Class 3 test requirements (NS_14) for n26 low range

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | N/A | 3 | 1.5 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 2 | 23 | N/A | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3 | 23 | N/A | 3 | 1.5 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 4 | 23 | N/A | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 5 | 23 | N/A | 3 | 1.5 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 6 | 23 | N/A | 8 | 1.5 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 7 | 23 | N/A | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 8 | 23 | 1 | N/A | 1.5 | 20 | 2.5 | 2 | 25+TT | 18-TT |
| 9 | 23 | 0 | N/A | 1.5 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 10 | 23 | 0 | N/A | 0 | 23 | 2 | 2 | 25+TT | 21-TT |
| 11 | 23 | N/A | 5 | 1.5 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 12 | 23 | N/A | 6 | 1.5 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| 13 | 23 | N/A | 8 | 1.5 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 14 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 15 | 23 | N/A | 5 | 1.5 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 16 | 23 | N/A | 6 | 1.5 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| 17 | 23 | N/A | 8 | 1.5 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 18 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19 | 23 | N/A | 5 | 1.5 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 20 | 23 | N/A | 6 | 1.5 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| 21 | 23 | N/A | 8 | 1.5 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 22 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 23 | 23 | 3 | N/A | 1.5 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 24 | 23 | 1.5 | N/A | 1.5 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 25 | 23 | 1.5 | N/A | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-27b: UE Power Class 3 test requirements (NS_14) for n26 high range

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | N/A | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 2 | 23 | N/A | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3 | 23 | N/A | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 4 | 23 | N/A | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 5 | 23 | N/A | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 6 | 23 | N/A | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 7 | 23 | N/A | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 8 | 23 | 1 | N/A | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 9 | 23 | 0 | N/A | 0 | 23 | 2 | 2 | 25+TT | 21-TT |
| 10 | 23 | 0 | N/A | 0 | 23 | 2 | 2 | 25+TT | 21-TT |
| 11 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 12 | 23 | N/A | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 13 | 23 | N/A | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 14 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 15 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 16 | 23 | N/A | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 17 | 23 | N/A | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 18 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 20 | 23 | N/A | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 21 | 23 | N/A | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 22 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 23 | 23 | 3 | N/A | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 24 | 23 | 1.5 | N/A | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 25 | 23 | 1.5 | N/A | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-28: UE Power Class 3 test requirements (NS_15) for n26

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | N/A | 9 | 1.5 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 2 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 3 | 23 | N/A | 5 | 1.5 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 4 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 5 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 6 | 23 | N/A | 9 | 1.5 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 7 | 23 | N/A | 13.5 | 1.5 | 8 | 6 | 2 | 25+TT | 2-TT |
| 8 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 9 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 10 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 11 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 12 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 13 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 14 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 15 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 16 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 17 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 18 | 23 | N/A | 9 | 1.5 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 19 | 23 | N/A | 13.5 | 1.5 | 8 | 6 | 2 | 25+TT | 2-TT |
| 20 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 21 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 22 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 23 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 24 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 25 | 23 | N/A | 13.5 | 0 | 9.5 | 6 | 2 | 25+TT | 3.5-TT |
| 26 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 27 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 28 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 29 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 30 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 31 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 32 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 33 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 34 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 35 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 36 | 23 | N/A | 9 | 1.5 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 37 | 23 | N/A | 13.5 | 1.5 | 8 | 6 | 2 | 25+TT | 2-TT |
| 38 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 39 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 40 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 41 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 42 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 43 | 23 | N/A | 13.5 | 0 | 9.5 | 6 | 2 | 25+TT | 3.5-TT |
| 44 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 45 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 47 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 48 | 23 | 1 | N/A | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 49 | 23 | 1 | N/A | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 50 | 23 | 0 | N/A | 0 | 23 | 2 | 2 | 25+TT | 21-TT |
| 51 | 23 | 0 | N/A | 0 | 23 | 2 | 2 | 25+TT | 21-TT |
| 52 | 23 | N/A | 10.5 | 1.5 | 11 | 6 | 2 | 25+TT | 5-TT |
| 53 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 54 | 23 | N/A | 6.5 | 1.5 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| 55 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 56 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 57 | 23 | N/A | 10.5 | 1.5 | 11 | 6 | 2 | 25+TT | 5-TT |
| 58 | 23 | N/A | 13.5 | 1.5 | 8 | 6 | 2 | 25+TT | 2-TT |
| 59 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 60 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 61 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 62 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 63 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 64 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 65 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 66 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 67 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 68 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 69 | 23 | N/A | 10.5 | 1.5 | 11 | 6 | 2 | 25+TT | 5-TT |
| 70 | 23 | N/A | 13.5 | 1.5 | 8 | 6 | 2 | 25+TT | 2-TT |
| 71 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 72 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 73 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 74 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 75 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 76 | 23 | N/A | 13.5 | 0 | 9.5 | 6 | 2 | 25+TT | 3.5-TT |
| 77 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 78 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 79 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 80 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 81 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 82 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 83 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 84 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 85 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 86 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 87 | 23 | N/A | 10.5 | 1.5 | 11 | 6 | 2 | 25+TT | 5-TT |
| 88 | 23 | N/A | 13.5 | 1.5 | 8 | 6 | 2 | 25+TT | 2-TT |
| 89 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 90 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 91 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 92 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 93 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 94 | 23 | N/A | 13.5 | 0 | 9.5 | 6 | 2 | 25+TT | 3.5-TT |
| 95 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 96 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 97 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 98 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 99 | 23 | 3 | N/A | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 100 | 23 | 3 | N/A | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 101 | 23 | 1.5 | N/A | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 102 | 23 | 1.5 | N/A | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 103 | 23 | N/A | 5.5 | 1.5 | 16 | 6 | 2 | 25+TT | 10-TT |
| 104 | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 105 | 23 | N/A | 2 | 0 | 21 | 3.5 | 2 | 25+TT | 17.5-TT |
| 106 | 23 | 1 | N/A | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 107 | 23 | N/A | 7 | 1.5 | 14.5 | 6 | 2 | 25+TT | 8.5-TT |
| 108 | 23 | N/A | 7 | 0 | 16 | 6 | 2 | 25+TT | 10-TT |
| 109 | 23 | N/A | 3 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 110 | 23 | 3 | N/A | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-29: UE Power Class 3 test requirements for NS_45 (contiguous allocation)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 3 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 4, 5 | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 6 | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7, 8 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 9 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 10, 11 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 12 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-30: UE Power Class 3 test requirements for NS_46 (contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 2.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 2 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 3 | 23 | 0 | 2.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 4 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 2 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 6 | 23 | 0 | 2.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 7 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 9 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 10 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 11 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 12 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 13 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 14 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 3 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 16 | 23 | 0 | 3.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 17 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 18 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 19 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 20 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 21 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 22 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-30a: UE Power Class 1 test requirements for NS_46 (contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 31 | 0 | 2.5 | 3 | 0 | 28 | 2 | 3 | 33+TT | 26-TT |
| 2 | 31 | 0 | 6.5 | 7 | 0 | 24 | 2 | 3 | 33+TT | 22-TT |
| 3 | 31 | 0 | 4.5 | 12.5 | 0 | 18.5 | 4 | 3 | 33+TT | 14.5-TT |
| 4 | 31 | 0 | 4.5 | 0 | 0 | 26.5 | 2 | 3 | 33+TT | 24.5-TT |
| 5 | 31 | 0 | 4.5 | 7 | 0 | 24 | 2 | 3 | 33+TT | 22-TT |
| 6 | 31 | 0 | 2.5 | 4 | 0 | 27 | 2 | 3 | 33+TT | 25-TT |
| 7 | 31 | 0 | 4.5 | 6 | 0 | 25 | 2 | 3 | 33+TT | 23-TT |
| 8 | 31 | 0 | 4.5 | 7 | 0 | 24 | 2 | 3 | 33+TT | 22-TT |
| 9 | 31 | 0 | 4.5 | 12.5 | 0 | 18.5 | 4 | 3 | 33+TT | 14.5-TT |
| 10 | 31 | 0 | 4.5 | 0 | 0 | 26.5 | 2 | 3 | 33+TT | 24.5-TT |
| 11 | 31 | 0 | 4.5 | 6 | 0 | 25 | 2 | 3 | 33+TT | 23-TT |
| 12 | 31 | 0 | 2.5 | 4 | 0 | 27 | 2 | 3 | 33+TT | 25-TT |
| 13 | 31 | 0 | 4.5 | 6 | 0 | 25 | 2 | 3 | 33+TT | 23-TT |
| 14 | 31 | 0 | 4.5 | 7 | 0 | 24 | 2 | 3 | 33+TT | 22-TT |
| 15 | 31 | 0 | 2.5 | 4 | 0 | 27 | 2 | 3 | 33+TT | 25-TT |
| 16 | 31 | 0 | 4.5 | 12.5 | 0 | 18.5 | 4 | 3 | 33+TT | 14.5-TT |
| 17 | 31 | 0 | 4.5 | 6 | 0 | 25 | 2 | 3 | 33+TT | 23-TT |
| 18 | 31 | 0 | 4.5 | 7 | 0 | 24 | 2 | 3 | 33+TT | 22-TT |
| 19 | 31 | 0 | 3.5 | 4 | 0 | 27 | 2 | 3 | 33+TT | 25-TT |
| 20 | 31 | 0 | 6.5 | 13 | 0 | 18 | 4 | 3 | 33+TT | 14-TT |
| 21 | 31 | 0 | 6.5 | 15 | 0 | 16 | 5 | 3 | 33+TT | 11-TT |
| 22 | 31 | 0 | 6.5 | 10 | 0 | 21 | 2 | 3 | 33+TT | 19-TT |
| 23 | 31 | 0 | 4.5 | 13 | 0 | 18 | 4 | 3 | 33+TT | 14-TT |
| 24 | 31 | 0 | 3.5 | 5 | 0 | 26 | 2 | 3 | 33+TT | 24-TT |
| 25 | 31 | 0 | 6.5 | 9 | 0 | 22 | 2 | 3 | 33+TT | 20-TT |
| 26 | 31 | 0 | 6.5 | 13 | 0 | 18 | 4 | 3 | 33+TT | 14-TT |
| 27 | 31 | 0 | 6.5 | 15 | 0 | 16 | 5 | 3 | 33+TT | 11-TT |
| 28 | 31 | 0 | 6.5 | 10 | 0 | 21 | 2 | 3 | 33+TT | 19-TT |
| 29 | 31 | 0 | 6.5 | 9 | 0 | 22 | 2 | 3 | 33+TT | 20-TT |
| 30 | 31 | 0 | 3.5 | 5 | 0 | 26 | 2 | 3 | 33+TT | 24-TT |
| 31 | 31 | 0 | 6.5 | 9 | 0 | 22 | 2 | 3 | 33+TT | 20-TT |
| 32 | 31 | 0 | 6.5 | 13 | 0 | 18 | 4 | 3 | 33+TT | 14-TT |
| 33 | 31 | 0 | 3.5 | 4 | 0 | 27 | 2 | 3 | 33+TT | 25-TT |
| 34 | 31 | 0 | 6.5 | 15 | 0 | 16 | 5 | 3 | 33+TT | 11-TT |
| 35 | 31 | 0 | 6.5 | 9 | 0 | 22 | 2 | 3 | 33+TT | 20-TT |
| 36 | 31 | 0 | 6.5 | 13 | 0 | 18 | 4 | 3 | 33+TT | 14-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-30b: UE Power Class 2 test requirements for NS_46 (contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 2.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 2 | 26 | 0 | 3.5 | 2.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 3 | 26 | 0 | 4.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 4 | 26 | 0 | 2.5 | 2.5 | 0 | 23.5 | 2 | 3 | 28+TT | 20.5-TT |
| 5 | 26 | 0 | 2.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 6 | 26 | 0 | 4.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 7 | 26 | 0 | 4.5 | 11 | 0 | 15 | 5 | 3 | 28+TT | 10-TT |
| 8 | 26 | 0 | 3.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 9 | 26 | 0 | 3.5 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 10 | 26 | 0 | 3.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 11 | 26 | 0 | 6.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 12 | 26 | 0 | 3.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 13 | 26 | 0 | 3.5 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 14 | 26 | 0 | 6.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 15 | 26 | 0 | 6.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 16 | 26 | 0 | 3.5 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-31: UE Power Class 3 test requirements (NS_44) for band n38

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 2 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 3 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 4 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 5 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 6 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 7 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 8 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 9 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 10 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 11 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 12 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 13 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 14 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 15 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 16 | 23 | 0 | 2.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 17 | 23 | 0 | 2.5 | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 18 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 19 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 20 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 21 | 23 | 0 | 4.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 22 | 23 | 0 | 4.5 | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 23 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 24 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 25 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 26 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 27 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 28 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 29 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 30 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 31 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 32 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 33 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 34 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 35 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 36 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 37 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 38 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 39 | 23 | 0 | 3.5 | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 40 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 41 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 42 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 43 | 23 | 0 | 6.5 | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 44 | 23 | 0 | 6.5 | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-32: UE Power Class 3 test requirements (NS_21)

| Test ID | ChBw (MHz) | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 5 | 23 | 0.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 10 | 23 | 0.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 3 | 5 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
|  | 10 | 23 | 0.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 5 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19 -TT |
| 4 | 10 | 23 | 0.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 5a | 5 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19 -TT |
| 5b | 10 | 23 | 0.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 6, 7 | 5 | 23 | 1 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
|  | 10 | 23 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 8 | 5 | 23 | 1 | 2.0 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
|  | 10 | 23 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 5 | 23 | 1 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18 -TT |
| 9 | 10 | 23 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 10a | 5 | 23 | 1 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18 -TT |
| 10b | 10 | 23 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 11, 12 | 5 | 23 | 2 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5 -TT |
|  | 10 | 23 | 2 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 13 | 5 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18 -TT |
|  | 10 | 23 | 2 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 5 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18 -TT |
| 14 | 10 | 23 | 2 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 15a | 5 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18 -TT |
| 15b | 10 | 23 | 2 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 16, 17 | 5 | 23 | 2.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5 -TT |
|  | 10 | 23 | 2.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 18 | 5 | 23 | 2.5 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18 -TT |
|  | 10 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 5 | 23 | 2.5 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18 -TT |
| 19 | 10 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 20a | 5 | 23 | 2.5 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18 -TT |
| 20b | 10 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 21, 22 | 5 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
|  | 10 | 23 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 23 | 5 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
|  | 10 | 23 | 4.5 | 4 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
|  | 5 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 24 | 10 | 23 | 4.5 | 3 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 25a | 5 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 25b | 10 | 23 | 4.5 | 3 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 26, 27 | 5 | 23 | 3 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5 -TT |
|  | 10 | 23 | 3 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 28 | 5 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5 -TT |
|  | 10 | 23 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
|  | 5 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 29 | 10 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 30a | 5 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 30b | 10 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 31, 32 | 5 | 23 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5 -TT |
|  | 10 | 23 | 3 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 33 | 5 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5 -TT |
|  | 10 | 23 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
|  | 5 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 34 | 10 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 35a | 5 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 35b | 10 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 36, 37 | 5 | 23 | 3.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
|  | 10 | 23 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 38 | 5 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5 -TT |
|  | 10 | 23 | 3.5 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
|  | 5 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 39 | 10 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 40a | 5 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 40b | 10 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 41, 42 | 5 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 10 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 43 | 5 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 10 | 23 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 5 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 44 | 10 | 23 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 45a | 5 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 45b | 10 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-33: UE Power Class 3 test requirements (NS_49)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 2 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 3 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 4 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 5 | 23 | 0 | 1 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 6 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 7 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 8 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 9 | 23 | 0 | 1 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 10 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 11 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 12 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 13 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 16 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 17 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 18 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 19 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 20 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 21 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 22 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 23 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 24 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 25 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 26 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 27 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 28 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 29 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 30 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 31 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 32 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 33 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 34 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 35 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 36 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 37 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 38 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 39 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 40 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 41 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 42 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 43 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 44 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 45 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 46 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 47 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 48 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 49 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 50 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 51 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 52 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 53 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 54 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 55 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 56 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 57 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 58 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 59 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 60 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 61 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 62 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 63 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 64 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 65 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 66 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 67 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 68 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 69 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 70 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 71 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 72 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 73 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 74 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 75 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 76 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 77 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 78 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 79 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 80 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 81 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 82 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 83 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 84 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 85 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 86 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 87 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 88 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 89 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 90 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 91 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 92 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 93 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 94 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 95 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 96 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 97 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 98 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 99 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 100 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 101 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 102 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 103 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 104 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-33a: UE Power Class 2 test requirements (NS_49)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 1 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 2 | 26 | 0 | 3.5 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 3 | 26 | 0 | 1 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 4 | 26 | 0 | 1 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 5 | 26 | 0 | 1 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 6 | 26 | 0 | 1 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 7 | 26 | 0 | 1 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 8 | 26 | 0 | 3.5 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 9 | 26 | 0 | 0 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 10 | 26 | 0 | 1 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 11 | 26 | 0 | 1 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 12 | 26 | 0 | 1 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 13 | 26 | 0 | 1 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 14 | 26 | 0 | 1 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 15 | 26 | 0 | 3.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 16 | 26 | 0 | 1 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 17 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 18 | 26 | 0 | 1 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 19 | 26 | 0 | 1 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 20 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 21 | 26 | 0 | 1 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 22 | 26 | 0 | 1 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 23 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 24 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 25 | 26 | 0 | 1 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 26 | 26 | 0 | 1 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 27 | 26 | 0 | 1 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 28 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 29 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 30 | 26 | 0 | 1 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 31 | 26 | 0 | 1 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 32 | 26 | 0 | 1 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 33 | 26 | 0 | 4.5 | 4 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 34 | 26 | 0 | 4.5 | 4 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 35 | 26 | 0 | 4.5 | 4 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 36 | 26 | 0 | 4.5 | 4 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 37 | 26 | 0 | 4.5 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 38 | 26 | 0 | 4.5 | 4 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 39 | 26 | 0 | 4.5 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 40 | 26 | 0 | 4.5 | 4 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 41 | 26 | 0 | 4.5 | 4 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 42 | 26 | 0 | 4.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 43 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 44 | 26 | 0 | 4.5 | 4 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 45 | 26 | 0 | 4.5 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 46 | 26 | 0 | 4.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 47 | 26 | 0 | 4.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 48 | 26 | 0 | 4.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 49 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 50 | 26 | 0 | 4.5 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 51 | 26 | 0 | 4.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 52 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 53 | 26 | 0 | 4.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 54 | 26 | 0 | 4.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 55 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 56 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 57 | 26 | 0 | 4.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 58 | 26 | 0 | 4.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 59 | 26 | 0 | 4.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 60 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 61 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 62 | 26 | 0 | 4.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 63 | 26 | 0 | 4.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 64 | 26 | 0 | 4.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 65 | 26 | 0 | 3 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 66 | 26 | 0 | 3.5 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 67 | 26 | 0 | 3 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 68 | 26 | 0 | 3 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 69 | 26 | 0 | 3 | 7.5 | 0 | 18.5 | 4 | 3 | 28+TT | 14.5-TT |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 70 | 26 | 0 | 3 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 71 | 26 | 0 | 3 | 7.5 | 0 | 18.5 | 4 | 3 | 28+TT | 14.5-TT |
| 72 | 26 | 0 | 3.5 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 73 | 26 | 0 | 1.5 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 74 | 26 | 0 | 3 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 75 | 26 | 0 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 76 | 26 | 0 | 3 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 77 | 26 | 0 | 3 | 7.5 | 0 | 18.5 | 4 | 3 | 28+TT | 14.5-TT |
| 78 | 26 | 0 | 3 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 79 | 26 | 0 | 3.5 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 80 | 26 | 0 | 3 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 81 | 26 | 0 | 1.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 82 | 26 | 0 | 3 | 7.5 | 0 | 18.5 | 4 | 3 | 28+TT | 14.5-TT |
| 83 | 26 | 0 | 3 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 84 | 26 | 0 | 1.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 85 | 26 | 0 | 3 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 86 | 26 | 0 | 3 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 87 | 26 | 0 | 1.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 88 | 26 | 0 | 1.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 89 | 26 | 0 | 3 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 90 | 26 | 0 | 3 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 91 | 26 | 0 | 3 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 92 | 26 | 0 | 1.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 93 | 26 | 0 | 1.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 94 | 26 | 0 | 3 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 95 | 26 | 0 | 3 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 96 | 26 | 0 | 3 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 97 | 26 | 0 | 6.5 | 5.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 98 | 26 | 0 | 6.5 | 5.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 99 | 26 | 0 | 6.5 | 5.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 100 | 26 | 0 | 6.5 | 5.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 101 | 26 | 0 | 6.5 | 7.5 | 0 | 18.5 | 4 | 3 | 28+TT | 14.5-TT |
| 102 | 26 | 0 | 6.5 | 5.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 103 | 26 | 0 | 6.5 | 7.5 | 0 | 18.5 | 4 | 3 | 28+TT | 14.5-TT |
| 104 | 26 | 0 | 6.5 | 5.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 105 | 26 | 0 | 6.5 | 5.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 106 | 26 | 0 | 6.5 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 107 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 108 | 26 | 0 | 6.5 | 5.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 109 | 26 | 0 | 6.5 | 7.5 | 0 | 18.5 | 4 | 3 | 28+TT | 14.5-TT |
| 110 | 26 | 0 | 6.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 111 | 26 | 0 | 6.5 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 112 | 26 | 0 | 6.5 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 113 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 114 | 26 | 0 | 6.5 | 7.5 | 0 | 18.5 | 4 | 3 | 28+TT | 14.5-TT |
| 115 | 26 | 0 | 6.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 116 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 117 | 26 | 0 | 6.5 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 118 | 26 | 0 | 6.5 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 119 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 120 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 121 | 26 | 0 | 6.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 122 | 26 | 0 | 6.5 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 123 | 26 | 0 | 6.5 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 124 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 125 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 126 | 26 | 0 | 6.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 127 | 26 | 0 | 6.5 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 128 | 26 | 0 | 6.5 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-34: UE Power Class 3 test requirements (NS_06) for band n12 and n14

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20.5-TT |
| 2 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20.5-TT |
| 3 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20.5-TT |
| 4 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 20-TT |
| 5 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 20-TT |
| 6 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 20-TT |
| 7 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 19-TT |
| 8 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 19-TT |
| 9 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 19-TT |
| 10 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 11 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 12 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 13 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 14 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 15 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 16 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 17 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 18 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 19 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 20 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 21 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 22 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 23 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 24 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 25 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 26 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 27 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-35: UE Power Class 1 test requirements (NS_06) for band n14

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 31 | 0.5 | 0 | 0 | 30.5 | 2 | 3 | 33+TT | 27.5-TT |
| 2 | 31 | 0.5 | 0 | 0 | 30.5 | 2 | 3 | 33+TT | 27.5-TT |
| 3 | 31 | 0.5 | 0 | 0 | 30.5 | 2 | 3 | 33+TT | 27.5-TT |
| 4 | 31 | 1 | 0 | 0 | 30 | 2 | 3 | 33+TT | 27-TT |
| 5 | 31 | 1 | 0 | 0 | 30 | 2 | 3 | 33+TT | 27-TT |
| 6 | 31 | 1 | 0 | 0 | 30 | 2 | 3 | 33+TT | 27-TT |
| 7 | 31 | 2 | 0 | 0 | 29 | 2 | 3 | 33+TT | 26-TT |
| 8 | 31 | 2 | 0 | 0 | 29 | 2 | 3 | 33+TT | 26-TT |
| 9 | 31 | 2 | 0 | 0 | 29 | 2 | 3 | 33+TT | 26-TT |
| 10 | 31 | 2.5 | 0 | 0 | 28.5 | 2 | 3 | 33+TT | 25.5-TT |
| 11 | 31 | 2.5 | 0 | 0 | 28.5 | 2 | 3 | 33+TT | 25.5-TT |
| 12 | 31 | 2.5 | 0 | 0 | 28.5 | 2 | 3 | 33+TT | 25.5-TT |
| 13 | 31 | 4.5 | 0 | 0 | 26.5 | 2 | 3 | 33+TT | 23.5-TT |
| 14 | 31 | 4.5 | 0 | 0 | 26.5 | 2 | 3 | 33+TT | 23.5-TT |
| 15 | 31 | 4.5 | 0 | 0 | 26.5 | 2 | 3 | 33+TT | 23.5-TT |
| 16 | 31 | 3 | 0 | 0 | 28 | 2 | 3 | 33+TT | 25-TT |
| 17 | 31 | 3 | 0 | 0 | 28 | 2 | 3 | 33+TT | 25-TT |
| 18 | 31 | 3 | 0 | 0 | 28 | 2 | 3 | 33+TT | 25-TT |
| 19 | 31 | 3 | 0 | 0 | 28 | 2 | 3 | 33+TT | 25-TT |
| 20 | 31 | 3 | 0 | 0 | 28 | 2 | 3 | 33+TT | 25-TT |
| 21 | 31 | 3 | 0 | 0 | 28 | 2 | 3 | 33+TT | 25-TT |
| 22 | 31 | 3.5 | 0 | 0 | 27.5 | 2 | 3 | 33+TT | 24.5-TT |
| 23 | 31 | 3.5 | 0 | 0 | 27.5 | 2 | 3 | 33+TT | 24.5-TT |
| 24 | 31 | 3.5 | 0 | 0 | 27.5 | 2 | 3 | 33+TT | 24.5-TT |
| 25 | 31 | 6.5 | 0 | 0 | 24.5 | 2 | 3 | 33+TT | 21.5-TT |
| 26 | 31 | 6.5 | 0 | 0 | 24.5 | 2 | 3 | 33+TT | 21.5-TT |
| 27 | 31 | 6.5 | 0 | 0 | 24.5 | 2 | 3 | 33+TT | 21.5-TT |
| 28 | 31 | 0.5 | 0 | 0 | 30.5 | 2 | 3 | 33+TT | 27.5-TT |
| 29 | 31 | 0.5 | 0 | 0 | 30.5 | 2 | 3 | 33+TT | 27.5-TT |
| 30 | 31 | 0 | 0 | 0 | 31 | 2 | 3 | 33+TT | 28-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-35a: UE Power Class 1 test requirements (NS_06) for band n85

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 31 | 0.5 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 2 | 31 | 0.5 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 3 | 31 | 0.5 | 0 | 0 | 30.5 | 2 | 2.5 | 31+TT | 28-TT |
| 4 | 31 | 1 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 5 | 31 | 1 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 6 | 31 | 1 | 0 | 0 | 30 | 2 | 2.5 | 31+TT | 27.5-TT |
| 7 | 31 | 2 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 8 | 31 | 2 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 9 | 31 | 2 | 0 | 0 | 29 | 2 | 2.5 | 31+TT | 26.5-TT |
| 10 | 31 | 2.5 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 11 | 31 | 2.5 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 12 | 31 | 2.5 | 0 | 0 | 28.5 | 2 | 2.5 | 31+TT | 26-TT |
| 13 | 31 | 4.5 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 14 | 31 | 4.5 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 15 | 31 | 4.5 | 0 | 0 | 26.5 | 2 | 2.5 | 31+TT | 24-TT |
| 16 | 31 | 3 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 17 | 31 | 3 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 18 | 31 | 3 | 0 | 0 | 28 | 2 | 2.5 | 31+TT | 25.5-TT |
| 19 | 31 | 3 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 20 | 31 | 3 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 21 | 31 | 3 | 0 | 0 | 28 | 2 | 2.5 | 31+TT | 25.5-TT |
| 22 | 31 | 3.5 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 23 | 31 | 3.5 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 24 | 31 | 3.5 | 0 | 0 | 27.5 | 2 | 2.5 | 31+TT | 25-TT |
| 25 | 31 | 6.5 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 26 | 31 | 6.5 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 27 | 31 | 6.5 | 0 | 0 | 24.5 | 2 | 2.5 | 31+TT | 22-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-36: UE Power Class 3 test requirements (NS_56) for band n24

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 2 | 23 | 0.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 3 | 23 | 0.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 4 | 23 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 5 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 6 | 23 | 1 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 7 | 23 | 1 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 8 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 9 | 23 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 10 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 11 | 23 | 2 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 12 | 23 | 2 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 13 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 14 | 23 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 15 | 23 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 16 | 23 | 2.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 17 | 23 | 2.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 18 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 19 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 20 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 21 | 23 | 4.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 22 | 23 | 4.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 23 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 24 | 23 | 4.5 | 4 | 0 | 18.5 | 3.5 | 3 | 25+TT | 15-TT |
| 25 | 23 | 4.5 | 4 | 0 | 18.5 | 3.5 | 3 | 25+TT | 15-TT |
| 26 | 23 | 0.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 27 | 23 | 0.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 28 | 23 | 0.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 29 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 30 | 23 | 0.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 31 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 32 | 23 | 0.5 | 2 | 0 | 21 | 2 | 3 | 25+TT | 18-TT |
| 33 | 23 | 0.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 34 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 35 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 36 | 23 | 0.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 37 | 23 | 1 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 38 | 23 | 1 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 39 | 23 | 1 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 40 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 41 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 42 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 43 | 23 | 1 | 2 | 0 | 21 | 2 | 3 | 25+TT | 18-TT |
| 44 | 23 | 1 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 45 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 46 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 47 | 23 | 1 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 48 | 23 | 2 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 49 | 23 | 2 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 50 | 23 | 2 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 51 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 52 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 53 | 23 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 54 | 23 | 2 | 2 | 0 | 21 | 2 | 3 | 25+TT | 18-TT |
| 55 | 23 | 2 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 56 | 23 | 1 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 57 | 23 | 1 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 58 | 23 | 2 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 59 | 23 | 2.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 23 | 2.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 61 | 23 | 2.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 62 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 63 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 64 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 65 | 23 | 2.5 | 2 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 66 | 23 | 2.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 67 | 23 | 2.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 68 | 23 | 2.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 69 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 70 | 23 | 4.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 71 | 23 | 4.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 72 | 23 | 4.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 73 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 74 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 75 | 23 | 4.5 | 4 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 76 | 23 | 4.5 | 2 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 77 | 23 | 4.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 78 | 23 | 4.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 79 | 23 | 4.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 80 | 23 | 4.5 | 3 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 81 | 23 | 3 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 82 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 83 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 84 | 23 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 85 | 23 | 1.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 86 | 23 | 3 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 87 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 88 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 89 | 23 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 90 | 23 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 91 | 23 | 3.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 92 | 23 | 3.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 93 | 23 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 94 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 95 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 96 | 23 | 6.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 97 | 23 | 6.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 98 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 99 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 100 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 101 | 23 | 3 | 12 | 0 | 9 | 6 | 3 | 25+TT | 5-TT |
| 102 | 23 | 3 | 8 | 0 | 11 | 5 | 3 | 25+TT | 10-TT |
| 103 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 104 | 23 | 1.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 105 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 106 | 23 | 1.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 107 | 23 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 108 | 23 | 3 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 109 | 23 | 1.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 110 | 23 | 1.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 111 | 23 | 3 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 112 | 23 | 3 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 113 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 114 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 115 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 116 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 117 | 23 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 118 | 23 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 119 | 23 | 3 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 120 | 23 | 2 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 121 | 23 | 2 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 122 | 23 | 3 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 123 | 23 | 3.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 124 | 23 | 3.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 125 | 23 | 3.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 126 | 23 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 127 | 23 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 128 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 129 | 23 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 130 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 3 | 25+TT | 14.5-TT |
| 131 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 3 | 25+TT | 14.5-TT |
| 132 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 3 | 25+TT | 14.5-TT |
| 133 | 23 | 3.5 | 3 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 134 | 23 | 6.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 135 | 23 | 6.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 136 | 23 | 6.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 137 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 138 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 139 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 140 | 23 | 6.5 | 2 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 141 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 142 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 143 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 144 | 23 | 6.5 | 3 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-37: UE Power Class 3 test requirements (NS_07) for band n13

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 2 | 23 | 0 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 3 | 23 | 0 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 4 | 23 | 0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 5 | 23 | 0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 6 | 23 | 0.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 7 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 8 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 9 | 23 | 1 | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 10 | 23 | 0 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 11 | 23 | 0 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 12 | 23 | 0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 13 | 23 | 0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 14 | 23 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 15 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 16 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 17 | 23 | 2 | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 18 | 23 | 1 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 19 | 23 | 1 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 20 | 23 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 21 | 23 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 22 | 23 | 2 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 23 | 23 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 24 | 23 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 25 | 23 | 2.5 | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 26 | 23 | 2.5 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 27 | 23 | 2.5 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 28 | 23 | 2.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 29 | 23 | 2.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 30 | 23 | 2.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 31 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 32 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 33 | 23 | 4.5 | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 34 | 23 | 4.5 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 35 | 23 | 4.5 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 36 | 23 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 37 | 23 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 38 | 23 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 39 | 23 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 40 | 23 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 41 | 23 | 3 | 14 | 0 | 9 | 6 | 2 | 25+TT | 3-TT |
| 42 | 23 | 1.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 43 | 23 | 1.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 44 | 23 | 1.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 45 | 23 | 1.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 46 | 23 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 47 | 23 | 1.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 48 | 23 | 1.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 49 | 23 | 3 | 14 | 0 | 9 | 6 | 2 | 25+TT | 3-TT |
| 50 | 23 | 2 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 51 | 23 | 2 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 52 | 23 | 2 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 53 | 23 | 2 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 54 | 23 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 55 | 23 | 2 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 56 | 23 | 2 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 57 | 23 | 3.5 | 14 | 0 | 9 | 6 | 2 | 25+TT | 3-TT |
| 58 | 23 | 3.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 59 | 23 | 3.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 60 | 23 | 3.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 61 | 23 | 3.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 62 | 23 | 3.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 63 | 23 | 3.5 | 3 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 64 | 23 | 3.5 | 3 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 65 | 23 | 6.5 | 14 | 0 | 9 | 6 | 2 | 25+TT | 3-TT |
| 66 | 23 | 6.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 67 | 23 | 6.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 68 | 23 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 69 | 23 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 70 | 23 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 71 | 23 | 6.5 | 3 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 72 | 23 | 6.5 | 3 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-38: UE Power Class 2 test requirements (NS_50) for band n39 (Step 4)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19.0 - TT |  |
| 2 | 26 | 0 | 1 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT |  |
| 3 | 26 | 0 | 0 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT |  |
| 4 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 5 | 26 | 0 | 1 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 6 | 26 | 0 | 0 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT |  |
| 7 | 26 | 0 | 3.5 | 5 | 0 |  | 21 |  | 2.0 |  | 3 | 28.0 + TT | 18 - TT |  |
| 8 | 26 | 0 | 1 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19.0 - TT |  |
| 9 | 26 | 0 | 0 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT |  |
| 10 | 26 | 0 | 1 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 11 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19.0 - TT |  |
| 12 | 26 | 0 | 0 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 – TT |  |
| 13 | 26 | 0 | 0 | 1 | 0 |  | 25 |  | 2.0 |  | 3 | 28.0 + TT | 22 - TT |  |
| 14 | 26 | 0 | 0 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 – TT |  |
| 15 | 26 | 0 | 1 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 16 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 17 | 26 | 0 | 0 | 1 | 0 |  | 25 |  | 2.0 |  | 3 | 28.0 + TT | 22 - TT |  |
| 18 | 26 | 0 | 0 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT |  |
| 19 | 26 | 0 | 0 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT |  |
| 20 | 26 | 0 | 1 | 11.5 | 0 |  | 14.5 |  | 5.0 |  | 3 | 28.0 + TT | 9.5 - TT |  |
| 21 | 26 | 0 | 1 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19.0 - TT |  |
| 22 | 26 | 0 | 1 | 10 | 0 |  | 16 |  | 5.0 |  | 3 | 28.0 + TT | 11 - TT |  |
| 23 | 26 | 0 | 1 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT |  |
| 24 | 26 | 0 | 3.5 | 8 | 0 |  | 18 |  | 4.0 |  | 3 | 28.0 + TT | 14.0 - TT |  |
| 25 | 26 | 0 | 1 | 11.5 | 0 |  | 14.5 |  | 5.0 |  | 3 | 28.0 + TT | 9.5 - TT |  |
| 26 | 26 | 0 | 0 | 10 | 0 |  | 16 |  | 2.0 |  | 3 | 28.0 + TT | 13 - TT |  |
| 27 | 26 | 0 | 0 | 4 | 0 |  | 22 |  | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT |  |
| 28 | 26 | 0 | 0 | 3.5 | 0 |  | 22.5 |  | 2.0 |  | 3 | 28.0 + TT | 19.5 – TT |  |
| 29 | 26 | 0 | 0 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT |  |
| 30 | 26 | 0 | 0 | 8 | 0 |  | 18 |  | 4.0 |  | 3 | 28.0 + TT | 14.0 - TT |  |
| 31 | 26 | 0 | 4.5 | 4 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 32 | 26 | 0 | 4.5 | 2.5 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 33 | 26 | 0 | 4.5 | 2.5 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 34 | 26 | 0 | 4.5 | 4 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 35 | 26 | 0 | 4.5 | 4 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 36 | 26 | 0 | 4.5 | 2.5 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 37 | 26 | 0 | 4.5 | 4 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 38 | 26 | 0 | 4.5 | 4 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 39 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 40 | 26 | 0 | 4.5 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 41 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 42 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 43 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 44 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 45 | 26 | 0 | 4.5 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 46 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 47 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 48 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 49 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 50 | 26 | 0 | 4.5 | 11.5 | 0 |  | 14.5 |  | 5.0 |  | 3 | 28.0 + TT | 9.5 - TT |  |
| 51 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 52 | 26 | 0 | 4.5 | 10 | 0 |  | 16 |  | 5.0 |  | 3 | 28.0 + TT | 11 - TT |  |
| 53 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 54 | 26 | 0 | 4.5 | 8 | 0 |  | 18.0 |  | 4.0 |  | 3 | 28.0 + TT | 14 - TT |  |
| 55 | 26 | 0 | 4.5 | 11.5 | 0 |  | 14.5 |  | 5.0 |  | 3 | 28.0 + TT | 9.5 - TT |  |
| 56 | 26 | 0 | 4.5 | 10 | 0 |  | 16 |  | 5.0 |  | 3 | 28.0 + TT | 11 - TT |  |
| 57 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 58 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 59 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 60 | 26 | 0 | 4.5 | 8 | 0 |  | 18 |  | 5.0 |  | 3 | 28.0 + TT | 13 - TT |  |
| 61 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 62 | 26 | 0 | 3 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 63 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 64 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 65 | 26 | 0 | 3 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 66 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 67 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 68 | 26 | 0 | 3 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 69 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 70 | 26 | 0 | 3 | 7 | 0 |  | 19 |  | 3.5 |  | 3 | 28.0 + TT | 15.5 - TT |  |
| 71 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 72 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 73 | 26 | 0 | 1.5 | 2 | 0 |  | 24 |  | 2.0 |  | 3 | 28.0 + TT | 21 - TT |  |
| 74 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 75 | 26 | 0 | 3 | 7 | 0 |  | 19 |  | 3.5 |  | 3 | 28.0 + TT | 15.5 - TT |  |
| 76 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 77 | 26 | 0 | 1.5 | 2 | 0 |  | 24 |  | 2.0 |  | 3 | 28.0 + TT | 21 - TT |  |
| 78 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 79 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 80 | 26 | 0 | 3 | 12.5 | 0 |  | 13.5 |  | 5.0 |  | 3 | 28.0 + TT | 8.5 - TT |  |
| 81 | 26 | 0 | 3 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 82 | 26 | 0 | 3 | 11 | 0 |  | 15.0 |  | 5.0 |  | 3 | 28.0 + TT | 10 - TT |  |
| 83 | 26 | 0 | 3 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 84 | 26 | 0 | 3.5 | 8 | 0 |  | 18 |  | 4.0 |  | 3 | 28.0 + TT | 14 - TT |  |
| 85 | 26 | 0 | 3 | 12.5 | 0 |  | 13.5 |  | 5.0 |  | 3 | 28.0 + TT | 8.5 - TT |  |
| 86 | 26 | 0 | 1.5 | 11 | 0 |  | 15 |  | 5.0 |  | 3 | 28.0 + TT | 10 - TT |  |
| 87 | 26 | 0 | 1.5 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 88 | 26 | 0 | 1.5 | 5 | 0 |  | 21 |  | 2.0 |  | 3 | 28.0 + TT | 18 - TT |  |
| 89 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 90 | 26 | 0 | 1.5 | 8 | 0 |  | 18 |  | 4 |  | 3 | 28.0 + TT | 14 - TT |  |
| 91 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 92 | 26 | 0 | 3 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 93 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 94 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 95 | 26 | 0 | 3 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 96 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 97 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 98 | 26 | 0 | 3 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 99 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 100 | 26 | 0 | 3 | 7 | 0 |  | 19 |  | 3.5 |  | 3 | 28.0 + TT | 15.5 - TT |  |
| 101 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 102 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 103 | 26 | 0 | 1.5 | 2 | 0 |  | 24 |  | 2.0 |  | 3 | 28.0 + TT | 21 - TT |  |
| 104 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 – TT |  |
| 105 | 26 | 0 | 3 | 7 | 0 |  | 19 |  | 3.5 |  | 3 | 28.0 + TT | 15.5 – TT |  |
| 106 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 107 | 26 | 0 | 1.5 | 2 | 0 |  | 24 |  | 2.0 |  | 3 | 28.0 + TT | 21 - TT |  |
| 108 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 109 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 – TT |  |
| 110 | 26 | 0 | 6.5 | 12.5 | 0 |  | 13.5 |  | 5.0 |  | 3 | 28.0 + TT | 8.5 - TT |  |
| 111 | 26 | 0 | 6.5 | 0 | 0 |  | 19.5 |  | 3.5 |  | 3 | 28.0 + TT | 16 - TT |  |
| 112 | 26 | 0 | 6.5 | 11 | 0 |  | 15 |  | 5.0 |  | 3 | 28.0 + TT | 10 - TT |  |
| 113 | 26 | 0 | 6.5 | 0 | 0 |  | 19.5 |  | 3.5 |  | 3 | 28.0 + TT | 16 - TT |  |
| 114 | 26 | 0 | 6.5 | 8 | 0 |  | 18 |  | 4.0 |  | 3 | 28.0 + TT | 14 - TT |  |
| 115 | 26 | 0 | 3 | 12.5 | 0 |  | 13.5 |  | 5.0 |  | 3 | 28.0 + TT | 8.5 - TT |  |
| 116 | 26 | 0 | 1.5 | 11 | 0 |  | 15 |  | 5.0 |  | 3 | 28.0 + TT | 10 - TT |  |
| 117 | 26 | 0 | 1.5 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 118 | 26 | 0 | 1.5 | 5 | 0 |  | 21 |  | 2.0 |  | 3 | 28.0 + TT | 18 – TT |  |
| 119 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 – TT |  |
| 120 | 26 | 0 | 1.5 | 8 | 0 |  | 18 |  | 4.0 |  | 3 | 28.0 + TT | 14 - TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Void. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-39: UE Power Class 3 test requirements (NS_50) for band n39 (Step 3)

| Test ID | PPowerClass (dBm) |  | ΔPPowerClass (dB) |  | MPR (dB) |  | A-MPR (dB) |  | ΔTC,c (dB) |  |  |  | PCMAX_L,f,c (dBm) |  |  |  | T(PCMAX_L,f,c) (dB) |  |  |  | TL,c (dB) |  | Upper limit (dBm) |  | Lower limit (dBm) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 |  | 0 |  | 1 |  | 5 |  | 0 |  |  |  | 18 |  |  |  | 4.0 |  |  |  | 2 |  | 25.0 + TT |  | 14.0- TT |  |  |  |
| 2 | 23 |  | 0 |  | 1 |  | 2 |  | 0 |  |  |  | 21 |  |  |  | 2.0 |  |  |  | 2 |  | 25.0 + TT |  | 19.0 - TT |  |  |  |
| 3 | 23 |  | 0 |  | 1 |  | 0 |  | 0 |  |  |  | 22 |  |  |  | 2.0 |  |  |  | 2 |  | 25.0 + TT |  | 20.0 - TT |  |  |  |
| 4 | 23 |  | 0 |  | 1 |  | 5 |  | 0 |  |  |  | 18 |  |  |  | 4.0 |  |  |  | 2 |  | 25.0 + TT |  | 14.0 - TT |  |  |  |
| 5 | 23 |  | 0 |  | 1 |  | 2 |  | 0 |  |  |  | 21 |  |  |  | 2.0 |  |  |  | 2 |  | 25.0 + TT |  | 19.0 - TT |  |  |  |
| 6 | 23 |  | 0 |  | 1 |  | 0 |  | 0 |  |  |  | 23 |  |  |  | 2.0 |  |  |  | 2 |  | 25.0 + TT |  | 21.0 - TT |  |  |  |
| 7 |  | 23 |  | 0 |  | 1 |  | 11 |  | 0 |  |  |  | 12 |  |  |  | 6.0 |  |  |  | 2 |  | 25.0 + TT |  | 6.0 - TT |  |  |
| 8 |  | 23 |  | 0 |  | 1 |  | 3 |  | 0 |  |  |  | 20 |  |  |  | 2.5 |  |  |  | 2 |  | 25.0 + TT |  | 17.5 - TT |  |  |
| 9 |  | 23 |  | 0 |  | 1 |  | 7 |  | 0 |  |  |  | 16 |  |  |  | 5.0 |  |  |  | 2 |  | 25.0 + TT |  | 11.0 - TT |  |  |
| 10 |  | 23 |  | 0 |  | 1 |  | 2 |  | 0 |  |  |  | 21 |  |  |  | 2.0 |  |  |  | 2 |  | 25.0 + TT |  | 19.0 - TT |  |  |
| 11 |  | 23 |  | 0 |  | 1 |  | 5 |  | 0 |  |  |  | 18 |  |  |  | 4.0 |  |  |  | 2 |  | 25.0 + TT |  | 14.0 - TT |  |  |
| 12 | 23 |  | 0 |  | 1 |  | 11 |  | 0 |  |  |  | 12 |  |  |  | 6.0 |  |  |  | 2 |  | 25.0 + TT |  | 6.0 - TT |  |  |  |
| 13 | 23 |  | 0 |  | 1 |  | 3 |  | 0 |  |  |  | 20 |  |  |  | 2.5 |  |  |  | 2 |  | 25.0 + TT |  | 17.5 - TT |  |  |  |
| 14 | 23 |  | 0 |  | 1 |  | 7 |  | 0 |  |  |  | 16 |  |  |  | 5.0 |  |  |  | 2 |  | 25.0 + TT |  | 11.0 - TT |  |  |  |
| 15 | 23 |  | 0 |  | 1 |  | 2 |  | 0 |  |  |  | 21 |  |  |  | 2.0 |  |  |  | 2 |  | 25.0 + TT |  | 19.0 - TT |  |  |  |
| 16 | 23 |  | 0 |  | 1 |  | 5 |  | 0 |  |  |  | 18 |  |  |  | 4.0 |  |  |  | 2 |  | 25.0 + TT |  | 14.0 - TT |  |  |  |
| 17 | 23 |  | 0 |  | 4.5 |  | 5 |  | 0 |  |  |  | 18 |  |  |  | 4.0 |  |  |  | 2 |  | 25.0 + TT |  | 14.0 - TT |  |  |  |
| 18 | 23 |  | 0 |  | 4.5 |  | 0 |  | 0 |  |  |  | 18.5 |  |  |  | 4.0 |  |  |  | 2 |  | 25.0 + TT |  | 14.5 - TT |  |  |  |
| 19 | 23 |  | 0 |  | 4.5 |  | 0 |  | 0 |  |  |  | 18.5 |  |  |  | 4.0 |  |  |  | 2 |  | 25.0 + TT |  | 14.5- TT |  |  |  |
| 20 | 23 |  | 0 |  | 4.5 |  | 5 |  | 0 |  |  |  | 18 |  |  |  | 4.0 |  |  |  | 2 |  | 25.0 + TT |  | 14.0 - TT |  |  |  |
| 21 | 23 |  | 0 |  | 4.5 |  | 0 |  | 0 |  |  |  | 18.5 |  |  |  | 4.0 |  |  |  | 2 |  | 25.0 + TT |  | 14.5 - TT |  |  |  |
| 22 | 23 |  | 0 |  | 4.5 |  | 0 |  | 0 |  |  |  | 18.5 |  |  |  | 4.0 |  |  |  | 2 |  | 25.0 + TT |  | 14.5 - TT |  |  |  |
| 23 |  | 23 |  | 0 |  | 4.5 |  | 11 |  | 0 |  |  |  | 12 |  |  |  | 6.0 |  |  |  | 2 |  | 25.0 + TT |  | 6.0 - TT |  |  |
| 24 |  | 23 |  | 0 |  | 4.5 |  | 0 |  | 0 |  |  |  | 18.5 |  |  |  | 4.0 |  |  |  | 2 |  | 25.0 + TT |  | 14.5 - TT |  |  |
| 25 |  | 23 |  | 0 |  | 4.5 |  | 7 |  | 0 |  |  |  | 16 |  |  |  | 5.0 |  |  |  | 2 |  | 25.0 + TT |  | 11.0 - TT |  |  |
| 26 |  | 23 |  | 0 |  | 4.5 |  | 0 |  | 0 |  |  |  | 18.5 |  |  |  | 4.0 |  |  |  | 2 |  | 25.0 + TT |  | 14.5 - TT |  |  |
| 27 |  | 23 |  | 0 |  | 4.5 |  | 5 |  | 0 |  |  |  | 18 |  |  |  | 4.0 |  |  |  | 2 |  | 25.0 + TT |  | 14.0 - TT |  |  |
| 28 | 23 |  | 0 |  | 4.5 |  | 11 |  | 0 |  |  |  | 12 |  |  |  | 6.0 |  |  |  | 2 |  | 25.0 + TT |  | 6.0 - TT |  |  |  |
| 29 | 23 |  | 0 |  | 4.5 |  | 0 |  | 0 |  |  |  | 18.5 |  |  |  | 4.0 |  |  |  | 2 |  | 25.0 + TT |  | 14.5 - TT |  |  |  |
| 30 | 23 |  | 0 |  | 3 |  | 7 |  | 0 |  |  |  | 16 |  |  |  | 5.0 |  |  |  | 2 |  | 25.0 + TT |  | 11.0 - TT |  |  |  |
| 31 | 23 |  | 0 |  | 3 |  | 0 |  | 0 |  |  |  | 20 |  |  |  | 2.5 |  |  |  | 2 |  | 25.0 + TT |  | 17.5 - TT |  |  |  |
| 32 | 23 |  | 0 |  | 3 |  | 5 |  | 0 |  |  |  | 18 |  |  |  | 4.0 |  |  |  | 2 |  | 25.0 + TT |  | 14.0 - TT |  |  |  |
| 33 | 23 |  | 0 |  | 3 |  | 6.5 |  | 0 |  |  |  | 16.5 |  |  |  | 5.0 |  |  |  | 2 |  | 25.0 + TT |  | 11.5 – TT |  |  |  |
| 34 | 23 |  | 0 |  | 3 |  | 0 |  | 0 |  |  |  | 20 |  |  |  | 2.5 |  |  |  | 2 |  | 25.0 + TT |  | 17.5 - TT |  |  |  |
| 35 | 23 |  | 0 |  | 3 |  | 0 |  | 0 |  |  |  | 20.0 |  |  |  | 2.5 |  |  |  | 2 |  | 25.0 + TT |  | 17.5 - TT |  |  |  |
| 36 | 23 |  | 0 |  | 3 |  | 6.5 |  | 0 |  |  |  | 16.5 |  |  |  | 5.0 |  |  |  | 2 |  | 25.0 + TT |  | 11.5 - TT |  |  |  |
| 37 | 23 |  | 0 |  | 3 |  | 0 |  | 0 |  |  |  | 20 |  |  |  | 2.5 |  |  |  | 2 |  | 25.0 + TT |  | 17.5 - TT |  |  |  |
| 38 |  | 23 |  | 0 |  | 3 |  | 12 |  | 0 |  |  |  | 11 |  |  |  | 6.0 |  |  |  | 2 |  | 25.0 + TT |  | 5.0 - TT |  |  |
| 39 |  | 23 |  | 0 |  | 3 |  | 4.5 |  | 0 |  |  |  | 18.5 |  |  |  | 4.0 |  |  |  | 2 |  | 25.0 + TT |  | 14.5 - TT |  |  |
| 40 |  | 23 |  | 0 |  | 3 |  | 8 |  | 0 |  |  |  | 15 |  |  |  | 5.0 |  |  |  | 2 |  | 25.0 + TT |  | 10.0 – TT |  |  |
| 41 |  | 23 |  | 0 |  | 3 |  | 3.5 |  | 0 |  |  |  | 19.5 |  |  |  | 3.5 |  |  |  | 2 |  | 25.0 + TT |  | 16 - TT |  |  |
| 42 |  | 23 |  | 0 |  | 3 |  | 5 |  | 0 |  |  |  | 18 |  |  |  | 4.0 |  |  |  | 2 |  | 25.0 + TT |  | 14.0 - TT |  |  |
| 43 | 23 |  | 0 |  | 3 |  | 0 |  | 0 |  |  |  | 20.0 |  |  |  | 2.5 |  |  |  | 2 |  | 25.0 + TT |  | 17.5 – TT |  |  |  |
| 44 | 23 |  | 0 |  | 3 |  | 12 |  | 0 |  |  |  | 11 |  |  |  | 6.0 |  |  |  | 2 |  | 25.0 + TT |  | 5.0 - TT |  |  |  |
| 45 | 23 |  | 0 |  | 3 |  | 4.5 |  | 0 |  |  |  | 18.5 |  |  |  | 4.0 |  |  |  | 2 |  | 25.0 + TT |  | 14.5 - TT |  |  |  |
| 46 | 23 |  | 0 |  | 3 |  | 3.5 |  | 0 |  |  |  | 19.5 |  |  |  | 3.5 |  |  |  | 2 |  | 25.0 + TT |  | 16 - TT |  |  |  |
| 47 | 23 |  | 0 |  | 3 |  | 5 |  | 0 |  |  |  | 18 |  |  |  | 4.0 |  |  |  | 2 |  | 25.0 + TT |  | 14.0 - TT |  |  |  |
| 48 | 23 |  | 0 |  | 6.5 |  | 6.5 |  | 0 |  |  |  | 16.5 |  |  |  | 5.0 |  |  |  | 2 |  | 25.0 + TT |  | 11.5 - TT |  |  |  |
| 49 | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  |  |  | 16.5 |  |  |  | 5.0 |  |  |  | 2 |  | 25.0 + TT |  | 11.5 - TT |  |  |  |
| 50 | 23 |  | 0 |  | 6.5 |  | 6 |  | 0 |  |  |  | 16.5 |  |  |  | 5.0 |  |  |  | 2 |  | 25.0 + TT |  | 11.5 - TT |  |  |  |
| 51 | 23 |  | 0 |  | 6.5 |  | 6.5 |  | 0 |  |  |  | 16.5 |  |  |  | 5.0 |  |  |  | 2 |  | 25.0 + TT |  | 11.5 - TT |  |  |  |
| 52 | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  |  |  | 16.5 |  |  |  | 5.0 |  |  |  | 2 |  | 25.0 + TT |  | 11.5 - TT |  |  |  |
| 53 | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  |  |  | 16.5 |  |  |  | 5.0 |  |  |  | 2 |  | 25.0 + TT |  | 11.5 - TT |  |  |  |
| 54 |  | 23 |  | 0 |  | 6.5 |  | 12 |  | 0 |  |  |  | 11 |  |  |  | 6.0 |  |  |  | 2 |  | 25.0 + TT |  | 5.0 - TT |  |  |
| 55 |  | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  |  |  | 16.5 |  |  |  | 5.0 |  |  |  | 2 |  | 25.0 + TT |  | 11.5 - TT |  |  |
| 56 |  | 23 |  | 0 |  | 6.5 |  | 8 |  | 0 |  |  |  | 15 |  |  |  | 5.0 |  |  |  | 2 |  | 25.0 + TT |  | 10.0 – TT |  |  |
| 57 |  | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  |  |  | 16.5 |  |  |  | 5.0 |  |  |  | 2 |  | 25.0 + TT |  | 11.5 - TT |  |  |
| 58 |  | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  |  |  | 16.5 |  |  |  | 5.0 |  |  |  | 2 |  | 25.0 + TT |  | 11.5 - TT |  |  |
| 59 | 23 |  | 0 |  | 6.5 |  | 12 |  | 0 |  |  |  | 11 |  |  |  | 6.0 |  |  |  | 2 |  | 25.0 + TT |  | 5.0 – TT |  |  |  |
| 60 | 23 |  | 0 |  | 6.5 |  | 4..5 |  | 0 |  |  |  | 16.5 |  |  |  | 5.0 |  |  |  | 2 |  | 25.0 + TT |  | 11.5 – TT |  |  |  |
| 61 | 23 |  | 0 |  | 6.5 |  | 8 |  | 0 |  |  |  | 15 |  |  |  | 5.0 |  |  |  | 2 |  | 25.0 + TT |  | 10.0 – TT |  |  |  |
| 62 | 23 |  | 0 |  | 6.5 |  | 3.5 |  | 0 |  |  |  | 16.5 |  |  |  | 5.0 |  |  |  | 2 |  | 25.0 + TT |  | 11.5 – TT |  |  |  |
| 63 | 23 |  | 0 |  | 6.5 |  | 5 |  | 0 |  |  |  | 16.5 |  |  |  | 5.0 |  |  |  | 2 |  | 25.0 + TT |  | 11.5 - TT |  |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Void NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

### 6.2.4	Configured transmitted power

Editor’s note: Message contents for R18 coverage enhancement are FFS due to the relative IEs are not introduced into TS 38.331 yet.

6.2.4.1	Test purpose

To verify the measured UE configured maximum output power PUMAX,f,c is within the specified bounds.

6.2.4.2	Test applicability

This test case applies to all types of NR Power Class 1 release 15 and forward.

This test case applies to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity.

6.2.4.3	Minimum conformance requirements

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤ PCMAX,f,c ≤ PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc) + ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c, PPowerClass – ΔPPowerClass  + ΔPPowerBoost}

where

PEMAX,c is the value given by either the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE, whichever is applicable according to TS 38.331[7];

PPowerClass is the maximum UE power specified in Table 6.2.1.3-1 without taking into account the tolerance specified in the Table 6.2.1.3-1;

When the IE powerBoostPi2BPSK is set to 1, PEMAX,c is increased by +3 dB for a power class 3 UE operating in TDD bands n40, n41, n77, n78, and n79 with PI/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and 40% or less slots in radio frame are used for UL transmission when PEMAX,c ≥ 20 dBm (The exact evaluation period is no less than one radio frame).

When the IE powerBoostPi2BPSK is set to 1, ΔPPowerClass = -3 dB for a power class 3 UE operating in TDD bands n40, n41, n77, n78, and n79 with PI/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and 40% or less slots in radio frame are used for UL transmission.

ΔPPowerClass =

-	3 dB for a power class 2 UE or 6 dB for a power class 1.5 UE when P-max of 23 dBm or lower is indicated; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame).

-	3 dB for a power class 1.5 UE when P-max of between 23 dBm and 26 dB is indicated; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is between 25% and 50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is between maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC2-FR1/2 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame).

-	3dB when the UE is configured with SUL configurations and the requirements of default power class are applied as specified in sub-clause 6.2C.1.3 on the band where UE indicates power class 2;

-	3dB is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ with configured SRS resources in each SRS resource set(s) consisting of one SRS port when PC2 UE with txDiversity-r16 capability or PC1.5 UE further indicates SRS-TxSwitch capability ‘t1r2’ or ‘t1r4’ or ‘t1r1-t1r2’ or ‘t1r1-t1r2-t1r4’ or further indicates srs-AntennaSwitchingBeyond4RX-r17 as ‘t1r8’;

-	0 dB otherwise;

NOTE:	UE reports ∆PPowerClass when deltaPowerClassReporting-r18 is present, dpc-Reporting-FR1 is configured and the reporting is triggered only by uplink duty cycle exceedance or by return to the ue-PowerClass after the duty cycle exceedance.

∆TIB,c is the additional tolerance for serving cell c as specified in clause 6.2A.4.0.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 [4] clause  6.2B.4.2 for EN-DC; ∆TIB,c = 0 dB otherwise; In case the UE supports more than one of band combinations for V2X operating bands for concurrent operation, CA, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ∆TIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2 for NR CA, clause 6.2C.2 for SUL in this specification and 6.2B.4.2 in TS 38.101-3 [4] for EN-DC, truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ∆TIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ∆TIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2 for NR CA, clause 6.2C.2 for SUL in this specification and 6.2B.4.2 in TS 38.101-3 [4] for EN-DC, for the applicable operating bands.

ΔPPowerBoost is defined as 1dB for power class 3 and 0.5dB for power class 2, when all of the following conditions are met:

-	If the UE indicates support for UE capability powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18, and if IE powerBoostPi2BPSK-r18 and/or powerBoostQPSK-r18 is set to 1 and PEMAX,c, if configured, is increased by at least ΔPPowerBoost

-	If UE indicates power class 2 in a TDD band or power class 3

-	IfΔPPowerClass is 0dB

-	If scheduled UL transmission is DFT-s-OFDM with either PI/2 BPSK modulation or QPSK modulation

-	If the RB allocation belongs to the inner region defined in clause 6.2.2.

-	If UE indicates power class 3, the percentage of uplink symbols transmitted in a certain evaluation period is less than 80%

-	If UE indicates power class 2, when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is less than 0.9*50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is less than 0.9*maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is less than 0.9*maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame)

-	0 dB otherwise;

∆TC,c = 1.5dB when NOTE 3 in Table 6.2.1.3-1 in 38.521-1 applies for a serving cell c, otherwise ∆TC,c = 0 dB ;

MPRc and A-MPRc for serving cell c are specified in clause 6.2.2.3 and clause 6.2.3.3, respectively;

∆MPRc for serving cell c is specified in clause 6.2.2.3.

∆TRxSRS is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ when

a)	UE transmits SRS on the second SRS resource in every configured SRS resource set when the SRS-TxSwitch capability is indicated as 't1r2' or 't1r1-t1r2'

b)	UE transmits SRS on the second, third and fourth SRS resources of the total 4 SRS resources from all configured SRS resource set(s) consisting of one SRS port when the SRS-TxSwitch capability is indicated as 't1r4' or, 't1r4-t2r4' or 't1r1-t1r2-t1r4' or, 't1r1-t1r2-t2r2-t1r4-t2r4'

c)	UE transmits SRS from the second SRS port pair on the second SRS resource in every configured SRS resource set consisting of two SRS ports when the SRS-TxSwitch capability is indicated as ' t2r4' or ' t1r4-t2r4', or 't1r1-t1r2-t2r2-t2r4' or 't1r1-t1r2-t2r2-t1r4-t2r4', or

d)	UE transmits SRS to a DL-only carrier

e)	UE transmits SRS on the second, third, fourth, fifth, sixth, seventh and eighth SRS resources of the total 8 SRS resources from all configured SRS resource set(s) consisting of one SRS port when the srs-AntennaSwitchingBeyond4RX-r17 capability is indicated as at least 't1r8', or

f)	UE transmits SRS from the SRS port pair on the second, third and fourth SRS resource in every configured SRS resource set consisting of two SRS ports when the srs-AntennaSwitchingBeyond4RX-r17 capability is indicated as at least 't2r8', or

g) 	UE transmits SRS from the set of SRS ports on the second SRS resource in every configured SRS resource set consisting of four SRS ports when the srs-AntennaSwitchingBeyond4RX-r17 capability is indicated as at least 't4r8',

The following ∆TRxSRS applies according to the indicated SRS-TxSwitch or srs-AntennaSwitchingBeyond4RX-r17 capabilities:

if  't1r8' and 't4r8' are indicated:

-	The value of ∆TRxSRS is 7.3 dB for bands whose FUL_high is higher than the FUL_low of n79 and 5.8 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

-	The value of ∆TRxSRS is 10.3 dB for bands whose FUL_high is higher than the FUL_low of n79 and 8.8 dB for bands whose FUL_high is lower than the FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

else, if 't1r8' and 't2r8' are indicated:

-	The value of ∆TRxSRS is 6.0 dB for bands whose FUL_high is higher than the FUL_low of n79 and 4.5 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating txDiversity-r16.

-	The value of ∆TRxSRS is 9.0 dB for bands whose FUL_high is higher than the FUL_low of n79 and 7.5 dB for bands whose FUL_high is lower than the FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

else, if 't1r8' or 't2r8', but not both is indicated:

-	The value of ∆TRxSRS is 5.5 dB for bands whose FUL_high is higher than the FUL_low of n79 and 4.0 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

-	The value of ∆TRxSRS is 8.5 dB for bands whose FUL_high is higher than the FUL_low of n79 and 7.0 dB for bands whose FUL_high is lower than the FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

else, if  't1r2', 't1r1-t1r2', 't1r4', 't1r4-t2r4', 't1r1-t1r2-t1r4', 't2r4', 't1r1-t1r2-t2r2-t2r4', 't1r1-t1r2-t2r2-t1r4-t2r4' or 't4r8' is indicated:

-	The value of ∆TRxSRS is 4.5dB for bands whose FUL_high is higher than the FUL_low of n79 and 3 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

-	The value of ∆TRxSRS is 7.5dB for bands whose FUL_high is higher than the FUL_low of n79 and 6 dB for bands whose FUL_high is lower than the FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

The value of ∆TRxSRS is 4.5dB for bands whose FUL_high is higher than the FUL_low of n79 and 3 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is capable of power class 3 or power class 5 or power class 1.5 in the band, or when the device is capable of power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating txDiversity-r16..

The value of ∆TRxSRS is 7.5dB for bands whose FUL_high is higher than the FUL_low of n79 and 6 dB for bands whose FUL_high is lower than the FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is capable of power class 2 in the band and ΔPPowerClass = 0 dB and not indicating txDiversity-r16.

For other SRS transmissions ∆TRxSRS is zero;

P-MPRc is the allowed maximum output power reduction for

a)	ensuring compliance with applicable electromagnetic energy absorption requirements and addressing unwanted emissions / self defence requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications;

b)	ensuring compliance with applicable electromagnetic energy absorption requirements in case of proximity detection is used to address such requirements that require a lower maximum output power.

The UE shall apply P-MPRc for serving cell c only for the above cases. For UE conducted conformance testing P-MPRc shall be 0 dB

NOTE 1:	P-MPRc was introduced in the PCMAX,f,c equation such that the UE can report to the eNB the available maximum output transmit power. This information can be used by the eNB for scheduling decisions.

NOTE 2:	P-MPRc may impact the maximum uplink performance for the selected UL transmission path.

TREF and Teval are specified in Table 6.2.4.3-1. For each TREF, the PCMAX,L,c for serving cell c are evaluated per Teval and given by the minimum value taken over the transmission(s) within the Teval; the minimum PCMAX_L,f,c over one or more Teval is then applied for the entire TREF

Table 6.2.4.3-1: Evaluation and reference periods for Pcmax

| TREF | Teval | Teval with frequency hopping |
| --- | --- | --- |
| Physical channel length | Physical channel length | Min(Tno_hopping, Physical Channel Length) |

The measured configured maximum output power PUMAX,f,c shall be within the following bounds:

PCMAX_L,f,c – MAX{TL,c, T(PCMAX_L,f,c)} ≤ PUMAX,f,c ≤ PCMAX_H,f,c + T(PCMAX_H,f,c).

where the tolerance T(PCMAX,f,c) for applicable values of PCMAX,f,c is specified in Table 6.2.4.3-2. The tolerance TL,c is the absolute value of the lower tolerance for the applicable operating band as specified in Table 6.2.1.3-1.

Table 6.2.4.3-2: PCMAX tolerance

| PCMAX,f,c (dBm) | Tolerance T(PCMAX,f,c) (dB) |
| --- | --- |
| 23 < PCMAX,c ≤ 33 | 2.0 |
| 21 ≤ PCMAX,c ≤ 23 | 2.0 |
| 20 ≤ PCMAX,c < 21 | 2.5 |
| 19 ≤ PCMAX,c < 20 | 3.5 |
| 18 ≤ PCMAX,c < 19 | 4.0 |
| 13 ≤ PCMAX,c < 18 | 5.0 |
| 8 ≤ PCMAX,c < 13 | 6.0 |
| -40 ≤ PCMAX,c < 8 | 7.0 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.4.

6.2.4.4	Test description

6.2.4.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2.4.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.4.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range (NOTE 4) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM QPSK | Inner Full |
| 33 |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: A power class 3 UE operating in TDD mode with PI/2 PBSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. NOTE 4: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2.4.4.3.

6.2.4.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.4.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step to ensure that the UE reaches the Pumax level of the test point.

3.	Measure the mean power of the UE in the channel bandwidth for each test point in table 6.2.4.5-1 according to the test configuration from table 6.2.4.4.1-1. The period of measurement shall be at least the continuous duration of one active slot and in the uplink symbols. For TDD slots with transient periods are not under test.

6.2.4.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with the following exceptions:

Table 6.2.4.4.3-0: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.2.4.4.3-1: FrequencyInfoUL-SIB: Test point 1

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | -10 |  |  |

Table 6.2.4.4.3-2: FrequencyInfoUL-SIB: Test point 2

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 10 |  |  |

Table 6.2.4.4.3-3: FrequencyInfoUL-SIB: Test point 3

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 15 |  |  |

Table 6.2.4.4.3-4: FrequencyInfoUL-SIB: Test point 4

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  |  |

Table 6.2.4.4.3-5: ServingCellConfig

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| powerBoostPi2BPSK | 0 |  | Test ID 1, 2 |
|  | 1 |  | Test ID 3 |
| } |  |  |  |
| } |  |  |  |

6.2.4.5	Test requirement

The maximum output power measured shall not exceed the values specified in Table 6.2.4.5-1.

Table 6.2.4.5-1: PCMAX configured UE output power

|  | Maximum output power |  |
| --- | --- | --- |
|  | Test ID 1,2 | Test ID 3 |
| Measured UE output power test point 1 | -10 dBm ± (7+TT) | -10 dBm ± (7+TT) |
| Measured UE output power test point 2 | 10 dBm ± (6+TT) | 10 dBm ± (6+TT) |
| Measured UE output power test point 3 | 15 dBm ± (5+TT) | 15 dBm ± (5+TT) |
| Measured UE output power test point 4 | Note 3 | Note 4 |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 6.2.4.5-2. Note 2: Power class 3 is default power class unless otherwise stated. Note 3: The maximum output power shall be within the range in Table 6.2.4.5-1a. Note 4: The maximum output power shall be within the range in Table 6.2.4.5-1b. |  |  |

Table 6.2.4.5-1a: Measured UE output power test point 4 for Test ID 1,2

| NR band | Tolerance (dB) |
| --- | --- |
| n1 | 20 dBm ±(2.5+TT) |
| n2 | 20 dBm ±(2.5+TT) |
| n3 | 20 dBm ±(2.5+TT) |
| n5 | 20 dBm ±(2.5+TT) |
| n7 | 20 dBm ±(2.5+TT) |
| n8 | 20 dBm ±(2.5+TT) |
| n12 | 20 dBm ±(2.5+TT) |
| n14 | 20 dBm ±(2.5+TT) |
| n20 | 20 dBm ±(2.5+TT) |
| n24 | 20 dBm + 2.5+TT/-3-TT |
| n25 | 20 dBm ±(2.5+TT) |
| n26 | 20 dBm ±(2.51+TT) |
| n28 | 20 dBm ±(2.5+TT) |
| n30 | 20 dBm ±(2.5+TT) |
| n34 | 20 dBm ±(2.5+TT) |
| n38 | 20 dBm ±(2.5+TT) |
| n39 | 20 dBm ±(2.5+TT) |
| n40 | 20 dBm ±(2.5+TT) |
| n41 | 20 dBm ±(2.5+TT) |
| n50 | 20 dBm ±(2.5+TT) |
| n51 | 20 dBm ±(2.5+TT) |
| n54 | 20 dBm ±(2.5+TT) |
| n65 | 20 dBm ±(2.5+TT) |
| n66 | 20 dBm ±(2.5+TT) |
| n70 | 20 dBm ±(2.5+TT) |
| n71 | 20 dBm ±(2.5+TT) |
| n74 | 20 dBm ±(2.5+TT) |
| n77 | 20 dBm + 2.5+TT/-3-TT |
| n78 | 20 dBm + 2.5+TT/-3-TT |
| n79 | 20 dBm + 2.5+TT/-3-TT |
| n105 | 20 dBm ±(2.5+TT) |
| NOTE 1: Void NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.4.5-2. |  |

Table 6.2.4.5-1b: Measured UE output power test point 4 for Test ID 3

| NR band | Tolerance (dB) |
| --- | --- |
| n40 | 23 dBm +2+TT/-2-TT |
| n41 | 23 dBm +2+TT/-2-TT |
| n77 | 23 dBm +2+TT/-3-TT |
| n78 | 23 dBm +2+TT/-3-TT |
| n79 | 23 dBm +2+TT/-3-TT |
| NOTE 1: Void NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.4.5-2. |  |

Table 6.2.4.5-2: Test Tolerance (Configured transmitted power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB1 1.0 dB2 | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB |
| Note 1: Applicable for test point with UL power ≥ 0 dBm. Note 2: Applicable for test point with UL power < 0 dBm. |  |  |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

## 6.2A	Transmitter power for CA

### 6.2A.1	UE maximum output power for CA

#### 6.2A.1.0	Minimum conformance requirements

##### 6.2A.1.0.1	Void

##### 6.2A.1.0.2	Void

##### 6.2A.1.0.3	UE maximum output power for Inter-band CA

For inter-band downlink carrier aggregation with one uplink carrier assigned to one NR band, the transmitter power requirements in Table 6.2.1.3-1 apply for power class 3 and other power classes if indicated in clause 5.5A.3.

For inter-band carrier aggregation with two uplink contiguous carrier assigned to one NR band, the transmitter power requirements specified in clause 6.2A.1.0.4 apply.

For inter-band carrier aggregation with two uplink non-contiguous carrier assigned to one NR band, the transmitter power requirements specified in clause 6.2A.1.0.5 apply.

For inter-band uplink carrier aggregation with uplink assigned to two NR bands, UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is defined as the sum of maximum output power from each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). The maximum output power is specified in Table 6.2A.1.0.3-1.

Table 6.2A.1.0.3-1: UE Power Class for uplink inter-band CA (two bands)

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n5A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n8A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n28A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n41A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n1A-n79A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n5A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n14A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n30A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n3A-n5A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n8A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n28A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n40A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n3A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n5A-n7A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n30A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n5A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n8A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n14A-n30A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n14A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n14A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n20A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n24A-n41A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n24A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n24A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n25A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n25A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n25A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n26A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n26A-n70A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n28A-n40A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n28A-n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n28A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n28A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n28A-n79A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n30A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n30A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n39A-n41A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n40A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n41A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n41A-n79A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n48A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n48A-n70A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n48A-n71A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n66A-n71A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n66A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n66A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n70A-n71A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n71A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n71A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: Void. NOTE 2: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1.3-1 is allowed to reduce the lower tolerance limit by 1.5dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: Power class 3 is the default power class unless otherwise stated. NOTE 6: The UE supports PC3 within NR FDD band, and supports either PC3 or PC2 within NR TDD band. NOTE 7: The UE that supports a PC2 uplink CA configuration with single carrier for each individual band and a composite of supporting PC3 within an NR TDD or FDD band and PC2 within a second NR TDD band may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise. |  |  |  |  |  |  |  |  |

If a UE supports a different power class than the default UE power class for the band combination listed in Table 6.2A.1.0.3-1 and the supported power class enables the higher maximum output power than that of the default power class:

–	if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is not absent and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

–	if the IE P-Max as defined in TS 38.331 [7] is provided and set to the maximum output power of the default power class or lower;

–	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2A.4;

–	else;

–	shall apply all requirements for the supported power class and set the configured transmitted power as specified in clause 6.2A.4 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent).

The average percentage of uplink symbols is defined as 50%  ( DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band as defined in TS 38.331.  For NR Band x or NR Band y,

–	if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%;

–	else if the band is configured with power class 3;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2A.1.3.

Table 6.2A.1.0.3-2: Void

##### 6.2A.1.0.4	UE maximum output power for Intra-band contiguous CA

For uplink intra-band contiguous carrier aggregation, the maximum output power is specified in Table 6.2A.1.0.4-1.

For downlink intra-band contiguous carrier aggregation with a single uplink component carrier configured in the NR band, the maximum output power is specified in Table 6.2.1.3-1 for power class 3 and other power classes if indicated in clause 5.5A.1.

Table 6.2A.1.0.4-1: UE Power Class for intra-band contiguous CA

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41C |  |  | 26 | +2/-3 | 23 | +2/-2 |  |  |
| CA_n48B |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n77C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n78C |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n79C |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: An uplink CA configuration in which the band has NOTE 3 in Table 6.2.1.3-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of the band are confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 3: For intra-band contiguous carrier aggregation the maximum power requirement shall apply to the total transmitted power over all component carriers (per UE). |  |  |  |  |  |  |  |  |

##### 6.2A.1.0.5	UE maximum output power for Intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation with one uplink carrier on the PCC, the requirements in clause 6.2.1 apply for power class 3 and other power classes if indicated in clause 5.5A.2.

For intra-band non-contiguous carrier aggregation with two uplink carriers the maximum output power is specified in Table 6.2A.1.0.5-1.

Table 6.2A.1.0.5-1: UE Power Class for intraband non-contiguous CA

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n77(2A) |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n78(2A) |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: An uplink CA configuration in which the band has NOTE 3 in Table 6.2.1.3-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of the band are confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 3: For intra-band non-contiguous carrier aggregation the maximum power requirement shall apply to the total transmitted power over all component carriers (per UE). |  |  |  |  |  |  |  |  |

#### 6.2A.1.1	UE maximum output power for CA (2UL CA)

Editor’s Note:

No test points are defined for intra-band contiguous UL CA and intra-band non-contiguous UL CA since there is no configuration satisfying MPR=0dB requirements in TS 38.101-1. Testing with lowest value of MPR will be covered in 6.2A.2.

6.2A.1.1.1	Test purpose

To verify that the error of the UE maximum output power in two uplink carrier aggregation does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2A.1.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support NR 2UL CA.

NOTE: Testing for intra-band contiguous and non-contiguous CA can’t be performed due to lack of appropriate test points.

6.2A.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2A.1.0.

6.2A.1.1.4	Test description

6.2A.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in 5.5A. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2A.1.1.4.1-1 and table 6.2A.1.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2A.1.1.4.1-1: Inter-band CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  | Modulation for all CCs | RB allocation (NOTE 1) |  |
|  |  | (NOTE 2) | PCC | SCC |
| 1 | N/A for this test | DFT-s-OFDM Pi/2 BPSK | Inner Full | Inner Full |
| 2 |  | DFT-s-OFDM Pi/2 BPSK | Inner 1RB Left | Inner 1RB Left |
| 3 |  | DFT-s-OFDM Pi/2 BPSK | Inner 1RB Right | Inner 1RB Right |
| 4 |  | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 5 |  | DFT-s-OFDM QPSK | Inner 1RB Left | Inner 1RB Left |
| 6 |  | DFT-s-OFDM QPSK | Inner 1RB Right | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |

Table 6.2A.1.1.4.1-2: Intra-band contiguous CA Test Configuration Table

NOTE:	No test points are defined since there is no configuration satisfying MPR=0dB requirements in TS 38.101-1.

Table 6.2A.1.1.4.1-3: Intra-band non-contiguous CA Test Configuration Table

NOTE:	No test points are defined since there is no configuration satisfying MPR=0dB requirements in RAN4.

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2A.1.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2A.1.1.4.3.

6.2A.1.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.2A.1.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2A.1.1.4.1-1 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

6.	Measure the sum of mean transmitted power over all component carriers in the CA configuration of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbols with transient periods are not under test. For FDD band in inter-band CA with both TDD band and FDD band, only slots overlapping with only UL symbols in TDD are under test.

6.2A.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions.

Table 6.2A.1.1.4.3-1: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.2A.1.1.4.3-2: FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  | Power class 3 and Inter-band CA |
|  | 23 |  | Power class 2 and Inter-band CA |
|  | Not Present |  | Value applicable to both carriers in case of inter-band CA when UE supports higherPowerLimit-r17 and the associated conditions apply as defined in Note 8 in table 6.2A.1.1.5-1a. |

6.2A.1.1.5	Test requirement

The maximum output power for CA, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.1.1.5-1 for Inter-band 2 UL CA configuration. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 6.2A.1.1.5-1: Maximum Output Power test requirement for Power Class 3

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n1A-n5A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n1A-n8A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n1A-n28A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n1A-n41A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n1A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n1A-n79A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n2A-n5A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n2A-n14A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n2A-n30A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n2A-n48A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n2A-n66A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n2A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n3A-n5A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n3A-n8A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n3A-n28A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n3A-n40A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n3A-n41A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n3A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n3A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n5A-n7A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n5A-n30A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n5A-n48A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n5A-n66A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n5A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n5A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n7A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n8A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n14A-n30A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n14A-n66A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n14A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n20A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n24A-n41A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n24A-n48A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n24A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n25A-n66A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n25A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n25A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n26A-n66A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n26A-n70A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n28A-n40A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n28A-n41A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n28A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n28A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n28A-n79A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n30A-n66A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n30A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n39A-n41A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n40A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n41A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n41A-n79A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n48A-n66A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n48A-n70A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n48A-n71A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n66A-n71A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n66A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n66A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n70A-n71A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n71A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n71A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| NOTE 1: Void. NOTE 2: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1.3-1 is allowed to reduce the lower tolerance limit by 1.5dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: Power class 3 is the default power class unless otherwise stated. NOTE 6: The TT for 2UL CA Maximum Output Power is in the maximum TT among all UL CCs. For TT of each UL CC refers to Table 6.2A.1.1.5-2. |  |  |  |  |  |  |  |  |

Table 6.2A.1.1.5-1a: Maximum Output Power test requirement for Power Class except class 3

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n78A |  |  | 267, 27.88 | +2+TT/-32-TT |  |  |  |  |
| CA_n2A-n77A |  |  | 267, 27.88 | +2+TT/-32-TT |  |  |  |  |
| CA_n3A-n41A |  |  | 267, 27.88 | +2+TT/-32-TT |  |  |  |  |
| CA_n3A-n78A |  |  | 267, 27.88 | +2+TT/-32-TT |  |  |  |  |
| CA_n5A-n77A |  |  | 267, 27.88 | +2+TT/-32-TT |  |  |  |  |
| CA_n14A-n77A |  |  | 267, 27.88 | +2+TT/-32-TT |  |  |  |  |
| CA_n28A-n41A |  |  | 267, 27.88 | +2+TT/-32-TT |  |  |  |  |
| CA_n28A-n79A |  |  | 267, 27.88 | +2+TT/-32-TT |  |  |  |  |
| CA_n30A-n77A |  |  | 267, 27.88 | +2+TT/-32-TT |  |  |  |  |
| CA_n41A-n79A |  |  | 267, 27.88 | +2+TT/-32-TT |  |  |  |  |
| CA_n66A-n77A |  |  | 267, 27.88 | +2+TT/-32-TT |  |  |  |  |
| NOTE 1: Void. NOTE 2: It is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of the band are confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: Power class 3 is the default power class unless otherwise stated. NOTE 6: The TT for 2UL CA Maximum Output Power is in the maximum TT among all UL CCs. For TT of each UL CC refer to Table6.2A.1.1.5-2. NOTE 7: The UE supports PC3 within NR FDD band, and supports either PC3 or PC2 within NR TDD band. NOTE 8: The UE that supports a PC2 uplink CA configuration with single carrier for each individual band and a composite of supporting PC3 within an NR TDD or FDD band and PC2 within a second NR TDD band and indicates support of higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise. |  |  |  |  |  |  |  |  |

Table 6.2A.1.1.5-2: Test Tolerance (UE maximum output power for CA)

| TT for overall output power (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |
|  |  |  | BW ≤ 40MHz |  | 40MHz < BW ≤ 100MHz |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
|  | BW ≤ 40MHz | f ≤ 3.0GHz | 0.7 | 1.0 | 1.0 | 1.0 |
| SCell |  | 3.0GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

### 6.2A.2	UE maximum output power reduction for CA

#### 6.2A.2.0	Minimum conformance requirements

##### 6.2A.2.0.1	FFS

##### 6.2A.2.0.2	FFS

##### 6.2A.2.0.3	Maximum Power Reduction for Inter-band CA

For inter-band carrier aggregation with one uplink carrier assigned to one NR band, the requirements in clause 6.2.2.3 apply.

For inter-band carrier aggregation with two uplink contiguous carrier assigned to one NR band, the maximum output power reduction requirements for intra-band contiguous carrier aggregation in clause 6.2A.2.0.4 apply for that band.

For inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in subclause 6.2.2.3 apply for each uplink component carrier.

The normative reference for this requirement is TS38.101-1[2] clause 6.2A.2.

##### 6.2A.2.0.4	Maximum Power Reduction for Intra-band contiguous CA

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2A.1.0.4-1 with contiguous RB allocation is specified in Table 6.2A.2.0.4-1 for UE power class 3 CA bandwidth classes B and C. The MPR with contiguous RB allocation is specified in Table 6.2A.2.0.4-1a for power class 2 CA bandwidth classes B and C when the signalling is absent for dualPA-Architecture IE, and for power class 2 CA bandwidth class C when the signalling is indicated for dualPA-Architecture IE. The MPR with contiguous RB allocation is specified in Table 6.2A.2.0.4-1b for power class 2 CA bandwidth classes B and C with TxD supported.

In case the modulation format or waveform type is different on different component carriers then the requirement is set by rules applied to the waveform type (DFT-s-OFDM or CP-OFDM) and modulation order used in the configuration with the largest MPR.

Unless otherwise specified, pi/2 BPSK in following MPR tables refers to both variants of pi/2 BPSK referenced in clause 6.2.2.3, Table 6.2.2.3-1.

Table 6.2A.2.0.4-1: Contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | QPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | 16QAM | 1.5 | 3.5 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.0 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 2.0 | 4.0 | 3.5 | 8 |
|  | 16QAM | 2.5 | 4.0 | 3.5 | 8 |
|  | 64QAM | 3.5 | 4.0 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |

Table 6.2A.2.0.4-1a: Contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 2.0 | 4.01 | 2.5 | 7 |
|  | QPSK | 2.0 | 4.01 | 2.5 | 7 |
|  | 16QAM | 2.5 | 4.01 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.51 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 2.5 | 5.01 | 3.5 | 8 |
|  | 16QAM | 3.0 | 5.01 | 3.5 | 8 |
|  | 64QAM | 3.5 | 5.01 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |
| NOTE 1 When 1 RB or 2 RB are allocated at the lower edge of lowest CC or upper edge of upper CC, MPR for outer is 5.5 dB. |  |  |  |  |  |

Table 6.2A.2.0.4-1b: Contiguous RB allocation for Power Class 2 with dual Tx2

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 3.0 | 5.01 | 3.5 | 8 |
|  | QPSK | 3.0 | 5.01 | 3.5 | 8 |
|  | 16QAM | 3.5 | 5.01 | 3.5 | 8 |
|  | 64QAM | 4.0 | 5.51 | 6 | 8 |
|  | 256QAM | 6.5 | 7.0 | 8 | 8.5 |
| CP-OFDM | QPSK | 3.0 | 5.51 | 4.0 | 8.5 |
|  | 16QAM | 3.5 | 5.51 | 4.0 | 8.5 |
|  | 64QAM | 4.0 | 5.51 | 5.5 | 8.5 |
|  | 256QAM | 7.0 | 7.0 | 7.5 | 8.5 |
| NOTE 1: When 1 RB or 2 RB are allocated at the lower edge of lowest CC or upper edge of upper CC, MPR for outer is 5.5 dB. NOTE 2: UE indicating TxD supported |  |  |  |  |  |

For CA bandwidth class B and bandwidth class C with contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner and Outer RB allocations:

An RB allocation is contiguous if LCRB1 = 0 or LCRB2 = 0 or (LCRB1  0 and LCRB2  0 and RBStart1 + LCRB1 = NRB1 and RBStart2 = 0), where RBStart1, LCRB1, and NRB1 are for CC1, RBStart2, LCRB2, and NRB2 are for CC2, CC1 is the component carrier with lower frequency.

In contiguous CA, a contiguous allocation is an inner allocation if

RBStart,Low ≤ RBStart_CA ≤ RBStart,High, and NRB_alloc ≤ ceil(NRB,agg /2),

where

RBStart,Low = max(1, floor(NRB_alloc /2)),

RBStart,High = NRB,agg – RBStart,Low – NRB,alloc,

with

NRB_alloc= (NRB1 - RBStart1)∙ 2^µ1 + (RBStart2 + LCRB2 ) ∙ 2^µ2,

NRB,agg=NRB1∙2^µ1+ NRB2∙2^µ2.

If LCRB1 =0, RBStart_CA = NRB1∙2^µ1+ RBStart2∙2^µ2,

if LCRB1 > 0, RBStart_CA = RBStart1∙2^µ1.

A contiguous allocation that is not an Inner contiguous allocation is an Outer contiguous allocation

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2A.1.0.4-1 with non-contiguous RB allocation is specified in Table 6.2A.2.0.4-2 for UE power class 3 CA bandwidth classes B and C. The MPR with non-contiguous RB allocation is specified in Table 6.2A.2.0.4-3 for power class 2 CA bandwidth classes B and C when the signalling is absent for dualPA-Architecture IE, and for power class 2 CA bandwidth class C when the signalling is indicated for dualPA-Architecture IE. The MPR with non-contiguous RB allocation is specified in Table 6.2A.2.0.4-4 for power class 2 CA bandwidth classes B and C with TxD supported.

Table 6.2A.2.0.4-2: non-contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 |
| DFT-s-OFDM | Pi/2 BPSK | 2 | 5.5 | 11.5 | 2.5 | 6 | 13 |
|  | QPSK | 2 | 5.5 |  | 2.5 | 6 |  |
|  | 16QAM | 2.5 | 5.5 |  | 3 | 6 |  |
|  | 64QAM | 4.5 | 6 |  | 5 | 6 |  |
|  | 256QAM | 6 | 6.5 |  | 6.5 | 6.5 |  |
| CP-OFDM | QPSK | 2.5 | 6.5 | 12 | 3.5 | 7 | 14 |
|  | 16QAM | 3 | 7 |  | 3.5 | 7 |  |
|  | 64QAM | 5 | 7 |  | 5 | 7 |  |
|  | 256QAM | 7.5 | 7.5 |  | 7.5 | 7.5 |  |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz |  |  |  |  |  |  |  |

Table 6.2A.2.0.4-3: non-contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer12 | Outer23 | Inner | Outer12 | Outer23 |
| DFT-s-OFDM | Pi/2 BPSK | 31 | 6.5 | 13 | 31 | 7.5 | 13.5 |
|  | QPSK | 31 | 6.5 |  | 31 | 7.5 |  |
|  | 16QAM | 31 | 6.5 |  | 31 | 7.5 |  |
|  | 64QAM | 5 | 6.5 |  | 5 | 7.5 |  |
|  | 256QAM | 6.5 | 7 |  | 6.5 | 7.5 |  |
| CP-OFDM | QPSK | 3.51 | 7 | 14 | 3.51 | 8 | 14.5 |
|  | 16QAM | 3.51 | 7 |  | 3.51 | 8 |  |
|  | 64QAM | 5 | 7 |  | 5 | 8 |  |
|  | 256QAM | 7.5 | 7.5 |  | 7.5 | 8 |  |
| NOTE 1: The allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz |  |  |  |  |  |  |  |

Table 6.2A.2.0.4-4: non-contiguous RB allocation for Power Class 2 with dual Tx4

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer12 | Outer23 | Inner | Outer12 | Outer23 |
| DFT-s-OFDM | Pi/2 BPSK | 41 | 7.5 | 14 | 41 | 8.5 | 14.5 |
|  | QPSK | 41 | 7.5 |  | 41 | 8.5 |  |
|  | 16QAM | 41 | 7.5 |  | 41 | 8.5 |  |
|  | 64QAM | 6 | 7.5 |  | 6 | 8.5 |  |
|  | 256QAM | 7.5 | 8 |  | 7.5 | 8.5 |  |
| CP-OFDM | QPSK | 4.51 | 8 | 15 | 4.51 | 9 | 15.5 |
|  | 16QAM | 4.51 | 8 |  | 4.51 | 9 |  |
|  | 64QAM | 6 | 8 |  | 6 | 9 |  |
|  | 256QAM | 8.5 | 8.5 |  | 8.5 | 9 |  |
| NOTE 1: The allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 4: UE indicating TxD supported |  |  |  |  |  |  |  |

For CA bandwidth classes B and C with non-contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner, Outer1 and Outer2 RB allocations:

Non-Contiguous RB allocation is defined as RBStart1 + LCRB1 < NRB1, or RBStart2 > 0 when all uplink CCs are activated and allocated with RB(s), where RBStart1, LCRB1, and NRB1 are for CC1, RBStart2, LCRB2, and NRB2 are for CC2, CC1 is the component carrier with lower frequency.

In contiguous CA, a non-contiguous RB allocation is a non-contiguous Inner RB allocation if the following conditions are met:

RBStart,Low ≤ RBStart_CA ≤ RBStart,High and NRB_alloc ≤ ceil((BWChannel_CA / 3 – BWgap ) / 0.18MHz),

where

NRB_alloc = (NRB1 - RBStart1)∙ 2^µ1 + (RBStart2 + LCRB2 ) ∙ 2^µ2, RBStart_CA = RBStart1∙2^1

RBStart,Low = max(1, floor(NRB_alloc + (BWgap – BWGB,low)/0.18MHz))

RBStart,High = floor((BWChannel_CA – 2 ∙ BWgap – BWGB,low)/0.18MHz – 2 ∙ NRB_alloc)

BWGB,low =Foffset,low – (NRB1∙12+1)∙SCS1/2

BWgap is the bandwidth of the gap between NRB1 and NRB2 possible allocations of CC1 and CC2 respectively.

In contiguous CA, a non-contiguous RB allocation is a non-contiguous outer 1 RB allocation if the following conditions are met:

RBStart,Low ≤ RBStart_CA ≤ RBStart,High and NRB_alloc ≤ ceil((3 BWChannel_CA / 5 – BWgap) / 0.18MHz)

where

RBStart,Low = max(1, 2 ∙ NRB_alloc – floor( (BWChannel_CA – 2 ∙ BWgap + BWGB,low)/0.18MHz)),

RBStart,High = floor((2 ∙ BWChannel_CA – 3 ∙ BWgap – BWGB,low) / 0.18MHz – 3 ∙ NRB_alloc)

NRB_alloc, RBStart_CA, BWgap and BWGB,low are as defined for the Inner region.

In contiguous CA, a non-contiguous allocation is an Outer 2 allocation if it is neither a non-contiguous Inner allocation nor an Outer 1 allocation.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2A.2.1.

##### 6.2A.2.0.5	Maximum Power Reduction for Intra-band non-contiguous CA

For intra-band non-contiguous CA, the allowed Maximum Power Reduction (MPR) for the maximum output power is specified into 2 types: MPR to meet -30dBm/MHz and -13dBm/MHz. The UE determines the MPR type as follows:

For intra-band non-contiguous CA, the allowed Maximum Power Reduction (MPR) for the maximum output power is specified into 2 types: MPR to meet -30dBm/MHz and -13dBm/MHz. The UE determines the MPR type as follows:

If AND( FIM3,low_block,low > SEM-13,low ,  FIM3,high_block,high < SEM-13,high )

MPRIM3 defined in Clause 6.2A.2.0.5.2

Else If AND( FIM3,low_block,low > SEM-25,low ,  FIM3,high_block,high < SEM-25,high )

MPRIM3 defined in Clause 6.2A.2.0.5.1

Else

MPRIM3 defined in Clause 6.2A.2.0.5.1

where

-	FIM3,high_block,high = (2 * Fhigh_alloc,high_edge ) – Flow_alloc,low_edge

-	FIM3,low_block,low = (2 * Flow_alloc,low_edge) – Fhigh_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of the lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of the lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of the upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of the upper transmission bandwidth allocation.

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.2.0

-	SEM-13,high = Threshold frequency where upper spectral emission mask above the upper channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.2.0.

-	SEM-25,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -25 dBm / MHz to -30 dBm / MHz, as specified in Clause 6.5A.2.2.0.

-	SEM-25,high = Threshold frequency where upper spectral emission mask above the upper channel drops from -25 dBm / MHz to -30 dBm / MHz, as specified in Clause 6.5A.2.2.0.

6.2A.2.0.5.1	MPR to meet -30dBm/MHz

MPR in this clause is for intra-band non-contiguous CA power class 3 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MA

Where MA is defined as follows:

MA =	15;	0 ≤ B < 1.08

14.5;	1.08 ≤ B < 2.16

13.5;	2.16 ≤ B < 3.24

12.5;	3.24 ≤ B < 5.04

11.5;	5.04≤ B < 10.08

10.5;	10.08 ≤ B < 16.38

10;	16.38 ≤ B < 21.78

9;	21.78 ≤ B

Where:

B=(LCRB_alloc, 1* 12* SCS1 + LCRB_alloc,2 * 12 * SCS2)/1,000

6.2A.2.0.5.2	MPR to meet -13dBm/MHz

MPR in this clause is for intra-band non-contiguous CA power class 3 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MA

Where MA is defined as follows:

MA =	9	;	 0 ≤ B < 0.54

8	;	 0.54 ≤ B < 1.08

7	;	1.08 ≤ B < 2.16

6.5	;	2.16 ≤ B < 3.24

5.5	;	3.24 ≤ B < 5.4

4	;	5.4 ≤ B

Where:

B=(LCRB_alloc, 1* 12* SCS1 + LCRB_alloc,2 * 12 * SCS2)/1,000

#### 6.2A.2.1	UE maximum output power reduction for CA (2UL CA)

Editor’s note:	Testing for Intra-band non-contiguous UL CA is not applicable for UE not supporting IE dualPA-Architecture due to the lack of core requirement.

6.2A.2.1.1	Test purpose

The number of RB identified in Table 6.2.2.3-1 is based on meeting the requirements for adjacent channel leakage ratio for CA and the maximum power reduction (MPR) for Inter-band CA due to Cubic Metric (CM).

6.2A.2.1.2	Test applicability

The requirements of this test apply to all types of NR Power class 3 UE release 15 and forward that support 2 UL CA.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5A.2.4.1.1 is executed.

6.2A.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.2A.2.0.

6.2A.2.1.4	Test description

6.2A.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR CA configuration specified in table 5.5A.3.1-1 for inter-band UL CA, table 5.5A.1-1 for intra-band contiguous UL CA and table 5.5A.2-1 for intra-band non-contiguous UL CA. All of these configurations shall be tested with applicable test parameters for each CA configuration of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2A.2.1.4.1-1 to table 6.2A.2.1.4.1-3b. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2A.2.1.4.1-1: Inter-band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |  |
|  |  | for PCC & SCC | Modulation for all | RB allocation (NOTE 1) |  |
|  |  |  | CCs (NOTE 2) | PCC | SCC |
| 13 | Default | N/A for Maximum Power Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full | Inner Full |
| 23 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | Edge_1RB_Left |
| 33 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | Edge_1RB_Right |
| 43 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full | Outer Full |
| 54 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full | Inner Full |
| 64 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | Edge_1RB_Left |
| 74 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | Edge_1RB_Right |
| 84 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full | Outer Full |
| 9 | Default |  | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 10 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM QPSK | Outer Full | Outer Full |
| 13 | Default |  | DFT-s-OFDM 16 QAM | Inner Full | Inner Full |
| 14 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 15 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 16 | Default |  | DFT-s-OFDM 16 QAM | Outer Full | Outer Full |
| 17 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 18 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 19 | Default |  | DFT-s-OFDM 64 QAM | Outer Full | Outer Full |
| 20 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 21 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 22 | Default |  | DFT-s-OFDM 256 QAM | Outer Full | Outer Full |
| 23 | Default |  | CP-OFDM QPSK | Inner Full | Inner Full |
| 24 | Low |  | CP-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 25 | High |  | CP-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM QPSK | Outer Full | Outer Full |
| 27 | Default |  | CP-OFDM 16 QAM | Inner Full | Inner Full |
| 28 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 29 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 30 | Default |  | CP-OFDM 16 QAM | Outer Full | Outer Full |
| 31 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 32 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right | Edge_1RB_Right |

| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |  |
|  |  | for PCC & SCC | Modulation for all | RB allocation (NOTE 1) |  |
|  |  |  | CCs (NOTE 2) | PCC | SCC |
| 33 | Default |  | CP-OFDM 64 QAM | Outer Full | Outer Full |
| 34 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 35 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 36 | Default |  | CP-OFDM 256 QAM | Outer Full | Outer Full |
| 37 | Default |  | NOTE 5 | Inner Full | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. NOTE 5: The modulation is DFT-s-OFDM QPSK for PCC and CP-OFDM 256 QAM for SCC. |  |  |  |  |  |

Table 6.2A.2.1.4.1-2a: Intra-band contiguous CA Test Configuration Table for PC3 and PC2 (contiguous RB allocation)

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  | Low range High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |  |
| Test Parameters for CA bandwidth class B and C |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |
|  | for PCC & SCC | Modulations for all CCs (NOTE 2) |  |  | RB allocation (NOTE 3) |
| 1 | N/A |  |  | Pi/2 BPSK | Inner Full |
| 2 |  |  |  | Pi/2 BPSK | Outer Full |
| 3 |  |  |  | QPSK | Inner Full |
| 4 |  |  |  | QPSK | Outer Full |
| 5 |  | DFT-s-OFDM |  | 16QAM | Inner Full |
| 6 |  |  |  | 16QAM | Outer Full |
| 7 |  |  |  | 64QAM | Inner Full |
| 8 |  |  |  | 64QAM | Outer Full |
| 9 |  |  |  | 256QAM | Inner Full |
| 10 |  |  |  | 256QAM | Outer Full |
| 11 |  |  |  | QPSK | Inner Full |
| 12 |  |  |  | QPSK | Outer Full |
| 13 |  |  |  | 16QAM | Inner Full |
| 14 |  | CP-OFDM |  | 16QAM | Outer Full |
| 15 |  |  |  | 64QAM | Inner Full |
| 16 |  |  |  | 64QAM | Outer Full |
| 17 |  |  |  | 256QAM | Inner Full |
| 18 |  |  |  | 256QAM | Outer Full |
| NOTE 1: The Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: The specific configuration of each RB allocation is defined in Table 6.1A-1a. NOTE 4: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |

Table 6.2A.2.1.4.1-2b: Intra-band contiguous CA Test Configuration Table  for PC3 and PC2 (non-contiguous RB allocation)

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  | Low range High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |  |
| Test Parameters for CA bandwidth class B and C |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |
|  | for PCC & SCC | Modulations for all CCs (NOTE 2) |  |  | RB allocation (NOTE 3) |
| 1 | N/A |  |  | Pi/2 BPSK | Inner |
| 2 |  |  |  | Pi/2 BPSK | Outer 1 |
| 3 |  |  |  | Pi/2 BPSK | Outer 2 |
| 4 |  |  |  | QPSK | Inner |
| 5 |  |  |  | QPSK | Outer 1 |
| 6 |  |  |  | QPSK | Outer 2 |
| 7 |  | DFT-s-OFDM |  | 16QAM | Inner |
| 8 |  |  |  | 16QAM | Outer 1 |
| 9 |  |  |  | 16QAM | Outer 2 |
| 10 |  |  |  | 64QAM | Inner |
| 11 |  |  |  | 64QAM | Outer 1 |
| 12 |  |  |  | 64QAM | Outer 2 |
| 13 |  |  |  | 256QAM | Inner |
| 14 |  |  |  | 256QAM | Outer 1 |
| 15 |  |  |  | 256QAM | Outer 2 |
| 16 |  |  |  | QPSK | Inner |
| 17 |  |  |  | QPSK | Outer 1 |
| 18 |  |  |  | QPSK | Outer 2 |
| 19 |  |  |  | 16QAM | Inner |
| 20 |  |  |  | 16QAM | Outer 1 |
| 21 |  | CP-OFDM |  | 16QAM | Outer 2 |
| 22 |  |  |  | 64QAM | Inner |
| 23 |  |  |  | 64QAM | Outer 1 |
| 24 |  |  |  | 64QAM | Outer 2 |
| 25 |  |  |  | 256QAM | Inner |
| 26 |  |  |  | 256QAM | Outer 1 |
| 27 |  |  |  | 256QAM | Outer 2 |
| NOTE 1: The Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: The specific configuration of each RB allocation is defined in Table 6.1A-1b. NOTE 4: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |

For intra-band non-contiguous CA, the initial condition for MPR is specified by 2 tables. Table 6.2A.2.1.4.1-3a applies for MPR to meet -30dBm/MHz and Table 6.2A.2.1.4.1-3b applies for MPR to meet -13dBm/MHz. UE determines to configure test parameters based on specific channel bandwidth configurations and following rules:

If  AND{ [FCC1-FCC2 +X] > (NRB1*SCS1*12*0.001) + (0.5*NRB2*SCS2*12*0.001) - 0.5*CBW1 ,

[FCC1-FCC2+Y] > (0.5*NRB1*SCS1*12*0.001) + (NRB2*SCS2*12*0.001) - 0.5*CBW2 }

Test parameters in table 6.2A.2.1.4.1-3a apply

Else

Test parameters in table 6.2A.2.1.4.1-3b apply

Where

CC1 is the lower component carrier. And FCC1 is the centre frequency of CC1

CC2 is the lower component carrier. And FCC2 is the centre frequency of CC2

NRB1 and NRB2 are defined in table 5.3.2-1 for each carrier bandwidth for CC1 and CC2.

SCS1 and SCS2 are the subcarrier spacing for CC1 and CC2.

X is the distance between threshold frequency, where lower spectral emission mask below CC1 drops from -13 dBm / MHz to -25 dBm / MHz, and the lower edge of CC1 channel bandwidth. The values of X is listed in table 6.2A.2.1.4.1-3.

Y is the distance between threshold frequency, where upper spectral emission mask above CC2 drops from -13 dBm / MHz to -25 dBm / MHz, and the upper edge of CC2 channel bandwidth. The values of Y is listed in table 6.2A.2.1.4.1-3.

Table 6.2A.2.1.4.1-3: Values specified for X and Y for each channel bandwidth

| CBW | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value | 6 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |

Table 6.2A.2.1.4.1-3a: Intra-band non-contiguous CA Test Configuration Table (MPRIM3 to meet -13dBm/MHz) when UE supporting IE dualPA-Architecture

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  |  |  |  | Low range and High range test frequencies as specified in tables for non-contiguous CA configuration with UL CA |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |  |
| Test ID | DL config for PCC & SCC | UL configuration |  |  |  |  |  |  |  |  |
|  |  | Modulations for all CCs (NOTE 2) |  | RB allocation LCRB@RBSTART (NOTE 2) |  |  |  |  |  |  |
|  |  |  |  | SCS 15 kHz |  |  | SCS 30 kHz |  | SCS 60 kHz |  |
|  |  |  |  | PCC | SCC |  | PCC | SCC | PCC | SCC |
| 1 | N/A |  | Pi/2 BPSK | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 2 |  |  | Pi/2 BPSK | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 3 |  |  | Pi/2 BPSK | 24@0 | 24@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 4 |  |  | Pi/2 BPSK | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 53 |  |  | Pi/2 BPSK | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 64 |  |  | Pi/2 BPSK | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 75 |  |  | Pi/2 BPSK | 150@0 | 150@0 |  | 75@0 | 75@0 | 40@0 | 40@0 |
| 85 |  |  | Pi/2 BPSK | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 9 |  |  | QPSK | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 10 |  |  | QPSK | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 11 |  |  | QPSK | 24@0 | 24@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 12 |  | D | QPSK | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 133 |  | F | QPSK | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 144 |  | T | QPSK | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 155 |  | - | QPSK | 150@0 | 150@0 |  | 75@0 | 75@0 | 40@0 | 40@0 |
| 165 |  | s | QPSK | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 17 |  | - | 16QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 18 |  | O | 16QAM | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 19 |  | F | 16QAM | 24@0 | 24@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 20 |  | D | 16QAM | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 213 |  | M | 16QAM | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 224 |  |  | 16QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 235 |  |  | 16QAM | 150@0 | 150@0 |  | 75@0 | 75@0 | 40@0 | 40@0 |
| 245 |  |  | 16QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 25 |  |  | 64QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 26 |  |  | 64QAM | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 27 |  |  | 64QAM | 24@0 | 24@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 28 |  |  | 64QAM | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 293 |  |  | 64QAM | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 304 |  |  | 64QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 315 |  |  | 64QAM | 150@0 | 150@0 |  | 75@0 | 75@0 | 40@0 | 40@0 |
| 325 |  |  | 64QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 33 |  |  | 256QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 34 |  |  | 256QAM | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 35 |  |  | 256QAM | 24@0 | 24@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 36 |  |  | 256QAM | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 373 |  |  | 256QAM | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 384 |  |  | 256QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 395 |  |  | 256QAM | 150@0 | 150@0 |  | 75@0 | 75@0 | 40@0 | 40@0 |
| 405 |  |  | 256QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 41 |  |  | QPSK | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 42 |  |  | QPSK | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 43 |  |  | QPSK | 23@0 | 23@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 44 |  |  | QPSK | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 453 |  |  | QPSK | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 464 |  |  | QPSK | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 475 |  |  | QPSK | 150@0 | 150@0 |  | 75@0 | 75@0 | 38@0 | 38@0 |
| 485 |  |  | QPSK | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 49 |  |  | 16QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 50 |  | C | 16QAM | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 51 |  | P | 16QAM | 23@0 | 23@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 52 |  | - | 16QAM | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 533 |  | O | 16QAM | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 544 |  | F | 16QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 555 |  | D | 16QAM | 150@0 | 150@0 |  | 75@0 | 75@0 | 38@0 | 38@0 |
| 565 |  | M | 16QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 57 |  |  | 64QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 58 |  |  | 64QAM | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 59 |  |  | 64QAM | 23@0 | 23@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 60 |  |  | 64QAM | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 613 |  |  | 64QAM | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 624 |  |  | 64QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 635 |  |  | 64QAM | 150@0 | 150@0 |  | 75@0 | 75@0 | 38@0 | 38@0 |
| 645 |  |  | 64QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 65 |  |  | 256QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 66 |  |  | 256QAM | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 67 |  |  | 256QAM | 23@0 | 23@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 68 |  |  | 256QAM | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 693 |  |  | 256QAM | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 704 |  |  | 256QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 715 |  |  | 256QAM | 150@0 | 150@0 |  | 75@0 | 75@0 | 38@0 | 38@0 |
| 725 |  |  | 256QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| NOTE 1: The Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1. NOTE 2: Edge_1RB_Left, Edge_1RB_Right and Outer_Full RB allocation are specified in table 6.1-1 for each component carrier. NOTE 3: Only being tested when the channel bandwidth of both carriers are larger than 10 MHz. NOTE 4: Only being tested when the channel bandwidth of both carriers are larger than 20 MHz. NOTE 5: Only being tested when the channel bandwidth of both carriers are larger than 25 MHz. NOTE 6: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.4.1-3b: Intra-band non-contiguous CA Test Configuration Table (MPRIM3 to meet -30dBm/MHz) when UE supporting IE dualPA-Architecture

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  |  |  |  | Low range and High range test frequencies as specified in tables for non-contiguous CA configuration with UL CA |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |  |
| Test ID | DL config for PCC & SCC | UL configuration |  |  |  |  |  |  |  |  |
|  |  | Modulations for all CCs (NOTE 2) |  | RB allocation LCRB@RBSTART (NOTE 2) |  |  |  |  |  |  |
|  |  |  |  | SCS 15 kHz |  |  | SCS 30 kHz |  | SCS 60 kHz |  |
|  |  |  |  | PCC | SCC |  | PCC | SCC | PCC | SCC |
| 1 | N/A |  | Pi/2 BPSK | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 2 |  |  | Pi/2 BPSK | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 3 |  |  | Pi/2 BPSK | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 43 |  |  | Pi/2 BPSK | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 54 |  |  | Pi/2 BPSK | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 65 |  |  | Pi/2 BPSK | 216@0 | 216@0 |  | 108@0 | 108@0 | 54@0 | 54@0 |
| 75 |  |  | Pi/2 BPSK | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 8 |  |  | QPSK | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 9 |  |  | QPSK | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 10 |  |  | QPSK | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 113 |  |  | QPSK | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 124 |  | D | QPSK | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 135 |  | F | QPSK | 216@0 | 216@0 |  | 108@0 | 108@0 | 54@0 | 54@0 |
| 145 |  | T | QPSK | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 15 |  | - | 16QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 16 |  | s | 16QAM | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 17 |  | - | 16QAM | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 183 |  | O | 16QAM | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 194 |  | F | 16QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 205 |  | D | 16QAM | 216@0 | 216@0 |  | 108@0 | 108@0 | 54@0 | 54@0 |
| 215 |  | M | 16QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 22 |  |  | 64QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 23 |  |  | 64QAM | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 24 |  |  | 64QAM | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 253 |  |  | 64QAM | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 264 |  |  | 64QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 275 |  |  | 64QAM | 216@0 | 216@0 |  | 108@0 | 108@0 | 54@0 | 54@0 |
| 285 |  |  | 64QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 29 |  |  | 256QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 30 |  |  | 256QAM | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 31 |  |  | 256QAM | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 323 |  |  | 256QAM | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 334 |  |  | 256QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 345 |  |  | 256QAM | 216@0 | 216@0 |  | 108@0 | 108@0 | 54@0 | 54@0 |
| 355 |  |  | 256QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 36 |  |  | QPSK | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 37 |  |  | QPSK | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 38 |  |  | QPSK | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 393 |  |  | QPSK | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 404 |  |  | QPSK | 115@0 | 115@0 |  | 58@0 | 58@0 | 29@0 | 29@0 |
| 415 |  | C | QPSK | 210@0 | 210@0 |  | 105@0 | 105@0 | 53@0 | 53@0 |
| 425 |  | P | QPSK | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 43 |  | - | 16QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 44 |  | O | 16QAM | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 45 |  | F | 16QAM | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 463 |  | D | 16QAM | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 474 |  | M | 16QAM | 115@0 | 115@0 |  | 58@0 | 58@0 | 29@0 | 29@0 |
| 485 |  |  | 16QAM | 210@0 | 210@0 |  | 105@0 | 105@0 | 53@0 | 53@0 |
| 495 |  |  | 16QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 50 |  |  | 64QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 51 |  |  | 64QAM | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 52 |  |  | 64QAM | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 533 |  |  | 64QAM | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 544 |  |  | 64QAM | 115@0 | 115@0 |  | 58@0 | 58@0 | 29@0 | 29@0 |
| 555 |  |  | 64QAM | 210@0 | 210@0 |  | 105@0 | 105@0 | 53@0 | 53@0 |
| 565 |  |  | 64QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 57 |  |  | 256QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 58 |  |  | 256QAM | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 59 |  |  | 256QAM | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 603 |  |  | 256QAM | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 614 |  |  | 256QAM | 115@0 | 115@0 |  | 58@0 | 58@0 | 29@0 | 29@0 |
| 625 |  |  | 256QAM | 210@0 | 210@0 |  | 105@0 | 105@0 | 53@0 | 53@0 |
| 635 |  |  | 256QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| NOTE 1: The Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1. NOTE 2: Edge_1RB_Left, Edge_1RB_Right and Outer_Full RB allocation are specified in table 6.1-1 for each component carrier. NOTE 3: Only being tested when the channel bandwidth of both carriers are larger than 10 MHz. NOTE 4: Only being tested when the channel bandwidth of both carriers are larger than 20 MHz. NOTE 5: Only being tested when the channel bandwidth of both carriers are larger than 25 MHz. NOTE 6: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Tables 6.2A.2.1.4.1-1 to 6.2A.2.1.4.1-3b as appropriate.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2A.2.1.4.3.

6.2A.2.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, and C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.2A.2.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 6.2A.2.1.4.1-1 to 6.2A.2.1.4.1-3b on both PCC and SCC as appropriate. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

6.	Measure the sum of mean transmitted power over all component carriers in the CA configuration of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms uplink. For TDD only slots consisting of only UL symbols are under test. For FDD band in inter-band CA with both TDD band and FDD band, only slots overlapping with only UL symbols in TDD are under test.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.2A.2.1.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2A.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4.

Table 6.2A.2.1.4.3-1: FrequencyInfoUL-SIB for inter-band CA (Power Class 3)

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 22 |  | Test IDs 6, 7, 10, 11, 37 |
|  | 21 |  | Test IDs 14, 15, 17, 18 |
|  | 20 |  | Test IDs 1-5, 8, 9, 12, 13, 16, 19, 23-33, 38 |
|  | 19 |  | Test IDs 20, 21, 22 |
|  | 17 |  | Test IDs 34, 35, 36 |

NOTE:	The value of p-Max is configured below:

p-Max = CEILING {MAX {Pcmax_L,CA – 3 +ΔTC,c, Pcmax_L,CA – Pcmax_L, SCell+ΔTC,c}}

Table 6.2A.2.1.4.3-1a: FrequencyInfoUL-SIB for inter-band CA (Power Class 2 with both bands supporting PC3)

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 23 |  | Test IDs 5-13,37 |
|  | 22 |  | Test IDs 14, 15, 16, 23, 27 |
|  | 21 |  | Test IDs 17, 18, 19, 24, 25, 26, 28, 29, 30 |
|  | 20 |  | Test IDs 31, 32, 33 |
|  | 19 |  | Test IDs 20, 21, 22 |
|  | 17 |  | Test IDs 34, 35, 36 |

Table 6.2A.2.1.4.3-1b: FrequencyInfoUL-SIB for inter-band CA (PCC supporting PC3 and SCC supporting PC2)

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 23 |  | Test IDs 5-13, 16, 19, 23, 27, 37 |
|  | 22 |  | Test IDs 14, 15, 17, 18, 24, 25, 26, 28-33 |
|  | 21 |  | Test IDs 20, 21, 22 |
|  | 19 |  | Test IDs 34, 35, 36 |

Table 6.2A.2.1.4.3-2: FrequencyInfoUL-SIB for intra-band contiguous CA (contiguous RB allocation) for CA_n41C, CA_n78C

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 18 |  | Power class 3 and Test IDs 1, 3 ,5 |
|  | 16 |  | Power class 3 and Test IDs 11, 13 |
|  | 14 |  | Power class 3 and Test IDs 7, 15 |
|  | 11 |  | Power class 3 and Test IDs 2, 4, 6, 8, 9, 17 |
|  | 10 |  | Power class 3 and Test IDs 10, 12, 14, 16, 18 |
|  | 20 |  | Power class 2 and Test IDs 1, 3, 5 |
|  | 19 |  | Power class 2 and Test IDs 11, 13 |
|  | 18 |  | Power class 2 and Test IDs 7, 15 |
|  | 15 |  | Power class 2 and Test IDs 2, 4, 6, 8, 9, 17 |
|  | 14 |  | Power class 2 and Test IDs 10, 12, 14, 16, 18 |

Table 6.2A.2.1.4.3-2a: FrequencyInfoUL-SIB for intra-band contiguous CA (contiguous RB allocation) for CA_n48B

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 19 |  | Power class 3 and Test IDs 1, 3, 5 |
|  | 18 |  | Power class 3 and Test IDs 5, 11 |
|  | 17 |  | Power class 3 and Test IDs 7, 13 |
|  | 16 |  | Power class 3 and Test IDs 2, 4, 6, 15 |
|  | 15 |  | Power class 3 and Test IDs 8, 12, 14, 16 |
|  | 12 |  | Power class 3 and Test IDs 9, 10 |
|  | 11 |  | Power class 3 and Test ID 17, 18 |

Table 6.2A.2.1.4.3-2b: FrequencyInfoUL-SIB for intra-band contiguous CA (contiguous RB allocation) for CA_n77C

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 17 |  | Power class 3 and Test IDs 1, 3 ,5 |
|  | 16 |  | Power class 3 and Test IDs 11, 13 |
|  | 14 |  | Power class 3 and Test IDs 7, 15 |
|  | 11 |  | Power class 3 and Test IDs 2, 4, 6, 8, 9, 17 |
|  | 10 |  | Power class 3 and Test IDs 10, 12, 14, 16, 18 |

6.2A.2.1.5	Test requirement

The maximum output power, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.2.1.5-1 to Table 6.2A.2.1.5-3.

Table 6.2A.2.1.5-1: UE Output Power for inter-band CA (2 UL CA) of PC3 test requirements

| Test ID | PPowerClass (dBm) | MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 23 | 0.2 | 0.2 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 212 | 52 | 25+TT | 16-TT2 |
| 3 | 23 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 212 | 52 | 25+TT | 16-TT2 |
| 4 | 23 | 1.2 | 1.2 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 5 | 23 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 6 | 23 | 0.5 | 0.5 | 1.52 | 1.52 | 3 | 23 | 3 | 25+TT | 20-TT |
| 7 | 23 | 0.5 | 0.5 | 1.52 | 1.52 | 3 | 23 | 3 | 25+TT | 20-TT |
| 8 | 23 | 0.5 | 0.5 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 9 | 23 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 10 | 23 | 1 | 1 | 1.52 | 1.52 | 3 | 23 | 3 | 25+TT | 20-TT |
| 11 | 23 | 1 | 1 | 1.52 | 1.52 | 3 | 23 | 3 | 25+TT | 20-TT |
| 12 | 23 | 1 | 1 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 13 | 23 | 1 | 1 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 14 | 23 | 2 | 2 | 1.52 | 1.52 | 3 | 22.52 | 52 | 25+TT | 17.5-TT2 |
| 15 | 23 | 2 | 2 | 1.52 | 1.52 | 3 | 22.52 | 52 | 25+TT | 17.5-TT2 |
| 16 | 23 | 2 | 2 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 17 | 23 | 2.5 | 2.5 | 1.52 | 1.52 | 3 | 222 | 52 | 25+TT | 17-TT2 |
| 18 | 23 | 2.5 | 2.5 | 1.52 | 1.52 | 3 | 222 | 52 | 25+TT | 17-TT2 |
| 19 | 23 | 2.5 | 2.5 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 20 | 23 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 202 | 52 | 25+TT | 15-TT2 |
| 21 | 23 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 202 | 52 | 25+TT | 15-TT2 |
| 22 | 23 | 4.5 | 4.5 | 0 | 0 | 3 | 21.5 | 5 | 25+TT | 16.5-TT |
| 23 | 23 | 1.5 | 1.5 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 24 | 23 | 3 | 3 | 1.52 | 1.52 | 3 | 21.52 | 52 | 25+TT | 16.5-TT2 |
| 25 | 23 | 3 | 3 | 1.52 | 1.52 | 3 | 21.52 | 52 | 25+TT | 16.5-TT2 |
| 26 | 23 | 3 | 3 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 27 | 23 | 2 | 2 | 0 (1.52) | 0 (1.52) | 3 | 23 (22.52) | 3 (52) | 25+TT | 20-TT (17.5-TT2) |
| 28 | 23 | 3 | 3 | 1.52 | 1.52 | 3 | 21.52 | 52 | 25+TT | 16.5-TT2 |
| 29 | 23 | 3 | 3 | 1.52 | 1.52 | 3 | 21.52 | 52 | 25+TT | 16.5-TT2 |

| Test ID | PPowerClass (dBm) | MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 30 | 23 | 3 | 3 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 31 | 23 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 212 | 52 | 25+TT | 16-TT2 |
| 32 | 23 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 212 | 52 | 25+TT | 16-TT2 |
| 33 | 23 | 3.5 | 3.5 | 0 | 0 | 3 | 22.5 | 5 2 | 25+TT | 17.5-TT |
| 34 | 23 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 182 | 52 | 25+TT | 13-TT2 |
| 35 | 23 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 182 | 52 | 25+TT | 13-TT2 |
| 36 | 23 | 6.5 | 6.5 | 0 | 0 | 3 | 19.5 | 5 | 25+TT | 14.5-TT |
| 37 | 23 | 0 | 6.5 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-2. |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-1a: UE Output Power for intra-band contiguous 2 UL CA of PC3 (contiguous RB allocation) test requirements (for CA_n41C)

| Configura | Power | MPR |  | TC | PCMAX_L |  | TL=2 | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tion ID | Class 3 (dBm) | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 23 | 1 | 2.5 | 0 | 22 | 20.5 | 2 | 2 | 2.5 | 25+TT | 20-TT | 25+TT | 18-TT |
| 2 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 2 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 3 | 23 | 1 | 2.5 | 0 | 22 | 20.5 | 2 | 2 | 2.5 | 25+TT | 20-TT | 25+TT | 18-TT |
| 4 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 2 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 5 | 23 | 1.5 | 2.5 | 0 | 21.5 | 20.5 | 2 | 2 | 2.5 | 25+TT | 19.5-TT | 25+TT | 18-TT |
| 6 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 2 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 7 | 23 | 3 | 5 | 0 | 20 | 18 | 2 | 2.5 | 4 | 25+TT | 17.5-TT | 25+TT | 14-TT |
| 8 | 23 | 4 | 7 | 0 | 19 | 16 | 2 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 11-TT |
| 9 | 23 | 5.5 | 7 | 0 | 17.5 | 16 | 2 | 5 | 5 | 25+TT | 12.5-TT | 25+TT | 11-TT |
| 10 | 23 | 6 | 7.5 | 0 | 17 | 15.5 | 2 | 5 | 5 | 25+TT | 12-TT | 25+TT | 10.5-TT |
| 11 | 23 | 2 | 3.5 | 0 | 21 | 19.5 | 2 | 2 | 3.5 | 25+TT | 19-TT | 25+TT | 16-TT |
| 12 | 23 | 4 | 8 | 0 | 19 | 15 | 2 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 13 | 23 | 2.5 | 3.5 | 0 | 20.5 | 19.5 | 2 | 2.5 | 3.5 | 25+TT | 18-TT | 25+TT | 16-TT |
| 14 | 23 | 4 | 8 | 0 | 19 | 15 | 2 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 15 | 23 | 3.5 | 5 | 0 | 19.5 | 18 | 2 | 3.5 | 4 | 25+TT | 16-TT | 25+TT | 14-TT |
| 16 | 23 | 4 | 8 | 0 | 19 | 15 | 2 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 17 | 23 | 6.5 | 7 | 0 | 16.5 | 16 | 2 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 11-TT |
| 18 | 23 | 6.5 | 8 | 0 | 16.5 | 15 | 2 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 10-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-1b: UE Output Power for intra-band contiguous 2 UL CA of PC3 (contiguous RB allocation) test requirements (for CA_n48B, CA_n77C)

| Configura | Power | MPR |  | TC | PCMAX_L |  | TL | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tion ID | Class 3 (dBm) | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 23 | 1 | 2.5 | 0 | 22 | 20.5 | 3 | 2 | 2.5 | 25+TT | 19-TT | 25+TT | 17.5-TT |
| 2 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 3 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 3 | 23 | 1 | 2.5 | 0 | 22 | 20.5 | 3 | 2 | 2.5 | 25+TT | 19-TT | 25+TT | 17.5-TT |
| 4 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 3 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 5 | 23 | 1.5 | 2.5 | 0 | 21.5 | 20.5 | 3 | 2 | 2.5 | 25+TT | 18.5-TT | 25+TT | 17.5-TT |
| 6 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 3 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 7 | 23 | 3 | 5 | 0 | 20 | 18 | 3 | 2.5 | 4 | 25+TT | 17-TT | 25+TT | 14-TT |
| 8 | 23 | 4 | 7 | 0 | 19 | 16 | 3 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 11-TT |
| 9 | 23 | 5.5 | 7 | 0 | 17.5 | 16 | 3 | 5 | 5 | 25+TT | 12.5-TT | 25+TT | 11-TT |
| 10 | 23 | 6 | 7.5 | 0 | 17 | 15.5 | 3 | 5 | 5 | 25+TT | 12-TT | 25+TT | 10.5-TT |
| 11 | 23 | 2 | 3.5 | 0 | 21 | 19.5 | 3 | 2 | 3.5 | 25+TT | 18-TT | 25+TT | 16-TT |
| 12 | 23 | 4 | 8 | 0 | 19 | 15 | 3 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 13 | 23 | 2.5 | 3.5 | 0 | 20.5 | 19.5 | 3 | 2.5 | 3.5 | 25+TT | 17.5-TT | 25+TT | 16-TT |
| 14 | 23 | 4 | 8 | 0 | 19 | 15 | 3 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 15 | 23 | 3.5 | 5 | 0 | 19.5 | 18 | 3 | 3.5 | 4 | 25+TT | 16-TT | 25+TT | 14-TT |
| 16 | 23 | 4 | 8 | 0 | 19 | 15 | 3 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 17 | 23 | 6.5 | 7 | 0 | 16.5 | 16 | 3 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 11-TT |
| 18 | 23 | 6.5 | 8 | 0 | 16.5 | 15 | 3 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 10-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-1c: UE Output Power for intra-band contiguous 2 UL CA of PC3 (non-contiguous RB allocation) test requirements (for CA_n41C)

| Configura | Power | MPR |  | TC | PCMAX_L |  | TL | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tion ID | Class 3 (dBm) | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 23 | 2 | 2.5 | 0 | 21 | 20.5 | 2 | 2 | 2.5 | 25+TT | 19-TT | 25+TT | 18-TT |
| 2 | 23 | 5.5 (3.5)1 | 6 (4)1 | 0 | 17.5 (19.5)1 | 17 (19)1 | 2 | 5 (3.5)1 | 5 (3.5)1 | 25+TT | 12.5-TT (16-TT)1 | 25+TT | 12-TT (15.5-TT)1 |
| 3 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 4 | 23 | 2 | 2.5 | 0 | 21 | 20.5 | 2 | 2 | 2.5 | 25+TT | 19-TT | 25+TT | 18-TT |
| 5 | 23 | 5.5 (3.5)1 | 6 (4)1 | 0 | 17.5 (19.5)1 | 17 (19)1 | 2 | 5 (3.5)1 | 5 (3.5)1 | 25+TT | 12.5-TT (16-TT)1 | 25+TT | 12-TT (15.5-TT)1 |
| 6 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 7 | 23 | 2.5 | 3 | 0 | 20.5 | 20 | 2 | 2.5 | 2.5 | 25+TT | 18-TT | 25+TT | 17.5-TT |
| 8 | 23 | 5.5 | 6 | 0 | 17.5 | 17 | 2 | 5 | 5 | 25+TT | 12.5-TT | 25+TT | 12-TT |
| 9 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 10 | 23 | 4.5 | 5 | 0 | 18.5 | 18 | 2 | 4 | 4 | 25+TT | 14.5-TT | 25+TT | 14-TT |
| 11 | 23 | 6 | 6 | 0 | 17 | 17 | 2 | 5 | 5 | 25+TT | 12-TT | 25+TT | 12-TT |
| 12 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 13 | 23 | 6 | 6.5 | 0 | 17 | 16.5 | 2 | 5 | 5 | 25+TT | 12-TT | 25+TT | 11.5-TT |
| 14 | 23 | 6.5 | 6.5 | 0 | 16.5 | 16.5 | 2 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 11.5-TT |
| 15 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 16 | 23 | 2.5 | 3.5 | 0 | 20.5 | 19.5 | 2 | 2.5 | 3.5 | 25+TT | 18-TT | 25+TT | 16-TT |
| 17 | 23 | 6.5 (4.5)1 | 7 (5)1 | 0 | 16.5 (18.5)1 | 16 (18)1 | 2 | 5 (4)1 | 5 (4)1 | 25+TT | 11.5-TT (14.5-TT)1 | 25+TT | 11-TT (14-TT)1 |
| 18 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 19 | 23 | 3 | 3.5 | 0 | 20 | 19.5 | 2 | 2.5 | 3.5 | 25+TT | 17.5-TT | 25+TT | 16-TT |
| 20 | 23 | 7 | 7 | 0 | 16 | 16 | 2 | 5 | 5 | 25+TT | 11-TT | 25+TT | 11-TT |
| 21 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 22 | 23 | 5 | 5 | 0 | 18 | 18 | 2 | 4 | 4 | 25+TT | 14-TT | 25+TT | 14-TT |
| 23 | 23 | 7 | 7 | 0 | 16 | 16 | 2 | 5 | 5 | 25+TT | 11-TT | 25+TT | 11-TT |
| 24 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 25 | 23 | 7.5 | 7.5 | 0 | 15.5 | 15.5 | 2 | 5 | 5 | 25+TT | 10.5-TT | 25+TT | 10.5-TT |
| 26 | 23 | 7.5 | 7.5 | 0 | 15.5 | 15.5 | 2 | 5 | 5 | 25+TT | 10.5-TT | 25+TT | 10.5-TT |
| 27 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-1d: UE Output Power for intra-band contiguous 2 UL CA of PC3 (non-contiguous RB allocation) test requirements (for CA_n48B, CA_n77C)

| Configura | Power | MPR |  | TC | PCMAX_L |  | TL | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tion ID | Class 3 (dBm) | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 23 | 2 | 2.5 | 0 | 21 | 20.5 | 3 | 2 | 2.5 | 25+TT | 18-TT | 25+TT | 17.5-TT |
| 2 | 23 | 5.5 (3.5)1 | 6 (4)1 | 0 | 17.5 (19.5)1 | 17 (19)1 | 3 | 5 (3.5)1 | 5 (3.5)1 | 25+TT | 12.5-TT (16-TT)1 | 25+TT | 12-TT (15.5-TT)1 |
| 3 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 4 | 23 | 2 | 2.5 | 0 | 21 | 20.5 | 3 | 2 | 2.5 | 25+TT | 18-TT | 25+TT | 17.5-TT |
| 5 | 23 | 5.5 (3.5)1 | 6 (4)1 | 0 | 17.5 (19.5)1 | 17 (19)1 | 3 | 5 (3.5)1 | 5 (3.5)1 | 25+TT | 12.5-TT (16-TT)1 | 25+TT | 12-TT (15.5-TT)1 |
| 6 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 7 | 23 | 2.5 | 3 | 0 | 20.5 | 20 | 3 | 2.5 | 2.5 | 25+TT | 17.5-TT | 25+TT | 17-TT |
| 8 | 23 | 5.5 | 6 | 0 | 17.5 | 17 | 3 | 5 | 5 | 25+TT | 12.5-TT | 25+TT | 12-TT |
| 9 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 10 | 23 | 4.5 | 5 | 0 | 18.5 | 18 | 3 | 4 | 4 | 25+TT | 14.5-TT | 25+TT | 14-TT |
| 11 | 23 | 6 | 6 | 0 | 17 | 17 | 3 | 5 | 5 | 25+TT | 12-TT | 25+TT | 12-TT |
| 12 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 13 | 23 | 6 | 6.5 | 0 | 17 | 16.5 | 3 | 5 | 5 | 25+TT | 12-TT | 25+TT | 11.5-TT |
| 14 | 23 | 6.5 | 6.5 | 0 | 16.5 | 16.5 | 3 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 11.5-TT |
| 15 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 16 | 23 | 2.5 | 3.5 | 0 | 20.5 | 19.5 | 3 | 2.5 | 3.5 | 25+TT | 17.5-TT | 25+TT | 16-TT |
| 17 | 23 | 6.5 (4.5)1 | 7 (5)1 | 0 | 16.5 (18.5)1 | 16 (18)1 | 3 | 5 (4)1 | 5 (4)1 | 25+TT | 11.5-TT (14.5-TT)1 | 25+TT | 11-TT (14-TT)1 |
| 18 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 19 | 23 | 3 | 3.5 | 0 | 20 | 19.5 | 3 | 2.5 | 3.5 | 25+TT | 17-TT | 25+TT | 16-TT |
| 20 | 23 | 7 | 7 | 0 | 16 | 16 | 3 | 5 | 5 | 25+TT | 11-TT | 25+TT | 11-TT |
| 21 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 22 | 23 | 5 | 5 | 0 | 18 | 18 | 3 | 4 | 4 | 25+TT | 14-TT | 25+TT | 14-TT |
| 23 | 23 | 7 | 7 | 0 | 16 | 16 | 3 | 5 | 5 | 25+TT | 11-TT | 25+TT | 11-TT |
| 24 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 25 | 23 | 7.5 | 7.5 | 0 | 15.5 | 15.5 | 3 | 5 | 5 | 25+TT | 10.5-TT | 25+TT | 10.5-TT |
| 26 | 23 | 7.5 | 7.5 | 0 | 15.5 | 15.5 | 3 | 5 | 5 | 25+TT | 10.5-TT | 25+TT | 10.5-TT |
| 27 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-1e: UE Output Power for intra-band non-contiguous 2 UL CA of PC3 test requirements (MPRIM3 to meet -13dBm/MHz)

| Test ID | PPowerClass，CA (dBm) | MPR (dB) | ΔTC (dB) | PCMAX_L (dBm) | T(PCMAX_L) (dB) | TL (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 3 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 4 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 5 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 6 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 7,8 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 9,10 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 11 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 12 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 13 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 14 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 15,16 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 17,18 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 19 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 20 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 21 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 22 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 23,24 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 25,26 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 27 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 28 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 29 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 30 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 31,32 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 33,34 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 35 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 36 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 37 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 38 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 39,40 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 41,42 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 43 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 44 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 45 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 46 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 47,48 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 49,50 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 51 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 52 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 53 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 54 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 55,56 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 57,58 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 59 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 60 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 61 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 62 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 63,64 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 65,66 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 67 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 68 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 69 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 70 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 71,72 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |

Table 6.2A.2.1.5-1f: UE Output Power for intra-band non-contiguous 2 UL CA of PC3 test requirements (MPRIM3 to meet - 30dBm/MHz)

| Test ID | PPowerClass，CA (dBm) | MPR (dB) | ΔTC (dB) | PCMAX_L (dBm) | T(PCMAX_L) (dB) | TL (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 3 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 4 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 5 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 6,7 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 8,9 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 10 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 11 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 12 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 13,14 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 15,16 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 17 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 18 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 19 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 20,21 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 22,23 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 24 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 25 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 26 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 27,28 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 29,30 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 31 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 32 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 33 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 34,35 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 36,37 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 38 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 39 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 40 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 41,42 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 43,44 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 45 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 46 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 47 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 48,49 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 50,51 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 52 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 53 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 54 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 55,56 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 57,58 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 59 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 60 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 61 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 62,63 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |

Table 6.2A.2.1.5-2: Test Tolerance forMaximum Power Reduction (MPR) for inter-band CA

| TT for overall output power (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |  |  |
|  |  |  | BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| SCell |  | f ≤ 3.0GHz | 0.7 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | BW ≤ 40MHz | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |

Table 6.2A.2.1.5-3: Test Tolerance for Maximum Power Reduction (MPR) for intra-band CA

| TT for overall output power (dB) |  |  |
| --- | --- | --- |
| Aggregation BW | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |
| 100MHz < BW ≤ 200MHz | FFS | FFS |

Table 6.2A.2.1.5-4: UE Output Power for intra-band contiguous 2 UL CA of PC2 (contiguous RB allocation) test requirements (for CA_n41C, CA_n78C)

| Configura | Power | MPR |  | DTC | PCMAX_L |  | TL | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tion ID | Class 2 (dBm) | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 26 | 2.0 | 2.5 | 0 | 24 | 23.5 | 3 | 3 | 3 | 28+TT | 21.-TT | 28+TT | 20.5-TT |
| 2 | 26 | 4.0 | 7.0 | 0 | 22 | 19.0 | 3 | 2 | 3.5 | 28+TT | 19.-TT | 28+TT | 15.5-TT |
| 3 | 26 | 2.0 | 2.5 | 0 | 24 | 23.5 | 3 | 3 | 3 | 28+TT | 21.-TT | 28+TT | 20.5-TT |
| 4 | 26 | 4.0 | 7.0 | 0 | 22 | 19.0 | 3 | 2 | 3.5 | 28+TT | 19.-TT | 28+TT | 15.5-TT |
| 5 | 26 | 2.5 | 2.5 | 0 | 23.5 | 23.5 | 3 | 3 | 3 | 28+TT | 20.5-TT | 28+TT | 20.5-TT |
| 6 | 26 | 4.0 | 7.0 | 0 | 22 | 19.0 | 3 | 2 | 3.5 | 28+TT | 19.-TT | 28+TT | 15.5-TT |
| 7 | 26 | 3.0 | 5.0 | 0 | 23 | 21.0 | 3 | 2 | 2 | 25+TT | 20.-TT | 25+TT | 18.-TT |
| 8 | 26 | 4.5 | 7.0 | 0 | 21.5 | 19.0 | 3 | 2 | 3.5 | 25+TT | 18.5-TT | 25+TT | 15.5-TT |
| 9 | 26 | 5.5 | 7.0 | 0 | 20.5 | 19.0 | 3 | 2.5 | 3.5 | 25+TT | 17.5-TT | 25+TT | 15.5-TT |
| 10 | 26 | 6.0 | 7.5 | 0 | 20 | 18.5 | 3 | 2.5 | 4 | 25+TT | 17.-TT | 25+TT | 14.5-TT |
| 11 | 26 | 2.5 | 3.5 | 0 | 23.5 | 22.5 | 3 | 3 | 2 | 25+TT | 20.5-TT | 25+TT | 19.5-TT |
| 12 | 26 | 5.0 | 8.0 | 0 | 21 | 18.0 | 3 | 2 | 4 | 25+TT | 18.-TT | 25+TT | 14.-TT |
| 13 | 26 | 3.0 | 3.5 | 0 | 23 | 22.5 | 3 | 2 | 2 | 25+TT | 20.-TT | 25+TT | 19.5-TT |
| 14 | 26 | 5.0 | 8.0 | 0 | 21 | 18.0 | 3 | 2 | 4 | 25+TT | 18.-TT | 25+TT | 14.-TT |
| 15 | 26 | 3.5 | 5.0 | 0 | 22.5 | 21.0 | 3 | 2 | 2 | 25+TT | 19.5-TT | 25+TT | 18.-TT |
| 16 | 26 | 5.0 | 8.0 | 0 | 21 | 18.0 | 3 | 2 | 4 | 25+TT | 18.-TT | 25+TT | 14.-TT |
| 17 | 26 | 6.5 | 7.0 | 0 | 19.5 | 19.0 | 3 | 3.5 | 3.5 | 25+TT | 16.-TT | 25+TT | 15.5-TT |
| 18 | 26 | 6.5 | 8.0 | 0 | 19.5 | 18.0 | 3 | 3.5 | 4 | 25+TT | 16.-TT | 25+TT | 14.-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-5: UE Output Power for intra-band contiguous 2 UL CA of PC2 with TxD (contiguous RB allocation) test requirements (for CA_n41C, CA_n78C)

FFS

Table 6.2A.2.1.5-6: UE Output Power for intra-band contiguous 2 UL CA of PC2 (non-contiguous RB allocation) test requirements (for CA_n41C)

| Configura | Power | MPR |  | DTC | PCMAX_L |  | TL | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tion ID | Class 2 (dBm) | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 26 | 3.0 | 3 | 0 | 23.0 | 23.0 | 3 | 2 | 2 | 28+TT | 20.-TT | 28+TT | 20.-TT |
| 2 | 26 | 6.5 (4.5)1 | 7.5 (5.5)1 | 0 | 19.5 (21.5)1 | 18.5 (20.5)1 | 3 | 3.5 (2)1 | 4 (2.5)1 | 28+TT | 16.-TT (18.5-TT)1 | 28+TT | 14.5-TT (17.5-TT)1 |
| 3 | 26 | 13.0 (8.5)2 | 13.5 (9.0)2 | 0 | 13 (17.5)2 | 12.5 (17.0)2 | 3 | 5 (5)2 | 6 (5)2 | 28+TT | 8.-TT (12.5-TT)2 | 28+TT | 6.5-TT (12.0-TT)2 |
| 4 | 26 | 3.0 | 3.0 | 0 | 23 | 23.0 | 3 | 2 | 2 | 28+TT | 20.-TT | 28+TT | 20.-TT |
| 5 | 26 | 6.5 (4.5)1 | 7.5 (5.5)1 | 0 | 19.5 (21.5)1 | 18.5 (20.5)1 | 3 | 3.5 (2)1 | 4 (2.5)1 | 28+TT | 16.-TT (18.5-TT)1 | 28+TT | 14.5-TT (17.5-TT)1 |
| 6 | 26 | 13.0 (8.5)2 | 13.5 (9.0)2 | 0 | 13 (17.5)2 | 12.5 (17.0)2 | 3 | 5 (5)2 | 6 (5)2 | 28+TT | 8.-TT (12.5-TT)2 | 28+TT | 6.5-TT (12.0-TT)2 |
| 7 | 26 | 3.0 | 3.0 | 0 | 23 | 23.0 | 3 | 2 | 2 | 28+TT | 20.-TT | 28+TT | 20.-TT |
| 8 | 26 | 6.5 | 7.5 | 0 | 19.5 | 18.5 | 3 | 3.5 | 4 | 28+TT | 16.-TT | 28+TT | 14.5-TT |
| 9 | 26 | 13.0 (8.5)2 | 13.5 (9.0)2 | 0 | 13 (17.5)2 | 12.5 (17.0)2 | 3 | 5 (5)2 | 6 (5)2 | 28+TT | 8.-TT (12.5-TT)2 | 28+TT | 6.5-TT (12.0-TT)2 |
| 10 | 26 | 5.0 | 5.0 | 0 | 21 | 21.0 | 3 | 2 | 2 | 28+TT | 18.-TT | 28+TT | 18.-TT |
| 11 | 26 | 6.5 | 7.5 | 0 | 19.5 | 18.5 | 3 | 3.5 | 4 | 28+TT | 16.-TT | 28+TT | 14.5-TT |
| 12 | 26 | 13.0 (8.5)2 | 13.5 (9.0)2 | 0 | 13 (17.5)2 | 12.5 (17.0)2 | 3 | 5 (5)2 | 6 (5)2 | 28+TT | 8.-TT (12.5-TT)2 | 28+TT | 6.5-TT (12.0-TT)2 |
| 13 | 26 | 6.5 | 6.5 | 0 | 19.5 | 19.5 | 3 | 3.5 | 3.5 | 28+TT | 16.-TT | 28+TT | 16.-TT |
| 14 | 26 | 7.0 | 7.5 | 0 | 19 | 18.5 | 3 | 3.5 | 4 | 28+TT | 15.5-TT | 28+TT | 14.5-TT |
| 15 | 26 | 13.0 (8.5)2 | 13.5 (9.0)2 | 0 | 13 (17.5)2 | 12.5 (17.0)2 | 3 | 5 (5)2 | 6 (5)2 | 28+TT | 8.-TT (12.5-TT)2 | 28+TT | 6.5-TT (12.0-TT)2 |
| 16 | 26 | 3.5 | 3.5 | 0 | 22.5 | 22.5 | 3 | 2 | 2 | 28+TT | 19.5-TT | 28+TT | 19.5-TT |
| 17 | 26 | 7.0 (5.0)1 | 8.0 (6.0)1 | 0 | 19 (21.0)1 | 18.0 (20.0)1 | 3 | 3.5 (2)1 | 4 (2.5)1 | 28+TT | 15.5-TT (18.0-TT)1 | 28+TT | 14.-TT (17.0-TT)1 |
| 18 | 26 | 14.0 (9.5)2 | 14.5 (10.0)2 | 0 | 12 (16.5)2 | 11.5 (16.0)2 | 3 | 6 (5)2 | 6 (5)2 | 28+TT | 6.-TT (11.5-TT)2 | 28+TT | 5.5-TT (11.0-TT)2 |
| 19 | 26 | 3.5 | 3.5 | 0 | 22.5 | 22.5 | 3 | 2 | 2 | 28+TT | 19.5-TT | 28+TT | 19.5-TT |
| 20 | 26 | 7.0 | 8.0 | 0 | 19 | 18.0 | 3 | 3.5 | 4 | 28+TT | 15.5-TT | 28+TT | 14.-TT |
| 21 | 26 | 14.0 (9.5)2 | 14.5 (10.0)2 | 0 | 12 (16.5)2 | 11.5 (16.0)2 | 3 | 6 (5)2 | 6 (5)2 | 28+TT | 6.-TT (11.5-TT)2 | 28+TT | 5.5-TT (11.0-TT)2 |
| 22 | 26 | 5.0 | 5.0 | 0 | 21 | 21.0 | 3 | 2 | 2 | 28+TT | 18.-TT | 28+TT | 18.-TT |
| 23 | 26 | 7.0 | 8.0 | 0 | 19 | 18.0 | 3 | 3.5 | 4 | 28+TT | 15.5-TT | 28+TT | 14.-TT |
| 24 | 26 | 14.0 (9.5)2 | 14.5 (10.0)2 | 0 | 12 (16.5)2 | 11.5 (16.0)2 | 3 | 6 (5)2 | 6 (5)2 | 28+TT | 6.-TT (11.5-TT)2 | 28+TT | 5.5-TT (11.0-TT)2 |
| 25 | 26 | 7.5 | 7.5 | 0 | 18.5 | 18.5 | 3 | 4 | 4 | 28+TT | 14.5-TT | 28+TT | 14.5-TT |
| 26 | 26 | 7.5 | 8.0 | 0 | 18.5 | 18.0 | 3 | 4 | 4 | 28+TT | 14.5-TT | 28+TT | 14.-TT |
| 27 | 26 | 14.0 (9.5)2 | 14.5 (10.0)2 | 0 | 12 (16.5)2 | 11.5 (16.0)2 | 3 | 6 (5)2 | 6 (5)2 | 28+TT | 6.-TT (11.5-TT)2 | 28+TT | 5.5-TT (11.0-TT)2 |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-7: UE Output Power for intra-band contiguous 2 UL CA of PC2 with TxD (non-contiguous RB allocation) test requirements (for CA_n41C, CA_n78C)

FFS

Table 6.2A.2.1.5-8: UE Output Power for inter-band CA (2 UL CA) of PC2 test requirements (both bands supporting PC3)

| Test ID | PPowerClass (dBm) | MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 5 | 26 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 26.0 (24.52) | 3 | 28+TT | 23-TT (21.5-TT2) |
| 6 | 26 | 0.5 | 0.5 | 1.52 | 1.52 | 3 | 24.0 | 3 | 28+TT | 21-TT |
| 7 | 26 | 0.5 | 0.5 | 1.52 | 1.52 | 3 | 24.0 | 3 | 28+TT | 21-TT |
| 8 | 26 | 0.5 | 0.5 | 0 | 0 | 3 | 25.5 | 3 | 28+TT | 22.5-TT |
| 9 | 26 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 26.0 (24.52) | 3 | 28+TT | 23-TT (21.5-TT2) |
| 10 | 26 | 1 | 1 | 1.52 | 1.52 | 3 | 23.5 | 3 | 28+TT | 20.5-TT |
| 11 | 26 | 1 | 1 | 1.52 | 1.52 | 3 | 23.5 | 3 | 28+TT | 20.5-TT |
| 12 | 26 | 1 | 1 | 0 | 0 | 3 | 25.0 | 3 | 28+TT | 22-TT |
| 13 | 26 | 1 | 1 | 0 (1.52) | 0 (1.52) | 3 | 25.0 (23.52) | 3 | 28+TT | 22-TT (20.5-TT2) |
| 14 | 26 | 2 | 2 | 1.52 | 1.52 | 3 | 22.5 | 5 | 28+TT | 17.5-TT |
| 15 | 26 | 2 | 2 | 1.52 | 1.52 | 3 | 22.5 | 5 | 28+TT | 17.5-TT |
| 16 | 26 | 2 | 2 | 0 | 0 | 3 | 24.0 | 3 | 28+TT | 21-TT |
| 17 | 26 | 2.5 | 2.5 | 1.52 | 1.52 | 3 | 22.0 | 5 | 28+TT | 17-TT2 |
| 18 | 26 | 2.5 | 2.5 | 1.52 | 1.52 | 3 | 22.0 | 5 | 28+TT | 17-T |
| 19 | 26 | 2.5 | 2.5 | 0 | 0 | 3 | 23.5 | 3 | 28+TT | 20.5-TT |
| 20 | 26 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 20.0 | 5 | 28+TT | 15-TT |
| 21 | 26 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 20.0 | 5 | 28+TT | 15-TT |
| 22 | 26 | 4.5 | 4.5 | 0 | 0 | 3 | 21.5 | 5 | 28+TT | 18.5-TT |
| 23 | 26 | 1.5 | 1.5 | 0 (1.52) | 0 (1.52) | 3 | 24.5 (232) | 3 | 28+TT | 21.5-TT (20-TT2) |
| 24 | 26 | 3 | 3 | 1.52 | 1.52 | 3 | 21.5 | 5 | 28+TT | 16.5-TT |
| 25 | 26 | 3 | 3 | 1.52 | 1.52 | 3 | 21.5 | 5 | 28+TT | 16.5-TT |
| 26 | 26 | 3 | 3 | 0 | 0 | 3 | 23.0 | 3 | 28+TT | 20-TT |
| 27 | 26 | 2 | 2 | 0 (1.52) | 0 (1.52) | 3 | 24.0 (22.52) | 3 (52) | 28+TT | 21-TT (17.5-TT2) |
| 28 | 26 | 3 | 3 | 1.52 | 1.52 | 3 | 21.5 | 5 | 28+TT | 16.5-TT |
| 29 | 26 | 3 | 3 | 1.52 | 1.52 | 3 | 21.5 | 5 | 28+TT | 16.5-TT |
| 30 | 26 | 3 | 3 | 0 | 0 | 3 | 23.0 | 3 | 28+TT | 20-TT |
| 31 | 26 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 21.0 | 5 | 28+TT | 16-TT |
| 32 | 26 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 21.0 | 5 | 28+TT | 16-TT |
| 33 | 26 | 3.5 | 3.5 | 0 | 0 | 3 | 22.5 | 5 | 28+TT | 19.5-TT |
| 34 | 26 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 18.0 | 5 | 28+TT | 13-TT |
| 35 | 26 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 18.0 | 5 | 28+TT | 13-TT |
| 36 | 26 | 6.5 | 6.5 | 0 | 0 | 3 | 19.5 | 5 | 28+TT | 14.5-TT |
| 37 | 26 | 0 | 6.5 | 0 | 0 | 3 | 23.9 | 3 | 28+TT | 20.9-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-2. |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-8a: UE Output Power for inter-band CA (2 UL CA) of PC2 test requirements (PCC supporting PC3 and SCC supporting PC2)

| Test ID | PPowerClass (dBm) | MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 5 | 26 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 26.0 | 3 | 28+TT | 23-TT |
| 6 | 26 | 0.5 | 3.5 | 1.52 | 1.52 | 3 | 24.0 | 3 | 28+TT | 21-TT |
| 7 | 26 | 0.5 | 3.5 | 1.52 | 1.52 | 3 | 24.0 | 3 | 28+TT | 21-TT |
| 8 | 26 | 0.5 | 0.5 | 0 | 0 | 3 | 26.0 | 3 | 28+TT | 23-TT |
| 9 | 26 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 26.0 | 3 | 28+TT | 23-TT |
| 10 | 26 | 1 | 3.5 | 1.52 | 1.52 | 3 | 23.8 | 3 | 28+TT | 20.8-TT |
| 11 | 26 | 1 | 3.5 | 1.52 | 1.52 | 3 | 23.8 | 3 | 28+TT | 20.8-TT |
| 12 | 26 | 1 | 1 | 0 | 0 | 3 | 26.0 | 3 | 28+TT | 23-TT |
| 13 | 26 | 1 | 1 | 0 (1.52) | 0 (1.52) | 3 | 26.0 (25.32) | 3 | 28+TT | 23-TT (22.3-TT2) |
| 14 | 26 | 2 | 3.5 | 1.52 | 1.52 | 3 | 23.3 | 3 | 28+TT | 20.3-TT |
| 15 | 26 | 2 | 3.5 | 1.52 | 1.52 | 3 | 23.3 | 3 | 28+TT | 20.3-TT |
| 16 | 26 | 2 | 2 | 0 | 0 | 3 | 25.8 | 3 | 28+TT | 22.8-TT |
| 17 | 26 | 2.5 | 3.5 | 1.52 | 1.52 | 3 | 23.1 | 3 | 28+TT | 20.1-TT |
| 18 | 26 | 2.5 | 3.5 | 1.52 | 1.52 | 3 | 23.1 | 3 | 28+TT | 20.1-TT |
| 19 | 26 | 2.5 | 2.5 | 0 | 0 | 3 | 25.3 | 3 | 28+TT | 22.3-TT |
| 20 | 26 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 21.8 | 5 | 28+TT | 16.8-TT |
| 21 | 26 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 21.8 | 5 | 28+TT | 16.8-TT |
| 22 | 26 | 4.5 | 4.5 | 0 | 0 | 3 | 23.3 | 3 | 28+TT | 20.3-TT |
| 23 | 26 | 1.5 | 1.5 | 0 (1.52) | 0 (1.52) | 3 | 26.0 (24.82) | 3 | 28+TT | 23-TT (21.8-TT2) |
| 24 | 26 | 3 | 3.5 | 1.52 | 1.52 | 3 | 22.9 | 5 | 28+TT | 17.9-TT |
| 25 | 26 | 3 | 3.5 | 1.52 | 1.52 | 3 | 22.9 | 5 | 28+TT | 17.9-TT |
| 26 | 26 | 3 | 3 | 0 | 0 | 3 | 24.8 | 3 | 28+TT | 21.8-TT |
| 27 | 26 | 2 | 2 | 0 (1.52) | 0 (1.52) | 3 | 25.8 (24.32) | 3 | 28+TT | 22.8-TT (21.3-TT2) |
| 28 | 26 | 3 | 3.5 | 1.52 | 1.52 | 3 | 22.9 | 5 | 28+TT | 17.9-TT |
| 29 | 26 | 3 | 3.5 | 1.52 | 1.52 | 3 | 22.9 | 5 | 28+TT | 17.9-TT |
| 30 | 26 | 3 | 3 | 0 | 0 | 3 | 24.8 | 3 | 28+TT | 21.8-TT |
| 31 | 26 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 22.8 | 5 | 28+TT | 17.8-TT |
| 32 | 26 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 22.8 | 5 | 28+TT | 17.8-TT |
| 33 | 26 | 3.5 | 3.5 | 0 | 0 | 3 | 24.3 | 3 | 28+TT | 21.3-TT |
| 34 | 26 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 19.8 | 5 | 28+TT | 14.8-TT |
| 35 | 26 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 19.8 | 5 | 28+TT | 14.8-TT |
| 36 | 26 | 6.5 | 6.5 | 0 | 0 | 3 | 21.3 | 5 | 28+TT | 16.3-TT |
| 37 | 26 | 0 | 6.5 | 0 | 0 | 3 | 24.6 | 3 | 28+TT | 21.6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-2. |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-9: UE Output Power for inter-band CA (2 UL CA) of PC2 test requirements with supporting higherPowerLimit-r17

| Test ID | PPowerClass (dBm) | MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 5 | 27.8 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 27.8 (26.32) | 3 | 29.8+TT | 24.8-TT (23.3-TT2) |
| 6 | 27.8 | 0.5 | 3.5 | 1.52 | 1.52 | 3 | 24.0 | 3 | 29.8+TT | 21.0-TT |
| 7 | 27.8 | 0.5 | 3.5 | 1.52 | 1.52 | 3 | 24.0 | 3 | 29.8+TT | 21.0-TT |
| 8 | 27.8 | 0.5 | 0.5 | 0 | 0 | 3 | 27.3 | 3 | 29.8+TT | 24.3-TT |
| 9 | 27.8 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 27.8 (26.32) | 3 | 29.8+TT | 24.8-TT (23.3-TT2) |
| 10 | 27.8 | 1 | 3.5 | 1.52 | 1.52 | 3 | 23.8 | 3 | 29.8+TT | 20.8-TT |
| 11 | 27.8 | 1 | 3.5 | 1.52 | 1.52 | 3 | 23.8 | 3 | 29.8+TT | 20.8-TT |
| 12 | 27.8 | 1 | 1 | 0 | 0 | 3 | 26.8 | 3 | 29.8+TT | 23.8-TT |
| 13 | 27.8 | 1 | 1 | 0 (1.52) | 0 (1.52) | 3 | 26.8 (25.32) | 3 | 29.8+TT | 23.8-TT (22.3-TT2) |
| 14 | 27.8 | 2 | 3.5 | 1.52 | 1.52 | 3 | 23.3 | 3 | 29.8+TT | 20.3-TT |
| 15 | 27.8 | 2 | 3.5 | 1.52 | 1.52 | 3 | 23.3 | 3 | 29.8+TT | 20.3-TT |
| 16 | 27.8 | 2 | 2 | 0 | 0 | 3 | 25.8 | 3 | 29.8+TT | 22.8-TT |
| 17 | 27.8 | 2.5 | 3.5 | 1.52 | 1.52 | 3 | 23.1 | 3 | 29.8+TT | 20.1-TT |
| 18 | 27.8 | 2.5 | 3.5 | 1.52 | 1.52 | 3 | 23.1 | 3 | 29.8+TT | 20.1-TT |
| 19 | 27.8 | 2.5 | 2.5 | 0 | 0 | 3 | 25.3 | 3 | 29.8+TT | 22.3-TT |
| 20 | 27.8 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 21.8 | 5 | 29.8+TT | 16.8-TT |
| 21 | 27.8 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 21.8 | 5 | 29.8+TT | 16.8-TT |
| 22 | 27.8 | 4.5 | 4.5 | 0 | 0 | 3 | 23.3 | 3 | 29.8+TT | 20.3-TT |
| 23 | 27.8 | 1.5 | 1.5 | 0 (1.52) | 0 (1.52) | 3 | 26.3 (24.82) | 3 | 29.8+TT | 23.3-TT (21.8-TT2) |
| 24 | 27.8 | 3 | 3.5 | 1.52 | 1.52 | 3 | 22.9 | 5 | 29.8+TT | 17.9-TT |
| 25 | 27.8 | 3 | 3.5 | 1.52 | 1.52 | 3 | 22.9 | 5 | 29.8+TT | 17.9-TT |
| 26 | 27.8 | 3 | 3 | 0 | 0 | 3 | 24.8 | 3 | 29.8+TT | 21.8-TT |
| 27 | 27.8 | 2 | 2 | 0 (1.52) | 0 (1.52) | 3 | 25.8 (24.32) | 3 | 29.8+TT | 22.8-TT (21.3-TT2) |
| 28 | 27.8 | 3 | 3.5 | 1.52 | 1.52 | 3 | 22.9 | 5 | 29.8+TT | 17.9-TT |
| 29 | 27.8 | 3 | 3.5 | 1.52 | 1.52 | 3 | 22.9 | 5 | 29.8+TT | 17.9-TT |
| 30 | 27.8 | 3 | 3 | 0 | 0 | 3 | 24.8 | 3 | 29.8+TT | 21.8-TT |
| 31 | 27.8 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 22.8 | 5 | 29.8+TT | 17.8-TT |
| 32 | 27.8 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 22.8 | 5 | 29.8+TT | 17.8-TT |
| 33 | 27.8 | 3.5 | 3.5 | 0 | 0 | 3 | 24.3 | 3 | 29.8+TT | 21.3-TT |
| 34 | 27.8 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 19.8 | 5 | 29.8+TT | 14.8-TT |
| 35 | 27.8 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 19.8 | 5 | 29.8+TT | 14.8-TT |
| 36 | 27.8 | 6.5 | 6.5 | 0 | 0 | 3 | 21.3 | 5 | 29.8+TT | 16.3-TT |
| 37 | 27.8 | 0 | 6.5 | 0 | 0 | 3 | 24.6 | 3 | 29.8+TT | 21.6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-2. |  |  |  |  |  |  |  |  |  |  |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

### 6.2A.3	UE additional maximum output power reduction for CA

#### 6.2A.3.0	Minimum conformance requirements

##### 6.2A.3.0.1	UE additional maximum output power reduction for Intra-band contiguous CA

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6]. Relation between NR CA band and NR frequency band is specified in Table 5.2A.1-1.

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2A.1.0.4-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2A.2.0.4. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2A.3.0.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable CA band(s) for each CA_NS value. The CA_NS_xy value indicates the additional unwanted emissions requirements that apply for intra-band contiguous CA bands with NS_xy indicated or configured in multiple uplink serving cells, except CA_NS_01 that indicates the general emission requirements for intra-band contiguous CA bands. The mapping of NR CA band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2A.3.0.1-2. For any NR CA band not listed in Table 6.2A.3.0.1-2 the network signalling label CA_NS_01 applies.

Table 6.2A.3.0.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR CA Band | Aggregated channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| CA_NS_01 | 6.5A.2.2.1 6.5A.3.2.1 | Table 5.2A.1-1 | All applicable NR CA bands | All applicable NR CA configurations | N/A |
| CA_NS_04 | 6.5A.2.3.1.1 6.5A.3.3.1.1 | CA_n41 | Table 5.5A.1-1 | 6.2A.3.1.1.1 | 6.2A.3.1.1.1 |
| CA_NS_27 | 6.5A.2.3.1.2 6.5A.3.3.1.2 | CA_n48 | Table 5.5A.1-1 | 6.2A.3.1.1.2 | 6.2A.3.1.1.2 |
| CA_NS_46 | 6.5A.3.3.1.3 | CA_n7 | Table 5.5A.1-1 | 6.2A.3.1.1.3 | 6.2A.3.1.1.3 |
| CA_NS_55 | See CA_NS_01 | CA_n77 | Table 5.5A.1-1 | 6.2A.2.1 | See CA_NS_01 |

For UEs configured with intra-band contiguous CA in n77 and if NS_01 is indicated for an uplink component carrier in the range 3450-3650 MHz and NS_01 or NS_57 for another uplink component carrier in the range 3650-3980 MHz, the allowed additional spurious emission and maximum output power reduction requirements are according to CA_NS_01.

Table 6.2A.3.0.1-2: Mapping of network signalling label

| NR CA band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| CA_n41 | CA_NS_01 | CA_NS_04 |  |  |  |  |  |  |
| CA_n48 | CA_NS_01 | CA_NS_27 |  |  |  |  |  |  |
| CA_n7 | CA_NS_01 | CA_NS_46 |  |  |  |  |  |  |
| CA_n77 | CA_NS_01 | CA_NS_55 |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

###### 6.2A.3.0.1.1	A-MPR for CA_NS_04

6.2A.3.0.1.1.1	Contiguous allocations

For all waveform type, modulations and scs when Fedge, low - BWChannel_CA ≥ 2490.5 MHz, A-MPR = MPR

For all modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

if the RB allocation is an inner allocation as defined in clause 6.2A.2.0.4, then A-MPR = MPR

Except for RBstart ≤ 0.33*BWchannel_CA/0.18MHz, AMPR= max (MPR, AMPRcc).

if the RB allocation is an outer allocation as defined in clause 6.2A.2.0.4,

then A-MPR = MPR+1.5dB for BW Class B A-MPR = MPR for BW class C.

Where

-	MPR is the MPR as defined in Table 6.2A.2.0.4-1, Table 6.2A.2.0.4-1a and Table 6.2A.2.0.4-1b for PC3 and PC2 respectively and the respective CA bandwidth class

-	AMPRcc is defined as the PC3_A2 or PC2_A4 AMPR in table 6.2.3.3.2-2 for PC3 and PC2 respectively.

6.2A.3.0.1.1.2	Non-contiguous allocations

For intra-band contiguous CA_n41B and CA_n41C and it receives IE CA_ NS_04, the UE determines the allowed Additional Maximum Power Reduction (AMPR) for the maximum output power as specified in this clause. The AMPR is specified by AMPRIM3 to meet -25dBm/MHz when IM3 falls in -25dBm/MHz region of Table 6.5A.2.3.0.1.1-1 or Table 6.5A.3.3.0.1.1-1. And uses MPR for all other cases.

The UE determines the AMPR type as follows:

For all waveform types, modulations and SCS when Fedge, low - BWChannel_CA ≥ 2490.5 MHz,

if allocation is an inner or outer 1 allocation as defined in Table 6.2A.2.0.4-2 then A-MPR = MPR

if allocation is an outer 2 allocation as defined in Table 6.2A.2.0.4-2 then A-MPR = MPR+1dB

For all waveform types, modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

If AND( MIN(FIM3,low_block,high, SEM-13,low) < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

if RB allocation is an inner or outer 1 allocation as defined in Table 6.2A.2.0.4-2 then A-MPR = MPR

if RB allocation is an outer 2 allocation as defined in Table6.2A.2.0.4-2 then A-MPR = MPR+1dB

Else

A-MPR = A-MPRIM3 defined in Clause 6.2A.3.0.1.1.3.

where

-	MPR is the MPR as defined in Table 6.2A.2.0.4-2, Table 6.2A.2.0.4-3 and Table 6.2A.2.0.4-4 for PC3 and PC2 respectively and the respective CA bandwidth class

-	FIM3,low_block,high = (2 * Flow_alloc,high_edge ) – Fhigh_alloc,low_edge

-	FIM3,high_block,low = (2 * Fhigh_alloc,low_edge) – Flow_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of upper transmission bandwidth allocation.

-	Ffilter,low = 2480 MHz

-	Ffilter,high = 2745 MHz

-	SEM-13,high = Threshold frequency where upper spectral emission mask for upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Clause 6.5A.2.3.0.1.1.

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.3.0.1.1.

6.2A.3.0.1.1.3	AMPRIM3 to meet -25dBm/MHz

AMPR in this clause is for intra-band contiguous CA_n41B and CA_n41C. The allowed maximum output power reduction is defined as:

AMPRIM3=MA, Where MA is defined as follows

MA =	13;	0 ≤ B < 2.16

11.5;	2.16 ≤ B < 3.24

10.5;	3.24 ≤ B < 5.04

9.5;	5.04 ≤ B < 10.08

8;	10.08 ≤ B < 16.56

7;	16.56 ≤ B < 21.96

6;	21.96 ≤ B

Where:

B=(LCRB1* 12* SCS1 + LCRB2 * 12 * SCS2)/1,000

and LCRB1, SCS1 are for CC1, LCRB2, SCS2 are for CC2, CC1 is the component carrier with lower frequency.

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.2A.3.1

##### 6.2A.3.0.2	FFS

##### 6.2A.3.0.3	UE additional maximum output power reduction for inter-band CA

Unless otherwise stated, for inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in subclause 6.2.3 apply for each uplink component carrier.

Unless specified in Table 6.2A.3.0.3-1, for inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in clause 6.2.3 apply only to the indicated carrier. The requirements in Table 6.2A.3.0.3-1 are specified in terms of an additional spectrum emission requirement with their associated network signalling values and the allowed A-MPR. Unless otherwise stated, the combined requirements and allowed A-MPR are applicable on both bands when both component carriers are active. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet the additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. In case of a power class 3 UE, when IE powerBoostPi2BPSK is set to 1, power class 2 A-MPR values apply.

For almost contiguous allocations in CP-OFDM waveforms in power class 3, the allowed A-MPR defined in clause 6.2.3 is increased by CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB, where NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs, and the parameter LCRB is replaced by NRB_alloc + NRB_gap in specifying the RB allocation regions.

Unless otherwise specified, pi/2 BPSK in following A-MPR tables refers to both variants of pi/2 BPSK referenced in 6.2.2 tables 6.2.2-1.

The emission requirements specified in Table 6.2A.3.0.3-1 also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.2A.3.0.3-1: Additional Requirements for uplink inter-band carrier aggregation (two-bands)

| NR CA combination | Band | Applied NS | Requirements (clause) | A-MPR (table/clause) | Note |
| --- | --- | --- | --- | --- | --- |
|  | n1 | 05 | 6.5.3.3.3.4 | Clause 6.2.3.3.4 |  |
| CA_n1-n3 |  | 05U | 6.5.3.3.3.4, 6.5.2.4.2 | Clause 6.2.3.3.4 | 1 |
|  | n3 | 100 | 6.5.2.4.2 | Table 6.2.3.3.1-2 |  |
|  | n1 | 05 | 6.5.3.3.3.4 | Clause 6.2.3.3.4 |  |
| CA_n1-n8 |  | 05U | 6.5.3.3.3.4, 6.5.2.4.2 | Clause 6.2.3.3.4 | 1 |
|  | n8 | 43 | 6.5.3.3.3.5 | Clause 6.2.3.3.6 |  |
|  |  | 43U | 6.5.3.3.3.5, 6.5.2.4.2 | Clause 6.2.3.3.6 |  |
| CA_n1-n28 | n1 | 05 | 6.5.3.3.4 | Clause 6.2.3.3.4 |  |
|  |  | 05U | 6.5.3.3.4, 6.5.2.4.2 | Clause 6.2.3.3.4 | 1,2 |
|  | n28 | 17 | 6.5.3.3.2 | N/A |  |
| CA_n1-n41 | n1 | 05 | 6.5.3.3.4 | Clause 6.2.3.4 |  |
|  |  | 05U | 6.5.3.3.4, 6.5.2.4.2 | Clause 6.2.3.4 | 1 |
|  | n41 | 47 | 6.5.3.3.15 | Table 6.2.3.18-2 |  |
| CA_n1-n78 | n1 | 05 | 6.5.3.3.3.4 | Clause 6.2.3.3.4 | 1 |
|  |  | 05U | 6.5.3.3.3.4, 6.5.2.4.2 | Clause 6.2.3.3.4 |  |
| CA_n1-n79 | n1 | 05 | 6.5.3.3.3.4 | Clause 6.2.3.3.4 | 1 |
|  |  | 05U | 6.5.3.3.3.4, 6.5.2.4.2 | Clause 6.2.3.3.4 |  |
| CA_n3-n28 | n3 | 100 | 6.5.2.4.2 | Table 6.2.3.3.1-2 | 1,2 |
|  | n28 | 17 | 6.5.3.3.2 | N/A |  |
| CA_n3-n40 | n3 | 100 | 6.5.2.4.2 | Table 6.2.3.3.1-2 | 1 |
| CA_n3-n41 | n3 | 100 | 6.5.2.4.2 | Table 6.2.3.3.1-2 | 1 |
|  | n41 | 47 | 6.5.3.3.3.15 | Table 6.2.3.3.18-2 |  |
| CA_n3-n78 | n3 | 100 | 6.5.2.4.2 | Table 6.2.3.3.1-2 | 1 |
| CA_n5-n77 | n5 | 100 | 6.5.2.4.2 | Table 6.2.3.3.1-2 | 1 |
| CA_n5-n78 | n5 | 100 | 6.5.2.4.2 | Table 6.2.3.3.1-2 | 1 |
| CA_n8-n78 | n8 | 43 | 6.5.3.3.3.5 | Clause 6.2.3.3.6 | 1 |
|  |  | 43U | 6.5.3.3.3.5, 6.5.2.4.2 | Clause 6.2.3.3.6 |  |
| CA_n24-n41 | n24 | 56 | 6.5.3.3.3.27 | Clause 6.2.3.3.30 |  |
|  | n41 | 04 | 6.5.2.3.3.2, 6.5.3.3.3.1 | Clause 6.2.3.3.2 |  |
| CA_n24-n48 | n24 | 56 | 6.5.3.3.3.27 | Clause 6.2.3.3.30 |  |
|  | n48 | 27 | 6.5.2.3.3.8, 6.5.3.3.3.14 | Clause 6.2.3.3.16 |  |
| CA_n24-n77 | n24 | 56 | 6.5.3.3.3.27 | Clause 6.2.3.3.30 |  |
|  | n77 | 55 | NOTE 6 | N/A |  |
| CA_n28-n40 | n28 | 17 | 6.5.3.3.3.2 | N/A | 2 |
| CA_n28-n41 | n28 | 17 | 6.5.3.3.3.2 | N/A | 2 |
|  | n41 | 47 | 6.5.3.3.3.15 | Table 6.2.3.3.18-2 |  |
| CA_n28-n78 | n28 | 17 | 6.5.3.3.3.2 | N/A | 2 |
| CA_n28-n79 | n28 | 17 | 6.5.3.3.3.2 | N/A | 2 |
| CA_n41-n77 | n41 | 47 | 6.5.3.3.15 | Table 6.2.3.18-2 |  |
| CA_n41-n79 | n41 | 47 | 6.5.3.3.3.15 | Table 6.2.3.3.18-2 |  |
| NOTE 1: NS_05U, NS_43U and NS_100 can be signalled for NR bands that have UTRA services deployed and the requirements in clause 6.5.2.4.2 are only applicable to the signalling carrier. NOTE 2: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 3: TBD NOTE 4: TBD NOTE 5: TBD NOTE 6: This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements. |  |  |  |  |  |

#### 6.2A.3.1	UE additional maximum output power reduction for CA (2UL CA)

Editor’s note: This clause is complete for AMPR testing. But the following aspects are either missing for not yet determined:

-	For intra-band non-contiguous CA, tests for network signalling value other than NS_04 are not complete.

6.2A.3.1.1	Test purpose

Additional emission requirements for CA can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the CA maximum output power as specified in Table 6.2A.1.0.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2A.2. Outer and inner allocation notation used in clause 6.2A.3 is defined in clause 6.2.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

6.2A.3.1.2	Test applicability

The requirements of this test apply in test case 6.5A.2.3 Additional Spectrum Emission mask for CA for network signalling values CA_NS_04 to all types of NR UE release 15 and forward.

The requirements of this test apply in test case 6.5A.3.3 Additional Spurious Emissions for CA for network signalling values CA_NS_04 to all types of NR UE release 15 and forward.

6.2A.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in 6.2A.3.0.

6.2A.3.1.4	Test description

6.2A.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. For the UE maximum output power modified by A-MPR specified in table 6.2A.3.0.3-1, the power limits specified in subclause 6.2A.4.1.3 apply.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing and are shown in table 6.2A.3.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.2A.3.1.4.1-1: Test Configuration Table for inter-band CA (network signalling value NS_47/NS_01)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] clause 4.1 |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] clause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  | Low range for PCC and SCC PCC specified in Table 6.2.3.4.1-18a and 6.2.3.4.1-18b |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] clause 4.3.1 |  |  |  |  | See Ch BW column for PCC Lowest NRB_agg for SCC |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest,Highest |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |
| Test ID | DL | UL configuration |  |  |  |  |  |  |
|  | configuration | PCC |  |  |  |  | SCC |  |
|  |  | FC (MHz) | Ch BW (MHz) | Modulation |  | RB allocation | Modulation | RB allocation |
| 1 | N/A for A-MPR | Default | 30 | DFT-s-OFDM QPSK |  | Edge_1RB_Right | DFT-s-OFDM QPSK | Inner Full |
| 2 | testing | Default | 30 | DFT-s-OFDM QPSK |  | Edge_1RB_Left | CP-OFDM 256QAM | Outer Full |
| 3 |  | Default | 30 | DFT-s-OFDM QPSK |  | Edge_1RB_Left | DFT-s-OFDM QPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.4.1-2: Test Configuration Table for inter-band CA (network signalling value NS_100/NS_01)

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest NRB_agg for PCC and SCC Highest NRB_agg for PCC and SCC |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest,Highest |  |  |
| Test Parameters |  |  |  |  |  |  |
|  | DL | UL configuration |  |  |  |  |
| Test ID | configur | Modulation |  |  | RB allocation |  |
|  | ation | PCC | SCC |  | PCC | SCC |
| 1 | N/A | DFT-s-OFDM QPSK | DFT-s-OFDM QPSK |  | Inner Full | Inner Full |
| 2 |  | CP-OFDM 256QAM | CP-OFDM 256QAM |  | Outer Full | Outer Full |
| 3 |  | DFT-s-OFDM QPSK | CP-OFDM 256QAM |  | Inner Full | Outer Full |

Table 6.2A.3.1.4.1-3: Test Configuration Table for inter-band CA (network signalling value NS_43/NS_01, NS_43U/NS_01)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  | Low range, High range for PCC and SCC unless otherwise stated in Fc column for PCC |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | See Ch BW column for PCC Lowest NRB_agg, Highest NRB_agg for SCC |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 15kHz |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |  |  |  |
|  |  | PCC |  |  |  |  | SCC |  |
|  |  | FC (MHz) | Ch BW (MHz) | Modulation |  | RB allocation | Modulation | RB allocation |
| 1 |  | 910 | 10MHz | DFT-s-OFDM QPSK |  | Inner Full | DFT-s-OFDM QPSK | Inner Full |
| 2 | N/A | 907.5 | 15MHz | CP-OFDM 256QAM |  | Outer Full | CP-OFDM 256QAM | Outer Full |
| 3 |  | 907.5 | 15MHz | CP-OFDM 256QAM |  | Outer Full | DFT-s-OFDM QPSK | Inner Full |

Table 6.2A.3.1.4.1-4: Void

Table 6.2A.3.1.4.1-5: Void

Table 6.2A.3.1.4.1-6: Void

Table 6.2A.3.1.4.1-7: Test Configuration Table for intra-band contiguous CA for CA_NS_04 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  | Refering to Test Frequency Column |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg Highest NRB_agg |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |
| Test ID | Test Frequency | Downlink Configuration | Uplink Configuration |  |
|  |  |  | Modulation | RB allocation for PCC&SCC (NOTE 1) |
| 1 | High range |  | CP-OFDM QPSK | Outer_Full |
| 2 | High range |  | CP-OFDM QPSK | Edge_1RB_Right |
| 3 | Low range |  | CP-OFDM QPSK | Outer_Full |
| 4 | Low range |  | CP-OFDM QPSK | Edge_1RB_Left |
| 5 | Low range |  | DFT-s-OFDM Pi/2 BPSK | Inner_Full |
| 6 | Low range |  | DFT-s-OFDM Pi/2 QPSK | Inner_Full |
| 7 | Low range |  | DFT-s-OFDM Pi/2 16QAM | Inner_Full |
| 8 | Low range |  | DFT-s-OFDM Pi/2 64QAM | Inner_Full |
| 9 | Low range |  | DFT-s-OFDM Pi/2 256QAM | Inner_Full |
| 10 | Low range |  | CP-OFDM QPSK | Inner_Full |
| 11 | Low range |  | CP-OFDM 16QAM | Inner_Full |
| 12 | Low range |  | CP-OFDM 64QAM | Inner_Full |
| 13 | Low range |  | CP-OFDM 256QAM | Inner_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1A-1a. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. |  |  |  |  |

Table 6.2A.3.1.4.1-8: Test Configuration Table for inter-band CA (network signalling value NS_17/NS_01)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  | Low range, High range for PCC and SCC unless otherwise stated in Fc column for PCC |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | See Ch BW column for PCC Lowest NRB_agg, Highest NRB_agg for SCC |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 15kHz |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |  |  |  |
|  |  | PCC |  |  |  |  | SCC |  |
|  |  | FC (MHz) | Ch BW (MHz) | Modulation |  | RB allocation | Modulation | RB allocation |
| 1 |  | 745.5 | 5 | DFT-s-OFDM QPSK |  | Inner Full | DFT-s-OFDM QPSK | Inner Full |
| 2 | N/A | 743 | 10 | CP-OFDM 256QAM |  | Outer Full | CP-OFDM 256QAM | Outer Full |
| 3 |  | 743 | 10 | CP-OFDM 256QAM |  | Outer Full | DFT-s-OFDM QPSK | Inner Full |

Table 6.2A.3.1.4.1-9: Test Configuration Table for PC3 inter-band CA or PC2 inter-band CA with both bands supporting PC3 (network signalling value NS_17/NS_47)

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  | Mid range for PCC Test frequencies as specified in table 6.2.3.4.1-18a and 6.2.3.4.1-18b for SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | 5MHz and 10MHz for PCC 30 MHz for SCC |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest, Highest as specified in test parameters |  |  |
| Test Parameters |  |  |  |  |  |  |
|  | DL | UL configuration |  |  |  |  |
| Test ID | configur | Modulation |  |  | RB allocation |  |
|  | ation | PCC | SCC |  | PCC | SCC |
| 1 | N/A | DFT-s-OFDM QPSK | DFT-s-OFDM QPSK |  | Inner Full | Edge_1RB_Right |
| 2 |  | CP-OFDM 256QAM | DFT-s-OFDM QPSK |  | Outer Full | Edge_1RB_Left |
| 3 |  | DFT-s-OFDM QPSK | DFT-s-OFDM QPSK |  | Inner Full | Edge_1RB_Left |

Table 6.2A.3.1.4.1-9a: Test Configuration Table for PC2 inter-band CA with PCC supporting PC3 and SCC supporting PC2 (network signalling value NS_17/NS_47)

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  | Mid range for PCC Test frequencies as specified in table 6.2.3.4.1-18a and 6.2.3.4.1-18b for SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | 5MHz and 10MHz for PCC 30 MHz for SCC |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest, Highest as specified in test parameters |  |  |
| Test Parameters |  |  |  |  |  |  |
|  | DL | UL configuration |  |  |  |  |
| Test ID | configur | Modulation |  |  | RB allocation |  |
|  | ation | PCC | SCC |  | PCC | SCC |
| 1 | N/A | DFT-s-OFDM QPSK | DFT-s-OFDM PI/2 BPSK |  | Inner Full | Edge_1RB_Right |
| 2 |  | CP-OFDM 256QAM | DFT-s-OFDM PI/2 BPSK |  | Outer Full | Edge_1RB_Left |
| 3 |  | DFT-s-OFDM QPSK | DFT-s-OFDM PI/2 BPSK |  | Inner Full | Edge_1RB_Left |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2A.3.1.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2A.3.1.4.3.

6.2A.3.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.2A.3.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to tables in 6.2A.3.1.4.1 as appropriate on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

6.	Measure the sum of mean transmitted power over all component carriers in the CA configuration of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms uplink. For TDD only slots consisting of only UL symbols are under test. For FDD band in inter-band CA with both TDD band and FDD band, only slots overlapping with only UL symbols in TDD are under test. For power class 2 intra-band contiguous carrier aggregation, the additional spectrum emission mask is measured as the sum from both UE transmit antenna connectors when UE indicates support for dualPA-Architecture IE.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration tables in 6.2A.3.1.4.1 send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2A.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions.

6.2A.3.1.4.3.1	Message contents exceptions (network signalling value " NS_100" on PCC)

Table 6.2A.3.1.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_100" on PCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (NS_100) |  | not for band n65 |
|  | 2 (NS_100) |  | for band n65 |

6.2A.3.1.4.3.2	Message contents exceptions (network signalling value " NS_43" on PCC)

Table 6.2A.3.1.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_43" on PCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 2 (NS_43) |  |  |

6.2A.3.1.4.3.3	Message contents exceptions (network signalling value " NS_43U" on PCC)

Table 6.2A.3.1.4.3.3-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_43U" on PCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 3 (NS_43U) |  |  |

6.2A.3.1.4.3.4	Message contents exceptions (network signalling value "NS_47" on PCC)

Table 6.2A.3.1.4.3.4-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_47" on PCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 2 (NS_47) |  |  |

6.2A.3.1.4.3.5	Void

Table 6.2A.3.1.4.3.5-1: Void

6.2A.3.1.4.3.6	Message contents exceptions (network signalling value "CA_NS_04" on PCC and SCC)

Table 6.2A.3.1.4.3.6-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "CA_NS_04" on PCC and SCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (CA_NS_04) |  |  |

Table 6.2A.3.1.4.3.6-2: FrequencyInfoUL-SIB for intra-band contiguous CA (contiguous RB allocation) for CA_NS_04

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 16 |  | Power class 3 and Test IDs 5 |
|  | 15 |  | Power class 3 and Test IDs 6, 7 |
|  | 14 |  | Power class 3 and Test IDs 2, 4, 8 |
|  | 12 |  | Power class 3 and Test IDs 10, 11, 12 |
|  | 11 |  | Power class 3 and Test IDs 9 |
|  | 10 |  | Power class 3 and Test IDs 1, 3, 13 |
|  | 19 |  | Power class 2 and Test IDs 5 |
|  | 19 |  | Power class 2 and Test IDs 2, 4 |
|  | 17 |  | Power class 2 and Test IDs 6, 7 |
|  | 16 |  | Power class 2 and Test IDs 8 |
|  | 14 |  | Power class 2 and Test IDs 10, 11, 12 |
|  | 14 |  | Power class 2 and Test IDs 1, 3, 9 |
|  | 11 |  | Power class 2 and Test IDs 13 |

6.2A.3.1.4.3.7	Message contents exceptions (network signalling value "NS_17" on PCC)

Table 6.2A.3.1.4.3.7-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_17" on PCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (NS_17) |  |  |

6.2A.3.1.4.3.8	Message contents exceptions (network signalling value "NS_47" on SCC)

Table 6.2A.3.1.4.3.8-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_47" on SCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 2 (NS_47) |  |  |

6.2A.3.1.5	Test requirement

The maximum output power, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.3.1.5-1, Table 6.2A.3.1.5-2, Table 6.2A.3.1.5-3 as appropriate. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in 6.2A.4.0 apply. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 6.2A.3.1.5-0: Test Tolerance for Additional Maximum Power Reduction (A-MPR)

| TT for overall output power (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |  |  |
|  |  |  | BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| SCell | BW ≤ 40MHz | f ≤ 3.0GHz | 0.7 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |

Table 6.2A.3.1.5-1: UE Power Class 3 test requirement (network signalling value NS_100/NS_01) for CA_n3-n78

| Test ID | PPowerclass(dBm) | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 19.5 | 5 | 25+TT | 14.5-TT |
| 3 | 23 | 0 | 6.5 | 0 | 0 | 0 (1.52) | 0 | 3 | 23 (22.72) | 3 (52) | 25+TT | 20-TT (17.7-TT2) |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-1a: UE Power Class 2 with both bands supporting PC3 test requirement (network signalling value NS_100) for CA_n3-n78

| Test ID | PPowerclass(dBm) | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 26 | 0 | 0 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 26 (24.52) | 3 | 28+TT | 23-TT (21.5-TT2) |
| 2 | 26 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 19.5 | 5 | 28+TT | 14.5-TT |
| 3 | 26 | 0 | 6.5 | 0 | 0 | 0 (1.52) | 0 | 3 | 23.9 (22.72) | 3 (52) | 28+TT | 20.9-TT (17.7-TT2) |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-1b: UE Power Class 2 with PCC supporting PC3 and SCC supporting PC2 test requirement (network signalling value NS_100) for CA_n3-n78

| Test ID | PPowerclass(dBm) | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 26 | 0 | 0 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 26 | 3 | 28+TT | 23-TT |
| 2 | 26 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 21.3 | 5 | 28+TT | 16.3-TT |
| 3 | 26 | 0 | 6.5 | 0 | 0 | 0 (1.52) | 0 | 3 | 24.6 (23.62) | 3 | 28+TT | 21.6-TT (20.62-TT) |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-1c: UE Power Class 2 with supporting higherPowerLimit-r17 test requirement (network signalling value NS_100) for CA_n3-n78

| Test ID | PPowerclass(dBm) | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 27.8 | 0 | 0 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 27.8 (26.32) | 3 | 29.8+TT | 24.8-TT (23.32-TT) |
| 2 | 27.8 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 21.3 | 5 | 29.8+TT | 16.3-TT |
| 3 | 27.8 | 0 | 6.5 | 0 | 0 | 0 (1.52) | 0 | 3 | 24.6 (23.62) | 3 | 29.8+TT | 21.6-TT (20.62-TT) |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-2: UE Power Class 3 test requirement (network signalling value NS_43/NS_01, NS_43U/NS_01) for CA_n8-n78

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 6.5 | 9.0 | 0 | 0 | 0 | 3 | 18.4 | 5 | 25+TT | 13.4-TT |
| 3 | 23 | 6.5 | 0 | 9.0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-3: UE Power Class 3 test requirement (network signalling value NS_47/NS_01) for CA_n41-n79

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 1 | 0 | 2 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 1 | 6.5 | 7 | 0 | 0 | 0 | 3 | 19 | 5 | 25+TT | 14-TT |
| 3 | 23 | 1 | 0 | 7 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-3a: UE Power Class 2 test requirement with both bands supporting PC3 (network signalling value NS_47/NS_01) for CA_n41-n79

| Test ID | PPowerclass | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 1 | 0 | 2 | 0 | 0 | 0 | 3 | 25 | 3 | 28+TT | 22-TT |
| 2 | 26 | 1 | 6.5 | 7 | 0 | 0 | 0 | 3 | 19 | 5 | 28+TT | 14-TT |
| 3 | 26 | 1 | 0 | 7 | 0 | 0 | 0 | 3 | 23.8 | 3 | 28+TT | 20.8-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-3b: UE Power Class 2 test requirement with PCC supporting PC2 and SCC supporting PC3 (network signalling value NS_47/NS_01) for CA_n41-n79

| Test ID | PPowerclass | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 3.5 | 0 | 5 | 0 | 0 | 0 | 3 | 25 | 3 | 28+TT | 22-TT |
| 2 | 26 | 3.5 | 6.5 | 10 | 0 | 0 | 0 | 3 | 19 | 5 | 28+TT | 14-TT |
| 3 | 26 | 3.5 | 0 | 10 | 0 | 0 | 0 | 3 | 23.8 | 3 | 28+TT | 20.8-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-3c: UE Power Class 2 test requirement with PCC and SCC supporting PC2 (network signalling value NS_47/NS_01) for CA_n41-n79

| Test ID | PPowerclass | MPRc (dB) |  | A-MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 3.5 | 0 | 5 | 0 | 0 | 0 | 3 | 26 | 3 | 28+TT | 23-TT |
| 2 | 26 | 3.5 | 6.5 | 10 | 0 | 0 | 0 | 3 | 21 | 5 | 28+TT | 16-TT |
| 3 | 26 | 3.5 | 0 | 10 | 0 | 0 | 0 | 3 | 26 | 3 | 28+TT | 23-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-4: Void

Table 6.2A.3.1.5-5: Void

Table 6.2A.3.1.5-6: Void

Table 6.2A.3.1.5-7: UE Power Class 3 test requirement (network signalling value CA_NS_04) for CA_n41C

| Test ID | PPowerclass | MPR (dB) | AMPRcc (dB) | AMPR(dB) | ΔTC,c (dB) | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) |  |  |  |  |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 8 | - | 8 | 0 | 2 | 15 | 5 | 25+TT | 10-TT |
| 2 | 23 | 8 | - | 8 | 1.5 | 2 | 15 | 5 | 25+TT | 10-TT |
| 3 | 23 | 8 | - | 8 | 0 | 2 | 15 | 5 | 25+TT | 10-TT |
| 4 | 23 | 8 | - | 8 | 1.5 | 2 | 15 | 5 | 25+TT | 10-TT |
| 5 | 23 | 2.5 | 3.5 | 3.5 | 0 | 2 | 19.5 | 3.5 | 25+TT | 16-TT |
| 6 | 23 | 2.5 | 4 | 4 | 0 | 2 | 19 | 3.5 | 25+TT | 15.5-TT |
| 7 | 23 | 2.5 | 4 | 4 | 0 | 2 | 19 | 3.5 | 25+TT | 15.5-TT |
| 8 | 23 | 5 | 4.5 | 5 | 0 | 2 | 18 | 4 | 25+TT | 14-TT |
| 9 | 23 | 7 | 6 | 7 | 0 | 2 | 16 | 5 | 25+TT | 11-TT |
| 10 | 23 | 3.5 | 5.5 | 5.5 | 0 | 2 | 17.5 | 5 | 25+TT | 12.5-TT |
| 11 | 23 | 3.5 | 5.5 | 5.5 | 0 | 2 | 17.5 | 5 | 25+TT | 12.5-TT |
| 12 | 23 | 5 | 5.5 | 5.5 | 0 | 2 | 17.5 | 5 | 25+TT | 12.5-TT |
| 13 | 23 | 7 | 8 | 8 | 0 | 2 | 15 | 5 | 25+TT | 10-TT |

Table 6.2A.3.1.5-8: UE Power Class 2 not supporting TxD test requirement (network signalling value CA_NS_04) for CA_n41C

| Test ID | PPowerclass | MPR (dB) | AMPRcc (dB) | AMPR(dB) | ΔTC,c (dB) | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) |  |  |  |  |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 8 | - | 8 | 0 | 3 | 18 | 4 | 28+TT | 14-TT |
| 2 | 26 | 8 | - | 8 | 1.5 | 3 | 18 | 4 | 28+TT | 1-TT |
| 3 | 26 | 8 | - | 8 | 0 | 3 | 18 | 4 | 28+TT | 14-TT |
| 4 | 26 | 8 | - | 8 | 1.5 | 3 | 18 | 4 | 28+TT | 14-TT |
| 5 | 26 | 2.5 | 5.5 | 5.5 | 0 | 3 | 20.5 | 2.5 | 28+TT | 17.5-TT |
| 6 | 26 | 2.5 | 6 | 6 | 0 | 3 | 20 | 2.5 | 28+TT | 17-TT |
| 7 | 26 | 2.5 | 6 | 6 | 0 | 3 | 20 | 2.5 | 28+TT | 17-TT |
| 8 | 26 | 5 | 6.5 | 6.5 | 0 | 3 | 19.5 | 3.5 | 28+TT | 16-TT |
| 9 | 26 | 7 | 8 | 8 | 0 | 3 | 18 | 4 | 28+TT | 14-TT |
| 10 | 26 | 3.5 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 11 | 26 | 3.5 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 12 | 26 | 5 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 13 | 26 | 7 | 10 | 10 | 0 | 3 | 16 | 5 | 28+TT | 11-TT |

Table 6.2A.3.1.5-8a: UE Power Class 2 supporting TxD test requirement (network signalling value CA_NS_04) for CA_n41C

| Test ID | PPowerclass | MPR (dB) | AMPRcc (dB) | AMPR(dB) | ΔTC,c (dB) | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) |  |  |  |  |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 8.5 | - | 8.5 | 0 | 3 | 17.5 | 5 | 28+TT | 12.5-TT |
| 2 | 26 | 8.5 | - | 8.5 | 1.5 | 3 | 17.5 | 5 | 28+TT | 12.5-TT |
| 3 | 26 | 8.5 | - | 8.5 | 0 | 3 | 17.5 | 5 | 28+TT | 12.5-TT |
| 4 | 26 | 8.5 | - | 8.5 | 1.5 | 3 | 17.5 | 5 | 28+TT | 12.5-TT |
| 5 | 26 | 3.5 | 5.5 | 5.5 | 0 | 3 | 20.5 | 2.5 | 28+TT | 18-TT |
| 6 | 26 | 3.5 | 6 | 6 | 0 | 3 | 20 | 2.5 | 28+TT | 17.5-TT |
| 7 | 26 | 3.5 | 6 | 6 | 0 | 3 | 20 | 2.5 | 28+TT | 17.5-TT |
| 8 | 26 | 6 | 6.5 | 6.5 | 0 | 3 | 19.5 | 3.5 | 28+TT | 16-TT |
| 9 | 26 | 8 | 8 | 8 | 0 | 3 | 18 | 4 | 28+TT | 14-TT |
| 10 | 26 | 4.0 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 11 | 26 | 4.0 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 12 | 26 | 5.5 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 13 | 26 | 7.5 | 10 | 10 | 0 | 3 | 16 | 5 | 28+TT | 11-TT |

Table 6.2A.3.1.5-9: UE Power Class 3 test requirement (network signalling value NS_17/NS_01) for CA_n28-n78 and CA_n28-n79

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 6.5 | 0 | 0 | 0 | 0 | 3 | 16.5 | 5 | 25+TT | 11.5-TT |
| 3 | 23 | 6.5 | 0 | 0 | 0 | 0 | 0 | 3 | 20 | 5 | 25+TT | 15-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-10: UE Power Class 2 test requirement (network signalling value NS_17/NS_01) for CA_n28-n79

| Test ID | PPowerclass | PPowerclass,c |  | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW (PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 23 | 26 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 26 | 3 | 28+TT | 23-TT |
| 2 | 26 | 23 | 26 | 6.5 | 6.5 | 0 | 0 | 0 | 0 | 3 | 21.2 | 5 | 28+TT | 16.2-TT |
| 3 | 26 | 23 | 26 | 6.5 | 0 | 0 | 0 | 0 | 0 | 3 | 26 | 3 | 28+TT | 23-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-11: UE Power Class 3 test requirement (network signalling value NS_17/NS_47) for CA_n28-n41

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 1 | 0 | 2 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 1 | 0 | 7 | 0 | 0 | 3 | 19 | 5 | 25+TT | 14-TT |
| 3 | 23 | 0 | 1 | 0 | 7 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-11a: UE Power Class 2 test requirement with both bands supporting PC3 (network signalling value NS_17/NS_47) for CA_n28-n41

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 0 | 1 | 0 | 2 | 0 | 0 | 3 | 25 | 3 | 28+TT | 22-TT |
| 2 | 26 | 6.5 | 1 | 0 | 7 | 0 | 0 | 3 | 19 | 5 | 28+TT | 14-TT |
| 3 | 26 | 0 | 1 | 0 | 7 | 0 | 0 | 3 | 23.8 | 3 | 28+TT | 20.8-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-11b: UE Power Class 2 test requirement with with PCC supporting PC3 and SCC supporting PC2 (network signalling value NS_17/NS_47) for CA_n28-n41

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 0 | 3.5 | 0 | 5 | 0 | 0 | 3 | 25 | 3 | 28+TT | 22-TT |
| 2 | 26 | 6.5 | 3.5 | 0 | 10 | 0 | 0 | 3 | 19 | 5 | 28+TT | 14-TT |
| 3 | 26 | 0 | 3.5 | 0 | 10 | 0 | 0 | 3 | 23.8 | 3 | 28+TT | 20.8-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

### 6.2A.4	Configured output power for CA

#### 6.2A.4.0	Minimum conformance requirements

##### 6.2A.4.0.1	Configured transmitted power level

6.2A.4.0.1.1	Void

6.2A.4.0.1.2	Void

6.2A.4.0.1.3	Configured transmitted power for Inter-band CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in subclause 6.2.4.3.

For uplink inter-band carrier aggregation, MPRc and A-MPRc apply per serving cell c and are specified in subclause 6.2.2.3 and subclause 6.2.3.3, respectively. P-MPR c accounts for power management for serving cell c. PCMAX,c  is calculated under the assumption that the transmit power is increased independently on all component carriers.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

For uplink inter-band carrier aggregation with one serving cell c per operating band when same slot symbol pattern is used in all aggregated serving cells,

PCMAX_L = MIN {10log10∑ MIN [ pEMAX,c/ (tC,c), pPowerClass,c/(MAX(mprc,a-mprc)·tC,c ·tIB,c·tRxSRS,c), pPowerClass,c/pmprc], PEMAX,CA, PPowerClass,CA-ΔPPowerClass, CA}

PCMAX_H = MIN{10 log10 ∑ pEMAX,c, PEMAX,CA, PPowerClass,CA-ΔPPowerClass, CA}

where

-	pEMAX,c is the linear value of PEMAX, c which is given by IE P-Max for serving cell c in [6];

-	PPowerClass,CA is the maximum UE power specified in Table 6.2A.1.0.3-1 without taking into account the tolerance specified in the Table 6.2A.1.0.3-1; If the UE indicates higherPowerLimit-r17 for an eligible CA configuration as specified in Table 6.2A.1.3-1 and ΔPPowerClass, CA = 0, PPowerClass,CA is replaced by 10 log10 ∑ pPowerClass,c.

-	pPowerClass,c is the linear value of the maximum UE power for serving cell c specified in Table 6.2.1.3-1 according to ue-PowerClassPerBandPerBC-r17  if indicated or ue-PowerClass otherwise without taking into account the tolerance;

-	ΔPPowerClass, CA = 3 dB for a power class 2 UE when the requirements of default power class are applied as specified in sub-clause 6.2.A.1.3; otherwise ΔPPowerClass, CA = 0 dB;

-	mpr c and a-mpr c are the linear values of MPR c and A-MPR c as specified in subclause 6.2.2.3 and subclause 6.2.3.3, respectively;

-	pmprc is the linear value of P-MPR c;

-	∆tRxSRS,c  is the linear value of ∆TRxSRS,c;

-	tC,c is the linear value of TC,ctC,c = 1.41 when NOTE 2 in Table 6.2A.1.0.3-1 applies for a serving cell c, otherwise tC,c = 1;

-	tIB,c  is the linear value of the inter-band relaxation term TIB,c of the serving cell c as specified in clause 6.2A.4.0.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 [4] clause 6.2B.4.2 for EN-DC; otherwise tIB,c In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional TIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [4], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ∆TIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ∆TIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [4] for the applicable operating bands.

-	PEMAX,CA is p-UE-FR1 value signalled by RRC and defined in [6]

For uplink inter-band carrier aggregation with one serving cell c per operating band when at least one different numerology/slot pattern is used in aggregated cells, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of slot numerology type i, and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c(i),i (p) in slot p of serving cell c(i) on slot numerology type i shall be set within the following bounds:

PCMAX_L,f,c(i),i (p) ≤  PCMAX,f,c(i), i (p) ≤  PCMAX_H,f,c(i),i (p)

where PCMAX_L,f,c (i),i (p) and PCMAX_H,f,c(i),i (p) are the limits for a serving cell c(i) of slot numerology type i as specified in subclause 6.2.4.3.

The total UE configured maximum output power PCMAX (p,q) in a slot p of slot numerology or symbol pattern i,  and a slot q of slot numerology or symbol pattern j that overlap in time shall be set within the following bounds unless stated otherwise:

PCMAX_L(p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

When slots p and q have different transmissions lengths and belong to different cells on different bands:

PCMAX_L (p,q) = MIN {10 log10 [pCMAX_L,f,c(i),i (p) + pCMAX_L,f,c(i),j (q)], PPowerClass}

PCMAX_H (p,q) = MIN {10 log10 [pCMAX_H,f,c(i),i (p) + pCMAX_H,f,c(i),j (q)], PPowerClass}

where pCMAX_L,f,c (i),i  and pCMAX_H,f,c(i),i  are the respective limits PCMAX_L,f,c (i),i and PCMAX_H,f,c(i),i expressed in linear scale and pPowerClass,c is the linear value of the maximum UE power for serving cell c specified in Table 6.2.1-1 according to ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise without taking into account the tolerance; If the UE indicates higherPowerLimit-r17, PPowerClass,CA is replaced by 10 log10 ∑ pPowerClass,c.

TREF and Teval are specified in Table 6.2A.4.0.1.3-0 when same and different slot patterns are used in aggregated carriers. For each TREF, the PCMAX_L is evaluated per Teval and given by the minimum value taken over the transmission(s) within the Teval; the minimum PCMAX_L over the one or more Teval is then applied for the entire TREF. PPowerClass shall not be exceeded by the UE during any period of time.

Table 6.2A.4.0.1.3-0: PCMAX evaluation window for different slot and channel durations

| TREF | Teval | Teval with frequency hopping |
| --- | --- | --- |
| TREF of largest slot duration over both UL CCs | Physical channel length | Min(Tno_hopping, Physical Channel Length) |

If the UE is configured with multiple TAGs and transmissions of the UE on slot i for any serving cell in one TAG overlap some portion of the first symbol of the transmission on slot i +1 for a different serving cell in another TAG, the UE minimum of PCMAX_L for slots i and i + 1 applies for any overlapping portion of slots i and i + 1. PPowerClass shall not be exceeded by the UE during any period of time.

The measured maximum output power PUMAX over all serving cells with same slot pattern shall be within the following range:

PCMAX_L  – MAX{TL, TLOW(PCMAX_L) }  ≤  PUMAX  ≤  PCMAX_H  +  THIGH(PCMAX_H)

PUMAX = 10 log10 ∑ pUMAX,c

where pUMAX,c  denotes the measured maximum output power for serving cell c expressed in linear scale. The tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX are specified in Table 6.2A.4.0.1.3-1. The tolerance TL is the absolute value of the lower tolerance for applicable NR CA configuration as specified in Table 6.2A.1.0.3-1 for inter-band carrier aggregation.

The measured maximum output power PUMAX over all serving cells, when at least one slot has a different transmission numerology or symbol pattern, shall be within the following range:

P’CMAX_L–  MAX{TL, TLOW (P’CMAX_L)} ≤  P’UMAX  ≤  P’CMAX_H + THIGH (P’CMAX_H)

P’UMAX = 10 log10 ∑ p’UMAX,c

where p’UMAX,c  denotes the average measured maximum output power for serving cell c expressed in linear scale over TREF. The tolerances TLOW(P’CMAX) and THIGH(P’CMAX) for applicable values of P’CMAX are specified in Table 6.2A.4.0.1.3-1 for inter-band carrier aggregation. The tolerance TL is the absolute value of the lower tolerance for applicable NR CA configuration as specified in Table 6.2A.1.0.3-1 for inter-band carrier aggregation.

where:

P’CMAX_L  = MIN{ MIN {10log10∑(pCMAX_L,f,c(i),i), PPowerClass} over all overlapping slots in TREF}

P’CMAX_H = MAX{ MIN{10 log10 ∑ pEMAX,c, PPowerClass} over all overlapping slots in TREF}

If the UE indicates higherPowerLimit-r17, PPowerClass,CA is replaced by 10 log10 ∑ pPowerClass,c

Table 6.2A.4.0.1.3-1: PCMAX tolerance for uplink inter-band CA (two bands)

| PCMAX (dBm) | Tolerance TLOW(PCMAX) (dB) | Tolerance THIGH(PCMAX) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 29 | 3.0 | 2.0 |
| 22 ≤ PCMAX < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX < 21 | 5.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAX < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |

6.2A.4.0.1.4	Configured transmitted power for Intra-band contiguous CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in subclause 6.2.4, but with MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR as determined by subclause 6.2A.2.0 and 6.2A.3.0 respectively. For PH reporting the following exception applies: if the UE is configured with multiple uplink serving cells, the power PCMAX,c  used for the purpose of PH reporting on first serving cell c = c1 does not consider for computation of the PH report transmissions on a second serving cell c2 as exempted in subclause 7.7.1 in [8]. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

For uplink intra-band contiguous carrier aggregation when same slot pattern is used in all aggregated serving cells,

PCMAX_L  = MIN{10 log10 ∑ pEMAX,c  - TC, PEMAX,CA,(PPowerClass,CA - – ΔPPowerClass,CA) – MAX(MAX(MPR, A-MPR) + ΔTIB,c + TC + TRxSRS, P-MPRc ) }

PCMAX_H  = MIN{10 log10 ∑ pEMAX,c, PEMAX,CA,PPowerClass,CA – ΔPPowerClass,CA }

where

-	pEMAX,c is the linear value of PEMAX,c which is given by IE P-Max for serving cell c in [7];

-	PPowerClass,CA is the maximum UE power specified in Table 6.2A.1.0.4-1 without taking into account the tolerance;

-	MPR and A-MPR are specified in subclause 6.2A.2.0 and 6.2A.3.0 respectively;

-	ΔPPowerClass,CA = 3 dB for a power class 2 UE when 10 log10 ∑ pEMAX,c of 23 dBm or lower is indicated; or when PEMAX,CA  of 23dBm or lower is indicated; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than 50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of total uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); otherwise ΔPPowerClass,CA = 0 dB;

-	TIB,c is the additional tolerance for serving cell c as specified in clause 6.2A.4.0.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 [4] clause 6.2B.4.2 for EN-DC In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ∆TIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [4], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ∆TIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ∆TIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [4] for the applicable operating bands.;

-	P-MPR is the power management term for the UE;

-	DTC is the highest value DTC,c among all serving cells c;

-	∆TRxSRS is the highest value among all serving cells c.

-	PEMAX,CA is the value indicated by p-NR-FR1 or by p-UE-FR1 whichever is the smallest if both are present.

For uplink intra-band contiguous carrier aggregation, when at least one different numerology/slot pattern is used in aggregated cells, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of slot numerology type i, and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c(i),i (p) in slot p of serving cell c(i) on slot numerology type i shall be set within the following bounds:

PCMAX_L,f,c(i),i (p) ≤  PCMAX,f,c(i), i (p) ≤  PCMAX_H,f,c(i),i (p)

where PCMAX_L,f,c (i),i (p) and PCMAX_H,f,c(i),i (p) are the limits for a serving cell c(i) of slot numerology type i as specified in subclause 6.2.4.

The total UE configured maximum output power PCMAX (p,q) in a slot p of slot numerology or symbol pattern i,  and a slot q of slot numerology or symbol pattern j that overlap in time shall be set within the following bounds unless stated otherwise:

PCMAX_L(p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

When slots p and q have different transmissions lengths and belong to different cells on different or same bands:

PCMAX_L (p,q) = MIN {10 log10 [pCMAX_L,f,c(i),i (p) + pCMAX_L,f,c(i),j (q)], PPowerClass,CA, PEMAX,CA}

PCMAX_H (p,q) = MIN {10 log10 [pCMAX_H,f,c(i),i (p) + pCMAX_H,f,c(i),j (q)], PPowerClass,CA, PEMAX,CA}

where pCMAX_L,f,c (i),i  and pCMAX_H,f,c(i),i  are the respective limits PCMAX_L,f,c (i),i and PCMAX_H,f,c(i),i expressed in linear scale.

TREF and Teval are specified in Table 6.2A.4.0.1.4-0 when same and different slot patterns are used in aggregated carriers. For each TREF, the PCMAX_L is evaluated per Teval and given by the minimum value taken over the transmission(s) within the Teval; the minimum PCMAX_L over the one or more Teval is then applied for the entire TREF. The lesser of PPowerClass,CA and PEMAX,CA shall not be exceeded by the UE during any period of time.

Table 6.2A.4.0.1.4-0: PCMAX evaluation window for different slot and channel durations

| TREF | Teval | Teval with frequency hopping |
| --- | --- | --- |
| TREF of largest slot duration over both UL CCs | Physical channel length | Min(Tno_hopping, Physical Channel Length) |

If the UE is configured with multiple TAGs and transmissions of the UE on slot i for any serving cell in one TAG overlap some portion of the first symbol of the transmission on slot i +1 for a different serving cell in another TAG, the UE minimum of PCMAX_L for slots i and i + 1 applies for any overlapping portion of slots i and i + 1. The lesser of PPowerClass,CA and PEMAX,CA shall not be exceeded by the UE during any period of time.

The measured maximum output power PUMAX over all serving cells with same slot pattern shall be within the following range:

PCMAX_L  – MAX{TL, TLOW(PCMAX_L) }  ≤  PUMAX  ≤  PCMAX_H  +  THIGH(PCMAX_H)

PUMAX = 10 log10 ∑ pUMAX,c

where pUMAX,c  denotes the measured maximum output power for serving cell c expressed in linear scale. The tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX are specified in Table 6.2A.4.0.1.4-1. The tolerance TL is the absolute value of the lower tolerance for applicable NR CA configuration as specified in Table 6.2A.1.0.4-1 for intra-band carrier aggregation.

The measured maximum output power PUMAX over all serving cells, when at least one slot has a different transmission numerology or slot pattern, shall be within the following range:

P'CMAX_L–  MAX{TL, TLOW (P'CMAX_L)} ≤  P'UMAX  ≤  P'CMAX_H + THIGH (P'CMAX_H)

P'UMAX = 10 log10 ∑ p'UMAX,c

where p'UMAX,c  denotes the average measured maximum output power for serving cell c expressed in linear scale over TREF. The tolerances TLOW(P'CMAX) and THIGH(P'CMAX) for applicable values of P'CMAX are specified in Table 6.2A.4.0.1.4-1 for intra-band carrier aggregation. The tolerance TL is the absolute value of the lower tolerance for applicable NR CA configuration as specified in Table 6.2A.1.0.4-1 for intra-band carrier aggregation.

where:

P'CMAX_L  = MIN{ MIN {10log10∑( pCMAX_L,f,c(i),i), PPowerClass,CA } over all overlapping slots in TREF}

P'CMAX_H = MAX{ MIN{10 log10 ∑ pEMAX,c, PPowerClass,CA } over all overlapping slots in TREF}

Table 6.2A.4.0.1.4-1: PCMAX tolerance for uplink intra-band contiguous CA

| PCMAX (dBm) | Tolerance TLOW(PCMAX) (dB) | Tolerance THIGH(PCMAX) (dB) |
| --- | --- | --- |
| 23 < PCMAX ≤ 26 | 3 | 2 |
| 21 ≤ PCMAX ≤ 23 | 2.0 |  |
| 20 ≤ PCMAX < 21 | 2.5 |  |
| 19 ≤ PCMAX < 20 | 3.5 |  |
| 18 ≤ PCMAX < 19 | 4.0 |  |
| 13 ≤ PCMAX < 18 | 5.0 |  |
| 8 ≤ PCMAX < 13 | 6.0 |  |
| -40 ≤ PCMAX < 8 | 7.0 |  |

6.2A.4.0.1.5	Configured transmitted power for Intra-band non-contiguous CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in subclause 6.2.4.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in subclause 6.2.4, but with MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR as determined by subclause 6.2A.2 and 6.2A.3, respectively. For PH reporting the following exception applies: if the UE is configured with multiple uplink serving cells, the power PCMAX,c  used for the purpose of PH reporting on first serving cell c = c1 does not consider for computation of the PH report transmissions on a second serving cell c2 as exempted  in subclause 7.7.1 in [8]. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

For uplink intra-band non-contiguous carrier aggregation when same slot pattern is used in all aggregated serving cells,

PCMAX_L  = MIN{10 log10 ∑ pEMAX,c  - TC , PEMAX,CA,PPowerClass,CA – MAX(MAX(MPR, A-MPR) + ΔTIB,c + TC + TRxSRS, P-MPR ) }

PCMAX_H  = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA ,PPowerClass,CA}

where

-	pEMAX,c is the linear value of PEMAX,c which is given by IE P-Max for serving cell c in [7];

-	PPowerClass,CA is the maximum UE power specified in Table 6.2A.1.2-1 without taking into account the tolerance;

-	MPR and A-MPR are specified in subclause 6.2A.2 and subclause 6.2A.3 respectively;

-	TIB,c is the additional tolerance for serving cell c as specified in clause 6.2A.4.0.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 clause  6.2B.4.2 for EN-DC; In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ∆TIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ∆TIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ∆TIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3] for the applicable operating bands.

-	P-MPR is the power management term for the UE;

-	TC is the highest value TC,c among all serving cells c;

- 	∆TRxSRS is the highest value among all serving cells c;

-	PEMAX,CA is the value indicated by p-NR-FR1 or by p-UE-FR1 whichever is the smallest if both are present.[For uplink intra-band non-contiguous carrier aggregation, when at least one different numerology/slot pattern is used in aggregated cells, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of slot numerology type i, and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c(i),i (p) in slot p of serving cell c(i) on slot numerology type i shall be set within the following bounds:

PCMAX_L,f,c(i),i (p) ≤  PCMAX,f,c(i), i (p) ≤  PCMAX_H,f,c(i),i (p)

where PCMAX_L,f,c (i),i (p) and PCMAX_H,f,c(i),i (p) are the limits for a serving cell c(i) of slot numerology type i as specified in subclause 6.2.4.

The total UE configured maximum output power PCMAX (p,q) in a slot p of slot numerology or symbol pattern i,  and a slot q of slot numerology or symbol pattern j that overlap in time shall be set within the following bounds unless stated otherwise:

PCMAX_L(p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

When slots p and q have different transmissions lengths and belong to different cells on different or same bands:

PCMAX_L (p,q) = MIN {10 log10 [pCMAX_L,f,c(i),i (p) + pCMAX_L,f,c(i),j (q)], PPowerClass,CA, PEMAX,CA}

PCMAX_H (p,q) = MIN {10 log10 [pCMAX_ H,f,c(i),i (p) + pCMAX_ H,f,c(i),j (q)], PPowerClass,CA, PEMAX,CA}

where pCMAX_L,f,c (i),i  and pCMAX_ H,f,c(i),i  are the respective limits PCMAX_L,f,c (i),i and PCMAX_H,f,c(i),i expressed in linear scale.]

TREF and Teval are specified in Table 6.2A.4.1.2-1 when same and different slot patterns are used in aggregated carriers. For each TREF, the PCMAX_L is evaluated per Teval and given by the minimum value taken over the transmission(s) within the Teval; the minimum PCMAX_L over the one or more Teval is then applied for the entire TREF. The lesser of PPowerClass,CA and PEMAX,CA shall not be exceeded by the UE during any period of time.

Table 6.2A.4.0.1.5-1: PCMAX evaluation window for different slot and channel durations

| TREF | Teval | Teval with frequency hopping |
| --- | --- | --- |
| TREF of largest slot duration over both UL CCs | Physical channel length | Min(Tno_hopping, Physical Channel Length) |

If the UE is configured with multiple TAGs and transmissions of the UE on slot i for any serving cell in one TAG overlap some portion of the first symbol of the transmission on slot i +1 for a different serving cell in another TAG, the UE minimum of PCMAX_L for slots i and i + 1 applies for any overlapping portion of slots i and i + 1. The lesser of PPowerClass,CA and PEMAX,CA shall not be exceeded by the UE during any period of time.

The measured maximum output power PUMAX over all serving cells with same slot pattern shall be within the following range:

PCMAX_L  – MAX{TL, TLOW(PCMAX_L) }  ≤  PUMAX  ≤  PCMAX_H  +  THIGH(PCMAX_H)

PUMAX = 10 log10 ∑ pUMAX,c

where pUMAX,c  denotes the measured maximum output power for serving cell c expressed in linear scale. The tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX are specified in Table 6.2A.4.0.1.5-2. The tolerance TL is the absolute value of the lower tolerance for applicable NR CA configuration as specified in Table 6.2A.1.0.5-1 for intra-band carrier aggregation.

The measured maximum output power PUMAX over all serving cells, when at least one slot has a different transmission numerology or slot pattern, shall be within the following range:

P'CMAX_L–  MAX{TL, TLOW (P'CMAX_L)} ≤  P'UMAX  ≤  P'CMAX_H + THIGH (P'CMAX_H)

P'UMAX = 10 log10 ∑ p'UMAX,c

where p'UMAX,c  denotes the average measured maximum output power for serving cell c expressed in linear scale over TREF. The tolerances TLOW(P'CMAX) and THIGH(P'CMAX) for applicable values of P'CMAX are specified in Table 6.2A.4.0.1.5-2 for intra-band carrier aggregation. The tolerance TL is the absolute value of the lower tolerance for applicable NR CA configuration as specified in Table 6.2A.1.0.5-1 for intra-band carrier aggregation.

where:

P'CMAX_L  = MIN{ MIN {10log10∑( pCMAX_L,f,c(i),i), PPowerClass,CA} over all overlapping slots in TREF}

P'CMAX_H = MAX{ MIN{10 log10 ∑ pEMAX,c , PPowerClass,CA} over all overlapping slots in TREF}

Table 6.2A.4.0.1.5-2: PCMAX tolerance for uplink intra-band non-contiguous CA

| PCMAX (dBm) | Tolerance TLOW(PCMAX) (dB) | Tolerance THIGH(PCMAX) (dB) |
| --- | --- | --- |
| 21 ≤ PCMAX ≤ 23 | 3.0 | 2.0 |
| 20 ≤ PCMAX < 21 | 2.5 |  |
| 19 ≤ PCMAX < 20 | 3.5 |  |
| 18 ≤ PCMAX < 19 | 4.0 |  |
| 13 ≤ PCMAX < 18 | 5.0 |  |
| 8 ≤ PCMAX < 13 | 6.0 |  |
| -40 ≤ PCMAX < 8 | 7.0 |  |

##### 6.2A.4.0.2	ΔTIB,c for CA

For the UE which supports inter-band NR CA configuration, ΔTIB,c in tables below applies. Unless otherwise stated, ΔTIB,c is set to zero.

##### 6.2A.4.0.2.1	FFS

##### 6.2A.4.0.2.2	FFS

##### 6.2A.4.0.2.3	ΔTIB,c for Inter-band CA

Table 6.2A.4.0.2.3-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination |  | NR Band | ΔTIB,c (dB) |  |
| --- | --- | --- | --- | --- |
| CA_n1-n3 |  | n1 | 0.3 |  |
|  |  | n3 | 0.3 |  |
| CA_n1-n5 |  | n1 | 0.3 |  |
|  |  | n5 | 0.3 |  |
| CA_n1-n8 |  | n1 | 0.3 |  |
|  |  | n8 | 0.3 |  |
| CA_n1-n28 |  | n1 | 0.3 |  |
|  |  | n28 | 0.6 |  |
| CA_n1-n41 |  | n1 | 0.5 |  |
|  |  | n41 | 0.5 |  |
| CA_n1-n77 |  | n1 | 0.6 |  |
|  |  | n77 | 0.8 |  |
| CA_n1-n78 |  | n1 | 0.3 |  |
|  |  | n78 | 0.8 |  |
| CA_n2-n5 |  | n2 | 0.3 |  |
|  |  | n5 | 0.3 |  |
| CA_n2-n14 |  | n2 | 0.3 |  |
|  |  | n14 | 0.3 |  |
| CA_n2-n30 |  | n2 | 0.5 |  |
|  |  | n30 | 0.3 |  |
| CA_n2-n48 |  | n2 | 0.6 |  |
|  |  | n48 | 0.8 |  |
| CA_n2-n66 |  | n2 | 0.5 |  |
|  |  | n66 | 0.5 |  |
| CA_n2-n77 |  | n2 | 0.6 |  |
|  |  | n77 | 0.8 |  |
| CA_n3-n5 |  | n3 | 0.3 |  |
|  |  | n5 | 0.3 |  |
| CA_n3-n8 |  | n3 | 0.3 |  |
|  |  | n8 | 0.3 |  |
| CA_n3-n28 |  | n3 | 0.3 |  |
|  |  | n28 | 0.3 |  |
| CA_n3-n40 |  | n3 | 0.5 |  |
|  |  | n40 | 0.5 |  |
| CA_n3-n41 |  | n3 | 0.5 |  |
|  |  | n41 | 0.34 |  |
|  |  |  | 0.85 |  |
| CA_n3-n77 |  | n3 | 0.6 |  |
|  |  | n77 | 0.8 |  |
| CA_n3-n78 |  | n3 | 0.6 |  |
|  |  | n78 | 0.8 |  |
| CA_n5-n30 |  | n5 | 0.3 |  |
|  |  | n30 | 0.3 |  |
| CA_n5-n48 |  | n5 | 0.3 |  |
|  |  | n48 | 0.3 |  |
| CA_n5-n66 |  | n5 | 0.3 |  |
|  |  | n66 | 0.3 |  |
| CA_n5-n77 |  | n5 | 0.6 |  |
|  |  | n77 | 0.8 |  |
| CA_n5-n78 |  | n5 | 0.6 |  |
|  |  | n78 | 0.8 |  |
| CA_n8-n78 |  | n8 | 0.6 |  |
|  |  | n78 | 0.8 |  |
| CA_n14-n30 |  | n14 |  | 0.3 |
|  |  | n30 |  | 0.3 |
| CA_n14-n66 |  | n14 |  | 0.3 |
|  |  | n66 |  | 0.3 |
| CA_n14-n77 |  | n14 |  | 0.5 |
|  |  | n77 |  | 0.8 |
| CA_n20-n78 |  | n20 | 0.6 |  |
|  |  | n78 | 0.8 |  |
|  |  | n24 | 0.3 |  |
| CA_n24-n41 |  | n41 | 0.44 |  |
|  |  |  | 0.95 |  |
| CA_n24-n48 |  | n24 | 0.6 |  |
|  |  | n48 | 0.8 |  |
| CA_n24-n77 |  | n24 | 0.6 |  |
|  |  | n77 | 0.8 |  |
| CA_n25-n66 |  | n25 | 0.5 |  |
|  |  | n66 | 0.5 |  |
| CA_n25-n77 |  | n25 | 0.6 |  |
|  |  | n77 | 0.8 |  |
| CA_n28-n40 |  | n28 | 0.3 |  |
|  |  | n40 | 0.3 |  |
| CA_n28-n41 |  | n28 | 0.3 |  |
|  |  | n41 | 0.3 |  |
| CA_n28-n77 |  | n28 | 0.5 |  |
|  |  | n77 | 0.8 |  |
| CA_n28-n78 |  | n28 | 0.5 |  |
|  |  | n78 | 0.8 |  |
| CA_n28-n79 |  | n28 | 0.5 |  |
|  |  | n79 | 0.8 |  |
| CA_n30-n66 |  | n30 | 0.5 |  |
|  |  | n66 | 0.8 |  |
| CA_n30-n77 |  | n30 | 0.3 |  |
|  |  | n77 | 0.8 |  |
| CA_n39-n41 |  | n39 | 02 |  |
|  |  | n41 | 02 |  |
|  |  | n39 | 0.5 |  |
|  |  | n41 | 0.5 |  |
| CA_n40-n77 |  | n77 | 0.5 |  |
| CA_n41-n771 |  | n41 | 0.3 |  |
|  |  | n77 | 0.8 |  |
| CA_n41-n79 |  | n41 | 0.3 |  |
|  |  | n79 | 0.8 |  |
| CA_n48-n66 |  | n48 | 0.8 |  |
|  |  | n66 | 0.6 |  |
| CA_n66-n70 |  | n66 | 0.5 |  |
|  |  | n70 | 0.5 |  |
| CA_n66-n71 |  | n66 | 0.3 |  |
|  |  | n71 | 0.3 |  |
| CA_n66-n77 |  | n66 | 0.6 |  |
|  |  | n77 | 0.8 |  |
| CA_n66-n78 |  | n66 | 0.6 |  |
|  |  | n78 | 0.8 |  |
| CA_n70-n71 |  | n70 | 0.3 |  |
|  |  | n71 | 0.6 |  |
| CA_n71-n77 |  | n71 | 0.5 |  |
|  |  | n77 | 0.8 |  |
| CA_n71-n78 |  | n71 | 0.5 |  |
|  |  | n78 | 0.8 |  |
| CA_n77-n79 |  | n77 | 0.5 |  |
|  |  | n79 | 0.5 |  |
|  |  | n78 | 0.5 |  |
| CA_n78-n79 |  |  | 1.56 |  |
|  |  | n79 | 0.5 |  |
|  |  |  | 1.56 |  |
| NOTE 1: The requirements only apply when the sub-frame and Tx-Rx timings are synchronized between the component carriers. In the absence of synchronization, the requirements are not within scope of these specifications. NOTE 2: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. NOTE 6: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |

Table 6.2A.4.0.2.3-2: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n1-n3-n77 | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |
| CA_n1-n3-n78 | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| CA_n1-n5-n78 | n1 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |
| CA_n1-n8-n78 | n1 | 0.3 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |
| CA_n1-n28-n77 | n1 | 0.6 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |
| CA_n1-n41-n77 | n1 | 0.5 |
|  | n41 | 0.5 |
|  | n77 | 0.8 |
| CA_n2-n5-n77 | n2 | 0.6 |
|  | n5 | 0.8 |
|  | n77 | 0.8 |
| CA_n2-n5-n48 | n2 | 0.6 |
|  | n5 | 0.3 |
|  | n48 | 0.8 |
| CA_n2-n5-n66 | n2 | 0.5 |
|  | n5 | 0.3 |
|  | n66 | 0.5 |
| CA_n2-n48-n66 | n2 | 0.6 |
|  | n48 | 0.8 |
|  | n66 | 0.6 |
| CA_n2-n48-n77 | n2 | 0.6 |
|  | n48 | 0.8 |
|  | n77 | 0.8 |
| CA_n2-n66-n77 | n2 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n3-n5-n78 | n3 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |
| CA_n3-n8-n78 | n3 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |
| CA_n3-n28-n40 | n3 | 0.5 |
|  | n28 | 0.3 |
|  | n40 | 0.5 |
| CA_n3-n28-n41 | n3 | 0.5 |
|  | n28 | 0.3 |
|  | n41 | 0.31/0.82 |
| CA_n3-n28-n77 | n3 | 0.6 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |
| CA_n3-n40-n77 | n3 | 0.6 |
|  | n40 | 0.5 |
|  | n77 | 0.8 |
| CA_n3-n41-n77 | n3 | 0.6 |
|  | n41 | 0.31/0.82 |
|  | n77 | 0.8 |
| CA_n5-n48-n66 | n5 | 0.3 |
|  | n48 | 0.8 |
|  | n66 | 0.6 |
| CA_n5-n48-n77 | n5 | 0.6 |
|  | n48 | 0.8 |
|  | n77 | 0.8 |
| CA_n5-n66-n77 | n5 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n28-n40-n77 | n28 | 0.5 |
|  | n40 | 0.3 |
|  | n77 | 0.8 |
| CA_n28-n41-n77 | n28 | 0.5 |
|  | n41 | 0.3 |
|  | n77 | 0.8 |
| CA_n28-n41-n79 | n28 | 0.5 |
|  | n41 | 0.3 |
|  | n79 | 0.8 |
| CA_n48-n66-n77 | n48 | 0.8 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. |  |  |

Table 6.2A.4.0.2.3-3: ΔTIB,c due to NR CA (four bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)5 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration6 |  |  |  |
| CA_n2-n5-n48-n66 | 0.6 | 0.3 | 0.8 | 0.6 |
| CA_n2-n5-n48-n77 | 0.6 | 0.3 | 0.8 | 0.8 |
| CA_n2-n5-n66-n77 | 0.5 | 0.3 | 0.5 | 0.8 |
| CA_n2-n48-n66-n77 | 0.6 | 0.8 | 0.6 | 0.8 |
| CA_n3-n28-n41-n77 | 1 | 0.5 | 0.31 / 0.82 | 0.8 |
| CA_n5-n48-n66-n77 | 0.6 | 0.8 | 0.6 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. NOTE 3: FFS NOTE 4: FFS NOTE 5: “-” denotes ΔTIB,c = 0. NOTE 6: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5-n78 the band order from left to right is n1, n3, n5 and n78. |  |  |  |  |

#### 6.2A.4.1	Configured transmitted power for CA (2UL CA)

6.2A.4.1.1	Test purpose

To verify that the total measured UE configured maximum output power PUMAX in two uplink carrier aggregation is within the specified bounds.

6.2A.4.1.2	Test applicability

The requirements of this test apply to all types of NR UE release 15 and forward that support NR 2 UL CA.

6.2A.4.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.2A.4.0.

6.2A.4.1.4	Test description

6.2A.4.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR CA configuration specified in 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2A.4.1.4.1-1 to table 6.2A.4.1.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2A.4.1.4.1-1: Inter-band CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range for PCC and SCC (NOTE 3) |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in 5.3.5-1 |  | Lowest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  | Modulation for all CCs (NOTE 2) | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A for minimum output power test case | DFT-s-OFDM Pi/2 BPSK | Inner Full | Inner Full |
| 2 |  | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |  |

Table 6.2A.4.1.4.1-2: Intra-band contiguous CA (contiguous RB allocation) Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test Parameters for CA bandwidth class B and C |  |  |  |  |
| Test ID | DL configuration for PCC & SCC | UL configuration |  |  |
|  |  | Modulations for all CCs (NOTE 2) |  | RB allocation (NOTE 3) |
| 1 | N/A | DFT-s-OFDM Pi/2 BPSK |  | Inner Full |
| 2 |  | DFT-s-OFDM QPSK |  | Inner Full |
| NOTE 1: The Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: The specific configuration of each RB allocation is defined in Table 6.1A-1a. NOTE 4: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |

Table 6.2A.4.1.4.1-3: Intra-band non-contiguous CA when UE supporting IE dualPA-Architecture Test Configuration

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | One frequency chosen arbitrarily from Low range or High range test frequencies as specified in tables for non-contiguous CA configuration with UL CA |  |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |  |  |  |  |  |
| Test SCS as specified in 5.3.5-1 |  | Lowest |  |  |  |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |  |  |  |  |
|  |  | Modulation for all CCs (NOTE 2) | RB allocation (NOTE 1) |  |  |  |  |  |
|  |  |  | SCS 15 kHz |  | SCS 30 kHz |  | SCS 60 kHz |  |
|  |  |  | PCC | SCC | PCC | SCC | PCC | SCC |
| 1 | N/A | DFT-s-OFDM Pi/2 BPSK | 45@0 | 45@0 | 24@0 | 24@0 | 12@0 | 12@0 |
| 2 |  | DFT-s-OFDM QPSK | 45@0 | 45@0 | 24@0 | 24@0 | 12@0 | 12@0 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Tables 6.2A.4.1.4.1-1, 6.2A.4.1.4.1-2 and Table 6.2A.4.1.4.1-3 as appropriate.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2A.4.1.4.3.

6.2A.4.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.2A.4.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 6.2A.4.1.4.1-1, 6.2A.4.1.4.1-2 and 6.2A.4.1.4.1-3 as appropriate on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level of the test point.

6.	Measure the sum of mean transmitted power over all component carriers in the CA configuration of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD, only slots consisting of only UL symbols are under test. For FDD band in inter-band CA with both TDD band and FDD band, only slots overlapping with only UL symbols in TDD are under test.

6.2A.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with the following exceptions.

Table 6.2A.4.1.4.3-0: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.2A.4.1.4.3-1: FrequencyInfoUL: Test point 1

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-61 FrequencyInfoUL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | -10 |  |  |

Table 6.2A.4.1.4.3-1a: FrequencyInfoUL-SIB: Test point 1

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | -10 |  |  |

Table 6.2A.4.1.4.3-2: FrequencyInfoUL: Test point 2

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-61 FrequencyInfoUL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 10 |  |  |

Table 6.2A.4.1.4.3-2a: FrequencyInfoUL-SIB: Test point 2

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 10 |  |  |

Table 6.2A.4.1.4.3-3: FrequencyInfoUL: Test point 3

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-61 FrequencyInfoUL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 5 | UL intra-band contiguous CA with contiguous RB allocation |  |

Table 6.2A.4.1.4.3-3a: FrequencyInfoUL-SIB: Test point 3

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 5 | UL intra-band contiguous CA with contiguous RB allocation |  |

Table 6.2A.4.1.4.3-4: FrequencyInfoUL: Test point 4

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-61 FrequencyInfoUL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 15 | UL intra-band contiguous CA with contiguous RB allocation |  |

Table 6.2A.4.1.4.3-4a: FrequencyInfoUL-SIB: Test point 4

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 15 | UL intra-band contiguous CA with contiguous RB allocation |  |

Table 6.2A.4.1.4.3-5: FrequencyInfoUL: Test point 5

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-61 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 9 | UL intra-band non-contiguous CA when MPRIM3 to meet -13dBm/MHz |  |

Table 6.2A.4.1.4.3-5a: FrequencyInfoUL-SIB: Test point 5

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 9 | UL intra-band non-contiguous CA when MPRIM3 to meet -13dBm/MHz |  |

6.2A.4.1.5	Test requirement

The maximum output power measured shall not exceed the values specified in Table 6.2A.4.1.5-1 to Table 6.2A.4.1.5-1b.

Table 6.2A.4.1.5-1: Configured UE Output Power for Inter-band CA

|  | Maximum output power |  |
| --- | --- | --- |
|  | Lower limit | Upper limit |
| Measured UE output power test point 1 | -14 dBm - TT | 0 dBm + TT |
| Measured UE output power test point 2 | 7 dBm - TT | 19 dBm + TT |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 6.2A.4.1.5-2. |  |  |

Table 6.2A.4.1.5-1a: Configured UE Output Power for Intra-band contiguous CA of PC2 and PC3 (contiguous RB allocation)

|  | Maximum output power |  |
| --- | --- | --- |
|  | Lower limit | Upper limit |
| Measured UE output power test point 1 | -14 dBm - TT | 0 dBm + TT |
| Measured UE output power test point 2 | 8 dBm - TT | 18 dBm + TT |
| Measured UE output power test point 3 | 2 dBm - TT | 14 dBm + TT |
| Measured UE output power test point 4 | 14 dBm - TT | 22 dBm + TT |
| Note 1: TT for each intra-band UL CA configuration is in table 6.2A.4.1.5-3. |  |  |

Table 6.2A.4.1.5-1b: Configured UE Output Power for Intra-band non-contiguous CA

|  | Maximum output power |  |
| --- | --- | --- |
|  | Lower limit | Upper limit |
| Measured UE output power test point 1 | -14 dBm - TT | 0 dBm + TT |
| Measured UE output power test point 5 (Note 2) | 6 dBm - TT | 18 dBm + TT |
| Note 1: TT for each intra-band UL CA configuration is in table 6.2A.4.1.5-3. Note 2: Only applicable when MPRIM3 to meet -13dBm/MHz |  |  |

Table 6.2A.4.1.5-2: Test Tolerance for Configured transmitted power for inter-band CA

| TT for overall output power (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |  |  |
|  |  |  | BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| SCell | BW ≤ 40MHz | f ≤ 3.0GHz | 0.71 1.02 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
| Note 1: Applicable for test point with UL power ≥ 0 dBm. Note 2: Applicable for test point with UL power < 0 dBm. |  |  |  |  |  |  |  |  |

Table 6.2A.4.1.5-3: Test Tolerance for Configured transmitted power for intra-band CA

| TT for overall output power (dB) |  |  |
| --- | --- | --- |
| Aggregation BW | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| BW ≤ 40MHz | 0.71 1.02 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |
| 100MHz < BW ≤ 200MHz | FFS | FFS |
| Note 1: Applicable for test point with UL power ≥ 0 dBm. Note 2: Applicable for test point with UL power < 0 dBm. |  |  |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

## 6.2B	Transmitter power for NR-DC

### 6.2B.0	General

The requirements apply for inter-band NR-DC with one uplink serving cell configured per CG.

### 6.2B.1	UE maximum output power for NR-DC

#### 6.2B.1.0	Minimum conformance requirements

##### 6.2B.1.0.1	FFS

##### 6.2B.1.0.2	FFS

##### 6.2B.1.0.3	UE maximum output power for inter-band NR-DC

For inter-band NR-DC with one uplink carrier assigned per NR band, the transmitter power requirements in clause 6.2.1.3 apply per band.

For inter-band NR-DC with one uplink assigned per band, the UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, the maximum output power is defined as the sum of maximum output power from each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). The maximum output power is specified in Table 6.2B.1.0.3-1.

Table 6.2B.1.0.3-1 UE Power Class for inter-band NR-DC

| Uplink DC Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_n48A-n70A |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: An uplink DC configuration in which at least one of the bands has NOTE 3 in Table 6.2.1.3-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: PPowerClass is the maximum UE power specified without account of the tolerance NOTE 3: The maximum power requirement applies to the total transmitted power over both the MCG and SCG. NOTE 4: Power class 3 is the default power class unless otherwise stated. |  |  |  |  |  |  |  |  |

#### 6.2B.1.1	UE maximum output power for NR-DC

6.2B.1.1.1	Test purpose

To verify that the error of the UE maximum output power in dual connectivity does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2B.1.1.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR-DC.

6.2B.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2B.1.0.

6.2B.1.1.4	Test description

6.2B.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR-DC configurations specified in 5.5B. All of NR-DC configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2B.1.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2B.1.1.4.1-1: Inter-band NR-DC Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range for PCell and PSCell High range for PCell and PSCell |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest for PCell and PSCell Highest for PCell and PSCell |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration for PCell & PSCell | Uplink Configuration |  |  |
|  |  | Modulation for all CCs | RB allocation (NOTE 1) |  |
|  |  | (NOTE 2) | PCell | PSCell |
| 1 | N/A for this test | DFT-s-OFDM Pi/2 BPSK | Inner Full | Inner Full |
| 2 |  | DFT-s-OFDM Pi/2 BPSK | Inner 1RB Left | Inner 1RB Left |
| 3 |  | DFT-s-OFDM Pi/2 BPSK | Inner 1RB Right | Inner 1RB Right |
| 4 |  | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 5 |  | DFT-s-OFDM QPSK | Inner 1RB Left | Inner 1RB Left |
| 6 |  | DFT-s-OFDM QPSK | Inner 1RB Right | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCell are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2B.1.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR-DC, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2B.1.1.4.3.

6.2B.1.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2B.1.1.4.1-1 on both PCell and PSCell for inter-band NR-DC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level corresponding to Power Class 3.

3.	Measure the sum of mean transmitted power over all component carriers in the NR-DC configuration of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbols with transient periods are not under test.

6.2B.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions.

Table 6.2B.1.1.4.3-1: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.2B.1.1.4.3-2: FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  | Power class 3 and Inter-band NR-DC |

6.2B.1.1.5	Test requirement

The maximum output power for NR-DC, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2B.1.1.5-1 for NR-DC configuration.

Table 6.2B.1.1.5-1: Maximum Output Power test requirement for NR-DC Power Class 3

| Uplink DC Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_n48A-n70A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| NOTE 1: An uplink DC configuration in which at least one of the bands has NOTE 3 in Table 6.2.1.3-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: PPowerClass is the maximum UE power specified without account of the tolerance NOTE 3: The maximum power requirement applies to the total transmitted power over both the MCG and SCG. NOTE 4: Power class 3 is the default power class unless otherwise stated. NOTE 5: The TT for NR-DC Maximum Output Power is in the maximum TT among all UL CCs. For TT of each UL CC refers to Table 6.2B.1.1.5-2. |  |  |  |  |  |  |  |  |

Table 6.2B.1.1.5-2: Test Tolerance (UE maximum output power)

| TT for overall output power (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |
|  |  |  | BW ≤ 40MHz |  | 40MHz < BW ≤ 100MHz |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| PSCell | BW ≤ 40MHz | f ≤ 3.0GHz | 0.7 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

### 6.2B.2	UE maximum output power reduction for NR-DC

#### 6.2B.2.0	Minimum conformance requirements

##### 6.2B.2.0.1	FFS

##### 6.2B.2.0.2	FFS

##### 6.2B.2.0.3	UE maximum output power reduction for Inter-band NR-DC

For inter-band NR-DC with one uplink assigned per band, the requirements in clause 6.2.2.3 apply for each uplink component carrier.

The normative reference for this requirement is TS38.101-1[2] clause 6.2B.2.

#### 6.2B.2.1	UE maximum output power reduction for NR-DC

6.2B.2.1.1	Test purpose

The number of RB identified in Table 6.2.2.3-1 is based on meeting the requirements for adjacent channel leakage ratio for NR-DC and the maximum power reduction (MPR) for Inter-band NR-DC due to Cubic Metric (CM).

6.2B.2.1.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR-DC.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5B.2.4 is executed.

6.2B.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2B.2.0.

6.2B.2.1.4	Test description

6.2B.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR-DC configurations specified in 5.5B. All of NR-DC configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing as specified in clause 6.2.2.4.1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test configurations tables are specified in clause 6.2.2.4.1

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCell are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to test configuration tables in clause 6.2.2.4.1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR-DC, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2B.2.1.4.3.

6.2B.2.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to clause 6.2.2.4.1 on both PCell and PSCell for inter-band NR-DC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the mean power of each component carrier of NR-DC configuration in the channel bandwidth of the radio access mode. For PC1.5 the measured power is the sum of the two ports. The period of measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

6.2B.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the exceptions in clause 6.2.2.4.3.

6.2B.2.1.5	Test requirement

The maximum output power for NR-DC, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table in clause 6.2.2.5.

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

### 6.2B.3	UE additional maximum output power reduction for NR-DC

#### 6.2B.3.0	Minimum conformance requirements

##### 6.2B.3.0.1	FFS

##### 6.2B.3.0.2	FFS

##### 6.2B.3.0.3	UE additional maximum output power reduction for inter-band NR-DC

For inter-band NR-DC with one uplink assigned per band, the requirements in clause 6.2.2.3 apply for each uplink component carrier.

Unless otherwise stated, the combined requirements and allowed A-MPR are applicable on both bands when both component carriers are active. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet the additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. In case of a power class 3 UE, when IE powerBoostPi2BPSK is set to 1, power class 2 A-MPR values apply.

Unless otherwise specified, pi/2 BPSK refers to both variants of pi/2 BPSK referenced in clause 6.2.2 table 6.2.2.3-1.

#### 6.2B.3.1	UE additional maximum output power reduction for NR-DC

Editor’s note: This clause is complete for A-MPR testing. But the following aspects are either missing for not yet determined:

FFS is left in Test applicability since there are no requirements for 6.5B.2.3 Additional Spectrum Emission mask for NR-DC and 6.5B.3.3 Additional Spurious Emissions for NR-DC in Release-16.

6.2B.3.1.1	Test purpose

Additional emission requirements for NR-DC can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the NR-DC maximum output power. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2B.2. Outer and inner allocation notation used in clause 6.2B.3 is defined in clause 6.2.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

6.2B.3.1.2	Test applicability

The requirements of this test apply in test case 6.5B.2.3 Additional Spectrum Emission mask for NR-DC for network signalling values FFS to all types of NR UE release 16 and forward that support NR-DC.

The requirements of this test apply in test case 6.5B.3.3 Additional Spurious Emissions for NR-DC for network signalling values FFS to all types of NR UE release 16 and forward that support NR-DC.

6.2B.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in 6.2B.3.0.

6.2B.3.1.4	Test description

6.2B.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR-DC configurations specified in 5.5B. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing as specified in clause 6.2.3.4.1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Test configurations tables for network signalling values are specified in clause 6.2.3.4.1

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCell are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to test configuration tables in clause 6.2.3.4.1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR-DC, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2B.3.1.4.3.

6.2B.3.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to clause 6.2.3.4.1 on both PCell and PSCell for inter-band NR-DC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the mean power of each component carrier of NR-DC configuration in the channel bandwidth of the radio access mode. For PC1.5 the measured power is the sum of the two ports. The period of measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

6.2B.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4. Message contents exceptions for network signalling values are specified in clause 6.2.3.4.3.1.

6.2B.3.1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table in clause 6.2.3.5. The allowed A-MPR values specified in table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2.1.3-1 apply.

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

### 6.2B.4	Configured output power for NR-DC

#### 6.2B.4.0	Minimum conformance requirements

##### 6.2B.4.0.1	Configured transmitted power level for NR-DC

6.2B.4.0.1.1	Void

6.2B.4.0.1.2	Void

6.2B.4.0.1.3	Configured transmitted power for Inter-band NR-DC

The UE is allowed to set its configured maximum output power PCMAX,f,c,MCG and PCMAX,f,c,SCG for the respective MCG and SCG and its total configured maximum output power for NR-DC operation  PTotalNR-DC=10log10(PTotalNR-DC) with PTotalNR-DC as specified in clause 7.6.2 of [9]. The UE is configured with an inter-CG power sharing mode by NR-DC-PC-mode. The requirements apply for one uplink serving cell configured per CG and for asynchronous and synchronous NR-DC if not otherwise stated.

Unless otherwise stated, the configured maximum output power PCMAX,f,c,MCG (q) in physical-channel q for carrier f of serving cell c shall be set within the bounds if contained in the MCG,

PCMAX_L,f,c,MCG (q) ≤  PCMAX,f,c,MCG (q) ≤  PCMAX_H,f,c,MCG (q)

and the corresponding PCMAX_L,f,c,SCG (q) for a serving cell contained in the SCG,

PCMAX_L,f,c,SCG (q) ≤  PCMAX,f,c,SCG (q) ≤  PCMAX_H,f,c,SCG (q)

where PCMAX_L,f,c,MCG, PCMAX_H,f,c,MCG, PCMAX_L,f,c,SCG and PCMAX_H,f,c,SCG are the limits for a serving cell c as specified in clause 6.2.4 modified as follows:

PCMAX_L,f,c,MCG = MIN{MIN(PEMAX,c , PEMAX,NR-DC, PNR) – ∆TC,c, (PPowerClass, NR-DC – ΔPPowerClass, NR-DC) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc)}

PCMAX_H,f,c,MCG = MIN{PEMAX,c, PEMAX,NR-DC, PNR, PPowerClass, NR-DC – ΔPPowerClass, NR-DC}

for the MCG and

PCMAX_L,f,c,SCG = MIN{MIN(PEMAX,c , PEMAX,NR-DC, PNR) – ∆TC,c, (PPowerClass, NR-DC – ΔPPowerClass, NR-DC) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc)}

PCMAX_H,f,c,SCG = MIN{PEMAX,c, PEMAX,NR-DC, PNR, PPowerClass, NR-DC – ΔPPowerClass, NR-DC}

for the SCG, where

-	PEMAX,NR-DC is the value given by the field p-UE-FR1 of the PhysicalCellGroupConfig IE for the MCG as defined in [6];

-	PNR is the value given by the field p-NR-FR1 of the PhysicalCellGroupConfig IE as defined in [6];

-	PPowerClass, NR-DC is the maximum UE power specified in Table 6.2B.1.0.3-1 without taking into account the tolerance specified in the Table 6.2B.1.0.3-1;

-	∆TIB,c is the additional tolerance for serving cell c as specified in clause 6.2B.4.0.2 for NR-DC; ∆TIB,c = 0 dB otherwise;

-	∆TC,c = 1.5dB when NOTE 2 in Table 6.2B.1.0.3-1 applies for a serving cell c, otherwise ∆TC,c = 0 dB;

-	∆MPRc for serving cell c is specified in clause 6.2.2.

-	ΔPPowerClass, NR-DC = 0 dB for a power class 3 UE.

For a UE provided with NR-DC-PC-mode = Semi-static-mode1,

PTotalNR-DC = MIN{PEMAX, NR-DC, PPowerClass, NR-DC} + 0.3 dB

with PPowerClass, NR-DC set to power class 3 in case the UE indicates a higher power class in any CG. The UE determines the maximum transmission power for the MCG and the SCG using the respective configured maximum power PCMAX,f,c,MCG and PCMAX,f,c,SCG.

If for synchronous NR-DC operation a UE is provided NR-DC-PC-mode = Semi-static-mode2, the PTotalNR-DC is determined as above and

-	if at least one symbol of slot i1 of the MCG/SCG is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided, overlaps with a symbol for any ongoing transmission overlapping with slot i2 of the SCG/MCG, the UE determines a maximum power for the transmission on the SCG/MCG overlapping with slot i2 using the configured maximum power PCMAX,f,c,SCG or PCMAX,f,c,MCG for the SCG or MSG, respectively,

-	otherwise (i.e. an ongoing transmission overlapping with slot i2 of the SCG/MCG overlaps with only semi-static downlink symbols within slot i1 of the MCG/SCG), the UE determines a maximum power for the transmission on MCG or the SCG overlapping with slot i2 using the configured maximum power as specified in clause 6.2.4.

If a UE indicates a capability for dynamic power sharing between the MCG and the SCG and is provided with NR-DC-PC-mode = Dynamic,

PTotalNR-DC = MIN{PEMAX, NR-DC, PPowerClass, NR-DC}

with PPowerClass, NR-DC set to power class 3 in case the UE indicates a higher power class in any CG. The UE determines the maximum transmission power for the MCG and the SCG using the respective configured maximum power PCMAX,f,c,MCG and PCMAX,f,c,SCG except

-	if UE transmission(s) in slot i1 of the MCG or in slot i2 of the SCG do not overlap in time with any UE transmission(s) on the SCG or the MCG, respectively, the UE determines a maximum transmission power in slot i1 of the MCG or in slot i2 of the SCG using the configured maximum power as specified in clause 6.2.4.

If a UE indicates a capability to determine a total transmission power on the SCG at a first symbol of a transmission occasion on the SCG by determining transmissions on the MCG as specified in clause 7.6.2 of [9], and is provided with NR-DC-PC-mode = Dynamic,

PTotalNR-DC = MIN{PEMAX, NR-DC, PPowerClass, NR-DC}

with PPowerClass, NR-DC set to power class 3 in case the UE indicates a higher power class in any CG. The UE determines the maximum transmission power for the MCG and the SCG using the respective configured maximum power PCMAX,f,c,MCG and PCMAX,f,c,SCG.

The measured total maximum output power PUMAX over both CGs measured over the transmission reference time duration is

PUMAX = 10 log10 (pUMAX,c,MCG + pUMAX,c,SCG),

where pUMAX,c,MSG and pUMAX,c,SCG denote the measured output power of serving cells c contained in the respective MSG and SCG expressed in linear scale.

The measured total configured maximum output power PUMAX shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

with the tolerances TLOW(PCMAX_H) and THIGH(PCMAX_H) for applicable values of PCMAX specified in Table 6.2B.4.0.1.3-2.

When a subframe p on the MSG overlap with a physical-channel q on the SCG, then for PUMAX evaluation, the subframe p on the MCG is taken as reference period TREF and always considered as the reference measurement duration and the following rules are applicable.

TREF and Teval are specified in Table 6.2B.4.0.1.3-1 when same or different subframe and physical-channel durations are used on the carriers. The PPowerClass shall not be exceeded by the UE during any evaluation period of time.

Table 6.2B.4.0.1.3-1: PCMAX evaluation window

| Transmission duration | TREF | Teval |
| --- | --- | --- |
| Different transmission duration in different CG carriers | MCG subframe | MIN(Tno_hopping, Physical Channel Length) |

For each TREF, the PCMAX_H is evaluated per Teval and given by the maximum value over the transmission(s) within the Teval as follows:

PCMAX_H  = MAX{PCMAX_NR-DC_H(p,q), PCMAX_NR-DC_H(p,q+1), … , PCMAX_NR-DC_H(p,q+n)}

where PCMAX_NR-DC_H entries are the applicable upper limits for each overlapping scheduling unit pairs (p,q), (p, q+1), up to (p, q+n) for each applicable Teval duration, where q+n is the last physical-channel on the SCG overlapping with subframe p on the MCG, while PCMAX_L is computed as follows:

PCMAX_L = MIN{PCMAX_NR-DC_L(p,q), PCMAX_NR-DC_L(p,q+1), … , PCMAX_NR-DC_L(p,q+n)}

where PCMAX_NR-DC_L entries are the applicable lower limits for each overlapping scheduling unit pairs (p,q), (p, q+1) up to (p, q+n) for each applicable Teval duration, where q+n is the last physical-channel on the SCG overlapping with subframe p on the MCG.

For a UE provided with NR-DC-PC-mode = Semi-static-mode1 and configured with pNR,MCG + pNR,SCG ≤ PTotalNR-DC with pNR,MCG and pNR,SCG the values of the PNR for the respective MCG and SCG expressed in linear scale

PCMAX_NR-DC_L(p,q) = 10 log10 [pCMAX_L,f,c,MCG (p) + pCMAX_L,f,c,SCG (q)]

PCMAX_NR-DC_H(p,q) = 10 log10 [pCMAX_H,f,c,MCG (p) + pCMAX_H,f,c,SCG (q)]

with pCMAX_L,f,c,MCG, pCMAX_H,f,c,MCG, pCMAX_L,f,c,SCG, and pCMAX_H,f,c,SCG the values of the respective PCMAX_L,f,c,MCG, PCMAX_H,f,c,MCG, PCMAX_L,f,c,MCG, and PCMAX_H,f,c,SCG expressed in linear scale, while the measured configured maximum power PUMAX  for each CG shall meet the requirements as specified in clause 6.2.4 but with bounds for PCMAX,f,c,MCG (p) and PCMAX,f,c,SCG  as specified in this clause.

If for synchronized NR-DC a UE is provided with NR-DC-PC-mode = Semi-static-mode2 and configured with pNR,MCG + pNR,SCG ≤ PTotalNR-DC with pNR,MCG and pNR,SCG the linear-scale values of the PNR for the respective MCG and SCG

PCMAX_NR-DC_L(p,q) = 10 log10 [pCMAX_L,f,c,MCG (p) + pCMAX_L,f,c,SCG (q)]

PCMAX_NR-DC_H(p,q) = 10 log10 [pCMAX_H,f,c,MCG (p) + pCMAX_H,f,c,SCG (q)]

while the measured configured maximum power PUMAX for each CG shall meet the requirements specified in Table 6.2.4.3-2 but with bounds for PCMAX,f,c,MCG (p) and PCMAX,f,c,SCG  as specified in this clause except

-	if an ongoing transmission overlapping with physical channel q of the SCG or subframe p of the MCG overlaps with only semi-static downlink symbols within the respective subframe p of the MCG or physical channel q of the SCG as indicated to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided,

then the measured configured maximum power PUMAX for the transmission subframe p on the MCG or physical channel q on the SCG shall meet the requirements as specified in clause 6.2.4 and with bounds for PCMAX,f,c,MCG (p) or PCMAX,f,c,SCG  as specified in clause 6.2.4.

For a UE provided with NR-DC-PC-mode = Dynamic,

PCMAX_NR-DC_L(p,q) = MIN{10 log10 [pCMAX_L,f,c,MCG (p) + pCMAX_L,f,c,SCG (q)], PTotalNR-DC}

PCMAX_NR-DC_H(p,q) = MIN{10 log10 [pCMAX_H,f,c,MCG (p) + pCMAX_H,f,c,SCG (q)], PTotalNR-DC}

while the measured configured maximum power PUMAX on the MCG shall meet the requirements as specified in clause 6.2.4.3-2 but with bounds for PCMAX,f,c,MCG (p) as specified in this clause, and the PUMAX on the SCG shall be within

PCMAX_L,f,c  –  MAX{TL,c, T(PCMAX_L,f,c)}  ≤  PUMAX,f,c  ≤  PCMAX_H,f,c  +  T(PCMAX_H,f,c).

where

PCMAX_L,f,c = MIN{PCMAX_L,f,c,SCG (p), 10 log10 (PTotalNR-DC – pNR,MSG)}

PCMAX_H,f,c = MIN{PCMAX_H,f,c,SCG (p), 10 log10 (PTotalNR-DC – pNR,MSG)}

with limits as specified in Table 6.2.4.3-2 and pNR,MCG the value of the PNR for the MCG expressed in linear scale.

Table 6.2B.4.0.1.3-2: PCMAX tolerance for NR-DC

| PCMAX(dBm) | Tolerance TLOW (PCMAX_L) (dB) | Tolerance THIGH (PCMAX_H) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 33 | 3.0 | 2.0 |
| 22 ≤ PCMAX < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX< 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX < 21 | 5.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAX < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |
| NOTE 1: For UEs provided with NR-DC-PC-mode = Semi-static-mode1 or with NR-DC-PC-mode = Semi-static-mode2, the upper tolerance Thigh shall be reduced by 0.3 dB for P ≥ 20 dBm. |  |  |

##### 6.2B.4.0.2	ΔTIB,c for NR-DC

For the UE which supports inter-band NR-DC with one uplink carrier assigned per NR band, the ΔTIB,c for the corresponding inter-band CA configurations as specified in clause 6.2A.4.0.2 applies.

###### 6.2B.4.0.2.1	FFS

###### 6.2B.4.0.2.2	FFS

###### 6.2B.4.0.2.3	Void

Table 6.2B.4.0.2.3-1: Void

#### 6.2B.4.1	Configured transmitted power level for NR-DC

6.2B.4.1.1	Test purpose

To verify that the total measured UE configured maximum output power PUMAX in dual connectivity is within the specified bounds.

6.2B.4.1.2	Test applicability

The requirements of this test apply to all types of NR UE release 16 and forward that support NR-DC.

6.2B.4.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.2B.4.0.

6.2B.4.1.4	Test description

6.2B.4.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR-DC configurations specified in 5.5B. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2B.4.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2B.4.1.4.1-1: Inter-band NR-DC Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range for PCell and PSCell |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest for PCell and PSCell Highest for PCell and PSCell |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Downlink Configuration for PCell & PSCell | Uplink Configuration |  |  |
|  |  | Modulation for all CCs (NOTE 2) | RB allocation (NOTE 1) |  |
|  |  |  | PCell | PSCell |
| 1 | N/A for minimum output power test case | DFT-s-OFDM Pi/2 BPSK | Inner Full | Inner Full |
| 2 |  | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2B.4.1.4.1-1 as appropriate.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR-DC, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2B.4.1.4.3.

6.2B.4.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2B.4.1.4.1-1 on both PCell and PSCell for inter-band NR-DC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level of the test point.

3.	Measure the sum of mean transmitted power over all component carriers in the NR-DC configuration of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD, only slots consisting of only UL symbols are under test.

6.2B.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with the following exceptions.

Table 6.2B.4.1.4.3-0: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.2B.4.1.4.3-1: FrequencyInfoUL: Test point 1

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-61 FrequencyInfoUL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | -10 |  |  |

Table 6.2B.4.1.4.3-1a: FrequencyInfoUL-SIB: Test point 1

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | -10 |  |  |

Table 6.2B.4.1.4.3-2: FrequencyInfoUL: Test point 2

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-61 FrequencyInfoUL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 10 |  |  |

Table 6.2B.4.1.4.3-2a: FrequencyInfoUL-SIB: Test point 2

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 10 |  |  |

6.2B.4.1.5	Test requirement

The maximum output power measured shall not exceed the values specified in Table 6.2B.4.1.5-1.

Table 6.2B.4.1.5-1: Configured UE Output Power for Inter-band NR-DC

|  | Maximum output power |  |
| --- | --- | --- |
|  | Lower limit | Upper limit |
| Measured UE output power test point 1 | -14 dBm - TT | 0 dBm + TT |
| Measured UE output power test point 2 | 7 dBm - TT | 19 dBm + TT |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 6.2B.4.1.5-2. |  |  |

Table 6.2B.4.1.5-2: Test Tolerance for Configured transmitted power for Inter-band NR-DC

| TT for overall output power (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |  |  |
|  |  |  | BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| PSCell | BW ≤ 40MHz | f ≤ 3.0GHz | 0.71 1.02 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
| Note 1: Applicable for test point with UL power ≥ 0 dBm. Note 2: Applicable for test point with UL power < 0 dBm. |  |  |  |  |  |  |  |  |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

## 6.2C	Transmitter power for SUL

### 6.2C.1	Configured transmitted power for SUL

6.2C.1.1	Test purpose

Same test purpose as in clause 6.2.4.1

6.2C.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.2C.1.3	Minimum conformance requirements

When a UE is configured with both NR UL and NR SUL carriers in a serving cell with active transmission either on the UL carrier or SUL carrier, the configured transmit power requirements specified in subclause 6.2.4 are applicable for the UL carrier and the SUL carrier, respectively.

For the UE which supports SUL band combination, ΔTIB,c in Table 6.2C.2-1 and Table 6.2C.2-2 applies.

The normative reference for this requirement is TS 38.101-1 [2] clauses 6.2C.

6.2C.1.4	Test description

Same test description as specified in clause 6.2.4.4 with following exceptions:

Instead of table 6.2.4.4.1-1  use Table 6.2C.1.4-1

Table 6.2C.1.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range for both SUL carrier and Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 |  |  |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 |  |  |  | DFT-s-OFDM QPSK | Inner Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 4: For NR band n83, 30MHz test channel bandwidth is tested with Low range test frequency. |  |  |  |  |  |

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.4 for TE diagram and section A.3.2 for UE diagram.

-	The parameter setting for the cell are set up according to the TS 38.508-1 [5] subclause 4.4.3.

-	Downlink signals are initially setup according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 with consideration of supplementary uplink physical channels.

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring that for SUL carrier, Tables 4.6.1-28 with condition SUL AND (RF OR RRM), 4.6.3-14 with condition SUL_SUL for SUL carrier and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL, additionally the following exceptions shown in Table 6.2C.1.4-2 ~ Table 6.2C.1.4-4 are considered.

Table 6.2C.1.4-2: SIB1: Test point 1

| Derivation Path: TS 38.508-1 [5] Table 4.6.1-28 SIB1 with condition SUL AND RF |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| supplementaryUplink ::= SEQUENCE { |  |  |  |
| frequencyInfoUL SEQUENCE { |  |  |  |
| p-Max | -10 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2C.1.4-3: SIB1: Test point 2

| Derivation Path: TS 38.508-1 [5] Table 4.6.1-28 SIB1 with condition SUL AND RF |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| supplementaryUplink ::= SEQUENCE { |  |  |  |
| frequencyInfoUL SEQUENCE { |  |  |  |
| p-Max | 10 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2C.1.4-4: SIB1: Test point 3

| Derivation Path: TS 38.508-1 [5] Table 4.6.1-28 SIB1 with condition SUL AND RF |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| supplementaryUplink ::= SEQUENCE { |  |  |  |
| frequencyInfoUL SEQUENCE { |  |  |  |
| p-Max | 15 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2C.1.4-5: Void

6.2C.1.5	Test requirement

The maximum output power measured on SUL carrier shall not exceed the values specified in Table 6.2C.1.5-1.

Table 6.2C.1.5-1: PCMAX configured UE output power

|  | Channel bandwidth / maximum output power |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz |
| Measured UE output power test point 1 | -10 dBm ± (7+TT) |  |  |  |  |  |
| Measured UE output power test point 2 | 10 dBm ± (6+TT) |  |  |  |  |  |
| Measured UE output power test point 3 | 15 dBm ± (5+TT) |  |  |  |  |  |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 6.2.4.5-2. Note 2: In addition note 3 in Table 6.2.1.3-1 shall apply to the tolerances. |  |  |  |  |  |  |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

### 6.2C.2	ΔTIB,c

For the UE which supports SUL band combination, ΔTIB,c in Tables below applies. Unless otherwise stated, ΔTIB,c is set to zero.

Table 6.2C.2-1: ΔTIB,c due to SUL

| Band combination for SUL | ΔTIB,c for NR bands (dB)3 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration4 |  |
| SUL_n41-n83 | 0.3 | 0.3 |
| SUL_n78-n80 | 0.8 | 0.6 |
| SUL_n78-n81 | 0.8 | 0.6 |
| SUL_n78-n82 | 0.8 | 0.6 |
| SUL_n78-n83 | 0.8 | 0.5 |
| SUL_n78-n84 | 0.8 | 0.3 |
| SUL_n78-n86 | 0.8 | 0.6 |
| SUL_n79-n83 | 0.8 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2515MHz. NOTE 3: “-” denotes ΔTIB,c = 0. NOTE 4: The component band order in the configuration should be listed by the order of NR bands such as for SUL_n41-n81 the order of band is n41 and n81. |  |  |

Table 6.2C.2-2: ΔTIB,c for SUL band combination (Three bands)

| Band combination for SUL | ΔTIB,c for NR bands (dB)3 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration4 |  |  |
| CA_n1_n78-n80 | 0.6 | 0.8 | 0.6 |
| CA_n1_n78-n84 | 0.6 | 0.8 | 0.6 |
| CA_n3_n78-n80 | 0.6 | 0.8 | 0.6 |
| CA_n28_n41-n83 | 0.3 | 0.3 | 0.3 |
| CA_n28_n79-n83 | 0.5 | 0.8 | 0.5 |
| CA_n41_n79-n83 | 0.3 | 0.8 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2515MHz. NOTE 3: “-” denotes ΔTIB,c = 0. NOTE 4: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

### 6.2C.3	UE maximum output power for SUL

6.2C.3.1	Test purpose

Same test purpose as in clause 6.2.1.1

6.2C.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.2C.3.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3 and 6.2.1.

6.2C.3.4	Test description

Same test description as specified in clause 6.2.1.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.2.1.4.1-1  use Table 6.2C.3.4-1

Table 6.2C.3.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range for Non-SUL carrier. Low, Mid, High range for SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Left |
| 3 |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Right |
| 4 |  |  |  | DFT-s-OFDM QPSK | Inner Full |
| 5 |  |  |  | DFT-s-OFDM QPSK | Inner 1RB Left |
| 6 |  |  |  | DFT-s-OFDM QPSK | Inner 1RB Right |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 4: For NR band n83, 30MHz test channel bandwidth is tested with Low range test frequency. |  |  |  |  |  |

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.4 for TE diagram and section A.3.2 for UE diagram.

-	The parameter setting for the cell are set up according to the TS 38.508-1 [5] subclause 4.4.3.

-	Downlink signals are initially setup according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 with consideration of supplementary uplink physical channels.

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Tables 4.6.3-14 with condition SUL_SUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL, additionally the following exceptions shown in Table 6.2C.3.4-2is considered.

Table 6.2C.3.4-2: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.2C.3.4-3: Void

6.2C.3.5	Test requirement

The maximum output power measured on SUL carrier shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2C.3.5-1.

Table 6.2C.3.5-1: Maximum Output Power test requirement for Power Class 3

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n80 |  |  |  |  | 23 | ±(2+TT) |
| n81 |  |  |  |  | 23 | ±(2+TT) |
| n82 |  |  |  |  | 23 | ±(2+TT) |
| n83 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n84 |  |  |  |  | 23 | ±(2+TT) |
| n86 |  |  |  |  | 23 | ±(2+TT) |
| n97 |  |  |  |  | 23 | ±(2+TT) |
| n99 |  |  |  |  | 23 | +2+TT/-3.03-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3 |  |  |  |  |  |  |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

### 6.2C.3_1	Void

### 6.2C.4	UE maximum output power reduction for SUL

6.2C.4.1	Test purpose

Same test purpose as in clause 6.2.2.1

6.2C.4.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5C.2.4.1 is executed.

6.2C.4.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3 and 6.2.2.

6.2C.4.4	Test description

Same test description as PC 3 with contiguous allocation specified in clause 6.2.2.4 with following exceptions:

Instead of table 6.2.2.4.1-1  use Table 6.2C.4.4-1.

Table 6.2C.4.4-1: Test Configuration Table for power class 3 contiguous allocation

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range and high range for SUL carrier Mid range for Non-SUL carrier. |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  |  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 | Default |  |  |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  |  |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  |  |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  |  |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  |  |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  |  |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  |  |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  |  |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  |  |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  |  |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  |  |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  |  |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  |  |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  |  |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  |  |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  |  |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  |  |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  |  |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  |  |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  |  |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  |  |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  |  |  | OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  |  |  | OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  |  |  | OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  |  |  | OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  |  |  | CP-OFDM 256 QAM | Outer Full |
| 334 | Low |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 344 | High |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 354 | Default |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 4: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. |  |  |  |  |  |  |

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.4 for TE diagram and section A.3.2 for UE diagram.

-	The parameter setting for the cell are set up according to the TS 38.508-1 [5] subclause 4.4.3.

-	Downlink signals are initially setup according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 with consideration of supplementary uplink physical channels.

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Table 4.6.3-14 with condition SUL_SUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL. Message contents in Table 6.2.2.4.3-3 in clause 6.2.2.4.3 only apply to Test ID 33-35 in Table 6.2C.4.4-1.

Table 6.2C.4.4-2: Void

6.2C.4.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2C.4.5-1 and Table 6.2C.4.5-2.

Table 6.2C.4.5-1: UE Power Class test requirements (for Bands n80, n81, n82, n84, n86, n97) for Power Class 3

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2 | 25.0 + TT | 21.0 - TT |
| 2 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2 | 25.0 + TT | 20.5 - TT |
| 3 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2 | 25.0 + TT | 20.5 - TT |
| 4 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2 | 25.0 + TT | 20.5 - TT |
| 5 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2 | 25.0 + TT | 21.0 - TT |
| 6 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2 | 25.0 + TT | 20.0 - TT |
| 7 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2 | 25.0 + TT | 20.0 - TT |
| 8 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2 | 25.0 + TT | 20.0 - TT |
| 9 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2 | 25.0 + TT | 20.0 - TT |
| 10 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 11 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 12 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 13 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18.0 - TT |
| 14 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18.0 - TT |
| 15 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25.0 + TT | 18.0 - TT |
| 16 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25.0 + TT | 14.5 - TT |
| 17 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25.0 + TT | 14.5 - TT |
| 18 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25.0 + TT | 14.5 - TT |
| 19 | 23 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25.0 + TT | 19.5 - TT |
| 20 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 21 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 22 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 23 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2 | 25.0 + TT | 19.0 - TT |
| 24 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 25 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2 | 25.0 + TT | 17.5 - TT |
| 27 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25.0 + TT | 16.0 - TT |
| 28 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25.0 + TT | 16.0 - TT |
| 29 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25.0 + TT | 16.0 - TT |
| 30 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25.0 + TT | 11.5 - TT |
| 31 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25.0 + TT | 11.5 - TT |
| 32 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25.0 + TT | 11.5 - TT |
| 33 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2 | 25.0 + TT | 20.5- TT |
| 34 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2 | 25.0 + TT | 20.5- TT |
| 35 | 23 | 0 | 0 | 0 | 23 | 2.0 | 2 | 25.0 + TT | 21- TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2C.4.5-3. |  |  |  |  |  |  |  |  |  |

Table 6.2C.4.5-2: UE Power Class test requirements (for Bands n83 with channel bandwidth other than 30MHz) for Power Class 3

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2.5 | 25.0 + TT | 20.5 - TT |
| 2 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 3 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 4 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 5 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2.5 | 25.0 + TT | 20.5 - TT |
| 6 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 7 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 8 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 9 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 10 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 11 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 12 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 13 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 14 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 15 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 16 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 17 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 18 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 19 | 23 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2.5 | 25.0 + TT | 19.0 - TT |
| 20 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 21 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 22 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 23 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 24 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 25 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 26 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 27 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 28 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 29 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 30 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 31 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 32 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 33 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 34 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 35 | 23 | 0 | 0 | 0 | 23 | 2.0 | 2.5 | 25.0 + TT | 20.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2C.4.5-3: UE Power Class test requirements (for Bands n83 with 30MHz channel bandwidth) for Power Class 3

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 0 | 22.5 | 2 | 2.5 | 25.0 + TT | 20-TT |
| 2 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 3 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 4 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 5 | 23 | 0 | 0.5 | 0 | 22.5 | 2 | 2.5 | 25.0 + TT | 20-TT |
| 6 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 7 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 8 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 9 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 10 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 11 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 12 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 13 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 14 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 15 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 16 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 17 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 18 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 19 | 23 | 0 | 2 | 0 | 21 | 2 | 2.5 | 25.0 + TT | 18.5-TT |
| 20 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 21 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 22 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 23 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 24 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 25 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 26 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 27 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 28 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 29 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 30 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 31 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 32 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 33 | 23 | 0 | 1 | 0 | 22 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 34 | 23 | 0 | 1 | 0 | 22 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 35 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2C.4.5-4: UE Power Class test requirements for Band n99 for Power Class 3

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | 25.0 + TT | 20.0 - TT | （17.0 - TT2） |
| 2 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （16.5 - TT2） |
| 3 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （16.5 - TT2） |
| 4 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 25.0 + TT | 20.5 - TT | （16.5 - TT2） |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | 25.0 + TT | 20.0 - TT | （17.0 - TT2） |
| 6 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 7 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 8 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 9 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 10 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （15.0 - TT2） |
| 11 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （15.0 - TT2） |
| 12 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （15.0 - TT2） |
| 13 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | 25.0 + TT | 17.5 – TT | （14.5 - TT2） |
| 14 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 15 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 16 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 17 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 18 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 19 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | 25.0 + TT | 18.5 - TT | （15.5 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.0 - TT2） |
| 21 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.0 - TT2） |
| 22 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.0 - TT2） |
| 23 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （15.0 - TT2） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 – TT | （14.0 - TT2） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 – TT | （14.0 - TT2） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.0 - TT2） |
| 27 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 25.0 + TT | 16.0 - TT | （13.5 - TT2） |
| 28 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （13.5 - TT2） |
| 29 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （13.5 - TT2） |
| 30 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 31 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 32 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 33 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 2 | 25.0 + TT | 19.5 - TT | （16.5 - TT2） |
| 34 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 2 | 25.0 + TT | 19.5 - TT | （16.5 - TT2） |
| 35 | 23 | 0 | 0 | 0 | （1.52） | 23 | （21.52） | 2.0 |  | 2 | 25.0 + TT | 20.0 - TT | （17.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

### 6.2C.5	UE additional maximum output power reduction for SUL

Editor’s note: Tests for network signalling values NS_17 not complete.

6.2C.5.1	Test purpose

Same test purpose as in clause 6.2.3.1

6.2C.5.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.2C.5.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1 to 5.5C-4, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3and 6.2.3.

6.2C.5.4	Test description

Same test description as specified in clause 6.2.3.4 with following exceptions:

For NS_03 and NS_03U: Instead of Table 6.2.3.4.1-3  use Table 6.2C.5.4-1

For NS_18: Instead of Table 6.2.3.4.1-11  use Table 6.2C.5.4-2

For NS_48: Instead of Table 6.2.3.4.1-19  use Table 6.2C.5.4-3

For NS_49: Instead of Table 6.2.3.4.1-29  use Table 6.2C.5.4-4

For NS_05: Instead of Table 6.2.3.4.1-4  use Table 6.2C.5.4-5

For NS_05U: Instead of Table 6.2.3.4.1-5  use Table 6.2C.5.4-6

For NS_100: Instead of Table 6.2.3.4.1-3  use Table 6.2C.5.4-7

For NS_56: Instead of Table 6.2.3.4.1-30  use Table 6.2C.5.4-8

For NS_43: Instead of Table 6.2.3.4.1-6  use Table 6.2C.5.4-9

For NS_43U: Instead of Table 6.2.3.4.1-7  use Table 6.2C.5.4-10

Table 6.2C.5.4-1: Test Configuration Table for NS_03 and NS_03U

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Low range, High range for SUL carrier Mid range for Non-SUL carrier |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest for SUL carrier Lowest for Non-SUL carrier |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |
| A-MPR test parameters for NS_03 and NS_03U |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | UL Configuration | SUL Configuration |  |  |
|  |  |  |  | Modulation |  | RB allocation (Note 2) |
| 1 | Low | N/A | N/A | DFT-s OFDM | PI/2 BPSK | Edge_1RB_Left |
| 2 | High |  |  |  | PI/2 BPSK | Edge_1RB_Right |
| 3 | Default |  |  |  | PI/2 BPSK | Outer_Full |
| 4 | Low |  |  |  | QPSK | Edge_1RB_Left |
| 5 | High |  |  |  | QPSK | Edge_1RB_Right |
| 6 | Default |  |  |  | QPSK | Outer_Full |
| 7 | Low |  |  |  | 16 QAM | Edge_1RB_Left |
| 8 | High |  |  |  | 16 QAM | Edge_1RB_Right |
| 9 | Default |  |  |  | 16 QAM | Outer_Full |
| 10 | Low |  |  |  | 64 QAM | Edge_1RB_Left |
| 11 | High |  |  |  | 64 QAM | Edge_1RB_Right |
| 12 | Default |  |  |  | 64 QAM | Outer_Full |
| 13 | Low |  |  |  | 256 QAM | Edge_1RB_Left |
| 14 | High |  |  |  | 256 QAM | Edge_1RB_Right |
| 15 | Default |  |  |  | 256 QAM | Outer_Full |
| 16 | Low |  |  | CP-s OFDM | QPSK | Edge_1RB_Left |
| 17 | High |  |  |  | QPSK | Edge_1RB_Right |
| 18 | Default |  |  |  | QPSK | Outer_Full |
| 19 | Low |  |  |  | 16 QAM | Edge_1RB_Left |
| 20 | High |  |  |  | 16 QAM | Edge_1RB_Right |
| 21 | Default |  |  |  | 16 QAM | Outer_Full |
| 22 | Low |  |  |  | 64 QAM | Edge_1RB_Left |
| 23 | High |  |  |  | 64 QAM | Edge_1RB_Right |

| A-MPR test parameters for NS_03 and NS_03U |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Freq | Downlink Configuration | UL Configuration | SUL Configuration |  |  |
|  |  |  |  | Modulation |  | RB allocation (Note 2) |
| 24 | Default |  |  |  | 64 QAM | Outer_Full |
| 25 | Low |  |  |  | 256 QAM | Edge_1RB_Left |
| 26 | High |  |  |  | 256 QAM | Edge_1RB_Right |
| 27 | Default |  |  |  | 256 QAM | Outer_Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |

Table 6.2C.5.4-2: Test Configuration Table for NS_18

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | 5MHz 10MHz 20MHz 30MHz for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| A-MPR test parameters for NS_18 |  |  |  |  |  |  |
| Test ID | ChBw SCS | Downlink Configuration | SUL Configuration |  |  |  |
|  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) | Comment |
| 1 | 5MHz, 10MHz, 20MHz | N/A | DFT-s OFDM | QPSK | Edge_1RB_Left | A1, A2 |
| 2 |  |  |  | QPSK | Outer_Full | A1, A2 |
| 3 |  |  |  | 16 QAM | Edge_1RB_Left | A1, A2 |
| 4 |  |  |  | 16 QAM | Outer_Full | A1, A2 |
| 5 |  |  |  | 64 QAM | Edge_1RB_Left | A1, A2 |
| 6 |  |  |  | 64 QAM | Outer_Full | A1, A2 |
| 7 |  |  |  | 256 QAM | Edge_1RB_Left | A1, A2 |
| 8 |  |  |  | 256 QAM | Outer_Full | A1, A2 |
| 9 |  |  | CP-s OFDM | QPSK | Edge_1RB_Left | A1, A2 |
| 10 |  |  |  | QPSK | Outer_Full | A1, A2 |
| 11 |  |  |  | 16 QAM | Edge_1RB_Left | A1, A2 |
| 12 |  |  |  | 16 QAM | Outer_Full | A1, A2 |
| 13 |  |  |  | 64 QAM | Edge_1RB_Left | A1, A2 |
| 14 |  |  |  | 64 QAM | Outer_Full | A1, A2 |
| 15 |  |  |  | 256 QAM | Edge_1RB_Left | A1, A2 |
| 16 |  |  |  | 256 QAM | Outer_Full | A1, A2 |
| 17 | 30MHz |  | DFT-s OFDM | 256 QAM | 81@70 | A3 |
| 18 |  |  |  | 256 QAM | Outer_Full | A4 |
| 19 |  |  | CP-s OFDM | 256 QAM | 87@73 | A3 |
| 20 |  |  |  | 256 QAM | Outer_Full | A4 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |

Table 6.2C.5.4-3: Test Configuration Table for NS_48

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Low range, High range for SUL carrier Mid range for non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | 25 MHz, 30MHz, 40MHz, 50MHz for SUL carrier Lowest for non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| A-MPR test parameters for NS_48 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | Uplink Configuration | Downlink Configuration | SUL Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | Default | 25 | N/A for A-MPR testing. | N/A | DFT-s-OFDM | QPSK | Outer_Full (A3) |
| 2 | Default | 25 |  |  |  | QPSK | Edge_1RB_Right (A3) |
| 3 | Default | 30 |  |  |  | QPSK | Outer_Full (A3) |
| 4 | Default | 30 |  |  |  | QPSK | Edge_1RB_Right (A5) |
| 5 | Default | 40 |  |  |  | QPSK | 16@0 (A2) |
| 6 | Default | 40 |  |  |  | QPSK | 90@0 (A3) |
| 7 | Default | 40 |  |  |  | QPSK | 150@0 (A4) |
| 8 | Default | 40 |  |  |  | QPSK | 192@0 (A2) |
| 9 | Default | 40 |  |  |  | QPSK | 5@187 (A3) |
| 10 | Default | 40 |  |  |  | QPSK | Outer_Full (A1) |
| 11 | Default | 50 |  |  |  | QPSK | 32@0 (A2) |
| 12 | Default | 50 |  |  |  | QPSK | 108@0 (A4) |
| 13 | Default | 50 |  |  |  | QPSK | 225@0 (A2) |
| 14 | Default | 50 |  |  |  | QPSK | 5@223 (A5) |
| 15 | Default | 50 |  |  |  | QPSK | Outer_Full (A1) |
| 16 | Default | 25 |  |  |  | 256 QAM | Outer_Full (A3) |
| 17 | Default | 25 |  |  |  | 256 QAM | Edge_1RB_Right (A3) |
| 18 | Default | 30 |  |  |  | 256 QAM | Outer_Full (A3) |
| 19 | Default | 30 |  |  |  | 256 QAM | Edge_1RB_Right (A5) |
| 20 | Default | 40 |  |  |  | 256 QAM | 16@0 (A2) |
| 21 | Default | 40 |  |  |  | 256 QAM | 90@0 (A3) |
| 22 | Default | 40 |  |  |  | 256 QAM | 150@0 (A4) |
| 23 | Default | 40 |  |  |  | 256 QAM | 192@0 (A2) |
| 24 | Default | 40 |  |  |  | 256 QAM | 5@187 (A3) |
| 25 | Default | 40 |  |  |  | 256 QAM | Outer_Full (A1) |
| 26 | Default | 50 |  |  |  | 256 QAM | 32@0 (A2) |
| 27 | Default | 50 |  |  |  | 256 QAM | 108@0 (A4) |
| 28 | Default | 50 |  |  |  | 256 QAM | 225@0 (A2) |
| 29 | Default | 50 |  |  |  | 256 QAM | 5@223 (A5) |
| 30 | Default | 50 |  |  |  | 256 QAM | Outer_Full (A1) |
| 31 | Default | 25 |  |  | CP-OFDM | QPSK | Outer_Full (A3) |
| 32 | Default | 25 |  |  |  | QPSK | Edge_1RB_Right (A3) |
| 33 | Default | 30 |  |  |  | QPSK | Outer_Full (A3) |
| 34 | Default | 30 |  |  |  | QPSK | Edge_1RB_Right (A5) |
| 35 | Default | 40 |  |  |  | QPSK | 16@0 (A2) |
| 36 | Default | 40 |  |  |  | QPSK | 95@0 (A3) |
| 37 | Default | 40 |  |  |  | QPSK | 152@0 (A4) |
| 38 | Default | 40 |  |  |  | QPSK | 192@0 (A2) |
| 39 | Default | 40 |  |  |  | QPSK | 5@187 (A3) |
| 40 | Default | 40 |  |  |  | QPSK | Outer_Full (A1) |
| 41 | Default | 50 |  |  |  | QPSK | 34@0 (A2) |
| 42 | Default | 50 |  |  |  | QPSK | 115@0 (A4) |
| 43 | Default | 50 |  |  |  | QPSK | 228@0 (A2) |
| 44 | Default | 50 |  |  |  | QPSK | 5@223 (A5) |
| 45 | Default | 50 |  |  |  | QPSK | Outer_Full (A1) |
| 46 | Default | 25 |  |  |  | 256 QAM | Outer_Full (A3) |
| 47 | Default | 25 |  |  |  | 256 QAM | Edge_1RB_Right (A3) |
| 48 | Default | 30 |  |  |  | 256 QAM | Outer_Full (A3) |
| 49 | Default | 30 |  |  |  | 256 QAM | Edge_1RB_Right (A5) |

| A-MPR test parameters for NS_48 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | Uplink Configuration | Downlink Configuration | SUL Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 50 | Default | 40 |  |  |  | 256 QAM | 16@0 (A2) |
| 51 | Default | 40 |  |  |  | 256 QAM | 95@0 (A3) |
| 52 | Default | 40 |  |  |  | 256 QAM | 152@0 (A4) |
| 53 | Default | 40 |  |  |  | 256 QAM | 192@0 (A2) |
| 54 | Default | 40 |  |  |  | 256 QAM | 5@187 (A3) |
| 55 | Default | 40 |  |  |  | 256 QAM | Outer_Full (A1) |
| 56 | Default | 50 |  |  |  | 256 QAM | 34@0 (A2) |
| 57 | Default | 50 |  |  |  | 256 QAM | 115@0 (A4) |
| 58 | Default | 50 |  |  |  | 256 QAM | 228@0 (A2) |
| 59 | Default | 50 |  |  |  | 256 QAM | 5@223 (A5) |
| 60 | Default | 50 |  |  |  | 256 QAM | Outer_Full (A1) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |

Table 6.2C.5.4-4: Test Configuration Table for NS_49

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range, High range for SUL carrier Mid range for non-SUL carrier |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | 25 MHz, 30MHz, 40MHz, 50MHz for SUL carrier Lowest for non-SUL carrier |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |  |
| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | Downlink Configuration | Uplink Configuration | SUL Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | Default | 25 | N/A for A-MPR testing. | N/A | DFT-s-OFDM | QPSK | Outer_Full (A3) |
| 2 | Default | 25 |  |  |  | QPSK | Edge_1RB_Right (A3) |
| 3 | Default | 25 |  |  |  | QPSK | Edge_1RB_Left (A3) |
| 4 | Default | 30 |  |  |  | QPSK | 20@0 (A1) |
| 5 | Default | 30 |  |  |  | QPSK | 36@0 (A5) |
| 6 | Default | 30 |  |  |  | QPSK | 80@0 (A3) |
| 7 | Default | 30 |  |  |  | QPSK | 120@0 (A4) |
| 8 | Default | 30 |  |  |  | QPSK | Outer_Full (A2) |
| 9 | Default | 30 |  |  |  | QPSK | Edge_1RB_Right (A5) |
| 10 | Default | 40 |  |  |  | QPSK | 40@0 (A1) |
| 11 | Default | 40 |  |  |  | QPSK | 5@53 (A5) |
| 12 | Default | 40 |  |  |  | QPSK | 100@0 (A4) |
| 13 | Default | 40 |  |  |  | QPSK | 150@33 (A2) |
| 14 | Default | 40 |  |  |  | QPSK | 5@187 (A5) |
| 15 | Default | 40 |  |  |  | QPSK | 192@0 (A1) |
| 16 | Default | 40 |  |  |  | QPSK | Outer_Full (A1) |
| 17 | Default | 50 |  |  |  | QPSK | 5@75 (A5) |
| 18 | Default | 50 |  |  |  | QPSK | 5@215 (A5) |
| 19 | Default | 50 |  |  |  | QPSK | 175@45 (A2) |
| 20 | Default | 50 |  |  |  | QPSK | 216@0 (A1) |
| 21 | Default | 50 |  |  |  | QPSK | Outer_Full (A1) |

| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | Downlink Configuration | Uplink Configuration | SUL Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 22 | Default | 25 |  |  |  | 256 QAM | Outer_Full (A3) |
| 23 | Default | 25 |  |  |  | 256 QAM | Edge_1RB_Right (A3) |
| 24 | Default | 25 |  |  |  | 256 QAM | Edge_1RB_Left (A3) |
| 25 | Default | 30 |  |  |  | 256 QAM | 20@0 (A1) |
| 26 | Default | 30 |  |  |  | 256 QAM | 36@0 (A5) |
| 27 | Default | 30 |  |  |  | 256 QAM | 80@0 (A3) |
| 28 | Default | 30 |  |  |  | 256 QAM | 120@0 (A4) |
| 29 | Default | 30 |  |  |  | 256 QAM | Outer_Full (A2) |
| 30 | Default | 30 |  |  |  | 256 QAM | Edge_1RB_Right (A5) |
| 31 | Default | 40 |  |  |  | 256 QAM | 40@0 (A1) |
| 32 | Default | 40 |  |  |  | 256 QAM | 5@53 (A5) |
| 33 | Default | 40 |  |  |  | 256 QAM | 100@0 (A4) |
| 34 | Default | 40 |  |  |  | 256 QAM | 150@33 (A2) |
| 35 | Default | 40 |  |  |  | 256 QAM | 5@187 (A5) |
| 36 | Default | 40 |  |  |  | 256 QAM | 192@0 (A1) |
| 37 | Default | 40 |  |  |  | 256 QAM | Outer_Full (A1) |
| 38 | Default | 50 |  |  |  | 256 QAM | 5@75 (A5) |
| 39 | Default | 50 |  |  |  | 256 QAM | 5@215 (A5) |
| 40 | Default | 50 |  |  |  | 256 QAM | 175@45 (A2) |
| 41 | Default | 50 |  |  |  | 256 QAM | 216@0 (A1) |
| 42 | Default | 50 |  |  |  | 256 QAM | Outer_Full (A1) |
| 43 | Default | 25 |  |  | CP-OFDM | QPSK | Outer_Full (A3) |
| 44 | Default | 25 |  |  |  | QPSK | Edge_1RB_Right (A3) |
| 45 | Default | 25 |  |  |  | QPSK | Edge_1RB_Left (A3) |
| 46 | Default | 30 |  |  |  | QPSK | 20@0 (A1) |
| 47 | Default | 30 |  |  |  | QPSK | 36@0 (A5) |
| 48 | Default | 30 |  |  |  | QPSK | 80@0 (A3) |
| 49 | Default | 30 |  |  |  | QPSK | 120@0 (A4) |

| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | Downlink Configuration | Uplink Configuration | SUL Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 50 | Default | 30 |  |  |  | QPSK | Outer_Full (A2) |
| 51 | Default | 30 |  |  |  | QPSK | Edge_1RB_Right (A5) |
| 52 | Default | 40 |  |  |  | QPSK | 40@0 (A1) |
| 53 | Default | 40 |  |  |  | QPSK | 5@53 (A5) |
| 54 | Default | 40 |  |  |  | QPSK | 100@0 (A4) |
| 55 | Default | 40 |  |  |  | QPSK | 159@33 (A2) |
| 56 | Default | 40 |  |  |  | QPSK | 5@187 (A5) |
| 57 | Default | 40 |  |  |  | QPSK | 192@0 (A1) |
| 58 | Default | 40 |  |  |  | QPSK | Outer_Full (A1) |
| 59 | Default | 50 |  |  |  | QPSK | 5@75 (A5) |
| 60 | Default | 50 |  |  |  | QPSK | 5@215 (A5) |
| 61 | Default | 50 |  |  |  | QPSK | 175@45 (A2) |
| 62 | Default | 50 |  |  |  | QPSK | 220@0 (A1) |
| 63 | Default | 50 |  |  |  | QPSK | Outer_Full (A1) |
| 64 | Default | 25 |  |  |  | 256 QAM | Outer_Full (A3) |
| 65 | Default | 25 |  |  |  | 256 QAM | Edge_1RB_Right (A3) |
| 66 | Default | 25 |  |  |  | 256 QAM | Edge_1RB_Left (A3) |
| 67 | Default | 30 |  |  |  | 256 QAM | 20@0 (A1) |
| 68 | Default | 30 |  |  |  | 256 QAM | 36@0 (A5) |
| 69 | Default | 30 |  |  |  | 256 QAM | 80@0 (A3) |
| 70 | Default | 30 |  |  |  | 256 QAM | 120@0 (A4) |
| 71 | Default | 30 |  |  |  | 256 QAM | Outer_Full (A2) |
| 72 | Default | 30 |  |  |  | 256 QAM | Edge_1RB_Right (A5) |
| 73 | Default | 40 |  |  |  | 256 QAM | 40@0 (A1) |
| 74 | Default | 40 |  |  |  | 256 QAM | 5@53 (A5) |
| 75 | Default | 40 |  |  |  | 256 QAM | 100@0 (A4) |
| 76 | Default | 40 |  |  |  | 256 QAM | 159@33 (A2) |
| 77 | Default | 40 |  |  |  | 256 QAM | 5@187 (A5) |

| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | Downlink Configuration | Uplink Configuration | SUL Configuration |  |  |
| 78 | Default | 40 |  |  |  | 256 QAM | 192@0 (A1) |
| 79 | Default | 40 |  |  |  | 256 QAM | Outer_Full (A1) |
| 80 | Default | 50 |  |  |  | 256 QAM | 5@75 (A5) |
| 81 | Default | 50 |  |  |  | 256 QAM | 5@215 (A5) |
| 82 | Default | 50 |  |  |  | 256 QAM | 175@45 (A2) |
| 83 | Default | 50 |  |  |  | 256 QAM | 220@0 (A1) |
| 84 | Default | 50 |  |  |  | 256 QAM | Outer_Full (A1) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |

Table 6.2C.5.4-5: Test Configuration Table for NS_05

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | SUL carrier: use Fc as specified in test parameters NUL carrier: Mid range |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | SUL carrier: 5 MHz, 10 MHz, 15 MHz, 20 MHz NUL carrier: Highest |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for NUL carrier |  |  |  |
| A-MPR test parameters for NS_05 |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Config. | Uplink Config | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
| 1 | 1922.5 | 5 | N/A for A-MPR testing | N/A for A-MPR testing | A3 | DFT-s-OFDM | PI/2 BPSK | Outer_Full |
| 2 | 1925 | 10 |  |  | A1 |  | PI/2 BPSK | Outer_Full |
| 3 | 1925 | 10 |  |  | A7 |  | PI/2 BPSK | 40@10 |
| 4 | 1925 | 10 |  |  | A2 |  | PI/2 BPSK | 6@40 |
| 5 | 1935 | 10 |  |  | A4 |  | PI/2 BPSK | Outer_Full |
| 6 | 1927.5 | 15 |  |  | A1 |  | PI/2 BPSK | Outer_Full |
| 7 | 1927.5 | 15 |  |  | A7 |  | PI/2 BPSK | 60@19 |
| 8 | 1927.5 | 15 |  |  | A2 |  | PI/2 BPSK | 6@56 |
| 9 | 1932.5 | 15 |  |  | A1 |  | PI/2 BPSK | Outer_Full |
| 10 | 1932.5 | 15 |  |  | A2 |  | PI/2 BPSK | 6@68 |
| 11 | 1942.5 | 15 |  |  | A5 |  | PI/2 BPSK | Outer_Full |
| 12 | 1930 | 20 |  |  | A1 |  | PI/2 BPSK | Outer_Full |
| 13 | 1930 | 20 |  |  | A7 |  | PI/2 BPSK | 72@28 |
| 14 | 1930 | 20 |  |  | A2 |  | PI/2 BPSK | 6@76 |
| 15 | 1950 | 20 |  |  | A6 |  | PI/2 BPSK | Outer_Full |
| 16 | 1922.5 | 5 |  |  | A3 |  | QPSK | Outer_Full |
| 17 | 1925 | 10 |  |  | A1 |  | QPSK | Outer_Full |
| 18 | 1925 | 10 |  |  | A7 |  | QPSK | 40@10 |
| 19 | 1925 | 10 |  |  | A2 |  | QPSK | 6@40 |
| 20 | 1927.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 21 | 1927.5 | 15 |  |  | A7 |  | QPSK | 60@19 |
| 22 | 1927.5 | 15 |  |  | A2 |  | QPSK | 6@56 |
| 23 | 1932.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 24 | 1932.5 | 15 |  |  | A2 |  | QPSK | 6@68 |
| 25 | 1942.5 | 15 |  |  | A5 |  | QPSK | Outer_Full |
| 26 | 1930 | 20 |  |  | A1 |  | QPSK | Outer_Full |
| 27 | 1930 | 20 |  |  | A7 |  | QPSK | 72@28 |
| 28 | 1930 | 20 |  |  | A2 |  | QPSK | 6@76 |
| 29 | 1950 | 20 |  |  | A6 |  | QPSK | Outer_Full |
| 30 | 1922.5 | 5 |  |  | A3 |  | 16 QAM | Outer_Full |
| 31 | 1925 | 10 |  |  | A1 |  | 16 QAM | Outer_Full |
| 32 | 1925 | 10 |  |  | A7 |  | 16 QAM | 40@10 |
| 33 | 1925 | 10 |  |  | A2 |  | 16 QAM | 6@40 |
| 34 | 1927.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 35 | 1927.5 | 15 |  |  | A7 |  | 16 QAM | 60@19 |
| 36 | 1927.5 | 15 |  |  | A2 |  | 16 QAM | 6@56 |
| 37 | 1932.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 38 | 1932.5 | 15 |  |  | A2 |  | 16 QAM | 6@68 |
| 39 | 1930 | 20 |  |  | A1 |  | 16 QAM | Outer_Full |
| 40 | 1930 | 20 |  |  | A7 |  | 16 QAM | 72@28 |
| 41 | 1930 | 20 |  |  | A2 |  | 16 QAM | 6@76 |
| 42 | 1922.5 | 5 |  |  | A3 |  | 64 QAM | Outer_Full |
| 43 | 1925 | 10 |  |  | A1 |  | 64 QAM | Outer_Full |
| 44 | 1925 | 10 |  |  | A7 |  | 64 QAM | 40@10 |

| A-MPR test parameters for NS_05 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Config. | Uplink Config | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
| 45 | 1925 | 10 |  |  | A2 |  | 64 QAM | 6@40 |
| 46 | 1927.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 47 | 1927.5 | 15 |  |  | A7 |  | 64 QAM | 60@19 |
| 48 | 1927.5 | 15 |  |  | A2 |  | 64 QAM | 6@56 |
| 49 | 1932.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 50 | 1932.5 | 15 |  |  | A2 |  | 64 QAM | 6@68 |
| 51 | 1930 | 20 |  |  | A1 |  | 64 QAM | Outer_Full |
| 52 | 1930 | 20 |  |  | A7 |  | 64 QAM | 72@28 |
| 53 | 1930 | 20 |  |  | A2 |  | 64 QAM | 6@76 |
| 54 | 1922.5 | 5 |  |  | A3 |  | 256 QAM | Outer_Full |
| 55 | 1925 | 10 |  |  | A1 |  | 256 QAM | Outer_Full |
| 56 | 1925 | 10 |  |  | A7 |  | 256 QAM | 40@10 |
| 57 | 1925 | 10 |  |  | A2 |  | 256 QAM | 6@40 |
| 58 | 1927.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 59 | 1927.5 | 15 |  |  | A7 |  | 256 QAM | 60@19 |
| 60 | 1927.5 | 15 |  |  | A2 |  | 256 QAM | 6@56 |
| 61 | 1932.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 62 | 1932.5 | 15 |  |  | A2 |  | 256 QAM | 6@68 |
| 63 | 1930 | 20 |  |  | A1 |  | 256 QAM | Outer_Full |
| 64 | 1930 | 20 |  |  | A7 |  | 256 QAM | 72@28 |
| 65 | 1930 | 20 |  |  | A2 |  | 256 QAM | 6@76 |
| 66 | 1922.5 | 5 |  |  | A3 | CP-OFDM | QPSK | Outer_Full |
| 67 | 1925 | 10 |  |  | A1 |  | QPSK | Outer_Full |
| 68 | 1925 | 10 |  |  | A7 |  | QPSK | 42@10 |
| 69 | 1925 | 10 |  |  | A2 |  | QPSK | 6@40 |
| 70 | 1935 | 10 |  |  | A4 |  | QPSK | Outer_Full |
| 71 | 1927.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 72 | 1927.5 | 15 |  |  | A7 |  | QPSK | 60@19 |
| 73 | 1927.5 | 15 |  |  | A2 |  | QPSK | 6@56 |
| 74 | 1932.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 75 | 1932.5 | 15 |  |  | A2 |  | QPSK | 6@68 |
| 76 | 1942.5 | 15 |  |  | A5 |  | QPSK | Outer_Full |
| 77 | 1930 | 20 |  |  | A1 |  | QPSK | Outer_Full |
| 78 | 1930 | 20 |  |  | A7 |  | QPSK | 78@28 |
| 79 | 1930 | 20 |  |  | A2 |  | QPSK | 6@76 |
| 80 | 1950 | 20 |  |  | A6 |  | QPSK | Outer_Full |
| 81 | 1922.5 | 5 |  |  | A3 |  | 16 QAM | Outer_Full |
| 82 | 1925 | 10 |  |  | A1 |  | 16 QAM | Outer_Full |
| 83 | 1925 | 10 |  |  | A7 |  | 16 QAM | 42@10 |
| 84 | 1925 | 10 |  |  | A2 |  | 16 QAM | 6@40 |
| 85 | 1935 | 10 |  |  | A4 |  | 16 QAM | Outer_Full |
| 86 | 1927.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 87 | 1927.5 | 15 |  |  | A7 |  | 16 QAM | 60@19 |
| 88 | 1927.5 | 15 |  |  | A2 |  | 16 QAM | 6@56 |
| 89 | 1932.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 90 | 1932.5 | 15 |  |  | A2 |  | 16 QAM | 6@68 |
| 91 | 1942.5 | 15 |  |  | A5 |  | 16 QAM | Outer_Full |
| 92 | 1930 | 20 |  |  | A1 |  | 16 QAM | Outer_Full |
| 93 | 1930 | 20 |  |  | A7 |  | 16 QAM | 78@28 |
| 94 | 1930 | 20 |  |  | A2 |  | 16 QAM | 6@76 |
| 95 | 1950 | 20 |  |  | A6 |  | 16 QAM | Outer_Full |
| 96 | 1922.5 | 5 |  |  | A3 |  | 64 QAM | Outer_Full |
| 97 | 1925 | 10 |  |  | A1 |  | 64 QAM | Outer_Full |
| 98 | 1925 | 10 |  |  | A7 |  | 64 QAM | 42@10 |
| 99 | 1925 | 10 |  |  | A2 |  | 64 QAM | 6@40 |

| A-MPR test parameters for NS_05 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Config. | Uplink Config | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
| 100 | 1927.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 101 | 1927.5 | 15 |  |  | A7 |  | 64 QAM | 60@19 |
| 102 | 1927.5 | 15 |  |  | A2 |  | 64 QAM | 6@56 |
| 103 | 1932.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 104 | 1932.5 | 15 |  |  | A2 |  | 64 QAM | 6@68 |
| 105 | 1930 | 20 |  |  | A1 |  | 64 QAM | Outer_Full |
| 106 | 1930 | 20 |  |  | A7 |  | 64 QAM | 78@28 |
| 107 | 1930 | 20 |  |  | A2 |  | 64 QAM | 6@76 |
| 108 | 1922.5 | 5 |  |  | A3 |  | 256 QAM | Outer_Full |
| 109 | 1925 | 10 |  |  | A1 |  | 256 QAM | Outer_Full |
| 110 | 1925 | 10 |  |  | A7 |  | 256 QAM | 42@10 |
| 111 | 1927.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 112 | 1927.5 | 15 |  |  | A7 |  | 256 QAM | 60@19 |
| 113 | 1932.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 114 | 1930 | 20 |  |  | A1 |  | 256 QAM | Outer_Full |
| 115 | 1930 | 20 |  |  | A7 |  | 256 QAM | 78@28 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |

Table 6.2C.5.4-6: Test Configuration Table for NS_05U

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | SUL carrier: Use Fc as specified in test parameters NUL carrier: Mid range |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | SUL carrier: 5 MHz, 10 MHz, 15 MHz, 20 MHz NUL carrier: Highest |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 15kHz for SUL and Lowest supported SCS for NUL carrier |  |  |  |
| AMPR Test parameters for NS_05U |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Config. | Uplink config | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
| 1 | 1922.5 | 5 | N/A | N/A | A3 | DFT-s-OFDM | PI/2 BPSK | Outer_Full |
| 2 | 1925 | 10 |  |  | A1 |  | PI/2 BPSK | Outer_Full |
| 3 | 1925 | 10 |  |  | A7 |  | PI/2 BPSK | 40@10 |
| 4 | 1925 | 10 |  |  | A2 |  | PI/2 BPSK | 6@40 |
| 5 | 1935 | 10 |  |  | A4 |  | PI/2 BPSK | Outer_Full |
| 6 | 1927.5 | 15 |  |  | A1 |  | PI/2 BPSK | Outer_Full |
| 7 | 1927.5 | 15 |  |  | A7 |  | PI/2 BPSK | 60@19 |
| 8 | 1927.5 | 15 |  |  | A2 |  | PI/2 BPSK | 6@56 |
| 9 | 1932.5 | 15 |  |  | A1 |  | PI/2 BPSK | Outer_Full |
| 10 | 1932.5 | 15 |  |  | A2 |  | PI/2 BPSK | 6@68 |
| 11 | 1942.5 | 15 |  |  | A5 |  | PI/2 BPSK | Outer_Full |
| 12 | 1930 | 20 |  |  | A1 |  | PI/2 BPSK | Outer_Full |
| 13 | 1930 | 20 |  |  | A7 |  | PI/2 BPSK | 72@28 |
| 14 | 1930 | 20 |  |  | A2 |  | PI/2 BPSK | 6@76 |
| 15 | 1950 | 20 |  |  | A6 |  | PI/2 BPSK | Outer_Full |
| 16 | 1922.5 | 5 |  |  | A3 |  | QPSK | Outer_Full |
| 17 | 1925 | 10 |  |  | A1 |  | QPSK | Outer_Full |
| 18 | 1925 | 10 |  |  | A7 |  | QPSK | 40@10 |
| 19 | 1925 | 10 |  |  | A2 |  | QPSK | 6@40 |
| 20 | 1935 | 10 |  |  | A4 |  | QPSK | Outer_Full |
| 21 | 1927.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 22 | 1927.5 | 15 |  |  | A7 |  | QPSK | 60@19 |
| 23 | 1927.5 | 15 |  |  | A2 |  | QPSK | 6@56 |
| 24 | 1932.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 25 | 1932.5 | 15 |  |  | A2 |  | QPSK | 6@68 |
| 26 | 1942.5 | 15 |  |  | A5 |  | QPSK | Outer_Full |
| 27 | 1930 | 20 |  |  | A1 |  | QPSK | Outer_Full |
| 28 | 1930 | 20 |  |  | A7 |  | QPSK | 72@28 |
| 29 | 1930 | 20 |  |  | A2 |  | QPSK | 6@76 |
| 30 | 1950 | 20 |  |  | A6 |  | QPSK | Outer_Full |
| 31 | 1922.5 | 5 |  |  | A3 |  | 16 QAM | Outer_Full |
| 32 | 1925 | 10 |  |  | A1 |  | 16 QAM | Outer_Full |
| 33 | 1925 | 10 |  |  | A7 |  | 16 QAM | 40@10 |
| 34 | 1925 | 10 |  |  | A2 |  | 16 QAM | 6@40 |
| 35 | 1935 | 10 |  |  | A4 |  | 16 QAM | Outer_Full |
| 36 | 1927.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 37 | 1927.5 | 15 |  |  | A7 |  | 16 QAM | 60@19 |
| 38 | 1927.5 | 15 |  |  | A2 |  | 16 QAM | 6@56 |
| 39 | 1932.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 40 | 1932.5 | 15 |  |  | A2 |  | 16 QAM | 6@68 |
| 41 | 1942.5 | 15 |  |  | A5 |  | 16 QAM | Outer_Full |
| 42 | 1930 | 20 |  |  | A1 |  | 16 QAM | Outer_Full |
| 43 | 1930 | 20 |  |  | A7 |  | 16 QAM | 72@28 |
| 44 | 1930 | 20 |  |  | A2 |  | 16 QAM | 6@76 |

| AMPR Test parameters for NS_05U |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Config. | Uplink config | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
| 45 | 1950 | 20 |  |  | A6 |  | 16 QAM | Outer_Full |
| 46 | 1922.5 | 5 |  |  | A3 |  | 64 QAM | Outer_Full |
| 47 | 1925 | 10 |  |  | A1 |  | 64 QAM | Outer_Full |
| 48 | 1925 | 10 |  |  | A7 |  | 64 QAM | 40@10 |
| 49 | 1925 | 10 |  |  | A2 |  | 64 QAM | 6@40 |
| 50 | 1935 | 10 |  |  | A4 |  | 64 QAM | Outer_Full |
| 51 | 1927.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 52 | 1927.5 | 15 |  |  | A7 |  | 64 QAM | 60@19 |
| 53 | 1927.5 | 15 |  |  | A2 |  | 64 QAM | 6@56 |
| 54 | 1932.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 55 | 1932.5 | 15 |  |  | A2 |  | 64 QAM | 6@68 |
| 56 | 1942.5 | 15 |  |  | A5 |  | 64 QAM | Outer_Full |
| 57 | 1930 | 20 |  |  | A1 |  | 64 QAM | Outer_Full |
| 58 | 1930 | 20 |  |  | A7 |  | 64 QAM | 72@28 |
| 59 | 1930 | 20 |  |  | A2 |  | 64 QAM | 6@76 |
| 60 | 1950 | 20 |  |  | A6 |  | 64 QAM | Outer_Full |
| 61 | 1922.5 | 5 |  |  | A3 |  | 256 QAM | Outer_Full |
| 62 | 1925 | 10 |  |  | A1 |  | 256 QAM | Outer_Full |
| 63 | 1925 | 10 |  |  | A7 |  | 256 QAM | 40@10 |
| 64 | 1925 | 10 |  |  | A2 |  | 256 QAM | 6@40 |
| 65 | 1935 | 10 |  |  | A4 |  | 256 QAM | Outer_Full |
| 66 | 1927.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 67 | 1927.5 | 15 |  |  | A7 |  | 256 QAM | 60@19 |
| 68 | 1927.5 | 15 |  |  | A2 |  | 256 QAM | 6@56 |
| 69 | 1932.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 70 | 1932.5 | 15 |  |  | A2 |  | 256 QAM | 6@68 |
| 71 | 1942.5 | 15 |  |  | A5 |  | 256 QAM | Outer_Full |
| 72 | 1930 | 20 |  |  | A1 |  | 256 QAM | Outer_Full |
| 73 | 1930 | 20 |  |  | A7 |  | 256 QAM | 72@28 |
| 74 | 1930 | 20 |  |  | A2 |  | 256 QAM | 6@76 |
| 75 | 1950 | 20 |  |  | A6 |  | 256 QAM | Outer_Full |
| 76 | 1922.5 | 5 |  |  | A3 | CP-OFDM | QPSK | Outer_Full |
| 77 | 1925 | 10 |  |  | A1 |  | QPSK | Outer_Full |
| 78 | 1925 | 10 |  |  | A7 |  | QPSK | 42@10 |
| 79 | 1925 | 10 |  |  | A2 |  | QPSK | 6@40 |
| 80 | 1935 | 10 |  |  | A4 |  | QPSK | Outer_Full |
| 81 | 1927.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 82 | 1927.5 | 15 |  |  | A7 |  | QPSK | 60@19 |
| 83 | 1927.5 | 15 |  |  | A2 |  | QPSK | 6@56 |
| 84 | 1932.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 85 | 1932.5 | 15 |  |  | A2 |  | QPSK | 6@68 |
| 86 | 1942.5 | 15 |  |  | A5 |  | QPSK | Outer_Full |
| 87 | 1930 | 20 |  |  | A1 |  | QPSK | Outer_Full |
| 88 | 1930 | 20 |  |  | A7 |  | QPSK | 78@28 |
| 89 | 1930 | 20 |  |  | A2 |  | QPSK | 6@76 |
| 90 | 1950 | 20 |  |  | A6 |  | QPSK | Outer_Full |
| 91 | 1922.5 | 5 |  |  | A3 |  | 16 QAM | Outer_Full |
| 92 | 1925 | 10 |  |  | A1 |  | 16 QAM | Outer_Full |
| 93 | 1925 | 10 |  |  | A7 |  | 16 QAM | 42@10 |
| 94 | 1925 | 10 |  |  | A2 |  | 16 QAM | 6@40 |
| 95 | 1935 | 10 |  |  | A4 |  | 16 QAM | Outer_Full |
| 96 | 1927.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 97 | 1927.5 | 15 |  |  | A7 |  | 16 QAM | 60@19 |
| 98 | 1927.5 | 15 |  |  | A2 |  | 16 QAM | 6@56 |
| 99 | 1932.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |

| AMPR Test parameters for NS_05U |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Config. | Uplink config | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
| 100 | 1932.5 | 15 |  |  | A2 |  | 16 QAM | 6@68 |
| 101 | 1942.5 | 15 |  |  | A5 |  | 16 QAM | Outer_Full |
| 102 | 1930 | 20 |  |  | A1 |  | 16 QAM | Outer_Full |
| 103 | 1930 | 20 |  |  | A7 |  | 16 QAM | 78@28 |
| 104 | 1930 | 20 |  |  | A2 |  | 16 QAM | 6@76 |
| 105 | 1950 | 20 |  |  | A6 |  | 16 QAM | Outer_Full |
| 106 | 1922.5 | 5 |  |  | A3 |  | 64 QAM | Outer_Full |
| 107 | 1925 | 10 |  |  | A1 |  | 64 QAM | Outer_Full |
| 108 | 1925 | 10 |  |  | A7 |  | 64 QAM | 42@10 |
| 109 | 1925 | 10 |  |  | A2 |  | 64 QAM | 6@40 |
| 110 | 1935 | 10 |  |  | A4 |  | 64 QAM | Outer_Full |
| 111 | 1927.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 112 | 1927.5 | 15 |  |  | A7 |  | 64 QAM | 60@19 |
| 113 | 1927.5 | 15 |  |  | A2 |  | 64 QAM | 6@56 |
| 114 | 1932.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 115 | 1932.5 | 15 |  |  | A2 |  | 64 QAM | 6@68 |
| 116 | 1942.5 | 15 |  |  | A5 |  | 64 QAM | Outer_Full |
| 117 | 1930 | 20 |  |  | A1 |  | 64 QAM | Outer_Full |
| 118 | 1930 | 20 |  |  | A7 |  | 64 QAM | 78@28 |
| 119 | 1930 | 20 |  |  | A2 |  | 64 QAM | 6@76 |
| 120 | 1950 | 20 |  |  | A6 |  | 64 QAM | Outer_Full |
| 121 | 1922.5 | 5 |  |  | A3 |  | 256 QAM | Outer_Full |
| 122 | 1925 | 10 |  |  | A1 |  | 256 QAM | Outer_Full |
| 123 | 1925 | 10 |  |  | A7 |  | 256 QAM | 42@10 |
| 124 | 1935 | 10 |  |  | A4 |  | 256 QAM | Outer_Full |
| 125 | 1927.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 126 | 1927.5 | 15 |  |  | A7 |  | 256 QAM | 60@19 |
| 127 | 1932.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 128 | 1942.5 | 15 |  |  | A5 |  | 256 QAM | Outer_Full |
| 129 | 1930 | 20 |  |  | A1 |  | 256 QAM | Outer_Full |
| 130 | 1930 | 20 |  |  | A7 |  | 256 QAM | 78@28 |
| 131 | 1950 | 20 |  |  | A6 |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |

Table 6.2C.5.4-7: Test Configuration Table for NS_100

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | SUL carrier:Low range, High range NUL carrier: Mid range |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | SUL carrier:Lowest, Highest NUL carrier: Highest |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | 15kHz for SUL and Lowest supported SCS for NUL carriers |
| Test parameters for NS_100 |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration | SUL Configuration |  |  |
|  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | Low | N/A | N/A | DFT-s-OFDM | PI/2 BPSK | Edge_1RB_Left |
| 2 | High |  |  |  | PI/2 BPSK | Edge_1RB_Right |
| 3 | Default |  |  |  | PI/2 BPSK | Outer_Full |
| 4 | Low |  |  |  | QPSK | Edge_1RB_Left |
| 5 | High |  |  |  | QPSK | Edge_1RB_Right |
| 6 | Default |  |  |  | QPSK | Outer_Full |
| 7 | Low |  |  |  | 16 QAM | Edge_1RB_Left |
| 8 | High |  |  |  | 16 QAM | Edge_1RB_Right |
| 9 | Default |  |  |  | 16 QAM | Outer_Full |
| 10 | Low |  |  |  | 64 QAM | Edge_1RB_Left |
| 11 | High |  |  |  | 64 QAM | Edge_1RB_Right |
| 12 | Default |  |  |  | 64 QAM | Outer_Full |
| 13 | Low |  |  |  | 256 QAM | Edge_1RB_Left |
| 14 | High |  |  |  | 256 QAM | Edge_1RB_Right |
| 15 | Default |  |  |  | 256 QAM | Outer_Full |
| 16 | Low |  |  | CP-s OFDM | QPSK | Edge_1RB_Left |
| 17 | High |  |  |  | QPSK | Edge_1RB_Right |
| 18 | Default |  |  |  | QPSK | Outer_Full |
| 19 | Low |  |  |  | 16 QAM | Edge_1RB_Left |
| 20 | High |  |  |  | 16 QAM | Edge_1RB_Right |
| 21 | Default |  |  |  | 16 QAM | Outer_Full |
| 22 | Low |  |  |  | 64 QAM | Edge_1RB_Left |
| 23 | High |  |  |  | 64 QAM | Edge_1RB_Right |
| 24 | Default |  |  |  | 64 QAM | Outer_Full |
| 25 | Low |  |  |  | 256 QAM | Edge_1RB_Left |
| 26 | High |  |  |  | 256 QAM | Edge_1RB_Right |
| 27 | Default |  |  |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Void. NOTE 4: Void |  |  |  |  |  |  |

Table 6.2C.5.4-8: Test Configuration Table for NS_56

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Use uplink carrier center frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 5 MHz, 10 MHz as specified in test parameters for SUL carrier Lowest for non-SUL carrier |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | 15kHz for SUL and Lowest supported SCS for NUL carriers |  |
| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Configuration | Uplink Configuration | A-MPR | Uplink Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
|  |  |  |  |  |  |  |  | SCS 15 kHz |
| 1 | Low | 5 | N/A for A-MPR testing | N/A | 14 | DFT-s-OFDM | PI/2 BPSK | Edge_1RB_Left |
| 2 | Low | 5 |  |  | 6 |  | PI/2 BPSK | Outer_Full |
| 3 | Low | 5 |  |  | 4 |  | PI/2 BPSK | 20@4 |
| 4 | Low | 5 |  |  | 4 |  | PI/2 BPSK | 1@4 |
| 5 | Low | 5 |  |  | 14 |  | QPSK | Edge_1RB_Left |
| 6 | Low | 5 |  |  | 6 |  | QPSK | Outer_Full |
| 7 | Low | 5 |  |  | 4 |  | QPSK | 20@4 |
| 8 | Low | 5 |  |  | 4 |  | QPSK | 1@4 |
| 9 | Low | 5 |  |  | 14 |  | 16 QAM | Edge_1RB_Left |
| 10 | Low | 5 |  |  | 6 |  | 16 QAM | Outer_Full |
| 11 | Low | 5 |  |  | 4 |  | 16 QAM | 20@4 |
| 12 | Low | 5 |  |  | 4 |  | 16 QAM | 1@4 |
| 13 | Low | 5 |  |  | 14 |  | 64 QAM | Edge_1RB_Left |
| 14 | Low | 5 |  |  | 6 |  | 64 QAM | Outer_Full |
| 15 | Low | 5 |  |  | 4 |  | 64 QAM | 20@4 |
| 16 | Low | 5 |  |  | 4 |  | 64 QAM | 1@4 |
| 17 | Low | 5 |  |  | 14 |  | 256 QAM | Edge_1RB_Left |
| 18 | Low | 5 |  |  | 6 |  | 256 QAM | Outer_Full |
| 19 | Low | 5 |  |  | 4 |  | 256 QAM | 20@4 |
| 20 | Low | 5 |  |  | 4 |  | 256 QAM | 1@4 |
| 21 | Low | 10 |  |  | 12 |  | PI/2 BPSK | Edge_1RB_Left |
| 22 | Low | 10 |  |  | 8 |  | PI/2 BPSK | Outer_Full |

| 23 | Low | 10 |  |  | 6 |  | PI/2 BPSK | 1@3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | Low | 10 |  |  | 6 |  | PI/2 BPSK | 40@9 |
| 25 | Low | 10 |  |  | 4 |  | PI/2 BPSK | 1@35 |
| 26 | Low | 10 |  |  | 2 |  | PI/2 BPSK | 16@35 |
| 27 | Low | 10 |  |  | 5 |  | PI/2 BPSK | Edge_1RB_Right |
| 28 | Low | 10 |  |  | 5 |  | PI/2 BPSK | 1@40 |
| 29 | Low | 10 |  |  | 3 |  | PI/2 BPSK | 8@44 |
| 30 | Low | 10 |  |  | 12 |  | QPSK | Edge_1RB_Left |
| 31 | Low | 10 |  |  | 8 |  | QPSK | Outer_Full |
| 32 | Low | 10 |  |  | 6 |  | QPSK | 1@3 |
| 33 | Low | 10 |  |  | 6 |  | QPSK | 40@9 |
| 34 | Low | 10 |  |  | 4 |  | QPSK | 1@35 |
| 35 | Low | 10 |  |  | 2 |  | QPSK | 16@35 |
| 36 | Low | 10 |  |  | 5 |  | QPSK | Edge_1RB_Right |
| 37 | Low | 10 |  |  | 5 |  | QPSK | 1@40 |
| 38 | Low | 10 |  |  | 3 |  | QPSK | 8@44 |
| 39 | Low | 10 |  |  | 12 |  | 16 QAM | Edge_1RB_Left |
| 40 | Low | 10 |  |  | 8 |  | 16 QAM | Outer_Full |
| 41 | Low | 10 |  |  | 6 |  | 16 QAM | 1@3 |
| 42 | Low | 10 |  |  | 6 |  | 16 QAM | 40@9 |
| 43 | Low | 10 |  |  | 4 |  | 16 QAM | 1@35 |
| 44 | Low | 10 |  |  | 2 |  | 16 QAM | 16@35 |
| 45 | Low | 10 |  |  | 5 |  | 16 QAM | Edge_1RB_Right |
| 46 | Low | 10 |  |  | 5 |  | 16 QAM | 1@40 |
| 47 | Low | 10 |  |  | 3 |  | 16 QAM | 8@44 |
| 48 | Low | 10 |  |  | 12 |  | 64 QAM | Edge_1RB_Left Outer_Full |
| 49 | Low | 10 |  |  | 8 |  | 64 QAM |  |
| 50 | Low | 10 |  |  | 6 |  | 64 QAM | 1@3 40@9 |
| 51 | Low | 10 |  |  | 6 |  | 64 QAM |  |
| 52 | Low | 10 |  |  | 4 |  | 64 QAM | 1@35 |
| 53 | Low | 10 |  |  | 2 |  | 64 QAM | 16@35 |

| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Configuration | Uplink Configuration | A-MPR | Uplink Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
|  |  |  |  |  |  |  |  | SCS 15 kHz |
| 54 | Low | 10 |  |  | 5 |  | 64 QAM | Edge_1RB_Right |
| 55 | Low | 10 |  |  | 5 |  | 64 QAM | 1@40 |
| 56 | Low | 10 |  |  | 3 |  | 64 QAM | 8@44 |
| 57 | Low | 10 |  |  | 12 |  | 256 QAM | Edge_1RB_Left |
| 58 | Low | 10 |  |  | 8 |  | 256 QAM | Outer_Full |
| 59 | Low | 10 |  |  | 6 |  | 256 QAM | 1@3 |
| 60 | Low | 10 |  |  | 6 |  | 256 QAM | 40@9 |
| 61 | Low | 10 |  |  | 4 |  | 256 QAM | 1@35 |
| 62 | Low | 10 |  |  | 2 |  | 256 QAM | 16@35 |
| 63 | Low | 10 |  |  | 5 |  | 256 QAM | Edge_1RB_Right |
| 64 | Low | 10 |  |  | 5 |  | 256 QAM | 1@40 |
| 65 | Low | 10 |  |  | 3 |  | 256 QAM | 8@44 |
| 66 | Low | 5 | N/A for A-MPR testing | N/A | 14 | CP-OFDM | QPSK | Edge_1RB_Left |
| 67 | Low | 5 |  |  | 6 |  | QPSK | Outer_Full |
| 68 | Low | 5 |  |  | 4 |  | QPSK | 21@4 |
| 69 | Low | 5 |  |  | 4 |  | QPSK | 1@4 |
| 70 | Low | 5 |  |  | 14 |  | 16 QAM | Edge_1RB_Left |
| 71 | Low | 5 |  |  | 6 |  | 16 QAM | Outer_Full |
| 72 | Low | 5 |  |  | 4 |  | 16 QAM | 21@4 |
| 73 | Low | 5 |  |  | 4 |  | 16 QAM | 1@4 |
| 74 | Low | 5 |  |  | 14 |  | 64 QAM | Edge_1RB_Left |
| 75 | Low | 5 |  |  | 6 |  | 64 QAM | Outer_Full |
| 76 | Low | 5 |  |  | 4 |  | 64 QAM | 21@4 |
| 77 | Low | 5 |  |  | 4 |  | 64 QAM | 1@4 |
| 78 | Low | 5 |  |  | 14 |  | 256 QAM | Edge_1RB_Left |
| 79 | Low | 5 |  |  | 6 |  | 256 QAM | Outer_Full |
| 80 | Low | 5 |  |  | 4 |  | 256 QAM | 21@4 |
| 81 | Low | 5 |  |  | 4 |  | 256 QAM | 1@4 |
| 82 | Low | 10 |  |  | 12 |  | QPSK | Edge_1RB_Left |
| 83 | Low | 10 |  |  | 8 |  | QPSK | Outer_Full |
| 84 | Low | 10 |  |  | 6 |  | QPSK | 1@3 |

| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Configuration | Uplink Configuration | A-MPR | Uplink Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
|  |  |  |  |  |  |  |  | SCS 15 kHz |
| 85 | Low | 10 |  |  | 6 |  | QPSK | 43@9 |
| 86 | Low | 10 |  |  | 4 |  | QPSK | 1@35 |
| 87 | Low | 10 |  |  | 2 |  | QPSK | 17@35 |
| 88 | Low | 10 |  |  | 5 |  | QPSK | Edge_1RB_Right |
| 89 | Low | 10 |  |  | 5 |  | QPSK | 1@40 |
| 90 | Low | 10 |  |  | 3 |  | QPSK | 8@44 |
| 91 | Low | 10 |  |  | 12 |  | 16 QAM | Edge_1RB_Left |
| 92 | Low | 10 |  |  | 8 |  | 16 QAM | Outer_Full |
| 93 | Low | 10 |  |  | 6 |  | 16 QAM | 1@3 |
| 94 | Low | 10 |  |  | 6 |  | 16 QAM | 43@9 |
| 95 | Low | 10 |  |  | 4 |  | 16 QAM | 1@35 |
| 96 | Low | 10 |  |  | 2 |  | 16 QAM | 17@35 |
| 97 | Low | 10 |  |  | 5 |  | 16 QAM | Edge_1RB_Right |
| 98 | Low | 10 |  |  | 5 |  | 16 QAM | 1@40 |
| 99 | Low | 10 |  |  | 3 |  | 16 QAM | 8@44 |
| 100 | Low | 10 |  |  | 12 |  | 64 QAM | Edge_1RB_Left |
| 101 | Low | 10 |  |  | 8 |  | 64 QAM | Outer_Full |
| 102 | Low | 10 |  |  | 6 |  | 64 QAM | 1@3 |
| 103 | Low | 10 |  |  | 6 |  | 64 QAM | 43@9 |
| 104 | Low | 10 |  |  | 4 |  | 64 QAM | 1@35 |
| 105 | Low | 10 |  |  | 2 |  | 64 QAM | 17@35 |
| 106 | Low | 10 |  |  | 5 |  | 64 QAM | Edge_1RB_Right |
| 107 | Low | 10 |  |  | 5 |  | 64 QAM | 1@40 |
| 108 | Low | 10 |  |  | 3 |  | 64 QAM | 8@44 |
| 109 | Low | 10 |  |  | 12 |  | 256 QAM | Edge_1RB_Left |

| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Configuration | Uplink Configuration | A-MPR | Uplink Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
|  |  |  |  |  |  |  |  | SCS 15 kHz |
| 110 | Low | 10 |  |  | 8 |  | 256 QAM | Outer_Full |
| 111 | Low | 10 |  |  | 6 |  | 256 QAM | 1@3 |
| 112 | Low | 10 |  |  | 6 |  | 256 QAM | 43@9 |
| 113 | Low | 10 |  |  | 4 |  | 256 QAM | 1@35 |
| 114 | Low | 10 |  |  | 2 |  | 256 QAM | 17@35 |
| 115 | Low | 10 |  |  | 5 |  | 256 QAM | Edge_1RB_Right |
| 116 | Low | 10 |  |  | 5 |  | 256 QAM | 1@40 |
| 117 | Low | 10 |  |  | 3 |  | 256 QAM | 8@44 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |

Table 6.2C.5.4-9: Test Configuration Table for NS_43

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | SUL carrier: Use uplink carrier centre frequency (Fc) as specified in test parameters NUL carrier: Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | SUL carrier: 5 MHz, 10 MHz, 15 MHz NUL carrier: Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | 15kHz for SUL and Lowest supported SCS for NUL carrier |  |
| A-MPR test parameters for NS_43 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | Downlink Configuration | Uplink Configuration | SUL Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | 910 | 10 | N/A | N/A | DFT-s--OFDM | PI/2 BPSK | Outer_Full (A2) |
| 2 | 907.5 | 15 |  |  |  | PI/2 BPSK | Edge_1RB_Left (A6) |
| 3 | 907.5 | 15 |  |  |  | PI/2 BPSK | Outer_Full (A6) |
| 4 | 902.5 | 5 |  |  |  | QPSK | Outer_Full (A1) |
| 5 | 910 | 10 |  |  |  | QPSK | Outer_Full (A4) |
| 6 | 907.5 | 15 |  |  |  | QPSK | Edge_1RB_Left (A6) |
| 7 | 907.5 | 15 |  |  |  | QPSK | Outer_Full (A6) |
| 8 | 910 | 10 |  |  |  | 16 QAM | Outer_Full (A5) |
| 9 | 907.5 | 15 |  |  |  | 16 QAM | Edge_1RB_Left (A6) |
| 10 | 907.5 | 15 |  |  |  | 16 QAM | Outer_Full (A6) |
| 11 | 910 | 10 |  |  |  | 64 QAM | Outer_Full (A3) |
| 12 | 907.5 | 15 |  |  |  | 64 QAM | Edge_1RB_Left (A6) |
| 13 | 907.5 | 15 |  |  |  | 64 QAM | Outer_Full (A6) |
| 14 | 907.5 | 15 |  |  |  | 256 QAM | Edge_1RB_Left (A6) |
| 15 | 907.5 | 15 |  |  |  | 256 QAM | Outer_Full (A6) |
| 16 | 902.5 | 5 |  |  | CP-OFDM | QPSK | Outer_Full (A1) |
| 17 | 910 | 10 |  |  |  | QPSK | Outer_Full (A5) |
| 18 | 907.5 | 15 |  |  |  | QPSK | Edge_1RB_Left (A6) |
| 19 | 907.5 | 15 |  |  |  | QPSK | Outer_Full (A6) |
| 20 | 902.5 | 5 |  |  |  | 16 QAM | Outer_Full (A1) |
| 21 | 910 | 10 |  |  |  | 16 QAM | Outer_Full (A5) |
| 22 | 907.5 | 15 |  |  |  | 16 QAM | Edge_1RB_Left (A6) |
| 23 | 907.5 | 15 |  |  |  | 16 QAM | Outer_Full (A6) |
| 24 | 910 | 10 |  |  |  | 64 QAM | Outer_Full (A3) |
| 25 | 907.5 | 15 |  |  |  | 64 QAM | Edge_1RB_Left (A6) |
| 26 | 907.5 | 15 |  |  |  | 64 QAM | Outer_Full (A6) |
| 27 | 907.5 | 15 |  |  |  | 256 QAM | Edge_1RB_Left (A6) |
| 28 | 907.5 | 15 |  |  |  | 256 QAM | Outer_Full (A6) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |

Table 6.2C.5.4-10: Test Configuration Table for NS_43U

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | SUL carrier: Use uplink carrier centre frequency (Fc) as specified in test parameters NUL carrier: Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | SUL carrier: 5 MHz, 10 MHz, 15 MHz NUL carrier: Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | 15kHz for SUL and Lowest supported SCS for NUL carrier |  |
| A-MPR test parameters for NS_43U |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | Downlink Configuration | Uplink Configuration | SUL Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | 902.5 | 5 | N/A | N/A | DFT-s--OFDM | PI/2 BPSK | Outer_Full |
| 2 | 910 | 10 |  |  |  | PI/2 BPSK | Outer_Full |
| 3 | 907.5 | 15 |  |  |  | PI/2 BPSK | Edge_1RB_Left |
| 4 | 907.5 | 15 |  |  |  | PI/2 BPSK | Outer_Full |
| 5 | 902.5 | 5 |  |  |  | QPSK | Outer_Full |
| 6 | 910 | 10 |  |  |  | QPSK | Outer_Full |
| 7 | 907.5 | 15 |  |  |  | QPSK | Edge_1RB_Left |
| 8 | 907.5 | 15 |  |  |  | QPSK | Outer_Full |
| 9 | 902.5 | 5 |  |  |  | 16 QAM | Outer_Full |
| 10 | 910 | 10 |  |  |  | 16 QAM | Outer_Full |
| 11 | 907.5 | 15 |  |  |  | 16 QAM | Edge_1RB_Left |
| 12 | 907.5 | 15 |  |  |  | 16 QAM | Outer_Full |
| 13 | 902.5 | 5 |  |  |  | 64 QAM | Outer_Full |
| 14 | 910 | 10 |  |  |  | 64 QAM | Outer_Full |
| 15 | 907.5 | 15 |  |  |  | 64 QAM | Edge_1RB_Left |
| 16 | 907.5 | 15 |  |  |  | 64 QAM | Outer_Full |
| 17 | 902.5 | 5 |  |  |  | 256 QAM | Outer_Full |
| 18 | 910 | 10 |  |  |  | 256 QAM | Outer_Full |
| 19 | 907.5 | 15 |  |  |  | 256 QAM | Edge_1RB_Left |
| 20 | 907.5 | 15 |  |  |  | 256 QAM | Outer_Full |
| 21 | 902.5 | 5 |  |  | CP-OFDM | QPSK | Outer_Full |
| 22 | 910 | 10 |  |  |  | QPSK | Outer_Full |
| 23 | 907.5 | 15 |  |  |  | QPSK | Edge_1RB_Left |
| 24 | 907.5 | 15 |  |  |  | QPSK | Outer_Full |
| 25 | 902.5 | 5 |  |  |  | 16 QAM | Outer_Full |
| 26 | 910 | 10 |  |  |  | 16 QAM | Outer_Full |
| 27 | 907.5 | 15 |  |  |  | 16 QAM | Edge_1RB_Left |
| 28 | 907.5 | 15 |  |  |  | 16 QAM | Outer_Full |
| 29 | 902.5 | 5 |  |  |  | 64 QAM | Outer_Full |
| 30 | 910 | 10 |  |  |  | 64 QAM | Outer_Full |
| 31 | 907.5 | 15 |  |  |  | 64 QAM | Edge_1RB_Left |
| 32 | 907.5 | 15 |  |  |  | 64 QAM | Outer_Full |
| 33 | 902.5 | 5 |  |  |  | 256 QAM | Outer_Full |
| 34 | 910 | 10 |  |  |  | 256 QAM | Outer_Full |
| 35 | 907.5 | 15 |  |  |  | 256 QAM | Edge_1RB_Left |
| 36 | 907.5 | 15 |  |  |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |

Editor’s note: The following lines belong at the end of subclause 6.2.3.4.1. As new tables are added to this section, these lines should always follow the tables

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.4 for TE diagram and section A.3.2 for UE diagram.

-	The parameter setting for the cell are set up according to the TS 38.508-1 [5] subclause 4.4.3.

-	Downlink signals are initially setup according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 with consideration of supplementary uplink physical channels.

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Tables 4.6.3-14 with condition SUL_SUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL. All the AdditionalSpectrumEmission in 6.2.3.4.3 are sent in SIB1 as part of supplementaryUplink instead of uplinkConfigCommon.

Table 6.2C.5.4-1: Void

6.2C.5.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2C.5.5-1 to table 6.2C.5.5-10. The allowed A-MPR values specified in table 6.2.3.3-1 are in addition to the allowed MPR requirements specified in clause 6.2C.4. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2.1.3-1 apply.

Table 6.2C.5.5-1: UE Power Class 3 test requirements (NS_03 and NS_03U) for band n86

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 4, 5 | NS_03, NS_03U | 23 | 1 | 1 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7 | NS_03, NS_03U | 23 | 1 | 1 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7, 8 | NS_03, NS_03U | 23 | 2 | 2.5 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 9 | NS_03, NS_03U | 23 | 2 | 2.5 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 10, 11 | NS_03, NS_03U | 23 | 2.5 | 3 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 12 | NS_03, NS_03U | 23 | 2.5 | 3 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 13, 14 | NS_03, NS_03U | 23 | 4.5 | 4.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 15 | NS_03, NS_03U | 23 | 4.5 | 4.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 16, 17 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19, 20 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 21 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22, 23 | NS_03, NS_03U | 23 | 3.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 24 | NS_03, NS_03U | 23 | 3.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 25, 26 | NS_03, NS_03U | 23 | 6.5 | 6.5 | 0 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| 27 | NS_03, NS_03U | 23 | 6.5 | 6.5 | 0 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2C.5.5-2: UE Power Class 3 test requirements (NS_18) for band n83

| Test ID | ChBw (MHz) | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 5 | 23 | 1 | 2 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
|  | 10, 15, 20 | 23 | 1 | 5 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
| 3, 4 | 5 | 23 | 2 | 3 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
|  | 10, 15, 20 | 23 | 2 | 6 | 0 | 17 | 5 | 2.5 | 25+TT | 12-TT |
| 5, 6 | 5 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 2.5 | 25+TT | 15.5-TT |
|  | 10, 15, 20 | 23 | 2.5 | 7 | 0 | 16 | 5 | 2.5 | 25+TT | 11-TT |
| 7, 8 | 5 | 23 | 4.5 | 6 | 0 | 17 | 5 | 2.5 | 25+TT | 12-TT |
|  | 10, 15, 20 | 23 | 4.5 | 9 | 0 | 14 | 5 | 2.5 | 25+TT | 9-TT |
| 9, 10 | 5 | 23 | 3 | 5 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
|  | 10, 15, 20 | 23 | 3 | 6.5 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 11, 12 | 5 | 23 | 3 | 5 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
|  | 10, 15, 20 | 23 | 3 | 7 | 0 | 16 | 5 | 2.5 | 25+TT | 11-TT |
| 13, 14 | 5 | 23 | 3.5 | 5.5 | 0 | 17.5 | 5 | 2.5 | 25+TT | 12.5-TT |
|  | 10, 15, 20 | 23 | 3.5 | 8.5 | 0 | 14.5 | 5 | 2.5 | 25+TT | 9.5-TT |
| 15, 16 | 5 | 23 | 6.5 | 8.5 | 0 | 14.5 | 5 | 2.5 | 25+TT | 9.5-TT |
|  | 10, 15, 20 | 23 | 6.5 | 11.5 | 0 | 11.5 | 6 | 2.5 | 25+TT | 5.5-TT |
| 17 | 30 | 23 | 5 | 3 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
| 18 | 30 | 23 | 5 | 8 | 0 | 15 | 5 | 2.5 | 25+TT | 10–TT |
| 19 | 30 | 23 | 7 | 4.5 | 0 | 16 | 5 | 2.5 | 25+TT | 11–TT |
| 20 | 30 | 23 | 7 | 9.5 | 0 | 13.5 | 5 | 2.5 | 25+TT | 8.5–TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2C.5.5-3: UE Power Class 3 test requirements (NS_48) for band n84

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 2 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 3 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 4 | 23 | 0 | 1 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 6 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 7 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 8 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 9 | 23 | 0 | 0 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 10 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 11 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 12 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 13 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 16 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 17 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 18 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 19 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 20 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 21 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 22 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 23 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 24 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 25 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 26 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 27 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 28 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 30 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 31 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 32 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 33 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 34 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 35 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 36 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 37 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 38 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 39 | 23 | 0 | 1.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 40 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 41 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 42 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 43 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 44 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 45 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 46 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 47 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 48 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 49 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 51 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 52 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 53 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 54 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 55 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 56 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 57 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 58 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 59 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 60 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2C.5.5-4: UE Power Class 3 test requirements (NS_49) for band n84

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 2 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 3 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 4 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 5 | 23 | 0 | 1 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 6 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 7 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 8 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 9 | 23 | 0 | 1 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 10 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 11 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 12 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 13 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 16 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 17 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 18 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 19 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 20 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 21 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 22 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 23 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 24 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 25 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 26 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 27 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 28 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 29 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 30 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 31 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 32 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 33 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 34 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 35 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 36 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 37 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 38 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 39 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 40 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 41 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 42 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 43 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 44 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 45 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 46 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 47 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 48 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 49 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 50 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 51 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 52 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 53 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 54 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 55 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 56 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 57 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 58 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 59 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 61 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 62 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 63 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 64 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 65 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 66 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 67 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 68 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 69 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 70 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 71 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 72 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 73 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 74 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 75 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 76 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 77 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 78 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 79 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 80 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 81 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 82 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 83 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 84 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2C.5.5-5: UE Power Class 3 test requirements (NS_05) for band n84

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 2 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 3 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 0.5 | 1 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 6 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 7 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 9 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 10 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 11 | 23 | 0 | 0.5 | 1 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 12 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 13 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 0.5 | 1 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 16 | 23 | 0 | 1.0 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 17 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 18 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 20 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 21 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 22 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 23 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 24 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 25 | 23 | 0 | 1.0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 26 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 27 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 28 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 29 | 23 | 0 | 1.0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 30 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 31 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 32 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 33 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 34 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 35 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 36 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 37 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 38 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 39 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 40 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 41 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 42 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 43 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 44 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 45 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 46 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 47 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 48 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 49 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 50 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 51 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 52 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 53 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 54 | 23 | 0 | 4.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 55 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 56 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 57 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 58 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 59 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 61 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 62 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 63 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 64 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 65 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 66 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 67 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 68 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 69 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 70 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 71 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 72 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 73 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 74 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 75 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 76 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 77 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 78 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 79 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 80 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 81 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 82 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 83 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 84 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 85 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 86 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 87 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 88 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 89 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 90 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 91 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 92 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 93 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 94 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 95 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 96 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 97 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 98 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 99 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 100 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 101 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 102 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 103 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 104 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 105 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 106 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 107 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 108 | 23 | 0 | 6.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 109 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 110 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 111 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 112 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 113 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 114 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 115 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2C.5.5-6: UE Power Class 3 test requirements (NS_05U) for band n84

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 2 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 3 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 6 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 7 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 9 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 10 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 11 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 12 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 13 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 16 | 23 | 0 | 1.0 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 17 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 18 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 20 | 23 | 0 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 21 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 22 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 23 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 24 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 25 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 26 | 23 | 0 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 27 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 28 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 30 | 23 | 0 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 31 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 32 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 33 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 34 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 35 | 23 | 0 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 36 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 37 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 38 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 39 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 40 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 41 | 23 | 0 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 42 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 43 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 44 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 45 | 23 | 0 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 46 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 47 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 48 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 49 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 50 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 51 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 52 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 53 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 54 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 55 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 56 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 57 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 58 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 59 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 61 | 23 | 0 | 4.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 62 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 63 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 64 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 65 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 66 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 67 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 68 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 69 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 70 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 71 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 72 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 73 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 74 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 75 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 76 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 77 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 78 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 79 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 80 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 81 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 82 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 83 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 84 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 85 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 86 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 87 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 88 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 89 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 90 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 91 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 92 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 93 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 94 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 95 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 96 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 97 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 98 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 99 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 100 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 101 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 102 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 103 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 104 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 105 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 106 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 107 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 108 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 109 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 110 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 111 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 112 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 113 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 114 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 115 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 116 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 117 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 118 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 119 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 120 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 121 | 23 | 0 | 6.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 122 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 123 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 124 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 125 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 126 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 127 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 128 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 129 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 130 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 131 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2C.5.5-7: UE Power Class 3 test requirements (NS_100) for band n84 and n86

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 4, 5 | 23 | 1 | 1 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 6 | 23 | 1 | 1 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7, 8 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 9 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 10, 11 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 12 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 13, 14 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 15 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 16, 17 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19, 20 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 21 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22, 23 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 24 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 25, 26 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 27 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2C.5.5-8: UE Power Class 3 test requirements (NS_56) for band n99

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 2 | 23 | 0.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 3 | 23 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 4 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 5 | 23 | 1 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 6 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 7 | 23 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 8 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 9 | 23 | 2 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 10 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 11 | 23 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 12 | 23 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 13 | 23 | 2.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 14 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 15 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 16 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 17 | 23 | 4.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 18 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 19 | 23 | 4.5 | 4 | 0 | 18.5 | 3.5 | 3 | 25+TT | 15-TT |
| 20 | 23 | 4.5 | 4 | 0 | 18.5 | 3.5 | 3 | 25+TT | 15-TT |
| 21 | 23 | 0.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 22 | 23 | 0.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 23 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 24 | 23 | 0.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 25 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 26 | 23 | 0.5 | 2 | 0 | 21 | 2 | 3 | 25+TT | 18-TT |
| 27 | 23 | 0.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 28 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 29 | 23 | 0.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 30 | 23 | 1 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 31 | 23 | 1 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 32 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 33 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 34 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 35 | 23 | 1 | 2 | 0 | 21 | 2 | 3 | 25+TT | 18-TT |
| 36 | 23 | 1 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 37 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 38 | 23 | 1 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 39 | 23 | 2 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 40 | 23 | 2 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 41 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 42 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 43 | 23 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 44 | 23 | 2 | 2 | 0 | 21 | 2 | 3 | 25+TT | 18-TT |
| 45 | 23 | 2 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 46 | 23 | 1 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 47 | 23 | 2 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 48 | 23 | 2.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 49 | 23 | 2.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 50 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 51 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 52 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 53 | 23 | 2.5 | 2 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 54 | 23 | 2.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 55 | 23 | 2.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 56 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 57 | 23 | 4.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58 | 23 | 4.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 59 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 60 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 61 | 23 | 4.5 | 4 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 62 | 23 | 4.5 | 2 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 63 | 23 | 4.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 64 | 23 | 4.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 65 | 23 | 4.5 | 3 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 66 | 23 | 3 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 67 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 68 | 23 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 69 | 23 | 1.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 70 | 23 | 3 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 71 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 72 | 23 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 73 | 23 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 74 | 23 | 3.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 75 | 23 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 76 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 77 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 78 | 23 | 6.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 79 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 80 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 81 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 82 | 23 | 3 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 83 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 84 | 23 | 1.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 85 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 86 | 23 | 1.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 87 | 23 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 88 | 23 | 3 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 89 | 23 | 1.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 90 | 23 | 3 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 91 | 23 | 3 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 92 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 93 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 94 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 95 | 23 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 96 | 23 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 97 | 23 | 3 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 98 | 23 | 2 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 99 | 23 | 3 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 100 | 23 | 3.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 101 | 23 | 3.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 102 | 23 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 103 | 23 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 104 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 105 | 23 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 106 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 3 | 25+TT | 14.5-TT |
| 107 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 3 | 25+TT | 14.5-TT |
| 108 | 23 | 3.5 | 3 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 109 | 23 | 6.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 110 | 23 | 6.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 111 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 112 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 113 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 114 | 23 | 6.5 | 2 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 115 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 116 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 117 | 23 | 6.5 | 3 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2C.5.5-9: UE Power Class 3 test requirements (NS_43) for band n81

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 2 | 23 | 0.5 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 3 | 23 | 0.5 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 4 | 23 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 5 | 23 | 1.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 6 | 23 | 1.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 7 | 23 | 1.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 8 | 23 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 9 | 23 | 2.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 10 | 23 | 2.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 11 | 23 | 2.5 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 12 | 23 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 13 | 23 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 14 | 23 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 15 | 23 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 16 | 23 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 17 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 19 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 20 | 23 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 21 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 23 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 24 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 25 | 23 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 26 | 23 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 27 | 23 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 28 | 23 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2C.5.5-10: UE Power Class 3 test requirements (NS_43U) for band n81

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 2 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3 | 23 | 0.5 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 4 | 23 | 0.5 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 5 | 23 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 6 | 23 | 1.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 7 | 23 | 1.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 8 | 23 | 1.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 9 | 23 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 10 | 23 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 11 | 23 | 2.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 12 | 23 | 2.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 13 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 14 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 15 | 23 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 16 | 23 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 17 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 18 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 19 | 23 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 20 | 23 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 21 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 23 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 24 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 25 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 26 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 27 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 28 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 29 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 30 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 31 | 23 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 32 | 23 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 33 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 34 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 35 | 23 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 36 | 23 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.