3GPP TS 29.522 V19.0.0 (2024-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

5G System; Network Exposure Function Northbound APIs;

Stage 3

(Release 19)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	32

1	Scope	33

2	References	33

3	Definitions and abbreviations	35

3.1	Definitions	35

3.2	Abbreviations	36

4	NEF Northbound Interface	37

4.1	Overview	37

4.2	Reference model	40

4.3	Functional elements	40

4.3.1	NEF	40

4.3.2	AF	41

4.4	Procedures over NEF Northbound Interface	41

4.4.1	Introduction	41

4.4.2	Procedures for Monitoring	41

4.4.3	Procedures for Device Triggering	49

4.4.4	Procedures for resource management of Background Data Transfer	50

4.4.5	Procedures for CP Parameters Provisioning	50

4.4.6	Procedures for PFD Management	52

4.4.7	Procedures for Traffic Influence	52

4.4.7.1	General	52

4.4.7.2	AF request identified by UE address	53

4.4.7.3	AF request not identified by UE address	54

4.4.7.4	Handling of UP path management event notification	54

4.4.7.5	Processing AF requests to influence traffic routing for HR-SBO session	55

4.4.8	Procedures for changing the chargeable party at session set up or during the session	56

4.4.9	Procedures for AF required QoS	57

4.4.9.1	General	57

4.4.9.2	Procedures for AF setting up an AF session with required QoS for target UE identified by UE address or for target list of UEs identified by list of UE addresses	57

4.4.9.3	Procedures for AF requested QoS for a target UE or group of UE(s) not identified by UE address(es)	67

4.4.10	Procedures for MSISDN-less Mobile Originated SMS	67

4.4.11	Procedures for Network Configuration Parameters Provisioning	68

4.4.12	Procedures for Non-IP data delivery	69

4.4.12.1	General	69

4.4.12.2	NIDD configuration Triggered by the NEF	69

4.4.12.3	NIDD configuration triggered by the AF and NIDD delivery	69

4.4.13	Procedures for RACS Parameter Provisioning	69

4.4.14	Procedures for analytics information exposure	70

4.4.14.1	Subscription/unsubscription to notification of analytics information	70

4.4.14.2	Fetch analytics information	72

4.4.15	Procedures for 5G LAN Parameter Provisioning	72

4.4.15.1	General	72

4.4.15.2	Creation of a new subscription for 5G LAN parameter provisioning	73

4.4.15.3	Modification of an existing subscription for 5G LAN parameter provisioning	73

4.4.15.4	Deletion of an existing subscription for 5G LAN parameter provisioning	73

4.4.15.5	5G LAN parameter provisioning event notification	73

4.4.16	Procedures for applying BDT policy	74

4.4.17	Procedures for Enhanced Coverage Restriction Control	74

4.4.18	Procedures for IPTV Configuration	75

4.4.20	Procedures for service specific parameter provisioning	76

4.4.21	Procedures for ACS configuration parameter provisioning	79

4.4.22	Procedures for Mobile Originated Location Request	80

4.4.22.1	General	80

4.4.22.2	Location Update Notification triggered by UE	80

4.4.23	Procedures for AKMA	80

4.4.23.1	General	80

4.4.23.2	AKMA Application Key Request	80

4.4.23.3	AKMA Service Disablement Notification	81

4.4.24	Procedures for Time Synchronization Exposure	81

4.4.24.0	General	81

4.4.24.1	Subscription and unsubscription to notification of Time Synchronization Capabilites	81

4.4.24.2	Time Synchronization Exposure Configuration	83

4.4.24.3	Management of 5G access stratum time distribution	84

4.4.25	Procedures for ECS address Provisioning	85

4.4.26	Procedures for AM Policy Authorization	86

4.4.26.1	General	86

4.4.26.2	Creation of a new Individual Application AM Context	86

4.4.26.3	Modification of an existing individual Application AM Context	87

4.4.26.4	Deletion of an existing individual Application AM Context	87

4.4.26.5	Create or modify subscription to notification of AM policy event	87

4.4.26.6	Unsubscription to notification of AM policy event	88

4.4.26.7	Notification of AM policy event	88

4.4.27	Procedures for AF triggered Access and Mobility Influence	88

4.4.27.1	General	88

4.4.27.2	Create the AM Influence Subscription	88

4.4.27.3	Modifiy the AM Influence Subscription	89

4.4.27.4	Delete the AM Influence Subscription	89

4.4.27.5	Notification of service area coverage outcome events	89

4.4.28	Procedures for Northbound EAS Deployment Information management	89

4.4.28.1	General	89

4.4.28.2	Creation of a new Individual EAS Deployment information resource	90

4.4.28.3	Modification of an existing individual EAS Deployment Information resource	90

4.4.28.4	Deletion of an existing individual EAS Deployment Information resource	91

4.4.28.5	Deletion of EAS Deplyoment Information based on given criteria	91

4.4.29	Procedures for MBS Management	91

4.4.29.1	General	91

4.4.29.2	Procedures for MBS TMGI management	92

4.4.29.2.1	General	92

4.4.29.2.2	Procedure for MBS TMGI(s) allocation or MBS TMGI(s) expiry time refresh	92

4.4.29.2.3	Procedure for MBS TMGI(s) deallocation	93

4.4.29.2.4	Procedure for MBS TMGI(s) timer expiry notification	93

4.4.29.3	Procedures for MBS session management	94

4.4.29.3.1	General	94

4.4.29.3.2	Procedure for MBS session creation	94

4.4.29.3.3	Procedure for MBS session update	96

4.4.29.3.4	Procedure for MBS session deletion	97

4.4.29.3.5	Procedure for MBS session status subscription	97

4.4.29.3.6	Procedure for MBS session status unsubscription	98

4.4.29.3.7	Procedure for MBS session status notification	98

4.4.29.4	Procedures for MBS Parameters Provisioning	99

4.4.29.4.1	General	99

4.4.29.4.2	Procedure for multicast MBS Session Authorization information provisioning	99

4.4.29.4.3	Procedure for multicast MBS Session Assistance information provisioning	100

4.4.29.5	Procedures for MBS User Service management	101

4.4.29.5.1	General	101

4.4.29.5.2	Procedure for MBS User Service creation	101

4.4.29.5.3	Procedure for MBS User Service retrieval	101

4.4.29.5.4	Procedure for MBS User Service update/modification	101

4.4.29.5.5	Procedure for MBS User Service deletion	102

4.4.29.6	Procedures for MBS User Data Ingest Session management	102

4.4.29.6.1	General	102

4.4.29.6.2	Procedure for MBS User Data Ingest Session creation	102

4.4.29.6.3	Procedure for MBS User Data Ingest Session retrieval	103

4.4.29.6.4	Procedure for MBS User Data Ingest Session update/modification	103

4.4.29.6.5	Procedure for MBS User Data Ingest Session deletion	104

4.4.29.6.6	Procedure for MBS User Data Ingest Session Status Subscription	104

4.4.29.6.7	Procedure for MBS User Data Ingest Session Status update/modification	104

4.4.29.6.8	Procedure for MBS User Data Ingest Session Status Unsubscription	105

4.4.29.6.9	Procedure for MBS User Data Ingest Session Status Notification	105

4.4.29.7	Procedures for MBS Group Message Delivery Management	106

4.4.29.7.1	General	106

4.4.29.7.2	Procedure for MBS Group Message Delivery Creation	106

4.4.29.7.3	Procedure for MBS Group Message Delivery Update	106

4.4.29.7.4	Procedure for MBS Group Message Delivery Deletion	107

4.4.29.7.5	Procedure for MBS Group Message Delivery Status Notification	107

4.4.30	Procedures for Data Reporting	107

4.4.30.1	General	107

4.4.30.2	Procedure for Data Reporting Session Management	108

4.4.30.3	Procedure for Data Report	108

4.4.31	Procedures for Data Reporting Provisioning	108

4.4.31.1	General	108

4.4.31.2	Procedure for Data Reporting Provisioning Session Management	108

4.4.31.3	Procedure for Data Reporting Configuration management	109

4.4.32	Procedures for UE ID retrieval	110

4.4.32.1	General	110

4.4.32.2	Retrieve AF specific UE ID service operation	110

4.4.32.3	Retrieve UE ID in the form of MSISDN	111

4.4.32.4	UE ID Mapping Information Provisioning	112

4.4.33	Procedures for Media Streaming Event Exposure	113

4.4.33.1	General	113

4.4.33.2	Procedure for Media Streaming Event Exposure Subscription Creation	113

4.4.33.3	Procedure for Media Streaming Event Exposure Subscription Update	114

4.4.33.4	Procedure for Media Streaming Event Exposure Unsubscription	114

4.4.33.5	Procedure for Media Streaming Event Exposure Notification	114

4.4.34	Procedures for DNAI Mapping	115

4.4.34.1	General	115

4.4.34.2	Creation of a new subscription for DNAI Mapping	115

4.4.34.3	Deletion of an existing individual DNAI Mapping subscription	115

4.4.34.4	Notification for updated DNAI Mapping information	116

4.4.35	Procedures for negotiation of planned data transfer with QoS requirements	116

4.4.36	Procedures for Member UE Selection Assistance	117

4.4.36.1	General	117

4.4.36.2	Procedure for Member UE Selection Assistance Subscription Creation	117

4.4.36.3	Procedure for Member UE Selection Assistance Subscription Update	117

4.4.36.4	Procedure for Member UE Selection Assistance Subscription Unsubscription	117

4.4.36.5	Procedure for Member UE Selection Assistance Notification	118

4.4.37	Procedures for Group Parameters Provisioning	118

4.4.37.1	General	118

4.4.37.2	Procedures for DNN and S-NSSAI specific Group Parameters Provisioning	118

4.4.38	Procedures for Network Slice Parameters Provisioning	119

4.4.38.1	General	119

4.4.38.2	Procedures for Network Slice Usage Control Parameters Provisioning	120

4.4.39	Procedures for UE Address Retrieval	120

4.4.39.1	General	120

4.4.39.2	Procedures for UE Address Retrieval	121

4.4.40	Procedures for ECS Address Configuration Information provisioning in roaming	121

4.4.40.1	General	121

4.4.40.2	Creation of new ECS Address Configuration Information	121

4.4.40.3	Modification of existing ECS Address Configuration Information	122

4.4.40.4	Deletion of existing Individual ECS Address Configuration Information	122

4.4.40.5	Deletion of ECS Address Configuration Information based on given criteria	123

4.4.41.1	General	123

4.4.41.2	Procedures for RSLPPI Parameters Provisioning	123

5	NEF Northbound APIs	124

5.1	Introduction	124

5.2	Information applicable to several APIs	126

5.3	Reused APIs	126

5.4	TrafficInfluence API	127

5.4.0	Introduction	127

5.4.1	Resources	128

5.4.1.1	Overview	128

5.4.1.2	Resource: Traffic Influence Subscription	128

5.4.1.2.1	Introduction	128

5.4.1.2.2	Resource Definition	128

5.4.1.2.3	Resource Methods	129

5.4.1.2.3.1	General	129

5.4.1.2.3.2	GET	129

5.4.1.2.3.3	POST	130

5.4.1.3	Resource: Individual Traffic Influence Subscription	130

5.4.1.3.1	Introduction	130

5.4.1.3.2	Resource Definition	130

5.4.1.3.3	Resource Methods	131

5.4.1.3.3.1	General	131

5.4.1.3.3.2	GET	131

5.4.1.3.3.3	PUT	132

5.4.1.3.3.4	PATCH	132

5.4.1.3.3.5	DELETE	133

5.4.1A	Custom Operations without associated resources	134

5.4.2	Notifications	134

5.4.2.1	Introduction	134

5.4.2.2	Event Notification	135

5.4.2.2.1	Description	135

5.4.2.2.2	Target URI	135

5.4.2.2.3	Operation Definition	135

5.4.2.3	Acknowledgement of event notification	136

5.4.2.3.1	Description	136

5.4.2.3.2	Target URI	136

5.4.2.3.3	Operation Definition	137

5.4.2.3.3.1	Notification via HTTP POST	137

5.4.3	Data Model	137

5.4.3.1	General	137

5.4.3.2	Reused data types	138

5.4.3.3	Structured data types	139

5.4.3.3.1	Introduction	139

5.4.3.3.2	Type: TrafficInfluSub	139

5.4.3.3.3	Type: TrafficInfluSubPatch	144

5.4.3.3.4	Type: EventNotification	148

5.4.3.3.5	Type: AfResultInfo	150

5.4.3.3.6	Type AfAckInfo	150

5.4.3.3.7	Type TrafficDataSet	150

5.4.3.3.8	Type TrafficDataSetRm	151

5.4.3.4	Simple data types and enumerations	151

5.4.3.4.1	Introduction	151

5.4.3.4.2	Simple data types	151

5.4.3.4.3	Enumeration: SubscribedEvent	151

5.4.3.4.4	Enumeration: AfResultStatus	151

5.4.4	Used Features	152

5.4.5	Error handling	152

5.4.5.1	General	152

5.4.5.2	Protocol Errors	153

5.4.5.3	Application Errors	153

5.5	NiddConfigurationTrigger API	153

5.5.0	Introduction	153

5.5.1	Resources	153

5.5.1A	Custom Operations without associated resources	153

5.5.2	Notifications	153

5.5.2.1	Introduction	153

5.5.2.2	Event Notification	154

5.5.2.3	Operation Definition	154

5.5.2.3.1	Notification via HTTP POST	154

5.5.2.3.2	Notification via Websocket	155

5.5.3	Data Model	155

5.5.3.1	General	155

5.5.3.2	Reused data types	155

5.5.3.3	Structured data types	155

5.5.3.3.1	Introduction	155

5.5.3.3.2	Type: NiddConfigurationTrigger	155

5.5.3.3.3	Type: NiddConfigurationTriggerReply	155

5.5.3.4	Simple data types and enumerations	156

5.5.3.4.1	Introduction	156

5.5.3.4.2	Simple data types	156

5.5.4	Used Features	156

5.5.5	Error handling	156

5.5.5.1	General	156

5.5.5.2	Protocol Errors	156

5.5.5.3	Application Errors	156

5.6	AnalyticsExposure API	157

5.6.1	Resources	157

5.6.0	Introduction	157

5.6.1.1	Overview	157

5.6.1.2	Resource: Analytics Exposure Subscriptions	158

5.6.1.2.1	Introduction	158

5.6.1.2.2	Resource Definition	158

5.6.1.2.3	Resource Methods	158

5.6.1.2.3.1	General	158

5.6.1.2.3.2	GET	158

5.6.1.2.3.3	POST	159

5.6.1.3	Resource: Individual Analytics Exposure Subscription	160

5.6.1.3.1	Introduction	160

5.6.1.3.2	Resource Definition	160

5.6.1.3.3	Resource Methods	160

5.6.1.3.3.1	General	160

5.6.1.3.3.2	GET	160

5.6.1.3.3.3	PUT	161

5.6.1.3.3.4	DELETE	162

5.6.1A	Custom Operations without associated resources	163

5.6.1A.1	Overview	163

5.6.1A.2	Operation: fetch	164

5.6.1A.2.1	Description	164

5.6.1A.2.2	Operation Definition	164

5.6.2	Notifications	165

5.6.2.1	Introduction	165

5.6.2.2	Event Notification	165

5.6.2.3	Operation Definition	165

5.6.2.3.1	Notification via HTTP POST	165

5.6.2.3.2	Notification via Websocket	166

5.6.3	Data Model	166

5.6.3.1	General	166

5.6.3.2	Reused data types	167

5.6.3.3	Structured data types	170

5.6.3.3.1	Introduction	170

5.6.3.3.2	Type: AnalyticsExposureSubsc	170

5.6.3.3.3	Type: AnalyticsEventNotification	172

5.6.3.3.4	Type: AnalyticsEventNotif	173

5.6.3.3.5	Type: AnalyticsEventSubsc	175

5.6.3.3.6	Type: AnalyticsEventFilterSubsc	176

5.6.3.3.7	Type TargetUeId	181

5.6.3.3.8	Void	182

5.6.3.3.9	Type UeMobilityExposure	182

5.6.3.3.10	Type UeLocationInfo	183

5.6.3.3.11	Void	183

5.6.3.3.12	Type: AnalyticsRequest	183

5.6.3.3.13	Type AnalyticsEventFilter	184

5.6.3.3.14	Type AnalyticsData	188

5.6.3.3.15	Type AbnormalExposure	190

5.6.3.3.16	Type CongestInfo	190

5.6.3.3.17	Type CongestionAnalytics	191

5.6.3.3.18	Type QosSustainabilityExposure	192

5.6.3.3.19	Type NetworkPerfExposure	193

5.6.3.3.20	Type AnalyticsFailureEventInfo	193

5.6.3.3.21	Type WlanPerformInfo	194

5.6.3.4	Simple data types and enumerations	194

5.6.3.4.1	Introduction	194

5.6.3.4.2	Simple data types	194

5.6.3.4.3	Enumeration: AnalyticsEvent	194

5.6.3.4.4	Enumeration: AnalyticsFailureCode	195

5.6.4	Used Features	196

5.6.5	Error handling	198

5.6.5.1	General	198

5.6.5.2	Protocol Errors	198

5.6.5.3	Application Errors	199

5.7	5GLANParameterProvision API	199

5.7.0	Introduction	199

5.7.1	Resources	199

5.7.1.1	Overview	199

5.7.1.2	Resource: 5GLAN Parameters Provision Subscriptions	200

5.7.1.2.1	Introduction	200

5.7.1.2.2	Resource Definition	200

5.7.1.2.3	Resource Methods	200

5.7.1.2.3.1	General	200

5.7.1.2.3.2	GET	201

5.7.1.2.3.3	POST	201

5.7.1.3	Resource: Individual 5GLAN Parameters Provision Subscription	202

5.7.1.3.1	Introduction	202

5.7.1.3.2	Resource Definition	202

5.7.1.3.3	Resource Methods	202

5.7.1.3.3.1	General	202

5.7.1.3.3.2	GET	202

5.7.1.3.3.3	PUT	203

5.7.1.3.3.4	DELETE	204

5.7.1.3.3.5	PATCH	205

5.7.1A	Custom Operations without associated resources	206

5.7.1B	Notifications	206

5.7.1B.1	General	206

5.7.1B.2	5G LAN Parameter Provisioning Event Notification	206

5.7.1B.2.1	Description	206

5.7.1B.2.2	Target URI	206

5.7.1B.2.3	Operation Definition	207

5.7.1B.2.3.1	Notification via HTTP POST	207

5.7.1B.2.3.2	Void	207

5.7.2	Data Model	207

5.7.2.1	General	207

5.7.2.2	Reused data types	208

5.7.2.3	Structured data types	209

5.7.2.3.1	Introduction	209

5.7.2.3.2	Type: 5GLanParametersProvision	209

5.7.2.3.3	Type: 5GLanParameters	209

5.7.2.3.4	Type: AppDescriptor	211

5.7.2.3.5	Type: 5GLanParametersProvisionPatch	212

5.7.2.3.6	Type: 5GLanParametersPatch	212

5.7.2.3.7	Type: AppDescriptorRm	213

5.7.2.3.8	Void	213

5.7.2.3.9	Type: CpParams	213

5.7.2.3.10	Type: NpConfigParams	214

5.7.2.3.11	Type: LpiParams	214

5.7.2.3.12	Type: AcsParams	214

5.7.2.3.13	Type: ECSAddrParams	214

5.7.2.3.14	Type: DnnSnssaiParams	215

5.7.2.3.15	Type: 5GLanParamProvNotif	215

5.7.2.3.16	Type: NpConfigNotif	215

5.7.2.3.17	Type: MaxGrpDataRateInfo	215

5.7.2.4	Simple data types and enumerations	216

5.7.2.4.1	Introduction	216

5.7.2.4.2	Simple data types	216

5.7.2.4.3	Enumeration: AaaUsage	216

5.7.3	Used Features	216

5.7.4	Error handling	216

5.7.4.1	General	216

5.7.4.2	Protocol Errors	217

5.7.4.3	Application Errors	217

5.8	ApplyingBdtPolicy API	217

5.8.0	Introduction	217

5.8.1	Resources	217

5.8.1.1	Overview	217

5.8.1.2	Resource: Applied BDT Policy Subscriptions	218

5.8.1.2.1	Introduction	218

5.8.1.2.2	Resource Definition	218

5.8.1.2.3	Resource Methods	218

5.8.1.2.3.1	General	218

5.8.1.2.3.2	GET	218

5.8.1.2.3.3	POST	219

5.8.1.3	Resource: Individual Applied BDT Policy Subscription	220

5.8.1.3.1	Introduction	220

5.8.1.3.2	Resource Definition	220

5.8.1.3.3	Resource Methods	220

5.8.1.3.3.1	General	220

5.8.1.3.3.2	GET	220

5.8.1.3.3.3	PATCH	221

5.8.1.3.3.4	DELETE	222

5.8.1A	Custom Operations without associated resources	223

5.8.2	Notifications	223

5.8.3	Data Model	223

5.8.3.1	General	223

5.8.3.2	Reused data types	223

5.8.3.3	Structured data types	224

5.8.3.3.1	Introduction	224

5.8.3.3.2	Type: AppliedBdtPolicy	224

5.8.3.3.3	Type: AppliedBdtPolicyPatch	224

5.8.3.4	Simple data types and enumerations	224

5.8.3.4.1	Introduction	224

5.8.3.4.2	Simple data types	225

5.8.4	Used Features	225

5.8.5	Error handling	225

5.8.5.1	General	225

5.8.5.2	Protocol Errors	225

5.8.5.3	Application Errors	225

5.9	IPTVConfiguration API	225

5.9.0	Introduction	225

5.9.1	Resources	226

5.9.1.1	Overview	226

5.9.1.2	Resource: IPTV Configurations	226

5.9.1.2.1	Introduction	226

5.9.1.2.2	Resource Definition	226

5.9.1.2.3	Resource Methods	227

5.9.1.2.3.1	General	227

5.9.1.2.3.2	GET	227

5.9.1.2.3.3	POST	228

5.9.1.3	Resource: Individual IPTV Configuration	228

5.9.1.3.1	Introduction	228

5.9.1.3.2	Resource Definition	228

5.9.1.3.3	Resource Methods	229

5.9.1.3.3.1	General	229

5.9.1.3.3.2	GET	229

5.9.1.3.3.3	PUT	230

5.9.1.3.3.4	DELETE	230

5.9.1.3.3.5	PATCH	231

5.9.1A	Custom Operations without associated resources	232

5.9.1B	Notifications	232

5.9.2	Data Model	232

5.9.2.1	General	232

5.9.2.2	Reused data types	233

5.9.2.3	Structured data types	233

5.9.2.3.1	Introduction	233

5.9.2.3.2	Type: IptvConfigData	234

5.9.2.3.3	Type: MulticastAccessControl	234

5.9.2.3.4	Type: IptvConfigDataPatch	235

5.9.2.4	Simple data types and enumerations	235

5.9.2.4.1	Introduction	235

5.9.2.4.2	Simple data types	235

5.9.2.4.3	Enumeration: AccessRightStatus	235

5.9.3	Used Features	235

5.9.4	Error handling	235

5.9.4.1	General	235

5.9.4.2	Protocol Errors	236

5.9.4.3	Application Errors	236

5.10	LpiParameterProvision API	236

5.10.0	Introduction	236

5.10.1	Resources	236

5.10.1.1	Overview	236

5.10.1.2	Resource: LPI Parameters Provisionings	237

5.10.1.2.1	Introduction	237

5.10.1.2.2	Resource Definition	237

5.10.1.2.3	Resource Methods	237

5.10.1.2.3.1	General	237

5.10.1.2.3.2	GET	237

5.10.1.2.3.3	POST	238

5.10.1.3	Resource: Individual LPI Parameters Provisioning	239

5.10.1.3.1	Introduction	239

5.10.1.3.2	Resource Definition	239

5.10.1.3.3	Resource Methods	239

5.10.1.3.3.1	General	239

5.10.1.3.3.2	GET	239

5.10.1.3.3.3	PUT	240

5.10.1.3.3.3A	PATCH	241

5.10.1.3.3.4	DELETE	242

5.10.1A	Custom Operations without associated resources	243

5.10.1B	Notifications	243

5.10.2	Data Model	243

5.10.2.1	General	243

5.10.2.2	Reused data types	243

5.10.2.3	Structured data types	244

5.10.2.3.1	Introduction	244

5.10.2.3.2	Type: LpiParametersProvision	244

5.10.2.3.3	Type: LpiParametersProvisionPatch	245

5.10.2.4	Simple data types and enumerations	245

5.10.2.4.1	Introduction	245

5.10.2.4.2	Simple data types	245

5.10.3	Used Features	245

5.10.4	Error handling	245

5.10.4.1	General	245

5.10.4.2	Protocol Errors	245

5.10.4.3	Application Errors	246

5.11	ServiceParameter API	246

5.11.0	Introduction	246

5.11.1	Resources	246

5.11.1.1	Overview	246

5.11.1.2	Resource: Service Parameter Subscriptions	247

5.11.1.2.1	Introduction	247

5.11.1.2.2	Resource Definition	247

5.11.1.2.3	Resource Methods	247

5.11.1.2.3.1	General	247

5.11.1.2.3.2	GET	247

5.11.1.2.3.3	POST	249

5.11.1.3	Resource: Individual Service Parameter Subscription	249

5.11.1.3.1	Introduction	249

5.11.1.3.2	Resource Definition	249

5.11.1.3.3	Resource Methods	249

5.11.1.3.3.1	General	249

5.11.1.3.3.2	GET	250

5.11.1.3.3.3	PUT	250

5.11.1.3.3.4	DELETE	251

5.11.1.3.3.5	PATCH	252

5.11.1A	Notifications	253

5.11.1A.1	Introduction	253

5.11.1A.2	AF Notifications	253

5.11.1A.2.1	Description	253

5.11.1A.2.2	Target URI	253

5.11.1A.3	Operation Definition	254

5.11.1A.3.1	Notification via HTTP POST	254

5.11.1A.3.2	Notification via Websocket	255

5.11.1B	Custom Operations without associated resources	255

5.11.2	Data Model	255

5.11.2.1	General	255

5.11.2.2	Reused data types	257

5.11.2.3	Structured data types	257

5.11.2.3.1	Introduction	257

5.11.2.3.2	Type: ServiceParameterData	258

5.11.2.3.3	Type: ServiceParameterDataPatch	260

5.11.2.3.4	Type: UrspRuleRequest	261

5.11.2.3.5	Type: RouteSelectionParameterSet	262

5.11.2.3.6	Type: AfNotification	262

5.11.2.3.7	Type: EventInfo	263

5.11.2.3.8	Type: TrafficDescriptorComponents	264

5.11.2.3.9	Type: NetworkDescription	265

5.11.2.3.10	Void	265

5.11.2.3.11	Void	265

5.11.2.4	Simple data types and enumerations	265

5.11.2.4.1	Introduction	265

5.11.2.4.2	Simple data types	265

5.11.2.4.3	Enumeration: Event	267

5.11.2.4.4	Enumeration: AuthorizationResult	267

5.11.2.4.5	Enumeration: Failure	267

5.11.2.4.6	Enumeration: ConnectionCapabilities	267

5.11.3	Used Features	268

5.11.4	Error handling	269

5.11.4.1	General	269

5.11.4.2	Protocol Errors	269

5.11.4.3	Application Errors	269

5.12	ACSParameterProvision API	270

5.12.0	Introduction	270

5.12.1	Resources	270

5.12.1.1	Overview	270

5.12.1.2	Resource: ACS Configuration Subscriptions	271

5.12.1.2.1	Introduction	271

5.12.1.2.2	Resource Definition	271

5.12.1.2.3	Resource Methods	271

5.12.1.2.3.1	General	271

5.12.1.2.3.2	GET	271

5.12.1.2.3.3	POST	272

5.12.1.3	Resource: Individual ACS Configuration Subscription	273

5.12.1.3.1	Introduction	273

5.12.1.3.2	Resource Definition	273

5.12.1.3.3	Resource Methods	273

5.12.1.3.3.1	General	273

5.12.1.3.3.2	GET	273

5.12.1.3.3.3	PUT	274

5.12.1.3.3.3A	PATCH	275

5.12.1.3.3.4	DELETE	276

5.12.1A	Custom Operations without associated resources	277

5.12.1B	Notifications	277

5.12.2	Data Model	277

5.12.2.1	General	277

5.12.2.2	Reused data types	277

5.12.2.3	Structured data types	277

5.12.2.3.1	Introduction	277

5.12.2.3.2	Type: AcsConfigurationData	278

5.12.2.3.3	Type: AcsConfigurationDataPatch	278

5.12.2.4	Simple data types and enumerations	278

5.12.2.4.1	Introduction	278

5.12.2.4.2	Simple data types	278

5.12.3	Used Features	279

5.12.4	Error handling	279

5.12.4.1	General	279

5.12.4.2	Protocol Errors	279

5.12.4.3	Application Errors	279

5.13	MoLcsNotify API	279

5.13.0	Introduction	279

5.13.1	Resources	279

5.13.1A	Custom Operations without associated resources	280

5.13.2	Notifications	280

5.13.2.1	Introduction	280

5.13.2.2	Event Notification	280

5.13.2.3	Operation Definition	280

5.13.2.3.1	Notification via HTTP POST	280

5.13.3	Data Model	281

5.13.3.1	General	281

5.13.3.2	Reused data types	281

5.13.3.3	Structured data types	281

5.13.3.3.1	Introduction	281

5.13.3.3.2	Type: LocUpdateData	281

5.13.3.3.3	Type: LocUpdateDataReply	282

5.13.3.4	Simple data types and enumerations	282

5.13.3.4.1	Introduction	282

5.13.3.4.2	Simple data types	282

5.13.4	Used Features	282

5.13.5	Error handling	283

5.13.5.1	General	283

5.13.5.2	Protocol Errors	283

5.13.5.3	Application Errors	283

5.14	AKMA API	283

5.14.1	Introduction	283

5.14.2	Resources	283

5.14.3	Custom Operations without associated resources	283

5.14.3.1	Overview	283

5.14.3.2	Operation: Retrieve	284

5.14.3.2.1	Description	284

5.14.3.2.2	Operation Definition	284

5.14.4	Notifications	285

5.14.4.1	General	285

5.14.4.2	AKMA Service Disablement Notification	285

5.14.4.2.1	Description	285

5.14.4.2.2	Target URI	285

5.14.4.2.3	Operation Definition	285

5.14.5	Data Model	286

5.14.5.1	General	286

5.14.5.2	Reused data types	287

5.14.5.3	Structured data types	287

5.14.5.3.1	Introduction	287

5.14.5.3.2	Type: AkmaAfKeyRequest	287

5.14.5.3.3	Type: AkmaAfKeyData	288

5.14.5.3.4	Type: ServiceDisableNotif	288

5.14.5.4	Simple data types and enumerations	288

5.14.5.4.1	Introduction	288

5.14.5.4.2	Simple data types	288

5.14.6	Used Features	289

5.14.7	Error handling	289

5.14.7.1	General	289

5.14.7.2	Protocol Errors	289

5.14.7.3	Application Errors	289

5.15	TimeSyncExposure API	290

5.15.0	Introduction	290

5.15.1	Resources	290

5.15.1.1	Overview	290

5.15.1.2	Resource: Time Synchronization Exposure Subscriptions	291

5.15.1.2.1	Introduction	291

5.15.1.2.2	Resource Definition	292

5.15.1.2.3	Resource Methods	292

5.15.1.2.3.1	General	292

5.15.1.2.3.2	GET	292

5.15.1.2.3.3	POST	293

5.15.1.3	Resource: Individual Time Synchronization Exposure Subscription	294

5.15.1.3.1	Introduction	294

5.15.1.3.2	Resource Definition	294

5.15.1.3.3	Resource Methods	294

5.15.1.3.3.1	General	294

5.15.1.3.3.2	GET	294

5.15.1.3.3.3	PUT	295

5.15.1.3.3.4	DELETE	296

5.15.1.4	Resource: Time Synchronization Exposure Configurations	297

5.15.1.4.1	Introduction	297

5.15.1.4.2	Resource Definition	297

5.15.1.4.3	Resource Methods	298

5.15.1.4.3.1	General	298

5.15.1.4.3.2	GET	298

5.15.1.4.3.3	POST	299

5.15.1.5	Resource: Individual Time Synchronization Exposure Configuration	299

5.15.1.5.1	Introduction	299

5.15.1.5.2	Resource Definition	299

5.15.1.5.3	Resource Methods	300

5.15.1.5.3.1	General	300

5.15.1.5.3.2	GET	300

5.15.1.5.3.3	PUT	301

5.15.1.5.3.4	DELETE	301

5.15.2	Custom Operations without associated resources	302

5.15.3	Notifications	303

5.15.3.1	Introduction	303

5.15.3.2	Time Synchronization Capability Notification	303

5.15.3.2.1	Description	303

5.15.3.2.2	Callback URI	303

5.15.3.2.3	Operation Definition	303

5.15.3.2.3.1	Notification via HTTP POST	303

5.15.3.2.3.2	Notification via Websocket	304

5.15.3.3	Time Synchronization Configuration Notification	304

5.15.3.3.1	Description	304

5.15.3.3.2	Callback URI	304

5.15.3.3.3	Operation Definition	305

5.15.3.3.3.1	Notification via HTTP POST	305

5.15.3.3.3.2	Void	305

5.15.4	Data Model	305

5.15.4.1	General	305

5.15.4.2	Reused data types	306

5.15.4.3	Structured data types	307

5.15.4.3.1	Introduction	307

5.15.4.3.2	Type: TimeSyncExposureSubsc	308

5.15.4.3.3	Type: TimeSyncCapability	310

5.15.4.3.4	Void	311

5.15.4.3.5	Void	311

5.15.4.3.6	Type: TimeSyncExposureConfig	311

5.15.4.3.7	Type: TimeSyncExposureSubsNotif	312

5.15.4.3.8	Type SubsEventNotification	312

5.15.4.3.9	Type: TimeSyncExposureConfigNotif	312

5.15.4.3.10	Type: EventFilter	313

5.15.4.3.11	Type: PtpCapabilitiesPerUe	313

5.15.4.3.12	Type: PtpInstance	313

5.15.4.3.13	Void	314

5.15.4.3.14	Void	314

5.15.4.3.15	Void	314

5.15.4.3.16	Void	314

5.15.4.3.17	Type: StateOfConfiguration	314

5.15.4.3.18	Type: ConfigForPort	315

5.15.4.3.19	Type: StateOfDstt	316

5.15.4.3.20	Void	317

5.15.4.3.21	Void	317

5.15.4.3.22	Type: AvailReport	317

5.15.4.4	Simple data types and enumerations	317

5.15.4.4.1	Introduction	317

5.15.4.4.2	Simple data types	317

5.15.4.4.3	Void	317

5.15.4.4.4	Enumeration: Protocol	317

5.15.4.4.5	Enumeration: GmCapable	317

5.15.4.4.6	Enumeration: SubscribedEvent	318

5.15.4.4.7	Enumeration: InstanceType	318

5.15.4.4.8	Enumeration: AsTimeResource	318

5.15.4.4.9	Enumeration: AcceptanceCriteriaResultIndication	318

5.15.4.4.10	Enumeration: AvailStatus	319

5.15.4.5	Data types describing alternative data types or combinations of data types	319

5.15.4.5.1	Type: ReportedCabability	319

5.15.5	Used Features	319

5.15.6	Error handling	319

5.15.6.1	General	319

5.15.6.2	Protocol Errors	319

5.15.6.3	Application Errors	320

5.16	EcsAddressProvision API	320

5.16.0	Introduction	320

5.16.1	Resources	320

5.16.1.1	Overview	320

5.16.1.2	Resource: ECS Address Provision Configurations	321

5.16.1.2.1	Introduction	321

5.16.1.2.2	Resource Definition	321

5.16.1.2.3	Resource Methods	321

5.16.1.2.3.1	General	321

5.16.1.2.3.2	GET	321

5.16.1.2.3.3	POST	322

5.16.1.3	Resource: Individual ECS Address Provision Configuration	323

5.16.1.3.1	Introduction	323

5.16.1.3.2	Resource Definition	323

5.16.1.3.3	Resource Methods	323

5.16.1.3.3.1	General	323

5.16.1.3.3.2	GET	323

5.16.1.3.3.3	PUT	324

5.16.1.3.3.4	DELETE	325

5.16.1A	Custom Operations without associated resources	326

5.16.1B	Notifications	326

5.16.2	Data Model	326

5.16.2.1	General	326

5.16.2.2	Reused data types	326

5.16.2.3	Structured data types	327

5.16.2.3.1	Introduction	327

5.16.2.3.2	Type: EcsAddressProvision	327

5.16.2.4	Simple data types and enumerations	327

5.16.2.4.1	Introduction	327

5.16.2.4.2	Simple data types	327

5.16.3	Used Features	327

5.16.4	Error handling	328

5.16.4.1	General	328

5.16.4.2	Protocol Errors	328

5.16.4.3	Application Errors	328

5.17	AMPolicyAuthorization API	328

5.17.0	Introduction	328

5.17.1	Resources	328

5.17.1.1	Overview	328

5.17.1.2	Resource: Application AM Contexts	330

5.17.1.2.1	Introduction	330

5.17.1.2.2	Resource Definition	330

5.17.1.2.3	Resource Methods	330

5.17.1.2.3.1	General	330

5.17.1.2.3.2	POST	330

5.17.1.3	Resource: Individual Application AM Context	331

5.17.1.3.1	Introduction	331

5.17.1.3.2	Resource Definition	331

5.17.1.3.3	Resource Methods	331

5.17.1.3.3.1	General	331

5.17.1.3.3.2	GET	331

5.17.1.3.3.3	PATCH	332

5.17.1.3.3.4	DELETE	333

5.17.1.4	Resource: AM Policy Events Subscription	334

5.17.1.4.1	Introduction	334

5.17.1.4.2	Resource Definition	334

5.17.1.4.3	Resource Methods	335

5.17.1.4.3.1	General	335

5.17.1.4.3.2	PUT	335

5.17.1.4.3.3	DELETE	337

5.17.1A	Custom Operations without associated resources	337

5.17.2	Notifications	338

5.17.2.1	Introduction	338

5.17.2.2	AM Event Notification	338

5.17.2.2.1	Description	338

5.17.2.2.2	Callback URI	338

5.17.2.2.3	Operation Definition	338

5.17.2.2.3.1	Notification via HTTP POST	338

5.17.2.2.3.2	Notification via Websocket	339

5.17.3	Data Model	339

5.17.3.1	General	339

5.17.3.2	Reused data types	340

5.17.3.3	Structured data types	340

5.17.3.3.1	Introduction	340

5.17.3.3.2	Type: AppAmContextExpData	341

5.17.3.3.3	Type: AppAmContextExpUpdateData	342

5.17.3.3.4	Type: GeographicalArea	342

5.17.3.4	Simple data types and enumerations	342

5.17.3.4.1	Introduction	342

5.17.3.4.2	Simple data types	342

5.17.3.5	Data types describing alternative data types or combinations of data types	342

5.17.3.5.1	Type: AppAmContextExpRespData	342

5.17.4	Used Features	343

5.17.5	Error handling	343

5.17.5.1	General	343

5.17.5.2	Protocol Errors	343

5.17.5.3	Application Errors	343

5.18	AMInfluence API	343

5.18.0	Introduction	343

5.18.1	Resources	344

5.18.1.1	Overview	344

5.18.1.2	Resource: AM Influence Subscription	344

5.18.1.2.1	Introduction	344

5.18.1.2.2	Resource Definition	345

5.18.1.2.3	Resource Methods	345

5.18.1.2.3.1	General	345

5.18.1.2.3.2	GET	345

5.18.1.2.3.3	POST	346

5.18.1.3	Resource: Individual AM Influence Subscription	346

5.18.1.3.1	Introduction	346

5.18.1.3.2	Resource Definition	346

5.18.1.3.3	Resource Methods	347

5.18.1.3.3.1	General	347

5.18.1.3.3.2	GET	347

5.18.1.3.3.3	PUT	348

5.18.1.3.3.4	PATCH	348

5.18.1.3.3.5	DELETE	349

5.18.1A	Custom Operations without associated resources	350

5.18.2	Notifications	350

5.18.2.1	Introduction	350

5.18.2.2	Event Notification	351

5.18.2.2.1	Description	351

5.18.2.2.2	Target URI	351

5.18.2.2.3	Operation Definition	351

5.18.3	Data Model	352

5.18.3.1	General	352

5.18.3.2	Reused data types	352

5.18.3.3	Structured data types	353

5.18.3.3.1	Introduction	353

5.18.3.3.2	Type: AmInfluSub	353

5.18.3.3.3	Type: AmInfluSubPatch	355

5.18.3.3.4	Type: AmInfluEventNotif	356

5.18.3.3.5	Type: DnnSnssaiInformation	357

5.18.3.4	Simple data types and enumerations	357

5.18.3.4.1	Introduction	357

5.18.3.4.2	Simple data types	357

5.18.3.4.3	Enumeration: AmInfluEvent	357

5.18.4	Used Features	357

5.18.5	Error handling	358

5.18.5.1	General	358

5.18.5.2	Protocol Errors	358

5.18.5.3	Application Errors	358

5.19	MBSTMGI API	358

5.19.1	Introduction	358

5.19.2	Resources	358

5.19.3	Custom Operations without associated resources	358

5.19.3.1	Overview	358

5.19.3.2	Operation: Allocate	359

5.19.3.2.1	Description	359

5.19.3.2.2	Operation Definition	359

5.19.3.3	Operation: Deallocate	360

5.19.3.3.1	Description	360

5.19.3.3.2	Operation Definition	360

5.19.4	Notifications	361

5.19.4.1	General	361

5.19.4.2	Notification of Allocated MBS TMGI(s) Timer Expiry	361

5.19.4.2.1	Description	361

5.19.4.2.2	Target URI	361

5.19.4.2.3	Operation Definition	362

5.19.4.2.3.1	Notification via HTTP POST	362

5.19.4.2.3.2	Notification via Websocket	362

5.19.5	Data Model	363

5.19.5.1	General	363

5.19.5.2	Structured data types	363

5.19.5.2.1	Introduction	363

5.19.5.2.2	Type: TmgiAllocRequest	364

5.19.5.2.3	Type: TmgiAllocResponse	365

5.19.5.2.4	Type: TmgiDeallocRequest	365

5.19.5.2.5	Type: ExpiryNotif	365

5.19.5.2.6	Type: ReducedMbsServArea	365

5.19.5.3	Simple data types and enumerations	366

5.19.5.3.1	Introduction	366

5.19.5.3.2	Simple data types	366

5.19.5.4	Data types describing alternative data types or combinations of data types	366

5.19.5.4.1	Type: ProblemDetailsTmgiAlloc	366

5.19.6	Used Features	366

5.19.7	Error handling	366

5.19.7.1	General	366

5.19.7.2	Protocol Errors	366

5.19.7.3	Application Errors	366

5.20	MBSSession API	367

5.20.1	Introduction	367

5.20.2	Resources	367

5.20.2.1	Overview	367

5.20.2.2	Resource: MBS sessions	369

5.20.2.2.1	Introduction	369

5.20.2.2.2	Resource Definition	369

5.20.2.2.3	Resource Methods	370

5.20.2.2.3.1	POST	370

5.20.2.2.4	Resource Custom Operations	370

5.20.2.3	Resource: Individual MBS Session	370

5.20.2.3.1	Introduction	370

5.20.2.3.2	Resource Definition	370

5.20.2.3.3	Resource Standard Methods	371

5.20.2.3.3.1	PATCH	371

5.20.2.3.3.3	DELETE	372

5.20.2.3.4	Resource Custom Operations	373

5.20.2.4	Resource: MBS Session Subscriptions	373

5.20.2.4.1	Introduction	373

5.20.2.4.2	Resource Definition	373

5.20.2.4.3	Resource Methods	373

5.20.2.4.3.1	GET	373

5.20.2.4.3.2	POST	374

5.20.2.4.4	Resource Custom Operations	375

5.20.2.5	Resource: Individual MBS Session Subscription	375

5.20.2.5.1	Introduction	375

5.20.2.5.2	Resource Definition	375

5.20.2.5.3	Resource Methods	375

5.20.2.5.3.1	GET	375

5.20.2.5.3.2	DELETE	376

5.20.2.5.4	Resource Custom Operations	377

5.20.2.6	Resource: MBS Parameters Provisionings	377

5.20.2.6.1	Introduction	377

5.20.2.6.2	Resource Definition	377

5.20.2.6.3	Resource Methods	378

5.20.2.6.3.1	GET	378

5.20.2.6.3.2	POST	379

5.20.2.6.4	Resource Custom Operations	379

5.20.2.7	Resource: Individual MBS Parameters Provisioning	379

5.20.2.7.1	Introduction	379

5.20.2.7.2	Resource Definition	380

5.20.2.7.3	Resource Methods	380

5.20.2.7.3.1	GET	380

5.20.2.7.3.2	PUT	381

5.20.2.7.3.3	PATCH	382

5.20.2.7.3.4	DELETE	383

5.20.2.7.4	Resource Custom Operations	384

5.20.3	Custom Operations without associated resources	384

5.20.4	Notifications	384

5.20.4.1	General	384

5.20.4.2	MBS Session Status Notification	384

5.20.4.2.1	Description	384

5.20.4.2.2	Target URI	384

5.20.4.2.3	Operation Definition	384

5.20.4.2.3.1	Notification via HTTP POST	384

5.20.5	Data Model	385

5.20.5.1	General	385

5.20.5.2	Structured data types	386

5.20.5.2.1	Introduction	386

5.20.5.2.2	Type: MbsSessionCreateReq	387

5.20.5.2.3	Type: MbsSessionCreateRsp	387

5.20.5.2.4	Type: MbsSessionSubsc	387

5.20.5.2.5	Type: MbsSessionStatusNotif	387

5.20.5.2.6	Type: MbsPpData	388

5.20.5.2.7	Type: MbsSessAuthData	388

5.20.5.2.8	Type: MbsPpDataPatch	389

5.20.5.2.9	Type: MbsSessAssistInfo	389

5.20.5.2.10	Type: MbsSessionUpdateResp	389

5.20.5.3	Simple data types and enumerations	389

5.20.5.3.1	Introduction	389

5.20.5.3.2	Simple data types	390

5.20.6	Used Features	390

5.20.7	Error handling	390

5.20.7.1	General	390

5.20.7.2	Protocol Errors	390

5.20.7.3	Application Errors	390

5.21	EASDeployment API	391

5.21.0	Introduction	391

5.21.1	Resources	391

5.21.1.1	Overview	391

5.21.1.2	Resource: EAS Deployment Information	392

5.21.1.2.1	Introduction	392

5.21.1.2.2	Resource Definition	392

5.21.1.2.3	Resource Methods	393

5.21.1.2.3.1	General	393

5.21.1.2.3.2	GET	393

5.21.1.2.3.3	POST	394

5.21.1.3	Resource: Individual EAS Deployment Information	394

5.21.1.3.1	Introduction	394

5.21.1.3.2	Resource Definition	394

5.21.1.3.3	Resource Methods	394

5.21.1.3.3.1	General	394

5.21.1.3.3.2	GET	395

5.21.1.3.3.3	PUT	395

5.21.1.3.3.4	DELETE	396

5.21.2	Custom Operations without associated resources	397

5.21.2.1	Overview	397

5.21.2.2	Operation: remove-edis	398

5.21.2.2.1	Description	398

5.21.2.2.2	Operation Definition	398

5.21.3	Notifications	398

5.21.4	Data Model	398

5.21.4.1	General	398

5.21.4.2	Reused data types	399

5.21.4.3	Structured data types	399

5.21.4.3.1	Introduction	399

5.21.4.3.2	Type: EasDeployInfo	400

5.21.4.3.3	Type: DnaiInformation	400

5.21.4.3.4	Type: DnsServerIdentifier	401

5.21.4.3.5	Type: EdiDeleteCriteria	401

5.21.4.4	Simple data types and enumerations	401

5.21.4.4.1	Introduction	401

5.21.4.4.2	Simple data types	401

5.21.5	Used Features	401

5.21.6	Error handling	401

5.21.6.1	General	401

5.21.6.2	Protocol Errors	402

5.21.6.3	Application Errors	402

5.22	ASTI API	402

5.22.0	Introduction	402

5.22.1	Resources	402

5.22.1.1	Overview	402

5.22.1.2	Resource: ASTI Configurations	403

5.22.1.2.1	Introduction	403

5.22.1.2.2	Resource Definition	403

5.22.1.2.3	Resource Methods	404

5.22.1.2.3.1	General	404

5.22.1.2.3.2	GET	404

5.22.1.2.3.3	POST	405

5.22.1.2.4	Resource Custom Operations	405

5.22.1.2.4.1	Overview	405

5.22.1.2.4.2	Operation: retrieve	405

5.22.1.2.4.2.1	Description	405

5.22.1.2.4.2.2	Operation Definition	406

5.22.1.3	Resource: Individual ASTI Configuration	406

5.22.1.3.1	Introduction	406

5.22.1.3.2	Resource Definition	406

5.22.1.3.3	Resource Methods	406

5.22.1.3.3.1	General	406

5.22.1.3.3.2	GET	406

5.22.1.3.3.3	PUT	407

5.22.1.3.3.4	DELETE	408

5.22.2	Custom Operations without associated resources	409

5.22.3	Notifications	409

5.22.3.1	General	409

5.22.3.2	ASTI Notification	409

5.22.3.2.1	Description	409

5.22.3.2.2	Target URI	409

5.22.3.2.3	Operation Definition	410

5.22.3.2.3.1	Notification via HTTP POST	410

5.22.3.2.3.2	Void	411

5.22.4	Data Model	411

5.22.4.1	General	411

5.22.4.2	Reused data types	411

5.22.4.3	Structured data types	411

5.22.4.3.1	Introduction	411

5.22.4.3.2	Type: AccessTimeDistributionData	412

5.22.4.3.3	Type: StatusRequestData	412

5.22.4.3.4	Type: StatusResponseData	413

5.22.4.3.5	Type: ActiveUe	413

5.22.4.3.6	Type AstiConfigNotification	413

5.22.4.3.7	Type AstiConfigStateNotification	413

5.22.4.4	Simple data types and enumerations	413

5.22.4.4.1	Introduction	413

5.22.4.4.2	Simple data types	414

5.22.5	Used Features	414

5.22.6	Error handling	414

5.22.6.1	General	414

5.22.6.2	Protocol Errors	414

5.22.6.3	Application Errors	414

5.23	DataReporting API	415

5.23.1	Introduction	415

5.23.2	Resources	415

5.23.2.1	Overview	415

5.23.2.2	Resource: Data Reporting Sessions	416

5.23.2.2.1	Introduction	416

5.23.2.2.2	Resource definition	416

5.23.2.2.3	Resource Methods	416

5.23.2.2.3.1	POST	416

5.23.2.3	Resource: Individual Data Reporting Session	417

5.23.2.3.1	Introduction	417

5.23.2.3.2	Resource Definition	417

5.23.2.3.3	Resource standard methods	417

5.23.2.3.3.1	GET	417

5.23.2.3.3.2	PUT	418

5.23.2.3.3.3	DELETE	420

5.23.2.3.4	Resource custom operations	421

5.23.3	Custom Operations without associated resources	422

5.23.4	Notifications	422

5.23.5	Data Model	422

5.23.3.1	General	422

5.23.6	Used Features	422

5.23.7	Error handling	422

5.23.7.1	General	422

5.23.7.2	Protocol Errors	422

5.23.7.3	Application Errors	422

5.24	DataReportingProvisioning API	423

5.24.1	Introduction	423

5.24.2	Resources	423

5.24.2.1	Overview	423

5.24.2.2	Resource: Data Reporting Provisioning Sessions	425

5.24.2.2.1	Introduction	425

5.24.2.2.2	Resource definition	425

5.24.2.2.3	Resource Methods	426

5.24.2.2.3.1	POST	426

5.24.2.3	Resource: Individual Data Reporting Provisioning Session	426

5.24.2.3.1	Introduction	426

5.24.2.3.2	Resource Definition	427

5.24.2.3.3	Resource standard methods	427

5.24.2.3.3.1	GET	427

5.24.2.3.3.2	Void	428

5.24.2.3.3.3	DELETE	428

5.24.2.4	Resource: Data Reporting Configurations	429

5.24.2.4.1	Introduction	429

5.24.2.4.2	Resource definition	430

5.24.2.4.3	Resource Methods	430

5.24.2.4.3.1	POST	430

5.24.2.5	Resource: Individual Data Reporting Configuration	431

5.24.2.5.1	Introduction	431

5.24.2.5.2	Resource Definition	431

5.24.2.5.3	Resource standard methods	431

5.24.2.5.3.2	GET	431

5.24.2.5.3.3	PUT	432

5.24.2.5.3.3A	PATCH	434

5.24.2.5.3.4	DELETE	435

5.24.3	Custom Operations without associated resources	435

5.24.4	Notifications	436

5.24.5	Data Model	436

5.24.5.1	General	436

5.24.6	Used Features	436

5.24.7	Error handling	436

5.24.7.1	General	436

5.24.7.2	Protocol Errors	436

5.24.7.3	Application Errors	437

5.25	UEId API	437

5.25.1	Introduction	437

5.25.2	Resources	437

5.25.2.1	Overview	437

5.25.2.2	Resource: UE ID Mapping Information Provisionings	438

5.25.2.2.1	Introduction	438

5.25.2.2.2	Resource Definition	438

5.25.2.2.3	Resource Methods	438

5.25.2.2.3.1	GET	438

5.25.2.2.3.2	POST	439

5.25.2.2.4	Resource Custom Operations	440

5.25.2.3	Resource: Individual UE ID Mapping Provisioning	440

5.25.2.3.1	Introduction	440

5.25.2.3.2	Resource Definition	440

5.25.2.3.3	Resource Methods	441

5.25.2.3.3.1	GET	441

5.25.2.3.3.2	PUT	442

5.25.2.3.3.3	PATCH	443

5.25.2.3.3.4	DELETE	444

5.25.3	Custom Operations without associated resources	445

5.25.3.1	Overview	445

5.25.3.2	Operation: Retrieve	446

5.25.3.2.1	Description	446

5.25.3.2.2	Operation Definition	446

5.25.3.2.3	Operation Definition	447

5.25.4	Notifications	448

5.25.5	Data Model	448

5.25.5.1	General	448

5.25.5.2	Structured data types	449

5.25.5.2.1	Introduction	449

5.25.5.2.2	Type: UeIdReq	450

5.25.5.2.3	Type: UeIdInfo	450

5.25.5.2.4	Type: MsisdnReq	451

5.25.5.2.5	Type: MsisdnInfo	451

5.25.5.2.6	Type: UeIdMappingInfo	452

5.25.5.2.7	Type: RangSlUeIdMappInfo	452

5.25.5.2.8	Type: UeIdMappingInfoPatch	452

5.25.5.3	Simple data types and enumerations	452

5.25.5.3.1	Introduction	452

5.25.5.3.2	Simple data types	452

5.25.6	Used Features	453

5.25.7	Error handling	453

5.25.7.1	General	453

5.25.7.2	Protocol Errors	453

5.25.7.3	Application Errors	453

5.25.8	Security	453

5.26	MBSUserService API	453

5.26.1	Introduction	453

5.26.2	Resources	454

5.26.2.1	Overview	454

5.26.2.2	Resource: MBS User Services	455

5.26.2.2.1	Introduction	455

5.26.2.2.2	Resource Definition	455

5.26.2.2.3	Resource Standard Methods	455

5.26.2.2.3.1	GET	455

5.26.2.2.3.2	POST	456

5.26.2.2.4	Resource Custom Operations	456

5.26.2.3	Resource: Individual MBS User Service	457

5.26.2.3.1	Introduction	457

5.26.2.3.2	Resource Definition	457

5.26.2.3.3	Resource Standard Methods	457

5.26.2.3.3.1	GET	457

5.26.2.3.3.2	PUT	458

5.26.2.3.3.3	PATCH	459

5.26.2.3.3.4	DELETE	460

5.26.2.3.4	Resource Custom Operations	461

5.26.3	Custom Operations without associated resources	461

5.26.4	Notifications	461

5.26.5	Data Model	461

5.26.5.1	General	461

5.26.5.2	Structured data types	462

5.26.5.2.1	Introduction	462

5.26.5.3	Simple data types and enumerations	462

5.26.5.3.1	Introduction	462

5.26.5.3.2	Simple data types	462

5.26.6	Used Features	462

5.26.7	Error handling	462

5.26.7.1	General	462

5.26.7.2	Protocol Errors	463

5.26.7.3	Application Errors	463

5.27	MBSUserDataIngestSession API	463

5.27.1	Introduction	463

5.27.2	Resources	463

5.27.2.1	Overview	463

5.27.2.2	Resource: MBS User Data Ingest Sessions	465

5.27.2.2.1	Introduction	465

5.27.2.2.2	Resource Definition	465

5.27.2.2.3	Resource Standard Methods	465

5.27.2.2.3.1	GET	465

5.27.2.2.3.2	POST	466

5.27.2.2.4	Resource Custom Operations	467

5.27.2.3	Resource: Individual MBS User Data Ingest Session	467

5.27.2.3.1	Introduction	467

5.27.2.3.2	Resource Definition	467

5.27.2.3.3	Resource Standard Methods	467

5.27.2.3.3.1	GET	467

5.27.2.3.3.2	PUT	468

5.27.2.3.3.3	PATCH	469

5.27.2.3.3.4	DELETE	471

5.27.2.3.4	Resource Custom Operations	471

5.27.2.4	Resource: MBS User Data Ingest Session Status Subscriptions	472

5.27.2.4.1	Introduction	472

5.27.2.4.2	Resource Definition	472

5.27.2.4.3	Resource Standard Methods	472

5.27.2.4.3.1	GET	472

5.27.2.4.3.2	POST	473

5.27.2.4.4	Resource Custom Operations	474

5.27.2.5	Resource: Individual MBS User Data Ingest Session Status Subscription	474

5.27.2.5.1	Introduction	474

5.27.2.5.2	Resource Definition	474

5.27.2.5.3	Resource Standard Methods	474

5.27.2.5.3.1	GET	474

5.27.2.5.3.2	PUT	475

5.27.2.5.3.3	PATCH	476

5.27.2.5.3.4	DELETE	477

5.27.2.5.4	Resource Custom Operations	478

5.27.3	Custom Operations without associated resources	478

5.27.4	Notifications	478

5.27.4.1	General	478

5.27.4.2	MBS User Data Ingest Session Status Change Notification	479

5.27.4.2.1	Description	479

5.27.4.2.2	Target URI	479

5.27.4.2.3	Operation Definition	479

5.27.4.2.3.1	Notification via HTTP POST	479

5.27.4.2.3.2	Void	480

5.27.5	Data Model	480

5.27.5.1	General	480

5.27.5.2	Structured data types	481

5.27.5.2.1	Introduction	481

5.27.5.3	Simple data types and enumerations	481

5.27.5.3.1	Introduction	481

5.27.5.3.2	Simple data types	481

5.27.6	Used Features	481

5.27.7	Error handling	482

5.27.7.1	General	482

5.27.7.2	Protocol Errors	482

5.27.7.3	Application Errors	482

5.28	MSEventExposure API	483

5.28.1	Introduction	483

5.28.2	Resources	484

5.28.2.1	Overview	484

5.28.2.2	Resource: Media Streaming Event Exposure Subscriptions	484

5.28.2.2.1	Introduction	484

5.28.2.2.2	Resource Definition	484

5.28.2.2.3	Resource Standard Methods	485

5.28.2.2.3.1	GET	485

5.28.2.2.3.2	POST	486

5.28.2.2.4	Resource Custom Operations	486

5.28.2.3	Resource: Individual Media Streaming Event Exposure Subscription	486

5.28.2.3.1	Introduction	486

5.28.2.3.2	Resource Definition	487

5.28.2.3.3	Resource Standard Methods	487

5.28.2.3.3.1	GET	487

5.28.2.3.3.2	PUT	488

5.28.2.3.3.3	DELETE	489

5.28.2.3.4	Resource Custom Operations	490

5.28.3	Custom Operations without associated resources	490

5.28.4	Notifications	490

5.28.4.1	General	490

5.28.4.2	Media Streaming Event Exposure Notification	490

5.28.4.2.1	Description	490

5.28.4.2.2	Target URI	490

5.28.4.2.3	Operation Definition	490

5.28.4.2.3.1	Notification via HTTP POST	490

5.28.4.2.3.2	Void	491

5.28.5	Data Model	491

5.28.5.1	General	491

5.28.5.2	Structured data types	492

5.28.5.2.1	Introduction	492

5.28.5.3	Simple data types and enumerations	492

5.28.5.3.1	Introduction	492

5.28.5.3.2	Simple data types	492

5.28.6	Used Features	492

5.28.7	Error handling	492

5.28.7.1	General	492

5.28.7.2	Protocol Errors	493

5.28.7.3	Application Errors	493

5.29	MBSGroupMsgDelivery API	493

5.29.1	Introduction	493

5.29.2	Resources	493

5.29.2.1	Overview	493

5.29.2.2	Resource: MBS Group Message Deliveries	494

5.29.2.2.1	Introduction	494

5.29.2.2.2	Resource Definition	494

5.29.2.2.3	Resource Standard Methods	495

5.29.2.2.3.1	GET	495

5.29.2.2.3.2	POST	496

5.29.2.2.4	Resource Custom Operations	496

5.29.2.3	Resource: Individual MBS Group Message Delivery	496

5.29.2.3.1	Introduction	496

5.29.2.3.2	Resource Definition	496

5.29.2.3.3	Resource Standard Methods	497

5.29.2.3.3.1	GET	497

5.29.2.3.3.2	PATCH	498

5.29.2.3.3.3	DELETE	499

5.29.2.3.4	Resource Custom Operations	500

5.29.3	Custom Operations without associated resources	500

5.29.4	Notifications	500

5.29.4.1	General	500

5.29.4.2	MBS Group Message Delivery Notification	500

5.29.4.2.1	Description	500

5.29.4.2.2	Target URI	500

5.29.4.2.3	Operation Definition	501

5.29.5	Data Model	501

5.29.5.1	General	501

5.29.5.2	Structured data types	502

5.29.5.2.1	Introduction	502

5.29.5.2.2	Void	503

5.29.5.2.3	Type: MbsGroupMsgDel	503

5.29.5.2.4	Void	504

5.29.5.2.5	Type: MbsGroupMsgDelPatch	504

5.29.5.2.6	Type: MbsGroupMsgDelStatusNotif	504

5.29.5.3	Simple data types and enumerations	504

5.29.5.3.1	Introduction	504

5.29.5.3.2	Simple data types	504

5.29.5.4	Data types describing alternative data types or combinations of data types	505

5.29.5.4.1	Type: MbsServArea	505

5.29.6	Used Features	505

5.29.7	Error handling	505

5.29.7.1	General	505

5.29.7.2	Protocol Errors	505

5.29.7.3	Application Errors	505

5.30	DNAIMapping API	505

5.30.2	Resources	506

5.30.2.1	Overview	506

5.30.2.2	Resource: DNAI Mapping Subscriptions	507

5.30.2.2.1	Introduction	507

5.30.2.2.2	Resource Definition	507

5.30.2.2.3	Resource Standard Methods	507

5.30.2.2.3.1	Void	507

5.30.2.2.3.2	GET	507

5.30.2.2.3.3	POST	508

5.30.2.2.4	Resource Custom Operations	509

5.30.2.3	Resource: Individual DNAI Mapping Subscription	509

5.30.2.3.1	Introduction	509

5.30.2.3.2	Resource Definition	509

5.30.2.3.3	Resource Standard Methods	509

5.30.2.3.3.1	Void	509

5.30.2.3.3.2	GET	509

5.30.2.3.3.3	DELETE	510

5.30.2.3.4	Resource Custom Operations	511

5.30.4	Notifications	512

5.30.4.1	General	512

5.30.4.2	DNAI Mapping Information Update Notification	512

5.30.4.2.1	Description	512

5.30.4.2.2	Target URI	512

5.30.4.2.3	Operation Definition	512

5.30.4.2.3.1	Notification via HTTP POST	512

5.30.4.2.3.2	Void	513

5.30.5	Data Model	513

5.30.5.1	General	513

5.30.5.2	Structured data types	514

5.30.5.2.1	Introduction	514

5.30.5.2.2	Type: DnaiMapSub	515

5.30.5.2.3	Type: DnaiMapUpdateNotif	516

5.30.5.2.4	Void	516

5.30.5.3	Simple data types and enumerations	516

5.30.5.3.1	Introduction	516

5.30.5.3.2	Simple data types	516

5.30.5.4	Data types describing alternative data types or combinations of data types	516

5.30.6	Used Features	516

5.30.7	Error handling	517

5.30.7.1	General	517

5.30.7.2	Protocol Errors	517

5.30.7.3	Application Errors	517

5.31	PdtqPolicyNegotiation API	517

5.31.0	Introduction	517

5.31.1	Resources	517

5.31.1.1	Overview	517

5.31.1.2	Resource: PDTQ Policy Subscriptions	518

5.31.1.2.1	Introduction	518

5.31.1.2.2	Resource Definition	518

5.31.1.2.3	Resource Standard Methods	519

5.31.1.2.3.1	Void	519

5.31.1.2.3.2	GET	519

5.31.1.2.3.3	POST	520

5.31.1.2.4	Resource Custom Operations	520

5.31.1.3	Resource: Individual PDTQ Policy Subscription	520

5.31.1.3.1	Introduction	520

5.31.1.3.2	Resource Definition	521

5.31.1.3.3	Resource Standard Methods	521

5.31.1.3.3.1	Void	521

5.31.1.3.3.2	GET	521

5.31.1.3.3.3	PATCH	522

5.31.1.3.3.4	DELETE	523

5.31.1.3.4	Resource Custom Operations	524

5.31.1A	Custom Operations without associated resources	524

5.31.2	Notifications	524

5.31.2.1	General	524

5.31.2.2	PDTQ Warning Notification	525

5.31.2.2.1	Description	525

5.31.2.2.2	Target URI	525

5.31.2.2.3	Operation Definition	525

5.31.2.2.3.1	Notification via HTTP POST	525

5.31.3	Data Model	526

5.31.3.1	General	526

5.31.3.2	Void	526

5.31.3.3	Structured data types	526

5.31.3.3.1	Introduction	526

5.31.3.3.2	Type: Pdtq	527

5.31.3.3.3	Type: PdtqPatch	528

5.31.3.3.4	Type: PdtqNotification	528

5.31.3.4	Simple data types and enumerations	528

5.31.3.4.1	Introduction	528

5.31.3.4.2	Simple data types	528

5.31.3.4	Data types describing alternative data types or combinations of data types	529

5.31.4	Used Features	529

5.31.5	Error handling	529

5.31.5.1	General	529

5.31.5.2	Protocol Errors	529

5.31.5.3	Application Errors	529

5.32	MemberUESelectionAssistance API	529

5.32.1	Introduction	529

5.32.2	Resources	530

5.32.2.1	Overview	530

5.32.2.2	Resource: Member UE Selection Assistance Subscriptions	531

5.32.2.2.1	Introduction	531

5.32.2.2.2	Resource definition	531

5.32.2.2.3	Resource Standard Methods	532

5.32.2.2.3.1	POST	532

5.32.2.2.3.2	GET	532

5.32.2.2.4	Resource Custom Operations	533

5.32.2.3	Resource: Individual Member UE Selection Assistance Subscription	533

5.32.2.3.1	Introduction	533

5.32.2.3.2	Resource Definition	534

5.32.2.3.3	Resource Standard Methods	534

5.32.2.3.3.1	GET	534

5.32.2.3.3.2	PUT	535

5.32.2.3.3.3	PATCH	536

5.32.2.3.3.4	DELETE	537

5.32.2.3.4	Resource Custom Operations	538

5.32.3	Custom Operations without associated resources	538

5.32.4	Notifications	538

5.32.5	Data Model	540

5.32.5.1	General	540

5.32.5.2	Structured data types	542

5.32.5.2.1	Introduction	542

5.32.5.2.2	Type: MemUeSelectAssistSubsc	543

5.32.5.2.3	Type: MemUeSeletAssistNotif	544

5.32.5.2.4	Type: QoSFilterCriteria	544

5.32.5.2.5	Type: AccessRatTypeFilterCriteria	545

5.32.5.2.6	Type: E2ETransTimeFilterCriteria	545

5.32.5.2.7	Type: UeLocFilterCriteria	546

5.32.5.2.8	Type: UeHisLocFilterCriteria	546

5.32.5.2.9	Type: UeDirectionFilterCriteria	546

5.32.5.2.10	Type: UeDistanceFilterCriteria	547

5.32.5.2.11	Type: ServiceExpFilterCriteria	547

5.32.5.2.12	Type: DnnFilterCriteria	547

5.32.5.2.13	Type: CandiUeListInfo	548

5.32.5.2.14	Type: MemUeSeletReport	548

5.32.5.2.15	Type: MemUeSelectAssistSubscPatch	549

5.32.5.2.16	Type: CandUeInfo	550

5.32.5.3	Simple data types and enumerations	550

5.32.5.3.1	Introduction	550

5.32.5.3.2	Simple data types	550

5.32.5.3.3	Enumeration: FilterCriterionType	551

5.32.5.4	Data types describing alternative data types or combinations of data types	551

5.32.6	Used Features	551

5.32.7	Error handling	551

5.32.7.1	General	551

5.32.7.2	Protocol Errors	551

5.32.7.3	Application Errors	551

5.33	GroupParametersProvisioning API	552

5.33.1	Introduction	552

5.33.2	Resources	552

5.33.2.1	Overview	552

5.33.2.2	Resource: Group Parameters Provisionings	553

5.33.2.2.1	Introduction	553

5.33.2.2.2	Resource Definition	553

5.33.2.2.3	Resource Methods	553

5.33.2.2.3.1	GET	553

5.33.2.2.3.2	POST	554

5.33.2.2.4	Resource Custom Operations	555

5.33.2.3	Resource: Individual Group Parameters Provisioning	555

5.33.2.3.1	Introduction	555

5.33.2.3.2	Resource Definition	555

5.33.2.3.3	Resource Methods	556

5.33.2.3.3.1	GET	556

5.33.2.3.3.2	PUT	557

5.33.2.3.3.3	PATCH	558

5.33.2.3.3.4	DELETE	559

5.33.2.3.4	Resource Custom Operations	560

5.33.3	Custom Operations without associated resources	560

5.33.4	Notifications	560

5.33.5	Data Model	560

5.33.5.1	General	560

5.33.5.2	Structured data types	561

5.33.5.2.1	Introduction	561

5.33.5.2.2	Type: GrpPpData	561

5.33.5.2.3	Type: GrpPpDataPatch	561

5.33.5.2.4	Type: DnnSnssaiGrpData	562

5.33.5.2.5	Type: AfReqDefaultQoS	562

5.33.5.2.6	Type: LadnServArea	562

5.33.5.3	Simple data types and enumerations	562

5.33.5.3.1	Introduction	562

5.33.5.3.2	Simple data types	563

5.33.5.4	Data types describing alternative data types or combinations of data types	563

5.33.6	Used Features	563

5.33.7	Error handling	563

5.33.7.1	General	563

5.33.7.2	Protocol Errors	563

5.33.7.3	Application Errors	563

5.34	SliceParamProvision API	563

5.34.1	Introduction	563

5.34.2	Resources	564

5.34.2.1	Overview	564

5.34.2.2	Resource: Slice Parameters Provisionings	565

5.34.2.2.1	Introduction	565

5.34.2.2.2	Resource Definition	565

5.34.2.2.3	Resource Methods	565

5.34.2.2.3.1	GET	565

5.34.2.2.3.2	POST	566

5.34.2.2.4	Resource Custom Operations	567

5.34.2.3	Resource: Individual Slice Parameters Provisioning	567

5.34.2.3.1	Introduction	567

5.34.2.3.2	Resource Definition	567

5.34.2.3.3	Resource Methods	568

5.34.2.3.3.1	GET	568

5.34.2.3.3.2	PUT	569

5.34.2.3.3.3	PATCH	570

5.34.2.3.3.4	DELETE	571

5.34.2.3.4	Resource Custom Operations	572

5.34.3	Custom Operations without associated resources	572

5.34.4	Notifications	572

5.34.5	Data Model	572

5.34.5.1	General	572

5.34.5.2	Structured data types	573

5.34.5.2.1	Introduction	573

5.34.5.2.2	Type: SlicePpData	573

5.34.5.2.3	Type: SlicePpDataPatch	573

5.34.5.3	Simple data types and enumerations	573

5.34.5.3.1	Introduction	573

5.34.5.3.2	Simple data types	573

5.34.6	Used Features	574

5.34.7	Error handling	574

5.34.7.1	General	574

5.34.7.2	Protocol Errors	574

5.34.7.3	Application Errors	574

5.35	UeAddress API	574

5.35.1	Introduction	574

5.35.2	Resources	575

5.35.3	Custom Operations without associated resources	575

5.35.3.1	Overview	575

5.35.3.2	Operation: Retrieve	575

5.35.3.2.1	Description	575

5.35.3.2.2	Operation Definition	575

5.35.4	Notifications	576

5.35.5	Data Model	576

5.35.5.1	General	576

5.35.5.2	Structured data types	577

5.35.5.2.1	Introduction	577

5.35.5.2.2	Type: UeAddressReq	577

5.35.5.2.3	Type: UeAddressInfo	577

5.35.5.3	Simple data types and enumerations	577

5.35.5.3.1	Introduction	577

5.35.5.3.2	Simple data types	578

5.35.5.4	Data types describing alternative data types or combinations of data types	578

5.35.6	Used Features	578

5.35.7	Error handling	578

5.35.7.1	General	578

5.35.7.2	Protocol Errors	578

5.35.7.3	Application Errors	578

5.36	ECSAddress API	578

5.36.1	Introduction	578

5.36.2	Resources	579

5.36.2.1	Overview	579

5.36.2.2	Resource: ECS Address Configuration Information	580

5.36.2.2.1	Introduction	580

5.36.2.2.2	Resource Definition	580

5.36.2.2.3	Resource Standard Methods	580

5.36.2.2.3.1	Void	580

5.36.2.2.3.2	GET	580

5.36.2.2.3.3	POST	581

5.36.2.2.4	Resource Custom Operations	582

5.36.2.3	Resource: Individual ECS Address Configuration Information	582

5.36.2.3.1	Introduction	582

5.36.2.3.2	Resource Definition	582

5.36.2.3.3	Resource Standard Methods	583

5.36.2.3.3.1	Void	583

5.36.2.3.3.2	GET	583

5.36.2.3.3.3	PUT	584

5.36.2.3.3.3A	PATCH	585

5.36.2.3.3.4	DELETE	586

5.36.2.3.4	Resource Custom Operations	587

5.36.3	Custom Operations without associated resources	587

5.36.3.1	Overview	587

5.36.3.2	Operation: RemoveEcsAddr	588

5.36.3.2.1	Description	588

5.36.3.2.2	Operation Definition	588

5.36.4	Notifications	589

5.36.5	Data Model	589

5.36.5.1	General	589

5.36.5.2	Void	590

5.36.5.3	Structured data types	590

5.36.5.3.1	Introduction	590

5.36.5.3.2	Type: EcsAddrInfo	590

5.36.5.3.3	Type: EcsAddrDeleteCriteria	591

5.36.5.3.4	Type: EcsAddrInfoPatch	591

5.36.5.4	Simple data types and enumerations	591

5.36.5.4.1	Introduction	591

5.36.5.4.2	Simple data types	591

5.36.5.4	Data types describing alternative data types or combinations of data types	592

5.36.6	Used Features	592

5.36.7	Error handling	592

5.36.7.1	General	592

5.36.7.2	Protocol Errors	592

5.36.7.3	Application Errors	592

5.37	RSLPPIParametersProvisioning API	592

5.37.1	Introduction	592

5.37.2	Resources	593

5.37.2.1	Overview	593

5.37.2.2	Resource: RSLPPI Parameters Provisionings	594

5.37.2.2.1	Introduction	594

5.37.2.2.2	Resource Definition	594

5.37.2.2.3	Resource Methods	594

5.37.2.2.3.1	GET	594

5.37.2.2.3.2	POST	595

5.37.2.2.4	Resource Custom Operations	596

5.37.2.3	Resource: Individual RSLPPI Parameters Provisioning	596

5.37.2.3.1	Introduction	596

5.37.2.3.2	Resource Definition	596

5.37.2.3.3	Resource Methods	596

5.37.2.3.3.1	GET	596

5.37.2.3.3.2	PUT	597

5.37.2.3.3.3	PATCH	598

5.37.2.3.3.4	DELETE	599

5.37.2.3.4	Resource Custom Operations	600

5.37.3	Custom Operations without associated resources	600

5.37.4	Notifications	601

5.37.5	Data Model	601

5.37.5.1	General	601

5.37.5.2	Structured data types	601

5.37.5.2.1	Introduction	601

5.37.5.2.2	Type: RslppiData	601

5.37.5.2.3	Type: RslppiPpData	602

5.37.5.2.4	Type: RslppiPpDataPatch	602

5.37.5.3	Simple data types and enumerations	602

5.37.5.3.1	Introduction	602

5.37.5.3.2	Simple data types	602

5.37.5.4	Data types describing alternative data types or combinations of data types	602

5.37.6	Used Features	602

5.37.7	Error handling	603

5.37.7.1	General	603

5.37.7.2	Protocol Errors	603

5.37.7.3	Application Errors	603

6	Security	603

7	Using Common API Framework	603

7.1	General	603

7.2	Security	604

Annex A (normative):	OpenAPI representation for NEF Northbound APIs	605

A.1	General	605

A.2	TrafficInfluence API	605

A.3	NiddConfigurationTrigger API	616

A.4	AnalyticsExposure API	618

A.5	5GLANParameterProvision API	635

A.6	ApplyingBdtPolicy API	644

A.7	IPTVConfiguration API	647

A.8	LpiParameterProvision API	653

A.9	ServiceParameter API	658

A.10	ACSParameterProvision API	671

A.11	MoLcsNotify API	675

A.12	AKMA API	677

A.13	TimeSyncExposure API	679

A.14	EcsAddressProvision API	694

A.15	AMPolicyAuthorization API	698

A.16	AMInfluence API	705

A.17	MBSTMGI API	711

A.18	MBSSession API	715

A.19	EASDeployment API	725

A.20	ASTI API	731

A.21	DataReporting API	737

A.22	DataReportingProvisioning API	741

A.23	UEId API	746

A.24	MBSUserService API	753

A.25	MBSUserDataIngestSession API	757

A.26	MSEventExposure API	767

A.27	MBSGroupMsgDelivery API	771

A.28	DNAIMapping API	776

A.29	PDTQPolicyNegotiation API	780

A.30	MemberUESelectionAssistance API	786

A.31	GroupParametersProvisioning API	796

A.32	SliceParamProvision API	802

A.33	UEAddress API	807

A.34	ECSAddress API	808

A.35	RSLPPIParametersProvisioning API	814

Annex B (informative):	Change history	819