# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present specification describes the protocol for the NEF Northbound interface between the NEF and the AF. The NEF Northbound interface and the related stage 2 functional requirements are defined in 3GPP TS 23.501 [3], 3GPP TS 23.502 [2], 3GPP TS 23.316 [28], 3GPP TS 23.288 [29], 3GPP TS 23.273 [36], 3GPP TS 23.548 [42], 3GPP TS 23.247 [53], 3GPP TS 23.503 [70], 3GPP TS 33.501 [6], 3GPP TS 33.535 [37], 3GPP TS 33.558 [56], 3GPP TS 26.531 [59], 3GPP TS 26.532 [60] 3GPP TS 26.502 [65] and 3GPP TS 23.586 [76].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.502: "Procedures for the 5G system".

[3]	3GPP TS 23.501: "System Architecture for the 5G".

[4]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[5]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[6]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[7]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[8]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[9]	3GPP TS 29.521: "5G System; Binding Support Management Service; Stage 3".

[10]	Void.

[11]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[12]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[13]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[14]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[15]	Void.

[16]	Void

[17]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[18]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[19]	3GPP TS 29.554: "5G System; Background Data Transfer Policy Control Service; Stage 3".

[20]	3GPP TS 29.504: "5G System; Unified Data Repository Services; Stage 3".

[21]	3GPP TR 21.900: "Technical Specification Group working methods".

[22]	3GPP TS 29.523: "5G System; Policy Control Event Exposure Service; Stage 3".

[23]	3GPP TS 29.519: "5G System; Usage of the Unified Data Repository service for Policy Control Data, Application Data and Structured Data for Exposure; Stage 3".

[24]	3GPP TS 29.541: "5G System; Network Exposure (NE) function services for Non-IP Data Delivery (NIDD) and Short Message Services (SMS); Stage 3".

[25]	3GPP TS 29.542: "5G System, Session management services for Non-IP Data Delivery (NIDD); Stage 3".

[26]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[27]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

[28]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G system (5GS)".

[29]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[30]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[31]	Void

[32]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[33]	3GPP TS 24.588: "Vehicle-to-Everything (V2X) services in 5G System (5GS); User Equipment (UE) policies; Stage 3".

[34]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[35]	3GPP TS 29.515: "5G System; Gateway Mobile Location Services; Stage 3".

[36]	3GPP TS 23.273: "5G System Location Services (LCS)".

[37]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[38]	3GPP TS 29.535: "5G System; AKMA Anchor Services; Stage 3".

[39]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[40]	IETF RFC 7542: "The Network Access Identifier".

[41]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[42]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[43]	3GPP TS 29.534: "5G System; Access and Mobility Policy Authorization Service; Stage 3".

[44]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[45]	IEEE Std 1588-2019: "IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control".

[46]	IEEE Std 802.1AS-2020: "IEEE Standard for Local and metropolitan area networks--Timing and Synchronization for Time-Sensitive Applications".

[47]	3GPP TS 29.536: "5G System; Network Slice Admission Control Services; Stage 3".

[48]	3GPP TS 24.526: "User Equipment (UE) policies for 5G System (5GS); Stage 3".

[49]	3GPP TS 24.555: "Proximity based services (ProSe) in 5G system (5GS); User Equipment (UE) policies; Stage 3".

[50]	3GPP TS 29.565: "5G System; Time Sensitive Communication and Time Synchronization Function Services; Stage 3".

[51]	IEEE 802.1Q: "Virtual Bridged Local Area Networks".

[52]	3GPP TS 29.532: "5G System; 5G Multicast-Broadcast Session Management Services; Stage 3".

[53]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[54]	IETF RFC 6733: "Diameter Base Protocol".

[55]	3GPP TS 23.003: "Numbering, addressing and identification".

[56]	3GPP TS 33.558: "Security aspects of enhancement of support for enabling edge applications; Stage 2".

[57]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

[58]	3GPP TS 29.517: "5G System; Application Function (AF) event exposure service".

[59]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[60]	3GPP TS 26.532: "Data Collection and Reporting; Protocols and Formats".

[61]	3GPP TS 29.564: "5G System; User Plane Function Services; Stage 3".

[62]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[63]	3GPP TS 29.537: "Multicast/Broadcast Policy Control Services; Stage 3".

[64]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[65]	3GPP TS 26.502: "5G multicast–broadcast services; User Service architecture".

[66]	3GPP TS 29.580: "Multicast/Broadcast Service Function Services; Stage 3".

[67]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[68]	3GPP TS 29.543: "5G System; Data Transfer Policy Control Services; Stage 3".

[69]	3GPP TS 24.578: "Aircraft-to-Everything (A2X) services in 5G System (5GS); UE policies".

[70]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[71]	3GPP TS 26.517: "5G Multicast-Broadcast User Services; Protocols and Formats".

[72]	3GPP TS 24.514: "Ranging based services and sidelink positioning in 5G system(5GS); Stage 3".

[73]	3GPP TS 29.591: "5G System; Network Exposure Function Southbound Services; Stage 3".

[74]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[75]	GSMA PRD NG.135, version 3.0: "E2E Network Slicing Requirements".

[76]	3GPP TS 23.586: "Architectural Enhancements to support Ranging based services and Sidelink Positioning".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Application AM context: Information about the capabilities that an AF application requires from the access network for a registered UE. It is established by the AF before or during the use of the service that requires it.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.503 [70], subclause 3.1 apply:

VPLMN specific URSP rules

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

A-KID	AKMA Key IDentifier

A-TID	AKMA Temporary UE IDentifier

A2X	Aircraft-to-Everything

AAnF	AKMA Anchor Function

ACS	Auto-Configuration Server

AI/ML	Artificial Intelligence/Machine Learning

AF	Application Function

AKMA	Authentication and Key Management for Applications

AM	Access and Mobility management

ASTI	Access Stratum TIme distribution

BAT	Burst Arrival Time

BDT	Background Data Transfer

CAPIF	Common API Framework

CP	Communication Pattern

DN	Data Network

DNAI	DN Access Identifier

DNN	Data Network Name

EAS	Edge Application Server

ECS	Edge Configuration Server

EHE	Edge Hosting Environment

FQDN	Fully Qualified Domain Name

GMLC	Global Mobile Location Centre

GPSI	Generic Public Subscription Identifier

IPTV	Internet Protocol Television

KAF	AKMA Application Key

MBS	Multicast/Broadcast Service

MB-SMF	Multicast/Broadcast Session Management Function

MCC	Mobile Country Code

MNC	Mobile Network Code

MO-LR	Mobile Originated Location Request

NAT	Network Address Translation

NAPT	Network Address Port Translation

NEF	Network Exposure Function

NSAC	Network Slice Admission Control

NSACF	Network Slice Admission Control Function

PCF	Policy Control Function

PEGC	PIN Element with Gateway Capability

PCRF	Policy and Charging Rule Function

PDTQ	Planned Data Transfer with QoS requirements

PFD	Packet Flow Description

PFDF	Packet Flow Description Function

PIN	Personal IoT Network

REST	Representational State Transfer

RNAA	Resource owner-aware Northbound API Access

RSLPPI	Ranging and SideLink Positioning Privacy Indication

SCEF	Service Capability Exposure Function

SFC	Service Function Chain

S-NSSAI	Single Network Slice Selection Assistance Information

SSM	Source Specific IP Multicast address

TAI	Traffic Area Identity

TMGI	Temporary Mobile Group Identity

TNAP	Trusted Network Access Point

TSC	Time Sensitive Communication

TSCAI	Time Sensitive Communication Assistance Information

TSCTSF	Time Sensitive Communication and Time Synchronization Function

UDR	Unified Data Repository

UP	User Plane

UPF	User Plane Function

URSP	UE Route Selection Policy

WB	Wide Band

# 4	NEF Northbound Interface

## 4.1	Overview

The NEF Northbound interface is between the NEF and the AF. It specifies RESTful/RPC APIs that allow the AF to access the services and capabilities provided by 3GPP network entities and securely exposed by the NEF.

This document also specifies the procedures triggered at the NEF by API requests from the AF and by event notifications received from 3GPP network entities.

The stage 2 level requirements and signalling flows for the NEF Northbound interface are defined in 3GPP TS 23.502 [2], 3GPP TS 23.247 [53] for MBS specific aspects and 3GPP TS 26.531 [59] for data reporting provisioning and Media Streaming Event Exposure specific aspects.

The NEF Northbound interface supports the following procedures:

1)	Procedures for Monitoring.

2)	Procedures for Device Triggering.

3)	Procedures for resource management of Background Data Transfer.

4)	Procedures for CP Parameters, Network Configuration Parameters Provisioning, 5G LAN Parameters Provisioning, ACS Configuration Parameter Provisioning, Location Privacy Indication Parameters Provisioning, ECS address provisioning, Slice Parameters Provisioning, DNN and S-NSSAI specific Group Parameters provisioning and Ranging and SideLink Positioning Privacy Indication (RSLPPI) Parameters Provisioning.

5)	Procedures for PFD Management.

6)	Procedures for Traffic Influence.

7)	Procedures for changing the chargeable party at session set up or during the session.

8)	Procedures for AF required QoS.

9)	Procedures for MSISDN-less Mobile Originated SMS.

10)	Procedures for non-IP data delivery.

11)	Procedures for analytics information exposure.

12)	Procedure for applying BDT policy.

13)	Procedures for Enhanced Coverage Restriction Control.

14)	Procedures for IPTV Configuration.

15)	Procedures for Service Parameter Provisioning.

16)	Procedures for RACS Parameter Provisioning.

17)	Procedures for Mobile Originated Location Request.

18)	Procedures for AKMA.

19)	Procedures for AF triggered Access and Mobility Influence.

20)	Procedures for AF triggered Access and Mobility Policy Authorization.

21)	Procedures for Time Synchronization Exposure.

22)	Procedures for EAS Deployment information provisioning.

23)	Procedures for TMGI allocation, deallocation, expiry timer refresh and timer expiry notification.

24)	Procedures for MBS session management and MBS parameters provisioning.

25)	Procedures for Data Reporting.

26)	Procedures for Data Reporting Provisioning.

27)	Procedures for AF specific UE ID retrieval.

28)	Procedures for Media Streaming Event Exposure.

29)	Procedures for MBS User Service management.

30)	Procedures for MBS User Data Ingest Session management.

31)	Procedures for MBS Group Message Delivery management.

32)	Procedures for DNAI mapping.

33)	Procedures for negotiation of Planned Data Transfer with QoS requirements.

34)	Procedures for Member UE Slection Assistance.

37)	Procedures for UE Address retrieval.

38)	Procedures for ECS Address configuration in roaming.

Which correspond to the following services respectively, supported by the NEF as defined in 3GPP TS 23.502 [2], 3GPP TS 23.247 [53] or 3GPP TS 26.531 [59]:

1)	Nnef_EventExposure service and Nnef_APISupportCapability service.

2)	Nnef_Trigger service.

3)	Nnef_BDTPNegotiation service.

4)	Nnef_ParameterProvision service.

5)	Nnef_PFDManagement service.

6)	Nnef_TrafficInfluence service.

7)	Nnef_ChargeableParty service.

8)	Nnef_AFsessionWithQoS service and Nnef_AF_Request_for_QoS service.

9)	Nnef_MSISDN-less_MO_SMS service.

10)	Nnef_NIDDConfiguration and Nnef_NIDD services.

11)	Nnef_AnalyticsExposure service.

12)	Nnef_ApplyPolicy service.

13)	Nnef_ECRestriction service.

14)	Nnef_IPTVConfiguration service.

15)	Nnef_ServiceParameter service.

16)	Nnef_UCMFProvisioning service.

17)	Nnef_Location service.

18)	Nnef_AKMA service.

19)	Nnef_AMInfluence service.

20)	Nnef_AMPolicyAuthorization service.

21)	Nnef_TimeSynchronization and Nnef_ASTI services.

22)	Nnef_EASDeployment service.

23)	Nnef_MBSTMGI service.

24)	Nnef_MBSSession service.

25)	Nnef_DataReporting service.

26) Nnef_DataReportingProvisioning service.

27)	Nnef_UEId service.

28)	Nnef_MSEventExposure service.

29)	Nnef_MBSUserService service.

30)	Nnef_MBSUserDataIngestSession service.

31)	Nnef_MBSGroupMsgDelivery service.

32)	Nnef_DNAIMapping service.

33)	Nnef_PDTQPolicyNegotiation service.

34)	Nnef_MemberUESelectionAssistance service.

37)	Nnef_UEAddress service.

38)	Nnef_ECSAddress service.

NOTE 1:	For the Nnef_PFDManagement service, only the Nnef_PFDManagement_Create/Update/Delete service operations are applicable for the NEF Northbound interface.

NOTE 2:	For the Nnef_NIDD service, NF consumer other than the AF does not use the NEF Northbound interface.

NOTE 3:	For the Nnef_NIDDConfiguration service, the Nnef_NIDDConfiguration_Trigger service operation is only applicable for the NEF Northbound interface.

NOTE 4:	The Nnef_APISupportCapability service is only applicable in the MonitoringEvent API when the monitoring type is set to "API_SUPPORT_CAPABILITY".

NOTE 5:	The Nnef_MSEventExposure service maps to the Nnef_EventExposure service and is applicable for the case where the event consumer AF in the Application Service Provider is deployed outside the trusted domain, as described in 3GPP TS 26.531 [59], and the subscribed event is set to "MS_QOE_METRICS", "MS_CONSUMPTION", "MS_NET_ASSIST_INVOCATION", "MS_DYN_POLICY_INVOCATION", or "MS_ACCESS_ACTIVITY".

NOTE 6:	The stage 2 Nnef_AF_Request_for_QoS API is defined by reusing the Nnef_AFsessionWithQoS API together with the support of the "GMEC" feature.

## 4.2	Reference model

The NEF Northbound interface resides between the NEF and the AF as depicted in figure 4.2.1. The overall NEF architecture is depicted in 3GPP TS 23.502 [2]. An AF can get services from multiple NEFs, and an NEF can provide services to multiple AFs.

NOTE:	The AF can be provided by a third party.

Figure 4.2-1: Reference Architecture for the Nnef Service; SBI representation

Figure 4.2-2: Reference Architecture for the Nnef Service; reference point representation

## 4.3	Functional elements

### 4.3.1	NEF

The Network Exposure Function (NEF) is a functional element that supports the related stage 2 functional requirements defined in the technical specifications listed in clause 1 and that are implemented via the procedures specified in clause 4.4. A specific NEF instance may support one or more of these functionalities, and consequently, an individual NEF may support one, several or all of the APIs specified for capability exposure.

In addition, the NEF shall also:

-	securely expose network capabilities and events provided by 3GPP NFs to AF;

-	provide means for the AF to securely provide information to 3GPP network and may authenticate, authorize and assist in throttling the AF;

-	be able to translate the information received from the AF to the one sent to internal 3GPP NFs, and vice versa; and

-	support to expose information (collected from other 3GPP NFs) to the AF.

NOTE:	The NEF can access the UDR located in the same PLMN as the NEF.

### 4.3.2	AF

The Application Function (AF) may interact with the 3GPP Core Network via the NEF in order to access network capabilities.

## 4.4	Procedures over NEF Northbound Interface

### 4.4.1	Introduction

All procedures that operate across the NEF Northbound interface, as specified in 3GPP TS 23.502 [2], and in 3GPP TS 23.247 [53] for MBS specific aspects, are specified in the following clauses.

### 4.4.2	Procedures for Monitoring

The procedures and provisions for event monitoring defined in clause 4.4.2 of 3GPP TS 29.122 [4] shall be applicable in 5GS with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	description of the HSS applies to the UDM, and the NEF shall interact with the UDM by using Nudm_EventExposure service as defined in 3GPP TS 29.503 [17];

-	description of the MME/SGSN applies to the AMF, the NEF shall resolve a location area to the involved AMF(s) either by local configuration or via the NRF and the NEF shall interact with the AMF by using the Namf_EventExposure service as defined in 3GPP TS 29.518 [18];

-	description about the PCRF is not applicable;

-	description about the change of IMSI-IMEI(SV) association monitoring event applies to the change of SUPI-PEI association monitoring event;

-	when the "monitoringType" sets to "LOCATION_REPORTING" within the MonitoringEventSubscription data type as defined in clause 5.3.2.1.2 of 3GPP TS 29.122 [4] during the monitoring event subscription, only "CGI_ECGI", "TA_RA", "GEO_AREA" and "CIVIC_ADDR" within the Accuracy data type, as defined in clause 5.3.2.4.7 of 3GPP TS 29.122 [4], are applicable for 5G event monitoring using the MonitoringEvent API;

-	after validation of the AF request, the NEF may determine a monitoring expiry time, based on operator policy and take into account the monitoring expire time if included in the request; and the NEF may provide an expiry time (determined by the NEF, UDM or AMF) to the AF even the AF does not provided before;

-	if the "Loss_of_connectivity_notification" feature as defined in clause 5.3.4 of 3GPP TS 29.122 [4] is supported, values 0-5 are not applicable for the lossOfConnectReason attribute within MonitoringEventReport data type, the lossOfConnectReason attribute shall be set to 6 if the UE is deregistered, 7 if the maximum detection timer expires, 8 if the UE is purged or 9 if the UE’s Unavailability Period Duration is available and the "Loss_of_connectivity_notification_5G" feature as defined in clause 5.3.4 of 3GPP TS 29.122 [4] is supported;

-	the AF may include a periodic reporting time indicated by the "repPeriod" attribute within MonitoringEventSubscription data type, which is only applicable for the "Location_notification", "Number_of_UEs_in_an_area_notification_5G" and "NSAC" features in the NEF;

-	if the "locationType" attribute sets to "LAST_KNOWN_LOCATION", the "maximumNumberOfReports" attribute shall set to 1 as a One-time Monitoring Request;

-	description about the PDN connectivity status event apply to the PDU session status event, the description of the MME/SGSN applies to the SMF during the reporting of monitoring event procedure, the NEF receives the event notification via Nsmf_EventExposure service as defined in 3GPP TS 29.508 [26];

-	if the "Session_Management_Enhancement" feature as defined in clause 5.3.4 of 3GPP TS 29.122 [4] is supported, the "dnn"and/or "snssai" may be provided in MonitoringEventSubscription data type for monitoring type provided "PDN_CONNECTIVITY_STATUS" or " DOWNLINK_DATA_DELIVERY_STATUS";

-	when sending the UDM/AMF/SMF event report to the AF, the NEF may store the event data in the report in the UDR as part of the data for exposure as specified in 3GPP TS 29.519 [23] by using Nudr_DataRepository service as specified in 3GPP TS 29.504 [20];

-	if the "Downlink_data_delivery_status_5G" feature as defined in clause 5.3.4 of 3GPP TS 29.122 [4] is supported, in order to support the downlink data delivery status notification;

1)	the AF shall send an HTTP POST message to the NEF to the resource "Monitoring Event Subscriptions" as defined in clause 5.3.3.2 of 3GPP TS 29.122 [4] for creating a subscription or send an HTTP PUT/PATCH message to the NEF to the resource "Individual Monitoring Event Subscription" defined in clause 5.3.3.3 of 3GPP TS 29.122 [4] for updating the subscription as follows:

A)	within the MonitoringEventSubscription data structure (or the requested modifications to the resource representation in case the HTTP PATCH method is used and the "Subscription_Patch" feature defined in clause 5.3.4 of 3GPP TS 29.122 [4] is supported), the AF may additionally include packet filter descriptor(s) within the "dddTraDescriptors" attribute and the list of monitoring downlink data delivery status event(s) within the "dddStati" attribute; and

B)	the NEF shall subscribe the events to the appropriate UDM(s) within the network by invoking the Nudm_EventExposure_Subscribe service operation as defined in clause 5.5.2.2 of 3GPP TS 29.503 [17];

2)	if the "Partial_group_modification" feature as defined in clause 5.3.4 of 3GPP TS 29.122 [4] is supported, in order to support partial cancellation or addition of certain UE(s) within the active group event subscription, the NEF shall map the "excludedExternalIds" and/or "excludedMsisdns" attributes to the "excludeGpsiList" attribute for the partial group cancellation, or shall map the "addedExternalIds" and/or "addedMsisdns" attributes to the "includeGpsiList" attribute within the Nudm_EventExposure service; and

3)	when the NEF receives the event notification as defined in clause 4.4.2 of 3GPP TS 29.508 [26], the NEF shall send an HTTP POST message to the AF as defined in clause 4.4.2.3 of 3GPP TS 29.122 [4] with the difference that within each MonitoringEventReport data structure, the NEF shall include:

A)	the downlink data delivery status within the "dddStatus" attribute;

B)	the downlink data descriptor impacted by the downlink data delivery status change within the "dddTraDescriptor" attribute;

C)	the estimated buffering time within the "maxWaitTime" attribute if the downlink data delivery status is set to "BUFFERED"; and

D)	if the "Availability_after_DDN_failure_notification_enhancement" feature as defined in clause 5.3.4 of 3GPP TS 29.122 [4] is supported, the AF shall send an HTTP POST message to the NEF to the resource "Monitoring Event Subscriptions" as defined in clause 5.3.3.2 of 3GPP TS 29.122 [4] for creating an subscription or send an HTTP PUT/PATCH message to the NEF to the resource "Individual Monitoring Event Subscription" as defined in clause 5.3.3.3 of 3GPP TS 29.122 [4] for updating the subscription with the difference that within the MonitoringEventSubscription data structure (or the requested modifications to the resource representation in case the HTTP PATCH method is used and the "Subscription_Patch" feature defined in clause 5.3.4 of 3GPP TS 29.122 [4] is supported), the AF shall include packet filter descriptions within the "dddTraDescriptors" attribute;

-	if the "eLCS" feature as defined in clause 5.3.4 of 3GPP TS 29.122 [4] is supported, the AF may send an HTTP POST message to the NEF to the resource "Monitoring Event Subscriptions" as defined in clause 5.3.3.2 of 3GPP TS 29.122 [4] for creating an subscription or send an HTTP PUT/PATCH message to the NEF to the resource "Individual Monitoring Event Subscription" defined in clause 5.3.3.3 of 3GPP TS 29.122 [4] for updating the subscription as follows:

1)	within the MonitoringEventSubscription data structure (or the requested modifications to the resource representation in case the HTTP PATCH method is used and the "Subscription_Patch" feature defined in clause 5.3.4 of 3GPP TS 29.122 [4] is supported), the AF may additionally include location QoS requirement within the "locQoS" attribute, the service identifier within the "svcId" attribute, Location deferred requested event type within the "ldrType" attribute, the validity start time and the validity end time within the "locTimeWindow" attribute, the maximum age of location estimate within the "maxAgeOfLocEst" attribute, the requesting target UE velocity within the "velocityRequested" attribute, the linear distance within the "linearDistance" attribute, the reporting target UE location estimate indication within the "reportingLocEstInd" attribute, the sampling interval within the "samplingInterval" attribute, the maximum reporting expire interval within the "maxRptExpireIntvl" attribute, the supported GAD shapes within the "supportedGADShapes" attribute, the Code word within the "codeword" attribute, and other attributes as defined in clause 5.3.2.3.2 of 3GPP TS 29.122 [4] for location information subscription; The MonitoringEventSubscription data structure (or the requested modifications to the resource representation in case the HTTP PATCH method is used and the "Subscription_Patch" feature defined in clause 5.3.4 of 3GPP TS 29.122 [4] is supported) may also include the "locationArea5G" attribute containing only the "geographicAreas" attribute and the "accuracy" attribute set to the value "GEO_AREA". The "accuracy" attribute and "locQoS" attribute are mutually exclusive. If the "MULTIQOS" feature is also supported, Multiple QoS Class is supported in the "lcsQosClass" attribute within the "locQoS" attribute for deferred MT-LR. If the "eLCS_en" feature is also supported, the AF may include the "upLocRepIndAf" attribute to indicate whether or not location reporting over user plane is required, and may also include the "upLocRepAddrAf" attribute to convery the AF's addressing information for location reporting over user plane. If the "Ranging_SL" feature as defined in clause 5.3.4 of 3GPP TS 29.122 [4] is also supported, the AF may additionally include the ranging and sidelink positioning result(s) type within the "reqRangSlRes" attribute, the list of the related UE(s) for the ranging and sidelink positioning and the corresponding information within the "relatedUEs" attribute within the MonitoringEventSubscription data structure (or the requested modifications to the resource representation in case the HTTP PATCH method is used and the "Subscription_Patch" feature defined in clause 5.3.4 of 3GPP TS 29.122 [4] is supported);

2)	if the NEF identifies the location request precision higher than cell level location accuracy is required based on the "locQoS" attribute received, the NEF shall interact with the appropriate GMLC within the network by invoking the Ngmlc_Location_ProvideLocation service operation as defined in clause 6.1 of 3GPP TS 29.515 [35];

3)	if the location request precision is lower than or equal to cell level, based on implementation, the NEF may interact with the GMLC by invoking the Ngmlc_Location_ProvideLocation service operation as defined in clause 6.1 of 3GPP TS 29.515 [35]; or retrieve the UE location privacy information from the UDM by using Nudm_SDM service as described in clause 5.2 of 3GPP TS 29.503 [17] and if the privacy setting is verified, the NEF shall interact with the UDM for the serving AMF address by invoking the Nudm_UECM service as described in clause 5.3 of 3GPP TS 29.503 [17]. After receiving the serving AMF address from the UDM, the NEF shall interact with the AMF by invoking the Namf_EventExposure_Subscribe service operation as defined in clause 5.3 of 3GPP TS 29.518 [18]; or may interact with UDM by using Nudm_EventExposure service as defined in clause 5.5 of 3GPP TS 29.503 [17] and the NEF receives the location event notification from the AMF via Namf_EventExposure service as defined in in clause 5.5 of 3GPP TS 29.518 [18];

4)	based on the received AF information and local authorization policy, the NEF shall derive the LCS client type with a suitable enumeration value for the AF location request, to be provided as the "externalClientType" attribute when invoking the Ngmlc_Location_ProvideLocation service operation as defined in clause 6.1 of 3GPP TS 29.515 [35];

5)	upon receipt of successful location response from the GMLC or the AMF or the UDM, the NEF shall create or update the "Individual Monitoring Event Subscription" resource and then send an HTTP POST or PUT/PATCH response to the AF as defined in clause 4.4.2.2 of 3GPP TS 29.122 [4]. Upon receipt of the location Report from the GMLC or the AMF, the NEF shall determine the monitoring event subscription associated with the corresponding Monitoring Event Report as defined in clause 4.4.2.3 of 3GPP TS 29.122 [4]; and

6)	in order to delete a previous active configured monitoring event subscription at the NEF:

-	the AF shall send either:

-	an HTTP DELETE request message to the NEF targeting the corresponding "Individual Monitoring Event Subscription" resource which is received in the response to the request that has created the monitoring events subscription resource; or

-	an HTTP PUT/PATCH request to the NEF targeting the corresponding "Individual Monitoring Event Subscription" resource to remove the subscription to concerned event(s) from the list of subscribed event(s) together with the related information, as defined in clauses 5.3.3.3.3.2 and 5.3.3.3.3.3 of 3GPP TS 29.122 [4];

-	the NEF shall interact with the GMLC, the AMF or the UDM to remove the corresponding events reporting; and

-	upon reception of the successful response from the GMLC, the AMF or the UDM, the NEF shall delete or update accordingly the corresponding "Individual Monitoring Event Subscription" resource and send an HTTP response to the AF with either an HTTP "204 No Content" status code, or an HTTP "200 OK" status code including the monitoring event report if received (in case of resource deletion) or the updated representation of the resource (in case of resource update);

-	based on local regulations' requirements and operator policies, user consent management specified in Annex V of 3GPP TS 33.501 [6] may be required for EDGE applications to access the Nnef_EventExposure API for UE's location retrieval. When it is the case and the NEF is used by the Edge Enabler Layer entities to access 3GPP 5GC services, the NEF acts as the consent enforcement entity, as specified in clause 5.1.3 of 3GPP TS 33.558 [56];

-	when user consent management shall be carried out for EDGE applications, then:

1)	if the AF (e.g. Edge Enabler Server) does not support the "UserConsentRevocation" feature or does not indicate its support for this feature in the HTTP POST request to create a new "Individual Monitoring Event Subscription" resource with the "monitoringType" attribute set to "LOCATION_REPORTING", the NEF shall reject the request and respond to the AF with an HTTP "403 Forbidden" status code with the response body including a ProblemDetails data structure containing the "CONSENT_REVOCATION_NOT_SUPPORTED" application error within the "cause" attribute;

2)	if the AF indicates its support for the "UserConsentRevocation" feature in the HTTP POST request to create a new "Individual Monitoring Event Subscription" resource with the "monitoringType" attribute set to "LOCATION_REPORTING", the NEF shall check user consent for the targeted UE(s) by retrieving the user consent subscription data via the Nudm_SDM service API of the UDM as specified in clause 5.2.2.2.24 of 3GPP TS 29.503 [17], subscribe to user consent revocation notifications only for those UE(s) for which user consent is granted also using the Nudm_SDM service API of the UDM and accept the request for the creation of the event monitoring subscription only for the UE(s) for which user consent is granted;

3)	if user consent is not granted for all the targeted UE(s), the NEF shall reject the request and respond to the AF with an HTTP "403 Forbidden" status code with the response body including a ProblemDetails data structure including the "USER_CONSENT_NOT_GRANTED" application error within the "cause" attribute;

4)	the AF shall provide within the HTTP POST request to create a new event monitoring subscription the URI via which it desires to receive user consent revocation notifications within the "revocationNotifUri" attribute. The AF may update this URI in subsequent HTTP PUT/PATCH requests to update/modify the corresponding "Individual Monitoring Event Subscription" resource;

5)	when becoming aware of user consent revocation for one or several UE(s), the NEF shall:

A)	stop processing the data related to the concerned UE(s);

B)	send a user consent revocation notification to the AF by sending an HTTP POST request with the request body including the ConsentRevocNotif data structure that shall contain the user consent revocation information (e.g. UE(s) for which user consent was revoked, etc.);

C)	remove the concerned UE(s) from the corresponding "Individual Monitoring Event Subscription" resource and from the related subscriptions at the GMLC, if any; and

D)	unsubscribe from user consent revocation notifications for the concerned UE(s) at the UDM;

6)	at the reception of the user consent revocation notification from the NEF, the AF shall take the necessary actions to stop processing the data related to the UE(s) for which user consent was revoked; and

7)	if user consent is revoked for all the UE(s), the AF shall delete the corresponding "Individual Monitoring Event Subscription" resource as specified above in this clause;

-	if the "NSAC" feature defined in clause 5.3.4 of 3GPP TS 29.122 [4] is supported, in order to support network slice status reporting:

1)	the AF shall send an HTTP POST request to the NEF targeting the "Monitoring Event Subscriptions" resource to create a subscription, as defined in clause 5.3.3.2.3.4 of 3GPP TS 29.122 [4], or send an HTTP PUT/PATCH request targeting the NEF to the "Individual Monitoring Event Subscription" resource to update an existing subscription, as defined in clauses 5.3.3.3.3.2 and 5.3.3.3.3.3 of 3GPP TS 29.122 [4], as follows:

A)	within the MonitoringEventSubscription data structure (or the requested modifications to the MonitoringEventSubscription data structure in case the HTTP PATCH method is used):

a)	either the concerned network slice identified by the "snssai" attribute, in the case of a trusted AF, or the AF service identifier within the "afServiceId" attribute, in the case of an untrusted AF, shall be provided;

b)	the value of the "monitoringType" attribute shall be set to either "NUM_OF_REGD_UES" or "NUM_OF_ESTD_PDU_SESSIONS";

c)	the "maximumNumberOfReports" attribute set to a value of 1 shall be provided, if one-time reporting of the current network slice status information is requested;

d)	the "immediateRep" attribute shall be set to "true", if immediate reporting of the current network slice status information is requested or one-time reporting of the current network slice status information is requested;

e)	if one-time reporting is not requested, either a targeted reporting threshold within the "tgtNsThreshold" attribute (if threshold-based reporting is requested) or a reporting periodicity within the "repPeriod" attribute (if periodic reporting is requested) shall be provided;

f)	if periodic reporting or one-time reporting is requested, the "nsRepFormat" attribute shall be provided to indicate the requested reporting format (i.e. numerical or percentage); and

2)	if needed, the NEF shall then further interact with the concerned NSACF(s) to create or update the associated subscription(s) to notifications by invoking the Nnsacf_SliceEventExposure_Subscribe service operation, as specified in 3GPP TS 29.536 [47];

3)	if an AF service identifier was provided by the AF (case of an untrusted AF), the NEF shall translate it to the corresponding S-NSSAI prior to sending the request(s) to the concerned NSACF(s);

NOTE 1:	There can be a single or multiple NSACF(s) deployed in a network as specified in clause 5.15.11 of 3GPP TS 23.501 [3]. Whether the NEF needs to interact with one or multiple NSACF(s) to establish and manage network slice status reporting depends on the deployed NSAC architecture option (cf. clause 4.15.3.2.10 of 3GPP TS 23.502 [2] and clause 5.15.11 of 3GPP TS 23.501 [3]).

NOTE 2:	If multiple NSACFs need to be contacted by the NEF to establish and manage network slice status reporting for the requested S-NSSAI, the NEF can set the event reporting type to periodic in its request(s) to these NSACFs, irrespective of the requested reporting type by the AF (i.e., threshold based reporting or periodic reporting).

4)	after receiving a successful response from the concerned NSACF(s), the NEF shall:

A)	for the HTTP POST request, respond to the AF as defined in clause 5.3.3.2.3.4 of 3GPP TS 29.122 [4] with either;

a)	an HTTP "201 Created" status code with the response body containing the created "Individual Monitoring Event Subscription" resource within the MonitoringEventSubscription data structure. The NEF shall include the current network slice status information received from the NSACF(s) within the "monitoringEventReport" attribute, if available and the "immediateRep" attribute was provided and set to "true" in the corresponding request; or

b)	an HTTP "200 OK" status code with the response body containing the current network slice status information received from the NSACF(s) within the MonitoringEventReport data structure, if the corresponding request is a one-time reporting request with the "immediateRep" attribute set to "true";

B)	for the HTTP PUT/PATCH request, respond to the AF with either:

-	an HTTP "200 OK" status code, as defined in clause 5.3.3.3.3.2 of 3GPP TS 29.122 [4], and the response body including the MonitoringEventSubscription data structure containing a representation of the updated "Individual Monitoring Event Subscription" resource. The NEF shall include the current network slice status information received from the NSACF(s) within the "monitoringEventReport" attribute, if available and the "immediateRep" attribute was provided and set to "true" in the request; or

-	an HTTP "204 No Content" status code;

NOTE 3:	When the "maximumNumberOfReports" attribute is provided and set to a value of 1 and the "immediateRep" attribute is provided and set to "true", the "Individual Monitoring Event Subscription" is immediately terminated after returning the current network slice status information in the HTTP POST response body.

NOTE 4:	When the "maximumNumberOfReports" attribute is either not present or present and set to a value above 1 and/or the "immediateRep" attribute is either not present or present and set to "false", the above steps 2 and 3 may occur after step 4, i.e., the NEF may acknowledge/respond to the request and create/update the "Individual Monitoring Event Subscription" resource accordingly before interacting with the the concerned NSACF(s).

NOTE 5:	After sending a subscription creation request for network slice status reporting with a particular reporting format (e.g., percentage) for periodic reporting, an AF cannot send a subsequent subscription creation request for the same network slice with a different reporting format (e.g., numerical) for periodic reporting.

5)	when the NEF receives event report(s) from the NSACF(s) as defined in 3GPP TS 29.536 [47], the NEF shall notify the AF via an HTTP POST request message (as defined in clause 5.3.3A.2.3 of 3GPP TS 29.122 [4]) as follows:

A)	within the MonitoringEventReport data structure of the MonitoringNotification data structure:

a)	the value of the "monitoringType" attribute shall be set to "NUM_OF_REGD_UES" or "NUM_OF_ESTD_PDU_SESSIONS" (i.e., the same value received in the HTTP POST or PUT/PATCH request that created or updated the subscription);

b)	the AF service identifier to which the notification is related, within the "afServiceId" attribute, if it was provided by the AF in the corresponding subscription request; and

c)	the current network slice status information within the "nSStatusInfo" attribute shall be provided, wherein:

i)	if the event reporting is threshold based (i.e., the "tgtNsThreshold" was provided within the MonitoringEventSubscription data type), the "nSStatusInfo" attribute shall contain a confirmation for reaching the targeted threshold value, i.e., by sending the current number of registered UEs, or if "eNSAC" feature is also supported, the current number of UEs with at least one PDU session/PDN connection, or the current number of established PDU Sessions, for the network slice identified by the "snssai" attribute provided during the corresponding subscription creation/update; and

ii)	if the event reporting is periodical (i.e., the "repPeriod" was provided within the MonitoringEventSubscription data type), the "nSStatusInfo" attribute shall provide the current network slice status information, i.e., the current number of registered UEs, or if "eNSAC" feature is also supported, the current number of UEs with at least one PDU session/PDN connection, or the current number of established PDU Sessions, for the network slice identified by the "snssai" attribute provided during the corresponding subscription creation/update;

NOTE 6:	The handling of threshold-based notifications is described in clause 4.15.3.2.10 of 3GPP TS 23.502 [2].

NOTE 7:	If the NEF interacts with multiple NSACFs for the requested S-NSSAI, the NEF performs the aggregation of the received network slice status reports from all these NSACFs and determines based on that whether a notification towards the subscribing AF needs to be sent or not (i.e., the reporting conditions to trigger a notification towards the AF are fulfilled or not).

and

6)	in order to unsubscribe from network slice status reporting:

A)	the AF shall either:

a)	send an HTTP DELETE request to the NEF targeting the corresponding "Individual Monitoring Event Subscription" resource, as defined in clause 5.3.3.3.3.5 of 3GPP TS 29.122 [4]; or

b)	send an HTTP PUT/PATCH request to the NEF targeting the corresponding "Individual Monitoring Event Subscription" resource to remove the subscription to network slice status reporting related event(s) from the list of subscribed events together with the related information, as defined in clauses 5.3.3.3.3.2 and 5.3.3.3.3.3 of 3GPP TS 29.122 [4];

and

B)	if needed, the NEF shall interact with the concerned NSACF(s) to delete the associated subscription(s) to notifications by invoking the Nnsacf_SliceEventExposure_Unsubscribe service operation as specified in 3GPP TS 29.536 [47];

-	if the "enNB1_5G" feature defined in clause 5.3.4 of 3GPP TS 29.122 [4] is supported, then:

-	the AF may require immediate reporting for the subscribed event(s) by providing the "immediateRep" attribute set to "true" within the MonitoringEventSubscription data structure in the corresponding subscription creation/update request; and

-	if there are available report(s) for the subscribed event(s) at the NEF, the corresponding subscription creation/update response shall contain these available report(s) within the "monitoringEventReport" attribute, and/or if the "enNB" feature is supported, the "addnMonEventReports" attribute, of the MonitoringEventSubscription data structure;

-	if the "UEId_retrieval" feature defined in clause 5.3.4 of 3GPP TS 29.122 [4] is supported, in order to support AF specific UE ID retrieval:

1)	the AF may request AF specific UE ID retrieval for an individual UE, by providing the UE's IP address in the "ueIpAddr" attribute or the UE's MAC address in the "ueMacAddr" attribute within the MonitoringEventSubscription data type;

2)	the AF may also provide the DNN, within the "dnn" attribute, and/or the S-NSSAI, within the "snssai" attribute, in the MonitoringEventSubscription data type;

3)	upon reception of the corresponding subscription request message from the AF, the NEF shall check whether the AF is authorized to perform this operation or not:

-	if the AF's request for AF specific UE ID retrieval is not authorized, the NEF shall respond to the AF with a "403 Forbidden" status code with the response body including the ProblemDetails data structure containing the "cause" attribute set to the "REQUEST_NOT_AUTHORIZED" application error indicating AF authorisation failure; and

-	if the AF request is for AF specific UE ID retrieval authorized by the NEF, then if the DNN and/or S-NSSAI information is not available in the request, the NEF shall determine the corresponding DNN and/or S-NSSAI information based on the received requesting AF Identifier, and if provided, the MTC Provider Information;

4)	the NEF shall then interact with the BSF using the UE address and IP domain (if the UE IPv4 address is provided), DNN and/or S-NSSAI to retrieve the session binding information of the UE by invoking the Nbsf_Management_Discovery service operation as described in 3GPP TS 29.521 [9];

5)	if the NEF receives an error response from the BSF, the NEF shall respond to the AF with a proper error status code. If the NEF received from the BSF an error response including a "ProblemDetails" data structure with the "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error. If no SUPI matching the provided UE information is returned by the BSF, the NEF shall respond to the AF with a "404 Not Found" status code with the response body including a ProblemDetails data structure containing the "cause" attribute set to the "UE_NOT_FOUND" application error to indicate that the requested UE address is not found;

6)	upon success and a SUPI is returned by the BSF, the NEF shall then interact with the UDM to retrieve the AF specific UE Identifier using the received SUPI and at least one of the Application Port ID, MTC Provider Information or AF Identifier information by invoking Nudm_SDM_Get service as described in clause 5.2.2.2 of 3GPP TS 29.503 [17];

7)	upon success, the UDM responds to the NEF with an AF specific UE Identifier represented as an External Identifier for the UE which is uniquely associated with the MTC provider Information and/or AF Identifier. The NEF shall then respond to the AF with the received information, i.e. the AF specific UE Identifier represented as an External Identifier that was received from the UDM;

8)	if the NEF receives an error response from the UDM, the NEF shall respond to the AF with a proper error status code. If the NEF received from the UDM an error response including a "ProblemDetails" data structure with the "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error. If the UDM indicates that the requested UE Identifier is not available in the subscription data, the NEF shall respond to the AF with a "404 Not Found" error status code with the response body including a ProblemDetails data structure containing the "cause" attribute set to the "UE_ID_NOT_AVAILABLE" application error to indicate that the AF specific UE ID is not available;

NOTE 8:	The case where the UE's IP address provided by the AF to the NEF corresponds to an IP address that has been NATed (Network and Port Address Translation) is not supported in this release of the specification.

-	if the "GMEC" feature defined in clause 5.3.4 of 3GPP TS 29.122 [4] is supported, in order to support group status change event reporting (e.g., the group member list is updated to add new group member(s) or remove existing group member(s)):

-	the AF shall send an HTTP POST request to the NEF targeting the "Monitoring Event Subscriptions" collection resource defined in clause 5.3.3.2.3.4 of 3GPP TS 29.122 [4] to request the creation of a subscription with the the request body including the MonitoringEventSubscription data structure that shall contain:

-	the external group identifier, to identify the targeted group (e.g. 5G VN group), within the "externalGroupId" attribute; and

-	the "monitoringType" attribute set to "GROUP_MEMBER_LIST_CHANGE" to indicate that the AF requests to be notified of the Group Members List changes event reporting;

-	the AF may also update/modify an existing subscription to add group status change reporting event(s) to the list of monitored event(s) or update/modify its properties by sending and an HTTP PUT/PATCH request to the NEF targeting the corresponding "Individual Monitoring Event Subscription" resource, as defined in clause 5.3.3.3.3.2/5.3.3.3.3.3 of 3GPP TS 29.122 [4], including the above-mentioned attributes when relevant;

-	the NEF shall then further interact with the UDM to create or update the associated subscription(s) to notifications by invoking the relevant service operations of the Nudm_EventExposure API as specified in 3GPP TS 29.503 [17];

-	upon reception of a successful response from the UDM, the NEF shall respond to the AF as defined in clause 5.3.3.2.3.4, 5.3.3.3.3.2 or 5.3.3.3.3.3 of 3GPP TS 29.122 [4];

-	when the NEF receives group status List change event report(s) via notification(s) from the UDM as defined in 3GPP TS 29.503 [17], the NEF shall in turn notify the AF by sending an HTTP POST request message as defined in clause 5.3.3A.2.3 of 3GPP TS 29.122 [4] as follows:

-	within an array element of the "monitoringEventReports" attribute (encoded via the MonitoringEventReport data structure) of the MonitoringNotification data type:

-		the "monitoringType" attribute shall be set to "GROUP_MEMBER_LIST_CHANGE" (i.e., the same value received during the HTTP POST or PUT/PATCH request that created or updated/modified the subscription); and

-	the information on the change(s) to the group members list shall be provided within the "groupMembListChanges" attribute;

and

-	in order to unsubscribe from group status change event(s) reporting:

-	if the AF subscribed to other monitoring event(s) in addition to the group status change event(s) reporting, the AF shall update/modify the corresponding subscription to remove the group status change event(s) reporting from the list of the subscribed monitoring event(s);

-	if the AF subscribed only to group status change reporting event(s) or the AF desires to unsubscribe from all the monitoring event(s) that it has subscribed to via this monitoring event subscription, then:

-	the AF shall send an HTTP DELETE (or PUT/PATCH) request message to the NEF targeting the corresponding "Individual Monitoring Event Subscription" resource, as defined in clause 5.3.3.3 of 3GPP TS 29.122 [4], to request the deletion (or update) of the related existing "Individual Monitoring Event Subscription" resource;

-	the NEF shall then interact with the UDM to request the deletion of the associated subscription(s) by invoking the relevant service operation of the Nudm_EventExposure API as specified in 3GPP TS 29.503 [17]; and

-	upon reception of a successful response from the UDM, the NEF shall delete (or update accordingly) the targeted subscription and respond to the AF as defined in clause 5.3.3.3 of 3GPP TS 29.122 [4];

-	if the "AppDetection_5G" feature defined in clause 5.3.4 of 3GPP TS 29.122 [4] is supported, in order to support application traffic detection (e.g., start/stop of application traffic) monitoring event(s) reporting:

-	the AF shall send either:

-	an HTTP POST request to the NEF targeting the "Monitoring Event Subscriptions" resource to request the creation of a subscription as defined in clause 5.3.3.2.3.4 of 3GPP TS 29.122 [4]; or

-	an HTTP PUT/PATCH request to the NEF targeting an existing "Individual Monitoring Event Subscription" resource to request the update of an existing subscription as defined in clause 5.3.3.3.3.2 or 5.3.3.3.3.3 of 3GPP TS 29.122 [4];

-	the MonitoringEventSubscription data structure (or the requested modifications to the MonitoringEventSubscription data structure when HTTP PATCH is used) shall include the targeted application traffic, i.e., any UE application traffic associated with the S-NSSAI, provided within the "snssai" attribute, and the DNN provided within the "dnn" attribute, for the application(s) identified by the "appIds" attribute;

-	the monitoring type of the MonitoringEventSubscription data structure shall be set to the "APPLICATION_START" or "APPLICATION_STOP";

-	upon reception of the subscription request message from the AF, the NEF shall check whether the AF is authorized to perform this operation or not:

-	if the AF is not authorized, the NEF shall respond to the AF with an HTTP "403 Forbidden" status code with the response body including the ProblemDetails data structure containing the "cause" attribute set to the "REQUEST_NOT_AUTHORIZED" application error; or

-	if the AF is authorized, the NEF shall subscribe to the requested application traffic detection event(s) reporting at the concerned PCF(s) (locally configured at the NEF for the authorized DNN/S-NSSAI) using the Npcf_EventExposure_Subscribe service operation as defined in clause 4.2.2.2 of 3GPP TS 29.523 [22];

and

-	when the NEF receives an event notification from the PCF(s) via the Npcf_EventExposure_Notify service operation as defined in clause 4.2.4 of 3GPP TS 29.523 [22] indicating that the subscribed event(s) has(ve) been detected, the NEF shall send a corresponding notification to the AF by sending an HTTP POST request message to the AF with each of the corresponding MonitoringEventReport data structure(s) (provided within the "monitoringEventReports" attribute of the MonitoringNotification data structure) containing:

-	the reported event (i.e., "APPLICATION_START" or "APPLICATION_STOP") within the "monitoringType" attribute;

-	the identifier of the detected application within the "appId" attribute, if the "appIds" attribute within the corresponding subscription resource contains more than one array element (i.e., more than one application identifier); and

-	the PDU session information related to the detected application within the "pduSessInfo" attribute, if available.

and

-	if the "DataTransfer" feature defined in clause 5.3.4 of 3GPP TS 29.122 [4] is supported, in order to support AF request for Session inactivity time, Traffic volume and UL/DL data rate events monitoring event notification, the AF shall send an HTTP POST request to the NEF targeting the "Monitoring Event Subscriptions" resource (defined in clause 5.3.3.2.3.4 of 3GPP TS 29.122 [4]) to request the creation of a subscription or send an HTTP PUT/PATCH message to the NEF to the "Individual Monitoring Event Subscription" resource as defined in clause 5.3.3.3 of 3GPP TS 29.122 [4] for updating the subscription as follows:

1)	targeting list of UE(s) in the MonitoringEventSubscription data type setting the monitoring type as "SESSION_INACTIVITY_TIME", "TRAFFIC_VOLUME" and/or "UL_DL_DATA_RATE"; and

2)	upon reception of the corresponding subscription request message from the AF, the NEF shall check whether the AF is authorized to perform this operation or not.

NOTE 9:	When the "enNB" feature defined in clause 5.3.4 of 3GPP TS 29.122 [4] is supported, the above provisions related to the "monitoringType" attribute apply also to the "addnMonTypes" attribute, and the monitoring type(s) to be subscribed can be provided via the "monitoringType" attribute, and if there are more than one monitoring type to be subscribed, the "addnMonTypes" attribute as well.

NOTE 10:	In the above provisions, subscribing to a monitoring event can be done either by creating a new "Individual Monitoring Event Subscription" resource or updating an existing "Individual Monitoring Event Subscription" resource (e.g., add the monitoring event to the list of subscribed monitoring events, replace the existing monitoring event with this new monitoring event). Similarly, unsubscribing from a monitoring event can be done either by deleting the corresponding "Individual Monitoring Event Subscription" or updating the corresponding existing "Individual Monitoring Event Subscription" (e.g., remove the monitoring event from the list of subscribed monitoring events).

### 4.4.3	Procedures for Device Triggering

The procedures for device triggering as described in clause 4.4.6 of 3GPP TS 29.122 [4] shall be applicable in 5G with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	description of the HSS applies to the UDM;

-	the NEF shall interact with the UDM by using the Nudm_SubscriberDataManagement service and the Nudm_UEContextManagement service as defined in 3GPP TS 29.503 [17]; and

-	the NEF acts as MTC-IWF.

### 4.4.4	Procedures for resource management of Background Data Transfer

The procedures for resource management of Background Data Transfer (BDT) in 5GS are described in clause 4.4.3 of 3GPP TS 29.122 [4] with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	If the feature Group_Id is supported, an external group identifier may be included in the HTTP POST or PUT request message by the NEF. If the external group Id is sent from the AF to the NEF, the NEF shall interact with the UDM by using Nudm_SubscriberDataManagement service as defined in 3GPP TS 29.503 [17] to translate the external group identifier into the corresponding internal group identifier;

-	description of the PCRF applies to the PCF;

-	the NEF shall interact with the PCF by using Npcf_BDTPolicyControl service as defined in 3GPP TS 29.554 [19];

NOTE:	When the AF sends a PUT request to the NEF to update BDT negotiation data different from selecting a transfer policy and/or toggling BDT warning notifications, the NEF can delete the existing resource and create a new one with the required values at the PCF using the Npcf_BDTPolicyControl service. When the AF contacts NEF to select a transfer policy and/or to enable/disable BDT warning notifications, the NEF will initiate a PATCH request for BDT negotiation data on Npcf_BDTPolicyControl service.

-	if the "BdtNotification_5G" feature is supported, the AF may include a notification URI within the "notificationDestination" attribute in the Bdt data type during the background data transfer policy negotiation. In addition, the AF may request to enable the BDT warning notification by setting the "warnNotifEnabled" attribute to true. When the NEF receives the BDT warning notification from the PCF as defined in clause 4.2.4.2 of 3GPP TS 29.554 [19] and the "warnNotifEnabled" attribute was set to true, the NEF shall send an HTTP POST message including the ExNotification data structure to the AF identified by the notification destination URI received during the background data transfer policy negotiation. The AF shall respond with an HTTP response to confirm the received notification. The AF may select one policy from the candidate of BDT policies if provided in the notification by using the HTTP PATCH message as described in clause 5.4.3.3.3.3 of 3GPP TS 29.122 [4]. If the selected policy is set to value "0" within the "selectedPolicy" attribute in the HTTP PATCH message, it implies no transfer policy is selected by the AF. The AF may also request to disable/enable the BDT warning notification by including the "warnNotifEnabled" attribute in the HTTP PATCH message; and

-	The AF may include a traffic descriptor of background data within the "trafficDes" attribute in the Bdt data type during the background data transfer policy negotiation.

-	if the "AspId_5G" feature is supported, the AF may include an ASP Identifier within the "aspId" attribute in the Bdt data type. If the "aspId" attribute is included, the NEF shall not map the AF Identifier to ASP Identifier.

### 4.4.5	Procedures for CP Parameters Provisioning

The procedures for CP parameters provisioning as described in clause 4.4.9 of 3GPP TS 29.122 [4] shall be applicable in 5G with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	description of the HSS applies to the UDM;

-	the NEF shall interact with the UDM by using Nudm_ParameterProvision service as defined in 3GPP TS 29.503 [17];

-	if the ExpectedUMT_5G feature as defined in clause 5.10.4 of 3GPP TS 29.122 [4] is supported, the expected UE moving trajectory within the "expectedUmts" attribute shall also be included in the HTTP POST/PUT request. In addition, if the ExpectedUmtTime_5G feature as defined in clause 5.10.4 of 3GPP TS 29.122 [4] is supported, the start time and duration may be provided in the "expectedUmts" attribute to indicate when the UE arrives at a location and how long the UE stays in the location and the periodicity in the "expectedUmtDays" attribute may be provided to indicate the effective days within a week. In addition, if the "ExpectedUmtTime_Add" feature defined in clause 5.10.4 of 3GPP TS 29.122 [4] is supported, the additional day(s) of the week may be provided within the "expectedUmtDaysAdd" attribute to indicate the additional day(s) if the "expectedUmtDays" attribute is also provided;

-	if the "AppExpUeBehaviour" feature as defined in clause 5.10.4 of 3GPP TS 29.122 [4] is supported, the Application-Specific Expected UE Behaviour parameters within the "appExpUeBehvs" attribute may also be included in the HTTP POST/PUT request; and

-	if the "UEId_retrieval" feature defined in clause 5.10.4 of 3GPP TS 29.122 [4] is supported, in order to support the AF specific UE ID retrieval:

1)	the AF may request AF specific UE ID retrieval for an individual UE, by providing the UE's IP address in the "ueIpAddr" attribute or the UE's MAC address in the "ueMacAddr" attribute within the CpInfo data type;

2)	the AF may also provide the DNN, within the "dnn" attribute, and/or the S-NSSAI, within the "snssai" attribute, within the CpInfo data type;

3)	upon reception of the corresponding request message from the AF:

-	if the AF's request for AF specific UE ID retrieval is not authorized, the NEF shall respond to the AF with a "403 Forbidden" status code with the response body including the ProblemDetails data structure containing the "cause" attribute set to the "REQUEST_NOT_AUTHORIZED" application error indicating AF authorisation failure; and

-	if the AF's request for AF specific UE ID retrieval is authorized by the NEF, then if the DNN and/or S-NSSAI information is not available in the request, the NEF shall determine the corresponding DNN and/or S-NSSAI information based on the received requesting AF Identifier, and if provided, the MTC Provider Information;

4)	the NEF shall interact using the the BSF with UE address and IP domain (if the UE IPv4 address is provided), DNN and/or S-NSSAI to retrieve the session binding information of the UE by invoking the Nbsf_Management_Discovery service operation as described in 3GPP TS 29.521 [9];

5)	if the NEF receives an error response from the BSF, the NEF shall respond to the AF with a proper error status code. If the NEF received from the BSF an error response including a "ProblemDetails" data structure with the "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error. If no SUPI matching the provided UE information is returned by the BSF, the NEF shall respond to the AF with a "404 Not Found" status code with the response body including a ProblemDetails data structure containing the "cause" attribute set to the "UE_NOT_FOUND" application error to indicate that the requested UE address is not found;

6)	upon success and a SUPI is returned by the BSF, the NEF shall interact with the UDM to retrieve the AF specific UE Identifier using the received SUPI and at least one of the Application Port ID, MTC Provider Information or AF Identifier information by invoking Nudm_SDM_Get service as described in clause 5.2.2.2 of 3GPP TS 29.503 [17];

7)	upon success, the UDM responds to the NEF with the AF specific UE Identifier represented as an External Identifier for the UE which is uniquely associated with the MTC provider Information and/or AF Identifier. The NEF shall then respond to the AF with the received information, i.e. the AF specific UE Identifier represented as an External Identifier that was received from the UDM;

8)	if the NEF receives an error response from the UDM, the NEF shall respond to the AF with a proper error status code. If the NEF received from the UDM an error response including a "ProblemDetails" data structure with the "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error. If the UDM indicates that the requested UE Identifier is not available in the subscription data, the NEF shall respond to the AF with a "404 Not Found" error status code with the response body including a ProblemDetails data structure containing the "cause" attribute set to the "UE_ID_NOT_AVAILABLE" application error to indicate that the AF specific UE ID is not available.

NOTE 1:	The case where UE's IP address provided by the AF to the NEF corresponds to an IP address that has been NATed (Network and Port Address Translation) is not supported in this release of the specification.

NOTE 2:	When multiple AF parameter provisioning requests with different values of the same Expected UE Behavior parameters are recieved from different AFs, the network behavior is unspecified.

### 4.4.6	Procedures for PFD Management

The procedures for PFD management as described in clause 4.4.10 of 3GPP TS 29.122 [4] shall be applicable for 5GS with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF; and

-	the NEF (PFDF) shall interact with the UDR for PFD management by using Nudr_DataRepository service as defined in 3GPP TS 29.504 [20]. The PFDF is functionality within the NEF.

-	The NEF (PFDF) may interact with the NWDAF to retrieve the PFD Determination analytics to assist the determination of PFDs for known application identifiers by using Nnwdaf_EventsSubscription service as defined in 3GPP TS 29.520 [27]. The PFD information for which the source NF type is equal to "NWDAF" shall not be exposed to the AF unless based on local configuration and with related SLA between the operator and the ASP.

-	If the PFDs are provisioned to at least one of the subscribed SMFs (but not all) within the allowed delay, the NEF (PFDF) may notify the AF about the failed PFD provisioning with the HTTP POST message by including the PfdReport data structure in the body of the message. In addition, the NEF may include the location area(s) of the user plane(s) which are unable to enforce the provisioned PFD(s) within the "locationArea" attribute of the PFD report(s). If the PFDs are provisioned to none of the subscribed SMFs within the allowed delay, the NEF (PFDF) shall notify the AF about the failed PFD provisioning with the HTTP POST message using appropriate failure code as defined in Table 5.11.2.2.3-1 of 3GPP TS 29.122 [4].

NOTE 1:	Unsuccessful PFDs provisioning to the subscribed SMFs within the allowed delay means that the PFDs are not provisioned successfully to the UPFs served by the failed SMFs.

NOTE 2:	The NEF maps the 3GPP network area(s) to the geographic area(s), civic address(es) or DNAI(s) if the 3GPP network area(s) is not allowed to be exposed to the 3rd party according to the operator policy.

### 4.4.7	Procedures for Traffic Influence

#### 4.4.7.1	General

In order to create a resource for the Traffic Influence to provide the information of application function influence on traffic routing and/or N6-LAN service function chaining(s), the AF shall send an HTTP POST message to the NEF to the resource "Traffic Influence Subscription", with the request body including the TrafficInfluSub data structure as specified in clause 5.4.3.3.2.

If the feature "CommonEASDNAI" is supported, the AF may include the "tfcCorreInfo" attribute within the "TrafficInfluSub" data type. Within the "TrafficCorrelationInfo" data type, AF may include the "COMMON_DNAI" within the "corrType" attribute to indicate that the traffic of the set of UEs associated with the same traffic correlation Id accessing the application identified by an Application Identifier or traffic filtering information should target the EAS(es) corresponding to a common DNAI from the list of DNAI(s) or include the "COMMON_EAS" within the "corrType" attribute to indicate that the traffic of the set of UEs associated with the same traffic correlation Id accessing the application identified by an Application Identifier or traffic filtering information should target a common EAS. In the case of common EAS within the "TrafficCorrelationInfo" data type, the AF shall additionally include the common EAS address(es) within the "comEasIpv4Addr" attribute and/or "comEasIpv6Addr" attribute and/or the FQDN range corresponding to the application within the "fqdnRange" attribute. When the NEF receives traffic correlation notification from the SMF, if the NEF determines that there is currently no common EAS IP address and/or common DNAI available for the set of UEs identified by the Traffic Correlation ID or it determines that the common EAS or common DNAI needs to be re-selected, it selects a common DNAI and/or common EAS using the list of DNAI(s), EAS IP address and number of PDU sessions each SMF is serving for the set of UEs received in traffic correlation notification from the SMF. Then the NEF shall update the traffic influence data in UDR with the 5GC determined common EAS/DNAI for the set of UEs by invoking the Nudr_DataRepository service as described in 3GPP TS 29.504 [20] and 3GPP TS 29.519 [23] and then responds by acknowledging the notification to the SMF.

NOTE 1:	Common EAS selection means the common DNAI is selected.

In order to update an existing traffic influence subscription, the AF shall send an HTTP PUT message to the resource "Individual Traffic Influence Subscription", with the request body including the TrafficInfluSub data structure as specified in clause 5.4.3.3.2 requesting to update the traffic influence parameters.

In order to modify an existing traffic influence subscription, the AF shall send an HTTP PATCH message to the resource "Individual Traffic Influence Subscription", with the request body including the TrafficInfluSubPatch data structure as specified in clause 5.4.3.3.3 requesting to modify the traffic influence parameters.

In order to delete an existing traffic influence subscription, the AF shall send an HTTP DELETE message to the NEF to the resource "Individual Traffic Influence Subscription".

Upon receipt of the HTTP request from the AF, if the AF is authorized, the NEF shall perform the mapping as described in 3GPP TS 23.501 [3], and then perform as described in clause 4.4.7.2 if the request is identified by UE address or perform as described in clause 4.4.7.3 if the request is not identified by UE address.

If the EDGEAPP feature is supported and the "subscribedEvents" attribute is provided in the received HTTP POST request, and immediate reporting was requested by the AF, then user plane path management report(s) shall be included in the HTTP POST response within the "eventReports" attribute, if available. They may also be included in the HTTP PUT/PATCH response, if available.

NOTE 2:	The EAS IP Replacement information and the information indicating the EAS rediscovery are not provided simultaneously.

#### 4.4.7.2	AF request identified by UE address

Upon receipt of the above AF request which is for an individual UE identified by IP or Ethernet address, if the NEF supports HR-SBO scenarios, it may determine whether the PDU session is in HR-SBO mode as described in clause 4.4.7.5.

If the NEF deduces that the PDU session is not working in HR-SBO mode, the procedure in this clause applies. Otherwise, the procedure described in clause 4.4.7.5 shall be performed instead.

The NEF may interact with the BSF to retrieve the related PCF information by invoking the Nbsf_Management_Discovery service operation as described in 3GPP TS 29.521 [9]. If the NEF receives an error response from the BSF, the NEF shall not create the resource and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

After receiving a successful response from the BSF, the NEF shall interact with the PCF by invoking the Npcf_PolicyAuthorization service as described in 3GPP TS 29.514 [7]. After receiving a successful response from the PCF, the NEF shall:

-	for the HTTP POST request, create a resource "Individual Traffic Influence Subscription" which represents the traffic influence subscription, addressed by a URI that contains the AF Identifier and an NEF-created subscription identifier, and shall respond to the AF with a "201 Created" status code, including a Location header field containing the URI for the created resource, with the response body including a representation of the created "Individual Traffic Influence Subscription" resource within the TrafficInfluSub data structure. The AF shall use the URI received in the Location header in subsequent requests to the NEF to refer to this traffic influence subscription:

-	for the HTTP PUT or PATCH request, update a resource "Individual Traffic Influence Subscription" which represents the traffic influence subscription, and shall responds to the AF with a "200 OK" status code with the "TrafficInfluSub" data structure as response body containing the representation of the modified "Individual Traffic Influence Subscription", or an HTTP "204 No Content" response; and

-	for the HTTP DELETE request, remove all properties of the resource and delete the corresponding active resource "Individual Traffic Influence Subscription" which represents the traffic influence subscription, then shall responds to the AF with a "204 No Content" status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

If the NEF receives a response with an error code from the PCF, the NEF shall not create, update or delete the resource and shall respond to the AF with a proper error status code.

#### 4.4.7.3	AF request not identified by UE address

For AF request not identified by UE address, it may target an individual UE, one or more groups of UEs or any UE.

If HR-SBO scenarios are supported by the NEF, it may determine whether the PDU session is in HR-SBO mode as described in clause 4.4.7.5.

If the NEF deduces that the PDU session is not working in HR-SBO mode, the procedure in this clause applies. Otherwise, the procedure described in clause 4.4.7.5 shall be performed instead.

For an individual UE identified by GPSI, or one or more groups of UEs identified by External Group Identifier, the NEF shall interact with the UDM by invoking the Nudm_SubscriberDataManagement service as described in 3GPP TS 29.503 [17] to retrieve the SUPI or Internal Group Identifier.

When the feature FinerGranUEs is supported, the NEF may map the External Subscriber Category(ies) and any UE indicator, or External Subscriber Category(ies) and External Group Identifier(s) to Internal Group Identifier(s) or Internal Group Identifier(s) and Subscriber Category(ies).

NOTE:	As a user can be associated with multiple Subscriber Category(ies), some values of Subscriber Category(ies) can correspond to an SLA between an application provider represented by an AF and the 5GC operator. The combination of application identifier and External Subscriber Category can also be used to refer to this SLA.

The NEF shall interact with the UDR to store the traffic influence parameters received from the AF, updated as required and mapped as applicable by invoking the Nudr_DataRepository service as described in 3GPP TS 29.504 [20] and 3GPP TS 29.519 [23]. If the NEF receives an error response from the UDR, the NEF shall not create, update or delete the resource and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

After receiving a successful response from the UDR, the NEF shall:

-	for the HTTP POST request, create a resource "Individual Traffic Influence Subscription" which represents the traffic influence subscription, addressed by a URI that contains the AF Identifier and an NEF-created subscription identifier, and shall respond to the AF with a "201 Created" status code, including a Location header field containing the URI for the created resource with the response body including a representation of the created "Individual Traffic Influence Subscription" resource within the TrafficInfluSub data structure. The AF shall use the URI received in the Location header in subsequent requests to the NEF to refer to this traffic influence subscription;

-	for the HTTP PUT or PATCH request, update a resource "Individual Traffic Influence Subscription" which represents the traffic influence subscription, and shall responds to the AF with a "200 OK" status code with the "TrafficInfluSub" data structure as response body containing the representation of the modified "Individual Traffic Influence Subscription", or an HTTP "204 No Content" response; and

-	for the HTTP DELETE request, delete the corresponding active resource "Individual Traffic Influence Subscription" which represents the traffic influence subscription, and shall responds to the AF with a "204 No Content" status code.

#### 4.4.7.4	Handling of UP path management event notification

If the NEF receives a UP path management event notification from the SMF indicating that the subscribed event has been detected, then the NEF shall provide a notification by sending an HTTP POST message that shall include the EventNotification data type at least with the subscribed event (e.g. UP Path has changed) to the AF identified by the notification destination received during creation or modification of the Individual Traffic Influence Subscription resource and, optionally, by the AF Transaction Identifier received during the creation of the Individual Traffic Influence Subscription resource. The NEF may include in the EventNotification data type further information as defined in clause 5.4.3.3.4. If a URI for AF acknowledgement within the "ackUri" attribute is provided by the SMF in the event notification as defined in 3GPP TS 29.508 [26], the NEF shall also provide a URI for AF acknowledgement within the "afAckUri" attribute in the EventNotification data.

Upon receipt of the event notification, the AF shall respond with a "204 No Content" status code to confirm the received event notification.

Afterwards, if a URI for AF acknowledgement within the "afAckUri" attribute is received during the UP path management event notification, the AF may determine that an application layer relocation is needed, and may then send an HTTP POST request as acknowledgement for the UP path management event notification to inform the NEF about the result of application layer relocation. If the application layer is ready and/or the application relocation is completed, within the payload of the HTTP POST request, the AF shall include the AfAckInfo data type with the "afStatus" attribute set to "SUCCESS" and may provide within the AfResultInfo data the N6 traffic routing information associated to the target DNAI as "trafficRoute" attribute and, if the "ULBuffering" feature is supported, an indication that buffering of uplink traffic to the target DNAI is needed as "upBuffInd" attribute and, if the "EASIPreplacement" feature is supported, EAS IP replacement information as "easIpReplaceInfos" attribute; otherwise, the AF shall indicate the failure by including the AfAckInfo data type in the payload with the "afStatus" attribute sets to the corresponding failure cause. The NEF Northbound interface transaction identifier generated by the AF shall also be provided as the "afTransId" attribute within the AfAckInfo data if the AF has previously provided it.

Upon receipt of the AF acknowledgement, the NEF shall respond with a "204 No Content" status code to confirm the received acknowledgement, and forward the AF acknowledgement to the SMF as described in 3GPP TS 29.508 [26].

#### 4.4.7.5	Processing AF requests to influence traffic routing for HR-SBO session

If HR-SBO scenarios are supported by the NEF, upon receiving the AF request, it shall determine whether the PDU Session is working in HR-SBO mode based on the availability of the information provided by the AF as follows:

If the AF supports the "HR-SBO" feature and includes within the TrafficInfluSub data type the "plmnId", "dnn" and "snssai" attributes indicating the HPLMN ID, HPLMN DNN and HPLMN S-NSSAI of the UE,, the NEF shall determine that the PDU session is working in HR-SBO mode when the PLMN of the UE (derived from the "plmnId", "dnn" and "snssai" received attributes) is not the PLMN that the NEF belongs to.

NOTE 1:	The DNN can be unique for a PLMN (see 3GPP TS 23.003 [55]) and can be mapped to a HPLMN.

If the NEF did not receive any of the "plmnId", "dnn" and "snssai" attributes, the NEF shall deduce if the PDU session is working in HR-SBO mode and obtain the related information to be stored in the V-UDR based on the target UE information as follows:

1.	If the "gpsi" attribute is received and the HPLMN of the UE is part of it, the NEF derives the HPLMN of the UE (and thus whether HR-SBO applies) from the received GPSI. The NEF shall contact the NEF of the HPLMN as described in 3GPP TS 23.502 [2], clause 4.3.6.5.5 in order to obtain the SUPI, and the DNN and S-NSSAI of the HPLMN.

NOTE 2:	If the GPSI is in the form of External Identifier (see 3GPP TS 23.003 [55]) the NEF can determine the HPLMN of UE based on Domain Identifier in the GPSI.

2.	If the "anyUeInd" attribute is received, the NEF determines based on configuration if the PDU Session is working in HR-SBO mode.

3.	If the UE address is received as part of "ipv6Addr" or "ipv4Addr" attribute and it corresponds to a private IP address, the NEF determines whether HR-SBO applies based on configuration, e.g., to compare whether the received private IP address is within the configured separate private IP address ranges for HR-SBO PDU Sessions or based on interaction with the UPF by invoking the Nupf_GetUEPrivateIPaddrAndIdentifiers service as described in 3GPP TS 23.502 [2], clause 4.3.6.1, according to local policies.

4.	If the UE address is received as part of "ipv6Addr" or "ipv4Addr"attribute and it corresponds to a public IP address that belongs to an IP address range notowned by the VPLMN, the NEF shall determine that HR-SBO applies.

5.	If the UE address is received as part of "ipv4Addr"attribute and it corresponds to a public IP address NATed by the PLMN that the NEF belongs to, the NEF shall deduce the PDU session is working in HR-SBO by interacting with the UPF using the Nupf_GetUEPrivateIPaddrAndIdentifiers_Get request service operation as described in 3GPP TS 23.502 [2], clause 4.3.6.5.3.

NOTE 3:	In this release, the HPLMN allows HR-SBO for a PDU session only if the UE IP address of the PDU Session has not been allocated in a range that may overlap with other PDU sessions to the same DNN and S-NSSAI of that HPLMN.

NOTE 4:	It is assumed that the NEF is configured with the NATed IP range of its own PLMN. It is assumed that the NEF is configured based on HR-SBO roaming agreements of Public IP address ranges with an HPLMN ID.

NOTE 5:	This procedure is not supported if the AF request targets includes "externalGroupId", "externalGroupIds" or "extSubscCats" attributes within the TrafficInfluSub data type.

The NEF shall derive the information to be stored in the UDR considering the information received from the AF, the H-PLMN derivation (e.g. from GPSI, DNN) and the information obtained from the V-UPF (e.g. SUPI, IP address, DNN, S-NSSAI) or H-NEF (e.g.DNN, S-NSSAI, SUPI) when applicable according to the scenarios described above.

After having derived the information as described above, the NEF shall interact with the UDR to store the traffic influence parameters as described in 3GPP TS 29.504 [20] and 3GPP TS 29.519 [23].

If the NEF receives an error response from the UDR, the NEF shall not create, update or delete the resource and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

After receiving a successful response from the UDR, the NEF shall:

-	for the HTTP POST request, create a resource "Individual Traffic Influence Subscription" which represents the traffic influence subscription, addressed by a URI that contains the AF Identifier and an NEF-created subscription identifier, and shall respond to the AF with a 201 Created status code, including a Location header field containing the URI for the created resource. The AF shall use the URI received in the Location header in subsequent requests to the NEF to refer to this traffic influence subscription;

-	for the HTTP PUT or PATCH request, update a resource "Individual Traffic Influence Subscription" which represents the traffic influence subscription, and shall respond to the AF with a 200 OK status code with the "TrafficInfluSub" data structure as response body containing the representation of the modified "Individual Traffic Influence Subscription", or an HTTP "204 No Content" response; and

-	for the HTTP DELETE request, delete the corresponding active resource "Individual Traffic Influence Subscription" which represents the traffic influence subscription, and shall responds to the AF with a 204 No Content status code.

### 4.4.8	Procedures for changing the chargeable party at session set up or during the session

The procedures for changing the chargeable party at session set up or during the session in 5GS shall reuse the procedures and provisions defined in clause 4.4.4 of 3GPP TS 29.122 [4] with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	description of the PCRF applies to the PCF;

-	in the HTTP POST request, the AF may include the AF session subscribed "dnn" attribute and/or "snssai" attribute;

-	if the EthChgParty_5G feature defined in clause 5.5.4 of 3GPP TS 29.122 [4] is supported and the request is for an Ethernet UE:

-	in the HTTP POST request:

-	the AF shall include the UE MAC address within the "macAddr" attribute instead of the UE IP address; and

-	instead of the IP Flow description:

-	if the AppId feature defined in clause 5.5.4 of 3GPP TS 29.122 [4] is not supported, the AF shall include the Ethernet Flow description within the "ethFlowInfo" attribute;

-	otherwise, the AF shall include either the External Application Identifier, within the "exterAppId" attribute, or the Ethernet Flow description, within the "ethFlowInfo" attribute;

and

-	in the HTTP PATCH request, the AF may update the Ethernet Flow description, within the "ethFlowInfo" attribute, or the External Application Identifier, within the "exterAppId" attribute;

-	the NEF may interact with the BSF by using the Nbsf_Management_Discovery service (as defined in 3GPP TS 29.521 [9]) to retrieve the PCF addressing information;

-	the NEF shall interact with the PCF by using the Npcf_PolicyAuthorization service as defined in 3GPP TS 29.514 [7]; and

-	if the ToSTC_5G feature defined in clause 5.5.4 of 3GPP TS 29.122 [4] is supported:

-	in the HTTP POST request, the AF may include the "tosTC" attribute within the "flowInfo" attribute; and

-	in the HTTP PATCH request, the AF may include the "tosTC" attribute within the "flowInfo" attribute.

### 4.4.9	Procedures for AF required QoS

#### 4.4.9.1	General

The following procedures are used for AF required QoS in 5GS:

-	Setting up an AF session with required QoS in 5GS for target UE identified by UE address (i.e. IP address or Mac address) or setting up a Multi-member AF session with required QoS in 5GS for target list of UEs identified by the list of UE addresses as described in clause 4.4.9.2.

-	AF required QoS in 5GS for target UE not identified by UE address as described in clause 4.4.9.3.

#### 4.4.9.2	Procedures for AF setting up an AF session with required QoS for target UE identified by UE address or for target list of UEs identified by list of UE addresses

The provisions and procedures for setting up an AF session with required QoS in 5GS targeting a UE identified by its UE address (IP address or Mac address) or setting up a Multi-member AF session with required QoS in 5GS for target list of UEs identified by the list of UE addresses are described in clause 4.4.13 of 3GPP TS 29.122 [4] with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	description of the PCRF applies to the PCF;

-	the NEF may interact with NRF to retrieve the BSF address of the serving UE IP address (es) as defined in 3GPP TS 29.510 [57];

-	the NEF may interact with BSF by using Nbsf_Management_Discovery service as defined in 3GPP TS 29.521 [9] to retrieve the PCF address;

-	the NEF shall interact with the PCF by using Npcf_PolicyAuthorization service as defined in 3GPP TS 29.514 [7];

-	when the "ListUE_5G" feature is supported, in case the NEF receives a list of UE addresses, the NEF shall interact with the NRF/BSF/PCF with above procedures for each UE address individually. The NEF may consolidate the outcome of the individual request for AF session with required QoS corresponding to each UE's IP address and consolidates them into a single response before forwarding it to the AF based on a locally configured consolidated outcome timer.

NOTE 1:	The consolidation of the outcome of the individual requests and the locally configured timer are implementation dependant, e.g., the consolidated outcome timer is locally configured in NEF whose default value is zero.

-	in the HTTP POST request, the AF may include a "dnn" attribute and/or a "snssai" attribute; and in the HTTP PUT request, the AF shall keep the same value(s) of the "dnn" attribute and/or the "snssai" attribute as set in the HTTP POST request if provided;

-	description about the INDICATION_OF_SUCCESSFUL_RESOURCES_ALLOCATION event and INDICATION_OF_FAILED_RESOURCES_ALLOCATION event apply to the SUCCESSFUL_RESOURCES_ALLOCATION event and FAILED_RESOURCES_ALLOCATION event respectively; In addition, description about the INDICATION_OF_RELEASE_OF_BEARER, INDICATION_OF_LOSS_OF_BEARER and INDICATION_OF_RECOVERY_OF_BEARER events are not applicable in this specification.

-	if the EthAsSessionQoS_5G feature as defined in clause 5.14.4 of 3GPP TS 29.122 [4] is supported and the request is for Ethernet UE:

-	in the HTTP POST/PUT request, the AF shall include the UE MAC address within the "macAddr" attribute instead of the UE IP address. If the AppId feature is not supported, the AF shall include the Ethernet Flow description within the "ethFlowInfo" attribute instead of the IP Flow description; otherwise, the AF shall include either the External Application Identifier within the "exterAppId" attribute or the Ethernet Flow description within the "ethFlowInfo" attribute;

-	in the HTTP PATCH request, the AF may update the Ethernet Flow description within the "ethFlowInfo" attribute or the External Application Identifier within the "exterAppId" attribute;

-	if the "ListUE_5G" feature as defined in clause 5.14.4 of 3GPP TS 29.122 [4] is supported;

-	in order to support the QoS handling and QoS monitoring for the list of UEs from AF, the AF may include:

a	the list of UE address(es) within the "listUeAddrs" attribute instead of the UE IP/MAC address; and

b.	the list of UE addresses subject for Consolidated Data Rate monitoring within the "listUeConsDtRt" attribute;

NOTE 2:	If the "listUeConsDtRt" attribute is provided, it is a subset of "listUeAddrs" attribute.

-	in order to support the QoS Monitoring for UL and/or DL data rate for the list of UEs, the AF shall include the required consolidated data rate monitoring information for the list of UEs within the "qosMonDatRate" attribute. The subscribed event is "QOS_MONITORING". The AF shall include within the "qosMonDatRate" attribute:

a)	the requested consolidated data rate parameter(s) to be measured (i.e., DL and/or UL data rate) within the "reqQosMonParams" attribute;

b)	one or more report frequency(ies) within the "repFreqs" attribute;

c)	when the "repFreqs" attribute is set to the value "EVENT_TRIGGERED":

-	the data rate threshold for downlink within the "consDataRateThrDl" attribute; and/or

-	the data rate threshold for uplink within the "consDataRateThrUl" attribute;

d)	when the "repFreqs" attribute is set to the value "PERIODIC", the periodic time for reporting within the "repPeriod" attribute; and

e)	when the "repFreqs" attribute is set to the value "EVENT_TRIGGERED", the minimum waiting time between subsequent reports within the "waitTime" attribute;

-	if the NEF recognizes, based on configuration, that the IP address(es) received within the "listUeAddrs" attribute are different from the IP address(es) assigned by 5GC (i.e. the UE(s) are behind a NAT in UPFs), the NEF shall invoke the UEId API as defined in clause 4.4.32 for each UE IP address with port number in order to identify the corresponding IP address (and IP domain, if necessary) that has been assigned by the 5GC. The NEF then uses the respective corresponding IP address (and IP domain, if necessary) in the following steps instead of the UE IP address provided by the AF;

if the NEF authorizes the AF request, the NEF subscribes to data rate QoS Monitoring for each UE in the list to the PCFs by invoking the Npcf_PolicyAuthorization service as defined in 3GPP TS 29.514 [7]. The NEF shall always set its notification URI as the Target of Reporting and set "directNotifInd" to "true" in the request to PCFs to ensure that QoS Monitoring reports can be sent by the UPF directly to the NEF regardless of whether the "directNotifInd" attribute was set to "true" in the request from the AF.

when the NEF receives the notification about data rate for the list of UEs from the UPFs, the NEF shall aggregate the QoS Monitoring reports for data rate for those UEs identified by the "listUeConsDtRt" attribute or "listUeAddrs" attribute and notify the AF with the "aggrDataRateRpts" attribute that may include:

a)	the uplink data rate within the "ulAggrDataRate" attribute; and/or

b)	the downlink data rate within the "dlAggrDataRate" attribute;

NOTE 3:	If the "consDataRateThrDl" and/or "consDataRateThrUl" attributes are provided, the QoS parameter(s) to be measured indicates the Guaranteed Bitrate needs to be provided.

-	if the "QoSMonitoring_5G" feature as defined in clause 5.14.4 of 3GPP TS 29.122 [4] is supported, in order to support the QoS Monitoring for packet delay, the AF shall include "qosMonInfo" attribute. The AF shall also include the "directNotifInd" attribute set to true if the "ExposureToEAS" feature is supported and the direct notification is required. Within the QosMonitoringInformation data structure, the AF shall include:

1.	one or more requested QoS Monitoring Parameter(s) (i.e., UL, DL and/or RTT delay) within the "reqQosMonParams"; and

2.	one or more report frequency within the "repFreqs" attribute; and

3.	when the "repFreqs" attribute includes the value "PERIODIC", the periodic time for reporting and, if the feature "PacketDelayFailureReport" is supported, the maximum period with no QoS measurement results reported within the "repPeriod" attribute; and

4.	when the "repFreqs" attribute includes the value "EVENT_TRIGGERED":

a.	delay threshold(s) as follows:

-	the delay threshold for downlink with the "repThreshDl" attribute;

-	the delay threshold for uplink with the "repThreshUl" attribute; and/or

-	the delay threshold for round trip with the "repThreshRp" attribute;

b.	the minimum waiting time between subsequent reports within the "waitTime" attribute; and

c.	if the feature "PacketDelayFailureReport", the maximum period with no QoS measurement results reported within the "repPeriod" attribute;

-	if the "EnQoSMon" feature is supported and QoS monitoring control is for packet delay and/or congestion and/or data rate and if the "MultiMedia" feature is supported, the request is not for multiple flows (i.e., the "multiModDatFlows" attribute is not included), the AF shall include:

-	the "qosMonInfo" attribute to request QoS monitoring for packet delay as described for the "QoSMonitoring_5G" feature, the "qosMonConReq" attribute to request QoS monitoring for congestion and/or the "qosMonDatRate" attribute to request QoS monitoring for data rate;

NOTE 4:	When the feature "MultiMedia" is supported and the request is for multiple flows (i.e., the "multiModDatFlows" attribute is included) the subscription for QoS monitoring can only be indicated within the corresponding "multiModDatFlows" entry.

-	if direct notification is required for the QoS measurement(s) provided in the "qosMonInfo", "qosMonConReq" and "qosMonDatRate" attribute(s), the "directNotifInd" attribute set to true;

-	within each of the provided QosMonitoringInformation data structure(s):

1.	one or more requested QoS Monitoring Parameter(s) for the concerned QoS monitoring parameter within the "reqQosMonParams" attribute;

2.	one or more report frequency within the "repFreqs" attribute, if applicable;

NOTE 5:	If the "reqQosMonParams" attribute indicates congestion measurement(s), the "repFreqs" attribute can only indicate "EVENT_TRIGGERED".

3.	when the "repFreqs" attribute includes the value "PERIODIC", the periodic time for reporting and the maximum period with no QoS measurement results reported within the "repPeriod" attribute; and

4.	when the "repFreqs" attribute includes the value "EVENT_TRIGGERED":

a.	for QoS monitoring for data rate:

-	the data rate threshold for downlink within the "repThreshDatRateDl" attribute; and/or

-	the data rate threshold for uplink within the "repThreshDatRateUl" attribute;

b.	for QoS monitoring for congestion information

-	the congestion threshold for downlink with the "conThreshDl" attribute; and/or

-	the congestion threshold for uplink with the "conThreshUl" attribute; and

c.	the minimum waiting time between subsequent reports within the "waitTime" attribute; and

d.	the maximum period with no QoS measurement results reported within the "repPeriod" attribute.

-	if the QoS monitoring control is for data rate, the AF may include the averaging window within the "avrgWndw" attribute.

If the NEF authorizes the AF request, the NEF may create a QoS monitoring notification correlation identifier for the AF transaction during the creation of the AF resource and may provision it together with the received QoS monitoring parameters to the PCF by invoking the Npcf_PolicyAuthorization service as defined in 3GPP TS 29.514 [7] or, if the "TSC_5G" feature is supported, to the TSCTSF by invoking the Ntsctsf_QoSandTSCAssistance service as defined in 3GPP TS 29.565 [50];

-	if the NEF receives from the PCF the indication that direct notification is not possible for the requested QoS monitoring parameters as specified in 3GPP TS 29.514 [7], the NEF shall include in the response to the AF request the "servAuthInfo" attribute with the value "DIRECT_NOTIF_NOT_POSSIBLE";

-	when the NEF receives the event notification for the AF transaction as defined in clause 4.2.2 of 3GPP TS 29.508 [26] or clause 4.2.5.14 of 3GPP TS 29.514 [7] or, if the "TSC_5G" feature is supported, clause 5.3.2.5.7 of 3GPP TS 29.565 [50], or when the AF requested direct notification, as defined in clause 5.2.2.3 of 3GPP TS 29.564 [61], the NEF shall include one or more QoS monitoring reports with the delay measurement within the "qosMonReports", the data rate measurements within the "qosMonDatRateReps" and/or the congestion measurements within "qosMonConInfoReps" attribute. Within the QosMonitoringReport data structure, the NEF shall include the received monitored QoS information.

-	for packet delay measurements, within "qosMonReports":

a.	the uplink packet delays within the "ulDelays" attribute; and/or

b.	the downlink packet delays within the "dlDelays" attribute; and/or

c.	the round trip packet delays within the "rtDelays" attribute;

NOTE 6:	The PCF, the SMF, the UPF or the TSCTSF report one UL, DL and/or round-trip packet delay measurement for each periodic and/or event-triggered report as described in 3GPP TS 29.514 [7], 3GPP TS 29.508 [26], 3GPP TS 29.564 [61] and 3GPP TS 29.565 [50], i.e, the NEF can include only one element within the "ulDelays", "dlDelays", and/or "rtDelays" array(s), each one with the received report from the PCF, SMF, UPF or the TSCTSF for the UL, DL and/or round trip delay(s).

-	for congestion information measurements, within the "qosMonConInfoReps":

a.	the uplink congestion information measurement within the "ulConInfo" attribute; and/or

b.	the downlink congestion information measurement within the "dlConInfo" attribute;

-	for data rate measurements, within "qosMonDatRateReps":

a.	one data rate measurement for the UL within the "ulDataRate" attribute; and/or

b.	one data rate measurement for the DL within the "dlDataRate" attribute; or

-	if the feature "PacketDelayFailureReport" is supported, the packet delay measurement failure indicator within the "pdmf" attribute;

-	if the "MultiMedia" feature is supported, when the NEF receives the event notification for the AF transaction as defined in clause 4.2.2 of 3GPP TS 29.508 [26] or clause 4.2.5.14 of 3GPP TS 29.514 [7], or when the AF requested direct notification, as defined in clause 5.2.2.3 of 3GPP TS 29.564 [61], the NEF shall include the affected single-modal identification number and the corresponding flows within the "multiModFlows" attribute.

-	if the "AlternativeQoS_5G" feature is supported, the AF may include an ordered list of QoS references within the "altQosReferences" attribute and, if the "DisableUENotification_5G" feature is also supported, an indication that the UE does not need to be informed about changes related to Alternative QoS Profiles within the "disUeNotif" attribute.

-	When the NEF interfaces directly with the PCF, the NEF shall transfer them to the PCF in the Npcf_PolicyAuthorization service and subscribe to PCF event "QOS_NOTIF" in the Npcf_PolicyAuthorization service. When the NEF receives the notification of PCF event "QOS_NOTIF", it shall notify the AF with "QOS_GUARANTEED" event or with "QOS_NOT_GUARANTEED" event and the currently applied QoS reference if received. When the NEF receives the notification of PCF event "SUCCESSFUL_RESOURCES_ALLOCATION", it shall notify the AF the event together with the currently applied QoS reference if received.

-	If the "TSC_5G" feature is supported, when the NEF interfaces with the TSCTSF, the NEF shall transfer the received alternative QoS references to the TSCTSF in the Ntsctsf_QoSandTSCAssistance service and subscribe with TSCTSF to "QOS_GUARANTEED" and "QOS_NOT_GUARANTEED" events. When the NEF receives the event notification from the TSCTSF, the NEF shall notify the AF with "QOS_GUARANTEED" event or with "QOS_NOT_GUARANTEED" event and the currently applied QoS reference if received. When the NEF receives the notification of TSCTSF event "SUCCESSFUL_RESOURCES_ALLOCATION", it shall notify the AF the event together with the currently applied QoS reference if received.

If the feature "AltQoSProfilesSupportReport" is supported, when the NEF receives the indication from the PCF or the TSCTSF about the support of alternative QoS profiles, the NEF shall notify the AF forwarding the received indication within the "altQosNotSuppInd" attribute.

NOTE 7:	Based on the operator configuration, the QoS reference identifiers received from the AF can be the same or different as the QoS reference identifiers known at the PCF. The NEF can perform a mapping for the QoS reference identifier.

-	if the "TSC_5G" feature is supported, the AF may include:

-	the TSC QoS requirement within the "tscQosReq" attribute. Within the TscQosRequirement data structure, the AF may include:

-	the input information to construct the TSC Assistance Container within the "tscaiInputUl" attribute and/or "tscaiInputDl"attribute, and the (g)PTP domain that the AF is located in within the "tscaiTimeDom" attribute;

NOTE 8:	For the adjustment of burst sending time and adjustment of periodicity within the "periodicityRange" attribute in the UL direction within the "tscaiInputUl" attribute, it is expected that the AF interacts with the application in the UE or devices behind the UE based on application layer signaling.

-	the capability for BAT adaptation within the "capBatAdaptation" attribute, if the "EnTSCAC" feature is also supported. The capability for BAT adaptation and the burst arrival time window ("burstArrivalTimeWnd" attribute within the "tscaiInputUl" attribute and/or "tscaiInputDl" attribute of the "tscQosReq" attribute) are mutually exclusive; and

-	if individual QoS parameters instead of QoS reference is provided, may include:

-	requested GBR within the "reqGbrDl" attribute and/or "reqGbrUl" attribute;

-	requested MBR within the "reqMbrDl" attribute and/or "reqMbrUl" attribute;

-	the maximum burst size within the "maxTscBurstSize" attribute;

-	the priority within the "priority" attribute;

-	the requested 5GS delay within the "req5Gsdelay" attribute; and

-	the requested packet error rate within the "reqPer" attribute, if the "ExtQoS_5G" feature is also supported.

If the NEF authorizes the AF request, the NEF may provision the received QoS requirements to the TSCTSF by invoking the Ntsctsf_QoSandTSCAssistance_Create/Update request as defined in 3GPP TS 29.565 [50]. The NEF determines whether to invoke the TSCTSF or to directly contact the PCF based on operator configuration. This determination may consider the AF identifier, whether the "tscaiInputUl" and/or "tscaiInputDl" attributes within the "tscQosReq" attribute were received in the subscription request, whether the "qosReference" attribute or individual QoS parameters within the "tscQosReq" attribute were received in the subscription request, and SLA between operator and application provider. A TSCTSF address may be locally configured in the NEF or the NEF uses the DNN/S-NSSAI (which may be provided in the request or determined based on the AF identifier) to discover the TSCTSF from the NRF. If the NEF directly contacts the PCF while the NEF determined to invoke the TSCTSF when authorizing the update request, the NEF shall reject the request message by sending an HTTP response to the AF with a status code set to 403 Forbidden and may include the "INVALID_SESSION_UPDATE" error in the "cause" attribute of the "ProblemDetails" structure and indicate which parameters can not be served in current session in the "invalidParams" attribute of the "ProblemDetails" structure.

NOTE 9:	The NEF can determine whether the TSCTSF needs to be involved based on the DNN/S-NSSAI for the AF session according to the SLA.

If the "EnTSCAC" feature is supported and the NEF receives the BAT offset information from the TSCTSF about the BAT offset and the optionally adjusted periodicity, the NEF shall send an Event Notification to the AF with the "event" attribute set to BAT_OFFSET_INFO and including the "ranBatOffsetNotif" attribute and optionally the "adjPeriod" attribute within the "batOffsetInfo" attribute.

-	if the "AltQosWithIndParams_5G" feature is supported, the AF may include:

-	an ordered list of alternative service requirements that include individual QoS parameter sets within the "altQosReqs" attribute and, if the "DisableUENotification_5G" feature is also supported, an indication that the UE does not need to be informed about changes related to Alternative QoS Profiles within the "disUeNotif" attribute. Within the AlternativeServiceRequirementsData data structure, the AF shall include:

-	a reference to the alternative individual QoS related parameter(s) included in this set within the "altQosParamSetRef" attribute; and

-	at least one of the following:

-	The guaranteed bandwidth in uplink within the "gbrUl" attribute and the guaranteed bandwidth in downlink within the "gbrDl" attribute;

-	The requested packet delay budget within the "pdb" attribute;

-	The requested packet error rate within the "per" attribute if the "ExtQoS_5G" feature is supported;

If the NEF authorizes the AF request, and if the "TSC_5G" feature is supported, the NEF may provision the received QoS requirements and subscribe with the TSCTSF to "QOS_GUARANTEED" and "QOS_NOT_GUARANTEED" events by invoking the Ntsctsf_QoSandTSCAssistance_Create request as defined in 3GPP TS 29.565 [50]. The NEF determines whether to invoke the TSCTSF or to directly contact the PCF based on operator configuration. This determination may consider the AF identifier, whether the "tscaiInputUl" and/or "tscaiInputDl" attributes within the "tscQosReq" attribute were received in the subscription request, whether the "qosReference" attribute or individual QoS parameters within the "altQosReqs" attribute were received in the subscription request, and SLA between operator and application provider. A TSCTSF address may be locally configured in the NEF or the NEF uses the DNN/S-NSSAI (which may be provided in the request or determined based on the AF identifier) to discover the TSCTSF from the NRF. When the NEF receives the notification of TSCTSF "QOS_GUARANTEED" event or "QOS_NOT_GUARANTEED" event, it shall notify the AF with "QOS_GUARANTEED" event or "QOS_NOT_GUARANTEED" event with the currently applied individual QoS parameter set within the "appliedQosRef" attribute if received. When the NEF receives the notification of the TSCTSF event "SUCCESSFUL_RESOURCES_ALLOCATION", it shall notify the AF the event together with the currently applied individual QoS parameter set within the "appliedQosRef" attribute if received. If the NEF directly contacts the PCF while the NEF determined to invoke the TSCTSF when authorizing the update request, the NEF shall reject the request message by sending an HTTP response to the AF with a status code set to 403 Forbidden and may include the "INVALID_SESSION_UPDATE" error in the "cause" attribute of the "ProblemDetails" structure and indicate which parameters can not be served in current session in the "invalidParams" attribute of the "ProblemDetails" structure.

NOTE 10:	The NEF can determine whether the TSCTSF needs to be involved based on the DNN/S-NSSAI for the AF session according to the SLA.

When the NEF interfaces directly with the PCF, the NEF shall transfer the received QoS requirements to the PCF in the Npcf_PolicyAuthorization service and subscribe to PCF event "QOS_NOTIF" in the Npcf_PolicyAuthorization service. When the NEF receives the notification of PCF event "QOS_NOTIF", it shall notify the AF with "QOS_GUARANTEED" event or with the "QOS_NOT_GUARANTEED" event and the currently applied QoS reference if received. When the NEF receives the notification of PCF event "SUCCESSFUL_RESOURCES_ALLOCATION", it shall notify the AF the event together with the currently applied QoS reference if received.

If the feature "AltQoSProfilesSupportReport" is supported, when the NEF receives the indication from the PCF or the TSCTSF about the support of alternative QoS profiles, the NEF shall notify the AF forwarding the received indication within the "altQosNotSuppInd" attribute.

-	if the "enNB_5G" feature defined in clause 5.14.4 of 3GPP TS 29.122 [4] is supported:

-	the AF may additionally subscribe to the "ACCESS_TYPE_CHANGE" and/or "PLMN_CHG" event(s); and

-	if the NEF authorizes the AF request, the NEF shall subscribe to the corresponding event(s) at the PCF by invoking the Npcf_PolicyAuthorization service API as defined in 3GPP TS 29.514 [7];

-	if the ToSTC_5G feature as defined in clause 5.14.4 of 3GPP TS 29.122 [4] is supported:

-	in the HTTP POST request, the AF may include the "tosTC" attribute within the "flowInfo" attribute of the AsSessionWithQoSSubscription data type; and

-	in the HTTP PATCH request, the AF may include the "tosTC" attribute within the "flowInfo" attribute of the AsSessionWithQoSSubscriptionPatch data type;

-	if the "EnQoSMon" feature is supported and QoS Monitoring control is for packet delay variation:

-	the AF shall include the required Packet Delay Variation monitoring information within "pdvMon" attribute. The subscribed event is "PACK_DELAY_VAR". The AF shall include within the "pdvMon" attribute:

a)	the requested Packet Delay Variation parameter(s) to be measured (i.e. DL, UL and/or round trip packet delay variation) within the "reqQosMonParams" attribute;

b)	one or more report frequency within the "repFreqs" attribute;

c)	when the "repFreqs" attribute is set to the value "EVENT_TRIGGERED":

-	the Packet Delay Variation threshold for downlink with the "repThreshDl" attribute;

-	the Packet Delay Variation threshold for uplink with the "repThreshUl" attribute; and/or

-	the Packet Delay Variation threshold for round trip with the "repThreshRp" attribute;

d)	when the "repFreqs" attribute is set to the value "PERIODIC", the periodic time for reporting within the "repPeriod" attribute; and

e)	when the "repFreqs" attribute is set to the value "EVENT_TRIGGERED", the minimum waiting time between subsequent reports within the "waitTime" attribute;

NOTE 11:	The direct notification "directNotifInd" attribute is not applicable for "pdvMon" attribute because the PDV monitoring calculation and notification is performed by the PCF. In case "directNotifInd" attribute is provided for packet delay, data rate, and/or congestion information along with PDV monitoring, the PDV monitoring follows the specified PCF notification mechanism and other QoS monitorings request follows the direct notification mechanism, if feasible.

-	when the NEF receives the notification about Packet Delay Variation event notification from the PCF as defined in clause 4.2.5.26 of 3GPP TS 29.514 [7], the NEF shall notify the AF with "PACK_DELAY_VAR" event and include the received monitored Packet Delay Variation information within the "pdvMonReports" attribute, it may include:

a)	the uplink packet delay variation measurement(s) within the "ulPdv" attribute;

b)	the downlink packet delay variation measurement(s) within the "dlPdv" attribute;

c)	the round trip packet delay variation measurement(s) within the "rtPdv" attribute;

NOTE 12:	QoS Monitoring for the round-trip delay over two QoS flows requires the support of the "Multimedia" feature and is subscribed at single-modal data flow(s) level. The "RT_DELAY_TWO_QOS_FLOWS" event cannot be provided within the "events" attribute.

-	if the "MultiMedia" feature is supported, the AF may include:

-	the multi-modal Service ID within the "multiModalId" attribute; and/or

-	the multi-modal data flow(s) information of the multi-modal service in the "multiModDatFlows" attribute. The AF shall include for each single-modal data flow(s) of the multi-modal service:

1.	the single-modal data identification number within the "medCompN" attribute;

2.	the IP data flow(s) description for the single-modal data flow within the "flowInfos" attribute; and

3.	the parameters that describe the requested QoS for the single-modal data flow, as follows:

a.	the single-modal data flow type within the "medType" attribute, if applicable;

b.	either a reference to a pre-defined QoS information for the single-modal data flow within the "qosReference" attribute, or individual QoS parameters within the corresponding attributes;

c.	if individual QoS parameters are provided, an ordered list of alternative service requirements for the single-modal data flow within the "altSerReqsData" attribute, if applicable;

d.	if a reference to pre-defined QoS information is provided, an ordered list of QoS references for the single-modal data flow within the "altSerReqs" attribute, if applicable;

e.	if the "RTLatency" feature is supported:

-	an indication that the a service data flow needs to meet the Round-Trip (RT) latency requirement within the "rTLatencyInd" attribute, or the indication that two service data flows together with the identification of the two service data flows need to meet the RT latency requirement within the "rTLatencyIndCorreId" attribute;

-	the RT latency requirement (i.e., PDB) of the service data flow(s) either explicitly within the "pdb" attribute or implicitly (to be derived by the PCF) in the "qosReference" attribute;

f.	if the "PDUSetHandling" feature is supported, PDU Set QoS related information for the single-modal data flow within the "pduSetQosDl" and/or "pduSetQosUl" attribute(s), if applicable, and the Protocol Description related information within the "protoDescDl" and/or "protoDescUl" attribute(s), if applicable;

NOTE 13:	For multi-modal communication services related to multiple UEs, multiple UE-specific AF requests are used, and the AF provided information to NEF is the same as single UE case. Multiple UE-specific AF requests can include the same multimodal Service ID within the "multiModalId" attribute. For the single UE case, the AF can provide the multiple single-modal data flows of the multi-modal communication service via single or multiple AF requests.

g.	if the "EnQoSMon" feature is supported, the subscription information which is applicable to the QoS monitoring events within the "evSubsc" attribute as specified in 3GPP TS 29.514 [7];

h.	if the "L4S" feature is supported, the Low Latency, Low Loss and Scalable Throughput (L4S) Support indication within the "l4sInd" attribute. In this case, the AF shall also subscribe to notifications of ECN marking for L4S support information not available in 5GS within the"evSubsc" attribute as specified in 3GPP TS 29.514 [7]; and

i.	if the "PowerSaving" feature is supported, the time period between the start of the two data bursts in Uplink and/or Downlink direction within the "periodUl" and "periodDl" attributes respectively, and the Downlink Protocol Description related information within the "protoDescDl" attribute;

NOTE 14:	When both, "EnQoSMon" and "L4S" features are supported, for each data flow of the multi-modal service, the AF can include either the indication of L4S support within the "l4sInd" attribute or the request for congestion measurements within the "evSubsc" attribute as specified in 3GPP TS 29.514 [7], but the request cannot include both attributes simultaneously. The Individual AS Session with Required QoS Subscription resource cannot contain for a single-modal data flow(s) simultaneously both, the indication of L4S support and the subscription to congestion monitoring.

-	if the NEF authorizes the AF request, the NEF shall provision the received multi-modal service information to the PCF by invoking the Npcf_PolicyAuthorization service as defined in 3GPP TS 29.514 [7]. If the multi-modal service information contains per flow subscription to events, the NEF, per flow, shall provide a notification URI and may provide a notification correlation identifer together with the received event(s) parameters by invoking the Npcf_PolicyAuthorization service as defined in 3GPP TS 29.514 [7]; and

-	if the NEF receives from the PCF the indication that direct notification is not possible for the requested QoS monitoring parameters as specified in 3GPP TS 29.514 [7], the NEF shall include in the response to the AF request the "servAuthInfo" attribute with the value "DIRECT_NOTIF_NOT_POSSIBLE";

-	when the NEF receives the QoS monitoring event notification for the AF transaction as defined in clause 4.2.5.14 of 3GPP TS 29.514 [7], the NEF shall identify the affected AF flow identifiers based on the flow identifiers received from the PCF. When the NEF receives the QoS monitoring event notification for the AF transaction as defined in clause 4.2.2 of 3GPP TS 29.508 [26] or when the AF requested direct notification, as defined in clause 5.2.2.3 of 3GPP TS 29.564 [61], the NEF may identify the affected AF flow identifiers based on the notification correlation identifier and/or target notification URI of the received notification;

NOTE 15:	When the NEF receives QoS monitoring reports from the SMF or UPF, the NEF could determine the affected flows of a QoS monitoring report based on the per flow combination of notification URI and notification correlation ID value(s) provided to the PCF during per flow subscription with the PCF.

When the NEF receives the ECN marking for L4S availability event notification from the PCF as specified in 3GPP TS 29.514 [7], the NEF shall notify the AF with the corresponding "L4S_NOT_AVAILABLE" or "L4S_AVAILABLE" event.

-	if the "MultiMedia" feature is supported, when the NEF receives the event notification for the AF transaction as defined in clause 4.2.2 of 3GPP TS 29.508 [26] or clause 4.2.5.14 of 3GPP TS 29.514 [7], or when the AF requested direct notification, as defined in clause 5.2.2.3 of 3GPP TS 29.564 [61], the NEF shall include the affected single-modal identification number and the corresponding flows within the "multiModFlows" attribute;

-	if the "RTLatency" feature is supported, the AF may include:

-	the indication that the service data flow needs to meet the Round-Trip (RT) latency requirement within the "rTLatencyInd" attribute; and

-	the RT latency requirement of the service data flow either explicitly within the "pdb" attribute or implicitly (to be derived by the PCF) in the "qosReference" attribute;

NOTE 16:	The single direction latency requirement between the UE and the PSA UPF can be either explicitly included within the "req5Gsdelay" attribute or can be derived from the "qosReference" attribute. The twice of the single direction latency is used as the Uplink-Downlink Round Trip latency of the indicated service.

-	if the NEF authorizes the AF request, the NEF shall transfer the received multi-modal service ID and, if applicable, the single-modal data flow(s) information of the multi-modal communication service to the PCF via the Npcf_PolicyAuthorization service.

-	if the "L4S" feature is supported, the AF may include:

-	the Low Latency, Low Loss and Scalable Throughput (L4S) Support within the "l4sInd" attribute. In this case, the AF shall also subscribe to notifications of ECN marking for L4S support information not available in 5GS and available again by including the "L4S_NOT_AVAILABLE" and "L4S_AVAILABLE" events in the "events" attribute. When the NEF receives the ECN marking for L4S availability event notification from the PCF as specified in 3GPP TS 29.514 [7], the NEF shall notify the AF with the corresponding "L4S_NOT_AVAILABLE" or "L4S_AVAILABLE" event;

NOTE 17:	When both, the "L4S" and "EnQoSMon" features are supported, the AF request can include either the indication of L4S support within the "l4sInd" attribute or the request for congestion measurements within the "qosMonConReq" attribute, but the request cannot include both attributes simultaneously. The Individual AS Session with Required QoS Subscription resource cannot contain simultaneously both, the indication of L4S support and the subscription to congestion monitoring.

-	if "PDUSetHandling" feature as defined in clause 5.14.4 of 3GPP TS 29.122 [4] is supported, the AF may include:

-	the protocol description within the "protoDescDl" and/or "protoDescUl" attribute(s) for the UPF to identify the PDU Set Information and/or identify the last PDU of a data burst in the DL traffic and/or for the UE to identify PDU Set information. The protocol description indicates transport protocol (e.g., RTP, SRTP), transport protocol header extensions (e.g., RTP Header Extension for PDU Set Marking in the DL as defined in 3GPP TS 26.522 [74]), payload type and format (e.g., H.264, H.265) used by the service data flow for the DL and/or the UL. In case of the multi-modal data flow(s), each flow may have the respective "protoDescDl" and/or "protoDescUl" attribute(s);

-	the PDU Set QoS parameters, "pduSetQosDl" and/or "pduSetQosUl" attribute(s);

-	if the NEF receives the AF request with PDU Set QoS parameters within the "pduSetQosDl" and/or "pduSetQosUl" attribute(s) and protocol description information within the "protoDescDl" and/or "protoDescUl" attribute(s), the NEF shall forward the attributes to PCF to support the PDU Set QoS configuration by invoking the Npcf_PolicyAuthorization_Create/Update service operation(s);

-	if the "PowerSaving" feature as defined in clause 5.14.4 of 3GPP TS 29.122 [4] is supported, the AF may include:

-	the Uplink and/or Downlink Periodicity information which indicates the time period between the start of the two data bursts in Uplink and/or Downlink direction within the "periodUl" and "periodDl" attributes respectively;

-	the protocol description within the "protoDescDl" attribute, to assist the UPF to identify the End of Burst. In case of the multi-modal data flow(s), each flow may have the respective "protoDescDl" attribute; and

-	if the NEF receives the AF request with the "periodUl", "periodDl" and/or "protoDescDl" attributes, the NEF shall forward the attributes to the PCF to support the UE Power Saving management;

-	if the "QoSTiming_5G" feature as defined in clause 5.14.4 of 3GPP TS 29.122 [4] is supported, NEF shall forward the following attributes to support the QoS Timing information:

-	"qosDuration" attribute to indicate the QoS duration to transfer data traffic (e.g., AI/ML traffic);

-	"qosInactInt" attribute for data traffic (e.g., AI/ML traffic) QoS inactivity interval;

if the NEF authorizes the AF request, the NEF shall provision the received QoS timing parameters to the PCF by invoking the Npcf_PolicyAuthorization service as defined in 3GPP TS 29.514 [7].

-	if the "ExtErrors" feature is supported, the NEF may send the following error responses based on failed request responses received from the 5GC (TSCTSF, as specified in 3GPP TS 29.565 [50], or PCF, as specified in 3GPP TS 29.514 [7]):

a.	if the NEF receives the indication that the 5GC failed in executing session binding, the NEF shall reject the HTTP POST request with an HTTP "500 Internal Server Error" response including the "cause" attribute set to "PDU_SESSION_NOT_AVAILABLE";

b.	if the service information provided in the body of the HTTP POST/PUT/PATCH request is rejected by the 5GC (e.g. the subscribed guaranteed bandwidth for a particular user is exceeded or the authorized data rate in that slice for a UE is exceeded), the NEF shall indicate in an HTTP "403 Forbidden" response message the cause for the rejection including the "cause" attribute set to "REQUESTED_SERVICE_NOT_AUTHORIZED";

c.	if the service information provided in the body of the HTTP POST/PUT/PATCH request is rejected due to a temporary condition in the network, the NEF may include in the "403 Forbidden" response the "cause" attribute set to "REQUESTED_SERVICE_TEMPORARILY_NOT_AUTHORIZED", as received. The NEF may also provide a received retry interval within the "Retry-After" HTTP header field. When the NF service consumer receives the retry interval within the "Retry-After" HTTP header field, the NF service consumer shall not send the same service information to the NEF again (for the same application session context) until the retry interval has elapsed. The "Retry-After" HTTP header is described in 3GPP TS 29.122 [4];

The NEF may additionally provide the acceptable bandwidth within the attribute "acceptableServInfo" included in the "ProblemDetailsAsSessionQos" data structure returned in the rejection response message.

d.	when the request to provision sponsored data connectivity information provided in the body of the HTTP POST/PUT/PATCH request is rejected, the NEF shall reject the request with the received status and error cause, as follows:

1.	HTTP "403 Forbidden" response message with the "cause" attribute set to "UNAUTHORIZED_SPONSORED_DATA_CONNECTIVITY"; or

2.	HTTP "403 Forbidden" response message with the "cause" attribute set to "REQUESTED_SERVICE_NOT_AUTHORIZED".

-	if the "QoSMonCapRepo" feature defined in clause 5.14.4 of 3GPP TS 29.122 [4] is supported:

-	the AF may subscribe to the "QOS_MON_CAP_REPO" event;

-	if the NEF authorizes the AF request, the NEF shall subscribe to the corresponding event at the PCF by invoking the Npcf_PolicyAuthorization service API as defined in 3GPP TS 29.514 [7] or at the TSCTSF by invoking the Ntsctsf_QoSandTSCAssistance service API as defined in 3GPP TS 29.565 [50]; and

-	when the NEF receives the QoS Monitoring Capability Report event notification as defined in 3GPP TS 29.514 [7] or 3GPP TS 29.565 [50], the NEF shall identify the affected flow identifiers and shall notify the AF including in the "qosMonCapRepo" attribute the received information.

Editor's Note:	Whether the QoS Monitoring Capability report can be applied separately to diffent QoS Monitoring Type(i.e., packetdelay, pdv, rtt, etc.) is FFS.

#### 4.4.9.3	Procedures for AF requested QoS for a target UE or group of UE(s) not identified by UE address(es)

When the "GMEC" feature is supported and the AF requested QoS is targeting a UE or group of UE(s) not identified by UE address(es) as defined in clause 4.15.6.14 of 3GPP TS 23.502 [2], the provisions and procedures of clause 4.4.9.2 shall apply with the following differences:

-	Either the "gpsi" attribute or the "extGroupId" attribute shall be used to identify the target UE or group of UE(s) within the "Individual AS Session with Required QoS Subscription" resource representation.

-	After the successful authorization of the request at the NEF, the NEF may further perform service specific authorization as defined in clause 4.15.6.10 of 3GPP TS 23.502 [2] and 3GPP TS 29.503 [63].

-	At the reception of requests from the AF, the NEF shall either invoke the relevant TSCTSF service, as defined in 3GPP TS 29.565 [50], in order to fulfil the request, or when it determines to not invoke the TSCTSF, invoke the Nudr_DataRepository service as defined in 3GPP TS 29.504 [20] and 3GPP TS 29.519 [23] to store the received AF requested QoS information for the target UE or group of UE(s) identified by the SUPI or Internal Group ID (derived respectively from the received GPSI or External Group ID) as Application Data in the UDR.

-	No direct interactions between the NEF and the PCF shall take place.

-	The NEF may receive notification(s) on the subscribed event(s) from the TSCTSF (in case the NEF decided to invoke the TSCTSF at the reception of the corresponding AF required QoS requests) or from the PCF (in case the NEF decided to not invoke the TSCTSF, but rather invoked the Nudr_DataRepository service at the reception of the corresponding AF required QoS request). In the latter case, the PCF notifies the NEF on the subscribed event(s) using the Npcf_PolicyAuthorization_Notify service operation, as specified in 3GPP TS 29.514 [7].

### 4.4.10	Procedures for MSISDN-less Mobile Originated SMS

The procedures are used by the NEF to send the MSISDN-less MO-SMS to the AF in 5GS are described in clause 4.4.14 of 3GPP TS 29.122 [4] with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	the NEF shall interact with UDM by using Nudm_SubscriberDataManagement service (as defined in 3GPP TS 29.503 [17]) to retrieve the external identifier; and

-	the NEF may receive an MSISDN-less MO-SMS via SM11 including a destination SME address (long/short code of the AF) upon the SMS-SC invoking Nnef_SMService.

### 4.4.11	Procedures for Network Configuration Parameters Provisioning

The procedures for network configuration parameters provisioning as described in clause 4.4.12 of 3GPP TS 29.122 [4] shall be applicable in 5GS with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	description of the HSS applies to the UDM;

-	the NEF shall interact with the UDM by using Nudm_ParameterProvision service as specified in 3GPP TS 29.503 [17]; and

-	if the "UEId_retrieval" feature defined in clause 5.13.4 of 3GPP TS 29.122 [4] is supported, in order to support the AF specific UE ID retrieval:

1)	the AF may request AF specific UE ID retrieval for an individual UE, by providing the UE's IP address in the "ueIpAddr" attribute or the UE's MAC address in the "ueMacAddr" attribute within the NpConfiguration data type;

2)	the AF may also provide the DNN, within the "dnn" attribute, and/or the S-NSSAI, within the "snssai" attribute, within the NpConfiguration data type;

3)	upon reception of the corresponding request message from the AF:

-	if the AF's request for AF specific UE ID retrieval is not authorized, the NEF shall respond to the AF with a "403 Forbidden" status code with the response body including the ProblemDetails data structure containing the "cause" attribute set to the "REQUEST_NOT_AUTHORIZED" application error indicating AF authorisation failure; and

-	if the AF's request for AF specific UE ID retrieval is authorized by the NEF, then if the DNN and/or S-NSSAI information is not available in the request, the NEF shall determine the corresponding DNN and/or S-NSSAI information based on the received requesting AF Identifier, and if provided, the MTC Provider Information;

4)	the NEF shall then interact with the BSF with the UE address and IP domain (if the UE IPv4 address is provided), DNN and/or S-NSSAI to retrieve the session binding information of the UE by invoking the Nbsf_Management_Discovery service operation as described in 3GPP TS 29.521 [9];

5)	if the NEF receives an error response from the BSF, the NEF shall respond to the AF with a proper error status code. If the NEF received from the BSF an error response including a "ProblemDetails" data structure with the "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error. If no SUPI matching the provided UE information is returned by the BSF, the NEF shall respond to the AF with a "404 Not Found" status code with the response body including a ProblemDetails data structure containing the "cause" attribute set to the "UE_NOT_FOUND" application error to indicate that the requested UE address is not found;

6)	upon success and a SUPI is returned by the BSF, the NEF shall interact with the UDM to retrieve the AF specific UE Identifier using the received SUPI and at least one of the Application Port ID, MTC Provider Information or AF Identifier information by invoking Nudm_SDM_Get service as described in clause 5.2.2.2 of 3GPP TS 29.503 [17];

7)	upon success, the UDM responds to the NEF with an AF specific UE Identifier represented as an External Identifier for the UE which is uniquely associated with the MTC provider Information and/or AF Identifier. The NEF shall then respond to the AF with the received information, i.e. the AF specific UE Identifier represented as an External Identifier that was received from the UDM;

8)	if the NEF receives an error response from the UDM, the NEF shall respond to the AF with a proper error status code. If the NEF received from the UDM an error response including a "ProblemDetails" data structure with the "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error. If the UDM indicates that the requested UE Identifier is not available in the subscription data, the NEF shall respond to the AF with a "404 Not Found" error status code with the response body including a ProblemDetails data structure containing the "cause" attribute set to the "UE_ID_NOT_AVAILABLE" application error to indicate that the AF specific UE ID is not available.

NOTE:	The case where UE IP address provided by the AF to the NEF corresponds to an IP address that has been NATed (Network and Port Address Translation) is not supported in this release.

### 4.4.12	Procedures for Non-IP data delivery

#### 4.4.12.1	General

The procedures are used by the NEF to send/receive the non-IP data to/from the AF. It comprises NIDD configuration and NIDD delivery.

The NIDD configuration may be triggered by the NEF or the AF. If it is triggered by the NEF, the NiddConfigurationTrigger API described in clause 5.5 is used and the procedure is described in clause 4.4.12.2.

#### 4.4.12.2	NIDD configuration Triggered by the NEF

If the NEF receives a NIDD connection establishment request from the SMF and if there is no NIDD configuration for the UE, the NEF may send a NIDD configuration trigger to the AF. The NEF determines the destination URI by local configuration. The NEF shall send to the determined destination URL an HTTP POST request that shall include a NiddConfiguarationTrigger data type with:

-	the NEF identifier,

-	the AF identifier, and

-	GPSI as UE identity.

The AF shall acknowledge the HTTP POST request with an HTTP 200 OK response. Then the AF may start NIDD configuration procedure as described in clause 4.4.12.3.

#### 4.4.12.3	NIDD configuration triggered by the AF and NIDD delivery

The procedures for NIDD configuration triggered by the AF and NIDD delivery are described in clause 4.4.5 of 3GPP TS 29.122 [4] with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	description of the MME/SGSN applies to the SMF;

-	for the connection establishment, the interaction between the NEF and the SMF shall use Nnef_SMContext service as specified in 3GPP TS 29.541 [24];

-	for MO NIDD, the interaction between the SMF and the NEF shall use Nnef_SMContext service as specified in 3GPP TS 29.541 [24]; and

-	for MT NIDD, the interaction between the SMF and the NEF shall use Nsmf_NIDD service as specified in 3GPP TS 29.542 [25].

### 4.4.13	Procedures for RACS Parameter Provisioning

The procedures for RACS parameter provisioning as described in clause 4.4.15 of 3GPP TS 29.122 [4] shall be applicable in 5G with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF.

### 4.4.14	Procedures for analytics information exposure

#### 4.4.14.1	Subscription/unsubscription to notification of analytics information

The procedures are used by the AF to subscribe/unsubscribe to retrieve analytics information via NEF, and are used by the NEF to notify the AF about the requested analytics information as described in 3GPP TS 23.288 [29].

In order to subscribe to retrieve analytics information, the AF shall send an HTTP POST message to the NEF to the resource "Analytics Exposure Subscriptions", the HTTP POST request message body shall include the AnalyticsExposureSubsc data structure that shall include:

-	the URI where to receive the requested notifications as "notifUri" attribute;

-	the Notification Correlation Identifier assigned by the NF service consumer for the requested notifications as "notifId" attribute; and

-	a description of the subscribed events as "analyEventsSubs" attribute that shall include for each event:

1)	an event identifier as "analyEvent" attribute.

The AnalyticsExposureSubsc data structure may include:

-	event reporting requirement information as "analyRepInfo" attribute, which applies for all events in a subscription and may contain the following attributes:

1)	event notification method (periodic, one time, on event detection) as "notifMethod" attribute;

2)	maximum Number of Reports as "maxReportNbr" attribute;

3)	monitoring Duration as "monDur" attribute;

4)	repetition period for periodic reporting as "repPeriod" attribute;

5)	immediate reporting indication as "immRep" attribute;

6)	sampling ratio as "sampRatio" attribute;

7)	group reporting guard time as "grpRepTime" attribute;

8)	partitioning criteria for partitioning the impacted UEs before performing sampling as "partitionCriteria" attribute if the "EneNA" feature is supported; and

9)	a notification flag (used for muting and retrieving notifications) as "notifFlag" attribute if the "EneNA" feature is supported.

Each AnalyticsEventSubsc data structure may include:

-	event specific filters via the "analyEventFilter" attribute; and

-	the indication of the UEs to which the subscription applies via "tgtUe" attribute, which if provided shall include one of the following attributes:

1)	identification of an individual UE via a "gpsi" attribute;

2)	identification of a group of UE(s) via a "exterGroupId" attribute; or

3)	identification of any UE via the "anyUeInd" attribute.

Upon receipt of the HTTP POST request from the AF, if the AF is authorized, the NEF shall interact with the UDM by using Nudm_SubscriberDataManagement service as defined in 3GPP TS 29.503 [17] to translate the GPSI or external group identifier into the corresponding SUPI or internal group identifier. If the NEF receives an error response from the UDM, the NEF shall not create the resource and shall respond to the AF with a proper error status code. If the NEF receives from the UDM an error response including a "ProblemDetails" data structure with the "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable. After receiving a successful response from the UDM, the NEF may perform further mappings and translations (e.g. map application identifiers to DNN and S-NSSAI information, or translate attributes of data type NetworkAreaInfo to attributes of data type LocationArea5G) and it shall interact with the NWDAF to subscribe to the subscription to the analytics information by using the Nnwdaf_EventsSubscription service as defined in 3GPP TS 29.520 [27]. If the NEF receives an error responsefrom the NWDAF, the NEF shall not create the resource and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

In order to update an existing analytics exposure subscription, the AF shall send an HTTP PUT message to the NEF to the resource "Individual Analytics Exposure Subscription" requesting to change the subscription.

In order to delete an existing analytics exposure subscription, the AF shall send an HTTP DELETE message to the NEF to the resource "Individual Analytics Exposure Subscription".

Upon receipt of the HTTP PUT or DELETE request from the AF, if the AF is authorized, the NEF may perform further mappings and translations (e.g. map application identifiers to DNN and S-NSSAI information, or translate attributes of data type LocationArea5G to attributes of data type NetworkAreaInfo as required by the data model) and it shall interact with the NWDAF to modify or cancel the subscription to the analytics information by using the Nnwdaf_EventsSubscription service as defined in 3GPP TS 29.520 [27]. If the NEF receives an error responsefrom the NWDAF, the NEF shall not update or delete the resource and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

After receiving a successful response from the NWDAF, the NEF shall:

-	for the HTTP POST request, create a resource "Individual Analytics Exposure Subscription" which represents the analytics exposure subscription, addressed by a URI that contains the AF Identifier and an NEF-created subscription identifier, and shall respond to the AF with a 201 Created status code, including a Location header field containing the URI for the created resource. The AF shall use the URI received in the Location header in subsequent requests to the NEF to refer to this analytics exposure subscription. If not all the requested analytics events in the subscription are accepted, then the NEF may include the "failEventReports" attribute indicating the event(s) for which the subscription failed and the associated reason(s):

-	for the HTTP PUT request, update a resource "Individual Analytics Exposure Subscription" which represents the analytics exposure subscription, and shall responds to the AF with a 200 OK or 204 No Content status code. When responding with a 200 OK status code, if not all the requested analytics events in the subscription are modified successfully, then the NEF may include the "failEventReports" attribute indicating the event(s) for which the modification failed and the associated reason(s); and

-	for the HTTP DELETE request, remove all properties of the resource and delete the corresponding active resource "Individual Analytics Exposure Subscription" which represents the analytics exposure subscription, then shall responds to the AF with a 204 No Content status code.

If the immediate reporting indication in the "immRep" attribute within the "analyRepInfo" attribute sets to true during the HTTP POST or PUT request, the NEF shall also include the reports of the events subscribed, if available, in the HTTP POST or PUT response to the AF.

If the NEF receives an analytics information notification from the NWDAF indicating that the subscribed analytics event has been detected, the NEF may perform further mappings and translations (e.g. translate attributes of data type NetworkAreaInfo to attributes of data type LocationArea5G as required by the data model), it may determine based on local configuration to hide from the Untrusted AF network internal information (e.g. DNN, S-NSSAI) which was included in the NWDAF notification, and it shall provide a notification by sending HTTP POST message that include the AnalyticsEventNotification data structure at least with the detected analytics event to the AF identified by the notification URI together with the notification correlation identifier received during creation/modification of the Individual Analytics Exposure Subscription. Upon receipt of the analytics event notification, the AF shall respond with a "204 No Content" status code to confirm the received notification.

When the "notifFlag" attribute is included during the creation of a subscription (HTTP POST request) and set to "DEACTIVATE", the NEF shall mute the event notification and store the available events until the NF service consumer requests to retrieve them by setting the "notifFlag" attribute to "RETRIEVAL" or until a muting exception occurs (e.g. full buffer).

When the "notifFlag" attribute is included during the update of a subscription (HTTP PUT request) and set to "DEACTIVATE", the NEF shall mute the event notification and store the available events until the NF service consumer requests to retrieve them by setting the "notifFlag" attribute to "RETRIEVAL" or until a muting exception occurs (e.g. full buffer); if the "notifFlag" attribute is set to the value "RETRIEVAL", the NEF shall send the stored events to the NF service consumer, mute the event notification again and store available events; if the "notifFlag" attribute is set to the value "ACTIVATE" and the event notifications are muted (due to a previously received "DECATIVATE" value), the NWDAF shall unmute the event notification, i.e. start sending again notifications for available events.

#### 4.4.14.2	Fetch analytics information

The procedures are used by the AF to fetch analytics information via NEF.

In order to fetch analytics information, the AF shall send an HTTP POST request message to the NEF targetingthe custom operation URI "{apiRoot}/3gpp-analyticsexposure/v1/{afId}/fetch", the HTTP POST request message body shall include the AnalyticsRequest data structure that shall include:

-	the identification of the analytics events, encoded within the "analyEvent" attribute;

and may include:

-	the description of the analytics reporting information, encoded within the "analyRep" attribute;

-	an event filter, encoded within the "analyEventFilter" attribute.

-	the indication of the UEs to which the analytics request applies via either:

a)	the identification of an individual UE via the "gpsi" attribute;

b)	the identification of a group of UE(s) via the "exterGroupId" attribute; or

c)	the identification of any UE via the "anyUeInd" attribute.

Upon the reception of an HTTP POST request, if the AF is authorized, the NEF shall interact with the UDM by using Nudm_SubscriberDataManagement service as defined in 3GPP TS 29.503 [17] to translate the GPSI or external group identifier into the corresponding SUPI or internal group identifier. If the NEF receives an error response from the UDM, the NEF shall respond to the AF with a proper error status code. If the NEF receives from the UDM an error response including a "ProblemDetails" data structure with the "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable. After receiving a successful response from the UDM, the NEF may perform further mappings and translations (e.g. map application identifiers to DNN and S-NSSAI information, or translate attributes of data type NetworkAreaInfo to attributes of data type LocationArea5G) and it shall interact with the NWDAF by using Nnwdaf_AnalyticsInfo service as defined in 3GPP TS 29.520 [27]. If the NEF receives an error responsefrom the NWDAF, the NEF shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable. If a successful response including analytics information is received from the NWDAF, the NEF shall translate the network internal information to external information (e.g. SUPI to GPSI, Internal Group ID to External Group ID, attributes of data type NetworkAreaInfo to attributes of data type LocationArea5G), it may determine based on local configuration to hide from the Untrusted AF network internal information (e.g. DNN, S-NSSAI) which was included in the NWDAF response, and it shall send an HTTP POST response to the AF by including analytics information within the AnalyticsData data structure.

### 4.4.15	Procedures for 5G LAN Parameter Provisioning

#### 4.4.15.1	General

The procedures are used by the AF to provision 5G LAN type service related parameters to the NEF. The following procedures support:

-	Management of 5G Virtual Network group membership;

-	Management of 5G Virtual Network group data; and/or

-	Management of 5G Virtual Network group parameters provisioning.

#### 4.4.15.2	Creation of a new subscription for 5G LAN parameter provisioning

In order to create a new subscription to provision 5G LAN related parameters, the AF shall initiate an HTTP POST request to the NEF for the "5GLAN Parameters Provision Subscriptions" resource. The body of the HTTP POST message shall include the 5G LAN service related parameters within the "5gLanParams" attribute.

When the "GMEC" feature is supported, the AF may also provision the 5G VN group related parameters (e.g., LPI parameters, ECS Address information, Network Parameter Configuration).

Upon receipt of the corresponding HTTP POST message, if the AF is authorized by the NEF to provision the parameters, the NEF shall interact with the UDM to create a subscription at the UDM by using Nudm_ParameterProvision service as defined in 3GPP TS 29.503 [17]. If the request is accepted by the UDM and the UDM informs the NEF with a successful response, the NEF shall create a new subscription and assign a subscription identifier for the "Individual 5GLAN Parameters Provision Subscription" resource. Then the NEF shall send a HTTP "201 Created" response with 5GLanParametersProvision data structure as response body and a Location header field containing the URI of the created individual subscription resource.

#### 4.4.15.3	Modification of an existing subscription for 5G LAN parameter provisioning

To modify an existing subscription to provision 5G LAN parameters, the AF shall initiate an HTTP PUT/PATCH request to the NEF for the "Individual 5GLAN Parameters Provision Subscription" resource. The body of the HTTP PUT message shall include the 5GLanParametersProvision data type as defined in clause 5.7.2.3.2. The body of the HTTP PATCH message shall include the 5GLanParametersProvisionPatch data as defined in clause 5.7.2.3.5. The External Group Identifier, DNN, S-NSSAI and PDU session type(s) within the "5gLanParams" attribute shall not be updated/modified.

When the "GMEC" feature is supported, the AF may also update/modify the 5G VN group related parameters (e.g., LPI parameters, ECS Address information, Network Parameter Configuration) within the "5gLanParams" attribute.

Upon reception of the corresponding HTTP PUT/PATCH message, if the AF is authorized by the NEF to provision the parameters, the NEF shall interact with the UDM to modify an existing subscription at the UDM by using Nudm_ParameterProvision service as defined in 3GPP TS 29.503 [17]. If the modification request is accepted by the UDM and the UDM informs the NEF with a successful response, the NEF shall update the existing subscription for the "Individual 5GLAN Parameters Provision Subscription" resource. Then the NEF shall send a HTTP response including "200 OK" status code with 5GLanParametersProvision data structure or "204 No Content" status code.

#### 4.4.15.4	Deletion of an existing subscription for 5G LAN parameter provisioning

To delete an existing subscription to 5GLAN provision parameters, the AF shall initiate an HTTP DELETE request to the NEF for the "Individual 5GLAN Parameters Provision Subscription" resource.

Upon receipt of the corresponding HTTP DELETE message, if the AF is authorized, the NEF shall interact with the UDM to delete an existing parameters provision subscription at the UDM by using Nudm_ParameterProvision service as defined in 3GPP TS 29.503 [17]. If the request is accepted by the UDM and informs the NEF with a successful response, the NEF shall delete the existing subscription for the "Individual 5GLAN Parameters Provision Subscription" resource. Then the NEF shall send a HTTP "204 No Content" response.

#### 4.4.15.5	5G LAN parameter provisioning event notification

If the "GMEC" feature is supported and a previously subscribed AF shall be notified on 5G LAN parameters provisioning event(s) (e.g., when Network Parameters Configuration information is provisioned), the NEF shall initiate an HTTP POST request to the AF targeting the notification URI provided during the creation of the corresponding "Individual 5GLAN Parameters Provision Subscription" resource within the "notifUri" attribute.

Upon successful reception and processing of the HTTP POST request message, the AF shall acknowledge the successful reception of the notification by responding with an HTTP "204 No Content" status code.

On failure, the AF shall take proper error handling actions and respond to the AF with an appropriate error status code as specified in clause 5.7.4.

### 4.4.16	Procedures for applying BDT policy

In order to create a resource for the applying a previously negotiated Background Data Transfer Policy to a UE or a Group of UEs, the AF shall send an HTTP POST message to the NEF to the resource "Applied BDT Policy Subscriptions". The body of the HTTP POST message shall contain the external Group Identifier or external Identifier, and the Background Data Transfer Reference ID for a previously negotiated policy of a background data transfer.

Upon receipt of the HTTP POST request from the AF, if the AF is authorized, the NEF shall interact with the UDM by invoking the Nudm_SubscriberDataManagement service as described in 3GPP TS 29.503 [17] to retrieve the SUPI or Internal Group Identifier.

In order to update an existing applied BDT policy subscription, the AF shall send an HTTP PATCH message to the resource "Individual Applied BDT Policy Subscription" requesting to change the applied BDT policy. The AF shall include in the body of the HTTP PATCH request the new Background Data Transfer Reference ID.

In order to delete an existing applied BDT policy subscription, the AF shall send an HTTP DELETE message to the NEF to the resource "Individual Applied BDT Policy Subscription".

The NEF shall interact with the UDR by invoking the Nudr_DataRepository service as described in 3GPP TS 29.504 [20], if the NEF receives an error responsefrom the UDR, the NEF shall not create, update or delete the resource and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

After receiving a successful response from the UDR, the NEF shall:

-	for the HTTP POST request, create a resource "Individual Applied BDT Policy Subscription" addressed by a URI that contains the AF Identifier and an NEF-created subscription identifier, and shall respond to the AF with a "201 Created" status code, including a Location header field containing the URI of the created resource. The AF shall use the URI received in the Location header in subsequent requests to the NEF to refer to this resource;

-	for the HTTP PATCH request, update a resource "Individual Applied BDT Policy Subscription" which represents the applied BDT policy subscription, and shall respond to the AF with a "200 OK" or "204 No Content" status code; and

-	for the HTTP DELETE request, delete the corresponding active resource "Individual Applied BDT Policy Subscription", and shall respond to the AF with a "204 No Content" status code.

### 4.4.17	Procedures for Enhanced Coverage Restriction Control

The procedures for network configuration parameters provisioning as described in clause 4.4.11 of 3GPP TS 29.122 [4] shall be applicable in 5GS with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	description of the HSS applies to the UDM; and

-	upon receipt of HTTP POST request from the AF to query the current status of enhanced coverage restriction, the NEF shall interact with the UDM by using the Nudm_SubscriberDataManagement service as specified in 3GPP TS 29.503 [17].

-	upon receipt of HTTP POST request from the AF to configure the enhanced converage restriction, the NEF shall interact with the UDM by using the Nudm_ParameterProvision service as specified in 3GPP TS 29.503 [17].

-	if the ECR_WB_5G feature is supported, in order to configure the enhanced coverage restriction for WB UE, the HTTP POST request message shall include the WB mode related enhanced coverage restriction information via the "ecrDataWbs" attribute for the WB UE.

### 4.4.18	Procedures for IPTV Configuration

The procedures are used by the AF to authorize the request and forward the request for IPTV configuration information via NEF.

In order to configure IPTV information, the AF shall send an HTTP POST message to the NEF to the resource "IPTV Configurations", the HTTP POST request message body shall include the IptvConfigData data structure that shall include:

-	indication of the UEs to which the subscription applies via:

a)	identification of an individual UE via a "gpsi" attribute; or

b)	identification of a group of UE(s) via a "exterGroupId" attribute;

-	an application identifier as "appId" attribute; and

-	a list of Multicast Access Control as "multiAccCtrls" attribute;

and may include:

-	an DNN as "dnn" attribute;

-	an S-NSSAI as "snssai" attribute; and

-	MTC Provider Information as "mtcProviderId" attribute.

NOTE:	The NEF can check the received MTC Provider Id information and reject the IPTV configuration request upon failure checking result.

In order to update an existing individual IPTV configuration, the AF shall send an HTTP PUT or HTTP PATCH message to the NEF to the resource "Individual IPTV Configuration" requesting to change the subscription. The External Group Identifier, GPSI, DNN, S-NSSAI and Application Identifier shall remain unchanged from previous values in the HTTP PUT message.

In order to delete an existing individual IPTV configuration, the AF shall send an HTTP DELETE message to the NEF to the resource "Individual IPTV Configuration".

Upon receipt of the HTTP request from the AF, if the AF is authorized, the NEF shall interact with the UDM by invoking the Nudm_SubscriberDataManagement service as described in 3GPP TS 29.503 [17] to retrieve the SUPI or Internal Group Identifier. Then the NEF shall interact with the UDR to create, update or delete the IPTV configuration by using the Nudr_DataRepository service as defined in 3GPP TS 29.519 [23]. If the NEF receives an error responsefrom the UDR, the NEF shall not create, update or delete the resource and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

After receiving a successful response from the UDR, the NEF shall:

-	for the HTTP POST request, create a resource "Individual IPTV Configuration" which represents the IPTV configuration request, addressed by a URI that contains the AF Identifier and an NEF-created configuration identifier, and shall respond to the AF with a 201 Created status code, including a Location header field containing the URI for the created resource. The AF shall use the URI received in the Location header in subsequent requests to the NEF to refer to this IPTV configuration.

-	for the HTTP PUT or HTTP PATCH request, update a resource "Individual IPTV Configuration" which represents the IPTV configuration, and shall responds to the AF with a 200 OK or 204 No Content status code; and

-	for the HTTP DELETE request, remove all properties of the resource and delete the corresponding active resource "Individual IPTV Configuration", then shall responds to the AF with a 204 No Content status code.

### 4.4.19	Procedures for Location Privacy Indication Parameters Provisioning

The procedures are used by the AF to provision Location Privacy Indication parameters to the NEF. The procedures are applicable for an individual UE or a group of UEs.

In order to provision Location Privacy Indication parameters, the AF shall initiate an HTTP POST request to the NEF for the "LPI Parameters Provisionings" resource. The body of the HTTP POST message shall include the Location Privacy Indication related parameters within the LpiParametersProvision data structure.

Upon receipt of the corresponding HTTP POST message, if the AF is authorized by the NEF to provision the parameters, the NEF shall interact with the UDM to create a resource at the UDM by using Nudm_ParameterProvision service as defined in 3GPP TS 29.503 [17]. If the request is accepted by the UDM and the UDM informs the NEF with a successful response, the NEF shall create a new resource and assign an identifier for the "Individual LPI Parameters Provisioning" resource. Then the NEF shall send a HTTP "201 Created" response with LpiParametersProvision data structure as response body and a Location header field containing the URI of the created individual resource.

In order to update an existing individual LPI Parameters Provisioning, the AF may send an HTTP PUT message to the resource " Individual LPI Parameters Provisioning" requesting the NEF to change all properties in the existing resource. The body of the HTTP PUT request message shall include LpiParametersProvision data type as defined in clause 5.10.2.3.2. The External Group Identifier or GPSI shall remain unchanged from previous values.

If the "PatchUpdate" feature defined in clause 5.10.3 is supported, in order to partially modify an existing LPI Parameters Provisioning resource, the AF may send an HTTP PATCH request message to the NEF on the "Individual LPI Parameters Provisioning" resource, with the request body containing the LpiParametersProvisionPatch data structure including only the attributes that shall be updated.

Upon receipt of the corresponding HTTP PUT/PATCH request message, if the AF is authorized by the NEF to provision the parameters, the NEF shall interact with the UDM to modify an existing resource at the UDM by using Nudm_ParameterProvision service as defined in 3GPP TS 29.503 [17]. If the modification request is accepted by the UDM and the UDM informs the NEF with a successful response, the NEF shall update the existing resource for the "Individual LPI Parameters Provisioning" resource. Then the NEF shall send a HTTP response including "200 OK" status code with LpiParametersProvision data structure or "204 No Content" status code.

To delete an existing individual LPI Parameters Provisioning, the AF shall initiate an HTTP DELETE request to the NEF for the "Individual LPI Parameters Provisioning" resource.

Upon receipt of the corresponding HTTP DELETE message, if the AF is authorized, the NEF shall interact with the UDM to delete an existing LPI Parameters Provisioning at the UDM by using Nudm_ParameterProvision service as defined in 3GPP TS 29.503 [17]. If the request is accepted by the UDM, the NEF shall delete the existing resource for the "Individual LPI Parameters Provisioning" resource. Then the NEF shall send a HTTP "204 No Content" response.

### 4.4.20	Procedures for service specific parameter provisioning

These procedures are used by an AF to provide service specific parameters to the 5G system via the NEF.

In order to provision service specific parameters to the 5G system, the AF shall send an HTTP POST message to the NEF targetting the resource "Service Parameter Subscriptions", the HTTP POST request message body shall include the ServiceParameterData data structure that shall include:

-	service description via one of the following:

a)	a combination of DNN and S-NSSAI within the "dnn" attribute and the "snssai" attribute respectively;

b)	an AF Service Identifier within the "afServiceId" attribute. In this case, the NEF may translate the received AF service identifier into a DNN and S-NSSAI combination; or

c)	an application identifier within the "appId" attribute;

NOTE 1:	When the feature "AfGuideURSP" is supported, the DNN, S-NSSAI and/or Application Identifier information can be provided in the "urspGuidance" attribute, hence only the "afServiceId" attribute needs to be included for providing guidance for URSP determination. When the "AfGuideTNAPs" feature is supported, and the attribute "tnaps" is included, the "appId" attribute cannot be included.

-	indication of the UEs to which the subscription applies via one of the following:

a)	identification of an individual UE within the "gpsi" attribute;

b)	an IPv4 address of the UE within the "ueIpv4" attribute;

c)	an IPv6 address of the UE within the "ueIpv6" attribute;

d)	a MAC address of the UE within the "ueMac" attribute;

e)	an identification of a group of UE(s) within the "externalGroupId" attribute;

NOTE 2:	When the feature "PIN" is supported, AF can use "externalGroupId" attribute to indicate the external group identifier if more than one PEGC is present within the PIN. If external group identifier is not used for the PIN, then AF will indicate "gpsi" attribute in the individual request for each of the PEGC within the PIN.

f)	an identification of any UE within the "anyUeInd" attribute; or

g)	when the feature "VPLMNSpecificURSP" is supported, the AF is interacting with the VPLMN, and the request is to influence the determination of VPLMN-specific URSP rules for any inbound roamer from one or more PLMN(s), an identification of the PLMN IDs of the roaming UEs within the "roamUeNetDescs" attribute; and

-	service parameters for at least one of the following:

1)	V2X service parameters via:

a)	configuration parameters for V2X communications over PC5 within the "paramOverPc5" attribute; and

b)	configuration parameters for V2X communications over Uu within the "paramOverUu" attribute;

2)	if the "ProSe" and/or "ProSe_Ph2" feature(s) is/are supported, 5G ProSe service parameters via:

a)	configuration parameters for 5G ProSe direct discovery within the "paramForProSeDd" attribute;

b)	configuration parameters for 5G ProSe direct communication within the "paramForProSeDc" attribute; and

c)	configuration parameters for 5G ProSe UE-to-network relay, including configuration parameters for 5G ProSe UE-to-network relay UE within the "paramForProSeU2NRelUe" attribute and configuration parameters for 5G ProSe remote UE within the "ParamForProSeRemUe" attribute;

d)	configuration parameters for 5G ProSe UE-to-UE relay, including configuration parameters for 5G ProSe UE-to-UE relay UE within the "paramForProSeU2URelUe" attribute and configuration parameters for 5G ProSe end UE within the "ParamForProSeEndUe" attribute, only if the "ProSe_Ph2" feature is supported;

3)	if the "AfGuideURSP" feature is supported, URSP service parameters via:

a)	contents for the AF guidance on URSP within the "urspGuidance" attribute, which shall include one or more URSP rule requests. Each URSP rule request may include:

1.	a traffic descriptor within the "trafficDesc" attribute;

-	if the "PIN" feature is supported and the provided URSP request applies to a PIN scenario, the traffic descriptor shall correspond to a PIN Identifier within the "pinId" attribute applicable for the PEGC;

2.	a relative precedence within the "relatPrecedence" attribute; and/or

3.	one or more route selection parameter sets within the "routeSelParamSets" attribute. Each route selection parameter set may include a precedence value within the "precedence" attribute, a DNN within the "dnn" attribute, an S-NSSAI within the "snssai" attribute, a spatial validity condition within the "spatialValidity" attribute, and if the "PduSessTypeChange" feature is also supported and the PDU Session type needs to be changed, the requested PDU Session type within the "pduSessType" attribute. If the request contains only one route selection parameter set, each of the optional attributes "dnn", "snssai", "precedence", and "spatialValidity" that is missing from the request may be complemented by the NEF based on local configuration for the provided AF service identifier. It is up to the NEF to transform the information of the "spatialValidity" attribute into a list of TAIs;

NOTE 3:	If the "PIN" feature is supported and the provided URSP request applies to a PIN scenario, the DNN and S-NSSAI need to be included.

b)	when the feature "VPLMNSpecificURSP" is supported, the "vpsUrspGuidance" attribute, where each URSP rule may contain the parameters of bullet a) above, and in addition, the description of the VPLMN(s) where the URSP rule applies within the "visitedNetDescs" attribute;

4)	if the "A2X" feature is supported, A2X service parameters via:

a)	configuration parameters for A2X communications over PC5 within the "a2xParamsPc5" attribute;

b)	configuration parameters for A2X communications over Uu within the "a2xParamsUu" attribute;

5)	if the "AfGuideTNAPs" feature is supported, TNAP ID(s) service parameters via:

a)	a list of the TNAP ID(s) collocated with the 5G-RG(s) of a specific user within the "tnaps" attribute;

NOTE 4:	When the "AfGuideTNAPs" feature is supported and the AF provides the "tnaps" attribute, the service specific parameter provisioning procedure is used for the provisioning of UE location related information to be applied for SM Policy Control.

and

6)	if the "Ranging_SL" feature is supported, the Ranging and sidelink positioning service parameters including:

a)	configuration parameters for ranging and sidelink positioning within the "paramForRangingSlPos" attribute;

and may include:

-	if the "AfNotifications" feature is supported:

a)	subscription to event notification of the outcome related to invocation of service parameter provisioning within the "subNotifEvents" attribute; and

b)	notification URI within the "notificationDestination" attribute.

In order to update an existing service parameter subscription, the AF shall send an HTTP PUT or HTTP PATCH message to the NEF targetting the resource "Individual Service Parameter Subscription" and requesting to change the subscription. When the HTTP PUT method is used, the NF service consumer should not update attributes that do not exist in the ServiceParameterDataPatch data type, i.e. such attributes should remain unchanged compared to the initial values provided in the HTTP POST request message.

In order to delete an existing service parameter subscription, the AF shall send an HTTP DELETE message to the NEF targetting the resource "Individual Service Parameter Subscription".

In non-roaming scenarios or roaming scenarios when the AF interacts with the HPLMN, upon receipt of the HTTP request from the AF, and if the AF is authorized, the NEF shall interact with the UDM by invoking the Nudm_SubscriberDataManagement service as described in 3GPP TS 29.503 [17] to retrieve the SUPI or Internal Group Identifier.

The NEF may, based on local configuration, complement missing service parameters. Additionally, based on operator's local policy, NEF may support service specific authorization as described in clause 4.15.6.10 in 3GPP TS 23.502 [2]. Then the NEF shall interact with the UDR to create, update or delete the associated service parameters by using the Nudr_DataRepository service as defined in 3GPP TS 29.519 [23]. If information related to AfNotifications feature are received from the AF, the NEF shall also include the required information (e.g. "policDelivNotifUri" and "policDelivNotifCorreId" attributes in 3GPP TS 29.519 [23]) in UDR data creation if the NEF supports the DeliveryOutcome feature (as described in 3GPP TS 29.504 [4]).

If the NEF receives an error response from the UDR or UDM, the NEF shall not create, update or delete the resource and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

After receiving a successful response from the UDR, the NEF shall:

-	for an HTTP POST request, create an "Individual Service Parameter Subscription" resource which represents the Service Parameter provisioning request, addressed by a URI that contains the AF Identifier and a NEF-created configuration identifier, and shall respond to the AF with a 201 Created status code, including a Location header field containing the URI for the created resource. The AF shall use the URI received in the Location header in subsequent requests to the NEF to refer to this Service Parameter Subscription;

-	for an HTTP PUT or HTTP PATCH request, update the "Individual Service Parameter Subscription" resource which represents the service parameter provisioning request, and respond to the AF with a 200 OK or 204 No Content status code; and

-	for an HTTP DELETE request, remove all properties of the resource and delete the corresponding active "Individual Service Parameter Subscription" resource, then respond to the AF with a 204 No Content status code.

When the NEF receives the Service Specific Authorization Update information from the UDM by Nudm_ServiceSpecificAuthorization_UpdateNotify service operation defined in 3GPP TS 29.503 [17], if the authorization is revoked, the NEF shall provide a notification to AF by sending HTTP POST message that include the one or more AfNotification data structure(s). Upon receipt of the notification, the AF shall respond with a "204 No Content" status code to confirm the received notification.

When the NEF receives the notification of the outcome of invocation related to AF provisioned service parameters from the PCF by Npcf_EventExposure_Notify service operation defined in 3GPP TS 29.523 [22], the NEF shall determine the corresponding service parameter subscription and provide a notification to AF by sending HTTP POST message that include the AfNotification data structure. Upon receipt of the notification, the AF shall respond with a "204 No Content" status code to confirm the received notification.

In the roaming scenarios when the AF interacts with the VPLMN, the interaction of the V-NEF with the UDM does not apply. The V-NEF stores in the V-UDR the service parameter information provided by the AF and receives from the V-PCF the notification of the outcome of the provisioning of the AF requested service parameters.

### 4.4.21	Procedures for ACS configuration parameter provisioning

The procedures are used by the AF to provide ACS configuration information to 5G system via NEF.

In order to provision the ACS configuration information, the AF shall send an HTTP POST message to the NEF to the resource "ACS Configuration Subscriptions", the HTTP POST message shall include AcsConfigurationData data structure as request body. The AcsConfigurationData data structure shall include:

-	the URL of the ACS or the address of the ACS within the "acsInfo" attribute; and

-	indication of the UEs to which the subscription applies via:

a)	identification of an individual UE via a "gpsi" attribute; or

b)	identification of a group of UE(s) via a "exterGroupId" attribute.

In order to update an existing ACS configuration subscription, the AF shall send an HTTP PUT message to the NEF to the resource "Individual ACS Configuration Subscription" requesting to change the subscription. The body of the HTTP PUT request message shall include AcsConfigurationData data type. The External Group Identifier or GPSI shall remain unchanged from previous values.

If the "PatchUpdate" feature defined in clause 5.12.3 is supported, in order to partially modify an existing ACS Configuration subscription, the AF shall send an HTTP PATCH request message to the NEF on the "Individual ACS Configuration Subscription" resource, with the request body containing the AcsConfigurationDataPatch data structure including only the attributes that shall be modified.

In order to delete an existing ACS configuration subscription, the AF shall send an HTTP DELETE message to the NEF to the resource "Individual ACS configuration Subscription".

Upon receipt of the corresponding HTTP message, if the AF is authorized by the NEF to provision the parameters, the NEF shall interact with the UDM to create a subscription at the UDM by using Nudm_ParameterProvision service as defined in 3GPP TS 29.503 [17].

After receiving a successful response from the UDM, the NEF shall,

-	for the HTTP POST request, create a resource "Individual ACS Configuration Subscription" which represents the ACS configuration parameter provisioning request, addressed by a URI that contains the AF Identifier and an NEF-created configuration identifier, and shall respond to the AF with a 201 Created status code, including a Location header field containing the URI for the created resource. The AF shall use the URI received in the Location header in subsequent requests to the NEF to refer to this ACS Configuration Subscription.

-	for the HTTP PUT/PATCH request, update/modify the concerned "Individual ACS Configuration Subscription" resource which represents the ACS configuration, and shall responds to the AF with an HTTP "200 OK" or an HTTP "204 No Content" status code.

-	for the HTTP DELETE request, remove all properties of the resource and delete the corresponding active resource "Individual ACS Configuration Subscription", then shall responds to the AF with a 204 No Content status code.

### 4.4.22	Procedures for Mobile Originated Location Request

#### 4.4.22.1	General

The procedure is used by NEF to transfer the updated UE location information to AF. The following procedure support:

-	Notify the AF of the updated UE location information as described in clause 6.2 of 3GPP TS 23.273 [36];

#### 4.4.22.2	Location Update Notification triggered by UE

In order to notify the AF of the updated UE location information received from GMLC, the NEF shall initiate an HTTP POST request to the AF. The body of the HTTP POST message shall include the location information related to UE MO-LR within the LocUpdateData data structure.

Upon receipt of the corresponding HTTP POST message, if the AF cannot handle the location estimate of the UE, e.g. the UE does not register to the AF, the AF shall respond to the NEF with an error code. Otherwise, the AF shall handle the location estimate according to the Service Identity if provided, and send a HTTP response including "200 OK" status code with LocUpdateDataReply data structure.

### 4.4.23	Procedures for AKMA

#### 4.4.23.1	General

The procedures support:

-	request AKMA application key by the AF to the AAnF via the NEF as described in clause 6.3 of 3GPP TS 33.535 [37];

#### 4.4.23.2	AKMA Application Key Request

In order to retrieve the AKMA application key, the AF shall send an HTTP POST request message to the resource URI "{apiRoot}/3gpp-akma/v1/retrieve". The HTTP POST request shall include the AkmaAfKeyRequest data structure that shall contain the identification of AF and an A-KID, and if the "RoamingRestiction" feature is supported, may contain a notification URI for the AF to receive notifications on the AKMA service disablement information.

Upon receipt of the corresponding HTTP POST message from the AF, if the AF's request is authorized by the NEF, then the NEF shall interact with the AAnF to retrieve the AKMA application key by using Naanf_AKMA service as defined in 3GPP TS 29.535 [38]. After receiving a successful response from the AAnF, the NEF shall respond to the AF with an HTTP "200 OK" status code, including a KAF and the expiration time of the KAF, and if "anonInd" attribute contained in AkmaAfKeyRequest data type is not set to "true" in the incoming request, optionally the GPSI (external ID) which may be translated from the SUPI received from the AAnF. The SUPI shall not be included in the response to the external AF. If the NEF receives an error response from the AAnF, the NEF shall respond to the AF with a proper error status code.

If the NEF receives a response from the AAnF with an HTTP "403 Forbidden" status code and the response message body including a ProblemDetails data structure with the "cause" attribute set to the "K_AKMA_NOT_PRESENT" or "ROAMING_AKMA_SERVICE_DENIED" application error, then the NEF shall relay this response to the AF.

#### 4.4.23.3	AKMA Service Disablement Notification

This procedure is used by the NEF to notify a previously subscribed AF of AKMA service disablement information (see also clause 6.8 of 3GPP TS 33.535 [37]).

When AKMA related sessions have already been started (before roaming was detected), and as soon as the PLMN change is detected at the AAnF, the AAnF may execute AKMA service disablement procedure via the NEF based on the roaming policy to indicate to the AF that the AKMA service is disabled.

In order to notify an AF of AKMA service disablement, the NEF shall send an HTTP POST request message to the AF with the request body including the ServiceDisableNotif data structure.

Upon reception of this notification request, the AF shall acknowledge its successful reception by sending a response message with an HTTP "204 No Content" status code.

On failure, the AF shall take proper error handling actions, as specified in clause 5.14.7, and respond to the NEF with an appropriate error status code.

### 4.4.24	Procedures for Time Synchronization Exposure

#### 4.4.24.0	General

Time synchronization exposure allows an AF to configure time synchronization in 5GS. For (g)PTP operation, the Time synchronization service allows an AF to subscribe to the UE and 5GC capabilities and availability for time synchronization service (as described in clause 4.4.24.1), to configure the (g)PTP instance in 5GS and monitor time synchronization service status as described in clause 4.4.24.2. For 5G access stratum based time distribution, the AF can influence the 5G access stratum time distribution and monitor time synchronization service status as described in clause 4.4.24.3. The time synchronization exposure is provided by NEF that uses the service provided by TSCTSF. The AF that is part of operator's trust domain may invoke the services directly with TSCTSF.

NOTE:	The AF can use either the procedure for configuring the (g)PTP instance in 5GS as described in clause 4.4.24.2 or the procedure for controlling the 5G access stratum time distribution as described in clause 4.4.24.3 for a particular UE. The procedures are not intended to be used in conjunction with each other by the AF. However, the (g)PTP instance activation, modification, and deactivation can influence the 5G access stratum time distribution for the UEs that are part of the impacted PTP instance.

#### 4.4.24.1	Subscription and unsubscription to notification of Time Synchronization Capabilites

The procedures are used by the AF to subscribe to notifications and to explicitly cancel a previous subscription to notification of capabilities of the time synchronization service for a list of UE(s), a group of UEs or any UE using a DNN/S-NSSAI combination via the NEF.

In order to subscribe to the notification of capabilities of UE and 5GC, and availability for the time synchronization service, the AF shall send an HTTP POST rmessage to the NEF to the customized operation URI "{apiRoot}/3gpp-time-sync/v1/{afId}/subscriptions". The HTTP POST request message body shall include the TimeSyncExposureSubsc data structure that shall include:

-	one of the indication of the UEs to which the time synchronization capabilities is requested via:

1)identification of a list of individual UEs within a "gpsis" attribute;

2)indication of any UE within the "anyUeInd" attribute if DNN and S-NSSAI are provisioned; or

3)identification of a group of UE(s) via a "exterGroupId" attribute.

-	subscription to event(s) notification as "subscribedEvents" attribute when the NF service consumer needs to subscribe to notifications;

-	notification URI within the "subsNotifUri" attribute; and

-	notification correlation Id within the "subsNotifId" attribute;

and may include:

-	either the DNN within the "dnn" attribute and the "snssai" attribute or the AF Service Identifier within the "afServiceId" attribute;

-	the requested event filter(s) within the "eventFilters" attribute;

-	notification methods within the "notifMethod" attribute;

-	maximum number of reports within the "maxReportNbr" attribute;

-	expiry time within the "expiry" attribute; and

-	report period within the "repPeriod" attribute.

Upon the reception of an HTTP POST request, if the AF is authorized, the NEF shall select a TSCTSF based on the local configuration or discover the TSCTSF via Nnrf_NFDiscovery service as defined in 3GPP TS 29.510 [57] for a DNN/S-NSSAI combination, if not configured. If the DNN and the S-NSSAI is omitted in the AF request, prior the TSCTSF discovery the NEF shall determine the corresponding DNN and S-NSSAI based on the received AF Service Identifier. After the NEF obtains the TSCTSF, the NEF shall invoke the Ntsctsf_TimeSynchronization_CapsSubscribe request service operation as defined in clause 5.2.2.2.2 of 3GPP TS 29.565 [50] to the selected TSCTSF. If the NEF receives an error response from the TSCTSF, the NEF shall not create the resource and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

NOTE:	It is assumed that there is only one TSCTSF set for a given DNN/S-NSSAI in this release of the specification.

After receiving a successful response from the TSCTSF, the NEF shall create an "Individual Time Synchronization Exposure Subscription" resource which represents the time synchronization exposure subscription request, addressed by a URI that contains the AF Identifier and a NEF-created configuration identifier, and shall respond to the AF with a 201 Created status code, including a Location header field containing the URI for the created resource. The AF shall use the URI received in the Location header in subsequent requests to the NEF to refer to this "Individual Time Synchronization Exposure Subscription".

In order to update an existing subscription, the AF shall send an HTTP PUT message to the NEF targeting the resource "Individual Time Synchronization Exposure Subscription". The body of the HTTP PUT request message shall include the TimeSyncExposureSubsc data type. Upon receipt of the corresponding HTTP PUT message, if the AF is authorized by the NEF, the NEF shall interact with the TSCTSF by invoking Ntsctsf_TimeSynchronization_CapsSubscribe request service operation as defined in clause 5.2.2.2.3 of 3GPP TS 29.565 [50]. After receiving a successful response from the TSCTSF, the NEF shall update a resource "Individual Time Synchronization Exposure Subscription" which represents the exposure subscription, and responds to the AF with a 200 OK with TimeSyncExposureSubsc data structure or 204 No Content status code.

If the "SupportReport" feature is supported, and the Time Synchronization Exposure Subscription creation or update is rejected by the TSCTSF because the AF is not authorized or the AF requested parameters are not allowed based on UE's Time Synchronization Subscription Data as specified in 3GPP TS 29.565 [50], the NEF shall relay the received error and include in the HTTP "403 Forbidden" error response the "cause" attribute set to the "UE_SERVICE_NOT_AUTHORIZED" application error.

When the NEF receives the notification of the capabilities of the time synchronization service from the TSCTSF as defined in clause 5.2.2.4.2 of 3GPP TS 29.565 [50], the NEF shall provide a notification to AF by sending HTTP POST message that includes the TimeSyncExposureSubsNotif data structure in the request body. Upon receipt of the notification, the AF shall respond with a "204 No Content" status code to confirm the received notification.

In order to delete an existing subscription, the AF shall send an HTTP DELETE message to the NEF targeting the resource "Individual Time Synchronization Exposure Subscription". The NEF shall interact with the TSCTSF by invoking the Ntsctsf_TimeSynchronization_CapsUnsubscribe service operation as defined in clause 5.2.2.3.2 of 3GPP TS 29.565 [50] and delete the corresponding active "Individual Time Synchronization Exposure Subscription" resource, then respond to the AF with a 204 No Content status code.

#### 4.4.24.2	Time Synchronization Exposure Configuration

The procedures are used by the AF to activate, modify or deactivate the (g)PTP instances by performing the time synchronization configuration at the NEF.

In order to configure the time synchronization parameters, the AF shall initiate an HTTP POST request to the NEF for the "Time Synchronization Exposure Configurations" resource. The body of the HTTP POST message shall include the Time Synchronization related parameters within the TimeSyncExposureConfig data structure. To subscribe to time synchronization status reports, and if the feature "NetTimeSyncStatus" is supported, the TimeSyncExposureConfig data structure shall provide the clock quality acceptance criteria.

Upon receipt of the corresponding HTTP POST message and the request is authorized by the NEF, the NEF translates any potentially received Time Synchronization Coverage Area from an external representation (e.g. geographical area) to an internal representation (e.g. TAI list) if the "CoverageArea" feature is supported, and invokes the Ntsctsf_TimeSynchronization_ConfigCreate service operation with the corresponding TSCTSF as defined in 3GPP TS 29.565 [50]. After receiving a successful response from the TSCTSF, the NEF shall create a new resource and assign an identifier for the "Individual Time Synchronization Exposure Configuration" resource. Then the NEF shall send a HTTP "201 Created" response with TimeSyncExposureConfig data structure as response body and a Location header field containing the URI of the created individual resource.

In order to update an existing Individual Time Synchronization Exposure Configuration, the AF may send an HTTP PUT message to the resource "Individual Time Synchronization Exposure Configuration" requesting the NEF to change all properties in the existing resource. The body of the HTTP PUT request message shall include TimeSyncExposureConfig data type as defined in clause 5.15.4.3.6. The user plane node Id shall remain unchanged from previous values.

Upon receipt of the corresponding HTTP PUT message and the request is authorized by the NEF, the NEF shall interact with the TSCTSF to modify an existing resource at the TSCTSF by using Ntsctsf_TimeSynchronization_ConfigUpdate service operation as defined in 3GPP TS 29.565 [50]. If the modification request is accepted by the TSCTSF and the TSCTSF informs the NEF with a successful response, the NEF shall update the existing resource for the "Individual Time Synchronization Exposure Configuration" resource. Then the NEF shall send a HTTP response including "200 OK" status code with TimeSyncExposureConfig data structure or "204 No Content" status code.

If the feature "SupportReport" feature is supported, and the time synchronization service configuration creation or update is rejected by the TSCTSF because either the AF request is not authorized or the AF requested parameter(s) are not allowed based on UE's Time Synchronization Subscription Data as specified in 3GPP TS 29.565 [50], the NEF shall relay the received error and include in the HTTP "403 Forbidden" error response the "cause" attribute set to the "UE_SERVICE_NOT_AUTHORIZED" application error.

When the NEF receives from the TSCTSF in the Ntsctsf_TimeSynchronization_ConfigUpdateNotify service operation defined in 3GPP TS 29.565 [50] the notification with the current state of time synchronization service configuration and/or, in case the feature "NetTimeSyncStatus" is supported, with the acceptable or not acceptable status of the time synchronization service, if applicable, the NEF shall provide a notification to the AF by sending HTTP POST message that include the TimeSyncExposureConfigNotif data structure in the request body. If the "CoverageArea" feature is supported and the NEF received the AF requested time synchronization coverage area as part of the configuration request, the NEF shall relay the report of the changes in the state of time synchronization service when the UE/DS-TT moves in and out of the authorized time synchronization coverage area as defined in 3GPP TS 29.565 [50]. Upon receipt of the notification, the AF shall respond with a "204 No Content" status code to confirm the received notification.

To delete an existing "Individual Time Synchronization Exposure Configuration", the AF shall initiate an HTTP DELETE request to the NEF for the "Individual Time Synchronization Exposure Subscription" resource.

Upon receipt of the corresponding HTTP DELETE message, if the AF is authorized, the NEF shall interact with the TSCTSF to delete an existing Individual Time Synchronization Exposure Configuration at the TSCTSF by using Ntsctsf_TimeSynchronization_ConfigDelete service operation as defined in 3GPP TS 29.565 [50]. If the request is accepted by the TSCTSF, the NEF shall delete the existing resource for the "Individual Time Synchronization Exposure Configuration" resource. Then the NEF shall send a HTTP "204 No Content" response.

#### 4.4.24.3	Management of 5G access stratum time distribution

The procedures are used by the AF to activate, update or delete the 5G access stratum time distribution for one UE or group of UE(s). The AF may also use this procedure to indicate a coverage area to provide the service, clock quality detail level to provide to the UE or group of UE(s) and to subscribe for time synchronization status reports.

When the "ASTIConfigReport" feature is supported, to receive notifications about changes in the 5G access stratum time distribution configuration, the NF service consumer shall also provide the notification URI within the "astiNotifUri" attribute and the notification correlation Id within the "astiNotifId" attribute.

When the "NetTimeSyncStatus" feature is supported, the NF service consumer may provide the clock quality reporting control information to manage the timing synchronization status reporting in 5GS within the clock quality detail level in the "clkQltDetLvl" attribute, and if applicable, the clock quality acceptance criteria in the "clkQltAcptCri" attribute. When the NF service consumer includes the clock quality acceptance criteria, the NF service consumer is also indicating the subscription to notification of the status of the access stratum time distribution service.

In order to configure the 5G access stratum time distribution parameters, the AF shall initiate an HTTP POST request to the NEF for the "ASTI Configurations" resource. The body of the HTTP POST message shall include the 5G access stratum time distribution parameters, and may indicate, when the feature "NetTimeSyncStatus" is supported, the clock quality detail level and, if applicable, the clock quality acceptance criteria, and when the feature "CoverageArea" is supported the time synchronization service coverage area, within the AccessTimeDistributionData data structure as defined in clause 5.22.4.3.2.

Upon receipt of the corresponding HTTP POST message and the request is authorized by the NEF, the NEF shall select a TSCTSF based on the local configuration or discover the TSCTSF via Nnrf_NFDiscovery service as defined in 3GPP TS 29.510 [57] for the GPSI or external group identifier, if not configured. After the NEF obtains the TSCTSF, the NEF translates any potentially received Time Synchronization Coverage Area from an external representation (e.g. geographical area) to an internal representation (e.g. TAI list) if the "CoverageArea" feature is supported, and invokes the Ntsctsf_ASTI_Create service operation with the corresponding TSCTSF, if available, as defined in 3GPP TS 29.565 [50]. After receiving a successful response from the TSCTSF, the NEF shall create a new resource and assign an identifier for the "Individual ASTI Configuration" resource. Then the NEF shall send a HTTP "201 Created" response with AccessTimeDistributionData data structure as response body and a Location header field containing the URI of the created individual resource. If the NEF receives an error response from the TSCTSF, the NEF shall not create the resource and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

In order to update an existing Individual ASTI Configuration, the AF may send an HTTP PUT message to the resource "Individual ASTI Configuration" requesting the NEF to change all properties in the existing resource. The body of the HTTP PUT request message shall include the AccessTimeDistributionData data type.

Upon receipt of the corresponding HTTP PUT message and the request is authorized by the NEF, the NEF shall interact with the TSCTSF to modify an existing resource at the TSCTSF by using Ntsctsf_ASTI_Update service operation as defined in 3GPP TS 29.565 [50]. If the modification request is accepted by the TSCTSF and the TSCTSF informs the NEF with a successful response, the NEF shall update the existing resource for the "Individual ASTI Configuration" resource. Then the NEF shall send a HTTP response including "200 OK" status response with AccessTimeDistributionData data structure or "204 No Content" status code. If the NEF receives an error code from the TSCTSF, the NEF shall not update the resource and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

If the "SupportReport" feature is supported, and the ASTI time synchronization service creation or update is rejected by the TSCTSF because the AF request or the AF requested parameter(s) are not authorized based on UE's Time Synchronization subscription data as specified in 3GPP TS 29.565 [50], the NEF shall relay the received error and include in the HTTP "403 Forbidden" error response the "cause" attribute set to the "UE_SERVICE_NOT_AUTHORIZED" application error.

When the "ASTIConfigReport" feature is supported and the NEF receives the notification for the 5G access stratum time distribution status change from the TSCSF by Ntsctsf_ASTI_UpdateNotify service operation defined in 3GPP TS 29.565 [50], the NEF shall provide a notification to AF by sending HTTP POST message that include the AstiConfigNotification data structure in the request body. If the "CoverageArea" feature is supported and the NEF received the AF requested time synchronization coverage area, the NEF shall relay the report of the enable/disable state of ASTI time synchronization service when the UE moves in and out of the authorized time synchronization coverage area as defined in 3GPP TS 29.565 [50]. If the "NetTimeSyncStatus" feature is supported and the NEF received the AF requested clock quality acceptance criteria, the NEF shall relay the acceptable/not acceptable status of the ASTI time synchronization service to the NF service consumer. Upon receipt of the notification, the AF shall respond with a "204 No Content" status code to confirm the received notification.

To delete an existing Individual ASTI Configuration, the AF shall initiate an HTTP DELETE request to the NEF for the "Individual ASTI Configuration" resource.

Upon receipt of the corresponding HTTP DELETE message, if the AF is authorized, the NEF shall interact with the TSCTSF to delete an existing Individual ASTI Configuration at the TSCTSF by using Ntsctsf_ASTI_Delete service operation as defined in 3GPP TS 29.565 [50]. If the request is accepted by the TSCTSF, the NEF shall delete the existing resource for the "Individual ASTI Configuration" resource. Then the NEF shall send a HTTP "204 No Content" response. If the NEF receives an error response from the TSCTSF, the NEF shall not delete the resource and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

AF may request and query the status of the access stratum time distribution sending the HTTP POST request, "retrieve" custom operation, to the resource "ASTI Configurations". The body of the HTTP POST request message shall include the StatusRequestData data type as defined in clause 5.22.4.3.3.

Upon receipt of the corresponding HTTP POST message, if the AF is authorized, the NEF shall interact with the TSCTSF by using Ntsctsf_ASTI_Get service operation as defined in 3GPP TS 29.565 [50]. Upon receipt of response from the TSCTSF, the NEF shall send a HTTP "200 OK" response with the StatusResponseData data structure as defined in clause 5.22.4.3.4 in the payload. If the NEF receives an error response from the TSCTSF, the NEF shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

### 4.4.25	Procedures for ECS address Provisioning

The procedures are used by the AF to provision ECS address(es) to the NEF. The procedures are applicable for an individual UE, any UE or a group of UEs.

In order to create an Individual ECS Address Provision Configuration resource, the AF shall initiate an HTTP POST request to the NEF for the "ECS Address Provision Configurations" resource. The body of the HTTP POST message shall include within the EcsAddressProvision data structure the ECS address(es) via the "ecsServerAddr" attribute, may include the spatial validity condition via the "spatialValidityCond" attribute, the target UE information via the "tgtUe" attirbute, if the "HR-SBO" feature is supported, the PLMN ID in which the provided information applies via the "plmnId" attribute, if the "ECSAuthMethods" feature is supported, may include the supported authentication methods via the "ecsAuthMethods" attribute and, if the "ECSSuppPlmns" feature is supported, may include the list of PLMNs and associated ECSPs for which EDN configuration information can be provided by the ECS, via the "supportedPlmns" attribute.

NOTE:	The NEF can derive DNN and S-NSSAI information from the AF identifier.

Upon receipt of the corresponding HTTP POST message, if the AF is authorized by the NEF to provision the ECS address(es), the NEF shall interact with the UDM to create a resource at the UDM by using Nudm_ParameterProvision service as defined in 3GPP TS 29.503 [17]. If the request is accepted by the UDM and the UDM informs the NEF with a successful response, the NEF shall create a new resource and assign an identifier for the "Individual ECS Address Provision Configuration" resource. Then the NEF shall send a HTTP "201 Created" response with EcsAddressProvision data structure as response body and a Location header field containing the URI of the created individual resource.

In order to update an existing Individual ECS Address Provision Configuration, the AF shall send an HTTP PUT message to the resource "Individual ECS Address Provision Configuration" requesting the NEF to change all properties in the existing resource. The body of the HTTP PUT request message shall include the EcsAddressProvision data type. Upon receipt of the corresponding HTTP PUT message, if the AF is authorized by the NEF to provision the ECS address(es), the NEF shall interact with the UDM to modify an existing resource at the UDM by using Nudm_ParameterProvision service as defined in 3GPP TS 29.503 [17]. If the modification request is accepted by the UDM and the UDM informs the NEF with a successful response, the NEF shall update the existing resource for the "Individual ECS Address Provision Configuration" resource. Then the NEF shall send a HTTP response including "200 OK" status code with EcsAddressProvision data structure or "204 No Content" status code.

To delete an existing Individual ECS Address Provision Configuration, the AF shall initiate an HTTP DELETE request to the NEF for the "Individual ECS Address Provision Configuration" resource. Upon receipt of the corresponding HTTP DELETE message, if the AF is authorized, the NEF shall interact with the UDM to delete the existing resource at the UDM by using Nudm_ParameterProvision service as defined in 3GPP TS 29.503 [17]. If the request is accepted by the UDM, the NEF shall delete the existing resource for the "Individual ECS Address Provision Configuration" resource. Then the NEF shall send a HTTP "204 No Content" response.

### 4.4.26	Procedures for AM Policy Authorization

#### 4.4.26.1	General

The procedures are used by AF to send request to NEF for AM Policy Authorization, and for NEF to authorize an AF triggered AM Policy Authorization request and trigger a respective Npcf_AMPolicyAuthorization request. This service also allows the AF to subscribe/unsubscribe the notification of event(s) for the existing AF application AM context.

The following procedures support:

-	Create/Modify/Delete of AF triggered application AM context; and

-	Subscribe/Unsubscribe/Notify event(s) for the existing AF application AM context.

#### 4.4.26.2	Creation of a new Individual Application AM Context

In order to create a new Individual application AM context resource for a given AF, the AF shall initiate an HTTP POST request to the NEF for the "Application AM Contexts" resource. The HTTP POST request message body shall include the AppAmContextExpData data structure that shall include:

-	identification of an individual UE via a "gpsi" attribute;

and may include:

-	subscription to AM policy event(s) notification as "evSubscs" attribute. For each subscribed event, the AF may include the description of the event reporting mode, as e.g. whether immediate reporting is required;

-	a high throughput requirement Indication as "highThruInd" attribute;

-	service coverage requirements as "covReqs" attribute; and

-	policy duration requirement as "policyDuration" attribute.

Upon receipt of the corresponding HTTP POST message, if the AF is authorized by the NEF to request the AM policy authorization, the NEF may interact with the BSF to retrieve the related PCF information by invoking the Nbsf_Management_Discovery service operation as described in 3GPP TS 29.521 [9]. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable. After receiving a successful response from the BSF, the NEF shall trigger a respective Npcf_AMPolicyAuthorization_Create request as defined in 3GPP TS 29.534 [43]. If the request is accepted by the PCF and the PCF informs the NEF with a successful response, the NEF shall create a new "Individual application AM Context" and assign an application AM context identifier for the "Individual application AM Context" resource.

Then the NEF shall send a HTTP "201 Created" response with:

-	AppAmContextExpRespData data structure as response body, including the created "Individual application AM Context" resource and, if immediate reporting was requested for the subscribed event(s), the currently available value(s), if received from the PCF; and

-	a Location header field containing the URI of the created "Individual application AM Context" resource to the AF.

If the NEF receives an error responsefrom the PCF, the NEF shall not create the resource and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

#### 4.4.26.3	Modification of an existing individual Application AM Context

In order to modify an existing individual Application AM Context resource, the AF shall initiate an HTTP PATCH request to the NEF for the "Individual application AM Context" resource. The body of the HTTP PATCH message shall include the AppAmContextExpUpdateData data type as defined in clause 5.17.1.3.3.3.

Upon receipt of the corresponding HTTP PATCH message, if the AF is authorized by the NEF to modify the AM policy authorization request, the NEF shall interact with the PCF to modify an existing application AM context by using Npcf_AMPolicyAuthorization_Update request as defined in 3GPP TS 29.534 [43]. If the modification request is accepted by the PCF and the PCF informs the NEF with a successful response, the NEF shall update the existing application AM context for the "Individual application AM Context" resource. Then the NEF shall send a HTTP response including "200 OK" status code with AppAmContextExpRespData data structure (including the updated resource representation and, if immediate reporting was requested for the new subscribed event(s), the currently available value(s), if received from the PCF) or "204 No Content" status code to the AF.

If the NEF receives an error responsefrom the PCF, the NEF shall not modify the resource and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

#### 4.4.26.4	Deletion of an existing individual Application AM Context

To delete an existing application AM context, the AF shall initiate an HTTP DELETE request to the NEF for the "Individual application AM Context" resource.

Upon receipt of the corresponding HTTP DELETE message, if the AF is authorized to delete the application AM context, the NEF shall interact with the PCF to delete an existing application AM context at the PCF by using Npcf_AMPolicyAuthorization_Delete request as defined in 3GPP TS 29.534 [43]. If the request is accepted by the PCF and informs the NEF with a successful response, the NEF shall delete the existing application AM context for the "Individual application AM Context" resource. Then the NEF shall send a HTTP "204 No Content" response to the AF.

If the NEF receives an error responsefrom the PCF, the NEF shall take proper error handling action and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

#### 4.4.26.5	Create or modify subscription to notification of AM policy event

In order to create or modify the subscription to notification of AM policy event(s) for the application AM context, the AF shall send an HTTP PUT message to the NEF to the sub-resource "AM Policy Events Subscription", the HTTP PUT message shall include the AmEventsSubscData data structure as request body.

Upon receipt of the HTTP request from the AF, if the AF is authorized, the NEF shall interact with the PCF to subscribe to, or modify the subscription to the AM policy event notification by using Npcf_AMPolicyAuthorization_Subscribe request as defined in 3GPP TS 29.534 [43]. If the request is accepted by the PCF and the PCF informs the NEF with a successful response, the NEF shall create a new AM policy event subscription sub-resource in an existing application AM context or modify an existing AM policy event subscription to the "AM Policy Events Subscription" sub-resource. Then the NEF shall send:

-	for a subscription creation request, an HTTP "201 Created" response with:

a.	AmEventsSubscRespData data structure as response body, including the created "AM Policy Events Subscription" resource and, if immediate reporting was requested for the subscribed event(s), the currently available value(s), if received from the PCF; and

b.	a Location header field containing the URI of the created individual subscription resource to the AF; or

-	for a subscription update request, an HTTP "200 OK" response code with AmEventsSubscRespData data structure with the updated "AM Policy Events Subscription" resource or HTTP "204 No Content" response code and, if immediate reporting was requested for the subscribed event(s), the currently available value(s), if received from the PCF;

as response body to the AF.

If the NEF receives an error responsefrom the PCF, the NEF shall not create or modify the sub-resource and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

#### 4.4.26.6	Unsubscription to notification of AM policy event

In order to delete existing subscribed AM policy event(s) within the existing Individual application AM context, the AF shall initiate the HTTP DELETE request message to the NEF to the "AM Policy Events Subscription" sub-resource.

Upon receipt of the corresponding HTTP DELETE message, if the AF is authorized to delete the notification of AM policy event(s), the NEF shall interact with the PCF to delete an existing subscription of notification to AM policy event(s) within the existing application AM context at the PCF by using Npcf_AMPolicyAuthorization_Unsubscribe request as defined in 3GPP TS 29.534 [43]. If the request is accepted by the PCF and informs the NEF with a successful response, the NEF shall delete the existing subscription to notification of AM policy event(s) within the existing application AM context for the "AM Policy Events Subscription" resource. Then the NEF shall send a HTTP "204 No Content" response to the AF.

If the NEF receives an error responsefrom the PCF, the NEF shall take proper error handling action and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

#### 4.4.26.7	Notification of AM policy event

If the NEF receives an AM policy event notification from the PCF indicating that the subscribed AM policy event has been detected, the NEF shall provide a notification to AF by sending HTTP POST message that include the AmEventsNotification data structure in the request body. Upon receipt of the AM policy event notification, the AF shall respond with a "204 No Content" status code to confirm the received notification to the NEF.

### 4.4.27	Procedures for AF triggered Access and Mobility Influence

#### 4.4.27.1	General

The procedures are used by the AF to provision the Access and Mobility(AM) policy related request via NEF to one or multiple UEs that may have already registered or not. This service also allows the NEF to send the notification of service area coverage outcome events to the AF.

#### 4.4.27.2	Create the AM Influence Subscription

In order to create a resource for the AM Influence, the AF shall send an HTTP POST request message to the NEF for the "AM Influence Subscription" resource. The request message may include the AF Transaction Identifier, GPSI, DNN, S-NSSAI, External Group Identifier, list of External Application Identifier(s), and for LBO roaming scenarios, a list of PLMN ID(s) for inbound roaming UEs if the "DCAMP_Roaming_LBO" feature is supported, AF Service Identifier, throughput requirements, service area coverage requirements represented by list of geographical areas, policy duration, subscribed event(s) and the notification destination address.

The request may target one or multiple UEs that may have already registered or not. For an individual UE identified by GPSI, or a group of UEs identified by External Group Identifier, the NEF shall interact with the UDM by invoking the Nudm_SubscriberDataManagement service as described in 3GPP TS 29.503 [17] to retrieve the SUPI or Internal Group Identifier. For all UEs, the NEF will not interact with the UDM.

The NEF shall interact with the UDR by invoking the Nudr_DataRepository service as described in 3GPP TS 29.504 [20] to store the policy data in the UDR.

If the NEF receives an error response from the UDR, the NEF shall not create the resource and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

After receiving a successful response from the UDR, the NEF shall create a resource "Individual AM Influence Subscription", which represents the AM influence subscription, addressed by a URI that contains the AF Identifier and an NEF-created subscription identifier. The NEF shall respond to the AF with a "201 Created" status code, including a Location header field containing the URI for the created resource. The AF shall use the URI received in the Location header when it subsequently sends requests to the NEF to reference this AM influence subscription.

#### 4.4.27.3	Modifiy the AM Influence Subscription

In order to update an existing AM influence subscription, the AF shall send an HTTP PUT or HTTP PATCH request message to the NEF for the "Individual AM Influence Subscription" resource. The NEF shall interact with the UDR by invoking the Nudr_DataRepository service as described in 3GPP TS 29.504 [20] to update the policy data in the UDR.

If the NEF receives an error responsefrom the UDR, the NEF shall not update the resource and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

After receiving a successful response from the UDR, the NEF shall update the "Individual AM Influence Subscription" resource which represents the AM influence subscription, and shall respond to the AF with an HTTP "200 OK" or "204 No Content" response message.

#### 4.4.27.4	Delete the AM Influence Subscription

In order to delete an existing AM influence subscription, the AF shall send an HTTP DELETE request message to the NEF for the "Individual AM Influence Subscription" resource. The NEF shall interact with the UDR by invoking the Nudr_DataRepository service as described in 3GPP TS 29.504 [20] to delete the policy data in the UDR. If the NEF receives an error responsefrom the UDR, the NEF shall take proper error handling actions and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

After receiving a successful response from the UDR, the NEF shall delete the "Individual AM Influence Subscription" resource which represents the AM influence subscription, and shall respond to the AF with an HTTP "204 No Content" response message.

#### 4.4.27.5	Notification of service area coverage outcome events

When the NEF receives the notification of service area coverage outcome events from the PCF as defined in 3GPP TS 29.534 [43], the NEF shall provide a notification by sending an HTTP POST message to the AF. The HTTP POST message shall include the subscribed event (service area coverage outcome event) to the AF identified by the notification destination received during the creation/modification of the AM Influence resource.

Upon receipt of the event notification, the AF shall respond with a "204 No Content" status code to confirm the received event notification.

### 4.4.28	Procedures for Northbound EAS Deployment Information management

#### 4.4.28.1	General

The procedures are used by AF to provide, update or delete EAS Deployment Information to NEF, and for NEF to authorize the AF provisioned EAS Deployment Information to be stored in the UDR.

The following procedures support:

-	Create/Update/Delete the AF provisioned EAS Deployment information;

#### 4.4.28.2	Creation of a new Individual EAS Deployment information resource

In order to create a new Individual EAS Deployment information resource for a given AF, the AF shall initiate an HTTP POST request to the NEF for the "EAS Deployment Information" resource. The HTTP POST request message body shall include the EasDeployInfo data structure that shall include:

-	FQDN(s) of an application deployed in the Local part of the DN via an "fqdnPatternList" attribute;

and may include:

-	an AF service identifier as the "afServiceId" attribute;

-	an DNN as "dnn" attribute;

-	an S-NSSAI as "snssai" attribute;

-	an external Group Identifier as "exterGroupId" attribute;

-	identification of an application as "appId" attribute;

-	list of DNS server identifier and/or IP address(s) of the EAS in the local DN for each DNAI as "dnaiInfos" attribute; and

-	the identifier of the AF that is responsible for the EAS associated with this EAS deployment information as "targetAfId" attribute, if the "EasRelocationEnh" feature is supported.

NOTE 1:	The AF responsible for the EAS (indicated by the "targetAfId" attribute) can be different from the AF that creates the EAS Deployment information (indicated by the "afId" attribute in the URI of the resource).

NOTE 2:	When the "targetAfId" attribute is provided, then all DNAI(s) correspond to the same EHE provider. The "targetAfId" attribute can be used in case of AF(s) involving different EHE providers, and the source EHE is unaware of other/target EHE specific deployment details.

Upon receipt of the corresponding HTTP POST message, if the AF is authorized by the NEF to provide the EAS Deployment Information, the NEF shall interact with the UDM by using Nudm_SubscriberDataManagement service as defined in 3GPP TS 29.503 [17] to translate the external group identifier into the corresponding internal group identifier and the NEF may derive DNN and S-NSSAI from the AF Service Identifier if not received explicitly. Then the NEF shall interact with the UDR to create the associated EAS Deployment information by using the Nudr_DataRepository service as defined in 3GPP TS 29.504 [20]. If the "EasDnaiConsistency" feature is supported and there are existing EAS-DNAI mappings configured via OAM in the UDR (see 3GPP TS 29.519 [23] clause 6.2.23), the NEF also ensures that the EAS Deployment Information received from the AF is not in conflict with the OAM-configured information. In case of conflict the NEF shall reject the request message by sending an HTTP response to the AF with a status code set to 403 Forbidden and may include the "CONFLICT_CONFIG_DATA" error in the "cause" attribute of the "ProblemDetails" structure. If the request is accepted by the UDR and the UDR informs the NEF with a successful response, the NEF shall create a new "Individual EAS Deployment Information" resource. Then the NEF shall send a HTTP "201 Created" response with the EasDeployInfo data structure including the contents of the created EAS Deployment Information resource in theresponse body and a Location header field containing the URI of the created individual EAS Deployment Information resource. If the NEF receives an error responsefrom the UDR, the NEF shall not create the resource and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

#### 4.4.28.3	Modification of an existing individual EAS Deployment Information resource

In order to modify an existing individual EAS Deployment Information resource, the AF shall initiate an HTTP PUT request to the "Individual EAS Deployment Information" resource. The request body shall include the EasDeployInfo data structure. The "afServiceId" value shall remain unchanged from the previous value, if available in the HTTP PUT message.

Upon receipt of the corresponding HTTP PUT request message, if the AF is authorized by the NEF to modify the existing individual EAS Deployment Information resource, the NEF shall interact with the UDR by invoking the Nudr_DataRepository service as described in 3GPP TS 29.504 [20] to modify the EAS Deployment Information in the UDR. If the "EasDnaiConsistency" feature is supported and there are existing EAS-DNAI mappings configured via OAM in the UDR (see 3GPP TS 29.519 [23] clause 6.2.23), the NEF also ensures that the EAS Deployment Information received from the AF is not in conflict with the OAM-configured information. In case of conflict the NEF shall reject the request message by sending an HTTP response to the AF with a status code set to 403 Forbidden and may include the "CONFLICT_CONFIG_DATA" error in the "cause" attribute of the "ProblemDetails" structure.

If the modification request is accepted by the UDR and the UDR informs the NEF with a successful response, the NEF shall update the existing individual EAS Deployment Information resource. Then the NEF shall send a HTTP response including "200 OK" status code with EasDeployInfo data structure or "204 No Content" status code.

If the NEF receives an error responsefrom the UDR, the NEF shall not update the "Individual EAS Deployment Information" resource and shall respond a proper error status code to the AF. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

#### 4.4.28.4	Deletion of an existing individual EAS Deployment Information resource

In order to delete an existing EAS Deployment Information, the AF shall send an HTTP DELETE request message to the NEF for the "Individual EAS Deployment Information" resource. The NEF shall interact with the UDR by invoking the Nudr_DataRepository service as described in 3GPP TS 29.504 [20] to delete the EAS Deployment Information in the application data in the UDR.

After receiving a successful response from the UDR, the NEF shall delete the "Individual EAS Deployment Information" resource and shall respond to the AF with an HTTP "204 No Content" response message.

If the NEF receives an error responsefrom the UDR, the NEF shall take proper error handling actions and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

#### 4.4.28.5	Deletion of EAS Deplyoment Information based on given criteria

In order to delete existing EAS Deployment Information resource(s) which match given attributes, the NF service consumer shall send an HTTP POST request with "{apiRoot}/3gpp-eas-deployment/<apiVersion>/remove-edis" as URI. The POST request body shall contain an EdiDeleteCriteria data structure. The EdiDeleteCriteria data structure provided in the request body shall include at least one of the following:

-	an AF identifier within the "afId" attribute;

-	DNN and slice information within the "dnnSnssai" attribute;

Upon the reception of this HTTP POST request, if the NF service consumer is authorized by the NEF to delete the EAS Deployment Information, the NEF shall determine the EAS Deployment Information resources that match the provided criteria and interact with the UDR to delete the associated EAS Deployment Information by using the Nudr_DataRepository service as defined in 3GPP TS 29.504 [20]. If the request is accepted by the UDR and the UDR informs the NEF with a successful response, the NEF shall send a HTTP "204 No Content" response. If the NEF receives an error code from the UDR, the NEF shall respond to the AF with a proper error status code.

### 4.4.29	Procedures for MBS Management

#### 4.4.29.1	General

The procedures described in the clauses below are used by an AF to interact with the 5GC for MBS management as defined in 3GPP TS 23.247 [53] and 3GPP TS 26.502 [65], in order to carry out the following procedures:

-	MBS TMGI management procedures.

-	MBS Session management procedures.

-	MBS User Service management procedures.

-	MBS User Data Ingest Session management procedures.

#### 4.4.29.2	Procedures for MBS TMGI management

##### 4.4.29.2.1	General

The procedures described in the clauses below are used by an AF to request and manage TMGI(s) for MBS session(s) as defined in clause 7.1 of 3GPP TS 23.247 [53].

##### 4.4.29.2.2	Procedure for MBS TMGI(s) allocation or MBS TMGI(s) expiry time refresh

This procedure is used by an AF to request the allocation of TMGI(s) for new MBS session(s) or the refresh of the expiry time of already allocated MBS TMGI(s).

In order to request the allocation of TMGI(s) for new MBS session(s) or the refresh of the expiry time of already allocated MBS TMGI(s), an AF shall send a Nnef_MBSTMGI_Allocation request message to the NEF using the HTTP POST method with the request body including the TmgiAllocRequest data structure that shall contain:

NOTE:	The Nnef_MBSTMGI_Allocation service operation corresponds to the stage 2 Nnef_MBSTMGI_Allocate service operation defined in 3GPP TS 23.247 [53].

-	within the "afId" attribute, the identifier of the AF that is sending the request;

-	within the "tmgiParams" attribute, the parameters (e.g. number of TMGI(s) to be allocated, etc.) to request the allocation of TMGI(s) for new MBS session(s) or the refresh of the expiry time of already allocated TMGI(s);

-	within the "suppFeat" attribute, the features supported by the AF, if feature negotiation needs to take place;

and may contain:

-	within the "notificationUri" attribute, the notification URI via which the AF desires to receive notifications on timer expiry for TMGI(s);

-	within the "requestTestNotification" attribute, an indication on whether the NEF should send a test notification, if the "Notification_test_event" feature is supported;

-	within the "websockNotifConfig" attribute, the configuration parameters to set up notification delivery over Websocket protocol, if the "Notification_websocket" feature is supported; and/or

-	within the either "mbsServiceArea" attribute or the "extMbsServiceArea" attribute, the MBS service area for the TMGI(s) to be allocated, which may be needed for a local MBS service.

The NEF shall then check whether the AF is authorized to perform this operation or not as defined in clause 6.1.1 of 3GPP TS 23.247 [53]. If the AF is authorized, then:

-	if the MBS Service Area information is provided via the "extMbsServiceArea" attribute, the NEF shall translate the received geographical area(s) or civic address(es) to a list of TAI(s) and/or cell ID(s);

-	the NEF shall determine the target MB-SMF either by querying the NRF to discover and select an MB-SMF (service) instance that can handle this request, or based on local configuration; and

-	if the received MBS Service Area information cannot be covered by the MB-SMF Service Area of a single MB-SMF, the NEF shall reject the request and respond to the AF with an HTTP "403 Forbidden" status code with the response body including the ProblemDetailsTmgiAlloc data structure containing:

-	the ProblemDetails data structure containing the "cause" attribute set to the "MBS_SERVICE_AREA_TOO_LARGE" application error; and optionally

-	the ReducedMbsServArea data structure containing the reduced MBS Service Area information;

-	the NEF shall convey this MBS TMGI(s) allocation request or expiry time refresh request to the selected MB-SMF using the Nmbsmf_TMGI service API as defined in 3GPP TS 29.532 [52]; and

-	if the received MBS Service Area is not supported (e.g., the received MBS Service Area cannot be covered by the service area(s) of any MB-SMF), the NEF shall skip the following steps below and respond to the AF with an HTTP "403 Forbidden" status code with the response body including the "MBS_SERVICE_AREA_NOT_SUPPORTED" application error.

Upon reception of a successful response from the MB-SMF as defined in 3GPP TS 29.532 [52], the NEF shall forward the received information (e.g. allocated MBS TMGI(s), expiry time or updated expiry time of existing MBS TMGI(s), etc.) to the AF in a Nnef_MBSTMGI_Allocation response message with an HTTP "200 OK" status code and the response body including the TmgiAllocResponse data structure that shall contain:

-	within the "tmgiInfo" attribute, the MBS TMGI(s) allocation information or the refreshed expiry time for already allocated MBS TMGI(s); and

-	within the "suppFeat" attribute, the features supported by both the AF and the NEF, if feature negotiation needs to take place and the AF provided the list of its supported features in the corresponding request body.

On failure or if the NEF receives an error response from the MB-SMF, the NEF shall take proper error handling actions, as specified in clause 5.19.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

##### 4.4.29.2.3	Procedure for MBS TMGI(s) deallocation

This procedure is used by an AF to request the deallocation of previously allocated MBS TMGI(s).

In order to request the deallocation of previously allocated MBS TMGI(s), an AF shall send a Nnef_MBSTMGI_Deallocation request message to the NEF using the HTTP POST method with the request body including the TmgiDeallocRequest data structure that shall contain :

NOTE:	The Nnef_MBSTMGI_Deallocation service operation corresponds to the stage 2 Nnef_MBSTMGI_Deallocate service operation defined in 3GPP TS 23.247 [53].

-	within the "afId" attribute, the identifier of the AF that is sending the request; and

-	within the "tmgis" attribute, the list of MBS TMGI(s) for which deallocation is requested.

The NEF shall then check whether the AF is authorized to perform this operation or not as defined in clause 6.1.1 of 3GPP TS 23.247 [53]. If the AF is authorized, the NEF shall convey this MBS TMGI(s) deallocation request to the MB-SMF using the Nmbsmf_TMGI service API as defined in 3GPP TS 29.532 [52].

Upon reception of a successful response from the MB-SMF confirming the deallocation of the TMGI(s), the NEF shall forward this confirmation to the AF in a Nnef_MBSTMGI_Deallocation response message with an HTTP "204 No Content" status code.

On failure or if the NEF receives an error responsefrom the MB-SMF, the NEF shall take proper error handling actions, as specified in clause 5.19.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

##### 4.4.29.2.4	Procedure for MBS TMGI(s) timer expiry notification

This procedure is used by the NEF to notify an already subscribed AF of timer expiry for previously allocated MBS TMGI(s).

In order to notify an AF of timer expiry for previously allocated MBS TMGI(s), the NEF shall send a Nnef_MBSTMGI_ExpiryNotify request message to the AF using the HTTP POST method with the request body including the ExpiryNotif data structure that shall contain:

-	within the "tmgis" attribute, the list of MBS TMGI(s) for which the timer has expired.

Upon reception of this notification request, the AF shall acknowledge its successful reception by sending a Nnef_MBSTMGI_ExpiryNotify response message with an HTTP "204 No Content" status code.

On failure, the AF shall take proper error handling actions, as specified in clause 5.19.7, and respond to the NEF with an appropriate error status code.

#### 4.4.29.3	Procedures for MBS session management

##### 4.4.29.3.1	General

The procedures described in the clauses below are used by an AF to create, update or delete MBS session(s) and to subscribe to / unsubscribe from MBS Session Status event(s) reporting at the NEF.

This service is applicable for both broadcast and multicast sessions or, for a location dependent MBS session, the part of an MBS Session within an MBS service area, as defined in 3GPP TS 23.247 [53].

##### 4.4.29.3.2	Procedure for MBS session creation

This procedure is used by an AF to request the creation of a multicast or a broadcast MBS session or, for a location dependent MBS session, the part of an MBS Session within an MBS service area.

In order to request the creation of an MBS Session, an AF shall send a Nnef_MBSSession_Create request to the NEF using the HTTP POST method and targeting the "MBS Sessions" collection resource with the request message body including the MbsSessionCreateReq data structure that shall contain:

-	within the "afId" attribute, the identifier of the AF that is sending the request; and

-	within the "mbsSession" attribute, the characteristics of the MBS session that is to be created.

The "mbsSession" attribute shall be encoded using the MbsSession data structure that shall contain:

-	within the "mbsSessionId" attribute, the identifier of the MBS Session (e.g. SSM, TMGI), if available;

-	within the "tmgiAllocReq" attribute, the TMGI allocation request indication, if the "mbsSessionId" attribute is either absent or does not contain a TMGI; and

-	within the "serviceType" attribute, the MBS service type (i.e. multicast or broadcast);

-	within the "locationDependent" attribute, the location dependent MBS session indication, if the request is related to a location dependent MBS;

and may further contain:

-	for a multicast or a broadcast MBS session:

-	within the "ingressAddrReq" attribute, the ingress transport address request indication to indicate whether the allocation of an ingress transport address is requested or not;

-	within the "extMbsServiceArea" attribute, the MBS service area, for a location dependent MBS session or a local MBS session;

-	within the "activationTime" attribute, the MBS session activation time;

-	within the "terminationTime" attribute, the MBS session termination time;

-	within the "mbsServInfo" attribute, the MBS Service Information for the MBS session; and

-	within the "mbsSessionSubsc" attribute, the parameters to request the creation of a subscription to MBS session status event(s) reporting;

-	for a multicast MBS session:

-	within the "activityStatus" attribute, the MBS session activity status (i.e. active or inactive); and

-	within the "anyUeInd" attribute, the indication of whether any UE may join the MBS session;

-	for a broadcast MBS session:

-	within the " mbsFsaIdList" attribute, the list of MBS frequency selection area Identifiers (i.e. FSA IDs); and

-	when the "5MBS2" feature is supported:

-	within the "associatedSessionId" attribute, the Associated Session ID; and

-	within the "nrRedCapUeInfo" attribute, the indication of whether the broadcast MBS session is for NR RedCap UEs only, non-RedCap UEs only or both.

At the reception of this HTTP POST request for MBS session creation:

-	the NEF may decide to interact with the PCF for MBS policy authorization of the received MBS Service Information;

-	if the NEF decides to interact with the PCF, then:

-	if the NEF did not receive an MBS Session Identifier or received a TMGI allocation request within the "tmgiAllocReq" attribute, the NEF shall request TMGI allocation to the MB-SMF using the Nmbsmf_TMGI service API, as specified in 3GPP TS 29.532 [52];

-	if the received MBS Session Creation request is for the creation of an MBS Session that is part of a location dependent MBS, i.e. the "locationDependent" attribute is present and set to "true", and there is a need to select the same PCF for all the MBS Sessions composing the location dependent MBS, the NEF shall interact with the BSF using the Nbsf_Management service API to check whether there is already a PCF serving the MBS Sessions of the location dependent MBS based on the MBS Session Identifier, as specified in 3GPP TS 29.532 [52]. Then:

NOTE 1:	Interacting with the BSF to discover whether there is already a PCF serving the MBS Session is not necessary in a deployment with a single PCF.

-	if there is a PCF already serving the MBS Sessions of the location dependent MBS, the NEF shall use this PCF for MBS policy authorization of the received MBS Service Information;

-	if there is no PCF already serving the MBS Sessions of the location dependent MBS or the NEF did not interact with the BSF, the NEF shall interact with the NRF using the Nnrf_NFDiscovery service API to discover a PCF (service) instance to serve the MBS Session possibly based on the MBS Session Identifier, as specified in 3GPP TS 29.510 [57];

-	the NEF shall then interact with the selected PCF (service) instance using the Npcf_MBSPolicyAuthorization service API for MBS policy authorization of the received MBS Service Information and the creation of a corresponding MBS Application Session Context at the PCF, as specified in 3GPP TS 29.537 [63]; and

-	if MBS session authorization is successful or when the NEF decides to not interact with the PCF for MBS policy authorization, the NEF shall interact with the MB-SMF using the Nmbsmf_MBSSession service API to request the creation of a corresponding MBS session at the MB-SMF as specified in 3GPP TS 29.532 [52];

-	if the MBS Service Area information is provided within the "extMbsServiceArea" attribute, the NEF shall translate the received geographical area(s) or civic address(es) to a list of cell ID(s) and/or list of TAI(s) before relaying it to the MB-SMF;

-	if the NEF discovers the target MB-SMF based on the MBS Service Area and the received MBS Service Area information cannot be covered by the MB-SMF Service Area of a single MB-SMF, the NEF shall reject the request and respond to the AF with an HTTP "403 Forbidden" status code with the response body including the ProblemDetailsTmgiAlloc data structure containing:

-	the ProblemDetails data structure containing the "cause" attribute set to the "MBS_SERVICE_AREA_TOO_LARGE" application error; and optionally

-	the ReducedMbsServArea data structure containing the reduced MBS Service Area information;

and

-	if the received MBS Service Area is not supported (e.g., the received MBS Service Area cannot be covered by the service area(s) of any MB-SMF), the NEF shall skip the following steps below and respond to the AF with an HTTP "403 Forbidden" status code with the response body including the "MBS_SERVICE_AREA_NOT_SUPPORTED" application error.

Upon reception of a successful response from the MB-SMF and successful MBS session creation at the NEF, the NEF shall return a Nnef_MBSSession_Create response with an HTTP "201 Created" status code to theAF including a "Location" header that shall contain the URI of the created "Individual MBS Session" resource, and the response body including the MbsSessionCreateRsp data structure that shall contain:

-	within the "mbsSession" attribute, a representation of the created Individual MBS Session resource encoded using the MbsSession data structure, including:

-	the area session ID assigned by the MB-SMF in the case of a location dependent MBS within the "areaSessionId" attribute of the MbsSession data structure;

-	the allocated TMGI for the MBS session, if the MBS session creation request included a "tmgiAllocReq" attribute requesting TMGI allocation for the MBS session, within the "tmgi" attribute;

-	if unicast transport is used over N6mb/Nmb9, the ingress MB-UPF tunnel information, within the "ingressTunAddr" attribute;

-	if the "serviceType" value is "BROADCAST" and any MBS FSA ID(s) received from the MB-SMF, the list of MBS FSA ID(s) within the "mbsFsaIdList" attribute; and

-	if the "ReducedMbsServArea" feature is supported and the MB-SMF reduced the MBS Service Area initially requested by the AF, the reduced MBS Service Area that can be supported by the network within the "reducedMbsServArea" attribute or the "reducedExtMbsServArea" attribute;

and

-	within the "eventList" attribute, a list of MBS Session Status Event(s) report(s), if available.

If the MBS session creation request contained a request to also create a subscription to MBS session status event(s) within the "mbsSessionSubsc" attribute, the the NEF shall also create a corresponding "Individual MBS Session Subscription" resource and return a representation of it in the HTTP POST response body within the "mbsSessionSubsc" attribute of the MbsSession data structure. The "mbsSessionSubsc" attribute shall contains the identifier of the created "Individual MBS Session Subscription" resource within the "subscriptionId" attribute. The AF shall construct the URI of the created "Individual MBS Session Subscription" resource by appending the path segments "/subscriptions/{subscriptionId}", where the "subscriptionId" takes the value of the received "subscriptionId" attribute, to the URI of the created "Individual MBS Session" resource received within the HTTP Location header.

On failure or if the NEF receives an error code from the PCF, the NRF or the MB-SMF, the NEF shall take proper error handling actions, as specified in clause 5.20.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

##### 4.4.29.3.3	Procedure for MBS session update

This procedure is used by an AF to request the modification of an existing multicast or a broadcast MBS session or, for a location dependent MBS session, the part of an MBS Session within an MBS service area.

In order to request the modification of an existing MBS Session, an AF shall send a Nnef_MBSSession_Update request using the HTTP PATCH method and targeting the URI of the corresponding "Individual MBS Session" resource and the request message body including an array of PatchItem data structure(s) containing the requested modifications. For a multicast or a broadcast MBS session, only the "mbsServiceArea" attribute, and/or the "mbsServInfo" attribute may be modified. For a multicast MBS session, the "activityStatus" attribute may also be modified. For a broadcast MBS session, the "mbsFsaIdList" attribute may also be modified.

At the reception of this HTTP PATCH request for MBS session modification:

-	if updated MBS Service Information is provided and the NEF decided to interact with the PCF during MBS Session Creation as specified in clause 4.4.29.3.2, the NEF shall also interact with the PCF for MBS policy authorization of the received updated MBS Service Information and the update of the corresponding MBS Application Session Context, as specified in 3GPP TS 29.537 [63];

-	if MBS session authorization is successful or when the NEF does not interact with the PCF, the NEF shall interact with the MB-SMF to request the modification of the corresponding MBS session at the MB-SMF as specified in 3GPP TS 29.532 [52];

-	if the NEF receives an "indication that the PCF shall be contacted" within the "contactPcfInd" attribute from the PCF as specified in 3GPP TS 29.537 [63], the NEF shall relay this indication to the MB-SMF;

and

-	if updated MBS Service Area information is provided within the "extMbsServiceArea" attribute, the NEF shall translate the received geographical area(s) or civic address(es) to a list of cell ID(s) and/or list of TAI(s) before relaying it to the MB-SMF.

Upon reception of a successful response from the MB-SMF and successful MBS session modification, the NEF shall return a Nnef_MBSSession_Update response with an HTTP "204 No Content" status code, or when the "ReducedMbsServArea" feature is supported, with either:

-	if no updated MBS Service Area was provided in the corresponding request or an updated MBS Service Area was provided in the corresponding request and was fully accepted by the MB-SMF, an HTTP "204 No Content" status code; or

-	if an updated MBS Service Area was provided in the corresponding request but only partially accepted by the MB-SMF (i.e., the MB-SMF reduced the MBS Service Area), an HTTP "200 OK" status code with the response body including the reduced MBS Service Area information within the MbsSessionUpdateResp data structure.

On failure or if the NEF receives an error responsefrom the PCF or the MB-SMF, the NEF shall take proper error handling actions, as specified in clause 5.20.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

##### 4.4.29.3.4	Procedure for MBS session deletion

This procedure is used by an AF to request the deletion of an existing multicast or a broadcast MBS session or, for a location dependent MBS session, the part of an MBS Session within an MBS service area.

In order to request the deletion of an existing MBS Session, an AF shall send a Nnef_MBSSession_Delete request using the HTTP DELETE method and targeting the URI of the corresponding "Individual MBS Session" resource.

At the reception of this HTTP DELETE request for MBS session deletion:

-	if the NEF decided to interact with the PCF during MBS Session Creation as specified in clause 4.4.29.3.2, the NEF shall also interact with the PCF to request the deletion of the corresponding MBS Application Session Context, as specified in 3GPP TS 29.537 [63]; and

-	the NEF shall interact with the MB-SMF to request the deletion of the corresponding MBS Session.

Upon success, the NEF shall return a Nnef_MBSSession_Delete response with an HTTP "204 No Content" status code.On failure or if the NEF receives an error responsefrom the PCF or the MB-SMF, the NEF shall take proper error handling actions, as specified in clause 5.20.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

##### 4.4.29.3.5	Procedure for MBS session status subscription

This procedure is used by an AF to request to create a subscription to MBS session status event(s) reportingfor a multicast or a broadcast MBS session or, for a location dependent MBS session, the part of an MBS Session within an MBS service area.

In order to request the creation of a new subscription to MBS Session status event(s) reporting, an AF shall send a Nnef_MBSSession_StatusSubscribe request to the NEF using the HTTP POST method and targeting the "MBS Session Subscriptions" collection resource, with the request body including the MbsSessionSubsc data structure.

On successful MBS session subscription creation, the NEF shall return a Nnef_MBSSession_StatusSubscribe response with an HTTP "201 Created" status code to the AF, including a "Location" header containing the URI of the created "Individual MBS Session Subscription" resource and the response body containing a representation of the created resource within the MbsSessionSubsc data structure.

On failure or if the NEF receives an error responsefrom the MB-SMF, the NEF shall take proper error handling actions, as specified in clause 5.20.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

##### 4.4.29.3.6	Procedure for MBS session status unsubscription

This procedure is used by an AF to request the deletion of an existing subscription to MBS session status event(s) reportingfor a multicast or a broadcast MBS session or, for a location dependent MBS session, the part of an MBS Session within an MBS service area.

In order to request the deletion of an existing subscription to MBS Session status event(s) reporting, an AF shall send a Nnef_MBSSession_StatusUnsubscribe request to the NEF using the HTTP DELETE method and targeting the corresponding "Individual MBS Session Subscription" resource.

On successful deletion of the subscription, the NEF shall return a Nnef_MBSSession_StatusUnsubcribe response with an HTTP "204 No Content" status code.

On failure or if the NEF receives an error responsefrom the MB-SMF, the NEF shall take proper error handling actions, as specified in clause 5.20.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

##### 4.4.29.3.7	Procedure for MBS session status notification

This procedure is used by the NEF to send MBS session status event(s) notifications to a previously subscribed AF.

In order to send an MBS Session status event(s) notification, the NEF shall send a Nnef_MBSSession_StatusNotify request to the AF using the HTTP POST method and targeting the notification URI provided by the AF during the corresponding MBS session subscription creation/modification, with the request body including the MbsSessionStatusNotif data structure that shall contain:

-	within the "eventList" attribute, the reported MBS session event(s) ) and the related information, encoded via the MbsSessionEventReportList data structure that shall contain:

-	within the "eventReportList" attribute, one or several MBS session event report(s), with each one of them encoded using the MbsSessionEventReport data structure that shall contain:

-	within the "eventType" attribute, the reported MBS session status event;

-	within the "timeStamp" attribute, the time at which the event is generated, if available;

-	within the "ingressTunAddrInfo" attribute, the ingress tunnel address to use to send MBS session data over N6mb/Nmb9 interface, if the "eventType" attribute is set to "INGRESS_TUNNEL_ADD_CHANGE";

and

-	within the "eventList" attribute, the list of MBS session events to be reported, encoded via the MbsSessionEventReportList data structure that shall contain;

-	within the "eventReportList" attribute, one or several individual MBS session event report(s), with each one of them encoded within the MbsSessionEventReport data structure that shall contain:

-	within the "broadcastDelStatus" attribute, the broadcast delivery status (e.g. whether the MBS session is STARTED or TERMINATED), if the "eventType" attribute is set to "BROADCAST_DELIVERY_STATUS".

Upon reception of this notification request, the AF shall acknowledge its successful reception by sending a Nnef_MBSSession_StatusNotify response with an HTTP "204 No Content" status code.

On failure, the AF shall take proper error handling actions, as specified in clause 5.20.7, and respond to the NEF with an appropriate error status code.

#### 4.4.29.4	Procedures for MBS Parameters Provisioning

##### 4.4.29.4.1	General

The procedures described in the clauses below are used by an AF to perform MBS parameters provisioning, in order to carry out one or more of the following procedures:

-	Multicast MBS Session Authorization information provisioning as defined in clause 7.2.9 of 3GPP TS 23.247 [53].

-	Multicast MBS Session Assistance Information provisioning as defined in clause 7.2.9a of 3GPP TS 23.247[53].

NOTE 1:	The stage 2 Nnef_ParameterProvision API for MBS Parameters Provisioning is implemented in stage 3 via the Nnef_MBSSession API.

NOTE 2:	An AF can perform Multicast MBS Session Authorization information provisioning and Multicast MBS Session Assistance Information provisioning simultaneously, i.e., via the same MBS Parameters Provisioning.

##### 4.4.29.4.2	Procedure for multicast MBS Session Authorization information provisioning

This procedure is used by an AF to request the creation/update/deletion of an MBS Session Authorization information provisioning for a multicast MBS group.

In order to request the creation of an MBS Parameters Provisioning for the purpose of MBS Session Authorization information provisioning for a multicast MBS group, an AF shall trigger the Nnef_MBSSession API by sending an HTTP POST request to the NEF targeting the "MBS Parameters Provisionings" collection resource, with the request body including the MbsPpData data structure that shall contain:

-	within the "afId" attribute, the identifier of the AF that is sending the request;

-	within the "mbsSessAuthData" attribute, the MBS Session Authorization information data to be provisioned, encoded via the MbsSessAuthData data structure that shall contain:

-	within the "extGroupId" attribute, the external group identifier of the targeted multicast MBS Group; and

-	within the "gpsisList" attribute, the list of the GPSI(s) of the member UE(s) constituting the multicast MBS group, if the multicast MBS group has not yet been created or the list of its member(s) needs to be updated; and

-	within the "mbsSessionIdList" attribute, the identifier(s) of the multicast MBS Session(s) that the multicast MBS group is authorized to join;

and

-	within the "suppFeat" attribute, the features supported by the AF, if applicable (i.e. feature negociation needs to take place).

The NEF shall then check whether the AF is authorized to perform this operation or not as defined in clause 7.2.9 of 3GPP TS 23.247 [53]. If the AF is authorized, the NEF shall trigger the Nudm_ParameterProvision service API of the UDM to request the provisioning of the received MBS Session Authorization information.

Upon success and reception of a successful response from the UDM as defined in 3GPP TS 29.503 [17], the NEF shall respond to the AF with an HTTP "201 Created" status code including a Location header field containing the URI of the created resource, and the response body containing the MbsPpData data structure containing a representation of the created "Individual MBS Parameters Provisioning" resource.

On failure or if the NEF receives an error response from the UDM, the NEF shall take proper error handling actions, as specified in clause 5.20.7, and respond to the AF with an appropriate error status code.

In order to request the update of an existing "Individual MBS Parameters Provisioning" resource for the purpose of MBS Session Authorization information provisioning for a multicast MBS group, an AF shall trigger the Nnef_MBSSession API by sending to the NEF either:

-	an HTTP PUT request targeting the concerned "Individual MBS Parameters Provisioning" resource with the request body including the MbsPpData data structure; or

-	an HTTP PATCH request targeting the concerned "Individual MBS Parameters Provisioning" resource with the request body including the MbsPpDataPatch data structure.

After authorizing the request, the NEF shall interact with the UDM via the the Nudm_ParameterProvision service API to request the provisioning of the received updated MBS Session Authorization information.

Upon success and reception of a successful response from the UDM as defined in 3GPP TS 29.503 [17], the NEF shall respond to the AF with an HTTP "200 OK" status code with the response body containing a representation of the updated Individual MBS Parameters Provisioning resource within the MbsPpData data structure, or an HTTP "204 No Content" status code.

On failure or if the NEF receives an error response from the UDM, the NEF shall take proper error handling actions, as specified in clause 5.20.7, and respond to the AF with an appropriate error status code.

In order to request the deletion of an existing "Individual MBS Parameters Provisioning" resource for the purpose of MBS Session Authorization information provisioning for a multicast MBS group, an AF shall trigger the Nnef_MBSSession API by sending an HTTP DELETE request targeting the concerned "Individual MBS Parameters Provisioning" resource to the NEF. After authorizing the request, the NEF shall interact with the UDM via the the Nudm_ParameterProvision service API to request to update accordingly the MBS Session Authorization information.

Upon success and reception of a successful response from the UDM as defined in 3GPP TS 29.503 [17], the NEF shall respond to the AF with an HTTP "204 No Content" status code.

On failure or if the NEF receives an error response from the UDM, the NEF shall take proper error handling actions, as specified in clause 5.20.7, and respond to the AF with an appropriate error status code.

##### 4.4.29.4.3	Procedure for multicast MBS Session Assistance information provisioning

When the "5MBS2" feature is supported, this procedure is used by an AF to request the creation/update/deletion of an MBS Session Assistance information provisioning for multicast MBS sessions.

In order to request the creation of an MBS Parameters Provisioning for the purpose of MBS Session Assistance information provisioning for multicast MBS sessions, an AF shall trigger the Nnef_MBSSession API by sending an HTTP POST request to the NEF targeting the "MBS Parameters Provisionings" collection resource, with the request body including the MbsPpData data structure that shall contain:

-	within the "afId" attribute, the identifier of the AF that is sending the request;

-	within the "mbsSessAssistInfo" attribute, the MBS Session Assistance information to be provisioned; and

-	within the "suppFeat" attribute, the features supported by the AF, which shall include the support of the "5MBS2" feature.

The NEF shall then check whether the AF is authorized to perform this operation or not, as defined in clause 7.2.9a of 3GPP TS 23.247 [53]. If the AF is authorized, the NEF shall trigger the Nudm_ParameterProvision service API of the UDM to request the provisioning of the received MBS Session Assistance information.

Upon success and reception of a successful response from the UDM, as defined in 3GPP TS 29.503 [17], the NEF shall respond to the AF with an HTTP "201 Created" status code including an HTTP Location header field containing the URI of the created resource, and the response body containing the representation of the created "Individual MBS Parameters Provisioning" resource within the MbsPpData data structure.

On failure and/or if the NEF receives an error response from the UDM, the NEF shall take proper error handling actions, as specified in clause 5.20.7, and respond to the AF with an appropriate error status code.

In order to request the update or deletion of an existing "Individual MBS Parameters Provisioning" resource for the purpose of MBS Session Assistance information provisioning/update/deletion for multicast MBS sessions, an AF shall trigger the Nnef_MBSSession API by reusing the same update/deletion procedures defined in clause 4.4.29.4.2.

#### 4.4.29.5	Procedures for MBS User Service management

##### 4.4.29.5.1	General

The procedures described in the clauses below are used by an external/untrusted AF (e.g. MBS Application Provider that lies outside the trusted DN) to manage MBS User Services via the NEF, i.e. create, retrieve, update and delete an MBS User Service, as defined in 3GPP TS 26.502 [65].

NOTE:	The procedures defined in the clauses below are not applicable for the MBS group message delivery feature specified in clauses 6.15 and 7.5 of 3GPP TS 23.247 [53]. For MBS group message delivery, the NEF plays the role of an AF and is hence responsible for the establishment and management of the related MBS User Service based on the received MBS group message delivery request from the AF. The NEF interacts for this purpose directly with the MBSF using the Nmbsf_MBSUserService API defined in 3GPP TS 29.580 [66]. The detailed procedures applicable for MBS group message delivery are defined in clause 4.4.29.7.

##### 4.4.29.5.2	Procedure for MBS User Service creation

This procedure is used by an AF to request the creation of a new MBS User Service at the NEF.

In order to request the creation of an MBS User Service, an AF shall send a Nnef_MBSUserService_Create request to the NEF using the HTTP POST method and targeting the "MBS User Services" collection resource, with the request message body including the MBSUserService data structure, as specified in clause 5.26.2.2.3.2.

The NEF shall then check whether the AF is authorized to perform this operation or not. If the AF is authorized, the NEF shall then trigger the Nmbsf_MBSUserService service API of the MBSF to request the creation of the corresponding MBS User Service at the MBSF, as specified in 3GPP TS 29.580 [66].

Upon reception of a successful response from the MBSF, as defined in 3GPP TS 29.580 [66], the NEF shall return a Nnef_MBSUserService_Create response with an HTTP "201 Created" status code including a "Location" header field that shall contain the URI of the created resource, and the response body containing a representation of the created "Individual MBS User Service" resource within the MBSUserService data structure, as specified in clause 5.26.2.2.3.2.

On failure or if the NEF receives an error response from the MBSF, the NEF shall take proper error handling actions, as specified in clause 5.26.7, and respond to the AF with an appropriate error status code.

##### 4.4.29.5.3	Procedure for MBS User Service retrieval

This procedure is used by an AF to request the retrieval of an existing MBS User Service at the NEF.

In order to request the retrieval of an existing MBS User Service, an AF shall send a Nnef_MBSUserService_Retrieve request using the HTTP GET method and targeting the URI of the concerned "Individual MBS User Service" resource, as specified in clause 5.26.2.3.3.1.

The NEF shall then check whether the AF is authorized to perform this operation or not. If the AF is authorized, the NEF shall then trigger the Nmbsf_MBSUserService service API of the MBSF to request the retrieval of the corresponding MBS User Service at the MBSF, as specified in 3GPP TS 29.580 [66].

Upon reception of a successful response from the MBSF, as defined in 3GPP TS 29.580 [66], the NEF shall return a Nnef_MBSUserService_Retrieve response with an HTTP "200 OK" status code and the response body containing a representation of the requested Individual MBS User Service resource within the MBSUserService data structure, as specified in clauses 5.26.2.3.3.1.

On failure or if the NEF receives an error code from the MBSF, the NEF shall take proper error handling actions, as specified in clause 5.26.7, and respond to the AF with an appropriate error status code.

##### 4.4.29.5.4	Procedure for MBS User Service update/modification

This procedure is used by an AF to request the update/modification of an existing MBS User Service at the NEF.

In order to request the update of an existing MBS User Service, an AF shall send a Nnef_MBSUserService_Update request using the HTTP PUT method and targeting the URI of the corresponding "Individual MBS User Service" resource, with the request body including the MBSUserService data structure, as specified in clause 5.26.2.3.3.2.

In order to request the modification of an existing MBS User Service, an AF shall send a Nnef_MBSUserService_Update request using the HTTP PATCH method and targeting the URI of the corresponding "Individual MBS User Service" resource, with the request body including the MBSUserServicePatch data structure, as specified in clause 5.26.2.3.3.3.

The NEF shall then check whether the AF is authorized to perform this operation or not. If the AF is authorized, the NEF shall then trigger the Nmbsf_MBSUserService service API of the MBSF to request the update/modification of the corresponding MBS User Service at the MBSF, as specified in 3GPP TS 29.580 [66].

Upon reception of a successful response from the MBSF, as defined in 3GPP TS 29.580 [66], the NEF shall return a Nnef_MBSUserService_Update response with an HTTP "200 OK" status code with the response body containing a representation of the updated Individual MBS User Service resource within the MBSUserService data structure, or an HTTP "204 No Content" status code, as specified in clause 5.26.2.3.3.2 or clause 5.26.2.3.3.3.

On failure or if the NEF receives an error code from the MBSF, the NEF shall take proper error handling actions, as specified in clause 5.26.7, and respond to the AF with an appropriate error status code.

##### 4.4.29.5.5	Procedure for MBS User Service deletion

This procedure is used by an AF to request the deletion of an existing MBS User Service at the NEF.

In order to request the deletion of an existing MBS User Service, an AF shall send a Nnef_MBSUserService_Delete request using the HTTP DELETE method and targeting the URI of the concerned "Individual MBS User Service" resource, as specified in clause 5.26.2.3.3.4.

NOTE:	The Nnef_MBSUserService_Delete service operation corresponds to the stage 2 Nnef_MBSUserService_Destroy service operation defined in 3GPP TS 26.502 [65].

The NEF shall then check whether the AF is authorized to perform this operation or not. If the AF is authorized, the NEF shall then trigger the Nmbsf_MBSUserService service API of the MBSF to request the deletion of the corresponding MBS User Service at the MBSF, as specified in 3GPP TS 29.580 [66].

Upon reception of a successful response from the MBSF, as defined in 3GPP TS 29.580 [66], the NEF shall return a Nnef_MBSUserService_Delete response with an HTTP "204 No Content" status code, as specified in clause 5.26.2.3.3.4.

On failure or if the NEF receives an error code from the MBSF, the NEF shall take proper error handling actions, as specified in clause 5.26.7, and respond to the AF with an appropriate error status code.

#### 4.4.29.6	Procedures for MBS User Data Ingest Session management

##### 4.4.29.6.1	General

The procedures described in the clauses below are used by an external/untrusted AF (e.g. MBS Application Provider that lies outside the trusted DN) to manage an MBS User Data Ingest Session along with its subordinate MBS Distribution Session(s) via the NEF, i.e. create, retrieve, update/modify and delete an MBS User Data Ingest Session, create, retrieve, update/modify and delete an MBS User Data Ingest Session Status subscription, and manage the related MBS User Data Ingest Session Status subscriptions/notifications, as defined in 3GPP TS 26.502 [65].

NOTE:	The procedures defined in the clauses below are not applicable for the MBS group message delivery feature specified in clauses 6.15 and 7.5 of 3GPP TS 23.247 [53]. For MBS group message delivery, the NEF plays the role of an AF and is hence responsible for the establishment and management of the related MBS User Data Ingest Session based on the received MBS group message delivery request from the AF. The NEF interacts for this purpose directly with the MBSF using the Nmbsf_MBSUserDataIngestSession API defined in 3GPP TS 29.580 [66]. The detailed procedures applicable for MBS group message delivery are defined in clause 4.4.29.7.

##### 4.4.29.6.2	Procedure for MBS User Data Ingest Session creation

This procedure is used by an AF to request the creation of a new MBS User Data Ingest Session at the NEF.

In order to request the creation of an MBS User Data Ingest Session, including a set of subordinate MBS Distribution Session(s), an AF shall send a Nnef_MBSUserDataIngestSession_Create request message to the NEF using the HTTP POST method and targeting the "MBS User Data Ingest Sessions" collection resource, with the request message body including the MBSUserDataIngSession data structure, as specified in clause 5.27.2.2.3.2.

The NEF shall then check whether the AF is authorized to perform this operation or not. If the AF is authorized, the NEF shall then trigger the Nmbsf_MBSUserDataIngestSession API of the MBSF to request the creation of the corresponding MBS User Data Ingest Session at the MBSF, as specified in 3GPP TS 29.580 [66].

Upon reception of a successful response from the MBSF, as defined in 3GPP TS 29.580 [66], the NEF shall return a Nnef_MBSUserDataIngestSession_Create response message with an HTTP "201 Created" status code including a "Location" header field that shall contain the URI of the created resource, and the response body containing a representation of the created "Individual MBS User Data Ingest Session" resource within the MBSUserDataIngSession data structure, as specified in clause 5.27.2.2.3.2.

On failure or if the NEF receives an error response from the MBSF, the NEF shall take proper error handling actions, as specified in clause 5.27.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a ProblemDetails data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

##### 4.4.29.6.3	Procedure for MBS User Data Ingest Session retrieval

This procedure is used by an AF to request the retrieval of an existing MBS User Data Ingest Session at the NEF.

In order to request the retrieval of an existing MBS User Data Ingest Session, an AF shall send a Nnef_MBSUserDataIngestSession_Retrieve request message using the HTTP GET method and targeting the URI of the concerned "Individual MBS User Data Ingest Session" resource, as specified in clause 5.27.2.3.3.1.

The NEF shall then check whether the AF is authorized to perform this operation or not. If the AF is authorized, the NEF shall then trigger the Nmbsf_MBSUserDataIngestSession service API of the MBSF to request the retrieval of the corresponding MBS User Data Ingest Session at the MBSF, as specified in 3GPP TS 29.580 [66].

Upon reception of a successful response from the MBSF, as defined in 3GPP TS 29.580 [66], the NEF shall return a Nnef_MBSUserDataIngestSession_Retrieve response message with an HTTP "200 OK" status code and the response body containing a representation of the requested Individual MBS User Data Ingest Session resource within the MBSUserDataIngSession data structure, as specified in clauses 5.27.2.3.3.1.

On failure or if the NEF receives an error code from the MBSF, the NEF shall take proper error handling actions, as specified in clause 5.27.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a ProblemDetails data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

##### 4.4.29.6.4	Procedure for MBS User Data Ingest Session update/modification

This procedure is used by an AF to request the update/modification of an existing MBS User Data Ingest Session at the NEF.

In order to request the update of an existing MBS User Data Ingest Session, an AF shall send a Nnef_MBSUserDataIngestSession_Update request message using the HTTP PUT method and targeting the URI of the corresponding "Individual MBS User Data Ingest Session" resource, with the request body including the MBSUserDataIngSession data structure, as specified in clause 5.27.2.3.3.2.

In order to request the modification of an existing MBS User Data Ingest Session, an AF shall send a Nnef_MBSUserDataIngestSession_Update request message using the HTTP PATCH method and targeting the URI of the corresponding "Individual MBS User Data Ingest Session" resource, with the request body including the MBSUserDataIngSessionPatch data structure, as specified in clause 5.27.2.3.3.3.

The NEF shall then check whether the AF is authorized to perform this operation or not. If the AF is authorized, the NEF shall then trigger the Nmbsf_MBSUserDataIngestSession service API of the MBSF to request the update/modification of the corresponding MBS User Data Ingest Session at the MBSF, as specified in 3GPP TS 29.580 [66].

Upon reception of a successful response from the MBSF, as defined in 3GPP TS 29.580 [66], the NEF shall return a Nnef_MBSUserDataIngestSession_Update response message with an HTTP "200 OK" status code with the response body containing a representation of the updated Individual MBS User Data Ingest Session resource within the MBSUserDataIngSession data structure, or an HTTP "204 No Content" status code, as specified in clause 5.27.2.3.3.2 or clause 5.27.2.3.3.3.

On failure or if the NEF receives an error code from the MBSF, the NEF shall take proper error handling actions, as specified in clause 5.27.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a ProblemDetails data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

##### 4.4.29.6.5	Procedure for MBS User Data Ingest Session deletion

This procedure is used by an AF to request the deletion of an existing MBS User Data Ingest Session at the NEF.

In order to request the deletion of an existing MBS User Data Ingest Session, an AF shall send a Nnef_MBSUserDataIngestSession_Delete request message using the HTTP DELETE method and targeting the URI of the concerned "Individual MBS User Data Ingest Session" resource, as specified in clause 5.27.2.3.3.4.

NOTE:	The Nnef_MBSUserDataIngestSession_Delete service operation corresponds to the stage 2 Nnef_MBSUserDataIngestSession_Destroy service operation defined in 3GPP TS 26.502 [65].

The NEF shall then check whether the AF is authorized to perform this operation or not. If the AF is authorized, the NEF shall then trigger the Nmbsf_MBSUserDataIngestSession service API of the MBSF to request the deletion of the corresponding MBS User Data Ingest Session at the MBSF, as specified in 3GPP TS 29.580 [66].

Upon reception of a successful response from the MBSF, as defined in 3GPP TS 29.580 [66], the NEF shall return a Nnef_MBSUserDataIngestSession_Delete response message with an HTTP "204 No Content" status code, as specified in clause 5.27.2.3.3.4.

On failure or if the NEF receives an error code from the MBSF, the NEF shall take proper error handling actions, as specified in clause 5.27.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a ProblemDetails data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

##### 4.4.29.6.6	Procedure for MBS User Data Ingest Session Status Subscription

This procedure is used by an AF to subscribe to MBS User Data Ingest Session status event(s) reporting at the NEF.

In order to request the creation of an MBS User Data Ingest Session Status Subscription, an AF shall send a Nnef_MBSUserDataIngestSession_StatusSubscribe request message to the NEF using the HTTP POST method and targeting the "MBS User Data Ingest Session Status Subscriptions" collection resource, with the request message body including the MBSUserDataIngStatSubsc data structure, as specified in clause 5.27.2.4.3.2.

The NEF shall then check whether the AF is authorized to perform this operation or not. If the AF is authorized, the NEF shall then trigger the Nmbsf_MBSUserDataIngestSession API of the MBSF to request the creation of the corresponding MBS User Data Ingest Session Status Subscription at the MBSF, as specified in 3GPP TS 29.580 [66].

Upon reception of a successful response from the MBSF, as defined in 3GPP TS 29.580 [66], the NEF shall return a Nnef_MBSUserDataIngestSession_StatusSubscribe response message with an HTTP "201 Created" status code including a "Location" header field that shall contain the URI of the created resource, and the response body containing a representation of the created "Individual MBS User Data Ingest Session Status Subscription" resource within the MBSUserDataIngStatSubsc data structure, as specified in clause 5.27.2.4.3.2.

On failure or if the NEF receives an error response from the MBSF, the NEF shall take proper error handling actions, as specified in clause 5.27.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a ProblemDetails data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

##### 4.4.29.6.7	Procedure for MBS User Data Ingest Session Status update/modification

This procedure is used by an AF to request the update/modification of an existing MBS User Data Ingest Session Status Subscription at the NEF.

In order to request the update of an existing MBS User Data Ingest Session Status Subscription, an AF shall send a Nnef_MBSUserDataIngestSession_StatusSubscribeMod request message using the HTTP PUT method and targeting the URI of the corresponding "Individual MBS User Data Ingest Session Status Subscription" resource, with the request body including the MBSUserDataIngStatSubsc data structure, as specified in clause 5.27.2.5.3.2.

In order to request the modification of an existing MBS User Data Ingest Session Status Subscription, an AF shall send a Nnef_MBSUserDataIngestSession_StatusSubscribeMod request message using the HTTP PATCH method and targeting the URI of the corresponding "Individual MBS User Data Ingest Session Status Subscription" resource, with the request body including the MBSUserDataIngStatSubscPatch data structure, as specified in clause 5.27.2.5.3.3.

The NEF shall then check whether the AF is authorized to perform this operation or not. If the AF is authorized, the NEF shall then trigger the Nmbsf_MBSUserDataIngestSession service API of the MBSF to request the update/modification of the corresponding MBS User Data Ingest Session Status Subscription at the MBSF, as specified in 3GPP TS 29.580 [66].

Upon reception of a successful response from the MBSF, as defined in 3GPP TS 29.580 [66], the NEF shall return a Nnef_MBSUserDataIngestSession_StatusSubscribeMod response message with an HTTP "200 OK" status code with the response body containing a representation of the updated Individual MBS User Data Ingest Session resource within the MBSUserDataIngStatSubsc data structure, or an HTTP "204 No Content" status code, as specified in clause 5.27.2.5.3.2 or clause 5.27.2.5.3.3.

On failure or if the NEF receives an error code from the MBSF, the NEF shall take proper error handling actions, as specified in clause 5.27.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a ProblemDetails data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

##### 4.4.29.6.8	Procedure for MBS User Data Ingest Session Status Unsubscription

This procedure is used by an AF to request the deletion of an existing MBS User Data Ingest Session Status Subscription at the NEF.

In order to request the deletion of an existing MBS User Data Ingest Session Status Subscription, an AF shall send a Nnef_MBSUserDataIngestSession_StatusUnsubscribe request message using the HTTP DELETE method and targeting the URI of the concerned Individual MBS User Data Ingest Session Stats Subscription resource, as specified in clause 5.27.2.5.3.4.

The NEF shall then check whether the AF is authorized to perform this operation or not. If the AF is authorized, the NEF shall then trigger the Nmbsf_MBSUserDataIngestSession service API of the MBSF to request the deletion of the corresponding MBS User Data Ingest Session Status Subscription at the MBSF, as specified in 3GPP TS 29.580 [66].

Upon reception of a successful response from the MBSF, as defined in 3GPP TS 29.580 [66], the NEF shall return a Nnef_MBSUserDataIngestSession_StatusUnsubscribe response message with an HTTP "204 No Content" status code, as specified in clause 5.27.2.5.3.4.

On failure or if the NEF receives an error code from the MBSF, the NEF shall take proper error handling actions, as specified in clause 5.27.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a ProblemDetails data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

##### 4.4.29.6.9	Procedure for MBS User Data Ingest Session Status Notification

This procedure is used by the NEF to send MBS User Data Ingest Session status change notifications to a previously subscribed AF.

Upon reception of an MBS User Data Ingest Session Status Notification from the MBSF, as specified in 3GPP TS 29.580 [66], the NEF shall relay this notification to the AF by sending a Nnef_MBSUserDataIngestSession_StatusNotify request message to the AF using the HTTP POST method and targeting the notification URI provided by the AF during the creation of the corresponding MBS User Data Ingest Session Status Subscription, with the request body including the MBSUserDataIngStatNotif data structure, as specified in clause 5.27.4.2.3.1.

Upon successful reception of this notification request, the AF shall acknowledge its successful reception by sending a Nnef_MBSUserDataIngestSession_StatusNotify response message with an HTTP "204 No Content" status code, as specified in clause 5.27.4.2.3.1.

On failure, the AF shall take proper error handling actions, as specified in clause 5.27.7, and respond to the NEF with an appropriate error status code.

#### 4.4.29.7	Procedures for MBS Group Message Delivery Management

##### 4.4.29.7.1	General

The procedures described in the clauses below are used by an AF to request and manage MBS Group Message Delivery as defined in clauses 6.15 and 7.5 of 3GPP TS 23.247 [53].

##### 4.4.29.7.2	Procedure for MBS Group Message Delivery Creation

This procedure is used by an AF to request the creation of an MBS Group Message Delivery at the NEF (see also clause 7.5.1 of 3GPP TS 23.247 [53]).

In order to request the creation of an MBS Group Message Delivery, an AF shall invoke the Nnef_MBSGroupMsgDelivery_Create service operation by sending an HTTP POST request message to the NEF, targeting the "MBS Group Message Deliveries" collection resource, with the request body including the MbsGroupMsgDel data structure.

The NEF shall then check whether the AF is authorized to perform this operation or not, as defined in clause 6.1.1 of 3GPP TS 23.247 [53]. If the AF is authorized, then:

-	the NEF may trigger MBS User Service and MBS User Data Ingest Session provisioning towards the MBSF using the Object Distribution Method, as specified 3GPP TS 29.580 [66] (see also 3GPP TS 26.502 [65]); and

-	if the AF provides the MBS Service Area in the form of geographical area(s) or civic address(es), the NEF shall translate this information into a list of TAI(s) and/or a list of cell ID(s) before sending the MBS Service Area information as part of MBS User Data Ingest Session provisioning towards the MBSF.

Upon success and/or reception of a successful response from the MBSF, the NEF shall send a Nnef_MBSGroupMsgDelivery_Create response to the AF with an HTTP "201 Created" status code and the response body including the representation of the created "Individual MBS Group Message Delivery" resource within the MbsGroupMsgDel data structure.

On failure or if the NEF receives an error response from the MBSF, the NEF shall take proper error handling actions, as specified in clause 5.29.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a ProblemDetails data structure with the "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

##### 4.4.29.7.3	Procedure for MBS Group Message Delivery Update

This procedure is used by an AF to request the modification of an existing "Individual MBS Group Message Delivery" resource at the NEF (see also clause 7.5.2 of 3GPP TS 23.247 [53]).

In order to modify a previously submitted MBS Group Message Delivery, an AF shall invoke the Nnef_MBSGroupMsgDelivery_Update service operation by sending an HTTP PATCH request message to the NEF, targeting the corresponding "Individual MBS Group Message Delivery" resource, with the request body including the MbsGroupMsgDelPatch data structure.

The NEF shall then check whether the AF is authorized to perform this operation or not, as defined in clause 6.1.1 of 3GPP TS 23.247 [53]. If the AF is authorized, then:

-	the NEF may further interact with the MBSF to request the modification of the associated MBS User Service and MBS User Data Ingest Session instances as specified in 3GPP TS 29.580 [66] (see also 3GPP TS 26.502 [65]); and

-	if the AF provides an updated MBS Service Area and it is in the form of geographical area(s) or civic address(es), the NEF shall translate this information into a list of TAI(s) and/or a list of cell ID(s) before sending the updated MBS Service Area information as part of MBS User Data Ingest Session modification towards the MBSF.

Upon success and/or reception of a successful response from the MBSF confirming that the requested modifications were successfully performed, the NEF shall send a Nnef_MBSGroupMsgDelivery_Update response to the AF with either:

-	an HTTP "200 OK" status code with the response body containing the updated representation of the "Individual MBS Group Message Delivery" resource within the MbsGroupMsgDel data structure; or

-	an HTTP "204 No Content" status code.

On failure or if the NEF receives an error response from the MBSF, the NEF shall take proper error handling actions, as specified in clause 5.29.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a ProblemDetails data structure with the "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

##### 4.4.29.7.4	Procedure for MBS Group Message Delivery Deletion

This procedure is used by an AF to request the deletion of an existing "Individual MBS Group Message Delivery" resource at the NEF (see also clause 7.5.3 of 3GPP TS 23.247 [53]).

In order to delete a previously submitted MBS Group Message Delivery, an AF shall invoke the Nnef_MBSGroupMsgDelivery_Delete service operation by sending an HTTP DELETE request message to the NEF, targeting the corresponding "Individual MBS Group Message Delivery" resource.

The NEF shall then check whether the AF is authorized to perform this operation or not, as defined in clause 6.1.1 of 3GPP TS 23.247 [53]. If the AF is authorized, the NEF may further interact with the MBSF to request the deletion of the associated MBS User Service and MBS User Data Ingest Session instances as specified in 3GPP TS 29.580 [66] (see also 3GPP TS 26.502 [65]).

NOTE:	When the Nnef_MBSGroupMsgDelivery_Delete request is received before the corresponding MBS Group Message is delivered or before the status of the corresponding MBS Group Message Delivery is received, then this indicates the revocation of the previously submitted MBS Group Message.

Upon success and/or reception of a successful response from the MBSF, the NEF shall send a Nnef_MBSGroupMsgDelivery_Delete response to the AF with an HTTP "204 No Content" status code.

On failure or if the NEF receives an error response from the MBSF, the NEF shall take proper error handling actions, as specified in clause 5.29.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a ProblemDetails data structure with the "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

##### 4.4.29.7.5	Procedure for MBS Group Message Delivery Status Notification

This procedure is used by the NEF to notify a previously subscribed AF of the status of the previously submitted MBS Group Message Delivery (see also clause 7.5.1 of 3GPP TS 23.247 [53]).

In order to notify an AF of the status of a previously submitted MBS Group Message Delivery, the NEF shall invoke the Nnef_MBSGroupMsgDelivery_StatusNotify service operation by sending an HTTP POST request message to the AF targeting the notification URI (i.e., "{notifUri}") received during the creation/update of the corresponding MBS Group Message Delivery, as defind in clauses 4.4.29.7.2 and 4.4.29.7.3, with the request body including the MbsGroupMsgDelStatusNotif data structure.

Upon reception of this notification request and its successful processing, the AF shall send a Nnef_MBSGroupMsgDelivery_StatusNotify response message with an HTTP "204 No Content" status code.

On failure, the AF shall take proper error handling actions, as specified in clause 5.29.7, and respond to the NEF with an appropriate error status code.

### 4.4.30	Procedures for Data Reporting

#### 4.4.30.1	General

The procedures in this clause are used by an AF to obtain data collection and reporting information and provide Data Reports, as defined in clause 4.2 of 3GPP TS 26.531 [59] and 3GPP TS 26.532 [60].

#### 4.4.30.2	Procedure for Data Reporting Session Management

This procedure is used by an AF to request the creation/update/delection of a Data Reporting Session in order to obtain data collection and reporting information.

In order to request the creation of a Data Reporting Session, an AF shall send a Nnef_DataReporting_Create request to the NEF using the HTTP POST method, targeting the "Data Reporting Sessions" collection resource with the request message body including the DataReportingSession data structure as defined in clause 5.23.2.2.3.1.

In order to read an existing Individual Data Reporting Session, an AF shall send a Nnef_DataReporting_Retrieve request to the NEF using the HTTP GET method, targeting the concerned "Individual Data Reporting Session" resource. If successful, the response message body contains the requested DataReportingSession data structure as defined in clause 5.23.2.3.3.1.

In order to request the update of an existing Data Reporting Session, an AF shall send a Nnef_DataReporting_Update request to the NEF using the HTTP PUT method, targeting the concerned "Individual Data Reporting Session" resource with the request message body including the updated resource representation within the DataReportingSession data structure as defined in clause 5.23.2.3.3.2.

In order to request the deletion of an existing Data Reporting Session, an AF shall send a Nnef_DataReporting_Delete request to the NEF using the HTTP DELETE method, targeting the concerned "Individual Data Reporting Session" resource as defined in clause 5.23.2.3.3.3.

At the reception of the HTTP POST GET/PUT/DELETE requests from the AF, the NEF shall trigger the necessary interaction with the DCAF as specified in 3GPP TS 26.532 [60] and:

-	for an HTTP GET request, retrieve the requested "Individual Data Reporting Session" resource and respond to the AF with an HTTP "200 OK" status code;

-	for an HTTP POST request, create a new "Individual Data Reporting Session" resource and respond to the AF with an HTTP "200 OK" status code including an HTTP Location header field containing the URI of the created resource and the response body including a representation of the created "Individual Data Reporting Session" resource within the DataReportingSession data structure;

-	for an HTTP PUT request, update the concerned "Individual Data Reporting Session" resource and respond to the AF with an HTTP "200 OK" status code with the response body including a representation of the updated "Individual Data Reporting Session" resource within the DataReportingSession data structure; and

-	for an HTTP DELETE request, delete the corresponding "Individual Data Reporting Session" resource, and respond to the AF with an HTTP "204 No Content" status code.

#### 4.4.30.3	Procedure for Data Report

This procedure is used by an AF to send collected UE Data Reports to the NEF.

In order to send a collected UE Data Report, an AF shall use the "Report" custom operation. The AF shall send for this purpose an HTTP POST request targeting the URI "{apiRoot}/3gpp-data-reporting/v1/sessions/{sessionId}/report", with the request message body including the DataReport data structure specified in 3GPP TS 26.532 [60]. Upon successful reception of the report, the NEF shall respond to the AF with an HTTP "200 OK" status code.

### 4.4.31	Procedures for Data Reporting Provisioning

#### 4.4.31.1	General

The procedures in this clause are used by an AF to supply data collection and reporting provisioning information in the form of Data Reporting Provisioning resources, as defined in clause 4.2 of 3GPP TS 26.531 [59] and 3GPP TS 26.532 [60].

#### 4.4.31.2	Procedure for Data Reporting Provisioning Session Management

This procedure is used by an AF to request the creation/deletion of a Data Reporting Provisioning Session in order to supply data collection and reporting provisioning information.

In order to request the creation of a Data Reporting Provisioning Session, an AF shall send a Nnef_DataReportingProvisioning_Create request to the NEF using the HTTP POST method and targeting the "Data Reporting Provisioning Sessions" collection resource, with the request message body including the DataReportingProvisioningSession data structure as defined in clause 5.24.2.2.3.1.

In order to read an existing "Individual Data Reporting Provisioning Session" resource, an AF shall send a Nnef_DataReportingProvisioning_Retrieve request to the NEF using the HTTP GET method and targeting the concerned "Individual Data Reporting Provisioning Session" resource, as defined in clause 5.24.2.3.3.1.

In order to request the deletion of an existing Data Reporting Provisioning Session, an AF shall send a Nnef_DataReportingProvisioning_Delete request to the NEF using the HTTP DELETE method and targeting the concerned "Individual Data Reporting Provisioning Session" resource as defined in clause 5.24.2.3.3.3.

At the reception of the HTTP POST/GET/DELETE request from the AF, the NEF shall trigger the necessary interactions with the DCAF as specified in 3GPP TS 26.532 [60] and:

-	for an HTTP POST request, create a new "Individual Data Reporting Provisioning Session" resource and respond to the AF with an HTTP "200 OK" status code including an HTTP Location header field containing the URI of the created resource and the response body including a representation of the created "Individual Data Reporting Provisioning Session" resource within the DataReportingProvisioningSession data structure;

-	for an HTTP GET request, respond to the AF with an HTTP "200 OK" status code with the response body including the representation of the requested "Individual Data Reporting Provisioning Session" resource within the DataReportingProvisioningSession data structure; and

-	for an HTTP DELETE request, delete the corresponding "Individual Data Reporting Provisioning Session" resource and respond to the AF with an HTTP "204 No Content" status code.

#### 4.4.31.3	Procedure for Data Reporting Configuration management

This procedure is used by an AF to manage Data Reporting Configuration.

In order to request the creation of a Data Reporting Configuration, an AF shall send a Nnef_DataReportingProvisioning_CreateConfiguration request to the NEF using the HTTP POST method and targeting the "Data Reporting Configurations" collection resource, with the request message body including the DataReportingConfiguration data structure as defined in clause 5.24.2.5.3.1.

In order to read an existing Data Reporting Configuration, an AF shall send a Nnef_DataReportingProvisioning_RetrieveConfiguration request to the NEF using the HTTP GET method and targeting the concerned "Individual Data Reporting Configuration" resource. , as defined in clause 5.24.2.4.3.1.

In order to request the update of an existing Data Reporting Configuration, an AF shall send a Nnef_DataReportingProvisioning_UpdateConfiguration request to the NEF using the HTTP PUT method, targeting the concerned "Individual Data Reporting Configuration" resource with the request message body including the updated resource representation within the DataReportingConfiguration data structure as defined in clause 5.24.2.5.3.3.

In order to request the modification of an existing Data Reporting Configuration, an AF shall send a Nnef_DataReportingProvisioning_UpdateConfiguration request to the NEF using the HTTP PATCH method and targeting the concerned "Individual Data Reporting Configuration" resource with the request message body containing the DataReportingConfigurationPatch data structure, as defined in clause 5.24.2.5.3.3A.

In order to request the deletion of an existing Data Reporting Configuration, an AF shall send a Nnef_DataReportingProvisioning_DeleteConfiguration request to the NEF using the HTTP DELETE method and targeting the concerned "Individual Data Reporting Configuration" resource as defined in clause 5.24.2.5.3.4.

At the reception of the HTTP POST/GET/PUT/PATCH/DELETE requests from the AF, the NEF shall trigger the necessary interactions with the DCAF as specified in 3GPP TS 26.532 [60] and:

-	for an HTTP POST request, create a new "Individual Data Reporting Configuration" resource and respond to the AF with an HTTP "200 OK" status code including an HTTP Location header field containing the URI of the created resource and the response body including a representation of the created "Data Reporting Configuration" resource within the DataReportingConfiguration data structure;

-	for an HTTP GET request, respond to the AF with an HTTP "200 OK" status code with the response body including the representation of the requested "Individual Data Reporting Configuration " resource within the DataReportingConfiguration data structure;

-	for an HTTP PUT/PATCH request, update/modify the concerned "Individual Data Reporting Configuration" resource and respond to the AF with an HTTP "200 OK" status code with the response body including a representation of the updated/modified "Individual Data Reporting Configuration" resource within the DataReportingConfiguration data structure, or with an HTTP "204 No Content" status code; and

-	for an HTTP DELETE request, delete the corresponding "Individual Data Reporting Configuration" resource and respond to the AF with an HTTP "204 No Content" status code.

### 4.4.32	Procedures for UE ID retrieval

#### 4.4.32.1	General

The procedures described in the clauses below are used by an AF in order to carry out the following procedures:

-	request the NEF to provide a UE ID as described in clause 4.15.10 and 4.15.10A of 3GPP TS 23.502 [2]. The UE ID retrieval procedures are used by an AF to request the NEF to retrieve either:

-	the AF specific UE ID (i.e. GPSI in the form of an External Identifier); or

-	the GPSI in the form of MSISDN;

and

-	perform UE ID Mapping provisioning as defined in 3GPP TS 23.502 [2] for:

-	Ranging/SL positioning as specified in 3GPP TS 23.586 [76].

NOTE:	In this release of the specification, only the Ranging/Sidelink UE ID Mapping Information between the Application Layer ID and the GPSI with 1:1 mapping is supported.

#### 4.4.32.2	Retrieve AF specific UE ID service operation

In order to retrieve AF specific UE ID information, the AF shall send an HTTP POST request message to the NEF targeting the custom operation URI "{apiRoot}/3gpp-ueid/v1/retrieve", with the request body including the UeIdReq data structure. If the feature "PortNumber" is supported, the port number associated with the UE IP address may be also included in the UeIdReq data structure.

Upon reception of the HTTP POST request message from the AF, the NEF shall check whether the AF is authorized to perform this operation or not:

-	if the AF request for AF specific UE ID retrieval is not authorized, the NEF shall respond to the AF with a "403 Forbidden" status code with the response body including the ProblemDetails data structure containing the "cause" attribute set to the "REQUEST_NOT_AUTHORIZED" application error indicating the AF authorisation failure; or

-	if the AF's request for AF specific UE ID retrieval is authorized, then if the DNN and/or S-NSSAI information is not available in the request, the NEF shall determine the corresponding DNN and/or S-NSSAI information based on the requesting AF Identifier, and if provided, the MTC Provider Information.

Upon success, if the port number associated with the UE IP address is received and based on configuration, the NEF may recognize that the IP address received is different from the actual private UE IP address assigned by 5GC, i.e. the UE is behind a NAT in UPF. If so, the NEF shall discover the UPF implementing NAT functionality for the UE (public) IP address via Nnrf_NFDiscovery service as defined in 3GPP TS 29.510 [57] and then the NEF shall request UE's (private) IP address and IP domain (if the UE IPv4 address is provided) from the UPF by invoking the Nupf_GetUEPrivateIPaddrAndIdentifiers_Get service operation as defined in 3GPP TS 29.564 [61]. If the UPF has the SUPI of the UE, the UPF may directly return the SUPI to the NEF then the NEF shall skip the interaction with the BSF for SUPI retrieval. Otherwise the NEF shall then interact with the BSF using the UE address and IP domain (if the UE IPv4 address is provided), DNN and/or S-NSSAI to retrieve the session binding information of the UE by invoking the Nbsf_Management_Discovery service operation, as described in 3GPP TS 29.521 [9].

If the NEF receives an error response from the UPF or BSF, the NEF shall respond to the AF with a proper error status code. If the NEF received from the BSF an error response including a "ProblemDetails" data structure with the "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error. If no SUPI matching the provided UE information is returned by the BSF, the NEF shall respond to the AF with a "404 Not Found" status code with the response body including a ProblemDetails data structure containing the "cause" attribute set to the "UE_NOT_FOUND" application error to indicate that the requested UE address is not found.

Upon success and a SUPI is returned from either the BSF or UPF, the NEF shall then interact with UDM to retrieve the AF specific UE Identifier using the received SUPI and at least one of the Application Port ID, MTC Provider Information or AF Identifier information by invoking Nudm_SDM_Get service, as described in clause 5.2.2.2 of 3GPP TS 29.503 [17]. Upon success, the UDM responds to the NEF with the AF specific UE Identifier represented as an External Identifier for the UE which is uniquely associated with the Application Port ID, MTC provider Information and/or AF Identifier. The NEF shall then respond to the AF with the received information, i.e. the AF specific UE Identifier represented as an External Identifier that was received from the UDM.

If the NEF receives an error response from the UDM, the NEF shall respond to the AF with a proper error status code. If the NEF received from the UDM an error response including a "ProblemDetails" data structure with the "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error. If the UDM indicates that the requested UE Identifier is not available in the subscription data, the NEF shall respond to the AF with a "404 Not Found" error status code with the response body including a ProblemDetails data structure containing the "cause" attribute set to the "UE_ID_NOT_AVAILABLE" application error to indicate that the AF specific UE ID is not available.

NOTE:	The combination of IP address and Port Number can be used by 5GC to derive the UE private IP address assigned by 5GC if the UE is behind a NAT deployed with NAPT within UPF.

#### 4.4.32.3	Retrieve UE ID in the form of MSISDN

If the operator policy and local regulation allows the UE ID exposure in the GPSI format of MSISDN to an authenticated and authorized AF, in order to retrieve the UE ID in the form of MSISDN, the AF shall send an HTTP POST request message to the NEF targeting the custom operation URI "{apiRoot}/3gpp-ueid/v1/get-msisdn", with the request body including the MsisdnReq data structure.

Upon reception of the HTTP POST request message from the AF, the NEF shall check whether the AF is authorized to perform this operation or not:

-	if the AF's request for MSISDN retrieval is not authorized, the NEF shall respond to the AF with a "403 Forbidden" status code with the response body including the ProblemDetails data structure containing the "cause" attribute set to the "REQUEST_NOT_AUTHORIZED" application error indicating the AF authorization failure; or

-	if the AF's request for MSISDN retrieval is authorized, then if the DNN and/or S-NSSAI information is not available in the request, the NEF shall determine the corresponding DNN and/or S-NSSAI information based on the requesting AF Identifier, and if provided, the MTC Provider Information.

Upon success, if the port number associated with the UE IP address is received and based on configuration, the NEF may recognize that the IP address received is different from the actual private UE IP address assigned by 5GC, i.e. the UE is behind a NAT in UPF. If so, the NEF shall discover the UPF implementing NAT functionality for the UE (public) IP address via Nnrf_NFDiscovery service as defined in 3GPP TS 29.510 [57] and then the NEF shall request UE's (private) IP address and IP domain (if the UE IPv4 address is provided) from the UPF by invoking the Nupf_GetUEPrivateIPaddrAndIdentifiers_Get service operation as defined in 3GPP TS 29.564 [61]. If the UPF has the SUPI of the UE, the UPF may directly return the SUPI to the NEF then the NEF shall skip the interaction with the BSF for SUPI retrieval. Otherwise, the NEF shall then interact with the BSF using the UE address and IP domain (if the UE IPv4 address is provided), DNN and/or S-NSSAI to retrieve the session binding information of the UE by invoking the Nbsf_Management_Discovery service operation, as described in 3GPP TS 29.521 [9].

If the NEF receives an error response from the UPF or BSF, the NEF shall respond to the AF with a proper error status code. If the NEF received from the BSF an error response including a "ProblemDetails" data structure with the "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error. If no SUPI matching the provided UE information is returned by the BSF, the NEF shall respond to the AF with a "404 Not Found" status code with the response body including a ProblemDetails data structure containing the "cause" attribute set to the "UE_NOT_FOUND" application error to indicate that the requested UE address is not found.

Upon success and a SUPI is returned by either the BSF or UPF, the NEF shall proceed as follows:

-	If local regulation and operator policy requires user consent for the retrieval of a UE ID in the form of MSISDN, the user consent for the targeted UE shall be checked via one of the following methods depending on whether CAPIF is used (i.e., using RNAA use case) or not:

-	If CAPIF is not used, then the user consent is checked by the UDM at the reception of the Nudm_SDM_Get request from the NEF as specified in clause 6.1.3.12 of 3GPP TS 29.503 [17] to check for user consent by setting the "uc-purpose" query parameter to "MSISDN_EXPOSURE".

-	If CAPIF is used, then the RNAA procedure defined in clause 5.6.2.3.2 of 3GPP TS 29.222 [12] shall be used.

-	The NEF shall interact with the UDM to retrieve the UE ID in the format of MSISDN using the received SUPI and MTC Provider Information or AF Identifier information by invoking the Nudm_SDM_Get service operation, as defined in clause 5.2.2.2 of 3GPP TS 29.503 [17]. Upon success (with user consent granted in place, unless user consent is not required), the UDM responds to the NEF with the requested MSISDN associated with the SUPI. The NEF shall then respond to the AF with the MSISDN that was received from the UDM.

If the user consent is required and not granted for the SUPI received from either the BSF or UPF, the NEF shall reject the request and respond to the AF with an HTTP "403 Forbidden" status code with the response body including a ProblemDetails data structure including the "USER_CONSENT_NOT_GRANTED" application error within the "cause" attribute.

NOTE:	In the case of RNAA-based consent check this error will occur before the invocation of the Nudm_SDM_Get service operation, which means that the interaction with the UDM will not take place at all.

If the NEF receives an error response from the UDM, the NEF shall respond to the AF with a proper error status code. If the NEF received from the UDM an error response including a "ProblemDetails" data structure with the "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error. If the UDM indicates that the requested UE Identifier is not available in the subscription data, the NEF shall respond to the AF with a "404 Not Found" error status code with the response body including a ProblemDetails data structure containing the "cause" attribute set to the "UE_ID_NOT_AVAILABLE" application error to indicate that the AF specific UE ID is not available.

NOTE:	The combination of IP address and Port Number can be used by 5GC to derive the UE private IP address assigned by 5GC if the UE is behind a NAT deployed with NAPT within UPF.

#### 4.4.32.4	UE ID Mapping Information Provisioning

This procedure is used by an AF to request the creation/update/deletion of UE ID Mapping Information provisioning.

In order to request the creation of a UE ID Mapping Information Provisioning, an AF shall trigger the UEId API by sending an HTTP POST request to the NEF targeting the UE ID Mapping Information Provisionings" collection resource, with the request body including the UeIdMappingInfo data structure.

The NEF shall then check whether the AF is authorized to perform this operation or not. If the AF is authorized, the NEF shall trigger the Nudr_DataRepository service API of the UDR (for Application Data) to store the received UE ID Mapping Information as specified in 3GPP TS 29.519 [23].

Upon success and reception of a successful response from the UDR as defined in 3GPP TS 29.519 [23], the NEF shall respond to the AF with an HTTP "201 Created" status code including a "Location" header field containing the URI of the created resource, and the response body containing the UeIdMappingInfo data structure containing a representation of the created "Individual UE ID Mapping Information Provisioning" resource.

On failure or if the NEF receives an error response from the UDR, the NEF shall take proper error handling actions, as specified in clause 5.25.7, and respond to the AF with an appropriate error status code.

In order to request the update of an existing "Individual UE ID Mapping Information Provisioning" resource, an AF shall trigger the UEId API by sending to the NEF either:

-	an HTTP PUT request targeting the concerned "Individual UE ID Mapping Information Provisioning" resource with the request body including the UeIdMappingInfo data structure; or

-	an HTTP PATCH request targeting the concerned "Individual UE ID Mapping Information Provisioning" resource with the request body including the UeIdMappingInfoPatch data structure.

After authorizing the request, the NEF shall interact with the UDR via the Nudr_DataRepository service API of the UDR (for Application Data) to store the received updated UE ID Mapping Information as specified in 3GPP TS 29.519 [23].

Upon success and reception of a successful response from the UDR as defined in 3GPP TS 29.519 [23], the NEF shall respond to the AF with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual UE ID Mapping Information Provisioning" resource within the UeIdMappingInfo data structure; or

-	an HTTP "204 No Content" status code.

On failure or if the NEF receives an error response from the UDR, the NEF shall take proper error handling actions, as specified in clause 5.25.7, and respond to the AF with an appropriate error status code.

In order to request the deletion of an existing "Individual UE ID Mapping Information Provisioning" resource, an AF shall trigger the UEId API by sending an HTTP DELETE request targeting the corresponding "Individual UE ID Mapping Information Provisioning" resource to the NEF. After authorizing the request, the NEF shall interact with the UDR via the Nudr_DataRepository service API of the UDR (for Application Data) to to request to update accordingly the UE ID Mapping Information as specified in 3GPP TS 29.519 [23].

Upon success and reception of a successful response from the UDR as defined in 3GPP TS 29.519 [23], the NEF shall respond to the AF with an HTTP "204 No Content" status code.

On failure or if the NEF receives an error response from the UDR, the NEF shall take proper error handling actions, as specified in clause 5.25.7, and respond to the AF with an appropriate error status code.

### 4.4.33	Procedures for Media Streaming Event Exposure

#### 4.4.33.1	General

The procedures described in the clauses below are used by an external/untrusted event consumer AF to subscribe, update and delete a subscription to Media Streaming Exposure event(s) reporting via the NEF, also for a data collection AF to notify the observed Media Streaming event(s) which has been subscribed, as defined in 3GPP TS 26.512 [67].

#### 4.4.33.2	Procedure for Media Streaming Event Exposure Subscription Creation

This procedure is used by an event consumer AF to subscribe to at least one Media Streaming Exposure event at the NEF.

In order to subscribe to at least one Media Streaming Exposure event, an event consumer AF shall send a Nnef_MSEventExposure_Subscribe request message to the NEF using the HTTP POST method and targeting the "Media Streaming Event Exposure Subscriptions" collection resource, with the request message body including the AfEventExposureSubsc data structure, as specified in clause 5.28.2.2.3.2.

The NEF shall then check whether the event consumer AF is authorized to perform this operation or not. If the event consumer AF is authorized, the NEF shall then trigger the Naf_EventExposure API of the data collection AF to request the creation of the corresponding Application Event Subscriptions at the AF, as specified in 3GPP TS 29.517 [58].

Upon reception of a successful response from the data collection AF, as defined in 3GPP TS 29.517 [58], the NEF shall return a Nnef_MSEventExposure_Subscribe response message with an HTTP "201 Created" status code including a "Location" header field that shall contain the URI of the created resource, i.e. "{apiRoot}/3gpp-ms-event-exposure/v1/subscriptions/{subscriptionId}", and the response body containing a representation of the created "Individual Media Streaming Event Exposure Subscription" resource within the AfEventExposureSubsc data structure, as specified in clause 5.28.2.2.3.2.

On failure or if the NEF receives an error response from the data collection AF, the NEF shall take proper error handling actions, as specified in clause 5.28.7, and respond to the event consumer AF with an appropriate error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

#### 4.4.33.3	Procedure for Media Streaming Event Exposure Subscription Update

This procedure is used by an event consumer AF to update an existing Media Streaming Event Exposure Subscription at the NEF.

In order to update an existing Media Streaming Event Exposure Subscription, the event consumer AF shall send a Nnef_MSEventExposure_Subscribe request message to the NEF using the HTTP PUT method and targeting the "Individual Media Streaming Event Exposure Subscription" resource, with the request message body including the AfEventExposureSubsc data structure, as specified in clause 5.28.2.3.3.2.

The NEF shall then check whether the event consumer AF is authorized to perform this operation or not. If the event consumer AF is authorized, the NEF shall then trigger the Naf_EventExposure API of the data collection AF to request the update of the corresponding Individual Application Event Subscription at the AF, as specified in 3GPP TS 29.517 [58].

Upon reception of a successful response from the data collection AF, as defined in 3GPP TS 29.517 [58], the NEF shall return a Nnef_MSEventExposure_Subscribe response message with an HTTP "200 OK" status code with the AfEventExposureSubsc data structure or “204 No Content” status code, as specified in clause 5.28.2.3.3.2.

On failure or if the NEF receives an error response from the data collection AF, the NEF shall take proper error handling actions, as specified in clause 5.28.7, and respond to the event consumer AF with an appropriate error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

#### 4.4.33.4	Procedure for Media Streaming Event Exposure Unsubscription

This procedure is used by an event consumer AF to request the deletion of an existing Media Streaming Event Exposure Subscription at the NEF.

In order to request the deletion of an existing Media Streaming Event Exposure Subscription, an event consumer AF shall send a Nnef_MSEventExposure_Unsubscribe request message using the HTTP DELETE method and targeting the URI of the concerned "Individual Media Streaming Event Exposure Subscription" resource.

The NEF shall then check whether the event consumer AF is authorized to perform this operation or not. If the AF is authorized, the NEF shall then trigger the Naf_EventExposure service API of the data collection AF to request the deletion of the corresponding Application Event Subscription at the AF, as specified in 3GPP TS 29.517 [58].

Upon reception of a successful response from the data collection AF, as defined in 3GPP TS 29.517 [58], the NEF shall return a Nnef_MSEventExposure_Unsubscribe response message with an HTTP "204 No Content" status code.

On failure or if the NEF receives an error code from the data collection AF, the NEF shall take proper error handling actions, as specified in clause 5.28.7, and respond to the event consumer AF with an appropriate error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

#### 4.4.33.5	Procedure for Media Streaming Event Exposure Notification

This procedure is used by the NEF to send a Media Streaming Event Exposure notification to a previously subscribed event consumer AF.

In order to send a Media Streaming Event Exposure notification, the NEF shall send a Nnef_MSEventExposure_Notify request message to the AF using the HTTP POST method and targeting the notification URI provided during the creation/update of the corresponding subscription, with the request body including the AfEventExposureNotif data structure as specified in clause 5.28.4.2.3.1.

Upon success, the event consumer AF shall send a Nnef_MSEventExposure_Notify response message with an HTTP "204 No Content" status code.

On failure, the event consumer AF shall take proper error handling actions, as specified in clause 5.28.7, and respond to the NEF with an appropriate error status code.

### 4.4.34	Procedures for DNAI Mapping

#### 4.4.34.1	General

The procedures are used by AF to create or delete subscription(s) of DNAI Mapping information to NEF and also by NEF to notify NF consumer about the update of the DNAI Mapping information as defined in clause 5.2.6.34 of 3GPP TS 23.502 [2]

#### 4.4.34.2	Creation of a new subscription for DNAI Mapping

In order to create a new subscription for DNAI Mapping for a given AF, the AF shall initiate an HTTP POST request to the NEF for the "DNAI Mapping Subscriptions" resource. The HTTP POST request message body shall include the DnaiMapSub data structure that shall include:

-	a notification URI within the "notifUri" attribute and a notification correlation identifier within the "notifCorrId" attribute;

-	either the FQDN(s) of the EAS(s) in the Local part of the DN as the "fqdns" attribute or the EAS(s) IP Address(es) in the Local part of the DN within the "easIpAddrs" attribute;

and may include:

-	a DNN as "dnn" attribute;

-	an S-NSSAI as "snssai" attribute;

-	event reporting requirements as "eventReq" attribute.

Upon receipt of the corresponding HTTP POST message, if the AF is authorized by the NEF to obtain the DNAI mapping information, the NEF may interact with the UDR by invoking the Nudr_DataRepository service as described in 3GPP TS 29.504 [20]. If the NEF receives an error response from the UDR, the NEF shall not create the resource and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

On successful DNAI Mapping subscription creation, the NEF shall return an HTTP POST response with an HTTP "201 Created" status code to the AF, including a "Location" header containing the URI of the created "Individual DNAI Mapping Subscription" resource and the response body containing a representation of the created resource within the DnaiMapSub data structure. If one-time reporting is not requested and the subscription can be expired based on the operator's policy, an expiry time shall be included within the "monDur" attribute of the ReportingInformation in the DnaiMapSub data structure. If immediate reporting is requested, the currently available DNAI Mapping information shall be included within the "immReports" attribute of the DnaiMapSub data structure.

On failure, the NEF shall take proper error handling actions, as specified in clause 5.30.7, and respond to the AF with an appropriate error status code.

#### 4.4.34.3	Deletion of an existing individual DNAI Mapping subscription

In order to delete an existing DNAI Mapping subscription, the AF shall send an HTTP DELETE request message and targeting the correponding "Individual DNAI Mapping Subscription" resource.

On successful deletion of the subscription, the NEF shall return an HTTP DELETE response with an HTTP "204 No Content" status code.

On failure, the NEF shall take proper error handling actions, as specified in clause 5.30.7, and respond to the AF with an appropriate error status code.

#### 4.4.34.4	Notification for updated DNAI Mapping information

This procedure is used by the NEF to send DNAI Mapping information update notifications to a previously subscribed AF.

When the NEF receives the notification of the updated DNAI Mapping information from the UDR as described in 3GPP TS 29.504 [20], the NEF shall provide a notification by sending an HTTP POST request message to the AF with the request body including the DnaiMapUpdateNotif data structure and targeting the notification URI provided by the AF during the corresponding DNAI mapping subscription.

Upon reception of this notification request, the AF shall acknowledge its successful reception by sending a HTTP POST response message with an HTTP "204 No Content" status code.

On failure, the AF shall take proper error handling actions, as specified in clause 5.30.7, and respond to the NEF with an appropriate error status code.

### 4.4.35	Procedures for negotiation of planned data transfer with QoS requirements

These procedures are used by an AF to perform negotiation of a viable time window for the planned application data transfer via the support of the NEF.

In order to create a resource for the PDTQ policy, the AF shall send an HTTP POST message to the NEF for the "PDTQ Policy Subscriptions" resource to negotiate the PDTQ policy. The body of the HTTP POST message shall include the ASP Identifier, Number of UEs, the list of Desired Time Windows, QoS reference or individual QoS parameters and optionally network area information, application identifier and Alternative Service Requirements. The AF may also include a notification URI and a request to enable the PDTQ warning notification during the planned data transfer policy negotiation.

After receiving the HTTP POST message, if the AF is authorized, the NEF may map the information received from the AF (e.g. QoS reference and network area information) based on local policies and negotiate the PDTQ policy with the PCF as described in 3GPP TS 29.543 [68]. After receiving the response including the determined PDTQ policies from the PCF, the NEF shall create an "Individual PDTQ Policy Subscription" resource which represents the PDTQ Subscription, addressed by an URI that contains the AF identifier and a NEF-created "Individual PDTQ Policy Subscription" resource identifier, and shall respond to the AF with a 201 Created message, including a Location header field containing the URI for the created resource and a message body, which shall also include a PDTQ Reference ID and a set of PDTQ policies. The AF shall use the URI received in the Location header in subsequent requests to the NEF to refer to this PDTQ Policy Subscription. If the NEF receives a response with an error code from the PCF, the NEF shall not create the resource and shall respond to the AF with a corresponding failure code as described in clause 5.31.5.

If more than one policy is included in the HTTP response, the AF shall send an HTTP PATCH message to inform the NEF for the "Individual PDTQ Policy Subscription" resource of the PDTQ policy selected by the AF. The AF may also send an HTTP PATCH to request to disable/enable the PDTQ warning notification at any moment.

After receiving the HTTP PATCH message, the NEF shall send an HTTP response to the AF with a "200 OK" status code and shall include the Pdtq data type in the response body, or with a "204 No Content" status code, then the NEF shall interact with the PCF as defined in 3GPP TS 29.543 [68]. If the NEF identifies any error (e.g. selected policy is not within the set of PDTQ policies), the NEF shall not update the resource and shall respond to the AF with a corresponding failure code as described in clause 5.31.5.

When the NEF receives the PDTQ warning notification from the PCF as defined in clause 5.2.2.4.2 of 3GPP TS 29.543 [68] and the "warnNotifEnabled" attribute was set to true, the NEF shall send an HTTP POST message including the Notification data structure to the AF identified by the notification destination URI received during the planned data transfer policy negotiation. The AF shall respond with an HTTP response to confirm the received notification and even if only one candidate PDTQ policy was provided within the PDTQ warning notification, the AF shall send the HTTP PATCH message with the "selectedPolicy" attribute to the NEF. The "selectedPolicy" attribute shall contain the identifier of the selected candidate PDTQ policy or, otherwise value "0" indicating no PDTQ policy is accepted by the AF. The AF may also request to disable/enable the PDTQ warning notification by including the "warnNotifEnabled" attribute in the HTTP PATCH message.

The AF may also send an HTTP DELETE message to the NEF for the "Individual PDTQ Policy Subscription" resource requesting to remove an individual resource identified by the URI received in the response to the request that has created resource a URI. After receiving such request, the NEF shall delete the resource and send an HTTP response to the AF with a corresponding status code. The NEF shall request to disable the PDTQ warning notification by including the "warnNotifEnabled" attribute in the HTTP PATCH message towards PCF as described in 3GPP TS 29.543 [68].

NOTE:	The NEF can also remove the resource when the last window end time in PDTQ policies expires.

### 4.4.36	Procedures for Member UE Selection Assistance

#### 4.4.36.1	General

The procedures described in the clauses below are used by an AF to subscribe, update and delete a subscription to Member UE selection assistance via the NEF, also for the NEF to notify the AF about the member UE selection assistance which has been subscribed.

#### 4.4.36.2	Procedure for Member UE Selection Assistance Subscription Creation

In order to subscribe to receive the Member UE selection assistance, the AF shall send an HTTP POST request message to the NEF targeting the "Member UE Selection Assistance Subscriptions" collection resource, with the request message body including the MemUeSelectAssistSubsc data structure, as specified in clause 5.32.5.2.2.

The NEF shall then check whether the AF is authorized to perform this operation or not. If the AF is authorized and based on the Member UE filtering criteria(s) provided by the AF, the NEF shall then interact with the corresponding different 5GC NFs via the services offered by the 5GC NFs.

Upon reception of a successful response from the 5GC NF, the NEF shall respond to the AF with a "201 Created" status code including a "Location" header field that shall contain the URI of the created resource, i.e. "{apiRoot}/3gpp-musa/<apiVersion>/{afId}/subscriptions/{subscriptionId}", and the response body containing a representation of the created "Individual Member UE Selection Assistance Subscription" resource within the MemUeSelectAssistSubsc data structure.

On failure or if the NEF receives an error response from the 5GC NF, the NEF shall take proper error handling actions, as specified in clause 5.32.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

#### 4.4.36.3	Procedure for Member UE Selection Assistance Subscription Update

In order to update an existing Individual Member UE Selection Assistance Subscription, the AF shall send an HTTP PUT or PATCH request message to the NEF targeting the "Individual Member UE Selection Assistance Subscription" resource, with the request message body including respectively the MemUeSelectAssistSubsc or MemUeSelectAssistSubscPatch data structure as specified in clauses 5.32.5.2.2 and 5.32.5.2.15.

The NEF shall then check whether the AF is authorized to perform this operation or not. If the AF is authorized and based on the Member UE filtering criteria(s) provided by the AF, the NEF shall then interact with the corresponding different 5GC NFs via the services offered by the 5GC NFs.

Upon reception of a successful response from the 5GC NF, the NEF shall respond to the AF with a "200 OK" status code with the MemUeSelectAssistSubsc data structure or "204 No Content" status code.

On failure or if the NEF receives an error response from the 5GC NF, the NEF shall take proper error handling actions, as specified in clause 5.32.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

#### 4.4.36.4	Procedure for Member UE Selection Assistance Subscription Unsubscription

In order to request the deletion of an existing Individual Member UE Selection Assistance Subscription, the AF shall send an HTTP DELETE request message targeting the URI of the concerned "Individual Member UE Selection Assistance Subscription" resource.

The NEF shall then check whether the AF is authorized to perform this operation or not. If the AF is authorized, the the NEF shall then unsubscribe from the different 5GC NFs to stop collecting the UE list.

Upon reception of a successful response from the 5GC NF, the NEF shall respond to the AF with a HTTP "204 No Content" status code.

On failure or if the NEF receives an error code from the 5GC AF, the NEF shall take proper error handling actions, as specified in clause 5.32.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

#### 4.4.36.5	Procedure for Member UE Selection Assistance Notification

In order to send a Member UE Selection Assistance notification, the NEF shall send the HTTP POST request message to the AF using and targeting the notification URI provided during the creation/update of the corresponding subscription, with the request body including the MemUeSeletAssistNotif data structure as specified in clause 5.32.5.2.3.

The NEF consolidates all the information collected from other 5GC NFs to derive one or more list(s) of candidate UEs based on the Member UE filtering criterias which were provided by the AF.

Upon success, the AF shall send a HTTP "204 No Content" status code.

On failure, the AF shall take proper error handling actions, as specified in clause 5.32.7, and respond to the NEF with an appropriate error status code.

### 4.4.37	Procedures for Group Parameters Provisioning

#### 4.4.37.1	General

The procedures described in the clauses below are used by an AF to interact with the 5GC for Group Parameters Provisioning, in order to carry out the following procedures:

-	DNN and S-NSSAI specific Group Parameters Provisioning procedures (see clause 4.15.6.3e of 3GPP TS 23.502 [2]).

#### 4.4.37.2	Procedures for DNN and S-NSSAI specific Group Parameters Provisioning

This procedure is used by an AF to request the creation/update/deletion of a DNN and S-NSSAI specific Group Parameters Provisioning.

In order to request the creation of a DNN and S-NSSAI specific Group Parameters Provisioning:

-	the AF shall trigger the GroupParametersProvisioning API by sending an HTTP POST request to the NEF targeting the "Group Parameters Provisionings" collection resource, with the request body including the GrpPpData data structure that shall contain:

-	within the "afId" attribute, the identifier of the AF that is sending the request;

-	within the "dnnSnssaiGrpData" attribute, the DNN and S-NSSAI specific Group parameters data that is to be provisioned; and

-	within the "suppFeat" attribute, the features supported by the AF, if applicable (i.e., feature negociation needs to take place);

-	the NEF shall then check whether the AF is authorized to perform this operation or not;

-	if the AF is authorized:

-	if the LADN Service Area is provided by the AF within the "dnnSnssaiGrpData" attribute (via the "ladnServArea" attribute) and in the form of a list of geographic area(s) or a list of civic address(es), the NEF shall translate this information into a list of TAI(s); and

-	the NEF shall then trigger the Nudm_ParameterProvision service API of the UDM to request the provisioning of the received DNN and S-NSSAI Group specific parameters data as specified in 3GPP TS 29.503 [17];

and

-	upon reception of a successful response from the UDM as defined in 3GPP TS 29.503 [17] and successful processing of the request, the NEF shall respond to the AF with an HTTP "201 Created" status code including an HTTP "Location" header field containing the URI of the created resource, and the response body including a representation of the created "Individual Group Parameters Provisioning" resource within the GrpPpData data structure.

In order to request the update of an existing DNN and S-NSSAI specific Group Parameters Provisioning:

-	the AF shall trigger the GroupParametersProvisioning API by sending to the NEF either:

-	an HTTP PUT request targeting the corresponding "Individual Group Parameters Provisioning" resource with the request body including the GrpPpData data structure; or

-	an HTTP PATCH request targeting the corresponding "Individual Group Parameters Provisioning" resource with the request body including the GrpPpDataPatch data structure;

-	after authorizing the request, the NEF shall interact with the UDM via the the Nudm_ParameterProvision service API to request the provisioning of the received updated DNN and S-NSSAI specific Group parameters data; and

-	upon reception of a successful response from the UDM as defined in 3GPP TS 29.503 [17] and successful processing of the request, the NEF shall respond to the AF with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual Group Parameters Provisioning" resource within the GrpPpData data structure; or

-	an HTTP "204 No Content" status code.

In order to request the deletion of an existing DNN and S-NSSAI specific Group Parameters Provisioning:

-	the AF shall trigger the GroupParametersProvisioning API by sending an HTTP DELETE request targeting the corresponding "Individual Group Parameters Provisioning" resource to the NEF;

-	after authorizing the request, the NEF shall interact with the UDM via the the Nudm_ParameterProvision service API as defined in 3GPP TS 29.503 [17] to request the deletion of the correponding DNN and S-NSSAI specific Group parameters data; and

-	upon reception of a successful response from the UDM as defined in 3GPP TS 29.503 [17] and successful processing of the request, the NEF shall respond to the AF with an HTTP "204 No Content" status code.

On failure or if the NEF receives an error response from the UDM, the NEF shall take proper error handling actions, as specified in clause 5.33.7, and respond to the AF with an appropriate error status code.

### 4.4.38	Procedures for Network Slice Parameters Provisioning

#### 4.4.38.1	General

The procedures described in the clauses below are used by an AF to interact with the 5GC for Network Slice Parameters Provisioning, in order to carry out one or more of the following procedures:

-	Network Slice Usage Control parameters provisioning procedures (see clause 4.15.6.3g of 3GPP TS 23.502 [2]).

In order to request the creation of a Network Slice Parameters Provisioning:

-	the AF shall trigger the SliceParamProvision API by sending an HTTP POST request to the NEF targeting the "Slice Parameters Provisionings" collection resource, with the request body including the SlicePpData data structure that shall include:

-	within the "afId" attribute, the identifier of the AF that is sending the request; and

-	within the "suppFeat" attribute, the features supported by the AF, if applicable (i.e., feature negociation needs to take place);

-	the NEF shall then check whether the AF is authorized to perform this operation or not;

-	if the AF is authorized, the NEF shall trigger the Nudm_ParameterProvision service API of the UDM to request the provisioning of the received Network Slice Parameters Provisioning data as specified in 3GPP TS 29.503 [17]; and

-	upon reception of a successful response from the UDM as defined in 3GPP TS 29.503 [17] and successful processing of the request, the NEF shall respond to the AF with an HTTP "201 Created" status code including an HTTP "Location" header field containing the URI of the created resource, and the response body including a representation of the created "Individual Slice Parameters Provisioning" resource within the SlicePpData data structure.

In order to request the update of an existing "Individual Slice Parameters Provisioning" resource:

-	the AF shall trigger the SliceParamProvision API by sending to the NEF either:

-	an HTTP PUT request targeting the corresponding "Individual Slice Parameters Provisioning" resource with the request body including the updated representation of the resource within the SlicePpData data structure; or

-	an HTTP PATCH request targeting the corresponding "Individual Slice Parameters Provisioning" resource with the request body including the requested modifications to the resource within the SlicePpDataPatch data structure;

-	after authorizing the request, the NEF shall interact with the UDM via the the Nudm_ParameterProvision service API to request the provisioning of the received updated Network Slice parameters provisioning data; and

-	upon reception of a successful response from the UDM as defined in 3GPP TS 29.503 [17] and successful processing of the request, the NEF shall respond to the AF with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual Slice Parameters Provisioning" resource within the SlicePpData data structure; or

-	an HTTP "204 No Content" status code.

In order to request the deletion of an existing "Individual Slice Parameters Provisioning" resource:

-	the AF shall trigger the SliceParamProvision API by sending an HTTP DELETE request targeting the corresponding "Individual Slice Parameters Provisioning" resource to the NEF;

-	after authorizing the request, the NEF shall interact with the UDM via the the Nudm_ParameterProvision service API as defined in 3GPP TS 29.503 [17] to request the deletion of the correponding Network Slice parameters provisioning data; and

-	upon reception of a successful response from the UDM as defined in 3GPP TS 29.503 [17] and successful processing of the request, the NEF shall respond to the AF with an HTTP "204 No Content" status code.

On failure or if the NEF receives an error response from the UDM, the NEF shall take proper error handling actions, as specified in clause 5.34.7, and respond to the AF with an appropriate error status code.

#### 4.4.38.2	Procedures for Network Slice Usage Control Parameters Provisioning

This procedure is used by an AF to request Network Slice Usage Control parameters provisioning by reusing the procedures defined in clause 4.4.38.1 with the following differences:

-	during the creation of the Network Slice Parameters Provisioning or update of the Network Slice Parameters Provisioning using HTTP PUT, the SlicePpData data structure shall include:

-	within the "sliceUsgCtrlData" attribute, the Network Slice Usage Control parameters data that is to be provisioned;

and

-	during the modification of the Network Slice Parameters Provisioning using HTTP PATCH, the SlicePpDataPatch data structure shall include:

-	within the "sliceUsgCtrlData" attribute, the requested modifications to the Network Slice Usage Control parameters data.

### 4.4.39	Procedures for UE Address Retrieval

#### 4.4.39.1	General

The procedures described in the clauses below are used by an AF to request the NEF to provide UE Address(es).

#### 4.4.39.2	Procedures for UE Address Retrieval

This procedure is used by an AF to retrieve UE Address(es).

In order to retrieve the UE Address(es):

-	an AF shall invoke the UEAddress API by sending an HTTP POST request to the NEF targeting the custom operation URI "{apiRoot}/3gpp-ue-address/v1/retrieve", with the request body including the UeAddressReq data structure that shall contain:

a)	within the "afId" attribute, the identifier of the AF that is sending the request;

b)	within the "gpsi" attribute, the GPSI of the UE; and

c)	within the "suppFeat" attribute, the features supported by the AF, if applicable (i.e., feature negotiation needs to take place); and

Upon reception of the HTTP POST request message from the AF, the NEF shall check whether the AF is authorized to perform this operation or not:

-	if the AF's request for UE Address retrieval is authorized, the NEF shall:

a)	determine the corresponding DNN(s) and/or S-NSSAI(s) information based on the local configuration for the requesting AF Identifier;

b)	invoke the Nudm_UECM_Get service operation to find the SMF serving the PDU session(s) for the GPSI, DNN, S-NSSAI including type of requested information set to SMF Registration Info and the S-NSSAI and DNN, as defined in clause 5.3.2.5.7 of TS 29.503 [17];

c)	invoke the Nsmf_EventExposure_Subscribe service operation to the identified SMF(s), including the identified PDU Session ID to find the UE IPv4 address and/or IPv6 prefix, as defined in clause 4.2.2 and clause 4.2.3 of TS 29.508 [26];

-	upon reception of a successful response from the SMF as defined in 3GPP TS 29.508 [26] and successful processing of the request, the NEF shall respond to the AF with an HTTP "200 OK" status code and the response body including the UE Address information within the UeAddressInfo data structure.

On failure or if the NEF receives an error code from the UDM or SMF, the NEF shall take proper error handling actions, as specified in clause 5.35.7, and respond to the AF with an appropriate error status code.

### 4.4.40	Procedures for ECS Address Configuration Information provisioning in roaming

#### 4.4.40.1	General

The procedures are used by the AF to provide, create, update or delete ECS Address Configuration Information to the V-NEF, and for the V-NEF to authorize the AF provisioned ECS Address Configuration Information to be stored in the V-UDR.

The following procedures support:

-	Create/Update/Delete the AF provisioned ECS Address Configuration Information.

#### 4.4.40.2	Creation of new ECS Address Configuration Information Set

In order to create a new "Individual ECS Address Configuration Information Set" resource for a given AF, the AF shall initiate an HTTP POST request to the V-NEF for the "ECS Address Configuration Information Sets" resource. The HTTP POST request message body shall include the EcsAddressInfo data structure that shall include:

-	the ECS Server Address information within the "ecsServerAddr" attribute;

and may include:

-	the spatial validity condition within the "spatialValidityCond" attribute;

-	the DNN within the "dnn" attribute;

-	the S-NSSAI within the "snssai" attribute;

-	the supported ECS authentication methods within the "ecsAuthMethods" attribute; and

-	the list of PLMNs and associated ECSPs for which EDN configuration information can be provided by the ECS, within the "supportedPlmns" attribute.

NOTE 1:	The V-NEF can derive HPLMN DNN and S-NSSAI information from the AF identifier, if the "dnn" and "snssai" attributes are not included in the EcsAddressInfo data structure.

NOTE 2:	The provided information applies to any UE.

Upon receipt of the corresponding HTTP POST message, the V-NEF authorizes the request and if the AF was authorized by the V-NEF to provide the ECS Address Configuration Information, the V-NEF may interact with the V-UDR to create the associated ECS Address Roaming Information by using the Nudr_DataRepository service as defined in 3GPP TS 29.519 [23]. If the request is accepted by the V-UDR and the V-UDR informs the V-NEF with a successful response or if no interaction with the UDR takes place and the V-NEF successfully handles the information locally, the V-NEF shall create a new "Individual ECS Address Configuration Information Set" resource and send an HTTP "201 Created" response with the EcsAddressInfo data structure including the contents of the created "Individual ECS Address Configuration Information Set" resource in the response body and a Location header field containing the URI of the created "Individual ECS Address Configuration Information Set" resource. If the V-NEF receives an error response from the V-UDR, the V-NEF shall not create the resource and shall respond to the AF with a proper error status code. If the V-NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the V-NEF shall relay this error response to the AF with a corresponding application error, if applicable.

#### 4.4.40.3	Modification of existing ECS Address Configuration Information Set

In order to modify an existing "Individual ECS Address Configuration Information Set" resource, the AF shall initiate an HTTP PUT or PATCH request to the "Individual ECS Address Configuration Information Set" resource. In the case of HTTP PUT, the request body shall include the EcsAddressInfo data structure, which shall include the same contents as described in clause 4.4.30.2. In the case of HTTP PATCH, the request body shall include the EcsAddressInfo data structure with the contents as defined in clause 5.36.5.3.4.

Upon receipt of the corresponding HTTP PUT  (or PATCH) request message,the V-NEF authorizes the request and if the AF was authorized by the V-NEF to provide the ECS Address Configuration Information, the V-NEF may interact with the V-UDR to update the associated ECS Address Roaming Data by using the Nudr_DataRepository service as defined in 3GPP TS 29.519 [23]. If the request is accepted by the V-UDR and the V-UDR informs the V-NEF with a successful response or if no interaction with the UDR takes place and the V-NEF successfully handles the information locally, the V-NEF shall update the "Individual ECS Address Configuration Information Set" resource and send an HTTP "200 OK" response with the EcsAddressInfo data structure including the contents of the modified "Individual ECS Address Configuration Information Set" resource in the response body or an HTTP "204 No Content" response. If the V-NEF receives an error response from the V-UDR, the V-NEF shall not update the resource and shall respond to the AF with a proper error status code. If the V-NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the V-NEF shall relay this error response to the AF with a corresponding application error, if applicable.

#### 4.4.40.4	Deletion of existing Individual ECS Address Configuration Information Set

In order to delete an existing "Individual ECS Address Configuration Information Set" resource, the AF shall send an HTTP DELETE request message to the V-NEF targetting the URI of this "Individual ECS Address Configuration Information Set" resource. The V-NEF may interact with the V-UDR by invoking the Nudr_DataRepository service as described in 3GPP TS 29.519 [23] to delete the ECS Address Roaming Data in the application data of the V-UDR.

After receiving a successful response from the V-UDR or after successful local handling, the V-NEF shall delete the "Individual ECS Address Configuration Information Set" resource and shall respond to the AF with an HTTP "204 No Content" response message.

If the V-NEF receives an error response from the V-UDR, the V-NEF shall take proper error handling actions and shall respond to the AF with a proper error status code. If the V-NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the V-NEF shall relay this error response to the AF with a corresponding application error, when applicable.

#### 4.4.40.5	Deletion of ECS Address Configuration Information based on given criteria

In order to delete existing "Individual ECS Address Configuration Information Set" which match given attributes, the AF shall send an HTTP POST request with "{apiRoot}/3gpp-ecs-address/<apiVersion>/remove-ecsaddr" as URI. The POST request body shall contain an EcsAddrDeleteCriteria data structure. The EcsAddrDeleteCriteria data structure provided in the request body shall include at least one of the following:

-	an AF identifier within the "afId" attribute;

-	a DNN within the "dnn" attribute;

-	an S-NSSAI within the "snssai" attribute; and

-	ECS Address Configuration Information which, if matched, the matching entries shall be deleted, as "ecsAddrInfo".

Upon the reception of this HTTP POST request, if the NF service consumer is authorized by the V-NEF to delete the ECS Address Configuration Information, the V-NEF shall determine the "Individual ECS Address Configuration Information Set" resources that match the provided criteria and may interact with the V-UDR to delete the associated ECS Address Roaming Data by using the Nudr_DataRepository service as defined in 3GPP TS 29.519 [23]. If the request is accepted by the V-UDR and the V-UDR informs the V-NEF with a successful response of if the V-NEF can succesfully handle the deletion locally, the V-NEF shall send a HTTP "204 No Content" response. If the V-NEF receives an error code from the V-UDR, the V-NEF shall respond to the AF with a proper error status code.

### 4.4.41	Procedures for RSLPPI Parameters Provisioning

#### 4.4.41.1	General

The procedures described in the clauses below are used by an AF to interact with the 5GC for RSLPPI Parameters Provisioning, in order to carry out the following procedures:

-	RSLPPI parameters provisioning procedures (see clause 4.15.6.2 of 3GPP TS 23.502 [2]).

#### 4.4.41.2	Procedures for RSLPPI Parameters Provisioning

This procedure is used by an AF to request the creation/update/deletion of a RSLPPI parameters provisioning.

In order to request the creation of a RSLPPI Parameters Provisioning:

-	an AF shall trigger the RSLPPIParametersProvisioning API by sending an HTTP POST request to the NEF targeting the "RSLPPI Parameters Provisionings" collection resource, with the request body including the RslppiPpData data structure;

-	the NEF shall then check whether the AF is authorized to perform this operation or not;

-	if the AF is authorized:

-	the NEF shall then trigger the Nudm_ParameterProvision service API of the UDM to request the provisioning of the received RSLPPI parameters provisioning data as specified in 3GPP TS 29.503 [17];

and

-	upon reception of a successful response from the UDM as defined in 3GPP TS 29.503 [17] and successful processing of the request, the NEF shall respond to the AF with an HTTP "201 Created" status code including an HTTP "Location" header field containing the URI of the created resource, and the response body including a representation of the created "Individual RSLPPI Parameters Provisioning" resource within the RslppiPpData data structure.

In order to request the update of an existing "Individual RSLPPI Parameters Provisioning" resource:

-	the AF shall trigger the RSLPPIParametersProvisioning API by sending to the NEF either:

-	an HTTP PUT request targeting the corresponding "Individual RSLPPI Parameters Provisioning" resource with the request body including the RslppiPpData data structure; or

-	an HTTP PATCH request targeting the corresponding "Individual RSLPPI Parameters Provisioning" resource with the request body including the RslppiPpDataPatch data structure;

-	after authorizing the request, the NEF shall interact with the UDM via the the Nudm_ParameterProvision service API as defined in 3GPP TS 29.503 [17] to request the provisioning of the received updated RSLPPI parameters provisioning data; and

-	upon reception of a successful response from the UDM as defined in 3GPP TS 29.503 [17] and successful processing of the request, the NEF shall respond to the AF with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual RSLPPI Parameters Provisioning" resource within the RslppiPpData data structure; or

-	an HTTP "204 No Content" status code.

In order to request the deletion of an existing "Individual RSLPPI Parameters Provisioning" resource:

-	the AF shall trigger the RSLPPIParametersProvisioning API by sending an HTTP DELETE request targeting the corresponding "Individual RSLPPI Parameters Provisioning" resource to the NEF;

-	after authorizing the request, the NEF shall interact with the UDM via the the Nudm_ParameterProvision service API as defined in 3GPP TS 29.503 [17] to request the deletion of the correponding RSLPPI parameters provisioning data; and

-	upon reception of a successful response from the UDM as defined in 3GPP TS 29.503 [17] and successful processing of the request, the NEF shall respond to the AF with an HTTP "204 No Content" status code.

On failure or if the NEF receives an error code from the UDM, the NEF shall take proper error handling actions, as specified in clause 5.37.7, and respond to the AF with an appropriate error status code.

# 5	NEF Northbound APIs

## 5.1	Introduction

The NEF Northbound APIs are a set of APIs defining the related procedures and resources for the interaction between the NEF and the AF.

Tables 5.1-1 summarizes the APIs defined in this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause defined | Description | OpenAPI Specification File | API Name | Annex |
| --- | --- | --- | --- | --- | --- |
| TrafficInfluence | 5.4 | Traffic Influence API | TS29522_TrafficInfluence.yaml | 3gpp-traffic-influence | A.2 |
| NiddConfigurationTrigger | 5.5 | NIDD (Non-IP Data Delivery) Configuration Trigger API | TS29522_NiddConfigurationTrigger.yaml | 3gpp-nidd-configuration-trigger | A.3 |
| AnalyticsExposure | 5.6 | Analytics Exposure API | TS29522_AnalyticsExposure.yaml | 3gpp-analyticsexposure | A.4 |
| 5GLANParameterProvision | 5.7 | 5G LAN Parameter Provision API | TS29522_5GLANParameterProvision.yaml | 3gpp-5glan-pp | A.5 |
| ApplyingBdtPolicy | 5.8 | Applying BDT Policy API | TS29522_ApplyingBdtPolicy.yaml | 3gpp-applying-bdt-policy | A.6 |
| IPTVConfiguration | 5.9 | IPTV Configuration API | TS29522_IPTVConfiguration.yaml | 3gpp-iptvconfiguration | A.7 |
| LpiParameterProvision | 5.10 | LPI (Location Privacy Indicator) Parameter Provision API | TS29522_LpiParameterProvision.yaml | 3gpp-lpi-pp | A.8 |
| ServiceParameter | 5.11 | Service Parameter API | TS29522_ServiceParameter.yaml | 3gpp-service-parameter | A.9 |
| ACSParameterProvision | 5.12 | ACS Parameter Provision API | TS29522_ACSParameterProvision.yaml | 3gpp-acs-pp | A.10 |
| MoLcsNotify | 5.13 | MO LCS Notify API | TS29522_MoLcsNotify.yaml | 3gpp-mo-lcs-notify | A.11 |
| AKMA | 5.14 | AKMA API | TS29522_AKMA.yaml | 3gpp-akma | A.12 |
| TimeSyncExposure | 5.15 | Time Sync Exposure API | TS29522_TimeSyncExposure.yaml | 3gpp-time-sync-exposure | A.13 |
| EcsAddressProvision | 5.16 | ECS Address Provision API | TS29522_EcsAddressProvision.yaml | 3gpp-ecs-address-provision | A.14 |
| AMPolicyAuthorization | 5.17 | AM Policy Authorization API | TS29522_AMPolicyAuthorization.yaml | 3gpp-am-policyauthorization | A.15 |
| AMInfluence | 5.18 | AM Influence API | TS29522_AMInfluence.yaml | 3gpp-am-influence | A.16 |
| MBSTMGI | 5.19 | MBS TMGI API | TS29522_MBSTMGI.yaml | 3gpp-mbs-tmgi | A.17 |
| MBSSession | 5.20 | MBS Session API | TS29522_MBSSession.yaml | 3gpp-mbs-session | A.18 |
| EASDeployment | 5.21 | EAS Deployment API | TS29522_EASDeployment.yaml | 3gpp-eas-deployment | A.19 |
| ASTI | 5.22 | ASTI API | TS29522_ASTI.yaml | 3gpp-asti | A.20 |
| DataReporting | 5.23 | DataReporting API | TS29522_DataReporting.yaml | 3gpp-data-reporting | A.21 |
| DataReportingProvisioning | 5.24 | DataReportingProvisioning API | TS29522_DataReportingProvisioning.yaml | 3gpp-data-reporting-provisioning | A.22 |
| UEId | 5.25 | UE ID API | TS29522_UEId.yaml | 3gpp-ueid | A.23 |
| MBSUserService | 5.26 | MBSUserService API | TS29522_MBSUserService.yaml | 3gpp-mb-us | A.24 |
| MBSUserDataIngestSession | 5.27 | MBSUserDataIngestSession API | TS29522_MBSUserDataIngestSession.yaml | 3gpp-mb-ud-ingest | A.25 |
| MSEventExposure | 5.28 | MSEventExposure API | TS29522_MSEventExposure.yaml | 3gpp-event-exposure | A.26 |
| MBSGroupMsgDelivery | 5.29 | MBSGroupMsgDelivery API | TS29522_MBSGroupMsgDelivery.yaml | 3gpp-mbs-group-msg | A.27 |
| DNAIMapping | 5.30 | DNAIMapping API | TS29522_DNAIMapping.yaml | 3gpp-dnai-mapping | A.28 |
| PDTQPolicyNegotiation | 5.31 | PDTQPolicyNegotiation API | TS29522_PDTQPolicyNegotiation.yaml | 3gpp-pdtq-policy-negotiation | A.29 |
| MemberUESelectionAssistance | 5.32 | MemberUESelectionAssistance API | TS29522_MemberUESelectionAssistance.yaml | 3gpp-musa | A.30 |
| GroupParametersProvisioning | 5.33 | Group Parameters Provisioning API | TS29.522_GroupParametersProvisioning.yaml | 3gpp-grp-pp | A.31 |
| SliceParamProvision | 5.34 | Network Slice Parameters Provisioning API | TS29.522_SliceParamProvision.yaml | 3gpp-slice-pp | A.32 |
| UEAddress | 5.35 | UE Address API | TS29522_UEAddress.yaml | 3gpp-ue-address | A.33 |
| ECSAddress | 5.36 | ECS Address Configuration Information API | TS29522_ECSAddress.yaml | 3gpp-ecs-address | A.34 |
| RSLPPIParametersProvisioning | 5.37 | RSLPPI Parameters Provisioning API | TS29522_RSLPPIParametersProvisioning.yaml | 3gpp-rslppi-pp | A.35 |

## 5.2	Information applicable to several APIs

The usage of HTTP, content type and URI structure definition, as specified in clauses 5.2.2, 5.2.3 and 5.2.4 of 3GPP TS 29.122 [4] respectively, shall be applicable for NEF Northbound APIs.

The notification, error handling, feature negotiation, HTTP custom headers as specified in clauses 5.2.5, 5.2.6, 5.2.7, 5.2.8 of 3GPP TS 29.122 [4] respectively, shall be applicable for NEF Northbound APIs except that the SCEF is replaced by the NEF and the SCS/AS is replaced by the AF.

The conventions for Open API specification files as specified in clause 5.2.9 of 3GPP TS 29.122 [4] shall be applicable for NEF Northbound APIs.

When 3GPP TS 29.122 [4] is referenced for the common protocol and interface aspects for API definition in the following clauses of this specification, the NEF takes the role of the SCEF and the service consumer (i.e., AF) takes the role of the SCS/AS, unless otherwise explicitly indicated.

## 5.3	Reused APIs

This clause describes the northbound APIs which are applicable for both EPS and 5GS.

Table 5.3-1: Reused APIs applicable for both EPS and 5GS

| API Name | Differences |
| --- | --- |
| ResourceManagementOfBdt | - The following 5G-only features defined in clause 5.4.4 of 3GPP TS 29.122 [4] may be supported only by the NEF: "LocBdt_5G", "Group_Id", "BdtNotification_5G", "AspId_5G". |
| PfdManagement | - The following 5G-only features defined in clause 5.11.4 of 3GPP TS 29.122 [4] may be supported only by the NEF: "FailureLocation_5G". |
| MonitoringEvent | - The following 5G-only features defined in clause 5.3.4 of 3GPP TS 29.122 [4] may be supported only by the NEF: "Number_of_UEs_in_an_area_notification_5G", "Downlink_data_delivery_status_5G", "Availability_after_DDN_failure_notification_enhancement", "eLCS", "eLCS_en", "NSAC", "MULTIQOS", "EDGEAPP", "UEId_retrieval", "Loss_of_connectivity_notification_5G", "GMEC", "enNB1_5G", "AppDetection_5G", "eNSAC", "Ranging_SL", "DataTransfer" and "RVAS_5G". - For the "Pdn_connectivity_status" feature, APN is equivalent to DNN; the non-IP PDN type is equivalent to the unstructured PDU session type; and the enumeration InterfaceIndication value "PDN_GATEWAY" stands for PDU session anchored in UPF in 5G. |
| DeviceTriggering |  |
| CpProvisioning | - The following 5G-only features defined in clause 5.10.4 of 3GPP TS 29.122 [4] may be supported only by the NEF: "ExpectedUMT_5G", "ExpectedUmtTime_5G", "ScheduledCommType_5G", "UEId_retrieval", "ExpectedUmtTime_Add", "AppExpUeBehaviour" and "ConfAccuLevels". |
| ChargeableParty | - The following 5G-only features defined in clause 5.5.4 of 3GPP TS 29.122 [4] may be supported only by the NEF: "EthChgParty_5G", "MacAddressRange_5G", "ToSTC_5G". - The "LOSS_OF_BEARER", "RECOVERY_OF_BEARER" and "RELEASE_OF_BEARER" events do not apply for 5G. |
| AsSessionWithQoS | - The following 5G-only features defined in clause 5.14.4 of 3GPP TS 29.122 [4] may be supported only by the NEF: "EthAsSessionQoS_5G", "QoSMonitoring_5G", "PacketDelayFailureReport", "MacAddressRange_5G", "AlternativeQoS_5G", "TSC_5G", "DisableUENotification_5G", "ExposureToEAS", "AltQosWithIndParams_5G", "EnEthAsSessionQoS_5G", "enNB_5G", "AltQoSProfilesSupportReport", "ExtQoS_5G", "EnTSCAC", "L4S", "MultiMedia", "PowerSaving", "EnQoSMon", "PDUSetHandling", "RTLatency", "ToSTC_5G", "QoSTiming_5G", "ListUE_5G", "GMEC" and "QoSMonCapRepo". - The "LOSS_OF_BEARER", "RECOVERY_OF_BEARER" and "RELEASE_OF_BEARER" events do not apply for 5G. |
| MsisdnLessMoSms |  |
| NpConfiguration | - The following 5G-only features defined in clause 5.13.4 of 3GPP TS 29.122 [4] may be supported only by the NEF: "NpExpiry_5G", "UEId_retrieval". |
| NIDD |  |
| RacsParameterProvisioning |  |
| ECRControl | - The following 5G-only features defined in clause 5.12.4 of 3GPP TS 29.122 [4] may be supported only by the NEF: "ECR_WB_5G". |

## 5.4	TrafficInfluence API

### 5.4.0	Introduction

The Nnef_TrafficInfuence service shall use the TrafficInfluence API.

The API URI of TrafficInfluence API shall be:

- {apiRoot}/3gpp-traffic-influence/v1

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-traffic-influence".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.4.1	Resources

#### 5.4.1.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.4.1.1-1 and the resources and HTTP methods used for the TrafficInfluence API.

Figure 5.4.1.1-1: Resource URI structure of the TrafficInfluence API

Table 5.4.1.1-1 provides an overview of the resources and HTTP methods applicable for the TrafficInfluence API.

Table 5.4.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Traffic Influence Subscription | /{afId}/subscriptions | GET | Read all subscriptions for a given AF |
|  |  | POST | Create a new subscription to traffic influence |
| Individual Traffic Influence Subscription | /{afId}/subscriptions/{subscriptionId} | GET | Read a subscription to traffic influence |
|  |  | PUT | Modify all of the properties of an existing subscription to traffic influence |
|  |  | PATCH | Modify part of the properties of an existing subscription to traffic influence |
|  |  | DELETE | Delete a subscription to traffic influence |

#### 5.4.1.2	Resource: Traffic Influence Subscription

##### 5.4.1.2.1	Introduction

This resource allows a AF to read all active traffic influence subscribtions for the given AF.

##### 5.4.1.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-traffic-influence/v1/{afId}/subscriptions

This resource shall support the resource URI variables defined in table 5.4.1.2.2-1.

Table 5.4.1.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |

##### 5.4.1.2.3	Resource Methods

###### 5.4.1.2.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.4.1.2.2.

###### 5.4.1.2.3.2	GET

The GET method allows to read all active subscriptions for a given AF. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.4.1.2.3.2-1.

Table 5.4.1.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.4.1.2.3.2-2 and the response data structures and response codes specified in table 5.4.1.2.3.2-3.

Table 5.4.1.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.4.1.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(TrafficInfluSub) | M | 0..N | 200 OK | The subscription information for the AF in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.4.1.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.4.1.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.4.1.2.3.3	POST

The POST method creates a new subscription resource to traffic influence subscription for a given AF. The AF shall initiate the HTTP POST request message and the NEF shall respond to the message. The NEF shall construct the URI of the created resource.

This method shall support the request data structures specified in table 5.4.1.2.3.3-1 and the response data structures and response codes specified in table 5.4.1.2.3.3-2.

Table 5.4.1.2.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TrafficInfluSub | M | 1 | Parameters to register a subscription to influencing traffic routing and/or notification about UP management events with the NEF. |

Table 5.4.1.2.3.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TrafficInfluSub | M | 1 | 201 Created | The subscription was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.4.1.2.3.3-3: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-traffic-Influence/v1/{afId}/subscriptions/{subscriptionId} |

#### 5.4.1.3	Resource: Individual Traffic Influence Subscription

##### 5.4.1.3.1	Introduction

This resource allows a AF to register a subscription to influencing traffic routing and/or notification about UP management events with the NEF.

##### 5.4.1.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-traffic-influence/v1/{afId}/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 5.4.1.3.2-1.

Table 5.4.1.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |
| subscriptionId | string | Identifier of the subscription. |

##### 5.4.1.3.3	Resource Methods

###### 5.4.1.3.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.4.1.3.2.

###### 5.4.1.3.3.2	GET

The GET method allows to read the active subscription for a given AF and subscription Id. The AF shall initiate the HTTP GET request message and theNEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.4.1.3.3.2-1.

Table 5.4.1.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.4.1.3.3.2-2 and the response data structures and response codes specified in table 5.4.1.3.3.2-3.

Table 5.4.1.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.4.1.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TrafficInfluSub | M | 1 | 200 OK | The subscription information for the AF in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.4.1.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.4.1.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.4.1.3.3.3	PUT

The PUT method is used to replace an existing subscription resource to update a subscription. The AF shall initiate the HTTP PUT request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.4.1.3.3.3-1 and the response data structures and response codes specified in table 5.4.1.3.3.3-2.

Table 5.4.1.3.3.3-1: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TrafficInfluSub | M | 1 | Modify an existing subscription to influencing traffic routing and/or notification about UP management events with the NEF. |

Table 5.4.1.3.3.3-2: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TrafficInfluSub | M | 1 | 200 OK | The subscription was replaced successfully and a representation is returned. |
| N/A |  |  | 204 No Content | The subscription was replaced successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.4.1.3.3.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.4.1.3.3.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.4.1.3.3.4	PATCH

The PATCH method allows to change some properties of an existing traffic influence subscription. The AF shall initiate the HTTP PATCH request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.4.1.3.3.4-1 and the response data structures and response codes specified in table 5.4.1.3.3.4-2.

Table 5.4.1.3.3.4-1: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TrafficInfluSubPatch | M | 1 | Partial update of a subscription to influencing traffic routing and/or notifications about UP management events with the NEF. |

Table 5.4.1.3.3.4-2: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TrafficInfluSub | M | 1 | 200 OK | The subscription was partial modified successfully and a representation is returned. |
| N/A |  |  | 204 No Content | The subscription was partial modified successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.4.1.3.3.4-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.4.1.3.3.4-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.4.1.3.3.5	DELETE

The DELETE method deletes the traffic influence subscription for a given AF. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.4.1.3.3.5-1.

Table 5.4.1.3.3.5-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.4.1.3.3.5-2 and the response data structures and response codes specified in table 5.4.1.3.3.5-3.

Table 5.4.1.3.3.5-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.4.1.3.3.5-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The subscription was terminated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.4.1.3.3.5-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.4.1.3.3.5-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

### 5.4.1A	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.4.2	Notifications

#### 5.4.2.1	Introduction

Upon receipt of a UP management event notification from the SMF indicating the subscribed event (e.g. a DNAI has changed) is detected, the NEF shall send an HTTP POST message including the notified event to the AF.

Upon receipt of the event notification, the AF may send an HTTP POST request as acknowledgement for the UP path management event notification to inform the NEF about the result of application layer relocation.

The NEF and the AF shall support the notification mechanism as described in clause 5.2.5 of 3GPP TS 29.122 [4].

Table 5.4.2.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Event Notification | {notificationDestination} | POST | The UP management event notification from the NEF to the AF. |
| Acknowledgement of event notification | {afAckUri} | POST | The Acknowledgement of Event Notification is used by the AF to acknowledge the NEF about handling result of the event notification. |

#### 5.4.2.2	Event Notification

##### 5.4.2.2.1	Description

The Event Notification is used by the NEF to report the UP path management event notification from the SMF to the AF.

##### 5.4.2.2.2	Target URI

The Callback URI "{notificationDestination}" shall be used with the callback URI variables defined in table 5.4.2.2.2-1.

Table 5.4.2.2.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notificationDestination | Link | Callback reference provided by the AF during creation/modification of the subscription within the TrafficInfluSub data type as defined in table 5.4.3.3.2-1 or the TrafficInfluSubPatch data type as defined in table 5.4.3.3.3-1. |

##### 5.4.2.2.3	Operation Definition

5.4.2.2.3.1	Notification via HTTP POST

This method shall support the request data structures specified in table 5.4.2.2.3.1-1 and the response data structures and response codes specified in table 5.4.2.2.3.1-2.

Table 5.4.2.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EventNotification | M | 1 | The UP management event notification is provided by the NEF to the AF. |

Table 5.4.2.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The event notification is received successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during event notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during event notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.4.2.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

Table 5.4.2.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

5.4.2.2.3.2	Notification via Websocket

If supported by both AF and NEF and successfully negotiated, the EventNotification may alternatively be delivered through the Websocket mechanism as defined in clause 5.2.5.4 of 3GPP TS 29.122 [4].

#### 5.4.2.3	Acknowledgement of event notification

##### 5.4.2.3.1	Description

The Acknowledgement of Event Notification is used by the AF to acknowledge the NEF about handling result of the event notification (e.g. UP path change).

##### 5.4.2.3.2	Target URI

The Callback URI "{afAckUri}" shall be used with the callback URI variables defined in table 5.4.2.3.2-1.

Table 5.4.2.3.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| afAckUri | Link | Callback reference provided by the NEF during event notification within the EventNotification data type as defined in Table 5.4.3.3.4-1. |

##### 5.4.2.3.3	Operation Definition

###### 5.4.2.3.3.1	Notification via HTTP POST

This method shall support the request data structures specified in table 5.4.2.3.3.1-1 and the response data structures and response codes specified in table 5.4.2.3.3.1-2.

Table 5.4.2.3.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AfAckInfo | M | 1 | Acknowledgement information of event notification. |

Table 5.4.2.3.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The acknowledgement of event notification is received successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during acknowledgement of event notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NEF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during acknowledgement of event notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NEF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.4.2.3.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative NEF towards which the notification should be redirected. |

Table 5.4.2.3.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative NEF towards which the notification should be redirected. |

### 5.4.3	Data Model

#### 5.4.3.1	General

This clause specifies the application data model supported by the TrafficInfluence API.

Table 5.4.3.1-1 specifies the data types defined for the TrafficInfluence API.

Table 5.4.3.1-1: TrafficInfluence API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AfAckInfo | 5.4.3.3.6 | Represents acknowledgement information of a traffic influence event notification. |  |
| AfResultInfo | 5.4.3.3.5 | Identifies the result of application layer handling. |  |
| AfResultStatus | 5.4.3.4.4 | Represents the status of application handling result. |  |
| EventNotification | 5.4.3.3.4 | Represents a traffic influence event notification. |  |
| SubscribedEvent | 5.4.3.4.3 | Represents the type of UP path management events for which the AF requests to be notified. |  |
| TrafficInfluSub | 5.4.3.3.2 | Represents a traffic influence subscription. |  |
| TrafficInfluSubPatch | 5.4.3.3.3 | Represents parameters to request the modification of a traffic influence subscription resource. |  |
| TrafficDataSet | 5.4.3.3.7 | Represents a set of traffic filters and the corresponding N6 traffic routing requirements. | MultiTrafficInflu |
| TrafficDataSetRm | 5.4.3.3.8 | Represents a set of traffic filters and the corresponding N6 traffic routing requirements. This data type is defined in the same way as the TrafficDataSet data type, but with the OpenAPI "nullable: true" property. | MultiTrafficInflu |

#### 5.4.3.2	Reused data types

The data types reused by the TrafficInfluence API from other specifications are listed in table 5.4.3.2-1.

Table 5.4.3.2-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Dnai | 3GPP TS 29.571 [8] | Identifies a DNAI. |
| DnaiChangeType | 3GPP TS 29.571 [8] | Describes the types of DNAI change. |
| Dnn | 3GPP TS 29.571 [8] | Identifies a DNN. |
| DurationSec | 3GPP TS 29.571 [8] | Identifies a period of time in units of seconds. |
| EasIpReplacementInfo | 3GPP TS 29.571 [8] | Represents EAS IP replacement information. |
| EthFlowDescription | 3GPP TS 29.514 [7] | Contains the Ethernet data flow information. (NOTE) |
| ExternalGroupId | 3GPP TS 29.122 [4] | External Group Identifier for a user group. |
| FlowInfo | 3GPP TS 29.122 [4] | Contains the IP data flow information. |
| GeographicalArea | Clause 5.17.3.3.4 | Identifies a geographical area. |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |
| Ipv4Addr | 3GPP TS 29.122 [4] | Identifies an IPv4 address. |
| Ipv6Addr | 3GPP TS 29.122 [4] | Identifies an IPv6 address. |
| Ipv6Prefix | 3GPP TS 29.571 [8] | Identifies an IPv6 Prefix. |
| Link | 3GPP TS 29.122 [4] | Identifies a referenced resource. |
| MacAddr48 | 3GPP TS 29.571 [8] | Identifies a MAC address. |
| Metadata | 3GPP TS 29.571 [8] | Contains opaque information for the service functions in the N6-LAN that is provided by AF and transparently sent to UPF. |
| PlmnId | 3GPP TS 29.571 [8] | Identifies a PLMN Identifier. |
| Port | 3GPP TS 29.122 [4] | Identifies a port number. |
| ReportingInformation | 3GPP TS 29.523 [22] | Represents the event reporting requirements. |
| RouteToLocation | 3GPP TS 29.571 [8] | Describes the traffic routes to the locations of the application. |
| Snssai | 3GPP TS 29.571 [8] | Identifies the S-NSSAI. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.4.4-1. |
| TemporalValidity | 3GPP TS 29.514 [7] | Indicates the time interval(s) during which the AF request is to be applied |
| TrafficCorrelationInfo | 3GPP TS 29.519 [23] | Contains the information for traffic correlation. |
| Uinteger | 3GPP TS 29.571 [8] | Unsigned integer. |
| UintegerRm | 3GPP TS 29.571 [8] | This data type is defined in the same way as the "Uinteger" data type, but with the OpenAPI "nullable: true" property. |
| WebsockNotifConfig | 3GPP TS 29.122 [4] | Contains the configuration parameters to set up notification delivery over Websocket protocol. |
| NOTE: In order to support a set of MAC addresses with a specific range in the traffic filter, feature MacAddressRange as specified in clause 5.4.4 shall be supported. |  |  |

#### 5.4.3.3	Structured data types

##### 5.4.3.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.4.3.3.2	Type: TrafficInfluSub

This type represents a traffic influence subscription. The same structure is used in the subscription request and subscription response.

Table 5.4.3.3.2-1: Definition of type TrafficInfluSub

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability (NOTE 1) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| afServiceId |  | string |  | O |  | 0..1 |  | Identifies a service on behalf of which the AF is issuing the request. |  |  |  |
| afAppId |  | string |  | O |  | 0..1 |  | Identifies an application. (NOTE 3) |  |  |  |
| afTransId |  | string |  | O |  | 0..1 |  | Identifies an NEF Northbound interface transaction, generated by the AF. |  |  |  |
| appReloInd |  | boolean |  | O |  | 0..1 |  | Identifies whether an application can be relocated once a location of the application has been selected. - Set to "true" if it shall be relocated. - Set to "false" if it shall not be relocated. - Default value is "false" if omitted. |  |  |  |
| dnn |  | Dnn |  | O |  | 0..1 |  | Identifies a DNN, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. |  |  |  |
| snssai |  | Snssai |  | O |  | 0..1 |  | Identifies an S-NSSAI. |  |  |  |
| externalGroupId |  | ExternalGroupId |  | O |  | 0..1 |  | Identifies a group of users. (NOTE 2) (NOTE 6) |  |  |  |
| externalGroupIds |  | array(ExternalGroupId) |  | O |  | 2..N |  | List of external group identifiers associated with the subscriber. (NOTE 2) (NOTE 6) (NOTE 7) |  | FinerGranUEs |  |
| extSubscCats |  | array(string) |  | O |  | 1..N |  | List of external categories associated with the subscriber. (NOTE 8) |  | FinerGranUEs |  |
| anyUeInd |  | boolean |  | O |  | 0..1 |  | Identifies whether the AF request applies to any UE (i.e. all UEs). - Set to "true": the AF request is applicable to any UE. - Set to "false": the AF request is not applicable to any UE. - Default value is "false" if omitted. (NOTE 2) |  |  |  |
| subscribedEvents |  | array(SubscribedEvent) |  | O |  | 1..N |  | Identifies the requirement to be notified of the event(s). |  |  |  |
| gpsi |  | Gpsi |  | O |  | 0..1 |  | Identifies a user. (NOTE 2) |  |  |  |
| ipv4Addr |  | Ipv4Addr |  | O |  | 0..1 |  | Identifies the IPv4 address. (NOTE 2) |  |  |  |
| ipDomain |  | string |  | O |  | 0..1 |  | The IPv4 address domain identifier. The attribute may only be provided if the ipv4Addr attribute is present. |  |  |  |
| ipv6Addr |  | Ipv6Addr |  | O |  | 0..1 |  | Identifies the IPv6 address. (NOTE 2) |  |  |  |
| macAddr |  | MacAddr48 |  | O |  | 0..1 |  | Identifies the MAC address. (NOTE 2) |  |  |  |
| dnaiChgType |  | DnaiChangeType |  | O |  | 0..1 |  | Identifies a type of notification regarding UP path management event. |  |  |  |
| notificationDestination |  | Link |  | C |  | 0..1 |  | Contains the Callback URL to receive the notification from the NEF. It shall be present if the "subscribedEvents" is present. |  |  |  |
| requestTestNotification |  | boolean |  | O |  | 0..1 |  | Indicates whether the AF requests the NEF to send a test notification. - Set to "true" by the AF to request the NEF to send a test notification as defined in clause 5.2.5.3 of 3GPP TS 29.122 [4]. - Set to "false" by the AF to not to request the NEF to send a test notification. - Default value is "false" if omitted. |  | Notification_test_event |  |
| websockNotifConfig |  | WebsockNotifConfig |  | O |  | 0..1 |  | Configuration parameters to set up notification delivery over Websocket protocol. |  | Notification_websocket |  |
| self |  | Link |  | C |  | 0..1 |  | Link to the created resource. This parameter shall be supplied by the NEF in HTTP responses that include an object of TrafficInfluSub type |  |  |  |
| trafficDataSets |  | map(TrafficDataSet) |  | O |  | 2..N |  | Contains multiple sets of traffic filters with the corresponding N6 traffic routing requirements. The key of the map shall be the value of the "setId" attribute of the TrafficDataSet data type. (NOTE 3, NOTE 11, NOTE 12, NOTE 13) |  | MultiTrafficInflu |  |
| trafficFilters |  | array(FlowInfo) |  | O |  | 1..N |  | Identifies IP packet filters. (NOTE 3) |  |  |  |
| ethTrafficFilters |  | array(EthFlowDescription) |  | O |  | 1..N |  | Identifies Ethernet packet filters. (NOTE 3) |  |  |  |
| trafficRoutes |  | array(RouteToLocation) |  | O |  | 1..N |  | Identifies the N6 traffic routing requirement. (NOTE 9, NOTE 11) |  |  |  |
| sfcIdDl |  | string |  | O |  | 0..1 |  | Reference to a pre-configured steering of user traffic to service function chain in downlink. (NOTE 5) |  | SFC |  |
| sfcIdUl |  | string |  | O |  | 0..1 |  | Reference to a pre-configured steering of user traffic to service function chain in uplink. (NOTE 5) |  | SFC |  |
| metadata |  | Metadata |  | O |  | 0..1 |  | Contains opaque information for the service functions in the N6-LAN that is provided by AF and transparently sent to UPF. May only be provided when "sfcIdDl" and/or "sfcIdUl" are provided. |  | SFC |  |
| tfcCorrInd |  | boolean |  | O |  | 0..1 |  | Indication of traffic correlation. May only be included when "externalGroupId" attribute was included within the TrafficInfluSub data type previously. It is used to indicate that for the group of UEs, the targeted PDU sessions should be correlated by a common DNAI. Set to "true" if it should be correlated; otherwise set to "false". Default value is "false" if omitted. (NOTE 4) (NOTE 10) |  |  |  |
| tfcCorreInfo |  | TrafficCorrelationInfo |  | O |  | 0..1 |  | Contains the information for traffic correlation. The "notifUri" and "notifCorrId" attributes are not applicable for "tfcCorreInfo" attribute. (NOTE 10) |  | CommonEASDNAI |  |
| tempValidities |  | array(TemporalValidity) |  | O |  | 1..N |  | Indicates the time interval(s) during which the AF request is to be applied. |  |  |  |
| validGeoZoneIds |  | array(string) |  | O |  | 1..N |  | Identifies a geographic zone that the AF request applies only to the traffic of UE(s) located in this specific zone. This attribute is deprecated; the attribute "geoAreas" should be used instead. |  |  |  |
| geoAreas |  | array(GeographicalArea) |  | O |  | 1..N |  | Identifies geographical areas within which the AF request applies. This attribute deprecates validGeoZoneIds attribute. |  |  |  |
| afAckInd |  | boolean |  | O |  | 0..1 |  | Identifies whether the AF acknowledgement of UP path event notification is expected. - "true" indicates that the AF acknowledgement of UP path event is expected. - "false" indicates that the AF acknowledgement of UP path event notification is not expected. - Default value is "false" if omitted. |  | URLLC |  |
| addrPreserInd |  | boolean |  | O |  | 0..1 |  | Indicates whether UE IP address shall be preserved. - "true" indicates that the UE IP address shall be preserved. - "false" indicates that the UE IP address shall not preserved. - Defalult value is "false" if omitted. |  | URLLC |  |
| simConnInd |  | boolean |  | O |  | 0..1 |  | Indication of whether simultaneous connectivity shall be temporarily maintained for the source and target PSA. - "true" indicates that the temporary simultaneous connectivity shall be kept. - "false" indicates that the temporary simultaneous connectivity shall not be kept. - Default value is "false" if omitted. |  | SimultConnectivity |  |
| simConnTerm |  | DurationSec |  | O |  | 0..1 |  | Indication of the minimum time interval to be considered for inactivity of the traffic routed via the source PSA during the edge re-location procedure. It may be included when the "simConnInd" attribute is set to true. |  | SimultConnectivity |  |
| maxAllowedUpLat |  | Uinteger |  | O |  | 0..1 |  | Indicates the target user plane latency in units of milliseconds. The SMF may use this value to decide whether edge relocation is needed to ensure that the user plane latency does not exceed the value. |  | AF_lantency |  |
| easIpReplaceInfos |  | array(EasIpReplacementInfo) |  | O |  | 1..N |  | Contains EAS IP replacement information. |  | EASIPreplacement |  |
| easRedisInd |  | boolean |  | O |  | 0..1 |  | Indicates whether the EAS rediscovery is required for the application. - "true" indicates that the EAS rediscovery is required for the application. - "false" indicates that the EAS rediscovery is not required for the application. - Defalult value is "false" if omitted. The indication shall be invalid after it was applied unless it is provided again. |  | EASDiscovery |  |
| eventReq |  | ReportingInformation |  | O |  | 0..1 |  | Indicates the event reporting requirements. This attribute may be provided if the "EDGEAPP" feature is supported and the "subscribedEvents" attribute is present. |  | EDGEAPP |  |
| eventReports |  | array(EventNotification) |  | C |  | 1..N |  | Represents user plane path management event report(s). This attribute shall be present in an HTTP POST response if the immediate reporting indication in the "immRep" attribute within the "eventReq" attribute is set to true and the "subscribedEvents" was present in the corresponding HTTP POST request and the report(s) are available. This attribute may also be present in an HTTP PUT or PATCH response when the report(s) are available. |  | EDGEAPP |  |
| candDnaiInd |  | boolean |  | O |  | 0..1 |  | Indication of reporting candidate DNAI(s). If it is included and set to "true", the candidate DNAI(s) for the PDU session need to be reported. Otherwise, the default value is "false" if omitted. |  | CommonEASDNAI |  |
| plmnId |  | PlmnId |  | O |  | 0..1 |  | Identifies the H-PLMN of the UE. |  | HR-SBO |  |
| portNumber |  | Port |  | O |  | 0..1 |  | Indicates the UDP or TCP port number associated with the UE IP address as provided in the "ipv4Addr" or "ipv6Addr" property. |  | HR-SBO |  |
| suppFeat |  | SupportedFeatures |  | C |  | 0..1 |  | Indicates the list of Supported features used as described in clause 5.4.4. This attribute shall be provided in the POST request and in the response of successful resource creation. |  |  |  |
| NOTE 1: Properties marked with a feature as defined in clause 5.4.4 are applicable as described in clause 5.2.7 of 3GPP TS 29.122 [4]. If no feature is indicated, the related property applies for all the features. NOTE 2: If "HR-SBO" feature is not supported, only one of individual UE identifier (i.e. "gpsi", "macAddr", "ipv4Addr" or "ipv6Addr"), External Group Identifier (i.e. "externalGroupId" or "externalGroupIds" (is included when FinerGranUEs feature is supported)) or any UE indication "anyUeInd" shall be included. If "HR-SBO" feature is supported and the AF requests to influence traffic routing is working in HR-SBO mode in the VPLMN, only one of individual UE identifier (i.e. "gpsi", "ipv4Addr" or "ipv6Addr") or any UE indication "anyUeInd" shall be included. NOTE 3: One of "afAppId", "trafficFilters", "ethTrafficFilters" or “trafficDataSets” shall be included. NOTE 4: The indication of traffic correlation shall be provided only when the AF requires that all the PDU sessions related to the 5G VN group member UEs should be correlated by a common DNAI in the user plane for the traffic as described in 3GPP TS 23.501 [3], clause 5.6.7.1 and clause 5.29. NOTE 5: When the SFC feature is supported, for the purpose of influencing service function chaining, at least one attribute shall be present. NOTE 6: The attributes "externalGroupId" and "externalGroupIds" are mutually exclusive attributes. NOTE 7: The AF request applies to the UE(s) that belong to all the External Group Identifiers indicated by the attribute "externalGroupIds", when included. NOTE 8: The AF request applies to the UE(s) that belong to all the External Subscriber Categories indicated by the attribute "extSubscCats", which is included only if either "externalGroupIds" attribute is included or "externalGroupId" is included or "anyUeInd" attribute is included. If "HR-SBO" feature is supported and the AF requests to influence is working in HR-SBO mode, the "extSubscCats" attribute shall not be provided. NOTE 9: When only one DNAI is included, and the Indication of traffic correlation within the "tfcCorrInd" attribute is available or the "corrType" attribute of the "tfcCorreInfo" includes the value "COMMON_DNAI", the DNAI is used as common DNAI for UEs identified by AF request. NOTE 10: The "tfcCorrInd" attribute and the "tfcCorreInfo" attribute are mutually exclusive. NOTE 11: These attributes are mutually exclusive. Either one of them may be present. NOTE 12: This attribute may be present only if one of "macAddr",attribute "ipv4Addr" attribute or the "ipv6Addr" attribute is provided. NOTE 13: If this attribute is present, then the "candDnaiInd", "tfcCorrInd" and "tfcCorreInfo" attributes shall not be present. |  |  |  |  |  |  |  |  |  |  |  |

##### 5.4.3.3.3	Type: TrafficInfluSubPatch

This type represents a subscription of traffic influence parameters provided by the AF to the NEF. The structure is used for HTTP PATCH request.

Table 5.4.3.3.3-1: Definition of type TrafficInfluSubPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appReloInd | boolean | O | 0..1 | Identifies whether an application should be relocated once a location of the application has been selected. - "true" indicates that an application shall be relocated once a location of the application has been selected. - "false" indicates that an application shall not be relocated once a location of the application has been selected. (NOTE 1) |  |
| trafficDataSets | map(TrafficDataSetRm) | O | 1..N | Contains one or several set(s) of traffic filters with the corresponding N6 traffic routing requirements. The key of the map shall be the value of the "setId" attribute of the TrafficDataSet data type. | MultiTrafficInflu |
| trafficFilters | array(FlowInfo) | O | 1..N | Identifies IP packet filters. |  |
| ethTrafficFilters | array(EthFlowDescription) | O | 1..N | Identifies Ethernet packet filters. |  |
| trafficRoutes | array(RouteToLocation) | O | 1..N | Identifies the N6 traffic routing requirement. (NOTE 1) |  |
| sfcIdDl | string | O | 0..1 | Reference to a pre-configured steering of user traffic to service function chain in downlink. | SFC |
| sfcIdUl | string | O | 0..1 | Reference to a pre-configured steering of user traffic to service function chain in uplink. | SFC |
| metadata | Metadata | O | 0..1 | Contains opaque information for the service functions in the N6-LAN that is provided by AF and transparently sent to UPF. | SFC |
| tfcCorrInd | boolean | O | 0..1 | Indication of traffic correlation. May only be included when "externalGroupId" attribute was included within the TrafficInfluSub data type previously. - "true" indicates that for the group of UEs, the targeted PDU sessions should be correlated by a common DNAI. - "false" indicates that for the group of UEs, the targeted PDU sessions should not be correlated by a common DNAI. (NOTE 2) | CommonEASDNAI |
| tfcCorreInfo | TrafficCorrelationInfo | O | 0..1 | Contains the information for traffic correlation. The "notifUri" and "notifCorrId" attributes are not applicable for "tfcCorreInfo" attribute. (NOTE 2) | CommonEASDNAI |
| tempValidities | array(TemporalValidity) | O | 1..N | Indicates the time interval(s) during which the AF request is to be applied. (NOTE 1) |  |
| validGeoZoneIds | array(string) | O | 1..N | Identifies a geographic zone that the AF request applies only to the traffic of UE(s) located in this specific zone. (NOTE 1) This attribute is deprecated; the attribute "geoAreas" should be used instead. |  |
| geoAreas | array(GeographicalArea) | O | 1..N | Identifies geographical areas within which the AF request applies. (NOTE 1) This attribute deprecates validGeoZoneIds attribute. |  |
| afAckInd | boolean | O | 0..1 | Identifies whether the AF acknowledgement of UP path event notification is expected. - "true" indicates that the AF acknowledgement of UP path event notification is expected. - "false" indicates that the AF acknowledgement of UP path event notification is not expected. (NOTE 3) | URLLC |
| addrPreserInd | boolean | O | 0..1 | Indicates whether UE IP address shall be preserved. - "true" indicates that the UE IP address shall be preserved. - "false" indicates that the UE IP address shall not be preserved. (NOTE 3) | URLLC |
| simConnInd | boolean | O | 0..1 | Indication of whether simultaneous connectivity shall be temporarily maintained for the source and target PSA. - "true" indicates that temporary simultaneous connectivity shall be kept. - "false" indicates that the temporary simultaneous connectivity shall not be kept. | SimultConnectivity |
| simConnTerm | DurationSec | O | 0..1 | Indication of the minimum time interval to be considered for inactivity of the traffic routed via the source PSA during the edge re-location procedure. | SimultConnectivity |
| maxAllowedUpLat | UintegerRm | O | 0..1 | Indicates the target user plane latency in units of milliseconds. The SMF may use this value to decide whether edge relocation is needed to ensure that the user plane latency does not exceed the value. | AF_latency |
| easIpReplaceInfos | array(EasIpReplacementInfo) | O | 1..N | Contains EAS IP replacement information. | EASIPreplacement |
| easRedisInd | boolean | O | 0..1 | Indicates whether the EAS rediscovery is required for the application. - "true" indicates that the EAS rediscovery is required for the application. - "false" indicates that the EAS rediscovery is not required for the application. The indication shall be invalid after it was applied unless it is provided again. | EASDiscovery |
| notificationDestination | Link | O | 0..1 | Contains the Callback URL to receive the notification from the NEF. |  |
| eventReq | ReportingInformation | O | 0..1 | Indicates the event reporting requirements. This attribute may be provided if the "EDGEAPP" feature is supported. | EDGEAPP |
| NOTE 1: The value of the property shall be set to NULL for removal. NOTE 2: The "tfcCorrInd" attribute and the "tfcCorreInfo" attribute are mutually exclusive. NOTE 3: The value of the property shall be set to NULL for removal, and in that case, the default value "false" applies. |  |  |  |  |  |

##### 5.4.3.3.4	Type: EventNotification

Table 5.4.3.3.4-1: Definition of type EventNotification

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability (NOTE 1) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| afTransId |  | string |  | O |  | 0..1 |  | Identifies an NEF Northbound interface transaction, generated by the AF. |  |  |  |
| dnaiChgType |  | DnaiChangeType |  | M |  | 1 |  | Identifies the type of notification regarding UP path management event. |  |  |  |
| sourceTrafficRoute |  | RouteToLocation |  | O |  | 0..1 |  | Identifies the N6 traffic routing information associated to the source DNAI. May be present if the "subscribedEvent" sets to "UP_PATH_CHANGE". (NOTE 3) |  |  |  |
| subscribedEvent |  | SubscribedEvent |  | M |  | 1 |  | Identifies a UP path management event the AF requested to be notified of. |  |  |  |
| targetTrafficRoute |  | RouteToLocation |  | O |  | 0..1 |  | Identifies the N6 traffic routing information associated to the target DNAI. May be present if the "subscribedEvent" sets to "UP_PATH_CHANGE". (NOTE 3) |  |  |  |
| sourceDnai |  | Dnai |  | O |  | 0..1 |  | Source DN Access Identifier. Shall be included for event "UP_PATH_CHANGE" if the DNAI changed (NOTE 2, NOTE 3). |  |  |  |
| targetDnai |  | Dnai |  | O |  | 0..1 |  | Target DN Access Identifier. Shall be included for event "UP_PATH_CHANGE" if the DNAI changed (NOTE 2, NOTE 3). |  |  |  |
| candidateDnais |  | array(Dnai) |  | O |  | 1..N |  | The candidate DNAI(s) for the PDU Session. May be included for event "UP_PATH_CHANGE". |  | CommonEASDNAI |  |
| candDnaisPrioInd |  | boolean |  | O |  | 0..1 |  | If provided and set to "true", it indicates that the candidate DNAIs provided in the "candidateDnais" attribute are in descending priority order, i.e. the lower the array index the higher the priority of the respective DNAI. If omitted, the default value is "false". It may only be provided if the "candidateDnais" attribute is provided and the "dnaiChgType" attribute is set to the value "EARLY". |  | CommonEASDNAI |  |
| easRediscoverInd |  | boolean |  | O |  | 0..1 |  | Indication of EAS re-discovery. If present and set to "true", it indicates the EAS re-discovery is performed, e.g. due to change of common EAS. Default value is "false" if omitted. May be included for event "UP_PATH_CHANGE". |  | CommonEASDNAI |  |
| gpsi |  | Gpsi |  | O |  | 0..1 |  | Identifies a user. |  |  |  |
| srcUeIpv4Addr |  | Ipv4Addr |  | O |  | 0..1 |  | The IPv4 Address of the served UE for the source DNAI. |  |  |  |
| srcUeIpv6Prefix |  | Ipv6Prefix |  | O |  | 0..1 |  | The Ipv6 Address Prefix of the served UE for the source DNAI. |  |  |  |
| tgtUeIpv4Addr |  | Ipv4Addr |  | O |  | 0..1 |  | The IPv4 Address of the served UE for the target DNAI. |  |  |  |
| tgtUeIpv6Prefix |  | Ipv6Prefix |  | O |  | 0..1 |  | The Ipv6 Address Prefix of the served UE for the target DNAI. |  |  |  |
| ueMac |  | MacAddr48 |  | O |  | 0..1 |  | UE MAC address of the served UE. |  |  |  |
| afAckUri |  | Link |  | O |  | 0..1 |  | The URI provided by the NEF for the AF acknowledgement. May only be included for event "UP_PATH_CHANGE". |  | URLLC |  |
| offloadPlmnId |  | PlmnId |  | O |  | 0..1 |  | If the event is "UP_PATH_CHANGE" and the UE has moved to a serving PLMN where local traffic offloading is allowed, this attribute contains the identifier of the serving PLMN. |  | HR-SBO |  |
| hDnn |  | Dnn |  | O |  | 0..1 |  | If the event is "UP_PATH_CHANGE" and the UE has moved to a serving PLMN where local traffic offloading is allowed, this attribute contains the DNN at the Home PLMN. |  | HR-SBO |  |
| hSnssai |  | Snssai |  | O |  | 0..1 |  | If the the event is "UP_PATH_CHANGE" and UE has moved to a serving PLMN where local traffic offloading is allowed, this attribute contains the S-NSSAI at the Home PLMN. |  | HR-SBO |  |
| suppFeatures |  | SupportedFeatures |  | C |  | 0..1 |  | List of Traffic Influence negotiated features as described in clause 5.4.4. This parameter shall be supplied by the NEF when it determines that it needs to update the supported features that were negotiated during the resource creation due to a notification received by the SMF which contained updated supported features on the SMF side. |  | HR-SBO |  |
| NOTE 1: Properties marked with a feature as defined in clause 5.4.4 are applicable as described in clause 5.2.7 of 3GPP TS 29.122 [4]. If no feature is indicated, the related property applies for all the features. NOTE 2: If the DNAI is not changed while the N6 traffic routing information is changed, the "sourceDnai" attribute and "targetDnai" attribute shall not be provided. NOTE 3: The change from the UP path status where no DNAI applies to a status where a DNAI applies indicates the activation of the related AF request and therefore only the target DNAI and N6 traffic routing information is provided in the event notification; the change from the UP path status where a DNAI applies to a status where no DNAI applies indicates the de-activation of the related AF request and therefore only the source DNAI and N6 traffic routing information is provided in the event notification. |  |  |  |  |  |  |  |  |  |  |  |

##### 5.4.3.3.5	Type: AfResultInfo

Table 5.4.3.3.5-1: Definition of type AfResultInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afStatus | AfResultStatus | M | 1 | Identifies the result of the application relocation. |  |
| trafficRoute | RouteToLocation | O | 0..1 | Identifies the N6 traffic routing information associated to the target DNAI. May only be present if the "afStatus" sets to "SUCCESS". |  |
| upBuffInd | boolean | O | 0..1 | Indicates whether buffering of uplink traffic to the target DNAI is needed. - "true" indicates that buffering of uplink traffic to the target DNAI is needed. - "false" indicates that buffering of uplink traffic to the target DNAI is not needed. - Default value is "false" if omitted. This attribute may only be present if the "afStatus" is set to "SUCCESS". | ULBuffering |
| easIpReplaceInfos | array(EasIpReplacementInfo) | O | 1..N | Contains EAS IP replacement information. | EASIPreplacement |

##### 5.4.3.3.6	Type AfAckInfo

Table 5.4.3.3.6-1: Definition of type AfAckInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afTransId | string | C | 0..1 | Identifies an NEF Northbound interface transaction, generated by the AF. It shall be provided if the AF has previously provided it. |  |
| ackResult | AfResultInfo | M | 1 | Identifies the result of application layer handling. |  |
| gpsi | Gpsi | O | 0..1 | Identifies a GPSI. |  |

##### 5.4.3.3.7	Type TrafficDataSet

Table 5.4.3.3.7-1: Definition of type TrafficDataSet

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| setId | string | M | 1 | Identifies the traffic data set. |  |
| trafficFilters | array(FlowInfo) | C | 1..N | Contains IP packet filters. (NOTE) |  |
| ethTrafficFilters | array(EthFlowDescription) | C | 1..N | Contains Ethernet packet filters. (NOTE) |  |
| trafficRoutes | array(RouteToLocation) | M | 1..N | Contains the N6 traffic routing requirements. |  |
| NOTE: These attributes are mutually exclusive. Either one of them shall be present. |  |  |  |  |  |

##### 5.4.3.3.8	Type TrafficDataSetRm

This data type is defined in the same way as the "TrafficDataSet" data type, but:

-	with the OpenAPI "nullable: true" property.

-	the removable attributes "trafficFilters" and "ethTrafficFilters" and "trafficRoutes" are defined as nullable in the OpenAPI.

Table 5.4.3.3.8-1: Definition of type TrafficDataSetRm

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| setId | string | M | 1 | Identifies the traffic data set. |  |
| trafficFilters | array(FlowInfo) | O | 1..N | Contains IP packet filters. |  |
| ethTrafficFilters | array(EthFlowDescription) | O | 1..N | Contains Ethernet packet filters. |  |
| trafficRoutes | array(RouteToLocation) | O | 1..N | Contains the N6 traffic routing requirements. |  |

#### 5.4.3.4	Simple data types and enumerations

##### 5.4.3.4.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.4.3.4.2	Simple data types

The simple data types defined in table 5.4.3.4.2-1 shall be supported.

Table 5.4.3.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 5.4.3.4.3	Enumeration: SubscribedEvent

The enumeration SubscribedEvent represents the type of UP path management events for which the AF requests to be notified. It shall comply with the provisions defined in table 5.4.3.4.3-1.

Table 5.4.3.4.3-1: Enumeration SubscribedEvent

| Enumeration value | Description |
| --- | --- |
| UP_PATH_CHANGE | The AF requests to be notified when the UP path changes for the PDU session. |

##### 5.4.3.4.4	Enumeration: AfResultStatus

The enumeration AfResultStatus represents the status of application handling result. It shall comply with the provisions defined in table 5.4.3.4.4-1.

Table 5.4.3.4.4-1: Enumeration AfResultStatus

| Enumeration value | Description |
| --- | --- |
| SUCCESS | The application layer is ready or the relocation is completed. |
| TEMP_CONGESTION | The application relocation fails due to temporary congestion. |
| RELOC_NO_ALLOWED | The application relocation fails because application relocation is not allowed. |
| OTHER | The application relocation fails due to other reason. |

### 5.4.4	Used Features

The table below defines the features applicable to the TrafficInfluence API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.4.4-1: Features used by TrafficInfluence API

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_websocket | The delivery of notifications over Websocket is supported as described in 3GPP TS 29.122 [4]. This feature requires that the Notification_test_event feature is also supported. |
| 2 | Notification_test_event | The testing of notification connection is supported as described in 3GPP TS 29.122 [4]. |
| 3 | URLLC | This feature indicates support of Ultra Reliable Low Latency Communication (URLLC) requirements (i.e. AF application relocation acknowledgement and UE address(es) preservation). |
| 4 | MacAddressRange | Indicates the support of a set of MAC addresses with a specific range in the traffic filter. |
| 5 | AF_latency | This feature indicates support for Edge relocation considering user plane latency. |
| 6 | EASDiscovery | This feature indicates the support of EAS (re)discovery. |
| 7 | EASIPreplacement | This feature indicates the support of provisioning of EAS IP replacement info. |
| 8 | ExposureToEAS | This feature indicates support for the indication provided by the AF of direct event notification of QoS monitoring events from the UPF to the Local NEF or the AF in 5GC. |
| 9 | SimultConnectivity | This feature indicates support of temporary simultaneous connectivity over source and target PSA at edge relocation. |
| 10 | ULBuffering | This feature indicates support for Uplink buffering indication for edge relocation. |
| 11 | EDGEAPP | This feature controls the support of EDGE applications related functionalities (e.g. support the provisioning of event reporting requirements). |
| 12 | SFC | This feature indicates support for application function influence on service function chaining(s). |
| 13 | FinerGranUEs | This feature indicates support for handling of more granular set of UEs. |
| 14 | CommonEASDNAI | This feature controls the support of the common EAS/DNAI selection. |
| 15 | HR-SBO | This feature indicates the support of HR-SBO scenarios. |
| 16 | MultiTrafficInflu | This feature indicates the support for providing more than one set of traffic filters and the corresponding N6 traffic routing requirements for traffic influence. |
| Feature: A short name that can be used to refer to the bit and to the feature, e.g. "Notification". Description: A clear textual description of the feature. |  |  |

### 5.4.5	Error handling

#### 5.4.5.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.4.5.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the TrafficInfluence API.

#### 5.4.5.3	Application Errors

The application errors defined for TrafficInfluence API are listed in table 5.4.5.3-1.

Table 5.4.5.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

## 5.5	NiddConfigurationTrigger API

### 5.5.0	Introduction

The Nnef_NiddConfigurationTrigger service shall use the NiddConfigurationTrigger API.

The API URI of NiddConfigurationTrigger API shall be:

- {apiRoot}/3gpp-nidd-configuration-trigger/v1

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-nidd-configuration-trigger".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.5.1	Resources

There is no resource defined for this API in this release of the specification.

### 5.5.1A	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.5.2	Notifications

#### 5.5.2.1	Introduction

Upon receipt of a NIDD connection establishment request from the SMF and there is no NIDD configuration for the UE, the NEF may send an HTTP POST message in order to trigger the AF to start the NIDD configuration procedure as described in clause 5.6.3.2.3.4 of 3GPP TS 29.122 [4].

Table 5.5.2.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Event Notification | {notificationUri} | POST | Request for the NIDD Configuration Trigger |

#### 5.5.2.2	Event Notification

The Callback URI "{notificationUri}" shall be used with the callback URI variables defined in table 5.5.2.2-1.

Table 5.5.2.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notificationUri | Link | A URI indicating the notification destination where N33 notification requests shall be delivered to. This URI shall be preconfigured in the NEF. |

#### 5.5.2.3	Operation Definition

##### 5.5.2.3.1	Notification via HTTP POST

This method shall support the request data structures specified in table 5.5.2.3.1-1 and the response data structures and response codes specified in table 5.5.2.3.1-2.

Table 5.5.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NiddConfigurationTrigger | M | 1 | The NIDD Configuration Trigger is provided by the NEF to the AF. |

Table 5.5.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NiddConfigurationTriggerReply | M | 1 | 200 OK | The trigger is received successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during Configuration Trigger. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during Configuration Trigger. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Tabl  5.5.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

Table 5.5.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

##### 5.5.2.3.2	Notification via Websocket

Not specified in the present specification.

### 5.5.3	Data Model

#### 5.5.3.1	General

This clause specifies the application data model supported by the NiddConfigurationTrigger API.

#### 5.5.3.2	Reused data types

The data types reused by the NiddConfigurationTrigger API from other specifications are listed in table 5.5.3.2-1.

Table 5.5.3.2-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.5.4-1. |

#### 5.5.3.3	Structured data types

##### 5.5.3.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.5.3.3.2	Type: NiddConfigurationTrigger

This type represents a NIDD configuration trigger which is sent from the NEF to the AF.

Table 5.5.3.3.2-1: Definition of type NiddConfigurationTrigger

| Attribute name | Data type | P | Cardinality | Description | Applicability (NOTE) |
| --- | --- | --- | --- | --- | --- |
| afId | string | M | 1 | Identifies the trigger receiving entity. |  |
| nefId | string | M | 1 | Identifies the trigger sending entity. |  |
| gpsi | Gpsi | M | 1 | Identifies a user. |  |
| suppFeat | SupportedFeatures | M | 1 | Indicates the list of Supported features used as described in clause 5.5.4. |  |
| NOTE: Properties marked with a feature as defined in clause 5.5.4 are applicable as described in clause 5.2.7 of 3GPP TS 29.122 [4]. If no feature is indicated, the related property applies for all the features. |  |  |  |  |  |

##### 5.5.3.3.3	Type: NiddConfigurationTriggerReply

This data type represents a reply to a NIDD configuration trigger and is sent from the AF to the NEF.

Table 5.5.3.3.3-1: Definition of type NiddConfigurationTriggerReply

| Attribute name | Data type | P | Cardinality | Description | Applicability (NOTE) |
| --- | --- | --- | --- | --- | --- |
| suppFeat | SupportedFeatures | M | 1 | Indicates the list of Supported features used as described in clause 5.5.4. |  |
| NOTE: Properties marked with a feature as defined in clause 5.5.4 are applicable as described in clause 5.2.7 of 3GPP TS 29.122 [4]. If no feature is indicated, the related property applies for all the features. |  |  |  |  |  |

#### 5.5.3.4	Simple data types and enumerations

##### 5.5.3.4.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.5.3.4.2	Simple data types

The simple data types defined in table 5.5.3.4.2-1 shall be supported.

Table 5.5.3.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

### 5.5.4	Used Features

The table below defines the features applicable to the NiddConfigurationTrigger API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.5.4-1: Features used by NiddConfigurationTrigger API

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 5.5.5	Error handling

#### 5.5.5.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.5.5.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the NiddConfigurationTrigger API.

#### 5.5.5.3	Application Errors

The application errors defined for NiddConfigurationTrigger API are listed in table 5.5.5.3-1.

Table 5.5.5.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

## 5.6	AnalyticsExposure API

### 5.6.1	Resources

### 5.6.0	Introduction

The Nnef_AnalyticsExposure service shall use the AnalyticsExposure API.

The API URI of AnalyticsExposure API shall be:

- {apiRoot}/3gpp-analyticsexposure/v1

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-analyticsexposure".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

All resource URIs in the clauses below are defined relative to the above API URI.

#### 5.6.1.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.6.1.1-1 and the resources and HTTP methods used for the AnalyticsExposure API.

Figure 5.6.1.1-1: Resource URI structure of the AnalyticsExposure API

Table 5.6.1.1-1 provides an overview of the resources and HTTP methods applicable for the AnalyticsExposure API.

Table 5.6.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Analytics Exposure Subscriptions | /{afId}/subscriptions | GET | Retrieve all the subscriptions to analytics exposure for a given AF. |
|  |  | POST | Create a new subscription to analytics exposure. |
| Individual Analytics Exposure Subscription | /{afId}/subscriptions /{subscriptionId} | GET | Retrieve an existing subscription to analytics exposure. |
|  |  | PUT | Update an existing subscription to analytics exposure. |
|  |  | DELETE | Delete an existing subscription to analytics exposure. |

#### 5.6.1.2	Resource: Analytics Exposure Subscriptions

##### 5.6.1.2.1	Introduction

This resource allows a AF to read all active analytics exposure subscribtions for the given AF, or allows a AF to create a new subscription to retrieve analytics information.

##### 5.6.1.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-analyticsexposure/v1/{afId}/subscriptions

This resource shall support the resource URI variables defined in table 5.6.1.2.2-1.

Table 5.6.1.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |

##### 5.6.1.2.3	Resource Methods

###### 5.6.1.2.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.6.1.2.2.

###### 5.6.1.2.3.2	GET

The GET method allows to read all active subscriptions for a given AF. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.6.1.2.3.2-1.

Table 5.6.1.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supp-feat | SupportedFeatures | O | 0..1 | The features supported by the NF service consumer. |

This method shall support the request data structures specified in table 5.6.1.2.3.2-2 and the response data structures and response codes specified in table 5.6.1.2.3.2-3.

Table 5.6.1.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.6.1.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(AnalyticsExposureSubsc) | M | 0..N | 200 OK | The subscription information for the AF in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| ProblemDetails | O | 0..1 | 404 Not Found | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure causes are described in clause 5.6.5. |  |  |  |  |

Table 5.6.1.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.6.1.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.6.1.2.3.3	POST

The POST method creates a new subscription resource to analytics exposure subscription for a given AF. The AF shall initiate the HTTP POST request message and the NEF shall respond to the message. The NEF shall construct the URI of the created resource.

This method shall support the request data structures specified in table 5.6.1.2.3.3-1 and the response data structures and response codes specified in table 5.6.1.2.3.3-2.

Table 5.6.1.2.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AnalyticsExposureSubsc | M | 1 | Parameters to request a subscription to retrieve analytics information with the NEF. |

Table 5.6.1.2.3.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AnalyticsExposureSubsc | M | 1 | 201 Created | The subscription was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| ProblemDetails | O | 0..1 | 400 Bad Request | (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| ProblemDetails | O | 0..1 | 404 Not Found | (NOTE 2) |
| ProblemDetails | O | 0..1 | 500 Internal Server Error | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure causes are described in clause 5.6.5. |  |  |  |  |

Table 5.6.1.2.3.3-3: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-analyticsexposure/v1/{afId}/subscriptions/{subscriptionId} |

#### 5.6.1.3	Resource: Individual Analytics Exposure Subscription

##### 5.6.1.3.1	Introduction

This resource allows a AF to read/modify/cancel a subscription to retrieve analytics information with the NEF.

##### 5.6.1.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-analyticsexposure/v1/{afId}/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 5.6.1.3.2-1.

Table 5.6.1.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |
| subscriptionId | string | Identifier of the subscription resource. |

##### 5.6.1.3.3	Resource Methods

###### 5.6.1.3.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.6.1.3.2.

###### 5.6.1.3.3.2	GET

The GET method allows to read the active subscription for a given AF and subscription Id. The AF shall initiate the HTTP GET request message and theNEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.6.1.3.3.2-1.

Table 5.6.1.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supp-feat | SupportedFeatures | O | 0..1 | The features supported by the NF service consumer. |

This method shall support the request data structures specified in table 5.6.1.3.3.2-2 and the response data structures and response codes specified in table 5.6.1.3.3.2-3.

Table 5.6.1.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.6.1.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AnalyticsExposureSubsc | M | 1 | 200 OK | The subscription information for the AF in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.6.1.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.6.1.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.6.1.3.3.3	PUT

The PUT method modifies an existing subscription resource to update a subscription. The AF shall initiate the HTTP PUT request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.6.1.3.3.3-1 and the response data structures and response codes specified in table 5.6.1.3.3.3-2.

Table 5.6.1.3.3.3-1: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AnalyticsExposureSubsc | M | 1 | Modify an existing subscription to retrieve analytics information with the NEF. |

Table 5.6.1.3.3.3-2: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AnalyticsExposureSubsc | M | 1 | 200 OK | The subscription was updated successfully. |
| N/A |  |  | 204 No Content | The subscription was updated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| ProblemDetails | O | 0..1 | 400 Bad Request | (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| ProblemDetails | O | 0..1 | 500 Internal Server Error | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure causes are described in clause 5.6.5. |  |  |  |  |

Table 5.6.1.3.3.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.6.1.3.3.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.6.1.3.3.4	DELETE

The DELETE method deletes the analytics exposure subscription for a given AF. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.6.1.3.3.4-1.

Table 5.6.1.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.6.1.3.3.4-2 and the response data structures and response codes specified in table 5.6.1.3.3.4-3.

Table 5.6.1.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.6.1.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The subscription was terminated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.6.1.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.6.1.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

### 5.6.1A	Custom Operations without associated resources

#### 5.6.1A.1	Overview

Custom operations used for this API are summarized in table 5.6.1A.1-1. "apiRoot" is set as described in clause 5.2.4 of 3GPP TS 29.122 [4].

Table 5.6.1A.1-1: Custom operations without associated resources

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| fetch | {apiRoot}/3gpp-analyticsexposure/v1/{afId}/fetch | POST | Request to fetch analytics information |

#### 5.6.1A.2	Operation: fetch

##### 5.6.1A.2.1	Description

The custom operation allows a service consumer to fetch analytics information via the NEF.

##### 5.6.1A.2.2	Operation Definition

This operation shall support the response data structures and response codes specified in tables 5.6.1A.2.2-1 and 5.6.1A.2.2-2.

Table 5.6.1A.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AnalyticsRequest | M | 1 | Parameters to request to fetch analytics information. |

Table 5.6.1A.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AnalyticsData | M | 1 | 200 OK | The requested analytics information was returned successfully. |
| n/a |  |  | 204 No Content | If the request Analytics data does not exist, the NEF shall respond with "204 No Content". |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during analytics information retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during analytics information retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| ProblemDetails | O | 0..1 | 400 Bad Request | (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| ProblemDetails | O | 0..1 | 500 Internal Server Error | (NOTE 2) |
| ProblemDetailsAnalyticsInfoRequest | O | 0..1 | 500 Internal Server Error | The request is rejected by the NEF and more details (not only the ProblemDetails) may be returned. (NOTE 2) |
| NOTE 1: The manadatory HTTP error status codes for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure causes are described in clause 5.6.5. |  |  |  |  |

Table 5.6.1A.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.6.1A.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

### 5.6.2	Notifications

#### 5.6.2.1	Introduction

Upon receipt of analytics information notification from the NWDAF indicating the subscribed analytics event is detected, the NEF shall send an HTTP POST message including the notified analytics event to the AF. The NEF and the AF shall support the notification mechanism as described in clause 5.2.5 of 3GPP TS 29.122 [4].

Table 5.6.2.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Event Notification | {notifUri} | POST | The analytics event notification is provided by the NEF to the AF. |

#### 5.6.2.2	Event Notification

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 5.6.2.2-1.

Table 5.6.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | Callback reference provided by the AF during creation/modification of the subscription within the AnalyticsExposureSubsc data type as defined in Table 5.6.3.3.2-1. |

#### 5.6.2.3	Operation Definition

##### 5.6.2.3.1	Notification via HTTP POST

This method shall support the request data structures specified in table 5.6.2.3.1-1 and the response data structures and response codes specified in table 5.6.2.3.1-2.

Table 5.6.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AnalyticsEventNotification | M | 1 | The analytics event notification is provided by the NEF to the AF. |

Table 5.6.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The event notification is received successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during event notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during event notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.6.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

Table 5.6.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

##### 5.6.2.3.2	Notification via Websocket

If supported by both AF and NEF and successfully negotiated, the AnalyticsEventNotification may alternatively be delivered through the Websocket mechanism as defined in clause 5.2.5.4 of 3GPP TS 29.122 [4].

### 5.6.3	Data Model

#### 5.6.3.1	General

This clause specifies the application data model supported by the AnalyticsExposure API.

Table 5.6.3.1-1 specifies the data types defined for the AnalyticsExposure API.

Table 5.6.3.1-1: AnalyticsExposure API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AbnormalExposure | 5.6.3.3.15 | Represents a user's abnormal behavior information. | Abnormal_Behavior |
| AnalyticsData | 5.6.3.3.14 | Represents analytics data. |  |
| AnalyticsEvent | 5.6.3.4.3 | Event that is subscribed. |  |
| AnalyticsEventFilter | 5.6.3.3.13 | Represents analytics event filter information. |  |
| AnalyticsEventFilterSubsc | 5.6.3.3.6 | Represents an analytics event filter. |  |
| AnalyticsEventNotif | 5.6.3.3.4 | Represents an analytics event to be reported. |  |
| AnalyticsEventNotification | 5.6.3.3.3 | Represents an analytics event(s) notification. |  |
| AnalyticsEventSubsc | 5.6.3.3.5 | Represents a subscribed analytics event. |  |
| AnalyticsExposureSubsc | 5.6.3.3.2 | Represents an analytics exposure subscription. |  |
| AnalyticsFailureCode | 5.6.3.4.4 | Identifies the failure reason. |  |
| AnalyticsFailureEventInfo | 5.6.3.3.20 | Represents an event for which the subscription request was not successful and including the associated failure reason. |  |
| AnalyticsRequest | 5.6.3.3.12 | Represents the parameters to request to retrieve analytics information. |  |
| CongestInfo | 5.6.3.3.16 | Represents a UE's user data congestion information. | Congestion |
| CongestionAnalytics | 5.6.3.3.17 | Represents data congestion analytics for transfer over the user plane, control plane or both. |  |
| NetworkPerfExposure | 5.6.3.3.19 | Represents network performance information. | Network_Performance |
| QosSustainabilityExposure | 5.6.3.3.18 | Represents a QoS sustainability information. | QoS_Sustainability |
| TargetUeId | 5.6.3.3.7 | Represents the target UE(s) information. |  |
| UeLocationInfo | 5.6.3.3.10 | Represents a UE location information. |  |
| UeMobilityExposure | 5.6.3.3.9 | Represents a UE mobility information. | Ue_Mobility |
| WlanPerformInfo | 5.6.3.3.21 | Prpresents WLAN performance information | WlanPerformance_AIML |

#### 5.6.3.2	Reused data types

The data types reused by the AnalyticsExposure API from other specifications are listed in table 5.6.3.2-1.

Table 5.6.3.2-1: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AccuracyInfo | 3GPP TS 29.520 [27] | The analytics accuracy information. | AnalyticsAccuracy |
| AccuracyReq | 3GPP TS 29.520 [27] | Represents the analytics accuracy requirement information. | AnalyticsAccuracy |
| AdditionalMeasurement | 3GPP TS 29.520 [27] | Represents additional measurement information. |  |
| AddrFqdn | 3GPP TS 29.517 [58] | Represents an IP address and/or an FQDN. | ServiceExperience DnPerformance |
| AnalyticsFeedbackInfo | 3GPP TS 29.520 [27] | Represents analytics feedback information. |  |
| AnalyticsSubset | 3GPP TS 29.520 [27] | Represents an analytics Subset used to indicate the content of the analytics. | EneNA |
| ApplicationId | 3GPP TS 29.571 [8] | Contains the application identifier. | Abnormal_Behavior Ue_Communication Dispersion DnPerformance ServiceExperience E2eDataVolTransTime NSLoad |
| BitRate | 3GPP TS 29.571 [8] | Represents a bit rate. |  |
| BwRequirement | 3GPP TS 29.520 [27] | Represents bandwidth requirements. | ServiceExperience |
| CongestionType | 3GPP TS 29.520 [27] | Represents a congestion analytics type. |  |
| DateTime | 3GPP TS 29.122 [4] | Represents a date and a time. |  |
| DirectionInfo | 3GPP TS 29.520 [27] | Represents the UE direction information | UeMobilityExt_AIML |
| DispersionInfo | 3GPP TS 29.520 [27] | Dispersion information. | Dispersion |
| DispersionRequirement | 3GPP TS 29.520 [27] | Dispersion requirement. | Dispersion |
| Dnai | 3GPP TS 29.571 [8] | Identifies a user plane access to one or more DN(s). | DnPerformance ServiceExperience E2eDataVolTransTime |
| Dnn | 3GPP TS 29.571 [8] | Represents a DNN. | Ue_Communication Abnormal_Behavior DnPerformance ServiceExperience E2eDataVolTransTime |
| DnPerfInfo | 3GPP TS 29.520 [27] | DN Performance information. | DnPerformance |
| DnPerformanceReq | 3GPP TS 29.520 [27] | DN Performance requirement. | DnPerformance |
| DurationSec | 3GPP TS 29.122 [4] | Seconds of duration. | EneNA NetworkPerfExt_eNA Ue_MobilityExt_eNA CongestionExt_eNA QoS_SustainabilityExt_eNA DispersionExt_eNA DnPerformanceExt_eNA |
| EventReportingRequirement | 3GPP TS 29.520 [27] | Represents the required type of reporting. |  |
| ExternalGroupId | 3GPP TS 29.122 [4] | External Group Identifier for a user group. | Abnormal_BehaviorUe_Mobility Ue_Communication Network_Performance Dispersion DnPerformance ServiceExperience |
| Exception | 3GPP TS 29.520 [27] | Represents exception information. |  |
| ExceptionId | 3GPP TS 29.520 [27] | Represents the identifier of an exception. | Abnormal_Behavior |
| ExpectedAnalyticsType | 3GPP TS 29.520 [27] | Represents the exception's trend. | Abnormal_Behavior |
| ExpectedUeBehaviourData | 3GPP TS 29.503 [17] | Represents the expected UE behaviour data. | Abnormal_Behavior |
| Float | 3GPP TS 29.571 [8] | Represents a number with the "float" format. |  |
| GeoDistributionInfo | 3GPP TS 29.520 [27] | Represents the geographical distribution of the UEs. | UeMobilityExt_AIML |
| GeographicalArea | 5.17.3.3.4 | Identifies the geographical information with shapes. | ServiceExperienceExt_eNA Ue_MobilityExt_eNA QoS_SustainabilityExt_eNA |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. | Abnormal_Behavior Congestion Ue_Mobility Ue_Communication Network_Performance Dispersion DnPerformance ServiceExperience |
| LocationArea5G | 3GPP TS 29.122 [4] | Represents a user location area when the UE is attached to 5G. | Abnormal_BehaviorExt_eNA DnPerformanceExt_eNA ServiceExperienceExt_eNA UeCommunicationExt_eNA E2eDataVolTransTime NSLoad |
| LocInfoGranularity | 3GPP TS 29.520 [27] | Represents the preferred granularity of location information. | ServiceExperienceExt_eNA Ue_MobilityExt_eNA DispersionExt_eNA |
| MatchingDirection | 3GPP TS 29.520 [27] | Matching direction | QoS_Sustainability Congestion, Network_Performance NSLoad |
| MovBehavInfo | 3GPP TS 29.520 [27] | Represents the Movement Behaviour information. | MovementBehaviour |
| MovBehavReq | 3GPP TS 29.520 [27] | Represents the Movement Behaviour analytics requirements. | MovementBehaviour |
| NetworkPerfRequirement | 3GPP TS 29.520 [27] | Represents a network performance requirement. | Network_Performance |
| NsiIdInfo | 3GPP TS 29.520 [27] | Represents the S-NSSAI and the optionally associated Network Slice Instance Identifier(s). | ServiceExperience DnPerformance NSLoad |
| NsiLoadLevelInfo | 3GPP TS 29.520 [27] | Represents the network slice load level information. | NSLoad |
| NwdafFailureCode | 3GPP TS 29.520 [27] | Identifies the analytics failure reason. |  |
| PduSessionInfo | 3GPP TS 29.520 [27] | Identifies combination of PDU Session parameters information. | ServiceExperienceExt_eNA |
| ProblemDetails | 3GPP TS 29.122 [4] | Represents error related information. |  |
| ProblemDetailsAnalyticsInfoRequest | 3GPP TS 29.520 [27] | Represents an extension to the ProblemDetails data structure with additional information on why the analytics request is rejected |  |
| QosRequirement | 3GPP TS 29.520 [27] | Represents QoS requirements. | QoS_Sustainability E2eDataVolTransTime |
| RatFreqInformation | 3GPP TS 29.520 [27] | Represents the RAT type and/or Frequency information. | ServiceExperience |
| RelProxInfo | 3GPP TS 29.520 [27] | Represents the Relative Proximity information. | RelProxInfo |
| RelProxReq | 3GPP TS 29.520 [27] | Represents the Relative Proximity analytics requirements. | RelProxReq |
| ReportingInformation | 3GPP TS 29.523 [22] | Describes the analytics reporting requirement information. |  |
| ResourceUsageRequirement | 3GPP TS 29.520 [27] |  | NetworkPerformanceExt_AIML |
| RetainabilityThreshold | 3GPP TS 29.520 [27] | Represents a QoS flow retainability threshold. | QoS_Sustainability |
| SamplingRatio | 3GPP TS 29.571 [8] | Indicates Sampling Ratio. |  |
| ScheduledCommunicationTime | 3GPP TS 29.122 [4] | Represents an offered scheduled communication time. |  |
| ServiceExperienceInfo | 3GPP TS 29.520 [27] | Represents the service experience information. | ServiceExperience |
| Snssai | 3GPP TS 29.571 [8] | Represents an S-NSSAI. | UeCommunication QoS_Sustainability Abnormal_Behavior Congestion Dispersion ServiceExperience DnPerformance E2eDataVolTransTime NSLoad |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features. |  |
| TermCause | 3GPP TS 29.520 [27] | Cause for requesting the termination of a subscription. | TermRequest |
| ThresholdLevel | 3GPP TS 29.520 [27] | Represents a threshold level. | Congestion |
| TimeWindow | 3GPP TS 29.122 [4] | Represents a time window. | NetworkPerfExt_eNA |
| TopApplication | 3GPP TS 29.520 [27] | Top application that contributes the most to the traffic. | CongestionExt |
| UeCommunication | 3GPP TS 29.520 [27] | Represents UE communication information. | Ue_Communication |
| UeCommReq | 3GPP TS 29.520 [27] | UE communication analytics requirement. | UeCommunicationExt_eNA |
| UeMobilityReq | 3GPP TS 29.520 [27] | UE mobility analytics requirement. | Ue_MobilityExt_eNA |
| Uinteger | 3GPP TS 29.571 [8] | Unsigned integer. |  |
| Uri | 3GPP TS 29.122 [4] | Identifies a referenced resource. |  |
| UserDataCongestReq | 3GPP TS 29.520 [27] | The User Data Congestion requirement. | CongestionExt_eNA |
| UserDataConOrderCrit | 3GPP TS 29.520 [27] | The ordering criterion for the list of User Data Congestion analytics. | CongestionExt_eNA |
| WlanPerformanceReq | 3GPP TS 29.520 [27] | Represents the WLAN performance analytics requirement. |  |
| WlanPerSsIdPerformanceInfo | 3GPP TS 29.520 [27] | WLAN performance information per SSID of WLAN access points deployed in the Area of Interest. |  |
| WlanPerUeIdPerformanceInfo | 3GPP TS 29.520 [27] | WLAN performance information per UE ID of WLAN access points deployed in the Area of Interest. |  |

#### 5.6.3.3	Structured data types

##### 5.6.3.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.6.3.3.2	Type: AnalyticsExposureSubsc

This type represents an analytics exposure subscription. The same structure is used in the subscription request and subscription response.

Table 5.6.3.3.2-1: Definition of type AnalyticsExposureSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability (NOTE 1) |
| --- | --- | --- | --- | --- | --- |
| analyEventsSubs | array(AnalyticsEventSubsc) | M | 1..N | Subscribed analytics events. |  |
| analyRepInfo | ReportingInformation | O | 0..1 | Reporting requirement information of the subscription. If omitted, the default values within the ReportingInformation data type apply. (NOTE 2) |  |
| notifUri | Uri | M | 1 | Notification URI for analytics event reporting. |  |
| notifId | string | M | 1 | Notification Correlation ID assigned by the NF service consumer. |  |
| eventNotifis | array(AnalyticsEventNotif) | C | 1..N | Represents the Events to be reported. Shall only be present if the immediate reporting indication in the "immRep" attribute within the "analyRepInfo" attribute sets to true during the event subscription, and the reports are available. |  |
| failEventReports | array(AnalyticsFailureEventInfo) | O | 1..N | Supplied by the NWDAF. When available, shall contain the event(s) for which the subscription is not successful, including the failure reason(s). |  |
| suppFeat | SupportedFeatures | C | 0..1 | Indicates the list of Supported features used as described in clause 5.6.4. This attribute shall be provided in the POST request and in the response of successful resource creation, or in the HTTP GET response if the "supp-feat" attribute query parameter is included in the HTTP GET request. |  |
| self | Link | C | 0..1 | Identifies the Individual Analytics Exposure Subscription resource. Shall be present in the HTTP GET response when reading all the subscriptions for an AF. |  |
| requestTestNotification | boolean | O | 0..1 | Set to true by the AF to request the NEF to send a test notification as defined in clause 5.2.5.3 of 3GPP TS 29.122 [4]. The default value is "false" if omitted. | Notification_test_event |
| websockNotifConfig | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol. | Notification_websocket |
| NOTE 1: Properties marked with a feature as defined in clause 5.6.4 are applicable as described in clause 5.2.7 of 3GPP TS 29.122 [4]. If no feature is indicated, the related property applies for all the features. NOTE 2: The attributes "partitionCriteria" and "notifFlag" of the data type ReportingInformation are applicable only if the "EneNA" feature is supported. |  |  |  |  |  |

##### 5.6.3.3.3	Type: AnalyticsEventNotification

Table 5.6.3.3.3-1: Definition of type AnalyticsEventNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifId | string | M | 1 | Notification Correlation ID assigned by the NF service consumer. |  |
| analyEventNotifs | array(AnalyticsEventNotif) | M | 1..N | Represents the analytics events to be reported according to the subscription corresponding to the Notification Correlation ID. |  |
| termCause | TermCause | O | 0..1 | A cause for which the NEF will send no further notifications for this subscription. Its presence indicates that the NEF requests the termination of the subscription. | TermRequest |

##### 5.6.3.3.4	Type: AnalyticsEventNotif

Table 5.6.3.3.4-1: Definition of type AnalyticsEventNotif

| Attribute name |  | Data type |  |  | P | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| analyEvent |  | AnalyticsEvent |  |  | M | 1 |  | Detected analytics event. |  |  |  |
| expiry |  | DateTime |  |  | O | 0..1 |  | Defines the expiration time after which the analytics information will become invalid. (NOTE 2) |  |  |  |
| timeStamp |  | DateTime |  |  | M | 1 |  | Time at which the event is observed. |  |  |  |
| failNotifyCode |  | NwdafFailureCode |  |  | C | 0..1 |  | Identifies the failure reason for the event notification. It shall only be included if the event notification is failed or the analytics information is not ready. (NOTE 1) |  | EneNA |  |
| rvWaitTime |  | DurationSec |  |  | O | 0..1 |  | Indicates a recommended time interval (in seconds) which is used to determine the time when analytics information is needed in similar future event subscriptions. It may only be included if the "failNotifyCode" attribute sets to "UNSATISFIED_REQUESTED_ANALYTICS_TIME". |  | EneNA |  |
| ueMobilityInfos |  | array(UeMobilityExposure) |  |  | C | 1..N |  | Contains the UE mobility information. Shall be present if the "analyEvent" attribute sets to "UE_MOBILITY". (NOTE 7) |  | Ue_Mobility |  |
| ueCommInfos |  | array(UeCommunication) |  |  | C | 1..N |  | Contains the application communication information. Shall be present if the "analyEvent" attribute sets to "UE_COMM". (NOTE 5) (NOTE 7) |  | Ue_Communication |  |
| abnormalInfos |  | array(AbnormalExposure) |  |  | C | 1..N |  | Contains the user's abnormal behavior information. Shall be present if the "analyEvent" attribute sets to "ABNORMAL_BEHAVIOR". (NOTE 7) |  | Abnormal_Behavior |  |
| congestInfos |  | array(CongestInfo) |  |  | C | 1..N |  | Contains the UE's user data congestion information. Shall be present if the "analyEvent" attribute sets to "CONGESTION". (NOTE 7) |  | Congestion |  |
| nwPerfInfos |  | array(NetworkPerfExposure) |  |  | C | 1..N |  | Contains the network performance information. Shall be present if the "analyEvent" attribute is set to "NETWORK_PERFORMANCE". (NOTE 7) |  | Network_Performance |  |
| qosSustainInfos |  | array(QosSustainabilityExposure) |  |  | C | 1..N |  | Contains the QoS sustainability information. Shall be present if the "analyEvent" attribute is set to "QOS_SUSTAINABILITY". (NOTE 7) |  | QoS_Sustainability |  |
| disperInfos |  | array(DispersionInfo) |  |  | C | 1..N |  | Contains the Dispersion information. Shall be present if the "analyEvent" attribute is set to "DISPERSION". (NOTE 7) |  | Dispersion |  |
| dnPerfInfos |  | array(DnPerfInfo) |  |  | C | 1..N |  | Contains the DN performance information. Shall be present if the "analyEvent" attribute is set to "DN_PERFORMANCE". (NOTE 4) (NOTE 7) |  | DnPerformance |  |
| svcExps |  | array(ServiceExperienceInfo) |  |  | C | 1..N |  | Contains the service experience information. Shall be present if the "analyEvent" attribute is set to "SERVICE_EXPERIENCE". (NOTE 7) (NOTE 8) |  | ServiceExperience |  |
| timeStampGen |  | DateTime |  |  | O | 0..1 |  | It defines the timestamp of analytics generation. |  | EneNA |  |
| start |  | DateTime |  |  | O | 0..1 |  | It defines the start time of which the analytics information will become valid. (NOTE 2) |  | EneNA |  |
| locArea |  | LocationArea5G |  |  | O | 0..1 |  | Identification of locationarea to which the notification applies within the subscribed location area. (NOTE 3) |  | Abnormal_BehaviorExt_eNA DnPerformanceExt_eNA ServiceExperienceExt_eNA UeCommunicationExt_eNA E2eDataVolTransTime NSLoad |  |
| dataVlTrnsTmIfs |  | array(E2eDataVolTransTimeInfo) |  |  | C | 1..N |  | E2E data volume transfer time information. Shall be present if the subscribed event is "E2E_DATA_VOL_TRANS_TIME". (NOTE 7) |  | E2eDataVolTransTime |  |
| accuInfo |  | AccuracyInfo |  | C | 0..1 |  |  | The analytics accuracy information. It shall be provided when accuracyReq was provided in the subscription request and the "cancelAccuInd" attribute is set to "false" or omitted. |  | AnalyticsAccuracy |  |
| cancelAccuInd |  | boolean |  | O | 0..1 |  |  | Indicates cancelled subscription of the analytics accuracy information. Set to "true" indicates the NWDAF cancelled subscription of analytics accuracy information as the NWDAF does not support the accuracy checking capability. Otherwise set to "false". Default value is "false" if omitted. |  | AnalyticsAccuracy |  |
| movBehavInfos |  | array(MovBehavInfo) |  | C | 1..N |  |  | The Movement Behaviour information. Shall be present if the "analyEvent" attribute is set to "MOVEMENT_BEHAVIOUR". (NOTE 7) |  | MovementBehaviour |  |
| relProxInfos |  | array(RelProxInfo) |  | C | 1..N |  |  | The Relative Proximity information. Shall be present if the "analyEvent" attribute is set to "RELATIVE_PROXIMITY". The "supis" attribute inside the RelProxInfo data type is not applicable in this API and only the "gpsis" attribute can be used. (NOTE 7) |  | RelativeProximity |  |
| wlanInfos |  | array(WlanPerformInfo) |  | C | 1..N |  |  | The WLAN performance related information. Shall be present if the "analyEvent" attribute is set to "WLAN_PERFORMANCE". (NOTE 7) |  | WlanPerformance_AIML |  |
| pauseInd |  | boolean |  | O | 0..1 |  |  | Pause analytics consumption indication applicable on analytics ID level. Set to "true" to indicate the consumer to stop the consumption of the analytics because the accuracy level needs to be increased. Default value is "false" if omitted. |  | AnalyticsAccuracy |  |
| resumeInd |  | boolean |  | O | 0..1 |  |  | Resume analytics consumption indication applicable on analytics ID level. Set to "true" to indicate the consumer to resume the consumption of the analytics because the accuracy has been improved. Default value is "false" if omitted. |  | AnalyticsAccuracy |  |
| nsiLoadLevelData |  | array(NsiLoadLevelInfo) |  | C | 1..N |  |  | Contains the network slice load level analytics information for each S-NSSAI. This attribute shall be present if the subscribed event is "NS_LOAD_LEVEL". (NOTE 6) (NOTE 7) |  | NSLoad |  |
| NOTE 1: The values of "BOTH_STAT_PRED_NOT_ALLOWED" of the NwdafFailureCode data type is not applicable for the "failNotifyCode" attribute. The value of "UNAVAILABLE_DATA" of the NwdafFailureCode data type is applicable for the "failNotifyCode" attribute only when the "StatisticsFailure" feature is supported. NOTE 2: If the "start" attribute and the "expiry" attribute are both provided, the DateTime of the "expiry" attribute shall not be earlier than the DateTime of the "start" attribute. NOTE 3: The NetworkAreaInfo data type within the "locArea" attribute is not applicable for the untrusted AF unless the corresponding SLA is agreed between the operator and application provider. The NEF may translate the network area information (received from the NWDAF, e.g. for "ABNORMAL_BEHAVIOR", "DN_PERFORMANCE", "SERVICE_EXPERIENCE", "UE_COMM" or "NS_LOAD_LEVEL" event) to an external representation of the area, which is provided within the "locArea" attribute. NOTE 4: The "minTrafficRate", "aggTrafficRate", "varTrafficRate", "trafRateUeIds", "avePacketDelay", "maxPacketDelay", "varPacketDelay", "packDelayUeIds", "maxPacketLossRate", "varPacketLossRate" and "packetLossUeIds" attribute(s) within the DnPerfInfo data type is applicable only if the "DnPerformanceExt_AIML" feature is supported. NOTE 5: If the "UeMobilityExt_eNA" feature is supported and the "locGranularity" attribute value "LON_AND_LAT_LEVEL" is subscribed, the "geoLoc" attribute within the "UeMobility" type may be provided to report the geographical location (longitude and latitude level). NOTE 6: When the "NSLoad" feature is supported, the "nsiId" attribute and NSI related analytics information within the NsiLoadLevelInfo data structure is not applicable for the "NS_LOAD_LEVEL" event within each array element of this attribute. NOTE 7: If the "AnalyticsAccuracy" feature is supported and the notification is only for notifying about the accuracy information of subscribed events, this attribute is not required to be included even if the respective event was subscribed. NOTE 8: The "geoLoc" attribute within the ServiceExperienceInfo data type is applicable only if the "ServiceExperienceExt_eNA" feature is supported. |  |  |  |  |  |  |  |  |  |  |  |

##### 5.6.3.3.5	Type: AnalyticsEventSubsc

Table 5.6.3.3.5-1: Definition of type AnalyticsEventSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyEvent | AnalyticsEvent | M | 1 | Requested analytics event. |  |
| analyEventFilter | AnalyticsEventFilterSubsc | O | 0..1 | Represents analytics event filter. |  |
| tgtUe | TargetUeId | O | 0..1 | Identifies target UE information |  |

##### 5.6.3.3.6	Type: AnalyticsEventFilterSubsc

Table 5.6.3.3.6-1: Definition of type AnalyticsEventFilterSubsc

| Attribute name |  |  |  | Data type |  |  |  | P |  |  |  | Cardinality |  |  |  | Description |  |  |  | Applicability |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| locArea |  |  |  | LocationArea5G |  |  |  | O |  |  |  | 0..1 |  |  |  | Identification of network area to which the subscription applies. (NOTE 1) (NOTE 7) (NOTE 14) |  |  |  | Abnormal_Behavior Congestion Ue_Communication Ue_Mobility QoS_Sustainability Network_Performance Dispersion DnPerformance ServiceExperience E2eDataVolTransTime MovementBehaviour RelativeProximity |  |  |  |
| fineGranAreas |  |  |  | array(GeographicalArea) |  |  |  | O |  |  |  | 1..N |  |  |  | Indicates the fine granularity areas to which the subscription applies. (i.e. with a finer granularity than cell). (NOTE 1, NOTE 14) |  |  |  | ServiceExperienceExt_eNA Ue_MobilityExt_eNA QoS_SustainabilityExt_eNA |  |  |  |
| dnn |  |  |  | Dnn |  |  |  | O |  |  |  | 0..1 |  |  |  | Identifies the DNN. (NOTE 7) |  |  |  | Ue_Communication Abnormal_Behavior ServiceExperience DnPerformance RelativeProximity |  |  |  |
| dnns |  |  |  | array(Dnn) |  |  |  | O |  |  |  | 1..N |  |  |  | Identifies the DNN(s) to which the subscription applies. (NOTE 7) |  |  |  | UeCommunicationExt_eNA Abnormal_BehaviorExt_eNA ServiceExperienceExt_eNA DnPerformanceExt_eNA E2eDataVolTransTime |  |  |  |
| dnais |  |  |  | array(Dnai) |  |  |  | O |  |  |  | 1..N |  |  |  | Identification(s) of user plane access to DN(s) which the subscription applies. |  |  |  | DnPerformance ServiceExperience E2eDataVolTransTime |  |  |  |
| appIds |  |  |  | array(ApplicationId) |  |  |  | O |  |  |  | 1..N |  |  |  | Each element identifies an application. (NOTE 7) (NOTE 13) |  |  |  | Abnormal_Behavior Ue_Communication Dispersion DnPerformance ServiceExperience E2eDataVolTransTime NSLoad |  |  |  |
| dataVlTrnsTmRqs |  |  |  | array(E2eDataVolTransTimeReq) |  |  |  | O |  |  |  | 1..N |  |  |  | Represents the E2E data volume transfer time requirements |  |  |  | E2eDataVolTransTime |  |  |  |
| excepRequs |  |  |  | array(Exception) |  |  |  | O |  |  |  | 1..N |  |  |  | Represents a list of Exception Ids with associated thresholds. (NOTE 2, NOTE 3) |  |  |  | Abnormal_Behavior |  |  |  |
| exptAnaType |  |  |  | ExpectedAnalyticsType |  |  |  | O |  |  |  | 0..1 |  |  |  | Represents expected UE analytics type. (NOTE 3) |  |  |  | Abnormal_Behavior |  |  |  |
| exptUeBehav |  |  |  | ExpectedUeBehaviourData |  |  |  | O |  |  |  | 0..1 |  |  |  | Represents expected UE behaviour. |  |  |  | Abnormal_Behavior |  |  |  |
| matchingDir |  |  |  | MatchingDirection |  |  |  | O |  |  |  | 0..1 |  |  |  | A matching direction may be provided alongside a threshold. If omitted, the default value is CROSSED. |  |  |  | QoS_Sustainability Congestion, Network_Performance NSLoad |  |  |  |
| reptThlds |  |  |  | array(ThresholdLevel) |  |  |  | O |  |  |  | 1..N |  |  |  | Represents the levels to be reached in order to be notified by the NEF. (NOTE 4) |  |  |  | Congestion |  |  |  |
| nwPerfReqs |  |  |  | array(NetworkPerfRequirement) |  |  |  | C |  |  |  | 1..N |  |  |  | Represents the network performance requirements. This attribute shall be included when subscribed event is "NETWORK_PERFORMANCE". |  |  |  | Network_Performance |  |  |  |
| snssai |  |  |  | Snssai |  |  |  | O |  |  |  | 0..1 |  |  |  | Identifies the network slice information. (NOTE 7) |  |  |  | UeCommunication QoS_Sustainability Abnormal_Behavior Congestion Dispersion ServiceExperience DnPerformance |  |  |  |
| snssais |  |  |  | array(Snssai) |  |  |  | O |  |  |  | 1..N |  |  |  | Identifies the network slice information. (NOTE 7) |  |  |  | Ue_CommunicationExt_eNA QoS_SustainabilityExt_eNA Abnormal_BehaviorExt_eNA CongestionExt_eNA DispersionExt_eNA ServiceExperienceExt_eNA DnPerformanceExt_eNA E2eDataVolTransTime RelativeProximity |  |  |  |
| nsiIdInfos |  |  |  | array(NsiIdInfo) |  |  |  | O |  |  |  | 1..N |  |  |  | Each element identifies the S-NSSAI and the optionally associated network slice instance(s). May be included when subscribed event is "SERVICE_EXPERIENCE", "DN_PERFORMANCE" or "NS_LOAD_LEVEL". (NOTE 15) |  |  |  | ServiceExperience DnPerformance NSLoad |  |  |  |
| qosReq |  |  |  | QosRequirement |  |  |  | C |  |  |  | 0..1 |  |  |  | Represents the QoS requirements. This attribute shall be included when subscribed event is "QOS_SUSTAINABILITY". |  |  |  | QoS_Sustainability E2eDataVolTransTime |  |  |  |
| qosFlowRetThds |  |  |  | array(RetainabilityThreshold) |  |  |  | C |  |  |  | 1..N |  |  |  | Represents the QoS flow retainability thresholds, Shall be supplied for the 5QI of GBR resource type. (NOTE 5) |  |  |  | QoS_Sustainability |  |  |  |
| ranUeThrouThds |  |  |  | array(BitRate) |  |  |  | C |  |  |  | 1..N |  |  |  | Represents the RAN UE throughput thresholds. Shall be supplied for the 5QI of non-GBR resource type. (NOTE 5) |  |  |  | QoS_Sustainability |  |  |  |
| disperReqs |  |  |  | array(DispersionRequirement) |  |  |  | O |  |  |  | 1..N |  |  |  | Represents the dispersion analytics requirements. |  |  |  | Dispersion |  |  |  |
| dnPerfReqs |  |  |  | array(DnPerformanceReq) |  |  |  | O |  |  |  | 1..N |  |  |  | Represents the DN performance analytics requirements. |  |  |  | DnPerformance |  |  |  |
| bwRequs |  |  |  | array(BwRequirement) |  |  |  | O |  |  |  | 1..N |  |  |  | Represents the bandwidth requirement for each application. |  |  |  | ServiceExperience |  |  |  |
| ratFreqs |  |  |  | array(RatFreqInformation) |  |  |  | O |  |  |  | 1..N |  |  |  | Identification(s) of the RAT type and/or frequencies of UE’s serving cell(s) which the subscriptiont applies. (NOTE 8) |  |  |  | ServiceExperience |  |  |  |
| appServerAddrs |  |  |  | array(AddrFqdn) |  |  |  | C |  |  |  | 1..N |  |  |  | Each of the element represents the Application Server Instance (IP address/FQDN of the Application Server) (NOTE 9) |  |  |  | ServiceExperience DnPerformance |  |  |  |
| wlanReqs |  |  |  | array(WlanPerformanceReq) |  |  |  | O |  |  |  | 1..N |  |  |  | Represents WLAN performance analytics requirements. |  |  |  | WlanPerformance_AIML |  |  |  |
| listOfAnaSubsets |  |  |  | array(AnalyticsSubset) |  |  |  | O |  |  |  | 1..N |  |  |  | The list of analytics subsets can be used to indicate the content of the analytics. (NOTE 12) |  |  |  | EneNA |  |  |  |
| extraReportReq |  |  |  | EventReportingRequirement |  |  |  | O |  |  |  | 0..1 |  |  |  | The extra event reporting requirement information. (NOTE 6) |  |  |  |  |  |  |  |
| maxNumOfTopAppUl |  |  |  | Uinteger |  |  |  | O |  |  |  | 0..1 |  |  |  | Indicates the requested maximum number of top applications that contribute the most to the traffic in Uplink direction. Minimum = 1. May be included when one of the element in the "listOfAnaSubsets" attribute is set to LIST_OF_TOP_APP_UL. |  |  |  | CongestionExt |  |  |  |
| maxNumOfTopAppDl |  |  |  | Uinteger |  |  |  | O |  |  |  | 0..1 |  |  |  | Indicates the requested maximum number of top applications that contribute the most to the traffic in Downlink direction. Minimum = 1. May be included when one of the element in the "listOfAnaSubsets" attribute is set to LIST_OF_TOP_APP_DL. |  |  |  | CongestionExt |  |  |  |
| visitedLocAreas |  |  |  | array(LocationArea5G) |  |  |  | O |  |  |  | 1..N |  |  |  | Identifications of network areas which the UEs had previously been in at least one of the Visited Area(s) of Interest. (NOTE 10) |  |  |  | Ue_Mobility |  |  |  |
| ueCommReqs |  |  |  | array(UeCommReq) |  |  |  | O |  |  |  | 1..N |  |  |  | Represents the UE communication requirements. This attribute may be included when the subscribed event is "UE_COMM". |  |  |  | UeCommunicationExt_eNA |  |  |  |
| userDataConOrderCri |  |  |  | UserDataConOrderCrit |  |  |  | O |  |  |  | 0..1 |  |  |  | The ordering criterion for the list of User Data Congestion analytics. (NOTE 11) |  |  |  | CongestionExt_eNA |  |  |  |
| locGranularity |  |  |  | LocInfoGranularity |  |  |  | O |  |  |  | 0..1 |  |  |  | The preferred granularity of UE location information. |  |  |  | ServiceExperienceExt_eNA Ue_MobilityExt_eNA DispersionExt_eNA |  |  |  |
| locOrientation |  |  |  | LocationOrientation |  |  |  | O |  |  |  | 0..1 |  |  |  | Indicates the preferred orientation of location information. |  |  |  | Ue_MobilityExt_eNA MovementBehaviour |  |  |  |
| ueMobilityReqs |  |  |  | array(UeMobilityReq) |  |  |  | O |  |  |  | 1..N |  |  |  | Represents the UE mobility requirements. This attribute may be included when the subscribed event is "UE_MOBILITY". |  |  |  | Ue_MobilityExt_eNA |  |  |  |
| movBehavReqs |  |  |  | array(MovBehavReq) |  |  |  | O |  |  |  | 1..N |  |  |  | Represents the Movement Behaviour analytics requirements. |  |  |  | MovementBehaviour |  |  |  |
| relProxReqs |  |  |  | array(RelProxReq) |  |  |  | O |  |  |  | 1..N |  |  |  | Represents the Relative Proximity analytics requirements. |  |  |  | RelativeProximity |  |  |  |
| pduSesInfos |  |  |  | array(PduSessionInfo) |  |  |  | O |  |  |  | 1..N |  |  |  | Represents combination of PDU Session parameters information. (NOTE 13) |  |  |  | ServiceExperienceExt_eNA |  |  |  |
| useCaseCxt |  |  |  | string |  |  |  | O |  |  |  | 0..1 |  |  |  | Indicates the context of usage of the analytics. The value and format of this parameter are not standardized. |  |  |  | ENAExt |  |  |  |
| accuReq |  |  |  | AccuracyReq |  |  |  | O |  |  |  | 0..1 |  |  |  | Represents the analytics accuracy requirement information. |  |  |  | AnalyticsAccuracy |  |  |  |
| pauseFlg |  |  |  | boolean |  |  |  | O |  |  |  | 0..1 |  |  |  | Pause analytics consumption flag and is applicable on analytics ID level. Set to "true" to indicate the NWDAF to stop including analytics of this event type in its notifications (without cancelling the subscription), because the accuracy level needs to be increased. Default value is "false" if omitted. This attribute may be present in a update request message if the "pauseInd" attribute was provided in the notification. |  |  |  | AnalyticsAccuracy |  |  |  |
| resumeFlg |  |  |  | boolean |  |  |  | O |  |  |  | 0..1 |  |  |  | Resume analytics consumption flag and is applicable on analytics ID level. Set to "true" to indicate the NWDAF to resume sending the notifications of analytics because the accuracy has been improved. Default value is "false" if omitted. This attribute may be present in a update request message if the "resumeInd" attribute was provided in the notification. |  |  |  | AnalyticsAccuracy |  |  |  |
| temporalGranSize |  |  |  | DurationSec |  |  |  | O |  |  |  | 0..1 |  |  |  | Indicates the minimum duration of each time slot for which the analytics are provided. |  |  |  | NetworkPerfExt_eNA Ue_MobilityExt_eNA CongestionExt_eNA QoS_SustainabilityExt_eNA DispersionExt_eNA DnPerformanceExt_eNA |  |  |  |
| spatialGranSizeTa |  |  |  | Uinteger |  |  |  | O |  |  |  | 0..1 |  |  |  | Indicates the maximum number of TAs used to define an area for which the analytics are provided. May be included when the "networkArea" attribute in the EventSubscription data type is provided. |  |  |  | NetworkPerfExt_eNA Ue_MobilityExt_eNA UeCommunicationExt_eNA QoS_SustainabilityExt_eNA DispersionExt_eNA DnPerformanceExt_eNA |  |  |  |
| spatialGranSizeCell |  |  |  | Uinteger |  |  |  | O |  |  |  | 0..1 |  |  |  | Indicates the maximum number of cells used to define an area for which the analytics are provided. May be included when the "networkArea" attribute in the EventSubscription data type is provided. |  |  |  | NetworkPerfExt_eNA Ue_MobilityExt_eNA UeCommunicationExt_eNA QoS_SustainabilityExt_eNA DispersionExt_eNA DnPerformanceExt_eNA |  |  |  |
| feedback |  |  |  | AnalyticsFeedbackInfo |  |  |  | O |  |  |  | 0..1 |  |  |  | Analytics feedback information. It may only be provided in requests to update an existing analytics subscription for predictions. |  |  |  | AnalyticsAccuracy |  |  |  |
| NOTE 1: The NetworkAreaInfo within the "locArea" attribute is not applicable for the untrusted AF. For "NETWORK_PERFORMANCE" or "CONGESTION" or "E2E_DATA_VOL_TRANS_TIME" event, the "locArea" attribute shall be provided if the event applied for all UEs (i.e. "anyUeInd" attribute set to true within the TargetUeId data). For "QOS_SUSTAINABILITY" or "MOVEMENT_BEHAVIOUR" event, at least one of the "locArea" attribute and "fineGranAreas" attribute shall be provided. NOTE 2: Only "excepId" and "excepLevel" within the Exception data type apply to the "excepRequs" attribute. NOTE 3: Either "excepRequs" or "exptAnaType" shall be provided if the subscribed event is "ABNORMAL_BEHAVIOR". NOTE 4: If the subscribed event is "CONGESTION" or "DN_PERFORMANCE", this attribute shall be provided if "notifMethod" within "analyRepInfo" sets to "ON_EVENT_DETECTION" or omitted. NOTE 5: For "QOS_SUSTAINABILITY", this property shall be provided if the "notifMethod" in "analyRepInfo" is set to "ON_EVENT_DETECTION" or omitted. NOTE 6: The "sampRatio" attribute and the "histAnaTimePeriod" attribute within EventReportingRequirement data type is not applicable for the present API. The attributes "accPerSubset", "offsetPeriod", and "timeAnaNeeded" within the EventReportingRequirement data type are applicable only if the "EneNA" feature is supported. NOTE 7: For "ABNORMAL_BEHAVIOR" event with "anyUeInd" attribute in "tgtUe" attribute sets to true, - at least one of the "locArea" and the "snssai" attribute should be included, if the expected analytics type via the"exptAnaType" attribute or the list of Exception Ids via the "excepRequs" attribute is mobility related; - at least one of the "locArea", "appIds", "dnn", ”dnns”, ”snssai” and "snssais" attribute should be included, if the expected analytics type via the"exptAnaType" attribute or the list of Exception Ids via the "excepRequs" attribute is communication related; - the expected analytics type via the"exptAnaType" attribute or the list of Exception Ids via "excepRequs" attribute shall not be requested for both mobility and communication related analytics at the same time. - “dnns” and “snssais” is mutually exclusive for “dnn” and “snssai”. “dnn” and “snssai” are available for the backward compatibility to the previous release of this specification. NOTE 8: If both the "allFreq" attribute and the "allRat" attribute within the "ratFreqs" attribute are present, then the cardinality shall be 1 as the "all" indication for all the RAT type and Frequency value the NWDAF has received for the application. NOTE 9: This parameter shall be provided when a consumer requires analytics for an edge application over a UP path. NOTE 10: The NetworkAreaInfo within the "visitedLocAreas" attribute is not applicable for the untrusted AF. If this attribute is provided, the analytics target period shall be a past time period (i.e. only statistics is supported). NOTE 11: If the the value of "userDataConOrderCri" attribute is "APPLICABLE_TIME_WINDOW", the "ASCENDING" direction indicates that the list of User Data Congestion analytics are in chronological order and the "DESCENDING" direction indicates that the list of User Data Congestion analytics are in reverse chronological order. NOTE 12: The "AGG_TRAFFIC_RATE", "VAR_TRAFFIC_RATE", "VAR_PACKET_DELAY", "MAX_PACKET_LOSS_RATE" and "VAR_PACKET_LOSS_RATE" enumeration value(s) within the AnalyticsSubset data type is applicable only if the "DnPerformanceExt_AIML” feature is supported. NOTE 13: When the "pduSesInfos" attribute is provided, the associated "appIds" attribute shall also be provided for the NWDAF to be able to compute the service experience per application. NOTE 14: If both "locArea" and "fineGranAreas" attributes are provided, the Area of Interest is interpreted as the intersection area indicated by these two attributes. NOTE 15: When the "NSLoad" feature is supported, only the "snssai" attribute of the NsiIdInfo data structure is applicable for the "NS_LOAD_LEVEL" event within each array element of this attribute. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 5.6.3.3.7	Type TargetUeId

Table 5.6.3.3.7-1: Definition of type TargetUeId

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| anyUeInd | boolean | C | 0..1 | Identifies whether the AF request applies to any UE. - Set to "true": the AF request is applicable to any UE. - Set to "false": the AF request is not applicable to any UE. - Default value is "false" if omitted. | Abnormal_Behavior Congestion Network_Performance QoS_Sustainability Dispersion DnPerformance ServiceExperience WlanPerformance_AIML |
| gpsi | Gpsi | C | 0..1 | Identifies a GPSI for an UE. | Abnormal_Behavior Congestion Ue_Mobility Ue_Communication Network_Performance Dispersion DnPerformance ServiceExperience WlanPerformance_AIML RelativeProximity |
| exterGroupId | ExternalGroupId | C | 0..1 | Represents an external group identifier and identifies a group of UEs. | Abnormal_BehaviorUe_Mobility Ue_Communication Network_Performance Dispersion DnPerformance ServiceExperience WlanPerformance_AIML RelativeProximity |
| NOTE: For an applicable feature, only one attribute identifying the target UE shall be provided. |  |  |  |  |  |

##### 5.6.3.3.8	Void

##### 5.6.3.3.9	Type UeMobilityExposure

Table 5.6.3.3.9-1: Definition of type UeMobilityExposure

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ts | DateTime | O | 0..1 | This attribute identifies the timestamp when the UE arrives the location. (NOTE 1) |  |
| recurringTime | ScheduledCommunicationTime | O | 0..1 | Identifies time of the day and day of the week which are valid within the observation period when the UE moves. (NOTE 1, NOTE 2) |  |
| duration | DurationSec | M | 1 | This attribute identifies the time duration the UE stays in the location. If the analytics result applies for a group of UEs, it indicates the average duration for the group of UEs. (NOTE 3) |  |
| durationVariance | Float | C | 0..1 | This attribute indicates the variance of the analysed durations for the group of UEs. It shall be provided if the analytics result applies for a group of UEs. |  |
| locInfo | array(UeLocationInfo) | M | 1..N | This attribute includes a list of UE location information during the time duration. (NOTE 4) |  |
| directionInfos | array(DirectionInfo) | O | 1..N | This attribute includes a list of UE direction information. | UeMobilityExt_AIML |
| NOTE 1: Either ts or recurringTime shall be provided. NOTE 2: If this attribute is present, it indicates the UE movement is periodic. This attribute is suitable to be present for a recurring mobility in a long observation time. NOTE 3: If the "temporalGranSize" attribute is provided in the request, the duration indicated by the "duration" attribute shall be greater than or equal to the value of the "temporalGranSize" attribute. NOTE 4: If the "spatialGranSizeTa" and/or "spatialGranSizeCell" attributes are provided in the request, the number of TAs or cells contained in "locInfo" attribute shall be smaller than or equal to the values of the "spatialGranSizeTa" and/or "spatialGranSizeCell" attributes. |  |  |  |  |  |

##### 5.6.3.3.10	Type UeLocationInfo

Table 5.6.3.3.10-1: Definition of type UeLocationInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| loc | LocationArea5G | M | 1 | This attribute contains the detailed location. (NOTE 3) |  |
| geoLoc | GeographicalArea | C | 0..1 | This attribute contains the geographical location in a fine granularity (e.g. smaller than a cell). Shall be present if one of the elements in the "listOfAnaSubsets" attribute in the request was set to "USER_LOCATION". (NOTE 2) (NOTE 3) | Ue_MobilityExt_eNA |
| ratio | SamplingRatio | C | 0..1 | This attribute contains the percentage of UEs in the group. Shall be present if the analytics result applies for a group of UEs. |  |
| confidence | Uinteger | C | 0..1 | Indicates the confidence of the prediction. (NOTE 1) Shall be present if the analytics result is a prediction. Minimum = 0. Maximum = 100. |  |
| geoDistrInfos | array(GeoDistributionInfo) | O | 1..N | Indicates the geographical distribution of the UEs that may be selected by the AF for application service. | UeMobilityExt_AIML |
| NOTE: If the requested period identified by the "startTs" and "endTs" attributes in the "EventReportingRequirement" type is a future time period, which means the analytics result is a prediction. If no sufficient data is collected to provide the confidence of the prediction before the time deadline, a zero confidence shall be returned. NOTE 2: When possible and applicable to the access type, the UE location is provided according to the preferred granularity subscribed or requested in the "locGranularity" attribute. NOTE 3: When the "geoLoc" attribute is present, the "loc" attribute shall be ignored. |  |  |  |  |  |

##### 5.6.3.3.11	Void

##### 5.6.3.3.12	Type: AnalyticsRequest

Table 5.6.3.3.12-1: Definition of type AnalyticsRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyEvent | AnalyticsEvent | M | 1 | Identifies the analytics type. |  |
| analyEventFilter | AnalyticsEventFilter | C | 0..1 | Shall be included to identify the analytics when filter information is needed for the related event. |  |
| analyRep | EventReportingRequirement | O | 0..1 | Identifies the analytics reporting requirement information. (NOTE) |  |
| tgtUe | TargetUeId | O | 0..1 | Identifies the target UE information. |  |
| suppFeat | SupportedFeatures | M | 1 | Represents the features supported by the NF service consumer. |  |
| NOTE: The attributes "accPerSubset", "offsetPeriod", and "timeAnaNeeded" within the EventReportingRequirement data type are applicable only if the "EneNA" feature is supported. |  |  |  |  |  |

##### 5.6.3.3.13	Type AnalyticsEventFilter

Table 5.6.3.3.13-1: Definition of type AnalyticsEventFilter

| Attribute name |  | Data type |  |  | P | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| locArea |  | LocationArea5G |  |  | C | 0..1 |  | This IE represents the network area where the NF service consumer wants to know the analytics result. (NOTE 2, NOTE 3, NOTE 10) |  | Ue_Mobility Ue_Communication Network_Performance QoS_Sustainability Abnormal_Behavior Congestion Dispersion DnPerformance ServiceExperience E2eDataVolTransTime MovementBehaviour RelativeProximity |  |
| fineGranAreas |  | array(GeographicalArea) |  | O | 1..N |  |  | Indicates the fine granularity areas to which the request applies. (i.e. with a finer granularity than cell). (NOTE 2, NOTE 10) |  | ServiceExperienceExt_eNA Ue_MobilityExt_eNA QoS_SustainabilityExt_eNA |  |
| temporalGranSize |  | DurationSec |  | O | 0..1 |  |  | Indicates the minimum duration of each time slot for which the analytics are provided. |  | NetworkPerfExt_eNA Ue_MobilityExt_eNA CongestionExt_eNA QoS_SustainabilityExt_eNA DispersionExt_eNA DnPerformanceExt_eNA |  |
| spatialGranSizeTa |  | Uinteger |  | O | 0..1 |  |  | Indicates the maximum number of TAs used to define an area for which the analytics are provided. May be included when the "networkArea" attribute in the EventSubscription data type is provided. |  | NetworkPerfExt_eNA Ue_MobilityExt_eNA UeCommunicationExt_eNA QoS_SustainabilityExt_eNA DispersionExt_eNA DnPerformanceExt_eNA |  |
| spatialGranSizeCell |  | Uinteger |  | O | 0..1 |  |  | Indicates the maximum number of cells used to define an area for which the analytics are provided. May be included when the "networkArea" attribute in the EventSubscription data type is provided. |  | NetworkPerfExt_eNA Ue_MobilityExt_eNA UeCommunicationExt_eNA QoS_SustainabilityExt_eNA DispersionExt_eNA DnPerformanceExt_eNA |  |
| dnn |  | Dnn |  |  | O | 0..1 |  | Identifies the DNN. (NOTE 3) |  | Ue_Communication Abnormal_Behavior DnPerformance ServiceExperience |  |
| dnns |  | array(Dnn) |  |  | O | 1..N |  | Identifies the DNN. (NOTE 3) |  | UeCommunicationExt_eNA Abnormal_BehaviorExt_eNA DnPerformanceExt_eNA ServiceExperienceExt_eNA E2eDataVolTransTime RelativeProximity |  |
| dnais |  | array(Dnai) |  |  | O | 1..N |  | Identification(s) of user plane access to DN(s) which the subscription applies. |  | DnPerformance ServiceExperience E2eDataVolTransTime |  |
| nwPerfTypes |  | array(NetworkPerfType) |  |  | C | 1..N |  | Represents the network performance requirements. This attribute shall be included when requested event is "NETWORK_PERFORMANCE". |  | Network_Performance |  |
| dataVlTrnsTmReqs |  | array(E2eDataVolTransTimeReq) |  |  | O | 1..N |  | Represents the E2E data volume transfer time requirements |  | E2eDataVolTransTime |  |
| appIds |  | array(ApplicationId) |  |  | O | 1..N |  | Each element identifies an application. The absence of appIds means all applications. (NOTE 3) (NOTE 9) |  | Ue_Communication Abnormal_Behavior DnPerformance ServiceExperience Dispersion E2eDataVolTransTime NSLoad |  |
| excepIds |  | array(ExceptionId) |  |  | O | 1..N |  | Represents a list of Exception Ids. (NOTE 1) |  | Abnormal_Behavior |  |
| exptAnaType |  | ExpectedAnalyticsType |  |  | O | 0..1 |  | Represents expected UE analytics type. (NOTE 1) |  | Abnormal_Behavior |  |
| exptUeBehav |  | ExpectedUeBehaviourData |  |  | O | 0..1 |  | Represents expected UE behaviour. |  | Abnormal_Behavior |  |
| snssai |  | Snssai |  |  | O | 0..1 |  | Identifies the network slice information. (NOTE 3) |  | Ue_Communication QoS_Sustainability Abnormal_Behavior Congestion Dispersion DnPerformance ServiceExperience |  |
| snssais |  | array(Snssai) |  |  | O | 1..N |  | Identifies the network slice information. (NOTE 3) |  | UeCommunicationExt_eNA QoS_SustainabilityExt_eNA Abnormal_BehaviorExt_eNA CongestionExt_eNA DispersionExt_eNA ServiceExperienceExt_eNA DnPerformanceExt_eNA E2eDataVolTransTime RelativeProximity |  |
| nsiIdInfos |  | array(NsiIdInfo) |  |  | O | 1..N |  | Each element identifies the S-NSSAI and the optionally associated network slice instance(s). May be included when requested event is "SERVICE_EXPERIENCE" "DN_PERFORMANCE" or "NS_LOAD_LEVEL". (NOTE 12) |  | ServiceExperience DnPerformance NSLoad |  |
| qosReq |  | QosRequirement |  |  | C | 0..1 |  | Represents the QoS requirements. This attribute shall be included when requested event is "QOS_SUSTAINABILITY". |  | QoS_Sustainability E2eDataVolTransTime |  |
| listOfAnaSubsets |  | array(AnalyticsSubset) |  |  | O | 1..N |  | The list of analytics subsets can be used to indicate the content of the analytics. (NOTE 8) |  | EneNA |  |
| dnPerfReqs |  | array(DnPerformanceReq) |  |  | O | 1..N |  | Represents the DN performance analytics requirements. |  | DnPerformance |  |
| bwRequs |  | array(BwRequirement) |  |  | O | 1..N |  | Represents the media/application bandwidth requirement for each application. It may only be present if "appIds" attribute is provided. |  | ServiceExperience |  |
| ratFreqs |  | array(RatFreqInformation) |  |  | O | 1..N |  | Identification(s) of the RAT type and/or frequencies of UE’s serving cell(s) which the subscriptiont applies. (NOTE 4) |  | ServiceExperience |  |
| appServerAddrs |  | array(AddrFqdn) |  |  | C | 1..N |  | Each of the element represents the Application Server Instance (IP address/FQDN of the Application Server). (NOTE 5) |  | ServiceExperience DnPerformance |  |
| wlanReqs |  | array(WlanPerformanceReq) |  |  | O | 1..N |  | Represents other WLAN performance analytics requirements. If the attribute contains no content, may take default handling action. |  | WlanPerformance_AIML |  |
| disperReqs |  | array(DispersionRequirement) |  |  | O | 1..N |  | Represents the requirements of dispersion analytics. |  | Dispersion |  |
| maxNumOfTopAppUl |  | Uinteger |  |  | O | 0..1 |  | Indicates the requested maximum number of top applications that contribute the most to the traffic in Uplink direction. Minimum = 1. May be included when one of the elements in the "listOfAnaSubsets" attribute is set to LIST_OF_TOP_APP_UL. |  | CongestionExt |  |
| maxNumOfTopAppDl |  | Uinteger |  |  | O | 0..1 |  | Indicates the requested maximum number of top applications that contribute the most to the traffic in Downlink direction. Minimum = 1. May be included when one of the elements in the "listOfAnaSubsets" attribute is set to LIST_OF_TOP_APP_DL. |  | CongestionExt |  |
| visitedLocAreas |  | array(LocationArea5G) |  |  | O | 1..N |  | Identifications of network areas which the UEs had previously been in at least one of the Visited Area(s) of Interest. (NOTE 6) |  | Ue_Mobility |  |
| ueCommReqs |  | array(UeCommReq) |  |  | O | 1..N |  | Represents the UE communication requirements. This attribute may be included when the subscribed event is "UE_COMM". |  | UeCommunicationExt_eNA |  |
| userDataConReq |  | UserDataCongestReq |  |  | O | 0..1 |  | The ordering criterion for the list of User Data Congestion analytics. (NOTE 7) |  | CongestionExt_eNA |  |
| locGranularity |  | LocInfoGranularity |  |  | O | 0..1 |  | The preferred granularity of UE location information. |  | ServiceExperienceExt_eNA Ue_MobilityExt_eNA DispersionExt_eNA |  |
| locOrientation |  | LocationOrientation |  | O | 0..1 |  |  | Indicates the preferred orientation of location information. |  | Ue_MobilityExt_eNA MovementBehaviour |  |
| ueMobilityReqs |  | array(UeMobilityReq) |  |  | O | 1..N |  | Represents the UE mobility requirements. This attribute may be included when the subscribed event is "UE_MOBILITY". |  | Ue_MobilityExt_eNA |  |
| movBehavReqs |  | array(MovBehavReq) |  | O | 1..N |  |  | Represents the Movement Behaviour analytics requirements. |  | MovementBehaviour |  |
| relProxReqs |  | array(RelProxReq) |  | O | 1..N |  |  | Represents the Relative Proximity analytics requirements. |  | RelativeProximity |  |
| pduSesInfos |  | array(PduSessionInfo) |  |  | O | 1..N |  | Represents combination of PDU Session parameters information. (NOTE 9) |  | ServiceExperienceExt_eNA |  |
| useCaseCxt |  | string |  |  | O | 0..1 |  | Indicates the context of usage of the analytics. The value and format of this parameter are not standardized. |  | ENAExt |  |
| accuReq |  | AccuracyReq |  |  | O | 0..1 |  | Represents the analytics accuracy requirement information. (NOTE 11) |  | AnalyticsAccuracy |  |
| NOTE 1: Either "excepIds" or "exptAnaType" shall be provided if the requested event is "ABNORMAL_BEHAVIOR". NOTE 2: The NetworkAreaInfo within the "locArea" attribute is not applicable for the untrusted AF. For "NETWORK_PERFORMANCE" or "CONGESTION" event, the "locArea" attribute shall be provided if the event applied for all UEs (i.e. "anyUeInd" attribute set to true within the TargetUeId data). For "QOS_SUSTAINABILITY" or "MOVEMENT_BEHAVIOUR" event, at least one of "locArea" attribute and "fineGranAreas" attribute shall be provided. NOTE 3: For "ABNORMAL_BEHAVIOR" event with "anyUeInd" attribute in "tgtUe" attribute sets to true, - at least one of the "locArea" and the "snssai" attribute should be included, if the expected analytics type via the"exptAnaType" attribute or the list of Exception Ids via the "excepIds" attribute is mobility related; - at least one of the "locArea", "appIds", "dnn" and "snssai" attribute should be included, if the expected analytics type via the"exptAnaType" attribute or the list of Exception Ids via the "excepIds" attribute is communication related; - the expected analytics type via the"exptAnaType" attribute or the list of Exception Ids via "excepIds" attribute shall not be requested for both mobility and communication related analytics at the same time. - “dnns” and “snssais” is mutually exclusive for “dnn” and “snssai”. “dnn” and “snssai” are available for the backward compatibility to the previous release of this specification. NOTE 4: If both the "allFreq" attribute and the "allRat" attribute within the "ratFreqs" attribute are present, then the cardinality shall be 1 as the "all" indication for all the RAT type and Frequency value the NWDAF has received for the application. NOTE 5: This parameter shall be provided when a consumer requires analytics for an edge application over a UP path. NOTE 6: The NetworkAreaInfo within the "visitedLocAreas" attribute is not applicable for the untrusted AF. If this attribute is provided, the analytics target period shall be a past time period (i.e. only statistics is supported). NOTE 7: If the the value of "orderCriterion" attribute contained in "userDataConReq" attribute is "APPLICABLE_TIME_WINDOW", the "ASCENDING" direction indicates that the list of User Data Congestion analytics are in chronological order and the "DESCENDING" direction indicates that the list of User Data Congestion analytics are in reverse chronological order. NOTE 8: The "AGG_TRAFFIC_RATE", "VAR_TRAFFIC_RATE", "VAR_PACKET_DELAY", "MAX_PACKET_LOSS_RATE" and "VAR_PACKET_LOSS_RATE" enumeration value(s) within the AnalyticsSubset data type is applicable only if the "DnPerformanceExt_AIML” feature is supported. NOTE 9: When the "pduSesInfos" attribute is provided, the associated "appIds" attribute shall also be provided for the NWDAF to be able to compute the service experience per application. NOTE 10: If both "locArea" and "fineGranAreas" attributes are provided, the Area of Interest is interpreted as the intersection area indicated by these two attributes. NOTE 11: Only the "accuTimeWin" and "minNum" attributes within the AccuracyReq data type are applicable. NOTE 12: When the "NSLoad" feature is supported, only the "snssai" attribute of the NsiIdInfo data structure is applicable for the "NS_LOAD_LEVEL" event within each array element of this attribute. |  |  |  |  |  |  |  |  |  |  |  |

##### 5.6.3.3.14	Type AnalyticsData

Table 5.6.3.3.14-1: Definition of type AnalyticsData

| Attribute name |  | Data type |  | P | Cardinality |  |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| start |  | DateTime |  | O | 0..1 |  |  | It defines the start time of which the analytics information will become valid. (NOTE 1) |  | EneNA |  |
| expiry |  | DateTime |  | O | 0..1 |  |  | Defines the expiration time after which the analytics information will become invalid. (NOTE 1) |  |  |  |
| timeStampGen |  | DateTime |  | O | 0..1 |  |  | It defines the timestamp of analytics generation. |  | EneNA |  |
| ueMobilityInfos |  | array(UeMobilityExposure) |  | C | 1..N |  |  | Contains the UE mobility information. Shall be present if the "analyEvent" attribute sets to "UE_MOBILITY". (NOTE 4) |  | Ue_Mobility |  |
| ueCommInfos |  | array(UeCommunication) |  | C | 1..N |  |  | Contains the application communication information. Shall be present if the "analyEvent" attribute sets to "UE_COMM" |  | Ue_Communication |  |
| nwPerfInfos |  | array(NetworkPerfExposure) |  | C | 1..N |  |  | Contains the network performance information. Shall be present if the "analyEvent" attribute is set to "NETWORK_PERFORMANCE". |  | Network_Performance |  |
| abnormalInfos |  | array(AbnormalExposure) |  | C | 1..N |  |  | Contains the user's abnormal behavior information. Shall be present if the "analyEvent" attribute sets to "ABNORMAL_BEHAVIOR". |  | Abnormal_Behavior |  |
| congestInfos |  | array(CongestInfo) |  | C | 1..N |  |  | Contains the UE's user data congestion information. Shall be present if the "analyEvent" attribute sets to "CONGESTION". |  | Congestion |  |
| dataVlTrnsTmInfos |  | array(E2eDataVolTransTimeInfo) |  | C | 1..N |  |  | Contains the E2E data volume transfer time information. Shall be present if the "analyEvent" attribute is set to "E2E_DATA_VOL_TRANS_TIME". |  | E2eDataVolTransTime |  |
| qosSustainInfos |  | array(QosSustainabilityExposure) |  | C | 1..N |  |  | Contains the QoS sustainability information. Shall be present if the "analyEvent" attribute is set to "QOS_SUSTAINABILITY". (NOTE 2) |  | QoS_Sustainability E2eDataVolTransTime |  |
| disperInfos |  | array(DispersionInfo) |  | C | 1..N |  |  | Contains the Dispersion information. Shall be present if the "analyEvent" attribute is set to "DISPERSION". |  | Dispersion |  |
| dnPerfInfos |  | array(DnPerfInfo) |  | C | 1..N |  |  | Contains the DN performance information. Shall be present if the "analyEvent" attribute is set to "DN_PERFORMANCE". (NOTE 3) |  | DnPerformance |  |
| movBehavInfos |  | array(MovBehavInfo) |  |  | C | 1..N |  | The Movement Behaviour information. Shall be present if the "analyEvent" attribute is set to "MOVEMENT_BEHAVIOUR". |  | MovementBehaviour |  |
| relProxInfos |  | array(RelProxInfo) |  |  | C | 1..N |  | The Relative Proximity information. Shall be present if the "analyEvent" attribute is set to "RELATIVE_PROXIMITY". |  | RelativeProximity |  |
| svcExps |  | array(ServiceExperienceInfo) |  | C | 1..N |  |  | Contains the service experience information. Shall be present if the "analyEvent" attribute is set to "SERVICE_EXPERIENCE". |  | ServiceExperience |  |
| wlanInfos |  | array(WlanPerformInfo) |  | C | 1..N |  |  | The WLAN performance related information. Shall be present if the "analyEvent" attribute is set to "WLAN_PERFORMANCE". |  | WlanPerformance_AIML |  |
| accuInfo |  | AccuracyInfo |  |  | C | 0..1 |  | The analytics accuracy information. It shall be provided when accuracyReq was provided in the request and the "cancelAccuInd" attribute is set to "false" or omitted. |  | AnalyticsAccuracy |  |
| cancelAccuInd |  | boolean |  |  | O | 0..1 |  | Indicates cancelled request of the analytics accuracy information. Set to "true" indicates the NWDAF cancelled request of analytics accuracy information as the NWDAF does not support the accuracy checking capability. Otherwise set to "false". Default value is "false" if omitted. |  | AnalyticsAccuracy |  |
| suppFeat |  | SupportedFeatures |  | M | 1 |  |  | Represents the features supported by both the AF and the NEF. |  |  |  |
| NOTE 1: If the "start" attribute and the "expiry" attribute are both provided, the DateTime of the "expiry" attribute shall not be earlier than the DateTime of the "start" attribute. NOTE 2: The "qosFlowRetThd" and "ranUeThrouThd" attributes in QosSustainabilityExposure data type are not applicable. NOTE 3: The "minTrafficRate", "aggTrafficRate", "varTrafficRate", "trafRateUeIds", "avePacketDelay", "maxPacketDelay", "varPacketDelay", "packDelayUeIds", "maxPacketLossRate", "varPacketLossRate" and "packetLossUeIds" attribute(s) within the DnPerfInfo data type is applicable only if the "DnPerformanceExt_AIML” feature is supported. NOTE 4: If the "UeMobilityExt_eNA" feature is supported and the "locGranularity" attribute value "LON_AND_LAT_LEVEL" is requested, the "geoLoc" attribute within the "UeMobility" type may be provided to report the geographical location (longitude and latitude level). |  |  |  |  |  |  |  |  |  |  |  |

##### 5.6.3.3.15	Type AbnormalExposure

Table 5.6.3.3.15-1: Definition of type AbnormalExposure

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| gpsis | array(Gpsi) | C | 1..N | Each element identifies a UE which is affected with the Exception. Shall be present if the subscription request applies to more than one UE. |  |
| excep | Exception | M | 1 | Contains the exception information. |  |
| appId | ApplicationId | O | 0..1 | Identifies an application. May only be present if the "appIds" attribute was provided within AnalyticsEventFilter during the subscription for event notification procedure. |  |
| dnn | Dnn | O | 0..1 | Identifies DNN, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. | Abnormal_Behavior_Ext |
| snssai | Snssai | O | 0..1 | Identifies the network slice information. | Abnormal_Behavior_Ext |
| ratio | SamplingRatio | C | 0..1 | This attribute contains the percentage of UEs with same analytics result in the group or among all UEs. Shall be present if the analytics result applies for a group of UEs or any UE. |  |
| confidence | Uinteger | C | 0..1 | Indicates the confidence of the prediction. (NOTE) Shall be present if the analytics result is a prediction. Minimum = 0. Maximum = 100. |  |
| addtMeasInfo | AdditionalMeasurement | O | 0..1 | Additional measurement. |  |
| NOTE: If the requested period identified by the "startTs" and "endTs" attributes in the "EventReportingRequirement" type is a future time period, which means the analytics result is a prediction. If no sufficient data is collected to provide the confidence of the prediction before the time deadline, a zero confidence shall be returned. |  |  |  |  |  |

##### 5.6.3.3.16	Type CongestInfo

Table 5.6.3.3.16-1: Definition of type CongestInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| locArea | LocationArea5G | M | 1 | Network area of interest. (NOTE) |  |
| cngAnas | array(CongestionAnalytics) | M | 1..N | Represents data congestion analytics for transfer over the user plane, control plane or both planes. |  |
| NOTE: The NetworkAreaInfo data within the LocationArea5G data is not applicable. |  |  |  |  |  |

##### 5.6.3.3.17	Type CongestionAnalytics

Table 5.6.3.3.17-1: Definition of type CongestionAnalytics

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| cngType | CongestionType | M | 1 | Represents congestion type. |  |
| tmWdw | TimeWindow | M | 1 | Represents a start time and a stop time observed for the congestion information. (NOTE 2) |  |
| nsi | ThresholdLevel | M | 1 | Represents network congestion level. |  |
| confidence | Uinteger | C | 0..1 | Indicates the confidence of the prediction. (NOTE 1) Shall be present if the analytics result is a prediction. Minimum = 0. Maximum = 100. |  |
| topAppListUl | array(TopApplication) | C | 1..N | List of top applications in Uplink. Shall be present if one of the element in the "listOfAnaSubsets" attribute was set to LIST_OF_TOP_APP_UL. | CongestionExt |
| topAppListDl | array(TopApplication) | C | 1..N | List of top applications in Downlink. Shall be present if one of the element in the "listOfAnaSubsets" attribute was set to LIST_OF_TOP_APP_DL. | CongestionExt |
| NOTE 1: If the requested period identified by the "startTs" and "endTs" attributes in the "EventReportingRequirement" type is a future time period, which means the analytics result is a prediction. If no sufficient data is collected to provide the confidence of the prediction before the time deadline, a zero confidence shall be returned. NOTE 2: If the "temporalGranSize" attribute is provided in the request, the duration indicated by the "tmWdw" attribute shall be greater than or equal to the value of the "temporalGranSize" attribute. |  |  |  |  |  |

##### 5.6.3.3.18	Type QosSustainabilityExposure

Table 5.6.3.3.18-1: Definition of type QosSustainabilityExposure

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| locArea |  | LocationArea5G |  | M |  | 1 |  | Identification(s) of applicable location areas where the analytics result applies. (NOTE 3, NOTE 5) |  |  |  |
| fineAreaInfos |  | array(GeographicalArea) |  | O |  | 1..N |  | This attribute contains the geographical locations in a fine granularity (e.g. smaller than a cell). May be provided when the "fineGranAreas" attribute is provided in the request. |  | QoS_SustainabilityExt_eNA |  |
| startTs |  | DateTime |  | M |  | 1 |  | Represents the start time of the applicable observing period. (NOTE 4) |  |  |  |
| endTs |  | DateTime |  | M |  | 1 |  | Represents the end time of the applicable observing period. |  |  |  |
| qosFlowRetThd |  | RetainabilityThreshold |  | O |  | 0..1 |  | The reporting QoS Flow Retainability Threshold that are met or crossed for 5QI of GBR resource type. (NOTE 1) |  |  |  |
| ranUeThrouThd |  | BitRate |  | O |  | 0..1 |  | The reporting RAN UE Throughput Threshold that are met or crossed for 5QI of non-GBR resource type. (NOTE 1) |  |  |  |
| snssai |  | Snssai |  | O |  | 0..1 |  | Identifies the network slice information. |  | QoS_Sustainability_Ext |  |
| confidence |  | Uinteger |  | C |  | 0..1 |  | Indicates the confidence of the prediction. (NOTE 2) Shall be present if the analytics result is a prediction. Minimum = 0. Maximum = 100. |  |  |  |
| NOTE 1: Either qosFlowRetThd or ranUeThrouThd shall be provided. NOTE 2: If the requested period identified by the "startTs" and "endTs" attributes in the "EventReportingRequirement" type is a future time period, which means the analytics result is a prediction. If no sufficient data is collected to provide the confidence of the prediction before the time deadline, a zero confidence shall be returned. NOTE 3: The NetworkAreaInfo data within the LocationArea5G data is not applicable. NOTE 4: If the "temporalGranSize" attribute is provided in the request, the duration indicated by the "startTs" and "endTs" attributes shall be greater than or equal to the value of the "temporalGranSize" attribute. NOTE 5: If the "spatialGranSizeTa" and/or "spatialGranSizeCell" attributes are provided in the request, the number of TAs or cells contained in "locArea" attribute shall be smaller than or equal to the values of the "spatialGranSizeTa" and/or "spatialGranSizeCell" attributes. |  |  |  |  |  |  |  |  |  |  |  |

##### 5.6.3.3.19	Type NetworkPerfExposure

Table 5.6.3.3.19-1: Definition of type NetworkPerfExposure

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| locArea |  | LocationArea5G |  | M |  | 1 |  | Identification of network area to which the subscription applies. (NOTE 3, NOTE 6) |  |  |  |
| nwPerfType |  | NetworkPerfType |  | M |  | 1 |  | The type of the network performance |  |  |  |
| anaPeriod |  | TimeWindow |  | O |  | 0..1 |  | Indicates the analytics target period subset within the requested analytics target period. (NOTE 5) |  | NetworkPerfExt_eNA |  |
| relativeRatio |  | SamplingRatio |  | O |  | 0..1 |  | The reported relative ratio expressed in percentage. (NOTE 1) |  |  |  |
| absoluteNum |  | Uinteger |  | O |  | 0..1 |  | The reported absolute number (NOTE 1) |  |  |  |
| rscUsgReq |  | ResourceUsageRequirement |  | O |  | 0..1 |  | Indicates more information when providing resource usage information for the network performance type indicated by the "nwPerfType" attribute. (NOTE 4) |  | NetworkPerformanceExt_AIML |  |
| confidence |  | Uinteger |  | C |  | 0..1 |  | Indicates the confidence of the prediction. (NOTE 2) Shall be present if the analytics result is a prediction. Minimum = 0. Maximum = 100. |  |  |  |
| NOTE 1: Either relativeRatio or absoluteNum shall be provided. NOTE 2: If the requested period identified by the "startTs" and "endTs" attributes in the "EventReportingRequirement" type is a future time period, which means the analytics result is a prediction. If no sufficient data is collected to provide the confidence of the prediction before the time deadline, a zero confidence shall be returned. NOTE 3: The NetworkAreaInfo data within the LocationArea5G data is not applicable. NOTE 4: The "rscUsgReq" value is only applicable when the "nwPerfType" attribute is set to "GNB_RSC_USAGE_OVERALL_TRAFFIC", "GNB_RSC_USAGE_GBR_TRAFFIC" or "GNB_RSC_USAGE_DELAY_CRIT_GBR_TRAFFIC". NOTE 5: If the "temporalGranSize" attribute is provided in the request, the duration indicated by the "anaPeriod" attribute is greater than or equal to the value of the "temporalGranSize" attribute. NOTE 6: If the "spatialGranSizeTa" and/or "spatialGranSizeCell" attributes are provided in the request, the number of TAs or cells contained in "locArea" attribute shall be smaller than or equal to the values of the "spatialGranSizeTa" and/or "spatialGranSizeCell" attributes. |  |  |  |  |  |  |  |  |  |  |  |

##### 5.6.3.3.20	Type AnalyticsFailureEventInfo

Table 5.6.3.3.20-1: Definition of type AnalyticsFailureEventInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | AnalyticsEvent | M | 1 | Event that is subscribed. |  |
| failureCode | AnalyticsFailureCode | M | 1 | Identifies the failure reason |  |

##### 5.6.3.3.21	Type WlanPerformInfo

Table 5.6.3.3.21-1: Definition of type WlanPerformInfo

| Attribute name |  | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- | --- |
| locArea |  | LocationArea5G | C | 0..1 | Identification of location area to which the notification applies within the subscribed location area. This attribute shall be present if the "locArea" attribute is included in the event subscription or analytics request. |  |
| wlanPerSsidInfos |  | array(WlanPerSsIdPerformanceInfo) | M | 1..N | WLAN performance information for SSID(s) of WLAN access points deployed in the Area of Interest. |  |
| wlanPerUeIdInfos |  | array(WlanPerUeIdPerformanceInfo) | O | 1..N | WLAN performance information for UE Id(s) of WLAN access points deployed in the Area of Interest. (NOTE) |  |
| NOTE: The "supi" attribute within the WlanPerUeIdPerformanceInfo data type is not applicable to the current specification. |  |  |  |  |  |  |

#### 5.6.3.4	Simple data types and enumerations

##### 5.6.3.4.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.6.3.4.2	Simple data types

The simple data types defined in table 5.6.3.4.2-1 shall be supported.

Table 5.6.3.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 5.6.3.4.3	Enumeration: AnalyticsEvent

The enumeration represents the type of analytics events of which the AF requests to be notified. It shall comply with the provisions defined in table 5.6.3.4.3-1.

Table 5.6.3.4.3-1: Enumeration AnalyticsEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UE_MOBILITY | The AF requests to be notified about analytics information of UE mobility. | Ue_Mobility |
| UE_COMM | The AF requests to be notified about analytics information of UE communication. | Ue_Communication |
| ABNORMAL_BEHAVIOR | The AF requests to be notified about analytics information of UE's abnormal behavior. | Abnormal_Behavior |
| CONGESTION | The AF requests to be notified about analytics information of user data congestion information. | Congestion |
| NETWORK_PERFORMANCE | The AF requests to be notified about analytics information of network performance information. | Network_Performance |
| QOS_SUSTAINABILITY | The AF requests to be notified about analytics information of QoS sustainability. | QoS_Sustainability |
| DISPERSION | The AF requests to be notified about analytics information of Dispersion information. | Dispersion |
| DN_PERFORMANCE | The AF requests to be notified about analytics information of DN performance information. | DnPerformance |
| SERVICE_EXPERIENCE | The AF requests to be notified about analytics information of service experience. | ServiceExperience |
| E2E_DATA_VOL_TRANS_TIME | The AF requests to be notified about analytics information of E2E data volume transfer time. | E2eDataVolTransTime |
| MOVEMENT_BEHAVIOUR | The AF requests to be notified about analytics information of Movement Behaviour. | MovementBehaviour |
| WLAN_PERFORMANCE | Indicates that the event subscribed is WLAN performance. | WlanPerformance_AIML |
| NS_LOAD_LEVEL | Indicates that the event subscribed is Network Slice load level information. | NSLoad |
| RELATIVE_PROXIMITY | The AF requests to be notified about analytics information of Relative Proximity. | RelativeProximity |

##### 5.6.3.4.4	Enumeration: AnalyticsFailureCode

Table 5.6.3.4.4-1: Enumeration AnalyticsFailureCode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| BOTH_STAT_PRED_NOT_ALLOWED | The event is rejected since the start time is in the past and the end time is in the future, which means the NF service consumer requested both statistics and prediction for the analytics. |  |
| UNAVAILABLE_DATA | The event is rejected since necessary data to perform the service is unavailable. |  |
| UNSATISFIED_REQUESTED_ANALYTICS_TIME | Indicates that the requested event is rejected since the analytics information is not ready when the time indicated by the "timeAnaNeeded" attribute (as provided during the creation or modification of subscription) is reached. | EneNA |
| NO_ROAMING_SUPPORT | Indicates that the request is rejected because roaming analytics or data are required and they could not be collected because the NWDAF(s) that were invoked by the NEF do not support the roaming exchange capability. | RoamingAnalytics |
| OTHER | The event is rejected due to other reasons. |  |

### 5.6.4	Used Features

The table below defines the features applicable to the AnalyticsExposure API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.6.4-1: Features used by AnalyticsExposure API

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Ue_Mobility | This feature indicates support for the analytics event related to UE mobility. |
| 2 | Ue_Communication | This feature indicates support for the analytics event related to UE communication information. |
| 3 | Abnormal_Behavior | This feature indicates support for the analytics event related to UE's abnormal behaviour. |
| 4 | Congestion | This feature indicates support for the analytics event related to UE's user data congestion information. |
| 5 | Network_Performance | This feature indicates support for the analytics event related to network performance. |
| 6 | QoS_Sustainability | This feature indicates support for the analytics event related to QoS sustainability. |
| 7 | Notification_websocket | The delivery of notifications over Websocket is supported as described in 3GPP TS 29.122 [4]. This feature requires that the Notification_test_event feature is also supported. |
| 8 | Notification_test_event | The testing of notification connection is supported as described in 3GPP TS 29.122 [4]. |
| 9 | Dispersion | This feature indicates support for the analytics event related to Dispersion analytics. |
| 10 | EneNA | This feature indicates support for the enhancements of network data analytics requirements. |
| 11 | DnPerformance | This feature indicates the support of the analytics event related to DN performance. |
| 12 | ServiceExperience | This feature indicates support for the event related to service experience. |
| 13 | CongestionExt | This feature indicates support for the extensions to the event related to user data congestion, including support of GPSI and/or list of Top applications. Supporting this feature also requires the support of feature Congestion. |
| 14 | Abnormal_Behavior_Ext | This feature indicates support for the extensions to the event related to abnormal behavior, including support of exposing DNN and S-NSSAI information. Supporting this feature also requires the support of feature Abnormal_Behavior. |
| 15 | QoS_Sustainability_Ext | This feature indicates support for the extensions to the event related to QoS sustainability, including support of exposing S-NSSAI information. Supporting this feature also requires the support of feature QoS_Sustainability. |
| 16 | TermRequest | This feature indicates support for Analytics Exposure Subscription termination requests sent by the NEF to the NF service consumer. |
| 17 | QoS_SustainabilityExt_eNA | This feature indicates support for the extensions related to eNA to the event related to QoS sustainability, including support of exposing S-NSSAI information. Supporting this feature also requires the support of feature QoS_Sustainability. |
| 18 | ServiceExperienceExt_eNA | This feature indicates support for the extensions to the event related to service experience supporting eNA, including support for DNN, S-NSSAI, Location Area, PDU Session parameters information for service experience analytics. Supporting this feature also requires the support of feature ServiceExperience. |
| 19 | Abnormal_BehaviorExt_eNA | This feature indicates support for the extensions to the event related to abnormal behavior related to eNA, including support of exposing DNN and S-NSSAI information. Supporting this feature also requires the support of feature Abnormal_Behavior. |
| 20 | CongestionExt_eNA | This feature indicates support for the extensions to the event related to user data congestion related to eNA, including support of GPSI and/or list of Top applications. Supporting this feature also requires the support of feature Congestion. |
| 21 | DispersionExt_eNA | This feature indicates support for the extensions associated with analytics event related to Dispersion analytics. Supporting this feature also requires the support of feature Congestion. |
| 22 | DnPerformanceExt_eNA | This feature indicates the support of the analytics event related to DN performance. Supporting this feature also requires the support of feature DnPerformance. |
| 23 | UeCommunicationExt_eNA | This feature indicates the support of the analytics event related to UE communication related to eNA. Supporting this feature also requires the support of feature Ue_Communication. |
| 24 | Ue_MobilityExt_eNA | This feature indicates the support of the analytics event related to UE Mobility supporting eNA, including ordering criterion and preferred granularity of location. Supporting this feature also requires the support of feature Ue_Mobility. |
| 25 | DnPerformanceExt_AIML | This feature indicates support for extensions to the event related to DN Performance supporting AIML, including support of extended DN Performance Analytics for group of UEs. Supporting this feature also requires the support of feature DnPerformance. |
| 26 | UeMobilityExt_AIML | This feature indicates support for further extensions to the event related to UE mobility supporting AIML, including support of UE’s geographical distribution and direction analytics. Supporting this feature also requires the support of feature UeMobility. |
| 27 | NetworkPerformanceExt_AIML | This feature indicates support of the network performance enhancements for AI/ML-based Services. Within this feature the following enhacements are covered: - support of providing gNB resource usage for GBR traffic and Delay-critical GBR traffic. Supporting this feature also requires the support of Network_Performance feature. |
| 28 | E2eDataVolTransTi me | This feature indicates support for E2E data volume transfer time analytics |
| 29 | ENAExt | This feature indicates support for the general enhancements of analytics exposure requirements, including support for use case context sent by the NF service consumer to the NEF. |
| 30 | NetworkPerfExt_eNA | This feature indicates support for the enhancements of network performance. Within this feature the following enhacements are covered: - support of providing target period subset in the analytics. Supporting this feature also requires the support of Network_Performance feature. |
| 31 | MovementBehaviour | This feature indicates support for the Movement Behaviour information. |
| 32 | WlanPerformance_AIML | This feature indicates support for the WLAN Performance information supporting AIML. |
| 33 | NSLoad | This feature indicates the support of Network Slice load level information reporting subscription/notification functionality as part of the support of network slice capability exposure application layer framework. The following functionalities are supported: - support to subscribe and get notified of Network Slice load level analytics information. |
| 34 | AnalyticsAccuracy | This feature indicates support for the Analytics Accuracy information. |
| 35 | RelativeProximity | This feature indicates support for the Relative Proximity analytics. |
| 36 | StatisticsFailure | This feature indicates support for partial failure report for statistics during event notification. This feature requires the support of the "EneNA" feature. |
| 37 | RoamingAnalytics | This feature indicates support for forwarding errors related to roaming analytics. |

### 5.6.5	Error handling

#### 5.6.5.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.6.5.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the AnalyticsExposure API.

#### 5.6.5.3	Application Errors

The application errors defined for the AnalyticsExposure API are listed in table 5.6.5.3-1.

Table 5.6.5.3-1: Application errors

| Application Error |  | HTTP status code |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| BOTH_STAT_PRED_NOT_ALLOWED |  | 400 Bad Request |  | For the requested observation period, the start time is in the past and the end time is in the future, which means the AF requested both statistics and prediction for the analytics. |  |
| USER_CONSENT_NOT_GRANTED |  | 403 Forbidden |  | Indicates that the request shall be rejected because an impacted user has not provided the required user consent. |  |
| NO_ROAMING_SUPPORT |  | 403 Forbidden |  | Indicates that the request is rejected because roaming analytics or data are required and they could not be collected because the NWDAF(s) that were invoked by the NEF do not support the roaming exchange capability. |  |
| DATA_NOT_FOUND |  | 404 Not Found |  | The requested UE subscription data is not found/does not exist. |  |
| USER_NOT_FOUND |  | 404 Not Found |  | The user does not exist. |  |
| GROUP_IDENTIFIER_NOT_FOUND |  | 404 Not Found |  | The requested Group Identifier does not exist. |  |
| SUBSCRIPTION_NOT_FOUND |  | 404 Not Found |  | The subscription does not exist. |  |
| UNAVAILABLE_DATA |  | 500 Internal Server Error |  | Indicates the requested statistics in the past is rejected since necessary data to perform the service is unavailable. |  |
| UNSATISFIED_REQUESTED_ANALYTICS_TIME |  | 500 Internal Server Error |  | Indicates that the requested event is rejected since the analytics information is not ready when the time indicated by the "timeAnaNeeded" attribute (as provided during the request) is reached. |  |

## 5.7	5GLANParameterProvision API

### 5.7.0	Introduction

The Nnef_ParameterProvision service shall use the 5GLANParameterProvision API for 5G LAN parameters provisioning.

The API URI of 5GLANParameterProvision API shall be:

- {apiRoot}/3gpp-5glan-pp/v1

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-5glan-pp".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.7.1	Resources

#### 5.7.1.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.7.1.1-1 and the resources and HTTP methods used for the 5GLANParameterProvision API.

Figure 5.7.1.1-1: Resource URI structure of the 5GLANParameterProvision API

Table 5.7.1.1-1 provides an overview of the resources and HTTP methods applicable for the 5GLANParameterProvision API.

Table 5.7.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| 5GLAN Parameters Provision Subscriptions | /{afId}/subscriptions | GET | Read all subscriptions for a given AF. |
|  |  | POST | Create a new subscription to provision parameters. |
| Individual 5GLAN Parameters Provision Subscription | /{afId}/subscriptions/{subscriptionId} | GET | Read an existing subscription identified by {subscriptionId}. |
|  |  | PUT | Update an existing subscription identified by {subscriptionId}. |
|  |  | PATCH | Modify an existing subscription identified by {subscriptionId}. |
|  |  | DELETE | Delete a subscription identified by {subscriptionId}. |

#### 5.7.1.2	Resource: 5GLAN Parameters Provision Subscriptions

##### 5.7.1.2.1	Introduction

This resource allows a AF to read all active 5GLAN parameters provision subscribtions for the given AF, or create an new individual 5GLAN parameters provision subscription to provision parameters to the NEF.

##### 5.7.1.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-5glan-pp/v1/{afId}/subscriptions

This resource shall support the resource URI variables defined in table 5.7.1.2.2-1.

Table 5.7.1.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |

##### 5.7.1.2.3	Resource Methods

###### 5.7.1.2.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.7.1.2.2.

###### 5.7.1.2.3.2	GET

The GET method allows to read all active subscriptions for a given AF. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.7.1.2.3.2-1.

Table 5.7.1.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.7.1.2.3.2-2 and the response data structures and response codes specified in table 5.7.1.2.3.2-3.

Table 5.7.1.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.7.1.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(5GLanParametersProvision) | M | 0..N | 200 OK | All the subscription information for the AF in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.7.1.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.7.1.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.7.1.2.3.3	POST

The POST method creates a new resource to individual 5GLAN parameters provision subscription for a given AF. The AF shall initiate the HTTP POST request message and the NEF shall respond to the message. The NEF shall construct the URI of the created resource.

This method shall support the request data structures specified in table 5.7.1.2.3.3-1 and the response data structures and response codes specified in table 5.7.1.2.3.3-2.

Table 5.7.1.2.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| 5GLanParametersProvision | M | 1 | Parameters to create a subscription to provision parameters. |

Table 5.7.1.2.3.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| 5GLanParametersProvision | M | 1 | 201 Created | The subscription was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.7.1.2.3.3-3: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-5glan-pp/v1/{afId}/subscriptions/{subscriptionId} |

#### 5.7.1.3	Resource: Individual 5GLAN Parameters Provision Subscription

##### 5.7.1.3.1	Introduction

This resource allows a AF to read, update or delete an existing subscription to provision 5GLAN parameters.

##### 5.7.1.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-5glan-pp/v1/{afId}/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 5.7.1.3.2-1.

Table 5.7.1.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |
| subscriptionId | string | Identifier of the subscription resource. |

##### 5.7.1.3.3	Resource Methods

###### 5.7.1.3.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.7.1.3.2.

###### 5.7.1.3.3.2	GET

The GET method allows to read the active subscription for a given AF and subscription Id. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.7.1.3.3.2-1.

Table 5.7.1.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.7.1.3.3.2-2 and the response data structures and response codes specified in table 5.7.1.3.3.2-3.

Table 5.7.1.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.7.1.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| 5GLanParametersProvision | M | 1 | 200 OK | The information for the subscription in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.7.1.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.7.1.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.7.1.3.3.3	PUT

The PUT method modifies an existing resource to update a subscription. The AF shall initiate the HTTP PUT request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.7.1.3.3.3-1 and the response data structures and response codes specified in table 5.7.1.3.3.3-2.

Table 5.7.1.3.3.3-1: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| 5GLanParametersProvision | M | 1 | Modify an existing subscription to provision parameters. |

Table 5.7.1.3.3.3-2: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| 5GLanParametersProvision | M | 1 | 200 OK | The subscription was updated successfully. |
| n/a |  |  | 204 No Content | The subscription was updated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.7.1.3.3.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.7.1.3.3.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.7.1.3.3.4	DELETE

The DELETE method deletes an existing individual 5GLAN parameters provision subscription for a given AF. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.7.1.3.3.4-1.

Table 5.7.1.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.7.1.3.3.4-2 and the response data structures and response codes specified in table 5.7.1.3.3.4-3.

Table 5.7.1.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.7.1.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The subscription was terminated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.7.1.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.7.1.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.7.1.3.3.5	PATCH

The PATCH method allows to change some properties of an existing resource to update a subscription. The AF shall initiate the HTTP PATCH request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.7.1.3.3.5-1 and the response data structures and response codes specified in table 5.7.1.3.3.5-2.

Table 5.7.1.3.3.5-1: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| 5GLanParametersProvisionPatch | M | 1 | Modify an existing subscription to provision parameters. |

Table 5.7.1.3.3.5-2: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| 5GLanParametersProvision | M | 1 | 200 OK | The subscription was updated successfully. |
| n/a |  |  | 204 No Content | The subscription was updated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.7.1.3.3.5-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.7.1.3.3.5-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

### 5.7.1A	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.7.1B	Notifications

#### 5.7.1B.1	General

Notifications shall comply to clause 5.2.5 of 3GPP TS 29.122 [4].

Table 5.7.1B.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| 5G LAN Parameter Provisioning Event Notification | {notifUri} | POST | This operation enables the NEF to notify a previously subscribed AF on 5G LAN Parameter Provisioning related Events. |

#### 5.7.1B.2	5G LAN Parameter Provisioning Event Notification

##### 5.7.1B.2.1	Description

The 5G LAN Parameter Provisioning Event Notification is used by the NEF to report 5G LAN Parameter Provisioning related Events to a previously subscribed AF.

##### 5.7.1B.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 5.7.1B.2.2-1.

Table 5.7.1B.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | Callback reference provided by the AF during the creation/update/modification of the related 5G LAN Parameters Provisioning. |

##### 5.7.1B.2.3	Operation Definition

###### 5.7.1B.2.3.1	Notification via HTTP POST

This method shall support the request data structures specified in table 5.7.1B.2.3.1-1 and the response data structures and response codes specified in table 5.7.1B.2.3.1-2.

Table 5.7.1B.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| 5GLanParamProvNotif | M | 1 | Provides information about the observed 5G LAN Parameter Provisioning related Events. |

Table 5.7.1B.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The 5G LAN Parameter Provisioning Event Notification is successfully received. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.7.1B.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

Table 5.7.1B.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

###### 5.7.1B.2.3.2	Void

### 5.7.2	Data Model

#### 5.7.2.1	General

This clause specifies the application data model supported by the 5GLANParameterProvision API.

Table 5.7.2.1-1 specifies the data types defined for the 5GLANParameterProvision API.

Table 5.7.2.1-1: 5GLANParameterProvision API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| 5GLanParameters | 5.7.2.3.3 | Represents 5G LAN service related parameters that need to be provisioned. |  |
| 5GLanParametersPatch | 5.7.2.3.6 | Represents 5G LAN service related parameters that need to be modified. |  |
| 5GLanParametersProvision | 5.7.2.3.2 | Represents an individual 5G LAN parameters provision subscription resource. |  |
| 5GLanParametersProvisionPatch | 5.7.2.3.5 | Represents the 5G LAN parameters to request the modification of a subscription to provision parameters. |  |
| 5GLanParamProvNotif | 5.7.3.15 | Represents a 5G LAN Parameters Provisioning Event Notification. | GMEC |
| AaaUsage | 5.7.2.4.3 | Represents the usage of the DN-AAA server. |  |
| AcsParams | 5.7.3.12 | Represents ACS configuration parameters. | GMEC |
| AppDescriptor | 5.7.2.3.4 | Represents an operation system and the corresponding applications. |  |
| AppDescriptorRm | 5.7.2.3.7 | Represents the same as the AppDescriptor data type but with the "nullable: true" property. |  |
| CpParams | 5.7.3.9 | Represents Communication Pattern parameters. | GMEC |
| DnnSnssaiParams | 5.7.3.14 | Represents DNN and S-NSSAI specific group parameters. | GMEC |
| ECSAddrParams | 5.7.3.13 | Represents ECS address configuration parameters. | GMEC |
| LpiParams | 5.7.3.11 | Represents Location Privacy Indication parameters. | GMEC |
| NpConfigNotif | 5.7.3.16 | Represents a Network Parameters Configuration related notification. | GMEC |
| NpConfigParams | 5.7.3.10 | Represents Network Parameters Configuration information. | GMEC |
| MaxGrpDataRateInfo | 5.7.2.3.17 | Represents the Maximum Group Data Rate related information. | GMEC |

#### 5.7.2.2	Reused data types

The data types reused by the 5GLANParameterProvision API from other specifications are listed in table 5.7.2.2-1.

Table 5.7.2.2-1: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AfReqDefaultQoS | Clause 5.33.5.2.5 | Represents the AF requested default QoS. |  |
| AppliedParameterConfiguration | 3GPP TS 29.122 [4] | Represents the parameter configuration applied in the network. |  |
| ApplicationId | 3GPP TS 29.571 [8] | Represents the identifier of an application. |  |
| AcsInfo | 3GPP TS 29.571 [8] | Contains the ACS information. |  |
| BitRate | 3GPP TS 29.571 [8] | Represents a bit rate. |  |
| CpParameterSet | 3GPP TS 29.122 [4] | Represents an offered Communication Pattern parameter set. |  |
| CpReport | 3GPP TS 29.122 [4] | Represents a CP report. |  |
| ConfigResult | 3GPP TS 29.122 [4] | Represents one configuration processing result for a group's members. |  |
| DateTime | 3GPP TS 29.122 [4] | Represents a data and a time. |  |
| Dnn | 3GPP TS 29.571 [8] | Identifies a DNN. |  |
| DurationSec | 3GPP TS 29.122 [4] | Indicates a time duration. |  |
| EcsAuthMethod |  | 3GPP TS 29.503 [17] |  | Represents the ECS Authentication Methods. |  | ECSAuthMethods |
| EcsServerAddr | 3GPP TS 29.571 [8] | Represents the Edge Configuration Server (ECS) address configuration information. |  |
| ExternalGroupId | 3GPP TS 29.122 [4] | External Group Identifier for a user group. |  |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |  |
| Ipv4Addr | 3GPP TS 29.571 [8] | Identifies an IPv4 address. |  |
| Ipv6Addr | 3GPP TS 29.571 [8] | Identifies an IPv6 address. |  |
| LadnServArea | Clause 5.33.5.2.6 | Represents an LADN Service Area. |  |
| Link | 3GPP TS 29.122 [4] | Represents a referenced resource. |  |
| Lpi | 3GPP TS 29.503 [17] | Represents the Location Privacy Indication information. |  |
| MtcProviderInformation | 3GPP TS 29.571 [8] | Indicates MTC provider information for 5G VN Group Configuration authorization. |  |
| OsId | 3GPP TS 29.519 [23] | Operating System. |  |
| PduSessionType | 3GPP TS 29.571 [8] | PDU session type. |  |
| Snssai | 3GPP TS 29.571 [8] | Identifies the S-NSSAI. |  |
| SpatialValidityCond | 3GPP TS 29.571 [8] | Represents the Spatial Validity Condition. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features. |  |
| SupportedPlmn | 3GPP TS 29.503 [17] |  | Represents the ID of the PLMN supported by the ECS along with the list of ECSPs associated with the PLMN. |  | ECSSuppPlmns |  |
| 5GVnGroupCommunicationType | 3GPP TS 29.503 [17] | Represents the 5G VN group communication type. |  |

#### 5.7.2.3	Structured data types

##### 5.7.2.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.7.2.3.2	Type: 5GLanParametersProvision

Table 5.7.2.3.2-1: Definition of type 5GLanParametersProvision

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| self | Link | C | 0..1 | Identifies the individual parameters provision subscription resource. Shall be present in the HTTP GET response when reading all the subscriptions for an AF. |  |
| 5gLanParams | 5GLanParameters | M | 1 | Represents the 5G LAN service related parameters. |  |
| suppFeat | SupportedFeatures | M | 1 | Indicates the negotiated supported features. |  |

##### 5.7.2.3.3	Type: 5GLanParameters

This type represents the 5G LAN service related parameters need to be provisioned.

Table 5.7.2.3.3-1: Definition of type 5GLanParameters

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| exterGroupId |  | ExternalGroupId |  | M |  | 1 |  | Identifies an 5G Virtual Network Group. |  |  |  |
| gpsis |  | map(Gpsi) |  | M |  | 1..N |  | Represents the list of 5G VN Group members, each member is identified by GPSI. Any string value can be used as a key of the map. |  |  |  |
| dnn |  | Dnn |  | M |  | 1 |  | DNN for the 5G VN group, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. |  |  |  |
| aaaIpv4Addr |  | Ipv4Addr |  | O |  | 1 |  | Identifies the DN-AAA server IPv4 address provided by AF, for the secondary authentication/authorization and/or UE IP address allocation by DN-AAA server. |  |  |  |
| aaaIpv6Addr |  | Ipv6Addr |  | O |  | 1 |  | Identifies the DN-AAA server IPv6 address provided by AF, for the secondary authentication/authorization and/or UE IP address allocation by DN-AAA server. |  |  |  |
| aaaUsgs |  | array(AaaUsage) |  | O |  | 1..N |  | Identifies the usage needs for secondary authentication/authorization and/or UE IP address allocation from the DN-AAA server. (NOTE 3) |  |  |  |
| mtcProviderId |  | MtcProviderInformation |  | O |  | 0..1 |  | Indicates MTC provider information for 5G VN Group Configuration authorization. (NOTE 1) |  |  |  |
| snssai |  | Snssai |  | M |  | 1 |  | S-NSSAI for the 5G VN group. |  |  |  |
| sessionType |  | PduSessionType |  | M |  | 1 |  | PDU Session Type allowed for 5G VN group. |  |  |  |
| sessionTypes |  | array(PduSessionType) |  | O |  | 1..N |  | If further PDU Session Types (in addition to the PDU Session Type indicated in the "sessionType" attribute) are allowed for the 5G VN group, they are provided in this attribute. (NOTE 2) |  | multipleSessionTypes |  |
| appDesps |  | map(AppDescriptor) |  | M |  | 1..N |  | Describes the operation systems and the corresponding applications for each operation systems. The key of map is osId. |  |  |  |
| vnGroupCommInd |  | boolean |  | O |  | 0..1 |  | Indicates whether the 5G VN group is associated with 5G VN group communication. - "true" indicates that the 5G VN group is associated with 5G VN group communication. - "false" indicates that the 5G VN group is not associated with 5G VN group communication. - The default value when omitted is "false". |  | GMEC |  |
| vnGroupCommType |  | 5GVnGroupCommunicationType |  | O |  | 0..1 |  | Contains the 5G VN group communication type. This attribute shall be present only when the "vnGroupCommInd" is present and set to "true". |  | GMEC |  |
| maxGrpDataRateInfo |  | MaxGrpDataRateInfo |  | O |  | 0..1 |  | Represents the Maximum Group Data Rate related information. |  | GMEC |  |
| cpParams |  | CpParams |  | O |  | 0..1 |  | Contains Communication Pattern Parameters for the 5G VN group. |  | GMEC |  |
| npConfigParams |  | NpConfigParams |  | O |  | 0..1 |  | Contains Network Parameters Configuration information for the 5G VN group. |  | GMEC |  |
| lpiParams |  | LpiParams |  | O |  | 0..1 |  | Contains Location Privacy Indication parameters for the 5G VN group. |  | GMEC |  |
| acsParams |  | AcsParams |  | O |  | 0..1 |  | Contains ACS configuration parameters for the 5G VN group. |  | GMEC |  |
| ecsAddrParams |  | ECSAddrParams |  | O |  | 0..1 |  | Contains ECS address configuration parameters for the 5G VN group. |  | GMEC |  |
| dnnSnssaiParams |  | DnnSnssaiParams |  | O |  | 0..1 |  | Contains DNN and S-NSSAI specific group parameters for the 5G VN group. |  | GMEC |  |
| notifUri |  | Link |  | O |  | 0..1 |  | Contains a URI indicating the notification destination where notification requests shall be delivered. This attribute may be present only when 5G LAN Parameters Provisioning Event notifications (e.g., Network Parameters Configuration related notifications) need to be delivered. |  | GMEC |  |
| NOTE 1: The NEF should check received MTC Provider information and then the NEF may: - override it with local configured value and send it to UDM; - send it directly to the UDM; or - reject the 5G VN Group Configuration request. NOTE 2: Only one PDU Session type is applied for a PDU Session of a VN group at a time. NOTE 3: This attribute shall contain at most 2 array elements. It is however kept defined as it is (i.e. with a cardinality of "1..N") for backward compatibility considerations. |  |  |  |  |  |  |  |  |  |  |  |

##### 5.7.2.3.4	Type: AppDescriptor

Table 5.7.2.3.4-1: Definition of type AppDescriptor

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| osId | OsId | M | 1 | Identifies an operating system supported by the UE. |  |
| appIds | map(ApplicationId) | M | 1..N | Identifies applications that is running on the UE's operating system. Any string value can be used as a key of the map. |  |

##### 5.7.2.3.5	Type: 5GLanParametersProvisionPatch

Table 5.7.2.3.5-1: Definition of type 5GLanParametersProvisionPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| 5gLanParamsPatch | 5GLanParametersPatch | O | 0..1 | Represents the 5G LAN servise related parameters. This attribute is deprecated. The "5gLanParams" attribute should be used instead. |  |
| 5gLanParams | 5GLanParameters | O | 0..1 | Represents the updated 5G LAN service related parameters. |  |

##### 5.7.2.3.6	Type: 5GLanParametersPatch

This data type is deprecated.

Table 5.7.2.3.6-1: Definition of type 5GLanParametersPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| gpsis | map(GpsiRm) | O | 1..N | Represents the list of 5G VN Group members, each member is identified by GPSI. Any string value can be used as a key of the map. |  |
| appDesps | map(AppDescriptorRm) | O | 1..N | Describes the operation systems and the corresponding applications for each operation system. The key of map is osId. |  |
| cpParams | CpParams | O | 0..1 | Contains the updated Communication Pattern Parameters for the 5G VN group. | GMEC |
| npConfigParams | NpConfigParams | O | 0..1 | Contains the updated Network Parameters Configuration information for the 5G VN group. | GMEC |
| lpiParams | LpiParams | O | 0..1 | Contains the updated Location Privacy Indication parameters for the 5G VN group. | GMEC |
| acsParams | AcsParams | O | 0..1 | Contains the updated ACS configuration parameters for the 5G VN group. | GMEC |
| ecsAddrParams | ECSAddrParams | O | 0..1 | Contains the updated ECS address parameters for the 5G VN group. | GMEC |
| dnnSnssaiParams | DnnSnssaiParams | O | 0..1 | Contains the updated DNN and S-NSSAI specific parameters for the 5G VN group. | GMEC |
| notifUri | Link | O | 0..1 | Contains the updated URI indicating the notification destination where notification requests shall be delivered. | GMEC |

##### 5.7.2.3.7	Type: AppDescriptorRm

Table 5.7.2.3.7-1: Definition of type AppDescriptorRm

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appIds | map(ApplicationIdRm) | O | 1..N | Identifies application(s) on the UE's operating system. Any string value can be used as a key of the map. |  |

##### 5.7.2.3.8	Void

##### 5.7.2.3.9	Type: CpParams

Table 5.7.2.3.9-1: Definition of type CpParams

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| cpParameterSets | map(CpParameterSet) | M | 1..N | Represents one or more set(s) of CP parameters information for the 5G VN group. Any string value may be used as a key of the map. |  |
| cpReports | map(CpReport) | C | 1..N | Contains the identifier(s) of the set(s) of CP parameters for which the provided CP parameters are not added or modified successfully with the corresponding failure reason. Each element provides the related information for one or more CP set identifier(s). The key of the map is a string representing the failure identifier. This attribute may only be present in responses from the NEF to the AF. (NOTE). |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

##### 5.7.2.3.10	Type: NpConfigParams

Table 5.7.2.3.10-1: Definition of type NpConfigParams

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| maximumLatency | DurationSec | O | 0..1 | Contains the maximum delay acceptable for downlink data transfers. |  |
| maximumResponseTime | DurationSec | O | 0..1 | Contains the time duration that the UE stays reachable to allow the AF to reliably deliver the required downlink data. |  |
| suggestedNumberOfDlPackets | integer | O | 0..1 | Contains the number of packets that the serving gateway shall buffer in case the UE is not reachable. |  |
| groupReportingGuardTime | DurationSec | O | 0..1 | Identifies the group reporting aggregation time duration that the NEF shall use when aggregating the reports detected for the UE(s) in the 5G VN group and report them together to the AF. |  |
| validityTime | DateTime | O | 0..1 | Identifies when the network parameter expires and shall be deleted locally if it expires. (NOTE) |  |
| NOTE: If this attribute is omitted, no expiry time applies for the provisioned network parameter configuration. |  |  |  |  |  |

##### 5.7.2.3.11	Type: LpiParams

Table 5.7.2.3.11-1: Definition of type LpiParams

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| lpi | Lpi | M | 1 | Contains the Location Privacy Indication parameters for the 5G VN group. |  |

##### 5.7.2.3.12	Type: AcsParams

Table 5.7.2.3.12-1: Definition of type AcsParams

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| acsInfo | AcsInfo | M | 1 | Contains the ACS parameters for the 5G VN group. |  |

##### 5.7.2.3.13	Type: ECSAddrParams

Table 5.7.2.3.13-1: Definition of type ECSAddrParams

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ecsServerAddr | EcsServerAddr | M | 1 | Represents the ECS address(es). |  |
| spatialValidityCond | SpatialValidityCond | O | 0..1 | Contains the spatial validity conditions. |  |
| ecsAuthMethods | array(EcsAuthMethod) | O | 1..N | Contains the supported ECS Authentication Method(s). | ECSAuthMethods |
| supportedPlmns | array(SupportedPlmn) | O | 1..N | Contains the List of PLMNs and associated ECSPs for which EDN configuration information can be provided by the ECS. | ECSSuppPlmns |

##### 5.7.2.3.14	Type: DnnSnssaiParams

Table 5.7.2.3.14-1: Definition of type DnnSnssaiParams

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| defQos | AfReqDefaultQoS | C | 0..1 | Represents the AF-requested default QoS parameters applicable to each UE within the 5G VN group. |  |
| ladnServArea | LadnServArea | C | 0..1 | Represents the AF-requested LADN Service Area applicable to each UE within the 5G VN group. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

##### 5.7.2.3.15	Type: 5GLanParamProvNotif

Table 5.7.2.3.15-1: Definition of type 5GLanParamProvNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| npConfigNotif | NpConfigNotif | C | 0..1 | Represents a configuration result notification related to the provisioned Network Parameters Configuration information. This attribute shall be present only if Network Parameters Configuration information was provisioned within the associated 5G LAN Parameters Provisioning and a configuration result notification needs to be sent. |  |

##### 5.7.2.3.16	Type: NpConfigNotif

Table 5.7.2.3.16-1: Definition of type NpConfigNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| configResults | array(ConfigResult) | O | 0..N | Each element identifies a notification of grouping configuration result. |  |
| appliedParam | AppliedParameterConfiguration | O | 0..1 | Indicates the applied parameter configuration in the network. The "maximumDetectionTime" attribute within the AppliedParameterConfiguration data type is not applicable for Network Parameter configuration. |  |

##### 5.7.2.3.17	Type: MaxGrpDataRateInfo

Table 5.7.2.3.17-1: Definition of type MaxGrpDataRateInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| maxGrpDataRateUl | BitRate | C | 0..1 | Contains the maximum allowed aggregate UL data rate across all GBR and Non-GBR QoS Flows within the 5G VN group. |  |
| maxGrpDataRateDl | BitRate | C | 0..1 | Contains the maximum allowed aggregate DL data rate across all GBR and Non-GBR QoS Flows within the 5G VN group. |  |
| NOTE: At least one of these attributes shall be provided. |  |  |  |  |  |

#### 5.7.2.4	Simple data types and enumerations

##### 5.7.2.4.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.7.2.4.2	Simple data types

The simple data types defined in table 5.7.2.4.2-1 shall be supported.

Table 5.7.2.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 5.7.2.4.3	Enumeration: AaaUsage

Table 5.7.2.4.3-1: Enumeration AaaUsage

The enumeration AaaUsage represents the usage of the DN-AAA server.

| Enumeration value | Description |
| --- | --- |
| "AUTH" | Secondary authentication/authorization by DN-AAA server |
| "IP_ALLOC" | UE IP address allocation by DN-AAA server |

### 5.7.3	Used Features

The table below defines the features applicable to the 5GLANParameterProvision API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.7.3-1: Features used by 5GLANParameterProvision API

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | multipleSessionTypes | Indicates that multiple allowed PDU Session Types can be provided for a 5G VN group. |
| 2 | GMEC | This feature indicates the support of Generic Group Management, Exposure and Communication Enhancements. The following functionalities are supported: - Support the provisioning of the Maximum Group Data Rate related information for 5G VN groups. - Support the simultaneous provisioning of 5G VN group information and 5G VN group parameters. - Support the provisioning of the indication on whether the 5G VN group is associated with 5G VN group communications or not. |
| 3 | Void |  |
| 4 | Void |  |
| 5 | ECSAuthMethods | This feature indicates the support of ECS authentication methods for 5G VN group. |
| 6 | ECSSuppPlmns | This feature indicates the support of ECS supported PLMNs. |

### 5.7.4	Error handling

#### 5.7.4.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.7.4.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the 5GLANParameterProvision API.

#### 5.7.4.3	Application Errors

The application errors defined for 5GLANParameterProvision API are listed in table 5.7.4.3-1.

Table 5.7.4.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

## 5.8	ApplyingBdtPolicy API

### 5.8.0	Introduction

The Nnef_ApplyingBdtPolicy service shall use the ApplyingBdtPolicy API.

The API URI of ApplyingBdtPolicy API shall be:

- {apiRoot}/3gpp-applying-bdt-policy/v1

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-applying-bdt-policy".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.8.1	Resources

#### 5.8.1.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.8.1.1-1 and the resources and HTTP methods used for the ApplyingBdtPolicy API.

Figure 5.8.1.1-1: Resource URI structure of the ApplyingBdtPolicy API

Table 5.8.1.1-1 provides an overview of the resources and HTTP methods applicable for the ApplyingBdtPolicy API.

Table 5.8.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Applied BDT Policy Subscriptions | /{afId}/subscriptions | GET | Read all applied BDT policy subscriptions for a given AF. |
|  |  | POST | Create a new applied BDT policy subscription. |
| Individual Applied BDT Policy Subscription | /{afId}/subscriptions/{subscriptionId} | GET | Read an applied BDT policy subscription. |
|  |  | PATCH | Modify BDT Reference ID of an existing subscription to an applied BDT policy. |
|  |  | DELETE | Delete an applied BDT policy subscription |

#### 5.8.1.2	Resource: Applied BDT Policy Subscriptions

##### 5.8.1.2.1	Introduction

This resource allows a AF to read all applied BDT policy subscriptions for the given AF.

##### 5.8.1.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-applying-bdt-policy/v1/{afId}/subscriptions

This resource shall support the resource URI variables defined in table 5.8.1.2.2-1.

Table 5.8.1.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |

##### 5.8.1.2.3	Resource Methods

###### 5.8.1.2.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.8.1.2.2.

###### 5.8.1.2.3.2	GET

The GET method allows to read all active applied BDT policy subscriptions for a given AF. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.8.1.2.3.2-1.

Table 5.8.1.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.8.1.2.3.2-2 and the response data structures and response codes specified in table 5.8.1.2.3.2-3.

Table 5.8.1.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.8.1.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(AppliedBdtPolicy) | M | 0..N | 200 OK | The applied BDT Policy subscriptions for the AF in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.8.1.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.8.1.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.8.1.2.3.3	POST

The POST method creates an applied BDT policy subscription for a given AF. The AF shall initiate the HTTP POST request message and the NEF shall respond to the message. The NEF shall construct the URI of the created resource.

This method shall support the request data structures specified in table 5.8.1.2.3.3-1 and the response data structures and response codes specified in table 5.8.1.2.3.3-2.

Table 5.8.1.2.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AppliedBdtPolicy | M | 1 | Parameters to create a subscription of the applied BDT policy. |

Table 5.8.1.2.3.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AppliedBdtPolicy | M | 1 | 201 Created | The subscription was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.8.1.2.3.3-3: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-applying-bdt-policy/v1/{afId}/subscriptions/{SubscriptionId} |

#### 5.8.1.3	Resource: Individual Applied BDT Policy Subscription

##### 5.8.1.3.1	Introduction

This resource allows a AF to read or delete an active subscription of applied BDT policy.

##### 5.8.1.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-applying-bdt-policy/v1/{afId}/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 5.8.1.3.2-1.

Table 5.8.1.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |
| subscriptionId | string | Identifier of the subscription resource. |

##### 5.8.1.3.3	Resource Methods

###### 5.8.1.3.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.8.1.3.2.

###### 5.8.1.3.3.2	GET

The GET method allows to read the active applied BDT policy for a given AF and subscription Id. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.8.1.3.3.2-1.

Table 5.8.1.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.8.1.3.3.2-2 and the response data structures and response codes specified in table 5.8.1.3.3.2-3.

Table 5.8.1.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.8.1.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AppliedBdtPolicy | M | 1 | 200 OK | The subscription information for the AF in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.8.1.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.8.1.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.8.1.3.3.3	PATCH

The PATCH method allows to change some properties of an existing applied BDT policy subscription. The AF shall initiate the HTTP PATCH request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.8.1.3.3.3-1 and the response data structures and response codes specified in table 5.8.1.3.3.3-2.

Table 5.8.1.3.3.3-1: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AppliedBdtPolicyPatch | M | 1 | Partial update of a subscription to applying BDT policy subscritpion. |

Table 5.8.1.3.3.3-2: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AppliedBdtPolicy | M | 1 | 200 OK | The subscription was modified successfully. |
| n/a |  |  | 204 No Content | The subscription was modified successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.8.1.3.3.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.8.1.3.3.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.8.1.3.3.4	DELETE

The DELETE method deletes an existing applied BDT policy subscription for a given AF. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.8.1.3.3.4-1.

Table 5.8.1.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.8.1.3.3.4-2 and the response data structures and response codes specified in table 5.8.1.3.3.4-3.

Table 5.8.1.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.8.1.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The subscription was terminated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.8.1.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.8.1.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

### 5.8.1A	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.8.2	Notifications

Notifications are not applicable to this API.

### 5.8.3	Data Model

#### 5.8.3.1	General

This clause specifies the application data model supported by the ApplyingBdtPolicy API.

Table 5.8.3.1-1 specifies the data types defined for the ApplyingBdtPolicy API.

Table 5.8.3.1-1: ApplyingBdtPolicy API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AppliedBdtPolicy | 5.8.3.3.2 | Represents an applied BDT policy. |  |
| AppliedBdtPolicyPatch | 5.8.3.3.3 | Represents the parameters to request the modification of a subscription to applied BDT policy. |  |

#### 5.8.3.2	Reused data types

The data types reused by the ApplyingBdtPolicy API from other specifications are listed in table 5.8.3.2-1.

Table 5.8.3.2-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| BdtReferenceId | 3GPP TS 29.122 [4] | Identifier of a selected BDT policy. |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |
| ExternalGroupId | 3GPP TS 29.122 [4] | External Group Identifier for a user group. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.8.4-1. |

#### 5.8.3.3	Structured data types

##### 5.8.3.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.8.3.3.2	Type: AppliedBdtPolicy

This type represents an applied BDT policy which is sent from the AF to the NEF.

Table 5.8.3.3.2-1: Definition of type AppliedBdtPolicy

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| bdtRefId | BdtReferenceId | M | 1 | Identifies a selected policy of background data transfer. |  |
| gpsi | Gpsi | C | 0..1 | Identifies a user. (NOTE) |  |
| externalGroupId | ExternalGroupId | C | 0..1 | Identifies a user group. (NOTE) |  |
| suppFeat | SupportedFeatures | M | 1 | Indicates the list of Supported features used as described in clause 5.8.4. |  |
| self | Link | C | 0..1 | Identifies the Individual Applied BDT Policy Subscription resource. Shall be present in the HTTP GET response when reading all the subscriptions for an AF. |  |
| NOTE: Only one of the properties "gpsi" or "externalGroupId" shall be included. |  |  |  |  |  |

##### 5.8.3.3.3	Type: AppliedBdtPolicyPatch

This type represents a subscription of applied BDT policy parameters provided by the AF to the NEF. The structure is used for HTTP PATCH request.

Table 5.8.3.3.2-1: Definition of type AppliedBdtPolicyPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| bdtRefId | BdtReferenceId | M | 1 | Identifies a selected policy of background data transfer. |  |

#### 5.8.3.4	Simple data types and enumerations

##### 5.8.3.4.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.8.3.4.2	Simple data types

The simple data types defined in table 5.8.3.4.2-1 shall be supported.

Table 5.8.3.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

### 5.8.4	Used Features

The table below defines the features applicable to the ApplyingBdtPolicy API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.8.4-1: Features used by ApplyingBdtPolicy API

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 5.8.5	Error handling

#### 5.8.5.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.8.5.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the ApplyingBdtPolicy API.

#### 5.8.5.3	Application Errors

The application errors defined for ApplyingBdtPolicy API are listed in table 5.8.5.3-1.

Table 5.8.5.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

## 5.9	IPTVConfiguration API

### 5.9.0	Introduction

The Nnef_IPTVConfiguration service shall use the IPTVConfiguration API.

The API URI of IPTVConfiguration API shall be:

- {apiRoot}/3gpp-iptvconfiguration/v1

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-iptvconfiguration".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.9.1	Resources

#### 5.9.1.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.9.1.1-1 and the resources and HTTP methods used for the IPTVConfiguration API.

Figure 5.9.1.1-1: Resource URI structure of the IPTVConfiguration API

Table 5.9.1.1-1 provides an overview of the resources and HTTP methods applicable for the IPTVConfiguration API.

Table 5.9.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| IPTV Configurations | /{afId}/configurations | GET | Read all configurations for a given AF. |
|  |  | POST | Create a new IPTV configuration. |
| Individual IPTV Configuration | /{afId}/configurations/{configurationId} | GET | Read an existing configuration identified by {configurationId}. |
|  |  | PUT | Update an existing configuration identified by {configurationId}. |
|  |  | PATCH | Modify an existing configuration identified by {configurationId}. |
|  |  | DELETE | Delete a configuration identified by {configurationId}. |

#### 5.9.1.2	Resource: IPTV Configurations

##### 5.9.1.2.1	Introduction

This resource allows a AF to read all active IPTV configurations for the given AF, or create an new individual IPTV configuration in the NEF.

##### 5.9.1.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-iptvconfiguration/v1/{afId}/configurations

This resource shall support the resource URI variables defined in table 5.9.1.2.2-1.

Table 5.9.1.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |

##### 5.9.1.2.3	Resource Methods

###### 5.9.1.2.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.9.1.2.2.

###### 5.9.1.2.3.2	GET

The GET method allows to read all active configurations for a given AF. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.9.1.2.3.2-1.

Table 5.9.1.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.9.1.2.3.2-2 and the response data structures and response codes specified in table 5.9.1.2.3.2-3.

Table 5.9.1.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.9.1.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(IptvConfigData) | M | 0..N | 200 OK | All the configuration information for the AF in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.9.1.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.9.1.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.9.1.2.3.3	POST

The POST method creates a new resource to individual IPTV configuration for a given AF. The AF shall initiate the HTTP POST request message and the NEF shall respond to the message. The NEF shall construct the URI of the created resource.

This method shall support the request data structures specified in table 5.9.1.2.3.3-1 and the response data structures and response codes specified in table 5.9.1.2.3.3-2.

Table 5.9.1.2.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| IptvConfigData | M | 1 | Parameters to create an IPTV Configuration resource. |

Table 5.9.1.2.3.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| IptvConfigData | M | 1 | 201 Created | The configuration resource was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.9.1.2.3.3-3: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-iptvconfiguration/v1/{afId}/configurations/{configurationId} |

#### 5.9.1.3	Resource: Individual IPTV Configuration

##### 5.9.1.3.1	Introduction

This resource allows a AF to read, update or delete an existing IPTV Configuration.

##### 5.9.1.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-iptvconfiguration/v1/{afId}/configurations/{configurationId}

This resource shall support the resource URI variables defined in table 5.9.1.3.2-1.

Table 5.9.1.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |
| configurationId | string | Identifier of the configuration resource. |

##### 5.9.1.3.3	Resource Methods

###### 5.9.1.3.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.9.1.3.2.

###### 5.9.1.3.3.2	GET

The GET method allows to read the active configuration for a given AF and subscription Id. The AF shall initiate the HTTP GET request message and theNEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.9.1.3.3.2-1.

Table 5.9.1.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.9.1.3.3.2-2 and the response data structures and response codes specified in table 5.9.1.3.3.2-3.

Table 5.9.1.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.9.1.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| IptvConfigData | M | 1 | 200 OK | The information for the configuration in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.9.1.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.9.1.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.9.1.3.3.3	PUT

The PUT method modifies an existing resource to update a configuration. The AF shall initiate the HTTP PUT request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.9.1.3.3.3-1 and the response data structures and response codes specified in table 5.9.1.3.3.3-2.

Table 5.9.1.3.3.3-1: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| IptvConfigData | M | 1 | Modify an existing configuration. |

Table 5.9.1.3.3.3-2: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| IptvConfigData | M | 1 | 200 OK | The configuration resource was updated successfully. |
| n/a |  |  | 204 No Content | The configuration resource was updated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during resource modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during resource modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.9.1.3.3.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.9.1.3.3.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.9.1.3.3.4	DELETE

The DELETE method deletes an existing individual configuration for a given AF. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.9.1.3.3.4-1.

Table 5.9.1.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.9.1.3.3.4-2 and the response data structures and response codes specified in table 5.9.1.3.3.4-3.

Table 5.9.1.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.9.1.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The configuration resource was terminated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.9.1.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.9.1.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.9.1.3.3.5	PATCH

The PATCH method allows to change some properties of an existing resource to update a configuration. The AF shall initiate the HTTP PATCH request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.9.1.3.3.5-1 and the response data structures and response codes specified in table 5.9.1.3.3.5-2.

Table 5.9.1.3.3.5-1: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| IptvConfigDataPatch | M | 1 | Partial update an existing configuration. |

Table 5.9.1.3.3.5-2: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| IptvConfigData | M | 1 | 200 OK | The configuration resource was updated successfully. |
| n/a |  |  | 204 No Content | The configuration resource was updated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during resource modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during resource modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.9.1.3.3.5-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.9.1.3.3.5-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

### 5.9.1A	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.9.1B	Notifications

There are no notifications defined for this API in this release of the specification.

### 5.9.2	Data Model

#### 5.9.2.1	General

This clause specifies the application data model supported by the IPTVConfiguration API.

Table 5.9.2.1-1 specifies the data types defined for the IPTVConfiguration API.

Table 5.9.2.1-1: IPTVConfiguration API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AccessRightStatus | 5.9.2.4.3 | Represents the access right status for parameter provision. |  |
| IptvConfigData | 5.9.2.3.2 | Represents an individual IPTV Configuration resource. |  |
| IptvConfigDataPatch | 5.9.2.3.4 | Represents the parameters to request the modification of an IPTV Configuration resource. |  |
| MulticastAccessControl | 5.9.2.3.3 | Represents multicast address access control information. |  |

#### 5.9.2.2	Reused data types

The data types reused by the IPTVConfiguration API from other specifications are listed in table 5.9.2.2-1.

Table 5.9.2.2-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Dnn | 3GPP TS 29.571 [8] | Identifies a DNN. |
| ExternalGroupId | 3GPP TS 29.122 [4] | External Group Identifier for a user group. |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |
| Ipv4Addr | 3GPP TS 29.571 [8] | Identifies an IPv4 address. |
| Ipv6Addr | 3GPP TS 29.571 [8] | Identifies an IPv6 address. |
| Link | 3GPP TS 29.122 [4] | Represents a referenced resource. |
| MtcProviderInformation | 3GPP TS 29.571 [8] | Indicates MTC provider information. |
| Snssai | 3GPP TS 29.571 [8] | Identifies the S-NSSAI. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.9.3-1. |

#### 5.9.2.3	Structured data types

##### 5.9.2.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.9.2.3.2	Type: IptvConfigData

Table 5.9.2.3.2-1: Definition of type IptvConfigData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| self | Link | C | 0..1 | Identifies the individual IPTV configuration resource URI. Shall be present in the HTTP GET response when reading all the configurations for an AF. |  |
| gpsi | Gpsi | C | 0..1 | Identifies GPSI. (NOTE) |  |
| exterGroupId | ExternalGroupId | C | 0..1 | Represents a group of users. (NOTE) |  |
| afAppId | string | M | 1 | Identifies an application. |  |
| dnn | Dnn | O | 0..1 | Identifies a DNN, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. |  |
| snssai | Snssai | O | 0..1 | Identifies an S-NSSAI. |  |
| multiAccCtrls | map(MulticastAccessControl) | M | 1..N | Identifies a list of multicast address access control information. Any string value can be used as a key of the map. |  |
| mtcProviderId | MtcProviderInformation | O | 0..1 | Indicates MTC provider information. |  |
| suppFeat | SupportedFeatures | M | 1 | Indicates the negotiated supported features. |  |
| NOTE: Only one of the "gpsi" or "exterGroupId" attribute shall be provided. |  |  |  |  |  |

##### 5.9.2.3.3	Type: MulticastAccessControl

Table 5.9.2.3.3-1: Definition of type MulticastAccessControl

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| srcIpv4Addr | Ipv4Addr | O | 0..1 | Identifies the source IPv4 address of IPTV multicast channel. |  |
| srcIpv6Addr | Ipv6Addr | O | 0..1 | Identifies the source IPv6 address of IPTV multicast channel. |  |
| multicastV4Addr | Ipv4Addr | O | 0..1 | Identifies the multicast IPv4 address of IPTV multicast channel. (NOTE) |  |
| multicastV6Addr | Ipv6Addr | O | 0..1 | Identifies the multicast IPv6 address of IPTV multicast channel. (NOTE) |  |
| accStatus | AccessRightStatus | M | 1 | Represents access right status of the multicast channel. |  |
| NOTE: At least one of the "multicastV4Addr" or "multicastV6Addr" attribute shall be provided. |  |  |  |  |  |

##### 5.9.2.3.4	Type: IptvConfigDataPatch

Table 5.9.2.3.4-1: Definition of type IptvConfigDataPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| multiAccCtrls | map(MulticastAccessControl) | O | 1..N | Identifies a list of multicast address access control information. Any string value can be used as a key of the map. |  |

#### 5.9.2.4	Simple data types and enumerations

##### 5.9.2.4.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.9.2.4.2	Simple data types

The simple data types defined in table 5.9.2.4.2-1 shall be supported.

Table 5.9.2.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 5.9.2.4.3	Enumeration: AccessRightStatus

The enumeration AccessRightStatus represents the parameters provision type of which the AF requests to provision. It shall comply with the provisions defined in table 5.9.2.4.3-1.

Table 5.9.2.4.3-1: Enumeration AccessRightStatus

| Enumeration value | Description |
| --- | --- |
| FULLY_ALLOWED | The User is fully allowed to access to the channel. |
| PREVIEW_ALLOWED | The User is preview allowed to access to the channel. |
| NO_ALLOWED | The User is not allowed to access to the channel. |

### 5.9.3	Used Features

The table below defines the features applicable to the IPTVConfiguration API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.9.3-1: Features used by IPTVConfiguration API

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 5.9.4	Error handling

#### 5.9.4.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.9.4.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the IPTVConfiguration API.

#### 5.9.4.3	Application Errors

The application errors defined for IPTVConfiguration API are listed in table 5.9.4.3-1.

Table 5.9.4.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

## 5.10	LpiParameterProvision API

### 5.10.0	Introduction

The Nnef_ParameterProvision service shall use the LpiParameterProvision API for LPI parameters provisioning.

The API URI of LpiParameterProvision API shall be:

- {apiRoot}/3gpp-lpi-pp/v1

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-lpi-pp".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.10.1	Resources

#### 5.10.1.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.10.1.1-1 and the resources and HTTP methods used for the LpiParameterProvision API.

Figure 5.10.1.1-1: Resource URI structure of the LpiParameterProvision API

Table 5.10.1.1-1 provides an overview of the resources and HTTP methods applicable for the LpiParameterProvision API.

Table 5.10.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| LPI Parameters Provisionings | /{afId}/provisionedLpis (NOTE) | GET | Read all LPI Parameters Provisioning resources for a given AF. |
|  |  | POST | Create a new Individual LPI Parameters Provisioning resource. |
| Individual LPI Parameters Provisioning | /{afId}/provisionedLpis/{provisionedLpiId} (NOTE) | GET | Read an existing Individual LPI Parameters Provisioning resource identified by {provisionedLpiId}. |
|  |  | PUT | Update an existing Individual LPI Parameters Provisioning resource identified by {provisionedLpiId}. |
|  |  | PATCH | Modify an existing Individual LPI Parameters Provisioning resource identified by {provisionedLpiId}. |
|  |  | DELETE | Delete an existing Individual LPI Parameters Provisioning resource identified by {provisionedLpiId}. |
| NOTE: The path segment "provisionedLpis" does not follow the related naming convention defined in clause 5.2. The path segment is however kept as currently defined in this specification for backward compatibility considerations. |  |  |  |

#### 5.10.1.2	Resource: LPI Parameters Provisionings

##### 5.10.1.2.1	Introduction

This resource allows a AF to read all active LPI Parameters Provisionings for the given AF, or create an new individual LPI Parameters Provisioning resource to provision parameters to the NEF.

##### 5.10.1.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-lpi-pp/v1/{afId}/provisionedLpis

This resource shall support the resource URI variables defined in table 5.10.1.2.2-1.

Table 5.10.1.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |

##### 5.10.1.2.3	Resource Methods

###### 5.10.1.2.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.10.1.2.3.

###### 5.10.1.2.3.2	GET

The GET method allows to read all active LPI Parameters Provisioning resources for a given AF. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.10.1.2.3.2-1.

Table 5.10.1.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.10.1.2.3.2-2 and the response data structures and response codes specified in table 5.10.1.2.3.2-3.

Table 5.10.1.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.10.1.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(LpiParametersProvision) | M | 0..N | 200 OK | All the LPI Parameters Provisioning information for the AF in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.10.1.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.10.1.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.10.1.2.3.3	POST

The POST method creates a new resource to LPI Parameters Provisionings for a given AF. The AF shall initiate the HTTP POST request message and the NEF shall respond to the message. The NEF shall construct the URI of the created resource.

This method shall support the request data structures specified in table 5.10.1.2.3.3-1 and the response data structures and response codes specified in table 5.10.1.2.3.3-2.

Table 5.10.1.2.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LpiParametersProvision | M | 1 | Parameters to create an Individual LPI Parameters Provisioning resource to provision parameters. |

Table 5.10.1.2.3.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LpiParametersProvision | M | 1 | 201 Created | The resource was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.10.1.2.3.3-3: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-lpi-pp/v1/{afId}/provisionedLpis/{provisionedLpiId} |

#### 5.10.1.3	Resource: Individual LPI Parameters Provisioning

##### 5.10.1.3.1	Introduction

This resource allows a AF to read, update or delete an existing Individual LPI Parameters Provisioning resource.

##### 5.10.1.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-lpi-pp/v1/{afId}/provisionedLpis/{provisionedLpiId}

This resource shall support the resource URI variables defined in table 5.10.1.3.2-1.

Table 5.10.1.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |
| provisionedLpiId | string | Identifier of the provisioning resource. |

##### 5.10.1.3.3	Resource Methods

###### 5.10.1.3.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.10.1.3.3.

###### 5.10.1.3.3.2	GET

The GET method allows to read an active Individual LPI Parameters Provisioning resource for a given AF and provisionedLpiId. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.10.1.3.3.2-1.

Table 5.10.1.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.10.1.3.3.2-2 and the response data structures and response codes specified in table 5.10.1.3.3.2-3.

Table 5.10.1.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.10.1.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LpiParametersProvision | M | 1 | 200 OK | The information for the source in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.10.1.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.10.1.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.10.1.3.3.3	PUT

The PUT method updates an existing resource to update an existing Individual LPI Parameters Provisioning resource. The AF shall initiate the HTTP PUT request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.10.1.3.3.3-1 and the response data structures and response codes specified in table 5.10.1.3.3.3-2.

Table 5.10.1.3.3.3-1: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LpiParametersProvision | M | 1 | Update an existing individual LPI Parameters Provisioning resource to provision parameters. |

Table 5.10.1.3.3.3-2: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LpiParametersProvision | M | 1 | 200 OK | The resource was updated successfully and a representation of the updated resource is returned. |
| n/a |  |  | 204 No Content | The resource was updated successfully and no additional content is sent in the response message. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during resource modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.10.1.3.3.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.10.1.3.3.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.10.1.3.3.3A	PATCH

The PATCH method modifies an existing resource to update an existing individual LPI Parameters Provisioning resource. The AF shall initiate the HTTP PATCH request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.10.1.3.3.3A-1 and the response data structures and response codes specified in table 5.10.1.3.3.3A-2.

Table 5.10.1.3.3.3A-1: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LpiParametersProvisionPatch | M | 1 | Modify an existing individual LPI Parameters Provisioning resource to provision parameters. |

Table 5.10.1.3.3.3A-2: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LpiParametersProvision | M | 1 | 200 OK | The resource was updated successfully and a representation of the updated resource is returned. |
| n/a |  |  | 204 No Content | The resource was updated successfully and no additional content is sent in the response message. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.10.1.3.3.3A-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.10.1.3.3.3A-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.10.1.3.3.4	DELETE

The DELETE method deletes an existing individual LPI Parameters Provisioning resource for a given AF. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.10.1.3.3.4-1.

Table 5.10.1.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.10.1.3.3.4-2 and the response data structures and response codes specified in table 5.10.1.3.3.4-3.

Table 5.10.1.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.10.1.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The resource was removed successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.10.1.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.10.1.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

### 5.10.1A	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.10.1B	Notifications

There are no notifications defined for this API in this release of the specification.

### 5.10.2	Data Model

#### 5.10.2.1	General

This clause specifies the application data model supported by the LpiParameterProvision API.

Table 5.10.2.1-1 specifies the data types defined for the LpiParameterProvision API.

Table 5.10.2.1-1: LpiParameterProvision API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| LpiParametersProvision | 5.10.2.3.2 | Represents an individual LPI Parameters Provisioning resource. |  |
| LpiParametersProvisionPatch | 5.10.2.3.3 | Represents the requested modifications to an LPI Parameters Provisioning. | PatchUpdate |

#### 5.10.2.2	Reused data types

The data types reused by the LpiParameterProvision API from other specifications are listed in table 5.10.2.2-1.

Table 5.10.2.2-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| ExternalGroupId | 3GPP TS 29.122 [4] | External Group Identifier for a user group. |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |
| Link | 3GPP TS 29.122 [4] | Identifies a referenced resource. |
| Lpi | 3GPP TS 29.503 [17] | Identifies the Location Privacy Indication information. |
| MtcProviderInformation | 3GPP TS 29.571 [8] | Indicates MTC provider information for LCS privacy parameter configuration authorization. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.10.3-1. |

#### 5.10.2.3	Structured data types

##### 5.10.2.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.10.2.3.2	Type: LpiParametersProvision

Table 5.10.2.3.2-1: Definition of type LpiParametersProvision

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| self | Link | C | 0..1 | Identifies the individual parameters provisioning resource. This attribute shall be supplied by the NEF in HTTP responses that include an object of LpiParametersProvision type. |  |
| exterGroupId | ExternalGroupId | O | 0..1 | Identifies a group of UEs. (NOTE 1) |  |
| gpsi | Gpsi | O | 0..1 | Identifies an UE with GPSI. (NOTE 1)) |  |
| lpi | Lpi | M | 1 | Location Privacy Indication parameters |  |
| mtcProviderId | MtcProviderInformation | O | 0..1 | Indicates MTC provider information for LCS privacy parameter configuration authorization. (NOTE 2)) |  |
| suppFeat | SupportedFeatures | M | 1 | Indicates the negotiated supported features. |  |
| NOTE 1: Only one of the "gpsi" or "exterGroupId" attribute shall be provided. NOTE 2: The NEF should check received MTC Provider information and then the NEF may: - override it with local configured value and send it to UDM; - send it directly to the UDM; or - reject the LPI Parameter Provisioning request. |  |  |  |  |  |

##### 5.10.2.3.3	Type: LpiParametersProvisionPatch

Table 5.10.2.3.3-1: Definition of type LpiParametersProvisionPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| lpi | Lpi | O | 0..1 | Location Privacy Indication parameters |  |
| mtcProviderId | MtcProviderInformation | O | 0..1 | Indicates MTC provider information for LCS privacy parameter configuration authorization. (NOTE 1) |  |
| NOTE 1: The NEF should check received MTC Provider information and then the NEF may: - override it with local configured value and send it to UDM; - send it directly to the UDM; or - reject the LPI Parameter Provisioning modification request. |  |  |  |  |  |

#### 5.10.2.4	Simple data types and enumerations

##### 5.10.2.4.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.10.2.4.2	Simple data types

The simple data types defined in table 5.10.2.4.2-1 shall be supported.

Table 5.10.2.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

### 5.10.3	Used Features

The table below defines the features applicable to the LpiParameterProvision API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.10.3-1: Features used by LpiParameterProvision API

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | PatchUpdate | Indicates the support of enhancements to the northbound interfaces (e.g. support the partial modification of an existing resource). |

### 5.10.4	Error handling

#### 5.10.4.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.10.4.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the LpiParameterProvision API.

#### 5.10.4.3	Application Errors

The application errors defined for LpiParameterProvision API are listed in table 5.10.4.3-1.

Table 5.10.4.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

## 5.11	ServiceParameter API

### 5.11.0	Introduction

The Nnef_ServiceParameter service shall use the ServiceParameter API.

The API URI of ServiceParameter API shall be:

- {apiRoot}/3gpp-service-parameter/v1

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-service-parameter".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.11.1	Resources

#### 5.11.1.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.11.1.1-1 and the resources and HTTP methods used for the ServiceParameter API.

Figure 5.11.1.1-1: Resource URI structure of the ServiceParameter API

Table 5.11.1.1-1 provides an overview of the resources and HTTP methods applicable for the ServiceParameter API.

Table 5.11.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Service Parameter Subscripions | /{afId}/subscriptions | GET | Read all subscriptions for a given AF. |
|  |  | POST | Create a new service parameter subscription. |
| Individual Service Parameter Subscripion | /{afId}/subscriptions/{subscriptionId} | GET | Read an existing service parameter subscription identified by {subscriptionId}. |
|  |  | PUT | Update an existing service parameter subscription identified by {subscriptionId}. |
|  |  | PATCH | Modify an existing service parameter subscription identified by {subscriptionId}. |
|  |  | DELETE | Delete an existing service parameter subscription identified by {subscriptionId}. |

#### 5.11.1.2	Resource: Service Parameter Subscriptions

##### 5.11.1.2.1	Introduction

This resource allows a AF to read all active Service Parameter Subscriptions for the given AF, or create an new individual service parameter subscription in the NEF.

##### 5.11.1.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-service-parameter/v1/{afId}/subscriptions

This resource shall support the resource URI variables defined in table 5.11.1.2.2-1.

Table 5.11.1.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |

##### 5.11.1.2.3	Resource Methods

###### 5.11.1.2.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.11.1.2.3.

###### 5.11.1.2.3.2	GET

The HTTP GET method allows to read all the active subscriptions for a given AF.

This method shall support the URI query parameters specified in table 5.11.1.2.3.2-1.

Table 5.11.1.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| gpsis | array(Gpsi) | O | 1..N | Contains the GPSI(s) of the requested UE(s). (NOTE 1, NOTE 2) | enNB |
| ip-addrs | array(IpAddr) | O | 1..N | Contains the IP address(es) of the requested UE(s). (NOTE 1, NOTE 2) | enNB |
| ip-domain | string | O | 1 | Contains the IPv4 address domain identifier. This query parameter may be present only if the "ip-addrs" query parameter is also present and contains at least one array element including an IPv4 address. | enNB |
| mac-addrs | array(MacAddr48) | O | 1..N | Contains the MAC address(es) of the requested UE(s). (NOTE 1, NOTE 2) | enNB |
| NOTE 1: These query parameters are mutually exclusive. Either one of them may be present. NOTE 2: If multiple array elements are provided within this query parameter, then each array element shall be treated as a separate query parameter. |  |  |  |  |  |

This method shall support the request data structures specified in table 5.11.1.2.3.2-2 and the response data structures and response codes specified in table 5.11.1.2.3.2-3.

Table 5.11.1.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.11.1.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(ServiceParameterData) | M | 0..N | 200 OK | All the subscription information for the AF in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.11.1.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

Table 5.11.1.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

###### 5.11.1.2.3.3	POST

The POST method creates a new resource to individual service parameter subscription for a given AF. The AF shall initiate the HTTP POST request message and the NEF shall respond to the message. The NEF shall construct the URI of the created resource.

This method shall support the request data structures specified in table 5.11.1.2.3.3-1 and the response data structures and response codes specified in table 5.11.1.2.3.3-2.

Table 5.11.1.2.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ServiceParameterData | M | 1 | Parameters to create a service parameter subscription resource. |

Table 5.11.1.2.3.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServiceParameterData | M | 1 | 201 Created | The subscription resource was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.11.1.2.3.3-3: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-service-parameter/v1/{afId}/subscriptions/{subscriptionId} |

#### 5.11.1.3	Resource: Individual Service Parameter Subscription

##### 5.11.1.3.1	Introduction

This resource allows a AF to read, update or delete an existing service parameter subscription.

##### 5.11.1.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-service-parameter/v1/{afId}/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 5.9.1.3.2-1.

Table 5.11.1.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |
| subscriptionId | string | Identifier of the subscription resource. |

##### 5.11.1.3.3	Resource Methods

###### 5.11.1.3.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.11.1.3.3.

###### 5.11.1.3.3.2	GET

The GET method allows to read the active subscription for a given AF and subscription Id. The AF shall initiate the HTTP GET request message and theNEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.11.1.3.3.2-1.

Table 5.11.1.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.11.1.3.3.2-2 and the response data structures and response codes specified in table 5.11.1.3.3.2-3.

Table 5.11.1.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.11.1.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServiceParameterData | M | 1 | 200 OK | The information for the subscription in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.11.1.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.11.1.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.11.1.3.3.3	PUT

The PUT method modifies an existing resource to update a configuration. The AF shall initiate the HTTP PUT request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.9.1.3.3.3-1 and the response data structures and response codes specified in table 5.9.1.3.3.3-2.

Table 5.11.1.3.3.3-1: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ServiceParameterData | M | 1 | Modify an existing subscription. |

Table 5.11.1.3.3.3-2: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServiceParameterData | M | 1 | 200 OK | The subscription resource was updated successfully. |
| n/a |  |  | 204 No Content | The subscription resource was updated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.11.1.3.3.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.11.1.3.3.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.11.1.3.3.4	DELETE

The DELETE method deletes an existing individual subscription for a given AF. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.11.1.3.3.4-1.

Table 5.11.1.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.11.1.3.3.4-2 and the response data structures and response codes specified in table 5.11.1.3.3.4-3.

Table 5.11.1.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.11.1.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The subscription resource was terminated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.11.1.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.11.1.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.11.1.3.3.5	PATCH

The PATCH method allows to change some properties of an existing resource to update a subscription. The AF shall initiate the HTTP PATCH request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.11.1.3.3.5-1 and the response data structures and response codes specified in table 5.11.1.3.3.5-2.

Table 5.11.1.3.3.5-1: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ServiceParameterDataPatch | M | 1 | Partial update an existing subscription. |

Table 5.11.1.3.3.5-2: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServiceParameterData | M | 1 | 200 OK | The subscription resource was updated successfully. |
| N/A |  |  | 204 No Content | The subscription resource was updated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.11.1.3.3.5-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.11.1.3.3.5-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

### 5.11.1A	Notifications

#### 5.11.1A.1	Introduction

Upon NEF notify a Service Parameter Authorization Update (e.g. to revoke an authorization) to AF, or forward a notification event related to invocation of service parameter provisioning, e.g. the notification of outcome of UE Policies Delivery to AF, the NEF shall send an HTTP POST message including the notified event to the AF. The NEF and the AF shall support the notification mechanism as described in clause 5.2.5 of 3GPP TS 29.122 [4].

Table 5.11.1A.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| AF Notifications | {notificationDestination} | POST | The notification of service parameter authorization updates and/or notification event related to the result of invocation of service parameter provisioning. |

#### 5.11.1A.2	AF Notifications

##### 5.11.1A.2.1	Description

The AF Notifications are used by the NEF to send the Service Parameter Authorization Update (e.g. to revoke an authorization) to the AF, and/or to send AF subscribed event notification of the outcome related to the invocation of service parameter provisioning to the AF.

##### 5.11.1A.2.2	Target URI

The Callback URI "{notificationDestination}" shall be used with the callback URI variables defined in table 5.11.1A.2.2-1.

Table 5.11.1A.2.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notificationDestination | Link | Callback reference provided by the AF during creation/modification of the subscription within the ServiceParameterData data type as defined in Table 5.11.2.3.2-1 or the ServiceParameterDataPatch data type as defined in Table 5.11.2.3.3-1. |

#### 5.11.1A.3	Operation Definition

##### 5.11.1A.3.1	Notification via HTTP POST

This method shall support the request data structures specified in table 5.11.1A.3.1-1 and the response data structure with response codes specified in table 5.11.1A.3.1-2.

Table 5.11.1A.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(AfNotification) | M | 1..N | Notifications upon AF Service Parameter Authorization Update, and/or AF subscribed event notification of the outcome related to the invocation of service parameter provisioning. |

Table 5.11.1A.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The event notification is received successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during event notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during event notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.11.1A.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

Table 5.11.1A.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

##### 5.11.1A.3.2	Notification via Websocket

If supported by both AF and NEF and successfully negotiated, the AF subscribed outcome event notification related to the invocation of service parameter provisioning may alternatively be delivered through the Websocket mechanism as defined in clause 5.2.5.4 of 3GPP TS 29.122 [4].

### 5.11.1B	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.11.2	Data Model

#### 5.11.2.1	General

This clause specifies the application data model supported by the ServiceParameter API.

Table 5.11.2.1-1 specifies the data types defined for the ServiceParameter API.

Table 5.11.2.1-1: ServiceParameter API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| A2xParamsPc5 | 5.11.2.4.2 | Represents the service parameters for A2X communication over PC5 reference point. | A2X |
| A2xParamsPc5Rm | 5.11.2.4.2 | This data type is defined in the same way as the A2xParamsPc5 data type, but with the OpenAPI nullable property set to true. | A2X |
| A2xParamsUu | 5.11.2.4.2 | Represents the service parameters for A2X communication over Uu reference point. | A2X |
| A2xParamsUuRm | 5.11.2.4.2 | This data type is defined in the same way as the A2xParamsUu data type, but with the OpenAPI nullable property set to true. | A2X |
| AfNotification | 5.11.2.3.6 | Contains the reported event notification or the service parameters authorization update result. | AfNotifications |
| AuthorizationResult | 5.11.2.4.4 | Represents the result of the service parameters authorization. | AfNotifications |
| ConnectionCapabilities | 5.11.2.4.6 | UE application requests a network connection with certain capabilities. | AfGuideURSP |
| Event | 5.11.2.4.3 | Identifies the AF subscribed events. | AfNotifications |
| EventInfo | 5.11.2.3.7 | Indicates the event information. | AfNotifications |
| Failure | 5.11.2.4.5 | Represents the failure reason for the unsuccessful result. | AfNotifications |
| NetworkDescription | 5.11.2.3.9 | Represents the description of a PLMN, by the definition of the PLMN ID, the MCC (and optionally applicable MNC(s)) or the indication of any PLMN. | VPLMNSpecificURSP |
| ParameterOverPc5 | 5.11.2.4.2 | Represents configuration parameters for V2X communications over PC5 reference point. |  |
| ParameterOverPc5Rm | 5.11.2.4.2 | Represents the same as the ParameterOverPc5 data type but with the "nullable: true" property. |  |
| ParameterOverUu | 5.11.2.4.2 | Represents configuration parameters for V2X communications over Uu reference point. |  |
| ParameterOverUuRm | 5.11.2.4.2 | Represents the same as the ParameterOverUu data type but with the "nullable: true" property. |  |
| ParamForProSeDc | 5.11.2.4.2 | Represents the service parameters for 5G ProSe direct communications. | ProSe |
| ParamForProSeDcRm | 5.11.2.4.2 | This data type is defined in the same way as the ParamForProSeDc data type, but with the OpenAPI nullable property set to true. | ProSe |
| ParamForProSeDd | 5.11.2.4.2 | Represents the service parameters for 5G ProSe direct discovery. | ProSe |
| ParamForProSeDdRm | 5.11.2.4.2 | This data type is defined in the same way as the ParamForProSeDd data type, but with the OpenAPI nullable property set to true. | ProSe |
| ParamForProSeEndUe | 5.11.2.4.2 | Represents the service parameters for 5G ProSe end UE. | ProSe_Ph2 |
| ParamForProSeEndUeRm | 5.11.2.4.2 | This data type is defined in the same way as the ParamForProSeEndUe data type, but with the OpenAPI nullable property set to true. | ProSe_Ph2 |
| ParamForProSeRemUe | 5.11.2.4.2 | Represents the service parameters for 5G ProSe remote UE. | ProSe |
| ParamForProSeRemUeRm | 5.11.2.4.2 | This data type is defined in the same way as the ParamForProSeRemUe data type, but with the OpenAPI nullable property set to true. | ProSe |
| ParamForProSeU2NRelUe | 5.11.2.4.2 | Represents the service parameters for 5G ProSe UE-to-network relay UE. | ProSe |
| ParamForProSeU2NRelUeRm | 5.11.2.4.2 | This data type is defined in the same way as the ParamForProSeU2NRelUe data type, but with the OpenAPI nullable property set to true. | ProSe |
| ParamForProSeU2URelUe | 5.11.2.4.2 | Represents the service parameters for 5G ProSe UE-to-UE relay UE. | ProSe_Ph2 |
| ParamForProSeU2URelUeRm | 5.11.2.4.2 | This data type is defined in the same way as the ParamForProSeU2URelUe data type, but with the OpenAPI nullable property set to true. | ProSe_Ph2 |
| ParamForRangingSlPos | 5.11.2.4.2 | Represents the service parameters for ranging and sidelink positioning. | Ranging_SL |
| ParamForRangingSlPosRm | 5.11.2.4.2 | This data type is defined in the same way as the ParamForRangingSlPos data type, but with the OpenAPI nullable property set to true. | Ranging_SL |
| RouteSelectionParameterSet | 5.11.2.3.5 | Contains parameters that can be used to guide the Route Selection Descriptors of the URSP. | AfGuideURSP |
| ServiceParameterData | 5.11.2.3.2 | Represents an individual Service Parameter subscription resource. |  |
| ServiceParameterDataPatch | 5.11.2.3.3 | Represents the parameters to request the modification of a service parameter subscription resource. |  |
| TrafficDescriptorComponents | 5.11.2.3.8 | Traffic descriptor components for the requested URSP. | AfGuideURSP |
| UrspRuleRequest | 5.11.2.3.4 | Contains parameters that can be used to guide the URSP. | AfGuideURSP |

#### 5.11.2.2	Reused data types

The data types reused by the ServiceParameter API from other specifications are listed in table 5.9.2.2-1.

Table 5.11.2.2-1: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AppDescriptor | 5.7.2.3.4 | Application descriptor describes the operation systems and the corresponding applications for each operation systems. |  |
| Bytes | 3GPP TS 29.571 [8] | Represents a string with format "byte". | ExtConnCapability |
| Dnn | 3GPP TS 29.571 [8] | Identifies a DNN. |  |
| EthFlowDescription | 3GPP TS 29.514 [7] | Defines a packet filter for an Ethernet flow. |  |
| ExternalGroupId | 3GPP TS 29.122 [4] | External Group Identifier for a user group. |  |
| GeographicalArea | 5.17.3.3.4 | Identifies the geographical area information. | AfGuideURSP |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |  |
| IpAddr | 3GPP TS 29.571 [8] | UE IP Address. |  |
| IPv4Addr | 3GPP TS 29.571 [8] | Identifies an IPv4 address. |  |
| IPv6Addr | 3GPP TS 29.571 [8] | Identifies an IPv6 address. |  |
| Link | 3GPP TS 29.122 [4] | Represents a referenced resource. |  |
| MacAddr48 | 3GPP TS 29.571 [8] | Identifies an MAC address. |  |
| Mcc | 3GPP TS 29.571 [8] | Mobile Country Code. |  |
| Mnc | 3GPP TS 29.571 [8] | Mobile Network Code. |  |
| PduSessionType | 3GPP TS 29.571 [8] | Represents the PDU session type. | PduSessTypeChange |
| PlmnId | 3GPP TS 29.571 [8] | Identifies a PLMN Identifier. |  |
| PlmnIdNid | 3GPP TS 29.571 [8] | Identifies a PLMN Identifier and optionally the Network Identity | PlmnIdNid |
| MtcProviderInformation | 3GPP TS 29.571 [8] | Indicates MTC provider information. |  |
| Snssai | 3GPP TS 29.571 [8] | Identifies the S-NSSAI. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.11.3-1. |  |
| Tai | 3GPP TS 29.571 [8] | Tracking Area Identity information. |  |
| TnapId | 3GPP TS 29.571 [8] | Trusted Network Access Point identifier. |  |
| Uinteger | 3GPP TS 29.571 [8] | Unsigned integer. |  |

#### 5.11.2.3	Structured data types

##### 5.11.2.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.11.2.3.2	Type: ServiceParameterData

Table 5.11.2.3.2-1: Definition of type ServiceParameterData

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| self |  | Link |  | C |  | 0..1 |  | Identifies the individual service parameter subscription resource URI. Shall be present by the NEF in HTTP responses that include an object of ServiceParameterData type. |  |  |  |
| dnn |  | Dnn |  | O |  | 0..1 |  | Identifies a DNN. (NOTE 2) (NOTE 3) |  |  |  |
| snssai |  | Snssai |  | O |  | 0..1 |  | Identifies an S-NSSAI. (NOTE 2) (NOTE 3) |  |  |  |
| afServiceId |  | string |  | O |  | 0..1 |  | Identifies a service on behalf of which the AF is issuing the request. (NOTE 2) (NOTE 3) |  |  |  |
| appId |  | string |  | O |  | 0..1 |  | Identifies an application identifier. (NOTE 2) |  |  |  |
| gpsi |  | Gpsi |  | O |  | 0..1 |  | Identifies GPSI. (NOTE 1) |  |  |  |
| ueIpv4 |  | Ipv4Addr |  | O |  | 0..1 |  | The IPv4 address of the served UE. (NOTE 1) |  |  |  |
| ueIpv6 |  | Ipv6Addr |  | O |  | 0..1 |  | The IPv6 address of the served UE. (NOTE 1) |  |  |  |
| ueMac |  | MacAddr48 |  | O |  | 0..1 |  | The MAC address of the served UE. (NOTE 1) |  |  |  |
| externalGroupId |  | ExternalGroupId |  | O |  | 0..1 |  | Represents a group of users. (NOTE 1) |  |  |  |
| anyUeInd |  | boolean |  | O |  | 0..1 |  | Identifies whether the service parameters apply to any non-roaming UE. - "true": the service parameters are applicable to any non-roaming UE. - "false": the service parameters are not applicable to any non-roaming UE. - Default value is "false" if omitted. (NOTE 1) (NOTE 3) |  |  |  |
| roamUeNetDescs |  | array(NetworkDescription) |  | O |  | 1..N |  | Each element identifies one (e.g., combination of MCC and MNC) or more (e.g. a MCC only) PLMN ID(s). It indicates the PLMN(s) of inbound roamers to which the provided AF guidance on VPLMN-specific URSP rules apply. (NOTE 1) |  | VPLMNSpecificURSP |  |
| subNotifEvents |  | array(Event) |  | C |  | 1..N |  | Identifies the AF subscribed event(s) notifications related to AF provisioned service parameters. |  | AfNotifications |  |
| notificationDestination |  | Uri |  | C |  | 0..1 |  | Contains the callback URI to receive the notifications from the NEF. Shall be present If "subNotifEvents" attribute is included. |  | AfNotifications |  |
| requestTestNotification |  | boolean |  | O |  | 0..1 |  | Set to true by the AF to request the NEF to send a test notification as defined in clause 5.2.5.3 of 3GPP TS 29.122 [4]. The default value is "false" if omitted. |  | Notification_test_event |  |
| websockNotifConfig |  | WebsockNotifConfig |  | O |  | 0..1 |  | Configuration parameters to set up notification delivery over Websocket protocol. |  | Notification_websocket |  |
| paramOverPc5 |  | ParameterOverPc5 |  | O |  | 0..1 |  | Contains the V2X service parameters used over PC5 |  |  |  |
| paramOverUu |  | ParameterOverUu |  | O |  | 0..1 |  | Contains the V2X service parameters used over Uu |  |  |  |
| paramForProSeDd |  | ParamForProSeDd |  | O |  | 0..1 |  | Contains the service parameters for 5G ProSe direct discovery. |  | ProSe |  |
| paramForProSeDc |  | ParamForProSeDc |  | O |  | 0..1 |  | Contains the service parameters for 5G ProSe direct communications. |  | ProSe |  |
| paramForProSeU2NRelUe |  | ParamForProSeU2NRelUe |  | O |  | 0..1 |  | Contains the service parameters for 5G ProSe UE-to-network relay UE. |  | ProSe |  |
| paramForProSeRemUe |  | ParamForProSeRemUe |  | O |  | 0..1 |  | Contains the service parameters for 5G ProSe remote UE. |  | ProSe |  |
| paramForProSeU2URelUe |  | ParamForProSeU2URelUe |  | O |  | 0..1 |  | Contains the service parameters for 5G ProSe UE-to-UE relay UE. |  | ProSe_Ph2 |  |
| paramForProSeEndUe |  | ParamForProSeEndUe |  | O |  | 0..1 |  | Contains the service parameters for 5G ProSe end UE. |  | ProSe_Ph2 |  |
| paramForRangingSlPos |  | ParamForRangingSlPos |  | O |  | 0..1 |  | Contains the service parameters for ranging and sidelink positioning. |  | Ranging_SL |  |
| urspGuidance |  | array(UrspRuleRequest) |  | O |  | 1..N |  | Contains the service parameters used to guide the URSP rule(s). |  | AfGuideURSP |  |
| vpsUrspGuidance |  | array(UrspRuleRequest) |  | O |  | 1..N |  | Contains the service parameters provided by an AF to guide the VPLMN-specific URSP rule(s). |  | VPLMNSpecificURSP |  |
| a2xParamsPc5 |  | A2xParamsPc5 |  | O |  | 0..1 |  | Contains the A2X service parameters used over PC5 reference point. |  | A2X |  |
| a2xParamsUu |  | A2xParamsUu |  | O |  | 0..1 |  | Contains the A2X service parameters used over Uu reference point. |  | A2X |  |
| tnaps |  | array(TnapId) |  | O |  | 1..N |  | Contains the TNAP ID(s) collocated with the 5G-RG(s) of a specific user. |  | AfGuideTNAPs |  |
| mtcProviderId |  | MtcProviderInformation |  | O |  | 0..1 |  | Indicates MTC provider information. |  |  |  |
| suppFeat |  | SupportedFeatures |  | C |  | 0..1 |  | Indicates the list of Supported features used as described in clause 5.11.3. This attribute shall be provided in the POST request and in the response of successful resource creation. |  |  |  |
| NOTE 1: One of individual UE identifier (i.e. "gpsi", "ueIpv4", "ueIpv6" or "ueMac" attribute), External Group Identifier (i.e. "externalGroupId" attribute) or any UE indication (i.e. "anyUeInd" attribute), and when the feature "VPLMNSpecificURSP" is supported, or any inbound roaming UE from the indicated PLMN(s) (i.e., "roamUeNetDescs" attribute) shall be included. For V2X, Prose (when the "ProSe" and/or "ProSe_Ph2" feature is supported), A2X (when the "A2X" feature is supported) and URSP service parameter provisioning (see clause 4.4.20), only "anyUeInd", "gpsi" and "externalGroupId" attributes are applicable. When the "VPLMNSpecificURSP" feature is supported, the "roamUeNetDescs" attribute only applies to URSP service parameter provisioning and may be included when the "vpsUrspGuidance" attribute is provided. When the "AfGuideTNAPs" feature is supported, when TNAP ID(s) is provisioned within the "tnaps" attribute, only "gpsi" shall be provided. NOTE 2: Either the "afServiceId" attribute, "appId" attribute or the combination of "snssai" and "dnn" attributes shall be provided. When the feature "AfGuideURSP" is supported, only the "afServiceId" attribute shall be provided for providing guidance for URSP determination. When the feature "AfGuideTNAPs" is supported, when TNAP ID(s) is provisioned within the "tnaps" attribute, only the "afServiceId" attribute shall be provided. NOTE 3: When "anyUeInd" attribute is present, "appId" attribute, "afServiceId" attribute or the combination of "snssai" attribute and "dnn" attribute shall be provided. When the feature "AfGuideURSP" is supported, only the "afServiceId" attribute shall be provided for providing guidance for URSP determination. |  |  |  |  |  |  |  |  |  |  |  |

##### 5.11.2.3.3	Type: ServiceParameterDataPatch

Table 5.11.2.3.3-1: Definition of type ServiceParameterDataPatch

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| paramOverPc5 |  | ParameterOverPc5Rm |  | O |  | 0..1 |  | Contains the V2X service parameters used over PC5 |  |  |  |
| paramOverUu |  | ParameterOverUuRm |  | O |  | 0..1 |  | Contains the V2X service parameters used over Uu |  |  |  |
| paramForProSeDd |  | ParamForProSeDdRm |  | O |  | 0..1 |  | Contains the service parameters for 5G ProSe direct discovery. |  | ProSe |  |
| paramForProSeDc |  | ParamForProSeDcRm |  | O |  | 0..1 |  | Contains the service parameters for 5G ProSe direct communications. |  | ProSe |  |
| paramForProSeU2NRelUE |  | ParamForProSeU2NRelUeRm |  | O |  | 0..1 |  | Contains the service parameters for 5G ProSe UE-to-network relay UE. |  | ProSe |  |
| paramForProSeRemUe |  | ParamForProSeRemUeRm |  | O |  | 0..1 |  | Contains the service parameters for 5G ProSe remote UE. |  | ProSe |  |
| paramForProSeU2URelUE |  | ParamForProSeU2URelUeRm |  | O |  | 0..1 |  | Contains the service parameters for 5G ProSe UE-to-UE relay UE. |  | ProSe_Ph2 |  |
| paramForProSeEndUe |  | ParamForProSeEndUeRm |  | O |  | 0..1 |  | Contains the service parameters for 5G ProSe end UE. |  | ProSe_Ph2 |  |
| paramForRangingSlPos |  | ParamForRangingSlPosRm |  | O |  | 0..1 |  | Contains the service parameters for ranging and sidelink positioning. |  | Ranging_SL |  |
| urspGuidance |  | array(UrspRuleRequest) |  | O |  | 1..N |  | Contains the service parameters used to guide the URSP rule(s). |  | AfGuideURSP |  |
| vpsUrspGuidance |  | array(UrspRuleRequest) |  | O |  | 1..N |  | Contains the service parameters provided by an AF to guide the VPLMN-specific URSP rule(s). |  | VPLMNSpecificURSP |  |
| a2xParamsPc5 |  | A2xParamsPc5Rm |  | O |  | 0..1 |  | Contains the A2X service parameters used over PC5 reference point. |  | A2X |  |
| a2xParamsUu |  | A2xParamsUuRm |  | O |  | 0..1 |  | Contains the A2X service parameters used over Uu reference point. |  | A2X |  |
| tnaps |  | array(TnapId) |  | O |  | 1..N |  | Contains the TNAP ID(s) collocated with the 5G-RG(s) of a specific user. |  | AfGuideTNAPs |  |
| subNotifEvents |  | array(Event) |  | O |  | 1..N |  | Identifies the AF subscribed event(s) notifications related to AF provisioned service parameters. |  | AfNotifications |  |
| notificationDestination |  | Uri |  | O |  | 0..1 |  | Contains the callback URI to receive the notifications from the NEF. May be present If "subNotifEvents" attribute is included. |  | AfNotifications |  |

##### 5.11.2.3.4	Type: UrspRuleRequest

Table 5.11.2.3.4-1: Definition of type UrspRuleRequest

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| trafficDesc |  | TrafficDescriptorComponents |  | O |  | 0..1 |  | Traffic descriptor components for the requested URSP. (NOTE 1) |  |  |  |
| relatPrecedence |  | Uinteger |  | O |  | 0..1 |  | Represents the relative precedence of the URSP rule within the same AF request, Lower values take precedence over higher values. Its absence means that the AF has no relative precedence requirement for the provided URSP rules. |  |  |  |
| visitedNetDescs |  | array(NetworkDescription) |  | O |  | 1..N |  | It indicates the VPLMN(s) to which the provided AF guidance on VPLMN-specific URSP rules apply. (NOTE 3) |  | VPLMNSpecificURSP |  |
| routeSelParamSets |  | array(RouteSelectionParameterSet) |  | O |  | 1..N |  | Route Selection Parameter Sets, i.e. sets of parameters that may be used to guide the Route Selection Descriptors of the URSP. (NOTE 2) |  |  |  |
| NOTE 1: If the "trafficDesc" attribute is not present, the NEF may derive the traffic descriptor components from the AF Service Identifier. NOTE 2: If the "routeSelParamSets" attribute is not present, the NEF may derive S-NSSAI/DNN and/or other related parameters from the AF Service Identifer according to SLA. NOTE 3: Each element of the "visitedNetDescs" attribute may describe one PLMN (the "plmnId" attribute is included) or more than one PLMNs (the "mcc" attribute and, optionally, the "mncs" attribute are included). To indicate any VPLMN, the "visitedNetDescs" attribute shall contain only one entry with the "anyPlmnInd" attribute set to true. |  |  |  |  |  |  |  |  |  |  |  |

##### 5.11.2.3.5	Type: RouteSelectionParameterSet

Table 5.11.2.3.5-1: Definition of type RouteSelectionParameterSet

| Attribute name |  | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- | --- |
| dnn |  | Dnn | C | 0..1 | DNN to be matched with the DNN of the PDU Session. (NOTE) |  |
| snssai |  | Snssai | C | 0..1 | S-NSSAI to be matched with the S-NSSAI of the PDU Session. (NOTE) |  |
| precedence |  | Uinteger | O | 0..1 | Determines the order in which the Route Selection Descriptors are to be applied. |  |
| spatialValidityAreas |  | array(GeographicalArea) | O | 1..N | Indicates where the route selection parameters apply. It may correspond to a geographical area (e.g. a geographic shape that is known to the AF and is configured by the operator to correspond to a list of TAIs). |  |
| spatialValidityTais |  | array(Tai) | O | 1..N | Indicates the TAIs in which the route selection parameters apply. This attribute is applicable only within the 5GC and it shall not be included in the request messages of untrusted AFs for URSP guidance. |  |
| pduSessType |  | PduSessionType | O | 0..1 | Represents the requested PDU Session type. | PduSessTypeChange |
| NOTE: This attribute shall be present when the "PIN" feature is supported and the "trafficDesc" attribute included in UrspRuleRequest data structure includes the "pinId" attribute. |  |  |  |  |  |  |

##### 5.11.2.3.6	Type: AfNotification

Table 5.11.2.3.6-1: Definition of type AfNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subscription | Link | M | 1 | Link to the subscription resource to which this notification is related. |  |
| reportEvent | Event | C | 0..1 | Identifies the reported event notification. May be present if the "subNotifEvents" attribute is included in the AF subscription transaction. (NOTE) |  |
| authResult | AuthorizationResult | C | 0..1 | Indicates the service parameters authorization update result. (e.g. to revoke an authorization). (NOTE) |  |
| gpsis | array(Gpsi) | C | 1..N | Identifies the list of GPSI(s) of the reported UE(s). May be present if the "exterGroupId" attribute or "anyUeInd" attribute is included in the AF subscription transaction and only if the "authResult" attribute is provided. |  |
| dnn | Dnn | O | 0..1 | Identifies a DNN. |  |
| snssai | Snssai | O | 0..1 | Identifies an S-NSSAI. |  |
| eventInfo | EventInfo | O | 0..1 | Indicates the event information. |  |
| NOTE: At least one of "reportEvent" attribute and "authResult" attribute shall be included. |  |  |  |  |  |

##### 5.11.2.3.7	Type: EventInfo

Table 5.11.2.3.7-1: Definition of type EventInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| failureCause | Failure | O | 0..1 | Identify the failure reason for an unsuccessful result. May present if the "reportEvent" attribute value is "UNSUCCESS_UE_POL_DEL_SP". |  |
| plmnId | PlmnIdNid | O | 0..1 | Identify the PLMN ID of the inbound roamer when the AF subscribes with V-NEF about the outcome of the provisioning of VPLMN-specific URSP rules. It may be present if the "reportEvent" attribute value is "SUCCESS_UE_POL_DEL_SP" or "UNSUCCESS_UE_POL_DEL_SP" | VPLMNSpecificURSP |

##### 5.11.2.3.8	Type: TrafficDescriptorComponents

Table 5.11.2.3.8-1: Definition of type TrafficDescriptorComponents

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appDescs | map(AppDescriptor) | C | 1..N | Describes the operation systems and the corresponding applications for each operation systems. The key of map is osId. (NOTE 2) |  |
| flowDescs | array(string) | C | 1..N | Destination IP 3 tuple(s) (IP address or IPv6 network prefix, port number, protocol ID of the protocol above IP). The content of the string has the same encoding as the IPFilterRule AVP value as defined in IETF RFC 6733 [54], applicable only to the destination IP 3 tuple(s). (NOTE 3) |  |
| domainDescs | array(string) | C | 1..N | FQDN(s) or a regular expression which are used as a domain name matching criteria. (NOTE 4) |  |
| ethFlowDescs | array(EthFlowDescription) | C | 1..N | Descriptor(s) for destination information of non-IP traffic in which only ethernet flow description is defined. (NOTE 3) |  |
| dnns | array(Dnn) | C | 1..N | This is matched against the DNN information provided by the application. |  |
| connCaps | array(ConnectionCapabilities) | C | 1..N | This is matched against the information provided by a UE application when it requests a network connection with certain capabilities. |  |
| opSpecConnCaps | array(Bytes) | C | 1..N | Operator specific connection capabilities. Its encoding shall comply with the encoding defined in clause 5.2 of 3GPP TS 24.526 [48]. | ExtConnCapability |
| pinId | string | C | 0..1 | This is matched against a PIN ID for a specific PIN configured in the PEGC. Its encoding shall comply with the UE policy part type URSP as defined in clause 5.2 of 3GPP TS 24.526 [48]. (NOTE 5) | PIN |
| NOTE 1: At least one attribute of the above Traffic descriptor components shall be present. NOTE 2: The information is used to identify the Application(s) that is(are) running on the UE's OS. The included "osId" attribute does not include an OS version number. The included "appIds" attribute does not include a version number for the application. NOTE 3: "flowDescs" attribute and "ethFlowDescs" attribute are mutually exclusive. NOTE 4: The match of this traffic descriptor component does not require successful DNS resolution of the FQDN provided by the UE Application. NOTE 5: The "pinId" attribute and other attributes are mutually exclusive. |  |  |  |  |  |

##### 5.11.2.3.9	Type: NetworkDescription

Table 5.11.2.3.9-1: Definition of type NetworkDescription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| plmnId | PlmnId | C | 0..1 | PLMN Identifier. It represents a combination of MCC and MNC. (NOTE) |  |
| mcc | Mcc | C | 0..1 | Mobile Country Code (MCC). (NOTE) |  |
| mncs | array(Mnc) | O | 1..N | Represents the applicable MNC(s) for the indicated MCC. It may be present when the "mcc" attribute is present. |  |
| anyPlmnInd | boolean | C | 0..1 | Indicates any PLMN. When present, it shall be set to true. true: any PLMN (i.e., any MCC and any MNC). (NOTE) |  |
| NOTE: One of the "plmnId", the "mcc" or the "anyPlmnInd" attributes shall be included. |  |  |  |  |  |

##### 5.11.2.3.10	Void

##### 5.11.2.3.11	Void

#### 5.11.2.4	Simple data types and enumerations

##### 5.11.2.4.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.11.2.4.2	Simple data types

The simple data types defined in table 5.11.2.4.2-1 shall be supported.

Table 5.11.2.4.2-1: Simple data types

| Type Name |  | Type Definition |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A2xParamsPc5 |  | string |  | Configuration parameters for A2X communication over PC5. Its encoding shall comply with the UE policies for A2X communication over PC5 reference point as defined in 3GPP TS 24.578 [69]. |  | A2X |  |
| A2xParamsPc5Rm |  | string |  | This data type is defined in the same way as the "A2xParamsPc5" data type, but with the OpenAPI "nullable: true" property. |  | A2X |  |
| A2xParamsUu |  | string |  | Configuration parameters for A2X communication over Uu reference point. Its encoding shall comply with the UE policies for A2X communication over Uu reference point as defined in 3GPP TS 24.578 [69]. |  | A2X |  |
| A2xParamsUuRm |  | string |  | This data type is defined in the same way as the "A2xParamsUu" data type, but with the OpenAPI "nullable: true" property. |  | A2X |  |
| ParameterOverPc5 |  | string |  | Configuration parameters for V2X communication over PC5. Its encoding shall comply with the UE policies for V2X communication over PC5 as defined in clause 5.3 of 3GPP TS 24.588 [33]. |  |  |  |
| ParameterOverPc5Rm |  | string |  | This data type is defined in the same way as the "ParameterOverPc5" data type, but with the OpenAPI "nullable: true" property. |  |  |  |
| ParameterOverUu |  | string |  | Configuration parameters for V2X communication over Uu. Its encoding shall comply with the UE policies for V2X communication over Uu as defined in clause 5.4 of 3GPP TS 24.588 [33]. |  |  |  |
| ParameterOverUuRm |  | string |  | This data type is defined in the same way as the "ParameterOverUu" data type, but with the OpenAPI "nullable: true" property. |  |  |  |
| ParamForProSeDd |  | string |  | Configuration parameters for 5G ProSe direct discovery. Its encoding shall comply with the UE policies for 5G ProSe direct discovery defined in clause 5.3 of 3GPP TS 24.555 [49]. |  | ProSe |  |
| ParamForProSeDdRm |  | string |  | This data type is defined in the same way as the "ParamForProSeDd" data type, but with the OpenAPI "nullable: true" property. |  | ProSe |  |
| ParamForProSeDc |  | string |  | Configuration parameters for 5G ProSe direct communications. Its encoding shall comply with the UE policies for 5G ProSe direct communications defined in clause 5.4 of 3GPP TS 24.555 [49]. |  | ProSe |  |
| ParamForProSeDcRm |  | string |  | This data type is defined in the same way as the "ParamForProSeDc" data type, but with the OpenAPI "nullable: true" property. |  | ProSe |  |
| ParamForProSeU2NRelUe |  | string |  | Configuration parameters for 5G ProSe UE-to-network relay UE. Its encoding shall comply with the UE policies for 5G ProSe UE-to-network relay UE defined in clause 5.5 of 3GPP TS 24.555 [49]. |  | ProSe |  |
| ParamForProSeU2NRelUeRm |  | string |  | This data type is defined in the same way as the "ParamForProSeU2NRelUe" data type, but with the OpenAPI "nullable: true" property. |  | ProSe |  |
| ParamForProSeRemUe |  | string |  | Configuration parameters for 5G ProSe remote UE. Its encoding shall comply with the UE policies for 5G ProSe remote UE defined in clause 5.6 of 3GPP TS 24.555 [49]. |  | ProSe |  |
| ParamForProSeRemUeRm |  | string |  | This data type is defined in the same way as the "ParamForProSeRemUe" data type, but with the OpenAPI "nullable: true" property. |  | ProSe |  |
| ParamForProSeU2URelUe |  | string |  | Configuration parameters for 5G ProSe UE-to-UE relay UE. Its encoding shall comply with the UE policies for 5G ProSe UE-to-UE relay UE defined in clause 5.8 of 3GPP TS 24.555 [49]. |  | ProSe_Ph2 |  |
| ParamForProSeU2URelUeRm |  | string |  | This data type is defined in the same way as the "ParamForProSeU2URelUe" data type, but with the OpenAPI "nullable: true" property. |  | ProSe_Ph2 |  |
| ParamForProSeEndUe |  | string |  | Configuration parameters for 5G ProSe end UE. Its encoding shall comply with the UE policies for 5G ProSe end UE defined in clause 5.9 of 3GPP TS 24.555 [49]. |  | ProSe_Ph2 |  |
| ParamForProSeEndUeRm |  | string |  | This data type is defined in the same way as the "ParamForProSeEndUe" data type, but with the OpenAPI "nullable: true" property. |  | ProSe_Ph2 |  |
| ParamForRangingSlPos |  | string |  | Configuration parameters for ranging and sidelink positioning. Its encoding shall comply with the UE policies for ranging and sidelink positioning defined in clause 5.2.3 of 3GPP TS 24.514 [72] |  | Ranging_SL |  |
| ParamForRangingSlPosRm |  | string |  | This data type is defined in the same way as the "ParamForRangingslpos data type, but with the OpenAPI "nullable: true" property. |  | Ranging_SL |  |

##### 5.11.2.4.3	Enumeration: Event

Table 5.11.2.4.3-1: Enumeration Event

The enumeration Event represents the AF subscribe to event notification of the outcome related to the invocation of AF provisioned service parameters.

| Enumeration value | Description |
| --- | --- |
| SUCCESS_UE_POL_DEL_SP | Successful UE Policy Delivery related to the invocation of AF provisioned Service Parameters. |
| UNSUCCESS_UE_POL_DEL_SP | Unsuccessful UE Policy Delivery related to the invocation of AF provisioned Service Parameters. |

##### 5.11.2.4.4	Enumeration: AuthorizationResult

The enumeration AuthorizationResult represents the NEF notify the AF about the service parameters authorization updates result, e.g. to revoke an authorization.

Table 5.11.2.4.4-1: Enumeration AuthorizationResult

| Enumeration value | Description |
| --- | --- |
| AUTH_REVOKED | Indicated the service parameters authorization is revoked. |

##### 5.11.2.4.5	Enumeration: Failure

The enumeration Failure represents the failure reason for the unsuccessful result.

Table 5.11.2.4.5-1: Enumeration Failure

| Enumeration value | Description |
| --- | --- |
| UNSPECIFIED | Indicates the PCF received the UE sent UE policy delivery service cause #111 (Protocol error, unspecified). |
| UE_NOT_REACHABLE | Indicates the PCF received the notification from the AMF that the UE is not reachable. |
| UNKNOWN | Indicates unknown reasons upon no response from the UE, e.g. UPDS message type is not defined or not implemented by the UE, or not compatible with the UPDS state, in which the UE shall ignore the UPDS message. |
| UE_TEMP_UNREACHABLE | Indicates the PCF received the notification from the AMF that the UE is not reachable but the PCF will retry again. |

##### 5.11.2.4.6	Enumeration: ConnectionCapabilities

The enumeration ConnectionCapabilities represents the information provided by a UE application when it requests a network connection with certain capabilities.

Table 5.11.2.4.6-1: Enumeration ConnectionCapabilities

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| IMS | Indicates the connection capability to support IMS service. |  |
| MMS | Indicates the connection capability to support MMS service. |  |
| SUPL | Indicates the connection capability to support SUPL service. |  |
| INTERNET | Indicates the connection capability to support Internet service. |  |
| IOT_DELAY_TOLERANT | Indicates the connection capability to support IoT delay-tolerant services. (NOTE) | ExtConnCapability |
| IOT_NON_DELAY_TOLERANT | Indicates the connection capability to support IoT non-delay-tolerant services. (NOTE) | ExtConnCapability |
| DL_STREAMING | Indicates the connection capability to support downlink streaming services. (NOTE) | ExtConnCapability |
| UL_STREAMING | Indicates the connection capability to support uplink streaming services. (NOTE) | ExtConnCapability |
| VEHIC_COMM | Indicates the connection capability to support vehicular communication services. (NOTE) | ExtConnCapability |
| REAL_TIME_INTERACTIVE | Indicates the connection capability to support real time interactive services. (NOTE) | ExtConnCapability |
| UNIFIED_COMM | Indicates the connection capability to support unified communication services. (NOTE) | ExtConnCapability |
| BACKGROUND | Indicates the connection capability to support background services. (NOTE) | ExtConnCapability |
| MISS_CRITICAL | Indicates the connection capability to support mission critical services. (NOTE) | ExtConnCapability |
| TIME_CRITICAL | Indicates the connection capability to support time critical services. (NOTE) | ExtConnCapability |
| LOW_LAT_LOSS_TOL_UNACK | Indicates the connection capability to support low latency loss tolerant communications in un-acknowledged mode. (NOTE) | ExtConnCapability |
| NOTE: These connection capabilities are traffic categories specified in GSMA PRD NG.135 [75]. |  |  |

### 5.11.3	Used Features

The table below defines the features applicable to the ServiceParameter API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.11.3-1: Features used by ServiceParameter API

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | ProSe | This feature indicates the support of UE policy and N2 information provisioning for 5G ProSe. |
| 2 | enNB | Indicates the support of enhancements to the northbound interfaces. |
| 3 | AfNotifications | This feature indicates the support of AF subscribed event(s) notifications. |
| 4 | Notification_websocket | The delivery of notifications over Websocket is supported as described in 3GPP TS 29.122 [4]. This feature requires that the Notification_test_event feature is also supported. |
| 5 | Notification_test_event | The testing of notification connection is supported as described in 3GPP TS 29.122 [4]. |
| 6 | AfGuideURSP | This feature indicates the support of AF guidance for URSP determination. |
| 7 | A2X | This feature indicates the support of A2X communication. |
| 8 | ProSe_Ph2 | This feature indicates the support of UE policy and N2 information provisioning for 5G ProSe UE-to-UE Relay function. This feature requires that the ProSe feature is also supported. |
| 9 | PIN | This feature indicates the support of Personal IoT Network requirements. |
| 10 | VPLMNSpecificURSP | This feature indicates the support of AF guidance on VPLMN-specific URSP rules. This feature requires that "AfGuideURSP" and "AfNotifications" features are also supported. |
| 11 | AfGuideTNAPs | This feature indicates the support of AF providing guidance to the HPLMN of the UE of the list of TNAP(s) collocated with the 5G-RG(s) of a specific user. |
| 12 | Ranging_SL | This feature indicates the support of the ranging and sidelink positioning functionality. The following functionalities are supported: - Support the provisioning/update/deletion of ranging and sidelink positioning service parameters. |
| 13 | PduSessTypeChange | This feature indicates the support of the provisioning/update of the requested PDU Session type functionality as part of the Generic Group Management, Exposure and Communication Enhancements. The following functionalities are supported: - Support the provisioning/update of the requested PDU Session type as part of the information provided by the AF for guiding URSP determination. This feature requires the support of the "AfGuideURSP" feature. |
| 14 | ExtConnCapability | This feature indicates the support of: - The connection capabilities defined as traffic categories in GSMA PRD NG.135 [75]. - Operator Specific traffic categories. |

### 5.11.4	Error handling

#### 5.11.4.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.11.4.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the ServiceParameter API.

#### 5.11.4.3	Application Errors

The application errors defined for ServiceParameter API are listed in table 5.11.4.3-1.

Table 5.11.4.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

## 5.12	ACSParameterProvision API

### 5.12.0	Introduction

The Nnef_ParameterProvision service shall use the ACSParameterProvision API for ACS parameters provisioning.

The API URI of ACSParameterProvision API shall be:

- {apiRoot}/3gpp-acs-pp/v1

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-acs-pp".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.12.1	Resources

#### 5.12.1.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.12.1.1-1 and the resources and HTTP methods used for the ACSParameterProvision API.

Figure 5.12.1.1-1: Resource URI structure of the ACSParameterProvision API

Table 5.12.1.1-1 provides an overview of the resources and HTTP methods applicable for the ACSParameterProvision API.

Table 5.12.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| ACS Configuration Subscripions | /{afId}/subscriptions | GET | Read all subscriptions for a given AF. |
|  |  | POST | Create a new ACS configuration subscription. |
| Individual ACS Configuration Subscripion | /{afId}/subscriptions/{subscriptionId} | GET | Read an existing ACS configuration subscription identified by {subscriptionId}. |
|  |  | PUT | Update an existing ACS configuration subscription identified by {subscriptionId}. |
|  |  | PATCH | Modify an existing ACS configuration subscription identified by {subscriptionId}. |
|  |  | DELETE | Delete an existing ACS configuration subscription identified by {subscriptionId}. |

#### 5.12.1.2	Resource: ACS Configuration Subscriptions

##### 5.12.1.2.1	Introduction

This resource allows a AF to read all active ACS Configuration Subscriptions for the given AF, or create an new individual ACS Configuration subscription in the NEF.

##### 5.12.1.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-acs-pp/v1/{afId}/subscriptions

This resource shall support the resource URI variables defined in table 5.12.1.2.2-1.

Table 5.12.1.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | Identifier of the AF of type string. |

##### 5.12.1.2.3	Resource Methods

###### 5.12.1.2.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.12.1.2.3.

###### 5.12.1.2.3.2	GET

The GET method allows to read all active subscriptions for a given AF. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.12.1.2.3.2-1.

Table 5.12.1.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.12.1.2.3.2-2 and the response data structures and response codes specified in table 5.12.1.2.3.2-3.

Table 5.12.1.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.12.1.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(AcsConfigurationData) | M | 0..N | 200 OK | All the subscription information for the AF in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.12.1.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.12.1.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.12.1.2.3.3	POST

The POST method creates a new resource to individual ACS Configuration subscription for a given AF. The AF shall initiate the HTTP POST request message and the NEF shall respond to the message. The NEF shall construct the URI of the created resource.

This method shall support the request data structures specified in table 5.12.1.2.3.3-1 and the response data structures and response codes specified in table 5.12.1.2.3.3-2.

Table 5.12.1.2.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AcsConfigurationData | M | 1 | Parameters to create an individual ACS Configuration subscription resource. |

Table 5.12.1.2.3.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AcsConfigurationData | M | 1 | 201 Created | The subscription resource was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.12.1.2.3.3-3: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-acs-pp/v1/{afId}/subscriptions/{subscriptionId} |

#### 5.12.1.3	Resource: Individual ACS Configuration Subscription

##### 5.12.1.3.1	Introduction

This resource allows a AF to read, update or delete an existing ACS Configuration subscription.

##### 5.12.1.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-acs-pp/v1/{afId}/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 5.9.1.3.2-1.

Table 5.12.1.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | Identifier of the AF of type string. |
| subscriptionId | Identifier of the subscription resource of type string. |

##### 5.12.1.3.3	Resource Methods

###### 5.12.1.3.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.12.1.3.3.

###### 5.12.1.3.3.2	GET

The GET method allows to read the active subscription for a given AF and subscription Id. The AF shall initiate the HTTP GET request message and theNEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.12.1.3.3.2-1.

Table 5.12.1.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.12.1.3.3.2-2 and the response data structures and response codes specified in table 5.12.1.3.3.2-3.

Table 5.12.1.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.12.1.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AcsConfigurationData | M | 1 | 200 OK | The information for the subscription in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.12.1.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.12.1.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.12.1.3.3.3	PUT

The PUT method updates an existing resource to update a configuration. The AF shall initiate the HTTP PUT request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.9.1.3.3.3-1 and the response data structures and response codes specified in table 5.9.1.3.3.3-2.

Table 5.12.1.3.3.3-1: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AcsConfigurationData | M | 1 | Modify an existing subscription. |

Table 5.12.1.3.3.3-2: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AcsConfigurationData | M | 1 | 200 OK | The subscription resource was updated successfully and a representation of the updated resource is returned. |
| n/a |  |  | 204 No Content | The subscription resource was updated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.12.1.3.3.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.12.1.3.3.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.12.1.3.3.3A	PATCH

The PATCH method modifies an existing resource to update an existing ACS Configuration Subscripion. The AF shall initiate the HTTP PATCH request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.9.1.3.3.3A-1 and the response data structures and response codes specified in table 5.9.1.3.3.3A-2.

Table 5.12.1.3.3.3A-1: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AcsConfigurationDataPatch | M | 1 | Modify an existing subscription. |

Table 5.12.1.3.3.3A-2: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AcsConfigurationData | M | 1 | 200 OK | The subscription resource was modified successfully and a representation of the updated resource is returned. |
| n/a |  |  | 204 No Content | The subscription resource was modified successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.12.1.3.3.3A-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.12.1.3.3.3A-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.12.1.3.3.4	DELETE

The DELETE method deletes an existing individual subscription for a given AF. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.12.1.3.3.4-1.

Table 5.12.1.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.12.1.3.3.4-2 and the response data structures and response codes specified in table 5.12.1.3.3.4-3.

Table 5.12.1.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.12.1.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The subscription resource was terminated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.12.1.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.12.1.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

### 5.12.1A	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.12.1B	Notifications

There are no notifications defined for this API in this release of the specification.

### 5.12.2	Data Model

#### 5.12.2.1	General

This clause specifies the application data model supported by the ACSParameterProvision API.

Table 5.12.2.1-1 specifies the data types defined for the ACSParameterProvision API.

Table 5.12.2.1-1: ACSParameterProvision API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AcsConfigurationData | 5.12.2.3.2 | Represents an individual ACS Configuration subscription resource. |  |
| AcsConfigurationDataPatch | 5.12.2.3.3 | Represents the parameters to request to modify an existing ACS Configuration subscription. | PatchUpdate |

#### 5.12.2.2	Reused data types

The data types reused by the ACSParameterProvision API from other specifications are listed in table 5.12.2.2-1.

Table 5.12.2.2-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| AcsInfo | 3GPP TS 29.571 [8] | Contains the information of ACS |
| ExternalGroupId | 3GPP TS 29.122 [4] | External Group Identifier for a user group. |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |
| Link | 3GPP TS 29.122 [4] | Represents a referenced resource. |
| MtcProviderInformation | 3GPP TS 29.571 [8] | Indicates MTC provider information. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.9.4-1. |

#### 5.12.2.3	Structured data types

##### 5.12.2.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.12.2.3.2	Type: AcsConfigurationData

Table 5.12.2.3.2-1: Definition of type AcsConfigurationData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| self | Link | C | 0..1 | Identifies the individual service parameter subscription resource URI. Shall be present by the NEF in HTTP responses that include an object of AcsConfigurationData Data type. |  |
| gpsi | Gpsi | O | 0..1 | Identifies GPSI. (NOTE) |  |
| exterGroupId | ExternalGroupId | O | 0..1 | Represents a group of users. (NOTE) |  |
| acsInfo | AcsInfo | M | 1 | Contains the information of ACS. |  |
| mtcProviderId | MtcProviderInformation | O | 0..1 | Indicates MTC provider information. |  |
| suppFeat | SupportedFeatures | M | 1 | Indicates the list of Supported features used as described in clause 5.12.3. This parameter shall be supplied by the NF service consumer in the POST request that requested the creation of an individual ACS configuration Subscription resource. |  |
| NOTE: Only one of the "gpsi" or "exterGroupId" attribute shall be provided. |  |  |  |  |  |

##### 5.12.2.3.3	Type: AcsConfigurationDataPatch

Table 5.12.2.3.3-1: Definition of type AcsConfigurationDataPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| acsInfo | AcsInfo | O | 0..1 | Contains the information of ACS. |  |
| mtcProviderId | MtcProviderInformation | O | 0..1 | Indicates MTC provider information. |  |

#### 5.12.2.4	Simple data types and enumerations

##### 5.12.2.4.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.12.2.4.2	Simple data types

The simple data types defined in table 5.12.2.4.2-1 shall be supported.

Table 5.12.2.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

### 5.12.3	Used Features

The table below defines the features applicable to the ACSParameterProvision API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.12.3-1: Features used by ACSParameterProvision API

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | PatchUpdate | Indicates the support of enhancements to the northbound interfaces (e.g. support the partial modification of an existing subscription resource). |

### 5.12.4	Error handling

#### 5.12.4.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.12.4.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the ACSParameterProvision API.

#### 5.12.4.3	Application Errors

The application errors defined for ACSParameterProvision API are listed in table 5.12.4.3-1.

Table 5.12.4.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

## 5.13	MoLcsNotify API

### 5.13.0	Introduction

The Nnef_MoLcsNotify service shall use the MoLcsNotify API.

The API URI of MoLcsNotify API shall be:

- {apiRoot}/3gpp-mo-lcs-notify/v1

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-mo-lcs-notify".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.13.1	Resources

There are no resources defined for this API in this release of the specification.

### 5.13.1A	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.13.2	Notifications

#### 5.13.2.1	Introduction

Upon receipt of a UE location information update notification from the GMLC, the NEF shall send an HTTP POST message in order to notify the AF of the updated UE location information.

#### 5.13.2.2	Event Notification

Callback URI: {notificationDestination} shall be used with the callback URI variables defined in table 5.13.2.2-1.

Table 5.13.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notificationDestination | A URI indicating the notification destination where N33 notification requests shall be delivered to. This URI shall be preconfigured in the NEF. |

#### 5.13.2.3	Operation Definition

##### 5.13.2.3.1	Notification via HTTP POST

This method shall support the request data structures specified in table 5.13.2.3.1-1 and the response data structures and response codes specified in table 5.13.2.3.1-2.

Table 5.13.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocUpdateData | M | 1 | Delivers UE location to AF during MO-LR procedure |

Table 5.13.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocUpdateDataReply | M | 1 | 200 OK | The notification is received successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.13.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

Table 5.13.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

### 5.13.3	Data Model

#### 5.13.3.1	General

This clause specifies the application data model supported by the MoLcsNotify API.

#### 5.13.3.2	Reused data types

The data types reused by the MoLcsNotify API from other specifications are listed in table 5.13.3.2-1.

Table 5.13.3.2-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.13.4-1. |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |
| LocationInfo | 3GPP TS 29.122 [4] | Represent user location information for exposure. |
| LcsQosClass | 3GPP TS 29.572 [34] | LCS QoS Class. |
| ServiceIdentity | 3GPP TS 29.515 [35] | Service identity |

#### 5.13.3.3	Structured data types

##### 5.13.3.3.1	Introduction

This clause defines the structured data types to be used by the MoLcsNotify API.

##### 5.13.3.3.2	Type: LocUpdateData

This type represents a UE updated location information from the NEF to the AF.

Table 5.13.3.3.2-1: Definition of type LocUpdateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| gpsi | Gpsi | M | 1 | Generic Public Subscription identifier |  |
| locInfo | LocationInfo | M | 1 | Represent user location information for exposure. |  |
| lcsQosClass | LcsQosClass | M | 1 | LCS QoS Class. |  |
| svcId | ServiceIdentity | O | 0..1 | Service Identity may be specified by the UE for LCS request. |  |
| suppFeat | SupportedFeatures | M | 1 | Indicates the list of Supported features used as described in clause 5.13.4. |  |
| additionalLocInfo | array(LocationInfo) | O | 1..N | Represent additionaly user location information for exposure when location information for multiple UEs is exposed. |  |

##### 5.13.3.3.3	Type: LocUpdateDataReply

This data type represents a reply to a MO LCS notification and is sent from the AF to the NEF.

Table 5.13.3.3.3-1: Definition of type LocUpdateDataReply

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| suppFeat | SupportedFeatures | M | 1 | Indicates the list of Supported features used as described in clause 5.13.4. |  |

#### 5.13.3.4	Simple data types and enumerations

##### 5.13.3.4.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.13.3.4.2	Simple data types

The simple data types defined in table 5.13.3.4.2-1 shall be supported.

Table 5.13.3.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

### 5.13.4	Used Features

The table below defines the features applicable to the MoLcsNotify API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.13.4-1: Features used by MoLcsNotify API

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 5.13.5	Error handling

#### 5.13.5.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.13.5.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the MoLcsNotify API.

#### 5.13.5.3	Application Errors

The application errors defined for MoLcsNotify API are listed in table 5.13.5.3-1.

Table 5.13.5.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

## 5.14	AKMA API

### 5.14.1	Introduction

The Nnef_AKMA service shall use the AKMA API.

The API URI of AKMA API shall be:

- {apiRoot}/3gpp-akma/v1

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-akma".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.14.2	Resources

There are no resources defined for this API in this release of the specification.

### 5.14.3	Custom Operations without associated resources

#### 5.14.3.1	Overview

The structure of the custom operation URIs of the AKMA API is shown in Figure 5.14.3.1-1.

Figure 5.14.3.1-1: Custom operation URI structure of the AKMA API

Table 5.14.3.1-1 provides an overview of the custom operations and applicable HTTP methods.

Table 5.14.3.1-1: Custom operations without associated resources

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Retrieve | /retrieve | POST | Request to retrieve AKMA Application Key information |

#### 5.14.3.2	Operation: Retrieve

##### 5.14.3.2.1	Description

The custom operation allows a service consumer to retrieve AKMA application key information via the NEF.

##### 5.14.3.2.2	Operation Definition

This operation shall support the request and response data structures and response codes specified in tables 5.14.3.2.2-1 and 5.14.3.2.2-2.

Table 5.14.3.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AkmaAfKeyRequest | M | 1 | Parameters to request to retrieve AKMA Application Key information. |

Table 5.14.3.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AkmaAfKeyData | M | 1 | 200 OK | The requested AKMA Application Key information was returned successfully. |
| n/a |  |  | 204 No Content | If the requested data does not exist, the NEF shall respond with "204 No Content". |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4] |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure cases are described in clause 5.14.7. |  |  |  |  |

Table 5.14.3.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.14.3.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

### 5.14.4	Notifications

#### 5.14.4.1	General

Notifications shall comply to clause 5.2.5 of 3GPP TS 29.122 [4].

Table 5.14.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| AKMA Service Disablement Notification | {notifUri} | POST | Enable the NEF to notify a previously subscribed AF of the disablement of the AKMA service for the UE. |

#### 5.14.4.2	AKMA Service Disablement Notification

##### 5.14.4.2.1	Description

The AKMA Service Disablement Notification is used by the NEF to notify the AF of AKMA service disablement for the UE.

##### 5.14.4.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 5.14.4.2.2-1.

Table 5.14.4.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | Callback URI provided by the AF during retrievel of AKMA application key as defined in table 5.14.3.2.2-1. |

##### 5.14.4.2.3	Operation Definition

This method shall support the request data structures specified in table 5.14.4.2.3.1-1 and the response data structures and response codes specified in table 5.14.4.2.3-2.

Table 5.14.4.2.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ServiceDiableNotif | M | 1 | Represents the AKMA Service Disablement Notification. |

Table 5.14.4.2.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The notification is received successfully and processed. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.14.4.2.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

Table 5.19.4.2.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

### 5.14.5	Data Model

#### 5.14.5.1	General

This clause specifies the application data model supported by the AKMA API. Table 5.14.5.1-1 specifies the data types defined for the AKMA API.

Table 5.14.5.1-1: AKMA API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AkmaAfKeyRequest | 5.14.5.3.2 | Represents AKMA application key retrieval request information. |  |
| AkmaAfKeyData | 5.14.5.3.3 | Represents AKMA application key information. |  |
| ServiceDisableNotif | 5.15.5.3.4 | Represents a AKMA service disable information. |  |

#### 5.14.5.2	Reused data types

The data types reused by the AKMA API from other specifications are listed in table 514.5.2-1.

Table 5.14.5.2-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| DateTime | 3GPP TS 29.122 [4] | Represents a data and a time. |
| Gpsi | 3GPP TS 29.571 [8] | Represents a GPSI. |
| ProblemDetails | 3GPP TS 29.122 [4] | Represents error related information. |
| Supi | 3GPP TS 29.571 [8] | Represents a SUPI. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features. |
| Uri | 3GPP TS 29.122 [4] | Represents a URI. |

#### 5.14.5.3	Structured data types

##### 5.14.5.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.14.5.3.2	Type: AkmaAfKeyRequest

Table 5.14.5.3.2-1: Definition of type AkmaAfKeyRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability (NOTE) |
| --- | --- | --- | --- | --- | --- |
| afId | AfId | M | 1 | Identification of AF |  |
| aKId | AKId | M | 1 | A-KID |  |
| anonInd | boolean | O | 0..1 | Indicates whether an anonymous user access. Set to "true" if an anonymous user access is requested; otherwise set to "false". Default value is "false" if omitted. |  |
| notifiUri | Uri | O | 0..1 | Contains the notification URI via which the AF desires to receive notifications on AKMA service disablement. | RoamingRestriction |
| suppFeat | SupportedFeatures | O | 0..1 | Indicates the list of Supported features used as described in clause 5.14.6. |  |
| NOTE: Properties marked with a feature as defined in clause 5.14.6 are applicable as described in clause 5.2.7 of 3GPP TS 29.122 [4]. If no feature is indicated, the related property applies for all the features. |  |  |  |  |  |

##### 5.14.5.3.3	Type: AkmaAfKeyData

Table 5.14.5.3.3-1: Definition of type AkmaAfKeyData

| Attribute name | Data type | P | Cardinality | Description | Applicability (NOTE) |
| --- | --- | --- | --- | --- | --- |
| kaf | string | M | 1 | KAF |  |
| expiry | DateTime | M | 1 | Expiration time of KAF. |  |
| gpsi | Gpsi | O | 0..1 | Indicates an external ID of the UE. (NOTE 2, NOTE 3) |  |
| supi | Supi | C | 0..1 | Indicates the SUPI of the UE. (NOTE 2) |  |
| suppFeat | SupportedFeatures | O | 0..1 | Indicates the features supported by both the AF and the NEF. |  |
| NOTE 1: Properties marked with a feature as defined in clause 5.14.6 are applicable as described in clause 5.2.7 of 3GPP TS 29.122 [4]. If no feature is indicated, the related property applies for all the features. NOTE 2: When the "AkmaAfKeyData" data structure is used in the current release of this specification, the "gpsi" attribute may be included and the "supi" attribute is not applicable. NOTE 3 When the "anonInd" attribute contained in AkmaAfKeyRequest data type is set to "true" in the incoming request, the "gpsi" attribute shall not be included. |  |  |  |  |  |

##### 5.14.5.3.4	Type: ServiceDisableNotif

Table 5.14.5.3.4-1: Definition of type ServiceDisableNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aKId | AKId | M | 1 | Contains the A-KID. | RoamingRestriction |

#### 5.14.5.4	Simple data types and enumerations

##### 5.14.5.4.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.14.5.4.2	Simple data types

The simple data types defined in table 5.14.5.4.2-1 shall be supported.

Table 5.14.5.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| AfId | string | Identification of AF which is formatted as the following string: "<FQDN>.<Ua* security protocol id>", wherein, <FQDN> is the FQDN of the AF and <Ua* security protocol id> is a string of 5 octet and the identification of the Ua* security protocol is specified as Ua security protocol identifier in Annex H of 3GPP TS 33.220 [39] that the AF will use with the UE. Example: 1. FQDN: www.app1.com, Ua* security protocol id: 0100BC0001, then AfId: www.app1.com.0100BC0001 |  |
| AKId | string | AKMA Key Identifier shall be in NAI format as specified in clause 2.2 of IETF RFC 7542 [40], which is formatted as the following string: "<username>@<realm>", wherein, <username> shall include Routing Indicatorand the A-TID in the format "rid<value>.atid<value>", where "rid" and "atid" are labels indicating Routing Indicator and A-TID and <realm> shall include Home Network Id. Example: 1. If Routing Indicator: 012, A-TID: 019345346 and Home Network Id: 5gc.mnc012.mcc345.3gppnetwork.org, then AKId: rid012.akid019345346@5gc.mnc012.mcc345.3gppnetwork.org Routing Indicator, Home Network Id are specified in 3GPP TS 23.003 [55]. A-TID is specified in 3GPP TS 33.535 [38]. |  |

### 5.14.6	Used Features

The table below defines the features applicable to the AKMA API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.14.6-1: Features used by AKMA API

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | RoamingRestriction | This feature indicates the support of roaming UE detection by the network and the denial of the AKMA services to roaming UEs. |

### 5.14.7	Error handling

#### 5.14.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.14.7.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the AKMA API.

#### 5.14.7.3	Application Errors

The application errors defined for the AKMA API are listed in table 5.14.7.3-1.

Table 5.14.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |
| K_AKMA_NOT_PRESENT | 403 Forbidden | Indicates that the KAKMA identified by the A-KID provided in the AKMA Application Key retrieval request body is not present at the AAnF. |  |
| ROAMING_AKMA_SERVICE_DENIED | 403 Forbidden | Indicates that the Network identifies the request is for a Roaming UE and denies the AKMA service. | RoamingRestriction |

## 5.15	TimeSyncExposure API

### 5.15.0	Introduction

The Nnef_TimeSyncExposure service shall use the TimeSyncExposure API.

The API URI of TimeSyncExposure API shall be:

- {apiRoot}/3gpp-time-sync/v1

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-time-sync".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.15.1	Resources

#### 5.15.1.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.15.1.1-1 and the resources and HTTP methods used for the TimeSyncExposure API.

Figure 5.15.1.1-1: Resource URI structure of the TimeSyncExposure API

Table 5.15.1.1-1 provides an overview of the resources and HTTP methods applicable for the TimeSyncExposure API.

Table 5.15.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Time Synchronization Exposure Subscriptions | /{afId}/subscriptions | GET | Read all subscriptions for a given AF. |
|  |  | POST | Create a new subscription to time synchronization exposure. |
| Individual Time Synchronization Exposure Subscription | /{afId}/subscriptions/{subscriptionId} | GET | Read a subscription to time synchronization exposure. |
|  |  | PUT | Modify all of the properties of an existing subscription to time synchronization exposure. |
|  |  | DELETE | Delete a subscription to time synchronization exposure. |
| Time Synchronization Exposure Configurations | /{afId}/subscriptions/{subscriptionId}/configurations | GET | Read all configurations for a given AF and subscription. |
|  |  | POST | Create a new configuration to time synchronization exposure |
| Individual Time Synchronization Exposure Configuration | /{afId}/subscriptions/{subscriptionId}/configurations/{instanceReference} | GET | Read a configuration to time synchronization exposure. |
|  |  | PUT | Modify all of the properties of an existing configuration to time synchronization exposure. |
|  |  | DELETE | Delete a configuration to time synchronization exposure. |
|  |  | DELETE | DELETE a new configuration of 5G access stratum time distribution. |

#### 5.15.1.2	Resource: Time Synchronization Exposure Subscriptions

##### 5.15.1.2.1	Introduction

This resource allows an AF to read all active time synchronization exposure subscribtions for the given AF, or allows an AF to create a new Individual Time Synchronization Exposure Subscription in the NEF.

##### 5.15.1.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-time-sync/v1/{afId}/subscriptions

This resource shall support the resource URI variables defined in table 5.15.1.2.2-1.

Table 5.15.1.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |

##### 5.15.1.2.3	Resource Methods

###### 5.15.1.2.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.15.1.2.2.

###### 5.15.1.2.3.2	GET

The GET method allows to read all active subscriptions for a given AF. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.15.1.2.3.2-1.

Table 5.15.1.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supp-feat | SupportedFeatures | O | 0..1 | The features supported by the NF service consumer. |

This method shall support the request data structures specified in table 5.15.1.2.3.2-2 and the response data structures and response codes specified in table 5.15.1.2.3.2-3.

Table 5.15.1.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.15.1.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(TimeSyncExposureSubsc) | M | 0..N | 200 OK | The subscription information for the AF in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.15.1.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

Table 5.15.1.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

###### 5.15.1.2.3.3	POST

The POST method creates a new subscription resource to time synchronization exposure subscription for a given AF. The AF shall initiate the HTTP POST request message and the NEF shall respond to the message. The NEF shall construct the URI of the created resource.

This method shall support the request data structures specified in table 5.15.1.2.3.3-1 and the response data structures and response codes specified in table 5.15.1.2.3.3-2.

Table 5.15.1.2.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TimeSyncExposureSubsc | M | 1 | Contains the information for the creation of a new Individual Time Synchronization Exposure Subscription resource. |

Table 5.15.1.2.3.3-2: Data structures supported by the POST Response Body on this resource

| Data type |  | P |  | Cardinality |  | Response codes |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TimeSyncExposureSubsc |  | M |  | 1 |  | 201 Created |  | The subscription was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |  |
| ProblemDetails |  | O |  | 0..1 |  | 403 Forbidden |  | (NOTE 2) |  |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure causes are described in clause 5.15.6. |  |  |  |  |  |  |  |  |  |

Table 5.15.1.2.3.3-3: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-time-sync/v1/{afId}/subscriptions/{subscriptionId} |

#### 5.15.1.3	Resource: Individual Time Synchronization Exposure Subscription

##### 5.15.1.3.1	Introduction

This resource allows an AF to read, update or delete an existing Individual Time Synchronization Exposure Subscription.

##### 5.15.1.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-time-sync/v1/{afId}/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 5.15.1.3.2-1.

Table 5.15.1.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |
| subscriptionId | string | Identifier of the subscription resource. |

##### 5.15.1.3.3	Resource Methods

###### 5.15.1.3.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.15.1.3.2.

###### 5.15.1.3.3.2	GET

The GET method allows to read the active subscription for a given AF and subscription Id. The AF shall initiate the HTTP GET request message and theNEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.15.1.3.3.2-1.

Table 5.15.1.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supp-feat | SupportedFeatures | O | 0..1 | The features supported by the NF service consumer. |

This method shall support the request data structures specified in table 5.15.1.3.3.2-2 and the response data structures and response codes specified in table 5.15.1.3.3.2-3.

Table 5.15.1.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.15.1.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TimeSyncExposureSubsc | M | 1 | 200 OK | The subscription information for the AF in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.15.1.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

Table 5.15.1.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

###### 5.15.1.3.3.3	PUT

The PUT method modifies an existing subscription resource to update a subscription. The AF shall initiate the HTTP PUT request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.15.1.3.3.3-1 and the response data structures and response codes specified in table 5.15.1.3.3.3-2.

Table 5.15.1.3.3.3-1: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TimeSyncExposureSubsc | M | 1 | Modify an existing Time Synchronization Exposure Subscription. |

Table 5.15.1.3.3.3-2: Data structures supported by the PUT Response Body on this resource

| Data type |  | P |  | Cardinality |  | Response codes |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TimeSyncExposureSubsc |  | M |  | 1 |  | 200 OK |  | The subscription was updated successfully. |  |
| N/A |  |  |  |  |  | 204 No Content |  | The subscription was updated successfully. |  |
| N/A |  |  |  |  |  | 307 Temporary Redirect |  | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| N/A |  |  |  |  |  | 308 Permanent Redirect |  | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| ProblemDetails |  | O |  | 0..1 |  | 403 Forbidden |  | (NOTE 2) |  |
| NOTE 1: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure causes are described in clause 5.15.6. |  |  |  |  |  |  |  |  |  |

Table 5.15.1.3.3.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

Table 5.15.1.3.3.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

###### 5.15.1.3.3.4	DELETE

The DELETE method deletes the time synchronization exposure subscription for a given AF. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.15.1.3.3.4-1.

Table 5.15.1.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.15.1.3.3.4-2 and the response data structures and response codes specified in table 5.15.1.3.3.4-3.

Table 5.15.1.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.15.1.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The subscription was terminated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.15.1.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

Table 5.15.1.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

#### 5.15.1.4	Resource: Time Synchronization Exposure Configurations

##### 5.15.1.4.1	Introduction

This resource allows an AF to read all active time synchronization exposure configuration for the given AF and subscription, or allows an AF to create a new time synchronization configuration and activate the time synchronization service with the configuration.

##### 5.15.1.4.2	Resource Definition

Resource URI: {apiRoot}/3gpp-time-sync/v1/{afId}/subscriptions/{subscriptionId}/configurations

This resource shall support the resource URI variables defined in table 5.15.1.4.2-1.

Table 5.15.1.4.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |
| subscriptionId | string | Identifier of the subscription resource. |

##### 5.15.1.4.3	Resource Methods

###### 5.15.1.4.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.15.1.4.3.

###### 5.15.1.4.3.2	GET

The GET method allows to read all active configurations for a given AF and subscription. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.15.1.4.3.2-1.

Table 5.15.1.4.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.15.1.4.3.2-2 and the response data structures and response codes specified in table 5.15.1.4.3.2-3.

Table 5.15.1.4.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.15.1.4.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(TimeSyncExposureConfig) | M | 0..N | 200 OK | The configuration information for the AF in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.15.1.4.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

Table 5.15.1.4.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

###### 5.15.1.4.3.3	POST

The POST method creates a new configuration resource to activate time synchronization service for a given AF. The AF shall initiate the HTTP POST request message and the NEF shall respond to the message. The NEF shall construct the URI of the created resource.

This method shall support the request data structures specified in table 5.15.1.4.3.3-1 and the response data structures and response codes specified in table 5.15.1.4.3.3-2.

Table 5.15.1.4.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TimeSyncExposureConfig | M | 1 | Parameters to create a configuration and to activate time synchronization service. |

Table 5.15.1.4.3.3-2: Data structures supported by the POST Response Body on this resource

| Data type |  | P |  | Cardinality |  | Response codes |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TimeSyncExposureConfig |  | M |  | 1 |  | 201 Created |  | The subscription was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |  |
| n/a |  |  |  |  |  | 307 Temporary Redirect |  | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| n/a |  |  |  |  |  | 308 Permanent Redirect |  | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| ProblemDetails |  | O |  | 0..1 |  | 403 Forbidden |  | (NOTE 2) |  |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure causes are described in clause 5.15.6. |  |  |  |  |  |  |  |  |  |

Table 5.15.1.4.3.3-3: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-time-sync/v1/{afId}/subscriptions/{subscriptionId}/configurations/{instanceReference} |

Table 5.15.1.4.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.15.1.4.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

#### 5.15.1.5	Resource: Individual Time Synchronization Exposure Configuration

##### 5.15.1.5.1	Introduction

This resource allows an AF to read/modify/cancel a configuration to active/modify/deactivate Time Synchronization service with the NEF.

##### 5.15.1.5.2	Resource Definition

Resource URI: {apiRoot}/3gpp-time-sync/v1/{afId}/subscriptions/{subscriptionId}/configuration/{instanceReference}

This resource shall support the resource URI variables defined in table 5.15.1.5.2-1.

Table 5.15.1.5.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |
| subscriptionId | string | Identifier of the subscription resource. |
| instanceReference | string | Identifier of the PTP instance resource |

##### 5.15.1.5.3	Resource Methods

###### 5.15.1.5.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.15.1.5.2.

###### 5.15.1.5.3.2	GET

The GET method allows to read the active configuration for a given AF, subscription Id and configuration Id. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.15.1.5.3.2-1.

Table 5.15.1.5.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.15.1.5.3.2-2 and the response data structures and response codes specified in table 5.15.1.5.3.2-3.

Table 5.15.1.5.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.15.1.5.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TimeSyncExposureConfig | M | 1 | 200 OK | The configuration information for the AF in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.15.1.5.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

Table 5.15.1.5.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

###### 5.15.1.5.3.3	PUT

The PUT method modifies an existing configuration resource to update a configuration. The AF shall initiate the HTTP PUT request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.15.1.5.3.3-1 and the response data structures and response codes specified in table 5.15.1.5.3.3-2.

Table 5.15.1.5.3.3-1: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TimeSyncExposureConfig | M | 1 | Modify an existing Time Synchronization Exposure Configuration. |

Table 5.15.1.5.3.3-2: Data structures supported by the PUT Response Body on this resource

| Data type |  | P |  | Cardinality |  | Response codes |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TimeSyncExposureConfig |  | M |  | 1 |  | 200 OK |  | The time synchronization configuration was updated successfully. |  |
| N/A |  |  |  |  |  | 204 No Content |  | The time synchronization configuration was updated successfully. |  |
| N/A |  |  |  |  |  | 307 Temporary Redirect |  | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| N/A |  |  |  |  |  | 308 Permanent Redirect |  | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| ProblemDetails |  | O |  | 0..1 |  | 403 Forbidden |  | (NOTE 2) |  |
| NOTE 1: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure causes are described in clause 5.15.6. |  |  |  |  |  |  |  |  |  |

Table 5.15.1.5.3.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

Table 5.15.1.5.3.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

###### 5.15.1.5.3.4	DELETE

The DELETE method deletes the time synchronization exposure subscription for a given AF. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.15.1.5.3.4-1.

Table 5.15.1.5.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.15.1.5.3.4-2 and the response data structures and response codes specified in table 5.15.1.5.3.4-3.

Table 5.15.1.5.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.15.1.5.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The configuration was terminated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.15.1.5.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

Table 5.15.1.5.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

### 5.15.2	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.15.3	Notifications

#### 5.15.3.1	Introduction

Table 5.15.3.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Time Synchronization Capability Notification | {subsNotifUri} | POST | Time Synchronization Capability Notification for a list of UEs. |
| Time Synchronization Configuration Notification | {configNotifUri} | POST | Current State of Time Synchronization configuration Notification. |

#### 5.15.3.2	Time Synchronization Capability Notification

##### 5.15.3.2.1	Description

The Notification is used by the NEF to report the Time Synchronization Capability to the AF.

##### 5.15.3.2.2	Callback URI

The Callback URI "{subsNotifUri}" shall be used with the callback URI variables defined in table 5.15.3.2.2-1.

Table 5.15.3.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| subsNotifUri | Callback reference provided by the AF during creation/modification of the subscription as defined in Table 5.15.4.3.2-1. |

##### 5.15.3.2.3	Operation Definition

###### 5.15.3.2.3.1	Notification via HTTP POST

This method shall support the request data structures specified in table 5.15.3.2.3.1-1 and the response data structures and response codes specified in table 5.15.3.2.3.1-2 and the Location Headers specified in table 5.15.3.2.3.1-3 and table 5.15.3.2.3.1-4.

Table 5.15.3.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TimeSyncExposureSubsNotif | M | 1 | Provides the time synchroniziation capabilities of a list of UEs by the NEF to the AF. |

Table 5.15.3.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The event notification is received successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.15.3.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

Table 5.15.3.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

###### 5.15.3.2.3.2	Notification via Websocket

If supported by both AF and NEF and successfully negotiated, the Time Synchroniaition Capability Notification may alternatively be delivered through the Websocket mechanism as defined in clause 5.2.5.4 of 3GPP TS 29.122 [4].

#### 5.15.3.3	Time Synchronization Configuration Notification

##### 5.15.3.3.1	Description

The Notification is used by the NEF to report the state of Time Synchronization service configuration to the AF.

##### 5.15.3.3.2	Callback URI

The Callback URI "{configNotifUri}" shall be used with the callback URI variables defined in table 5.15.3.3.2-1.

Table 5.15.3.3.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| configNotifUri | Callback reference provided by the AF during creation/modification of the configuration as defined in Table 5.15.4.3.6-1. |

##### 5.15.3.3.3	Operation Definition

###### 5.15.3.3.3.1	Notification via HTTP POST

This method shall support the request data structures specified in table 5.15.3.3.3.1-1 and the response data structures and response codes specified in table 5.15.3.3.3.1-2 and the Location Headers specified in table 5.15.3.3.3.1-3 and table 5.15.3.3.3.1-4.

Table 5.15.3.3.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TimeSyncExposureConfigNotif | M | 1 | Provides the current state of time synchroniziation service configuration by the NEF to the AF. |

Table 5.15.5.3.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The notification is received successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during event notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during event notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.15.3.3.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

Table 5.15.2.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

###### 5.15.3.3.3.2	Void

### 5.15.4	Data Model

#### 5.15.4.1	General

This clause specifies the application data model supported by the TimeSyncExposure API.

Table 5.15.4.1-1 specifies the data types defined for the TimeSyncExposure API.

Table 5.15.4.1-1: TimeSyncExposure API specific Data Types

| Data type |  | Clause defined |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AsTimeResource |  | 5.15.4.4.8 |  | Identifies the supported 5G clock quality. |  |  |  |
| AcceptanceCriteriaResultIndication |  | 5.15.4.4.9 |  | Contains the acceptable/not acceptable indication of the clock quality acceptance criteria result information. |  | NetTimeSyncStatus |  |
| AvailReport |  | 5.15.4.3.22 |  | Contains the availability status for a UE/DS-TT. |  | SupportReport |  |
| AvailStatus |  | 5.15.4.4.10 |  | Defines the availability status. |  | SupportReport |  |
| ConfigForPort |  | 5.15.4.3.18 |  | Contains configuration information for each port. |  |  |  |
| EventFilter |  | 5.15.4.3.10 |  | Contains the filter conditions to match for notifying the event(s) of time synchronization capabilities. |  |  |  |
| GmCapable |  | 5.15.4.4.5 |  | Indicates separately whether 5GS supports acting as a gPTP or PTP grandmaster. |  |  |  |
| InstanceType |  | 5.15.4.4.7 |  | Identifies supported PTP instance type. |  |  |  |
| Protocol |  | 5.15.4.4.4 |  | Identifies the supported protocol. |  |  |  |
| PtpCapabilitiesPerUe |  | 5.15.4.3.11 |  | Contains the supported PTP capabilities per UE. |  |  |  |
| PtpInstance |  | 5.15.4.3.12 |  | Contains PTP instance configuration and activation information requested by the AF. |  |  |  |
| ReportedCapability |  | 5.15.4.5.1 |  | Extends the EventFilter data type with the AvailReport data type to allows to indicate the availability status of the UE/DS-TT. |  | SupportReport |  |
| StateOfConfiguration |  | 5.15.4.3.17 |  | Contains the PTP port state of the time synchronization configuration. |  |  |  |
| StateOfDstt |  | 5.15.4.3.19 |  | Contains the PTP port state of a DS-TT |  |  |  |
| SubscribedEvent |  | 5.15.4.4.6 |  | Identifies the subscribed event. |  |  |  |
| SubsEventNotification |  | 5.15.4.3.8 |  | Represents the notification about a subscribed Individual Event. |  |  |  |
| TimeSyncCapability |  | 5.15.4.3.3 |  | Contains the time synchronization capability. |  |  |  |
| TimeSyncExposureConfig |  | 5.15.4.3.6 |  | Contains the Time Synchronization Configuration parameters. |  |  |  |
| TimeSyncExposureConfigNotif |  | 5.15.4.3.9 |  | Contains the notification of time synchronization service state. |  |  |  |
| TimeSyncExposureSubsc |  | 5.15.4.3.2 |  | Contains the requested parameters for the subscription to time synchronization capability notifications. |  |  |  |
| TimeSyncExposureSubsNotif |  | 5.15.4.3.7 |  | Contains the notification of time synchronization capability. |  |  |  |

#### 5.15.4.2	Reused data types

The data types reused by the TimeSyncExposure API from other specifications are listed in table 5.15.4.2-1.

Table 5.15.4.2-1: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ClockQualityAcceptanceCriterion | 3GPP TS 29.571 [8] | Identifies clock quality acceptance criteria information. | NetTimeSyncStatus |
| ClockQualityDetailLevel | 3GPP TS 29.571 [8] | Identifies clock quality detail level information. | NetTimeSyncStatus |
| DateTime | 3GPP TS 29.122 [4] | Represents a data and a time. |  |
| Dnn | 3GPP TS 29.571 [8] | Identifies a DNN. |  |
| DurationSec | 3GPP TS 29.122 [4] | Indicates the time duration. |  |
| ExternalGroupId | 3GPP TS 29.122 [4] | External Group Identifier for a user group. |  |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |  |
| NotificationMethod | 3GPP TS 29.508 [26] | Represents the notification method. |  |
| ProblemDetails | 3GPP TS 29.122 [4] | Represents error related information. |  |
| Snssai | 3GPP TS 29.571 [8] | Identifies the S-NSSAI. |  |
| SpatialValidityCond | 3GPP TS 29.571 [8] | Represents the Spatial Validity Condition. | CoverageArea |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.15.5-1. |  |
| TemporalValidity | 3GPP TS 29.514 [7] | Indicates the time interval(s) during which the AF request is to be applied. |  |
| Uinteger | 3GPP TS 29.571 [8] | Unsigned integer. |  |
| Uri | 3GPP TS 29.122 [4] | Identifies a referenced resource. |  |
| WebsockNotifConfig | 3GPP TS 29.122 [4] | Contains the configuration parameters to set up notification delivery over Websocket protocol. | Notification_websocket |

#### 5.15.4.3	Structured data types

##### 5.15.4.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.15.4.3.2	Type: TimeSyncExposureSubsc

Table 5.15.4.3.2-1: Definition of type TimeSyncExposureSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| exterGroupId | ExternalGroupId | C | 0..1 | Identifies a group of UE(s) for which the time synchronization capabilities is requested. (NOTE 1) |  |
| gpsis | array(Gpsi) | C | 1..N | Contains a list of UE for which the time synchronization capabilities is requested. (NOTE 1) |  |
| anyUeInd | boolean | C | 0..1 | Identifies whether the AF request applies to any UE (i.e. all UEs). - "true": the AF request is applicable to any UE. - "false": the AF request is not applicable to any UE. - Default value is "false" if omitted. (NOTE 1) (NOTE 2) |  |
| notifMethod | NotificationMethod | O | 0..1 | If "notifMethod" is not supplied, the default value "ON_EVENT_DETECTION" applies. |  |
| dnn | Dnn | C | 0..1 | Identifies a DNN, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. (NOTE 2) |  |
| snssai | Snssai | C | 0..1 | Identifies an S-NSSAI. (NOTE 2) |  |
| afServiceId | string | O | 0..1 | Identifies a service on behalf of which the AF is issuing the request. |  |
| subscribedEvents | array(SubscribedEvent) | O | 1..N | Identifies the requirement to be notified of the event(s). |  |
| eventFilters | array(EventFilter) | O | 1..N | Contains the filter conditions to match for notifying the event(s) of time synchronization capabilities for a list of UE(s). |  |
| subsNotifUri | Uri | M | 1 | Notification URI for time sensitive capability reporting. |  |
| subsNotifId | string | M | 1 | Notification Correlation ID assigned by the NF service consumer. |  |
| maxReportNbr | Uinteger | O | 0..1 | If omitted, there is no limit. |  |
| expiry | DateTime | C | 0..1 | This attribute indicates the expiry time of the subscription, after which the NEF shall not send any event notifications and the subscription becomes invalid. It may be included in an event subscription request and may be included in an event subscription response based on operator policies. If an expiry time was included in the request, then the expiry time returned in the response should be less than or equal to that value. If the expiry time is not included in the response, the NF service consumer shall not associate an expiry time for the subscription. |  |
| repPeriod | DurationSec | C | 0..1 | Is supplied for notification Method "periodic". |  |
| requestTestNotification | boolean | O | 0..1 | Indicates whether the AF requests the NEF to send a test notification. - Set to "true" by the AF to request the NEF to send a test notification as defined in clause 5.2.5.3 of 3GPP TS 29.122 [4]. - Set to "false" by the AF not to request the NEF to send a test notification. - Default value is "false" if omitted. | Notification_test_event |
| websockNotifConfig | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol. | Notification_websocket |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the features supported by the NF service consumer. This parameter shall be supplied by the NF service consumer in the POST request and the response that requested the creation of an Individual Time Synchronization Subscription resource. |  |
| NOTE 1: Only one of the properties "gpsis", "anyUeInd" or "exterGroupId" shall be included. NOTE 2 The properties of "anyUeInd" may be included only when the properties of "dnn" and "snssai" are included. |  |  |  |  |  |

##### 5.15.4.3.3	Type: TimeSyncCapability

Table 5.15.4.3.3-1: Definition of type TimeSyncCapability

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| upNodeId | Uint64 | M | 1 | Identifies the applicable NW-TT. Contains a TSC user plane node Id. If integrated with TSN, the user plane node Id is a bridge Id defined in IEEE 802.1Q [51] clause 14.2.5. |  |
| gmCapables | array(GmCapable) | C | 1..N | Indicates whether user plane node supports acting as a gPTP and/or PTP grandmaster. (NOTE 1) |  |
| asTimeRes | AsTimeResource | C | 0..1 | Indicates the supported 5G clock quality (i.e. the source of time used by the 5GS). (NOTE 1) |  |
| ptpCapForUes | map(PtpCapabilitiesPerUe) | C | 1..N | Contains the PTP capabilities supported by the list of UE(s). The key of the map is the gpsi. Shall be present if the "gmCapables" attribute is included. (NOTE 2) |  |
| NOTE 1: At least one of the "gmCapables" attribute and "asTimeRes" attribute shall be included. NOTE 2: The "ptpCapForUes" attribute shall contain the PTP capabilities for previously target UE(s) for which the PTP capabilities changed and/or the PTP capabilities for new UE(s). |  |  |  |  |  |

##### 5.15.4.3.4	Void

##### 5.15.4.3.5	Void

##### 5.15.4.3.6	Type: TimeSyncExposureConfig

Table 5.15.4.3.6-1: Definition of type TimeSyncExposureConfig

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| upNodeId | Uint64 | M | 1 | Identifies the applicable NW-TT. Contains a TSC user plane node Id. If integrated with TSN, the user plane node Id is a bridge Id defined in IEEE 802.1Q [41] clause 14.2.5. |  |
| reqPtpIns | PtpInstance | M | 1 | Identifies the PtP instance configuration and activation requested by the AF. |  |
| gmEnable | boolean | C | 0..1 | Indicates whether the AF requests 5GS to act as a grandmaster for PTP or gPTP. - "true" indicates that the AF requests the 5GS to act as a grandmaster for PTP or gPTP. - "false" indicates that the AF does not request the 5GS to act as a grandmaster for PTP or gPTP. - Default value is "false" if omitted. |  |
| gmPrio | Uinteger | C | 0..1 | Indicates a priority used as defaultDS.priority1 when generating Announce message when 5GS acts as (g)PTP GM. It may be present if the "gmEnable" is set to true. |  |
| timeDom | Uinteger | M | 1 | Indicate the (g)PTP domain that the (TSN)AF is located in. |  |
| timeSyncErrBdgt | Uinteger | O | 0..1 | Indicates the time synchronization error budget for the time synchronization service in units of nanoseconds. Minimum = 1. |  |
| tempValidity | TemporalValidity | O | 0..1 | Indicates the time period when the time synchronization service for a PTP instance is active. |  |
| configNotifUri | Uri | M | 1 | Notification URI for configuration state reporting. |  |
| configNotifId | string | M | 1 | Notification Correlation ID assigned by the NF service consumer. |  |
| coverageArea | SpatialValidityCond | O | 0..1 | Time Synchronization Coverage Area. (NOTE) | CoverageArea |
| clkQltDetLvl | ClockQualityDetailLevel | O | 0..1 | Indicates the clock quality detail level. For (g)PTP services, its value, if provided, shall be set to "ACCEPT_INDICATION". | NetTimeSyncStatus |
| clkQltAcptCri | ClockQualityAcceptanceCriterion | C | 0..1 | Indicates the clock quality acceptance criteria, and it is used to determine whether the time synchronization status for the (g)PTP service is acceptable / not acceptable. This attribute shall be present when the "clkQltDetLvl" attribute is present. | NetTimeSyncStatus |
| NOTE: The "trackingAreaList" attribute within the "coverageArea" attribute is not applicable for the untrusted AF. |  |  |  |  |  |

##### 5.15.4.3.7	Type: TimeSyncExposureSubsNotif

Table 5.15.4.3.7-1: Definition of type TimeSyncExposureSubsNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subsNotifId | string | M | 1 | Notification Correlation ID assigned by the NF service consumer. |  |
| eventNotifs | array(SubsEventNotification) | M | 1..N | Notifications about subscribed Individual Events. |  |

##### 5.15.4.3.8	Type SubsEventNotification

Table 5.15.4.3.8-1: Definition of type SubsEventNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | SubscribedEvent | M | 1 | Subscribed events. |  |
| timeSyncCapas | array(TimeSyncCapability) | C | 1..N | Contains a list of time synchronization capabilities for the 5GS (list of User Plane Node Id) and the UE(s) (per User Plane Node Id). This attribute shall be present if the reported event is "AVAILABILITY_FOR_TIME_SYNC_SERVICE". |  |

##### 5.15.4.3.9	Type: TimeSyncExposureConfigNotif

Table 5.15.4.3.9-1: Definition of type TimeSyncExposureConfigNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| configNotifId | string | M | 1 | Notification Correlation ID assigned by the NF service consumer. |  |
| stateOfConfig | StateOfConfiguration | M | 1 | Indicates the current state of time synchroniztion service configuration |  |

##### 5.15.4.3.10	Type: EventFilter

Table 5.15.4.3.10-1: Definition of type EventFilter

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| instanceTypes | array(InstanceType) | O | 1..N | Indicates the PTP instance type(s). |  |
| transProtocols | array(Protocol) | O | 1..N | Indicates the transport protocol type(s). |  |
| ptpProfiles | array(string) | O | 1..N | Identifies the supported PTP profiles. |  |

##### 5.15.4.3.11	Type: PtpCapabilitiesPerUe

Table 5.15.4.3.11-1: Definition of type PtpCapabilitiesPerUe

| Attribute name |  | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- | --- |
| gpsi |  | Gpsi | M | 1 | Identifies the UE to which the reported PTP instance below apply. |  |
| ptpCaps |  | array(ReportedCapability) | M | 1..N | Contains the reported PTP capabilities for the UE. (NOTE) |  |
| NOTE: When the "SupportReport" feature is supported, it also indicates the PTP capabilities of the UE changed because of the termination of the PDU session. In this case, the "ptpCaps" attribute shall contain a ReportedCapability entry with the "avStatus" attribute set to "PDU_SESSION_TERMINATION". |  |  |  |  |  |  |

##### 5.15.4.3.12	Type: PtpInstance

Table 5.15.4.3.12-1: Definition of type PtpInstance

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| instanceType | InstanceType | M | 1 | Indicates the PTP instance type. |  |
| protocol | Protocol | M | 1 | Indicates the protocol type. |  |
| ptpProfile | string | M | 1 | Identifies the PTP profile. |  |
| portConfigs | arrary(ConfigForPort) | O | 1..N | Contains the configurations for the PTP port(s) in the PTP instance. |  |

##### 5.15.4.3.13	Void

##### 5.15.4.3.14	Void

##### 5.15.4.3.15	Void

##### 5.15.4.3.16	Void

##### 5.15.4.3.17	Type: StateOfConfiguration

Table 5.15.4.3.17-1: Definition of type StateOfConfiguration

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| stateOfNwtt |  | boolean |  | O |  | 0..1 |  | When the PTP port state is Leader, Follower or Passive, it is included and set to true to indicate the state of configuration for NW-TT port is active; when PTP port state is in any other case, it is included and set to false to indicate the state of configuration for NW-TT port is inactive. Default value is false. (NOTE) |  |  |  |
| clkQltIndOfNwtt |  | AcceptanceCriteriaResultIndication |  | O |  | 0..1 |  | Indicates the clock quality acceptance criteria changes ("ACCEPTABLE", "NOT_ACCEPTABLE") for any of the PTP port(s) in NW-TT of the PTP instance. |  | NetTimeSyncStatus |  |
| stateOfDstts |  | array(StateOfDstt) |  | O |  | 1..N |  | Contains the PTP port states of the DS-TT(s). |  |  |  |
| NOTE: Leader and Follower terms in this specification are aligned with NOTE 2 in clause 5.27.1.2.2.1 of 3GPP TS 23.501 [3]. |  |  |  |  |  |  |  |  |  |  |  |

##### 5.15.4.3.18	Type: ConfigForPort

Table 5.15.4.3.18-1: Definition of type ConfigForPort

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| gpsi | Gpsi | C | 0..1 | Identifies the UE/DS-TT to which the configuration parameters below apply. (NOTE) |  |
| n6Ind | boolean | C | 0..1 | Indicates whether the N6 termination that the configuration parameters below apply to the N6 interface. - "true" indicates that the N6 termination which the parameters below apply. - "false" indicates that the N6 termination which the parameters below do not apply. - Default value is "false" if omitted. (NOTE) |  |
| ptpEnable | boolean | O | 0..1 | This is used to set the portDS.portEnable. - 'true" indicates to set the portDS.portEnable. - "false" indicates not to set the portDS.portEnable. - Default value is "false" if omitted, the default value as described in the PTP Profile is used. |  |
| logSyncInter | integer | O | 0..1 | Specifies the mean time interval between successive Sync messages. This is applicable for IEEE Std 1588 [45] Boundary Clock or IEEE Std 802.1AS [46] operation. If omitted, the default value as described in the PTP Profile is used. |  |
| logSyncInterInd | boolean | O | 0..1 | Indicates how the value of the "logSyncInter" attribute is to be used. - When set to "false", the value of "logSyncInter" attribute is used to set the initialLogSyncInterval as described in IEEE Std 802.1AS [46]. - When set to "true", the value of "logSyncInter" attribute is used to set the mgtSettableLogSyncInterval as described in IEEE Std 802.1AS [46]. - If omitted, the default value as described in the IEEE Std 802.1AS [46] is used. |  |
| logAnnouInter | integer | O | 0..1 | Specifies the mean time interval between successive Announce messages. This is applicable for IEEE Std 1588 [45] Boundary Clock or IEEE Std 802.1AS [46] operation. If omitted, the default value as described in the PTP Profile is used. |  |
| logAnnouInterInd | boolean | O | 0..1 | Indicates how the value of the "logSyncInter" attribute is to be used. - When set to "false", the value of "logAnnouInter" attribute is used to set the initialLogAnnounceInterval as described in IEEE Std 802.1AS [46]. - When set to "true", the value of "logAnnouInter" attribute is used to set the mgtSettableLogAnnounceInterval as described in IEEE Std 802.1AS [46]. - If omitted, the default value as described in the IEEE Std 802.1AS [46] is used. |  |
| NOTE: Either "gpsi" or "n6Ind" attribute shall be included. |  |  |  |  |  |

##### 5.15.4.3.19	Type: StateOfDstt

Table 5.15.4.3.19-1: Definition of type StateOfDstt

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| gpsi |  | Gpsi |  | M |  | 1 |  | Identifies the UE/DS-TT which the parameters below apply. |  |  |  |
| state |  | boolean |  | M |  | 1 |  | When the PTP port state is Leader, Follower or Passive, it is included and set to true to indicate the state of configuration for DS-TT port is active; when PTP port state is in any other case, it is included and set to false to indicate the state of configuration for DS-TT port is inactive. Default value is false. |  |  |  |
| clkQltIndOfDstt |  | AcceptanceCriteriaResultIndication |  | O |  | 0..1 |  | Indicates the clock quality acceptance criteria changes ("ACCEPTABLE", "NOT_ACCEPTABLE") for the indicated DS-TT port of the PTP instance. |  | NetTimeSyncStatus |  |
| NOTE: Leader and Follower terms in this specification are aligned with NOTE 2 in clause 5.27.1.2.2.1 of TS 23.501 [3]. |  |  |  |  |  |  |  |  |  |  |  |

##### 5.15.4.3.20	Void

##### 5.15.4.3.21	Void

##### 5.15.4.3.22	Type: AvailReport

Table 5.15.4.3.22-1: Definition of type AvailReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| avStatus | AvailStatus | O | 0..1 | Indicates the availability status. |  |

#### 5.15.4.4	Simple data types and enumerations

##### 5.15.4.4.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.15.4.4.2	Simple data types

The simple data types defined in table 5.15.4.4.2-1 shall be supported.

Table 5.15.4.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 5.15.4.4.3	Void

##### 5.15.4.4.4	Enumeration: Protocol

Table 5.15.4.4.4-1: Enumeration Protocol

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ETH | Indicates Ethernet as defined in IEEE Std 1588 [45] Annex E is supported. |  |
| IPV4 | Indicates IPv4 as defined in IEEE Std 1588 [45] Annex C is supported. |  |
| IPV6 | Indicates IPv6 as defined in IEEE Std 1588 [45] Annex D is supported |  |

##### 5.15.4.4.5	Enumeration: GmCapable

Table 5.15.4.4.5-1: Enumeration GmCapable

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| GPTP | gPTP grandmaster is supported. |  |
| PTP | PTP grandmaste is supported. |  |

##### 5.15.4.4.6	Enumeration: SubscribedEvent

Table 5.15.4.4.6-1: Enumeration SubscribedEvent

| Enumeration value | Description |
| --- | --- |
| AVAILABILITY_FOR_TIME_SYNC_SERVICE | The AF requests to be notified about 5GS and/or UE availablility and capability for time synchronization service. |

##### 5.15.4.4.7	Enumeration: InstanceType

Table 5.15.4.4.7-1: Enumeration InstanceType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| BOUNDARY_CLOCK | Indicates Boundary Clock as defined in IEEE Std 1588 [45]. |  |
| E2E_TRANS_CLOCK | Indicates End-to-End Transparent Clock as defined in IEEE Std 1588 [45]. |  |
| P2P_TRANS_CLOCK | Indicates Peer-to-Peer Transparent Clock as defined in IEEE Std 1588 [45]. |  |
| P2P_RELAY_INSTANCE | Indicates PTP Relay instance as defined in IEEE Std 802.1AS [46] |  |

##### 5.15.4.4.8	Enumeration: AsTimeResource

The enumeration AsTimeResource represents the supported 5G clock quality (i.e. the source of time used by the 5GS). It shall comply with the provisions defined in table 5.15.4.4.8-1.

Table 5.15.4.4.8-1: Enumeration AsTimeResource

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ATOMIC_CLOCK | Indicates atomic clock is supported. |  |
| GNSS | Indicates Global Navigation Satellite System is supported. |  |
| TERRESTRIAL_RADIO | Indicates terrestrial radio is supported. |  |
| SERIAL_TIME_CODE | Indicates serial time code is supported. |  |
| PTP | Indicates PTP is supported. |  |
| NTP | Indicates NTP is supported. |  |
| HAND_SET | Indicates hand set is supported. |  |
| INTERNAL_OSCILLATOR | Indicates internal oscillator is supported. |  |
| OTHER | Indicates other source of time is supported. |  |

##### 5.15.4.4.9	Enumeration: AcceptanceCriteriaResultIndication

Table 5.15.4.4.9-1: Enumeration AcceptanceCriteriaResultIndication

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ACCEPTABLE | PTP port in (g)PTP service meets the clock quality acceptance criteria. |  |
| NON_ACCEPTABLE | PTP port in (g)PTP service does not meet the clock quality acceptance criteria. |  |

##### 5.15.4.4.10	Enumeration: AvailStatus

Table 5.15.4.4.10-1: Definition of type AvailStatus

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| PDU_SESSION_TERMINATION | The UE is not available for (g)PTP services because the PDU session is terminated. |  |

#### 5.15.4.5	Data types describing alternative data types or combinations of data types

##### 5.15.4.5.1	Type: ReportedCabability

The ReportedCapability data type extends the EventFilter data type with the attribute(s) defined in the AvailReport data type, to enable to indicate the availability status of the UE/DS-TT.

Table 5.15.4.5.1-1: Definition of type ReportedCabability as a list of non-exclusive alternatives

| Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- |
| EventFilter | 1 | It represents the conditions to match for notifying the event(s) of time synchronization capabilities for a list of UE(s). |  |
| AvailReport | 1 | It represents the availability status report of the UE. | SupportReport |

### 5.15.5	Used Features

The table below defines the features applicable to the TimeSyncExposure API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.15.5-1: Features used by TimeSyncExposure API

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_websocket | The delivery of notifications over Websocket is supported as defined in 3GPP TS 29.122 [4]. This feature requires that the Notification_test_event feature is also supported. |
| 2 | Notification_test_event | The testing of notification connection is supported as defined in 3GPP TS 29.122 [4]. |
| 3 | CoverageArea | Indicates support of the inclusion of the Time Synchronization Coverage Area in time synch configuration requests. |
| 4 | NetTimeSyncStatus | This feature indicates the support of network timing synchronization status and reporting. |
| 5 | SupportReport | This feature indicates the support of the report of whether the requested Time Synchronization is available and, based on the UE's Time Synchronization Subscription Data in UDM, authorized for the requested UEs. |

### 5.15.6	Error handling

#### 5.15.6.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.15.6.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the TimeSyncExposure API.

#### 5.15.6.3	Application Errors

The application errors defined for the TimeSyncExposure API are listed in table 5.15.6.3-1.

Table 5.15.6.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |
| UE_SERVICE_NOT_AUTHORIZED | 403 Forbidden | The AF request is not authorized or the AF requested parameter(s) are not allowed by UE's Time Synchronization Subscription Data in UDM. | SupportReport |

## 5.16	EcsAddressProvision API

### 5.16.0	Introduction

The Nnef_EcsAddressProvision service shall use the EcsAddressProvision API.

The API URI of EcsAddressProvision API shall be:

- {apiRoot}/3gpp-ecs-address-provision/v1

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-ecs-address-provision".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.16.1	Resources

#### 5.16.1.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.16.1.1-1 and the resources and HTTP methods used for the EcsAddressProvision API.

Figure 5.16.1.1-1: Resource URI structure of the EcsAddressProvision API

Table 5.16.1.1-1 provides an overview of the resources and HTTP methods applicable for the EcsAddressProvision API.

Table 5.16.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| ECS Address Provision Configurations | /{afId}/configurations | GET | Read all configurations for a given AF identified by {afId}. |
|  |  | POST | Create a new configuration to provision ECS address(es). |
| Individual ECS Address Provision Configuration | /{afId}/configurations/{configurationId} | GET | Read an existing configuration identified by {configurationId}. |
|  |  | PUT | Modify all of the properties of an existing configuration identified by {configurationId}. |
|  |  | DELETE | Delete a configuration identified by {configurationId}. |

#### 5.16.1.2	Resource: ECS Address Provision Configurations

##### 5.16.1.2.1	Introduction

This resource allows a AF to read all active ECS Address Provision Configurations for a given AF, or create an new Individual ECS Address Provision Configuration to the NEF.

##### 5.16.1.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-ecs-address-provision/v1/{afId}/configurations

This resource shall support the resource URI variables defined in table 5.16.1.2.2-1.

Table 5.16.1.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |

##### 5.16.1.2.3	Resource Methods

###### 5.16.1.2.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.16.1.2.2.

###### 5.16.1.2.3.2	GET

The GET method allows to read all active configurations for a given AF. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.16.1.2.3.2-1.

Table 5.16.1.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.16.1.2.3.2-2 and the response data structures and response codes specified in table 5.16.1.2.3.2-3.

Table 5.16.1.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.16.1.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(EcsAddressProvision) | M | 0..N | 200 OK | All the configurations for the AF are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during configuration retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during configuration retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.16.1.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.16.1.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.16.1.2.3.3	POST

The POST method creates a new resource to Individual ECS Address Provision Configuration for a given AF. The AF shall initiate the HTTP POST request message and the NEF shall respond to the message. The NEF shall construct the URI of the created resource.

This method shall support the request data structures specified in table 5.16.1.2.3.3-1 and the response data structures and response codes specified in table 5.16.1.2.3.3-2.

Table 5.16.1.2.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EcsAddressProvision | M | 1 | Parameters to create a confirguation to provision ECS address. |

Table 5.16.1.2.3.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EcsAddressProvision | M | 1 | 201 Created | The configuration was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.16.1.2.3.3-3: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-ecs-address-provision/v1/{afId}/configurations/{configurationId} |

#### 5.16.1.3	Resource: Individual ECS Address Provision Configuration

##### 5.16.1.3.1	Introduction

This resource allows a AF to read, update or delete an existing Individual ECS Address Provision Configuration.

##### 5.16.1.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-ecs-address-provision/v1/{afId}/configurations/{configurationId}

This resource shall support the resource URI variables defined in table 5.16.1.3.2-1.

Table 5.16.1.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |
| configurationId | string | Identifier of the configuration resource. |

##### 5.16.1.3.3	Resource Methods

###### 5.16.1.3.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.16.1.3.2.

###### 5.16.1.3.3.2	GET

The GET method allows to read the active configuration for a given AF and a given configuration Id. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.16.1.3.3.2-1.

Table 5.16.1.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.16.1.3.3.2-2 and the response data structures and response codes specified in table 5.16.1.3.3.2-3.

Table 5.16.1.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.16.1.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EcsAddressProvision | M | 1 | 200 OK | The information for the configuration in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during configuration retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during configuration retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.16.1.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.16.1.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.16.1.3.3.3	PUT

The PUT method is used to replace an existing Individual ECS Address Provision Configuration. The AF shall initiate the HTTP PUT request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.16.1.3.3.3-1 and the response data structures and response codes specified in table 5.16.1.3.3.3-2.

Table 5.16.1.3.3.3-1: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EcsAddressProvision | M | 1 | Replace an existing Individual ECS Address Provision Configuration. |

Table 5.16.1.3.3.3-2: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EcsAddressProvision | M | 1 | 200 OK | The configuration was updated successfully. |
| N/A |  |  | 204 No Content | The configuration was updated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during configuration modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during configuration modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.16.1.3.3.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.16.1.3.3.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.16.1.3.3.4	DELETE

The DELETE method deletes an existing Individual ECS Address Provision Configuration. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.7.1.3.3.4-1.

Table 5.7.1.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.7.1.3.3.4-2 and the response data structures and response codes specified in table 5.7.1.3.3.4-3.

Table 5.7.1.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.7.1.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The configuration was terminated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during configuration termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during configuration termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.7.1.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.7.1.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

### 5.16.1A	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.16.1B	Notifications

There are no notifications defined for this API in this release of the specification.

### 5.16.2	Data Model

#### 5.16.2.1	General

This clause specifies the application data model supported by the EcsAddressProvision API.

Table 5.16.2.1-1 specifies the data types defined for the EcsAddressProvision API.

Table 5.16.2.1-1: EcsAddressProvision API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| EcsAddressProvision | 5.16.2.3.2 | Represents ECS address provision configuration information. |  |

#### 5.16.2.2	Reused data types

The data types reused by the EcsAddressProvision API from other specifications are listed in table 5.16.2.2-1.

Table 5.16.2.2-1: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| EcsAuthMethod | 3GPP TS 29.503 [17] | Represents the ECS Authentication Methods. | ECSAuthMethods |
| EcsServerAddr | 3GPP TS 29.571 [8] | Represents the Edge Configuration Server (ECS) address configuration information. |  |
| Link | 3GPP TS 29.122 [4] | Represents a referenced resource. |  |
| MtcProviderInformation | 3GPP TS 29.571 [8] | Represents the MTC Provider Information. | enNB1 |
| PlmnIdNid | 3GPP TS 29.571 [8] | Represents the PLMN identifier. | HR-SBO |
| SpatialValidityCond | 3GPP TS 29.571 [8] | Represents the Spatial Validity Condition. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.16.3-1. |  |
| SupportedPlmn | 3GPP TS 29.503 [17] | ID of the PLMN supported by the ECS along with the list of ECSPs associated with the PLMN. | ECSSuppPlmns |
| TargetUeId | 5.6.3.3.7 | Represents the target UE(s) information. |  |

#### 5.16.2.3	Structured data types

##### 5.16.2.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.16.2.3.2	Type: EcsAddressProvision

Table 5.16.2.3.2-1: Definition of type EcsAddressProvision

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| self |  | Link |  | C |  | 0..1 |  | Identifies the individual configuration resource. Shall be present in the HTTP GET response when reading all the configurations for an AF. |  |  |  |
| ecsServerAddr |  | EcsServerAddr |  | M |  | 1 |  | Represents the ECS address(es). |  |  |  |
| mtcProviderId |  | MtcProviderInformation |  | O |  | 0..1 |  | Identifies the MTC Service Provider and/or MTC Application. |  | enNB1 |  |
| spatialValidityCond |  | SpatialValidityCond |  | O |  | 0..1 |  | Represents the spatial validity condition. |  |  |  |
| tgtUe |  | TargetUeId |  | O |  | 0..1 |  | Indicates the target UE information. |  |  |  |
| plmnId |  | PlmnIdNid |  | O |  | 0..1 |  | Identifier of the serving PLMN in which the provisioned information applies. |  | HR-SBO |  |
| ecsAuthMethods |  | array(EcsAuthMethod) |  | O |  | 1..N |  | Indicates the Supported ECS Authentication Method(s). |  | ECSAuthMethods |  |
| supportedPlmns |  | array(SupportedPlmn) |  | O |  | 1..N |  | Indicates the List of PLMNs and associated ECSPs for which EDN configuration information can be provided by the ECS. |  | ECSSuppPlmns |  |
| suppFeat |  | SupportedFeatures |  | M |  | 1 |  | Indicates the negotiated supported features. |  |  |  |

#### 5.16.2.4	Simple data types and enumerations

##### 5.16.2.4.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.16.2.4.2	Simple data types

The simple data types defined in table 5.16.2.4.2-1 shall be supported.

Table 5.16.2.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

### 5.16.3	Used Features

The table below defines the features applicable to the EcsAddressProvision API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.16.3-1: Features used by EcsAddressProvision API

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | HR-SBO | This feature indicates the support of provisioning ECS Address Configuration Information for when the serving PLMN is different from the PLMN of the NEF. |
| 2 | enNB1 | This feature indicates the support of Rel-18 enhancements to this northbound API. |
| 3 | ECSAuthMethods | This feature indicates the support of ECS authentication methods provided to the NEF. |
| 4 | ECSSuppPlmns | This feature indicates the support of ECS supported PLMNs. |

### 5.16.4	Error handling

#### 5.16.4.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.16.4.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the EcsAddressProvision API.

#### 5.16.4.3	Application Errors

The application errors defined for EcsAddressProvision API are listed in table 5.16.4.3-1.

Table 5.16.4.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

## 5.17	AMPolicyAuthorization API

### 5.17.0	Introduction

The Nnef_AMPolicyAuthorization service shall use the AMPolicyAuthorization API.

The API URI of AMPolicyAuthorization API shall be:

- {apiRoot}/3gpp-am-policyauthorization/v1

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-am-policyauthorization".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.17.1	Resources

#### 5.17.1.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.17.1.1-1 and the resources and HTTP methods used for the AMPolicyAuthorization API.

Figure 5.17.1.1-1: Resource URI structure of the AMPolicyAuthorization API

Table 5.17.1.1-1 provides an overview of the resources and HTTP methods applicable for the AMPolicyAuthorization API.

Table 5.17.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Application AM Contexts | /{afId}/app-am-contexts | POST | Create a new Individual application AM context resource and may create the child AM Policy Events Subscription sub-resource. |
| Individual application AM Context | /{afId}/app-am-contexts/{appAmContextId} | GET | Reads an existing Individual application AM context resource. |
|  |  | PATCH | Updates an existing Individual application AM context resource. It can also create or update an AM Policy Events Subscription sub-resource. |
|  |  | DELETE | Deletes an existing Individual application AM context resource and the child AM Policy Events Subscription sub-resource. |
| AM Policy Events Subscription | /{afId}/app-am-contexts/{appAmContextId}/events-subscription | PUT | Creates a new AM Policy Events Subscription sub-resource or modifies an existing AM Policy Events Subscription sub-resource. |
|  |  | DELETE | Deletes an AM Policy Events Subscription sub-resource. |

#### 5.17.1.2	Resource: Application AM Contexts

##### 5.17.1.2.1	Introduction

This resource allows an AF to request the creation of a new Individual application AM context resource.

##### 5.17.1.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-am-policyauthorization/v1/{afId}/app-am-contexts

This resource shall support the resource URI variables defined in table 5.17.1.2.2-1.

Table 5.17.1.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |

##### 5.17.1.2.3	Resource Methods

###### 5.17.1.2.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.17.1.2.2.

###### 5.17.1.2.3.2	POST

The POST method creates a new resource to Individual application AM context for a given AF. The AF shall initiate the HTTP POST request message and the NEF shall respond to the message. The NEF shall construct the URI of the created resource.

This method shall support the request data structures specified in table 5.17.1.2.3.2-1 and shall support the response data structures and response codes specified in table 5.17.1.2.3.2-2.

Table 5.17.1.2.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AppAmContextExpData | M | 1 | Contains the exposure information for the creation of a new Individual application AM context resource. |

Table 5.17.1.2.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AppAmContextExpRespData | M | 1 | 201 Created | Successful case. The Individual application AM context resource was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| ProblemDetails | O | 0..1 | 500 Internal Server Error | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure causes are described in clause 5.17.5. |  |  |  |  |

Table 5.17.1.2.3.2-3: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-am-policyauthorization/v1/{afId}/ app-am-contexts/{appAmContextId} |

#### 5.17.1.3	Resource: Individual Application AM Context

##### 5.17.1.3.1	Introduction

This resource allows an AF to read, update or delete an existing Individual application AM context.

##### 5.17.1.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-am-policyauthorization/v1/{afId}/app-am-contexts/{appAmContextId}

This resource shall support the resource URI variables defined in table 5.17.1.3.2-1.

Table 5.17.1.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |
| appAmContextId | string | Identifier of the application AM context formatted according to IETF RFC 3986 [44]. |

##### 5.17.1.3.3	Resource Methods

###### 5.17.1.3.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.17.1.3.2.

###### 5.17.1.3.3.2	GET

The GET method allows to read the existing application AM context for a given AF and a given application AM context Id. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.17.1.3.3.2-1.

Table 5.17.1.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.17.1.3.3.2-2, the response data structures and response codes specified in table 5.17.1.3.3.2-3 and the Location Headers specified in table 5.17.1.3.3.2-4 and table 5.17.1.3.3.2-5.

Table 5.17.1.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.17.1.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AppAmContextExpData | M | 1 | 200 OK | Successful case. The exposure information of an existing Individual application AM context in the request URI is returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during the AM context retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during the AM context retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| ProblemDetails | O | 0..1 | 404 Not Found | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure causes are described in clause 5.17.5. |  |  |  |  |

Table 5.17.1.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.17.1.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.17.1.3.3.3	PATCH

The PATCH method is used to modify an existing Individual application AM context. The AF shall initiate the HTTP PATCH request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.17.1.3.3.3-1.

Table 5.17.1.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.17.1.3.3.3-2, the response data structures and response codes specified in table 5.17.1.3.3.3-3 and the Location Headers specified in table 5.17.1.3.3.3-4 and table 5.17.1.3.3.3-5.

Table 5.17.1.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AppAmContextExpUpdateData | M | 1 | Contains the modification(s) to be applied to the Individual application AM context resource. |

Table 5.17.1.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AppAmContextExpRespData | M | 1 | 200 OK | Successful case. The exposure information of the updated application AM context. |
| N/A |  |  | 204 No Content | The application AM context was updated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during the AM context modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during the AM context modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| ProblemDetails | O | 0..1 | 400 Bad Request | (NOTE 2) |
| ProblemDetails | O | 0..1 | 404 Not Found | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure causes are described in clause 5.17.5. |  |  |  |  |

Table 5.17.1.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.17.1.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.17.1.3.3.4	DELETE

The DELETE method deletes an existing Individual application AM context. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.17.1.3.3.4-1.

Table 5.17.1.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.17.1.3.3.4-2 and the response data structures and response codes specified in table 5.17.1.3.3.4-3, and the Location Headers specified in table 5.17.1.3.3.4-4 and table 5.17.1.3.3.4-5.

Table 5.17.1.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.17.1.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | Successful case. The application AM context was terminated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during the AM context termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during the AM context termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| ProblemDetails | O | 0..1 | 404 Not Found | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure causes are described in clause 5.17.5. |  |  |  |  |

Table 5.17.1.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.17.1.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

#### 5.17.1.4	Resource: AM Policy Events Subscription

##### 5.17.1.4.1	Introduction

This resource allows an AF to create a new AM policy events subscription sub-resource or modifies an existing AM policy events subscription sub-resource.

##### 5.17.1.4.2	Resource Definition

Resource URI: {apiRoot}/3gpp-am-policyauthorization/v1/{afId}/app-am-contexts/{appAmContextId}/events-subscription

This sub-resource shall support the resource URI variables defined in table 5.17.1.4.2-1.

Table 5.17.1.4.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |
| appAmContextId | string | Identifier of the application AM context formatted according to IETF RFC 3986 [44]. |

##### 5.17.1.4.3	Resource Methods

###### 5.17.1.4.3.1	General

The following clauses specify the resource methods supported by the sub-resource as described in clause 5.17.1.4.2.

###### 5.17.1.4.3.2	PUT

The PUT method allows to create a new AM policy events subscription sub-resource in an existing application AM context or modifies an existing AM policy events subscription sub-resource. The AF shall initiate the HTTP PUT request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.17.1.4.3.2-1.

Table 5.17.1.4.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.17.1.4.3.2-2, the response data structures and response codes specified in table 5.17.1.4.3.2-3 and the Location Headers specified in table 5.17.1.4.3.2-4, table 5.17.1.4.3.2-5 and table 5.17.1.4.3.2-6.

Table 5.17.1.4.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AmEventsSubscData | M | 1 | Contains the information for the creation and/or modification of the AM Policy Events Subscription. |

Table 5.17.1.4.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AmEventsSubscRespData | M | 1 | 201 Created | Successful case. The AM policy events subscription sub-resource was created. The representation of the AM Policy Events Subscription sub-resource is included within the properties of the AmEventsSubscData data type. The one or more matched events, if available, are included within the properties of the AmEventsNotification data type. |
| AmEventsSubscRespData | M | 1 | 200 OK | Successful case. The AM policy events subscription sub-resource was modified and a representation of the sub-resource is returned. The representation of the AM Policy Events Subscription sub-resource is included within the properties of the AmEventsSubscData data type. The one or more matched events, if available, are included within the properties of the AmEventsNotification data type. |
| N/A |  |  | 204 No Content | Successful case. The AM policy events subscription sub-resource was modified successfully, with no content to be sent in the response message body. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during the AM policy events subscription or modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during the AM policy events subscription or modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| ProblemDetails | O | 0..1 | 404 Not Found | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure causes are described in clause 5.17.5. |  |  |  |  |

Table 5.17.1.4.3.2-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains the URI of the resource in which an AM policy events subscription sub-resource has been created, according to the structure: {apiRoot}/3gpp-am-policyauthorization/v1/{afId}/app-am-contexts/{appAmContextId}/events-subscription |

Table 5.17.1.4.3.2-5: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.17.1.4.3.2-6: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.17.1.4.3.3	DELETE

The DELETE method deletes existing subscribed AM policy event(s) within the existing Individual application AM context. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.17.1.4.3.3-1.

Table 5.17.1.4.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.17.1.4.3.3-2 and the response data structures and response codes specified in table 5.17.1.4.3.3-3 and the Location Headers specified in table 5.17.1.4.3.3-4 and table 5.17.1.4.3.3-5.

Table 5.17.1.4.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.17.1.4.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | Successful case. The AM policy event(s) subscription resource is deleted. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during the AM policy events deletion. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during the AM policy events deletion. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| ProblemDetails | O | 0..1 | 404 Not Found | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure causes are described in clause 5.17.5. |  |  |  |  |

Table 5.17.1.4.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.17.1.4.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | An alternative URI of the resource located in an alternative NEF. |

### 5.17.1A	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.17.2	Notifications

#### 5.17.2.1	Introduction

Upon receipt of AM Event Notification from the PCF indicating the subscribed AM policy event is detected, the NEF shall send an HTTP POST message including the notified AM policy event to the AF. The NEF and the AF shall support the notification mechanism as described in clause 5.2.5 of 3GPP TS 29.122 [4].

Table 5.17.2.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| AM Event Notification | {eventNotifUri} | POST | The AM policy changes event notification is provided by the NEF to the AF. |

#### 5.17.2.2	AM Event Notification

##### 5.17.2.2.1	Description

The AM Event Notification is used by the NEF to report one or several observed AM policy change events to AF that has subscribed to such Notifications via the AM policy events subscription sub-resource.

##### 5.17.2.2.2	Callback URI

The Callback URI "{eventNotifUri}" shall be used with the callback URI variables defined in table 5.17.2.2.2-1.

Table 5.17.2.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| eventNotifUri | Callback reference provided by the AF during creation/modification of the subscription within the AM policy events subscription sub-resource as defined in Table 5.17.3.3.2-1 or Table 5.17.3.3.3-1 or Table 5.6.2.4-1 of 3GPP TS 29.534 [43]. |

##### 5.17.2.2.3	Operation Definition

###### 5.17.2.2.3.1	Notification via HTTP POST

This method shall support the request data structures specified in table 5.17.2.2.3.1-1 and the response data structures and response codes specified in table 5.17.2.2.3.1-2 and the Location Headers specified in table 5.17.2.2.3.1-3 and table 5.17.2.2.3.1-4.

Table 5.17.2.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AmEventsNotification | M | 1 | Provides information about the observed access and mobility policy change events by the NEF to the AF. |

Table 5.17.2.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The event notification is received successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during event notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during event notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.17.2.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

Table 5.17.2.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

###### 5.17.2.2.3.2	Notification via Websocket

If supported by both AF and NEF and successfully negotiated, the AM Event Notification may alternatively be delivered through the Websocket mechanism as defined in clause 5.2.5.4 of 3GPP TS 29.122 [4].

### 5.17.3	Data Model

#### 5.17.3.1	General

This clause specifies the application data model supported by the AMPolicyAuthorization API.

Table 5.17.3.1-1 specifies the data types defined for the AMPolicyAuthorization API.

Table 5.17.3.1-1: AMPolicyAuthorization API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AppAmContextExpData | 5.17.3.3.2 | Represents an Individual application AM context exposure resource. |  |
| AppAmContextExpRespData | 5.17.3.5.1 | Represents a response to a modification or creation request of an Individual Application AM context resource. It may contain the notification of the already met events. |  |
| AppAmContextExpUpdateData | 5.17.3.3.3 | Contains the modification(s) to be applied to the Individual application AM context exposure resource. |  |
| GeographicalArea | 5.17.3.3.4 | Contains geographical area information (e.g. a civic address or shapes). |  |

#### 5.17.3.2	Reused data types

The data types reused by the AMPolicyAuthorization API from other specifications are listed in table 5.17.3.2-1.

Table 5.17.3.2-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| AmEventsNotification | 3GPP TS 29.534 [43] | Describes the notification about the events occurred within an Individual application AM context resource. |
| AmEventsSubscData | 3GPP TS 29.534 [43] | Identifies the AM policy events the application subscribes to. |
| AmEventsSubscDataRm | 3GPP TS 29.534 [43] | This data type is defined in the same way as the "AmEventsSubscData" data type, but with the OpenAPI "nullable: true" property. |
| AmEventsSubscRespData | 3GPP TS 29.534 [43] | It represents a response to an AM policy events subscription request and contains the created/updated AM Policy Events Subscription resource. It may also include the Notification of the events met at the time of subscription. |
| CivicAddress | 3GPP TS 29.572 [34] | Identifies the civic address. |
| DurationSec | 3GPP TS 29.122 [4] | Indicates the time duration. |
| DurationSecRm | 3GPP TS 29.122 [4] | Indicates the time duration, same as the "DurationSec" data type, but with the OpenAPI "nullable: true" property. |
| GeographicArea | 3GPP TS 29.572 [34] | Identifies the geographical information with shapes. |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |
| Link | 3GPP TS 29.122 [4] | Identifies a referenced resource. |
| ProblemDetails | 3GPP TS 29.122 [4] | Represents error related information. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.17.4-1. |
| WebsockNotifConfig | 3GPP TS 29.122 [4] | Contains the configuration parameters to set up notification delivery over Websocket protocol. |

#### 5.17.3.3	Structured data types

##### 5.17.3.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.17.3.3.2	Type: AppAmContextExpData

Table 5.17.3.3.2-1: Definition of type AppAmContextExpData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| self | Link | C | 0..1 | Identifies the individual configuration resource. Shall be present in the HTTP GET response when reading all the configurations for an AF. |  |
| evSubscs | AmEventsSubscData | O | 0..1 | Represents the subscription to one or more AM policy events. |  |
| gpsi | Gpsi | M | 1 | Identifies the GPSI. |  |
| highThruInd | boolean | C | 0..1 | Indicates whether high throughput is desired for the indicated UE traffic. Set to "true" if high throughput is desired; otherwise set to "false". Default value is "false" if omitted. (NOTE) |  |
| covReqs | array(GeographicalArea) | C | 1..N | Identifies the allowed geographical areas. (NOTE) |  |
| policyDuration | DurationSec | O | 0..1 | Indicates the time duration that the policy shall last. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Indicates the negotiated supported features.. It shall be supplied by the AF in the POST request that requests a creation of an Individual application AM context resource. It shall be supplied by the NEF in the response to the POST request that requests a creation of an Individual application AM context resource. |  |
| requestTestNotification | boolean | O | 0..1 | Set to true by the AF to request the NEF to send a test notification as defined in clause 5.2.5.3 of 3GPP TS 29.122 [4]. Set to false or omitted otherwise. | Notification_test_event |
| websockNotifConfig | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol. | Notification_websocket |
| NOTE: Either "highThruInd" attribute or "covReqs" attribute or both shall be included. |  |  |  |  |  |

##### 5.17.3.3.3	Type: AppAmContextExpUpdateData

Table 5.17.3.3.3-1: Definition of type AppAmContextExpUpdateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| evSubscs | AmEventsSubscDataRm | O | 0..1 | Represents the subscription to one or more AM policy events. |  |
| highThruInd | boolean | O | 0..1 | Indicates whether high throughput is desired for the indicated UE traffic. Set to "true" if high throughput is desired; otherwise set to "false". |  |
| covReqs | array(GeographicalArea) | O | 1..N | Identifies the allowed geographical areas. (NOTE) |  |
| policyDuration | DurationSecRm | O | 0..1 | Indicates the time duration that the policy shall last. |  |
| NOTE: The value of the property shall be set to NULL for removal. |  |  |  |  |  |

##### 5.17.3.3.4	Type: GeographicalArea

Table 5.17.3.3.4-1: Definition of type GeographicalArea

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| civicAddress | CivicAddress | C | 0..1 | Identifies a civic address. |  |
| shapes | GeographicArea | C | 0..1 | Identifies a geographic area specified by different shapes. |  |
| NOTE: One of "civicAddress" attribute or "shapes" attribute shall be included. |  |  |  |  |  |

#### 5.17.3.4	Simple data types and enumerations

##### 5.17.3.4.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.17.3.4.2	Simple data types

The simple data types defined in table 5.17.3.4.2-1 shall be supported.

Table 5.17.3.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

#### 5.17.3.5	Data types describing alternative data types or combinations of data types

##### 5.17.3.5.1	Type: AppAmContextExpRespData

Table 5.17.3.5.1-1: Definition of type AppAmContextExpRespData as a list of non-exclusive alternatives

| Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- |
| AmEventsNotification | 0..1 | It represents the notification of a match event during the creation or modification of the Individual application AM context data. |  |
| AppAmContextExpData | 1 | It represents the Individual application AM context resource. |  |

### 5.17.4	Used Features

The table below defines the features applicable to the AMPolicyControl API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.17.4-1: Features used by AMPolicyAuthorization API

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_websocket | The delivery of notifications over Websocket is supported as described in 3GPP TS 29.122 [4]. This feature requires that the Notification_test_event feature is also supported. |
| 2 | Notification_test_event | The testing of notification connection is supported as described in 3GPP TS 29.122 [4]. |

### 5.17.5	Error handling

#### 5.17.5.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.17.5.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the AMPolicyAuthorization API.

#### 5.17.5.3	Application Errors

The application errors defined for the AMPolicyAuthorization API are listed in table 5.17.5.3-1.

Table 5.17.5.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| INVALID_POLICY_REQUEST | 400 Bad Request | The HTTP request is rejected because the service information for the AM context is invalid or insufficient for the PCF to perform the requested action. |
| APPLICATION_AM_CONTEXT_NOT_FOUND | 404 Not Found | The HTTP request is rejected because the specified Individual Application AM Context does not exist. |
| POLICY_ASSOCIATION_NOT_AVAILABLE | 500 Internal Server Error | The PCF failed in executing binding with the UE/AM Policy Context. |

## 5.18	AMInfluence API

### 5.18.0	Introduction

The Nnef_AMInfluence service shall use the AMInfluence API.

The API URI of AMInfluence API shall be:

- {apiRoot}/3gpp-am-influence/v1

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-am-influence".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.18.1	Resources

#### 5.18.1.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.18.1.1-1 and the resources and HTTP methods used for the AMInfluence API.

Figure 5.18.1.1-1: Resource URI structure of the AMInfluence API

Table 5.18.1.1-1 provides an overview of the resources and HTTP methods applicable for the AMInfluence API.

Table 5.18.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| AM Influence Subscription (NOTE) | /{afId}/subscriptions | GET | Read all subscriptions for a given AF. |
|  |  | POST | Create a new subscription to AM influence. |
| Individual AM Influence Subscription | /{afId}/subscriptions/{subscriptionId} | GET | Read an existing subscription to AM influence. |
|  |  | PUT | Update an existing subscription to AM influence. |
|  |  | PATCH | Modify an existing subscription to AM influence. |
|  |  | DELETE | Delete an existing subscription to AM influence. |
| NOTE: The "AM Influence Subscription" resource is a collection resource and corresponds to the collection of AM Influence Subscriptions managed by the NEF. |  |  |  |

#### 5.18.1.2	Resource: AM Influence Subscription

##### 5.18.1.2.1	Introduction

This resource allows an AF to read all active AM influence subscribtions and create a new subscription resource for a given AF.

##### 5.18.1.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-am-influence/v1/{afId}/subscriptions

This resource shall support the resource URI variables defined in table 5.18.1.2.2-1.

Table 5.18.1.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |

##### 5.18.1.2.3	Resource Methods

###### 5.18.1.2.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.18.1.2.2.

###### 5.18.1.2.3.2	GET

The GET method allows to read all active subscriptions for a given AF. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.18.1.2.3.2-1.

Table 5.18.1.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.18.1.2.3.2-2 and the response data structures and response codes specified in table 5.18.1.2.3.2-3.

Table 5.18.1.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.18.1.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(AmInfluSub) | M | 0..N | 200 OK | The subscription information for the AF in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.18.1.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.18.1.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.18.1.2.3.3	POST

The POST method creates a new subscription resource to AM influence subscription for a given AF. The AF shall initiate the HTTP POST request message and the NEF shall respond to the message. The NEF shall construct the URI of the created resource.

This method shall support the request data structures specified in table 5.18.1.2.3.3-1 and the response data structures and response codes specified in table 5.18.1.2.3.3-2.

Table 5.18.1.2.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AmInfluSub | M | 1 | Parameters to create a resource for the AM Influence and/or notification about service area coverage outcome events with the NEF. |

Table 5.18.1.2.3.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AmInfluSub | M | 1 | 201 Created | The subscription was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.18.1.2.3.3-3: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-am-Influence/v1/{afId}/subscriptions/{subscriptionId} |

#### 5.18.1.3	Resource: Individual AM Influence Subscription

##### 5.18.1.3.1	Introduction

This resource allows an AF to read, update or delete an existing AM Influence subscription.

##### 5.18.1.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-am-influence/v1/{afId}/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 5.18.1.3.2-1.

Table 5.18.1.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |
| subscriptionId | string | Identifier of the subscription. |

##### 5.18.1.3.3	Resource Methods

###### 5.18.1.3.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.18.1.3.2.

###### 5.18.1.3.3.2	GET

The GET method allows to read the active subscription for a given AF and subscription Id. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.18.1.3.3.2-1.

Table 5.18.1.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.18.1.3.3.2-2 and the response data structures and response codes specified in table 5.18.1.3.3.2-3.

Table 5.18.1.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.18.1.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AmInfluSub | M | 1 | 200 OK | The subscription information for the AF in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.18.1.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.18.1.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.18.1.3.3.3	PUT

The PUT method is used to replace an existing subscription resource to update a subscription. The AF shall initiate the HTTP PUT request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.18.1.3.3.3-1 and the response data structures and response codes specified in table 5.18.1.3.3.3-2.

Table 5.18.1.3.3.3-1: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AmInfluSub | M | 1 | Modify an existing AM Influence subscription. |

Table 5.18.1.3.3.3-2: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AmInfluSub | M | 1 | 200 OK | The subscription was replaced successfully and a representation is returned. |
| N/A |  |  | 204 No Content | The subscription was replaced successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.18.1.3.3.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.18.1.3.3.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.18.1.3.3.4	PATCH

The PATCH method allows to change some properties of an existing AM influence subscription. The AF shall initiate the HTTP PATCH request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.18.1.3.3.4-1 and the response data structures and response codes specified in table 5.18.1.3.3.4-2.

Table 5.18.1.3.3.4-1: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AmInfluSubPatch | M | 1 | Partial update of a subscription to AM influence and/or notifications about service area coverage outcome events with the NEF. |

Table 5.18.1.3.3.4-2: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AmInfluSub | M | 1 | 200 OK | The subscription was partial modified successfully and a representation is returned. |
| N/A |  |  | 204 No Content | The subscription was partial modified successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.18.1.3.3.4-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.18.1.3.3.4-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.18.1.3.3.5	DELETE

The DELETE method deletes an existing individual AM influence subscription for a given AF. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.18.1.3.3.5-1.

Table 5.18.1.3.3.5-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.18.1.3.3.5-2 and the response data structures and response codes specified in table 5.18.1.3.3.5-3.

Table 5.18.1.3.3.5-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.18.1.3.3.5-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The subscription was terminated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during subscription termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during subscription termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.18.1.3.3.5-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.18.1.3.3.5-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

### 5.18.1A	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.18.2	Notifications

#### 5.18.2.1	Introduction

Upon receipt of a service area coverage outcome event from the PCF, the NEF shall send an HTTP POST message including the notified event to the AF.The NEF and the AF shall support the notification mechanism as described in clause 5.2.5 of 3GPP TS 29.122 [4].

Table 5.18.2.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Event Notification | {notificationDestination} | POST | The service area coverage outcome event from the NEF to the AF. |

#### 5.18.2.2	Event Notification

##### 5.18.2.2.1	Description

The Event Notification is used by the NEF to report notification of the service area coverage outcome event from the PCF to the AF.

##### 5.18.2.2.2	Target URI

The Callback URI "{notificationDestination}" shall be used with the callback URI variables defined in table 5.18.2.2.2-1.

Table 5.18.2.2.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notificationDestination | Link | Callback reference provided by the AF during creation/modification of the subscription within the AmInfluSub data type as defined in Table 5.18.3.3.2-1 or the AmInfluSubPatch data type as defined in Table 5.18.3.3.3-1. |

##### 5.18.2.2.3	Operation Definition

5.18.2.2.3.1	Notification via HTTP POST

This method shall support the request data structures specified in table 5.18.2.2.3.1-1 and the response data structures and response codes specified in table 5.18.2.2.3.1-2.

Table 5.18.2.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AmInfluEventNotif | M | 1 | The service area coverage outcome event notification is provided by the NEF to the AF. |

Table 5.18.2.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The event notification is received successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during event notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during event notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.18.2.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

Table 5.18.2.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

5.18.2.2.3.2	Notification via Websocket

If supported by both AF and NEF and successfully negotiated, the AmInfluEventNotif may alternatively be delivered through the Websocket mechanism as defined in clause 5.2.5.4 of 3GPP TS 29.122 [4].

### 5.18.3	Data Model

#### 5.18.3.1	General

This clause specifies the application data model supported by the AMInfluence API.

Table 5.18.3.1-1 specifies the data types defined for the AMInfluence API.

Table 5.18.3.1-1: AMInfluence API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AmInfluEvent | 5.18.3.4.3 | Represents the service area coverage outcome event. |  |
| AmInfluEventNotif | 5.18.3.3.4 | Represents an AM influence event notification. |  |
| AmInfluSub | 5.18.3.3.2 | Represents an AM influence subscription. |  |
| AmInfluSubPatch | 5.18.3.3.3 | Represents parameters to request the modification of an AM influence subscription resource. |  |
| DnnSnssaiInformation | 5.18.3.3.5 | Represents a (DNN, SNSSAI) combination. |  |

#### 5.18.3.2	Reused data types

The data types reused by the AMInfluence API from other specifications are listed in table 5.18.3.2-1.

Table 5.18.3.2-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Dnn | 3GPP TS 29.571 [8] | Identifies a DNN. |
| DurationSec | 3GPP TS 29.122 [4] | Indicates the time duration. |
| DurationSecRm | 3GPP TS 29.122 [4] | Indicates the time duration, same as the "DurationSec" data type, but with the OpenAPI "nullable: true" property. |
| ExternalGroupId | 3GPP TS 29.122 [4] | External Group Identifier for a user group. |
| GeographicalArea | 5.17.3.3.4 | Identifies the geographical area information. |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |
| LinkRm | 3GPP TS 29.122 [4] | Indicates a referenced resource, same as the "Link" data type, but with the OpenAPI "nullable: true" property. |
| Link | 3GPP TS 29.122 [4] | Identifies a referenced resource. |
| PlmnId | 3GPP TS 29.571 [8] | Identifies a PLMN. |
| Snssai | 3GPP TS 29.571 [8] | Identifies the S-NSSAI. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.18.4-1. |
| WebsockNotifConfig | 3GPP TS 29.122 [4] | Contains the configuration parameters to set up notification delivery over Websocket protocol. |

#### 5.18.3.3	Structured data types

##### 5.18.3.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.18.3.3.2	Type: AmInfluSub

This type represents an AM influence subscription. The same structure is used in the subscription request and subscription response.

Table 5.18.3.3.2-1: Definition of type AMInfluSub

| Attribute name | Data type | P | Cardinality | Description | Applicability (NOTE 1) |
| --- | --- | --- | --- | --- | --- |
| afTransId | string | M | 1 | Identifies an NEF Northbound interface transaction request, generated by the AF. |  |
| gpsi | Gpsi | O | 0..1 | Identifies a user with GPSI. (NOTE 3) |  |
| externalGroupId | ExternalGroupId | O | 0..1 | Identifies a group of users. (NOTE 3) |  |
| anyUeInd | boolean | O | 0..1 | Identifies whether the AF request applies to any UE (i.e. all non-roaming UEs). - Set to "true": the AF request is applicable to any UE (i.e. all non-roaming UEs). - Set to "false": the AF request is not applicable to any UE (i.e. all non-roaming UEs). - Default value is "false" if omitted. (NOTE 2) (NOTE 3) |  |
| roamUePlmnIds | array(PlmnId) | O | 1..N | Indicates a list of PLMNs representing the home PLMN for the inbound roaming UEs in LBO roaming scenarios. (NOTE 3) (NOTE 5) | DCAMP_Roaming_LBO |
| dnnSnssaiInfos | array(DnnSnssaiInformation) | O | 1..N | Each of the element identifies a combination of (DNN, S-NSSAI). |  |
| afAppIds | array(string) | O | 1..N | Identifies application(s). |  |
| highThruInd | boolean | C | 0..1 | Indicates whether high throughput is desired for UE traffic. Set to "true" if high throughput is desired; otherwise set to "false". Default value is "false" if omitted. (NOTE 4) |  |
| geoAreas | array(GeographicalArea) | C | 1..N | Identifies geographical areas of the user where the request is applicable. (NOTE 4) |  |
| policyDuration | DurationSec | O | 0..1 | Indicates the time duration that the policy shall last. |  |
| self | Link | C | 0..1 | Link to the created resource. This parameter shall be supplied by the NEF in HTTP responses that include an object of AmInfluSub type. |  |
| subscribedEvents | array(AmInfluEvent) | O | 1..N | Identifies the requirement to be notified of the event(s). |  |
| notificationDestination | Link | C | 0..1 | Contains the Callback URL to receive the notification from the NEF. It shall be present if the "subscribedEvents" is present. |  |
| requestTestNotification | boolean | O | 0..1 | Set to true by the AF to request the NEF to send a test notification as defined in clause 5.2.5.3 of 3GPP TS 29.122 [4]. The default value is "false" if omitted. | Notification_test_event |
| websockNotifConfig | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol. | Notification_websocket |
| suppFeat | SupportedFeatures | C | 0..1 | Indicates the list of Supported features used as described in clause 5.18.4. This attribute shall be provided in the POST request and in the response of successful resource creation. |  |
| NOTE 1: Properties marked with a feature as defined in clause 5.18.4 are applicable as described in clause 5.2.7 of 3GPP TS 29.122 [4]. If no feature is indicated, the related property applies for all the features. NOTE 2: If target to any non-roaming UE, then "anyUeInd" attribute together with "dnnSnssaiInfos" attribute or "afAppIds" attribute shall be included. NOTE 3: One of individual UE identifier (i.e. "gpsi" attribute), External Group Identifier (i.e. "externalGroupId" attribute) or any UE indication (i.e. "anyUeInd" attribute) or any inbound roaming UEs PLMN ID (i.e. "roamUePlmnIds") shall be included. NOTE 4: Any of the "highThruInd" attribute or "geoAreas" attribute shall be included. NOTE 5: "roamUePlmnIds" attribute is applicable only in LBO roaming scenarios and if "afAppIds" attribute or "dnnSnssaiInfos" is provided. |  |  |  |  |  |

##### 5.18.3.3.3	Type: AmInfluSubPatch

This type represents AM influence subscription parameters provided by the AF to the NEF. The structure is used for HTTP PATCH request.

Table 5.18.3.3.3-1: Definition of type AmInfluSubPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| highThruInd | boolean | O | 0..1 | Indicates whether high throughput is desired for UE traffic. Set to "true" if high throughput is desired; Set to "false" if low throughput is desired; Otherwise, set to "false". |  |
| geoAreas | array(GeographicalArea) | O | 1..N | Identifies geographical areas of the user where the UE is located. (NOTE 1) |  |
| policyDuration | DurationSecRm | O | 0..1 | Indicates the time duration that the policy shall last. (NOTE 1) |  |
| dnnSnssaiInfos | array(DnnSnssaiInformation) | O | 1..N | Each of the element identifies a combination of (DNN, S-NSSAI). (NOTE 1) |  |
| subscribedEvents | array(AmInfluEvent) | O | 1..N | Identifies the requirement to be notified of the event(s). (NOTE 1) |  |
| notificationDestination | LinkRm | O | 0..1 | Contains the Callback URL to receive the notification from the NEF. (NOTE 1, NOTE 2) |  |
| afAppIds | array(string) | O | 1..N | Identifies application(s). (NOTE 1) |  |
| NOTE 1: The value of the property shall be set to NULL for removal. NOTE 2: The notificationDestination attribute set to NULL may be included only if the subscribedEvents attribute is included and set to NULL. |  |  |  |  |  |

##### 5.18.3.3.4	Type: AmInfluEventNotif

Table 5.18.3.3.4-1: Definition of type AmInfluEventNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability (NOTE 1) |
| --- | --- | --- | --- | --- | --- |
| afTransId | string | M | 1 | Identifies the AF request for AM policy influence that the event report is related to. |  |
| event | AmInfluEvent | M | 1 | Notified event. |  |
| geoAreas | array(GeographicalArea) | C | 1..N | Identifies geographical areas of the user where the UE is located. |  |

##### 5.18.3.3.5	Type: DnnSnssaiInformation

Table 5.18.3.3.5-1: Definition of type DnnSnssaiInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dnn | Dnn | O | 0..1 | Identifies a DNN, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. |  |
| snssai | Snssai | O | 0..1 | Identifies an S-NSSAI. |  |

#### 5.18.3.4	Simple data types and enumerations

##### 5.18.3.4.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.18.3.4.2	Simple data types

The simple data types defined in table 5.18.3.4.2-1 shall be supported.

Table 5.18.3.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 5.18.3.4.3	Enumeration: AmInfluEvent

The enumeration AmInfluEvent represents the service area coverage outcome event. It shall comply with the provisions defined in table 5.18.3.4.3-1.

Table 5.18.3.4.3-1: Enumeration AmInfluEvent

| Enumeration value | Description |
| --- | --- |
| SERVICE_AREA_COVRG_OUTCOME | Indicates the service area coverage outcome. |

### 5.18.4	Used Features

The table below defines the features applicable to the AMInfluence API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.18.4-1: Features used by AMInfluence API

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_websocket | The delivery of notifications over Websocket is supported as described in 3GPP TS 29.122 [4]. This feature requires that the Notification_test_event feature is also supported. |
| 2 | Notification_test_event | The testing of notification connection is supported as described in 3GPP TS 29.122 [4]. |
| 3 | DCAMP_Roaming_LBO | This feature indicates support for dynamically changing AM policy for LBO roaming scenarios for inbound roaming UE(s). |
| Feature: A short name that can be used to refer to the bit and to the feature, e.g. "Notification". Description: A clear textual description of the feature. |  |  |

### 5.18.5	Error handling

#### 5.18.5.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.18.5.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the AMInfluence API.

#### 5.18.5.3	Application Errors

The application errors defined for AMInfluence API are listed in table 5.18.5.3-1.

Table 5.18.5.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

## 5.19	MBSTMGI API

### 5.19.1	Introduction

The Nnef_MBSTMGI service shall use the MBSTMGI API.

The API URI of MBSTMGI API shall be:

- {apiRoot}/3gpp-mbs-tmgi/v1

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-mbs-tmgi".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.19.2	Resources

There are no resources defined for this API in this release of the specification.

### 5.19.3	Custom Operations without associated resources

#### 5.19.3.1	Overview

The structure of the custom operation URIs of the MBSTMGI API is shown in Figure 5.19.3.1-1.

Figure 5.19.3.1-1: Custom operation URI structure of the MBSTMGI API

Table 5.19.3.1-1 provides an overview of the custom operations and applicable HTTP methods.

Table 5.19.3.1-1: Custom operations without associated resources

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Allocate | /allocate | POST | Request the allocation of TMGI(s) for new MBS session(s) or the refresh of the expiry time of already allocated TMGI(s). |
| Deallocate | /deallocate | POST | Request the deallocation of TMGI(s). |

#### 5.19.3.2	Operation: Allocate

##### 5.19.3.2.1	Description

The custom operation enables an AF to request the allocation of TMGI(s) for new MBS session(s) or the refresh of the expiry time of already allocated TMGI(s).

##### 5.19.3.2.2	Operation Definition

This operation shall support the request and response data structures and response codes specified in table 5.19.3.2.2-1 and table 5.19.3.2.2-2.

Table 5.19.3.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TmgiAllocRequest | M | 1 | Represents the parameters to request the allocation of TMGI(s) for new MBS session(s) or the refresh of the expiry time of already allocated MBS TMGI(s). |

Table 5.19.3.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TmgiAllocResponse | M | 1 | 200 OK | Successful case: The TMGI allocation information (e.g. allocated MBS TMGIs, expiry time) or the refreshed expiry time for the concerned already allocated MBS TMGI(s) is/are returned to the requesting AF. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4] |
| ProblemDetailsTmgiAlloc | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure cases are described in clause 5.19.7. |  |  |  |  |

Table 5.19.3.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI of the resource located in an alternative NEF. |

Table 5.19.3.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI of the resource located in an alternative NEF. |

#### 5.19.3.3	Operation: Deallocate

##### 5.19.3.3.1	Description

The custom operation enables an AF to request the deallocation of TMGI(s).

##### 5.19.3.3.2	Operation Definition

This operation shall support the request and response data structures and response codes specified in table 5.19.3.3.2-1 and table 5.19.3.3.2-2.

Table 5.19.3.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TmgiDeallocRequest | M | 1 | Represents the MBS TMGI(s) deallocation request information (e.g. list of MBS TMGI(s) to be deallocated). |

Table 5.19.3.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case: The requested MBS TMGI(s) are deallocated. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4] |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.19.3.3.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI of the resource located in an alternative NEF. |

Table 5.19.3.3.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI of the resource located in an alternative NEF. |

### 5.19.4	Notifications

#### 5.19.4.1	General

Notifications shall comply to clause 5.2.5 of 3GPP TS 29.122 [4].

Table 5.19.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Notification of Allocated MBS TMGI(s) Timer Expiry | {notificationUri} | POST | Enable the NEF to notify an AF of the timer expiry for already allocated MBS TMGI(s). |

#### 5.19.4.2	Notification of Allocated MBS TMGI(s) Timer Expiry

##### 5.19.4.2.1	Description

The Notification is used by the NEF to report timer expiry of already allocated TMGI(s) to the AF.

##### 5.19.4.2.2	Target URI

The Callback URI "{notificationUri}" shall be used with the callback URI variables defined in table 5.19.4.2.2-1.

Table 5.19.4.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notificationUri | Callback URIprovided by the AF during the MBS TMGI(s) allocation or expiry time refresh request as defined in table 5.19.5.2.2-1. |

##### 5.19.4.2.3	Operation Definition

###### 5.19.4.2.3.1	Notification via HTTP POST

This method shall support the request data structures specified in table 5.19.4.2.3.1-1 and the response data structures and response codes specified in table 5.19.4.2.3.1-2.

Table 5.19.4.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ExpiryNotif | M | 1 | Represents the MBS TMGI(s) timer expiry notification information (e.g. list of MBS TMGI(s) for which the timer has expired). |

Table 5.19.4.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The notification is received successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during event notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during event notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.19.4.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

Table 5.19.4.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

###### 5.19.4.2.3.2	Notification via Websocket

If supported by both AF and NEF and successfully negotiated, the Notification of Allocated MBS TMGI(s) Timer Expiry may alternatively be delivered through the Websocket mechanism as defined in clause 5.2.5.4 of 3GPP TS 29.122 [4].

### 5.19.5	Data Model

#### 5.19.5.1	General

This clause specifies the application data model supported by the MBSTMGI API. Table 5.19.5.1-1 specifies the data types defined for the MBSTMGI API.

Table 5.19.5.1-1: MBSTMGI specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| ExpiryNotif | 5.19.5.2.5 | Represents TMGI(s) timer expiry notification information. |  |
| ExternalMbsServiceArea | 3GPP TS 29.571 [8] | Represents an external MBS service area. |  |
| MbsServiceArea | 3GPP TS 29.571 [8] | Represents an MBS service area. |  |
| ProblemDetailsTmgiAlloc | 5.19.5.4.1 | Represents an extension to the ProblemDetails data structure with additional error information related to TMGI Allocation. |  |
| ReducedMbsServArea | 5.19.5.2.6 | Represents the reduced MBS Service Area information. |  |
| TmgiAllocRequest | 5.19.5.2.2 | Represents the full set of parameters to initiate a TMGI(s) allocation request or the refresh of the expiry time of already allocated TMGI(s). |  |
| TmgiAllocResponse | 5.19.5.2.3 | Represents TMGI(s) allocation information or the refreshed expiry time for already allocated TMGI(s). |  |
| TmgiDeallocRequest | 5.19.5.2.4 | Represents information to request the deallocation of TMGI(s). |  |

Table 5.19.5.1-2 specifies data types re-used by the MBSTMGI API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the MBSTMGI API.

Table 5.19.5.1-2: MBSTMGI API re-used Data Types

| Data type |  | Reference |  | Comments |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ProblemDetails |  | 3GPP TS 29.122 [4] |  | Represents error related information. |  |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [8] |  | Represents the list of supported feature(s) and is used to negotiate the applicability of the optional features. |  |  |  |
| Tmgi |  | 3GPP TS 29.571 [8] |  | Contains a TMGI. |  |  |  |
| TmgiAllocate |  | 3GPP TS 29.532 [52] |  | Contains parameters to initiate a TMGI(s) allocation request or the refresh of the expiry time of already allocated TMGI(s). |  |  |  |
| TmgiAllocated |  | 3GPP TS 29.532 [52] |  | Contains the TMGI(s) allocation information or the refreshed expiry time for already allocated TMGI(s). |  |  |  |
| Uri |  | 3GPP TS 29.122 [4] |  | Contains a TMGI. |  |  |  |
| WebsockNotifConfig |  | 3GPP TS 29.122 [4] |  | Contains the configuration parameters to set up notification delivery over Websocket protocol. |  |  |  |

#### 5.19.5.2	Structured data types

##### 5.19.5.2.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.19.5.2.2	Type: TmgiAllocRequest

Table 5.19.5.2.2-1: Definition of type TmgiAllocRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afId | string | M | 1 | Contains the identifier of the AF that is sending the request. |  |
| tmgiParams | TmgiAllocate | M | 1 | Contains the parameters to request the allocation of TMGI(s) for new MBS session(s) or the refresh of the expiry time of already allocated MBS TMGI(s). |  |
| notificationUri | Uri | O | 0..1 | The notification URI via which the AF desires to receive notifications on timer expiry for MBS TMGI(s). |  |
| mbsServiceArea | MbsServiceArea | O | 0..1 | Indicates the MBS service area for the TMGI(s) to be allocated. This attribute may be provided by the AF for a local MBS service. (NOTE) |  |
| extMbsServiceArea | ExternalMbsServiceArea | O | 0..1 | Indicates the external MBS service area for the TMGI(s) to be allocated. This attribute may be provided by the AF for a local MBS service. (NOTE) |  |
| requestTestNotification | boolean | O | 0..1 | Indicates whether sending a test notification shall be performed. Set to "true" by the AF to request the NEF to send a test notification as defined in clause 5.2.5.3 of 3GPP TS 29.122 [4]. Set to "false" or omitted otherwise. | Notification_test_event |
| websockNotifConfig | WebsockNotifConfig | O | 0..1 | Represents configuration parameters to set up notification delivery over the Websocket protocol. | Notification_websocket |
| suppFeat | SupportedFeatures | C | 0..1 | Indicates the features supported by the AF. This attribute shall be provided if feature negotiation needs to take place. |  |
| NOTE: These attributes are mutually exclusive. Either one of them may be present. |  |  |  |  |  |

##### 5.19.5.2.3	Type: TmgiAllocResponse

Table 5.19.5.2.3-1: Definition of type TmgiAllocResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| tmgiInfo | TmgiAllocated | M | 1 | Contains the MBS TMGI(s) allocation information or the refreshed expiry time for already allocated MBS TMGI(s). |  |
| suppFeat | SupportedFeatures | C | 0..1 | Indicates the features supported by both the AF and the NEF. This attribute shall be provided if feature negotiation needs to take place and it was provided by the AF in the corresponding request body. |  |

##### 5.19.5.2.4	Type: TmgiDeallocRequest

Table 5.19.5.2.4-1: Definition of type TmgiDeallocRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afId | string | M | 1 | Contains the identifier of the AF that is sending the request. |  |
| tmgis | array(Tmgi) | M | 1..N | Contains the list of TMGI(s) to be deallocated. |  |

##### 5.19.5.2.5	Type: ExpiryNotif

Table 5.19.5.2.5-1: Definition of type ExpiryNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| tmgis | array(Tmgi) | M | 1..N | Contains the list of previously allocated MBS TMGI(s) for which the timer expired. |  |

##### 5.19.5.2.6	Type: ReducedMbsServArea

Table 5.19.5.2.6-1: Definition of type ReducedMbsServArea

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| reducedMbsServArea |  | MbsServiceArea |  | C |  | 0..1 |  | Represents the reduced MBS Service Area information that can be supported by a single MB-SMF. (NOTE 1) (NOTE 3) |  |  |  |
| reducedExtMbsServArea |  | ExternalMbsServiceArea |  | C |  | 0..1 |  | Represents the reduced external MBS Service Area information that can be supported by a single MB-SMF. (NOTE 1) (NOTE 4) |  |  |  |
| reducedMbsServAreas |  | array(MbsServiceArea) |  | C |  | 1..N |  | Represents the additional reduced MBS Service Areas. Each reduced MBS Service Area provided within each array element of this attribute can be supported by a single MB-SMF. (NOTE 2) (NOTE 4) |  |  |  |
| reducedExtMbsServAreas |  | ExternalMbsServiceArea |  | C |  | 1..N |  | Represents the additional reduced external MBS Service Areas. Each reduced MBS Service Area provided within each array element of this attribute can be supported by a single MB-SMF. (NOTE 2) (NOTE 3) |  |  |  |
| NOTE 1: These attributes are mutually exclusive. Either one of them shall be present. NOTE 2: These attributes are mutually exclusive. Either one of them may be present. NOTE 3: If the "reducedMbsServArea" attribute is present, then the "reducedExtMbsServAreas" attribute shall not be present. NOTE 4: If the "reducedExtMbsServArea" attribute is present, then the "reducedMbsServAreas" attribute shall not be present. |  |  |  |  |  |  |  |  |  |  |  |

#### 5.19.5.3	Simple data types and enumerations

##### 5.19.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.19.5.3.2	Simple data types

The simple data types defined in Table 5.19.5.3.2-1 shall be supported.

Table 5.19.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

#### 5.19.5.4	Data types describing alternative data types or combinations of data types

##### 5.19.5.4.1	Type: ProblemDetailsTmgiAlloc

Table 5.19.5.4.1-1: Definition of type ProblemDetailsTmgiAlloc as a list of to be combined data types

| Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- |
| ProblemDetails | 1 | Contains additional error related information. |  |
| ReducedMbsServArea | 0..1 | Contains the reduced MBS Service Area information. |  |

### 5.19.6	Used Features

The table below defines the features applicable to the MBSTMGI API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.19.6-1: Features used by MBSTMGI API

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_websocket | The delivery of notifications over Websocket is supported as described in 3GPP TS 29.122 [4]. This feature requires that the Notification_test_event feature is also supported. |
| 2 | Notification_test_event | The testing of notification connection is supported as described in 3GPP TS 29.122 [4]. |

### 5.19.7	Error handling

#### 5.19.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.19.7.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the MBSTMGI API.

#### 5.19.7.3	Application Errors

The application errors defined for the MBSTMGI API are listed in table 5.19.7.3-1.

Table 5.19.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| MBS_SERVICE_AREA_TOO_LARGE | 403 Forbidden | Indicates that the TMGI allocation request is rejected because the provided MBS Service Area cannot be supported by the network as it is too large to be served by a single MB-SMF. |
| MBS_SERVICE_AREA_NOT_SUPPORTED | 403 Forbidden | Indicates that the TMGI allocation request is rejected because the requested MBS Service Area is not supported (e.g., the MBS Service Area received from the AF cannot be covered by the service area(s) of any MB-SMF). |

## 5.20	MBSSession API

### 5.20.1	Introduction

The Nnef_MBSSession service shall use the MBSSession API.

The API URI of MBSSession API shall be:

- {apiRoot}/3gpp-mbs-session/v1

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-mbs-session".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.20.2	Resources

#### 5.20.2.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.20.2.1-1 and the resources and HTTP methods used for the MBSSession API.

Figure 5.20.2.1-1: Resource URI structure of the MBSSession API

Table 5.20.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.20.2.1-1: Resources and methods overview

| Resource name | Resource URI (relative path under API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| MBS Sessions | /mbs-sessions | POST | Create an MBS Session. |
| Individual MBS Session | /mbs-sessions/{mbsSessionRef} | PATCH | Modify an existing Individual MBS Session resource. |
|  |  | DELETE | Delete an existing Individual MBS Session resource. |
| MBS Session Subscriptions | /mbs-sessions/subscriptions | GET | Retrieve all the MBS Session Subscriptions |
|  |  | POST | Create an MBS Session Subscription. |
| Individual MBS Session Subscription | /mbs-sessions/subscriptions/{subscriptionId} | GET | Retrieve an existing Individual MBS Session Subscription resource. |
|  |  | DELETE | Delete an existing Individual MBS Session Subscription resource. |
| MBS Parameters Provisionings | /mbs-pp | GET | Retrieve all the MBS Parameters Provisioning resources. |
|  |  | POST | Request the creation of a new MBS Parameters Provisioning. |
| Individual MBS Parameters Provisioning | /mbs-pp/{mbsPpId} | GET | Retrieve an existing individual MBS Parameters Provisioning resource. |
|  |  | PUT | Request the update of an existing Individual MBS Parameters Provisioning resource. |
|  |  | PATCH | Request the modification of an existing Individual MBS Parameters Provisioning resource. |
|  |  | DELETE | Request the deletion of an existing Individual MBS Parameters Provisioning resource. |

#### 5.20.2.2	Resource: MBS sessions

##### 5.20.2.2.1	Introduction

This resource represents the collection of MBS sessions managed by the NEF.

This resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [3]).

##### 5.20.2.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-mbs-session/v1/mbs-sessions

This resource shall support the resource URI variables defined in table 5.20.2.2.2-1.

Table 5.20.2.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.20.1. |

##### 5.20.2.2.3	Resource Methods

###### 5.20.2.2.3.1	POST

This method enables an AF to request the creation of an MBS session resource at the NEF.

This method shall support the URI query parameters specified in table 5.20.2.2.3.1-1.

Table 5.20.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| N/A |  |  |  |  |  |

This method shall support the request data structures specified in table 5.20.2.2.3.1-2 and the response data structures and response codes specified in table 5.20.2.2.3.1-3.

Table 5.20.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MbsSessionCreateReq | M | 1 | Representation of the MBS session to be created at the NEF. |

Table 5.20.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MbsSessionCreateRsp | M | 1 | 201 Created | Successful case. A representation of the created Individual MBS Session resource is returned. The URI of the created resource shall be returned in an HTTP "Location" header. |
| ProblemDetails | O | 0..1 | 400 Bad Request | (NOTE 2) |
| ProblemDetailsTmgiAlloc | O | 0..1 | 403 Forbidden | (NOTE 2) |
| ProblemDetails | O | 0..1 | 404 Not Found | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure cases are described in clause 5.20.7. |  |  |  |  |

Table 5.20.2.2.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-mbs-session/v1/mbs-sessions/{mbsSessionRef} |

##### 5.20.2.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 5.20.2.3	Resource: Individual MBS Session

##### 5.20.2.3.1	Introduction

This resource represents an Individual MBS Session managed by the NEF.

This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [3]).

##### 5.20.2.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-mbs-session/v1/mbs-sessions/{mbsSessionRef}

This resource shall support the resource URI variables defined in table 5.20.2.3.2-1.

Table 5.20.2.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.20.1. |
| mbsSessionRef | string | Contains the identifier of the Individual MBS Session resource assigned by the NEF. |

##### 5.20.2.3.3	Resource Standard Methods

###### 5.20.2.3.3.1	PATCH

The HTTP PATCH method enables an AF to request the modification of an existing Individual MBS Session resource at the NEF.

This method shall support the URI query parameters specified in table 5.20.2.3.3.1-1.

Table 5.20.2.3.3.1-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.20.2.3.3.1-2 and the response data structures and response codes specified in table 5.20.2.3.3.1-3.

Table 5.20.2.3.3.1-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1 | Represents the list of modifications to be applied to the concerned existing Individual MBS Session resource, as specified in clause 4.6.1.1.3 of 3GPP TS 29.501 [32]. |

Table 5.20.2.3.3.1-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful response. The Individual MBS Session resource was successfully modified. |
| MbsSessionUpdateResp | M | 1 | 200 OK | Successful response. The Individual MBS Session was successfully updated and MBS Session update related information shall be returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| ProblemDetails | O | 0..1 | 400 Bad Request | (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| ProblemDetails | O | 0..1 | 404 Not Found | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure cases are described in clause 5.20.7. |  |  |  |  |

Table 5.20.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.20.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.20.2.3.3.3	DELETE

This method enables an AF to request the deletion of an Individual MBS Session resource at the NEF.

This method shall support the URI query parameters specified in table 5.20.2.3.3.3-1.

Table 5.20.2.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.20.2.3.3.3-2 and the response data structures and response codes specified in table 5.20.2.3.3.3-3.

Table 5.20.2.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.20.2.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful response. The Individual MBS Session resource was successfully deleted. |
|  |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
|  |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| ProblemDetails | O | 0..1 | 404 Not Found | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure cases are described in clause 5.20.7. |  |  |  |  |

Table 5.20.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.20.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

##### 5.20.2.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 5.20.2.4	Resource: MBS Session Subscriptions

##### 5.20.2.4.1	Introduction

This resource represents the collection of MBS Session Subscriptions managed by the NEF.

This resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [3]).

##### 5.20.2.4.2	Resource Definition

Resource URI: {apiRoot}/3gpp-mbs-session/v1/mbs-sessions/subscriptions

This resource shall support the resource URI variables defined in table 5.20.2.4.2-1.

Table 5.20.2.4.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.20.1. |

##### 5.20.2.4.3	Resource Methods

###### 5.20.2.4.3.1	GET

This method enables an AF to request to retrieve all the MBS Session Subscription resources managed by the NEF.

This method shall support the URI query parameters specified in table 5.20.2.4.3.1-1.

Table 5.20.2.4.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.20.2.4.3.1-2 and the response data structures and response codes specified in table 5.20.2.4.3.1-3.

Table 5.20.2.4.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.20.2.4.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(MbsSessionSubsc) | M | 0..N | 200 OK | Successful case. All the MBS Session Subscription resources managed by the NEF are returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status code for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.20.2.4.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI of the resource located in an alternative NEF. |

Table 5.20.2.4.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI of the resource located in an alternative NEF. |

###### 5.20.2.4.3.2	POST

This method enables an AF to request the creation of an MBS Session Subscription resource at the NEF.

This method shall support the URI query parameters specified in table 5.20.2.4.3.2-1.

Table 5.20.2.4.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.20.2.4.3.2-2 and the response data structures and response codes specified in table 5.20.2.4.3.2-3.

Table 5.20.2.4.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MbsSessionSubsc | M | 1 | Representation of the MBS session to be created at the NEF. |

Table 5.20.2.4.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MbsSessionSubsc | M | 1 | 201 Created | Successful case. A representation of the created Individual MBS Session Subscription resource is returned. The URI of the created resource is returned in an HTTP "Location" header. |
| ProblemDetails | O | 0..1 | 404 Not Found | (NOTE 2) |
| NOTE: The mandatory HTTP error status code for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.20.2.4.3.2-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-mbs-session/v1/mbs-sessions/subscriptions/{subscriptionId} |

##### 5.20.2.4.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 5.20.2.5	Resource: Individual MBS Session Subscription

##### 5.20.2.5.1	Introduction

This resource represents an Individual MBS Session Subscription managed by the NEF.

This resource is modelled with the Document resource archetype (see clause C.2 of 3GPP TS 29.501 [3]).

##### 5.20.2.5.2	Resource Definition

Resource URI: {apiRoot}/3gpp-mbs-session/v1/mbs-sessions/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 5.20.2.5.2-1.

Table 5.20.2.5.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.20.1. |
| subscriptionId | string | Identifier of the Individual MBS Session Subscription resource. |

##### 5.20.2.5.3	Resource Methods

###### 5.20.2.5.3.1	GET

This method enables an AF to request to retrieve an existing Individual MBS Session Subscription resource at the NEF.

This method shall support the URI query parameters specified in table 5.20.2.5.3.1-1.

Table 5.20.2.5.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.20.2.5.3.1-2 and the response data structures and response codes specified in table 5.20.2.5.3.1-3.

Table 5.20.2.5.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.20.2.5.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MbsSessionSubsc | M | 1 | 200 OK | Successful case. The requested Individual MBS Session Subscription resource is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status code for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.20.2.5.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI of the resource located in an alternative NEF. |

Table 5.20.2.5.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI of the resource located in an alternative NEF. |

###### 5.20.2.5.3.2	DELETE

This method enables an AF to request the deletion of an existing Individual MBS Session Subscription resource at the NEF.

This method shall support the URI query parameters specified in table 5.20.2.5.3.2-1.

Table 5.20.2.5.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.20.2.5.3.2-2 and the response data structures and response codes specified in table 5.20.2.5.3.2-3.

Table 5.20.2.5.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.20.2.5.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The concerned Individual MBS Session Subscription resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status code for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.20.2.5.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI of the resource located in an alternative NEF. |

Table 5.20.2.5.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI of the resource located in an alternative NEF. |

##### 5.20.2.5.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 5.20.2.6	Resource: MBS Parameters Provisionings

##### 5.20.2.6.1	Introduction

This resource represents the collection of MBS Parameters Provisionings managed by the NEF.

This resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [3]).

##### 5.20.2.6.2	Resource Definition

Resource URI: {apiRoot}/3gpp-mbs-session/v1/mbs-pp

This resource shall support the resource URI variables defined in table 5.20.2.6.2-1.

Table 5.20.2.6.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.20.1. |

##### 5.20.2.6.3	Resource Methods

###### 5.20.2.6.3.1	GET

This method enables an AF to request to retrieve all the MBS Parameters Provisionings resources managed by the NEF.

This method shall support the URI query parameters specified in table 5.20.2.6.3.1-1.

Table 5.20.2.6.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.20.2.6.3.1-2 and the response data structures and response codes specified in table 5.20.2.6.3.1-3.

Table 5.20.2.6.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.20.2.6.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(MbsPpData) | M | 1..N | 200 OK | Successful case. All the Individual MBS Parameters Provisionings resources managed by the NEF are returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status code for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.20.2.6.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI of the resource located in an alternative NEF. |

Table 5.20.2.6.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI of the resource located in an alternative NEF. |

###### 5.20.2.6.3.2	POST

This method enables an AF to request the creation of a new MBS Parameters Provisioning at the NEF.

This method shall support the URI query parameters specified in table 5.20.2.6.3.2-1.

Table 5.20.2.6.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.20.2.6.3.2-2 and the response data structures and response codes specified in table 5.20.2.6.3.2-3.

Table 5.20.2.6.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MbsPpData | M | 1 | Contain the representation of the new MBS Parameters Provisioning to be created at the NEF. |

Table 5.20.2.6.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MbsPpData | M | 1 | 201 Created | Successful case. A representation of the created Individual MBS Parameters Provisioning resource is returned. The URI of the created resource shall be returned in an HTTP "Location" header. |
| NOTE: The mandatory HTTP error status code for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.20.2.6.3.2-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-mbs-session/v1/mbs-pp/{mbsPpId} |

##### 5.20.2.6.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 5.20.2.7	Resource: Individual MBS Parameters Provisioning

##### 5.20.2.7.1	Introduction

This resource represents an Individual MBS Parameters Provisioning resource managed by the NEF.

This resource is modelled with the Document resource archetype (see clause C.2 of 3GPP TS 29.501 [3]).

##### 5.20.2.7.2	Resource Definition

Resource URI: {apiRoot}/3gpp-mbs-session/v1/mbs-pp/{mbsPpId}

This resource shall support the resource URI variables defined in table 5.20.2.7.2-1.

Table 5.20.2.7.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.20.1. |
| mbsPpId | string | Identifier of the Individual MBS Parameters Provisioning resource. |

##### 5.20.2.7.3	Resource Methods

###### 5.20.2.7.3.1	GET

This method enables an AF to request to retrieve an existing Individual MBS Parameters Provisioning resource managed by the NEF.

This method shall support the URI query parameters specified in table 5.20.2.7.3.1-1.

Table 5.20.2.7.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.20.2.7.3.1-2 and the response data structures and response codes specified in table 5.20.2.7.3.1-3.

Table 5.20.2.7.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.20.2.7.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MbsPpData | M | 1 | 200 OK | Successful case. The requested Individual MBS Parameters Provisioning resource is successfully returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status code for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.20.2.7.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI of the resource located in an alternative NEF. |

Table 5.20.2.7.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI of the resource located in an alternative NEF. |

###### 5.20.2.7.3.2	PUT

This method enables an AF to request the update of an existing Individual MBS Parameters Provisioning resource at the NEF.

This method shall support the URI query parameters specified in table 5.20.2.7.3.2-1.

Table 5.20.2.7.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.20.2.7.3.2-2 and the response data structures and response codes specified in table 5.20.2.7.3.2-3.

Table 5.20.2.7.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MbsPpData | M | 1 | Represents the updated Individual MBS Parameters Provisioning resource representation. |

Table 5.20.2.7.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MbsPpData | M | 1 | 200 OK | Successful response. The Individual MBS Parameters Provisioning resource is successfully updated and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful response. The Individual MBS Parameters Provisioning resource is successfully updated and no content is to be returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE 1: The mandatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.20.2.7.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.20.2.7.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.20.2.7.3.3	PATCH

This method enables an AF to request the modification of an existing Individual MBS Parameters Provisioning resource at the NEF.

This method shall support the URI query parameters specified in table 5.20.2.7.3.3-1.

Table 5.20.2.7.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.20.2.7.3.3-2 and the response data structures and response codes specified in table 5.20.2.7.3.3-3.

Table 5.20.2.7.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MbsPpDataPatch | M | 1 | Represents the requested modifications to the Individual MBS Parameters Provisioning resource. |

Table 5.20.2.7.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MbsPpData | M | 1 | 200 OK | Successful response. The Individual MBS Parameters Provisioning resource is successfully modified and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful response. The Individual MBS Parameters Provisioning resource is successfully modified and no content is to be returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE 1: The mandatory HTTP error status code for the PATCH method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.20.2.7.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.20.2.7.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.20.2.7.3.4	DELETE

This method enables an AF to request the deletion of an existing Individual MBS Parameters Provisioning resource at the NEF.

This method shall support the URI query parameters specified in table 5.20.2.7.3.4-1.

Table 5.20.2.7.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.20.2.7.3.4-2 and the response data structures and response codes specified in table 5.20.2.7.3.4-3.

Table 5.20.2.7.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.20.2.7.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The concerned Individual MBS Parameters Provisioning resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status code for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.20.2.7.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI of the resource located in an alternative NEF. |

Table 5.20.2.7.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI of the resource located in an alternative NEF. |

##### 5.20.2.7.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

### 5.20.3	Custom Operations without associated resources

There are no customoperations without associated resources defined for this API in this release of the specification.

### 5.20.4	Notifications

#### 5.20.4.1	General

Notifications shall comply to clause 5.2.5 of 3GPP TS 29.122 [4].

Table 5.20.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| MBS Session Status Notification | {notificationUri} | POST | Enables to notify a previously subscribed AF on MBS session status information. |

#### 5.20.4.2	MBS Session Status Notification

##### 5.20.4.2.1	Description

The MBS Session Status Notification is used by the NEF to report MBS session status information to a subscribed AF.

##### 5.20.4.2.2	Target URI

The Callback URI "{notificationUri}" shall be used with the callback URI variables defined in table 5.20.4.2.2-1.

Table 5.20.4.2.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notificationUri | Uri | Callback URIprovided by the AF during the creation/modification of the subscription. |

##### 5.20.4.2.3	Operation Definition

###### 5.20.4.2.3.1	Notification via HTTP POST

This method shall support the request data structures specified in table 5.20.4.2.3.1-1 and the response data structures and response codes specified in table 5.20.4.2.3.1-2.

Table 5.20.4.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MbsSessionStatusNotif | M | 1 | Represents the MBS Session Status information to be reported to the AF. |

Table 5.20.4.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The MBS Session Status notification is successfully received. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.20.4.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

Table 5.20.4.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

### 5.20.5	Data Model

#### 5.20.5.1	General

This clause specifies the application data model supported by the MBSSession API. Table 5.20.5.1-1 specifies the data types defined for the MBSSession API.

Table 5.20.5.1-1: MBSSession specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| MbsPpData | 5.20.5.2.6 | Represents MBS Parameters Provisioning data. |  |
| MbsPpDataPatch | 5.20.5.2.8 | Represents the requested modification to existing MBS Parameters Provisioning data. |  |
| MbsSessAuthData | 5.20.5.2.7 | Represents the MBS Session Authorization data. |  |
| MbsSessAssistInfo | 5.20.5.2.9 | Represents the MBS Session Assistance information data set(s). | 5MBS2 |
| MbsSessionCreateReq | 5.20.5.2.2 | Represents the parameters to request MBS Session creation. |  |
| MbsSessionCreateResp | 5.20.5.2.3 | Represents the parameters to be returned in an MBS Session creation response. |  |
| MbsSessionSubsc | 5.20.5.2.4 | Represents an MBS Session Subscription. |  |
| MbsSessionStatusNotif | 5.20.5.2.5 | Represents an MBS Session Status notification. |  |
| MbsSessionUpdateResp | 5.20.5.2.10 | Represents MBS Session update related information. | ReducedMbsServArea |

Table 5.20.5.1-2 specifies data types re-used by the MBSSession API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the MBSSession API.

Table 5.20.5.1-2: Re-used Data Types

| Data type |  | Reference |  | Comments |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5MbsAuthorizationInfo |  | 3GPP TS 29.503 [17] |  | Contains the MBS Session authorization information. |  |  |  |
| DateTime |  | 3GPP TS 29.122 [4] |  | Represents a date and a time |  |  |  |
| ExternalGroupId |  | 3GPP TS 29.122 [4] |  | Represents the External Group Identifier for a user group. |  |  |  |
| ExternalMbsServiceArea |  | 3GPP TS 29.571 [8] |  | Represents an external MBS service area. |  |  |  |
| Gpsi |  | 3GPP TS 29.571 [8] |  | Represents a GPSI. |  |  |  |
| MbsServiceArea |  | 3GPP TS 29.571 [8] |  | Represents an MBS service area. |  |  |  |
| MbsAssistanceInfo |  | 3GPP TS 29.503 [17] |  | Represents the MBS Session Assistance information. |  | 5MBS2 |  |
| MbsSession |  | 3GPP TS 29.571 [8] |  | Represents MBS session information. |  |  |  |
| MbsSessionId |  | 3GPP TS 29.571 [8] |  | Represents the identifier of an MBS Session. |  |  |  |
| MbsSessionEventReportList |  | 3GPP TS 29.571 [8] |  | Represents the list of MBS Session Event Report(a). |  |  |  |
| MbsSessionSubscription |  | 3GPP TS 29.571 [8] |  | Represents an MBS Session Subscription |  |  |  |
| MtcProviderInformation |  | 3GPP TS 29.571 [8] |  | Represents the MTC provider information. |  | enNB1 |  |
| PatchItem |  | 3GPP TS 29.571 [8] |  | Represents the requested modifications to a resource via the PATCH method. |  |  |  |
| ProblemDetails |  | 3GPP TS 29.122 [4] |  | Represents error related information. |  |  |  |
| ProblemDetailsTmgiAlloc |  | 5.19.5.4.1 |  | Represents an extension to the ProblemDetails data structure with additional information. |  |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [8] |  | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |  |  |
| Tmgi |  | 3GPP TS 29.571 [8] |  | Represents a TMGI. |  |  |  |
| TunnelAddress |  | 3GPP TS 29.571 [8] |  | Represents a Tunnel Address (UDP/IP). |  |  |  |
| Uri |  | 3GPP TS 29.122 [4] |  | Represents a URI. |  |  |  |

#### 5.20.5.2	Structured data types

##### 5.20.5.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 5.20.5.2.2	Type: MbsSessionCreateReq

Table 5.20.5.2.2-1: Definition of type MbsSessionCreateReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afId | string | M | 1 | Contains the identifier of the AF that is sending the request. |  |
| mbsSession | MbsSession | M | 1 | MBS session to be created. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features. This attribute shall be provided in the HTTP POST request to create a new MBS session, if feature negotiation needs to take place. |  |

##### 5.20.5.2.3	Type: MbsSessionCreateRsp

Table 5.20.5.2.3-1: Definition of type MbsSessionCreateRsp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsSession | MbsSession | M | 1 | Represents the created MBS session. |  |
| eventList | MbsSessionEventReportList | C | 0..1 | Contains a list of MBS Session Status Event(s) report(s), if available. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features. This attribute shall be provided in the HTTP POST response to a request to create a new MBS session, if it was present in the corresponding HTTP POST request. |  |

##### 5.20.5.2.4	Type: MbsSessionSubsc

Table 5.20.5.2.4-1: Definition of type MbsSessionSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afId | string | M | 1 | Contains the identifier of the AF that is sending the request. |  |
| subscription | MbsSessionSubscription | M | 1 | Represents the parameters of the MBS Session Status subscription to be created |  |
| subscriptionId | string | C | 0..1 | Contains the identifier of the created Individual MBS Session Subscription resource. This attribute shall only be present in the HTTP POST response to an MBS session creation request. |  |

##### 5.20.5.2.5	Type: MbsSessionStatusNotif

Table 5.20.5.2.5-1: Definition of type MbsSessionStatusNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventList | MbsSessionEventReportList | M | 1 | Contains the reported MBS session event(s) and the related information. |  |

##### 5.20.5.2.6	Type: MbsPpData

Table 5.20.5.2.6-1: Definition of type MbsPpData

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| afId |  | string |  | M |  | 1 |  | Contains the identifier of the AF that is sending the request. |  |  |  |
| mtcProviderId |  | MtcProviderInformation |  | O |  | 0..1 |  | Identifies the MTC Service Provider and/or MTC Application. |  | enNB1 |  |
| mbsSessAuthData |  | MbsSessAuthData |  | C |  | 0..1 |  | Contains the MBS Session Authorization data that the AF requests to provision. This attribute shall be present when the AF requests to provision MBS Session Authorization data. |  |  |  |
| mbsSessAssistInfo |  | MbsSessAssistInfo |  | C |  | 0..1 |  | Contains the MBS Session Assistance information data set(s) that the AF requests to provision. This attribute shall be present when the AF requests to provision one or several MBS Session Assistance information data set(s). |  | 5MBS2 |  |
| suppFeat |  | SupportedFeatures |  | C |  | 0..1 |  | Contains the list of supported features among the ones defined in clause 5.20.6. This attribute shall be present only when feature negotiation needs to take place. |  |  |  |

##### 5.20.5.2.7	Type: MbsSessAuthData

Table 5.20.5.2.7-1: Definition of type MbsSessAuthData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| extGroupId | ExternalGroupId | M | 1 | Represents the external group identifier of the targeted multicast MBS Group. |  |
| gpsisList | map(Gpsi) | C | 1..N | Represents the list of the GPSI(s) of the member UE(s) constituting the multicast MBS group. Any value of type string can be used as a key of the map. This attribute shall be present only if the multicast MBS group has not yet been created or the list of its member(s) needs to be updated. |  |
| mbsSessionIdList | 5MbsAuthorizationInfo | M | 1 | Contains the identifier(s) of the multicast MBS Session(s) that the multicast MBS group is authorized to join. |  |

##### 5.20.5.2.8	Type: MbsPpDataPatch

Table 5.20.5.2.8-1: Definition of type MbsPpDataPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsSessAuthData | MbsSessAuthData | O | 0..1 | Contains the updated MBS Session Authorization data that the AF requests to provision. |  |
| mbsSessAssistInfo | MbsSessAssistInfo | O | 0..1 | Contains the updated MBS Session Assistance information data set(s) that the AF requests to provision. | 5MBS2 |

##### 5.20.5.2.9	Type: MbsSessAssistInfo

Table 5.20.5.2.9-1: Definition of type MbsSessAssistInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsSessAssistData | array(MbsAssistanceInfo) | M | 1..N | Contains one or several MBS Session Assistance information data set(s) for one or several MBS Session(s). |  |

##### 5.20.5.2.10	Type: MbsSessionUpdateResp

Table 5.20.5.2.10-1: Definition of type MbsSessionUpdateResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| reducedMbsServArea | MbsServiceArea | C | 0..1 | This attribute shall be present only if the requested MBS Service Area cannot be covered by the MB-SMF Service Area of a single MB-SMF, and the MB-SMF reduced the MBS Service Area accordingly. When present, it shall contain the reduced MBS Service Area in which the MBS session has been updated. (NOTE) |  |
| reducedExtMbsServArea | ExternalMbsServiceArea | C | 0..1 | This attribute shall be present only if the requested MBS Service Area cannot be covered by the MB-SMF Service Area of a single MB-SMF, and the MB-SMF reduced the MBS Service Area accordingly. When present, it shall contain the reduced external MBS Service Area in which the MBS session has been updated. (NOTE) |  |
| NOTE: These attributes are mutually exclusive. Either one of them may be present. |  |  |  |  |  |

#### 5.20.5.3	Simple data types and enumerations

##### 5.20.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.20.5.3.2	Simple data types

The simple data types defined in table 5.20.5.3.2-1 shall be supported.

Table 5.20.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

### 5.20.6	Used Features

The table below defines the features applicable to the MBSSession API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.20.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | 5MBS2 | This feature indicates the support of the Rel-18 enhancements to the 5G Multicast/Broadcast services. The following functionalities are supported: - MBS Session Assistance information provisioning. - Support the provisioning of the Associated Session Identifier to enable 5MBS MOCN Network Sharing scenarios (e.g., MOCN with multiple broadcast MBS sessions transmitting the same content via different Core Networks). - NR RedCap UEs indication information provisioning during MBS Session creation/update. |
| 2 | ReducedMbsServArea | This feature indicates the support of the MBS Service Area reduction functionality. The following sub-functionalities are supported: - Support to return the reduced MBS Service Area in an MBS Session creation/update response. |
| 3 | enNB1 | This feature indicates the support of Rel-18 enhancements to this northbound API. The following sub-functionalities are supported: - Support the provisioning of the MTC provider ID. |

### 5.20.7	Error handling

#### 5.20.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.20.7.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the MBSSession API.

#### 5.20.7.3	Application Errors

The application errors defined for the MBSSession API are listed in table 5.20.7.3-1.

Table 5.20.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| INVALID_MBS_SERVICE_INFO | 400 Bad Request | The provided MBS Service Information is invalid (e.g. invalid QoS reference), incorrect or insufficient to perform MBS policy authorization. |
| FILTER_RESTRICTIONS_NOT_OBSERVED | 400 Bad Request | The MBS IP flow(s) description provided within the MBS Service Information cannot be handled due to the restrictions defined in clause 5.3.8 of 3GPP TS 29.214 [64] not being observed. |
| MBS_SERVICE_AREA_TOO_LARGE | 403 Forbidden | Indicates that the MBS Session creation request is rejected because the provided MBS Service Area cannot be supported by the network as it is too large to be served by a single MB-SMF. |
| MBS_SERVICE_AREA_NOT_SUPPORTED | 403 Forbidden | Indicates that the MBS Session Creation/Update is rejected because the requested MBS Service Area is not supported (e.g., the MBS Service Area received from the AF cannot be covered by the service area(s) of any MB-SMF). |
| MBS_SERVICE_INFO_NOT_AUTHORIZED | 403 Forbidden | The provided MBS Service Information is rejected. |
| MBS_SESSION_ALREADY_CREATED | 403 Forbidden | The requested MBS session has already been created at the NEF/MB-SMF. |
| OVERLAPPING_MBS_SERVICE_AREA | 403 Forbidden | The provided MBS service area overlaps with the MBS service area of an existing MBS Session that shares the same MBS session Identifier. |
| UNKNOWN_TMGI | 404 Not Found | The TMGI provided in the request does not exist. |
| MBS_SESSION_CONTEXT_NOT_FOUND | 404 Not Found | The targeted Individual MBS Session does not exist. |
| UNKNOWN_MBS_SERVICE_AREA | 404 Not Found | The requested MBS service area (e.g. identified by the Area Session ID) cannot be found. |

## 5.21	EASDeployment API

### 5.21.0	Introduction

The Nnef_EASDeployment service shall use the EASDeployment API.

The API URI of EASDeployment API shall be:

- {apiRoot}/3gpp-eas-deployment/v1

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-eas-deployment".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.21.1	Resources

#### 5.21.1.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.21.1.1-1 and the resources and HTTP methods used for AF provisioned EAS Deployment information management in the northbound EASDeployment API.

Figure 5.21.1.1-1: Resource URI structure of the northbound EASDeployment API

Table 5.21.1.1-1 provides an overview of the resources and HTTP methods applicable for the northbound EASDeployment API.

Table 5.21.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| EAS Deployment Information | /{afId}/eas-deployment-info | GET | Read all EAS Deployment information for a given AF. |
|  |  | POST | Create a new Individual EAS Deployment information resource. |
| Individual EAS Deployment Information | /{afId}/eas-deployment-info/{easDeployInfoId} | GET | Reads an active Individual EAS Deployment Information resource. |
|  |  | PUT | Update an existing Individual EAS Deployment Information resource. |
|  |  | DELETE | Deletes an existing Individual EAS Deployment Information resource. |

#### 5.21.1.2	Resource: EAS Deployment Information

##### 5.21.1.2.1	Introduction

This resource allows an AF to request the creation of a new Individual EAS Deployment Information resource.

##### 5.21.1.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-eas-deployment/v1/{afId}/eas-deployment-info

This resource shall support the resource URI variables defined in table 5.21.1.2.2-1.

Table 5.21.1.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |

##### 5.21.1.2.3	Resource Methods

###### 5.21.1.2.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.21.1.2.2.

###### 5.21.1.2.3.2	GET

The GET method allows to read all active EAS Deployment information for a given AF and subscription. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.21.1.2.3.2-1.

Table 5.21.1.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.21.1.2.3.2-2, the response data structures and response codes specified in table 5.21.1.2.3.2-3, and the location headers specified in table 5.21.1.2.3.2-4 and table 5.21.1.2.3.2-5.

Table 5.21.1.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.21.1.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(EasDeployInfo) | M | 0..N | 200 OK | The configuration information for the AF in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during EAS deployment information retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during EAS deployment information retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.21.1.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.21.1.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.21.1.2.3.3	POST

The POST method creates a new resource of Individual EAS Deployment Information for a given AF. The AF shall initiate the HTTP POST request message and the NEF shall respond to the message. The NEF shall construct the URI of the created resource.

This method shall support the request data structures specified in table 5.21.1.2.3.3-1, the response data structures and response codes specified in table 5.21.1.2.3.3-2, and the Location Headers specified in table 5.21.1.2.3.3-3.

Table 5.21.1.2.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EasDeployInfo | M | 1 | EAS Deployment Information, indicates how edge services are deployed in each Local DN. |

Table 5.21.1.2.3.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EasDeployInfo | M | 1 | 201 Created | The Individual EAS Deployment Information resource was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.21.1.2.3.3-3: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-eas-deployment/v1/{afId}/eas-deployment-info/{easDeployInfoId} |

#### 5.21.1.3	Resource: Individual EAS Deployment Information

##### 5.21.1.3.1	Introduction

This resource allows an AF to read, update or delete an existing Individual EAS Deployment Information.

##### 5.21.1.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-eas-deployment/v1/{afId}/eas-deployment-info/{easDeployInfoId}

This resource shall support the resource URI variables defined in table 5.21.1.3.2-1.

Table 5.21.1.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |
| easDeployInfoId | string | Identifier of the EAS Deployment Information formatted according to IETF RFC 3986 [44]. |

##### 5.21.1.3.3	Resource Methods

###### 5.21.1.3.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.21.1.3.2.

###### 5.21.1.3.3.2	GET

The GET method allows to read the existing EAS Deployment Information for a given AF and a given EAS Deployment Information Id. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.21.1.3.3.2-1.

Table 5.21.1.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.21.1.3.3.2-2, the response data structures and response codes specified in table 5.21.1.3.3.2-3, and the Location Headers specified in table 5.21.1.3.3.2-4 and table 5.21.1.3.3.2-5.

Table 5.21.1.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.21.1.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EasDeployInfo | M | 1 | 200 OK | Successful case. The exposure information of an existing Individual EAS Deployment Information in the request URI is returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during the EAS Deployment Information retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during the EAS Deployment Information retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.21.1.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.21.1.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.21.1.3.3.3	PUT

The PUT method is used to modify an existing Individual EAS Deployment Information resource. The AF shall initiate the HTTP PUT request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.21.1.3.3.3-1.

Table 5.21.1.3.3.3-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.21.1.3.3.3-2, the response data structures and response codes specified in table 5.21.1.3.3.3-3, and the Location Headers specified in table 5.21.1.3.3.3-4 and table 5.21.1.3.3.3-5.

Table 5.21.1.3.3.3-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EasDeployInfo | M | 1 | Modify the Individual EAS Deployment Information resource. |

Table 5.21.1.3.3.3-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EasDeployInfo | M | 1 | 200 OK | Successful case. The exposure information of the updated EAS Deployment Information. |
| N/A |  |  | 204 No Content | The EAS Deployment change was updated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during the EAS Deployment Information modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during the EAS Deployment Information modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.21.1.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.21.1.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.21.1.3.3.4	DELETE

The DELETE method deletes an existing Individual EAS Deployment Information resource. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.21.1.3.3.4-1.

Table 5.21.1.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.21.1.3.3.4-2, the response data structures and response codes specified in table 5.21.1.3.3.4-3, and the Location Headers specified in table 5.21.1.3.3.4-4 and table 5.21.1.3.3.4-5.

Table 5.21.1.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.21.1.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The Individual EAS Deployment Information resource was terminated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during the termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during the termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.21.1.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.21.1.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

### 5.21.2	Custom Operations without associated resources

#### 5.21.2.1	Overview

The structure of the custom operation URIs of the Nnef_EASDeployment service is shown in Figure 5.21.2.1-1.

Figure 5.21.2.1-1: Custom operation URI structure of the Nnef_EASDeployment API

Table 5.21.2.1-1 provides an overview of the custom operations and applicable HTTP methods.

Table 5.21.2.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| {apiRoot}/3gpp-eas-deployment/<apiVersion>/remove-edis | POST | Request the NEF to delete EAS Deployment Information based on given attributes. |

#### 5.21.2.2	Operation: remove-edis

##### 5.21.2.2.1	Description

The operation is used by the NF service consumer to delete EAS Deployment Information based on given attributes.

##### 5.21.2.2.2	Operation Definition

This operation shall support the request data structures shown in Table 5.21.2.2.2-1 and the response data structures and error codes specified in Table 5.21.2.2.2-2.

Table 5.21.2.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EdiDeleteCriteria | M | 1 | Information about the criteria to be used for EAS Deployment Information deletion. |

Table 5.21.2.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful request to remove EAS Deployment Information based on given criteria. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

### 5.21.3	Notifications

There are no notifications defined for this API in this release of the specification.

### 5.21.4	Data Model

#### 5.21.4.1	General

This clause specifies the application data model supported by the EASDeployment API. Table 5.21.4.1-1 specifies the data types defined for the EASDeployment API.

Table 5.21.4.1-1: EASDeployment API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| EasDeployInfo | 5.21.4.3.2 | EAS Deployment Information, indicates how edge services are deployed in each Local DN. |  |
| EdiDeleteCriteria | 5.21.4.3.5 | Contains criteria for deleting EAS Deployment Information. |  |
| DnaiInformation | 5.21.4.3.3 | list of DNS server identifier and/or IP address(s) of the EAS in the local DN for the DNAI. |  |
| DnsServerIdentifier | 5.21.4.3.4 | DNS server identifier (consisting of IP address and port). |  |

#### 5.21.4.2	Reused data types

The data types reused by the EASDeployment API from other specifications are listed in table 5.21.4.2-1.

Table 5.21.4.2-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| AfId | 5.14.5.4.2 | Represents an AF identifier. |
| Dnai | 3GPP TS 29.571 [8] | Identifies a DNAI. |
| Dnn | 3GPP TS 29.571 [8] | Identifies a DNN. |
| DnnSnssaiInformation | 5.18.3.3.5 | Contains DNN and S-NSSAI information. |
| ExternalGroupId | 3GPP TS 29.122 [4] | External Group Identifier for a user group. |
| FqdnPatternMatchingRule | 3GPP TS 29.571 [8] | Identifies the FQDN pattern matching rule. |
| IpAddr | 3GPP TS 29.571 [8] | IP Address. |
| Snssai | 3GPP TS 29.571 [8] | Identifies the S-NSSAI. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |
| Uinteger | 3GPP TS 29.571 [8] | Unsigned integer. |
| Link | 3GPP TS 29.122 [4] | Identifies a referenced resource. |

#### 5.21.4.3	Structured data types

##### 5.21.4.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.21.4.3.2	Type: EasDeployInfo

Table 5.21.4.3.2-1: Definition of type EasDeployInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| self | Link | C | 0..1 | Identifies the individual EAS Deployment information resource. Shall be present in the HTTP GET response when reading all the configurations for an AF. |  |
| afServiceId | string | O | 0..1 | Identifies a service on behalf of which the AF is issuing the request. |  |
| fqdnPatternList | array(FqdnPatternMatchingRule) | M | 1..N | Contains the supported FQDN pattern(s) for application(s) deployed in the Local part of the DN where each FQDN pattern is described by a FQDN Pattern Matching Rule. |  |
| appId | string | O | 0..1 | Identifies the application for which the EAS Deployment Information corresponds to. |  |
| dnn | Dnn | O | 0..1 | Contains the DNN for the EAS Deployment Information. |  |
| snssai | Snssai | O | 0..1 | Contains the S-NSSAI for the EAS Deployment Information. |  |
| exterGroupId | ExternalGroupId | O | 0..1 | Contains the external Group ID for the EAS Deployment Information. |  |
| dnaiInfos | map(DnaiInformation) | O | 1..N | Contains the list of DNS server identifier (consisting of IP address and port) and/or IP address(s) of the EAS in the local DN for each DNAI. The key of map is the DNAI. |  |
| targetAfId | string | O | 0..1 | Identifier of the AF that is responsible for the EAS associated with this EAS deployment information. | EasRelocationEnh |
| suppFeat | SupportedFeatures | C | 0..1 | Indicates the list of Supported features used as described in clause 5.21.5. |  |

##### 5.21.4.3.3	Type: DnaiInformation

Table 5.21.4.3.3-1: Definition of type DnaiInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dnai | Dnai | M | 1 | Identifer the DNAI. |  |
| dnsServIds | array(DnsServerIdentifier) | C | 1..N | list of DNS server identifier (consisting of IP address and port) for each DNAI. |  |
| easIpAddrs | array(IpAddr) | C | 1..N | IP address(s) of the EASs in the local DN for each DNAI. |  |
| NOTE: At least one of the "dnsServIds" or "easIpAddrs" attribute shall be provided. |  |  |  |  |  |

##### 5.21.4.3.4	Type: DnsServerIdentifier

Table 5.21.4.3.4-1: Definition of type DnsServerIdentifier

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dnsServIpAddr | IpAddr | M | 1 | DNS server IP address. |  |
| portNumber | Uinteger | M | 1 | DNS port number. |  |

##### 5.21.4.3.5	Type: EdiDeleteCriteria

Table 5.21.4.3.5-1: Definition of type EdiDeleteCriteria

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afId | AfId | C | 0..1 | AF identifier to be used as deletion criterion. (NOTE) |  |
| dnnSnssai | DnnSnssaiInformation | C | 0..1 | DNN and S-NSSAI information to be used as deletion criterion. (NOTE) |  |
| NOTE: At least one of those attributes shall be provided. |  |  |  |  |  |

#### 5.21.4.4	Simple data types and enumerations

##### 5.21.4.4.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.21.4.4.2	Simple data types

The simple data types defined in table 5.21.4.4.2-1 shall be supported.

Table 5.21.4.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

### 5.21.5	Used Features

The table below defines the features applicable to the EASDeployment API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.21.5-1: Features used by EASDeployment API

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | EasRelocationEnh | This feature indicates enhanced support of EAS relocation procedures via additional information about the AFs that are responsible for certain EAS. |
| 2 | EasDnaiConsistency | This feature indicates support of consistency checking between AF-provisioned and operator-configured data related to EAS Deployment. |

### 5.21.6	Error handling

#### 5.21.6.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.21.6.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the EASDeployment API.

#### 5.21.6.3	Application Errors

The application errors defined for EASDeployment API are listed in table 5.21.6.3-1.

Table 5.21.6.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |
| CONFLICT_CONFIG_DATA | 403 Forbidden | The provided EAS Deployment Information is in conflict with the OAM-configured information. | EasDnaiConsistency |

## 5.22	ASTI API

### 5.22.0	Introduction

The Nnef_ASTI service shall use the ASTI API.

The API URI of ASTI API shall be:

- {apiRoot}/3gpp-asti/v1

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-asti".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.22.1	Resources

#### 5.22.1.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.22.1.1-1 and the resources and HTTP methods used for the ASTI API.

Figure 5.22.1.1-1: Resource URI structure of the ASTI API

Table 5.22.1.1-1 provides an overview of the resources and HTTP methods applicable for the ASTI API.

Table 5.22.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| ASTI Configurations | /{afId}/configurations | GET | Read all configurations of 5G access stratum time distribution for a given AF and subscription. |
|  |  | POST | Create a new configuration of 5G access stratum time distribution. |
|  | /{afId}/configurations/retrieve | retrieve (POST) | Retrieval of the status of access stratum time distribution. |
| Individual ASTI Configuration | /{afId}/configurations/{configId} | PUT | Modify a configuration configuration of 5G access stratum time distribution. |
|  |  | DELETE | Delete a configuration of 5G access stratum time distribution. |
|  |  | GET | Query the status of the access time distribution |

#### 5.22.1.2	Resource: ASTI Configurations

##### 5.22.1.2.1	Introduction

This resource allows an AF to read all active configurations of 5G access stratum time distribution for a given AF, or allows an AF to create a new new configuration of 5G access stratum time distribution.

##### 5.22.1.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-asti/v1/{afId}/configurations

This resource shall support the resource URI variables defined in table 5.22.1.2.2-1.

Table 5.22.1.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |

##### 5.22.1.2.3	Resource Methods

###### 5.22.1.2.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.22.1.2.2.

###### 5.22.1.2.3.2	GET

The GET method allows to read all active configurations of 5G access stratum time distribution for a given AF and subscription. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.22.1.2.3.2-1.

Table 5.22.1.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.22.1.2.3.2-2 and the response data structures and response codes specified in table 5.22.1.2.3.2-3.

Table 5.22.1.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.22.1.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(AccessTimeDistributionData) | M | 0..N | 200 OK | The configuration information for the AF are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during configuration retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during configuration retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.22.1.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.22.1.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.22.1.2.3.3	POST

The POST method creates a new configuration of 5G access stratum time distribution for a given AF. The AF shall initiate the HTTP POST request message and the NEF shall respond to the message. The NEF shall construct the URI of the created resource.

This method shall support the request data structures specified in table 5.22.1.2.3.3-1 and the response data structures and response codes specified in table 5.22.1.2.3.3-2.

Table 5.22.1.2.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AccessTimeDistributionData | M | 1 | Parameters to create a configuration of 5G access stratum time distribution. |

Table 5.22.1.2.3.3-2: Data structures supported by the POST Response Body on this resource

| Data type |  | P |  | Cardinality |  | Response codes |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AccessTimeDistributionData |  | M |  | 1 |  | 201 Created |  | The configuration was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |  |
| ProblemDetails |  | O |  | 0..1 |  | 403 Forbidden |  | (NOTE 2) |  |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure causes are described in clause 5.22.6. |  |  |  |  |  |  |  |  |  |

Table 5.22.1.2.3.3-3: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-asti/v1/{afId}/configurations/{configId} |

##### 5.22.1.2.4	Resource Custom Operations

###### 5.22.1.2.4.1	Overview

Table 5.22.1.2.4.1-1: Custom operations

| Operation Name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| retrieve | /configurations/retrieve | retrieve (POST) | Request the status of the 5G access stratum time distribution for a list of UEs. |

###### 5.22.1.2.4.2	Operation: retrieve

5.22.1.2.4.2.1	Description

This custom operation retrieves the status of the access stratum time distribution for a list of UEs.

5.22.1.2.4.2.2	Operation Definition

This operation shall support the request data structures specified in table 5.22.1.2.4.2.2-1 and the response data structure and response codes specified in table 5.22.1.2.4.2.2-2.

Table 5.22.1.2.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| StatusRequestData | M | 1 | Parameters to be sent by the NF service consumer when the status of the 5G access stratum time distribution for a list of UEs is requested. |

Table 5.22.1.2.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| StatusResponseData | M | 1 | 200 OK | Status of the 5G access stratum time distribution for a list of UEs is returned. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

#### 5.22.1.3	Resource: Individual ASTI Configuration

##### 5.22.1.3.1	Introduction

This resource allows an AF to read/modify/cancel a configuration of 5G access stratum time distribution with the NEF.

##### 5.22.1.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-asti/v1/{afId}/configurations/{configId}

This resource shall support the resource URI variables defined in table 5.22.1.3.2-1.

Table 5.22.1.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Clause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |
| configId | string | Identifier of the configuration resource. |

##### 5.22.1.3.3	Resource Methods

###### 5.22.1.3.3.1	General

The following clauses specify the resource methods supported by the resource as described in clause 5.22.1.3.2.

###### 5.22.1.3.3.2	GET

The GET method allows to read the active configuration for a given AF and configuration Id. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.22.1.3.3.2-1.

Table 5.22.1.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.22.1.3.3.2-2 and the response data structures and response codes specified in table 5.22.1.3.3.2-3.

Table 5.22.1.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.22.1.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AccessTimeDistributionData | M | 1 | 200 OK | The configuration information for the AF in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during configuration retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during configuration retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.22.1.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.22.1.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.22.1.3.3.3	PUT

The PUT method modifies an existing configuration resource to update a configuration. The AF shall initiate the HTTP PUT request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.22.1.3.3.3-1 and the response data structures and response codes specified in table 5.22.1.3.3.3-2.

Table 5.22.1.3.3.3-1: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AccessTimeDistributionData | M | 1 | Modify an existing configuration of 5G access stratum time distribution. |

Table 5.22.1.3.3.3-2: Data structures supported by the PUT Response Body on this resource

| Data type |  | P |  | Cardinality |  | Response codes |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AccessTimeDistributionData |  | M |  | 1 |  | 200 OK |  | The configuration was updated successfully. |  |
| N/A |  |  |  |  |  | 204 No Content |  | The configuration was updated successfully. |  |
| N/A |  |  |  |  |  | 307 Temporary Redirect |  | Temporary redirection, during configuration modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| N/A |  |  |  |  |  | 308 Permanent Redirect |  | Permanent redirection, during configuration modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| ProblemDetails |  | O |  | 0..1 |  | 403 Forbidden |  | (NOTE 2) |  |
| NOTE 1: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure causes are described in clause 5.22.6. |  |  |  |  |  |  |  |  |  |

Table 5.22.1.3.3.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.22.1.3.3.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.22.1.3.3.4	DELETE

The DELETE method deletes the configuration of 5G access stratum time distribution for a given AF. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.22.1.3.3.4-1.

Table 5.22.1.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.22.1.3.3.4-2 and the response data structures and response codes specified in table 5.22.1.3.3.4-3.

Table 5.22.1.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.22.1.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The configuration was terminated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during configuration termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during configuration termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.22.1.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.22.1.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

### 5.22.2	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.22.3	Notifications

#### 5.22.3.1	General

Notifications shall comply to clause 5.2.5 of 3GPP TS 29.122 [4].

Table 5.22.3.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| ASTI Notification | {astiNotifUri} | POST | ASTI notification. |

#### 5.22.3.2	ASTI Notification

##### 5.22.3.2.1	Description

The ASTI Notification is used by the NEF to report the changes on the ASTI service to a previously subscribed AF.

##### 5.22.3.2.2	Target URI

The Callback URI "{astiNotifUri}" shall be used with the callback URI variables defined in table 5.22.3.2.2-1.

Table 5.22.3.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| astiNotifUri | Callback reference provided by the AF during the creation/update/modification of the corresponding Individual ASTI Configuration. |

##### 5.22.3.2.3	Operation Definition

###### 5.22.3.2.3.1	Notification via HTTP POST

This method shall support the request data structures specified in table 5.22.3.2.3.1-1 and the response data structures and response codes specified in table 5.22.3.2.3.1-2.

Table 5.22.3.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AstiConfigNotification | M | 1 | Provides information about the change in the 5G Access Stratum Time Distribution configuration notification by the NEF to the AF. |

Table 5.22.3.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The event notification is successfully received. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.22.3.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

Table 5.22.3.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

###### 5.22.3.2.3.2	Void

### 5.22.4	Data Model

#### 5.22.4.1	General

This clause specifies the application data model supported by the ASTI API.

Table 5.22.4.1-1 specifies the data types defined for the ASTI API.

Table 5.22.4.1-1: ASTI API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AccessTimeDistributionData | 5.22.4.3.2 | Contains the parameters for the creation of 5G access stratum time distribution configuration and for the subscription of 5G access stratum time distribution status. |  |
| ActiveUe | 5.22.4.3.5 | Contains the UE identifier whose status of the access stratum time distribution is active and the optional requested time synchronization error budget. |  |
| AstiConfigNotification | 5.22.4.3.6 | Contains the report of a change in the 5G Access Stratum Time Distribution parameters applied to the UE(s). | ASTIConfigReport |
| AstiConfigStateNotification | 5.22.4.3.7 | Contains the report about a change in the 5G Access Stratum Time Distribution parameters for a UE | ASTIConfigReport |
| StatusRequestData | 5.22.4.3.3 | Contains the parameters for retrieval of the status of the access stratum time distribution for a list of UEs. |  |
| StatusResponseData | 5.22.4.3.4 | Contains the parameters for the status of the access stratum time distribution for a list of UEs. |  |

#### 5.22.4.2	Reused data types

The data types reused by the ASTI API from other specifications are listed in table 5.22.4.2-1.

Table 5.22.4.2-1: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AstiEvent | 3GPP TS 29.565 [50] | Contains the ASTI Event. | ASTIConfigReport |
| AfAsTimeDistributionParam | 3GPP TS 29.565 [50] | Contains the 5G AF requested access stratum time distribution parameters. |  |
| ExternalGroupId | 3GPP TS 29.122 [4] | External Group Identifier for a user group. |  |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |  |
| ProblemDetails | 3GPP TS 29.122 [4] | Represents error related information. |  |
| SpatialValidityCond | 3GPP TS 29.571 [8] | Represents the Spatial Validity Condition. | CoverageArea |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.22.5-1. |  |
| Uri | 3GPP TS 29.571 [8] | Identifies a referenced resource. | ASTIConfigReport |

#### 5.22.4.3	Structured data types

##### 5.22.4.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.22.4.3.2	Type: AccessTimeDistributionData

Table 5.22.4.3.2-1: Definition of type AccessTimeDistributionData

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| gpsis |  | array(Gpsi) |  | C |  | 1..N |  | Identifies a list of UE(s). (NOTE 1) |  |  |  |
| exterGroupId |  | ExternalGroupId |  | C |  | 0.1 |  | Represents a group of users. (NOTE 1) |  |  |  |
| asTimeDisParam |  | AfAsTimeDistributionParam |  | M |  | 1 |  | 5G access stratum time distribution parameters. (NOTE 3) |  |  |  |
| coverageArea |  | SpatialValidityCond |  | O |  | 0..1 |  | Time Synchronization Coverage Area. (NOTE 2) |  | CoverageArea |  |
| astiNotifUri |  | Uri |  | C |  | 0..1 |  | Notification URI for reporting changes in 5G access stratum time distribution status, and/or reporting the 5G access stratum time distribution information. It shall be provided if the feature ASTIConfigReport is supported. |  | ASTIConfigReport |  |
| astiNotifId |  | string |  | C |  | 0..1 |  | Notification Correlation ID assigned by the NF service consumer. It shall be provided if the ASTIConfigReport feature is supported. |  | ASTIConfigReport |  |
| suppFeat |  | SupportedFeatures |  | C |  | 0..1 |  | Represents the features supported by the NF service consumer. This parameter shall be supplied by the NF service consumer in the POST request and the response that requested the creation of an Individual ASTI Configuration resource. |  |  |  |
| NOTE 1: One of "gpsis" or "exterGroupId" attribute shall be provided. NOTE 2: The "trackingAreaList" attribute within the "coverageArea" attribute is not applicable for the untrusted AF. NOTE 3: The "clkQltDetLvl" attribute and the "clkQltAcptCri" attribute (if applicable) within the "asTimeDisParam" attribute may be provided only if the "NetTimeSyncStatus" feature is supported. |  |  |  |  |  |  |  |  |  |  |  |

##### 5.22.4.3.3	Type: StatusRequestData

Table 5.22.4.3.3-1: Definition of type StatusRequestData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| gpsis | array(Gpsi) | M | 1..N | Identifies a list of UE(s). |  |

##### 5.22.4.3.4	Type: StatusResponseData

Table 5.22.4.3.4-1: Definition of type StatusResponseData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| inactiveUes | array(Gpsi) | O | 1..N | Indicate the UE(s) whose status of the access stratum time distribution is inactive. |  |
| activeUes | array(ActiveUe) | O | 1..N | Contains the UE identifier(s) whose status of the access stratum time distribution is active and the optional requested time synchronization error budget. |  |

##### 5.22.4.3.5	Type: ActiveUe

Table 5.22.4.3.5-1: Definition of type ActiveUe

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| gpsi | Gpsi | M | 1 | Indicate the UE whose status of the access stratum time distribution is active. |  |
| timeSyncErrBdgt | Uinteger | O | 0..1 | Indicates the time synchronization error budget in terms of time units of nanoseconds. |  |

##### 5.22.4.3.6	Type AstiConfigNotification

Table 5.22.4.3.6-1: Definition of type AstiConfigNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| astiNotifId | string | M | 1 | It is used to set the value of Notification Correlation ID in the corresponding notification. |  |
| stateConfigs | array(AstiConfigStateNotification) | M | 1..N | Contains change of state of 5G access stratum time distribution configuration. |  |

##### 5.22.4.3.7	Type AstiConfigStateNotification

Table 5.22.4.3.7-1: Definition of type AstiConfigStateNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| gpsi | Gpsi | M | 1 | Identifies the UE to which the status below apply. |  |
| event | AstiEvent | M | 1 | Indicates the reported event. |  |

#### 5.22.4.4	Simple data types and enumerations

##### 5.22.4.4.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.22.4.4.2	Simple data types

The simple data types defined in table 5.22.4.4.2-1 shall be supported.

Table 5.22.4.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

### 5.22.5	Used Features

The table below defines the features applicable to the ASTI API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.22.5-1: Features used by ASTI API

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | CoverageArea | Indicates support of the inclusion of the Time Synchronization Coverage Area in ASTI requests. |
| 2 | ASTIConfigReport | This feature indicates the support of the report of changes in 5G access stratum time distribution status. |
| 3 | NetTimeSyncStatus | This feature indicates the support of network timing synchronization status and reporting. This feature requires the support of the "ASTIConfigReport" feature. |
| 4 | SupportReport | This feature indicates the support of the report of whether the AF requested Time Synchronization is authorized for the requested UE's. |

### 5.22.6	Error handling

#### 5.22.6.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.22.6.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the ASTI API.

#### 5.22.6.3	Application Errors

The application errors defined for the ASTI API are listed in table 5.22.6.3-1.

Table 5.22.6.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |
| UE_SERVICE_NOT_AUTHORIZED | 403 Forbidden | The AF request is not authorized or the AF requested parameter(s) are not authorized by UE's Time Synchronization Data in UDM. | SupportReport |

## 5.23	DataReporting API

### 5.23.1	Introduction

The Nnef_DataReporting service shall use the DataReporting API.

The API URI of DataReporting API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [4], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-data-reporting".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [4].

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.23.2	Resources

#### 5.23.2.1	Overview

This clause describes the structure for the Resource URIs as shown in Figure 5.23.2.1-1 and the resources and HTTP methods used for the DataReporting API.

The structure of the resource URIs of the DataReporting API is shown in Figure 5.23.2.1-1.

Figure 5.23.2.1-1: Resource URI structure of the DataReporting API

Table 5.23.2.1-1 provides an overview of the resources and applicable HTTP methods applicable for the DataReporting API.

Table 5.23.2.1-1: Resources and methods overview

| Resource name | Resource URI (relative path under API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Data Reporting Sessions | /sessions | POST | Create a Data Reporting Session |
| Individual Data Reporting Session | /sessions/{sessionId} | GET | Retreive an existing Individual Data Reporting Session resource. |
|  |  | PUT | Update an Individual existing Data Reporting Session resource. |
|  |  | DELETE | Delete an existing Individual Data Reporting Session resource. |
|  |  | report (POST) | Report data. |

#### 5.23.2.2	Resource: Data Reporting Sessions

##### 5.23.2.2.1	Introduction

This resource represents the collection of Data Reporting Sessions managed by the NEF.

This resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [3]).

##### 5.23.2.2.2	Resource definition

Resource URI: {apiRoot}/3gpp-data-reporting/<apiVersion>/sessions

This resource shall support the resource URI variables defined in table 5.23.2.2.2-1.

Table 5.23.2.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.23.1. |

##### 5.23.2.2.3	Resource Methods

###### 5.23.2.2.3.1	POST

This method enables an AF to request the creation of a Data Reporting Session at the NEF.

This method shall support the URI query parameters specified in table 5.23.2.2.3.1-1.

Table 5.23.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.23.2.2.3.1-2 and the response data structures and response codes specified in table 5.23.2.2.3.1-4.

Table 5.23.2.2.3.1-2: Data structures supported by the POST request body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataReportingSession | M | 1 | Representation of the Data Reporting Session to be created in the NEF. (NOTE) |
| NOTE: The "sessionId" attribute of the DataReportingSession data type shall not be provided as it is not applicable. |  |  |  |

Table 5.23.2.2.3.1-3: Data structures supported by the POST response body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataReportingSession | M | 1 | 201 Created | Successful case. A representation of the created "Individual Data Reporting Session" resource is returned. An HTTP "Location" header that contains the URI of the created resource shall be returned in the response body. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: The "sessionId" attribute of the DataReportingSession data type shall not be provided as it is not applicable. |  |  |  |  |

Table 5.23.2.2.3.1-4: Headers supported by the 201 response code on this resource

| HTTP response header | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | The URI of the newly created resource, according to the structure: {apiRoot}/3gpp-data-reporting/<apiVersion>/sessions/{sessionId} |

#### 5.23.2.3	Resource: Individual Data Reporting Session

##### 5.23.2.3.1	Introduction

This resource represents an Individual Data Reporting Session managed by the NEF.

This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [3]).

##### 5.23.2.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-data-reporting/<apiVersion>/sessions/{sessionId}

This resource shall support the resource URI variables defined in table 5.23.2.3.2-1.

Table 5.23.2.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.23.1. |
| sessionId | string | Represents the identifier of the "Individual Data Reporting Session" resource. |

##### 5.23.2.3.3	Resource standard methods

###### 5.23.2.3.3.1	GET

This method enables an AF to retrieve an existing "Individual Data Reporting Session" resource at the NEF.

This method shall support the URI query parameters specified in table 5.23.2.3.3.1-1.

Table 5.23.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.23.2.3.3.1-2 and the response data structures and response codes specified in table 5.23.2.3.3.1-3.

Table 5.23.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.23.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataReportingSession | M | 1 | 200 OK | Successful case. The requested "Individual Data Reporting Session" resource is returned to the AF. (NOTE 2) |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE 1: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: The "sessionId" attribute of the DataReportingSession data type shall not be provided as it is not applicable. |  |  |  |  |

Table 5.23.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative NEF. |

Table 5.23.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.23.2.3.3.2	PUT

This method enables an AF to request the update of an existing "Individual Data Reporting Session" resource at the NEF.

This method shall support the URI query parameters specified in table 5.23.2.3.3.2-1.

Table 5.23.2.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 5.23.2.3.3.2-2 and the response data structures and response codes specified in table 5.23.2.3.3.2-3.

Table 5.23.2.3.3.2-2: Data structures supported by the PUT request body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataReportingSession | M | 1 | Parameters to update the "Individual Data Reporting Session" resource. |
| NOTE: The "sessionId" attribute of the DataReportingSession data type shall not be provided as it is not applicable. |  |  |  |

Table 5.23.2.3.3.2-3: Data structures supported by the PUT response body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataReportingSession | M | 1 | 200 OK | Successful response. The "Individual Data Reporting Session" resource was updated successfully and a representation of the created resource is returned in the response body. (NOTE 2) |
| n/a |  |  | 204 No Content | Successful response. The "Individual Data Reporting Session" resource was successfully updated and no content is to be returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE 1: The mandatory HTTP error status codes for the HTTP PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: The "sessionId" attribute of the DataReportingSession data type shall not be provided as it is not applicable. |  |  |  |  |

Table 5.23.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.23.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.23.2.3.3.3	DELETE

This method enables an AF to request the deletion of an existing "Individual Data Reporting Session" resource at the NEF.

This method shall support the URI query parameters specified in table 5.23.2.3.3.3-1.

Table 5.23.2.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.23.2.3.3.3-2 and the response data structures and response codes specified in table 5.23.2.3.3.3-3.

Table 5.23.2.3.3.3-2: Data structures supported by the DELETE request body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.23.2.3.3.3-3: Data structures supported by the DELETE response body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful response. The "Individual Data Reporting Session" resource is successfully deleted. |
|  |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
|  |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.23.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.23.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

##### 5.23.2.3.4	Resource custom operations

5.23.2.3.4.1	Overview

Table 5.23.2.3.4.1-1: Custom operations

| Operation name | Custom operaration URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Report | /sessions/{sessionId}/report | POST | Enables to send collected UE data reports. |

5.23.2.3.4.2	Operation: Report

5.23.2.3.4.2.1	Description

The custom operation enables an AF to send collected UE data reports to the NEF.

5.23.2.3.4.2.2	Operation Definition

This operation shall support the request data structures specified in table 5.23.2.3.4.2.2-1 and the response data structures and response codes specified in table 5.23.2.3.4.2.2-2.

Table 5.23.2.3.4.2.2-1: Data structures supported by the POST request body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataReport | M | 1 | UE data reported by the data collection client. |

Table 5.23.2.3.4.2.2-2: Data structures supported by the POST response body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataReportingSession | O | 0..1 | 200 OK | The UE data report was successfully received. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4] |
| NOTE 1: The mandatory HTTP error status code for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: The “sessionId” attribute of the DataReportingSession data type shall not be provided as it is not applicable. |  |  |  |  |

Table 5.23.2.3.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.23.2.3.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

### 5.23.3	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.23.4	Notifications

There are no notifications defined for this API in this release of the specification.

### 5.23.5	Data Model

#### 5.23.3.1	General

This clause specifies the application data model supported by the DataReporting API. Table 5.23.5.1-1 specifies the data types defined for the DataReporting API.

Table 5.23.5.1-1: DataReporting specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| n/a |  |  |

Table 5.23.5.1-2 specifies data types re-used by the DataReporting API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the DataReporting API.

Table 5.23.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DataReport | 3GPP TS 26.532 [60] | Reported data by the AF. |  |
| DataReportingSession | 3GPP TS 26.532 [60] | Configuration by the AF specifying the data to be reported. |  |

### 5.23.6	Used Features

The table below defines the features applicable to the DataReporting API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.23.6-1: Features used by DataReporting API

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 5.23.7	Error handling

#### 5.23.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.23.7.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the DataReporting API.

#### 5.23.7.3	Application Errors

The application errors defined for the DataReporting API are listed in table 5.23.7.3-1.

Table 5.23.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

## 5.24	DataReportingProvisioning API

### 5.24.1	Introduction

The Nnef_DataReportingProvisioning service shall use the DataReportingProvisioning API.

The API URI of the DataReportingProvisioning API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [4], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-data-reporting-provisioning".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [4].

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.24.2	Resources

#### 5.24.2.1	Overview

This clause describes the structure for the Resource URIs as shown in Figure 5.24.2.1-1 and the resources and HTTP methods used for the DataReportingProvisioning API.

The structure of the resource URIs of the DataReportingProvisioning API is shown in Figure 5.24.2.1-1.

Figure 5.24.2.1-1: Resource URI structure of the DataReportingProvisioning API

Table 5.24.2.1-1 provides an overview of the resources and applicable HTTP methods applicable for the DataReportingProvisioning API.

Table 5.24.2.1-1: Resources and methods overview

| Resource name | Resource URI (relative path under API URI) | HTTP method | Description (service operation) |
| --- | --- | --- | --- |
| Data Reporting Provisioning Sessions | /sessions | POST | Create a Data Reporting Provisioning Session. |
| Individual Data Reporting Provisioning Session | /sessions/{sessionId} | GET | Retreive an existing "Individual Data Reporting Provisioning Session" resource. |
|  |  | DELETE | Delete an existing "Individual Data Reporting Provisioning Session" resource. |
| Data Reporting Configurations | /sessions/{sessionId}/configurations | POST | Create a new Data Reporting Configuration. |
| Individual Data Reporting Configuration | /sessions/{sessionId}/configurations/{configurationId} | GET | Retrieves an existing "Individual Data Reporting Configuration" resource. |
|  |  | PUT | Update an existing "Individual Data Reporting Configuration" resource. |
|  |  | PATCH | Modify an existing "Individual Data Reporting Configuration" resource. |
|  |  | DELETE | Delete an existing "Individual Data Reporting Configuration" resource. |

#### 5.24.2.2	Resource: Data Reporting Provisioning Sessions

##### 5.24.2.2.1	Introduction

This resource represents the collection of Data Reporting Provisioning Sessions managed by the NEF.

This resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [3]).

##### 5.24.2.2.2	Resource definition

Resource URL: {apiRoot}/3gpp-data-reporting-provisioning/v1/sessions

This resource shall support the resource URI variables defined in table 5.24.2.2.2-1.

Table 5.24.2.2.2-1: Resource URL variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.24.1. |

##### 5.24.2.2.3	Resource Methods

###### 5.24.2.2.3.1	POST

This method enables an AF to request the creation of a Data Reporting Provisioning Session at the NEF.

This method shall support the URI query parameters specified in table 5.24.2.2.3.1-1.

Table 5.24.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.24.2.2.3.1-2 and the response data structures and response codes specified in table 5.24.2.2.3.1-4.

Table 5.24.2.2.3.1-2: Data structures supported by the POST request body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataReportingProvisioningSession | M | 1 | Representation of the Data Reporting Provisioning Session to be created in the NEF. (NOTE) |
| NOTE: The "provisioningSessionId" attribute of the DataReportingProvisioningSession data type shall not be provided as it is not applicable. |  |  |  |

Table 5.24.2.2.3.1-3: Data structures supported by the POST response body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataReportingProvisioningSession (NOTE 2) | M | 1 | 201 Created | Successful case. A representation of the created "Individual Data Reporting Provisioning Session" resource is returned. An HTTP "Location" header that contains the URI of the created "Individual Data Reporting Provisioning Session" resource shall be returned. |
| NOTE 1: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: The "provisioningSessionId" attribute of the DataReportingProvisioningSession data type shall not be provided as it is not applicable. |  |  |  |  |

Table 5.24.2.2.3.1-4: Headers supported by the 201 response code on this resource

| HTTP response header | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | The URI of the newly created resource, according to the structure: {apiRoot}/3gpp-data-reporting-provisioning/<apiVersion>/sessions/{sessionId} |

#### 5.24.2.3	Resource: Individual Data Reporting Provisioning Session

##### 5.24.2.3.1	Introduction

This resource represents an "Individual Data Reporting Provisioning Session" resource managed by the NEF.

This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [3]).

##### 5.24.2.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-data-reporting-provisioning/<apiVersion>/sessions/{sessionId}

This resource shall support the resource URI variables defined in table 5.24.2.3.2-1.

Table 5.24.2.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.24.1. |
| sessionId | string | Represents the identifierof the "Individual Data Reporting Provisioning Session" resource. |

##### 5.24.2.3.3	Resource standard methods

###### 5.24.2.3.3.1	GET

This method enables an AF to retrieve an existing "Individual Data Reporting Provisioning Session" resource at the NEF.

This method shall support the URI query parameters specified in table 5.24.2.3.3.1-1.

Table 5.24.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.24.2.3.3.1-2 and the response data structures and response codes specified in table 5.24.2.3.3.1-3.

Table 5.24.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.24.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataReportingProvisioningSession | M | 1 | 200 OK | Successful case. The requested "Individual Data Reporting Provisioning Session" resource is successfully returned to the AF. (NOTE 2) |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE 1: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: The "provisioningSessionId" attribute of the DataReportingProvisioningSession data type shall not be provided as it is not applicable. |  |  |  |  |

Table 5.24.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative NEF. |

Table 5.24.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.24.2.3.3.2	Void

###### 5.24.2.3.3.3	DELETE

This method enables an AF to request the deletion of an existing "Individual Data Reporting Provisioning Session" resource at the NEF.

This method shall support the URI query parameters specified in table 5.24.2.3.3.3-1.

Table 5.24.2.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.24.2.3.3.3-2 and the response data structures and response codes specified in table 5.24.2.3.3.3-3.

Table 5.24.2.3.3.3-2: Data structures supported by the DELETE request body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.24.2.3.3.3-3: Data structures supported by the DELETE response body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful response. The "Individual Data Reporting Provisioning Session" resource was successfully deleted. |
|  |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
|  |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.24.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located on an alternative service instance within the same NEF. |

Table 5.24.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains an alternative URI of the resource located on an alternative service instance within the same NEF. |

#### 5.24.2.4	Resource: Data Reporting Configurations

##### 5.24.2.4.1	Introduction

This resource represents the collection of Data Reporting Configurations managed by the NEF.

This resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [3]).

##### 5.24.2.4.2	Resource definition

Resource URIL: {apiRoot}/3gpp-data-reporting-provisioning/<apiVersion>/sessions/{sessionId}/configurations

This resource shall support the resource URI variables defined in table 5.24.2.4.2-1.

Table 5.24.2.4.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.24.1. |
| sessionId | string | Represents the identifier of the existing "Individual Data Reporting Provisioning Session" resource. |

##### 5.24.2.4.3	Resource Methods

###### 5.24.2.4.3.1	POST

This method enables an AF to request the creation of a Data Reporting Configuration at the NEF.

This method shall support the URI query parameters specified in table 5.24.2.4.3.1-1.

Table 5.24.2.4.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.24.2.4.3.1-2 and the response data structures and response codes specified in table 5.24.2.4.3.1-4.

Table 5.24.2.4.3.1-2: Data structures supported by the POST request body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataReportingConfiguration | M | 1 | Representation of the Data Reporting Configuration to be created in the NEF. (NOTE) |
| NOTE: The "dataReportingConfigurationId" attribute of the DataReportingConfiguration data type shall not be provided as it is not applicable. |  |  |  |

Table 5.24.2.4.3.1-3: Data structures supported by the POST response body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataReportingConfiguration | M | 1 | 201 Created | Successful case. A representation of the created "Individual Data Reporting Configuration" resource is returned. An HTTP "Location" header that contains the URI of the created resource shall be returned in the response body. (NOTE 2) |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE 1: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: The "dataReportingConfigurationId" attribute of the DataReportingConfiguration data type shall not be provided as it is not applicable. |  |  |  |  |

Table 5.24.2.4.3.1-4: Headers supported by the 201 response code on this resource

| HTTP response header | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | The URI of the newly created resource, according to the structure: {apiRoot}/3gpp-data-reporting-provisioning/<apiVersion>/sessions/{sessionId}/configurations/{configurationId} |

Table 5.24.2.4.3.1-5: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.24.2.4.3.1-6: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

#### 5.24.2.5	Resource: Individual Data Reporting Configuration

##### 5.24.2.5.1	Introduction

This resource represents an Individual Data Reporting Configuration resource managed by the NEF.

This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [3]).

##### 5.24.2.5.2	Resource Definition

Resource URI: {apiRoot}/3gpp-data-reporting-provisioning/<apiVersion>/sessions/{sessionId}/configurations/{configurationId}

This resource shall support the resource URI variables defined in table 5.24.2.5.2-1.

Table 5.24.2.5.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.24.1 |
| sessionId | string | Represents the identifier of the "Individual Data Reporting Provisioning Session" resource. |
| configurationId | string | Represents the identifier of the "Individual Data Reporting Configuration" resource. |

##### 5.24.2.5.3	Resource standard methods

###### 5.24.2.5.3.2	GET

This method enables an AF to retrieve an existing "Individual Data Reporting Configuration" resource at the NEF.

This method shall support the URI query parameters specified in table 5.24.2.5.3.2-1.

Table 5.24.2.5.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.24.2.5.3.2-2 and the response data structures and response codes specified in table 5.24.2.5.3.2-3.

Table 5.24.2.5.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.24.2.5.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataReportingConfiguration | M | 1 | 200 OK | Successful case. The requested Individual Data Reporting Configuration resource is returned. (NOTE 2) |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE 1: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: The "dataReportingConfigurationId" attribute of the DataReportingConfiguration data type shall not be provided as it is not applicable. |  |  |  |  |

Table 5.24.2.5.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative NEF. |

Table 5.24.2.5.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.24.2.5.3.3	PUT

This method enables an AF to update an existing "Individual Data Reporting Configuration" resource at the NEF.

This method shall support the URI query parameters specified in table 5.24.2.5.3.3-1.

Table 5.24.2.5.3.3-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 5.24.2.5.3.3-2 and the response data structures and response codes specified in table 5.24.2.5.3.3-3.

Table 5.24.2.5.3.3-2: Data structures supported by the PUT request body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataReportingConfiguration | M | 1 | Parameters to updatethe Individual Data Reporting Configuration resource. (NOTE) |
| NOTE: The "dataReportingConfigurationId" attribute of the DataReportingConfiguration data type shall not be provided as it is not applicable. |  |  |  |

Table 5.24.2.5.3.3-3: Data structures supported by the PUT response body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataReportingConfiguration | M | 1 | 200 OK | Successful case. The "Individual Data Reporting Configuration" resource was updated successfully and a representation of the updated resource is returned in the response body. (NOTE 2) |
| n/a |  |  | 204 No Content | Successful case. The "Individual Data Reporting Configuration" resource was successfully updated and no content is to be returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE 1: The mandatory HTTP error status codes for the HTTP PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: The "dataReportingConfigurationId" attribute of the DataReportingConfiguration data type shall not be provided as it is not applicable. |  |  |  |  |

Table 5.24.2.5.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

Table 5.24.2.5.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

###### 5.24.2.5.3.3A	PATCH

The PATCH method is used to modify an existing "Individual Data Reporting Configuration" resource.

This method shall support the request data structures specified in table 5.24.2.5.3.3A-1, and the response data structures and response codes specified in table 5.24.2.5.3.3A-2.

Table 5.24.2.5.3.3A-1: Data structures supported by the PATCH request body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataReportingConfigurationPatch | M | 1 | Contains the parameters to request the modification of the "Individual Data Reporting Configuration" resource. |

Table 5.24.2.5.3.3A-2: Data structures supported by the PATCH response body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataReportingConfiguration | M | 1 | 200 OK | Successful case. The "Individual Data Reporting Configuration" resource was updated successfully by configuration data provided. (NOTE 2) |
| n/a |  |  | 204 No Content | Successful case. The "Individual Data Reporting Configuration" resource was successfully updated and no content is to be returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE 1: The mandadatory HTTP error status codes for the HTTP PATCH method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: The "dataReportingConfigurationId" attribute of the DataReportingConfiguration data type shall not be provided as it is not applicable. |  |  |  |  |

Table 5.24.2.5.3.3A-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

Table 5.24.2.5.3.3A-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

###### 5.24.2.5.3.4	DELETE

This method enables an AF to request the deletion of an existing "Individual Data Reporting Configuration" resource at the NEF.

This method shall support the URI query parameters specified in table 5.24.2.5.3.4-1.

Table 5.24.2.5.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.24.2.5.3.4-2 and the response data structures and response codes specified in table 5.24.2.5.3.4-3.

Table 5.24.2.5.3.4-2: Data structures supported by the DELETE request body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.24.2.5.3.4-3: Data structures supported by the DELETE response body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful response. The "Individual Data Reporting Configuration" resource was successfully deleted. |
|  |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
|  |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.24.2.5.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

Table 5.24.2.5.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

### 5.24.3	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.24.4	Notifications

There are no notifications defined for this API in this release of the specification.

### 5.24.5	Data Model

#### 5.24.5.1	General

This clause specifies the application data model supported by the DataReportingProvisioning API. Table 5.24.5.1-1 specifies the data types defined for the DataReportingProvisioning API.

Table 5.24.5.1-1: DataReportingProvisioning specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| n/a |  |  |

Table 5.24.5.1-2 specifies data types re-used by the DataReportingProvisioning API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the DataReportingProvisioning API.

Table 5.24.5.1-2: DataReportingProvisioning re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DataReportingProvisioningSession | 3GPP TS 26.532 [60] | Configuration by the AF specifying the data to be collected, processed and reported. |  |
| DataReportingConfiguration | 3GPP TS 26.532 [60] | Configuration data related to UE data collection and reporting. |  |
| DataReportingConfigurationPatch | 3GPP TS 26.532 [60] | Contains the requested modifications to the configuration data regarding UE data collection and reporting. |  |

### 5.24.6	Used Features

The table below defines the features applicable to the DataReportingProvisioning API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.24.6-1: Features used by DataReportingProvisioning API

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 5.24.7	Error handling

#### 5.24.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.24.7.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the DataReportingProvisioning API.

#### 5.24.7.3	Application Errors

The application errors defined for the DataReportingProvisioning API are listed in table 5.24.7.3-1.

Table 5.24.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

## 5.25	UEId API

### 5.25.1	Introduction

The Nnef_UEId service shall use the UEId API.

The API URI of UEId API shall be:

- {apiRoot}/3gpp-ueid/v1

with the following components:

-	"apiRoot" is set as described in clause 5.2.4 in 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-ueid".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.25.2	Resources

#### 5.25.2.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.25.2.1-1 and the resources and HTTP methods used for the UEId API.

Figure 5.25.2.1-1: Resource URI structure of the UEId API

Table 5.25.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.25.2.1-1: Resources and methods overview

| Resource name | Resource URI (relative path under API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| UE ID Mapping Information Provisionings | /{afId}/pp | GET | Retrieve all the active UE ID Mapping Information Provisionings managed by the NEF. |
|  |  | POST | Request the creation of a new UE ID Mapping Information Provisioning at the NEF. |
| Individual UE ID Mapping Information Provisioning | /{afId}/pp/{ppId} | GET | Retrieve an existing "Individual UE ID Mapping Information Provisioning" managed by the NEF. |
|  |  | PUT | Update an existing "Individual UE ID Mapping Information Provisioning" managed by the NEF. |
|  |  | PATCH | Modify an existing "Individual UE ID Mapping Information Provisioning" managed by the NEF. |
|  |  | DELETE | Delete an existing "Individual UE ID Mapping Information Provisioning" managed by the NEF. |

#### 5.25.2.2	Resource: UE ID Mapping Information Provisionings

##### 5.25.2.2.1	Introduction

This resource represents the collection of UE ID Mapping Information Provisionings managed by the NEF.

This resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [32]).

##### 5.25.2.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-ueid/<apiVersion>/{afId}/pp

This resource shall support the resource URI variables defined in table 5.25.2.2.2-1.

Table 5.25.2.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.25.1. |
| afId | string | Represents the identifier of the AF. |

##### 5.25.2.2.3	Resource Methods

###### 5.25.2.2.3.1	GET

This method enables to request to retrieve all the active UE ID Mapping Information Provisionings managed by the NEF.

This method shall support the URI query parameters specified in table 5.25.2.2.3.1-1.

Table 5.25.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.25.2.2.3.1-2 and the response data structures and response codes specified in table 5.25.2.2.3.1-3.

Table 5.25.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.25.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(UeIdMappingInfo) | M | 0..N | 200 OK | Successful case. All the "Individual UE ID Mapping Information Provisioning" resource(s) managed by the NEF are returned. If there are no existing "Individual UE ID Mapping Information Provisioning" resources managed at the NEF, an empty array is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |  |

Table 5.25.2.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.25.2.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.25.2.2.3.2	POST

This method enables an AF to request the creation of a new UE ID Mapping Information Provisioning at the NEF.

This method shall support the URI query parameters specified in table 5.25.2.2.3.2-1.

Table 5.25.2.2.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.25.2.2.3.2-2 and the response data structures and response codes specified in table 5.25.2.2.3.2-3.

Table 5.25.2.2.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UeIdMappingInfo | M | 1 | Contains the representation of the UE ID Mapping Information Provisioning to be created at the NEF. |

Table 5.25.2.2.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UeIdMappingInfo | M | 1 | 201 Created | Successful case. A representation of the created "Individual UE ID Mapping Information Provisioning" resource is returned in the response body. The URI of the created resource shall be returned in an HTTP "Location" header. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.25.2.2.3.2-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-ueid/<apiVersion>/{afId}/pp/{ppId} |

##### 5.25.2.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 5.25.2.3	Resource: Individual UE ID Mapping Provisioning

##### 5.25.2.3.1	Introduction

This resource represents an "Individual UE ID Mapping Provisioning" resource managed by the NEF.

This resource is modelled with the Document resource archetype (see clause C.2 of 3GPP TS 29.501 [32]).

##### 5.25.2.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-ueid/<apiVersion>/{afId}/pp/{ppId}

This resource shall support the resource URI variables defined in table 5.25.2.3.2-1.

Table 5.25.2.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.25.1. |
| afId | string | Represents the identifier of the AF. |
| ppId | string | Represents the identifier of the "Individual UE ID Mapping Provisioning" resource. |

##### 5.25.2.3.3	Resource Methods

###### 5.25.2.3.3.1	GET

This method enables an AF to request to retrieve an existing "Individual UE ID Mapping Information Provisioning" resource at the NEF.

This method shall support the URI query parameters specified in table 5.25.2.3.3.1-1.

Table 5.25.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.25.2.3.3.1-2 and the response data structures and response codes specified in table 5.25.2.3.3.1-3.

Table 5.25.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.25.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UeIdMappingInfo | M | 1 | 200 OK | Successful case. The requested "Individual UE ID Mapping Information Provisioning" resource is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.25.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.25.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.25.2.3.3.2	PUT

This method enables an AF to request the update of an existing "Individual UE ID Mapping Information Provisioning" resource at the NEF.

This method shall support the URI query parameters specified in table 5.25.2.3.3.2-1.

Table 5.25.2.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.25.2.3.3.2-2 and the response data structures and response codes specified in table 5.25.2.3.3.2-3.

Table 5.25.2.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UeIdMappingInfo | M | 1 | Represents the updated "Individual UE ID Mapping Information Provisioning" resource representation. |

Table 5.25.2.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |  |
| --- | --- | --- | --- | --- | --- |
| UeIdMappingInfo | M | 1 | 200 OK | Successful response. The "Individual UE ID Mapping Information Provisioning" resource is successfully updated and a representation of the updated resource is returned in the response body. |  |
| n/a |  |  | 204 No Content | Successful response. The "Individual UE ID Mapping Information Provisioning" resource is successfully updated and no content is returned in the response body. |  |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.25.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

Table 5.25.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

###### 5.25.2.3.3.3	PATCH

This method enables an AF to request the modification of an existing "Individual UE ID Mapping Information Provisioning" resource at the NEF.

This method shall support the URI query parameters specified in table 5.25.2.3.3.3-1.

Table 5.25.2.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.25.2.3.3.3-2 and the response data structures and response codes specified in table 5.25.2.3.3.3-3.

Table 5.25.2.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UeIdMappingInfoPatch | M | 1 | Represents the requested modifications to the "Individual UE ID Mapping Information Provisioning" resource. |

Table 5.25.2.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |  |
| --- | --- | --- | --- | --- | --- |
| UeIdMappingInfo | M | 1 | 200 OK | Successful response. The "Individual UE ID Mapping Information Provisioning" resource is successfully modified and a representation of the updated resource is returned in the response body. |  |
| n/a |  |  | 204 No Content | Successful response. The "Individual UE ID Mapping Information Provisioning" resource is successfully modified and no content is returned in the response body. |  |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.25.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.25.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.25.2.3.3.4	DELETE

This method enables an AF to request the deletion of an existing "Individual UE ID Mapping Information Provisioning" resource at the NEF.

This method shall support the URI query parameters specified in table 5.25.2.3.3.4-1.

Table 5.25.2.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.25.2.3.3.4-2 and the response data structures and response codes specified in table 5.25.2.3.3.4-3.

Table 5.25.2.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.25.2.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual UE ID Information Mapping Provisioning" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.25.2.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.25.2.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

### 5.25.3	Custom Operations without associated resources

#### 5.25.3.1	Overview

The structure of the custom operation URIs of the UEId API is shown in Figure 5.25.3.1-1.

Figure 5.25.3.1-1: Custom operation URI structure of the UEId API

Table 5.25.3.1-1 provides an overview of the custom operations and applicable HTTP methods.

Table 5.25.3.1-1: Custom operations without associated resources

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Retrieve | /retrieve | POST | Request to retrieve AF specific UE ID information. |
| GetMsisdn | /get-msisdn | POST | Request to get the UE ID in the form of MSISDN of the UE. |

#### 5.25.3.2	Operation: Retrieve

##### 5.25.3.2.1	Description

The custom operation allows a service consumer to retrieve AF specific UE ID information via the NEF.

##### 5.25.3.2.2	Operation Definition

This operation shall support the request and response data structures and response codes specified in table 5.25.3.2.2-1 and table 5.25.3.2.2-2.

Table 5.25.3.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UeIdReq | M | 1 | Parameters to request to retrieve AF specific UE ID information. |

Table 5.25.3.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UeIdInfo | M | 1 | 200 OK | The requested AF specific UE ID information was returned successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4] |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| ProblemDetails | O | 0..1 | 404 Not Found | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure cases are described in clause 5.25.7. |  |  |  |  |

Table 5.25.3.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.25.3.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

##### 5.25.3.2.3	Operation Definition

This operation shall support the request and response data structures and response codes specified in table 5.25.3.2.3-1 and table 5.25.3.2.3-2.

Table 5.25.3.2.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MsisdnReq | M | 1 | Contains the parameters to request to retrieve the UE ID in the form of MSISDN of the UE. |

Table 5.25.3.2.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MsisdnInfo | M | 1 | 200 OK | Successful case. The requested UE ID in the form of MSISDN of the UE is returned successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| ProblemDetails | O | 0..1 | 404 Not Found | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. NOTE 2: Failure cases are described in clause 5.25.7. |  |  |  |  |

Table 5.25.3.2.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

Table 5.25.3.2.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

### 5.25.4	Notifications

There are no Notifications defined for this API in this release of the specification.

### 5.25.5	Data Model

#### 5.25.5.1	General

This clause specifies the application data model supported by the UEId API. Table 5.25.5.1-1 specifies the data types defined for the UEId API.

Table 5.25.5.1-1: UEId service specific Data Types

| Data type |  | Clause defined |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MsisdnReq |  | 5.25.5.2.4 |  | Represents the parameters to request to retrieve the UE ID in the form of MSISDN of the UE. |  |  |  |
| MsisdnInfo |  | 5.25.5.2.5 |  | Represents the UE ID in the form of MSISDN information. |  |  |  |
| UeIdReq |  | 5.25.5.2.2 |  | Represents the parameters to requestAF specific UE ID retrieval. |  |  |  |
| UeIdInfo |  | 5.25.5.2.3 |  | Represents AF specific UE ID information. |  |  |  |
| UeIdMappingInfo |  | 5.25.5.2.6 |  | Represents the UE ID Mapping Information Provisioning. |  |  |  |
| UeIdMappingInfoPatch |  | 5.25.5.2.8 |  | Represents the requested modifications to a UE ID Mapping Information Provisioning. |  |  |  |
| RangSlUeIdMappInfo |  | 5.25.5.2.7 |  | Represents the Ranging/Sidelink UE ID mapping information between the Application Layer ID and the GPSI. |  |  |  |

Table 5.25.5.1-2 specifies data types re-used by the UEId API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the UEId API.

Table 5.25.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ApplicationlayerId | 3GPP TS 29.571 [8] | Represents an Application Layer ID. |  |
| Dnn | 3GPP TS 29.571 [8] | Identifies a DNN. |  |
| ExternalId | 3GPP TS 29.122 [4] | Represents an External Identifier. |  |
| Gpsi | 3GPP TS 29.571 [8] | Represents a GPSI. |  |
| IpAddr | 3GPP TS 29.571 [8] | Identifes an IP address. |  |
| MacAddr48 | 3GPP TS 29.571 [8] | Identifies a MAC address. |  |
| Msisdn | 3GPP TS 29.122 [4] | Represents the MSISDN. |  |
| MtcProviderInformation | 3GPP TS 29.571 [8] | Indicates MTC provider information. |  |
| Port | 3GPP TS 29.122 [4] | Identifies a port, unsigned integer with valid values between 0 and 65535. | PortNumber |
| ProblemDetails | 3GPP TS 29.122 [4] | Represents error related information. |  |
| Snssai | 3GPP TS 29.571 [8] | Identifies the S-NSSAI. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| Uinteger | 3GPP TS 29.571 [8] | Represents a unsigned integer. |  |

#### 5.25.5.2	Structured data types

##### 5.25.5.2.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.25.5.2.2	Type: UeIdReq

Table 5.25.5.2.2-1: Definition of type UeIdReq

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability (NOTE 1) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| afId |  | string |  | M |  | 1 |  | Represents the identifier of theAF that is sending the request. |  |  |  |
| appPortId |  | Port |  | O |  | 0..1 |  | Identifies an application port ID. See clause 9.2.3.24.4 of 3GPP TS 23.040 [62] for further details. |  |  |  |
| dnn |  | Dnn |  | O |  | 0..1 |  | Identifies a DNN. |  |  |  |
| ipDomain |  | string |  | O |  | 0..1 |  | The IPv4 address domain identifier. The attribute may only be present if the IPv4 address is provided in the "ueIpAddr" attribute. |  |  |  |
| mtcProviderId |  | MtcProviderInformation |  | O |  | 0..1 |  | Indicates MTC provider information. |  |  |  |
| portNumber |  | Port |  | O |  | 0..1 |  | Indicates the UDP or TCP port number associated with the UE IP address as provided in the "ueIpAddr" attribute. |  | PortNumber |  |
| snssai |  | Snssai |  | O |  | 0..1 |  | Identifies an S-NSSAI. |  |  |  |
| ueIpAddr |  | IpAddr |  | C |  | 0..1 |  | Identifies a UE IP Address. (NOTE 2) |  |  |  |
| ueMacAddr |  | MacAddr48 |  | C |  | 0..1 |  | Identifies a UE MAC Address. (NOTE 2) |  |  |  |
| suppFeat |  | SupportedFeatures |  | C |  | 0..1 |  | Indicates the list of Supported features used as described in clause 5.25.6. This attribute shall be provided when feature negotiation needs to take place. |  |  |  |
| NOTE 1: Properties marked with a feature as defined in clause 5.25.6 are applicable as described in clause 5.2.7 of 3GPP TS 29.122 [4]. If no feature is indicated, the related property is always applied. NOTE 2: One of the "ueIpAddr" attribute or "ueMacAddr" attribute shall be included. |  |  |  |  |  |  |  |  |  |  |  |

##### 5.25.5.2.3	Type: UeIdInfo

Table 5.25.5.2.3-1: Definition of type UeIdInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| externalId | ExternalId | M | 1 | Contains the AF specific UE ID in the form of an external identifier uniquely identifying the user. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the features supported by both the AF and the NEF. This attribute shall be provided if feature negotiation needs to take place and it was provided by the AF in the corresponding request body. |  |

##### 5.25.5.2.4	Type: MsisdnReq

Table 5.25.5.2.4-1: Definition of type MsisdnReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afId | string | M | 1 | Represents the identifier of theAF that is sending the request. |  |
| appPortId | Port | O | 0..1 | Identifies an application port ID. See clause 9.2.3.24.4 of 3GPP TS 23.040 [62] for further details. |  |
| dnn | Dnn | O | 0..1 | Identifies a DNN. |  |
| ipDomain | string | O | 0..1 | The IPv4 address domain identifier. The attribute may only be present if the IPv4 address is provided in the "ueIpAddr" attribute. |  |
| mtcProviderId | MtcProviderInformation | O | 0..1 | Indicates MTC provider information. |  |
| portNumber | Port | O | 0..1 | Indicates the UDP or TCP port number associated with the UE IP address as provided in the "ueIpAddr" attribute. |  |
| snssai | Snssai | O | 0..1 | Identifies an S-NSSAI. |  |
| ueIpAddr | IpAddr | C | 0..1 | Identifies a UE IP Address. (NOTE) |  |
| ueMacAddr | MacAddr48 | C | 0..1 | Identifies a UE MAC Address. (NOTE) |  |
| suppFeat | SupportedFeatures | C | 0..1 | Indicates the list of Supported features used as described in clause 5.25.6. This attribute shall be provided when feature negotiation needs to take place. |  |
| NOTE: One of the "ueIpAddr" attribute or "ueMacAddr" attribute shall be included. |  |  |  |  |  |

##### 5.25.5.2.5	Type: MsisdnInfo

Table 5.25.5.2.5-1: Definition of type MsisdnInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| msisdn | Msisdn | M | 1 | Contains the MSISDN of the UE. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Indicates the list of Supported features used as described in clause 5.25.6. This attribute shall be provided when feature negotiation needs to take place. |  |

##### 5.25.5.2.6	Type: UeIdMappingInfo

Table 5.25.5.2.6-1: Definition of type UeIdMappingInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| rsUeIdMappingInfo | RangSlUeIdMappInfo | C | 0..1 | Contains the Ranging/Sidelink UE ID Mapping information between the Application Layer ID and the GPSI. This attribute shall be present only for Ranging/Sidelink UE ID Mapping provisioning. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Indicates the list of supported features. This attribute shall be present only when feature negotiation needs to take place. |  |

##### 5.25.5.2.7	Type: RangSlUeIdMappInfo

Table 5.25.5.2.7-1: Definition of type RangSlUeIdMappInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appLayerId | ApplicationlayerId | M | 1 | Contains the Application Layer ID. |  |
| gpsi | Gpsi | M | 1 | Contains the GPSI. |  |

##### 5.25.5.2.8	Type: UeIdMappingInfoPatch

Table 5.25.5.2.8-1: Definition of type UeIdMappingInfoPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| rsUeIdMappingInfo | RangSlUeIdMappInfo | O | 0..1 | Contains the updated Ranging/Sidelink UE ID Mapping information between the Application Layer ID and the GPSI. This attribute may be present only for Ranging/Sidelink UE ID Mapping provisioning. |  |

#### 5.25.5.3	Simple data types and enumerations

##### 5.25.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.25.5.3.2	Simple data types

The simple data types defined in table 5.25.5.3.2-1 shall be supported.

Table 5.25.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

### 5.25.6	Used Features

The table below defines the features applicable to the UEId API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.25.6-1: Features used by UEId API

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | PortNumber | This feature indicates supporting AF providing Port Number associated with the UE IP address in the request. |

### 5.25.7	Error handling

#### 5.25.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.25.7.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the UEId API.

#### 5.25.7.3	Application Errors

The application errors defined for the UEId API are listed in table 5.25.7.3-1.

Table 5.25.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |
| REQUEST_NOT_AUTHORIZED | 403 Forbidden | Indicates that the AF specific UE ID retrieval request is not authorized. |  |
| UE_ID_NOT_AVAILABLE | 404 Not Found | Indicates that the requested AF specific UE ID is not available. |  |
| UE_NOT_FOUND | 404 Not Found | Indicates that the requested UE address is not found. |  |
| USER_CONSENT_NOT_GRANTED | 403 Forbidden | Indicates that the request is rejected because user consent is not granted. |  |

### 5.25.8	Security

The provisions of clause 6 and 7.2 of shall apply for the Nnef_UeId API. In addition:

-	For the "GetMsisdn" custom operation defined in clause 5.25.3.2.3, RNAA procedures specified in clause 5.6.2.3.2 of 3GPP TS 29.222 [12] may be supported and the "authorization code" grant type may be supported in addition to the "Client Credentials".

## 5.26	MBSUserService API

### 5.26.1	Introduction

The Nnef_MBSUserService service shall use the MBSUserService API.

The API URI of MBSUserService API shall be:

- {apiRoot}/3gpp-mbs-us/v1

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-mbs-us".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.26.2	Resources

#### 5.26.2.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.26.2.1-1 and the resources and HTTP methods used for the MBSUserService API.

Figure 5.26.2.1-1: Resource URI structure of the MBSUserService API

Table 5.26.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.26.2.1-1: Resources and methods overview

| Resource name | Resource URI (relative path under API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| MBS User Services | /mbs-user-services | GET | Retrieve all the active MBS User Services managed by the NEF. |
|  |  | POST | Request the creation of a new MBS User Service. |
| Individual MBS User Service | /mbs-user-services/{mbsUserServId} | GET | Retrieve an existng MBS User Service managed by the NEF. |
|  |  | PUT | Update an existng MBS User Service managed by the NEF. |
|  |  | PATCH | Modify an existng MBS User Service managed by the NEF. |
|  |  | DELETE | Delete an existng MBS User Service managed by the NEF. |

#### 5.26.2.2	Resource: MBS User Services

##### 5.26.2.2.1	Introduction

This resource represents the collection of MBS User Services managed by the NEF.

This resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [3]).

##### 5.26.2.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-mbs-us/v1/mbs-user-services

This resource shall support the resource URI variables defined in table 5.26.2.2.2-1.

Table 5.26.2.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.26.1. |

##### 5.26.2.2.3	Resource Standard Methods

###### 5.26.2.2.3.1	GET

This method allows an AF to retrieve all the active MBS User Service resources at the NEF.

This method shall support the URI query parameters specified in table 5.26.2.2.3.1-1.

Table 5.26.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.26.2.2.3.1-2 and the response data structures and response codes specified in table 5.26.2.2.3.1-3.

Table 5.26.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.26.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(MBSUserService) | M | 0..N | 200 OK | Successful case. All the active MBS User Services managed by the NEF are returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status code for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.26.2.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.26.2.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.26.2.2.3.2	POST

This method enables an AF to request the creation of an MBS User Service resource at the NEF.

This method shall support the URI query parameters specified in table 5.26.2.2.3.2-1.

Table 5.26.2.2.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.26.2.2.3.2-2 and the response data structures and response codes specified in table 5.26.2.2.3.2-3.

Table 5.26.2.2.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserService | M | 1 | Contains the parameters to request the creation of a new MBS User Service at the NEF. |

Table 5.26.2.2.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserService | M | 1 | 201 Created | Successful case. A new MBS User Service is successfully created and a representation of the created Individual MBS User Service resource is returned. An HTTP "Location" header that contains the resource URI of the created Individual MBS User Service resource shall also be included. |
| NOTE: The mandatory HTTP error status code for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.26.2.2.3.2-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-mbs-us/v1/mbs-user-services/{mbsUserServId} |

##### 5.26.2.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 5.26.2.3	Resource: Individual MBS User Service

##### 5.26.2.3.1	Introduction

This resource represents an Individual MBS User Service managed by the NEF.

This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [3]).

##### 5.26.2.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-mbs-us/v1/mbs-user-services/{mbsUserServId}

This resource shall support the resource URI variables defined in table 5.26.2.3.2-1.

Table 5.26.2.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.26.1. |
| mbsUserServId | string | Contains the unique identifier of the Individual MBS User Service resource assigned by the NEF. |

##### 5.26.2.3.3	Resource Standard Methods

###### 5.26.2.3.3.1	GET

This method allows an AF to retrieve an existing Individual MBS User Service resource at the NEF.

This method shall support the URI query parameters specified in table 5.26.2.3.3.1-1.

Table 5.26.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.26.2.3.3.1-2 and the response data structures and response codes specified in table 5.26.2.3.3.1-3.

Table 5.26.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.26.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserService | M | 1 | 200 OK | Successful case. The requested Individual MBS User Service resource is successfully returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status code for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.26.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.26.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.26.2.3.3.2	PUT

This method enables an AF to request the update of an existing Individual MBS User Service resource at the NEF.

This method shall support the URI query parameters specified in table 5.26.2.3.3.2-1.

Table 5.26.2.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.26.2.3.3.2-2 and the response data structures and response codes specified in table 5.26.2.3.3.2-3.

Table 5.26.2.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserService | M | 1 | Contains the updated representation of the Individual MBS User Service resource that is to be updated. |

Table 5.26.2.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserService | M | 1 | 200 OK | Successful case. The concerned Individual MBS User Service resource is successfully updated and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The concerned Individual MBS User Service resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status code for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.26.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.26.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.26.2.3.3.3	PATCH

This method enables an AF to request the modification of an existing Individual MBS User Service resource at the NEF.

This method shall support the URI query parameters specified in table 5.26.2.3.3.3-1.

Table 5.26.2.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.26.2.3.3.3-2 and the response data structures and response codes specified in table 5.26.2.3.3.3-3.

Table 5.26.2.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserServicePatch | M | 1 | Contains the parameters to request the modification of the Individual MBS User Service resource. |

Table 5.26.2.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserService | M | 1 | 200 OK | Successful case. The concerned Individual MBS User Service resource is successfully modified and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful response. The Individual MBS User Service resource was successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status code for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.26.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.26.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.26.2.3.3.4	DELETE

This method enables an AF to request the deletion of an existing Individual MBS User Service resource at the NEF.

This method shall support the URI query parameters specified in table 5.26.2.3.3.4-1.

Table 5.26.2.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.26.2.3.3.4-2 and the response data structures and response codes specified in table 5.26.2.3.3.4-3.

Table 5.26.2.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.26.2.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful response. The Individual MBS User Service resource is successfully deleted. |
|  |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
|  |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status code for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.26.2.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.26.2.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

##### 5.26.2.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

### 5.26.3	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.26.4	Notifications

There are no notifications defined for this API in this release of the specification.

### 5.26.5	Data Model

#### 5.26.5.1	General

This clause specifies the application data model supported by the MBSUserService API. Table 5.26.5.1-1 specifies the data types defined for the MBSUserService API.

Table 5.26.5.1-1: MBSUserService specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |

Table 5.26.5.1-2 specifies data types re-used by the MBSUserService API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the MBSUserService API.

Table 5.26.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| MBSUserService | 3GPP TS 29.580 [66] | Represents MBS User Service parameters. |  |
| MBSUserServicePatch | 3GPP TS 29.580 [66] | Represents the requested modifications to an MBS User Service resource representation. |  |

#### 5.26.5.2	Structured data types

##### 5.26.5.2.1	Introduction

This clause defines the structures to be used in resource representations.

There are no structured data types defined for this API in this release of the specification.

#### 5.26.5.3	Simple data types and enumerations

##### 5.26.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.26.5.3.2	Simple data types

The simple data types defined in table 5.26.5.3.2-1 shall be supported.

Table 5.26.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

### 5.26.6	Used Features

The table below defines the features applicable to the MBSUserService API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.26.6-1: Features used by MBSUserService API

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 5.26.7	Error handling

#### 5.26.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.26.7.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the MBSUserService API.

#### 5.26.7.3	Application Errors

The application errors defined for the MBSUserService API are listed in table 5.26.7.3-1.

Table 5.26.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

## 5.27	MBSUserDataIngestSession API

### 5.27.1	Introduction

The Nnef_MBSUserDataIngestSession service shall use the MBSUserDataIngestSession API.

The API URI of MBSUserDataIngestSession API shall be:

- {apiRoot}/3gpp-mbs-ud-ingest/v1

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-mbs-ud-ingest".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.27.2	Resources

#### 5.27.2.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.27.2.1-1 and the resources and HTTP methods used for the MBSUserDataIngestSession API.

Figure 5.27.2.1-1: Resource URI structure of the MBSUserDataIngestSession API

Table 5.27.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.27.2.1-1: Resources and methods overview

| Resource name | Resource URI (relative path under API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| MBS User Data Ingest Sessions | /sessions | GET | Retrieve all the active MBS User Data Ingest Session(s) managed by the NEF. |
|  |  | POST | Request the creation of a new MBS User Data Ingest Session. |
| Individual MBS User Data Ingest Session | /sessions/{sessionId} | GET | Retrieve an existing "Individual MBS User Data Ingest Session" resource managed by the NEF. |
|  |  | PUT | Update an existing "Individual MBS User Data Ingest Session" resource managed by the NEF. |
|  |  | PATCH | Modify an existing "Individual MBS User Data Ingest Session" resource managed by the NEF. |
|  |  | DELETE | Delete an existing "Individual MBS User Data Ingest Session" resource managed by the NEF. |
| MBS User Data Ingest Session Status Subscriptions | /status-subscriptions | GET | Retrieve all the active MBS User Data Ingest Session Status Subscription(s) managed by the NEF. |
|  |  | POST | Request the creation of a new MBS User Data Ingest Session Status Subscription. |
| Individual MBS User Data Ingest Session Status Subscription | /status-subscriptions/{subscriptionId} | GET | Retrieve an existing "Individual MBS User Data Ingest Session Status Subscription" resource managed by the NEF. |
|  |  | PUT | Update an existing "Individual MBS User Data Ingest Session Status Subscription" resource managed by the NEF. |
|  |  | PATCH | Modify an existing "Individual MBS User Data Ingest Session Status Subscription" resource managed by the NEF. |
|  |  | DELETE | Delete an existing "Individual MBS User Data Ingest Session Status Subscription" resource managed by the NEF. |

#### 5.27.2.2	Resource: MBS User Data Ingest Sessions

##### 5.27.2.2.1	Introduction

This resource represents the collection of MBS User Data Ingest Sessions managed by the NEF.

This resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [3]).

##### 5.27.2.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-mbs-ud-ingest/v1/sessions

This resource shall support the resource URI variables defined in table 5.27.2.2.2-1.

Table 5.27.2.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.27.1. |

##### 5.27.2.2.3	Resource Standard Methods

###### 5.27.2.2.3.1	GET

This method allows an AF to retrieve all the active MBS User Data Ingest Sessions managed by the NEF.

This method shall support the URI query parameters specified in table 5.27.2.2.3.1-1.

Table 5.27.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.27.2.2.3.1-2 and the response data structures and response codes specified in table 5.27.2.2.3.1-3.

Table 5.27.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.27.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(MBSUserDataIngSession) | M | 0..N | 200 OK | Successful case. All the active MBS User Data Ingest Sessions managed by the NEF are returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status code for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.27.2.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.27.2.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.27.2.2.3.2	POST

This method enables an AF to request the creation of an MBS User Data Ingest Session at the NEF.

This method shall support the URI query parameters specified in table 5.27.2.2.3.2-1.

Table 5.27.2.2.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.27.2.2.3.2-2 and the response data structures and response codes specified in table 5.27.2.2.3.2-3.

Table 5.27.2.2.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserDataIngSession | M | 1 | Contains the parameters to request the creation of a new MBS User Data Ingest Session at the NEF. |

Table 5.27.2.2.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserDataIngSession | M | 1 | 201 Created | Successful case. A new MBS User Data Ingest Session is successfully created and a representation of the created Individual MBS User Data Ingest Session resource is returned. An HTTP "Location" header that contains the URI of the created Individual MBS User Data Ingest Session resource is also included. |
| ProblemDetailsMBS | O | 0..1 | 400 Bad Request | (NOTE 2) |
| ProblemDetailsMBS | O | 0..1 | 403 Forbidden | (NOTE 2) |
| ProblemDetailsMBS | O | 0..1 | 404 Not Found | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure cases are described in clause 5.27.7. |  |  |  |  |

Table 5.27.2.2.3.2-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-mbs-ud-ingest/v1/sessions/{sessionId} |

##### 5.27.2.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 5.27.2.3	Resource: Individual MBS User Data Ingest Session

##### 5.27.2.3.1	Introduction

This resource represents an Individual MBS User Data Ingest Session resource managed by the NEF.

This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [3]).

##### 5.27.2.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-mbs-ud-ingest/v1/sessions/{sessionId}

This resource shall support the resource URI variables defined in table 5.27.2.3.2-1.

Table 5.27.2.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.27.1. |
| sessionId | string | Contains the unique identifier of the Individual MBS User Data Ingest Session resource assigned by the NEF. |

##### 5.27.2.3.3	Resource Standard Methods

###### 5.27.2.3.3.1	GET

This method allows an AF to retrieve an existing "Individual MBS User Data Ingest Session" resource at the NEF.

This method shall support the URI query parameters specified in table 5.27.2.3.3.1-1.

Table 5.27.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.27.2.3.3.1-2 and the response data structures and response codes specified in table 5.27.2.3.3.1-3.

Table 5.27.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.27.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserDataIngSession | M | 1 | 200 OK | Successful case. The requested Individual MBS User Data Ingest Session resource is successfully returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status code for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.27.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.27.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.27.2.3.3.2	PUT

This method enables an AF to request the update of an existing "Individual MBS User Data Ingest Session" resource at the NEF.

This method shall support the URI query parameters specified in table 5.27.2.3.3.2-1.

Table 5.27.2.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.27.2.3.3.2-2 and the response data structures and response codes specified in table 5.27.2.3.3.2-3.

Table 5.27.2.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserDataIngSession | M | 1 | Contains the updated representation of the Individual MBS User Data Ingest Session resource that is to be updated. |

Table 5.27.2.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserDataIngSession | M | 1 | 200 OK | Successful case. The concerned Individual MBS User Data Ingest Session resource is successfully updated and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The concerned Individual MBS User Data Ingest Session resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| ProblemDetailsMBS | O | 0..1 | 400 Bad Request | (NOTE 2) |
| ProblemDetailsMBS | O | 0..1 | 403 Forbidden | (NOTE 2) |
| ProblemDetailsMBS | O | 0..1 | 404 Not Found | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure cases are described in clause 5.27.7. |  |  |  |  |

Table 5.27.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.27.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.27.2.3.3.3	PATCH

This method enables an AF to request the modification of an existing "Individual MBS User Data Ingest Session" resource at the NEF.

This method shall support the URI query parameters specified in table 5.27.2.3.3.3-1.

Table 5.27.2.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.27.2.3.3.3-2 and the response data structures and response codes specified in table 5.27.2.3.3.3-3.

Table 5.27.2.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserDataIngSessionPatch | M | 1 | Contains the parameters to request the modification of the Individual MBS User Data Ingest Session resource. |

Table 5.27.2.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserDataIngSession | M | 1 | 200 OK | Successful case. The concerned Individual MBS User Data Ingest Session resource is successfully modified and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful response. The Individual MBS User Data Ingest Session resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| ProblemDetailsMBS | O | 0..1 | 400 Bad Request | (NOTE 2) |
| ProblemDetailsMBS | O | 0..1 | 403 Forbidden | (NOTE 2) |
| ProblemDetailsMBS | O | 0..1 | 404 Not Found | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. NOTE 2: Failure cases are described in clause 5.27.7. |  |  |  |  |

Table 5.27.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.27.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.27.2.3.3.4	DELETE

This method enables an AF to request the deletion of an existing "Individual MBS User Data Ingest Session" resource at the NEF.

This method shall support the URI query parameters specified in table 5.27.2.3.3.4-1.

Table 5.27.2.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.27.2.3.3.4-2 and the response data structures and response codes specified in table 5.27.2.3.3.4-3.

Table 5.27.2.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.27.2.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful response. The Individual MBS User Data Ingest Session resource is successfully deleted. |
|  |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
|  |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE 1: The mandatory HTTP error status code for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.27.2.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.27.2.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

##### 5.27.2.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 5.27.2.4	Resource: MBS User Data Ingest Session Status Subscriptions

##### 5.27.2.4.1	Introduction

This resource represents the collection of MBS User Data Ingest Session Status Subscriptions managed by the NEF.

This resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [3]).

##### 5.27.2.4.2	Resource Definition

Resource URI: {apiRoot}/3gpp-mbs-ud-ingest/v1/status-subscriptions

This resource shall support the resource URI variables defined in table 5.27.2.4.2-1.

Table 5.27.2.4.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.27.1. |

##### 5.27.2.4.3	Resource Standard Methods

###### 5.27.2.4.3.1	GET

This method allows an AF to retrieve all the active MBS User Data Ingest Session Status Subscriptions managed by the NEF.

This method shall support the URI query parameters specified in table 5.27.2.4.3.1-1.

Table 5.27.2.4.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.27.2.4.3.1-2 and the response data structures and response codes specified in table 5.27.2.4.3.1-3.

Table 5.27.2.4.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.27.2.4.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(MBSUserDataIngStatSubsc) | M | 0..N | 200 OK | Successful case. All the active MBS User Data Ingest Session Status Subscriptions managed by the NEF are returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status code for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.27.2.4.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.27.2.4.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.27.2.4.3.2	POST

This method enables an AF to request the creation of a new MBS User Data Ingest Sesstion Status Subscription at the NEF.

This method shall support the URI query parameters specified in table 5.27.2.4.3.2-1.

Table 5.27.2.4.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.27.2.4.3.2-2 and the response data structures and response codes specified in table 5.27.2.4.3.2-3.

Table 5.27.2.4.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserDataIngStatSubsc | M | 1 | Contains the parameters to request the creation of a new MBS User Data Ingest Session Status Subscription at the NEF. |

Table 5.27.2.4.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserDataIngStatSubsc | M | 1 | 201 Created | Successful case. A new MBS User Data Ingest Session Status Subscription is successfully created and a representation of the created Individual MBS User Data Ingest Session Status Subscription resource is returned. An HTTP "Location" header that contains the URI of the created Individual MBS User Data Ingest Session Status Subscription resource is also included. |
| NOTE: The mandatory HTTP error status code for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.27.2.4.3.2-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-mbs-ud-ingest/v1/status-subscriptions/{subscriptionId} |

##### 5.27.2.4.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 5.27.2.5	Resource: Individual MBS User Data Ingest Session Status Subscription

##### 5.27.2.5.1	Introduction

This resource represents an Individual MBS User Data Ingest Session Status Subscription managed by the NEF.

This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [3]).

##### 5.27.2.5.2	Resource Definition

Resource URI: {apiRoot}/3gpp-mbs-ud-ingest/v1/status-subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 5.27.2.5.2-1.

Table 5.27.2.5.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.27.1. |
| sessionId | string | Contains the unique identifier of the Individual MBS User Data Ingest Session Status Subscription resource assigned by the NEF. |

##### 5.27.2.5.3	Resource Standard Methods

###### 5.27.2.5.3.1	GET

This method allows an AF to retrieve an existing "Individual MBS User Data Ingest Session Status Subscription" resource at the NEF.

This method shall support the URI query parameters specified in table 5.27.2.5.3.1-1.

Table 5.27.2.5.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.27.2.5.3.1-2 and the response data structures and response codes specified in table 5.27.2.5.3.1-3.

Table 5.27.2.5.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.27.2.5.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserDataIngStatSubsc | M | 1 | 200 OK | Successful case. The requested Individual MBS User Data Ingest Session Status Subscription resource is successfully returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status code for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.27.2.5.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.27.2.5.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.27.2.5.3.2	PUT

The PATCH method allows an AF to update an existing "Individual MBS User Data Ingest Session Status Subscription" resource managed by the NEF.

This method shall support the URI query parameters specified in table 5.27.2.5.3.2-1.

Table 5.27.2.5.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.27.2.5.3.2-2 and the response data structures and response codes specified in table 5.27.2.5.3.2-3.

Table 5.27.2.5.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserDataIngStatSubsc | M | 1 | Contains the parameters to request the modification of an existing "Individual MBS User Data Ingest Session Status Subscription" resource. |

Table 5.27.2.5.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserDataIngStatSubsc | M | 1 | 200 OK | Successful case. The concerned "Individual MBS User Data Ingest Session Status Subscription" resource is successfully updated and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The concerned "Individual MBS User Data Ingest Session Status Subscription" resource is successfully updated, and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.27.2.5.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.27.2.5.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.27.2.5.3.3	PATCH

The PATCH method allows an AF to modify an existing "Individual MBS User Data Ingest Session Status Subscription" resource managed by the NEF.

This method shall support the URI query parameters specified in table 5.27.2.5.3.3-1.

Table 5.27.2.5.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.27.2.5.3.3-2 and the response data structures and response codes specified in table 5.27.2.5.3.3-3.

Table 5.27.2.5.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserDataIngStatSubscPatch | M | 1 | Contains the parameters to request the modification of an existing "Individual MBS User Data Ingest Session Status Subscription" resource. |

Table 5.27.2.5.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserDataIngStatSubsc | M | 1 | 200 OK | Successful case. The concerned "Individual MBS User Data Ingest Session Status Subscription" resource is successfully modified, and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The concerned "Individual MBS User Data Ingest Session Status Subscription" resource is successfully modified and no content is returned in the response body. |
| n//a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status code for the PATCH method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.27.2.5.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.27.2.5.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.27.2.5.3.4	DELETE

This method enables an AF to request the deletion of an existing Individual MBS User Data Ingest Session Status Subscription resource at the NEF.

This method shall support the URI query parameters specified in table 5.27.2.5.3.4-1.

Table 5.27.2.5.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.27.2.5.3.4-2 and the response data structures and response codes specified in table 5.27.2.5.3.4-3.

Table 5.27.2.5.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.27.2.5.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful response. The Individual MBS User Data Ingest Session Status Subscription resource is successfully deleted. |
|  |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
|  |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE 1: The mandatory HTTP error status code for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.27.2.5.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.27.2.5.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

##### 5.27.2.5.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

### 5.27.3	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.27.4	Notifications

#### 5.27.4.1	General

Notifications shall comply to clause 5.2.5 of 3GPP TS 29.122 [4].

Table 5.27.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| MBS User Data Ingest Session Status Notification | {notifUri} | POST | This operation enables the NEF to notify a previously subscribed AF on status changes of an MBS User Data Ingest Session. |

#### 5.27.4.2	MBS User Data Ingest Session Status Change Notification

##### 5.27.4.2.1	Description

The MBS user data ingest session status change notification is used by the NEF to report one or several observed MBS user data ingest session status change events to a previously subscribed AF.

##### 5.27.4.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 5.27.4.2.2-1.

Table 5.27.4.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | Callback reference provided by the AF during the creation/update/modification of the corresponding MBS User Data Ingest Session Status Subscription. |

##### 5.27.4.2.3	Operation Definition

###### 5.27.4.2.3.1	Notification via HTTP POST

This method shall support the request data structures specified in table 5.27.4.2.3.1-1 and the response data structures and response codes specified in table 5.27.4.2.3.1-2.

Table 5.27.4.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserDataIngStatNotif | M | 1 | Provides information about the observed MBS user data ingest session status change event notification by the NEF to the AF. |

Table 5.27.4.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The MBS User Data Ingest Session Status Change Notification is successfully received. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.27.4.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

Table 5.27.4.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

###### 5.27.4.2.3.2	Void

### 5.27.5	Data Model

#### 5.27.5.1	General

This clause specifies the application data model supported by the MBSUserDataIngestSession API. Table 5.27.5.1-1 specifies the data types defined for the MBSUserDataIngestSession API.

Table 5.27.5.1-1: MBSUserDataIngestSession specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |

Table 5.27.5.1-2 specifies data types re-used by the MBSUserDataIngestSession API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the MBSUserDataIngestSession API.

Table 5.27.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| MBSUserDataIngSession | 3GPP TS 29.580 [66] | Represents MBS User Data Ingest Session parameters. |  |
| MBSUserDataIngSessionPatch | 3GPP TS 29.580 [66] | Represents the requested modifications to an MBS User Data Ingest Session resource representation. |  |
| MBSUserDataIngStatNotif | 3GPP TS 29.580 [66] | Represents an MBS User Data Ingest Session Status Notification. |  |
| MBSUserDataIngStatSubsc | 3GPP TS 29.580 [66] | Represents an MBS User Data Ingest Session Status Subscription. |  |
| MBSUserDataIngStatSubscPatch | 3GPP TS 29.580 [66] | Represents the requested modifications to an MBS User Data Ingest Session Status Subscription. |  |
| ProblemDetailsMBS | 3GPP TS 29.580 [66] | Represents an extension to the ProblemDetails data structure with potentially additional error information related to MBS. | MBSErrorHandling |

#### 5.27.5.2	Structured data types

##### 5.27.5.2.1	Introduction

This clause defines the structures to be used in resource representations.

There are no structured data types defined for this API in this release of the specification.

#### 5.27.5.3	Simple data types and enumerations

##### 5.27.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.27.5.3.2	Simple data types

The simple data types defined in table 5.27.5.3.2-1 shall be supported.

Table 5.27.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

### 5.27.6	Used Features

The table below defines the features applicable to the MBSUserDataIngestSession API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.27.6-1: Features used by MBSUserDataIngestSession API

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Void |  |
| 2 | Void |  |
| 3 | 5MBS2 | This feature indicates the support of the Rel-18 enhancements to the 5G Multicast/Broadcast services. The following functionalities are supported: - Support the provisioning of the Associated Session Identifier to enable 5MBS MOCN Network Sharing scenarios (e.g., MOCN with multiple broadcast MBS sessions transmitting the same content via different Core Networks). - NR RedCap UEs indication information provisioning during MBS Distribution Session creation/update. |
| 4 | MBSErrorHandling | Represents the support of the MBS related error handling procedures. The following functionalities are supported: - Support of the missing MBS Session related error handling procedures to enable end-to-end relaying of errors. - Support MBS Data Ingest Session specific error handling. - Support partial MBS Distribution Session creation/update failure management. |

### 5.27.7	Error handling

#### 5.27.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.27.7.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the MBSUserDataIngestSession API.

#### 5.27.7.3	Application Errors

The application errors defined for the MBSUserDataIngestSession API are listed in table 5.27.7.3-1.

Table 5.27.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |
| INVALID_MBS_SERVICE_INFO | 400 Bad Request | Indicates that the provided MBS Service Information is invalid (e.g. invalid QoS reference), incorrect or insufficient to perform MBS policy authorization. | MBSErrorHandling |
| MBS_SERVICE_AREA_NOT_SUPPORTED | 403 Forbidden | Indicates that the requested MBS Service Area is not supported by the 3GPP Core Network. | MBSErrorHandling |
| MBS_SERVICE_INFO_NOT_AUTHORIZED | 403 Forbidden | Indicates the provided MBS Service Information is rejected. | MBSErrorHandling |
| MBS_DIST_SESSION_ALREADY_CREATED | 403 Forbidden | Indicates that the requested MBS Distribution Session has already been created. | MBSErrorHandling |
| OVERLAPPING_MBS_SERVICE_AREA | 403 Forbidden | Indicates that the provided MBS service area overlaps with the MBS service area of an existing MBS Distribution Session that shares the same MBS session Identifier. | MBSErrorHandling |
| UNKNOWN_MBS_SERVICE_AREA | 404 Not Found | Indicates that the requested MBS service area (e.g., identified by the Area Session ID) cannot be found. | MBSErrorHandling |

## 5.28	MSEventExposure API

### 5.28.1	Introduction

The Nnef_MSEventExposure service shall use the MSEventExposure API.

The API URI of MSEventExposure API shall be:

- {apiRoot}/3gpp-ms-event-exposure/v1

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-ms-event-exposure".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.28.2	Resources

#### 5.28.2.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.28.2.1-1 and the resources and HTTP methods used for the MSEventExposure API.

Figure 5.28.2.1-1: Resource URI structure of the MSEventExposure API

Table 5.28.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.28.2.1-1: Resources and methods overview

| Resource name | Resource URI (relative path under API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Media Streaming Event Exposure Subscriptions | /subscriptions | GET | Reads all the existing Media Streaming Event Exposure Subscriptions. |
|  |  | POST | Request the creation of a Media Streaming Event Subscription. |
| Individual Media Sreaming Event Exposure Subscription | /subscriptions/{subscriptionId} | GET | Read an existing Individual Media Streaming Event Exposure Subscription resource. |
|  |  | PUT | Update an existing Individual Media Streaming Event Exposure Subscription. |
|  |  | DELETE | Delete an existing Individual Media Streaming Event Exposure Subscription. |

#### 5.28.2.2	Resource: Media Streaming Event Exposure Subscriptions

##### 5.28.2.2.1	Introduction

This resource represents the collection of Media Streaming Event Exposure Subscription resources managed by the NEF.

This resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [3]).

##### 5.28.2.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-ms-event-exposure/v1/subscriptions

This resource shall support the resource URI variables defined in table 5.28.2.2.2-1.

Table 5.28.2.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.28.1. |

##### 5.28.2.2.3	Resource Standard Methods

###### 5.28.2.2.3.1	GET

This method allows an AF to retrieve all the active Media Streaming Event Exposure Subscription resource at the NEF.

This method shall support the URI query parameters specified in table 5.28.2.2.3.1-1.

Table 5.28.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.28.2.2.3.1-2 and the response data structures and response codes specified in table 5.28.2.2.3.1-3.

Table 5.28.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.28.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(AfEventExposureSubsc) | M | 0..N | 200 OK | Successful case. All the active Media Streaming Event Exposure Subscriptions managed by the NEF are returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status code for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.28.2.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.28.2.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.28.2.2.3.2	POST

This method enables an AF to request the creation of a new Media Streaming Event Exposure Subscription at the NEF.

This method shall support the URI query parameters specified in table 5.28.2.2.3.2-1.

Table 5.28.2.2.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.28.2.2.3.2-2 and the response data structures and response codes specified in table 5.28.2.2.3.2-3.

Table 5.28.2.2.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AfEventExposureSubsc | M | 1 | Contains the parameters to request the creation of a new Media Streaming Event Exposure Subscription at the NEF. |

Table 5.28.2.2.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| AfEventExposureSubsc | M | 1 | 201 Created | Successful case. A new Media Streaming Event Exposure Subscription is successfully created and a representation of the created Individual Media Streaming Event Exposure Subscription resource is returned. An HTTP "Location" header that contains the resource URI of the created Individual Media Streaming Event Exposure Subscription resource shall also be included. |
| NOTE: The mandatory HTTP error status code for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.28.2.2.3.2-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-ms-event-exposure/v1/subscriptions/{subscriptionId} |

##### 5.28.2.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 5.28.2.3	Resource: Individual Media Streaming Event Exposure Subscription

##### 5.28.2.3.1	Introduction

This resource represents an Individual Media Streaming Event Exposure Subscription managed by the NEF.

This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [3]).

##### 5.28.2.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-ms-event-exposure/v1/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 5.28.2.3.2-1.

Table 5.28.2.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.28.1. |
| subscriptionId | string | Contains the unique identifier of the Individual Media Streaming Event Exposure Subscription resource assigned by the NEF. |

##### 5.28.2.3.3	Resource Standard Methods

###### 5.28.2.3.3.1	GET

This method allows an AF to retrieve an existing Individual Media Streaming Event Exposure Subscription resource at the NEF.

This method shall support the URI query parameters specified in table 5.28.2.3.3.1-1.

Table 5.28.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.28.2.3.3.1-2 and the response data structures and response codes specified in table 5.28.2.3.3.1-3.

Table 5.28.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.28.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AfEventExposureSubsc | M | 1 | 200 OK | Successful case. The requested Individual Media Streaming Event Exposure Subscription resource is successfully returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status code for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.28.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.28.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.28.2.3.3.2	PUT

This method enables an AF to request the update of an existing Individual Media Streaming Event Exposure Subscription resource at the NEF.

This method shall support the URI query parameters specified in table 5.28.2.3.3.2-1.

Table 5.28.2.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.28.2.3.3.2-2 and the response data structures and response codes specified in table 5.28.2.3.3.2-3.

Table 5.28.2.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AfEventExposureSubsc | M | 1 | Contains the updated representation of the Individual Media Streaming Event Exposure Subscription resource that is to be updated. |

Table 5.28.2.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AfEventExposureSubsc | M | 1 | 200 OK | Successful case. The concerned Individual Media Streaming Event Exposure Subscription resource is successfully updated and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The concerned Individual Media Streaming Event Exposure Subscription resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status code for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.28.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.28.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

###### 5.28.2.3.3.3	DELETE

This method enables an AF to request the deletion of an existing Individual Media Streaming Event Exposure resource at the NEF.

This method shall support the URI query parameters specified in table 5.28.2.3.3.3-1.

Table 5.28.2.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.28.2.3.3.3-2 and the response data structures and response codes specified in table 5.28.2.3.3.3-3.

Table 5.28.2.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.28.2.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful response. The Individual Media Streaming Event Exposure resource is successfully deleted. |
|  |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
|  |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status code for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.28.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.28.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

##### 5.28.2.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

### 5.28.3	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.28.4	Notifications

#### 5.28.4.1	General

Notifications shall comply to clause 5.2.5 of 3GPP TS 29.122 [4].

Table 5.28.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Media Streaming Event Exposure Notification | {notifUri} | POST | This operation enables the NEF to notify a previously subscribed AF on the Media Streaming Exposure Event(s). |

#### 5.28.4.2	Media Streaming Event Exposure Notification

##### 5.28.4.2.1	Description

The Media Streaming Event Exposure notification is used by the NEF to report one or several observed Media Streaming event(s) to a previously subscribed AF.

##### 5.28.4.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 5.28.4.2.2-1.

Table 5.28.4.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | Callback reference provided by the AF during the creation/update of the corresponding Media Streaming Event Exposure Subscription. |

##### 5.28.4.2.3	Operation Definition

###### 5.28.4.2.3.1	Notification via HTTP POST

This method shall support the request data structures specified in table 5.28.4.2.3.1-1 and the response data structures and response codes specified in table 5.28.4.2.3.1-2.

Table 5.28.4.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AfEventExposureNotif | M | 1 | Contains the Media Streaming Event Exposure Notification. |

Table 5.28.4.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The Media Streaming Event Notification is successfully received. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.28.4.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

Table 5.28.4.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

###### 5.28.4.2.3.2	Void

### 5.28.5	Data Model

#### 5.28.5.1	General

This clause specifies the application data model supported by the MSEventExposure API. Table 5.28.5.1-1 specifies the data types defined for the MSEventExposure API.

Table 5.28.5.1-1: MSEventExposure specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |

Table 5.28.5.1-2 specifies data types re-used by the MSEventExposure API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the MSEventExposure API.

Table 5.28.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AfEventExposureNotif | 3GPP TS 29.517 [58] | Represents a Media Streaming event notification. |  |
| AfEventExposureSubsc | 3GPP TS 29.517 [58] | Represents a Media Streaming event exposure subscription. Only applicable to the UE application events exposed via Data Collection AF as defined in clause 4.1.1 of 3GPP TS 29.517 [58]. |  |

#### 5.28.5.2	Structured data types

##### 5.28.5.2.1	Introduction

This clause defines the structures to be used in resource representations.

There are no structured data types defined for this API in this release of the specification.

#### 5.28.5.3	Simple data types and enumerations

##### 5.28.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.28.5.3.2	Simple data types

The simple data types defined in table 5.28.5.3.2-1 shall be supported.

Table 5.28.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

### 5.28.6	Used Features

The table below defines the features applicable to the MSEventExposure API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.28.6-1: Features used by MSEventExposure API

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 5.28.7	Error handling

#### 5.28.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses shall apply.

#### 5.28.7.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the MSEventExposure API.

#### 5.28.7.3	Application Errors

The application errors defined for the MSEventExposure API are listed in table 5.28.7.3-1.

Table 5.28.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

## 5.29	MBSGroupMsgDelivery API

### 5.29.1	Introduction

The Nnef_MBSGroupMsgDelivery service shall use the MBSGroupMsgDelivery API.

The API URI of the MBSGroupMsgDelivery API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [4], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-mbs-group-msg".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [4].

All resource URIs in the clauses below are defined relative to the above root URI.

### 5.29.2	Resources

#### 5.29.2.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.29.2.1-1 and HTTP methods used for the MBSGroupMsgDelivery API.

The structure of the resource URIs of the MBSGroupMsgDelivery API is shown in Figure 5.29.2.1-1.

Figure 5.29.2.1-1: Resource URI structure of the MBSGroupMsgDelivery API

Table 5.29.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.29.2.1-1: Resources and methods overview

| Resource name | Resource URI (relative path under API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| MBS Group Message Deliveries | /deliveries | GET | Retrieve the existing MBS Group Message Deliveries. |
|  |  | POST | Request the creation of a new MBS Group Message Delivery. |
| Individual MBS Group Message Delivery | /deliveries/{delRef} | GET | Retrieve a previously submitted MBS Group Message Delivery. |
|  |  | PATCH | Request the modification of a previously submitted MBS Group Message Delivery. |
|  |  | DELETE | Request the deletion of a previously submitted MBS Group Message Delivery. |

#### 5.29.2.2	Resource: MBS Group Message Deliveries

##### 5.29.2.2.1	Introduction

This resource represents the collection of MBS Group Message Deliveries managed by the NEF.

This resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [3]).

##### 5.29.2.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-mbs-group-msg/v1/deliveries

This resource shall support the resource URI variables defined in table 5.29.2.2.2-1.

Table 5.29.2.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.29.1. |

##### 5.29.2.2.3	Resource Standard Methods

###### 5.29.2.2.3.1	GET

This method allows an AF to retrieve all the active MBS Group Message Deliveries managed by the NEF.

This method shall support the URI query parameters specified in table 5.29.2.2.3.1-1.

Table 5.29.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.29.2.2.3.1-2 and the response data structures and response codes specified in table 5.29.2.2.3.1-3.

Table 5.29.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.29.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(MbsGroupMsgDel) | M | 0..N | 200 OK | Successful case. All the active MBS Group Message Deliveries managed by the NEF shall be returned. If there are no active MBS Group Message Delivery(ies) available at the NEF, an empty array shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.29.2.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

Table 5.29.2.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

###### 5.29.2.2.3.2	POST

This method enables an AF to request the creation of an MBS Group Message Delivery at the NEF.

This method shall support the URI query parameters specified in table 5.29.2.2.3.2-1.

Table 5.29.2.2.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This operation shall support the request and response data structures and response codes specified in table 5.29.2.2.3.2-2 and table 5.29.2.2.3.2-3.

Table 5.29.2.2.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MbsGroupMsgDel | M | 1 | Represents the Group Message Delivery to be created at the NEF. |

Table 5.29.2.2.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MbsGroupMsgDel | M | 1 | 201 Created | Successful case: A representation of the created "Individual MBS Group Message Delivery" resource is returned in the response body. An HTTP "Location" header that contains the URI of the created "Individual MBS Group Messge Delivery" resource shall also be included. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. NOTE 2: Failure cases are described in clause 5.29.7. |  |  |  |  |

Table 5.29.2.2.3.2-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-mbs-group-msg/v1/deliveries/{delRef} |

##### 5.29.2.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 5.29.2.3	Resource: Individual MBS Group Message Delivery

##### 5.29.2.3.1	Introduction

This resource represents an "Individual MBS Group Message Delivery" resource managed by the NEF.

This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [3]).

##### 5.29.2.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-mbs-group-msg/v1/deliveries/{delRef}

This resource shall support the resource URI variables defined in table 5.29.2.3.2-1.

Table 5.29.2.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.29.1. |
| delRef | string | Contains the identifier of the "Individual MBS Group Message Delivery" resource assigned by the NEF. |

##### 5.29.2.3.3	Resource Standard Methods

###### 5.29.2.3.3.1	GET

This method allows an AF to retrieve an existing "Individual MBS Group Message Delivery" resource at the NEF.

This method shall support the URI query parameters specified in table 5.29.2.3.3.1-1.

Table 5.29.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.29.2.3.3.1-2 and the response data structures and response codes specified in table 5.29.2.3.3.1-3.

Table 5.29.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.29.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MbsGroupMsgDel | M | 1 | 200 OK | Successful case. The requested "Individual MBS Group Message Delivery" resource is successfully returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.29.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

Table 5.29.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

###### 5.29.2.3.3.2	PATCH

This method enables an AF to request the modification of an existing "Individual MBS Group Message Delivery" resource at the NEF.

This method shall support the URI query parameters specified in table 5.29.2.3.3.2-1.

Table 5.29.2.3.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.29.2.3.3.2-2 and the response data structures and response codes specified in table 5.29.2.3.3.2-3.

Table 5.29.2.3.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MbsGroupMsgDelPatch | M | 1 | Contains the parameters to request the modification of the "Individual MBS Group Message Delivery" resource. |

Table 5.29.2.3.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MbsGroupMsgDel | M | 1 | 200 OK | Successful case. The concerned "Individual MBS Group Message Delivery" resource is successfully modified and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful response. The "Individual MBS Group Message Delivery" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. NOTE 2: Failure cases are described in clause 5.29.7. |  |  |  |  |

Table 5.29.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

Table 5.29.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

###### 5.29.2.3.3.3	DELETE

This method enables an AF to request the deletion of an existing "Individual MBS Group Message Delivery" resource at the NEF.

This method shall support the URI query parameters specified in table 5.29.2.3.3.3-1.

Table 5.29.2.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.29.2.3.3.3-2 and the response data structures and response codes specified in table 5.29.2.3.3.3-3.

Table 5.29.2.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.29.2.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful response. The "Individual MBS Group Message Delivery" resource is successfully deleted. |
|  |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
|  |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.29.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

Table 5.29.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

##### 5.29.2.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

### 5.29.3	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.29.4	Notifications

#### 5.29.4.1	General

Notifications shall comply to clause 5.2.5 of 3GPP TS 29.122 [4].

Table 5.29.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| MBS Group Message Delivery Notification | {notifUri} | POST | Enables the NEF to notify a previously subscribed AF of the status of an MBS Group Message Delivery. |

#### 5.29.4.2	MBS Group Message Delivery Notification

##### 5.29.4.2.1	Description

This notification is used by the NEF to report the status of an MBS Group Message Delivery to a previously subscribed AF.

##### 5.29.4.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 5.29.4.2.2-1.

Table 5.29.4.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | Represents the callback URI encoded as a string formatted as a URI. |

##### 5.29.4.2.3	Operation Definition

This method shall support the request data structures specified in table 5.29.4.2.3-1 and the response data structures and response codes specified in table 5.29.4.2.3-2.

Table 5.29.4.2.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MbsGroupMsgDelStatusNotif | M | 1 | Represents the MBS Group Message Delivery Notification. |

Table 5.29.4.2.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The notification is successfully received and acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.29.4.2.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

Table 5.29.4.2.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

### 5.29.5	Data Model

#### 5.29.5.1	General

This clause specifies the application data model supported by the MBSGroupMsgDelivery API. Table 5.29.5.1-1 specifies the data types defined for the MBSGroupMsgDelivery API.

Table 5.29.5.1-1: MBSGroupMsgDelivery API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| MbsGroupMsgDel | 5.29.5.2.3 | Represents an MBS Group Message Delivery. |  |
| MbsGroupMsgDelPatch | 5.29.5.2.5 | Represents the parameters to request the modification of an existing MBS Group Message Delivery. |  |
| MbsGroupMsgDelStatusNotif | 5.29.5.2.6 | Represents an MBS Group Message Delivery Notification. |  |
| MbsServArea | 5.29.5.4.1 | Represents an MBS Service Area. |  |

Table 5.29.5.1-2 specifies data types re-used by the MBSGroupMsgDelivery API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the MBSGroupMsgDelivery API.

Table 5.29.5.1-2: MBSGroupMsgDelivery API re-used Data Types

| Data type |  | Reference |  | Comments |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bytes |  | 3GPP TS 29.122 [4] |  | Represents a sequence of bytes. |  |  |  |
| DateTime |  | 3GPP TS 29.122 [4] |  | Represents a date and a time. |  |  |  |
| ExternalGroupId |  | 3GPP TS 29.122 [4] |  | Represents an external Group Identifier. |  |  |  |
| ExternalMbsServiceArea |  | 3GPP TS 29.571 [8] |  | Represents an external MBS service area. |  |  |  |
| MbsServiceArea |  | 3GPP TS 29.571 [8] |  | Represents an MBS service area. |  |  |  |
| NrRedCapUeInfo |  | 3GPP TS 29.571 [8] |  | Represents NR RedCap UE Information. |  |  |  |
| ProblemDetails |  | 3GPP TS 29.571 [8] |  | Represents error related information. |  |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [8] |  | Represents the list of supported feature(s) and is used to negotiate the applicability of the optional features. |  |  |  |
| Uri |  | 3GPP TS 29.122 [4] |  | Represents a Uri. |  |  |  |
| UserServiceDescription |  | 3GPP TS 26.517 [71] |  | Represents MBS User Service Announcement information. |  |  |  |

#### 5.29.5.2	Structured data types

##### 5.29.5.2.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.29.5.2.2	Void

##### 5.29.5.2.3	Type: MbsGroupMsgDel

Table 5.29.5.2.3-1: Definition of type MbsGroupMsgDel

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| afId |  | string |  | C |  | 0..1 |  | Contains the identifier of the AF that is sending the request. This attribute shall be present only in MBS Group Message Delivery creation requests. |  |  |  |
| extGroupId |  | ExternalGroupId |  | M |  | 1 |  | Contains the identifier of the targeted group of UEs. |  |  |  |
| payload |  | Bytes |  | M |  | 1 |  | Contains the payload of the requested MBS Group Message Delivery. |  |  |  |
| mbsServArea |  | MbsServArea |  | M |  | 1 |  | Contains the targeted MBS Service Area. |  |  |  |
| startTime |  | DateTime |  | M |  | 1 |  | Contains the start time of the MBS Group Message Delivery. |  |  |  |
| endTime |  | DateTime |  | M |  | 1 |  | Contains the end time of the MBS Group Message Delivery. |  |  |  |
| notifUri |  | Uri |  | M |  | 1 |  | Contains the notification URI via which the AF desires to receive notifications on the status of this MBS Group Message Delivery. |  |  |  |
| delStatus |  | boolean |  | C |  | 0..1 |  | Indicates the status of this MBS Group Message Delivery. - "true": Successful delivery. - "false": Failed delivery. This attribute shall be present only in MBS Group Message Delivery creation/modification responses. |  |  |  |
| mbsUserServAnmt |  | UserServiceDescription |  | C |  | 0..1 |  | Represents the MBS User Service Announcement information currently associated with the MBS group message delivery. This attribute shall be present in the response to an MBS Group Message Delivery Creation request and may be present in the response to an MBS Group Message Delivery Modification request. |  |  |  |
| servAreaWithoutMbs |  | MbsServArea |  | O |  | 0..1 |  | Contains the service area without MBS capability. This attribute may be present only in MBS Group Message Delivery creation/modification responses. |  |  |  |
| nrRedCapUeInfo |  | NrRedCapUeInfo |  | O |  | 0..1 |  | Contains the NR RedCap UE Information and is used to indicate whether the MBS session is intended only for NR RedCap UEs, for both NR RedCap UEs and non-RedCap UEs or only for non-RedCap UEs. |  |  |  |
| suppFeat |  | SupportedFeatures |  | C |  | 0..1 |  | Indicates the list of supported features. This attribute shall be present only when feature negotiation needs to take place. |  |  |  |

##### 5.29.5.2.4	Void

##### 5.29.5.2.5	Type: MbsGroupMsgDelPatch

Table 5.29.5.2.5-1: Definition of type MbsGroupMsgDelPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| payload | Bytes | O | 0..1 | Contains the updated payload of the requested MBS Group Message Delivery. |  |
| mbsServArea | MbsServArea | O | 0..1 | Contains the updated MBS Service Area. |  |
| startTime | DateTime | O | 0..1 | Contains the updated start time of the MBS Group Message Delivery. |  |
| endTime | DateTime | O | 0..1 | Contains the updated end time of the MBS Group Message Delivery. |  |
| notifUri | Uri | O | 0..1 | Contains the updated notification URI via which the AF desires to receive notifications on the status of this MBS Group Message Delivery. |  |
| nrRedCapUeInfo | NrRedCapUeInfo | O | 0..1 | Contains the NR RedCap UE Information and is used to indicate whether the MBS session is intended only for NR RedCap UEs, for both NR RedCap UEs and non-RedCap UEs or only for non-RedCap UEs. |  |

##### 5.29.5.2.6	Type: MbsGroupMsgDelStatusNotif

Table 5.29.5.2.6-1: Definition of type MbsGroupMsgDelStatusNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| delStatus | boolean | M | 1 | Indicate the status of Group Message Delivery. - "true": Successful delivery. - "false": Failed delivery. |  |

#### 5.29.5.3	Simple data types and enumerations

##### 5.29.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.29.5.3.2	Simple data types

The simple data types defined in Table 5.29.5.3.2-1 shall be supported.

Table 5.29.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

#### 5.29.5.4	Data types describing alternative data types or combinations of data types

##### 5.29.5.4.1	Type: MbsServArea

Table 5.29.5.4.1-1: Definition of type MbsServArea as a list of mutually exclusive alternatives

| Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- |
| MbsServiceArea | 0..1 | Represents the internal MBS Service Area (i.e., in the form of a list of cell ID(s) and/or list of TAI(s)). |  |
| ExternalMbsServiceArea | 0..1 | Represents the external MBS Service Area (i.e., in the form of geographical area(s) or civic address(es)). |  |

### 5.29.6	Used Features

The optional features listed in table 5.29.6-1 are defined for the MBSGroupMsgDelivery API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.29.6-1: Features used by the MBSGroupMsgDelivery API

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 5.29.7	Error handling

#### 5.29.7.1	General

For the MBSGroupMsgDelivery API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [4] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses are applicable for the MBSGroupMsgDelivery API.

#### 5.29.7.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the MBSGroupMsgDelivery API.

#### 5.29.7.3	Application Errors

The application errors defined for the MBSGroupMsgDelivery API are listed in table 5.29.7.3-1.

Table 5.29.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |
| MBS_SERVICE_AREA_NOT_SUPPORTED | 403 Forbidden | The MBS Group Message Delivery creation/update request is rejected because the requested MBS Service Area is not supported by the 3GPP Core Network. |  |

## 5.30	DNAIMapping API

### 5.30.1	Introduction

The Nnef_DNAIMapping service shall use the DNAIMapping API.

The API URI of the DNAIMapping API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [4], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-dnai-mapping".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [4].

All resource URIs in the clauses below are defined relative to the above root URI.

### 5.30.2	Resources

#### 5.30.2.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.30.2.1-1 and the resources and HTTP methods used for the DNAIMapping API.

Figure 5.30.2.1-1: Resource URI structure of the DNAIMapping API

Table 5.30.2.1-1 provides an overview of the resources and HTTP methods applicable for the DNAIMapping API.

Table 5.30.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| DNAI Mapping Subscriptions | /{afId}/subscriptions | GET | Retrieve all the active DNAI Mapping Subscriptions at the NEF. |
|  |  | POST | Create a new DNAI Mapping Subscription. |
| Individual DNAI Mapping Subscription | /{afId}/subscriptions/{subscriptionId} | GET | Retrieve an existing DNAI Mapping Subscription. |
|  |  | DELETE | Delete an existing DNAI Mapping Subscription. |

#### 5.30.2.2	Resource: DNAI Mapping Subscriptions

##### 5.30.2.2.1	Introduction

This resource represents all the active DNAI Mapping Subscriptions managed by the NEF for a given AF.

This resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [3]).

##### 5.30.2.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-dnai-mapping/<apiVersion>/{afId}/subscriptions

This resource shall support the resource URI variables defined in table 5.30.2.2.2-1.

Table 5.30.2.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.30.1. |
| afId | string | Represents the identifier of the AF. |

##### 5.30.2.2.3	Resource Standard Methods

###### 5.30.2.2.3.1	Void

###### 5.30.2.2.3.2	GET

The HTTP GET method allows to retrieve all the active DNAI Mapping Subscriptions managed by the NEF for a given AF.

This method shall support the URI query parameters specified in table 5.30.2.2.3.2-1.

Table 5.30.2.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.30.2.2.3.2-2 and the response data structures and response codes specified in table 5.30.2.2.3.2-3.

Table 5.30.2.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.30.2.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(DnaiMapSub) | M | 0..N | 200 OK | Successful case. All the "Individual DNAI Mapping Subscription" resource(s) managed by the NEF are returned. If there are no active "Individual DNAI Mapping Subscription" resources at the NEF, an empty array is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.30.2.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.30.2.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.30.2.2.3.3	POST

The HTTP POST method allows to create a new DNAI Mapping Subscription at the NEF for a given AF.

This method shall support the URI query parameters specified in table 5.30.2.2.3.3-1.

Table 5.30.2.2.3.3-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.30.2.2.3.3-1 and the response data structures and response codes specified in table 5.30.2.2.3.3-2.

Table 5.30.2.2.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DnaiMapSub | M | 1 | Contains the parameters to request the creation of a new DNAI Mapping Subscription at the NEF. |

Table 5.30.2.2.3.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DnaiMapSub | M | 1 | 201 Created | Successful case. A representation of the created "Individual DNAI Mapping Subscription" resource is returned in the response body. The URI of the created resource shall be returned in an HTTP "Location" header. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.30.2.2.3.3-3: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-dnai-mapping/<apiVersion>/{afId}/subscriptions/{subscriptionId} |

##### 5.30.2.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 5.30.2.3	Resource: Individual DNAI Mapping Subscription

##### 5.30.2.3.1	Introduction

This resource represents an "Individual DNAI Mapping Subscription" resource managed by the NEF.

This resource is modelled with the Document resource archetype (see clause C.2 of 3GPP TS 29.501 [3]).

##### 5.30.2.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-dnai-mapping/<apiVersion>/{afId}/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 5.30.2.3.2-1.

Table 5.30.2.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.30.1. |
| afId | string | Represents the identifier of the AF. |
| subscriptionId | string | Represents the identifier of the "Individual DNAI Mapping Subscription" resource. |

##### 5.30.2.3.3	Resource Standard Methods

###### 5.30.2.3.3.1	Void

###### 5.30.2.3.3.2	GET

The HTTP GET method allows to retrieve an existing "Individual DNAI Mapping Subscription" resource at the NEF.

This method shall support the URI query parameters specified in table 5.30.2.3.3.2-1.

Table 5.30.2.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.30.2.3.3.2-2 and the response data structures and response codes specified in table 5.30.2.3.3.2-3.

Table 5.30.2.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.30.2.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DnaiMapSub | M | 1 | 200 OK | Successful case. The requested "Individual DNAI Mapping Subscription" resource is successfully returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.30.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.30.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.30.2.3.3.3	DELETE

The HTTP DELETE method allows to deletes an existing "Individual DNAI Mapping Subscription" resource at the NEF.

This method shall support the URI query parameters specified in table 5.30.2.3.3.3-1.

Table 5.30.2.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.30.2.3.3.3-2 and the response data structures and response codes specified in table 5.30.2.3.3.3-3.

Table 5.30.2.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.30.2.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual DNAI Mapping Subscription" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.30.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.30.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

##### 5.30.2.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

### 5.30.3	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.30.4	Notifications

#### 5.30.4.1	General

Notifications shall comply to clause 5.2.5 of 3GPP TS 29.122 [4].

Table 5.30.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| DNAI Mapping Information Update Notification | {notifUri} | POST | Enbales the NEF to notify a previously subscribed AF on the updates of the DNAI-EAS address(es) information. |

#### 5.30.4.2	DNAI Mapping Information Update Notification

##### 5.30.4.2.1	Description

This notification is used by the NEF to report the updates of the DNAI Mapping information to a previously subscribed AF.

##### 5.30.4.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 5.30.4.2.2-1.

Table 5.30.4.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | Represents the callback URI encoded as a string formatted as a URI. |

##### 5.30.4.2.3	Operation Definition

###### 5.30.4.2.3.1	Notification via HTTP POST

This method shall support the request data structures specified in table 5.30.4.2.3.1-1 and the response data structures and response codes specified in table 5.30.4.2.3.1-2.

Table 5.30.4.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DnaiMapUpdateNotif | M | 1 | Represents the DNAI Mapping Information Update Notification. |

Table 5.30.4.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The notification is successfully received and acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.30.4.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

Table 5.30.4.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

###### 5.30.4.2.3.2	Void

### 5.30.5	Data Model

#### 5.30.5.1	General

This clause specifies the application data model supported by the DNAIMapping API. Table 5.30.5.1-1 specifies the data types defined for the DNAIMapping API.

Table 5.30.5.1-1: DNAIMapping API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| DnaiMapSub | 5.30.5.2.2 | Represents a DNAI Mapping Subscription. |  |
| DnaiMapUpdateNotif | 5.30.5.2.3 | Represents a DNAI Mapping Information Update Notification. |  |

Table 5.30.5.1-2 specifies data types re-used by the DNAIMapping API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the DNAIMapping API.

Table 5.30.5.1-2: DNAIMapping API re-used Data Types

| Data type |  | Reference |  | Comments |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DnaiEasInfo |  | 3GPP TS 29.519 [23] |  | Represents EAS information for a DNAI. |  |  |  |
| Dnn |  | 3GPP TS 29.571 [8] |  | Represents a DNN. |  |  |  |
| Fqdn |  | 3GPP TS 29.571 [8] |  | Represents an FQDN. |  |  |  |
| IpAddr |  | 3GPP TS 29.571 [8] |  | Represents an IP address. |  |  |  |
| ReportingInformation |  | 3GPP TS 29.523 [22] |  | Represents the event reporting requirements. |  |  |  |
| Snssai |  | 3GPP TS 29.571 [8] |  | Represents an S-NSSAI. |  |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [8] |  | Represents the list of supported feature(s) and is used to negotiate the applicability of the optional features. |  |  |  |
| Uri |  | 3GPP TS 29.122 [4] |  | Represents a URI. |  |  |  |

#### 5.30.5.2	Structured data types

##### 5.30.5.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 5.30.5.2.2	Type: DnaiMapSub

Table 5.30.5.2.2-1: Definition of type DnaiMapSub

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| easIpAddrs |  | array(IpAddr) |  | C |  | 1..N |  | Contains the IP address(es) of the EAS(s) in the Local part of the DN or the IP address range(s) (i.e., IPv4 subnetwork(s) and/or IPv6 prefix(es)) of the Local part of the DN where the EAS(s) is/are deployed. (NOTE) |  |  |  |
| fqdns |  | array(Fqdn) |  | C |  | 1..N |  | Contains the FQDN(s) of the EAS(s) in the Local part of the DN where the EAS(s) is/are deployed. (NOTE) |  |  |  |
| dnn |  | Dnn |  | O |  | 0..1 |  | Contains a DNN, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. |  |  |  |
| snssai |  | Snssai |  | O |  | 0..1 |  | Contains an S-NSSAI. |  |  |  |
| immReports |  | array(DnaiMapUpdateNotif) |  | C |  | 1..N |  | Contains the DNAI EAS mapping information. This attribute shall be present only in subscription creation/update responses and only when immediate reporting was requested in the corresponding subscription creation/update request and the information is available. |  |  |  |
| eventReq |  | ReportingInformation |  | O |  | 0..1 |  | Contains the event reporting requirements. |  |  |  |
| notifUri |  | Uri |  | M |  | 1 |  | Contains the URI via which the DNAI Mapping Information Update notifications shall be delivered. |  |  |  |
| notifCorrId |  | string |  | M |  | 1 |  | Contains the notification correlation identifier. |  |  |  |
| suppFeat |  | SupportedFeatures |  | C |  | 0..1 |  | Contains the list of supported features used as defined in clause 5.30.6. This attribute shall be present only when feature negotiation needs to take place. |  |  |  |
| NOTE: These attributes are mutually exclusive. Either one of them shall be present. |  |  |  |  |  |  |  |  |  |  |  |

##### 5.30.5.2.3	Type: DnaiMapUpdateNotif

Table 5.30.5.2.3-1: Definition of type DnaiMapUpdateNotif

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| dnaiEasAddrMap |  | array(DnaiEasInfo) |  | C |  | 1..N |  | Contains the mapping information between the DNAI(s) and the EAS address(es). This attribute shall be present unless the notification is sent to notify deletion. |  |  |  |
| mappingId |  | string |  | M |  | 1 |  | Identifies the mapping information provided in the "dnaiEasAddrMap" attribute. For notifying deletion, this attribute shall be provided without the "dnaiEasAddrMap" attribute. |  |  |  |
| notifCorrId |  | string |  | M |  | 1 |  | Contains the notification correlation identifier. |  |  |  |

##### 5.30.5.2.4	Void

#### 5.30.5.3	Simple data types and enumerations

##### 5.30.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.30.5.3.2	Simple data types

The simple data types defined in table 5.30.5.3.2-1 shall be supported.

Table 5.30.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

#### 5.30.5.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

### 5.30.6	Used Features

The optional features listed in table 5.30.6-1 are defined for the DNAIMapping API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.30.6-1: Features used by DNAIMapping API

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Void |  |
| 2 | Void |  |

### 5.30.7	Error handling

#### 5.30.7.1	General

For the DNAIMapping API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [4] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses are applicable for the DNAIMapping API.

#### 5.30.7.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the DNAIMapping API.

#### 5.30.7.3	Application Errors

The application errors defined for the DNAIMapping API are listed in table 5.30.7.3-1.

Table 5.30.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

## 5.31	PdtqPolicyNegotiation API

### 5.31.0	Introduction

The Nnef_PdtqPolicyNegotiation service shall use the PdtqPolicyNegotiation API.

The API URI of PdtqPolicyNegotiation API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [4], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-pdtq-policy-negotiation".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [4].

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.31.1	Resources

#### 5.31.1.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.29.2.1-1 and HTTP methods used for the PdtqPolicyNegotiation API.

The structure of the resource URIs of the PdtqPolicyNegotiation API is shown in Figure 5.31.1.1-1.

Figure 5.31.1.1-1: Resource URI structure of the PdtqPolicyNegotiation API

Table 5.31.1.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.31.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| PDTQ Policy Subscriptions | /{afId}/subscriptions | GET | Read all the active PDTQ Policy Subscription resources for a given AF. |
|  |  | POST | Create a new PDTQ Policy Subscription. |
| Individual PDTQ Policy Subscription | /{afId}/subscriptions/{subscriptionId} | GET | Retrieve an existing "Individual PDTQ Policy Subscription" resource. |
|  |  | PATCH | Modify an existing "Individual PDTQ Policy Subscription" resource. |
|  |  | DELETE | Delete an existing "Individual PDTQ Policy Subscription" resource. |

#### 5.31.1.2	Resource: PDTQ Policy Subscriptions

##### 5.31.1.2.1	Introduction

This resource represents all the active PDTQ Policy Subscriptions managed by the NEF for a given AF.

This resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [3]).

##### 5.31.1.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-pdtq-policy-negotiation/<apiVersion>/{afId}/subscriptions

This resource shall support the resource URI variables defined in table 5.31.1.2.2-1.

Table 5.31.1.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.31.0. |
| afId | string | Represents the identifier of the AF. |

##### 5.31.1.2.3	Resource Standard Methods

###### 5.31.1.2.3.1	Void

###### 5.31.1.2.3.2	GET

The HTTP GET method allows to retrieve all the active PDTQ Policy Subscriptions managed by the NEF.

This method shall support the URI query parameters specified in table 5.31.1.2.3.2-1.

Table 5.31.1.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.31.1.2.3.2-2 and the response data structures and response codes specified in table 5.31.1.2.3.2-3.

Table 5.31.1.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.31.1.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(Pdtq) | M | 0..N | 200 OK | Successful case. All the "Individual PDTQ Policy Subscription" resource(s) managed by the NEF are returned. If there are no active "Individual PDTQ Policy Subscription" resources at the NEF, an empty array is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.31.1.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.31.1.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.31.1.2.3.3	POST

The HTTP POST method allows to create a new PDTQ Policy Subscription at the NEF.

This method shall support the URI query parameters specified in table 5.31.1.2.3.3-1.

Table 5.31.1.2.3.3-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.31.1.2.3.3-2 and the response data structures and response codes specified in table 5.31.1.2.3.3-3.

Table 5.31.1.2.3.3-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| Pdtq | M | 1 | Contains the parameters to request the creation of a new PDTQ Policy Subscription at the NEF. |

Table 5.31.1.2.3.3-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| Pdtq | M | 1 | 201 Created | Successful case. A representation of the created "Individual PDTQ Policy Subscription" resource is returned in the response body. The URI of the created resource shall be returned in an HTTP "Location" header. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.31.1.2.3.3-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-pdtq-policy-negotiation/<apiVersion>/{afId}/subscriptions/{subscriptionId} |

##### 5.31.1.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 5.31.1.3	Resource: Individual PDTQ Policy Subscription

##### 5.31.1.3.1	Introduction

This resource represents an "Individual PDTQ Policy Subscription" resource managed by the NEF.

This resource is modelled with the Document resource archetype (see clause C.2 of 3GPP TS 29.501 [3]).

##### 5.31.1.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-pdtq-policy-negotiation/<apiVersion>/{afId}/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 5.31.1.3.2-1.

Table 5.31.1.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.31.0. |
| afId | string | Represents the identifier of the AF. |
| subscriptionId | string | Represents the identifier of the "Individual PDTQ Policy Subscription" resource. |

##### 5.31.1.3.3	Resource Standard Methods

###### 5.31.1.3.3.1	Void

###### 5.31.1.3.3.2	GET

The HTTP GET method allows to retrieve an existing "Individual PDTQ Policy Subscription" resource at the NEF.

This method shall support the URI query parameters specified in table 5.31.1.3.3.2-1.

Table 5.31.1.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.31.1.3.3.2-2 and the response data structures and response codes specified in table 5.31.1.3.3.2-3.

Table 5.31.1.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.31.1.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| Pdtq | M | 1 | 200 OK | Successful case. The requested "Individual PDTQ Policy Subscription" resource is successfully returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.31.1.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.31.1.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.31.1.3.3.3	PATCH

The HTTP PATCH method allows to modify an existing "Individual PDTQ Policy Subscription" resource at the NEF.

This method shall support the URI query parameters specified in table 5.31.1.3.3.3-1.

Table 5.31.1.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.31.1.3.3.3-2 and the response data structures and response codes specified in table 5.31.1.3.3.3-3.

Table 5.31.1.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PdtqPatch | M | 1 | Represents the requested modifications to the "Individual PDTQ Policy Subscription" resource. |

Table 5.31.1.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| Pdtq | M | 1 | 200 OK | Successful response. The "Individual PDTQ Policy Subscription " resource is successfully modified and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful response. The "Individual PDTQ Policy Subscription" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.31.1.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.31.1.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.31.1.3.3.4	DELETE

The HTTP DELETE method allows to delete an existing "Individual PDTQ Policy Subscription" resource at the NEF.

This method shall support the URI query parameters specified in table 5.31.1.3.3.4.-1.

Table 5.31.1.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.31.1.3.3.4-2 and the response data structures and response codes specified in table 5.31.1.3.3.4-3.

Table 5.31.1.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.31.1.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual PDTQ Policy Subscription" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.31.1.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.31.1.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

##### 5.31.1.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

### 5.31.1A	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.31.2	Notifications

#### 5.31.2.1	General

Notifications shall comply to clause 5.2.5 of 3GPP TS 29.122 [4].

Table 5.31.2.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| PDTQ Warning Notification | {notifUri} | POST | Enables the NEF to notify a previously subscribed AF on PDTQ warning event(s). |

#### 5.31.2.2	PDTQ Warning Notification

##### 5.31.2.2.1	Description

This notification is used by the NEF to notify a previously subscribed AF on PDTQ warning event(s), i.e., the changed conditions for a planned data transfer with QoS requirements (e.g., a network performance or DN performance in the area of interest reaches the criteria set by the operator).

##### 5.31.2.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 5.31.2.2.2-1.

Table 5.31.2.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | Represents the callback URI encoded as a string formatted as a URI. |

##### 5.31.2.2.3	Operation Definition

###### 5.31.2.2.3.1	Notification via HTTP POST

This method shall support the request data structures specified in table 5.31.2.2.3.1-1 and the response data structures and response codes specified in table 5.31.2.2.3.1-2.

Table 5.31.2.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PdtqNotification | M | 1 | Represents the PDTQ Warning Notification. |

Table 5.31.2.2.3.1-2: Data structures supported by the POST Response Body

| Data type | Cardinality | Response codes | Description |
| --- | --- | --- | --- |
| n/a |  | 204 No Content | Successful case. The notification is successfully received and acknowledged. |
| n/a |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |

Table 5.31.2.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

Table 5.31.2.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

### 5.31.3	Data Model

#### 5.31.3.1	General

This clause specifies the application data model supported by the PdtqPolicyNegotiation API. Table 5.31.3.1-1 specifies the data types defined for the PdtqPolicyNegotiation API.

Table 5.31.3.1-1: PdtqPolicyNegotiation API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| Pdtq | 5.31.3.3.2 | Represents a PDTQ Policy Subscription. |  |
| PdtqNotification | 5.31.3.3.4 | Represents a PDTQ Warning Notification. |  |
| PdtqPatch | 5.31.3.3.3 | Represents the requested modifications to a PDTQ Policy Subscription. |  |

Table 5.31.3.1-2 specifies data types re-used by the PdtqPolicyNegotiation API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the PdtqPolicyNegotiation API.

Table 5.31.3.1-2: PdtqPolicyNegotiation API re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| AltQosParamSet | 3GPP TS 29.543 [68] | Represents the alternative QoS requirements expressed as a list of individual QoS parameters sets. |
| ApplicationId | 3GPP TS 29.571 [8] | Represents the application identifier. |
| LocationArea5G | 3GPP TS 29.122 [4] | Represents the user location area information. |
| PdtqPolicy | 3GPP TS 29.543 [68] | Represents a PDTQ policy. |
| PdtqReferenceId | 3GPP TS 29.543 [68] | Represents a PDTQ Reference ID. |
| QosParamSet | 3GPP TS 29.543 [68] | Represents the QoS requirements expressed as one or more individual QoS parameters. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Represents the list of supported feature(s) and is used to negotiate the applicability of the optional features. |
| TimeWindow | 3GPP TS 29.122 [4] | Represents a time window. |
| Uri | 3GPP TS 29.122 [4] | Represents a URI. |

#### 5.31.3.2	Void

#### 5.31.3.3	Structured data types

##### 5.31.3.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.31.3.3.2	Type: Pdtq

Table 5.31.3.3.2-1: Definition of type Pdtq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| altQosParamSets | array(AltQosParamSet) | O | 1..N | Contains the alternative QoS requirements expressed as a list of individual QoS parameters sets in a prioritized order. The lower the index of the array for a given entry, the higher the priority. (NOTE 4) |  |
| altQosRefs | array(string) | O | 1..N | Contains the alternative QoS requirements expressed as a list of QoS References in a prioritized order. The lower the index of the array for a given entry, the higher the priority. (NOTE 3) |  |
| appId | ApplicationId | O | 0..1 | Contains the identifier of the application. |  |
| aspId | string | M | 1 | Contains the identifier of the application service provider. |  |
| desTimeInts | array(TimeWindow) | M | 1..N | Contains the time interval(s). |  |
| locationArea5G | LocationArea5G | O | 0..1 | Contains the area within which the AF request shall apply. |  |
| notifUri | Uri | O | 0..1 | Contains the URI via which the PDTQ warning notifications should be delivered. |  |
| numberOfUEs | integer | M | 1 | Contains the number of UEs. |  |
| pdtqPolicies | array(PdtqPolicy) | O | 1..N | Contains the PDTQ policies. |  |
| qosParamSet | QosParameterSet | C | 0..1 | Contains the requested QoS requirements expressed as one or more individual QoS parameters. (NOTE 2) |  |
| qosReference | string | C | 0..1 | Contains the requested QoS requirements expressed as a QoS Reference. (NOTE 2) |  |
| referenceId | PdtqReferenceId | O | 0..1 | Contains the identifier of the PDTQ policies of planned data transfer with QoS requirements for an AF. |  |
| selectedPolicy | integer | O | 0..1 | Contains the identity of the selected PDTQ policy. This attribute shall not be present during resource creation, it may be provided only in a subsequent resource update. (NOTE 5) |  |
| warnNotifEnabled | boolean | O | 0..1 | Indicates whether the PDTQ warning notification is enabled or not. - "true" indicates that the PDTQ warning notification is enabled. - "false" indicates that the PDTQ warning notification is disabled. - The default value is "false" when this attribute is omitted. |  |
| suppFeat | SupportedFeatures | O | 0..1 | Contains the list of supported features among the ones defined in clause 5.31.4. |  |
| NOTE 1: Void. NOTE 2: These attributes are mutually exclusive. Either one of them shall be present. NOTE 3: This attribute may be present only if the "qosReference" attribute is present. NOTE 4: This attribute may be presentonly if the "qosParamSet" attribute is present. NOTE 5: When this attribute is present and set to the value "0", it indicates that no PDTQ policy is selected. |  |  |  |  |  |

##### 5.31.3.3.3	Type: PdtqPatch

Table 5.31.3.3.3-1: Definition of type PdtqPatch

| Attribute name |  | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- | --- |
| selectedPolicy |  | integer | O | 0..1 | Contains the identifier of the selected PDTQ policy. (NOTE) |  |
| warnNotifEnabled |  | boolean | O | 0..1 | Indicates whether the PDTQ warning notification is enabled or not. - "true" indicates that the PDTQ warning notification is enabled; - "false" indicates that the PDTQ warning notification is not enabled. |  |
| notifUri |  | Uri | O | 0..1 | Contains the updated URI via which the PDTQ warning notifications should be delivered. |  |
| NOTE: When this attribute is present and set to the value "0", it indicates that no PDTQ policy is selected. |  |  |  |  |  |  |

##### 5.31.3.3.4	Type: PdtqNotification

Table 5.31.3.3.4-1: Definition of type Notification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pdtqRefId | PdtqReferenceId | M | 1 | Contains the identifier of the PDTQ policy to which the notification corresponds. |  |
| candPolicies | array(PdtqPolicy) | M | 1..N | Contains a list of the candidate PDTQ policies from which the AF may select a new PDTQ policy. |  |

#### 5.31.3.4	Simple data types and enumerations

##### 5.31.3.4.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.31.3.4.2	Simple data types

The simple data types defined in table 5.31.3.4.2-1 shall be supported.

Table 5.31.3.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

#### 5.31.3.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

### 5.31.4	Used Features

The optional features listed in table 5.31.4-1 are defined for the PdtqPolicyNegotiation API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.31.4-1: Features used by PdtqPolicyNegotiation API

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 5.31.5	Error handling

#### 5.31.5.1	General

For the PdtqPolicyNegotiation API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [4] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses are applicable for the PdtqPolicyNegotiation API.

#### 5.31.5.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the PdtqPolicyNegotiation API.

#### 5.31.5.3	Application Errors

The application errors defined for PdtqPolicyNegotiation API are listed in table 5.31.5.3-1.

Table 5.31.5.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

## 5.32	MemberUESelectionAssistance API

### 5.32.1	Introduction

The Nnef_MemberUESelectionAssistance service shall use the MemberUESelectionAssistance API.

The API URI of MemberUESelectionAssistance API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [2], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-musa".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [4].

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.32.2	Resources

#### 5.32.2.1	Overview

This clause describes the structure for the Resource URIs as shown in Figure 5.32.2.1-1 and the resources and HTTP methods used for the MemberUESelectionAssistance API.

Figure 5.32.2.1-1: Resource URI structure of the MemberUESelectionAssistance API

Table 5.32.2.1-1 provides an overview of the resources and applicable HTTP methods applicable for the MemberUESelectionAssistance API.

Table 5.32.2.1-1: Resources and methods overview

| Resource name | Resource URI (relative path under API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Member UE Selection Assistance Subscriptions | /{afId}/subscriptions | POST | Create a new Member UE Selection Assistance Subscription. |
|  |  | GET | Retreive all the active Member UE Selection Assistance Subscription resources. |
| Individual Member UE Selection Assistance Subscription | /{afId}/subscriptions/{subscriptionId} | GET | Retreive an existing "Individual Member UE Selection Assistance Subscription" resource. |
|  |  | PUT | Update an existing "Individual Member UE Selection Assistance Subscription" resource. |
|  |  | PATCH | Modify an existing "Individual Member UE Selection Assistance Subscription" resource. |
|  |  | DELETE | Delete an existing "Individual Member UE Selection Assistance Subscription" resource. |

#### 5.32.2.2	Resource: Member UE Selection Assistance Subscriptions

##### 5.32.2.2.1	Introduction

This resource represents all the active Member UE Selection Assistance Subscriptions managed by the NEF for a given AF.

This resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [3]).

##### 5.32.2.2.2	Resource definition

Resource URL: {apiRoot}/3gpp-musa/<apiVersion>/{afId}/subscriptions

This resource shall support the resource URL variables defined in table 5.32.2.2.2-1.

Table 5.32.2.2.2-1: Resource URL variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.32.1. |
| afId | string | Represents the identifier of the AF. |

##### 5.32.2.2.3	Resource Standard Methods

###### 5.32.2.2.3.1	POST

The HTTP POST method enables to request the creation of a new Member UE Selection Assistance Subscription at the NEF.

This method shall support the URI query parameters specified in table 5.32.2.2.3.1-1.

Table 5.32.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.32.2.2.3.1-2 and the response data structures and response codes specified in table 5.32.2.2.3.1-3.

Table 5.32.2.2.3.1-2: Data structures supported by the POST request body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MemUeSelectAssistSubsc | M | 1 | Contains the parameters to request the creation of a new Member UE Selection Assistance Subscription at the NEF. |

Table 5.32.2.2.3.1-3: Data structures supported by the POST response body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MemUeSelectAssistSubsc | M | 1 | 201 Created | Successful case. A representation of the created "Individual Member UE Selection Assistance Subscription" resource is returned in the response body. The URI of the created resource shall be returned in an HTTP "Location" header. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.32.2.2.3.1-4: Headers supported by the 201 response code on this resource

| HTTP response header | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | The URI of the newly created resource, according to the structure: {apiRoot}/3gpp-musa/<apiVersion>/{afId}/subscriptions/{subscriptionId} |

###### 5.32.2.2.3.2	GET

The HTTP GET method enables to retrieve all the active Member UE Selection Assistance Subscriptions managed by the NEF.

This method shall support the URI query parameters specified in table 5.32.2.2.3.2-1.

Table 5.32.2.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.32.2.2.3.2-2 and the response data structures and response codes specified in table 5.32.2.2.3.2-3.

Table 5.32.2.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.32.2.2.3.2-3-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(MemUeSelectAssistSubsc) | M | 0..N | 200 OK | Successful case. All the "Individual Member UE Selection Assistance Subscription" resource(s) managed by the NEF are returned. If there are no active "Individual Member UE Selection Assistance Subscription" resources at the NEF, an empty array is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.32.2.2.3.2-3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.32.2.2.3.2-3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

##### 5.32.2.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 5.32.2.3	Resource: Individual Member UE Selection Assistance Subscription

##### 5.32.2.3.1	Introduction

This resource represents an "Individual Member UE Selection Assistance Subscription" resource managed by the NEF.

This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [3]).

##### 5.32.2.3.2	Resource Definition

Resource URL: {apiRoot}/3gpp-musa/<apiVersion>/{afId}/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 5.32.2.3.2-1.

Table 5.32.2.3.2-1: Resource URL variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.32.1. |
| afId | string | Represents the identifier of the AF. |
| subscriptionId | string | Represents the identifier of the "Individual Member UE Selection Assistance Subscription" resource. |

##### 5.32.2.3.3	Resource Standard Methods

###### 5.32.2.3.3.1	GET

The HTTP GET method enables to retrieve an existing "Individual Member UE Selection Assistance Subscription" resource at the NEF.

This method shall support the URI query parameters specified in table 5.32.2.3.3.1-1.

Table 5.32.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.32.2.3.3.1-2 and the response data structures and response codes specified in table 5.32.2.3.3.1-3.

Table 5.32.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.32.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MemUeSelectAssistSubsc | M | 1 | 200 OK | Successful case. The requested "Individual Member UE Selection Assistance Subscription" resource is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.32.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.32.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.32.2.3.3.2	PUT

The HTTP PUT method enables to request the update of an existing "Individual Member UE Selection Assistance Subscription" resource at the NEF.

This method shall support the URI query parameters specified in table 5.32.2.3.3.2-1.

Table 5.32.2.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.32.2.3.3.2-2 and the response data structures and response codes specified in table 5.32.2.3.3.2-4.

Table 5.32.2.3.3.2-2: Data structures supported by the PUT request body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MemUeSelectAssistSubsc | M | 1 | Contains the updated "Individual Member UE Selection Assistance Subscription" resource representation. |

Table 5.32.2.3.3.2-3: Data structures supported by the PUT response body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MemUeSelectAssistSubsc | M | 1 | 200 OK | Successful response. The "Individual Member UE Selection Assistance Subscription" resource is successfully updated and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful response. The "Individual Member UE Selection Assistance Subscription" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.32.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.32.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.32.2.3.3.3	PATCH

This method enables to request the modification of an existing "Individual Member UE Selection Assistance Subscription" resource at the NEF.

This method shall support the URI query parameters specified in table 5.32.2.3.3.3-1.

Table 5.32.2.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.32.2.3.3.3-2 and the response data structures and response codes specified in table 5.32.2.3.3.3-3.

Table 5.32.2.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MemUeSelectAssistSubscPatch | M | 1 | Contains the requested modifications to the "Individual Member UE Selection Assistance Subscription" resource. |

Table 5.32.2.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MemUeSelectAssistSubsc | M | 1 | 200 OK | Successful response. The "Individual Member UE Selection Assistance Subscription" resource is successfully modified and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful response. The "Individual Member UE Selection Assistance Subscription" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.32.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.32.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.32.2.3.3.4	DELETE

The HTTP DELETE method enables to request the deletion of an existing "Individual Member UE Selection Assistance Subscription" resource at the NEF.

This method shall support the URI query parameters specified in table 5.32.2.3.3.4-1.

Table 5.32.2.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.32.2.3.3.4-2 and the response data structures and response codes specified in table 5.32.2.3.3.4-3.

Table 5.32.2.3.3.4-2: Data structures supported by the DELETE request body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.32.2.3.3.4-3: Data structures supported by the DELETE response body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful response. The "Individual Member UE Selection Assistance Subscription" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.32.2.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.32.2.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

##### 5.32.2.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

### 5.32.3	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.32.4	Notifications

#### 5.32.4.1	General

Notifications shall comply to clause 5.2.5 of 3GPP TS 29.122 [4].

Table 5.32.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Member UE Selection Assistance Notification | {notifUri} | POST | Enable the NEF to notify a previously subscribed AF on Member UE Selection Assistance. |

#### 5.32.4.2	Member UE Selection Assistance Notification

##### 5.32.4.2.1	Description

The Member UE Selection Assistance Notification is used by the NEF to report Member UE Selection Assistance related event(s) to a previously subscribed AF.

##### 5.32.4.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 5.32.4.2.2-1.

Table 5.32.4.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | Represents the callback URI encoded as a string formatted as a URI. |

##### 5.32.4.2.3	Operation Definition

This method shall support the request data structures specified in table 5.32.4.2.3-1 and the response data structures and response codes specified in table 5.32.4.2.3-2.

Table 5.32.4.2.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(MemUeSeletAssistNotif) | M | 1..N | Represents the Member UE Selection Assistance Notification. |

Table 5.32.4.2.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The notification is successfully received and acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AF towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.32.4.2.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

Table 5.32.4.2.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AF towards which the notification should be redirected. |

### 5.32.5	Data Model

#### 5.32.5.1	General

This clause specifies the application data model supported by the MemberUESelectionAssistance API. Table 5.32.5.1-1 specifies the data types defined for the MemberUESelectionAssistance API.

Table 5.32.5.1-1: MemberUESelectionAssistance API specific Data Types

| Data type |  | Clause defined |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| AccessRatTypeFilterCriteria |  | 5.32.5.2.5 |  | Represents the Access types and Rat types filtering criteria for Member UE selection. |  |
| CandUeInfo |  | 5.32.5.2.16 |  | Represents the candidate UE information. |  |
| CandiUeListInfo |  | 5.32.5.2.13 |  | Represents the lists of candidate UEs and recommended time window for performing the application operation. |  |
| DnnFilterCriteria |  | 5.32.5.2.12 |  | Represents the DNN filtering criteria for Member UE selection. |  |
| E2ETransTimeFilterCriteria |  | 5.32.5.2.6 |  | Represents the End-to-end data volume transfer time filtering criteria for Member UE selection. |  |
| FilterCriterionType |  | 5.32.5.3.3 |  | Represents the filter criterion type. |  |
| MemUeSeletAssistNotif |  | 5.32.5.2.3 |  | Represents a Member UE Selection Assistance Notification. |  |
| MemUeSelectAssistSubsc |  | 5.32.5.2.2 |  | Represents a Member UE Selection Assistance Subscription. |  |
| MemUeSelectAssistSubscPatch |  | 5.32.5.2.15 |  | Represents parameters to request the modification of a Member UE Selection Assistance Subscription. |  |
| MemUeSeletReport |  | 5.32.5.2.14 |  | Represents a Member UE selection report. |  |
| UeDirectionFilterCriteria |  | 5.32.5.2.9 |  | Represents the UE direction filtering criteria for Member UE selection. |  |
| UeDistanceFilterCriteria |  | 5.32.5.2.10 |  | Represents the UE distance filtering criteria for Member UE selection. |  |
| UeHisLocFilterCriteria |  | 5.32.5.2.8 |  | Represents the UE historical location filtering criteria for Member UE selection. |  |
| UeLocFilterCriteria |  | 5.32.5.2.7 |  | Represents the UE location filtering criteria for Member UE selection. |  |
| QoSFilterCriteria |  | 5.32.5.2.4 |  | Represents the QoS filtering criteria for Member UE selection. |  |
| ServiceExpFilterCriteria |  | 5.32.5.2.11 |  | Represents the Service Experience filtering criteria for Member UE selection. |  |

Table 5.32.5.1-2 specifies data types re-used by the MemberUESelectionAssistance API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the MemberUESelectionAssistance API.

Table 5.32.5.1-2: MemberUESelectionAssistance API re-used Data Types

| Data type |  | Reference |  | Comments |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AccessType |  | 3GPP TS 29.571 [8] |  | Represents the access type. |  |  |  |
| AmfEventType |  | 3GPP TS 29.518 [18] |  | Represents the events exposed by an AMF. |  |  |  |
| DataVolumeTransferTime |  | 3GPP TS 29.520 [27] |  | Represents the End-to-end data volume transfer time. |  |  |  |
| DateTime |  | 3GPP TS 29.122 [4] |  | Represents a date and a time. |  |  |  |
| DurationSec |  | 3GPP TS 29.122 [4] |  | Represents the duration time in units of seconds. |  |  |  |
| DurationSecRm |  | 3GPP TS 29.122 [4] |  | Represents the time duration, same as the DurationSec data type, but with the OpenAPI "nullable: true" property. |  |  |  |
| Direction |  | 3GPP TS 29.520 [27] |  | Represents the UE direction. |  |  |  |
| Dnai |  | 3GPP TS 29.571 [8] |  | Represents a user plane access to one or more DN(s). |  |  |  |
| Dnn |  | 3GPP TS 29.571 [8] |  | Represents a DNN. |  |  |  |
| Gpsi |  | 3GPP TS 29.571 [8] |  | Represents a GPSI. |  |  |  |
| GeoDistributionInfo |  | 3GPP TS 29.520 [27] |  | Represents the geographical distribution of the UEs. |  |  |  |
| IpAddr |  | 3GPP TS 29.571 [8] |  | Represents an IP address. |  |  |  |
| LocationArea5G |  | 3GPP TS 29.122 [4] |  | Represents a user location area when the UE is attached to 5G. |  |  |  |
| NwdafEvent |  | 3GPP TS 29.520 [27] |  | Represents the events exposed by an NWDAF. |  |  |  |
| RatType |  | 3GPP TS 29.571 [8] |  | Represents the RAT types. |  |  |  |
| ServiceExperienceType |  | 3GPP TS 29.520 [27] |  | Represents the type of Service Experience Analytics. |  |  |  |
| SmfEvent |  | 3GPP TS 29.508 [26] |  | Represents the events exposed by an SMF. |  |  |  |
| Snssai |  | 3GPP TS 29.571 [8] |  | Represents an S-NSSAI. |  |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [8] |  | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |  |  |
| TimeWindow |  | 3GPP TS 29.122 [4] |  | Represents a time window. |  |  |  |
| Uinteger |  | 3GPP TS 29.571 [8] |  | Represents an unsigned integer. |  |  |  |
| Uri |  | 3GPP TS 29.122 [4] |  | Represents a URI. |  |  |  |

#### 5.32.5.2	Structured data types

##### 5.32.5.2.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.32.5.2.2	Type: MemUeSelectAssistSubsc

Table 5.32.5.2.2-1: Definition of type MemUeSelectAssistSubsc

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| afId |  | string |  | O |  | 0..1 |  | Contains the identifier of the AF. |  |  |  |
| tgtUeIds |  | array(Gpsi) |  | C |  | 1..N |  | Identifies the list of identifier(s) of the UE(s) for which Member UE Selection Assistance Reporting is requested. (NOTE 2) |  |  |  |
| tgtUeIps |  | array(IpAddr) |  | C |  | 1..N |  | Contains the IP address(es) of the UE(s) for which Member UE Selection Assistance Reporting is requested. (NOTE 2) |  |  |  |
| notifUri |  | Uri |  | M |  | 1 |  | Contains the URI via which Member UE Selection Assistance notifications shall be delivered. |  |  |  |
| notifId |  | string |  | M |  | 1 |  | Contains the Notification Correlation ID assigned by the AF. |  |  |  |
| qosFilters |  | array(QoSFilterCriteria) |  | C |  | 1..N |  | Contains the QoS filtering criteria for Member UE selection. (NOTE 1) |  |  |  |
| accRatTypeFilters |  | array(AccessRatTypeFilterCriteria) |  | C |  | 1..N |  | Contains The Access types and Rat types filtering criteria for Member UE selection. (NOTE 1) |  |  |  |
| e2eTransTimeFilters |  | array(E2ETransTimeFilterCriteria) |  | C |  | 1..N |  | Contains the End-to-end data volume transfer time filtering criteria for Member UE selection. (NOTE 1) |  |  |  |
| ueLocFilters |  | array(UeLocFilterCriteria) |  | C |  | 1..N |  | Contains the UE location filtering criteria for Member UE selection. (NOTE 1) |  |  |  |
| ueHisLocFilters |  | array(UeHisLocFilterCriteria) |  | C |  | 1..N |  | Contains the UE historical location filtering criteria for Member UE selection. (NOTE 1) |  |  |  |
| ueDirFilters |  | array(UeDirectionFilterCriteria) |  | C |  | 1..N |  | Contains the UE direction filtering criteria for Member UE selection. (NOTE 1) |  |  |  |
| ueDistanceFilters |  | array(UeDistanceFilterCriteria) |  | C |  | 1..N |  | Contains the UE distance filtering criteria for Member UE selection. (NOTE 1) |  |  |  |
| serviceExpFilters |  | array(ServiceExpFilterCriteria) |  | C |  | 1..N |  | Contains the Service Experience filtering criteria for Member UE selection. (NOTE 1) |  |  |  |
| dnnFilters |  | array(DnnFilterCriteria) |  | C |  | 1..N |  | Contains the DNN filtering criteria for Member UE selection. (NOTE 1) |  |  |  |
| memUpdatePeriod |  | DurationSec |  | O |  | 0..1 |  | Contains the periodicity information for updating the member UEs. |  |  |  |
| maxUeNum |  | Uinteger |  | O |  | 0..1 |  | Contains The maximum number of candidate UEs. |  |  |  |
| timeWin |  | TimeWindow |  | O |  | 0..1 |  | Contains the start time and end time for selecting the candidate UEs. |  |  |  |
| expTime |  | DateTime |  | O |  | 0..1 |  | Contains the subscription expiry time. |  |  |  |
| suppFeat |  | SupportedFeatures |  | C |  | 0..1 |  | Contains the list of Supported features used as defined in clause 5.32.6. This attribute shall be present only when feature negotiation needs to take place. |  |  |  |
| NOTE 1: At least one of these attribtues shall be present. NOTE 2: These attributes are mutually exclusive and one of them shall be present. |  |  |  |  |  |  |  |  |  |  |  |

##### 5.32.5.2.3	Type: MemUeSeletAssistNotif

Table 5.32.5.2.3-1: Definition of type MemUeSeletAssistNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifId | string | M | 1 | Contains the Notification Correlation ID. |  |
| candiUeInfos | array(CandiUeListInfo) | M | 1..N | Contains the list of candidate UEs and the related information. |  |
| memUeSelectRpts | array(MemUeSeletReport) | O | 1..N | Contains the Member UE Selection report(s). |  |

##### 5.32.5.2.4	Type: QoSFilterCriteria

Table 5.32.5.2.4-1: Definition of type QoSFilterCriteria

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | SmfEvent | O | 0..1 | Contains the SMF event which may be used to retrieve the QoS monitoring information. The only applicable value for this attribute is "QOS_MON". |  |
| appId | string | O | 0..1 | Contains the identifier of the targeted application. |  |
| dnn | Dnn | O | 0..1 | Contains the DNN, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. |  |
| snssai | Snssai | O | 0..1 | Contains the network slice information. |  |
| ulDelay | Uinteger | O | 0..1 | Contains the uplink packet delay in units of milliseconds. |  |
| dlDelay | Uinteger | O | 0..1 | Contains the downlink packet delay in units of milliseconds. |  |
| rtDelay | Uinteger | O | 0..1 | Contains the round trip delay in units of milliseconds. |  |

##### 5.32.5.2.5	Type: AccessRatTypeFilterCriteria

Table 5.32.5.2.5-1: Definition of type AccessRatTypeFilterCriteria

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| events | array(SmfEvent) | O | 1..N | Contains the SMF event(s) which may be used to retrieve the Access Type and/or RAT Type of the selected UE. The only applicable values within each array element of this attribute are "AC_TY_CH" or "RAT_TY_CH". |  |
| dnn | Dnn | O | 0..1 | Contains a DNN, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. |  |
| snssai | Snssai | O | 0..1 | Contains the network slice information. |  |
| accTypes | array(AccessType) | O | 1..N | Contains the Access Type of the selected UE. |  |
| ratTypes | array(RatType) | O | 1..N | Contains the RAT Type(s) of the selected UE. |  |

##### 5.32.5.2.6	Type: E2ETransTimeFilterCriteria

Table 5.32.5.2.6-1: Definition of type E2ETransTimeFilterCriteria

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| event |  | NwdafEvent |  | O |  | 0..1 |  | Contains the NWDAF event which may be used to retrieve the End-to-end data volume transfer time. The only applicable value for this attribute is "E2E_DATA_VOL_TRANS_TIME". |  |  |  |
| appId |  | string |  | O |  | 0..1 |  | Contains the identifier of the targeted an application. |  |  |  |
| dnn |  | Dnn |  | O |  | 0..1 |  | Contains the DNN, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. |  |  |  |
| snssai |  | Snssai |  | O |  | 0..1 |  | Contains the network slice information. |  |  |  |
| dataVolTransTime |  | DataVolumeTransferTime |  | O |  | 0..1 |  | Contains the E2E data volume transfer time and the data volume used to derive the transfer time. |  |  |  |
| geoDistrInfos |  | array(GeoDistributionInfo) |  | O |  | 1..N |  | Contains the request for geographical distribution of UEs. |  |  |  |
| locationArea |  | LocationArea5G |  | O |  | 0..1 |  | Contains the location area of the candidate UEs. |  |  |  |
| numDataTrans |  | Uinteger |  | O |  | 0..1 |  | Contains the target number of data transmission repetitions. |  |  |  |
| timeInterval |  | DurationSec |  | O |  | 0..1 |  | Contains the target time interval (in units of seconds) between data transmissions. |  |  |  |

##### 5.32.5.2.7	Type: UeLocFilterCriteria

Table 5.32.5.2.7-1: Definition of type UeLocFilterCriteria

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | AmfEventType | O | 0..1 | Contains the AMF event which may be used to retrieve the UE location. The only applicable value for this attribute is "LOCATION_REPORT". |  |
| loc | LocationArea5G | O | 0..1 | Contains the location area within which the selected UE is currently located. |  |

##### 5.32.5.2.8	Type: UeHisLocFilterCriteria

Table 5.32.5.2.8-1: Definition of type UeHisLocFilterCriteria

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | NwdafEvent | O | 0..1 | Contains the NWDAF event which may be used to retrieve the UE Mobility. The only applicable value for this attribute is "UE_MOBILITY". |  |
| loc | LocationArea5G | O | 0..1 | Contains the historical location of the UEs. |  |

##### 5.32.5.2.9	Type: UeDirectionFilterCriteria

Table 5.32.5.2.9-1: Definition of type UeDirectionFilterCriteria

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | NwdafEvent | O | 0..1 | Contains the NWDAF event which may be used to retrieve the UE Mobility. The only applicable value for this attribute is "UE_MOBILITY". |  |
| directions | array(Direction) | O | 1..N | Contains the moving directions of the UEs. |  |

##### 5.32.5.2.10	Type: UeDistanceFilterCriteria

Table 5.32.5.2.10-1: Definition of type UeDistanceFilterCriteria

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | NwdafEvent | O | 0..1 | Contains the NWDAF event which may be used to retrieve the Relative Proximity. The only applicable value for this attribute is "RELATIVE_PROXIMITY". |  |
| distance | Uinteger | O | 0..1 | Contains the minimum UE to UE separating distance of the selected UEs. |  |

##### 5.32.5.2.11	Type: ServiceExpFilterCriteria

Table 5.32.5.2.11-1: Definition of type ServiceExpFilterCriteria

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | NwdafEvent | O | 0..1 | Contains the NWDAF event which may be used to retrieve the Service Experience. The only applicable value for this attribute is "SERVICE_EXPERIENCE". |  |
| snssai | Snssai | O | 0..1 | Contains the network slice information. |  |
| dnn | Dnn | O | 0..1 | Contains the DNN, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. |  |
| appId | string | O | 0..1 | Contains the identifier of the targeted application. |  |
| dnai | Dnai | O | 0..1 | Contains the DNAI indicating the user plane access to DN to which the subscription applies. |  |
| loc | LocationArea5G | O | 0..1 | Contains the area of interest. |  |
| contribWeightThr | Uinteger | O | 0..1 | Contains the Service Experience Contribution weight reporting threshold. |  |
| expTypes | array(ServiceExperienceType) | O | 1..N | Contains the Service Experience Type. |  |

##### 5.32.5.2.12	Type: DnnFilterCriteria

Table 5.32.5.2.12-1: Definition of type DnnFilterCriteria

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | SmfEvent | O | 0..1 | Contains the SMF event which may be used to retrieve the DNN. The only applicable value for this attribute is "QFI_ALLOC". |  |
| dnn | Dnn | O | 0..1 | Contains the DNN of the selected UE for the PDU Session used by the application. |  |

##### 5.32.5.2.13	Type: CandiUeListInfo

Table 5.32.5.2.13-1: Definition of type CandiUeListInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| candUeInfos | array(CandUeInfo) | M | 1..N | Contains the list of the identifier(s) of the candidate UE(s). |  |
| remdTimeWin | TimeWindow | O | 0..1 | Contains the recommended time window for performing the application operation. It shall be a subset of the time window specified by "timeWin" attribute of the MemUeSelectAssistSubsc or MemUeSelectAssistSubscPatch data types representing the corresponding subscription resource. |  |

##### 5.32.5.2.14	Type: MemUeSeletReport

Table 5.32.5.2.14-1: Definition of type MemUeSeletReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| criterionType | FilterCriterionType | M | 1 | Contains the filter criterion type. |  |
| numForCriterion | Uinteger | M | 1 | Contains the number of UEs that do not meet the filter criterion. |  |

##### 5.32.5.2.15	Type: MemUeSelectAssistSubscPatch

Table 5.32.5.2.15-1: Definition of type MemUeSelectAssistSubscPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | O | 0..1 | Contains the updated URI via which Member UE Selection Assistance notifications shall be delivered. |  |
| notifId | string | O | 0..1 | Contains the updated Notification Correlation ID assigned by the AF. |  |
| qosFilters | array(QoSFilterCriteria) | O | 1..N | Contains the updated QoS filtering criteria for Member UE selection. |  |
| accRatTypeFilters | array(AccessRatTypeFilterCriteria) | O | 1..N | Contains the updated Access types and Rat types filtering criteria for Member UE selection. |  |
| e2eTransTimeFilters | array(E2ETransTimeFilterCriteria) | O | 1..N | Contains the updated End-to-end data volume transfer time filtering criteria for Member UE selection. |  |
| ueLocFilters | array(UeLocFilterCriteria) | O | 1..N | Contains the updated UE location filtering criteria for Member UE selection. |  |
| ueHisLocFilters | array(UeHisLocFilterCriteria) | O | 1..N | Contains the updated UE historical location filtering criteria for Member UE selection. |  |
| ueDirFilters | array(UeDirectionFilterCriteria) | O | 1..N | Contains the updated UE direction filtering criteria for Member UE selection. |  |
| ueDistanceFilters | array(UeDistanceFilterCriteria) | O | 1..N | Contains the updated UE distance filtering criteria for Member UE selection. |  |
| serviceExpFilters | array(ServiceExpFilterCriteria) | O | 1..N | Contains the updated Service Experience filtering criteria for Member UE selection. |  |
| dnnFilters | array(DnnFilterCriteria) | O | 1..N | Contains the updated DNN filtering criteria for Member UE selection. |  |
| expTime | DateTime | O | 0..1 | Contains the updated expiry time of the subscription. |  |
| memUpdatePeriod | DurationSecRm | O | 0..1 | Contains the updated periodicity information for updating the member UEs. |  |
| maxUeNum | UintegerRm | O | 0..1 | Contains the updated maximum number of candidate UEs. |  |
| timeWin | TimeWindow | O | 0..1 | Contains the updated the start time and end time for selecting the candidate UEs. |  |

##### 5.32.5.2.16	Type: CandUeInfo

Table 5.32.5.2.16-1: Definition of type CandUeInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| candUeId | Gpsi | C | 0..1 | Contains the GPSI of a candidate UE. (NOTE) |  |
| candUeIp | IpAddr | C | 0..1 | Containsthe IP address of a candidate UE. (NOTE) |  |
| qosFilter | QoSFilterCriteria | O | 0..1 | Contains the QoS filtering criteria for Member UE selection. |  |
| accRatTypeFilter | AccessRatTypeFilterCriteria | O | 0..1 | Contains the Access types and RAT types filtering criteria for Member UE selection. |  |
| e2eTransTimeFilter | E2ETransTimeFilterCriteria | O | 0..1 | Contains the End-to-end data volume transfer time filtering criteria for Member UE selection. |  |
| ueLocFilter | UeLocFilterCriteria | O | 0..1 | Contains the UE location filtering criteria for Member UE selection. |  |
| ueHisLocFilter | UeHisLocFilterCriteria | O | 0..1 | Contains the UE historical location filtering criteria for Member UE selection. |  |
| ueDirFilter | UeDirectionFilterCriteria | O | 0..1 | Contains the UE direction filtering criteria for Member UE selection. |  |
| ueDistanceFilter | UeDistanceFilterCriteria | O | 0..1 | Contains the UE distance filtering criteria for Member UE selection. |  |
| serviceExpFilter | ServiceExpFilterCriteria | O | 0..1 | Contains the Service Experience filtering criteria for Member UE selection. |  |
| dnnFilter | DnnFilterCriteria | O | 0..1 | Contains the DNN filtering criteria for Member UE selection. |  |
| NOTE: Either the "candUeId" attribute or the "candUeIp" attribute shall be present. |  |  |  |  |  |

#### 5.32.5.3	Simple data types and enumerations

##### 5.32.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.32.5.3.2	Simple data types

The simple data types defined in table 5.32.5.3.2-1 shall be supported.

Table 5.32.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 5.32.5.3.3	Enumeration: FilterCriterionType

The enumeration FilterCriterionType represents the type of filter criterion event for which the AF requests to be notified. It shall comply with the provisions defined in table 5.32.5.3.3-1.

Table 5.32.5.3.3-1: Enumeration FilterCriterionType

| Enumeration value | Description |
| --- | --- |
| QOS | Indicates that the type of filter criterion is QoS criterion. |
| ACCESS_RAT_TYPE | Indicates that the type of filter criterion is Access and Rat types criterion. |
| E2E_DATA_VOLUME_TRANSFER_TIME | Indicates that the type of filter criterion is End-to-end data volume transfer time criterion. |
| UE_LOCATION | Indicates that the type of filter criterion is UE location criterion. |
| UE_HISTORICAL_LOCATION | Indicates that the type of filter criterion is UE historical location criterion. |
| UE_DIRECTION | Indicates that the type of filter criterion is UE direction criterion. |
| UE_DISTANCE | Indicates that the type of filter criterion is UE distance criterion. |
| SERVICE_EXPERIENCE | Indicates that the type of filter criterion is Service Experience criterion. |
| DNN | Indicates that the type of filter criterion is DNN criterion. |

#### 5.32.5.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

### 5.32.6	Used Features

The optional features listed in table 5.32.6-1 are defined for the MemberUESelectionAssistance API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.32.6-1: Features used by MemberUESelectionAssistance API

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 5.32.7	Error handling

#### 5.32.7.1	General

For the MemberUESelectionAssistance API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [4] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses are applicable for the MemberUESelectionAssistance API.

#### 5.32.7.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the MemberUESelectionAssistance API.

#### 5.32.7.3	Application Errors

The application errors defined for the MemberUESelectionAssistance API are listed in table 5.32.7.3-1.

Table 5.32.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

## 5.33	GroupParametersProvisioning API

### 5.33.1	Introduction

The Nnef_ParameterProvision service shall use the GroupParametersProvisioning API for:

-	DNN and S-NSSAI specific Group Parameters provisioning.

The API URI of the GroupParametersProvisioning API shall be:

- {apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [4], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-grp-pp".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [4].

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.33.2	Resources

#### 5.33.2.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.33.2.1-1 and the resources and HTTP methods used for the GroupParametersProvisioning API.

Figure 5.33.2.1-1: Resource URI structure of the GroupParametersProvisioning API

Table 5.33.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.33.2.1-1: Resources and methods overview

| Resource name | Resource URI (relative path under API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Group Parameters Provisionings | /pp | GET | Retrieve all the active Group Parameters Provisionings managed by the NEF. |
|  |  | POST | Request the creation of a new Group Parameters Provisioning at the NEF. |
| Individual Group Parameters Provisioning | /pp/{ppId} | GET | Retrieve an existing "Individual Group Parameters Provisioning" managed by the NEF. |
|  |  | PUT | Update an existing "Individual Group Parameters Provisioning" managed by the NEF. |
|  |  | PATCH | Modify an existing "Individual Group Parameters Provisioning" managed by the NEF. |
|  |  | DELETE | Delete an existing "Individual Group Parameters Provisioning" managed by the NEF. |

#### 5.33.2.2	Resource: Group Parameters Provisionings

##### 5.33.2.2.1	Introduction

This resource represents the collection of Group Parameters Provisionings managed by the NEF.

This resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [32]).

##### 5.33.2.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-grp-pp/<apiVersion>/pp

This resource shall support the resource URI variables defined in table 5.33.2.2.2-1.

Table 5.33.2.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.33.1. |

##### 5.33.2.2.3	Resource Methods

###### 5.33.2.2.3.1	GET

This method enables an AF to request to retrieve all the active "Group Parameters Provisionings" resources managed by the NEF.

This method shall support the URI query parameters specified in table 5.33.2.2.3.1-1.

Table 5.33.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.33.2.2.3.1-2 and the response data structures and response codes specified in table 5.33.2.2.3.1-3.

Table 5.33.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.33.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(GrpPpData) | M | 0..N | 200 OK | Successful case. All the "Individual Group Parameters Provisioning" resources managed by the NEF are returned. If there are no existing "Individual Group Parameters Provisioning" resources managed at the NEF, an empty array is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |  |

Table 5.33.2.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.33.2.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.33.2.2.3.2	POST

This method enables an AF to request the creation of a new Group Parameters Provisioning at the NEF.

This method shall support the URI query parameters specified in table 5.33.2.2.3.2-1.

Table 5.33.2.2.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.33.2.2.3.2-2 and the response data structures and response codes specified in table 5.33.2.2.3.2-3.

Table 5.33.2.2.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| GrpPpData | M | 1 | Contains the representation of the Group Parameters Provisioning to be created at the NEF. |

Table 5.33.2.2.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| GrpPpData | M | 1 | 201 Created | Successful case. A representation of the created "Individual Group Parameters Provisioning" resource is returned in the response body. The URI of the created resource shall be returned in an HTTP "Location" header. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.33.2.2.3.2-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-grp-pp/<apiVersion>/pp/{ppId} |

##### 5.33.2.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 5.33.2.3	Resource: Individual Group Parameters Provisioning

##### 5.33.2.3.1	Introduction

This resource represents an "Individual Group Parameters Provisioning" resource managed by the NEF.

This resource is modelled with the Document resource archetype (see clause C.2 of 3GPP TS 29.501 [32]).

##### 5.33.2.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-grp-pp/<apiVersion>/pp/{ppId}

This resource shall support the resource URI variables defined in table 5.33.2.3.2-1.

Table 5.33.2.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.33.1. |
| ppId | string | Represents the identifier of the "Individual Group Parameters Provisioning" resource. |

##### 5.33.2.3.3	Resource Methods

###### 5.33.2.3.3.1	GET

This method enables an AF to request to retrieve an existing "Individual Group Parameters Provisioning" resource at the NEF.

This method shall support the URI query parameters specified in table 5.33.2.3.3.1-1.

Table 5.33.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.33.2.3.3.1-2 and the response data structures and response codes specified in table 5.33.2.3.3.1-3.

Table 5.33.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.33.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| GrpPpData | M | 1 | 200 OK | Successful case. The requested "Individual Group Parameters Provisioning" resource is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.33.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.33.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.33.2.3.3.2	PUT

This method enables an AF to request the update of an existing "Individual Group Parameters Provisioning" resource at the NEF.

This method shall support the URI query parameters specified in table 5.33.2.3.3.2-1.

Table 5.33.2.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.33.2.3.3.2-2 and the response data structures and response codes specified in table 5.33.2.3.3.2-3.

Table 5.33.2.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| GrpPpData | M | 1 | Represents the updated "Individual Group Parameters Provisioning" resource representation. |

Table 5.33.2.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |  |
| --- | --- | --- | --- | --- | --- |
| GrpPpData | M | 1 | 200 OK | Successful response. The "Individual Group Parameters Provisioning" resource is successfully updated and a representation of the updated resource is returned in the response body. |  |
| n/a |  |  | 204 No Content | Successful response. The "Individual Group Parameters Provisioning" resource is successfully updated and no content is returned in the response body. |  |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.33.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.33.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.33.2.3.3.3	PATCH

This method enables an AF to request the modification of an existing "Individual Group Parameters Provisioning" resource at the NEF.

This method shall support the URI query parameters specified in table 5.33.2.3.3.3-1.

Table 5.33.2.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.33.2.3.3.3-2 and the response data structures and response codes specified in table 5.33.2.3.3.3-3.

Table 5.33.2.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| GrpPpDataPatch | M | 1 | Represents the requested modifications to the "Individual Group Parameters Provisioning" resource. |

Table 5.33.2.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |  |
| --- | --- | --- | --- | --- | --- |
| GrpPpData | M | 1 | 200 OK | Successful response. The "Individual Group Parameters Provisioning" resource is successfully modified and a representation of the updated resource is returned in the response body. |  |
| n/a |  |  | 204 No Content | Successful response. The "Individual Group Parameters Provisioning" resource is successfully modified and no content is returned in the response body. |  |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.33.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.33.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.33.2.3.3.4	DELETE

This method enables an AF to request the deletion of an existing "Individual Group Parameters Provisioning" resource at the NEF.

This method shall support the URI query parameters specified in table 5.33.2.3.3.4-1.

Table 5.33.2.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.33.2.3.3.4-2 and the response data structures and response codes specified in table 5.33.2.3.3.4-3.

Table 5.33.2.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.33.2.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual Group Parameters Provisioning" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.33.2.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.33.2.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

##### 5.33.2.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

### 5.33.3	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.33.4	Notifications

There are no notifications defined for this API in this release of the specification.

### 5.33.5	Data Model

#### 5.33.5.1	General

This clause specifies the application data model supported by the GroupParametersProvisioning API. Table 5.33.5.1-1 specifies the data types defined for the GroupParametersProvisioning API.

Table 5.33.5.1-1: GroupParametersProvisioning API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AfReqDefaultQoS | 5.33.5.2.5 | Represents the AF requested default QoS. |  |
| DnnSnssaiGrpData | 5.33.5.2.4 | Represents DNN and S-NSSAI specific Group Parameters data. |  |
| GrpPpData | 5.33.5.2.2 | Represents Group Parameters Provisioning data. |  |
| GrpPpDataPatch | 5.33.5.2.3 | Represents the requested modification to an existing Group Parameters Provisioning data. |  |
| LadnServArea | 5.33.5.2.6 | Represents an LADN Service Area. |  |

Table 5.33.5.1-2 specifies data types re-used by the GroupParametersProvisioning API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the GroupParametersProvisioning API.

Table 5.33.5.1-2: GroupParametersProvisioning API re-used Data Types

| Data type |  | Reference |  | Comments |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5Qi |  | 3GPP TS 29.571 [8] |  | Represents a 5G QoS Identifier. |  |  |  |
| 5QiPriorityLevelRm |  | 3GPP TS 29.571 [8] |  | Represents the 5QI Priority Level. |  |  |  |
| Arp |  | 3GPP TS 29.571 [8] |  | Represents an ARP. |  |  |  |
| BitRate |  | 3GPP TS 29.571 [8] |  | Represents a bit rate. |  |  |  |
| BitRateRm |  | 3GPP TS 29.571 [8] |  | This data type is defined in the same way as the BitRate data type, but with the OpenAPI "nullable: true" property. |  |  |  |
| CivicAddress |  | 3GPP TS 29.572 [34] |  | Represents a civic address. |  |  |  |
| Dnn |  | 3GPP TS 29.571 [8] |  | Represents a DNN. |  |  |  |
| ExternalGroupId |  | 3GPP TS 29.122 [4] |  | Represents the External Group Identifier for a user group. |  |  |  |
| GeographicArea |  | 3GPP TS 29.572 [34] |  | Represents a geographical area. |  |  |  |
| MtcProviderInformation |  | 3GPP TS 29.571 [8] |  | Represents the MTC provider information. |  |  |  |
| Snssai |  | 3GPP TS 29.571 [8] |  | Represents an S-NSSAI. |  |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [8] |  | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |  |  |
| Tai |  | 3GPP TS 29.571 [8] |  | Represents a Tracking Area Identifier. |  |  |  |

#### 5.33.5.2	Structured data types

##### 5.33.5.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 5.33.5.2.2	Type: GrpPpData

Table 5.33.5.2.2-1: Definition of type GrpPpData

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| afId |  | string |  | M |  | 1 |  | Contains the identifier of the AF that is sending the request. |  |  |  |
| mtcProviderId |  | MtcProviderInformation |  | O |  | 0..1 |  | Contains the identifier of the MTC Service Provider and/or MTC Application. |  |  |  |
| dnnSnssaiGrpData |  | DnnSnssaiGrpData |  | C |  | 0..1 |  | Contains the DNN and S-NSSAI specific Group data that the AF requests to provision. This attribute shall be present only when the AF requests to provision DNN and S-NSSAI specific Group parameters. |  |  |  |
| suppFeat |  | SupportedFeatures |  | C |  | 0..1 |  | Contains the list of supported features among the ones defined in clause 5.33.6. This attribute shall be present only when feature negotiation needs to take place. |  |  |  |

##### 5.33.5.2.3	Type: GrpPpDataPatch

Table 5.33.5.2.3-1: Definition of type GrpPpDataPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dnnSnssaiGrpData | DnnSnssaiGrpData | O | 0..1 | Contains the updated DNN and S-NSSAI specific Group data that the AF requests to provision. |  |

##### 5.33.5.2.4	Type: DnnSnssaiGrpData

Table 5.33.5.2.4-1: Definition of type DnnSnssaiGrpData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| extGroupId | ExternalGroupId | M | 1 | Represents the external group identifier of the targeted group. |  |
| dnn | Dnn | M | 1 | Represents the concerned DNN. |  |
| snssai | Snssai | M | 1 | Represents the concerned S-NSSAI. |  |
| defQos | AfReqDefaultQoS | C | 0..1 | Represents the AF-requested default QoS parameters to be applied to each UE within the targeted group. (NOTE) |  |
| ladnServArea | LadnServArea | C | 0..1 | Represents the AF-requested LADN Service Area to be applied to each UE within the targeted group. (NOTE) |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

##### 5.33.5.2.5	Type: AfReqDefaultQoS

Table 5.33.5.2.5-1: Definition of type AfReqDefaultQoS

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| 5qi | 5Qi | M | 1 | Represents the AF requested default 5G QoS Identifier. |  |
| arp | Arp | M | 1 | Represents the AF requested default Allocation and Retention Priority. |  |
| priorityLevel | 5QiPriorityLevelRm | O | 0..1 | Represents the AF requested 5QI Priority Level. |  |

##### 5.33.5.2.6	Type: LadnServArea

Table 5.33.5.2.6-1: Definition of type LadnServArea

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| geographicAreas | array(GeographicArea) | C | 1..N | Represents the LADN Service Area in the form of a list of geographic area(s). (NOTE) |  |
| civicAddresses | array(CivicAddress) | C | 1..N | Represents the LADN Service Area in the form of a list of civic addresse(s). (NOTE) |  |
| tais | array(Tai) | C | 1..N | Represents the LADN Service Area in the form of a list of Tracking Area Identifier(s). (NOTE) |  |
| NOTE: These attributes are mutually exclusive. Either one of them shall be present. |  |  |  |  |  |

#### 5.33.5.3	Simple data types and enumerations

##### 5.33.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.33.5.3.2	Simple data types

The simple data types defined in table 5.33.5.3.2-1 shall be supported.

Table 5.33.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

#### 5.33.5.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

### 5.33.6	Used Features

The table below defines the features applicable to the GroupParametersProvisioning API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.33.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 5.33.7	Error handling

#### 5.33.7.1	General

For the GroupParametersProvisioning API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [4] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses are applicable for the GroupParametersProvisioning API.

#### 5.33.7.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the GroupParametersProvisioning API.

#### 5.33.7.3	Application Errors

The application errors defined for the GroupParametersProvisioning API are listed in table 5.33.7.3-1.

Table 5.33.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

## 5.34	SliceParamProvision API

### 5.34.1	Introduction

The Nnef_ParameterProvision service shall use the SliceParamProvision API for:

-	Network Slice Parameters provisioning.

The API URI of the SliceParamProvision API shall be:

- {apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [4], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-slice-pp".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [4].

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.34.2	Resources

#### 5.34.2.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.34.2.1-1 and the resources and HTTP methods used for the SliceParamProvision API.

Figure 5.34.2.1-1: Resource URI structure of the SliceParamProvision API

Table 5.34.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.34.2.1-1: Resources and methods overview

| Resource name | Resource URI (relative path under API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Slice Parameters Provisionings | /pp | GET | Retrieve all the active Slice Parameters Provisionings managed by the NEF. |
|  |  | POST | Request the creation of a new Slice Parameters Provisioning at the NEF. |
| Individual Slice Parameters Provisioning | /pp/{ppId} | GET | Retrieve an existing "Individual Slice Parameters Provisioning" resource managed by the NEF. |
|  |  | PUT | Update an existing "Individual Slice Parameters Provisioning" resource managed by the NEF. |
|  |  | PATCH | Modify an existing "Individual Slice Parameters Provisioning" resource managed by the NEF. |
|  |  | DELETE | Delete an existing "Individual Slice Parameters Provisioning" resource managed by the NEF. |

#### 5.34.2.2	Resource: Slice Parameters Provisionings

##### 5.34.2.2.1	Introduction

This resource represents the collection of Slice Parameters Provisionings managed by the NEF.

This resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [32]).

##### 5.34.2.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-slice-pp/<apiVersion>/pp

This resource shall support the resource URI variables defined in table 5.34.2.2.2-1.

Table 5.34.2.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.34.1. |

##### 5.34.2.2.3	Resource Methods

###### 5.34.2.2.3.1	GET

This method enables an AF to request to retrieve all the active Slice Parameters Provisionings managed by the NEF.

This method shall support the URI query parameters specified in table 5.34.2.2.3.1-1.

Table 5.34.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.34.2.2.3.1-2 and the response data structures and response codes specified in table 5.34.2.2.3.1-3.

Table 5.34.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.34.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(SlicePpData) | M | 0..N | 200 OK | Successful case. All the "Individual Slice Parameters Provisioning" resource(s) managed by the NEF are returned. If there are no active "Individual Slice Parameters Provisioning" resources at the NEF, an empty array is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |  |

Table 5.34.2.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.34.2.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.34.2.2.3.2	POST

This method enables an AF to request the creation of a new Slice Parameters Provisioning at the NEF.

This method shall support the URI query parameters specified in table 5.34.2.2.3.2-1.

Table 5.34.2.2.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.34.2.2.3.2-2 and the response data structures and response codes specified in table 5.34.2.2.3.2-3.

Table 5.34.2.2.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SlicePpData | M | 1 | Contains the representation of the Slice Parameters Provisioning to be created at the NEF. |

Table 5.34.2.2.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SlicePpData | M | 1 | 201 Created | Successful case. A representation of the created "Individual Slice Parameters Provisioning" resource is returned in the response body. The URI of the created resource shall be returned in an HTTP "Location" header. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.34.2.2.3.2-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-slice-pp/<apiVersion>/pp/{ppId} |

##### 5.34.2.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 5.34.2.3	Resource: Individual Slice Parameters Provisioning

##### 5.34.2.3.1	Introduction

This resource represents an "Individual Slice Parameters Provisioning" resource managed by the NEF.

This resource is modelled with the Document resource archetype (see clause C.2 of 3GPP TS 29.501 [32]).

##### 5.34.2.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-slice-pp/<apiVersion>/pp/{ppId}

This resource shall support the resource URI variables defined in table 5.34.2.3.2-1.

Table 5.34.2.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.34.1. |
| ppId | string | Represents the identifier of the "Individual Slice Parameters Provisioning" resource. |

##### 5.34.2.3.3	Resource Methods

###### 5.34.2.3.3.1	GET

This method enables an AF to request to retrieve an existing "Individual Slice Parameters Provisioning" resource at the NEF.

This method shall support the URI query parameters specified in table 5.34.2.3.3.1-1.

Table 5.34.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.34.2.3.3.1-2 and the response data structures and response codes specified in table 5.34.2.3.3.1-3.

Table 5.34.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.34.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SlicePpData | M | 1 | 200 OK | Successful case. The requested "Individual Slice Parameters Provisioning" resource is successfully returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.34.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.34.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.34.2.3.3.2	PUT

This method enables an AF to request the update of an existing "Individual Slice Parameters Provisioning" resource at the NEF.

This method shall support the URI query parameters specified in table 5.34.2.3.3.2-1.

Table 5.34.2.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.34.2.3.3.2-2 and the response data structures and response codes specified in table 5.34.2.3.3.2-3.

Table 5.34.2.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SlicePpData | M | 1 | Represents the updated "Individual Slice Parameters Provisioning" resource representation. |

Table 5.34.2.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |  |
| --- | --- | --- | --- | --- | --- |
| SlicePpData | M | 1 | 200 OK | Successful response. The "Individual Slice Parameters Provisioning" resource is successfully updated and a representation of the updated resource is returned in the response body. |  |
| n/a |  |  | 204 No Content | Successful response. The "Individual Slice Parameters Provisioning" resource is successfully updated and no content is returned in the response body. |  |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.34.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

Table 5.34.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

###### 5.34.2.3.3.3	PATCH

This method enables an AF to request the modification of an existing "Individual Slice Parameters Provisioning" resource at the NEF.

This method shall support the URI query parameters specified in table 5.34.2.3.3.3-1.

Table 5.34.2.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.34.2.3.3.3-2 and the response data structures and response codes specified in table 5.34.2.3.3.3-3.

Table 5.34.2.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SlicePpDataPatch | M | 1 | Represents the requested modifications to the "Individual Slice Parameters Provisioning" resource. |

Table 5.34.2.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |  |
| --- | --- | --- | --- | --- | --- |
| SlicePpData | M | 1 | 200 OK | Successful response. The "Individual Slice Parameters Provisioning" resource is successfully modified and a representation of the updated resource is returned in the response body. |  |
| n/a |  |  | 204 No Content | Successful response. The "Individual Slice Parameters Provisioning" resource is successfully modified and no content is returned in the response body. |  |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.34.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

Table 5.34.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

###### 5.34.2.3.3.4	DELETE

This method enables an AF to request the deletion of an existing "Individual Slice Parameters Provisioning" resource at the NEF.

This method shall support the URI query parameters specified in table 5.34.2.3.3.4-1.

Table 5.34.2.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.34.2.3.3.4-2 and the response data structures and response codes specified in table 5.34.2.3.3.4-3.

Table 5.34.2.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.34.2.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual Slice Parameters Provisioning" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.34.2.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.34.2.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

##### 5.34.2.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

### 5.34.3	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.34.4	Notifications

There are no notifications defined for this API in this release of the specification.

### 5.34.5	Data Model

#### 5.34.5.1	General

This clause specifies the application data model supported by the SliceParamProvision API. Table 5.34.5.1-1 specifies the data types defined for the SliceParamProvision API.

Table 5.34.5.1-1: SliceParamProvision API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| SlicePpData | 5.34.5.2.1 | Represents Slice Parameters Provisioning data. |  |
| SlicePpDataPatch | 5.34.5.2.2 | Represents the requested modification to existing Slice Parameters Provisioning data. |  |

Table 5.34.5.1-2 specifies data types re-used by the SliceParamProvision API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the SliceParamProvision API.

Table 5.34.5.1-2: SliceParamProvision API re-used Data Types

| Data type |  | Reference |  | Comments |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MtcProviderInformation |  | 3GPP TS 29.571 [8] |  | Represents the MTC provider information. |  |  |  |
| SliceUsageControlInfo |  | 3GPP TS 29.571 [8] |  | Represents the Network Slice Usage Control information. |  |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [8] |  | Represents the list of supported feature(s) and is used to negotiate the applicability of the optional features. |  |  |  |

#### 5.34.5.2	Structured data types

##### 5.34.5.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 5.34.5.2.2	Type: SlicePpData

Table 5.34.5.2.2-1: Definition of type SlicePpData

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| afId |  | string |  | M |  | 1 |  | Represents the identifier of the AF that is sending the request. |  |  |  |
| mtcProviderId |  | MtcProviderInformation |  | O |  | 0..1 |  | Contains the identifier of the MTC Service Provider and/or MTC Application. |  |  |  |
| sliceUsgCtrlData |  | map(SliceUsageControlInfo) |  | C |  | 1..N |  | Represents the Network Slice Usage Control information to be provisioned. The key of the map shall be the AF-dedicated S-NSSAI (that is provided within the "snssai" attribute of the corresponding map value encoded via the SliceUsageControlInfo data structure) to which the provisioned Network Slice Usage Control information is related. This attribute shall be present only when the AF requests to provision Network Slice Usage Control information. |  |  |  |
| suppFeat |  | SupportedFeatures |  | C |  | 0..1 |  | Represents the list of supported features among the ones defined in clause 5.34.6. This attribute shall be present only when feature negotiation needs to take place. |  |  |  |

##### 5.34.5.2.3	Type: SlicePpDataPatch

Table 5.34.5.2.3-1: Definition of type SlicePpDataPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sliceUsgCtrlData | map(SliceUsageControlInfo) | O | 1..N | Represents the Network Slice Usage Control information to be provisioned. The key of the map shall be the AF-dedicated S-NSSAI (that is provided within the "snssai" attribute of the corresponding map value encoded via the SliceUsageControlInfo data structure) to which the provisioned Network Slice Usage Control information isrelated. |  |

#### 5.34.5.3	Simple data types and enumerations

##### 5.34.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.34.5.3.2	Simple data types

The simple data types defined in table 5.34.5.3.2-1 shall be supported.

Table 5.34.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

### 5.34.6	Used Features

The table below defines the features applicable to the SliceParamProvision API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.34.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 5.34.7	Error handling

#### 5.34.7.1	General

For the SliceParamProvision API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [4] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses are applicable for the SliceParamProvision API.

#### 5.34.7.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the SliceParamProvision API.

#### 5.34.7.3	Application Errors

The application errors defined for the SliceParamProvision API are listed in table 5.34.7.3-1.

Table 5.34.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

## 5.35	UeAddress API

### 5.35.1	Introduction

The Nnef_UeAddress service shall use the UeAddress API.

The API URI of UeAddress API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [4], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	"apiRoot" is set as described in clause 5.2.4 in 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-ue-address".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [4].

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.35.2	Resources

There are no resources defined for this API in this release of the specification.

### 5.35.3	Custom Operations without associated resources

#### 5.35.3.1	Overview

The structure of the custom operation URIs of the UeAddress API is shown in Figure 5.35.3.1-1.

Figure 5.35.3.1-1: Custom operation URI structure of the UeAddress API

Table 5.35.3.1-1 provides an overview of the custom operations and applicable HTTP methods defined for the UeAddress API.

Table 5.35.3.1-1: Custom operations without associated resources

| Custom operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Retrieve | /retrieve | POST | Enables an AF to request to retrieve UE Address information. |

The custom operations shall support the URI variables defined in table 5.35.3.1-2.

Table 5.35.3.1-2: URI variables for this custom operation

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.35.1. |

#### 5.35.3.2	Operation: Retrieve

##### 5.35.3.2.1	Description

The custom operation allows to retrieve UE Address information via the NEF.

##### 5.35.3.2.2	Operation Definition

This operation shall support the request and response data structures and response codes specified in table 5.35.3.2.2-1 and table 5.35.3.2.2-2.

Table 5.35.3.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UeAddressReq | M | 1 | Contains the parameters to request to retrieve UE Address information. |

Table 5.35.3.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UeAddressInfo | M | 1 | 200 OK | Successful case. The UE Address information request is successfully processed, and UE address information is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4] |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.35.3.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.35.3.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

### 5.35.4	Notifications

There are no Notifications defined for this API in this release of the specification.

### 5.35.5	Data Model

#### 5.35.5.1	General

This clause specifies the application data model supported by the UeAddress API. Table 5.35.5.1-1 specifies the data types defined for the UeAddress API.

Table 5.35.5.1-1: UeAddress service specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| UeAddressReq | 5.35.5.2.2 | Represents the parameters to request UE Address information. |  |
| UeAddressInfo | 5.35.5.2.3 | Represents UE Address information. |  |

Table 5.35.5.1-2 specifies data types re-used by the UeAddress API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the UeAddress API.

Table 5.35.5.1-2: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Gpsi | 3GPP TS 29.571 [8] | Represents a GPSI. |
| IpAddr | 3GPP TS 29.571 [8] | Represents an IP address. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Represents the list of supported feature(s) and is used to negotiate the applicability of the optional features. |

#### 5.35.5.2	Structured data types

##### 5.35.5.2.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.35.5.2.2	Type: UeAddressReq

Table 5.35.5.2.2-1: Definition of type UeAddressReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afId | string | M | 1 | Represents the identifier of the AF that is sending the request. |  |
| gpsi | Gpsi | M | 1 | Contains the target GPSI. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features used as defined in clause 5.35.6. This attribute shall be present only when feature negotiation needs to take place. |  |

##### 5.35.5.2.3	Type: UeAddressInfo

Table 5.35.5.2.3-1: Definition of type UeAddressInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ueIpAddrs | array(IpAddr) | M | 1..N | Contains the UE Address(es) corresponding to the requested GPSI. |  |

#### 5.35.5.3	Simple data types and enumerations

##### 5.35.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.35.5.3.2	Simple data types

The simple data types defined in table 5.35.5.3.2-1 shall be supported.

Table 5.35.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

#### 5.35.5.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

### 5.35.6	Used Features

The optional features listed in table 5.35.6-1 are defined for the UeAddress API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.35.6-1: Features used by UeAddress API

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 5.35.7	Error handling

#### 5.35.7.1	General

For the UeAddress API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [4] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses are applicable for the UeAddress API.

#### 5.35.7.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the UeAddress API.

#### 5.35.7.3	Application Errors

The application errors defined for the UeAddress API are listed in table 5.35.7.3-1.

Table 5.35.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

## 5.36	ECSAddress API

### 5.36.1	Introduction

The Nnef_ECSAddress service shall use the ECSAddress API.

The API URI of ECSAddress API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [4], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-ecs-address".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [4].

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.36.2	Resources

#### 5.36.2.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.36.2.1-1 and the resources and HTTP methods used for the ECSAddress API.

Figure 5.36.2.1-1: Resource URI structure of the northbound ECSAddress API

Table 5.36.2.1-1 provides an overview of the resources and HTTP methods applicable for the northbound ECSAddress API.

Table 5.36.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| ECS Address Configuration Information Sets | /{afId}/configurations | GET | Retrieve all the active ECS Address Configuration Information Sets. |
|  |  | POST | Create a new ECS Address Configuration Information Set. |
| Individual ECS Address Configuration Information Set | /{afId}/configurations/{configId} | GET | Retrieve an existing "Individual ECS Address Configuration Information Set" resource. |
|  |  | PUT | Update an existing "Individual ECS Address Configuration Information Set" resource. |
|  |  | PATCH | Modify an existing "Individual ECS Address Configuration Information Set" resource. |
|  |  | DELETE | Delete an existing "Individual ECS Address Configuration Information Set" resource. |

#### 5.36.2.2	Resource: ECS Address Configuration Information Sets

##### 5.36.2.2.1	Introduction

This resource represents all the active ECS Address Configuration Information Sets managed by the NEF for a given AF.

This resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [3]).

##### 5.36.2.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-ecs-address/<apiVersion>/{afId}/configurations

This resource shall support the resource URI variables defined in table 5.36.2.2.2-1.

Table 5.36.2.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.36.1. |
| afId | string | Represents the identifier of the AF. |

##### 5.36.2.2.3	Resource Standard Methods

###### 5.36.2.2.3.1	Void

###### 5.36.2.2.3.2	GET

The HTTP GET method allows to retrieve all the active ECS Address Configuration Information Sets at the NEF for a given AF.

This method shall support the URI query parameters specified in table 5.36.2.2.3.2-1.

Table 5.36.2.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.36.2.2.3.2-2, the response data structures and response codes specified in table 5.36.2.2.3.2-3, and the location headers specified in table 5.36.2.2.3.2-4 and table 5.36.2.2.3.2-5.

Table 5.36.2.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.36.2.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(EcsAddrInfo) | M | 0..N | 200 OK | Successful case. All the active "Individual ECS Address Configuration Information Set" resource(s) are returned. If there are no active "Individual ECS Address Configuration Information Set" resources at the NEF, an empty array is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.36.2.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.36.2.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.36.2.2.3.3	POST

The HTTP POST method allows to create a new ECS Address Configuration Information Set at the NEF for a given AF.

This method shall support the URI query parameters specified in table 5.36.2.2.3.3-1.

Table 5.36.2.2.3.3-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.36.2.2.3.3-1, the response data structures and response codes specified in table 5.36.2.2.3.3-2, and the Location Headers specified in table 5.36.2.2.3.3-3.

Table 5.36.2.2.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EcsAddrInfo | M | 1 | Contains the parameters to request the cretaion of an ECS Address Configuration Information Set. |

Table 5.36.2.2.3.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EcsAddrInfo | M | 1 | 201 Created | Successful case. A representation of the created "Individual ECS Address Configuration Information Set" resource is returned in the response body. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.36.2.2.3.3-3: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-ecs-address/<apiVersion>/{afId}/configurations/{configId} |

##### 5.36.2.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 5.36.2.3	Resource: Individual ECS Address Configuration Information Set

##### 5.36.2.3.1	Introduction

This resource represents an "Individual ECS Address Configuration Information Set" resource managed by the NEF.

##### 5.36.2.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-ecs-address/<apiVersion>/{afId}/configurations/{configId}

This resource shall support the resource URI variables defined in table 5.36.2.3.2-1.

Table 5.36.2.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.36.1. |
| afId | string | Represents the identifier of the AF. |
| configId | string | Represents the identifier of the "Individual ECS Address Configuration Information Set" resource. |

##### 5.36.2.3.3	Resource Standard Methods

###### 5.36.2.3.3.1	Void

###### 5.36.2.3.3.2	GET

The HTTP GET method allows to retrieve an existing "Individual ECS Address Configuration Information Set" resource at the NEF.

This method shall support the URI query parameters specified in table 5.36.2.3.3.2-1.

Table 5.36.2.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.36.2.3.3.2-2 and the response data structures and response codes specified in table 5.36.2.3.3.2-3.

Table 5.36.2.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.36.2.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EcsAddrInfo | M | 1 | 200 OK | Successful case. The requested "Individual ECS Address Configuration Information Set" resource is successfully returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.36.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.36.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.36.2.3.3.3	PUT

The HTTP PUT method allows to update an existing "Individual ECS Address Configuration Information Set" resource at the NEF.

This method shall support the URI query parameters specified in table 5.36.2.3.3.3-1.

Table 5.36.2.3.3.3-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.36.2.3.3.3-2 and the response data structures and response codes specified in table 5.36.2.3.3.3-3.

Table 5.36.2.3.3.3-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EcsAddrInfo | M | 1 | Contains the updated representation of the "Individual ECS Address Configuration Information Set" resource. |

Table 5.36.2.3.3.3-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EcsAddrInfo | M | 1 | 200 OK | Successful case. The "Individual ECS Address Configuration Information Set" resource is successfully updated and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual ECS Address Configuration Information Set" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.36.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.36.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.36.2.3.3.3A	PATCH

This method enables to request the modification of an existing "Individual ECS Address Configuration Information Set" resource at the NEF.

This method shall support the URI query parameters specified in table 5.36.2.3.3.3A-1.

Table 5.36.2.3.3.3A-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.36.2.3.3.3A-2 and the response data structures and response codes specified in table 5.36.2.3.3.3A-3.

Table 5.36.2.3.3.3A-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EcsAddrInfoPatch | M | 1 | Contains the requested modifications to the "Individual ECS Address Configuration Information Set" resource. |

Table 5.36.2.3.3.3A-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EcsAddrInfo | M | 1 | 200 OK | Successful response. The "Individual ECS Address Configuration Information Set" resource is successfully modified and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful response. The "Individual ECS Address Configuration Information Set" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.36.2.3.3.3A-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.36.2.3.3.3A-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.36.2.3.3.4	DELETE

The HTTP DELETE method allows to delete an existing "Individual ECS Address Configuration Information Set" resource at the NEF.

This method shall support the URI query parameters specified in table 5.36.2.3.3.4-1.

Table 5.36.2.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.36.2.3.3.4-2 and the response data structures and response codes specified in table 5.36.2.3.3.4-3.

Table 5.36.2.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.36.2.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | Successful case. The "Individual ECS Address Configuration Information Set" resource is successfully deleted. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.36.2.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.36.2.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

##### 5.36.2.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

### 5.36.3	Custom Operations without associated resources

#### 5.36.3.1	Overview

The structure of the custom operation URIs of the ECSAddress API is shown in Figure 5.36.3.1-1.

Figure 5.36.3.1-1: Custom operation URI structure of the ECSAddress API

Table 5.36.3.1-1 provides an overview of the custom operations and applicable HTTP methods defined for the ECSAddress API.

Table 5.36.3.1-1: Custom operations without associated resources

| Custom operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| RemoveEcsAddr | /remove-ecsaddr | POST | Enables to request the NEF to delete ECS Address Configuration information based on given attributes. |

The custom operations shall support the URI variables defined in table 5.36.3.1-2.

Table 5.36.3.1-2: URI variables for this custom operation

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.36.1. |

#### 5.36.3.2	Operation: RemoveEcsAddr

##### 5.36.3.2.1	Description

The custom operation enables to delete ECS Address Configuration information based on given attributes.

##### 5.36.3.2.2	Operation Definition

This operation shall support the request data structures shown in Table 5.36.3.2.2-1 and the response data structures and error codes specified in Table 5.36.3.2.2-2.

Table 5.36.3.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EcsAddrDeleteCriteria | M | 1 | Contains the information about the criteria to be used for ECS Address Configuration information deletion. |

Table 5.36.3.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The request to remove ECS Address Configuration information based on given criteria is successfully received and processed. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4] |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.36.3.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.36.3.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

### 5.36.4	Notifications

There are no notifications defined for this API in this release of the specification.

### 5.36.5	Data Model

#### 5.36.5.1	General

This clause specifies the application data model supported by the ECSAddress API. Table 5.36.5.1-1 specifies the data types defined for the ECSAddress API.

Table 5.36.5.1-1: ECSAddress API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| EcsAddrDeleteCriteria | 5.36.5.3.3 | Contains criteria for deleting ECS Address Configuration information. |  |
| EcsAddrInfo | 5.36.5.3.2 | Represents an ECS Address Configuration Information. |  |
| EcsAddrInfoPatch | 5.36.5.3.4 | Represents the requested modifications to an ECS Address Configuration Information. |  |

Table 5.36.5.1-2 specifies data types re-used by the ECSAddress API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the ECSAddress API.

Table 5.36.5.1-2: ECSAddress API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Dnn | 3GPP TS 29.571 [8] | Represents a DNN. |  |
| EcsAuthMethod | 3GPP TS 29.503 [17] | Represents the ECS Authentication Methods. |  |
| EcsServerAddr | 3GPP TS 29.571 [8] | Represents the Edge Configuration Server (ECS) address configuration information. |  |
| Link | 3GPP TS 29.122 [4] | Represents a link. |  |
| Snssai | 3GPP TS 29.571 [8] | Represents an S-NSSAI. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Represents the list of supported feature(s) and is used to negotiate the applicability of the optional features. |  |
| SupportedPlmn | 3GPP TS 29.503 [17] | ID of the PLMN supported by the ECS along with the list of ECSPs associated with the PLMN. |  |
| TargetUeId | 5.6.3.3.7 | Represents the target UE(s) information. |  |

#### 5.36.5.2	Void

#### 5.36.5.3	Structured data types

##### 5.36.5.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.36.5.3.2	Type: EcsAddrInfo

Table 5.36.5.3.2-1: Definition of type EcsAddrInfo

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| self |  | Link |  | C |  | 0..1 |  | Identifies the individual resource. This attribute shall be present in the HTTP GET response when retrieving all the resources for an AF. |  |  |  |
| ecsServerAddr |  | EcsServerAddr |  | M |  | 1 |  | Contains the ECS address(es). |  |  |  |
| spatialValidityCond |  | SpatialValidityCond |  | O |  | 0..1 |  | Contains the spatial validity conditions. |  |  |  |
| dnn |  | Dnn |  | O |  | 0..1 |  | Contains an HPLMN DNN. |  |  |  |
| snssai |  | Snssai |  | O |  | 0..1 |  | Contains an HPLMN S-NSSAI. |  |  |  |
| ecsAuthMethods |  | array(EcsAuthMethod) |  | O |  | 1..N |  | Contains the Supported ECS Authentication Method(s). |  |  |  |
| supportedPlmns |  | array(SupportedPlmn) |  | O |  | 1..N |  | The List of PLMNs and associated ECSPs for which EDN configuration information can be provided by the ECS. |  |  |  |
| suppFeat |  | SupportedFeatures |  | C |  | 0..1 |  | Contains the list of supported features used as defined in clause 5.36.6. This attribute shall be present only when feature negotiation needs to take place. |  |  |  |

##### 5.36.5.3.3	Type: EcsAddrDeleteCriteria

Table 5.36.5.3.3-1: Definition of type EcsAddrDeleteCriteria

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| afIds |  | array(string) |  | C |  | 1..N |  | Contains the targeted identifier(s) of the AF(s). (NOTE 1) |  |  |  |
| dnn |  | Dnn |  | C |  | 0..1 |  | Contains the DNN to be used as deletion criterion. (NOTE 1) |  |  |  |
| snssai |  | Snssai |  | C |  | 0..1 |  | Contains the S-NSSAI to be used as deletion criterion. (NOTE 1) |  |  |  |
| ecsAddrInfo |  | EcsAddrInfo |  | C |  | 0..1 |  | Contains the ECS Address Configuration information to be used as deletion criterion. Only entries that are exact matches of this attribute shall be deleted. (NOTE 1, NOTE 2) |  |  |  |
| suppFeat |  | SupportedFeatures |  | C |  | 0..1 |  | Contains the list of supported features used as defined in clause 5.36.6. This attribute shall be present only when feature negotiation needs to take place. |  |  |  |
| NOTE 1: At least one of these attributes shall be present. NOTE 2: The "self" and "suppFeat" attributes of the "ecsAddrInfo" attribute (encoded using the EcsAddrInfo data type) shall not be provided and shall not be considered when finding the matching entries. |  |  |  |  |  |  |  |  |  |  |  |

##### 5.36.5.3.4	Type: EcsAddrInfoPatch

Table 5.36.5.3.4-1: Definition of type EcsAddrInfoPatch

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ecsServerAddr |  | EcsServerAddr |  | O |  | 0..1 |  | Represents the updated ECS address(es) information. |  |  |  |
| ecsAuthMethods |  | array(EcsAuthMethod) |  | O |  | 1..N |  | Contains the Supported ECS Authentication Method(s). |  |  |  |
| supportedPlmns |  | array(SupportedPlmn) |  | O |  | 1..N |  | Contains the List of PLMNs and associated ECSPs for which EDN configuration information can be provided by the ECS. |  |  |  |
| spatialValidityCond |  | SpatialValidityCond |  | O |  | 0..1 |  | Contains the updated spatial validity conditions. |  |  |  |

#### 5.36.5.4	Simple data types and enumerations

##### 5.36.5.4.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.36.5.4.2	Simple data types

The simple data types defined in table 5.36.5.4.2-1 shall be supported.

Table 5.36.5.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

#### 5.36.5.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

### 5.36.6	Used Features

The optional features listed in table 5.36.6-1 are defined for the ECSAddress API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.36.6-1: Features used by ECSAddress API

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 5.36.7	Error handling

#### 5.36.7.1	General

For the ECSAddress API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [4] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses are applicable for the ECSAddress API.

#### 5.36.7.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the ECSAddress API.

#### 5.36.7.3	Application Errors

The application errors defined for ECSAddress API are listed in table 5.36.7.3-1.

Table 5.36.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

## 5.37	RSLPPIParametersProvisioning API

### 5.37.1	Introduction

The Nnef_ParameterProvision service shall use the RSLPPIParametersProvisioning API for:

-	RSLPPI Parameters provisioning.

The API URI of the RSLPPIParametersProvisioning API shall be:

- {apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [4], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	"apiRoot" is set as defined in clause 5.2.4 of 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-rslppi-pp".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [4].

All resource URIs in the clauses below are defined relative to the above API URI.

### 5.37.2	Resources

#### 5.37.2.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.37.2.1-1 and the resources and HTTP methods used for the RSLPPIParametersProvisioning API.

Figure 5.37.2.1-1: Resource URI structure of the RSLPPIParametersProvisioning API

Table 5.37.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.37.2.1-1: Resources and methods overview

| Resource name | Resource URI (relative path under API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| RSLPPI Parameters Provisionings | /pp | GET | Retrieve all the active RSLPPI Parameters Provisionings managed by the NEF. |
|  |  | POST | Request the creation of a new RSLPPI Parameters Provisioning at the NEF. |
| Individual RSLPPI Parameters Provisioning | /pp/{ppId} | GET | Retrieve an existing "Individual RSLPPI Parameters Provisioning" managed by the NEF. |
|  |  | PUT | Update an existing "Individual RSLPPI Parameters Provisioning" managed by the NEF. |
|  |  | PATCH | Modify an existing "Individual RSLPPI Parameters Provisioning" managed by the NEF. |
|  |  | DELETE | Delete an existing "Individual RSLPPI Parameters Provisioning" managed by the NEF. |

#### 5.37.2.2	Resource: RSLPPI Parameters Provisionings

##### 5.37.2.2.1	Introduction

This resource represents the collection of RSLPPI Parameters Provisionings managed by the NEF.

This resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [32]).

##### 5.37.2.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-rslppi-pp/<apiVersion>/pp

This resource shall support the resource URI variables defined in table 5.37.2.2.2-1.

Table 5.37.2.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.37.1. |

##### 5.37.2.2.3	Resource Methods

###### 5.37.2.2.3.1	GET

This method enables an AF to request to retrieve all the active "RSLPPI Parameters Provisionings" resources managed by the NEF.

This method shall support the URI query parameters specified in table 5.37.2.2.3.1-1.

Table 5.37.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.37.2.2.3.1-2 and the response data structures and response codes specified in table 5.37.2.2.3.1-3.

Table 5.37.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.37.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(RslppiPpData) | M | 0..N | 200 OK | Successful case. All the "Individual RSLPPI Parameters Provisioning" resources managed by the NEF are returned. If there are no existing "Individual RSLPPI Parameters Provisioning" resources managed at the NEF, an empty array is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |  |

Table 5.37.2.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.37.2.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.37.2.2.3.2	POST

This method enables an AF to request the creation of a new RSLPPI Parameters Provisioning at the NEF.

This method shall support the URI query parameters specified in table 5.37.2.2.3.2-1.

Table 5.37.2.2.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.37.2.2.3.2-2 and the response data structures and response codes specified in table 5.37.2.2.3.2-3.

Table 5.37.2.2.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| RslppiPpData | M | 1 | Contains the representation of the RSLPPI Parameters Provisioning to be created at the NEF. |

Table 5.37.2.2.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| RslppiPpData | M | 1 | 201 Created | Successful case. A representation of the created "Individual RSLPPI Parameters Provisioning" resource is returned in the response body. The URI of the created resource shall be returned in an HTTP "Location" header. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.37.2.2.3.2-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-rslppi-pp/<apiVersion>/pp/{ppId} |

##### 5.37.2.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 5.37.2.3	Resource: Individual RSLPPI Parameters Provisioning

##### 5.37.2.3.1	Introduction

This resource represents an "Individual RSLPPI Parameters Provisioning" resource managed by the NEF.

This resource is modelled with the Document resource archetype (see clause C.2 of 3GPP TS 29.501 [32]).

##### 5.37.2.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-rslppi-pp/<apiVersion>/pp/{ppId}

This resource shall support the resource URI variables defined in table 5.37.2.3.2-1.

Table 5.37.2.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.37.1. |
| ppId | string | Represents the identifier of the "Individual RSLPPI Parameters Provisioning" resource. |

##### 5.37.2.3.3	Resource Methods

###### 5.37.2.3.3.1	GET

This method enables an AF to request to retrieve an existing "Individual RSLPPI Parameters Provisioning" resource at the NEF.

This method shall support the URI query parameters specified in table 5.37.2.3.3.1-1.

Table 5.37.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.37.2.3.3.1-2 and the response data structures and response codes specified in table 5.37.2.3.3.1-3.

Table 5.37.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.37.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| RslppiPpData | M | 1 | 200 OK | Successful case. The requested "Individual RSLPPI Parameters Provisioning" resource is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.37.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.37.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

###### 5.37.2.3.3.2	PUT

This method enables an AF to request the update of an existing "Individual RSLPPI Parameters Provisioning" resource at the NEF.

This method shall support the URI query parameters specified in table 5.37.2.3.3.2-1.

Table 5.37.2.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.37.2.3.3.2-2 and the response data structures and response codes specified in table 5.37.2.3.3.2-3.

Table 5.37.2.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| RslppiPpData | M | 1 | Represents the updated "Individual RSLPPI Parameters Provisioning" resource representation. |

Table 5.37.2.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |  |
| --- | --- | --- | --- | --- | --- |
| RslppiPpData | M | 1 | 200 OK | Successful response. The "Individual RSLPPI Parameters Provisioning" resource is successfully updated and a representation of the updated resource is returned in the response body. |  |
| n/a |  |  | 204 No Content | Successful response. The "Individual RSLPPI Parameters Provisioning" resource is successfully updated and no content is returned in the response body. |  |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.37.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

Table 5.37.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

###### 5.37.2.3.3.3	PATCH

This method enables an AF to request the modification of an existing "Individual RSLPPI Parameters Provisioning" resource at the NEF.

This method shall support the URI query parameters specified in table 5.37.2.3.3.3-1.

Table 5.37.2.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.37.2.3.3.3-2 and the response data structures and response codes specified in table 5.37.2.3.3.3-3.

Table 5.37.2.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| RslppiPpDataPatch | M | 1 | Represents the requested modifications to the "Individual RSLPPI Parameters Provisioning" resource. |

Table 5.37.2.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |  |
| --- | --- | --- | --- | --- | --- |
| RslppiPpData | M | 1 | 200 OK | Successful response. The "Individual RSLPPI Parameters Provisioning" resource is successfully modified and a representation of the updated resource is returned in the response body. |  |
| n/a |  |  | 204 No Content | Successful response. The "Individual RSLPPI Parameters Provisioning" resource is successfully modified and no content is returned in the response body. |  |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |  |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.37.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

Table 5.37.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NEF. |

###### 5.37.2.3.3.4	DELETE

This method enables an AF to request the deletion of an existing "Individual RSLPPI Parameters Provisioning" resource at the NEF.

This method shall support the URI query parameters specified in table 5.37.2.3.3.4-1.

Table 5.37.2.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.37.2.3.3.4-2 and the response data structures and response codes specified in table 5.37.2.3.3.4-3.

Table 5.37.2.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.37.2.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual RSLPPI Parameters Provisioning" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] shall also apply. |  |  |  |  |

Table 5.37.2.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

Table 5.37.2.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative NEF. |

##### 5.37.2.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

### 5.37.3	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.37.4	Notifications

There are no notifications defined for this API in this release of the specification.

### 5.37.5	Data Model

#### 5.37.5.1	General

This clause specifies the application data model supported by the RSLPPIParametersProvisioning API. Table 5.37.5.1-1 specifies the data types defined for the RSLPPIParametersProvisioning API.

Table 5.37.5.1-1: RSLPPIParametersProvisioning API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| RslppiData | 5.37.5.2.2 | Represents RSLPPI parameters data. |  |
| RslppiPpData | 5.37.5.2.3 | Represents RSLPPI Parameters Provisioning data. |  |
| RslppiPpDataPatch | 5.37.5.2.4 | Represents the requested modification to an existing RSLPPI Parameters Provisioning data. |  |

Table 5.37.5.1-2 specifies data types re-used by the RSLPPIParametersProvisioning API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the RSLPPIParametersProvisioning API.

Table 5.37.5.1-2: RSLPPIParametersProvisioning API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ExternalGroupId | 3GPP TS 29.122 [4] | Represents the External Group Identifier for a user group. |  |
| Gpsi | 3GPP TS 29.571 [8] | Represents a GPSI. |  |
| Rslppi | 3GPP TS 29.503 [17] | Represents the RSLPPI information. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |

#### 5.37.5.2	Structured data types

##### 5.37.5.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 5.37.5.2.2	Type: RslppiData

Table 5.37.5.2.2-1: Definition of type RslppiData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| extGroupId | ExternalGroupId | C | 0..1 | Represents the external group identifier of the targeted group. (NOTE) |  |
| gpsi | Gpsi | C | 0..1 | Represents the GPSI of the targeted UE. (NOTE) |  |
| rslppi | Rslppi | M | 1 | Contains the RSLPPI parameters. |  |
| NOTE: These attributes are mutually exclusive. Either one of them shall be present. |  |  |  |  |  |

##### 5.37.5.2.3	Type: RslppiPpData

Table 5.37.5.2.3-1: Definition of type RslppiPpData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afId | string | M | 1 | Contains the identifier of the AF that is sending the request. |  |
| rslppiData | RslppiData | C | 0..1 | Contains the RSLPPI data that the AF requests to provision. This attribute shall be present only when the AF requests to provision RSLPPI parameters. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 5.37.6. This attribute shall be present when only feature negotiation needs to take place. |  |

##### 5.37.5.2.4	Type: RslppiPpDataPatch

Table 5.37.5.2.4-1: Definition of type RslppiPpDataPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| rslppiData | RslppiData | O | 0..1 | Contains the modified RSLPPI data that the AF requests to provision. |  |

#### 5.37.5.3	Simple data types and enumerations

##### 5.37.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.37.5.3.2	Simple data types

The simple data types defined in table 5.37.5.3.2-1 shall be supported.

Table 5.37.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

#### 5.37.5.4	Data types describing alternative data types or combinations of data types

There are no custom operations without associated resources defined for this API in this release of the specification.

### 5.37.6	Used Features

The table below defines the features applicable to the RSLPPIParametersProvisioning API. Those features are negotiated as described in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.37.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 5.37.7	Error handling

#### 5.37.7.1	General

For the RSLPPIParametersProvisioning API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [4] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses are applicable for the RSLPPIParametersProvisioning API.

#### 5.37.7.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the RSLPPIParametersProvisioning API.

#### 5.37.7.3	Application Errors

The application errors defined for the RSLPPIParametersProvisioning API are listed in table 5.37.7.3-1.

Table 5.37.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

# 6	Security

TLS shall be used to support secure communications between the NEF and the AF over the NEF Northbound interface as defined in clause 12.3 of 3GPP TS 33.501 [6].

When CAPIF is not supported and/or not used, the access to the NEF Northbound APIs shall be authorized by means of OAuth2 protocol (see IETF RFC 6749 [13]), based on local configuration, using the "Client Credentials" authorization grant as defined in clause 12.4 of 3GPP TS 33.501 [6]. When OAuth2 is used, a client shall obtain a "token" from the authorization server prior to consuming services offered by any NEF Northbound API.

When the NEF supports CAPIF for external exposure and CAPIF is used, the provisions of clause 7.2 shall apply for authorizing the access to the NEF Northbound APIs.

# 7	Using Common API Framework

## 7.1	General

When CAPIF is used with an NEF that is used for external exposure, the NEF shall support the following as defined in 3GPP TS 29.222 [12]:

-	the API exposing function and related APIs over CAPIF-2/2e and CAPIF-3/3e reference points;

-	the API publishing function and related APIs over CAPIF-4/4e reference point;

-	the API management function and related APIs over CAPIF-5/5e reference point; and

-	at least one of the security methods for authentication and authorization, and related security mechanisms.

In a centralized deployment as defined in 3GPP TS 23.222 [11], where the CAPIF core function and API provider domain functions are co-located, the interactions between the CAPIF core function and API provider domain functions may be independent of CAPIF-3/3e, CAPIF-4/4e and CAPIF-5/5e reference points.

## 7.2	Security

When CAPIF is used for external exposure, before invoking the API exposed by the NEF, the AF as API invoker shall negotiate the security method (PKI, TLS-PSK or OAUTH2) with CAPIF core function and ensure the NEF has enough credential to authenticate the AF (see 3GPP TS 29.222 [12], clause 5.6.2.2 and clause 6.2.2.2).

If PKI or TLS-PSK is used as the selected security method between the AF and the NEF, upon API invocation, the NEF shall retrieve the authorization information from the CAPIF core function as described in 3GPP TS 29.222 [12], clause 5.6.2.4.

As indicated in 3GPP TS 33.122 [14], the access to the NEF northbound APIs may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [13]), using the "Client Credentials" authorization grant, where the CAPIF core function (see 3GPP TS 29.222 [12]) plays the role of the authorization server.

NOTE 1:	In this release, only "Client Credentials" authorization grant is supported.

If OAuth2 is used as the selected security method between the AF and the NEF, the AF, prior to consuming services offered by the NEF northbound APIs, shall obtain a "token" from the authorization server, by invoking the Obtain_Authorization service, as described in 3GPP TS 29.222 [12], clause 5.6.2.3.2.

The NEF northbound APIs do not define any scopes for OAuth2 authorization. It is the NEF responsibility to check whether the AF is authorized to use an API based on the "token". Once the NEF verifies the "token", it shall check whether the NEF identifier in the "token" matches its own published identifier, and whether the API name in the "token" matches its own published API name. If those checks are passed, the AF has full authority to access any resource or operation for the invoked API.

NOTE 2:	For aforementioned security methods, the NEF needs to apply admission control according to access control policies after performing the authorization checks.

NOTE 3:	The security requirement in the current clause does not apply for the NiddConfigurationTrigger and the MsisdnLessMoSms APIs since they are the NEF initiated interaction with the AF. How the security scheme works for the NiddConfigurationTrigger and MsisdnLessMoSms APIs is left to configuration.