| 3GPP TS 29.576 V0.3.0 (2021-11) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Messaging Framework Adaptor Services Stage 3 (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions, symbols and abbreviations	9

3.1	Definitions	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Services offered by the MFAF	9

4.1	Introduction	9

4.2	Nmfaf_3daDataManagement Service	10

4.2.1	Service Description	10

4.2.1.1	Overview	10

4.2.1.2	Service Architecture	11

4.2.1.3	Network Functions	11

4.2.1.3.1	Messaging Framework Adaptor Function (MFAF)	11

4.2.1.3.2	NF Service Consumers	11

4.2.2	Service Operations	11

4.2.2.1	Introduction	11

4.2.2.2	Nmfaf_3daDataManagement_Configure service operation	12

4.2.2.2.1	General	12

4.2.2.2.2	Initial configuration for mapping data or analytics	12

4.2.2.2.3	Update the configuration of existing individual mapping data or analytics	13

4.2.2.3	Nmfaf_3daDataManagement_Deconfigure  service operation	13

4.2.2.3.1	General	13

4.2.2.3.2	Stop mapping data or analytics	14

4.3	Nmfaf_3caDataManagement Service	14

4.3.1	Service Description	14

4.3.1.1	Overview	14

4.3.1.2	Service Architecture	14

4.3.1.3	Network Functions	15

4.3.1.3.1	Messaging Framework Adaptor Function (MFAF)	15

4.3.1.3.2	NF Service Consumers	15

4.3.2	Service Operations	15

4.3.2.1	Introduction	15

4.3.2.2	Nmfaf_3caDataManagement_Fetch service operation	15

4.3.2.2.1	General	15

4.3.2.2.n	<Procedure n using Nmfaf_3caDataManagement_Fetch service operation>	16

4.3.2.3	Nmfaf_3caDataManagement_Notify service operation	16

4.3.2.3.1	General	16

4.3.2.3.n	<Procedure n using Nmfaf_3caDataManagement_Notify service operation>	16

5	API Definitions	16

5.1	Nmfaf_3daDataManagement Service API	16

5.1.1	Introduction	16

5.1.2	Usage of HTTP	16

5.1.2.1	General	16

5.1.2.2	HTTP standard headers	17

5.1.2.2.1	General	17

5.1.2.2.2	Content type	17

5.1.2.3	HTTP custom headers	17

5.1.3	Resources	17

5.1.3.1	Overview	17

5.1.3.2	Resource: MFAF Configurations	18

5.1.3.2.1	Description	18

5.1.3.2.2	Resource Definition	18

5.1.3.2.3	Resource Standard Methods	18

5.1.3.2.3.1	POST	18

5.1.3.2.4	Resource Custom Operations	19

5.1.3.3	Resource: Individual MFAF Configuration	19

5.1.3.2.1	Description	19

5.1.3.3.2	Resource Definition	19

5.1.3.3.3	Resource Standard Methods	19

5.1.3.3.3.1	PUT	19

5.1.3.3.3.2	DELETE	20

5.1.4	Custom Operations without associated resources	20

5.1.5	Notifications	20

5.1.6	Data Model	20

5.1.6.1	General	20

5.1.6.2	Structured data types	21

5.1.6.2.1	Introduction	21

5.1.6.2.2	Type: MfafConfiguration	21

5.1.6.2.3	Type: MessageConfiguration	22

5.1.6.2.3	Type: MfafNotiInfo	22

5.1.6.3	Simple data types and enumerations	22

5.1.6.3.1	Introduction	22

5.1.6.3.2	Simple data types	22

5.1.6.3.3	Enumeration: <EnumType1>	23

5.1.6.3.4	Enumeration: <EnumType2>	23

5.1.6.4	Data types describing alternative data types or combinations of data types	23

5.1.6.4.1	Type: <TypeName 1>	23

5.1.6.4.2	Type: <TypeName 2>	24

5.1.6.5	Binary data	24

5.1.6.5.1	Binary Data Types	24

5.1.7	Error Handling	24

5.1.7.1	General	24

5.1.7.2	Protocol Errors	24

5.1.7.3	Application Errors	25

5.1.8	Feature negotiation	25

5.1.9	Security	25

5.2	Nmfaf_3caDataManagement Service API	25

5.2.1	Introduction	25

5.2.2	Usage of HTTP	26

5.2.2.1	General	26

5.2.2.2	HTTP standard headers	26

5.2.2.2.1	General	26

5.2.2.2.2	Content type	26

5.2.2.3	HTTP custom headers	26

5.2.3	Resources	26

5.2.3.1	Overview	26

5.2.3.2	Resource: <resource 1>	27

5.2.3.2.1	Description	27

5.2.3.2.2	Resource Definition	27

5.2.3.2.3	Resource Standard Methods	28

5.2.3.2.3.1	< method 1 >	28

5.2.3.2.3.2	< method 2 >	29

5.2.3.2.4	Resource Custom Operations	29

5.2.3.2.4.1	Overview	29

5.2.3.2.4.2	Operation: < operation 1 >	29

5.2.3.2.4.2.1	Description	29

5.2.3.2.4.2.2	Operation Definition	30

5.2.3.2.4.3	Operation: < operation 2 >	30

5.2.3.3	Resource: <resource 2>	30

5.2.4	Custom Operations without associated resources	30

5.2.4.1	Overview	30

5.2.4.2	Operation: <operation 1>	30

5.2.4.2.1	Description	31

5.2.4.2.2	Operation Definition	31

5.2.4.3	Operation: < operation 2>	31

5.2.5	Notifications	31

5.2.5.1	General	31

5.2.5.2	MFAF Notification	32

5.2.5.2.1	Description	32

5.2.5.2.2	Target URI	32

5.2.5.2.3	Standard Methods	32

5.2.5.2.3.1	POST	32

5.2.5.3	<notification 2>	32

5.2.6	Data Model	32

5.2.6.1	General	32

5.2.6.2	Structured data types	33

5.2.6.2.1	Introduction	33

5.2.6.2.2	Type: <TypeName 1>	33

5.2.6.2.3	Type: <TypeName 2>	34

5.2.6.3	Simple data types and enumerations	34

5.2.6.3.1	Introduction	34

5.2.6.3.2	Simple data types	34

5.2.6.3.3	Enumeration: <EnumType1>	34

5.2.6.3.4	Enumeration: <EnumType2>	34

5.2.6.4	Data types describing alternative data types or combinations of data types	34

5.2.6.4.1	Type: <TypeName 1>	34

5.2.6.4.2	Type: <TypeName 2>	35

5.2.6.5	Binary data	35

5.2.6.5.2	Binary Data Types	35

5.2.7	Error Handling	36

5.2.7.1	General	36

5.2.7.2	Protocol Errors	36

5.2.7.3	Application Errors	36

5.2.8	Feature negotiation	36

5.2.9	Security	36

Annex A (normative): OpenAPI specification	38

A.1	General	38

A.2	Nmfaf_3daDataManagement API	38

A.3	Nmfaf_3caDataManagement API	38

Annex B (informative): Change history	39

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

This clause is optional. If it exists, it is always the second unnumbered clause.

1	Scope

This clause will describe the scope of the corresponding service specification.

The present document specifies the stage 3 protocol and data model for the MFAF Service Based Interface. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the MFAF.

The 5G System stage 2 architecture and procedures are specified in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3]. The stage 2 definition and procedures of Messaging Framework Adaptation are contained in 3GPP TS 23.288 [14] and 3GPP TS 23.502 [3].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [4] and 3GPP TS 29.501 [5].

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 7807: "Problem Details for HTTP APIs".

[14]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services"

[15]	3GPP TS 29.574: "5G System; Data Collection Coordination Services; Stage 3".

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

MFAF	Messaging Framework Adaptor Function

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ACRONYM>	<Explanation>

4	Services offered by the MFAF

4.1	Introduction

The Messaging Framework Adaptor Services are used for the Messaging Framework Adaptor Function (MFAF) to enable the 5GS to interact with the messaging framework using Nmfaf services. The MFAF offers to other NFs the following services:

-	Nmfaf_3daDataManagement; and

-	Nmfaf_3caDataManagement.

Table 4.1-1: Service provided by MFAF

| Service Name | Description | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- | --- |
| Nmfaf_3daDataManagement | 3GPP DCCF Adaptor (3DA) Data Management Service enables the DCCF to convey to the messaging framework, information about the data the messaging framework will receive from a Data Source, formatting and processing instructions and the Data Consumer and notification endpoints. | Configure | Request / Response | DCCF |
|  |  | Deconfigure | Request / Response | DCCF |
| Nmfaf_3caDataManagement | 3GPP Consumer Adaptor (3CA) Data Management Service delivers data to each Data Consumer or notification endpoint after formatting and processing of data received by the messaging framework. | Notify | Subscribe / Notify | NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, UDM, OAM, CEF |
|  |  | Fetch | Request / Response | NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, UDM, OAM, CEF |
| NOTE: The services correspond to the Nmfaf_3caDataManagement service and Nmfaf_3daDataManagement service as defined in 3GPP TS 23.288 [14]. |  |  |  |  |

Table 4.1-2 summarizes the corresponding APIs defined for this specification.

Table 4.1-x: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Nmfaf_3daDataManagement | 4.2 | API for Nmfaf_3daDataManagement |  | nmfaf_3dadatamanagement | Annex A.2 Nmfaf_3daDataManagement API |
| Nmfaf_3caDataManagement | 4.3 | API for Nmfaf_3caDataManagement |  | nmfaf_3cadatamanagement | Annex A.3 Nmfaf_3caDataManagement API |

4.2	Nmfaf_3daDataManagement Service

4.2.1	Service Description

4.2.1.1	Overview

The Nmfaf_3daDataManagement service as defined in 3GPP TS 23.288 [14], is provided by the Messaging Framework Adaptor Function (MFAF).

This service:

-	allows NF consumers to configure or reconfigure the MFAF to map data or analytics received by the MFAF to out-bound notification endpoints; and

-	allows NF consumers to reconfigure the MFAF to stop mapping data or analytics received by the MFAF to out-bound notification endpoints.

4.2.1.2	Service Architecture

The 5G System Architecture is defined in 3GPP TS 23.501 [2]. The Network Data Analytics Exposure architecture is defined in 3GPP TS 23.288 [14].

The Nmfaf_3daDataManagement service is part of the Nmfaf service-based interface exhibited by the Messaging Framework Adaptor Function (MFAF).

Known consumer of the Nmfaf_3daDataManagement service is:

-	Data Collection Coordination Function (DCCF)

Figure 4.2.1.2-1: Reference Architecture for the Nmfaf_3daDataManagement Service; SBI representation

4.2.1.3	Network Functions

4.2.1.3.1	Messaging Framework Adaptor Function (MFAF)

The Messaging Framework Adaptor Function (MFAF) provides the functionality to allow NF consumers to configure or reconfigure the behaviour of mapping data or analytics received by the MFAF to out-bound notification endpoints.

4.2.1.3.2	NF Service Consumers

The Data Collection Coordination Function (DCCF):

-	supports configuring the MFAF to map data or analytics received by the MFAF to out-bound notification endpoints and to format and process the out-bound data or analytics; and

-	supports reconfiguring the MFAF to stop the sending of data to consumers.

4.2.2	Service Operations

4.2.2.1	Introduction

Table 4.2.2.1-1: Operations of the Nmfaf_3daDataManagement Service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Nmfaf_3daDataManagement_Configure | This service operation is used by an NF to configure or reconfigure the MFAF to map data or analytics received by the MFAF to out-bound notification endpoints and to format and process the outbound data or analytics | NF service consumer (e.g. DCCF) |
| Nmfaf_3daDataManagement_Deconfigure | This service operation is used by an NF to stop mapping data or analytics received by the MFAF to one or more outbound notification endpoints. | NF service consumer (e.g. DCCF) |

4.2.2.2	Nmfaf_3daDataManagement_Configure service operation

4.2.2.2.1	General

The Nmfaf_3daDataManagement_Configure service operation is used by an NF service consumer to configure or update the configuration of the MFAF for mapping data or analytics received by the MFAF to out-bound notification endpoints, and formatting and processing the out-bound data or analytics.

4.2.2.2.2	Initial configuration for mapping data or analytics

Figure 4.2.2.2.2-1 shows a scenario where the NF service consumer (e.g. DCCF) sends a request to the MFAF to request the configuration of mapping data or analytics (as shown in 3GPP TS 23.288 [14]).

Figure 4.2.2.2.2-1: NF service consumer create the configuration

The NF service consumer shall invoke the Nmfaf_3daDataManagement_Configure service operation to create the configuration(s). The NF service consumer shall send an HTTP POST request with "{apiRoot}/nmfaf-3dadatamanagement/v1/configurations" as Resource URI representing the "MFAF Configurations", as shown in figure 4.2.2.2.2-1, step 1, to create a configuration for an "Individual MFAF Configurations" according to the information in message body. The MfafConfiguration data structure provided in the request body

shall include:

-	a description of the configurations as "messageConfigurations" attribute that, for each configuration, the MessageConfiguration data type shall include

1)	a notification URI of Data Consumer or Analytics Consumer or other endpoint where to receive the requested mapping data or analytics as "notificationURI" attribute; and

2)-	if the configuration is used for mapping analytics or data collection, a Notification Correlation ID for the Data or Analytics Consumer (or other endpoint) as "correId" attribute;

Editor's Note:	whether the Analytics Consumer Notification Correlation ID could be used for mapping data is FFS.

and may include:

1)	 the formatting instructions as "formatIstruct" attribute;

2)	the processing instructions as "processIntruct" attribute; and

3)	the MFAF notification information to identify the MFAF endpoint where Data or Analytics Event Notifications should be received as "mfafNotiInfo" attribute;

Upon the reception of an HTTP POST request with: "{apiRoot}/nmfaf-3dadatamanagement/v1/configurations" as Resource URI and MfafConfiguration data structure as request body, the MFAF shall:

-	create a new configuration;

-	assign a transaction reference id;

-	if no MFAF notification information has been provided in the request, determine the MFAF notification information and add it to the configuration that is created and will be returned to the NF service consumer;

-	store the configuration.

If the MFAF created an "Individual MFAF Configuration" resource, the MFAF shall respond with "201 Created" with the message body containing a representation of the created subscription, as shown in figure 4.2.2.2.2-1, step 2.

4.2.2.2.3	Update the configuration of existing individual mapping data or analytics

Figure 4.2.2.2.3-1 shows a scenario where the NF service consumer sends a request to the MFAF to update the configuration of mapping data or analytics (as shown in 3GPP TS 23.288 [14])

Figure 4.2.2.2.3-1: NF service consumer updates configuration

The NF service consumer shall invoke the Nmfaf_3daDataManagement_Configure service operation to update the configuration(s). The NF service consumer shall send an HTTP PUT request with "{apiRoot}/nmfaf-3dadatamanagement/v1/configuration/{transRefId}" as Resource URI representing the "Individual MFAF Configuration", as shown in figure 4.2.2.2.3-1, step 1, to update the subscription for an "Individual MFAF Configuration" resource identified by the {transRefId}. The MfafConfiguration data structure provided in the request body shall include:

-	a description of the configurations as "messageConfigurations" attribute that, for each configuration, the MfafConfiguration data structure provided in the request body shall include the same contents as described in 4.2.2.2.2.

Upon the reception of an HTTP PUT request with: "{apiRoot}/nmfaf-3dadatamanagement/v1/configuration/{transRefId}" as Resource URI and MfafConfiguration data structure as request body, the MFAF shall:

-	update the configuration of corresponding transaction reference Id; and

-	store the configuration.

If the MFAF successfully processed and accepted the received HTTP PUT request, the MFAF shall update an "Individual MFAF Configuration" resource, and shall respond with:

a)	HTTP "200 OK" status code with the message body containing a representation of the updated configuration, as shown in figure 4.2.2.2.3-1, step 2a. or

b)	HTTP "204 No Content" status code, as shown in figure 4.2.2.2.3-1, step 2b.

4.2.2.3	Nmfaf_3daDataManagement_Deconfigure  service operation

4.2.2.3.1	General

The Nmfaf_3daDataManagement_Deconfigure service operation is used by an NF service consumer to stop mapping data or analytics received by the MFAF to one or more out-bound notification endpoints.

4.2.2.3.2	Stop mapping data or analytics

Figure 4.2.2.3.2-1 shows a scenario where the NF service consumer sends a request to the MFAF to update the configuration to stop mapping data or analytics (as shown in 3GPP TS 23.288 [14])

Figure 4.2.2.3.2-1: NF service consumer stops mapping data or analytics

The NF service consumer shall invoke the Nmfaf_3daDataManagement_Deconfigure service operation to stop the MFAF to map data or analytics. The NF service consumer shall send an HTTP DELETE request with "{apiRoot}/nmfaf-3dadatamanagement/v1/configurations/{transRefId}" as Resource URI, where {transRefId} represents the "Individual MFAF Configuration" to be deleted, as shown in figure 4.2.2.3.2-1, step 1.

Upon the reception of an HTTP DELETE request and if the MFAF successfully processed and accepted the received HTTP DELETE request from the NF service consumer, the MFAF shall acknowledge the request by sending a "204 No Content" response to the NF service consumer, as shown in figure 4.2.2.3.2-1, step 2. Further, the MFAF shall remove the individual resource linked to the delete request.

4.3	Nmfaf_3caDataManagement Service

4.3.1	Service Description

4.3.1.1	Overview

The Nmfaf_3caDataManagement service as defined in 3GPP TS 23.288 [14], is provided by the Messaging Framework Adaptor Function (MFAF).

This service:

-	allows NF consumers to collect the data or analytics from the MFAF.

4.3.1.2	Service Architecture

The 5G System Architecture is defined in 3GPP TS 23.501 [2]. The Network Data Analytics Exposure architecture is defined in 3GPP TS 23.288 [14].

The Nmfaf_3caDataManagement service is part of the Nmfaf service-based interface exhibited by the Messaging Framework Adaptor Function (MFAF).

Known consumers of the Nmfaf_3caDataManagement service are:

-	Network Data Analytics Function (NWDAF) ;

-	Policy Control Function (PCF)

-	Network Slice Selection Function (NSSF)

-	Access and Mobility Management Function (AMF)

-	Session Management Function (SMF)

-	Network Exposure Function (NEF)

-	Application Function (AF)

Figure 4.3.1.2-1: Reference Architecture for the Nmfaf_3caDataManagement Service; SBI representation

4.3.1.3	Network Functions

4.3.1.3.1	Messaging Framework Adaptor Function (MFAF)

The Messaging Framework Adaptor Function (MFAF) provides the functionality to supply data or analytics, or an indication of availability of data or analytics to notification endpoints configured in Nmfaf_3caDataManagement service as described in clause 4.2.1.

4.3.1.3.2	NF Service Consumers

Editor's Note:	The NF Service consumers should be added. Currently there is no difference identified between different consumers.

4.3.2	Service Operations

One clause per service operation.

This clause will include a description of the different service operationssupported by the service. For RESTful service operations, the service operations depict the resources and the methods they support.

4.3.2.1	Introduction

This clause will contain a generic introduction of the service operationsdescribed in the following clauses.

4.3.2.2	Nmfaf_3caDataManagement_Fetch service operation

4.3.2.2.1	General

The Nmfaf_3caDataManagement_Fetch service operation allows consumer to retrieves data or analytics from the MFAF as indicated by Nmfaf_3caDataManagement_Notify Fetch Instruction.

4.3.2.2.n	<Procedure n using Nmfaf_3caDataManagement_Fetch service operation>

4.3.2.3	Nmfaf_3caDataManagement_Notify service operation

4.3.2.3.1	General

The Nmfaf_3caDataManagement_Notify service operation provides data or analytics or notification of availability of data or analytics to notification endpoints.

4.3.2.3.n	<Procedure n using Nmfaf_3caDataManagement_Notify service operation>

5	API Definitions

5.1	Nmfaf_3daDataManagement Service API

One clause per service, where <service 1> is to be replaced by the service name (e.g. Nsmf_PDUSession).

5.1.1	Introduction

This clause specifies the API Name and Version.

The Nmfaf_3daDataManagement shall use the Nmfaf_3daDataManagement  API.

The API URI of the Nmfaf_3daDataManagement API shall be:

{apiRoot}/<apiName>/<apiVersion>/

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nmfaf-3dadatamanagement".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 5.1.3.

5.1.2	Usage of HTTP

5.1.2.1	General

This clause will include a reference to TS 29.500 for the description of the Transport and HTTP/2.0 protocol requirements and for the security requirements.

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Nmfaf_3daDataManagement API is contained in Annex A.

5.1.2.2	HTTP standard headers

5.1.2.2.1	General

See clause 5.1.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

Add specific information for the API if applicable.

5.1.2.2.2	Content type

This clause will indicate the encoding of HTTP requests/responses and the applicable MIME media type for the related Content-Type header. Adjust the text below if additional payload types are used e.g. for HATEOS.

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

5.1.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.1.3.2 of 3GPP TS 29.500 [4] shall be applicable.

Add specific information for the API if applicable.

5.1.3	Resources

5.1.3.1	Overview

This clause will describe the structure for the Resource URIs and the resources and methods used for the service.

Figure 5.1.3.1-1: Resource URI structure of the Nmfaf_3daDataManagement API

Table 5.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| MFAF Configurations | /configurations | POST | Creates a new individual MFAF Configuration resource. |
| Individual MFAF Configuration | /configurations/{transRefId} | PUT | Modifies an existing Individual MFAF Configuration subresource. |
|  |  | DELETE | Deletes an Individual MFAF Configuration identified by subresource {transRefId}. |

5.1.3.2	Resource: MFAF Configurations

5.1.3.2.1	Description

The MFAF Configurations resource represents all configuration to the Nmfaf_3daDataManagement Service at a given MFAF. The resource allows an NF service consumer to create a new Individual MFAF Configuration resource.

5.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/nmfaf-3dadatamanagement/<apiVersion>/configurations

This resource shall support the resource URI variables defined in table 5.1.3.2.2-1.

Table 5.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.1.1 |
| apiVersion | string | See clause 5.1.1 |

5.1.3.2.3	Resource Standard Methods

5.1.3.2.3.1	POST

This method shall support the URI query parameters specified in table 5.1.3.2.3.1-1.

Table 5.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.1.3.2.3.1-2 and the response data structures and response codes specified in table 5.1.3.2.3.1-3.

Table 5.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MfafConfiguration | M | 1 | Create a new Individual MFAF Configuration resource. |

Table 5.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MfafConfiguration | M | 1 | 201 Created | The creation of an Individual MFAF Configuration resource is confirmed and a representation of that resource is returned. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 5.1.3.2.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nmfaf-3dadatamanagement/v1/configurations/{transRefId} |

5.1.3.2.4	Resource Custom Operations

None in this release of the specification.

5.1.3.3	Resource: Individual MFAF Configuration

5.1.3.2.1	Description

The Individual MFAF Configurations resource represents an individual configuration created in the MFAF and associated with transaction reference Id.

5.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/nmfaf-3dadatamanagement/<apiVersion>/configurations/{transRefId}

This resource shall support the resource URI variables defined in table 5.1.3.3.2-1.

Table 5.1.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.1.1 |
| apiVersion | string | See clause 5.1.1 |
| transRefId | string | Unique identifier of the individual MFAF Configurations resource. |

5.1.3.3.3	Resource Standard Methods

5.1.3.3.3.1	PUT

This method shall support the URI query parameters specified in table 5.1.3.3.3.1-1.

Table 5.1.3.3.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.1.3.3.3.1-2 and the response data structures and response codes specified in table 5.1.3.3.3.1-3.

Table 5.1.3.3.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MfafConfiguration | M | 1 | The updated MFAF Configuration. |

Table 5.1.3.3.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MfafConfiguration | M | 1 | 200 OK | The update of an Individual MFAF Configuration resource is confirmed and a representation of that resource is returned. |
| n/a |  |  | 204 No Content | Successful case: The Individual MFAF Configuration resource was modified. |
| NOTE: The manadatory HTTP error status code for the PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

5.1.3.3.3.2	DELETE

This method shall support the URI query parameters specified in table 5.1.3.3.3.2-1.

Table 5.1.3.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.1.3.3.3.2-2 and the response data structures and response codes specified in table 5.1.3.3.3.2-3.

Table 5.1.3.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.1.3.3.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case: The Individual MFAF Configuration resource matching the transRefId was deleted. |
| NOTE: The manadatory HTTP error status code for the DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

5.1.4	Custom Operations without associated resources

None in this release of the specification.

5.1.5	Notifications

None in this release of the specification.

5.1.6	Data Model

5.1.6.1	General

This clause specifies the application data model supported by the API.

Table 5.1.6.1-1 specifies the data types defined for the NMFAF service based interface protocol.

Table 5.1.6.1-1: NMFAF specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| MfafConfiguration | 5.1.6.2.1 | The description of MFAF configuration |  |
| MessageConfiguration | 5.1.6.2.2 | The description of the configurations. |  |
| MfafNotiInfos | 5.1.6.2.3 | The MFAF notification information. |  |

Table 5.1.6.1-2 specifies data types re-used by the NMFAF service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the N<NF> service based interface.

Table 5.1.6.1-2: NMFAF re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| FormattingInstruction | 3GPP TS 29.574 [15] | Contains data or analytics formatting Instructions. |  |
| ProcessingInstruction | 3GPP TS 29.574 [15] | Contains instructions related to the processing |  |
| Uri | 3GPP TS 29.571 [17] | URI. |  |

5.1.6.2	Structured data types

5.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

5.1.6.2.2	Type: MfafConfiguration

Table 5.1.6.2.2-1: Definition of type MfafConfiguration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| messageConfigurations | array(MessageConfiguration) | M | 1..n | The configuration of the MFAF for mapping data or analytics. |  |

5.1.6.2.3	Type: MessageConfiguration

Table 5.1.6.2.Y-1: Definition of type MessageConfiguration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| correId | string | C | 0..1 | If the configuration is used for mapping analytics or data collection, representing the Analytics Consumer Notification Correlation ID or Data Consumer Notification Correlation ID. |  |
| formatInstruct | FormattingInstruction | O | 0..1 | Formatting instructions to be used for sending event notifications. |  |
| mfafNotiInfo | MfafNotiInfo | C | 0..1 | The MFAF notification information. It shall be provided in a response message if it had not been provided in the respective request message. |  |
| notificationURI | Uri | M | 1 | The notification URI of Data Consumer or Analytics Consumer or other endpoint where to receive the requested mapping data or analytics |  |
| procInstruct | ProcessingInstruction | O | 0..1 | Processing instructions to be used for sending event notifications. |  |

5.1.6.2.3	Type: MfafNotiInfo

Table 5.1.6.2.3-1: Definition of type MfafNotiInfos

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mfafNotifUri | Uri | M | 1 | The notification URI of MFAF Notification Target Address. |  |
| mfafCorrreId | String | M | 1 | The MFAF Notification Correlation ID |  |

5.1.6.3	Simple data types and enumerations

This clause will define simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

5.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

5.1.6.3.2	Simple data types

The simple data types defined in table 5.1.6.3.2-1 shall be supported.

Table 5.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
|  | <one simple data type, i.e. boolean, integer, number, or string> |  |  |

5.1.6.3.3	Enumeration: <EnumType1>

The enumeration <EnumType1> represents <something>. It shall comply with the provisions defined in table 5.1.6.3.3-1.

Table 5.1.6.3.3-1: Enumeration < EnumType1>

| Enumeration value | Description | Applicability |
| --- | --- | --- |

5.1.6.3.4	Enumeration: <EnumType2>

And so on if there are more enumerations to define.

5.1.6.4	Data types describing alternative data types or combinations of data types

5.1.6.4.1	Type: <TypeName 1>

The data types describing alternative data types or combinations of data types shall represent an OpenAPI schema object using the "oneOf", "anyOf" or "allOf"  keyword to list alternative or to be combined data types (see the OpenAPI specification [4] and https://swagger.io/docs/specification/data-models/oneof-anyof-allof-not/).

An instance (i.e. a corresponding part of a JSON file to be evaluated against the schema) matches , i.e. a list of mutually exclusive alternative data types., as described using the OpenAPI "oneOf" keyword, if the instance matches against one and only one of the alternative data types.

An instance (i.e. a corresponding part of a JSON file to be evaluated against the schema) matches a list of non-exclusive alternative data types, as described using the OpenAPI "anyOf" keyword, if the instance matches against one or more of the alternative data types.

An instance (i.e. a corresponding part of a JSON file to be evaluated against the schema) matches a list of to be combined data types, as described using the OpenAPI "allOf" keyword, if the instance matches against all of the to be combined data types."Attribute name": Name of attributes that belong to the specified data type. The attribute names within a structured data type shall be unique, and their relative order inside the structured data type shall not imply any specific ordering of the corresponding JSON elements in a JSON object.

"Data type": Data type of the alternative or to be combined data type. If the data type is indicated as "<type>", the alternative or to be combined data type shall be of data type <type>. If the data type is indicated as "array(<type>)", the alternative or to be combined data type shall be an array (see IETF RFC 8259 [3]) that contains elements of data type <type>. If the data type is indicated as "map(<type>)", the alternative or to be combined data type shall be an object (see IETF RFC 8259 [3]) encoded in the corresponding OpenAPI specification as a map which values are of data type <type>. <type> can either be "integer", "number", "string" or "boolean" (as defined in the OpenAPI specification [4]), or a data type defined in a 3GPP specification.

"Cardinality": Defines the allowed number of occurrence of data type <type>. A cardinality of "M..N", is only allowed for data types "array(<type>)" and "map(<type>)" and indicates the number of elements within the array or map; the values M and N can either be the characters "M" and "N", respectively, or integer numbers; with M being greater than or equal 0, and N being greater than 0. For data type "<type>", the cardinality shall be set to "1".

"Description": Describes the meaning and use of the attribute and may contain normative statements.Applicability: If the attribute is only applicable for optional feature(s) negotiated using the mechanism defined in clause 6.6 of 3GPP TS 29.500 [4], the name of the corresponding feature(s) shall be indicated in this column. If no feature is indicated. the attribute can be used with any feature.

Applicability: If the type is only applicable for optional feature(s) negotiated using the mechanism defined in clause 6.6 of 3GPP TS 29.500 [2], the name of the corresponding feature(s) shall be indicated in this column. If no feature is indicated. the type can be used with any feature. If no optional features are defined for an API, the applicability column can be omitted for that API.

Table 5.1.6.4.1-1: Definition of type <Type name 1> as a list of <"mutually exclusive alternatives" / "non-exclusive alternatives" / "to be combined data types">

| Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "1" or "M..N" | <only if applicable> |  |

5.1.6.4.2	Type: <TypeName 2>

And so on if there are more types to specify.

5.1.6.5	Binary data

This clause will specify what is encoded in binary part, if multipart media type is agreed to be supported by CT4 and is supported by the API. It shall be omitted if not applicable.

5.1.6.5.1	Binary Data Types

Table 5.1.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |
| < Binary Data 1 > e.g. N1 SM Message | < clause > e.g. 5.1.6.5.1 | <content type> e.g. vnd.3gpp.5gnas |

5.1.6.5.1	< Binary Data 1 >

And so on if there are more binary data to specify

5.1.7	Error Handling

This clause will include a reference to the general error handling principles specified in TS 29.501, and further specify any general error handling aspect specific to the API, if any Error handling specific to each method (and resource) is specified in clauses 5.1.3. and 6.1.4.

5.1.7.1	General

For the Nmfaf_3daDataManagement API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.1.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.1.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Nmfaf_3daDataManagement API.

5.1.7.2	Protocol Errors

No specific procedures for the Nmfaf_3daDataManagement service are specified.

Or add specific information for the API if applicable.

5.1.7.3	Application Errors

The application errors defined for the Nmfaf_3daDataManagement service are listed in Table 5.1.7.3-1.

Table 5.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

5.1.8	Feature negotiation

The optional features in table 5.1.8-1 are defined for the Nmfaf_3daDataManagement API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 5.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

5.1.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Nmfaf_3daDataManagement API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nmfaf_3daDataManagement API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nmfaf_3daDataManagement service.

The Nmfaf_3daDataManagement API defines a single scope "nmfaf_3dadatamanagement" for the entire service, and it does not define any additional scopes at resource or operation level.

5.2	Nmfaf_3caDataManagement Service API

One clause per service, where <service 1> is to be replaced by the service name (e.g. Nsmf_PDUSession).

5.2.1	Introduction

This clause specifies the API Name and Version.

The Nmfaf_3caDataManagement shall use the Nmfaf_3caDataManagement API.

The API URI of the Nmfaf_3caDataManagement API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nmfaf-3cadatamanagement".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 5.2.3.

5.2.2	Usage of HTTP

5.2.2.1	General

This clause will include a reference to TS 29.500 for the description of the Transport and HTTP/2.0 protocol requirements and for the security requirements.

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Nmfaf_3caDataManagement API is contained in Annex A.

5.2.2.2	HTTP standard headers

5.2.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

Add specific information for the API if applicable.

5.2.2.2.2	Content type

This clause will indicate the encoding of HTTP requests/responses and the applicable MIME media type for the related Content-Type header. Adjust the text below if additional payload types are used e.g. for HATEOS.

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

5.2.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be applicable.

Add specific information for the API if applicable.

5.2.3	Resources

5.2.3.1	Overview

This clause will describe the structure for the Resource URIs and the resources and methods used for the service.

Example:

Figure 5.2.3.1-1: Resource URI structure of the Nmfaf_3caDataManagement API

Table 5.2.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.2.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| <Resource name> | <relative URI below root> | GET | <Operation executed by GET> |
|  |  | PUT | <Operation executed by PUT> |
|  |  | PATCH | <Operation executed by PATCH> |
|  |  | POST | <Operation executed by POST> |
|  |  | DELETE | <Operation executed by DELETE> |
|  |  | Custom operation | <Operation executed by custom operation> |

5.2.3.2	Resource: <resource 1>

Where <resource 1> is to be replaced by the resource name, e.g. PduSession.

5.2.3.2.1	Description

This clause will specify what the resource represents or what it is used for.

5.2.3.2.2	Resource Definition

This clause will describe the Resource URI and the supported resource variables.

Resource URI: {apiRoot}/<apiName>/<apiVersion>/xxx

This resource shall support the resource URI variables defined in table 5.2.3.2.2-1.

Table 5.2.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.2.1 |
| apiVersion | string | See clause 5.2.1 |
| <name> | <type> | <definition> |

5.2.3.2.3	Resource Standard Methods

The following clauses will specify the standard methods supported by the resource.

It will describe, for each method, the use of the method, the URI query parameters supported by the method, request and response data structures and response codes, and if applicable, HTTP headers specific to the operation.

5.2.3.2.3.1	< method 1 >

This clause will specify the meaning of the method applied on the resource.

This method shall support the URI query parameters specified in table 5.2.3.2.3.1-1.

Table 5.2.3.2.3.1-1: URI query parameters supported by the <method 1> method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| <name> or n/a | <type> or <leave empty> | <M, C or O> | 0..1 or 1 or 0..N or 1..N or <leave empty> | <only if applicable> |  |

This method shall support the request data structures specified in table 5.2.3.2.3.1-2 and the response data structures and response codes specified in table 5.2.3.2.3.1-3.

Table 5.2.3.2.3.1-2: Data structures supported by the <method 1> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" or n/a | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 5.2.3.2.3.1-3: Data structures supported by the <method 1> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" or n/a | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |
| NOTE: The manadatory HTTP error status code for the <method 1> method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 5.2.3.2.3.1-4: Headers supported by the <e.g. GET> method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| <header name> | <data type> e.g. string | "M", "C" or "O" | "0..1", "1", "1..N", "1..N", or <leave empty> | <description> |

Table 5.2.3.2.3.1-5: Headers supported by the <e.g. 200> response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| <header name> | <data type> e.g. string | "M", "C" or "O" | "0..1", "1", "1..N", "1..N", or <leave empty> | <description> |

Table 5.2.3.2.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| <link name> e.g. search | <resource 1> e.g. Stored Search (Document) | <method 1> e.g. GET | <parameter> e.g. searchId | <description of the link> |

5.2.3.2.3.2	< method 2 >

And so on if there are more than two methods supported by the resource. Same structure as in clause 5.2.3.2.3.1.

5.2.3.2.4	Resource Custom Operations

The following clauses will specify the custom operations supported by the resource.

It will describe, for each custom operation, the use and the URI of the operation, the HTTP method on which it is mapped, request and response data structures and response codes, and if applicable, HTTP headers specific to the operation.

5.2.3.2.4.1	Overview

Table 5.2.3.2.4.1-1: Custom operations

| Operation name | Custom operaration URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| <custom operation name> | <custom operation URI> | e.g.POST | <Operation executed by Custom operation> |

5.2.3.2.4.2	Operation: < operation 1 >

This clause will specify the meaning of the operation applied on the resource.

5.2.3.2.4.2.1	Description

This sublause will describe the custom operation and what it is used for, and the custom operation's URI.

5.2.3.2.4.2.2	Operation Definition

This clause will specify the custom operation and the HTTP method on which it is mapped.

This operation shall support the request data structures specified in table 5.2.3.2.4.2.2-1 and the response data structure and response codes specified in table 5.2.3.2.4.2.2-2.

Table 5.2.3.2.4.2.2-1: Data structures supported by the <e.g. POST> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 5.2.3.2.4.2.2-2: Data structures supported by the <e.g. POST> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |
| NOTE: The manadatory HTTP error status code for the <e.g. POST> method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

5.2.3.2.4.3	Operation: < operation 2 >

And so on if there are more than two operations supported by the resource. Same structure as in clause 5.2.3.2.4.1.

5.2.3.3	Resource: <resource 2>

And so on if there are more than two resources supported by the service. Same structure as in clause 5.2.3.2.

5.2.4	Custom Operations without associated resources

5.2.4.1	Overview

This clause will specify custom operations without any associated resource (i.e. RPC) supported by this API.

Table 5.2.4.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| <custom operation URI> | e.g.POST | <Operation executed by Custom operation> |

5.2.4.2	Operation: <operation 1>

Where <operation 1> is to be replaced by the name of the custom operation, e.g. Authentication_Information_Request.

It will describe, for each custom operation, the use and the URI of the operation, the HTTP method on which it is mapped, request and response data structures and response codes, and if applicable, HTTP headers specific to the operation.

5.2.4.2.1	Description

This sublause will describe the custom operation and what it is used for, and the custom operation's URI.

5.2.4.2.2	Operation Definition

This clause will specify the custom operation and the HTTP method on which it is mapped.

This operation shall support the response data structures and response codes specified in tables 5.2.4.2.2-1 and 5.2.4.2.2-2.

Table 5.2.4.2.2-1: Data structures supported by the <e.g. POST> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 5.2.4.2.2-2: Data structures supported by the <e.g. POST> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |
| NOTE: The manadatory HTTP error status code for the <e.g. POST> method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

5.2.4.3	Operation: < operation 2>

And so on if there are more than one custom operations supported by the service. Same structure as in clause 5.2.4.2.

5.2.5	Notifications

5.2.5.1	General

This clause will specify the use of notifications and corresponding protocol details if required for the specific service. When notifications are supported by the API, it will include a reference to the general description of notifications support over the 5G SBIs specified in TS 29.500 / TS 29.501.

Notifications shall comply to clause 6.2 of 3GPP TS 29.500 [4] and clause 4.6.2.3 of 3GPP TS 29.501 [5].

Table 5.2.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| MFAF Notification | {notificationUri} | POST | MFAF Notification |

5.2.5.2	MFAF Notification

5.2.5.2.1	Description

The MFAF Notification is used by the MFAF to provide data or analytics or notification of availability of data or analytics to notification endpoints.

5.2.5.2.2	Target URI

The Callback URI "{notificationUri}" shall be used with the callback URI variables defined in table 5.2.5.2.2-1.

Table 5.2.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notificationUri | The notification URI of Data Consumer or Analytics Consumer or other endpoint where to receive the requested data or analytics |

5.2.5.2.3	Standard Methods

5.2.5.2.3.1	POST

This method shall support the request data structures specified in table 5.2.5.2.3.1-1 and the response data structures and response codes specified in table 5.2.5.2.3.1-1.

Table 5.2.5.2.3.1-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MfafNotification | M | 1 | The data or analytics or notification of availability of data or analytics to notification endpoints. |

Editor's Note:	The definition of MfafNotification is FFS.

Table 5.2.5.2.3.1-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The MFAF notification is treated successfully. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

5.2.5.3	<notification 2>

And so on if there are more than one notifications supported by the service. Same structure as in clause 5.2.5.2.

5.2.6	Data Model

5.2.6.1	General

This clause specifies the application data model supported by the API.

Data types that may be common to multiple APIs (offered by the same or different NFs) should be specified in a new separate TS (similar approach as for TS 29.230 for Diameter AVPs).

Table 5.2.6.1-1 specifies the data types defined for the N<NF> service based interface protocol.

Table 5.2.6.1-1: N<NF> specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |

Table 5.2.6.1-2 specifies data types re-used by the N<NF> service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the N<NF> service based interface.

Table 5.2.6.1-2: N<NF> re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |

5.2.6.2	Structured data types

This clause will specify the structured data types.

5.2.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

5.2.6.2.2	Type: <TypeName 1>

"Attribute name": Name of attributes that belong to the specified data type. The attribute names within a structured data type shall be unique, and their relative order inside the structured data type shall not imply any specific ordering of the corresponding JSON member names in a JSON object.

"Data type": Data type of the attribute values. If the data type is indicated as "<type>", the attribute value shall be of data type <type>. If the data type is indicated as "array(<type>)", the attribute value shall be an array (see IETF RFC 825]) that contains elements of data type <type>. If the data type is indicated as "map(<type>)", the attribute value shall be an object (see IETF RFC 8259) encoded in the corresponding OpenAPI specification as a map which values are data type <type>. <type> can either be "integer", "number", "string" or "boolean" (as defined in the OpenAPI specification [4]), or a data type defined in a 3GPP specification.

"P": Presence condition of a data structure in request body. It shall be one of "M" (for Mandatory), "C" (for Conditional) and "O" (for Optional).

"Cardinality": Defines the allowed number of occurrence of data type <type>. A cardinality of "M..N", is only allowed for data types "array(<type>)" and "map(<type>)" and indicates the number of elements within the array or map; the values M and N can either be the characters "M" and "N", respectively, or integer numbers; with M being greater than or equal 0, and N being greater than 0 and M. For data type "<type>", the cardinality shall be set to "0..1" if the Presence condition is "C" or "O", and to "1" if the Presence condition is "M".

"Description": Describes the meaning and use of the attribute and may contain normative statements..

Applicability: If the attribute is only applicable for optional feature(s) negotiated using the mechanism defined in clause 6.6 of 3GPP TS 29.500 [4], the name of the corresponding feature(s) shall be indicated in this column. If no feature is indicated. the attribute can be used with any feature.If no optional features are defined for an API, the applicability column can be omitted for that API

Table 5.2.6.2.2-1: Definition of type <TypeName 1>

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| <attribute name> | "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N" | <only if applicable> |  |

5.2.6.2.3	Type: <TypeName 2>

And so on if there are more types to specify.

5.2.6.3	Simple data types and enumerations

This clause will define simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

5.2.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

5.2.6.3.2	Simple data types

The simple data types defined in table 5.2.6.3.2-1 shall be supported.

Table 5.2.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
|  | <one simple data type, i.e. boolean, integer, number, or string> |  |  |

5.2.6.3.3	Enumeration: <EnumType1>

The enumeration <EnumType1> represents <something>. It shall comply with the provisions defined in table 5.2.6.3.3-1.

Table 5.2.6.3.3-1: Enumeration < EnumType1>

| Enumeration value | Description | Applicability |
| --- | --- | --- |

5.2.6.3.4	Enumeration: <EnumType2>

And so on if there are more enumerations to define.

5.2.6.4	Data types describing alternative data types or combinations of data types

5.2.6.4.1	Type: <TypeName 1>

The data types describing alternative data types or combinations of data types shall represent an OpenAPI schema object using the "oneOf", "anyOf" or "allOf"  keyword to list alternative or to be combined data types (see the OpenAPI specification [4] and https://swagger.io/docs/specification/data-models/oneof-anyof-allof-not/).

An instance (i.e. a corresponding part of a JSON file to be evaluated against the schema) matches , i.e. a list of mutually exclusive alternative data types., as described using the OpenAPI "oneOf" keyword, if the instance matches against one and only one of the alternative data types.

An instance (i.e. a corresponding part of a JSON file to be evaluated against the schema) matches a list of non-exclusive alternative data types, as described using the OpenAPI "anyOf" keyword, if the instance matches against one or more of the alternative data types.

An instance (i.e. a corresponding part of a JSON file to be evaluated against the schema) matches a list of to be combined data types, as described using the OpenAPI "allOf" keyword, if the instance matches against all of the to be combined data types."Attribute name": Name of attributes that belong to the specified data type. The attribute names within a structured data type shall be unique, and their relative order inside the structured data type shall not imply any specific ordering of the corresponding JSON elements in a JSON object.

"Data type": Data type of the alternative or to be combined data type. If the data type is indicated as "<type>", the alternative or to be combined data type shall be of data type <type>. If the data type is indicated as "array(<type>)", the alternative or to be combined data type shall be an array (see IETF RFC 8259 [3]) that contains elements of data type <type>. If the data type is indicated as "map(<type>)", the alternative or to be combined data type shall be an object (see IETF RFC 8259 [3]) encoded in the corresponding OpenAPI specification as a map which values are of data type <type>. <type> can either be "integer", "number", "string" or "boolean" (as defined in the OpenAPI specification [4]), or a data type defined in a 3GPP specification.

"Cardinality": Defines the allowed number of occurrence of data type <type>. A cardinality of "M..N", is only allowed for data types "array(<type>)" and "map(<type>)" and indicates the number of elements within the array or map; the values M and N can either be the characters "M" and "N", respectively, or integer numbers; with M being greater than or equal 0, and N being greater than 0. For data type "<type>", the cardinality shall be set to "1".

"Description": Describes the meaning and use of the attribute and may contain normative statements.Applicability: If the attribute is only applicable for optional feature(s) negotiated using the mechanism defined in clause 6.6 of 3GPP TS 29.500 [4], the name of the corresponding feature(s) shall be indicated in this column. If no feature is indicated. the attribute can be used with any feature.

Applicability: If the type is only applicable for optional feature(s) negotiated using the mechanism defined in clause 6.6 of 3GPP TS 29.500 [2], the name of the corresponding feature(s) shall be indicated in this column. If no feature is indicated. the type can be used with any feature. If no optional features are defined for an API, the applicability column can be omitted for that API.

Table 5.2.6.4.1-1: Definition of type <Type name 1> as a list of <"mutually exclusive alternatives" / "non-exclusive alternatives" / "to be combined data types">

| Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "1" or "M..N" | <only if applicable> |  |

5.2.6.4.2	Type: <TypeName 2>

And so on if there are more types to specify.

5.2.6.5	Binary data

This clause will specify what is encoded in binary part, if multipart media type is agreed to be supported by CT4 and is supported by the API. It shall be omitted if not applicable.

5.2.6.5.2	Binary Data Types

Table 5.2.6.5.2-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |
| < Binary Data 1 > e.g. N1 SM Message | < clause > e.g. 5.2.6.5.2 | <content type> e.g. vnd.3gpp.5gnas |

5.2.6.5.2	< Binary Data 1 >

And so on if there are more binary data to specify

5.2.7	Error Handling

This clause will include a reference to the general error handling principles specified in TS 29.501, and further specify any general error handling aspect specific to the API, if any Error handling specific to each method (and resource) is specified in clauses 5.2.3. and 6.1.4.

5.2.7.1	General

For the Nmfaf_3caDataManagement API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Nmfaf_3caDataManagement API.

5.2.7.2	Protocol Errors

No specific procedures for the Nmfaf_3caDataManagement service are specified.

Or add specific information for the API if applicable.

5.2.7.3	Application Errors

The application errors defined for the Nmfaf_3caDataManagement service are listed in Table 5.2.7.3-1.

Table 5.2.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

5.2.8	Feature negotiation

The optional features in table 5.2.8-1 are defined for the Nmfaf_3caDataManagement API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 5.2.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

5.2.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Nmfaf_3caDataManagement API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nmfaf_3caDataManagement API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nmfaf_3caDataManagement service.

The Nmfaf_3caDataManagement API defines a single scope "nmfaf_3cadatamanagement" for the entire service, and it does not define any additional scopes at resource or operation level.

Annex A (normative):
OpenAPI specification

A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI 3.0.0 specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see 3GPP TS 29.501 [5] clause 5.3.1 and 3GPP TR 21.900 [7] clause 5B).

A.2	Nmfaf_3daDataManagement API

A.3	Nmfaf_3caDataManagement API

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-06 | CT3#116e |  |  |  |  | TS skeleton of Messaging Framework Adaptor Services specification | 0.0.0 |
| 2021-06 | CT3#116e | C3-213502 |  |  |  | Inclusion of documents agreed in CT3#116e C3-213377. | 0.1.0 |
| 2021-08 | CT3#117e | C3-214580 |  |  |  | Inclusion of documents agreed in CT3#117e C3-214479, C3-214358, C3-214359, C3-214360, C3-214361, C3-214362, C3-214363 and C3-214480. | 0.2.0 |
| 2021-11 | CT3#119e | C3-216522 |  |  |  | Inclusion of documents agreed in CT3#119e C3-216422, C3-216423, C3-216441, C3-216465, C3-216467. | 0.3.0 |