| 3GPP TS29.538 0.2.0 (2021-11) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Enabling MSGin5G Service; Application Programming Interfaces (API) specification; Stage 3; (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	11

Introduction	12

1	Scope	13

2	References	13

3	Definitions of terms, symbols and abbreviations	13

3.1	Terms	13

3.2	Symbols	14

3.3	Abbreviations	14

4	Overview	15

5	Services offered by the MSGin5G Servers	15

5.1	Introduction	15

5.2	MSGS_ASRegistration Service	15

5.2.1	Service Description	15

5.2.2	Service Operations	15

5.2.2.1	Introduction	15

5.2.2.2	MSGS_ASRegistration_Request	16

5.2.2.2.1	General	16

5.3.2.2.2	Application Server registering to MSGin5G Server using MSGS_ASRegistration_Request operation	16

5.2.2.3	MSGS_ASRegistration_Deregister	16

5.2.2.3.1	General	16

5.3.2.3.2	Application Server deregistering from MSGin5G Server using MSGS_ASRegistration_Deregister operation	16

5.3	MSGS_MSGDelivery Service	17

5.3.1	Service Description	17

5.3.2	Service Operations	17

5.3.2.1	Introduction	17

5.3.2.2	MSGS_MSGDelivery_Subscribe	18

5.3.2.2.1	General	18

5.3.2.2.2	Creating a subscription for MSGin5G messages delivery	18

5.3.2.3	MSGS_MSGDelivery_Unsubscribe	18

5.3.2.3.1	General	18

5.3.2.3.2	Deleting a subscription for MSGin5G messages delivery	19

5.3.2.4	MSGS_MSGDelivery_ASODelivery	19

5.3.2.4.1	General	19

5.3.2.4.2	AS Originating MSGin5G Message Delivery	19

5.3.2.5	MSGS_MSGDelivery_ASODeliveryReport	20

5.3.2.5.1	General	20

5.3.2.5.2	AS Originating Message Delivery Status Report	21

5.3.2.6	MSGS_MSGDelivery_UEODelivery	21

5.3.2.6.1	General	21

5.3.2.6.2	UE Originating Message Delivery	22

5.3.2.7	MSGS_MSGDelivery_UEODeliveryReport	23

5.3.2.7.1	General	23

5.3.2.7.2	UE Originating Message Delivery Status Report	23

5.x	<MSGS_xxx> Service	24

5.x.1	Service Description	24

5.x.2	Service Operations	25

5.x.2.1	Introduction	25

5.x.2.2	<Service operation 1>	25

5.x.2.2.1	General	25

5.x.2.2.2	<Description> <Service Operation Name> operation	25

5.x.2.3	<Service operation 2>	25

6	Services offered by the Message Gateway	25

6.1	Introduction	25

6.2	MSGG_L3GDelivery Service	25

6.2.1	Service Description	25

6.2.2	Service Operations	26

6.2.2.1	Introduction	26

6.2.2.2	MSGG_L3GDelivery_Subscribe	26

6.2.2.2.1	General	26

6.2.2.2.2	Legacy 3GPP Message Gateway creating a subscription for MSGin5G messages delivery	26

6.2.2.3	MSGG_L3GDelivery_Unsubscribe	26

6.2.2.3.1	General	26

6.2.2.3.2	Legacy 3GPP Message Gateway deleting a subscription for MSGin5G messages delivery	26

6.2.2.4	MSGG_L3GDelivery_GTDelivery	26

6.2.2.4.1	General	26

6.2.2.4.2	Legacy 3GPP Message Gateway Terminating Message Delivery	26

6.2.2.5	MSGG_L3GDelivery_GTDeliveryReport	26

6.2.2.5.1	General	26

6.2.2.5.2	Legacy 3GPP Message Gateway Terminating Message Delivery Status Report	26

6.3	MSGG_N3GDelivery Service	27

6.3.1	Service Description	27

6.3.2	Service Operations	27

6.3.2.1	Introduction	27

6.3.2.2	MSGG_N3GDelivery_Subscribe	27

6.3.2.2.1	General	27

6.3.2.2.2	Non-3GPP Message Gateway creating a subscription for MSGin5G messages delivery	27

6.3.2.3	MSGG_N3GDelivery_Unsubscribe	27

6.3.2.3.1	General	27

6.3.2.3.2	Non-3GPP Message Gateway deleting a subscription for MSGin5G messages delivery	27

6.3.2.4	MSGG_N3GDelivery_GTDelivery	27

6.3.2.4.1	General	27

6.3.2.4.2	Non-3GPP Message Gateway Terminating Message Delivery	27

6.3.2.5	MSGG_N3GDelivery_GTDeliveryReport	27

6.3.2.5.1	General	27

6.3.2.5.2	Non-3GPP Message Gateway Terminating Message Delivery Status Report	28

6.x	<MSGG_xxx> Service	28

6.x.1	Service Description	28

6.x.2	Service Operations	28

6.x.2.1	Introduction	28

6.x.2.2	<Service operation 1>	28

6.x.2.2.1	General	28

6.x.2.2.2	<Description> <Service Operation Name> operation	28

6.x.2.3	<Service operation 2>	28

7.	Common information applicable to several APIs	28

7.1	General	28

7.2	Data Types	29

7.2.1	General	29

7.2.2	Referenced structured data types	29

7.2.3	Referenced Simple data types and enumerations	29

7.3	Usage of HTTP	29

7.4	Content type	29

7.5	URI structure	29

7.5.1	Resource URI structure	29

7.5.2	Custom operations URI structure	29

7.6	Notifications	29

7.7	Error Handling	29

7.8	Feature negotiation	30

7.9	HTTP headers	30

7.10	Conventions for Open API specification files	30

8	Message Server API definition	30

8.1	MSGS_ASRegistration API	30

8.1.1	API URI	30

8.1.2	Resources	31

8.1.2.1	Overview	31

8.1.2.2	Resource: AS Registrations	31

8.1.2.2.1	Description	31

8.1.2.2.2	Resource Definition	31

8.1.2.2.3	Resource Standard Methods	32

8.1.2.2.3.1	POST	32

8.1.2.3	Resource: AS DeRegistration	33

8.1.2.3.1	Description	33

8.1.2.3.2	Resource Definition	33

8.1.2.3.3	Resource Standard Methods	34

8.1.2.3.3.1	DELETE	34

8.1.3	Custom Operations without associated resources	35

8.1.4	Notifications	35

8.1.5	Data Model	35

8.1.5.1	General	35

8.1.5.2	Structured data types	36

8.1.5.2.1	Introduction	36

8.1.5.2.2	Type: ASRegistration	36

8.1.5.2.3	Type: ASRegistrationAck	36

8.1.5.2.4	Type: ASProfile	37

8.1.5.3	Simple data types and enumerations	37

8.1.6	Error Handling	37

8.1.7	Feature negotiation	37

8.2	MSGS_MSGDelivery API	37

8.2.1	API URI	37

8.2.2	Resources	38

8.2.2.1	Overview	38

8.2.2.2	Resource: AS Message Deliveries	38

8.2.2.2.1	Description	38

8.2.2.2.2	Resource Definition	38

8.2.2.2.3	Resource Standard Methods	39

8.2.2.2.3.1	POST	39

8.2.2.3	Resource: UE Message Deliveries	40

8.2.2.3.1	Description	40

8.2.2.3.2	Resource Definition	40

8.2.2.3.3	Resource Standard Methods	40

8.2.2.3.3.1	POST	40

8.2.2.4	Resource: Report Deliveries	41

8.2.2.4.1	Description	41

8.2.2.4.2	Resource Definition	41

8.2.2.4.3	Resource Standard Methods	42

8.2.2.4.3.1	POST	42

8.2.3	Custom Operations without associated resources	43

8.2.4	Notifications	43

8.2.5	Data Model	43

8.2.5.1	General	43

8.2.5.2	Structured data types	45

8.2.5.2.1	Introduction	45

8.2.5.2.2	Type: ASMessageDelivery	45

8.2.5.2.3	Type:UEMessageDelivery	47

8.2.5.2.4	Type: MessageDeliveryAck	48

8.2.5.2.5	Type:MessageSegmentParameters	48

8.2.5.2.6	Type:StoreAndForwardParameters	48

8.2.5.2.7	Type:DeliveryStatusReport	49

8.2.5.3	Simple data types and enumerations	49

8.2.5.3.1	Introduction	49

8.2.5.3.2	Simple data types	49

8.2.5.3.3	Enumeration:Status	49

8.2.5.3.4	Enumeration:DeliveryStatus	50

8.2.5.3.5	Enumeration:Priority	50

8.2.6	Error Handling	50

8.2.7	Feature negotiation	50

8.x	<API Name – MSGS_xxx> API	50

8.x.1	API URI	51

8.x.2	Resources	51

8.x.2.1	Overview	51

8.x.2.2	Resource: <Resource name>	51

8.x.2.2.1	Description	51

8.x.2.2.2	Resource Definition	51

8.x.2.2.3	Resource Standard Methods	51

8.x.2.2.3.1	<Method Name>	51

8.x.2.2.4	 Resource Custom Operations	53

8.x.2.2.4.1	 Overview	53

8.x.2.2.4.2	 Operation: < operation 1 >	53

8.x.2.2.4.2.1	Description	53

8.x.2.2.4.2.2	Operation Definition	53

8.x.3	Custom Operations without associated resources	54

8.x.3.1	Overview	54

8.x.3.2	Operation: <operation 1>	54

8.x.3.2.1	Description	54

8.x.3.2.2	Operation Definition	55

8.x.3.3	Operation: < operation 2>	55

8.x.4	Notifications	56

8.x.4.1	General	56

8.x.4.2	<notification 1>	56

8.x.4.2.1	Description	56

8.x.4.2.2	Notification definition	56

8.x.5	Data Model	57

8.x.5.1	General	57

8.x.5.2	Structured data types	58

8.x.5.2.1	Introduction	58

8.x.5.2.2	Type: <Data type name>	58

8.x.5.3	Simple data types and enumerations	58

8.x.5.3.1	Introduction	58

8.x.5.3.2	Simple data types	58

8.x.5.3.3	Enumeration: <EnumType1>	58

8.x.6	Error Handling	59

8.x.7	Feature negotiation	59

9	Message Gateway API definition	59

9.1	MSGG_L3GDelivery API	59

9.1.1	API URI	59

9.1.2	Resources	59

9.1.3	Custom Operations without associated resources	59

9.1.4	Notifications	59

9.1.5	Data Model	59

9.1.6	Error Handling	60

9.1.7	Feature negotiation	60

9.2	MSGG_N3GDelivery API	60

9.2.1	API URI	60

9.2.2	Resources	60

9.2.3	Custom Operations without associated resources	60

9.2.4	Notifications	60

9.2.5	Data Model	60

9.2.6	Error Handling	60

9.2.7	Feature negotiation	60

9.x	<API Name – MSGG_xxx> API	60

9.x.1	API URI	61

9.x.2	Resources	61

9.x.2.1	Overview	61

9.x.2.2	Resource: <Resource name>	61

9.x.2.2.1	Description	61

9.x.2.2.2	Resource Definition	61

9.x.2.2.3	Resource Standard Methods	61

9.x.2.2.3.1	<Method Name>	61

9.x.2.2.4	 Resource Custom Operations	63

9.x.2.2.4.1	 Overview	63

9.x.2.2.4.2	 Operation: < operation 1 >	63

9.x.2.2.4.2.1	Description	63

9.x.2.2.4.2.2	Operation Definition	63

9.x.3	Custom Operations without associated resources	64

9.x.3.1	Overview	64

9.x.3.2	Operation: <operation 1>	64

9.x.3.2.1	Description	64

9.x.3.2.2	Operation Definition	65

9.x.3.3	Operation: < operation 2>	65

9.x.4	Notifications	66

9.x.4.1	General	66

9.x.4.2	<notification 1>	66

9.x.4.2.1	Description	66

9.x.4.2.2	Notification definition	66

9.x.5	Data Model	67

9.x.5.1	General	67

9.x.5.2	Structured data types	68

9.x.5.2.1	Introduction	68

9.x.5.2.2	Type: <Data type name>	68

9.x.5.3	Simple data types and enumerations	68

9.x.5.3.1	Introduction	68

9.x.5.3.2	Simple data types	68

9.x.5.3.3	Enumeration: <EnumType1>	68

9.x.6	Error Handling	69

9.x.7	Feature negotiation	69

10	Security	69

11	Using Common API Framework	69

11.1	General	69

11.2	Security	69

Annex A (informative): Change history	70

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

# x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

1	Scope

The present document specified the Application Programming Interface (API) for enabling the MSGin5G Service over MSGin5G-2/3/4 interfaces.The application layer architecture, functional requirements, procedures and information flows necessary for MSGin5G Service are contained in 3GPP TS 23.554 [2]. The requirements for MSGin5G are specified in 3GPP TS 22.262 [3].

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.554: "Application architecture for MSGin5G Service".

[3]	3GPP TS 22.262: "Message Service within the 5G System".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[6]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

It is preferred that the reference to 21.905 be the first in the list.

3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

For the purposes of the present document, the following terms and its definitions given in 3GPP TS 23.554 [2] shall apply:

MSGin5G Service

MSGin5G message

MSGin5G UE

MSGin5G Group

MSGin5G Client

MSGin5G Server

Legacy 3GPP Message Gateway

Non-3GPP Message Gateway

Legacy 3GPP UE

Non-3GPP UE

Non-MSGin5G UE

Point-to-Point messaging

Point-to-Application messaging

Application-to-Point messaging

Group messaging

Broadcast messaging

Messaging Topic

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

AS	Application Server

CAPIF	Common API Framework

MSG-C	MSGin5G Client

MSG-S	MSGin5G Server

NIDD	Non IP Data Delivery

SEAL	Service Enabler Architecture Layer for Verticals

4	Overview

The MSGin5G Service is designed and optimized for massive IoT device communication including thing-to-thing communication and person-to-thing communication and provides messaging capability in 5GS with messaging communication models including point-to-point, application-to-point, group and broadcast messaging.

3GPP TS 23.554 [2] has specified the application layer architecture, architectural requirements, procedures, information flows and some APIs, in order to support the MSGin5G Service. Various features are defined to ensure the efficient use and deployment of MSGin5G Service, including configuration, registration, message delivery, message aggregation, segmentation and reassembly.

The present document specifies MSGin5G services offered by MSGin5G servers and MSGin5G Gateway, and APIs in detail, needed over MSGin5G-2/3/4 interfaces for interworking between MSGin5G Server and Legacy 3GPP UE, Non-3GPP UE or Application Server, with following functionalities need to be supported:

1.	Server-side functionality with the sending and receiving of messages to/from Application Servers and/or other MSGin5G Service endpoints on other UEs, provided by MSGin5G server.

2.	Interconnecting two different messaging delivery mechanisms and assure the message integrity between different message delivery mechanisms, provided by Message Gateway.

And the definition of APIs specified in TS 23.554 [2] clause 9 are introduced in present document.

5	Services offered by the MSGin5G Servers

5.1	Introduction

5.2	MSGS_ASRegistration Service

5.2.1	Service Description

The MSGS_ASRegistration API, as defined in 3GPP TS 23.554 [2], allows an AS via Mm5s interface to register, and deregister at a given MSGin5G Server.

5.2.2	Service Operations

5.2.2.1	Introduction

The service operation defined for MSGS_ASRegistration API is shown in the table 5.2.2.1-1.

Table 5.2.2.1-1: Operations of the MSGS_ASRegistration API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| MSGS_ASRegistration_Request | This service operation is used by the AS to register itself to MSGin5G Server. | AS |
| MSGS_ASRegistration_Deregister | This service operation is used by the AS to deregister itself from a MSGin5G Server. | AS |

5.2.2.2	MSGS_ASRegistration_Request

5.2.2.2.1	General

This service operation is used by the AS to register itself to MSGin5G Server.

5.3.2.2.2	Application Server registering to MSGin5G Server using MSGS_ASRegistration_Request operation

To register itself at the MSGin5G Server, the AS shall send an HTTP POST message to the MSGin5G Server on the "AS Registrations" collection resource. The body of the HTTP POST message shall include the AS profile information, may include notification target URI for the registration, as specified in clause 8.1.2.2.3.1.

Upon receiving the HTTP POST message from the AS, the MSGin5G Server shall:

1.	process the AS registration request information;

2.	verify the identity of the AS and check if the AS is authorized to register itself at MSGin5G Server; and

3.	if the AS is authorized to register to MSGin5G Server, then the MSGin5G Server shall:

a.	store the AS registration information and create a new resource with the AS registration information as specified in clause 8.1.2.1; and

b.	return the AS registration information, the resource URI of the AS registration information, in the response message.

5.2.2.3	MSGS_ASRegistration_Deregister

5.2.2.3.1	General

This service operation is used by the AS to deregister itself from a MSGin5G Server.

5.3.2.3.2	Application Server deregistering from MSGin5G Server using MSGS_ASRegistration_Deregister operation

To deregister itself from the MSGin5G Server, the AS shall send HTTP DELETE message to the MSGin5G Server on the "AS DeRegistration" collection resource, as specified in clause 8.1.2.3.3.2.

Upon receiving the HTTP DELETE request, the MSGin5G Server shall:

- verify the identity of the AS and check if the AS is authorized to deregister the AS registration information;

- if the AS is authorized to deregister the AS registration information, then the MSGin5G Server shall deregister the AS profile from the MSGin5G Server and delete the resource representing AS registration information; and

- return the "200 OK" message to the AS, indicating the successful deregistration of the AS information.

5.3	MSGS_MSGDelivery Service

All of the messaging scenarios, i.e. Point-to-Point message, Application-to-Point message/ Point-to-Application message, Group message and Broadcast message, are included in this clause. Segmentation and Reassembly procedures will be added to needed procedure in this clause.

5.3.1	Service Description

The MSGS_MSGDelivery Service corresponding to Mm5s as defined in 3GPP TS 23.554 [2], is provided by the MSGin5G Server.

This service:

-	allows AS invokes services provided by MSGin5G Server to send MSGin5G Messages to MSGin5G Server; and

-	allows L3G/N3G invokes services provided by MSGin5G Server to send MSGin5G Messages to MSGin5G Server on behalf of Legacy 3GPP UE or Non-3GPP UE.

5.3.2	Service Operations

5.3.2.1	Introduction

The service operation defined for MSGS_MSGDelivery Service is shown in the table 5.3.2.1-1.

Table 5.3.2.1-1: Operations of the MSGS_MSGDelivery Service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| MSGS_MSGDelivery_ASODelivery | This service operation is used by AS to deliver MSGin5G message to the MSGin5G Server. This service operation corresponds to the Send_MSGin5G_Message operation in M5S_AS_Originating_Message_Delivery API defined in 3GPP TS 23.554 [2]. | AS |
| MSGS_MSGDelivery_ASODeliveryReport | This service operation is used by AS to deliver the delivery status report to the MSGin5G Server. This service operation corresponds to the Send_MSGin5G_Message operation in M5S_AS_Originating_Delivery_Status_Report API defined in 3GPP TS 23.554 [2]. | AS |
| MSGS_MSGDelivery_UEODelivery | This service operation is used by Legacy 3GPP Message Gateway or Non-3GPP Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) to deliver MSGin5G message to the MSGin5G Server. This service operation corresponds to the Report_Message_Delivery_Status operation in M5S_UE_Originating_Message_Delivery API defined in 3GPP TS 23.554 [2]. | Legacy 3GPP Message Gateway Non-3GPP Message Gateway |
| MSGS_MSGDelivery_UEODeliveryReport | This service operation is used by Legacy 3GPP Message Gateway or Non-3GPP Message Gateway (on behalf Legacy 3GPP UE or Non-3GPP UE) to deliver the delivery status report to the MSGin5G Server. This service operation corresponds to the Report_Message_Delivery_Status operation in M5S_UE_Originating_Delivery_Status_Report API defined in 3GPP TS 23.554 [2]. | Legacy 3GPP Message Gateway Non-3GPP Message Gateway |

5.3.2.2	MSGS_MSGDelivery_Subscribe

5.3.2.2.1	General

5.3.2.2.2	Creating a subscription for MSGin5G messages delivery

5.3.2.3	MSGS_MSGDelivery_Unsubscribe

5.3.2.3.1	General

.

5.3.2.3.2	Deleting a subscription for MSGin5G messages delivery

5.3.2.4	MSGS_MSGDelivery_ASODelivery

This clause describes the service operation of AS originating MSGin5G message delivery API.

5.3.2.4.1	General

This service operation corresponding to the Send_MSGin5G_Message operation in M5S_AS_Originating_Message_Delivery API defined in 3GPP TS 23.554 [2], is used by AS to deliver MSGin5G message to the MSGin5G Server.

5.3.2.4.2	AS Originating MSGin5G Message Delivery

Figure 5.3.2.4.2-1: AS Originating MSGin5G Message Delivery

When the AS needs to send the message to the MSGin5G Server, the AS shall send the HTTP POST method as step 1 of the figure 5.3.2.4.2-1 to request to create "AS Message Deliveries".

The AS shall include ASMessageDelivery data structure in the payload body of the HTTP POST to request a creation of representation of the "AS Message Deliveries" resource. The "AS Message Deliveries" resource is created as described below.

The ASMessageDelivery data structure shall include:

-	The AS Service ID within the "originServiceId" attribute;

-	The Message ID within the "messageId" attribute; and

-	The store and forward flag within the "storeAndForwardFlag" attribute;

and may include:

-	The Recipient UE Service ID within the "recipientServiceId" attribute;

-	The Group Service ID within the "groupServiceId" attribute;

-	The Broadcast Area ID within the "broadcastAreaId" attribute;

-	The Messaging Topic within the "messageTopic" attribute;

NOTE:	Only one of Recipient UE Service ID, Group Service ID, Broadcast Area ID, Messaging Topic shall be included.

-	The Application ID within the "appId" attribute;

-	The security credentials within the "securityCredentials" attribute;

Editor's Note:	The security credentials is to be updated based on SA3 conclusion.

-	The indication whether the message delivery status report is required within the "deliveryStatusRequired" attribute;

-	The Payload within the "payload" attribute;

-	The priority type within the "priority" attribute;

-	The message segment flag within the "messageSegmentFlag" attribute;

-	The message segment parameters within the "messageSegmentParameters" attribute, this attribute may include:

-	The segmentation set identifier within the "segIdentifier" attribute;

-	The total number of message segments within the "totalNumSeg" attribute;

-	The message segment number within the "segNum" attribute;

-	The last segment flag within the "lastSegFlag" attribute;

-	The store and forward parameters within the "storeAndForwardParameters" attribute, this attribute may include:

-	The message expiration time within the "expirationTime" attribute;

-	The latency within the "latency" attribute.

Editor's Note:	whether the above attributes are all necessary is FFS.

When the MSGin5G Server receives the HTTP POST request from the AS, the MSGin5G server shall make an authorization based on the information received from the AS. If the authorization is successful, the MSGin5G Server shall create a new resource, which represents "AS Message Deliveries", addressed by a URI as defined in clause 8.2.2.2.2. The MSGin5G Server shall respond to the AS with a 201 Created message.

If errors occur when processing the HTTP POST request, the MSGin5G Server shall apply error handling procedures as specified in subclause 8.2.6.

Editor's Note:	error handling will be defined in clause 8.2.6.

5.3.2.5	MSGS_MSGDelivery_ASODeliveryReport

This clause describes the service operation of AS originating MSGin5G message delivery status report API.

5.3.2.5.1	General

This service operation corresponds to the Send_MSGin5G_Message operation in M5S_AS_Originating_Delivery_Status_Report API defined in 3GPP TS 23.554 [2], is used by AS to deliver the delivery status report to the MSGin5G Server.

5.3.2.5.2	AS Originating Message Delivery Status Report

Figure 5.3.2.5.2-1: AS Originating MSGin5G Delivery Report

When the AS needs to send the delivery report to the MSGin5G Server, the AS shall send the HTTP POST method as step 1 of the figure 5.3.2.5.2-1 to request to create "Report Deliveries".

The AS shall include DeliveryStatusReport data structure in the payload body of the HTTP POST to request a creation of representation of the "Report Deliveries" resource. The "Report Deliveries" resource is created as described below.

The DeliveryStatusReport data structure shall include:

-	The AS Service ID within the "originServiceId" attribute;

-	The Recipient UE Service ID within the "recipientServiceId" attribute;

-	The Message ID within the "messageId" attribute; and

-	The delivery status within the "deliveryStatus" attribute;

and may include:

-	The security credentials within the "securityCredentials" attribute; and

Editor's Note:	The security credentials is to be updated based on SA3 conclusion.

-	The failure cause within the "failureCause" attribute;

When the MSGin5G Server receives the HTTP POST request from the AS, the MSGin5G server shall make an authorization based on the information received from the AS. If the authorization is successful, the MSGin5G Server shall create a new resource, which represents "Report Deliveries", addressed by a URI as defined in clause 8.2.2.4.2. The MSGin5G Server shall respond to the AS with a 201 Created message.

If errors occur when processing the HTTP POST request, the MSGin5G Server shall apply error handling procedures as specified in subclause 8.2.6.

Editor's Note:	error handling will be defined in clause 8.2.6.

5.3.2.6	MSGS_MSGDelivery_UEODelivery

5.3.2.6.1	General

This service operation corresponds to the Report_Message_Delivery_Status operation in M5S_UE_Originating_Message_Delivery API defined in 3GPP TS 23.554 [2], is used by Legacy 3GPP Message Gateway or Non-3GPP Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) to deliver MSGin5G message to the MSGin5G Server.

5.3.2.6.2	UE Originating Message Delivery

Figure 5.3.2.6.2-1: Legacy 3GPP UE or Non-3GPP UE Originating MSGin5G Message Delivery

When the Legacy 3GPP Message Gateway or Non-3GPP Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) needs to send the message to the MSGin5G Server, the Legacy 3GPP Message Gateway or Non-3GPP Message Gateway shall send the HTTP POST method as step 1of the figure 5.3.2.6.2-1 to request to create a "UE Message Deliveries".

The Legacy 3GPP Message Gateway or Non-3GPP Message Gateway shall include UEMessageDelivery data structure in the payload body of the HTTP POST to request a creation of representation of the "UE Message Deliveries" resource. The "UE Message Deliveries" resource is created as described below.

The UEMessageDelivery data structure shall include:

-	The Legacy 3GPP UE Service ID or Non-3GPP UE Service ID within the "originServiceId" attribute;

-	The Recipient UE Service ID within the "recipientServiceId" attribute;

-	The Message ID within the "messageId" attribute; and

-	The store and forward flag within the "storeAndForwardFlag" attribute;

and may include:

-	The Application ID within the "appId" attribute;

-	The security credentials within the "securityCredentials" attribute;

Editor's Note:	The security credentials is to be updated based on SA3 conclusion.

-	The Payload within the "payload" attribute;

-	The indication whether the message delivery status report is required within the "deliveryStatusRequired" attribute; and

-	The message segment flag within the "messageSegmentFlag" attribute;

-	The message segment parameters within the "messageSegmentParameters" attribute, this attribute may include:

-	The segmentation set identifier within the "segIdentifier" attribute;

-	The total number of message segments within the "totalNumSeg" attribute;

-	The message segment number within the "segNum" attribute;

-	The last segment flag within the "lastSegFlag" attribute;

-	The store and forward parameters within the "storeAndForwardParameters" attribute, this attribute may include:

-	The message expiration time within the "expirationTime" attribute;

Editor's Note:	whether the above attributes are all necessary is FFS.

When the MSGin5G Server receives the HTTP POST request from the Legacy 3GPP Message Gateway or Non-3GPP Message Gateway, the MSGin5G server shall make an authorization based on the information received from the Legacy 3GPP Message Gateway or Non-3GPP Message Gateway. If the authorization is successful, the MSGin5G Server shall create a new resource, which represents "UE Message Deliveries", addressed by a URI as defined in clause 8.2.2.3.2. The MSGin5G Server shall respond to the Legacy 3GPP Message Gateway or Non-3GPP Message Gateway with a 201 Created message.

If errors occur when processing the HTTP POST request, the MSGin5G Server shall apply error handling procedures as specified in subclause 8.2.6.

Editor's Note:	error handling will be defined in clause 8.2.6.

5.3.2.7	MSGS_MSGDelivery_UEODeliveryReport

5.3.2.7.1	General

This service operation corresponds to the Report_Message_Delivery_Status operation in M5S_UE_Originating_Delivery_Status_Report API defined in 3GPP TS 23.554 [2], is used by Legacy 3GPP Message Gateway or Non-3GPP Message Gateway (on behalf Legacy 3GPP UE or Non-3GPP UE) to deliver the delivery status report to the MSGin5G Server.

5.3.2.7.2	UE Originating Message Delivery Status Report

Figure 5.3.2.7.2-1: Legacy 3GPP UE or Non-3GPP UE Originating MSGin5G Delivery Report

When the Legacy 3GPP Message Gateway or Non-3GPP Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) needs to send the delivery report to the MSGin5G Server, the Legacy 3GPP Message Gateway or Non-3GPP Message Gateway shall send the HTTP POST method as step 1 of the figure 5.3.2.7.2-1 to request to create "Report Deliveries".

The Legacy 3GPP Message Gateway or Non-3GPP Message Gateway shall include DeliveryStatusReport data structure in the payload body of the HTTP POST to request a creation of representation of the "Report Deliveries" resource. The "Report Deliveries" resource is created as described below.

The DeliveryStatusReport data structure shall include:

-	The Legacy 3GPP UE Service ID or Non-3GPP UE Service ID within the "originServiceId" attribute;

-	The Recipient UE Service ID within the "recipientServiceId" attribute;

-	The Message ID within the "messageId" attribute;and

-	The delivery status within the "deliveryStatus" attribute;

and may include:

-	The security credentials within the "securityCredentials" attribute;

Editor's Note:	The security credentials is to be updated based on SA3 conclusion.

-	The failure cause within the "failureCause" attribute;

When the MSGin5G Server receives the HTTP POST request from the Legacy 3GPP Message Gateway or Non-3GPP Message Gateway, the MSGin5G server shall make an authorization based on the information received from the Legacy 3GPP Message Gateway or Non-3GPP Message Gateway. If the authorization is successful, the MSGin5G Server shall create a new resource, which represents "Report Deliveries", addressed by a URI as defined in clause 8.2.2.4.2. The MSGin5G Server shall respond to the Legacy 3GPP Message Gateway or Non-3GPP Message Gateway with a 201 Created message.

If errors occur when processing the HTTP POST request, the MSGin5G Server shall apply error handling procedures as specified in subclause 8.2.6.

Editor's Note:	error handling will be defined in clause 8.2.6.

5.x	<MSGS_xxx> Service

Add a copy of this clause for a new API, adding the overview of the API, Details of service operations, detailed description of each service operation. Yellow highlighted text needs to be replaced with appropriate clause number and the API, Service operation name.

5.x.1	Service Description

This clause will provide a general description of the related service, include a description of the functional elements involved in the invocation of the service.

5.x.2	Service Operations

One clause per service operation. This clause will include a description of the different service operations supported by the service.

5.x.2.1	Introduction

The service operation defined for <API Name – MSGS_xxx> API is shown in the table 5.x.2.1-1.

Table 5.x.2.1-1: Operations of the <API Name> API

| Service operation name | Description | Initiated by |
| --- | --- | --- |

5.x.2.2	<Service operation 1>

5.x.2.2.1	General

Provide the general description of the service operation.

5.x.2.2.2	<Description> <Service Operation Name> operation

5.x.2.3	<Service operation 2>

And so on if there are more than 2 service operations to be described for the service.

6	Services offered by the Message Gateway

6.1	Introduction

6.2	MSGG_L3GDelivery Service

All of the messaging scenarios, i.e. Point-to-Point message, Application-to-Point message/ Point-to-Application message, Group message and Broadcast message, are included in this clause. Segmentation and Reassembly procedures will be added to needed procedure in this clause.

6.2.1	Service Description

6.2.2	Service Operations

6.2.2.1	Introduction

6.2.2.2	MSGG_L3GDelivery_Subscribe

6.2.2.2.1	General

6.2.2.2.2	Legacy 3GPP Message Gateway creating a subscription for MSGin5G messages delivery

6.2.2.3	MSGG_L3GDelivery_Unsubscribe

6.2.2.3.1	General

.

6.2.2.3.2	Legacy 3GPP Message Gateway deleting a subscription for MSGin5G messages delivery

6.2.2.4	MSGG_L3GDelivery_GTDelivery

6.2.2.4.1	General

6.2.2.4.2	Legacy 3GPP Message Gateway Terminating Message Delivery

6.2.2.5	MSGG_L3GDelivery_GTDeliveryReport

6.2.2.5.1	General

6.2.2.5.2	Legacy 3GPP Message Gateway Terminating Message Delivery Status Report

6.3	MSGG_N3GDelivery Service

All of the messaging scenarios, i.e. Point-to-Point message, Application-to-Point message/ Point-to-Application message, Group message and Broadcast message, are included in this clause. Segmentation and Reassembly procedures will be added to needed procedure in this clause.

6.3.1	Service Description

6.3.2	Service Operations

6.3.2.1	Introduction

6.3.2.2	MSGG_N3GDelivery_Subscribe

6.3.2.2.1	General

6.3.2.2.2	Non-3GPP Message Gateway creating a subscription for MSGin5G messages delivery

6.3.2.3	MSGG_N3GDelivery_Unsubscribe

6.3.2.3.1	General

.

6.3.2.3.2	Non-3GPP Message Gateway deleting a subscription for MSGin5G messages delivery

6.3.2.4	MSGG_N3GDelivery_GTDelivery

6.3.2.4.1	General

6.3.2.4.2	Non-3GPP Message Gateway Terminating Message Delivery

6.3.2.5	MSGG_N3GDelivery_GTDeliveryReport

6.3.2.5.1	General

6.3.2.5.2	Non-3GPP Message Gateway Terminating Message Delivery Status Report

6.x	<MSGG_xxx> Service

Add a copy of this clause for a new API, adding the overview of the API, Details of service operations, detailed description of each service operation. Yellow highlighted text needs to be replaced with appropriate clause number and the API, Service operation name.

6.x.1	Service Description

This clause will provide a general description of the related service, include a description of the functional elements involved in the invocation of the service.

6.x.2	Service Operations

One clause per service operation. This clause will include a description of the different service operations supported by the service.

6.x.2.1	Introduction

The service operation defined for <API Name – MSGG_xxx> API is shown in the table 6.x.2.1-1.

Table 6.x.2.1-1: Operations of the <API Name> API

| Service operation name | Description | Initiated by |
| --- | --- | --- |

6.x.2.2	<Service operation 1>

6.x.2.2.1	General

Provide the general description of the service operation.

6.x.2.2.2	<Description> <Service Operation Name> operation

6.x.2.3	<Service operation 2>

And so on if there are more than 2 service operations to be described for the service

7.	Common information applicable to several APIs

7.1	General

7.2	Data Types

7.2.1	General

7.2.2	Referenced structured data types

7.2.3	Referenced Simple data types and enumerations

7.3	Usage of HTTP

7.4	Content type

7.5	URI structure

7.5.1	Resource URI structure

7.5.2	Custom operations URI structure

7.6	Notifications

7.7	Error Handling

7.8	Feature negotiation

7.9	HTTP headers

7.10	Conventions for Open API specification files

8	Message Server API definition

8.1	MSGS_ASRegistration API

8.1.1	API URI

The MSGS_ASRegistration service shall use the MSGS_ASRegistration API.

The request URIs used in HTTP requests from the AS towards the MSGin5G Server shall have the Resource URI structure as defined in clause 7.5 with the following clarifications:

-	The <apiName> shall be "msgs_asregistration".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 8.1.2.

8.1.2	Resources

8.1.2.1	Overview

Figure 8.1.2.1-1: Resource URI structure of the MSGS_ASRegistration API

Table 8.1.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| AS Registrations | /registrations | POST | Registers a new AS at the MSGin5G Server. |
| AS DeRegistration | /registrations/{registrationId} | DELETE | Removes an AS registration resource. |

8.1.2.2	Resource: AS Registrations

8.1.2.2.1	Description

This resource represents all the Application Servers that are registered at a given MSGin5G Server.

8.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/msgs-asregistration/<apiVersion>/registrations

This resource shall support the resource URI variables defined in the table 8.1.2.2.2-1.

Table 8.1.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 8.1.1 |

8.1.2.2.3	Resource Standard Methods

8.1.2.2.3.1	POST

This method shall support the URI query parameters specified in table 8.1.2.2.3.1-1.

Table 8.1.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.1.2.2.3.1-2 and the response data structures and response codes specified in table 8.1.2.2.3.1-3.

Table 8.1.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ASRegistration | M | 1 | AS registration request information. |

Table 8.1.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ASRegistrationAck | M | 1 | 201 Created | AS information is registered successfully at MSGin5G Server. The URI of the created resource shall be returned in the “Location” HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. |  |  |  |  |

Table 8.1.2.2.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.1.2.2.3.1-5: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/msgs-asregistration/<apiVersion>/registrations/{registrationId} |

Table 8.1.2.2.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

8.1.2.3	Resource: AS DeRegistration

8.1.2.3.1	Description

This resource represents all the Application Servers that are deregistered at a given MSGin5G Server.

8.1.2.3.2	Resource Definition

Resource URI: {apiRoot}/msgs_asregistration/<apiVersion>/registrations/{registrationId}

This resource shall support the resource URI variables defined in the table 8.1.2.3.2-1.

Table 8.1.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 8.1.1 |
| registrationId | string | The AS registration resource id |

8.1.2.3.3	Resource Standard Methods

8.1.2.3.3.1	DELETE

This method deregisters an AS registration from the MSGin5G Server. This method shall support the URI query parameters specified in the table 8.1.2.3.3.1-1.

Table 8.1.2.3.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.1.2.3.3.1-2 and the response data structures and response codes specified in table 8.1.2.3.3.1-3.

Table 8.1.2.3.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.1.2.3.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ASRegistrationAck | M | 1 | (NOTE) | The AS deregistration result. |
| NOTE: The manadatory HTTP error status code for the DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 8.1.2.3.3.1-4: Headers supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.1.2.3.3.1-5: Headers supported by the 204 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.1.2.3.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

8.1.3	Custom Operations without associated resources

None.

8.1.4	Notifications

None.

8.1.5	Data Model

8.1.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API Table 8.1.5.1-1 specifies the data types defined specifically for the MSGS_ASRegistration API service.

Table 8.1.5.1-1: MSGS_ASRegistration API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ASRegistration | 8.1.5.2.2 | The AS registration request information. |  |
| ASRegistrationAck | 8.1.5.2.3 | The AS registration response information. |  |
| ASProfile | 8.1.5.2.4 | The profile information related to the AS in the ASRegistration data type. |  |

Table 8.1.5.1-2 specifies data types re-used by the MSGS_ASRegistration API service.

Table 8.1.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ProblemDetails | 3GPP TS 29.571 [5] |  |  |

8.1.5.2	Structured data types

8.1.5.2.1	Introduction

8.1.5.2.2	Type: ASRegistration

Table 8.1.5.2.2-1: Definition of type ASRegistration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| asSeviceId | string | M | 1 | The MSGin5G identifier of the Application Server. |  |
| appId | string | M | 1 | The identifier of the application specified by the application provider |  |
| targetUri | string | O | 0..1 | The URL for receiving message, message delivery status report, etc. The MSGin5G Server uses this URL to interact to AS. |  |
| asProf | ASProfile | O | 0..1 | The profile information of the AS. |  |

Editor's Note:	The data model to be updated with security credentials information, based on security aspects defined by SA3.

8.1.5.2.3	Type: ASRegistrationAck

Table 8.1.5.2.3-1: Definition of type ASRegistration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| asSeviceId | string | M | 1 | The MSGin5G identifier of the Application Server. |  |
| result | ProblemDetails | M | 1 | The result of the registration. |  |

8.1.5.2.4	Type: ASProfile

Table 8.1.5.2.4-1: Definition of type ASProfile

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appName | string | O | 1 | The name of the Application Server |  |
| appProvider | array(string) | O | 1 | The provider of the Application Server |  |
| appSenario | array(string) | O | 1..N | The application scenario description. |  |
| appCategory | string | O | 0..1 | The category or type of Application Server. |  |
| status | string | O | 0..1 | AS status (e.g. Enabled, Disabled etc.) |  |

8.1.5.3	Simple data types and enumerations

None.

8.1.6	Error Handling

General error responses will be defined in clause 7.7.

8.1.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.1.7-1 lists the supported features for AS_Registration API.

Table 8.1.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

8.2	MSGS_MSGDelivery API

8.2.1	API URI

The MSGS_MSGDelivery service shall use the MSGS_MSGDelivery API, The MSGS_MSGDelivery API corresponding to Mm5s APIs as defined in in 3GPP TS 23.554 [2].

The request URIs used in HTTP requests from the Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) or the Application Server towards the MSGin5G Server shall have the Resource URI structure as defined in clause 7.5 with the following clarifications:

-	The <apiName> shall be "msgs_msgdelivery".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 8.2.2.

8.2.2	Resources

8.2.2.1	Overview

Figure 8.2.2.1-1: Resource URI structure of the MSGS_MSGDelivery API

Table 8.2.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.2.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| AS Message Deliveries | /as-message-deliveries | POST |  |
| UE Message Deliveries | /ue-message-deliveries | POST |  |
| Report Deliveries | /report-deliveries | POST |  |

8.2.2.2	Resource: AS Message Deliveries

8.2.2.2.1	Description

This resource represents the Application Server originating message delivery to a given MSGin5G Server.

8.2.2.2.2	Resource Definition

Resource URI: {apiRoot}/msgs-msgdelivery/<apiVersion>/as-message-deliveries

This resource shall support the resource URI variables defined in the table 8.2.2.2.2-1.

Table 8.2.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 8.2.1 |

8.2.2.2.3	Resource Standard Methods

8.2.2.2.3.1	POST

This method shall support the URI query parameters specified in table 8.2.2.2.3.1-1.

Table 8.2.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.2.2.2.3.1-2 and the response data structures and response codes specified in table 8.2.2.2.3.1-3.

Table 8.2.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ASMessageDelivery | M | 1 | Parameters to create AS Message Delivery resource. |

Table 8.2.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MessageDeliveryAck | M | 1 | 201 Created | AS Message Deliveries resource for the AS is created successfully. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. |  |  |  |  |

Table 8.2.2.2.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.2.2.2.3.1-5: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.2.2.2.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

8.2.2.3	Resource: UE Message Deliveries

8.2.2.3.1	Description

This resource represents the Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) originating message delivery to a given MSGin5G Server.

8.2.2.3.2	Resource Definition

Resource URI: {apiRoot}/msgs-msgdelivery/<apiVersion>/ue-message-deliveries

This resource shall support the resource URI variables defined in the table 8.2.2.3.2-1.

Table 8.2.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 8.1.1 |

8.2.2.3.3	Resource Standard Methods

8.2.2.3.3.1	POST

This method shall support the URI query parameters specified in table 8.2.2.3.3.1-1.

Table 8.2.2.3.3-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.2.2.3.3.1-2 and the response data structures and response codes specified in table 8.2.2.3.3.1-3.

Table 8.2.2.3.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UEMessageDelivery | M | 1 | Parameters to create UE Message Delivery resource. |

Table 8.2.2.3.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MessageDeliveryAck | M | 1 | 201 Created | UE Message Deliveries resource for the UE is created successfully. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. |  |  |  |  |

Table 8.2.2.3.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.2.2.3.3.1-5: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.2.2.3.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

8.2.2.4	Resource: Report Deliveries

8.2.2.4.1	Description

This resource represents the Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) or the Application Server send delivery report to a given MSGin5G Server.

8.2.2.4.2	Resource Definition

Resource URI: {apiRoot}/msgs-msgdelivery/<apiVersion>/report-deliveries

This resource shall support the resource URI variables defined in the table 8.2.2.4.2-1.

Table 8.2.2.4.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 8.1.1 |

8.2.2.4.3	Resource Standard Methods

8.2.2.4.3.1	POST

This method shall support the URI query parameters specified in table 8.2.2.4.3.1-1.

Table 8.2.2.4.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.2.2.4.3.1-2 and the response data structures and response codes specified in table 8.2.2.4.3.1-3.

Table 8.2.2.4.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DeliveryStatusReport | M | 1 | Parameters to create Report Delivery resource. |

Table 8.2.2.4.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MessageDeliveryAck | M | 1 | 201 Created | Report Deliveries resource for the UE or AS is created successfully. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. |  |  |  |  |

Table 8.2.2.4.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.2.2.4.3.1-5: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.2.2.4.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

8.2.3	Custom Operations without associated resources

None.

8.2.4	Notifications

None.

8.2.5	Data Model

8.2.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API. Table 8.2.5.1-1 specifies the data types defined specifically for the MSGS_MSGDelivery API service.

Table 8.2.5.1-1:  API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ASMessageDelivery | 8.2.5.2.2 | The AS message delivery request information. |  |
| UEMessageDelivery | 8.2.5.2.3 | The UE message delivery request information. |  |
| MessageDeliveryAck | 8.2.5.2.4 | The message delivery response information. |  |
| MessageSegmentParameters | 8.2.5.2.5 | Contains the message segment information of the message. |  |
| StoreAndForwardParameters | 8.2.5.2.6 | Contains the store forward information of the message. |  |
| DeliveryStatusReport | 8.2.5.2.7 | The message delivery status report request information. |  |
| Status | 8.2.5.3.3 | Indicates if delivery is a failure, or if the message is stored for deferred delivery. |  |
| DeliveryStatus | 8.2.5.3.4 | The delivery status description, including success or failure in delivery. |  |
| Priority | 8.2.5.3.5 | Application priority level requested for this message. |  |

Table 8.2.5.1-2 specifies data types re-used by the MSGS_MSGDelivery API service.

Table 8.2.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| integer | 3GPP TS 29.571 [5] | As defined in OpenAPI Specification [6] |  |
| string | 3GPP TS 29.571 [5] | As defined in OpenAPI Specification [6] |  |
| boolean | 3GPP TS 29.571 [5] | As defined in OpenAPI Specification [6] |  |
| DateTime | 3GPP TS 29.571 [5] | String with format "date-time" as defined in OpenAPI Specification [6]. |  |

8.2.5.2	Structured data types

8.2.5.2.1	Introduction

8.2.5.2.2	Type: ASMessageDelivery

Table 8.2.5.2.2-1: Definition of type ASMessageDelivery

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| originServiceId | string | M | 1 | The service identity of the sending Application Server. |  |
| recipientServiceId (see NOTE 1) | string | O | 1 | The service identity of the receiving Legacy 3GPP UE, Non-3GPP UE or MSGin5G UE. |  |
| groupServiceId (see NOTE 1) | string | O | 0..1 | The service identifier of the target MSGin5G Group. |  |
| broadcastAreaId(see NOTE 1) | string | O | 0..1 | The service identifier of the Broadcast Service Area where the message needs to be broadcast. |  |
| messageTopic (see NOTE 1) | string | O | 0..1 | Indicates which Messaging Topic this message is related to. This IE is mandatory for a message distribution based on topic and is not present in other message scenarios. |  |
| appId | string | O | 0..1 | Identifies the application(s) for which the payload is intended. This list of Application IDs IE is required when the message is sent to one or multiple Application Clients served by same MSGin5G Client. |  |
| messageId | string | M | 1 | Unique identifier of this message. |  |
| security Credentials | string | O | 0..1 | Security information required by the MSGin5G Server. This is a placeholder for SA3 security information. |  |
| deliveryStatusRequired | string | O | 0..1 | Indicates if delivery acknowledgement from the recipient is requested. |  |
| payload | string | O | 0..1 | Payload of the message. |  |
| priority | Priority | O | 0..1 | Application priority level requested for this message. |  |
| messageSegmentFlag | boolean | O | 0..1 | Indicates this message is part of a segmented message. |  |
| messageSegmentParameters | MessageSegmentPapameters | O | 0..1 | The message segment parameters. This IE shall be included only if the value of the message Segment Flag IE indicates that message Segment services are requested. |  |
| storeAndForwardFlag | boolean | M | 1 | An indicator of whether store and forward services are requested for this message. |  |
| storeAndForwardParameters | StoreAndForwardParameters | O | 0..1 | Parameters used by MSGin5G Server for providing store and forward services, This IE shall be included only if the value of the Store and forward flag IE indicates that store and forward services are requested. |  |
| latency | integer | O | 0..1 | The latency requirement for the message which only applies to AS Originating MSGin5G Message. Unit: millisecond. |  |
| NOTE 1: Only one of these IEs shall be included to represent the type of message request. |  |  |  |  |  |

Editor's Note:	whether the attributes in table 8.2.5.2.2-1 are all necessary is FFS.

8.2.5.2.3	Type:UEMessageDelivery

Table 8.2.5.2.3-1: Definition of type UEMessageDelivery

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| originServiceId | string | M | 1 | The service identity of the sending Legacy 3GPP UE or Non-3GPP UE. |  |
| recipientServiceId | string | M | 1 | The service identity of the receiving Application Server or MSGin5G UE. |  |
| appId | string | O | 0..1 | Identifies the application(s) for which the payload is intended. This list of Application IDs IE is required when the message is sent to one or multiple Application Clients served by same MSGin5G Client. |  |
| messageId | string | M | 1 | Unique identifier of this message. |  |
| securityCredentials | string | O | 0..1 | Security information required by the MSGin5G Server. This is a placeholder for SA3 security information. |  |
| deliveryStatusRequired | string | O | 0..1 | Indicates if delivery acknowledgement from the recipient is requested. |  |
| payload | string | O | 0..1 | Payload of the message. |  |
| messageSegmentFlag | boolean | O | 0..1 | Indicates this message is part of a segmented message. |  |
| messageSegmentParameters | MessageSegmentPapameters | O | 0..1 | The message segment parameters. This IE shall be included only if the value of the message Segment Flag IE indicates that message Segment services are requested. |  |
| storeAndForwardFlag | boolean | M | 1 | An indicator of whether store and forward services are requested for this message. |  |
| storeAndForwardParameters | StoreAndForwardParameters | O | 0..1 | Parameters used by MSGin5G Server for providing store and forward services, This IE shall be included only if the value of the Store and forward flag IE indicates that store and forward services are requested. |  |

Editor's Note:	whether the attributes in table 8.2.5.2.3-1 are all necessary is FFS.

8.2.5.2.4	Type: MessageDeliveryAck

Table 8.2.5.2.4-1: Definition of type MessageDeliveryAck

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| originServiceId | string | M | 1 | The service identity of the sending Legacy 3GPP UE, Non-3GPP UE or the sending Application Server. |  |
| messageId | string | M | 1 | Unique identifier of this message. |  |
| status | Status | O | 0..1 | Indicates if delivery is a failure, or if the message is stored for deferred delivery. |  |
| failureCause | string | O | 0..1 | The reason for failure. |  |

8.2.5.2.5	Type:MessageSegmentParameters

Table 8.2.5.2.5-1: Definition of type MessageSegmentParameters

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| segIdentifier | string | O | 0..1 | All segmented messages associated within the same set of segmented messages (i.e. associated with the same MSGin5G message) are assigned the same unique identifier. |  |
| totalNumSeg | integer | O | 0..1 | Indicates the total number of segments for the message. |  |
| segNum | integer | O | 0..1 | Indicates segmented message number of each segmented message within a set of segmented messages. |  |
| lastSegFlag | boolean | O | 0..1 | An indicator of whether this segmented message is the last segment in the set of segmented messages or not. |  |

8.2.5.2.6	Type:StoreAndForwardParameters

ble 8.2.5.2.6-1: Definition of type StoreAndForwardParameters

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| expirationTime | DateTime | O | 0..1 | Indicates message expiration time. |  |

8.2.5.2.7	Type:DeliveryStatusReport

ble 8.2.5.2.5-1: Definition of type DeliveryStatusReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| originServiceId | string | M | 1 | The service identity of the sending Legacy 3GPP UE, Non-3GPP UE or the sending Application Server. |  |
| recipientServiceId(see NOTE) | string | M | 1 | The service identity of the receiving Legacy 3GPP UE, Non-3GPP UE or the receiving Application Server. |  |
| messageId | string | M | 1 | Unique identifier of this message. |  |
| securityCredentials | string | O | 0..1 | Security information required by the MSGin5G Server. This is a placeholder for SA3 security information. |  |
| failureCause | string | O | 0..1 | The Failure Cause indicates the failure reason, if applicable. |  |
| deliveryStatus | DeliveryStatus | M | 1 | The delivery status description, including success or failure in delivery. |  |
| NOTE: When the originator is an Application Server, (i.e. Originating AS Service ID is present), this IE shall be a UE Service ID. |  |  |  |  |  |

8.2.5.3	Simple data types and enumerations

8.2.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

8.2.5.3.2	Simple data types

None.

8.2.5.3.3	Enumeration:Status

Table 8.2.5.3.3-1: Enumeration Status

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| FAILED | Indicates that the message delivery is failed. |  |
| STORED | Indicates that the message is stored for deferred delivery. |  |

8.2.5.3.4	Enumeration:DeliveryStatus

Table 8.2.5.3.4-1: Enumeration DeliveryStatus

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SUCCESS | Indicates that the message delivery is successful. |  |
| FAILED | Indicates that the message delivery is failed. |  |

8.2.5.3.5	Enumeration:Priority

Table 8.2.5.3.5-1: Enumeration Priority

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| HIGH | Indicates the messages should be sent in high priority. |  |
| MIDDLE | Indicates the messages should be sent in middle priority. |  |
| LOW | Indicates the messages should be sent in low priority. |  |

8.2.6	Error Handling

General error responses will be defined in clause 7.7.

8.2.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.2.7-1 lists the supported features for MSGS_MSGDelivery API.

Table 8.2.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

8.x	<API Name – MSGS_xxx> API

Add a copy of this clause for a new API definition, adding all the clauses below. All the clauses are mandatory for each API. Yellow highlighted text needs to be replaced with appropriate clause number and the API, Service operation name.

8.x.1	API URI

8.x.2	Resources

8.x.2.1	Overview

Figure 8.x.2.1-1: Resource URI structure of the <API Name> API

Table 8.x.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.x.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |

8.x.2.2	Resource: <Resource name>

8.x.2.2.1	Description

8.x.2.2.2	Resource Definition

8.x.2.2.3	Resource Standard Methods

8.x.2.2.3.1	<Method Name>

Table 8.x.2.2.3.1-1: URI query parameters supported by the <Method Name> method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| <name> or n/a | <type> or <leave empty> | <M, C or O> | 0..1 or 1 or 0..N or 1..N or <leave empty> | <only if applicable> |

This method shall support the request data structures specified in table 8.x.2.2.3.1-2 and the response data structures and response codes specified in table 8.x.2.2.3.1-3.

Table 8.x.2.2.3.1-2: Data structures supported by the <Method Name> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" or n/a | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 8.x.2.2.3.1-3: Data structures supported by the <Method Name> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" or n/a | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |
| NOTE: The manadatory HTTP error status code for the <method 1> method listed in <Table X of 3GPP TS 29.xxx [x]> also apply. |  |  |  |  |

Table 8.x.2.2.3.1-4: Headers supported by the <e.g. GET> method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| <header name> | <data type> e.g. string | "M", "C" or "O" | "0..1", "1", "1..N", "1..N", or <leave empty> | <description> |

Table 8.x.2.2.3.1-5: Headers supported by the <e.g. 200> response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| <header name> | <data type> e.g. string | "M", "C" or "O" | "0..1", "1", "1..N", "1..N", or <leave empty> | <description> |

Table 8.x.2.2.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| <link name> e.g. search | <resource 1> e.g. Stored Search (Document) | <method 1> e.g. GET | <parameter> e.g. searchId | <description of the link> |

8.x.2.2.4		Resource Custom Operations

The following clauses will specify the custom operations supported by the resource.

It will describe, for each custom operation, the use and the URI of the operation, the HTTP method on which it is mapped, request and response data structures and response codes, and if applicable, HTTP headers specific to the operation.

8.x.2.2.4.1		Overview

Table 8.x.2.2.4.1-1: Custom operations

| Operation name | Custom operaration URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| <custom operation name> | <custom operation URI> | e.g.POST | <Operation executed by Custom operation> |

8.x.2.2.4.2		Operation: < operation 1 >

This clause will specify the meaning of the operation applied on the resource.

8.x.2.2.4.2.1	Description

This sublause will describe the custom operation and what it is used for, and the custom operation's URI.

8.x.2.2.4.2.2	Operation Definition

This clause will specify the custom operation and the HTTP method on which it is mapped.

This operation shall support the request data structures specified in table 8.x.2.2.4.2.2-1 and the response data structure and response codes specified in table 8.x.2.2.4.2.2-2.

Table 8.x.2.2.4.2.2-1: Data structures supported by the <e.g. POST> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 8.x.2.2.4.2.2-2: Data structures supported by the <e.g. POST> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |
| NOTE: The manadatory HTTP error status code for the <e.g. POST> method listed in <Table X of 3GPP TS 29.xxx [x]> also apply. |  |  |  |  |

8.x.3	Custom Operations without associated resources

8.x.3.1	Overview

This clause will specify custom operations without any associated resource supported by this API.

Table 8.x.3.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| <custom operation URI> | e.g.POST | <Operation executed by Custom operation> |

8.x.3.2	Operation: <operation 1>

Where <operation 1> is to be replaced by the name of the custom operation, e.g. Authentication_Information_Request.

It will describe, for each custom operation, the use and the URI of the operation, the HTTP method on which it is mapped, request and response data structures and response codes, and if applicable, HTTP headers specific to the operation.

8.x.3.2.1	Description

This sublause will describe the custom operation and what it is used for, and the custom operation's URI.

8.x.3.2.2	Operation Definition

This clause will specify the custom operation and the HTTP method on which it is mapped.

This operation shall support the response data structures and response codes specified in tables 8.x.3.2.2-1 and 8.x.3.2.2-2.

Table 8.x.3.2.2-1: Data structures supported by the <e.g. POST> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 8.x.3.2.2-2: Data structures supported by the <e.g. POST> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |
| NOTE: The manadatory HTTP error status code for the <e.g. POST> method listed in <Table X of 3GPP TS 29.xxx [x]> also apply. |  |  |  |  |

8.x.3.3	Operation: < operation 2>

And so on if there are more than one custom operations supported by the service. Same structure as in clause 8.x.3.2

8.x.4	Notifications

8.x.4.1	General

Table 8.x.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| <notification 1> e.g. Status Change Notification | < Callback URI > e.g. {StatusCallbackUri} | e.g POST | e.g. Notify Event |

8.x.4.2	<notification 1>

8.x.4.2.1	Description

8.x.4.2.2	Notification definition

Callback URI: <Notification resource URI>

This method shall support the URI query parameters specified in table 8.x.4.2.2-1.

Table 8.x.4.2.2-1: URI query parameters supported by the <Method Name> method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 8.x.4.2.2-2 and the response data structures and response codes specified in table 8.x.4.2.2-3.

Table 8.x.4.2.2-2: Data structures supported by the <Method Name> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 8.x.4.2.2-3: Data structures supported by the <Method Name> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |

8.x.5	Data Model

8.x.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause <7.X related to EdgeApp design aspects common for all APIs> apply to this API

Table 8.x.5.1-1 specifies the data types defined specifically for the <API Name> API service.

Table 8.x.5.1-1: <API Name> API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |

Table 8.x.5.1-2 specifies data types re-used by the <API Name> API service.

Table 8.x.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |

8.x.5.2	Structured data types

8.x.5.2.1	Introduction

8.x.5.2.2	Type: <Data type name>

Table 8.x.5.2.2-1: Definition of type <Data Type name>

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| <attribute name> | "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N" | <only if applicable> |  |

8.x.5.3	Simple data types and enumerations

This clause will define simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

8.x.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

8.x.5.3.2	Simple data types

The simple data types defined in table 8.x.5.3.2-1 shall be supported.

Table 8.x.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
|  | <one simple data type, i.e. boolean, integer, number, or string> |  |  |

8.x.5.3.3	Enumeration: <EnumType1>

The enumeration <EnumType1> represents <something>. It shall comply with the provisions defined in table 8.x.5.3.3-1.

Table 8.x.5.3.3-1: Enumeration < EnumType1>

| Enumeration value | Description | Applicability |
| --- | --- | --- |

8.x.6	Error Handling

8.x.7	Feature negotiation

General feature negotiation procedures are defined in clause <7.X>. Table 8.x.7-1 lists the supported features for <API name> API.

Table 8.x.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

9	Message Gateway API definition

9.1	MSGG_L3GDelivery API

9.1.1	API URI

9.1.2	Resources

9.1.3	Custom Operations without associated resources

.

9.1.4	Notifications

.

9.1.5	Data Model

9.1.6	Error Handling

9.1.7	Feature negotiation

9.2	MSGG_N3GDelivery API

9.2.1	API URI

9.2.2	Resources

9.2.3	Custom Operations without associated resources

.

9.2.4	Notifications

.

9.2.5	Data Model

9.2.6	Error Handling

9.2.7	Feature negotiation

9.x	<API Name – MSGG_xxx> API

Add a copy of this clause for a new API definition, adding all the clauses below. All the clauses are mandatory for each API. Yellow highlighted text needs to be replaced with appropriate clause number and the API, Service operation name.

9.x.1	API URI

9.x.2	Resources

9.x.2.1	Overview

Figure 9.x.2.1-1: Resource URI structure of the <API Name> API

Table 9.x.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 9.x.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |

9.x.2.2	Resource: <Resource name>

9.x.2.2.1	Description

9.x.2.2.2	Resource Definition

9.x.2.2.3	Resource Standard Methods

9.x.2.2.3.1	<Method Name>

Table 9.x.2.2.3.1-1: URI query parameters supported by the <Method Name> method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| <name> or n/a | <type> or <leave empty> | <M, C or O> | 0..1 or 1 or 0..N or 1..N or <leave empty> | <only if applicable> |

This method shall support the request data structures specified in table 9.x.2.2.3.1-2 and the response data structures and response codes specified in table 9.x.2.2.3.1-3.

Table 9.x.2.2.3.1-2: Data structures supported by the <Method Name> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" or n/a | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 9.x.2.2.3.1-3: Data structures supported by the <Method Name> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" or n/a | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |
| NOTE: The manadatory HTTP error status code for the <method 1> method listed in <Table X of 3GPP TS 29.xxx [x]> also apply. |  |  |  |  |

Table 9.x.2.2.3.1-4: Headers supported by the <e.g. GET> method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| <header name> | <data type> e.g. string | "M", "C" or "O" | "0..1", "1", "1..N", "1..N", or <leave empty> | <description> |

Table 9.x.2.2.3.1-5: Headers supported by the <e.g. 200> response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| <header name> | <data type> e.g. string | "M", "C" or "O" | "0..1", "1", "1..N", "1..N", or <leave empty> | <description> |

Table 9.x.2.2.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| <link name> e.g. search | <resource 1> e.g. Stored Search (Document) | <method 1> e.g. GET | <parameter> e.g. searchId | <description of the link> |

9.x.2.2.4		Resource Custom Operations

The following clauses will specify the custom operations supported by the resource.

It will describe, for each custom operation, the use and the URI of the operation, the HTTP method on which it is mapped, request and response data structures and response codes, and if applicable, HTTP headers specific to the operation.

9.x.2.2.4.1		Overview

Table 9.x.2.2.4.1-1: Custom operations

| Operation name | Custom operaration URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| <custom operation name> | <custom operation URI> | e.g.POST | <Operation executed by Custom operation> |

9.x.2.2.4.2		Operation: < operation 1 >

This clause will specify the meaning of the operation applied on the resource.

9.x.2.2.4.2.1	Description

This sublause will describe the custom operation and what it is used for, and the custom operation's URI.

9.x.2.2.4.2.2	Operation Definition

This clause will specify the custom operation and the HTTP method on which it is mapped.

This operation shall support the request data structures specified in table 9.x.2.2.4.2.2-1 and the response data structure and response codes specified in table 9.x.2.2.4.2.2-2.

Table 9.x.2.2.4.2.2-1: Data structures supported by the <e.g. POST> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 9.x.2.2.4.2.2-2: Data structures supported by the <e.g. POST> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |
| NOTE: The manadatory HTTP error status code for the <e.g. POST> method listed in <Table X of 3GPP TS 29.xxx [x]> also apply. |  |  |  |  |

9.x.3	Custom Operations without associated resources

9.x.3.1	Overview

This clause will specify custom operations without any associated resource supported by this API.

Table 9.x.3.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| <custom operation URI> | e.g.POST | <Operation executed by Custom operation> |

9.x.3.2	Operation: <operation 1>

Where <operation 1> is to be replaced by the name of the custom operation, e.g. Authentication_Information_Request.

It will describe, for each custom operation, the use and the URI of the operation, the HTTP method on which it is mapped, request and response data structures and response codes, and if applicable, HTTP headers specific to the operation.

9.x.3.2.1	Description

This sublause will describe the custom operation and what it is used for, and the custom operation's URI.

9.x.3.2.2	Operation Definition

This clause will specify the custom operation and the HTTP method on which it is mapped.

This operation shall support the response data structures and response codes specified in tables 9.x.3.2.2-1 and 9.x.3.2.2-2.

Table 9.x.3.2.2-1: Data structures supported by the <e.g. POST> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 9.x.3.2.2-2: Data structures supported by the <e.g. POST> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |
| NOTE: The manadatory HTTP error status code for the <e.g. POST> method listed in <Table X of 3GPP TS 29.xxx [x]> also apply. |  |  |  |  |

9.x.3.3	Operation: < operation 2>

And so on if there are more than one custom operations supported by the service. Same structure as in clause 9.x.3.2

9.x.4	Notifications

9.x.4.1	General

Table 9.x.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| <notification 1> e.g. Status Change Notification | < Callback URI > e.g. {StatusCallbackUri} | e.g POST | e.g. Notify Event |

9.x.4.2	<notification 1>

9.x.4.2.1	Description

9.x.4.2.2	Notification definition

Callback URI: <Notification resource URI>

This method shall support the URI query parameters specified in table 9.x.4.2.2-1.

Table 9.x.4.2.2-1: URI query parameters supported by the <Method Name> method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 9.x.4.2.2-2 and the response data structures and response codes specified in table 9.x.4.2.2-3.

Table 9.x.4.2.2-2: Data structures supported by the <Method Name> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 9.x.4.2.2-3: Data structures supported by the <Method Name> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |

9.x.5	Data Model

9.x.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause <7.X related to EdgeApp design aspects common for all APIs> apply to this API

Table 9.x.5.1-1 specifies the data types defined specifically for the <API Name> API service.

Table 9.x.5.1-1: <API Name> API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |

Table 9.x.5.1-2 specifies data types re-used by the <API Name> API service.

Table 9.x.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |

9.x.5.2	Structured data types

9.x.5.2.1	Introduction

9.x.5.2.2	Type: <Data type name>

Table 9.x.5.2.2-1: Definition of type <Data Type name>

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| <attribute name> | "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N" | <only if applicable> |  |

9.x.5.3	Simple data types and enumerations

This clause will define simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

9.x.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

9.x.5.3.2	Simple data types

The simple data types defined in table 9.x.5.3.2-1 shall be supported.

Table 9.x.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
|  | <one simple data type, i.e. boolean, integer, number, or string> |  |  |

9.x.5.3.3	Enumeration: <EnumType1>

The enumeration <EnumType1> represents <something>. It shall comply with the provisions defined in table 9.x.5.3.3-1.

Table 9.x.5.3.3-1: Enumeration < EnumType1>

| Enumeration value | Description | Applicability |
| --- | --- | --- |

9.x.6	Error Handling

9.x.7	Feature negotiation

General feature negotiation procedures are defined in clause <7.X>. Table 9.x.7-1 lists the supported features for <API name> API.

Table 9.x.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

10	Security

11	Using Common API Framework

11.1	General

11.2	Security

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-10 | CT3#118-e | C3-215030 |  |  |  | Draft skeleton provided by the rapporteur | 0.0.0 |
| 2021-10 | CT3#118-e | C3-215479 |  |  |  | Inclusion of documents agreed in CT3#118e C3-215116, C3-215165, C3-215206, C3-215432, C3-215441, C3-215462. Editorial change from the rapporteur. Correction from the rapporteur. | 0.1.0 |
| 2021-11 | CT3#119-e |  |  |  |  | Inclusion of documents agreed in CT3#119e C3-216175, C3-216525, C3-216528, C3-216529, C3-216545, C3-216546, C3-216581. Correction from the rapporteur. | 0.2.0 |