| 3GPP TS 29.257 V1.2.0 (2022-02) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Application layer support for Uncrewed Aerial System (UAS); UAS Application Enabler (UAE) Server Services; Stage 3 (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions, symbols and abbreviations	8

3.1	Definitions	8

3.2	Symbols	9

3.3	Abbreviations	9

4	Overview	9

5	Services offered by the UAE Server	10

5.1	Introduction	10

5.2	UAE_C2OperationModeManagement Service	10

5.2.1	Service Description	10

5.2.2	Service Operations	11

5.2.2.1	Introduction	11

5.2.2.2	UAE_C2OperationModeManagement_Initiate	11

5.2.2.2.1	General	11

5.2.2.2.2	C2 Operation Mode Initiation	11

5.2.2.3	UAE_C2OperationModeManagement_Notify	12

5.2.2.3.1	General	12

5.2.2.3.2	C2 Operation Mode Management Completion Notification	13

5.2.2.3.3	Selected C2 Communication Mode Notification	13

5.2.2.3.4	C2 Communication Mode Switching Notification	14

5.3	UAE_RealtimeUAVStatus Service	15

5.3.1	Service Description	15

5.3.2	Service Operations	15

5.3.2.1	Introduction	15

5.3.2.2	UAE_RealtimeUAVStatus_Subscribe	16

5.3.2.2.1	General	16

5.3.2.2.2	Subscribe to real-time UAV status information reporting	16

5.3.2.2.3	Update an existing real-time UAV status information reporting subscription	17

5.3.2.3	UAE_RealtimeUAVStatus_Unsubscribe	17

5.3.2.3.1	General	17

5.3.2.3.2	Unsubscribe from real-time UAV status information reporting	18

5.3.2.4	UAE_RealtimeUAVStatus_Notify	18

5.3.2.4.1	General	18

5.3.2.4.2	Real-time UAV Status Notification	18

6	API Definitions	19

6.1	UAE_C2OperationModeManagement Service API	19

6.1.1	Introduction	19

6.1.2	Usage of HTTP	19

6.1.3	Resources	19

6.1.4	Custom Operations without associated resources	20

6.1.4.1	Overview	20

6.1.4.2	Operation: Initiate	20

6.1.4.2.1	Description	20

6.1.4.2.2	Operation Definition	20

6.1.5	Notifications	21

6.1.5.1	General	21

6.1.5.2	C2 Operation Mode Management Completion Notification	22

6.1.5.2.1	Description	22

6.1.5.2.2	Target URI	22

6.1.5.2.3	Standard Methods	22

6.1.5.2.3.1	POST	22

6.1.5.3	Selected C2 Communication Mode Notification	23

6.1.5.3.1	Description	23

6.1.5.3.2	Target URI	23

6.1.5.3.3	Standard Methods	23

6.1.5.3.3.1	POST	23

6.1.5.4	C2 Communication Mode Switching Notification	24

6.1.5.4.1	Description	24

6.1.5.4.2	Target URI	24

6.1.5.4.3	Standard Methods	25

6.1.5.4.3.1	POST	25

6.1.6	Data Model	26

6.1.6.1	General	26

6.1.6.2	Structured data types	27

6.1.6.2.1	Introduction	27

6.1.6.2.2	Type: ConfigureData	28

6.1.6.2.3	Type: SelectedC2CommModeNotif	29

6.1.6.2.4	Type: C2CommModeSwitchNotif	29

6.1.6.2.5	Type: C2Result	30

6.1.6.2.6	Type: UasId	30

6.1.6.2.7	Type: UavId	30

6.1.6.2.8	Type: C2ServiceArea	31

6.1.6.2.9	Type: C2OpModeMngtCompStatus	31

6.1.6.2.10	Type: C2SwitchPolicies	31

6.1.6.2.11	Type: C2LinkQualityThrlds	32

6.1.6.3	Simple data types and enumerations	32

6.1.6.3.1	Introduction	32

6.1.6.3.2	Simple data types	32

6.1.6.3.3	Enumeration: C2CommMode	32

6.1.6.3.4	Enumeration: C2CommModeSwitching	33

6.1.6.3.5	Enumeration: C2SwitchingCause	33

6.1.6.3.6	Enumeration: C2OpModeStatus	34

6.1.6.4	Data types describing alternative data types or combinations of data types	34

6.1.6.5	Binary data	34

6.1.6.5.1	Binary Data Types	34

6.1.7	Error Handling	35

6.1.7.1	General	35

6.1.7.2	Protocol Errors	35

6.1.7.3	Application Errors	35

6.1.8	Feature negotiation	35

6.1.9	Security	35

6.2	UAE_RealtimeUAVStatus Service API	35

6.2.1	Introduction	35

6.2.2	Usage of HTTP	36

6.2.3	Resources	36

6.2.3.1	Overview	36

6.2.3.2	Resource: Real-time UAV Status Subscriptions	37

6.2.3.2.1	Description	37

6.2.3.2.2	Resource Definition	37

6.2.3.2.3	Resource Standard Methods	37

6.2.3.2.3.1	GET	37

6.2.3.2.3.2	POST	38

6.2.3.2.4	Resource Custom Operations	38

6.2.3.3	Resource: Individual Real-time UAV Status Subscription	39

6.2.3.3.1	Description	39

6.2.3.3.2	Resource Definition	39

6.2.3.3.3	Resource Standard Methods	39

6.2.3.3.3.1	GET	39

6.2.3.3.3.2	PUT	40

6.2.3.3.3.3	DELETE	41

6.2.3.3.4	Resource Custom Operations	42

6.2.4	Custom Operations without associated resources	42

6.2.5	Notifications	42

6.2.5.1	General	42

6.2.5.2	Real-time UAV Status Notification	42

6.2.5.2.1	Description	42

6.2.5.2.2	Target URI	42

6.2.5.2.3	Standard Methods	42

6.2.5.2.3.1	POST	42

6.2.6	Data Model	43

6.2.6.1	General	43

6.2.6.2	Structured data types	44

6.2.6.2.1	Introduction	44

6.2.6.2.2	Type: RTUavStatusSubsc	44

6.2.6.2.3	Type: RTUavStatusNotif	44

6.2.6.2.4	Type: RTUavStatus	45

6.2.6.2.5	Type: UavNetConnStatus	45

6.2.6.3	Simple data types and enumerations	45

6.2.6.3.1	Introduction	45

6.2.6.3.2	Simple data types	45

6.2.6.4	Data types describing alternative data types or combinations of data types	45

6.2.6.5	Binary data	46

6.2.6.5.1	Binary Data Types	46

6.2.7	Error Handling	46

6.2.7.1	General	46

6.2.7.2	Protocol Errors	46

6.2.7.3	Application Errors	46

6.2.8	Feature negotiation	46

6.2.9	Security	46

7	Using Common API Framework	46

7.1	General	46

7.2	Security	47

Annex A (normative): OpenAPI specification	48

A.1	General	48

A.2	UAE_C2OperationModeManagement API	48

A.3	UAE_RealtimeUAVStatus API	54

Annex B (informative): Withdrawn API versions	60

B.1	General	60

B.2	UAE_C2OperationModeManagement API	60

B.3	UAE_RealtimeUAVStatus API	60

Annex C (informative): Change history	61

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the stage 3 Protocol and data model for the UAS Application Enabler (UAE) Server services, for enabling the support of Uncrewed Aerial System (UAS) applications over 3GPP networks. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the UAE Server.

The stage 2 application layer architecture for Uncrewed Aerial System (UAS), functional requirements, procedures and information flows necessary for enabling Uncrewed Aerial System (UAS) applications over 3GPP networks are specified in 3GPP TS 23.255 [6].

The common protocol and interface aspects for API definition are specified in clause 5.2 of 3GPP TS 29.122 [2].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 29.122: "T8 reference point for Northbound Application Programming Interfaces (APIs)".

[3]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[4]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[5]	3GPP TR 21.900: "Technical Specification Group working methods".

[6]	3GPP TS 23.255: "Application layer support for Uncrewed Aerial System (UAS); Functional architecture and information flows".

[7]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[8]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[9]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[10]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[11]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[12]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purpose of the present document, the terms and definitions given in clause 3 of 3GPP TS 23.255 [6] also apply, including the ones referencing other specifications.

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BVLOS	Beyond Visual Line Of Sight

C2	Command and Control

CAA	Civil Aviation Authorities

RSRP	Reference Signal Received Power

UAE	UAS Application Enabler

UAS	Uncrewed Arial System

UASS	UAS Application Specific Server

UAV	Uncrewed Aerial Vehicle

UAV-C	Uncrewed Aerial Vehicle – Controller

USS	UAS Service Supplier

UTM	UAS Traffic Management

# 4	Overview

The UAS Application Enabler (UAE) Server forms part of the UAS application enabler layer that aims to ensure the efficient use and deployment of UAS over 3GPP systems. The UAE Server supports for this purpose, among other functionalities defined in 3GPP TS 23.255 [6], the following functionalities:

-	UAS application layer support functions to a UASS (e.g. USS/UTM) over the Us reference point, i.e.:

-	C2 operation mode configuration management for a UAS (i.e. pair of UAV and UAV-C);

-	C2 communication modes switching control and management for a UAS (i.e. pair of UAV and UAV-C);

-	Real-Time UAV Connection Status Monitoring and Location reporting; and

-	interaction with other UAE Servers over the UAE-E reference point, in order to support distributed UAE Server deployments.

Figure 4-1 shows the reference model of the UAS Application Layer, with a focus on the UAE Server:

Figure 4-1: UAS Application Layer functional model

# 5	Services offered by the UAE Server

## 5.1	Introduction

The UAE Server provides the following services:

-	UAE_C2OperationModeManagement

-	UAE_RealtimeUAVStatus

Table 5.1-1 summarizes the corresponding APIs defined in this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | API Name | Annex |
| --- | --- | --- | --- | --- | --- |
| UAE_C2OperationModeManagement | 5.2 | UAE Server C2 Operation Mode Management Service | TS29257_UAE_C2OperationModeManagement.yaml | uae-c2opmode-mngt | A.2 |
| UAE_RealtimeUAVStatus | 5.3 | UAE Server Real-time UAV Status Service | TS29257_UAE_RealtimeUAVStatus.yaml | uae-uav-status | A.3 |

## 5.2	UAE_C2OperationModeManagement Service

### 5.2.1	Service Description

The UAE_C2OperationModeManagement service exposed by the UAE Server enables a UASS (e.g. USS/UTM) to:

-	communicate C2 operation mode configuration information to the UAE Server for a UAS (i.e. pair of UAV and UAV-C);

-	receive notifications from the UAE Server on the C2 operation mode management completion;

-	receive notifications from the UAE Server on the C2 communication mode selected by a UAS (i.e. pair of UAV and UAV-C); and

-	receive notifications from the UAE Server when C2 communication mode switching is carried out and decide whether to authorize it or not.

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

The service operations defined for the UAE_C2OperationModeManagement service are shown in table 5.2.2.1-1.

Table 5.2.2.1-1: UAE_C2OperationModeManagement Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| UAE_C2OperationModeManagement_Initiate | This service operation enables a UASS to initiate the configuration of C2 operation modes for a UAS (i.e. pair of UAV and UAV-C) by communicating the associated C2 operation mode configuration information to the UAE Server. | e.g. UASS |
| UAE_C2OperationModeManagement_Notify | This service operation enables a UAE Server to notify a previously subscribed UASS either: - on C2 operation mode management completion; - on the C2 communication mode selected by a UAS (i.e. pair of UAV and UAV-C); or - when C2 communication mode switching is carried out. The UASS may then confirm the targeted C2 communication mode switching or not. | UAE Server |

#### 5.2.2.2	UAE_C2OperationModeManagement_Initiate

##### 5.2.2.2.1	General

This service operation is used by a UASS to request the provisioning of C2 operation mode configuration information for a UAS (i.e. pair of UAV and UAV-C) to the UAE Server.

The following procedures are supported by the "UAE_C2OperationModeManagement_Initiate" service operation:

-	C2 Operation Mode Initiation procedure.

##### 5.2.2.2.2	C2 Operation Mode Initiation

Figure 5.2.2.2.2-1 depicts a scenario where a UASS sends a request to the UAE Server to request the provisioning of C2 operation mode configuration information for a UAS (i.e. pair of UAV and UAV-C) (see also clause 7.4 of 3GPP°TS°23.255°[6]).

Figure 5.2.2.2.2-1: C2 Operation Mode Initiation procedure

- The UASS shall send for this purpose an HTTP POST request (custom operation: "Initiate") to the UAE Server, with the request URI set to "{apiRoot}/uae-c2opmode-mngt/<apiVersion>/initiate" and the request body including the ConfigureData data structure that shall contain:

-	the identifier of the UASS that is sending the request, within the "uassId" attribute;

-	the identifier of the target UAS (i.e. pair of UAV and UAV-C) to which the C2 Operation Mode configuration information is destined, within the "uasId" attribute;

-	the allowed C2 communication modes for the UAS (i.e. pair of UAV and UAV-C) identified by the "uasId" attribute, within the "allowedC2CommModes" attribute;

-	the C2 Operation Mode switching types to be supported by the UAE Server, within the "c2CommModeSwitchTypes" attribute;

-	the notification URI via which the UASS desires to receive notifications from the UAE Server, within the "notificationUri" attribute;

-	the primary C2 communication mode (i.e. either Direct C2 Communication mode or Network-Assisted C2 Communication mode) to be used by the UAS (i.e. pair of UAV and UAV-C) identified by the "uasId" attribute, within the "primaryC2CommMode" attribute; and

-	the C2 operation mode switching policies, within the "c2SwitchPolicies" attribute;

and may also contain:

-	the secondary C2 communication mode (i.e. either Direct C2 Communication mode or Network-Assisted C2 Communication mode) to be used by the UAS (i.e. pair of UAV and UAV-C) identified by the "uasId" attribute, within the "secondaryC2CommMode" attribute;

-	the service area within which the C2 operation mode management request applies (i.e. a geographical area or a topological area), within the "c2ServiceArea" attribute; and

-	the list of features supported by the UASS among the ones defined in clause 6.1.8, within the "suppFeat" attribute.

2a.	Upon success, the UAE Server shall respond with an HTTP "200 OK" status code with the response body including the C2Result data structure which shall contain a feedback from the UAE Server on whether the request for C2 Operation Mode configuration information provisioning is confirmed (i.e. can be undertaken by the UAE Server) or not. The C2Result data structure may also contain the list of negotiated supported features.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body.

#### 5.2.2.3	UAE_C2OperationModeManagement_Notify

##### 5.2.2.3.1	General

This service operation is used by a UAE Server to notify a previously subscribed UASS either on C2 operation mode management completion, on the C2 communication mode selected by a UAS (i.e. pair of UAV and UAV-C) or when C2 communication mode switching is carried out. For the latter, the UASS may then confirm the targeted C2 communication mode switching or not. See also clause 7.4 of 3GPP°TS°23.255 [6].

The following procedures are supported by the "UAE_C2OperationModeManagement_Notify" service operation:

-	C2 Operation Mode Management Completion Notification.

-	Selected C2 Communication Mode Notification.

-	C2 Communication Mode Switching Notification.

##### 5.2.2.3.2	C2 Operation Mode Management Completion Notification

Figure 5.2.2.3.2-1 depicts a scenario where the UAE Server sends a request to notify a previously subscribed UASS on the C2 operation mode management completion status for a UAS (i.e. pair of UAV and UAV-C). See also clause 7.4 of 3GPP°TS°23.255°[6].

Figure 5.2.2.3.2-1: C2 Operation Mode Management Completion Notification procedure

1.	The UAE Server shall send for this purpose an HTTP POST request to the UASS with the request URI set to "{notificationUri}/c2mode-mngt-completion", where the "notificationUri" is set to the value received from the UASS during the C2 Operation Mode Initiation procedure defined in clause 5.2.2.2, and the request body including the C2OpModeMngtCompStatus data structure that shall contain:

-	the identifier of the UAS (i.e. pair of UAV and UAV-C) to which the notification is related, within the "uasId" attribute; and

-	the C2 operation mode management completion status (i.e. either successful or not successful) for the concerned UAS (i.e. pair of UAV and UAV-C), within the "status" attribute.

2a.	Upon success, the UASS shall respond to the UAE Server with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

If the UASS is not able to handle the notification request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative UASS where the notification should be sent, as defined in clause 5.2.10 of 3GPP TS 29.122 [2].

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body.

##### 5.2.2.3.3	Selected C2 Communication Mode Notification

Figure 5.2.2.3.3-1 depicts a scenario where the UAE Server sends a request to notify a previously subscribed UASS on the C2 communication mode selected by a UAS (i.e. pair of UAV and UAV-C). See also clause 7.4 of 3GPP°TS°23.255°[6].

Figure 5.2.2.3.3-1: Selected C2 Communication Mode Notification procedure

1.	The UAE Server shall send for this purpose an HTTP POST request to the UASS with the request URI set to "{notificationUri}/inform-selec-c2mode", where the "notificationUri" is set to the value received from the UASS during the C2 Operation Mode Initiation procedure defined in clause 5.2.2.2, and the request body including the SelectedC2CommModeNotif data structure that shall contain:

-	the identifier of the UAS (i.e. pair of UAV and UAV-C) to which the notification is related, within the "uasId" attribute; and

-	the primary C2 communication mode selected by the UAS (i.e. pair of UAV and UAV-C) identified by the "uasId" attribute, within the "selPrimaryC2CommMode" attribute;

and may also contain:

-	the secondary C2 communication mode selected by the UAS (i.e. pair of UAV and UAV-C) identified by the "uasId" attribute, within the "selSecondaryC2CommMode" attribute.

2a.	Upon success, the UASS shall respond to the UAE Server with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

If the UASS is not able to handle the notification request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative UASS where the notification should be sent, as defined in clause 5.2.10 of 3GPP TS 29.122 [2].

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body.

##### 5.2.2.3.4	C2 Communication Mode Switching Notification

Figure 5.2.2.3.4-1 depicts a scenario where the UAE Server sends a request to notify a previously subscribed UASS on the targeted C2 communication mode switching for a UAS (i.e. pair of UAV and UAV-C) and may request confirmation from the UASS. See also clause 7.4 of 3GPP°TS°23.255°[6].

Figure 5.2.2.3.4-1: C2 Communication Mode Switching Notification procedure

1.	The UAE Server shall send for this purpose an HTTP POST request to the UASS with the request URI set to "{notificationUri}/inform-c2mode-switch", where the "notificationUri" is set to the value received from the UASS during the C2 Operation Mode Initiation procedure defined in clause 5.2.2.2, and the request body including the C2CommModeSwitchNotif data structure that shall contain:

-	the identifier of the UAE Server that is sending the notification and possibly requesting C2 Communication Mode switching confirmation for a UAS (i.e. pair of UAV and UAV-C) from the UASS, within the "uaeServerId" attribute;

-	the identifier of the UAS (i.e. pair of UAV and UAV-C) to which the provided C2 Communication Mode switching information is related, within the "uasId" attribute; and

-	the targeted C2 Communication Mode switching for the UAS (i.e. pair of UAV and UAV-C) identified by the "uasId" attribute, within the "c2CommModeSwitchType" attribute;

And may contain:

-	the C2 Communication Mode switching cause, within the "switchingCause" attribute.

2a.	Upon success, if the UASS has to confirm (i.e. approve) the C2 Communication Mode switching operation to the UAE Server, the UASS shall respond with an HTTP "200 OK" status code with the response body including the C2Result data structure which shall contain a feedback from the UASS on whether this C2 Communication Mode switching is confirmed (i.e. approved) or not.

2b.	Otherwise, upon success, if the UASS does not have to confirm (i.e. approve) the C2 Communication Mode switching operation to the UAE Server, the UASS shall respond to the UAE Server with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

If the UASS is not able to handle the notification request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative UASS where the notification should be sent, as defined in clause 5.2.10 of 3GPP TS 29.122 [2].

2c.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body.

## 5.3	UAE_RealtimeUAVStatus Service

### 5.3.1	Service Description

The UAE_RealtimeUAVStatus service exposed by the UAE Server enables a UASS (e.g. USS/UTM) to:

-	subscribe to real-time UAV status information reporting;

-	update an existing real-time UAV status information reporting subscription;

-	receive real-time UAV status notifications; and

-	unsubscribe from real-time UAV status information reporting.

The UAV status information includes the UAV network connection status information and the UAV location information.

### 5.3.2	Service Operations

#### 5.3.2.1	Introduction

The service operations defined for the UAE_RealtimeUAVStatus service are shown in table 5.3.2.1-1.

Table 5.3.2.1-1: UAE_RealtimeUAVStatus Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| UAE_RealtimeUAVStatus_Subscribe | This service operation enables a UASS to subscribe to real-time UAV status information reporting or update an existing real-time UAV status information reporting subscription. | e.g. UASS |
| UAE_RealtimeUAVStatus_Unsubscribe | This service operation enables a UASS to unsubscribe from real-time UAV status information reporting. | e.g. UASS |
| UAE_RealtimeUAVStatus_Notify | This service operation enables a UAE Server to notify a previously subscribed UASS on real-time UAV status information. | UAE Server |

#### 5.3.2.2	UAE_RealtimeUAVStatus_Subscribe

##### 5.3.2.2.1	General

This service operation is used by a UASS to subscribe to real-time UAV status information reporting or update an existing real-time UAV status information reporting subscription.

The following procedures are supported by the "UAE_RealtimeUAVStatus_Subscribe" service operation:

-	Subscribe to real-time UAV status information reporting.

-	Update an existing real-time UAV status information reporting subscription.

##### 5.3.2.2.2	Subscribe to real-time UAV status information reporting

Figure 5.3.2.2.2-1 depicts a scenario where a UASS sends a request to the UAE Server to request the creation of a subscription to real-time UAV status information reporting (see also clause 7.5 of 3GPP°TS°23.255°[6]).

Figure 5.3.2.2.2-1: Procedure for subscribing to real-time UAV status information reporting

1.	In order to subscribe to real-time UAV status reporting, the UASS shall send an HTTP POST request to the UAE Server, with the request URI set to "{apiRoot}/uae-uav-status/<apiVersion>/subscriptions" and the request body including the RTUavStatusSubsc data structure that shall contain:

-	the identifier of the UASS that is sending the request, within the "uassId" attribute;

-	the identifier(s) of the target UAV(s) to which the subscription is related, within the "uavIds" attribute;

-	the notification URI via which the UASS desires to receive real-time UAV status notifications from the UAE Server, within the "notificationUri" attribute; and

-	the list of features supported by the UASS among the ones defined in clause 6.2.8, within the "suppFeat" attribute.

2a.	Upon success, the UAE Server shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created Individual Real-time UAV Status Subscription resource within the RTUavStatusSubsc data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body.

##### 5.3.2.2.3	Update an existing real-time UAV status information reporting subscription

Figure 5.3.2.2.3-1 depicts a scenario where a UASS sends a request to the UAE Server to request the update of an existing subscription to real-time UAV status information reporting.

Figure 5.3.2.2.3-1: Procedure for updating a real-time UAV status information reporting subscription

- In order to update an existing real-time UAV status reporting subscription, the UASS shall send an HTTP PUT request to the UAE Server, with the request URI set to "{apiRoot}/uae-uav-status/<apiVersion>/subscriptions/{subscriptionId}", requesting to update the Individual Real-time UAV Status Subscription resource identified by the provided "subscriptionId" path segment. The request body shall include an updated representation of the resource within the RTUavStatusSubsc data structure that shall contain:

-	the identifier of the UASS that is sending the request, within the "uassId" attribute;

NOTE:	An alternative UASS than the one that requested the creation of the subscription resource can send this subscription update request.

-	the same or an updated list of identifier(s) of the target UAV(s) to which the subscription is related, within the "uavIds" attribute; and

-	the same or an updated notification URI via which the UASS desires to receive real-time UAV status notifications from the UAE Server, within the "notificationUri" attribute.

2a.	Upon success, the UAE Server shall update the concerned Individual Real-time UAV Status Subscription resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated Individual Real-time UAV Status Subscription resource within the RTUavStatusSubsc data structure; or

-	an HTTP "204 No Content" status code.

If the UAE Server is not able to handle the request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI of the resource located in an alternative UAE Server, as defined in clause 5.2.10 of 3GPP TS 29.122 [2].

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT response body.

#### 5.3.2.3	UAE_RealtimeUAVStatus_Unsubscribe

##### 5.3.2.3.1	General

This service operation is used by a UASS to unsubscribe from real-time UAV status information reporting.

The following procedures are supported by the "UAE_RealtimeUAVStatus_Unsubscribe" service operation:

-	Unsubscribe from real-time UAV status information reporting.

##### 5.3.2.3.2	Unsubscribe from real-time UAV status information reporting

Figure 5.3.2.3.2-1 depicts a scenario where a UASS sends a request to the UAE Server to request the deletion of an existing Individual Real-time UAV Status Subscription resource (see also clause 7.5 of 3GPP°TS°23.255°[6]).

Figure 5.3.2.3.2-1: Procedure for unsubscribing from real-time UAV status information reporting

- In order to unsubscribe from real-time UAV status reporting, the UASS shall send an HTTP DELETE request to the UAE Server, with the request URI set to "{apiRoot}/uae-uav-status/<apiVersion>/subscriptions/{subscriptionId}", requesting to delete the Individual Real-time UAV Status Subscription resource identified by the provided "subscriptionId" path segment.

2a.	Upon success, the UAE Server shall respond with an HTTP "204 No Content" status code.

If the UAE Server is not able to handle the request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI of the resource located in an alternative UAE Server, as defined in clause 5.2.10 of 3GPP TS 29.122 [2].

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body.

#### 5.3.2.4	UAE_RealtimeUAVStatus_Notify

##### 5.3.2.4.1	General

This service operation is used by a UAE Server to notify a previously subscribed UASS on real-time UAV status information. See also clause 7.5 of 3GPP°TS°23.255 [6].

The following procedures are supported by the "UAE_RealtimeUAVStatus_Notify" service operation:

-	Real-time UAV Status Notification.

##### 5.3.2.4.2	Real-time UAV Status Notification

Figure 5.3.2.4.2-1 depicts a scenario where the UAE Server sends a request to notify a previously subscribed UASS on real-time UAV status information. See also clause 7.5 of 3GPP°TS°23.255°[6].

Figure 5.3.2.4.2-1: Real-time UAV Status Notification procedure

1.	The UAE Server shall send for this purpose an HTTP POST request to the UASS with the request URI set to "{notificationUri}/uav-status", where the "notificationUri" is set to the value received from the UASS during the real-time UAV status reporting subscription creation/update procedures defined in clause 5.3.2.2, and the request body including the RTUavStatusNotif data structure that shall contain:

-	The identifier of the Individual Real-time UAV Status Subscription to which the notification is related, within the "subscriptionId" attribute; and

-	The real-time UAV status information for the concerned UAV(s), within the "rTUavStatus" attribute.

2a.	Upon success, the UASS shall respond with an HTTP "204 No Content" status code to acknowledge the reception of the notification to the UAE Server.

If the UASS is not able to handle the notification request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative UASS where the notification should be sent, as defined in clause 5.2.10 of 3GPP TS 29.122 [2].

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body.

# 6	API Definitions

## 6.1	UAE_C2OperationModeManagement Service API

### 6.1.1	Introduction

The UAE_C2OperationModeManagement service shall use the UAE_C2OperationModeManagement API.

The API URI of the UAE_C2OperationModeManagement API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [2], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [2].

-	The <apiName> shall be "uae-c2opmode-mngt".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [2].

### 6.1.2	Usage of HTTP

The provisions of clause 5.2.2 of 3GPP TS 29.122 [2] shall apply for the UAE_C2OperationModeManagement API.

### 6.1.3	Resources

There are no resources defined for this API in this release of the specification.

### 6.1.4	Custom Operations without associated resources

#### 6.1.4.1	Overview

The structure of the custom operation URIs of the UAE_C2OperationModeManagement API is shown in Figure 6.1.4.1-1.

Figure 6.1.4.1-1: Custom operation URI structure of the UAE_C2OperationModeManagement API

Table 6.1.4.1-1 provides an overview of the custom operations and applicable HTTP methods defined for the UAE_C2OperationModeManagement API.

Table 6.1.4.1-1: Custom operations without associated resources

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Initiate | /initiate | POST | Enables a UASS to request to provision C2 Operation Mode configuration information for a UAS (i.e. pair of UAV and UAV-C) to the UAE Server. |

#### 6.1.4.2	Operation: Initiate

##### 6.1.4.2.1	Description

The custom operation enables a UASS to initiate the configuration of C2 operation modes for a UAS (i.e. pair of UAV and UAV-C) by communicating the associated C2 Operation Mode configuration information to the UAE Server.

##### 6.1.4.2.2	Operation Definition

This operation shall support the request data structures and the response data structures and response codes specified in tables 6.1.4.2.2-1 and 6.1.4.2.2-2.

Table 6.1.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ConfigureData | M | 1 | Contains the parameters to request to provision C2 Operation Mode configuration information for a UAS (i.e. pair of UAV and UAV-C). |

Table 6.1.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| C2Result | M | 1 | 200 OK | The communicated C2 Operation Mode configuration information was successfully received. The response body shall contain the feedback of the UAE Server on whether this C2 Operation Mode configuration request is confirmed (i.e. can be undertaken by the UAE Server) or not. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative UAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative UAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2] |
| NOTE: The manadatory HTTP error status code for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.1.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI located in an alternative UAE Server. |

Table 6.1.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI located in an alternative UAE Server. |

### 6.1.5	Notifications

#### 6.1.5.1	General

Notifications shall comply to clause 5.2.5 of 3GPP TS 29.122 [2].

Table 6.1.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| C2 Operation Mode Management Completion Notification | {notificationUri}/c2mode-mngt-completion | c2mode-mngt-completion (POST) | This service operation enables a UAE Server to notify a previously subscribed UASS on the C2 operation mode management completion status for the concerned UAS (i.e. pair of UAV and UAV-C). |
| Selected C2 Communication Mode Notification | {notificationUri}/inform-selec-c2mode | inform-selec-c2mode (POST) | This service operation enables a UAE Server to notify a previously subscribed UASS on the C2 communication mode selected by the concerned UAS (i.e. pair of UAV and UAV-C). |
| C2 Communication Mode Switching Notification | {notificationUri}/inform-c2mode-switch | inform-c2mode-switch (POST) | This service operation enables a UAE Server to notify a previously subscribed UASS when C2 communication mode switching is carried out for the concerned UAS (i.e. pair of UAV and UAV-C) and possibly request confirmation from the UASS. |

#### 6.1.5.2	C2 Operation Mode Management Completion Notification

##### 6.1.5.2.1	Description

The C2 Operation Mode Management Completion Notification is used by a UAE Server to notify a previously subscribed UASS on the C2 operation mode management completion status for a UAS (i.e. pair of UAV and UAV-C).

##### 6.1.5.2.2	Target URI

The Callback URI "{notificationUri}/c2mode-mngt-completion" shall be used with the callback URI variables defined in table 6.1.5.2.2-1.

Table 6.1.5.2.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notificationUri | Uri | String formatted as a URI containing the Callback URI. |

##### 6.1.5.2.3	Standard Methods

###### 6.1.5.2.3.1	POST

This method shall support the request data structures specified in table 6.1.5.2.3.1-1 and the response data structures and response codes specified in table 6.1.5.2.3.1-2.

Table 6.1.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| C2OpModeMngtCompStatus | M | 1 | Contains the C2 operation mode management completion status for the concerned UAS (i.e. pair of UAV and UAV-C). |

Table 6.1.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The C2 operation mode management completion status for the concerned UAS (i.e. pair of UAV and UAV-C) is successfully received and acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative UASS where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative UASS where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.1.5.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative UASS towards which the notification should be redirected. |

Table 6.1.5.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative UASS towards which the notification should be redirected. |

#### 6.1.5.3	Selected C2 Communication Mode Notification

##### 6.1.5.3.1	Description

The Selected C2 Communication Mode Notification is used by a UAE Server to notify a previously subscribed UASS on the C2 communication mode selected by a UAS (i.e. pair of UAV and UAV-C).

##### 6.1.5.3.2	Target URI

The Callback URI "{notificationUri}/inform-selec-c2mode" shall be used with the callback URI variables defined in table 6.1.5.3.2-1.

Table 6.1.5.3.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notificationUri | Uri | String formatted as a URI containing the Callback URI. |

##### 6.1.5.3.3	Standard Methods

###### 6.1.5.3.3.1	POST

This method shall support the request data structures specified in table 6.1.5.3.3.1-1 and the response data structures and response codes specified in table 6.1.5.3.3.1-2.

Table 6.1.5.3.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SelectedC2CommModeNotif | M | 1 | Contains information on the C2 Communication Mode selected by the concerned UAS (i.e. pair of UAV and UAV-C). |

Table 6.1.5.3.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The C2 Communication Mode selected by the concerned UAS (i.e. pair of UAV and UAV-C) is successfully received and acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative UASS where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative UASS where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.1.5.3.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative UASS towards which the notification should be redirected. |

Table 6.1.5.3.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative UASS towards which the notification should be redirected. |

#### 6.1.5.4	C2 Communication Mode Switching Notification

##### 6.1.5.4.1	Description

The C2 Communication Mode Switching Notification is used by a UAE Server to notify a previously subscribed UASS on the targeted C2 Communication Mode switching for a UAS (i.e. pair of UAV and UAV-C).

##### 6.1.5.4.2	Target URI

The Callback URI "{notificationUri}/inform-c2mode-switch" shall be used with the callback URI variables defined in table 6.1.5.4.2-1.

Table 6.1.5.4.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notificationUri | Uri | String formatted as a URI containing the Callback URI. |

##### 6.1.5.4.3	Standard Methods

###### 6.1.5.4.3.1	POST

This method shall support the request data structures specified in table 6.1.5.4.3.1-1 and the response data structures and response codes specified in table 6.1.5.4.3.1-2.

Table 6.1.5.4.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| C2CommModeSwitchNotif | M | 1 | Contains information on the targeted C2 Communication Mode switching for the concerned UAS (i.e. pair of UAV and UAV-C). |

Table 6.1.5.4.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| C2Result | M | 1 | 200 OK | The targeted C2 Communication Mode switching for the concerned UAS (i.e. pair of UAV and UAV-C) is successfully received. The response body shall contain the feedback of the UASS on whether this C2 Communication Mode switching is confirmed (i.e. validated) or not. |
| n/a |  |  | 204 No Content | The targeted C2 Communication Mode switching for the concerned UAS (i.e. pair of UAV and UAV-C) is successfully received and acknowledged, and the UASS does not need to confirm (i.e. validate) it. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative UASS where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative UASS where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.1.5.4.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative UASS towards which the notification should be redirected. |

Table 6.1.5.4.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative UASS towards which the notification should be redirected. |

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the UAE_C2OperationModeManagement API.

Table 6.1.6.1-1: UAE_C2OperationModeManagement API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| ConfigureData | 6.1.6.2.2 | Represents the parameters to request to provision C2 Operation Mode configuration information for a UAS (i.e. pair of UAV and UAV-C). |  |
| SelectedC2CommModeNotif | 6.1.6.2.3 | Represents information on the C2 Communication Mode slected by a UAS (i.e. pair of UAV and UAV-C). |  |
| C2CommModeSwitchNotif | 6.1.6.2.4 | Represents information on the targeted C2 Communication Mode switching for a UAS (i.e. pair of UAV and UAV-C). |  |
| C2LinkQualityThrlds | 6.1.6.2.11 | Represents the C2 link quality thresholds. |  |
| C2OpModeMngtCompStatus | 6.1.6.2.9 | Represents the C2 operation mode management completion status for a UAS (i.e. pair of UAV and UAV-C). |  |
| C2OpModeStatus | 6.1.6.3.6 | Represents the C2 operation mode management completion status. |  |
| C2Result | 6.1.6.2.5 | Represents the result of an action related to C2 of a UAS. |  |
| C2ServiceArea | 6.1.6.2.8 | Represents a C2 service area. |  |
| C2SwitchPolicies | 6.1.6.2.10 | Represents the C2 operation mode switching policies. |  |
| UasId | 6.1.6.2.6 | Represents the identifier of a UAS (i.e. pair of UAV and UAV-C). |  |
| UavId | 6.1.6.2.7 | Represents the identifier of a UAV (e.g. UAV, UAV-C). |  |
| C2CommMode | 6.1.6.3.3 | Represents the C2 Communication Modes. |  |
| C2CommModeSwitching | 6.1.6.3.4 | Represents the C2 Communication Mode Switching types. |  |
| C2SwitchingCause | 6.1.6.3.5 | Represents the C2 Communication Mode switching cause. |  |

Table 6.1.6.1-2 specifies data types re-used by the UAE_C2OperationModeManagement API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the UAE_C2OperationModeManagement API.

Table 6.1.6.1-2: UAE_C2OperationModeManagement API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ExternalGroupId | 3GPP TS 29.122 [2] | Represents an external group identifier. |  |
| GeographicArea | 3GPP TS 29.572 [8] | Represents a geographical area. |  |
| Gpsi | 3GPP TS 29.571 [7] | Represents a GPSI. |  |
| Ncgi | 3GPP TS 29.571 [7] | Represents an NCGI. |  |
| PacketLossRate | 3GPP TS 29.571 [7] | Represents the packet loss rate. |  |
| SupportedFeatures | 3GPP TS 29.571 [7] | Used to negotiate the applicability of the optional features. |  |
| Tai | 3GPP TS 29.571 [7] | Represents a tracking area identifier. |  |
| Uri | 3GPP TS 29.122 [2] | Represents a URI. |  |

#### 6.1.6.2	Structured data types

##### 6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.1.6.2.2	Type: ConfigureData

Table 6.1.6.2.2-1: Definition of type ConfigureData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| uassId | Uri | M | 1 | Contains the identity of the UASS communicating the C2 Operation Mode configuration information for a UAS (i.e. pair of UAV and UAV-C). It takes the form of a URI. |  |
| uasId | UasId | M | 1 | Contains the identity of the UAS (i.e. pair of UAV and UAV-C) to which the provided C2 Operation Mode configuration information is destined. This shall be either in the form of a UAS identifier (e.g. group ID) or a collection of individual identifiers (e.g. CAA level UAV ID, GPSI) of the UAV and UAV-C composing the UAS. |  |
| allowedC2CommModes | array(C2CommMode) | M | 1..N | Contains the allowed C2 communication modes for the UAS (i.e. pair of UAV and UAV-C) identified by the "uasId" attribute. |  |
| c2CommModeSwitchTypes | array(C2CommModeSwitching) | M | 1..N | Contains the C2 Communication Mode switching types to be supported by the UAE Server for the UAS (i.e. pair of UAV and UAV-C) identified by the "uasId" attribute. The possible switching types are: - from "Direct C2 Communication" to "Network-Assisted C2 Communication"; - from "Network-Assisted C2 Communication" to "Direct C2 Communication"; - from "Direct C2 Communication" to "UTM-Navigated C2 Communication"; and/or - from "Network-Assisted C2 Communication" to "UTM-Navigated C2 Communication". |  |
| notificationUri | Uri | M | 1 | Contains the notification URI via which the UASS desires to receive notifications from the UAE Server. |  |
| primaryC2CommMode | C2CommMode | M | 1 | Contains the primary C2 communication mode to be used by the UAS (i.e. pair of UAV and UAV-C) identified by the "uasId" attribute. It shall be set to either "DIRECT_C2_COMMUNICATION" or "NETWORK_ASSISTED_C2_COMMUNICATION". |  |
| secondaryC2CommMode | C2CommMode | O | 0..1 | Contains the secondary C2 communication mode to be used by the UAS (i.e. pair of UAV and UAV-C) identified by the "uasId" attribute. When provided, it shall be set to either "DIRECT_C2_COMMUNICATION" or "NETWORK_ASSISTED_C2_COMMUNICATION". |  |
| c2SwitchPolicies | C2SwitchPolicies | M | 1 | Contains the C2 operation mode switching policies. |  |
| c2ServiceArea | C2ServiceArea | O | 0..1 | Contains the service area within which the C2 operation mode management request applies. This shall be either a geographical area or a topological area. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.1.8. This attribute shall be provided if at least one feature is supported by the UASS. |  |

##### 6.1.6.2.3	Type: SelectedC2CommModeNotif

Table 6.1.6.2.3-1: Definition of type SelectedC2CommModeNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| uasId | UasId | M | 1 | Contains the identity of the UAS (i.e. pair of UAV and UAV-C) to which the notification is related. This shall be either in form of a UAS identifier (e.g. group ID) or a collection of individual identifiers (e.g. CAA level UAV ID, GPSI) of the UAV and UAV-C composing the UAS. |  |
| selPrimaryC2CommMode | C2CommMode | M | 1 | Contains the primary C2 communication mode selected by the UAS (i.e. pair of UAV and UAV-C) identified by the "uasId" attribute. It shall be set to either "DIRECT_C2_COMMUNICATION" or "NETWORK_ASSISTED_C2_COMMUNICATION". |  |
| selSecondaryC2CommMode | C2CommMode | O | 0..1 | Contains the secondary C2 communication mode to be used by the UAS (i.e. pair of UAV and UAV-C) identified by the "uasId" attribute. When provided, it shall be set to either "DIRECT_C2_COMMUNICATION" or "NETWORK_ASSISTED_C2_COMMUNICATION". |  |

##### 6.1.6.2.4	Type: C2CommModeSwitchNotif

Table 6.1.6.2.4-1: Definition of type C2CommModeSwitchNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| uaeServerId | Uri | M | 1 | Contains the identifier of the UAE Server that is sending the notification and requesting C2 Communication Mode switching confirmation for a UAS (i.e. pair of UAV and UAV-C) from the UASS. |  |
| uasId | UasId | M | 1 | Contains the identifier of the UAS (i.e. pair of UAV and UAV-C) to which the provided C2 Communication Mode switching information is related. This shall be either in form of a UAS identifier (e.g. group ID) or a collection of individual identifiers (e.g. CAA level UAV ID, GPSI) of the UAV and UAV-C composing the UAS. |  |
| c2CommModeSwitchType | C2CommModeSwitching | M | 1 | Contains the targeted C2 Communication Mode switching for the UAS (i.e. pair of UAV and UAV-C) identified by the "uasId" attribute. |  |
| switchingCause | C2SwitchingCause | O | 0..1 | Contains the cause that triggers the C2 Communication Mode switching. |  |

##### 6.1.6.2.5	Type: C2Result

Table 6.1.6.2.5-1: Definition of type C2Result

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| c2OpConfirmed | Boolean | M | 1 | This attribute indicates whether the requested action (e.g. targeted C2 Communication Mode switching, C2 Operation Mode configuration information provisioning) is confirmed or not. - "true" means that the requested action is confirmed or approved. - "false" means that the requested action is not confirmed or not approved. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Indicates the list of negotiated supported features. This attribute shall be provided by the UAE Server in the response to a request in which the UASS provided the list of features that it supports. |  |

##### 6.1.6.2.6	Type: UasId

Table 6.1.6.2.6-1: Definition of type UasId

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| groupId | ExternalGroupId | C | 0..1 | Contains the identity of a UAS (i.e. a pair of UAV and UAV-C) in the form of a group identifier. (NOTE) |  |
| individualUasId | array(UavId) | C | 0..N | Contains the identity of a UAS (i.e. a pair of UAV and UAV-C) in the form of a collection of individual identifiers of the UAV and UAV-C composing the UAS. (NOTE) |  |
| NOTE: The "groupId" attribute and the "individualUasId" attribute are mutually exclusive. |  |  |  |  |  |

##### 6.1.6.2.7	Type: UavId

Table 6.1.6.2.7-1: Definition of type UavId

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| gpsi | Gpsi | C | 0..1 | Contains the identity of a UAV or UAV-C in the form of a GPSI. (NOTE) |  |
| caaId | string | C | 0..1 | Contains the identity of a UAV or UAV-C in the form of a CAA level UAV ID. (NOTE) |  |
| NOTE: At least one of the "groupId" attribute or the "caaId" attribute shall be provided within the UavId data type. |  |  |  |  |  |

##### 6.1.6.2.8	Type: C2ServiceArea

Table 6.1.6.2.8-1: Definition of type C2ServiceArea

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ncgiList | array(Ncgi) | C | 0..N | Contains a list of NR cell identifier(s) that constitutes the C2 service area. |  |
| taiList | array(Tai) | C | 0..N | Contains a list of tracking area identifier(s) that constitutes the C2 service area. |  |
| geographicAreaList | array(GeographicArea) | C | 0..N | Contains a list of geographic area(s) that constitutes the C2 service area. |  |
| NOTE: Either the "geographicAreaList" attribute or the "ncgiList" attribute and/or the "taiList" attribute shall be provided. |  |  |  |  |  |

##### 6.1.6.2.9	Type: C2OpModeMngtCompStatus

Table 6.1.6.2.9-1: Definition of type C2OpModeMngtCompStatus

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| uasId | UasId | M | 1 | Contains the identifier of the UAS (i.e. pair of UAV and UAV-C) to which the provided C2 operation mode management completion status information is related. |  |
| status | C2OpModeStatus | M | 1 | Contains the C2 operation mode management completion status. |  |

##### 6.1.6.2.10	Type: C2SwitchPolicies

Table 6.1.6.2.10-1: Definition of type C2SwitchPolicies

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| directC2LinkQualityThrlds | C2LinkQualityThrlds | O | 0..1 | Contains the threshold(s) used to evaluate the quality of the direct C2 link. |  |
| uuC2LinkQualityThrlds | C2LinkQualityThrlds | O | 0..1 | Contains the threshold(s) used to evaluate the quality of the Network-Assisted (i.e. Uu based) C2 link. |  |
| NOTE: Either the "directC2LinkQualityThrlds" attribute, the "uuC2LinkQualityThrlds" attribute or both shall be provided. |  |  |  |  |  |

##### 6.1.6.2.11	Type: C2LinkQualityThrlds

Table 6.1.6.2.11-1: Definition of type C2LinkQualityThrlds

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| nrRsrpThrldLow | integer | O | 0..1 | Represents the lower RSRP value threshold for the direct C2 link. Value range: 0-127. (NOTE 1) |  |
| nrRsrpThrldHigh | integer | O | 0..1 | Represents the upper RSRP value threshold for the direct C2 link. Value range: 0-127. (NOTE 2) |  |
| nrRsrqThrldLow | integer | O | 0..1 | Represents the lower RSRQ value threshold for the direct C2 link. Value range: 0-127. (NOTE 1) |  |
| nrRsrqThrldHigh | integer | O | 0..1 | Represents the upper RSRQ value threshold for the direct C2 link. Value range: 0-127. (NOTE 2) |  |
| packetLossThrldLow | PacketLossRate | O | 0..1 | Represents the lower packet loss rate value threshold for the direct C2 link. (NOTE 1) |  |
| packetLossThrldHigh | PacketLossRate | O | 0..1 | Represents the upper packet loss rate value threshold for the direct C2 link. (NOTE 2) |  |
| NOTE 1: At least one of the "nrRsrpThrldLow", "nrRsrqThrldLow" or "packetLossThrldLow" attributes shall be provided. NOTE 2: At least one of the "nrRsrpThrldHigh", "nrRsrqThrldHigh" or "packetLossThrldHigh" attributes shall be provided. |  |  |  |  |  |

#### 6.1.6.3	Simple data types and enumerations

##### 6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 6.1.6.3.3	Enumeration: C2CommMode

The enumeration C2CommMode represents C2 Communication Modes. It shall comply with the provisions of table 6.1.6.3.3-1.

Table 6.1.6.3.3-1: Enumeration C2CommMode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| DIRECT_C2_COMMUNICATION | Represents Direct C2 Communication mode. |  |
| NETWORK_ASSISTED_C2_COMMUNICATION | Represents Network-Assisted C2 Communication mode. |  |
| UTM_NAVIGATED_C2_COMMUNICATION | Represents UTM-Navigated C2 communication mode. |  |

##### 6.1.6.3.4	Enumeration: C2CommModeSwitching

The enumeration C2CommModeSwitching represents C2 Communication Mode Switching types. It shall comply with the provisions of table 6.1.6.3.4-1.

Table 6.1.6.3.4-1: Enumeration C2CommModeSwitching

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| DIRECT_TO_NETWORK_ASSISTED_C2 | Represents the C2 Communication Mode switching from Direct C2 Communication mode to Network-Assisted C2 Communication mode. |  |
| NETWORK_ASSISTED_TO_DIRECT_C2 | Represents the C2 Communication Mode switching from Network-Assisted C2 Communication mode to Direct C2 Communication mode. |  |
| DIRECT_TO_UTM_NAVIGATED_C2 | Represents the C2 Communication Mode switching from Direct C2 Communication mode to UTM-Navigated C2 communication mode. |  |
| NETWORK_ASSISTED_TO_UTM_NAVIGATED_C2 | Represents the C2 Communication Mode switching from Network-Assisted C2 Communication mode to UTM-Navigated C2 communication mode. |  |

##### 6.1.6.3.5	Enumeration: C2SwitchingCause

The enumeration C2SwitchingCause represents the C2 Communication Mode switching cause. It shall comply with the provisions of table 6.1.6.3.5-1.

Table 6.1.6.3.5-1: Enumeration C2SwitchingCause

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| DIRECT_LINK_QUALITY_DEGRADATION | Indicates that the C2 Communication Mode switching was triggered due to a degradation in the direct radio link quality. |  |
| DIRECT_LINK_AVAILABLE | Indicates that the C2 Communication Mode switching was triggered due to the availability of a direct link, i.e. direct radio link quality enables its usage. |  |
| MOVING_BVLOS | Indicates that the C2 Communication Mode switching was triggered due to the UAV moving BVLOS. |  |
| LOCATION_CHANGE | Indicates that the C2 Communication Mode switching was triggered due to an actual or expected location/mobility of the UAV (e.g. which impacts the UAV-to-UAV-C location). |  |
| TRAFFIC_CONTROL_NEEDED | Indicates that the C2 Communication Mode switching was triggered due to the necessity to have air traffic control. |  |
| SECURITY_REASONS | Indicates that the C2 Communication Mode switching was triggered due to security reasons. |  |
| OTHER_REASONS | Indicates that the C2 Communication Mode switching was triggered due to other reasons (e.g. unpredictable event, unknown reason, weather conditions, topography, etc.). |  |

##### 6.1.6.3.6	Enumeration: C2OpModeStatus

The enumeration C2OpModeStatus represents C2 Operation Mode Management Completion status. It shall comply with the provisions of table 6.1.6.3.6-1.

Table 6.1.6.3.6-1: Enumeration C2CommMode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SUCCESSFUL | Indicates that the C2 operation mode configuration was successful. |  |
| NOT_SUCCESSFUL | Indicates that the C2 operation mode configuration was not successful. |  |

#### 6.1.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

#### 6.1.6.5	Binary data

##### 6.1.6.5.1	Binary Data Types

Table 6.1.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

### 6.1.7	Error Handling

#### 6.1.7.1	General

For the UAE_C2OperationModeManagement API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [2]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [2] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [2].

In addition, the requirements in the following clauses are applicable for the UAE_C2OperationModeManagement API.

#### 6.1.7.2	Protocol Errors

No specific protocol errors for the UAE_C2OperationModeManagement API are specified.

#### 6.1.7.3	Application Errors

The application errors defined for the UAE_C2OperationModeManagement API are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

### 6.1.8	Feature negotiation

The optional features listed in table 6.1.8-1 are defined for the UAE_C2OperationModeManagement API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [2].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.1.9	Security

The provisions of clause 6 of 3GPP TS 29.122 [2] shall apply for the UAE_C2OperationModeManagement API.

## 6.2	UAE_RealtimeUAVStatus Service API

### 6.2.1	Introduction

The UAE_RealtimeUAVStatus service shall use the UAE_RealtimeUAVStatus API.

The API URI of the UAE_RealtimeUAVStatus API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [2], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [2].

-	The <apiName> shall be "uae-uav-status".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [2].

### 6.2.2	Usage of HTTP

The provisions of clause 5.2.2 of 3GPP TS 29.122 [2] shall apply for the UAE_RealtimeUAVStatus API.

### 6.2.3	Resources

#### 6.2.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.2.3.1-1 depicts the resource URIs structure for the UAE_RealtimeUAVStatus API.

Figure 6.2.3.1-1: Resource URIs structure of the UAE_RealtimeUAVStatus API

Table 6.2.3.1-1 provides an overview of the resources and applicable HTTP methods for the UAE_RealtimeUAVStatus API.

Table 6.2.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Real-time UAV Status Subscriptions | /subscriptions | GET | Retrieve all the active real-time UAV status subscriptions managed by the UAE Server. |
|  |  | POST | Request the creation of a subscription to real-time UAV status reporting. |
| Individual Real-time UAV Status Subscription | /subscriptions/{subscriptionId} | GET | Retrieve a real-time UAV status subscription resource identified by the provided subscription identifier. |
|  |  | PUT | Update an existing real-time UAV status subscription resource identified by the provided subscription identifier. |
|  |  | DELETE | Request the deletion of a real-time UAV status subscription resource identified by the provided subscription identifier. |

#### 6.2.3.2	Resource: Real-time UAV Status Subscriptions

##### 6.2.3.2.1	Description

This resource represents the collection of real-time UAV status subscriptions managed by the UAE Server.

##### 6.2.3.2.2	Resource Definition

Resource URI: {apiRoot}/uae-uav-status/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 6.2.3.2.2-1.

Table 6.2.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.2.4 of 3GPP TS 29.122 [2]. |

##### 6.2.3.2.3	Resource Standard Methods

The following clauses specify the standard methods supported by the resource.

###### 6.2.3.2.3.1	GET

The GET method allows a UASS to retrieve all the active real-time UAV status subscriptions managed by the UAE Server.This method shall support the URI query parameters specified in table 6.2.3.2.3.1-1.

Table 6.2.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.2.3.1-2 and the response data structures and response codes specified in table 6.2.3.2.3.1-3.

Table 6.2.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(RTUavStatusSubsc) | M | 1..N | 200 OK | Successful case. All the active real-time UAV status subscriptions managed by the UAE Server shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative UAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative UAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.2.3.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative UAE Server. |

Table 6.2.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative UAE Server. |

###### 6.2.3.2.3.2	POST

The POST method allows a UASS to request the creation of a subscription to real-time UAV status reporting at the UAE Server. This method shall support the URI query parameters specified in table 6.2.3.2.3.2-1.

Table 6.2.3.2.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.2.3.2-2 and the response data structures and response codes specified in table 6.2.3.2.3.2-3.

Table 6.2.3.2.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| RTUavStatusSubsc | M | 1 | Represents the parameters to request the creation of a subscription to real-time UAV status reporting. |

Table 6.2.3.2.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| RTUavStatusSubsc | M | 1 | 201 Created | Successful case. The subscription is successfully created and a representation of the created Individual Real-time UAV Status Subscription resource shall be returned. An HTTP "Location" header that contains the resource URI of the created Individual Real-time UAV Status Subscription resource shall also be included. |
| NOTE: The manadatory HTTP error status code for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.2.3.2.3.2-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/uae-uav-status/<apiVersion>/subscriptions/{subscriptionId} |

##### 6.2.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.2.3.3	Resource: Individual Real-time UAV Status Subscription

##### 6.2.3.3.1	Description

This resource represents an individual real-time UAV status subscription managed by the UAE Server.

##### 6.2.3.3.2	Resource Definition

Resource URI: {apiRoot}/uae-uav-status/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.2.3.3.2-1.

Table 6.2.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.2.4 of 3GPP TS 29.122 [2]. |
| subscriptionId | string | Represents the subscription identifier. |

##### 6.2.3.3.3	Resource Standard Methods

The following clauses specify the standard methods supported by the resource.

###### 6.2.3.3.3.1	GET

The GET method allows a UASS to retrieve a real-time UAV status subscription identified by the subscription identifier included in the request URI (i.e. within the "/{subscriptionId}" path segment).This method shall support the URI query parameters specified in table 6.2.3.3.3.1-1.

Table 6.2.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.1-2 and the response data structures and response codes specified in table 6.2.3.3.3.1-3.

Table 6.2.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| RTUavStatusSubsc | M | 1 | 200 OK | Successful case. The requested Individual Real-time UAV Status Subscription resource shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative UAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative UAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.2.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative UAE Server. |

Table 6.2.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative UAE Server. |

###### 6.2.3.3.3.2	PUT

The PUT method allows a UASS to request the update of an existing real-time UAV status subscription identified by the subscription identifier included in the request URI (i.e. within the "/{subscriptionId}" path segment). This method shall support the URI query parameters specified in table 6.2.3.3.3.2-1.

Table 6.2.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.2-2 and the response data structures and response codes specified in table 6.2.3.3.3.2-3.

Table 6.2.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| RTUavStatusSubsc | M | 1 | Represents the parameters to request the update of an existing subscription to real-time UAV status reporting. |

Table 6.2.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| RTUavStatusSubsc | M | 1 | 200 OK | Successful case. The real-time UAV status subscription is successfully updated and a representation of the updated Individual Real-time UAV Status Subscription resource shall be returned. |
| n/a |  |  | 204 No Content | Successful case. The real-time UAV status subscription is successfully updated. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative UAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative UAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.2.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative UAE Server. |

Table 6.2.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative UAE Server. |

###### 6.2.3.3.3.3	DELETE

The DELETE method allows a UASS to request the deletion of an existing real-time UAV status subscription identified by the subscription identifier included in the request URI (i.e. within the "/{subscriptionId}" path segment). This method shall support the URI query parameters specified in table 6.2.3.3.3.3-1.

Table 6.2.3.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.3-2 and the response data structures and response codes specified in table 6.2.3.3.3.3-3.

Table 6.2.3.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The real-time UAV status subscription is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative UAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative UAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.2.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative UAE Server. |

Table 6.2.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative UAE Server. |

##### 6.2.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

### 6.2.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 6.2.5	Notifications

#### 6.2.5.1	General

Notifications shall comply to clause 5.2.5 of 3GPP TS 29.122 [2].

Table 6.2.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Real-time UAV Status Notification | {notificationUri}/uav-status | uav-status (POST) | This service operation enables a UAE Server to notify a previously subscribed UASS on the real-time UAV status information. |

#### 6.2.5.2	Real-time UAV Status Notification

##### 6.2.5.2.1	Description

The Real-time UAV Status Notification is used by a UAE Server to notify a previously subscribed UASS on the real-time UAV status information.

##### 6.2.5.2.2	Target URI

The Callback URI "{notificationUri}/uav-status" shall be used with the callback URI variables defined in table 6.2.5.2.2-1.

Table 6.2.5.2.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notificationUri | Uri | String formatted as a URI containing the Callback URI. |

##### 6.2.5.2.3	Standard Methods

###### 6.2.5.2.3.1	POST

This method shall support the request data structures specified in table 6.2.5.2.3.1-1 and the response data structures and response codes specified in table 6.2.5.2.3.1-2.

Table 6.2.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| RTUavStatusNotif | M | 1 | Represents a real-time UAV status notification. |

Table 6.2.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The real-time UAV status notification is successfully received and acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative UASS where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative UASS where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.2.5.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative UASS towards which the notification should be redirected. |

Table 6.2.5.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative UASS towards which the notification should be redirected. |

### 6.2.6	Data Model

#### 6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.6.1-1 specifies the data types defined for the UAE_RealtimeUAVStatus API.

Table 6.2.6.1-1: UAE_RealtimeUAVStatus API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| RTUavStatusSubsc | 6.2.6.2.2 | Represents the parameters to request the creation of a subscription to real-time UAV status reporting. |  |
| RTUavStatusNotif | 6.2.6.2.3 | Represents a real-time UAV status notification. |  |
| RTUavStatus | 6.2.6.2.4 | Represents real-time UAV status information. |  |
| UavNetConnStatus | 6.2.6.2.5 | Represents the UAV network connection status information. |  |

Table 6.2.6.1-2 specifies data types re-used by the UAE_RealtimeUAVStatus API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the UAE_RealtimeUAVStatus API.

Table 6.2.6.1-2: UAE_RealtimeUAVStatus API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTime | 3GPP TS 29.122 [2] | Represents a date and a time. |  |
| MonitoringType | 3GPP TS 29.122 [2] | Represents a monitoring event type. |  |
| LocationInfo | 3GPP TS 29.122 [2] | Represents user location information. |  |
| SupportedFeatures | 3GPP TS 29.571 [7] | Used to negotiate the applicability of the optional features. |  |
| UavId | Clause 6.1.6.2.7 | Represents a UAV identifier. |  |
| Uri | 3GPP TS 29.122 [2] | Represents a URI. |  |

#### 6.2.6.2	Structured data types

##### 6.2.6.2.1	Introduction

This clause defines the data structures to be used in resource representations.

##### 6.2.6.2.2	Type: RTUavStatusSubsc

Table 6.2.6.2.2-1: Definition of type RTUavStatusSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| uassId | Uri | M | 1 | Contains the identity of the UASS that is sending the request. It takes the form of a URI. |  |
| uavIds | array(UavId) | M | 1..N | Contains the identity of the UAV(s) to which the real-time UAV status subscription is related. |  |
| notificationUri | Uri | M | 1 | Contains the notification URI via which the UASS desires to receive real-time UAV status notifications from the UAE Server. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.2.8. This attribute shall be provided in the HTTP POST request for subscription resource creation and in the associated successful response. |  |

##### 6.2.6.2.3	Type: RTUavStatusNotif

Table 6.2.6.2.3-1: Definition of type RTUavStatusNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subscriptionId | string | M | 1 | Contains the identifier of the Individual Real-time UAV Status Subscription to which the notification is related. |  |
| rTUavStatus | array(RTUavStatus) | M | 1..N | Contains the real-time UAV status information for a UAV. |  |

##### 6.2.6.2.4	Type: RTUavStatus

Table 6.2.6.2.4-1: Definition of type RTUavStatus

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| uavId | UavId | M | 1 | Contains the identity of the UAV to which the real-time UAV status information is related. |  |
| uavNetConnStatus | UavNetConnStatus | C | 0..1 | Contains the network connection status information for the UAV. (NOTE) |  |
| uavLocInfo | LocationInfo | M | 0..1 | Contains the location information for the UAV. (NOTE) |  |
| NOTE: Either only the "uavLocInfo" attribute or both the "uavNetConnStatus" attribute and the "uavLocInfo" attribute shall be present. |  |  |  |  |  |

##### 6.2.6.2.5	Type: UavNetConnStatus

Table 6.2.6.2.5-1: Definition of type UavNetConnStatus

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| statusInfo | MonitoringType | M | 1 | Contains the network connection status monitoring event that occurred. Only the "LOSS_OF_CONNECTIVITY", "UE_REACHABILITY", "COMMUNICATION_FAILURE" and "PDN_CONNECTIVITY_STATUS" values are applicable. |  |
| timestamp | DateTime | M | 1 | Contains the timestamp of the provided network connection status information. |  |

#### 6.2.6.3	Simple data types and enumerations

##### 6.2.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.2.6.3.2	Simple data types

The simple data types defined in table 6.2.6.3.2-1 shall be supported.

Table 6.2.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

#### 6.2.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

#### 6.2.6.5	Binary data

##### 6.2.6.5.1	Binary Data Types

Table 6.2.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

### 6.2.7	Error Handling

#### 6.2.7.1	General

For the UAE_RealtimeUAVStatus API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [2]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [2] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [2].

In addition, the requirements in the following clauses are applicable for the UAE_RealtimeUAVStatus API.

#### 6.2.7.2	Protocol Errors

No specific protocol errors for the UAE_RealtimeUAVStatus API are specified.

#### 6.2.7.3	Application Errors

The application errors defined for the UAE_RealtimeUAVStatus API are listed in Table 6.2.7.3-1.

Table 6.2.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

### 6.2.8	Feature negotiation

The optional features listed in table 6.2.8-1 are defined for the UAE_RealtimeUAVStatus API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [2].

Table 6.2.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.2.9	Security

The provisions of clause 6 of 3GPP TS 29.122 [2] shall apply for the UAE_RealtimeUAVStatus API.

# 7	Using Common API Framework

## 7.1	General

When CAPIF is used with a UAE Server service, the UAE Server shall support the following functionalities as defined in 3GPP TS 29.222 [10]:

-	the API exposing function and the related APIs over CAPIF-2/2e and CAPIF-3/3e reference points;

-	the API publishing function and the related APIs over CAPIF-4/4e reference point;

-	the API management function and the related APIs over CAPIF-5/5e reference point; and

-	at least one of the security methods for authentication and authorization, and the related security mechanisms.

In a centralized deployment as defined in 3GPP TS 23.222 [9], where the CAPIF core function and the API provider domain functions are co-located, the interactions between the CAPIF core function and the API provider domain functions may be independent of the CAPIF-3/3e, CAPIF-4/4e and CAPIF-5/5e reference points.

When CAPIF is used with a UAE Server service, the UAE Server shall register all the northbound APIs features in the CAPIF Core Function.

## 7.2	Security

When CAPIF is used for external exposure, before invoking an API exposed by the UAE Server, the service API consumer (e.g. UASS) acting as an API invoker shall negotiate the security method (PKI, TLS-PSK or OAuth 2.0) with the CAPIF core function and ensure that the UAE Server has enough credentials to authenticate the service API consumer (e.g. UASS), as defined in clauses 5.6.2.2 and 6.2.2.2 of 3GPP TS 29.222 [10].

If PKI or TLS-PSK is selected as the security method to be used between the service API consumer (e.g. UASS) and the UAE Server, upon API invocation, the UAE Server shall retrieve the authorization information from the CAPIF core function as described in clause 5.6.2.4 of 3GPP TS 29.222 [10].

As indicated in 3GPP TS 33.122 [11], the access to the UAE Server APIs may be authorized by means of the OAuth 2.0 protocol (see IETF RFC 6749 [12]), using the "Client Credentials" authorization grant, where the CAPIF core function (see 3GPP TS 29.222 [10]) plays the role of the authorization server.

NOTE 1:	In this release, only "Client Credentials" authorization grant is supported.

If OAuth 2.0 is selected as the security method to be used between the service API consumer (e.g. UASS) and the UAE Server, the service API consumer (e.g. UASS) shall, prior to consuming the services offered by the UAE Server APIs, obtain a "token" from the authorization server, by invoking the Obtain_Authorization service operation as described in clause 5.6.2.3.2 of 3GPP TS 29.222 [10].

The UAE Server APIs do not define any scopes for OAuth 2.0 authorization. It is the UAE Server responsibility to check whether the service API consumer (e.g. UASS) is authorized to use an API based on the provided "token". Once the UAE Server verifies the "token", it shall check whether the UAE Server identifier in the "token" matches its own published identifier, and whether the API name in the "token" matches its own published API name. If those checks are passed, the service API consumer (e.g. UASS) has full authority to access any resource or operation provided by the invoked API.

NOTE 2:	For the aforementioned security methods, the UAE Server needs to apply admission control according to access control policies after performing the authorization checks.

Annex A (normative):
OpenAPI specification

## A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see clause 5.3.1 of 3GPP TS 29.501 [3] and clause 5B of 3GPP TR 21.900 [5]).

## A.2	UAE_C2OperationModeManagement API

openapi: 3.0.0

info:

title: UAE Server C2 Operation Mode Management Service

version: 1.0.0-alpha.4

description: |

UAE Server C2 Operation Mode Management Service.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.257 V1.2.0; Application layer support for Uncrewed Aerial System (UAS);

UAS Application Enabler (UAE) Server Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.257/

servers:

- url: '{apiRoot}/uae-c2opmode-mngt/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

/initiate:

post:

summary: Request the provisioning of C2 Operation Mode configuration information for a UAS (i.e. pair of UAV and UAV-C).

operationId: InitiateC2OpModeConfig

tags:

- Initiate C2 Operation Mode configuration

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ConfigureData'

responses:

'200':

description: >

The communicated C2 Operation Mode configuration information was successfully received.

The response body contains the feedback of the UAE Server on whether this C2 Operation

Mode configuration request is confirmed (i.e. can be undertaken by the UAE Server) or

not.

content:

application/json:

schema:

$ref: '#/components/schemas/C2Result'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

C2OpModeMngtCompletionNotification:

'{$request.body#/notificationUri}/c2mode-mngt-completion':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/C2OpModeMngtCompStatus'

responses:

'204':

description: >

No Content. The notification was succesfull and the C2 Operation Mode

Management Completion status for the concerned UAS (i.e. pair of UAV

and UAV-C) was successfully received and acknowledged by the UASS.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

SelectedC2CommunicationModeNotification:

'{$request.body#/notificationUri}/inform-selec-c2mode':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SelectedC2CommModeNotif'

responses:

'204':

description: >

No Content. The notification was succesfull and the C2 Communication Mode

selected by the concerned UAS (i.e. pair of UAV and UAV-C) was successfully

received and acknowledged by the UASS.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

C2CommunicationModeSwitchingNotification:

'{$request.body#/notificationUri}/inform-c2mode-switch':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/C2CommModeSwitchNotif'

responses:

'200':

description: >

OK. The targeted C2 Communication Mode switching for the concerned UAS

(i.e. pair of UAV and UAV-C) is successfully received. The response body

contains the feedback of the UASS on whether this C2 Communication Mode

switching is confirmed (i.e. validated) or not.

content:

application/json:

schema:

$ref: '#/components/schemas/C2Result'

'204':

description: >

No Content. The targeted C2 Communication Mode switching for the concerned

UAS (i.e. pair of UAV and UAV-C) is successfully received and acknowledged,

and the UASS does not need to confirm (i.e. validate) it.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

ConfigureData:

description: >

Represents the parameters to request to provision C2 Operation Mode configuration

information for a UAS (i.e. pair of UAV and UAV-C).

type: object

properties:

uassId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

uasId:

$ref: '#/components/schemas/UasId'

allowedC2CommModes:

type: array

items:

$ref: '#/components/schemas/C2CommMode'

minItems: 1

c2CommModeSwitchTypes:

type: array

items:

$ref: '#/components/schemas/C2CommModeSwitching'

minItems: 1

notificationUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

primaryC2CommMode:

$ref: '#/components/schemas/C2CommMode'

secondaryC2CommMode:

$ref: '#/components/schemas/C2CommMode'

c2SwitchPolicies:

$ref: '#/components/schemas/C2SwitchPolicies'

c2ServiceArea:

$ref: '#/components/schemas/C2ServiceArea'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- uassId

- uasId

- allowedC2CommModes

- c2CommModeSwitchTypes

- notificationUri

- primaryC2CommMode

- c2SwitchPolicies

SelectedC2CommModeNotif:

description: >

Represents information on the C2 Communication Mode selected by a UAS (i.e. pair of

UAV and UAV-C).

type: object

properties:

uasId:

$ref: '#/components/schemas/UasId'

selPrimaryC2CommMode:

$ref: '#/components/schemas/C2CommMode'

selSecondaryC2CommMode:

$ref: '#/components/schemas/C2CommMode'

required:

- uasId

- selPrimaryC2CommMode

C2CommModeSwitchNotif:

description: >

Represents information on the targeted C2 Communication Mode switching for a UAS

(i.e. pair of UAV and UAV-C).

type: object

properties:

uaeServerId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

uasId:

$ref: '#/components/schemas/UasId'

c2CommModeSwitchType:

$ref: '#/components/schemas/C2CommModeSwitching'

switchingCause:

$ref: '#/components/schemas/C2SwitchingCause'

required:

- uaeServerId

- uasId

- c2CommModeSwitchType

C2Result:

description: Represents the result of an action related to C2 of a UAS.

type: object

properties:

c2OpConfirmed:

type: boolean

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- c2OpConfirmed

UasId:

description: Represents the identifier of a UAS (i.e. pair of UAV and UAV-C).

type: object

properties:

groupId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ExternalGroupId'

individualUasId:

type: array

items:

$ref: '#/components/schemas/UavId'

minItems: 2

oneOf:

- required: [groupId]

- required: [individualUasId]

UavId:

description: Represents the identifier of a UAV (e.g. UAV, UAV-C).

type: object

properties:

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

caaId:

type: string

anyOf:

- required: [gpsi]

- required: [caaId]

C2ServiceArea:

description: Represents a C2 service area.

type: object

properties:

ncgiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

taiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

geographicAreaList:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

oneOf:

- required: [geographicAreaList]

- anyOf:

- required: [ncgiList]

- required: [taiList]

C2OpModeMngtCompStatus:

description: >

Represents the C2 Operation Mode Management Completion status for a UAV

(e.g. UAV, UAV-C).

type: object

properties:

uasId:

$ref: '#/components/schemas/UasId'

status:

$ref: '#/components/schemas/C2OpModeStatus'

required:

- status

C2SwitchPolicies:

description: Represents the C2 operation mode switching policies.

type: object

properties:

directC2LinkQualityThrlds:

$ref: '#/components/schemas/C2LinkQualityThrlds'

uuC2LinkQualityThrlds:

$ref: '#/components/schemas/C2LinkQualityThrlds'

C2LinkQualityThrlds:

description: Represents the C2 link quality thresholds.

type: object

properties:

nrRsrpThrldLow:

type: integer

minimum: 0

maximum: 127

nrRsrpThrldHigh:

type: integer

minimum: 0

maximum: 127

nrRsrqThrldLow:

type: integer

minimum: 0

maximum: 127

nrRsrqThrldHigh:

type: integer

minimum: 0

maximum: 127

packetLossThrldLow:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

packetLossThrldHigh:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

# ENUMS:

C2CommMode:

description: Represents the C2 Communication Modes.

anyOf:

- type: string

enum:

- DIRECT_C2_COMMUNICATION

- NETWORK_ASSISTED_C2_COMMUNICATION

- UTM_NAVIGATED_C2_COMMUNICATION

- type: string

C2CommModeSwitching:

description: Represents the C2 Communication Mode Switching types.

anyOf:

- type: string

enum:

- DIRECT_TO_NETWORK_ASSISTED_C2

- NETWORK_ASSISTED_TO_DIRECT_C2

- DIRECT_TO_UTM_NAVIGATED_C2

- NETWORK_ASSISTED_TO_UTM_NAVIGATED_C2

- type: string

C2SwitchingCause:

description: Represents the C2 Communication Mode Switching cause.

anyOf:

- type: string

enum:

- DIRECT_LINK_QUALITY_DEGRADATION

- DIRECT_LINK_AVAILABLE

- MOVING_BVLOS

- LOCATION_CHANGE

- TRAFFIC_CONTROL_NEEDED

- SECURITY_REASONS

- OTHER_REASONS

- type: string

C2OpModeStatus:

description: Represents the C2 Operation Mode Management Completion status.

anyOf:

- type: string

enum:

- SUCCESSFUL

- NOT_SUCCESSFUL

- type: string

## A.3	UAE_RealtimeUAVStatus API

openapi: 3.0.0

info:

title: UAE Server Real-time UAV Status Service

version: 1.0.0-alpha.3

description: |

UAE Server Real-time UAV Status Service.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.257 V1.2.0; Application layer support for Uncrewed Aerial System (UAS);

UAS Application Enabler (UAE) Server Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.257/

servers:

- url: '{apiRoot}/uae-uav-status/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

/subscriptions:

get:

summary: Retrieve all the active real-time UAV status subscriptions managed by the UAE Server.

operationId: GetRTUavStatusSubscriptions

tags:

- Real-time UAV Status Subscriptions (Collection)

responses:

'200':

description: >

OK. All the active real-time UAV status subscriptions managed by the UAE Server

shall be returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/RTUavStatusSubsc'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Request the creation of a subscription to real-time UAV status reporting.

operationId: CreateRTUavStatusSubsc

tags:

- Real-time UAV Status Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/RTUavStatusSubsc'

responses:

'200':

description: >

OK. The subscription is successfully created and a representation of the created

Individual Real-time UAV Status Subscription resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/RTUavStatusSubsc'

headers:

Location:

description: >

Contains the URI of the created Individual Real-time UAV Status Subscription

resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

RTUavStatusNotification:

'{$request.body#/notificationUri}/uav-status':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/RTUavStatusNotif'

responses:

'204':

description: >

No Content. The real-time UAV status notification is successfully

received and acknowledged.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

get:

summary: Retrieve a real-time UAV status subscription resource.

operationId: GetRTUavStatusSubscription

tags:

- Individual Real-time UAV Status Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Individual Real-time UAV Status Subscription identifier.

required: true

schema:

type: string

responses:

'200':

description: OK. The requested real-time UAV status subscription resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/RTUavStatusSubsc'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing real-time UAV status subscription.

operationId: UpdateRTUavStatusSubscription

tags:

- Individual Real-time UAV Status Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Individual Real-time UAV Status Subscription identifier.

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/RTUavStatusSubsc'

responses:

'200':

description: >

OK. The real-time UAV status subscription is successfully updated and a

representation of the updated Individual Real-time UAV Status Subscription

resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/RTUavStatusSubsc'

'204':

description: No Content. The real-time UAV status subscription is successfully updated.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing real-time UAV status subscription.

operationId: DeleteRTUavStatusSubscription

tags:

- Individual Real-time UAV Status Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Individual Real-time UAV Status Subscription identifier.

required: true

schema:

type: string

responses:

'204':

description: >

No Content. The Individual Real-time UAV Status Subscription resource

is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

RTUavStatusSubsc:

description: >

Represents the parameters to request the creation or update of a subscription

to real-time UAV status reporting.

type: object

properties:

uassId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

uavIds:

type: array

items:

$ref: 'TS29257_UAE_C2OperationModeManagement.yaml#/components/schemas/UavId'

minItems: 1

notificationUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- uassId

- uavIds

- notificationUri

RTUavStatusNotif:

description: Represents a real-time UAV status notification.

type: object

properties:

subscriptionId:

type: string

rTUavStatus:

type: array

items:

$ref: '#/components/schemas/RTUavStatus'

minItems: 1

required:

- subscriptionId

- rTUavStatus

RTUavStatus:

description: Represents real-time UAV status information.

type: object

properties:

uavId:

$ref: 'TS29257_UAE_C2OperationModeManagement.yaml#/components/schemas/UavId'

uavNetConnStatus:

$ref: '#/components/schemas/UavNetConnStatus'

uavLocInfo:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

allOf:

- required: [uavId]

- oneOf:

- required: [uavLocInfo]

- allOf:

- required: [uavLocInfo]

- required: [uavNetConnStatus]

UavNetConnStatus:

description: Represents UAV network connection status information.

type: object

properties:

statusInfo:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/MonitoringType'

timestamp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

required:

- statusInfo

- timestamp

Annex B (informative):
Withdrawn API versions

## B.1	General

This Annex lists withdrawn API versions of the APIs defined in the present specification. Clause 4.3.1.6 of 3GPP TS 29.501 [5] describes the withdrawal of API versions.

## B.2	UAE_C2OperationModeManagement API

The API versions listed in table B.2-1 are withdrawn for the UAE_C2OperationModeManagement API.

Table B.2-1: Withdrawn API versions of the UAE_C2OperationModeManagement service

| API version number | Remarks |
| --- | --- |

## B.3	UAE_RealtimeUAVStatus API

The API versions listed in table B.3-1 are withdrawn for the UAE_RealtimeUAVStatus API.

Table B.3-1: Withdrawn API versions of the UAE_RealtimeUAVStatus service

| API version number | Remarks |
| --- | --- |

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-05 | CT3#116-e |  | - | - | - | Skeleton for the new UASAPP TS | 0.0.0 |
| 2021-05 | CT3#116-e | C3-213168 |  |  |  | Inclusion of C3-213539 | 0.0.0 |
| 2021-09 | CT3#117-e | C3-213503 | - | - | - | Inclusion of: C3-214294, C3-214295, C3-214296, C3-214297, C3-214487, C3-214299, C3-214300, C3-214488, C3-214489 | 0.1.0 |
| 2021-10 | CT3#118-e | C3-214619 |  |  |  | Inclusion of: C3-215442, C3-215443, C3-215444, C3-215445, C3-215446, C3-215447, C3-215448, C3-215449, C3-215450, C3-215451 | 0.2.0 |
| 2021-11 | CT3#119-e | C3-215478 |  |  |  | Inclusion of: C3-215442, C3-215443, C3-215444, C3-215445, C3-215446, C3-215447, C3-215448, C3-215449, C3-215450, C3-215451 | 0.3.0 |
| 2021-11 | CT3#119-e | C3-216551 | - | - | - | Inclusion of: C3-216211, C3-216212, C3-216213, C3-216214, C3-216215, C3-216216, C3-216217, C3-216218, C3-216219 | 0.4.0 |
| 2021-12 | CT#94-e | CP-213206 | - | - | - | Presented for information | 1.0.0 |
| 2022-01 | CT3#119-bis-e | C3-220456 |  |  |  | Inclusion of: C3-220308, C3-220309, C3-220310, C3-220311, C3-220312, C3-220313, C3-220314, C3-220315 | 1.1.0 |
| 2022-02 | CT3#120-e | C3-221557 |  |  |  | Inclusion of: C3-221342, C3-221343, C3-221344, C3-221345, C3-221346, C3-221347, C3-221348, C3-221349, C3-221352, C3-221353, C3-221638, C3-221639, C3-221640 | 1.2.0 |