| 3GPP TS 29.580 V17.1.0 (2022-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Multicast/Broadcast Service Function Services; Stage 3 (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

1	Scope	9

2	References	9

3	Definitions, symbols and abbreviations	10

3.1	Definitions	10

3.2	Symbols	10

3.3	Abbreviations	10

4	Overview	10

5	Services offered by the MBSF	11

5.1	Introduction	11

5.2	Nmbsf_MBSUserService Service	12

5.2.1	Service Description	12

5.2.2	Service Operations	12

5.2.2.1	Introduction	12

5.2.2.2	Nmbsf_MBSUserService_Create service operation	12

5.2.2.2.1	General	12

5.2.2.2.2	MBS User Service Creation	12

5.2.2.3	Nmbsf_MBSUserService_Retrieve service operation	13

5.2.2.3.1	General	13

5.2.2.3.2	MBS User Service Retrieval	13

5.2.2.4	Nmbsf_MBSUserService_Update service operation	14

5.2.2.4.1	General	14

5.2.2.4.2	MBS User Service Update	14

5.2.2.5	Nmbsf_MBSUserService_Delete service operation	15

5.2.2.5.1	General	15

5.2.2.5.2	MBS User Service Deletion	15

5.3	Nmbsf_MBSUserDataIngestSession Service	16

5.3.1	Service Description	16

5.3.2	Service Operations	16

5.3.2.1	Introduction	16

5.3.2.2	Nmbsf_MBSUserDataIngestSession_Create service operation	17

5.3.2.2.1	General	17

5.3.2.2.2	MBS User Data Ingest Session Creation	17

5.3.2.3	Nmbsf_MBSUserDataIngestSession_Retrieve service operation	18

5.3.2.3.1	General	18

5.3.2.3.2	MBS User Data Ingest Session Retrieval	19

5.3.2.4	Nmbsf_MBSUserDataIngestSession_Update service operation	19

5.3.2.4.1	General	19

5.3.2.4.2	MBS User Data Ingest Session Update	19

5.3.2.5	Nmbsf_MBSUserDataIngestSession_Delete service operation	21

5.3.2.5.1	General	21

5.3.2.5.2	MBS User Data Ingest Session Deletion	21

5.3.2.6	Nmbsf_MBSUserDataIngestSession_StatusSubscribe service operation	22

5.3.2.6.1	General	22

5.3.2.6.2	MBS User Data Ingest Session Status Subscription Creation	22

5.3.2.7	Nmbsf_MBSUserDataIngestSession_StatusSubscribeMod service operation	23

5.3.2.7.1	General	23

5.3.2.7.2	MBS User Data Ingest Session Status Subscription Update	23

5.3.2.8	Nmbsf_MBSUserDataIngestSession_StatusUnsubscribe service operation	23

5.3.2.8.1	General	23

5.3.2.8.2	MBS User Data Ingest Session Status Subscription Deletion	24

5.3.2.9	Nmbsf_MBSUserDataIngestSession_StatusNotify service operation	24

5.3.2.9.1	General	24

5.3.2.9.2	MBS User Data Ingest Session Status Notification	24

6	API Definitions	25

6.1	Nmbsf_MBSUserService Service API	25

6.1.1	Introduction	25

6.1.2	Usage of HTTP	26

6.1.2.1	General	26

6.1.2.2	HTTP standard headers	26

6.1.2.2.1	General	26

6.1.2.2.2	Content type	26

6.1.2.3	HTTP custom headers	26

6.1.3	Resources	26

6.1.3.1	Overview	26

6.1.3.2	Resource: MBS User Services	27

6.1.3.2.1	Description	27

6.1.3.2.2	Resource Definition	27

6.1.3.2.3	Resource Standard Methods	27

6.1.3.2.3.1	GET	27

6.1.3.2.3.2	POST	28

6.1.3.2.4	Resource Custom Operations	29

6.1.3.3	Resource: Individual MBS User Service	29

6.1.3.3.1	Description	29

6.1.3.3.2	Resource Definition	29

6.1.3.3.3	Resource Standard Methods	29

6.1.3.3.3.1	GET	29

6.1.3.3.3.2	PUT	30

6.1.3.3.3.3	PATCH	31

6.1.3.3.3.4	DELETE	32

6.1.4	Custom Operations without associated resources	33

6.1.5	Notifications	33

6.1.6	Data Model	33

6.1.6.1	General	33

6.1.6.2	Structured data types	34

6.1.6.2.1	Introduction	34

6.1.6.2.2	Type: MBSUserService	35

6.1.6.2.3	Type: ServiceNameDescription	35

6.1.6.2.4	Type: MBSUserServicePatch	36

6.1.6.3	Simple data types and enumerations	36

6.1.6.3.1	Introduction	36

6.1.6.3.2	Simple data types	36

6.1.6.3.3	Enumeration: ServiceAnnouncementMode	36

6.1.6.4	Data types describing alternative data types or combinations of data types	36

6.1.6.5	Binary data	37

6.1.6.5.1	Binary Data Types	37

6.1.7	Error Handling	37

6.1.7.1	General	37

6.1.7.2	Protocol Errors	37

6.1.7.3	Application Errors	37

6.1.8	Feature negotiation	37

6.1.9	Security	37

6.2	Nmbsf_MBSUserDataIngestSession Service API	38

6.2.1	Introduction	38

6.2.2	Usage of HTTP	38

6.2.2.1	General	38

6.2.2.2	HTTP standard headers	38

6.2.2.2.1	General	38

6.2.2.2.2	Content type	38

6.2.2.3	HTTP custom headers	38

6.2.3	Resources	39

6.2.3.1	Overview	39

6.2.3.2	Resource: MBS User Data Ingest Sessions	40

6.2.3.2.1	Description	40

6.2.3.2.2	Resource Definition	40

6.2.3.2.3	Resource Standard Methods	41

6.2.3.2.3.1	GET	41

6.2.3.2.3.2	POST	41

6.2.3.2.4	Resource Custom Operations	42

6.2.3.3	Resource: Individual MBS User Data Ingest Session	42

6.2.3.3.1	Description	42

6.2.3.3.2	Resource Definition	42

6.2.3.3.3	Resource Standard Methods	43

6.2.3.3.3.1	GET	43

6.2.3.3.3.2	PUT	44

6.2.3.3.3.3	PATCH	45

6.2.3.3.3.4	DELETE	46

6.2.3.3.4	Resource Custom Operations	47

6.2.3.4	Resource: MBS User Data Ingest Session Status Subscriptions	47

6.2.3.4.1	Description	47

6.2.3.4.2	Resource Definition	47

6.2.3.4.3	Resource Standard Methods	47

6.2.3.4.3.1	GET	47

6.2.3.4.3.2	POST	48

6.2.3.4.4	Resource Custom Operations	49

6.2.3.5	Resource: Individual MBS User Data Ingest Session Status Subscription	49

6.2.3.5.1	Description	49

6.2.3.5.2	Resource Definition	49

6.2.3.5.3	Resource Standard Methods	49

6.2.3.5.3.1	GET	49

6.2.3.5.3.2	PUT	50

6.2.3.5.3.3	PATCH	51

6.2.3.5.3.4	DELETE	52

6.2.3.5.4	Resource Custom Operations	53

6.2.4	Custom Operations without associated resources	53

6.2.5	Notifications	53

6.2.5.1	General	53

6.2.5.2	MBS User Data Ingest Session Status Notification	54

6.2.5.2.1	Description	54

6.2.5.2.2	Target URI	54

6.2.5.2.3	Standard Methods	54

6.2.5.2.3.1	POST	54

6.2.6	Data Model	55

6.2.6.1	General	55

6.2.6.2	Structured data types	57

6.2.6.2.1	Introduction	57

6.2.6.2.2	Type: MBSUserDataIngSession	58

6.2.6.2.3	Type: MBSDistributionSessionInfo	59

6.2.6.2.4	Type: MBSUserDataIngSessionPatch	63

6.2.6.2.5	Type: ObjectDistrMethInfo	63

6.2.6.2.6	Type: PacketDistrMethInfo	64

6.2.6.2.7	Type MBSUserDataIngStatSubsc	64

6.2.6.2.8	Type SubscribedEvent	65

6.2.6.2.9	Type MBSUserDataIngStatNotif	65

6.2.6.2.10	Type EventNotification	65

6.2.6.2.11	Type MBSUserServAnmt	66

6.2.6.2.12	Type MBSDistSessionAnmt	67

6.2.6.2.13	Type ObjectDistMethAnmtInfo	68

6.2.6.2.14	Type: FECConfig	68

6.2.6.2.15	Type: AddFecParams	69

6.2.6.2.16	Type MBSUserDataIngStatSubscPatch	69

6.2.6.3	Simple data types and enumerations	69

6.2.6.3.1	Introduction	69

6.2.6.3.2	Simple data types	69

6.2.6.3.3	Enumeration: DistributionMethod	69

6.2.6.3.4	Enumeration: Event	69

6.2.6.4	Data types describing alternative data types or combinations of data types	70

6.2.6.5	Binary data	70

6.2.6.5.1	Binary Data Types	70

6.2.7	Error Handling	71

6.2.7.1	General	71

6.2.7.2	Protocol Errors	71

6.2.7.3	Application Errors	71

6.2.8	Feature negotiation	71

6.2.9	Security	71

Annex A (normative): OpenAPI specification	72

A.1	General	72

A.2	Nmbsf_MBSUserService API	72

A.3	Nmbsf_MBSUserDataIngestSession API	77

Annex B (informative): Withdrawn API versions	91

B.1	General	91

B.2	Nmbsf_MBSUserService API	91

B.3	Nmbsf_MBSUserDataIngestSession API	91

Annex C (informative): Change history	93

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the stage 3 protocol and data model for the Nmbsf Service Based Interface. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the MBSF.

The 5G System stage 2 architecture and procedures are specified in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3]. The stage 2 architecture and procedures for 5G Multicast/Broadcast Services are specified in 3GPP TS 23.247 [14] and 3GPP TS 26.502 [15].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [4] and 3GPP TS 29.501 [5].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 7807: "Problem Details for HTTP APIs".

[14]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[15]	3GPP TS 26.502: "5G Multicast-Broadcast User Service Architecture".

[16]	3GPP TS 29.554: "5G System; Background Data Transfer Policy Control Service; Stage 3".

[17]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[18]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[19]	OMA: "OMNA BCAST Service Class Registry", https://technical.openmobilealliance.org/OMNA/bcast/bcast-service-class-registry.html.

[20]	3GPP TS 29.581: "5G System; Multicast/Broadcast Service Transport Services; Stage 3".

[21]	IANA: "Reliable Multicast Transport (RMT) FEC Encoding IDs and FEC Instance IDs", https://www.iana.org/assignments/rmt-fec-parameters/rmt-fec-parameters.xhtml#rmt-fec-parameters-1

[22]	IETF RFC 7396: "JSON Merge Patch".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purpose of the present document, the terms and definitions given in clause 3 of 3GPP TS 23.247 [14] and clause 3 of 3GPP TS 26.502 [15] also apply, including the ones referencing other specifications.

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

FEC	Forward Erasure Correction

MBS	Multicast/Broadcast Service.

MBSF	Multicast/Broadcast Service Function

MBSTF	Multicast/Broadcast Service Transport Function

TMGI	Temporary Mobile Group Identity

URI	Uniform Resource Identifier

# 4	Overview

In the frame of Multicast/Broadcast Services (MBS), the Multicast/Broadcast Service Function (MBSF) provides services to NF service consumers (e.g. AF, NEF) via the Nmbsf service based interface. The MBSF supports for this purpose the functionalities defined in 3GPP TS 26.502 [15] and 3GPP TS 23.247 [14], i.e. service level functionalities to support MBS and the control of the MBSTF, when used.

Figures°4-1 and 4.2 depict the Multicast/Broadcast related reference architecture of the MBSF respectively in SBI representation and reference point representation.

Figure 4-1: Reference model for the MBSF Services – SBI representation

Figure 4-2: Reference Model for the MBSF Services – Reference point representation

# 5	Services offered by the MBSF

## 5.1	Introduction

The MBSF provides the following services:

-	Nmbsf_MBSUserService

-	Nmbsf_MBSUserDataIngestSession

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Nmbsf_MBSUserService | 5.2 | MBS User Management Service | TS29580_Nmbsf_MBSUserService.yaml | nmbsf-mbs-us | A.2 |
| Nmbsf_MBSUserDataIngestSession | 5.3 | MBS User Data Ingest Session Management Service | TS29580_Nmbsf_MBSUserDataIngestSession.yaml | nmbsf-mbs-ud-ingest | A.3 |

## 5.2	Nmbsf_MBSUserService Service

### 5.2.1	Service Description

The Nmbsf_MBSUserService service exposed by the MBSF enables an NF service consumer to:

-	create a new MBS User Service;

-	retrieve the properties of an existing MBS User Service;

-	update/modify the properties of an existing MBS User Service; and

-	delete an existing MBS User Service.

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

The service operations defined for the Nmbsf_MBSUserService service are shown in table 5.2.2.1-1.

Table 5.2.2.1-1: Nmbsf_MBSUserService Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| Nmbsf_MBSUserService_Create | This service operation enables the NF service consumer to create a new MBS User Service. | AF, NEF |
| Nmbsf_MBSUserService_Retrieve | This service operation enables the NF service consumer to retrieve the properties of an existing MBS User Service. | AF, NEF |
| Nmbsf_MBSUserService_Update | This service operation enables the NF service consumer to update an existing MBS User Service. | AF, NEF |
| Nmbsf_MBSUserService_Delete (NOTE) | This service operation enables the NF service consumer to delete an existing MBS User Service. | AF, NEF |
| NOTE: This service operation corresponds to the Nmbsf_MBSUserService_Destroy service operation defined in 3GPP TS 26.502 [15]. |  |  |

#### 5.2.2.2	Nmbsf_MBSUserService_Create service operation

##### 5.2.2.2.1	General

This service operation is used by an NF service consumer to request the creation of a new MBS User Service at the MBSF.

The following procedures are supported by the "Nmbsf_MBSUserServie_Create" service operation:

-	MBS User Service Creation.

##### 5.2.2.2.2	MBS User Service Creation

Figure 5.2.2.2.2-1 depicts a scenario where an NF service consumer requests the creation of a new MBS User Service at the MBSF.

Figure 5.2.2.2.2-1: MBS User Service Creation procedure

1.	In order to request the creation of a new MBS User Service, the NF service consumer shall send an HTTP POST request message to the MBSF targeting the "MBS User Services" collection resource, using the URI "{apiRoot}/nmbsf-mbs-us/<apiVersion>/mbs-user-services" as shown in step 1 of figure 5.2.2.2.2-1, with the request body including the MBSUserService data structure which shall contain:

-	a list of external service identifier(s), within the "extServiceIds" attribute;

-	the service type, within the the "servType" attribute;

-	the service class, within the "servClass" attribute;

-	a list of service announcement mode(s), within the "servAnnModes" attribute;

-	one or several set(s) of per language service name(s) and/or service description(s), with the "servNameDescs" attribute; and

-	the list of supported features, if feature negotiation needs to take place, within the "suppFeat" attribute;

and may include:

-	the main service language, within the "mainServLang" attribute.

2.	Upon success, the MBSF shall create a new "Individual MBS User Service" resource and respond to the NF service consumer with a "201 Created" status code, as shown in step 2 of figure 5.2.2.2.2-1, including an HTTP Location header field containing the URI of the created resource, and the response body containing a representation of the created "Individual MBS User Service" resource within the MBSUserService data structure.

On failure, the MBSF shall take proper error handling actions, as specified in clause 6.1.7, and respond to the NF service consumer with an appropriate error status code.

#### 5.2.2.3	Nmbsf_MBSUserService_Retrieve service operation

##### 5.2.2.3.1	General

This service operation is used by an NF service consumer to retrieve the properties of an existing MBS User Service at the MBSF.

The following procedures are supported by the "Nmbsf_MBSUserServie_Retrieve" service operation:

-	MBS User Service Retrieval.

##### 5.2.2.3.2	MBS User Service Retrieval

Figure 5.2.2.3.2-1 depicts a scenario where an NF service consumer requests the retrieval of the properties of an existing MBS User Service from the MBSF.

Figure 5.2.2.3.2-1: MBS User Service Retrieval procedure

1.	In order to retrieve the properties of an existing MBS User Service, the NF service consumer shall send an HTTP GET request message targeting the corresponding "Individual MBS User Service" resource, using the URI "{apiRoot}/nmbsf-mbs-us/<apiVersion>/mbs-user-services/{mbsUserServId}", as shown in step 1 of figure 5.2.2.3.2-1.

If the MBSF determines that the received HTTP GET request needs to be redirected, the MBSF shall send an HTTP redirect response, as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

2.	Upon success, the MBSF shall respond to the NF service consumer with an HTTP "200 OK" status code, as shown in step 2 of figure 5.2.2.3.2-1, with the response body including a representation of the requested "Individual MBS User Service" resource within the MBSUserService data structure.

On failure, the MBSF shall take proper error handling actions, as specified in clause 6.1.7, and respond to the NF service consumer with an appropriate error status code.

#### 5.2.2.4	Nmbsf_MBSUserService_Update service operation

##### 5.2.2.4.1	General

This service operation is used by an NF service consumer to request the update or modification of an existing MBS User Service at the MBSF.

The following procedures are supported by the "Nmbsf_MBSUserServie_Update" service operation:

-	MBS User Service Update.

##### 5.2.2.4.2	MBS User Service Update

Figure 5.2.2.4.2-1 depicts a scenario where an NF service consumer requests the update of an existing MBS User Service at the MBSF.

Figure 5.2.2.4.2-1: MBS User Service Update procedure

1.	In order to request the update or modification of an existing MBS User Service, the NF service consumer shall send an HTTP PUT or PATCH request message targeting the corresponding "Individual MBS User Service" resource, using the URI "{apiRoot}/nmbsf-mbs-us/<apiVersion>/mbs-user-services/{mbsUserServId}" as shown in step 1 of figure 5.2.2.4.2-1, with the request body including the MBSUserService data structure (when HTTP PUT is used) or the MBSUserServicePatch data structure (when HTTP PATCH is used). Only the "servType" attribute shall not be updated.

If the MBSF determines that the received HTTP PUT or PATCH request message needs to be redirected, the MBSF shall send an HTTP redirect response, as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

2.	Upon success, the MBSF shall update the concerned "Individual MBS User Service" resource and respond to the NF service consumer with either:

a)	an HTTP "200 OK" status code with the response body containing the updated representation of the resource within the MBSUserService data structure, as shown in step 2a of figure 5.2.2.4.2-1; or

b)	an HTTP "204 No Content" status code, as shown in step 2b of figure 5.2.2.4.2-1.

On failure, the MBSF shall take proper error handling actions, as specified in clause 6.1.7, and respond to the NF service consumer with an appropriate error status code.

#### 5.2.2.5	Nmbsf_MBSUserService_Delete service operation

##### 5.2.2.5.1	General

This service operation is used by the NF service consumer to request the deletion of an existing MBS User Service at the MBSF.

The following procedures are supported by the "Nmbsf_MBSUserServie_Delete" service operation:

-	MBS User Service Deletion.

##### 5.2.2.5.2	MBS User Service Deletion

Figure 5.2.2.5.2-1 depicts a scenario where an NF service consumer requests the deletion of an existing MBS User Service at the MBSF.

Figure 5.2.2.5.2-1: MBS User Service Deletion procedure

1.	In order to request the deletion of an existing MBS User Service, the NF service consumer shall send an HTTP DELETE request message targeting the corresponding "Individual MBS User Service" resource using the URI "{apiRoot}/nmbsf-mbs-us/<apiVersion>/mbs-user-services/{mbsUserServId}", as shown in step 1 of figure 5.2.2.5.2-1.

If the MBSF determines that the received HTTP DELETE request needs to be redirected, the MBSF shall send an HTTP redirect response, as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

2.	Upon success, the MBSF shall:

-	delete the targeted "Individual MBS User Service" resource; and

-	respond to the NF service consumer with an HTTP "204 No Content" status code, as shown in step 2 of figure 5.2.2.5.2-1.

On failure, the MBSF shall take proper error handling actions, as specified in clause 6.1.7, and respond to the NF service consumer with an appropriate error status code.

## 5.3	Nmbsf_MBSUserDataIngestSession Service

### 5.3.1	Service Description

The Nmbsf_MBSUserDataIngestSession service exposed by the MBSF enables an NF service consumer to:

-	create an MBS User Data Ingest Session, including a set of subordinate MBS Distribution Session(s);

-	retrieve the properties of an existing MBS User Data Ingest Session;

-	update an existing MBS User Data Ingest Session and its set of subordinate MBS Distribution Session(s);

-	delete an MBS User Data Ingest Session along with its subordinate MBS Distribution Session(s);

-	create a subscription to monitor event(s) related to the MBS User Data Ingest Session;

-	update or modify an existing subscription to MBS User Data Ingest Session event(s) monitoring;

-	delete an existing subscription to MBS User Data Ingest Session event(s) monitoring; and

-	receive notification(s) about the event(s) related to the MBS User Data Ingest Session.

### 5.3.2	Service Operations

#### 5.3.2.1	Introduction

The service operations defined for the Nmbsf_MBSUserDataIngestSession service are shown in table 5.3.2.1-1.

Table 5.3.2.1-1: Nmbsf_MBSUserDataIngestSession Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| Nmbsf_MBSUserDataIngestSession_Create | This service operation enables the NF service consumer to request the creation of an MBS User Data Ingest Session, including a set of subordinate MBS Distribution Session(s). | AF, NEF |
| Nmbsf_MBSUserDataIngestSession_Retrieve | This service operation enables the NF service consumer to retrieve the properties of an existing MBS User Data Ingest Session. | AF, NEF |
| Nmbsf_MBSUserDataIngestSession_Update | This service operation enables the NF service consumer to update an existing MBS User Data Ingest Session and its set of subordinate MBS Distribution Session(s). | AF, NEF |
| Nmbsf_MBSUserDataIngestSession_Delete (NOTE) | This service operation enables the NF service consumer to delete an existing MBS User Data Ingest Session along with its subordinate MBS Distribution Session(s). | AF, NEF |
| Nmbsf_MBSUserDataIngestSession_StatusSubscribe | This service operation enables the NF service consumer to request the creation of a subscription to monitor event(s) related to an MBS User Data Ingest Session. | AF, NEF |
| Nmbsf_MBSUserDataIngestSession_StatusSubscribeMod | This service operation enables the NF service consumer to request the update or modification of an existing subscription to monitor event(s) related to an MBS User Data Ingest Session. | AF, NEF |
| Nmbsf_MBSUserDataIngestSession_StatusUnsubscribe | This service operation enables the NF service consumer to request the deletion of an existing subscription to MBS User Data Ingest Session event(s) monitoring. | AF, NEF |
| Nmbsf_MBSUserDataIngestSession_StatusNotify | This service operation enables the NF service consumer to receive notification(s) from the MBSF about the event(s) related to an MBS User Data Ingest Session. | MBSF |
| NOTE: This service operation corresponds to the Nmbsf_MBSUserDataIngestSession_Destroy service operation defined in 3GPP TS 26.502 [15]. |  |  |

#### 5.3.2.2	Nmbsf_MBSUserDataIngestSession_Create service operation

##### 5.3.2.2.1	General

This service operation is used by the NF service consumer to request the creation of an MBS User Data Ingest Session including a set of subordinate MBS Distribution Session(s).

The following procedures are supported by the "Nmbsf_MBSUserDataIngestSession_Create" service operation:

-	MBS User Data Ingest Session Creation.

##### 5.3.2.2.2	MBS User Data Ingest Session Creation

Figure 5.3.2.2.2-1 depicts a scenario where an NF service consumer requests the creation of an MBS User Data Ingest Session, including a set of subordinate MBS Distribution Session(s), at the MBSF.

Figure 5.3.2.2.2-1: MBS User Data Ingest Session Creation procedure

1.	In order to create a new MBS User Data Ingest Session, including a set of subordinate MBS Distribution Session(s), the NF service consumer shall send an HTTP POST request message targeting the "Individual MBS User Data Ingest Sessions" collection resource, using the URI "{apiRoot}/nmbsf-mbs-ud-ingest/<apiVersion>/sessions" as shown in step 1 of figure 5.3.2.2.2-1, with the request body including the MBSUserDataIngSession data structure which shall contain:

-	the identifier of the parent MBS User Service, within the "mbsUserServId" attribute;

-	one or several MBS Distribution Session(s), within the "mbsDisSessInfos" attribute; and

-	the list of supported features, if feature negotiation needs to take place, within the "suppFeat" attribute;

and may include:

-	one or several set(s) of period(s) of time during which the MBS User Data Ingest Session is active in the MBS System, within the "actPeriods" attribute.

NOTE:	At the end of the last time period provided within the "actPeriods" attribute, the MBS User Data Ingest Session is automatically released and deleted by the MBSF.

Within the "mbsDisSessInfos" attribute, the parameters of each MBS Distribution Session to be created are provided within the MBSDistributionSessionInfo data structure encoding the corresponding map entry, and:

-	if no MBS session identifier is provided, i.e. the "mbsSessionId" is not provided, the MBSF shall request TMGI allocation as part of the creation of the corresponding MBS session at the MB-SMF; and

-	if a source specific multicast address (SSM) is provided within the "mbsSessionId" attribute and the "locationDependent" attribute is present and set to "true" (i.e. to indicate a location dependent MBS service), the MBSF shall also request TMGI allocation as part of the creation of the corresponding MBS session at the MB-SMF.

2.	Upon success, the MBSF shall create a new "Individual MBS User Data Ingest Session" resource and respond to the NF service consumer with an HTTP "201 Created" status code, as shown in step 2 of figure 5.3.2.2.2-1, including an HTTP Location header field containing the URI of the created resource, and the response body containing a representation of the created "Individual MBS User Data Ingest Session" resource within the MBSUserDataIngSession data structure.

On failure, the MBSF shall take proper error handling actions, as specified in clause 6.2.7, and respond to the NF service consumer with an appropriate error status code.

#### 5.3.2.3	Nmbsf_MBSUserDataIngestSession_Retrieve service operation

##### 5.3.2.3.1	General

This service operation is used by the NF service consumer to retrieve the properties of an existing MBS User Data Ingest Session.

The following procedures are supported by the "Nmbsf_MBSUserDataIngestSession_Retrieve" service operation:

-	MBS User Data Ingest Session Retrieval.

##### 5.3.2.3.2	MBS User Data Ingest Session Retrieval

Figure 5.3.2.3.2-1 depicts a scenario where an NF service consumer retrieves the properties of an existing MBS User Data Ingest Session from the MBSF.

Figure 5.3.2.3.2-1: MBS User Data Ingest Session Retrieval procedure

1.	In order to retrieve the properties of an existing MBS User Data Ingest Session, the NF service consumer shall send an HTTP GET request message targeting the corresponding "Individual MBS User Data Ingest Session" resource using the URI "{apiRoot}/nmbsf-mbs-ud-ingest/<apiVersion>/sessions/{sessionId}" as shown in step 1 of figure 5.3.2.3.2-1.

If the MBSF determines that the received HTTP GET request message needs to be redirected, the MBSF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

2.	Upon the reception of the HTTP GET request message, the MBSF shall respond with an HTTP "200 OK" status code, as shown in figure 5.3.2.3.2-1, step 2, with the response body including a representation of the requested "Individual MBS User Data Ingest Session" resource within the MBSUserDataIngSession data structure.

On failure, the MBSF shall take proper error handling actions, as specified in clause 6.2.7, and respond to the NF service consumer with an appropriate error status code.

#### 5.3.2.4	Nmbsf_MBSUserDataIngestSession_Update service operation

##### 5.3.2.4.1	General

This service operation is used by the NF service consumer to request the update of an existing MBS User Data Ingest Session and potentially also its set of subordinate MBS Distribution Session(s) at the MBSF.

The following procedures are supported by the "Nmbsf_MBSUserDataIngestSession_Update" service operation:

-	MBS User Data Ingest Session Update.

##### 5.3.2.4.2	MBS User Data Ingest Session Update

Figure 5.3.2.4.2-1 depicts a scenario where an NF service consumer requests the update or modification of an existing MBS User Data Ingest Session and potentially also its set of subordinate MBS Distribtion Session(s) at the MBSF.

Figure 5.3.2.4.2-1: MBS User Data Ingest Session Update procedure

1.	In order to request the update or modification of an existing MBS User Data Ingest Session and potentially also its set of subordinate MBS Distribution Session(s), the NF service consumer shall send an HTTP PUT or PATCH request message targeting the corresponding "Individual MBS User Data Ingest Session" resource using the URI "{apiRoot}/nmbsf-mbs-ud-ingest/<apiVersion>/sessions/{sessionId}", as shown in step 1 of figure 5.3.2.4.2-1, with the request body including the MBSUserDataIngSession data structure (for an HTTP PUT request) or the MBSUserDataIngSessionPatch data structure (for an HTTP PATCH request).

The attributes that may be updated/modified at any time are as follows:

-	the active periods of the MBS User Data Ingest Session, within the "actPeriods" attribute; and

-	within each map entry of the "mbsDisSessInfos" attribute encoded using the MBSDistributionSessionInfo data structure:

-	the MBS Service Information, within the "mbsServInfo" attribute;

-	the MBS Frequency Selection Area (FSA) Identifier, for a broadcast service type, within the "mbsFSAId" attribute;

-	the target service area(s), within the "tgtServAreas" attribute.

The other attributes, except the "mbsSessionId", the "mbsDisSessionId" and the "locationDependent" attributes, which shall not be updated, all the other attributes within each map entry of the "mbsDisSessInfos" attribute encoded using the MBSDistributionSessionInfo data structure may be updated only if the MBS Distribution Session is in "INACTIVE" state.

As part of an MBS User Data Ingest Session update procedure, the AF may also add new MBS Distribution Session(s) and/or remove existing MBS Distribution Session(s). In order to do so:

-	if a new MBS Distribution Session shall be created, the AF shall include its properties encoded using the MBSDistributionSessionInfo data structure as a map entry within the "mbsDisSessInfos" attribute with a newly assigned string based map key that shall be unique within the scope of the parent MBS User Data Ingest Session; and

-	if an existing MBS Distribution Session shall be deleted, the AF shall include the corresponding map entry set to the value "NULL" within the "mbsDisSessInfos" attribute with the map key set to its string based map key provisioned during the request that initially created the MBS Distribution Session.

If the MBSF determines that the received HTTP PUT or PATCH request needs to be redirected, the MBSF shall send an HTTP redirect response, as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

2.	Upon success, the MBSF shall respond with either:

a)	an HTTP "200 OK" status code with the response body containing the updated representation of the "Individual MBS User Data Ingest Session" resource within the MBSUserDataIngSession data structure, as shown in step 2a of figure 5.3.2.4.2-1; or

b)	an HTTP "204 No Content" status code, as shown in step 2b of figure 5.3.2.4.2-1.

On failure, the MBSF shall take proper error handling actions, as specified in clause 6.2.7, and respond to the NF service consumer with an appropriate error status code.

#### 5.3.2.5	Nmbsf_MBSUserDataIngestSession_Delete service operation

##### 5.3.2.5.1	General

This service operation is used by the NF service consumer to request the deletion of an MBS User Data Ingest Session along with its subordinate MBS Distribution Session(s) at the MBSF.

The following procedures are supported by the "Nmbsf_MBSUserDataIngestSession_Delete" service operation:

-	MBS User Data Ingest Session Deletion.

##### 5.3.2.5.2	MBS User Data Ingest Session Deletion

Figure 5.3.2.5.2-1 depicts a scenario where an NF service consumer requests the deletion of an existing MBS User Data Ingest Session at the MBSF.

Figure 5.3.2.5.2-1: MBS User Data Ingest Session Deletion procedure

1.	In order to request the deletion of an existing MBS User Data Ingest Session along with its subordinate MBS Distribution Session(s), the NF service consumer shall send an HTTP DELETE request message targeting the corresponding "Individual MBS User Data Ingest Session" resource using the URI "{apiRoot}/nmbsf-mbs-ud-ingest/<apiVersion>/sessions/{sessionId}", as shown in step 1 of figure 5.3.2.5.2-1.

If the MBSF determines that the received HTTP DELETE request message needs to be redirected, the MBSF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

2.	Upon success, the MBSF shall:

-	delete the targeted "Individual MBS User Data Ingest Session" resource; and

-	respond with an HTTP "204 No Content" status code, as shown in step 2 of figure 5.3.2.5.2-1.

On failure, the MBSF shall take proper error handling actions, as specified in clause 6.2.7, and respond to the NF service consumer with an appropriate error status code.

#### 5.3.2.6	Nmbsf_MBSUserDataIngestSession_StatusSubscribe service operation

##### 5.3.2.6.1	General

This service operation is invoked by an NF service consumer to request the creation of a subscription to MBS User Data Ingest Session Status event(s) reporting at the MBSF.

The following procedures are supported by the "Nmbsf_MBSUserDataIngestSession_StatusSubscribe" service operation:

-	 MBS User Data Ingest Session Status Subscription Creation.

##### 5.3.2.6.2	MBS User Data Ingest Session Status Subscription Creation

Figure 5.3.2.6.2-1 depicts a scenario where an NF service consumer requests the creation of a subscription to MBS User Data Ingest Session Status event(s) reporting at the MBSF.

Figure 5.3.2.6.2-1: MBS User Data Ingest Session Status Subscription Creation procedure

1.	In order to request the creation of a new MBS User Data Ingest Session Status Subscription, the NF service consumer shall send an HTTP POST request message targeting the "MBS User Data Ingest Session Status Subscriptions" resource using the URI "{apiRoot}/nmbsf-mbs-ud-ingest/<apiVersion>/status-subscriptions" as shown in step 1 of figure 5.3.2.6.2-1, with the request body including the MBSUserDataIngStatSubsc data structure that shall contain:

-	the identifier of the MBS User Data Ingest Session to which the subscription is related, within the "mbsIngSessionId" attribute;

-	the URI towards which the notifications should be sent, within the "notifUri" attribute; and

-	the list of subscribed MBS User Data Ingest Session Status event(s), within the "eventSubscs" attribute.

2.	Upon success, the MBSF shall create a new "Individual MBS User Data Ingest Session Status Subscription" resource and respond to the NF service consumer with an HTTP "201 Created" status code, as shown in step 2 of figure 5.3.2.6.2-1, including an HTTP Location header field containing the URI of the created resource, i.e. "{apiRoot}/nmbsf-mbs-ud-ingest/<apiVersion>/status-subscriptions/{subscriptionId}", and the response body containing a representation of the created "Individual MBS User Data Ingest Session Status Subscription" resource within the MBSUserDataIngStatSubsc data structure.

On failure, the MBSF shall take proper error handling actions, as specified in clause 6.2.7, and respond to the NF service consumer with an appropriate error status code.

#### 5.3.2.7	Nmbsf_MBSUserDataIngestSession_StatusSubscribeMod service operation

##### 5.3.2.7.1	General

This service operation is invoked by an NF service consumer to request the update/modification of a subscription to MBS User Data Ingest Session Status event(s) reporting at the MBSF.

The following procedures are supported by the "Nmbsf_MBSUserDataIngestSession_StatusSubscribeMod" service operation:

-	MBS User Data Ingest Session Status Subscription Update.

##### 5.3.2.7.2	MBS User Data Ingest Session Status Subscription Update

Figure 5.3.2.6.2-1 depicts a scenario where an NF service consumer requests the update/modification of a subscription to MBS User Data Ingest Session Status event(s) reporting at the MBSF.

Figure 5.3.2.7.2-1: MBS User Data Ingest Session Status Subscription Update procedure

1.	In order to request the update or modification of an existing MBS User Data Ingest Session Status Subscription, the NF service consumer shall send an HTTP PUT or PATCH request message targeting the corresponding "Individual MBS User Data Ingest Session Status Subscription" resource using the URI "{apiRoot}/nmbsf-mbs-ud-ingest/<apiVersion>/status-subscriptions/{subscriptionId}", as shown in step 1 of figure 5.3.2.7.2-1, with the request body including the MBSUserDataIngStatSubsc data structure (for an HTTP PUT request) or the MBSUserDataIngStatSubscPatch (for an HTTP PATCH request).

If the MBSF determines that the received HTTP PUT or PATCH request message needs to be redirected, the MBSF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

2.	Upon success, the MBSF shall update/modify the corresponding "Individual MBS User Data Ingest Session Status Subscription" resource and respond to the NF service consumer with either:

a)	an HTTP "200 OK" status code with the response body containing the updated representation of the "Individual MBS User Data Ingest Session Status Subscription" resource within the MBSUserDataIngStatSubsc data structure, as shown in step 2a of figure 5.3.2.7.2-1; or

b)	an HTTP "204 No Content" status code, as shown in step 2b of figure 5.3.2.7.2-1.

On failure, the MBSF shall take proper error handling actions, as specified in clause 6.2.7, and respond to the NF service consumer with an appropriate error status code.

#### 5.3.2.8	Nmbsf_MBSUserDataIngestSession_StatusUnsubscribe service operation

##### 5.3.2.8.1	General

This service operation is used by an NF service consumer to request the deletion of an existing MBS User Data Ingest Session Status Subscription at the MBSF.

The following procedures are supported by the "Nmbsf_MBSUserDataIngestSession_StatusUnsubscribe" service operation:

-	MBS User Data Ingest Session Status Subscription Deletion.

##### 5.3.2.8.2	MBS User Data Ingest Session Status Subscription Deletion

Figure 5.3.2.8.2-1 depicts a scenario where an NF service consumer requests the deletion of an existing MBS User Data Ingest Session Status Subscription at the MBSF.

Figure 5.3.2.8.2-1: MBS User Data Ingest Session Status Subscription Deletion procedure

1.	In order to request the deletion of an existing MBS User Data Ingest Session Status Subscription, the NF service consumer shall send an HTTP DELETE request message targeting the corresponding "Individual MBS User Data Ingest Session Status Subscription" resource using the URI "{apiRoot}/nmbsf-mbs-ud-ingest/<apiVersion>/status-subscriptions/{subscriptionId}" as shown in step 1 of figure 5.3.2.8.2-1.

If the MBSF determines that the received HTTP DELETE request message needs to be redirected, the MBSF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

2.	Upon success, the MBSF shall:

-	delete the corresponding "Individual MBS User Data Ingest Session Status Subscription" resource; and

-	respond to the NF service consumer with an HTTP "204 No Content" status code, as shown in step 2 of figure 5.3.2.8.2-1.

On failure, the MBSF shall take proper error handling actions, as specified in clause 6.2.7, and respond to the NF service consumer with an appropriate error status code.

#### 5.3.2.9	Nmbsf_MBSUserDataIngestSession_StatusNotify service operation

##### 5.3.2.9.1	General

This service operation is used by the MBSF to notify a previously subscribed NF service consumer on MBS User Data Ingest Session Status event(s).

The following procedures are supported by the "Nmbsf_MBSUserDataIngestSession_StatusNotify" service operation:

-	MBS User Data Ingest Session Status Notification.

##### 5.3.2.9.2	MBS User Data Ingest Session Status Notification

Figure 5.3.2.9.2-1 depicts a scenario where the MBSF sends a notification request to a previously subscribed NF service consumer on MBS User Data Ingest Session Status event(s).

Figure 5.3.2.9.2-1: MBS User Data Ingest Session Status Notification procedure

1.	In order to notify the NF service consumer on the occurrence of previously subscribed MBS User Data Ingest Session Status event(s), the MBSF shall send an HTTP POST request targeting the URI "{notifUri}", with the "notifUri" variable set to the notification URI received during the creation of the corresponding MBS User Data Ingest Session Status Subscription as specified in clause 5.3.2.6.2, and the request body including the MBSUserDataIngStatNotif data structure that shall include:

-	the identifier of the MBS User Data Ingest Session to which the notification is related, within the "mbsIngSessionId" attribute; and

-	the reported MBS User Data Ingest Session Status event(s), within the "eventNotifs" attribute.

If the NF service consumer determines that the received HTTP POST request message needs to be redirected, the NF service consumer shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [4].

2.	Upon success, the NF Service Consumer shall respond to the MBSF with an HTTP "204 No Content" status code.

On failure, the NF service consumer shall take proper error handling actions, as specified in clause 6.2.7, and respond to the MBSF with an appropriate error status code.

# 6	API Definitions

## 6.1	Nmbsf_MBSUserService Service API

### 6.1.1	Introduction

The Nmbsf_MBSUserService service shall use the Nmbsf_MBSUserService API.

The API URI of the Nmbsf_MBSUserService API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nmbsf-mbs-us".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.1.3.

### 6.1.2	Usage of HTTP

#### 6.1.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Nmbsf_MBSUserService API is contained in Annex A.2.

#### 6.1.2.2	HTTP standard headers

##### 6.1.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

##### 6.1.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

JSON object used in the HTTP PATCH request shall be encoded according to "JSON Merge Patch" and shall be signalled by the content type "application/merge-patch+json", as defined in IETF RFC 7396 [22].

The "Problem Details" JSON object shall be used to indicate additional details of the error in an HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

#### 6.1.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4] may be supported.

### 6.1.3	Resources

#### 6.1.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.3.1-1 depicts the resource URIs structure for the Nmbsf_MBSUserService API.

Figure 6.1.3.1-1: Resource URI structure of the Nmbsf_MBSUserService API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| MBS User Services | /mbs-user-services | GET | Retrieve all the active MBS User Services managed by the MBSF. |
|  |  | POST | Request the creation of a new MBS User Service. |
| Individual MBS User Service | /mbs-user-services/{mbsUserServId} | GET | Retrieve an existing MBS User Service managed by the MBSF. |
|  |  | PUT | Request the update of an existing MBS User Service managed by the MBSF. |
|  |  | PATCH | Request the modification of an existing MBS User Service managed by the MBSF. |
|  |  | DELETE | Request the deletion of an existing MBS User Service managed by the MBSF. |

#### 6.1.3.2	Resource: MBS User Services

##### 6.1.3.2.1	Description

This resource represents the collection of MBS User Services managed by the MBSF.

##### 6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/nmbsf-mbs-us/<apiVersion>/mbs-user-services

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1. |

##### 6.1.3.2.3	Resource Standard Methods

###### 6.1.3.2.3.1	GET

The GET method allows an NF service consumer (e.g. AF, NEF) to retrieve all the active MBS User Services managed by the MBSF.

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(MBSUserService) | M | 0..N | 200 OK | Successful case. All the active MBS User Services managed by the MBSF are returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.1.3.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.1.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

###### 6.1.3.2.3.2	POST

The POST method allows an NF service consumer (e.g. AF, NEF) to request the creation of a new MBS User Service.

This method shall support the URI query parameters specified in table 6.1.3.2.3.2-1.

Table 6.1.3.2.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.2-2 and the response data structures and response codes specified in table 6.1.3.2.3.2-3.

Table 6.1.3.2.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserService | M | 1 | Contains the parameters to request the creation of a new MBS User Service. |

Table 6.1.3.2.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserService | M | 1 | 201 Created | Successful case. The MBS User Service is successfully created and a representation of the created Individual MBS User Service resource is returned. An HTTP "Location" header that contains the URI of the created Individual MBS User Service resource shall also be included. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.1.3.2.3.2-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nmbsf-mbs-us/<apiVersion>/mbs-user-services/{mbsUserServId} |

##### 6.1.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.1.3.3	Resource: Individual MBS User Service

##### 6.1.3.3.1	Description

This resource represents an "Individual MBS User Service" resource managed by the MBSF.

##### 6.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/nmbsf-mbs-us/<apiVersion>/mbs-user-services/{mbsUserServId}

This resource shall support the resource URI variables defined in table 6.1.3.3.2-1.

Table 6.1.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1. |
| mbsUserServId | string | Represents the unique identifier of the Individual MBS User Service resource, assigned by the MBSF. |

##### 6.1.3.3.3	Resource Standard Methods

###### 6.1.3.3.3.1	GET

The GET method allows an NF service consumer (e.g. AF, NEF) to retrieve an existing Individual MBS User Service resource managed by the MBSF.

This method shall support the URI query parameters specified in table 6.1.3.3.3.1-1.

Table 6.1.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.3.3.3.1-3.

Table 6.1.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserService | M | 1 | 200 OK | Successful case. The requested Individual MBS User Service resource is returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.1.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.1.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

###### 6.1.3.3.3.2	PUT

The PUT method allows an NF service consumer (e.g. AF, NEF) to request the update of an existing MBS User Service at the MBSF.

This method shall support the URI query parameters specified in table 6.1.3.3.3.2-1.

Table 6.1.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.2-2 and the response data structures and response codes specified in table 6.1.3.3.3.2-3.

Table 6.1.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserService | M | 1 | Contains the updated representation of the existing Individual MBS User Service resource that is to be updated. |

Table 6.1.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserService | M | 1 | 200 OK | Successful case. The concerned Individual MBS User Service resource is successfully updated and a representation of the updated resource is returned to the NF service consumer in the response body. |
| n/a |  |  | 204 No Content | Successful case. The concerned Individual MBS User Service resource is successfully updated and no content is returned in the response body. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.1.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.1.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

###### 6.1.3.3.3.3	PATCH

The PATCH method allows an NF service consumer (e.g. AF, NEF) to request the modification of an existing MBS User Service at the MBSF.

This method shall support the URI query parameters specified in table 6.1.3.3.3.3-1.

Table 6.1.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.3-2 and the response data structures and response codes specified in table 6.1.3.3.3.3-3.

Table 6.1.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserServicePatch | M | 1 | Contains the parameters to request the modification of an existing Individual MBS User Service resource. |

Table 6.1.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserService | M | 1 | 200 OK | Successful case. The concerned Individual MBS User Service resource is successfully modified and a representation of the updated resource is returned to the NF service consumer in the response body. |
| n/a |  |  | 204 No Content | Successful case. The concerned Individual MBS User Service resource is successfully modified and no content is returned in the response body. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.1.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.1.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

###### 6.1.3.3.3.4	DELETE

The DELETE method allows an NF service consumer (e.g. AF, NEF) to request the deletion of an existing MBS User Service at the MBSF.

This method shall support the URI query parameters specified in table 6.1.3.3.3.4-1.

Table 6.1.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.4-2 and the response data structures and response codes specified in table 6.1.3.3.3.4-3.

Table 6.1.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The concerned Individual MBS User Service resource is successfully deleted. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.1.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.1.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

### 6.1.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 6.1.5	Notifications

There are no notifications defined for this API in this release of the specification.

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the Nmbsf_MBSUserService API.

Table 6.1.6.1-1 specifies the data types defined for the Nmbsf_MBSUserService service based interface protocol.

Table 6.1.6.1-1: Nmbsf_MBSUserService specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| MBSUserService | 6.1.6.2.2 | Represents the parameters of an MBS User Service. |  |
| MBSUserServicePatch | 6.1.6.2.4 | Represents the requested modifications to the parameters of an MBS User Service. |  |
| ServiceAnnouncementMode | 6.1.6.3.3 | Represents a service announcement mode. |  |
| ServiceNameDescription | 6.1.6.2.3 | Represents a set of per language service Name and/or service description. |  |
| ServiceType | 6.1.6.3.4 | Represents MBS User Service distribution type. |  |

Table 6.1.6.1-2 specifies data types re-used by the Nmbsf_MBSUserService service based interface protocol from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the Nmbsf_MBSUserService service based interface.

Table 6.1.6.1-2: Nmbsf_MBSUserService re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| MbsServiceType | 3GPP TS 29.571 [17] | Indicates whether this MBS User Service is distributed via Multicast MBS Session(s) or Broadcast MBS Session(s). |  |
| NetworkAreaInfo | 3GPP TS 29.554 [16] | Contains the service areas in which this MBS User Service is to be made available. |  |
| SupportedFeatures | 3GPP TS 29.571 [17] | Used to negotiate the applicability of optional features. |  |
| Uri | 3GPP TS 29.571 [17] | Represents a URI. |  |

#### 6.1.6.2	Structured data types

##### 6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.1.6.2.2	Type: MBSUserService

Table 6.1.6.2.2-1: Definition of type MBSUserService

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| extServiceIds | array(Uri) | M | 1..N | Represents the external service identifier(s) of this MBS User Service. |  |
| servType | MbsServiceType | M | 1 | Indicates the requested service type (i.e. multicast or broadcast). |  |
| servClass | Uri | M | 1 | Represents the class of the MBS User Service, expressed as a term identifier from the OMA BCAST Service Class Registry [19], e.g.: urn:oma:bcast:oma_bsc:st:1.0. |  |
| servAnnModes | array(ServiceAnnouncementMode) | M | 1..N | Represents the MBS User Service Announcement Mode(s), i.e. how the MBS User Service Announcement compiled by the MBSF is advertised to the MBSF Client. |  |
| servNameDescs | array(ServiceNameDescription) | M | 1..N | Contains one or several set(s) of per language distinguishing service name and/or service description for this MBS User Service. |  |
| mainServLang | string | O | 0..1 | Represents the main service language of this MBS User Service. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the supported optional features of the API described in clause 6.1.8. This attribute shall be provided in the HTTP POST request and response if feature negotiation needs to take place. |  |

##### 6.1.6.2.3	Type: ServiceNameDescription

Table 6.1.6.2.3-1: Definition of type ServiceNameDescription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| servName | string | C | 0..1 | Represents a distinguishing name for this MBS User Service per language. (NOTE) |  |
| servDescrip | string | C | 0..1 | Contains a description of this MBS User Service per language. (NOTE) |  |
| language | string | M | 1 | Represents the language of the distinguishing name and description for this MBS User Service. |  |
| NOTE: At least one of the "servName" attribute and the "servDescrip" attribute shall be included. |  |  |  |  |  |

##### 6.1.6.2.4	Type: MBSUserServicePatch

Table 6.1.6.2.4-1: Definition of type MBSUserServicePatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| extServiceIds | array(Uri) | O | 1..N | Represents the updated set of external service identifier(s) of the MBS User Service. |  |
| servClass | Uri | O | 0..1 | Represents the class of the MBS User Service, expressed as a term identifier from the OMNA BCAST Service Class Registry [19]. |  |
| servAnnModes | array(ServiceAnnouncementMode) | O | 1..N | Represents the updated MBS User Service Announcement Mode(s). |  |
| servNameDescs | array(ServiceNameDescription) | O | 1..N | Contains the updated set(s) of per language distinguishing service name and/or service description for the MBS User Service. |  |
| mainServLang | string | O | 0..1 | Represents the main service language of the MBS User Service. |  |

#### 6.1.6.3	Simple data types and enumerations

##### 6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 6.1.6.3.3	Enumeration: ServiceAnnouncementMode

The enumeration ServiceAnnouncementMode represents Service Announce Modes. It shall comply with the provisions of table 6.1.6.3.3-1.

Table 6.1.6.3.3-1: Enumeration ServiceAnnouncementMode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| VIA_MBS_5 | Indicates that the MBS User Service Announcement compiled by the MBSF is advertised to the MBSF Client at reference point MBS-5. |  |
| VIA_MBS_DISTRIBUTION_SESSION | Indicates that the MBS User Service Announcement compiled by the MBSF is advertised to the MBSF Client via the MBS Distribution Session. |  |
| PASSED_BACK | Indicates that the MBS User Service Announcement compiled by the MBSF is passed back to the MBS Application Provider. |  |

#### 6.1.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

#### 6.1.6.5	Binary data

##### 6.1.6.5.1	Binary Data Types

Table 6.1.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

### 6.1.7	Error Handling

#### 6.1.7.1	General

For the Nmbsf_MBSUserService API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Nmbsf_MBSUserService API.

#### 6.1.7.2	Protocol Errors

No specific procedures for the Nmbsf_MBSUserService service are specified.

#### 6.1.7.3	Application Errors

The application errors defined for the Nmbsf_MBSUserService service are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

### 6.1.8	Feature negotiation

The optional features listed in table 6.1.8-1 are defined for the Nmbsf_MBSUserService API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.1.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Nmbsf_MBSUserService API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nmbsf_MBSUserService API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nmbsf_MBSUserService service.

The Nmbsf_MBSUserService API defines a single scope "nmbsf-mbs-us" for the entire service, and it does not define any additional scopes at resource or operation level.

## 6.2	Nmbsf_MBSUserDataIngestSession Service API

### 6.2.1	Introduction

The Nmbsf_MBSUserDataIngestSession service shall use the Nmbsf_MBSUserDataIngestSession API.

The API URI of the Nmbsf_MBSUserDataIngestSession API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nmbsf-mbs-ud-ingest".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.2.3.

### 6.2.2	Usage of HTTP

#### 6.2.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Nmbsf_MBSUserDataIngestSession API is contained in Annex A.3.

#### 6.2.2.2	HTTP standard headers

##### 6.2.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

##### 6.2.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

JSON object used in the HTTP PATCH request shall be encoded according to "JSON Merge Patch" and shall be signalled by the content type "application/merge-patch+json", as defined in IETF RFC 7396 [22].

The "Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

#### 6.2.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4] may be supported.

### 6.2.3	Resources

#### 6.2.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.2.3.1-1 depicts the resource URIs structure for the Nmbsf_MBSUserDataIngestSession API.

Figure 6.2.3.1-1: Resource URI structure of the Nmbsf_MBSUserDataIngestSession API

Table 6.2.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.2.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| MBS User Data Ingest Sessions | /sessions | GET | Retrieve all the active MBS User Data Ingest Sessions managed by the MBSF. |
|  |  | POST | Request the creation of a new MBS User Data Ingest Session. |
| Individual MBS User Data Ingest Session | /sessions/{sessionId} | GET | Retrieve an existing MBS User Data Ingest Session managed by the MBSF. |
|  |  | PUT | Update an existing MBS User Data Ingest Session managed by the MBSF. |
|  |  | PATCH | Modify an existing MBS User Data Ingest Session managed by the MBSF. |
|  |  | DELETE | Delete an existing MBS User Data Ingest Session managed by the MBSF. |
| MBS User Data Ingest Session Status Subscriptions | /status-subscriptions | GET | Retrieve all the active MBS User Data Ingest Session Status Subscriptions managed by the MBSF. |
|  |  | POST | Request the creation of a new MBS User Data Ingest Session Status Subscription. |
| Individual MBS User Data Ingest Session Status Subscription | /status-subscriptions/{subscriptionId} | GET | Retrieve an existing MBS User Data Ingest Session Status Subscription managed by the MBSF. |
|  |  | PUT | Update an existing MBS User Data Ingest Session Status Subscription managed by the MBSF. |
|  |  | PATCH | Modify an existing MBS User Data Ingest Session Status Subscription managed by the MBSF. |
|  |  | DELETE | Delete an existing MBS User Data Ingest Session Status Subscription managed by the MBSF. |

#### 6.2.3.2	Resource: MBS User Data Ingest Sessions

##### 6.2.3.2.1	Description

This resource represents the collection of MBS User Data Ingest Sessions managed by the MBSF.

##### 6.2.3.2.2	Resource Definition

Resource URI: {apiRoot}/nmbsf-mbs-ud-ingest/<apiVersion>/sessions

This resource shall support the resource URI variables defined in table 6.2.3.2.2-1.

Table 6.2.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1. |

##### 6.2.3.2.3	Resource Standard Methods

###### 6.2.3.2.3.1	GET

The GET method allows an NF service consumer (e.g. AF, NEF) to retrieve all the active MBS User Data Ingest Sessions managed by the MBSF.

This method shall support the URI query parameters specified in table 6.2.3.2.3.1-1.

Table 6.2.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.2.3.1-2 and the response data structures and response codes specified in table 6.2.3.2.3.1-3.

Table 6.2.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(MBSUserDataIngSession) | M | 0..N | 200 OK | Successful case. All the active MBS User Data Ingest Sessions managed by the MBSF are returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The manadatory HTTP error status codes for the HTTP GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.2.3.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.2.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

###### 6.2.3.2.3.2	POST

The POST method allows an NF service consumer (e.g. AF, NEF) to request the creation of a new MBS User Data Ingest Session including one or several subordinate MBS Distribution Session(s).

This method shall support the URI query parameters specified in table 6.2.3.2.3.2-1.

Table 6.2.3.2.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.2.3.2-2 and the response data structures and response codes specified in table 6.2.3.2.3.2-3.

Table 6.2.3.2.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserDataIngSession | M | 1 | Contains the parameters to request the creation of a new MBS User Data Ingest Session. |

Table 6.2.3.2.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserDataIngSession | M | 1 | 201 Created | Successful case. The MBS User Data Ingest Session is successfully created and a representation of the created "Individual MBS User Data Ingest Session" resource is returned. An HTTP "Location" header that contains the resource URI of the created "Individual MBS User Data Ingest Session" resource is also included. |
| NOTE: The manadatory HTTP error status codes for the HTTP POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.2.3.2.3.2-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nmbsf-mbs-ud-ingest/<apiVersion>/sessions/{sessionId} |

##### 6.2.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.2.3.3	Resource: Individual MBS User Data Ingest Session

##### 6.2.3.3.1	Description

This resource represents an "Individual MBS User Data Ingest Session" resource managed by the MBSF.

##### 6.2.3.3.2	Resource Definition

Resource URI: {apiRoot}/nmbsf-mbs-ud-ingest/<apiVersion>/sessions/{sessionId}

This resource shall support the resource URI variables defined in table 6.2.3.3.2-1.

Table 6.2.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1. |
| sessionId | string | Represents the unique identifier of the "Individual MBS User Data Ingest Session" resource, assigned by the MBSF. |

##### 6.2.3.3.3	Resource Standard Methods

###### 6.2.3.3.3.1	GET

The GET method allows an NF service consumer (e.g. AF, NEF) to retrieve an existing "Individual MBS User Data Ingest Session" resource managed by the MBSF.

This method shall support the URI query parameters specified in table 6.2.3.3.3.1-1.

Table 6.2.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.1-2 and the response data structures and response codes specified in table 6.2.3.3.3.1-3.

Table 6.2.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserDataIngSession | M | 1 | 200 OK | Successful case. The requested "Individual MBS User Data Ingest Session" resource is successfully returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The manadatory HTTP error status codes for the HTTP GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.2.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.2.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

###### 6.2.3.3.3.2	PUT

The PUT method allows an NF service consumer (e.g. AF, NEF) to update an existing "Individual MBS User Data Ingest Session" resource managed by the MBSF.

This method shall support the URI query parameters specified in table 6.2.3.3.3.2-1.

Table 6.2.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.2-2 and the response data structures and response codes specified in table 6.2.3.3.3.2-3.

Table 6.2.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserDataIngSession | M | 1 | Contains the updated representation of the existing "Individual MBS User Data Ingest Session" resource that is to be updated. |

Table 6.2.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserDataIngSession | M | 1 | 200 OK | Successful case. The concerned "Individual MBS User Data Ingest Session" resource is successfully updated and a representation of the updated resource is returned to the NF service consumer in the response body. |
| n/a |  |  | 204 No Content | Successful case. The concerned "Individual MBS User Data Ingest Session" resource is successfully updated and no content is returned in the response body. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.2.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.2.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

###### 6.2.3.3.3.3	PATCH

The PATCH method allows an NF service consumer (e.g. AF, NEF) to modify an existing "Individual MBS User Data Ingest Session" resource managed by the MBSF.

This method shall support the URI query parameters specified in table 6.2.3.3.3.3-1.

Table 6.2.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.3-2 and the response data structures and response codes specified in table 6.2.3.3.3.3-3.

Table 6.2.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserDataIngSessionPatch | M | 1 | Contains the parameters to request the modification of an existing "Individual MBS User Data Ingest Session" resource. |

Table 6.2.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserDataIngSession | M | 1 | 200 OK | Successful case. The concerned "Individual MBS User Data Ingest Session" resource is successfully modified and a representation of the updated resource is returned to the NF service consumer in the response body. |
| n/a |  |  | 204 No Content | Successful case. The concerned "Individual MBS User Data Ingest Session" resource is successfully modified and no content is returned in the response body. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.2.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.2.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

###### 6.2.3.3.3.4	DELETE

The DELETE method allows an NF service consumer (e.g. AF, NEF) to delete an existing "Individual MBS User Data Ingest Session" resource managed by the MBSF.

This method shall support the URI query parameters specified in table 6.2.3.3.3.4-1.

Table 6.2.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.4-2 and the response data structures and response codes specified in table 6.2.3.3.3.4-3.

Table 6.2.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The concerned "Individual MBS User Data Ingest Session" resource is successfully deleted. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.2.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.2.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

##### 6.2.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.2.3.4	Resource: MBS User Data Ingest Session Status Subscriptions

##### 6.2.3.4.1	Description

This resource represents the collection of MBS User Data Ingest Session Status Subscriptions managed by the MBSF.

##### 6.2.3.4.2	Resource Definition

Resource URI: {apiRoot}/nmbsf-mbs-ud-ingest/<apiVersion>/status-subscriptions

This resource shall support the resource URI variables defined in table 6.2.3.4.2-1.

Table 6.2.3.4.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1. |

##### 6.2.3.4.3	Resource Standard Methods

###### 6.2.3.4.3.1	GET

The GET method allows an NF service consumer (e.g. AF, NEF) to retrieve all the active MBS User Data Ingest Session Status Subscriptions managed by the MBSF.

This method shall support the URI query parameters specified in table 6.2.3.4.3.1-1.

Table 6.2.3.4.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.4.3.1-2 and the response data structures and response codes specified in table 6.2.3.4.3.1-3.

Table 6.2.3.4.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.4.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(MBSUserDataIngStatSubsc) | M | 0..N | 200 OK | Successful case. All the active MBS User Data Ingest Session Status Subscriptions managed by the MBSF are returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The manadatory HTTP error status codes for the HTTP GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.2.3.4.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.2.3.4.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

###### 6.2.3.4.3.2	POST

The POST method allows an NF service consumer (e.g. AF, NEF) to request the creation of a new MBS User Data Ingest Session Status Subscription.

This method shall support the URI query parameters specified in table 6.2.3.4.3.2-1.

Table 6.2.3.4.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.4.3.2-2 and the response data structures and response codes specified in table 6.2.3.4.3.2-3.

Table 6.2.3.4.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserDataIngStatSubsc | M | 1 | Contains the parameters to request the creation of a new MBS User Data Ingest Session Status Subscription. |

Table 6.2.3.4.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserDataIngStatSubsc | M | 1 | 201 Created | Successful case. The MBS User Data Ingest Session Status Subscription is successfully created and a representation of the created "Individual MBS User Data Ingest Session Status Subscription" resource is returned. An HTTP "Location" header that contains the resource URI of the created "Individual MBS User Data Ingest Session Status Subscription" resource shall also be included. |
| NOTE: The manadatory HTTP error status codes for the HTTP POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.2.3.4.3.2-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nmbsf-mbs-ud-ingest/<apiVersion>/status-subscriptions/{subscriptionId} |

##### 6.2.3.4.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.2.3.5	Resource: Individual MBS User Data Ingest Session Status Subscription

##### 6.2.3.5.1	Description

This resource represents an "Individual MBS User Data Ingest Session Status Subscription" resource managed by the MBSF.

##### 6.2.3.5.2	Resource Definition

Resource URI: {apiRoot}/nmbsf-mbs-ud-ingest/<apiVersion>/status-subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.2.3.5.2-1.

Table 6.2.3.5.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1. |
| subscriptionId | string | Represents the unique identifier of the "Individual MBS User Data Ingest Session Status Subscription" resource, assigned by the MBSF. |

##### 6.2.3.5.3	Resource Standard Methods

###### 6.2.3.5.3.1	GET

The GET method allows an NF service consumer (e.g. AF, NEF) to retrieve an existing "Individual MBS User Data Ingest Session Status Subscription" resource managed by the MBSF.

This method shall support the URI query parameters specified in table 6.2.3.5.3.1-1.

Table 6.2.3.5.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.5.3.1-2 and the response data structures and response codes specified in table 6.2.3.5.3.1-3.

Table 6.2.3.5.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.5.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserDataIngStatSubsc | M | 1 | 200 OK | Successful case. The requested "Individual MBS User Data Ingest Session Status Subscription" resource is returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The manadatory HTTP error status codes for the HTTP GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.2.3.5.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.2.3.5.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

###### 6.2.3.5.3.2	PUT

The PATCH method allows an NF service consumer (e.g. AF, NEF) to update an existing "Individual MBS User Data Ingest Session Status Subscription" resource managed by the MBSF.

This method shall support the URI query parameters specified in table 6.2.3.5.3.2-1.

Table 6.2.3.5.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.5.3.2-2 and the response data structures and response codes specified in table 6.2.3.5.3.2-3.

Table 6.2.3.5.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserDataIngStatSubsc | M | 1 | Contains the updated representation of the existing "Individual MBS User Data Ingest Session Status Subscription" resource that is to be updated. |

Table 6.2.3.5.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserDataIngStatSubsc | M | 1 | 200 OK | Successful case. The concerned "Individual MBS User Data Ingest Session Status Subscription" resource is successfully updated and a representation of the updated resource is returned to the NF service consumer in the response body. |
| n/a |  |  | 204 No Content | Successful case. The concerned "Individual MBS User Data Ingest Session Status Subscription" resource is successfully updated and no content is returned in the response body. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.2.3.5.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.2.3.5.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

###### 6.2.3.5.3.3	PATCH

The PATCH method allows an NF service consumer (e.g. AF, NEF) to modify an existing "Individual MBS User Data Ingest Session Status Subscription" resource managed by the MBSF.

This method shall support the URI query parameters specified in table 6.2.3.5.3.3-1.

Table 6.2.3.3.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.5.3.3-2 and the response data structures and response codes specified in table 6.2.3.5.3.3-3.

Table 6.2.3.5.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserDataIngStatSubscPatch | M | 1 | Contains the parameters to request the modification of an existing "Individual MBS User Data Ingest Session Status Subscription" resource. |

Table 6.2.3.5.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSUserDataIngStatSubsc | M | 1 | 200 OK | Successful case. The concerned "Individual MBS User Data Ingest Session Status Subscription" resource is successfully modified and a representation of the updated resource is returned to the NF service consumer in the response body. |
| n/a |  |  | 204 No Content | Successful case. The concerned "Individual MBS User Data Ingest Session Status Subscription" resource is successfully modified and no content is returned in the response body. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.2.3.5.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.2.3.5.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

###### 6.2.3.5.3.4	DELETE

The DELETE method allows an NF service consumer (e.g. AF, NEF) to delete an existing "Individual MBS User Data Ingest Session Status Subscription" resource managed by the MBSF.

This method shall support the URI query parameters specified in table 6.2.3.5.3.4-1.

Table 6.2.3.5.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.5.3.4-2 and the response data structures and response codes specified in table 6.2.3.5.3.4-3.

Table 6.2.3.5.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.5.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The concerned "Individual MBS User Data Ingest Session Status Subscription" resource is successfully deleted. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MBSF (service) instance. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.2.3.5.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.2.3.5.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MBSF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

##### 6.2.3.5.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

### 6.2.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 6.2.5	Notifications

#### 6.2.5.1	General

Notifications shall comply to clause 6.2 of 3GPP TS 29.500 [4] and clause 4.6.2.3 of 3GPP TS 29.501 [5].

Table 6.2.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| MBS User Data Ingest Session Status Notification | {notifUri} | POST | This operation enables the MBSF to notify the NF service consumer (e.g. AF, NEF) on status changes of an MBS User Data Ingest Session. |

#### 6.2.5.2	MBS User Data Ingest Session Status Notification

##### 6.2.5.2.1	Description

The MBS User Data Ingest Session Status Notification is used by the MBSF to notify the NF service consumer (e.g. AF, NEF) about event(s) related to an MBS User Data Ingest Session.

##### 6.2.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 6.2.5.2.2-1.

Table 6.2.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | String formatted as URI with the Callback URI towards which the MBS User Data Ingest Session Status Notifications should be sent. |

##### 6.2.5.2.3	Standard Methods

###### 6.2.5.2.3.1	POST

This method shall support the request data structures specified in table 6.2.5.2.3.1-1 and the response data structures and response codes specified in table 6.2.5.2.3.1-2.

Table 6.2.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSUserDataIngStatNotif | M | 1 | Represents an MBS User Data Ingest Session Status Notification. |

Table 6.2.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The MBS User Data Ingest Session Status Notification is successfully received. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF service consumer (service) instance where the notification should be sent. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF service consumer (service) instance where the notification should be sent. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.2.5.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected. |

Table 6.2.5.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected. |

### 6.2.6	Data Model

#### 6.2.6.1	General

This clause specifies the application data model supported by the Nmbsf_MBSUserDataIngestSession API.

Table 6.2.6.1-1 specifies the data types defined for the Nmbsf_MBSUserDataIngestSession service based interface protocol.

Table 6.2.6.1-1: Nmbsf_MBSUserDataIngestSession specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AddFecParams | 6.2.6.2.15 | Represents additional scheme-specific parameters for AL-FEC configuration. |  |
| DistributionMethod | 6.2.6.3.3 | Represents the MBS Distribution method. |  |
| Event | 6.2.6.3.4 | Represents MBS User Data Ingest Session Status events. |  |
| EventNotification | 6.2.6.2.10 | Represents an MBS User Data Ingest Session Status event notification related information. |  |
| FECConfig | 6.2.6.2.14 | Represents FEC configuration information. |  |
| MBSDistSessionAnmt | 6.2.6.2.12 | Represents the set of MBS Distribution Session Announcements currently associated with this MBS User Service Announcement. |  |
| MBSDistributionSessionInfo | 6.2.6.2.3 | Represents MBS Distribution Session information. |  |
| MBSUserDataIngSession | 6.2.6.2.2 | Represents MBS User Data Ingest Session information. |  |
| MBSUserDataIngSessionPatch | 6.2.6.2.4 | Represents the requested modification to an MBS User Data Ingest Session. |  |
| MBSUserDataIngStatNotif | 6.2.6.2.9 | Represents an MBS User Data Ingest Session Status Notification. |  |
| MBSUserDataIngStatSubsc | 6.2.6.2.7 | Represents an MBS User Data Ingest Session Status Subscription. |  |
| MBSUserDataIngStatSubscPatch | 6.2.6.2.16 | Represents the requested modifications to an MBS User Data Ingest Session Status Subscription. |  |
| MBSUserServAnmt | 6.2.6.2.11 | Represents the MBS User Service Announcement currently associated with the MBS User Data Ingest Session. |  |
| ObjectDistMethAnmtInfo | 6.2.6.2.13 | Represents MBS Distribution Session Announcement parameters for Object Distribution Method. |  |
| ObjectDistrMethInfo | 6.2.6.2.5 | Represents additional MBS Distribution Session parameters for Object Distribution Method. |  |
| PacketDistrMethInfo | 6.2.6.2.6 | Represents additional MBS Distribution Session parameters for Packet Distribution Method. |  |
| SubscribedEvent | 6.2.6.2.8 | Represents a subscribed MBS User Data Ingest Session Status event and the related information. |  |

Table 6.2.6.1-2 specifies data types re-used by the Nmbsf_MBSUserDataIngestSession service based interface protocol from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the Nmbsf_MBSUserDataIngestSession service based interface.

Table 6.2.6.1-2: Nmbsf_MBSUserDataIngestSession re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| BitRate | 3GPP TS 29.571 [17] | Represents Bit Rate. |  |
| MbsServiceInfo | 3GPP TS 29.571 [17] | Represents MBS Service Information. |  |
| ExternalMbsServiceArea | 3GPP TS 29.571 [17] | Represents an external MBS Service Area. |  |
| DateTime | 3GPP TS 29.122 [18] | Represents an absolute date time with the format "date-time", as defined in OpenAPI Specification [6]. |  |
| DistSessionState | 3GPP TS 29.581 [19] | Represents state of an MBS Distribution Session. |  |
| MbsFsaId | 3GPP TS 29.571 [17] | Represents an MBS Frequency Selection Area ID, for a broadcast MBS session. |  |
| MbsServiceArea | 3GPP TS 29.571 [17] | Represents an MBS service area. |  |
| MbsServiceInfo | 3GPP TS 29.571 [17] | Represents the MBS Service Information. |  |
| MbsSessionId | 3GPP TS 29.571 [17] | Represents an MBS Session Identifier. |  |
| MbStfIngestAddr | 3GPP TS 29.581 [19] | Represents MBSTF ingest endpoint addresses. |  |
| ObjAcquisitionMethod | 3GPP TS 29.581 [19] | Represents the Object Acquisition Method. |  |
| ObjDistributionOperatingMode | 3GPP TS 29.581 [19] | Represents the operation mode for an Object distribution method. |  |
| PacketDelBudget | 3GPP TS 29.571 [17] | Represents Unsigned integer indicating Packet Delay Budget expressed in milliseconds. |  |
| ServiceNameDescription | Clause 6.1.6.2.3 | Represents a set of per language service Name and/or service description. |  |
| PktDistributionOperatingMode | 3GPP TS 29.581 [19] | Represents the operation mode for a Packet distribution method. |  |
| PktIngestMethod | 3GPP TS 29.581 [19] | Represents packets ingest method. |  |
| TimeWindow | 3GPP TS 29.122 [18] | Represents a Time Window. |  |
| Tmgi | 3GPP TS 29.571 [17] | Represents Temporary Mobile Group Identity. |  |
| Uri | 3GPP TS 29.571 [17] | Represents Uniform Resource Identifier. |  |
| SupportedFeatures | 3GPP TS 29.571 [17] | Used to negotiate the applicability of optional features. |  |

#### 6.2.6.2	Structured data types

##### 6.2.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.2.6.2.2	Type: MBSUserDataIngSession

Table 6.2.6.2.2-1: Definition of type MBSUserDataIngSession

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsDisSessInfos | map(MBSDistributionSessionInfo) | M | 1..N | Represents one or more MBS Distribution Session(s) composing the MBS User Data Ingest Session. The key of the map shall be set to the value of the "mbsDistSessionId" attribute of the MBSDistributionSessionInfo data structure encoding the corresponding map entry. |  |
| actPeriods | array(TimeWindow) | O | 1..N | Represents periods of time during which the MBS User Data Ingest Session is active in the MBS System. If omitted, the session shall stay active until it is explicitly terminated by the AF. |  |
| mbsUserServAnmt | MBSUserServAnmt | O | 0..1 | Represents the MBS User Service Announcement currently associated with the MBS User Data Ingest Session. This attribute may only be present in an HTTP POST/PATCH response to an MBS User Data Ingest Session creation/modification request and only if all the constituent MBS Distribution Sessions are in the "ESTABLISHED" or "ACTIVE" state and the "PASSED_BACK" service announcement mode is provisioned within the service announcement modes supported for the parent Individual MBS User Service identified by the "mbsUserServId" attribute. |  |
| mbsUserServId | string | M | 1 | Represents the Identifier of the parent MBS User Service. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the supported optional features of the API as described in clause 6.2.8. This attribute shall be present only in an HTTP POST/PUT request or response and if feature negotiation needs to take place. |  |

##### 6.2.6.2.3	Type: MBSDistributionSessionInfo

Table 6.2.6.2.3-1: Definition of type MBSDistributionSessionInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsDistSessionId | string | C | 0..1 | Represents the identifier of the MBS Distribution Session. This attribute shall only be present in the response to an MBS User Data Ingest Session creation request or a subsequent MBS User Data Ingest Session update/modification request. |  |
| mbsSessionId | MbsSessionId | O | 0..1 | Represents the identifier of the MBS Session to which the MBS Distribution Session is related. It can be set to the Temporary Mobile Group Identity (TMGI) or Source-Specific Multicast (SSM) IP address of the MBS Session corresponding to this MBS Distribution Session. When this attribute is absent or present and contains a source specific multicast address and the "locationDependent" attribute is present and set to "true", then: - TMGI allocation shall be performed by the MBSF; and - This attribute shall be present in the HTTP POST response to the corresponding MBS User Data Ingest session creation request and contain the allocated TMGI value. |  |
| mbsServInfo | MbsServiceInfo | O | 0..1 | Contains the MBS Service Information for the MBS session. |  |
| maxContBitRate | BitRate | M | 1 | Represents the maximum bit rate for content in this MBS Distribution Session. |  |
| maxContDelay | PacketDelBudget | O | 0..1 | Represents the maximum end-to-end distribution delay that is tolerated for content in this MBS Distribution Session. |  |
| distrMethod | DistributionMethod | M | 1 | Represents the distribution method for this MBS Distribution Session. |  |
| fecConfig | FECConfig | O | 0..1 | Represents the AL-FEC (Application Level – Forward Error Correction) configuration information to be used by the MBSTF to protect this MBS Distribution Session. |  |
| objDistrInfo | ObjectDistrMethInfo | C | 0..1 | Represents the MBS Distribution Session parameters for Object Distribution Method. This attribute shall only be present when the "distrMethod" attribute value is set as "OBJECT". |  |
| pckDistrInfo | PacketDistrMethInfo | C | 0..1 | Represents the MBS Distribution Session parameters for Packet Distribution Method. This attribute shall only be present when the "distrMethod" attribute is set to "PACKET". |  |
| trafficMarkingInfo | string | O | 0..1 | Contains traffic marking information (e.g. a Differentiated Services Code Point) to be applied by the MBSTF to outgoing traffic. This attribute shall be encoded as two octet string in hexadecimal representation. The first octet shall contain the DSCP value in the IPv4 Type-of-Service or the IPv6 Traffic-Class field, and the second octet shall contain the ToS/Traffic Class mask field, which shall be set to "0xFC". |  |
| mbsDistSessState | DistSessionState | C | 0..1 | Represents the state of the MBS Distribution Session. This attribute shall only be present in the HTTP POST/PUT/PATCH response to the corresponding MBS User Data Ingest session creation or update/modification request. |  |
| tgtServAreas | MbsServiceArea | O | 0..1 | Represents the set of target service area(s) constituting the MBS Service Area of the MBS Distribution Session. This attribute may be present only in the case of a trusted/internal AF (i.e. MBS Application Provider). |  |
| extTgtServAreas | ExternalMbsServiceArea | O | 0..1 | Represents the set of target service area(s) constituting the external MBS Service Area (i.e. list of geographical area(s) or civic address(es)) of the MBS Distribution Session. This attribute may be present only in the case of an untrusted/external AF (MBS Application Provider). |  |
| mbsFSAId | MbsFsaId | O | 0..1 | Represents MBS Frequency Selection Assistance information corresponding to this MBS Distribution Session. This attribute may be included only if the parent MBS User Service is of Broadcast service type. |  |
| locationDependent | boolean | O | 0..1 | Represents an indication that this MBS Distribution Session belongs to a location-dependent MBS. This attribute shall be: - set to "true" to indicate that the MBS Distribution Session belongs to a location-dependent MBS; or - set to "false" to indicate that the MBS Distribution Session does not belong to a location-dependent MBS. The default value is "false", if omitted. |  |
| multiplexedServFlag | boolean | O | 0..1 | Represents an indication that this MBS Distribution Session belongs to a multiplex, i.e. forms part of a set of MBS Distribution Sessions under the same parent MBS User Data Ingest Session with identical or empty sets of target service areas and multiplexed onto the same MBS Session at the MB-SMF. This attribute shall be: - set to "true" to indicate that the MBS Distribution Session belongs to a multiplex; or - set to "false" to indicate that the MBS Distribution Session does not belong to a multiplex. The default value is "false", if omitted. |  |
| restrictedFlag | boolean | O | 0..1 | Represents an indication that this MBS Distribution Session is not open to any UE, i.e. restricted to a set of UEs according to their MBS related subscription information. This attribute may be included only if the parent MBS User Service is of Multicast service type. This attribute shall be: - set to "true" to indicate that this MBS Distribution Session is restricted to a set of UE(s); or - set to "false" to indicate that this MBS Distribution Session is open to any UE. The default value is "false", if omitted. |  |

##### 6.2.6.2.4	Type: MBSUserDataIngSessionPatch

Table 6.2.6.2.4-1: Definition of type MBSUserDataIngSessionPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| actPeriods | array(TimeWindow) | O | 1..N | Represents periods of time during which the MBS User Data Ingest Session is active in the MBS System. |  |
| mbsDisSessInfos | map(MBSDistributionSessionInfo) | O | 1..N | Contains the requested modifications to one or more MBS Distribution Session(s) composing the MBS User Data Ingest Session. |  |

##### 6.2.6.2.5	Type: ObjectDistrMethInfo

Table 6.2.6.2.5-1: Definition of type ObjectDistrMethInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| operatingMode | ObjDistributionOperatingMode | M | 1 | Represents the operating mode for the Object distribution method. |  |
| objAcqMethod | ObjAcquisitionMethod | M | 1 | Represents the object(s) acquisition method. |  |
| objAcqIds | array(Uri) | M | 1..N | Represents the object(s) to be ingested and distributed by the MBSTF during this MBS Distribution Session. |  |
| objIngUri | Uri | O | 0..1 | Represents a prefix substituted by the MBSTF with the Object distribution base URL prior to the distribution of the ingested objects. When the "objAcqMethod" is set to "PULL", this attribute is provided by the AF and present in the creation and/or update/modification of the corresponding MBS User Data Ingest Session. When the "objAcqMethod" is set to "PUSH", this attribute is provided by the MBSF and present in the response to the creation and/or update/modification request of the corresponding MBS User Data Ingest Session. |  |
| objDistrUri | Uri | O | 0..1 | Represents a prefix substituted by the MBSTF in place of the Object ingest base URL prior to the distribution of the ingested objects. |  |
| objRepairUri | Uri | O | 0..1 | Represents the object repair base URI. It contains a URI prefix substituted by the MBSTF Client in place of the Object distribution base URI when repairing objects not received completely intact from this MBS Distribution Session. The value shall point to the MBS AS. This attribute may only be present in responses to MBS User Data Ingest Session creation/update/modification requests. |  |

##### 6.2.6.2.6	Type: PacketDistrMethInfo

Table 6.2.6.2.6-1: Definition of type PacketDistrMethInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| operatingMode | PktDistributionOperatingMode | M | 1 | Contains the desired operating mode for the Packet distribution method. |  |
| pckIngMethod | PktIngestMethod | M | 1 | Represents the packets ingest method, i.e. unicast ingest or multicast ingest. |  |
| ingEndpointAddrs | MbStfIngestAddr | M | 1 | The endpoint addresses used by the AF (e.g. MBS Application Provider) and the MBSTF to establish a connection at reference point Nmb8 prior to the commencement of the MBS User Data Ingest Session. In the case of Proxy mode, this shall be the Source-Specific Multicast (SSM) endpoint addresses (including the source IP address, destination multicast group address and destination UDP port) and provided either by the AF (e.g. MBS Application Provider) or by the MBSF. In the case of Forward-only mode, this shall be the IP addresses and UDP port numbers at the source and destination ends of the content ingest tunnel, and provided respectively by the AF (e.g. MBS Application Provider) and the MBSTF. |  |

##### 6.2.6.2.7	Type MBSUserDataIngStatSubsc

Table 6.2.6.2.7-1: Definition of type MBSUserDataIngStatSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsIngSessionId | string | M | 1 | Represents the identifier of the MBS User Data Ingest Session to which the subscription is related. |  |
| eventSubscs | array(SubscribedEvent) | M | 1..N | Represents the list of subscribed MBS User Data Ingest Session Status event(s). |  |
| notifUri | Uri | M | 1 | Represents the notification URI to be used for MBS User Data Ingest Session Status event(s) reporting. |  |

##### 6.2.6.2.8	Type SubscribedEvent

Table 6.2.6.2.8-1: Definition of type SubscribedEvent

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| statusEvent | Event | M | 1 | Represents the subscribed MBS User Data Ingest Session Status event. |  |
| mbsDistSessionId | string | C | 0..1 | Represents the identifier for the MBS Distribution Session to which the subscribed MBS User Data Ingest Session Status event is related. This attribute shall be provided if the subscribed event is related to a particular MBS Distribution Session within the concerned Individual MBS User Data Ingest Session. |  |

##### 6.2.6.2.9	Type MBSUserDataIngStatNotif

Table 6.2.6.2.9-1: Definition of type MBSUserDataIngStatNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsIngSessionId | string | M | 1 | Represents the identifier for the MBS User Data Ingest Session to which the notification is related . |  |
| eventNotifs | array(EventNotification) | M | 1..N | Represents the set of reported MBS User Data Ingest Session Status event(s) and the related information. |  |

##### 6.2.6.2.10	Type EventNotification

Table 6.2.6.2.10-1: Definition of type EventNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| statusEvent | Event | M | 1. | Represents the reported MBS User Data Ingest Session Status event. |  |
| mbsDisSessionId | string | C | 0..1 | Represents the identifier for the MBS Distribution Session to which the reported event is related. This attribute shall be provided if the reported event relates to a particular MBS Distribution Session within the concerned Individual MBS User Data Ingest Session. |  |
| statusAddInfo | string | O | 0..1 | Represents additional information on the reported MBS User Data Ingest Session Status event within the "statusEvent" attribute. |  |
| timeStamp | DateTime | M | 1 | Represents the time at which the MBS User Data Ingest Session Status event is observed. |  |

##### 6.2.6.2.11	Type MBSUserServAnmt

Table 6.2.6.2.11-1: Definition of type MBSUserServAnmt

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| extServiceId | array(string) | M | 1..N | Represents the external service identifier(s) of this MBS User Service. |  |
| servClass | string | M | 1 | Represents the class of the MBS User Service, expressed as a term identifier from the OMA BCAST Service Class Registry [19]. |  |
| startTime | DateTime | O | 0..1 | Represents the start time from which this MBS User Service Announcement is valid. If not present, the announcement is already valid. |  |
| endTime | DateTime | O | 0..1 | Represents the end time after which this MBS User Service Announcement is no longer valid. If not present, the announcement is valid indefinitely. |  |
| servNameDescs | array(ServiceNameDescription) | M | 1..N | Contains one or several set(s) of per language distinguishing service name and/or service description for this MBS User Service. |  |
| mainServLang | string | O | 0..1 | Represents the main service language of this MBS User Service. |  |
| mbsDistSessAnmt | map(MBSDistSessionAnmt) | C | 1..N | Represents the set of MBS Distribution Session Announcements currently associated with this MBS User Service Announcement. The key of the map shall be set to any string value. |  |

##### 6.2.6.2.12	Type MBSDistSessionAnmt

Table 6.2.6.2.12-1: Definition of type MBSDistSessionAnmt

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsSessionId | MbsSessionId | O | 0..1 | Represents the MBS Distribution Session Identifier with the Temporary Mobile Group Identity (TMGI) or Source-Specific Multicast (SSM) IP address of the MBS Session supporting this MBS Distribution Session. |  |
| mbsFSAId | MbsFsaId | O | 0..1 | Represents MBS Frequency Selection Assistance information corresponding to the MBS Distribution Session. This attribute may be included only if the parent MBS User Service is of Broadcast service type. |  |
| distrMethod | DistributionMethod | M | 1 | Represents the distribution method of this MBS Distribution Session. |  |
| objDistrAnnInfo | ObjectDistMethAnmtInfo | O | 0..1 | Represents MBS Distribution Session Announcement parameters for Object Distribution Method. May only be present when the "distrMethod" attribute value is set as "OBJECT". |  |
| sesDesInfo | array(string) | M | 1..N | Represnts the additional parameters needed to receive the MBS Distribution Session from which this announcement is derived, including relevant User Plane traffic flow parameters. |  |

##### 6.2.6.2.13	Type ObjectDistMethAnmtInfo

Table 6.2.6.2.13-1: Definition of type ObjectDistMethAnmtInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| objDistrSched | TimeWindow | O | 0..1 | Represents a schedule indicating when individual objects are to be delivered on the corresponding MBS Distribution Session. This attribute may be present only when this information has been provided in the Object acquisition identifiers of the corresponding MBS Distribution Session. |  |
| objDistrBaseUri | Uri | O | 0..1 | Represents a URI prefix substituted by the MBSTF Client with the Object repair base URI when repairing objects not received completely intact from the corresponding MBS Distribution Session. This attribute may be present only when object repair is provisioned for the corresponding MBS Distribution Session. |  |
| objRepBaseUri | Uri | O | 0..1 | Represents the base URI of the MBS AS to be used for object repair of the corresponding MBS Distribution Session. This attribute may be present only when object repair is provisioned for the corresponding MBS Distribution Session. |  |

##### 6.2.6.2.14	Type: FECConfig

Table 6.2.6.2.14-1: Definition of type FECConfig

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| fecScheme | Uri | M | 1 | Contains the AL-FEC scheme to be used by the MBSTF. It shall be identified using a term from the IANA: "Reliable Multicast Transport (RMT) FEC Encoding IDs and FEC Instance IDs" [20] expressed as a URN, e.g.: urn:ietf:rmt:fec:encoding:0 |  |
| fecOverhead | integer | M | 1 | The overhead of AL-FEC protection, corresponding to a proportion of the (unprotected) MBS data, expressed in the form of a percentage. |  |
| additionalParams | array(AddFecParams) | O | 1..N | Represents additional scheme-specific parameters for AL-FEC configuration, encoded using uncontrolled {name, value} pairs. |  |

##### 6.2.6.2.15	Type: AddFecParams

Table 6.2.6.2.15-1: Definition of type AddFecParams

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| paramName | string | M | 1 | Contains the name of the FEC configuration parameter. |  |
| paramValue | string | M | 1 | Contains the value of the FEC configuration parameter. |  |

##### 6.2.6.2.16	Type MBSUserDataIngStatSubscPatch

Table 6.2.6.2.16-1: Definition of type MBSUserDataIngStatSubscPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventSubscs | array(SubscribedEvent) | O | 1..N | Represents the updated list of subscribed MBS User Data Ingest Session Status event(s). |  |
| notifUri | Uri | O | 0..1 | Represents the updated notification URI to be used for MBS User Data Ingest Session Status event(s) reporting. |  |

#### 6.2.6.3	Simple data types and enumerations

##### 6.2.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.2.6.3.2	Simple data types

The simple data types defined in table 6.2.6.3.2-1 shall be supported.

Table 6.2.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 6.2.6.3.3	Enumeration: DistributionMethod

The enumeration DistributionMethod represents the MBS Distribution Method. It shall comply with the provisions of table 6.2.6.3.3-1.

Table 6.2.6.3.3-1: Enumeration DistributionMehod

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| OBJECT | Indicates the Object Distribution Method. |  |
| PACKET | Indicates the Packet Distribution Method. |  |

##### 6.2.6.3.4	Enumeration: Event

The enumeration Event represents the MBS User Data Ingest Session Status events. It shall comply with the provisions of table 6.2.6.3.4-1.

Table 6.2.6.3.4-1: Enumeration Event

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| USER_DATA_ING_SESS_STARTING | Indicates that the MBS User Data Ingest Session is starting. This is an "MBS User Data Ingest Session" level event. |  |
| USER_DATA_ING_SESS_STARTED | Indicates that the MBS User Data Ingest Session started. This is an "MBS User Data Ingest Session" level event. |  |
| USER_DATA_ING_SESS_TERMINATED | Indicates that the MBS User Data Ingest Session is terminated. This is an "MBS User Data Ingest Session" level event. |  |
| DIST_SESS_STARTING | Indicates that the MBS Distribution Session is starting. This is an "MBS Distribution Session" level event. |  |
| DIST_SESS_STARTED | Indicates that the MBS Distribution Session started. This is an "MBS Distribution Session" level event. |  |
| DIST_SESS_TERMINATED | Indicates that the MBS Distribution Session is terminated. This is an "MBS Distribution Session" level event. |  |
| DIST_SESS_SERV_MNGT_FAILURE | Indicates that the MBS Distribution Session could not be started (e.g. the necessary resources could not be allocated by the MBS system). This is an "MBS Distribution Session" level event. |  |
| DIST_SESS_POL_CRTL_FAILURE | Indicates that the MBS Distribution Session could not be started because of a policy authorization/control failure or rejection. This is an "MBS Distribution Session" level event. |  |
| DATA_INGEST_FAILURE | The MBS User Data Ingest failed because the MBSTF is expecting data (the MBS Session is active), but not receiving it. This is an "MBS Distribution Session" level event. |  |
| DELIVERY_STARTED | The MBS User Data delivery is started. |  |
| SESSION_TERMINATED | The MBS User Data Ingest Session is terminated. |  |

#### 6.2.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

#### 6.2.6.5	Binary data

##### 6.2.6.5.1	Binary Data Types

Table 6.2.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

### 6.2.7	Error Handling

#### 6.2.7.1	General

For the Nmbsf_MBSUserDataIngestSession API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Nmbsf_MBSUserDataIngestSession API.

#### 6.2.7.2	Protocol Errors

No specific procedures for the Nmbsf_MBSUserDataIngestSession service are specified.

#### 6.2.7.3	Application Errors

The application errors defined for the Nmbsf_MBSUserDataIngestSession service are listed in Table 6.2.7.3-1.

Table 6.2.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

### 6.2.8	Feature negotiation

The optional features listed in table 6.2.8-1 are defined for the Nmbsf_MBSUserDataIngestSession API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.2.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.2.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Nmbsf_MBSUserDataIngestSession API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nmbsf_MBSUserDataIngestSession API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nmbsf_MBSUserDataIngestSession service.

The Nmbsf_MBSUserDataIngestSession API defines a single scope "nmbsf-mbs-ud-ingest" for the entire service, and it does not define any additional scopes at resource or operation level.

Annex A (normative):
OpenAPI specification

# A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see 3GPP TS 29.501 [5] clause 5.3.1 and 3GPP TR 21.900 [7] clause 5B).

# A.2	Nmbsf_MBSUserService API

openapi: 3.0.0

info:

title: nmbsf-mbs-us

version: 1.0.1

description: |

API for MBS User Service.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.580 V17.1.0; 5G System; Multicast/Broadcast Service Function Services.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.580/'

servers:

- url: '{apiRoot}/nmbsf-mbs-us/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials: []

paths:

/mbs-user-services:

get:

summary: Retrieve all the active MBS User Service resources managed by the MBSF.

tags:

- MBS User Services (Collection)

operationId: RetrieveMBSUserServices

responses:

'200':

description: >

OK. All the active MBS User Services managed by the MBSF are returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/MBSUserService'

minItems: 0

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

post:

summary: Request the creation of a new MBS User Service.

tags:

- MBS User Services (Collection)

operationId: CreateMBSUserService

requestBody:

description: >

Contains the parameters to request the creation of a new MBS User Service at the MBSF.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MBSUserService'

responses:

'201':

description: >

Created. A new MBS User Service is successfully created and a representation of the

created Individual MBS User Service resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MBSUserService'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/nmbsf-mbs-us/v1/mbs-user-services/{mbsUserServId}

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/mbs-user-services/{mbsUserServId}:

parameters:

- name: mbsUserServId

in: path

description: Identifier of the Individual MBS User Service resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual MBS User Service resource.

tags:

- Individual MBS User Service (Document)

operationId: RetrieveIndMBSUserService

responses:

'200':

description: >

OK. The requested Individual MBS User Service resource is successfully returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MBSUserService'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual MBS User Service resource.

tags:

- Individual MBS User Service (Document)

operationId: UpdateIndMBSUserService

requestBody:

description: >

Contains the updated representation of the Individual MBS User Service resource.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MBSUserService'

responses:

'200':

description: >

OK. The concerned Individual MBS User Service resource is successfully updated and a

representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MBSUserService'

'204':

description: >

No Content. The concerned Individual MBS User Service resource is successfully updated

and no content is returned in the response body.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual MBS User Service resource.

tags:

- Individual MBS User Service (Document)

operationId: ModifyIndMBSUserService

requestBody:

description: >

Contains the parameters to request the modification of the Individual MBS User Service

resource.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/MBSUserServicePatch'

responses:

'200':

description: >

OK. The concerned Individual MBS User Service resource is successfully modified and a

representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MBSUserService'

'204':

description: >

No Content. The concerned Individual MBS User Service resource is successfully modified

and no content is returned in the response body.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual MBS User Service resource.

tags:

- Individual MBS User Service (Document)

operationId: DeleteIndMBSUserService

responses:

'204':

description: >

No Content. The concerned Individual MBS User Service resource is successfully deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUri}'

scopes: {}

description: >

For trusted MBSF, the 'nmbsf-mbs-us' shall be used as 'scopes' and

'{nrfApiRoot}/oauth2/token' shall be used as 'tokenUri'.

#

# STRUCTURED DATA TYPES

#

schemas:

MBSUserService:

description: Represents the parameters of an MBS User Service.

type: object

properties:

extServiceIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

minItems: 1

servType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsServiceType'

servClass:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

servAnnModes:

type: array

items:

$ref: '#/components/schemas/ServiceAnnouncementMode'

minItems: 1

servNameDescs:

type: array

items:

$ref: '#/components/schemas/ServiceNameDescription'

minItems: 1

mainServLang:

type: string

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- extServiceIds

- servType

- servClass

- servAnnModes

- servNameDescs

ServiceNameDescription:

description: >

Represents a set of per language service names and/or service descriptions.

type: object

properties:

servName:

type: string

servDescrip:

type: string

language:

type: string

required:

- language

anyOf:

- required: [servName]

- required: [servDescrip]

MBSUserServicePatch:

description: >

Represents the requested modifications to the parameters of an MBS User Service.

type: object

properties:

extServiceIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

minItems: 1

servClass:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

servAnnModes:

type: array

items:

$ref: '#/components/schemas/ServiceAnnouncementMode'

minItems: 1

servNameDescs:

type: array

items:

$ref: '#/components/schemas/ServiceNameDescription'

minItems: 1

mainServLang:

type: string

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

ServiceAnnouncementMode:

anyOf:

- type: string

enum:

- VIA_MBS_5

- VIA_MBS_DISTRIBUTION_SESSION

- PASSED_BACK

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Possible values are:

- VIA_MBS_5: Indicates the MBS User Service Announcement compiled by the MBSF is advertised to the MBSF Client at reference point MBS-5.

- VIA_MBS_DISTRIBUTION_SESSION: Indicates the MBS User Service Announcement compiled by the MBSF is advertised to the MBSF Client via the MBS Distribution Session.

- PASSED_BACK: Indicates the MBS User Service Announcement compiled by the MBSF is passed back to the MBS Application Provider.

# A.3	Nmbsf_MBSUserDataIngestSession API

openapi: 3.0.0

info:

title: nmbsf-mbs-ud-ingest

version: 1.0.1

description: |

API for MBS User Data Ingest Session Service.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.580 V17.1.0; 5G System; Multicast/Broadcast Service Function Services.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.580/'

servers:

- url: '{apiRoot}/nmbsf-mbs-ud-ingest/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials: []

paths:

/sessions:

get:

summary: Retrieve all the active MBS User Data Ingest Sessions managed by the MBSF.

tags:

- MBS User Data Ingest Sessions (Collection)

operationId: RetrieveMBSUserDataIngSessions

responses:

'200':

description: >

OK. All the active MBS User Data Ingest Sessions managed by the MBSF are returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/MBSUserDataIngSession'

minItems: 0

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

post:

summary: Request the creation of a new MBS User Data Ingest Session.

tags:

- MBS User Data Ingest Sessions (Collection)

operationId: CreateMBSUserDataIngSession

requestBody:

description: >

Contains the parameters to request the creation of a new MBS User Data Ingest Session

at the MBSF.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MBSUserDataIngSession'

responses:

'201':

description: >

Created. A new MBS User Data Ingest Session is successfully created and a representation

of the created Individual MBS User Data Ingest Session resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MBSUserDataIngSession'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/nmbs-mbs-ud-ingest/v1/sessions/{sessionId}

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/sessions/{sessionId}:

parameters:

- name: sessionId

in: path

description: Identifier of the Individual MBS User Data Ingest Session resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual MBS User Data Ingest Session resource.

tags:

- Individual MBS User Data Ingest Session (Document)

operationId: RetrieveIndMBSUserDataIngSession

responses:

'200':

description: >

OK. The requested Individual MBS User Data Ingest Session resource is successfully

returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MBSUserDataIngSession'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual MBS User Data Ingest Session resource.

tags:

- Individual MBS User Data Ingest Session (Document)

operationId: UpdateIndMBSUserDataIngSession

requestBody:

description: >

Contains the updated representation of the Individual MBS User Data Ingest Session

resource.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MBSUserDataIngSession'

responses:

'200':

description: >

OK. The concerned Individual MBS User Data Ingest Session resource is successfully

updated and a representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MBSUserDataIngSession'

'204':

description: >

No Content. The concerned Individual MBS User Data Ingest Session resource is

successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual MBS User Data Ingest Session resource.

tags:

- Individual MBS User Data Ingest Session (Document)

operationId: ModifyIndMBSUserDataIngSession

requestBody:

description: >

Contains the parameters to request the modification of the Individual MBS User Data Ingest

Session resource.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/MBSUserDataIngSessionPatch'

responses:

'200':

description: >

OK. The concerned Individual MBS User Data Ingest Session resource is successfully

modified and a representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MBSUserDataIngSession'

'204':

description: >

No Content. The concerned Individual MBS User Data Ingest Session resource is

successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual MBS User Data Ingest Session resource.

tags:

- Individual MBS User Data Ingest Session (Document)

operationId: DeleteIndMBSUserDataIngSession

responses:

'204':

description: >

No Content. The Individual MBS User Data Ingest Session resource is successfully

deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/status-subscriptions:

get:

summary: Retrieve all the active MBS User Data Ingest Session Status Subscription resources managed by the MBSF.

tags:

- MBS User Data Ingest Session Status Subscriptions (Collection)

operationId: RetrieveMBSUserDataIngStatSubscs

responses:

'200':

description: >

OK. All the active MBS User Data Ingest Session Status Subscriptions managed by the MBSF

are returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/MBSUserDataIngStatSubsc'

minItems: 0

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

post:

summary: Request the creation of a new MBS User Data Ingest Session Status Subscription.

tags:

- MBS User Data Ingest Session Status Subscriptions (Collection)

operationId: CreateMBSUserDataIngStatSubsc

requestBody:

description: >

Contains the parameters to request the creation of a new MBS User Data Ingest Session

Status Subscription.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MBSUserDataIngStatSubsc'

responses:

'201':

description: >

Created. Successful creation of a new Individual MBS User Data Ingest Session

Status Subscription resource.

content:

application/json:

schema:

$ref: '#/components/schemas/MBSUserDataIngStatSubsc'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/nmbs-mbs-ud-ingest/v1/status-subscriptions/{subscriptionId}

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

mbsUserDataIngestSessionStatusNotif:

'{request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MBSUserDataIngStatNotif'

responses:

'204':

description: No Content. Successful reception of the notification.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/status-subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Identifier of the Individual MBS User Data Ingest Session Status Subscription resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual MBS User Data Ingest Session Status Subscription resource.

tags:

- Individual MBS User Data Ingest Session Status Subscription (Document)

operationId: RetrieveIndMBSUserDataIngStatSubsc

responses:

'200':

description: >

OK. Successful retrieval of the requested Individual MBS User Data Ingest Session

Status Subscription resource.

content:

application/json:

schema:

$ref: '#/components/schemas/MBSUserDataIngStatSubsc'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual MBS User Data Ingest Session Status Subscription resource.

tags:

- Individual MBS User Data Ingest Session Status Subscription (Document)

operationId: UpdateIndMBSUserDataIngStatSubsc

requestBody:

description: >

Contains the updated representation of the Individual MBS User Data Ingest Session Status

Subscription resource.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MBSUserDataIngStatSubsc'

responses:

'200':

description: >

OK. The concerned Individual MBS User Data Ingest Session Status Subscription resource

is successfully updated and a representation of the updated resource is returned in the

response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MBSUserDataIngStatSubsc'

'204':

description: >

No Content. The concerned Individual MBS User Data Ingest Session Status Subscription

resource is successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual MBS User Data Ingest Session Status Subscription resource.

tags:

- Individual MBS User Data Ingest Session Status Subscription (Document)

operationId: ModifyIndMBSUserDataIngStatSubsc

requestBody:

description: >

Contains the parameters to request the modification of the Individual MBS User Data Ingest

Session Status Subscription resource.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/MBSUserDataIngStatSubscPatch'

responses:

'200':

description: >

OK. The concerned Individual MBS User Data Ingest Session Status Subscription resource

is successfully modified and a representation of the updated resource is returned in the

response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MBSUserDataIngStatSubsc'

'204':

description: >

No Content. The concerned Individual MBS User Data Ingest Session Status Subscription

resource is successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual MBS User Data Ingest Session Status Subscription resource.

tags:

- Individual MBS User Data Ingest Session Status Subscription (Document)

operationId: DeleteMBSUserDataIngStatSubsc

responses:

'204':

description: >

No Content. Successful deletion of the existing Individual MBS User Data Ingest Session

Status Subscription resource.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

description: >

For trusted MBSF, the 'nmbsf-mbs-ud-ingest' shall be used as 'scopes' and

'{nrfApiRoot}/oauth2/token' shall be used as 'tokenUri'.

#

# STRUCTURED DATA TYPES

#

schemas:

MBSUserDataIngSession:

description: Represents MBS User Data Ingest Session information.

type: object

properties:

mbsUserServId:

type: string

mbsDisSessInfos:

type: object

additionalProperties:

$ref: '#/components/schemas/MBSDistributionSessionInfo'

minProperties: 1

description: >

Represents one or more MBS Distribution Session(s) composing the MBS User Data Ingest

Session. The key of the map shall be set to the value ofthe "mbsDistSessionId" attribute

of the MBSDistributionSessionInfo data structure encoding the corresponding map entry.

actPeriods:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

minItems: 1

mbsUserServAnmt:

$ref: '#/components/schemas/MBSUserServAnmt'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- mbsUserServId

- mbsDisSessInfos

MBSDistributionSessionInfo:

description: Represents MBS Distribution Session information.

type: object

properties:

mbsDistSessionId:

type: string

mbsSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSessionId'

mbsServInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsServiceInfo'

maxContBitRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

maxContDelay:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketDelBudget'

distrMethod:

$ref: '#/components/schemas/DistributionMethod'

fecConfig:

$ref: '#/components/schemas/FECConfig'

objDistrInfo:

$ref: '#/components/schemas/ObjectDistrMethInfo'

pckDistrInfo:

$ref: '#/components/schemas/PacketDistrMethInfo'

trafficMarkingInfo:

type: string

mbsDistSessState:

$ref: 'TS29581_Nmbstf_DistSession.yaml#/components/schemas/DistSessionState'

tgtServAreas:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsServiceArea'

extTgtServAreas:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ExternalMbsServiceArea'

mbsFSAId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsFsaId'

locationDependent:

type: boolean

description: >

Represents an indication that this MBS Distribution Session belongs to a location-

dependent MBS. This attribute shall be set to "true" to indicate that the MBS

Distribution Session belongs to a location-dependent MBS; or set to "false" to

indicate that the MBS Distribution Session does not belong to a location-dependent MBS.

The default value is "false", if omitted.

default: false

multiplexedServFlag:

type: boolean

description: >

Represents an indication that this MBS Distribution Session belongs to a multiplex, i.e.

forms part of a set of MBS Distribution Sessions under the same parent MBS User Data

Ingest Session with identical or empty sets of target service areas and multiplexed onto

the same MBS Session at the MB-SMF.

default: false

restrictedFlag:

type: boolean

description: >

Represents an indication that this MBS Distribution Session is not open to any UE, i.e.

restricted to a set of UEs according to their MBS related subscription information.

This attribute may be included only if the parent MBS User Service is of Multicast

service type. This attribute shall be set to "true" to indicate that this MBS

Distribution Session is restricted to a set of UE(s); or set to "false" to indicate that

this MBS Distribution Session is open to any UE.

The default value is "false", if omitted.

default: false

required:

- distrMethod

- maxContBitRate

MBSUserDataIngSessionPatch:

description: >

Represents the requested modifications to an MBS User Data Ingest Session Status

Subscription.

type: object

properties:

mbsDisSessInfos:

type: object

additionalProperties:

$ref: '#/components/schemas/MBSDistributionSessionInfo'

minProperties: 1

description: >

Contains the requested modifications to one or more MBS Distribution Session(s)

composing the MBS User Data Ingest Session.

actPeriods:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

minItems: 1

ObjectDistrMethInfo:

description: >

Represents additional MBS Distribution Session parameters for the case of an Object

Distribution Method.

type: object

properties:

operatingMode:

$ref: 'TS29581_Nmbstf_DistSession.yaml#/components/schemas/ObjDistributionOperatingMode'

objAcqMethod:

$ref: 'TS29581_Nmbstf_DistSession.yaml#/components/schemas/ObjAcquisitionMethod'

objAcqIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

minItems: 1

objIngUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

objDistrUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

objRepairUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

required:

- operatingMode

- objAcqMethod

- objAcqIds

PacketDistrMethInfo:

description: >

Represents additional MBS Distribution Session parameters for the case of Packet

Distribution Method.

type: object

properties:

operatingMode:

$ref: 'TS29581_Nmbstf_DistSession.yaml#/components/schemas/PktDistributionOperatingMode'

pckIngMethod:

$ref: 'TS29581_Nmbstf_DistSession.yaml#/components/schemas/PktIngestMethod'

ingEndpointAddrs:

$ref: 'TS29581_Nmbstf_DistSession.yaml#/components/schemas/MbStfIngestAddr'

required:

- operatingMode

- pckIngMethod

- ingEndpointAddrs

MBSUserDataIngStatSubsc:

description: >

Represents an MBS User Data Ingest Session Status Subscription.

type: object

properties:

mbsIngSessionId:

type: string

eventSubscs:

type: array

items:

$ref: '#/components/schemas/SubscribedEvent'

minItems: 1

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

required:

- mbsIngSessionId

- eventSubscs

- notifUri

MBSUserDataIngStatSubscPatch:

description: >

Represents the requested modifications to an MBS User Data Ingest Session Status

Subscription.

type: object

properties:

eventSubscs:

type: array

items:

$ref: '#/components/schemas/SubscribedEvent'

minItems: 1

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

SubscribedEvent:

description: >

Represents a subscribed MBS User Data Ingest Session Status event and the related

information.

type: object

properties:

statusEvent:

$ref: '#/components/schemas/Event'

mbsDistSessionId:

type: string

required:

- statusEvent

MBSUserDataIngStatNotif:

description: >

Represents an MBS User Data Ingest Session Status Notification.

type: object

properties:

mbsIngSessionId:

type: string

eventNotifs:

type: array

items:

$ref: '#/components/schemas/EventNotification'

minItems: 1

required:

- mbsIngSessionId

- eventNotifs

EventNotification:

description: Represents Event Notification.

type: object

properties:

statusEvent:

$ref: '#/components/schemas/Event'

mbsDisSessionId:

type: string

statusAddInfo:

type: string

timeStamp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

required:

- statusEvent

- timeStamp

MBSUserServAnmt:

description: >

Represents the MBS User Service Announcement currently associated with the MBS User Data

Ingest Session.

type: object

properties:

extServiceId:

type: array

items:

type: string

minItems: 1

servClass:

type: string

startTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

endTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

servNameDescs:

type: array

items:

$ref: 'TS29580_Nmbsf_MBSUserService.yaml#/components/schemas/ServiceNameDescription'

minItems: 1

mainServLang:

type: string

mbsDistSessAnmt:

additionalProperties:

$ref: '#/components/schemas/MBSDistSessionAnmt'

minProperties: 1

description: >

Represents the set of MBS Distribution Session Announcements currently associated with

this MBS User Service Announcement.

required:

- extServiceId

- servClass

- servNameDescs

MBSDistSessionAnmt:

description: >

Represents the set of MBS Distribution Session Announcements currently associated with this

MBS User Service Announcement.

type: object

properties:

mbsSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSessionId'

mbsFSAId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsFsaId'

distrMethod:

$ref: '#/components/schemas/DistributionMethod'

objDistrAnnInfo:

$ref: '#/components/schemas/ObjectDistMethAnmtInfo'

sesDesInfo:

type: array

items:

type: string

minItems: 1

required:

- distrMethod

- sesDesInfo

ObjectDistMethAnmtInfo:

description: >

Represents MBS Distribution Session Announcement parameters for Object Distribution Method.

type: object

properties:

objDistrSched:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

objDistrBaseUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

objRepBaseUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

FECConfig:

description: Represents FEC configuration information.

type: object

properties:

fecScheme:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

fecOverHead:

type: integer

additionalParams:

type: array

items:

$ref: '#/components/schemas/AddFecParams'

minItems: 1

required:

- fecScheme

- fecOverHead

AddFecParams:

description: Represents additional scheme-specific parameters for AL-FEC configuration.

type: object

properties:

paramName:

type: string

paramValue:

type: string

required:

- paramName

- paramValue

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

DistributionMethod:

anyOf:

- type: string

enum:

- OBJECT

- PACKET

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Possible values are:

- OBJECT: Indicates the Object Distribution Method.

- PACKET: Indicates the Packet Distribution Method.

Event:

anyOf:

- type: string

enum:

- USER_DATA_ING_SESS_STARTING

- USER_DATA_ING_SESS_STARTED

- USER_DATA_ING_SESS_TERMINATED

- DIST_SESS_STARTING

- DIST_SESS_STARTED

- DIST_SESS_TERMINATED

- DIST_SESS_SERV_MNGT_FAILURE

- DIST_SESS_POL_CRTL_FAILURE

- DATA_INGEST_FAILURE

- DELIVERY_STARTED

- SESSION_TERMINATED

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Possible values are:

- USER_DATA_ING_SESS_STARTING: >

Indicates that the MBS User Data Ingest Session is starting. This is an "MBS User Data

Ingest Session" level event.

- USER_DATA_ING_SESS_STARTED: >

Indicates that the MBS User Data Ingest Session started. This is an "MBS User Data

Ingest Session" level event.

- USER_DATA_ING_SESS_TERMINATED: >

Indicates that the MBS User Data Ingest Session is terminated. This is an "MBS User Data

Ingest Session" level event.

- DIST_SESS_STARTING: >

Indicates that the MBS Distribution Session is starting. This is an "MBS Distribution

Session" level event.

- DIST_SESS_STARTED: >

Indicates that the MBS Distribution Session started. This is an "MBS Distribution

Session" level event.

- DIST_SESS_TERMINATED: >

Indicates that the MBS Distribution Session is terminated. This is an "MBS Distribution

Session" level event.

- DIST_SESS_SERV_MNGT_FAILURE: >

Indicates that the MBS Distribution Session could not be started (e.g. the necessary

resources could not be allocated by the MBS system). This is an "MBS Distribution

Session" level event.

- DIST_SESS_POL_CRTL_FAILURE: >

Indicates that the MBS Distribution Session could not be started because of a policy

authorization/control failure or rejection. This is an "MBS Distribution Session"

level event.

- DATA_INGEST_FAILURE: >

The MBS User Data Ingest is failed because the MBSTF is expecting data (the MBS Session

is active), but not receiving it. This is an "MBS Distribution Session" level event.

- DELIVERY_STARTED: >

The MBS User Data delivery is started.

- SESSION_TERMINATED: >

The MBS User Data Ingest Session is terminated.

Annex B (informative):
Withdrawn API versions

# B.1	General

This Annex lists withdrawn API versions of the APIs defined in the present specification. Clause 4.3.1.6 of 3GPP TS 29.501 [5] describes the withdrawal of API versions.

# B.2	Nmbsf_MBSUserService API

The API versions listed in table B.2-1 are withdrawn for the Nmbsf_MBSUserService API.

Table B.2-1: Withdrawn API versions of the Nmbsf_MBSUserService service

| API version number | Remarks |
| --- | --- |

# B.3	Nmbsf_MBSUserDataIngestSession API

The API versions listed in table B.3-1 are withdrawn for the Nmbsf_MBSUserDataIngestSession API.

Table B.3-1: Withdrawn API versions of the Nmbsf_MBSUserDataIngestSession service

| API version number | Remarks |
| --- | --- |

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-02 | CT3#120-e |  | - | - | - | Skeleton for the new MBSF Services TS | 0.0.0 |
| 2022-02 | CT3#120-e | C3-221312 | - | - | - | Inclusion of C3-221312, C3-221313. | 0.1.0 |
| 2022-04 | CT3#121-e | C3-222484 | - | - | - | Inclusion of C3-222373, C3-222374, C3-222408, C3-222475, C3-222476. | 0.2.0 |
| 2022-05 | CT3#122-e | C3-223507 | - | - | - | Inclusion of C3-223604, C3-223310, C3-223311, C3-223605, C3-223313, C3-223314, C3-223315, C3-223316, C3-223317, C3-223410, C3-223543, C3-223412, C3-223544, C3-223545, C3-223546, C3-223547, C3-223417, C3-223418, C3-223419, C3-223745, C3-223421, C3-223422, C3-223423, C3-223747, C3-223748, C3-223750. | 0.3.0 |
| 2022-06 | CT#96 | CP-221098 |  |  |  | Presentation to TSG CT for information | 1.0.0 |
| 2022-09 | CT#97e |  |  |  |  | Inclusion of C3-224776, C3-224495, C3-224387, C3-224653, C3-224654, C3-224712, C3-224713, C3-224438, C3-224439 | 1.0.1 |
| 2022-09 | CT#97e | CP-222130 |  |  |  | Presentation to TSG CT for approval | 2.0.0 |
| 2022-09 | CT#97e | CP-222130 |  |  |  | Approved by TSG CT | 17.0.0 |
| 2022-12 | CT#98e | CP-223167 | 0001 | 1 | F | attribute and Misc corrections in the description and data model clause in Nmbsf_MBSUserDataIngestSession service | 17.1.0 |
| 2022-12 | CT#98e | CP-223166 | 0002 | - | F | Data type Cardinality corrections for GET response in Nmbsf_MBSUserDataIngestSession API | 17.1.0 |
| 2022-12 | CT#98e | CP-223166 | 0003 | - | F | Data type Cardinality corrections for GET response in Nmbsf_MBSUserService Service API | 17.1.0 |
| 2022-12 | CT#98e | CP-223166 | 0005 | - | F | Corrections on MBS User Data Ingest Session Status Subscription Update | 17.1.0 |
| 2022-12 | CT#98e | CP-223166 | 0006 | - | F | Correct the Cardinality of the FECConfig definition | 17.1.0 |
| 2022-12 | CT#98e | CP-223167 | 0007 | 1 | F | Enumeration and data type definitions in the OpenAPI files | 17.1.0 |
| 2022-12 | CT#98e | CP-223166 | 0008 | - | F | Correction to content type of Nmbsf service | 17.1.0 |
| 2022-12 | CT#98e | CP-223167 | 0009 | - | F | Corrections on MBS User Data Ingest Session Status subscribed events | 17.1.0 |
| 2022-12 | CT#98e | CP-223167 | 0010 | 1 | F | Correct the attribute names | 17.1.0 |
| 2022-12 | CT#98e | CP-223188 | 0014 | - | F | Update of info and externalDocs fields | 17.1.0 |