| 3GPP TR 29.889 V0.3.0 (2024-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Study on Protocol for AI Data Collection from UPF; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Architectural Requirements	8

4.1	Overview of the UPF data collection architecture	8

4.2	Architectural Requirements	10

4A	Use cases and protocol requirements for intensive data collection from UPF	10

4A.1	Use cases for intensive data collection from UPF	10

4A.1.1	General	10

4A.1.2	Example use cases for intensive data collection from UPF	11

4A.1.2.1	User-level experience data reporting for a group of users	11

4A.2	Protocol requirements for intensive data collection from UPF	11

5	Key Issues	12

5.1	Key Issue#1: Identifying and lowering the network performance impacts of intensive data collection from UPF	12

5.1.1	General	12

5.1.2	Key Issue Definition	12

5.2	Key Issue#2: Data collection protocol selection	12

5.2.1	General	12

5.2.2	Key Issue Definition	13

6	Solution	13

6.0	Mapping of Solutions to Key Issues	13

6.1	Solution #1: Reduced Report Instructions Information	13

6.1.1	Description	13

6.1.2	Procedures	14

6.1.3	Impacts on Existing Services, Entities and Interfaces	15

6.1.4	Pros	15

6.1.5	Cons	15

6.2	Solution #2: on Optimized UPF data collection based on user and/or PDU session’s markers	15

6.2.1	Description	15

6.2.2	Procedures	16

6.2.3	Impacts on services, entities and interfaces	17

6.2.4	Pros	18

6.2.5	Cons	18

6.3	Solution #3: Selection of a UPF Event Reports Delivery Mechanism	18

6.3.1	Description	18

6.3.2	Procedures	19

6.3.3	Impacts on services, entities and interfaces	20

6.3.4	Pros	21

6.3.5	Cons	21

6.4	Solution #4: Optimized UPF data collection based on existing 5G VN group mechanisms, PDU session type or RAT type	21

6.4.1	Description	21

6.4.2	Procedures	22

6.4.3	Impacts on services, entities and interfaces	22

6.4.4	Pros	23

6.4.5	Cons	23

6.5	Solution #5: Dynamic provisioning of user and/or PDU session’s markers	23

6.5.1	Description	23

6.5.2	Procedures	23

6.5.3	Impacts on services, entities and interfaces	25

6.5.4	Pros	26

6.5.5	Cons	26

6.6	Solution #6: Alternative protocol DRP+UDP on Data collection	26

6.6.1	Description	26

6.6.2	Impacts on services, entities and interfaces	27

6.6.3	Pros	28

6.6.4	Cons	28

7	Evaluation	28

8	Conclusion	28

8.1	Conclusions of KI#1	28

Annex A (informative): Change history	28

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In drafting the TS/TR, pay particular attention to the use of modal auxiliary verbs! TRs shall not contain any normative provisions.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document aims at studying UPF data Collection for AI/ML and whether alternative protocols, or enhancements to the existing SBI protocol, are needed to optimize the AI/ML data collection while ensuring secure, scalable and reliable data transfers across the core network identifies. The present document will focus on the following aspects:

-	To identify the use cases for UPF data Collection for AI/ML where current mechanisms do not suffice.

-	To study on the potential optimized protocol solutions for efficient AI/ML data collection.

-	To study on the mechanisms to optimize the protocol selection when different protocols exist.

NOTE 1:	Data collection from 5GC NFs other than UPF is out of scope of this study.

NOTE 2:	The study remains within the limits of the existing 5GC architecture and stage 2 requirements. Only the protocol for collecting the data is the object of this study.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 29.564: "User Plane Function Services; Stage 3".

[6]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[7]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[10]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AF	Application Function

DCCF	Data Collection Coordination Function

MFAF	Messaging Framework Adaptor Function

NEF	Network Exposure Function

NWDAF	Network Data Analytics Function

UPF	User Plane Function

SMF	Session Management Function

TSCTSF	Time Sensitive Communication and Time Synchronization Function

TSN	Time Sensitive Networking

4	Architectural Requirements

4.1	Overview of the UPF data collection architecture

This clause provides an overview of the architecture and mechanisms supported to collect data from the UPF:

1)	The UPF can provide event exposure via indirect subscription by SMF (PFCP or HTTP) or direct subscription by other NF consumers. Three different UPF event subscription/notification modes can be identified as below:

-	Mode 1: NF consumer subscribes the UPF event via SMF using the PFCP protocol via N4 interface as illustrated in Figure 4.1-1.

Figure 4.1-1: UPF event subscription/notification mode 1

-	Mode 2: NF consumer subscribes the UPF event via SMF using the HTTP protocol via the Nupf SBI interface as illustrated in Figure 4.1-2.

Figure 4.1-2: UPF event subscription/notification mode 2

-	Mode 3: NF consumer subscribes the UPF event directly using the HTTP protocol via the Nupf SBI interface as illustrated in Figure 4.1-3.

Figure 4.1-3: UPF event subscription/notification mode 3

2)	As of Rel-18, the UPF supports four types of UPF events, and one specific UPF event subscription/notification mode was specified for each supported event as specified in clause 5.2.1.3 of 3GPP TS 29.564 [5]. Table 4.1-1 provides a summary of the UPF event exposure.

Table 4.1-1: UPF event exposure supported by the Nupf_EventExposure service

| Event Type | Subscription Mode | Report Type | Data consumers |
| --- | --- | --- | --- |
| QoS Monitoring | Mode 1 | Continuous (event triggered) Report Periodic Report | NEF, AF, NWDAF, TSCTSF, TSNAF, DCCF, MFAF |
| User Data Usage Measurement User Data Usage Trends | Mode 2 (Target to a specific UE or a group of UEs; or any UE with AoI, BSSID/SSID or DNAI information) | One-Time Report Periodic Report | NWDAF, DCCF, MFAF |
|  | Mode 3 (Target to any UE without any AoI, BSSID/SSID or DNAI information) |  |  |
| TSC Management Information | Mode 1 | Continuous (event triggered) Report. | TSCTSF, TSNAF |

3)	As defined in clause 6.2.1 of 3GPP TS 23.288 [7]), the following mechanisms can be used by an NF service consumer (e.g. NWDAF) to control the event report signaling from the UPF (NF service producer):

a)	An NF service consumer may reduce the amount of data collected from the UPF to reduce the signaling load by reducing the extent (e.g. duration, scope) of data collection or by providing/modifying sampling ratios defining the random subset of PDU sessions among the target PDU sessions, in which case the UPF only reports the events related to the selected subset of PDU sessions. The UPF may indicate to the NF service consumer the sampling ratio achieved by the UPF; this sampling ratio may change over time, e.g. when the load of the condition varies.

When using sampling ratio, the NWDAF may, depending on the analytics required and based on local configuration, provide additional partitioning criteria to the UPF to allow for a better representation of the PDU sessions and to request that the UPF first partitions the PDU sessions before applying sampling ratio. Up to Rel-18, the partitioning criteria for UPF event exposure include S-NSSAI and DNN.

b)	An NF service consumer subscribing to UPF event exposure can provide Reporting suggestion information to the UPF, including reporting urgency information indicating whether the event reporting can be delayed (i.e. it is delay-tolerant) and if so, the reporting time information defining the last valid reporting time for the UPF to report the detected event. This mechanism can help spreading over time (e.g. during periods of lower traffic) the reporting of delay-tolerant events and thus smoothing the load of the UPF over time.

c)	An NF service consumer may request the UPF to mute event notifications for a subscription, i.e. to collect, store the requested events but halt the notification to the NF service consumer (see clause 6.2.7 of 3GPP TS 23.288 [7]). The number of stored events may be limited based on the UPF configuration; when this threshold is exceeded or another exception occurs (e.g. buffering capacity or duration exceeded), the UPF may send all stored events, discard all stored events or discard some older events as instructed by the muting exception handling instructions received from the NF service consumer. This mechanism can help an NF service consumer to reduce temporarily the signaling from the UPF, e.g. in periods of high load, while allowing the NF service consumer to retrieve the stored events e.g. in periods of lower load.

4.2	Architectural Requirements

The solutions studied in the present study shall comply with the stage 2 architecture and requirements specified in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3], and 3GPP TS 23.503 [4]. Specifically, the proposed solution shall build on the 5GS UPF event subscription and notification principles as specified in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3] and 3GPP TS 29.564 [5].

The N4 and Nupf interface shall be based on the existing interface design.

Solutions developed in this TR shall be backward compatible.

4A	Use cases and protocol requirements for intensive data collection from UPF

4A.1	Use cases for intensive data collection from UPF

Editor's note: this clause should describe the use cases for data collection from UPF, focusing in particular on use cases with intensive data collection for which the current delivery mechanism of the Nupf_EventExposure service is deemed insufficient. It should be described, for each use case, whether event reporting is delay-tolerant or delay-critical.

4A.1.1	General

The NF (e.g. NWDAF or DCCF) retrieves data from the UPF and from other data sources (e.g. other 5GC NFs, OAM) to generate analytic outputs, i.e. statistics or predictions, as described in 3GPP TS 23.288 [7]. This can be used e.g. for network performance monitoring or to identify in advance potential network failure or network performance degradation.

3GPP TS 23.288 [7] identifies the following use cases relying on data exposed by the UPF:

-	Observed Service Experience related network data analytics (see clause 6.4 of 3GPP TS 23.288 [7]), e.g. to provide outputs on Service Experience for a Network Slice, an Application, Application transferring data over a PDU Session.

-	NF load analytics (see clause 6.5 of 3GPP TS 23.288 [7]);

-	UE Communication Analytics (see clause 6.7.3 of 3GPP TS 23.288 [7]);

-	User Data Congestion Analytics (see clause 6.8 of 3GPP TS 23.288 [7]);

-	Dispersion Analytics (see clause 6.10 of 3GPP TS 23.288 [7]);

-	WLAN performance analytics (see clause 6.11 of 3GPP TS 23.288 [7]);

-	PFD Determination Analytics (see clause 6. 16 of 3GPP TS 23.288 [7]);

-	End-to-end data volume transfer time analytics (see clause 6. 18 of 3GPP TS 23.288 [7]);

-	PDU Session traffic analytics (see clause 6. 20 of 3GPP TS 23.288 [7]).

3GPP TS 23.288 [7] also identifies use cases relying on 5G performance measurements and 5G e2e Key Performance Indicators (KPIs) (see 3GPP TS 28.552 [8] and 3GPP TS 28.554 [9]) reported by the UPF to OAM, e.g. for QoS Sustainability Analytics in clause 6.9 of 3GPP TS 23.288 [7].

4A.1.2	Example use cases for intensive data collection from UPF

4A.1.2.1	User-level experience data reporting for a group of users

The user experience data of all users of a certain group (e.g. VIP, gold, silver or bronze users) may be collected with a given frequency, e.g. every 5 minutes, while other users need only to be sampled to collect user experience data with a smaller frequency. For PLMNs with a large number of subscribers, this can result in a very large number of subscriptions at the UPF and a high frequency of event notifications by the UPF.

The term "all users of a certain group" mentioned above can includes a range of approaches for categorizing, e.g.,

- Categorizing a group of users by membership level defined by operator, e.g., VIP, gold, silver or bronze users.

- Categorizing users according to the specific services provided by 5GC, e.g., NR_REDCAP, 5G-LAN type service.

- Categorizing a group of users by the user’s PDU session type or RAT type, e.g., RAT type, Ethernet PDU session type.

- Categorizing a group of users which are located in the specified location, represented by areas of validity.

Editor's note: It should be described for the above use case whether event reports are delay-critical or delay-tolerant. If the event reports are delay-critical, it should be further described why the event reports are so (e.g. provide an example of how event reports are used) and the magnitude of the related delay/latency requirement.

4A.2	Protocol requirements for intensive data collection from UPF

Editor's note: this clause will capture protocol requirements for intensive data collection from UPF. It should consider the different use cases identified in clause 4A.1, including whether event reporting is delay-tolerant or delay-critical.

Editor's note: some reporting solutions/protocols have already been defined by other 3GPP WGs to collect data from data producers. It should be studied whether these solutions/protocols might be suitable for UPF data collection. See e.g. clause 11 of TS 28.104 (Management and orchestration; Management Data Analytics) and the streaming data and file data reporting services in clauses 11.5, 11.6, 12.5 and 12.6 of TS 28.532 (Management and orchestration; Generic management services) which define a solution using WebSocket for reporting streaming data and a solution for notifying and retrieving new data files.

5	Key Issues

5.1	Key Issue#1: Identifying and lowering the network performance impacts of intensive data collection from UPF

5.1.1	General

The service-based architecture in 5G facilitates flexible information exchange between network functions via standardized Service Based Interfaces (SBI). This definition provides a suitable framework for 5G system especially the AI/ML functions (e.g. NWDAF) to collect required data from various sources.

However, as AI/ML adoption grows for 5G use cases like network automation, analytics and others, which like results in massive network signalling messages, including both event subscription creation/modification/deletion signalling and the event reports generated for the corresponding subscriptions, this massive signalling will consume considerable processing and memory in the UPF, which downgrades the UPF performance/capability for packets forwarding which is the main task of the UPF.

Therefore it is proposed to study potential optimisations of the Nupf_EventExposure service which aim at reducing the network signalling and the processing required for events subscription and events reporting, improving the quality of event reports which do help the business logic. For use cases where the subscription triggers the UPF to generate very frequent event reports with a large volume, alternative protocols or enhancements to the existing protocol to deliver those event reports may be investigated.

5.1.2	Key Issue Definition

This Key Issue will study the following aspects:

-	Identify the use cases which leads extra signalling and unnecessary processing load and study corresponding potential optimisations.

-	Study whether the existing protocol can be enhanced to reduce the signaling and processing required to subscribe to the UPF for UPF data collection;

-	Identify the use cases where very frequent event reports with a large volume will be generated whichsignificantly impacts the UPF performance, and if so:

-	Study whether the existing protocol can be enhanced to reduce the signaling and processing required to report UPF data;

-	Study whether alternative protocols for collecting the data from the UPF may help enhancing the performances compared to the existing protocol.

5.2	Key Issue#2: Data collection protocol selection

5.2.1	General

As AI/ML adoption grows for 5G use cases like network automation, analytics and others, various NF service consumers (including those indirect service consumer) for Nupf_EventExposure service, for different application traffic and different use cases, may prefer to use different protocols to request the UPF to deliver those event reports to the relevant NF service consumer in more efficient ways, while allowing the UPF to send the event reports in a single format, i.e. using the existing JSON object "NotificationData" to avoid transcoding between SBI and other protocols.

However, per existing specification, a NF service consumer has only one alternative to provide an eventNotifyUri URI together with a notifyCorrelationId to receive the event reports; and the UPF, as the NF service producer can only send the report to this Uri together with the correlationId using the HTTP POST as specified in clause 5.2.2.3 of 3GPP TS 29.564 [5].

Based on the study of Key Issue #1, the original SBI based protocol and one or more alternative protocols may be possible to be used between data collector (e.g. NWDAF) and the UPF. After the NF consumer subscribes to UPF events, the UPF needs to have enough information to decide which protocol shall be used to fulfil the data collection requirement from NF consumer. So, this KI targets to study the data collection protocol selection and possible enhancement to UPF event exposure for the data collection protocol selection.

5.2.2	Key Issue Definition

This Key Issue will study the following aspects:

-	Whether and how to negotiate the supported and the selected data collection protocol between the NF service consumer (of Nupf_EventExposure service) and the UPF. The data collection protocols can be used for delivering the event reports generated for the given subscription based on the existing SBI framework.

-	Whether and how UPF event subscription and notification procedures need to be enhanced to support the data collection protocol selection.

6	Solution

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | 1 | 2 |
| 1 | X |  |
| 2 | X |  |
| 3 |  | X |
| 4 | X |  |
| 5 | X |  |
| 6 | X |  |

6.1	Solution #1: Reduced Report Instructions Information

6.1.1	Description

The solution addresses the Key Issue #1 on Identifying and lowering the network performance impacts of intensive data collection from UPF.

3GPP specifies two UPF events User Data Usage Measures and User Data Usage Trends in Rel-18, where the UPF Event Triggers for these two events is defined with either one-time or periodic.

In many scenarios, receiving an event report where the measurement is "zero" byte is not useful to the NF service consumer, i.e. such report does not result any business logic, especially receiving those "zero" measurement reports continuously when UPF Event trigger is defined as periodic reporting results in extra network signalling, extra processing load in both NF service consumer and NF service producer handle such signaling message. In certain scenarios, it may be further preferred to only receive measurements exceeding a certain threshold, i.e. to only receive measurements for applications that cause a high traffic in the network.

Different applications may have different characteristics, e.g. some application may only be mostly used in some specific time periods during a day or a week, or different NF service consumers may only have interest to collect the measurement reports for certain time period. To create or delete an event subscription according to the time schedule adds extra network signaling and complexities in the NF service consumer.

This solution provides an optimization for the above issues.

The solution introduces a reduced report instruction information which is included by the NF service consumer (of the Nupf_EventExposure service) in a subscription request, to instruct the UPF to skip event reports when one of conditions specified in the reduced report instruction information) is met.

The reduced report instruction information includes a set of conditions: e.g.,

-	to skip the reporting if the measurement is zero;

-	to skip the reporting, if the measurement does not exceed a threshold (provided as part of the subscription parameters);

-	an array of validity times so that the UPF shall perform measurement and send event reports only during these time periods.

NOTE 1:	The zero byte need not be absolute zero, it is up to operator's configuration per DNN or per application.

NOTE 2:	Details of new datatype(s) would be specified during the normative phase, if the solution is retained.

The NF service consumer may infer from the absence of event reports that the measurements requested by the subscription are null (or below the threshold). For delay-tolerant event reporting (i.e. when the Reporting Suggestion information indicates that it is possible to defer the sending of the events till the reporting time information), the NF service consumer may infer so, after the reporting time information, when no event reports are received before the reporting time information.

The NF service consumer assumes that there is no measurement report eligible to be reported, e.g. zero-measurement, if it does not receive a report at the scheduled time. For exceptional failure scenario in the UPF, the NF service consumer may monitor the NF status of the UPF or use an implementation specific mechanism, e.g. check if there is any event report from the same UPF, to avoid doing wrong inferences due to the absence of event reports.

6.1.2	Procedures

The solution includes the following steps as illustrate in Figure 6.1.2-1:

1.	The NF consumer includes reduced report instruction information in an event subscription request message, where the reduced report instruction information contains a set of conditions, e.g.  to request the UPF, as a service producer to skip the event report when the condition is met. The intermediate NF, i.e. the SMF, transparently forwards the reduced report instruction information to the UPF.

2.	The UPF creates event subscription resource if the request is accepted and sends 201 Created or returns relevant error as specified in clause 5.2.2.2.2 of 3GPP TS 29.564 [5].

3. 	The UPF performs relevant measurement and send an event report according to the UPF Event Triggers and the reduced report instruction information.

Figure 6.1.2-1

6.1.3	Impacts on Existing Services, Entities and Interfaces

This solution has impacts on the UPF, NF service consumer (of Nupf_EventExposure service), e.g. the SMF and NWDAF, and Nsmf_EventExposure service.

6.1.4	Pros

The solution enables NF service consumer to instruct the NF service producer, i.e. the UPF, to skip sending event reports according to the Reduced Report Instructions Information, so to avoid extra or unnecessary network signaling to send those meaningless event reports, and extra processing load in the UPF and NF consumer to send and to receive those signalling messages, and extra energy consumption.

6.1.5	Cons

FFS

6.2	Solution #2: on Optimized UPF data collection based on PDU session markers

6.2.1	Description

The solution addresses the Key Issue #1 on Identifying and lowering the network performance impacts of intensive data collection from UPF.

The UPF exposes data to other NFs (e.g. NWDAF, AF) using the Nupf_EventExposure service defined in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3] and 3GPP TS 29.564 [5]. The NF service consumer (e.g. SMF, NWDAF) subscribes to this service as described in 3GPP TS 29.564 [5]. The subscription may target ANY UE or a specific UE's PDU session identified by its IP address. For each subscription, the UPF then performs the required measurements (e.g. user data usage or user data trends, see 3GPP TS 29.564 [5]) and sends event reports, per subscription, reporting the measurements.

In several use cases, the operator needs to collect frequent User Plane measurements for PDU sessions or users having specific characteristics (e.g. specific subscriber category such as VIP or gold/silver/bronze users). The only way to achieve this with the current Nupf_EventExposure service design is to create an event exposure subscription for each such individual PDU session (e.g. of each VIP user), which can result in a very large number of subscriptions at the UPF and to lots of signaling to create (and terminate) these subscriptions. Furthermore, the UPF reports measurements per subscription, causing massive signaling (i.e. very high number of HTTP requests and responses) for reporting the measurements from all such PDU sessions (e.g. from VIP users).

EXAMPLE 1: Providing user-level experience data reporting

The user experience data for some VIP users (or gold, silver or bronze users) may be collected with a given frequency, e.g. every 5 minutes, while other users need only to be sampled to collect user experience data with a smaller frequency. For PLMNs with a large number of subscribers, this can result in a very large number of subscriptions at the UPF and a high frequency of event notifications by the UPF.

EXAMPLE 2: Providing service-level experience data reporting

The experience data of users who have signed up for VIP application services, or who have subscribed to/are using specific services, may be collected every minute. For PLMNs with a large number of subscribers, this can result in a very large number of subscriptions at the UPF and a high frequency of event notifications by the UPF.

The solution relies on enhancing the Nupf_EventExposure to support the creation of a UPF event exposure subscription targeting all the PDU sessions associated with operator defined markers.

The solution relies on the existing SBI protocol for the Nupf_EventExposure.

NOTE 1:	The principles of the solution can also be supported if a different protocol was defined for data collection from the UPF.

Principles of the solution:

1)	The solution enables operators to associate operator defined markers to PDU sessions -at the PCF (policy data). For instance, a marker may be associated with PDU sessions of users of a group, of an enterprise, of a specific subscriber category (e.g. VIP user, or gold/silver/bronze users) or users who have subscribed to a specific package or who are using specific services, or users who are roaming or not roaming. Each marker may be defined as a combination of a marker type/ID and marker value, or simply as a marker value.

2)	For a PDU session associated with such markers, the SMF provides these markers to the UPF, during the PFCP session establishment or modification. The UPF stores the PDU session marker in its PFCP session context.

3)	The UPF Event Exposure is enhanced to enable an NF service consumer to create an event exposure subscription at the UPF targeting all PDU sessions matching specific PDU sessions' markers (the subscription may contain an "AND" or an "OR" combination of markers, e.g. to target PDU sessions matching the markers A and B, or PDU sessions matching the markers A or B or C). The NF service consumer is configured with the markers to use for subscribing to the UPF.

NOTE 2:	The solution #5 (which complements solution #2) provides support for dynamic provisioning of PDU session markers by an NF service consumer via the NEF.

4)	The UPF reports the measurements of such PDU sessions (up to the maximum size of the message) in the same notification message towards the NF service consumer (the message indicates to which PDU session each measurement applies).

A marker is a tag associated to PDU sessions sharing one or more (same) specific characteristics, that is stored by the UPF (as an abstract value, i.e. without the UPF knowing the semantic of the marker) in the PFCP session contexts of corresponding PDU sessions, and that can be used in subscriptions to UPF event exposure to target all the PDU sessions associated with the marker.

NOTE 3:	This enables to subscribe to UPF event exposure for a set of PDU sessions sharing specific characteristics without the UPF having to receive, for all PDU sessions, any information about these characteristics.

An example marker is shown below.

EXAMPLE: Operator defining a marker for VIP users

Marker type/ID: USER_CATEGORY, Marker value: VIP

or alternatively just a Marker value: VIP

The NWDAF can then subscribe to the UPF with a new marker criterion set with the above Marker information.

6.2.2	Procedures

Figure 6.2.2-1 depicts an example call flow enabling an NF service consumer (e.g. NWDAF) to subscribe to UPF event exposure for PDU sessions associated with a specific marker.

Figure 6.2.2-1: Subscribing to UPF event exposure for PDU sessions associated with specific markers

0.	Markers with associated filters may be configured in the policy data in UDR.

1.	During the PDU session establishment, the PCF retrieves policy data from UDR and provides markers applicable to the PDU session to the SMF. The SMF receives the markers with the policy information, if any was configured in the policy data. The PCF updates the SMF about any later change of the marker information in the policy data.

2.	During the establishment of the PFCP session, the SMF provides the markers associated with the PDU session (i.e. the markers received in step 1, if any) to the UPF. The SMF updates the UPF about any later change of the marker information.

3.	The NF service consumer (e.g. NWDAF) subscribes to the UPF providing the markers to identify the PDU sessions targeted by the subscription.

NOTE 1:	The NWDAF determines, based on local configuration, which markers may be used to serve Nnwdaf_AnalyticsInfo service requests (based on the Targets of Analytic Reporting and Analytics Filter Information).

NOTE 2:	The solution assumes that the UPF supports UPF event subscriptions with markers. Otherwise, the NF service consumer subscribes to the UPF using the existing UPF event subscription solution (i.e. per PDU session).

4.	The UPF reports the measurements of such PDU sessions (up to the maximum size of the message) in a same notification message towards the NF service consumer.

6.2.3	Impacts on services, entities and interfaces

PCF/UDR APIs:

-	The Npcf_SMPolicyControl service and the Nudr_DataRepository service for Policy Data are extended to enable the retrieval of operator defined markers per user and/or PDU session (new policy data) during the SM policy association establishment and the notifications of markers' updates. The markers may correspond e.g. to the Allowed services and/or Subscriber categories defined in 3GPP TS 23.503 [4].

UPF/SMF:

-	The PFCP session establishment/modification requests are extended to enable the SMF to associate markers with a PFCP session (PDU session).

UPF/SMF/NWDAF:

-	The Nupf_EventExposure service is enhanced to enable an NF service consumer to create an event exposure subscription at the UPF targeting all PDU sessions matching specific PDU sessions' markers.

6.2.4	Pros

The solution provides the following benefits:

-	The solution enables to reduce drastically the number of UPF event exposure subscriptions to be created at the UPF. For instance, to collect UPF data for PDU sessions of VIP users, instead of creating as many subscriptions as there are VIP users, the NF service consumer can create (only once) a single UPF event exposure subscription targeting all the PDU sessions of VIP users.

-	The UPF can report the measurements from PDU sessions matching the desired markers in the same notification message, significantly reducing the number of notification messages (and therefore HTTP requests/responses) required to report the requested data.

-	Processing overhead is reduced as a single subscription can target all the PDU sessions matching operator defined criteria, and since the number of notifications is reduced.

-	The solution is flexible in allowing each operator to define its own PDU sessions' markers, not requiring new standardization work for each possible use case/operator need.

6.2.5	Cons

Editor's note: this is FFS.

The solution has the following drawbacks:

-	The solution requires pre-configuring of the markers in policy data and at the consumer of the UPF event exposure (e.g. NWADF).

NOTE:	The solution #5 (which complements solution #2) provides support for dynamic provisioning of markers via the NEF.

6.3	Solution #3: Selection of a UPF Event Reports Delivery Mechanism

6.3.1	Description

This solution is to address Key Issue#2 Data collection protocol selection.

To collect data, i.e. event reports, from the UPF using the Nupf_EventExposure service as specified in 3GPP TS 23.502 [3], clause 4.15.4.5, the current protocol is to use the HTTP POST to send the report to eventNotifyUri URI together with an notifyCorrelationId provided by the NF service consumer as specified in clause 5.2.2.3 of 3GPP TS 29.564 [5]. This follows the mechanism "Subscribe-Notify" interaction between a NF service consumer and a NF service producer as specified in clause 7.1.2 of 3GPP TS 23.501 [2].

So, a solution between the NF service consumer (of Nupf_EventExposure service) and the UPF needs to be defined along the existing event subscription procedure to enable them to negotiate the among supported protocol alternatives and/or various enhancements based on the existing protocol and then determine how to populate the subsequent event reports generated for the subscription.

6.3.2	Procedures

The solution includes the following steps:

1-2.	The UPF, as the NF service producer, announces its support of a list of protocols alternatives and enhancements based on the existing protocol to deliver the event reports, where each protocol alternative and each enhancement to the current protocol as specified in clause 5.2.2.3 of 3GPP TS 29.564 [5] may be defined as an enumeration value of a data type "supportedReportDeliveryMethods" in the upfInfo.

3-4. 	Before invoking Nupf_EventExposure service, the NF service consumer may learn the support of different protocol alternatives and enhancements for delivery of event reports by retrieving the UPF NF Profile stored in the NRF if it is possible, e.g. when the NF service consumer directly invoke the UPF event exposure service.

5.	The NF service consumer determines its "preferredReportDeliveryMethods" based on the UPF's supportedReportDeliveryMethods and includes the same in the service request message sent to the UPF, together with the corresponding protocolSupportInformation to enable this alternative from the consumer side. If the NF service consumer doesn't get the "supportedReportDeliveryMethods" for the UPF, the NF service consumer determines the "preferredReportDeliveryMethods" based on the local configuration. The intermediate NF, e.g. the SMF, transparently forwards the preferredReportDeliveryMethods and relevant protocolSupportInformation to the UPF.

6.	The NF service producer, i.e. the UPF, in the service response message, determines which alternative, based on the ones included in the "preferredReportDeliveryMethods" if received, is selected for this subscription resource, and includes it in the "selectedReportDeliveryMethod" together with the corresponding protocolSupportInformation to enable this alternative from the UPF side. If the delivery method included in the "preferredReportDeliveryMethods" is not part of the "supportedReportDeliveryMethods", the UPF selects a delivery method based on the local configuration and the UPF includes in addition its supportedReportDeliveryMethods in the service response. The intermediate NF, e.g. the SMF, transparently forwards the selectedReportDeliveryMethods, the relevant protocolSupportInformation, and supportedReportDeliveryMethods to the NF service consumer.

NOTE 1:	The UPF will select one of alternatives which it supports and is included in the "preferredReportDeliveryMethods" if possible, otherwise the UPF determines which alternative to use based on its local configuration. This can result in that a Report Delivery Method which is not preferred by the NF service consumer is selected, so, the supportedReportDeliveryMethods included the service response is to allow the NF service consumer to change the data delivery methods in the subsequent subscription update service request.

7. 	When event reports are delivered in an alternative other than the existing one (where the event report(s) is sent towards the eventNotifyUri, together with the correlation id, where the event reports can be sufficiently differentiated in the NF service consumer), event report(s) for a given subscription shall reuse the existing data type NotificationData (i.e. a JSON string), however, the NotificationData is extended to include the subscription id (allocated by the UPF) and the eventNotifyUri (included by the NF service consumer in the service request) to enable the NF service consumer to identify which subscription the event report is for.

8.	The following is to exemplify how different protocols alternative can be used to deliver the event reports:

a. 	reusing an existing protocol, e.g., the GTP prime, or a new Type, Length, Value (TLV based) protocol. In this case, the NF service consumer needs to include it as its "preferredReportDeliveryMethods" and provide the protocolSupportInformation includes its IP address and UDP/TCP port number to enable the UPF to initiate the communication to deliver the event reports.

NOTE 2: 	The GTP Prime is specified in 3GPP TS 32.295 [6] to deliver Charging Data Records for different PDN Connections from Charging Data Function to Charging Gateway Function, which is very similar to deliver the event reports applicable to different subscriptions, e.g. corresponding to different PDU sessions.

b. 	using ftp instead of http or https. In this case, if the NF service consumer is served as a FTP server, it will include it as its "preferredReportDeliveryMethods" and provide the ftp IP address and port in the protocolSupportInformation; otherwise it doesn't provide any ftp server address.

NOTE 3: 	Above protocol alternatives are only used to illustrate how to determine the event reports delivery mechanism for a given subscription. Any alternative protocol that would be proposed in this study for event reports delivery shall be contributed as a specific solution.

Figure 6.3.2-1

6.3.3	Impacts on services, entities and interfaces

This solution has impacts on the UPF, NF service consumer (of Nupf_EventExposure service), e.g. the SMF and NWDAF, NEF and the NRF.

UPF:	Nupf_EventExposure service to handle preferred/selected ReportDeliveryMethods and relevant protocol support information;

SMF:	Nsmf_EventExposure service to transfer preferred/selected ReportDeliveryMethods and relevant protocol support information if received;

NEF:	NorthBound services that supports subscribing UPF events to transfer preferred/selected ReportDeliveryMethods and relevant protocol support information if received;

NRF:	Nnrf_NFManagement and Nnrf_NFDiscovery services where a new attribute "supportedReportDeliveryMethods" is added in UpfInfo.

6.3.4	Pros

The solution provides a framework with backwards compatible manner enabling the NF service consumer (of Nupf_EventExposure) to negotiate a preferred protocol to deliver the event reports efficiently for a given subscription.

6.3.5	Cons

FFS

6.4	Solution #4: Optimized UPF data collection based on existing 5G VN group mechanisms, PDU session type or RAT type

6.4.1	Description

The solution addresses the Key Issue #1 on Identifying and lowering the network performance impacts of intensive data collection from UPF.

As illustrated in solution #2, in several use cases, the operator needs to collect frequent User Plane measurements for PDU sessions or users having specific characteristics. The only way to achieve this with the current Nupf_EventExposure service design is to create an event exposure subscription for each such individual PDU session (e.g. of each 5G VN group members), which can result in a very large number of subscriptions at the UPF and to lots of signalling to create (and terminate) these subscriptions. Furthermore, the UPF reports measurements per subscription, causing massive signalling (i.e. very high number of HTTP requests and responses) for reporting the measurements from all such PDU sessions. So, the Nupf_EventExposure can be enhanced to support the creation of a UPF event exposure subscription targeting all the users and/or PDU sessions associated with the specific characteristics (operator defined marker in solution#2).

Various mechanisms/concepts/identifiers defined in 5GS can be used to identify the Users or PDU sessions with the same characteristics, e.g.,

-	Users or PDU sessions belongs to 5G VN group

-	Users establishing a specific PDU session type, e.g., Ethernet PDU session, IP PDU session

-	Users with same RAT type

This solution relies on reusing the existing mechanisms/concepts/identifiers above to identify the users or PDU sessions with the same characteristics and enhancing the Nupf_EventExposure to support the creation of a UPF event exposure subscription targeting all the users and/or PDU sessions associated with same characteristics.

Principles of the solution:

1)	During the UE’s PDU session establishment/modification, the SMF provides the following information to the UPF:

-	5G VN group identifier. The existing 5G VN group provisioning and subscription mechanism applied to this solution. During the 5G VN group member's PDU session establishment, the SMF provides the 5G VN group identifier, which is retrieved from UDM to the UPF, during the PFCP session establishment or modification. The UPF stores the 5G VN group identifier in its PFCP session context.

-	PDU session type, specified as PDN Type IE in clause 7.5.2 of 3GPP TS 29.244 [10].

-	RAT type, specified as RAT type IE in clause 7.5.2 of 3GPP TS 29.244 [10].

2)	The UPF Event Exposure is enhanced to enable an NF service consumer to create an event exposure subscription at the UPF with the following information.

-	A specific 5G VN group identifier, this information enables the NF service consumer to create UPF event exposure subscription targeting to all the users or PDU sessions belongs to this specific 5G VN group.

-	PDU session type. this information enables the NF service consumer to create UPF event exposure subscription targeting to all the users establishing the same PDU session type, e.g., Ethernet PDU session.

-	RAT type, this information enables the NF service consumer to create UPF event exposure subscription targeting to all the users with the same RAT type, e.g., NG-REDCAP.

3)	The UPF reports the measurements of such PDU sessions (up to the maximum size of the message) in a same notification message towards the NF service consumer (the message indicates to which PDU session each measurement applies).

6.4.2	Procedures

The solution includes the PDU session establishment/modification procedure as specified in clause 4.3.2 of 3GPP TS 23.502 [2]. During the PDU session establishment/modification procedure, SMF provide 5G VN group identifier, PDU session type or RAT type over N4 interface to UPF.

The solution also includes the following steps as illustrate in Figure 6.4.2-1:

1.	The NF consumer sends the Nupf_EventExposure Subscription request to the UPF to subscribe to UE data, including the 5G VN group identifier, PDU session type or RAT type to identify which users or PDU sessions should be subscribed. The intermediate NF, i.e. the SMF, transparently forwards this information to the UPF.

NOTE:	The solution assumes that the UPF supports UPF event subscriptions with the new subscription parameters. Otherwise, the NF service consumer subscribes to the UPF using the existing UPF event subscription solution (i.e. per PDU session).

2.	The UPF creates event subscription resource if the request is accepted and sends 201 Created or returns relevant error as specified in clause 5.2.2.2.2 of 3GPP TS 29.564 [5].

3.	The UPF sends the locally collected User data or PDU session data by invoking Nupf_EventExposure_Notify service operation to the UPF event consumer. All the event reports of these PDU sessions (up to the maximum size of the message) are contained in a same notification message towards the NF service consumer (the message indicates to which PDU session each measurement applies)

Figure 6.4.2-1

6.4.3	Impacts on services, entities and interfaces

UPF/SMF:

-	the PFCP session establishment/modification requests are extended to enable the SMF to send the 5G VN group identification for a 5G VN group member's PFCP session (PDU session).

UPF/SMF/NWDAF:

-	the Nupf_EventExposure service is enhanced to enable an NF service consumer to create an event exposure subscription at the UPF targeting all PDU sessions within the specific 5G VN group, or all the PDU sessions with the same PDU session type, or all the PDU sessions with the same RAT type.

6.4.4	Pros

The solution provides the following benefits:

-	The solution enables to reduce drastically the number of UPF event exposure subscriptions to be created at the UPF for subscriptions targetting to PDU sessions within a 5G VN group, with the same PDU session type, or with the same RAT type. For instance, to collect UPF data for PDU sessions of a 5G VN group, instead of creating as many subscriptions as there are 5G VN group members, the NF service consumer can create (only once) a single UPF event exposure subscription targeting all the PDU sessions of 5G VN group members.

-	The UPF can report the measurements from PDU sessions within a 5G VN group, with the same PDU session type, or with the same RAT type in the same notification message, significantly reducing the number of notification messages (and therefore HTTP requests/responses) required to report the requested data.

-	Processing overhead is reduced as a single subscription can target all the PDU sessions, and since the number of notifications is reduced, for subscriptions targetting PDU sessions within a 5G VN group, with the same PDU session type, or with the same RAT type.

-	The solution re-used the existing mechanisms/concepts/identifiers in allowing reducing the number of subscription/notification messages for UPF which will not cause UE subscription impact.

6.4.5	Cons

-	The solution requires standards and implementation changes (SMF, UPF for each possible use case/operator need.

-	The solution only provides benefits for subscriptions targeting PDU sessions within a 5G VN group, with the same PDU session type, or with the same RAT type.

6.5	Solution #5: Dynamic provisioning of PDU session markers

Editor's note:	this solution should be reviewed and agreed by SA2 and CT3, if CT4 would like to proceed with this solution after the evaluation of the different solutions.

Editor's note: details of the solution are FFS.

6.5.1	Description

The solution addresses the Key Issue #1 on Identifying and lowering the network performance impacts of intensive data collection from UPF. The solution complements the Solution #2 on Optimized UPF data collection based on PDU session markers.

The NEF provides a new Nnef_MarkerManagement service that allows an NF service consumer (e.g. AF, NWDAF) to create, modify or delete markers and associate them with the relevant PDU sessions. When a new marker is created, the NEF returns a Marker ID to the NF service consumer. A marker can be modified or deleted using the Marker ID.

The NEF further provides a new Nnef_MarkerManagement service that enables an NF service consumer (e.g. AF, NWDAF) to discover existing markers (i.e. already provisioned markers).

6.5.2	Procedures

Figure 6.5.2-1 depicts the call flow enabling an NF service consumer (e.g. AF, NWDAF) to provision markers and associate them with the relevant PDU sessions.

Figure 6.5.2-1: Dynamic provisioning of markers (with AF as example consumer 
of the new NEF service)

1.	The AF determines that a new Marker is needed. It issues a Nnef_MarkerManagement_Create request (requesting NF Identifier, metadata, filters). The metadata describes the purpose of the marker (e.g. "VIP-users"). The filters identify the traffic (PDU sessions) to be associated with the marker.

The filters may comprise a combination of one or more filters defined in Table 6.5.2-1.

Table 6.5.2-1: Filters for PDU session marking

| Information Name | Applicable for PCF or NEF | Applicable to NEF only |
| --- | --- | --- |
| Traffic Description | Defines the target traffic (PDU sessions) to be marked, represented by the combination of DNN and optionally S-NSSAI. It may also indicate a PDU session type. (NOTE 3) (NOTE 4) | The target traffic can be represented by AF-Service-Identifier, instead of combination of DNN and optionally S-NSSAI. |
| Target UE Identifier(s) | Indicates the UE(s) that the request is targeting, i.e. one or a list of individual UE(s), a group of UE represented by Internal Group Identifier(s) (NOTE 1), or any UE accessing the combination of DNN, S-NSSAI and DNAI(s). It may indicate UEs with specific Subscriber Category (such as VIP, gold, silver, bronze users). It may indicate UEs using specific services (i.e. allowed services as defined in Table 6.2.1.3-2 of 3GPP TS 23.503 [4]). | GPSI can be applied to identify the individual UE, or External Group Identifier(s) can be applied to identify a group of UE. External Subscriber Category(s) (NOTE 2). |
| Spatial Validity Condition | Indicates that the request applies only to the traffic of UE(s) located in the specified location, represented by areas of validity. | The specified location can be represented by geographical area. |
| Operator defined filters | Operator defined filters allowing to support operator specific use cases that cannot be covered by the above filters. (NOTE 5) |  |
| NOTE 1: Internal Group ID can only be used by an AF controlled by the operator and only towards PCF. NOTE 2: External Subscriber category(s) can be combined with External Group ID(s) or any UE. When External Subscriber Category(s) is provided, the NEF maps External Subscriber Category(s) into Subscriber Category(s), and the PCF maps the AF request with user subscription and then creates PCC rules for UEs that have the Subscriber Category(s) in their subscription. NOTE 3: The Traffic Description needs not contain application identifiers or traffic filtering information since markers are associated with PDU sessions. Application identifiers or traffic filtering information may be included in the Nupf_EventExposure Subscribe request, if the NF service consumer requests measurements from the UPF for specific application or traffic filters of the marked PDU sessions. NOTE 4: Even though the Nupf_EventExposure Subscribe request already supports including the DNN/S-NSSAI, the Traffic Description filter is useful in combination with other filters to limit the set of PDU sessions to be marked to only those of interest for the NF service consumer. This avoids unnecessary signaling to mark PDU sessions which would match all filters but the Traffic Description filter. NOTE 5: An operator defined filter may comprise one or more operator defined attributes that enable the NF Service Consumer to invoke an operator defined filtering functionality. |  |  |

The above filters are similar to those supported by the NEF and PCF e.g.:

-	for AF influence on traffic routing (see Table 5.6.7-1 of 3GPP TS 23.501 [2]); or

-	for AF influence on Handling of Payload Headers (see Table 5.6.x.2-1 of 3GPP TS 23.501 [2]).

As an example, the AF may request to create a marker for all the PDU sessions of VIP users to a specific S-NSSAI/DNN (and create then a subscription to the UPF with the same marker to retrieve measurements for all these PDU sessions).

To modify or delete an existing Marker, the AF invokes a Nnef_MarkerManagement_Update or Nnef_MarkerManagement_Delete service operation providing the corresponding Marker ID and, for the case of the Update service operation, the modified metadata or filters. 

The NEF authorizes the request. The NEF retrieves the Internal Group Id corresponding to the external Group ID from UDM (not shown in the figure) if the UE filters refer to users of a specific group.

NOTE 1:	The NF (e.g. AF or NWDAF) that creates a marker is the NF that subscribes to the UPF event exposure. For instance, upon receiving a Nnwdaf_AnalyticsInfo service request including the Targets of Analytic Reporting (that can be a specific UE, a list of UEs, any UE or a Group ID) and Analytics Filter Information (that comprises e.g. S-NSSAI, DNN, Area of Interest), the NDWAF can decide, based on local policy (e.g. for Targets of Analytic Reporting targeting any UE with Analytics Filter Information other than just an S-NSSAI/DNN, or e.g. considering a maximal number of markers allowed by the operator to be created in the 5GS), to create a marker corresponding to these targets and filters (if none already exists), and then subscribe to the UPF with the corresponding marker.

NOTE 2:	Whether the NDWAF/AF performs an Nnef_MarkerDiscovery_Get service request or not depends on implementation/deployment, e.g. NDWAF/AF would typically locally store the info about the markers that it has already created. NWDAFs of a same NWDAF set may also share this information. For deployment with a single NWDAF or NWDAF set, the NWDAFs might never use the Nnef_MarkerDiscovery_Get requests.

NOTE 3:	Markers are typically intended for subscriptions that target a large number of PDU sessions (e.g. subscriptions targeting any UE with a filter other than just an S-NSSAI/DNN, e.g. a specific subscriber category).

NOTE 4:	The solution proposes to define a new Nnef_MarkerManagement service distinct from the Nnef_TrafficInfluence service, because the functionalities of associating markers to PDU sessions and influencing the traffic of PDU sessions are distinct (e.g. a marker may need to be created for collecting UPF data without having to influence the traffic).

2.	For a Nnef_MarkerManagement_Create or _Update request, the NEF stores the Marker information in the UDR (Data Set = Application Data; Data Subset = AF Marker information, Data Key = Marker ID, S-NSSAI and DNN and/or Internal Group Identifier). For a Nnef_MarkerManagement_Delete request, the NEF deletes the Marker information in the UDR (Data Set = Application Data; Data Subset = AF Marker information, Data Key = Marker ID).

3.	The NEF responds to the AF indicating the outcomes of the request and, in case of successful Marker creation, returns a Marker ID.

4.	The PCF(s) that have subscribed to modifications of AF requests (Data Set = Application Data; Data Subset = AF Marker information, Data Key = S-NSSAI and DNN and/or Internal Group Identifier) receive(s) an Nudr_DM_Notify notification of data change from the UDR.

5.	The PCF determines if existing PDU Sessions are potentially impacted by the AF request. For each of these PDU Sessions, the PCF updates the SMF with corresponding policy information about the PDU Session by invoking Npcf_SMPolicyControl_UpdateNotify service operation as described in steps 4 and 5 in clause 4.16.5.2 of 3GPP TS 23.502 [3]. The policy information includes the marker information (Marker ID and filters information).

When the updated policy information about the PDU Session is received from the PCF, the SMF may take appropriate actions to update the PFCP session (associated to the PDU Session) in the UPF with any new/updated/deleted Marker information.

NOTE 5:	The solution involves the PCF because the filters for marking PDU sessions can include policy related parameters (e.g. subscriber category) that only the PCF can check.

Figure 6.5.2-2 depicts the call flow enabling an NF service consumer (e.g. AF, NWDAF) to discover existing markers.

Figure 6.5.2-2: Discovering existing markers (with AF as example consumer 
of the new NEF service)

1.	The AF may use the Nnef_MarkerDiscovery_Get request to request existing markers matching certain criteria (e.g. specific metadata and/or creator NF identifier and/or filters). The NEF authorizes the request.

2.	The NEF responds with the list of markers that match the criteria, including the Marker ID, metadata, creator NF ID and associated filters.

The AF may decide after the discovery that it can use an existing marker or that it needs to create a new one.

6.5.3	Impacts on services, entities and interfaces

NEF APIs:

-	Two new NEF services (Nnef_MarkerManagement and Nnef_MarkerDiscovery) are defined to enable the discovery and dynamic provisioning of markers.

UDR APIs:

-	A new Data Subset is defined for AF Marker information.

PCF APIs:

-	The Npcf_SMPolicyControl API is updated to support providing marker information to the SMF.

NOTE:	The solution does not impact the Nnwdaf_AnalyticsInfo service.

6.5.4	Pros

The solution provides the following benefits:

-	The solution enables an NF service consumer (e.g. AF, NWDAF) to discover existing markers and/or to provision new markers, e.g. NWDAF can create a marker for PDU sessions of VIP users, and then subscribe to the UPF event exposure with this marker.

6.5.5	Cons

FFS

6.6	Solution #6: Alternative protocol DRP+UDP on Data collection

6.6.1	Description

The solution addresses the Key Issue #1 on Identifying and lowering the network performance impacts of intensive data collection from UPF.

The NF service consumer (e.g. SMF, NWDAF) subscribes to the Nupf_EventExposure service as described in 3GPP TS 29.564 [5]. The subscription may target, e.g. ANY UE, and the UPF reports the events to the consumer, which may result in massive network signalling messages. Currently, SBI interface based on HTTP/2+TCP is used to send the notification messages from UPF to the NF service consumer, the solution proposes an alternative protocol DRP+UDP to support the data collection of subscriptions, e.g. ANY UE subscriptions.

Principles of the solution:

1)	The alternative protocol stack DRP+UDP is defined as below:

Figure 6.6.1-1 Data Reporting Protocol

2)	DRP protocol is a new protocol defined in 3GPP, it is only used for data collection, the SBI protocol is still used for subscription, and the format of the notification in DRP is defined in Figure 6.6.1-2. The notification message is formatted to include the list of record headers and records for the subscribed events.

Figure 6.6.1-2 notification message format

3)	The Record header is a variable length header whose minimum length is 6 bytes.

| Octets |  | 8 | 7 | 6 |  | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Version |  |  | Spare |  |  |  |  |  |
| 2 |  | Message Type |  |  |  |  |  |  |  |  |
| 3 |  | Length (1st Octet) |  |  |  |  |  |  |  |  |
| 4 |  | Length (2nd Octet) |  |  |  |  |  |  |  |  |
| 5 to 6 |  | Sequence Number |  |  |  |  |  |  |  |  |

Figure 6.6.1-3 Record Header format

-	Version field: This field is used to determine the version of the DRP protocol.

-	Message Type: This field indicates the type of DRP message.

-	Sequence Number: sequence number of the record.

4)	The Record field contains the information elements (i.e. the IEs defined in NotificationData data type of 3GPP TS 29.564 [5]) as Octetstring encoding representing a JSON object.

During subscription, the NWDAF will provide callback URI and correlation Id in subscription request, UPF sends the related notification to the IP address/Port number retrieved from the callback URI and include the notification correlation ID in the record, the value of the correlation ID shall be unique per subscription for the NWDAF. The notification message is formatted based on record header and record per subscription, the UPF can combine the notifications related to several subscriptions from one UE or different UEs into one message (i.e. targeting the same notification IP and port), which will contain several record headers and records, each record header and record is related to one subscription.

The protocol is changed from the GTP protocol, some basic mechanisms, e.g. the Echo Request and Echo Response is also supported by the protocol.

Editor's note: It should be clarified on how to support reliable delivery and overload control based on DRP+UDP protocol.

6.6.2	Impacts on services, entities and interfaces

UPF/NWDAF:

-	support of data collection using DRP+UDP protocol stack to event notification for subscriptions, e.g. ANY UE type subscription.

SMF/NRF:

-	selection of the UPF with the support the DRP+UPF protocol based on the solution defined in clause 6.3.

6.6.3	Pros

Editor's note: this is FFS.

6.6.4	Cons

Editor's note: this is FFS.

7	Evaluation

8	Conclusion

8.1	Conclusions of KI#1

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | CT4#124 | C4-243602 |  |  |  | TR skeleton | 0.0.0 |
| 2024-08 | CT4#124 | C4-243672 |  |  |  | Inclusion of pCRs agreed at CT4#124, including C4-243556, C4-243595, C4-243600, C4-243603, C4-243605. | 0.1.0 |
| 2024-10 | CT4#125 | C4-244500 |  |  |  | Inclusion of pCRs agreed at CT4#125, including C4-244226, C4-244451, C4-244464, C4-244529, C4-244530, C4-244531, C4-244548 | 0.2.0 |
| 2024-11 | CT4#126 | C4-245538 |  |  |  | Inclusion of pCRs agreed at CT4#126, including C4-245388, C4-245389, C4-245390. | 0.3.0 |