3GPP TSG-RAN WG2 Meeting #125	R2-24xxxxx

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Agenda item:		7.4.1.3.1

Source:		Lenovo

Title:		[B100] TP on SCG LTM with fast MCG link recovery

Document for:		Discussion and Decision

# Introduction

This contribution aims to discuss [B100] for TS38.331.

# Discussion

In Rel-18 LTM, RAN2 confirmed in RAN2#123 that an LTM cell switch procedure should not be triggered while an MCG failure recovery procedure is ongoing. That means SCG LTM is not allowed to be triggered via LTM cell switching command MAC CE when UE is performing MCG failure information procedure i.e. T316 is running. However, RAN2 did not discuss how to ensure SCG LTM is not triggered while T316 is running.

Observation 1: RAN2 agreed that a SCG LTM cell switch procedure should not be triggered while an MCG failure recovery procedure is ongoing. But it has not been captured in RAN2 or RAN3 specification.

There are two options for ensuring SCG LTM cell switch procedure should not be triggered when T316 is running as follows.

- Option 1 (Network-based option): SN-DU should be aware that T316 is running since SCG LTM is triggered by SN-DU. Therefore, SN-CU needs to inform SN-DU once MCGFailureInformation is triggered which will impact RAN3 specification.

- Option 2 (UE-based option) (proposed by Samsung): It might be simpler to initiate Reestablishment if SCG LTM cell switch is triggered while T316 is running. Samsung further pointed out that UE may stop reporting LTM measurements for SCG in this case to prevent/minimize the DU from triggering the SCG LTM.

Proposal 1: RAN2 to discuss which option should be agreed to ensure SCG LTM cell switch procedure should not be triggered when T316 is running.

- Option 1 (Network-based option): SN-CU needs to inform SN-DU once MCGFailureInformation is triggered.

- Option 2 (UE-based option): UE initiates reestablishment procedure if SCG LTM cell switch is triggered when the fast MCG link recovery procedure is ongoing.

Proposal 2a: If Option 1 in P1 is agreed, LS should be sent to RAN3.

Proposal 2b: If Option 2 in P1 is agreed, RAN2 to agree the TP show below.

# Text Proposal

<begin>

#### 5.3.7.2	Initiation

The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure of the MCG and t316 is not configured, in accordance with 5.3.10; or

1>	upon detecting radio link failure of the MCG while SCG transmission is suspended, in accordance with 5.3.10; or

1>	upon detecting radio link failure of the MCG while PSCell change or PSCell addition is ongoing, in accordance with 5.3.10; or

1>	upon detecting radio link failure of the MCG while the SCG is deactivated, in accordance with 5.3.10; or

1>	upon re-configuration with sync failure of the MCG, in accordance with clause 5.3.5.8.3; or

1>	upon mobility from NR failure, in accordance with clause 5.4.3.5; or

1>	upon integrity check failure indication from lower layers concerning SRB1 or SRB2, except if the integrity check failure is detected on the RRCReestablishment message; or

1>	upon an RRC connection reconfiguration failure, in accordance with clause 5.3.5.8.2; or

1>	upon detecting radio link failure for the SCG while MCG transmission is suspended, in accordance with clause 5.3.10.3 in NR-DC or in accordance with TS 36.331 [10] clause 5.3.11.3 in NE-DC; or

1>	upon reconfiguration with sync failure of the SCG while MCG transmission is suspended in accordance with clause 5.3.5.8.3; or

1>	upon SCG change failure while MCG transmission is suspended in accordance with TS 36.331 [10] clause 5.3.5.7a; or

1>	upon SCG configuration failure while MCG transmission is suspended in accordance with clause 5.3.5.8.2 in NR-DC or in accordance with TS 36.331 [10] clause 5.3.5.5 in NE-DC; or

1>	upon integrity check failure indication from SCG lower layers concerning SRB3 while MCG is suspended; or

1>	upon T316 expiry, in accordance with clause 5.7.3b.5; or

1>	upon detecting sidelink radio link failure by L2 U2N Remote UE in RRC_CONNECTED which is not configured with MP, in accordance with clause 5.8.9.3; or

1>	upon reception of NotificationMessageSidelink including indicationType by L2 U2N Remote UE in RRC_CONNECTED which is not configured with MP, in accordance with clause 5.8.9.10; or

1>	upon PC5 unicast link release for the serving L2 U2N Relay UE indicated by upper layer at L2 U2N Remote UE in RRC_CONNECTED which is not configured with MP while T301 is not running; or

1>	if MP is configured, upon detecting radio link failure of the MCG (i.e. direct path) in accordance with clause 5.3.10 while the transmission of indirect path is suspended as specified in 5.3.5.17; or

1>	if MP is configured, upon detecting sidelink radio link failure of SL indirect path by L2 U2N Remote UE, in accordance with clause 5.8.9.3, while MCG transmission (i.e. direct path) is suspended as specified in clause 5.7.3b; or

1>	if MP is configured, upon reception of NotificationMessageSidelink including indicationType in accordance with clause 5.8.9.10, while MCG transmission (i.e. direct path) is suspended as specified in clause 5.7.3b; or

1>	if MP is configured, upon PC5 unicast link release indicated by upper layer at L2 U2N Remote UE, while MCG transmission (i.e. direct path) is suspended as specified in clause 5.7.3b; or

- if MP is configured, upon detecting the failure of N3C indirect path by N3C remote UE in accordance with clause 5.7.3c, while MCG transmission (i.e. direct path) is suspended.

- Upon the indication by lower layers that an LTM cell switch procedure for SCG is triggered while T316 is running.

NOTE 0:	It is up to UE implementation whether to initiate the procedure while T346g is running.

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T312, if running;

1>	stop timer T304, if running;

1>	start timer T311;

1>	stop timer T316, if running;

1>	if UE is not configured with attemptCondReconfig; and

- if UE is not configured with attemptLTM-Switch:

……

……

<end>

# Conclusion

In this contribution, the following observation and proposals are given based on the discussion:

Proposal 1: RAN2 to discuss which option should be agreed to ensure SCG LTM cell switch procedure should not be triggered when T316 is running.

- Option 1 (Network-based option): SN-CU needs to inform SN-DU once MCGFailureInformation is triggered.

- Option 2 (UE-based option): UE initiates reestablishment procedure if SCG LTM cell switch is triggered when the fast MCG link recovery procedure is ongoing.

Proposal 2a: If Option 1 in P1 is agreed, LS should be sent to RAN3.

Proposal 2b: If Option 2 in P1 is agreed, RAN2 to agree the TP show below.

# References

[1] TS 38.331 v18.0.0