ö3GPP TSG-RAN2#AH-1801	Tdoc R2-18xxxxx

Vancouver, Canada, 22-26 Jan 2018

Agenda Item:		x.y

Source:				Ericsson

Title:					Review issue list for TS 38.331 EN-DC ASN.1 freeze

Version:				0.0

Document for:		Discussion and decision

# Introduction

This document provides an overview of list of issues resulting from the review of the PDU specification and related procedure text and field descriptions.

For some issues the proposed solution is indicated as well as the company & Tdoc introducing this in the standard. For some of the issues this document includes further considerations. The following companies volunteered for the review.

“ID” identifies the company, and consists of “X” (<letter>), as “E” for Ericsson.

| ID | Companies | Contact persons | Email | Last iss |
| --- | --- | --- | --- | --- |
| E | Ericsson | Håkan Palm Riikka Susitaival Henning Wiemann | hakan.l.palm@ericsson.com riikka.susitaival@ericsson.com henning.wiemann@ericsson.com |  |
| S | Samsung | Himke van der Velde | himke.vandervelde@samsung.com |  |
| N | Nokia | Tero Henttonen Amaanat Ali | tero.henttonen@nokia.com amaanat.ali@nokia.com |  |
| Z | ZTE | Eswar LiuJing HuangHe Sergio | eswar.vutukuri@zte.com.cn liu.jing30@zte.com.cn huang.he4@zte.com.cn Sergio.Parolari@zte.com.cn |  |
| H | Huawei | Nathan Tenny David Lecompte Brian Martin | nathan.tenny@huawei.com david.lecompte@huawei.com brian.alexander.martin@huawei.com |  |
| I | Intel | Guo, Yi Jeong, Kyeongin Palat, Sudeep | yi.guo@intel.com kyeongin.jeong@intel.com K sudeep.k.palat@intel.com |  |
| V | Vivo | Kimba Dit AdamouChen, Li Wu, Yumin | boubacar kimba@vivo.com chenli5g@vivo.com wuyumin@vivo.com |  |
| D | DOCOMO | Hideaki Takahashi Wuri Hapsari Toru Uchino Kenji Kai | hideaki.takahashi.vx@nttdocomo.com wuri@nttdocomo.com tooru.uchino.fv@nttdocomo.com kenji.kai.gx@nttdocomo.com |  |
| C | CATT | Chandrika Worrall Jing Liang | chandrika@catt.cn liangjing@catt.cn |  |
| O | OPPO | Lei Yixue | leiyixue@oppo.com |  |
| Q | QUALCOMM | Keiichi Kubota Umesh Phuyal Masato Kitazoe | keiichik@qti.qualcomm.com uphuyal@qti.qualcomm.com mkitazoe@qti.qualcomm.com |  |
| L | LGE | Bokyung Jaewook Oanyong | bokyung.byun@lge.com jaewook.lee@lge.com aidoy.lee@lge.com |  |
| M | Mediatek | Alex Hsu YuanYuan Zhang Li-Chuan Tseng Johan Johansson | alex.hsu@mediatek.com YuanY.Zhang@mediatek.com Li-Chuan.Tseng@mediatek.com Chun-Fan.Tsai@mediatek.com johan.johansson@mediatek.com |  |

# Instructions for RIL and CR storage

RIL and Editorial CR is stored in ftp://ftp.3gpp.org/Email_Discussions/RAN2/.

Companies are requested to provide their review comments and change proposals directly in the stored documents (see Classification below). Companies are encouraged to continuously introduce their comments/changes in the RILs/CR, e.g. when one area have been reviewed (and not provide all comments together on the last day).

When storing the documents after providing updates, companies should add their Company ID (one letter, see section 1) to the file name.

Companies are encouraged to try to resolve collisions. Also the Rapporteur will take an active role in this (and storage/merging problems can be discussed via email). In future reviews, we can potentially use more sophisticated tools.

# Instructions for RIL

Issue Number (I-No)

All issues should be numbered in a format Xyyy where

- X is the unique ID (<letter>) assigned to each company, see the table in clause 1.

- yyy is a running number starting from 001, i.e. 001, 002, …. 999.

- Ex: “E103”.

To avoid duplicated I-No numbers, companies may use the table very last in this document.

Description

Describe the issue in a few words.

Classification (Class):

| Class 1 | Straigthforward clarification/correction will not be included in RIL but company will directly include in “Minor corrections CR”. This can include small things like addition of need codes (as long as relatively straightforward) |
| --- | --- |
| Class 2 | Small issue i.e. solution requires some discussion but possible to concluded as part of ASN.1 review. Within column Details in the RIL, the company raising the issue is invited to suggest a proposed way forward, that other companies review and if not agreeable may suggest alternatives for. |
| Class 3 | More significant issue, i.e. requiring more extensive analysis by a contribution. Class 3 issues are within the scope of the ASN.1 review (i.e. does not concern more functional aspects). Companies are requested to volunteer for drafting a contribution (CR). A contribution may address multiple issues, but these should be clearly marked. |
| Class 4 | Issue of type 4 are like type 3, with the exception that the issue is not only adressing ASN.1 aspects but also more functional aspects. Companies are still invited to draft a contribution, but this would be treated in the agenda item covering the concerned related functionality. |

Companies are requested to provide contribution details, to have an overview of the status (in particular regarding which issues are not covered).

Details (proposed solution/ discussion)

Mainly relevant for issues of class 2, the cell is intended to discuss/ agree the proposed way forward. The company raising the issue is invited to suggest a proposed way forward, that other companies review and if not agreeable may suggest alternatives for.

Companies are encouraged to descripe solutions in the same manner as they correct issues in CRs, i.e. propose changes that are shown in the same manner using change marks.

It is therefore suggested to use “simulated change marks” for the issue reporting, i.e.

- Added parts are marked with underlined red coloured text, e.g. new text .

- Deleted parts are marked with strikethough red coloured text, e.g. delated text.

- If there is a need to high-light something by marking text with a colour, e.g. to high-light small changes, it is recommended that yellow colour is used, e.g. spelling error.

- Reason for these “simulated change marks” is to alow for more easy moving/copy/paste without loosing the changes.

Companies are encouraged to comment issues introduced by other companies, both on agreeing or objecting. These comments shall be tagged with the company name for easy search. E.g. “Ericsson: We agree”.

Status/Ref (to be filled in by the Rapporteur)

Status of the issue, in particular:

Class 2: indicate FFS if no (confirmed) way forward yet

Class 3: indicate company planning to bring a contribution

Class 4: same as 3, but also indicate agenda item

(coding/coloring TBD)

# Conclusion & recommendation

This paper includes a of list of issues resulting from the review of [1]. RAN2 is requested to endorse the status including the solutions proposed.

# References

[1] 	TS 38.331 RRC specification

# Review issue list (Annex)

#### Foreword

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 1	Scope

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 2	References

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 3	Definitions, symbols and abbreviations

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 3.1	Definitions

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M001 | - Generally, definitions of the terms exist in 36.331 for LTE should also be listed in 38.331, e.g. Dual connectivity, Primary cell, Primary secondary cell, Master cell group, Secondary cell group, etc. - We have special cell definition ‘Special Cell: For Dual Connectivity operation the term Special Cell refers to the PCell of the MCG or the PSCell of the SCG, otherwise the term Special Cell refers to the PCell.’, but no definition of PCell and PSCell. - The definition of bandwidth part shold also be added. | Class 2 |  |  |

#### 3.2	Abbreviations

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M002 | - Several abbreviations are missing - Whether to use bandwidth part or abbrevation of BWP? | Class 2 | - Add following abbrevations, which exist in LTE, e.g. CA, DC, TDD, TTT, DL-SCH, SFN - Add new abbrevations for NR, e.g. SFTD. - Use abbrevation of BWP for bandwidth part in the procedure and ASN.1. |  |

#### 4	General

#### 4.1	Introduction

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M003 | - “clause 6 specifies the RRC messages in ASN.1;” is not proper reflecting ‘The contents of each RRC message is specified in sub-clause 6.2 using ASN.1 to specify the message syntax and using tables when needed to provide further detailed information about the fields specified in the message syntax.’. | Class 1 | Suggest to change to -clause 6 specifies the RRC message in a mixed format (i.e. tabular & ASN.1 together) |  |

#### 4.2	Architecture

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 4.2.1	UE states and state transitions including inter RAT

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M004 | Indent for the UE behavior description in RRC_IDLE. | Class 1 | - The UE: - Monitors a Paging channel; - Performs neighbouring cell measurements and cell (re-)selection; - Acquires system information |  |

#### 4.2.2	Signalling radio bearers

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 4.3	Services

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 4.3.1	Services provided to upper layers

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 4.3.2	Services expected from lower layers

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 4.4	Functions

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5	Procedures

#### 5.1	General

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.1.1	Introduction

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M005 | Correction to “and UE capability transfer (5.86)” | Class 1 | and UE capability transfer(5.6) |  |

#### 5.1.2	General requirements

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.2	System information

Only MIB acquisition applicable for EN-DC.

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3	Connection control

#### 5.3.1	Introduction

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.2	Paging

Targeted for completion in June 2018.

#### 5.3.3	RRC connection establihshment

Targeted for completion in June 2018.

#### 5.3.4	Initial security activation

Targeted for completion in June 2018.

#### 5.3.5	RRC reconfiguration

#### 5.3.5.1	General

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M006 | How to capture BWP addition/modification/release. It was agreed that one subclause for BWP addition/modification and one subclause for BWP release are needed. | Class 3 | It’s beneficial to have a separate subclause for BWP addition/modification/release considering the flexibility and extendability. Otherwise, if BWP addition/modification/release is included in the Scell addition/modification procedure, the whole section need to be revised for either BWP or Scell reconfiguration. A CR can be provided for BWP addition/modification and BWP release. |  |
| M007 | RAN2 agreed to use “reconfiguration with sync” instead of “handover” in RRC SPEC. | Class 1 | The purpose of this procedure is to modify an RRC connection, e.g. to establish/ modify/ release RBs, to perform handover reconfiguration with sync, to setup/ modify/ release measurements, to add/ modify/ release SCells and cell groups. As part of the procedure, NAS dedicated information may be transferred from RAN to the UE. |  |

#### 5.3.5.2	Initiation

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M008 | There is no IE named “secondaryCellGroup”. The procedure text should base the IE “secondaryCellGroupToAddModList” | Class 2 | 1> if the received RRCReconfiguration includes the SecondaryCellGroupToReleaseList: 2> perform the SCG release according to 5.3.5.4; 1> if the received RRCReconfiguration includes the secondaryCellGroup secondaryCellGroupToAddModList : 2> for each CellGroupId in the secondaryCellGroupToAddModList: 23> perform the cell group configuration for the SCG according to 5.3.5.5; 1> if the RRCReconfiguration message contains the radioBearerConfig: 2> perform the radio bearer configuration according to 5.3.5.6; |  |

#### 5.3.5.3	Reception of an RRCReconfiguration by the UE

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.4	Secondary cell group release

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.5	Cell Group configuration

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.5.1	General

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M009 | The IE name should be italic. | Class 1 | The UE performs the following actions based on a received CellGroupConfig IE: 1> if the received CellGroupConfig contains the spCellConfig with reconfigurationWithSync: 2> perform Reconfiguration with sync according to 5.3.5.5.2; 2> resume all suspended radio bearers and resume SCG transmission for all radio bearers, if suspended; 1> if the CellGroupConfig contains the rlc-BearerToReleaseList: 2> perform RLC bearer release as specified in 5.3.5.5.3; 1> for each element in RLC-BeaererToAddModList: 2> configure the RLC bearer as specified in 5.3.5.5.4; 1> if the CellGroupConfig contains the mac-CellGroupConfig: |  |

#### 5.3.5.5.2	Reconfiguration with synch

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.5.3	Logical Channel release

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M010 | Correct typo “lrlc-BearerToReleaseList”  “rlc-BearerToReleaseList” | Class 1 | The UE shall: 1> for each LogicalChannelIdentity value included in the lrlc-BearerToReleaseList that is part of the current UE configuration (LCH release), or 1> for each LogicalChannelIdentity value that is to be released as the result of full configuration option according to 5.3.5.7 or as the result of an SCG release according to 5.3.5.4: |  |

#### 5.3.5.5.4	Logical Channel addition/modification

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M012 | Remove duplicate clause. The deleted clause has been covered by previous if-else clauses. | Class 2 | 2> if the logical channel ID corresponds to an SRB (i.e. ID less than or equal to 3) and if mac-LogicalChannelConfig is not included: 3> configure this MAC entity with a logical channel in accordance to the default configuration defined in 9.2 for the corresponding SRB; 2> else: 3> configure this MAC entity with a logical channel in accordance to the received mac-LogicalChannelConfig; 2> configure this MAC entity with a logical channel in accordance to the received mac-LogicalChannelConfig; |  |

#### 5.3.5.5.5	MAC entity configuration

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.5.6	RLF Timers & Constants configuration

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M013 | Whether to not to harmanize RLF timers and constants for MCG and SCG. | Class 4 | Two CRs with/without harmonization of RLF timers and constants can be provided for comparision. |  |

#### 5.3.5.5.7	SpCell Configuration

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.5.8	SCell Release

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.5.9	SCell Addition/Modification

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.6	Radio Bearer configuration

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.6.1	General

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M014 | The IE name should be italic. | Class 1 | The UE shall perform the following actions based on a received RadioBearerConfig IE: 1> if the RadioBearerConfig includes the srb-ToReleaseList: 2> perform the SRB release as specified in 5.3.5.6.2; 1> if the RadioBearerConfig includes the srb-ToAddModList: 2> perform the SRB addition or reconfiguration as specified in 5.3.5.6.3; 1> if the RadioBearerConfig includes the drb-ToReleaseList: 2> perform DRB release as specified in 5.3.5.6.4; 1> if the RadioBearerConfig includes the drb-ToAddModList: 2> perform DRB addition or reconfiguration as specified in 5.3.5.6.5; |  |

#### 5.3.5.6.2	SRB release

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.6.3	SRB addition/ modification

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M015 | The key used for SRB us not KUPenc. | Class 1 | 1> for each srb-Identity value included in the srb-ToAddModList that is not part of the current UE configuration or configured with pdcp-Config (SRB establishment or reconfiguration from E-UTRA PDCP to NR PDCP): 2> establish a PDCP entity and configure it with the security algorithms according to securityConfig and apply the keys (KUPenc KRRCint and KRRCenc) associated with the KeNB/S-KgNB as indicated in keyToUse, if applicable; 2> if the current UE configuration as specified in TS 36.331 includes an SRB identified with the same srb-Identity value: |  |

#### 5.3.5.6.4	DRB release

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.6.5	DRB addition/ modification

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M016 | The IE name should be italic. | Class 1 | 1> for each drb-Identity value included in the drb-ToAddModList that is not part of the current UE configuration (DRB establishment including the case when full configuration option is used): 2> establish a PDCP entity and configure it in accordance with the received pdcp-Config; |  |
| M017 | Use “the network” instead of “the eNB” in NR RRC SPEC. | Class 1 | NOTE: Removal and addition of the same drb-Identity in a single radioResourceConfig is not supported. In case drb-Identity is removed and added due to handover or re-establishment with the full configuration option, the eNB network can use the same value of drb-Identity. |  |

#### 5.3.5.7	Full configuration

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.8	Security key update

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.9	Reconfiguration failure

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.9.1	Integrity check failure

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.9.2	Inability to comply with RRCReconfiguration

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.9.3	T304 expiry (Reconfiguration with synch Failure)

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.6	Counter check

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.7	RRC connection re-establishment

Targeted for completion in June 2018.

#### 5.3.8	RRC connection release

Targeted for completion in June 2018.

#### 5.3.9	RRC connection release requested by upper layers

Targeted for completion in June 2018.

#### 5.3.10	Radio resource configuration

Targeted for completion in June 2018.

#### 5.3.11	Radio link failure related actions

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M018 | Whether to harmanize the RLF for MCG and SCG. | Class 4 | Considering the conditions and following behaviors for MCG RLF and SCG RLF are different, it is suggested that the procedure for MCG and SCG RLF detection is not harmonized. |  |

#### 5.3.11.1	Detection of physical layer problems in RRC_CONNECTED

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.11.2	Recovery of physical layer problems

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.11.3	Detection of radio link failure

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M019 | Correct typo on reference section. The SCG Failure section is 5.7.3 (not 5.7.34). | Class 1 | The UE shall: 1> upon T313 expiry, or 1> upon random access problem indication from SCG MAC, or 1> upon indication from SCG RLC that the maximum number of retransmissions has been reached: 2> consider radio link failure to be detected for the SCG i.e. SCG-RLF; Editor’s Note: FFS: How to handle RLC failure in CA duplication for SCG DRB and SRB. 2> initiate the SCG failure information procedure as specified in 5.7.34 to report SCG radio link failure; |  |

#### 5.3.12	UE actions upon leaving RRC_CONNECTED

Targeted for completion in June 2018.

#### 5.3.13	UE actions upon PUCCH/SRS release request

Targeted for completion in June 2018.

#### 5.4	Inter-RAT mobility

Targeted for completion in June 2018.

#### 5.5	Measurements

#### 5.5.1	Introduction

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.2	Measurement configuration

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.2.1	General

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.2.2	Measurement identity removal

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.2.3	Measurement identity addition/ modification

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.2.4	Measurement object removal

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.2.5	Measurement object addition/ modification

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.2.6	Reporting configuration removal

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.2.7	Reporting configuration addition/ modification

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.2.8	Quantity configuration

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.2.9	Measurement gap configuration

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.2.10	Reference signal measurement timing configuration

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.3	Performing measurements

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.3.1	General

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.3.2	Layer 3 filtering

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.3.3	Derivation of measurement results

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.4	Measurement report triggering

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.4.1	General

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.4.2	Event A1 (Serving becomes better than threshold)

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.4.3	Event A2 (Serving becomes worse than threshold)

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.4.4	Event A3 (Neighbour becomes offset better than PCell/ PSCell)

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.4.5	Event A4 (Neighbour becomes better than threshold)

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.4.6	Event A5 (PCell/ PSCell becomes worse than threshold1 and neighbour becomes better than threshold2)

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.4.7	Event A6 (Neighbour becomes offset better than SCell)

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.5	Measurement reporting

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.5.1	Reporting of beam measurement information

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.6	UE capabilities

#### 5.6.1	UE capability transfer

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.7	Other

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.7.1	DL information transfer

Targeted for completion in June 2018.

#### 5.7.2	UL information transfer

Targeted for completion in June 2018.

#### 5.7.3	SCG failure information

#### 5.7.3.1	General

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.7.3.2	Initiation

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M020 | Correct the reference section number. | Class 1 | 1> upon detecting radio link failure for the SCG, in accordance with subclause 5.3.11.3, 1> upon reconfiguration with sync failure of the SCG, in accordance with subclause 5.3.5.9.3, 1> upon stopping uplink transmission towards the SCG’s SpCell due to exceeding the maximum uplink transmission timing difference, in accordance with subclause x.x.x of TS 38.133 [xx]FFS_Ref. Editor’s Note: FFS on RAN1 decision on powerControlMode; 1> upon SCG configuration failure, in accordance with subclause 5.3.5.89.2; 1> upon integrity check failure indication from SCG lower layers, in accordance with subclause 5.3.5.9.13; |  |

#### 5.7.3.3	Failure type determination

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.7.3.4	Setting the contents of FailureReportSCGtoOtherRAT

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 6	Protocol data units, formats and parameters (ASN.1)

#### 6.1	General

#### 6.1.1	Introduction

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 6.1.2	Need codes for optional downlink fields

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 6.2	RRC messages

#### 6.2.1	General message structure

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	NR-RRC-Definitions

#### –	BCCH-BCH-Message

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	DL-DCCH-Message

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	UL-DCCH-Message

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 6.2.2	Message definitions

#### –	MIB

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	MeasurementReport

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	RRCReconfiguration

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	RRCReconfigurationComplete

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M021 | Direction of NR RRC message should not to E-UTRAN. | Class 1 | The RRCReconfigurationComplete message is used to confirm the successful completion of an RRC connection reconfiguration. Signalling radio bearer: SRB1 or SRB3 RLC-SAP: AM Logical channel: DCCH Direction: UE to NetworkE-UTRAN |  |

#### –	SIB1

#### 6.3	RRC information elements

#### –	SetupRelease Information Element

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 6.3.1	System information blocks

#### 6.3.2	Radio resource control information elements

#### –	Alpha

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	DRB-Identity

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	BandwidthPart-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M022 | - Abbreviation of BWP can be used instead of bandwidthpart, just for IE alignment between different specs. - Generally, the name of the IEs for L1 parameters should be aligned with RAN1 specs. - New IE for BWP-pair-index is conditional optional for TDD. - There is no IE for location. The location and bandwidth for the BWP is jointly configured by one IE. | Class 2 | - BandwidthPart-> BWP - Align the name of IE with RAN1 specs. - Add new IE BWP-pair-index for TDD. - Remove the IE location for BWP configuration. |  |
| M023 | - PDSCH, PDCCH, PUSCH and PUCCH is per BWP or per cell. | Class 3 | - PDSCH, PDCCH, PUSCH and PUCCH is per BWP. |  |

#### –	CellGroupConfig

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M024 | The need code for “rlc-BearerToAddModList” and “rlc-BearerToReleaseList” are missed. It should be “Need M” | Class 1 | CellGroupConfig ::= SEQUENCE { cellGroupId CellGroupId, -- Logical Channel configuration and association with radio bearers: rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF LCH-Config OPTIONAL, -- Need M rlc-BearerToReleaseList SEQUENCE (SIZE(1..maxLCH)) OF LogicalChannelIdentity OPTIONAL, -- Need M -- Parameters applicable for the entire cell group: mac-CellGroupConfig MAC-CellGroupConfig OPTIONAL, -- Need M rlf-TimersAndConstants RLF-TimersAndConstants OPTIONAL, -- Need M physical-CellGroupConfig PhysicalCellGroupConfig OPTIONAL, -- Need M -- Serving Cell specific parameters (PCell and SCells) spCellConfig SpCellConfig OPTIONAL, -- Need M sCellToAddModList SCellToAddModList OPTIONAL, -- Need M sCellToReleaseList SCellToReleaseList OPTIONAL -- Need M } |  |
| M025 | We suggest to add extension marker “…” in the IE “CellGroupConfig” so that it is easier be extended in the future. | Class 2 | CellGroupConfig ::= SEQUENCE { cellGroupId CellGroupId, -- Logical Channel configuration and association with radio bearers: rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF LCH-Config OPTIONAL, rlc-BearerToReleaseList SEQUENCE (SIZE(1..maxLCH)) OF LogicalChannelIdentity OPTIONAL, -- Parameters applicable for the entire cell group: mac-CellGroupConfig MAC-CellGroupConfig OPTIONAL, -- Need M rlf-TimersAndConstants RLF-TimersAndConstants OPTIONAL, -- Need M physical-CellGroupConfig PhysicalCellGroupConfig OPTIONAL, -- Need M -- Serving Cell specific parameters (PCell and SCells) spCellConfig SpCellConfig OPTIONAL, -- Need M sCellToAddModList SCellToAddModList OPTIONAL, -- Need M sCellToReleaseList SCellToReleaseList OPTIONAL -- Need M ... } |  |
| M026 | The cell group Id should be started with value 0 to conver the MCG configuration. | Class 1 | CellGroupId ::= INTEGER (10.. maxSCellGroups) |  |
| M027 | In the enum of IE t304. It is not necessary to use ms10000-v1310, ms10000 is enough. | Class 1 | SpCellConfig ::= SEQUENCE { -- Parameters for the synchronous reconfiguration to the target SpCell: reconfigurationWithSync SEQUENCE { spCellConfigCommon ServingCellConfigCommon, newUE-Identity RNTI-Value, t304 ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000-v1310}, rach-ConfigDedicated RACH-ConfigDedicated OPTIONAL -- Need M } OPTIONAL, -- Cond SpCellChange spCellConfigDedicated ServingCellConfigDedicated OPTIONAL -- Need M } |  |
| M028 | UE does not maintain the “rach-ConfigDedicated” configuration while this IE is not present. The need code of this IE should be “Need S”. | Class 1 | SpCellConfig ::= SEQUENCE { -- Parameters for the synchronous reconfiguration to the target SpCell: reconfigurationWithSync SEQUENCE { spCellConfigCommon ServingCellConfigCommon, newUE-Identity RNTI-Value, t304 ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000-v1310}, rach-ConfigDedicated RACH-ConfigDedicated OPTIONAL -- Need MS } OPTIONAL, -- Cond SpCellChange spCellConfigDedicated ServingCellConfigDedicated OPTIONAL -- Need M } |  |

#### –	CellIndexList

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	ControlResourceIndex

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	CrossCarrierSchedulingConfig

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	CSI-MeasConfig

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M029 | SSBResourceMeasList (38.331) is not aligned with resource-config-SS-list (38.214). | 1 | -- List of SSB resources used for beam measurement and reporting in a resource set -- Corresponds to L1 parameter 'SSBResourceMeasListresource-config-SS-list' (see 38,214, section FFS_Section) ssb-Resources SEQUENCE (SIZE (1..maxNrofSSB-Resources-1)) OF CSI-SSB-Resource OPTIONAL, --Cond OnlyWithNZPResourceSets |  |

#### –	FailureReportSCGtoOtherRAT

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	FrequencyInfoDL

#### –	FrequencyInfoUL

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	LogicalChannelConfig

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M030 | We suggest to add extension marker “…” in the IE “LogicalChannelConfig” so that it is easier be extended in the future. | Class 2 | LogicalChannelConfig ::= SEQUENCE { ul-SpecificParameters SEQUENCE { priority INTEGER (1..16), prioritisedBitRate ENUMERATED {kBps0, kBps8, kBps16, kBps32, kBps64, kBps128, kBps256, kBps512,kBps1024, kBps2048, kBps4096, kBps8192, kBps16384, kBps32768, kBps65536, infinity}, bucketSizeDuration ENUMERATED {ms50, ms100, ms150, ms300, ms500, ms1000, spare2, spare1}, -- FFS: Detailed handling of restrictions (UP email discussion) -- Defined in L1 parameters but the value range must be checked. allowedSubCarrierSpacing SubcarrierSpacing OPTIONAL, allowedTiming ENUMERATED {ffsTypeAndValue} OPTIONAL, logicalChannelGroup INTEGER (0..maxLCid) OPTIONAL, logicalChannelSR-Mask BOOLEAN, logicalChannelSR-DelayTimerApplied BOOLEAN } OPTIONAL -- Cond UL -- other parameters ..., } |  |
| M031 | LCP restriction for carrier and configured grant is not yet added. | Class 1 | Add the followin two parameters (as already specified in TS 38.321) - lcp-configuredGrantType1Allowed which sets whether a Configured Grant Type 1 can be used for transmission; - lcp-allowedServingCells |  |
| M032 | SchedulingRequestId is missing in LogicalChannelConfig | Class 1 | Inlcude SchedulingRequestId in “LogicalChannelConfig” as “optional” |  |
|  | To control logicalChannelSR-DelayTimer, TS 38.331 uses the term logicalChannelSR-DelayTimerApplied, while TS 38.321 uses the term logicalChannelSR-Delay. |  | Use the same term in MAC and RRC spec |  |

#### –	MAC-CellGroupConfig

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	MeasConfig

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	MeasId

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M033 | Naming convention: For IDs, the infix of “Nrof” is not needed. We should simply use maxMeasId, as in LTE. | Class 2 | MeasId ::= INTEGER (1..maxNrofMeasId) |  |

#### –	MeasIdToAddModList

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	MeasObjectEUTRA

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	MeasObjectId

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M034 | Naming convention: For IDs, the infix of “Nrof” is not needed. We should simply use maxObjectId, as in LTE. | 2 | MeasObjectId ::= INTEGER (1..maxNrofObjectId) |  |

#### –	MeasObjectNR

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M035 | Definition of maxQuantityConfigId is unclear | 3 | In measObjectNR, we have quantityConfigIndex INTEGER (1..maxQuantityConfigId), In QualityConfig, however, we have QuantityConfigNR-List::= SEQUENCE (SIZE (1..maxNroQuantityConfig)) OF QuantityConfigNR It seems that maxQuantityConfigId and maxNroQuantityConfig are the same thing? The naming should be aligned. |  |
| M036 | (typo) Description of absThreshSS-BlocksConsolidation and absThreshCSI-RS-Consolidation | 1 | absThreshCSI-RS-Consolidation Absolute threshold for the consolidation of measurement results per CSI-RS resource(s) from L1 filter(s). The values above the threshold are used as input to the derivation of cell measurement results as described in 5.3.x and the L3 filter(s) per CSI-RS resource as described in 5.5.3.2. absThreshSS-BlocksConsolidation Absolute threshold for the consolidation of measurement results per SS/PBCH block(s) from L1 filter(s). The values above the threshold are used as input to the derivation of cell measurement results as described in 5.3.x and the L3 filter(s) per SS/PBCH block index as described in 5.5.3.2. |  |
| M037 | (typo) MeasObjectNR field descriptions | 1 | nrofCSI-RS-ResourcesToAverage nrofSS-BlocksToAverage |  |
| M038 | ReferenceSignalConfig should be mandatory | 2 | referenceSignalConfig ReferenceSignalConfig OPTIONAL, |  |

#### –	MeasObjectToAddModList

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	MeasResults

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M040 | Naming convention for csi-rsIndex | 2 | In ResultsPerCSI-RSIndex, we have csi-rsIndex CSI-RSIndex, Like other fields (e.g., csi-rs-Cellrsrp), there should be a ‘-‘ between “rs” and “Index”, i.e., csi-rs-Index CSI-RS-Index, |  |

#### –	PDCCH-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	PDCP-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M041 | RAN2 agreed that out-of-order delivery does not apply to SRB. So, we suggest to put IE “” inside DRB configuration. | Class 2 | PDCP-Config ::= SEQUENCE { drb SEQUENCE { -- Note that some IE are skips here integrityProtection BOOLEAN, statusReportRequired BOOLEAN, OPTIONAL -- Cond Rlc-AM outOfOrderDelivery BOOLEAN } OPTIONAL, -- Cond DRB -- Note that some IE are skips here outOfOrderDelivery BOOLEAN, ... } |  |

#### –	PDSCH-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	PhysCellId

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	PUCCH-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	PUSCH-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	Q-OffsetRange

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	QuantityConfig

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M042 | QuantityConfig: should be either (1) CHOICE, or (2) simply a list of QualityConfigNR. It should not be a SEQUENCE | 3 | (1) QuantityConfig ::= CHOICE { quantityConfigNR QuantityConfigNR OPTIONAL, quantityConfigNR-list QuantityConfigNR-List OPTIONAL } (2) QuantityConfig::= SEQUENCE (SIZE (1..maxNroQuantityConfig)) OF QuantityConfigNR |  |
| M043 | Naming convention for maxNroQuantityConfig | 2 | In QualityConfig, we have QuantityConfigNR-List::= SEQUENCE (SIZE (1..maxNroQuantityConfig)) OF QuantityConfigNR In measObjectNR, however, we have quantityConfigIndex INTEGER (1..maxQuantityConfigId), It seems that maxNroQuantityConfig and maxQuantityConfigId are the same thing? The naming should be aligned. Also, if we adopt the “Nr” naming, it should be maxNrofQuantityConfig |  |

#### –	RACH-ConfigCommon

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M062 | For contention RACH, R1 has agreed that SCS for Msg 2 is the same as the numerology of RMSI. So, there is no need to signal msg2 SCS, | 1 | msg2-SubcarrierSpacing SubcarrierSpacing, |  |
| M063 | RAN1 and RAN2 has agreed separate SSB and preamble association. Need to find a way to align the behavior. | 4 | RAN1 association -- Corresponds to L1 parameter 'CB-preambles-per-SSB' (see 38.211?, section FFS_Section) -- FFS_CHECK: Relation to (old) RAN2 CBRA-SSB-ResourceList handling the CB-RA preambles/resources per beam. -- FFS_Value: RAN1 indicated ”4 bit” but there should be actual values here... and not hidden in a table. cb-preamblesPerSSB FFS_Value OPTIONAL, -- Number of SSBs per RACH occasion. By multiplying with cb-preamblesPerSSB, the UE determines the total number of CB preambles. -- Corresponds to L1 parameter 'SSB-per-rach-occasion' (see 38.211?, section FFS_Section) -- FFS_CHECK: Relation to (old) RAN2 CBRA-SSB-ResourceList handling the CB-RA preambles/resources per beam. -- FFS_Value: RAN1 indicated ”3 bit” but there should be actual values here... and not hidden in a table. ssb-perRACH-Occasion FFS_Value OPTIONAL, RAN2 association CBRA-SSB-ResourceList ::= SEQUENCE (SIZE(1..maxRAssbResources)) OF CBRA-SSB-Resource CBRA-SSB-Resource ::= SEQUENCE { ssb SSB-Id, startIndexRA-PreambleGroupA PreambleStartIndex, numberofRA-PreamblesGroupA NumberOfRA-Preambles, numberOfRA-Preambles NumberOfRA-Preambles, -- PRACH configuration for SSB configuration (i.e. time and frequency location) -- FFS / TODO: Type Definition for RA-Resources. ra-Resources RA-Resources } |  |

#### –	RACH-ConfigDedicated

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	RadioBearerConfig

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M044 | There is no Cond HO in the Conditional Presence table. It should be Cond KeyChange. | Class 1 | DRB-ToAddMod ::= SEQUENCE { cnAssociation CHOICE { -- The EPS bearer ID determines the EPS bearer when NR connects to EPC using EN-DC eps-BearerIdentity INTEGER (0..15), -- EPS-DRB-Setup -- The SDAP configuration determines how to map QoS flows to DRBs when NR connects to the 5GC sdap-Config SDAP-Config -- NGC }, drb-Identity DRB-Identity, -- may only be set if the cell groups of all linked logical channels are reset or released reestablishPDCP ENUMERATED{true} OPTIONAL, -- Cond HOKeyChange recoverPDCP ENUMERATED{true} OPTIONAL, -- Need N pdcp-Config PDCP-Config OPTIONAL, -- Cond PDCP ... } |  |
| M045 | The description of “Cond PDCP” should apply both SRB and DRB. In NR, it SRB could also configured with PDCP-config. | Class 2 | The field is mandatory present if the corresponding DRB is being setup or reconfigured with NR PDCP; otherwise the field is optionally present, need M |  |
| M064 | We suggest to add extension marker “…” in the IE “RadioBearerConfig” so that it is easier be extended in the future. | Class 2 | RadioBearerConfig ::= SEQUENCE { srb-ToAddModList SRB-ToAddModList OPTIONAL, -- Need M srb-ToReleaseList INTEGER (3) OPTIONAL, -- Need M drb-ToAddModList DRB-ToAddModList OPTIONAL, -- Need M drb-ToReleaseList DRB-ToReleaseList OPTIONAL, -- Need M securityConfig SecurityConfig OPTIONAL -- Cond KeyChange ... } |  |

#### –	ReportConfigId

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M046 | Naming convention: For IDs, the infix of “Nrof” is not needed. We should simply use maxReportConfigId, as in LTE. | 2 | ReportConfigId ::= INTEGER (1..maxNrofReportConfigId) |  |

#### –	ReportConfigNR

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M047 | Remove “Editor’s Note: FFS Whether MeasTriggerQuantityOffset should be a CHOICE instead of a SEQUENCE e.g. in case multiple trigger quantities are supported.” | 2 | Each event should be triggered by one type of measurement quantity, so MeasTriggerQuantityOffset (also MeasTriggerQuantity) should be a CHOICE. The ASN.1 is correct and we can remove the FFS. |  |
| M048 | Definition of maxNrofIndexesToReport is confusing | 2 | In EventTriggerConfig, the maxNrofIndexesToReport is defined as maxNrofIndexesToReport INTEGER (1..maxNrofIndexesToReport) OPTIONAL, The naming is confusing. This can be resolved by using the naming as in PeriodicalReportConfig, i.e., maxNrofRsIndexesToReport INTEGER (1..maxNrofIndexesToReport) OPTIONAL, |  |

#### –	ReportConfigToAddModList

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	RLC-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	RLF-TimersAndConstants

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	SCellIndex

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	SchedulingRequest-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	SchedulingRequestResource-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M049 | The association between SR configuration and the corresponding appliable SR resource is not specified | Class 1 | In SchedulingRequestResource-Config, add schedulingRequestId |  |

#### –	SDAP-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	SecurityAlgorithmConfig

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	ServCellIndex

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	ServingCellConfigCommon

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M050 | - Initial UL BWP | Class 2 | Add one IE initial-UL-BWP for initial UL BWP configuration. |  |

#### –	ServingCellConfigDedicated

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	SRB-Identity

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	SPS-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M051 | TS 38.331 still uses sps-RNTI, while MAC has chaged the usage and term to be cs-RNTI |  | Change the usage of sps-RNTI throughout TS 38.331 |  |

#### –	SRS-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	SubcarrierSpacing

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 6.3.3	UE capability information elements

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| M052 | For MR-DC, which basebandProcessingCombinationIndex, basebandProcessingCombinationLinkedIndex is link to NR (or LTE)? was the intention to describe in field description? | 2 | LinkedBasebandProcessingCombination ::= SEQUENCE { basebandProcessingCombinationIndex-EUTRAN BasebandProcessingCombinationIndex, basebandProcessingCombinationLinkedIndex-NR SEQUENCE (SIZE (1..maxBasebandProcComb)) OF BasebandProcessingCombinationIndex } |  |
| M053 | Do we still need NR band list? | 3 | SupportedBandListNR ::= SEQUENCE (SIZE (1..maxBands)) OF BandNR |  |
| M054 | maxNrofSCells, SCell is confusing | 1 | Rename maxNrofCC |  |
| M055 | CA-bandwidthClass is sufficient to describle supported BW | 2 | Remove BW-PerCC |  |
| M056 | UL and DL support the same modulationOrder? | 3 | Decouple UL/DL modulationOrder, modulationOrder ModulationOrder, modulationOrder-UL ModulationOrder optional, |  |
| M057 | UL and DL support the same SubCarrierSpacing? | 3 | Decouple UL/DL SubCarrierSpacing subCarrierSpacing SubCarrierSpacing subCarrierSpacing-UL SubCarrierSpacing optional, |  |
| M058 | numberOfSR-Configurations is optional with fixed number | 3 | numberOfSR-Configurations ENUMERATED {supported} OPTIONAL, -- The number is 8 if supported |  |
| M059 | Value range for numberOfConfiguredGrantConfigurations | 3 | numberOfConfiguredGrantConfigurations ENUMERATED {n1, n2, n4, spar0} OPTIONAL |  |
| M060 | Spare value numbering start from spare1 or spare0? | 1 | Either one |  |
| M061 | RAN does not need to signal the capability of DRB IP. This is for June version, so can be removed for now. | 3 | Remove dataRateDRB-IP |  |

#### 6.3.4	Other information elements

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 6.4	RRC multiplicity and type constraint values

#### –	Multiplicity and type constraint definitions

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 7	Variables and constants

#### 7.1	Timers

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 7.1.1	Timers (Informative)

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 7.1.2	Timer handling

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 7.2	Counters

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 7.3	Constants

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 7.4	UE variables

#### –	VarMeasConfig

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	VarMeasReportList

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 8	Protocol data unit abstract syntax

#### 8.1	General

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 8.2	Structure of encoded RRC messages

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 8.3	Basic production

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 8.4	Extension

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 8.5	Padding

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 9	Specified and default radio configurations

#### 9.1	Specified configurations

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 9.2	Default radio configurations

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 9.2.1	SRB configurations

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 9.2.1.1	SRB1/SRB1S

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 9.2.1.2	SRB2/SRB2S

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 9.2.1.3	SRB3

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 9.2.2	SRB configurations

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 9.2.2.1	SRB1/SRB1S

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 9.2.2.2	SRB2/SRB2S

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 9.2.2.3	SRB3

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 10	Generic error handling

#### 10.1	General

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 10.2	ASN.1 violation or encoding error

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 10.3	Field set to a not comprehended value

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 10.4	Mandatory field missing

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 10.5	Not comprehended field

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 11	Radio information related interactions between network nodes

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 11.1	General

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 11.2	Inter-node RRC messages

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 11.2.1	General

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 11.2.2	Message definitions

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	HandoverCommand

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	HandoverPreparationInformation

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	SCG-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	SCG-ConfigInfo

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 11.3	Inter-node RRC information element definitions

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	CandidateCellInfoList

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 11.4	Inter-node RRC multiplicity and type constraint values

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 12	Processing delay requirements for RRC procedures

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### Annex A (informative):	Guidelines, mainly on use of ASN.1

#### A.3.8	Guidelines on use of parameterised SetupRelease type

#### –	ParentIE-WithEM

#### –	ChildIE1-WithoutEM

#### –	ChildIE2-WithoutEM

#### A.6	Guidelines regarding use of need codes

#### Annex <X> (informative): Change history	180

# Sections not part of the review (for information)

-

# List of last I-No (Issue Number)

Companies indicate their last used I-No, to avoid duplication.

| Company | Last used I-No |
| --- | --- |