3GPP TS 38.423 V0.9.0 (2018-05)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NG-RAN;

Xn application protocol (XnAP)

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners’ Publications Offices.

MCC selects keywords from stock list.

Keywords

<keyword[, keyword, …]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles – Sophia Antipolis

Valbonne – FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	8

1	Scope	9

2	References	9

3	Definitions, symbols and abbreviations	10

3.1	Definitions	10

3.2	Symbols	10

3.3	Abbreviations	10

4	General	11

4.1	Procedure specification principles	11

4.2	Forwards and backwards compatibility	11

4.3	Specification notations	12

5	XnAP services	12

5.1	XnAP procedure modules	12

5.2	Parallel transactions	12

6	Services expected from signalling transport	12

7	Functions of XnAP	12

8	XnAP procedures	13

8.1	Elementary procedures	13

8.2	Basic mobility procedures	15

8.2.1	Handover Preparation	15

8.2.1.1	General	15

8.2.1.2	Successful Operation	15

8.2.1.3	Unsuccessful Operation (if applicable)	17

8.2.1.4	Abnormal Conditions	17

8.2.2	SN Status Transfer	17

8.2.2.1	General	17

8.2.2.2	Successful Operation	18

8.2.2.3	Unsuccessful Operation	18

8.2.2.4	Abnormal Conditions	18

8.2.3	Handover Cancel	18

8.2.3.1	General	18

8.2.3.2	Successful Operation	18

8.2.3.3	Unsuccessful Operation	19

8.2.3.4	Abnormal Conditions	19

8.2.4	Retrieve UE Context	19

8.2.4.1	General	19

8.2.4.2	Successful Operation	19

8.2.4.3	Unsuccessful Operation	20

8.2.4.4	Abnormal Conditions	20

8.2.5	RAN Paging	20

8.2.5.1	General	20

8.2.5.2	Successful operation	20

8.2.5.3	Unsuccessful Operation	21

8.2.5.4	Abnormal Condition	21

8.2.6	Data Forwarding Address Indication	21

8.2.6.1	General	21

8.2.6.2	Successful Operation	21

8.2.6.3	Unsuccessful Operation	21

8.2.6.4	Abnormal Conditions	21

8.2.7	UE Context Release	22

8.2.7.1	General	22

8.2.7.2	Successful Operation	22

8.2.7.3	Unsuccessful Operation	22

8.2.7.4	Abnormal Conditions	22

8.3	Procedures for Dual Connectivity	22

8.3.1	S-NG-RAN node Addition Preparation	23

8.3.1.1	General	23

8.3.1.2	Successful Operation	23

8.3.1.3	Unsuccessful Operation	24

8.3.1.4	Abnormal Conditions	24

8.3.2	S-NG-RAN node Reconfiguration Completion	24

8.3.2.1	General	24

8.3.2.2	Successful Operation	24

8.3.2.3	Abnormal Conditions	25

8.3.3	M-NG-RAN node initiated S-NG-RAN node Modification Preparation	25

8.3.3.1	General	25

8.3.3.2	Successful Operation	25

8.3.3.3	Unsuccessful Operation	27

8.3.3.4	Abnormal Conditions	27

8.3.4	S-NG-RAN node initiated S-NG-RAN node Modification	27

8.3.4.1	General	27

8.3.4.2	Successful Operation	28

8.3.4.3	Unsuccessful Operation	28

8.3.4.4	Abnormal Conditions	28

8.3.5	M-NG-RAN node initiated S-NG-RAN node Release	29

8.3.5.1	General	29

8.3.5.2	Successful Operation	29

8.3.5.3	Unsuccessful Operation	29

8.3.5.4	Abnormal Conditions	30

8.3.6	S-NG-RAN node initiated S-NG-RAN node Release	30

8.3.6.1	General	30

8.3.6.2	Successful Operation	30

8.3.6.3	Unsuccessful Operation	30

8.3.6.4	Abnormal Conditions	30

8.3.7	S-NG-RAN node Counter Check	30

8.3.7.1	General	30

8.3.7.2	Successful Operation	31

8.3.7.3	Unsuccessful Operation	31

8.3.7.4	Abnormal Conditions	31

8.3.8	RRC Transfer	31

8.3.8.1	General	31

8.3.8.2	Successful Operation	31

8.3.8.3	Abnormal Conditions	32

8.4	Global procedures	32

8.4.1	Xn Setup	32

8.4.1.1	General	32

8.4.1.2	Successful Operation	32

8.4.1.3	Unsuccessful Operation	32

8.4.1.4	Abnormal Conditions	33

8.4.2	NG-RAN node Configuration Update	33

8.4.2.1	General	33

8.4.2.2	Successful Operation	33

8.4.2.3	Unsuccessful Operation	34

8.4.2.4	Abnormal Conditions	34

8.4.3	Cell Activation	35

8.4.3.1	General	35

8.4.3.2	Successful Operation	35

8.4.3.3	Unsuccessful Operation	35

8.4.3.4	Abnormal Conditions	35

8.4.4	Reset	36

8.4.4.1	General	36

8.4.4.2	Successful Operation	36

8.4.4.3	Unsuccessful Operation	36

8.4.4.4	Abnormal Conditions	36

8.4.5	Error Indication	36

8.4.5.1	General	36

8.4.5.2	Successful Operation	37

8.4.5.3	Unsuccessful Operation	37

8.4.5.4	Abnormal Conditions	37

9	Elements for XnAP Communication	37

9.0	General	37

9.1	Message Functional Definition and Content	37

9.1.1	Messages for Basic Mobility Procedures	37

9.1.1.1	HANDOVER REQUEST	37

9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE	40

9.1.1.3	HANDOVER PREPARATION FAILURE	40

9.1.1.4	SN STATUS TRANSFER	41

9.1.1.5	UE CONTEXT RELEASE	41

9.1.1.6	HANDOVER CANCEL	41

9.1.1.7	RAN PAGING	42

9.1.1.8	RETRIEVE UE CONTEXT REQUEST	42

9.1.1.9	RETRIEVE UE CONTEXT RESPONSE	43

9.1.1.10	RETRIEVE UE CONTEXT FAILURE	43

9.1.2	Messages for Dual Connectivity Procedures	43

9.1.2.1	S-NODE ADDITION REQUEST	43

9.1.2.2	S-NODE ADDITION REQUEST ACKNOWLEDGE	45

9.1.2.3	S-NODE ADDITION REQUEST REJECT	47

9.1.2.4	S-NODE RECONFIGURATION COMPLETE	47

9.1.2.5	S-NODE MODIFICATION REQUEST	48

9.1.2.6	S-NODE MODIFICATION REQUEST ACKNOWLEDGE	50

9.1.2.7	S-NODE MODIFICATION REQUEST REJECT	53

9.1.2.8	S-NODE MODIFICATION REQUIRED	53

9.1.2.9	S-NODE MODIFICATION CONFIRM	54

9.1.2.10	S-NODE MODIFICATION REFUSE	55

9.1.2.11	S-NODE RELEASE REQUEST	55

9.1.2.12	S-NODE RELEASE REQUEST ACKNOWLEDGE	56

9.1.2.13	S-NODE RELEASE REJECT	57

9.1.2.14	S-NODE RELEASE REQUIRED	57

9.1.2.15	S-NODE RELEASE CONFIRM	57

9.1.2.16	S-NODE COUNTER CHECK REQUEST	58

9.1.2.17	RRC TRANSFER	59

9.1.3	Messages for Global Procedures	60

9.1.3.1	XN SETUP REQUEST	60

9.1.3.2	XN SETUP RESPONSE	61

9.1.3.3	XN SETUP FAILURE	62

9.1.3.4	NG-RAN NODE CONFIGURATION UPDATE	62

9.1.3.5	NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE	63

9.1.3.6	NG-RAN NODE CONFIGURATION UPDATE FAILURE	63

9.1.3.7	CELL ACTIVATION REQUEST	63

9.1.3.8	CELL ACTIVATION RESPONSE	63

9.1.3.9	CELL ACTIVATION FAILURE	64

9.1.3.10	RESET REQUEST	64

9.1.3.11	RESET RESPONSE	64

9.1.3.12	ERROR INDICATION	65

9.2	Information Element definitions	65

9.2.0	General	65

9.2.1	Container and List IE definitions	65

9.2.1.1	PDU Session Resources To Be Setup List	65

9.2.1.2 	PDU Session Resources Admitted List	66

9.2.1.3	PDU Session Resource Admitted Response Transfer	67

9.2.1.4	PDU Session Resources Not Admitted List	68

9.2.1.5	QoS Flow List	68

9.2.1.6	PDU Session Resource Setup Info – SN terminated	68

9.2.1.7	PDU Session Resource Setup Response Info – SN terminated	69

9.2.1.8	PDU Session Resource Setup Info – MN terminated	70

9.2.1.9	PDU Session Resource Setup Response Info – MN terminated	71

9.2.1.10	PDU Session Resource Modification Info – SN terminated	71

9.2.1.11	PDU Session Resource Modification Response Info – SN terminated	72

9.2.1.12	PDU Session Resource Modification Info – MN terminated	74

9.2.1.13	PDU Session Resource Modification Response Info – MN terminated	76

9.2.1.14	UE Context Information	76

9.2.1.15	DRBs Subject To Status Transfer List	76

9.2.1.16	DRB to QoS Flow Mapping List	77

9.2.1.17	Data Forwarding Info from target NG-RAN node	77

9.2.1.18	Served Cells NR To Update	78

9.2.1.19	Served Cells E-UTRA to Update	79

9.2.2	General IE definitions	79

9.2.2.1	QoS Flow Level QoS Parameters	79

9.2.2.2	GBR QoS Flow Information	79

9.2.2.3	Bit Rate	80

9.2.2.4	TAI Support List	80

9.2.2.5	Slice Support List	81

9.2.2.6	Allocation and Retention Priority	81

9.2.2.7	Dynamic 5QI Descriptor	82

9.2.2.8	Non dynamic 5QI Descriptor	83

9.2.2.9	Global NG-RAN Node ID	83

9.2.2.10	Global ng-eNB ID	83

9.2.2.11	SCG Configuration Query	83

9.2.2.12	RLC Mode	84

9.2.2.13	Served Cell Information NR	84

9.2.2.14	SUL Information	85

9.2.2.15	NR Frequency Info	85

9.2.2.16	NR Transmission Bandwidth	85

9.2.2.17	TAC	85

9.2.2.18	S-NSSAI	85

9.2.2.19	Delivery Status	86

9.2.2.20	Security Indication	86

9.2.2.21	Message Type	86

9.2.2.22	Global gNB ID	86

9.2.2.23	NR CGI	87

9.2.2.24	E-UTRA CGI	87

9.2.2.25	PLMN Identity	87

9.2.2.26	Cause	87

9.2.2.27	RAN Paging Priority	89

9.2.2.28	UP Transport Layer Information	89

9.2.2.29	CP Transport Layer Information	89

9.2.2.30	DRB ID	89

9.2.2.31	PDU Session Type	90

9.2.2.32	Criticality Diagnostics	90

9.2.2.33	NG-RAN node UE XnAP ID	90

9.2.2.34	QoS Flow Indicator	91

9.2.2.35	PDU Session ID	91

9.2.2.37	 Cell Assistance Information	91

9.2.2.38	Index to RAT/Frequency Selection Priority	92

9.2.2.39	GUAMI	92

9.2.2.40	Target Cell Global ID	92

9.2.2.41	Masked IMEISV	92

9.2.2.42	NG-RAN Cell Identity	92

9.2.2.43	DL Forwarding	93

9.2.2.44	Data Forwarding Accepted	93

9.2.2.45	Trace Activation	94

9.2.2.46	 RAN Paging Area	94

9.2.2.47	RAN Area ID	95

9.2.2.48	 Location Reporting Information	95

9.2.2.49	Area of Interest	95

9.2.2.50	Transport Layer Address	96

9.3	Message and Information Element Abstract Syntax (with ASN.1)	97

9.3.1	General	97

9.3.2	Usage of Private Message Mechanism for Non-standard Use	97

9.3.3	Elementary Procedure Definitions	98

9.3.4	PDU Definitions	100

9.3.5	Information Element definitions	101

9.3.6	Common definitions	102

9.3.7	Constant definitions	103

9.3.8	Container definitions	104

9.4	Message transfer syntax	109

9.5	Timers	109

10	Handling of unknown, unforeseen and erroneous protocol data	109

Annex A (informative): Change history	110

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the radio network layer signalling procedures of the control plane between NG-RAN nodes in NG RAN. XnAP supports the functions of Xn interface by signalling procedures defined in this document. XnAP is developed in accordance to the general principles stated in TS 38.401 [2] and TS 38.420 [3].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.401: "NG RAN; Architecture Description".

[3]	3GPP TS 38.420: "NG RAN; Xn General Aspects and Principles".

[4]	3GPP TS 38.422: "NG RAN; Xn Signalling Transport".

[5]	3GPP TS 38.413: "NG RAN; NG Application Protocol (NGAP) ".

[6]	3GPP TS 25.921: "Guidelines and principles for protocol description and error handling".

[7]	3GPP TS 23.501: "System Architecture for the 5G System".

[8]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[9]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[10]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) Protocol specification".

[11]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[12]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[13]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[14]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) protocol specification".

[15]	ITU-T Recommendation X.691 (2002-07): "Information technology - ASN.1 encoding rules - Specification of Packed Encoding Rules (PER) ".

[16]	ITU-T Recommendation X.680 (2002-07): "Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[17]	ITU-T Recommendation X.681 (2002-07): "Information technology – Abstract Syntax Notation One (ASN.1): Information object specification".

[18]	3GPP TS 29.281: "General Packet Radio Service (GPRS); Tunnelling Protocol User Plane (GTPv1-U)".

[19]	3GPP TS 38.424: "NG-RAN; Xn data transport".

[20]	3GPP TS 38.414: "NG-RAN; NG data transport".

[21]	3GPP TS 38.412: "NG RAN; NG Signalling Transport".

[22]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[23]	3GPP TS 32.422: "Trace control and configuration management".

[24]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[25] 	3GPP TS 36.104: "Base Station (BS) radio transmission and reception ".

[26]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation".

[27]	3GPP TS 36.101: "User Equipment (UE) radio transmission and reception".

[28]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[29]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[30]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Elementary Procedure: XnAP protocol consists of Elementary Procedures (EPs). An XnAP Elementary Procedure is a unit of interaction between two NG-RAN nodes. An EP consists of an initiating message and possibly a response message. Two kinds of EPs are used:

-	Class 1: Elementary Procedures with response (success or failure),

-	Class 2: Elementary Procedures without response.

NG-RAN node: as defined in TS 38.300 [9].

PDU Session Resource: As defined in TS 38.401 [2].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5QI		5G QoS Identifier

AMF		Access and Mobility Management Function

ARFCN			Absolute Radio Frequency Channel Number

CG			Cell Group

CGI		Cell Global Identifier

CP		Control Plane

DRB		Data Radio Bearer

DRX			Discontinuous Reception

E-RAB		E-UTRAN Radio Access Bearer

GBR		Guaranteed Bit Rate

GTP		GPRS Tunnelling Protocol

GUAMI		Globally Unique AMF Identifier

IMEISV 			International Mobile station Equipment Identity and Software Version number

MCG			Master Cell Group

M-NG-RAN node	Master NG-RAN node

NSSAI		Network Slice Selection Assistance Information

PDCP		Packet Data Convergence Protocol

QoS		Quality of Service

RANAC		RAN Area Code

RLC		Radio Link Control

RNTI			Radio Network Temporary Identifier

RRC		Radio Resource Control

SCG			Secondary Cell Group

S-NG-RAN node			Secondary NG-RAN node

SRB			Signalling Radio Bearer

SUL			Supplementary Uplink

TAC		Tracking Area Code

TAI		Tracking Area Identity

TNL		Transport Network Layer

UP		User Plane

UPF		User Plane Function

# 4	General

## 4.1	Procedure specification principles

The principle for specifying the procedure logic is to specify the functional behaviour of the terminating NG-RAN node exactly and completely. Any rule that specifies the behaviour of the originating NG-RAN node shall be possible to be verified with information that is visible within the system.

The following specification principles have been applied for the procedure text in clause 8:

-	The procedure text discriminates between:

1)	Functionality which “shall” be executed

The procedure text indicates that the receiving node “shall” perform a certain function Y under a certain condition. If the receiving node supports procedure X but cannot perform functionality Y requested in the initiating message of a Class 1 EP, the receiving node shall respond with the message used to report unsuccessful outcome for this procedure, containing an appropriate cause value.

2)	Functionality which “shall, if supported” be executed

The procedure text indicates that the receiving node “shall, if supported,” perform a certain function Y under a certain condition. If the receiving node supports procedure X, but does not support functionality Y, the receiving node shall proceed with the execution of the EP, possibly informing the requesting node about the not supported functionality.

-	Any required inclusion of an optional IE in a response message is explicitly indicated in the procedure text. If the procedure text does not explicitly indicate that an optional IE shall be included in a response message, the optional IE shall not be included. For requirements on including Criticality Diagnostics IE, see section 10.

## 4.2	Forwards and backwards compatibility

The forwards and backwards compatibility of the protocol is assured by a mechanism where all current and future messages, and IEs or groups of related IEs, include ID and criticality fields that are coded in a standard format that will not be changed in the future. These parts can always be decoded regardless of the standard version.

## 4.3	Specification notations

For the purposes of the present document, the following notations apply:

Procedure	When referring to an elementary procedure in the specification the Procedure Name is written with the first letters in each word in upper case characters followed by the word “procedure”, e.g. Handover Preparation procedure.

Message	When referring to a message in the specification the MESSAGE NAME is written with all letters in upper case characters followed by the word “message”, e.g. HANDOVER REQUEST message.

IE	When referring to an information element (IE) in the specification the Information Element Name is written with the first letters in each word in upper case characters and all letters in Italic font followed by the abbreviation “IE”, e.g. PDU Session ID IE.

Value of an IE	When referring to the value of an information element (IE) in the specification the “Value” is written as it is specified in sub clause 9.2 enclosed by quotation marks, e.g. “Value”.

# 5	XnAP services

The present clause describes the services an NG-RAN node offers to its neighbours.

## 5.1	XnAP procedure modules

The Xn interface XnAP procedures are divided into two modules as follows:

1.	XnAP Basic Mobility Procedures;

2.	XnAP Global Procedures;

The XnAP Basic Mobility Procedures module contains procedures used to handle the UE mobility within E-UTRAN.

The Global Procedures module contains procedures that are not related to a specific UE. The procedures in this module are in contrast to the above module involving two peer NG-RAN nodes.

## 5.2	Parallel transactions

Unless explicitly indicated in the procedure specification, at any instance in time one protocol peer shall have a maximum of one ongoing XnAP procedure related to a certain UE.

# 6	Services expected from signalling transport

The signalling connection shall provide in sequence delivery of XnAP messages. XnAP shall be notified if the signalling connection breaks.

Xn signalling transport is specified in TS 38.422 [4].

# 7	Functions of XnAP

The functions of XnAP are specified in TS 38.420 [3].

# 8	XnAP procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | Xn REMOVAL REQUEST | Xn REMOVAL RESPONSE | Xn REMOVAL FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Data Forwarding Address Indication | DATA FORWARDING ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |

## 8.2	Basic mobility procedures

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

If the Signalling TNL association address at source NG-C side IE is included in the HANDOVER REQUEST message the target NG-RAN node shall behave as specified in TS 23.502 [13].

For each E-RAB ID IE included in the Qos Flow List IE in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the content of the IE in the UE context and use it for subsequent inter-system handover.

If the Masked IMEISV IE is contained in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

At reception of the HANDOVER REQUEST message the target NG-RAN node shall prepare the configuration of the AS security relation between the UE and the target NG-RAN node by using the information in the UE Security Capabilities IE and the AS Security Information IE in the UE Context Information IE, as specified in TS 33.501 [28].

Upon reception of the PDU Session Resource Setup List IE, contained in the HANDOVER REQUEST message, the target NG-RAN node shall behave the same as specified in TS 38.413 [5] for the PDU Session Resource Setup procedure. The target NG-RAN node shall report in the HANDOVER REQUEST ACKNOWLEDGE message the successful establishment of the result for all the requested PDU session resources. When the target NG-RAN node reports the unsuccessful establishment of a PDU session resource, the cause value should be precise enough to enable the source NG-RAN node to know the reason for the unsuccessful establishment.

For each QoS flow for which the source NG-RAN node proposes to perform forwarding of downlink data, the source NG-RAN node shall include the DL Forwarding IE set to "DL forwarding proposed" within the QoS Flows To Be Setup List IE in the PDU Session Resource To Be Setup List IE in the HANDOVER REQUEST message. For each PDU session that the target NG-RAN node decides to admit the data forwarding for at least one QoS flow, the target NG-RAN node includes the PDU Session level DL data forwarding GTP-U Tunnel Endpoint IE within the Data Forwarding Info from target NG-RAN node IE in the PDU Session Admitted Response Transfer IE contained in the PDU Sessions Admitted List IE in the HANDOVER REQUEST ACKNOWLEDGE message.

For each DRB for which the source NG-RAN node proposes to perform forwarding of downlink data, the source NG-RAN node shall include the DRB ID IE and the mapped Qos flow list IE within the Source DRB to QoS Flow Mapping List IE contained in the PDU Session Resources To Be Setup List IE in the HANDOVER REQUEST message. If the target NG-RAN node decides to use the same DRB configuration and to map the same QoS flows as the source NG-RAN node, the target NG-RAN node includes the DL Forwarding GTP Tunnel Endpoint IE within the Data Forwarding Response DRB List IE in the HANDOVER REQUEST ACKNOWLEDGE message to indicate that it accepts the proposed forwarding of downlink data for this DRB.

If the HANDOVER REQUEST ACKNOWLEDGE message contains the UL Forwarding GTP Tunnel Endpoint IE for a given DRB in the Data Forwarding Response DRB List IE within Data Forwarding Info from target NG-RAN node IE in the PDU Session Resources Admitted List IE and the source NG-RAN node accepts the data forwarding proposed by the target NG-RAN node, the source NG-RAN node shall perform forwarding of uplink data for the DRB.

If the HANDOVER REQUEST includes PDU session resources for PDU sessions associated to S-NSSAIs not supported by target NG-RAN, the target NG-RAN shall reject such PDU session resources. In this case, and if at least one PDU Session Resource To Be Setup Item IE is admitted, the target NG-RAN shall send the HANDOVER REQUEST ACKNOWLEDGE message including the PDU Session Resources Not Admitted List IE listing corresponding PDU sessions rejected at the target NG-RAN.

If the Mobility Restriction List IE is

-	contained in the HANDOVER REQUEST message, the target NG-RAN node shall

-	store the information received in the Mobility Restriction List IE in the UE context;

-	use this information to determine a target for the UE during subsequent mobility action for which the NG-RAN node provides information about the target of the mobility action towards the UE, except when one of the PDU sessions has a particular ARP value (TS 23.501 [7]) in which case the information shall not apply;

-	use this information to select a proper SCG during dual connectivity operation.

-	use this information to select proper RNA(s) for the UE when moving the UE to RRC_INACTIVE.

-	not contained in the HANDOVER REQUEST message, the target NG-RAN node shall consider that no roaming and no access restriction apply to the UE.

If the Trace Activation IE is included in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, initiate the requested trace function as specified in TS 32.422 [23].

If the Index to RAT/Frequency Selection Priority IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall store this information and use it as defined in TS 23.501 [7].

If the UE Context Reference at the S-NG-RAN IE is contained in the HANDOVER REQUEST message the target NG-RAN node may use it as specified in TS 37.340 [8]. In this case, the source NG-RAN node may expect the target NG-RAN node to include the UE Context Kept Indicator IE set to "True" in the HANDOVER REQUEST ACKNOWLEDGE message, which shall use this information as specified in TS 37.340 [8].

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Be Setup List IE and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "required", the target NG-RAN node shall perform user plane integrity protection or ciphering, respectively. If the NG-RAN node is not able to perform the user plane integrity protection or ciphering, it shall reject the setup of the PDU Session Resources with an appropriate cause value.

If the NG-RAN node is an ng-eNB, it shall reject all PDU sessions for which the Integrity Protection Indication IE is set to "required".

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Be Setup List IE and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "preferred", the target NG-RAN node should, if supported, perform user plane integrity protection or ciphering, respectively and shall notify the SMF whether it succeeded the user plane integrity protection or ciphering or not for the concerned security policy.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Be Setup List IE and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "not needed", the target NG-RAN node shall not perform user plane integrity protection nor ciphering for the concerned PDU session

If the Location Reporting Information IE is included in the HANDOVER REQUEST message, then the target NG-RAN node should initiate the requested location reporting functionality as defined in TS 38.413 [5].

8.2.1.3	Unsuccessful Operation (if applicable)

Figure 8.2.1.3-1: Handover Preparation, unsuccessful operation

If the target NG-RAN node does not admit at least one PDU session resource, or a failure occurs during the Handover Preparation, the target NG-RAN node shall send the HANDOVER PREPARATION FAILURE message to the source NG-RAN node. The message shall contain the Cause IE with an appropriate value.

Interactions with Handover Cancel procedure:

If there is no response from the target NG-RAN node to the HANDOVER REQUEST message before timer TXnRELOCprep expires in the source NG-RAN node, the source NG-RAN node should cancel the Handover Preparation procedure towards the target NG-RAN node by initiating the Handover Cancel procedure with the appropriate value for the Cause IE. The source NG-RAN node shall ignore any HANDOVER REQUEST ACKNOWLEDGE or HANDOVER PREPARATION FAILURE message received after the initiation of the Handover Cancel procedure and remove any reference and release any resources related to the concerned Xn UE-associated signalling.

#### 8.2.1.4	Abnormal Conditions

.

If the supported algorithms for encryption defined in the UE Security Capabilities IE in the UE Context Information IE, plus the mandated support of the EEA0 and NEA0 algorithms in all UEs (TS 33.501 [28]), do not match any allowed algorithms defined in the configured list of allowed encryption algorithms in the NG-RAN node (TS 33.501 [28]), the NG-RAN node shall reject the procedure using the HANDOVER PREPARATION FAILURE message.

If the supported algorithms for integrity defined in the UE Security Capabilities IE in the UE Context Information IE, plus the mandated support of the EIA0 and NIA0 algorithms in all UEs (TS 33.501 [28]), do not match any allowed algorithms defined in the configured list of allowed integrity protection algorithms in the NG-RAN node (TS 33.501 [28]), the NG-RAN node shall reject the procedure using the HANDOVER PREPARATION FAILURE message.

### 8.2.2	SN Status Transfer

#### 8.2.2.1	General

The purpose of the SN Status Transfer procedure is to transfer the uplink PDCP SN and HFN receiver status and the downlink PDCP SN and HFN transmitter status either, from the source to the target NG-RAN node during an Xn handover, for each respective DRB of the source DRB configuration for which PDCP SN and HFN status preservation applies.

The procedure uses UE-associated signalling.

#### 8.2.2.2	Successful Operation

Figure 8.2.2.2-1: SN Status Transfer, successful operation

8.2.2.3	Unsuccessful Operation

Not applicable.

#### 8.2.2.4	Abnormal Conditions

If the target NG-RAN node receives this message for a UE for which no prepared handover exists at the target NG-RAN node, the target NG-RAN node shall ignore the message.

### 8.2.3	Handover Cancel

#### 8.2.3.1	General

The Handover Cancel procedure is used to enable a source NG-RAN node to cancel an ongoing handover preparation or an already prepared handover.

The procedure uses UE-associated signalling.

#### 8.2.3.2	Successful Operation

Figure 8.2.4.2-1: Handover Cancel, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER CANCEL message to the target NG-RAN node. The source NG-RAN node shall indicate the reason for cancelling the handover by means of an appropriate cause value.

8.2.3.3	Unsuccessful Operation

Not applicable.

#### 8.2.3.4	Abnormal Conditions

If the HANDOVER CANCEL message refers to a context that does not exist, the target NG-RAN node shall ignore the message.

### 8.2.4	Retrieve UE Context

#### 8.2.4.1	General

The purpose of the Retrieve UE Context procedure is to retrieve the UE context from the old NG-RAN node and transfer it to the NG-RAN node where the UE RRC Connection has been requested to be established.

The procedure uses UE-associated signalling.

8.2.4.2	Successful Operation

Figure 8.2.4.2-1: Retrieve UE Context, successful operation

The new NG-RAN node initiates the procedure by sending the RETRIEVE UE CONTEXT REQUEST message to the old NG-RAN node.

If the old NG-RAN node is able to identify the UE context by means of the UE Context ID, and to successfully verify the UE by means of the integrity protection contained in the RETRIEVE UE CONTEXT REQUEST message, it shall respond to the new NG-RAN node with the RETRIEVE UE CONTEXT RESPONSE message.

If the Index to RAT/Frequency Selection Priority IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall store this information and use it as defined in TS 23.501 [7].

If the Location Reporting Information IE is included in the RETRIEVE UE CONTEXT RESPONSE message, then the new NG-RAN node should initiate the requested location reporting functionality as defined in TS 38.413 [5].

8.2.4.3	Unsuccessful Operation

Figure 8.2.4.3-1: Retrieve UE Context, unsuccessful operation

If the old NG-RAN node is not able to identify the UE context by means of the I-RNTI, or if the integrity protection contained in the RETRIEVE UE CONTEXT REQUEST message is not valid, it shall respond to the new NG-RAN node with the RETRIEVE UE CONTEXT FAILURE message.

#### 8.2.4.4	Abnormal Conditions

Void	.

### 8.2.5	RAN Paging

#### 8.2.5.1	General

The purpose of the RAN Paging procedure is to enable the NG-RAN node1 to request paging of a UE in the NG-RAN node2.

The procedure uses non UE-associated signalling.

#### 8.2.5.2	Successful operation

Figure 8.2.5.2-1: RAN Paging: successful operation

The RAN Paging procedure is triggered by the NG-RAN node1 by sending the PAGING message to the NG-RAN node2, in which the necessary information e.g. UE Paging Identity should be provided.

If the RAN Paging Priority IE may be included in the RAN PAGING message, the NG-RAN node2 may use it to prioritize paging.

The Assistance Data for Paging IE may be included in the RAN PAGING message, and if present the NG-RAN node2 may use it according to TS 38.300 [9].

If the Assistance Data for Paging IE is included in the RAN PAGING message, the NG-RAN node2 may use it according to TS 38.300 [9].

8.2.5.3	Unsuccessful Operation

Not applicable.

#### 8.2.5.4	Abnormal Condition

Void.

### 8.2.6	Data Forwarding Address Indication

#### 8.2.6.1	General

For the retrieval of a UE context, the Data Forwarding Address Indication procedure is used to provide forwarding addresses from the new NG-RAN node to the old NG-RAN node for all PDU session resources successfully established at the new NG-RAN node for which forwarding was requested.

The procedure uses UE-associated signalling.

8.2.6.2	Successful Operation

Figure 8.2.6.2-1: Data Forwarding Address Indication, successful operation

The Data Forwarding Address Indication procedure is initiated by the new NG-RAN node. Sending the DATA FORWARDING ADDRESS INDICATION message, the new NG-RAN node informs the old NG-RAN node of successfully established PDU Session Resource contexts to which DL user data pending at the old NG-RAN node can be forwarded.

Upon reception of the DATA FORWARDING ADDRESS INDICATION message, the old NG-RAN node should forward pending DL user data to the indicated TNL addresses.

8.2.6.3	Unsuccessful Operation

Not applicable.

#### 8.2.6.4	Abnormal Conditions

Void.

8.2.7	UE Context Release

#### 8.2.7.1	General

For handover, the UE Context Release procedure is initiated by the target NG-RAN node to indicate to the source NG-RAN node that radio and control plane resources for the associated UE context are allowed to be released.

The procedure uses UE-associated signalling.

#### 8.2.7.2	Successful Operation

Figure 8.2.7.2-1: UE Context Release, successful operation for handover

Handover

The UE Context Release procedure is initiated by the target NG-RAN node. By sending the UE CONTEXT RELEASE message the target NG-RAN node informs the source NG-RAN node of Handover success and triggers the release of resources.

8.2.7.3	Unsuccessful Operation

Not applicable.

#### 8.2.7.4	Abnormal Conditions

If the UE Context Release procedure is not initiated towards the source NG-RAN node from any prepared NG-RAN node before the expiry of the timer TXnRELOCoverall, the source NG-RAN node shall request the AMF to release the UE context.

If the UE returns to source NG-RAN node before the reception of the UE CONTEXT RELEASE message or the expiry of the timer TXnRELOCoverall, the source NG-RAN node shall stop the TXnRELOCoverall and continue to serve the UE.

8.3	Procedures for Dual Connectivity

Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018, all the text of section 8.3 is FFS.

8.3.1	S-NG-RAN node Addition Preparation

8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE.

The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

When the M-NG-RAN node sends the S-NODE ADDITION REQUEST message, it shall start the timer TXnDCprep.

The allocation of resources according to the values of the Allocation and Retention Priority IE included in the PDU Session Level QoS Parameters IE shall follow the principles specified for the PDU Session Resource Setup procedure in TS 38.413 [5].

The S-NG-RAN node shall choose the ciphering algorithm based on the information in the UE Security Capabilities IE and locally configured priority list of AS encryption algorithms and apply the key indicated in the S-NG-RAN node Security Key IE as specified in TS 33.501 [28].

If the Additional QoS Flow Information IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [5].

If the S-NODE ADDITION REQUEST message contains the Selected PLMN IE, the S-NG-RAN node may use it for RRM purposes.

If the S-NODE ADDITION REQUEST message contains the Expected UE Behaviour IE, the S-NG-RAN node shall, if supported, store this information and may use it to optimize resource allocation.

If the S-NODE ADDITION REQUEST message contains the Handover Restriction List IE, the S-NG-RAN node, if supported, shall store this information and use it to select an appropriate SCG.

If the S-NODE ADDITION REQUEST message contains the Index to RAT/Frequency Selection Priority IE, the S-NG-RAN node may use it for RRM purposes.

The S-NG-RAN node shall report to the M-NG-RAN node, in the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the result for all the requested PDU session resouces in the following way:

-	A list of PDU session resources which are successfully established shall be included in the PDU Session Resources Admitted To Be Added List IE.

-	A list of PDU session resources which failed to be established shall be included in the PDU Session Resources Not Admitted List IE.

Upon reception of the S-NODE ADDITION REQUEST ACKNOWLEDGE message the M-NG-RAN node shall stop the timer TXnDCprep.

If the S-NG-RAN node UE XnAP ID IE is contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store this information and use it as defined in TS 37.340 [8].

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits at least one PDU session resource, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE ADDITION REQUEST ACKNOWLEDGE message to the M-NG-RAN node. The reception of the S-NODE RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall.

#### 8.3.1.3	Unsuccessful Operation

Figure 8.3.1.3-1: S-NG-RAN node Addition Preparation, unsuccessful operation

If the S-NG-RAN node is not able to accept any of the bearers or a failure occurs during the S-NG-RAN node Addition Preparation, the S-NG-RAN node sends the S-NODE ADDITION REQUEST REJECT message with an appropriate cause value to the M-NG-RAN node.

#### 8.3.1.4	Abnormal Conditions

Void.

### 8.3.2	S-NG-RAN node Reconfiguration Completion

8.3.2.1	General

The purpose of the S-NG-RAN node Reconfiguration Completion procedure is to provide information to the S-NG-RAN node whether the requested configuration was successfully applied by the UE.

The procedure uses UE-associated signalling.

#### 8.3.2.2	Successful Operation

Figure 8.3.2.2-1: S-NG-RAN node Reconfiguration Complete procedure, successful operation.

The M-NG-RAN node initiates the procedure by sending the S-NODE RECONFIGURATION COMPLETE message to the S-NG-RAN node.

The S-NODE RECONFIGURATION COMPLETE message may contain information that

-	either the UE has successfully applied the configuration requested by the S-NG-RAN node. The M-NG-RAN node may also provide configuration information in the M-NG-RAN node to S-NG-RAN node Container IE.

-	or the M-NG-RAN node has not triggered configuration requested by the S-NG-RAN node. The M-NG-RAN node shall provide information with sufficient precision in the included Cause IE to enable the S-NG-RAN node to know the reason for an unsuccessful reconfiguration. The M-NG-RAN node may also provide configuration information in the M-NG-RAN node to S-NG-RAN node Container IE.

Upon reception of the S-NODE RECONFIGURATION COMPLETE message the S-NG-RAN node shall stop the timer TXnDCoverall.

#### 8.3.2.3	Abnormal Conditions

Void.

### 8.3.3	M-NG-RAN node initiated S-NG-RAN node Modification Preparation

8.3.3.1	General

This procedure is used to enable an M-NG-RAN node to request an S-NG-RAN node to modify the UE context at the S-NG-RAN node or to query the current SCG configuration for supporting delta signalling in M-NG-RAN node initiated S-NG-RAN node change.

The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: M-NG-RAN node initiated S-NG-RAN node Modification Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE MODIFICATION REQUEST message to the S-NG-RAN node.

When the M-NG-RAN node sends the S-NODE MODIFICATION REQUEST message, it shall start the timer TXnDCprep.

The S-NODE MODIFICATION REQUEST message may contain

-	within the UE Context Information IE;

-	PDU session resources to be added within the PDU Session Resources To Be Added Item IE;

-	PDU session resources to be modified within the PDU Session Resources To Be Modified Item IE;

-	PDU session resources to be released within the PDU Session Resources To Be Released Item IE;

-	the S-NG-RAN node Security Key IE;

-	the S-NG-RAN node UE Aggregate Maximum Bit Rate IE;

-	the M-NG-RAN node to S-NG-RAN node Container IE;

-	the SCG Change Indication IE;

-	the SCG Configuration Query IE;

- 	the Requested split SRBs IE;

- 	the Requested split SRBs release IE.

If the S-NODE MODIFICATION REQUEST message contains the Selected PLMN IE, the S-NG-RAN node may use it for RRM purposes.

If the S-NODE MODIFICATION REQUEST message contains the Handover Restriction List IE, the S-NG-RAN node shall

-	replace the previously provided Handover Restriction List by the received Handover Restriction List in the UE context;

-	use this information to select an appropriate SCG.

If the S-NG-RAN node UE Aggregate Maximum Bit Rate IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall:

-	replace the previously provided S-NG-RAN node UE Aggregate Maximum Bit Rate by the received S-NG-RAN node UE Aggregate Maximum Bit Rate in the UE context;

-	use the received S-NG-RAN node UE Aggregate Maximum Bit Rate for non-GBR Bearers for the concerned UE as defined in TS 37.340 [8].

If the S-NODE MODIFICATION REQUEST message contains the Index to RAT/Frequency Selection Priority IE, the S-NG-RAN node may use it for RRM purposes.

The allocation of resources according to the values of the Allocation and Retention Priority IE included in the PDU Session Level QoS Parameters IE shall follow the principles specified for the PDU Session Resource Setup procedure in TS 38.413 [5].

If the Additional QoS Flow Information IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [5].

If at least one of the requested modifications is admitted by the S-NG-RAN node, the S-NG-RAN node shall modify the related part of the UE context accordingly and send the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message back to the M-NG-RAN node.

The S-NG-RAN node shall include the PDU sessions for which resources have been either added or modified or released at the S-NG-RAN node either in the PDU Session Resources Admitted To Be Added List IE or the PDU Session Resources Admitted To Be Modified List IE or the PDU Session Resources Admitted To Be Released List IE. The S-NG-RAN node shall include the PDU sessions that have not been admitted in the PDU Session Resources Not Admitted List IE with an appropriate cause value.

If the PDU Session level QoS parameter IE is included in the S-NODE MODIFICATION REQUEST message for an PDU session resource to be modified the S-NG-RAN node shall allocate respective resources and provide corresponding radio configuration information within the S-NG-RAN node to M-NG-RAN node Container IE as specified in TS 37.340 [8].

If the S-NODE MODIFICATION REQUEST message contains for an PDU session resource to be modified which is configured with the SCG bearer option the NG UL UP Address IE the S-NG-RAN node shall use it as the new UL NG-U address.

If the S-NODE MODIFICATION REQUEST message contains for an PDU session resource to be modified which is configured with the split bearer option the M-NG-RAN node UP Address IE the S-NG-RAN node shall use it as the new UL Xn-U address.

For an PDU session resource to be modified which is configured with the SCG bearer option the S-NG-RAN node may include in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message the NG DL UP Address IE.

For an PDU session resource to be modified which is configured with the split bearer option the S-NG-RAN node may include in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message the S-NG-RAN node UP Address IE.

If the SCG Change Indication IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall act as specified in TS 37.340 [8].

Upon reception of the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message the M-NG-RAN node shall stop the timer TXnDCprep. If the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message has included the S-NG-RAN node to M-NG-RAN node Container IE the M-NG-RAN node is then defined to have a Prepared S-NG-RAN node Modification for that Xn UE-associated signalling.

If the SCG Configuration Query IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall provide corresponding radio configuration information within the S-NG-RAN node to M-NG-RAN node Container IE as specified in TS 37.340 [8].

If the S-NODE MODIFICATION REQUEST message contains the Requested Split SRBs IE, the S-NG-RAN node may use it to add split SRBs. If the S-NODE MODIFICATION REQUEST message contains the Requested Split SRBs release IE, the S-NG-RAN node may use it to release split SRBs.

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits a modification of the UE context requiring the M-NG-RAN node to report about the success of the RRC connection reconfiguration procedure, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message to the M-NG-RAN node. The reception of the S-NG-RAN node RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall.

#### 8.3.3.3	Unsuccessful Operation

Figure 8.3.3.3-1: M-NG-RAN node initiated S-NG-RAN node Modification Preparation, unsuccessful operation

If the S-NG-RAN node does not admit any modification requested by the M-NG-RAN node, or a failure occurs during the M-NG-RAN node initiated S-NG-RAN node Modfication Preparation, the S-NG-RAN node shall send the S-NODE MODIFICATION REQUEST REJECT message to the M-NG-RAN node. The message shall contain the Cause IE with an appropriate value.

If the S-NG-RAN node receives a S-NODE MODIFICATION REQUEST message containing the M-NG-RAN node to S-NG-RAN node Container IE that does not include required information as specified in TS 37.340 [8], the S-NG-RAN node shall send the S-NODE MODIFICATION REQUEST REJECT message to the M-NG-RAN node.

#### 8.3.3.4	Abnormal Conditions

Void.

### 8.3.4	S-NG-RAN node initiated S-NG-RAN node Modification

8.3.4.1	General

This procedure is used by the S-NG-RAN node to modify the UE context in the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: S-NG-RAN node initiated S-NG-RAN node Modification, successful operation.

The S-NG-RAN node initiates the procedure by sending the S-NODE MODIFICATION REQUIRED message to the M-NG-RAN node.

When the S-NG-RAN node sends the S-NODE MODIFICATION REQUIRED message, it shall start the timer TXnDCoverall.

The S-NODE MODIFICATION REQUIRED message may contain

-	the S-NG-RAN node to M-NG-RAN node Container IE.

-	PDU session resources to be released within the PDU Session Resources To Be Released Item IE;

-	the SCG Change Indication IE.

If the M-NG-RAN node receives a S-NODE MODIFICATION REQUIRED message containing the SCG Change Indication IE, the M-NG-RAN node shall act as specified in TS 37.340 [8].

If the M-NG-RAN node is able to perform the modifications requested by the S-NG-RAN node, the M-NG-RAN node shall send the S-NODE MODIFICATION CONFIRM message to the S-NG-RAN node. The S-NODE MODIFICATION CONFIRM message may contain the M-NG-RAN node to S-NG-RAN node Container IE.

Upon reception of the S-NODE MODIFICATION CONFIRM message the S-NG-RAN node shall stop the timer TXnDCoverall.

#### 8.3.4.3	Unsuccessful Operation

Figure 8.3.4.3-1: S-NG-RAN node initiated S-NG-RAN node Modification, unsuccessful operation.

In case the request modification cannot be performed successfully the M-NG-RAN node shall respond with the S-NODE MODIFICATION REFUSE message to the S-NG-RAN node with an appropriate cause value in the Cause IE.

The M-NG-RAN node may also provide configuration information in the M-NG-RAN node to S-NG-RAN node Container IE.

#### 8.3.4.4	Abnormal Conditions

Void.

### 8.3.5	S-NG-RAN node initiated S-NG-RAN node Change

#### 8.3.5.1	General

This procedure is used by the S-NG-RAN node to trigger the change of the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.5.2	Successful Operation

Figure 8.3.5.2-1: S-NG-RAN node initiated S-NG-RAN node Change, successful operation.

The S-NG-RAN node initiates the procedure by sending the S-NODE CHANGE REQUIRED message to the M-NG-RAN node.

#### 8.3.5.3	Unsuccessful Operation

Figure 8.3.5.3-1: S-NG-RAN node initiated S-NG-RAN node Change, unsuccessful operation.

In case the request modification cannot accept the request to change the S-NG-RAN node the the M-NG-RAN node shall respond with the S-NODE CHANGE REFUSE message to the S-NG-RAN node with an appropriate cause value in the Cause IE.

#### 8.3.5.4	Abnormal Conditions

Void.

### 8.3.6	M-NG-RAN node initiated S-NG-RAN node Release

8.3.6.1	General

The M-NG-RAN node initiated S-NG-RAN node Release procedure is triggered by the M-NG-RAN node to initiate the release of the resources for a specific UE.

The procedure uses UE-associated signalling.

#### 8.3.6.2	Successful Operation

Figure 8.3.6.2-1: M-NG-RAN node initiated S-NG-RAN node Release, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE RELEASE REQUEST message. Upon reception of the S-NODE RELEASE REQUEST message the S-NG-RAN node shall stop providing user data to the UE.

The S-NG-RAN node UE XnAP ID IE shall be included if it has been obtained from the S-NG-RAN node. The M-NG-RAN node shall provide appropriate information within the Cause IE.

If the PDU session resource context in the S-NG-RAN node was configured with the SCG bearer option, for each SCG bearer for which the M-NG-RAN node requests forwarding of uplink/downlink data, the M-NG-RAN node includes the UL Forwarding UP address/ DL Forwarding UP Address IE within the PDU Session Resources To Be Released Item IE of the S-NODE RELEASE REQUEST message to indicate that the S-NG-RAN node should perform data forwarding of uplink/downlink packets for that SCG bearer.

Upon reception of the S-NODE RELEASE REQUEST message containing UE Context Kept Indicator IE set to “True”, the S-NG-RAN node shall, if supported, only initiate the release of the resources related to the UE-associated signalling connection between the M-NG-RAN node and the S-NG-RAN node.

If the S-NG-RAN node confirms the request to release S-NG-RAN node resources it shall send the S-NODE RELEASE REQUEST ACKNOWLEDGE message to the M-NG-RAN node.

#### 8.3.6.3	Unsuccessful Operation

Figure 8.3.6.3-1: M-NG-RAN node initiated S-NG-RAN node Release, unsuccessful operation

If the S-NG-RAN node cannot confirms the request to release S-NG-RAN node resources it shall send the S-NODE  RELEASE REJECT message to the M-NG-RAN node with an appropriate cause indicated in the Cause IE.

#### 8.3.6.4	Abnormal Conditions

Void.

### 8.3.7	S-NG-RAN node initiated S-NG-RAN node Release

8.3.7.1	General

This procedure is triggered by the S-NG-RAN node to initiate the release of the resources for a specific UE.

The procedure uses UE-associated signalling.

#### 8.3.7.2	Successful Operation

Figure 8.3.7.2-1: S-NG-RAN node initiated S-NG-RAN node Release, successful operation.

The S-NG-RAN node initiates the procedure by sending the S-NODE RELEASE REQUIRED message to the M-NG-RAN node.

Upon reception of the S-NODE RELEASE REQUIRED message, the M-NG-RAN node replies with the S-NODE RELEASE CONFIRM message.

For each PDU session resource configured with the SCG bearer option, the M-NG-RAN node may include the DL Forwarding UP Address IE and the UL Forwarding UP Address IE within the PDU Session Resources To Be Released Item IE to indicate that it requests data forwarding of uplink and downlink packets to be performed for that bearer.

The S-NG-RAN node may start data forwarding and stop providing user data to the UE upon reception of the S-NODE RELEASE CONFIRM message,

#### 8.3.7.3	Unsuccessful Operation

Not applicable.

#### 8.3.7.4	Abnormal Conditions

Void.

### 8.3.8	S-NG-RAN node Counter Check

8.3.8.1	General

This procedure is initiated by the S-NG-RAN node to request the M-NG-RAN node to execute a counter check procedure to verify the value of the PDCP COUNTs associated with SCG bearers established in the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.8.2	Successful Operation

Figure 8.3.8.2-1: S-NG-RAN node Counter Check procedure, successful operation.

The S-NG-RAN node initiates the procedure by sending the S-NODE COUNTER CHECK REQUEST message to the M-NG-RAN node.

Upon reception of the S-NODE COUNTER CHECK REQUEST message, the M-NG-RAN node may perform the RRC counter check procedure as specified in TS 38.331 [10].

#### 8.3.8.3	Unsuccessful Operation

Not applicable.

#### 8.3.8.4	Abnormal Conditions

Void.

### 8.3.9	RRC Transfer

#### 8.3.9.1	General

The purpose of the RRC Transfer procedure is to deliver an LTE RRC message encapsulated in a PDCP-C PDU to the S-NG-RAN-NODE that it may then be forwarded to the UE, or from the S-NG-RAN-NODE, if it was received from the UE. Delivery status may also be provided from the S-NG-RAN-NODE to the M-NG-RAN-NODE using the RRC Transfer.

The procedure is also used to enable transfer of the NR RRC message container with the NR measurements from the M-NG-RAN-NODE to the S-NG-RAN-NODE, when received from the UE.

The procedure uses UE-associated signalling.

#### 8.3.9.2	Successful Operation

Figure 8.3.9.2-1: RRC Transfer procedure, successful operation.

The M-NG-RAN-NODE initiates the procedure by sending the RRC TRANSFER message to the S-NG-RAN-NODE or the S-NG-RAN-NODE initiates the procedure by sending the RRC TRANSFER message to the M-NG-RAN-NODE.

If the S-NG-RAN-NODE receives a RRC TRANSFER message without the RRC Container IE in the Split SRB IE, or with the Delivery Status IE in the Split SRB IE, it shall ignore the message. If the S-NG-RAN-NODE receives the RRC Container IE in the Split SRB IE, it shall deliver the contained RRC message to the UE.

If the M-NG-RAN-NODE receives the Delivery Status IE in the Split SRB IE the M-NG-RAN-NODE shall consider RRC messages up to the indicated NR PDCP SN as having been successfully delivered to UE by S-NG-RAN-NODE.

#### 8.3.9.3	Abnormal Conditions

In case of the split SRBs, the receiving node may ignore the message, if the M-NG-RAN-NODE has not indicated possibility of RRC transfer at the bearer setup.

## 8.4	Global procedures

### 8.4.1	Xn Setup

#### 8.4.1.1	General

The purpose of the Xn Setup procedure is to exchange application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface.

The procedure uses non UE-associated signalling.

#### 8.4.1.2	Successful Operation

Figure 8.4.1.2: Xn Setup, successful operation

The NG-RAN node1 initiates the procedure by sending the XN SETUP REQUEST message to the candidate NG-RAN node2. The candidate NG-RAN node2 replies with the XN SETUP RESPONSE message.

If Supplementary Uplink is configured at the NG-RAN node1, the NG-RAN node1 shall include in the XN SETUP REQUEST message the SUL Information IE and the Supported SUL band List IE for each served cell where supplementary uplink is configured.

If Supplementary Uplink is configured at the NG-RAN node2, the candidate NG-RAN node2 shall include in the XN SETUP RESPONSE message the SUL Information IE and the Supported SUL band List IE for each served cell where supplementary uplink is configured.

The NG-RAN node1 may include the TAI Slice Support List IE in the XN SETUP REQUEST message. The candidate NG-RAN node2 may also include TAI Slice Support List IE in the XN SETUP RESPONSE message. The NG-RAN node receiving the IE may use it accordingly.

8.4.1.3	Unsuccessful Operation

Figure 8.4.1.3-1: Xn Setup, unsuccessful operation

If the candidate NG-RAN node2 cannot accept the setup it shall respond with an XN SETUP FAILURE message with appropriate cause value.

8.4.1.4	Abnormal Conditions

Void.

8.4.2	NG-RAN node Configuration Update

#### 8.4.2.1	General

The purpose of the NG-RAN node Configuration Update procedure is to update application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface.

The procedure uses non UE-associated signalling.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: NG-RAN node Configuration Update, successful operation

The NG-RAN node1 initiates the procedure by sending an NG-RAN NODE CONFIGURATION UPDATE message to a peer NG-RAN node2.

If Supplementary Uplink is configured at the NG-RAN node1, the NG-RAN node1 shall include in the NG-RAN NODE CONFIGURATION UPDATE message the SUL Information IE and the Supported SUL band List IE for each cell added in the Served NR Cells To Add IE and in the Served NR Cells To Modify IE.

NOTE:	The text above on Supplementary Uplink may need to be refined once details on NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message are available.

If the TAI Support List IE is included in the NG-RAN NODE CONFIGURATION UPDATE message, the receiving node shall, if supported, replace any previously provided TAI Support List IE by the received TAI Support List IE.

If the Cell Assistance Information NR IE is present, the NG-RAN node2 may use it to generate the List of Served NR Cells IE and include the list in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

Upon reception of an NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall update the information for NG-RAN node1 as follows:

Update of Served Cell Information NR:

-	If Served Cells NR To Add IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall add cell information according to the information in the Served Cell Information NR IE.

-	If Served Cells NR To Modify IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall modify information of cell indicated by Old NR-CGI IE according to the information in the Served Cell Information NR IE.

-	If Served Cells NR To Delete IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall delete information of cell indicated by Old NR-CGI IE.

Update of Served Cell Information E-UTRA:

-	If Served Cells E-UTRA To Add IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall add cell information according to the information in the Served Cell Information E-UTRA IE.

-	If Served Cells E-UTRA To Modify IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall modify information of cell indicated by Old ECGI IE according to the information in the Served Cell Information E-UTRA IE.

-	If Served Cells E-UTRA To Delete IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall delete information of cell indicated by Old ECGI IE.

NOTE: 	the following alignment with Energy saving should be considered when details available (tabular not yet available) and may need to be refined: If the Deactivation Indication IE is contained in Served NR Cells To Modify IE, it indicates that the concerned NR cell was switched off to lower energy consumption, and is available for activation on request from the eNB

#### 8.4.2.3	Unsuccessful Operation

Figure 8.4.2.3-1: NG-RAN node Configuration Update, unsuccessful operation

If the NG-RAN node2 cannot accept the update it shall respond with an NG-RAN NODE CONFIGURATION UPDATE FAILURE message and appropriate cause value.

8.4.2.4	Abnormal Conditions

Void.

8.4.3	Cell Activation

#### 8.4.3.1	General

The purpose of the Cell Activation procedure is to enable an NG-RAN node to request a neighbouring NG-RAN node to switch on one or more cells, previously reported as inactive due to energy saving.

The procedure uses non UE-associated signalling.

#### 8.4.3.2	Successful Operation

Figure 8.4.3.2-1: EN-DC Cell Activation, successful operation

An NG-RAN node1 initiates the procedure by sending a CELL ACTIVATION REQUEST message to a peer NG-RAN node2.

Upon receipt of this message, the NG-RAN node2 should activate the cell/s indicated in the CELL ACTIVATION REQUEST message and shall indicate in the CELL ACTIVATION RESPONSE message for which cells the request was fulfilled.

Interactions with NG-RAN Configuration Update procedure:

The NG-RAN node2 shall not send an NG-RAN CONFIGURATION UPDATE message to the NG-RAN node1 just for the reason of the cell/s indicated in the CELL ACTIVATION REQUEST message changing cell activation state, as the receipt of the CELL ACTIVATION RESPONSE message by the NG-RAN node1 is used to update the information about the activation state of NG-RAN node2 cells in the NG-RAN node1.

#### 8.4.3.3	Unsuccessful Operation

Figure 8.4.3.3-1: EN-DC Cell Activation, unsuccessful operation

If the NG-RAN node2 cannot activate any of the cells indicated in the CELL ACTIVATION REQUEST message, it shall respond with a CELL ACTIVATION FAILURE message with an appropriate cause value.

#### 8.4.3.4	Abnormal Conditions

Void.

### 8.4.4	Reset

#### 8.4.4.1	General

The purpose of the Reset procedure is to align the resources in the NG-RAN node1 and the NG-RAN node2 in the event of an abnormal failure. The procedure either resets the Xn interface or selected UE contexts. This procedure doesn’t affect the application level configuration data exchanged during, e.g., the Xn Setup procedure.

The procedure uses non UE-associated signalling.

#### 8.4.4.2	Successful Operation

Figure 8.4.4.2-1: Reset, successful operation

- The procedure is initiated with a RESET REQUEST message sent from the NG-RAN node1 to the NG-RAN node2. Upon receipt of this message,

-	if the RESET REQUEST message indicates full reset the NG-RAN node2 shall abort any other ongoing procedures over Xn between the NG-RAN node1 and the NG-RAN node2. The NG-RAN node2 shall delete all the context information related to the NG-RAN node1, except the application level configuration data exchanged during the Xn Setup or the NG-RAN node Configuration Update procedures and release the corresponding resources. After completion of release of the resources, the NG-RAN node2 shall respond with a RESET RESPONSE message.

- -	if the RESET REQUEST message indicates partial reset, the NG-RAN node2 shall abort any other ongoing procedures only for the indicated UE associated signalling connections identified either by the NG-RAN node1 UE XnAP ID IE or the NG-RAN node1 UE XnAP ID IE or both, for which the NG-RAN node2 shall delete all the context information related to the NG-RAN node1 and release the corresponding resources. After completion of release of the resources, the NG-RAN node2 shall respond with a RESET RESPONSE message indicating the UE contexts admitted to be released. The NG-RAN node2 receiving the request for partial reset does not need to wait for the release or reconfiguration of radio resources to be completed before returning the RESET RESPONSE message. The NG-RAN node2 receiving the request for partial reset shall include in the RESET RESPONSE message, for each UE association to be released, the same list of UE-associated logical Xn-connections over Xn. The list shall be in the same order as received in the RESET REQUEST message and shall include also unknown UE-associated logical Xn-connections.

Interactions with other procedures:

If the RESET REQUEST message indicates full reset, the NG-RAN node2 shall abort any other ongoing procedure (except for a Reset procedures).

If the RESET REQUEST message indicates partial reset, the NG-RAN node2 shall abort any other ongoing procedure (except for a Reset procedures) on the same Xn interface related to a UE associated signalling connection indicated in the RESET REQUEST message.

#### 8.4.4.3	Unsuccessful Operation

Void.

#### 8.4.4.4	Abnormal Conditions

If the RESET REQUEST message is received, any other ongoing procedure (except another Reset procedure) on the same Xn interface shall be aborted.

If Reset procedure is ongoing and the responding node receives the RESET REQUEST message from the peer entity on the same Xn interface, it shall respond with the RESET RESPONSE message as specified in 8.4.4.2.

If the initiating node does not receive RESET RESPONSE message, the initiating node may reinitiate the Reset procedure towards the same NG-RAN node, provided that the content of the new RESET REQUEST message is identical to the content of the previously unacknowledged RESET REQUEST message.

### 8.4.5	Error Indication

#### 8.4.5.1	General

The Error Indication procedure is initiated by an NG-RAN node to report detected errors in one incoming message, provided they cannot be reported by an appropriate failure message.

If the error situation arises due to reception of a message utilising UE associated signalling, then the Error Indication procedure uses UE-associated signalling. Otherwise the procedure uses non UE-associated signalling.

#### 8.4.5.2	Successful Operation

- Figure 8.4.5.2-1: Error Indication, successful operation.

When the conditions defined in clause 10 are fulfilled, the Error Indication procedure is initiated by an ERROR INDICATION message sent from the node detecting the error situation.

The ERROR INDICATION message shall contain at least either the Cause IE or the Criticality Diagnostics IE.

In case the Error Indication procedure is triggered by UE associated signalling, in the course of handover signalling and signalling for dual connectivity, the Old NG-RAN node UE XnAP ID IE and the New NG-RAN node UE XnAP ID IE shall be included in the ERROR INDICATION message. If any of Old NG-RAN node UE XnAP ID IE and New NG-RAN node UE XnAP ID IE is not correct, the cause shall be set to an appropriate value.

#### 8.4.5.3	Unsuccessful Operation

Not applicable.

#### 8.4.5.4	Abnormal Conditions

Void.

### 8.4.6	Xn Removal

#### 8.4.6.1	General

The purpose of the Xn Removal procedure is to remove the signaling connection between two NG-RAN nodes in a controlled manner. If successful, this procedure erases any existing application level configuration data in the two nodes.

The procedure uses non UE-associated signaling.

#### 8.4.6.2	Successful Operation

Figure 8.4.6.2-1: Xn Removal, successful operation

An NG-RAN node1 initiates the procedure by sending the XN REMOVAL REQUEST message to a candidate NG-RAN node2. Upon reception of the XN REMOVAL REQUEST message the candidate NG-RAN node2 shall reply with the XN REMOVAL RESPONSE message. After receiving the XN REMOVAL RESPONSE message, the initiating NG-RAN node1 shall initiate removal of the TNL association towards NG-RAN node2 and may remove all resources associated with that signaling connection. The candidate NG-RAN node2 may then remove all resources associated with that signaling connection.

If the Xn Removal Threshold IE is included in the XN REMOVAL REQUEST message, the candidate NG-RAN node2 shall, if supported, accept to remove the signalling connection with NG-RAN node1 if the Xn Benefit Value of the signalling connection determined at the candidate NG-RAN node2 is lower than the value of the Xn Removal Threshold IE.

#### 8.4.6.3	Unsuccessful Operation

Figure 8.4.6.3-1: Xn Removal, unsuccessful operation

If the candidate NG-RAN node2 cannot accept to remove the signaling connection with NG-RAN node1 it shall respond with an XN REMOVAL FAILURE message with an appropriate cause value.

#### 8.4.6.4	Abnormal Conditions

Void.

# 9	Elements for XnAP Communication

## 9.0	General

Sub clauses 9.1 and 9.2 describe the structure of the messages and information elements required for the XnAP protocol in tabular format. Sub clause 9.3 provides the corresponding ASN.1 definition.

The following attributes are used for the tabular description of the messages and information elements: Presence, Range Criticality and Assigned Criticality. Their definition and use can be found in TS 38.413 [5].

NOTE:	The messages have been defined in accordance to the guidelines specified in TR 25.921 [6].

## 9.1	Message Functional Definition and Content

### 9.1.1	Messages for Basic Mobility Procedures

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.2.26 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.2.40 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.2.39 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.2.67 | Allocated at the AMF on the source NG-C connection . | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.2.29 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. | – |  |
| >UE Security Capabilities | M |  | 9.2.2.64 |  |  |  |
| >AS Security Information | M |  | 9.2.2.65 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.2.38 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.2.36 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition the source side QoS flow  DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.2.48 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.2.68 |  | – |  |
| Trace Activation | O |  | 9.2.2.45 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.2.41 |  | YES | ignore |

9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target NG-RAN node to inform the source NG-RAN node about the prepared resources at the target.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the source NG-RAN node | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the target NG-RAN node | YES | ignore |
| PDU Session Resources Admitted List | M |  | 9.2.1.2 |  | YES | ignore |
| PDU Session Resources Not Admitted List | O |  | 9.2.1.4 |  | YES | ignore |
| Target NG-RAN node To Source NG-RAN node Transparent Container | M |  | OCTET STRING | Either includes the HandoverCommand message as defined in subclause 10.2.2 of TS 36.331 [14] , if the target NG-RAN node is an ng-eNB, or the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10] , if the target NG-RAN node is a gNB. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2.32 |  | YES | ignore |

9.1.1.3	HANDOVER PREPARATION FAILURE

This message is sent by the target NG-RAN node to inform the source NG-RAN node that the Handover Preparation has failed.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the source NG-RAN node | YES | ignore |
| Cause | M |  | 9.2.2.26 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2.32 |  | YES | ignore |

9.1.1.4	SN STATUS TRANSFER

This message is sent by the source NG-RAN node to the target NG-RAN node to transfer the uplink/downlink PDCP SN and HFN status during a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | ignore |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated for handover at the source NG-RAN node | YES | reject |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated for handover at the target NG-RAN node | YES | reject |
| DRBs Subject To Status Transfer List |  | 1 | 9.2.1.15 |  | YES | ignore |

9.1.1.5	UE CONTEXT RELEASE

This message is sent by the target NG-RAN node to the source NG-RAN node to indicate that resources can be released.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | ignore |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated for handover at the source NG-RAN node. | YES | reject |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated for handover at the target NG-RAN node. | YES | reject |

9.1.1.6	HANDOVER CANCEL

This message is sent by the source NG-RAN node to the target NG-RAN node to cancel an ongoing handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | ignore |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the source NG-RAN node | YES | reject |
| Target NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the target NG-RAN node | YES | ignore |
| Cause | M |  | 9.2.2.26 |  | YES | ignore |

9.1.1.7	RAN PAGING

This message is sent by the NG-RAN node1 to NG-RAN node2 to page a UE.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| UE Identity Index Value | M |  | INTEGER (SIZE(0..63)) | This IE may need to be refined | YES | reject |
| UE RAN Paging Identity | M |  | 9.2.2.75 |  | YES | ignore |
| Paging DRX | M |  | INTEGER (SIZE(0..63)) | This IE may need to be refined | YES | ignore |
| RAN Paging Area | M |  | 9.2.2.46 |  | YES | reject |
| RAN Paging Priority | O |  | 9.2.2.27 |  | YES | ignore |
| Assistance Data for Paging | O |  | 9.2.2.74 |  | YES | ignore |

9.1.1.8	RETRIEVE UE CONTEXT REQUEST

This message is sent by the new NG-RAN node to request the old NG-RAN node to transfer the UE Context to the new NG-RAN.

Direction: new NG-RAN node  old NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | ignore |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the new NG-RAN node | YES | reject |
| UE Context ID | M |  | 9.2.2.71 |  | YES | reject |
| Integrity protection | M |  | BIT STRING (SIZE (16)) | ShortMAC-I either contained in the RRCConnection ResumeRequest message as defined in TS 38.331 [10]) or in the RRCConnection ResumeRequest message as defined in TS 36.331 [14]) | YES | reject |
| New Cell Identifier | M |  | NG-RAN Cell Identity 9.2.2.42 | The Cell Identifier of the cell where the RRC connection has been requested to be resumed or to be re-established. | YES | reject |

9.1.1.9	RETRIEVE UE CONTEXT RESPONSE

This message is sent by the old NG-RAN node to transfer the UE context to the new NG-RAN node.

Direction: old NG-RAN node  new NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | ignore |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the new NG-RAN node | YES | ignore |
| Old NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the old NG-RAN node | YES | ignore |
| GUAMI | M |  | 9.2.2.39 |  | YES | reject |
| UE Context Information | M |  | 9.2.1.14 |  | YES | reject |
| Trace Activation | O |  | 9.2.2.45 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.2.41 |  | YES | ignore |
| Location Reporting Information | O |  | 9.2.2.48 | Includes the necessary parameters for location reporting. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2.32 |  | YES | ignore |

#### 9.1.1.10	RETRIEVE UE CONTEXT FAILURE

This message is sent by the old NG-RAN node to inform the new NG-RAN node that the Retrieve UE Context procedure has failed.

Direction: old NG-RAN node  new NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | ignore |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the new NG-RAN node | YES | ignore |
| Cause | M |  | 9.2.2.26 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2.32 |  | YES | ignore |

9.1.1.11	DATA FORWARDING ADDRESS INDICATION

This message is sent by the new NG-RAN node to transfer data forwarding information to the new NG-RAN node.

Direction: new NG-RAN node  old NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | ignore |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the new NG-RAN node | YES | ignore |
| Old NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the old NG-RAN node | YES | ignore |
| Data Forwarding Info per PDU Session Resources |  | 1 |  |  | – |  |
| > Data Forwarding Info per PDU Session Resources Item IEs |  | 1..<maxnoofPDUSessions> |  |  | EACH | reject |
| >>PDU Session ID | M |  | 9.2.2.35 |  | – |  |
| >>DL Forwarding UP TNL Information | O |  | UP Transport Layer Information 9.2.2.28 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |

### 9.1.2	Messages for Dual Connectivity Procedures

Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018, all the text of section 9.1.2 is FFS.

9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.2.64 | This IE may need to be refined. | YES | reject |
| S-NG-RAN node Security Key | M |  | 9.2.2.66 |  | YES | reject |
| S-NG-RAN node UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.2.36 | The UE Aggregate Maximum Bit Rate is split into M-NG-RAN node UE Aggregate Maximum Bit Rate and S-NG-RAN node UE Aggregate Maximum Bit Rate which are enforced by M-NG-RAN node and S-NG-RAN node respectively. | YES | reject |
| Selected PLMN | O |  | PLMN Identity 9.2.2.25 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Mobility Restriction List | O |  | 9.2.2.68 | This IE may need to be refined. | YES | ignore |
| Index to RAT/Frequency Selection Priority | O |  | 9.2.2.38 |  | YES | reject |
| PDU Session Resources To Be Added List |  | 1 |  |  | YES | reject |
| >PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | reject |
| >>PDU Session ID | M |  | 9.2.2.35 |  | – |  |
| >>S-NSSAI | M |  | 9.2.2.18 |  | – |  |
| >>Bearer Configurations To Be Added |  | 1 .. <maxnoofBearerConfigs> |  |  | EACH | reject |
| >>>CHOICE Bearer Configuration | M |  |  |  | – |  |
| >>>>SN terminated Bearer |  |  |  |  | – |  |
| >>>>>PDU Session Resource Setup Info – SN terminated | M |  | 9.2.1.6 |  | – |  |
| >>>>MN terminated Bearer |  |  |  |  | – |  |
| >>>>>PDU Session Resource Setup Info – MN terminated | M |  | 9.2.1.8 |  | – |  |
| M-NG-RAN node to S-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the S-NG-RAN node | YES | reject |
| Expected UE Behaviour | O |  | OCTET STRING | This IE may need to be revised | YES | ignore |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofBearerConfigs | Maximum no. of BearerConfigurations Value is 2. The value may need to be refined. So far, only MN- and SN-terminated bearer configurations are considered. |

#### 9.1.2.2	S-NODE ADDITION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node about the S-NG-RAN node addition preparation.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the S-NG-RAN node | YES | reject |
| PDU Session Resources Admitted To Be Added List |  | 1 |  |  | YES | ignore |
| >PDU Session Resources Admitted To Be Added Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>PDU Session ID | M |  | 9.2.2.35 |  | – |  |
| >>Bearer Configurations Admitted To Be Added |  | 1 .. <maxnoofBearerConfigs> |  |  | EACH | ignore |
| >>>CHOICE Bearer Configuration | M |  |  |  | – |  |
| >>>>SN terminated Bearer |  |  |  |  | – |  |
| >>>>> PDU Session Resource Setup Response Info – SN terminated | M |  | 9.2.1.7 |  | – |  |
| >>>>MN terminated Bearer |  |  |  |  | – |  |
| >>>>>PDU Session Resource Setup Response Info – MN terminated | M |  | 9.2.1.9 |  | – |  |
| PDU Session Resources Not Admitted List | O |  |  |  | YES | ignore |
| >PDU Session Resources Not Admitted List – SN terminated | O |  | PDU Session Resources Not Admitted List 9.2.1.4 | A value for PDU Session ID shall only be present once in PDU Session Resources Admitted List IE and in PDU Session Resources Not Admitted List IE. | – |  |
| >PDU Session Resources Not Admitted List – MN terminated | O |  | PDU Session Resources Not Admitted List 9.2.1.4 | A value for PDU Session ID shall only be present once in PDU Session Resources Admitted List IE and in PDU Session Resources Not Admitted List IE. | – |  |
| S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | reject |
| Admitted Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs | YES | reject |
| Criticality Diagnostics | O |  | 9.2.2.32 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofBearerConfigs | Maximum no. of BearerConfigurations Value is 2. This value may need to be refined. So far, only MN- and SN-terminated bearer configurations are considered. |

#### 9.1.2.3	S-NODE ADDITION REQUEST REJECT

This message is sent by the S-NG-RAN node to inform the M-NG-RAN node that the S-NG-RAN node Addition Preparation has failed.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | 9.2.2.26 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2.32 |  | YES | ignore |

#### 9.1.2.4	S-NODE RECONFIGURATION COMPLETE

This message is sent by the M-NG-RAN node to the S-NG-RAN node to indicate whether the configuration requested by the S-NG-RAN node was applied by the UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the S-NG-RAN node | YES | reject |
| Response Information | M |  |  |  | YES | ignore |
| >CHOICE Response Type | M |  |  |  | – |  |
| >>Configuration successfully applied |  |  |  |  | – |  |
| >>>M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the RRCReconfigurationComplete message as defined in subclause 6.2.2 of TS 38.331 [10]. | – |  |
| >>Configuration rejected by the M-NG-RAN node |  |  |  |  | – |  |
| >>>Cause | M |  | 9.2.2.26 |  | – |  |
| >>>M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in as defined in subclause 11.2.2 of TS 38.331. | – |  |

#### 9.1.2.5	S-NODE MODIFICATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation to modify S-NG-RAN node resources for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | 9.2.2.26 |  | YES | ignore |
| SCG Change Indication | O |  | ENUMERATED (true, ...) | This IE may need to be refined. | YES | ignore |
| Selected PLMN | O |  | PLMN Identity 9.2.2.25 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Mobility Restriction List | O |  | 9.2.2.68 | This IE may need to be refined. | YES | ignore |
| SCG Configuration Query | O |  | 9.2.2.11 |  | YES | ignore |
| UE Context Information |  | 0..1 |  |  | YES | reject |
| >UE Security Capabilities | O |  | 9.2.2.64 | This IE may need to be refined. | – |  |
| >S-NG-RAN node Security Key | O |  | 9.2.2.66 |  | – |  |
| >S-NG-RAN node UE Aggregate Maximum Bit Rate | O |  | UE Aggregate Maximum Bit Rate 9.2.2.36 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.2.38 |  | – |  |
| >PDU Session Resources To Be Added List |  | 0..1 |  |  | – |  |
| >>PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>>PDU Session ID | M |  | 9.2.2.35 |  | – |  |
| >>>S-NSSAI | M |  | 9.2.2.18 |  | – |  |
| >>>Bearer Configurations To Be Added |  | 1 .. <maxnoofBearerConfigs> |  |  | EACH | reject |
| >>>>CHOICE Bearer Configuration | M |  |  |  | – |  |
| >>>>>SN terminated Bearer |  |  |  |  | – |  |
| >>>>>PDU Session Resource Setup Info – SN terminated | M |  | 9.2.1.6 |  | – |  |
| >>>>>MN terminated Bearer |  |  |  |  | – |  |
| >>>>>>PDU Session Resource Setup Info – MN terminated |  |  | 9.2.1.8 |  | – |  |
| >PDU Session Resources To Be Modified List |  | 0..1 |  |  | – |  |
| >>PDU Session Resources To Be Modified Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>>PDU Session ID | M |  | 9.2.2.35 |  | – |  |
| >>>Bearer Configurations To Be Modified |  | 1 .. <maxnoofBearerConfigs> |  |  | EACH | reject |
| >>>>CHOICE Bearer Configuration | M |  |  |  | – |  |
| >>>>>SN terminated Bearer |  |  |  |  | – |  |
| >>>>>>PDU Session Resource Modification Info – SN terminated |  |  | 9.2.1.10 |  | – |  |
| >>>>>MN terminated Bearer |  |  |  |  | – |  |
| >>>>>>PDU Session Resource Modification Info – MN terminated |  |  | 9.2.1.12 |  | – |  |
| > PDU Session Resources To Be Released List |  | 0..1 |  |  | – |  |
| >>PDU Session Resources To Be Released List – SN terminated | O |  | OCTET STRING | A value for PDU Session ID shall only be present once in PDU Session Resources Admitted List IE and in PDU Session Resources Not Admitted List IE. This IE may need to be refined. | – |  |
| >>PDU Session Resources to Be Released List – MN terminated | O |  | OCTET STRING | A value for PDU Session ID shall only be present once in PDU Session Resources Admitted List IE and in PDU Session Resources Not Admitted List IE. This IE may need to be refined. | – |  |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2. of TS 38.331 [10] | YES | ignore |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. | YES | ignore |
| Requested Split SRBs release | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested to be released. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofBearerConfigs | Maximum no. of BearerConfigurations Value is 2. This value may need to be refined. So far, only MN- and SN-terminated bearer configurations are considered. |

#### 9.1.2.6	S-NODE MODIFICATION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node’s request to modify the S-NG-RAN node resources for a specific UE.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the S-NG-RAN node | YES | ignore |
| PDU Session Resources Admitted List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources Admitted To Be Added List |  | 1 |  |  | – |  |
| >>PDU Session Resources Admitted To Be Added Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>>PDU Session ID | M |  | 9.2.2.35 |  | – |  |
| >>>Bearer Configurations Admitted To Be Added |  | 1 .. <maxnoofBearerConfigs> |  |  | EACH | ignore |
| >>>>CHOICE Bearer Configuration | M |  |  |  | – |  |
| >>>>>SN terminated Bearer |  |  |  |  | – |  |
| >>>>>> PDU Session Resource Setup Response Info – SN terminated | M |  | 9.2.1.7 |  | – |  |
| >>>>>MN terminated Bearer |  |  |  |  | – |  |
| >>>>>>PDU Session Resource Setup Response Info – MN terminated | M |  | 9.2.1.9 |  | – |  |
| >PDU Session Resources Admitted To Be Modified List |  | 0..1 |  |  | – |  |
| >>PDU Session Resources Admitted To Be Modified Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>>PDU Session ID | M |  | 9.2.2.35 |  | – |  |
| >>>Bearer Configurations Admitted To Be Modified |  | 1 .. <maxnoofBearerConfigs> |  |  | EACH | ignore |
| >>>>CHOICE Bearer Configuration | M |  |  |  | – |  |
| >>>>>SN terminated Bearer |  |  |  |  | – |  |
| >>>>>>PDU Session Resource Modification Response Info – SN terminated | M |  | 9.2.1.11 |  | – |  |
| >>>>>MN terminated Bearer |  |  |  |  | – |  |
| >>>>>>PDU Session Resource Modification Response Info – MN terminated | M |  | 9.2.1.13 |  | – |  |
| >PDU Session Resources Admitted To Be Released List |  | 0..1 |  |  | – |  |
| >>PDU Session Resources admitted to be released List – SN terminated | O |  | OCTET STRING | A value for PDU Session ID shall only be present once in PDU Session Resources Admitted List IE and in PDU Session Resources Not Admitted List IE. This IE may need to be refined. | – |  |
| >>PDU Session Resources admitted to be released List – MN terminated | O |  | OCTET STRING | A value for PDU Session ID shall only be present once in PDU Session Resources Admitted List IE and in PDU Session Resources Not Admitted List IE. This IE may need to be refined. | – |  |
| PDU Session Resources Not Admitted to be Added List | O |  |  |  | YES | ignore |
| >PDU Session Resources Not Admitted List – SN terminated | M |  | PDU Session Resources Not Admitted List 9.2.1.4 | A value for PDU Session ID shall only be present once in PDU Session Resources Admitted List IE and in PDU Session Resources Not Admitted List IE. | – |  |
| >PDU Session Resources Not Admitted List – MN terminated | M |  | PDU Session Resources Not Admitted List 9.2.1.4 | A value for PDU Session ID shall only be present once in PDU Session Resources Admitted List IE and in PDU Session Resources Not Admitted List IE. | – |  |
| S-NG-RAN node to M-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | ignore |
| Admitted Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs | YES | ignore |
| Admitted Split SRBs release | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs release | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2.32 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |

#### 9.1.2.7	S-NODE MODIFICATION REQUEST REJECT

This message is sent by the S-NG-RAN node to inform the M-NG-RAN node that the M-NG-RAN node initiated S-NG-RAN node Modification Preparation has failed.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the S-NG-RAN node | YES | ignore |
| Cause | M |  | 9.2.2.26 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2.32 |  | YES | ignore |

#### 9.1.2.8	S-NODE MODIFICATION REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to request the modification of S-NG-RAN node resources for a specific UE.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | 9.2.2.26 |  | YES | ignore |
| SCG Change Indication | O |  | ENUMERATED (true, ...) | This IE may need to be refined. | YES | ignore |
| PDU Session Resources To Be Modified List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources To Be Modified Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>PDU Session ID | M |  | 9.2.2.35 |  | – |  |
| >>NG-U DL UP TNL Information at NG-RAN | O |  | UP Transport Layer Information 9.2.2.28 | S-NG-RAN node endpoint of the NG transport bearer. For delivery of DL PDUs. | – |  |
| PDU Session Resources To Be Released List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources To Be Released Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>PDU Session ID | M |  | 9.2.2.35 |  | – |  |
| >>Cause | M |  | 9.2.2.26 |  | – |  |
| S-NG-RAN node to M-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |

#### 9.1.2.9	S-NODE MODIFICATION CONFIRM

This message is sent by the M-NG-RAN node to inform the S-NG-RAN node about the successful modification.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the S-NG-RAN node | YES | ignore |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2.32 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |

#### 9.1.2.10	S-NODE MODIFICATION REFUSE

This message is sent by the M-NG-RAN node to inform the S-NG-RAN node that the S-NG-RAN node initiated S-NG-RAN node Modification has failed.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the S-NG-RAN node | YES | ignore |
| Cause | M |  | 9.2.2.26 |  | YES | ignore |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2.32 |  | YES | ignore |

#### 9.1.2.11	S-NODE CHANGE REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to trigger the change of the S-NG-RAN node.

Direction: S-NG-RAN node  M-NG-RAN node.

#### 9.1.2.12	S-NODE CHANGE CONFIRM

This message is sent by the M-NG-RAN node to inform the S-NG-RAN node that the preparation of the S-NG-RAN node initiated S-NG-RAN node Modification was successful.

Direction: M-NG-RAN node  S-NG-RAN node.

#### 9.1.2.13	S-NODE MODIFICATION REFUSE

This message is sent by the M-NG-RAN node to inform the S-NG-RAN node that the preparation of the S-NG-RAN node initiated S-NG-RAN node Modification has failed.

Direction: M-NG-RAN node  S-NG-RAN node.

#### 9.1.2.14	S-NODE RELEASE REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the release of resources.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | 9.2.2.26 |  | YES | ignore |
| PDU Session Resources To Be Released List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources To Be Released Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>CHOICE Bearer Option | M |  |  |  | – |  |
| >>>SCG Bearer |  |  |  |  | – |  |
| >>>>PDU Session ID | M |  | 9.2.2.35 |  | – |  |
| >>>>UL Forwarding UP TNL Information | O |  | UP Transport Layer Information 9.2.2.28 | Identifies the Xn transport bearer used for forwarding of UL PDUs | – |  |
| >>>>DL Forwarding UP TNL Information | O |  | UP Transport Layer Information 9.2.2.28 | Identifies the Xn transport bearer. used for forwarding of DL PDUs | – |  |
| >>>Split Bearer |  |  |  |  | – |  |
| >>>>PDU Session ID | M |  | 9.2.2.35 |  | – |  |
| >>>>DL Forwarding UP TNL Information | O |  | UP Transport Layer Information 9.2.2.28 | Identifies the Xn transport bearer. used for forwarding of DL PDUs | – |  |
| UE Context Kept Indicator | O |  | ENUMERATED (true, ...) | This IE may need to be refined. | YES | ignore |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |

#### 9.1.2.15	S-NODE RELEASE REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to the M-NG-RAN node to confirm the request to release S-NG-RAN node resources.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the S-NG-RAN node | YES | reject |
| Criticality Diagnostics | O |  | 9.2.2.32 |  | YES | ignore |

#### 9.1.2.16	S-NODE RELEASE REJECT

This message is sent by the S-NG-RAN node to the M-NG-RAN node to reject the request to release S-NG-RAN node resources.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | 9.2.2.26 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2.32 |  | YES | ignore |

#### 9.1.2.17	S-NODE RELEASE REQUIRED

This message is sent by the S-NG-RAN node to request the release of all resources for a specific UE at the S-NG-RAN node.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | 9.2.2.26 |  | YES | ignore |

#### 9.1.2.18	S-NODE RELEASE CONFIRM

This message is sent by the M-NG-RAN node to confirm the release of all resources for a specific UE at the S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the S-NG-RAN node | YES | ignore |
| PDU Session Resources to be Released List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources To Be Released Item |  | 1 .. <maxnoofPDUsessions> |  |  | – |  |
| >>CHOICE Bearer Option | M |  |  |  | – |  |
| >>>SCG Bearer |  |  |  |  | – |  |
| >>>>PDU Session ID | M |  | 9.2.2.35 |  | – |  |
| >>>>DL Forwarding UP TNL Information | O |  | UP Transport Layer Information 9.2.2.28 | Identifies the Xn transport bearer used for forwarding of DL PDUs | – |  |
| >>>>UL Forwarding UP TNL Information | O |  | UP Transport Layer Information 9.2.2.28 | Identifies the Xn transport bearer used for forwarding of UL PDUs | – |  |
| >>>Split Bearer |  |  |  |  | – |  |
| >>>>PDU Session ID | M |  | 9.2.2.35 |  | – |  |
| >>>>DL Forwarding UP TNL Information | O |  | UP Transport Layer Information 9.2.2.28 | Identifies the Xn transport bearer used for forwarding of DL PDUs | – |  |
| Criticality Diagnostics | O |  | 9.2.2.32 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |

#### 9.1.2.19	S-NODE COUNTER CHECK REQUEST

This message is sent by the S-NG-RAN node to request the verification of the value of the PDCP COUNTs associated with SN terminated bearers established in the S-NG-RAN node.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the S-NG-RAN node | YES | ignore |
| Bearers Subject to Counter Check List |  | 1 |  |  | YES | ignore |
| >Bearers Subject to Counter Check Item |  | 1 .. <maxnoofDRBss> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.2.2.30 |  | – |  |
| >>UL COUNT | M | INTEGER (0.. 4294967295) |  | Indicates the value of uplink COUNT associated to this DRB. | – |  |
| >>DL COUNT | M | INTEGER (0.. 4294967295) |  | Indicates the value of downlink COUNT associated to this DRB. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs. Value is 32 |

#### 9.1.2.20	RRC TRANSFER

This message is sent by the M-NG-RAN-NODE to the S-NG-RAN-NODE to transfer an RRC message.

Direction: M-NG-RAN node  S-NG-RAN node or S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the S-NG-RAN node | YES | reject |
| Split SRB |  | 0..1 |  |  | YES | reject |
| >RRC Container | O |  | OCTET STRING | RRC message encapsulated in a PDCP-C PDU and ciphered with the key of the M-NG-RAN node |  |  |
| >SRB Type | M |  | ENUMERATED (srb1, srb2, ...) | The SRB type to be used | – |  |
| >Delivery Status | O |  | 9.2.2.19 | DL RRC delivery status of split SRB | – |  |
| NR UE Measurement Report |  | 0..1 |  |  | YES | reject |
| >RRC Container | M |  | OCTET STRING | Contains the MeasurementReport message as defined in subclause 6.2.2 of TS 38.331 [10]. | – |  |

### 9.1.3	Messages for Global Procedures

#### 9.1.3.1	XN SETUP REQUEST

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer application data for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| Global NG-RAN Node ID | M |  | 9.2.2.9 |  | YES | reject |
| TAI Support List | O |  | 9.2.2.4 | List of supported TAs and associated characteristics. | GLOBAL | reject |
| AMF Pool information | M |  | OCTET STRING | List of all the AMF pools to which the NG-RAN node belongs. This IE may need to be refined. | GLOBAL | reject |
| List of Served Cells NR |  | 0 .. <maxnoofCellsinNG-RAN node> |  | Complete list of cells served by the gNB | YES | reject |
| >Served Cell Information NR | M |  | 9.2.2.13 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.61 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.62 |  | – |  |
| List of Served Cells E-UTRA | O | 0 .. <maxnoofCellsinNG-RAN node> |  | Complete list of cells served by the ng-eNB. | YES | reject |
| >Served Cell Information E-UTRA | M |  | 9.2.2.56 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.61 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.62 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

9.1.3.2	XN SETUP RESPONSE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer application data for an Xn-C interface instance.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| Global NG-RAN Node ID | M |  | 9.2.2.9 |  | YES | reject |
| TAI Support List | O |  | 9.2.2.4 | List of supported TAs and associated characteristics. | GLOBAL | reject |
| List of Served Cells NR |  | 0 .. <maxnoofCellsinNG-RAN node> |  | Complete list of cells served by the gNB | YES | reject |
| >Served Cell Information NR | M |  | 9.2.2.13 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.61 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.62 |  | – |  |
| List of Served Cells E-UTRA |  | 0 .. <maxnoofCellsinNG-RAN node> |  | Complete list of cells served by the ng-eNB | YES | reject |
| >Served Cell Information E-UTRA | M |  | 9.2.2.56 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.61 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.62 |  | – |  |
| Criticality Diagnostics | O |  | 9.2.2.32 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

9.1.3.3	XN SETUP FAILURE

This message is sent by the neighbouring NG-RAN node to indicate Xn Setup failure.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| Cause | M |  | 9.2.2.26 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2.32 |  | YES | ignore |

#### 9.1.3.4	NG-RAN NODE CONFIGURATION UPDATE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer updated information for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| TAI Support List | O |  | 9.2.2.4 | List of supported TAs and associated characteristics. | GLOBAL | reject |
| CHOICE Initiating NodeType | M |  |  |  | YES | ignore |
| >gNB |  |  |  |  | – |  |
| >>Served Cells NR to Update | O |  | 9.2.1.18 |  | YES | ignore |
| >>Cell Assistance Information NR | O |  | 9.2.2.37 |  | YES | ignore |
| >ng-eNB |  |  |  |  | – |  |
| >>Served Cells E-UTRA to Update | O |  | 9.2.1.19 |  | YES | ignore |

9.1.3.5	NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by a neighbouring NG-RAN node to a peer node to acknowledge update of information for a TNL association.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| CHOICE Responding NodeType | M |  |  |  |  |  |
| >ng-eNB |  |  |  |  |  |  |
| >gNB |  |  |  |  |  |  |
| >>List of Served NR Cells |  | 0 .. < maxCellinNG-RAN node> |  | Complete or limited list of cells served by a gNB, if requested by the en-gNB. |  |  |
| >>>Served NR Cell Information | M |  | 9.2.2.13 |  |  |  |
| >>> Neighbour Information NR | O |  | 9.2.2.61 | NR neighbours. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2.32 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellinNG-RAN node | Maximum no. cells that can be served by an NG-RAN node. Value is FFS. |

#### 9.1.3.6	NG-RAN NODE CONFIGURATION UPDATE FAILURE

This message is sent by the neighbouring NG-RAN node to indicate NG-RAN node Configuration Update failure.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| Cause | M |  | 9.2.2.26 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2.32 |  | YES | ignore |

#### 9.1.3.7	CELL ACTIVATION REQUEST

This message is sent by an NG-RAN node1 to a peer NG-RAN node2 to request a previously switched-off cell/s to be re-activated.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| CHOICE Served Cells To Activate | M |  |  |  | YES | reject |
| >NR Cells |  |  |  |  | – |  |
| >>NR Cells List |  | 1 |  |  | – |  |
| >>>NR Cells item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>>>NR CGI | M |  | 9.2.2.23 |  | – |  |
| >E-UTRA Cells |  |  |  |  | – |  |
| >>E-UTRA Cells List |  | 1 |  |  | – |  |
| >>>E-UTRA Cells item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>>>E-UTRA CGI | M |  | 9.2.2.24 |  | – |  |
| Activation ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |

#### 9.1.3.8	CELL ACTIVATION RESPONSE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that one or more cell(s) previously switched-off has (have) been activated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| CHOICE Activated Served Cells | M |  |  |  | YES | reject |
| >NR Cells |  |  |  |  | – |  |
| >>NR Cells List |  | 1 |  |  | – |  |
| >>>NR Cells Item |  | 1 .. < maxnoffCellsinNG-RANnode> |  |  | – |  |
| >>>>NR CGI | M |  | 9.2.2.23 |  | – |  |
| >E-UTRA Cells |  |  |  |  | – |  |
| >>E-UTRA Cells List |  | 1 |  |  | – |  |
| >>>E-UTRA Cells Item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>>>E-UTRA CGI | M |  | 9.2.2.24 |  | – |  |
| Activation ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.2.32 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |

#### 9.1.3.9	CELL ACTIVATION FAILURE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate cell activation failure.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| Activation ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Cause | M |  | 9.2.2.26 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2.32 |  | YES | ignore |

#### 9.1.3.10	RESET REQUEST

This message is sent from one NG-RAN node to another NG-RAN node and is used to request the Xn interface to be reset.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| CHOICE Reset Type |  |  |  |  | YES | reject |
| >Full Reset |  |  |  |  |  |  |
| >Partial Reset |  |  |  |  |  |  |
| >>UE contexts to be released List |  |  |  |  | YES | reject |
| >>>UE Contexts to be released Item |  | 1 .. <maxnoof UE contexts> |  |  | EACH | reject |
| >>>>NG-RAN node1 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the NG-RAN node1 | – |  |
| >>>>NG-RAN node2 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the NG-RAN node2 | – |  |
| Cause | M |  | 9.2.6 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEContexts | Maximum no. of UE Contexts. Value is 8192. |

#### 9.1.3.11	RESET RESPONSE

This message is sent by an NG-RAN node as a response to a RESET REQUEST message.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| CHOICE Reset Type |  |  |  |  | YES | ignore |
| >Full Reset |  |  |  |  |  |  |
| >Partial Reset |  |  |  |  |  |  |
| >>Admitted UE contexts to be released List |  |  |  |  | YES | ignore |
| >>>Admitted UE Contexts to be released Item |  | 1 .. <maxnoof UE contexts> |  |  | EACH | ignore |
| >>>>NG-RAN node1 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the NG-RAN node1 | – |  |
| >>>>NG-RAN node2 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated at the NG-RAN node2 | – |  |
| Criticality Diagnostics | O |  | 9.2.2.32 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEContexts | Maximum no. of UE Contexts. Value is 8192. |

#### 9.1.3.12	ERROR INDICATION

This message is used to indicate that some error has been detected in the NG-RAN node.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | ignore |
| Old NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated for handover at the source NG-RAN node and for dual connectivity at the S-NG-RAN node or at the NG-RAN node from which a DRB is offloaded. | YES | ignore |
| New NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.2.33 | Allocated for handover at the target NG-RAN node and for dual connectivity at the M-NG-RAN node or the NG-RAN node to which a DRB is offloaded. | YES | ignore |
| Cause | O |  | 9.2.2.26 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2.32 |  | YES | ignore |

#### 9.1.3.13	XN REMOVAL REQUEST

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to initiate the removal of the signaling connection.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| Global NG-RAN Node ID | M |  | 9.2.2.9 |  | YES | reject |
| Xn Removal Threshold | O |  | Xn Benefit Value 9.2.2.78 |  | YES | reject |

#### 9.1.3.14	XN REMOVAL RESPONSE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to acknowledge the initiation of removal of the signaling connection.

Direction: NG-RAN node 2  NG-RAN node 1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| Global NG-RAN Node ID | M |  | 9.2.2.9 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.2.32 |  | YES | ignore |

#### 9.1.3.15	XN REMOVAL FAILURE

This message is sent by the NG-RAN node to indicate that removing the signaling connection cannot be accepted.

Direction: NG-RAN node 2  NG-RAN node 1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.2.21 |  | YES | reject |
| Cause | M |  | 9.2.2.26 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2.32 |  | YES | ignore |

## 9.2	Information Element definitions

### 9.2.0	General

When specifying information elements which are to be represented by bit strings, if not otherwise specifically stated in the semantics description of the concerned IE or elsewhere, the following principle applies with regards to the ordering of bits:

-	The first bit (leftmost bit) contains the most significant bit (MSB);

-	The last bit (rightmost bit) contains the least significant bit (LSB);

-	When importing bit strings from other specifications, the first bit of the bit string contains the first bit of the concerned information.

### 9.2.1	Container and List IE definitions

#### 9.2.1.1	PDU Session Resources To Be Setup List

This IE contains PDU session resource related information used at UE context transfer between NG-RAN nodes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resources To Be Setup List |  | 1 |  |  | YES | reject |
| >PDU Session Resources To Be Setup Item IEs |  | 1 .. <maxnoof PDU sessions > |  |  | EACH | reject |
| >>PDU Session ID | M |  | 9.2.2.35 |  | – |  |
| >>S-NSSAI | M |  | 9.2.2.18 |  | – |  |
| >>PDU Session Resource Aggregate Maximum Bitrate | M |  | OCTET STRING | This IE may need to be refined. | – |  |
| >>UL NG-U UP TNL Information at UPF | M |  | UP Transport Layer Information 9.2.2.28 | UPF endpoint of the NG-U transport bearer. For delivery of UL PDUs | – |  |
| >>Security Indication | O |  | 9.2.2.20 |  | – |  |
| >>PDU Session Type | M |  | 9.2.2.31 |  | – |  |
| >>QoS Flows To Be Setup List |  | 1 |  |  | – |  |
| >>>QoS Flows To Be Setup Item IEs |  | 1 .. <maxnoof QoS Flows> |  |  | EACH | reject |
| >>>>QoS Flow Indicator | M |  | 9.2.2.34 |  | – |  |
| >>>>DL data Forwarding | O |  | 9.2.2.43 |  | – |  |
| >>>>QoS Flow Level QoS Parameters | M |  | 9.2.2.1 |  | – |  |
| >>>>E-RAB ID | O |  | INTEGER (0..15, …) |  | – |  |
| >>Source DRB to QoS Flow Mapping List | O |  | DRB to QoS Flow Mapping List 9.2.1.16 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

#### 9.2.1.2 	PDU Session Resources Admitted List

This IE contains PDU session resource related information to report success of PDU session resource configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resources Admitted List |  | 1 |  |  | – |  |
| >PDU Session Resources Admitted Item IEs |  | 1..<maxnoofPDUSessions> |  |  | EACH | reject |
| >>PDU Session ID | M |  | 9.2.2.35 |  | – |  |
| >>PDU Session Resource Admitted Response Transfer | O |  | 9.2.1.3 | This IE includes the admitted PDU session resource related information. | – |  |
| >>QoS Flows Admitted List |  | 0..1 |  |  | – |  |
| >>>QoS Flows Admitted Item IEs |  | 1..<maxnoofQoSFlows> |  |  | EACH | reject |
| >>>>QoS Flow Indicator | M |  | 9.2.2.34 |  |  |  |
| >>>>Data Forwarding Accepted | O |  | 9.2.2.44 |  | – |  |
| >>QoS Flows not Admitted List |  | 0..1 |  |  | – |  |
| >>>QoS Flows not Admitted Item IEs | O | 1..<maxnoofQoSFlows> |  |  | EACH | Reject |
| >>>>QoS Flow Indicator | M |  | 9.2.2.34 |  | – |  |
| >>>>Cause | M |  | 9.2.2.26 |  | – |  |
| >>Data Forwarding Info from target NG-RAN node | O |  | 9.2.1.17 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

9.2.1.4	PDU Session Resources Not Admitted List

This IE contains a list of PDU session resources which were not admitted to be added or modified.

| This IE contains a list of PDU session resources which were not admitted to be added or modified.IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resources Not Admitted List |  | 1 |  |  | – |  |
| >PDU Session Resources Not Admitted Item IEs |  | 1..<maxnoofPDUSessions> |  |  | EACH | reject |
| >>PDU Session ID | M |  | 9.2.2.35 |  | – |  |
| >>Cause | O |  | 9.2.2.26 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |

#### 9.2.1.5	QoS Flow List

This IE contains a list of QoS flows with a cause value.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| QoS Flow Item IEs |  | 1..<maxnoofQoSFlows> |  |  | EACH | reject |
| >QoS Flow Indicator | M |  | 9.2.2.34 |  | – |  |
| >Cause | O |  | 9.2.2.26 | [FFS] | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

#### 9.2.1.6	PDU Session Resource Setup Info – SN terminated

Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018, the content of this section is FFS.

This IE contains information for the addition of S-NG-RAN node resources related to a PDU session for DRBs configured with an SN terminated bearer option.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UL NG-U UP TNL Information at UPF | M |  | UP Transport Layer Information 9.2.2.28 | UPF endpoint of the NG-U transport bearer. For delivery of UL PDUs | – |  |
| PDU Session Type | M |  | 9.2.2.31 |  | – |  |
| QoS Flows To Be Setup List |  | 1 |  |  | – |  |
| >QoS Flows To Be Setup Item IEs |  | 1 .. <maxnoofQoSFlows> |  |  | EACH | reject |
| >>QoS Flow Indicator | M |  | 9.2.2.34 |  | – |  |
| >>QoS Flow Level QoS Parameters | M |  | 9.2.2.1 | For GBR QoS flows, this IE contains GBR QoS flow information as received at NG-C | – |  |
| >>Offered GBR QoS Flow Information | O |  | GBR QoS Flow Information 9.2.2.2 | This IE contains M-Node offered GBR QoS Flow Information. | – |  |
| DL Forwarding | O |  | 9.2.2.43 | This IE may need to be refined. Placeholder IE only. | – |  |
| Security Indication | O |  | 9.2.2.20 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows. Value is 64 |

#### 9.2.1.7	PDU Session Resource Setup Response Info – SN terminated

Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018, the content of this section is FFS.

This IE contains the result of the addition of S-NG-RAN node resources related to a PDU session for DRBs configured with an SN terminated bearer option.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NG-U DL UP TNL Information at NG-RAN | M |  | UP Transport Layer Information 9.2.2.28 | S-NG-RAN node endpoint of the NG transport bearer. For delivery of DL PDUs. | – |  |
| DRBs To Be Setup List |  | 1 |  |  | – |  |
| >DRBS to Be Setup Item IEs |  | 1 .. <maxnoof DRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.2.2.30 |  | – |  |
| >>SN UL PDCP UP TNL Information | M |  | UP Transport Layer Information 9.2.2.28 | S-NG-RAN node endpoint of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs. | – |  |
| >>QoS Flows To Be Setup List |  | 1 |  | Editor’s Note: It is FFS whether the corresponding node decides the “overall” QoS based on information of QoS flows mapped to it and whether mapping info is needed at all. | – |  |
| >>>QoS Flows To Be Setup Item IEs |  | 1 .. <maxnoof QoS Flows> |  |  | EACH | reject |
| >>>>QoS Flow Indicator | M |  | 9.2.2.34 |  | – |  |
| >>>>MCG requested GBR QoS Flow Information | O |  | GBR QoS Flow Information 9.2.2.2 | This IE contains GBR QoS Flow Information necessary for the MCG part. | – |  |
| QoS Flows Not Admitted List | O |  | QoS Flow List 9.2.1.5 |  | – |  |
| DL Forwarding UP TNL Information | O |  | UP Transport Layer Information 9.2.2.28 | Identifies the Xn transport bearer used for forwarding of DL PDUs This IE may need to be refined. Placeholder only. | – |  |
| UL Forwarding UP TNL Information | O |  | UP Transport Layer Information 9.2.2.28 | Identifies the Xn transport bearer used for forwarding of UL PDUs This IE may need to be refined. Placeholder only. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs allowed towards one UE. Value is 32. |
| maxnoofQoSFlows | Maximum no. of QoS flows. Value is 64 |

#### 9.2.1.8	PDU Session Resource Setup Info – MN terminated

Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018, the content of this section is FFS.

This IE contains information for the addition of S-NG-RAN node resources related to a PDU session for DRBs configured with an MN terminated bearer option.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Type | M |  | 9.2.2.31 |  | – |  |
| DRBs To Be Setup List |  | 1 |  |  | – |  |
| >DRBS to Be Setup Item IEs |  | 1 .. <maxnoof DRBs> |  |  | EACH | reject |
| >>MN UL PDCP UP TNL Information | M |  | UP Transport Layer Information 9.2.2.28 | M-NG-RAN node endpoint of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs. | – |  |
| >>RLC Mode |  |  | 9.2.2.12 |  | – |  |
| >>QoS Flows To Be Setup List |  | 1 |  | Editor’s Note: It is FFS whether the corresponding node decides the “overall” QoS based on information of QoS flows mapped to it and whether mapping info is needed at all. | – |  |
| >>>QoS Flows To Be Setup Item IEs |  | 1 .. <maxnoof QoS Flows> |  |  | EACH | reject |
| >>>>QoS Flow Indicator | M |  | 9.2.2.34 |  | – |  |
| >>>>QoS Flow Level QoS Parameters | M |  | 9.2.2.1 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs allowed towards one UE. Value is 32. |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

#### 9.2.1.9	PDU Session Resource Setup Response Info – MN terminated

Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018, the content of this section is FFS.

This IE contains the result of the addition of S-NG-RAN node resources related to a PDU session for DRBs configured with an MN terminated bearer option.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DRBs Admitted List |  | 1 |  |  | – |  |
| >DRBS Admitted Item IEs |  | 1 .. <maxnoof DRBs> |  |  | EACH | reject |
| >>S-Node DL SCG UP TNL Information | M |  | UP Transport Layer Information 9.2.2.28 | S-NG-RAN node GTP-U tunnel endpoint of the DRB’s Xn transport at its Lower Layer SCG resource. For delivery of DL PDUs. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs allowed towards one UE. Value is 32. |

#### 9.2.1.10	PDU Session Resource Modification Info – SN terminated

Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018, the content of this section is FFS.

This IE contains information related to a PDU session resource for an M-NG-RAN node initiated request to modify DRBs configured with an SN terminated bearer option.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UL NG-U UP TNL Information at UPF | O |  | UP Transport Layer Information 9.2.2.28 | UPF endpoint of the NG-U transport bearer. For delivery of UL PDUs | – |  |
| QoS Flows To Be Setup List |  | 0..1 |  |  | – |  |
| >QoS Flows To Be Setup Item IEs |  | 1 .. <maxnoofQoSFlows> |  |  | EACH | reject |
| >>QoS Flow Indicator | M |  | 9.2.2.34 |  | – |  |
| >>QoS Flow Level QoS Parameters | M |  | 9.2.2.1 | For GBR QoS flows, this IE contains GBR QoS flow information as received at NG-C | – |  |
| >>Offered GBR QoS Flow Information | O |  | GBR QoS Flow Information 9.2.2.2 | This IE contains M-Node offered GBR QoS Flow Information. | – |  |
| DL Forwarding | O |  | 9.2.2.43 | This IE may need to be refined. Placeholder only. | – |  |
| QoS Flows To Be Modified List |  | 0..1 |  |  | – |  |
| >QoS Flows To Be Modified Item IEs |  | 1 .. <maxnoofQoSFlows> |  |  | EACH | reject |
| >>QoS Flow Indicator | M |  | 9.2.2.34 |  | – |  |
| >>QoS Flow Level QoS Parameters | O |  | 9.2.2.1 | For GBR QoS flows, this IE contains GBR QoS flow information as received at NG-C | – |  |
| >>Offered GBR QoS Flow Information | O |  | GBR QoS Flow Information 9.2.2.2 | This IE contains M-Node offered GBR QoS Flow Information. | – |  |
| QoS Flows To Be Released List |  | 0..1 | QoS Flow List 9.2.1.5 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows. Value is 64. |

#### 9.2.1.11	PDU Session Resource Modification Response Info – SN terminated

Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018, the content of this section is FFS.

This IE contains the PDU session resource related result of an M-NG-RAN node initiated request to modify DRBs configured with an SN terminated bearer option.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Sessions Level QoS Parameters |  |  | OCTET STRING | Includes QoS parameters to be modified. This IE may need to be refined. | – |  |
| NG-U DL UP TNL Information at NG-RAN | O |  | UP Transport Layer Information 9.2.2.28 | S-NG-RAN node endpoint of the NG transport bearer. For delivery of DL PDUs. | – |  |
| DRBs To Be Setup List |  | 0..1 |  |  | – |  |
| >DRBS to Be Setup Item IEs |  | 1 .. <maxnoof DRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.2.2.30 |  | – |  |
| >>SN UL PDCP UP TNL Information | M |  | UP Transport Layer Information 9.2.2.28 | S-NG-RAN node endpoint of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs. | – |  |
| >>QoS Flows To Be Setup List |  | 1 |  | Editor’s Note: It is FFS whether the corresponding node decides the “overall” QoS based on information of QoS flows mapped to it and whether mapping info is needed at all. | – |  |
| >>>QoS Flows To Be Setup Item IEs |  | 1 .. <maxnoof QoS Flows> |  |  | EACH | reject |
| >>>>QoS Flow Indicator | M |  | 9.2.2.34 |  | – |  |
| >>>>MCG requested GBR QoS Flow Information | O |  | GBR QoS Flow Information 9.2.2.2 | This IE contains GBR QoS Flow Information necessary for the MCG part. | – |  |
| DRBs To Be Modified List |  | 0..1 |  |  | – |  |
| >DRBS to Be Modified Item IEs |  | 1 .. <maxnoof DRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.2.2.30 |  | – |  |
| >>SN UL PDCP UP TNL Information | O |  | UP Transport Layer Information 9.2.2.28 | S-NG-RAN node endpoint of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs. | – |  |
| >>QoS Flows List |  | 1 |  | Overwriting the existing QoS Flow List | – |  |
| >>>QoS Flows Item IEs |  | 1 .. <maxnoof QoS Flows> |  | Editor’s Note: It is FFS whether the corresponding node decides the “overall” QoS based on information of QoS flows mapped to it and whether mapping info is needed at all. | EACH | reject |
| >>>>QoS Flow Indicator | M |  | 9.2.2.34 |  | – |  |
| >>>>MCG requested GBR QoS Flow Information | O |  | GBR QoS Flow Information 9.2.2.2 | This IE contains GBR QoS Flow Information necessary for the MCG part. | – |  |
| DRBs To Be Released List |  | 0..1 |  |  | – |  |
| >DRBS to Be Released Item IEs |  | 1 .. <maxnoof DRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.2.2.30 |  | – |  |
| >>Cause | O |  | 9.2.2.26 |  | – |  |
| QoS Flows Not Admitted to be Added List | O |  | QoS Flow List 9.2.1.5 |  | – |  |
| QoS Flows Admitted to be Modified List | O |  | QoS Flow List 9.2.1.5 |  | – |  |
| QoS Flows Not Admitted to be Modified List | O |  | QoS Flow List 9.2.1.5 |  | – |  |
| QoS Flows Admitted to be Released List | O |  | QoS Flow List 9.2.1.5 |  | – |  |
| DL Forwarding UP TNL Information | O |  | UP Transport Layer Information 9.2.2.28 | Identifies the Xn transport bearer used for forwarding of DL PDUs. This IE may need to be refined. Placeholder only. | – |  |
| UL Forwarding UP TNL Information | O |  | UP Transport Layer Information 9.2.2.28 | Identifies the Xn transport bearer used for forwarding of UL PDUs. This IE may need to be refined. Placeholder only. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs allowed towards one UE. Value is 32. |
| maxnoofQoSFlows | Maximum no. of QoS flows. Value is 64. |

#### 9.2.1.12	PDU Session Resource Modification Info – MN terminated

Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018, the content of this section is FFS.

This IE contains information related to PDU session resource for an M-NG-RAN node initiated request to modify DRBs configured with an MN terminated bearer option.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Type | M |  | 9.2.2.31 |  | – |  |
| DRBs To Be Setup List |  | 1 |  |  | – |  |
| >DRBS to Be Setup Item IEs |  | 1 .. <maxnoof DRBs> |  |  | EACH | reject |
| >>MN UL PDCP UP TNL Information | M |  | UP Transport Layer Information 9.2.2.28 | M-NG-RAN node endpoint of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs. | – |  |
| >>RLC Mode |  |  | 9.2.2.12 |  | – |  |
| >>QoS Flows To Be Setup List |  | 1 |  | This IE may need to be refined, especially, whether the corresponding node decides the “overall” QoS based on information of QoS flows mapped to it and whether mapping info is needed at all. | – |  |
| >>>QoS Flows To Be Setup Item IEs |  | 1 .. <maxnoof QoS Flows> |  |  | EACH | reject |
| >>>>QoS Flow Indicator | M |  | 9.2.2.34 |  | – |  |
| >>>>QoS Flow Level QoS Parameters | M |  | 9.2.2.1 |  | – |  |
| DRBs To Be Modified List |  | 0..1 |  |  | – |  |
| >DRBS to Be Modified Item IEs |  | 1 .. <maxnoof DRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.2.2.30 |  | – |  |
| >>SN UL PDCP UP TNL Information | O |  | UP Transport Layer Information 9.2.2.28 | S-NG-RAN node endpoint of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs. | – |  |
| >>QoS Flows List |  | 1 |  | Overwriting the existing QoS Flow List | – |  |
| >>>QoS Flows Item IEs |  | 1 .. <maxnoof QoS Flows> |  | This IE may need to be refined, especially, whether the corresponding node decides the “overall” QoS based on information of QoS flows mapped to it and whether mapping info is needed at all. | EACH | reject |
| >>>>QoS Flow Indicator | M |  | 9.2.2.34 |  | – |  |
| >>>>MCG requested GBR QoS Flow Information | O |  | GBR QoS Flow Information 9.2.2.2 | This IE contains GBR QoS Flow Information necessary for the MCG part. | – |  |
| DRBs To Be Released List | O |  | OCTET STRING | This IE may need to be refined. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs allowed towards one UE. Value is 32. |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

#### 9.2.1.13	PDU Session Resource Modification Response Info – MN terminated

Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018, the content of this section is FFS.

This IE contains the PDU session resource related result of an M-NG-RAN node initiated modification of DRBs configured with an MN terminated bearer option.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DRBs Admitted to be Setup or Modified List |  | 1 |  |  | – |  |
| >DRBS Admitted to be Setup or Modified Item IEs |  | 1 .. <maxnoof DRBs> |  |  | EACH | reject |
| >>S-Node DL SCG UP TNL Information | O |  | UP Transport Layer Information 9.2.2.28 | S-NG-RAN node GTP-U tunnel endpoint of the DRB’s Xn transport at its Lower Layer SCG resource. For delivery of DL PDUs. | – |  |
| DRBs Admitted To Be Released List | O |  | OCTET STRING | This IE may need to be refined. | – |  |
| DRBs Not Admitted To Be Setup or Modified List | O |  | OCTET STRING | This IE may need to be refined. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs allowed towards one UE. Value is 32. |

#### 9.2.1.14	UE Context Information

This IE contains the UE context information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.2.67 | Allocated at the AMF on the old NG-C connection. |
| Signalling TNL Association Address at source NG-C side | M |  | CP Transport Layer Information 9.2.2.29 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. |
| UE Security Capabilities | M |  | 9.2.2.64 |  |
| Security Information | M |  | 9.2.2.65 |  |
| UE Aggregate Maximum Bit Rate | M |  | 9.2.2.36 | [FFS] |
| PDU Session Resources To Be Setup List | M |  | 9.2.1.1 |  |
| RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331[10], if the old and new serving NG-RAN nodes are gNBs, or the HandoverPreparationInformation message as defined in subclause 10.2.2 of TS 36.331 [14], if the old and new serving NG-RAN nodes are ng-eNBs. |
| Mobility Restriction List | O |  | 9.2.2.68 |  |
| Index to RAT/Frequency Selection Priority | O |  | 9.2.2.38 |  |

#### 9.2.1.15	DRBs Subject To Status Transfer List

This IE contains a list of DRBs containing information about PDCP PDU transfer status.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DRBs Subject To Status Transfer Item |  | 1 .. <maxnoof DRBs> |  |  | EACH | ignore |
| >DRB ID | M |  | 9.2.2.30 |  | – |  |
| >CHOICE PDCP SN | M |  |  |  | – |  |
| >>12 bits |  |  |  |  |  |  |
| >>>Receive Status Of PDCP SDU | O |  | BIT STRING (1.. 2048) | The IE is used in case of 12-bit long PDCP-SN. The first bit indicates the status of the SDU after the First Missing UL PDCP SDU. The Nth bit indicates the status of the UL PDCP SDU in position (N + First Missing SDU Number) modulo (1 + the maximum value of the PDCP-SN). 0: PDCP SDU has not been received. 1: PDCP SDU has been received correctly. | – |  |
| >>>UL COUNT Value | M |  | COUNT Value for PDCP SN Length 12 9.2.2.69 | PDCP-SN and Hyper Frame Number of the first missing UL SDU in case of 12-bit long PDCP-SN | – |  |
| >>>DL COUNT Value | M |  | COUNT Value for PDCP SN Length 12 9.2.2.69 | PDCP-SN and Hyper Frame Number that the target NG-RAN node should assign for the next DL SDU not having an SN yet in case of 12-bit long PDCP-SN | – |  |
| >>18 bits |  |  |  |  |  |  |
| >>>Receive Status Of PDCP SDU | O |  | BIT STRING (1.. 131072) | The IE is used in case of 18-bit long PDCP-SN. The first bit indicates the status of the SDU after the First Missing UL PDCP SDU. The Nth bit indicates the status of the UL PDCP SDU in position (N + First Missing SDU Number) modulo (1 + the maximum value of the PDCP-SN). 0: PDCP SDU has not been received. 1: PDCP SDU has been received correctly. | – |  |
| >>>UL COUNT Value | M |  | COUNT Value for PDCP SN Length 18 9.2.2.70 | PDCP-SN and Hyper Frame Number of the first missing UL SDU in case of 18-bit long PDCP-SN | – |  |
| >>>DL COUNT Value | M |  | COUNT Value for PDCP SN Length 18 9.2.2.70 | PDCP-SN and Hyper Frame Number that the target NG-RAN node should assign for the next DL SDU not having an SN yet in case of 18-bit long PDCP-SN | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs allowed towards one UE. Value is 32. |

#### 9.2.1.16	DRB to QoS Flow Mapping List

This IE contains a list of DRBs containing information about the mapped QoS flows.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DRBS Item IEs |  | 1 .. <maxnoof DRBs> |  |  | EACH | reject |
| >DRB ID | M |  | 9.2.2.30 |  | – |  |
| >QoS Flows List |  | 1 |  |  | – |  |
| >>QoS Flow Item IEs |  | 1..<maxnoofQoSFlows> |  |  | EACH | reject |
| >>>QoS Flow Indicator | M |  | 9.2.2.34 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs allowed towards one UE. Value is 32. |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

#### 9.2.1.17	Data Forwarding Info from target NG-RAN node

This IE contains TNL information for the establishment of data forwarding tunnels towards the target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session level DL data forwarding UP TNL Information | O |  | UP Transport Layer Information 9.2.2.28 | To forward NG-U DL data to the target node for which no PDCP SN has been assigned yet. | – |  |
| Data Forwarding Response DRB List |  | 0..1 |  |  | – |  |
| >Data Forwarding Response DRB Item IEs |  | 1..<maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.2.2.30 |  | – |  |
| >>DL Forwarding UP TNL Information | O |  | UP Transport Layer Information 9.2.2.28 |  | – |  |
| >>UL Forwarding UP TNL Information | O |  | UP Transport Layer Information 9.2.2.28 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs. Value is 32. |

#### 9.2.1.18	Served Cells NR To Update

This IE contains updated configuration information for served NR cells exchanged between NG-RAN nodes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Served Cells NR To Add |  | 0 .. < maxnoofCellsinNG-RAN node> |  |  | GLOBAL | reject |
| >Served Cell Information NR | M |  | 9.2.2.13 |  | – |  |
| > Neighbour Information NR | O |  | 9.2.2.61 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.62 |  | – |  |
| Served Cells NR To Modify |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | GLOBAL | reject |
| >Old NR CGI | M |  | NR CGI 9.2.2.23 |  | – |  |
| >Served Cell Information NR | M |  | 9.2.2.13 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.61 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.62 |  | – |  |
| Served Cells NR To Delete |  | 0 .. < maxnooffCellsinNG-RAN node > |  |  | GLOBAL | reject |
| >Old NR-CGI | M |  | NR CGI 9.2.2.23 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

#### 9.2.1.19	Served Cells E-UTRA to Update

This IE contains updated configuration information for served E-UTRA cells exchanged between NG-RAN nodes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Served Cells E-UTRA To Add |  | 0 .. < maxnoofCellsinNG-RAN node> |  |  | GLOBAL | reject |
| >Served Cell Information E-UTRA | M |  | 9.2.2.56 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.61 |  | YES | ignore |
| >Neighbour Information E-UTRA | O |  | 9.2.2.62 |  | YES | ignore |
| Served Cells E-UTRA To Modify |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | GLOBAL | reject |
| >Old ECGI | M |  | E-UTRA CGI 9.2.2.24 |  | – |  |
| >Served Cell Information E-UTRA | M |  | 9.2.2.56 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.61 |  | YES | ignore |
| >Neighbour Information E-UTRA | O |  | 9.2.2.62 |  | YES | ignore |
| Served Cells E-UTRA To Delete |  | 0 .. < maxnoofCellsinNG-RAN node > |  |  | GLOBAL | reject |
| >Old ECGI | M |  | E-UTRA CGI 9.2.2.24 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

### 9.2.2	General IE definitions

#### 9.2.2.1	QoS Flow Level QoS Parameters

This IE defines the QoS Parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  |
| >Non Dynamic 5QI |  |  |  |  |
| >>Non dynamic 5QI Desciptor | M |  | 9.2.2.8 |  |
| >Dynamic 5QI |  |  |  |  |
| >>Dynamic 5QI Desciptor | M |  | 9.2.2.7 |  |
| Allocation and Retention Priority | M |  | 9.2.2.6 | Note: presence needs to be checked with latest SA2 status. |
| GBR QoS Flow Information | O |  | 9.2.2.2 | This IE shall be present for GBR QoS Flows only. |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, ...) | Reflective QoS is specified in TS 23.501 [7]. This IE applies to non-GBR bearers only and shall be ignored otherwise. |
| Additional QoS flow Information | O |  | ENUMERATED (more likely, …) | If this IE is set to "more likely", this indicates that traffic for this QoS flow is likely to appear more often than traffic for other flows established for the PDU session. This IE may be present in case of non-GBR flows only and shall be ignored otherwise. |
| PPI | O |  | INTEGER (1..8, …) | Paging Policy Indicator used in PPD feature (Paging Policy Differentiation) as specified in TS 23.501 [7]. This IE applies to PDU sessions of IP type. |

#### 9.2.2.2	GBR QoS Flow Information

This IE indicates QoS Parameters for a GBR QoS Flow for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Maximum Flow Bit Rate Downlink | M |  | Bit Rate 9.2.2.3 | Maximum Bit Rate in DL. Flow Bit Rates are specified in TS 23.501 [7]. |
| Maximum Flow Bit Rate Uplink | M |  | Bit Rate 9.2.2.3 | Maximum Bit Rate in UL. Flow Bit Rates are specified in TS 23.501 [7]. |
| Guaranteed Flow Bit Rate Downlink | M |  | Bit Rate 9.2.2.3 | Guaranteed Bit Rate (provided that there is data to deliver) in DL. Flow Bit Rates are specified in TS 23.501 [7]. |
| Guaranteed Flow Bit Rate Uplink | M |  | Bit Rate 9.2.2.3 | Guaranteed Bit Rate (provided that there is data to deliver). Flow Bit Rates are specified in TS 23.501 [7]. |
| Notification Control | O |  | ENUMERATED (notification requested, ...) | Notification control is specified in TS 23.501 [7] |
| Maximum Packet Loss Rate Downlink | O |  | Packet Loss Rate 9.2.2.51 | Indicates the maximum rate for lost packets that can be tolerated in the downlink direction. Maximum Packet Loss Rate is specified in TS 23.501 [7]. |
| Maximum Packet Loss Rate Uplink | O |  | Packet Loss Rate 9.2.2.51 | Indicates the maximum rate for lost packets that can be tolerated in the uplink direction. Maximum Packet Loss Rate is specified in TS 23.501 [7]. |
| Additional QoS flow Information | O |  | ENUMERATED (more likely, …) | If this IE is set to "more likely", this indicates that traffic for this QoS flow is likely to appear more often than traffic for other flows established for the PDU session. This IE may be present in case of non-GBR flows only and shall be ignored otherwise. |

#### 9.2.2.3	Bit Rate

This IE indicates the number of bits delivered by NG RAN in UL or to NG RAN in DL within a period of time, divided by the duration of the period. It is used, for example, to indicate the maximum or guaranteed bit rate for a GBR QoS flow, or an aggregate maximum bit rate.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Bit Rate | M |  | INTEGER (0..4,000,000,000,000,…) | The unit is: bit/s |

#### 9.2.2.4	TAI Support List

This IE indicates the list of TAIs supported by NG-RAN node and associated characteristics e.g. supported slices.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TAI Support Item IEs |  | 1..<maxnoofsupportedTACs> |  |  | EACH | reject |
| >TAC | M |  | 9.2.2.17 | Broadcast TAC | – |  |
| >Broadcast PLMNs |  | 1..<maxnoofsupportedPLMNs> |  |  | – |  |
| >>PLMN Identity | M |  | 9.2.2.25 | Broadcast PLMN | – |  |
| >>TAI Slice Support List | O |  | Slice Support List 9.2.2.5 | Supported S-NSSAIs per TA | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofsupportedTACs | Maximum no. of TACs supported by an NG-RAN node. Value is FFS. |
| maxnoofsupportedPLMNs | Maximum no. of PLMNs supported by an NG-RAN node. Value is FFS. |

#### 9.2.2.5	Slice Support List

This IE indicates the list of supported slices.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice Support Item IEs |  | 1..<maxnoofSliceItems> |  |  |
| >S-NSSAI | M |  | 9.2.2.18 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

9.2.2.6	Allocation and Retention Priority

This IE specifies the relative importance compared to other QoS flows for allocation and retention of the NR RAN resource.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Allocation/Retention Priority |  | 1 |  |  |
| >Priority Level | M |  | INTEGER (0..15) | Desc.: This defines the relative importance of a resource request. (see TS 23.501 [7]). Usage: Values between 1 and 15 are ordered in decreasing order of priority, i.e., 1 is the highest and 15 is the lowest. |
| >Pre-emption Capability | M |  | ENUMERATED (shall not trigger pre-emption, may trigger pre-emption) | Desc.: This IE indicates the pre-emption capability of the request on other QoS flows Usage: The QoS flow shall not pre-empt other QoS flow or, the QoS flow may pre-empt other QoS flows The Pre-emption Capability indicator applies to the allocation of resources for a QoS flow and as such it provides the trigger to the pre-emption procedures/processes of the gNB. |
| >Pre-emption Vulnerability | M |  | ENUMERATED (not pre-emptable, pre-emptable) | Desc.: This IE indicates the vulnerability of the QoS flow to preemption of other QoS flows. Usage: The QoS flow shall not be pre-empted by other QoS flows or the QoS flow may be pre-empted by other QoS flows. Pre-emption Vulnerability indicator applies for the entire duration of the QoS flow, unless modified and as such indicates whether the QoS flow is a target of the pre-emption procedures/processes of the gNB. |

#### 9.2.2.7	Dynamic 5QI Descriptor

This IE defines the QoS characteristics for a non-standardized or not pre-configured 5QI for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Priority level | M |  | INTEGER (1..127) | Priority level is specified in TS 23.501 [7]. |
| Packet Delay Budget | M |  | 9.2.2.52 | Packet Delay Budget is specified in TS 23.501 [7]. |
| Packet Error Rate | M |  | 9.2.2.53 | Packet Error Rate is specified in TS 23.501 [7]. |
| Delay Critical | C-ifGBRflow |  | ENUMERATED (Delay critical, Non-delay critical) | This IE indicates whether the GBR Qos flow is delay critical as specified in TS 23.501 [7]. |
| Averaging window | C-ifGBRflow |  | 9.2.2.54 | Averaging window is specified in TS 23.501 [7]. |
| Maximum Data Burst Volume | O |  | 9.2.2.55 | Maximum Data Burst Volume is specified in TS 23.501 [7]. |

| Condition | Explanation |
| --- | --- |
| ifGBRflow | This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE. |

#### 9.2.2.8	Non dynamic 5QI Descriptor

This IE defines QoS characteristics for a standardized or pre-configured 5QI for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| 5QI | M |  | INTEGER (0.255) | The 5QI is specified in TS 23.501 [7} |
| Priority level | O |  | INTEGER (1..127) | Priority level is specified in TS 23.501 [7]. When included, it overrides standardized or pre-configured value. |
| Averaging window | O |  | 9.2.2.54 | This IE applies to GBR QoS Flows only. Averaging window is specified in TS 23.501 [7]. When included, it overrides standardized or pre-configured value. |
| Maximum Data Burst Volume | O |  | 9.2.2.55 | Maximum Data Burst Volume is specified in TS 23.501 [7]. When included, it overrides standardized or pre-configured value. |

#### 9.2.2.9	Global NG-RAN Node ID

This IE is used to globally identify an NG-RAN node (see TS 38.300 [9]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE NG-RAN node | M |  |  |  |
| >gNB |  |  |  |  |
| >>Global gNB ID | M |  | 9.2.2.22 |  |
| >ng-eNB |  |  |  |  |
| >>Global ng-eNB ID | M |  | 9.2.2.10 |  |

#### 9.2.2.10	Global ng-eNB ID

This IE is used to globally identify an ng-eNB (see TS 38.300 [9]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.2.25 |  |
| CHOICE ng-eNB ID | M |  |  |  |
| >Macro ng-eNB ID |  |  |  |  |
| >>Macro ng-eNB ID | M |  | BIT STRING (SIZE(20)) | Equal to the 20 leftmost bits of the E-UTRA Cell Identity IE contained in the E-UTRA CGI IE of each cell served by the ng-eNB. |
| >Short Macro ng-eNB ID |  |  |  |  |
| >>Short Macro ng-eNB ID | M |  | BIT STRING (SIZE(18)) | Equal to the 18 leftmost bits of the E-UTRA Cell Identity IE contained in the E-UTRA CGI IE of each cell served by the ng-eNB. |
| >Long Macro ng-eNB ID |  |  |  |  |
| >>Long Macro ng-eNB ID | M |  | BIT STRING (SIZE(21)) | Equal to the 21 leftmost bits of the E-UTRA Cell Identity IE contained in the E-UTRA CGI IE of each cell served by the ng-eNB. |

#### 9.2.2.11	SCG Configuration Query

The SCG Configuration Query IE is used to request the S-NG-RAN node to provide current SCG configuration.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SCG Configuration Query | M |  | ENUMERATED (True, …) |  |

#### 9.2.2.12	RLC Mode

The RLC Mode IE indicates the RLC Mode used for a DRB.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| RLC Mode | M |  | ENUMERATED ( RLC-AM, RLC-UM) |  |

#### 9.2.2.13	Served Cell Information NR

This IE contains cell configuration information of an NR cell that a neighbouring NG-RAN node may need for the Xn AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR-PCI | M |  | INTEGER (0..1007, …) | NR Physical Cell ID |
| NR CGI | M |  | 9.2.2.23 |  |
| TAC | M |  | 9.2.2.17 | Tracking Area Code |
| RANAC | O |  | 9.2.2.73 | RAN Area Code |
| Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs |
| >PLMN Identity | M |  | 9.2.2.25 |  |
| CHOICE NR-Mode-Info | M |  |  |  |
| >FDD |  |  |  |  |
| >>FDD Info |  | 1 |  |  |
| >>>UL NR Frequency Info | M |  | NR Frequency Info 9.2.2.15 |  |
| >>>DL NR Frequency Info | M |  | NR Frequency Info 9.2.2.15 |  |
| >>>UL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.16 |  |
| >>>DL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.16 |  |
| >TDD |  |  |  |  |
| >>TDD Info |  | 1 |  |  |
| >>>Frequency Info | M |  | NR Frequency Info 9.2.2.15 |  |
| >>>Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.16 |  |
| SUL Information |  |  |  |  |
| Measurement Timing Configuration | M |  | OCTET STRING | Contains the MeasurementTimingConfiguration inter-node message for the served cell, as defined in TS 38.331 [10]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is FFS. |

#### 9.2.2.14	SUL Information

This IE contains information about the SUL carrier.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SUL Frequency Info | M |  | NR Frequency Info 9.2.2.15 INTEGER (0..maxNRARFCN) | RF Reference Frequency as defined in TS 38.104 [24] section 5.4.2.1. The frequency provided in this IE identifies the absolute frequency position of the reference resource block (Common RB 0) of the SUL carrier. Its lowest subcarrier is also known as Point A. |
| SUL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.16 |  |

| Range bound | Explanation |
| --- | --- |
| maxNRARFCN | Maximum value of NRARFCNs. Value is 3279165. |

#### 9.2.2.15	NR Frequency Info

The NR Frequency Info defines the carrier frequency and bands used in a cell for a given direction (UL or DL) in FDD or for both UL and DL directions in TDD or for SUL carrier.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NR ARFCN | M |  | INTEGER (0.. maxNRARFCN) | RF Reference Frequency as defined in TS38.104 [24], section 5.4.2.1. The frequency provided in this IE identifies the absolute frequency position of the reference resource block (Common RB 0) of the carrier. Its lowest subcarrier is also known as Point A. |
| SUL Information | O |  | 9.2.2.14 |  |
| NR Frequency Band List |  | 1 |  |  |
| >NR Frequency Band Item |  | 1..<maxnoofNrCellBands> |  |  |
| >>NR Frequency Band | M |  | INTEGER (1.. 1024, ...) | Primary NR Operating Band as defined in TS38.104 [24], section 5.4.2.3. The value 1 corresponds e n1, value 2 corresponds to NR operating band n2, etc. |
| >>Supported SUL band List |  | 0..<maxnoofNrCellBands> |  |  |
| >>>Supported SUL band Item | M |  | INTEGER (1.. 1024, ...) | Supplementary NR Operating Band as defined in TS 38.104 [24] section 5.4.2.3 that can be used for SUL duplex mode as per TS 38.101-1 table 5.2.-1.. The value 80 corresponds to NR operating band n80, value 81 corresponds to NR operating band n81, etc. |

| Range bound | Explanation |
| --- | --- |
| maxNRARFCN | Maximum value of NRARFCNs. Value is 3279165. |
| maxnoofNrCellBands | Maximum no. of frequency bands supported for a NR cell. Value is 32. |

#### 9.2.2.16	NR Transmission Bandwidth

The NR Transmission Bandwidth IE is used to indicate either the UL or the DL transmission bandwidth.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NR Transmission Bandwidth | M |  | INTEGER (0.. 65535) | This IE may need to be refined. |

#### 9.2.2.17	TAC

This information element is used to uniquely identify a Tracking Area within a PLMN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TAC | M |  | OCTET STRING (SIZE (3)) |  |

#### 9.2.2.18	S-NSSAI

This IE indicates the S-NSSAI.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| >SST | M |  | OCTET STRING (SIZE(1)) |  |
| >SD | O |  | OCTET STRING (SIZE(3)) |  |

#### 9.2.2.19	Delivery Status

This IE provides the delivery status of RRC PDUs provided by RRC Transfer message.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Delivery Status | M |  | INTEGER (0..212-1) | Highest successfully delivered NR PDCP SN, as defined in 38.323 [11]. |

#### 9.2.2.20	Security Indication

This IE indicates whether the UP integrity is configured for corresponding PDU sessions, respectively.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Integrity Protection Indication | M |  | ENUMERATED (required, preferred, not needed,…) | Indicates whether UP integrity protection shall apply, should apply, or shall not apply for the concerned PDU session. |
| Confidentiality Protection Indication | M |  | ENUMERATED (required, preferred, not needed, …) | Indicates whether UP ciphering shall apply, should apply, or shall not apply for the concerned PDU session. |

#### 9.2.2.21	Message Type

The Message Type IE uniquely identifies the message being sent. It is mandatory for all messages.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Procedure Code | M |  | INTEGER (0..255) |  |
| Type of Message | M |  | CHOICE (Initiating Message, Successful Outcome, Unsuccessful Outcome, …) |  |

#### 9.2.2.22	Global gNB ID

This IE is used to globally identify a gNB (see TS 38.300 [9]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.2.25 |  |
| CHOICE gNB ID | M |  |  |  |
| >gNB ID |  |  |  |  |
| >>gNB ID | M |  | BIT STRING (SIZE(22..32)) | Equal to the leftmost bits of the NR Cell Identity IE contained in the NR CGI IE of each cell served by the gNB. |

#### 9.2.2.23	NR CGI

This IE is used to globally identify an NR cell (see TS 38.300 [9]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.2.25 |  |
| NR Cell Identity | M |  | BIT STRING (SIZE(36)) | The leftmost bits of the NR Cell Identity IE correspond to the gNB ID (defined in subclause 9.2.2.22). |

#### 9.2.2.24	E-UTRA CGI

This IE is used to globally identify an E-UTRA cell (see TS 36.300 [12]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.2.25 |  |
| E-UTRA Cell Identity | M |  | BIT STRING (SIZE(28)) | The leftmost bits of the E-UTRA Cell Identity IE correspond to the ng-eNB ID (defined in subclause 9.2.2.10). |

#### 9.2.2.25	PLMN Identity

This IE indicates the PLMN Identity.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | OCTET STRING (SIZE(3)) | Digits 0 to 9 encoded 0000 to 1001, 1111 used as filler digit. Two digits per octet: - bits 4 to 1 of octet n encoding digit 2n-1 - bits 8 to 5 of octet n encoding digit 2n PLMN Identity consists of 3 digits from MCC followed by either: - a filler digit plus 2 digits from MNC (in case of 2 digit MNC) or - 3 digits from MNC (in case of 3 digit MNC). |

#### 9.2.2.26	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the NGAP protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED ( Cell not Available, Handover Desirable for Radio Reasons, Handover Target not Allowed, Invalid AMF Region ID, No Radio Resources Available in Target Cell, Partial Handover, Reduce Load in Serving Cell, Resource Optimisation Handover, Time Critical Handover, TXnRELOCoverall Expiry, TXnRELOCprep Expiry, Unknown GUAMI ID, Unknown Local NG-RAN node UE XnAP ID, Inconsistent Remote NG-RAN node UE XnAP ID, Encryption And/Or Integrity Protection Algorithms Not Supported, Protection Algorithms Not Supported, Multiple PDU Session ID Instances, Unknown PDU Session ID, Unknown QoS Flow ID, Multiple QoS Flow ID Instances, Switch Off Ongoing, Not supported 5QI value, TXnDCoverall Expiry, TXnDCprep Expiry, Action Desirable for Radio Reasons, Reduce Load, Resource Optimisation, Time Critical action, Target not Allowed, No Radio Resources Available, Invalid QoS combination, Encryption Algorithms Not Supported, Procedure cancelled, RRM purpose, Improve User Bit Rate, User Inactivity, Radio Connection With UE Lost, Failure in the Radio Interface Procedure, Bearer Option not Supported, UP integrity protection not possible, UP confidentiality protection not possible, Unspecified, …) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Transport Resource Unavailable, Unspecified, …) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), Unspecified, …) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Hardware Failure, O&M Intervention, Not enough User Plane Processing Resources, Unspecified, …) |  |

- The meaning of the different cause values is specified in the following table. In general, “not supported” cause values indicate that the related capability is missing. On the other hand, “not available” cause values indicate that the related capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause |  | Meaning |  |
| --- | --- | --- | --- |
|  | Cell not Available |  | The concerned cell is not available. |
|  | Handover Desirable for Radio Reasons |  | The reason for requesting handover is radio related. |
|  | Handover Target not Allowed |  | Handover to the indicated target cell is not allowed for the UE in question. |
|  | Invalid AMF Region ID |  | The target NG-RAN node doesn’t belong to the same pool area of the source NG-RAN node, i.e. NG handovers should be attempted instead. |
|  | No Radio Resources Available in Target Cell |  | The target cell doesn’t have sufficient radio resources available. |
|  | Partial Handover |  | Provides a reason for the handover cancellation. The target NG-RAN node did not admit all PDU Sessions included in the HANDOVER REQUEST and the source NG-RAN node estimated service continuity for the UE would be better by not proceeding with handover towards this particular target NG-RAN node. |
|  | Reduce Load in Serving Cell |  | Load in serving cell needs to be reduced. When applied to handover preparation, it indicates the handover is triggered due to load balancing. |
|  | Resource Optimisation Handover |  | The reason for requesting handover is to improve the load distribution with the neighbour cells. |
|  | Time Critical Handover |  | Handover is requested for time critical reason i.e. this cause value is reserved to represent all critical cases where the connection is likely to be dropped if handover is not performed. |
|  | TXnRELOCoverall Expiry |  | The reason for the action is expiry of timer TXnRELOCoverall. |
|  | TXnRELOCprep Expiry |  | Handover Preparation procedure is cancelled when timer TXnRELOCprep expires. |
|  | Unknown GUAMI ID |  | The target NG-RAN node belongs to the same AMF Region of the source NG-RAN node and recognizes the AMF Region ID. However, the GUAMI value is unknown to the target NG-RAN node. |
|  | Unknown Local NG-RAN node UE XnAP ID |  | The action failed because the receiving NG-RAN node does not recognise the local NG-RAN node UE XnAP ID. |
|  | Inconsistent Remote NG-RAN node UE XnAP ID |  | The action failed because the receiving NG-RAN node considers that the received remote NG-RAN node UE XnAP ID is inconsistent.. |
|  | Encryption And/Or Integrity Protection Algorithms Not Supported |  | The target NG-RAN node is unable to support any of the encryption and/or integrity protection algorithms supported by the UE. |
|  | Multiple PDU Session ID Instances |  | The action failed because multiple instances of the same PDU Session had been provided to the NG-RAN node. |
|  | Unknown PDU Session ID |  | The action failed because the PDU Session ID is unknown in the NG-RAN node. |
|  | Unknown QoS Flow ID |  | The action failed because the QoS Flow ID is unknown in the NG-RAN node. |
|  | Multiple QoS Flow ID Instances |  | The action failed because multiple instances of the same QoS flow had been provided to the NG-RAN node. |
|  | Switch Off Ongoing |  | The reason for the action is an ongoing switch off i.e. the concerned cell will be switched off after offloading and not be available. It aides the receiving NG-RAN node in taking subsequent actions, e.g. selecting the target cell for subsequent handovers. |
|  | Not supported 5QI value |  | The action failed because the requested 5QI is not supported. |
|  | TXnDCoverall Expiry |  | The reason for the action is expiry of timer TXnDCoverall. |
|  | TXnDCprep Expiry |  | The reason for the action is expiry of timer TXnDCprep |
|  | Action Desirable for Radio Reasons |  | The reason for requesting the action is radio related. In the current version of this specification applicable for Dual Connectivity only. |
|  | Reduce Load |  | Load in the cell(group) served by the requesting node needs to be reduced. In the current version of this specification applicable for Dual Connectivity only. |
|  | Resource Optimisation |  | The reason for requesting this action is to improve the load distribution with the neighbour cells. In the current version of this specification applicable for Dual Connectivity only. |
|  | Time Critical action |  | The action is requested for time critical reason i.e. this cause value is reserved to represent all critical cases where radio resources are likely to be dropped if the requested action is not performed. In the current version of this specification applicable for Dual Connectivity only. |
|  | Target not Allowed |  | Requested action towards the indicated target cell is not allowed for the UE in question. In the current version of this specification applicable for Dual Connectivity only. |
|  | No Radio Resources Available |  | The cell(s) in the requested node don’t have sufficient radio resources available. In the current version of this specification applicable for Dual Connectivity only. |
|  | Invalid QoS combination |  | The action was failed because of invalid QoS combination. In the current version of this specification applicable for Dual Connectivity only. |
|  | Encryption Algorithms Not Supported |  | The requested NG-RAN node is unable to support any of the encryption algorithms supported by the UE. In the current version of this specification applicable for Dual Connectivity only. |
|  | Procedure cancelled |  | The sending node cancelled the procedure due to other urgent actions to be performed. In the current version of this specification applicable for Dual Connectivity only. |
|  | RRM purpose |  | The procedure is initiated due to node internal RRM purposes. In the current version of this specification applicable for Dual Connectivity only. |
|  | Improve User Bit Rate |  | The reason for requesting this action is to improve the user bit rate. In the current version of this specification applicable for Dual Connectivity only. |
|  | User Inactivity |  | The action is requested due to user inactivity on all PDU Sessions, e.g., NG is requested to be released in order to optimise the radio resources; or S-NG-RAN node didn’t see activity on the PDU session recently. In the current version of this specification applicable for Dual Connectivity only. The semantics of this value may need to be refined. Cause Values for RRC_INACTIVITY should be discussed first. |
|  | Radio Connection With UE Lost |  | The action is requested due to losing the radio connection to the UE. In the current version of this specification applicable for Dual Connectivity only. |
|  | Failure in the Radio Interface Procedure |  | Radio interface procedure has failed. In the current version of this specification applicable for Dual Connectivity only. |
|  | Bearer Option not Supported |  | The requested bearer option is not supported by the sending node. In the current version of this specification applicable for Dual Connectivity only. |
| UP integrity protection not possible |  | The PDU session cannot be accepted according to the required user plane integrity protection policy. |  |
| UP confidentiality protection not possible |  | The PDU session cannot be accepted according to the required user plane confidentiality protection policy. |  |
|  | Unspecified |  | Sent for radio network layer cause when none of the specified cause values applies. |

| Transport Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the above cause values applies but still the cause is Transport Network Layer related. |

| NAS cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the above cause values applies but still the cause is NAS related. |

| Protocol cause | Meaning |
| --- | --- |
| Transfer Syntax Error | The received message included a transfer syntax error. |
| Abstract Syntax Error (Reject) | The received message included an abstract syntax error and the concerning criticality indicated “reject”. |
| Abstract Syntax Error (Ignore And Notify) | The received message included an abstract syntax error and the concerning criticality indicated “ignore and notify”. |
| Message Not Compatible With Receiver State | The received message was not compatible with the receiver state. |
| Semantic Error | The received message included a semantic error. |
| Abstract Syntax Error (Falsely Constructed Message) | The received message contained IEs or IE groups in wrong order or with too many occurrences. |
| Unspecified | Sent when none of the above cause values applies but still the cause is Protocol related. |

| Miscellaneous cause | Meaning |
| --- | --- |
| Control Processing Overload | NG-RAN node control processing overload. |
| Hardware Failure | NG-RAN node hardware failure. |
| Not enough User Plane Processing Resources | NG-RAN node has insufficient user plane processing resources available. |
| O&M Intervention | Operation and Maintenance intervention related to NG-RAN node equipment. |
| Unspecified | Sent when none of the above cause values applies and the cause is not related to any of the categories Radio Network Layer, Transport Network Layer or Protocol. |

#### 9.2.2.27	RAN Paging Priority

This information element contains an indication of the priority to be considered for the paging request.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAN Paging Priority | M |  | INTEGER (1..256) |  |

#### 9.2.2.28	UP Transport Layer Information

This element is used to provide the transport layer information associated with NG or Xn user plane transport. In this release it corresponds to an IP adress and a GTP Tunnel Endpoint Identifier.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE UP Transport Layer Information | M |  |  |  |
| >GTP tunnel |  |  |  |  |
| >>Transport Layer Address | M |  | 9.2.2.50 | The Transport Layer Address is specified in TS 38.424 [19] and TS 38.414 [20]. |
| >>GTP-TEID | M |  | OCTET STRING (4) | The Tunnel Endpoint Identifier (TEID) is specified in TS 29.281 [18] |

#### 9.2.2.29	CP Transport Layer Information

This element is used to provide the transport layer information associated with NG or Xn control plane transport.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE CP Transport Layer Information |  |  |  |  |
| >Endpoint IP address |  |  |  |  |
| >>Endpoint IP Address Address | M |  | Transport Layer Address 9.2.2.50 | The Transport Layer Address is specified in TS 38.424 [19] and TS 38.414 [20].The Transport Layer Address is specified in TS 38.422 [4] and TS 38.412 [21]. |

#### 9.2.2.30	DRB ID

This IE contains the DRB ID.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DRB ID | M |  | INTEGER (1..32, ...) |  |

#### 9.2.2.31	PDU Session Type

This IE defines the PDU Session Type as specified in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Session Type | M |  | ENUMERATED (IPv4, IPv6, IPv4v6, Ethernet, Unstructured, ...) |  |

#### 9.2.2.32	Criticality Diagnostics

The Criticality Diagnostics IE is sent by the NG-RAN node when parts of a received message have not been comprehended or were missing, or if the message contained logical errors. When applicable, it contains information about which IEs were not comprehended or were missing.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Procedure Code | O |  | INTEGER (0..255) | Procedure Code is to be used if Criticality Diagnostics is part of Error Indication procedure, and not within the response message of the same procedure that caused the error. |
| Triggering Message | O |  | ENUMERATED (initiating message, successful outcome, unsuccessful outcome) | The Triggering Message is used only if the Criticality Diagnostics is part of Error Indication procedure. |
| Procedure Criticality | O |  | ENUMERATED (reject, ignore, notify) | This Procedure Criticality is used for reporting the Criticality of the Triggering message (Procedure). |
| Information Element Criticality Diagnostics |  | 0..<maxNrOfErrors> |  |  |
| >IE Criticality | M |  | ENUMERATED (reject, ignore, notify) | The IE Criticality is used for reporting the criticality of the triggering IE. The value "ignore" shall not be used. |
| >IE ID | M |  | INTEGER (0..65535) | The IE ID of the not understood or missing IE |
| >Type Of Error | M |  | ENUMERATED(not understood, missing, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxNrOfErrors | Maximum no. of IE errors allowed to be reported with a single message. The Value is 256. |

#### 9.2.2.33	NG-RAN node UE XnAP ID

The NG-RAN node UE XnAP ID uniquely identifies a UE over the Xn interface within the NG-RAN node.

The use of this IE is defined in TS 38.401 [2].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NG-RAN node UE XnAP ID | M |  | INTEGER (0 .. 232 -1) |  |

#### 9.2.2.34	QoS Flow Indicator

This IE identifies a QoS Flow within a PDU Session. Definition and use of the QoS Flow Indicator is specified in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QoS Flow Indicator | M |  | INTEGER (0 ..63, ...) |  |

#### 9.2.2.35	PDU Session ID

This IE identifies a PDU Session for a UE. Definition and use of the PDU Session ID is specified in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Session ID | M |  | INTEGER (0 ..255) | The value range of this IE may need to be refined. |

#### 9.2.2.36	UE Aggregate Maximum Bit Rate

The UE Aggregate Maximum Bitrate is applicable for all Non-GBR QoS flows per UE which is defined for the Downlink and the Uplink direction and a subscription parameter provided by the AMF to the NG-RAN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Aggregate Maximum Bit Rate |  |  |  | Applicable for Non-GBR QoS flows. |
| >UE Aggregate Maximum Bit Rate Downlink | M |  | Bit Rate 9.2.2.3 | This IE indicates the UE Aggregate Maximum Bit Rate as specified in TS 23.501 [7] in the downlink direction. |
| >UE Aggregate Maximum Bit Rate Uplink | M |  | Bit Rate 9.2.2.3 | This IE indicates the UE Aggregate Maximum Bit Rate as specified in TS 23.501 [7] in the uplink direction. |

#### 9.2.2.37		Cell Assistance Information NR

The Cell Assistance Information IE is used by the NG-RAN node to request information about NR cells.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cell Assistance Type | M |  |  |  |
| >Limited NR List |  |  |  |  |
| >>List of Requested NR Cells |  | 1 .. < maxnoofCellsinNG-RAN node> |  | Included when the NG-RAN node requests a limited list of served NR cells. |
| >>>NR-CGI | M |  | 9.2.2.23 | NR cell for which served NR cell information is requested. |
| >Full NR List |  |  |  |  |
| >>Complete Information Request Indicator | M |  | ENUMERATED (allServedCellsNR, …) | Included when the NG-RAN node requests the complete list of served cells for a gNB |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

#### 9.2.2.38	Index to RAT/Frequency Selection Priority

The Index to RAT/Frequency Selection Priority IE is used to define local configuration for RRM strategies such as camp priorities and control of inter-RAT/inter-frequency mobility in RRC_CONNECTED, as specified in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Index to RAT/Frequency Selection Priority | M |  | INTEGER (1..256) |  |

#### 9.2.2.39	GUAMI

This IE contains the Globally Unique AMF Identifier (GUAMI) as defined in TS 23.003 [22].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN ID | M |  | 9.2.2.25 |  |
| AMF Identifier |  | 1 |  |  |
| >AMF Region ID | M |  | OCTET STRING (SIZE (2)) |  |
| >AMF Set ID | M |  | BIT STRING (SIZE (4)) |  |
| >AMF Pointer | M |  | BIT STRING (SIZE (4)) |  |

#### 9.2.2.40	Target Cell Global ID

This IE contains either an NR CGI or an E-UTRA CGI.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Target Cell | M |  |  |  |
| >NR |  |  |  |  |
| >>NR CGI | M |  | 9.2.2.23 |  |
| >E-UTRA |  |  |  |  |
| >>E-UTRA CGI | M |  | 9.2.2.24 |  |

#### 9.2.2.41	Masked IMEISV

This information element contains the IMEISV value with a mask, to identify a terminal model without identifying an individual Mobile Equipment.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Masked IMEISV | M |  | BIT STRING (SIZE(64)) | Coded as the International Mobile station Equipment Identity and Software Version Number (IMEISV) defined in TS 23.003 [22] with the last 4 digits of the SNR masked by setting the corresponding bits to 1. |

#### 9.2.2.42	NG-RAN Cell Identity

This IE contains either an NR or an E-UTRA Cell Identity.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Cell Identifier | M |  |  |  |
| >NR |  |  |  |  |
| >>NR Cell Identity | M |  | BIT STRING (SIZE(36)) | The leftmost bits of the NR Cell Identity IE correspond to the gNB ID (defined in subclause 9.2.2.22). |
| >E-UTRA |  |  |  |  |
| >>E-UTRA Cell Identity | M |  | BIT STRING (SIZE(28)) | The leftmost bits of the E-UTRA Cell Identity IE correspond to the ng-eNB ID (defined in subclause 9.2.2.10). |

#### 9.2.2.43	DL Forwarding

This element indicates a proposal for forwarding of downlink packets.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL Forwarding | M |  | ENUMERATED (DL forwarding proposed, …) |  |

#### 9.2.2.44	Data Forwarding Accepted

This element indicates that data forwarding was accepted.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL Forwarding | M |  | ENUMERATED (data forwarding accepted, …) |  |

#### 9.2.2.45	Trace Activation

This IE defines parameters related to a trace activation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NG-RAN Trace ID | M |  | OCTET STRING (SIZE(8)) | This IE is composed of the following: Trace Reference defined in TS 32.422 [23] (leftmost 6 octets, with PLMN information encoded as in 9.2.2.25), and Trace Recording Session Reference defined in TS 32.422 [23] (last 2 octets). |
| Interfaces To Trace | M |  | BIT STRING (SIZE(8)) | Each position in the bitmap represents an NG-RAN node interface: first bit = NG-C, second bit = Xn-C, third bit = Uu, fourth bit = F1-C, fifth bit = E1: other bits reserved for future use. Value ‘1’ indicates ‘should be traced’. Value ‘0’ indicates ‘should not be traced’. |
| Trace Depth | M |  | ENUMERATED (minimum, medium, maximum, MinimumWithoutVendorSpecificExtension, MediumWithoutVendorSpecificExtension, MaximumWithoutVendorSpecificExtension, …) | Defined in TS 32.422 [23]. |
| Trace Collection Entity IP Address | M |  | Transport Layer Address 9.2.2.50 | Defined in TS 32.422 [23] |

#### 9.2.2.46		RAN Paging Area

The RAN Paging Area IE defines the paging area for RAN paging UEs in RRC_INACTIVE state.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.2.25 |  |
| CHOICE RAN Paging Area Choice | M |  |  |  |
| >Cell List |  |  |  |  |
| >>Cell List Item |  | 1 .. < maxnoofCellsinRNA> |  |  |
| >>>NG-RAN Cell Identity | M |  | 9.2.2.42 |  |
| >RAN Area ID List |  |  |  |  |
| >>RAN Area ID List Item |  | 1 .. <maxnoofRanAreasinRNA> |  |  |
| >>>RAN Area ID | M |  | 9.2.2.47 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinRNA | Maximum no. of cells in a RAN notification area. Value is 32. |
| maxnoofRanAreasinRNA | Maximum no. of RAN area IDs in a RAN notification area. Value is 16. |

#### 9.2.2.47	RAN Area ID

This IE defines the RAN Area ID.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| TAC | M |  | 9.2.2.17 | Tracking Area Code |
| RANAC | O |  | 9.2.2.73 |  |

#### 9.2.2.48		Location Reporting Information

This information element indicates how the location information should be reported.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Event Type | M |  | ENUMERATED ( report upon change of serving cell, report UE moving presence into or out of the Area of Interest, …) |  |
| Report Area | M |  | ENUMERATED (Cell, …) |  |
| Area Of Interest | O |  | 9.2.2.49 |  |

#### 9.2.2.49	Area of Interest

This IE indicates the area of interest.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| List of Area Of Interest |  | 1.. <maxnoofAOIs> |  |  |
| >List of TAIs |  | 0..1 |  |  |
| >>TAI List Item |  | 1..< maxnoofTAIsinAoI > |  |  |
| >>>PLMN | M |  | 9.2.2.25 |  |
| >>>TAC | M |  | 9.2.2.17 |  |
| >List of Cells |  | 0..1 |  | This IE may need to be refined with SA2. |
| >>Cell List Item |  | 1..<maxnoofcellsinAoI> |  |  |
| >>>PLMN | M |  | 9.2.2.25 |  |
| >>>NG-RAN Cell Identity | M |  | 9.2.2.42 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofAOIs | Maximum no. of areas of interest. Value is 62. |
| maxnoofTAIsinAoI | Maximum no. of tracking areas in an area of interest. Value is 16. |
| maxnoofcellsinAoI | Maximum no. of cells in an Area of Interest. Value is 256. This value may need to be refined. |

#### 9.2.2.50	Transport Layer Address

This IE is defined to contain an IP address.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transport Layer Address |  |  | BIT STRING (1..160, ...) |  |

#### 9.2.2.51	Packet Loss Rate

This IE indicates the Packet Loss Rate.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Packet Loss Rate | M |  | INTEGER(0..1000) | Ratio of lost packets per number of packets sent, expressed in tenth of percent. |

#### 9.2.2.52	Packet Delay Budget

This IE indicates the Packet Delay Budget.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Packet Delay Budget | M |  | INTEGER (0..63) | This IE may need to be refined |

#### 9.2.2.53	Packet Error Rate

This IE indicates the Packet Error Rate.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Packet Error Rate | M |  | INTEGER (0..63) | This IE may need to be refined |

#### 9.2.2.54	Averaging Window

This IE indicates the Averaging Window.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Averaging Window | M |  | INTEGER (0..63) | This IE may need to be refined |

#### 9.2.2.55	Maximum Data Burst Volume

This IE indicates the Maximum Data Burst Volume.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Maximum Data Burst Volume | M |  | INTEGER (0..63) | This IE may need to be refined |

#### 9.2.2.56		Served Cell Information E-UTRA

This IE contains cell configuration information of an E-UTRA cell that a neighbour NG-RAN node may need for the Xn AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| E-UTRA PCI | M |  | INTEGER (0..503, …) | E-UTRA Physical Cell ID |
| ECGI | M |  | 9.2.2.24 |  |
| TAC | M |  | 9.2.2.17 | Tracking Area Code |
| Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs |
| >PLMN Identity | M |  | 9.2.2.25 |  |
| CHOICE E-UTRA-Mode-Info | M |  |  |  |
| >FDD |  |  |  |  |
| >>FDD Info |  | 1 |  |  |
| >>>UL EARFCN | M |  | E-UTRA ARFCN 9.2.2.57 | Corresponds to NUL in TS 36.104 [25] for E-UTRA operating bands for which it is defined; ignored for E-UTRA operating bands for which NUL is not defined |
| >>>DL EARFCN | M |  | E-UTRA ARFCN 9.2.2.57 | Corresponds to NDL in TS 36.104 [25] |
| >>>UL E-UTRA Transmission Bandwidth | M |  | E-UTRA Transmission Bandwidth 9.2.2.58 | Same as DL Transmission Bandwidth in this release; ignored in case UL EARFCN value is ignored |
| >>>DL E-UTRA Transmission Bandwidth | M |  | E-UTRA Transmission Bandwidth 9.2.2.58 |  |
| >TDD |  |  |  |  |
| >>TDD Info |  | 1 |  |  |
| >>>EARFCN | M |  | E-UTRA ARFCN 9.2.2.57 | Corresponds to NDL/NUL in TS 36.104 [25] |
| >>>E-UTRA Transmission Bandwidth | M |  | 9.2.2.58 |  |
| >>>Subframe Assignment | M |  | ENUMERATED (sa0, sa1, sa2, sa3, sa4, sa5, sa6, ...) | Uplink-downlink subframe configuration information defined in TS 36.211 [26] |
| >>>Special Subframe Info |  | 1 |  | Special subframe configuration information defined in TS 36.211 [26] |
| >>>>Special Subframe Patterns | M |  | ENUMERATED (ssp0, ssp1, ssp2, ssp3, ssp4, ssp5, ssp6, ssp7, ssp8, ssp9, ssp10, ...) |  |
| >>>>Cyclic Prefix DL | M |  | ENUMERATED (Normal, Extended,…) |  |
| >>>>Cyclic Prefix UL | M |  | ENUMERATED (Normal, Extended, ...) |  |
| Number of Antenna Ports | O |  | 9.2.2.59 |  |
| PRACH Configuration | O |  | E-UTRA PRACH Configuration 9.2.2.63 |  |
| MBSFN Subframe Info |  | 0..<maxnoofMBSFN> |  | MBSFN subframe defined in TS 36.331 [14] |
| >Radioframe Allocation Period | M |  | ENUMERATED (n1, n2, n4, n8, n16, n32, …) |  |
| >Radioframe Allocation Offset | M |  | INTEGER (0..7, ...) |  |
| >Subframe Allocation | M |  | 9.2.51 |  |
| MultibandInfoList | O |  | 9.2.2.60 |  |
| FreqBandIndicatorPriority | O |  | ENUMERATED (not-broadcasted, broadcasted, ...) | This IE indicates that the eNodeB supports FreqBandIndicationPriority, and whether FreqBandIndicatorPriority is broadcasted in SIB 1 (see TS 36.331 [14]) |
| BandwidthReducedSI | O |  | ENUMERATED (scheduled, ...) | This IE indicates that the SystemInformationBlockType1-BR is scheduled in the cell (see TS 36.331 [14]) |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. The value is 6. The constant needs to be refined. |
| maxnoofMBSFN | Maximum no. of MBSFN frame allocation with different offset. Value is 8. |

#### 9.2.2.57		E-UTRA ARFCN

The E-UTRA Absolute Radio Frequency Channel Number defines the carrier frequency used in an E-UTRAN cell for a given direction (UL or DL) in FDD or for both UL and DL directions in TDD.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| E-UTRA ARFCN | M |  | INTEGER (0..maxEARFCN) | The relation between EARFCN and carrier frequency (in MHz) are defined in TS 36.104 [25]. |

| Range bound | Explanation |
| --- | --- |
| maxEARFCN | Maximum value of EARFCNs. Value is 262143. |

#### 9.2.2.58	E-UTRA	Transmission Bandwidth

The E-UTRA Transmission Bandwidth IE is used to indicate the UL or DL transmission bandwidth expressed in units of resource blocks "NRB" (TS 36.104 [25]). The values bw1, bw6, bw15, bw25, bw50, bw75, bw100 correspond to the number of resource blocks "NRB" 6, 15, 25, 50, 75, 100.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| E-UTRA Transmission Bandwidth | M |  | ENUMERATED (bw6, bw15, bw25, bw50, bw75, bw100,... , bw1) |  |

#### 9.2.2.59		Number of Antenna Ports E-UTRA

The Number of Antenna Ports E-UTRA IE is used to indicate the number of cell specific antenna ports supported by an E-UTRA cell.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Number of Antenna Ports |  |  | ENUMERATED (an1, an2, an4,...) | an1 = One antenna port an2 = Two antenna ports an4 = Four antenna ports |

#### 9.2.2.60	MultibandInfoEutraList

The MultibandInfoEutraList IE contains the additional frequency band indicators that an E-UTRA cell belongs to listed in decreasing order of preference, see TS 36.331 [14].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BandInfo |  | 1..<maxnoofEutraBands> |  |  |
| >FrequencyBandIndicator | M |  | INTEGER (1.. 256, ...) | E-UTRA operating band as defined in TS 36.101 [27, table 5.5-1] |

| Range bound | Explanation |
| --- | --- |
| maxnoofEutraBands | Maximum number of frequency bands that an E-UTRA cell belongs to. The value is 16. |

#### 9.2.2.61	Neighbour Information NR

This IE contains cell configuration information of NR cells that a neighbour NG-RAN node may need to properly operate its own served cells.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Neighbour Information NR |  | 1 .. <maxnoofNeighbours> |  |  |
| >NRPCI | M |  | INTEGER (0..1007) | NR Physical Cell ID |
| >NR-CGI | M |  | 9.2.2.23 |  |
| >TAC | M |  | 9.2.2.17 | Tracking Area Code |
| >CHOICE NR-Mode-Info | M |  |  |  |
| >>FDD |  |  |  |  |
| >>>FDD Info |  | 1 |  |  |
| >>>>UL NR FreqInfo | M |  | NR Frequency Info 9.2.2.15 |  |
| >>>>DL NR FreqInfo | M |  | NR Frequency Info 9.2.2.15 |  |
| >>TDD |  |  |  |  |
| >>>TDD Info |  | 1 |  |  |
| >>>>NR FreqInfo | M |  | NR ARFCN Frequency Info 9.2.2.15 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofNeighbours | Maximum no. of neighbour cells associated to a given served cell. Value is 1024. |

#### 9.2.2.62	Neighbour Information E-UTRA

This IE contains cell configuration information of E-UTRA cells that a neighbour NG-RAN node may need to properly operate its own served cells.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| E-UTRA Neighbour Information E-UTRA |  | 1 .. <maxnoofNeighbours> |  |  |
| >ECGI | M |  | ECGI 9.2.2.24 |  |
| >E-UTRA PCI | M |  | INTEGER (0..503, …) | E-UTRA Physical Cell Identifier of the neighbour cell |
| >EARFCN | M |  | E-UTRA ARFCN 9.2.2.57 | DL EARFCN for FDD or EARFCN for TDD |
| >TAC | M |  | 9.2.2.17 | Tracking Area Code |

| Range bound | Explanation |
| --- | --- |
| maxnoofNeighbours | Maximum no. of neighbour cells associated to a given served cell. Value is 1024. |

#### 9.2.2.63	E-UTRA PRACH Configuration

This IE indicates the E-UTRA PRACH resources used in an E-UTRA neighbour cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RootSequenceIndex | M |  | INTEGER (0..837) | See section 5.7.2. in TS 36.211 [26] |
| ZeroCorrelationZoneConfiguration | M |  | INTEGER (0..15) | See section 5.7.2. in TS 36.211 [26] |
| HighSpeedFlag | M |  | ENUMERATED (true, false, ...) | "true" corresponds to Restricted set and "false" to Unrestricted set. See section 5.7.2 in TS 36.211 [26] |
| PRACH-FrequencyOffset | M |  | INTEGER (0..94) | See section 5.7.1 of TS 36.211 [26] |
| PRACH-ConfigurationIndex | O |  | INTEGER (0..63) | Mandatory for TDD, shall not be present for FDD. See section 5.7.1. in TS 36.211 [26] |

#### 9.2.2.64	UE Security Capabilities

The UE Security Capabilities IE defines the supported algorithms for encryption and integrity protection in the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NR Encryption Algorithms | M |  | BIT STRING (SIZE(16, …)) | Each position in the bitmap represents an encryption algorithm: “all bits equal to 0” – UE supports no other NR algorithm than NEA0, “first bit” – 128-NEA1, “second bit” – 128-NEA2, “third bit” – 128-NEA3, other bits reserved for future use. Value ‘1’ indicates support and value ‘0’ indicates no support of the algorithm. Algorithms are defined in TS 33.501 [28]. |
| NR Integrity Protection Algorithms | M |  | BIT STRING (SIZE(16, …)) | Each position in the bitmap represents an integrity protection algorithm: “all bits equal to 0” – UE supports no other NR algorithm than NIA0, “first bit” – 128-NIA1, “second bit” – 128-NIA2, “third bit” – 128-NIA3, other bits reserved for future use. Value ‘1’ indicates support and value ‘0’ indicates no support of the algorithm. Algorithms are defined in TS 33.501 [28]. |
| E-UTRA Encryption Algorithms | M |  | BIT STRING (SIZE(16, …)) | Each position in the bitmap represents an encryption algorithm: “all bits equal to 0” – UE supports no other algorithm than EEA0, “first bit” – 128-EEA1, “second bit” – 128-EEA2, “third bit” – 128-EEA3, other bits reserved for future use. Value ‘1’ indicates support and value ‘0’ indicates no support of the algorithm. Algorithms are defined in TS 33.401 [29]. |
| E-UTRA Integrity Protection Algorithms | M |  | BIT STRING (SIZE(16, …)) | Each position in the bitmap represents an integrity protection algorithm: “all bits equal to 0” – UE supports no other algorithm than EIA0, “first bit” – 128-EIA1, “second bit” – 128-EIA2, “third bit” – 128-EIA3, other bits reserved for future use. Value ‘1’ indicates support and value ‘0’ indicates no support of the algorithm. Algorithms are defined in TS 33.401 [29]. |

#### 9.2.2.65	AS Security Information

The AS Security Information IE is used to generate the key material to be used for AS security with the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Key NG-RAN Star | M |  | BIT STRING (256) | KNG-RAN* defined in TS 33.501 [28]. |
| Next Hop Chaining Count | M |  | INTEGER (0..7) | Next Hop Chaining Count (NCC) defined in TS 33.501 [28] |

#### 9.2.2.66	S-NG-RAN node Security Key

The S-NG-RAN node Security Key IE is used to apply security in the S-NG-RAN node as defined in TS 33.501 [28].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| S-NG-RAN node Security Key | M |  | BIT STRING (SIZE(256)) | The S-KSN which is provided by the M-NG-RAN node, see TS 33.501 [28]. |

#### 9.2.2.67	AMF UE NGAP ID

This IE is defined in TS 38.413 [5] and used to refer to the UE Context in the serving AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| AMF UE NGAP ID | M |  | INTEGER (0 .. 232 -1) | As defined in TS 38.413 [5]. |

#### 9.2.2.68	Mobility Restriction List

This IE defines roaming or access restrictions for subsequent mobility actions for which the NR-RAN provides information about the target of the mobility action towards the UE, e.g., handover, or for SCG selection during dual connectivity operation or for assigning proper RNAs. If the NG-RAN receives the Mobility Restriction List IE, it shall overwrite previously received restriction information. NG-RAN behaviour upon receiving this IE is specified in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Serving PLMN | M |  | PLMN Identity 9.2.2.25 |  |
| Equivalent PLMNs |  | 0..<maxnoofEPLMNs> |  | Allowed PLMNs in addition to Serving PLMN. This list corresponds to the list of “equivalent PLMNs” as defined in TS 24.501 [30]. This list is part of the roaming restriction information. Roaming restrictions apply to PLMNs other than the Serving PLMN and Equivalent PLMNs. |
| >PLMN Identity | M |  | 9.2.2.25 |  |
| RAT Restrictions | O | 0..<maxnoofPLMNs> |  | This IE contains RAT restriction related information as specified in TS 23.501 [7]. |
| >PLMN Identity | O |  | 9.2.2.25 |  |
| >RAT Restriction Information | O |  | BIT STRING { e-UTRA (0), nR (1) } (SIZE(8, …)) | Each position in the bitmap represents a RAT. If a bit is set to "1", the respective RAT is restricted for the UE. If a bit is set to "0", the respective RAT is not restricted for the UE. This version of the specification does not use bits 2-7, the sending node shall set bits 2-7 to "0", the sender shall ignore bits 2-7. |
| Forbidden Area Information |  | 0..<maxnoofPLMNs> |  | This IE contains Forbidden Area information as specified in TS 23.501 [7]. |
| >PLMN Identity | M |  | 9.2.2.25 |  |
| >Forbidden TACs |  | 1..<maxnoofForbTACs> |  |  |
| >>TAC | M |  | 9.2.2.17 | The TAC of the forbidden TAI. |
| Service Area Information |  | 0..<maxnoofPLMNs> |  | This IE contains Service Area Restriction information as specified in TS 23.501 [7]. |
| >PLMN Identity | M |  | 9.3.3.5 |  |
| >Allowed TACs |  | 0..<maxnooAllowedAreas> |  |  |
| >>TAC | M |  | 9.3.3.10 | The TAC of the allowed TAI. |
| >Not Allowed TACs |  | 0..<maxnooAllowedAreas> |  |  |
| >>TAC | M |  | 9.3.3.10 | The TAC of the not-allowed TAI. |

| Range bound | Explanation |
| --- | --- |
| maxnoofEPLMNs | Maximum no. of equivalent PLMNs. Value is 15. |
| maxnoofPLMNs | Maximum no. of allowed PLMNs. Value is 16. |
| maxnoofForbTACs | Maximum no. of forbidden Tracking Area Codes. Value is 4096. |
| maxnoofAllowedAreas | Maximum no. of allowed or not allowed Tracking Areas. Value is 16. |

#### 9.2.2.69	COUNT Value for PDCP SN Length 12

This information element indicates the 12-bit long PDCP sequence number and the corresponding 20 bit long Hyper Frame Number.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDCP-SN Length 12 | M |  | INTEGER (0..4095) |  |
| HFN for PDCP-SN Length 12 | M |  | INTEGER (0..1048575) |  |

#### 9.2.2.70	COUNT Value for PDCP SN Length 18

This information element indicates the 18-bit long PDCP sequence number and the corresponding 14 bit long Hyper Frame Number.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDCP-SN Length 18 | M |  | INTEGER (0..262143) |  |
| HFN for PDCP-SN Length 18 | M |  | INTEGER (0..16383) |  |

#### 9.2.2.71	UE Context ID

This IE is used to address a UE Context within an NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE UE Context ID | M |  |  |  |
| >RRC Resume |  |  |  |  |
| >>I-RNTI | M |  | 9.2.2.77 | How the new NG-RAN node is able to resolve the old NG-RAN ID from the I-RNTI is a matter of proper configuration in the old and new NG-RAN node. |
| >RRC Reestablishment |  |  |  |  |
| >>C-RNTI | M |  | BIT STRING (SIZE (16)) | C-RNTI contained in the RRC Re-establishment Request message (TS 38.331 [10]) |
| >> Failure Cell PCI | M |  | Cell PCI 9.2.2.72 |  |

#### 9.2.2.72	Cell PCI

This IE defines physical cell ID of a cell served by an NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE NG-RAN node | M |  |  |  |
| >gNB |  |  |  |  |
| >>NR PCI | M |  | INTEGER (0..1007, …) | NR Physical Cell ID |
| >ng-eNB |  |  |  |  |
| >>E-UTRA PCI | M |  | INTEGER (0..503, …) | E-UTRA Physical Cell ID |

#### 9.2.2.73	RAN Area Code

This IE defines the RAN Area Code.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| RANAC | M |  | BIT STRING (SIZE (6)) |  |

#### 9.2.2.74	Assistance Data for RAN Paging

This IE provides assistance information for RAN paging.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAN Paging Attempt Information | O |  | 9.2.2.75 |  |

#### 9.2.2.75	RAN Paging Attempt Information

This IE includes information related to the RAN paging attempt over Xn.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging Attempt Count | M |  | INTEGER (1..16,...) | Number of the RAN paging attempt. |
| Intended Number of Paging Attempts | M |  | INTEGER (1..16,…) | Intended number of RAN paging attempts. |
| Next Paging Area Scope | O |  | ENUMERATED (same, changed, …) | Indicates whether the RAN paging area scope will change at next RAN paging attempt. |

#### 9.2.2.76	UE RAN Paging Identity

The IE defines the UE Identity for RAN paging a UE in RRC_INACTIVE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE UE RAN Paging Identity | M |  |  |  |
| >I-RNTI |  |  |  |  |
| >>I-RNTI | M |  | 9.2.2.77 |  |

#### 9.2.2.77	I-RNTI

The I-RNTI is defined for allocation in an NR or E-UTRA serving cell as a reference to a UE Context within an NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| I-RNTI | M |  | BIT STRING (SIZE (40)) | This IE may need to be refined. |

#### 9.2.2.78	Xn Benefit Value

The Xn Benefit Value IE indicates the quantified benefit of the signalling connection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Xn Benefit Value | M |  | INTEGER (1..8, …) | Value 1 indicates lowest benefit, and 8 indicates highest benefit. |

## 9.3	Message and Information Element Abstract Syntax (with ASN.1)

Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018, the content of section 9.3 related to Dual Connectivity is FFS.

### 9.3.1	General

XnAP ASN.1 definition conforms to ITU-T Rec. X.680 [16] and ITU-T Rec. X.681 [17].

Sub clause 9.3 presents the Abstract Syntax of the XnAP protocol with ASN.1. In case there is contradiction between the ASN.1 definition in this sub clause and the tabular format in sub clause 9.1 and 9.2, the ASN.1 shall take precedence, except for the definition of conditions for the presence of conditional elements, in which the tabular format shall take precedence.

The ASN.1 definition specifies the structure and content of XnAP messages. XnAP messages can contain any IEs specified in the object set definitions for that message without the order or number of occurrence being restricted by ASN.1. However, for this version of the standard, a sending entity shall construct an XnAP message according to the PDU definitions module and with the following additional rules:

-	IEs shall be ordered (in an IE container) in the order they appear in object set definitions.

-	Object set definitions specify how many times IEs may appear. An IE shall appear exactly once if the presence field in an object has value "mandatory". An IE may appear at most once if the presence field in an object has value "optional" or "conditional". If in a tabular format there is multiplicity specified for an IE (i.e. an IE list) then in the corresponding ASN.1 definition the list definition is separated into two parts. The first part defines an IE container list in which the list elements reside. The second part defines list elements. The IE container list appears as an IE of its own. For this version of the standard an IE container list may contain only one kind of list elements.

NOTE:	In the above, "IE" means an IE in the object set with an explicit ID. If one IE needs to appear more than once in one object set, then the different occurrences have different IE IDs.

If an XnAP message that is not constructed as defined above is received, this shall be considered as Abstract Syntax Error, and the message shall be handled as defined for Abstract Syntax Error in clause 10.

### 9.3.2	Usage of Private Message Mechanism for Non-standard Use

The private message mechanism for non-standard use may be used:

-	for special operator (and/or vendor) specific features considered not to be part of the basic functionality, i.e. the functionality required for a complete and high-quality specification in order to guarantee multivendor inter-operability.

-	by vendors for research purposes, e.g. to implement and evaluate new algorithms/features before such features are proposed for standardisation.

The private message mechanism shall not be used for basic functionality. Such functionality shall be standardised.

### 9.3.3	Elementary Procedure Definitions

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

XnAP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM XnAP-CommonDataTypes

HandoverRequest,

HandoverRequestAcknowledge,

HandoverPreparationFailure,

SNStatusTransfer,

UEContextRelease,

HandoverCancel,

RANPaging,

RetrieveUEContextRequest,

RetrieveUEContextResponse,

RetrieveUEContextFailure,

DataForwardingAddressIndication,

SNodeAdditionRequest,

SNodeAdditionRequestAcknowledge,

SNodeAdditionRequestReject,

SNodeReconfigurationComplete,

SNodeModificationRequest,

SNodeModificationRequestAcknowledge,

SNodeModificationRequestReject,

SNodeModificationRequired,

SNodeModificationConfirm,

SNodeModificationRefuse,

SNodeReleaseRequest,

SNodeReleaseRequestAcknowledge,

SNodeReleaseReject,

SNodeReleaseRequired,

SNodeReleaseConfirm,

SNodeCounterCheckRequest,

SNodeChangeRequired,

SNodeChangeConfirm,

SNodeChangeRefuse,

RRCTransfer,

XnSetupRequest,

XnSetupResponse,

XnSetupFailure,

NGRANNodeConfigurationUpdate,

NGRANNodeConfigurationUpdateAcknowledge,

NGRANNodeConfigurationUpdateFailure,

CellActivationRequest,

CellActivationResponse,

CellActivationFailure,

ResetRequest,

ResetResponse,

ErrorIndication,

PrivateMessage

FROM XnAP-PDU-Contents

id-handoverPreparation,

id-sNStatusTransfer,

id-handoverCancel,

id-retrieveUEContext,

id-rANPaging,

id-dataForwardingAddressIndication,

id-uEContextRelease,

id-sNGRANnodeAdditionPreparation,

id-sNGRANnodeReconfigurationCompletion,

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-mNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeCounterCheck,

id-sNGRANnodeChange,

id-rRCTransfer,

id-xnSetup,

id-nGRANnodeConfigurationUpdate,

id-cellActivation,

id-reset,

id-errorIndication,

id-privateMessage

FROM XnAP-Constants;

-- **************************************************************

--

-- Interface Elementary Procedure Class

--

-- **************************************************************

XNAP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome				OPTIONAL,

&UnsuccessfulOutcome				OPTIONAL,

&procedureCode			ProcedureCode 	UNIQUE,

&criticality			Criticality 	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE		&InitiatingMessage

[SUCCESSFUL OUTCOME		&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE			&procedureCode

[CRITICALITY			&criticality]

}

-- **************************************************************

--

-- Interface PDU Definition

--

-- **************************************************************

XnAP-PDU ::= CHOICE {

initiatingMessage	InitiatingMessage,

successfulOutcome	SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

XNAP-ELEMENTARY-PROCEDURES XNAP-ELEMENTARY-PROCEDURE ::= {

XNAP-ELEMENTARY-PROCEDURES-CLASS-1			|

XNAP-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

XNAP-ELEMENTARY-PROCEDURES-CLASS-1 XNAP-ELEMENTARY-PROCEDURE ::= {

handoverPreparation										|

retrieveUEContext										|

sNGRANnodeAdditionPreparation							|

mNGRANnodeinitiatedSNGRANnodeModificationPreparation	|

sNGRANnodeinitiatedSNGRANnodeModificationPreparation	|

mNGRANnodeinitiatedSNGRANnodeRelease					|

sNGRANnodeinitiatedSNGRANnodeRelease					|

sNGRANnodeChange										|

xnSetup													|

nGRANnodeConfigurationUpdate							|

cellActivation											|

reset													,

...

}

XNAP-ELEMENTARY-PROCEDURES-CLASS-2 XNAP-ELEMENTARY-PROCEDURE ::= {

sNStatusTransfer						|

handoverCancel							|

rANPaging								|

dataForwardingAddressIndication			|

uEContextRelease						|

sNGRANnodeReconfigurationCompletion		|

sNGRANnodeCounterCheck					|

rRCTransfer								|

errorIndication							|

privateMessage							,

...

}

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

handoverPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverRequest

SUCCESSFUL OUTCOME		HandoverRequestAcknowledge

UNSUCCESSFUL OUTCOME	HandoverPreparationFailure

PROCEDURE CODE			id-handoverPreparation

CRITICALITY				reject

}

sNStatusTransfer	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNStatusTransfer

PROCEDURE CODE			id-sNStatusTransfer

CRITICALITY				ignore

}

handoverCancel	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverCancel

PROCEDURE CODE			id-handoverCancel

CRITICALITY				ignore

}

retrieveUEContext	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RetrieveUEContextRequest

SUCCESSFUL OUTCOME		RetrieveUEContextResponse

UNSUCCESSFUL OUTCOME	RetrieveUEContextFailure

PROCEDURE CODE			id-retrieveUEContext

CRITICALITY				reject

}

rANPaging	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RANPaging

PROCEDURE CODE			id-rANPaging

CRITICALITY				reject

}

dataForwardingAddressIndication	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DataForwardingAddressIndication

PROCEDURE CODE			id-dataForwardingAddressIndication

CRITICALITY				reject

}

uEContextRelease	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextRelease

PROCEDURE CODE			id-uEContextRelease

CRITICALITY				reject

}

sNGRANnodeAdditionPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeAdditionRequest

SUCCESSFUL OUTCOME		SNodeAdditionRequestAcknowledge

UNSUCCESSFUL OUTCOME	SNodeAdditionRequestReject

PROCEDURE CODE			id-sNGRANnodeAdditionPreparation

CRITICALITY				reject

}

sNGRANnodeReconfigurationCompletion	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeReconfigurationComplete

PROCEDURE CODE			id-sNGRANnodeReconfigurationCompletion

CRITICALITY				reject

}

mNGRANnodeinitiatedSNGRANnodeModificationPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeModificationRequest

SUCCESSFUL OUTCOME		SNodeModificationRequestAcknowledge

UNSUCCESSFUL OUTCOME	SNodeModificationRequestReject

PROCEDURE CODE			id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation

CRITICALITY				reject

}

sNGRANnodeinitiatedSNGRANnodeModificationPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeModificationRequired

SUCCESSFUL OUTCOME		SNodeModificationConfirm

UNSUCCESSFUL OUTCOME	SNodeModificationRefuse

PROCEDURE CODE			id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation

CRITICALITY				reject

}

mNGRANnodeinitiatedSNGRANnodeRelease	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeReleaseRequest

SUCCESSFUL OUTCOME		SNodeReleaseRequestAcknowledge

UNSUCCESSFUL OUTCOME	SNodeReleaseReject

PROCEDURE CODE			id-mNGRANnodeinitiatedSNGRANnodeRelease

CRITICALITY				reject

}

sNGRANnodeinitiatedSNGRANnodeRelease	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeReleaseRequired

SUCCESSFUL OUTCOME		SNodeReleaseConfirm

PROCEDURE CODE			id-sNGRANnodeinitiatedSNGRANnodeRelease

CRITICALITY				reject

}

sNGRANnodeCounterCheck	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeCounterCheckRequest

PROCEDURE CODE			id-sNGRANnodeCounterCheck

CRITICALITY				reject

}

sNGRANnodeChange		XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNodeChangeRequired

SUCCESSFUL OUTCOME		SNodeChangeConfirm

UNSUCCESSFUL OUTCOME	SNodeChangeRefuse

PROCEDURE CODE			id-sNGRANnodeChange

CRITICALITY				reject

}

rRCTransfer	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RRCTransfer

PROCEDURE CODE			id-rRCTransfer

CRITICALITY				ignore

}

xnSetup	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		XnSetupRequest

SUCCESSFUL OUTCOME		XnSetupResponse

UNSUCCESSFUL OUTCOME	XnSetupFailure

PROCEDURE CODE			id-xnSetup

CRITICALITY				reject

}

nGRANnodeConfigurationUpdate	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NGRANNodeConfigurationUpdate

SUCCESSFUL OUTCOME		NGRANNodeConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	NGRANNodeConfigurationUpdateFailure

PROCEDURE CODE			id-nGRANnodeConfigurationUpdate

CRITICALITY				reject

}

cellActivation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CellActivationRequest

SUCCESSFUL OUTCOME		CellActivationResponse

UNSUCCESSFUL OUTCOME	CellActivationFailure

PROCEDURE CODE			id-cellActivation

CRITICALITY				reject

}

reset	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResetRequest

SUCCESSFUL OUTCOME		ResetResponse

PROCEDURE CODE			id-reset

CRITICALITY				reject

}

errorIndication	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-errorIndication

CRITICALITY				ignore

}

privateMessage			XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-privateMessage

CRITICALITY				ignore

}

END

### 9.3.4	PDU Definitions

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ActivationIDforCellActivation,

ActivationIDforCellActivation,

AMF-Pool-Information,

AssistanceDataForPaging,

Cause,

CellAssistanceInfoNR,

CPTransportLayerInformation,

CriticalityDiagnostics,

DRBsSubjectToStatusTransfer-List,

DRBToQoSFlowMapping-List,

E-UTRA-CGI,

GlobalNG-RANNode-ID,

GUAMI,

I-RNTI,

List-of-served-cells-E-UTRA,

List-of-served-cells-NR,

List-of-served-cells-ToUpdate-E-UTRA,

List-of-served-cells-ToUpdate-NR,

MAC-I,

MaskedIMEISV,

MaskedIMEISV,

NG-RAN-Cell-Identity,

NG-RANnodeUEXnAPID,

NG-RANnodeUEXnAPID,

NR-CGI,

PagingDRX,

PDUSession-ID,

PDUSessionResourcesAdmitted-List,

PDUSessionResourcesNotAdmitted-List,

PDUSessionResourcesToBeSetup-List,

PDUSessionType,

QoSFlows-List,

RANPagingArea,

RANPagingPriority,

SecurityIndication,

S-NSSAI,

TAISupport-List,

Target-CGI,

TraceActivation,

TraceActivation,

UEAggregateMaximumBitRate,

UEContextInfoRetrUECtxtResp,

UEIdentityIndexValue,

UEPagingIdentity,

UPTransportLayerInformation

FROM XnAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

XNAP-PRIVATE-IES,

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES,

XNAP-PROTOCOL-IES-PAIR

FROM XnAP-Containers

id-ActivatedServedCells,

id-ActivationIDforCellActivation,

id-AMF-Pool-Information,

id-AssistanceDataForPaging,

id-Cause,

id-cellAssistanceInfoNR,

id-ConfigurationUpdateInitiatingNodeChoice,

id-CriticalityDiagnostics,

id-DRBsSubjectToStatusTransfer-List,

id-GlobalNG-RAN-node-ID,

id-GUAMI,

id-I-RNTI,

id-List-of-served-cells-E-UTRA,

id-List-of-served-cells-NR,

id-MAC-I,

id-MaskedIMEISV,

id-MaskedIMEISV,

id-new-NG-RAN-Cell-Identity,

id-newNG-RANnodeUEXnAPID,

id-oldNG-RANnodeUEXnAPID,

id-PagingDRX,

id-PDUSessionResourcesAdmitted-List,

id-PDUSessionResourcesNotAdmitted-List,

id-RANPagingArea,

id-RANPagingPriority,

id-ServedCellsToActivate,

id-servedCellsToUpdate-E-UTRA,

id-ServedCellsToUpdateInitiatingNodeChoice,

id-servedCellsToUpdate-NR,

id-sourceNG-RANnodeUEXnAPID,

id-TAISupport-list,

id-Target2SourceNG-RANnodeTranspContainer,

id-targetCellGlobalID,

id-targetNG-RANnodeUEXnAPID,

id-TraceActivation,

id-TraceActivation,

id-UEContextInfoHORequest,

id-UEContextInfoRetrUECtxtResp,

id-UEIdentityIndexValue,

id-UEPagingIdentity,

maxnoofCellsinNGRANnode,

maxnoofPDUSessions

FROM XnAP-Constants;

-- **************************************************************

--

- -- HANDOVER REQUEST

--

-- **************************************************************

HandoverRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverRequest-IEs}},

...

}

HandoverRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID				CRITICALITY reject	TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY reject	TYPE Cause										PRESENCE mandatory}|

{ ID id-targetCellGlobalID					CRITICALITY reject	TYPE Target-CGI									PRESENCE mandatory}|

{ ID id-GUAMI								CRITICALITY reject	TYPE GUAMI										PRESENCE mandatory}|

{ ID id-UEContextInfoHORequest				CRITICALITY reject	TYPE UEContextInfoHORequest						PRESENCE mandatory}|

{ ID id-TraceActivation						CRITICALITY ignore	TYPE TraceActivation							PRESENCE optional }|

{ ID id-MaskedIMEISV						CRITICALITY ignore	TYPE MaskedIMEISV								PRESENCE optional },

...

}

UEContextInfoHORequest ::= SEQUENCE {

ng-c-UE-reference					OCTET STRING,	-- FFS

cp-TNL-info-source					CPTransportLayerInformation,

security-info						OCTET STRING,   -- FFS

ue-AMBR								UEAggregateMaximumBitRate,

pduSessionResourcesToBeSetup-List	PDUSessionResourcesToBeSetup-List,

rrc-Context							OCTET STRING,

hlr									OCTET STRING														OPTIONAL, -- FFS

iE-Extensions						ProtocolExtensionContainer { {UEContextInfoHORequest-ExtIEs} }	OPTIONAL,

...

}

UEContextInfoHORequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

-- **************************************************************

--

- -- HANDOVER REQUEST ACKNOWLEDGE

--

-- **************************************************************

HandoverRequestAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverRequestAcknowledge-IEs}},

...

}

HandoverRequestAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID				CRITICALITY ignore	TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID				CRITICALITY ignore	TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-PDUSessionResourcesAdmitted-List		CRITICALITY ignore	TYPE PDUSessionResourcesAdmitted-List			PRESENCE mandatory}|

{ ID id-PDUSessionResourcesNotAdmitted-List		CRITICALITY ignore	TYPE PDUSessionResourcesNotAdmitted-List		PRESENCE optional }|

{ ID id-Target2SourceNG-RANnodeTranspContainer	CRITICALITY ignore	TYPE OCTET STRING									PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

- -- HANDOVER PREPARATION FAILURE

--

-- **************************************************************

HandoverPreparationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverPreparationFailure-IEs}},

...

}

HandoverPreparationFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID				CRITICALITY ignore	TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause										PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional },

...

}

-- **************************************************************

--

- -- SN STATUS TRANSFER

--

-- **************************************************************

SNStatusTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{SNStatusTransfer-IEs}},

...

}

SNStatusTransfer-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-DRBsSubjectToStatusTransfer-List		CRITICALITY ignore		TYPE DRBsSubjectToStatusTransfer-List		PRESENCE mandatory},

...

}

-- **************************************************************

--

- -- UE CONTEXT RELEASE

--

-- **************************************************************

UEContextRelease ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{UEContextRelease-IEs}},

...

}

UEContextRelease-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory},

...

}

-- **************************************************************

--

- -- HANDOVER CANCEL

--

-- **************************************************************

HandoverCancel ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverCancel-IEs}},

...

}

HandoverCancel-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE optional }|

{ ID id-Cause										CRITICALITY ignore		TYPE Cause											PRESENCE mandatory},

...

}

-- **************************************************************

--

- -- RAN PAGING

--

-- **************************************************************

RANPaging ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{RANPaging-IEs}},

...

}

RANPaging-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-UEIdentityIndexValue				CRITICALITY reject		TYPE UEIdentityIndexValue							PRESENCE mandatory}|

{ ID id-UEPagingIdentity					CRITICALITY ignore		TYPE UEPagingIdentity								PRESENCE mandatory}|

{ ID id-PagingDRX							CRITICALITY ignore		TYPE PagingDRX										PRESENCE mandatory}|

{ ID id-RANPagingArea						CRITICALITY reject		TYPE RANPagingArea									PRESENCE mandatory}|

{ ID id-RANPagingPriority					CRITICALITY ignore		TYPE RANPagingPriority								PRESENCE optional }|

{ ID id-AssistanceDataForPaging				CRITICALITY ignore		TYPE AssistanceDataForPaging						PRESENCE optional },

...

}

-- **************************************************************

--

- -- RETRIEVE UE CONTEXT REQUEST

--

-- **************************************************************

RetrieveUEContextRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{RetrieveUEContextRequest-IEs}},

...

}

RetrieveUEContextRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-I-RNTI									CRITICALITY reject		TYPE I-RNTI											PRESENCE mandatory}|

{ ID id-MAC-I									CRITICALITY reject		TYPE MAC-I											PRESENCE mandatory}|

{ ID id-new-NG-RAN-Cell-Identity				CRITICALITY reject		TYPE NG-RAN-Cell-Identity						PRESENCE mandatory},

...

}

-- **************************************************************

--

- -- RETRIEVE UE CONTEXT RESPONSE

--

-- **************************************************************

RetrieveUEContextResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ RetrieveUEContextResponse-IEs}},

...

}

RetrieveUEContextResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-oldNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-GUAMI									CRITICALITY reject		TYPE GUAMI											PRESENCE mandatory}|

{ ID id-UEContextInfoRetrUECtxtResp				CRITICALITY reject		TYPE UEContextInfoRetrUECtxtResp				PRESENCE mandatory}|

{ ID id-TraceActivation							CRITICALITY ignore		TYPE TraceActivation								PRESENCE optional }|

{ ID id-MaskedIMEISV							CRITICALITY ignore		TYPE MaskedIMEISV									PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

- -- RETRIEVE UE CONTEXT FAILURE

--

-- **************************************************************

RetrieveUEContextFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ RetrieveUEContextFailure-IEs}},

...

}

RetrieveUEContextFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

- -- DATA FORWARDING ADDRESS INDICATION

--

-- **************************************************************

DataForwardingAddressIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ DataForwardingAddressIndication-IEs}},

...

}

DataForwardingAddressIndication-IEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

- -- S-NODE ADDITION REQUEST

--

-- **************************************************************

-- Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018.

-- ASN.1 for this message is FFS.

SNodeAdditionRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeAdditionRequest-IEs}},

...

}

SNodeAdditionRequest-IEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

- -- S-NODE ADDITION REQUEST ACKNOWLEDGE

--

-- **************************************************************

-- Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018.

-- ASN.1 for this message is FFS.

SNodeAdditionRequestAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeAdditionRequestAcknowledge-IEs}},

...

}

SNodeAdditionRequestAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

- -- S-NODE ADDITION REQUEST REJECT

--

-- **************************************************************

-- Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018.

-- ASN.1 for this message is FFS.

SNodeAdditionRequestReject ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeAdditionRequestReject-IEs}},

...

}

SNodeAdditionRequestReject-IEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

- -- S-NODE RECONFIGURATION COMPLETE

--

-- **************************************************************

-- Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018.

-- ASN.1 for this message is FFS.

SNodeReconfigurationComplete ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReconfigurationComplete-IEs}},

...

}

SNodeReconfigurationComplete-IEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

- -- S-NODE MODIFICATION REQUEST

--

-- **************************************************************

-- Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018.

-- ASN.1 for this message is FFS.

SNodeModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationRequest-IEs}},

...

}

SNodeModificationRequest-IEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

- -- S-NODE MODIFICATION REQUEST ACKNOWLEDGE

--

-- **************************************************************

-- Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018.

-- ASN.1 for this message is FFS.

SNodeModificationRequestAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationRequestAcknowledge-IEs}},

...

}

SNodeModificationRequestAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

- -- S-NODE MODIFICATION REQUEST REJECT

--

-- **************************************************************

-- Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018.

-- ASN.1 for this message is FFS.

SNodeModificationRequestReject ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationRequestReject-IEs}},

...

}

SNodeModificationRequestReject-IEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

- -- S-NODE MODIFICATION REQUIRED

--

-- **************************************************************

-- Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018.

-- ASN.1 for this message is FFS.

SNodeModificationRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationRequired-IEs}},

...

}

SNodeModificationRequired-IEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

- -- S-NODE MODIFICATION CONFIRM

--

-- **************************************************************

-- Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018.

-- ASN.1 for this message is FFS.

SNodeModificationConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationConfirm-IEs}},

...

}

SNodeModificationConfirm-IEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

- -- S-NODE MODIFICATION REFUSE

--

-- **************************************************************

-- Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018.

-- ASN.1 for this message is FFS.

SNodeModificationRefuse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationRefuse-IEs}},

...

}

SNodeModificationRefuse-IEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

- -- S-NODE RELEASE REQUEST

--

-- **************************************************************

-- Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018.

-- ASN.1 for this message is FFS.

SNodeReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseRequest-IEs}},

...

}

SNodeReleaseRequest-IEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

- -- S-NODE RELEASE REQUEST ACKNOWLEDGE

--

-- **************************************************************

-- Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018.

-- ASN.1 for this message is FFS.

SNodeReleaseRequestAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseRequestAcknowledge-IEs}},

...

}

SNodeReleaseRequestAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

- -- S-NODE RELEASE REJECT

--

-- **************************************************************

-- Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018.

-- ASN.1 for this message is FFS.

SNodeReleaseReject ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseReject-IEs}},

...

}

SNodeReleaseReject-IEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

- -- S-NODE RELEASE REQUIRED

--

-- **************************************************************

-- Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018.

-- ASN.1 for this message is FFS.

SNodeReleaseRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseRequired-IEs}},

...

}

SNodeReleaseRequired-IEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

- -- S-NODE RELEASE CONFIRM

--

-- **************************************************************

-- Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018.

-- ASN.1 for this message is FFS.

SNodeReleaseConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseConfirm-IEs}},

...

}

SNodeReleaseConfirm-IEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

- -- S-NODE COUNTER CHECK REQUEST

--

-- **************************************************************

-- Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018.

-- ASN.1 for this message is FFS.

SNodeCounterCheckRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeCounterCheckRequest-IEs}},

...

}

SNodeCounterCheckRequest-IEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

- -- S-NODE CHANGE REQUIRED

--

-- **************************************************************

-- Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018.

-- ASN.1 for this message is FFS.

SNodeChangeRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeChangeRequired-IEs}},

...

}

SNodeChangeRequired-IEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

- -- S-NODE CHANGE CONFIRM

--

-- **************************************************************

-- Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018.

-- ASN.1 for this message is FFS.

SNodeChangeConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeChangeConfirm-IEs}},

...

}

SNodeChangeConfirm-IEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

- -- S-NODE CHANGE REFUSE

--

-- **************************************************************

-- Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018.

-- ASN.1 for this message is FFS.

SNodeChangeRefuse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeChangeRefuse-IEs}},

...

}

SNodeChangeRefuse-IEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

- -- RRC TRANSFER

--

-- **************************************************************

-- Editor’s Note: Dual Connectivity is not complete and is targeted for completion in December 2018.

-- ASN.1 for this message is FFS.

RRCTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ RRCTransfer-IEs}},

...

}

RRCTransfer-IEs XNAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

- -- XN SETUP REQUEST

--

-- **************************************************************

XnSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ XnSetupRequest-IEs}},

...

}

XnSetupRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-GlobalNG-RAN-node-ID		CRITICALITY reject	TYPE GlobalNG-RANNode-ID			PRESENCE mandatory}|

{ ID id-TAISupport-list				CRITICALITY reject	TYPE TAISupport-List				PRESENCE optional }|

{ ID id-AMF-Pool-Information		CRITICALITY reject	TYPE AMF-Pool-Information			PRESENCE mandatory}|

{ ID id-List-of-served-cells-NR		CRITICALITY reject	TYPE List-of-served-cells-NR		PRESENCE optional }|

{ ID id-List-of-served-cells-E-UTRA	CRITICALITY reject	TYPE List-of-served-cells-E-UTRA	PRESENCE optional },

...

}

-- **************************************************************

--

- -- XN SETUP RESPONSE

--

-- **************************************************************

XnSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ XnSetupResponse-IEs}},

...

}

XnSetupResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-GlobalNG-RAN-node-ID		CRITICALITY reject	TYPE GlobalNG-RANNode-ID			PRESENCE mandatory}|

{ ID id-TAISupport-list				CRITICALITY reject	TYPE TAISupport-List				PRESENCE optional }|

{ ID id-List-of-served-cells-NR		CRITICALITY reject	TYPE List-of-served-cells-NR		PRESENCE optional }|

{ ID id-List-of-served-cells-E-UTRA	CRITICALITY reject	TYPE List-of-served-cells-E-UTRA	PRESENCE optional }|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional },

...

}

-- **************************************************************

--

- -- XN SETUP FAILURE

--

-- **************************************************************

XnSetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ XnSetupFailure-IEs}},

...

}

XnSetupFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional },

...

}

-- **************************************************************

--

- -- NG-RAN NODE CONFIGURATION UPDATE

--

-- **************************************************************

NGRANNodeConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ NGRANNodeConfigurationUpdate-IEs}},

...

}

NGRANNodeConfigurationUpdate-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-TAISupport-list							CRITICALITY reject	TYPE TAISupport-List									PRESENCE optional }|

{ ID id-ConfigurationUpdateInitiatingNodeChoice	CRITICALITY ignore	TYPE ConfigurationUpdateInitiatingNodeChoice 	PRESENCE mandatory},

...

}

ConfigurationUpdateInitiatingNodeChoice ::= CHOICE {

gNB									ProtocolIE-Container	{ {ConfigurationUpdate-gNB} },

ng-eNB								ProtocolIE-Container	{ {ConfigurationUpdate-ng-eNB} },

choice-extension					ProtocolExtensionContainer { {ServedCellsToUpdateInitiatingNodeChoice-ExtIEs} },

...

}

ServedCellsToUpdateInitiatingNodeChoice-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ConfigurationUpdate-gNB XNAP-PROTOCOL-IES ::= {

{ ID id-servedCellsToUpdate-NR		CRITICALITY ignore TYPE	List-of-served-cells-ToUpdate-NR		PRESENCE optional }|

{ ID id-cellAssistanceInfoNR		CRITICALITY ignore TYPE	CellAssistanceInfoNR					PRESENCE optional },

...

}

ConfigurationUpdate-ng-eNB XNAP-PROTOCOL-IES ::= {

{ ID id-servedCellsToUpdate-E-UTRA	CRITICALITY ignore TYPE	List-of-served-cells-ToUpdate-E-UTRA		PRESENCE optional },

...

}

-- **************************************************************

--

- -- NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE

--

-- **************************************************************

NGRANNodeConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ NGRANNodeConfigurationUpdateAcknowledge-IEs}},

...

}

NGRANNodeConfigurationUpdateAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional },

...

}

-- **************************************************************

--

- -- NG-RAN NODE CONFIGURATION UPDATE FAILURE

--

-- **************************************************************

NGRANNodeConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ NGRANNodeConfigurationUpdateFailure-IEs}},

...

}

NGRANNodeConfigurationUpdateFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional },

...

}

-- **************************************************************

--

- -- CELL ACTIVATION REQUEST

--

-- **************************************************************

CellActivationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ CellActivationRequest-IEs}},

...

}

CellActivationRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-ServedCellsToActivate					CRITICALITY reject		TYPE ServedCellsToActivate						PRESENCE mandatory}|

{ ID id-ActivationIDforCellActivation			CRITICALITY reject		TYPE ActivationIDforCellActivation			PRESENCE mandatory},

...

}

ServedCellsToActivate ::= CHOICE {

nr-cells							SEQUENCE (SIZE(1..maxnoofCellsinNGRANnode)) OF NR-CGI,

e-utra-cells						SEQUENCE (SIZE(1..maxnoofCellsinNGRANnode)) OF E-UTRA-CGI,

choice-extension					ProtocolExtensionContainer { {ServedCellsToActivate-ExtIEs} },

...

}

ServedCellsToActivate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

- -- CELL ACTIVATION RESPONSE

--

-- **************************************************************

CellActivationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{CellActivationResponse-IEs}},

...

}

CellActivationResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-ActivatedServedCells					CRITICALITY reject		TYPE ActivatedServedCells						PRESENCE mandatory}|

{ ID id-ActivationIDforCellActivation			CRITICALITY reject		TYPE ActivationIDforCellActivation			PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

ActivatedServedCells ::= CHOICE {

nr-cells							SEQUENCE (SIZE(1..maxnoofCellsinNGRANnode)) OF NR-CGI,

e-utra-cells						SEQUENCE (SIZE(1..maxnoofCellsinNGRANnode)) OF E-UTRA-CGI,

choice-extension					ProtocolExtensionContainer { {ActivatedServedCells-ExtIEs} },

...

}

ActivatedServedCells-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

- -- CELL ACTIVATION FAILURE

--

-- **************************************************************

CellActivationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{CellActivationFailure-IEs}},

...

}

CellActivationFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-ActivationIDforCellActivation			CRITICALITY reject		TYPE ActivationIDforCellActivation			PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

- -- RESET REQUEST

--

-- **************************************************************

ResetRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ResetRequest-IEs}},

...

}

ResetRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory},

...

}

-- **************************************************************

--

- -- RESET RESPONSE

--

-- **************************************************************

ResetResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ResetResponse-IEs}},

...

}

ResetResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

- -- ERROR INDICATION

--

-- **************************************************************

ErrorIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ErrorIndication-IEs}},

...

}

ErrorIndication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-oldNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE optional }|

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE optional }|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional },

...

}

-- **************************************************************

--

-- PRIVATE MESSAGE

--

-- **************************************************************

PrivateMessage ::= SEQUENCE {

privateIEs		PrivateIE-Container	{{PrivateMessage-IEs}},

...

}

PrivateMessage-IEs XNAP-PRIVATE-IES ::= {

...

}

END

### 9.3.5	Information Element definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AreaOfInterest-Item,

id-DataForwardingResponseDRBItem,

id-DRBsSubjectToStatusTransfer-Item,

id-DRBToQoSFlowMapping-Item,

id-PDUSessionResourceAdmittedResponseTransferItem,

id-PDUSessionResourcesAdmitted-Item,

id-PDUSessionResourcesNotAdmitted-Item,

id-PDUSessionResourcesToBeSetup-Item,

id-QoSFlowAdmitted-Item,

id-QoSFlow-Item,

id-QoSFlowNotAdmitted-Item,

id-QoSFlowsToBeSetup-Item,

id-TAISupport-Item,

maxnoofAoIs,

maxnoofBPLMNs,

maxnoofCellsinNGRANnode,

maxnoofCellsinRNA,

maxnoofDRBs,

maxnoofNRCellBands,

maxnoofPDUSessions,

maxnoofQoSFlows,

maxnoofRANAreaCodes,

maxnoofRANAreasinRNA,

maxnoofSliceItems,

maxnoofsupportedTACs,

maxnoofTAI,

maxnoofTAIinAoI,

maxNRARFCN,

maxNrOfErrors

FROM XnAP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM XnAP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-Single-Container{},

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES

FROM XnAP-Containers;

- -- A

- ActivationIDforCellActivation	::= INTEGER (0..255)

- AllocationandRetentionPriority ::= SEQUENCE {

- priorityLevel					INTEGER (0..15,...),

- pre-emption-capability			ENUMERATED {shall-not-trigger-preemptdatDion, may-trigger-preemption, ...},

- pre-emption-vulnerability		ENUMERATED {not-preemptable, preemptable, ...},

iE-Extensions					ProtocolExtensionContainer { {AllocationandRetentionPriority-ExtIEs} } OPTIONAL,

...

}

AllocationandRetentionPriority-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AMF-Pool-Information ::= OCTET STRING -- FFS

AreaOfInterest ::= SEQUENCE (SIZE(1..maxnoofAoIs)) OF ProtocolIE-Single-Container { {AreaOfInterest-ItemIEs} }

AreaOfInterest-ItemIEs 	XNAP-PROTOCOL-IES ::= {

{ ID id-AreaOfInterest-Item	 CRITICALITY ignore 	TYPE AreaOfInterest-Item 	PRESENCE mandatory },

...

}

AreaOfInterest-Item ::= SEQUENCE {

referenceID						ReferenceID,

listOfTAI						ListOfTAI,

iE-Extensions					ProtocolExtensionContainer { {AreaOfInterest-Item-ExtIEs} } OPTIONAL,

...

}

AreaOfInterest-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AssistanceDataForPaging ::= OCTET STRING

- -- B

BitRate	::= INTEGER (0..4000000000000,...)

BroadcastPLMNs ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF PLMN-Identity

BroadcastPLMNinTAISupport-Item ::= SEQUENCE {

plmn-id							PLMN-Identity,

tAISliceSupport-List			SliceSupport-List,

iE-Extension					ProtocolExtensionContainer { {BroadcastPLMNinTAISupport-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastPLMNinTAISupport-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

- -- C

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

transport			CauseTransport,

protocol			CauseProtocol,

misc				CauseMisc,

...

}

CauseRadioNetwork ::= ENUMERATED {

unspecified,

up-integrity-protection-not-possible,

up-confidentiality-protection-not-possible,

...

}

CauseTransport ::= ENUMERATED {

unspecified,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

abstract-syntax-error-falsely-constructed-message,

unspecified,

...

}

CauseMisc ::= ENUMERATED {

unspecified,

...

}

CellAssistanceInfoNR	::= CHOICE {

limitedNR-List				SEQUENCE (SIZE(1..maxnoofCellsinNGRANnode)) OF NR-CGI,

full-List					ENUMERATED {all-served-cells-NR, ...},

choice-extension			ProtocolExtensionContainer { {CellAssistanceInfoNR-ExtIEs} },

...

}

CellAssistanceInfoNR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPTransportLayerInformation ::= CHOICE {

endpointIPAddress			TransportLayerAddress,

choice-extension			ProtocolExtensionContainer { {CPTransportLayerInformation-ExtIEs} },

...

}

CPTransportLayerInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics ::= SEQUENCE {

procedureCode					ProcedureCode					OPTIONAL,

triggeringMessage				TriggeringMessage				OPTIONAL,

procedureCriticality			Criticality						OPTIONAL,

iEsCriticalityDiagnostics		CriticalityDiagnostics-IE-List	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {CriticalityDiagnostics-ExtIEs} }	OPTIONAL,

...

}

CriticalityDiagnostics-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics-IE-List ::= SEQUENCE (SIZE (1..maxNrOfErrors)) OF

SEQUENCE {

iECriticality			Criticality,

iE-ID					ProtocolIE-ID,

typeOfError				TypeOfError,

iE-Extensions			ProtocolExtensionContainer { {CriticalityDiagnostics-IE-List-ExtIEs} } OPTIONAL,

...

}

CriticalityDiagnostics-IE-List-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

- -- D

DataForwardingAccepted	::= ENUMERATED {data-forwarding-accepted, ...}

DataForwardingInfoFromTargetNGRANnode ::= SEQUENCE {

pduSessionLevelDLDataForwardingInfo				UPTransportLayerInformation,

dataForwardingResponseDRBItemList				DataForwardingResponseDRBItemList						OPTIONAL,

- iE-Extension		ProtocolExtensionContainer { {DataForwardingInfoFromTargetNGRANnode-ExtIEs} }	OPTIONAL,

- ...

- }

DataForwardingInfoFromTargetNGRANnode-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DataForwardingResponseDRBItemList ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-Single-Container { {DataForwardingResponseDRBItem-IEs} }

DataForwardingResponseDRBItem-IEs 	XNAP-PROTOCOL-IES ::= {

{ ID id-DataForwardingResponseDRBItem	 CRITICALITY reject 	TYPE DataForwardingResponseDRBItem 	PRESENCE mandatory },

...

}

DataForwardingResponseDRBItem ::= SEQUENCE {

drb-ID				DRB-ID,

dlForwardingUPTNL	UPTransportLayerInformation												OPTIONAL,

ulForwardingUPTNL	UPTransportLayerInformation												OPTIONAL,

- iE-Extension		ProtocolExtensionContainer { {DataForwardingResponseDRBItem-ExtIEs} }	OPTIONAL,

- ...

- }

DataForwardingResponseDRBItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DeliveryStatus	::= INTEGER (0..4095, ...)

DLForwarding	::= ENUMERATED {dl-forwarding-proposed, ...}

DRB-ID	::= INTEGER (1..32, ...)

DRBsSubjectToStatusTransfer-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF ProtocolIE-Single-Container { {DRBsSubjectToStatusTransfer-ItemIEs} }

DRBsSubjectToStatusTransfer-ItemIEs 	XNAP-PROTOCOL-IES ::= {

{ ID id-DRBsSubjectToStatusTransfer-Item	 CRITICALITY ignore 	TYPE DRBsSubjectToStatusTransfer-Item 	PRESENCE mandatory },

...

}

DRBsSubjectToStatusTransfer-Item ::= SEQUENCE {

drbID				DRB-ID,

receiveStatusUL-PDCP		BIT STRING		OPTIONAL, 	-- FFS

ul-COUNT-Value				BIT STRING,				-- FFS

dl-COUNT-Value				BIT STRING,				-- FFS

- iE-Extension		ProtocolExtensionContainer { {DRBsSubjectToStatusTransfer-Item-ExtIEs} }	OPTIONAL,

- ...

- }

DRBsSubjectToStatusTransfer-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBToQoSFlowMapping-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF ProtocolIE-Single-Container { {DRBToQoSFlowMapping-ItemIEs} }

DRBToQoSFlowMapping-ItemIEs 	XNAP-PROTOCOL-IES ::= {

{ ID id-DRBToQoSFlowMapping-Item	 CRITICALITY ignore 	TYPE DRBToQoSFlowMapping-Item 	PRESENCE mandatory },

...

}

DRBToQoSFlowMapping-Item ::= SEQUENCE {

drbID				DRB-ID,

qosFlows-List		QoSFlows-List,

- iE-Extension		ProtocolExtensionContainer { {DRBToQoSFlowMapping-Item-ExtIEs} }	OPTIONAL,

- ...

- }

DRBToQoSFlowMapping-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Dynamic5QIDescriptor ::= SEQUENCE {

priorityLevel				OCTET STRING, -- FFS

packetDelayBudget			OCTET STRING, -- FFS

packetErrorRate				OCTET STRING, -- FFS

delayCritical				ENUMERATED {delay-critical, non-delay-critical}	OPTIONAL,

-- This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE.

averagingWindow				OCTET STRING	OPTIONAL, -- FFS

-- This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE.

- maximumDataBurstVolumne		OCTET STRING, -- FFS

- iE-Extension		ProtocolExtensionContainer { {Dynamic5QIDescriptor-ExtIEs } }	OPTIONAL,

- ...

- }

Dynamic5QIDescriptor-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

- -- E

- E-RAB-ID		::= INTEGER (0..15, ...)

- E-UTRA-Cell-Identity			::= BIT STRING (SIZE(28))

- E-UTRA-CGI ::= SEQUENCE {

- plmn-id				PLMN-Identity,

- e-utra-CI			E-UTRA-Cell-Identity,

- iE-Extension		ProtocolExtensionContainer { {E-UTRA-CGI-ExtIEs} } 	OPTIONAL,

- ...

- }

E-UTRA-CGI-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

EventType ::= ENUMERATED {

direct,

change-of-serving-cell,

ue-presence-in-AoI,

stop-change-of-serving-cell,

stop-ue-presence-in-AoI,

...

}

- -- F

- -- G

GBRQoSFlowInfo ::= SEQUENCE {

maxFlowBitRateDL			BitRate,

maxFlowBitRateUL			BitRate,

guaranteedFlowBitRateDL		BitRate,

guaranteedFlowBitRateUL		BitRate,

notificationControl			ENUMERATED {notification-requested, ...}	OPTIONAL,

maxPacketLossRateDL			OCTET STRING								OPTIONAL, -- FFS

maxPacketLossRateUL			OCTET STRING								OPTIONAL, -- FFS

iE-Extensions				ProtocolExtensionContainer { {GBRQoSFlowInfo-ExtIEs} } OPTIONAL,

...

}

GBRQoSFlowInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

GlobalgNB-ID	::= SEQUENCE {

plmn-id			PLMN-Identity,

gnb-id			GNB-ID-Choice,

iE-Extensions		ProtocolExtensionContainer { {GlobalgNB-ID-ExtIEs} } OPTIONAL,

...

}

GlobalgNB-ID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

GNB-ID-Choice ::= CHOICE {

gnb-ID					BIT STRING (SIZE(22..32)),

choice-extension		ProtocolExtensionContainer { {GNB-ID-Choice-ExtIEs} },

...

}

GNB-ID-Choice-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

GlobalngeNB-ID	::= SEQUENCE {

plmn-id			PLMN-Identity,

enb-id			ENB-ID-Choice,

iE-Extensions		ProtocolExtensionContainer { {GlobaleNB-ID-ExtIEs} } OPTIONAL,

...

}

GlobaleNB-ID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ENB-ID-Choice ::= CHOICE {

enb-ID-macro			BIT STRING (SIZE(20)),

enb-ID-shortmacro		BIT STRING (SIZE(18)),

enb-ID-longmacro		BIT STRING (SIZE(21)),

choice-extension		ProtocolExtensionContainer { {ENB-ID-Choice-ExtIEs} },

...

}

ENB-ID-Choice-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

GlobalNG-RANNode-ID ::= CHOICE {

gNB						GlobalgNB-ID,

ng-eNB					GlobalngeNB-ID,

choice-extension		ProtocolExtensionContainer { {GlobalNG-RANNode-ID-ExtIEs} },

...

}

GlobalNG-RANNode-ID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

GTP-TEID	::= OCTET STRING (SIZE(4))

GTPtunnelTransportLayerInformation ::= SEQUENCE {

tnl-address			TransportLayerAddress,

gtp-teid			GTP-TEID,

iE-Extensions		ProtocolExtensionContainer { {GTPtunnelTransportLayerInformation-ExtIEs} } OPTIONAL,

...

}

GTPtunnelTransportLayerInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

GUAMI ::= SEQUENCE {

plmn-ID				PLMN-Identity,

amf-region-if		OCTET STRING (SIZE (2)),

amf-set-id			BIT STRING (SIZE (4)),

amf-pointer			BIT STRING (SIZE (4)),

iE-Extensions		ProtocolExtensionContainer { {GUAMI-ExtIEs} } OPTIONAL,

...

}

GUAMI-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

- -- H

- -- I

I-RNTI ::= BIT STRING (SIZE(40)) -- FFS

- -- J

- -- K

- -- L

List-of-served-cells-E-UTRA ::= SEQUENCE (SIZE (1..maxnoofCellsinNGRANnode)) OF Served-cells-E-UTRA-Item

Served-cells-E-UTRA-Item ::= SEQUENCE {

served-cell-info-E-UTRA		OCTET STRING, -- FFS

neighbour-info-NR			OCTET STRING 				OPTIONAL, -- FFS

neighbour-info-E-UTRA		OCTET STRING				OPTIONAL, -- FFS

iE-Extensions		ProtocolExtensionContainer { {Served-cells-E-UTRA-Item-ExtIEs} } OPTIONAL,

...

}

Served-cells-E-UTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

List-of-served-cells-NR ::= SEQUENCE (SIZE (1..maxnoofCellsinNGRANnode)) OF Served-cells-NR-Item

Served-cells-NR-Item ::= SEQUENCE {

served-cell-info-NR			ServedCellInformationNR,

neighbour-info-NR			OCTET STRING 				OPTIONAL, -- FFS

neighbour-info-E-UTRA		OCTET STRING				OPTIONAL, -- FFS

iE-Extensions		ProtocolExtensionContainer { {Served-cells-NR-Item-ExtIEs} } OPTIONAL,

...

}

Served-cells-NR-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

List-of-served-cells-NR-ToModify ::= SEQUENCE (SIZE (1..maxnoofCellsinNGRANnode)) OF Served-cells-NR-ToModify-Item

Served-cells-NR-ToModify-Item ::= SEQUENCE {

old-NR-CGI					NR-CGI,

served-cell-info-NR			ServedCellInformationNR,

neighbour-info-NR			OCTET STRING 				OPTIONAL, -- FFS

neighbour-info-E-UTRA		OCTET STRING				OPTIONAL, -- FFS

iE-Extensions		ProtocolExtensionContainer { {Served-cells-NR-ToModify-Item-ExtIEs} } OPTIONAL,

...

}

Served-cells-NR-ToModify-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

List-of-served-cells-ToUpdate-E-UTRA ::= SEQUENCE {

served-Cells-E-UTRA-ToAdd		OCTET STRING 																	OPTIONAL, -- FFS

served-Cells-E-UTRA-ToModify	OCTET STRING 																	OPTIONAL, -- FFS

served-Cells-E-UTRA-ToDelete	SEQUENCE (SIZE (1..maxnoofCellsinNGRANnode)) OF E-UTRA-CGI 						OPTIONAL, -- FFS

iE-Extensions					ProtocolExtensionContainer { {List-of-served-cells-ToUpdate-E-UTRA-ExtIEs} }	OPTIONAL,

...

}

List-of-served-cells-ToUpdate-E-UTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

List-of-served-cells-ToUpdate-NR ::= SEQUENCE {

served-Cells-NR-ToAdd		List-of-served-cells-NR															OPTIONAL,

served-Cells-NR-ToModify	List-of-served-cells-NR-ToModify					OPTIONAL, -- FFS

served-Cells-NR-ToDelete	SEQUENCE (SIZE (1..maxnoofCellsinNGRANnode)) OF NR-CGI 						OPTIONAL, -- FFS

iE-Extensions		ProtocolExtensionContainer { {List-of-served-cells-ToUpdate-NR-ExtIEs} } OPTIONAL,

...

}

List-of-served-cells-ToUpdate-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ListOfTAI::= SEQUENCE (SIZE(1..maxnoofTAI)) OF TAI-Item

TAI-Item ::= SEQUENCE {

pLMN-Identity		PLMN-Identity,

tAC					TAC,

iE-Extensions		ProtocolExtensionContainer { {TAI-Item-ExtIEs} } OPTIONAL,

...

}

TAI-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ListOfTAIinAoI ::= SEQUENCE (SIZE(1..maxnoofTAIinAoI)) OF TAIinAoI-Item

TAIinAoI-Item ::= SEQUENCE {

pLMN-Identity		PLMN-Identity,

tAC					TAC,

iE-Extensions		ProtocolExtensionContainer { {TAIinAoI-Item-ExtIEs} } OPTIONAL,

...

}

TAIinAoI-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LocationReportingInformation ::= SEQUENCE {

eventType			EventType,

reportArea			ReportArea,

areaOfInterest		AreaOfInterest	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LocationReportingInformation-ExtIEs} } OPTIONAL,

...

}

LocationReportingInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

- -- M

MAC-I ::= BIT STRING (SIZE(16))

- MaskedIMEISV	::= BIT STRING (SIZE(64))

- -- N

- NG-RAN-Cell-Identity ::= CHOICE {

- nr		NR-Cell-Identity,

- e-utra	E-UTRA-Cell-Identity,

- ext		ProtocolExtensionContainer { {NG-RAN-Cell-Identity-ExtIEs} },

- ...

- }

NG-RAN-Cell-Identity-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

- NG-RANnodeUEXnAPID ::= INTEGER (0.. 4294967295)

NonDynamic5QIDescriptor ::= SEQUENCE {

fiveQI						INTEGER (0..255),

priorityLevel				OCTET STRING	OPTIONAL, -- FFS

averagingWindow				OCTET STRING	OPTIONAL, -- FFS

- maximumDataBurstVolumne		OCTET STRING	OPTIONAL, -- FFS

- iE-Extension				ProtocolExtensionContainer { {NonDynamic5QIDescriptor-ExtIEs } }	OPTIONAL,

- ...

- }

NonDynamic5QIDescriptor-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

- NRARFCN	::= INTEGER (0.. maxNRARFCN)

- NR-Cell-Identity		::= BIT STRING (SIZE (36))

NR-Cell-Identity-ListinRANPagingArea ::= SEQUENCE (SIZE (1..maxnoofCellsinRNA)) OF NR-Cell-Identity

- NR-CGI ::= SEQUENCE {

- plmn-id				PLMN-Identity,

- nr-CI				NR-Cell-Identity,

- iE-Extension		ProtocolExtensionContainer { {NR-CGI-ExtIEs} } 	OPTIONAL,

- ...

- }

NR-CGI-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NRFrequencyBand ::= INTEGER (1..1024, ...)

NRFrequencyBand-List ::= SEQUENCE (SIZE(1..maxnoofNRCellBands)) OF NRFrequencyBand

NRFrequencyInfo ::= SEQUENCE {

nrARFCN				NRARFCN,

frequencyBand-List	NRFrequencyBand-List,

- iE-Extension		ProtocolExtensionContainer { {NRFrequencyInfo-ExtIEs} } 	OPTIONAL,

- ...

- }

NRFrequencyInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NRModeInfo ::= CHOICE {

fdd							NRModeInfoFDD,

tdd							NRModeInfoTDD,

- choice-extension			ProtocolExtensionContainer { {NRModenInfor-ExtIEs} },

- ...

- }

- NRModeInfoFDD ::= SEQUENCE {

- ulNRFrequencyInfo			NRFrequencyInfo,

- dlNRFrequencyInfo			NRFrequencyInfo,

- ulNRTransmissonBandwidth	NRTransmissionBandwidth,

- dlNRTransmissonBandwidth	NRTransmissionBandwidth,

- iE-Extension		ProtocolExtensionContainer { {NRModeInfoFDD-ExtIEs} } 	OPTIONAL,

- ...

- }

NRModeInfoFDD-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

- NRModeInfoTDD ::= SEQUENCE {

- nrFrequencyInfo			NRFrequencyInfo,

- nrTransmissonBandwidth	NRTransmissionBandwidth,

- iE-Extension			ProtocolExtensionContainer { {NRModeInfoTDD-ExtIEs} } 	OPTIONAL,

- ...

- }

NRModeInfoTDD-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NRModenInfor-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NRPCI ::= INTEGER (0..1007, ...)

NRTransmissionBandwidth	::= INTEGER (0..65535, ...)

- -- O

- -- P

- PagingDRX	::= OCTET STRING -- FFS

PDUSessionResourcesAdmitted-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions))OF ProtocolIE-Single-Container { {PDUSessionResourcesAdmitted-ItemIEs} }

PDUSessionResourcesAdmitted-ItemIEs 	XNAP-PROTOCOL-IES ::= {

{ ID id-PDUSessionResourcesAdmitted-Item	 CRITICALITY reject 	TYPE PDUSessionResourcesAdmitted-Item 	PRESENCE mandatory },

...

}

PDUSessionResourcesAdmitted-Item ::= SEQUENCE {

pduSessionId									PDUSession-ID,

pduSessionResourceAdmittedResponseTransfer		PDUSessionResourceAdmittedResponseTransfer		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourcesAdmitted-Item-ExtIEs} } OPTIONAL,

...

}

PDUSessionResourcesAdmitted-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceAdmittedResponseTransfer ::= ProtocolIE-Single-Container { {PDUSessionResourceAdmittedResponseTransferIEs} }

PDUSessionResourceAdmittedResponseTransferIEs 	XNAP-PROTOCOL-IES ::= {

{ ID id-PDUSessionResourceAdmittedResponseTransferItem	 CRITICALITY reject 	TYPE PDUSessionResourceAdmittedResponseTransferItem 	PRESENCE mandatory },

...

}

PDUSessionResourceAdmittedResponseTransferItem ::= SEQUENCE {

qosFlowsAdmitted-List			QoSFlowsAdmitted-List							OPTIONAL,

qosFlowsNotAdmitted-List		QoSFlowsNotAdmitted-List						OPTIONAL,

dataForwardingInfoFromTarget	DataForwardingInfoFromTargetNGRANnode			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceAdmittedResponseTransferItem-ExtIEs} } OPTIONAL,

...

}

PDUSessionResourceAdmittedResponseTransferItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourcesNotAdmitted-List ::= SEQUENCE (SIZE (1..maxnoofPDUSessions)) OF ProtocolIE-Single-Container { {PDUSessionResourcesNotAdmitted-ItemIEs} }

PDUSessionResourcesNotAdmitted-ItemIEs 	XNAP-PROTOCOL-IES ::= {

{ ID id-PDUSessionResourcesNotAdmitted-Item	 CRITICALITY reject 	TYPE PDUSessionResourcesNotAdmitted-Item 	PRESENCE mandatory },

...

}

PDUSessionResourcesNotAdmitted-Item ::= SEQUENCE {

pduSessionId				PDUSession-ID,

cause						Cause				OPTIONAL,

- iE-Extension		ProtocolExtensionContainer { {PDUSessionResourcesNotAdmitted-Item-Item-ExtIEs} }	OPTIONAL,

- ...

- }

PDUSessionResourcesNotAdmitted-Item-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourcesToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions))OF ProtocolIE-Single-Container { {PDUSessionResourcesToBeSetup-ItemIEs} }

PDUSessionResourcesToBeSetup-ItemIEs 	XNAP-PROTOCOL-IES ::= {

{ ID id-PDUSessionResourcesToBeSetup-Item	 CRITICALITY reject 	TYPE PDUSessionResourcesToBeSetup-Item 	PRESENCE mandatory },

...

}

PDUSessionResourcesToBeSetup-Item ::= SEQUENCE {

pduSessionId					PDUSession-ID,

s-NSSAI							S-NSSAI,

pduSessionAMBR					OCTET STRING, -- FFS

uL-NG-U-TNLatUPF				UPTransportLayerInformation,

securityIndication				SecurityIndication						OPTIONAL,

pduSessionType					PDUSessionType,

qosFlowsToBeSetup-List			QoSFlowsToBeSetup-List,

sourceDRBtoQoSFlowMapping		DRBToQoSFlowMapping-List			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourcesToBeSetup-Item-ExtIEs} } OPTIONAL,

...

}

PDUSessionResourcesToBeSetup-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

- PDUSessionType ::= ENUMERATED {ipv4, ipv6, ipv4v6, ethernet, unstructured, ...}

- PDUSession-ID	::= INTEGER (0..255)

PLMN-Identity ::= OCTET STRING (SIZE(3))

- -- Q

- QoSCharacteristics ::= CHOICE {

- non-dynamic						NonDynamic5QIDescriptor,

- dynamic							Dynamic5QIDescriptor,

choice-extension				ProtocolExtensionContainer { {QoSCharacteristics-ExtIEs} },

...

}

QoSCharacteristics-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlowIndicator	::= INTEGER (0..63, ...)

QoSFlowLevelQoSParameters ::= SEQUENCE {

qos-characteristics			QoSCharacteristics,

allocationAndRetentionPrio	AllocationandRetentionPriority,

gBRQoSFlowInfo				GBRQoSFlowInfo,

relectiveQoS				ReflectiveQoSAttribute,

iE-Extensions				ProtocolExtensionContainer { {QoSFlowLevelQoSParameters-ExtIEs} } OPTIONAL,

...

}

QoSFlowLevelQoSParameters-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlows-List ::= SEQUENCE (SIZE (1..maxnoofQoSFlows)) OF ProtocolIE-Single-Container { {QoSFlow-ItemIEs} }

QoSFlow-ItemIEs 	XNAP-PROTOCOL-IES ::= {

{ ID id-QoSFlow-Item	 CRITICALITY reject 	TYPE QoSFlow-Item 	PRESENCE mandatory },

...

}

QoSFlow-Item ::= SEQUENCE {

qfi					QoSFlowIndicator,

cause				Cause				OPTIONAL,

- iE-Extension		ProtocolExtensionContainer { {QoSFlow-Item-ExtIEs} }	OPTIONAL,

- ...

- }

QoSFlow-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlowsAdmitted-List ::= SEQUENCE (SIZE (1..maxnoofQoSFlows)) OF ProtocolIE-Single-Container { {QoSFlowAdmitted-ItemIEs} }

QoSFlowAdmitted-ItemIEs 	XNAP-PROTOCOL-IES ::= {

{ ID id-QoSFlowAdmitted-Item	 CRITICALITY reject 	TYPE QoSFlowAdmitted-Item 	PRESENCE mandatory },

...

}

QoSFlowAdmitted-Item ::= SEQUENCE {

qfi								QoSFlowIndicator,

dataForwardingAccepted			DataForwardingAccepted	OPTIONAL,

- iE-Extension		ProtocolExtensionContainer { {QoSFlowAdmitted-Item-ExtIEs} }	OPTIONAL,

- ...

- }

QoSFlowAdmitted-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlowsNotAdmitted-List ::= SEQUENCE (SIZE (1..maxnoofQoSFlows)) OF ProtocolIE-Single-Container { {QoSFlowNotAdmitted-ItemIEs} }

QoSFlowNotAdmitted-ItemIEs 	XNAP-PROTOCOL-IES ::= {

{ ID id-QoSFlowNotAdmitted-Item	 CRITICALITY reject 	TYPE QoSFlowNotAdmitted-Item 	PRESENCE mandatory },

...

}

QoSFlowNotAdmitted-Item ::= SEQUENCE {

qfi							QoSFlowIndicator,

cause						Cause,

- iE-Extension		ProtocolExtensionContainer { {QoSFlowNotAdmitted-Item-ExtIEs} }	OPTIONAL,

- ...

- }

QoSFlowNotAdmitted-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlowsToBeSetup-List ::= SEQUENCE (SIZE (1..maxnoofQoSFlows)) OF ProtocolIE-Single-Container { {QoSFlowsToBeSetup-ItemIEs} }

QoSFlowsToBeSetup-ItemIEs 	XNAP-PROTOCOL-IES ::= {

{ ID id-QoSFlowsToBeSetup-Item	 CRITICALITY reject 	TYPE QoSFlowsToBeSetup-Item 	PRESENCE mandatory },

...

}

QoSFlowsToBeSetup-Item ::= SEQUENCE {

qfi								QoSFlowIndicator,

dlDataForwarding				DLForwarding												OPTIONAL,

qosFlowLevelQoSParameters		QoSFlowLevelQoSParameters,

e-RAB-ID						E-RAB-ID													OPTIONAL,

cause						Cause,

- iE-Extension		ProtocolExtensionContainer { {QoSFlowsToBeSetup-Item-ExtIEs} }	OPTIONAL,

- ...

- }

QoSFlowsToBeSetup-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

- -- R

RANAC ::= INTEGER (0..63)	-- FFS

RANAreaID ::= SEQUENCE {

tAC			TAC,

rANAC-List	RANAC-List	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {RANAreaID-ExtIEs} } OPTIONAL,

...

}

RANAreaID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RANAreaID-List ::= SEQUENCE (SIZE(1..maxnoofRANAreasinRNA)) OF RANAreaID

RANAC-List ::= SEQUENCE (SIZE(1..maxnoofRANAreaCodes)) OF RANAC-Item

RANAC-Item ::= SEQUENCE {

rANAC		RANAC,

iE-Extensions		ProtocolExtensionContainer { {RANAC-Item-ExtIEs} } OPTIONAL,

...

}

RANAC-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RANPagingArea ::= SEQUENCE {

pLMN-Identity			PLMN-Identity,

rANPagingAreaChoice		RANPagingAreaChoice,

iE-Extensions			ProtocolExtensionContainer { {RANPagingArea-ExtIEs} } OPTIONAL,

...

}

RANPagingArea-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RANPagingAreaChoice ::= CHOICE {

cell-List			NR-Cell-Identity-ListinRANPagingArea,

rANAreaID-List		RANAreaID-List,

choice-ext			ProtocolExtensionContainer { {RANPagingAreaChoice-ExtIEs} },

...

}

RANPagingAreaChoice-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RANPagingPriority ::= INTEGER (1..256)

ReferenceID ::= INTEGER (1..64, ...) -- FFS

ReflectiveQoSAttribute ::= ENUMERATED {subject-to-reflective-QoS, ...}

ReportArea ::= ENUMERATED {

cell,

...

}

RLCMode	::= ENUMERATED {rlc-am, rlc-um}

RSFP-Index ::= INTEGER (1..256)

- -- S

- SCGConfigurationQuery	::= ENUMERATED {true, ...}

- SecurityIndication ::= SEQUENCE {

- integrityProtectionIndication			ENUMERATED {required, preferred, not-needed, ...},

- confidentialityProtectionIndication		ENUMERATED {required, preferred, not-needed, ...},

iE-Extensions							ProtocolExtensionContainer { {SecurityIndication-ExtIEs} } OPTIONAL,

...

}

SecurityIndication-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ServedCellInformationNR ::= SEQUENCE {

nrPCI					NRPCI,

cellID					NR-Cell-Identity,

tac						TAC,

broadcastPLMN			BroadcastPLMNs,

nrModeInfo				NRModeInfo,

sulInfo					SUL-Information,

iE-Extensions			ProtocolExtensionContainer { {ServedCellInformationNR-ExtIEs} } OPTIONAL,

...

}

ServedCellInformationNR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

- SliceSupport-List	::= SEQUENCE (SIZE(1..maxnoofSliceItems)) OF S-NSSAI

S-NSSAI ::= SEQUENCE {

sst						OCTET STRING (SIZE(1)),

sd						OCTET STRING (SIZE(3)),

iE-Extensions			ProtocolExtensionContainer { {S-NSSAI-ExtIEs} } OPTIONAL,

...

}

S-NSSAI-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SUL-Information ::= SEQUENCE {

sulFrequencyInfo			NRFrequencyInfo,

sulTransmissionBandwidth	NRTransmissionBandwidth,

iE-Extensions				ProtocolExtensionContainer { {SUL-Information-ExtIEs} } OPTIONAL,

...

}

SUL-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

- -- T

TAC ::= OCTET STRING (SIZE (3))

TAISupport-List	::= SEQUENCE (SIZE(1..maxnoofsupportedTACs)) OF ProtocolIE-Single-Container { {TAISupport-ItemIEs} }

TAISupport-ItemIEs 	XNAP-PROTOCOL-IES ::= {

{ ID id-TAISupport-Item	 CRITICALITY reject 	TYPE TAISupport-Item 	PRESENCE mandatory },

...

}

TAISupport-Item ::= SEQUENCE {

tac								TAC,

broadcastPLMN					BroadcastPLMNinTAISupport-Item,

iE-Extensions					ProtocolExtensionContainer { {TAISupport-Item-ExtIEs} } OPTIONAL,

...

}

TAISupport-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Target-CGI ::= CHOICE {

nr							NR-CGI,

e-utra						E-UTRA-CGI,

choice-extension			ProtocolExtensionContainer { {TargetCGI-ExtIEs} },

...

}

TargetCGI-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TransportLayerAddress ::= BIT STRING (SIZE(1..160, ...))

TraceActivation ::= SEQUENCE {

ng-ran-TraceID			OCTET STRING (SIZE (8)),

interfaces-to-trace BIT STRING { ng-c (0), x-nc (1), uu (2), f1-c (3), e1 (4)} (SIZE(8)),

trace-depth 		Trace-Depth,

trace-coll-address	TransportLayerAddress,

ie-Extension 	ProtocolExtensionContainer { {TraceActivation-ExtIEs} } OPTIONAL,

...

}

TraceActivation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Trace-Depth ::= ENUMERATED {

minimum,

medium,

maximum,

minimumWithoutVendorSpecificExtension,

mediumWithoutVendorSpecificExtension,

maximumWithoutVendorSpecificExtension,

...

}

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

- -- U

- UEAggregateMaximumBitRate ::= SEQUENCE {

- dl	BitRate,

- ul	BitRate,

iE-Extension			ProtocolExtensionContainer { {UEAggregateMaximumBitRate-ExtIEs} } OPTIONAL,

...

}

UEAggregateMaximumBitRate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

- UEContextInfoRetrUECtxtResp ::= SEQUENCE {

- ng-c-UE-signalling-ref				OCTET STRING, -- FFS

- signalling-TNL-at-source			CPTransportLayerInformation,

- securityInformation					OCTET STRING, -- FFS

ue-AMBR								UEAggregateMaximumBitRate,

pduSessionResourcesToBeSet-List		PDUSessionResourcesToBeSetup-List,

rrc-Context							OCTET STRING,

hlr									OCTET STRING					OPTIONAL,  -- FFS

iE-Extension			ProtocolExtensionContainer { {UEContextInfoRetrUECtxtResp-ExtIEs} } OPTIONAL,

...

}

UEContextInfoRetrUECtxtResp-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

- UEIdentityIndexValue ::= OCTET STRING -- FFS

- UEPagingIdentity ::= OCTET STRING -- FFS

UPTransportLayerInformation ::= CHOICE {

gtpTunnel					GTPtunnelTransportLayerInformation,

choice-extension			ProtocolExtensionContainer { {UPTransportLayerInformation-ExtIEs} },

...

}

UPTransportLayerInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

- -- V

- -- W

- -- X

- -- Y

- -- Z

END

### 9.3.6	Common definitions

-- **************************************************************

--

-- Common definitions

--

-- **************************************************************

XnAP-CommonDataTypes {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-CommonDataTypes (3) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- Extension constants

--

-- **************************************************************

maxPrivateIEs 									INTEGER ::= 65535

maxProtocolExtensions 							INTEGER ::= 65535

maxProtocolIEs									INTEGER ::= 65535

-- **************************************************************

--

-- Common Data Types

--

-- **************************************************************

Criticality		::= ENUMERATED { reject, ignore, notify }

Presence		::= ENUMERATED { optional, conditional, mandatory }

PrivateIE-ID	::= CHOICE {

local				INTEGER (0.. maxPrivateIEs),

global				OBJECT IDENTIFIER

}

ProcedureCode		::= INTEGER (0..255)

ProtocolIE-ID		::= INTEGER (0..maxProtocolIEs)

TriggeringMessage	::= ENUMERATED { initiating-message, successful-outcome, unsuccessful-outcome}

END

### 9.3.7	Constant definitions

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

-- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-sNStatusTransfer												ProcedureCode ::= 1

id-handoverCancel												ProcedureCode ::= 2

id-retrieveUEContext											ProcedureCode ::= 3

id-rANPaging													ProcedureCode ::= 4

id-dataForwardingAddressIndication								ProcedureCode ::= 5

id-uEContextRelease												ProcedureCode ::= 6

id-sNGRANnodeAdditionPreparation								ProcedureCode ::= 7

id-sNGRANnodeReconfigurationCompletion							ProcedureCode ::= 8

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 9

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 10

id-mNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 11

id-sNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 12

id-sNGRANnodeCounterCheck										ProcedureCode ::= 13

id-sNGRANnodeChange												ProcedureCode ::= 14

id-rRCTransfer													ProcedureCode ::= 15

id-xnSetup														ProcedureCode ::= 16

id-nGRANnodeConfigurationUpdate									ProcedureCode ::= 17

id-cellActivation												ProcedureCode ::= 18

id-reset														ProcedureCode ::= 19

id-errorIndication												ProcedureCode ::= 20

id-privateMessage												ProcedureCode ::= 21

-- **************************************************************

--

-- Lists

--

-- **************************************************************

maxnoofAoIs									INTEGER ::= 32   -- FFS

maxnoofBPLMNs								INTEGER ::= 6	 -- FFS

maxnoofCellsinNGRANnode						INTEGER ::= 16384

maxnoofCellsinRNA							INTEGER ::= 32

maxnoofDRBs									INTEGER ::= 32

maxnoofNRCellBands							INTEGER ::= 32

maxnoofPDUSessions							INTEGER ::= 256

maxnoofQoSFlows								INTEGER ::= 64

maxnoofRANAreaCodes							INTEGER ::= 32

maxnoofRANAreasinRNA						INTEGER ::= 16

maxnoofSliceItems							INTEGER ::= 256  -- FFS

maxnoofsupportedTACs						INTEGER ::= 1024 -- FFS

maxnoofTAI									INTEGER ::= 16

maxnoofTAIinAoI 							INTEGER ::= 16   -- FFS

maxNRARFCN									INTEGER ::= 3279165

maxNrOfErrors								INTEGER ::= 256

-- **************************************************************

--

-- IEs

--

-- **************************************************************

id-ActivatedServedCells																				ProtocolIE-ID ::= 0

id-ActivationIDforCellActivation																	ProtocolIE-ID ::= 1

id-AMF-Pool-Information																				ProtocolIE-ID ::= 2

id-AreaOfInterest-Item																				ProtocolIE-ID ::= 3

id-AssistanceDataForPaging																			ProtocolIE-ID ::= 4

id-Cause																							ProtocolIE-ID ::= 5

id-cellAssistanceInfoNR																				ProtocolIE-ID ::= 6

id-ConfigurationUpdateInitiatingNodeChoice															ProtocolIE-ID ::= 7

id-CriticalityDiagnostics																			ProtocolIE-ID ::= 8

id-DataForwardingResponseDRBItem																	ProtocolIE-ID ::= 9

id-DRBsSubjectToStatusTransfer-Item																	ProtocolIE-ID ::= 10

id-DRBsSubjectToStatusTransfer-List																	ProtocolIE-ID ::= 11

id-DRBToQoSFlowMapping-Item																			ProtocolIE-ID ::= 12

id-globalNG-RAN-node-ID																				ProtocolIE-ID ::= 13

id-GlobalNG-RAN-node-ID																				ProtocolIE-ID ::= 14

id-GUAMI																							ProtocolIE-ID ::= 15

id-I-RNTI																							ProtocolIE-ID ::= 16

id-List-of-served-cells-E-UTRA																		ProtocolIE-ID ::= 17

id-List-of-served-cells-NR																			ProtocolIE-ID ::= 18

id-MAC-I																							ProtocolIE-ID ::= 19

id-MaskedIMEISV																						ProtocolIE-ID ::= 20

id-new-NG-RAN-Cell-Identity																			ProtocolIE-ID ::= 21

id-newNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 22

id-oldNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 23

id-PagingDRX																						ProtocolIE-ID ::= 24

id-PDUSessionResourceAdmittedResponseTransferItem													ProtocolIE-ID ::= 25

id-PDUSessionResourcesAdmitted-Item																	ProtocolIE-ID ::= 26

id-PDUSessionResourcesAdmitted-List																	ProtocolIE-ID ::= 27

id-PDUSessionResourcesNotAdmitted-Item																ProtocolIE-ID ::= 28

id-PDUSessionResourcesNotAdmitted-List																ProtocolIE-ID ::= 29

id-PDUSessionResourcesToBeSetup-Item																ProtocolIE-ID ::= 30

id-QoSFlowAdmitted-Item																				ProtocolIE-ID ::= 31

id-QoSFlow-Item																						ProtocolIE-ID ::= 32

id-QoSFlowNotAdmitted-Item																			ProtocolIE-ID ::= 33

id-QoSFlowsToBeSetup-Item																			ProtocolIE-ID ::= 34

id-RANPagingArea																					ProtocolIE-ID ::= 35

id-RANPagingPriority																				ProtocolIE-ID ::= 36

id-ServedCellsToActivate																			ProtocolIE-ID ::= 37

id-servedCellsToUpdate-E-UTRA																		ProtocolIE-ID ::= 38

id-ServedCellsToUpdateInitiatingNodeChoice															ProtocolIE-ID ::= 39

id-servedCellsToUpdate-NR																			ProtocolIE-ID ::= 40

id-sourceNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 41

id-TAISupport-Item																					ProtocolIE-ID ::= 42

id-TAISupport-list																					ProtocolIE-ID ::= 43

id-Target2SourceNG-RANnodeTranspContainer															ProtocolIE-ID ::= 44

id-targetCellGlobalID																				ProtocolIE-ID ::= 45

id-targetNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 46

id-TraceActivation																					ProtocolIE-ID ::= 47

id-UEContextInfoHORequest																			ProtocolIE-ID ::= 48

id-UEContextInfoRetrUECtxtResp																		ProtocolIE-ID ::= 49

id-UEIdentityIndexValue																				ProtocolIE-ID ::= 50

id-UEPagingIdentity																					ProtocolIE-ID ::= 51

END

### 9.3.8	Container definitions

-- **************************************************************

--

-- Container definitions

--

-- **************************************************************

XnAP-Containers {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-Containers (5) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

maxPrivateIEs,

maxProtocolExtensions,

maxProtocolIEs,

Criticality,

Presence,

PrivateIE-ID,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

-- Class Definition for Protocol IEs

--

-- **************************************************************

XNAP-PROTOCOL-IES ::= CLASS {

&id				ProtocolIE-ID 			UNIQUE,

&criticality	Criticality,

&Value,

&presence		Presence

}

WITH SYNTAX {

ID				&id

CRITICALITY		&criticality

TYPE			&Value

PRESENCE		&presence

}

-- **************************************************************

--

-- Class Definition for Protocol IEs

--

-- **************************************************************

XNAP-PROTOCOL-IES-PAIR ::= CLASS {

&id						ProtocolIE-ID 		UNIQUE,

&firstCriticality		Criticality,

&FirstValue,

&secondCriticality		Criticality,

&SecondValue,

&presence				Presence

}

WITH SYNTAX {

ID						&id

FIRST CRITICALITY 		&firstCriticality

FIRST TYPE				&FirstValue

SECOND CRITICALITY 		&secondCriticality

SECOND TYPE				&SecondValue

PRESENCE				&presence

}

-- **************************************************************

--

-- Class Definition for Protocol Extensions

--

-- **************************************************************

XNAP-PROTOCOL-EXTENSION ::= CLASS {

&id					ProtocolIE-ID 		UNIQUE,

&criticality		Criticality,

&Extension,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

EXTENSION			&Extension

PRESENCE			&presence

}

-- **************************************************************

--

-- Class Definition for Private IEs

--

-- **************************************************************

XNAP-PRIVATE-IES ::= CLASS {

&id					PrivateIE-ID,

&criticality		Criticality,

&Value,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

TYPE				&Value

PRESENCE			&presence

}

-- **************************************************************

--

-- Container for Protocol IEs

--

-- **************************************************************

ProtocolIE-Container {XNAP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Single-Container {XNAP-PROTOCOL-IES : IEsSetParam} ::=

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Field {XNAP-PROTOCOL-IES : IEsSetParam} ::= SEQUENCE {

id				XNAP-PROTOCOL-IES.&id					({IEsSetParam}),

criticality		XNAP-PROTOCOL-IES.&criticality			({IEsSetParam}{@id}),

value			XNAP-PROTOCOL-IES.&Value				({IEsSetParam}{@id})

}

-- **************************************************************

--

-- Container for Protocol IE Pairs

--

-- **************************************************************

ProtocolIE-ContainerPair {XNAP-PROTOCOL-IES-PAIR : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-FieldPair {{IEsSetParam}}

ProtocolIE-FieldPair {XNAP-PROTOCOL-IES-PAIR : IEsSetParam} ::= SEQUENCE {

id					XNAP-PROTOCOL-IES-PAIR.&id					({IEsSetParam}),

firstCriticality	XNAP-PROTOCOL-IES-PAIR.&firstCriticality	({IEsSetParam}{@id}),

firstValue			XNAP-PROTOCOL-IES-PAIR.&FirstValue			({IEsSetParam}{@id}),

secondCriticality	XNAP-PROTOCOL-IES-PAIR.&secondCriticality	({IEsSetParam}{@id}),

secondValue			XNAP-PROTOCOL-IES-PAIR.&SecondValue			({IEsSetParam}{@id})

}

-- **************************************************************

--

-- Container Lists for Protocol IE Containers

--

-- **************************************************************

ProtocolIE-ContainerList {INTEGER : lowerBound, INTEGER : upperBound, XNAP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-Container {{IEsSetParam}}

ProtocolIE-ContainerPairList {INTEGER : lowerBound, INTEGER : upperBound, XNAP-PROTOCOL-IES-PAIR : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-ContainerPair {{IEsSetParam}}

-- **************************************************************

--

-- Container for Protocol Extensions

--

-- **************************************************************

ProtocolExtensionContainer {XNAP-PROTOCOL-EXTENSION : ExtensionSetParam} ::=

SEQUENCE (SIZE (1..maxProtocolExtensions)) OF

ProtocolExtensionField {{ExtensionSetParam}}

ProtocolExtensionField {XNAP-PROTOCOL-EXTENSION : ExtensionSetParam} ::= SEQUENCE {

id					XNAP-PROTOCOL-EXTENSION.&id				({ExtensionSetParam}),

criticality			XNAP-PROTOCOL-EXTENSION.&criticality	({ExtensionSetParam}{@id}),

extensionValue		XNAP-PROTOCOL-EXTENSION.&Extension		({ExtensionSetParam}{@id})

}

-- **************************************************************

--

-- Container for Private IEs

--

-- **************************************************************

PrivateIE-Container {XNAP-PRIVATE-IES : IEsSetParam} ::=

SEQUENCE (SIZE (1..maxPrivateIEs)) OF

PrivateIE-Field {{IEsSetParam}}

PrivateIE-Field {XNAP-PRIVATE-IES : IEsSetParam} ::= SEQUENCE {

id				XNAP-PRIVATE-IES.&id			({IEsSetParam}),

criticality		XNAP-PRIVATE-IES.&criticality	({IEsSetParam}{@id}),

value			XNAP-PRIVATE-IES.&Value			({IEsSetParam}{@id})

}

END

## 9.4	Message transfer syntax

XnAP shall use the ASN.1 Basic Packed Encoding Rules (BASIC-PER) Aligned Variant as transfer syntax, as specified in ITU-T Rec. X.691 [15].

## 9.5	Timers

TXnRELOCprep

-	Specifies the maximum time for the Handover Preparation procedure in the source NG-RAN node.

TXnRELOCoverall

-	Specifies the maximum time for the protection of the overall handover procedure in the source NG-RAN node.

TXnDCprep

-	Specifies the maximum time for the S-NG-RAN node Addition Preparation or M-NG-RAN node initiated S-NG-RAN node Modification Preparation.

TXnDCoverall

-	Specifies the maximum time in the S-NG-RAN node for either the S-NG-RAN node initiated S-NG-RAN node  Modification procedure or the protection of the NG-RAN actions necessary to configure UE resources at S-NG-RAN node Addition or M-NG-RAN node initiated S-NG-RAN node Modification.

# 10	Handling of unknown, unforeseen and erroneous protocol data

Section 10 of TS 38.413 [5] is applicable for the purposes of the present document.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-04 | RAN3#95bis | R3-171316 |  |  |  | Implementing agreements from meeting RAN3#95bis: R3-171147 (removing last two IEs and FFS on NG-C UE), R3-171372, R3-171351 (only NSSAI related text), R3-171338 (with Editor’s Note on text and message structure), R3-171371 (with Editor’s Note in generic section and name for RAN Paging FFS), R3-171345, R3-171347 | 0.0.1 |
| 2017-05 | RAN3#96 |  |  |  |  | Add SGNB MODIFICATION REQUEST in tabular. Editorial change | 0.0.2 |
| 2017-05 | RAN3#96 |  |  |  |  | Implementing agreements from meeting RAN3#96: R3-171925 (Handover messages – tabular format), R3-171928 (additions for RAN Paging) Editorials (remove highlight, change style sheet assignments, correcting and adding references to other TSs and TRs, replacing some FFSs by Editor’s Notes) | 0.1.0 |
| 2017-06 | RAN3#ad-hoc2 | R3-172548 |  |  |  | Submission | 0.1.1 |
| 2017-06 | RAN3#ad-hoc2 | R3-173452 |  |  |  | Implementing agreed R3-172612 and agreed node naming conventions. | 0.2.0 |
| 2017-08 | RAN3#97 | R3-173462 |  |  |  | Implement the agreed pCRs from RAN3#97 meeting: R3-173237, R3-173337, R3-173416, R3-173429, R3-173431 | 0.3.0 |
| 2017-10 | RAN3#97bis | R3-174242 |  |  |  | Implementing the agreed pCRs from RAN3#97bis meeting: R3-173976, R3-174097, R3-174183, R3-174192, R3-174205 | 0.4.0 |
| 2017-12 | RAN3#98 | R3-175058 |  |  |  | Implementing agreed pCRs from RAN3#98 meeting: R3-175024, R3-174817, R3-174920, R3-174920, R3-174924, R3-174934, R3-174837, R3-175077 | 0.5.0 |
| 2018.01 | RAN3 AH 1801 | R3-180656 |  |  |  | Implementing agreed pCRs from RAN3 AH 1801: R3-180114, R3-180545, R3-180548, R3-180561, R3-180569, R3-180601, R3-180607, R3-180615, R3-180629, R3-180631, R3-180638 | 0.6.0 |
| 2018-03 | RAN3#99 | R3-181593 |  |  |  | Implementing agreed pCRs from RAN3#99: R3-180850, R3-180980, R3-181247, R3-181280, R3-181350, R3-181385, R3-181390, R3-181415, R3-181418, R3-181461, R3-181504, R3-181509 | 0.7.0 |
| 2018-04 | RAN3#99bis | R3-182527 |  |  |  | Implementing agreements from RAN3#99bis: R3-182213, R3-182396, R3-182401, R3-181855, R3-182488, R3-182371, R3-182157, R3-182373, R3-182375, R3-182376, R3-182163, R3-182384, R3-182392, R3-181825, R3-182494, R3-181980, R3-182433, update along R3-182378, update along R3-182344, update along R3-181899 | 0.8.0 |
| 2018-05 | RAN3#100 | R3-18xxxx |  |  |  | Implementing agreements from RAN3#100: R3-182614, R3-182615, R3-182635, R3-182815, R3-182935, R3-183091, R3-183154, R3-183165, R3-183252, R3-183314, R3-183369, R3-183376, R3-183386, R3-183389, R3-183393, R3-183404, R3-183407, R3-183411, R3-183441, R3-183442, R3-183444, R3-183450, R3-183455, R3-183497, R3-183511, R3-183517, R3-183519, R3-183534, R3-183541 | 0.9.0 |