3GPP TSG-SA3 Meeting #121	S3-XXXXXX

# Gothenburg, Sweden, 7 - 11 April 2025

Source:	NIST

Title:	Pseudo-CR on 3GPP Cryptographic Inventory for MIKEY-SAKKE

Document for:	Approval

Agenda item:	5.20

Spec:	3GPP TR 33.9xy

Version:	0.0.0

Work Item:	FS_CryptoInv

Comments

This document presents an inventory of services that that make use of the MIKEY-SAKKE protocol in response to the work item Study on 3GPP Cryptographic Inventory.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[aa]	3GPP TS 33.180: "Security of the Mission Critical (MC) service".

[ab]	IETF RFC 6509: ''MIKEY-SAKKE: Sakai-Kasahara Key Encryption in Multimedia Internet KEYing (MIKEY)''

* * * Next Change * * * *

## 4.4	Detailed Protocol List

### 4.4.X	MIKEY-SAKKE

MIKEY-SAKKE is used in the 5G system to securely transport cryptographic keys for Mission Critical Services. It is used in the following scenarios:

Group Master Keys (GMKs) from a Group Management Server to a Group Management Client on a MC UE (see Annex E clause E.2 TS 33.180[aa])

Private Call Keys (PCKs) between MC UEs (see Annex E clause E.3 TS 33.180[aa])

Client-Server keys (CSKs) between MCX Server and MC client (see Annex E clause E.3 TS 33.180[aa])

Multicast Signalling Keys (MuSiK) from MCX Servers to MC clients (see Annex E clause E.3 TS 33.180[aa])

Security profiles for MIKEY-SAKKE are left for implementation.

MIKEY-SAKKE as defined in IETF RFC 6509 [ab]

MIKEY-SAKKE employs symmetric cryptography for key distribution.

* * * End of Changes * * * *