3GPP TSG-SA3 Meeting #107-e 	S3-22xxxx

- e-meeting, 16 - 20 May 2022

- Source:	Nokia, Nokia Shanghai Bell

- Title:	derive 5GPRUK based on Kausf

- Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

ask approval of the pCR.

# 3	Rationale

In current specification, the relationship of keys in PC5 Key Hierarchy over Control Plane are not clear enough, and how to derive the keys are inconsistent or missed in different chapterss of the spec. Therefore, further clarification and refinement for PC5 Key Hierarchy over Control Plane are required.

This pCR fixs 6.3.3.3.2 and 6.3.3.3.3 to make clear that the 5GPRUK is derived from Kausf. In addition, remove Kausf_p from the PC5 Key Hierarchy over Control Plane .

# 4	Detailed proposal

(For pseudo CR, include the complete clause(s) or subclause(s) of the latest draft TS/TR to be modified, with clear clause and sub-clause headings included and all modifications shown with revision marks, unambiguously showing where the changes shall be made or inserted in the draft TS/TR. It is not sufficient to just state, for example, “add the following text to the draft TS/TR…”.)

################# start the 1st  change

#### 6.3.3.3	Security procedure over Control Plane

##### 6.3.3.3.1	General

This subclause describes the security mechanisms for the 5G ProSe Layer-3 UE-to-Network Relay authentication, authorization and key management using the primary authentication for PC5 keys establishment. Network entities AMF, AUSF and UDM are involved for key derivation and distribution of keys used for 5G ProSe UE-to-Network Relay communication. The UE shall be provisioned with necessary policies and parameters to use 5G ProSe services, as part of the UE ProSe Policy information as defined in TS 23.503 [7] clause 4.2.2. PCF shall provision the authorization policy and parameters for 5G ProSe UE-to-Network Relay discovery and communication as specified in 5.1.4 in TS 23.304 [2].

##### 6.3.3.3.2	Connection with 5G ProSe UE-to-Network Relay connection with setup of network Prose security context during PC5 link establishment

This subclause describes a procedure for a 5G ProSe Remote UE to establish a PC5 link between a 5G ProSe Remote UE and a 5G ProSe UE-to-Network Relay. The procedure includes how the 5G ProSe Remote UE is authenticated by AUSF via 5G ProSe UE-to-Network Relay and 5G ProSe UE-to-Network Relay's AMF during 5G ProSe PC5 establishment. The mechanism can be used by a 5G ProSe Remote UE while out of coverage.

Figure 6.3.3.3.2-1: 5G ProSe UE-to-Network Relay security procedure with setup of network Prose security context during PC5 link establishment

0.	The 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay shall be registered with the network. The 5G ProSe UE-to-Network Relay shall be authenticated and authorized by the network to provide UE-to-Network relay service. The 5G ProSe Remote UE shall be authenticated and authorized by the network to receive UE-to-Network relay service. PC5 security policies are provisioned to the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay respectively during this authorization and information provisioning procedure.

1.	The 5G ProSe Remote UE shall initiate discovery procedure using any of Model A or Model B method as specified in clause 6.3.1.2 or 6.3.1.3 of TS 23.304 [2] respectively.

2-5.	After the discovery of the 5G ProSe UE-to-Network Relay, the 5G ProSe Remote UE shall send a Direct Communication Request to the 5G ProSe UE-to-Network Relay for establishing secure PC5 unicast link. The 5G ProSe Remote UE shall include its security capabilities and PC5 security signalling policy in the DCR message as specified in TS 33.536 [6]. The message shall also include SUCI, Relay Service Code, Nonce_1. Upon receiving the DCR message, the 5G ProSe UE-to-Network Relay shall send the Relay Key Request to the AMF of the 5G ProSe UE-to-Network Relay, including partial parameters received in the DCR message. The 5G ProSe UE-to-Network Relay shall also include in the message a transaction identifier that identifies the 5G ProSe Remote UE for the subsequent messages over 5G ProSe UE-to-Network Relay's NAS messages and PC5 messages. The AMF of the 5G ProSe UE-to-Network Relay shall verify whether the 5G ProSe UE-to-Network Relay is authorized to provide the UE-to-Network relay service. The AMF of the 5G ProSe UE-to-Network Relay shall select an AUSF based on SUCI and forward the parameters received in Relay Key Request to the AUSF in Nausf_UEAuthentication_ProseAuthenticate Request message. The Nausf_UEAuthentication_ProseAuthenticate Request message shall contain the 5G ProSe Remote UE’s SUCI, Relay Service Code, Nonce_1. The AUSF shall initiate a 5G ProSe Remote UE specific authentication using the ProSe specific parameters received (i.e., RSC, etc.). The serving network name handling is same as defined in TS 33.501 [3]. The security policy negotiation and protection of messages hereafter shall follow the one-to-one security establishment described in clause 6.2.3 of the present document.

6.	The AUSF of the 5G ProSe Remote UE shall retrieve the Authentication Vectors from the UDM via Nudm_UEAuthentication_GetProseAv Request message and trigger authentication of the 5G ProSe Remote UE . This authentication is performed between the AUSF of the 5G ProSe Remote UE and the 5G ProSe Remote UE via the AMF of the 5G ProSe UE-to-Network Relay and the 5G ProSe UE-to-Network Relay. Based on SUPI, the UDM shall choose the authentication method.

7a. If EAP-AKA' is selected by UDM, the AUSF of  the 5G ProSe Remote UE shall trigger authentication of the 5G ProSe Remote UE based on EAP-AKA'. The AUSF of the 5G ProSe Remote UE generates the EAP-Request/AKA'-Challenge message defined in clause 6.1.3.1 of TS 33.501 and send EAP-Request/AKA'-Challenge message to the AMF of the 5G ProSe UE-to-Network Relay in a Nausf_UEAuthentication_ProSeAuthenticate Response message.

7b. The AMF of the 5G ProSe UE-to-Network Relay shall forward the Relay Authentication Request (including the EAP-Request/AKA'-Challenge) to the 5G ProSe UE-to-Network Relay over NAS message, including transaction identifier of the 5G ProSe Remote UE in the message. The NAS message is protected using the NAS security context created for the 5G ProSe UE-to-Network Relay.

7c. Based on the transaction identifier, the 5G ProSe UE-to-Network Relay shall forwards the EAP-Request/AKA'-Challenge to the 5G ProSe Remote UE over PC5 messages.

The USIM in the 5G ProSe Remote UE verifies the freshness of the received values by checking whether AUTN can be accepted as described in TS 33.102 [11].

For EAP-AKA', the USIM computes a response RES. The USIM shall return RES, CK, IK to the ME. The ME shall derive CK' and IK' according to Annex A.3 in TS 33.501.

7d. The 5G ProSe Remote UE shall return EAP-Response/AKA'-Challenge to the 5G ProSe UE-to-Network Relay over PC5 messages.

7e. The 5G ProSe UE-to-Network Relay forwards the EAP-Response/AKA'-Challenge together with the transaction identifier of the 5G ProSe Remote UE to the AMF of the 5G ProSe UE-to-Network Relay in a NAS message Relay Authentication Response.

7f. The AMF of the 5G ProSe UE-to-Network Relay forwards EAP-Response/AKA'-Challenge to the AUSF of the 5G ProSe Remote UE via Nausf_UEAuthentication_ProSeAuthenticate Request.

The AUSF of the 5G ProSe Remote UE performs the UE authentication by verifying the received information as described in TS33.501.

For EAP-AKA’, the AUSF of the 5G ProSe Remote UE and the 5G ProSe Remote UE may exchange EAP-Request/AKA’-Notification and EAP-Response /AKA’-Notification messages via the AMF of the 5G ProSe UE-to-Network Relay.

NAS SMC procedure is not performed between 5G ProSe Remote UE and AMF of the 5G ProSe UE-to-Network Relay.

8.	On successful authentication, the AUSF of the 5G ProSe Remote UE and the 5G ProSe Remote UE shall generate 5GPRUK as specified in Annex A.2 and 5GPRUK ID as specified in Annex A.3 using the existing KAUSF which was generated during primary authetncaiton of the remote UE .

9.	The AUSF of the 5G ProSe Remote UE shall generate the KNR_ProSe key as defined in Annex A.4.

10-11.	The AUSF of the 5G ProSe Remote UE shall send the KNR_ProSe, Nonce_2 in Nausf_UEAuthentication_ProseAuthenticate Response message to the 5G ProSe UE-to-Network Relay via the AMF of the 5G ProSe UE-to-Network Relay. When receiving a KNR_ProSe from the AUSF of the 5G ProSe Remote UE, the AMF of the 5G ProSe UE-to-Network Relay shall not attempt to trigger NAS SMC procedure with the 5G ProSe Remote UE. The 5G ProSe UE-to-Network Relay derives PC5 session key Krelay-sess and confidentiality and integrity keys from KNR_ProSe, as defined in clause 6.3.3.3.3 of this document. KNR_ProSe ID and Krelay-sess ID are established in the same way as KNRP ID and KNRP-sess ID in TS 33.536 [6].

12.	The 5G ProSe UE-to-Network Relay shall send the received Nonce_2 to the 5G ProSe Remote UE in Direct Security mode command message, which is protected using Krelay-int or/and Krelay-enc derived from Krelay-sess according to the negotiated PC5 signalling policies between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay.

13-15.	The 5G ProSe Remote UE shall use the 5GPRUK ID to locate the 5GPRUK to be used for the PC5 link security. The 5G ProSe Remote UE shall generate the KNR_ProSe key to be used for Remote access via the 5G ProSe UE-to-Network Relay in the same way as defined in step 9. The 5G ProSe Remote UE shall derive PC5 session key Krelay-sess and confidentiality and integrity keys from KNR_ProSe the same way as defined in step 11. The 5G ProSe Remote UE shall send the Direct Security Mode Complete message containing its PC5 user plane security policies to the 5G ProSe UE-to-Network relay, which is protected by Krelay-int or/and Krelay-enc derived from Krelay-sess according to the negotiated PC5 signalling policies between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay. After the successful verification of the Direct Security Mode complete message, the 5G ProSe UE-to-Network Relay responds a Direct Communication Accept message to the 5G ProSe Remote UE to finish the PC5 connection establishment procedures.

Further communication between the 5G ProSe Remote UE and the Network takes place securely via the 5G ProSe UE-to-Network Relay.

Editor's note:	Further details on the needs and usage of 5GPRUK ID are FFS.

##### 6.3.3.3.3	PC5 Key Hierarchy over Control Plane

Figure 6.3.3.3.3-1: PC5 Key Hierarchy for 5G ProSe UE-to-Network Relay security over Control Plane

The different layers of keys (see Figure 6.3.3.3.3-1) are the following:

-	5GPRUK: The root credential derived from KAUSF which is the root of security of the 5G ProSe Remote UE for the 5G ProSe layer 3 UE-to-Network Relay service.

-	KNR_ProSe: This is a 256-bit root key that is established between the two entities that communicating using NR PC5 unicast link. It may be refreshed by re-running the authentication to derive a fresh 5GPRUK.

-	Krelay-sess: This is the 256-bit key that is derived by UE from KNR_ProSe and is used derive keys that to protect the transfer of data between the UEs. The Krelay-sess is derived per unicast link same as KNRP-sess specified in TS 33.536 [6]. During activated unicast communication session between the UEs, the Krelay-sess may be refreshed by running the rekeying procedure. The keys for confidentiality and integrity algorithms are derived directly from Krelay-sess. The 16-bit Krelay-sess ID identifies the Krelay-sess.

-	Krelay-int, Krelay-enc: The Krelay-int and Krelay-enc are used in the chosen confidentiality and integrity algorithms respectively for protecting PC5-S signalling, PC5 RRC signalling, and PC5 user plane data. These keys are equivalent to NRPIK and NRPEK as specified in TS 33.536 [6]. They are derived from Krelay-sess and are refreshed automatically every time Krelay-sess is changed.

################# end of the change