********** Start of Changes ***************

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [100] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [100].

AI/ML	Artificial Intelligence/Machine Learning

AS	Application Server

BD	Billing Domain

CDF	Charging Data Function

CGF	Charging Gateway Function

CTF	Charging Trigger Function

ECUR	Event Charging with Unit Reservation

IE	Information Element

IEC	Immediate Event Charging

NEF	Network Exposure Function

PEC	Post Event Charging

SCEF	Service Capability Exposure Function

SCS	Services Capability Server

SGSN	Serving GPRS Support Node

RCAF	RAN Congestion Awareness Function

********** Next Change ***************

##### 6.3.1.4.1	Definition of the NEF API Charging Information

Network Exposure Function API specific charging information is provided within the NEF API Charging Information. The detailed structure of the NEF API Charging Information can be found in table 6.3.1.4.1.

Table 6.3.1.4.1: Structure of the NEF API Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| External Individual Identifier | OC | This parameter holds the external Identifier of the individual UE, e.g., the GPSI. |
| Internal Individual Identifier | OC | This parameter holds the internal Identifier of the individual UE e.g., the SUPI. |
| External Group Identifier | OC | This parameter holds the external identifier for a group of individual UE(s), if available. |
| Internal Group Identifier | OC | The internal Identifier identifying a group of individual UE(s). |
| List of UEs | OC | This field holds the list of GPSIs for the reported UEs. Applicable for the case of AI/ML operation e.g. Federated Learning operation as defined by the TS 23.501 [200], clause 5.45 and by the TS 23.502 [201], clause 4.15.6.13. |
| Consolidated Data Rate | OC | The upper bound of the aggregated data rate across all traffic flows corresponding to the “List of UEs”. Applicable for the case of AI/ML operation e.g. Federated Learning operation as defined by the TS 23.501 [200], clause 5.45 and by the TS 23.502 [201], clause 4.15.6.13. |
| API Direction | M | This field holds the direction to indicate if it is an API invocation from an AF or notification to an AF. |
| API Target Network Function | OC | This field holds the identifier of the network function that either is the destination of the API invocation or triggers the notification: e.g. PCF, UDM, AMF, SMF, UPF. |
| API Result Code | OC | This parameter holds the result of API Invocation. |
| API Name | M | This field holds the name of the API invoked. |
| API Operation | OC | This field holds the operation of the API invoked, together with detailed description. |
| API Reference | OC | This field holds the reference to the definition of the format of the API invocation, this can be a URI or refence to the standard where it's specified |
| API Content | OC | This field holds the actual content of the API invocation, in the format described by the API Reference |

********** Next Change ***************

### 6.3.4	Detailed message format for converged charging

The operation types are listed in the following order: I  (Initial) / U (Update)/T  (Termination)/E  (Event). Therefore, when all Operation types are possible it is marked as IUTE. If only some operation types are allowed for a node, only the appropriate letters are used ( e.g., IUT or E) as indicated in the table heading. The omission of an operation type for a particular field is marked with "-" ( e.g., I-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.3.4.1 illustrates the basic structure of the supported fields in the Charging Data Request for exposure function API online charging.

Table 6.3.4.1: Supported fields in Charging Data Request message

| Information Element | Node Type | NEF |
| --- | --- | --- |
|  | Supported Operation Types | IUTE |
| Session Identifier |  | I-TE |
| Subscriber Identifier |  | I-TE |
| NF Consumer Identification |  | I-TE |
| Charging Identifier |  | I-TE |
| Invocation Timestamp |  | I-TE |
| Invocation Sequence Number |  | I-TE |
| Retransmission Indicator |  | - |
| One-time Event |  | ---E |
| One-time Event Type |  | ---E |
| Notify URI |  | I--- |
| Supported Features |  | I--E |
| Triggers |  | I-TE |
| Multiple Unit Usage |  | I-TE |
| NEF API Charging Information |  |  |
| External Individual Identifier |  | I-TE |
| Internal Individual Identifier |  | ITE |
| External Group Identifier |  | I-TE |
| Internal Group Identifier |  | I-TE |
| List of UEs |  | I-TE |
| Consolidated Data Rate |  | I-TE |
| API Direction |  | I-TE |
| API Target Network Function |  | I-TE |
| API Result Code |  | I-TE |
| API Name |  | I-TE |
| API Operation |  | ITE |
| API Reference |  | I-TE |
| API Content |  | I-TE |

Table 6.3.4.2 illustrates the basic structure of the supported fields in the Charging Data Response for exposure function API converged charging.

Table 6.3.4.2: Supported fields in Charging Data Response Message

| Information Element | Node Type | NEF |
| --- | --- | --- |
|  | Supported Operation Types | IUTE |
| Session Identifier |  | I-TE |
| Invocation Timestamp |  | I-TE |
| Invocation Result |  | I-TE |
| Invocation Sequence Number |  | I-TE |
| Session Failover |  | I-TE |
| Supported Features |  | I--E |
| Triggers |  | - |
| Multiple Unit Information |  | I--E |

********** End of Changes ***************