| 3GPP TS 28.111 0.3.0 (2023-11) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Fault Management (FM) (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Concepts and overview	10

4.1	Overview	10

5	 Requirements	10

6	Fault Management service Components overview	12

7	Solution description	13

7.1	Overview	13

7.2	Behaviour of the Fault Management	13

7.3	Alarm States	14

7.4	Locking the AlamList	16

7.5	Unlocking the AlarmList	16

7.6	Subscribing to alarm notifications	17

7.7	Unsubscribing from alarm notifications	17

7.8	Notifying Alarms	17

7.8.1	Notifying new alarms	17

7.8.2	Notifying changed alarms	18

7.9	Retrieving alarms	19

7.10	Notifying a potentially faulty alarm list	19

7.11	Notifying alarm list is rebuilt	19

7.12	Acknowledging alarms	20

7.13	Clearing alarms	20

7.14	Virtualized resource alarm correlation	21

8	Model	21

8.1	Imported information entities and local labels	21

8.2	Class diagrams	21

8.2.1	Relationships	21

8.2.2	Inheritance	22

8.3	Class definitions	22

8.3.1	AlarmRecord <<dataType>>	22

8.3.1.1	Definition	22

8.3.1.2	Attributes	23

8.3.1.3	Attribute constraints	24

8.3.1.4	Notifications	24

8.3.2	AlarmList	24

8.3.2.1	Definition	24

8.3.2.2	Attributes	24

8.3.2.3	Attribute constraints	24

8.3.2.4	Notifications	25

8.3.3	AlarmComment <<dataType>>	25

8.3.3.1	Definition	25

8.3.3.2	Attributes	25

8.3.3.3	Attribute constraints	25

8.3.3.4	Notifications	25

8.3.4	NameValuePair <<dataType>>	25

8.3.4.1	Definition	25

8.3.4.2	Attributes	25

8.3.4.3	Attribute constraints	25

8.3.4.4	Notifications	25

8.3.5	AttributeValueChange <<dataType>>	25

8.3.5.1	Definition	25

8.3.5.2	Attributes	26

8.3.5.3	Attribute constraints	26

8.3.5.4	Notifications	26

8.3.6	CorrelatedNotification <<dataType>>	26

8.3.6.1	Definition	26

8.3.6.2	Attributes	26

8.3.6.3	Attribute constraints	26

8.3.6.4	Notifications	26

8.4	Attribute definitions	27

8.4.1	Attribute properties	27

8.4.2	Constraints	34

8.5	Common notifications	34

8.5.1	Alarm notifications	34

8.5.2	Configuration notifications	34

9	 Notifications	34

9.1	Overview	34

9.2	notifyNewAlarm	35

9.2.1	Definition	35

9.2.2	Input parameters	35

9.3	notifyClearedAlarm	36

9.3.1	Definition	36

9.3.2	Input parameters	36

9.4	notifyChangedAlarmGeneral	37

9.4.1	Definition	37

9.4.2	Input parameters	37

9.5	notifyAlarmListRebuilt	38

9.5.1	Definition	38

9.5.2	Input parameters	38

9.6	notifyChangedAlarm	39

9.6.1	Definition	39

9.6.2	Input parameters	39

9.7	notifyCorrelatedNotificationChanged	40

9.7.1	Definition	40

9.7.2	Input parameters	40

9.8	notifyAckStateChanged	41

9.8.1	Definition	41

9.8.2	Input parameters	41

9.9	notifyComments	42

9.9.1	Definition	42

9.9.2	Input parameters	42

9.10	notifyPotentialFaultyAlarmList	43

9.10.1	Definition	43

9.10.2	Input parameters	43

10	Stage 3 – Solution Sets	44

10.1	 YANG Definitions	44

10.2	OpenAPI Definitions	44

10.3	RESTful HTTP-based solution set	44

10.3.1	General	44

10.3.2	Mapping of notifications	45

10.4	RESTful HTTP-based solution set for integration with ONAP VES API	45

10.4.1	General	45

10.4.2	Mapping of notifications	45

10.4.2.1	General	45

10.4.2.2	Resources	45

10.4.3	Integration with ONAP VES	45

Annex A (normative):  Probable Causes	46

Annex <B> (informative): PlantUML source text of Figures	52

B.1	PlantUML for Figure 7.3.1-1: Alarm state diagram with consumer acknowledgement	52

B.2	PlantUML for Figure 7.3.1-2: Alarm state diagram with automatic acknowledgement	53

B.3	PlantUML for Figure 7.6-1:Subscribing to alarm notifications – sequence diagram	54

B.4	PlantUML for Figure 7.8-1: Notifying new alarms – sequence diagram	54

B.5	PlantUML for Figure 7.11-1: Notify alarm list is rebuilt – sequence diagram	55

Annex <X> (informative): Change history	55

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

This document describes the SBMA based Fault Management service. It includes stage 1,2 and 3.

This specification of the Fault Management MnS is based on the SBMA principles using CRUD operations, modeled OAM data in the NRM together with fault management specific notifications. An IRP based solution for fault management is out of scope for this document.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.532: "Management and orchestration; Management services".

[3]	ETSI TS 101 251 V6.3.0 “Digital cellular telecommunications system (Phase 2+); Fault management of the Base Station System (BSS) (GSM 12.11 version 6.3.0 Release 1997)”

[4]	3GPP TS 28.516: "Fault Management (FM) for mobile networks that include virtualized network functions; Procedure".

[5]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[6]	ITU-T Recommendation X.721 (01/92) : “Interface between Data Terminal Equipment and Data Circuit-terminating Equipment for synchronous operation on public data networks  ”3	Definitions of terms, symbols and abbreviations”.

[7]	ITU-T Recommendation M.3100 “Generic network information model”.

[8]	ITU-T Recommendation X.733 (02/92): "Information technology - Open Systems Interconnection - Systems Management: Alarm reporting function".

[9]	Text Attribution: Creator: ONAP, under Creative Commons Attribution 4.0 International License, https://creativecommons.org/licenses/by/4.0/, URI to access the text: https://github.com/onap/vnfrqts-requirements/blob/05f26fac2b941513a7d0e856b99fd8c61d688299/docs/Chapter8/ves7_1spec.rst#resource-structure.

[10]	3GPP TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS)".

[11]	Management and Orchestration APIs Stage 3 Repository https://forge.3gpp.org/rep/sa5/MnS/-/tree/Tag_Rel18_SA102/

[12]	3GPP TS 32.401: "Telecommunication management; Performance Measurement (PM); Concept and requirements".

[13]	ITU-T Recommendation X.736 (01/92): “Information technology - Open Systems Interconnection - Systems Management: Security alarm reporting function”

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Event: Anything that occurs at a certain point in time, for example a configuration change, a threshold crossing, a transition to an error state or a transition to a failure state. Events do not have states.

Error: A state of the system different from the correct system state. An error may or may not lead to a service failure. An error has a begin and end time.

Failure: A state of inability to deliver the correct service as defined by the service specification. A service failure may be the result of an error or a poor service function design. A failure has a begin and end time.

Fault: The (hypothesized or adjudged) cause for an error or a failure.

MonitoredEntity: Any class that can have an alarmed state.

Alarm: A representation of an error or failure that requires attention or reaction by an operator or some machine. Alarms have state.

Alarm-info-set: A set of information associated with an alarm. The infoset can be used in multiple ways e.g., stored by the provider like an alarmRecord or sent in a notification. The same set of information should be applicable in all scenarios.

Notification: A message sent from the provider to the consumer based on some internal event that informs the receiver about an alarm or about an event e.g. a configuration change.

Root cause: The primary fault (cause), if any, leading to one or multiple errors or failures.

Matching-Criteria-Attributes: A set of attributes (objectInstance, alarmType, probableCause and specificProblem, if present) that identify an alarm. ObjectInstance identifies the network resource ,while alarmType, probableCause and specificProblem (if present) identify the alarming condition.

## 3.2	Symbols

Void

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ADAC	Automatically Detected and Automatically Cleared

ADMC	Automatically Detected and Manually Cleared

CRUD	Create, Read, Update, Delete basic data manipulation operations

FM	Fault Management

ME	Managed Element

MnS	Management Service

NRM	Network Resource Model

# 4	Concepts and overview

## 4.1	Overview

Fault Management is considered a generic management service. It shall be able to support fault indications about any radio technology and any resource that can be addressed by a distinguished name e.g., ManagedElements, GNBDUs or NetworkSlices.

Fault management can handle alarms about any kind of fault in a 3GPP system from small hardware errors to service failures effecting many users.

# 5		Requirements

| Requirement label | Description | Motivation |
| --- | --- | --- |
| REQ-FM-1 | The 3GPP management system shall have the capability to provide alarm notifications to authorized consumers. | Motivation: the consumer should receive information about alarms immediately when an alarm is raised or changed. |
| REQ-FM-2 | The 3GPP management system shall have the capability to allow authorized consumers to subscribe to alarm notifications. | Motivation: Needed for REQ-FM-1. Producers will not send notification without an explicit subscription. |
| REQ-FM-3 | The 3GPP management system shall have the capability to allow authorized consumers to unsubscribe from alarm notifications. | Motivation: The consumer needs to be able to indicate that it is no longer interested in receiving immediate alarm information |
| REQ-FM-4 | The 3GPP management system should have the capability to allow authorized consumers to provide a filter for alarm notifications. | Motivation: The consumer shall be able to indicate that it is interested only in a subset of alarms. |
| REQ-FM-5 | The 3GPP management system shall have the capability to allow authorized consumers to retrieve the alarm list. | Motivation: The consumer shall be able to read all current alarms. It needs this if the sequence of received alarm notifications does not provide a reliable and complete view of the alarm situation. This may happen after the start-up of the consumer fault management service, if the connection or some alarm notifications are lost, or if the alarm producer was not able to provide on-time indication of all alarm changes. |
| REQ-FM-6 | The 3GPP management system should have the capability to allow authorized consumers to retrieve a filtered subset of the alarm list. | Motivation: If the consumer is interested only in a subset of alarms, it shall be able to retrieve only that subset. |
| REQ-FM-7 | The 3GPP management system shall have the capability to provide changed alarm notifications to its authorized consumer. | Motivation: the consumer should receive information about changed alarms immediately. |
| REQ-FM-8 | The 3GPP management system shall have the capability to provide cleared alarm notifications to its authorized consumer. | Motivation: the consumer should receive information about cleared alarms immediately. |
| REQ-FM-9 | The 3GPP management system shall have the capability to provide new generated alarm notifications to its authorized consumer. | Motivation: the consumer should receive information about alarms immediately when an alarm is raised. |
| REQ-FM-10 | The 3GPP management system should have the capability to indicate that the alarm list is potentially faulty. | Motivation: the consumer should receive information when the alarm list is corrupt or out-of-date. |
| REQ-FM-11 | The 3GPP management system shall have the capability to indicate that the alarm list was rebuilt and is reliable again after a previous disturbance. | Motivation: the consumer should receive information when the correct alarm information is available again. |
| REQ-FM-12 | The 3GPP management system should have the capability to satisfy the request to acknowledge one or multiple alarms. If this capability is not supported, then the producer shall be able to automatically acknowledge alarms. | Motivation: the consumer should be able to register in the producer that it has received the alarm and has done some vendor specific level of processing of the alarm information. |
| REQ-FM-13 | The 3GPP management system should have the capability to satisfy the request to clear one or multiple alarms. This capability is only applicable if one or more of the alarms supported by the producer is of type ADMC. | Motivation: If the producer supports ADMC alarms, the consumer shall be able to clear those. |
| REQ-FM-14 | The 3GPP management system should have the capability to provide acknowledgement state change notifications to its authorized consumer. | Motivation: the consumer should receive information about acknowledged alarms immediately. |

# 6	Fault Management service Components overview

The service consists of a “Base” feature (with manadatory and optional items) and additional features that are optional to support:

- Acknowledgment of alarms by the consumer

- Clearing alarms by the consumer

- Setting comments for alarms.

- Correlated notifications

List of components used by Fault Management:

| Feature | S | Component A (Operations, notifications) | Component B (NRM) |
| --- | --- | --- | --- |
| FM NmS Base | M | createMOI |  |
|  | M | getMOIAttributes |  |
|  | M | modifyMOIAttributes |  |
|  | M | deleteMOI |  |
|  | M | notifyNewAlarm |  |
|  | M | notifyClearedAlarm |  |
|  | M | notifyChangedAlarmGeneral |  |
|  | O | notifyChangedAlarm |  |
|  | O | notifyPotentialFaultyAlarmList |  |
|  | O | notifyAlarmListRebuilt |  |
|  | O | notifyCorrelatedNotificationChanged |  |
|  | O | notifyMOICreation |  |
|  | O | notifyMOIAttributeValueChanges |  |
|  | O | notifyMOIDeletion |  |
|  | O | notifyMOIChanges |  |
|  | M |  | AlarmList, IOC |
|  | M |  | AlarmRecord, dataType |
|  | O |  | CorrelatedNotification, datatype |
|  | M |  | NtfSubscriptionControl, IOC |
|  | O |  | HeartbeatControl, IOC |
| Acknowledgment of alarms | O | notifyAckStateChanged | AlarmRecord.ackTime AlarmRecord.ackUserId AlarmRecord.ackSystemId AlarmRecord.ackState |
| Clearing alarms by the consumer | O |  | AlarmRecord.clearUserId AlarmRecord.clearSystemId |
| Setting comments for alarms | O | notifyComments | AlarmComment dataType AlarmRecord.comments |

Note 1:

Some parts of the AlarmRecord dataType are only needed for the support the optional features.

# 7	Solution description

## 7.1	Overview

In managed systems faults occur. When a fault occurs this often results in an error or failure. If the issue is deemed important, then this will be represented by an alarm.

An alarm consist of identifying attributes (matching-criteria-attributes) and a set of related information (Alarm-info-set embodied by the alarmRecord). Active alarms are stored in the AlarmList object in the NRM. If there are any subscribed and authorized consumers, alarm notifications are sent to them to inform them about the alarm.

An  alarm has a lifecycle of different states as descibed in clause “Alarm states”.

Changes to the alarm state or other parts of  the Alarm-info-set are stored in the alarm records. Subscribed and authorized consumers are informed about the changes with notification messages.

Once the alarm reaches the states “ack_clear” (cleared and acknowledged) the alarm will be considerd inactive. It will be removed from the AlarmList and notifications will no longer be sent about it. The information about the alarm might still be maintained in the producer e.g. logged, but that is out of scope for the current specification.

Fault management receives input from other parts of a system (external to fault management e.g. an interface card  hardware) and from the MnS consumers via CRUD operations. It stores this information and send out notifications about it to subscribed consumers.

Fault Management uses SBMA principles to set and read data in the NRM. CRUD operations (createMOI, getMOIAttributes, modifyMOIAttributes , deleteMOI) implemented by the provisioning MnS are used to manipulate fault management related data. Fault management itself does not implement any operations. Additionally, notifications are also used.

NRM data is written to control the behaviour of the fault management.

Data provided to the fault management consumer is made available in two ways. Data that should be provided as soon as it is available in the MnS producer is sent to subscribed and MnS consumers in notifications (e.g., information about a new alarm). Other data is available to the MnS consumers using the provisioning getMOIAttributes operation (e.g., information about a subscription).

To receive notifications consumers need to subscribe to them. When notifications are no longer needed the consumer shall unsubscribe.

Access control may be used to restrict read or write access to alarm related data or to restrict sending of notifications.

Note: The name of operations, notifications IOCs, attributes defined in stage 2 and stage 3 in clauses 8,9 and 10 are indicated in italics.

Note: In the following sequence diagrams all internal actions and participants inside the producer are only presented to help understanding, but are not in scope of this specification. The sequence diagrams represent typical flows, but are only provided as informative.

## 7.2	Behaviour of the Fault Management

When the MnS producer is installed or started it automatically creates the AlarmList MOI(s). The AlarmList MOI and thus the Fault Management service may be created in an enabled or disabled state.

Individual alarms are represented by an attribute element (an individual value) of the AlarmList.alarmRecords attribute; this attribute element will be referred to as the alarmRecord in this document. An alarmRecord is created by the MnS producer as a result of a new alarm being raised. AlarmRecords cannot be created or deleted by the MnS consumer.

Alarm records are maintained only for active alarms. Inactive alarms are automatically deleted by the MnS producer from the AlarmList. Active alarms are alarms whose

a)	perceivedSeverity is not "CLEARED", or whose

b)	perceivedSeverity is "CLEARED" and its ackState is not "ACKNOWLEDGED" and alarm acknowledgement by the consumer is supported.

In the NRM AlarmList object the individual alarm instances represented by a specific alarmRecord element in the  alarmRecord are identified by the alarmRecord.alarmId.

Both in NRM and in the notifications alarms can be identified by the Matching-Criteria-Attributes; see clause  “3.1 Definitions”.

## 7.3	Alarm States

An alarm is a stateful concept. It has the following states:

- unack&unclear – the alarm has been raised (created) but is neither acknowledged nor cleared. If the MnS producer implements automatic acknowledgement this state might not be used.

- ack&unclear – the alarm has been raised (created) and acknowledged but has not been cleared.

- unack&clear – the alarm has been raised (created) and cleared, but is not acknowledged. If the MnS producer implements automatic acknowledgement this state might not be used.

- ack&clear – this is the transient, terminal state of the alarm. At this point the alarm is removed from the AlarmList, but usually still visible in (vendor specific) alarm log.

Alarm states are visible in the AlarmRecord in AlarmList indicated by the attribute fields alarmRecord.ackState and alarmRecord.perceivedSeverity.

For brevity in the following state diagrams some possible activities are excluded. Data in the AlarmRecord may be updated e.g., backedUpStatus, backUpObject, trendIndication, thresholdInfo, stateChangeDefinition, monitoredAttributes, proposedRepairActions, additionalText, additionalInformation, serviceUser, serviceProvider or securityAlarmDetector, comments, correlated notifications.

The only changes that result in alarm state changes are changing the  alarmRecord.ackState and setting alarmRecord.perceivedSeverity. The alarm states and diagrams are applicable both to ADAC and ADMC alarms.

The solid circle icon represents the Start State. The double circle icon represents the End State. In this state, the alarm is cleared and acknowledged. The AlarmRecord shall not be accessible via the Service interface and is removed from the AlarmList.

"PS" used in the state diagram stands for "perceived severity".

Figure 7.3.1-1:Alarm state diagram with consumer acknowledgement

If the system uses automatic acknowledgement (not allowing acknowledgement by the consumer) the following simpler state diagram is applicable:

Figure 7.3.1-2: Alarm state diagram with automatic acknowledgement

## 7.4	Locking the AlamList

The AlarmList.administrativeState attribute can be used to lock/unlock Fault management.

When the AlarmList is in a locked state, its attributes (except the administrativeState) may contain any unreliable data. No alarm notifications are sent by the MnS producer.

When the AlarmList is unlocked and in a disabled state, its attributes (except the administrativeState and operationalState) may contain any unreliable data. No alarm notifications are sent by the MnS producer.

## 7.5	Unlocking the AlarmList

The AlarmList.administrativeState attribute can be used to enable/disable Fault management.

When the AlarmList is modified from a LOCKED administrative state to UNLOCKED, for a short period, the alarmRecords attribute may contain out-of-date elements, as the MnS producer may need time to correctly present its internal alarm state by adding, removing or changing alarmRecords.

After the alarm list is unlocked/enabled and the AlarmRecords are updated the MnS producer shall execute the Notify Alarm List Rebuilt use case if that is supported (see clause 7.11).

## 7.6	Subscribing to alarm notifications

To receive notifications consumer needs to subscribe to them. A subscription can be created by the consumer by using the creating or updating a NtfSubscriptionControl MOI. The MOI attributes include data about the recipient of the notifications and attributes to filter which notification types are sent and the scope of resources about which the faults are reported. For details about NtfSubscriptionControl MOI see TS 28.622 [5]. Usually, the consumer should subscribe to at least the following four notificationTypes: notifyNewAlarm, notifyChangedAlarmGeneral, notifyAlarmListRebuilt, notifyClearedAlarm.

Figure 7.6-1: Subscribing to alarm notifications – sequence diagram

The consumer can modify its subscription at any time using e.g., the modifyMOIAttributes operation targeted at the NtfSubscriptionControl MOI created previously.

Heartbeat notifications based on the HeartBeatControl IOC can be used in conjuction with fault management.

## 7.7	Unsubscribing from alarm notifications

When notifications are no longer needed the consumer shall unsubscribe. This can be accomplished by deleting or updating an NtfSubscriptionControl MOI created/updated during subscription.

## 7.8	Notifying Alarms

### 7.8.1	Notifying new alarms

A new alarm is raised and AlarmList.administrativeState=UNLOCKED. An alarm is considered new if the AlarmList does not contain an alarmRecord that has identical matching-criteria-attributes values. The MnS producer will

- Add a new attribute element to the AlarmList.alarmRecords attribute, this attribute element will be referred to as the alarmRecord in this document. alarmRecord represents the new alarm. A system may support multiple AlarmLists. The relevant AlarmList instance can be found by finding the ancestor MOI of type ManagedElement and/or Subnetwork containing the alarming resource. The AlarmList will be contained under this ancestor MOI.

- Send a notifyNewAlarm notification to each MnS consumer subscribed. The notification will carry the same data as stored in the new attribute element to each subscribed consumer. Sending of the notification is dependent on the attributes set in the NtfSubscriptionControl MOI e.g., notificationFilter.

Figure 7.8-1: Notifying new alarms – sequence diagram

### 7.8.2	Notifying changed alarms

This clause describes use-cases "Notify changed alarm", "Notify cleared alarm", " Notify acknowledgement state change of alarm", “Notify correlated notifications changed”, “Notify comments changed”.

The alarmRecord can be changed due to an internal request or by the consumer sending e.g., a modifyMOIAttributes request depending on the isWritable property. An existing alarmRecord can be identified based on the matching-criteria-attributes values.

When an attribute field of the alarmRecord has changed and if the AlarmList.administrativeState=UNLOCKED, a the MnS producer will

- Update the alarmRecord for the alarm accordingly. This might involve deleting the alarmRecord. If the alarmRecord is not deleted ackTime, ackUserId and ackSystemId are updated to contain no information; ackState is updated to "UNACKNOWLEDGED”.

- Send a notification to each MnS consumer subscribed. The notification will carry the data as updated in the alarmRecord to each subscribed consumer. Sending of the notification is dependent on the attributes set in the NtfSubscriptionControl MOI e.g., notificationFilter. Depending on which field changed different notifications will be sent. If the changed field is

  - perceivedSeverity then either notifyChangedAlarmGeneral, (notifyChangedAlarm) or notifyClearedAlarm is sent. notifyChangedAlarm may only be sent if the new value of perceivedSeverity is not CLEARED. notifyClearedAlarm may be sent only if the new value of perceivedSeverity is CLEARED.

  - correlatedNotifications then notifyCorrelatedNotificationChanged will be sent.

  - ackState then notifyAckStateChanged will be sent.

  - commentUserId, commentSystemId, commentText, commentTime then notifyComments will be sent.

  - backedUpStatus, backUpObject, trendIndication, thresholdInfo, stateChangeDefinition, monitoredAttributes, proposedRepairActions, additionalText, additionalInformation, serviceUser, serviceProvider or securityAlarmDetector then notifyChangedAlarmGeneral will be sent.

## 7.9	Retrieving alarms

The MnS consumer might read the list of alarms by retrieving data from the AlarmList MOI using the getMOIAttributes operation. getMOIAttributes allows scoping and filtering to retrieve only a subset of alarmRecords.

## 7.10	Notifying a potentially faulty alarm list

When the MnS producer detects that the AlarmList or a part of it becomes potentially unreliable the producer will

- Update AlarmList.unreliableAlarmScope to indicate the part of the AlarmList’s scope that may not be reliable.

- Send the notifyPotentialFaultyAlarmList notification to the consumer. The notification is usually followed by the notification notifyAlarmListRebuilt when the AlarmList is once again reliable.

## 7.11	Notifying alarm list is rebuilt

MnS producer has cold-started, initialized, re-initialized, rebooted or discovered that the AlarmList is faulty and it has initiated the procedure to rebuild its AlarmList. When the MnS producer rebuilds the AlarmList (completely or partially) it will

- Update the AlarmList. This should involve updating AlarmList.unreliableAlarmScope and might involve adding, deleting or updating one or more alarmRecord elements. Individual notifications about new or changed alarmRecords will not be sent in this case.

- Send a notifyAlarmListRebuilt notification to each MnS consumer subscribed (see clause 7.2). Sending of the notification is dependent on the attributes set in the NtfSubscriptionControl MOI e.g., notificationFilter. The consumer is expected to read the AlarmList as described in clause 7.8.

Figure 7.11-1: Notify alarm list is rebuilt – sequence diagram

## 7.12	Acknowledging alarms

The MnS consumer may optionally support acknowledging alarms by the MnS consumer.

If the functionality is supported, the MnS consumer may acknowledge an alarm by updating the attribute fields alarmRecords.ackUserId., alarmRecords.ackSystemId, alarmRecords.ackState e.g., by using the modifyMOIAttributes operation.

Acknowledging an alarm may result in its removal from the AlarmList.

The MnS producer will send a notifyAckStateChanged to all subscribed consumers.

## 7.13	Clearing alarms

If the MnS producer supports any ADMC alarms, it is possible for the MnS consumer to clear these alarms. ADAC alarms cannot be cleared by the consumer.

If the functionality is supported, the MnS consumer may clear an alarm by updating  the attribute fields alarmRecords.clearUserId., alarmRecords.clearSystemId  e.g., by using the operation modifyMOIAttributes.

Clearing an alarm may result in its removal from the AlarmList.

The MnS producer will send a notifyClearedAlarm to all subscribed consumers.

## 7.14	Virtualized resource alarm correlation

The MnS consumer may collect alarm information about virtual resources by using the services of the Fault Management MnS. Suitably defined alarms may carry virtualization related information e.g. VNF/VNFC instance identifiers. TS 28.516[4] describes in more detail how to handle alarms related to virtual resources and how to correlate alarms related to virtual resources and VNF applications.

# 8	Model

## 8.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| 3GPP TS 28.532 [2], notification, notifyMOICreation | notifyMOICreation |
| 3GPP TS 28.532 [2], notification, notifyMOIDeletion | notifyMOIDeletion |
| 3GPP TS 28.532 [2], notification, notifyMOIAttributeValueChanges | notifyMOIAttributeValueChanges |
| 3GPP TS 28.532 [2], notification, notifyMOIChanges | notifyMOIChanges |
| 3GPP TS 28.622 [5], IOC, Top | Top |
| 3GPP TS 28.622 [5], IOC, ManagedElement | ManagedElement |
| 3GPP TS 28.622 [5], IOC, SubNetwork | SubNetwork |
| 3GPP TS 28.622 [5], IOC, NtfSubscriptionControl | NtfSubscriptionControl |
| 3GPP TS 28.622 [5], IOC, HeartbeatControl | HeartbeatControl |
| 3GPP TS 28.622 [5], data type, ThresholdInfo | ThresholdInfo |

## 8.2	Class diagrams

### 8.2.1	Relationships

This clause depicts the set of classes (e.g. IOCs) implemented by Fault Management. This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

Figure 8.2.1-6: FM control NRM fragment

### 8.2.2	Inheritance

This clause depicts the inheritance relationships.

Figure 8.2.2-5: FM control NRM fragment

## 8.3	Class definitions

### 8.3.1	AlarmRecord <<dataType>>

#### 8.3.1.1	Definition

An AlarmRecord contains alarm information of an alarmed object instance. A new record is created in the alarm list when an alarmed object instance generates an alarm and no alarm record exists with the same values for objectInstance, alarmType, probableCause and specificProblem. When a new record is created the MnS producer creates an alarmId, that unambiguously identifies an alarm record in the AlarmList.

Alarm records are maintained only for active alarms. Inactive alarms are automatically deleted by the MnS producer from the AlarmList. Active alarms are alarms whose

a)	perceivedSeverity is not "CLEARED", or whose

b)	perceivedSeverity is "CLEARED" and its ackState is not "ACKNOWLEDGED" and alarm acknowledgement by the consumer is supported.

#### 8.3.1.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| alarmId | M | T | F | T | F |
| objectInstance | M | T | F | T | F |
| notificationId | M | T | F | F | F |
| alarmRaisedTime | M | T | F | F | F |
| alarmChangedTime | O | T | F | F | F |
| alarmClearedTime | M | T | F | F | F |
| alarmType | M | T | F | T | F |
| probableCause | M | T | F | T | F |
| specificProblem | O | T | F | T | F |
| perceivedSeverity | M | T | T (note 4) | F | F |
| backedUpStatus | O | T | F | F | F |
| backUpObject | O | T | F | F | F |
| trendIndication | O | T | F | F | F |
| thresholdInfo | O | T | F | F | F |
| stateChangeDefinition | O | T | F | F | F |
| monitoredAttributes | O | T | F | F | F |
| proposedRepairActions | O | T | F | F | F |
| additionalText | O | T | F | F | F |
| additionalInformation | O (see note 3) | T | F | F | F |
| rootCauseIndicator | O | T | F | F | F |
| ackTime | M | T | F | F | F |
| ackUserId | M | T | T (see note 5) | F | F |
| ackSystemId | O | T | T (see note 5) | F | F |
| ackState | M | T | T (see note 5) | F | F |
| clearUserId | O (see note 1) | T | T | F | F |
| clearSystemId | O (see note 1) | T | T | F | F |
| serviceUser | O (see note 2) | T | F | F | F |
| serviceProvider | O (see note 2) | T | F | F | F |
| securityAlarmDetector | O (see note 2) | T | F | F | F |
| comments | O | T | T | F | F |
| correlatedNotifications | O | T | F | F | F |
| NOTE 1: These attributes and qualifiers are applicable only if producer supports alarm acknowledgement by the consumer. The attributes are writable only for ADMC alarms. NOTE 2: These attributes are supported if the producer emits notifyNewAlarm that carries security alarm information. NOTE 3: This attribute is supported to carry vendor specific information. NOTE 4: This isWritable property is True only for ADMC alarms. NOTE 5: This isWritable property is True only if producer supports the consumer to acknowledge alarms. |  |  |  |  |  |

#### 8.3.1.3	Attribute constraints

| Name | Definition |
| --- | --- |
| ackTime ackUserId ackSystemId ackState | These attributes shall be present only if the producer supports acknowledgement by the consumer. |
| clearUserId clearSystemId | If the producer does not supports acknowledgement by the consumer the attribute is not readable or writable because the alarmRecord is deleted immeditely on clearing the alarm, however the same information will be available in the notifyClearedAlarm notification. |
| comments | This attribute shall be present only if the producer supports comments on alarms. |
| correlatedNotifications rootCauseIndicator | These attributes shall be present only if the producer supports correlated notifications |

#### 8.3.1.4	Notifications

See clause 8.5.

### 8.3.2	AlarmList

#### 8.3.2.1	Definition

The AlarmList represents the capability to store and manage alarm records. It can be name-contained by SubNetwork or ManagedElement. The management scope of an AlarmList is defined by all descendant objects of the base managed object, which is the object name-containing the AlarmList, and the base object itself. AlarmList MOIs should not be contained by a ManagedElement MOI if the ManagedElement MOI is contained in a Subnetwork that also contains an AlarmList MOI: multiple AlarmList MOIs with overlapping scopes should be avoided. In  case an AlarmList is created under a ManagedElement that is also contained under a SubNetwork which also has an AlarmList child MOI, alarms in scope of that ManagedElement shall only be handled by the ManagedElement’s AlarmList and shall not be visible in the SubNetwork’s AlarmList.

AlarmList instance(s) are created by the system or are pre-installed. They cannot be created nor deleted by MnS consumers.

An instance of SubNetwork or ManagedElement has at most one name-contained instance of AlarmList.

When the alarm list is locked or disabled, its attributes (except the administrativeState/operationalState) may contain any unreliable data. No alarm notifications are sent by the MnS producer.

#### 8.3.2.2	Attributes

The AlarmList IOC includes attributes inherited from Top IOC and the following attributes:

| Attribute Name |  | S | isReadable |  | isWritable |  | isInvariant |  | isNotifyable |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| administrativeState |  | O | T |  | T |  | F |  | T |  |
| operationalState |  | M | T |  | F |  | F |  | T |  |
| numOfAlarmRecords |  | M | T |  | F |  | F |  | F |  |
| lastModification |  | M | T |  | F |  | F |  | F |  |
| alarmRecords |  | M | T |  | T |  | F |  | F |  |
| unreliableAlarmScope |  | O | T |  | F |  | F |  | F |  |

#### 8.3.2.3	Attribute constraints

None

#### 8.3.2.4	Notifications

The common notifications defined in clause 8.5 are valid for this IOC, without exceptions or additions.

### 8.3.3	AlarmComment <<dataType>>

#### 8.3.3.1	Definition

This data type represents a comment on an alarm.

#### 8.3.3.2	Attributes

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| commentTime | M | T | F | T | F |
| commentUserId | M | T | T | F | F |
| commentSystemId | O | T | F | F | F |
| commentText | M | T | T | F | F |

#### 8.3.3.3	Attribute constraints

None

#### 8.3.3.4	Notifications

See clause 8.5.

### 8.3.4	NameValuePair <<dataType>>

#### 8.3.4.1	Definition

This data type represents a named value, could be used to represent attributes.

#### 8.3.4.2	Attributes

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| name | M | T | T | F | T |
| value | M | T | T | F | T |

#### 8.3.4.3	Attribute constraints

None

#### 8.3.4.4	Notifications

See clause 8.5.

### 8.3.5	AttributeValueChange <<dataType>>

#### 8.3.5.1	Definition

This data type represents an attribute’s new and old value.

#### 8.3.5.2	Attributes

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| newValue | M | T | T | F | T |
| oldValue | O | T | T | F | T |

#### 8.3.5.3	Attribute constraints

None

#### 8.3.5.4	Notifications

See clause 8.5.

### 8.3.6	CorrelatedNotification <<dataType>>

#### 8.3.6.1	Definition

The sourceObjectInstance attribute of CorrelatedNotification identifies one MonitoredEntity. For the MonitoredEntity identified, a set of notification identifiers is also identified. One or more CorrelatedNotification instances can be included in an AlarmRecord. In this case, the information of the AlarmRecord is said to be correlated to information carried in the notifications identified by the CorrelatedNotification instances. See further definition of correlated notification in ITU-T Recommendation X.733 [8], clause 8.1.2.9.

The notification identified by the CorrelatedNotification, as defined in ITU-T and used here, can carry all types of information and is not restricted to carrying alarm information only. For example, a notification, identified by the CorrelatedNotification, can indicate a managed instance attribute value change. In this case, the information of the AlarmRecord is said to be correlated to the managed instance attribute value change event.

If a CorrelatedNotification references an alarm (e.g., by referencing the notificationId of a notifyNewAlarm notification), the alarmRecord for that alarm may or may not exist in the AlarmList. For example, the alarm may have been acknowledged and cleared and therefore, removed from the AlarmList.

#### 8.3.6.2	Attributes

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sourceObjectInstance | M | T | F | F | F |
| notificationIdSet | M | T | F | F | F |

#### 8.3.6.3	Attribute constraints

None

#### 8.3.6.4	Notifications

See clause 8.5.

## 8.4	Attribute definitions

### 8.4.1	Attribute properties

The following table defines the properties of attributes specified in the present document.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| objectClass | Class of a managed object instance. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| objectInstance | Managed object instance identified by its DN. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| systemDN | Distinguished Name (DN) of an MnSAgent. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| administrativeState | Administrative state of a managed object instance. The administrative state describes the permission to use or prohibition against using the object instance. The administrative state is set by the MnS consumer. allowedValues: LOCKED, UNLOCKED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: LOCKED isNullable: False |
| operationalState | Operational state of manged object instance. The operational state describes if an object instance is operable ("ENABLED") or inoperable ("DISABLED"). This state is set by the object instance or the MnS producer and is hence READ-ONLY. allowedValues: ENABLED, DISABLED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: DISABLED isNullable: False |
| alarmRecords | List of alarm records | type: AlarmRecord multiplicity: * isOrdered: False isUnique: True default value: None isNullable: False |
| numOfAlarmRecords | Number of alarm records in the AlarmList. allowedValues: Non-negative numbers. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| lastModification | Time an alarm record was modified the last time | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| unreliableAlarmScope | Identifies, the part of the alarm scope that may not be reliable. If this parameter is equal to the instance carried in systemDN, then all AlarmRecord instances in the AlarmList may not be reliable. If this parameter is equal to some instance represented by MonitoredEntity, then only AlarmRecord related to this instance and its descendants may not be reliable. | type: DN multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| alarmId | Identifies an AlarmRecord in the AlarmList. The value is unique within the AlarmList MOI. | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| notificationId | The Id of the last notification updating the AlarmRecord. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| alarmRaisedTime | Date and time the alarm was raised. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| alarmChangedTime | It indicates the last date and time when the AlarmInformation is changed by the alarmed resource. Changes to AlarmInformation caused by invocations of the management service consumer would not change this date and time. | type: DateTime multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| alarmClearedTime | Date and time the alarm was cleared. | type: DateTime multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| alarmType | It indicates the type of alarm. Communications Alarm: An alarm of this type is associated with the procedure and/or process required conveying information from one point to another (ITU-T Recommendation X.733 [8]). Processing Error Alarm: An alarm of this type is associated with a software or processing fault (ITU T Recommendation X.733 [8]). Environmental Alarm: An alarm of this type is associated with a condition related to an enclosure in which the equipment resides (ITU-T Recommendation X.733 [8]). Quality of Service Alarm: An alarm of this type is associated with degradation in the quality of a service (ITU T Recommendation X.733 [8]). Equipment Alarm: An alarm of this type is associated with an equipment fault (ITU-T Recommendation X.733 [8]). Integrity Violation: An indication that information may have been illegally modified, inserted or deleted. Operational Violation: An indication that the provision of the requested service was not possible due to the unavailability, malfunction or incorrect invocation of the service. Physical Violation: An indication that a physical resource has been violated in a way that suggests a security attack. Security Service or Mechanism Violation: An indication that a security attack has been detected by a security service or mechanism. Time Domain Violation: An indication that an event has occurred at an unexpected or prohibited time. Allow values: COMMUNICATIONS_ALARM, QUALITY_OF_SERVICE_ALARM, PROCESSING_ERROR_ALARM, EQUIPMENT_ALARM, ENVIRONMENTAL_ALARM, INTEGRITY_VIOLATION, OPERATIONAL_VIOLATION, PHYSICAL_VIOLATION, SECURITY_SERVICE_OR_MECHANISM_VIOLATION, TIME_DOMAIN_VIOLATION | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| probableCause | It qualifies alarm and provides further information than alarmType. This attribute value shall be single-value and of simple type such as integer or string. See Annex A for a complete listing. | type: string or integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| specificProblem | It provides further refinement to the probableCause. This attribute value shall be single-valued and of simple type such as integer or string. See definition in ITU-T Recommendation X.733 [8] clause 8.1.2.2. | type: string or integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| perceivedSeverity | It indicates the relative level of urgency for operator attention. allowedValues: CRITICAL, MAJOR, MINOR, WARNING, INDETERMINATE, CLEARED | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| backedUpStatus | It indicates if an object (the MonitoredEntity) has a back up. See definition in ITU-T Recommendation X.733 [8] clause 8.1.2.4. | type: boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| backUpObject | Backup object of the alarmed object as defined in ITU-T Rec. X. 733 [8] | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trendIndication | It indicates if some observed condition is getting better, worse, or not changing. AllowedValues: MORE_SEVERE, NO_CHANGE, LESS_SEVERE | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| thresholdInfo | It indicates the crossed threshold information such as: - The identifier of the monitored attribute whose value has crossed a threshold, - The threshold settings, - The observed value that have crossed a threshold, etc. See definition in ITU-T Recommendation X.733 [8] clause 8.1.2.7. See also for information in TS 32.401 [12] clause 5.6. | type: ThresholdInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| stateChangeDefinition | It indicates attribute value changes associated with the alarm for state attributes of the monitored entity (state transitions). The change is reported with the name of the state attribute, the new value and an optional old value. See definition in ITU-T Recommendation X.733 [8] clause 8.1.2.11. The content of the attribute is a list of attributeNames and attributeValues. AttibuteValues may be complex types. | type: AttributeValueChange multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| monitoredAttributes | It indicates attributes of the monitored entity and their values at the time the alarm occurred that are of interest for the alarm report. How these attributes are chosen is outside of the scope of the present document. See definition in ITU-T Recommendation X.733 [8] clause 8.1.2.11. The content of the attribute is a list of attributeNames and attributeValues. AttibuteValues may be complex types. | type: NameValuePair multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| proposedRepairActions | Used if the cause is known and the system being managed can suggest one or more solutions to fix the problem causing the alarm as defined in ITU-T Rec. X. 733 [8] | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| additionalText | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [8]. | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| additionalInformation | This attribute when present allows the inclusion of a set of vendor specific alarm information in the alarm. A specific condition for this optional population is when an alarm presented by the Management System (e.g. via the user interface) has different values of perceived severity, and / or alarm type, compared with the values presented to the Itf-N. Any other uses of additional information on the alarm and its semantics is outside the scope of the present document The content of the attribute is a list of attributeNames and string attributeValues. | type: NameValuePair multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| rootCauseIndicator | It indicates that this AlarmInformation is the root cause of the events captured by the notifications whose identifiers are in the related CorrelatedNotification instances. | type: boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ackTime | It identifies the time when the alarm has been acknowledged or unacknowledged the last time, i.e. it registers the time when ackState changes. | type: DateTime multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ackUserId | It identifies the last user who has changed the acknowledgement state. | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ackSystemId | It identifies the system that last changed the ackState of an alarm, i.e. acknowledged or unacknowledged the alarm. | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ackState | It identifies the acknowledgement state of an alarm. AllowedValues: ACKNOWLEDGED, UNACKNOWLEDGED | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| clearUserId | It carries the identity of the user who invokes the clearAlarms operation. | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| clearSystemId | Identifier of a system clearing an alarm | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| serviceUser | It identifies the service-user whose request for service provided by the serviceProvider led to the generation of the security alarm. | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| serviceProvider | It identifies the service-provider whose service is requested by the serviceUser and the service request provokes the generation of the security alarm. | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| securityAlarmDetector | It carries the identity of the detector of the security alarm. | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| comments | List of comments and data about the comments. | type: AlarmComment multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| correlatedNotifications | List of correlated notifications. | type: CorrelatedNotification multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| commentTime | Date and Time the comment was last updated. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| commentUserId | It carries the identification of the user who made the comment. | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| commentSystemId | It carries the identification of the system (Management System) from which the comment is made. That system supports the user that made the comment. | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| commentText | It carries the textual comment. | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CorrelatedNotification.sourceObjectInstance | It identifies one MonitoredEntity. It is unique within a multivalue attribute based on the CorrelatedNotification data type. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CorrelatedNotification.notificationIdSet | A list of correlated notificationIds. | type: integer multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NameValuePair.name | Name of a value e.g., an attribute name | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NameValuePair.value | Value in a name value pair | type: any multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AttributeValueChange.newValue | New or current value | type: NameValuePair multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AttributeValueChange.oldValue | Previous value | type: NameValuePair multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NOTEs: none |  |  |

### 8.4.2	Constraints

None

## 8.5	Common notifications

### 8.5.1	Alarm notifications

This clause presents a list of notifications, defined in clause 9, that a MnS consumer can receive. The notification header attribute objectClass/objectInstance captures the DN of an instance of an IOC defined in the present document.

| Name | Notes |
| --- | --- |
| notifyNewAlarm |  |
| notifyClearedAlarm |  |
| notifyAlarmListRebuilt |  |
| notifyChangedAlarmGeneral |  |
| notifyChangedAlarm |  |
| notifyCorrelatedNotificationChanged |  |
| notifyAckStateChanged |  |
| notifyComments |  |
| notifyPotentialFaultyAlarmList |  |

### 8.5.2	Configuration notifications

This clause presents a list of notifications, defined in [2], that a MnS consumer can receive. The notification header attribute objectClass/objectInstance, captures the DN of an instance of an IOC defined in the present document.

| Name | Notes |
| --- | --- |
| notifyMOICreation |  |
| notifyMOIDeletion |  |
| notifyMOIAttributeValueChanges |  |
| notifyMOIChanges |  |

# 9		Notifications

## 9.1	Overview

Clause 8.5.1 defines a list of notifications defined and implemented by Fault Management.

## 9.2	notifyNewAlarm

### 9.2.1	Definition

This notification is generated by the MnS producer when a new alarm is raised and an AlarmRecord is added to the AlarmList. The notification parameters depend on the alarmType and are different for non-security and security alarms.

### 9.2.2	Input parameters

If the alarmType is “Communications Alarm", "Processing Error Alarm", "Environmental Alarm". "Quality Of Service Alarm" or "Equipment Alarm" the alarm is considered to be non-security related. If the alarmType is  "Integrity Violation", "Operational Violation", "Physical Violation", "Security Service or Mechanism Violation" or "Time Domain Violation" the alarm is considered to be security related.

Table 9.2.2-1: Input parameters for notifyNewAlarm

| Parameter Name | S | Matching Information/ Information Type / Legal Values | Description |
| --- | --- | --- | --- |
| objectClass | M | String ClassName of the object identified by objectInstance. |  |
| objectInstance | M | alarmRecord.objectInstance DN of the MonitoredEntity that is the source of the alarm |  |
| notificationId | M | This is an identifier for the notification, which may be used to correlate notifications. | The identifier of the notification shall be chosen to be unique across all notifications of a particular managed object instance throughout the time that correlation is significant, it uniquely identifies the notification from other notifications generated by the subject MOI. |
| notificationType | M | "notifyNewAlarm" |  |
| eventTime | M | alarmRecord.alarmRaisedTime |  |
| systemDN | M | It shall carry the DN of management service providers; the DN of an MnsAgent MOI [5]. |  |
| alarmId | M | alarmRecord.alarmId |  |
| alarmType | M | alarmRecord.alarmType |  |
| probableCause | M | alarmRecord.probableCause |  |
| perceivedSeverity | M | alarmRecord.perceivedSeverity |  |
| specificProblem | O | alarmRecord.specificProblem |  |
| backedUpStatus | CO | alarmRecord.backedUpStatus | Used only in non-security notifications. |
| backUpObject | CO | alarmRecord.backUpObject | Used only in non-security notifications. |
| trendIndication | CO | alarmRecord.trendIndication | Used only in non-security notifications. |
| thresholdInfo | CO | alarmRecord.thresholdInfo | Used only in non-security notifications. |
| correlatedNotifications | O | alarmRecord.correlatedNotifications |  |
| stateChangeDefinition | CO | alarmRecord.stateChangeDefinition | Used only in non-security notifications. |
| monitoredAttributes | CO | alarmRecord.monitoredAttributes | Used only in non-security notifications. |
| proposedRepairActions | CO | alarmRecord.proposedRepairActions | Used only in non-security notifications. |
| additionalText | O | alarmRecord.additionalText |  |
| additionalInformation | O | alarmRecord.additionalInformation |  |
| rootCauseIndicator | O | alarmRecord.rootCauseIndicator |  |
| serviceUser | CM | alarmRecord.securityServiceUser | Used only in security notifications. This may contain no information if the identify of the service-user (requesting the service) is not known. |
| serviceProvider | CM | alarmRecord.securityServiceProvider | Used only in security notifications. This shall always identify the service-provider receiving a service request, from serviceUser, that provokes the security alarm. |
| securityAlarmDetector | CM | alarmRecord.securityAlarmDetector | Used only in security notifications. This may contain no information if the detector of the security alarm is the serviceProvider. |

## 9.3	notifyClearedAlarm

### 9.3.1	Definition

This notification is generated by the MnS producer when the perceivedSeverity of an existing AlarmRecord changes to "CLEARED"; the AlarmRecord may be removed when sending the notification.

### 9.3.2	Input parameters

### Table 9.3.2-1: Input parameters for notifyClearedAlarm

| Parameter Name | S | Matching Information/ Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | See clause 9.2.1 |  |
| objectInstance | M | alarmRecord.objectInstance DN of the MonitoredEntity that is the source of the alarm |  |
| notificationId | M | See clause 9.2.1 |  |
| notificationType | M | "notifyClearedAlarm" |  |
| eventTime | M | alarmRecord.alarmClearedTime |  |
| systemDN | M | See clause 9.2.1 |  |
| alarmId | M | alarmRecord.alarmId |  |
| alarmType | M | alarmRecord.alarmType |  |
| probableCause | M | alarmRecord.probablaCause |  |
| perceivedSeverity | M | alarmRecord.perceivedSeverity | Value shall be "CLEARED" |
| correlatedNotifications | O | alarmRecord.correlatedNotifications |  |
| clearUserId | O | alarmRecord.clearUserId | This parameter shall be present if the AlarmRecord is cleared by the consumer. |
| clearSystemId | O | alarmRecord.clearSystemId | This parameter shall be present if clearUserId is present |

## 9.4	notifyChangedAlarmGeneral

### 9.4.1	Definition

This notification is generated by the MnS producer when one or more of the following attributes of an AlarmRecord instance in the AlarmList changes its value: perceivedSeverity, backedUpStatus, backUpObject, trendIndication, thresholdInfo, stateChangeDefinition, monitoredAttributes, proposedRepairActions, additionalText, additionalInformation, serviceUser, serviceProvider or securityAlarmDetector. From the attributes listed above, only those that changed value shall be included in the notification.

The notification parameters depend on the alarmType and are different for non-security and security alarms. If the alarmType is  "Communications Alarm", "Processing Error Alarm", "Environmental Alarm". "Quality Of Service Alarm" or "Equipment Alarm" the alarm is considered to be non-security related. If  the alarmType is  "Integrity Violation", "Operational Violation", "Physical Violation", "Security Service or Mechanism Violation" or "Time Domain Violation" the alarm is considered to be security related.

### 9.4.2	Input parameters

### Table 9.4.2-1: Input parameters for notifyChangedAlarmGeneral

| Parameter Name | S | Matching Information/ Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | See clause 9.2.1 |  |
| objectInstance | M | alarmRecord.objectInstance DN of the MonitoredEntity that is the source of the alarm |  |
| notificationId | M | See clause 9.2.1 |  |
| notificationType | M | "notifyChangedAlarmGeneral" |  |
| eventTime | M | alarmRecord.alarmChangedTime |  |
| systemDN | M | See clause 9.2.1 |  |
| alarmId | M | alarmRecord.alarmId |  |
| alarmType | M | alarmRecord.alarmType |  |
| probableCause | M | alarmRecord.probableCause |  |
| specificProblem | O | alarmRecord.specificProblem |  |
| perceivedSeverity | O | alarmRecord.perceivedSeverity |  |
| backedUpStatus | CO | alarmRecord.backedUpStatus | Used only in non-security notifications. |
| backUpObject | CO | alarmRecord.backUpObject | Used only in non-security notifications. |
| trendIndication | CO | alarmRecord.trendIndication | Used only in non-security notifications. |
| thresholdInfo | CO | alarmRecord.thresholdInfo | Used only in non-security notifications. |
| correlatedNotifications | O | alarmRecord.correlatedNotifications |  |
| stateChangeDefinition | CO | alarmRecord.stateChange | Used only in non-security notifications. |
| monitoredAttributes | CO | alarmRecord.monitoredAttributes | Used only in non-security notifications. |
| proposedRepairActions | CO | alarmRecord.proposedRepairActions | Used only in non-security notifications. |
| additionalText | O | alarmRecord.additionalText |  |
| additionalInformation | O | alarmRecord.additionalInformation |  |
| rootCauseIndicator | O | alarmRecord.rootCauseIndicator |  |
| serviceUser | C | alarmRecord.securityServiceUser | Used only in security notifications. This may contain no information if the identify of the service-user (requesting the service) is not known. |
| serviceProvider | C | alarmRecord.securityServiceProvider | Used only in security notifications. This shall always identify the service-provider receiving a service request, from serviceUser, that provokes the security alarm. |
| securityAlarmDetector | C | alarmRecord.securityAlarmDetector | Used only in security notifications. This may contain no information if the detector of the security alarm is the serviceProvider. |
| changedAlarmAttributes | O | LIST OF SEQUENCE <AttributeName, OldAttributeValue> | The changed alarm attributes (name/value pairs) (with old values). |

## 9.5	notifyAlarmListRebuilt

### 9.5.1	Definition

This notification is generated by the MnS producer when the AlarmList has been completely or partially rebuilt.

### 9.5.2	Input parameters

### Table 9.5.2-1: Input parameters for notifyAlarmListRebuilt

| Parameter Name | S | Matching Information/ Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | See clause 9.2.1 |  |
| objectInstance | M | DN | Identifies the part of the alarm scope that has been rebuilt. If this parameter is equal to the instance carried in systemDN, then all AlarmRecord instances in the AlarmList may have been rebuilt. If this parameter is equal to some other instance, then only alarmRecords related to this instance and its descendants may have been rebuilt. |
| notificationId | M | See clause 9.2.1. |  |
| notificationType | M | "notifyAlarmListRebuilt" |  |
| eventTime | M | DateTime | The time when the alarm list has been rebuilt. |
| systemDN | M | See clause 9.2.1 |  |
| reason | M | String "System-NE communication error", "System restarts", "indeterminate". Other values can be added. | The reason why the system has rebuilt the AlarmList. This may carry different reasons than that carried by the immediate previous notifyPotentialFaultyAlarmList. |
| alarmListAlignmentRequirement | O | "alignmentRequired", "alignmentNotRequired". | Indicates whether the AlarmList was changed during the rebuild. If the AlarmList was changed, alignment is required, the consumer is recommended to re-read it. |

## 9.6	notifyChangedAlarm

### 9.6.1	Definition

This notification is generated by the MnS producer when the perceivedSeverity of an existing AlarmRecord changes (except to the value "CLEARED").

The notification is deprecated, use notifyChangedAlarmGeneral instead.

### 9.6.2	Input parameters

### Table 9.6.2-1: Input parameters for notifyChangedAlarm

| Parameter Name | S | Matching Information/ Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | See clause 9.2.1 |  |
| objectInstance | M | alarmRecord.objectInstance DN of the MonitoredEntity that is the source of the alarm |  |
| notificationId | M | See clause 9.2.1 |  |
| notificationType | M | "notifyChangedAlarm" |  |
| eventTime | M | alarmRecord.alarmChangedTime |  |
| systemDN | M | See clause 9.2.1 |  |
| alarmId | M | alarmRecord.alarmId |  |
| alarmType | M | alarmRecord.alarmType |  |
| probableCause | M | alarmRecord.probableCause |  |
| perceivedSeverity | M | alarmRecord.perceivedSeverity |  |

## 9.7	notifyCorrelatedNotificationChanged

### 9.7.1	Definition

This notification is generated by the MnS producer when the set of correlatedNotifications is created, updated or deleted.

### 9.7.2	Input parameters

### Table 9.7.2-1: Input parameters for notifyCorrelatedNotificationChanged

| Parameter Name | S | Matching Information/ Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | See clause 9.2.1 |  |
| objectInstance | M | alarmRecord.objectInstance DN of the MonitoredEntity that is the source of the alarm |  |
| notificationId | M | See clause 9.2.1. |  |
| notificationType | M | "notifyCorrelatedNotificationChanged" |  |
| eventTime | M | alarmRecord.alarmChanedTime It carries the time when the CorrelatedNotification is created, updated or deleted. |  |
| systemDN | M | See clause 9.2.1 |  |
| alarmId | M | alarmRecord.alarmId |  |
| correlatedNotifications | M | alarmRecord.correlatedNotifications |  |
| rootCauseIndicator | O | alarmRecord.rootCauseIndicator |  |

## 9.8	notifyAckStateChanged

### 9.8.1	Definition

This notification is generated by the MnS producer when the acknowledgement state of an alarm changes from "UNACKNOWLEDGED" to "ACKNOWLEDGED" or back from "ACKNOWLEDGED" to "UNACKNOWLEDGED".

### 9.8.2	Input parameters

Table 9.8.2-1: Input parameters for notifyAckStateChanged

| Parameter Name | S | Matching Information/ Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | See clause 9.2.1 |  |
| objectInstance | M | alarmRecord.objectInstance DN of the MonitoredEntity that is the source of the alarm |  |
| notificationId | M | See clause 9.2.1 |  |
| notificationType | M | "notifyAckStateChanged" |  |
| eventTime | M | alarmRecord.ackTime |  |
| systemDN | M | See clause 9.2.1 |  |
| alarmId | M | alarmRecord.alarmId |  |
| alarmType | M | alarmRecord.alarmType |  |
| probableCause | M | alarmRecord.probableCause |  |
| perceivedSeverity | M | alarmRecord.perceivedSeverity |  |
| ackState | M | alarmRecord.ackState |  |
| ackUserId | M | alarmRecord.ackUserId |  |
| ackSystemId | O | alarmRecord.ackSystemId |  |

## 9.9	notifyComments

### 9.9.1	Definition

This notification is generated by the MnS producer when a Comment instance is updated in an AlarmRecord instance in the AlarmList.

### 9.9.2	Input parameters

### Table 9.9.2-1: Input parameters for notifyComments

| Parameter Name | S | Matching Information/ Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | See clause 9.2.1 |  |
| objectInstance | M | alarmRecord.objectInstance DN of the MonitoredEntity that is the source of the alarm |  |
| notificationId | M | See clause 9.2.1 |  |
| notificationType | M | "notifyComments" |  |
| eventTime | M | alarmRecord.alarmChangedTime | The time the comment was updated |
| systemDN | M | See clause 9.2.1 |  |
| alarmId | M | alarmRecord.alarmId |  |
| alarmType | M | alarmRecord.alarmType |  |
| probableCause | M | alarmRecord.probableCause |  |
| perceived Severity | M | alarmRecord.perceivedSeverity |  |
| comments | M | The Comment instances related to this AlarmRecord. Type: AlarmComment |  |

## 9.10	notifyPotentialFaultyAlarmList

### 9.10.1	Definition

This notification is generated by the MnS producer when the MnS producer looses confidence in the integrity of its alarm list.

The MnS producer may then rebuild the faulty alarm list. When the alarm List is rebuilt or confidence in the existing alarm list is re-established the MnS producer should generate a notifyAlarmListRebuilt notification.

The parameters objectClass and objectInstance are used to specify if the complete alarm list is unreliable or only parts thereof.

### 9.10.2	Input parameters

Table 9.10.2-1: Input parameters for notifyPotentialFaultyAlarmList

| Parameter Name | S | Matching Information/ Information Type / Legal Values |  | Comment |  |
| --- | --- | --- | --- | --- | --- |
| objectClass | M | See clause 9.2.1 |  |  |  |
| objectInstance | M | It identifies the instance identified by systemDN or an instance of MonitoredEntity. |  | Identifies, together with the objectClass parameter, the part of the alarm scope that may be unreliable. If this parameter is equal to the instance carried in systemDN, then all AlarmRecord instances in the AlarmList may be unreliable. If this parameter is equal to some other instance, then only AlarmRecords related to this instance and its descendants may be unreliable. |  |
| notificationId | M | T See clause 9.2.1. |  |  |  |
| notificationType | M | "notifyPotentialFaultyAlarmList" |  |  |  |
| eventTime | M | DateTime |  | Time when the MnS producer lost confidence in the integrity of the alarm list |  |
| systemDN | M | See clause 9.2.1 |  |  |  |
| reason | M | "serviceprovider-NE communication error", " serviceprovider restarts", "indeterminate". Other values can be added. |  | Reason why the MnS producer has to rebuild its AlarmList. |  |

# 10	Stage 3 – Solution Sets

## 10.1		YANG Definitions

The YANG-Netconf solution set uses the same notifications as OpenAPI, see clause 10.2.

YANG definitions for NRM are specified in Forge [11].

Directory: yang-models

Files:

_3gpp-common-fm.yang

## 10.2	OpenAPI Definitions

OpenAPI/YAML definitions are specified in Forge [11].

Directory: OpenAPI

Files:

TS28111_Fault.yaml

## 10.3	RESTful HTTP-based solution set

### 10.3.1	General

Mapping of Classes and attributes follows the general rules in TS 32.160. See clause 10.3.

### 10.3.2	Mapping of notifications

Principles:

- Only information not documented in the OpenAPI files is included in this clause.

- The following items are documented in the OpenAPI files: HTTP-Method, parameter name and type.

- The name of the parameter is the same in the stage 2 information model (clause 8 and 9) and in the stage 3 OpenAPI definition. Exceptions, if any, are listed below.

Table 12.3.2-1: Mapping of IS notification input parameters to SS equivalents (HTTP POST)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type |
| --- | --- | --- | --- |
| objectClass | request body | href | Uri (see [10]) |
| objectInstance |  |  |  |

## 10.4	RESTful HTTP-based solution set for integration with ONAP VES API

### 10.4.1	General

Mapping of Classes, attributes and notifications is identical to those described in clause  10.4 with the differences listed in the following sub-clauses of 10.5.

### 10.4.2	Mapping of notifications

#### 10.4.2.1	General

The URI of the notification target on the MnS consumer is defined by the notificationRecipientAddress in the NtfSubscriptionControl IOC (See 4.3.22.2 in TS 28.622 [5]. The resource URI is extended with  /eventListener.

#### 10.4.2.2	Resources

Figure 10.2.2.2.2-1 shows the resource structure of the fault supervision data report MnS in the context of its integration with VES Event Listener 7.1.1 [9].

Figure 10.4.2.2-1: Resource URI structure of the fault supervision data report MnS for integration with ONAP VES Event Listener 7.1.1 (Resource structure section) [9]

See also Resource structure section in [9].

### 10.4.3	Integration with ONAP VES

Detailed guidelines for integration of performance assurance MnS notifications with ONAP VES are provided in Annex B of TS 28.532[2].

# Annex A (normative):  Probable Causes

This annex lists probable causes.

Sources of these probable causes are ITU-T Recommendation M.3100 [7], ITU-T Recommendation X.721 [6], ITU-T Recommendation X.733 [8], and ITU-T Recommendation X.736 [13]. In addition, probable causes for wireless systems are listed in ETSI TS 101 251 V6.3.0 (1999-07) [3].

Reserved values for future standardization extensions have been indicated.

Vendor specific value range are allowed between 1000-1999.

NOTE: for backward compatibility, the table is based on 32.111-3.

NOTE: Probable causes that are defined by more than one standard have been removed to ensure unicity.

Table A.1: Probable Causes from ITU-T Recommendation M.3100 [7]

| M.3100 Probable cause (string) | Probable cause (integer) | Event Type |
| --- | --- | --- |
| Indeterminate | 0 | Unknown |
| Alarm Indication Signal (AIS) | 1 | Communications |
| Call Setup Failure | 2 | Communications |
| Degraded Signal | 3 | Communications |
| Far End Receiver Failure (FERF) | 4 | Communications |
| Framing Error | 5 | Communications |
| Loss Of Frame (LOF) | 6 | Communications |
| Loss Of Pointer (LOP) | 7 | Communications |
| Loss Of Signal (LOS) | 8 | Communications |
| Payload Type Mismatch | 9 | Communications |
| NOTE - Values 10 correspond to a duplicated probable cause |  |  |
| Remote Alarm Interface | 11 | Communications |
| Excessive Bit Error Rate (EBER) | 12 | Communications |
| Path Trace Mismatch | 13 | Communications |
| Unavailable | 14 | Communications |
| Signal Label Mismatch | 15 | Communications |
| Loss Of Multi Frame | 16 | Communications |
| Communications Receive Failure | 17 | Communications |
| Communications Transmit Failure | 18 | Communications |
| Modulation Failure | 19 | Communications |
| Demodulation Failure | 20 | Communications |
| NOTE - Values 21-26 correspond to duplicated probable causes |  |  |
| NOTE – Values 27-50 are reserved for M.3100 potential future extensions |  |  |
| Back Plane Failure | 51 | Equipment |
| Data Set Problem | 52 | Equipment |
| Equipment Identifier Duplication | 53 | Equipment |
| External IF Device Problem | 54 | Equipment |
| Line Card Problem | 55 | Equipment |
| Multiplexer Problem | 56 | Equipment |
| NE Identifier Duplication | 57 | Equipment |
| Power Problem | 58 | Equipment |
| Power Supply Failure |  | Equipment |
| Processor Problem | 59 | Equipment |
| Protection Path Failure | 60 | Equipment |
| Receiver Failure | 61 | Equipment |
| Replaceable Unit Missing | 62 | Equipment |
| Replaceable Unit Type Mismatch | 63 | Equipment |
| Synchronization Source Mismatch | 64 | Equipment |
| Terminal Problem | 65 | Equipment |
| Timing Problem | 66 | Equipment |
| Transmitter Failure | 67 | Equipment |
| Trunk Card Problem | 68 | Equipment |
| Replaceable Unit Problem | 69 | Equipment |
| Real Time Clock Failure | 70 | Equipment |
| NOTE – Values 71-80 correspond to duplicated values |  |  |
| Protection Mechanism Failure | 81 | Equipment |
| Protecting Resource Failure | 82 | Equipment |
| NOTE – Values 83-100 are reserved for M.3100 potential future extensions |  |  |
| Air Compressor Failure | 101 | Environmental |
| Air Conditioning Failure | 102 | Environmental |
| Air Dryer Failure | 103 | Environmental |
| Battery Discharging | 104 | Environmental |
| Battery Failure | 105 | Environmental |
| Commercial Power Failure | 106 | Environmental |
| Cooling Fan Failure | 107 | Environmental |
| Engine Failure | 108 | Environmental |
| Fire Detector Failure | 109 | Environmental |
| Fuse Failure | 110 | Environmental |
| Generator Failure | 111 | Environmental |
| Low Battery Threshold | 112 | Environmental |
| Pump Failure | 113 | Environmental |
| Rectifier Failure | 114 | Environmental |
| Rectifier High Voltage | 115 | Environmental |
| Rectifier Low F Voltage | 116 | Environmental |
| Ventilation System Failure | 117 | Environmental |
| Enclosure Door Open | 118 | Environmental |
| Explosive Gas | 119 | Environmental |
| Fire | 120 | Environmental |
| Flood | 121 | Environmental |
| High Humidity | 122 | Environmental |
| High Temperature | 123 | Environmental |
| High Wind | 124 | Environmental |
| Ice Build Up | 125 | Environmental |
| Intrusion Detection | 126 | Environmental |
| Low Fuel | 127 | Environmental |
| Low Humidity | 128 | Environmental |
| Low Cable Pressure | 129 | Environmental |
| Low Temperature | 130 | Environmental |
| Low Water | 131 | Environmental |
| Smoke | 132 | Environmental |
| Toxic Gas | 133 | Environmental |
| NOTE – Values 137-150 are reserved for M.3100 potential future extensions |  |  |
| Storage Capacity Problem | 151 | Processing Error |
| Memory Mismatch | 152 | Processing Error |
| Corrupt Data | 153 | Processing Error |
| Out Of CPU Cycles | 154 | Processing Error |
| Software Environment Problem | 155 | Processing Error |
| Software Download Failure | 156 | Processing Error |
| Loss of Real Time | 157 | Processing Error |
| Reinitialized | 158 | Processing Error |
| NOTE - Values 159-167 correspond to duplicated probable causes |  |  |
| NOTE – Values 168-200 are reserved for M.3100 potential future extensions |  |  |
| NOTE - Values 201-202 correspond to duplicated probable causes |  |  |
| Excessive Error Rate | 203 | Quality of service |
| NOTE - Values 204-207 correspond to duplicated probable causes |  |  |
| NOTE – Values 208-300 are reserved for M.3100 potential future extensions |  |  |

Table A.2: Probable Causes from ITU-T Recommendation X.721 [6], X.733 [8], X.736 [13]

| X.721/X.733/X.736 Probable Cause (string) | (integer) | Even Type |
| --- | --- | --- |
| Adapter Error | 301 | Equipment |
| Application Subsystem Failure | 302 | Processing error |
| Bandwidth Reduction | 303 | Security Service or Mechanism Violation |
| NOTE - Values 304 correspond to duplicated probable cause |  |  |
| Communication Protocol Error | 305 | Communications |
| Communication Subsystem Failure | 306 | Communications |
| Configuration or Customizing Error | 307 | Processing error |
| Congestion | 308 | Quality of service |
| NOTE - Values 309 correspond to duplicated probable cause |  |  |
| CPU Cycles Limit Exceeded | 310 | Processing error |
| Data Set or Modem Error | 311 | Equipment |
| NOTE - Values 312 correspond to duplicated probable cause |  |  |
| DTE-DCE Interface Error | 313 | Communications |
| NOTE - Values 314 correspond to duplicated probable cause |  |  |
| Equipment Malfunction | 315 | Communications |
| Excessive Vibration | 316 | Integrity Violation |
| File Error | 317 | Environmental |
| NOTE - Values 318-320 correspond to duplicated probable cause |  | Equipment |
| Heating or Ventilation or Cooling System Problem | 321 | Environmental |
| Humidity Unacceptable | 322 | Environmental |
| Input/Output Device Error | 323 | Equipment |
| Input Device Error | 324 | Environmental |
| LAN Error | 325 | Processing error |
| Leak Detection | 326 | Environmental |
| Local Node Transmission Error | 327 | Communications |
| NOTE - Values 328-329 correspond to duplicated probable cause |  |  |
| Material Supply Exhausted | 330 | Environmental |
| NOTE - Values 331 correspond to duplicated probable cause |  |  |
| Out of Memory | 332 | Processing error |
| Output Device Error | 333 | Equipment |
| Performance Degraded | 334 | Quality of service |
| NOTE - Values 335 correspond to duplicated probable cause |  |  |
| Pressure Unacceptable | 336 | Operational Violation |
| NOTE - Values 337-338 correspond to duplicated probable cause |  |  |
| Queue Size Exceeded | 339 | Quality of service |
| Receive Failure | 340 | Equipment |
| NOTE - Values 341 correspond to duplicated probable cause |  |  |
| Remote Node Transmission Error | 342 | Communications |
| Resource at or Nearing Capacity | 343 | Quality of service |
| Response Time Excessive | 344 | Quality of service |
| Re-transmission Rate Excessive | 345 | Quality of service |
| Software Error | 346 | Processing error |
| Software Program Abnormally Terminated | 347 | Processing error |
| Software Program Error | 348 | Processing error |
| NOTE - Values 349 correspond to duplicated probable cause |  |  |
| Temperature Unacceptable | 350 | Environmental |
| Threshold Crossed | 351 | Quality of service |
| NOTE - Values 352 correspond to duplicated probable cause |  |  |
| Toxic Leak Detected | 353 | Environmental |
| Transmit Failure | 354 | Equipment |
| NOTE - Values 355 correspond to duplicated probable cause |  |  |
| Underlying Resource Unavailable | 356 | Processing error |
| Version Mismatch | 357 | Processing error |
| NOTE – Values 358-500 are reserved for X.721 potential future extensions |  |  |

Table A.3: Probable Causes for Wireless Systems from ETSI TS 101 251 V6.3.0 (1999-07) [18]

| Wireless Systems (string) | (integer) | Event Type |
| --- | --- | --- |
| A-bis to BTS interface failure | 501 | Equipment |
| A-bis to TRX interface failure | 502 | Equipment |
| Antenna problem | 503 | Equipment |
| Battery breakdown | 504 | Equipment |
| Battery charging fault | 505 | Equipment |
| Clock synchronization problem | 506 | Equipment |
| Combiner problem | 507 | Equipment |
| Disk problem | 508 | Equipment |
| NOTE - Values 509 correspond to duplicated probable cause |  |  |
| Excessive receiver temperature | 510 | Equipment |
| Excessive transmitter output power | 511 | Equipment |
| Excessive transmitter temperature | 512 | Equipment |
| Frequency hopping degraded | 513 | Equipment |
| Frequency hopping failure | 514 | Equipment |
| Frequency redefinition failed | 515 | Equipment |
| Line interface failure | 516 | Equipment |
| Link failure | 517 | Equipment |
| Loss of synchronization | 518 | Equipment |
| Lost redundancy | 519 | Equipment |
| Mains breakdown with battery back-up | 520 | Equipment |
| Mains breakdown without battery back-up | 521 | Equipment |
| Power supply failure | 522 | Equipment |
| Receiver antenna fault | 523 | Equipment |
| NOTE - Values 524 correspond to duplicated probable cause |  |  |
| Receiver multicoupler failure | 525 | Equipment |
| Reduced transmitter output power | 526 | Equipment |
| Signal quality evaluation fault | 527 | Equipment |
| Timeslot hardware failure | 528 | Equipment |
| Transceiver problem | 529 | Equipment |
| Transcoder problem | 530 | Equipment |
| Transcoder or rate adapter problem | 531 | Equipment |
| Transmitter antenna failure | 532 | Equipment |
| Transmitter antenna not adjusted | 533 | Equipment |
| NOTE - Values 534 correspond to duplicated probable cause |  |  |
| Transmitter low voltage or current | 535 | Equipment |
| Transmitter off frequency | 536 | Equipment |
| Database inconsistency | 537 | Processing error |
| File system call unsuccessful | 538 | Processing error |
| Input parameter out of range | 539 | Processing error |
| Invalid parameter | 540 | Processing error |
| Invalid pointer | 541 | Processing error |
| Message not expected | 542 | Processing error |
| Message not initialized | 543 | Processing error |
| Message out of sequence | 544 | Processing error |
| System call unsuccessful | 545 | Processing error |
| Timeout expired | 546 | Processing error |
| Variable out of range | 547 | Processing error |
| Watch dog timer expired | 548 | Processing error |
| Cooling system failure | 549 | Environmental |
| External equipment failure | 550 | Environmental |
| External power supply failure | 551 | Environmental |
| External transmission device failure | 552 | Environmental |
| NOTE - Values 553-560 correspond to duplicated probable cause |  |  |
| Reduced alarm reporting | 561 | Quality of service |
| Reduced event reporting | 562 | Quality of service |
| Reduced logging capability | 563 | Quality of service |
| System resources overload | 564 | Quality of service |
| Broadcast channel failure | 565 | Communications |
| Connection establishment error | 566 | Communications |
| Invalid message received | 567 | Communications |
| Invalid MSU received | 568 | Communications |
| LAPD link protocol failure | 569 | Communications |
| Local alarm indication | 570 | Communications |
| Remote alarm indication | 571 | Communications |
| Routing failure | 572 | Communications |
| SS7 protocol failure | 573 | Communications |
| Transmission error | 574 | Communications |
| NOTE - Values 575 correspond to duplicated probable cause |  |  |
| NOTE - Values 576-700 are reserved for ETSI potential future extensions |  |  |

Table A.4: Probable Causes for Secure Alarm  from M3100 X.736

| Wireless Systems (string) | (integer) | Even Type |
| --- | --- | --- |
| Authentication Failure | 701 | security service or mechanism violation |
| Breach of Confidentiality | 702 | security service or mechanism violation |
| Cable Tamper | 703 | physical violation |
| Delayed Information | 704 | time domain violation |
| Denial of Service | 705 | operational violation |
| Duplicate Information | 706 | integrity violation |
| Information Missing | 707 | integrity violation |
| Information Modification Detected | 708 | integrity violation |
| Information Out of Sequence | 709 | integrity violation |
| Intrusion Detection | 710 | physical violation |
| Key Expired | 711 | time domain violation |
| Non Repudiation Failure | 712 | security service or mechanism violation |
| Out of Hours Activity | 713 | time domain violation |
| Out of Service | 714 | operational violation |
| Procedural Error | 715 | operational violation |
| Unauthorised Access Attempt | 716 | security service or mechanism violation |
| Unexpected Information | 717 | integrity violation |
| Unspecified Reason | 718 | security service or mechanism violation |
| NOTE - Values 719-800 are reserved for M.3100 potential future extensions |  |  |

# Annex <B> (informative): PlantUML source text of Figures

## B.1	PlantUML for Figure 7.3.1-1: Alarm state diagram with consumer acknowledgement

@startuml

'Alarm State diagram with consumer acknowledgement

hide empty description

[*] --> unack_unclear

unack_unclear : ackState == False

unack_unclear : PS != cleared

note on link

Alarm raised and added to AlarmList

send

notifyNewAlarm

endnote

unack_unclear --> ack_unclear

ack_unclear : ackState == True

ack_unclear : PS != cleared

note on link

Alarm acknowledged

by consumer

send

notifyAckStateChanged

endnote

unack_unclear <-- ack_unclear

note on link

Alarm unacknowledged

by consumer

send

notifyAckStateChanged

endnote

ack_unclear --> unack_unclear

note on link

PS changed

send

notifyChangedAlarmGeneral

endnote

unack_unclear --> unack_clear

unack_clear : ackState == False

unack_clear : PS == cleared

note on link

PS changed by

system to cleared

send

notifyClearedAlarm

endnote

unack_unclear <-- unack_clear

note on link

PS changed by system

send

notifyChangedAlarmGeneral

endnote

unack_clear --> ack_clear

ack_clear : ackState == True

ack_clear : PS == cleared

note on link

Alarm acknowledged by consumer

send

notifyAckStateChanged

endnote

ack_unclear --> ack_clear

note left on link

PS changed by

system to cleared

send

notifyClearedAlarm

endnote

ack_clear --> [*]

note on link: Alarm removed from AlarmList

@enduml

## B.2	PlantUML for Figure 7.3.1-2: Alarm state diagram with automatic acknowledgement

@startuml

'Alarm State diagram - with automatic acknowledgement

hide empty description

[*] --> ack_unclear

ack_unclear : ackState == True

ack_unclear : PC != cleared

note on link

Alarm raised and added to AlarmList

Alarm automatically acknowledged

send notifyNewAlarm

endnote

ack_unclear --> ack_clear

ack_clear : ackState == True

ack_clear : PC == cleared

note left on link

Alarm cleared

send notifyClearedAlarm

endnote

ack_clear --> [*]

note on link: Alarm removed from AlarmList

@enduml

## B.3	PlantUML for Figure 7.6-1:Subscribing to alarm notifications – sequence diagram

@startuml

'Subscribing to Alarm Notifications

hide footbox

skinparam SequenceBoxBackgroundColor AliceBlue

skinparam BoxPadding 100

participant Consumer

box Producer

participant Provisioning

participant Fault_Mgmt

participant "  "

end box

Provisioning <-- Consumer: createMOI NtfSubscriptionControl

note over Provisioning: "NtfSubscriptionControl\ncreated"

Provisioning --> Consumer: Operation succeeded

note across: The following parts are part of the Notify New Alarm use case

note over Consumer

The FM and provisioning consumers

may or may not be the same entity.

end note

"  " --> Fault_Mgmt: fault, state-change or \nother indication

Provisioning <-- Fault_Mgmt: Read subscribers from \nNtfSubscriptionControl

Provisioning --> Fault_Mgmt: NtfSubscriptionControl_list

Fault_Mgmt --> Consumer: notifyNewAlarm

@enduml

## B.4	PlantUML for Figure 7.8-1: Notifying new alarms – sequence diagram

@startuml

'Notifying New Alarms

hide footbox

skinparam SequenceBoxBackgroundColor AliceBlue

skinparam BoxPadding 100

participant Consumer

box Producer

participant Provisioning

participant Fault_Mgmt

participant "  "

end box

"  " --> Fault_Mgmt: fault, state-change or \nother indication

Provisioning <-- Fault_Mgmt: Add  alarmRecord

note over Provisioning: alarmRecord added

Provisioning <-- Fault_Mgmt: Read subscribers from \nNtfSubscriptionControl

Provisioning --> Fault_Mgmt: NtfSubscriptionControl_list

Fault_Mgmt --> Consumer: notifyNewAlarm

@enduml

## B.5	PlantUML for Figure 7.11-1: Notify alarm list is rebuilt – sequence diagram

@startuml

'Notify alarm list rebuilt

hide footbox

skinparam SequenceBoxBackgroundColor AliceBlue

skinparam BoxPadding 100

participant Consumer

box Producer

participant Provisioning

participant Fault_Mgmt

participant "  "

end box

"  " --> Fault_Mgmt: some problem

Provisioning <-- Fault_Mgmt: Update AlarmList.unreliableAlarmScope

Note over Provisioning: AlarmList updated

Provisioning <-- Fault_Mgmt: Read subscribers from \nNtfSubscriptionControl

Provisioning --> Fault_Mgmt: NtfSubscriptionControl_list

Fault_Mgmt --> Consumer: notifyPotentialFaultyAlarmList

"  " --> Fault_Mgmt: problem corrected

note over Fault_Mgmt

AlarmList rebuilt

Proprietary internal process

May result in added, deleted,

changed alarmRecords

end note

Provisioning <-- Fault_Mgmt: Update AlarmList

Note over Provisioning: AlarmList updated

Provisioning <-- Fault_Mgmt: Read subscribers from \nNtfSubscriptionControl

Provisioning --> Fault_Mgmt: NtfSubscriptionControl_list

Fault_Mgmt --> Consumer: notifyAlarmListRebuilt

note over Consumer

Following is optional

behavior of the Consumer

The FM and provisioning consumers

may or may not be the same entity.

end note

Provisioning <-- Consumer: getMOIAttributes

Provisioning --> Consumer: return AlarmList

@enduml

# Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023--08 | SA5#150 | S5-235867 | - | - | - | Initial skeleton | 0.0.0 |
| 2023--08 | SA5#150 | S5-235093 | - | - | - | Rel-18 pCR 28.111 FM service first draft | 0.1.0 |
| 2023--08 | SA5#150 | S5-235106 | - | - | - | Rel-18 pCR 28.111 FM service, definition updates | 0.1.0 |
| 2023-10 | SA5#151 | S5-237046 | - | - | - | Remove requirements on virtual resources Stage 3 Code to be specified in Forge. | 0.2.0 |
| 2023-11 | SA5#152 | S5-238142 |  |  |  | Rel-18 pCR 28.111 FM full updates | 0.3.0 |