| 3GPP TR 28.873 V0.1.0 (2024-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on data management, subscriptions and reporting (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Concept and background	7

5	Use cases and potential requirements	7

5.1	Use case#1: Trace/MDT/QoE Identity uniqueness	7

5.1.1	 Description	7

5.1.2	Potential requirements	8

5.2	Use case#1: Redundant Subscriptions	8

5.1.1	 Description	8

5.2.2	Potential requirements	8

6	Potential solutions	8

6.X	Potential solution #<a>: <Title for potential solution a>	8

6.X.1	 Description	8

7	Conclusions and recommendations	8

Annex <A> (informative): Change history	9

For definitive guidance on drafting 3GPP TSs and TRs, see 3GPP TS 21.801 supplemented by the 3GPP web page http://www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec.

Ensure all blue guidance text is removed before submitting the TS/TR to the TSG for approval.

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

In the existing system it is a need that the “job reference” (e.g. Trace Reference for TraceJob, jobId for PerfMetricJob and qoEReference for QMCJob) is unique within a PLMN so that the producer can send the requested data to the right consumer.

In the existing system it is the consumers that sets the “job reference”. With a number of consumers that are separated from each other, it is not possible to fulfil the requirement of uniqueness, as the consumers can be different Network Functions (NFs) as well as different MnFs (Management Function).

With an increasing number of consumers, it is likely that the same data is requested by different consumers. It is investigated whether the NF needs to capture those data several times or if the reporting to different consumers can be moved out from the NF without any significant drawback.

# 1	Scope

The present document  investigates how unique “job reference id” (e.g. Trace Reference, QoE Reference, Job Id) can be unique within a 3GPP system and whether it is possible to decrease the load in the NFs when there are several consumers asking for the same data within a PLMN.

The study is done for SBMA.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.622: “Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)”.

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

MnF	Management Function

NF	Network Function

SBMA	Service Based Management Architecture

# 4	Concept and background

Editor’s note: This clause provides a description of concepts and background.

# 5	Use cases and potential requirements

Editor's note: this clause will use cases and potential requirements for issues to be included in this study.

## 5.1	Use case#1: Trace/MDT/QoE Identity uniqueness

### 5.1.1		Description

In the current 3GPP system, the number of automation functions are increasing. There are domain specific entities and interdomain entities. Both entities are performing automated functionality. All of these require collecting measurement data from the NFs and/or 3GPP management system. A unique identity is required to identify the measurement collection requests to avoid collisions. The example of this unique measurement job identity, which is used for all management and orchestration jobs, are:

- traceReference in TraceJob, refer to 3GPP TS28.622 [2]

- qoEReference in QMCJob, refer to 3GPP TS28.622 [2]

- jobId in PerfMetricJob, TraceJob, and QMCJob, refer to 3GPP TS28.622 [2]

With the large number of different consumers the uniqueness of the measurement job identities cannot be guaranteed. One example is:

Consumer A (e.g., NWDAF) request MDT data “1”, “2”, “5”, and “22” in Cell “a” and want the collected data to be sent to itself.

Consumer B (e.g. a customer care central) request MDT data “1”, “5” and “22” in Cell “a” and want the collected data to be sent to second line support.

These two requests are received within a short time difference.

As the two consumers are different and do not have any possibility to correlate the value for the Trace Reference, both use the value “123”, which will lead to that the base station supporting Cell “a” cannot differ on which collected data shall be sent where.

The examples of consumer are: SLS assurance function, Centralized coverage and capacity optimization function, Distributed load balancing function, Operator technician and Customer care center.

The examples of producer are: Base station, AMF-node, UDR-node and Element manager.

### 5.1.2	Potential requirements

REQ-PM-Y1: The identity used in a measurement job shall be globally unique between consumers and producers.

Editor’s note: The requirement may be clarified on the entity performing this activity after discussing the solutions.

## 5.2	Use case#1: Redundant Subscriptions

### 5.1.1		Description

In the current 3GPP system, the number of automation functions are increasing. There are domain specific entities and interdomain entities. Both entities are performing automated functionality. All of these require collecting measurement data from the NFs and/or 3GPP management system. There are large possibility that the measurement collection requests are redundant. The followings are some examples on how the redundant PM/Trace/MDT/QoE subscriptions are created:

- Multiple Trace/MDT/QoE activation requests from different consumers with different measurements on the same target, e.g. same UE or same area scope

- Multiple Trace/MDT/QoE activation requests from different consumers with overlapped measurements on the same target, e.g. same UE or same area scope

- Multiple Trace/MDT/QoE activation requests from different consumers with different measurements and overlapped target, e.g. overlapped area scope

- Multiple Trace/MDT/QoE activation requests from different consumers with overlapped measurements and overlapped target, e.g. overlapped area scope

When the number of redundant subscriptions increased, there are many potential issues:

- RRC interface may be overloaded due to too many MDT/QoE activations and reporting.

- Inefficient due to duplicated measurements and reports which may require too many resources on the same task.

- Exhausted network signaling in order to support multiple Trace/MDT at mobility.

### 5.2.2	Potential requirements

REQ-PM-Y1: The traffic node shall support a limitation of a maximum number of a specific PM/Trace/MDT/QoE measurement.

# 6	Potential solutions

## 6.X	Potential solution #<a>: <Title for potential solution a>

### 6.X.1		Description

Editor's note: this clause will contain proposed solutions for the different requirements and what impact it will have on the existing system. One solution can cover several potential requirements.

# 7	Conclusions and recommendations

Editor's note: this clause will contain conclusions and recommendations for normative work.

# Annex <A> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-04 |  |  |  |  |  | Initial skeleton | 0.0.0 |
| 2024-04 | SA5#154 | S5-241794 |  |  |  | pCR TR 28.873 Add structure for data management regarding subscriptions and reporting | 0.1.0 |
| 2024-04 | SA5#154 | S5-242093 |  |  |  | pCR TR 28.873 Add introduction for data management regarding subscriptions and reporting | 0.1.0 |
| 2024-04 | SA5#154 | S5-242094 |  |  |  | pCR TR 28.873 Add scope for data management regarding subscriptions and reporting | 0.1.0 |
| 2024-04 | SA5#154 | S5-242095 |  |  |  | pCR TR 28.873 Redundant Trace/MDT Subscriptions Use Case | 0.1.0 |
| 2024-04 | SA5#154 | S5-242096 |  |  |  | pCR TR 28.873 Unique Trace/MDT/QoE Identity | 0.1.0 |