| 3GPP TR 28.880 V1.2.0 (2024-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on energy efficiency and energy saving aspects of 5G networks and services (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	10

3.1	Terms	10

3.2	Symbols	11

3.3	Abbreviations	11

4	Concepts and background	11

4.1	Overall view of network energy efficiency	11

5	Use cases	12

5.1	Use case #1: Estimation of containerized VNF/VNFC energy consumption	12

5.1.1	Description	12

5.1.2	Potential requirements	12

5.1.3	Potential solutions	12

5.1.4	Evaluation of potential solutions	12

5.2	Use case #2: Alternative option to obtain energy consumption of VNF/VNFC	13

5.2.1	Description	13

5.2.2	Potential Requirements	13

5.2.3	Potential solutions	13

5.2.3.1	Potential solution #1: Obtain energy consumption of a NF realized by VM or OS container based VNF/VNFC	13

5.2.3.1.1	Introduction	13

5.2.3.1.2	Description	13

5.2.3.1.3	Formula of ECNF	14

5.2.3.1.4	Formula of estimated energy consumption of VNF	14

5.2.3.1.5	Formula of estimated energy consumption of containerized VNFC based on resource usage metrics	15

5.2.4	Evaluation of potential solutions	17

5.3	Use case #3: Enabling renewable energy consumption and carbon emission information reporting	17

5.3.1	Description	17

5.3.2	Potential requirements	18

5.3.3	Potential solutions	18

5.3.3.1	Potential solution #1: Carbon emission of non-split gNB	18

5.3.3.1.1	Part 1: Carbon emission of operator site	18

5.3.3.1.1.1	Introduction	18

5.3.3.1.1.2	Description	18

5.3.3.1.2	Part 2: carbon emission of non-split gNB	19

5.3.3.1.2.1	Introduction	19

5.3.3.1.2.2	Description	19

5.3.3.1.2.2.1	General	19

5.3.3.1.2.2.2	Carbon Emission of non-split gNB	19

5.3.3.2	Potential solution #2: Renewable energy factor of non-split gNB	20

5.3.3.2.1	Introduction	20

5.3.3.2.2	Description	20

5.3.3.2.2.1	General	20

5.3.3.2.2.2	Renewable Energy Factor of non-split gNB	20

5.3.3.3	Potential solution #3: Enable association of power supply and energy related information with network elements and network functions	21

5.3.3.3.1	Introduction	21

5.3.3.3.2	Description	21

5.3.4	Evaluation of potential solutions	22

5.4	Use case #4: Exposure of carbon and renewable energy related information	22

5.4.1	Description	22

5.4.2	Potential requirements	23

5.4.3	Potential solutions	23

5.4.3.1	Potential solution #1: Based on PerfMetricJob	23

5.4.3.1.1	Introduction	23

5.4.3.1.2	Description	23

5.4.3.2	Potential solution #2: Based on ManagementDataCollection	24

5.4.3.2.1	Introduction	24

5.4.3.2.2	Description	24

5.4.3.3	Potential solution #3: Retrieval of energy related information information associated with managed functions	24

5.4.3.3.1	Introduction	24

5.4.3.3.2	Description	25

5.4.4	Evaluation of potential solutions	25

5.5	Use case #5: Estimating averaged gNB energy consumption per UE	25

5.5.1	Description	25

5.5.2	Potential requirements	25

5.5.3	Potential solutions	25

5.6	Use case #6: Multi-dimensional network energy efficiency metrics	25

5.6.1	Description	25

5.6.2	Potential requirements	26

5.6.3	Potential solutions	26

5.6.3.1	Potential solution #1: EE KPIs evaluated from network data traffic performance dimension	26

5.6.3.1.1	Introduction	26

5.6.3.1.2	Description	27

5.6.3.2	Potential solution #2: EE KPIs evaluated for a network from availability dimension	27

5.6.3.2.1	Introduction	27

5.6.3.2.2	Description	28

5.6.3.3	Potential solution #3: EE KPIs evaluated from network quality dimension	29

5.6.3.3.1	Introduction	29

5.6.3.3.2	Description	29

5.6.4	Evaluation of potential solutions	33

5.7	Use case #7: Renewable energy-based LBO	34

5.7.1	Description	34

5.7.2	Potential requirements	34

5.7.3	Potential solutions	35

5.7.3.1	Potential solution #1	35

5.7.3.1.1	Introduction	35

5.7.3.1.2	Description	35

5.7.4	Evaluation of potential solutions	36

5.8	Use case #8: Energy saving by converting QoS of a service	36

5.8.1	Description	36

5.8.2	Potential requirements	37

5.8.3	Potential solutions	37

5.9	Use case #9: Renewable energy enabling 5GC NF re-selection	37

5.9.1	Description	37

5.9.2	Potential requirements	38

5.9.3	Potential solutions	38

5.9.3.1	Potential solution #1: Introduce a new attribute	38

5.9.3.1.1	Introduction	38

5.9.3.1.2	Description	38

5.9.4	Evaluation of potential solutions	38

5.10	Use case #10: Deployment of Network Slices depending on the energy source of the operator site	38

5.10.1	Description	38

5.10.2	Potential requirements	38

5.10.3	Potential solutions	39

5.10.3.1	Potential solution #1	39

5.10.4	Evaluation of potential solutions	40

5.11	Use case #11: Handling of power shortages	40

5.11.1	Description	40

5.11.2	Potential requirements	40

5.11.3	Potential solutions	40

5.11.3.1	Potential solution #1: Service adjustment for energy saving, based on available energy information	40

5.11.3.1.1	Introduction	40

5.11.3.1.2	Description	40

5.11.4	Evaluation of potential solutions	42

5.12	Use case #12: Cell proximity-based energy saving	43

5.12.1	Description	43

5.12.1.1	Overview	43

5.12.1.2	Degree of neighborliness	43

5.12.1.3	Cell grouping and categorization	43

5.12.1.4	Cell deactivation sequence	43

5.12.2	Potential requirements	44

5.12.3	Potential solutions	45

5.12.3.1	Potential solution #1: AI/ML-assisted Network Energy Saving	45

5.12.3.1.1	Introduction	45

5.12.4	Evaluation of potential solutions	45

5.13	Use case #13:  Per-network slice gNB and 5GC NF Energy Consumption	45

5.13.1	Description	45

5.13.2	Potential requirements	45

5.13.3	Potential solutions	45

5.13.3.1	Potential solution #1: Enhancement of TS 28.554 clause 6.7.3.3	45

5.13.3.1.1	Introduction	45

5.13.3.1.2	Description	45

5.13.4	Evaluation of potential solutions	46

6	Conclusions and recommendations	46

6.1	Use case #1: Estimation of containerized VNF/VNFC energy consumption	46

6.2	Use case #2: Alternative option to obtain energy consumption of VNF/VNFC	46

6.3	Use case #3: Enabling renewable energy consumption and carbon emission information reporting	46

6.4	Use case #4: Exposure of carbon and renewable energy related information	47

6.5	Use case #5: Estimating averaged gNB energy consumption per UE	47

6.6	Use case #6: Multi-dimensional network energy efficiency metrics	47

6.7	Use case #7: Renewable energy based LBO	47

6.8	Use case #8: Energy saving by converting QoS of a service	47

6.9	Use case #9: Renewable energy enabling 5GC NF re-selection	48

6.10	Use case #10: Deployment of Network Slices depending on the energy source of the operator site	48

6.11	Use case #11: Handling of power shortages	48

6.12	Use case #12: Cell proximity-based energy saving	48

6.13	Use case #13: Per-network slice gNB and 5GC NF Energy Consumption	48

Annex A: Rel-19 SA1 requirements on energy consumption / energy efficiency	48

Annex B: Standard emission factors	52

Annex C: Types of energy related information	53

Annex D: Change history	55

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document investigates the opportunities for defining new energy related information, new Energy Consumption (EC) KPIs, new Energy Efficiency (EE) KPIs and new Energy Saving (ES) solutions for 5G. It identifies and documents use cases and requirements related to energy related information (e.g. carbon emission, carbon emission efficiency, renewable energy factor), energy consumption, energy efficiency and energy saving. It documents and evaluates potential solutions, and provides recommendations for the normative work.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[3]	ETSI GR NFV-IFA 029 (V3.3.1): "Network Functions Virtualisation (NFV) Release 3; Architecture; Report on the Enhancements of the NFV architecture towards "Cloud-native" and "PaaS"".

[4]	ETSI GS NFV-IFA 040 (V4.3.1): "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Requirements for service interfaces and object model for OS container management and orchestration specification".

[5]	ETSI GS NFV-IFA 027 (V4.3.1): "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Performance Measurements Specification".

[6]	ETSI GS NFV-IFA 027 (V5.2.1): "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Performance Measurements Specification".

[7]	3GPP TS 22.261: "Service requirements for the 5G system".

[8]	ETSI GS OEU 020: "Operational energy Efficiency for Users (OEU); Carbon equivalent Intensity measurement; Operational infrastructures; Global KPIs; Global KPIs for ICT Sites".

[9]	Void.

[10]	ISO/IEC 30134-3:2016: "Information technology -- Data centres -- Key performance indicators -- Part 3: Renewable energy factor (REF)".

[11]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[12]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[13]	3GPP TS 28.313: "Management and orchestration; Self-Organizing Networks (SON) for 5G networks".

[14]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[15]	Energy Benchmark 2023: Telco Insights and Industry Health Check.

[16]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[17]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[18]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[19]	ETSI EN 303 471 V1.1.1 (2019-01): "Environmental Engineering (EE); Energy Efficiency measurement methodology and metrics for Network Function Virtualisation (NFV)".

[20]	ITU-T Recommendation L.1333: "Carbon data intensity for network energy performance monitoring".

[21]	ETSI ES 202 336-1 V1.2.1: "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 1: Generic Interface"

[22]	ETSI ES 202 336-2 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 2: DC power system control and monitoring information model"

[23]	ETSI ES 202 336-3 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 3: AC UPS power system control and monitoring information model"

[24]	ETSI ES 202 336-4 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 4: AC distribution power system control and monitoring information model"

[25]	ETSI ES 202 336-5 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 5: AC diesel back-up generator system control and monitoring information model"

[26]	ETSI ES 202 336-6 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 6: Air Conditioning System control and monitoring information model"

[27]	ETSI ES 202 336-7 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 7: Other utilities system control and monitoring information model"

[28]	ETSI ES 202 336-8 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 8: Remote Power Feeding System control and monitoring information model"

[29]	ETSI ES 202 336-9 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 9: Alternative Power Systems"

[30]	ETSI ES 202 336-10 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 10: AC inverter power system control and monitoring information model"

[31]	ETSI ES 202 336-11 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 11: Battery system with integrated control and monitoring information model"

[32]	ETSI ES 202 336-12 V1.2.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 12: ICT equipment power, energy and environmental parameters monitoring information model"

[33] 	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)"

[34]	3GPP TS 28.531: " Management and orchestration; Provisioning"

[35]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[36]	3GPP TS 28.532: "Management and orchestration; Generic management services"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

carbon emission: quantity of equivalent carbon dioxide emitted (e.g. kg of CO2 equivalent)

NOTE:	This definition is taken from 3GPP TS 22.261 [7].

carbon emission factor: kilograms of equivalent carbon dioxide emitted per kWh (kg of CO2eq/kWh)

NOTE 1:	This definition is taken from ETSI GS OEU 020 [8].

NOTE 2:	In the present document, the carbon emission factor considered is the operational carbon emission factor, which considers only emissions that directly result from the operation of a given type of energy source. Operational carbon emission factor is different from life-cycle carbon emission factor which considers all emissions, including those generated during the construction of power plants, the extraction of fuels, etc.

carbon emission efficiency: ratio between the carbon emission and the performance of a network entity

NOTE:	The definition of the performance of a network entity depends on the type of network entity. In the present document, network entity can represent a gNB, a 5GC NF, etc. The performance of a network entity can be e.g. data volume.

Energy Consumption (EC): See definition in 3GPP TS 28.310 [12].

Energy Efficiency (EE): See definition in 3GPP TS 28.310 [12].

renewable energy: energy from renewable non-fossil sources

NOTE 1:	This definition is taken from 3GPP TS 22.261 [7].

NOTE 2:	Examples of renewable energy sources include wind, solar, aerothermal, geothermal, hydrothermal and ocean energy, hydropower, biomass, landfill gas, sewage treatment plant gas and biogases

NOTE 3:	A renewable energy source is an energy source not depleted by extraction as it is naturally replenished at a rate faster than it is extracted ISO/IEC 30134-3:2016 [10].

NOTE 4:	Criteria to categorize an energy as renewable can differ among jurisdictions, based on local environmental or other reasons ISO/IEC 30134-3:2016 [10].

renewable energy factor: ratio of the renewable energy to the total energy

NOTE:	This definition is taken from ISO/IEC 30134-3:2016 [10].

Operator site: premises accommodating Network Telecommunications Equipment (NTE) providing direct connection to the core and access networks and which may also accommodate Information Technology Equipment (ITE).

NOTE 1:	This definition is taken from ETSI EN 303 471 V1.1.1 (2019-01) [19].

NOTE 2: Network Telecommunications Equipment (NTE) and Information Technology Equipment (ITE) are both defined in ETSI EN 303 471 V1.1.1 (2019-01) [19].

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CE	Carbon Emission

CEE	Carbon Emission Efficiency

CEF	Carbon Emission Factor

GST 	Generic Slice Template

NEST	Network Slice Type

P-NEST 	Private NEST

REF	Renewable Energy Factor

S-NEST 	Standardized NEST

# 4	Concepts and background

## 4.1	Overall view of network energy efficiency

In addition to traditional policy-based energy-saving solutions, operators and vendors are developing many solutions aimed at enhancing overall energy efficiency throughout 3GPP networks, such as deployment of intent driven RAN energy saving solutions and AI-driven energy saving solutions focused on improving energy efficiency and preventing unnecessary energy consumption in networks.

According to network energy distribution published by GSMA Intelligence[15], which illustrates the area of energy consumption, RAN is still the most power consuming part in mobile networks. See reference [15] for further info of the data based on GSMA Intelligence Telco Energy Benchmark study in 2024 with 65 mobile networks.

There are many initiatives on energy efficiency and energy saving in 3GPP and other SDOs/fora. During the past 3GPP releases, SA5 has been working jointly with external SDOs/groups on EE and will continue to synchronize with e.g. ETSI TC EE, ITU-T SG5, NGMN GFN (Green Future Networks), ETSI NFV ISG, and more recently with ITU-T SG11.

# 5	Use cases

## 5.1	Use case #1: Estimation of containerized VNF/VNFC energy consumption

### 5.1.1	Description

In Rel-18, in order to be able to estimate the energy consumption of 5GC Network Functions, the 3GPP management system estimates the energy consumption of VNF/VNFCs which compose the 5GC NFs. To achieve this, it collects, from NFV-MANO, metrics such as mean virtual CPU usage, mean virtual disk usage, mean virtual memory usage and I/O traffic for all VNF/VNFCs which compose the 5GC NFs.

The existing definition of the Energy Consumption (EC) of VNF/VNFCs (see 3GPP TS 28.554 [2], clauses 6.7.3.1.2 and 6.7.3.1.3) is valid for VM-based VNFs, i.e. when VNF/VNFC(s) are implemented on Virtual Machine(s) (VM).

However, according to ETSI ISG NFV, VNF/VNFCs may also be implemented using OS container technology (see e.g. ETSI GR NFV-IFA 029 [3] and ETSI GS NFV-IFA 040 [4]).

In the context of this use case, a VNF (respectively VNFC) running using OS container technology is called a 'containerized VNF' (resp. 'containerized VNFC'), and is similar to a 'containerized workload' as defined in ETSI GS NFV-IFA 040 [4] clause 3.1.

This use case addresses the situation in which the network operator wants to estimate the energy consumption of 5GC NFs which are based on containerized VNF/VNFCs, using NFV-MANO.

### 5.1.2	Potential requirements

REQ-Energy_Consumption_Containerized-CON-1: The 3GPP management system should be able to estimate the energy consumption of 5GC NFs based on containerized VNF/VNFCs.

REQ-Energy_Consumption_Containerized-CON-2: The 3GPP management system should be able to expose the energy consumption of 5GC NFs based on containerized VNF/VNFCs to authorized consumer(s).

### 5.1.3	Potential solutions

The potential requirement #1 for this use case is the same as the potential requirement #2 for use case #2. Hence, the potential solution #1 of use case #2 is also applicable to satisfy the potential requirement #1 for this use case.

The potential requirement #2 for this use case could be satisfied by existing solutions.

### 5.1.4	Evaluation of potential solutions

The potential solutions described in clause 5.1.3 satisfy both potential requirements from clause 5.1.2.

To satisfy the potential requirement #1, there is a dependency on the conclusion for the potential solution #1 of use case #2.

The potential solution satisfies the potential requirement #2 with no new normative work.

## 5.2	Use case #2: Alternative option to obtain energy consumption of VNF/VNFC

### 5.2.1	Description

For Energy Consumption (EC) of VNF/VNFCs based on VMs, estimated measurement is supported as defined in clauses 6.7.3.1.2 and 6.7.3.1.3 of 3GPP TS 28.554 [2]). The way to calculate the estimated VM-based VNF/VNFC energy consumption requires obtaining the sum of the vCPU mean usage of all virtual compute resource instances running on the same NFVI Node during the same observation period, all separately provided by NFV-MANO (see clause 7.1.2 of ETSI GS NFV-IFA 027 [6]).

The issue is that there is no easy way to obtain all virtual compute resource instances information running on the same NFVI Node and measure the vCPU, vMemory or vDisk mean usage for all of them. The measurement method as described in clause 7.1.2 of ETSI GS NFV-IFA 027 [6] does not support setting the measurement scope on the same NFVI Node. Some other mechanism to know all the virtual compute resource instances information running on the same NFVI Node is required.

In addition, the  mechanisms for obtaining energy consumption (EC) of VNF/VNFC in practical implementations are more complicated than the estimation method currently defined in in clauses 6.7.3.1.2 and 6.7.3.1.3 of 3GPP TS 28.554 [2], which is simply based on vCPU, vMemory , vDisk mean usage or I/O traffic volume. Diverse power measurement models and tools for obtaining the EC are possible depending on the operator deployment scenarios.

Alternative options for obtaining energy consumption of VNF/VNFC in addition to the existing estimation methods defined in clauses 6.7.3.1.2 and 6.7.3.1.3 of 3GPP TS 28.554 [2] are needed.

As described in clause 7.1.18 and clause 7.4.12 of ETSI GS NFV-IFA 027 [6], the actual energy consumption of virtual compute and OS container workload is provided. As an enhancement, this use case is to study:

a)	potential enhancements to existing (EC of VM-based VNF/VNFCs estimation method; and

b)	potential extension to EC estimation of containerized VNF/VNFCs.

### 5.2.2		Potential Requirements

REQ-ENHANCEMENT-MEAS-01: The 3GPP management system should have a capability to generate KPIs about the energy consumption of an NF realized by VM-based VNF/VNFCs.

REQ-ENHANCEMENT-MEAS-02: The 3GPP management system should have a capability to generate KPIs about the energy consumption of an NF realized by OS container-based VNF/VNFCs.

### 5.2.3	Potential solutions

#### 5.2.3.1	Potential solution #1: Obtain energy consumption of a NF realized by VM or OS container based VNF/VNFC

##### 5.2.3.1.1	Introduction

In this potential solution, new alternative formula is proposed to calculate and generate KPIs about the energy consumption (EC) of a Network Function (NF). All the options listed only consider that 3GPP management system performs either some calculation based on other existing measurements available to the 3GPP management system, or that it performs the new KPI by means of object mapping

##### 5.2.3.1.2	Description

The KPI for EC of a 5G Network Function (NF) is defined in clause 6.7.3.1.1 in 3GPP TS 28.554 [2], and it is considered that ECVNF (Energy Consumption of a VNF) is estimated based on vCPU, vMemory, vDisk mean usage ratio or I/O traffic volume.

According to ETSI GS NFV-IFA 027 V5.1.1 [6], energy consumption of a VM or an OS container workload can be also directly provided by the ETSI NFV's NFV-MANO system, which is computed by the NFV-MANO measurement producer based on energy consumption measurements received from NFVI or Container Infrastructure Services (CIS), and the exact method for estimating or measuring the energy consumption in the NFVI or CIS is not defined and left to implementation. It makes it flexible for vendors and operators to implement and/or deploy more advanced solutions to provide better energy consumption estimations.  Therefore, it is proposed to modify the current definition of ECNF by using a generic definition of energy consumption of VNF enabling both the current (vCPU, vMemory, vDisk mean usage based or I/O traffic volume) and new (ETSI GS NFV-IFA 027 V5.2.1 [6] based) options to exist.

##### 5.2.3.1.3	Formula of ECNF

Same as currently defined in clauses 6.7.3.1.1 of 3GPP TS 28.554 [2], the formula of ECNF is described as below:

and ECVNF,estimated represents the total energy consumption of a VNF, which is obtained by summing up the energy consumption of its constituent Virtualized Network Function Components (VNFC). The unit of this KPI is J.

##### 5.2.3.1.4	Formula of estimated energy consumption of VNF

Same as currently defined in clauses 6.7.3.1.2 of 3GPP TS 28.554 [2], the formula of ECVNF, estimated is described as below:

and ECVNFC, estimated represents the energy consumption of a VNFC.

When VNFC is realized with a virtual machine, ECVNFC, estimated is obtained by the following three options:

a)	By the existing mechanism which estimates the energy consumption of the VNFC based on its resource usage metric provided by ETSI NFV’s NFV-MANO, which is either:

-	mean vCPU usage, as per clause 6.7.3.1.4 of 3GPP TS 28.554 [2]; or

-	mean vMemory usage as per clause 6.7.3.1.5 of 3GPP TS 28.554 [2]; or

-	mean vDisk usage as per clause 6.7.3.1.6 of 3GPP TS 28.554 [2]; or

-	I/O traffic volume as per clause 6.7.3.1.7 of 3GPP TS 28.554 [2].

b)	Or by collecting the total energy consumption of the virtual resource on which VNFC runs over the collection period provided by ETSI NFV’s NFV-MANO (see clause 7.1.18 of ETSI GS NFV-IFA 027 [5] and [6]) which equals to the energy consumption of VNFC.

c)	Or by collecting the total energy consumption of the VNFC over the collection period provided by ETSI NFV’s NFV-MANO (see clause 7.2.16 of ETSI GS NFV-IFA 027 [6]).

When VNFC is realized with OS containers, ECVNFC, estimated is obtained by the following three options:

a)	By estimating the energy consumption of the VNFC based on the resource usage metric provided by ETSI NFV’s NFV-MANO which is either:

-	mean vCPU usage as per clause 7.4.2 of ETSI GS NFV-IFA 027 [6], or

-	mean memory usage as per clause 7.4.4 of ETSI GS NFV-IFA 027 [6], or

-	mean usage of storage resource as per clause 7.4.10 of ETSI GS NFV-IFA 027 [6], or

-	number of incoming and outgoing bytes as per clause 7.4.6 and 7.4.7 of ETSI GS NFV-IFA 027 [6].

Similar to clause 6.7.3.1.4, 6.7.3.1.5, 6.7.3.1.6 and 6.7.3.1.7 of 3GPP TS 28.554 [2], the energy consumption of the VNFC is estimated as a proportion of the energy consumption of the compute node on which the VNFC runs, , which in the case of OS containers is referred to as "container infrastructure services (CIS) instance" as defined in ETSI GR NFV-IFA 029 [3]. This proportion is obtained by dividing the resource mean usage of the VNFC instance realized as an OS container workload, by the sum of the resource mean usage of all VNFC instances running on the same compute node. The formulas for computing this KPI are the same as specified in clauses 6.7.3.1.2 and 6.7.3.1.3 of 3GPP TS 28.554 [2] for the ECVNFC, estimated, while using the corresponding resource usage metrics from ETSI GS NFV-IFA 027 [6] listed above. This option needs to consider the same pre-conditions documented in clause 6.3.2 of TS 28.310 [12]. More detailed formula is provided in clause 5.2.3.1.5.

b)	Or by collecting the energy consumption of the container workload realizes the VNFC over the collection period provided by ETSI NFV’s NFV- MANO (see clause 7.4.12 of ETSI GS NFV-IFA 027 [6]) which equals to the energy consumption of VNFC.

c)	Or by collecting the total energy consumption of the VNFC over the collection period provided by ETSI NFV’s NFV-MANO (see clause 7.2.16 of ETSI GS NFV-IFA 027 [6]).

ECVNF,estimated can also be collected, regardless of its composition based on VNFC realized by virtual machine or OS containers by collecting the total energy consumption of the VNF over the collection period provided by ETSI NFV’s NFV-MANO (see clause 7.2.18 of ETSI GS NFV-IFA 027 [6]).

##### 5.2.3.1.5	Formula of estimated energy consumption of containerized VNFC based on resource usage metrics

The detailed formula of the energy consumption of the containerized VNFC based on resource usage metrics is as follows. The estimated energy consumption of containerized VNFC is obtained by taking the estimated energy consumption of the OS container workload instance on which the VNFC runs.

ECcontainerized VNFC, estimated=ECOS container workload, estimated

The energy consumption of an OS container workload instance X is estimated as a proportion of the energy consumption of the compute node (i.e., a container infrastructure services (CIS) instance) on which the OS container workload instance X runs.

NOTE:	The following formulation assumes the case that OS container workloads are deployed on CIS instances that are bare-metal compute nodes, which can be mapped 1:1 to an NFVI Node.

This proportion can be obtained by considering the following four metrics:

-	mean vCPU usage as per clause 7.4.2 of ETSI GS NFV-IFA 027 [6].

The proportion is obtained by dividing the vCPU mean usage of the OS container workload instance X, by the sum of the vCPU mean usage of all OS container workload instances running on the same NFVI Node as X, as defined by the equation below:

ECOS container workload, estimated, VCpuUsageMean=VCpuUsageMeanOS container workloadVCpuUsageMean*ECNFVINode, measured

where:

-	VCpuUsageMean is the mean virtual CPU usage measurement for the OS container workload instance during the observation period, provided by NFV-MANO (see clause 7.4.2 of ETSI GS NFV-IFA 027 [6]);

-	OS container workloadVCpuUsageMean is the sum of the mean virtual CPU usage of all OS container workload instances running on the same NFVI Node during the same observation period, all separately provided by NFV-MANO;

-	ECNFVINode, measured is the energy consumption of the NFVI Node on which the OS container workload instance runs, measured during the same observation period, as per ETSI ES 202 336-12 [32].

-	mean vMemory usage as per clause 7.4.4 of ETSI GS NFV-IFA 027 [6].

The proportion is obtained by dividing the vMemory mean usage of the OS container workload instance X, by the sum of the vMemory mean usage of all OS container workload instances running on the same NFVI Node as X, as defined by the equation below:

ECOS container workload, estimated, VMemoryUsageMean=VMemoryUsageMeanOS container workloadVMemoryUsageMean*ECNFVINode, measured

where:

-	VMemoryUsageMean is the mean virtual memory usage measurement for the OS container workload instance during the observation period, provided by NFV-MANO (see clause 7.4.4 of ETSI GS NFV-IFA 027 [6]);

-	OS container workloadVMemoryUsageMean is the sum of the mean virtual memory usage of all OS container workload instances running on the same NFVI Node during the same observation period, all separately provided by NFV-MANO;

-	ECNFVINode, measured is the energy consumption of the NFVI Node on which the OS container workload instance runs, measured during the same observation period, as per ETSI ES 202 336-12 [32].

-	mean usage of vStorage Resource as per clause 7.4.10 of ETSI GS NFV-IFA 027 [6].

The proportion is obtained by dividing the vStorage Resource mean usage of the OS container workload instance X, by the sum of the vStorage Resource mean usage of all OS container workload instances running on the same NFVI Node as X, as defined by the equation below:

ECOS container workload, estimated, VDiskUsageMean

=VStorageResourceUsageMeanOS container workloadVStorageResourceUsageMean*ECNFVINode, measured

where:

-	VStorageResourceUsageMean is the mean virtual storage resource usage measurement for the OS container workload instance during the observation period, provided by NFV-MANO (see clause 7.4.10 of ETSI GS NFV-IFA 027 [6]);

-	OS container workloadVStorageResourceUsageMean is the sum of the mean virtual storage resource usage of all OS container workload instances running on the same NFVI Node during the same observation period, all separately provided by NFV-MANO;

-	ECNFVINode, measured is the energy consumption of the NFVI Node on which the OS container workload instance runs, measured during the same observation period, as per ETSI ES 202 336-12 [32].

-	number of incoming bytes and outgoing bytes as per clause 7.4.6 and 7.4.7 of ETSI GS NFV-IFA 027 [6].

The proportion is obtained by dividing the I/O traffic volume of the OS container workload instance X, by the sum of the I/O traffic volume of all OS container workload instances running on the same NFVI Node as X, as defined by the equation below:

ECOS container workload, estimated, IOTrafficVolume=IOTrafficVolumeOS container workloadIOTrafficVolume*ECNFVINode, measured

where:

-	IOTrafficVolume is the sum of the incoming and outgoing traffic volumes of the OS container workload instance during the observation period, provided by NFV-MANO;

-	Incoming traffic volume is obtained by measuring the number of incoming bytes on OS container workload (VNetByteIncoming - cf. clause 7.4.6 of [6]) during the observation period;

-	Outgoing traffic volume is obtained by measuring the number of outgoing bytes on OS container workload (VNetByteOutgoing - cf. clause 7.4.7 of [6]) during the observation period.

IOTrafficVolume=VNetByteIncoming+VNetByteOutcoming

-	OS container workloadIOTrafficVolume is the sum of the incoming and outgoing traffic volumes of all OS container workload instances running on the same NFVI Node during the same observation period, all separately provided by NFV MANO (see clause 7.4.6 and 7.4.7 of [6]);

-	ECNFVINode, measured is the energy consumption of the NFVI Node on which the OS container workload instance runs, measured during the same observation period, as per ETSI ES 202 336-12 [32].

### 5.2.4	Evaluation of potential solutions

The solution described in clause 5.2.3.1 provides the possibility of 3GPP management system to generate KPI about the energy consumption of an NF realized by VM-based and/or OS container-based VNF/VNFC either based on deriving the KPI from energy consumption associated to VNF/VNFC and/or their virtualised compute/OS container workload resources (as provided by ETSI NFV’s NFV-MANO), or by deriving the KPI based on other resource related metrics like CPU, memory, disk/storage resource or traffic volume. Thus, the solution makes diverse power measurement models and tools for obtaining the EC possible to meet specific operator deployment scenarios and requirements. Furthermore, the option of deriving the KPI based on other resource related metrics like CPU, memory, disk/storage resource or traffic volume and the energy consumption from NFVI Nodes has dependency on the availability of such NFVI Node energy consumption to the 3GPP management system, as also documented in clause 6.3.2 of TS 28.310 [12].

Among the different alternatives describe, those that are based on collecting energy consumption associated to VNF/VNFC and/or their virtualised compute/OS container workload resources (refer to options b and c in respective VM/OS container-based cases) are likely to be more accurate since the sourced energy consumption information is associated at collection and reporting level to virtualised compute/OS container resources by mechanisms available in the infrastructure and its management and orchestration platforms.

The potential solution #1 fulfils the requirements REQ-ENHANCEMENT-MEAS-01 and REQ-ENHANCEMENT-MEAS-02. This is a feasible candidate as input to the normative phase.

## 5.3	Use case #3: Enabling renewable energy consumption and carbon emission information reporting

### 5.3.1	Description

In Rel-18, the 3GPP management system is capable to manage the following types of energy related information:

-	energy consumption measurements and/or KPIs;

-	energy saving state.

In Rel-19, new aspects of energy related information are expected to be provided by the 5G system. 3GPP TS 22.261 [7] clause 6.15a states:

"Energy consumption and efficiency information and network energy states can be exposed to third parties and energy consumption can be constrained.

Energy related information can include ratio of renewable energy and carbon emission information when available. Calculation of energy related information as described in the following requirements is done by means of averaging or applying a statistical model. The requirements do not imply that some form of 'real time' monitoring is required.".

There are different energy sources existing, producing different energy types, e.g. non-renewable, renewable etc. Thus, the equipment and infrastructure of the network could be powered by different types of energy sources and the energy consumption measurements could correspond to such different energy types.

Due to the highly variable and unpredictable nature of renewable energy sources, the supply of renewable energy varies substantially by time and location. There is a need for the 3GPP management system to report the energy consumption metrics along with the type of consumed energy and carbon emission information, for example:

-	Network slice customers may request some minimal amount/ratio for renewable energy to be used for the slice realization, and hence the network slice provider needs to have a means to monitor and report on the energy consumption for different energy types.

-	Awareness of energy consumption of different energy types is useful for energy type aware network management, e.g. defining energy saving policies such that the consumption of renewable energy can be favoured.

In this use case, the 3GPP management system is capable, based on energy related information obtained from various sources, to measure per 5GC NF and per gNB energy related information, i.e.:

- carbon emission information, i.e.:

-	carbon emission;

-	carbon emission efficiency.

- ratio of renewable energy;

and to provide this information to authorized consumers.

How this energy related information is exposed to authorized consumers is addressed by the use case #4: Exposure of carbon and renewable energy related information, see clause 5.4.

### 5.3.2	Potential requirements

REQ-ENERGY_INFO-CON-1: The 3GPP management system should be able to measure per 5GC NF and gNB carbon emission related information and report it to authorized consumers.

REQ-ENERGY_INFO-CON-2: The 3GPP management system should be able to measure per 5GC NF and gNB energy related information, i.e. renewable energy and/or non-renewable energy related information and report it to authorized consumers.

REQ-ENERGY_INFO-CON-3: The 3GPP management system should allow its authorized consumers to associate power supply information and energy related information, i.e., carbon emission, renewable energy and/or non-renewable energy related information, related to the managed functions, to be utilized within the 3GPP system.

### 5.3.3	Potential solutions

#### 5.3.3.1	Potential solution #1: Carbon emission of non-split gNB

##### 5.3.3.1.1	Part 1: Carbon emission of operator site

###### 5.3.3.1.1.1	Introduction

This potential solution describes how the carbon emission of an operator site can be obtained.

###### 5.3.3.1.1.2	Description

In this potential solution, the Network Operator gathers information about:

- the source(s) of the energy used to provide electricity to the operator site

- for each energy source, the respective energy consumed during the observation period

, and provides the 3GPP management system with this information.

NOTE 1: the source(s) of energy can be renewable, i.e. from renewable non-fossil sources (cf. the definition of renewable energy in clause 3.1), or non-renewable, i.e. from fossil sources (e.g. coal, natural gas, oil) or nuclear plants.

NOTE 2: how the Network Operator gets this information from the energy supplier is out of scope of the present study.

NOTE 3: how the 3GPP management system stores this information is FFS.

In case of multiple types of energy sources, CESite can be obtained by summing up, for each type of energy source i used to power the operator site, the energy consumed using this type of energy source multiplied by the Carbon Emission Factor of the type of energy source i:

Equation 1: Carbon Emission of a multi-energy-source operator site

, where:

- ECi is the Energy Consumption corresponding to the type of energy source i used to power the operator site. Its unit is kWh;

- CEFi is the Carbon Emission Factor of the type of energy source i. Its unit is kg CO2 / kWh. The Carbon Emission Factor is equivalent to the standard emission factor from annexes A.3 and A.4 of ETSI GS OEU 020 [8], whose tables are partially copy/pasted in Annex X for information;

- CESite represents the Carbon Emission of the site. Its unit is kg CO2eq.

##### 5.3.3.1.2	Part 2: carbon emission of non-split gNB

###### 5.3.3.1.2.1	Introduction

This potential solution describes how, based on the carbon emission of operator site (see Part 1 in clause 5.3.3.1.1), the carbon emissions of non-split gNB can be estimated by the 3GPP management system.

###### 5.3.3.1.2.2	Description

5.3.3.1.2.2.1		General

In this potential solution:

- The 3GPP management system collects Energy Consumption measurements for the non-split gNB, as those defined in TS 28.552 [11] clause 5.1.1.19.3;

- The Network Operator gathers information, from the energy supplier, about the energy consumption of the operator site (as defined in ETSI EN 303 471 [x]) where the non-split gNB resides, and provides the 3GPP management system with this information;

- The 3GPP management system can then calculate the Carbon Emission of the non-split gNB.

NOTE 1: all information collected above should correspond to the same observation period.

NOTE 2: how the Network Operator gets information about the energy consumption of the operator site from the energy supplier is out of scope of the present study.

NOTE 3: how the 3GPP management system stores this information is FFS.

NOTE 4: this potential solution assumes that all gNBs in the operator site are powered from the same source(s) of energy.

NOTE 5: an operator site may contain zero to many non-split gNB(s). A non-split gNB pertains to one and only one operator site.

5.3.3.1.2.2.2		Carbon Emission of non-split gNB

The Carbon Emission of a non-split gNB (CENonSplitgNB) over a time period can be defined as a proportion of the Carbon Emission of the operator site (CESite) where the non-split gNB resides, the proportion being given by the ratio of the Energy Consumption of the non-split gNB (ECNonSplitgNB) to the total Energy Consumption of the operator site during the same observation period. Its unit is kg CO2 / kWh.

Equation 2: Carbon Emission of a non-split gNB

, where:

- CESite is the Carbon Emission of the operator site where the non-split gNB resides (cf. clause 5.3.3.1.1). Its unit is kg CO2eq;

- ECNonSplitgNB is the Energy Consumption of the gNB; it corresponds to PEE.Energy as defined in TS 28.552 [11] clause 5.1.1.19.3. Its unit is kWh;

- ECSite represents the total Energy Consumption of the operator site where the non-split gNB resides. Its unit is kWh.

NOTE 1: in this potential solution, CESite and ECSite are provided as input from the operator.

NOTE 2: whether and how all the energy related information used in this potential solution will be modelled (e.g. NRM IOC/attributes, KPIs, etc.) is to be elaborated during the normative phase.

NOTE 3: which IOC(s) can be used to represent non-split gNB in TS 28.622 [14] and/or TS 28.541 [16] is out of the scope of this study.

NOTE 4: extension of this potential solution to UPFs is to be elaborated during the normative phase.

#### 5.3.3.2	Potential solution #2: Renewable energy factor of non-split gNB

##### 5.3.3.2.1	Introduction

In clause 3.1, the Renewable Energy Factor (REF) is defined as the ratio of the renewable energy to the total energy. This potential solution describes how the REF of a non-split gNB can be calculated by the 3GPP management system.

##### 5.3.3.2.2	Description

###### 5.3.3.2.2.1		General

In this potential solution:

- The Renewable Energy Factor of an operator site applies equally to all equipment that resides in the site, including the non-split gNB. Consequently, this potential solution focuses on calculating the Renewable Energy Factor of sites;

- The 3GPP management system collects the information about Energy Consumption of the site during the observation period, per energy source;

- The 3GPP management system sumps up the Energy consumptions coming from renewable sources only;

- The 3GPP management can then calculate the Renewable Energy Factor of the site.

###### 5.3.3.2.2.2		Renewable Energy Factor of non-split gNB

The Renewable Energy Factor of a non-split gNB (REFNonSplit gNB) is equal to the Renewable Energy Factor of the operator site (REFSite) where the non-split gNB resides.

Equation 1: Renewable Energy Factor of a non-split gNB

The Renewable Energy Factor of an operator site is defined as the ratio of the Energy Consumption of the operator site coming from renewable energy sources to the total Energy Consumption of the operator site:

Equation 2: Renewable Energy Factor of a site

, where:

- ECSite,Ren is the Energy Consumption of the operator site coming from renewable energy sources only. Its unit is kWh;

- ECSite is the total Energy Consumption of the operator site. Its unit is kWh:

- REFSite represents the Renewable Energy Factor of the operator site during the observation period. 0 ≤ REFSite ≤ 1.

NOTE 1: The following energy sources may be considered as renewable (see ISO/IEC 30134-3:2016 [10]):

- bioenergy,

- geothermal,

- hydropower,

- ocean, solar and wind energy.

NOTE 2: in this potential solution, ECSite and ECSite,Ren are provided as input from the operator.

NOTE 3: whether and how all the energy related information used in this potential solution will be modelled (e.g. NRM IOC/attributes, KPIs, etc.) is to be elaborated during the normative phase.

NOTE 4: which IOC(s) can be used to represent non-split gNB in TS 28.622 [14] and/or TS 28.541 [16] is out of the scope of this study.

NOTE 5: extension of this potential solution to UPFs is to be elaborated during the normative phase.

#### 5.3.3.3	Potential solution #3: Enable association of power supply and energy related information with network elements and network functions

##### 5.3.3.3.1	Introduction

The power supply of network elements and network functions can change over different time periods. These power supply information includes grid electricity, Backup energy and locally generated energy (see clause 6 of ITU-T Recommendation L.1333 [20]). This information is not managed by 3GPP system but are defined by ETSI.  (ETSI ES 202 336-1 [21] to ETSI ES 202 336-12 [32]). Nevertheless, the 3GPP system requires this information which can be distributed and utilized within the 3GPP system (including 5GC and external 3rd parties).

This potential solution proposes a list of information elements to be associated with the network functions, for which the operator collects information and inputs into the 3GPP management system.

##### 5.3.3.3.2	Description

This potential solution proposes that new information element(s) be defined to enable the operator to associate this information with the managed functions that are managed by 3GPP management system. This information can be further used for the carbon emission and renewable energy related KPIs and for any network optimizations.

Associated to the 5G NFs (represented by ManagedFunction), the power supply information :

-	can consist of power supply, which can be power grids, batteries, UPS (Uninterrupted Power Supplies), generators (e.g., diesel back-up generator), on-site generated power (e.g., solar power, wind power).

-	allows to indicate if different power supply types are used at certain planned schedules. (E.g., if the locally generated energy is used during the day, and energy from the grid is used during the night times to power the network functions).

This information may be associated with energy related information described in other potential solutions (see clauses X, Y, Z), such as e.g.:

- carbon emission of the 5G NF,

- renewable energy factor of the 5G NF,

- carbon intensity of the 5G NF.

### 5.3.4	Evaluation of potential solutions

Potential solutions #1, #2 and #3 are not mutually exclusive. Instead, they may complement each other.

The potential solution #1 proposes that, based on input from external data, the 3GPP management system is able to associate the carbon emission information of non-split gNBs.

This solution is feasible by providing the operator with configuration options to associate energy related information such as carbon emission factor per type of energy source, energy consumption of operator sites per type of energy source, carbon emission of operator sites.

The potential solution #2 proposes that, based on input from external data, the 3GPP management system is able to handle the renewable energy factor of non-split gNBs.

This solution is feasible by providing the operator with configuration options to associate energy related information such as the part of energy consumption of operator sites coming from renewable energy.

Annex C gives an overview of all energy related information.

NOTE 1: whether and how the potential solutions #1 and #2 can apply to split gNBs and 5GC NFs will be part of the normative work.

NOTE 2: it is the responsibility of the MNO to check the validity and accuracy of the external data; this is not in the scope of the present document.

The potential solution #3 proposes to introduce information element(s) to enable the operator to associate the power supply information to 5G network functions (represented by ManagedFunction).

This solution is feasible by providing configuration options for the operator to provide the power supply types used by network functions, and corresponding planned usage periods to the 3GPP management system.

NOTE 3: The definition of the information elements (e.g., an IOC or attribute(s) representing this information) and how this is associated with the network functions (represented by ManagedFunction) will be part of the normative phase.

## 5.4	Use case #4: Exposure of carbon and renewable energy related information

### 5.4.1	Description

In this use case, authorized consumers collect, from the 3GPP management system, various per 5GC NF and per gNB energy related information, e.g. (non-exhaustive list):

- energy consumption measurements and/or KPIs;

- energy saving state;

- carbon and renewable energy related information:

-	carbon emission information;

-	carbon emission efficiency;

-	renewable energy factor.

### 5.4.2	Potential requirements

REQ-ENERGY_INFO_REPORT-CON-1: The 3GPP management system should have the capability allowing its authorized consumers to request to collect per 5GC NF energy related information such as energy consumption measurements and/or KPIs, energy saving state, carbon emission and renewable energy related information.

REQ-ENERGY_INFO_REPORT-CON-2: The 3GPP management system should have the capability allowing its authorized consumers to request to collect per gNB energy related information such as energy consumption measurements and/or KPIs, energy saving state, carbon emission and renewable energy related information.

### 5.4.3	Potential solutions

#### 5.4.3.1	Potential solution #1: Based on PerfMetricJob

##### 5.4.3.1.1	Introduction

In this potential solution, authorized consumers collect, from the 3GPP management system, energy related information by means of performance metrics jobs. In this case, an authorized consumer requests the 3GPP management system to create a PerfMetricJob instance in order to collect per 5GC NF and per gNB energy information.

##### 5.4.3.1.2	Description

The authorized Management Service consumer requests the 3GPP management system (as Management Service producer) to create a PerfMetricJob instance defined in clause 4.3.31 of 3GPP TS 28.622 [14], with the following input parameters:

- performanceMetrics, to indicate the expected performance metrics, i.e. measurements defined in 3GPP TS 28.552 [11] (e.g. PEE.Energy) and/or KPIs defined in 3GPP TS 28.554 [2] (e.g. ECNF);

- granularityPeriod, to indicate the granularity period to be applied to produce expected performance metrics;

- objectInstances, to indicate (if present) the object instances for which performance metrics are expected (e.g. it may contain a (list of) gNB Distinguished Name(s));

- rootObjectInstances, to indicate (if present) the subtrees whose root objects are identified by this parameter, for which performance metrics are expected (e.g. it may contain a (list of) SubNetwork Distinguished Name(s));

- reportingCtrl, to indicate the method and associated control parameters for reporting the produced measurements to the consumer. Three methods are available:

-	file-based reporting with selection of the file location by the MnS producer (i.e. 3GPP management system);

-	file-based reporting with selection of the file location by the MnS consumer;

-	stream-based reporting.

It should be noted that the creation of PerfMetricJob instances by MnS consumers (e.g. a 5GC NF) is optional; when not supported, PerfMetricJob instances may be created and deleted by the system or be pre-installed.

#### 5.4.3.2	Potential solution #2: Based on ManagementDataCollection

##### 5.4.3.2.1	Introduction

In this potential solution, authorized consumers collect, from the 3GPP management system, energy related information by means of management data services. In this case, the authorized consumer requests the 3GPP management system to create a ManagementDataCollection instance in order to collect per 5GC NF and per gNB energy information.

##### 5.4.3.2.2	Description

The authorized Management Service consumer requests the 3GPP management system (as Management Service producer) to create a ManagementDataCollection instance defined in clause 4.3.47 of 3GPP TS 28.622 [14], with the following input parameters:

- managementData: to indicate the management data which should be reported. This may either include a list of data categories (e.g. "ENERGY_EFFICIENCY"), or a list of management data identified with their name (e.g. PEE.Energy or ECNF);

- targetNodeFilter: to indicate the target object instance(s) producing the required management data. As it is likely that the 5GC NF (as consumer) may not have detailed knowledge of the network, it may not be able to identify the exact object instance producing the required management data. In this case, the consumer can request management data produced by certain object instance(s) based on:

-	a particular location (e.g. list of TAI, a list of cells (identified either by NG-RAN CGI, E-UTRAN CGI or UTRAN CGI) or by a geographical area); and/or

-	the domain of the object instances (e.g. CN or RAN); and/or

-	the type of traffic handled by the object instances (CP or UP);

- collectionTimeWindow: to indicate the time window for which the management data should be reported;

- reportingCtrl: to indicate the method and associated control parameters for reporting the produced management data to the consumer (i.e. the 5GC NF). Three methods are available:

-	file-based reporting with selection of the file location by the MnS producer (i.e. 3GPP management system);

-	file-based reporting with selection of the file location by the MnS consumer;

-	stream-based reporting;

- dataScope: to indicate whether the management data should be reported per S-NSSAI or per 5QI or per PLMN, if applicable.

#### 5.4.3.3	Potential solution #3: Retrieval of energy related information information associated with managed functions

##### 5.4.3.3.1	Introduction

The energy related information can be represented using MnS component type B as Network Resource Model (NRM) (see clause 4.2.3.1 of TS 28.533 [35]) or MnS component type C as performance information of the managed entity (see clause 4.2.3.1 of TS 28.533 [35]).

Energy saving state(s) is represented by MnS component type B, and energy consumption measurements and/or KPIs are represented by MnS component type C. Carbon and renewable energy related information can be represented by MnS component type B and MnS component type C.

This potential solution describes how an authorized MnS consumer can get the data represented by MnS component type B using MnS component type A (operations and notifications).

##### 5.4.3.3.2	Description

An authorized MnS consumer can get the energy related information that is represented as MnS component type B as Network Resource Model (NRM) using MnS component type A (operations and notifications), described as follows:

- Using getMOIAttributes operation (see clause 11.1.1.2 of TS 28.532 [36])

- By subscribing to notifications, using createMOI operation (see clause 11.1.1.1 of TS 28.532 [36]) with NtfSubscriptionControl IOC (see clause 4.3.22 of TS 28.622 [14]). The MnS consumer will notify the MnS consumer for the subscribed scope using the notifications notifyMOICreation, notifyMOIDeletion notifyMOIAttributeValueChanges and notifyMOIChanges (see TS 28.532 [36]).

### 5.4.4	Evaluation of potential solutions

The potential solution #1 and #2 describe how an authorized MnS consumer can get the data represented by MnS component type C, and potential solution #3 describes how an authorized MnS consumer can get the data represented by MnS component type B using MnS component type A. The 3GPP management system, supports the NRMs and operations proposed by the potential solutions.

The solutions proposed in clauses 5.4.3.1, 5.4.3.2 and 5.4.3.3 satisfy the requirements and these solutions are already available.

## 5.5	Use case #5: Estimating averaged gNB energy consumption per UE

### 5.5.1	Description

In this use case, the 3GPP management system measures averaged gNB energy consumption per UE, and exposes it to authorized consumers, including to the 5G core network.

### 5.5.2	Potential requirements

REQ-EC_GNB_UE-CON-1: The 3GPP management system should have the capability to measure the averaged gNB energy consumption per UE.

REQ-EC_GNB_UE-CON-2: The 3GPP management system should have the capability to report the averaged gNB energy consumption per UE to authorized consumers.

### 5.5.3	Potential solutions

None.

## 5.6	Use case #6: Multi-dimensional network energy efficiency metrics

### 5.6.1	Description

Telecommunication networks energy efficiency (EE) KPIs are defined by various SDOs and are of various natures. They can be applied to either:

-	whole networks (i.e. end-to-end); or to

-	sub-networks (e.g. radio access network); or to

-	single network elements; or to

-	telecommunication sites, which contain network elements and site equipment.

Generally, EE KPI is defined as the ratio between performance and energy consumption. The performance may be measured based on e.g. data volume, latency, number of active users, etc. Typical EE KPIs for network elements are expressed in terms of Data Volume divided by the Energy Consumption of the considered network elements.

The generic network slice EE KPI is defined as the ratio between the performance of network slice to the Energy Consumption of the network slice.

3GPP TS 28.310 [12] defines high-level mobile network data EE KPIs and solutions for assessment of mobile network EE. A number of different EE KPIs have been defined in 3GPP TS 28.554 [2] in previous 3GPP releases:

-	NG-RAN data Energy Efficiency.

-	Network slice Energy Efficiency.

-	5GC Energy Efficiency.

Most of the EE metrics (i.e. NG-RAN Energy Efficiency and 5GC Energy Efficiency) only consider the aspect of data volume, but not the other network performance aspects. For example, two gNBs may have similar EE evaluation results using the EE KPI nowadays which uses Data Volume (DV) as the traffic volume performance indicator, but the user experienced service quality (such as user throughput or coverage quality) of these two serving gNBs may vary largely.

Performance indicators can be classified into different dimensions. Here are some performance dimensions:

-	Network data traffic, usually a measure of the number or amount of network traffic provided. For example, the amount of data transferred as the Data Volume used in EE KPI, the number of connections supported, etc.

-	Network quality, which measures the quality of network provided, such as user throughput, coverage quality, or connection quality, etc.

-	Network availability, which measures the availability of network. A typical example in wireless networks is the cell availability.

To enhance the EE KPIs to more comprehensive in different network scenarios, it is needed to consider other performance dimensions.

### 5.6.2	Potential requirements

REQ-MDEE-CON-X: The 3GPP management system should have the capability to measure energy efficiency based on network data traffic.

REQ-MDEE-CON-Y: The 3GPP management system should have the capability to measure energy efficiency based on network quality.

REQ-MDEE-CON-Z: The 3GPP management system should have the capability to measure energy efficiency based on network availability.

### 5.6.3	Potential solutions

#### 5.6.3.1	Potential solution #1: EE KPIs evaluated from network data traffic performance dimension

##### 5.6.3.1.1	Introduction

In this potential solution, existing EE KPIs are considered from network data traffic performance dimension. It is no need to change the definition of the following existing EE KPIs.

##### 5.6.3.1.2	Description

Network data traffic performance, reflected from the following performance measurements

- The amount of data transferred as the Data Volume.

- the number of connections including:

-	The number of registered subscribers of MIoT network slice.

-	The number of active UEs in MIoT the network slice.

The existing EE KPIs evaluated from network data traffic performance dimension are captured in Table 5.6.3.1.2-1.

Table 5.6.3.1.2-1: Existing EE KPIs evaluated from network data traffic performance dimension

| Performance dimension to evaluate EE KPIs |  | Existing EE KPIs | Reference to the clause number in 3GPP TS 28.554 [2] for the definition of EE KPIs | Note |
| --- | --- | --- | --- | --- |
| Network data traffic | Data Volume (DV) as the network data traffic performance indicator | NG-RAN data Energy Efficiency | 6.7.1 | This EE KPI shows mobile network data energy efficiency in operational NG-RAN. |
|  |  | Energy efficiency of eMBB network slice | 6.7.2.2 | This EE KPI shows the energy efficiency of network slices of type eMBB. |
|  |  | Energy efficiency of eMBB network slice - RAN-based | 6.7.2.2a | This EE KPI shows the energy efficiency of network slices of type eMBB based on NR measurements. |
|  |  | Energy Efficiency of the 5GC (Based on the useful output of 5GC user plane) | 6.7.4.2 | This EE KPI shows the energy efficiency of 5GC. This KPI is based on the useful output of 5GC user plane. |
|  | The number of connections as the network data traffic performance indicator | Energy efficiency of MIoT network slice (Based on the number of registered subscribers of the network slice) | 6.7.2.4.1 | This EE KPI shows the energy efficiency of network slices of type MIoT. |
|  |  | Energy efficiency of MIoT network slice (Based on the number of active UEs in the network slice) | 6.7.2.4.2 | This EE KPI shows the energy efficiency of network slices of type MIoT. |

#### 5.6.3.2	Potential solution #2: EE KPIs evaluated for a network from availability dimension

##### 5.6.3.2.1	Introduction

As defined in TS 28.310 [12], EE is the ratio between performance and energy consumption. In this potential solution, new EE KPIs are considered in terms of availability performance dimension for a network.

##### 5.6.3.2.2	Description

Availability KPIs exist as follows

-	Cell Availability.

-	Radio access network availability.

The proposed performance dimension of new EE KPIs, i.e. the numerator part of the formula of new EE KPIs, are evaluated from availability performance dimension for a network as reflected by some existing KPIs in 3GPP TS 28.554 [2], see Table 5.6.3.2.2-1.

Table 5.6.3.2.2-1: Network availability performance dimension from which new EE KPIs are evaluated

| Performance dimension to evaluate EE KPIs | Performance part of new EE KPIs | Reference to the clause number in 3GPP TS 28.554 [2] | Note |
| --- | --- | --- | --- |
| Availability | Cell Availability KPI | 6.10.1.1.1 | This availability KPI provides "average availability time duration per cell" in a gNB-CU. |
|  | Radio access network availability KPI | 6.10.1.1.2 | This availability KPI provides "average availability time duration per cell" of an entire RAN sub-network. |

The new EE KPIs, which are evaluated as the ratio between performance (see availability KPIs captured in Table 5.6.3.2.2-1) and energy consumption, are captured in Table 5.6.3.2.2-2.

Table 5.6.3.2.2-2: New EE KPIs evaluated from network availability performance dimension

| Performance dimension to evaluate EE KPIs |  | New EE KPIs | Note |
| --- | --- | --- | --- |
| Network availability | RAN Availability as the availability performance indicator | Radio access network availabilityECper cell of sub-network | This new EE KPI provides the energy efficiency of a RAN sub-network. The unit of this KPI is sec/J. This new EE KPI is obtained by dividing Radio access network availability KPI by the average energy consumption per cell of the RAN sub-network over the same observation period. The average energy consumption per cell of the RAN sub-network is obtained by considering summation of energy consumption of all gNB/CU in a RAN subnetwork for a given time period and dividing it by total number of cells in a RAN sub-network that are operational in same time period.. NG-RAN EC is defined in clause 6.7.3.4.1 of TS 28.554 [2]. |
|  | Cell Availability as the availability performance indicator | Cell availability divided by average energy consumption per cell in a gNB | This new EE KPI provides the energy efficiency of a cell in a gNB based on its availability performance. The unit of this KPI is sec/J.. This new EE KPI is obtained by considering Cell Availability KPI in numerator and dividing it by the average energy consumption per cell in a gNB over the same observation period. The average energy consumption per cell in a gNB is obtained by considering energy consumption of gNB for a given time period and dividing it by total number of cells in a gNB that are operational in same time duration/period. ECgNB is defined in clause 6.7.3.4.2 of TS 28.554 [2]. |

#### 5.6.3.3	Potential solution #3: EE KPIs evaluated from network quality dimension

##### 5.6.3.3.1	Introduction

As defined in TS 28.310 [12], EE is the ratio between performance and energy consumption. In this potential solution, new EE KPIs are considered from network quality performance dimension.

##### 5.6.3.3.2	Description

Network quality performance, reflected from the following performance measurements

- DL RAN UE throughput

- UL RAN UE throughput

NOTE: The network quality performance include other aspects such as coverage quality, or connection quality, etc. This solution focuses only on the user throughput aspect.

The UE perceived throughput in NG-RAN is an important performance parameter for operating 5G network. If the UE throughput of the NR cell cannot meet the performance requirement, some actions need to be performed to the network, e.g. reconfiguration or capacity increase. So it is necessary to define UE throughput KPI to evaluate whether the end-users are satisfied. See clause A.7 of TS 28.554 [2].

The proposed performance dimension of new EE KPIs, i.e. the numerator part of the formula of new EE KPIs, is evaluated from network quality performance dimension as reflected by some existing KPIs in TS 28.554 [2], see Table 5.6.3.3.2-1.

Table 5.6.3.3.2-1: Network quality performance dimension from which new EE KPIs are evaluated

| Performance dimension to evaluate EE KPIs | Performance part of new EE KPIs | Reference to the clause number in TS 28.554 [2] | Note |
| --- | --- | --- | --- |
| UE throughput | DL RAN UE throughput for a sub-network | 6.3.6.3.2 | This UE throughput based KPI describes the average DL RAN UE throughput for a sub-network. |
|  | DL RAN UE throughput for a network slice subnet | 6.3.6.3.3 | This UE throughput based KPI describes the average DL RAN UE throughput for a network slice subnet. |
|  | UL RAN UE throughput for a sub-network | 6.3.6.4.2 | This UE throughput based KPI describes the average UL RAN UE throughput for a sub-network. |
|  | UL RAN UE throughput for a network slice subnet | 6.3.6.4.3 | This UE throughput based KPI describes the average UL RAN UE throughput for a network slice subnet. |

The new EE KPIs, which are evaluated as the ratio between performance (see UE throughput KPIs captured in Table 5.6.3.3.2-1) and energy consumption, are captured in Table 5.6.3.3.2-2.

Table 5.6.3.3.2-2: New EE KPIs evaluated from network quality performance dimension

| Performance dimension to evaluate EE KPIs |  | New EE KPIs | Note |
| --- | --- | --- | --- |
| Network quality | UE throughput as the network quality performance indicator | DL RAN UE throughput for a sub-networkECper gNB of sub-network | This new EE KPI provides the energy efficiency of a sub-network. The unit of this KPI is kbit/(sec*J) This new EE KPI is obtained by DL RAN UE throughput for a sub-network KPI divided by the average energy consumption per gNB of the sub-network over the same observation period. The average energy consumption per gNB of the sub-network is obtained by summing up the Energy Consumption (ECgNB) of all the gNBs that constitute the sub-network and then dividing the result by the total number of gNBs present in the sub-network. ECgNB is defined in clause 6.7.3.4.2 of TS 28.554 [2]. |
|  |  | DL RAN UE throughput for a network slice subnetECper gNB of network slice subnet | This new EE KPI provides the energy efficiency of a network slice subnet. The unit of this KPI is kbit/(sec*J). This new EE KPI is obtained by DL RAN UE throughput for a network slice subnet KPI divided by the average energy consumption per gNB of the network slice subnet over the same observation period. The average energy consumption per gNB of the network slice subnet is obtained by summing up the Energy Consumption (ECgNB) of all the gNBs that constitute the network slice subnet and then dividing the result by the total number of gNBs present in the network slice subnet. ECgNB is defined in clause 6.7.3.4.2 of TS 28.554 [2]. |
|  |  | UL RAN UE throughput for a sub-networkECper gNB of sub-network | This new EE KPI provides the energy efficiency of a sub-network. The unit of this KPI is kbit/(sec*J). This new EE KPI is obtained by UL RAN UE throughput for a sub-network KPI divided by the average energy consumption per gNB of the sub-network over the same observation period. The average energy consumption per gNB of the sub-network is obtained by summing up the Energy Consumption (ECgNB) of all the gNBs that constitute the sub-network and then dividing the result by the total number of gNBs present in the sub-network. ECgNB is defined in clause 6.7.3.4.2 of TS 28.554 [2]. |
|  |  | UL RAN UE throughput for a network slice subnetECper gNB of network slice subnet | This new EE KPI provides the energy efficiency of a network slice subnet. The unit of this KPI is kbit/(sec*J). This new EE KPI is obtained by UL RAN UE throughput for a network slice subnet KPI divided by the energy consumption per gNB of the network slice subnet over the same observation period. The average energy consumption per gNB of the network slice subnet is obtained by summing up the Energy Consumption (ECgNB) of all the gNBs that constitute the network slice subnet and then dividing the result by the total number of gNBs present in the slice subnet. ECgNB is defined in clause 6.7.3.4.2 of TS 28.554 [2]. |

The existing EE KPIs evaluated from network quality and data traffic performance dimensions are captured in Table 5.6.3.3.2-3.

Table 5.6.3.3.2-3: Existing EE KPIs evaluated from network quality and data traffic performance dimensions

| Performance dimensions to evaluate EE KPIs |  | Existing EE KPIs | Reference to the clause number in TS 28.554 [2] for the definition of EE KPIs | Note |
| --- | --- | --- | --- | --- |
| Network quality and network data traffic | Latency as the network quality performance indicator and Data Volume (DV) as the network data traffic performance indicator in combined dimensions mode | Energy efficiency of URLLC network slice (Based on both latency and Data Volume (DV) of the network slice) | 6.7.2.3.3 | This EE KPI shows the energy efficiency of network slices of type URLLC. |

### 5.6.4	Evaluation of potential solutions

All potential solutions described in clause 5.6.3 can be used to enhance the EE KPIs to be more comprehensive in different network scenarios.

The potential solution #1, EE KPIs evaluated from network data traffic performance dimension, allows fulfilling the requirement REQ-MDEE-CON-X.

No need to introduce new EE KPIs. The EE KPIs listed in Table 5.6.3.1.2-1 can be classified under the category of network data traffic to form the overall multi-dimensional EE metrics together with other categories.

The potential solution #2, EE KPIs evaluated from network availability performance dimension, allows fulfilling the requirement REQ-MDEE-CON-Z.

The potential solution #3, EE KPIs evaluated from network quality performance dimension, allows fulfilling the requirement REQ-MDEE-CON-Y.

The potential solutions #1, #2 and #3 are feasible.

## 5.7	Use case #7: Renewable energy-based LBO

### 5.7.1	Description

To reduce the carbon emission caused by the communication network serving network services, the operator may decide to deploy base stations which are powered by renewable energy, such as gNB with solar PV (i.e. Photovoltaic). Therefore, there is the scenario where gNB powered by solar PV and gNB powered by Energy Grid both exist and their coverage area may overlap as shown in Figure 5.7.1-1. The Energy Grid provides non-fully renewable energy for gNB.

Figure 5.7.1-1: Neighbor gNBs powered by different energy types

Figure 5.7.1-1 illustrates the scenario where neighbour gNBs are powered by different energy types. The pre-condition is that the 3GPP management system is able to collect the energy source types of gNBs from entities who produce the energy to gNBs.

The gNB A uses fully renewable energy which is generated by its attached solar PV, while its neighbor, gNB B, uses the traditional powering method - Energy Grid, which may use fossil energy and emits carbon dioxide. Since the traffic load on gNB B is higher than gNB A, to make full use of renewable energy, the operator decides to transfer part of traffic load from gNB B to gNB A to reduce the energy consumption of gNB B and reduce its carbon emission.

NOTE:	The assumption for this use case is that all cells belong to the same gNB are powered by the same energy type.

### 5.7.2	Potential requirements

REQ-Renewable_energy-01: The 3GPP management system should have a capability allowing an authorized MnS consumer to give indication to gNBs to consider renewable energy information for load transferring.

REQ- Renewable_energy-02: The 3GPP management system should be able to identify gNB which is powered by renewable energy.

### 5.7.3	Potential solutions

#### 5.7.3.1	Potential solution #1

##### 5.7.3.1.1	Introduction

In this potential solution, MnS producer (e.g. D-SON management function) collects renewable energy related information from external entity (e.g. energy supplier), such as carbon emission, carbon emission efficiency and provides the LBO management service with consideration of renewable energy related information.

NOTE:	How the MnS producer gets the renewable energy related information is out of the scope of present study.

##### 5.7.3.1.2	Description

1)	MnS consumer sends a renewable energy usage policy to MnS producer, which indicates the high priority of transferring load to the gNB that is powered by renewable energy if there are multiple candidates when performing load balancing optimization. To support this capability, the enhancement could add one parameter (i.e. dlboRenewableControl, as shown in the Table 5.7.3.1.2-1) for IOC DLBOFunction defined in 3GPP TS 28.541 [16], clause 4.3.69, which indicates whether the LBO should be done with consideration of renewable energy when the operator enables the LBO functionality.

Table 5.7.3.1.2-1: New attribute for existing IOC DLBOFunction

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dlboControl | M | T | T | F | T |
| maximumDeviationHoTriggerLow | M | T | T | F | T |
| maximumDeviationHoTriggerHigh | M | T | T | F | T |
| minimumTimeBetweenHoTriggerChange | M | T | T | F | T |
| dlboRenewableControl | CO | T | T | F | T |

The condition for dlboRenewableControl is that the renewable energy is used by gNBs.

NOTE 1:	Only the last attribute is new.

Table 5.7.3.1.2-2

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| dlboRenewableControl | This attribute determines whether the LBO should be done with consideration of renewable energy when the operator enables the LBO functionality. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

NOTE 2:	How this new parameter impacts the procedure of RAN Load balancing action is For Further Study.

2)	MnS producer collects renewable energy related information and takes it into consideration when updating LBO configurations. Figure 5.7.3.1.2-1 shows the LBO between gNB A and gNB B. The gNB A and gNB C are both candidates to receive traffic load transferred from gNB B. The only difference between gNB A and gNB C is that gNB A is fully powered by renewable energy. With the guidance of renewable energy usage policy, gNB B gives gNB A higher priority and chooses gNB A. To support this capability, the enhancement could be for procedure of SON LBO described in 3GPP TS 28.313 [13], clause 8.2.4, the performance measurements related to LBO may include renewable energy related information, such as carbon emission, carbon emission efficiency and renewable energy usage.

Figure 5.7.3.1.2-1: LBO between gNB A and gNB B

### 5.7.4	Evaluation of potential solutions

The solution described in clause 5.7.3.1 provides the enhancement for IOC DLBOFunction to allow the MnS consumer to express the indication on transferring loads to gNBs that are powered by renewable energy when the LBO is triggered. The procedures by which the MnS producer gets the renewable energy related information and how RAN Load balancing actions are performed are not described in the solution, as they are out of the scope of the present study.

As the only solution for use case Renewable energy-based LBO, it involves simple extensions to the NRM which are implementable. Therefore, the solution described in clause 5.7.3.1 is a feasible solution.

## 5.8	Use case #8: Energy saving by converting QoS of a service

### 5.8.1	Description

5G system architecture characterize the 5G services with the QoS model. 5G QoS model is based on QoS flows. A QoS Flow ID (QFI) identifies a QoS flow in the 5G system where same traffic forwarding treatment is applied to the user plane traffic with the same QFI. Each QoS flow is specified by a set of QoS parameters and QoS characteristics in 3GPP TS 23.501 [17]. Energy-saving service can be realized by lowering some of the QoS requirements for the service. For example, any service identified by 5QI=X1 requiring high throughput can be reconfigured into a Energy-saving service by adjusting the QoS parameters such as Guaranteed Flow Bit Tate (GFBR) and Maximum Flow Bit Rate (MFBR) to lower down throughput requirements for the network. Alternatively, a service's all QoS criteria can be removed turning that service into a best-effort traffic (3GPP TS 22.261 [7]). This modification of QoS criteria would result in reduced performance requirements for the network and hence, resulting in reduced energy consumption. Thus a Energy saving method can be based on defining an Eco-friendly Energy-saving service and dynamically switching users to such service when a significant energy saving is predicted in network. Energy-saving service will only be utilized when a much larger energy saving is predicted by the network, so only during limited time intervals.

This can also be used in extending the time for which a network node remains in energy saving mode. A cell energy saving mode time is depicted in Figure 5.8.1-1, where a capacity cell is put to energy saving mode when the load is lower than the energy saving mode threshold. An adaptable QoS threshold is defined which will specify a threshold at which service(s) can be converted into Energy-saving service. When cell load is above the energy saving mode threshold but lower than adaptable QoS threshold, user's service QoS parameters can be reconfigured so that they utilize less resources in the cell. Consequently, cell load is expected to reduce down to the energy saving mode threshold earlier and the cell is put to energy saving mode earlier to further increase energy saving. Figure 5.8.1-1 illustrates the cell load reduction due to adaptable QoS reconfiguration and the extended time of energy saving mode for a cell.

Figure 5.8.1-1

### 5.8.2	Potential requirements

REQ-Energy_Saving_AdaptQoS-ES-1: The management service producer responsible for energy saving should have capability enabling authorized consumers to request for updating an existing 5QI characteristics available with 5GS for the purpose of energy saving.

NOTE 1:	An existing service can be modified to save energy by lowering some of its QoS parameters. For example, any service identified by 5QI=X1 (where X1 indicates an operator defined 5QI number) requiring high throughput can be reconfigured by adjusting its QoS parameters such as Guaranteed Flow Bit Rate (GFBR) and maximum flow bit rate (MFBR) in order to reduce performance requirements on the network. Alternatively, a service's QoS criteria can be removed all together turning that service flow into a best-effort traffic.

REQ-Energy_Saving_AdaptQoS-ES-2: The management service producer responsible for energy saving should have capability enabling authorized consumers to configure a load threshold which when crossed, the 5QI characteristics should be updated.

NOTE 2:	Load can be related with for example a cell load, PDU session load, edge server load etc.

Editor's Note: Usage of threshold monitor and generic provisioning MnS for the solutions of this use case is FFS.

### 5.8.3	Potential solutions

None.

## 5.9	Use case #9: Renewable energy enabling 5GC NF re-selection

### 5.9.1	Description

To cut down on emissions and increase network efficiency, operators have an interest in powering their network using renewable energy sources. It is also important for operators to understand and track the proportion of energy consumed in their networks that is sourced from renewable sources, which can be utilized by the operators to decide to deploy 5GC NFs which can be powered by renewable energy, such as optimizing the energy utilization of UPFs, which powered by either renewable or non-renewable energy sources.

By incorporating UPFs that utilize renewable energy sources, the 3GPP management system can manage and adjust the working mode of the UPFs (e.g. turn on or turn off), which are powered by either renewable or non-renewable energy sources. This can allow the 5GC to utilize renewable energy for optimization.

### 5.9.2	Potential requirements

REQ-5GC renewable_energy-01: The 3GPP management system should be able to report the renewable energy consumption information of a 5GC NFs to an authorized MnS consumer.

### 5.9.3	Potential solutions

#### 5.9.3.1	Potential solution #1: Introduce a new attribute

##### 5.9.3.1.1	Introduction

This potential solution proposes to have an attribute to indicate whether 5GC NFs are powered by renewable energy and which types of renewable energy sources these 5GC NFs utilize.

##### 5.9.3.1.2	Description

The attribute may have to be added to the ManagedFunction IOC, to indicate whether the corresponding 5GC NF is powered by renewable energy and which types of renewable energy sources the 5GC NF utilizes.

Whether the functionality offered by this attribute is to be supported by all types of 5GC NFs will be addressed in the normative phase.

This potential solution will leverage on the potential solution(s) from use case #3.

### 5.9.4	Evaluation of potential solutions

The solution described in clause 5.9.3 proposes to use the potential solutions from use case #3 to enable an authorized MnS consumer to get information on whether the 5GC NF is powered by renewable energy and which types of renewable energy sources the 5GC NF utilizes. The potential solution described in clause 5.9.3 fulfills the requirement REQ-5GC renewable_energy-01 and is a feasible solution.

## 5.10	Use case #10: Deployment of Network Slices depending on the energy source of the operator site

### 5.10.1	Description

In a Service Provider network, the operator sites are supplied with a different mix of energy sources, that can be renewable or not. This renewable energy might come from the electricity grid or from local infrastructure in the operator site. Prioritizing those operator sites that use renewable sourced energy would have an immediate positive impact in potentially reducing the cost of energy, in case such renewable energy is sourced locally, and also in reducing the carbon footprint. It can be either the Service Provider or the actual customer who decides to request a minimum percentage of renewable sourced energy.

### 5.10.2	Potential requirements

REQ-ENERGY-RENEWABLE-1: The 3GPP management system shall be able to consider as a requirement when a slice is deployed the percentage of energy from a renewable source in an operator site.

### 5.10.3	Potential solutions

#### 5.10.3.1	Potential solution #1

In the provision stage of the Network Slice a new parameter will be added stating the minimum percentage of renewable sourced energy that the slice will use. For knowing where the Network Slice can be deployed a label would be assigned per operator site that would have the information about the percentage of renewable sourced energy and its origin (either local or from the electrical grid) The 3GPP management system would take the requested percentage of renewable sourced energy and will check in the inventory which operator sites will be the ones where this requirement can be fulfilled. That will trigger the feasibility check of the network slice to check the rest of parameters before providing the final feasibility and reservation conclusion.

The properties of the network slice related requirements that should be supported by a NetworkSlice instance in a 5G network are covered in the ServiceProfile data type (3GPP TS 28.541 [16]). The GST (generic slice template) defined by GSMA and the service performance requirements defined in 3GPP TS 22.261 [7] and 3GPP TS 22.104 [18] are all considered as input for the network slice related requirements. An NSP may add additional requirements not directly derived from SLA's, associated to the NSP internal goals as it can be seen in Figure 5.10.3.1-1.

Figure 5.10.3.1-1: From GST to S-(/P-)NEST based product ordering

The use of renewable sourced energy can be an additional requirement. That would mean that in the provisioning stage the operator site with a sufficient percentage of renewable sourced energy will be chosen.

For defining it a new parameter would be added in the ServiceProfile.

The ServiceProfile input is translated into SliceProfile attribute of NetworkSliceSubnet IOC that can follow this criteria.

As stated in 3GPP TS 28.541 [16], The NetworkSliceSubnet can be categorized by following types:

-	RANSliceSubnet represent the RAN network slice subnet in a 5G network, which is associated to one or multiple "RANSliceSubnetProfile".

-	CNSliceSubnet represent the CN network slice subnet in a 5G network, which is associated to one or multiple "CNSliceSubnetProfile".

-	TopSliceSubnet represent the top network slice subnet in a 5G network, which is associated to one or multiple "TopSliceSubnetProfile".

There might not be much flexibility in the RAN as the slice might cover specific areas so this translation might not happen for RAN, but in the CN there is more flexibility for allocating the slice where the percentage of renewable sourced energy is enough for fulfilling the parameter as part of the feasibility process.

It is proposed to include in the list of requirements in the ServiceProfile a new one with the following parameter as an implementation example.

Table 5.10.3.1-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| RenewableEnergyPercentage | M | T | T | F | T |

### 5.10.4	Evaluation of potential solutions

The solution described in clause 5.10.3.1 provides the possibility of requesting the minimum percentage of renewable energy required for a slice when it is deployed. It gives the management system the information for taking the needed decisions to fulfil the requirement.

The potential solution #1 fulfils the requirement REQ-ENERGY-RENEWABLE-1. This is a feasible candidate as input to the normative phase.

## 5.11	Use case #11: Handling of power shortages

### 5.11.1	Description

In this use case, the 3GPP network management system becomes aware of current or upcoming power shortages. The network operator in turn takes actions (e.g. at service, network and/or resource level) to ensure the delivery of communication services such as emergency calls, first responder communication, voice calls etc. while reducing energy use.

### 5.11.2	Potential requirements

REQ-Energy_Shortage_Handling-01: The 3GPP management system, in case it is made aware of current energy shortages or upcoming energy shortages, should be able to take appropriate measures to conserve energy use in the network while ensuring delivery of important communication services.

NOTE:	Examples for communication services that need to be preserved include emergency calls, first responder communication, voice calls etc.

### 5.11.3	Potential solutions

#### 5.11.3.1	Potential solution #1: Service adjustment for energy saving, based on available energy information

##### 5.11.3.1.1	Introduction

In this potential solution, it is proposed to enhance the MDA assisted Energy Saving use case (see clause 7.2.4 of TS 28.104 [33]) to consider the power shortage information, available energy from the backup energy, and re-use the existing inputs such as traffic trends and provides service adjustment recommendations for different periods of time, and thus conserving available energy. This solution is applicable for the network connected to power source(s) (grid electricity, Backup energy and locally generated energy (see clause 6 of ITU-T Recommendation L.1333 [20])) and for the situations when the power outages and energy shortage information from the power source is available in advance.

NOTE: This solution does not consider LTE network elements involved in providing the communication services, i.e., it is applicable for 5G SA scenarios.

##### 5.11.3.1.2	Description

MDA assisted Energy Saving use case is defined in clause 7.2.4 of TS 28.104 [33], and the corresponding data definitions are defined in clause 8.4.4 of TS 28.104 [33]. This potential solution proposes enhancement to this MDA use case.

It is proposed to enhance the enabling data for this MDA use case with information on power shortage and outage information and enhance the analytics output from this MDA use case with service adjustment recommendations for different periods of time thus conserving available energy to extend network service lifetime.

Power shortage, outage information and available energy information (in case of backup battery-based power supply) of the power supply associated with the managed functions is needed as input from the operator. The operator should be able to associate this information for the managed functions. The available energy information for the power supply can be associated using the information that is available outside of 3GPP (e.g., Estimated remaining battery autonomy (time) during discharge as per Annex B of ETSI ES 202 336-11 [31] and alarms State of Charge (SoC) low, State of Health (SoH) abnormal, Battery discharge as per Annex A of ETSI ES 202 336-11 [31]).

NOTE 1:	This potential solution assumes that the power supply information is associated with the network elements and network functions (see clause 5.3.3.3).

NOTE 2:	This potential solution assumes that energy information for the power supply, such as e.g. the information defined in ETSI ES 202 336-11 [31] can be made available to the 3GPP management system. Which interface(s) may be used is out of scope of the present document.

NetworkSlice (see clause 6.3.1 of TS 28.541 [16]) represents the 5G services deployed as network slice, while ServiceProfile (see clause 6.3.3 of TS 28.541 [16]) data type includes attributes that can represent service SLA requirements. Hence, the service adjustment recommendations can be represented using the ServiceProfile.

The enhancements to the enabling data and analytics output for the MDA assisted Energy Saving use case (see clause 8.4.4 of TS 28.104 [33]) are described below.

Extensions to the enabling data for this MDA use case are described in table 5.11.3.1.2-1.

Table 5.11.3.1.2-1: MDA enabling data extensions

| Data category | Description | References |
| --- | --- | --- |
| Configuration data | Power shortage, outage information of the power supply associated with the managed functions, i.e., associated with MOIs of the network functions (gNBs, cells, UPFs). |  |
|  | Available energy information (in case of back-up battery-based power supply) of the power supply associated with the managed functions | Estimated remaining battery autonomy (time) during discharge as per Annex B of ETSI ES 202 336-11 [31] |
| Performance measurements | Energy consumption KPIs as listed below: - NF Energy Consumption (EC) - Network Slice Energy Consumption (EC) - NG-RAN Energy Consumption (EC) | TS 28.554 [2] |
| Alarm notifications | alarms State of Charge (SoC) low, State of Health (SoH) abnormal, Battery discharge of the backup battery. | alarms State of Charge (SoC) low, State of Health (SoH) abnormal, Battery discharge as per Annex A of ETSI ES 202 336-11 [31] |

Extensions to the analytics output from this MDA use case are described in table 5.11.3.1.2-2.

Table 5.11.3.1.2-2: MDA analytics output extensions

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| networkServiceOperabilityRecommendations | This field holds the recommended actions of network service levels based on energy availability to conserving available energy and to extend network service lifetime. The recommended action may be (but not limited to): - Re-configuration of SLAs for the services and network slices, i.e., NetworkSlice IOC (see TS 28.541 [16]). The updated SLA can be expressed as ServiceProfile (see TS 28.541 [16]). The recommended values in ServiceProfile could indicate different levels of SLAs (i.e., the recommended QoS) for different time periods (expressed using attribute sliceAvailability) and applicable for entire network or parts of the network (expressed using coverageArea) - Modification of network slices, network slice subnets and/or individual 3GPP NFs, such as those deriving into scaling down VNF instance as virtualized parts of a 3GPP managed function (see TS 28.531 [34]). | M | type: Recommended3GPPAction multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

### 5.11.4	Evaluation of potential solutions

The potential solution #1 proposes to extend the MDA assisted Energy Saving use case to utilize power shortage and outage information, and the available energy from the backup energy that is associated with the network functions, along with network analytics and to provide recommendations to ensure the delivery of communication services for different time periods thus conserving available energy to extend network service lifetime.

This solution is feasible by identifying that power shortage information needs to be associated with the managed functions in the 3GPP management system and by using MDA for providing the recommendations.

NOTE 1:	The definition of the information elements (e.g., an IOC or attribute(s) representing the information) related to power shortage and outage information and how this is associated with the network functions (represented by ManagedFunction) will be part of the normative phase.

NOTE 2:	This potential solution assumes that energy information for the power supply can be acquired using interfaces defined outside of 3GPP, i.e., ETSI ES 202 336-11 [31]. If external interfaces can be consumed by the 3GPP management system, or if this information needs to be collected by the MNO and configured into the 3GPP management system will be part of normative phase.

NOTE 3:	This potential solution applicable for the scenario where network elements have backup battery system. The general solution for this use case will be for future study.

## 5.12	Use case #12: Cell proximity-based energy saving

### 5.12.1	Description

#### 5.12.1.1	Overview

TS 28.310 defines a capacity booster cell which can be deactivated when the load is low. For a capacity booster cell to be deactivated, a candidate cell should have adequate overlap with the capacity booster cell to make sure that deactivation of the booster cell does not lead to a coverage hole. A candidate cell may have several capacity booster cells that overlap with its coverage and can be deactivated. This applies when several capacity booster cells are added to improve the capacity of an area covered by a larger coverage cell.

NOTE: "Overlap" here is the generalization of "overlaid" as used in TS28.310, overlaid means that the area covered by the small cell is also fully covered by the larger cell while overlap means that the area covered by the small cell can be partly or fully covered by the larger cell.

Example: A candidate cell has 2 or more capacity booster cells that are associated with it, say on the different sides of the candidate cell's border.  For cell deactivation in this scenario, there is need to decide which of those 2 or more capacity booster cells should be deactivated and in which order.

An energy saving function, e.g., the "domain-centralized ES function", needs to select the best among the capacity booster cells to be deactivated at a given time. The energy saving function should be enabled to support the selection for and sequencing of deactivation of the capacity booster cells.

The proposed cell-level activation and deactivation is for centralized energy saving.

#### 5.12.1.2	Degree of neighborliness

To activate or deactivate cells, an energy saving function (be it the Domain-centralized ES function, see TS 28.310 [12]) computes the degree to which the coverage of cells overlap among each other to ensure that the coverage of a deactivated cell (the capacity booster cell) is compensated by a candidate cell. The degree of neighborliness measures and characterizes the degree to which a capacity booster cell overlaps with the candidate cell. The energy saving function determines the degree of neighborliness among the cells. Alternatively, the degree of neighborliness can be configured onto the energy saving function, e.g., by the operator or another function. As input, the entity computing the degree of neighborliness takes 1) the cell configuration data including transmit powers, applied antennas characteristics and tilts; 2) the propagation characteristics like building information; and 3) performance information like cell handover statistics.

NOTE: the degree of neighborliness is information contained by the energy saving function, it does not need to be defined in the RAN.

#### 5.12.1.3	Cell grouping and categorization

To optimize the selection of cells to be deactivated or reactivated, the energy saving function needs to group the cells that need to be treated togther for energy saving into a group, say called the Energy Saving Cell Group. Within the Energy Saving Cell Group, the cells are categorized into two sets - the primary candidate cells which are not deactivated and provide primary coverage and capacity booster cells which only enhance capacity and can thus be deactivated when the load is low. The entity computing the cell grouping can inform  the operator or an energy saving function about the group of cells that need to be treated together, i.e., the Energy Saving Cell Group and the categorization of cells either as a primary candidate or a capacity booster cell., Where the energy saving function does not have the capability to compute the Energy Saving Cell Group, it should be possible  to configure the Energy Saving Cell Group and cell categorisations to the energy saving function.

NOTE: the Energy Saving Cell Group is quasi-static information on the energy saving function, when configured it remains unchanged until reconfigured.

#### 5.12.1.4	Cell deactivation sequence

When selecting cells to deactivate, the energy saving function considers the load in a set of cells to then decides the most optimal cell(s) to be deactivated and the order in which they can be deactivated. Based on the degree of neighborliness among cells as well as the cell grouping and categorization as primary candidate or capacity booster cells, the the energy saving function can compute the order in which the cells can be deactivated. The energy saving function can inform the e.g., the operator of the cell deactivation order, e.g., by indicating each capacity booster cell’s rank in the deactivation process. The an Operator can be enabled to request for such a deactivation order. Where the energy saving function does not have the capability to determine the cell deactivation order, another function(e.g. an AIML inference function) can be enabled to configure the cell deactivation order onto the energy saving function.

Given the cell deactivation order, the energy saving function routinely compares the load in the set of cells (the Cell Group load) to the low and high load thresholds to decide if deactivation can be executed. The Cell Group load needs to be computed for the set of active cells. Moreover, an automaton function can be enabled to configure the activation or deactivation load thresholds.

NOTE 1: the term deactivation is used to represent a total switching off of the cell or any disabling of a part of the cell, e.g., a disabling of the cell's power amplifier.

NOTE 2: the activation sequence is expected to be the opposite order to the deactivation sequence, but whether extra details are required for the activation sequence will be addressed in the normatve work.

NOTE 3: For all the input information, the energy saving function may be configured by or based on the output of other automation functions including MDA functions and AIML inferenc efunctions. For example, MDA may derive the cell grouping which is then configured onto the Energy saving function

### 5.12.2	Potential requirements

REQ-NES-01: The 3GPP management system should have a capability allowing an authorized MnS consumer to configure an energy saving function with degree of neighborliness, describing the degree to which the coverage areas of any candidate neighbour cells overlap with each other.

REQ-NES-02:	The 3GPP management system should have a capability allowing an authorized MnS consumer to configure an energy saving function with information on the group of cells that need to be treated together (called the Energy Saving Cell Group) when selecting cell to be activated or deactivated.

NOTE REQ-1: Configuration is needed only if the energy saving function cannot compute the Energy Saving Cell Groups.

REQ-NES-03:	The 3GPP management system should have a capability allowing an authorized MnS consumer to configure an energy saving function with information on the cell categories indicating candidate cells and capacity booster cells.

NOTE REQ-2: Primary candidate cells are those that provide primary coverage and are not deactivated while capacity booster cells are those which only enhance capacity and can thus be deactivated when the load is low.

REQ-NES-04:	The 3GPP management system should have a capability allowing an authorized MnS consumer to configure an energy saving function with information on the cell deactivation order which is the capacity booster cell’s rank in the deactivation and reactivation process indicating the order in which the cells in a given area should be deactivated.

NOTE REQ-3: Configuration is needed only if the energy saving function does not determine the cell deactivation order.

REQ-NES-05:	The 3GPP management system should have a capability allowing an authorized MnS consumer to configure an energy saving function with the deactivation and reactivation load thresholds.

### 5.12.3	Potential solutions

#### 5.12.3.1	Potential solution #1: AI/ML-assisted Network Energy Saving

##### 5.12.3.1.1	Introduction

The centralized energy saving function can compute the sequence of deactivation of cells, e.g. with support from AIML inference functions or MDA Mns services. Based on the sequence, the ES function determines the cells to be deactivated at a given time. The deactivation of cells can be executed by the centralized energy saving function.

NOTE: This solution needs to be improved in the normative work with details about the usage of the information elements

### 5.12.4	Evaluation of potential solutions

Only one solution in clause 5.A.3.1 is provided for this use case. The solution needs further elaboration in the normative work.

NOTE: the use case, requirements and solution are a baseline which can be enhanced or extended in the normative work, e.g. to address support for an activation sequence if different from the deactivation sequence.

## 5.13	Use case #13:  Per-network slice gNB and 5GC NF Energy Consumption

### 5.13.1	Description

gNBs as well as 5GC network functions may serve multiple network slices. In this context, part of the energy consumption of gNBs and 5GC NFs may be attributed to a network slice or to another network slice. To this respect, TS 28.554 [2] clause 6.7.3.3	 (Network Slice Energy Consumption (EC)) needs to be enhanced with definitions of KPIs for the energy consumption of gNB and 5GC NFs attributable to each network slice served by the gNB and 5GC NFs.

### 5.13.2	Potential requirements

REQ-EC_NS_SharedgNBs-CON-1: The 3GPP management system should be able to provide per-network slice energy consumption measurements of gNBs.

REQ-EC_NS-SharedNFs-CON-2: The 3GPP management system should be able to provide per-network slice energy consumption of 5GC NFs, at minimum for AMF, SMF, UPF and PSA UPF.

### 5.13.3	Potential solutions

#### 5.13.3.1	Potential solution #1: Enhancement of TS 28.554 clause 6.7.3.3

##### 5.13.3.1.1	Introduction

This potential solution proposes to update the Network Slice Energy Consumption KPI definition already existing in TS 28.554 [2].

##### 5.13.3.1.2	Description

In this potential solution, it is proposed to update the Network Slice Energy Consumption KPI definition already existing in TS 28.554 [2] clause 6.7.3.3.

In TS 28.554, the Network Slice Energy Consumption is obtained by summing up the Energy Consumption of all the Network Functions (ECNF) that compose the network slice.

In case a NF is shared between multiple network slices, the participation of the NF to the energy consumption of the network slice has to be estimated based on criteria which may depend on the type of NF.

To satisfy REQ-EC_NS_SharedgNBs-CON-1 and REQ-EC_NS-SharedNFs-CON-2, TS 28.554 [2] clause 6.7.3.3 has to be enhanced by mathematical formulas to calculate the participation of the gNB and 5GC NFs, at minimum AMF, SMF, UPF and PSA UPF, to the energy consumption of the network slice.

### 5.13.4	Evaluation of potential solutions

The potential solution #1 meets the potential requirements from clause 5.13.2 and is feasible.

# 6	Conclusions and recommendations

## 6.1	Use case #1: Estimation of containerized VNF/VNFC energy consumption

To satisfy the potential requirement #1, whether normative work is needed depends on the conclusion for the use case #2 - see clause 6.2.

## 6.2	Use case #2: Alternative option to obtain energy consumption of VNF/VNFC

The use case, potential requirements and potential solutions for alternatives for obtaining energy consumption of VNF/VNFC are described in clause 5.2.

The solution proposes to document different options for obtaining the energy consumption of VNF/VNFC based on:

- Estimation of energy consumption of VNFC based on resource usage metrics provided by ETSI NFV’s NFV-MANO (already supported by 3GPP TS 28.554 [2] in the case of VM-based VNF/VNFC),

- Collection of energy consumption of the OS container workloads or virtualized resource (VM) realizing the VNFC over a collection period as provided by ETSI NFV’s NFV-MANO,

- Collection of total energy consumption of the VNFC over the collection period as provided by ETSI NFV’s NFV-MANO, and

- Collection of total energy consumption of the VNF over the collection period as provided by ETSI NFV’s NFV-MANO.

The additional options are proposed to be documented in the relevant defined KPIs in the TS 28.554 [2] and TS 28.310 [12]. Where applicable, depending on the specific option, applicable 3GPP management system pre-conditions needs to be considered, like specified in clause 6.3.2 of TS 28.310 [12]. The further normative specification should also consider compatibility and/or alignments with updates that might be pursued to further support cloud-native and OS container-based deployments, where applicable.

It is recommended to specify the additional options for deriving the energy consumption of VNF/VNFC in the 28.554 [2] as part of the normative work.

## 6.3	Use case #3: Enabling renewable energy consumption and carbon emission information reporting

Potential solutions #1, #2 and #3

a) complement each other,

b) are feasible and

c) partially fulfill potential requirements REQ-ENERGY_INFO-CON-1, REQ-ENERGY_INFO-CON-2 and REQ-ENERGY_INFO-CON-3.

Whether and how they can completely fulfill the potential requirements is proposed to be addressed in the normative phase, inclusive of applicability of potential solutions #1, #2 and #3 to split gNBs and 5GC NFs.

## 6.4	Use case #4: Exposure of carbon and renewable energy related information

The use case, requirements and potential solutions for the exposure of carbon and renewable energy related information are described in clause 5.4. For normative work, it is proposed to document use case #4, related requirements and solutions in TS 28.310 [12].

## 6.5	Use case #5: Estimating averaged gNB energy consumption per UE

The use case and potential requirements for estimating the average gNB energy consumption per UE are described in clause 5.5. There is no potential solution for this use case. Therefore, no normative work is foreseen.

## 6.6	Use case #6: Multi-dimensional network energy efficiency metrics

The use case, requirements and potential solutions for multi-dimensional network energy efficiency are described in clause 5.6. It is recommended to add the use case and requirements in TS 28.310 [12] for normative work. Since other SDOs like ETSI EE are working on multi-dimensional network energy efficiency, it is recommended to continue investigating solutions in normative work.

## 6.7	Use case #7: Renewable energy based LBO

The use case, requirements and solution of renewable energy-based LBO is described in clause 5.7. In this use case, the MnS producer needs to support following capabilities:

- MnS producer can identify the gNB which is powered by renewable energy.

- An authorized MnS consumer can send renewable energy usage policy to MnS producer, which indicates gNBs to consider renewable energy information for load transferring.

The solution proposes to add one new attribute for IOC DLBOFunction defined in TS 28.541 clause 4.3.69 [16], which indicates whether the LBO should be done with consideration of renewable energy when the operator enables the LBO functionality.

It is recommended to add this use case and requirements in TS 28.310 [12] and take the solution as baseline for normative work. This solution has dependency with RAN WG for enhancement of LBO capability when it goes for normative work.

## 6.8	Use case #8: Energy saving by converting QoS of a service

The use case and potential requirement for energy saving by converting QoS of a service are described in clause 5.8. There is no potential solution for this use case. Therefore, no Rel-19 normative work is foreseen.

## 6.9	Use case #9: Renewable energy enabling 5GC NF re-selection

The use case, potential requirement and potential solution #1 for renewable energy enabling 5GC NF re-selection are described in clause 5.9. It is recommended to pursue the work in the normative phase.

## 6.10	Use case #10: Deployment of Network Slices depending on the energy source of the operator site

The use case, requirement and solution for deployment of network slices depending on the energy source of the operator site is described in clause 5.10. In this use case, it is described how to provide the requirement of the minimum percentage of renewable energy requested for a network slice that is being deployed.

It is recommended to add this use case and requirement in TS 28.310 [12] and take the solution as baseline for normative work. It is also recommended to add the new attribute proposed in the solution to TS 28.541 [16].

NOTE: The feasibility of this solution is dependent on the availability of the needed information at the 3GPP management system.

## 6.11	Use case #11: Handling of power shortages

The use case, requirement and solution for handling of power shortages. According the corresponding requirement defined in clause 5.11, the solution proposes an approach to extend the MDA assisted Energy Saving for network elements with backup battery.

It is recommended to extend MDA assisted Energy Saving use case in TS 28.104 [33] for normative work. It is also recommended to update NRM with information related to power shortage, outage information and available energy information, and how this is associated with the network functions (represented by ManagedFunction) will be part of the normative phase.

## 6.12	Use case #12: Cell proximity-based energy saving

The use case, requirement and solution for Cell proximity-based energy saving is described in clause 5.A. In this use case, four input information for energ saving is described including how the energ saving may be configured with that information. it is also described how an energy saving function may rely on an AIML inference function to obtain the four input information

It is recommended to add this use case and in TS 28.310 [12] and take the solution as baseline for normative work.

## 6.13	Use case #13: Per-network slice gNB and 5GC NF Energy Consumption

The use case, potential requirements and potential solution for per-network slice energy consumption KPI for gNBs and 5GC NFs are described in clause 5.13.

It is recommended to further elaborate the potential solution #1 in TS 28.554 [2] during the normative work

Annex A:
Rel-19 SA1 requirements on energy consumption / energy efficiency

Table A-1 provides the list of SA1 requirements with respect to 5G Energy Consumption/Energy Efficiency (see 3GPP TS 22.261 [7]) and identifies which of those may find a solution to be provided in Rel-19 by SA5 OAM.

Table A-1: Analysis of Rel-19 SA1 requirements on energy consumption / energy efficiency

| Id. | Requirement | 3GPP TS 22.261 [7], clause | In scope of SA5 OAM in Rel-19 | Observation |
| --- | --- | --- | --- | --- |
| Energy efficiency |  |  |  |  |
| 1 | The 5G access network shall support an energy saving mode with the following characteristics: - the energy saving mode can be activated/deactivated either manually or automatically; - service can be restricted to a group of users (e.g. public safety user, emergency callers). NOTE: When in energy saving mode the UE's and Access transmit power may be reduced or turned off (deep sleep mode), end-to-end latency and jitter may be increased with no impact on set of users or applications still allowed. | 6.15.2 | Yes | Already partially supported. - Energy saving mode already supported at cell level. - Restriction of service to a group of users for energy saving purposes is not supported. |
| 2 | The 5G system shall support mechanisms to improve battery life for a UE over what is possible in EPS. | 6.15.2 | FFS |  |
| 3 | The 5G system shall optimize the battery consumption of a relay UE via which a UE is in indirect network connection mode. | 6.15.2 | FFS |  |
| 4 | The 5G system shall support UEs using small rechargeable and single coin cell batteries (e.g. considering impact on maximum pulse and continuous current). | 6.15.2 | FFS |  |
| Energy related information as a service criteria |  |  |  |  |
| 5 | Subject to operator's policy, the 5G system shall support subscription policies that define a maximum energy credit limit for services without QoS criteria. | 6.15a.2.2 | No |  |
| 6 | Subject to operator's policy, the 5G system shall support a means to associate energy consumption information with charging information based on subscription policies for services without QoS criteria. | 6.15a.2.2 | No |  |
| 7 | Subject to operator's policy, the 5G system shall support a mechanism to perform energy consumption credit limit control for services without QoS criteria. NOTE 1: The result of the credit control is not specified by this requirement. NOTE 2: Credit control compares against a credit control limit. It is assumed charging events are assigned a corresponding energy consumption and this is compared against a policy of energy credit limit. It is assumed there can be a new policy to limit energy consumption allowed. | 6.15a.2.2 | No |  |
| 8 | Subject to operator's policy, the 5G system shall support a means to define subscription policies and means to enforce the policy that define a maximum energy consumption (i.e. quantity of energy for a specified period of time) for services without QoS criteria. NOTE 3: The granularity of the subscription policies can either apply to the subscriber (all services), or to particular services. | 6.15a.2.2 | No |  |
| 9 | The 5G system shall provide a mechanism to include Energy related information as part of charging information. | 6.15a.2.2 | No |  |
| 10 | Subject to operator policy and agreement with 3rd party, the 5G system shall provide a mechanism to support the selection of an application server based on energy related information associated with a set of application servers. | 6.15a.2.2 | FFS |  |
| 11 | Subject to user consent and operator policy, 5G system shall be able to provide means to modify a communication service based on energy related information criteria based on subscription policies. | 6.15a.2.2 | No | The use case and potential requirement for energy saving by converting QoS of a service are described in clause 5.8. There is no potential solution for this use case. Therefore, no Rel-19 normative work is foreseen. |
| 12 | Subject to user consent, operator policy and regulatory requirements, the 5G system shall be able to provide means to operate part or the whole network according to energy consumption requirements, which may be based on subscription policies or requested by an authorized 3rd party. | 6.15a.2.2 | Yes | Clause 5.11 Use case #11: Handling of power shortages |
| Support of different energy states |  |  |  |  |
| 13 | The 5G system shall support different energy states of network elements and network functions. | 6.15a.3.2 | Yes | Cell and UPF already support two energy saving state values. |
| 14 | 5G system shall support dynamic changes of energy states of network elements and network functions. NOTE: This requirement also includes the condition when providing network elements or functions to an authorized 3rd party, the dynamic changes can be based on pre-configured policy (the time of changing energy states, which energy state map to which level of load, etc.) | 6.15a.3.2 | Yes | Cell and UPF already support two energy saving state values. |
| 15 | The 5G system shall support different charging mechanisms based on the different energy states of network elements and network functions. | 6.15a.3.2 | No |  |
| Monitoring and measurement |  |  |  |  |
| 16 | Subject to operator's policy, the 5G network shall support energy consumption monitoring at per network slice and per subscriber granularity. NOTE 1: Energy consumption monitoring as described in the preceding requirement is done by means of averaging or applying a statistical model. The requirement does not imply that some form of 'real time' monitoring is required. The granularity of the subscription policies can either apply to the subscriber (all services), or to particular services. | 6.15a.4.2 | Partially | Per network slice EE KPI already defined. Per-subscriber EC/EE KPI is not in scope of SA5 OAM. |
| 17 | Subject to operator's policy and agreement with 3rd party, the 5G system shall be able to monitor energy consumption for serving this 3rd party. NOTE 2: The granularity of energy consumption measurement could vary according to different situations, for example, when several services share a same network slice, etc. NOTE 3: The energy consumption information can be related to the network resources of network slice, NPNs, etc. | 6.15a.4.2 | FFS |  |
| 18 | Subject to operator policy and regulatory requirements, the 5G system shall be able to monitor the energy consumption for serving the 3rd party, together with the network performance statistic information for the services provided by that network, related to same time interval e.g. hourly or daily. NOTE 4: The network performance statistic information could be the data rate, packet delay and packet loss, etc. | 6.15a.4.2 | Yes | Network performance statistic information for network slice see TS 28.552 [11]. |
| Information exposure |  |  |  |  |
| 19 | Subject to operator's policy and agreement with 3rd party, the 5G system shall be able to expose information on energy consumption for serving this 3rd party. NOTE 1: Energy consumption information can include ratio of renewable energy and carbon emission information when available. The reporting period could be set, e.g. on monthly or yearly basis and can vary based on location. NOTE 2: The energy consumption information can be related to the network resources of network slice, NPNs, etc. | 6.15a.5.2 | Yes | Clause 5.4 Use case #4: Exposure of carbon and renewable energy related information |
| 20 | Subject to operator's policy, agreement with 3rd party and consent by the customer, the 5G system shall be able to expose the network performance statistic information (e.g. the data rate, packet delay and packet loss) together with energy consumption information resulting from service provided to the customer, to the authorized third party, related to the same time interval e.g. hourly or daily. | 6.15a.5.2 | Yes | Network performance statistic information for network slice see TS 28.552 [11]. |
| 21 | Subject to operator's policy, the 5G system shall support a means to expose energy consumption to authorized third parties for services, including energy consumption information related to the condition of energy credit limit (e.g. when the energy consumption is reaching the energy credit limit). | 6.15a.5.2 | FFS |  |
| 22 | Subject to operator policy, the 5G system shall provide means for the trusted 3rd party, to configure which network performance statistic information (e.g. the data rate, packet delay and packet loss) for the communication service provided to the 3rd party, needs to be exposed along with the information on energy consumption for serving this 3rd party. | 6.15a.5.2 | FFS | Use case #4: Exposure of carbon and renewable energy related information enables network performance statistic information (e.g. the data rate, packet delay and packet loss) for the communication service provided to the 3rd party to be exposed along with the information on energy consumption for serving this 3rd party. |
| 23 | Based on operator's policy and agreement with 3rd party, the 5G system shall be able to expose energy consumption information and prediction on energy consumption of the 5G network per application service to the 3rd party. | 6.15a.5.2 | FFS |  |
| 24 | Subject to operator's policy and agreement with 3rd party, the 5G system shall support a mechanism for the 3rd party to provide current or predicted energy consumption information over a specific period of time. | 6.15a.5.2 | Partially | Use case #4: Exposure of carbon and renewable energy related information enables providing measured and estimated energy consumption information over a specific period of time. |
| Network actions leveraging energy efficiency as a service criteria |  |  |  |  |
| 25 | Subject to regulatory requirements and operators' policies, the 5G system shall enable an operator to temporarily serve UEs of other operators within a geographical area for the purpose of saving energy of the other operators. NOTE 1: The other operators are assumed to stop providing access to their own network infrastructure within the same geographical area to save energy during that time. NOTE 2: Policies may include predefined times/locations, energy consumption/efficiency thresholds, etc. NOTE 3: It is assumed that the 5G system can collect charging information associated with serving UEs of other operators. | 6.15a.6.2 | FFS |  |

Annex B:
Standard emission factors

Tables from annexes A.3 and A.4 of ETSI GS OEU 020 [8] providing standard emission factors are partially copy/pasted here below for information:

Figure B-1: ETSI GS OEU 020 [8] Standard emission factor per type of energy source

Annex C:
Types of energy related information

Energy related information identified in use case #3 can be classified in 3 types:

- Energy related information about the different types of energy source used by the MNO. Such information can generally be obtained from Standards Development Organizations (SDO) or government agencies or regulation bodies, etc. This information mainly includes:

- CEFi: Carbon Emission Factor for the type of energy source i

It is the responsibility of each MNO to collect this information by its own means. How the MNO obtains this information is not in the scope of the present document. An example of this information is the Standard Emission Factors given by ETSI GS OEU 020 [8], as depicted in the left-most column in the diagram below;

- Energy related information about energy consumption at operator site level, per type of energy source and indication about renewable / non-renewable energy type of energy source – see middle column. This information mainly includes:

- ECi: Energy Consumption corresponding to the type of energy source i used to power the operator site,

- ECSite: total Energy Consumption of the operator site, which can be calculated by summing up all ECi for a given site,

- CESite: Carbon Emission of the operator site, which can be calculated from CEFi and ECi (see clause 5.3.3.1.1.2 – Equation 1),

- ECSite,Ren: Energy Consumption of the operator site coming from renewable energy sources only,

- REFSite: Renewable Energy Factor of the operator site, which can be calculated from ECSite and ECSite,Ren (see clause 5.3.3.2.2.2 – Equation 2)

This information may typically be managed by the MNO Energy Management System. How the MNO Element Management System obtains this information is not in the scope of 3GPP

- Energy related information relative to the 3GPP system – see right-most column in the diagram below. In this version of the document, this information focuses on non-split gNB:

- ECNonSplit gNB: Energy Consumption, which is collected by the 3GPP management system,

- CENonSplitgNB: Carbon Emission, which can be calculated from ECNonSplit gNB, ECSite and CESite (see clause 5.3.3.1.2.2.2 – Equation 2),

- REFNonSplitgNB: Renewable Energy Factor, which can be obtained from REFSite (see clause 5.3.3.2.2.2 – Equation 1),

This information is to be managed by the 3GPP management system.

Annex D:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-12 |  |  |  |  |  | TR skeleton | 0.0.0 |
| 2024-02 | SA5#153 | S5-241058 |  |  |  | Clause titles for Key Issues and Potential Solutions | 0.1.0 |
| 2024-04 | SA5#154 | S5-241997 | pCR |  |  | New Use Case on Estimation of containerized VNF/VNFC energy consumption | 0.2.0 |
| 2024-04 | SA5#154 | S5-241998 | pCR |  |  | New Use case on New measurement method of VNF energy consumption | 0.2.0 |
| 2024-04 | SA5#154 | S5-242000 | pCR |  |  | Rel-19 pCR TR 28.880 Add use case and potential requirements to Enable renewable energy consumption and carbon emission information reporting | 0.2.0 |
| 2024-04 | SA5#154 | S5-242001 | pCR |  |  | New Use Case on new aspects of energy related information | 0.2.0 |
| 2024-04 | SA5#154 | S5-242004 | pCR |  |  | New Use Case on exposure of energy related information | 0.2.0 |
| 2024-04 | SA5#154 | S5-242056 | pCR |  |  | New Use Case on RAN Energy Consumption per UE | 0.2.0 |
| 2024-04 | SA5#154 | S5-242217 | pCR |  |  | New Use Case on multi-dimensional network energy efficiency metrics | 0.2.0 |
| 2024-06 | SA5#155 | S5‑243426 | pCR |  |  | Add potential solution for actual energy consumption of VNF | 0.3.0 |
| 2024-06 | SA5#155 | S5‑243240 | pCR |  |  | New Use Case on renewable energy based LBO | 0.3.0 |
| 2024-06 | SA5#155 | S5‑243249 | pCR |  |  | Update terms | 0.3.0 |
| 2024-06 | SA5#155 | S5‑243245 | pCR |  |  | Update use case enable renewable energy consumption and carbon emission information reporting | 0.3.0 |
| 2024-06 | SA5#155 | S5‑243246 | pCR |  |  | Update reqs and solutions for use case exposure of carbon and renewable energy related information | 0.3.0 |
| 2024-06 | SA5#155 | S5‑243247 | pCR |  |  | Update reqs for use case estimating averaged gNB energy consumption per UE | 0.3.0 |
| 2024-06 | SA5#155 | S5‑243248 | pCR |  |  | Add reqs for use case multi-dimensional network energy efficiency metrics | 0.3.0 |
| 2024-08 | SA5#156 | S5-244736 | pCR |  |  | Rel-19 pCR 28.880 Energy Saving Service | 0.4.0 |
| 2024-08 | SA5#156 | S5-244734 | pCR |  |  | pCR TR 28.880 Add use case of renewable energy enabling NF re-selection | 0.4.0 |
| 2024-08 | SA5#156 | S5‑244732 | pCR |  |  | pCR TR 28.880 Overall view of energy efficient networks | 0.4.0 |
| 2024-08 | SA5#156 | S5‑244902 | pCR |  |  | pCR TR 28.880 Update use case of multi-dimensional network energy efficiency metrics | 0.4.0 |
| 2024-08 | SA5#156 | S5‑244897 | pCR |  |  | pCR TR 28.880 Potential solution of EE KPIs evaluated from network data traffic performance dimension | 0.4.0 |
| 2024-08 | SA5#156 | S5‑244899 | pCR |  |  | pCR TR 28.880 Potential solution of EE KPIs evaluated from network availability performance dimension | 0.4.0 |
| 2024-08 | SA5#156 | S5‑244731 | pCR |  |  | pCR TR 28.880 Add scope | 0.4.0 |
| 2024-08 | SA5#156 | S5‑244900 | pCR |  |  | pCR TR 28.880 Rel-19 SA1 requirements on EC-EE | 0.4.0 |
| 2024-08 | SA5#156 | S5‑244738 | pCR |  |  | pCR TR 28.880 Potential solution of renewable energy based LBO | 0.4.0 |
| 2024-08 | SA5#156 | S5‑244737 | pCR |  |  | Enhance potential solution for obtaining energy consumption of VNF/VNFC | 0.4.0 |
| 2024-08 | SA5#156 | S5‑244735 | pCR |  |  | pCR TR 28.880 Add use case Network Slice deployment considering renewable sourced energy | 0.4.0 |
| 2024-08 | SA5#156 | S5‑244733 | pCR |  |  | use case for handling energy shortages / brown outs | 0.4.0 |
| 2024-09 | SA#105 | SP-241136 |  |  |  | Presented at SA#105 for Information | 1.0.0 |
| 2024-09 |  |  |  |  |  | After editHelp cleanup | 1.0.1 |
| 2024-10 | SA5#157 | S5‑245289 | pCR |  |  | pCR TR 28.880 Update potential solution of EE KPIs evaluated from network data traffic performance dimension | 1.1.0 |
| 2024-10 | SA5#157 | S5‑246113 | pCR |  |  | pCR TR 28.880 Potential solution of EE KPIs evaluated from network quality performance dimension | 1.1.0 |
| 2024-10 | SA5#157 | S5‑246029 | pCR |  |  | pCR TR 28.880 Conclusion and recommendation for renewable energy based LBO | 1.1.0 |
| 2024-10 | SA5#157 | S5‑246027 | pCR |  |  | pCR TR 28.880 New clause for conclusions and recommendations | 1.1.0 |
| 2024-10 | SA5#157 | S5‑245417 | pCR |  |  | pCR TR 28.880 Updates after the publication of ETSI GS NFV IFA 027 V5.2.1 | 1.1.0 |
| 2024-10 | SA5#157 | S5‑246030 | pCR |  |  | pCR TR 28.880 Potential solution for use case #3 - Carbon emission of non-split gNB | 1.1.0 |
| 2024-10 | SA5#157 | S5‑246031 | pCR |  |  | pCR TR 28.880 Potential solution for use case #3: Renewable Energy Factor of non-split gNB | 1.1.0 |
| 2024-10 | SA5#157 | S5‑246033 | pCR |  |  | pCR TR 28.880 Closure of use case 5 | 1.1.0 |
| 2024-10 | SA5#157 | S5‑246293 | pCR |  |  | Rel-19 pCR TR 28.880 Potential solution for use case Enabling renewable energy consumption and carbon emission information reporting | 1.1.0 |
| 2024-10 | SA5#157 | S5‑246273 | pCR |  |  | pCR TR 28.880 Correction to use case #10 | 1.1.0 |
| 2024-10 | SA5#157 | S5‑246274 | pCR |  |  | pCR TR 28.880 Conclusion and recommendation for the use case #10 | 1.1.0 |
| 2024-11 | SA5#158 | S5-246423 | pCR |  |  | Update titles of clause 6 sub-clauses | 1.2.0 |
| 2024-11 | SA5#158 | S5-247115 | pCR |  |  | Update annex A | 1.2.0 |
| 2024-11 | SA5#158 | S5-247116 | pCR |  |  | Editorial corrections_Edit-help | 1.2.0 |
| 2024-11 | SA5#158 | S5-247117 | pCR |  |  | Use case #2 updates and evaluation | 1.2.0 |
| 2024-11 | SA5#158 | S5-247118 | pCR |  |  | Potential solution, conclusion and recommendation for use case 1 | 1.2.0 |
| 2024-11 | SA5#158 | S5-247350 | pCR |  |  | Add use case on cell proximity-based energy saving | 1.2.0 |
| 2024-11 | SA5#158 | S5-246796 | pCR |  |  | Closure of use case 8 | 1.2.0 |
| 2024-11 | SA5#158 | S5-247120 | pCR |  |  | Potential solution for use case Handling of power shortages | 1.2.0 |
| 2024-11 | SA5#158 | S5-247121 | pCR |  |  | New use case for per-network slice gNB and 5GC NF EC | 1.2.0 |
| 2024-11 | SA5#158 | S5-247122 | pCR |  |  | Potential solution for use case Exposure of carbon and renewable energy related information | 1.2.0 |
| 2024-11 | SA5#158 | S5-247123 | pCR |  |  | Update evaluation of potential solutions, add conclusion and recommendation to use case 3 | 1.2.0 |
| 2024-11 | SA5#158 | S5-247165 | pCR |  |  | Add informative annex on types of energy related information | 1.2.0 |
| 2024-11 | SA5#158 | S5-247166 | pCR |  |  | TR 28.880 Add solution for renewable energy enabling NF re-selection | 1.2.0 |
| 2024-11 | SA5#158 | S5-247279 | pCR |  |  | Update potential solution of EE KPIs evaluated from network availability dimension | 1.2.0 |
| 2024-11 | SA5#158 | S5-246560 | pCR |  |  | Update potential solution of EE KPIs evaluated from network quality dimension | 1.2.0 |
| 2024-11 | SA5#158 | S5-247167 | pCR |  |  | Evaluation of potential solutions for multi-dimensional energy efficiency metrics | 1.2.0 |
| 2024-11 | SA5#158 | S5-247168 | pCR |  |  | Conclusion and recommendation for multi-dimensional energy efficiency metrics | 1.2.0 |