5GAA Working Group 4								WG4-243231

| 5GAA LIAISON STATEMENT |  |
| --- | --- |
| Title: LS on Physical Properties of Sensing Targets in Automotive Scenarios for ISAC |  |
| To: 3GPP RAN1 | Cc: - |
| Request for: Action/Information |  |
| Issuing date: 1 August |  |
| Source: 5GAA Liaison |  |
| 5GAA Work Item: WI ISAC |  |
| 5GAA Contact Person: #### Name: José Mauricio Perdomo Zelaya, David Gonzalez Gonzalez, Maxime Flament E-mail Address: jose.mauricio.zelaya@huawei.com, david.gonzalez.gonzalez@continental-corporation.com, maxime.flament@5gaa.org |  |
| Send any reply LS to: 5gaa-liaison@5gaa.org |  |

Attachments:

Annex

- Overall description:

5GAA is monitoring 3GPP discussion about ISAC in Rel-19, in particular, the Study Item on “Channel Modelling for Integrated Sensing and Communication (ISAC) for NR”. 5GAA wants to share its early findings with 3GPP to be further considered in 3GPP analysis with respect to physical characteristics of sensing targets in ISAC deployment scenarios relevant to automotive scenarios.

RAN1#116b established a template to define sensing deployment scenarios. The template includes physical characteristics of sensing targets (e.g., material and size). 5GAA wants to inform 3GPP about physical characteristics of the following sensing targets relevant for automotive scenarios: human outdoor, automotive vehicle and objects creating hazards on roads/railways.

The physical characteristics of sensing targets include the aspects indicated in Table 1, 2 and 3 in the Annex section. At the first stage, the focus is to detect unknown/unclassified objects which have physical properties as the sensing targets indicated in the Annex section.

- Actions:

5GAA respectfully asks 3GPP RAN to consider the above findings in this LS in Rel-19 channel modeling activities.

- Date of Next 5GAA WG4 Meetings:

| 5GAA WG4 meeting | October, 21 – 24, 2024 | Berlin, Germany |
| --- | --- | --- |

# Annex

Table 1. Physical Characteristics of Human Outdoor Sensing Target

| VRU ID | VRU Type | Size (Length x Width x Height) | Typical Velocity | Material | Remarks |
| --- | --- | --- | --- | --- | --- |
| A.1 | Pedestrian | 0.3m x 0.36m x 1.8m | 3 km/h | Mass of Water | At high frequency bands (e.g., @77 GHz), thickness of worn clothes influences significantly the RCS. |
| A.2 | E-Scooter | 1m x 0.5m x 1.9m | 20 km/h | Mass of Water including Aluminium | Height measured from ground to E-scooter deck plus road user height. Aluminium Pole with small RCS. |
| B.1 | Cyclist | 1.7m x 0.5m x 1.5m | (17-25) km/h | Mass of water including Steel | Dimensions and material composition based on road user riding non-electric bicycle. |
| B.2 | Motorcyclist | 2m x 0.8m x 1.5m | 60 km/h | Mass of Water including Steel | Dimensions based on road user riding motorcycle. Assuming speed from vehicle in urban V2X scenario. |

Table 2. Physical Characteristics of Automotive Vehicle Sensing Target

|  | Size (Length x Width x Height) | Typical Velocity | Material | Remarks |
| --- | --- | --- | --- | --- |
| Type 1/2 (passenger vehicle) | 5m x 2m x 1.6m | 16.67 m/s (60 km/h) | Steel | Assuming speed from vehicle in urban V2X scenarios. Signatures are L-shaped. |
| Type 3 (truck/bus) | 13m x 2.6m x 3m | 16.67 m/s (60 km/h) | Steel | Signatures are L-shaped. Aggregation is necessary to resolve as single sensing target. |

Table 3. Physical Characteristics of Objects creating Hazards on Roads/Railways Sensing Targets

|  | Size (Length x Width x Height) [m] | Typical velocity [km/h] | Material | Remarks |
| --- | --- | --- | --- | --- |
| Animal (Sheep/deer) | 1.5m x 0.5m x 1 m | 5 m/s (18 km/h) | Mass of Water | Typical size of sheep/deer as in 3GPP use case on pedestrian/animal intrusion detection on a highway in TR 22.837. |
| Obstacle (e.g., lost cargo) | (0.5-1) m x (0.5-1) m x 0.1m | Stationary object | Wood | The typical obstacle on roads is a fallen object like box which may impact HV’s manoeuvres. |
| Pothole | >0.25 m (horizontal dimension) >0.04 m (depth) | Stationary object | Asphalt | For a carriageway a pothole has been defined as a sharp-edged depression anywhere in the carriageway where part or all of the surface layers have been removed including carriageway collapses, surrounds to ironwork and missing cat’s eyes. |
| Precipitation (e.g., Smog, Rainfall) | N/A | 9m/s |  | Based on rainfall monitoring use case in TR 22.837. |