|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2022-2024 |  |  | SG15-TD4R1/PLEN |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | STUDY GROUP 15 |  |
|  |  |  |  | Original: English |  |
| Question(s): |  | 13/15 |  | Geneva, 19 – 30 September 2022 |  |
| TD |  |  |  |  |  |
| Source: |  | Editors, Recommendation G.781 |  |  |  |
| Title: |  | ITU-T Recommendation G.781 Amd1: Latest Draft (for consent) |  |  |  |
| Contact: |  |  | Geoffrey M. Garner Huawei Technologies Co., Ltd. China |  | Tel: +1 732 758 0335 Fax: +1 732 758 0335 Email: gmgarner@alum.mit.edu |  |
| Contact: |  |  | Jingfei Lv Huawei Technologies Co., Ltd. China |  | Email: lvjingfei@huawei.com |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |

Abstract

This document contains the latest draft of ITU-T Recommendation G.781, Amendment 1, based on agreements at the ITU-T Q13/15, Geneva, held 19 – 30 September 2022. Track changes are relative to the published ITU-T Recommendation G.781 (04/2020).

Note to TSB: There are several references in this document that are candidates for consent at Sept 2022 meeting.  The references have been updated to indicate the new revision dates of 2022 but may need to be adjusted based on actual date of publication.

Amendment 1 to Recommendation ITU-T G.781 Synchronization layer functions for frequency synchronization based on the physical layer – Amendment 1

Summary Recommendation ITU-T G.781 defines the atomic functions that are part of the two synchronization layers, the synchronization distribution (SD) layer and the network synchronization (NS) layer. It also defines some atomic functions, part of the transport layer, which are related to synchronization. These functions describe the synchronization of SDH, Ethernet, and OTN NEs and how these NEs are involved in network synchronization. The specifications in this Recommendation are the superset of functionality of three regional standards bodies. Care should be taken when selecting from this Recommendation. Not every atomic function defined in this Recommendation is required for every application. Different subsets of atomic functions may be assembled in different ways according to the combination rules given in Recommendations ITU-T G.783, ITU-T G.705, ITU-T G.8021, ITU-T G.8121, and ITU-T G.798 to provide a variety of different capabilities. Network operators and equipment suppliers may choose which functions must be implemented for each application. Amendment 1 to Recommendation G.781 provides the following updates: - Addition of FlexE to SD layer adaptation functions - Addition of FlexE layer clock adaptation functions - Minor corrections and clarifications.

Keywords Atomic function, functional model, network synchronization, network synchronization layer, synchronization distribution layer, quality level.

Amendment 1 to Recommendation ITU-T G.781

Synchronization layer functions for frequency synchronization 
based on the physical layer

Amendment 1

# 1	Scope

This Recommendation specifies a library of basic synchronization distribution building blocks, referred to as "atomic functions" and a set of rules by which they are combined in order to describe a digital transmission equipment's synchronization functionality. The library defined in this Recommendation forms part of the set of libraries defined furthermore in Recommendations [ITU-T G.783], [ITU-T G.705], [ITU-T G.798], [ITU-T G.8021], and [ITU-T G.8121]. The library comprises the functional building blocks needed to completely specify the generic functional structure of the ITU-T digital transmission hierarchy. Equipment that is compliant with these Recommendations should be describable as an interconnection of a subset of these functional blocks contained within these Recommendations. The interconnection of these blocks should obey the combination rules given in [ITU-T G.806]. The generic functionality is described in [ITU-T G.806].

The specifications in this Recommendation are the superset of functionality of three regional standards bodies. When different processing within a single atomic function is required to support the specific regional processing, this is identified by means of options I, II and III processing. "Option I" applies to networks optimized for the 2048 kbit/s hierarchy. "Option II" applies to networks optimized for the 1544 kbit/s hierarchy that includes the rates 1544 kbit/s, 6312 kbit/s and 44 736 kbit/s. "Option III" applies to networks optimized for the 1544 kbit/s hierarchy that includes the rates 1544 kbit/s, 6312 kbit/s, 33 064 kbit/s, 44 736 kbit/s and 97 728 kbit/s.

As a general rule, this Recommendation does not specify the atomic functions that are specific to PRC ([ITU-T G.811]) and SSU ([ITU-T G.812]) clock equipment; the SSM selection algorithm specified in this Recommendation is only generally applicable to SEC ([ITU-T G.813] and [ITU-T G.8262]) clock equipment. However, strictly for option II networks, there are some specific topologies listed containing both SEC and SSU clock equipment, for which the application of the SSM selection algorithm is described. The general case of using the SSM selection algorithm simultaneously in SSU and SEC clock equipment is for further study for all options.

Full integration of the ePRTC, PRTC, and eSEC is for further study.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T G.703]	Recommendation ITU-T G.703 (2016), Physical/electrical characteristics of hierarchical digital interfaces.
<http://www.itu.int/rec/T-REC-G.703>

[ITU-T G.704]	Recommendation ITU-T G.704 (1998), Synchronous frame structures used at 1544, 6312, 2048, 8448 and 44 736 kbit/s hierarchical levels.
<http://www.itu.int/rec/T-REC-G.704>

[ITU-T G.705]	Recommendation ITU-T G.705 (2000), Characteristics of plesiochronous digital hierarchy (PDH) equipment functional blocks.
<http://www.itu.int/rec/T-REC-G.705>

[ITU-T G.706]	Recommendation ITU-T G.706 (1991), Frame alignment and cyclic redundancy check (CRC) procedures relating to basic frame structures defined in Recommendation G.704.
<http://www.itu.int/rec/T-REC-G.706>

[ITU-T G.707]	Recommendation ITU-T G.707/Y.1322 (2007), Network node interface for the synchronous digital hierarchy (SDH).
<http://www.itu.int/rec/T-REC-G.707>

[ITU-T G.709]	Recommendation G.709/Y.1331 (2020), Interfaces for the optical transport network.
<http://www.itu.int/rec/T-REC-G.709>

[ITU-T G.709.1]	Recommendation G.709.1/Y.1331.1 (2018), Flexible OTN short-reach interface.
<http://www.itu.int/rec/T-REC-G.709.1>

[ITU-T G.775]	Recommendation ITU-T G.775 (1998), Loss of Signal (LOS), Alarm Indication Signal (AIS) and Remote Defect Indication (RDI) defect detection and clearance criteria for PDH signals.
<http://www.itu.int/rec/T-REC-G.775>

[ITU-T G.783]	Recommendation ITU-T G.783 (2006), Characteristics of synchronous digital hierarchy (SDH) equipment functional blocks.
<http://www.itu.int/rec/T-REC-G.783>

[ITU-T G.798]	Recommendation ITU-T G.798 (2017), Characteristics of optical transport network hierarchy equipment functional blocks.
<http://www.itu.int/rec/T-REC-G.798>

[ITU-T G.803]	Recommendation ITU-T G.803 (2000), Architecture of transport networks based on the synchronous digital hierarchy (SDH).
<http://www.itu.int/rec/T-REC-G.803>

[ITU-T G.806]	Recommendation ITU-T G.806 (2012), Characteristics of transport equipment – Description methodology and generic functionality.
<http://www.itu.int/rec/T-REC-G.806>

[ITU-T G.810]	Recommendation ITU-T G.810 (1996), Definitions and terminology for synchronization networks.
<http://www.itu.int/rec/T-REC-G.810>

[ITU-T G.811]	Recommendation ITU-T G.811 (1997), Timing characteristics of primary reference clocks.
<http://www.itu.int/rec/T-REC-G.811>

[ITU-T G.811.1]	Recommendation ITU-T G.811.1 (2017), Timing characteristics of enhanbced primary reference clocks.
<http://www.itu.int/rec/T-REC-G.811.1>

[ITU-T G.812]	Recommendation ITU-T G.812 (2004), Timing requirements of slave clocks suitable for use as node clocks in synchronization networks.
<http://www.itu.int/rec/T-REC-G.812>

[ITU-T G.813]	Recommendation ITU-T G.813 (2003), Timing characteristics of SDH equipment slave clocks (SEC).
<http://www.itu.int/rec/T-REC-G.813>

[ITU-T G.832]	Recommendation ITU-T G.832 (1998), Transport of SDH elements on PDH networks: Frame and multiplexing structures.
<http://www.itu.int/rec/T-REC-G.832>

[ITU-T G.7041]	Recommendation ITU-T G.7041/Y.1303 (2016), Generic framing procedure.
<http://www.itu.int/rec/T-REC-G.7041>

[ITU-T G.8021]	Recommendation ITU-T G.8021/Y.1341 (2018), Characteristics of Ethernet transport network equipment functional blocks.
<http://www.itu.int/rec/T-REC-G.8021>

[ITU-T G.8121]	Recommendation ITU-T G.8121/Y.1381 (2018), Characteristics of MPLS-TP equipment functional blocks.
<http://www.itu.int/rec/T-REC-G.8121>

[ITU-T G.8251]	Recommendation ITU-T G.8251 (2022), The control of jitter and wander within the optical transport network (OTN). 
<http://www.itu.int/rec/T-REC-G.8251>

[ITU-T G.8261]	Recommendation ITU-T G.8261/Y.1361 (2019), Timing and synchronization aspects in packet networks.
<http://www.itu.int/rec/T-REC-G.8261>

[ITU-T G.8262]	Recommendation ITU-T G.8262/Y.1362 (2018), Timing characteristics of synchronous equipment slave clock.
<http://www.itu.int/rec/T-REC-G.8262>

[ITU-T G.8262.1]	Recommendation ITU-T G.8262.1/Y.1362.1 (2022), Timing characteristics of synchronous equipment slave clock.
<http://www.itu.int/rec/T-REC-G.8262.1>

[ITU-T G.8264]	Recommendation ITU-T G.8264/Y.1364 (2017), Distribution of timing information through packet networks.
<http://www.itu.int/rec/T-REC-G.8264>

[ITU-T G.8272]	Recommendation ITU-T G.8272/Y.1367 (2018), Timing characteristics of primary reference time clocks.
<http://www.itu.int/rec/T-REC-G.8272>

[ITU-T G.8272.1] Recommendation ITU-T G.8272.1 (2016), Timing characteristics of enhanced primary reference time clocks.
<http://www.itu.int/rec/T-REC-G.8272.1>

[ITU-T Z.100]	Recommendation ITU-T Z.100 (2019), Specification and Description Language – Overview of SDL-2010.
<http://www.itu.int/rec/T-REC-Z.100>

[IEEE 802.3]	IEEE Standard 802.3-2022, IEEE Standard for Ethernet.
<http://ieeexplore.ieee.org/document/7428776/>

[OIF FLEXE IA] 	Optical Internetworking Forum, IA OIF-FLEXE-02.2 (2021), FlexE 2.2 Implementation Agreement.

# 3	Terms and definitions

This Recommendation defines the following terms:

3.1	timing loop: This is a network condition where a slave clock providing synchronization becomes locked to its own timing signal. It is generally created when the slave clock timing information is looped back to its own input, either directly or via other network equipments. Timing loops should be prevented in networks by careful network design.

3.2	QL_minimum: QL_minimum is a configurable parameter used in the squelching of clock output signals. If the quality level of the signal used to derive the output falls below QL_minimum then the output will be squelched (cut-off or set to AIS).

3.3	clock-source quality-level: The clock-source quality-level of an SEC or SASE is defined as the grade of clock to which it is ultimately traceable, i.e., the grade of clock to which it is synchronized directly or indirectly via a chain of SECs and SASEs, however long this chain of clocks is. For example, the clock-source quality-level may be a primary reference clock complying with [ITU-T G.811], or it may be a slave clock in holdover-mode, complying with [ITU-T G.812], or a [ITU-T G.813] or [ITU-T G.8262] clock in holdover or free-run.

The clock-source quality-level is essentially, therefore, an indication only of the long-term accuracy of the NE clock.

3.4	station clock: This is a node clock as defined in [ITU-T G.810].

The functional definitions are given in [ITU-T G.783].

The symbols and diagrammatic conventions are given in [ITU-T G.783].

3.5	squelch: An action that cuts off (i.e., shuts down) an output signal. For some signals (e.g., 2 Mbit/s), squelching may be realized by means of inserting AIS instead of shutting down the signal.

3.6	synchronous equipment clock: A generic term representing the SDH equipment clock ([ITU-T G.813]), the Ethernet equipment clock (EEC) ([ITU-T G.8262]), and the OTN equipment clock (OEC) ([ITU-T G.8262]).

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

AI		Adapted Information

AIS		Alarm Indication Signal

AP		Access Point

BITS		Building Integrated Timing Supply

CI		Characteristic Information

CK		Clock signal (timing information)

CLR		Clear

CP		Connection Point

CS		Clock Source (timing information)

CSid		Clock Source identifier

D		Data

DL		Data Link

DNU		Do Not Use

DUS		Do not Use for Sync

EEC		synchronous Ethernet Equipment Clock

ES1		Electrical Section, level 1

ESF		Extended SuperFrame

ESMC		Ethernet Synchronization Messaging Channel

eSSM		enhanced SSM

eQL-TLV	extended QL-TLV

EXTCMD	EXTernal COMmand messaging channel

FS		Frame Start (timing information)

FSw		Forced Switch

HO		Hold-Off time

HO		HoldOver mode

ID		IDentifier

INVx		INValid x

LC		Layer Clock

LO		Locked mode

LO		LockOut

LOS		Loss Of Signal

LSB		Least Significant Bit

LTI		Loss of Timing Information

MA		Maintenance and Adaptation

MFP		MultiFrame Present

MFS		MultiFrame Start

MI		Management Information

MON		Monitored

MS		Multiplex Section

MSB		Most Significant Bit

MSw		Manual Switch

MTIE		Maximum Time Interval Error

NE		Network Element

NS		Network Synchronization

NSUPP		Not SUPPorted

OSC		Optical Supervisory Channel

OSn		Optical Section layer (STM-n)

OTN		Optical Transport Network

OTU		Optical Transport Unit

P12s		2048 kbit/s PDH path layer with synchronous 125 s frame structure according to [ITU-T G.704]

P31s		34 368 kbit/s PDH path layer with synchronous 125 s frame structure according to [ITU-T G.832]

P4s		139 264 kbit/s PDH path layer with synchronous 125 s frame structure according to [ITU-T G.832]

PDH		Plesiochronous Digital Hierarchy

PRC		Primary Reference Clock

PRS		Primary Reference Source

QL		Quality Level

RES		REServed for network synchronization use

RI		Remote Information

RSn		Regenerator Section layer STM-n

SASE		Stand Alone Synchronization Equipment

SD		Synchronization Distribution

SDH		Synchronous Digital Hierarchy

SDL		Specification and Description Language

SEC		Synchronous Equipment Clock

SF		Signal Fail

Sk		Sink

SMC		SONET Minimum Clock

So		Source

SQLCH		Squelch

SSF		Server Signal Fail

SSM		Synchronization Status Message

SSU		Synchronization Supply Unit

SSU-A		primary level SSU

SSU-B		secondary level SSU

ST2		STratum 2

ST3		STratum 3

ST3E		STratum 3 Enhanced

ST4		STratum 4

STM-N		Synchronous Transport Module, level N

STU		Synchronization Traceability Unknown

TCP		Termination Connection Point

TDEV		Time DEViation

TI		Timing Information

TL		Transport Layer

TM		Timing Marker

TNC		Transit Node Clock

TSF		Trail Signal Fail

TT		Trail Termination

UNC		UNConnected

UNK		UNKnown

VC-n		Virtual Container, level n

WTR		Wait-To-Restore

# 5	Synchronization principles

## 5.1	Network synchronization

Synchronization network architecture is specified in [ITU-T G.803].

Synchronization information is transmitted through the network via synchronization network connections. These synchronization network connections can transport different synchronization levels. Each synchronization network connection is provided by one or more synchronization link connection(s), each supported by a synchronized PDH trail, SDH multiplex section trail, OTN OTU trail, OTN OSC trail, or IEEE 802.3 physical media trail.

Some of these synchronized PDH trail, SDH multiplex section trail, OTN OTU trail, OTN OSC trail, or IEEE 802.3 physical media trail signals contain a communication channel, the synchronization status message (SSM), the timing marker (TM), the OTN synchronization message channel (OSMC), or the ethernet synchronization message channel (ESMC) transporting a quality-level identifier. This quality-level identifier can be used to select the highest synchronization level incoming reference signal from a set of nominated synchronization references available at the network element.

Synchronization network connections are unidirectional and generally point-to-multipoint. [ITU-T G.803] specifies a master-slave synchronization technique for synchronizing SDH and OTN networks and [ITU-T G.8261] specifies it for packet transport networks. Figures 1 to 4 illustrate the synchronization network connection model.

Figure 1 – General representation of a synchronization network

Figure 2 – Representation of the PRC network connection

Figure 3 – Representation of the synchronization network
connection in case of failure

Figure 4 – Example of restoration of the synchronization

## 5.2	Synchronization distribution trails

Synchronization distribution trails transport timing between two adjacent equipments.

From a synchronization viewpoint, adjacent network elements are those network elements that are interconnected via section signals. Between two such adjacent network elements (NEs), a unidirectional synchronization distribution trail exists.

An SD trail starts at the input of the SD_TT_So function and ends at the output of the SD_TT_Sk function.

An SD link connection transports synchronization timing information between two adjacent connection points (CPs) of the NS_C function.

An NS network connection transports synchronization timing information over a series of synchronization link connection (see Figure 5).

Figure 5 – Example of series of synchronization distribution network
connection transporting PRC quality timing reference information

## 5.3	Synchronization interfaces

Synchronization trails can be carried through the network by a number of interfaces. Currently, the following signals are defined for such transport (refer also to Figures I.1 to I.4):

### 5.3.1	Overview

#### 5.3.1.1	Option I synchronization networking

–	Without traffic:

•	2048 kHz (T12);

•	2048 kbit/s (E12+P12s).

–	With traffic:

•	9 953 280 kbit/s (OS64+RS64+MS64);

•	2 488 320 kbit/s (OS16+RS16+MS16);

•	622 080 kbit/s (OS4+RS4+MS4);

•	155 520 kbit/s (OS1 (or ES1)+RS1+MS1);

•	139 264 kbit/s (E4+P4s);

•	34 368 kbit/s (E31+P31s);

•	2048 kbit/s (E12+P12s);

•	100 000 kbit/s (ETY2[+ETH]);

•	1 000 000 kbit/s (ETY3[+ETH]);

•	10 000 000 kbit/s (ETY4[+ETH]);

•	40 000 000 kbit/s (ETY5[+ETH]);

•	100 000 000 kbit/s (ETY6[+ETH]);

•	255/238  2 488 320 kbit/s (OPSn+OChr+OTU1+ODU1);

•	255/237  9 953 280 kbit/s (OPSn+OChr+OTU2+ODU2);

•	255/236  39 813 120 kbit/s (OPSn+OChr+OTU3+ODU3);

•	255/227  99 532 800 kbit/s (OPSM44+OTU4+ODU4);

•	X kbit/s (OSC).

NOTE – The ETYn synchronization interfaces are full duplex ones with continuous bit stream, as specified in [ITU-T G.8262].

#### 5.3.1.2	Option II synchronization networking

–	Without traffic:

•	64 kHz (T01);

•	1544 kbit/s (E11+P11s).

–	With traffic:

•	9 953 280 kbit/s (OS64+RS64+MS64);

•	2 488 320 kbit/s (OS16+RS16+MS16);

•	622 080 kbit/s (OS4+RS4+MS4);

•	155 520 kbit/s (OS1 (or ES1)+RS1+MS1);

•	1544 kbit/s (E11+P11s);

•	100 000 kbit/s (ETY2[+ETH]);

•	1 000 000 kbit/s (ETY3[+ETH]);

•	10 000 000 kbit/s (ETY4[+ETH]);

•	40 000 000 kbit/s (ETY5[+ETH]);

•	100 000 000 kbit/s (ETY6[+ETH]);

•	255/238  2 488 320 kbit/s (OPSn+OChr+OTU1+ODU1);

•	255/237  9 953 280 kbit/s (OPSn+OChr+OTU2+ODU2);

•	255/236  39 813 120 kbit/s (OPSn+OChr+OTU3+ODU3);

•	255/227  99 532 800 kbit/s (OPSM44+OTU4+ODU4);

•	X kbit/s (OSC).

NOTE – The ETYn synchronization interfaces are full duplex ones with continuous bit stream, as specified in [ITU-T G.8262].

#### 5.3.1.3	Option III synchronization networking

–	Without traffic:

•	64 kHz (T02) [from SASE/BITS to NE];

•	6312 kHz (T21) [from NE to SASE/BITS].

–	With traffic:

•	9 953 280 kbit/s (OS64+RS64+MS64);

•	2 488 320 kbit/s (OS16+RS16+MS16);

•	622 080 kbit/s (OS4+RS4+MS4);

•	155 520 kbit/s (OS1 (or ES1)+RS1+MS1);

•	100 000 kbit/s (ETY2[+ETH]);

•	1 000 000 kbit/s (ETY3[+ETH]);

•	10 000 000 kbit/s (ETY4[+ETH]);

•	40 000 000 kbit/s (ETY5[+ETH]);

•	100 000 000 kbit/s (ETY6[+ETH]);

•	255/238  2 488 320 kbit/s (OPSn+OChr+OTU1+ODU1);

•	255/237  9 953 280 kbit/s (OPSn+OChr+OTU2+ODU2);

•	255/236  39 813 120 kbit/s (OPSn+OChr+OTU3+ODU3);

•	255/227  99 532 800 kbit/s (OPSM44+OTU4+ODU4);

•	X kbit/s (OSC).

NOTE – The ETYn synchronization interfaces are full duplex ones with continuous bit stream, as specified in [ITU-T G.8262].

### 5.3.2	STM-N

The STM-N transport signals carry (in addition to the payload) reference timing information and an indication of the quality level of the source generating this timing information, via the synchronization status message (SSM) as defined in [ITU-T G.707].

NOTE – Old equipment may not be able to support SSM via their STM-N interfaces.

### 5.3.3	2 Mbit/s

The 2 Mbit/s transport signals may carry (in addition to the payload) reference timing information.

The 2 Mbit/s timing reference signals (without payload) carry reference timing information to specific synchronization ports.

Both signals can carry an indication of the quality level of the source generating the timing information via the SSM as specified in [ITU-T G.704].

NOTE 1 – Old equipment may not be able to support SSM on their 2 Mbit/s interfaces.

NOTE 2 – The 2 Mbit/s timing reference signal can be connected to either a SASE/BITS or another NE. The SSM specifications in this version of this Recommendation include the interconnect between two NEs. The SSM processing with respect to the interface between NE and SASE is for further study.

### 5.3.4	2 MHz

Synchronization can be carried through 2 MHz signals to specific synchronization ports (so-called station clock ports). This signal does not carry an indication of the quality level of the source generating the timing information.

NOTE – The 2 MHz timing reference signal can be connected to either a SASE/BITS or another NE.

### 5.3.5	34 Mbit/s and 140 Mbit/s with 125 s frame structure

34 Mbit/s and 140 Mbit/s signals with 125 s frame structure as defined in [ITU-T G.832] carry a full 4-bit SSM code.

NOTE – For interworking with equipments compliant with the initial edition of [ITU-T G.832], new equipments should be able to be configured to recognize and generate the timing marker which is located in bit 8 of the MA (maintenance and adaptation) byte: the timing marker is set to "0" to indicate that the timing source is traceable to a primary reference clock (PRC), and is otherwise set to "1".

### 5.3.6	1.5 Mbit/s

The 1.5 Mbit/s transport signals may carry (in addition to the payload) reference timing information.

The 1.5 Mbit/s timing reference signals (without payload) carry reference timing information to specific synchronization ports.

Both signals can carry an indication of the quality level of the source generating the timing information via the SSM transported within the 1544 kbit/s signal's extended super frame (ESF) data link (DL) as specified in [ITU-T G.704].

NOTE 1 – The format of the data link messages in ESF frame format is "0xxx xxx0 1111 1111", transmitted right-most bit first. The 6 bits denoted "xxx xxx" contain the actual message; some of these messages are reserved for synchronization messaging. It takes 32 frames (i.e., 4 ms) to transmit all 16 bits of a complete DL word. Note that [ITU-T G.704] presents the data link messages in reverse order "1111 1111 0xxx xxx0".

NOTE 2 – Old equipment may not be able to support SSM on their 1.5 Mbit/s interfaces.

### 5.3.7	64 kHz

Within option II networks, synchronization can be carried through 64 kHz (T01) interface signals (see clause 4.2.2 of [ITU-T G.703] composite timing signal) to specific synchronization input ports (so-called station clock ports). This signal does not carry an indication of the quality level of the source generating the timing information.

Within option III networks, synchronization can be carried through 64 kHz (T02) interface signals (see Appendix II of [ITU-T G.703] composite timing signal) from the SASE to specific synchronization input ports (so-called station clock ports) on an NE. This signal does not carry an indication of the quality level of the source generating the timing information.

### 5.3.8	6312 kHz

Within option III networks, synchronization can be carried through 6312 kHz signals (see Appendix II of [ITU-T G.703]) from specific synchronization output ports (so-called station clock ports) on an NE to inputs of the SASE. This signal does not carry an indication of the quality level of the source generating the timing information.

### 5.3.9	100 Mbit/s, 1 Gbit/s, 10 Gbit/s, 40 Gbit/s, 100 Gbit/s

The IEEE 802.3 transport signals may carry (in addition to the payload) reference timing information, and an indication of the quality level of the source generating this timing information, via the synchronization status message (SSM) within the QL-TLV and the enhanced SSM (eSSM) within the eQL-TLV, as defined in [ITU-T G.8264].

NOTE – It is possible that equipment is equipped with both non-synchronous and synchronous IEEE 802.3 interfaces as defined in [ITU-T G.8264]. Non-synchronous IEEE 802.3 interfaces are excluded from the synchronization distribution process.

### 5.3.10	OTN interfaces without OSC

OTN interface signals without an OSC may carry (in addition to the payload) reference timing information in the OTUk (k = 1, 2, 3, 4) and FlexO, and an indication of the quality level of the source generating this timing information, via the synchronization status message (SSM) and, optionally, via the eSSM within the eQL-TLV, as defined in [ITU-T G.7041], [ITU-T G.709], and [ITU-T G.8264].

### 5.3.11	OTN interfaces with OSC

An OTN interface signal with an OSC may carry (in addition to the payload):

a)	reference timing information in the optical supervisory channel (OSC) signal, and

b)	an indication of the quality level of the source generating this timing information, via the synchronization status message (SSM) and, optionally, the eSSM within the eQL-TLV, as defined in [ITU-T G.7041], [ITU-T G.709], and [ITU-T G.8264].

NOTE 1 – In cases where the OCh goes through a ROADM network and the connectivity for the OCh can be changed via protection or restoration processes, the OTUk within the OCh cannot be used to carry time synchronization because, due to optical switching or cross-connecting, its latency cannot be controlled (and could produce delay asymmetry, which could result in time synchronization error). However, the OSC is terminated at every optical node (including OAs and ROADMs), so this is not an issue for the OSC. Note also that, in principle, delay asymmetry is not an issue for frequency synchronization; however, it is unlikely that one would want a link that is suitable for carrying frequency synchronization but not time synchronization.

NOTE 2 – Equipment developed prior to Edition 3.0 of this Recommendation may not be able to carry reference timing information and/or support SSM or eQL-TLV via an OTUk signal in its OTN interfaces.

NOTE 3 – It is possible that equipment is equipped with both non-synchronous and synchronous OTN intra-domain interfaces. Non-synchronous OTN intra-domain interfaces are excluded from the synchronization distribution process.

## 5.4	Clock-source quality-level

### 5.4.1	Clock-source quality-level definitions

SDH and packet transport networks throughout the world are based on different synchronization philosophies. Those differences are identified in this Recommendation as three options: I, II and III.

#### 5.4.1.1	Option I synchronization networking

The following clock-source quality-levels are defined in the synchronization process of option I networks.

QL-ePRTC:		This synchronization trail transports a timing quality generated by an enhanced primary reference time clock, which is defined in [ITU-T G.8272.1].

QL-PRTC:		This synchronization trail transports a timing quality generated by a primary reference time clock, which is defined in [ITU-T G.8272].

QL-ePRC:		This synchronization trail transports a timing quality generated by an enhanced primary reference clock, which is defined in [ITU-T G.811.1].

QL-PRC:		This synchronization trail transports a timing quality generated by a primary reference clock, which is defined in [ITU-T G.811].

QL-SSU-A:		This synchronization trail transports a timing quality generated by a type I or V slave clock, which is defined in [ITU-T G.812].

QL-SSU-B:		This synchronization trail transports a timing quality generated by a type VI slave clock, which is defined in [ITU-T G.812].

QL-eSEC:		This synchronization trail transports a timing quality generated by an enhanced SEC, which is defined in [ITU-T G.8262.1].

QL-SEC:		This synchronization trail transports a timing quality generated by a synchronous equipment clock (SEC) that is defined in [ITU-T G.813] or [ITU-T G.8262], option I.

NOTE – For option I, QL-SEC represents the same quality level as QL-EEC1 in [ITU-T G.8264].

QL-DNU:		This signal should not be used for synchronization.

NOTE – The "unknown" quality level was defined to characterize the quality of an existing network. This QL is no longer supported by the SSM algorithm in option I networks. Instead, a synchronization reference input port which receives a signal without SSM can be provisioned to assume a quality level for that received signal (see clause 5.4.3).

#### 5.4.1.2	Option II synchronization networking

The following clock-source quality-levels are defined in the synchronization selection process of an option II network.

QL-ePRTC:		This synchronization trail transports a timing quality generated by an enhanced primary reference time clock, which is defined in [ITU-T G.8272.1].

QL-PRTC:		This synchronization trail transports a timing quality generated by a primary reference time clock, which is defined in [ITU-T G.8272].

QL-ePRC:		This synchronization trail transports a timing quality generated by an enhanced primary reference clock, which is defined in [ITU-T G.811.1].

QL-PRS:		PRS traceable ([ITU-T G.811]).

QL-STU:		Synchronized – Traceability unknown.

QL-ST2:		Traceable to stratum 2 ([ITU-T G.812], type II).

QL-TNC:		Traceable to transit node clock ([ITU-T G.812], type V).

QL-ST3E:		Traceable to stratum 3E ([ITU-T G.812], type III).

QL-ST3:		Traceable to stratum 3 ([ITU-T G.812], type IV or [ITU-T G.8262], option II).

NOTE – For option II, QL-ST3 represents the same quality level as QL-EEC2 in [ITU-T G.8264].

QL-SMC:		Traceable to SONET clock self timed ([ITU-T G.813]).

QL-ST4:		Traceable to stratum 4 free-run (only applicable to 1.5 Mbit/s signals).

QL-PROV:		Provisionable by the network operator.

QL-DUS:		This signal should not be used for synchronization.

First-generation quality levels do not define QL-ST3E and QL-TNC as separate quality levels and QL-PROV was identified as QL-RES.

#### 5.4.1.3	Option III synchronization networking

The following clock-source quality-levels are defined in the synchronization process of an option III SDH network.

QL-ePRTC:	 This synchronization trail transports a timing quality generated by an enhanced primary reference time clock, which is defined in [ITU-T G.8272.1].

QL-PRTC:   This synchronization trail transports a timing quality generated by a primary reference time clock, which is defined in [ITU-T G.8272].

QL-UNK:	This synchronization trail transports a timing quality generated by an unknown clock source. It is at least of quality SSU.

QL-eSEC:	This synchronization trail transports a timing quality generated by an enhanced SEC, which is defined in [ITU-T G.8262.1].

QL-SEC:	This synchronization trail transports a timing quality generated by a synchronous equipment clock (SEC) that is defined in [ITU-T G.813] or [ITU-T G.8262], option I.

NOTE – The use of other quality levels is for further study.

#### 5.4.1.4	Squelching

The first purpose of the "squelch" function is to prevent transmission of a timing signal with a quality that is lower than the quality of the clock in the receiving network element or SASE. It is also used for the prevention of timing loops (see clause 5.13).

### 5.4.2	Hierarchy of clock-source quality-levels (QL) or (CS_QL)

The following tables define the QL hierarchy.

#### 5.4.2.1	Option I synchronization networking

Table 1 – Hierarchy of quality levels in option I synchronization networks

| Quality level | Order |
| --- | --- |
| QL-ePRTC | Highest |
| QL-PRTC | ¦ |
| QL-ePRC | ¦ |
| QL-PRC | ¦ |
| QL-SSU-A | ¦ |
| QL-SSU-B | ¦ |
| QL-eSEC | ¦ |
| QL-SEC | ¦ |
| QL-DNU | ¦ |
| QL-INVx, QL-INV, QL-FAILED, QL-UNC, QL-NSUPP | Lowest |

The quality levels QL-INVx, QL-FAILED, QL-UNC and QL-NSUPP are internal QLs inside the NE and are never generated at an output port.

QL-INVx is generated by the XX/SD_A_Sk function if an unallocated SSM value is received, where x represents the binary value of this SSM.

QL-INV is generated by the XX/SD_A_Sk function if an allocated SSM value and an unallocated eSSM value are received.

QL-NSUPP is generated by the XX/SD_A_Sk function when the function is not supporting the SSM (TM) processing.

QL-FAILED is generated by the SD_TT_Sk function when the terminated SD trail is in the signal fail state.

QL-UNC is generated by the SD_C or NS_C function when the output signal is not connected to an input, but instead to the internal unconnected signal generator.

#### 5.4.2.2	Option II synchronization networking

| Table 2 – Hierarchy of quality levels in option II synchronization networks |  |  |
| --- | --- | --- |
| Quality level | Order |
| QL-ePRTC | Highest |
| QL-PRTC | ¦ |
| QL-PRS | ¦ |
| QL-STU | ¦ |
| QL-ST2 | ¦ |
| QL-TNC (Note) | ¦ |
| QL-ST3E (Note) | ¦ |
| QL-ST3 | ¦ |
| QL-SMC | ¦ |
| QL-ST4 | ¦ |
| QL-PROV (default position) | ¦ |
| QL-DUS | ¦ |
| QL-INVx, QL-INV, QL-FAILED, QL-UNC, QL-NSUPP | Lowest |
| NOTE 1 – QL-TNC and QL-ST3E are not defined for first-generation synchronization networking (refer to clause 5.4.1.2) and QL-PROV was identified as QL-RES. NOTE 2 – The addition of QL-eSEC, for the enhanced SEC, is for further study. |  |

The quality levels QL-INVx, QL-FAILED, QL-UNC and QL-NSUPP are internal QLs inside the NE and are never generated at an output port.

QL-INVx is generated by the XX/SD_A_Sk function if an unallocated SSM value is received, where x represents the binary value of this SSM.

QL-INV is generated by the XX/SD_A_Sk function if an allocated SSM value and an unallocated eSSM value are received.

QL-NSUPP is generated by the XX/SD_A_Sk function when the function is not supporting the SSM (TM) processing.

QL-FAILED is generated by the SD_TT_Sk function when the terminated SD trail is in the signal fail state.

QL-UNC is generated by the SD_C or NS_C function when the output signal is not connected to an input, but instead to the internal unconnected signal generator.

The quality level QL-PROV is provisionable by the network operator and may take different order positions. The default position for QL-PROV is as shown in Table 2.

#### 5.4.2.3	Option III synchronization networking

Table 3 – Hierarchy of quality levels in option III synchronization networks

| Quality level | Order |
| --- | --- |
| QL-ePRTC | Highest |
| QL-PRTC | ¦ |
| QL-UNK | ¦ |
| QL-eSEC | ¦ |
| QL-SEC | ¦ |
| QL-INVx, QL-FAILED, QL-UNC, QL-NSUPP | Lowest |

The quality levels QL-INVx, QL-FAILED, QL-UNC and QL-NSUPP are internal QLs inside the NE and are never generated at an output port.

QL-INVx is generated by the XX/SD_A_Sk function if an unallocated SSM value is received, where x represents the binary value of this SSM.

QL-NSUPP is generated by the XX/SD_A_Sk function when the function is not supporting the SSM (TM) processing.

QL-FAILED is generated by the SD_TT_Sk function when the terminated SD trail is in the signal fail state.

QL-UNC is generated by the SD_C or NS_C function when the output signal is not connected to an input, but instead to the internal unconnected signal generator.

### 5.4.3	Forcing and defaulting of clock-source quality-levels

For synchronization source signals/interfaces not supporting SSM transport/processing, it is possible in option I networks to force the quality level to a fixed provisioned value. This allows using these signals/interfaces as synchronization sources in an automatic reference selection process operating in QL-enabled mode.

Forcing of quality levels is used for new equipment operating in QL-enabled mode in order to:

–	interwork with old equipment not supporting SSM/TM generation;

–	interwork with new equipment operating in QL-disabled mode;

–	select interfaces not supporting SSM/TM processing;

–	select signals for which SSM/TM is not defined (e.g., 2 MHz).

In option II and III networks, synchronization input ports assume a default QL-STU/QL-UNK for synchronization source signals not supporting SSM.

#### 5.4.3.1	Option I synchronization networking

The quality level of the input signal (STM-N, 2 Mbit/s, 2 MHz, 34 Mbit/s, 140 Mbit/s) can be forced to either QL-PRC, QL-SSU-A, QL-SSU-B or QL-SEC.

#### 5.4.3.2	Option II synchronization networking

QL-STU is the default quality level for signals from equipment not supporting or not enabled for SSM in option II.

NOTE – 1544 kbit/s signals transport the SSM as a specific message within the data link (see [ITU-T G.704]). When a timing input port does not receive any SSM message, it defaults to QL-STU.

#### 5.4.3.3	Option III synchronization networking

QL-UNK is the default quality level for 64 kHz station clock input signals.

### 5.4.4	Application of quality level "Unknown"

#### 5.4.4.1	Option I synchronization networking

Option I synchronization networks do not support the unknown quality level. Instead, the network operator is required to force the quality level to one of the four quality levels: PRC, SSU-A, SSU-B or SEC. Engineering rules for this selection are for further study.

NOTE 1 – Equipment built according to the initial STM-N signal specification in which the S1 byte was still a Z1 byte with no defined value, may output any of the 16 codes within what is now bits 5 to 8 of byte S1.

NOTE 2 – Engineering rules for the forcing of quality levels in option I networks could be the following: if the reference signal is sourced by an SASE or NE with SSU-A clock, the forced QL should be QL-SSU-A; if the reference signal is sourced by an NE with SSU-B clock, the forced QL should be QL-SSU-B; if the reference signal is sourced by an NE with a SEC clock, the forced QL should be QL-SEC.

#### 5.4.4.2	Option II synchronization networking

Quality level "unknown" (QL-STU) is intended to be used in option II applications where SSM functionality is not supported by all synchronization equipment. Without SSM capabilities, it is impossible for synchronization equipment to communicate its quality level. Consequently, the QL-STU message is used in applications where equipment with SSM functionality is timed from a reference without an SSM. The QL-STU message indicates that the equipment is locked to a reference (i.e., it is not in holdover) but the quality of the source is not known.

Typically, this QL-STU quality level is assumed to apply to signals at an NE's station clock input port (connected to a BITS). 1.5 Mbit/s signals into the clock input ports default to STU if SSMs are not supported by the BITS. STM-N signals into the NE contain SSM code "0000" and are interpreted as QL-STU if SSMs are not supported by the STM-N output port in the previous NE.

When the input signal with forced QL-STU is selected as the synchronization reference, the output signals supporting SSM will indicate QL-STU in their SSM bits. Network elements supporting SSM processing will accept incoming QL-STU indications as one of the normal quality-level indications.

The clock-source quality-level of a signal with QL-STU indication is assumed to be equal or less than PRS and better than or equal to ST2 (clause 5.4.2).

#### 5.4.4.3	Option III synchronization networking

In the case of option III applications, an NE's clock is usually locked to an ePRTC, PRTC, PRC, or SSU clock. Consequently, the quality level of the NE's clock signal is usually better than or equal to SSU. However, if there is an eSEC or SEC, which can enter holdover or free-run mode while part of a synchronization network connection, it is necessary to ensure the clock quality.

The quality level "unknown" (QL-UNK) message indicates that the equipment is locked to a reference (i.e., it is not in holdover) but the quality of the source is not known. Typically, this QL-UNK quality level is assumed to apply to 64 kHz signals at an NE's station clock input port (connected to an SASE). The quality of the source will then be at least SSU.

When the input signal with QL-UNK is selected as synchronization reference, the output signals supporting SSM will indicate QL-UNK in their SSM bits. Network elements supporting SSM processing will accept incoming QL-UNK indications as one of the normal quality-level indications. The clock-source quality-level of a signal with QL-UNK indication is assumed to be equal or less than PRC and better than or equal to SSU.

If the clock-source quality-level is lower than QL-UNK at the clock input port of the NE in the upper synchronization layer, quality level "synchronous equipment clock" (QL-SEC) or quality level "enhanced Ethernet equipment clock" (QL-eSEC) shall be transmitted from an NE in the upper synchronization layer to an NE in the lower synchronization layer. When the NE in the lower synchronization layer receives the QL-SEC or QL-eSEC, the NE in the lower synchronization layer squelches the output to the SSU to avoid the clock quality level degradation in other lower synchronization layers. The clock-source quality-level of a signal with QL-SEC or QL-eSEC indication means that its quality level is lower than SSU.

NOTE – Engineering rules for the forcing of quality levels in option III networks are for further study.

### 5.4.5	Application of quality level "provisioned"

The quality level QL-PROV defined for option II networks is provisionable to a QL order chosen by the operator.

## 5.5	Synchronization status messages (SSMs), extended QL TLV, and timing marker (TM) channels

The following signals have a four-bit SSM channel defined:

–	STM-N (N = 1, 4, 16): bits 5 to 8 of the byte S1 (called SSMB, synchronization status message byte) of the multiplex section overhead as defined in [ITU-T G.707].

–	2 Mbit/s octet structured according to [ITU-T G.704]: bits Sax1 to Sax4 (x = 4, 5, 6, 7 or 8) of TS0.

–	1.5 Mbit/s octet structured according to clause 2.1 of [ITU-T G.704].

–	34 Mbit/s as defined in [ITU-T G.832]: bit 8 of MA byte with a 4-frame multiframe.

–	140 Mbit/s as defined in [ITU-T G.832]: bit 8 of MA byte with a 4-frame multiframe.

–	100 Mbit/s, 1 Gbit/s, 10 Gbit/s, 40 Gbit/s, and 100 Gbit/s, as defined in [IEEE 802.3]: 4-bit SSM field in the SSM PDU (i.e., the ESMC) as defined in [ITU-T G.8264].

The following signals may have a one bit TM channel:

–	34 Mbit/s with a 125 s frame structure as defined in [ITU-T G.832]: bit 8 of byte MA.

–	140 Mbit/s with a 125 s frame structure as defined in [ITU-T G.832]: bit 8 of byte MA.

The following signals have a packet-based SSM channel:

–	100 Mbit/s, 1 Gbit/s, 10 Gbit/s, 40 Gbit/s, and 100 Gbit/s Ethernet as defined in [ITU-T G.8262]: Ethernet Synchronization Message Channel (ESMC) [ITU-T G.8264]

–	OTN OTUk, k=1,2,3,4 defined in [ITU-T G.709]: OTN Synchronization Message Channel (OSMC) [ITU-T G.709]

–	OTN OSC as defined in [ITU-T G.709]: OTN Synchronization Message Channel (OSMC) [ITU-T G.709]

–	FlexO as defined in [ITU-T G.709.1]: OTN Synchronization Message Channel (OSMC) [ITU-T G.709.1]

The following signals have an optional eQL-TLV, as defined in [ITU-T G.8264]:

–	100 Mbit/s, 1 Gbit/s, 10 Gbit/s, 40 Gbit/s, and 100 Gbit/s Ethernet as defined in [ITU-T G.8262] and [ITU-T G.8262.1]

–	OTN OTUk, k=1,2,3,4 defined in [ITU-T G.709]

–	OTN OSC as defined in [ITU-T G.709]

–	FlexO as defined in [ITU-T G.709.1]

### 5.5.1	SSM, eSSM, and TM message sets

NOTE – The extended QL-TLV, and eSSM codes, are specified in [ITU-T G.8264].

#### 5.5.1.1	Option I synchronization networking

Five SSM codes are defined to represent clock-source QL as listed below:

–	code 0010 (quality PRC) means that the source of the trail is a PRC clock ([ITU-T G.811]);

–	code 0100 (quality SSU-A), means that the source of the trail is a type I or V SSU clock [ITU-T G.812];

–	code 1000 (quality SSU-B), means that the source of the trail is a type VI SSU clock ([ITU-T G.812]);

–	code 1011 (quality SEC), means that the source of the trail is an SEC clock ([ITU-T G.813] or [ITU-T G.8262], option I);

–	code 1111 (quality DNU), means that the signal carrying this SSM shall not be used for synchronization because a timing loop situation could result if it is used.

Two TM codes are defined in [ITU-T G.832]:

–	code 0 (quality PRC), means that the source of the trail is a PRC clock ([ITU-T G.811]);

–	code 1 (quality less_than_PRC), means that the source of the trail is not a PRC clock.

#### 5.5.1.2	Option II synchronization networking

Nine SSM codes are defined to represent clock-source QL as listed below on STM-N, Ethernet, and OTN signals:

–	code 0001 (quality PRS) means that the source of the trail is a PRS clock ([ITU-T G.811]);

–	code 0000 (quality STU) means that the signal does not carry the QL message of the source of the trail;

–	code 0111 (quality ST2), means that the source of the trail is a stratum 2 clock ([ITU-T G.812], type II);

–	code 0100 (quality TNC), means that the source of the trail is a transit node clock ([ITU-T G.812], type V);

–	code 1101 (quality ST3E), means that the source of the trail is a stratum 3E clock ([ITU-T G.812], type III);

–	code 1010 (quality ST3), means that the source of the trail is a stratum 3 clock ([ITU-T G.812], type IV or [ITU-T G.8262], option II);

–	code 1100 (quality SMC), means that the source of the trail is a SONET self-timed clock ([ITU-T G.813]);

–	code 1110 (quality PROV), is provisionable by the network operator;

–	code 1111 (quality DUS), means that the signal carrying this SSM shall not be used for synchronization because a timing loop situation could result if it is used.

NOTE 1 – Code 1101 and code 0100 are not recognized by equipment supporting only first-generation SSM and are treated as invalid (see clause 5.4.1.2), code 1110 is identified as QL-RES.

Ten SSM codes are defined to represent clock-source QL as listed below on 1544 kbit/s signals:

–	code 04FFH (quality PRS) means that the source of the trail is a PRS clock ([ITU-T G.811]);

–	code 08FFH (quality unknown) means that the source of the trail is unknown;

–	code 0CFFH (quality ST2) means that the source of the trail is a stratum 2 clock ([ITU-T G.812], type II);

–	code 78FFH (quality TNC) means that the source of the trail is a transit node clock ([ITU-T G.812], type V);

–	code 7CFFH (quality ST3E) means that the source of the trail is a stratum 3E clock ([ITU-T G.812, type III);

–	code 10FFH (quality ST3) means that the source of the trail is a stratum 3 clock ([ITU-T G.812, type IV);

–	code 22FFH (quality SMC) means that the source of the trail is a SONET/Ethernet self-timed clock ([ITU-T G.813] or [ITU-T G.8262], option II);

–	code 28FFH (quality ST4) means that the source of the trail is a stratum 4 clock;

–	code 40FFH (quality PROV) is provisionable by the network operator;

–	code 30FFH (quality DUS) means that the signal carrying this SSM shall not be used for synchronization because a timing loop situation could result if it is used.

NOTE 2 – Code 78FFH and code 7CFFH are not recognized by equipment supporting only second-generation SSM and are treated as invalid (refer to clause 5.4.1.2), code 40FFH is identified as QL-RES.

#### 5.5.1.3	Option III synchronization networking

Two SSM codes are defined to represent clock-source QL as listed below:

–	code 0000 (quality unknown) means that the source of the trail is unknown;

–	code 1011 (quality SEC) means that the source of the trail is an SEC clock ([ITU-T G.813] or [ITU-T G.8262], option I).

### 5.5.2	SSM and TM code word generation

The SSM can be viewed as an application-specific data communication channel with a limited message set. The message that shall be generated and inserted depends on the applied quality-level indication that is input to the adaptation source function. The following tables present the relation between the existing set of QLs and SSM codes for the three options.

It is possible to disable the SSM generation and insert "1111". An application for this could be, e.g., at network boundaries where timing information should not be forwarded to the other network.

#### 5.5.2.1	Option I synchronization networking

Table 4 – Quality level set and coding in synchronization status message
in option I synchronization networks

| Quality level (QL) | SSM usage | SSM coding [MSB..LSB] |
| --- | --- | --- |
| QL-PRC | Enabled | 0010 |
| QL-SSU-A | Enabled | 0100 |
| QL-SSU-B | Enabled | 1000 |
| QL-SEC | Enabled | 1011 |
| QL-DNU | Enabled | 1111 |
| – | Disabled | 1111 |

The TM can be viewed as an application-specific data communication channel with a limited message set. The message that shall be generated and inserted depends on the applied quality level indication that is input to the adaptation source function. The following table presents the relation between the existing set of QLs and TM codes.

Table 5 – Quality level set and coding in timing marker
in option I synchronization networks

| Quality Level (QL) | TM usage | TM coding |
| --- | --- | --- |
| QL-PRC | Enabled | 0 |
| QL-SSU-A | Enabled | 1 |
| QL-SSU-B | Enabled | 1 |
| QL-SEC | Enabled | 1 |
| QL-DNU | Enabled | 1 |
| – | Disabled | 1 |

At network boundaries, it should be possible to prevent synchronization information passing the interface. This can be achieved by disabling the SSM (TM) usage.

#### 5.5.2.2	Option II synchronization networking

In order to provide stable interoperability between equipment supporting first generation and equipment supporting second-generation quality levels, Table 6 defines the SSM coding for both "second-generation output connected to a second-generation input", and for "second-generation output connected to first-generation input".

This translation table is for newer equipment supporting second-generation quality-level definitions only. Message translation is provided on a per port basis as a provisionable option. This allows for equipment supporting second-generation messaging to simultaneously pass valid SSMs to both second-generation and first-generation equipment.

The shaded fields in Table 6 identify the quality levels that require translation in order to be understood by first-generation equipment: QL-TNC and QL-ST3E are translated into QL-ST3 messages, QL-PROV SSM code is interpreted as QL-RES by first-generation equipment.

| Table 6 – Quality level set and coding in synchronization status message in option II synchronization networks |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Quality level (QL) | SSM usage | Second-generation SSM |  | First-generation SSM |  |
|  |  | SSM coding [MSB..LSB] in STM-N, Ethernet, and OTN signal (BINARY) | SSM coding [MSB..LSB] in 1544 kbit/s signal with ESF (HEX) | SSM coding [MSB..LSB] in STM-N signal (BINARY) | SSM coding [MSB..LSB] in 1544 kbit/s signal with ESF (HEX) |
| QL-PRS | Enabled | 0001 | 04FF | 0001 | 04FF |
| QL-STU | Enabled | 0000 | 08FF | 0000 | 08FF |
| QL-ST2 | Enabled | 0111 | 0CFF | 0111 | 0CFF |
| QL-TNC | Enabled | 0100 | 78FF | 1010c) | 10FFc) |
| QL-ST3E | Enabled | 1101 | 7CFF | 1010c) | 10FFc) |
| QL-ST3 | Enabled | 1010 | 10FF | 1010 | 10FF |
| QL-SMC | Enabled | 1100 | 22FF | 1100 | 22FF |
| QL-ST4 | Enabled | – | 28FF | – | 28FF |
| QL-PROV | Enabled | 1110 | 40FF | 1110a), b), c) | 40FFa), b), c) |
| QL-DUS | Enabled | 1111 | 30FF | 1111 | 30FF |
| – | Disabled | 1111 | 08FF | 1111 | 08FF |
| a) The assignment of the GEN1 message "reserved for network synchronization use (QL-RES)" needs to be done on a network-wide basis. b) In a given network, the GEN1 message "reserved for network synchronization use (QL-RES)" can only reflect a single GEN2 message assignment. c) The translation of the TNC or stratum 3E message to the reserved for network synchronization use (RES) message, is a user-definable option. In this case, the quality level of the RES message is between 3 and 4. In this way, GEN1 equipment that supports a TNC or stratum 3E quality clock can always receive traceable timing of an equal or better quality-level which would then maintain the hierarchical distribution of timing. Because of the nature of the RES message, all NEs in the network need to be provisioned to recognize the RES message as TNC or stratum 3E. The RES message may only reflect one unique message on a network-wide basis. |  |  |  |  |  |

At network boundaries, it shall be possible to prevent synchronization information passing the interface. This can be achieved by disabling the SSM usage, or by provisioning output signals to have QL-STU or QL-DUS.

The application of the QL-PROV message is at the discretion of the network operator.

#### 5.5.2.3	Option III synchronization networking

Table 7 – Quality level set and coding in synchronization status message
in option III synchronization networks

| Quality level (QL) | SSM coding [MSB..LSB] |
| --- | --- |
| QL-UNK | 0000 |
| QL-SEC | 1011 |

### 5.5.3	SSM and TM code word interpretation

At the receive side, the received SSM bits are to be validated by a persistency check and then interpreted to determine the QL.

#### 5.5.3.1	Option I synchronization networking

Table 8 – Interpretation of synchronization status message codes
in option I synchronization networks

| SSM code [MSB..LSB] | QL interpretation |
| --- | --- |
| 0000 | QL-INV0 |
| 0001 | QL-INV1 |
| 0010 | QL-PRC |
| 0011 | QL-INV3 |
| 0100 | QL-SSU-A |
| 0101 | QL-INV5 |
| 0110 | QL-INV6 |
| 0111 | QL-INV7 |
| 1000 | QL-SSU-B |
| 1001 | QL-INV9 |
| 1010 | QL-INV10 |
| 1011 | QL-SEC |
| 1100 | QL-INV12 |
| 1101 | QL-INV13 |
| 1110 | QL-INV14 |
| 1111 | QL-DNU |

Table 9 – Interpretation of timing marker codes

| TM code | QL interpretation |
| --- | --- |
| 0 | QL-PRC |
| 1 | QL-DNU |

#### 5.5.3.2	Option II synchronization networking

| Table 10 – Interpretation of synchronization status message codes in signals that are not 1544 kbit/s in option II synchronization networks |  |
| --- | --- |
| SSM code [MSB..LSB] | QL interpretation |
| 0000 | QL-STU |
| 0001 | QL-PRS |
| 0010 | QL-INV2 |
| 0011 | QL-INV3 |
| 0100 | QL-TNC |
| 0101 | QL-INV5 |
| 0110 | QL-INV6 |
| 0111 | QL-ST2 |
| 1000 | QL-INV8 |
| 1001 | QL-INV9 |
| 1010 | QL-ST3 |
| 1011 | QL-INV11 |
| 1100 | QL-SMC |
| 1101 | QL-ST3E |
| 1110 | QL-PROV |
| 1111 | QL-DUS |
| NOTE – First-generation quality-level equipment will not receive SSM codes 0100 (QL-TNC) or 1101 (QL-ST3E); output ports of second-generation quality-level equipment will be provisioned in this case to output 1010 (QL-ST3) instead [or 1110 (QL-PROV)]. Refer to clause 5.5.2.2. |  |

Table 11 – Interpretation of synchronization status message codes 
in 1544 kbit/s signals in option II synchronization networks

| SSM code in 1544 kbit/s signals |  | QL interpretation |
| --- | --- | --- |
| 0xxx xxx0 1111 1111 | HEX |  |
| 000 010 | 04FF | QL-PRS |
| 000 100 | 08FF | QL-STU |
| 000 110 | 0CFF | QL-ST2 |
| 111 100 | 78FF | QL-TNC |
| 111 110 | 7CFF | QL-ST3E |
| 001 000 | 10FF | QL-ST3 |
| 010 001 | 22FF | QL-SMC |
| 010 100 | 28FF | QL-ST4 |
| 011 000 | 30FF | QL-DUS |
| 100 000 | 40FF | QL-PROV |
| Other (see Note 1) | Other (see Note 1) | QL-INV |
| NOTE 1 – Other codes, not related to synchronization, are defined in Table 2 of [ITU-T G.704]. NOTE 2 – [ITU-T G.704] presents the data link messages in reverse order "1111 1111 0xxx xxx0". NOTE 3 – First-generation quality-level equipment will not receive SSM codes 111 100 (QL-TNC) or 111 110 (QL-ST3E); output ports of second-generation quality-level equipment will be provisioned in this case to output 001 000 (QL-ST3) instead [or 100 000 (QL-RES)]. Refer to clause 5.5.2.2. |  |  |

#### 5.5.3.3	Option III synchronization networking

| Table 12 – Interpretation of synchronization status message codes in option III synchronization networks |  |  |
| --- | --- | --- |
| SSM code [MSB..LSB] | QL interpretation |  |
| 0000 | QL-UNK |  |
| 0001 | QL-INV1 |  |
| 0010 | QL-INV2 |  |
| 0011 | QL-INV3 |  |
| 0100 | QL-INV4 |  |
| 0101 | QL-INV5 |  |
| 0110 | QL-INV6 |  |
| 0111 | QL-INV7 |  |
| 1000 | QL-INV8 |  |
| 1001 | QL-INV9 |  |
| 1010 | QL-INV10 |  |
| 1011 | QL-SEC |  |
| 1100 | QL-INV12 |  |
| 1101 | QL-INV13 |  |
| 1110 | QL-INV14 |  |
| 1111 | QL-INV15 |  |

## 5.6	Selection process

The process of selecting a synchronization source from the set of physical ports is performed in three steps, plus one for use of the station clock output:

Figure 6 – Visualization of the synchronization source selection process(es)

1)	Assignment of a physical port to be a synchronization source: Select a (limited) set of interface signals (from the total set of interfaces) to act as synchronization sources.

This is performed in the SD_C function by means of adding matrix connections between a group of inputs (connected to the server layer) and outputs (connected to the SD_TT_Sk functions).

2)	Nomination of a synchronization source for an automatic selection process: Select a (sub)set of the synchronization sources to contribute to a selection process.

This is performed in the NS_C function by means of assigning a priority to the synchronization source (see clause 5.10).

3)	Automatic selection process: Selects the "best" synchronization source of the set from nominated sources according to the selection algorithm (see clause 5.12).

The next step is required only for use of the station clock output:

4)	Configuration of the station clock output selector: The selector is normally configured to allow the selected line synchronization source to be outputted to the SSU via the station clock output. An operator command allows the selection of the internal clock to be outputted, if required. This selection is dependent only on the operator command and not on the status of the selected signal.

NOTE – The specifications in this Recommendation allow a selection to be made between any set of synchronization interface signals input to a network element, independent of the actual synchronization network architecture deployed in the network. It is the network operator's responsibility to ensure that timing loops are not created.

## 5.7	Signal fail

Signal fail for a synchronization source is activated in case of defects detected in the server layers. In addition, an unconnected synchronization signal has also signal fail active in order to allow correct processing in the QL-disabled mode. Inclusion of specific synchronization failures (e.g., exceeded frequency deviation, exceeded wander limits), as signal fail criteria for SSU, are for further study.

In order to avoid reactions on short pulses or intermittent signal fail information, the signal fail information is passed through hold-off and wait-to-restore processes before it is considered by the selection process.

NOTE 1 – The delay of the signal fail information is only performed for the information passed to the selection process. The signal fail information for the main data path to the output of the NS_C function is not delayed.

In QL-enabled mode, the QL of a synchronization source with active signal fail is set to QL-FAILED. The selection process will react to this QL value instead of the signal fail signal in this mode.

NOTE 2 – Due to different persistence times for defect detection and the SSM acceptance process, a defect leading to signal fail could also result in a change of the QL value shortly before signal fail is activated. The implementation has to ensure that the selection process does not select a new synchronization source based on this intermediate QL value.

## 5.8	Hold-off time

The hold-off time ensures that short activation of signal fail are not passed to the selection process.

In QL-disabled mode, signal fail shall be active for the hold-off time before it is passed to the selection process.

In QL-enabled mode, a QL value of QL-FAILED shall exist for the hold-off time before it is passed to the selection process. In the meantime, the previous QL value is passed to the selection process.

NOTE – Other QL values than QL-FAILED will be passed to the selection process immediately.

Separate hold-off timers are used for each input to a selection process (nominated source).

The hold-off time is fixed in the range of 300 ms to 1800 ms.

## 5.9	Wait-to-restore time

The wait-to-restore time ensures that a previous failed synchronization source is only again considered as available by the selection process if it is fault-free for a certain time.

In QL-disabled mode, after deactivation of signal fail, it shall be false for the wait-to-restore time before signal fail false is passed to the selection process. In the meantime, signal fail true is passed to the selection process.

In QL-enabled mode, after a change of the quality level from QL-FAILED to any other value, the quality value shall be different from QL-FAILED for the wait-to-restore time before the new QL value is passed to the selection process. In the meantime, the quality level QL-FAILED is passed to the selection process.

Separate wait-to-restore timers are used for each input to a selection process (nominated source).

The wait-to-restore time is configurable in the range of 0 to 12 minutes, in steps that are no greater than 1 minute, for all inputs of a selection process in common. The default value is 5 minutes.

Each wait-to-restore timer can be cleared with a separate clear command. If a wait-to-restore timer is cleared, the new QL value (in QL-enabled mode), or signal fail value (in QL-disabled mode), is immediately passed to the selection process.

## 5.10	Synchronization source priorities

In order to define a preferred network synchronization flow, priority values are allocated to assigned synchronization sources within a network element (see Table 13).

Different priorities reflect a preference of one synchronization source over the other. Equal synchronization source priorities reflect that no preference exists between the synchronization sources. Within the group of synchronization sources with equal priorities, the selection process has a non-revertive behaviour.

A priority of "dis" (disabled) identifies that this assigned synchronization source is not nominated for the selection process.

Table 13 – Priority order

| Priority value | Order |
| --- | --- |
| 1 | Highest |
| 2 | ¦ |
| 3 | ¦ |
| : | ¦ |
| K | ¦ |
| dis, undef | Lowest |
| NOTE – The priority value is not numerically ordered. The following relation is present: "1" > "2" > "3" > .. >"K" > "undef", "dis". |  |

The priority value "undef" is associated with the unconnected signal of the NS_C function and is not configurable from the outside.

NOTE – The assigning of equal priorities to synchronization sources, in order to allow for non-revertive operation, does not allow for a pre-defined initialization state of known synchronization configuration following failure of a higher priority source.

## 5.11	External commands

Several external commands are available to the user (e.g., for maintenance purposes). These commands are independent and have different impacts on the selection processes.

### 5.11.1	External commands per nominated synchronization source

It is possible to temporarily remove a timing source as available synchronization source for the selection process.

This is controlled by the lockout commands. Lockout commands are accepted for nominated synchronization sources (synchronization sources that are not disabled) of each selection process.

The lockout status of a disabled synchronization source is "off".

NOTE – A locked out source is still nominated to the selection process and retains its synchronization source priority.

#### 5.11.1.1	Set_Lockout#p command

The Set_Lockout#p command sets the lockout state of input p to "on". This causes this input to be no longer considered available by the selection process.

#### 5.11.1.2	Clear_Lockout#p command

The Clear_Lockout#p command sets the lockout state of input p to "off". This causes this input to be considered available again by the selection process.

### 5.11.2	External commands per selection process

The activation and deactivation of external commands associated with the synchronization selection process are defined below. Furthermore, only one of these external commands is active at a time, per the selection process.

#### 5.11.2.1	Clear command

A clear (CLR) command clears the forced switch and manual switch commands.

#### 5.11.2.2	Forced switch #p command

A forced switch (FSw) to #p command can be used to override the currently selected synchronization source, assuming the synchronization source #p is enabled and not locked out.

The forced switch overrides the manual switch, and a subsequent forced switch pre-empts the previous forced switch.

If the source selected by the forced switch command (#p) is disabled or locked out, the forced switch command is automatically rejected.

The forced switch command can be cleared by the "clear" command.

NOTE – A forced switch command to a synchronization source #p that is in the SF state or has a QL of DNU in QL enabled mode will result in the network element entering holdover.

#### 5.11.2.3	Manual switch #p command

A manual switch (MSw) to #p command selects the synchronization source #p, assuming it is enabled, not locked out, not in signal fail condition, and has a QL better than DNU in QL-enabled mode. Furthermore, in QL-enabled mode, a manual switch can be performed only to a source which has the highest available QL. As such, these conditions have the effect that manual switching can only be used to override the assigned synchronization source priorities.

A manual switch request overrides a previous manual switch request.

If the source selected by the manual switch command (#p) is disabled, locked out, in signal fail or has a QL of DNU or lower than one of the other source signals, the manual switch command is automatically rejected.

The manual switch command can be cleared by the "clear" command.

## 5.12	Automatic reference selection process

One or more reference selection processes operate independently to select the reference signal for the internal clock and, where present, the station clock output(s). However, the SD connection function delivering SD_CI to the station clock output atomic functions (see Figure 15) is only operated by operator command and not by an automatic process.

The selection process(es) can work in two distinct modes: QL-enabled and QL-disabled. If multiple selection processes are present in a network element, all processes work in the same mode.

The following is a brief description of the automatic reference selection process. The specific details (SDL diagrams) are defined in Annex A.

### 5.12.1	QL-enabled mode

In QL-enabled mode, the following parameters contribute to the selection process specified in Annex A:

–	quality level;

–	signal fail via QL-FAILED;

–	priority;

–	external commands.

NOTE – The number of cascaded SECs/eSECs considered in the selection process is for further study.

Note that if no overriding external commands are active, the algorithm selects the reference with the highest quality level that is not experiencing a signal fail condition. If multiple inputs have the same highest quality level, the input with the highest priority is selected. For the case that multiple inputs have the same highest priority and quality level, the current existing selected reference is maintained if it belongs to this group, otherwise an arbitrary reference from this group is selected.

If no input could be selected, the function outputs the unconnected signal.

### 5.12.2	QL-disabled mode

In QL-disabled mode, the following parameters contribute to the selection process specified in Annex A:

–	signal fail;

–	priority;

–	external commands.

Note that if no overriding external commands are active, the algorithm selects the reference with the highest priority that is not experiencing a signal fail condition. For the case that multiple inputs have the same highest priority, the current existing selected reference is maintained if it belongs to this group, otherwise an arbitrary reference from this group is selected.

If no input could be selected, the function outputs the unconnected signal.

## 5.13	Timing loop prevention

Synchronization network architectures should be designed such that timing loops will not occur under fault-free or failure conditions (refer to [ITU-T G.803]). Synchronization network architectures can be designed such that NE or SASE A will time NE or SASE B, and NE or SASE B will never time NE or SASE A. In cases where such rigorous synchronization network architecture is not viable, special timing loop prevention techniques should be used within network elements and SASEs that will open the potential loop. Some of these are described hereafter.

### 5.13.1	Station clock input used as a source for station clock output

This Recommendation allows the use of the station clock input as a source for the station clock output, either directly or via the SEC. When this functionality is present in a network element, the operator should be aware that this functionality is intended for timing quality monitoring purposes and that its use for other purposes could result in timing loops being created (see Figure 7). If a timing loop could be created, the operator should prevent it by a reconfiguration of the synchronization architecture.

Figure 7 – Station clock output derived from station clock input

NOTE – The inputs and outputs shown in Figure 7 are from the perspective of the NE.

### 5.13.2	Between NEs with SEC type clocks

The master-slave synchronization over several NEs with multiple possible synchronization inputs for protection of synchronization, as defined in [ITU-T G.803], could lead to timing loops between NEs. To avoid timing loops, an NE should insert an SSM/TM value of DNU in the direction of the NE, which is used as the actual synchronization source for the NE clock (SEC).

Figure 8 – Automatic DNU generation in an NE with SEC timing

The clock-source identifier CSid has been introduced to support the above feature as shown in Figure 8. To each transport and station clock input port, a unique CSid is assigned (MI_CSid). This ID is processed in the synchronization layers together with the clock and quality level of the port. The CSid of the selected source for the SEC is distributed to all output ports. If a transport output port receives the same CSid as its associated input port (signalled via RI_CS) via the synchronization distribution layer (SD_CI_CS), the outgoing SSM/TM is set to DNU.

NOTE – The above principle may be extended to generate DNUs on groups, "bundles", of ports, which are known to have the same timing source. A provisional agreement is that processing of DNU generation on all the ports of the "bundle", when any one of them has been selected as the reference source, does not require additional information between the atomic functions. Use of an identical CS within the "bundle" has been considered but this is left for further study.

### 5.13.3	Between NEs with an SEC clock and an NE or SASE/BITS with an SSU/ST2 clock and only one link

NOTE 1 – As stated in clause 1, the application of the SSM message algorithm is not recommended in SSUs and between SECs and SSUs since this would, in a general sense, result in using SSMs over a whole operator domain. Given the current uncertainties in the specification of the SSM algorithm in synchronization networks with mixed SSU and SEC clock equipment, this cannot be recommended as a general rule. Interworking between SEC and SSU clock equipment is assumed to take place over interfaces that do not support SSM transport (see clause 5.13.3.1). An exception can be made for certain specific topologies in option II networks (see clause 5.13.3.2). The general case remains for further study.

NOTE 2 – "One link" refers to the logical connection between NE and SASE/BITS. One (logical) link may be realized by multiple physical connections (e.g., for availability reasons), each carrying the same clock information.

An NE can be interconnected with an SASE/BITS via its (64 kbit/s and 6 MHz, 2 MHz, 2 Mbit/s, 1.5 Mbit/s) station clock input and output ports. If the SASE/BITS is used as the actual synchronization source for the NE clock, the mechanism defined in clause 5.13.2 above has to be extended to support automatic DNU insertion also for this case.

It is not possible to detect that the SASE/BITS has selected the station clock output port of the NE as the actual clock source, but several conditions exist that indicate that the station clock output port is not used as the clock source by the SASE/BITS.

If an NE is connected to a SASE/BITS, remote information is exchanged between the normally unidirectional station clock input and output ports connected to the same SASE/BITS. The remote information transfers the CSid (CI_CS = RI_CS) and, in addition in case of QL enabled mode, the QL (CI_QL = RI_QL) of the clock signal selected for the station clock output to the station clock input. The user has to enable this feature by activating the remote indication connection between the station clock ports.

#### 5.13.3.1	QL/SSM processing not supported between SASE/BITS and NE

If QL/SSM processing is not supported by the SASE/BITS, station clock ports or NE, a squelched/AIS station clock output port is the only criteria that indicates that the output port is not used as the clock source by the SASE/BITS. As long as the station clock output is not squelched (for 2 and 6 MHz station clock ports) or set to AIS (for 2 Mbit/s station clock ports), it is assumed that the SASE/BITS selects the station clock output of the NE as the reference clock. The station clock input port uses, in this case, the remote CSid (from the station clock output) as CSid for the clock signal to the selection process (RI_CS = CI_CS) instead of its own CSid (MI_CSid). This will result in DNU insertion in the traffic output port associated with the traffic input port used as source for the station clock (see Figure 9). If the station clock output is squelched or set to AIS, the remote CSid is replaced by the own CSid (MI_CSid) and the automatic DNU insertion in the traffic output port associated with the traffic input port used as source for the station clock is removed (see Figure 10).

There are still conditions in which the SASE/BITS does not select the station clock output of the NE as the synchronization source, but the automatic DNU insertion is still performed, e.g., if the SASE/BITS selects another source if the station clock output is still transmitting valid timing information (see Figure 11).

Figure 9 – Automatic DNU generation in an NE with SASE/BITS timing 
(SSM/QL not supported)

Figure 10 – Removal of automatic DNU generation in an NE with 
SASE/BITS timing (SSM/QL not supported)

Figure 11 – Limitation of automatic DNU generation in an NE with SASE/BITS timing
(SSM/QL not supported)

#### 5.13.3.2	QL/SSM processing supported between SASE/BITS and NE (option II)

For the following specific topologies, and only within option II networks, the usage of SSMs in SSUs, and on the interface between SSUs and SECs, is recommended:

–	A single ring consisting of SEC-based equipment, with collocated SSUs that filter the timing signal in some or all of the ring nodes. At present, it is recommended that SSMs should not be exchanged with the rest of the network.

•	In this application, the frequency derived from the SDH line and the SSM (i.e., the value indicated by the S1 byte) needs to be transferred from the SDH line to the SASE. In this case, the SDH NE is externally timed and must translate the SSM received from the SASE to the S1 bytes on SDH lines that are transmitted from that NE.

•	Field trials of preliminary SSM implementations have confirmed that mode switching (i.e., switching between externally timed and line timed operation) may lead to network instability and should not be used. Subsequent trials also indicate that appropriate and desirable synchronization switching is achieved using SSMs when mode switching is not used. These trials have also indicated some sensitivity to SSM processing delay times both in the NE and in the SASE.

•	Careful synchronization engineering remains mandatory to avoid potential timing loops or instabilities in the SSM algorithm.

–	A linear chain consisting of SEC-based equipment with collocated SSUs that filter the timing signal in some or all of the nodes. At present, it is recommended that SSMs should not be exchanged with the rest of the network.

For option I and III networks these exceptions do not apply.

If QL/SSM processing is supported by the SASE/BITS and all other involved components (NE, station clock ports), different SSM values at the output and input station clock ports indicate that the output port is not used as the clock source by the SASE/BITS. As long as the transmitted QL at the station clock output and the received QL at the station clock input of the NE are identical, it is assumed that the SASE/BITS selects the station clock output of the NE as the reference clock. The station clock input port uses, in this case, the remote CSid (from the station clock output) as CSid for the clock signal to the selection process (RI_CS = CI_CS) instead of its own CSid (MI_CSid). This will result in DNU insertion in the traffic output port associated with the traffic input port used as the source for the station clock (see Figure 12). If the transmitted and received QL are no longer identical, the remote CSid is replaced by the own CSid (MI_CSid) and the automatic DNU insertion in the traffic output port associated with the traffic input port used as the source for the station clock, is removed (see Figure 13). There are still conditions in which the SASE/BITS does not select the station clock output of the NE as the synchronization source, but the automatic DNU insertion is still performed, e.g., if the SASE/BITS selects another source with the same QL as the station clock output of the NE (see Figure 14).

Figure 12 – Automatic DNU generation in an NE with SASE/BITS timing
(SSM/QL supported)

Figure 13 – Removal of automatic DNU generation in an NE with SASE/BITS timing (SSM/QL supported)

Figure 14 – Limitation of automatic DNU generation in an NE with SASE/BITS timing (SSM/QL supported)

### 5.13.4	Between NEs with an SEC clock and an NE or SASE/BITS with an SSU clock and several links

NOTE – "Several links" refers to the logical connections from an NE to an SASE/BITS. Each link carries information from a different clock.

A generalization of the timing loop prevention mechanism described in clause 5.13.3, applicable when NE and SASE/BITS are interconnected by several independent links, is for further study.

The necessity to support multiple independent links from an NE to an SASE in option I and option III SDH, OTN and packet transport network elements is for further study. SDH, OTN and packet transport network elements deployed in option II networks are required to support a minimum of two independent links from an NE to a BITS (see Figure 15).

Figure 15 – Example of multiple independent links from NE to BITS

NOTE – The inputs and outputs shown in Figure 15 are from the perspective of the NE.

## 5.14	Delay times for NEs with SEC/ST3/SIC

### 5.14.1	Delay times for NEs with SEC in option I and III synchronization networks

The following delay times are caused by the atomic functions which perform the selection of the input synchronization reference. Three delays are defined:

–	Holdover message delay THM

This delay applies when the SEC shall enter holdover because of loss of signal of the input reference and lack of any other available reference. When this event occurs, the SEC goes immediately into holdover but changes the output SSM to the holdover code after a delay which has been defined to be between 300 ms and 2000 ms.

–	Non-switching message delay TNSM

This delay applies when the QL of the selected synchronization source changes but no switchover to another source is performed. The outgoing SSM/TM follows this change at the input within a time defined to be less than 200 ms.

–	Switching message delay TSM

This delay applies when a new synchronization source is selected. The output SSM change, if any, is done after a delay that has been defined to be between 180 ms and 500 ms.

NOTE 1 – The CS signal is used in the SD/NS-SEC_A_So function to detect that a new synchronization source has been selected.

A full description of these times is in Appendix III.

Change of the synchronization direction within a chain of 20 SECs

The above delay times allow the reversal of the synchronization flow in a chain of 20 NEs with SEC timing within 15.6 seconds. The change of the synchronization direction through 20 SECs requires 39 steps, as shown in Table 14.

Figure 16 – Linear chain of SECs

NOTE 2 – The interfaces between the NEs in Figure 16 might be a combination of STM-N and synchronous ethernet interfaces.

Table 14 – Steps of reference switch with a linear chain of 20 SECs

| Step | Action |
| --- | --- |
| 1 | Ref. 1 disappears from the first NE of the chain, NE 1 goes into holdover mode and transmits a new SSM. (THM 2 s maximum) |
| 2 to 19 | NE n (n = 2,3,.,19) transmits the new SSM without switch of reference to NE n. (TNSM 200 ms maximum) |
| 20 | NE 20 switches to Ref. 2. (TSM 500 ms maximum) |
| 21 to 39 | NE n (n = 19,18,.,1) changes to sync received from NE n+1. (TSM 500 ms maximum) |

This leads to a maximum restoration time of 15.6 seconds (THM + 18 TNSM + 20 TSM).

### 5.14.2	Delay times for NEs with ST3/SMC in option II synchronization networks

For further study.

## 5.15	Delay times for NEs with SSU/ST2 or for SASE/BITS

For further study.

## 5.16	Synchronization layer functions

The atomic functions which are involved in the transport of synchronization within the NE are shown in Figure 17.

This figure shows two synchronization layers plus the transport layer:

a)	Synchronization distribution layer: This layer terminates and adapts the synchronization trails to the network synchronization layer and performs a pre-selection of candidate input ports.

b)	Network synchronization layer: This layer performs the selection of a timing reference.

c)	Transport layer: This layer provides the synchronization-related SD_CI information.

Figure 17 – Synchronization distribution and network synchronization layer atomic functions

## 5.17	Overview of the processes performed within the atomic functions

A list of the synchronization atomic functions and a short description of their functionality is given in Table 15. For a more detailed description, see clauses 6 to 9.

| Table 15 – Functional overview of atomic functions |  |
| --- | --- |
| Atomic function | Functionality |
| XX-LC_A_So (Note) | Generation of the layer timing. |
| XX/SD_A_Sk (Note) | Access to reference clock. SSM(TM) acceptance and extraction of QL. Generate QL-NotSupported if signal does not support SSM. Generation of CS. |
| XX/SD_A_So (Note) | Insertion of QL into SSM(TM). Generate QL-DNU or squelch to prevent timing loops. |
| SD_C | Preselection of transport interfaces as possible synchronization sources. Selection of sources for the station clock outputs. |
| SD_TT_Sk | Report of QL to management. Manual insertion of a fixed QL value. |
| SD_TT_So | None. |
| SD/NS-SEC_A_Sk | None. |
| SD/NS_A_So | Generation of the clock for the station clock outputs from the selected synchronization reference. |
| SD/NS-SEC_A_So | Generation of holdover, locked and free-run modes timings. Generation of the NE clock (SEC type), locked to the selected synchronization reference. |
| NS_C | Selection of synchronization reference sources. |
| NOTE – 'XX' represents MSn, Tq, Sn, ETY, ODUk, OSC, or Pqs. |  |

## 5.18	Interworking between option I, II and III synchronization networks

Interworking between option I and II, option I and III and option II and III synchronization networks is not required.

# 6	Synchronization distribution layer atomic functions

Figure 18 – Synchronization distribution layer atomic functions

SD layer CP

The CI at this point is a clock signal with associated server signal fail, quality level and clock-source identifier.

SD layer AP

The AI at this point is a clock signal with associated trail signal fail, quality level and clock-source identifier.

## 6.1	SD connection function (SD_C)

Symbol:

Figure 19 – SD_C symbol

Interfaces:

Table 16 – SD_C input and output signals

| Input(s) | Output(s) |
| --- | --- |
| Per SD_CI, n for the function: SD_CI_CK SD_CI_QL SD_CI_SSF SD_CI_CS SD_CI_CLKID SD_CI_mixedChain SD_CI_partialChain SD_CI_numeSECs SD_CI_numSECs Per input output connection point SD_C_MI_ConnectionPortIds | Per SD_CI, m for the function: SD_CI_CK SD_CI_QL SD_CI_SSF SD_CI_CS SD_CI_CLKID SD_CI_mixedChain SD_CI_partialChain SD_CI_numeSECs SD_CI_numSECs |

Processes:

In the SD_C function, SD layer characteristic information is routed between input (termination) connection points [(T)CPs] and output (T)CPs by means of matrix connections.

NOTE 1 – Neither the number of input/output signals to the connection function, nor the connectivity is specified in this Recommendation. That is a property of individual network elements.

Routing: The function shall be able to connect a specific input with a specific output by means of establishing a matrix connection between the specified input and output. It shall be able to remove an established matrix connection.

NOTE 2 – Broadcast connections are handled as separate connections to the same input CP.

Unconnected SD signal generation: The function shall generate an unconnected SD signal, specified as: SSF true, CS value none, QL value QL-UNC and undefined clock.

NOTE 3 – The unconnected signal is a logical signal defined for the purpose of this formal specification; it is not observable at any of the network element transport interfaces.

Defects: None.

Consequent actions:

If an output of this function is not connected to one of its inputs, the function shall connect the unconnected SD signal to the output.

Defect correlations: None.

Performance monitoring: None.

## 6.2	SD trail termination functions

### 6.2.1	SD trail termination source function (SD_TT_So)

Symbol:

Figure 20 – SD_TT_So symbol

Interfaces:

Table 17 – SD_TT_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_AI_CK SD_AI_QL SD_AI_CS SD_AI_TSF SD_AI_CLKID SD_AI_mixedChain SD_AI_partialChain SD_AI_numeSECs SD_AI_numSECs | SD_CI_CK SD_CI_QL SD_CI_CS SD_CI_SSF SD_CI_CLKID SD_CI_mixedChain SD_CI_partialChain SD_CI_numeSECs SD_CI_numSECs |

Processes:

Output SD_CI_CK is derived from and locked to SD_AI_CK.

Defects: None.

Consequent actions:

aSSF		AI_TSF

Defect correlations: None.

Performance monitoring: None.

### 6.2.2	SD trail termination sink function (SD_TT_Sk)

Symbol:

Figure 21 – SD_TT_Sk symbol

Interfaces:

Table 18 – SD_TT_Sk input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_CK SD_CI_QL SD_CI_SSF SD_CI_CS SD_CI_CLKID SD_CI_mixedChain SD_CI_partialChain SD_CI_numeSECs SD_CI_numSECs SD_TT_Sk_MI_QLoverwrite SD_TT_Sk_MI_QLfixedValue SD_TT_Sk_MI_QLmode SD_TT_Sk_MI_Tpmode SD_TT_Sk_MI_SSF_Reported | SD_AI_CK SD_AI_QL SD_AI_TSF SD_AI_CS SD_AI_CLKID SD_AI_mixedChain SD_AI_partialChain SD_AI_numeSECs SD_AI_numSECs SD_TT_Sk_MI_cSSF SD_TT_Sk_MI_QL SD_MI_CLKID SD_MI_mixedChain SD_MI_partialChain SD_MI_numeSECs SD_MI_numSECs |

Processes:

This function terminates a synchronization trail transmitted via one of the synchronization information's transport layers, processes and reports the incoming quality. It can operate in QL-enabled mode and QL-disabled mode.

QL-disabled mode:

In QL-disabled mode, the function shall report the status of the trail (MI_cSSF).

QL-enabled mode:

In QL-enabled mode, the function shall report the status of the trail (MI_cSSF) and the incoming quality level value (CI_QL) via MI_QL.

In QL-enabled mode, the function shall report the incoming SyncE clockIdentity of the originator of the eQL-TLV (CI_CLKID) via MI_CLKID.

In QL-enabled mode, the function shall report the incoming mixed chain Flag bit of the eQL-TLV (CI_mixedChain) via MI_mixedChain.

In QL-enabled mode, the function shall report the incoming partial chain Flag bit of the eQL-TLV (CI_partialChain) via MI_partialChain.

In QL-enabled mode, the function shall report the incoming number of cascaded eSECs from the nearest SSU/PRC of the eQL-TLV (CI_numeSECs) via MI_numeSECs.

In QL-enabled mode, the function shall report the incoming number of cascaded SECs from the nearest SSU/PRC of the eQL-TLV (CI_numSECs) via MI_numSECs.

In QL-enabled mode, AI_CS=CI_CS, in pass through and overwrite operations.

The function shall support the ability to pass through or overwrite the incoming quality level information.

Pass through:

The quality-level output (AI_QL) shall be related to the quality-level input (CI_QL) signal as specified in clause 5 and Tables 19-21.

AI_CLKID = CI_CLKDI

AI_mixedChain = CI_mixedChain

AI_partialChain = CI_partialChain

AI_numeSECs = CI_numeSECs

AI_numSECs = CI_numSECs

Overwrite:

For option I, the quality level-output (AI_QL) is a fixed value provisioned via MI_QLfixedValue. The selection between pass through and overwrite mode shall be controlled via MI_QLoverwrite. The default value of MI_QLoverwrite shall be FALSE. The default for MI_QLfixedValue shall be QL-DNU.

For options II and III, overwrite is not defined.

| Table 19 – Conversion of quality levels for option I synchronization networks |  |  |  |  |
| --- | --- | --- | --- | --- |
| CI_QL | MI_QLoverwrite | CI_SSF | AI_QL |  |
| QL-INV | False | False | QL-INV |  |
| QL-INV0 | False | False | QL-INV |  |
| QL-INV1 | False | False | QL-INV |  |
| QL-ePRTC | False | False | QL-ePRTC |  |
| QL-PRTC | False | False | QL-PRTC |  |
| QL-PRC | False | False | QL-PRC |  |
| QL-INV3 | False | False | QL-INV |  |
| QL-SSU-A | False | False | QL-SSU-A |  |
| QL-INV5 | False | False | QL-INV |  |
| QL-INV6 | False | False | QL-INV |  |
| QL-INV7 | False | False | QL-INV |  |
| QL-SSU-B | False | False | QL-SSU-B |  |
| QL-INV9 | False | False | QL-INV |  |
| QL-INV10 | False | False | QL-INV |  |
| QL-eSEC | False | False | QL-eSEC |  |
| QL-SEC | False | False | QL-SEC |  |
| QL-INV12 | False | False | QL-INV |  |
| QL-INV13 | False | False | QL-INV |  |
| QL-INV14 | False | False | QL-INV |  |
| QL-DNU | False | False | QL-DNU |  |
| QL-NSUPP | False | False | QL-NSUPP |  |
| QL-UNC | False | True | QL-FAILED |  |
| All | True | False | MI_QLfixedValue |  |
| All | x | True | QL-FAILED |  |

| Table 20 – Conversion of quality levels for option II synchronization networks |  |  |
| --- | --- | --- |
| CI_QL | CI_SSF | AI_QL |
| QL-INV | False | QL-INV |
| QL-STU | False | QL-STU |
| QL-ePRTC | False | QL-ePRTC |
| QL-PRTC | False | QL-PRTC |
| QL-PRS | False | QL-PRS |
| QL-INV2 | False | QL-INV |
| QL-INV3 | False | QL-INV |
| QL-TNC | False | QL-TNC |
| QL-INV5 | False | QL-INV |
| QL-INV6 | False | QL-INV |
| QL-ST2 | False | QL-ST2 |
| QL-INV8 | False | QL-INV |
| QL-INV9 | False | QL-INV |
| QL-ST3 | False | QL-ST3 |
| QL-INV11 | False | QL-INV |
| QL-SMC | False | QL-SMC |
| QL-ST3E | False | QL-ST3E |
| QL-PROV | False | QL-PROV |
| QL-DUS | False | QL-DUS |
| QL-NSUPP | False | QL-NSUPP |
| QL-UNC | True | QL-FAILED |
| All | True | QL-FAILED |

| Table 21 – Conversion of quality levels for option III synchronization networks |  |  |
| --- | --- | --- |
| CI_QL | CI_SSF | AI_QL |
| QL-ePRTC | False | QL-ePRTC |
| QL-PRTC | False | QL-PRTC |
| QL-UNK | False | QL-UNK |
| QL-INV1 | False | QL-INV |
| QL-INV2 | False | QL-INV |
| QL-INV3 | False | QL-INV |
| QL-INV4 | False | QL-INV |
| QL-INV5 | False | QL-INV |
| QL-INV6 | False | QL-INV |
| QL-INV7 | False | QL-INV |
| QL-INV8 | False | QL-INV |
| QL-INV9 | False | QL-INV |
| QL-INV10 | False | QL-INV |
| QL-eSEC | False | QL-eSEC |
| QL-SEC | False | QL-SEC |
| QL-INV12 | False | QL-INV |
| QL-INV13 | False | QL-INV |
| QL-INV14 | False | QL-INV |
| QL-INV15 | False | QL-INV |
| QL-NSUPP | False | QL-NSUPP |
| QL-UNC | True | QL-FAILED |
| All | True | QL-FAILED |

Defects: None.

Consequent actions:

aTSF		CI_SSF

Defect correlations:

cSSF		MON and CI_SSF and SSF_Reported

Performance monitoring: None.

## 6.3	SD adaptation functions

NOTE – This clause describes the SD layer to NS layer quality adaptation functions for SEC. These functions for other clock types (e.g., [ITU-T G.812]) are for further study.

### 6.3.1	SD layer to NS layer SEC quality adaptation source function (SD/NS-SEC-A_So)

Symbol:

Figure 22 – SD/NS-SEC_A_So symbol

Interfaces:

Table 22 – SD/NS-SEC_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| NS_CI_CK NS_CI_QL NS_CI_SSF NS_CI_CS SD_CI_CLKID SD_CI_mixedChain SD_CI_partialChain SD_CI_numeSECs SD_CI_numSECs SD/NS-SEC_A_So_MI_CkOperation SD/NS-SEC_A_So_MI_QLMode | SD_AI_CK SD_AI_QL SD_AI_CS SD_AI_CLKID SD_AI_mixedChain SD_AI_partialChain SD_AI_numeSECs SD_AI_numSECs SD/NS-SEC_A_So_MI_CkMode SD/NS-SEC_A_So_MI_cLTI |

Processes:

This function generates a SEC type system clock as defined in [ITU-T G.803] and specified in [ITU-T G.813] or [ITU-T G.8262]. The function shall operate in QL-enabled or QL-disabled mode as selected by MI_QLMode.

The function shall support three types of operation:

–	forced free-run operation working in the free-run mode;

–	forced holdover operation, working in the holdover mode;

–	normal operation, working in the locked or holdover mode depending on the input signals.

These three types of operation are activated by user management input (CkOperation) while modes, defined in [ITU-T G.810], are automatically activated by the status of input signals. Figure 23 shows the relationship between types of operation and modes.

Figure 23 – Operational types

Bandwidth, transients, pull in and pull out ranges, noise, input and output jitter for locked mode operation, holdover accuracy and output phase deviation for holdover mode operation, frequency accuracy, transients, noise and output jitter for free-run mode operation shall be as specified in [ITU-T  G.813] or [ITU-T G.8262].

Forced free-run operation:

This type of operation is activated by a management command, the function is in free-run mode:

–	Clock generation:

The outgoing clock (AI_CK) is not defined by an incoming reference or stored incoming reference data in the holdover memory. The holdover memory is reset to a default value.

–	QL processing (in QL-enabled mode):

The outgoing QL of the free-run mode is QL-SEC.

–	Processing of other eQL-TLV information, if present and supported:

CI_CLKID is copied to AI_CLKID. CI_mixedChain, CI_partialChain, CI_numeSECs, and CI_numSECs are processed as described in clauses 11.3.1.3 and 11.3.1.4 of [ITU-T G.8264].

–	CS processing:

The outgoing CS of the free-run mode is "none".

NOTE 1 – The term "free-run" is equivalent to the term "free running" in [ITU-T G.810].

Forced holdover operation:

This type of operation is activated by a management command, the function is in holdover mode.

–	Clock generation:

The outgoing clock (AI_CK) is defined by stored reference data in the holdover memory.

–	QL processing (in QL-enabled mode):

The outgoing QL of the holdover mode is QL-SEC.

–	Processing of other eQL-TLV information, if present and supported:

CI_CLKID is copied to AI_CLKID. CI_mixedChain, CI_partialChain, CI_numeSECs, and CI_numSECs are processed as described in clauses 11.3.1.3 and 11.3.1.4 of [ITU-T G.8264].

–	CS processing:

The outgoing CS of the holdover mode is "none".

Auto selection operation:

This type of operation is activated by a management command. The auto selection operation works according to four modes: free-run, locked-acquiring holdover, locked-holdover acquired and holdover:

–	Free-run mode: The free-run mode conforms to the free-run mode defined above. It is essentially a temporary mode until a valid source is available.

–	Locked mode, acquiring holdover: This is a temporary mode, when coming from free-run, to acquire holdover memory. It behaves as specified below.

As shown on Figure 23, some time is required before the holdover memory is acquired.

–	Locked mode, holdover acquired: This is a steady state mode, entered when holdover memory is acquired. It behaves as specified below.

–	Holdover mode: The holdover mode conforms to the holdover mode defined above. The holdover memory is no longer updated by the incoming reference clock.

The selection between the modes is done automatically depending on the quality of the incoming reference signal and the selected QL mode:

–	QL-enabled mode:

The locked mode is selected if the incoming reference is not in the signal fail state (SSF = false) and the quality level of the incoming reference is better than or equal to QL-SEC.

The holdover mode is selected without delay when the incoming reference goes into the signal fail state (SSF = true) or the quality level of the incoming signal is lower than QL-SEC.

The holdover mode is left when both the signal fail clears (SSF = false) and the quality level of the incoming signal is equal to or better than QL-SEC.

–	QL-disabled mode:

The locked mode is selected if the incoming reference is not in the signal fail state (SSF = false).

The holdover mode is selected when the incoming reference goes into the signal fail state (SSF = true).

The actual selected mode is reported to the management (MI_CkMode).

–	Clock generation:

In free-run mode, the outgoing clock (AI_CK) is generated as specified under forced free-run operation.

In the locked mode, the outgoing clock (AI_CK) is locked to the incoming reference clock (CI_CK) and the holdover memory is constantly updated with this reference clock.

In holdover mode, the outgoing clock (AI_CK) is generated as specified under forced holdover operation.

–	QL processing (in QL-enabled mode):

If the function is in the locked mode, the outgoing QL follows the incoming QL.

In case of a change of the selected synchronization source (which is detected by a change of the incoming CS), the outgoing QL shall be set to the new incoming QL after the settling time, ts, to allow the internal oscillator to adjust to a possible frequency change.

If the incoming QL changes without a change of the selected synchronization source (no change of the CI_CS), the outgoing QL shall follow without settling time.

If the function is in the holdover mode, the outgoing QL shall be set to QL-SEC as soon as the incoming CS value is "none" or if the incoming QL is too low (NS_CI_QL<"QL-SEC").

After leaving the holdover mode, the outgoing QL shall be set to the new incoming QL after the settling time, ts.

The settling time, ts, shall be in the range of 180 ms to 300 ms.

Processing of other eQL-TLV information, if present and supported:

CI_CLKID is copied to AI_CLKID. CI_mixedChain, CI_partialChain, CI_numeSECs, and CI_numSECs are processed as described in clauses 11.3.1.3 and 11.3.1.4 of [ITU-T G.8264].

–	CS processing:

Normally, the outgoing CS shall follow the incoming CS immediately.

If the function is in the holdover mode due to a too low QL value of the selected source (NS_CI_QL<"QL-SEC"), the outgoing CS shall be set to "none".

Defects:

Loss of timing inputs (dLTI)

The function shall detect a loss of timing inputs (dLTI) if an unconnected signal is present at its connection point (no input selected in NS_C) or if the input signal is failed (CI_SSF active). The defect is raised if CI_SSF = "true" or CI_CS = "none" for at least X seconds. The defect is cleared if CI_SSF = "false" and CI_CS  "none" for at least Y seconds. The values of X and Y are for further study.

Clock unlock defect

The function shall detect a clock unlock if the status of the system clock is "unlocked." The defect is raised if the status of the system clock is "unlocked." The defect is cleared if the status of the system clock is "locked". Note that "unlocked" is a status of the clock, but is not a clock mode.

Clock unlock status is for further study.

Frequency offset source performance degradation defect

If the monitored estimation of the frequency offset between the selected active input signal and the clock's internal oscillator is over a limit threshold for at least X seconds, then a frequency offset performance degradation defect is detected.

NOTE – In some cases, the source of the defect can be determined by detecting a degradation in the quality level of the selected active input signal.

The defect is cleared if the monitored estimation of the frequency offset between the selected active input signal and the clock's internal oscillator is below the threshold for at least Y seconds.

The frequency offset threshold, and the values of X and Y are for further study.

The dependency between the frequency offset performance degradation defect and the SSM level degradation defect is for further study.

Consequent actions: None.

Defect correlation:

cLTI		dLTI

Performance monitoring:

Frequency offset monitoring

The function shall monitor the estimated frequency offset between the selected active input signal and the clock's internal oscillator over an observation interval. The observation interval is for further study.

### 6.3.2	SD layer to NS layer SEC quality adaptation sink function (SD/NS-SEC_A_Sk)

Symbol:

Figure 24 – SD/NS-SEC_A_Sk symbol

Interfaces:

Table 23 – SD/NS-SEC_A_Sk input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_AI_CK SD_AI_QL SD_AI_TSF SD_AI_CS SD_AI_CLKID SD_AI_mixedChain SD_AI_partialChain SD_AI_numeSECs SD_AI_numSECs | NS_CI_CK NS_CI_QL NS_CI_SSF NS_CI_CS SD_CI_CLKID SD_CI_mixedChain SD_CI_partialChain SD_CI_numeSECs SD_CI_numSECs |

Processes:

This function connects input with output only. Currently, no processes are defined within this function.

Defects: None.

Consequent actions:

ASSF		AI_TSF

Defect correlation: None.

Performance monitoring: None.

### 6.3.3	SD layer to NS layer adaptation source function (SD/NS_A_So)

Symbol:

Figure 25 – SD/NS_A_So symbol

Interfaces:

Table 24 – SD/NS_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| NS_CI_CK NS_CI_QL NS_CI_SSF NS_CI_CS SD_CI_CLKID SD_CI_mixedChain SD_CI_partialChain SD_CI_numeSECs SD_CI_numSECs | SD_AI_CK SD_AI_QL SD_AI_CS SD_AI_CLKID SD_AI_mixedChain SD_AI_partialChain SD_AI_numeSECs SD_AI_numSECs SD_AI_TSF SD/NS_A_So_MI_cLTI |

Processes:

This function produces the station output clock process.

Wander limitation: The wander at the output of this function shall be within the MTIE mask specified in [ITU-T G.813] or [ITU-T G.8262].

NOTE – There might be a need for an AIS generator, this is for further study.

Defects:

The function shall detect a loss of timing inputs (dLTI) if an unconnected signal is present at its connection point (no input selected in NS_C) or if the input signal is failed (CI_SSF active). The defect is raised if CI_SSF = "true" or CI_CS = "none" for at least X seconds. The defect is cleared if CI_SSF = "false" and CI_CS  "none" for at least Y seconds. The values of X and Y are for further study.

Consequent actions:

aTSF		CI_SSF

Defect correlation:

cLTI		dLTI

Performance monitoring: None.

# 7	Network synchronization layer atomic functions

Within this layer, the same connection function is used for one or two independent selection processes, which may have independent inputs:

–	a single selection of an input reference for the NE synchronization distribution;

–	a single or no selection of an input reference for the station clock output.

The use of several independent selection processes for this station clock output is for further study.

These two processes shall work in the same QL mode.

Figure 26 – Network synchronization layer atomic functions

NS layer CP

The CI at this point is a clock signal with associated server signal fail, quality level, and clock-source identifier.

## 7.1	NS_connection functions (NS_C)

Symbol:

Figure 27 – NS_C symbol

Interfaces:

Table 25 – NS_C input and output signals

| Input(s) | Output(s) |
| --- | --- |
| Per input: NS_CI_CK NS_CI_SSF NS_CI_QL NS_CI_CS NS_CI_CLKID NS_CI_mixedChain NS_CI_partialChain NS_CI_numeSECs NS_CI_numSECs Per function: NS_C_MI_QLmode NS_C_MI_OptII_QL-PROV_Priority Per selector: NS_C_MI_WTR NS_C_MI_EXTCMD Per input of a selector: NS_C_MI_priority NS_C_MI_CLR_WTR NS_C_MI_Set_lockout NS_C_MI_Clr_Lockout | Per output: NS_CI_CK NS_CI_QL NS_CI_SSF NS_CI_CS NS_CI_CLKID NS_CI_mixedChain NS_CI_partialChain NS_CI_numeSECs NS_CI_numSECs Per selector: NS_C_MI_SelectedInput NS_C_MI_Reject_Request Per input of a selector: NS_C_MI_State |

Processes:

This function performs one or more independent selection processes. Each selection process selects a synchronization source out of the nominated set of synchronization source inputs determined by the selection algorithm.

The function can operate in QL-enabled or QL-disabled mode, as defined by MI_QLmode.

NOTE 1 – The number of input signals to the connection process and the number of connection processes in the function are not specified in this Recommendation. That is a property of individual network elements. Examples are presented in Appendix II.

Automatic reference selection process:

The function shall perform the automatic reference selection process as defined in clause 5.6 and Annex A.

External commands:

The function shall support the use of external commands as defined in clause 5.11.

Priority:

The function shall support the use of synchronization source priorities as defined in clause 5.10.

Hold-off time:

The function shall support a hold-off timer per input of a selection process (nominated source) as defined in clause 5.8.

Wait-to-restore time:

The function shall support a wait-to-restore timer per input of a selection process (nominated source) as defined in clause 5.9.

Via MI_CLR_WTR the WTR timer can be cleared before the WTR time is expired.

Signal fail extension:

For each input to a selection process, the signal fail information to the selector is a combination (OR function) of the incoming signal fail information (CI_SSF) and the signal fail information delayed by the WTR and hold-off process. Detailed information is provided in Figure A.1.

SF[m] = CI_SF[m] or WTR/HO[CI_SF[m]]

Status report:

The state of each input to a selection process (available, failed, WTR) shall be reported via MI_State.

The actual selected source of a selection process shall be reported via MI_SelectedInput.

Unconnected NS signal generator:

The function shall generate an unconnected NS signal. The unconnected NS signal has an undefined clock, a quality level of QL-UNC, a CS value of "none" and signal fail true.

NOTE 2 – This signal is a logical signal defined for the purpose of this Recommendation; it is not observable as such at any of the network element's interfaces.

Defects:

Quality level degradation defect

If the quality level of the selected source (NS_CI_QL) is degraded below a threshold for at least X seconds, a quality level degradation defect is detected. The defect is cleared if the quality level of the selected source is above the threshold for at least Y seconds. The values of X and Y are for further study.

This defect and the values of X and Y are independent of the hold-off and wait-to-restore timer.

Consequent actions:

If an output of this function is not connected to one of its inputs, the function shall connect the unconnected NS signal to the output.

Defect correlations: None.

Performance monitoring: For further study.

# 8	Transport layer to SD layer atomic functions

## 8.1	STM-n multiplex section adaptation functions

### 8.1.1	STM-N multiplex section to SD adaptation source (MSn/SD_A_So)

Symbol:

Figure 28 – MSn/SD_A_So symbol

Interfaces:

Table 26 – MSn/SD_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_QL SD_CI_CS STMn_TI_CK STMn_TI_FS MSn_RI_CS MSn/SD_A_So_MI_SSMdis MSn/SD_A_So_MI_QLmode MSn/SD_A_So_MI_OptII_QLGEN MSn/SD_A_So_MI_OptII_QLRES | MSn_AI_D |

Processes:

This function converts the CI_QL into the 4-bit SSM code (bits 5 to 8 of byte S1), as defined in [ITU-T G.707].

The SSM message that shall be generated and inserted depends on the applied quality-level indication that is input to the adaptation source function (CI_QL). Tables 27 to 29 present the relationship between the existing set of QLs and the output SSM.

Table 27 – Quality level set coding into synchronization status message (SSM) 
in option I synchronization networks

| Quality level (CI_QL) | SSM coding [MSB..LSB] |
| --- | --- |
| QL-PRC | 0010 |
| QL-SSU-A | 0100 |
| QL-SSU-B | 1000 |
| QL-SEC | 1011 |

Table 28 – Quality level set coding into synchronization status message (SSM)
in option II synchronization networks

| Quality level (CI_QL) | SSM coding [MSB..LSB] MI_OptII_QLGEN=GEN2 | SSM coding [MSB..LSB] MI_OptII_QLGEN=GEN1 |
| --- | --- | --- |
| QL-PRS | 0001 | 0001 |
| QL-STU | 0000 | 0000 |
| QL-ST2 | 0111 | 0111 |
| QL-TNC | 0100 | 1010 (Note) |
| QL-ST3E | 1101 | 1010 (Note) |
| QL-ST3 | 1010 | 1010 |
| QL-SMC | 1100 | 1100 |
| QL-PROV | 1110 | 1110 |
| NOTE – Instead of code "1010", the operator could choose to generate code "1110" (MI_OptII_QLRES is true). Refer to clause 5.5.2.2. |  |  |

Table 29 – Quality level set coding into synchronization status message (SSM) 
in option III synchronization networks

| Quality level (CI_QL) | SSM coding [MSB..LSB] |
| --- | --- |
| QL-UNK | 0000 |
| QL-SEC | 1011 |

QLmode: In the case where the function operates in QL-disabled mode (MI_QLmode = dis), the transmitted SSM code shall be forced to the "1111" pattern.

Timing loop prevention: If RI_CS equals CI_CS, the transmitted SSM shall be forced to the "1111" pattern to prevent a timing loop condition to occur. (See clause 5.13.)

SSM usage: The function supports the capability to prevent synchronization quality information from passing the interface (see clause 5.5.2). In the case where MI_SSMdis is true, the function shall force the SSM to the "1111" pattern.

S1[5-8]: Bits 5 through 8 (bit 5 as MSB) of byte S1 shall transport the 4-bit SSM code.

Defects: None.

Consequent actions:

if (MI_QLmode == dis)

then	S1[5-8] = 1111

else		if (RI_CS == CI_CS) or (SSMdis == true)

then	S1[5-8] = 1111

else		S1[5-8] = SSM[CI_QL]

fi

fi

Defect correlations: None.

Performance monitoring: None.

### 8.1.2	STM-N multiplex section to SD adaptation sink (MSn/SD_A_Sk)

Symbol:

Figure 29 – MSn/SD_A_Sk symbol

Interfaces:

Table 30 – MSn/SD_A_Sk input and output signals

| Input(s) | Output(s) |
| --- | --- |
| MSn_AI_D MSn_AI_CK MSn_AI_FS MSn_AI_TSF MSn/SD_A_Sk_MI_SSMsupp MSn/SD_A_Sk_MI_CSid MSn/SD_A_Sk_MI_QLmode | SD_CI_CK SD_CI_SSF SD_CI_CS SD_CI_QL MSn_RI_CS |

Processes:

This function extracts and accepts the 4-bit synchronization status message (SSM), transmitted via bits 5 to 8 of byte S1 as defined in [ITU-T G.707]. It supplies the timing signal, recovered by the physical section layer, to the synchronization distribution layer.

S1[5-8]: In QL-enabled mode, and if SSMsupp is true, bits 5 to 8 of byte S1 shall be recovered.

For options I and III, the recovered bits 5 to 8 of byte S1 shall be accepted if the same code is present in three consecutive frames.

For option II, a received SSM code is accepted after receiving, in at least 8 consecutive samples (not necessarily consecutive frames), an identical (new) value. The sampling rate must be such that a new value can be validated within 1 second (assuming no bit errors). If no validated SSM is detected for a period greater than 10 seconds, the value "1111" shall be assumed. At least once per 10 seconds, the evaluation of the received SSM shall be restarted.

The accepted code shall be converted to a quality level QL[SSM] as specified in Table 8 (option I), Table 10 (option II), Table 12 (option III) and output via CI_QL.

QLmode: In the case where the function operates in QL-disabled mode (MI_QLmode = dis), the received SSM code shall be ignored and the CI_QL shall be forced to the QL-NSUPP.

SSM support: In the case where MI_SSMsupp is false, the received SSM bit in the S1 byte should not be interpreted as a valid QL value and the CI_QL shall be forced to the QL-NSUPP.

Clock source identifier: The function shall insert the clock-source identifier received via MI_CSid into CI_CS and RI_CS to support timing loop prevention as described in clause 5.13.

Defects: None.

Consequent actions:

aSSF		AI_TSF

if (MI_QLmode == disabled) or (MI_SSMsupp == false)

then	CI_QL = QL-NSUPP

else		CI_QL = QL[SSM]

fi

Defect correlations: None.

Performance monitoring: None.

## 8.2	Pqs adaptation functions

### 8.2.1	Pqs to SD adaptation source (Pqs/SD_A_So)

Symbol:

Figure 30 – Pqs/SD_A_So symbol

Interfaces:

Table 31 – Pqs/SD_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_QL SD_CI_CS Pqs_TI_CK Pqs_TI_FS Pqs_TI_MFS Pqs_RI_CS Pqs/SD_A_So_MI_TMmode Pqs/SD_A_So_MI_SSMdis Pqs/SD_A_So_MI_QLmode | Pqs_AI_D |

Processes:

This function converts the CI_QL and CI_SSF information into the 4-bit SSM code (multiframed bit 8 of byte MA), or into the 1-bit TM code, as defined in [ITU-T G.832]. This is controlled by MI_TMmode.

TMmode: In the case where TMmode is disabled, the function shall generate the SSM code. In the case where TMmode is enabled, the function shall generate the TM code.

MA[6-7]: If TMmode is disabled, the value of the multiframe indicator bits shall be set as specified by [ITU-T G.832], 500 s TU multiframe sequence, and aligned with Pqs_TI_MFS. Such a multiframe indicator is available for SSM processing. If TMmode is enabled, the multiframe indicator is not required for this mode of operation.

|  |  |  |  |  | Multiframe indicator |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |

Figure 31 – Multiframe indicator bits in byte MA

The SSM or TM message that shall be generated and inserted depends on the applied quality-level indication that is input to the adaptation source function (CI_QL). Table 32 presents the relation between the existing set of QLs and the output SSM and TM codes.

NOTE – There may be another parallel adaptation function, e.g., Pqs/TUG_A_So that also generates a multiframe sequence. The equipment should take care that the multiframe sequences generated are the same from all adaptation functions involved.

MA[8], MA[8][1-4]: In the case where TMmode is disabled, bit 8 of byte MA in a four-frame multiframe (first frame as MSB) shall transport the 4-bit SSM code. In the case where TMmode is enabled, bit 8 of byte MA shall transport the 1-bit TM code.

Table 32 – Quality level set coding into synchronization status message and timing marker

| Quality level (CI_QL) | SSM coding [MSB..LSB] | TM coding |
| --- | --- | --- |
| QL-PRC | 0010 | 0 |
| QL-SSU-A | 0100 | 1 |
| QL-SSU-B | 1000 | 1 |
| QL-SEC | 1011 | 1 |

QLmode: In the case where the function operates in QL-disabled mode (MI_QLmode = dis), the transmitted SSM code shall be forced to the "1111" pattern, while the transmitted TM code shall be forced to the "1" pattern.

Timing loop prevention: If RI_CS equals CI_CS, the transmitted SSM[TM] shall be forced to the "1111" ["1"] pattern to prevent a timing loop condition from occurring. See clause 5.13.

SSM/TM usage: The function supports the capability to prevent synchronization quality information from passing the interface (see clause 5.5.2). In the case where MI_SSMdis is true, the function shall force the SSM[TM] to the "1111" ["1"] pattern.

Defects: None.

Consequent actions:

if (MI_TMmode == dis)

then	if (MI_QLmode == dis)

then	MA[8][1-4] = 1111

else	if (RI_CS == CI_CS) or (SSMdis == true)

then	MA[8][1-4] = 1111

else	MA[8][1-4] = SSM[CI_QL]

fi

fi

else		if (MI_QLmode == dis)

then	MA[8] = 1

else	if (RI_CS == CI_CS) or (SSMdis == true)

then	MA[8] = 1

else	MA[8] = TM[CI_QL]

fi

fi

fi

Defect correlations: None.

Performance monitoring: None.

### 8.2.2	Pqs to SD adaptation sink (Pqs/SD_A_Sk)

Symbol:

Figure 32 – Pqs/SD_A_Sk symbol

Interfaces:

Table 33 – Pqs/SD_A_Sk input and output signals

| Input(s) | Output(s) |
| --- | --- |
| Pqs_AI_D Pqs_AI_CK Pqs_AI_FS Pqs_AI_TSF Pqs/SD_A_Sk_MI_TMmode Pqs/SD_A_Sk_MI_QLmode Pqs/SD_A_Sk_MI_SSMsupp Pqs/SD_A_Sk_MI_CSid | SD_CI_CK SD_CI_SSF SD_CI_CS SD_CI_QL Pqs_RI_CS Pqs/SD_A_Sk_MI_cLOM |

Processes:

This function extracts and accepts the 4-bit synchronization status message (SSM), transmitted via the multiframed bit 8 of byte MA, or the 1-bit timing marker (TM), transmitted via bit 8 of byte MA as defined in [ITU-T G.832]. It supplies the timing signal, recovered by the physical section layer, to the synchronization distribution layer.

TMmode: In the case where TMmode is disabled, the function shall interpret bit 8 of byte MA as the SSM code. In the case where TMmode is enabled, the function shall interpret bit 8 of byte MA as the TM code.

MA[6-7]: In QL-enabled mode, and if TMmode is disabled and if SSMsupp is true, the function shall recover the 500 s (multi)frame start phase performing multi-frame alignment on bits 6 and 7 of byte MA. Out-of-multiframe (OOM) shall be assumed once an error is detected in the MA bit 6 and 7 sequence. Multiframe alignment shall be assumed to be recovered, and the in-multiframe (IM) state shall be entered, when an error-free MA sequence is found in four consecutive Pqs frames.

MA[8][1-4]: In QL-enabled mode, and if TMmode is disabled and SSMsupp is true, bit 8 of byte MA in a four frame multiframe (first frame as MSB) shall be recovered and accepted if the same code is present in three consecutive 4-bit multiframes. The accepted code shall be converted to a quality level QL[SSM] as specified in Table 8 (option I) and output via CI_QL.

MA[8]: In QL-enabled mode, and if TMmode is enabled and SSMsupp is true, bit 8 of byte MA shall be recovered and accepted if the same code is present in three consecutive frames. The accepted code shall be converted to a quality level QL[TM] as specified in Table 9 and output via CI_QL.

QLmode: In the case where the function operates in QL-disabled mode (MI_QLmode = dis), the received SSM or TM code shall be ignored and the CI_QL shall be forced to the QL-NSUPP.

SSM/TM support: In the case where MI_SSMsupp is false, the received SSM or TM code shall be ignored and the CI_QL shall be forced to the QL-NSUPP.

Clock source identifier: The function shall insert the clock-source identifier received via MI_CSid into CI_CS and RI_CS to support timing loop prevention as described in clause 5.13.

Defects:

If the multiframe alignment process is in the OOM state, and the MA[6-7] multiframe is not recovered within X ms, a dLOM defect shall be declared. Once in a dLOM state, this state shall be exited when the multiframe is recovered (multiframe alignment process enters the IM state). X shall be a value in the range 1 ms to 5 ms. X is not configurable. dLOM shall be cleared when QLmode is disabled, or SSMsupp is false, or TMmode is enabled.

Consequent actions:

aSSF		dLOM or AI_TSF

if (MI_QLmode == disabled) or (MI_SSMsupp == false)

then	CI_QL = QL-NSUPP

else		if (MI_TMmode == disabled)

then	CI_QL = QL[SSM]

else	CI_QL = QL[TM]

fi

fi

Defect correlations:

cLOM		dMFP and (not AI_TSF)

NOTE – There may be another parallel adaptation function, e.g., Pqs/TUG_A_sk that also generates cLOM. The EMF should take care that fLOM is reported only once.

Performance monitoring: None.

## 8.3	P12s layer adaptation functions

### 8.3.1	P12s layer adaptation source functions

Two types of P12s/SD_A_So functions are defined:

–	type 1 for a 2 Mbit/s station clock output supporting SSM: P12s/SD-sc-1_A_So;

–	type 2 for a 2 Mbit/s station clock output not supporting SSM: P12s/SD-sc-2_A_So.

Other types are for further study.

#### 8.3.1.1	Type 1 P12s to SD adaptation source for station clock output supporting SSM (P12s/SD-sc-1_A_So)

Symbol:

Figure 33 – P12s/SD-sc-1_A_So symbol

Interfaces:

Table 34 – P12s/SD-sc-1_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_QL SD_CI_CS SD_CI_SSF P12s_TI_CK P12s_TI_FS P12s_TI_MFS P12s/SD-sc-1_A_So_MI_SelSaSSM P12s/SD-sc-1_A_So_MI_QLmode P12s/SD-sc-1_A_So_MI_SSMsupp P12s/SD-sc-1_A_So_MI_QLminimum | P12s_AI_D P12s_AI_CK P12s_AI_FS P12s_AI_MFS P12s_AI_AISinsert P12s_RI_CS |

Processes:

This function converts the CI_QL and CI_SSF information into the 4-bit SSM code transmitted in one of the five Sa bits, as defined in [ITU-T G.704], and an AISinsert control signal.

The SSM message that shall be generated and inserted depends on the applied quality-level indication that is input to the adaptation source function (CI_QL). Table 35 presents the relationship between the existing set of QLs and the output SSM codes.

Table 35 – Quality level set coding into synchronization status message in option I synchronization network

| Quality level (CI_QL) | SSM coding [MSB..LSB] |
| --- | --- |
| QL-PRC | 0010 |
| QL-SSU-A | 0100 |
| QL-SSU-B | 1000 |
| QL-SEC | 1011 |
| QL-UNC | 1111 |

QLmode: In the case where the function operates in QL-disabled mode (MI_QLmode = dis), the transmitted SSM code shall be forced to the "1111" pattern and AI_AISinsert shall be used to signal that no synchronization source is available.

Sax: The 4-bit SSM code shall be inserted in one of the Sa bits (Sax, x = 4 to 8) as selected via MI_SelSaSSM. The 4-bit SSM code shall be transported in alignment with the CRC-4 submultiframe.

Interworking: For interworking with old equipment not supporting SSM processing, AIS insertion can be used instead of SSM insertion to pass synchronization quality information via the interface. In the case where MI_SSMsupp is false, the function shall force the SSM to the "1111" pattern and AI_AISinsert shall be used to signal that no synchronization source is available or CI_QL is below MI_QLminimum.

Clock source identifier and quality level: The function shall insert the clock-source identifier received via CI_CS into RI_CS to support timing loop prevention as described under consequent actions (see clause 5.13).

Defects: None.

Consequent actions:

if (MI_QLmode == dis)

then	Sax[1-4] = 1111

if (CI_SSF == true)

then	AI_AISinsert = true

RI_CS = none

else		AI_AISinsert = false

RI_CS = CI_CS

fi

else		if (MI_SSMsupp == false)

then	Sax[1-4] = 1111

if (CI_SSF == true) or (CI_QL<MI_QLminimum)

then	AI_AISinsert = true

RI_CS = none

else	AI_AISinsert = false

RI_CS = CI_CS

fi

else		AI_AISinsert = false

if (CI_SSF == true)

then	Sax[1-4] = 1111

RI_CS = none

else	Sax[1-4] = SSM[CI_QL]

RI_CS = CI_CS

fi

fi

fi

Defect correlations: None.

Performance monitoring: None.

#### 8.3.1.2	Type 2 P12s to SD adaptation source for station clock output port not supporting SSM (P12s/SD-sc-2_A_So)

Symbol:

Figure 34 – P12s/SD-sc-2_A_So symbol

Interfaces:

Table 36 – P12s/SD-sc-2_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_QL SD_CI_CS SD_CI_SSF P12s_TI_CK P12s_TI_FS P12s_TI_MFS P12s/SD-sc-2_A_So_MI_QLminimum P12s/SD-sc-2_A_So_MI_QLmode | P12s_AI_CK P12s_AI_FS P12s_AI_MFS P12s_AI_AISinsert P12s_RI_CS |

Processes:

This function converts the CI_QL and CI_SSF information into an AISinsert control signal.

QLmode: In the case where the function operates in QL-disabled mode (MI_QLmode = dis), AI_AISinsert shall be activated if CI_SSF is true. In the case of QL-enabled mode, AI_AISinsert shall be activated if CI_SSF is true or CI_QL is below MI_QLminimum.

Clock source identifier and quality level: The function shall insert the clock-source identifier received via CI_CS into RI_CS to support timing loop prevention as described under consequent actions (see also clause 5.13).

Defects: None.

Consequent actions:

if (MI_QLmode == dis)

then	if (CI_SSF == true)

then	AI_AISinsert = true

RI_CS = none

else		AI_AISinsert = false

RI_CS = CI_CS

fi

else		if (CI_SSF == true) or (CI_QL < MI_QLminimum)

then	AI_AISinsert = true

RI_CS = none

else		AI_AISinsert = false

RI_CS = CI_CS

fi

fi

Defect correlations: None.

Performance monitoring: None.

### 8.3.2	P12s layer adaptation sink functions

Two types of P12s/SD_A_Sk functions are defined:

–	type 1 for a 2 Mbit/s traffic input port: P12s/SD-tf_A_Sk;

–	type 2 for a 2 Mbit/s station clock input port: P12s/SD-sc_A_Sk.

#### 8.3.2.1	Type 1 P12s to SD adaptation sink for traffic input port (P12s/SD-tf_A_Sk)

Symbol:

Figure 35 – P12s/SD-tf_A_Sk symbol

Interfaces:

Table 37 – P12s/SD-tf_A_Sk input and output signals

| Input(s) | Output(s) |
| --- | --- |
| P12s_AI_D P12s_AI_CK P12s_AI_FS P12s_AI_TSF P12s_AI_MFS P12s_AI_MFP P12s/SD-tf_A_Sk_MI_SSMsupp P12s/SD-tf_A_Sk_MI_SelSaSSM P12s/SD-tf_A_Sk_MI_QLmode P12s/SD-tf_A_Sk_MI_CSid | SD_CI_CK SD_CI_SSF SD_CI_CS SD_CI_QL P12s_RI_CS (for further study) |

Processes:

This function extracts and accepts the 4-bit synchronization status message (SSM), transmitted via one of the Sa bits as defined in [ITU-T G.704]. It supplies the timing signal, recovered by the physical section layer, to the synchronization distribution layer.

Sax[1-4]: In QL-enabled mode, and if SSMsupp is true, bits Sax[1] to Sax[4] (x = MI_SelSaSSM is a value in the set [4, 5, 6, 7, 8]) shall be recovered and accepted if the same code is present in three consecutive frames. The accepted code shall be converted to a quality level QL[SSM] as specified in Table 8 (option I) and output via CI_QL.

QLmode: In the case where the function operates in QL-disabled mode (MI_QLmode = dis), the received SSM code shall be ignored and the CI_QL shall be forced to the QL-NSUPP.

SSM support: In the case where MI_SSMsupp is false, the received SSM code shall be ignored and the CI_QL shall be forced to the QL-NSUPP.

Clock source identifier: The function shall insert the clock-source identifier received via MI_CSid into CI_CS to support timing loop prevention as described in clause 5.13. RI_CS generation is for further study.

Defects: None.

Consequent actions:

aSSF		AI_TSF or (AI_ MFP==false and QLmode==enabled and SSMsupp==true)

if (MI_QLmode == disabled) or (MI_SSMsupp == false)

then	CI_QL = QL-NSUPP

else 	CI_QL = QL[SSM]

fi

Defect correlations: None

Performance monitoring: None.

#### 8.3.2.2	Type 2 P12s to SD adaptation sink for station clock input port (P12s/SD-sc_A_Sk)

Symbol:

Figure 36 – P12s/SD-sc_A_Sk symbol

Interfaces:

Table 38 – P12s/SD-sc_A_Sk input and output signals

| Input(s) | Output(s) |
| --- | --- |
| P12s_AI_D P12s_AI_CK P12s_AI_FS P12s_AI_TSF P12s_AI_MFS P12s_AI_MFP P12s_RI_CS P12s/SD-sc_A_Sk_MI_SSMsupp P12s/SD-sc_A_Sk_MI_SelSaSSM P12s/SD-sc_A_Sk_MI_CSid P12s/SD-sc_A_Sk_MI_QLmode | SD_CI_CK SD_CI_SSF SD_CI_CS SD_CI_QL |

Processes:

This function extracts and accepts the 4-bit synchronization status message (SSM), transmitted via one of the Sa bits as defined in [ITU-T G.704]. It supplies the timing signal, recovered by the physical section layer, to the synchronization distribution layer.

Sax: In QL-enabled mode, and if SSMsupp is true, bits Sax[1] to Sax[4] (x = MI_SelSaSSM is a value in the set [4, 5, 6, 7, 8]) shall be recovered and accepted if the same code is present in three consecutive frames. The accepted code shall be converted to a quality level QL[SSM] as specified in Table 8 and output via CI_QL.

QLmode: In the case where the function operates in QL-disabled mode (MI_QLmode = dis), the received SSM code shall be ignored and the CI_QL shall be forced to the QL-NSUPP.

SSM support: In the case where MI_SSMsupp is false, the received SSM code shall be ignored and the CI_QL shall be forced to the QL-NSUPP.

Clock source identifier: The function shall process the clock-source identifier received via RI_CS to support timing loop prevention (see clause 5.13). The function shall determine the value of the CI_CS output signal as follows:

if (RI_CS == none)

then	CI_CS = MI_CSid

else		CI_CS = RI_CS

fi

Defects: None.

Consequent actions:

aSSF		AI_TSF or (AI_MFP==false and QLmode==enabled and SSMsupp==true)

if (MI_QLmode == disabled) or (MI_SSMsupp == false)

then	CI_QL = QL-NSUPP

else		CI_QL = QL[SSM]

fi

Defect correlations: None

Performance monitoring: None.

## 8.4	T12 layer adaptation functions

### 8.4.1	T12 to SD adaptation source (T12/SD_A_So)

Symbol:

Figure 37 – T12/SD_A_So symbol

Interfaces:

Table 39 – T12/SD_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_QL SD_CI_CS SD_CI_SSF T12_TI_CK T12/SD_A_So_MI_QLminimum T12/SD_A_So_MI_QLmode | T12_AI_CK T12_AI_SQLCH T12_RI_CS |

Processes:

This function converts the CI_QL and CI_SSF information into an SQLCH control signal.

QLmode: In the case where the function operates in QL-disabled mode (MI_QLmode = dis), AI_SQLCH shall be activated if CI_SSF is true. In the case of QL-enabled mode, AI_SQLCH shall be activated if CI_SSF is true or CI_QL is below MI_QLminimum.

Clock source identifier and quality level: The function shall insert the clock-source identifier received via CI_CS into RI_CS to support timing loop prevention as described under consequent actions (see clause 5.13).

Defects: None.

Consequent actions:

if (MI_QLmode == dis)

then	if (CI_SSF == true)

then 	AI_SQLCH = true

RI_CS = none

else		AI_SQLCH = false

RI_CS = CI_CS

fi

else		if (CI_SSF == true) or (CI_QL < MI_QLminimum)

then	AI_SQLCH = true

RI_CS = none

else		AI_SQLCH = false

RI_CS = CI_CS

fi

fi

Defect correlations: None.

Performance monitoring: None.

### 8.4.2	T12 to SD adaptation sink (T12/SD_A_Sk)

Symbol:

Figure 38 – T12/SD_A_Sk symbol

Interfaces:

Table 40 – T12/SD_A_Sk input and output signals

| Input(s) | Output(s) |
| --- | --- |
| T12_AI_CK T12_AI_TSF T12_RI_CS T12/SD_A_Sk_MI_Csid | SD_CI_CK SD_CI_SSF SD_CI_CS SD_CI_QL |

Processes:

This function adapts the 2048 kHz timing information from an external reference to equipment-specific timing characteristic information. This function regenerates the received clock signal and supplies the recovered timing signal to the synchronization distribution layer.

Regeneration: The function shall output a valid clock signal when any combination of the following signal conditions exists at the input:

–	an input electrical amplitude level with any value in the range specified by [ITU-T G.703];

–	jitter modulation applied to the input signal with any value defined in [ITU-T G.813] or [ITU-T G.8262];

–	the input signal frequency has any value in the range 2048 kHz  50 ppm.

NOTE – The frequency and jitter/wander tolerance is further constrained by the requirements of the client (SD) layer. For example, in the MS SDH layer, the frequency offset should not exceed 4.6 ppm.

SSM support: CI_QL shall be forced to the QL-NSUPP.

Clock source identifier: The function shall process the clock-source identifier received via RI_CS to support timing loop prevention (see clause 5.13). The function shall determine the value of the CI_CS output signal as follows:

if (RI_CS == none)

then	CI_CS = MI_CSid

else		CI_CS = RI_CS

fi

Defects: None.

Consequent actions:

aSSF		AI_TSF

Defect correlations: None

Performance monitoring: None.

## 8.5	P11s layer adaptation functions

### 8.5.1	P11s to SD adaptation source (P11s/SD_A_So)

For further study.

### 8.5.2	P11s to SD adaptation sink (P11s/SD_A_Sk)

For further study.

## 8.6	T01 layer adaptation functions

### 8.6.1	T01 to SD adaptation source (T01/SD_A_So)

Symbol:

Figure 39 – T01/SD_A_So symbol

Interfaces:

Table 41 – T01/SD_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_QL SD_CI_CS SD_CI_SSF T01_TI_CK T01/SD_A_So_MI_QLminimum T01/SD_A_So_MI_QLmode | T01_AI_CK T01_AI_SQLCH T01_RI_CS |

Processes:

This function converts the CI_QL and CI_SSF information into an SQLCH control signal.

QLmode: In the case where the function operates in QL-disabled mode (MI_QLmode = dis), AI_SQLCH shall be activated if CI_SSF is true. In the case of QL-enabled mode, AI_SQLCH shall be activated if CI_SSF is true or CI_QL is below MI_QLminimum.

Clock source identifier and quality level: The function shall insert the clock-source identifier received via CI_CS into RI_CS to support timing loop prevention as described under consequent actions (see clause 5.13).

Defects: None.

Consequent actions:

if (MI_QLmode == dis)

then	if (CI_SSF == true)

then 	AI_SQLCH = true

RI_CS = none

else		AI_SQLCH = false

RI_CS = CI_CS

fi

else		if (CI_SSF == true) or (CI_QL < MI_QLminimum)

then	AI_SQLCH = true

RI_CS = none

else		AI_SQLCH = false

RI_CS = CI_CS

fi

fi

Defect correlations: None.

Performance monitoring: None.

### 8.6.2	T01 to SD adaptation sink (T01/SD_A_Sk)

Symbol:

Figure 40 – T01/SD_A_Sk symbol

Interfaces:

Table 42 – T01/SD_A_Sk input and output signals

| Input(s) | Output(s) |
| --- | --- |
| T01_AI_CK T01_AI_TSF T01_RI_CS T01/SD_A_Sk_MI_Csid | SD_CI_CK SD_CI_SSF SD_CI_CS SD_CI_QL |

Processes:

This function adapts the 64 kHz T01 timing information from an external reference to equipment-specific timing characteristic information. This function regenerates the received clock signal and supplies the recovered timing signal to the synchronization distribution layer.

Regeneration: The function shall output a valid clock signal when any combination of the following signal conditions exists at the input:

–	an input electrical amplitude level with any value in the range specified by [ITU-T G.703];

–	jitter modulation applied to the input signal with any value defined in [ITU-T G.813] or [ITU-T G.8262];

–	the input signal frequency has any value in the range 64 kHz  TBD ppm.

NOTE – The frequency and jitter/wander tolerance is further constrained by the requirements of the client (SD) layer.

SSM support: CI_QL shall be forced to the QL-UNK.

Clock source identifier: The function shall process the clock-source identifier received via RI_CS to support timing loop prevention (see clause 5.13). The function shall determine the value of the CI_CS output signal as follows:

if (RI_CS == none)

then	CI_CS = MI_CSid

else		CI_CS = RI_CS

fi

Defects: None.

Consequent actions:

aSSF		AI_TSF

Defect correlations: None

Performance monitoring: None.

## 8.7	T02 layer adaptation functions

### 8.7.1	T02 to SD adaptation source (T02/SD_A_So)

For further study.

### 8.7.2	T02 to SD adaptation sink (T02/SD_A_Sk)

Symbol:

Figure 41 – T02/SD_A_Sk symbol

Interfaces:

Table 43 – T02/SD_A_Sk input and output signals

| Input(s) | Output(s) |
| --- | --- |
| T02_AI_CK T02_AI_TSF T02/SD_A_Sk_MI_CSid | SD_CI_CK SD_CI_SSF SD_CI_CS SD_CI_QL |

Processes:

This function adapts the 64 kHz T02 timing information from an external reference to equipment-specific timing characteristic information. This function regenerates the received clock signal and supplies the recovered timing signal to the synchronization distribution layer.

Regeneration: The function shall output a valid clock signal when any combination of the following signal conditions exists at the input:

–	an input electrical amplitude level with any value in the range specified by [ITU-T G.703];

–	jitter modulation applied to the input signal with value TBD;

–	the input signal frequency has any value in the range 64 kHz  TBD ppm.

NOTE – The frequency and jitter/wander tolerance is further constrained by the requirements of the client (SD) layer.

SSM support: CI_QL shall be forced to the QL-UNK.

Clock source identifier: The function shall assign MI_CSid to CI_CS.

Defects: None.

Consequent actions:

aSSF		AI_TSF

Defect correlations: None

Performance monitoring: None.

## 8.8	T21 layer adaptation functions

### 8.8.1	T21 to SD adaptation source (T21/SD_A_So)

Symbol:

Figure 42 – T21/SD_A_So symbol

Interfaces:

Table 44 – T21/SD_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_QL SD_CI_CS SD_CI_SSF T21_TI_CK T21/SD_A_So_MI_QLminimum T21/SD_A_So_MI_QLmode | T21_AI_CK T21_AI_SQLCH |

Processes:

This function converts the CI_QL and CI_SSF information into an SQLCH control signal.

QLmode: In the case where the function operates in QL-disabled mode (MI_QLmode = dis), AI_SQLCH shall be activated if CI_SSF is true. In the case of QL-enabled mode, AI_SQLCH shall be activated if CI_SSF is true or CI_QL is below MI_QLminimum.

Defects: None.

Consequent actions:

if (MI_QLmode == dis)

then	if (CI_SSF == true)

then 	AI_SQLCH = true

else		AI_SQLCH = false

fi

else		if (CI_SSF == true) or (CI_QL < MI_QLminimum)

then	AI_SQLCH = true

else		AI_SQLCH = false

fi

fi

Defect correlations: None.

Performance monitoring: None.

### 8.8.2	T21 to SD adaptation sink (T21/SD_A_Sk)

For further study.

## 8.9	ETH layer adaptation functions

### 8.9.1	ETH to SD adaptation source (ETH/SD_A_So)

Symbol:

Figure 43 – ETH/SD_A_So symbol

Interfaces:

Table 45 – ETH/SD_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_QL SD_CI_CS SD_CI_CLKID SD_CI_mixedChain SD_CI_partialChain SD_CI_numeSECs SD_CI_numSECs ETH_RI_CS ETH/SD_A_So_MI_SSMdis ETH/SD_A_So_MI_OptII_QLGEN ETH/SD_A_So_MI_OptII_QLRES | ETH_CI_ESMC |

Processes:

eQL-TLV not supported:

If the eQL-TLV is not supported, this function converts the CI_QL into the 4-bit SSM code, as defined in 5.5, and inserts it in the SSM field in the CI_ESMC, i.e., in the ESMC PDU (see clause 11.3 and its subclauses of [ITU-T G.8264]). The ESMC carries event and information messages, as defined in [ITU-T G.8264].

The SSM message (i.e., ESMC PDU) that shall be generated and inserted depends on the applied quality-level indication that is input to the adaptation source function (CI_QL). Tables 27 to 29 present the relationship between the existing set of QLs and the output SSM.

eQL-TLV supported:

If the eQL-TLV is supported, this function converts the CI_QL into the 4-bit SSM code and the 8-bit eSSM code, as defined in [ITU-T G.8264]. The 4-bit SSM code is inserted in the SSM field in the CI_ESMC, and the 8-bit eSSM code is inserted in the eSSM code field in the CI _ESMC , i.e., in the ESMC PDU (see clause 11.3 and its subclauses of [ITU-T G.8264]). The ESMC carries event and information messages, as defined in [ITU-T G.8264].

The SSM message (i.e., ESMC PDU) that shall be generated and inserted depends on the applied quality-level indication that is input to the adaptation source function (CI_QL). Tables 11-7 and 11-8 of [ITU-T G.8264] present the relationship between the existing set of QLs and the output SSM and eSSM.

The CI_CLKID is copied to the SyncE clockIdentity of the originator of the eQL-TLV field of the eQL-TLV (see Table 11-5 of [ITU-T G.8264]).

The CI_mixedChain is copied to bit 0 of the Flag field of the eQL-TLV.

The CI_partialChain is copied to bit 1 of the Flag field of the eQL-TLV.

The CI_numeSECs is copied to the Number of cascaded eSECs from the nearest SSU/PRC field of the eQL-TLV.

The CI_numSECs is copied to the Number of cascaded SECs from the nearest SSU/PRC field of the eQL-TLV.

QLmode: This function operates in the QL-enabled mode.

NOTE – The use of QL-disabled mode for synchronous Ethernet is for further study. It is an operator choice to utilize a synchronous Ethernet interface as a candidate synchronization interface in the absence of an ESMC and QL value. Such usage is under the responsibility of the operator (see [ITU-T G.8264]).

Timing loop prevention: If RI_CS equals CI_CS, the transmitted SSM shall be forced to the "1111" pattern to prevent a timing loop condition to occur (see clause 5.13).

SSM usage: The function supports the capability to prevent synchronization quality information to pass the interface (see clause 5.5.2). In the case where MI_SSMdis is true, no ESMC is activated, i.e., neither event nor information ESMC PDU messages are generated.

SSM: The SSM field in the SSM PDU shall transport the 4-bit SSM code.

The ESMC message channel, specified in [ITU-T G.8264], defines 2 types of messages, event and information ones. The generation of an event-based message is triggered by a change of the SSM code, SD_CI_QL value, within the delay times (THM, TNSM, TSM) specified in clause 5.14. The information messages are generated with a periodicity of one second.

Defects: None.

Consequent actions:

if (MI_SSMdis == true)

then no message is generated

else

if (RI_CS == CI_CS)

then SSM = 1111

else 	SSM = SSM[CI_QL]

fi

fi

Defect correlations: None.

Performance monitoring: None.

### 8.9.2	ETH to SD adaptation sink (ETH/SD_A_Sk)

Symbol:

Figure 44 – ETH/SD_A_Sk symbol

Interfaces:

Table 46 – ETH/SD_A_Sk input and output signals

| Input(s) | Output(s) |
| --- | --- |
| ETH_CI_ESMC ETH_CI_CK ETH_CI_SSF ETH_AI_TSF ETH/SD_A_Sk_MI_CSid | SD_CI_CK SD_CI_SSF SD_CI_CS SD_CI_QL SD_CI_CLKID SD_CI_mixedChain SD_CI_partialChain SD_CI_numeSECs SD_CI_numSECs ETH_RI_CS |

Processes:

This function extracts and accepts the 4-bit synchronization status messages (SSMs) that are transmitted in the SSM event-based and information messages carried by the ESMC, as defined in [ITU-T G.8264]. If the eQL-TLV is supported and is present, this function extracts and accepts the 8-bit eSSM, SyncE clockIdentity of the originator of the eQL-TLV, Flag, Number of cascaded eSECs from the nearest SSU/PRC, and Number of cascaded SECs from the nearest SSU/PRC.

SSM: For all options, the recovered SSM bits and, if present, eSSM bits are accepted immediately if no CI_SSF or AI_TSF condition applies.

If the 8-bit eSSM is not present, the eQL-TLV is not supported, or the network is Option III, the accepted code shall be converted to a quality level QL[SSM] as specified in Table 8 (option I), Table 10 (option II), or Table 12 (option III) and output via CI_QL.

If the 8-bit eSSM is present, the eQL-TLV is supported, and the network is Option I or Option II, the accepted code shall be converted to a quality level QL[SSM] as specified in Table 11-7 and Table 11-8 of [ITU-T G.8264]. If the SSM code is valid and the eSSM code is not valid, QL-INV shall be generated. [ITU-T G.8264] indicates that SSM codes and eSSM codes for synchronous Ethernet for Option III are for further study. The SD_CI_QL output is generated with the SSM extracted from both event-based and information messages.

QLmode: This function operates in the QL-enabled mode.

NOTE – The use of QL-disabled mode for synchronous Ethernet is for further study. It is an operator choice to utilize a synchronous Ethernet interface as a candidate synchronization interface in the absence of an ESMC and QL value. Such usage is under the responsibility of the operator (see [ITU-T G.8264]).

SSM support: SSMs are supported.

Clock source identifier: The function shall insert the clock-source identifier received via MI_CSid into CI_CS and RI_CS to support timing loop prevention as described in clause 5.13.

SyncE clockIdentity: The SyncE clockIdentity of the eQL-TLV (see Table 11-5 of [ITU-T G.8264]), if present and supported, is copied to CI_CLKID.

Flag field: Bits 0 and 1 of the Flag field of the eQL-TLV, if present and supported, are copied to CI_mixedChain and CI_partialChain, respectively.

Number of cascaded eSECs from the nearest SSU/PRC: The Number of cascaded eSECs from the nearest SSU/PRC field of the eQL-TLV, if present and supported, is copied to CI_numeSECs.

Number of cascaded SECs from the nearest SSU/PRC: The Number of cascaded SECs from the nearest SSU/PRC field of the eQL-TLV, if present and supported, is copied to CI_numSECs.

Defects:

dLOESMC: If no valid ESMC PDU is received during 5 seconds, a loss of ESMC (LOESMC) defect is detected. The LOESMC defect is cleared on receipt of the first ESMC PDU.

Consequent actions:

aSSF		CI_SSF or AI_TSF or dLOESMC

CI_QL = QL[SSM]

Defect correlations:

cLOESMC		dLOESMC and (not CI_SSF)

Performance monitoring: None.

## 8.10	OTUk layer adaptation functions

### 8.10.1	OTUk to SD adaptation source (OTUk/SD_A_So)

NOTE – OTN interfaces with vendor specific application identifiers may support an OSMC function. The encapsulation of the messages and overhead location are then vendor specific.

Symbol:

Figure 45 – OTUk/SD_A_So symbol

Interfaces:

Table 47 – OTUk/SD_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_QL SD_CI_CS SD_CI_CLKID SD_CI_mixedChain SD_CI_partialChain SD_CI_numeSECs SD_CI_numSECs OTUk_AI_CK OTUk_AI_FS OTUk_RI_CS OTUk/SD_A_So_MI_SSMdis OTUk/SD_A_So_MI_OptII_QLGEN OTUk/SD_A_So_MI_OptII_QLRES | OTUk_AI_D |

Processes:

eQL-TLV not supported:

If the eQL-TLV is not supported, this function converts the CI_QL into the 4-bit SSM code, as defined in 5.5, and inserts it in the SSM field in the GFP-F(SSM) frame transported in the OSMC. The OSMC carries event and information messages, as defined in [ITU-T G.709] and clause 7.11 of [ITU-T G.7041].

The SSM message that shall be generated and inserted depends on the applied quality-level indication that is input to the adaptation source function (CI_QL). Tables 27 to 29 present the relationship between the existing set of QLs and the output SSM.

eQL-TLV supported:

If the eQL-TLV is supported, this function converts the CI_QL into the 4-bit SSM code and the 8-bit eSSM code, as defined in [ITU-T G.8264]. The 4-bit SSM code is inserted in the SSM field in the GFP-F(SSM) frame transported in the OSMC, and the 8-bit eSSM code is inserted in eSSM field of eQL-TLV of the GFP-F(SSM) frame transported in the OSMC. The OSMC carries event and information messages, as defined in [ITU-T G.709] and clause 7.11 of [ITU-T G.7041].

The SSM message that shall be generated and inserted depends on the applied quality-level indication that is input to the adaptation source function (CI_QL). Tables 11-7 and 11-8 of [ITU-T G.8264] present the relationship between the existing set of QLs and the output SSM and eSSM.

The CI_CLKID is copied to the SyncE clockIdentity of the originator of the eQL-TLV field of the eQL-TLV (see Table 11-5 of [ITU-T G.8264]).

The CI_mixedChain is copied to bit 0 of the Flag field of the eQL-TLV.

The CI_partialChain is copied to bit 1 of the Flag field of the eQL-TLV.

The CI_numeSECs is copied to the Number of cascaded eSECs from the nearest SSU/PRC field of the eQL-TLV.

The CI_numSECs is copied to the Number of cascaded SECs from the nearest SSU/PRC field of the eQL-TLV.

QLmode: This function operates in the QL-enabled mode.

Timing loop prevention: If RI_CS equals CI_CS, the transmitted SSM shall be forced to the "1111" pattern to prevent a timing loop condition to occur (see clause 5.13).

SSM usage: The function supports the capability to prevent synchronization quality information from passing the interface (see clause 5.5.2). In the case where MI_SSMdis is true, no OSMC is activated, i.e., neither event nor information OSMC PDU messages are generated.

SSM: The SSM field in the GFP-F(SSM) frame shall transport the 4-bit SSM code.

The OSMC message channel, specified in [ITU-T G.709], defines 2 types of messages, event and information ones. The generation of an event-based message is triggered by a change of the SSM code, SD_CI_QL value, within the delay times (THM, TNSM, TSM) specified in clause 5.14. The information messages are generated with a periodicity of one second.

Defects: None.

Consequent actions:

if (MI_SSMdis == true)

then no message is generated

else

if (RI_CS == CI_CS)

then SSM = 1111

else 	SSM = SSM[CI_QL]

fi

fi

Defect correlations: None.

Performance monitoring: None.

### 8.10.2	OTUk to SD adaptation sink (OTUk/SD_A_Sk)

Symbol:

Figure 46 – OTUk/SD_A_Sk symbol

Interfaces:

Table 48 – OTUk/SD_A_Sk input and output signals

| Input(s) | Output(s) |
| --- | --- |
| OTUk_AI_D OTUk_AI_CK OTUk_AI_TSF OTUk/SD_A_Sk_MI_CSid | SD_CI_CK SD_CI_SSF SD_CI_CS SD_CI_QL SD_CI_CLKID SD_CI_mixedChain SD_CI_partialChain SD_CI_numeSECs SD_CI_numSECs OTUk_RI_CS |

Processes:

This function extracts and accepts the 4-bit synchronization status messages (SSMs) that are transmitted in the SSM event-based and information messages carried by the OSMC, as defined in [ITU-T G.709] and [ITU-T G.7041]. If the eQL-TLV is supported and is present, this function extracts and accepts the 8-bit eSSM, SyncE clockIdentity of the originator of the eQL-TLV, Flag, Number of cascaded eSECs from the nearest SSU/PRC, and Number of cascaded SECs from the nearest SSU/PRC.

SSM: For all options, the recovered SSM bits and, if present, eSSM bits are accepted immediately if no CI_SSF or AI_TSF condition applies.

If the 8-bit eSSM is not present, the eQL-TLV is not supported, or the network is Option III, the accepted code shall be converted to a quality level QL[SSM] as specified in Table 8 (option I), Table 10 (option II), or Table 12 (option III) and output via CI_QL.

If the 8-bit eSSM is present, the eQL-TLV is supported, and the network is Option I or Option II, the accepted code shall be converted to a quality level QL[SSM] as specified in Table 11-7 and Table 11-8 of [ITU-T G.8264]. If the SSM code is valid and the eSSM code is not valid, QL-INV shall be generated. [ITU-T G.8264] indicates that SSM codes and eSSM codes for synchronous Ethernet for Option III are for further study.  The SD_CI_QL output is generated with the SSM extracted from both event-based and information messages.

QLmode: This function operates in the QL-enabled mode.

SSM support: SSMs are supported.

Clock source identifier: The function shall insert the clock-source identifier received via MI_CSid into CI_CS and RI_CS to support timing loop prevention as described in clause 5.13.

SyncE clockIdentity: The SyncE clockIdentity of the eQL-TLV (see Table 11-5 of [ITU-T G.8264]), if present and supported, is copied to CI_CLKID.

Flag field: Bits 0 and 1 of the Flag field of the eQL-TLV, if present and supported, are copied to CI_mixedChain and CI_partialChain, respectively.

Number of cascaded eSECs from the nearest SSU/PRC: The Number of cascaded eSECs from the nearest SSU/PRC field of the eQL-TLV is copied to CI_numeSECs.

Number of cascaded SECs from the nearest SSU/PRC: The Number of cascaded SECs from the nearest SSU/PRC field of the eQL-TLV is copied to CI_numSECs.

Defects:

dLOSSM: If no valid GFP-F(SSM) is received during 5 seconds, a loss of SSM (LOSSM) defect is detected. The LOSSM defect is cleared on receipt of the first GFP-F(SSM) frame.

Consequent actions:

aSSF		CI_SSF or AI_TSF or dLOSSM

CI_QL = QL[SSM]

Defect correlations:

cLOSSM		dLOSSM and (not CI_SSF)

Performance monitoring: None.

## 8.11	OSC layer adaptation functions

### 8.11.1 	OSC to SD adaptation source (OSC/SD_A_So)

Symbol:

Figure 47 – OSC/SD_A_So symbol

Interfaces:

Table 49 – OSC/SD_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_QL SD_CI_CS SD_CI_CLKID SD_CI_mixedChain SD_CI_partialChain SD_CI_numeSECs SD_CI_numSECs OSC_TI_CK OSC_RI_CS OSC/SD_A_So_MI_SSMdis OSC/SD_A_So_MI_OptII_QLGEN OSC/SD_A_So_MI_OptII_QLRES | OSC_AI_D |

Processes:

eQL-TLV not supported:

If the eQL-TLV is not supported, this function converts the CI_QL into a vendor-specific SSM code and inserts it in the OSMC overhead of the OSC.

The SSM message that shall be generated and inserted depends on the applied quality-level indication that is input to the adaptation source function (CI_QL).

eQL-TLV supported:

If the eQL-TLV is supported, this function converts the CI_QL into a vendor-specific SSM code and vendor-specific eSSM code, and inserts them in the OSMC overhead of the OSC.

The SSM and eSSM messages that shall be generated and inserted depend on the applied quality-level indication that is input to the adaptation source function (CI_QL).

The CI_CLKID is copied to the SyncE clockIdentity of the originator of the eQL-TLV field of the eQL-TLV (see Table 11-5 of [ITU-T G.8264]).

The CI_mixedChain is copied to bit 0 of the Flag field of the eQL-TLV.

The CI_partialChain is copied to bit 1 of the Flag field of the eQL-TLV.

The CI_numeSECs is copied to the Number of cascaded eSECs from the nearest SSU/PRC field of the eQL-TLV.

The CI_numSECs is copied to the Number of cascaded SECs from the nearest SSU/PRC field of the eQL-TLV.

QLmode: This function operates in the QL-enabled mode.

Timing loop prevention: If RI_CS equals CI_CS, the transmitted SSM shall be forced to an SSM code that corresponds to the "1111" pattern in Tables 8 and 10 to prevent a timing loop condition to occur (see clause 5.13).

SSM usage: The function supports the capability to prevent synchronization quality information from passing the interface (see clause 5.5.2). In the case where MI_SSMdis is true, no OSMC is activated, i.e., neither event nor information OSMC PDU messages are generated.

SSM: The SSM message shall transport the vendor-specific SSM code.

Defects: None.

Consequent actions:

if (MI_SSMdis == true)

then no message is generated

else

if (RI_CS == CI_CS)

then SSM = vendor-specific equivalent of 1111

else 	SSM = SSM[CI_QL]

fi

fi

Defect correlations: None.

Performance monitoring: None.

### 8.11.2	OSC to SD adaptation sink (OSC/SD_A_Sk)

Symbol:

Figure 48 – OSC/SD_A_Sk symbol

Interfaces:

Table 50 – OSC/SD_A_Sk input and output signals

| Input(s) | Output(s) |
| --- | --- |
| OSC_AI_D OSC_AI_CK OSC_AI_TSF OSC/SD_A_Sk_MI_CSid | SD_CI_CK SD_CI_SSF SD_CI_CS SD_CI_QL SD_CI_CLKID SD_CI_mixedChain SD_CI_partialChain SD_CI_numeSECs SD_CI_numSECs OSC_RI_CS |

Processes:

This function extracts and accepts the vendor-specific synchronization status messages (SSMs) carried by the OSMC. If the eQL-TLV is supported and is present, this function extracts and accepts the vendor-specific eSSM, SyncE clockIdentity of the originator of the eQL-TLV, Flag, Number of cascaded eSECs from the nearest SSU/PRC, and Number of cascaded SECs from the nearest SSU/PRC.

SSM: For all options, the recovered SSM bits and, if present and supported, eSSM bits are accepted immediately if no CI_SSF or AI_TSF condition applies.

The accepted SSM code and, if present and supported, eSSM code shall be converted to a quality level QL[SSM] and output via CI_QL.

QLmode: --This function operates in the QL-enabled mode.

SSM support: SSMs are supported.

Clock source identifier: The function shall insert the clock-source identifier received via MI_CSid into CI_CS and RI_CS to support timing loop prevention as described in clause 5.13.

SyncE clockIdentity: The SyncE clockIdentity of the eQL-TLV (see Table 11-5 of [ITU-T G.8264]), if present and supported, is copied to CI_CLKID.

Flag field: Bits 0 and 1 of the Flag field of the eQL-TLV, if present and supported, are copied to CI_mixedChain and CI_partialChain, respectively.

Number of cascaded eSECs from the nearest SSU/PRC: The Number of cascaded eSECs from the nearest SSU/PRC field of the eQL-TLV, if present and supported, is copied to CI_numeSECs.

Number of cascaded SECs from the nearest SSU/PRC: The Number of cascaded SECs from the nearest SSU/PRC field of the eQL-TLV, if present and supported, is copied to CI_numSECs.

Defects:

dLOSSM: If no valid SSM is received during a vendor-specific period, a loss of SSM (LOSSM) defect is detected. The LOSSM defect is cleared on receipt of the first SSM message.

Consequent actions:

aSSF		CI_SSF or AI_TSF or dLOSSM

CI_QL = QL[SSM]

Defect correlations:

cLOSSM		dLOSSM and (not CI_SSF)

Performance monitoring: None.

## 8.12	FlexO layer adaptation functions

### 8.12.1	FlexO to SD adaptation source (FlexO/SD_A_So)

NOTE – FlexO interfaces with vendor specific application identifiers may support an OSMC function. The encapsulation of the messages and overhead location are then vendor specific.

Symbol:

Figure 49 – FlexO/SD_A_So symbol

Interfaces:

Table 51 – FlexO/SD_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_QL SD_CI_CS SD_CI_CLKID SD_CI_mixedChain SD_CI_partialChain SD_CI_numeSECs SD_CI_numSECs FlexO/SD_A_So_MI_SSMdis FlexO/SD_A_So_MI_OptII_QLGEN FlexO/SD_A_So_MI_OptII_QLRES FlexO_AP: FlexO_AI_CK FlexO_AI_FS FlexO_RI_CS | FlexO_AI_D |

Processes:

eQL-TLV not supported:

If the eQL-TLV is not supported, this function converts the CI_QL into the 4-bit SSM code, as defined in clause 5.5, and inserts it in the SSM field in the GFP-F(SSM) frame transported in the OSMC. The OSMC carries event and information messages, as defined in [ITU-T G.709.1] and [ITU-T G.7041].

The SSM message that shall be generated and inserted depends on the applied quality-level indication that is input to the adaptation source function (CI_QL). Tables 27 to 29 present the relationship between the existing set of QLs and the output SSM.

eQL-TLV supported:

If the eQL-TLV is supported, this function converts the CI_QL into the 4-bit SSM code and the 8-bit eSSM code, as defined in [ITU-T G.8264]. The 4-bit SSM code is inserted in the SSM field in the GFP-F(SSM) frame transported in the OSMC, and the 8-bit eSSM code is inserted in the eSSM field of the eQL-TLV of the GFP-F(SSM) frame transported in the OSMC.  The OSMC carries event and information messages, as defined in [ITU-T G.709] and clause 7.11 of [ITU-T G.7041].

The SSM message that shall be generated and inserted depends on the applied quality-level indication that is input to the adaptation source function (CI_QL). Tables 11-7 and 11-8 of [ITU-T G.8264] present the relationship between the existing set of QLs and the output SSM and eSSM.

The CI_CLKID is copied to the SyncE clockIdentity of the originator of the eQL-TLV field of the eQL-TLV (see Table 11-5 of [ITU-T G.8264]).

The CI_mixedChain is copied to bit 0 of the Flag field of the eQL-TLV.

The CI_partialChain is copied to bit 1 of the Flag field of the eQL-TLV.

The CI_numeSECs is copied to the Number of cascaded eSECs from the nearest SSU/PRC field of the eQL-TLV.

The CI_numSECs is copied to the Number of cascaded SECs from the nearest SSU/PRC field of the eQL-TLV.

QLmode: This function operates in the QL-enabled mode.

Timing loop prevention: If RI_CS equals CI_CS, the transmitted SSM shall be forced to the "1111" pattern to prevent a timing loop condition to occur (see clause 5.13).

SSM usage: The function supports the capability to prevent synchronization quality information from passing the interface (see clause 5.5.2). In the case where MI_SSMdis is true, no OSMC is activated, i.e., neither event nor information OSMC PDU messages are generated.

SSM: The SSM field in the GFP-F(SSM) frame shall transport the 4-bit SSM code.

The OSMC message channel, specified in [ITU-T G.709.1], defines two types of messages, event and information ones. The generation of an event-based message is triggered by a change of the SSM code, SD_CI_QL value, within the delay times (THM, TNSM, TSM) specified in clause 5.14. The information messages are generated with a periodicity of one second.

Defects: None.

Consequent actions:

if (MI_SSMdis == true)

then no message is generated

else

if (RI_CS == CI_CS)

then SSM = 1111

else 	SSM = SSM[CI_QL]

fi

fi

Defect correlations: None.

Performance monitoring: None.

### 8.12.2	FlexO to SD adaptation sink (FlexO/SD_A_Sk)

Symbol:

Figure 50 – FlexO/SD_A_Sk symbol

Interfaces:

Table 52 – FlexO/SD_A_Sk input and output signals

| Input(s) | Output(s) |
| --- | --- |
| FlexO_AI_D FlexO_AI_CK FlexO_AI_FS FlexO_AI_TSF FlexO/SD_A_Sk_MI_CSid | SD_CI_CK SD_CI_SSF SD_CI_CS SD_CI_QL SD_CI_CLKID SD_CI_mixedChain SD_CI_partialChain SD_CI_numeSECs SD_CI_numSECs FlexO_RI_CS |

Processes:

This function extracts and accepts the 4-bit synchronization status messages (SSMs) that are transmitted in the SSM event-based and information messages carried by the OSMC, as defined in [ITU-T G.709.1] and [ITU-T G.7041]. If the eQL-TLV is supported and is present, this function extracts and accepts the 8-bit eSSM, SyncE clockIdentity of the originator of the eQL-TLV, Flag, Number of cascaded eSECs from the nearest SSU/PRC, and Number of cascaded SECs from the nearest SSU/PRC.

SSM: For all options, the recovered SSM bits are accepted immediately if no CI_SSF or AI_TSF condition applies.

If the 8-bit eSSM is not present, the eQL-TLV is not supported, or the network is Option III, the accepted code shall be converted to a quality level QL[SSM] as specified in Table 8 (option I), Table 10 (option II), or Table 12 (option III) and output via CI_QL.

If the 8-bit eSSM is present, the eQL-TLV is supported, and the network is Option I or Option II, the accepted code shall be converted to a quality level QL[SSM] as specified in Table 11-7 and Table 11-8 of [ITU-T G.8264]. If the SSM code is valid and the eSSM code is not valid, QL-INV shall be generated. [ITU-T G.8264] indicates that SSM codes and eSSM codes for synchronous Ethernet for Option III are for further study. The SD_CI_QL output is generated with the SSM extracted from both event-based and information messages.

QLmode: This function operates in the QL-enabled mode.

SSM support: SSMs are supported.

Clock source identifier: The function shall insert the clock-source identifier received via MI_CSid into CI_CS and RI_CS to support timing loop prevention as described in clause 5.13.

SyncE clockIdentity: The SyncE clockIdentity of the eQL-TLV (see Table 11-5 of [ITU-T G.8264]), if present and supported, is copied to CI_CLKID.

Flag field: Bits 0 and 1 of the Flag field of the eQL-TLV, if present and supported, are copied to CI_mixedChain and CI_partialChain, respectively.

Number of cascaded eSECs from the nearest SSU/PRC: The Number of cascaded eSECs from the nearest SSU/PRC field of the eQL-TLV is copied to CI_numeSECs.

Number of cascaded SECs from the nearest SSU/PRC: The Number of cascaded SECs from the nearest SSU/PRC field of the eQL-TLV is copied to CI_numSECs.

Defects:

dLOSSM: If no valid GFP-F(SSM) is received during 5 seconds, a loss of SSM (LOSSM) defect is detected. The LOSSM defect is cleared on receipt of the first GFP-F(SSM) frame.

Consequent actions:

aSSF		CI_SSF or AI_TSF or dLOSSM

CI_QL = QL[SSM]

Defect correlations:

cLOSSM		dLOSSM and (not CI_SSF)

Performance monitoring: None.

## 8.13	FlexE layer adaptation functions

The synchronization information (i.e. SSM messages) is carried in the synchronization messaging channel (SMC) of FlexE overhead on the first FlexE Instance of the FlexE group, as defined in [OIF FLEXE IA].

### 8.13.1	FlexE to SD adaptation source (FlexE/SD_A_So)

Symbol:

Figure 50A – FlexE/SD_A_So symbol

Interfaces:

Table 52A – FlexE/SD_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_QL SD_CI_CS SD_CI_CLKID SD_CI_mixedChain SD_CI_partialChain SD_CI_numeSECs SD_CI_numSECs FlexE/SD_A_So_MI_SSMdis FlexE/SD_A_So_MI_OptII_QLGEN FlexE/SD_A_So_MI_OptII_QLRES FlexE/SD_A_So_MI_SC FlexE_AI_CK FlexE_AI_FS FlexE_AI_MFS | FlexE_AI_D |

Processes:

eQL-TLV not supported:

If the eQL-TLV is not supported, this function converts the CI_QL into the 4-bit SSM code, as defined in clause 5.5, SSM inserts it in the SSM field of the QL TLV of the ESMC PDU. The ESMC PDU is 66B block encoded per [IEEE 802.3] clause 82, and then is transported in the Synchronization Messaging Channel. The Synchronization Messaging Channel of FlexE is defined in [OIF FLEXE IA].

The SSM message that shall be generated and inserted depends on the applied quality-level indication that is input to the adaptation source function (CI_QL). Tables 27 to 29 present the relationship between the existing set of QLs and the output SSM.

eQL-TLV supported:

If the eQL-TLV is supported, this function converts the CI_QL into the 4-bit SSM code and the 8-bit eSSM code, as defined in [ITU-T G.8264].

The 4-bit SSM code is inserted in the SSM field in the QL TLVand the 8-bit eSSM code is inserted in the eSSM field of the eQL-TLV in an ESMC PDU as defined in [ITU-T G.8264] which is then transported in the Synchronization Messaging Channel.  The Synchronization Messaging Channel of FlexE is defined in [OIF FLEXE IA].

The SSM message that shall be generated and inserted depends on the applied quality-level indication that is input to the adaptation source function (CI_QL). Tables 11-7 and 11-8 of [ITU-T G.8264] present the relationship between the existing set of QLs and the output SSM and eSSM.

The CI_CLKID  is copied to the SyncE clockIdentity of the originator of the eQL-TLV field of the eQL-TLV (see Table 11-5/G.8264).

The CI_mixedChain is copied to bit 0 of the Flag field of the eQL-TLV.

The CI_partialChain is copied to bit 1 of the Flag field of the eQL-TLV.

The CI_numeSECs is copied to the Number of cascaded eSECs from the nearest SSU/PRC field of the eQL-TLV.

The CI_numSECs is copied to the Number of cascaded SECs from the nearest SSU/PRC field of the eQL-TLV.

QLmode: This function operates in the QL-enabled mode.

Timing loop prevention: If RI_CS equals CI_CS, the transmitted SSM shall be forced to the "1111" pattern to prevent a timing loop condition to occur (see clause 5.13).

SSM usage: The function supports the capability to prevent synchronization quality information from passing the interface (see clause 5.5.2). In the case where MI_SSMdis is true, no ESMC is activated, i.e., neither event nor information ESMC PDUs are generated.

SSM: If eQL-TLV is not supported, the SSM code field in the QL TLV shall transport the 4-bit SSM code. If eQL-TLV is supported, the eSSM code field in the eQL-TLV shall transport the 8-bit eSSM code.

The ESMC, specified in [ITU-T G.8264], defines 2 types of messages, event and information ESMC PDUs. The generation of an ESMC event PDU is triggered by a change of the SSM code, SD_CI_QL value, within the delay times (THM, TNSM, TSM) specified in clause 5.14. The ESMC information PDUs are generated with a periodicity of one second.

SC: This function operates when the Synchronization Configuration (SC) bit is 1, which indicates that the FlexE group is configured to support SMC (see clause 7.3.5 in [OIF FLEXE IA]).

Defects: None.

Consequent actions:

if (MI_SSMdis == true OR MI_SC == false)

then no message is generated

else

if (RI_CS == CI_CS)

then SSM = 1111

else 	SSM = SSM[CI_QL]

fi

fi

Defect correlations: None.

Performance monitoring: None.

### 8.13.2	FlexE to SD adaptation sink (FlexE/SD_A_Sk)

Symbol:

Figure 50B – FlexE/SD_A_Sk symbol

Interfaces:

Table 52B – FlexE/SD_A_Sk input and output signals

| Input(s) | Output(s) |
| --- | --- |
| FlexE_AI_D FlexE_AI_CK FlexE_AI_FS FlexE_AI_MFS FlexE_AI_TSF FlexE/SD_A_Sk_MI_CSid FlexE/SD_A_So_MI_SC | SD_CI_CK SD_CI_SSF SD_CI_CS SD_CI_QL SD_CI_CLKID SD_CI_mixedChain SD_CI_partialChain SD_CI_numeSECs SD_CI_numSECs |

Processes:

This function extracts the 4-bit SSMs from the QL TLV of the ESMC PDU, as defined in [ITU-T G.8264]. If the eQL-TLV is supported and is present, this function extracts the 8-bit eSSM, SyncE clockIdentity of the originator of the eQL-TLV, Flag, Number of cascaded eSECs from the nearest SSU/PRC, and Number of cascaded SECs from the nearest SSU/PRC.

Acceptance: For all options, the recovered SSM bits and, if supported and present, eSSM bits, SyncE clockIdentity of the originator of the eQL-TLV, Flag, Number of cascaded eSECs from the nearest SSU/PRC, and Number of cascaded SECs from the nearest SSU/PRC are accepted immediately if no CI_SSF or AI_TSF condition applies.

SSM/eSSM to QL:

- If the 8-bit eSSM is not present, the eQL-TLV is not supported, or the network is Option III, the accepted code shall be converted to a quality level QL[SSM] as specified in Table 8 (option I), Table 10 (option II), Table 12 (option III) and output via CI_QL.

- If the 8-bit eSSM is present, the eQL-TLV is supported, and the network is Option I or Option II, the accepted code shall be converted to a quality level QL[SSM] as specified in Table 11-7 and Table 11-8 of [ITU-T G.8264]. If the SSM code is valid and the eSSM code is not valid, QL-INV shall be generated. [ITU-T G.8264] indicates that SSM codes and eSSM codes for synchronous Ethernet for Option III are for further study. The SD_CI_QL output is generated with the SSM extracted from both event-based and information messages.

QLmode: This function operates in the QL-enabled mode.

SSM support: SSMs are supported.

Clock source identifier: The function shall insert the clock-source identifier received via MI_CSid into CI_CS and RI_CS to support timing loop prevention as described in clause 5.13.

SyncE clockIdentity: The accepted SyncE clockIdentity of the eQL-TLV (see Table 11-5/G.8264) is copied to CI_CLKID.

Flag field: Bits 0 and 1 of the accepted Flag field of the eQL-TLV are copied to CI_mixedChain and CI_partialChain, respectively.

Number of cascaded eSECs from the nearest SSU/PRC: The accepted Number of cascaded eSECs from the nearest SSU/PRC is copied to CI_numeSECs.

Number of cascaded SECs from the nearest SSU/PRC: The accepted Number of cascaded SECs from the nearest SSU/PRC is copied to CI_numSECs.

SC: This function operates when the Synchronization Configuration (SC) bit is 1, which indicates that the FlexE group is configured to support SMC (see clause 7.3.5 in [OIF FLEXE IA]).

Defects:

dLOESMC: If no valid ESMC PDU is received during 5 seconds, a loss of ESMC (LOESMC) defect is detected. The LOESMC defect is cleared on receipt of the first ESMC PDU.

Consequent actions:

aSSF		CI_SSF or AI_TSF or dLOESMC

CI_QL = QL[SSM]

Defect correlations:

cLOESMC		dLOESMC and (not CI_SSF)

Performance monitoring: None.

# 9	Equipment clock to transport layers clock adaptation functions

## 9.1	STM-N layer

### 9.1.1	STM-N layer clock adaptation source (MSn-LC_A_So)

Symbol:

Figure 51 – MSn-LC_A_So symbol

Interfaces:

Table 53 – MSn-LC_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_CK | STMn_TI_CK STMn_TI_FS |

Processes:

This function performs the STM-N clock and frame start signal generation locked to the network element clock signal SD_CI_CK, to time the adaptation source functions in this layer (and its server layers).

Clock generation: The function shall generate the clock (bit) reference signal STMn_TI_CK for the STM-N signal. The STMn_TI_CK frequency shall be 155 520 kHz (N=1), 622 080 kHz (N=4), 2 488 320 kHz (N=16), 9 953 280 kHz (N=64) locked to the input signal SD_CI_CK.

Jitter limiter: The function shall process the signal such that, in the absence of input jitter at the synchronization interface, the intrinsic jitter at the STM-N output interface shall be as specified in [ITU-T G.813].

Frame start signal generation: The function shall generate the frame start reference signal STMn_TI_FS for the STM-N signal. The STMn_TI_FS signal shall be active once per 19 440 (N=1), 77 760 (N=4), 311 040 (N=16), 1 244 160 (N=64) clock cycles.

Defects: None.

Consequent actions: None.

Defect correlations: None.

Performance monitoring: None.

## 9.2	VC layers

### 9.2.1	VC-n layer clock adaptation source (Sn-LC_A_So)

Symbol:

Figure 52 – Sn-LC_A_So symbol

Interfaces:

Table 54 – Sn-LC_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_CK | Sn_TI_CK Sn_TI_FS Sn_TI_MFS |

Processes:

This function performs the VC-n (n=4-64c, 4-16c, 4-4c, 4, 3) clock and frame start signal generation locked to the network element clock signal SD_CI_CK to time the adaptation source and connection functions in this layer.

Clock generation: The function shall generate the clock (bit) reference signal Sn_TI_CK for the VC-n signal. The Sn_TI_CK frequency shall be 9 621 504 kHz (n=4-64c), 2 405 376 kHz (n=4-16c), 601 344 kHz (n=4-4c), 150 336 kHz (n=4), 48 960 kHz (n=3), locked to the input signal SD_CI_CK.

Jitter limiter: For further study.

Frame start signal generation: The function shall generate the frame start reference signal Sn_TI_FS for the VC-n signal. The Sn_TI_FS signal shall be active once per 1 202 688 (n=4-64c), 300 672 (n=4-16c), 75 168 (n=4-4c), 18 792 (n=4), 6120 (n=3) clock cycles, and the multiframe reference Sn_TI_MFS shall be active once every 4 frames.

Defects: None.

Consequent actions: None.

Defect correlations: None.

Performance monitoring: None.

### 9.2.2	VC-m layer clock adaptation source (Sm-LC_A_So)

Symbol:

Figure 53 – Sm-LC_A_So symbol

Interfaces:

Table 55 – Sm-LC_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_CK | Sm_TI_CK Sm_TI_FS |

Processes:

This function performs the VC-m (m=2, 12, 11) clock and frame start signal generation locked to the network element clock signal SD_CI_CK to time the adaptation source and connection functions in this layer.

Clock generation: The function shall generate the clock (bit) reference signal Sm_TI_CK for the VC-m signal. The Sm_TI_CK frequency shall be 6848 kHz (m=2), 2240 kHz (m=12), 1664 kHz (m=11), locked to the input signal SD_CI_CK.

Jitter limiter: For further study.

Frame start signal generation: The function shall generate the frame start reference signal Sm_TI_FS for the VC-m signal. The Sm_TI_FS signal shall be active once per 856 (m=2), 280 (m=12), 208 (m=11) clock cycles.

Defects: None.

Consequent actions: None.

Defect correlations: None.

Performance monitoring: None.

## 9.3	Pxx layers

### 9.3.1	Pqs layer clock adaptation source (Pqs-LC_A_So)

Symbol:

Figure 54 – Pqs-LC_A_So symbol

Interfaces:

Table 56 – Pqs-LC_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_CK | Pqs_TI_CK Pqs_TI_FS Pqs_TI_MFS |

Processes:

This function performs the Pqs (q=4, 31) clock and frame start signal generation locked to the network element clock signal SD_CI_CK to time the adaptation source functions in this layer.

Clock generation: The function shall generate the clock (bit) reference signal Pqs_TI_CK for the Pqs signal. The Pqs_TI_CK frequency shall be 139 264 kHz (q=4), 34 368 kHz (q=31) locked to the input signal SD_CI_CK.

Jitter limiter: For further study.

Frame start signal generation: The function shall generate the frame start reference signal Pqs_TI_FS for the Pqs signal. The Pqs_TI_FS signal shall be active once per 17 408 (q=4), 4296 (q=31) clock cycles. Pqs_TI_MFS shall be active once every 4 frames.

Defects: None.

Consequent actions: None.

Defect correlations: None.

Performance monitoring: None.

### 9.3.2	P12s layer clock adaptation source (P12s-LC_A_So)

Symbol:

Figure 55 – P12s-LC_A_So symbol

Interfaces:

Table 57 – P12s-LC_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_CK | P12s_TI_CK P12s_TI_FS P12s_TI_MFS |

Processes:

This function performs the P12s clock and frame start signal generation locked to the clock signal SD_CI_CK to time the adaptation source functions in this layer.

Clock generation: The function shall generate the clock (bit) reference signal P12s_TI_CK for the P12s signal. The P12s_TI_CK frequency shall be 2048 kHz locked to the input signal SD_CI_CK.

NOTE – If an SD_CI_SSF is present at the input of the function, it is also present at the input of P12s/SD and an AIS is generated by P12_TT.

Jitter limiter: The function shall process the signal such that, in the absence input jitter at the sychronization interface, the intrinsic jitter at the E12 output interface is compatible with [ITU-T G.813] or [ITU-T G.8262].

Frame start signal generation: The function shall generate the frame start reference signal P12s_TI_FS for the P12s signal. The P12s_TI_FS signal shall be active once per 256 clock cycles and P12s_TI_MFS once every 16 frames.

Defects: None.

Consequent actions: None.

Defect correlations: None.

Performance monitoring: None.

### 9.3.3	P11s layer clock adaptation source (P11s-LC_A_So)

For further study.

## 9.4	T12 layer

### 9.4.1	T12 layer clock adaptation source (T12-LC_A_So)

Symbol:

Figure 56 – T12-LC_A_So symbol

Interfaces:

Table 58 – T12-LC_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_CK | T12_TI_CK |

Processes:

This function performs the T12 clock signal generation locked to the clock signal SD_CI_CK to time the adaptation source functions T12/SD_A_So.

Clock generation: The function shall generate the clock reference signal T12_TI_CK for the 2048 kHz signal. The T12_TI_CK frequency shall be 2048 kHz locked to the input signal SD_CI_CK.

NOTE – If an SD_CI_SSF is present at the input of the function, it is also present at the input of T12s/SD and a squelch action is activated.

Jitter limiter: The function shall process the signal such that, in the absence of input jitter at the synchronization interface, the intrinsic jitter at the 2048 kHz output interface is compatible with [ITU-T G.813] or [ITU-T G.8262].

Defects: None.

Consequent actions: None.

Defect correlations: None.

Performance monitoring: None.

## 9.5	T01 layer

### 9.5.1	T01 layer clock adaptation source (T01-LC_A_So)

For further study.

## 9.6	T02 layer

### 9.6.1	T02 layer clock adaptation source (T02-LC_A_So)

For further study.

## 9.7	T21 layer

### 9.7.1	T21 layer clock adaptation source (T21-LC_A_So)

Symbol:

Figure 57 – T21-LC_A_So symbol

Interfaces:

Table 59 – T21-LC_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_CK | T21_TI_CK |

Processes:

This function performs the T21 clock signal generation locked to the clock signal SD_CI_CK to time the adaptation source functions T21/SD_A_So.

Clock generation: The function shall generate the clock reference signal T21_TI_CK for the 6312 kHz signal. The T21_TI_CK frequency shall be 6312 kHz locked to the input signal SD_CI_CK.

NOTE – If an SD_CI_SSF is present at the input of the function, it is also present at the input of T21/SD and a squelch action is activated.

Jitter limiter: For further study.

Defects: None.

Consequent actions: None.

Defect correlations: None.

Performance monitoring: None.

## 9.8	ETYn layer

### 9.8.1	ETYn layer clock adaptation source (ETYn-LC_A_So)

Symbol:

Figure 58 – ETYn-LC_A_So symbol

Interfaces:

Table 60 – ETY-LC_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_CK | ETYn_TI_CK |

Processes:

This function performs the ETYn (n=4, 3, 2) clock signal generation locked to the network element clock signal SD_CI_CK, to time the adaptation source functions in this layer.

Clock generation: The function shall generate the clock (bit) reference signal ETYn_TI_CK for the ETYn signal. The ETYn_TI_CK frequency shall be 10 000 000 kHz (n=4), 1 000 000 kHz (n=3), 100 000 kHz (n=2) locked to the input signal SD_CI_CK.

NOTE – The ETY_TI_CK signal provides the reference clock to the ETY/ETH adaptation source function. Several processes within this adaptation source function operate on this clock. The line encoding process in this adaptation source function generates the line clock using the ETY_CI_CK as input signal.

Jitter limiter: The function shall process the signal such that, in the absence of input jitter at the synchronization interface, the intrinsic jitter at the ETYn output interface shall be as specified in [ITU-T G.8262].

Defects: None.

Consequent actions: None.

Defect correlations: None.

Performance monitoring: None.

## 9.9	ODU layer

### 9.9.1	ODUk layer clock adaptation source (ODUk-LC_A_So)

Symbol:

Figure 59 – ODUk-LC_A_So symbol

Interfaces:

Table 61 – ODUk-LC_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_CK | ODUk_TI_CK |

Processes:

This function performs the ODUk (k=4, 3, 2, 1) clock signal generation locked to the network element clock signal SD_CI_CK, to time the adaptation source functions in this layer.

Clock generation: The function shall generate the clock (bit) reference signal ODUk_TI_CK for the ODUk signal. The ODUk_TI_CK frequency shall be 239/227  99 532 800 kHz (k=4), 239/236  39 813 120 kHz (k=3), 239/237  9 953 280 kHz (k=2), 239/238  2 488 320 kHz (k=1) locked to the input signal SD_CI_CK.

NOTE – The ODUk_TI_CK signal provides the reference clock to the ODUkP/ODUj adaptation source function. Several processes within this adaptation source function operate on this clock as well as processes in the following ODUkP termination, OTUk/ODUk adaptation, OTUk termination and OCh/OTUk adaptation functions.

Jitter limiter: The function shall process the signal such that, in the absence of input jitter at the synchronization interface, the intrinsic jitter at the ODUk output interface shall be as specified in [ITU-T G.8251].

Defects: None.

Consequent actions: None.

Defect correlations: None.

Performance monitoring: None.

### 9.9.2	ODUCn layer clock adaptation source (ODUCn-LC_A_So)

Symbol:

Figure 60 – ODUCn-LC_A_So symbol

Interfaces:

Table 62 – ODUCn-LC_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_CK | ODUCn_TI_CK |

Processes:

This function performs the ODUCn clock signal generation locked to the network element clock signal SD_CI_CK, to time the adaptation source functions in this layer.

Clock generation: The function shall generate the clock (bit) reference signal ODUCn_TI_CK for the ODUCn signal. The ODUCn_TI_CK frequency shall be n  239/226  99 532 800 kHz locked to the input signal SD_CI_CK.

NOTE – The ODUCn_TI_CK signal provides the reference clock to the ODUCnP/ODUk adaptation source function. Several processes within this adaptation source function operate on this clock as well as processes in the following ODUCnP termination, OTUCn/ODUCn adaptation, OTUCn termination and OTSiA/OTUCn adaptation functions.

Jitter limiter: The function shall process the signal such that, in the absence of input jitter at the synchronization interface, the intrinsic jitter at the ODUCn output interface shall be as specified in [ITU-T G.8251].

Defects: None.

Consequent actions: None.

Defect correlations: None.

Performance monitoring: None.

## 9.10	OSC layer

### 9.10.1	OSC layer clock adaptation source (OSC-LC_A_So)

Symbol:

Figure 61 – OSC-LC_A_So symbol

Interfaces:

Table 63 – OSC-LC_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_CK | OSC_TI_CK |

Processes:

This function performs the optical supervisory signal clock signal generation locked to the network element clock signal SD_CI_CK, to time the adaptation source functions in this layer.

Clock generation: The function shall generate the clock (bit) reference signal OSC_TI_CK for the optical supervisory signals (OSC) signal. The OSC_TI_CK frequency is vendor specific and shall be locked to the input signal SD_CI_CK.

NOTE – The OSC_TI_CK signal provides the reference clock to the OSC/OTS-O adaptation source function. The OSC processes within this function operate on this clock.

Jitter limiter: The function shall process the signal such that, in the absence of input jitter at the synchronization interface, the intrinsic jitter at the OSC output interface shall be as specified in [ITU-T G.8251].

Defects: None.

Consequent actions: None.

Defect correlations: None.

Performance monitoring: None.

## 9.11	FlexO layer

### 9.11.1	FlexO layer clock adaptation source (FlexO-LC_A_So)

Symbol:

Figure 62 – FlexO-LC_A_So symbol

Interfaces:

Table 64 – FlexO-LC_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_CK | FlexO_TI_CK |

Processes:

This function performs the FlexO clock signal generation locked to the network element clock signal SD_CI_CK, to time the adaptation source functions in this layer. This atomic function is not required for OTUCn over FlexO, because the mapping is bitsynchronous. However, this atomic function is needed for 400 GE or ODUk over FlexO, because those mappings are not bitsynchronous.

Clock generation: The function shall generate the clock (bit) reference signal FlexO_TI_CK for the FlexO signal. The FlexO_TI_CK frequency shall be n  491384/462961  99 532 800 kHz locked to the input signal SD_CI_CK.

NOTE – The FlexO_TI_CK signal provides the reference clock to the FlexO-n/OTUCn adaptation source function. Several processes within this adaptation source function operate on this clock as well as processes in the following FlexO termination and OTSiG/FlexO adaptation functions.

Jitter limiter: The function shall process the signal such that, in the absence of input jitter at the synchronization interface, the intrinsic jitter at the FlexO output interface shall be as specified in [ITU-T G.8251].

Defects: None.

Consequent actions: None.

Defect correlations: None.

Performance monitoring: None.

## 9.12	FlexE layer

### 9.12.1	FlexE layer clock adaptation source (FlexE-LC_A_So)

Symbol:

Figure 62A – FlexE-LC_A_So symbol

Interfaces:

Table 64A – FlexE-LC_A_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| SD_CI_CK | FlexE_TI_CK |

Processes:

This function performs the FlexE clock signal generation locked to the network element clock signal SD_CI_CK, to time the adaptation source functions in this layer.

Clock generation: The function shall generate the clock (bit) reference signal FlexE_TI_CK for the FlexE signal. The FlexE_TI_CK frequency shall be n  16383/16384 × 103 125 000 kHz locked to the input signal SD_CI_CK.

NOTE – The FlexE_TI_CK signal provides the reference clock to the FlexE-n/FlexEC adaptation source function. Several processes within this adaptation source function operate on this clock as well as processes in the following FlexE termination and OTSiG/FlexE adaptation functions.

Jitter limiter: The function shall process the signal such that, in the absence of input jitter at the synchronization interface, the intrinsic jitter at the FlexE output interface shall be as specified in [ITU-T G.8262] or [ITU-T G.8262.1].

Defects: None.

Consequent actions: None.

Defect correlations: None.

Performance monitoring: None.

# 10	T12 section layer functions

Figure 63 – T12 section atomic functions

T12 layer CP

The characteristic information T12_CI of the intra-station electrical layer CP is an electrical 2048 kHz signal of defined amplitude, frequency and pulse shape specified by clause 13 of [ITU-T G.703].

T12 layer AP

The information passing across the T12/SD AP is a 2048 kHz synchronization signal.

## 10.1	T12 connection function T12_C

Not applicable.

## 10.2	T12 trail termination functions

### 10.2.1	T12 trail termination source (T12-Z_TT_So)

NOTE – Z () will be one value out of the set: {75, 120} ().

Symbol:

Figure 64 – T12-Z_TT_So symbol

Interfaces:

Table 65 – T12-Z_TT_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| T12_AI_CK T12_AI_SQLCH | T12_CI_CK |

Processes:

This function generates the electrical 2048 kHz signal used for transmission of synchronization signals to an external equipment on a plesiochronous intra-station section specified by clause 13 of [ITU-T G.703].

Pulse shape: The function shall meet the requirement specified by [ITU-T G.703].

Maximum peak voltage: The function shall meet the requirement specified by [ITU-T G.703].

Minimum peak voltage: The function shall meet the requirement specified by [ITU-T G.703].

Pair in each direction: The function shall meet the requirement specified by [ITU-T G.703].

Defects: None.

Consequent actions:

On activation of T12_AI_SQLCH, the function shall shut down the output within 250 s; on clearing of T12_AI_SQLCH, the function shall output a normal signal within 250 s.

Defect correlations: None.

Performance monitoring: None.

### 10.2.2	T12 trail termination sink (T12-Z_TT_Sk)

NOTE 1 – Z () will be one value out of the set: {75, 120} ().

Symbol:

Figure 65 – T12-Z_TT_Sk symbol

Interfaces:

Table 66 – T12-Z_TT_Sk input and output signals

| Input(s) | Output(s) |
| --- | --- |
| T12_CI_CK T12_TT_Sk_MI_PortMode | T12_AI_CK T12_AI_TSF T12_TT_Sk_MI_cLOS |

Processes:

This function recovers the electrical 2048 kHz signal used for transmission of synchronization signals from an external equipment on a plesiochronous intra-station section specified by clause 13 of [ITU-T G.703].

Input return loss: The function shall meet the requirement specified by [ITU-T G.703].

Port mode: The function shall have a port mode as specified in clause 6.1 of [ITU-T G.806].

NOTE 2 – The AUTO state of the port mode process is optional.

Defects:

The function shall detect 2048 kHz loss of signal defect (dLOS) as defined for the 2048 kbit/s dLOS specification in [ITU-T G.775].

Consequent actions:

aTSF		dLOS

Defect correlations:

cLOS		MON and dLOS

Performance monitoring: None.

# 11	T01 section layer functions

Figure 66 – T01 section atomic functions

T01 layer CP

The characteristic information T01_CI of the intra-station electrical layer CP is an electrical 64 kHz composite timing signal of defined amplitude, frequency and pulse shape specified by clause 4.2.2 of [ITU-T G.703].

T01 layer AP

The information passing across the T01/SD AP is a 64 kHz synchronization signal.

## 11.1	T01 connection function T01_C

Not applicable.

## 11.2	T01 trail termination functions

### 11.2.1	T01 trail termination source (T01_TT_So)

Symbol:

Figure 67 – T01_TT_So symbol

Interfaces:

Table 67 – T01_TT_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| T01_AI_CK T01_AI_SQLCH | T01_CI_CK |

Processes:

This function generates the electrical 64 kHz composite timing signal used for transmission of synchronization signals to an external equipment on a plesiochronous intra-station section specified by clause 4.2.2 of [ITU-T G.703].

Pulse shape: The function shall meet the requirement specified by [ITU-T G.703].

Maximum peak voltage: The function shall meet the requirement specified by [ITU-T G.703].

Minimum peak voltage: The function shall meet the requirement specified by [ITU-T G.703].

Pair in each direction: The function shall meet the requirement specified by [ITU-T G.703].

Defects: None.

Consequent actions:

On activation of T01_AI_SQLCH, the function shall shut down the output within 250 s; on clearing of T01_AI_SQLCH, the function shall output a normal signal within 250 s.

Defect correlations: None.

Performance monitoring: None.

### 11.2.2	T01 trail termination sink (T01_TT_Sk)

Symbol:

Figure 68 – T01_TT_Sk symbol

Interfaces:

Table 68 – T01_TT_Sk input and output signals

| Input(s) | Output(s) |
| --- | --- |
| T01_CI_CK T01_TT_Sk_MI_PortMode | T01_AI_CK T01_AI_TSF T01_TT_Sk_MI_cLOS |

Processes:

This function recovers the electrical 64 kHz composite timing signal used for transmission of synchronization signals from an external equipment on a plesiochronous intra-station section specified by clause 4.2.2 of [ITU-T G.703].

Port Mode: The function shall have a port mode as specified in clause 6.1 of [ITU-T G.806].

NOTE – The AUTO state of the port mode process is optional.

Defects:

The function shall detect 64 kHz loss of signal defect (dLOS) as defined for the 64 kbit/s dLOS specification in [ITU-T G.775].

Consequent actions:

aTSF		dLOS

Defect correlations:

cLOS		MON and dLOS

Performance monitoring: None.

# 12	T02 section layer functions

Figure 69 – T02 section atomic functions

T02 layer CP

The characteristic information T02_CI of the intra-station electrical layer CP is an electrical 64 kHz composite timing signal of defined amplitude, frequency and pulse shape specified by Appendix II of [ITU-T G.703].

T02 layer AP

The information passing across the T02/SD AP is a 64 kHz synchronization signal.

## 12.1	T02 connection function T02_C

Not applicable.

## 12.2	T02 trail termination functions

### 12.2.1	T02 trail termination source (T02_TT_So)

For further study.

### 12.2.2	T02 trail termination sink (T02_TT_Sk)

Symbol:

Figure 70 – T02_TT_Sk symbol

Interfaces:

Table 69 – T02_TT_Sk input and output signals

| Input(s) | Output(s) |
| --- | --- |
| T02_CI_CK | T02_AI_CK T02_AI_TSF T02_TT_Sk_MI_cLOS |

Processes:

This function recovers the electrical 64 kHz composite timing signal used for transmission of synchronization signals from an external equipment on a plesiochronous intra-station section specified by Appendix II of [ITU-T G.703].

Defects:

The function shall detect 64 kHz loss of signal defect (dLOS) as defined in Appendix II of [ITU-T G.703].

Consequent actions:

aTSF		dLOS

Defect correlations:

cLOS		MON and dLOS

Performance monitoring: None.

# 13	T21 section layer functions

Figure 71 – T21 section atomic functions

T21 layer CP

The characteristic information T21_CI of the intra-station electrical layer CP is an electrical 6312 kHz clock signal of defined amplitude, frequency and pulse shape specified by Appendix II of [ITU-T G.703].

T21 layer AP

The information passing across the T21/SD AP is a 6312 kHz synchronization signal.

## 13.1	T21 connection function T21_C

Not applicable.

## 13.2	T21 trail termination functions

### 13.2.1	T21 trail termination source (T21_TT_So)

Symbol:

Figure 72 – T21_TT_So symbol

Interfaces:

Table 70 – T21_TT_So input and output signals

| Input(s) | Output(s) |
| --- | --- |
| T21_AI_CK T21_AI_SQLCH | T21_CI_CK |

Processes:

This function generates the electrical 6312 kHz clock signal used for transmission of synchronization signals to an external equipment on a plesiochronous intra-station section specified by Appendix II of [ITU-T G.703].

Pulse shape: The function shall meet the requirement specified by [ITU-T G.703].

Maximum peak voltage: The function shall meet the requirement specified by [ITU-T G.703].

Minimum peak voltage: The function shall meet the requirement specified by [ITU-T G.703].

Pair in each direction: The function shall meet the requirement specified by [ITU-T G.703].

Defects: None.

Consequent actions:

On activation of T21_AI_SQLCH, the function shall shut down the output; on clearing of T21_AI_SQLCH, the function shall output a normal signal.

Defect correlations: None.

Performance monitoring: None.

### 13.2.2	T21 trail termination sink (T21_TT_Sk)

For further study.

Annex A

Synchronization selection process

(This annex forms an integral part of this Recommendation.)

This annex specifies the detailed operation of the automatic synchronization reference selection process located in the NS_C function. Refer to clause 5 for an introduction to this process.

A selection process needs the quality level (NS_CI_QL) and signal fail (NS_CI_SSF) information from each input (i.e., each SD_TT_Sk and SD/NS_A_Sk combination; the pair is henceforth called a "timing source") after it has passed through a hold-off/wait-to-restore process (HO/WtR).

Via the management interface it receives the priority (which includes disabling) of each timing source and its lock-out status. Switch requests (clear, manual, forced) and requests to change the mode of operation between QL-enabled and QL-disabled also come in through the management interface.

The output of the selection control process is the currently selected input ("select q") and its QL. The currently selected input is reported towards the management interface. In addition, rejection messages are sent towards the management interface. Figure A.1 shows the interfaces between the selection control process and its environment.

Figure A.1 – Environment of selection control process

The selector supports M inputs (1 to M) and one output. In addition, the selector has a M+1th input, the null (0) input connected to an "unconnect" signal generator process.

Unconnect signal: A signal where CI_CK is undefined, CI_SSF is true and CI_CS is "none".

In the SDL diagrams that describe the selection process, there are six states which correspond to the two modes of operation [QL-mode enabled (1) and QL-mode disabled (2)] and, within each of these modes, three "maintenance" states: no request active (A), manual switch active (B) and forced switch active (C). For each of these six states, the reaction to all possible input variations is given. SDL notation is summarized in Appendix V. For detailed information on SDL, see [ITU-T Z.100].

In the SDL diagrams "report q" is MI_selected input information and QL_out equals CI_CL, as defined in Figure A.1.

Table A.1 – Notational conventions and parameters used in the SDL diagrams

| Parameter | Abbreviation | Values (high to low)/Explanation |
| --- | --- | --- |
| Quality level [input] | QL[p] | Option I: ePRTC, PRTC, ePRC, PRC, SSU-A, SSU-B, eSEC, SEC, DNU, INV, INVx, FAILED, undef(ined) |
| *) The position of QL-PROV in the QL order in option II networks may be changed via MI_OptII_QL-PROV_Priority. |  | Option II: ePRTC, PRTC, ePRC, PRS, STU, ST2, TNC, ST3E, ST3, SMC, ST4, PROV*, DUS, INV, FAILED, undef(ined) |
|  |  | Option III: ePRTC, PRTC, UNK, eSEC, SEC, INVx, FAILED, undef(ined) |
| Quality level [0] | QL[0] | UNConnected |
| Priority [input] | Pr[p] | 1, 2, ...., K, dis(abled) |
| Priority [0] | Pr[0] | Undef(ined) |
| Signal fail [input] | SF[p] | False, true |
| Signal fail [0] | SF[0] | True |
| Lock out status [input] | LO[p] | On, off |
| Lock out status [0] | LO[0] | Off |
| Input | p | 1, 2, ..., M |
| Selected input | q | 0,1, 2, ..., M |
| Number of timing sources | M |  |
| Number of different priorities | K | K = M |
|  | := | Assignment symbol |
|  | == | Equality test symbol |
|  | <= | Less or equal test symbol |
| NOTE – The QLs DNU and DUS are represented as DNU in the following figures. |  |  |

Figure A.2 – QL-enabled mode, no active switch request
(state 1A)

Figure A.3 – QL-enabled mode, active manual switch request
(state 1B)

Figure A.4 – QL-enabled mode, active forced switch
(state 1C)

Figure A.5 – QL-disabled mode, no external switch request active
(state 2A)

Figure A.6 – QL-disabled mode, active manual switch request
(state 2B)

Figure A.7 – QL-disabled mode, active forced switch request
(state 2C)

Figure A.8 – Continuations of previous states

Figure A.9 – Continuations of previous states

Figure A.10 – Synchronization selection algorithms 
for QL mode enabled (1) and QL mode disabled (2)

Appendix I

Transport layer models for synchronization information

(This appendix does not form an integral part of this Recommendation.)

This appendix shows the interfaces (sink and source), between NNI and SD_CP, that are able to transport synchronization information using the atomic functions described in [ITU-T G.783], [ITU-T G.798], [ITU-T G.705], and [ITU-T G.8021].

NOTE – The figures of this appendix do not include all details of functional models, which are described by the relevant clauses of this recommendation and other recommendations.

Figure I.1 – Synchronization transport port models: Station clock inputs

Figure I.2 – Synchronization transport port models: Station clock outputs

Figure I.3 – Synchronization transport port models: Traffic (line and tributary) inputs

Figure I.4 – Synchronization transport port models: Traffic (line and tributary) inputs

Figure I.5 – Synchronization transport port models: Traffic (line and tributary) outputs

Figure I.6 – Synchronization transport port models: Traffic (line and tributary) outputs

Appendix II

Examples of synchronization functionality in the NE

(This appendix does not form an integral part of this Recommendation.)

## II.1	Examples of synchronization functionality in the NE for option I networks

Figure II.1 – Example 1 of a network element's synchronization 
distribution layer functional model

Figure II.1 presents an example of the SD layer functionality within a network element providing SEC quality timing. The NE in the example offers four timing ports that can be connected to the transport ports carrying synchronization information, selected from the set of line and tributary transport ports and/or station clock ports within the NE.

Output B of NS_C function may use all four input signals to select the best synchronization reference input signal. Output A should exclude input signals derived from station clock ports if output A is connected to the source of the station clock input signal (see clause 5.13.1); otherwise, output A may select from all four input signals. Both outputs select independently of each other an input out of the set of configured inputs for that particular output.

NOTE 1 – The correct provisioning is a responsibility of the user of the equipment.

The signal at output B of NS_C is connected to the system clock process (NS/SD-SEC_A_So). When it meets certain criteria it is used as a reference signal for the system clock process. Otherwise, the clock process will enter holdover.

The output signal of the system clock process is used to time the atomic functions inside the network element. In addition, it is output also via the station clock output dedicated to monitoring the internal clock signal.

The signal at output A of NS_C is connected to the station clock output.

Figure II.2 – Example 2 of a network element's synchronization 
distribution layer functional model

Figure II.2 presents a second example of the SD layer functionality within a network element providing SEC quality timing. The NE in the example offers two station clock timing ports, each of them being either of the type 2 MHz or 2 Mbit/s. These station clock-based synchronization reference input signals can both be connected to a timing port (SD_TT_Sk #1, #2).

Furthermore, the NE offers four timing ports that can be connected to a number of transport ports carrying synchronization information, selected from the set of line and tributary transport ports within the NE. Signals from timing ports within the range #3 to #6 that are not connected to a transport port will be disconnected in the NS_C function.

Output B of NS_C function may use all six input signals to select the best synchronization reference input signal. Output A should exclude input signals #1 and #2. Both outputs select independently of each other an input out of the set of configured inputs for that particular output.

The signal at output B of NS_C is connected to the system clock process (NS/SD-SEC_A_So). When it meets certain criteria, it is used as a reference signal for the system clock process. Otherwise, the clock process will enter holdover.

The output signal of the system clock process is used to time the atomic functions inside the network element. As an option, it can also be output via one or both station clock outputs. The latter option is used to support monitoring of the internal clock signal, or to provide a synchronization signal to, e.g., a small synchronous network element that is the last in the chain.

The signal at output A of NS_C is connected to the station clock output selector (SD_C). Depending on the application in the network, station clock outputs #1 and #2 can operate as a protection pair both sourced by the same input of SD_C, or as two independent outputs sourced by the same or different input signals (as appropriate for the application).

NOTE 2 – Figure II.2 presents two instances of SD_C functions (the first connected to SD_TT_Sk functions #1 and #2 and the second connected to SD_TT_Sk functions #3 to #6) to reflect explicitly the supported connectivity in the network element. The station clock input signals can be connected to timing ports 1, and 2 and not to timing ports 3 to 6. Similarly, the line and tributary input signals can be connected to timing ports 3 to 6, and not to timing ports 1 and 2.

## II.2	Examples of synchronization functionality in the NE for option II networks

For further study.

## II.3	Examples of synchronization functionality in the NE for option III networks

Figure II.3 – Example of a network element's synchronization
distribution layer functional model for option III network

Figure II.3 presents a third example of the SD layer functionality within a network element operating in an option III network.

In option III, the output C0 of NS_C function should use only two input signals derived from station clock ports (#C1 or #C2) to select the best synchronization reference input signal, and should not use input signals derived from each transport port.

The signal at output C0 of NS_C is connected to the system clock process (NS/SD-SEC_A_So). When it meets certain criteria, it is used as a reference signal for the system clock process. The output signal of the system clock process is used to time atomic functions inside the network element.

Output S0 of NS_C may use all of the input signals that are offered from transport ports (from #S1 to #Sn) to select the synchronization reference signal. The selection process to output S0 in NS_C function is only provided by the operator.

The signal at output S0 of NS_C is connected to the station clock output.

Appendix III

Delay time allocation for option I and III networks

(This appendix does not form an integral part of this Recommendation.)

## III.1	Delay and processing times for the synchronization selection process

The following delay and processing times are defined for an SEC using the QL-enabled mode for the reference selection process in Option I and III networks. They are based on a ring configuration with 20 NEs. Delay and processing times for other applications (e.g., SSU) are for further study.

Three delay time values are defined for the synchronization selection process of the SEC. These are the non-switching message delay TNSM, the switching message delay TSM and the holdover message delay THM. These times are measurable at the interfaces of the NE.

These delay times are caused by internal delay and processing times of the synchronization distribution atomic functions. The hold-off time th and processing time tp are part of the reference selection process of the NS_C function. The settling time ts is part of the SD/NS-SEC_A_So function. For a detailed description, see clause 6.

Figure III.1 – Example configuration for clock selection

The timing diagrams in the following description are based on a configuration with two clock reference sources as shown in Figure III.1.

The dotted functions in the figure do not contribute to delay and processing times. The persistence check for SSM acceptance in the XX/SD_A_Sk functions is also not considered in the following as it is small compared to the overall time.

## III.2	Non-switching message delay TNSM

This delay applies when the QL of the selected reference signal changes and the sync source is maintained. TNSM defines the maximum time between the change of the input QL and the change of the output QL (Figure III.2).

Figure III.2 – TNSM

TNSM is due to the processing time tp of the reference selection process in the NS_C function.

A maximum value of 200 ms is defined for TNSM.

TNSM = tp = 0 ms to 200 ms.

## III.3	Switching message delay TSM

This delay applies if a switch over to another reference source is performed with a different QL value. TSM defines the time between the triggering of the new selection (e.g., change of the QL of a reference, external command, etc.) and the change of the QL at the output (Figure III.3).

Figure III.3 – TSM

TSM is due to the processing time tp of the selection process in the NS_C function and the settling time ts of the oscillator in the SD/NS-SEC_A_So function.

A range of 180 ms to 500 ms is defined for TSM.

TSM = (tp + ts) = 180 ms to 500 ms.

## III.4	Holdover message delay THM

This delay applies when the SEC should enter a holdover mode due to a failure condition of the selected sync source and the unavailability of any other synchronization source. When this event occurs, the SEC goes immediately into holdover mode. The outgoing QL changes to QL_SEC after the time THM (Figure III.4).

Figure III.4 – THM

NOTE – The internal QL of NS_C after hold-off characterizes a signal located between the WTR block and the selection control process box defined in Figure A.1.

THM is due to the QL-FAILED (SSF) hold-off time th and the processing time tp of the selection process in the NS_C function.

A range of 300 ms to 2000 ms is defined for THM.

THM = th + tp = 300 ms to 2000 ms.

## III.5	Wait-to-restore time TWTR

The wait-to-restore time applies when a synchronization source signal recovers from a failure condition. This signal comes available only for the selection process after signal fail is cleared at least for the time TWTR (Figure III.5).

Figure III.5 – TWTR

NOTE – The internal QL of NS_C after WTR characterizes a signal located between the WTR block and the selection control process box defined in Figure A.1.

TWTR is implemented in the NS_C function. The definition of WTR is in clause 4.

Appendix IV

Interworking of option II equipment supporting second-generation SSM 
and first-generation SSM using a translation function

(This appendix does not form an integral part of this Recommendation.)

Translation between second generation (GEN2) and first generation (GEN1) SSM is necessary to ensure interoperability. Both the number of quality levels and the quality-level definitions differ between GEN2 and GEN1. However, the GEN1 SSMs are a subset of GEN2. The translation function is a provisionable option on a per-port basis (both STM-N and 1544 kbit/s station clock) in equipment supporting GEN2 SSM to provide acceptable SSM to equipment supporting GEN1 SSM. Figures IV.1 and IV.2 show examples of message flows between equipment supporting GEN2 and GEN1 SSM. In Figure IV.1, a NE supporting GEN2 SSM is co-located with a BITS supporting GEN1 SSM. In this case, the 1544 kbit/s station clock output port of the NE is provisioned to provide the translation from GEN2 to GEN1 SSM.

Figure IV.1 – Example of SSM translation between an option II NE (GEN2) 
and a BITS (GEN1)

In Figure IV.2, an SDH option II NE is co-located with a BITS supporting GEN2 SSM. In this case, the NE receives GEN1 SSM. In this case, there is no need for any translation provisioning and the 1544 kbit/s station clock output just passes the received SSM to the BITS.

Figure IV.2 – Example of SSM translation between
an option II NE (GEN2) and a BITS (GEN2)

Appendix V

SDL descriptions

(This appendix does not form an integral part of this Recommendation.)

In this Recommendation, detailed characteristics of equipment functional blocks are described with SDL diagrams specified in [ITU-T Z.100]. The SDL diagrams use the following conventions.

Figure V.1 – SDL symbols

Appendix VI

Clarification of the use of the acronym 'SEC'

(This appendix does not form an integral part of this Recommendation.)

In this Recommendation, the acronym 'SEC' stands for 'synchronous equipment clock' (see clause 4). In clause 3.6, 'synchronous equipment clock' is defined as "a generic term representing the SDH equipment clock ([ITU-T G.813]), the Ethernet equipment clock (EEC) ([ITU-T G.8262]), and the OTN equipment clock (OEC) ([ITU-T G.8262])."

In several other ITU-T Recommendations, e.g., [ITU-T G.813], [ITU-T G.783], the acronym 'SEC' stands for 'SDH equipment clock.' The usage in this Recommendation is more generic, as here the term includes the SDH Equipment clock, the Ethernet Equipment clock, and the OTN Equipment clock.

Finally, the quality level 'QL-SEC' designates a synchronization trail that transports a timing quality generated by a synchronous equipment clock that is defined in [ITU-T G.813] or [ITU-T G.8262], option I (see clause 5.4.1.1 for option I synchronization networking and clause 5.4.1.3 for option III synchronization networking). This quality level is used only in option I and option III synchronization networking, and applies only to the option I SEC.

Appendix VII

Use cases of mixing SSM and eSSM

(This appendix does not form an integral part of this Recommendation.)

While the eQL-TLV was developed for use with the eSEC, the basic mechanism could be applied to the SEC. This results in three possible combinations of clocks: eSEC with eQL-TLV support, SEC with no eQL-TLV support, and SEC with eQL-TLV support.

Therefore, for studying different cases of node combination, the node is categorized as: eQL-TLV support (eQL-TLV) and no eQL-TLV support (QL-TLV).

Figure VII.1 – Different cases for node combination

Different carriers have different strategies to deploy and upgrade their networks.

Case 1 in Figure VII.1:

All nodes are SEC without eQL-TLV support. This case is typical for legacy deployment as shown in Figure VII.2(a).

Case 2-Case5 in Figure VII.1:

At least one of the nodes supports eQL-TLV. It is mixing SSM and eSSM.

These cases are typical at the edge of networks and during network upgrades, as in Figure VII.2(b).

Case 6 in Figure VII.1:

All nodes support eQL-TLV. This case is typical for a greenfield scenario or in case of a fully upgraded network, as in Figure VII.2(c).

Figure VII.2 – Example of the network mixing QL-TLV and eQL-TLV

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series D | Tariff and accounting principles and international telecommunication/ICT economic and policy issues |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Environment and ICTs, climate change, e-waste, energy efficiency; construction, installation and protection of cables and other elements of outside plant |
| Series M | Telecommunication management, including TMN and network maintenance |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling, and associated measurements and tests |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks, open system communications and security |
| Series Y | Global information infrastructure, Internet protocol aspects, next-generation networks, Internet of Things and smart cities |
| Series Z | Languages and general software aspects for telecommunication systems |