ETSI TS 103 141 V0.0.4 (2022-05)

Intelligent Transportation Systems (ITS);

Facilities layer function;

Multi-Channel Operation (MCO) for Cooperative ITS (C-ITS);

Release 2

<

TECHNICAL SPECIFICATION

Reference

<Workitem>

Keywords

<keywords>

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-préfecture de Grasse (06) N° 7803/88

Important notice

The present document can be downloaded from:
http://www.etsi.org/standards-search

The present document may be made available in electronic versions and/or in print. The content of any electronic and/or print versions of the present document shall not be modified without the prior written authorization of ETSI. In case of any existing or perceived difference in contents between such versions and/or in print, the prevailing version of an ETSI deliverable is the one made publicly available in PDF format at www.etsi.org/deliver.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at https://portal.etsi.org/TB/ETSIDeliverableStatus.aspx.

If you find errors in the present document, please send your comment to one of the following services:
https://portal.etsi.org/People/CommiteeSupportStaff.aspx

Copyright Notification

No part may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm except as authorized by written permission of ETSI.

The content of the PDF version shall not be modified without the written authorization of ETSI.

The copyright and the foregoing restriction extend to reproduction in all media.

© ETSI yyyy.

All rights reserved.

DECTTM, PLUGTESTSTM, UMTSTM and the ETSI logo are trademarks of ETSI registered for the benefit of its Members.
3GPPTM and LTETM are trademarks of ETSI registered for the benefit of its Members and
of the 3GPP Organizational Partners.
oneM2M™ logo is a trademark of ETSI registered for the benefit of its Members and
of the oneM2M Partners.
GSM® and the GSM logo are trademarks registered and owned by the GSM Association.

Contents

Intellectual Property Rights	4

Foreword	4

Modal verbs terminology	4

Introduction	4

1.	Scope	5

2.	References	5

2.1	Normative references	5

2.2	Informative references	5

3.	Definition of terms, symbols and abbreviations	6

3.1	Abbreviations	6

4.	Facilities layer MCO functional operation	6

4.1	Introduction	6

4.2	Bandwidth Management entity operation	7

4.3	Message Handling entity operation	8

4.4	Message Collecting entity operation	9

5.	Interface Requirements	9

5.1	Introduction	9

5.2	MCO_APP_FAC_MMT	10

5.3	MCO_FAC_MS_DATA	11

Annex (informative): Change History	14

History	15

# Intellectual Property Rights

Essential patents

IPRs essential or potentially essential to normative deliverables may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (https://ipr.etsi.org).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

Trademarks

The present document may include trademarks and/or tradenames which are asserted and/or registered by their owners. ETSI claims no ownership of these except for any which are indicated as being the property of ETSI and conveys no right to use or reproduce any trademark and/or tradename. Mention of those trademarks in the present document does not constitute an endorsement by ETSI of products, services or organizations associated with those trademarks.

# Foreword

This Technical Specification (TS) has been produced by ETSI Technical Committee {ETSI Technical Committee|ETSI Project|<other>} <long techbody> (<short techbody>).

# Modal verbs terminology

In the present document "shall", "shall not", "should", "should not", "may", "need not", "will", "will not", "can" and "cannot" are to be interpreted as described in clause 3.2 of the ETSI Drafting Rules (Verbal forms for the expression of provisions).

"must" and "must not" are NOT allowed in ETSI deliverables except when used in direct citation.

# Introduction

The present document specifies the Facilities layer functionalities and interfaces of the Multi-Channel Operations (MCO) architecture specified in the ETSI TS 103 697 [i.1] with backward compatibility with C-ITS Release 1.

# Scope

The present document specifies the Facilities layer MCO functionalities of the Release 2 of ETSI specifications. It specifies the entities and its interfaces with protocols in relation with other MCO entities at other layers and with the applications and services at the Facilities and Application layers.

# References

## Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

Referenced documents which are not found to be publicly available in the expected location might be found at https://docbox.etsi.org/Reference.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long-term validity.

The following referenced documents are necessary for the application of the present document.

[1]	ETSI TS 103 836-4-1 V2.1.1: “Intelligent Transportation Systems (ITS); Communications; GeoNetworking: Part 4: Geographical addressing and forwarding for point-to-point and point-to-multipoint communications; Sub-part 1: Media-Independent Functionality; Release 2”

[2]	ETSI TS 103 695 V2.1.1: “Intelligent Transportation Systems (ITS); Access Layer specification for Multi-Channel Operation (MCO) in the 5GHz frequency band; Release 2”

## Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long-term validity.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	ETSI TS 103 697 V2.1.1: “Intelligent Transport Systems (ITS); C-ITS Communication Architecture for Multi-Channel Operation (MCO); Release 2”

[i.2]ETSI TS 103 696 V2.1.1: “Intelligent Transport Systems (ITS); Communication Architecture for Multi-Channel Operation (MCO); Release 2”

[i.3]ETSI TR 103 439 V2.1.1: “Intelligent Transport Systems (ITS); Multi-Channel Operation study; Release 2”

# Definition of terms, symbols and abbreviations

## Abbreviations

For the purposes of the present document, the following abbreviations apply:

AER					Application Error

BME					Bandwidth Management Entity

C-ITS					Cooperative Intelligent Transport Systems

CLR					Channel Load Ratio

FCL					Functional Configuration Limits

FCP					Functional Configuration Profile

MCI					MCO Control Information

MCO					Multi-Channel Operation

MCO_APP_FAC_MMT 	MCO Application Facilities Management interface

MCO_FAC					Multi-Channel Operation Facilities

MCO_FAC_MS_DATA	MCO Facilities Message Services interface

MCO_FAC_NET_MMT	MCO Facilities Networking Management interface

MCO_FAC_NET_DATA	MCO Facilities Networking Data interface

MCE					Message Collecting Entity

MGE					Message Generation Entity

MHE					Message Handling Entity

MRE					Message Receiving Entity

MTE					Message Triggering Entity

PCI					Protocol Control Information

# Facilities layer MCO functional operation

## Introduction

The Facilities layer MCO functional architecture is part of the MCO architecture as specified in ETSI TS 103 697 [i.1] based on general C-ITS architectural aspects defined in ETSI TS 103 696 [i.2] from which functional and technical aspects have been identified in the ETSI TR 103 439 [i.3].

The ETSI TS 103 697 [i.1] introduces the MCO services at the different layers and identifies the Facilities layer functionalities as depicted in Figure 1. The 3 functionalities of the MCO_FAC service is realized by 3 entities:

- Bandwidth Management Entity (BME): in charge of collecting application requirements, capabilities of the available radio interfaces and channels, sending feedback to applications to adapt the amount of information they generate based on collected information, errors and notifications, and configuring the lower layers to use the most adequate channels and radio access technologies taking into account the information collected.

- Message Handling Entity (MHE): in charge of the transmission of messages towards the Networking & Transport layer and configure their parameters on a per-message basis considering the capabilities of the available radio interfaces and channels.

- Message Collecting Entity (MCE): in charge of collecting received messages and forwarding them towards the corresponding message receiving entities.

The present document specifies these entities operation, and their relation with other Application layer and Facilities layer entities and interfaces. As identified in ETSI TS 103 697 [i.1] only interface aspects related to the lower layers are defined in related layer documents ETSI TS 103 836-4-1 [1] and ETSI TS 103 195 [2]. The yellow marked interfaces in Figure 1 are the interfaces specified in the present document. The white ones in the lower layer specifications.

Figure 1: MCO_FAC and main related entities at Facilities and Application Layers

## Bandwidth Management entity operation

The Bandwidth Management Entity (BME) is responsible for the collection of the communication requirements and parameters of the C-ITS applications and awareness services via the MCO_APP_FAC_MMT interface specified in clause 5.2 and may collect IPv6 protocol requests from the Networking & Transport layer using the MCO_FAC_NET_MMT interface as specified in ETSI TS 103 836-4-1.

The BME collects the communication requirements and parameters of the C-ITS applications and awareness services when these are activated and during the active state of the application when applicable. Alternatively, this information can be preconfigured for statically operating C-ITS applications and awareness services. The communication requirements and parameters are provided by means of Functional Configuration Profiles (FCPs), see Figure 2, which are composed by the following parameters:

- Preferred settings (e.g. channel, technology, message priority, validity time…)

- Alternative settings (e.g. channels, technologies, message priorities, validity time…)

- Resource requests (e.g. bits per second)

Each received set of communication requirements and parameters is associated to a unique identifier, an FcpID.

NOTE: A preferred channel is channel to which an application prefers to have its messages being disseminated. An alternative channel is a channel which can be used by MCO_FAC in case conditions are reached set by the FCP.

Additionally, as depicted in Figure 2, the BME is responsible for the collection of the lower layer communication capabilities and status notifications. It collects or receives this information through the MCO_FAC_NET_MMT interface from the start of operation till the moment of deactivation of the C-ITS. This information may include information such as the CLR (Channel Load Ratio).

The BME estimates the Functional Configuration Limits (FCL) for each of the active FcpIDs and informs the related C-ITS applications and awareness services of their dissemination possibilities. The BME calculates the maximum amount of available resources (e.g. kilobits per second) for each FcpIP on each channel, and identifies related settings. The calculated limits are provided as part of the FCL via the MCO_APP_FAC_MMT interface to the corresponding application or awareness service and MCO_FAC internally to the MCO Message Handling Entity (MHE), as specified in clause 5.2.

Based on the collected C-ITS application or awareness service communication requirements and parameters, and the collected lower layer capabilities, the BME shall configure the channel and radio access technology at the lower layers by using the MCO_FAC_NET_MMT interface.

Figure 2: Bandwidth management

## Message Handling entity operation

All active C-ITS applications and awareness services that make use of MCO send message generation requests by providing their application data to the applicable MGEs, which in turn generates message handling requests to the Message Handling Entity (MHE) for the dissemination of the messages. Each message handling request sent to the MHE is associated with an FcpID, as shown in Figure 3. The MHE is constantly updated with the latest information related to the FCLs by the BME as specified in 4.2 via the management plane.

When the MHE receives a message handling request from the MGE, the MHE verifies whether it complies to the conditions provided by the BME in the corresponding FCL. If this verification is positive, the MHE disseminates the message to the lower layer via the MCO_FAC_NET_DATA interface according to the current dissemination possibilities identified by the BME. The message is disseminated to the lower layers via the MCO_FAC_NET_DATA interface.

Figure 3: Message handling

If the message handling request received by the MHE from the MGE does not comply to the corresponding FCL, the MHE may send a Functional Error (FER) to the BME for evaluation, which may in turn provide these FERs to the Application via the MCO_APP_FAC_MMT interface, as shown in Figure 3. The MRE may still forward the message to the lower layers with an alternate parameter setting if allowed by the associated FCP.

In case an IPv6 data packet exchange is provided by the lower layer, an IPv6 routing request is sent by the lower layers via the MCO_FAC_NET_MMT interface to the BME. The BME assess the situation and identifies whether this can be granted or not. In case this can be granted, the BME enables the IPv6 transfer by sending the appropriate parameter settings via MCO_FAC_NET_MMT interface to the lower layers. The MHE is not involved in this process.

## Message Collecting entity operation

The Message Collecting Entity (MCE) collects the messages received from the lower layers through the MCO_FAC_NET_DATA interface and provides the messages to the appropriate message service (MRE) through the MCO_FAC_MS_DATA interface as depicted in Figure 4.

Figure 4: Message reception from lower layer

# Interface Requirements

## Introduction

As identified in the ETSI TS 103 697 [i.1] and in clause 4 and as illustrated in Figure 5, the MCO_FAC has the following interfaces MCO_APP_FAC_MMT, MCO_FAC_MS_DATA, MCO_FAC_NET_MMT and MCO_FAC_NET_DATA. The present document specifies MCO_APP_FAC_MMT and MCO_FAC_MS_DATA interfaces in the following sub-clauses. The MCO_FAC_NET_MMT and MCO_FAC_NET_DATA interfaces are specified in ETSI TS 103 836-4-1.

Figure 5: The MCO_FAC interfaces

## MCO_APP_FAC_MMT

Applications may define their communication requirements and default parameters to the MCO_FAC based on the data format as presented in Table 1. Each set of communication requirements and parameters is associated to an FcpID.

Table 1: Parameters for the MCO_APP_FAC_MMT interface: requirements

| M-Param.No | Name of M-Param | Access | Format | Description | Status |
| --- | --- | --- | --- | --- | --- |
| 0 | FcpID | R | INTEGER | Identifier of the functional configuration profile. If FcpID is 0, the command defines the requirements of a new FCP. If FcpID is equal to the ID of an active FCP, the command describes the new requirements of the FCP. | Mandatory |
| 1 | resources | R | INTEGER | Indicates the amount of resources in kilobits per second that would be consumed at the Facilities layer by the FcpID. | Optional |
| 2 | range | R | INTEGER | Intended minimum one-hop transmission range in meters | Optional |
| 3 | validity_time | R | INTEGER | Maximum packet lifetime in milliseconds | Optional |
| 4 | num_channels | R | INTEGER | Number of channels for which information is attached. | Optional |
| 5 | channel_list | R | INTEGER STRING of length num_channels | List of channels in order of preference for the transmission of the messages. The first one is the preferred channel, and the rest are alternate channels. | Conditional (if num_channels > 0) and optional |
| 6 | technology_list | R | INTEGER STRING of length num_channels | List of radio access technologies in order of preference for the transmission of the messages. The first one is the preferred technology, and the rest are alternate ones. | Conditional (if num_channels > 0) and optional |
| 7 | priority_list | R | INTEGER STRING of length num_channels | List of priorities of the message when transmitted in each of the channels indicated in the channel_list; the priorities indicated in this field for each channel do not affect the order of preference of the channels. | Conditional (if num_channels > 0) and optional |
| 8 | criteria_list | R | INTEGER STRING of length num_channels | List of criteria by which MCO can decide to not use the specified channel for which the criteria is set for. The default criteria is congestion level reached. | Conditional (if num_channels > 0) and optional |
| 9 | saturation_indicator | R | 1 bit 0 = discard 1 = use any other channel | Indicates if, in the case that a message cannot be send through any of the channels in channel_list, that message should be dropped or can be sent through any of the other channels | Conditional (if num_channels > 0) and optional |

The MCO_FAC shall inform each application about the maximum amount of resources it can use associated to the corresponding FcpID using the MCO_APP_FAC_MMT interface and the parameters in Table 2.

Table 2: Parameters for the MCO_APP_FAC_MMT interface: limits

| M-Param.No | Name of M-Param | Access | Format | Description | Status |
| --- | --- | --- | --- | --- | --- |
| 0 | FcpID | W | INTEGER | Identifier of the functional configuration profile | Mandatory |
| 1 | resources_limit | W | INTEGER STRING of length num_channels | Indicates the maximum amount of resources in kilobits per second that can be consumed at the Facilities layer by the FcpID in each of the channels indicated in the channel_list. | Conditional (if num_channels > 0) and optional |
| 2 | range_limit | W | INTEGER STRING of length num_channels | Indicates the maximum one-hop transmission range in meters that can be provided to the FcpID in each of the channels indicated in the channel_list | Conditional (if num_channels > 0) and optional |
| 3 | validity_time_limit | W | INTEGER STRING of length num_channels | Indicates the maximum packet latency in milliseconds that can be provided to the FcpID in each of the channels indicated in the channel_list | Conditional (if num_channels > 0) and optional |
| 4 | num_channels | W | INTEGER | Number of channels for which information is attached. | Optional |
| 5 | channel_list | W | INTEGER STRING of length num_channels | List of channels in order of preference for the transmission of the messages. The first one is the preferred channel, and the rest are alternate channels. | Conditional (if num_channels > 0) and optional |
| 6 | technology_list | W | INTEGER STRING of length num_channels | List of radio access technologies in order of preference for the transmission of the messages. The first one is the preferred technology, and the rest are alternate ones. |  |
| 7 | priority_list | W | INTEGER STRING of length num_channels | List of priorities of the message when transmitted in each of the channels indicated in the channel_list; the priorities indicated in this field for each channel do not affect the order of preference of the channels. | Conditional (if num_channels > 0) and optional |

The MCO_FAC may also notify the corresponding application that a given message handling request associated to a given FcpID was not satisfied, using the parameters in Table 3.

Table 3: Parameters for the MCO_APP_FAC_MMT interface: notification

| M-Param.No | Name of M-Param | Access | Format | Description | Status |
| --- | --- | --- | --- | --- | --- |
| 0 | FcpID | W | INTEGER | Identifier of the functional configuration profile | Mandatory |
| 1 | FER (Functional Error) | W | INTEGER 0 = dropped 1 = sent through alternate channel 1 2 = sent to alternate channel 2 … 255 = sent using a different channel | Indicates that the requirements of a message handling request associated to the FcpID could not be satisfied and the message was either dropped or sent through a channel different than the preferred channel. | Mandatory |

## MCO_FAC_MS_DATA

MGEs shall send to MCO_FAC a message handling request with the information in Table 4 for the transmission of each message. The message shall be embedded in a Facility-layer message together with the protocol control information (PCI) required to be passed to the ITS Networking & Transport Layer. MGEs may attach MCO control information (MCI) as shown in Table 4 and specified in Table 5, to override the default parameters of the FcpID for the specific message handling request.

Table 4: Parameters for the MCO_FAC_MS_DATA interface: message handling request

| Data | Data requirement | Mandatory/Optional |
| --- | --- | --- |
| Facility-layer message | As specified in the corresponding ASN1 description | Mandatory |
| MCI | MCO control information used by MCO_FAC to handle the message | Conditional (if MCO is used and default parameters are not defined in the FcpID or want to be override) |
| PCI | Protocol control information that depends on the protocol stack applied in the networking and transport layer | Mandatory |

Table 5: MCO control information (MCI)

| M-Param.No | Name of M-Param | Access | Type | Description | Status |
| --- | --- | --- | --- | --- | --- |
| 0 | FcpID | R | INTEGER | Identifier of the functional configuration profile | Mandatory |
| 1 | message_length | R | INTEGER | Message length in bytes | Mandatory |
| 2 | validity_time | R | INTEGER | Maximum packet lifetime in milliseconds | Optional When included, the default parameters of the FcpID are overwritten for the specific message handling request. |
| 3 | range | R | INTEGER | Intended minimum one-hop transmission range in meters | Optional When included, the default parameters of the FcpID are overwritten for the specific message handling request. |
| 4 | num_channels | R | INTEGER | Number of channels for which information is attached. | Optional When included, the default parameters of the FcpID are overwritten for the specific message handling request. |
| 5 | channel_list | R | INTEGER STRING of length num_channels | List of channels in order of preference for the transmission of the message. The first one is the preferred channel, and the rest are alternate channels. | Conditional (if num_channels > 0) and optional When included, the default parameters of the FcpID are overwritten for the specific message handling request. |
| 6 | technology_list | R | INTEGER STRING of length num_channels | List of radio access technologies in order of preference for the transmission of the messages. The first one is the preferred technology, and the rest are alternate ones. | Conditional (if num_channels > 0) and optional When included, the default parameters of the FcpID are overwritten for the specific message handling request. |
| 7 | priority_list | R | INTEGER STRING of length num_channels | List of priorities of the message when transmitted in each of the channels indicated in the channel_list; the priorities indicated in this field for each channel do not affect the order of preference of the channels. | Conditional (if num_channels > 0) and optional When included, the default parameters of the FcpID are overwritten for the specific message handling request. |
| 8 | criteria_list | R | INTEGER STRING of length num_channels | List of criteria by which MCO can decide to not use the specified channel for which the criteria is set for. The default criteria is congestion level reached. | Conditional (if num_channels > 0) and optional When included, the default parameters of the FcpID are overwritten for the specific message handling request. |
| 9 | saturation_indicator | R | 1 bit 0 = discard 1 = use any other channel | Indicates if the message should be dropped or sent through any other channel if it is not possible to send it through any of the channels in channel_list | Conditional (if num_channels > 0) and optional When included, the default parameters of the FcpID are overwritten for the specific message handling request. |

Annex (informative):
Change History

| Date | Version | Information about changes |
| --- | --- | --- |
| 2021-06-28 | 0.0.1 | Skeleton of the document |
| 2022-01-15 | 0.0.2 | Early draft with definition of MCO_FAC entity |
| 2022-03-15 | 0.0.3 | Stable draft with all clauses completed |
| 2022-05-23 | 0.0.4 | Final draft |

# History

| Document history |  |  |
| --- | --- | --- |
| <Version> | <Date> | <Milestone> |

Latest changes made on 2022-01-12