ETSI TS 103 695 V0.0.8 (2022-05)

Intelligent Transport System (ITS);

Access layer specification in the 5GHz frequency band;

Multi-Channel Operation (MCO) for Cooperative ITS (C-ITS);

Release 2

<

TECHNICAL SPECIFICATION

Reference

DTS/ITS-004204

Keywords

Multi-Channel Operation (MCO), ITS, Access Layer

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-préfecture de Grasse (06) N° 7803/88

Important notice

The present document can be downloaded from:
http://www.etsi.org/standards-search

The present document may be made available in electronic versions and/or in print. The content of any electronic and/or print versions of the present document shall not be modified without the prior written authorization of ETSI. In case of any existing or perceived difference in contents between such versions and/or in print, the prevailing version of an ETSI deliverable is the one made publicly available in PDF format at www.etsi.org/deliver.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at https://portal.etsi.org/TB/ETSIDeliverableStatus.aspx.

If you find errors in the present document, please send your comment to one of the following services:
https://portal.etsi.org/People/CommiteeSupportStaff.aspx

Copyright Notification

No part may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm except as authorized by written permission of ETSI.

The content of the PDF version shall not be modified without the written authorization of ETSI.

The copyright and the foregoing restriction extend to reproduction in all media.

© ETSI 2022.

All rights reserved.

DECTTM, PLUGTESTSTM, UMTSTM and the ETSI logo are trademarks of ETSI registered for the benefit of its Members.
3GPPTM and LTETM are trademarks of ETSI registered for the benefit of its Members and
of the 3GPP Organizational Partners.
oneM2M™ logo is a trademark of ETSI registered for the benefit of its Members and
of the oneM2M Partners.
GSM® and the GSM logo are trademarks registered and owned by the GSM Association.

Contents

Intellectual Property Rights	4

Foreword	4

Modal verbs terminology	4

Introduction	4

1	Scope	4

2	References	5

2.1	Normative references	5

2.2	Informative references	5

3	Definition of terms, symbols and abbreviations	5

3.1	Terms	5

3.2	Symbols	6

3.3	Abbreviations	6

4	Background	7

5	Access layer MCO operation	7

5.1	Introduction	7

5.2	Access layer data flow	8

5.3	Access layer management flow	8

6	MCO Access layer extension	9

6.1	Introduction	9

6.2	Access layer instance Identifier (AliID)	10

6.3	Active ALI groups	10

6.4	MCO_ACC Interfaces	11

6.4.1	Overview	11

6.4.2 	Input parameters from higher layers	11

6.4.3	Output parameter to higher layers	11

Annex A (normative): AliID reliability indicator for ITS-G5	12

Annex B (normative): AliID reliability indicator for enhanced ITS-G5	12

Annex C (normative): AliID reliability indicator for LTE-V2X Rel14 and Rel15	13

Annex D (normative): AliID reliability indicator for 5G-NR V2X	14

Annex E (informative): Output and inputs formats	15

E.1 	Input parameters from higher layers	15

E.2	Output parameter to higher layers	15

Annex F (informative): Change History	16

History	17

# Intellectual Property Rights

Essential patents

IPRs essential or potentially essential to normative deliverables may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (https://ipr.etsi.org).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

Trademarks

The present document may include trademarks and/or tradenames which are asserted and/or registered by their owners. ETSI claims no ownership of these except for any which are indicated as being the property of ETSI, and conveys no right to use or reproduce any trademark and/or tradename. Mention of those trademarks in the present document does not constitute an endorsement by ETSI of products, services or organizations associated with those trademarks.

# Foreword

This Technical Specification (TS) has been produced by ETSI Technical Committee Intelligent Transportation Systems (ITS).

# Modal verbs terminology

In the present document "shall", "shall not", "should", "should not", "may", "need not", "will", "will not", "can" and "cannot" are to be interpreted as described in clause 3.2 of the ETSI Drafting Rules (Verbal forms for the expression of provisions).

"must" and "must not" are NOT allowed in ETSI deliverables except when used in direct citation.

# Introduction

ETSI EN 302 663 [i.1] specifies the access layer for ITS-G5 for Release 1 and EN 302 613 [i.2] specifies the access layer for LTE-V2X for Release 1

The present document provides Release 2 C-ITS access layer updates and extensions specific to Multi-Channel Operation (MCO).

Where possible the specifications are access layer agnostic.

# 1	Scope

The present document specifies Release 2 C-ITS access layer MCO extensions based on Release 1 backward compatibility.

# 2	References

## 2.1	Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

Referenced documents which are not found to be publicly available in the expected location might be found at https://docbox.etsi.org/Reference.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long-term validity.

The following referenced documents are necessary for the application of the present document.

## 2.2	Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long-term validity.

The following referenced documents are not necessary for the application of the present document, but they assist the user with regard to a particular subject area.

[i.1]EN 302 663: “Intelligent Transport Systems (ITS); ITS-G5 Access layer specification for Intelligent Transport Systems operating in the 5 GHz frequency band”

[i.2]EN 302 613: “Intelligent Transport Systems (ITS); LTE-V2X Access layer specification for Intelligent Transport Systems operating in the 5 GHz frequency band”

[i.3]ETSI TS103 697: “Intelligent Transport System (ITS); Architecture; Multi-Channel Operation (MCO) for Cooperative ITS (C-ITS); Release 2

[i.4]ETSI TS136 213 v15.15.0: “LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures (3GPP TS 36.213 version 15.15.0 Release 15)”

[i.5]ETSI TS138 214 v17.1.0: “5G; NR; Physical layer procedures for data (3GPP TS 38.214 version 17.1.0 Release 17)”

[i.6]	IEEE 802.11bd: ”P802.11bd - Standard for Information technology--Telecommunications and information exchange between systems Local and metropolitan area networks--Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications Amendment: Enhancements for Next Generation V2X

[i.7]	IEEE802.11-2020:” IEEE Standard for Information technology—Telecommunications and information exchange between systems Local and metropolitan area networks—Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications”

# 3	Definition of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the following terms apply:

Access layer instance: fully parametrized (service channel, technology, TX power, …) transceiver entity in the access layer

Access layer instance ID: Unique Identifier for an access layer instance

Access layer instance group: A set of access layer instances using the same service channel and access layer technology.

Access layer instance Group ID: Unique Identifier for an access layer instance group

Access layer technology: A consistent set of protocols covering the physical layer and link layer functionalities of the ISO/OSI model defined in an independent specification or set of specifications.

AliID generation: process of extracting the AliID from a received frame

AliID processing: The operation of receiving an AliID from higher layers and configuring the transceiver correspondingly.

Instantiation: Process of parametrizing an entity.

Instance: Fully parametrized entity.

Channel load ratio: channel load value per ALI group evaluated at the receiving C-ITS-S.

Cooperative-Intelligent Transport System: ITS system that realise applications bringing awareness of road user behaviour by sharing static and dynamic information of its behaviour and recognized behaviour of others based on a wireless communication system. These applications form a subset of ITS applications.

Enhanced ITS-G5: ITS-G5 based access layer supporting the functionality provided by the IEEE802.11bd [i.6] and IEEE802.11p [i.7] access layer specification.

Entity: Singular, identifiable, and separate object realising a function or set of functions. It refers to individuals, organizations, systems, bits of data, or even distinct system components that are considered significant in and of themselves.

Function: Self-contained module that accomplishes a specific task. Functions usually "take in" data, process it, and "return" a result.

Intelligent Transport System (ITS): System which aims to provide innovative services relating to different modes of transport and traffic management and enable users to be better informed and make safer, more coordinated, and “smarter” use of transport networks.

Plane: In a networking context, it is a logical grouping of functions over different layers. The main planes recognized are the management or control plane, the data plane, and the security plane.

Subchannel: technology dependent group of contiguous physical resource blocks (PRBs) representing the minimum granularity for the allocation of a packet in the frequency domain

## 3.2	Symbols

For the purposes of the present document, the [following] symbols [given in ... and the following] apply:

## 3.3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

ALI	Access layer instance

AliID	Access layer instance identifier

AliGroupID	Access layer instance group identifier

ALN	Access Layer Notification

CC	Convolutional Coder

CL	Channel Load

CLR	Channel Load Ratio

C-ITS	Cooperative Intelligent Transport System

C-ITS-S	Cooperative Intelligent Transport System Station

C-ITS-U	Cooperative Intelligent Transport System User

CAM	Cooperative Awareness Message

CCAM	Cooperative, Connected, and Automated Mobility

DCC	Decentralized Congestion Control

E-CALL	Emergency-Call

EU	European Union

ICT	Information Communication Technology

ITS	Intelligent Transport Systems

LDPC	Low Density parity check codes

MCO	Multi-Channel Operation

RX	Receiver

SAP	Service Access Point

TRX	Transceiver

TX	Transmitter

TC	Turbo-Code

V2I	Vehicle to Infrastructure

V2V	Vehicle to Vehicle

V2X	Vehicle to Everything (all transport equipment used or implemented in or by, vehicle, truck, PTW, bike, pedestrian, train, ship, and other transport equipment.)

Note: The abbreviations AliID and AliGroupID are compliant with an ASN.1 representation of an identifier.

# 4	Background

An access layer implementation of a C-ITS-S capable of handling more than a single channel simultaneously requires specific multi-channel functionalities. These functionalities are needed to allow the higher layers to take efficiently advantage of the available resources to transmit and receive packets over the air. An access layer supporting multiple channels will have to implement more than one transceiver including all required building blocks like channel coding, multiplexing, potential input queues, channel load estimation entity and power amplifier. Each of these transceivers may be switched or configured to different operational channels and may be parameterized to deploy different combinations of the available access layer resources.

For proper multichannel operation of a C-ITS-S the different possible configurations of the transceivers need to be accessible to the higher layer entities for the control of the channel load and the related transmission properties.

# 5	Access layer MCO operation

## 5.1	Introduction

The access layer of an MCO capable C-ITS-S implements and operate more than a single transceiver active simultaneously. This set of transceivers may be capable of implementing different access layer technologies (e.g., ITS-G5, LTE-V2X, etc.). Each of which may be parameterized to be adapted to the requirements of the applications and the related message dissemination. A single transceiver may be instantiated using a set of different access layer parameters. These parameters are:

- Access layer technology (ITS-G5, LTE-V2X, 5G-NR-V2X, …)

- Used physical channel (SCH0, …, SCH6)

- Modulation and coding scheme (MCS) or reliability indicator

  - Channel coder (CC, TC, LDPC, ...)

  - Channel Coding rate

  - Modulation scheme

- TX power

A parametrized transceiver called “Access Layer Instance” (ALI) represents a specific access layer instantiation of a transceiver. The ALI implements the access layer technology (ITS-G5, LTE-V2X, 5G-NR), the used channels, the channel coding, the modulation and the transmit power. The ALI may be changed from message-to-message dissemination.

A specific set of parameters is identifiable by a unique access layer instance identifier, AliID.

The combination of the access layer technology and the used physical channel is called “Access Layer Instance Group” (ALI group). Each ALI group is identified by a unique access layer instance group identifier, AliGroupID.

The data flow and management flow of the MCO capable access layer are depicted in Figure 1, Figure 2 and Figure 3 and are described in the following subclauses.

## 5.2	Access layer data flow

The access layer data plane as depicted in Figure 1 provides all functionalities required to transmit a PPDU over the physical channel defined as part of the AliID. In the presented figure it is assumed that the set of transceivers may be flexibly parametrized to provide different access layer technology functionalities, thus may be instantiated as technology agnostic transceivers based on the actual AliID.

Another representation of an MCO technology agnostic C-ITS-S is depicted in Figure 2, where the different access layer technologies are limited to a technology specific set of transceivers. Here, an access layer instance may only run on a transceiver that may implement the access layer technology associated to that ALI.

Figure 1: MCO Access layer, Data flow with full flexible transceiver design

Figure 2: MCO Access layer, Data flow with separate set of transceiver design

## 5.3	Access layer management flow

As identified in the ETSI TS 103 697 [i.3] at the management plane there are two Access layer entities as part of MCO_ACC depicted in Figure 3.

Figure 3: MCO Access layer Management Plane, combined and separate TRXs

ALI Operations: In case higher layers provide an AliID to the ALI Operations, this entity shall configure the corresponding transceivers. The used channel and the access layer technology (ALI group) are semi dynamic characteristics whereas the other parameters (MCS, TX power) may change on a message-by-message basis. Semi dynamic means that these parameters need a longer configuration time. The actual required configuration time for the semi dynamic characteristics is implementation dependent. The ALI operation entity shall provide the actual state of all transceivers to the higher layer.

TRX Info Collection: Transceiver information collection entity; this entity collects all status and measurement information from the available transceivers and provide this information to higher layer. This information are the access layer notifications per transceiver (ALN) and the channel load ratio per transceiver (CLR)

As part of the TRX Info Collection, an access layer notifications (ALN) function collects the communication conditions flags and provide them to higher layers. The ALN provides information about the following events at the access layer related to the delivery of the data frames:

  - Queue overflow

  - Drop of packets

  - Time out of packets

The generated notifications shall be specific for a data frame. The ALN shall be provided to the Networking & Transport layer.

As part of the TRX Info Collection, a received AliID generation function shall generate an AliID for each received data frame based on the access layer information contained in the physical layer control of the frame. The generated AliID shall be provided to the higher layers. The format of the received AliID shall be as defined in clause 6.2.

As part of the TRX Info Collection, the Channel load ratio (CLR) measurement function estimates the actual channel load ratio for a specific ALI group for a given technology. This function is not MCO specific and thus will not be further specified in the present document. This technology specific ratio is as such provided to higher layers.

The AliID for the frames to be transmitted and the AliID of the received frames may be transferred between the layers as part of the data packets. This would simplify the required synchronization between the configuration information contained in the AliID and the related data frame.

# 6	MCO Access layer extension

## 6.1	Introduction

An MCO enabled access layer needs to support certain functionalities to permit the distribution of the C-ITS traffic onto the available physical channels with a specific set of parameters as defined by the AliID. These extensions shall be fully backward compatible with the Release 1 devices.

## 6.2	Access layer instance Identifier (AliID)

The access layer instance identifier (AliID) shall represent a unique identifier for the access layer instance. Based on the AliID it shall be possible to set all relevant parameters of a transceiver. A specific transceiver might not be capable of supporting all possible parameter combinations.

The AliID may consist of eight positions (1 letter and 8 decimal digit numerical positions):

- AliID contains a letter to identify the access layer technology, first position of the AliID:

  - A: ITS-G5;

  - B: enhanced ITS-G5 (11bd capable ITS-G5);

  - C: LTE-V2X;

  - D: 5G-NR-V2X.

- The first and second figure specifies the use physical channel, 2. and 3. position:

  - 00: SCH0;

  - 01: SCH1;

  - …

  - 06: SCH6;

  - 07: ….

NOTE: 	two figures are required to cover future extension like 60 GHz, 2570 MHz to 2620 MHz, channel bounding;

- Third and fourth figure reliability indicator (RID)

  - In the case of ITS-G5, the RID figure indicates directly the MCS, following Annex A, Table 1;

  - In the case of Enhanced ITS-G5, the RID figure indicates directly the MCS, following Annex B, Table 2;

  - In the case of LTE-V2X, the RID figure indicates the number of subchannels as detailed in Annex C Table 3, which implicitly indicates the RID to use given the message size;

  - In the case of 5G-NR-V2X, the MCS figure indicates the number of subchannels as detailed in Annex D Table 4, which implicitly indicates the MCS to use given the message size.

- The fifth figure specify the number of access layer repetitions: 0 - 9

- The sixth and seventh figure specifies the TX power:

  - 00: 0dBm

  - 01: 3dBm

  - 02: 6dBm

  - ..

  - 07: 21dBm

  - 08: 24dBm

  - ..

  - 99: not known

NOTE: 	In the AliID corresponding to a received message, the power level is set to 99 (not known).

- The eighth figure may be used for specific configurations

  - 0: received message.

  - 1: standard configuration.

## 6.3	Active ALI groups

A transceiver may be switched between the different ALI groups. This switching process may take some time and thus could lead to a significant delay. A transceiver already active on an ALI group may be easily configured with the MCS/RDI and TX power parameters on a message-by-message basis.

Thus, two different sets of ALI parameters exist, highly dynamically configurable parameters (MCS, TX power, …) and quasi static parameters (channel, technology: ALI group). For the higher layers, an information about the available ALI groups shall be provided. To capture this behaviour and the related information flow to higher layers different states of an ALI group are defined:

- 0: Not Active

- 1: RX only active

- 2: TX/RX active

- 3: not available

The actual state of an ALI group shall be set by higher layers. The setting of the ALI group state may be signaled to higher layers.

## 6.4	MCO_ACC Interfaces

### 6.4.1	Overview

The MCO_ACC entity shall support the MCO related interfaces illustrated in Figure 1 to Figure 3. All MCO related specified interfaces in the present document are station internal only. The detailed format of the parameters to be exchanged are implementation specific and need not to be specified in the present document.

### 6.4.2 	Input parameters from higher layers

Table 1: List of M-Param for the ACC_NET_MMT interface

| M-Param.No | Name of M-Param | Access | Format | Description |
| --- | --- | --- | --- | --- |
| 0 | AliID | R | not relevant (internal parameter, see Annex E) | AliID per message to be transmitted to ALI operation entity |
| 1 | AliGroupIDStatus | R | not relevant (internal parameter, see Annex E) | Set the status of the available ALI groups in the access layer. - 0: Not defined - 1: activate RX only - 2: activate TX/RX |

## 6.4.3	Output parameter to higher layers

Table 1: List of M-Param for the ACC_NET_MMT interface

| M-Param.No | Name of M-Param | Access | Format | Description |
| --- | --- | --- | --- | --- |
| 2 | CLR | W | not relevant (internal parameter, see Annex E) | Channel load ratio per ALI group, to “higher layers every [100] ms: - Channel Load Ratio as value 0 to 1 with a resolution of 0.01 - The parameter shall contain the CLR value and the AliGroupID |
| 3 | ALN | W | not relevant (internal parameter, see Annex E) | Access layer notification per message to higher layers |
| 4 | AliID | W | not relevant (internal parameter, see Annex E) | AliID of received message |
| 5 | AliGroupIDStatus | W | not relevant (internal parameter, see Annex E) | Status of the available ALI groups in the access layer. - 0: Not Active - 1: RX only active - 2: TX/RX active |

Annex A (normative):
AliID reliability indicator for ITS-G5

Table 1: MCS for ITS-G5

| RID_Nr (figure 3 and 4 of AliID) | Modulation | Rate R | Channel Coder |
| --- | --- | --- | --- |
| 00 | BPSK | 1/2 | CC |
| 01 | BPSK | 3/4 | CC |
| 02 | QPSK | 1/2 | CC |
| 03 | QPSK | 3/4 | CC |
| 04 | 16-QAM | 1/2 | CC |
| 05 | 16-QAM | 3/4 | CC |
| 06 | 64-QAM | 2/3 | CC |
| 07 | 64-QAM | 3/4 | CC |

Annex B (normative):
AliID reliability indicator for enhanced ITS-G5

Table 1: MCS for enhanced ITS-G5

| RID_Nr (figure 3 and 4 of AliID) | Modulation | Rate R | Channel Coder |
| --- | --- | --- | --- |
| ITS-G5 compatible RIDs |  |  |  |
| 00 | BPSK | 1/2 | CC |
| 01 | BPSK | 3/4 | CC |
| 02 | QPSK | 1/2 | CC |
| 03 | QPSK | 3/4 | CC |
| 04 | 16-QAM | 1/2 | CC |
| 05 | 16-QAM | 3/4 | CC |
| 06 | 64-QAM | 2/3 | CC |
| 07 | 64-QAM | 3/4 | CC |
| Enhanced ITS-G5 RIDs |  |  |  |
| 08 | BPSK | 1/2 | LDPC |
| 09 | QPSK | 1/2 | LDPC |
| 10 | QPSK | 3/4 | LDPC |
| 11 | 16-QAM | 1/2 | LDPC |
| 12 | 16-QAM | 3/4 | LDPC |
| 13 | 64-QAM | 2/3 | LDPC |
| 14 | 64-QAM | 3/4 | LDPC |
| 15 | 64-QAM | 5/6 | LDPC |
| 16 | 256-QAM | 3/4 | LDPC |
| 17 | 256-QAM | 5/6 | LDPC |
| 18 - 22 | Reserved |  |  |
| 23 | BPSK with DCM | ½ | LDPC |

Annex C (normative):
AliID reliability indicator for LTE-V2X Rel14 and Rel15

Table 3: MCS for LTE-V2X [i.4]

| RID_Nr (figure 3 and 4 of AliID) | MCS Index | Modulation Order | TBS Index | Redundancy Version rvidx |
| --- | --- | --- | --- | --- |
| 00 | 0 | 2 | 0 | 0 |
| 01 | 1 | 2 | 1 | 0 |
| 02 | 2 | 2 | 2 | 0 |
| 03 | 3 | 2 | 3 | 0 |
| 04 | 4 | 2 | 4 | 0 |
| 05 | 5 | 2 | 5 | 0 |
| 06 | 6 | 2 | 6 | 0 |
| 07 | 7 | 2 | 7 | 0 |
| 08 | 8 | 2 | 8 | 0 |
| 09 | 9 | 2 | 9 | 0 |
| 10 | 10 | 2 | 10 | 0 |
| 11 | 11 | 4 | 10 | 0 |
| 12 | 12 | 4 | 11 | 0 |
| 13 | 13 | 4 | 12 | 0 |
| 14 | 14 | 4 | 13 | 0 |
| 15 | 15 | 4 | 14 | 0 |
| 16 | 16 | 4 | 15 | 0 |
| 17 | 17 | 4 | 16 | 0 |
| 18 | 18 | 4 | 17 | 0 |
| 19 | 19 | 4 | 18 | 0 |
| 20 | 20 | 4 | 19 | 0 |
| 21 | 21 | 6 | 19 | 0 |
| 22 | 22 | 6 | 20 | 0 |
| 23 | 23 | 6 | 21 | 0 |
| 24 | 24 | 6 | 22 | 0 |
| 25 | 25 | 6 | 23 | 0 |
| 26 | 26 | 6 | 24 | 0 |
| 27 | 27 | 6 | 25 | 0 |
| 28 | 28 | 6 | 26 | 0 |
| 29 | 29 | 6 | 30 | 1 |
| 30 | 30 | 6 | 31 | 2 |
| 31 | 31 | 6 | 33 | 3 |

Annex D (normative):
AliID reliability indicator for 5G-NR V2X

Table 4: MCS for 5G-Nr V2X [i.5]

| RID_Nr (figure 3 and 4 of AliID) | MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] |
| --- | --- | --- | --- |
| 00 | 0 | 2 | 120 |
| 01 | 1 | 2 | 157 |
| 02 | 2 | 2 | 193 |
| 03 | 3 | 2 | 251 |
| 04 | 4 | 2 | 308 |
| 05 | 5 | 2 | 379 |
| 06 | 6 | 2 | 449 |
| 07 | 7 | 2 | 526 |
| 08 | 8 | 2 | 602 |
| 09 | 9 | 2 | 679 |
| 10 | 10 | 4 | 340 |
| 11 | 11 | 4 | 378 |
| 12 | 12 | 4 | 434 |
| 13 | 13 | 4 | 490 |
| 14 | 14 | 4 | 553 |
| 15 | 15 | 4 | 616 |
| 16 | 16 | 4 | 658 |
| 17 | 17 | 6 | 438 |
| 18 | 18 | 6 | 466 |
| 19 | 19 | 6 | 517 |
| 20 | 20 | 6 | 567 |
| 21 | 21 | 6 | 616 |
| 22 | 22 | 6 | 666 |
| 23 | 23 | 6 | 719 |
| 24 | 24 | 6 | 772 |
| 25 | 25 | 6 | 822 |
| 26 | 26 | 6 | 873 |
| 27 | 27 | 6 | 910 |
| 28 | 28 | 6 | 948 |
| 29 | 29 | 2 | Reserved |
| 30 | 30 | 4 | Reserved |
| 31 | 31 | 6 | Reserved |

Annex E (informative):
Output and inputs formats

## E.1 	Input parameters from higher layers

Table 7: List of M-Param for the MCO_ACC_NET interface, AliID related

| M-Param.No | Name of M-Param | Access | Format | Description |
| --- | --- | --- | --- | --- |
| 1 | AliID | R | One letter, 8 decimal digits See clause 6.2 | AliID per message to be transmitted, to ALI operation entity |

Table 8: List of M-Param for the MCO_ACC_NET interface, AliGroupID related

| M-Param.No | Name of M-Param | Access | Format | Description |
| --- | --- | --- | --- | --- |
|  | AliGroupID | R |  |  |
| 2 | AliGroupIDStatus | R | One decimal digit: - 0: Not Active - 1: RX only active - 3: TX/RX active - 4 – 9: reserved | Set the status of the available ALI groups in the access layer. |

## E.2	Output parameter to higher layers

Table 8: List of M-Param for the MCO_ACC_NET interface

| M-Param.No | Name of M-Param | Access | Format | Description |
| --- | --- | --- | --- | --- |
| 5 | ALN | W | 8 bit vector: | Access layer notification per message to higher layers |
| 6 | AliID | W | One letter, 8 decimal digits See clause 6.2 | AliID of received message |
| 7 | AliGroupIDStatus | W | One decimal digit: - 0: Not Active - 1: RX only active - 3: TX/RX active - 4 – 9: reserved | Status of the available ALI groups in the access layer. - 0: Not Active - 1: RX only active - 3: TX/RX active |

Table 8: List of M-Param for the MCO_ACC_NET interface. Channel load ratio

| M-Param.No | Name of M-Param | Access | Format | Description |
| --- | --- | --- | --- | --- |
|  | AliGroupID | W |  |  |
| 4 | CLR | W | - Number between 0 and 1 with a resolution of 0.01 | Channel load ratio per ALI group, to “higher layers every [100] ms: - Channel Load Ratio as value 0 to 1 with a resolution of 0.01 |

Annex F (informative):
Change History

| Date | Version | Information about changes |
| --- | --- | --- |
| 06/2021 | 0.0.1_0.0.1 | Initial skeleton created as early draft |
| 09/2021 | 0.0.1_0.0.1 | Some details have been added, still early draft version |
| 03/2022 | 0.0.6 | Stable draft version |
| 04/2022 | 0.0.7_V01 | Update based in STF feedback |
| 04/2022 | 0.0.7 V02 | Update after STF meeting 4.4.2022 |
| 05/2022 | 0.0.7_V04 | Updated after STF meeting 2.5.2022 |
| 05/2022 | 0.0.7_V05 | Updated from Paul |
| 05/2022 | 0.0.7_V07 | Updated after STF call 9.5.2022 |

# History

| Document history |  |  |
| --- | --- | --- |
| <Version> | <Date> | <Milestone> |