ETSI TS 103 836-4-1 V0.0.4 (2022-05)

Intelligent Transport Systems (ITS);

Vehicular Communications;

GeoNetworking;

Part 4: Geographical addressing and forwarding for

point-to-point and point-to-multipoint communications;

Sub-part 1: Media-Independent Functionality;

Release 2

<

TECHNICAL SPECIFICATION

Reference

REN/ITS-00358

Keywords

autonomic networking, ITS, network, safety

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-préfecture de Grasse (06) N° 7803/88

Important notice

The present document can be downloaded from:
http://www.etsi.org/standards-search

The present document may be made available in electronic versions and/or in print. The content of any electronic and/or print versions of the present document shall not be modified without the prior written authorization of ETSI. In case of any existing or perceived difference in contents between such versions and/or in print, the prevailing version of an ETSI deliverable is the one made publicly available in PDF format at www.etsi.org/deliver.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at https://portal.etsi.org/TB/ETSIDeliverableStatus.aspx.

If you find errors in the present document, please send your comment to one of the following services:
https://portal.etsi.org/People/CommiteeSupportStaff.aspx

Copyright Notification

No part may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm except as authorized by written permission of ETSI.

The content of the PDF version shall not be modified without the written authorization of ETSI.

The copyright and the foregoing restriction extend to reproduction in all media.

© ETSI 2022.

All rights reserved.

DECTTM, PLUGTESTSTM, UMTSTM and the ETSI logo are trademarks of ETSI registered for the benefit of its Members.
3GPPTM and LTETM are trademarks of ETSI registered for the benefit of its Members and
of the 3GPP Organizational Partners.
oneM2M™ logo is a trademark of ETSI registered for the benefit of its Members and
of the oneM2M Partners.
GSM® and the GSM logo are trademarks registered and owned by the GSM Association.

Contents

Intellectual Property Rights	8

Foreword	8

Modal verbs terminology	8

Executive summary	8

Introduction	9

1	Scope	10

2	References	10

2.1	Normative references	10

2.2	Informative references	10

3	Definition of terms, symbols and abbreviations	11

3.1	Terms	11

3.2	Symbols	11

3.3	Abbreviations	12

4	Services provided by the GeoNetworking protocol	13

5	Format convention	14

6	GeoNetworking address	15

6.1	General	15

6.2	GeoNetworking address format	15

6.3	Fields of the GeoNetworking address	16

7	Security and privacy	16

8	Data structures	17

8.1	General	17

8.2	Location table	17

8.2.1	General	17

8.2.2	Minimum data elements of a Location Table Entry	17

8.2.3	Maintenance of the Location Table	18

8.3	Ego Position Vector	18

8.3.1	General	18

8.3.2	Minimum data elements	18

8.3.3	Maintenance	18

8.4	Sequence number	19

8.4.1	General	19

8.4.2	Maintenance	19

8.5	Location service packet buffer	19

8.5.1	General	19

8.5.2	Buffer size	19

8.5.3	Maintenance	19

8.6	Forwarding packet buffer	20

8.6.1	General	20

8.6.2	Buffer size	20

8.6.3	Maintenance	20

9	GeoNetworking packet structure and formats	21

9.1	Overview	21

9.2	Packet structure	21

9.2.1	General	21

9.2.2	Overall packet structure	21

9.2.3	Maximum Transmit Unit	21

9.3	GeoNetworking header structure	22

9.4	GeoNetworking Secured Packet	22

9.5	Position vectors	22

9.5.1	Overview	22

9.5.2	Long Position Vector	22

9.5.2.1	Structure	22

9.5.2.2	Fields	22

9.5.3	Short Position Vector	23

9.5.3.1	Structure	23

9.5.3.2	Fields	23

9.6	Basic Header	24

9.6.1	Composition of the Basic Header	24

9.6.2	Fields of the Basic Header	24

9.6.3	Encoding of the NH field in the Basic Header	24

9.6.4	Encoding of the LT field	24

9.7	Common Header	25

9.7.1	Composition of the Common Header	25

9.7.2	Fields of the Common Header	25

9.7.3	Encoding of the NH field in the Common Header	26

9.7.4	Encoding of the HT and HST fields	26

9.7.5	Encoding of the TC field	27

9.8	GeoNetworking packet header types	27

9.8.1	Overview	27

9.8.2	GUC packet header	27

9.8.2.1	Composition of the GUC packet header	27

9.8.2.2	Fields of the GUC packet header	28

9.8.3	TSB packet header	28

9.8.3.1	Composition of the TSB packet header	28

9.8.3.2	Fields of the TSB packet header	29

9.8.4	SHB packet header	29

9.8.4.1	Composition of the SHB packet header	29

9.8.4.2	Fields of the SHB packet header	29

9.8.5	GBC/GAC packet header	30

9.8.5.1	Composition of the GBC/GAC packet header	30

9.8.5.2	Fields of the GBC/GAC packet header	30

9.8.6	BEACON packet header	31

9.8.6.1	Composition of the BEACON packet header	31

9.8.6.2	Fields of the BEACON packet header	31

9.8.7	LS Request packet header	31

9.8.7.1	Composition of the LS Request packet header	31

9.8.7.2	Fields of the LS Request packet header	32

9.8.8	LS Reply packet header	32

9.8.8.1	Composition of the LS Reply packet header	32

9.8.8.2	Fields of the LS Reply packet header	33

10	Protocol operation	33

10.1	General	33

10.2	Network management	34

10.2.1	Overview	34

10.2.1	Address configuration	34

10.2.1.1	General	34

10.2.1.2	Auto-address configuration	35

10.2.1.3	Managed address configuration	35

10.2.1.3.1	General Requirements	35

10.2.1.3.2	Initial address configuration	35

10.2.1.3.3	Address update	35

10.2.1.4	Anonymous address configuration	35

10.2.1.5	Duplicate address detection	36

10.2.2	Ego position vector and time update	36

10.2.2.1	Overview	36

10.2.2.2	Ego Position Vector update	36

10.2.2.3	Time update	36

10.2.3	Beaconing	37

10.2.4	Location service	37

10.3	Packet handling	37

10.3.1	Overview	37

10.3.2	Basic Header field settings	39

10.3.3	Basic Header processing	39

10.3.4	Common Header field settings	41

10.3.5	Common Header processing	42

10.3.6	Beacon packet handling	42

10.3.6.1	General	42

10.3.6.2	Source operations	42

10.3.6.3	Receiver operations	44

10.3.7	Location service packet handling	44

10.3.7.1	Source operations	44

10.3.7.1.1	Overview	44

10.3.7.1.2	Operation for initial LS Request	44

10.3.7.1.3	Operation for LS Request re-transmission	45

10.3.7.1.4	Operation for LS Reply	45

10.3.7.2	Forwarder operations	46

10.3.7.3	Destination operations	46

10.3.8	GUC packet handling	47

10.3.8.1	General	47

10.3.8.2	Source operations	48

10.3.8.3	Forwarder operations	49

10.3.8.4	Destination operations	51

10.3.9	TSB packet handling	52

10.3.9.1	General	52

10.3.9.2	Source operations	52

10.3.9.3	Forwarder and receiver operations	53

10.3.10	SHB packet handling	54

10.3.10.1	General	54

10.3.10.2	Source operations	55

10.3.10.3	Receiver operations	56

10.3.11	GBC packet handling	56

10.3.11.1	General	56

10.3.11.2	Source operations	57

10.3.11.3	Forwarder and receiver operations	58

10.3.12	GAC packet handling	60

10.3.12.1	General	60

10.3.12.2	Source operations	60

10.3.12.3	Forwarder and receiver operations	60

10.4	MCO management	62

10.4.1	Overview	62

10.4.2	MCO Parameters Updates (MPU)	62

10.4.2.1	Overview	62

10.4.2.2	GAGH state	63

10.4.2.3	GAGH state update	63

10.4.3	Network Updates (NU)	64

10.4.3.1	Overview	64

10.4.3.1	Access layer update	64

10.4.3.2	Network updates to the Facilities layer	64

10.4.4	ALI Request Handler (ARH)	65

Annex A (normative): Duplicate packet detection	66

A.1	General	66

A.2	SN-based duplicate packet detection	66

Annex B (normative): Packet data rate and geographical area size control	67

B.1	Overview	67

B.2	Packet data rate control	67

B.3	Geographical area size control	67

Annex C (normative): Position vector update	68

C.1	Overview	68

C.2	Update of LocT position vector	68

C.3	Update of GeoNetworking packet position vector	68

Annex D (normative): GeoNetworking forwarding algorithm selection procedure	70

Annex E (normative): Non-area forwarding algorithms	72

E.1	Overview	72

E.2	Greedy forwarding algorithm	72

E.3	Non-area contention-based forwarding algorithm	73

Annex F (normative): Area forwarding algorithms	76

F.1	Overview	76

F.2	Simple GeoBroadcast forwarding algorithm	76

F.3	Area contention-based forwarding algorithm	76

F.4	Area advanced forwarding algorithm	79

Annex G (normative): GeoNetworking traffic classification	84

Annex H (normative): GeoNetworking protocol constants	85

Annex I (informative): ASN.1 encoding of the GeoNetworking MIB	87

I.1	Use of modules	87

I.2	ASN.1 module	87

Annex J (informative): GeoNetworking data interface	93

J.1	General	93

J.2	TRANSP_CORE.request	93

J.3	TRANSP_CORE.confirm	94

J.4	TRANSP_CORE.indication	94

Annex K (informative): GN Core network management services	96

K.1	General	96

K.2	CORE_GAGH.request	96

K.3	CORE_GAGH.response	96

K.4 	CORE_GAGH.indication	97

Annex L (informative): MCO_NET services for the GN Core	98

L.1	General	98

L.2	CORE_MCO.request	98

L.3	CORE_MCO.response	98

L.4	CORE_MCO.update	98

Annex M (informative): MCO_NET services for the GAGHs	100

M.1	General	100

M.2	GAGH_MCO.indicate	100

M.3	GAGH_MCO.update	100

Annex N (informative): MCO services for the Facilities layer	101

N.1	General	101

N.2	MCO_FAC_NET.request	101

N.3	MCO_FAC_NET.update	101

N.4	MCO_FAC_NET.indicate	102

Annex O (informative): Interface to the security entity	103

Annex P (informative): Bibliography	104

Annex (informative): Change History	106

History	107

# Intellectual Property Rights

Essential patents

IPRs essential or potentially essential to normative deliverables may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (https://ipr.etsi.org).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

Trademarks

The present document may include trademarks and/or tradenames which are asserted and/or registered by their owners. ETSI claims no ownership of these except for any which are indicated as being the property of ETSI, and conveys no right to use or reproduce any trademark and/or tradename. Mention of those trademarks in the present document does not constitute an endorsement by ETSI of products, services or organizations associated with those trademarks.

# Foreword

This Technical Specification (TS) has been produced by ETSI Technical Committee Intelligent Transport Systems (ITS).

The present document is part 4, sub-part 1, of a multi-part deliverable defined in part 1 [i.9].

# Modal verbs terminology

In the present document "shall", "shall not", "should", "should not", "may", "need not", "will", "will not", "can" and "cannot" are to be interpreted as described in clause 3.2 of the ETSI Drafting Rules (Verbal forms for the expression of provisions).

"must" and "must not" are NOT allowed in ETSI deliverables except when used in direct citation.

# Executive summary

# Introduction

The GeoNetworking protocol is a network layer protocol that provides packet routing in an ad hoc network. It makes use of geographical positions for packet transport. GeoNetworking supports the communication among individual ITS stations as well as the distribution of packets in geographical areas.

GeoNetworking can be executed over different ITS access technologies for short-range wireless technologies, such as ITS-G5 and infrared. The ITS access technologies for short-range wireless technologies have many technical commonalities, but also differences. In order to reuse the GeoNetworking protocol specification for multiple ITS access technologies, the specification is separated into media-independent and media-dependent functionalities. Media-independent functionalities are those which are common to all ITS access technologies for short-range wireless communication to be used for GeoNetworking. The media-dependent functionalities extend the media-independent functionality for a specific ITS access technology. Therefore, the GeoNetworking protocol specification consists of the standard for media-independent functionality and at least one standard for media-dependent functionality. However, it should be noted that the media-dependent extensions do not represent distinct protocol entities.

# 1	Scope

The present document specifies the Release 2 of the GeoNetworking protocol. Compared to the Release 1 specification, Release 2 primarily adds support for multi-channel operation.

# 2	References

## 2.1	Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

Referenced documents which are not found to be publicly available in the expected location might be found at https://docbox.etsi.org/Reference.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

The following referenced documents are necessary for the application of the present document.

[1]	ETSI EN 302 931 (V1.1.1): "Intelligent Transport Systems (ITS); Vehicular Communications; Geographical Area Definition".

[2]	ETSI TS 102 894-2 (V0.0.2): "Intelligent Transport Systems (ITS); Users and applications requirements; Part 2: Applications and facilities layer common data dictionary".

## 2.2	Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	ETSI TS 103 836-3 (V2.1.1): "Intelligent Transport Systems (ITS); Vehicular Communications; GeoNetworking; Part 3: Network architecture; Release 2".

[i.2]	ETSI TS 103 697 (V2.1.1): “Intelligent Transport System (ITS); Multi-Channel Operation (MCO) for ITS, Release 2“

[i.3]	IETF RFC 2578: "Structure of Management Information Version 2 (SMIv2)".

[i.4]	National Imagery and Mapping Agency (NIMA), US Department of Defense: "World Geodetic System 1984 - Its Definition and Relation with Local Geodetic Systems", Third Edition - Amendment 1, NIMA TR 8350.2.

[i.5]	IETF RFC 2579: "Textual Conventions for SMIv2".

[i.6]	IEEE 802.3:2008™: "IEEE Standard for Information Technology - Telecommunications and information exchange between systems-Local and metropolitan area networks - Specific requirements - Part 3: Carrier Sense Multiple Access with Collision Detection (CSMA/CD) Access Method and Physical Layer Specifications".

[i.7]	ETSI TS 102 965: "Intelligent Transport Systems (ITS); Application Object Identifier (ITS-AID); Registration; Release 2".

[i.8]	ETSI TS 103 695: " Intelligent Transport System (ITS); Access layer specification in the 5GHz frequency band; Multi-Channel Operation (MCO) for Cooperative ITS (C-ITS); Release 2"

[i.9]	ETSI TS 103 836-1: "Intelligent Transport Systems (ITS); Vehicular Communications; GeoNetworking; Part 1: Requirements; Release 2"

# 3	Definition of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in ETSI TS 103 836-3 [i.1], TS 103 697 [i.2],--- and the following apply:

destination: receiver that processes a packet and delivers it to upper protocol entities, but does not relay the packet to other GeoAdhoc routers

forwarder: GeoAdhoc router that processes a packet and relays it to other GeoAdhoc routers

GeoAdhoc router: ad hoc router that implements the GeoNetworking protocol

local position vector: position vector for the local GeoAdhoc router

media dependent procedures: packet handling steps that are specific to the Access Layer technology over which the GeoNetworking packet will be transmitted

neighbour: GeoAdhoc router in direct (single-hop) communication range

packet: GeoNetworking PDU

packet transport type: method of handling GeoNetworking packets

position accuracy indicator: binary that indicates whether a position is within a specific confidence interval

position vector: position information of a GeoAdhoc router represented by a tuple of address, timestamp, geographical position, speed, heading and corresponding accuracy information

receiver: GeoAdhoc router that processes a packet, delivers its data to upper protocol entities

sender: GeoAdhoc router that has sent the GeoNetworking packet

source: GeoAdhoc router that originates a GeoNetworking packet

traffic class: identifier assigned to a GeoNetworking packet that expresses its requirements on data transport

## 3.2	Symbols

For the purposes of the present document, the terms given in TS 103 697 [i.2] and the following symbols apply:

ALI_ADDR	Address of the Access Layer instance that identifies the ITS-S at the link layer protocol entity in the Access Layer

GEO_MAX	Maximum size of the GeoNetworking packet header

H(GN_ADDR)	Heading of the ITS-S GN_ADDR

LAT	Latitude

LL_ADDR_NH	Address of the link layer of of the next hop

LONG	Longitude

LS_PENDING 	Location Service pending flag

MTU_AL	MTU of the Access Layer

PAI(POS, GN_ADDR)	Position accuracy indicator for geographical position POS of the ITS-S GN_ADDR

PDR(GN_ADDR)	Packet data rate (exponential moving average)

POS(GN_ADDR)	Geographical position of the ITS-S GN_ADDR

PV(GN_ADDR)	Position vector of the ITS-S GN_ADDR

RAND[x,y]	Function that returns a random (integer) number from a uniform distribution in the given interval [x,y]

S(GN_ADDR)	Speed of the ITS-S GN_ADDR

SN_MAX	Largest possible value of the sequence number

SN(P)	Value of the sequence number field carried in a GeoNetworking packet

T(LocTE)	Lifetime of an entry in the location table

TO_CBF_MIN	Timeout; minimum duration a packet is buffered in the CBF cache

TO_CBF_MAX	Timeout; maximum duration a packet is buffered in the CBF cache

TST(GN_ADDR)	Last timestamp received from a GeoAdhoc router

TST(P)	Value of the timestamp field carried in a GeoNetworking packet

TST(TAI)	Number of elapsed TAI milliseconds since 2004-01-01 00:00:00.000 UTC

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in ETSI TS 103 836-3 [i.1]--, TS 103 697 [i.2] and the following apply:

ALI 	Access Layer Instance

AliID				Access layer instance identifier

ARH				ALI Request Handler

ASN	Abstract Syntax Notation

BC	BroadCast

BTP	Basic Transport Protocol

CBF	Contention-Based Forwarding

CLR		Channel Load Ratio

DAD	Duplicate Address Detection

DE	Destination

DPC	Duplicate Packet Counter

DPL	Duplicate Packet List

DPD	Duplicate Packet Detection

EMA	Exponential Moving Average

EPV	Ego Position Vector

FIFO	First In First Out

GAC	Geographically-Scoped Anycast

GAGH	GeoNetworking Access Layer Instance Group Handler

GBC	Geographically-Scoped Broadcast

GF	Greedy Forwarding

GN	GeoNetworking

GN_ADDR	GeoNetworking ADDRess

GN-PDU	GeoNetworking Protocol Data Unit

GN6-SDU	GN6 Service Data Unit

GN6ASL	GeoNetworking to IPv6 Adaptation Sub-Layer

GN-SDU	GeoNetworking Service Data Unit

GUC	Geographically-Scoped Unicast

HST	Header Sub-Type

HT	Header Type

LocT	Location Table

LocTE	Location Table Entry

LPV	Local Position Vector

LS	Location Service

LT	LifeTime

LTE	Long Term Evolution

MAC	Medium Access Control

MFR	Most Forward within Radius

MHL	Maximum Hop Limit

MHVB	Multi-Hop Vehicular Broadcast

MIB	Management Information Base

MID	MAC ID

MPU 		MCO Parameters Update

MTU	Maximum Transmit Unit

NH	Next Header

NU		Network Updates

PAI	Position Accuracy Indicator

PCI	Protocol Control Information

PDR	Packet Data Rate

PDU	Protocol Data Unit

PL	Payload Length

POS	POSition

PV	Position Vector

RHL	Remaining Hop Limit

RTC	Retransmit Counter

SCF	Store Carry & Forward

SDU	Service Data Unit

SE	SEnder

SHB	Single Hop Broadcast

SN	Sequence Number

SO	SOurce

SPV	Short Position Vector

SSP	Service Specific Permissions

ST	Station Type

TAI	Temps Atomique International (International Atomic Time)

TC	Traffic Class

TC ID	Traffic Class Identifier

TSB	Topologically Scoped Broadcast

T-SDU	Transport Service Data Unit

TST	TimeSTamp

UC	UniCast

V2X	Vehicle-to-Everything

UTC	Universal Time Coordinated

WGS	World Geodetic System

# 4	Services provided by the GeoNetworking protocol

The GeoNetworking protocol is a network protocol that resides in the Networking & Transport layer. Requirements on the GeoNetworking protocol are specified in ETSI TS 103 836-3 [i.1]. It is executed in the ad hoc router, specifically in the GeoAdhoc router. It provides the transport of packets in the ITS ad hoc network (ETSI TS 103 836-3 [i.1])-.

GeoNetworking comprises several components, namely GeoNetworking Core (GN Core), GeoNetworking ALI Group Handlers (GAGHs) and MCO_NET (figure 1). GN Core provides channel- and media-independent GeoNetworking functionalities. A GAGH realizes channel dependent, media-dependent, and media-independent GeoNetworking functionalities. The MCO_NET entity provides MCO management functionalities as part of the GeoNetworking protocol.

The GeoNetworking protocol provides services to upper protocol entities, i.e., the transport protocol, such as the Basic Transport Protocol (BTP), the GeoNetworking to IPv6 Adaptation Sub-Layer (GN6ASL), and MCO-related services to the Facilities layer entities.

In order to provide its packet transport services, the GeoNetworking protocol uses the services of the Access Layer via the NET_ACC_DATA interface. To provide MCO functionalities to upper layers, MCO_NET uses services provided by the Access layer via the NET_ACC_MMT interface.

Figure 1: GeoNetworking-related components and interfaces

The present standard specifies the internal interfaces of the Networking & Transport layer as defined in ETSI TS 103 836-3 [i.1] (figure 1), i.e.,

the interface between the GeoNetworking protocol and the ITS transport protocol, such as the Basic Transport Protocol (BTP), the GeoNetworking IPv6 Adaptation Sub-Layer (GN6ASL), and other transport protocols,

the interface between the GeoNetworking Core and the GeoNetworking ALI Group Handler,

the interface between the GeoNetworking Core and the MCO_NET entity,

the interface between the GeoNetworking ALI Group Handlers and the MCO_NET entity.

- To provide MCO services to the Facilities layer entities, the present standard specifies the MCO_FAC_NET_MMT interface between the Networking & Transport layer and the Facilities layer.

# 5	Format convention

The basic convention for the specification of packet formats is illustrated in figure 2. The bits are grouped into octets. The bits of an octet are always shown horizontally and are numbered from 0 to 7. Up to 4 octets are shown horizontally; multiple sets of 4 octets are grouped vertically. Octets are numbered from 0 to N-1.

| 0 |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Octet 0 |  |  |  |  |  |  |  |  | Octet 1 |  |  |  |  |  |  |  |  | Octet 2 |  |  |  |  |  |  |  |  | Octet 3 |  |  |  |  |  |  |  |
| Octet 4 to Octet 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet N-1 |  |  |  |  |  |  |  |

Figure 2: Format convention

When (a part of) an Octet represents a numeric quantity the left most bit in the diagram is the most significant bit (Big Endian). Similarly, when a numeric value spans multiple octet fields the left most field is the most significant.

Octets are transmitted in ascending numerical order (left to right).

EXAMPLE:	The decimal value 199 shall be represented as shown below.

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |

# 6	GeoNetworking address

## 6.1	General

Every GeoAdhoc router, irrespective of the number of GAGHs, shall have a unique GeoNetworking address. This address shall be used in the header of a GeoNetworking packet and identify the communicating GeoNetworking entities. In order to ensure the uniqueness of the GeoNetworking address, duplicate detection as specified in clause 10.2.1.5 is applied. For multi-channel operation, an ITS-S with several GAGHs shall use a single GeoNetworking address.

NOTE:	In case the GN MID changes, the link-layer address should also be changed. In one possible implementation the link-layer address is set from the GeoNetworking protocol entity.

## 6.2	GeoNetworking address format

The format of the GeoNetworking address shall be as described in figure 3.

| 0 |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 |  | 2 | 3 | 4 | 5 | 6 |  | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| M |  | ST |  |  |  |  |  | Reserved |  |  |  |  |  |  |  |  |  |  | MID |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MID |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 3: GeoNetworking address format

## 6.3	Fields of the GeoNetworking address

The GeoNetworking address shall be comprised of the fields specified in table 1.

Table 1: Fields of the GeoNetworking address

| Field # | Field name | Octet/bit position |  | Type | Description |
| --- | --- | --- | --- | --- | --- |
|  |  | First | Last |  |  |
| 1 | M | Octet 0 Bit 0 | Octet 0 Bit 0 | 1 bit unsigned integer | This bit allows distinguishing between manually configured network address (clause 10.2.1.3.3) (update) and the initial GeoNetworking address (clause 10.2.1.3.2). M is set to 1 if the address is manually configured otherwise it equals 0. |
| 2 | ST | Octet 0 Bit 1 | Octet 0 Bit 5 | 5 bit unsigned integer | ITS-S type To identify the ITS-S type: 0 - Unknown 1 - Pedestrian 2 - Cyclist 3 - Moped 4 - Motorcycle 5 - Passenger Car 6 - Bus 7 - Light Truck 8 - Heavy Truck 9 - Trailer 10 - Special Vehicle 11 - Tram 15 - Road Side Unit (see note). |
| 4 | Reserved | Octet 0 Bit 6 | Octet 1 Bit 7 | 10 bit unsigned integer | Reserved |
| 5 | MID | Octet 2 | Octet 7 | 48 bit address | Used to derive the Link Layer address. |
| NOTE: The values of the ITS-S type are aligned with ETSI TS 102 894-2 [2]. |  |  |  |  |  |

The first bit is reserved for the recognition of manual configured GeoNetworking addresses.

The MID field shall be used to derive an Access layer address.

In order to allow for the resolution of a GeoUnicast destination GN_ADDR from an IPv6 destination address using virtual interfaces of type Ethernet V2.0/IEEE 802.3 LAN [i.6], the GeoNetworking address space shall remain 48-bit wide (size of the MID field in the GeoNetworking address). In particular, the GN6ASL resolves an MID from a unicast destination IPv6 address and passes it to GeoNetworking via the service primitive TRANSP_CORE.request (annex J.2). Then, the GeoNetworking protocol is responsible for deriving a full GN_ADDR from the MID. This full GN_ADDR shall be derived from a LocTE (if it exists) or by executing the Location Service with Request GN_ADDR field containing only the MID part and the other bits set to 0.

To be compliant with the IPv6 over GeoNetworking architecture, the GeoNetworking address space shall remain 48-bit wide (size of the MID field in the GeoNetworking address) in order to provide a virtual interface of Ethernet type to IPv6 and to perform the forwarding via GeoNetworking in a transparent way.

If the address is updated for privacy reasons, i.e., by assignment of an alias identity, only the last field of the address shall be updated and derived from the alias identity.

# 7	Security and privacy

In order to ensure confidentiality, integrity, availability, accountability and authenticity. The Networking and Transport layer shall support security mechanisms and related interfaces to security entities when required. This may include:

cryptographic protection by digital signatures;

encryption;

consistency checks; and

plausibility checks.

The Networking & Transport layer functionalities such as GeoNetworking shall restrict the forwarding of packets by hop count, lifetime, geographical area size and packet data rate when required. Furthermore, GeoNetworking shall support anonymous address configuration for the use of pseudonyms when applicable.

For a particular ITS implementation, all GeoAdhoc routers should implement the same minimum-security measures. The application of cryptographic protection may be controlled by the GN protocol constant itsGnSecurity (see annex O).

NOTE:	This, to ensure that there is no mix of GeoAdhoc routers in an ITS implementation, for example one GeoAdhoc router applies security, others do not.

# 8	Data structures

## 8.1	General

A GeoAdhoc router shall maintain different types of local data structures per GAGH, i.e.,

location table (clause 8.2),

sequence number (clause 8.4)

location service packet buffer (clause 8.5),

forwarding packet buffer (clause 8.6).

A GeoAdhoc router shall also maintain a data structure for the ego position vector (clause 8.3) independently of the GAGH.

NOTE: 	Compared to the other data structures, it is assumed that the ego position vector is independent of GAGH and therefore expected to be implemented in the GN Core (see figure 1).

## 8.2	Location table

### 8.2.1	General

A GeoAdhoc router shall maintain a local data structure, referred to as location table (LocT). This data structure holds information about other ITS-Ss received by the same GAGH that execute the GeoNetworking protocol. The data elements of a location table entry are specified in clause 8.2.2 and the maintenance of the location table in clause 8.2.3.

### 8.2.2	Minimum data elements of a Location Table Entry

A Location Table Entry (LocTE) shall contain at least the following data elements:

GeoNetwork address of the ITS-S GN_ADDR.

Link layer address of the ITS-S .

Type of the ITS-S (e.g. vehicle ITS-S, roadside ITS-S).

Version of the GeoNetworking protocol used by the ITS-S.

Position vector PV, i.e., Long Position Vector LPV (clause 9.5.2), of the ITS-S, comprised of:

Geographical position POS(GN_ADDR);

Speed S(GN_ADDR);

Heading H(GN_ADDR);

Timestamp of the geographical position TST(POS, GN_ADDR);

Position accuracy indicator PAI(POS, GN_ADDR).

Flag LS_PENDING(GN_ADDR): Flag indicating that a Location Service (LS) (clause 10.2.4) is in progress.

Flag IS_NEIGHBOUR(GN_ADDR): Flag indicating that the GeoAdhoc router is in direct communication range, i.e., is a neighbour.

DPL(GN_ADDR): Duplicate packet list for source GN_ADDR.

Timestamp TST(GN_ADDR): The timestamp of the last packet from the source GN_ADDR that was identified as 'not duplicated'.

Packet data rate PDR(GN_ADDR) as Exponential Moving Average (EMA) (annex B.2).

NOTE 1:	The LocTE may contain more data elements defined in media-dependent functionalities of GeoNetworking.

NOTE 2:	The format of the data in the LocT is implementation-specific and, therefore, not further specified.

NOTE 3:	LS_PENDING(GN_ADDR) equals TRUE indicates that for the GN_ADDR a location service has been invoked and is in process.

### 8.2.3	Maintenance of the Location Table

The entries in the location table shall be soft-state, i.e., entries are added with a lifetime T(LocTE) set to the value of the GN protocol constant itsGnLifetimeLocTE and shall be removed when the lifetimes expire.

The flag LS_PENDING(GN_ADDR) shall be soft-state, i.e., it shall be unset when the flag is not renewed within the lifetime 3 × itsGnBeaconServiceRetransmitTimer.

## 8.3	Ego Position Vector

### 8.3.1	General

A GeoAdhoc router shall maintain a local data structure that holds position-related information for the local GeoAdhoc router, i.e., the ego position vector EPV. The data elements of a EPV are specified in clause 8.3.2 and the maintenance of the location table in clause 8.3.3.

### 8.3.2	Minimum data elements

The EPV shall contain at least the following data elements:

1)	Geographical position POS_EPV,

2)	Speed S_EPV,

3)	Heading H_EPV,

4)	Timestamp TST_EPV indicating when the geographical position POS_EPV was generated,

5)	Accuracy of the geographical position PAI_EPV.

### 8.3.3	Maintenance

At start-up, all data elements of the EPV shall be initialized with 0 to indicate an unknown value.

The EPV shall be updated with a frequency of the GN protocol constant itsGnMinUpdateFrequencyEPV or higher.

In case of a stationary ITS-S, the timestamp of the EPV shall be updated with a frequency of the GN protocol constant itsGnMinUpdateFrequencyEPV or higher. The position field in the EPV shall be set to a value indicating the position of the stationary ITS-S. Speed and heading fields of the EPV shall be set to 0. While the ITS-S is stationary, the value of the position field should not change.

## 8.4	Sequence number

### 8.4.1	General

Each GeoAdhoc router shall maintain a local sequence number per GAGH that determines the Sequence Number (SN) field of the next GeoNetworking packet to be transmitted.

### 8.4.2	Maintenance

For a GAGH, the SN shall be initialized to 0. For every GeoNetworking packet P, the sequence number SN(P) shall be incremented as follows:

with SN(P) being the sequence number of the GeoNetworking packet and SN_MAX the largest possible sequence number. The resulting sequence number shall be included in the GeoNetworking packet.

The SN is incremented for multi-hop GeoNetworking packets only. Single-hop GeoNetworking packets (BEACON, SHB) do not carry a SN field.

## 8.5	Location service packet buffer

### 8.5.1	General

Upon invocation of the LS (clause 10.2.4) by a GAGH, a GeoAdhoc router shall queue a GeoNetworking packet in a LS packet buffer for the sought destination until the LS is completed. Subsequent GeoNetworking packets, which are processed while the LS is in progress, shall also be buffered (see clause 10.2.4).

### 8.5.2	Buffer size

The LS packet buffer shall have a minimum size of the value stored in the GN protocol constant itsGnLocationServicePacketBufferSize.

### 8.5.3	Maintenance

The LS packet buffer shall work as follows:

GeoNetworking packets arriving at the LS packet buffer for a destination (GN_ADDR of a certain ITS-S) shall be queued at the tail of the queue.

When a new GeoNetworking packet arrives at the LS packet buffer and exceeds the buffer capacity (buffer overflow), GeoNetworking packets from the head of the queue are removed and the new GeoNetworking packet queued at the tail (head drop).

When the LS is completed, the LS packet buffer shall be flushed, i.e., all GeoNetworking packets stored in the buffer shall be sent in a FIFO manner.

When the queuing time of the GeoNetworking packet in the LS packet buffer exceeds the packet lifetime carried in the GeoNetworking packet's LT field in the Basic Header, the GeoNetworking packet shall be discarded.

When a stored GeoNetworking packet is sent:

a)	the LT field shall be reduced by the queuing time in the LS packet buffer;

b)	it is recommended to update the SO PV.

NOTE 1:	When security is enabled, i.e., the GN protocol constant itsGnSecurity is set to ENABLED, and the local GeoAdhoc router is the source of the GeoNetworking packet, the signature may need to be updated. Signatures of forwarded packets are not updated.

When the LS does not complete, all stored GeoNetworking packets shall be discarded triggered by the LS.

NOTE 2:	The mechanism to detect that a LS does not complete is implementation dependent.

## 8.6	Forwarding packet buffer

### 8.6.1	General

A GeoAdhoc router shall use forwarding packet buffers to temporarily keep packets in a GeoAdhoc router during the forwarding process.

A GeoAdhoc router shall maintain the following forwarding packet buffers:

1)	UC forwarding packet buffer to buffer GUC packets per GN_ADDR per GAGH.

2)	BC forwarding packet buffer to buffer TSB, GBC and GAC packets per GAGH.

The GeoAdhoc router shall maintain a CBF packet buffer if Contention-Based Forwarding (CBF) is enabled, i.e., if:

1)	the GN protocol constant itsGnNonAreaForwardingAlgorithm is set to 2 (CBF);

2)	the GN protocol constant itsGnAreaForwardingAlgorithm is set to 2 (CBF) or 3 (ADVANCED).

### 8.6.2	Buffer size

The UC forwarding packet buffer shall have a minimum size given by the value of the GN protocol constant itsGnUcForwardingPacketBufferSize.

The BC forwarding packet buffer shall have a minimum size given by the value of the GN protocol constant itsGnBcForwardingPacketBufferSize.

The CBF packet buffer shall have a minimum size given by the value of the GN protocol constant itsGnCbfPacketBufferSize.

### 8.6.3	Maintenance

The UC forwarding packet buffer and the BC forwarding packet buffer shall work as follows:

- GeoNetworking packets arriving at the forwarding packet buffer shall be queued at the tail of the queue.

When a new GeoNetworking packet arrives at the forwarding packet buffer and exceeds the buffer capacity, GeoNetworking packets from the head of the queue are removed and the new GeoNetworking packet queued at the tail (head drop).

When the forwarding packet buffer is flushed, the GeoNetworking packets stored in the buffer shall be forwarded in a FIFO manner.

When the queuing time of the GeoNetworking packet in the forwarding packet buffer exceeds the packet lifetime carried in the packet's LT field in the Basic Header, the GeoNetworking packet shall be discarded.

When a stored GeoNetworking packet is sent:

a)	the LT field shall be reduced by the queuing time in the forwarding packet buffer;

b)	it is recommended to update the SO PV.

NOTE 1:	When security is enabled, i.e., the GN protocol constant itsGnSecurity is set to ENABLED, and the local GeoAdhoc router is the source of the GeoNetworking packet, the signature may need to be updated. Signatures of forwarded packets are not updated.

The CBF packet buffer shall work as follows:

- Packets arriving at the CBF packet buffer shall be queued at the tail of the queue.

When a new GeoNetworking packet arrives at the CBF packet buffer and exceeds the buffer capacity, GeoNetworking packets from the head of the queue are removed and the new GeoNetworking packet queued at the tail (head drop).

Every GeoNetworking packet in the buffer is associated with a timer. When the timer expires the GeoNetworking packet is removed from the queue and sent.

When a stored GeoNetworking packet is sent:

- the LT field shall be reduced by the queuing time in the CBF packet buffer;

NOTE 2:	Due the encoding of the LT field (see clause 9.6.4) the reduction of the LT field by the queuing time may not have an effect.

b)	the SO PV in the sent packet should be updated.

NOTE 3:	When security is enabled, i.e., the GN protocol constant itsGnSecurity is set to ENABLED, and the local GeoAdhoc router is the source of the GeoNetworking packet, the signature may need to be updated. Signatures of forwarded packets are not updated.

NOTE 4:	The value of the timer is set by the CBF forwarding algorithm specified in annex E.3.

# 9	GeoNetworking packet structure and formats

## 9.1	Overview

This clause specifies the structure and the format of the GeoNetworking packet.

## 9.2	Packet structure

### 9.2.1	General

As specified in ETSI TS 103 836-3 [i.1], the GeoNetworking protocol is either used in the GeoNetworking protocol stack or in the protocol stack that combines the GeoNetworking protocol and IPv6.

### 9.2.2	Overall packet structure

A GeoNetworking packet is part of the overall frame/packet structure depicted in figure 4 (without security) and figure 6 (with security), respectively:

1)	The Access layer header is the header of the specific technology at the Access layer with which the packet is transmitted.

NOTE 1:	The Access layer header is not specified by the present document. However, the GeoNetworking protocol sets the link layer address, or more generally the link layer address, in order to define and identify the next hop of a GeoNetworking packet.

2)	The GeoNetworking header is the header of the GeoNetworking packet as defined in the present document.

3)	The optional payload represents the user data that are created by upper protocol entities, i.e., the T-SDU or GN6-SDU. It is passed to the GeoNetworking protocol for transmission.

NOTE 2:	Some GeoNetworking packets do not carry a payload, such as Beacon.

| Access Layer Header | GeoNetworking Header | Payload (optional) |
| --- | --- | --- |

Figure 4: GeoNetworking packet structure (without security)

### 9.2.3	Maximum Transmit Unit

The Maximum Transmit Unit (MTU), which the GeoNetworking protocol supports via the TRANSP_CORE interface, i.e., the MTU_GN depends on the MTU of the Access layer technology (MTU_ALI) over which the GeoNetworking packet is transported. In particular, MTU_GN shall be less or equal to MTU_ALI reduced by the size of the largest GeoNetworking protocol header (GEO_MAX) including Basic Header, Common Header and Extended Header and security overhead:

MTU_GN ≤MTU_ALI-GEO_MAX

GEO_MAX is set by the GN protocol constant itsGnMaxGeoNetworkingHeaderSize.

## 9.3	GeoNetworking header structure

The GeoNetworking header shall be comprised of a Basic Header, Common Header and an optional Extended Header (figure 5).

| Basic Header | Common Header | Extended Header (optional) |
| --- | --- | --- |

Figure 5: GeoNetworking header structure

Basic Header, Common Header and Extended Header are specified in clause 9.6, clause 10.3.6 and clause 9.8.

NOTE:	The composition of the Basic Header and Common Header equals for all packet transport types and differs for the Extended Header.

## 9.4	GeoNetworking Secured Packet

The overall packet structure may be protected by security services, by digital signatures, certificates, and by encryption (annex O).

With enabled security (GN protocol constant itsGnSecurity is set to ENABLED), the overall packet structure is depicted in figure 6.

Security operations are executed by the security entity via the GAGH_SEC interface (figure 1) and as specified in clause 10.3 and annex O.

| Access Layer Header | GeoNetworking Basic Header | GeoNetworking Secured Packet with GeoNetworking Common Header, Optional Extended Header and Optional Payload |
| --- | --- | --- |

Figure 6: GeoNetworking packet structure (with security)

## 9.5	Position vectors

### 9.5.1	Overview

For simplicity, a set of position-related fields of the GeoNetworking header are subsumed to a position vector (PV). Two types of PV are defined:

1)	Long position vector as specified in clause 9.5.2;

2)	Short position vector as specified in clause 9.5.3.

### 9.5.2	Long Position Vector

#### 9.5.2.1	Structure

The Long Position Vector (LPV) shall consist of the fields specified in figure 7.

| 0 |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| GN_ADDR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lat |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Long |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PAI | S |  |  |  |  |  |  |  |  |  |  |  |  |  |  | H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 7: Long Position Vector

#### 9.5.2.2	Fields

The Long Position Vector (LPV) shall consist of the fields as specified in table 2.

Table 2: Fields of Long Position Vector

| Field # | Field name | Octet/bit position |  | Type | Unit | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | First | Last |  |  |  |
| 1 | GN_ADDR | Octet 0 | Octet 7 | 64 bit address | n/a | Network address for the GeoAdhoc router entity in the ITS-S |
| 2 | TST | Octet 8 | Octet 11 | 32 bit unsigned integer | [ms] | Expresses the time in milliseconds at which the latitude and longitude of the ITS-S were acquired by the GeoAdhoc router. The time is encoded as: where TST(TAI) is the number of elapsed TAI milliseconds since 2004-01-01 00:00:00.000 UTC |
| 3 | Lat | Octet 12 | Octet 15 | 32 bit signed integer | [1/10 micro-degree] | WGS 84 [i.4] latitude and longitude of the GeoAdhoc router reference position expressed in 1/10 micro degree |
| 4 | Long | Octet 16 | Octet 19 | 32 bit signed integer | [1/10 micro-degree] |  |
| 5 | PAI | Octet 20 Bit 0 | Octet 20 Bit 0 | 1 bit unsigned integer | n/a | Position accuracy indicator of the GeoAdhoc router reference position Set to 1 (i.e., True) if the semiMajorConfidence of the PosConfidenceEllipse as specified in ETSI TS 102 894-2 [2] is smaller than the GN protocol constant itsGnPaiInterval / 2 Set to 0 (i.e., False) otherwise |
| 6 | S | Octet 20 Bit 1 | Octet 21 | 15 bit signed integer | [1/100 m/s] | Speed of the GeoAdhoc router expressed in signed units of 0,01 metre per second |
| 7 | H | Octet 22 | Octet 23 | 16 bit unsigned integer | [1/10 degrees] | Heading of the GeoAdhoc router, expressed in unsigned units of 0,1 degree from North |

### 9.5.3	Short Position Vector

#### 9.5.3.1	Structure

The Short Position Vector (SPV) shall consist of the fields specified in figure 8.

| 0 |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| GN_ADDR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lat |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Long |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 8: Short Position Vector

#### 9.5.3.2	Fields

The Short Position Vector (SPV) shall consist of the fields as specified in table 3.

Table 3: Fields of Short Position Vector

| Field # | Field name | Octet/bit position |  | Type | Unit | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | First | Last |  |  |  |
| 1 | GN_ADDR | Octet 0 | Octet 7 | 64 bit address | n/a | GN_ADDR field as specified in table 2 |
| 2 | TST | Octet 8 | Octet 11 | 32 bit unsigned integer | [ms] | Timestamp TST field as specified in table 2 |
| 3 | Lat | Octet 12 | Octet 15 | 32 bit signed integer | [1/10 micro-degree] | Latitude (Lat) field as specified in table 2 |
| 4 | Long | Octet 16 | Octet 19 | 32 bit signed integer | [1/10 micro-degree] | Longitude (Long) field as specified in table 2 |

NOTE:	The timestamp TST field indicates the time when the position (LAT, LONG) of the SPV was acquired.

## 9.6	Basic Header

### 9.6.1	Composition of the Basic Header

The Basic Header shall be present in every GeoNetworking packet and consists of the fields as depicted in figure 9.

| 0 |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Version |  |  |  | NH |  |  |  | Reserved |  |  |  |  |  |  |  | LT |  |  |  |  |  |  |  | RHL |  |  |  |  |  |  |  |

Figure 9: Basic Header format

### 9.6.2	Fields of the Basic Header

The Basic Header shall carry the fields as specified in table 4.

Table 4: Fields of the Basic Header

| Field # | Field name | Octet/bit position |  | Type | Unit | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | First | Last |  |  |  |
| 1 | Version | Octet 0 Bit 0 | Octet 0 Bit 3 | 4 bit unsigned integer | n/a | Identifies the version of the GeoNetworking protocol |
| 2 | NH | Octet 0 Bit 4 | Octet 0 Bit 7 | 4 bit unsigned integer | n/a | Identifies the type of header immediately following the GeoNetworking Basic Header as specified in table 5 |
| 3 | Reserved | Octet 1 | Octet 1 | 8-bit unsigned integer | n/a | Reserved Set to 0 |
| 4 | LT | Octet 2 | Octet 2 | 8 bit unsigned integer | n/a | Lifetime field. Indicates the maximum tolerable time a packet may be buffered until it reaches its destination Bit 0 to Bit 5: LT sub-field Multiplier Bit 6 to Bit 7: LT sub-field Base Encoded as specified in clause 9.6.4 |
| 5 | RHL | Octet 3 | Octet 3 | 8 bit unsigned integer | [hops] | Decremented by 1 by each GeoAdhoc router that forwards the packet The packet shall not be forwarded if RHL is decremented to zero |

### 9.6.3	Encoding of the NH field in the Basic Header

For the Next Header (NH) field in the Basic Header the values as specified in table 5 shall be used.

Table 5: Next Header (NH) field in the GeoNetworking Basic Header

| Next Header (NH) | Encoding | Description |
| --- | --- | --- |
| ANY | 0 | Unspecified |
| Common Header | 1 | GeoNetworking Common Header as specified in clause 10.3.6 |
| Secured Packet | 2 | GeoNetworking Secured Packet as specified in clause 9.4 GeoNetworking Secured Packet |
| NOTE 1: The Common Header also carries a NH field. |  |  |

### 9.6.4	Encoding of the LT field

The Lifetime (LT) field shall indicate the maximum tolerable time a packet may be buffered until it reaches its destination.

NOTE 1:	This parameter is relevant for safety and traffic efficiency information that do not have strict real-time requirements. In sparse network scenarios, this lifetime may also be used to avoid re-transmission and forwarding of outdated information.

NOTE 2:	When a GeoNetworking packet is buffered, the value of the Lifetime (LT) field is reduced by the queuing time in the packet buffer.

The following method for encoding of the LT field uses a non-linear encoding, which provides a high resolution for low numbers and progressively lower resolution for higher numbers.

The LT field shall be comprised of two sub-fields: a LTMultiplier sub-field (Multiplier) and a LTBase sub-field (Base) (figure 10) and shall be encoded as follows:

Lifetimedecoded=LTMultiplier×TBase

The LTBase sub-field represents a two bit unsigned selector that chooses one out of four predefined values as specified in table 6.

Table 6: Encoding of LT sub-field LT Base

| Value | LTbase |
| --- | --- |
| 0 | 50 ms |
| 1 | 1 s |
| 2 | 10 s |
| 3 | 100 s |

The LTMultiplier is a 6 bit unsigned integer, which represents a multiplier range from 0 to 26 - 1 = 63.

The default value of the LT field is set to the GN protocol constant itsGnDefaultPacketLifetime. The value shall be smaller than the GN protocol constant itsGnMaxPacketLifetime.

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Multiplier [0 to 63] |  |  |  |  |  | Base: 50 ms, 1 s, 10 s, 100 s |  |

Figure 10: Composition of the LT field

## 9.7	Common Header

### 9.7.1	Composition of the Common Header

The Common Header shall be present in every GeoNetworking packet and consists of the fields as depicted in figure 11.

| 0 |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| NH |  |  |  | Reserved |  |  |  | HT |  |  |  | HST |  |  |  | TC |  |  |  |  |  |  |  | Flags |  |  |  |  |  |  |  |
| PL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MHL |  |  |  |  |  |  |  | Reserved |  |  |  |  |  |  |  |

Figure 11: Common Header format

### 9.7.2	Fields of the Common Header

The Common Header shall carry the fields as specified in table 7.

Table 7: Fields of the Common Header

| Field # | Field name | Octet/bit position |  | Type | Unit | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | First | Last |  |  |  |
| 1 | NH | Octet 0 Bit 0 | Octet 0 Bit 3 | 4 bit unsigned integer | n/a | Identifies the type of header immediately following the GeoNetworking headers as specified in table 8 |
| 2 | Reserved | Octet 0 Bit 4 | Octet 0 Bit 7 | 4 bit unsigned integer | n/a | Reserved Set to 0 |
| 3 | HT | Octet 1 Bit 0 | Octet 1 Bit 3 | 4 bit unsigned integer | n/a | Identifies the type of the GeoNetworking header as specified in table 9 |
| 4 | HST | Octet 1 Bit 4 | Octet 1 Bit 7 | 4 bit unsigned integer | n/a | Identifies the sub-type of the GeoNetworking header as specified in table 9 |
| 5 | TC | Octet 2 | Octet 2 | 8 bit unsigned integer | n/a | Traffic class that represents Facilities layer requirements on packet transport. Encoding is specified in clause 9.7.5 |
| 6 | Flags | Octet 3 | Octet 3 | Bit field | n/a | Bit 0: Indicates whether the ITS-S is mobile or stationary (GN protocol constant itsGnIsMobile) Bit 1 to Bit 7: Reserve, set to 0 |
| 7 | PL | Octet 4 | Octet 5 | 16 bit unsigned integer | [octets] | Length of the GeoNetworking payload, i.e., the rest of the packet following the whole GeoNetworking header in octets, for example BTP + CAM |
| 8 | MHL | Octet 6 | Octet 6 | 8 bit unsigned integer | [hops] | Maximum hop limit (see note) |
| 9 | Reserved | Octet 7 | Octet 7 | 8 bit unsigned integer | n/a | Reserved Set to 0 |
| NOTE: The Maximum hop limit is not decremented by a GeoAdhoc router that forwards the packet. |  |  |  |  |  |  |

### 9.7.3	Encoding of the NH field in the Common Header

For the Next Header (NH) field in the Common Header the values as specified in table 8 shall be used.

Table 8: Next Header (NH) field in the GeoNetworking Common Header

| Next Header (NH) | Encoding | Description |
| --- | --- | --- |
| ANY | 0 | Unspecified |
| BTP-A | 1 | Transport protocol (BTP-A for interactive packet transport), see ETSI TS 103 836-3 [i.1] |
| BTP-B | 2 | Transport protocol (BTP-B for non-interactive packet transport), see ETSI TS 103 836-3 [i.1] |
| IPv6 | 3 | IPv6 header, see ETSI TS 103 836-3 [i.1] |

NOTE:	The Basic Header also carries a NH field.

### 9.7.4	Encoding of the HT and HST fields

For the Header Type (HT) and the Header Sub-Type (HST) fields in the Common Header the values as specified in table 9 shall be used.

Table 9: GeoNetworking Header Types and Header Sub-Types

| Header Type (HT) | Header Sub-type (HST) | Encoding | Description |
| --- | --- | --- | --- |
| ANY |  | 0 | Unspecified |
|  | UNSPECIFIED | 0 | Unspecified |
| BEACON |  | 1 | Beacon |
|  | UNSPECIFIED | 0 | Unspecified |
| GEOUNICAST |  | 2 | GeoUnicast |
|  | UNSPECIFIED | 0 | Unspecified |
| GEOANYCAST |  | 3 | Geographically-Scoped Anycast (GAC) |
|  | GEOANYCAST_CIRCLE | 0 | Circular area |
|  | GEOANYCAST_RECT | 1 | Rectangular area |
|  | GEOANYCAST_ELIP | 2 | Ellipsoidal area |
| GEOBROADCAST |  | 4 | Geographically-Scoped broadcast (GBC) |
|  | GEOBROADCAST_CIRCLE | 0 | Circular area |
|  | GEOBROADCAST_RECT | 1 | Rectangular area |
|  | GEOBROADCAST_ELIP | 2 | Ellipsoidal area |
| TSB |  | 5 | Topologically-scoped broadcast (TSB) |
|  | SINGLE_HOP | 0 | Single-hop broadcast (SHB) |
|  | MULTI_HOP | 1 | Multi-hop TSB |
| LS |  | 6 | Location service (LS) |
|  | LS_REQUEST | 0 | Location service request |
|  | LS_REPLY | 1 | Location service reply |

### 9.7.5	Encoding of the TC field

The TC field shall consist of the fields as depicted in figure 12.

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SCF | Reserved | TC ID |  |  |  |  |  |

Figure 12: Traffic Class (TC) field composition

Table 10: TC field in the GeoNetworking Common Header

| Field # | Field name | Octet/bit position |  | Type | Unit | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | First | Last |  |  |  |
| 1 | SCF | Bit 0 | Bit 0 | Bit | n/a | Indicates whether the packet shall be buffered when no suitable neighbour exists (store-carry-forward, SCF) Length: 1 bit |
| 2 | Reserved | Bit 1 | Bit 1 | Bit | n/a | Reserved. Length: 1 bit |
| 3 | TC ID | Bit 2 | Bit 7 | 6-bit unsigned integer | n/a | TC ID as specified in the media-dependent part of GeoNetworking corresponding to the interface over which the packet will be transmitted . Length: 6 bits |

The default value for the TC field is set by the GN protocol constant itsGnDefaultTrafficClass.

## 9.8	GeoNetworking packet header types

### 9.8.1	Overview

The following GeoNetworking packet header types are defined:

1)	GUC packet header (clause 9.8.2).

2)	TSB packet header (clause 9.8.3).

3)	SHB packet header (clause 9.8.4).

4)	GBC and GAC packet headers (clause 9.8.5).

5)	BEACON packet header (clause 9.8.6).

6)	LS Request and LS Reply packet headers (clause 9.8.7 and clause 9.8.8).

### 9.8.2	GUC packet header

#### 9.8.2.1	Composition of the GUC packet header

The GUC header shall be comprised of the Basic Header, the Common Header and the Extended Header as shown in figure 13.

NOTE:	The Extended Header comprises all fields except the Basic Header and the Common Header.

| 0 |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Basic Header |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Common Header |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SO PV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DE PV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 13: Packet header format: GUC

#### 9.8.2.2	Fields of the GUC packet header

The GUC packet header shall consist of the fields as specified in table 11.

Table 11: Fields of the GUC packet header

| Field # | Field name | Octet/bit position |  | Type | Unit | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | First | Last |  |  |  |
| 1 | Basic Header | Octet 0 | Octet 3 | Basic Header | n/a | Basic Header as specified in clause 9.6 Length: 4 octets |
| 2 | Common Header | Octet 4 | Octet 11 | Common Header | n/a | Common Header as specified in clause 10.3.6 Length: 8 octets |
| 3 | SN | Octet 12 | Octet 13 | 16-bit unsigned integer | n/a | Sequence number field. Indicates the index of the sent GUC packet (clause 8.4) and used to detect duplicate GeoNetworking packets (annex A) |
| 4 | Reserved | Octet 14 | Octet 15 | 16-bit unsigned integer | n/a | Reserved Set to 0 |
| 5 | SO PV | Octet 16 | Octet 39 | Long position vector | n/a | Long Position Vector containing the reference position of the source as specified in clause 9.5.2 (Long Position Vector) Length: 24 octets |
| 6 | DE PV | Octet 40 | Octet 59 | Short position vector | n/a | Short Position Vector containing the position of the destination. It shall consist of the fields as specified in clause 9.5.3 (SPV) Length: 20 octets |

### 9.8.3	TSB packet header

#### 9.8.3.1	Composition of the TSB packet header

The TSB header shall be comprised of the Basic Header, the Common Header and the Extended Header as shown in figure 14.

NOTE:	The Extended Header comprises all fields except the Basic Header and the Common Header.

| 0 |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Basic Header |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Common Header |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SO PV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 14: Packet header format: TSB

#### 9.8.3.2	Fields of the TSB packet header

The TSB packet header shall consist of the fields as specified in table 12.

Table 12: Fields of the TSB packet header

| Field # | Field name | Octet/bit position |  | Type | Unit | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | First | Last |  |  |  |
| 1 | Basic Header | Octet 0 | Octet 3 | Basic Header | n/a | Basic Header as specified in clause 9.6 Length: 4 octets |
| 2 | Common Header | Octet 4 | Octet 11 | Common Header | n/a | Common Header as specified in clause 10.3.6 Length: 8 octets |
| 3 | SN | Octet 12 | Octet 13 | 16-bit unsigned integer | n/a | Sequence number field. Indicates the index of the sent TSB packet (clause 8.4) and used to detect duplicate GeoNetworking packets (annex A) |
| 4 | Reserved | Octet 14 | Octet 15 | 16-bit unsigned integer | n/a | Reserved Set to 0 |
| 5 | SO PV | Octet 16 | Octet 39 | Long Position Vector | n/a | Long Position Vector containing the reference position of the source as specified in clause 9.5.2 (Long Position Vector) Length: 24 octets |

### 9.8.4	SHB packet header

#### 9.8.4.1	Composition of the SHB packet header

The SHB header shall consist of the Basic Header, the Common Header and the Extended Header as shown in figure 15.

NOTE:	The Extended Header comprises all fields except the Basic Header and the Common Header.

| 0 |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Basic Header |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Common Header |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SO PV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 15: Packet header format: SHB

#### 9.8.4.2	Fields of the SHB packet header

The SHB packet header shall consist of the fields as specified in table 13.

Table 13: Fields of the SHB packet header

| Field # | Field name | Octet/bit position |  | Type | Unit | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | First | Last |  |  |  |
| 1 | Basic Header | Octet 0 | Octet 3 | Basic Header | n/a | Basic Header as specified in clause 9.6 Length: 4 octets |
| 2 | Common Header | Octet 4 | Octet 11 | Common Header | n/a | Common Header as specified in clause 10.3.6 Length: 8 octets |
| 3 | SO PV | Octet 12 | Octet 35 | Long Position Vector | n/a | Long Position Vector containing the reference position of the source. It shall carry the fields as specified in clause 9.5.2 (Long Position Vector) Length: 24 octets |
| 4 | Media-dependent data | Octet 36 | Octet 39 | 32-bit unsigned integer | n/a | Used for media-dependent operations If not used, it shall be set to 0 |

### 9.8.5	GBC/GAC packet header

#### 9.8.5.1	Composition of the GBC/GAC packet header

The GBC and GAC packets shall have the same header structure. They are distinguished by the HT field in the Common Header.

The header shall be comprised of the Basic Header, the Common Header and the Extended Header as shown in figure 16.

NOTE:	The Extended Header comprises all fields except the Basic Header and the Common Header.

| 0 |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Basic Header |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Common Header |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SO PV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GeoAreaPos Latitude |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GeoAreaPos Longitude |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Distance a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Distance b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Angle |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 16: Packet header format: GBC/GAC

#### 9.8.5.2	Fields of the GBC/GAC packet header

The GBC/GAC packet header shall consist of the fields as specified in table 14.

Table 14: Fields of the GBC/GAC packet header

| Field # | Field name | Octet/bit position |  | Type | Unit | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | First | Last |  |  |  |
| 1 | Basic Header | Octet 0 | Octet 3 | Basic Header | n/a | Basic Header as specified in clause 9.6 Length: 4 octets |
| 2 | Common Header | Octet 4 | Octet 11 | Common Header | n/a | Common Header as specified in clause 10.3.6 Length: 8 octets |
| 3 | SN | Octet 12 | Octet 13 | 16-bit unsigned integer | n/a | Sequence number field. Indicates the index of the sent GBC/GAC packet (clause 8.4) and used to detect duplicate GeoNetworking packets (annex A) |
| 4 | Reserved | Octet 14 | Octet 15 | 16-bit unsigned integer | n/a | Reserved Set to 0 |
| 5 | SO PV | Octet 16 | Octet 39 | Long position vector | n/a | Long Position Vector containing the reference position of the source as specified in clause 9.5.2 (Long Position Vector) Length: 24 octets |
| 6 | GeoAreaPos Latitude | Octet 40 | Octet 43 | 32-bit signed integer | [1/10 micro-degree] | WGS 84 [i.4] latitude for the centre position of the geometric shape as defined in ETSI EN 302 931 [1] in 1/10 micro degree |
| 7 | GeoAreaPos Longitude | Octet 44 | Octet 47 | 32-bit signed integer | [1/10 micro-degree] | WGS 84 [i.4] longitude for the centre position of the geometric shape as defined in ETSI EN 302 931 [1] in 1/10 micro degree |
| 8 | Distance a | Octet 48 | Octet 49 | 16-bit unsigned integer | [m] | Distance a of the geometric shape as defined in ETSI EN 302 931 [1] in metres |
| 9 | Distance b | Octet 50 | Octet 51 | 16-bit unsigned integer | [m] | Distance b of the geometric shape as defined in ETSI EN 302 931 [1] in metres |
| 10 | Angle | Octet 52 | Octet 53 | 16-bit unsigned integer | [°] | Angle of the geometric shape as defined in ETSI EN 302 931 [1] in degrees from North |
| 11 | Reserved | Octet 54 | Octet 55 | 16-bit unsigned integer | n/a | Reserved Set to 0 |

In case of a circular area (GeoNetworking packet sub-type HST = 0), the fields shall be set to the following values:

1)	Distance a is set to the radius r.

2)	Distance b is set to 0.

3)	Angle is set to 0.

### 9.8.6	BEACON packet header

#### 9.8.6.1	Composition of the BEACON packet header

A BEACON packet shall consist of the Basic Header, the Common Header, and the Extended Header as shown in figure 17.

NOTE:	The Extended Header comprises all fields except the Basic Header and the Common Header.

| 0 |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Basic Header |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Common Header |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SO PV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 17: Packet header format: BEACON

#### 9.8.6.2	Fields of the BEACON packet header

The BEACON shall consist of the fields of the Basic Header, the Common Header and the Extended Header as specified in table 15.

Table 15: Fields of the BEACON packet header

| Field # | Field name | Octet/bit position |  | Type | Unit | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | First | Last |  |  |  |
| 1 | Basic Header | Octet 0 | Octet 3 | Basic Header | n/a | Basic Header as specified in clause 9.6 Length: 4 octets |
| 2 | Common Header | Octet 4 | Octet 11 | Common Header | n/a | Common header as specified in clause 10.3.6 Length: 8 octets |
| 3 | SO PV | Octet 12 | Octet 35 | Long Position Vector | n/a | Long Position Vector containing the reference position of the source. It shall carry the fields as specified in clause 9.5.2 (Long Position Vector) Length: 24 octets |

### 9.8.7	LS Request packet header

#### 9.8.7.1	Composition of the LS Request packet header

The LS Request packet header shall be comprised of the Common Header and the Extended Header as shown in figure 18.

NOTE:	The Extended Header comprises all fields except the Basic Header and the Common Header.

| 0 |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Basic Header |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Common Header |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SO PV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Request GN_ADDR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 18: Packet header format: LS Request

#### 9.8.7.2	Fields of the LS Request packet header

The LS Request packet header shall carry the fields as specified in table 16.

Table 16: Fields of the LS Request packet header

| Field # | Field name | Octet/bit position |  | Type | Unit | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | First | Last |  |  |  |
| 1 | Basic Header | Octet 0 | Octet 3 | Basic Header | n/a | Basic Header as specified in clause 9.6 Length: 4 octets |
| 2 | Common Header | Octet 4 | Octet 11 | Common Header | n/a | Common Header as specified in clause 10.3.6 Length: 8 octets |
| 3 | SN | Octet 12 | Octet 13 | 16-bit unsigned integer | n/a | Sequence number field. Indicates the index of the sent LS Request packet (clause 8.4) and used to detect duplicate GeoNetworking packets (annex A) |
| 4 | Reserved | Octet 14 | Octet 15 | 16-bit unsigned integer | n/a | Reserved Set to 0 |
| 5 | SO PV | Octet 16 | Octet 39 | Long position vector | n/a | Long Position Vector containing the position of the source as specified in clause 9.5.2 (Long Position Vector) Length: 24 octets |
| 6 | Request GN_ADDR | Octet 40 | Octet 47 | 64-bit address | n/a | The GN_ADDR address for the GeoAdhoc router entity for which the location is being requested |

### 9.8.8	LS Reply packet header

#### 9.8.8.1	Composition of the LS Reply packet header

The LS Reply packet header shall be comprised of the Basic Header, the Common Header and the Extended Header as shown in figure 19.

NOTE:	The Extended Header comprises all fields except the Basic Header and the Common Header.

| 0 |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Basic Header |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Common Header |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SO PV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DE PV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 19: Packet header format: LS Reply

#### 9.8.8.2	Fields of the LS Reply packet header

The LS Reply packet header shall carry the fields as specified in table 17.

Table 17: Fields of the LS Reply packet header

| Field # | Field name | Octet/bit position |  | Type | Unit | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | First | Last |  |  |  |
| 1 | Basic Header | Octet 0 | Octet 3 | Basic Header | n/a | Basic Header as specified in clause 9.6 Length: 4 octets |
| 2 | Common Header | Octet 4 | Octet 11 | Common Header | n/a | Common Header as specified in clause 10.3.6 Length: 8 octets |
| 3 | SN | Octet 12 | Octet 13 | 16-bit unsigned integer | n/a | Sequence number field. Indicates the index of the sent LS Reply packet (clause 8.4) and used to detect duplicate GeoNetworking packets (annex A) |
| 4 | Reserved | Octet 14 | Octet 15 | 16-bit unsigned integer | n/a | Reserved Set to 0 |
| 5 | SO PV | Octet 16 | Octet 39 | Long position vector | n/a | Long Position Vector containing the reference position of the source, which represents the Request GN_ADDR in the corresponding LS Request, as specified in clause 9.5.2 (Long Position Vector) Length: 24 octets |
| 6 | DE PV | Octet 40 | Octet 59 | Short position vector | n/a | Short Position Vector containing the reference position of the destination. It shall carry the fields as specified in clause 9.5.3 (Short Position Vector) Length: 20 octets |

# 10	Protocol operation

## 10.1	General

This clause specifies the media-independent operations of the GeoNetworking protocol.

The operations include:

1)	Network management:

Address configuration (clause 10.2.1);

Local position vector and time update (clause 10.2.2);

Beaconing (clause 10.2.3);

Location service (clause 10.2.4).

2)	Packet handling:

GUC (clause 10.3.8);

TSB (clause 10.3.9);

SHB (clause 10.3.10);

GBC (clause 10.3.11);

GAC (clause 10.3.12).

- 3)		MCO management:

MCO parameters updates (clause 10.4.2);

Network updates (clause 10.4.3);

Handling of packets without assigned ALI (clause 10.4.4).

## 10.2	Network management

### 10.2.1	Overview

In the Management plane of the Networking & Transport layer, network management for the GeoNetworking protocol is handled by the GN Core and the GAGHs as depicted in figure 20. The GN Core provides the GeoNetworking address and current local position and time information upon request from a GAGH as specified in clause 10.2.1 and 10.2.2, respectively.

Each GAGH applies the duplicate address detection mechanism as specified in clause 10.2.1.5 performs beaconing, and realizes LS operations as specified in clause 10.2.3 and 10.2.4, respectively.

Figure 20: Overview of Network Management functions and control flow

### 10.2.1	Address configuration

#### 10.2.1.1	General

When initialized, the GN Core shall have a self-assigned initial GeoNetworking address with the format specified in clause 6. GeoNetworking defines three methods for the configuration of the local GN_ADDR:

1)	Auto-address configuration (clause 10.2.1.2);

2)	Managed address configuration (clause 10.2.1.3);

3)	Anonymous address configuration (clause 10.2.1.4).

The method is defined in the GN protocol constant itsGnLocalAddrConfMethod.

In the auto-address configuration, the GeoNetworking address cannot be changed. In the managed address configuration, the initial GeoNetworking address (clause 10.2.1.3.2) can be updated (clause 10.2.1.3.3) by the GN Core. In the anonymous address configuration, the address is configured by the security entity and updated to the GAGHs by the GN Core.

Operations for duplicate address detection (DAD) are specified in clause 10.2.1.5. DAD is applied for auto-address configuration (clause 10.2.1.2) and managed address configuration (clause 10.2.1.3).

NOTE: 	For multi-channel operation, an ITS-S with several GAGHs uses a single GeoNetworking address (see clause 6 “GeoNetworking address”).

#### 10.2.1.2	Auto-address configuration

The auto-address configuration method shall be used if the GN protocol constant itsGnLocalAddrConfMethod is set to AUTO (0).

When activated, a GAGH shall assign the MID field of the local GN_ADDR from the GN protocol constant itsGnLocalGnAddr.

NOTE:	The setting of the GN protocol constant itsGnLocalGnAddr is implementation dependent. One example implementation is the usage of randomly-generated addresses.

The local GN_ADDR shall not be changed unless the GN protocol constant itsGnLocalAddrConfMethod is set to MANAGED (1) or ANONYMOUS (2) or when DAD is invoked.

#### 10.2.1.3	Managed address configuration

##### 10.2.1.3.1	General Requirements

The managed address configuration method shall be used if the GN protocol constant itsGnLocalAddrConfMethod is set to MANAGED (1).

With managed address configuration, the GN Core is responsible for providing the MID field of the GN_ADDR.

##### 10.2.1.3.2	Initial address configuration

At startup, a GAGH shall request an MID field for the GN_ADDR from the GN Core (see annex K.2, service primitive CORE_GAGH.request). The GN Core is responsible for generating the appropriate GeoNetworking address (see annex K.3 using the service primitive CORE_GAGH.response).

##### 10.2.1.3.3	Address update

The update of the MID field of the local GN_ADDR may be triggered by the GN Core using an unsolicited CORE_GAGH.response to the active GAGHs.

Upon reception of the CORE_GAGH.response, a GAGH shall update its local GN_ADDR.

NOTE 1:	For privacy reasons, the GN_ADDR may be derived from the current authorization ticket. The frequency of update and the algorithm of generating pseudonyms are beyond the scope of the present document.

NOTE 2:	From communication point of view, a frequent update of the GN_ADDR may impair the performance of the GeoNetworking protocol.

#### 10.2.1.4	Anonymous address configuration

The anonymous address configuration method shall be used if the GN protocol constant itsGnLocalAddrConfMethod is set to ANONYMOUS (2). This method allows for configuration of anonymous addresses controlled by the security entity. The services are provided via the CORE_SEC interface .

In this method, the GN Core subscribes to the ID-CHANGE-SUBSCRIBE service at the security entity (annex O). In one possible implementation, it may register a callback function, which is executed when the pseudonym is changed.

At startup, the GN Core shall execute the following operations:

1)	subscribe to the IDCHANGE-SUBSCRIBE service provided by the security entity and send a service primitive SN-IDCHANGE-SUBSCRIBE.request as specified in annex O;

2)	process the service primitive SN-IDCHANGE-SUBSCRIBE.confirm that returns the subscription handle as specified in annex O;

3)	process the service primitive SN-IDCHANGE-EVENT.indication that provides the parameter id as specified in annex O. The GN Core shall update the GN Address of the active GAGHs using an unsolicited CORE_GAGH.response;

4)	generate the service primitive SN-IDCHANGE-EVENT.response as specified in annex O to acknowledge the given command.

When the GeoAdhoc router is shutdown or restarted, the GN Core should execute the following operations:

1)	unsubscribe from the IDCHANGE-SUBSCRIBE service provided by the security entity and send a service primitive SN-IDCHANGE-UNSUBSCRIBE.request as specified in annex O.

.

#### 10.2.1.5	Duplicate address detection

In order to achieve uniqueness of the GeoNetworking address configuration with the auto-address configuration method, i.e., if the GN protocol constant itsGnLocalAddrConfMethod is set to AUTO (0), each GAGH shall execute the following operations for DAD:

1)	Upon reception of a GeoNetworking packet, a GAGH compares:

a)	its local GN_ADDR and the GN_ADDR of the SO carried in the GeoNetworking packet header; and

b)	its local link layer address (i.e., the MID field of the GN_ADDR, clause 6), with the sender link layer address of the frame.

2)	If a conflict is detected, the GAGH shall request a new MID field for the GeoNetworking address from the GN Core entity using the service primitive CORE_GAGH.request (annex K.2) indicating Duplicate address as the Request cause.

In case of conflict, the GN Core shall update the new GN_ADDR to all active GAGHs using an unsolicited CORE_GAGH.response.

NOTE 1:	In case the GN MID changes, the link layer address also changes. In one possible implementation the link layer address is set from the GAGHs.

### 10.2.2	Ego position vector and time update

#### 10.2.2.1	Overview

Ego position vector and time are set by the GN Core for the GAGH via the CORE_GAGH interface (annex K.3).

#### 10.2.2.2	Ego Position Vector update

For position update, the GN Core shall send an unsolicited CORE_GAGH.response with the Ego position vector parameter (annex K.3) to the GeoAdhoc router. Upon reception of the CORE_GAGH.response with the Ego position vector parameter, the GeoAdhoc router shall update its EPV (clause 8.3).

As specified in clause 8.3.3, the EPV shall be updated with a minimum frequency of the GN protocol constant itsGnMinUpdateFrequencyEPV.

NOTE: 	The ego position vector is not GAGH-specific.

#### 10.2.2.3	Time update

For time update, the GN_CORE entity shall send an unsolicited CORE_GAGH.response with the Time parameter (annex K.3) to the GeoAdhoc router. Upon reception of the CORE_GAGH.response with the Time parameter, the GeoAdhoc router should set its local system time at a reasonable time interval.

It should be noted that time management shall support TAI.

NOTE 1: 	The time is not GAGH-specific.

NOTE 2:	Details of the system time management and usage are implementation specific.

### 10.2.3	Beaconing

Beaconing is used to periodically advertise a GeoAdhoc router's position vector to its neighbours.

Each active GAGH shall send a BEACON periodically unless the GAGH dissiminated another GeoNetworking packet that carries the GeoAdhoc router's EPV. At startup a GeoAdhoc router shall sent an initial beacon to announce its presence with every active GAGHs to other GeoAdhoc routers.

NOTE 1: 	In one possible implementation a timer schedules the transmission of BEACON packets and is reset upon transmission of a GeoNetworking packet that carries a LPV, i.e., a SHB packet.

NOTE 2: Per Access layer technology, a default ALI to disseminate the BEACONS may be configured.

### 10.2.4	Location service

The location service is used if a GeoAdhoc router needs to determine the position of another GeoAdhoc router using one of its GAGH. This is the case if a GeoAdhoc router is in the process to send a T/GN6-SDU as a GUC packet to another GeoAdhoc router, 
i.e., from the source to the destination, and does not have the position information for the destination in its LocT.

The execution of a location service is fully transparent to protocol entities of higher layers and resides on top of the forwarding function.

The location service is based on the exchange of control packets between GeoAdhoc routers (figure 21). The querying GeoAdhoc router (source) issues a LS Request packet with the GN_ADDR of the sought GeoAdhoc router (destination). The LS Request packet is forwarded by intermediate GeoAdhoc routers (forwarders) until it reaches the destination. The destination replies with a LS Reply packet.

Figure 21: Message sequence chart for the location service
(example scenario with two forwarders)

## 10.3	Packet handling

### 10.3.1	Overview

This clause defines the behaviour of the protocol in the source, forwarder and destination. Packet handling includes the procedures to determine the destination (GeoAdhoc router, geographical area) of the GeoNetworking packet, execute security functions, execute functions that are specific to the packet type and associated ALIG, and pass the packet to the ALI via the NET_ACC_DATA interface (figure 22).

The following GN packet types are handled by every GAGH:

- Beacon packet (clause 10.3.6);

- LS packet (clause 10.3.7);

- GUC packet (clause 10.3.8);

- TSB packet (clause 10.3.9);

- SHB packet (clause 10.3.10);

- GBC packet (clause 10.3.11); and

- GAC packet (clause 10.3.12).

The packet handling is further specified in the following clauses.

Figure 22: GeoNetworking packet handling

### 10.3.2	Basic Header field settings

For all GeoNetworking packets, the fields of the Basic Header shall be set as specified in table 18.

Table 18: Field settings for the Basic Header

| Field name | Field setting | Description |
| --- | --- | --- |
| Version | GN protocol constant itsGnProtocolVersion | Version of the GeoNetworking protocol |
| NH | 1 (Common Header) if TRANSP_CORE.request parameter Security profile indicates that the packet is unsecured. 2 (Secured Packet) if TRANSP_CORE.request parameter Security profile indicates that the packet is secured. For Header type HT = 1 (BEACON) set the value of the NH field to 1 (Common Header) if GN protocol constant itsGnSecurity is DISABLED or to 2 (Secured Header) if it is ENABLED | Next header (table 5 in clause 9.6.3) |
| Reserved | Set to 0 | Reserved Set to 0 |
| LT | Source: - Value of optional Maximum packet lifetime parameter from service primitive TRANSP_CORE.request, or - GN protocol constant itsGnDefaultPacketLifetime if the Maximum packet lifetime parameter is not set, or if Header type HT = 1 (BEACON) Forwarder: When the GeoNetworking packet is stored in a packet buffer, the value of the received LT field shall be reduced by the queuing time in the packet buffer | Lifetime of the packet |
| RHL | Shall always be smaller than or equal to the maximum hop limit (MHL) in the Common Header (clause 10.3.4) Source: - 1 if parameter Packet transport type in the service primitive TRANSP_CORE.request is SHB, or if Header type HT = 1 (BEACON) - Value of optional Maximum hop limit parameter from service primitive TRANSP_CORE.request - Otherwise GN protocol constant itsGnDefaultHopLimit if TRANSP_CORE.request parameter Packet transport type is GUC, GBC, GBC or TSB Forwarder: Decrement RHL by one | Remaining hop limit |

### 10.3.3	Basic Header processing

When a GAGH in a forwarder, receiver or destination GeoAdhoc router processes a Basic Header, the GAGH shall execute the following operations upon reception of a GeoNetworking packet:

1)	check the Version field of the Basic Header:

a)	if the value of the Version field equals the GN protocol constant itsGnProtocolVersion, continue the execution of further steps;

NOTE 1:	For other values of the Version field, the implementation may select the appropriate protocol decoder.

2)	check the NH field of the Basic Header (table 5):

a)	if NH = 0 (ANY) or NH = 1 (Common Header), proceed processing the Common Header as specified in clause 10.3.5;

b)	if NH = 2 (Secured Packet):

i)	execute the SN-DECAP service as specified in annex O and the parameter setting in table 19;

Table 19: Parameter settings in the service primitive SN-DECAP.request

| Parameter name | Parameter setting |
| --- | --- |
| sec_packet_length | Length of the Secured Packet [octets] |
| sec_packet | The Secured Packet to be verified |

ii)	process the service primitive SN-DECAP.confirm;

iii)	if the parameter report of the service primitive SN-ENCAP.confirm indicates that the packet was correctly verified and decrypted (parameter report = SUCCESS):

- process the parameters plaintext_packet, certificate_id, its_aid_length, its_aid, permissions length and permissions carried in the SN-DECAP.confirm parameter (see annex O) and proceed processing the Common Header as specified in clause 10.3.5;

NOTE 2:	In an implementation the Secured Packet should be kept, in order to forward the signed/encrypted packet without additional security processing.

iv)	otherwise (parameter report != SUCCESS):

1)	if the GN protocol constant itsGnSnDecapResultHandling is set to STRICT (0) discard the packet and omit the execution of further steps;

2)	if the GN protocol constant itsGnSnDecapResultHandling is set to NON-STRICT (1) pass the payload of the GN-PDU to the upper protocol entity by means of a service primitive TRANSP_CORE.indication.

NOTE 3:	The purpose for passing the GN-PDU to the upper protocol entity with incorrect result of verification and decryption may improve security assessment of messages at the Facilities layer. Details are implementation specific.

### 10.3.4	Common Header field settings

For all GeoNetworking packets, the fields of the Common Header shall be set as specified in table 20.

Table 20: Field settings for the Common Header

| Field name | Field setting | Description |
| --- | --- | --- |
| NH | 0 (ANY) if Header type HT = 1 (BEACON) 1 (BTP-A) if TRANSP_CORE.request parameter Upper protocol entity is BTP-A 2 (BTP-B) if TRANSP_CORE.request parameter Upper protocol entity is BTP-B 3 (IPv6) if TRANSP_CORE.request parameter Upper protocol entity is Ipv6 | Next header (table 8 in clause 9.7.3) |
| Reserved | Set to 0 | Reserved |
| HT | 1 (BEACON) if the GeoNetworking packet is a Beacon 2 (GEOUNICAST) if TRANSP_CORE.request parameter Packet transport type is GeoUnicast 3 (GEOANYCAST) if TRANSP_CORE.request parameter Packet transport type is GeoAnycast 4 (GEOBROADCAST) if TRANSP_CORE.request parameter Packet transport type is GBC 5 (TSB) if TRANSP_CORE.request parameter Packet transport type is TSB 5 (TSB) if TRANSP_CORE.request parameter Packet transport type is SHB 6 (LS) if the GeoNetworking packet is a LS Request or a LS Reply packet | Header type (table 9 in clause 9.7.4) |
| HST | As specified in table 9 in clause 9.7.4 | Header sub-type |
| TC | Traffic class defined in service primitive TRANSP_CORE.request parameter Traffic class or GN protocol constant itsGnDefaultTrafficClass if the service primitive TRANSP_CORE.request parameter Traffic class is not available | Traffic class encoding specified in clause 9.7.5 |
| Flags | Bit 0: GN protocol constant itsGnIsMobile Bit 1 to 7: Reserved; set to 0 | Bit 0: Indicates whether the ITS-S is mobile or stationary |
| PL | 0 for Beacon, LS Request and LS Reply packets Size of T/GN6-SDU defined in service primitive TRANSP_CORE.request otherwise | Payload length in octets |
| MHL | Source: 1 if TRANSP_CORE.request parameter Packet transport type is SHB or GeoNetworking packet is Beacon Value of optional Maximum hop limit parameter from service primitive TRANSP_CORE.request GN protocol constant itsGnDefaultHopLimit if TRANSP_CORE.request parameter Packet transport type is GUC, GAC, GBC or TSB GN protocol constant itsGnDefaultHopLimit for LS Request and LS Reply packets | Maximum hop limit |
| Reserved | Set to 0 | Reserved |
| NOTE: TSB and SHB carry the same value in the HT field (equals 5), but have a different value in the HST field, i.e., HST = 0 for SHB and HST = 1 for TSB (table 9 in clause 9.7.4). |  |  |

### 10.3.5	Common Header processing

When a GAGH (forwarder, receiver, destination) processes a Common Header, the GAGH shall execute the following operations upon reception of a GeoNetworking packet:

- check the MHL field of the Common Header:

a)	compare MHL with the value of the RHL field of the Basic Header; if MHL < RHL discard the packet and omit the execution of further steps;

- process the BC forwarding packet buffer:

a)	if the BC forwarding packet buffer (clause 8.6) is not empty, forward the stored packets and remove them from the BC forwarding packet buffer;

NOTE 1:	The forwarding algorithm having caused the buffering needs to be re-executed.

- check the HT field of the Common Header:

a)	if HT = 0 (ANY) discard the packet and omit the execution of further steps;

b)	if HT = 1 (BEACON) execute the steps specified in clause 10.3.6;

c)	if HT = 2 (GEOUNICAST) execute the steps specified in clause 10.3.8.3 and clause 10.3.8.4;

d)	if HT = 3 (GEOANYCAST) execute the steps specified in clause 10.3.12.3;

e)	if HT = 4 (GEOBROADCAST) execute the steps specified in clause 10.3.11.3;

f)	if HT = 5 (TSB) execute the steps specified in clause 10.3.9.3 (HST = MULTI_HOP) and clause 10.3.10.3 (HST = SINGLE_HOP);

g)	if HT = 6 (LS) execute the steps specified in clause 10.3.7.2 and clause 10.3.7.3.

NOTE 2:	In 3a) to 3g) only the steps after "Common Header processing" need to be executed in the corresponding clauses defining the packet handling procedures.

### 10.3.6	Beacon packet handling

#### 10.3.6.1	General

Beaconing is used to periodically advertise a GeoAdhoc router's position vector to its neighbours.

A BEACON packet shall be sent periodically by each active GAGH unless the GAGH sends another GeoNetworking packet that carries the EPV.

NOTE:	In one possible implementation a timer is reset upon transmission of a SHB packet.

#### 10.3.6.2	Source operations

Upon activation, a GAGH shall execute the following operations:

1)	create a GN-PDU with a Beacon packet header (clause 9.8.6):

a)	set the fields of the Basic Header (clause 10.3.2);

b)	set the fields of the Common Header (clause 10.3.4);

c)	set the fields of the Beacon Extended Header (table 21).

Table 21: Field settings for the Beacon Extended Header

| Field name | Field setting | Description |
| --- | --- | --- |
| SO PV | Actual values of the EPV as specified in clause 8.3 | PV of the ego GeoAdhoc router (source of the GeoNetworking packet) |

2)	if the GN protocol constant itsGnSecurity is set to ENABLED:

a)	send a service primitive SN-ENCAP.request as specified in annex O and the parameter setting in table 22;

Table 22: Parameter settings in the service primitive SN-ENCAP.request 
for a BEACON packet

| Parameter name | Parameter setting (in case of security as specified in note) |
| --- | --- |
| tbe_packet_length | Length of the Beacon |
| tbe_packet | Common header + Beacon Extended header to be signed |
| sec_profile | If the GN protocol constant itsGnSecurity is set to ENABLED, the value of the parameter sec_profile is set to a default security profile. The specification of the default security profile is out of scope of the present document. |
| Its_aid _length | 2 |
| its_aid | 141 = 0x8d |
| permissions_length | 0 |
| permissions | Void |
| context_information | 0 |
| target_id_list_length | 0 |
| target_id_list | Void |
| NOTE: See ETSI TS 102 965 [i.7]. The permissions_length parameter being set to zero indicates that the SSP is null and absent. |  |

b)	process the service primitive SN-ENCAP.confirm and append the Secured Packet carried by the sec_packet parameter of the service primitive SN-ENCAP.confirm to the Basic Header;

3)	execute media-dependent procedures; if the GN protocol constant itsGnIfType is set to:

a)	UNSPECIFIED then no media-dependent procedures are specified;

b)	for other values of itsGnIfType media dependent procedures may be defined elsewhere;

4)	pass the GN-PDU to the ALI via the NET_ACC_DATA interface and set the destination address to the Broadcast address of the ALI;

5)	initialize the timer for the periodic transmission of beacons Tbeacon with a timeout set to (itsGnBeaconServiceRetransmitTimer + RAND[0,itsGnBeaconServiceMaxJitter]), whereas itsGnBeaconServiceRetransmitTimer and itsGnBeaconServiceMaxJitter represent GN protocol constant values.

NOTE 1:	The RAND function introduces a random component for the timer to avoid synchronization issues among GeoAdhoc routers.

If the timer Tbeacon expires, the source shall execute the following operations:

1)	create a GN-PDU with a BEACON packet header (clause 9.8.6):

a)	set the fields of the Basic Header to the values specified in clause 10.3.2;

b)	set the fields of the Common Header to the values specified in clause 10.3.4;

c)	set the fields of the Beacon Extended Header as specified in table 21;

2)	if the GN protocol constant itsGnSecurity is set to ENABLED:

a)	send a service primitive SN-ENCAP.request as specified in annex O and the parameter setting in table 22;

b)	process the service primitive SN-ENCAP.confirm and append the Secured Packet carried by the sec_packet parameter of the service primitive SN-ENCAP.confirm to the Basic Header;

3)	execute media-dependent procedures; if the GN protocol constant itsGnIfType is set to:

a)	UNSPECIFIED then no media-dependent procedures are specified;

b)	for other values of itsGnIfType media dependent procedures may be defined elsewhere;

4)	pass the GN-PDU to the ALI via the NET_ACC_DATA interface and set the destination address to the Broadcast address of the ALI;

5)	set the timer Tbeacon to a timeout value (itsGnBeaconServiceRetransmitTimer + RAND[0,itsGnBeaconServiceMaxJitter]).

NOTE 2:	The GAGH resets the timer Tbeacon for every sent GeoNetworking packet that carries a LPV.

#### 10.3.6.3	Receiver operations

Receiver operations of Beacon packets are identical to the handling procedures of the SHB packet (clause 10.3.10.3) except step 8 (pass the payload of the GN-PDU to the upper protocol entity).

### 10.3.7	Location service packet handling

#### 10.3.7.1	Source operations

##### 10.3.7.1.1	Overview

Three cases are distinguished for the source operations:

1)	source operation for initial LS Request (clause 10.3.7.1.2);

2)	source operation for LS Request re-transmission (clause 10.3.7.1.3);

3)	source operation for LS Reply (clause 10.3.7.1.4).

##### 10.3.7.1.2	Operation for initial LS Request

When a source has a T/GN6-SDU to send and has no position vector information for the destination address, the source shall invoke the location service and shall execute the following operations:

1)	check whether a LS for the sought GN_ADDR is in progress, i.e., the flag LS_pending is set TRUE:

a)	if LS_pending is TRUE for the sought GN_ADDR, the packet shall be buffered in the LS packet buffer (clause 8.5) and the execution of the next steps shall be omitted;

2)	create a GN-PDU with the T/GN6-SDU as payload and a TSB packet header (clause 9.8.3):

a)	set the fields of the Basic Header to the values specified in clause 10.3.2;

b)	set the fields of the Common Header to the values specified in clause 10.3.4;

c)	set the fields of the LS Request Extended Header to the values specified in table 23;

Table 23: Field settings for the LS Request Extended Header

| Field name | Field setting | Description |
| --- | --- | --- |
| SN | Actual value of the local sequence number as specified in clause 8.4 | Sequence number of the packet |
| SO PV | Actual values of the EPV as specified in clause 8.3 | Position vector containing the reference position of the ego GeoAdhoc router (source of the GeoNetworking packet) |
| Request GN_ADDR | GN_ADDR from TRANSP_CORE.request parameter Destination (annex J.2) | GeoNetworking address of the sought GeoAdhoc router |

3)	if the GN protocol constant itsGnSecurity is set to ENABLED:

a)	send a service primitive SN-ENCAP.request as specified in annex O the parameter setting in table 24;

Table 24: Parameter settings in the service primitive SN-ENCAP.request 
for a LS Request packet

| Parameter name | Parameter setting (in case of security as specified in note) |
| --- | --- |
| tbe_packet_length | Length of the LS Request |
| tbe_packet | Common header + LS Request Extended header to be signed |
| sec_profile | If the GN protocol constant itsGnSecurity is set to ENABLED, the value of the parameter sec_profile is set to a default security profile. The specification of the default security profile is out of scope of the present document |
| its_aid _length | 2 |
| its_aid | 141 = 0x8d |
| permissions_length | 0 |
| permissions | void |
| context_information | 0 |
| target_id_list_length | 0 |
| target_id_list | void |
| NOTE: See ETSI TS 102 965 [i.7]. The permissions_length parameter being set to zero indicates that the SSP is null and absent. |  |

b)	process the service primitive SN-ENCAP.confirm and append the Secured Packet carried by the sec_packet parameter of the service primitive SN-ENCAP.confirm to the Basic Header;

4)	execute media-dependent procedures; if the Communication profile parameter of the service primitive
TRANSP_CORE.request is set to:

a)	UNSPECIFIED then no media-dependent procedures are specified;

b)	for other values media dependent procedures may be defined elsewhere;

5)	start a timer TLS, GN_ADDR with a timeout set to the value of the GN protocol constant itsGnLocationServiceRetransmitTimer;

6)	initialize the LS retransmit counter for the GeoAdhoc router GN_ADDR RTCLS, GN_ADDR to 0;

7)	add a LocTE for the sought GN_ADDR in the LocT and set the flag LS_pending to TRUE;

8)	pass the GN-PDU to the ALI via the NET_ACC_DATA interface and set the destination address to the Broadcast address of the ALI.

##### 10.3.7.1.3	Operation for LS Request re-transmission

If the timer TLS, GN_ADDR for the GN_ADDR expires, the source shall execute the following operations:

1)	check the retransmit counter RTCLS, GN_ADDR:

a)	if the retransmit counter is less than the maximum number of LS retransmissions set by the GN protocol constant itsGnLocationServiceMaxRetrans, 
i.e., RTCLS, GN_ADDR < itsGnLocationServiceMaxRetrans, the GAGH shall:

i)	re- issue a LS Request packet with the format as specified in clause 9.8.7 as a TSB packet and the field setting for the LS Request Extended Header as specified in clause 10.3.7.1.2, table 23;

ii)	restart the timer TLS, GN_ADDR with a timeout set to the value of the GN protocol constant itsGnLocationServiceRetransmitTimer; and

iii)	increment the retransmit counter RTCLS, GN_ADDR;

b)	if the retransmit counter is greater than or equals the maximum number of LS retransmissions set by the GN protocol constant itsGnLocationServiceMaxRetrans, 
i.e., RTCLS, GN_ADDR ≥ itsGnLocationServiceMaxRetrans, the GAGH shall:

i)	remove the pending packets for the sought GN_ADDR from the LS packet buffer (clause 8.5);

ii)	remove the LocTE for the sought GN_ADDR.

##### 10.3.7.1.4	Operation for LS Reply

If the source receives a LS Reply packet for the sought GN_ADDR, the source shall execute the following operations:

1)	Basic Header processing (clause 10.3.3);

2)	Common Header processing (clause 10.3.5);

3)	execute DPD as specified in annex A.2; if the LS Reply packet is a duplicate, discard the packet and omit the execution of further steps;

4)	update PV(SO) in the LocT with the SO PV of the LS Reply Extended Header (annex C.2);

5)	update PDR(SO) in the LocT (annex B.2);

6)	flush the LS packet buffer (clause 8.5) for the sought GN_ADDR and forward the stored packets;

7)	flush packet buffers (SO LS packet buffer, SO UC forwarding packet buffer):

a)	if SO LS_pending is TRUE:

i)	forward the packets in the SO LS packet buffer and remove them from the buffer (clause 8.5);

ii)	set SO LS_pending to false;

b)	if the UC forwarding packet buffer (clause 8.6) for SO is not empty, forward the stored packets and remove them from the UC forwarding packet buffer;

8)	set the flag LS_pending for the sought GN_ADDR to false;

9)	stop the timer TLS, GN_ADDR;

10)	reset the re-transmit counter RTCLS, GN_ADDR.

#### 10.3.7.2	Forwarder operations

If a GAGH receives a LS Request packet and the Request GN_ADDR field in the LS Request header does not match its GN_ADDR, the GAGH shall handle the packet according to the packet handling procedure for TSB (clause 10.3.9.3), except step 7 for passing the payload of the GN-PDU to the upper protocol entity.

If a GAGH receives a LS Reply packet and the GN_ADDR in the DE PV of the LS Reply packet does not match its GN_ADDR, the GAGH shall handle the packet according to the packet handling operations (forwarder) for GUC (clause 10.3.8.3).

NOTE:	The Basic Header and Common Header processing are part of the GeoUnicast and TSB packet handling procedure, respectively.

#### 10.3.7.3	Destination operations

On reception of a LS Request packet, the GAGH shall check the Request GN_ADDR field. If this MID field matches the MID field of its GN_ADDR, the GAGH shall execute the following operations:

1)	Basic Header processing (clause 10.3.3);

2)	Common Header processing (clause 10.3.5);

3)	execute DPD as specified in annex A.2; if the LS Request packet is a duplicate, discard the packet and omit the execution of further steps;

4)	execute DAD as specified in clause 10.2.1.5;

5)	if the LocTE(SO) does not exist:

a)	create PV(SO) in the LocT with the SO PV fields of the LS Request Extended Header (annex C.2);

b)	set the IS_NEIGHBOUR flag of the SO LocTE to FALSE;

c)	set PDR(SO) in the SO LocTE (annex B.2);

6)	if the LocTE(SO) exists:

a)	update PV(SO) in the LocT with the SO PV fields of the LS Request Extended Header (annex C.2);

b)	update PDR(SO) in the SO LocTE (annex B.2);

NOTE:	The IS_NEIGHBOUR flag of the SO LocTE remains unchanged.

7)	if the return value of the forwarding algorithm is 0 (packet is buffered in the UC forwarding packet buffer) or 
-1 (packet is discarded), omit the execution of further steps;

8)	create a GN-PDU with a GUC packet header (clause 9.8.2):

a)	set the fields of the Basic Header (clause 10.3.2);

b)	set the fields of the Common Header (clause 10.3.4);

c)	set the fields of the GUC Extended Header (table 25);

Table 25: Field settings for the GUC Extended Header in LS

| Field name | Field setting | Description |
| --- | --- | --- |
| SN | Actual value of the local sequence number (clause 8.4) | Sequence number of the packet |
| SO PV | Actual values of the EPV (clause 8.3) | Position vector containing the reference position of the ego GeoAdhoc router (source of the GeoNetworking packet) |
| DE PV | Actual values of the LocTE for the destination | Position vector containing the reference position of the destination |

9)	execute the forwarding algorithm (annex E):

a)	if the GN protocol constant itsGnNonAreaForwardingAlgorithm is set to 0 (UNSPECIFIED), execute the GF algorithm as specified in annex E.2;

b)	if the GN protocol constant itsGnNonAreaForwardingAlgorithm is set to 1 (GREEDY), execute the GF algorithm as specified in annex E.2;

c)	if the GN protocol constant itsGnNonAreaForwardingAlgorithm is set to 2 (CBF), set the ALI address to the Broadcast ALI address;

10)	if the optional Security profile parameter in the service primitive TRANSP_CORE.request is set:

a)	send a service primitive SN-ENCAP.request as specified in annex O and the parameter setting in table 26;

Table 26: Parameter settings in the service primitive SN-ENCAP.request 
for the GUC packet in LS

| Parameter name | Parameter setting (in case of security as specified in note 2) |
| --- | --- |
| tbe_packet_length | Length of the GUC header + BTP header + payload |
| tbe_packet | Common header + GUC header + BTP header + payload to be signed |
| sec_profile | The value of the parameter Security profile in the service primitive TRANSP_CORE.request (see note 1) |
| its_aid _length | 2 |
| its_aid | 141 = 0x8d |
| permissions_length | 0 |
| permissions | void |
| context_information | 0 |
| target_id_list_length | void |
| target_id_list | 0 |
| NOTE 1: If the parameter Security profile in the service primitive TRANSP_CORE.request is not set and the GN protocol constant itsGnSecurity is set to ENABLED, a default security profile is used. The specification of the default security profile is out of scope of the present document. NOTE 2: See ETSI TS 102 965 [i.7]. The permissions_length parameter being set to zero indicates that the SSP is null and absent. |  |

b)	process the service primitive SN-ENCAP.confirm and append the Secured Packet carried by the sec_packet parameter of the service primitive SN-ENCAP.confirm to the Basic Header;

11)	execute media-dependent procedures; if the Communication profile parameter of the service primitive
TRANSP_CORE.request is set to:

a)	UNSPECIFIED then no media-dependent procedures are specified;

b)	for other values media dependent procedures may be defined elsewhere;

12)	pass the GN-PDU to the ALI via the NET_ACC_DATA interface and set the destination address to the link layer address of the next hop LL_ADDR_NH.

### 10.3.8	GUC packet handling

#### 10.3.8.1	General

This clause specifies the operations of a GAGH to handle a GUC packet. The following clauses define the operations of the source, forwarder and destination.

GeoUnicast forwarding applies the algorithm selected by the setting of the value in the GN protocol constant itsGnNonAreaForwardingAlgorithm and specified in annex E.

#### 10.3.8.2	Source operations

On reception of a service primitive TRANSP_CORE.request with a Packet transport type parameter set to GeoUnicast, the source shall execute the following operations:

1)	create a GN-PDU with the T/GN6-SDU as payload and a GUC packet header (clause 9.8.2):

a)	set the fields of the Basic Header (clause 10.3.2);

b)	set the fields of the Common Header (clause 10.3.4);

c)	set the fields of the GUC Extended Header (table 27);

Table 27: Field settings for the GUC Extended Header

| Field name | Field setting | Description |
| --- | --- | --- |
| SN | Actual value of the local sequence number (clause 8.4) | Sequence number of the packet |
| SO PV | Actual values of the EPV (clause 8.3) | Position vector containing the reference position of the ego GeoAdhoc router (source of the GeoNetworking packet) |
| DE PV | Actual values of the LocTE for the destination | Position vector containing the reference position of the destination or empty in case the destination position is not yet available |

2)	check whether the entry of the position vector for DE in its LocT is valid:

a)	if no valid position vector information is available, the source shall invoke the location service as specified in clause 10.3.7.1.2 and omit the execution of further steps. Otherwise, the source shall proceed with step 2;

3)	if no neighbour exists, i.e., the LocT does not contain a LocTE with the IS_NEIGHBOUR flag set to TRUE, and SCF for the traffic class in the service primitive TRANSP_CORE.request parameter Traffic class is enabled, then buffer the GUC packet in the UC forwarding packet buffer and omit the execution of further steps;

4)	execute the forwarding algorithm (see annex E):

a)	if the GN protocol constant itsGnNonAreaForwardingAlgorithm is set to 0 (UNSPECIFIED), execute the GF algorithm as specified in annex E.2;

b)	if the GN protocol constant itsGnNonAreaForwardingAlgorithm is set to 1 (GREEDY), execute the GF algorithm as specified in annex E.2;

c)	if the GN protocol constant itsGnNonAreaForwardingAlgorithm is set to 2 (CBF), set the LL_ADDR_NH address to the Broadcast link layer address;

5)	if the return value of the forwarding algorithm is 0 (packet is buffered in the UC forwarding packet buffer) or 
-1 (packet is discarded), omit the execution of further steps;

6)	if the optional Security profile parameter in the service primitive TRANSP_CORE.request is set:

a)	send a service primitive SN-ENCAP.request as specified in annex O and the parameter setting in table 28;

Table 28: Parameter settings in the service primitive SN-ENCAP.request for the GUC packet

| Parameter name | Parameter setting |
| --- | --- |
| tbe_packet_length | Length of the GUC header + BTP header + payload |
| tbe_packet | Common header + GUC header + BTP header + payload to be signed |
| sec_profile | Value of the corresponding parameter in the service primitive TRANSP_CORE.request (annex J.2) (optional) |
| its_aid_length |  |
| its_aid |  |
| permissions_length |  |
| permissions |  |
| context_information |  |
| target_id_list_length |  |
| target_id_list |  |
| NOTE: If the security-related parameters in the service primitive TRANSP_CORE.request are not set and the GN protocol constant itsGnSecurity is set to ENABLED, a default security profile is used. The specification of the default security profile is out of scope of the present document. |  |

b)	process the service primitive SN-ENCAP.confirm and append the Secured Packet carried by the sec_packet parameter of the service primitive SN-ENCAP.confirm to the Basic Header;

7)	if the optional Repetition interval parameter in the TRANSP_CORE.request parameter is set:

a)	save the GUC packet;

b)	retransmit the packet with a period as specified in Repetition interval until the maximum repetition time of the packet is expired;

NOTE 1:	The maximum repetition time of the packet is specified in the Maximum repetition time parameter of the service primitive TRANSP_CORE.request.

NOTE 2:	For every retransmission, the source operations need to be re-executed.

NOTE 3:	The functionality of packet repetition is optional.

8)	execute media-dependent procedures; if the Communication profile parameter of the service primitive TRANSP_CORE.request is set to:

a)	UNSPECIFIED then no media-dependent procedures are specified;

b)	for other values media dependent procedures may be defined elsewhere;

9)	pass the GN-PDU to the ALI via the NET_ACC_DATA interface and set the destination address to the link layer address of the next hop LL_ADDR_NH.

#### 10.3.8.3	Forwarder operations

On reception of a GUC packet, a GAGH shall check the GN_ADDR field in the DE PV of the GUC packet header. If this address does not match its GN_ADDR, the GAGH shall execute the following operations:

1)	Basic Header processing (clause 10.3.3);

2)	Common Header processing (clause 10.3.5);

3)	if the GN protocol constant itsGnNonAreaForwardingAlgorithm is set to 0 (UNSPECIFIED) or to 1 (GREEDY), execute DPD as specified in annex A.2; if the GUC packet is a duplicate, discard the packet and omit the execution of further steps;

NOTE 1:	For CBF (itsGnNonAreaForwardingAlgorithm is set to 2), the algorithm relies on the processing of duplicate packets and their handling is part of the forwarding algorithm.

4)	execute DAD as specified in clause 10.2.1.5;

5)	if the LocTE(SO) does not exist:

a)	create PV(SO) in the LocT with the SO PV fields of the GUC Extended Header (annex C.2);

b)	set the IS_NEIGHBOUR flag of the SO LocTE to FALSE;

c)	update PDR(SO) in the LocT (annex B.2);

6)	if the LocTE(SO) exists:

a)	update PV(SO) in the LocT with the SO PV fields of the GUC Extended Header (annex C.2);

b)	update PDR(SO) in the LocT (annex B.2);

NOTE 2:	The IS_NEIGHBOUR flag of the SO LocTE remains unchanged.

7)	if the DE LocTE does not exist or the IS_NEIGHBOUR flag of the existing DE LocTE is not set:

a)	if the NH field of the Basic Header NH = 0 (ANY) or NH = 1 (Common Header), update the PV(DE) in the LocT with the DE PV value of the GUC packet (annex C.2);

NOTE 3:	The DE PV is a Short Position Vector (SPV) and does not carry all fields required to set the PV(DE) in the LocT, i.e., no PAI, speed, heading. Therefore, if the PV in the LocT is updated, the fields PAI, speed, heading are set to 0.

NOTE 4:	If the IS_NEIGHBOUR flag of the DE LocTE is set, the DE LocTE is not updated.

8)	if the DE LocTE exists and the IS_NEIGHBOUR flag of the existing DE LocTE is set:

a)	if the NH field of the Basic Header NH = 0 (ANY) or NH = 1 (Common Header), update the DE PV field in the GUC packet with the PV(DE) in the LocT (annex C.3);

NOTE 5:	If the DE PV overwrites an existing LocTE with an IS_NEIGHBOUR flag set, this entry would not be considered in the forwarding algorithm due to the PAI set to 0.

NOTE 6:	The DE PV fields are not updated if NH = 2 (Secured Packet).

9)	flush packet buffers (SO LS packet buffer, SO UC forwarding packet buffer):

a)	if LS_pending(SO) is TRUE:

i)	forward the stored packets and remove them from the SO LS packet buffer (clause 8.5);

ii)	set LS_pending(SO) to false;

b)	if the UC forwarding packet buffer (clause 8.6) for SO is not empty, flush the UC forwarding packet buffer and forward the stored packets;

10)	decrement the RHL value:

a)	if RHL = 0 discard the packet and omit the execution of further steps;

b)	if RHL > 0 update the field of the Basic Header, i.e., the RHL field with the decremented RHL value;

11)	if no neighbour exists, i.e., the LocT does not contain a LocTE with the IS_NEIGHBOUR flag set to TRUE, and SCF for the traffic class in the TC field of the Common Header is set, buffer the GUC packet in the UC forwarding packet buffer and omit the execution of further steps;

12)	execute the forwarding algorithm (annex E):

a)	if the GN protocol constant itsGnNonAreaForwardingAlgorithm is set to 0 (UNSPECIFIED), execute the GF algorithm as specified in annex E.2;

b)	if the GN protocol constant itsGnNonAreaForwardingAlgorithm is set to 1 (GREEDY), execute the GF algorithm as specified in annex E.2;

c)	if the GN protocol constant itsGnNonAreaForwardingAlgorithm is set to 2 (CBF), execute the CBF algorithm as specified in annex E.3;

13)	if the return value of the forwarding algorithm is 0 (packet is buffered in a forwarding packet buffer) or -1 (packet is discarded), omit the execution of further steps;

14)	execute media-dependent procedures; if the GN protocol constant itsGnIfType is set to:

a)	UNSPECIFIED then no media-dependent procedures are specified;

b)	for other values of itsGnIfType media dependent procedures may be defined elsewhere;

15)	pass the GN-PDU to the ALI via the NET_ACC_DATA interface and set the destination address to the link layer address of the next hop LL_ADDR_NH.

#### 10.3.8.4	Destination operations

On reception of a GUC packet, the GeoAdhoc router shall check the GN_ADDR field in the DE PV of the GUC packet header. If this address matches its GN_ADDR, the GeoAdhoc router shall execute the following operations:

1)	Basic Header processing (clause 10.3.3);

2)	Common Header processing (clause 10.3.5);

3)	execute DPD as specified in annex A.2; if the GUC packet is a duplicate, discard the packet and omit the execution of further steps;

4)	execute DAD as specified in clause 10.2.1.5;

5)	if the LocTE(SO) does not exist:

a)	create PV(SO) in the LocT with the SO PV fields of the GUC Extended Header (annex C.2);

b)	set the IS_NEIGHBOUR flag of the SO LocTE to FALSE;

c)	set the PDR(SO) in SO LocT (annex B.2);

6)	if the LocTE(SO) exists:

a)	update PV(SO) in the LocT with the SO PV fields of the GUC Extended Header (annex C.2);

b)	update the PDR(SO) in the LocT (annex B.2);

NOTE:	The IS_NEIGHBOUR flag of the SO LocTE remains unchanged.

7)	flush packet buffers (SO LS packet buffer, SO UC forwarding packet buffer):

a)	if LS_pending(SO) is TRUE:

i)	forward the stored packets and remove them from the SO LS packet buffer (clause 8.5);

ii)	set LS_pending(SO) to false;

b)	if the UC forwarding packet buffer (clause 8.6) for SO is not empty, flush the UC forwarding packet buffer and forward the stored packets;

8)	pass the payload of the GN-PDU to the upper protocol entity by means of a service primitive
TRANSP_CORE.indication with the parameter settings in table 29.

Table 29: Parameter settings in the service primitive TRANSP_CORE.indication 
to indicate a received GUC packet

| Parameter name | Parameter setting |
| --- | --- |
| Upper protocol entity | BTP if NH = 1 (BTP-A) BTP if NH = 2 (BTP-B) Ipv6 if NH = 3 (Ipv6) (NH encoding see table 8 in clause 9.7.3) |
| Packet transport type | GeoUnicast |
| Destination | DE GN_ADDR |
| Source position vector | Values of SO PV from Extended Header |
| Security report | Value of the corresponding security-related parameter in the service primitive SN-DECAP.indication (annex O) |
| Certificate id |  |
| ITS-AID length |  |
| ITS-AID |  |
| Security permissions length |  |
| Security permissions |  |
| Traffic class | Value of TC field from Common Header |
| Remaining packet lifetime | Value of LT from Basic Header |
| Remaining hop limit | Value of RHL from Basic Header |
| Length | Length of the GN-PDU payload |
| Data | GN-PDU payload |

### 10.3.9	TSB packet handling

#### 10.3.9.1	General

This clause specifies the operations of a GAGH to handle a TSB packet. The following clauses define the operations of the source and forwarder/receiver.

NOTE:	In TSB, a forwarder is also always a receiver. Therefore, the roles are not distinguished.

#### 10.3.9.2	Source operations

On reception of a service primitive TRANSP_CORE.request with a Packet transport type parameter set to TSB, the source shall execute the following operations:

1)	create a GN-PDU with the T/GN6-SDU as payload and a TSB packet header (clause 9.8.3):

a)	set the fields of the Basic Header (clause 10.3.2);

b)	set the fields of the Common Header (clause 10.3.4);

c)	set the fields of the TSB Extended Header (table 30);

Table 30: Field settings for the TSB Extended Header

| Field name | Field setting | Description |
| --- | --- | --- |
| SN | Actual value of the local sequence number (clause 8.4) | Sequence number of the packet |
| Reserved | Set to 0 if not used for media-dependent operations | Reserved for media-dependent operations |
| SO PV | Actual values of the EPV (clause 8.3) | Position vector containing the reference position of the ego GeoAdhoc router (source of the GeoNetworking packet) |

2)	if the optional Security profile parameter in the service primitive TRANSP_CORE.request is set:

a)	send a service primitive SN-ENCAP.request as specified in annex O and the parameter setting in table 31;

Table 31: Parameter settings in the service primitive SN-ENCAP.request for the TSB packet

| Parameter name | Parameter setting |
| --- | --- |
| tbe_packet_length | Length of the TSB header + BTP header + payload |
| tbe_packet | Common header + TSB header + BTP header + payload to be signed |
| sec_profile | The value of the corresponding parameter in the service primitive TRANSP_CORE.request (annex J.2) |
| its_aid_length |  |
| its_aid |  |
| permissions length |  |
| permissions |  |
| context_information |  |
| target_id_list_length |  |
| target_id_list |  |
| NOTE: If the security-related parameters in the service primitive TRANSP_CORE.request are not set and the GN protocol constant itsGnSecurity is set to ENABLED, a default security profile is used. The specification of the default security profile is out of scope of the present document. |  |

b)	process the service primitive SN-ENCAP.confirm and append the Secured Packet carried by the sec_packet parameter of the service primitive SN-ENCAP.confirm to the Basic Header;

3)	if no suitable neighbour exists, i.e., the LocT does not contain a LocTE with the IS_NEIGHBOUR flag set to TRUE, and SCF for the traffic class in the service primitive TRANSP_CORE.request parameter Traffic class is enabled, then buffer the TSB packet in the BC forwarding packet buffer and omit the execution of further steps;

NOTE 1:	If SCF for the traffic class is disabled, the TSB packet is never buffered but sent immediately with a broadcast link layer destination address.

NOTE 2:	Buffered packets may be further processed when the GAGH receives a packet (e.g. SHB, GBC, BEACON, etc.), see the corresponding clauses on forwarder and receiver operations.

4)	if the optional Repetition interval parameter in the TRANSP_CORE.request parameter is set:

a)	save the TSB packet;

b)	retransmit the packet with period as specified in Repetition interval until the maximum repetition time of the packet is expired;

NOTE 3:	The maximum repetition time of the packet is specified in the Maximum repetition time parameter of the service primitive TRANSP_CORE.request.

NOTE 4:	For every retransmission, the source operations need to be re-executed.

NOTE 5:	The functionality of packet repetition is optional.

5)	execute media-dependent procedures; if the Communication profile parameter of the service primitive TRANSP_CORE.request is set to:

a)	UNSPECIFIED then no media-dependent procedures are specified;

b)	for other values media dependent procedures may be defined elsewhere;

6)	pass the GN-PDU to the ALI via the NET_ACC_DATA interface and set the destination address to the Broadcast address of the ALI.

#### 10.3.9.3	Forwarder and receiver operations

On reception of a TSB packet, the GAGH shall execute the following operations:

1)	Basic Header processing (clause 10.3.3);

2)	Common Header processing (clause 10.3.5);

3)	execute DPD as specified in annex A.2; if the TSB packet is a duplicate, discard the packet and omit the execution of further steps;

4)	execute DAD as specified in clause 10.2.1.5;

5)	if the LocTE(SO) does not exist:

a)	create PV(SO) in the LocT with the SO PV fields of the TSB Extended Header (annex C.2);

b)	set the IS_NEIGHBOUR flag of the SO LocTE to FALSE;

c)	set PDR(SO) in the SO LocTE (annex B.2);

6)	if the LocTE(SO) exists:

a)	update PV(SO) in the LocT with the SO PV fields of the TSB Extended Header (annex C.2);

b)	update PDR(SO) in the LocT (annex B.2);

NOTE 1:	The IS_NEIGHBOUR flag of the SO LocTE remains unchanged.

7)	pass the payload of the GN-PDU to the upper protocol entity by means of a service primitive TRANSP_CORE.indication with the parameter settings in table 32;

Table 32: Parameter settings in the service primitive TRANSP_CORE.indication 
to indicate a received TSB packet

| Parameter name | Parameter setting |
| --- | --- |
| Upper protocol entity | BTP if NH = 1 (BTP-A) BTP if NH = 2 (BTP-B) Ipv6 if NH = 3 (Ipv6) (NH encoding see table 8 in clause 9.7.3) |
| Packet transport type | TSB |
| Source position vector | Values of SO PV from Extended Header |
| Security report | Set to the value of the corresponding security-related parameter in the service primitive SN-DECAP.indication (see annex O) (optional) |
| Certificate id |  |
| ITS-AID length |  |
| ITS-AID |  |
| Security permissions length |  |
| Security permissions |  |
| Traffic class | Value of TC field from Common Header |
| Remaining packet lifetime | Value of LT from Basic Header |
| Remaining hop limit | Value of RHL from Basic Header |
| Length | Length of the GN-PDU payload |
| Data | GN-PDU payload |

8)	flush packet buffers (SO LS packet buffer, SO UC forwarding packet buffer):

a)	if LS_pending(SO) is TRUE:

i)	forward the stored packets and remove them from the SO LS packet buffer (clause 8.5);

ii)	set LS_pending(SO) to false;

b)	if the UC forwarding packet buffer (clause 8.6) for SO is not empty, flush the UC forwarding packet buffer and forward the stored packets;

9)	decrement the RHL value:

a)	if RHL = 0 discard the packet and omit the execution of further steps;

b)	if RHL > 0 update the field of the Basic Header, i.e., the RHL field with the decremented RHL value;

10)	if no suitable neighbour exists, i.e., the LocT does not contain a LocTE with the IS_NEIGHBOUR flag set to TRUE, and SCF for the traffic class in the TC field of the Common Header is set:

a)	buffer the TSB packet in the BC forwarding packet buffer and omit the execution of further steps;

NOTE 2:	If SCF for the traffic class is disabled, the TSB packet is never buffered but sent immediately with a broadcast link layer destination address.

NOTE 3:	Buffered packets may be further processed when the GAGH receives a packet (e.g. SHB, GBC, BEACON, etc.), see the corresponding clauses on forwarder and receiver operations.

11)	execute media-dependent procedures; if the GN protocol constant itsGnIfType is set to:

a)	UNSPECIFIED then no media-dependent procedures are specified;

b)	for other values of itsGnIfType media dependent procedures may be defined elsewhere;

12)	pass the GN-PDU to the ALI via the NET_ACC_DATA interface and set the destination address to the Broadcast address of the ALI.

### 10.3.10	SHB packet handling

#### 10.3.10.1	General

This clause specifies the operations of a GAGH to handle a SHB packet. The following clauses define the operations of the source and receiver.

NOTE:	SHB packets are not forwarded. Therefore, no forwarder operations are specified.

#### 10.3.10.2	Source operations

On reception of a service primitive TRANSP_CORE.request with a Packet transport type parameter set to SHB, the source shall execute the following operations:

1)	create a GN-PDU with the T/GN6-SDU as payload and a SHB packet header (clause 9.8.4):

a)	set the fields of the Basic Header (clause 10.3.2);

b)	set the fields of the Common Header (clause 10.3.4);

c)	set the fields of the SHB Extended Header (table 33);

Table 33: Field settings for the SHB Extended Header

| Field name | Field setting | Description |
| --- | --- | --- |
| SO PV | Actual values of the EPV as specified in clause 8.3 | PV of the ego GeoAdhoc router (source of the GeoNetworking packet) |
| Reserved | Set to 0 if not used for media-dependent operations | Reserved for media-dependent operations |

2)	if the optional Security profile parameter in the service primitive TRANSP_CORE.request is set:

a)	send a service primitive SN-ENCAP.request as specified in annex O and the parameter setting in table 34;

Table 34: Parameter settings in the service primitive SN-ENCAP.request for the SHB packet

| Parameter name | Parameter setting |
| --- | --- |
| tbe_packet_length | Length of the SHB header + BTP header + payload |
| tbe_packet | Common header + SHB header + BTP header + payload to be signed |
| sec_profile | The value of the corresponding parameter in the service primitive TRANSP_CORE.request (annex J.2) |
| its_aid_length |  |
| its_aid |  |
| permissions length |  |
| permissions |  |
| context_information |  |
| target_id_list_length |  |
| target_id_list |  |
| NOTE: If the security-related parameters in the service primitive TRANSP_CORE.request are not set and the GN protocol constant itsGnSecurity is set to ENABLED, a default security profile is used. The specification of the default security profile is out of scope of the present document. |  |

b)	process the service primitive SN-ENCAP.confirm and append the Secured Packet carried by the sec_packet parameter of the service primitive SN-ENCAP.confirm to the Basic Header;

3)	if no suitable neighbour exists, i.e., the LocT does not contain a LocTE with the IS_NEIGHBOUR flag set to TRUE, and SCF for the traffic class in the service primitive TRANSP_CORE.request parameter Traffic class is set:

a)	buffer the SHB packet in the BC forwarding packet buffer and omit the execution of further steps;

NOTE 1:	If SCF for the traffic class is disabled, the SHB packet is never buffered but sent immediately with a broadcast link layer destination address.

NOTE 2:	Buffered packets may be further processed when the GAGH receives a packet (e.g. SHB, GBC, BEACON, etc.), see the corresponding clauses on forwarder and receiver operations.

4)	if the optional Repetition interval parameter in the TRANSP_CORE.request parameter is set:

a)	save the SHB packet;

b)	retransmit the packet with a period as specified in Repetition interval parameter until the maximum repetition time of the packet is expired;

NOTE 3:	The maximum repetition time of the packet is specified in the Maximum repetition time parameter of the service primitive TRANSP_CORE.request.

NOTE 4:	For every retransmission, the source operations need to be re-executed.

NOTE 5:	The functionality of packet repetition is optional.

5)	execute media-dependent procedures; if the Communication profile parameter of the service primitive TRANSP_CORE.request is set to:

a)	UNSPECIFIED then no media-dependent procedures are specified;

b)	for other values media dependent procedures may be defined elsewhere;

6)	pass the GN-PDU to the ALI via the NET_ACC_DATA interface and set the destination address to the Broadcast address of the ALI;

7)	reset the beacon timer Tbeacon to prevent the dissemination of an unnecessary beacon packet.

#### 10.3.10.3	Receiver operations

On reception of a SHB packet, the GAGH shall execute the following operations:

1)	Basic Header processing (clause 10.3.3);

2)	Common Header processing (clause 10.3.5);

3)	execute DAD as specified in clause 10.2.1.5;

4)	update the PV in the SO LocTE with the SO PV fields of the SHB Extended Header (annex C.2);

5)	update the PDR in the SO LocTE (annex B.2);

6)	set the IS_NEIGHBOUR flag of the SO LocTE to TRUE;

7)	pass the payload of the GN-PDU to the upper protocol entity by means of a service primitive TRANSP_CORE.indication with the parameter settings in table 35;

Table 35: Parameter settings in the service primitive TRANSP_CORE.indication 
to indicate a received SHB packet

| Parameter name | Parameter setting |
| --- | --- |
| Upper protocol entity | BTP if NH = 1 (BTP-A) BTP if NH = 2 (BTP-B) Ipv6 if NH = 3 (Ipv6) (NH encoding see table 8 in clause 9.7.3) |
| Packet transport type, | SHB |
| Source position vector | Values of SO PV from SHB Common Header |
| Security report | Value of the corresponding security-related parameter in the service primitive SN-DECAP.indication (annex O) (optional) |
| Certificate id |  |
| ITS-AID length |  |
| ITS-AID |  |
| Security permissions length |  |
| Security permissions |  |
| Traffic class | Value of TC field from Common Header |
| Remaining packet lifetime | Value of LT from Basic Header |
| Remaining hop limit | Value of RHL from Basic Header |
| Length | Length of the GN-PDU payload |
| Data | GN-PDU payload |

8)	flush packet buffers (SO LS packet buffer an SO UC forwarding packet buffer):

a)	if SO LS_pending is TRUE:

i)	forward the stored packets and remove them from the buffer;

ii)	set SO LS_pending to false;

b)	if the UC forwarding packet buffer (clause 8.6) for the SO is not empty, forward the stored packets and remove them from the UC forwarding packet buffer.

### 10.3.11	GBC packet handling

#### 10.3.11.1	General

This clause specifies the operations of a GAGH to handle a GBC packet. The following clauses define the operations of the source and forwarder/receiver.

NOTE:	In GBC, a forwarder inside the target area acts also always as a receiver. Therefore, the roles are not distinguished.

#### 10.3.11.2	Source operations

On reception of a service primitive TRANSP_CORE.request with a Packet transport type parameter set to GeoBroadcast, the source shall execute the following operations:

1)	create a GN-PDU with the T/GN6-SDU as payload and a GBC packet header (clause 9.8.5):

a)	set the fields of the Basic Header (clause 10.3.2);

b)	set the fields of the Common Header (clause 10.3.4);

c)	set the fields of the GBC Extended Header (table 36);

Table 36: Field settings for the GBC Extended Header

| Field name | Field setting | Description |
| --- | --- | --- |
| SN | Actual value of the local sequence number (clause 8.4) | Sequence number of the packet |
| Reserved | Set to 0 | Reserved |
| SO PV | Actual values of the EPV (clause 8.3) | Position vector containing the reference position of the ego GeoAdhoc router (source of the GeoNetworking packet) |
| GeoAreaPos Latitude | GeoAreaPos Latitude from service primitive TRANSP_CORE.request | GeoArea Area specification according to ETSI EN 302 931 [1] |
| GeoAreaPos Longitude | GeoAreaPos Longitude from service primitive TRANSP_CORE.request |  |
| Distance a | Distance a from service primitive TRANSP_CORE.request |  |
| Distance b | Distance a from service primitive TRANSP_CORE.request |  |
| Angle | Angle from service primitive TRANSP_CORE.request |  |
| Reserved | Set to 0 | Reserved |

2)	if no neighbour exists, i.e., the LocT does not contain a LocTE with the IS_NEIGHBOUR flag set to TRUE, and SCF for the traffic class in the service primitive TRANSP_CORE.request parameter Traffic class is enabled, then buffer the GBC packet in the BC forwarding packet buffer and omit the execution of further steps;

3)	execute the forwarding algorithm procedures (starting with annex D);

4)	if the return value of the forwarding algorithm is 0 (packet is buffered in the BC forwarding packet buffer or in the CBF buffer) or -1 (packet is discarded), omit the execution of further steps;

5)	if the optional Security profile parameter in the service primitive TRANSP_CORE.request is set:

a)	send a service primitive SN-ENCAP.request as specified in annex O and the parameter setting in table 37;

Table 37: Parameter settings in the service primitive SN-ENCAP.request for the GBC packet

| Parameter name | Parameter setting |
| --- | --- |
| tbe_packet_length | Length of the GBC header + BTP header + payload |
| tbe_packet | Common header + GBC header + BTP header + payload to be signed |
| sec_services | Value of the corresponding parameter in the service primitive TRANSP_CORE.request (annex J.2) |
| its_aid_length |  |
| its_aid |  |
| permissions length |  |
| permissions |  |
| context_information |  |
| target_id_list_length |  |
| target_id_list |  |
| NOTE: If the security-related parameters in the service primitive TRANSP_CORE.request are not set and the GN protocol constant itsGnSecurity is set to ENABLED, a default security profile is used. The specification of the default security profile is out of scope of the present document. |  |

b)	process the service primitive SN-ENCAP.confirm and append the Secured Packet carried by the sec_packet parameter of the service primitive SN-ENCAP.confirm to the Basic Header;

6)	if the optional Repetition interval parameter in the TRANSP_CORE.request parameter is set:

a)	save the GBC packet;

b)	retransmit the packet with period as specified in Repetition interval until the maximum repetition time of the packet is expired;

NOTE 1:	The maximum repetition time of the packet is specified in the Maximum repetition time parameter of the service primitive TRANSP_CORE.request.

NOTE 2:	For every retransmission, the source operations need to be re-executed.

NOTE 3:	The functionality of packet repetition is optional.

7)	execute media-dependent procedures; if the Communication profile parameter of the service primitive TRANSP_CORE.request is set to:

a)	UNSPECIFIED then no media-dependent procedures are specified;

b)	for other values media dependent procedures may be defined elsewhere;

8)	pass the GN-PDU to the ALI via the NET_ACC_DATA interface and set the destination address to the link layer address of the next hop LL_ADDR_NH.

#### 10.3.11.3	Forwarder and receiver operations

On reception of a GBC packet, the GAGH shall execute the following operations:

1)	Basic Header processing (clause 10.3.3);

2)	Common Header processing (clause 10.3.5);

3)	determine function F(x,y) as specified in ETSI EN 302 931 [1] clause 5:

a)	if  (GeoAdhoc router is outside the geographical area) and the GN protocol constant itsGnNonAreaForwardingAlgorithm is set to 0 (UNSPECIFIED) or to 1 (GREEDY), execute DPD as specified in annex A.2; if the GBC packet is a duplicate, discard the packet and omit the execution of further steps;

NOTE 1a:	For CBF forwarding algorithm (itsGnNonAreaForwardingAlgorithm is set to 2 or 3), the algorithm relies on the processing of duplicate packets and their handling is part of the forwarding algorithm.

- if  (GeoAdhoc router is inside or at the border of the geographical area) and the GN protocol constant itsGnAreaForwardingAlgorithm is set to 0 (UNSPECIFIED) or to 1 (SIMPLE), execute DPD as specified in annex A.2; if the GBC packet is a duplicate, discard the packet and omit the execution of further steps;

NOTE 1b:	For CBF and the Advanced forwarding algorithm (itsGnAreaForwardingAlgorithm is set to 2 or 3), the algorithm relies on the processing of duplicate packets and their handling is part of the forwarding algorithm.

4)	execute DAD as specified in clause 10.2.1.5;

5)	if the LocTE(SO) does not exist:

a)	create PV(SO) in the LocT with the SO PV fields of the GBC Extended Header (annex C.2);

b)	set the IS_NEIGHBOUR flag of the SO LocTE to FALSE;

c)	set PDR(SO) in the LocTE (annex B.2);

6)	if the LocTE(SO) exists:

a)	update PV(SO) in the LocT with the SO PV fields of the GBC Extended Header (annex C.2);

b)	update PDR(SO) in the LocT (annex B.2);

NOTE 2:	The IS_NEIGHBOUR flag of the SO LocTE remains unchanged.

7)	determine function F(x,y) as specified in ETSI EN 302 931 [1] clause 5:

a)	if  (GeoAdhoc router is inside or at the border of the geographical area), pass the payload of the GN-PDU to the upper protocol entity by means of a service primitive TRANSP_CORE.indication with the parameter settings in table 38;

NOTE 3:	If the GeoAdhoc router is outside the geographical area, the GN-PDU will not be passed to the upper protocol entity.

NOTE 4:	When an ITS-S invokes the function F(x,y), it is recommended to transform the GNSS coordinates into Cartesian coordinates using a suitable method to avoid large rounding errors on low-precision floating point systems. Such a suitable method is the haversine formula, for instance.

Table 38: Parameter settings in the service primitive TRANSP_CORE.indication
to indicate a received GBC packet

| Parameter name | Parameter setting |
| --- | --- |
| Upper protocol entity | BTP if NH = 1 (BTP-A) BTP if NH = 2 (BTP-B) Ipv6 if NH = 3 (Ipv6) (NH encoding see table 8 in clause 9.7.3) |
| Packet transport type, | GeoBroadcast |
| Destination | GeoArea (GeoPos, distance a, distance b, angle) |
| Source position vector | Values of SO PV from Extended Header |
| Security report | Value of the corresponding security-related parameter in the service primitive SN-DECAP.indication (annex O) (optional) |
| Certificate id |  |
| ITS-AID length |  |
| ITS-AID |  |
| Security permissions length |  |
| Security permissions |  |
| Traffic class | Value of TC field from Common Header |
| Remaining packet lifetime | Value of LT from Basic Header |
| Remaining hop limit | Value of RHL from Basic Header |
| Length | Length of the GN-PDU payload |
| Data | GN-PDU payload |

8)	flush packet buffers (SO LS packet buffer, SO UC forwarding packet buffer):

a)	if LS_pending(SO) is TRUE:

i)	forward the stored packets and remove them from the SO LS packet buffer (clause 8.5);

ii)	set LS_pending(SO) to false;

b)	if the UC forwarding packet buffer (clause 8.6) for SO is not empty, flush the UC forwarding packet buffer and forward the stored packets;

9)	decrement the RHL value:

a)	if RHL = 0 discard the packet and omit the execution of further steps;

b)	if RHL > 0 update the field of the Basic Header, i.e., the RHL field with the decremented RHL value;

10)	if no neighbour exists, i.e., the LocT does not contain a LocTE with the IS_NEIGHBOUR flag set to TRUE, and SCF for the traffic class in the TC field of the Common Header is set, buffer the GBC packet in the BC forwarding packet buffer and omit the execution of further steps;

11)	execute the forwarding algorithm procedures (starting with annex D);

12)	if the return value of the forwarding algorithm is 0 (packet is buffered in a forwarding packet buffer) or -1 (packet is discarded), omit the execution of further steps;

NOTE 5:	If SCF for the traffic class is disabled, the GBC packet is never buffered but sent immediately, except for the CBF packet buffer.

NOTE 6:	Buffered packets may be further processed when the GAGH receives a packet (e.g. SHB, GBC, BEACON, etc.), see the corresponding clauses on forwarder and receiver operations.

13)	execute media-dependent procedures; if the GN protocol constant itsGnIfType is set to:

a)	UNSPECIFIED then no media-dependent procedures are specified;

b)	for other values of itsGnIfType media dependent procedures may be defined elsewhere;

14)	pass the GN-PDU to the ALI via the NET_ACC_DATA interface and set the destination address to the link layer address of the next hop LL_ADDR_NH.

### 10.3.12	GAC packet handling

#### 10.3.12.1	General

This clause specifies the operations of a GAGH to handle a GAC packet. The following clauses define the operations of the source and forwarder/receiver.

The operations for GAC packet handling are similar to those for GBC packet.

#### 10.3.12.2	Source operations

The operations of the source of a GAC packet are identical with the source of a GBC packet as specified in clause 10.3.11.2.

NOTE:	The procedure ensures that the GAC packet is sent at least once when the GeoAdhoc router is inside or at the border of the geographical area.

#### 10.3.12.3	Forwarder and receiver operations

On reception of a GAC packet, the GAGH shall execute the following operations:

1)	Basic Header processing (clause 10.3.3);

2)	Common Header processing (clause 10.3.5);

3)	execute DPD as specified in annex A.2; if the GAC packet is a duplicate, discard the packet and omit the execution of further steps;

4)	execute DAD as specified in clause 10.2.1.5;

5)	if the LocTE(SO) does not exist:

a)	create PV(SO) in the LocT with the SO PV fields of the GAC Extended Header (annex C.2);

b)	set the IS_NEIGHBOUR flag of the SO LocTE to FALSE;

c)	set PDR(SO) in the LocT (annex B.2);

6)	if the LocTE(SO) exists:

a)	update PV(SO) in the LocT with the SO PV fields of the GAC Extended Header (annex C.2);

b)	update PDR(SO) in the LocT (annex B.2);

NOTE 1:	The IS_NEIGHBOUR flag of the SO LocTE remains unchanged.

7)	determine function F(x,y) as specified in ETSI EN 302 931 [1] clause 5;

8)	flush packet buffers (SO LS packet buffer, SO UC forwarding packet buffer):

a)	if LS_pending(SO) is TRUE:

i)	forward the stored packets and remove them from the SO LS packet buffer (clause 8.5);

ii)	set LS_pending(SO) to false;

b)	if the UC forwarding packet buffer (clause 8.6) for SO is not empty, flush the UC forwarding packet buffer and forward the stored packets;

9)	if  (GeoAdhoc router is inside or at the border of the geographical area):

a)	pass the payload of the GN-PDU to the upper protocol entity by means of a service primitive TRANSP_CORE.indication with the parameter settings in table 39;

Table 39: Parameter settings in the service primitive TRANSP_CORE.indication
to indicate a received GAC packet

| Parameter name | Parameter setting |
| --- | --- |
| Upper protocol entity | BTP if NH = 1 (BTP-A) BTP if NH = 2 (BTP-B) Ipv6 if NH = 3 (Ipv6) (NH encoding see table 8 in clause 9.7.3) |
| Packet transport type, | GeoAnycast |
| Destination | GeoArea (GeoPos, distance a, distance b, angle) |
| Source position vector | Values of SO PV from Extended Header |
| Security report | Value of the corresponding security-related parameter in the service primitive SN-DECAP.indication (annex O) (optional) |
| Certificate id |  |
| ITS-AID length |  |
| ITS-AID |  |
| Security permissions length |  |
| Security permissions |  |
| Traffic class | Value of TC field from Common Header |
| Remaining packet lifetime | Value of LT from Basic Header |
| Remaining hop limit | Value of RHL from Basic Header |
| Length | Length of the GN-PDU payload |
| Data | GN-PDU payload |

b)	omit the execution of further steps;

10)	if  (GeoAdhoc router is outside the geographical area):

a)	decrement the RHL value:

i)	if RHL = 0, discard the packet and omit the execution of further steps;

ii)	if RHL > 0, update the field of the Basic Header, i.e., the RHL field with the decremented RHL value;

- if no neighbour exists, i.e., the LocT does not contain a LocTE with the IS_NEIGHBOUR flag set to TRUE, and SCF for the traffic class in the TC field of the Common Header is set, buffer the GAC packet in the BC forwarding packet buffer and omit the execution of further steps;

NOTE 2:	If the GeoAdhoc router is outside the geographical area, the GN-PDU will not be passed to the upper layer entity.

11)	if the return value of the forwarding algorithm is 0 (packet is buffered in a forwarding packet buffer) or -1 (packet is discarded), omit the execution of further steps;

12)	execute media-dependent procedures; if the GN protocol constant itsGnIfType is set to:

a)	UNSPECIFIED then no media-dependent procedures are specified;

b)	for other values of itsGnIfType media dependent procedures may be defined elsewhere;

13)	pass the GN-PDU to the ALI via the NET_ACC_DATA interface and set the destination address to the link layer address of the next hop LL_ADDR_NH.

## 10.4	MCO management

### 10.4.1	Overview

The Networking & Transport layer MCO functionalities are all part of the MCO_NET service. MCO_NET has interfaces with the GN Core, GN ALI group handlers, and the Facilities and Access layer functionalities.

The entities realizing MCO_NET required functionalities are (see figure 23):

- MCO Parameters Updates (MPU) (clause 10.4.2);

- Network Updates (NU) (clause 10.4.3);

- ALI Request Handler (ARH) (clause 10.4.4).

## Figure 23: Overview of MCO management functions

### 10.4.2	MCO Parameters Updates (MPU)

#### 10.4.2.1	Overview

#### Each ALI group at the Access layer is associated with a GAGH at the Networking & Transport layer. The state of a GAGH depends on the state of its associated ALI group (see ETSI TS 103 695 [i.8]). The Networking & Transport layer shall support MCO parameters update requests, e.g., to update an ALI group status, from the Facilities layer and shall provide to the Facilities layer MCO parameters updates indicated by the Access layer. The MPU entity handles MCO parameter requests from the Facilities layer and forwards them to the Access layer (figure 24). Additionally, the MPU entity handles MCO parameter updates from the Access layer. It forwards the MCO parameters updates to the Facilities layer and changes the states of the GAGHs if necessary.

### Figure 24: MCO parameters updates and flow control

#### 10.4.2.2	GAGH state

A GAGH can be in one of the following states:

- Active: an active GAGH performs GN protocol operations and maintains GN data structures and buffers. A GAGH should be active as long its associated ALI group is in an active state;

- Inactive: an inactive GAGH does not perform GN protocol operations but maintains its data structures and buffers. A GAGH can only be in an inactive state when its associated ALI group is in an inactive state;

- Disabled: a disabled GAGH does not perform GN protocol operations and has its data structures and buffers cleared. A GAGH can only be disabled when its associated ALI group is in an inactive or not available state;

### NOTE:	 See ETSI TS 103 695 [i.8] for more information about the ALI group states.

#### 10.4.2.3	GAGH state update

The Networking & Transport layer shall provide the possibility to the Facilities layer to request MCO parameters updates at the lower layers and to receive MCO parameter updates from the lower layers.

When receiving a request to configure MCO parameters from the Facilities layer, MPU shall forward the request to the Access layer. This can be realized via the MCO_NET_ACC_MMT interface (see ETSI TS 103 695 [i.8] for the specification of the interface).

When receiving an MCO parameters update from the Access layer, the Networking & Transport layer shall forward the MCO parameters updates to the Facilities layer. If the state of an ALI group was modified, MPU indicates the corresponding change on the GN_CORE entity via the CORE_MCO interface (annex L) and update the state of the associated GAGH via the GAGH_MCO interface (annex M).

### 10.4.3	Network Updates (NU)

#### 10.4.3.1	Overview

To efficiently allocate resources at the Facilities layer, the Networking & Transport layer shall provide network updates to the Facilities layer when necessary. For example, the network updates may contain the CLRs perceived locally and by neighboring stations. The information contained in the network update shall be media-independent.

The Access layer provides ALI group network updates. Information related to the GAGHs, such as the CLR, are passed to them by the NU entity. The NU entity may collect GAGH network updates containing perceived network conditions of neighboured stations. It then provides network updates to the Facilities layer (figure 25).

Figure 25: Network updates control flow

### 10.4.3.1	Access layer update

When receiving an Access layer update from the NET_ACC_MMT interface, the NU entity performs the following operations:

- In case a CLR is provided, the CLR is indicated, if necessary, to the associated GAGH via the GAGH_MCO interface (annex M).

- In case an Access Layer notification is indicated, the NU entity shall forward the notification to the Facilities layer via the MCO_FAC_NET_MMT interface (see annex N).

#### 10.4.3.2	Network updates to the Facilities layer

To provide better knowledge of the current communication capabilities, the NU entity collects network-related information from the GAGHs and the Access layer.

The network update provided by the NU entity to the Facilities layer via the MCO_FAC_NET_MMT interface shall be media-independent.

NOTE:	To provide media-independent network updates to the Facilities layer, scaling functions may be used to align for example the CLR perceived by LTE-V2X to ITS-G5.

### 10.4.4	ALI Request Handler (ARH)

When a packet disseminiation is request by the transport layer protocol entities, the GN Core shall ensure the AliID indicated for the packet is associated to an active GAGH.

In case of BTP as the upper protocol entity and if the AliID does not correspond to an active GAGH, a TRANS_CORE.confirm is generated with the result code non-valid AliID and returned to the BTP entity (see annex J). When the GN6ASL entity is the upper protocol entity, an AliID is requested to the Facilities layer. This is realized by the ARH entity with the CORE_MCO interface (see annex L and figure 26). The GN Core requests the ARH entity for an AliID, then the ARH entity informs the Facilities layer via the MCO_FAC_NET_MMT interface of the request. The Facilities layer indicates the selected AliID to the ARH entity via the MCO_FAC_NET_MMT interface. The ARH entity then replies to the GN Core with the AliID.

NOTE:	For clarification, an AliID contains the information of which ALI group it belongs to. This can be used by the GN CORE entity to ensure that an active GAGH is associated to the ALI group referred in the AliID.

### Figure 26: Handling of packet with no assigned ALI

Annex A (normative):
Duplicate packet detection

# A.1	General

A GAGH may receive multiple copies of the same packet. Reasons for packet duplications may be the forwarding of the packet from multiple GeoAdhoc routers, routing loops, misconfiguration or replay of packets from misbehaving GeoAdhoc routers. In order to control (e.g. prevent) the forwarding of duplicate packets, the GeoNetworking protocol uses mechanisms for duplicate packet detection (DPD). The present document specifies a method for DPD that is based on sequence number (annex A.2).

The GeoNetworking protocol applies the sequence number-based method for DPD (annex A.2) to multi-hop packets (GUC, TSB, GBC, GAC, LS Request, LS Reply). This method is not applied to GeoNetworking single-hop packets (BEACON and SHB) since these packet types do not carry a SN field.

# A.2	SN-based duplicate packet detection

For the SN-based method for DPD, a GAGH maintains a duplicate packet list DPL(GN_ADDR) for every entry in its LocT (see clause 8.2.2). When a GAGH processes a packet, the DPL is used to detect whether this packet is a duplicate. The DPL is a list with sequence numbers of a length itsGnDPLLength, which shall be much smaller than the maximum size of sequence numbers, i.e., 2^16-1. Optionally, the DPL may also contain a counter that indicates how often the packet with a particular sequence number has already been received from the source (duplicate packet counter, DPC).

When the GAGH processes a packet from source SO, it compares the value of the SN field carried in the GeoNetworking packet SN(P) and searches for the corresponding entry in the DPL. If SN(P) is already included in the DPL, the packet is marked as a duplicate. If the optional DPC is maintained, it is incremented. Otherwise, the new sequence number is added at the head of the DPL, overwriting the oldest entry.

NOTE:	In one possible implementation, the DPL is a modified ring buffer (also called circular or cyclic buffer), i.e., a queue where the data to be added may wrap around to the beginning. The start of the ring buffer (head) contains the freshest newly received sequence number. The end of the queue (tail) contains the oldest received sequence number. Compared to a “classical” ring buffer, the DPL as a modified ring buffer is searched whether the received SN is already included.

The following algorithm shall be used:

| 1 | -- DPL is the Duplicate Packet List for source SO with length DPL_Length |
| --- | --- |
| 1 | -- P is the received GeoNetworking packet |
| 2 | -- SN(P) is the sequence number in the received GeoNetworking packet |
| 3 | -- DPC is the duplicate packet counter for packet SN(P) from source SO, i.e., DPL(SN(P).DPC |
| 4 | If (SN(P) DPL) # Received SN is already included in DPL for source SO |
| 5 | Indicate packet as duplicate |
| 6 | OPTIONAL: DPL(SN(P)).DPC++ # Increment packet counter for received SN |
|  | ELSE # Received SN is outside window |
| 7 | Add SN(P) |
| 8 | ENDIF |

Annex B (normative):
Packet data rate and geographical area size control

# B.1	Overview

Packet rate and geographical area size control is executed in the GeoNetworking forwarding process (clause 10.3) to control that a GAGH does not exceed a predefined packet rate or geographical area size.

# B.2	Packet data rate control

A GAGH shall maintain the Exponential Moving Average (EMA) of the packet data rate PDR for every LocTE as calculated in equation ((B.1).

Equation (B.1) Calculation of Exponential Moving Average of the packet data rate PDR:

(B.1)

where:

xt	is the measured instantaneous value of the packet data rate upon reception of the GeoNetworking packet.

PDR	is the average value of the packet data rate at time t; PDRt-1 is the previous value at time (t-1) maintained in the LocTE.

Β	Weight factor (0 < ß < 1), set to the value of the GN protocol constant itsGnMaxPacketDataRateEmaBeta / 100.

If the packet data rate PDR of a GAGH exceeds the value of the GN protocol constant itsGnMaxPacketDataRate, packets from this GeoAdhoc router (source or sender) shall not be forwarded.

# B.3	Geographical area size control

If the geographical area size carried in a GBC or GAC packet exceeds the maximum value specified in the GN protocol constant itsGnMaxGeoAreaSize, the GeoNetworking packet shall not be sent by the source and shall not be forwarded by the forwarder.

Annex C (normative):
Position vector update

# C.1	Overview

The position vector update is executed in the GeoNetworking forwarding process (clause 10.3) when a PV in a LocTE is updated by PV carried in a GeoNetworking packet header. The algorithm ensures that always the newer PV is used indicated by the timestamp that is contained in the PV.

The algorithm is utilized in two cases:

1)	When a GeoNetworking packet is received, the forwarding procedure updates the PV in the LocT by the PV carried in the GeoNetworking packet.

2)	When a GeoNetworking packet is forwarded, the forwarding procedure updates the PV in the packet to be forwarded by the PV in the LocT.

The algorithm makes use of the timestamp that is associated with the position information and is part of the position vector fields. It handles the wraparound of the timestamp values that occur due to the limited number of bits that represent a timestamp.

NOTE:	With a 32 bit timestamp in [ms], a wraparound occurs after 50 days:
(((((232) – 1) / 1 000) / 60) / 60) / 24 = 49,7102696.

# C.2	Update of LocT position vector

The following algorithm shall be applied to update a PV in the LocT. The algorithm shall also reset the lifetime of the location table entry T(LocTE) (clause 8.2.3).

| 1 | -- RP is the received GeoNetworking packet |
| --- | --- |
| 2 | -- PVRP is the position vector in the received GeoNetworking packet |
| 3 | -- PVLocT is the position vector in the LocT to be updated |
| 4 | -- TSTPV,RP is the timestamp for the position vector in the received |
| 5 | -- GeoNetworking packet |
| 6 | -- TSTPV,LocT is the timestamp for the position vector in the location table |
| 7 | -- to be updated |
| 8 | -- TSMax is the maximum value of the timestamp = 2^32-1 |
| 9 | -- T(LocTE)is the lifetime of the location table entry |
| 10 | -- itsGnLifetimeLocTE is the value of the GN protocol constant itsGnLifetimeLocTE |
| 11 | IF (((TSTPV,RP > TSTPV,LocT) AND ((TSTPV,RP – TSTPV,LocT) <= TSTMax/2)) OR |
| 12 | ((TSTPV,LocT > TSTPV,RP) AND ((TSTPV,LocT – TSTPV,RP) > TSTMax/2))) THEN |
| 13 | TSTPV,RP is greater than TSTPV,LocT |
| 14 | PVLocT ← PVRP |
| 15 | T(LocTE) ← value(itsGnLifetimeLocTE) |
| 16 | ELSE |
| 17 | TSTPV,RP is not greater than TSTPV,LocT |
| 18 | ENDIF |

# C.3	Update of GeoNetworking packet position vector

The following algorithm shall be applied to update a PV in a packet to be forwarded:

| 1 | -- FP is the GeoNetworking packet to be forwarded |
| --- | --- |
| 2 | -- PVFP is the position vector in the GeoNetworking packet to be forwarded |
| 3 | -- PVLocT is the position vector in the LocT |
| 4 | -- TSTPV,FP is the timestamp for the position vector in the GeoNetworking |
| 5 | -- packet to be forwarded |
| 6 | -- TSTPV,LocT is the timestamp for the position vector in the location table |
| 7 | -- TSMax is the maximum value of the timestamp = 2^32-1 |
| 8 | IF (((TSTPV,LocT > TSTPV,FP) AND ((TSTPV,LocT – TSTPV,FP) <= TSTMax/2)) OR |
| 9 | ((TSTPV,FP > TSTPV,LocT) AND ((TSTPV,FP – TSTPV,LocT) > TSTMax/2))) THEN |
| 10 | # TSTPV,LocT is greater than TSTPV,FP |
| 11 | PVFP ← PVLocT |
| 12 | ELSE |
| 13 | # TSTPV,FP is not greater than TSTPV,LocT |
| 14 | ENDIF |

NOTE:	The algorithm is used in to update the DE PV fields with the PV(DE) in the LocT (clause 10.3.8.3 Forwarder operations for GUC packet handling). It determines whether the timestamp of the PV in the LocT is fresher than the one in the GeoNetworking packet taking into account wrapping of the timestamp.

Annex D (normative):
GeoNetworking forwarding algorithm selection procedure

The procedure utilizes the function F(x,y) specified in ETSI EN 302 931 [1] clause 5 in order to determine whether the GeoAdhoc router is located inside, at the border or outside of the geographical target area carried in the GeoBroadcast or GeoAnycast packet header.

If the GeoAdhoc router is inside or at the border of the area, the packet shall be processed according to the selected area forwarding algorithm (itsGnAreaForwardingAlgorithm).

If the GeoAdhoc router is outside of the area and the packet is received from a sender located outside of the area, the packet shall be forwarded according to the selected non-area forwarding algorithm (itsGnNonAreaForwardingAlgorithm).

If the GeoAdhoc router is outside of the area and the packet is received from a sender located inside or at the border of the area, the packet shall be discarded.

NOTE 1:	Packet duplicate detection is not part of the forwarding algorithm but of the packet handling operations.

NOTE 2:	As defined in ETSI EN 302 931 [1] clause 5:

(D.1)

The algorithm returns one of the following three values:

AREA-FORWARDING indicates that forwarding continues with one of the algorithms defined in annex F selected by itsGnAreaForwardingAlgorithm;

NON-AREA-FORWARDING indicates that forwarding continues with one of the algorithms defined in annex E selected by itsGnNonAreaForwardingAlgorithm;

-1 indicates that the packet is discarded.

The pseudo-code of the algorithm is below:

| 1 | -- P is the GeoNetworking packet to be forwarded |
| --- | --- |
| 2 | -- LAT_EGO and LONG_EGO are latitude and longitude of the EPV, respectively |
| 3 | -- PV_SE is the sender position vector in the LocTE for the sender of P |
| 4 | -- with LAT_SE and LONG_SE as latitude and longitude |
| 5 | -- and position accuracy indicator PAI_SE |
| 6 | -- SE_POS_VALID is true if sender position is valid (known and accurate) |
| 7 |  |
| 8 | SET F_EGO ← F(LAT_EGO, LONG_EGO) # Eq. D.1 in NOTE 2 |
| 9 |  |
| 10 | IF (F_EGO ≥ 0) THEN |
| 11 | # Local GeoAdhoc router is inside or at the border of target area |
| 12 | RETURN AREA-FORWARDING # Continue with area forwarding algorithm in annex F |
| 13 | ELSE |
| 14 | # Local GeoAdhoc router is outside of target area |
| 15 | SET F_SE ← F(LAT_SE, LONG_SE) # Eq. D.1 in NOTE 2 |
| 16 | SET SE_POS_VALID ← PV_SE EXISTS AND PAI_SE = TRUE |
| 17 | IF (SE_POS_VALID AND F_SE ≥ 0)THEN |
| 18 | # Prevents transition from area forwarding to non-area forwarding |
| 19 | RETURN -1 # P is to be discarded |
| 20 | ELSE |
| 21 | RETURN NON-AREA-FORWARDING # Continue with non-area forwarding algorithm in annex E |
| 22 | ENDIF |
| 23 | ENDIF |

The activity diagram of the forwarding algorithm selection procedure is depicted in figure D.1 for illustration.

Figure D.1: GeoNetworking forwarding algorithm selection procedure

Annex E (normative):
Non-area forwarding algorithms

# E.1	Overview

The non-area forwarding algorithms are used to route a packet towards a destination. It is executed by a GeoAdhoc router to relay a packet to the next hop.

The present document defines two non-area forwarding algorithms:

1)	Greedy Forwarding (GF) algorithm (annex E.2);

2)	Contention-based forwarding (CBF) algorithm (annex E.3).

# E.2	Greedy forwarding algorithm

With the Greedy Forwarding (GF) algorithm, the GeoAdhoc router uses the location information of the destination carried in the GN packet header and selects one of the neighbours as the next hop.

The algorithm applies the most forward within radius (MFR) policy, which selects the neighbour with the smallest geographical distance to the destination, thus providing the greatest progress when the GN packet is forwarded.

The algorithm returns one of the following two values:

the link layer address of the next hop LL_ADDR_NH,

0 indicates that no forwarder could be found and the packet is buffered in the appropriate forwarding packet buffer.

The pseudo-code of the algorithm is below:

| 1 | -- P is the GN packet to be forwarded |
| --- | --- |
| 2 | -- I is the i-th LocTE |
| 3 | -- NH is the LocTE identified as next hop, NH.ALI_ADDR its Access layer instance address |
| 4 | -- NH_ALI_ADDR is the Accesss layer instance address of the next hop |
| 5 | -- EPV is the ego position vector |
| 6 | -- PV_P is the destination position vector in the GeoNetworking packet to be forwarded |
| 7 | -- PV_I is the position vector of the i-th LocTE |
| 8 | -- MFR indicates the progress according to the MFR policy |
| 9 | -- B is the forwarding packet buffer |
| 10 | (UC forwarding buffer or BC forwarding buffer, depending on type of P) |
| 11 | -- TC is the traffic class of the GN-Data.request (source operations) |
| 12 | or the field in the received Common header (forwarder operations) |
| 13 | MFR = DIST(PV_P, EPV) Initialize MFR |
| 14 | FOR (iLocT) |
| 15 | IF (i.IS_NEIGHBOUR) THEN # LocTE i is neighbour |
| 16 | IF (DIST(PV_P, PV_I) < MFR) THEN |
| 17 | NH ← i |
| 18 | MFR ← DIST(PV_P, PV_I) |
| 19 | ENDIF |
| 20 | ENDIF |
| 21 | ENDFOR |
| 22 | IF (MFR < DIST(PV_P, EPV)) THEN |
| 23 | SET NH_ALI_ADDR ← NH.ALI_ADDR |
| 24 | ELSE # Forwarder is at a local optimum |
| 25 | IF (TC.SCF_IS_ENABLED) THEN |
| 26 | ADD P TO B |
| 27 | SET NH_ALI_ADDR ← 0 # Indicates that packet is buffered |
| 28 | ELSE |
| 29 | SET NN_ALI_ADDR ← BCAST # No buffering allowed, fall back to BCAST |
| 30 | ENDIF |
| 31 | ENDIF |
| 32 | RETURN NH_ALI_ADDR |

NOTE:	If no neighbour with greater progress than the local GeoAdhoc router exists, i.e., no suitable neighbour exists, the packet has reached a local optimum and the result is returned indicating that no forwarder could be found.

# E.3	Non-area contention-based forwarding algorithm

With the Contention-Based Forwarding (CBF) algorithm, a receiver decides to be a forwarder of a GN packet. This is contrary to the sender-based forwarding scheme specified in annex E.2, where the sender determines the next hop. The CBF algorithm utilizes timer-based re-broadcasting with overhearing of duplicates in order to enable an implicit forwarding of a packet by the optimal node.

With CBF, the GeoAdhoc router broadcasts the GN packet. All neighbours, which receive the packet, process it: those routers with a positive progress buffer the packet in the CBF packet buffer and start a timer with a timeout that is inversely proportional to the forwarding progress of the GeoAdhoc router (equation (E.1)).

Equation (E.1) Calculation of timeout TO_CBF for buffering packets in the CBF packet buffer:

(E.1)

where:

TO_CBF_MIN	is the minimum duration the packet shall be buffered in the CBF packet buffer.

TO_CBF_MAX	is the maximum duration the packet shall be buffered in the CBF packet buffer.

PROG	is the forwarding progress of the local GeoAdhoc router towards the destination, i.e., the difference between the sende’'s distance and GeoAdhoc route’'s local distance from the destination. The sender position is taken from its LocTE.

DIST_MAX	is the theoretical maximum communication range of the wireless access technology.

NOTE 1:	For PROG = DIST_MAX, TO_CBF becomes TO_CBF_MIN. For the (theoretical) PROG = 0, TO_CBF becomes TO_CBF_MAX.

TO_CBF_MIN and TO_CBF_MAX shall be set to the GN protocol constants itsGnCbfMinTime and itsGnCbfMaxTime, respectively. If DIST_MAX is not defined in the specification of GeoNetworking media-dependent functionality for the specific ITS access technology, it shall be set to the GN protocol constant itsGnDefaultMaxCommunicationRange.

Upon expiration of the timer, the GeoAdhoc router re-broadcasts the GN packet. Before the timer expires, the GeoAdhoc router may receive a duplicate of the packet from a GeoAdhoc router with a shorter timeout, i.e., with a smaller distance to the destination. In this case, the GeoAdhoc router inspects its CBF packet buffer, stops the timer and removes the GN packet from the CBF packet buffer.

NOTE 2:	Compared to the GF algorithm (annex E.2), CBF has an implicit reliability mechanism at the cost of larger forwarding delay and additional processing. The reliability mechanism ensures that a packets is re-forwarded by an alternative forwarder if the theoretically optimal forwarder does not receive the packet, e.g. due to wireless link errors.

The algorithm returns one of the following three values:

the Broadcast ALI address BCAST;

0 indicates that the packet is buffered in the CBF packet buffer and will further processed when the timer expires or a packet duplicate is handled;

-1 indicates that the packet is discarded.

The activity diagram of the CBF algorithm is depicted in figure E.1 for illustration.

Figure E.1: Non-area contention-based forwarding algorithm

The pseudo-code of the algorithm is below:

| 1 | -- P is the GN packet to be forwarded |
| --- | --- |
| 2 | -- EPV is the ego position vector with position accuracy indicator PAI_EGO |
| 3 | -- PV_P is DE PV of GUC header or destination area’s centre position for GBC/GAC header |
| 4 | -- PV_SE is the sender position vector in the LocT with position accuracy indicator PAI_SE |
| 5 | -- B is the CBF packet buffer |
| 6 | -- TO is the timeout that triggers the re-broadcast of the packet |
| 7 | -- NH_ALI_ADDR is the ALIaddress of the next hop |
| 8 | -- BCAST is the Broadcast LL address |
| 9 |  |
| 10 | IF (P is newly generated by ego node) THEN |
| 11 | SET NH_LL_ADDR ← BCAST |
| 12 | RETURN NH_LL_ADDR # Immediate transmission at source router |
| 13 |  |
| 14 | ELSE IF (P IN B) THEN # Contending |
| 15 | REMOVE P FROM B |
| 16 | STOP TIMER |
| 17 | DISCARD P |
| 18 | RETURN -1 # Indicates that packet is discarded |
| 19 | ELSE # New packet |
| 20 | SET SE_POS_VALID ← PV_SE EXISTS AND PAI_SE = TRUE |
| 21 | # Indicates that sender position is valid |
| 22 | IF (SE_POS_VALID AND PAI_EGO = TRUE) THEN |
| 23 | # Sender and ego position are reliable, can calculate PROG |
| 24 | SET PROG ← (DIST(PV_P, PV_SE)–- DIST(PV_P, EPV)) |
| 25 | IF (PROG > 0) THEN # Forwarding progress |
| 26 | ADD P TO B |
| 27 | SET TO ← TO_CBF(PROG) # Eq. E.1 in the present clause |
| 28 | START TIMER(TO) |
| 29 | RETURN 0 # Indicates that packet is buffered |
| 30 | ELSE |
| 31 | DISCARD P |
| 32 | RETURN -1 # Indicates that packet is discarded |
| 33 | ENDIF |
| 34 | ELSE |
| 35 | # Use maximum timer if PROG cannot be calculated reliably |
| 36 | ADD P TO B |
| 37 | SET TO ← TO_CBF_MAX |
| 38 | START TIMER(TO) |
| 39 | RETURN 0 # Indicates that packet is buffered |
| 40 | ENDIF |
| 41 | ENDIF |
| 42 |  |
| 43 | IF (TIMER(TO) EXPIRES) THEN |
| 44 | FETCH P FROM B # Removes P from B |
| 45 | SET NH_LL_ADDR ← BCAST |
| 46 | RETURN NH_LL_ADDR # Indicates that packet can be forwarded |
| 47 | ENDIF |

Annex F (normative):
Area forwarding algorithms

# F.1	Overview

The GeoBroadcast forwarding algorithm is used to distribute a data packet within a geographical target area. The present document defines three forwarding algorithms:

1)	Simple area forwarding algorithm (annex F.2).

2)	Contention-based area forwarding algorithm (annex F.3).

3)	Advanced GeoBroadcast forwarding algorithm (annex F.4).

The area forwarding algorithms assume that the sender of the data packet is located inside or at the border of the target area. If this is not the case, the packet can be transported from the sender towards the target area, i.e., using non-area forwarding algorithms as specified in annex E, which is also referred to as line forwarding.

# F.2	Simple GeoBroadcast forwarding algorithm

The algorithm applies when the GeoAdhoc router is located inside or at the border of the geographical target area carried in the GeoBroadcast packet header. In this case, the packet shall be re-broadcasted.

NOTE 1:	Packet duplicate detection is not part of the forwarding algorithm but of the packet handling operations.

NOTE 2:	The algorithm described in annex D determines whether the present algorithm applies.

The algorithm returns the Broadcast LL address BCAST.

# F.3	Area contention-based forwarding algorithm

Similar to the non-area contention-based forwarding algorithm, with the area contention-based forwarding (CBF) algorithm, a receiver decides to be a forwarder of a GN packet.

The algorithm applies when the GeoAdhoc router is located inside or at the border of the geographical target area carried in the GeoBroadcast or GeoAnycast packet header. When a node broadcasts a GBC/GAC packet with the CBF algorithm, all receiving neighbours process and buffer this packet in their respective CBF packet buffer. The associated CBF timer starts with a timeout that is proportional to the distance between the GeoAdhoc route’'s local position and the position of the sender. Thus, the node with the maximum forwarding progress will have the smallest timeout (equation (F.1)). When the timer expires the node will re-broadcast the GBC packet and implicitly inform the GeoAdhoc routers in its communication range to not forward the packet. Upon reception of the duplicate these GeoAdhoc routers stop the timer and remove the packet from the CBF packet buffer.

NOTE 1:	The definition of the distance is different from the non-area CBF algorithm (annex E.3), where the distance is the forwarding progress between the GeoAdhoc route’'s local position and the destination position.

NOTE 2:	Procedure described in annex D determines whether the present algorithm applies.

Equation (F.1): Calculation of timeout TO_CBF for buffering packets in the CBF packet buffer:

TO_CBF= TO_CBF_MAX+ TO_CBF_MIN –- TO_CBF_MAXDIST_MAX×DISTTO_CBF_MIN    forfor DIST≤DIST_MAX DIST>DIST_MAX	(F.1)

where:

TO_CBF_MIN	is the minimum duration the packet shall be buffered in the CBF packet buffer.

TO_CBF_MAX	is the maximum duration the packet shall be buffered in the CBF packet buffer.

DIST	is the distance between the GeoAdhoc route’'s local position and the sender (i.e., previous forwarder or source) position. The sender position is taken from its LocTE.

DIST_MAX	is the theoretical maximum communication range of the wireless access technology.

NOTE 3:	For DIST = DIST_MAX, TO_CBF_GBC becomes TO_CBF_MIN. For the (theoretical) distance DIST = 0, TO_CBF_GBC becomes TO_CBF_MAX.

TO_CBF_MIN and TO_CBF_MAX shall be set to the GN protocol constants itsGnCbfMinTime and itsGnCbfMaxTime, respectively. If DIST_MAX is not defined in the specification of GeoNetworking media-dependent functionality for the specific ITS access technology, it shall be set to the GN protocol constant itsGnDefaultMaxCommunicationRange.

Upon expiration of the timer, the GAGH of the GeoAdhoc router re-broadcasts the GBC/GAC packet. Before the timer expires, the GAGH may receive a duplicate of the packet from a GeoAdhoc router with a shorter timeout, i.e., with a smaller distance to the destination. In this case, the GAGH inspects its CBF packet buffer, stops the timer and removes the GBC/GAC packet from the CBF packet buffer.

The activity diagram of the area CBF algorithm is depicted in figure F.1 for illustration.

Figure F.1: Area contention-based forwarding algorithm

The algorithm returns one of the following four values:

the Broadcast LL address BCAST;

the LL address of the next hop NH_LL_ADDR;

0 indicates that the packet is buffered in the CBF buffer;

-1 indicates that the packet is discarded.

The pseudo-code of the algorithm is below:

| 1 | -- P is the GN packet to be forwarded |
| --- | --- |
| 2 | -- EPV is the ego position vector with position accuracy indicator PAI_EGO |
| 3 | -- PV_SE is the sender position vector in the LocTE with position accuracy indicator PAI_SE |
| 4 | -- B is the CBF packet buffer |
| 5 | -- TO is the timeout that triggers the re-broadcast of the packet |
| 6 | -- NH_LL_ADDR is the LL address of the next hop |
| 7 | -- BCAST is the Broadcast LL address |
| 8 |  |
| 9 | IF (P is newly generated by ego node) THEN |
| 10 | SET NH_LL_ADDR ← BCAST |
| 11 | RETURN NH_LL_ADDR # Immediate transmission by source router |
| 12 | ELSE IF (P IN B) THEN # Contending |
| 13 | REMOVE P FROM B |
| 14 | STOP TIMER |
| 15 | DISCARD P |
| 16 | RETURN -1 # Indicates that packet is discarded |
| 17 | ELSE # New packet |
| 18 | ADD P TO B |
| 19 | SET SE_POS_VALID ← PV_SE EXISTS AND PAI_SE = TRUE |
| 20 | # Indicates that sender position is valid |
| 21 | IF (SE_POS_VALID AND PAI_EGO = TRUE) THEN |
| 22 | SET DIST ← DIST(PV_SE, EPV) |
| 23 | SET TO ← TO_CBF(DIST) # Eq. F.1 in the present clause |
| 24 | ELSE |
| 25 | # Use maximum timer if DIST cannot be calculated reliably |
| 26 | SET TO ← TO_CBF_MAX |
| 27 | ENDIF |
| 28 | START TIMER(TO) |
| 29 | RETURN 0 # Indicates that packet is buffered |
| 30 | ENDIF |
| 31 |  |
| 32 | IF (TIMER(TO) EXPIRES) THEN |
| 33 | FETCH P FROM B |
| 34 | SET NH_LL_ADDR ← BCAST |
| 35 | RETURN NH_LL_ADDR |
| 36 | ENDIF |

# F.4	Area advanced forwarding algorithm

The area advanced forwarding algorithm includes mechanisms from the Greedy Forwarding (GF) algorithm (annex E.2) and the area contention-based forwarding (CBF) algorithm (annex F.3). As such it is both sender-based and receiver-based. It also includes further enhancements of CBF in order to improve the efficiency and reliability.

The relies on four main mechanisms:

1)	CBF is used to deal with uncertainties in terms of reception failure caused by mobility of ITS-S, fading phenomena and collisions on the wireless medium.

2)	In order to minimize the additional forwarding delay introduced by CBF, CBF is complemented with the selection of one specific forwarder, referred to as next hop, at the sender. Upon reception of the packet, the next hop–- in case of correct reception–- forwards the message immediately.

3)	The efficiency of CBF is improved by choosing potential forwarders only from a specific sector of the circular forwarding area; i.e., GeoAdhoc routers located inside the sector (defined by an angle and the maximum communication range) refrain from retransmission of the packet (sectorial backfire).

4)	The reliability of the dissemination process is increased by a controlled packet retransmission scheme within the geographical target area.

The algorithm returns one of the following four values:

the LL address of the next hop (NH_LL_ADDR);

the Broadcast LL address (BCAST);

0 indicates that the packet is buffered in the CBF buffer;

-1 indicates that the packet is discarded.

The algorithm applies when the GeoAdhoc router is located inside or at the border of the geographical target area carried in the GeoBroadcast packet header. At the source, the algorithm selects the next forwarder from its location table, forwards the packet to the neighbour with the greatest progress (GF) and additionally enters CBF mode (i.e., buffers the packet in the CBF buffer and starts a timer). When a GAGH of a GeoAdhoc router receives a packet, it checks whether it is located inside/at the border of the area. If so and the packet is received as a unicast link layer frame (i.e., the GeoAdhoc router was selected as next hop by the sender of the packet), it again forwards the packet by GF. Otherwise, the GAGH checks whether it is already contending (i.e., the packet is already in the CBF buffer). In this case, the packet is regarded as a duplicate and the GAGH of the GeoAdhoc router counts how often the packet is received and where the sender of the duplicate is located: If the counter exceeds a threshold (COUNTER ≥ MAX_COUNTER) and the local GeoAdhoc router is inside/at the border of the sectorial area, the contention is stopped and the packet is discarded.

For the sectorial backfire (mechanism 3, see above), the algorithm uses a function G with the properties in equation (F.2) and specified in equation (F.3) in order to determine whether the GeoAdhoc router is located inside, at the border or outside a sectorial area that is defined by the sender's position, the distance between sender and local GeoAdhoc router, the distance between the sender and the forwarder, the (theoretical) maximum communication range of the wireless technology and an angle between the forwarder, the sender and the local GeoAdhoc router positions (figure F.2). In principle, if a GeoAdhoc router is contenting in CBF mode and located outside the sectorial area, the packet is scheduled for re-broadcast. If a GeoAdhoc router is located inside the sectorial area, it refrains from contending.

Figure F.2: Sectorial contention area

Equation (F.2): Properties of the geometric function G:

(F.2)

Equation (F.3): Calculation of the sectorial contention area:

(F.3)

where:

DIST_R	is the distance between the GeoAdhoc router's local position and the sender position. The sender position is taken from GeoAdhoc router's local LocTE.

DIST_F	is the distance between the forwarder position and the sender's position. The forwarder and sender positions are taken from the corresponding LocTE of the local GeoAdhoc router.

DIST_MAX	is the theoretical maximum communication range of the wireless access technology.

is the angle between the positions of the forwarder, the sender and the local GeoAdhoc router.

ANGLE_TH	is a threshold value for the angle. This threshold shall have a minimum and a maximum value of 30° and 60°, respectively. It shall vary accordingly to neighbour node density and the default value is given by the GN protocol constant itsGnBroadcastCBFDefSectorAngle.

NOTE:	In a possible implementation, the neighbour nodes density depends on the neighbour density, which is defined by the number of neighbour nodes seen by the local GeoAdhoc router over the geographical area covered by the theoretical maximum communication range of the wireless access technology, see the examples 1, 2 and 3 below.

EXAMPLE 1:	When DEN_NEIGH < 0,025 node/m2 	→ ANGLE_TH = 30°

EXAMPLE 2:	When 0,025 node/m2 < DEN_NEIGH < 0,05 node/m2	→ ANGLE_TH = 45°

EXAMPLE 3:	When DEN_NEIGH ≥ 0,05 node/m2 	→ ANGLE_TH = 60°

In order to increase the reliability of the dissemination process by controlled packet retransmission, a GeoAdhoc router in CBF mode maintains a counter for the number of re-transmissions for a packet. This counter is incremented every time this packet is received. When the number of re-transmissions for this packet reaches a threshold, the GeoAdhoc router stops contending for the packet. By this mechanism, the packet is allowed to be re-transmitted several times for better reliability, but the data overhead is controlled.

The activity diagram of the area advanced forwarding algorithm is illustrated in figure F.3.

Figure F.3: Area advanced forwarding activity diagram

The pseudo-code of the algorithm is below:

| 1 | -- P is the GBC packet to be forwarded |
| --- | --- |
| 2 | -- L_LL_ADDR is the LL address of the GAGH of the local GeoAdhoc router |
| 3 | -- NH_LL_ADDR is the LL address of the next hop |
| 4 | -- DEST_LL_ADDR is the LL destination address carried in P |
| 5 | -- B is the CBF packet buffer |
| 6 | -- EPV is the local position vector with latitude LAT and longitude LONG |
| 7 | -- PV_SE is the sender position vector in its LocTE with latitude LAT_SE, longitude LONG_SE |
| 8 | -- and position accuracy indicator PAI_SE |
| 9 | -- TO is the timeout that triggers the re-broadcast of the packet |
| 10 | -- COUNTER is the retransmit counter for the packet P |
| 11 | -- MAX_COUNTER is the retransmit threshold |
| 12 | -- BCAST is the Broadcast LL address |
| 13 | -- GREEDY() is the GF algorithm as specified in annex E.2 |
| 14 | -- INOUT1 indicates whether the local GeoAdhoc router is outside the target area or not |
| 15 | -- INOUT2 indicates whether the local GeoAdhoc router is outside the sectorial contention |
| 16 | -- area or not. |
| 17 | -- INOUT3 indicates whether the sender is outside the target area or not |
| 18 |  |
| 19 | IF (P is from local node) THEN # If local node is origin, can send directly |
| 20 | SET NH_LL_ADDR ← BCAST |
| 21 | RETURN NH_LL_ADDR |
| 22 | END |
| 23 | SET NH_LL_ADDR ← -1 # Initialize NH_LL_ADDR |
| 24 | SET INOUT1 ← F(LAT,LONG) # Eq. D.1 in Note 2 of annex D |
| 25 | IF (P IN B) THEN # Contending |
| 26 | IF (B.P.COUNTER ≥ MAX_COUNTER) THEN # Stop contending |
| 27 | REMOVE P FROM B # Remove packet from CBF buffer |
| 28 | STOP TIMER |
| 29 | DISCARD P # Discard packet |
| 30 | RETURN -1 # Indicates that packet is discarded |
| 31 | ELSE |
| 32 | SET INOUT2 ← G() # Eq. E.4 for sectorial contention area |
| 33 | IF (INOUT2 ≥ 0) THEN # Inside or at the border of sectorial area |
| 34 | REMOVE P FROM B # Remove packet from CBF buffer |
| 35 | STOP TIMER |
| 36 | DISCARD P # Discard packet |
| 37 | RETURN -1 # Indicates that packet is discarded |
| 38 | ELSE # Outside of sectorial area |
| 39 | SET COUNTER++ |
| 40 | SET TO ← TO_CBF_GBC # Eq. F.1 in annex F.3 |
| 41 | START TIMER(TO) |
| 42 | RETURN 0 # Indicates that packet is buffered |
| 43 | ENDIF |
| 44 | ENDIF |
| 45 | ELSE # New packet |
| 46 | ADD P TO B |
| 47 | IF (DEST_LL_ADDR = L_LL_ADDR) THEN # Greedy forwarding |
| 48 | SET P.COUNTER ← 1 # Initialize COUNTER |
| 49 | SET NH_LL_ADDR ← GREEDY(A) # Greedy()returns LL address of next hop or 0 |
| 50 | SET P.TO ← MAX # Set to TO_CBF_MAX (F.1 in annex F.3) |
| 51 | START TIMER(TO) |
| 52 | RETURN NH_LL_ADDR |
| 53 | ELSE # CBF |
| 54 | IF ((PV_SE EXISTS) OR (PV_SE = EPV)) AND (PAI_SE = TRUE)) THEN |
| 55 | SET DIST ← DIST(PV_SE, EPV) |
| 56 | SET TO ← TO_CBF_GBC # Eq. F.1 in annex F.3 |
| 57 | ELSE |
| 58 | SET TO ← TO_CBF_MAX |
| 59 | ENDIF |
| 60 | START TIMER(TO) |
| 61 | RETURN 0 # Indicates that packet is buffered |
| 62 | ENDIF |
| 63 | ENDIF |
| 64 | IF (TIMER(TO) EXPIRES) THEN |
| 65 | FETCH P FROM B |
| 66 | SET NH_LL_ADDR ← BCAST |
| 67 | RETURN P, NH_LL_ADDR |
| 68 | ENDIF |

Annex G (normative):
GeoNetworking traffic classification

GeoNetworking shall support traffic classification where each GeoNetworking packet is placed into a limited number of traffic classes. The traffic classification of GeoNetworking packets shall be based on the TC field in the GeoNetworking Common Header (clause 9.7.5).

GeoNetworking applies particular mechanisms for data traffic management to each traffic class differently. The GeoNetworking media-independent operations in the present document support SCF per traffic class. Further data traffic management mechanisms are specified in the media-dependent parts of GeoNetworking protocol specification.

A mapping between a traffic class to data traffic management mechanisms is configured by the GN management services at start-up (see annex K).

Annex H (normative):
GeoNetworking protocol constants

The GeoNetworking protocol constants and their default/initial values shall be as specified in table H.1.

The protocol constants represent MIB attributes specified in annex I.

Table H.1: GeoNetworking protocol constants

| Item | GeoNetworking protocol constant | Default/initial value | Comment |
| --- | --- | --- | --- |
| 1 | itsGnLocalGnAddr | 1 | GeoNetworking address of the GAGH of the GeoAdhoc router |
| 2 | itsGnLocalAddrConfMethod | MANAGED (1) | AUTO (0): Local GN_ADDR is configured from MIB MANAGED (1): Local GN_ADDR is configured via the GN management using the service primitive CORE_GAGH (annex K) ANONYMOUS (2): Local GN_ADDR is configured by the security entity |
| 3 | itsGnProtocolVersion | 1 | Version of the GeoNetworking protocol set in the GeoNetworking protocol headers |
| 4 | itsGnIsMobile | Stationary (0) Mobile (1) | Indicates whether ITS-S is stationary or mobile |
| 5 | itsGnIfType | Unspecified (0) ITS-G5 (1) LTE-V2X (2) | Indicates type of interface |
| 6 | itsGnMinUpdateFrequencyEPV | 1 000 | Minimum update frequency of EPV in [1/ms] |
| 7 | itsGnPaiInterval | 80 | Distance related to the confidence interval for latitude and longitude [m]. Used to determine the PAI (clause 9.5.2) |
| 8 | itsGnMaxSduSize | 1 398 | Maximum size of GN-SDU [octets] 1 500–- GN_MAX (88)–- GNSEC_MAX (0) |
| 9 | itsGnMaxGeoNetworkingHeaderSize | 88 | GN_MAX: Maximum size of GeoNetworking header [octets] Without security, the size is determined by the GeoUnicast packet header as defined in clause 9.8.2. If the GeoNetworking packet is secured (clause 9.4) the maximum size of the GeoNetworking header is set to the size of the Basic Header and the size of the Secured Packet |
| 10 | itsGnLifetimeLocTE | 20 | Lifetime of location table entry [s] |
| 11 | itsGnSecurity | DISABLED (0) ENABLED (1) | Indicates whether GN security is enabled (1) or disabled (0) |
| 12 | itsGnSnDecapResultHandling | STRICT (0) NON-STRICT (1) | Indicates the handling of the SN-DECAP result code (service primitive SN-ENCAP.confirm parameter report). If the GN protocol constant itsGnSnDecapResultHandling is set to STRICT (0), received GN packets that are not correctly verified and decrypted (service primitive SN-ENCAP.confirm parameter report != SUCCESS) are always dropped. If itsGnSnDecapResultHandling is set to NON-STRICT (1), GN packets that are not correctly verified and decrypted can be passed to the upper protocol entity for further processing |
| 13 | itsGnLocationServiceMaxRetrans | 10 | Maximum number of retransmissions of LS Request packets |
| 14 | itsGnLocationServiceRetransmitTimer | 1 000 | Duration of Location service retransmit timer [ms] |
| 15 | itsGnLocationServicePacketBufferSize | 1 024 | Size of Location service packet buffer [Octets] |
| 16 | itsGnBeaconServiceRetransmitTimer | 3 000 | Duration of Beacon service retransmit timer [ms] |
| 17 | itsGnBeaconServiceMaxJitter | itsGnBeaconServiceRetransmitTimer / 4 | Maximum beacon jitter [ms] |
| 18 | itsGnDefaultHopLimit | 10 | Default hop limit indicating the maximum number of hops a packet travels |
| 19 | itsGnDPLLength | 8 | Length of Duplicate Packet List (DPL) per source (annex A.2) |
| 20 | itsGnMaxPacketLifetime | 600 | Upper limit of the maximum lifetime [s] |
| 21 | itsGnDefaultPacketLifetime | 60 | Default packet lifetime [s] |
| 22 | itsGnMaxPacketDataRate | 100 | Maximum packet data rate for a GAGH of a GeoAdhoc router [Ko/s]. If the mean (EMA) packet data rate a of a GAGH exceeds the value, packets from this GeoAdhoc router (source or sender) are not forwarded |
| 23 | itsGnMaxPacketDataRateEmaBeta | 90 | Weight factor for the Exponential Moving Average of the packet data rate PDR (annex B.2) in percent |
| 24 | itsGnMaxGeoAreaSize | 10 | Maximum size of the geographical area for a GBC and GAC packet [km2]. If the geographical area size exceeds the maximum value, the GeoNetworking packet shall not be sent (source) and not be forwarded (forwarder) |
| 25 | itsGnMinPacketRepetitionInterval | 100 | Lower limit of the packet repetition interval [ms] |
| 26 | itsGnNonAreaForwardingAlgorithm | GREEDY (1) | Default forwarding algorithm outside target area |
| 27 | itsGnAreaForwardingAlgorithm | CBF (2) | Default forwarding algorithm inside target area |
| 28 | itsGnCbfMinTime | 1 | Minimum duration a GN packet shall be buffered in the CBF packet buffer [ms] |
| 29 | itsGnCbfMaxTime | 100 | Maximum duration a GN packet shall be buffered in the CBF packet buffer [ms] |
| 30 | itsGnDefaultMaxCommunicationRange | 1 000 | Default theoretical maximum communication range [m] |
| 31 | itsGnBroadcastCBFDefSectorAngle | 30 | Default threshold angle for advanced GeoBroadcast algorithm in annex F.4 [degrees] |
| 32 | itsGnUcForwardingPacketBufferSize | 256 | Size of UC forwarding packet buffer [Ko] |
| 33 | itsGnBcForwardingPacketBufferSize | 1 024 | Size of BC forwarding packet buffer [Ko] |
| 34 | itsGnCbfPacketBufferSize | 256 | Size of CBF packet buffer [Ko] |
| 35 | itsGnDefaultTrafficClass | 0x00 | Forwarding: Default traffic class |

Annex I (informative):
ASN.1 encoding of the GeoNetworking MIB

# I.1	Use of modules

The ASN.1 module of the present document is ITSGN {itu-t(0) identified-organization(4) etsi(0) itsgn(2636-4)} as specified in the following clause.

The ASN.1 module adopts textual conventions defined separately and imported as modules. Objects defined using textual conventions are always encoded by means of the rules that define their primitive type. The adapted subsets of ASN.1 notation described in IETF RFC 2578 [i.3] (SNMPv2-SMI) and IETF RFC 2579 [i.5] (SNMPv2-TC) are imported.

# I.2	ASN.1 module

-- *****************************************************************************

-- * ETSI TC ITS TS 103 836-4-1 GeoNetworking MIB

-- *****************************************************************************

ITSGN-MIB DEFINITIONS::=BEGIN

IMPORTS

MODULE-IDENTITY, OBJECT-TYPE,

Unsigned32, Integer32,

enterprises                      		FROM SNMPv2-SMI

SnmpAdminString                      	FROM SNMP-FRAMEWORK-MIB

TEXTUAL-CONVENTION, TruthValue        	FROM SNMPv2-TC

InterfaceIndex                        	FROM IF-MIB;

-- *****************************************************************************

-- * MODULE IDENTITY

-- *****************************************************************************

itsGn MODULE-IDENTITY

LAST-UPDATED"202205230000Z"

ORGANIZATION"ETSI Technical Committee ITS WG"

CONTACT-INFO

"WG Email:    ITS_WG3@LIST.ETSI.OR"

DESCRIPTION

"The MIB module for TS 103 836-4 (GeoNetworking) entities

itu-t(0).identified-organization(4).etsi(0).itsgn(38364"

REVISION     "202205230000Z"

DESCRIPTION "Initial version: TS 103 836-4-1 V0.0.4"

::= { enterprises 13019 26364 }

-- *****************************************************************************

-- * PRIMARY GROUPS

-- *****************************************************************************

itsGnObjects     OBJECT IDENTIFIER ::= { itsGn 1 }

itsGnStatistics  OBJECT IDENTIFIER ::= { itsGn 2 }

itsGnConformance OBJECT IDENTIFIER ::= { itsGn 3 }

-- *****************************************************************************

-- * SUB GROUPS

-- *****************************************************************************

itsGnMgmt     OBJECT IDENTIFIER ::= { itsGnObjects 1 }

-- *****************************************************************************

-- * SUB GROUPS

-- *****************************************************************************

itsGnSystem           OBJECT IDENTIFIER ::= { itsGnMgmt 1 }

itsGnConfig           OBJECT IDENTIFIER ::= { itsGnMgmt 2 }

itsGnLocationService  OBJECT IDENTIFIER ::= { itsGnMgmt 3 }

itsGnBeaconService    OBJECT IDENTIFIER ::= { itsGnMgmt 4 }

itsGnPacketForwarding OBJECT IDENTIFIER ::= { itsGnMgmt 5 }

-- *****************************************************************************

-- * TEXTUAL CONVENTIONS

-- *****************************************************************************

GnAddress ::= TEXTUAL-CONVENTION

DISPLAY-HINT"2x:2x:2x:2"

STATUS       current

DESCRIPTION

"Represents a GeoNetworking address:

Octets   Contents         Encoding

1-8     GN address       network-byte order"

SYNTAX       OCTET STRING (SIZE (8))

-- *****************************************************************************

-- * GN OBJECTS GROUP

-- *****************************************************************************

-- *****************************************************************************

-- * GN SYSTEM GROUP

-- *****************************************************************************

itsGnIfTable        OBJECT-TYPE

SYNTAX      SEQUENCE OF ItsGnIfEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A table representing the interfaces that will be used by the

GeoAdhoc router for communication.  Each entry in this table

represents a configured egress interface.

"

::= { itsGnSystem 1 }

itsGnIfEntry OBJECT-TYPE

SYNTAX      ItsGnIfEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"An entry in the interface table.  It

represents a single interface entry.

"

INDEX  { itsGnIfIndex }

::= { itsGnIfTable 1 }

ItsGnIfEntry ::=

SEQUENCE {

itsGnIfIndex             InterfaceIndex,

itsGnIfPriority          Unsigned32,

itsGnIfDescription       SnmpAdminString

}

itsGnIfIndex  OBJECT-TYPE

SYNTAX      InterfaceIndex

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The index of the interface of the GeoAdhoc router.

"

::= { itsGnIfEntry 1 }

itsGnIfPriority   OBJECT-TYPE

SYNTAX      Unsigned32 (0..255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The priority configured to the interface.

This value will be configured to a value between 0

and 255.

"

::= { itsGnIfEntry 2 }

itsGnIfDescription   OBJECT-TYPE

SYNTAX      SnmpAdminString

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A human-readable textual description of the

interface on the GeoAdhoc router.

"

::= { itsGnIfEntry 3 }

-- *****************************************************************************

-- * GN CONFIGURATION SUB GROUP

-- *****************************************************************************

itsGnLocalGnAddr OBJECT-TYPE

SYNTAX      GnAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"GeoNetworking address of the GeoAdhoc router"

::= { itsGnConfig 1 }

itsGnLocalAddrConfMethod OBJECT-TYPE

SYNTAX      INTEGER {

auto(0),

managed(1),

anonymous(2)

}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"AUTO: Local GN_ADDR is configured from MIB

MANAGED: Local GN_ADDR is configured via the GN management using the service primitive CORE_GAGH

ANONYMOUS: Local GN_ADDR is configured by the security entit"

::= { itsGnConfig 2 }

itsGnProtocolVersion OBJECT-TYPE

SYNTAX      Unsigned32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"GeoNetworking protocol version"

::= { itsGnConfig 3 }

itsGnIsMobile OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates whether ITS station is stationary or mobile"

::= { itsGnConfig 4 }

itsGnIfType OBJECT-TYPE

SYNTAX      INTEGER{

unspecified(0),

its-g5(1),

lte-v2x(2)

}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"ITS interface type"

::= { itsGnConfig 5 }

itsGnMinUpdateFrequencyEPV OBJECT-TYPE

SYNTAX      Integer32(0..65635)

UNITS      "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Minimum update frequency of local position vector (EPV) in ms"

::= { itsGnConfig 6 }

itsGnPaiInterval OBJECT-TYPE

SYNTAX      Integer32(0..100)

UNITS      "meter"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Distance related to the confidence interval of latitude and longitude in m. Used to determine the Position Accuracy Indicator (PAI)"

::= { itsGnConfig 7 }

itsGnMaxSduSize OBJECT-TYPE

SYNTAX      Integer32(0..65635)

UNITS      "Byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Maximum size of GN-SDU in bytes"

::= { itsGnConfig 8 }

itsGnMaxGeoNetworkingHeaderSize OBJECT-TYPE

SYNTAX      Integer32(0..65635)

UNITS      "Byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Maximum size of GeoNetworking header in bytes"

::= { itsGnConfig 9 }

itsGnLifetimeLocTE OBJECT-TYPE

SYNTAX      Integer32(0..65635)

UNITS      "Second"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

" Location table maintenance: Lifetime of an entry in the location table

in s"

::= { itsGnConfig 10 }

itsGnSecurity OBJECT-TYPE

SYNTAX      INTEGER {

disabled   (0),

enabled    (1)

}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates whether GN security is enabled (1) or disabled (0)"

::= { itsGnConfig 11 }

itsGnSnDecapResultHandling OBJECT-TYPE

SYNTAX      INTEGER {

strict     (0),

non-strict (1)

}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the handling of the SN-DECAP result code (service

primitive SN-ENCAP.confirm parameter report). If set to STRICT

(0), received GN packets that are not correctly verified and

decrypted (service primitive SN-ENCAP.confirm parameter report

!= CORRECT) are always dropped. If set to NON-STRICT (1), GN

packets that are not correctly verified and decrypted can be

passed to the upper protocol entity for further processing"

::= { itsGnConfig 12 }

-- *****************************************************************************

-- * GN LOCATION SERVICE SUB GROUP

-- *****************************************************************************

itsGnLocationServiceMaxRetrans OBJECT-TYPE

SYNTAX      Integer32(0..255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Location service: Maximum number of retransmissions for a LS Request"

::= { itsGnLocationService 1 }

itsGnLocationServiceRetransmitTimer OBJECT-TYPE

SYNTAX      Integer32(0..65535)

UNITS      "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Location service: Duration of LS request retransmit timer in ms"

::= { itsGnLocationService 2 }

itsGnLocationServicePacketBufferSize OBJECT-TYPE

SYNTAX      Integer32(0..65535)

UNITS      "Byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Location service: Size of LS packet buffer in bytes"

::= { itsGnLocationService 3 }

-- *****************************************************************************

-- * GN BEACON SERVICE SUB GROUP

-- *****************************************************************************

itsGnBeaconServiceRetransmitTimer OBJECT-TYPE

SYNTAX      Integer32(0..65535)

UNITS      "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Beacon service: Duration of Beacon retransmit timer in ms"

::= { itsGnBeaconService 1 }

itsGnBeaconServiceMaxJitter OBJECT-TYPE

SYNTAX      Integer32(0..65535)

UNITS      "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Beacon service: Maximum Beacon jitter in ms"

::= { itsGnBeaconService 2 }

-- *****************************************************************************

-- * GN PACKET FORWARDING SUB GROUP

-- *****************************************************************************

itsGnDefaultHopLimit OBJECT-TYPE

SYNTAX      Integer32(0..255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Default hop limit indicating the maximum number of hops a packet travels"

::= { itsGnPacketForwarding 1 }

itsGnDPLLength OBJECT-TYPE

SYNTAX      Integer32(0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Length of Duplicate Packet List (DPL) per source"

::= { itsGnPacketForwarding 2 }

itsGnMaxPacketLifetime OBJECT-TYPE

SYNTAX      Integer32(0..6300)

UNITS      "second"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Upper limit of the maximum lifetime of a packet in s"

::= { itsGnPacketForwarding 3 }

itsGnDefaultPacketLifetime OBJECT-TYPE

SYNTAX      Integer32(0..6300)

UNITS      "second"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Default value of the maximum lifetime of a packet in s"

::= { itsGnPacketForwarding 4 }

itsGnMaxPacketDataRate OBJECT-TYPE

SYNTAX      Integer32

UNITS      "kBytes/"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Maximum packet data rate for a GAGH in a GeoAdhoc router in

[kBytes/s]. If the mean (EMA) packet data rate exceeds the

value, packets from this GAGH (source or sender) are

not forwarded"

::= { itsGnPacketForwarding 5 }

itsGnMaxPacketDataRateEmaBeta OBJECT-TYPE

SYNTAX      Integer32

UNITS      "NA"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Weight factor for the Exponential Moving Average (EMA) of the packet data rate PDR in percent"

::= { itsGnPacketForwarding 6 }

itsGnMaxGnAreaSize OBJECT-TYPE

SYNTAX      Integer32

UNITS      "km"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Maximum size of the geographical area for a GBC and GAC packet

[km2]. If the geographical area size exceeds the maximum value,

The GeoNetworking packet shall not be sent (source) and not be

forwarder (forwarder"

::= { itsGnPacketForwarding 7 }

itsGnMinPacketRepetitionInterval OBJECT-TYPE

SYNTAX      Integer32(0..1000)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Lower limit of the packet repetition interval in ms"

::= { itsGnPacketForwarding 8 }

itsGnGNNonAreaForwardingAlgorithm OBJECT-TYPE

SYNTAX      INTEGER {

unspecified   (0),

greedy        (1),

cbf           (2)

}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Default GeoUnicast forwarding algorithm"

::= { itsGnPacketForwarding 9 }

itsGnAreaForwardingAlgorithm OBJECT-TYPE

SYNTAX      INTEGER {

unspecified   (0),

simple        (1),

cbf		(2),

advanced	(3)

}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Default GeoBroadcast forwarding algorithm"

::= { itsGnPacketForwarding 10 }

itsGnCbfMinTime OBJECT-TYPE

SYNTAX      Integer32(0..65635)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Minimum duration a GeoBroadcast packet shall be buffered in the CBF packet buffer in ms"

::= { itsGnPacketForwarding 11 }

itsGnCbfMaxTime OBJECT-TYPE

SYNTAX      Integer32(0..65635)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Maximum duration a GeoBroadcast packet shall be buffered in the CBF packet buffer in ms"

::= { itsGnPacketForwarding 12 }

itsGnDefaultMaxCommunicationRange OBJECT-TYPE

SYNTAX      Integer32(0..65635)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Default theoretical maximum communication range in m"

::= { itsGnPacketForwarding 13 }

itsGnBroadcastCBFDefSectorAngle OBJECT-TYPE

SYNTAX      Integer32(0..180)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Default threshold angle for advanced GeoBroadcast algorithm in degrees"

::= { itsGnPacketForwarding 14 }

itsGnUcForwardingPacketBufferSize OBJECT-TYPE

SYNTAX      Integer32(0..255)

UNITS      "kByt"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Forwarding: Size of UC forwarding packet buffer in kByte"

::= { itsGnPacketForwarding 15 }

itsGnBcForwardingPacketBufferSize OBJECT-TYPE

SYNTAX      Integer32(0.. 65535)

UNITS      "kByt"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Forwarding: Size of BC forwarding packet buffer in kByte"

::= { itsGnPacketForwarding 16 }

itsGnCbfPacketBufferSize OBJECT-TYPE

SYNTAX      Integer32(0.. 65535)

UNITS      "kByt"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Forwarding: Size of CBF packet buffer [Kbytes]."

::= { itsGnPacketForwarding 17 }

itsGnTrafficClass OBJECT-TYPE

SYNTAX      Integer32(0..255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Forwarding: Default traffic class"

::= { itsGnPacketForwarding 18 }

END

Annex J (informative):
GeoNetworking data interface

# J.1	General

The GN data service primitives allow entities of ITS transport protocols to send and receive packets via the TRANSP_CORE interface (figure 1).

# J.2	TRANSP_CORE.request

The service primitive TRANSP_CORE.request is used by the ITS transport protocol entities to request sending a GN packet. Upon reception of the service primitive TRANSP_CORE.request, the GN Core delivers the transport packet to the active GAGH associated to the AliID provided via the CORE_GAGH interface.

The parameters of the TRANSP_CORE.request are as follows:

TRANSP_CORE.request (

Upper protocol entity,

Packet transport type,

Destination address,

AliID,

Security profile, (optional)

ITS-AID length, (optional)

ITS-AID, (optional)

Security permissions length, (optional)

Security permissions, (optional)

Security context information, (optional)

Security target ID list length, (optional)

Security target ID list,(optional)

Maximum packet lifetime, (optional)

Repetition interval, (optional)

Maximum repetition time, (optional)

Maximum hop limit, (optional)

Traffic class,

Length,

Data

)

The Upper protocol entity parameter specifies whether the service primitive was triggered by an ITS Transport protocol (e.g., BTP) or by the GeoNetworking to IPv6 Adaptation Sub-Layer (GN6ASL). In case the upper protocol entity is the IPv6 Adaptation Sub-Layer, the AliID for the message is requested to the MCO_NET entity.

The Packet transport type parameter specifies the packet transport type. The parameter can be either: (0) GUC, (1) SHB, (2) TSB, (3) GBC, (4) GAC.

The Destination parameter specifies the destination address for GeoUnicast or the geographical area for GBC/GAC. The destinations address for GeoUnicast can optionally contain the MID field only; with the other fields set to 0 (see figure 3 and table 1).

The AliID parameter indicates the ALI at the Access layer that should be used for transmission and the GAGH to use to handle this packet.

The Security profile parameter determines the security service to invoke.

The ITS-AID length parameter specifies the length of the value provided in the ITS-AID parameter.

The ITS-AID parameter specifies the ITS-AID for the payload to be sent.

The Security permissions length parameter specifies the length of the value provided in the Security permissions parameter.

The Security permissions parameter specifies the SSP associated with the ITS-AID.

The Security context information parameter specifies information to be used to selecting properties of the security protocol.

The Security target ID list length parameter specifies the length for the value of the SecurityTarget ID List parameter.

The Security target ID list parameter specifies an unordered collection of target IDs used by the security entity, for specifying multiple recipients.

The Maximum lifetime parameter specifies the maximum tolerable time in [s] a GeoNetworking packet can be buffered until it reaches its destination. The parameter is optional. If it is not used, the GN protocol constant itsGnDefaultPacketLifetime is used.

The Repetition interval parameter specifies the duration between two consecutive transmissions of the same GeoNetworking packet during maximum repetition time of a packet in [ms]. The parameter is optional. If it is not used, the packet is not repeated.

The Maximum repetition time parameter specifies the duration in [ms] for which the packet will be repeated if the Repetition interval is set. The parameter is optional; if the Repetition interval is not used, it is omitted.

The Maximum Hop Limit specifies the number of hops a packet is allowed to have in the network, i.e., how often the packet is allowed to be forwarded.

The Traffic class parameter specifies the traffic class for the message.

The Length parameter indicates the length of the Data.

The Data parameter represents the payload of the GeoNetworking packet to be sent.

# J.3	TRANSP_CORE.confirm

The service primitive TRANSP_CORE.confirm is used to confirm that the GeoNetworking packet was successfully processed in response to a TRANSP_CORE.request. For the reception of the primitive, no behaviour is specified.

The parameters of the service primitive are as follows:

TRANSP_CORE.confirm (

ResultCode

)

The ResultCode parameter specifies whether the service primitive TRANSP_CORE.request is:

1)	accepted;

2)	rejected due to maximum length exceeded if the size of the T/GN6-PDU exceeds the GN protocol constant itsGnMaxSduSize;

3)	rejected due to maximum lifetime exceeded if the lifetime exceeds the maximum value of the GN protocol constant itsGnMaxPacketLifetime;

4)	rejected due to repetition interval too small, if the repetition interval is smaller than the GN protocol constant itsGnMinPacketRepetitionInterval;

5)	rejected due to unsupported traffic class;

6)	rejected due to non-valid AliID;

7)	rejected due to geographical area exceeds the maximum geographical area size in the GN protocol constant itsGnMaxGeoAreaSize; or

8)	rejected for unspecified reasons if the service primitive TRANSP_CORE.request cannot be accepted for any other reason.

# J.4	TRANSP_CORE.indication

The service primitive TRANSP_CORE.indication indicates to an upper protocol entity that a GeoNetworking packet has been received. The service primitive is generated by the GN Core to deliver data contained in a received GeoNetworking packet to upper protocol entity. The data of the GeoNetworking packet are processed as determined by the receiving upper protocol entity.

The parameters of the service primitive TRANSP_CORE.indication are as follows:

TRANSP_CORE.indication (

Upper protocol entity,

Packet transport type,

AliID,

Destination, (optional)

Source position vector,

Security report, (optional)

Certificate id, (optional)

ITS-AID length, (optional)

ITS-AID, (optional)

Security permissions length, (optional)
			Security permissions, (optional)

Traffic class,

Remaining packet lifetime, (optional),

Remaining hop limit, (optional)

Length,

Data    -- T/GN6-PDU

)

The Upper protocol entity parameter determines the protocol entity that processes the service primitive (BTP or GN6).

The Packet transport type parameter is the packet transport type (GUC, SHB, TSB, GBC, GAC) of the received packet.

The AliID parameter indicates the ALI at the Access layer that received the packet.

The Destination parameter is the destination address for GeoUnicast or the geographical area for GeoBroadcast/GeoAnycast with which the GeoNetworking packet was generated by the source.

The Source position vector parameter is the geographical position for the source of the received GeoNetworking packet.

The Security report contains result information from the security operations for decryption and verification (parameter report in the service primitive SN-DECAP.confirm).

The Certificate id contains the identification of source certificate, for example the certificate hash (parameter certificate_id in the service primitive SN-DECAP.confirm).

The ITS-AID length parameter specifies the length of the value provided in the ITS-AID parameter (parameter its_aid_length in the service primitive SN-DECAP.confirm).

The ITS-AID parameter specifies the ITS-AID for the received payload (parameter its_aid in the service primitive SN-DECAP.confirm).

The Security permissions length parameter specifies the length of the value provided in the Security permissions parameter (parameter permissions_length in the service primitive SN-DECAP.confirm).

The Security permissions parameter contains the sender permissions (parameter permissions in the service primitive SN-DECAP.confirm).

The Traffic class parameter is the traffic class, with which the GeoNetworking packet was generated by the source.

The Remaining packet lifetime parameter is the remaining lifetime of the packet.

The Remaining hop limit parameter is the remaining hop limit of the packet.

The Length parameter is the length of the Data parameter.

The Data parameter is the payload of the received GeoNetworking packet, i.e., the T-PDU/GN6-PDU.

Annex K (informative):
GN Core network management services

# K.1	General

The GN network management service primitives provide position, time and GeoNetworking address updates to the GAGHs.

# K.2	CORE_GAGH.request

The service primitive CORE_GAGH.request is generated by the GeoNetworking protocol upon activation of a GAGH in order to request management information, i.e., time, position vector, GeoNetworking address, TC mapping. After receiving the service primitive CORE_GAGH.request, the GN Core is in charge of providing the GAGH with the requested management information.

The parameter of the CORE_GAGH.request is as follows:

CORE_GAGH.request (

Request cause

)

The Request cause parameter specifies the type of requested information, i.e., time, position vector, GeoNetworking address, TC mapping. In case the GeoNetworking address is requested, the parameter also indicates whether the address request is caused by DAD or is an initial request.

# K.3	CORE_GAGH.response

The service primitive CORE_GAGH.response is generated by the GN Core to indicate an update of management information, i.e., time, position vector, GeoNetworking address and TC mapping. The service primitive can be triggered upon reception of a CORE_GAGH.request primitive or can be generated unsolicited, i.e., without a service primitive CORE_GAGH.request.

The parameters of the CORE_GAGH.response are as follows:

CORE_GAGH.response (

Time (optional)

Local position vector (optional)

GeoNetworking address (optional)
				TC mapping (optional)

)

The Time parameter specifies the timestamp that is used as a reference to determine the freshness of received information carried in packets.

The Local position vector parameter specifies the ITS-S's most recent position vector (geographical position, speed, heading, timestamp when the position vector was generated, and corresponding accuracy information).

The GeoNetworking address parameter specifies the GeoNetworking address that is used by the GeoNetworking protocol.

The TC mapping parameter specifies the mapping of Traffic class IDs to TC-related parameters (annex G).

All parameters are optional, whereas at least one parameter is present.

# K.4 	CORE_GAGH.indication

The service primitive CORE_GAGH.indication indicates to the GN Core entity that a GeoNetworking packet has been received. The service primitive is generated by a GAGH to deliver data contained in a received GeoNetworking packet to upper protocol entity. The data of the GeoNetworking packet are processed as determined by the receiving upper protocol entity.

The parameters of the service primitive CORE_GAGH.indication are as follows:

TRANSP_CORE.indication (

Upper protocol entity,

Packet transport type,

AliID,

Destination, (optional)

Source position vector,

Security report, (optional)

Certificate id, (optional)

ITS-AID length, (optional)

ITS-AID, (optional)

Security permissions length, (optional)
			Security permissions, (optional)

Traffic class,

Remaining packet lifetime, (optional),

Remaining hop limit, (optional)

Length,

Data    -- T/GN6-PDU

)

The Upper protocol entity parameter determines the protocol entity that processes the service primitive (BTP or GN6).

The Packet transport type parameter is the packet transport type (GUC, SHB, TSB, GBC, GAC) of the received packet.

The AliID parameter indicates the ALI at the Access layer that received the packet.

The Destination parameter is the destination address for GeoUnicast or the geographical area for GeoBroadcast/GeoAnycast with which the GeoNetworking packet was generated by the source.

The Source position vector parameter is the geographical position for the source of the received GeoNetworking packet.

The Security report contains result information from the security operations for decryption and verification (parameter report in the service primitive SN-DECAP.confirm).

The Certificate id contains the identification of source certificate, for example the certificate hash (parameter certificate_id in the service primitive SN-DECAP.confirm).

The ITS-AID length parameter specifies the length of the value provided in the ITS-AID parameter (parameter its_aid_length in the service primitive SN-DECAP.confirm).

The ITS-AID parameter specifies the ITS-AID for the received payload (parameter its_aid in the service primitive SN-DECAP.confirm).

The Security permissions length parameter specifies the length of the value provided in the Security permissions parameter (parameter permissions_length in the service primitive SN-DECAP.confirm).

The Security permissions parameter contains the sender permissions (parameter permissions in the service primitive SN-DECAP.confirm).

The Traffic class parameter is the traffic class, with which the GeoNetworking packet was generated by the source.

The Remaining packet lifetime parameter is the remaining lifetime of the packet.

The Remaining hop limit parameter is the remaining hop limit of the packet.

The Length parameter is the length of the Data parameter.

The Data parameter is the payload of the received GeoNetworking packet, i.e., the T-PDU/GN6-PDU.

Annex L (informative):
MCO_NET services for the GN Core

# L.1	General

The MCO_NET entity provides services to the GN Core to provide GAGH status updates and to request ALI for packets without assigned AliID.

# L.2	CORE_MCO.request

The service primitive CORE_MCO.request is generated by the GN Core upon reception of a packet request without assigned AliID, e.g., in the case of transmission request from the IPv6 Adaptation Sub-Layer.

The parameter of the CORE_MCO.request is as follows:

CORE_GAGH.request (

ALI request

Request number

)

The ALI request parameter identifies the ALI request type. The request type can have the following type: (0) IPv6 ALI request, (1) Other.

The Request number parameter identifies the request that will be used by MCO_NET for the response.

# L.3	CORE_MCO.response

The service primitive CORE_MCO.response is generated by MCO_NET to indicate a response of a previously performed CORE_MCO.request.

CORE_MCO.response (

ALI request

Request number

AliID

Expiration Time

)

The ALI request parameter identifies the ALI request type. The type of the request can be: (0) IPv6 ALI request, or (1) Other.

The Request number parameter is the same as the one indicated in the CORE_MCO.request.

The AliID parameter indicates the ALI that should be used for the packet type as described in the ALI request.

The Expiration Time is the time until which the provided AliID is valid for the performed request.

# L.4	CORE_MCO.update

The service primitive CORE_MCO.update is generated by MCO_NET when a GAGH update should be provided to the GN Core.

The parameters of the CORE_MCO.update are as follows:

CORE_MCO.update (

GAGH

GAGH Status

)

The GAGH parameter indicates the GAGH for which the status update is.

The GAGH Status parameter contains the new status for the GAGH indicated. The status can be (1) Active, (2) Inactive, or (3) Disabled.

Annex M (informative):
MCO_NET services for the GAGHs

# M.1	General

The MCO_NET entity provides services to the GAGHs to receive and collect network updates and status updates.

# M.2	GAGH_MCO.indicate

The service primitive GAGH_MCO.indicate is generated by a GAGH to provide a network status update to MCO_NET.

The parameter of the GAGH_MCO.indicate is as follows:

CORE_GAGH.requestindicate (

GAGH ID

Network Update

Access Technology

)

The GAGH ID parameter is the ID of the GAGH indicating its network update.

The Network Update parameter contains the parameter values of the network update. These parameters may depend on the access technology handled by the GAGH.

The Access Technology identifies the access technology handled by the GAGH.

# M.3	GAGH_MCO.update

The service primitive GAGH_MCO.update is generated by MCO_NET to provide either a status update or a received local CLR from the Access layer to a GAGH.

The parameter of the GAGH_MCO.indicate is as follows:

GAGH_MCO.update	(

State (optional)

CLR (optional)

)

The optional State parameter indicates the new state of the GAGH as described in clause 10.4.2.2.

The optional CLR parameter indicates the Channel Load Ratio perceived for an ALI group.

Annex N (informative):
MCO services for the Facilities layer

# N.1	General

The MCO_NET data service primitives for the Facilities layer enable entities of the Facilities layer to request MCO parameters configurations, receive MCO parameters and network updates with the MCO_FAC_NET interface (figure 1).

# N.2	MCO_FAC_NET.request

The service primitive MCO_FAC_NET.request is generated by a Facilities layer entity to perform a request for MCO parameters configuration to MCO_NET.

The parameter of the MCO_FAC_NET.request is as follows:

CORE_GAGH.requestrequest (

AliGroupID

AliGroupID status

)

The AliGroupID is the ALI group for which the parameters are provided.

The AliGroupID status parameter contains the new requested status for the AliGroupID indicated. The status can be either: (0) deactivation, (1) activate message reception, (2) active message transmission and reception.

# N.3	MCO_FAC_NET.update

The service primitive MCO_FAC_NET.update is generated by MCO_NET to provide a network update to the Facilities layer.

MCO_FAC_NET.update	(

AliGroupID (optional)

Network status (optional)

Access layer notification (optional)

ALI request (optional)

Request number (optional)

)

The AliGroupID parameter is the ALI group for which the update is provided. This parameter is only optional in the case the update is about an ALI request.

The Network status parameter contains information about the network status for the indicated AliGroupID. It may for example contain the local CLR or the CLRs received by other ITS-Ss.

The Access layer notification parameter is provided by the Access layer entities and forwarded to the Facilities layer via this interface.

The ALI request parameter indicates if an AliID is requested for a packet for which no AliID was assigned. The ALI request can either be for: (0) IPv6, (1) other.

The Request number parameter identities the request. It can then be used by the Facilities layer entities to indicate the selected AliID for the request.

# N.4	MCO_FAC_NET.indicate

The service primitive MCO_FAC_NET.indicate is generated by the Facilities layer entities to the MCO_NET entity to indicate the selected AliID for an ALI request performed by MCO_NET.

MCO_FAC_NET.indicate	(

ALI request

Request number

AliID

Expiration time

)

The ALI request should be the same as indicated in MCO_FAC_NET.update service primitive used to perform the AliID request.

The Request number parameter should have the same value as the initial request performed by the MCO_NET entity.

The AliID parameter indicates the selected AliID for the performed ALI request.

The Expiration time parameter indicated the period of time for which the selected AliID can be used for the ALI request.

Annex O (informative):
Interface to the security entity

The GeoNetworking protocol exchanges information with the security entity via the CORE_SEC and GAGH_SEC interfaces (figure 1). The GeoNetworking protocol may use the following security services (see [i.1] to find corresponding specifications):

1)	SN-ENCAP;

2)	SN-DECAP;

3)	SN-IDCHANGE-SUBSCRIBE;

4)	SN-IDCHANGE-EVENT;

5)	SN-IDCHANGE-UNSUBSCRIBE;

6)	SN-IDCHANGE-TRIGGER.

Annex P (informative):
Bibliography

ETSI TS 102 890-1: "Intelligent Transport Systems (ITS); Facilities layer function; Part 1: Services Announcement (SA) specification".

ETSI TS 102 890-2: "Intelligent Transport Systems (ITS); Facilities layer function; Part 2: Position and time facility specification".

ETSI TR 102 707: "Intelligent Transport Systems (ITS); ETSI object identifier tree; ITS domain".

ETSI TS 102 723-1: "Intelligent Transport Systems; OSI Cross-Layer Topics; Part 1: Architecture and Addressing Schemes".

EU FP7 GEONET Project: "Deliverable D2.2 Final GeoNet Specification", ETSI Document # ITSWG3(10)0011, January 2010.

SIM TD Project: "Deliverable D21.4 Spezifikation der Kommunikationsprotokolle", September 2009.

ISO/TS 1824-2:2006: "Traffic and Travel Information (TTI) - TTI via Transport Protocol Expert Group (TPEG) data-streams - Part 2: Syntax, Semantics and Framing Structure (SSF)".

M. Torrent Moreno, J. Mittag, P. Santi, H. Hartenstein: "Vehicle-to-Vehicle Communication: Fair Transmit Power Control for Safety-Critical Information", IEEE Transactions on Vehicular Technology, Volume 58, Issue 7, pp. 3684-3707, September 2009.

A. Festag, P. Papadimitratos, T. Tielert: "Design and Performance of Secure Geocast for Vehicular Communication", IEEE Transactions on Vehicular Technology, Volume 59, Issue 5, pp. 1 - 16, June 2010.

M. Wetterwald, F. Hrizi, P. Cataldi: "Cross-layer identities management in ITS stations", 10th IEEE International Conference on ITS Telecommunications (ITST), November 2010, Kyoto, Japan.

M.N. Mariyasagayam, T. Osafune, M. Lenardi: "Enhanced Multi-Hop Vehicular Broadcast (MHVB) for Active Safety Applications", 7th IEEE International Conference on ITS Telecommunications (ITST), June 2007.

ETSI EN 302 663: "Intelligent Transport Systems (ITS); ITS-G5 Access layer specification for Intelligent Transport Systems operating in the 5 GHz frequency band".

ISO/IEC 8802-2:1998: "Information technology -Telecommunications and information exchange between systems-Local and metropolitan area networks-Specific requirements - Part 2: Logical link control".

Annex (informative):
Change History

| Date | Version | Information about changes |
| --- | --- | --- |
| 2022-01 | 0.0.2 | Created base version of draft based on EN 302 636-4-1 v1.4.1 GeoNetworking media-independent functionality (Release 1) and converted from EN template to TS template |
| 2022-02 | 0.0.3 | Changes based on ITSWG3(22)057004r1 “Proposed MCO-related extensions to GeoNetworking media-independent Release 2 (rev 1)” |
| 2022-05 | 0.0.4 | Clean up references, add informative annexes L, M, and N and modify Clause 10 figures and text. |

# History

| Document history |  |  |
| --- | --- | --- |
| <Version> | <Date> | <Milestone> |