# 3GPP TSG SA WG3 (Security) Meeting #94 ad-hoc	S3-190702

# 11 – 15 March 2019, Kista (Sweden)	revision of S3-19xabc

# Source:	Huawei, Hisilicon

# Title:	Key issue on Key freshness in AKMA

# Document for:	Approval

Agenda Item:	5.5

1	Decision/action requested

It is requested to approve the proposal into TS 33.835.

2	References

[1]	3GPP TR 33.835 Study on authentication and key management for applications based on 3GPP credential in 5G

3	Rationale

A weakness of AKMA as a key agreement scheme is that there is no guarantee of freshness of the application key KAF without forcing a re-run of AKA. That is if an Application Function that requests a key from the Anchor Function, that key might have already been used. In general key freshness is a desirable property of any method used to establish keys and should be included in AKMA.

4	Detailed proposal

*************** Start of Change 1 ****************

5.X	Key Issue #X: Key freshness in AKMA

5.X.1 Issue details

A weakness of AKMA as a key agreement scheme is that there is no guarantee of freshness of the application key KAF without forcing a re-run of AKA. That is if an Application Function that requests a key from the Anchor Function, that key might have already been used. In general key freshness is a desirable property of any method used to establish keys and should be included in AKMA.

The application key KAF is derived from the anchor key KAKMA. If KAKMA is not expired, it is complicated to refresh KAF through a run of AKA. Thus, it is better to refresh KAF without a re-run of AKA.

5.X.2 Security Threats

If a KAF is used without freshness, then a weakness between UE and Application Function may allow an attacker to pretend to be a particular Application Function and obtain KAF. The attacker can masquerade as the UE towards the real Application Function.

### 5.X.3	Potential security requirements

The application key KAF shall keep fresh without a forcing re-run of AKA.

*************** End of Change 1 ****************