3GPP TSG-CT3 Meeting #137	C3-245240

# Hefei, China, 14th Oct 2024 - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0377 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for Provider management API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCAPIF |  |  |  |  |  | Date: |  |  | 2024-10-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per old CR C3-202398 in Rel-16, CAPIF_API_Provider_Management API’s attribute names were optimized which included the name change of the attribute from apiProviderDomainId to apiProvDomId and from supportedFeatures to suppFeat tin the APIProviderEnrolmentDetails data structure, however the new attribute names were not updated in the respective service operation procedure and corresponding resource standards method for this API. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to: - Correct the attribute name in the procedure and in the resource standards method. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | wrong specification implementation in stage-3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.11.2.3.2, 8.9.2.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact the OpenAPI descriptions defined in this specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st Change * * * *

##### 5.11.2.3.2	API management function updating API provider domain function details on CAPIF using Update_API_Provider service operation

To update the API provider domain profile and its individual functions details on CAPIF domain, the API management function shall send a HTTP PUT message to its resource representation in the CAPIF core function as specified in clause 8.9.2.3.3.1, requesting to replace all properties in the existing resource, addressed by the URI received in the response to the request that has created the API provider domain profile resource. The property "apiProvDomId", shall remain unchanged from the previously provided values. The body of the HTTP PUT message shall include the APIProviderEnrolmentDetails data structure that need to be updated. If the "PatchUpdate" feature defined in clause 8.9.6 is supported for modification of the API provider domain profile, the consumer (e.g. API publishing function) may send an HTTP PATCH request message to the concerned service API resource URI in the CAPIF core function. The body of the HTTP PATCH request message shall include the APIProviderEnrolmentDetailsPatch data structure.

Upon receiving the described HTTP PUT or PATCH request message:

1.	the CAPIF core function shall process the updates received in the HTTP PUT or PATCH request message and determine if the request sent by API management function is authorized or not;

2.	verify that the "apiProvDomId" property is same as in the API provider domain resource on CAPIF Core Function;

3.	if the API management function is authorized and the property "apiProvDomId" matches, then the CAPIF core function shall:

a.	replace/modify the representation of the resource identified by the CAPIF Resource URI of the API management function's HTTP PUT or PATCH request with updated information in the request;

b.	update the individual API provider domain function profiles as per the request. CAPIF core function shall create new API provider domain function profiles along with assignment of identities, if the API provider domain functions profiles in the request do not exist in CAPIF; and

c.	return a "200 OK" status code with the updated API provider domain information, or a "204 No Content" status code;

and

4.	if errors occur when processing the request, the CAPIF core function shall respond to the API management function with an appropriate error status code as defined in clause 8.9.5.

# * * * * 2nd Change * * * *

###### 8.9.2.3.3.1	PUT

The PUT method allows updating the registered API provider domain's detail. The properties "apiProvDomId", and "suppFeat" shall remain unchanged from previously provided values. This method shall support the URI query parameters specified in table 8.9.2.3.3.1-1.

Table 8.9.2.3.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in the table 8.9.2.3.3.1-2 and the response data structures and response codes specified in the table 8.9.2.3.3.1-3.

Table 8.9.2.3.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| APIProviderEnrolmentDetails | M | 1 | Updated details of the API provider domain. |

Table 8.9.2.3.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| APIProviderEnrolmentDetails | M | 1 | 200 OK | API provider domain's information updated successfully. Updated details of the API provider domain is part of the APIProviderEnrolmentDetails, which is provided in the response body. The list of successfully updated individual API provider domain functions, registration update specific failure information of failed API provider domain function registration updates, are included in APIProviderEnrolmentDetails which is provided in the response body. |
| n/a |  |  | 204 No Content | API provider domain's information updated successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative CAPIF core function. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [14]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative CAPIF core function. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [14]. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [14] also apply. |  |  |  |  |

Table 8.9.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative CAPIF core function. |

Table 8.9.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative CAPIF core function. |

# * * * * End of Changes * * * *