3GPP TSG-CT Meeting #106	CP-243169

# Madrid, Spain; 9th – 10th December 2024

Source:	CT4 Chair

Title:	Discussion on API Version Update

Agenda item:	21.5

Document for:	DISCUSSION

1. Background

1.1 Problem Description

For all the 3GPP defined 5G APIs, it has been decided since Rel-15 that when there is at least one change agreed on the OpenAPI description of the API, the version of that API shall be updated. This depends on the state of the API (i.e. published or under development), the type of the agreed change (i.e. correction or new feature), and the backward compatibility of the agreed change (i.e. backward compatible or backward incompatible).

There are two main cases where the API version needs to be updated:

- Case-1: A CR directly modifying the API is agreed. For instance, a CR on TS 29.503 which modifies a data type defined for Nudm_sdm API is agreed, then the API version of Nudm_sdm shall be updated.

- Case-2: A CR modifying the re-used data type of the API is agreed. For instance, a CR on TS 29.503 modifies “AccessAndMobilitySubscriptionData” data type, then Nudm_sdm API version shall be updated,. In addition, “AccessAndMobilitySubscriptionData” data type is referenced by Nudr_dr API, therefore the API version of Nudr_dr shall also be updated accordingly.

For case-1, a procedure was defined and has been executed since Rel-15. In this procedure, the rapporteurs of SBI specifications shall collect all CRs against his/her responsible specification which have been agreed during the plenary cycle, and based on the aforementioned criteria to draft a dedicated CR updating the corresponding API version.

For case-2, it is not very obvious to the rapporteurs which CRs impact the APIs defined in his/her responsible specifications. In theory, the rapporteurs of SBI specs should go through all agreed CRs to find out whether there is any CR against other specs will impact his/her responsible specs.

The working procedure and/or 3GPP tools should be improved to guarantee that the versions of the APIs are updated correctly for case-2.

1.2 Potential Solutions

Solution#1: Manual Solution

- The authors of the CRs against SBI specs shall always list all impacted APIs in “other comments” field of the coversheet.

- The rapporteurs of SBI specs locally maintain a list of SBI specs from which his/her responsible specs make data type references

- Rapporteurs shall always check the agreed CRs against specs in the list mentioned in step 2

- Rapporteurs update the version of API based on the checking as mentioned in step 3

Solution#2: Automated Solution

- To develop a tool which provides a summary of the impacted APIs based on the agreed CRs

- Based on the output of the tool, rapporteurs update the API version

A tool is under development, the beta version is already available. The tool can parse the agreed CRs and digest information on the coversheet and produce an excel file containing the list of impacted APIs (see attached excel file as an example).

2. Discussion

2.1 Standard format of “other comments” field on CR coversheet

The aforementioned tool can parse CRs and digest API impact information based on the assumption of a standard format of text in “other comments” field of the CR coversheet, e.g.

This CR introduces backward [compatible/incompatible] [feature/correction] to the following APIs:

TS<Spec_Number>_<API_name>.yaml

TS<Spec_Number>_<API_name>.yaml

…

TS<Spec_Number>_<API_name>.yaml

To make sure all the CRs subject to OpenAPI changes contain the standard format text, the format should be permanent and public available to all delegates. Possible approaches include:

1)	To produce a dedicated WG CR template in addition to existing CR template

2)	To capture the above blue text in the meeting guidance for each meeting

2.2 The working procedure of processing all agreed CRs and providing the impacted API list

The data type references happen not only within one working group among different specifications, but also across different working groups, e.g. the SBI specs under remit of CT3 make a lot of date type references to specs under remit of CT4. Therefore the tool should parse all the SBI related CRs agreed by all relevant working groups together. Then it should be discussed how different working groups coordinate on this issue. Possible approaches include:

1)	The working groups coordinate, the chair/MCC of one working group take the lead, collect all the CRs agreed by other relevant working groups, and run the tool. Once the tool produces a list of impacted APIs, the leading chair/MCC distribute it among all relevant working groups.

2)	Each working group does the work on its own, the chair/MCC of each working group collect all the CRs agreed by other relevant working groups, run the tool, and make use of the output for their own working groups respectively.

3)	Each rapporteur does the work on its own, collect all the CRs agreed by other relevant working groups, run the tool, and make use of the output for the rapporteur itself. This approach requires a way to make the aforementioned tool public available to rapporteurs.

4)	The MCC take the lead of doing above work

3. Proposal

It is proposed to have a discussion on the above topic and reach a consensus, then CT instructs the CT WGs to enforce the conclusion.

Considering this issue also subject to SA4, SA5, CT can discuss whether the relevant information should be conveyed to SA.