3GPP TSG-CT Meeting #106	CP-243245

# Madrid, Spain; 9th – 10th December 2024                             was C4-245334

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.572 | CR | 0300 | rev | 2 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to support NRPPa related measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CATT, Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As agreed in the Stage 2 CR (S2-2410896, TS 23.273 CR#0615), time windows could be referred to a LPP IE or a NRPPa IE (clause 8.3.2.6 of TS 23.273): #### 8.3.2.6 Nlmf_Location_MeasurementData service operation Service operation name: Nlmf_Location_MeasurementData Description: Provides PRU location measurements to the consumer NF. Input, Required: Target UE cell ID. Input, Optional: Pre-calculated location of target UE, time window(s). NOTE y: The time window(s) is the Time Window Information SRS List and the Time Window Information Measurement List specified in TS 38.455[15] when network assisted positioning is used and is the NR-DL-PRS-MeasurementTimeWindowsConfig specified in TS 37.355[20] when UE assisted positioning is used. Output, Required: PRU location measurement(s) and associated PRU known location. Output, Optional: None. See clause 6.17 for an example of usage of this service operation. In current specification, only LPP IE is defined, this CR proposes to add new time windows to support NRPPa related measurements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1/ add new IE for NRPPa measurements and update OpenAPI. 2/ add new referred TS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NRPPa related measurement transfer cannot be supported and stage 2 requirent cannot be fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.1.6.1, 6.1.6.2.45, 6.1.6.2.xx (new), 6.1.6.3, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.273 CR 0615 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible correction to Nlmf_Location API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: 1/ add co-source and new affected clauses. 2/ update the new added data and the OpenAPI. Rev 2: 1/ correct the error in OpenAPI. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	IETF RFC 4776: "Dynamic Host Configuration Protocol (DHCPv4 and DHCPv6) Option for Civic Addresses Configuration Information".

[7]	IETF RFC 5139: "Revised Civic Location Format for Presence Information Data Format Location Object (PIDF-LO)".

[8]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[9]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[10]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[11]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

[12]	IETF RFC 9113: "HTTP/2".

[13]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[14]	OpenAPI Initiative, "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[15]	IETF RFC 9457: "Problem Details for HTTP APIs".

[16]	3GPP TR 21.900: "Technical Specification Group working methods".

[17]	3GPP TS 22.071: "Location Services (LCS); Service description; Stage 1".

[18]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[19]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[20]	3GPP TS 24.080: "Mobile radio interface layer 3 Supplementary services specification; Formats and coding".

[21]	3GPP TS 37.355: " LTE Positioning Protocol (LPP)".

[22]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[23]	3GPP TS 29.518: "Access and Mobility Management Services".

[24]	3GPP TS 29.171: "Location Services (LCS); LCS Application Protocol (LCS-AP) between the Mobile Management Entity (MME) and Evolved Serving Mobile Location Centre (E-SMLC); SLs interface".

[25]	IETF RFC 4119: "A Presence-based GEOPRIV Location Object Format".

[26]	3GPP TS 33.256: "Security aspects of Uncrewed Aerial Systems (UAS)".

[27]	3GPP TS 29.515: "5G System; Gateway Mobile Location Services Stage 3".

[28]	3GPP TS 29.515: "5G System; Gateway Mobile Location Services Stage 3".

[29]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[30]	3GPP TS 38.355: "NR; Sidelink Positioning Protocol (SLPP); Protocol Specification".

[XX]	3GPP TS 38.455: "NR Positioning Protocol A (NRPPa)".

* * * Next Change * * * *

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nlmf_Location service based interface protocol.

Table 6.1.6.1-1: Nlmf_Location specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| InputData | 6.1.6.2.2 | Information within Determine Location Request |
| LocationData | 6.1.6.2.3 | Information within Determine Location Response |
| GeographicalCoordinates | 6.1.6.2.4 | Geographical coordinates |
| GeographicArea | 6.1.6.2.5 | Geographic area specified by different shape |
| Point | 6.1.6.2.6 | Ellipsoid Point |
| PointUncertaintyCircle | 6.1.6.2.7 | Ellipsoid point with uncertainty circle |
| PointUncertaintyEllipse | 6.1.6.2.8 | Ellipsoid point with uncertainty ellipse |
| Polygon | 6.1.6.2.9 | Polygon |
| PointAltitude | 6.1.6.2.10 | Ellipsoid point with altitude |
| PointAltitudeUncertainty | 6.1.6.2.11 | Ellipsoid point with altitude and uncertainty ellipsoid |
| EllipsoidArc | 6.1.6.2.12 | Ellipsoid Arc |
| LocationQoS | 6.1.6.2.13 | QoS of Location request |
| CivicAddress | 6.1.6.2.14 | Indicates a Civic address |
| PositioningMethodAndUsage | 6.1.6.2.15 | Indicates the usage of a positioning method |
| GnssPositioningMethodAndUsage | 6.1.6.2.16 | Indicates the usage of a Global Navigation Satellite System (GNSS) positioning method |
| VelocityEstimate | 6.1.6.2.17 | Velocity estimate |
| HorizontalVelocity | 6.1.6.2.18 | Horizontal velocity |
| HorizontalWithVerticalVelocity | 6.1.6.2.19 | Horizontal and vertical velocity |
| HorizontalVelocityWithUncertainty | 6.1.6.2.20 | Horizontal velocity with speed uncertainty |
| HorizontalWithVerticalVelocityAndUncertainty | 6.1.6.2.21 | Horizontal and vertical velocity with speed uncertainty |
| UncertaintyEllipse | 6.1.6.2.22 | Ellipse with uncertainty |
| UeLcsCapability | 6.1.6.2.23 | Indicates the LCS capability supported by the UE. |
| PeriodicEventInfo | 6.1.6.2.24 | Indicates the information of periodic event reporting |
| AreaEventInfo | 6.1.6.2.25 | Indicates the information of area based event reporting |
| ReportingArea | 6.1.6.2.26 | Indicates an area for event reporting |
| MotionEventInfo | 6.1.6.2.27 | Indicates the information of motion based event reporting |
| CancelLocData | 6.1.6.2.29 | Information within Cancel Location Request |
| LocContextData | 6.1.6.2.30 | Information within Transfer Location Context Request |
| EventReportMessage | 6.1.6.2.31 | Indicates an event report message |
| EventReportingStatus | 6.1.6.2.32 | Indicates the status of event reporting |
| UELocationInfo | 6.1.6.2.33 | Indicates location information of a UE |
| EventNotifyData | 6.1.6.2.34 | Information within Event Notify Request |
| UeConnectivityState | 6.1.6.2.35 | Indicates the connectivity state of a UE |
| LocalOrigin | 6.1.6.2.36 | Indicates a Local origin in a reference system. |
| RelativeCartesianLocation | 6.1.6.2.37 | Relative Cartesian Location |
| Local2dPointUncertaintyEllipse | 6.1.6.2.38 | Local 2D point with uncertainty ellipse |
| Local3dPointUncertaintyEllipsoid | 6.1.6.2.39 | Local 3D point with uncertainty ellipsoid |
| UncertaintyEllipsoid | 6.1.6.2.40 | Ellipsoid with uncertainty |
| LocalArea | 6.1.6.2.41 | Local area specified by different shape |
| UeAreaIndication | 6.1.6.2.42 | UE area Indication |
| MinorLocationQoS | 6.1.6.2.43 | Minor Location QoS |
| MbsrInfo | 6.1.6.2.44 | MBSR Information |
| LocMeasurementReq | 6.1.6.2.45 | Location Measurement Request |
| LocMeasurementResp | 6.1.6.2.46 | Location Measurement Response |
| LocMeasurements | 6.1.6.2.47 | Location Measurements result |
| HighAccuracyGnssMetrics | 6.1.6.2.48 | High Accuracy GNSS Metrics |
| UpNotifyData | 6.1.6.2.49 | UP Notify Data |
| UpSubscription | 6.1.6.2.50 | UP Subscription |
| RelatedUe | 6.1.6.2.51 | Specifies information for related UE for ranging and sidelink positioning |
| UpConfig | 6.1.6.2.52 | UP Config |
| RangeDirection | 6.1.6.2.53 | Represents the distance and direction between two points. |
| 2DRelativeLocation | 6.1.6.2.54 | Represents 2D local co-ordinates with origin corresponding to another known point. |
| 3DRelativeLocation | 6.1.6.2.55 | Represents 3D local co-ordinates with origin corresponding to another known point. |
| AddLocationDatas | 6.1.6.2.56 | Contains one or more LocationData. |
| AddEventNotifyDatas | 6.1.6.2.57 | Contains one or more EventNotifyData. |
| LocationDataExt | 6.1.6.2.58 | Represents location data with one or more corresponding UEs. |
| EventNotifyDataExt | 6.1.6.2.59 | Represents notified data with one or more corresponding UEs, containing one or more EventNotifyData. |
| MappedLocationQoSEps | 6.1.6.2.60 | Mapped Location QoS for EPS |
| AdditionalUeInfo | 6.1.6.2.61 | MBSR UE Information |
| TimeWindowsNrppa | 6.1.6.2.xx | Time windows for scheduling of PRU measurements when network assisted positioning is used. |
| Altitude | 6.1.6.3.2 | Indicates value of altitude |
| Angle | 6.1.6.3.2 | Indicates value of angle |
| Uncertainty | 6.1.6.3.2 | Indicates value of uncertainty |
| Orientation | 6.1.6.3.2 | Indicates value of orientation angle |
| HorizAxesOrientation | 6.1.6.3.2 | Orientation of Horizontal Coordinate System |
| Confidence | 6.1.6.3.2 | Indicates value of confidence |
| Accuracy | 6.1.6.3.2 | Indicates value of accuracy |
| InnerRadius | 6.1.6.3.2 | Indicates value of the inner radius |
| CorrelationID | 6.1.6.3.2 | LCS Correlation ID |
| AgeOfLocationEstimate | 6.1.6.3.2 | Indicates value of the age of the location estimate |
| HorizontalSpeed | 6.1.6.3.2 | Indicates value of horizontal speed |
| VerticalSpeed | 6.1.6.3.2 | Indicates value of vertical speed |
| SpeedUncertainty | 6.1.6.3.2 | Indicates value of speed uncertainty |
| BarometricPressure | 6.1.6.3.2 | Specifies the measured uncompensated atmospheric pressure |
| LcsServiceType | 6.1.6.3.2 | LCS service type |
| LdrReference | 6.1.6.3.2 | LDR Reference |
| LirReference | 6.1.6.3.2 | LIR Reference |
| ReportingAmount | 6.1.6.3.2 | Number of required periodic event reports |
| ReportingInterval | 6.1.6.3.2 | Event reporting periodic interval |
| MinimumInterval | 6.1.6.3.2 | Minimum interval between event reports |
| MaximumInterval | 6.1.6.3.2 | Maximum interval between event reports |
| SamplingInterval | 6.1.6.3.2 | Maximum time interval between consecutive evaluations by a UE of a trigger event |
| ReportingDuration | 6.1.6.3.2 | Maximum duration of event reporting |
| LinearDistance | 6.1.6.3.2 | Minimum straight line distance moved by a UE to trigger a motion event report |
| LMFIdentification | 6.1.6.3.2 | LMF identification |
| EventReportCounter | 6.1.6.3.2 | Number of event reports received from the target UE |
| EventReportDuration | 6.1.6.3.2 | Duration of event reporting |
| UePositioningCapabilities | 6.1.6.3.2 | Indicates the positioning capabilities supported by the UE. |
| TimeWindow | 6.1.6.3.2 | Indicates the Time Window for scheduling of PRU measurements when UE assisted positioning is used. |
| TimeWindowInfoMeasurementList | 6.1.6.3.2 | Contains the Time Window Information Measurement List when network assisted positioning is used. |
| TimeWindowInfoSrsList | 6.1.6.3.2 | Contains the Time Window Information SRS List when network assisted positioning is used. |
| ExternalClientType | 6.1.6.3.3 | Indicates types of External Clients |
| SupportedGADShapes | 6.1.6.3.4 | Indicates supported GAD shapes |
| ResponseTime | 6.1.6.3.5 | Indicates acceptable delay of location request |
| PositioningMethod | 6.1.6.3.6 | Indicates supported positioning methods |
| PositioningMode | 6.1.6.3.7 | Indicates supported modes used for positioning method |
| GnssId | 6.1.6.3.8 | Global Navigation Satellite System (GNSS) ID |
| Usage | 6.1.6.3.9 | Indicates usage made of the location measurement |
| LcsPriority | 6.1.6.3.10 | Indicates priority of the LCS client |
| VelocityRequested | 6.1.6.3.11 | Indicates velocity requirement |
| AccuracyFulfilmentIndicator | 6.1.6.3.12 | Indicates fulfilment of requested accuracy |
| VerticalDirection | 6.1.6.3.13 | Indicates direction of vertical speed |
| LdrType | 6.1.6.3.14 | Indicates LDR types |
| ReportingAreaType | 6.1.6.3.15 | Indicates type of event reporting area |
| OccurrenceInfo | 6.1.6.3.16 | Specifies occurrence of event reporting |
| ReportingAccessType | 6.1.6.3.17 | Specifies access types of event reporting |
| EventClass | 6.1.6.3.18 | Specifies event classes |
| ReportedEventType | 6.1.6.3.19 | Specifies type of event reporting |
| TerminationCause | 6.1.6.3.20 | Specifies causes of event reporting termination |
| LcsQosClass | 6.1.6.3.21 | Specifies LCS QoS class |
| UeLocationServiceInd | 6.1.6.3.22 | Specifies location service types requested by UE |
| IndoorOutdoorInd | 6.1.6.3.23 | Indoor Outdoor Indication |
| FixType | 6.1.6.3.24 | Fix Type |
| LosNlosMeasureInd | 6.1.6.3.25 | LOS/NLOS measurement indication |
| UpConnectionStatus | 6.1.6.3.26 | UP Connection Status |
| RangingSlResult | 6.1.6.3.27 | Specifies result type for ranging and sidelink positioning |
| RelatedUeType | 6.1.6.3.28 | Specifies type of related UE for ranging and sidelink positioning |
| LcsUpConnectionInd | 6.1.6.3.29 | LCS UP Connection Indication |
| UeUpPositioningCapabilities | 6.1.6.3.30 | Indicates the user plane positioning capabilities supported by the UE. |

Table 6.1.6.1-2 specifies data types re-used by the Nlmf_Location service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nlmf_Location service based interface.

Table 6.1.6.1-2: Nlmf_Location re-used Data Types

| Data type | Reference | Comments |  |
| --- | --- | --- | --- |
| Supi | 3GPP TS 29.571 [8] | Subscription Permanent Identifier |  |
| Pei | 3GPP TS 29.571 [8] | Permanent Equipment Identifier |  |
| Gpsi | 3GPP TS 29.571 [8] | Generic Public Subscription Identifier |  |
| Ecgi | 3GPP TS 29.571 [8] | E-UTRA Cell Identity |  |
| Ncgi | 3GPP TS 29.571 [8] | NR Cell Identity |  |
| NfInstanceId | 3GPP TS 29.571 [8] | Network Function Instance ID |  |
| Uri | 3GPP TS 29.571 [8] | Uniform Resource Identifier |  |
| RefToBinaryData | 3GPP TS 29.571 [8] | Reference to binary data |  |
| AccessType | 3GPP TS 29.571 [8] | Access type |  |
| CmState | 3GPP TS 29.518 [23] | Connection Management State |  |
| Guami | 3GPP TS 29.571 [8] | GUAMI |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Supported Features |  |
| RedirectResponse | 3GPP TS 29.571 [8] | Redirect Response |  |
| TwapId | 3GPP TS 29.571 [8] | TWAP identifier |  |
| TnapId | 3GPP TS 29.571 [8] | TNAP identifier |  |
| DateTime | 3GPP TS 29.571 [8] | Date and Time |  |
| DurationSec | 3GPP TS 29.571 [8] |  | Duration Second |
| LpHapType | 3GPP TS 29.518 [23] |  | Type of Low Power and/or High Accuracy Positioning |
| ReportingInd | 3GPP TS 29.515 [27] |  | Reporting indication |
| IntegrityRequirements | 3GPP TS 29.515 [27] |  | Integrity Requirements |
| UpLocRepAddrAfRm | 3GPP TS 29.122 [29] |  | Endpoint address for location reporting over user plane |
| UpCumEvtRptCriteria | 3GPP TS 29.515 [27] |  | Criteria for sending cumulative events reports over control plane |
| IntegrityResult | 3GPP TS 29.515 [27] |  | Integrity Result |

* * * Next Change * * * *

##### 6.1.6.2.45	Type: LocMeasurementReq

Table 6.1.6.2.45-1: Definition of type LocMeasurementReq

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ecgi | Ecgi | O | 0..1 | E-UTRA Cell Identity (NOTE X) |
| ncgi | Ncgi | O | 0..1 | NR Cell Identity (NOTE X) |
| preCalcuLocEstimate | GeographicArea | O | 0..1 | Pre-calculated location estimate of target UE |
| timestampOfPreCalcuLocEstimate | DateTime | O | 0..1 | Timestamp (in UTC) of pre-calculated location estimate of target UE |
| timeWindows | array(TimeWindow) | C | 1..N | Time windows for scheduling of PRU measurements when UE assisted positioning is used. (NOTE Y) |
| timeWindowsNrppa | TimeWindowsNrppa | C | 0..1 | Time windows for scheduling of PRU measurements when network assisted positioning is used. (NOTE Y) |
| NOTE X: Either the "ecgi" attribute or the "ncgi" attribute shall be included. NOTE Y: Either the "timeWindows" attribute or the "timeWindowsNrppa" attribute shall be included. |  |  |  |  |

* * * Next Change * * * *

##### 6.1.6.2.xx	Type: TimeWindowsNrppa

Table 6.1.6.2.xx-1: Definition of type TimeWindowsNrppa

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| measurementList | TimeWindowInfoMeasurementList | O | 0..1 | Contains the Time Window Information Measurement List. (NOTE) |  |
| srsList | TimeWindowInfoSrsList | O | 0..1 | Contains the Time Window Information SRS List. (NOTE) |  |
| NOTE: At least one of attributes measurementList, srsList shall be present. |  |  |  |  |  |

* * * Next Change * * * *

##### 6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| Altitude | number | Double-precision float value of the altitude, expressed in meters. Minimum: -32767. Maximum: 32767. Format: double. |
| Angle | integer | Integer value of the angle, expressed in degrees. Minimum: 0. Maximum: 360. |
| Uncertainty | number | Float value of uncertainty, expressed in meters. Minimum: 0 Format: float. |
| Orientation | integer | Integer value of the orientation angle, expressed in degrees. Minimum: 0. Maximum: 180. |
| HorizAxesOrientation | integer | Integer value of the "y" axis orientation angle clockwise from northing, expressed in 0.1 degrees. Minimum: 0. Maximum: 3600. |
| Confidence | integer | Integer value of the confidence, expressed in percentage value. Minimum: 0. Maximum: 100. |
| Accuracy | number | Float value of accuracy, expressed in meters. Minimum: 0 Format: float. |
| InnerRadius | integer | Integer value of the inner radius, expressed in meters. Minimum: 0. Maximum: 327675. Format: int32. |
| CorrelationID | string | LCS Correlation ID. The correlation ID shall be of a minimum length of 1 character and maximum length of 255 characters. |
| AgeOfLocationEstimate | integer | Integer value of the age of the location estimate, expressed in minutes. Minimum: 0. Maximum: 32767. |
| HorizontalSpeed | number | Float value of horizontal speed, expressed in kilometres per hour. Minimum: 0. Maximum: 2047. Format: float. |
| VerticalSpeed | number | Float value of horizontal speed, expressed in kilometres per hour. Minimum: 0. Maximum: 255. Format: float. |
| SpeedUncertainty | number | Float value of speed uncertainty, expressed in kilometres per hour. Minimum: 0. Maximum: 255. Format: float. |
| BarometricPressure | integer | This IE specifies the measured uncompensated atmospheric pressure in units of Pascal (Pa). Minimum: 30000. Maximum: 115000. |
| LcsServiceType | integer | The LCS service type as defined in 3GPP TS 22.071 [17] and clause 17.7.8 of 3GPP TS 29.002 [18]. Minimum: 0. Maximum: 127. |
| LdrReference | string | LDR Reference encoded as a string of hexadecimal characters. The LdrReference shall be of a minimum length of 2 characters and maximum length of 510 characters. |
| LirReference | string | LIR Reference encoded as a string of hexadecimal characters. The LdrReference shall be of a minimum length of 2 characters and maximum length of 510 characters. |
| ReportingAmount | integer | Number of required periodic event reports. Minimum: 1. Maximum: 8639999. |
| ReportingInterval | integer | Event reporting periodic interval in seconds. Minimum: 1. Maximum: 8639999. ReportingInterval x ReportingAmount shall not exceed 8639999. |
| ReportingIntervalMs | integer | Event reporting periodic interval in milliseconds. Minimum: 1. Maximum: 999. |
| MinimumInterval | integer | Minimum interval between event reports in seconds. Minimum: 1. Maximum: 32767. |
| MaximumInterval | integer | Maximum interval between event reports in seconds. Minimum: 1. Maximum: 86400. |
| SamplingInterval | integer | Maximum time interval between consecutive evaluations by a UE of a trigger event, in seconds. Minimum: 1. Maximum: 3600 |
| ReportingDuration | integer | Maximum duration of event reporting, in seconds. Minimum: 1. Maximum: 8640000. |
| LinearDistance | integer | The minimum straight line distance moved by a UE to trigger a motion event report, in meters. Minimum: 1. Maximum: 10000. |
| LMFIdentification | string | The serving LMF identification as defined in 3GPP TS 23.273 [19], encoded as a string of hexadecimal characters. |
| EventReportCounter | integer | Number of event reports received from the target UE. Minimum: 1. Maximum: 8640000. Note: the current event report is included in the count. |
| EventReportDuration | integer | Duration of event reporting, in seconds. Minimum: 0. Maximum: 8640000. Note: the duration starts when event reporting is activated in the UE and extends to the current time. |
| UePositioningCapabilities | Bytes | Positioning capabilities supported by the UE. String with format "byte" as defined in OpenAPI Initiative [14], i.e. base64-encoded characters, encoding the "ProvideCapabilities-r9-IEs" IE as specified in clause 6.3 of 3GPP TS 37.355 [21] (start from octet 1). |
| TimeWindow | Bytes | Time window of the target UE when UE assisted positioning is used. String with format "byte" as defined in OpenAPI Initiative [14], i.e. base64-encoded characters, encoding the "NR-DL-PRS-MeasurementTimeWindowsConfig-r18" IE as specified in clause 6.4.3 of 3GPP TS 37.355 [21] (start from octet 1). |
| TimeWindowInfoMeasurementList | Bytes | Contains the Time Window Information Measurement List when network assisted positioning is used. String with format "byte" as defined in OpenAPI Initiative [14], i.e. base64-encoded characters, encoding the "Time Window Information Measurement List" IE as specified in clause 9.2.91 of 3GPP TS 38.455 [XX] (start from octet 1). |
| TimeWindowInfoSrsList | Bytes | Contains the Time Window Information SRS List when network assisted positioning is used. String with format "byte" as defined in OpenAPI Initiative [14], i.e. base64-encoded characters, encoding the "Time Window Information SRS List" IE as specified in clause 9.2.90 of 3GPP TS 38.455 [XX] (start from octet 1). |
| RangingSlCapability | Bytes | Ranging/Sidelink Positioning Capability supported by the UE. String with format "byte" as defined in OpenAPI Initiative [14], i.e. base64-encoded characters, encoding the "ProvideCapabilities" IE as specified in clause 6.2.2 of 3GPP TS 38.355 [30] (start from octet 1). |

* * * Next Change * * * *

# A.2	Nlmf_Location API

openapi: 3.0.0

info:

version: '1.3.0'

title: 'LMF Location'

description: |

LMF Location Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

***Text Skipped for Clarity***

LocMeasurementReq:

description: Location Measurement Request.

type: object

properties:

ncgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

ecgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

preCalcuLocEstimate:

$ref: '#/components/schemas/GeographicArea'

timestampOfPreCalcuLocEstimate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

timeWindows:

type: array

items:

$ref: '#/components/schemas/TimeWindow'

minItems: 1

timeWindowsNrppa:

$ref: '#/components/schemas/TimeWindowsNrppa'

LocMeasurementResp:

description: Location Measurement Response.

type: object

required:

- locMeasurements

properties:

locMeasurements:

type: array

items:

$ref: '#/components/schemas/LocMeasurements'

minItems: 1

LocMeasurements:

description: PRU Location Measurements.

type: object

properties:

locInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

HighAccuracyGnssMetrics:

description: High Accuracy GNSS Positioning Metrics.

type: object

properties:

nrOfUsedSatellites:

type: integer

minimum: 0

maximum: 64

hdopi:

type: integer

minimum: 1

maximum: 256

pdopi:

type: integer

minimum: 1

maximum: 256

age:

type: integer

minimum: 0

maximum: 99

fixType:

$ref: '#/components/schemas/FixType'

UpNotifyData:

description: UP Subscription

type: object

required:

- notifCorrelationId

- upConnectionStatus

properties:

notifCorrelationId:

$ref: '#/components/schemas/CorrelationID'

upConnectionStatus:

$ref: '#/components/schemas/UpConnectionStatus'

targetLMFId:

$ref: '#/components/schemas/LMFIdentification'

UpSubscription:

description: UP Subscription

type: object

required:

- upNotifyCallBackUri

- notifCorrelationId

- supi

properties:

upNotifyCallBackUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifCorrelationId:

$ref: '#/components/schemas/CorrelationID'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

RelatedUe:

description: Related UE Information

type: object

required:

- applicationlayerId

- relatedUeType

properties:

applicationlayerId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationlayerId'

relatedUeType:

$ref: '#/components/schemas/RelatedUeType'

UpConfig:

description: UP Config

type: object

required:

- upNotifyCallBackUri

- notifCorrelationId

anyOf:

- required: [supi]

- required: [gpsi]

properties:

upNotifyCallBackUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifCorrelationId:

$ref: '#/components/schemas/CorrelationID'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

amfReallocationInd:

type: boolean

default: false

lcsUpConnectionInd:

$ref: '#/components/schemas/LcsUpConnectionInd'

targetLMFId:

$ref: '#/components/schemas/LMFIdentification'

RangeDirection:

description: Represents a distance and direction from a point A to a point B.

type: object

properties:

distance:

type: number

azimuthDirection:

$ref: '#/components/schemas/Angle'

elevationDirection:

$ref: '#/components/schemas/Angle'

2DRelativeLocation:

description: Represents a relative 2D location with uncertainty ellipse.

type: object

properties:

semiMinor:

$ref: '#/components/schemas/Uncertainty'

semiMajor:

$ref: '#/components/schemas/Uncertainty'

orientationAngle:

$ref: '#/components/schemas/Angle'

3DRelativeLocation:

description: Represents a relative 3D location with uncertainty ellipsoid.

type: object

properties:

semiMinor:

$ref: '#/components/schemas/Uncertainty'

semiMajor:

$ref: '#/components/schemas/Uncertainty'

verticalUncertainty:

$ref: '#/components/schemas/Uncertainty'

orientationAngle:

$ref: '#/components/schemas/Angle'

AddLocationDatas:

description: Additional Location Data.

type: object

properties:

addLocationDatas:

type: array

items:

$ref: '#/components/schemas/LocationData'

minItems: 1

AddEventNotifyDatas:

description: Additional Event Notify Data.

type: object

properties:

addEventNotifyDatas:

type: array

items:

$ref: '#/components/schemas/EventNotifyData'

minItems: 1

MappedLocationQoSEps:

description: Mapped Location QoS for EPS.

type: object

required:

- hAccuracy

properties:

hAccuracy:

$ref: '#/components/schemas/Accuracy'

vAccuracy:

$ref: '#/components/schemas/Accuracy'

AdditionalUeInfo:

description: MBSR UE Information

type: object

properties:

ncgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

ecgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

TimeWindowsNrppa:

description: Time windows when network assisted positioning is used.

type: object

properties:

measurementList:

$ref: '#/components/schemas/TimeWindowInfoMeasurementList'

srsList:

$ref: '#/components/schemas/TimeWindowInfoSrsList'

anyOf:

- required: [measurementList]

- required: [srsList]

#

#

# SIMPLE TYPES

#

Altitude:

description: Indicates value of altitude.

type: number

format: double

minimum: -32767

maximum: 32767

Angle:

description: Indicates value of angle.

type: integer

minimum: 0

maximum: 360

Uncertainty:

description: Indicates value of uncertainty.

type: number

format: float

minimum: 0

Orientation:

description: Indicates value of orientation angle.

type: integer

minimum: 0

maximum: 180

HorizAxesOrientation:

description: Horizontal axes orientation angle clockwise from northing in 0.1 degrees.

type: integer

minimum: 0

maximum: 3600

Confidence:

description: Indicates value of confidence.

type: integer

minimum: 0

maximum: 100

Accuracy:

description: Indicates value of accuracy.

type: number

format: float

minimum: 0

InnerRadius:

description: Indicates value of the inner radius.

type: integer

format: int32

minimum: 0

maximum: 327675

CorrelationID:

description: LCS Correlation ID.

type: string

minLength: 1

maxLength: 255

AgeOfLocationEstimate:

description: Indicates value of the age of the location estimate.

type: integer

minimum: 0

maximum: 32767

HorizontalSpeed:

description: Indicates value of horizontal speed.

type: number

format: float

minimum: 0

maximum: 2047

VerticalSpeed:

description: Indicates value of vertical speed.

type: number

format: float

minimum: 0

maximum: 255

SpeedUncertainty:

description: Indicates value of speed uncertainty.

type: number

format: float

minimum: 0

maximum: 255

BarometricPressure:

description: Specifies the measured uncompensated atmospheric pressure.

type: integer

minimum: 30000

maximum: 115000

LcsServiceType:

description: LCS service type.

type: integer

minimum: 0

maximum: 127

LdrReference:

description: LDR Reference.

type: string

minLength: 2

maxLength: 510

LirReference:

description: LIR Reference.

type: string

minLength: 2

maxLength: 510

ReportingAmount:

description: Number of required periodic event reports.

type: integer

minimum: 1

maximum: 8639999

ReportingInterval:

description: Event reporting periodic interval in seconds.

type: integer

minimum: 1

maximum: 8639999

ReportingIntervalMs:

description: Event reporting periodic interval in milliseconds.

type: integer

minimum: 1

maximum: 999

MinimumInterval:

description: Minimum interval between event reports.

type: integer

minimum: 1

maximum: 32767

MaximumInterval:

description: Maximum interval between event reports.

type: integer

minimum: 1

maximum: 86400

SamplingInterval:

description: Maximum time interval between consecutive evaluations by a UE of a trigger event.

type: integer

minimum: 1

maximum: 3600

ReportingDuration:

description: Maximum duration of event reporting.

type: integer

minimum: 1

maximum: 8640000

LinearDistance:

description: Minimum straight line distance moved by a UE to trigger a motion event report.

type: integer

minimum: 1

maximum: 10000

LMFIdentification:

description: LMF identification.

type: string

EventReportCounter:

description: Number of event reports received from the target UE.

type: integer

minimum: 1

maximum: 8640000

EventReportDuration:

description: Duration of event reporting.

type: integer

minimum: 1

maximum: 8640000

UePositioningCapabilities:

description: >

Positioning capabilities supported by the UE. A string encoding the

"ProvideCapabilities-r9-IEs" IE as specified in clause 6.3 of 3GPP

TS 37.355 (start from octet 1).

type: string

format: byte

TimeWindow:

description: Time Window when UE assisted positioning is used.

type: string

format: byte

TimeWindowInfoMeasurementList:

description: Contains the Time Window Information Measurement List.

type: string

format: byte

TimeWindowInfoSrsList:

description: Contains the Time Window Information SRS List.

type: string

format: byte

RangingSlCapability:

description: Ranging/Sidelink Positioning Positioning capabilities supported by the UE.

type: string

format: byte

***Text Skipped for Clarity***

* * * End of Changes * * * *