3GPP TSG-CT WG1 Meeting #152	C1-246725

# Orlando, US, 18-22 November 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.571 | CR | 0101 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to missing RELEASE COMPLETE for UE initiated location service operations. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GProtoc18, 5G_eLCS |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current specifications, in many procedures and figuers it is shown that the UE can send a RELEASE COMPLETE message, it is not specified when or how this RELEASE COMPLETE message is sent. This creates confusion in both UE and the NW and can end up in wrong state as the procedure can be considered ongoing for a long period of time. This needs to be fixed from Rel-18 otherwise it can cause confusion to implementors. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the condition when to send RELEASE COMPLETE in several procedures. Backward compatability anaylsis:- The changes are backward compatable. The procedure was incomplete and the conditions to send the message is added which was missing before. An older version of the NW may ignore the message. FASMO analysis:- This issue is FASMO as it is uncertain both from the network side and the UE send on when to send a message and when to expect a messag and also when the procedure is ending. This can cause both the UE and NW to be in a wrong state as the procedure can be considered ongoing for a long period of time. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusing specification which can lead to misoperation at both UE and NW side. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.2, 5.2.2.242, 5.2.2.5.2, 5.2.2.11.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Change between rev0 and rev1:- Corrected the spec number in cover sheet |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 5.2.2.2.2	Normal operation

The UE invokes a cancel deferred location procedure by sending a REGISTER message containing a LCS-MSCancelDeferredLocation invoke component to the LMF as defined in 3GPP TS 24.080 [5].

The LMF shall terminate the ongoing periodic or triggered location if this can be identified from the information in the LCS-MSCancelDeferredLocation invoke component.

The LMF shall then return a RELEASE COMPLETE message containing an LCS-MSCancelDeferredLocation return result component (see Figure 5.2.2.2.2-1).

NOTE:	If the UE has requested the cancellation of the deferred location procedure for the location events reporting over user plane connection and the UE receives the acknowledgment of the cancellation from the LMF, the UE releases the user plane connection to the LCS Client or AF if the release has not been yet initiated.

If the LMF is unable to process the request received from the UE or can not identify the ongoing periodic or triggered location to be terminated, it shall return an error indication by sending a RELEASE COMPLETE message containing a return error component. Error values are specified in 3GPP TS 24.080 [5].

During the UE initiated cancel deferred location operation, the UE shall run a timer T. This timer is started when the operation is sent, and stopped when a response is received from the network. If this timer expires the UE shall assume that the operation has failed, and may terminate the dialogue by sending a RELEASE COMPLETE message, and may inform the user of the failure.

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Facility (Invoke = LCS-MSCancelDeferredLocation (referenceNumberExt, h-gmlc-callBackUri))

RELEASE COMPLETE

<------------------------------------------------------------------------------------------------------------------------

Facility (Return result = LCS-MSCancelDeferredLocation)

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility (Reject (Invoke_problem))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Figure 5.2.2.2.2-1: Cancel Deferred Location

NOTE:	Only the following IEs defined in MSCancelDeferredLocation operations in 3GPP TS 24.080 [5] are used for NG-RAN LCS:

-	referenceNumberExt

-	h-gmlc-callBackUri

* * * Next Change * * * *

##### 5.2.2.4.2	Normal operation

The UE invokes an EventReport procedure by sending a REGISTER message containing an EventReport component to the LMF as defined in 3GPP TS 24.080 [5].

The LMF shall return a supplementary services acknowledgement of event report if it can handle this event report.

The LMF shall then return a RELEASE COMPLETE message containing an EventReport Acknowledge component (see Figure 5.2.2.4.2-1).

If the LMF is unable to process the request received from the UE or cannot identify the ongoing periodic or triggered location to be terminated, it shall return an error indication by sending a RELEASE COMPLETE message containing a return error component. Error values are specified in 3GPP TS 24.080 [5].

During the UE initiated event reporting procedure operation, the UE shall run a timer T. This timer is started when the operation is sent, and stopped when a response is received from the network. If this timer expires the UE shall assume that the operation has failed, and may terminate the dialogue by sending a RELEASE COMPLETE message, and may inform the user of the failure.

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Facility (Invoke = LCS-EventReport (eventType, referenceNumberExt, h-gmlc-callBackUri, lcs-QoS, locationInfo, supportedGADShapes, multiplePositioningProtocolPDUs, terminationCause, cumulativeEventReport))

RELEASE COMPLETE

<------------------------------------------------------------------------------------------------------------------------

Facility (Return result = LCS-EventReport Acknowledgement)

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility (Reject (Invoke_problem))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Figure 5.2.2.4.2-1: EventReport

NOTE:	Only the following identifiers defined in EventReport operations in 3GPP TS 24.080 [5] are used for NG-RAN LCS:

-	eventType

-	referenceNumberExt

-	h-gmlc-callBackUri

-	lcs-QoS

-	locationInfo

-	supportedGADShapes

-	multiplePositioningProtocolPDUs

-	terminationCause

-	cumulativeEventReport

* * * Next Change * * * *

##### 5.2.2.5.2	Normal operation

The UE invokes a Location Privacy Setting procedure by sending a REGISTER message containing an LocationPrivacySetting message as defined in 3GPP TS 24.080 [5]. UE's Location Privacy Indication information is included in LocationPrivacySetting message.

The AMF shall transfer the UE Location Privacy Indication information in LocationPrivacySetting message to UDM and response a supplementary services acknowledgement of LocationPrivacySetting message.

The AMF shall then return a RELEASE COMPLETE message containing a LocationPrivacySetting Acknowledgement component (see figure 5.2.2.5.2-1) if it can handle this LocationPrivacySetting message correctly.

If the AMF is unable to process the request received from the UE or cannot identify that provisioning the parameters of UE Location Privacy Setting information to UDM is executed successfully, it shall return an error indication by sending a RELEASE COMPLETE message containing a return error component. Error values are specified in 3GPP TS 24.080 [5].

During the UE initiated location privacy setting procedure operation, the UE shall run a timer T. This timer is started when the operation is sent, and stopped when a response is received from the network. If this timer expires the UE shall assume that the operation has failed, and may terminate the dialogue by sending a RELEASE COMPLETE message, and may inform the user of the failure.

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Facility (Invoke = LCS-LocationPrivacySetting (locationPrivacyIndication, validTimePeriod, eventReportExpectedArea, areaUsageIndication)

RELEASE COMPLETE

<------------------------------------------------------------------------------------------------------------------------

Facility (Return result = LCS-LocationPrivacySetting Acknowledgement)

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility (Reject (Invoke_problem))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Figure 5.2.2.5.2-1: LocationPrivacySetting

NOTE:	Only the following identifiers defined in LocationPrivacySetting operations in 3GPP TS 24.080 [5] are used for NG-RAN LCS:

-	locationPrivacyIndication

-	validTimePeriod

-	eventReportExpectedArea

-	areaUsageIndication

* * * Next Change * * * *

##### 5.2.2.11.2	Normal operation

In order to initate the UE initiated RSPP supplementary information transport procedure, the initiating UE shall send a REGISTER message to the LMF containing the ULRSPPTransport invoke component as defined in 3GPP TS 24.080 [5].

In the ULRSPPTransport invoke component included in the REGISTER message, the UE shall include rangingSLPPList IE. In the rangingSLPPList, the UE:

a)	may include the embedded SLPP message as defined in 3GPP TS 38.355 [4a] for the initiating UE; and

b)	shall include the embedded SLPP message(s) as defined in 3GPP TS 38.355 [4a] for the related SL reference UE(s) or the related located UE(s) with the associated application layer ID(s).

NOTE:	The SLPP message(s) sent by the initiating UE include the SLPP message(s) to provide capability, to request assistance data or to provide location information as defined in 3GPP TS 38.355 [4a].

The LMF shall then return a RELEASE COMPLETE message containing an ULRSPPTransport return result component (see Figure 5.2.2.11.2-1) if the LMF accepts the ULRSPPTransport invoke component.

If the LMF does not accept the ULRSPPTransport invoke component, it shall return an error indication by sending a RELEASE COMPLETE message containing a return error component. Error values are specified in 3GPP TS 24.080 [5].

During the UE initiated RSPP supplementary information transport operation, the UE shall run a timer T. This timer is started when the operation is sent, and stopped when a response is received from the network. If this timer expires the UE shall assume that the operation has failed, and may terminate the dialogue by sending a RELEASE COMPLETE message, and may inform the user of the failure.

Figure 5.2.2.11.2.1 illustrates the signalling for normal operation between the UE and the network.

UE	Network

REGISTER

------------------------------------------------------------------------------------------------------------------------>

Facility (Invoke = LCS-ULRSPPTransport (rangingSLPPList))

RELEASE COMPLETE

<------------------------------------------------------------------------------------------------------------------------

Facility (Return result = LCS-ULRSPPTransport())

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility (Return error (Error))

RELEASE COMPLETE

<-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -

Facility (Reject (Invoke_problem))

RELEASE COMPLETE

-  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  ->

Figure 5.2.2.11.2-1: UE initiated RSPP supplementary information transport

NOTE 1: Only the following IEs defined in ULRSPPTransport operations in 3GPP TS 24.080 [5] are used for ranging and sidelink positioning:

-	rangingSLPPList

NOTE 2:	The rangingSLPPList IE is added to ULRSPPTransport to allow for passing multiple SLPP messages for the UE connecting with the LMF and/or the other related UE. Its ASN.1 description is given in 3GPP TS 24.080 [5].

* * * End of Change * * * *