3GPP TSG-RAN WG Meeting #106	                                          RP-242787

Madrid, Spain, 9 – 12, Dec, 2024

Source: 	Xiaomi

Title:  	Views on R20 AI/ML for Mobility

Agenda Item:	15.2

Document for:	Discussion and Decision

Introduction

AI/ML for mobility is studied by RAN2 in R19. In this contribution, we discuss the scope of R20 AI/ML for mobility.

Discussion

The R19 SI includes following use cases,

- AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2]

Regarding RRM prediction, RAN2 has done lots of simulation results on prediction accuracy. The simulation results showed the prediction can be close to the ground truth in some cases. The RSRP difference can be very small. RAN2 is going to further simulate the generalization and system level performance. We believe the performance gain of RRM prediction can be expected. Therefore, RRM prediction can be included in the R20 WI. Furthermore, the RRM prediction is categorized into following cases,

| Case number | Evaluation scenario combination | target study goal |
| --- | --- | --- |
| 1 | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal |
| 2 | FR1 to FR1 intra-frequency temporal domain case B | 1st goal |
| 3 | FR1 to FR1 inter-frequency (frequency domain) | 1st goal |
| 4 | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal |
| 5 | FR2 to FR2 intra-frequency temporal domain case B | 1st goal |
| 6 | FR2 to FR2 intra-frequency spatial domain | 1st goal |

Simulation is focused on temporal and frequency domain prediction i.e. case 2, 3, 4. RAN2 didn’t discuss much on spatial domain prediction, due to lack of simulation results. Therefore, the RRM prediction on temporal and frequency domain should be prioritized over spatial domain.

Proposal 1: RRM prediction on temporal and frequency domain is included in R20 WI.

Event and RLF prediction can be done in direct or indirect way. RAN2 has agreed to prioritize indirect prediction and the related simulation assumptions. Simulation results are expected to be submitted in the following meetings. Event and RLF prediction can be useful to improve the mobility performance. If the simulation results can show acceptable prediction accuracy, performance gain can be expected. Therefore, event and RLF prediction can be included in R20 WI.

Proposal 2: Event and RLF prediction can be included in the R20 WI, if simulation results show acceptable prediction accuracy and performance gain.

Proposal 3: Only indirect prediction is supported for event and RLF prediction.

HOF prediction is deprioritized by RAN2. No simulation is done so far. Due to the heavy load, RAN2 may not be able to do any simulation regarding HOF prediction. Therefore, HOF prediction may not be included in the R20 WI. However, we understand HOF is a key performance KPI in mobility. HOF prediction may be useful. RAN2 can study the HOF prediction if possible.

Proposal 4: Further study on HOF prediction can be considered in R20.

The work is mainly on RAN2. Besides RAN2, RAN4 should also be involved to specify the necessary RRM/test requirement.

Conclusions

According to the discussion above, we have the following proposals:

Proposal 1: RRM prediction on temporal and frequency domain is included in R20 WI.

Proposal 2: Event and RLF prediction can be included in the R20 WI, if simulation results show acceptable prediction accuracy and performance gain.

Proposal 3: Only indirect prediction is supported for event and RLF prediction.

Proposal 4: Further study on HOF prediction can be considered in R20.

Reference

[1] RP-242393