3GPP TSG RAN Meeting #106								RP-242799

Madrid, Spain, December 9-12, 2024

Agenda Item:	10.4.5

Source:	OPPO

Title:	Discussion on WID Scope for IoT NTN TDD mode

Document for:	Discussion & Decision

Introduction

In this paper, we discuss the checkpoint status and some needed clarifications identified during the WG discussion on this WI. Finally, we provide some suggestions to address the expected WID update.

Discussion

Checkpoint for DL synchronization and other aspects

In RAN#105 meeting, RAN plenary assigned the task to RAN1 to evaluate the DL synchronization and other aspects for periodic pattern. The evaluation outcomes are summarized below:

Observation

Based on simulations submitted to this meeting, in terms of downlink synchronization (NPSS/NSSS/NPBCH), all the following cases meet the link budget requirements for IOT-NTN TDD and are feasible (from the downlink synchronization point of view) from RAN1 perspective:

- Case 1: N=9, D=8

- Case 2: N=9, D=20

- Case 3: N=9, D=30

Observation

In Rel-19, RAN1 concludes that at least N=9 is feasible from the following points of view:

- DL synchronization, based on the link level evaluations submitted at RAN#119

- Coexistence with the TDD frame structure of the legacy system in the 1616-1626.5 MHz MSS band

Observation

RAN1 concludes that at least D=8 and U=8 is feasible with N=9 from point of view of the design constraints (from earlier agreement) imposed by the TDD frame structure of the legacy system in the 1616-1626.5 MHz MSS band.

Conclusion

For N=9, D=8, RAN1 concludes that SIB1-NB reception following current specifications is feasible for some SIB1-NB TBS values, at least when the configured number of repetitions is 16 and when SIB1-NB TBS value <=X bits.

- When considering only NPSS/NSSS/NPBCH/SIB1-NB, and if the number of SIB1-NB repetitions is 16:

  - If additionalTransmissionSIB1 is set to TRUE, the overhead of NPSS/NSSS/NPBCH/SIB1-NB is 43.75% in the anchor carrier.

  - If additionalTransmissionSIB1 is not set to TRUE, the overhead of NPSS/NSSS/NPBCH/SIB1-NB is 37.5% in the anchor carrier.

  - NOTE: In the above calculations, overhead due to transmission of other SIBs has not been included.

- The value of X may be further studied during the normative phase. Note: X is a TBS value supported by the specifications for SIB1-NB.

Based on the RAN1 observations and conclusions, the feasibility of periodic pattern in 1616-1626.5 MHz MSS band can be confirmed and the normative work can start from Feb’2025.

Proposal 1: The feasibility of periodic pattern in 1616-1626.5 MHz MSS band can be confirmed and the normative work can start from Feb’2025.

Target use case and design restrictions clarification

During the WG discussions, there are two different understandings on the necessity of the value of N as well as the motivation of this WI. In one part, in the WID justification part (cited below), it seems to say that the new feature of using periodic subset of UL and DL subframes in N radio frames is intended for power consumption. Thus, the value of N can be made more flexible to tailored to existing 3GPP system, as long as the power consumption gain is still there.

This Work Item Description proposes the introduction of a new feature that allows the operator to use the radio resources in a periodic subset of the UL and DL subframes in N radio frames to achieve TDD operation in the SAN (Satellite Access Node) and IoT NTN UE, thus limiting power consumption. This feature allows extending 3GPP NB-IoT NTN operation with support for additional NGSO satellite system. The new feature will extend the deployment of NB-IoT NTN, by extending support to additional existing, in-orbit satellite resources, in particular additional existing Non-Geostationary Satellite Orbit (NGSO) systems, and including enabling additional lower-complexity satellite payloads, such as implementations without diplexer. With this proposed feature, there is a significant opportunity to further expand global NB-IoT NTN service coverage. This includes providing connectivity to polar regions. This new feature should be designed leveraging commonalities with the NB-IoT FDD NTN operation, for NGSO operating in 1616-1626.5MHz, assuming this band will be defined as unpaired spectrum in 3GPP. This will define a TDD mode for NB-IoT NTN systems. The feature is not intended to be applicable to existing 3GPP bands.

On the other side, the understanding is that the new feature is to be used to co-exist with existing Iridium service, where there are restrictions in practice so that the IoT NTN TDD design in this WI should stick to N=9. However, even in the case of N=9, there are still different understandings about how to treat the legacy system restrictions. In RAN1#119 meeting, Iridium clarified the restrictions for the legacy system, i.e.,

Agreement

Regarding operation within the same band as the TDD frame structure of legacy system in the 1.6GHz MSS band (shown below), RAN1 work in Rel-19 considers the following design constraints:

- At the satellite, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods.

  - The same downlink slot is used in all the 90ms periods.

- At the satellite, all uplink NB-IoT channels in a cell can only use one of the uplink slots in the TDD frame structure (UL1, UL2, UL3 or UL4) across 90ms periods.

  - The same uplink slot is used in all the 90ms periods.

- The one uplink slot and one downlink slot in the TDD frame structure have the same index (DL1 & UL1, DL2 & UL2, DL3 & UL3, or DL4 & UL4).

- NOTE: this does not imply that the only configuration(s) to be specified are according to these constraints.

There are diverged views on whether all of the restrictions or part of them should be taken into account for the specification design. If the co-existence use case is the target scenario, there needs to be a clarification to require that all the legacy restrictions should be taken into account during the normative work, no exceptions. However, if the co-existence use case the not the only target scenario, then more flexible design can be pursued. But in this case, even the possibility N value other than 9 can also be considered, e.g. N=8 as was also discussed in RAN1.

Proposal 2: Given the time limit, normative work can only focus on the co-existence scenario. But in any case RAN plenary needs to clarify if IoT-NTN TDD co-exists with legacy Iridium service in 1.6GHz MSS band is the only target scenario or non co-existence scenario is also considered.

Proposal 3: If RAN decides that IoT-NTN TDD co-exists with legacy Iridium service in 1.6GHz MSS band is the only target scenario, the WID can be updated as follows:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern including confirming the value of N, and associated UE procedures [RAN1, RAN2] - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Note: all the design restrictions agreed in RAN1#119 meeting should be taken into account.

Proposal 4: If RAN decides that other non co-existence scenario can also be considered, the WID can be updated as follows:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern including confirming the value of N, and associated UE procedures [RAN1, RAN2] - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Note: all or some of the design restrictions agreed in RAN1#119 meeting can be considered. - Note: For value of N other than 9 can be considered.

Conclusion

In this paper, we discussed the checkpoint status and some needed clarifications identified during the WG discussion on this WI. Finally, we provided some suggestions to address the expected WID update.

Proposal 1: The feasibility of periodic pattern in 1616-1626.5 MHz MSS band can be confirmed and the normative work can start from Feb’2025.

Proposal 2: Given the time limit, normative work can only focus on the co-existence scenario. But in any case RAN plenary needs to clarify if IoT-NTN TDD co-exists with legacy Iridium service in 1.6GHz MSS band is the only target scenario or non co-existence scenario is also considered.

Proposal 3: If RAN decides that IoT-NTN TDD co-exists with legacy Iridium service in 1.6GHz MSS band is the only target scenario, the WID can be updated as follows:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern including confirming the value of N, and associated UE procedures [RAN1, RAN2] - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Note: all the design restrictions agreed in RAN1#119 meeting should be taken into account.

Proposal 4: If RAN decides that other non co-existence scenario can also be considered, the WID can be updated as follows:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern including confirming the value of N, and associated UE procedures [RAN1, RAN2] - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Note: all or some of the design restrictions agreed in RAN1#119 meeting can be considered. - Note: For value of N other than 9 can be considered.

Reference

[1] RP-242415 New WID on introduction of IoT-NTN TDD mode