3GPP TSG RAN Meeting #106								RP-242802

Madrid, Spain, December 9-12, 2024

Agenda Item:	9.3.1.5

Source:	OPPO

Title:	Discussion on WID Scope for R19 network energy saving enhancement

Document for:	Discussion & Decision

Introduction

In this paper, we provide our views on the issues which are raised in the WG discussion and require WID update, including

- Whether to apply OD-SIB1 to RRC_CONNECTED.

- Whether to allow Cell A to provide its own WUS configuration.

- Whether to merge SSB adaptation from objective 3 to objective 1.

Discussion

OD-SIB1 for RRC_CONNECTED

R2#127 agreed to avoid the impact on the RRC connected UE due to OD-SIB1

Impacts to RRC connected UEs:

7.	RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

While RAN2#128 discusses to extend the OD-SIB1 work to RRC_CONNECTED. The proponents see the benefit in this extension for the use cases of 1) RRC Connection re-establishment (i.e., when T311 is running), and 2) RRC CONNECTED while T311 is not running.

For scenario-1) above, there seem mainly two approaches, either to bar the OD-SIB1 cell to exclude it as a candidate for cell selection (during RRC re-establishment), or to utilize the UL WUS configuration to request SIB1, in a similar way as by RRC_IDLE / RRC_INACTIVE UE.

For scenario-2) above, similarly, there seems also two approaches, either rely on network implementation to send the related SIBs proactively (which is only necessary when the concerned SIB content has been changed compared to the previous version, and when the UE is operating for SL, MBS, ATG, Pos services), or rely on UE request for SIB1, for which the solution may vary depending on whether the CSS has been configured for the UE or not (or UE can blindly send DedicatedSIBRequest without reading si-BroadcastStatus in SIB1).

So, there are various ways to solve this issue, while the difference is just different trade-off between complexity and performance. In our view, this issue has to be concluded anyway, in the sense that the RRC_CONNECTED UE behaviour has to be clarified in case of OD-SIB1 cells, while whether we go for a solution requiring less specification effort (which is our preference) or a solution further optimizing the performance can be left to WG level discussion.

On the other hand, there is no need to update the WID since this discussion does not necessarily lead to enabler(s) to support OD-SIB1 request by RRC_CONNECTED UEs (considering the former approach for both scenarios above), and also considering it is not preferred to hint an extended scope at this late stage of Rel-19 normative stage.

Proposal 1: R2 discusses how to handle RRC_CONNECTED UE in a cell operating OD-SIB1, without further WID update.

Cell A definition

The following shows the agreements achieved on the role of Cell A.

The latest WID: - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized RAN1 Agreements For discussion purpose, the following assumption will be used in RAN1 - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 studies the following options. On target cell of UL WUS transmission: - Option 1: UE transmits UL WUS to NES Cell - Option 2: UE transmits UL WUS to Cell A On configuration provision for UL WUS transmission - Option A: UE obtains the UL WUS configuration from NES Cell - Option B: UE obtains the UL WUS configuration from Cell A Other options are not precluded Conclusion For on-demand SIB1 in idle/inactive mode, RAN1 was not able to achieve consensus to support Case 1 (Option 1+A+X) in Rel-19, while Case 1 is technically possible from RAN1’s perspective. RAN2 Agreements Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells: - More than one Cell A may provide configuration for the same NES cell. - The same Cell A may assist more than one NES Cells. No consensus for RAN1 case 3 in RAN2. RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1. Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB A NES cell can include neighbouring NES cell’s WUS configuration. NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

On the other hand, R2#128 discussed the issue of flexible tunning the role of Cell-A and NES-Cell:

R2-2410842	Considerations on OD-SIB1 feature	Deutsche Telekom, Vodafone, Lenovo, Fraunhofer IIS   	discussion	Rel-19

Proposal 1: Clarify that Cell A can also transmit the WUS configuration for itself in case it intends to become a SIB1-less cell.

- Noted.

Discussion:

[IDC]: Specification will not specify cell A or NES cell, but since we already agreed NES cell can include its own WUS configuration, in principle agree with proposal 1. [Huawei]: Agree with IDC. [NTTDCM]: Having the clarification is good to other WGs since there are some confusion there.

[Qualcomm]: We should make sure the proposal 1 does not mean standalone case is supported. [Ericsson]: With the proposal, cell A needs to provide its own WUS configuration. [Nokia]: In addition, NES cell also needs to provide any neighboring cells’ WUS configuration (no restriction to neighboring cell’s NES cell). [Huawei]: We do not need to agree any further thing except what we already agreed. [Qualcomm]: Concerns on agreeing with the proposal. We already finished SI based on the terms and now we’re changing the terms. [Apple]: It was discussed in RAN1 and no conclusion was made. Would like to suggest to discuss in RAN.

Obviously, Cell A is the cell that at least provides its own SIB1 periodically, so, typically, Cell A should be a macro cell to provide coverage and avoid access holes. On the other hand, if Cell A is also required to transmit the WUS configuration for itself in case it intends to become a SIB1-less cell, it means,

- Following the current Cell-A/NES-Cell framework, in case a Cell-A changes the role to NES-Cell, there still needs another Cell-A for the system to work. Otherwise, if no other normal cell is deployed for coverage, the idle UE cannot find a suitable cell in case that it has not acquired the WUS configuration of NES cell or Cell A before, and thus would lead to coverage hole. In short, standalone NES-Cell cannot work if we still want to avoid coverage hole issue.

- Under the assumption of 1) (i.e., the Cell-A/NES-Cell framework that WG have been developing so far is kept), there is no specification effort foreseen to support the Cell-A/NES-Cell role change, considering ASN.1 encoding does not impose a restriction on which cell(s) the WUS configuration of a cell can/cannot cover. E.g., a NES-Cell/Cell-A may include WUS configuration of another Cell-A, if the latter one is to change to a NES-Cell soon. And surely this should not cause R1 work like carrying UL WUS configuration in SSB or pre-defined UL WUS (as what has been discussed for case-1), which is a major change of WID scope.

Proposal 2: Confirm there is no need to restrict which cell providing WUS configuration of which cell. While this shall not cause R1 work like delivering UL WUS configuration via SSB and/or pre-defined (as what has been discussed for case-1).

SSB adaptation in Objective 3

Up to RAN1#119 meeting, based on the discussion outcomes on SSB adaptation in Objective 3, it can be summarized that

- SSB adaptation is only targeted for RRC connected UE

- SSB adaptation is only supported for SCell

- SSB adaptation is supported for NCD-SSB on sync raster or not on sync raster

RAN1#118 Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’) RAN1#118bis Conclusion There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19 RAN1#119 Conclusion There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode)

It seems that with all the above mentioned conditions, the SSB adaptation in Objective 3 can be merged to Objective 1, because the first two conditions are same as Objective 1, while the Objective 1 does not preclude the SSB to be CD-SSB on the sync raster. This means that the applicability scope of Objective 1 is even broader than Objective 3. Moreover, according to the latest progress, the on-demand SSB can already support time domain adaptation at least for SSB periodicity. So, the adaptation mechanism designed from Objective 1 can be reused directly for Objective 3.  Moreover, with regard to design progress, Objective 1 is way more ahead to Objective 3, where most of the work on the signaling design as well as UE behavior for SSB reception has been done. On the other hand, Objective 3 has not yet started to discuss signaling and UE behavior. Therefore, given the limited time left, we suggest removing the SSB adaptation from Objective 3, since SSB adaptation in time domain is already supported by Objective 1.

Proposal 3: Update the WID by removing “Adaptation of SSB in time domain, e.g. adapting periodicity” from Objective 3 with the understanding that it can be covered by the work of Objective 1 already.

Conclusion

We have the following proposals:

Proposal 1: R2 discusses how to handle RRC_CONNECTED UE in a cell operating OD-SIB1, without further WID update.

Proposal 2: Confirm there is no need to restrict which cell providing WUS configuration of which cell. While this shall not cause R1 work like delivering UL WUS configuration via SSB and/or pre-defined (as what has been discussed for case-1).

Proposal 3: Update the WID by removing “Adaptation of SSB in time domain, e.g. adapting periodicity” from Objective 3.

Reference

[1] RP-242354 Revised WID: Enhancements of network energy savings for NR