3GPP TSG RAN Meeting #106												RP-242941

Madrid, Spain, December 9-12, 2024

Title: 	On request from ETSI MGS TFES on co-existence requirement limits

Agenda Item:	7

Source: 	Huawei, HiSilicon, Ericsson, Nokia, NEC, ZTE

Document for:	Discussion

# Introduction

LS from ETSI TC MSG TFES was received in RP-242432 [1] on co-existence TX spurious emission limits for high frequency bands. It claims, that re-use of the legacy TX spurious emissions co-existence limits for the high frequency bands may require revisiting.

# Discussion

Referring to the co-existence limits captured in the table below (extract from TS 38.141-1, table 6.6.5.5.1.3-1), one can notice that the same basic limit of -52 dBm was used for frequency bands from 450 MHz up to 7 GHz.

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limit | Measurement bandwidth |
| --- | --- | --- | --- |
| E-UTRA Band 31 or NR Band n31 | 462.5 -467.5 MHz | -52 dBm | 1 MHz |
|  | 452.5 -457.5 MHz | -49 dBm |  |
| E-UTRA Band 46 or NR Band n46 | 5150 – 5925 MHz | -52 dBm |  |
| E-UTRA Band 47 | 5855 – 5925 MHz | -52 dBm |  |
| E-UTRA Band 48 or NR Band n48 | 3550 – 3700 MHz | -52 dBm |  |
| NR Band n102 | 5925 – 6425 MHz | -52 dBm |  |
| NR Band n104 | 6425 – 7125 MHz | -52 dBm |  |

The above observation has triggered discussion on relevance of co-existence TX spurious emission requirements for such a wide frequencies range. ETSI MSG TFES asked 3GPP RAN WG4 to re-evaluate whether those limits are appropriate for all operating bands considered above. To facilitate work in RAN4 the work needs to be divided in multiple frequency regions. Initially, it would be appropriate to start with bands in the upper region of FR1.

This topic was discussed during RAN4#113 meeting in [2]. RAN4 has reached conclusion that this topic requires RAN4 work. Due to relatively high workload expected for this task to be completed (co-existence simulations are expected to be required), it was identified that proper work placement in RAN4 is required.

The following two alternatives were identified as potential solutions to address TX co-ex limit re-evaluation in RAN4, based on ETSI TC MSG TFES request:

- RAN to task RAN4.

- Introduction of a new objective in NR_BS_RF_req_evo Rel-19 work item in [3].

Other solutions are not precluded.

# Conclusion

Based on the above discussion, the following proposal was formulated:

Proposal 1: RAN to provide guidance to RAN4, on how to address TX co-ex limit re-evaluation based on ETSI TC MSG TFES request, considering (but not limited to) the following alternatives:

- RAN task to RAN4,

- Introduction of a new objective in NR_BS_RF_req_evo Rel-19 work item in [3].

Other solutions are not precluded.

# References

[1]	RP-242432	LS on Transmitter spurious emissions: co-existence requirements limits for high frequency bands (TFES(24)081026r2; to: RAN4; cc: RAN; contact: Ericsson), LSin, ETSI TC MSG TFES

[2]	R4-2418396	On the topic of the need to reevaluate requirement levels for BS transmitter co-existence spurious emission for bands in upper region of FR1, RAN4#113, Ericsson

[3]	RP-241284 	NR base station (BS) RF requirement evolution for FR1/FR2 and testing, NR_BS_RF_req_evo, WID