3GPP TSG RAN#106	RP-243137

Madrid, Spain, December 9-12, 2024

Agenda Item:	15.2

Source:	Ericsson

Title:	Discussion on Rel-20 Data Collection for SON/MDT

Document for:	Discussion, Decision

# 1	Introduction

SON was first introduced in LTE to support self-configuration and self-optimization of the network. In NR, SON/MDT was introduced in Rel-16, and has in the following releases been enhanced to support more use cases and accommodate new features in the network. Also in Rel-20, we anticipate the need for enhancements of SON/MDT to match new features introduced in Rel-19.

# 2	Discussion

## 2.1	MRO Enhancements for Rel-19 Mobility

SON/MDT includes support for Mobility Robustness Optimization (MRO), which has over the NR development been enhanced to follow the development of NR mobility. The MRO solutions typically focus on reporting of Radio Link Failures (RLF) related to mobility and Near Failures in order to optimize mobility configurations. Based on RLF reports, failures can be classified into too late handover, too early handover, and handover to wrong cell. Sometimes also the (UE collected) Mobility History Information (MHI) and the (network collected) UE History information (UHI) tracking the history of cells the UE has visited is considered as MRO features.

In release-19, NR mobility enhancements Phase 4 [1] is introduced. The objective of the work item includes

- Inter-CU LTM support

- Measurement related enhancements supporting LTM

- Conditional LTM

With the enhancements of the mobility support, the MRO framework needs to be updated to also account for the enhanced mobility features, as well as to ensure that legacy MRO solutions are not in the risk of breaking, for example by “pollution” of the statistics from UEs with other prerequisites due to usage of the enhanced mobility solutions.

Before knowing the final outcome of the Rel-19 mobility enhancements it will be hard to say what SON/MDT enhancements are needed, but there may for example be an impact to

- MRO inter-node messaging
Inter-CU LTM support may result in need for updated MRO-related inter-node messaging.

- UE reporting
New measurements and usage of these for triggering handover might result in the need for updated information in the MRO related reports, for example the RLF report.

- Failure classification definitions
The failure classification in MRO today is coupled with which type of handover was performed/configured in order to be able to adjust the configurations of the right handover type when optimizing based on the failures. With for example introduction of conditional LTM, new failure classes might be needed, and failure classification definitions need to be updated.

Include MRO enhancements for NR mobility enhancements Phase 4 as an objective for SON/MDT Rel-20.

## 2.2	SON/MDT for Wake-Up Signal Observability

In Release-18, a study [2] was performed on low-power wake-up signal and receiver for NR, which in Rel-19 is followed by a work item [3] specifying the wake-up signal for both RRC Idle/Inactive and RRC connected states. The wake-up receiver (WUR) is a low power receiver in UEs that can detect a wake-up signal (WUS) and if needed wake up the main receiver (MR) to detect an incoming message, typically a paging message.

MDT logging and reporting performed by the UE is used to observe the network to understand coverage and performance. With the introduction of WUS and WUR, MDT needs to be updated both to provide WUS observability, and to ensure that the legacy MDT solution is not breaking by “pollution” of the statistics from UEs using WUS. The wake-up receiver and the main receiver have different characteristics, and to understand the measurements collected by MDT it needs to be clear whether the UE has measured using the wake-up receiver or the main receiver. This would not be possible to understand using the existing MDT framework.

Further, one usage of MDT measurements is optimization aiming at avoiding or decreasing coverage holes. Since the wake-up receiver link performance can be expected to be worse than legacy main receiver performance (Rel-19 WUR work item description states the LP-WUS reception should match Msg3 PUSCH performance, which is several dB worse than paging PDCCH performance), the coverage of DL WUS needs careful consideration. Hence it is beneficial to enable the network to configure MDT measurements for the Wake up signal.

In addition, Rel-19 “Enhancements of network energy savings for NR” WI [4] considers OnDemand SIB1 acquisition. The purpose of this objective is to help the network to conserve energy by avoiding SIB1 transmission in certain cells instead of, as in legacy, periodically transmitting SIB1 in all NR cells. Rather, SIB1 is to be transmitted on-demand when a UE needs it. If a UE needs SIB1 it will transmit an UL wakeup signal (WUS) based on the configuration received from assisting cell which in turn triggers the OnDemand SIB1 by the network.

Such coordination between NES cells and the assisting cells which broadcast the UL WUS configuration requires at least alignment in terms of coverage as otherwise the mismatch of coverage between such cells leads to sub-optimal performance in the OnDemand SIB1 acquisition.

Include SON/MDT for WUS observability as an objective for SON/MDT Rel-20.

## 2.3	Renewable energy-based SON MLB

Within a network, the sources of energy that power the NG-RAN nodes can be of different nature, some of which can be renewable energy sources such as, e.g., solar, wind, etc.

SON Mobility Load Balancing (MLB) is an existing SON feature enabling the neighbour NG-RAN nodes to distribute the load among them by considering criteria such as local load, available capacity, etc.

The purpose of this use case is to enhance the SON MLB feature to consider the energy source as a criterion for load balancing, along with the existing criteria.

Include renewable energy-based SON MLB as an objective for SON/MDT Rel-20.

# 3	Proposal

Based on the discussion in the previous sections we propose the following:

Proposal 1	Include MRO enhancements for NR mobility enhancements Phase 4 as an objective for SON/MDT Rel-20.

Proposal 2	Include SON/MDT for WUS observability as an objective for SON/MDT Rel-20.

Proposal 3	Include renewable energy-based SON MLB as an objective for SON/MDT Rel-20.

# 4	References

RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN Meeting #105, Sep 9-12, 2024

RP-232672, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, vivo, 3GPP TSG RAN Meeting #101, Sep 11-15, 2023

RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO, 3GPP TSG RAN Meeting #105, Sep 9-12, 2024

RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson, 3GPP TSG RAN Meeting #105, Sep 9-12, 2024