3GPP TSG RAN Meeting #106	                                   	RP-243168

Madrid, Spain, December 9-12, 2024

Title: 	Discussion on Rel-19 LP-WUS separate band issues

Source: 	ZTE Corporation, Sanechips

Agenda item:	9.3.1.4

Document for:	Discussion and decision

Introduction

It has been discussed in RAN WGs regarding LR and MR operating in the same carrier or different carriers. However, there is no consensus about whether separate band can be supported for LR and MR operation.

In RAN1#118 meeting, RAN1 triggered the discussion and sent the LS to RAN2 and RAN4.

Agreement Send an LS to RAN2 and RAN4 to convey the following In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. Request RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs. Final LS in R1-2407559.

RAN2 started the discussion with the following reply LS [2] sent to RAN1 and RAN4 to show the RAN2 status:

State of discussions in RAN WG2: During RAN WG2 discussions, many companies raised concerns that if LP-Receivers support only subset of the bands supported by the main receiver and if LP-WUS, Paging and RACH are executed in the LP-WUS bands it might lead to overload of these bands if the NW set the LP-WUS frequency as a higher priority. This will impact user performance and decrease the utilization of higher frequency bands. In order to ensure the LP-WUS capable UEs are camping on the band LR implemented, the NW would need to offload LP capable UEs to LR band. On the other hand if the NW operator does not offload the UEs to the band LP receiver supports, only a subset of UEs would profit from LP-WUS feature. Some companies do not see that overload problem would occur as the NW implementation can control the offload of the UEs to the lower bands in dedicated way. In regards to the overload problem, RAN WG2 has not finished evaluations and would need to study the solutions more.

RAN4 also made the following agreements in previous meetings as below [3,4]:

RRM session Agreement: - RAN4 will further discuss whether any technical issues regarding the LS from RAN1. - From Rel-19 RAN4 RRM requirement of MR offloading and relaxation perspective, RAN4 assumed LR and MR are operating on the same carrier frequency as baseline. FFS for the case of MR and LR working on different carrier frequencies if it is supported in RAN1/2. Issue for further discussion based on contribution driven: - For WUS monitoring assumption, further discuss whether or not the same carrier frequency is assumed for LR and MR.] RF session Agreements: No specific RF work related to RAN1 LS.

Given the situation that there is no consensus about whether separate band could be supported, the guidance from RAN plenary is helpful for the future work in WGs.

Discussion about separate band

During the SI stage, the following is captured in the TR regarding this issue.

RAN1 studied LP-WUS bandwidth, at least for IDLE/Inactive mode, at least one BW-size smaller or equal to 5MHz is recommended to be supported for FR1. Other BW sizes are not precluded, however if additional BW-size(s) are recommended to be supported, BW-size can be up to 20MHz. LP-WUS bandwidth size (including guard-bands) is assumed to be an integer number of PRBs. From RAN1 perspective, LP-WUS and signals/channels by MR can be at least on the same carrier in the same band. From RAN1 perspective, for multiplexing with other NR signals and channels, it is beneficial if LP-WUS can be flexibly configured within a carrier.

From RAN1 perspective, LR and MR supporting separate bands can help WUR to have less operation restriction which could be beneficial for WUR complexity, power consumption reduction, and coverage. Also, it may have less restriction on the NW deployment since the WUR does not need to be deployed in the same band as the initial DL BWP, and also the required system overhead would be reduced due to the better coverage in a low frequency band.

For example, if one cell transmits the SSB at 2.6GHz or at other high frequency, based on same band restriction, the LR probably cannot support this frequency due to the high complexity, power consumption, less coverage and larger system overhead if LR can only deployed when SSB is located in the supported bands. If separate band operation can be supported, LP-WUS can be deployed at some lower frequency bands, which is more flexible for the NW, and the LP-WUS operating at lower frequency has better coverage and cost less system overhead.

Observation 1: LR and MR supporting separate bands operation can help reduce WUR complexity and power consumption. It also helps NW achieve better LR coverage, save system overhead, and deploy LP-WUS more flexibly.

Additionally, the spec impact in RAN1 is negligible. For example, the RSRP measurement metrics could be applied for RRC_IDLE inter-frequency, RRC_INACTIVE inter-frequency, in addition to RRC_IDLE intra-frequency, RRC_INACTIVE intra-frequency.

Observation 2: The spec impact of separate bands in LR and MR in RAN1 is negligible.

From RAN2’s perspective, the following options can be considered for the case that LR and MR are on the different bands, i.e. LP-Receivers support only subset of the bands supported by the main receiver:

- Option 1: Assign higher reselection priority for LP-WUS frequency so that LP-WUS UE would be guided to such frequency supporting both LR and MR

- Option 2: UE receives LP-WUS in one band and reselects to another band for the follow up random access via MR

As mentioned in the reply LS to RAN1 and RAN4, some companies had the concern that it might lead to overload of these bands if the NW set the LP-WUS frequency as a higher priority and LP-WUS, Paging and RACH are executed in these LP-WUS bands, i.e. option 1. While for option 2, it depends on whether UE can support such switch from LR to MR band for LP-WUS monitoring and follow up access separately.

There are pros and cons for these two options but we understand supporting different bands for LR and MR operation is feasible from RAN2’s perspective with these two options to down select.

Observation 3: Supporting different bands for LR and MR operation is feasible from RAN2’s perspective with the following two options considered. Some companies are concerned about the potential overload issue caused by option 1 while option 2 requires UE to support the switch from LR to MR for LP-WUS monitoring and follow up access separately.

- Option 1: Assign higher reselection priority for LP-WUS frequency so that LP-WUS UE would be guided to such frequency supporting both LR and MR

- Option 2: UE receives LP-WUS in one band and reselects to another band for the follow up random access via MR

RAN4 also studied this issue, the following is captured in the TR.

Based on the study, RAN4 reached the following conclusions: - RAN4 studied ACS and ASCS of LP-WUS, RAN4 concluded that the LP-WUS signal can coexist with existing NR signal/channels in the same channel, or in an adjacent channel(s) with proper number of guard RBs. - It is beneficial to enable power boosting for LP-WUS to improve the WUS signal coverage. The supporting of LP-WUS power boosting and boosting level can be declared by manufacturer. - RAN4 discussed band operation for LP-WUS and concluded that the operating band for the low-power radio (LR) and main radio (MR) can be the same or different from RF perspective.

RAN4 has discussed the band operation for LP-WUS according to the scenarios listed below with the understanding that the associated RF aspects could be analysed in the work item phase, depending on the scope of the related normative work. 1) The band for the low-power radio (LR) and main radio (MR) can be the same, e.g., WUS is located within the NR band used for LP-WUR. In this scenario the WUS and NR DL could be multiplexed according to TDM/FDM modes. 2) The band for LR and MR can be different, e.g., WUS is located within an NR band for LP-WUR, and another NR band is used for the MR.

Based on the RAN4 discussion, the identified use case is the co-sited scenario. Compared with same band operation, the following analysis can be considered:

From UE perspective, LR is not implemented as many bands as that of the MR. From NW perspective, for a specific band, it may not support LP-WUR operation when MR is implemented for that band. To ensure more UEs could obtain power saving gain and reduce the implementation cost of this feature, we think it is reasonable to consider LR and MR at different bands. For instance, when a LP-WUR is implemented at a low-frequency band, the MR will camp on higher frequency band via higher priority frequency layer search in the Idle/Inactive mode. On the contrary, the LP-WUR feature may be quite limited if MR supported bands via higher frequency layer search may be not align with the LR supported bands and inter-band operation is not supported.

Observation 4: If the separate band on LR and MR operation is not supported, the potential of the LP-WUR feature may not be fully exploited.

Since we know that the LR measurement only depends on LP-RSRP/RSRQ or OFDM based existing PSS/SSS. Thus, LR measurement requirements are not related to MR operating band, that is, LP-WUR measurement requirements independent from whether LP-WUR and MR work at the same band or not. Based on this point, there is no extra specification work for RAN4 LP-WUR measurement requirements if LP-WUR and MR work at the different bands.

Observation 5: There is no extra specification work for RAN4 LP-WUR measurement requirements if LP-WUR and MR work at the different bands.

Proposal: RAN to discuss and decide whether separate band for WUR is supported or not for co-sited scenario in Rel-19.

Conclusion

In this contribution, we have the following observations and proposal:

Observation 1: LR and MR supporting separate bands operation can help reduce WUR complexity and power consumption. It also helps NW achieve better LR coverage, save system overhead, and deploy LP-WUS more flexibly.

Observation 2: The spec impact of separate bands in LR and MR in RAN1 is negligible.

Observation 3: Supporting different bands for LR and MR operation is feasible from RAN2’s perspective with the following two options considered. Some companies are concerned about the potential overload issue caused by option 1 while option 2 requires UE to support the switch from LR to MR for LP-WUS monitoring and follow up access separately.

- Option 1: Assign higher reselection priority for LP-WUS frequency so that LP-WUS UE would be guided to such frequency supporting both LR and MR

- Option 2: UE receives LP-WUS in one band and reselects to another band for the follow up random access via MR

Observation 4: If the separate band on LR and MR operation is not supported, the potential of the LP-WUR feature may not be fully exploited.

Observation 5: There is no extra specification work for RAN4 LP-WUR measurement requirements if LP-WUR and MR work at the different bands.

Proposal: RAN to discuss and decide whether separate band for WUR is supported or not for co-sited scenario in Rel-19.

References

- RP-240801 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

- R2-2410957 Reply LS to R2-2407921/ R1-2407559

- R4-2416861 WF on RRM requirements for NR_LPWUS, RAN4 112bis, vivo

- R4-2417112 WF on UE RF requirements for LP-WUS, RAN4#112bis, vivo