3GPP TSG RAN Meeting #106	  RP-243322

Madrid, Spain, December 9-12, 2024

Agenda item:	9.1.4 Proposals led by RAN4 (not spectrum related)

Source:	T-Mobile USA, UScellular

Title:	Way Forward on Irregular Channel Bandwidths

Document for:	Discussion

# 1	Introduction

At RAN#106 a proposal for an irregular channel bandwidth Work Item was discussed [1][2].

In this paper, we propose a Way Forward.

# 2	Discussion

## 2.1	Motivation & proposal

For details on the motivation for the new Work Item, please see the motivation paper in [2]. To summarize,

- Several operators have spectrum that is not a multiple of 5 MHz, and some have been waiting for over 5 years for the ability to efficiently deploying NR in these “irregular” channel bandwidths.

- The next wider channel bandwidth approach with PRB blanking is not ideal for low frequency “coverage” bands with non-co-located spectral neighbours due to the potential strong blockers in the blanked PRBs

- The overlapping carrier from the network perspective technique would require frequency overlapped and time-offset SSBs for 6 and 7 MHz channel bandwidths because one SSB will not fit in the overlapped spectrum, and two frequency-adjacent SSBs will not fit. Frequency-overlapped, time-offset SSBs may not be supported by legacy UEs.

- The overlapping channel from the network perspective approach would be compatible with channel bandwidths ≥ 9 MHz.

- The next wider channel bandwidth approach would be appropriate for capacity bands, where the chance of blocking may be acceptable.

## 2.2	Proposal

Proposal:

Considering the needs of operators and workload in RAN4, it is proposed:

- Approve the creation of a new Release-19 Work Item [3] which includes only 7 MHz channel bandwidth for n5 and n26.

  - The 7 MHz channel bandwidths can be applied to other bands as needed via a spectrum related Work Item.

- Additional irregular channel bandwidth work, including 6 MHz channel bandwidth, will be deferred to Release 20 for possible consideration.

- Other than 6 and 7 MHz, it is agreed that no other new irregular channel bandwidths will be created for FR1 in the future.

  - For other “irregular” channel bandwidths, operators will use the overlapping carrier or next wider channel bandwidth approaches.

# 3	Conclusions

Proposal:

Considering the needs of operators and workload in RAN4, it is proposed:

- Approve the creation of a new Release-19 Work Item [3] which includes only 7 MHz channel bandwidth for n5 and n26.

  - The 7 MHz channel bandwidths can be applied to other bands as needed via a spectrum related Work Item.

- Additional irregular channel bandwidth work, including 6 MHz channel bandwidth, will be deferred to Release 20 for possible consideration.

- Other than 6 and 7 MHz, it is agreed that no other new irregular channel bandwidths will be created for FR1 in the future.

  - For other “irregular” channel bandwidths, operators will use the overlapping carrier or next wider channel bandwidth approaches.

# References

RP-242871	New WID on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths, China Telecom, T-Mobile USA

RP-242872	Motivation for an Irregular Channel Bandwidth Work Item in Rel-19, China Telecom, T-Mobile USA

RP-243323	New WID on 7 MHz Channel Bandwidth for n26 and n5	T-Mobile USA, China Telecom