3GPP TSG-RAN WG1 Meeting #119	R1-2409391

Orlando, USA, 18th – 22nd November 2024

Agenda Item:	9.4.1.2

Source:	Ericsson

Title:	Ambient IoT device architectures

Document for:	Discussion, Decision

# 1	Introduction

The “Study on solutions for Ambient IoT (Internet of Things) in NR” [1][2][3] targets a further assessment at RAN WG level of Ambient IoT (A-IoT), a new 3GPP IoT technology, suitable for deployment in a 3GPP system, which relies on ultra-low complexity devices with ultra-low power consumption for the very low-end IoT applications. The study follows an initial study captured in TR 38.848 [4].

This contribution presents our perspective on unresolved issues regarding Ambient IoT device architectures.

# 2	Issues

## 2.1	RF energy harvester sensitivity

Literature shows RF harvesting sensitivity values at 900MHz ranging between -15dBm and -25.5dBm [5]. In [6], a harvester sensitivity of -30dBm has been reported at 3.5GHz using a loop antenna and rectifier based on 65nm TSMC Si technology. We believe that it should be feasible to achieve a harvesting sensitivity of -30dBm at 900MHz with proper co-design of antenna and rectifier.

Regarding the power conversion efficiency (PCE), typical values reported in the literature at low input power levels range between 5% and 22% for the 900 MHz band. The power conversion efficiency is not constant with RF input power and usually decreases at low input power levels.

RF harvesting sensitivity of around -30 dBm can be achieved at 900 MHz.

In literature, for 900 MHz band, the RF power conversion efficiencies (at the harvester sensitivity levels) ranging between 5% and 22% are reported.

## 2.2	Filter assumptions and selectivity

An RF BPF is preferable to be used after the antenna since the antenna bandwidth should cover both the uplink and downlink bands. This RF BPF will provide similar bandwidth as the RF band to suppress out of band interference, but it cannot perform channel selection. The RF BPF may be omitted when an LNA is present in the RF-ED receiver since the quality factor of the LNA’s input matching network will need to be high and thus the matching network will provide enough filtering. However, note that without the BPF, the LNA may amplify more noise and it may need to have better linearity (since it will be more likely that the LNA will saturate generating intermodulation products).

RF-ED:

For the RF-ED architecture, channel selectivity is expected to be not feasible. In addition, the rejection of interferers will be poor since the RF-ED down-converts the wanted channel and the adjacent channels within the ED bandwidth (which corresponds to that of RF BPF bandwidth and can be in the order of one or a few tens of MHz) to baseband at the same time. This makes it difficult to filter out adjacent channels with RF-ED architecture.

- For Device 1, a 2nd order input matching filter can be used with a possibility to add an optional sharper SAW filter.

- For Device 2a/b, a 4th order on-chip filter can be used with option to add a sharper SAW filter off chip.

Zero-IF and IF-ED:

For non-RF-ED architectures (IF-ED and zero-IF), where a mixer is used, the channel selection can be performed by a filter after the mixer if the LO accuracy is sufficient. In IF-ED architecture, an IF BPF is used after the mixer, while in a zero-IF architecture, a BB LPF is used after the mixer to perform channel selection.

- RF filter: for Device 2a/b, a 2nd order on-chip RF filter can be used.

- IF filter: In the IF-ED architecture, a sharper lower bandwidth on-chip filter, compared to the RF filter, can be used at IF, after the mixer. Optionally, an external SAW filter may be added to protect the receiver from strong out-of-band signals.

- BB Low pass filer: A 3rd order RC filter, might be sufficient depending on the adjacent channel environment.

DL FDMA

Since Device 1 only supports RF-ED architectures, it will not be able to support DL FDMA. Device 2a/b will instead have the capability to support DL FDMA. We think that it is more important for Devices 2a and 2b to support DL FDMA compared to Device 1, the reason being the potentially larger coverage targets for Devices 2a and 2b compared to Device 1. In fact, RAN1#116bis agreed that the coverage targets can be different for different device types. This means the capacity requirements might be higher for Devices 2a and 2b as opposed to Device 1 if the device density is the same. For instance, assuming 150 devices per 100 m2, the number of devices within a 10-meter cell radius is 471 and the number of devices in a 50-meter cell radius is 11781.

In Device 1, a 2nd order RF BPF (either separate or integrated in the input matching network) can be used after the antenna since the antenna bandwidth should cover both the uplink and downlink bands.

In Device 2a/b with RF-ED architecture, a 4th order on-chip filter can be used with option to add an off-chip SAW filter.

It is beneficial, especially for Devices 2a/2b, to support DL FDMA as they have a larger coverage target (and hence greater number of devices to be inventoried) than that of Device 1.

Capture in TR 38.769 that channel selection filter, and hence DL FDMA, may not be feasible if the RX architecture is based on RF-ED.

Capture in TR 38.769 that channel selection filter, and hence DL FDMA, is feasible with other RX architectures, such as IF-ED and ZIF.

## 2.3	Comparator/ADC

There are practical issues when using a comparator, such as the comparator having an unknown offset that is slowly varying because of flicker noise and temperature. In addition, a comparator also has metastability issues, i.e., if the signal is too weak it cannot take a decision in due time. An n-bit ADC will reduce this problem with minimal power consumption increase compared to a comparator, provided that only few bits are used. Due to the low data rate requirements, the ADC will not need to be fast and hence it can be low power. However, using an ADC may require the implementation of automatic gain control (AGC) to ensure that the input signal to the ADC falls within the conversion range. For this reason, we believe that, for Device 1, due to the extremely low power budget, a 1-bit comparator may be the best solution. For Devices 2a and 2b, a 2-bit or 4-bit ADC is feasible due to the higher power budget.

For Device 1, a 1-bit comparator would be preferable if no AGC can be implemented.

For Devices 2a and 2b, a 2-bit or 4-bit ADC could be feasible due to the higher power budget.

## 2.4	Clock/LO

RAN1#118 made the following agreement related to clock/LO:

Agreement Adopt the following table for clock Description; Applicable device types; Clock speed; Power consumption; Initial clock accuracy; Accuracy after clock sync / calibration; Clock drift. Purpose #1 of the clock; Sampling; Device 1, 2a, 2b; a few MHz; < [1]uW for device 1 FFS for device 2a/2b; [10^4 ~ 10^5] ppm for device 1 [10^3~ 10^4] ppm for device 2a/2b; FFS (if applicable for device 1); FFS. Purpose #2 of the clock; Small frequency shift; At least for device 1 and device 2a. [Purpose #3 of the clock]; [Time counting (if supported)]; [Device 1, 2a, 2b]; FFS; FFS the same or different for different devices; FFS; FFS (if applicable); FFS. Purpose #4 of the clock; Large frequency shift (if supported for device 2a); Device 2a; 10s MHz; [10s] uW; FFS; FFS; FFS. Purpose #5 of the clock; LO for carrier frequency (for up/down conversion); Device 2b; e.g., [900] MHz; [10s ~ 100s] uW; FFS; FFS; FFS. Note: It does not necessarily imply that different purposes of LOs/clocks correspond to separate discrete LOs/clocks, which is up to implementation..

The table format was further updated in RAN1#118bis as follows:

Agreement Adopt following updated table format. - One or more rows under a single purpose may exist to capture different options, if any. #; Purpose; Clock speed; Applicable Device types; Power consumption; Initial clock Accuracy (ppm); Accuracy after clock sync / calibration at device side (ppm). ….

Additionally, for purposes #3, #4, and #5, RAN1#118bis agreed to the following values:

Agreement For clock purpose #3, adopt following update. #; Purpose; Clock speed; Applicable Device types; Power consumption; Initial clock Accuracy (ppm); Accuracy after clock sync / Calibration at device side (ppm). [ #3 ]; [ Time counting (if supported) ]; e.g. tens kHz; 1 (if applicable) 2a, 2b; <0.1uW; [10^4 - 10^5]; Calibration may not always be feasible for this clock purpose. 2a, 2b; <0.1uW; [10^3-10^4]; FFS N/A. Agreement For clock purpose #4, adopt following update. #; Purpose; Clock speed; Applicable Device types; Power consumption; Initial clock Accuracy (ppm); Accuracy after clock sync / calibration at device side (ppm). #4; Large frequency shift (if supported for device 2a); 10s MHz (e.g., 50MHz); 2a; [10s] uW; [10^3-10^4]; [10^3] (Note 2). Note2: Digital clock counter can be assumed for calibration clock counting based on R2D signals how to calibrate. No drastic temperature change is assumed during calibration.. Agreement For clock purpose #5, adopt following update. #; Purpose; Clock speed; Applicable Device types; Power consumption; Initial clock Accuracy (ppm); Accuracy after clock sync / calibration at device side (ppm). #5; LO for carrier frequency (for up/down conversion); According to carrier frequency e.g., 900 MHz; 2b; 10s ~ 100s uW; [10^23 ~ 10^4]; Option 1: 10s (Note2) Option 2: 100s (Note2). Note 2: Either internal clock counter for calibration clock counting based on at least R2D signals or clock counting based on transmission from reader Calibration is based on existing or new signal from reader. FFS how to calibrate (9.4.2.2). No drastic temperature change is assumed during calibration.. - Further study above options and strive to down select considering its implication on performance, system design, necessity, etc. - Note: study of whether additional signal is necessary or not for calibration is discussed under agenda 9.4.2.2.

Based on our analysis, we propose the following values for purpose #1:

| # | Purpose | Clock speed | Applicable Device types | Power consumption | Initial clock Accuracy (ppm) | Accuracy after clock sync / calibration at device side (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| #1 | Sampling | a few MHz | Device 1 | < 1 uW | 10^4 ~ 10^5 | Not applicable |
|  |  |  | Devices 2a and 2b | < 10 uW | 10^3 ~ 10^4 | 10^2 ~ 10^3 |

We propose the following values for purpose #2:

| # | Purpose | Clock speed | Applicable Device types | Power consumption | Initial clock Accuracy (ppm) | Accuracy after clock sync / calibration at device side (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| #2 | Small frequency shift | a few MHz | Device 1 | < 1 uW | 10^4 ~ 10^5 | Not applicable |
|  |  |  | Device 2a | < 10 uW | 10^3 ~ 10^4 | 10^2 ~ 10^3 |

We propose the following updates for purpose #3 (updates compared to the RAN1#118bis agreement is highlighted in red):

| # | Purpose | Clock speed | Applicable Device types | Power consumption | Initial clock Accuracy (ppm) | Accuracy after clock sync / calibration at device side (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| #3 | Time counting (if supported) | e.g. tens kHz | 1 (if applicable), 2a, 2b | <0.1uW | [10^4 - 10^5] | Calibration may not always be feasible for this clock purpose |

We propose the following updates for purpose #4 (updates compared to the RAN1#118bis agreement is highlighted in red):

| # | Purpose | Clock speed | Applicable Device types | Power consumption | Initial clock Accuracy (ppm) | Accuracy after clock sync / calibration at device side (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| #4 | Large frequency shift (if supported for device 2a) | 10s MHz (e.g., 50MHz) | 2a | [10s] uW | [10^3-10^4] | 10^2 ~ 10^3 [10^3] (Note 2) |
| Note2: No drastic temperature change is assumed during calibration. |  |  |  |  |  |  |

We propose the following updates for purpose #5 (updates compared to the RAN1#118bis agreement is highlighted in red):

| # | Purpose | Clock speed | Applicable Device types | Power consumption | Initial clock Accuracy (ppm) | Accuracy after clock sync / calibration at device side (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| #5 | LO for carrier frequency (for up/down conversion) | According to carrier frequency e.g., 900 MHz | 2b | 10s ~ 100s uW | [10^3 ~ 10^4] | Option 1: 10s (Note2) Option 2: 100s (Note2) |
| Note 2: No drastic temperature change is assumed during calibration. |  |  |  |  |  |  |

Adopt the values in the tables above for the different clock purposes.

## 2.5	Power consumption

The breakdown of power consumption for different architectures are proposed in Table 1 below.

Table 1: Power consumption breakdown

|  | Device 1 |  | Device 2a |  |  |  | Device 2b |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ED |  | Zero-IF |  |  |  |
|  | Component | Power Consumption [µW] | Component | Power Consumption [µW] | Component | Power Consumption [µW] | Component | Power Consumption [µW] |
| Rx | Passive/Active ED | 0/2 | Active ED | 2 | Mixer | 8 | Mixer | 8 |
|  | Comparator/ADC | ≤1 | Comparator/ADC | 1-2 | ADC | 2 | ADC | 6 |
|  | State machine | ≤1 | Digital Processing | 20 | Digital Processing | 20 | Digital Processing | 20 |
|  | Clock | 0.1-1 | Clock | 0.1-1 | FLL | 20 | PLL | 100 |
|  | NV-Memory | 1.5-3 | NV-Memory | 1.5-3 | NV-Memory | 1.5-3 | NV-Memory | 1.5-3 |
|  | V-Memory | 0.1 | V-Memory | 0.1 | V-Memory | 0.1 | V-Memory | 0.1 |
|  |  |  | RF LNA | 40 | RF LNA | 40 | RF LNA | 75 |
|  |  |  | BB AMP | 10 | BB AMP | 10 | BB AMP | 10 |
|  | Total | ~2-7 | Total | ~40-80 | Total | ~100 | Total | ~220 |
| Tx | Digital Encoder | 1e-3 | Digital Encoder |  | 1e-3 |  | PA | 100-300 |
|  | Clock | 0.1-1 | Ring Oscillator |  | 20 |  | DCO+LO Driver + DCO Buffer | 50 |
|  | State machine | 0.1 | RA |  | 100 |  | ADPLL | 150 |
|  | NV-Memory | 1.5-3 | Digital Processing |  | 20 |  | Digital processing | 20 |
|  | V-Memory | 0.1 | NV-Memory |  | 1.5-3 |  | NV-Memory | 1.5-3 |
|  |  |  | V-Memory |  | 0.1 |  | V-Memory | 0.1 |
|  | Total | ~1.8/4.2 | Total |  | ~145 |  | Total | ~300-500 |

For Device 1, the expected Tx power consumption is approximately 2-4 μW, and the expected Rx power consumption is approximately 2-7 μW.

For Device 2a, the expected Tx power consumption is approximately 145 μW, and the expected Rx power consumption is approximately 100 μW.

For Device 2b, the expected Tx power consumption is approximately ~300-500 μW, and the expected Rx power consumption is approximately ~220 μW.

## 2.6	Energy storage and operating duration

The amount of harvested energy can vary vastly depending on the harvesting source (e.g., RF, indoor light, etc.), input power (for energy harvesting), harvesting area (e.g., for indoor light), and efficiency. In the table below, some examples are provided for different energy sources assuming 30s harvesting time:

| Energy Harvesting | Harvesting time | Input Power | Efficiency | Total harvested energy |
| --- | --- | --- | --- | --- |
| RF | 30s | -22.6dBm | 10% | 16.5μ𝐽 |
| RF | 30s | -19.5dBm | 12.8% | 43.2μ𝐽 |
| RF | 30s | -14.7dBm | 13% | 132μ𝐽 |
| Indoor light (area: 2cm2) | 30s | 100μW/cm2 (power density) | 10% | 600μ𝐽 |

Although the harvested energy can be large, the energy storage size, e.g., capacitor size, may not be very large due to cost and size considerations. It should also be considered that larger capacitors tend to have higher leakage compared to smaller capacitors. In addition, ceramic capacitors can offer lower leakage than capacitors with other dielectric materials but are typically very expensive for large sizes.

For Device 1, a 10 μF capacitor (or perhaps even smaller) may be enough to sustain for the entire inventory round due to its low peak power consumption. However, for Devices 2a and 2b, a much larger capacitor is needed to sustain for the entire inventory round due to their much higher peak power consumption. The consequence of having a smaller capacitor (e.g., a 10 or 20 μF capacitor) is that the device may need to harvest energy (perhaps for several times) within a single inventory round. We think such aspects are important to study in RAN1 and can be used as an input during protocol design.

Operation duration:

Assuming four different capacitor values of 1μF, 10μF, 24μF and 100μF as energy storage, assuming current draw of 1µA, 100µA and 200µA for Device 1, 2a and 2b, respectively, and assuming that when the capacitor is discharged from 1V to 0.5V the device switches back to harvesting mode, the expected operation duration based on the exponential decay model (RC discharge model) is shown in the table below:

| Capacitor Size | Device 1 [s] | Device 2a [s] | Device 2b [s] |
| --- | --- | --- | --- |
| 1μF | 0.693 | 0.00693 | 0.003465 |
| 10μF | 6.93 | 0.0693 | 0.0347 |
| 24μF | 16.63 | 0.1663 | 0.08315 |
| 100μF | 69.3 | 0.693 | 0.3465 |

Based on the calculation in our companion paper [7], for the single device case, the average inventory completion time considering the re-attempts with 10% probability for the “inventory-only” use case is 24 ms and for “inventory and command” use case is 29 ms. Therefore, a capacitor size of 1 uF seems sufficient for Device 1, whereas a capacitor size of 10 or 20 uF seems like a reasonable choice for Device 2a/2b.

Note that, for the multiple device case, it takes much longer for devices to complete the inventory (depending on the number of devices to be inventoried) [7]. Therefore, with the above choices of capacitor sizes, mechanisms to allow the device to sleep within an inventory round may be needed for the multiple device case.

For Device 1, a capacitor size in the order of 1μF may be sufficient to allow operation for a full inventory round.

For Device 2a and 2b, a storage capacitor of 10 or 24 μF may be needed to operate for the full inventory round.

Charging time:

The Table below shows the time required to charge capacitors of different sizes from different energy sources, assuming they are completely empty.

| Energy Source | PCE | 1μF Capacitor | 10μF Capacitor | 24μF Capacitor | 100μF Capacitor |
| --- | --- | --- | --- | --- | --- |
| RF @ -25dBm | 10% | 3.3s | 33s | 80s | 333s |
| RF @ -30dBm | 4% | 25s | 250s | 600s | 2500s |
| Indoor light 10 μW/cm2 | 5% | 2s | 20s | 48s | 200s |
| Indoor light 100 μW/cm2 | 5% | 0.2s | 2s | 4.8s | 20s |
| Vibration 1 mW/cm2 | 30% | 0.003s | 0.03s | 0.08s | 0.33s |

From the above table, it´s clear that larger storage capacitors will take a long time to charge if RF harvesting is used as energy source. A small solar panel or a vibration harvester (piezoelectric, electromagnetic, or triboelectric) may be considered in certain application scenarios.

Energy storage size:

The size, volume and price of typical ceramic capacitors are shown in the table below. Electrolytic capacitors have the advantage of lower cost compared to ceramic capacitors [8], but they have not been considered since they have a high self-discharging current.

| Capacitor Size | Model | Technology | Volume [mm3] | Price [Euro/piece] | Amount of units |
| --- | --- | --- | --- | --- | --- |
| 1μF | GRM033R60J105MEA2J | Ceramic Capacitor | 0.054 | 0.004 | 50k |
| 10μF | GRM155C60E106ME16J | Ceramic Capacitor | 0.25 | 0.01 | 40k |
| 22μF | MEASP105CC6226MF1A01 | Ceramic Capacitor | 0.25 | 0.03 | 100k |
| 100μF | MLASA21GBB5107MT | Ceramic Capacitor | 3 | 0.4 | 24k |

The Table below from [9] shows that capacitors self-discharge more quickly and their volume are typically larger compared to rechargeable batteries of same size.

| Technology | Maximum Energy Capacity | Self-Discharge (1.5V to 1.25V) | Trickle Charge | Volume (mm3) |
| --- | --- | --- | --- | --- |
| Ceramic Capacitor 100μF | 112.5μJ | 2.25 hours | 1.5V | 3.375 |
| Polymer Capacitor 100μF | 112.5μJ | 39 hours | 1.5V | 9.216 |
| Film Capacitor 100μF | 112.5μJ | 7.75 hours | 1.5V | 100625 |
| LiPo rechargeable battery | 666J | 30 days | 4.2V | 621 |
| NiMH rechargeable battery | 9.07J | 50 days | 1.5V | 42.3 |

# 3	Conclusion

In the previous sections, we made the following observations:

Observation 1	RF harvesting sensitivity of around -30 dBm can be achieved at 900 MHz.

Observation 2	In literature, for 900 MHz band, the RF power conversion efficiencies (at the harvester sensitivity levels) ranging between 5% and 22% are reported.

Observation 3	In Device 1, a 2nd order RF BPF (either separate or integrated in the input matching network) can be used after the antenna since the antenna bandwidth should cover both the uplink and downlink bands.

Observation 4	In Device 2a/b with RF-ED architecture, a 4th order on-chip filter can be used with option to add an off-chip SAW filter.

Observation 5	It is beneficial, especially for Devices 2a/2b, to support DL FDMA as they have a larger coverage target (and hence greater number of devices to be inventoried) than that of Device 1.

Observation 6	For Device 1, a capacitor size in the order of 1μF may be sufficient to allow operation for a full inventory round.

Observation 7	For Device 2a and 2b, a storage capacitor of 10 or 24 μF may be needed to operate for the full inventory round.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	Capture in TR 38.769 that channel selection filter, and hence DL FDMA, may not be feasible if the RX architecture is based on RF-ED.

Proposal 2	Capture in TR 38.769 that channel selection filter, and hence DL FDMA, is feasible with other RX architectures, such as IF-ED and ZIF.

Proposal 3	For Device 1, a 1-bit comparator would be preferable if no AGC can be implemented.

Proposal 4	For Devices 2a and 2b, a 2-bit or 4-bit ADC could be feasible due to the higher power budget.

Proposal 5	Adopt the values in the tables above for the different clock purposes.

Proposal 6	For Device 1, the expected Tx power consumption is approximately 2-4 μW, and the expected Rx power consumption is approximately 2-7 μW.

Proposal 7	For Device 2a, the expected Tx power consumption is approximately 145 μW, and the expected Rx power consumption is approximately 100 μW.

Proposal 8	For Device 2b, the expected Tx power consumption is approximately ~300-500 μW, and the expected Rx power consumption is approximately ~220 μW.

# References

RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA, RAN#103, March 2024.

R1-2400328, “Ambient IoT Study Item work plan”, CMCC, Huawei, T-Mobile USA, RAN1#116, February 2024.

TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR”, V1.1.0, November 2024.

TR 38.848 V18.0.0, “Study on Ambient IoT (Internet of Things) in RAN (Release 18)”, 3GPP, September 2023.

A High-Sensitivity Wide Input-Power-Range Ultra-Low-Power RF Energy Harvester for IoT Applications

D. U. Yildirim et al., “A 0.7cm2 3.5GHz, -31 dBm Sensitivity Batteryless 5G Energy Harvester Backscattering Chip for Asset Identification in IoT-Enabled Warehouses,” 2024 IEEE Custom Integrated Circuits Conference (CICC), Denver, CO, USA, 2024, pp. 1-2

R1-2409390, “Evaluation assumptions and results for Ambient IoT”, Ericsson, RAN1#119, November 2024.

https://eu.mouser.com

Ultra-low power 2.4GHz RF energy harvesting and storage system with −25dBm sensitivity