3GPP TSG-RAN WG1 Meeting #119	  R1-2409397

Orlando, US, November 18 – 22, 2024

Agenda Item:	9.1.3

Source:	Huawei, HiSilicon

Title:	Discussion on AI/ML for CSI prediction

Document for:	Discussion and Decision

# Introduction

In the RAN#105 meeting, a revised work item on air-interface AI/ML was approved [1]. Normative work for CSI prediction will start from Q1 of 2025. Before normative work, there is a study aspect of consistency between training and inference in CSI prediction:

Provide specification support for the following aspects: … - CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025 …

In this contribution, we will provide our views on whether to ensure consistency by using indications of NW side additional conditions for CSI prediction.

# Consistency of training/inference for CSI prediction

The benefit of ensuring consistency is to align the NW side additional conditions for UE-sided models from performance perspective. In the beam management agenda, it was agreed to support associated ID for consistency. However, whether this kind of indication should be reused in CSI prediction is questionable. In our view, the feasibility and necessity for introducing associated ID to keep consistency between training and inference for CSI prediction should be discussed first.

## 2.1 Feasibility of introducing associated ID for consistency

Before we discuss the consistency, we need to first sort out the factors that may impact the misalignment between training and inference in CSI prediction. From our understanding, there may be multiple factors which may potentially impact the data distribution, including antenna layout, TxRU mapping, down tilt angles, direction of the site, deployment scenario, gNB height, etc.

As a result, NW side actually may not have the knowledge on the factor or combination of factors used for data categorization (e.g., to the associated IDs). Additionally, for each single factor, there may be different quantitative values. NW side may not have the knowledge under what exact quantitative values the two data samples should be categorized with the same associated ID. For example, for two sites with down tilt angles 5° and 7°, should it categorize them with the same associated ID or different associated IDs? How about 5° and 10°? The fact is, for any two gNBs, they may not be 100% identical. If the NW side categorizes the data of two sites with different associated IDs as long as they have any difference from any factor and any quantitative value of the factor, it may assign millions of associated IDs to the UE, which actually would be useless to the UE side – as the CGI (cell global ID) already is available to the UE, it can use it to train cell specific model. In addition, it is not clear how the NW will know to what extent for the bias of a different factor/quantitative value, it needs to assign a separate categorization ID. To harmonize the understanding of the data categorization principle and generalization capability between NW side and UE side, they may need non-trivial offline interaction. Moreover, even if NW knows how to assign the associated ID, with an increasing number of gNB configurations to be maintained, it may incur burdens for NW side to manage the cell group information and the huge number of associated ID information. Therefore, it is difficult to categorize the additional conditions across cells.

Observation 1: For the data collection for the UE-side model in CSI prediction, the massive number of impacting factors for consistency between training and inference across cells are difficult to be categorized to associated IDs by NW.

- If associated IDs are applicable across a group of cells, non-trivial NW burden is needed for the generation and management of the cell group information and the associated ID information.

As another issue, proprietary preservation is not likely to be guaranteed even though it is using associated ID to implicitly represent the NW side additional condition. If the associated ID is a globally unique ID, it implies the information of linkage across gNBs - that means if a number of gNBs indicate the same associated ID to the UE side, it discloses that these gNBs have the same additional condition (RF module type, deployment scenario, etc.) and, therefore, the network planning information (e.g., what type of RF modules are deployed at which locations in a large geographic area) is exposed.

Observation 2: For UE side data collection for the UE-side model in CSI prediction, there is risk of proprietary disclosure especially if the associated IDs are applicable for a group of cells.

## 2.2 Necessity of introducing associated ID for consistency

Generalization performance

In RAN1#118 meeting, the following high-level observations of generalization performance of CSI prediction were provided.

Observation (#118) Based on the evaluation for CSI prediction, the following high-level observations are provided: - ... - From the perspective of generalization over various scenarios/configurations (e.g., various UE speed, deployment scenario, carrier frequency) that have been evaluated, compared to generalization Case 1 where the AI/ML model is trained with dataset subject to a certain scenario#B/configuration#B and applied for inference with a same scenario#B/configuration#B - For generalization Case 2 where the AI/ML model is trained with dataset from a different scenario#A/configuration#A, generalized performance may be achieved for some certain combinations of scenario#A/configuration#A and scenario#B/configuration#B but not for others. - For generalization Case 3 where the training dataset is constructed with data samples subject to multiple scenarios/configurations including scenario#B/configuration#B, generalized performance of the AI/ML model can be achieved.

According to the observations, the generalized performance over various scenarios can be achieved under generalization Case 3. Therefore, even if UE collects training data under different NW side additional conditions without any indications such as associated ID, a generalized model can still be trained at UE side by mixing the collected data, which can also achieve satisfying performance during inference under various NW side additional conditions.

Besides the above evaluated scenarios in Rel-19, CSI prediction performance may also be affected by other factors such as different TxRU mappings which can impact the beamforming of CSI-RS. Although this factor was not evaluated in CSI prediction, the generalization evaluations of CSI compression in Rel-18 can provide some insights. In CSI compression, an observation captured in TR 38.843 shows that the performance loss compared to the overfitting case (i.e., generalization Case 1) is less than 4.4%, when different TxRU mappings were considered and the model was trained with a mixed data set (i.e. generalization Case 3). Since both of CSI compression and CSI prediction utilize the channel information measured from CSI-RS as model input/label, the generalization performance over various TxRU mapping patterns could be extended to the CSI prediction case.

TR 38.843 #### 6.2.2.2 Generalization evaluations for CSI compression … Generalization over TxRU mappings For the generalization verification of AI/ML based CSI compression over various TxRU mappings, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain TxRU mapping#B and applied for inference with a same TxRU mapping#B, - … - For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-4.4% loss or positive gain) for TxRU mapping#B subject to any of [2,8,2], [4,4,2], and [8,2,2] from the perspective of the layouts of antenna ports, or subject to any of 8x8x2 and 2x8x2 from the perspective of the layouts of antenna element mapping, if the training dataset is constructed with data samples subject to TxRU mappings including TxRU mapping#B, as observed by 4 sources. - Minor loss (0%~-2%) are observed by 4 sources. - Moderate loss (-2.5%~-4.4%) are observed by 1 source. - Positive gains are observed by 1 source. …

According to the agreement of 118bis, two aspects are to be optionally evaluated: tilt angle and TXRU mapping.

Agreement (#118bis) For generalization evaluation to identify potential NW-side additional conditions to ensure consistency between training and inference, consider one or more of the following aspects, if evaluated: - [Optional evaluation] Various tilt angle (e.g., [102] degrees, [110] degrees) - [Optional evaluation] Various TXRU mapping - With TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (8x8x2) and (12 x 8 x 2) antenna elements) - Without TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (2x8x2) and (4 x 4 x 2) antenna elements, respectively) - Other configurations are not precluded Note: for other evaluation assumptions, reuse baseline of TR 38.843 Note: report how to map antenna port to antenna elements Note: report the backbone/structure of AI/ML model used

The training and inference data used for each configuration are illustrated in the table below. We consider different tilt angles between 102 degrees and 110 degrees, and the UE speed is set as 30km/h.

Table 1 Evaluation on generalization performance of different tilt angles

| Testing (metric: SGCS) | Training (tilt angle) |  |  |
| --- | --- | --- | --- |
|  | 102 degrees | 110 degrees | Mixed 102 degrees and 110 degrees |
| 102 degrees | 0.93 (Case 1) | 0.69 (Case 2) | 0.90 (Case 3) 3% loss vs Case 1 |
| 110 degrees | 0.66 (Case 2) | 0.92 (Case 1) | 0.91 (Case 3) 1% loss vs Case 1 |

From the evaluation results, it can be observed that there is minor performance gap between Case 1 and Case 3, i.e., generalized performance can be achieved under Case 3.

Observation 3: Generalized performance of the AI/ML models may be achieved in CSI prediction with a mixed data set (i.e., generalization Case 3).

- For the evaluation of generalization performance over tilt angles, generalization Case 3 trained with mixed {102 degrees and 110 degrees} and tested on 102 degrees/110 degrees has minor SGCS loss of 3%/1%, respectively, compared with the corresponding generalization Case 1 performance.

Comparison of consistency issues with other use cases

In AI/ML based beam management, models can be sensitive to various DL Tx beam or beam set/list configurations. Since the data sample is composed of a set of beams each of which is typically with narrow beam width, any changes of beam properties like beam ordering in the beam set or beam codebook may cause significant performance degradations for UE-side model. That is why there is observation at Rel-18 for the benefit of ensuring consistency. Hence, associated ID is supported for UE to assume consistency of a DL Tx beam or beam set/list between training and inference, as agreed in RAN1#118.

TR 38.843 #### 7.1.3 Beam management … For BM-Case1 and BM-Case2 with a UE-side AI/ML model, consistency / association of Set B beams and Set A beams across training and inference is beneficial from performance perspective. Note: Whether specification impact is needed is a separate discussion.

Agreement (#118) For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

In CSI prediction, unlike using analog beamforming with narrow beam width, CSI-RS can be transmitted with digital beamforming, and the beam width is larger than the beam management case due to much smaller number of antenna elements. Moreover, the data sample for CSI prediction could be derived from measuring a single CSI-RS rather than a beam set, so CSI prediction does not have a beam ordering issue. Thus, the diversity of data distribution for CSI prediction case is much less than the beam management case, and the generalization issue can be easily addressed by mixing the data set as mentioned earlier.

It was also observed in Rel-18 AI/ML based positioning that generalization performance will deteriorate across drops/clutter parameters. This is because positioning models learn the mapping between channel information (e.g., power information and/or timing information) and locations (direct positioning) or intermediate parameters such as LOS indicator or RTOA/RSTD (assisted positioning) of one specific area (e.g., one drop). The performance of fingerprint based positioning is sensitive to the variations of this mapping relationship, which is easily impacted by the channel propagation environment at different scenarios. For CSI prediction, on the other hand, it learns the changing trend of the channel (rather than the mapping relationship of channel information) subject to a short time scale (TTI level), within which the channel propagation environment will not have significant fluctuations.

Observation 4: In beam management and positioning, AI/ML models may be sensitive to some variations of NW side additional conditions, e.g., Tx codebooks, deployment scenarios, while these factors almost do not impact the generalization of CSI prediction model.

Based on the necessity analysis, the motivation for introducing associated ID or other NW side additional condition for the CSI prediction case is not clear.

Observation 5: The motivation of introducing associated ID to differentiate NW side additional conditions for CSI prediction is not clear.

Based on the above observations from Section 2.1 and Section 2.2, for the issue of ensuring consistency of training/inference, we think that the necessity of introducing associated ID or other indication of NW side additional condition for CSI prediction is not clear, and the feasibility of introducing such indications is questionable especially for the multiple cell case. Therefore, we make the following proposal:

Proposal 1: No need to introduce indications from NW (e.g., explicit indication or associated ID) for ensuring consistency between training/inference for CSI prediction.

## 2.3 Performance monitoring for consistency

In beam management discussions, performance monitoring for consistency was proposed in RAN1 #116bis.

Agreement (#116bis) Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B: - Opt1: Based on associated ID (Referring to AI 9.1.3.3) - FFS on what can be assumed by UE with the same associated ID across training and inference - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework - Opt 2: Performance monitoring based - FFS details - Other options are not precluded.

In our understanding, if UE has multiple models which are associated with different NW side additional conditions, considering that gNB can configure monitoring resources for performance monitoring at UE side, UE is able to obtain inputs and ground truth labels for its candidate models. UE can compare the performance of these models, and select the model that best matches the current NW side additional condition in an implementation manner. In this case, the consistency between training and inference can be ensured. Note that the NW side additional condition is mostly static, so after monitoring, UE side can map the cell ID with the corresponding models with implementation manner. Therefore, if a new UE accesses to the same cell, the UE side can find the corresponding best model without monitoring.

Proposal 2: The consistency can be achieved by performance monitoring at UE side with an implementation manner.

# Conclusions

According to the discussions, following observations and proposals are provided:

Observation 1: For the data collection for the UE-side model in CSI prediction, the massive number of impacting factors for consistency between training and inference across cells are difficult to be categorized to associated IDs by NW.

- If associated IDs are applicable across a group of cells, non-trivial NW burden is needed for the generation and management of the cell group information and the associated ID information.

Observation 2: For UE side data collection for the UE-side model in CSI prediction, there is risk of proprietary disclosure especially if the associated IDs are applicable for a group of cells.

Observation 3: Generalized performance of the AI/ML models may be achieved in CSI prediction with a mixed data set (i.e., generalization Case 3).

- For the evaluation of generalization performance over tilt angles, generalization Case 3 trained with mixed {102 degrees and 110 degrees} and tested on 102 degrees/110 degrees has minor SGCS loss of 3%/1%, respectively, compared with the corresponding generalization Case 1 performance.

Observation 4: In beam management and positioning, AI/ML models may be sensitive to some variations of NW side additional conditions, e.g., Tx codebooks, deployment scenarios, while these factors almost do not impact the generalization of CSI prediction model.

Observation 5: The motivation of introducing associated ID to differentiate NW side additional conditions for CSI prediction is not clear.

Proposal 1: No need to introduce indications from NW (e.g., explicit indication or associated ID) for ensuring consistency between training/inference for CSI prediction.

Proposal 2: The consistency can be achieved by performance monitoring at UE side with an implementation manner.

# References

- RP-242399 “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#105, Melbourne, Australia, September 9 - 12, 2024.