3GPP TSG-RAN WG1 Meeting #119	Tdoc R1-2409443

Orlando, US, November 18th – 22nd, 2024

Agenda Item:	9.1.2

Source:	Ericsson

Title:	AI/ML for Positioning Accuracy Enhancement

Document for:	Discussion, Decision

1	Introduction	2

2	Integrating AI/ML based positioning methods with existing protocol	3

2.1	Cases with AI/ML model hosted in the UE (Case 1)	3

2.2	Case with AI/ML model hosted in the TRP/gNB (Case 3a)	4

2.3	Case with AI/ML model hosted in the LMF (case 3b)	5

3	Signaling enhancement for model inputs	5

3.1	Sample-based vs path-based measurements	5

3.1.1	Sample-based measurements	6

3.1.1.1	Remaining issue for the definition of sample-based measurements	6

3.1.1.2	Value range for parameters {Nt, Nt', k}	9

3.1.1.3	Evaluation results for measurement report window placement	11

3.1.1.4	Issue of timing error when generating measurement	17

6.4.2.2	Generalization Aspects	20

3.1.1.5	Total-power PDP report for Rel-19	22

3.1.2	Challenges of using Rel-17 path-based measurement as model input	23

3.1.2.1	High receiver complexity with path-based measurement	25

3.1.2.2	Lack of robustness with path-based measurement	27

3.1.2.3	Processing bandwidth/capability issues	32

3.1.2.4	Performance degradation with reduced number of TRPs	34

3.1.2.5	Burden of higher signaling size with path-based measurements	38

3.2	Analysis of supporting phase information measurement	41

3.2.1	Inapplicability of Rel-18 carrier phase positioning	42

3.2.2	Issues with random initial phase	42

3.2.3	Difficulty to align phase measurements between model training and model inference	45

3.2.4	Signaling size vs positioning accuracy tradeoff analysis	46

3.2.4.1	Simple signaling size comparison based on the number of real numbers to signal	47

3.2.4.2	Signaling sizes based on simple direct sample timing bitmaps	49

3.3	Reference Time for timing info of PDP and DP	51

3.4	Additional information included in the measurement report	52

3.5	Impact of AI/ML model location for Case 3a	53

4	Model output of AI/ML assisted positioning	54

4.1	Details for reporting model output of AI/ML assisted positioning	54

4.1.1	LOS/NLOS indicator	54

4.1.1.1	Interpretation of LOS/NLOS indicator, benefits for reporting LOS/NLOS indication	54

4.1.1.2	Reporting of a of LOS/NLOS indicator based on model output	56

4.1.1.3	Reporting of a LOS/NLOS indicator as standalone or with timing measurements	57

4.1.1.4	Summary of LOS/NLOS indicator reporting	58

4.1.2	Timing information	59

4.1.3	On the need of multipath timing/power reporting for Case 3a	60

4.2	Indication of AI/ML assistance in generating measurement reports	61

5	Training data collection	61

5.1	Content of training data samples	61

5.1.1	Time stamp of training data samples	62

5.1.2	Signalling and quality reporting for Part A	65

5.1.3	Signalling and quality reporting for Part B	66

5.1.4	Overhead reduction for label delivery	69

5.2	Metadata for training dataset	69

5.2.1	Elements in metadata of the training dataset	69

5.2.2	Assistance data for consistency between training and inference	71

6	Model performance monitoring	73

6.1	Principles for model performance monitoring	73

6.2	Model performance monitoring metric calculation for Case 1	75

6.3	Signaling to support decision making related to model monitoring	77

7	Model inference	78

7.1	Signaling to support model inference	78

7.2	Consistency aspects in general	79

7.2.1	Consistency of validity area	79

7.2.2	Consistency of RS configuration between model inference and model training	81

7.2.3	Consistency of network-side additional conditions between model inference and model training	81

7.2.4	Consistency of measurements reported between model inference and model training	83

7.3	Consistency of network side conditions for Case 1	83

8	Conclusion	86

9	References	94

10	Appendix A. High accuracy uncertainty and confidence as defined in TS 23.032	94

11	Appendix B. Model performance monitoring approaches	95

11.1.1	Monitoring approach 1 (Label-based): Opportunistic LoS links	96

11.1.2	Monitoring approach 2 (Label-based) for assisted AI/ML positioning	99

11.1.3	Monitoring approach 3 (Label-free): ML model self-monitoring	101

# Introduction

In RAN#105 meeting, the WID was revised for Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface [5].

The objective of the normative work is copied below. In this contribution, we focus on the normative work for the use case of positioning accuracy enhancements.

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2 - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024. - CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025 - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

# Integrating AI/ML based positioning methods with existing protocol

In order to specify enhancements to enable AI/ML based positioning, a framework issue to consider is how to integrate the AI/ML based methods with existing protocol. Different phases of AIML operations will map to different procedures and it is important to understand what is already available and what must be introduced to the specifications. Even if RAN2 is the driving force for such new high level procedure, RAN1 needs to understand these procedures to go forward.  In our RAN2 contribution, we present our view for AI/ML positioning integration into the positioning framework.  At a high level, the LCM procedures for each case are model inference, model monitoring and data collection.

## Cases with AI/ML model hosted in the UE (Case 1)

Case 1 is a UE based positioning case. The 3GPP positioning framework currently supports two methods for UE based positioning, namely DL-AoD and DL-TDOA based positioning. Additionally, these methods can be initiated either by the network (LMF) via MT LR procedures, or by the UE via MO LR procedures. To support Case 1, an existing procedure  can be reused or a completely new procedure with associated AD transfer, capability transfer and location request procedures can be designed.

The template for specifying Case 1 should be decided by RAN WG, choosing between existing DL-TDOA, DL-AoD or a new procedure as the baseline procedure.

In our view, it should be sufficient for Case 1 to take DL-TDOA as the base positioning method for inference, data collection and monitoring.  The DL-TDOA procedures can be further extended to cater to AIML positioning Case 1 specific needs if needed.

RAN1 assumes Case 1 is supported using UE based DL-TDOA procedures as a foundation. It is up to RAN2 to decide whether a set of new procedure is to be defined for Case 1, or extension to DL-TDOA suffices. RAN1 confirms the understanding with RAN2.

## Case with AI/ML model hosted in the TRP/gNB (Case 3a)

In RAN1#118, uplink positioning for case 3a was discussed with the following outcome:

Conclusion From RAN1 perspective, for Case 3a measurements, - The existing procedures can be reused in terms of SRS configuration. - Note: parameter values for SRS configuration can be further discussed - These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures. - Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

Even though some measurement reporting has been agreed, it was not clarified what legacy method should be used as a baseline for Case 3a.  For this RAN1 should consider the following:

- The Rel-18 study item phase of the AI/ML positioning work focused on time-based methods.

- Positioning methods and enhancements introduced in Rel-18 study item have not been investigated thoroughly for specification.

Based on the lack of studies on angle-based methods, it is reasonable to focus only on time-based positioning methods that use the reported AI/ML model output as model input, i.e. UL-DTOA and Multi-RTT. Thus in our view, the above agreement and conclusion are sufficient for defining model output types of assisted positioning.  Other types of measurements, such as angle measurements, were not thoroughly evaluated in Rel-18 SI and should not be considered for Rel-19.

Based on the agreements, RAN1 should assume that the procedures for UL-RTOA and multi RTT should be the basis for Case 3a.  Some of the signalling requests and reports in the procedure may need further extension to properly support the training and operation of an AI model in the gNB/TRP.

RAN1 assumes Case 3a is supported using UL-TDOA or multi RTT  procedure as a foundation. It is up to RAN3 to decide whether a set of new procedure is to be defined for Case 3a, or extension to existing procedure suffices. RAN1 confirms the understanding with RAN2 and RAN3.

## Case with AI/ML model hosted in the LMF (case 3b)

For model inference of Case 2b and 3b, the LMF requests measurements and runs its own model.

For Case 3b, the framework in RAN3 is flexible enough to be reused. However, for sample based reporting, the gNB  behaviour should be clarified, i.e. how the LMF requests sample-based measurements.  From the RAN1 perspective, it should be sufficient to assume a separate procedure, specific to Case 3b, is used to support sample-based measurements. How the procedure is implemented is up to RAN3.

RAN1 assumes Case 3b with sample-based reporting should be supported using a separate procedure from legacy UL RTOA and gNB RxTxTimeDiff reporting for measurement reporting for model input. RAN1 confirm the understanding with RAN2 and RAN3.

# Signaling enhancement for model inputs

In this Section, we investigate the issues related to model input. We first note that, for the following two positioning cases with UE-side model:

- (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

- (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

the measurement generation process is collocated with the AI/ML models. The exact details of the model inputs are largely implementation issues. There is no need to specify model input for model inference stage. Whether model input need any specification work for training data collection can be further discussed.

For Case 1/2a (i.e., UE-side model), channel measurement generation process is collocated in the same node with the AI/ML models. The exact details of the model inputs are implementation issues at least for model inference.

For the two direct AI/ML positioning cases:

- (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

- (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

the measurements obtained at one node (i.e., gNB or UE) need to be signaled to a second node (i.e., LMF) that applies the AI/ML models. Thus model input need to be specified at least for Case 3b/2b, and there is a need to clarify the model input formats.

For Case 3b (1st priority) and Case 2b (2nd priority), due to the LMF sided model, there is a need to clarify the target model input formats to be supported for the AI/ML positioning models.

For the 1st priority Case 3a, the need to specify channel measurements for model input depends on the node that hosts the AI/ML model, as discussed in section 3.7. If the need is confirmed by RAN3, the same design choices made for Case 2b/3b can be applied to Case 3a as well. Thus RAN1 can proceed with the model input discussion on Case 2b/3b.

## Sample-based vs path-based measurements

In RAN1#116, it was agreed to investigate the way timing information was conveyed, i.e., sample-based vs path-based measurements:

Agreement In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values. - Whether the same Alternative(s) applies to all cases or not - Applicability and necessity of specifying the Alternative(s) to different cases - Note: different sub-cases may have different issues. Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation.

Evaluation results show that, compared to path-based measurements, sample-based measurements

- requires lower receiver implementation complexity in the measurement node,

- is more robust to measurement reports from different measurement node implementations,

- achieves better positioning accuracy and

- requires lower signaling overhead for sending the measurement report.

For Cases that require sending measurement reports over standardized interfaces for inference (Case 2b/3b and potentially 3a), sample-based measurements are clearly preferred over path-based measurements, due to the lack of robustness and generalizability of path-based measurements, and its higher signaling overhead.

Even for Cases where measurements and inferences are performed by the same node (Case 1/2a and potentially 3a), sample-based measurements are preferred due to the higher receiver complexity and potentially reduced bandwidth processing capabilities associated with the path-based measurements.

Thus sample-based measurements should be supported for Rel-19 AI/ML based positioning. As a compromise, legacy path-based measurement can be supported as well. In this case, an indicator needs to be included in the measurement request.

For Rel-19 AI/ML based positioning Case 3b/2b, the measurement report of (PDP, DP) support sample-based measurement.

For Rel-19 AI/ML based positioning Case 3b, LMF includes an indicator in the measurement request to gNB, where the indicator is on the LMF preferred measurement report: either (a) sample measurement, or (b) legacy path-based measurement.

### Sample-based measurements

#### Remaining issue for the definition of sample-based measurements

In RAN1#117 and RAN1#118, the following agreements were made on the definition of sample-based measurement and path-based measurement.

Agreement Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed). Agreement For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples - The Nt samples have timing granularity T. - FFS: the starting time of the list of Nt samples - FFS: the value range of Nt For the sample-based measurement (if accepted in Rel-19), - For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power. - Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation. Agreement For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa.

During RAN1#118bis, the following options were discussed for the starting time of the sample window:

Proposal 2.1.1.4-1 For the definition of sample-based measurement, for an estimated channel response, RAN1 study the following options for down-selection for determining the starting time of the list of Nt samples: - Option A. the starting time of the list of Nt samples is the timing of the first detected path (quantized with timing granularity T). - Option B. the starting point of the Nt consecutive samples is determined by a fixed offset relative (quantized with timing granularity T) to the reference time. - Proponent companies provide details on the value of the fixed offset. - Option C. the starting point of the Nt consecutive samples is determined by a configured offset (quantized with timing granularity T) relative to the reference time. - Proponent companies provide details on the value range and signaling details of the configured offset. Note: since Nt' samples are reported with a timing granularity T and relative to a reference time, the starting time of the list of Nt samples also follows timing granularity T and relative to the reference time.

In our view, for Case 3b (and 3a if applicable), the only remaining issues for sample-based measurement is to define the starting point of the list of Nt consecutive samples. This can be resolved by: (a) defining the timing grid for generating the samples; (b) determining the starting sample on the timing grid for the list of Nt consecutive samples. In the following, the procedure is outlined for determining the Nt’ samples.

Step 1: Define the timing grid using (a) the reference time Tref (seconds) and (b) the timing granularity of T (seconds), where T = 2k * Tc;

Note that here the reference time Tref only anchors the grid points, i.e., Tref is on the grid where consecutive grid points are spaced by T (sec). Tref is not intended to set the boundary of the measurement report window. In general, numerous grid points may exist before and after Tref. One comment in RAN1#118bis was that the starting time for the reference TRP need not be quantized.  This assumes that for cases based on DL PRS reception, the starting time for the reference TRP is used as reference time. In general, the reference time can be placed freely (i.e., no quantization grid exists before the reference time is defined), and the grid is hereafter anchored to the reference.  This is reflected by the bullet in the agreement above: “The timing information is defined relative to a reference time”.

For NG-RAN side measurements (i.e., Case 3a and 3b), the reference time is the existing UL RTOA reference time T0+tSRS as defined in TS 38.215, considering that this reference time has been agreed for Case 3b.

Step 2: Determine the starting point T0,meas_window of the sequence of Nt consecutive samples according to the timing grid laid down in Step 1. Thus, T0,meas_window corresponds to a grid point, and the neighboring samples are spaced by T (sec).

Together (a) the starting point T0,meas_window and (b) a sequence of Nt consecutive samples spaced by T (sec) define the measurement report window. That is, the measurement report window is:

[T0,meas_window, T0,meas_window + (Nt -1)*T]	(sec).

For NG-RAN side measurements (i.e., Case 3a and 3b), our evaluations show that T0,meas_window can be simply the first detected path, which is quantized onto the timing grid laid down in Step 1.

Step 3: Select Nt’ samples from the sequence of Nt consecutive samples based on a selection rule.

For NG-RAN side measurements (i.e., Case 3a and 3b), the selection rule was agreed in RAN1#118, i.e., the Nt’ selected samples are expected to be those with the highest power.

To illustrate the procedures above, two examples are shown in Figure 1 and Figure 2.

Figure 1. An example of generating channel measurement samples, where the start time of measurement report window (T0,meas_window) is later than the reference time (Tref).  From the measurement report window, Nt’=9 samples are selected from Nt=16 consecutive samples for determining model input.

Figure 2. An example of generating channel measurement samples, where the start time of measurement report window (T0,meas_window) is earlier than the reference time (Tref).  From the measurement report window, Nt’=9 samples are selected from Nt=16 consecutive samples for determining model input.

The procedure described in Step 1-3 above applies to all cases when model input is needed. The difference is in how to set the parameters in the procedure, for example, uplink and downlink measurements use different reference time Tref. Thus this procedure can be adopted to support sample-based measurements for all cases. Furthermore, as shown by evaluation results in 3.1.1.3, the list of Nt consecutive samples can start at the first detected path without any performance loss, and without any negative ramification from including a path detection step in the sample-based measurement (e.g., no sensitivity to channel estimator algorithm).

Option B and C in the RAN1#118bis proposal both include fixing the starting point of sample detection window, either by configuration, or using a constant (fixed) offset. If the offset is configured, or fixed, the network will have to make a conservative decision and select an offset located earlier than the earliest time possible based on the deployment.  Since the information for finger-printing the channel between the UE and TRP is  in the inter-arrival time between the different paths (samples), and the earliest detectable sample is up to the UE implementation, we think that the captured sample set of non-zero samples in option B and C will be the same samples as in option A, if the window size Nt is set large enough. thus the starting point set in option B and C may only lead to more zero-power samples, while requiring larger Nt value to capture the same set of N’t samples.

Moreover, in our view, the starting point of the list of Nt samples may be different for different TRP-UE links, due to their different propagation time. Thus for Option B and C, NTRP offsets (either fixed or configured) need to be provided for NTRP TRP-UE links, increasing the burden of specification or configuration. If considering the alternative of defining a single offset to cover NTRP TRP-UE links, then the window size Nt need to be increased substantially to capture the N’t samples for each of the NTRP link.

Thus Option B and C are much less efficient than Option A, and it is very difficult to configure or define the parameter values properly for them.

Option B and option C may not result in additional information available for reporting.

For the definition of sample-based measurement, the timing grid for the Nt consecutive samples is determined by the reference time Tref (seconds), together with the timing granularity T.

The starting time of the Nt consecutive samples is the timing of the first detected path, quantized to the timing grid (option A in Proposal 2.1.1.4-1 from RAN1#118bis).

#### Value range for parameters {Nt, Nt', k}

Another open issue in the agreed definition of sample-based measurement is the value range of parameters Nt, Nt' and k for sample-based measurement.

For Case 3b, parameters Nt, Nt' and k are selected by LMF and sent to gNB in the measurement request, and the gNB provide measurement report based on the {Nt, Nt', k} values by LMF. The value range of parameters {Nt, Nt', k} should not be excessive, because the LMF need to collect data and train a different model for different parameter settings, for example, a different model for each combination of {Nt, Nt', k}.

The evaluation results in Rel-18 study item phase as well as Rel-19 show that the maximum value of Nt can be set as 64, since it shows no tangible performance degradation compared to 128. The minimum value of Nt can be set as 9, the same as the maximum number of paths in the existing measurement report. Between 9 and 64, power-of-2 values 16 and 32 can be adopted for Nt as well.

For Nt', Nt'<= Nt can be supported, i.e., Nt'= Nt is allowed for smaller value of Nt. Also, when Nt'= Nt, the selection rule is equivalently avoided, and all samples in the range of Nt are accepted. For larger values of Nt, Nt'< Nt is followed to save signaling overhead.

In summary, the following parameter combinations can be supported:

  - For Nt = 9: {Nt, Nt'} = {9, 9};

  - For Nt = 16: {Nt, Nt'} = {16, 16}, {16, 9};

  - For Nt = 32: {Nt, Nt'} = {32, 16}, {32, 9};

  - For Nt = 64: {Nt, Nt'} = {64, 32}, {64, 16}, {64, 9};

The exact {Nt, Nt'} value to configure may depends on the deployment environment (e.g., heavily NLoS or not), system parameters including SRS configuration (e.g., carrier frequency, SRS bandwidth), and the k values for timing granularity. Exactly how to select the {Nt, Nt'} value can be up to LMF implementation.

In terms of k values, for legacy measurement report for DL-TDOA and multi-RTT,

- Up to Rel-17, the set of possible k values are: {0…5}. That is, the timing measurement granularity ranges from Tc (i.e., 0.51 ns) to 32×Tc (i.e., 18.75 ns).

- In Rel-18, new ReportingGranularityfactor values are added for Carrier Phase Positioning: {-6 … -1}. That is, the new timing measurement granularity ranges from Tc/64 (i.e., 0.008 ns) to Tc/2 (i.e.,0.25 ns).

Note: Tc is the basic NR timing unit, and Tc = 1/ (Δ𝑓max ⋅ Nf) = 1/(480*103*4096) = 0.5086 (ns).

According to the existing specification (see extracted texts for DL-TDOA below), the Rel-16 ReportingGranularityfactor and Rel-18 timingReportingGranularityFactorExt are not used at the same time, i.e., only one is included.

TS 37.355 v18.2.0 NR-DL-TDOA-ReportConfig-r16 ::= SEQUENCE { maxDL-PRS-RSTD-MeasurementsPerTRP-Pair-r16 INTEGER (1..4) OPTIONAL, -- Need ON timingReportingGranularityFactor-r16 INTEGER (0..5) OPTIONAL, -- Need ON ..., [[ measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 ENUMERATED { n0, n2, n3, n4, n6, n8, ... } OPTIONAL, -- Need ON reducedDL-PRS-ProcessingSamples-r17 ENUMERATED { requested, ... } OPTIONAL, -- Need ON lowerRxBeamSweepingFactor-FR2-r17 ENUMERATED { requested } OPTIONAL -- Need ON ]], [[ timingReportingGranularityFactorExt-r18 INTEGER (-6..-1) OPTIONAL, -- Need ON nr-DL-PRS-MeasurementTimeWindowsConfig-r18 NR-DL-PRS-MeasurementTimeWindowsConfig-r18 OPTIONAL -- Need ON ]] } timingReportingGranularityFactor, timingReportingGranularityFactorExt This field specifies the recommended reporting granularity for the DL RSTD measurements. Value (0..5) corresponds to (k0..k5) and value (-6..-1) corresponds to (kMinus6..kMinus1) used for nr-RSTD and nr-RSTD-ResultDiff in NR-DL-TDOA-MeasElement. The UE may select a different granularity value for nr-RSTD and nr-RSTD-ResultDiff. The timingReportingGranularityFactorExt should not be included by the location server and shall be ignored by the target device if timingReportingGranularityFactor is included.The timingReportingGranularityFactor should not be included by the location server and shall be ignored by the target device if timingReportingGranularityFactorExt is included..

For AI/ML based positioning, vast majority of companies used k=4 (i.e., sampling period = 16×Tc) in Rel-18 study item [4]. The Rel-18 evaluations demonstrated that the AI/ML based positioning was able to achieve sub-meter positioning accuracy with k=4.

Clearly, the ultra-fine timing granularity introduced for Rel-18 CPP are excessive and unnecessary for AI/ML positioning. There is no justification to duplicate the signaling for CPP over to AI/ML based positioning even though k={-6,..,-1} can continue to be used for measurement reporting of legacy methods DL-TDOA and multi-RTT.

Even for the 6 values {0..5} up to Rel-17, there is no justification to support all 6 values. Having a wide range of k values complicates the AI/ML LCM. For each k values, the full set of AI/ML procedures have to be supported separately, including training data collection, model training, model monitoring, model update, etc.

In our view, it is preferrable to support a small set of k values in Rel-19, which is the first release to adopt AI/ML based positioning. In fact, it may be sufficient to support one k values in Rel-19, k=4, which has been thoroughly evaluated in Rel-18 study item by numerous participating companies. If more options are needed considering FR1/FR2 and different PRS bandwidth, it can be expanded to support four k values, for example, k={2,3,4,5}. Future releases can consider more options of k values if proven necessary for the desired positioning accuracy.

For Case 3b (and 3a if needed), support {Nt, Nt'} parameter combinations: {9, 9}; {16, 16}, {16, 9}; {32, 16}, {32, 9}; {64, 32}, {64, 16}, {64, 9}.

For Case 3b (and 3a if needed), a small number of k values, e.g., k={4}, k={3,4}, or k={2,3,4,5}, is supported for timing reporting granularity factor in Rel-19.

#### Evaluation results for measurement report window placement

In this section, we investigate the placement of the measurement report window, specifically focusing on the starting time of the window. Our objective is to identify the proper window of size Nt from which to select Nt' samples for generating model input. Assume τ1 represents the first detected path identified by a channel searcher/estimator, and let δ be parameter (e.g., 3) that allows a shift of the size-Nt window, relative to τ1.  Mt denotes the sample scope used as input by the ML model, which typically corresponds to the IDFT size. Consequently, the window begins at the sample index

S=maxτ1-δ,0,

and ends at the sample index

E=minS+Nt-1,Mt.

In the following sub-sections, we evaluate the impact of the δ values, specifically δ=0 and δ=3 , on the model's performance when the ML model utilizes input from the 9, 6, and 4 TRPs with the strongest received signal power. Throughout these evaluations, the UE transmission power is maintained at 23 dBm, and the SRS bandwidth is set to 100 MHz. Moreover, we employ seven different channel estimators to capture varying levels of accuracy in determining τ1 (i.e., the first detected path), providing a comprehensive assessment of the model's performance for different measurement report window placement. Detailed information about the various channel estimators can be found in Section 3.1.2.2. We also provide the performance bound, where the Nt’ samples with the highest power are selected from the IDFT window of size Mt, i.e., without any degradation due to the choice of the starting position of the list of Nt samples.

One of the focus area is to investigate whether it is beneficial to start the Nt samples earlier than the first detected path. There was some concern at RAN1#118 that some information could be lost if starting at the first detected path, causing performance degradation. Thus, it is evaluated below whether (B) brings better performance than (A):

- δ=0, i.e., the list of Nt consecutive samples starts at the first detected path, rounded to the timing grid.

- δ=3, i.e., the list of Nt consecutive samples starts 3 samples before the timing of the first detected path to capture preceding data.

As can be observed from the evaluation results in the sub-sections, (A) provides the same or better performance compared to (B), for all 7 channel estimators tested. Even though different channel estimator may set the start of Nt samples somewhat differently, the sample-based measurement outlined in Step 1-3 in section 3.1 is robust to such implementation variations. In fact, for the purpose of setting the start of Nt samples, there is no need to implement sophisticated channel estimator to pinpoint the first detected path. It is acceptable to use a less accurate (but simpler and cheaper to implement) channel estimator, since the AI/ML model does not rely on accurately detecting the first path to triangulate the UE, unlike legacy methods.

Thus RAN1 is recommended to adopt δ=0, i.e., there is no need to introduce any new offset parameters like δ and RAN1 should simply set the start of Nt samples at the timing of the first detected path.

The positioning accuracy of the AI/ML model is not sensitive to channel estimation method when the timing of the first detected path is used to determine the start of Nt samples.

Setting d = 0, i.e., starting the Nt samples at the timing of the first detected path, provides the same or better accuracy compared to d = 3 , i.e., starting the Nt samples 3 samples before the timing of the first detected path.

The size Nt can be <=1/2 the measurement range Mt without significantly degrading the positioning accuracy.

Setting Nt=Nt' leads to clear performance degradation when the number of active TRPs is small (e.g., 4 TRPs), while performance degradation is minor when the number of active TRPs is adequate (e.g., 6 or 9 TRPs).

##### Performance comparison of δ=0 vs δ=3 for a range of active TRPs

###### 9 TRPs

Table 1 and Table 2 resent the evaluation results for measurement report window placement with  δ=0 and δ=3, respectively. The ML model uses input from the 9 TRPs with the highest received signal power. The “Sample-based signaling” column (green highlighted) represents the performance bound, independent of the starting position of the list of Nt consecutive samples. For this column, the Nt’ samples with the highest power are selected from the IDFT window of size Mt.

Table 1 90%tile 2D positioning accuracy (meters) of PDP models using direct sample-based reporting for the {60%, 6m, 2m} InF-DH scenario. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6]. For the models trained on the sample-based reports, the same channel estimator was used to determine the first detected path, τ1, for the training and inference, using seven different channel estimation algorithms.

| Test cases |  |  | 90%tile 2D positioning errors [m] based on 100 MHz RS bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mt | Nt' | Nt | Sample based signaling | Sample-based signaling with δ=0 |  |  |  |  |  |  |  |
|  |  |  |  | Average across estimators | Different channel estimators to determine τ1 |  |  |  |  |  |  |
|  |  |  |  |  | G | F | E | D | C | B | A |
| 128 | 16 | 64 | 1.00 | 1.01 | 1.03 | 1.00 | 1.00 | 1.02 | 1.01 | 1.01 | 1.00 |
|  |  | 32 |  | 1.07 | 1.06 | 1.10 | 1.07 | 1.09 | 1.07 | 1.05 | 1.07 |
|  |  | 16 |  | 1.20 | 1.21 | 1.23 | 1.23 | 1.21 | 1.17 | 1.15 | 1.18 |
| 64 | 16 | 32 | 1.07 | 1.11 | 1.13 | 1.12 | 1.11 | 1.10 | 1.09 | 1.08 | 1.11 |
|  |  | 16 |  | 1.22 | 1.25 | 1.24 | 1.23 | 1.22 | 1.20 | 1.22 | 1.19 |
| 64 | 9 | 32 | 1.27 | 1.24 | 1.24 | 1.24 | 1.24 | 1.29 | 1.24 | 1.23 | 1.21 |
|  |  | 16 |  | 1.27 | 1.27 | 1.25 | 1.28 | 1.30 | 1.28 | 1.26 | 1.24 |
|  |  | 9 |  | 1.37 | 1.38 | 1.38 | 1.34 | 1.38 | 1.36 | 1.37 | 1.35 |

Table 2 90%tile 2D positioning accuracy (meters) of PDP models using direct sample-based reporting for the {60%, 6m, 2m} InF-DH scenario. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6]. For the models trained on the sample-based reports, the same channel estimator was used to determine the first detected path, τ1, for the training and inference, using seven different channel estimation algorithms.

| Test cases |  |  | 90%tile 2D positioning errors [m] based on 100 MHz RS bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mt | Nt' | Nt | Sample based signaling | Sample-based signaling with δ=3 |  |  |  |  |  |  |  |
|  |  |  |  | Average across estimators | Different channel estimators to determine τ1 |  |  |  |  |  |  |
|  |  |  |  |  | G | F | E | D | C | B | A |
| 128 | 16 | 64 | 1.00 | 1.01 | 1.02 | 1.00 | 1.02 | 1.04 | 1.02 | 1.01 | 1.00 |
|  |  | 32 |  | 1.07 | 1.09 | 1.07 | 1.07 | 1.04 | 1.08 | 1.07 | 1.08 |
|  |  | 16 |  | 1.19 | 1.18 | 1.20 | 1.20 | 1.19 | 1.19 | 1.18 | 1.17 |
| 64 | 16 | 32 | 1.07 | 1.09 | 1.10 | 1.09 | 1.09 | 1.10 | 1.07 | 1.09 | 1.09 |
|  |  | 16 |  | 1.22 | 1.23 | 1.20 | 1.25 | 1.20 | 1.26 | 1.21 | 1.19 |
| 64 | 9 | 32 | 1.27 | 1.23 | 1.24 | 1.26 | 1.21 | 1.21 | 1.23 | 1.25 | 1.22 |
|  |  | 16 |  | 1.25 | 1.25 | 1.24 | 1.25 | 1.25 | 1.24 | 1.27 | 1.25 |
|  |  | 9 |  | 1.36 | 1.37 | 1.35 | 1.36 | 1.35 | 1.41 | 1.36 | 1.34 |

###### 6 TRPs

Table 3 and Table 4 resent the evaluation results for measurement report window placement with  δ=0 and δ=3, respectively. The ML model uses input from the 6 TRPs with the highest received signal power. The “Sample-based signaling” column (green highlighted) represents the performance bound, independent of the starting position of the list of Nt consecutive samples. For this column, the Nt’ samples with the highest power are selected from the IDFT window of size Mt.

Table 3 90%tile 2D positioning accuracy (meters) of PDP models using direct sample-based reporting for the {60%, 6m, 2m} InF-DH scenario. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6]. For the models trained on the sample-based reports, the same channel estimator was used to determine the first detected path, τ1, for the training and inference, using seven different channel estimation algorithms.

| Test cases |  |  | 90%tile 2D positioning errors [m] based on 100 MHz RS bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mt | Nt' | Nt | Sample based signaling | Sample-based signaling with δ=0 |  |  |  |  |  |  |  |
|  |  |  |  | Average across estimators | Different channel estimators to determine τ1 |  |  |  |  |  |  |
|  |  |  |  |  | G | F | E | D | C | B | A |
| 128 | 16 | 64 | 1.38 | 1.32 | 1.34 | 1.31 | 1.33 | 1.31 | 1.31 | 1.35 | 1.30 |
|  |  | 32 |  | 1.45 | 1.44 | 1.46 | 1.49 | 1.47 | 1.47 | 1.43 | 1.42 |
|  |  | 16 |  | 1.71 | 1.73 | 1.77 | 1.73 | 1.73 | 1.72 | 1.64 | 1.66 |
| 64 | 16 | 32 | 1.36 | 1.47 | 1.50 | 1.48 | 1.47 | 1.46 | 1.43 | 1.47 | 1.48 |
|  |  | 16 |  | 1.73 | 1.76 | 1.74 | 1.71 | 1.76 | 1.71 | 1.72 | 1.71 |
| 64 | 9 | 32 | 1.65 | 1.63 | 1.62 | 1.64 | 1.63 | 1.65 | 1.64 | 1.63 | 1.65 |
|  |  | 16 |  | 1.81 | 1.80 | 1.90 | 1.84 | 1.79 | 1.78 | 1.80 | 1.79 |
|  |  | 9 |  | 1.94 | 2.01 | 1.99 | 1.96 | 1.96 | 1.84 | 1.91 | 1.91 |

Table 4 90%tile 2D positioning accuracy (meters) of PDP models using direct sample-based reporting for the {60%, 6m, 2m} InF-DH scenario. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6]. For the models trained on the sample-based reports, the same channel estimator was used to determine the first detected path, τ1, for the training and inference, using seven different channel estimation algorithms.

| Test cases |  |  | 90%tile 2D positioning errors [m] based on 100 MHz RS bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mt | Nt' | Nt | Sample based signaling | Sample-based signaling with δ=3 |  |  |  |  |  |  |  |
|  |  |  |  | Average across estimators | Different channel estimators to determine τ1 |  |  |  |  |  |  |
|  |  |  |  |  | G | F | E | D | C | B | A |
| 128 | 16 | 64 | 1.38 | 1.33 | 1.35 | 1.32 | 1.31 | 1.36 | 1.31 | 1.34 | 1.35 |
|  |  | 32 |  | 1.47 | 1.48 | 1.46 | 1.46 | 1.47 | 1.45 | 1.49 | 1.50 |
|  |  | 16 |  | 1.72 | 1.76 | 1.73 | 1.72 | 1.72 | 1.71 | 1.71 | 1.72 |
| 64 | 16 | 32 | 1.36 | 1.46 | 1.47 | 1.48 | 1.42 | 1.51 | 1.45 | 1.46 | 1.44 |
|  |  | 16 |  | 1.73 | 1.78 | 1.76 | 1.77 | 1.72 | 1.75 | 1.68 | 1.69 |
| 64 | 9 | 32 | 1.65 | 1.66 | 1.68 | 1.67 | 1.68 | 1.59 | 1.65 | 1.70 | 1.69 |
|  |  | 16 |  | 1.80 | 1.82 | 1.84 | 1.82 | 1.78 | 1.80 | 1.78 | 1.80 |
|  |  | 9 |  | 2.07 | 2.09 | 2.09 | 2.07 | 2.05 | 2.13 | 2.05 | 2.03 |

###### 4 TRPs

Table 5 and Table 6 resent the evaluation results for measurement report window placement with  δ=0 and δ=3, respectively. The ML model uses input from the 4 TRPs with the highest received signal power. The “Sample-based signaling” column (green highlighted) represents the performance bound, independent of the starting position of the list of Nt consecutive samples. For this column, the Nt’ samples with the highest power are selected from the IDFT window of size Mt.

Table 5 90%tile 2D positioning accuracy (meters) of PDP models using direct sample-based reporting for the {60%, 6m, 2m} InF-DH scenario. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6]. For the models trained on the sample-based reports, the same channel estimator was used to determine the first detected path, τ1, for the training and inference, using seven different channel estimation algorithms.

| Test cases |  |  | 90%tile 2D positioning errors [m] based on 100 MHz RS bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mt | Nt' | Nt | Sample based signaling | Sample-based signaling with δ=0 |  |  |  |  |  |  |  |
|  |  |  |  | Average across estimators | Different channel estimators to determine τ1 |  |  |  |  |  |  |
|  |  |  |  |  | G | F | E | D | C | B | A |
| 128 | 16 | 64 | 1.91 | 1.99 | 1.92 | 2.04 | 2.09 | 1.94 | 1.89 | 2.03 | 1.98 |
|  |  | 32 |  | 2.27 | 2.29 | 2.27 | 2.33 | 2.32 | 2.20 | 2.23 | 2.21 |
|  |  | 16 |  | 2.72 | 2.85 | 2.79 | 2.69 | 2.75 | 2.69 | 2.63 | 2.64 |
| 64 | 16 | 32 | 2.10 | 2.25 | 2.30 | 2.29 | 2.26 | 2.27 | 2.24 | 2.25 | 2.18 |
|  |  | 16 |  | 2.75 | 2.76 | 2.80 | 2.83 | 2.70 | 2.71 | 2.73 | 2.74 |
| 64 | 9 | 32 | 2.62 | 2.56 | 2.53 | 2.52 | 2.58 | 2.53 | 2.60 | 2.58 | 2.55 |
|  |  | 16 |  | 2.88 | 2.97 | 2.91 | 2.92 | 2.95 | 2.85 | 2.84 | 2.75 |
|  |  | 9 |  | 3.26 | 3.29 | 3.43 | 3.30 | 3.29 | 3.22 | 3.08 | 3.21 |

Table 6 90%tile 2D positioning accuracy (meters) of PDP models using direct sample-based reporting for the {60%, 6m, 2m} InF-DH scenario. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6]. For the models trained on the sample-based reports, the same channel estimator was used to determine the first detected path, τ1, for the training and inference, using seven different channel estimation algorithms.

| Test cases |  |  | 90%tile 2D positioning errors [m] based on 100 MHz RS bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mt | Nt' | Nt | Sample based signaling | Sample-based signaling with δ=3 |  |  |  |  |  |  |  |
|  |  |  |  | Average across estimators | Different channel estimators to determine τ1 |  |  |  |  |  |  |
|  |  |  |  |  | G | F | E | D | C | B | A |
| 128 | 16 | 64 | 1.91 | 1.97 | 1.95 | 1.98 | 1.95 | 1.93 | 2.08 | 1.92 | 1.99 |
|  |  | 32 |  | 2.29 | 2.25 | 2.34 | 2.22 | 2.32 | 2.22 | 2.27 | 2.38 |
|  |  | 16 |  | 2.77 | 2.82 | 2.81 | 2.82 | 2.72 | 2.79 | 2.63 | 2.78 |
| 64 | 16 | 32 | 2.10 | 2.26 | 2.22 | 2.33 | 2.20 | 2.30 | 2.25 | 2.27 | 2.26 |
|  |  | 16 |  | 2.81 | 2.73 | 2.82 | 2.76 | 2.89 | 2.90 | 2.72 | 2.84 |
| 64 | 9 | 32 | 2.62 | 2.56 | 2.55 | 2.58 | 2.52 | 2.60 | 2.62 | 2.52 | 2.54 |
|  |  | 16 |  | 2.91 | 2.87 | 2.91 | 2.98 | 2.89 | 2.89 | 2.96 | 2.89 |
|  |  | 9 |  | 3.46 | 3.69 | 3.49 | 3.46 | 3.42 | 3.46 | 3.35 | 3.40 |

##### Train with one channel estimator, test with a different estimator

In this subsection, it is tested whether the sample-based measurement suffers from sensitivity of path detection method, if the start of Nt samples require a path detection step. Recall that path-based measurement suffers from the lack of robustness with channel estimator algorithm, as analyzed in details in section 3.1.2.2.

Analogous to the evaluation results shown in Figure 11 for path-based measurement, the same exercise is done for sample-based measurement, with the results plotted in Figure 3. This shows a clear contrast in the model performance.

  - Figure 11 demonstrates that path-based measurement requires that the model input is generated using the same channel estimator between training data collection and model inference. Otherwise, the model suffers from a varying amount of performance degradation.

Figure 3 demonstrates that sample-based measurement as described in section 3.1.1.1 is robust to channel estimator algorithm. The model does not suffer any performance degradation even when different channel estimator is used between training data collection and model inference for determining the start of Nt samples.

This evaluation results confirms that the sample-based measurement has a big advantage over path-based measurement. For both training data collection stage and model inference stage, there is no need to consider the impact from different vendors’ implementation of channel estimation algorithm.

Unlike path-based measurement, sample-based measurement is robust to channel estimation algorithm, even though the start of Nt consecutive samples is determined using the first detected path.

Figure 3 CDF of 2D positioning errors of a Mt=64, Nt'=9 and Nt=9 model trained on PDP sample reports with the first detected path,  τ1, from channel estimator G and tested on sample reports with the first detected path,  τ1, from different channel estimators in the {60%, 6m, 2m} InF-DH scenario.

##### Selected Nt’ samples from the list of Nt consecutive samples

In the following, two exemplary channel measurements are shown, one for a LoS link between a simulated TRP-UE link, the other for a NLoS link. The selected Nt’ samples from the list of Nt consecutive samples are illustrated.

The two versions of Nt’ samples of channel measurements are:

(a). Select Nt’ samples with the highest power from the IDFT window of size Mt. This corresponds to the performance bound of sample-based measurement, whose performance are green-highlighted in section 3.1.1.3.1.

(b). Select Nt’ samples with the highest power from the list of Nt consecutive samples, where the list starts with the first detected path (i.e., d=0). The Mt-Nt' samples in the IDFT window but are not selected are set to zero to illustrate the measurements as seen by the AI/ML model.

The examples demonstrate that (b), i.e., the method described in section 3.1.1.1, does shrink the channel measurements, sometimes substantially, as shown in Figure 5. However, the evaluation results in section 3.1.1.3.1 shows that the AI/ML model shows little or no performance degradation when Nt is sufficiently large (e.g., Nt >=16) and enough TRPs are active (e.g., >=6 active TRPs). Thus, the method described in section 3.1.1.1 can be viewed as an efficient way to perform feature engineering, which extract the most important finger-printing information from the channel measurements.

| - Select Nt’ samples in IDFT window | - Select Nt’ samples from the list of Nt samples starting with the first detected path |
| --- | --- |

Figure 4. Nt’ samples selected using two methods for a representative LoS link. Mt=128, Nt= 32, Nt'=16.

| - Select Nt’ samples in IDFT window | - Select Nt’ samples from the list of Nt samples starting with the first detected path |
| --- | --- |

Figure 5. Nt’ samples selected using two methods for a representative NLoS link. Mt=128, Nt= 32, Nt'=16.

#### Issue of timing error when generating measurement

One issue that have been debated by companies is the impact of timing error when measurements are generated. Two issues were suggested:

Issue (1): Timing error can cause two sample-based measurements to be distinct from each other, which leads to ambiguity in sample-based measurement report. When the model is fed with sample-based measurements with different timing errors, the model cannot perform well.

Issue (2): Path-based measurement is robust to timing error in measurement generation, while sample-based measurement is sensitive to timing error.

In the following sub-sections, we provide analysis and evaluation results to clarify why these issues are in practice not seen.

##### Issue (1): timing error causes devastating ambiguity in sample-based measurement

Regarding issue (1), it is true to observe that timing error causes the receiver to generate distinct sample-based measurements.

- In Figure 6 (a), we plot the TD PDP of a three-tap channel with delays at 92.97, 112.26  151.65 ns (or, equivalently at 11.42, 13.79, 18.64 taps at 122.88 MHz sampling rate) and gains of [0.63+3.83j, -3.38+4.27j, 0.047-0.93j] ×10-7.

- In Figure 6 (b), we plot the TD PDP of the same three-tap channel but with −3.8 ns timing error.

- In Figure 6 (c), we plot the TD PDP of the same three-tap channel but with +18.7 ns timing error.

It can be observed that,

- The shapes of continuous PDP responses remain the same and are just shifted in time according to the timing errors.

- The discrete PDP samples however appear to be rather different. For instance,

  - The four strongest samples in Figure 6 (a) have the magnitudes of 3.5, 2.1, 2.1, 5.3×10-7 at sample #11, 12, 13, 14.

  - The four strongest samples in Figure 6 (b) have the magnitudes of 1.5,  3.6, 4.4, 3.6×10-7 at sample #10, 11, 13, 14.

  - The four strongest samples in Figure 6 (c) have the magnitudes of 2.6, 3.1, 5.1, 2.2×10-7 at sample #13, 14, 16, 17.

While the PDP samples appear different with different timing errors, they are conveying the same magnitude and time shift information as the continuous PDP response though at coarse levels (caused by finite sampling rates). Nonetheless, evaluations have been performed in Rel-18 study item stage, where it was shown by several companies that generalization capability is achievable by providing the proper training dataset containing timing errors to train the ML models. On the other hand, if the model is not trained to handle time error, then positioning accuracy is affected.

(a) no timing error

(b) −3.8 ns timing error					(c) +18.7 ns timing error

Figure 6 PDP samples with or without timing errors.

As an example, R1-2306454 [6] provided the following evaluation results for sample-based measurements, which shows that the model can be trained to have generalization capability over timing error. Even though the sample-based measurement appears different with different timing error, the AI/ML model is able to learn from the training dataset and extract the correct information to position the UE. Indeed, random timing error can be artificially injected to the training data samples as a data augmentation strategy, even if the collected training data samples do not contain a wide range of timing errors.

R1-2306454: Table 45. 90%tile 2D positioning accuracy using PDP inputs for different UE synchronization STD [ns] in the {60%, 6m, 2m} InF-DH scenario for small model (Model I) trained with different UE synchronization STD [ns].

| Model details | Positioning approach | Model complexity [# paras] | Computational complexity [FLOPs] | 90%tile 2D positioning error [m] with different UE timing error STD in {60%, 6m, 2m} InF-DH |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 0 ns | 5 ns | 10 ns | 25 ns | 50 ns |
| PDP trained with 0 ns | Cent. Assist. | 0.36 M | 9 M | 0.586 | 0.747 | 1.281 | 6.76 | 20.081 |
|  | Cent. Direct | 0.36 M | 9 M | 0.571 | 0.754 | 1.354 | 6.886 | 20.350 |
| PDP trained with 25 ns | Cent. Assist. | 0.36 M | 9 M | 0.703 | 0.711 | 0.711 | 0.748 | 1.303 |
|  | Cent. Direct | 0.36 M | 9 M | 0.534 | 0.538 | 0.543 | 0.562 | 1.041 |
| PDP trained with 50 ns | Cent. Assist. | 0.36 M | 9 M | 0.762 | 0.768 | 0.762 | 0.776 | 0.810 |
|  | Cent. Direct | 0.36 M | 9 M | 0.567 | 0.571 | 0.566 | 0.577 | 0.589 |

Based on the evaluation results summarized in TR 38.843 below, the model should be trained with a dataset with t1 > t2, where t1 is UE/gNB RX and TX timing error for the training dataset, t2 is the timing error at deployment.

TR 38.843 v2.0.1 #### 6.4.2.2 Generalization Aspects Observations: Direct AI/ML positioning … For direct AI/ML positioning, based on evaluation results of timing error in the range of 0-50 ns, when the model is trained by a dataset with UE/gNB RX and TX timing error t1 (ns) and tested in a deployment scenario with UE/gNB RX and TX timing error t2 (ns), for a given t1, - For a case evaluated by a given source, the positioning accuracy of cases with t2 smaller than t1 is better than the cases with t2 equal to t1. For example, - For the case of (t1, t2)=(50ns, 30ns), evaluation results show the positioning error of (t1, t2)=(50ns, 30ns) is 0.82~0.86 times that of (t1, t2)=(50ns, 50ns). - For the case of (t1, t2)=(50ns, 0ns), evaluation results show the positioning error of (t1, t2)=(50ns, 0ns) is 0.80~0.82 times that of (t1, t2)=(50ns, 50ns). - For a case evaluated by a given source, the positioning accuracy of cases with t2 greater than t1 is worse than the cases with t2 equal to t1. The larger the difference between t1 and t2, the more the degradation. For example, - For the case of (t1, t2)=(0ns, 10ns), evaluation results show the positioning error of (t1, t2)=(0ns, 10ns) is 1.25~18.7 times that of (t1, t2)=(0ns, 0ns). - For the case of (t1, t2)=(0ns, 50ns), evaluation results show the positioning error of (t1, t2)=(0ns, 50ns) is 3.5~18.3 times that of (t1, t2)=(0ns, 0ns). Note: here the positioning error is the horizonal positioning error (meters) at CDF=90%. … AI/ML assisted positioning … For AI/ML assisted positioning with timing information (e.g., ToA) as model output, based on evaluation results of timing error in the range of 0-50 ns, when the model is trained by a dataset with UE/gNB RX and TX timing error t1 (ns) and tested in a deployment scenario with UE/gNB RX and TX timing error t2 (ns), for a given t1, - For a case evaluated by a given source, the positioning accuracy of cases with t2 smaller than t1 is better than the cases with t2 equal to t1. For example, - For the case of (t1, t2)=(50ns, 20~25ns), evaluation results submitted to RAN1#113 show the positioning error of (t1, t2)=(50ns, 20~25ns) is 0.75~1.00 times that of (t1, t2)=(50ns, 50ns). - For the case of (t1, t2)=(50ns, 0ns), evaluation results submitted to RAN1#113 show the positioning error of (t1, t2)=(50ns, 0ns) is 0.76~0.99 times that of (t1, t2)=(50ns, 50ns). - For a case evaluated by a given source, the positioning accuracy of cases with t2 greater than t1 is worse than the cases with t2 equal to t1. The larger the difference between t1 and t2, the more the degradation. For example, - For the case of (t1, t2)=(0ns, 10ns), evaluation results submitted to RAN1#113 show the positioning error of (t1, t2)=(0ns, 10ns) is 1.34~5.43 times that of (t1, t2)=(0ns, 0ns). - For the case of (t1, t2)=(0ns, 20~25ns), evaluation results submitted to RAN1#113 show the positioning error of (t1, t2)=(0ns, 20~25ns) is 5.66~13.0 times that of (t1, t2)=(0ns, 0ns). - For the case of (t1, t2)=(0ns, 50ns), evaluation results submitted to RAN1#113 show the positioning error of (t1, t2)=(0ns, 50ns) is 10.62~51.52 times that of (t1, t2)=(0ns, 0ns). Note: here the positioning error is the horizonal positioning error (meters) at CDF=90%.

##### Issue (2): Path-based measurement is robust to timing error, but sample-based measurement is not

As can be observed from R1-2306454 Table 45 (quoted in previous sub-section), sample-based measurement can be trained to be robust to timing error, as long as a proper training dataset is provided.

We further provide new evaluation results in Table 7 for both

- direct sample-based reporting, and

- Rel-17 path-based reporting from channel estimation algorithm F (described in Section 3.1.2.2)

with two reporting configurations:

- Nt=128 and Nt'=16

- Nt=64 and Nt'=9.

The evaluation assumes 18 TRP, 100 MHz RS bandwidth, and {60%, 6m, 2m} InF-DH. In a first evaluation scenario where the models using either reporting approaches are trained with datasets containing no UE timing errors, the model performance is found to deteriorate when tested with samples containing UE timing errors.

- For both direct sample-based reporting and Rel-17 path-based reporting, the trained models maintain 90%-tile positioning errors below 1 m for UE timing error STD up to 10 ns.

- For both direct sample-based reporting and Rel-17 path-based reporting, the 90%-tile positioning errors of the trained models deteriorate to around 1.2 m for UE timing error STD of 25 ns.

- For both direct sample-based reporting and Rel-17 path-based reporting, the 90%-tile positioning errors of the trained models exceed 10 m for UE timing error STD of 50 ns.

In a second set of evaluation scenarios where the models using either reporting approaches are trained with datasets containing UE timing errors, the trained models are found to maintain positioning accuracy up to the level of UE timing errors contained in the training datasets.

- When trained with datasets containing UE timing error STD of 25 ns, models using both direct sample-based reporting and Rel-17 path-based reporting maintains <1 m 90%-tile positioning errors for test UE timing error STD up to 25 ns. However, for both, the 90%-tile positioning errors for test UE timing error STD of 50 ns degrade to above 1 m.

- When trained with datasets containing UE timing error STD of 50 ns, models using both direct sample-based reporting and Rel-17 path-based reporting maintains <1 m 90%-tile positioning errors for test UE timing error STD up to 50 ns.

In a third set of evaluation scenarios where the models using either reporting approaches are trained with datasets containing samples with and without UE timing errors, the trained models are found to achieve high positioning accuracy across the test cases with no, low or high UE timing errors.

For these extensive evaluation results, it can be observed that training/test timing error distributional mismatch has the same impact on both direct sample-based reporting and Rel-17 path-based reporting. When there is no or low degrees of distributional mismatch, models using either reporting approach can maintain high positioning accuracy. When there are high degrees of distributional mismatch, positioning accuracy of models using either reporting approach deteriorates with the degrees of distributional mismatch. There is no special timing error robustness advantage for one reporting approach over the other.

Training/test timing error distributional mismatch has the same impact on both direct sample-based reporting and Rel-17 path-based reporting.

Table 7. 90%tile 2D positioning accuracy (meters) of PDP models using direct sample-based reporting or Rel-17 path-based reporting from channel estimation algorithm F for the {60%, 6m, 2m} InF-DH scenario. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6].

| Nt | Nt' | Reporting approach | UE timing error STD in train dataset | 90%tile 2D positioning error [m] with different UE timing error STD in {60%, 6m, 2m} InF-DH |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 0 ns | 5 ns | 10 ns | 25 ns | 50 ns |
| 128 | 16 | path | 0 ns | 0.701 | 0.701 | 0.753 | 1.198 | 10.427 |
| 128 | 16 | sample | 0 ns | 0.666 | 0.689 | 0.732 | 1.235 | 11.614 |
| 128 | 16 | path | 25 ns | 0.735 | 0.744 | 0.748 | 0.782 | 1.079 |
| 128 | 16 | sample | 25 ns | 0.738 | 0.742 | 0.752 | 0.767 | 1.103 |
| 128 | 16 | path | 50 ns | 0.811 | 0.826 | 0.821 | 0.836 | 0.906 |
| 128 | 16 | sample | 50 ns | 0.791 | 0.803 | 0.810 | 0.802 | 0.865 |
| 128 | 16 | path | 0, 25 ns | 0.692 | 0.702 | 0.696 | 0.741 | 1.109 |
| 128 | 16 | sample | 0, 25 ns | 0.649 | 0.655 | 0.650 | 0.707 | 1.076 |
| 128 | 16 | path | 0, 50 ns | 0.706 | 0.703 | 0.702 | 0.740 | 0.814 |
| 128 | 16 | sample | 0, 50 ns | 0.688 | 0.701 | 0.688 | 0.710 | 0.788 |
| 128 | 16 | path | 0, 25, 50 ns | 0.694 | 0.693 | 0.695 | 0.712 | 0.808 |
| 128 | 16 | sample | 0, 25, 50 ns | 0.642 | 0.645 | 0.653 | 0.670 | 0.765 |
| 64 | 9 | path | 0 ns | 0.856 | 0.897 | 1.026 | 2.694 | 14.193 |
| 64 | 9 | sample | 0 ns | 0.822 | 0.837 | 0.926 | 2.205 | 13.831 |
| 64 | 9 | path | 25 ns | 0.968 | 0.987 | 0.996 | 1.053 | 1.821 |
| 64 | 9 | sample | 25 ns | 0.939 | 0.949 | 0.959 | 1.042 | 1.730 |
| 64 | 9 | path | 50 ns | 1.131 | 1.162 | 1.159 | 1.174 | 1.298 |
| 64 | 9 | sample | 50 ns | 1.037 | 1.038 | 1.046 | 1.068 | 1.129 |
| 64 | 9 | path | 0, 25 ns | 0.904 | 0.915 | 0.923 | 1.058 | 1.940 |
| 64 | 9 | sample | 0, 25 ns | 0.804 | 0.810 | 0.821 | 0.913 | 1.717 |
| 64 | 9 | path | 0, 50 ns | 0.936 | 0.957 | 0.984 | 1.046 | 1.225 |
| 64 | 9 | sample | 0, 50 ns | 0.859 | 0.864 | 0.875 | 0.917 | 1.067 |
| 64 | 9 | path | 0, 25, 50 ns | 0.876 | 0.906 | 0.903 | 0.954 | 1.144 |
| 64 | 9 | sample | 0, 25, 50 ns | 0.815 | 0.824 | 0.824 | 0.866 | 1.007 |

#### Total-power PDP report for Rel-19

For a UE or a gNB equipped with multiple receive antenna ports, sources had investigated tradeoffs of using the following two different PDP input options during the SI phase:

- total-power PDP (summed over all antenna ports)

pd≜ahad2

- Multi-port PDP (measurement at each receive antenna port individually)

pa[d]≜had2

It is noted that the reference signals defined for positioning (DL PRS, UL positioning SRS) is single-port at transmit antenna. Hence 'multi-port' in this discussion refers to receive antenna ports only.

One set of evaluation results are copied in Table 8 where the 90%tile positioning errors of using 2-port PDP or the summed PDP are compared. It can be observed that

- The 2-port PDP input type doubles the signal sizes, requires higher computational complexity, and achieves marginal performance improvements when compared to the total PDP input type.

- Using only 1-port to compute PDP discards signal power and radio channel information that is readily available at the measurement node, which causes lower positioning accuracy when compared to the total PDP input type. The positioning accuracy degradation ranges from 11% with very large training datasets to 20% with smaller training datasets.

When compared to the total-power PDP input type, the 2-port PDP input type (1) doubles the signal sizes; (2) requires higher computational complexity; and (3) achieves marginal performance improvements.

When compared to the total-power PDP input type, the 1-port PDP input type (1) discards signal power and radio channel information that is readily available, and (2) achieves lower positioning accuracy.

Table 8 90%tile 2D positioning accuracy (meters) using summed total-power PDP or 2-port PDP inputs for the small models and different training dataset sizes in the {60%, 6m, 2m} InF-DH scenario. Nt=Nt'=256. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6].

| Input | Model complexity [# parameters] | Computational complexity [FLOPs] | 90%tile 2D positioning error [m] with different train set sizes in {60%, 6m, 2m} InF-DH |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 80,000 | 40,000 | 20,000 | 10,000 |
| total-power PDP | 0.36 M | 9 M | 0.43 | 0.51 | 0.66 | 0.86 |
| 2-port PDP | 0.37 M | 11 M | 0.40 | 0.50 | 0.64 | 0.83 |
| 1-port PDP | 0.36 M | 9 M | 0.47 | 0.58 | 0.76 | 1.04 |

It can be concluded that the performance gains of using multi-port PDP do not justify the multiple increases in signaling sizes; and 1-port PDP discards useful information unnecessarily. Therefore, Rel-19 PDP reports should be based on PDP samples containing sample powers from all antenna ports.

Furthermore, the nonzero value positions of the DP report type should be determined based on PDP samples containing sample powers summed from all RX antenna ports.

Accordingly, definition of DP and PDP measurements in 38.215 should be made according to the above for AI/ML positioning.

For the model input types for Case 3b and 3a (if applicable), consider input based on DP or PDP samples containing sample powers summed over all receive antenna ports, i.e., total-power PDP.

### Challenges of using Rel-17 path-based measurement as model input

We showed

- PDP sampled at regular sampling grid points, and

- (revised) RSRPP reports at estimated channel tap delays (not necessarily on the regular sampling grid)

are both sub-samples of the time domain PDP samples, there are nonetheless significant complexity and statistical differences between them. Most notably,

- PDP sub-samples taken from a regular sampling grid points according to strongest powers are straightforward computation from directly observable TD CIR.

  - The potential ambiguity between samples collected during data collection/training and samples reported for inference can be easily removed. A procedure can be consistently implemented by the reporting nodes (UE or gNB) for both data collection and inference, if there is a clear requirement or selection criteria on the power calculation. To secure consistency, a clear RAN4 requirement on the sample RSRPP should be defined, as discussed in Section 2.1.1.2.  Since the timing info for each sample is quantized to a predefined sample grid, there is no ambiguity for the choice of sample timing.

- For path based reporting, RSRPP reports depend heavily on the UE’s capability to estimate the true channel tap delays accurately as well as the UE’s capability to interpolate between directly observable TD CIR samples well.

  - This dependency in choosing the right path time-of-arrival creates potential ambiguity between data collected during training and reports received during inference.

- The Rel-17 path-based reporting impose substantially higher computation complexity loads on the measurement nodes to prepare such reports.

While both PDP sub-samples from regular sampling grid points and legacy path-based reports are both sub-samples of the TD PDP samples, there are statistical differences between them that may impact AI/ML model performance. Most notably, the former is obtained via straightforward computation, but the latter depend heavily on the UE’s channel estimation and interpolation capabilities. The Rel-17 path-based reporting impose substantially higher computation complexity loads on the measurement nodes than the sample-based reporting approach.

AI/ML models taking PDP sub-samples from a regular sampling grid points according to strongest powers had been extensively studied by the majority of sources during the SI phase as evidently documented in the TR 38.843. On the other hand, there has been no systematic study on the use of (off-grid) RSRPP reports as AI/ML model inputs in 3GPP. Particularly, the issues of accurate modeling of UE’s channel estimation and interpolation capabilities should be studied to check the feasibility of using such reports as AI/ML model inputs. At the minimum, the reported delays should be from an actual tap delay estimator operating at the correct SNR. For instance, a UE is not expected to find as accurate or as many tap delays for a low SNR (e.g., -5 dB) link to a far-away TRP than for a high SNR (e.g., 30 dB) link to a close-by TRP. Moreover, the configured reference signal (PRS or SRS) bandwidth is inversely related to the path timing detection accuracy, i.e., it significantly affects the achievable accuracy of the reported delays. Thus, without correct modeling of reference signal detection at the receiver, the performance of (off-grid) RSRPP reports as AI/ML model inputs can be exaggerated and there will be model input distributional mismatch between training and inference phases.

There has been no systematic study on the use of (off-grid) RSRPP reports as AI/ML model inputs in 3GPP. Without correct modeling of UE channel estimation and interpolation capabilities, the performance of (off-grid) RSRPP reports as AI/ML model inputs can be exaggerated.

If the UE receiver implementation issues are not studied, there will be model input distributional mismatch between training and inference phases, when different UEs with different receiver implementation are used in training vs inference.

When studying the feasibility of (off-grid) RSRPP reports as AI/ML model inputs, the investigation should be based on at least realistic modeling of UE’s channel estimation capabilities. The reported per-path delays should be from an actual path delay estimator operating at the correct SNR.

In legacy measurement report of RSRPP together with per-path delay, there was no requirement on how to determine the reported values. Thus, different UE implementations are likely to produce different {per-path delay, per-path power} for a same radio channel. The observed channel measurement is the composite result of transmitter, radio channel, and receiver. Even if the transmitter and the radio channel stay constant, the UE receiver implementation differences will cause the observed channel measurements to vary from UE to UE. This causes the fingerprint of the radio channels of a given location to vary from UE to UE.

Consider the simplified example, where model training of LMF-side model uses channel measurements produced by UE#1, and model inference uses channel measurements from UE#2 to determine UE#2 location. If the radio channel fingerprint is different between UE#1 and UE#2 for a same location due to different UE receiver implementation, the LMF-side model cannot be expected to find the correct location for UE#2.

In the remaining of this section, we present extensive investigation on the following issues when comparing sample-based and path-based signaling:

- Measurement node receiver complexity (section 3.1.2.1)

- ML model robustness with measurement reports from different measurement nodes (section 3.1.2.2)

- Measurement node bandwidth processing capabilities (section 3.1.2.3)

- Positioning accuracy vs signaling size tradeoff (section 3.1.2.5)

#### High receiver complexity with path-based measurement

For direct sample-based reporting, as illustrated in the upper branch of Figure 7, the receiver performs common and low complexity processing of the received reference symbols to generate the reporting samples.

In contrast, the Rel-17 path-based reporting requires the receiver to perform additional high complexity processing of the received signals. Because the locations of the true channel taps are not directly observable, their estimates can be obtained by employing a path searcher and channel estimator. In general, a channel estimator searches L estimated paths with complex gains ca,ll and delays dll to minimize the differences to the measured FD CIR samples:

argminca,l,dll kHak-l=0L-1ca,lexp-j2πdlNFFTk2

or to minimize the differences to the measured TD CIR samples:

argminca,l,dll dhad-NNFFTl=0L-1ca,lexp-jπd-dlNFFTsincNd-dlNFFTsincd-dlNFFT2

The estimated complex gains and delays are then used to interpolate into the channel responses as required by the Rel-17 RAN4 path definition:

hadp=NNFFTl=0L-1ca,lexp-jπdp-dlNFFTsincNdp-dlNFFTsincdp-dlNFFT

Clearly, this is a set of much more computationally intensive workload than simply truncating or selecting the samples with highest powers for the sample-based reporting approach.

Figure 7 Receiver processing flow comparison between direct sample-based reporting and Rel-17 path-based reporting schemes.

Furthermore, in order to control the computational complexity, channel estimation algorithms from different vendors are expected to be different based on the respective tradeoff between the target channel estimate accuracy and hardware/software computation constraints. The following lists a few design choice dimensions:

- Sophistication level of noise floor estimation

- Iterative or one-shot estimation

- Successive cancellation of found paths

- Linear or nonlinear estimation

- How much super-resolution is employed

In short, the channel estimation algorithm is up to implementation; alignment among all vendors is not possible. It is then questionable that an ML model trained with the path-based reports from one channel estimation algorithm can perform as reliably when used on path-based reports from a different channel estimation algorithm.

The effect of using two specific realizations of channel estimator is provided in Figure 8 to further the robustness discussion. In this example, the true channel tap locations are

[35.8, 39.6, 42.7, 42.8, 46.6, 48.3, 52.5, 52.6, 53.1, 54.5, 54.7, 56.7, 60.4, 62.9, 64.7, 66.2, 73.6].

Two channel estimators are applied to the same channel in Figure 8 where different choices are made for the third bullet above (i.e., whether to apply successive cancellation of found paths) while holding other design choices the same: (a) a channel estimator without successive cancelation of found paths; and (b) a channel estimator with successive cancelation of found paths.

- For channel estimator choice (a), it was able to find the path clustered around 42.7. But the true channel tap at 39.6 was missed because it was overshadowed by the sidelobes of the 42.7 cluster. Similarly, the two true channel taps at 46.6 and 48.3 was treated as one path.

- For a channel estimator choice (b), it was able to separate the path at 39.6 from the 42.7 cluster. Similarly, both paths at 46.6 and 48.3 are found by this more capable channel estimator.

Note further that the estimated channel coefficients are closer to true channel coefficients when such successive cancelation of found paths is used in the estimation process. For instance, compare the estimated magnitudes for the taps found around 47 or those around 63. It can also enable more accurate path delay estimation. For instance, it can be observed that the delay of the first path is estimated more accurately with successive cancelation of found paths.

It should be clear such successive cancellation or other equivalent techniques for improving channel estimation accuracy require substantial increase in processing complexity for preparing the measurement reports.

Rel-17 path-based reporting requires much higher computational complexity for the measurement node than the sample-based reporting approach, particularly when more capable and accurate channel estimation algorithms are used.

| (a). Using a channel estimator without successive cancelation of found paths | - (b). Using a channel estimator with successive cancelation of found paths |
| --- | --- |

Figure 8 Comparison of estimated path delays from (a) a channel estimator without successive cancelation of found paths and (b) one with successive cancelation.

#### Lack of robustness with path-based measurement

Based on the above discussion, it becomes paramount to investigate whether an ML model trained with the path-based reports from one channel estimation algorithm can perform as reliably when used on path-based reports from a different channel estimation algorithm.

Toward this end, we prepare a dataset of the path delay reports from seven different channel estimation algorithms. A basic characterization of their behaviors is provided in Figure 9 which shows the CDF of the number of detect paths by the seven different channel estimators for the 1000x18 links between 1000 random UE locations and the 18 TRPs in the {60%, 6m, 2m} InF-DH scenario.

- The true number of channel taps generated by the 3GPP channel model is around 25. But some of these taps can be very close to each other (see an example discussed below) and not distinguishable even with a 100 MHz reference signal bandwidth.

- The number of paths found by the channel estimators depends on

  - How close different true channel taps are

  - The SNR of the link

  - The computational complexity budgeted for the channel estimators

  - Sophistication level of the channel estimation algorithms, see the design choice dimensions in section 3.1.2.1.

Note that, for channel estimators having similar overall found tap CDF, their behavior on a specific input channel impulse response can be different. For instance, channel estimator B and G have basically identical overall CDF for the number of detected taps as shown in Figure 9. But as can be observed from their joint PDF shown in Figure 10, the two channel estimators more often find different number of paths.

Figure 9 CDF of number of detect paths by the seven different channel estimators for the 1000x18 links between 1000 random UE locations and the 18 TRPs in the {60%, 6m, 2m} InF-DH scenario.

Figure 10 Joint PDF of number of detect paths for channel estimator B and G for the 1000x18 links between 1000 random UE locations and the 18 TRPs in the {60%, 6m, 2m} InF-DH scenario.

ML positioning models trained on the path reports from these seven channel estimators can achieve high positioning accuracy when applied to path reports from the same channel estimators the models were trained on. The 90%tile 2D positioning accuracy values of the Rel-17 path-based PDP models trained and tested on reports from the same channel estimators are compared to those of the sample-based PDP models in Table 9.

The columns of the Rel-17 path-based model accuracy are sorted based on the channel estimators’ tendency to find less or more paths as shown in Figure 9. That is, the curves from left to right in Figure 9 are corresponding to the columns from left to right in Table 9. This enables the following general observations.

- All these models using Rel-17 path-based reporting can achieve similar 2D positioning accuracy as models using sample-based reporting.

- Models using Rel-17 path-based reports from more capable channel estimators (i.e., capable of finding more channel taps) can generally achieve better performance than those using reports from less capable models.

Positioning accuracy of ML models using Rel-17 path-based reports, which are trained and tested with data generated by a specific measurement implementation, depends significantly on the signal processing capabilities of the measurement nodes.

Table 9. 90%tile 2D positioning accuracy (meters) of PDP models using direct sample-based reporting or Rel-17 path-based reporting from seven different channel estimation algorithms for the {60%, 6m, 2m} InF-DH scenario. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6]. For the models trained on the Rel-17 path-based reports, the same channel estimator was used for the training and inference.

| Test cases |  | 90%tile 2D positioning errors [m] based on 100 MHz RS bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nt | Nt' | Sample based signaling | Path-based signaling |  |  |  |  |  |  |  |
|  |  |  | Average across estimators | Different channel estimators |  |  |  |  |  |  |
|  |  |  |  | D | E | A | F | B | G | C |
| 256 | 16 | 0.69 | 0.70 | 0.83 | 0.78 | 0.77 | 0.68 | 0.69 | 0.57 | 0.61 |
| 128 | 16 | 0.67 | 0.70 | 0.82 | 0.77 | 0.73 | 0.70 | 0.65 | 0.65 | 0.60 |
| 64 | 16 | 0.75 | 0.82 | 0.88 | 0.86 | 0.78 | 0.86 | 0.76 | 0.87 | 0.70 |
| 256 | 9 | 0.82 | 0.80 | 0.85 | 0.82 | 0.80 | 0.78 | 0.79 | 0.80 | 0.77 |
| 128 | 9 | 0.81 | 0.78 | 0.83 | 0.80 | 0.78 | 0.79 | 0.76 | 0.77 | 0.74 |
| 64 | 9 | 0.82 | 0.83 | 0.90 | 0.87 | 0.79 | 0.86 | 0.77 | 0.87 | 0.74 |

While the models using Rel-17 path-based reports from the same channel estimators as their respective training set can achieve comparable 2D positioning accuracy as the models using sample-based reporting, such good positioning accuracy does not carry to Rel-17 path-based reports from different channel estimators. In Figure 11, we show the 2D positioning error CDF of a Nt=128 and Nt'=16 model trained on PDP path reports from channel estimator G and tested on path reports from different channel estimators.

- The model achieves 90%tile 2D positioning error of 0.65 m when tested using reports from channel estimator G.

- As discussed in the above, channel estimator B and G have basically identical overall CDF for found taps as shown in Figure 9. However, the 90%tile 2D positioning error of the model degrades to 1.17 m, which is an 80% increase in positioning error.

  - On the other hand, channel estimator C has substantially different number of detected paths than channel estimator G. But the 90%tile 2D positioning error of the model only degrades to 0.82 m.

- Furthermore, channel estimator A and F also have very similar overall CDF for found taps as shown in Figure 9. The model’s responses to the Rel-17 path-based reports from these channel estimators are quite different.

  - When tested on reports from channel estimator A, the 90%tile 2D positioning error of the model degrades to 1.54 m.

  - When tested on reports from channel estimator F, the 90%tile 2D positioning error of the model degrades to 0.79 m.

The above evaluation results show that, even setting aside the large dimensions and ranges of possible channel estimation algorithm design tradeoffs, channel estimators that yield similar number of detected paths can still lead to widely different ML model performance.

Figure 11 CDF of 2D positioning errors of a Nt=128 and Nt'=16 model trained on PDP path reports from channel estimator G and tested on path reports from different channel estimators in the {60%, 6m, 2m} InF-DH scenario.

While the evaluations above focused on path-based PDP as model input, the same experiment is performed below using path-based CIR as model input. The 90%tile 2D positioning accuracy values of the Rel-17 path-based 2-port CIR models trained and tested on reports from the same channel estimators are compared to those of the sample-based PDP models in Table 10. Compared to using the PDP path-based reports, ML models using the CIR path-based reports achieves a reduction of 4.4% in 90%tile 2D positioning errors though some channel estimators enable more improvements than the others.

In Figure 12, we show the 2D positioning error CDF of a Nt=128 and Nt'=16 model trained on 2-port CIR path reports from channel estimator G and tested on path reports from different channel estimators.

- The model achieves 90%tile 2D positioning error of 0.56 m when tested using reports from channel estimator G.

- As discussed in the above, channel estimator B and G have basically identical overall CDF for found taps as shown in Figure 9. However, the 90%tile 2D positioning error of the model degrades to 1.21 m, which is an 116% increase in positioning error.

  - On the other hand, channel estimator C has substantially different number of detected paths than channel estimator G. But the 90%tile 2D positioning error of the model only degrades to 0.70 m.

- Furthermore, channel estimator A and F also have very similar overall CDF for found taps as shown in Figure 9. The model’s responses to the Rel-17 path-based reports from these channel estimators are quite different.

  - When tested on reports from channel estimator A, the 90%tile 2D positioning error of the model degrades to 1.86 m.

  - When tested on reports from channel estimator F, the 90%tile 2D positioning error of the model degrades to 0.72 m.

Overall, compared to path-based PDP as model input, path-based CIR reports enable marginal positioning accuracy improvements (4.4% error reduction averaged across different channel estimators) while requiring quadruple signaling sizes. The ML models using CIR path reports are even more sensitive to channel estimator mismatch between training and inference phases.

Models trained using Rel-17 path-based reports prepared by one channel estimator do not generalize to reports from different channel estimators.

Given the large dimensions and ranges of possible channel estimation algorithm design tradeoffs and algorithm behaviors, fingerprinting type ML models do not generalize to measurement reports generated from different channel estimation implementations.

Models using path-based PDP and path-based CIR as model input are sensitive to channel estimator mismatch between training and inference phases, with higher sensitivity observed for path-based CIR.

Table 10. 90%tile 2D positioning accuracy (meters) of 2-port CIR models using direct sample-based reporting or Rel-17 path-based reporting from seven different channel estimation algorithms for the {60%, 6m, 2m} InF-DH scenario. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6]. For the models trained on the Rel-17 path-based reports, the same channel estimator was used for the training and inference.

| Test cases |  | 90%tile 2D positioning errors [m] based on 100 MHz RS bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nt | Nt' | Sample based signaling | Path-based signaling |  |  |  |  |  |  |  |
|  |  |  | Average across estimators | Different channel estimators |  |  |  |  |  |  |
|  |  |  |  | D | E | A | F | B | G | C |
| 256 | 16 | 0.68 | 0.68 | 0.80 | 0.74 | 0.79 | 0.65 | 0.71 | 0.50 | 0.60 |
| 128 | 16 | 0.64 | 0.67 | 0.76 | 0.74 | 0.72 | 0.64 | 0.64 | 0.56 | 0.59 |
| 64 | 16 | 0.70 | 0.78 | 0.85 | 0.79 | 0.80 | 0.77 | 0.73 | 0.75 | 0.75 |
| 256 | 9 | 0.82 | 0.76 | 0.81 | 0.78 | 0.81 | 0.74 | 0.76 | 0.73 | 0.71 |
| 128 | 9 | 0.82 | 0.73 | 0.78 | 0.78 | 0.76 | 0.72 | 0.72 | 0.70 | 0.68 |
| 64 | 9 | 0.80 | 0.79 | 0.86 | 0.79 | 0.80 | 0.81 | 0.76 | 0.77 | 0.73 |

Figure 12 CDF of 2D positioning errors of a Nt=128 and Nt'=16 model trained on 2-port CIR path reports from channel estimator G and tested on path reports from different channel estimators in the {60%, 6m, 2m} InF-DH scenario.

#### Processing bandwidth/capability issues

In this section, we investigate the dependency of path-based measurements on RS bandwidth and receiver processing capability.

In Table 9 and Table 10, we compare the 90%tile 2D positioning accuracy of models using direct sample-based reporting or Rel-17 path-based reporting assuming an RS bandwidth of 100 MHz. We found that

- For an RS bandwidth of 100 MHz, all models using Rel-17 path-based reporting can achieve similar 2D positioning accuracy as models using sample-based reporting. Models using Rel-17 path-based reports from more capable channel estimators (i.e., capable of finding more channel taps) can generally achieve better performance than those using reports from less capable models. On the other hand, when simpler and low computational complexity channel estimators are used for preparing the measurement reports, models using Rel-17 path-based reports can slightly underperform models using sample-based reporting.

On the basis of a same large RS bandwidth (e.g., 100 MHz), models using Rel-17 path-based measurements on average achieve similar positioning accuracy as models using sample-based measurements.

The performance variation of the path-based measurements depends on whether more capable / high computational complexity channel estimators or simpler / lower complexity channel estimators are used for preparing the path measurement reports.

In Table 11, we compare the 90%tile 2D positioning accuracy of models using direct sample-based reporting or Rel-17 path-based reporting assuming an RS bandwidth of 25 MHz. We observe that

- For an RS bandwidth of 25 MHz, models using sample-based reporting achieve better 2D positioning accuracy than models using Rel-17 path-based reporting. ML models using Rel-17 path-based reporting on average have 28% higher 90%tile 2D positioning error than ML models using sample-based reporting.

On the basis of a same small RS bandwidth (e.g., 25 MHz), ML models using Rel-17 path-based reporting on average have 26% higher 90%tile 2D positioning error than ML models using sample-based reporting.

Table 11. 90%tile 2D positioning accuracy (meters) of PDP models using direct sample-based reporting or Rel-17 path-based reporting from seven different channel estimation algorithms for the {60%, 6m, 2m} InF-DH scenario. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6]. For the models trained on the Rel-17 path-based reports, the same channel estimator was used for the training and inference.

| Test cases |  | 90%tile 2D positioning errors [m] based on 25 MHz RS bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nt | Nt' | Sample based signaling | Path-based signaling |  |  |  |  |  |  |  |
|  |  |  | Average across estimators | Different channel estimators |  |  |  |  |  |  |
|  |  |  |  | D | E | A | F | B | G | C |
| 64 | 16 | 0.81 | 0.99 | 1.22 | 1.13 | 0.98 | 1.00 | 0.89 | 0.94 | 0.77 |
| 32 | 16 | 0.84 | 1.09 | 1.26 | 1.26 | 1.00 | 1.19 | 0.90 | 1.18 | 0.86 |
| 16 | 16 | 1.20 | 1.83 | 2.18 | 2.10 | 1.46 | 2.20 | 1.41 | 2.15 | 1.32 |
| 64 | 9 | 1.00 | 1.00 | 1.20 | 1.14 | 0.98 | 0.99 | 0.92 | 0.93 | 0.84 |
| 32 | 9 | 0.99 | 1.10 | 1.29 | 1.20 | 1.02 | 1.22 | 0.94 | 1.19 | 0.87 |
| 16 | 9 | 1.29 | 1.81 | 2.14 | 2.16 | 1.47 | 2.07 | 1.41 | 2.11 | 1.31 |

However, because of the high computational complexity of path searching and channel estimation for preparing the Rel-17 path-based reports as discussed in Section 3.1.2.1, a measurement node with complexity constraints may not be able to process very wide RS bandwidth. This may limit the maximum PRS processing bandwidth capabilities that a UE may be able to report.

On the other hand, a measurement node capable of supporting a maximum carrier bandwidth of 100 MHz is definitely capable of performing FFT/IFFT on 100 MHz. Truncating and down-sampling the TD CIR/PDP/DP requires much lower complexity than path searching, channel estimation and channel interpolation. Hence, a complexity-constrained measurement node can support a wider RS bandwidth for sample-based reporting than for Rel-17 path-based reporting. It’s clear from signal processing theory, a larger transmitted and measured RS BW will enable higher positioning accuracy than a smaller transmitted and measured RS BW.

To provide a concrete example to demonstrate the issue, we compare 90%tile 2D positioning accuracy of

- ML models using sample-based reporting on 100 MHz RS bandwidth

- ML models using Rel-17 path-based reporting on 50 MHz RS bandwidth

- ML models using Rel-17 path-based reporting on 25 MHz RS bandwidth

The evaluation results are shown in Table 12. We can observe that ML models using Rel-17 path-based reporting underperform ML models using sample-based reporting significantly, which is the consequence of using a narrower bandwidth to detect the paths.

- ML models using Rel-17 path-based reporting on 50 MHz RS bandwidth on average have 20% higher 90%tile 2D positioning error than ML models using sample-based reporting on 100 MHz RS bandwidth.

- ML models using Rel-17 path-based reporting on 25 MHz RS bandwidth on average have 71% higher 90%tile 2D positioning error than ML models using sample-based reporting on 100 MHz RS bandwidth.

Given the same receiver computational complexity, a measurement node can support a wider RS bandwidth for sample-based reporting than for Rel-17 path-based reporting. The wider RS bandwidth measurement reports enable better positioning accuracy for the ML models using sample-based reporting.

Table 12. 90%tile 2D positioning accuracy Δ90% (meters) of PDP models using direct sample-based reporting or Rel-17 path-based reporting from seven different channel estimation algorithms for the {60%, 6m, 2m} InF-DH scenario. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6]. For the models trained on the Rel-17 path-based reports, the same channel estimator was used for the training and inference.

| Sample-based over 100 MHz |  |  | Path-based over 50 MHz |  |  |  |  | Path-based over 25 MHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nt | Nt' | Δ90% | Nt | Nt' | Δ90% |  |  | Nt | Nt' | Δ90% |  |  |
|  |  |  |  |  | Average | Worst | Best |  |  | Average | Worst | Best |
| 256 | 16 | 0.69 | 128 | 16 | 0.79 | 0.96 | 0.65 | 64 | 16 | 0.99 | 1.22 | 0.77 |
| 128 | 16 | 0.67 | 64 | 16 | 0.81 | 0.93 | 0.67 | 32 | 16 | 1.09 | 1.26 | 0.86 |
| 64 | 16 | 0.75 | 32 | 16 | 1.07 | 1.20 | 0.86 | 16 | 16 | 1.83 | 2.20 | 1.32 |
| 256 | 9 | 0.82 | 128 | 9 | 0.85 | 0.97 | 0.76 | 64 | 9 | 1.00 | 1.20 | 0.84 |
| 128 | 9 | 0.81 | 64 | 9 | 0.85 | 0.94 | 0.76 | 32 | 9 | 1.10 | 1.29 | 0.87 |
| 64 | 9 | 0.82 | 32 | 9 | 1.07 | 1.19 | 0.89 | 16 | 9 | 1.81 | 2.16 | 1.31 |

#### Performance degradation with reduced number of TRPs

In this section, we investigate the positioning accuracy with reduced number of TRPs for both sample- and path-based reporting approaches. Simulations are carried out with different RS bandwidth. For path-based reporting, channel estimator F in Table 11 is used.

To provide a detailed insight into the problem, we compare the 90%-tile positioning accuracy of

- ML models using sample-based reporting on 50 MHz RS bandwidth

- ML models using sample-based reporting on 25 MHz RS bandwidth

- ML models using sample-based reporting on 12.5 MHz RS bandwidth

- ML models using Rel-17 path-based reporting on 50 MHz RS bandwidth

- ML models using Rel-17 path-based reporting on 25 MHz RS bandwidth

- ML models using Rel-17 path-based reporting on 12.5 MHz RS bandwidth

- For all cases above, TRP numbers are selected as: 18, 15, 12, 9, 6, 4, 3

Evaluation results are shown in Table 13 to Table 15. From those tables, we can see that as the number of TRPs reduces, the positioning accuracy degrades significantly for all cases. Positioning error is almost doubled when the number of TRPs drops to 6. As the RS bandwidth becomes smaller, the positioning accuracy also degrades for both sample- and path-based reporting. However, sample-based reporting outperforms path-based reporting in general. It can be observed that the degradation with path-base reporting becomes more significant if fewer number of TRPs are used, especially for 25 and 12.5 MHz RS bandwidth. The improvement in positioning accuracy with sample-based reporting over 50 MHz bandwidth is about 10% for 6 TRPs with N't=16 . For bandwidth of 25 MHz with 6 TRPs, the improvement in positioning accuracy with sample-based reporting is about 35% with N't=16. For bandwidth of 12.5 MHz with 6 TRPs, the improvement in positioning accuracy with sample-based reporting is about 40%.

Table 13 90%tile 2D positioning accuracy Δ90% (meters) of PDP models using direct sample-based reporting or Rel-17 path-based reporting from different number of TRPs for the {60%, 6m, 2m} InF-DH scenario. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6]. For all the models trained on the Rel-17 path-based reports, the same channel estimator was used for the training and inference.

| Test cases |  | 90%tile 2D positioning errors [m] based on 50 MHz RS bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Nt | N't | Different number of TRPs |  |  |  |  |  |  |
|  |  |  | 18 | 15 | 12 | 9 | 6 | 4 | 3 |
| Sample-based reporting | 128 | 16 | 0.73 | 0.83 | 0.95 | 1.10 | 1.45 | 2.10 | 3.11 |
|  | 64 | 16 | 0.73 | 0.80 | 0.94 | 1.07 | 1.38 | 2.09 | 3.12 |
|  | 32 | 16 | 0.89 | 0.97 | 1.13 | 1.30 | 1.66 | 2.58 | 3.77 |
|  | 128 | 9 | 0.91 | 0.99 | 1.13 | 1.32 | 1.85 | 2.84 | 4.09 |
|  | 64 | 9 | 0.86 | 0.96 | 1.07 | 1.29 | 1.80 | 2.72 | 4.03 |
|  | 32 | 9 | 0.94 | 1.05 | 1.21 | 1.39 | 1.82 | 2.67 | 4.28 |
| Path-based reporting | 128 | 16 | 0.79 | 0.93 | 1.02 | 1.21 | 1.71 | 2.68 | 3.94 |
|  | 64 | 16 | 0.83 | 0.93 | 1.09 | 1.26 | 1.76 | 2.73 | 4.16 |
|  | 32 | 16 | 1.18 | 1.27 | 1.50 | 1.78 | 2.54 | 4.02 | 6.19 |
|  | 128 | 9 | 0.84 | 0.98 | 1.09 | 1.30 | 1.80 | 2.89 | 4.32 |
|  | 64 | 9 | 0.86 | 0.97 | 1.10 | 1.29 | 1.82 | 2.88 | 4.41 |
|  | 32 | 9 | 1.13 | 1.32 | 1.52 | 1.81 | 2.50 | 4.09 | 5.86 |

Table 14 90%tile 2D positioning accuracy Δ90% (meters) of PDP models using direct sample-based reporting or Rel-17 path-based reporting from different number of TRPs for the {60%, 6m, 2m} InF-DH scenario. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6]. For all the models trained on the Rel-17 path-based reports, the same channel estimator was used for the training and inference.

| Test cases |  |  | 90%tile 2D positioning errors [m] based on 25 MHz RS bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Nt | N't | Different number of TRPs |  |  |  |  |  |  |
|  |  |  | 18 | 15 | 12 | 9 | 6 | 4 | 3 |
| Sample-based reporting | 64 | 16 | 0.81 | 0.91 | 1.04 | 1.26 | 1.60 | 2.35 | 3.64 |
|  | 32 | 16 | 0.84 | 0.95 | 1.03 | 1.25 | 1.60 | 2.38 | 3.54 |
|  | 16 | 16 | 1.20 | 1.33 | 1.52 | 1.77 | 2.25 | 3.54 | 4.98 |
|  | 64 | 9 | 1.01 | 1.11 | 1.22 | 1.46 | 2.00 | 2.99 | 4.56 |
|  | 32 | 9 | 0.99 | 1.13 | 1.27 | 1.42 | 1.88 | 2.98 | 4.25 |
|  | 16 | 9 | 1.29 | 1.34 | 1.60 | 1.85 | 2.40 | 3.52 | 5.19 |
| Path-based reporting | 64 | 16 | 1.00 | 1.20 | 1.36 | 1.60 | 2.45 | 3.89 | 5.94 |
|  | 32 | 16 | 1.19 | 1.31 | 1.50 | 1.77 | 2.63 | 4.24 | 6.13 |
|  | 16 | 16 | 2.20 | 2.27 | 2.50 | 3.13 | 4.36 | 6.80 | 8.76 |
|  | 64 | 9 | 0.99 | 1.19 | 1.39 | 1.62 | 2.31 | 3.95 | 5.79 |
|  | 32 | 9 | 1.22 | 1.29 | 1.46 | 1.78 | 2.62 | 4.18 | 6.07 |
|  | 16 | 9 | 2.07 | 2.25 | 2.52 | 3.02 | 4.32 | 6.47 | 9.05 |

Table 15 90%tile 2D positioning accuracy Δ90% (meters) of PDP models using direct sample-based reporting or Rel-17 path-based reporting from different number of TRPs for the {60%, 6m, 2m} InF-DH scenario. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6]. For all the models trained on the Rel-17 path-based reports, the same channel estimator was used for the training and inference.

| Test cases |  |  | 90%tile 2D positioning errors [m] based on 12.5 MHz RS bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Nt | N't | Different number of TRPs |  |  |  |  |  |  |
|  |  |  | 18 | 15 | 12 | 9 | 6 | 4 | 3 |
| Sample-based reporting | 32 | 16 | 0.99 | 1.15 | 1.28 | 1.43 | 2.01 | 3.02 | 4.55 |
|  | 16 | 16 | 1.24 | 1.33 | 1.52 | 1.62 | 2.27 | 3.17 | 4.91 |
|  | 32 | 9 | 1.13 | 1.29 | 1.50 | 1.67 | 2.19 | 3.46 | 4.95 |
|  | 16 | 9 | 1.26 | 1.37 | 1.61 | 1.82 | 2.38 | 3.44 | 5.13 |
| Path-based reporting | 32 | 16 | 1.47 | 1.68 | 1.84 | 2.33 | 3.50 | 5.75 | 8.35 |
|  | 16 | 16 | 1.85 | 2.05 | 2.31 | 2.79 | 4.08 | 6.44 | 9.08 |
|  | 32 | 9 | 1.42 | 1.68 | 1.93 | 2.34 | 3.71 | 5.80 | 8.36 |
|  | 16 | 9 | 1.90 | 2.01 | 2.19 | 2.73 | 3.91 | 6.80 | 8.79 |

A subset of the evaluation results shown in Table 13 to Table 15 are also plotted in Figure 13 to Figure 16. Each figure show the positioning accuracy achieved of sample-based measurement vs path-based measurement, for a given time window duration and a given N't . The evaluation results show a clear advantage of sample-based measurement for a range of RS bandwidth, time window size, and N't.

Sample-based reporting can achieve significantly better positioning accuracy compared to path-based reporting when fewer number of TRPs are used on smaller RS bandwidth (i.e., BW=50, 25 and 12.5 MHz).

Sample-based measurement allows a substantially smaller PRS bandwidth to be configured to achieve the same positioning accuracy as path-based measurement. The PRS resource allocation savings can be 50% or more, especially for smaller number of active TRPs (e.g., 3-12 TRPs).

Sample-based measurement allows a substantially smaller number of reported TRPs to achieve the same positioning accuracy as path-based measurement. The reported TRP overhead savings can be 50% or more, especially for smaller BWs.

Figure 13. 90%tile 2D positioning accuracy Δ90% (meters) of sample-based measurement vs path-based measurement. Time window size of 2083.84 ns, corresponding to Nt=128 for 50 MHz. N’t=16. {60%, 6m, 2m} InF-DH scenario.

Figure 14. 90%tile 2D positioning accuracy Δ90% (meters) of sample-based measurement vs path-based measurement. Time window size of 1041.92 ns, corresponding to Nt=64 for 50 MHz. N’t=16. {60%, 6m, 2m} InF-DH scenario.

Figure 15. 90%tile 2D positioning accuracy Δ90% (meters) of sample-based measurement vs path-based measurement. Time window size of 2083.84 ns, corresponding to Nt=128 for 50 MHz. N’t=9. {60%, 6m, 2m} InF-DH scenario.

Figure 16. 90%tile 2D positioning accuracy Δ90% (meters) of sample-based measurement vs path-based measurement. Time window size of 1041.92 ns, corresponding to Nt=64 for 50 MHz. N’t=9. {60%, 6m, 2m} InF-DH scenario.

#### Burden of higher signaling size with path-based measurements

In this section, we provide a preliminary but more detailed signaling size vs positioning accuracy trade-off analysis between the sample- and path-based signaling approaches.

For the sample-based signaling, most companies used

"Sampling period = 1/(Nf ×∆f). For FR1, sampling period = 1/(4096×30)=8.14 (ns), where Nf =4096 according to 38.211, and ∆f =30 kHz is the subcarrier spacing."

That is, sampling-based signaling with a resolution of 16´Tc has been studied extensively by companies. Details of the signaling sizes for sample-based reporting are provided in Section 3.2.4.2.

For the path-based reporting, the existing timing information report (e.g., RSTD) allows the resolution step of 2k´Tc is commonly used, where the parameter k can be configured by the LMF within the range of 0<=k<=5. Thus, the timing information can be configured to be reported with granularity in the range of Tc to 32´Tc. The number of bits needed to express a path delay has the following range

- 16 – 21 bits for the first path

  - Reporting of the first path is still based on the unit of the configured granularity. So, this is still a grid point.

- 9 – 14 bits for each additional relative path (7 bits shorter than for the first path)

However, to achieve more granular path resolution than the sample-based signaling approach with 16´Tc resolution, the parameter k for the path-based signaling should be configured in the range of 0<=k<=3 such that the timing information is configured to be reported with granularity in the range of Tc to 8´Tc. Hence, the number of bits needed to express a path delay has the following range

  - 18 – 21 bits for the first path

  - 11 – 14 bits for each additional relative path (7 bits shorter than for the first path)

The signaling of one path report can contain up to three parts:

- Estimated delay

- Interpolated power of the path

- Interpolated phase of the path

If the signaling contains feedbacks for NTRP links, the number of bits for each part is

- Delays: NTRP × Bdelay bits

- Powers: NTRP × Nport × Nt' × Breal bits

- Phases: NTRP × Nport × Nt' × Breal bits

where

- NTRP = 18 or dynamic TRP selection to 9, 6, 3 TRPs

- Nport = 2 for CIR and Nport = 1 for PDP

- Breal = 8 bits

- Bdelay = (11 × Nt' + 7) to (14 × Nt' + 7) bits for signaling Nt' delays according to existing specs

That is, for one positioning request, the total numbers of feedback bits are

- DP

  - NTRP × Bdelay bits

  - Optional: NTRP × (Bdelay + Breal) bits (including the RS RSRP for the TRP links)

- PDP

  - NTRP × (Bdelay + Nt' × Breal) bits

- CIR

  - NTRP × (Bdelay + 4 × Nt' × Breal) bits

Comparing these number of signaling bits using the Rel-17 path reporting approach with those using the regular sampling grid based signaling (see section 3.2.4.2), it can be seen that the only difference is whether the Nt bitmap is smaller or larger than the per-path delay reporting size Bdelay for the specific combinations of lengths, sub-sampling sizes and TRP down-selection case.

Using the smallest per-path delay reporting format (11 × Nt' + 7 bits) that is more granular (8´Tc) than the sample-based reporting as an example for the Rel-17 path based reporting, we plot the comparison of signal sizes using the regular sampling grid signaling approach or the Rel-17 path reporting approach in Figure 17. To avoid over-crowding the plot, the comparison is based on the PDP input type.

The 90%tile 2D UE positioning errors for the different channel estimators are averaged. Note that, since measurement nodes with computational constraints using simpler channel estimators, the ML models using the Rel-17 path-based reports tend to perform worse than models using sample-based reporting which requires much lower processing complexity. The averaged performance may be too optimistic when the measurement node is a UE.

We can observe that

- The sizes of path-based signaling scale with the number of nonzero subsamples Nt'.

- The sizes of sample-based signaling scale with the number of available samples for subsampling, i.e., Nt.

As a result of the above general behaviors of the two feedback approaches, we can see that

- Sample-based signaling requires fewer signaling bits than path-based signaling for the following cases

  - Nt = 64, Nt' = 9

  - Nt = 64, Nt' = 16

  - Nt = 128, Nt' = 16

- Path-based signaling requires fewer signaling bits than sample-based signaling for the following cases

  - Nt = 128, Nt' = 9

  - Nt = 256, Nt' = 9

  - Nt = 256, Nt' = 16

- The efficiency/performance frontier formed by the sample-based signaling using the following combinations offer strictly better positioning accuracy and lower signaling sizes than path-based signaling:

  - Sampled based Nt = 64, Nt' = 9

  - Sampled based Nt = 64, Nt' = 16

  - Sampled based Nt = 128, Nt' = 16

  - Sampled based Nt = 128, Nt' = 24

  - Sampled based Nt = 128, Nt’ = 32

Figure 17 Scatter plot of measurement report sizes and the achieved 90%tile 2D UE positioning errors in the {60%, 6m, 2m} InF-DH scenario for PDP input types with different combinations of lengths Nt and sub-sampling sizes Nt' using the regular sampling grid signaling approach or the Rel-17 path reporting approach. The points are labelled by the Nt,Nt' parameters. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6]. The sizes of the signaling for the sample-based feedback are based direct bitmaps for the timings of the nonzero subsamples described in Section 3.2.4.2.

Furthermore, following the unified measurement framework and further compression of path/sampling timing information bits, the number of bits needed to report the timings of the nonzero subsamples can be substantially reduced. As an example, we replot the comparison shown in Figure 17 with the more efficient representation for the sample-based feedback in Figure 18.

We can observe that the more efficient bitmap representation enables the sample-based feedback to reduce the number of signal bits substantially.

- Furthermore, the sizes of the signaling bits for the sample-based feedback scale mainly with the number the number of nonzero subsamples Nt', too.

- The efficiency/performance frontier formed by the sample-based signaling offer strictly better positioning accuracy and lower signaling sizes than path-based signaling.

Sample-based signaling can achieve better positioning accuracy and lower signaling sizes than legacy path-based signaling using substantially simpler measurement node processing.

Figure 18 Scatter plot of measurement report sizes and the achieved 90%tile 2D UE positioning errors in the {60%, 6m, 2m} InF-DH scenario for PDP input types with different combinations of lengths Nt and sub-sampling sizes Nt' using the regular sampling grid signaling approach or the Rel-17 path reporting approach. The points are labelled by the Nt,Nt' parameters. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6]. The sizes of the signaling for the sample-based feedback are based on more efficient representation of nonzero subsample timing bitmaps.

## Analysis of supporting phase information measurement

In RAN1#116, the following agreement was made to investigate whether phase information should be supported as a part of model input, for example, as list of phase information as in CIR or one phase value for the first detected path only.

Agreement For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input. The issues to study include: - Tradeoff of positioning accuracy and signaling overhead - The impact of transmitter and receiver implementation - Specification impact - Other aspects are not precluded Note: the phase information may be used in different ways, e.g., one phase value for the first path or first sample only; triplet of {timing information, power information, phase information} for CIR, etc.

In the sub-sections below, analysis is provided for the following aspects:

- Inapplicability of Rel-18 carrier phase positioning;

- Issues with non-spatial-dependent initial phases;

- Difficulty to align phase measurements between model training and model inference;

- Signaling overhead

Considering all the pros and cons of phase measurement as model input, we conclude that it is not feasible nor necessary to support phase information for determining model input, including CIR and single phase value for first path.

Do not support phase information for determining model input, including CIR and single phase value for first path/sample.

### Inapplicability of Rel-18 carrier phase positioning

During RAN1#116bis, there was a discussion on supporting Rel18 CPP measurements (DL RSCPD, DL RSCP, UL RSCP) for AI/ML positioning as model input. Since release 18 positioning was specified in parallel to the AI/ML study item phase, CPP was not evaluated during the Rel-18 AI/ML study. Moreover, CPP was specified for environement with very high line of sight probability. During Rel18 CPP specification, it was concluded  that LOS was critical to proper operation of CPP.  Additionally, In AI/ML cases, the TRP/UE links are generally NLOS, and the phase information of the first path/sample does not carry much information of the UE position.  Additionally, NLOS phase information is very sensitive to even minimal environement changes, such as changes in the nature of the material reflecting the path.  Finally, CPP requires PRU to be operating next to the UE during both training and inference, which adds to deployment cost and signalling overhead.

Rel-18 CPP measurements (DL RSCPD, DL RSCP, UL RSCP) assumes LoS environment, which is contradictory with the target use case of Rel-19 AI/ML based positioning.

Single phase value for first path/sample (e.g., DL RSCPD, DL RSCP, UL RSCP) brings no observable performance benefits to PDP, but incurs substantially high deployment cost and signalling overhead.

### Issues with random initial phase

The clock and/or frequency generation circuits are essential components in modern digital wireless communications systems. The circuits are used to drive reference waveform to demodulate signals received over the air. Due to the random states of clock/frequency generation circuits in the measurement radio node, the phase of the ML measurement data such as the time domain channel impulse responses may contain random and unhelpful information to AI/ML models. For the AI/ML positioning application, this means for UE revisiting the exact same location, the TD CIR sample sequences may still differ because the state of the clock/frequency generation circuits may be at different state. A TD CIR sample ha(1)[d] measured at a first visit and ha(2)d measured at a second visit may be different because of the different states of clock/frequency generation circuits.

More specifically, let the sequence of Nt' complex-valued TD CIR samples measured on receive antenna port a be

had0, had1, …, ha[dNt'-1]

where d0, d1, …, dNt'-1 are the sampling time locations of the TD CIR samples. The sampling time locations may be on a regular sampling grid with integer values or off the regular grid with non-integer values as discussed in the last section. The complex CIR samples can be equivalently represented by the power and phase parts:

pad0,pad1, …, pa[dNt-1]

θad0, θad1, …, θa[dNt-1]

where had=pad⋅expjθad, for d=d0, d1, …, dNt'-1.

The above discussion indicates that the initial phase value θad0 may in fact be random and contains no useful spatial dependent information. An AI/ML model trained with samples containing such spurious values will nonetheless learn to associate the target positions with these random initial phases in the training dataset. When fed with new measurements with new random phases during inference, the model will be confused and generates inaccurate position estimates.

For instance, we trained a Nt=256, Nt'=128 CIR model using a training dataset of 40,000 samples with the measured phases (i.e., containing the random initial phases). In Table 16, we tested the trained model under two different conditions:

- When the trained model is tested with consistent initial phase (that is, the initial phase values for different UE positions reflects only the spatial differences), the model achieves a 90%tile 2D UE positioning error of 0.38 m.

- When the trained model is tested with random initial phase (that is, the initial phase values for different UE positions reflects the spatial differences and random phases), the 90%tile 2D UE positioning error increases by 20% to 0.46 m.

Table 16 Comparison of 2D UE positioning errors Δ90% (in meters) for a Nt=256, Nt'=128 model trained without random initial phase and (1) tested without random initial phases or (2) tested with random initial phases in the {60%, 6m, 2m} InF-DH scenario. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6].

| CDF Percentile | UE 2D positioning errors [m] in {60%, 6m, 2m} Model I trained with 40,000 CIR samples without random θad0 |  |
| --- | --- | --- |
|  | Tested without random θad0 | Tested with random θad0 |
| 50 | 0.19 | 0.23 |
| 67 | 0.25 | 0.30 |
| 80 | 0.31 | 0.38 |
| 90 | 0.38 | 0.46 |

The initial phase values of measured CIR samples may be random and contains no useful spatial dependent information for an AI/ML model to learn the association between measured CIR samples and the target positions. Without addressing such spurious information during training and inference, fingerprinting ML models using the CIR inputs can produce inaccurate position estimates.

Before CIR can be adopted as model input, RAN1 need to investigate whether fingerprinting ML models can handle CIR phase measurements, which vary not only with the radio channel environment but also with the transmitter/receiver circuits.

While the initial phase θad0 may be random and contains no useful channel dependent information, the relative phases of the samples

∆θad=θad-θad0,  for d=d1, d2, …, dNt-1

still contain useful information relating the channel and the sampling time. Hence, one way to circumvent the impact of the random initial phases is to scrub it from the model input. That is, instead of recording/signaling Nt' values of powers and Nt' values of phases, only 2Nt'-1 values are signaled/recorded:

pad0,pad1, …, pa[dNt-1]

θad1, ∆θad2, …, ∆θa[dNt-1]

The phase of the initial sample θad0 is taken as zero and not signaled. Equivalently, this is just applying a common phase de-rotation to the complex-valued CIR samples to set the initial phase to zero:

pad0, e-θad0had1, …,e-θad0ha[dNt'-1]

In Table 17, we compared the 90%tile 2D UE positioning errors Δ90% (in meters) for two sets of models:

- The first set of models are trained and tested with CIR samples containing no random initial phases. Note that it’s not possible to collect such type of CIR samples in reality because of the random states of the clock and/or frequency generation circuits discussed in the above.

- The second set of models are trained and tested CIR samples containing relative phases as just described.

It can be observed that the models trained and tested CIR relative phase samples achieve essentially identical positioning accuracy (up to simulation noises) as the models trained and tested with idealized (but not physically feasible) CIR samples.

CIR samples containing relative phases can be used as inputs to AI/ML models to circumvent the impact of random initial phase values of measured CIR samples.

Table 17 Comparison of 90%tile 2D UE positioning errors Δ90% (in meters) for (1) models trained and tested with CIR samples containing no random initial phases and (2) models trained and tested with CIR samples containing relative phases in the {60%, 6m, 2m} InF-DH scenario. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6].

| Nt | Nt' | 90%tile 2D positioning error [m] in {60%, 6m, 2m} InF-DH Model I trained with 40,000 CIR samples |  |
| --- | --- | --- | --- |
|  |  | Trained and tested without random initial phases | Trained and tested with relative phases |
| 256 | 128 | 0.38 | 0.39 |
| 256 | 64 | 0.43 | 0.44 |
| 128 | 64 | 0.48 | 0.46 |
| 256 | 32 | 0.54 | 0.55 |
| 128 | 32 | 0.53 | 0.55 |
| 64 | 32 | 0.67 | 0.66 |
| 256 | 16 | 0.68 | 0.69 |
| 128 | 16 | 0.64 | 0.65 |
| 64 | 16 | 0.70 | 0.71 |
| 256 | 9 | 0.82 | 0.84 |
| 128 | 9 | 0.82 | 0.81 |
| 64 | 9 | 0.80 | 0.80 |

If the relative phase described above is to be supported, then new measurement types needs to be introduced in 38.215. An example is shown below and called UL RSRCP, which is for UL measurements based on SRS. Similar new measurement type is needed for DL PRS as well.

UL reference signal relative carrier phase (RSRCP) is defined as a sequence of relative phase values of the channel response derived from the resource elements carrying sounding reference signals (SRS) configured for the measurement. For the j-th path, the relative phase is the difference between the phase of the j-th path and the phase of the first path.

It is noted that the UL RSRCP illustrated above needs to be added in addition to the existing UL reference signal carrier phase (RSCP) introduced in Rel-18, which is only for the phase of the first detected path delay. Related to the new measurement type UL RSRCP, standardization effort is also needed in RAN1/RAN2/RAN3 for signaling support for training data collection and model inference, and potentially in RAN4 for requirements. Similar standardization effort is needed for the corresponding DL PRS measurement as well. RAN1 needs to take these into account in determining whether to support CIR in Rel-19.

Furthermore, the analysis above does not consider inter-path phase difference caused by the TRP’s and the UEs antenna patterns. As discussed in section 3.2.3, it is unclear how to remove such inter-path phase difference between model training and model inference, especially the phase offset caused by UE Tx or Rx antenna.

To decide whether to support phase information to enable CIR as model input, RAN1 should weigh the small positioning accuracy improvement against the standardization effort, the signaling overhead, and the difficulty to align phase information between training and inference.

If phase information is supported to enable CIR as model input, the phase values are reported in the format of relative phase.

### Difficulty to align phase measurements between model training and model inference

Assuming a magnitude-phase measurement is used as model input, e.g., a complex CIR or even a single path, some ambiguity will need to be resolved. In the downlink, the phase component of a received path consists of the following:

θue=θ(d1u)+Δ1(αTRP)+Δu(βue)

In this equation, we can see that the receive phase is made of:

- The channel path phase θ(d1u), which is a function of the distance between the TRP and the UE.

- The TRP phase at the time of transmission Δ1αTRP, which includes a random phase rotation caused by the free-running oscillator, and a deterministic rotation caused by the beamforming weights and the Tx antenna pattern. Thus the initial phase depends on the direction of departure of the path.

- The UE phase at the time of reception Δu(βue), which includes a random phase rotation caused by the free-running oscillator, and a deterministic rotation caused by the beamforming weights and the Rx antenna pattern. Thus the initial phase depends on the direction of arrival of the path.

Let’s assume that the model input is the RSCP/RSCPD measurement defined in release 18, i.e., the (complex) phase/ phase difference for the first path. Carrier phase positioning specified in release 18 measures the phase difference between two DL PRSs received from two TRPs:

θuediff=θd1u+Δ1αTRP1+Δuβue- θd2u+Δ2αTRP2+Δuβue

Doing so cancels out the UE’s own random phase error contribution to the phase error. We can assume there is no UE phase error contribution left if the UE knows its antenna pattern’s phase offset and can remove it. The remaining phase difference becomes:

θuediff=θd1u+Δ1αTRP1- θd2u+Δ2αTRP2

To cancel out the TRPs contribution to the phase error, a PRU is used to collect a measurement from each TRP and form its own phase difference:

θPRUdiff=θd1,PRU+Δ1PRUαTRP1- θd2,PRU+Δ2PRUαTRP2

The assumption is that the PRU allows the estimator to perfectly know the distance between the TRP and the PRU, so that the differential phase errors between the TRPs can be estimated as:

θPRUdiff-θd1,PRU+θd2,PRU=Δ1PRUαTRP1- Δ2PRUαTRP2

The final phase difference then can be expressed as following and can be used to find θd1u-θd2u:

θuediff=θd1u-θd2u+θPRUdiff-θd1,PRU+θd2,PRU

In the above, it is assumed that Δ1PRUαTRP1- Δ2PRUαTRP2=Δ1αTRP1-Δ2αTRP2. If the PRU and the UE are collocated well enough, and the measurements are taken very close in time, the PRU-estimate of the differential phase error between TRPs corresponds to the TRP phase error experienced by the UE. for this reason, release 18 specified a measurement time window for RSCP/RSCPD measurements. Hence, if the Rel-18 measurement is used for measurement input, PRUs need to assist the measuring UE during inference, since the phase errors from TRPs are time-dependent.

For the RSCPD measurement PRU assisted phase error compensation requires PRUs to be used during training and inference, further increasing the deployment cost and signaling overhead.

During RAN1#116, it was also discussed to use inter-path phase information to resolve the issue of random phase at transmitter and receiver, see also analysis in section 3.2.2. Inter-path phase information will indeed remove the random phase error caused by the transmitter and receiver oscillators. What remains is the inter-path phase difference caused by the TRP’s and the UEs antenna patterns.  Phase errors due to the TRP antenna will be the same during training and inference and thus can be resolved, but the UE antenna pattern will be difficult to keep consistent between inference and training, since the UE’s antenna pattern as well as its relative orientation toward the TRP may be different between training and inference. One other alternative would be to remove the phase difference between path caused by the antenna, which means the UE phase error caused by the Rx (Tx) antenna must be known to the model, for each direction of arrival (departure).

For inter-path phase measurements, the model input at training and inference must know the UE TX or RX antenna phase contribution, either by reporting the antenna phase contribution to the model, removing it, or assuming the same antenna pattern is used during inference and training.

It is unclear how a mobile UE would know the direction of departure / arrival and associated phase offset for each path toward a given TRP.

### Signaling size vs positioning accuracy tradeoff analysis

The complex-valued multi-port CIR samples have been found in the Rel-18 SI to achieve high positioning accuracy. However, these CIR samples require significantly larger signaling sizes than the total-power PDP. To reduce signaling sizes, many different down-sampling techniques such as truncation and sub-sampling have been investigated during the SI phase. It is therefore necessary to investigate the trade-off between signaling size requirements and achievable positioning accuracy of various combinations of sample lengths, sub-sample sizes and with CIR or PDP.

In the following, we present two signaling size vs positioning accuracy trade-off comparison.

- In Section 3.2.4.1, we use the number of real numbers to be signaled as a proxy of the signaling size.

- In Section 3.2.4.2, we construct a more detailed signaling size estimation based on specific feedback formats.

From both signaling sizes vs positioning accuracy trade-off analysis exercises, we arrive at the following observation.

For small or moderate signaling sizes, PDP and DP samples can achieve better positioning accuracy than CIR samples at the same or smaller signaling size. Multi-port CIR samples can achieve higher positioning accuracy only with very large signaling requirements.

For small or moderate signaling sizes, PDP and DP samples can achieve better positioning accuracy than CIR samples at the same or smaller signaling size as well as with substantially lower AI/ML model complexity.

For most given 90%tile 2D UE positioning error requirements, the DP samples requires the smallest signaling sizes.

Furthermore, the assumption of two receive antenna ports for the UE was a simplifying assumption made for Rel-17 Positioning WI. For the Rel-19 AI for PHY, it may be necessary to consider four receive antenna ports for the typical NR UEs. This would double the required numbers of real values to represent the multi-port CIR again for Case 2b (2nd priority).

For Case 3b (1st priority), a typical gNB/TRP may be equipped with dozens of receive antenna ports. Forwarding such large multi-port CIR samples to LMF will have significantly negative impact on radio and core networks.

Multi-port complex-valued CIR samples for both Case 3b (1st priority) and Case 2b (2nd priority) require very large signaling sizes, which can cause significantly negative impacts on the radio and core networks.

Based on the extensive evaluation and analysis presented in this section, we propose

RAN1 to down-prioritize the signaling approach(es) and/or measurement definitions to support CIR model input types for Case 3b (1st priority) and Case 2b (2nd priority).

#### Simple signaling size comparison based on the number of real numbers to signal

In this section, we make a first quick comparison of feedback sizes and achievable positioning accuracy based on the number of real numbers needed in the signaling. An NR radio node is normally equipped with multiple RX antenna ports. AI/ML models with multi-port CIR inputs have been evaluated by several companies during the SI phase. As shown below, multi-port CIR can enable AI/ML models to achieve high positioning accuracy but will also require large signaling overhead. In the following, we investigate whether this accuracy vs overhead tradeoff can be improved by reducing the dimensions of the multi-port CIR inputs.

We consider two dimension-reduction approaches to reduce multi-port CIR inputs to a single vector:

- Strongest port CIR

The measuring radio node computes the RSRP of each RX antenna ports and signals CIR samples only for the port with the strongest received power.

- First principal component (1st PC) CIR

Let the Nt'×A matrix H collects the TD CIR samples from all antenna ports. The first principal component is given by Hv, where the length-A vector v is the eigenvector corresponding to the largest eigenvalue of HHH.

Our positioning accuracy evaluation results for 2-port CIR, strongest port CIR, 1st PC CIR and PDP inputs are shown in Table 18. In this comparison, we classify different combinations of sample lengths, sub-sample sizes and with CIR or PDP based on the number of real numbers to represent the samples. Similarly, in Table 19, we compare the positioning accuracy vs signaling sizes with dynamic TRP selection for Ntrp'= 18, 9, 6 or 3. The total number of real numbers to signal is given by Ntrp'×Nt' for PDP and 4×Ntrp'×Nt' for 2-port CIR.

- For instance, a length Nt=128 2-port CIR with Nt'=32 sub-samples require 128 real numbers per reported TRP to represent the complex-valued CIR samples.

- For the same number of real numbers in signaling, the strongest port CIR or the 1st PC CIR samples can be of length Nt=256 with Nt'=64 sub-samples per reported TRP.

- For the same number of real numbers in signaling, the PDP samples can be of length Nt=256 with Nt'=128 sub-samples per reported TRP.

It is then a fair comparison to check which approach achieves better performance when they require the same number of real values to represent the samples. From the table, we make the following observations:

- For small or moderate signaling sizes, PDP samples can achieve better positioning accuracy than 2-port CIR samples at the same or smaller signaling sizes. Using PDP samples, a 90%tile 2D positioning error of 0.52 m can be achieved.

- The higher positioning accuracy achieved by 2-port CIR is only possible with very large signaling sizes.

- While dimension reduction techniques reduce the signal sizes of CIR samples when compared to using the multi-port CIR directly, the achievable positioning accuracy is also compromised. Overall, applying dimension reduction to reduce the signaling sizes of the multi-port CIR samples does not appear to improve its accuracy vs overhead tradeoff.

  - The first principal component approach enables marginally better AI/ML model positioning accuracy than the strongest port selection approach.

From a model input design point of view, one can conclude from these evaluation results that

- With small to moderate number of samples available to an ML model, the total-power PDP input type is a very helpful induction bias to impose on the ML model based on human domain knowledge that the sample powers contain more important information about the UE positions.

  - Using the real-valued PDP inputs also reduces the AI/ML model complexity substantially as shown in Table 20.

- Only with very large number of samples available to the ML model, the model can start to tease out how to use the additional information in the sample phases on its own.

With small to moderate number of samples available to an ML model, the total-power PDP input type is a very helpful induction bias to impose on the ML model based on human domain knowledge that the sample powers contain more important information about the UE positions. It’s only with very large number of samples that the model can start to tease out how to use the additional information in the sample phases on its own.

Dimension reduction techniques can be used to reduce the signal sizes of multi-port CIR samples. But the achievable positioning accuracy is also compromised. The overall accuracy vs overhead tradeoff situation of CIR samples is not improved by the two considered dimension reduction techniques.

Table 18 Comparison of 90%tile 2D UE positioning errors Δ90% (in meters) for 2-port CIR, 1-port CIR and PDP using different combination of sample lengths and sub-sampling sizes in the {60%, 6m, 2m} InF-DH scenario. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6] and Ntrp'=Ntrp= 18.

| Number of real values | 2-port CIR |  |  | 1st PC CIR |  |  | Strongest port CIR |  |  | PDP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Nt | Nt' | Δ90% | Nt | Nt' | Δ90% | Nt | Nt' | Δ90% | Nt | Nt' | Δ90% |
| 1024×18 | 256 | 256 | 0.37 |  |  |  |  |  |  |  |  |  |
| 512×18 | 256 | 128 | 0.38 | 256 | 256 | 0.49 | 256 | 256 | 0.48 |  |  |  |
|  | 128 | 128 | 0.44 |  |  |  |  |  |  |  |  |  |
| 256×18 | 256 | 64 | 0.43 | 256 | 128 | 0.50 | 256 | 128 | 0.50 | 256 | 256 | 0.51 |
|  | 128 | 64 | 0.48 | 128 | 128 | 0.62 | 128 | 128 | 0.60 |  |  |  |
|  | 64 | 64 | 0.67 |  |  |  |  |  |  |  |  |  |
| 128×18 | 256 | 32 | 0.54 | 256 | 64 | 0.54 | 256 | 64 | 0.57 | 256 | 128 | 0.52 |
|  | 128 | 32 | 0.53 | 128 | 64 | 0.62 | 128 | 64 | 0.63 | 128 | 128 | 0.56 |
|  | 64 | 32 | 0.67 | 64 | 64 | 0.83 | 64 | 64 | 0.88 |  |  |  |
|  | 32 | 32 | 1.60 |  |  |  |  |  |  |  |  |  |
| 64×18 | 256 | 16 | 0.68 | 256 | 32 | 0.66 | 256 | 32 | 0.68 | 256 | 64 | 0.52 |
|  | 128 | 16 | 0.64 | 128 | 32 | 0.69 | 128 | 32 | 0.70 | 128 | 64 | 0.57 |
|  | 64 | 16 | 0.70 | 64 | 32 | 0.84 | 64 | 32 | 0.84 | 64 | 64 | 0.71 |
|  |  |  |  | 32 | 32 | 1.80 | 32 | 32 | 1.81 |  |  |  |
| (36 or 32) ×18 | 256 | 9 | 0.82 | 256 | 16 | 0.80 | 256 | 16 | 0.85 | 256 | 32 | 0.58 |
|  | 128 | 9 | 0.82 | 128 | 16 | 0.78 | 128 | 16 | 0.83 | 128 | 32 | 0.59 |
|  | 64 | 9 | 0.80 | 64 | 16 | 0.87 | 64 | 16 | 0.88 | 64 | 32 | 0.72 |
|  |  |  |  |  |  |  |  |  |  | 32 | 32 | 1.17 |
| (18 or 16) ×18 |  |  |  | 256 | 9 | 0.97 | 256 | 9 | 1.01 | 256 | 16 | 0.69 |
|  |  |  |  | 128 | 9 | 0.95 | 128 | 9 | 0.99 | 128 | 16 | 0.67 |
|  |  |  |  | 64 | 9 | 0.99 | 64 | 9 | 0.99 | 64 | 16 | 0.75 |
| 9×18 |  |  |  |  |  |  |  |  |  | 256 | 9 | 0.82 |
|  |  |  |  |  |  |  |  |  |  | 128 | 9 | 0.81 |
|  |  |  |  |  |  |  |  |  |  | 64 | 9 | 0.82 |

Table 19 Comparison of 90%tile 2D UE positioning errors Δ90% (in meters) for 2-port CIR and PDP using different combination of sample lengths and sub-sampling sizes in the {60%, 6m, 2m} InF-DH scenario. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6] and dynamic TRP selection with Ntrp'= 18, 9, 6 or 3.

| Number of real values | 2-port CIR |  |  |  | PDP |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ntrp' | Nt | Nt' | Δ90% | Ntrp' | Nt | Nt' | Δ90% |
| 18432 | 18 | 256 | 256 | 0.37 |  |  |  |  |
| 9216 | 9 | 256 | 256 | 0.95 |  |  |  |  |
| 6144 | 6 | 256 | 256 | 1.39 |  |  |  |  |
| 4608 |  |  |  |  | 18 | 256 | 256 | 0.51 |
| 3072 | 3 | 256 | 256 | 2.18 |  |  |  |  |
| 2304 |  |  |  |  | 9 | 256 | 256 | 1.07 |
| 1536 |  |  |  |  | 6 | 256 | 256 | 1.55 |
| 768 |  |  |  |  | 3 | 256 | 256 | 2.16 |

Table 20 Complexity comparison for models with (1) 2-port CIR, (2) 1-port CIR, and (3) total Rx power (1-port) PDP inputs.

| 2-port CIR |  | 1-port CIR |  | (1-port/total) PDP |  |
| --- | --- | --- | --- | --- | --- |
| # paras | FLOPs | # paras | FLOPs | # paras | FLOPs |
| 0.73 M | 32 M | 0.52 M | 21 M | 0.36 M | 9 M |

#### Signaling sizes based on simple direct sample timing bitmaps

The signaling of one TRP link for the DP input type consists of one part:

- Part 1: a bitmap of length Nt to indicate locations of non-zero samples

  - This part is to support sub-sampling the TD measurement samples.

    - The number of indicated non-zero samples in this bitmap is denoted by Nt'.

  - For PDP and CIR, if sub-sampling is not configured, then this part is not needed and Nt'= Nt in the following.

The signaling of one TRP link for the PDP input type consists of Part 1 as defined above and

- Part 2: Nt' powers (or magnitudes) of non-zero samples starting from the lowest tap indices.

The signaling of one TRP link for the CIR input type consists of Part 1 and Part 2 as defined above as well as

- Part 3: Nt' phase of non-zero samples starting from the lowest tap indices.

If the signaling contains feedbacks for NTRP links, the number of bits for each part is

- Part 1: NTRP × Nt bits

- Part 2: NTRP × Nport × Nt' × Breal bits

- Part 3: NTRP × Nport × Nt' × Breal bits

where

- NTRP = 18 or dynamic TRP selection to 9, 6, 3 TRPs

- Nport = 2 for 2-port CIR and Nport = 1 for PDP and 1st PC CIR

- Breal = 8 bits

That is, for one positioning request, the total numbers of feedback bits for reporting measurement are

- DP

  - NTRP × Nt bits

  - Optional: NTRP × (Nt + Breal) bits (including the RS RSRP for the TRP links)

- PDP

  - NTRP × Nt × Breal bits if sub-sampling is not configured

  - NTRP × (Nt + Nt' × Breal) bits if sub-sampling is configured

- 2-port CIR

  - NTRP × 4 × Nt × Breal bits if sub-sampling is not configured

  - NTRP × (Nt + 4 × Nt' × Breal) bits if sub-sampling is configured

  - Note: factor 4 is used above since CIR uses 2-port and complex values

- 1st PC CIR

  - NTRP × 2 × Nt × Breal bits if sub-sampling is not configured

  - NTRP × (Nt + 2 × Nt' × Breal) bits if sub-sampling is configured

  - Note: factor 2 is used above since CIR uses complex values

In Figure 19, we show the scatter plot of feedback sizes (i.e., number of bits to report measurements) and the achieved 90%tile 2D UE positioning errors for different AI/ML model input types with the following different combinations of lengths, sub-sampling sizes and TRP down-selection [6]:

- CIR/PDP

  - Truncation to Nt= 256, 128, 64, 32 taps without sub-sampling

  - Nt= 256-sample with Nt'= 128, 64, 32, 16, 9 sub-samples

  - Nt= 128-sample with Nt'= 64, 32, 16, 9 sub-samples

  - Nt= 64-sample with Nt'= 32, 16, 9 sub-samples

- DP

  - Truncation to Nt=256, 128, 64 taps with Nt' non-zero samples

  - With magnitude 1 or optionally with RS RSRP

- Dynamic TRP selection to 9, 6, 3 TRPs with Nt=Nt'= 256.

From the figure, we make the following observations:

- For small or moderate signaling sizes, PDP samples can achieve better positioning accuracy than 2-port CIR samples at the same or smaller signaling sizes. Using PDP samples, a 90%tile 2D positioning error of 0.52 m can be achieved.

- The higher positioning accuracy achieved by 2-port CIR is only possible with very large signaling sizes.

- For most given 90%tile 2D UE positioning error requirements, the DP samples requires the smallest signaling sizes.

Figure 19 Scatter plot of feedback sizes and the achieved 90%tile 2D UE positioning errors in the {60%, 6m, 2m} InF-DH scenario for different AI/ML model input types with different combinations of lengths, sub-sampling sizes and TRP down-selection using the regular sampling grid signaling approach. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6].

## Reference Time for timing info of PDP and DP

In this section, we discuss the reference time for timing information of PDP and DP.

During RAN1#116bis, the following agreement was made:

Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT

As discussed in RAN1#117, the commonly acknowledged way forward is:

RAN1 proceed with the investigation assuming each case is deployed individually, unless RAN plenary expand the scope of the WID to explicitly support certain combination of cases.

Since RAN plenary meetings didn't update the WID to explicitly support multi-RTT, RAN1 does not need to consider the support of multi-RTT in discussions. Thus the FFS bullet in the agreement above should be deleted.

Update the agreement for Case 3b channel measurement reference time by deleting the FFS.

Furthermore, Case 3a may require channel measurements for model input to be reported over a standardized interface (i.e., F1-AP) as well. If so, Case 3a should use the same reference time as that of Case 3b. That is, for uplink channel measurements corresponding to model input (Case 3a/3b), the timing information of DP and PDP is always represented relative to the UL RTOA reference time T0+tSRS as defined in TS 38.215.

For AI/ML based positioning Case 3a, the reference time for uplink channel measurements is the same as that of Case 3b, i.e., the timing information of DP and PDP is represented relative to the UL RTOA reference time T0+tSRS as defined in TS 38.215.

## Additional information included in the measurement report

In legacy positioning, measurement reports contain additional information detailing the main measurements. For example, the IE used to report RSTD in DL-TDOA also provide the origin of the measurement (PRS resource, resource set and PFL ID,) quality index, power (RSRP/RSRPP) measurements, time stamp of the measurements, and TEG information. Some auxiliary information can be used as model input as well, e.g., TRP ID which was studied in TRP reduction in Rel-18 study item stage. At the minimum, measurements reported for the input to the AI/ML model should always be associated with the configuration details of the signals used to perform the measurements (as provided in NR-DL-TDOA-MeasElement for example for DL TDOA measurement reports).

The following is currently supported for time-based methods in legacy positioning:

| Method | Report content |
| --- | --- |
| Uplink: UL-TDOA method or Multi RTT (gnB leg) method | (see Table 8.13.2.2-1 in 38.305) Measurements: - UL-RTOA / gNB Rx-Tx time difference measurement - UL-SRS-RSRP - UL-SRS-RSRPP - UL-RSCP measurement - when used with UL-AoA for hybrid positioning: - Single/multiple UL Angle of Arrival Attached information: - NCGI and TRP ID of the measurement - TRP Rx TEG - SRS Resource Type - Time stamp of the measurement - Quality for each measurement - Beam Information for each measurement (spatial correspondence) - LOS/NLOS information for each measurement - ARP ID of the measurement |
| Downlink: DL-TDOA method, UE assisted Multi-RTT (UE leg) method note: UE based does not report measurements to the LMF, only the location is reported) | Measurements: - DL RSTD or UE Rx-Tx time difference measurement - DL-PRS-RSRP measurement or DL-RSCPD measurement - DL-RSCP measurement - DL-PRS-RSRPP measurement - TA offset used by UE Attached information: - PCI, GCI, and PRS ID, ARFCN, PRS resource ID, PRS resource set ID - Time stamp of the measurement - Quality for each measurement - UE Rx TEG IDs, UE Tx TEG IDs, and UE RxTx TEG IDs for UE RxTx measurements, or UE Rx TEG IDs for DL RSTD measurements - LOS/NLOS information for UE measurements - The association of UE Tx TEG ID and SRS (RTT only) - Indication that DL-PRS bandwidth aggregation has been used for UE Rx-Tx time difference measurement or Indication that DL-PRS bandwidth aggregation has been used for DL RSTD measurement - Indication that the reported measurements are based on receiving single or multiple hops of DL-PRS - UE Rx – Tx time difference subframe offset (RTT only) - DL timing drift (RTT only) |

To support model input in case 2b and 3b, the measurements should also be followed with similar information regarding the reference signal the measurement is based on, as well as the time the measurement was acquired. Further, the measurement report should also include quality indicator for each of the measurements, e.g. for the quality of the path/sample power as well as the quality of the time measurement (e.g. the path TOA measurement or the initial time reference for sample measurements). The accuracy of the measurement is also up to RAN4 discussion, similar to previous release positioning measurements.

For Case 2b/3b, the measurement reported for input to the AI/ML model inference should include at least the following side information:

- Source of the measurement (TRP ID, PRS resource and resource set ID, PFL ID in the downlink, UL SRS resource and resource set ID in the uplink)

- Time stamp of the measurement

- Measurement quality indicator for the power information (for PDP) and timing information (for PDP, DP), for the reported path/samples.

Use of TEG indication:

TEGs (timing error groups) were introduced in Rel-17 as a way for the LMF to group measurements reported according to their relative consistency. Measurements with the same Tx TEGs and Rx TEGs were assumed to be consistent in terms of timing error. The Tx TEGs are associated with RS transmissions (e.g. the DL PRS Tx TEGs in the TRP, the UL SRS tx TEGs in the UE) and are thus found in assistance data information. The Rx TEGs are mapped to measurements and the UE can optionally provide a UE Rx TEG ID as part of a measurement element in a measurement report. The principle is that two measurements with the same UE Rx TEG ID have the same timing error margin, and could therefore be combined by the LMF.

While the TEG was useful for geometry-based location calculation, AI/ML based positioning does not follow the same principle. Rel-17 TEG was suggested to be considered in the beginning of the Rel-18 study item. However, this information was not used in companies' AI/ML evaluations throughout Rel-18 SI. The AI/ML model follows the finger-printing principle, hence TEG is implicitly incorporated in the radio finger-print by the model. For AI/ML based  positioning, the most important aspect is that there must be consistency between data collected during training data collection and data present in reported measurement for model inference. Data collected for training and the gNB (or UE) measurement reports for inference are in all likelihood not even associated with the same device, hence TEGs are not useful for aligning data between model training and model inference. Moreover, the information in AI/ ML is carried by the inter-arrival time (and power if PDP) of the channel samples/paths for each TRP-UE link. This information is thus not subject to the UE/TRP timing changes. Hence, we do not think it is valuable to include TEG information in AI/ML measurement reports.

For Case 2b/3b measurements reported for input to the LMF-side AI/ML model, do not include TEG information.

To secure consistency between measurements used for model inference and the data collected for model training, it is important that the model input is based on reference signals either identical or very close to reference signals used for model training. We provide our view on the issue in section 6.3.

## Impact of AI/ML model location for Case 3a

For Case 3a, the location of the AI/ML model will impact whether there is a need for a new measurement to be introduced. Between the LMF and the gNB, the model output generated by the NG-RAN model can be forwarded using the legacy measurement report format (e.g., RTOA). However, depending on whether the ML model is deployed at the gNB-DU or the gNB-CU, the SRS measurement entity may or may not be the same entity hosting model inference.

- If SRS measurement entity = model inference entity, then model input does not need to be specified. This is similar to Case 1 and 2a.

- If SRS measurement entity ¹ model inference entity, the input to the model (i.e., SRS measurements) may need to be specified for F1-AP.

For Case 3a, the location of the AI/ML model (e.g., gNB-DU vs gNB-CU) impacts the need to specify new measurements for model input.

Send an LS to RAN3 to request feedback on whether the input to the AI/ML model for case 3a need to be specified.

# Model output of AI/ML assisted positioning

The study of model input is commonly applicable to direct and assisted AI/ML positioning, and the same design choices can be made for model input regardless of the AI/ML approach. This is not the case for model output. While the model output is predefined (i.e., target UE location) for direct AI/ML positioning, the design choices of model output need to be considered carefully.

As discussed in section 1, we assume case 3a is an AI-extended UL-TDOA or multi RTT positioning method, while case 2a is an AI-extended DL-TDOA or multi RTT positioning method. In this section, we discuss the different aspects specific to AI/ML assisted cases (case 2a and 3a). For these cases, the following issues should be addressed:

- In order to support data collection and inference, what are the assumed model output(s)?

- What modifications to legacy measurement reporting (to the LMF) are necessary when transmitting the model output to the LMF?

## Details for reporting model output of AI/ML assisted positioning

As discussed, RAN1 has agreed to support two types of model output: (a) LOS/NLOS indicator; (b) timing information. In the following, we investigate in detail how they can be specified in Rel-19.

### LOS/NLOS indicator

In this section we discuss the following three issues:

  - What is the meaning of the LOS/NLOS indicator when reported in case 3a

  - Should the LMF be able to differentiate whether the report LOS/NLOS indicator is based on a model inference

  - Can the LOS / NLOS indicator be reported as a standalone information, or should it always be reported as a complementing information to a timing measurements

#### Interpretation of LOS/NLOS indicator, benefits for reporting LOS/NLOS indication

In legacy positioning methods, the LOS indicator can take either a binary value (LOS, NLOS) or a soft value that provides a confidence index of the LOS estimate reported by the UE or gNB. The purpose of this LOS indicator is to inform the LMF so that measurements can be grouped according to their LOS status, and measurements which have non-line-of-sight are typically excluded from the location estimation algorithm. The LOS/NLOS indicator and the measurements (e.g., RTOA) are all about the physical, observable propagation path of the radio link.

In contrast, when AI/ML assisted positioning is used, two types of LOS paths are addressed by the AI/ML model considering the two types of model output:

- LOS/NLOS indicator. As in legacy report, this model output provides information on the likelihood of a line-of-sight propagation path, i.e., a physical LOS path. For example, for Figure 20(a), LOS/NLOS indicator='TRUE' (i.e., physical LOS path exists); for Figure 20(b), LOS/NLOS indicator='FALSE' (i.e., no physical LOS path).

  - Thus: the meaning of LOS/NLOS indicator is unchanged when AI/ML is introduced to generate it.

- Time information (e.g., RTOA). The timing information generated by the model is for the direct path between TRP and UE, regardless of LOS/NLOS status of the physical link. For example, for Figure 20(a), the timing information is for the physical LOS path (=virtual LOS path); for Figure 20(b), the timing information is for the virtual LOS path.

  - Thus: the meaning of timing information is changed when AI/ML is introduced to generate it. In legacy report, the timing information is for the observed physical path arrival. In AI/ML report, the timing information is always for the direct path (i.e., virtual LOS path).

In summary, it can be understood that the LOS/NLOS indicator address the physical LOS path, while time information (e.g., RTOA) address the virtual LOS path. Here the virtual LOS path is the direct path regardless of LOS/NLOS status. In legacy DL-TDOA and DL-RTOA reports, LOS/NLOS indication is always associated with a TOA measurement. For AI/ML measurements, this association cannot be re-used, and the TOA measurement should be decoupled from the LOS indication.

| - Physical LoS path exist | - No physical LoS path |
| --- | --- |

Figure 20. Two types of LoS paths are addressed by the assisted AI/ML model.

Thus, when AI/ML assisted positioning is applied (Case 2a, 3a), the measurement report needs to clearly indicate to LMF that the timing information should be interpreted in a new way, i.e., LMF should not blindly reuse the meaning in the legacy report. Conversely, if this is not clearly indicated, then the LMF may run the legacy positioning methods the same as before, i.e., exclude links that are marked with LOS/NLOS indicator = 'FALSE'. This would completely defeat the purpose of generating enhanced timing information by AI/ML.

To indicate that the timing information for case 2a/3a is always based on LOS TOA, regardless of the true physical LOS state of the channel, it would be sufficient to add an indicator to the report, such as a “made with AI” flag, or a “virtual LOS TOA” flag.  Alternatively, the UE or gNB may be “forced” to produce an AI-based measurement as demanded in the measurement request from LMF. In that case the indication is not needed.  During RAN1#118bis, a common understanding was reached that a differentiation between a rel19 report and a rel18 (and before) report would be enough, without the need of identifying a model output explicitly, as reflected in the agreement on the report of timing information:

Agreement From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. FFS: LOS/NLOS indicator. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

The LOS/NLOS indicator in legacy positioning report is a measure of the reliability of the measurement report.

AI/ML assisted positioning can provide measurement report of the NLOS channel with high confidence.

Support reporting (optionally) a LOS/NLOS indicator when timing information is reported for Rel-19 AI/ML positioning Case 3a.

For AI/ML assisted positioning Case 3a, when the LOS/NLOS indicator is reported, from RAN1 perspective, the LOS/NLOS indicator provides information on the likelihood of a physical Line-of-Sight propagation path from the source to the receiver with a value of 1 corresponding to LOS and a value of 0 corresponding to NLOS; the LOS/NLOS indicator reuse the same format as the existing IE "LOS/NLOS Information" in 38.455.

From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, one LOS/NLOS indicator is optionally reported per TRP in a measurement report from gNB to LMF, as in the existing specification.

For case 3a and 2a, the timing measurement for the first path is associated with a virtual line of sight path.

- If a timing measurement is reported together with the LOS/NLOS indicator, the indicator status (for physical LOS/NLOS) is decoupled from the reported timing measurement (for virtual LOS)

- Note: when the TRP-UE link is LOS, the virtual LOS coincides with the physical LOS.

- Details of the association with virtual LOS is up to RAN2/3, e.g. indication in measurement report, or configuration in measurement request.

#### Reporting of a of LOS/NLOS indicator based on model output

As explained in section 4.1.1.1, the LOS information in a case 3a report must be decoupled from a timing measurement. When generated by the AI/ML model, the timing measurement is always a “LOS” measurement, that is to say a time-of-flight measurement, so that the corresponding indicator would always be a “LOS”, and thus be completely meaningless to report. A decoupled LOS estimate, reflecting the physical nature of the channel, would however convey useful information.

The definition of the LOS indicator should in our view be left unchanged, and reflect the physical nature of the channel as in release 18. What remains to discuss is whether the LOS indicator should be identified based on how it was derived, e.g. based on a model output or pre-rel19 methods (which does not say anything on how the indicator was computed).

If RAN1 wants to support the LCM for a model generating LOS NLOS beyond self-monitoring, there may be a need for identifying the LOS indicator as model output, so that e.g. the LMF may feedback to the gNB that the LOS estimate is not correct based on other information the LMF may have. Otherwise, the motivation for identifying the LOS output is not clear, since the information carried by the indicator is not changed from pre-Rel-19.

The interpretation of a LOS indicator as reflecting the physical nature of the channel should be  the same in release 19 and legacy.

If the LMF is not involved in the model monitoring of the model generating the LOS indicator, the need for identifying a LOS indicator as generated by a model output is not clear.

The LOS indicator reported for Case 3a does not need to be differentiated based on how it is computed (i.e. model output)

#### Reporting of a LOS/NLOS indicator as standalone or with timing measurements

During ran1#118bis, we discussed whether the LOS/NLOS indicator could be reported based on model output, while the timing information was computed using  legacy positioning method, i.e. pre-rel19 RTOA methods.

Proposal 3.1.5-1 For AI/ML assisted positioning, when the LOS/NLOS indicator is reported for Rel-19 Case 3a, and the other associated measurement (e.g., timing information) is obtained by legacy positioning methods, from RAN1 perspective, - The LOS/NLOS indicator provides the likelihood of a line-of-sight propagation path for the channel measured over the resource for which the measurement is reported; - The LOS/NLOS indicator can reuse the existing IE "LoS/NLoS Information" in 38.455, which can be soft indicator or hard indicator. - Note: it is up to RAN3 to decide how to differentiate reporting for Rel-19 Case 3a from reports from pre- Rel-19 positioning. FFS: whether LOS/NLOS indicator is reported when the associated timing information is generated by AI/ML, and if so, the meaning of the LOS/NLOS indicator.

RAN1#118bis also discussed two separate candidates for studies:

Proposal 3.1.5-2 For AI/ML assisted positioning, when the LOS/NLOS indicator is reported for Rel-19 Case 3a, RAN1 study whether the following reporting are supported: (1) LOS/NLOS indicator is reported together with other associated measurement (e.g., timing information), which is also reported for Rel-19 Case 3a; - If supported, study the meaning of the LOS/NLOS indicator; (2) LOS/NLOS indicator is reported without any other associated measurement (e.g., timing information); - If supported, the LOS/NLOS indicator provides the likelihood of a line-of-sight propagation path for the channel measured over the resource for which the measurement is reported.

As discussed in section 4.1.1.1 and 4.1.1.2, the LOS indication should be decoupled from the timing measurements, and whether the LOS indication is computed by AI ML is of little relevance for the LMF. It is also clear that there is a meaningful value to the LOS indicator if the indicator is the true physical channel LOS state.  Thus, we support the (1) part of the proposal, and we can keep the LOS indicator as optional as in previous releases.

For the (2) part of the proposal, we see this as very much a RAN3 issue. Again, the two measurements of timing and LOS are decoupled anyway, they are separate information items, even if they describe channel for the same time stamp.  We note, however, that the current structure defines LOS indication as complementary information to the reported measurement as seen in 38.305:

Table 21: copy of 38.305 table for content of a UL-TDOA measurement report.

Current specification only support LOS indication as additional information to another measurement.

Reporting of LOS as a separate measurement is not supported.

#### Summary of LOS/NLOS indicator reporting

In Table 22, the possible combinations of timing measurement and LOS/NLOS indicator are listed, together with their support, if applicable.

Table 22. The possible combinations of timing measurement and LOS/NLOS indicator

|  |  | Timing measurement |  |  |
| --- | --- | --- | --- | --- |
|  |  | Absent | Legacy measurement* (for physical path) | Generated by AI/ML (for virtual path) |
| LOS/NLOS indicator | Absent | N/A | Existing specification | Support in Rel-19 |
|  | Indicator for physical path (can be generated by legacy methods or by AI/ML) | N/A | Existing specification | Support in Rel-19 |

* When the associated legacy measurement is reported, it is not limited to timing measurements. It can be timing measurement or angle measurement.

For reference, in TS 38.455, LOS/NLOS indicator is reported per TRP, with one indicator for a given ULRTOA measurement (containing one or more paths) defined as follow: soft values provide the likelihood of a LOS propagation path in the range between 0 and 1 with 0.1 steps resolution. Hard value '0' indicates NLOS and hard value '1' indicates LOS.

In summary, we see LOS/NLOS indication in a Case 3a report as follow:

- The definition of LOS/NLOS indicator is the same as legacy. It is unnecessary and confusing to introduce new meaning of LOS/NLOS indicator, depending on which report format is used.

- For Case 3a, the LOS/NLOS indicator, when reported with a Rel-19 ULRTOA measurement identified as such (as agreed in RAN1#118bis), should be understood as decoupled from the timing measurement. The understanding may or may not be captured explicitly in the specifications, however, depending on whether such understanding is clear implicitly.

- The LOS/NLOS indicator can optionally be reported, the same as in legacy.

- The indicator does not need to be made a separate measurement, the same as in legacy.

Thus we propose the following revised proposals on LOS/NLOS indicator for Case 3a.

When timing information is reported for Rel-19 AI/ML positioning Case 3a, a LOS/NLOS indicator can be optionally reported, the LOS/NLOS indicator reuse the same format as the existing IE "LOS/NLOS Information" in 38.455, and its definition. If a timing measurement is reported together with the LOS/NLOS indicator, the indicator status (for physical LOS/NLOS) is decoupled from the reported timing measurement (for virtual LOS). Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

(for conclusion) For AI/ML assisted positioning, when the LOS/NLOS indicator is reported for Rel-19 Case 3a, do not support an indication of whether the LOS/NLOS indicator is obtained based on a model output.

(for conclusion) For AI/ML assisted positioning, when the LOS/NLOS indicator is reported for Rel-19 Case 3a, do not support reporting of the LOS/NLOS indicator as a separate measurement (i.e., without other associated measurement).

### Timing information

For AI/ML assisted positioning, RAN1 has agreed that the reported timing information are reported in the format of UL-RTOA, DL-RSTD, gNB RxTxTimeDiff and UE RxTxTimeDiff. Thus, the existing measurement reporting in LPP and NRPPa can be easily reused.

Reuse the existing measurement reporting in LPP and NRPPa to report timing information for the model output of AI/ML assisted positioning.

For Case 3a, after model output (UL-RTOA) is generated, post processing can be applied to the model output to generate the timing information for measurement report, where the reported timing information can follow any desired format according to the associated legacy positioning method. This is illustrated in Figure 21.

Figure 21. Illustration of postprocessing AI/ML assisted model output to provide measurements for reporting to LMF.

As illustrated in Figure 21, thanks to the postprocessing of model output, only one model needs to be trained and maintained for Case 3a, regardless of the TRP measurement result to report. This has several benefits compared to having different models for different measurement results (i.e., UL RTOA, gNB RxTxTimeDiff):

- For all LCM stages including training data collection, model training and model monitoring, the complexity and overhead are much lower when only one model is needed instead of two;

- For model inference, using one model instead of two saves chip area, memory requirement, cost, and power consumption.

- For model inference, the LMF can freely make a request for UL RTOA or gNB RxTxTimeDiff without considering that gNB may need an extra delay to switch models, if the gNB needs to activate a different model according to the measurement request.

Based on the discussion above, the following are proposed.

For AI/ML assisted positioning at gNB (Case 3a), the model output is uplink relative time of arrival (TUL-RTOA).

For AI/ML assisted positioning at gNB (Case 3a), postprocessing is applied to generate gNB RxTxTimeDiff for measurement reporting of multi-RTT method.

### On the need of multipath timing/power reporting for Case 3a

Rel16-18 legacy positioning methods support multipath reporting for time-based methods. The rational is that the paths detected by the UE and gNB are not corresponding to the true channel propagation, and that inter-path interference exists in each detected path. By getting additional information to the LMF, it is possible to at least mitigate the contribution from each path, and also assess the LOS state of the link.

In AI/ML assisted positioning, the UE and gNB models will have the possibility to detect the ideal time of arrival of the reference signal, i.e. the time-of-flight between the UE and TRP. Model training labels are based on precise coordinates for each training point. Under such condition, multipath reporting is not useful and instead may even provide more error sources. In fact, the AI/ML model is trained to derive timing information on the virtual LoS path (i.e., a single direct path between transmitter and receiver) by digesting measurements that contain the multipath information. Indeed, no evaluations on AI/ML assisted in Rel-18 SI generated measurements for multiple paths as model output. Hence, we think that single path reporting is sufficient for AI/ML assisted positioning (case 3a and 2a). however, additional path requests are under control of the network within the measurement requests using the additionalPaths fields in the measurement requests in LPP measurement request to the UE or the “Extended Additional Path List” request in NRPPa. Similarly, the first physical path RSRPP may not be the most significant metric, since the model may be able to detect a heavily obstructed first path.

For Case 2a/3a first physical path RSRPP is not an indication of quality/reliability for AI/ML generated measurement, since the model may be able to obtain accurate timing information even if the direct physical path is heavily obstructed.

As discussed, the model of AI/ML assisted approach (gNB-side or UE-side) takes multipath information as model input (PDP/DP), while the model output is timing information for the “best” LOS first path (i.e., the virtual LoS path) only. Thus, there is no need to include "additional path" in measurement reports to LMF. For measurements in "first path" IE, there is no need to include RSRPP (UL SRS-RSRPP or DL PRS-RSRPP). Only timing information needs to be reported in "first path" IE. Therefore, power information can be limited to RSRP (Optional), which is computed separately from the AI model, and is thus not part of model output.

Considering the updated WID with guidance to focus on first priority use case, our proposals are as follows:

(for conclusion) For AI/ML assisted positioning at gNB (Case 3a), the gNB is not expected to report additional paths for AI/ML based measurement report enhancements. 
Note: the network can simply omit the optional field for additional paths. It is not necessary to make specification changes to the measurement report IE.

For AI/ML assisted positioning at gNB (Case 3a), power is optionally reported using UL SRS RSRP (Case 3a). The gNB is not expected to report per-path power.

## Indication of AI/ML assistance in generating measurement reports

Based on the previous section, it is clear that there is a need to differentiate measurements generated with AI/ML model assistance from legacy measurements, since the actual interpretation of the timing measurement differs.

To identify the meaning of the timing information sent to LMF, and more generally what part of the UE/gNB report is produced with assistance from the AI/ML model, we see three possible options:

- Option 1: A new report message specific to AI/ML assisted positioning cases 3a/2a

- Option 2: an indicator in the existing report, indicating that the report was produced using AI/ML assistance.

- Option 3: an indicator is attached to the elements of the reports where AI/ML was used to generate it.

All options could be discussed by RAN3/RAN2 in terms of signaling optimization, but RAN1 is responsible for answering whether the LMF needs to know exactly what part of the report was using AIML.  In our understanding, Case 3a and Case 2a measurement reports will consist of measurements only produced by AI/ML. For example, line of sight indication, UL-RTOA, DL RSTD measurement will be based on the model output. Hence from our perspective, it is sufficient to indicate to the LMF that the measurement report as a whole is based on AI/ML model output, without having to tag each quantity in the report (e.g. LOS indication, RSTD measurement, additional measurement, etc) with a “made with AI” indicator.

For AI/ML assisted positioning at gNB (Case 3a) and UE (Case 2a), the measurement report contains an indicator that AI/ML is used to produce the measurements. The signaling details are up to RAN3/RAN2.

# Training data collection

## Content of training data samples

For the data samples to be collected for training, the following agreement was made in RAN1#116bis.

Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement

For supervised learning, ground truth label needs to be collected for each entry of training data. On the other hand, it may be difficult to obtain a large number of accurate ground truth labels. When ground truth label is not available, the measurement data (corresponding to model input) can still be collected and used by semi-supervised learning. Thus both labelled and un-labelled training data samples can be collected.

For Rel-19 AI/ML based positioning, support collecting labelled and optionally un-labelled training data samples.

In the following sub-sections, details on Part A and Part B are further discussed.

### Time stamp of training data samples

In RAN1#118, the following agreement was made on the time stamp of channel measurement:

Agreement For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective - Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications

Also, the following proposals were discussed in RAN1#118:

Proposal 4.1.3-2A For training data collection of AI/ML based positioning, for time stamp of channel measurement, the existing IEs are reused. - For channel measurement generated by TRP/gNB, reuse the existing IE “Time Stamp” in TS 38.455, with field “Measurement time” mandatory. - FFS: the time stamp for channel measurement by PRU or non-PRU UE Proposal 4.1.3-2B For training data collection of AI/ML based positioning, for time stamp of label, the existing IEs are reused. - For label generated by PRU or non-PRU UE, reuse the existing IE both NR-TimeStamp and UTCTime (e.g., for utc-time-r16) in TS 37.355; - NR-TimeStamp and UTCTime are intended to refer to the same time.

In RAN2#127, the following agreement was made on NW side data collection:

Agreements - As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support. - UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. This is across all use cases When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging. Measurements for data collection purposes and logging based can be controlled based on power state of the UE. It is up to UE implementation how the UE determines power state. FFS whether the UE stops autonomously or if it reports to the network.

As can be seen in RAN2 agreement above, the baseline procedure is that the UE stores training data in its memory, and report a batch of logged data at a time. That is, the training data reporting time and measurement time are separate, and can be far apart, i.e., likely the reporting time is far later than the SFN wrap-around time of 10240 ms (=10.24 sec). Therefore, one cannot rely on an entity that receives the training data to add time stamp; rather the measurement entity itself should add time stamp as the training data is being collected.

While the RAN2 agreement above is on UE procedure, it is reasonable to expect that the same principle applies when the training data generation entity is on the NW-side (i.e., gNB or LMF for positioning). RAN3 specifications support the use of periodicity of up to 30 minutes for reporting of measurements. In that case, SFN/slot level time stamps become ambiguous and a measurement time reference is necessary.

To clear the ambiguity, the report should always include measurement time whenever the reporting periodicity is too large for the reported measurement to be unambiguously mapped to a time instant using only SFN/slot numbers. This can be handled by RAN2 and RAN3 groups, but RAN1 should at least agree on the need for unambiguous mapping between part A and part B.

Thus the time stamp for both Part A and Part B should be mandatorily added by the training data generation entity, and the time stamp should use a much longer range than SFN. Specifically,

- for channel measurement and label generated by PRU or non-PRU UE, always include the existing IE UTCTime (e.g., for utc-time-r16) in TS 37.355 (i.e., not a choice between SNF and UTCTime);

- for channel measurement generated by TRP/gNB, either change the field “Measurement time” in TS 38.455 from optional to mandatory, or specify that the field “Measurement time” is present whenever the measurement reporting periodicity exceeds 10.24 sec.

Regarding the time stamp resolution, several different granularities exist in current specifications, for example:

  - LPP:

    - UTCTime with a granularity of seconds;

    - utcTime-ms with a granularity of milliseconds;

  - “Measurement time” with a granularity of 1 /2**32 second.

In our view, there is no need to align to a same granularity for time stamp of Part A and Part B, and existing IEs can be reused. Regarding how close the stamped time should be between Part A and B so that they can be paired, this is likely related to the positioning accuracy requirement, UE speed, and also related to the implementation of training data collection entity and model training entity. From RAN1 perspective, there is no need to impose any restriction on how to perform the pairing using the time stamps, including the allowed time difference between Part A/B so that they can be paired.

For training data collection of AI/ML based positioning, for both Part A and Part B, time stamp is mandatorily provided by the training data generation entity.

For training data collection of AI/ML based positioning, the time stamp should allow unambiguous mapping between Part A and Part B.

  - For channel measurement generated by TRP/gNB, at least when measurement periodicity exceeds the SFN/slot wrap around time (10.24 sec), field “Measurement time” is included in the time stamp.

  - For channel measurement and label generated by PRU or non-PRU UE, UTC Time (e.g., for utc-time-r16) is included in the time stamp.

  - Exactly how to specify time stamp for Part A and Part B is up to RAN2/RAN3.

From RAN1 perspective, the existing IEs and their associated granularities can be reused for time stamp of Part A and Part B. It is acceptable to have different time stamp granularity for Part A and Part B. There is no need for RAN1 to discuss how to perform the pairing using the time stamps.

For reference, existing signaling in 37.355 (LPP):

| Terrestrial Beacon System Positioning | TBS-MeasurementInformation-r13 ::= SEQUENCE { measurementReferenceTime-r13 UTCTime OPTIONAL, mbs-SgnMeasList-r13 MBS-BeaconMeasList-r13 OPTIONAL, -- Cond MBS … } measurementReferenceTime This field provides the UTC time when the TBS measurements are performed and should take the form of YYMMDDhhmmssZ.. |
| --- | --- |
| NR DL-TDOA Positioning | NR-DL-TDOA-LocationInformation-r16 ::= SEQUENCE { measurementReferenceTime-r16 CHOICE { systemFrameNumber-r16 NR-TimeStamp-r16, utc-time-r16 UTCTime, … } OPTIONAL, measurementReferenceTime This field specifies the time for which the location estimate is valid.. |
| NR DL-AoD Positioning | NR-DL-AoD-LocationInformation-r16 ::= SEQUENCE { measurementReferenceTime-r16 CHOICE { sfn-time-r16 NR-TimeStamp-r16, utc-time-r16 UTCTime, measurementReferenceTime This field specifies the time for which the location estimate is valid.. |

UTC-Time-r15 ::= SEQUENCE {

utcTime-r15					UTCTime,

utcTime-ms-r15				INTEGER (0..999),

...

}

utcTime This field provides the time stamp of the refTime in UTC time and comprises the following subfields: - utcTime in the form of YYMMDDhhmmssZ. - utcTime-ms specifies the fractional part of the UTC time in ms resolution.

For reference, existing signaling in 38.455 (NRPPa):

9.2.42		Time Stamp

This information element contains the time stamp.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| System Frame Number | M |  | INTEGER(0..1023) |  |
| CHOICE Slot Index | M |  |  |  |
| >SCS-15 |  |  | INTEGER(0..9) |  |
| >SCS-30 |  |  | INTEGER(0..19) |  |
| >SCS-60 |  |  | INTEGER(0..39) |  |
| >SCS-120 |  |  | INTEGER(0..79) |  |
| Measurement time | O |  | Relative Time 1900 9.2.36 |  |

9.2.36	Relative Time 1900

This information element indicates the initialisation time (e.g. SFN Initalisation Time for a cell, requested time for an action, etc).

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Relative Time 1900 | M |  | BIT STRING (SIZE(64)) | Time in seconds relative to 00:00:00 on 1 January 1900 (calculated as continuous time without leap seconds and traceable to a common time reference) where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. The fraction part is expressed with a granularity of 1 /2**32 second |

### Signalling and quality reporting for Part A

From the RAN 1 perspective, data collection for Part A is the same procedure as measurement reporting /requests for inference. Additionally, legacy measurement request may be re-used.

For Case 1 and Case 2a, data collection for Part A between a UE and another PRU UEs is already supported by specification. For data collection between non-PRU UEs, 3GPP should not specify a method for data collection. Rather, an OTT solution is assumed.

From the RAN1 perspective, current specification support for Part A data collection in Case 1 is sufficient.

For Case 3b and 2b, data collection for Part A should not be different from measurement request during inference. For Case 3a with model in the gNB (CU if split architecture) the situation is the same.

From the RAN1 perspective, Part A data collection in Case 2b and 3b is identical to measurement reporting for inference.

For Case 3a with model in the TRP, the TRP can handle its own measurements. Thus measurement request for Part A in Case 3a is not an issue.

From the RAN1 perspective, Part A data collection in Case 3a is either up to the network implementation or identical to measurement reporting for inference.

Based on this analysis, RAN1 does not need further discussion on specific procedure for Part A data collection.

RAN1 does not need to discuss further signalling to enable Part A data collection.  Measurement reporting is jointly discussed for training data collection and inference.

For quality indication of model input, i.e. Part A, the measurement quality can also be jointly discussed for training data collection and inference, since the same measurement quality indicator type is expected to be used for measurement reported during inference.

Regarding timing measurements in Part A, e.g., the sample timing in PDP and DP, it was agreed in RAN1#118bis that the existing IE for timing quality is reused, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455.

Regarding power measurements in Part A for PDP, quality indicator similar to NR-TimingQuality can be provided. An example is shown below.

NR-PowerQuality-r19 ::= SEQUENCE {

powerQualityValue-r19			INTEGER (0..9),

powerQualityResolution-r19		ENUMERATED {dBdot1, dB1,…},

...

}

NR-PowerQuality field descriptions

powerQualityValue This field provides an estimate of uncertainty ±X dB of the power value for which the IE NR-PowerQuality is provided in units of dB. Represents the deviation (uncertainty) in power as compared to what is reported for power level. Scale factor is provided in powerQualityResolution.

powerQualityResolution This field provides the resolution used in the powerQualityValue field. Enumerated values dBdot1, dB1 correspond to 0.1, 1 dB, respectively.

For training data collection in all cases, the quality of power measurements is provided for PDP in Part A in units of dB. The candidate resolution includes 0.1 dB and 1 dB.

For reference, existing signaling in 37.355 (LPP):

–	NR-TimingQuality

The IE NR-TimingQuality defines the quality of a timing value (e.g., of a TOA measurement).

-- ASN1START

NR-TimingQuality-r16 ::= SEQUENCE {

timingQualityValue-r16			INTEGER (0..31),

timingQualityResolution-r16		ENUMERATED {mdot1, m1, m10, m30, …},

…

}

-- ASN1STOP

NR-TimingQuality field descriptions

timingQualityValue This field provides an estimate of uncertainty of the timing value for which the IE NR-TimingQuality is provided in units of metres.

timingQualityResolution This field provides the resolution used in the timingQualityValue field. Enumerated values mdot1, m1, m10, m30 correspond to 0.1, 1, 10, 30 metres, respectively.

### Signalling and quality reporting for Part B

For training data collection of Part B, the label generation entities for each case is already agreed by RAN1.  The working assumptions could be confirmed in order to close the discussion.

Confirm the existing WAs on label generation for all cases and send the agreements to RAN2.

Regarding ground truth label in Part B, in our view, the training dataset is agnostic of the model architecture and model training methodology. To reduce the burden of collecting, storing, and updating training datasets, a same training dataset should be usable to train a variety of AI/ML models for positioning.

- For DL measurement, it should be possible to use the same collected training dataset to train a model for Case 1, and/or 2a, and/or 2b.

- For UL measurement, it should be possible to use the same collected training dataset to train a model for Case 3a and/or 3b.

- For AI/ML assisted positioning, it should be possible to use the same collected training dataset to train a model that produces {timing measurement} as output, or produces {timing measurement, LOS/NLOS indicator} as output.

The details of whether/how to train a range of models using a given training dataset is up to each vendors’ design choice.

Considering the above, for all the positioning cases, the ground truth label should be collected in the format of UE/PRU location coordinates (x, y, z). This has the benefit of convenient data collection (as compared to ground truth for RTOA or LOS/NLOS indicator, for example). This also makes the training dataset versatile and generally applicable to all positioning cases.

Then, depending on the design choice of the model, the collected label can be further processed either before model training or during model training. The details are up to implementation. For example, if it is desired to train a Case 3a (or 2a) model with RTOA as model output, then the RTOA ground truth can be calculated from the collected UE/PRU location coordinates (x, y, z), based on the knowledge of TRP locations. Similarly, if it is desired to train a Case 3a (or 2a) model with LOS/NLOS indicator as model output, then the model training procedure can be designed to use coordinates (x, y, z) as proxy labels.

Training data collection methods traditionally rely on direct ground-truth labels on radio measurements, which can be difficult, costly, and even impossible to obtain. It is possible to circumvent those issues by using "ground truth" position data as indirect labels, rather than directly labelling radio measurements.

If the estimated UE position p is obtained by minimizing a loss function fθp with respect to p, assuming a vector of neural network outputs θ=θ1⋯θm∈Rm containing e.g., TOAs, soft LOS indicators, etc., then the derivative of p with respect to θj is:

∂p∂θk=-Hpfθp-1∇p∂fθ∂θkp

where Hpfθp is the Hessian with respect to p at p. This expression can be used in training to back propagate over this optimization step using a ground truth label for the UE position. By leveraging real-world location data as a proxy, this method enables the model to infer labels on radio measurements (such as soft LOS indicators, etc) indirectly during training.

This technique involves back-propagation across both trainable and untrainable optimization steps, allowing the model’s outputs to align with the desired positioning estimates without needing direct measurement labels. By substituting position data for radio measurement labels, the approach enhances the flexibility and scalability of model training. This indirect labelling strategy not only simplifies the data collection process but also improves the model's adaptability across different network environments, facilitating accurate and efficient positioning.

For training data collection Part B, support only ground truth labels in the format of UE location coordinates (x, y, z).

For label quality, RAN1 extensively discussed how to attach a quality to part B, with the following being the latest proposal from RAN1#118bis:

Proposal 4.1.5-1A For training data collection of AI/ML based positioning Cases 1, 3a, 3b, from RAN1 perspective, when label reporting is applicable, quality indicator of label is represented using one of the following options. RAN1 study and select between Option 1 and Option 2. - Option 1: reuse an existing IE. For each case, proponent companies provide input on the existing IE to use. - Option 2: define a new IE on UE location quality. Use hard (0 or 1) and soft values (0, 0.1, 0.2, …, 1.0), with value 1 indicating the highest quality. - Proponent companies provide information on how to determine the value, entity to generate the label quality, and the applicable signaling.

Option 1 can already be supported by a UE or LMF for positioning solutions prior to Rel-19.  Note that when providing a location information, a UE also reports the method used to generate that information (e.g. GNSS, WLAN, DL-TDOA).

Option 2 provides a generic metric for quality. However, the quality level depends also on the specific positioning use case,  the desired positioning accuracy, and the AI/ML model design (e.g., structure, size, parameter quantization, etc). Such information may not be known at the time the training data is collected. Furthermore, a given training data set can be used to train multiple models, potentially each with a different accuracy target and a different model design. Depending on the training context, as an example, some deployment/model may treat 1m accuracy as low quality, while another treat 1m accuracy as high quality. Also, Option 2 requires high standardization effort to define the quality values, e.g., how to determine the quality value given a UE location error in meters.

Hence, keeping legacy quality metrics is preferable. We support Option 1, and don’t support Option 2.

Regarding the specific quality metrics, the existing IE LocationUncertainty has been defined to provide horizontal and vertical information for ARP, as shown below from LPP. Similar IEs exist for NRPPa. Label uncertainty is analogous to ARP location in that these location information are considered known, though with error margins in real-world applications. This is in contrast to location of target UE, which is unknown, and to be determined using the positioning methods.

locationUNC-r16				LocationUncertainty-r16				OPTIONAL,		-- Need OP

LocationUncertainty-r16 ::= SEQUENCE {

horizontalUncertainty-r16				INTEGER (0..255),

horizontalConfidence-r16				INTEGER (0..100),

verticalUncertainty-r16					INTEGER (0..255),

verticalConfidence-r16					INTEGER (0..100)

}

locationUNC This field specifies the uncertainty of the location coordinates and comprises the following sub-fields: - horizontalUncertainty indicates the horizontal uncertainty of the ARP latitude/longitude. The ′horizontalUncertainty′ corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and ′horizontalConfidence′ corresponds to confidence as defined in TS 23.032 [15]. - verticalUncertainty indicates the vertical uncertainty of the ARP altitude. The ‘verticalUncertainty’ corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and ‘verticalConfidence’ corresponds to confidence as defined in TS 23.032 [15]. If this field is absent, the uncertainty is the same as for the associated reference point location.

For horizontal and vertical uncertainty, the value range of (0..255) indicate a location error range between 0 and 46.49129 metres, with the smallest indicated value of 6 millimeters, see Appendix A. In our view, the existing horizontalUncertainty-r16 and verticalUncertainty-r16 can be reused to indicate the UE location quality for labels, including the encoded high accuracy uncertainty as defined in TS 23.032. The definition of the fields would naturally clarify that the uncertainty values are for UE location (PRU or non-PRU UE), instead of ARP.

Additionally, one question is whether the label quality should be provided differently depending on model output, for example, timing information (UL RTOA) for assisted positioning or UE location for direct positioning. In our view, the training dataset is agnostic of the model architecture and model training methodology. Thus, for the purpose of training data collection, label quality only needs to be recorded as UE location error in meters. If the model training entity has a need of a quality of an intermediate metric (e.g., UL RTOA), then such quality can be calculated using the UE location error in meters. Such procedure is out of scope for RAN1 standardization in our view.

From RAN1 perspective, the same label (UE location) and label quality (UE location uncertainty) are collected, irrespective of the type of model architecture (e.g., assisted or direct positioning) that may be selected by the model training entity.

For training data collection, the label quality for Part B reuse the existing uncertainty IEs for indicating UE location error, e.g., horizontalUncertainty-r16 and verticalUncertainty-r16.

### Overhead reduction for label delivery

When the UE is either static or slowly moving, it is not efficient for labels to be recorded for every measurement. Instead, a single label can be applied for multiple time stamps, or a span of time stamps.

A label provided by a LMF, PRU, or non-PRU UE can be attached with one or more timestamp, or a span of time stamps, for which the label is valid.

## Metadata for training dataset

### Elements in metadata of the training dataset

To keep consistency between training and inference phase, it is crucial that the node operating the model can secure that the assistance data and signals received during inference are the same as what was used for training the model. For this purpose, in addition to the list of training data samples, the training dataset also needs to record the context information of the collected data. Such context information is attached to the training dataset as its metadata. The context information should include basic information like:

- Where the training data is collected

- When the training data is collected, for example, year/month/date

- Which nodes provided the measurement data, for example, TRP ID, UE ID.

- Which nodes provided the label data, for example, PRU ID, UE ID, LMF ID.

- What kind of reference signals are used in generating the measurement data, for example, PRS configuration for DL measurement, SRS configuration for UL measurement.

- Type and format of the measurement data, for example,

  - Type (e.g., DP/PDP) and size (e.g., Nt, N't) of the measurement data

  - Quantization granularity used in recording the measurement values

- Type and format of the label data, for example,

  - Type of information for label data, for example, UE location or information corresponding to model output of AI/ML assisted positioning models;

  - Format used to record the location information (for labels),

  - 2D or 3D location coordinates

- Criteria (if any) applied before a data sample can be accepted as part of the training dataset, for example, label quality requirement, SINR requirement, etc.

- Version ID of the training dataset, if the training dataset may be updated over time.

Regarding the criteria applied to collect data samples of adequate quality, it should cover both measurement data (corresponding to model input) and label data (corresponding to model output). This gives the high-level information on the training dataset:

- The accepted range of measurement data quality.

- The accepted range of label error (or: uncertainty of ground truth label).

When the training dataset is used for training a model, the model training entity may further decide to include only data samples with sufficient quality, where the quality is determined according to the targeted positioning accuracy requirement. For instance, during training data collection, data samples with label error < 2.0 meters are collected into the training dataset. During model training, two models may be trained, one model is trained using filtered data samples with label error < 1.0 m for higher accuracy applications, while the other using all data samples (thus label error < 2.0 m) for lower accuracy applications.

Specific to training data collection of Part A, the model host (TRP, gNB, LMF or UE)  will integrate measurements together with related metadata. To enable accurate inference result, the metadata should ideally not differ between the training data set and the measurement performed during inference.  The metadata that need to be recorded for Part A of the training dataset can be grouped as follows:

- Group 1, assistance data.  In the positioning framework, assistance data is sent from LMF to either gNBs or UEs to enable reference signal measurements. For training data collection, the LMF should provide suitable assistance data to the measuring nodes.

- Group 2, signal measurement configuration, for example, PDP or DP, measurement parameters {Nt, Nt', k}. For Case 2b, 3b, and potentially 3a, this is the information included the measurement request, sent from LMF to UE or gNB, for model inference. For cases where the channel measurement is not sent to LMF for model inference (Case 1, 2a, and potentially 3a), the analogous signal measurement information should be recorded as well.

- Group 3, measurement environment condition and quality. This may include: network synchronization error range, UE/gNB RX and TX timing error range, interference range, channel estimation error range, etc.

Thus RAN1 should discuss for each of these groups firstly whether new signalling is needed, and second how to keep consistency between training and inference.

For Group 3, Rel-18 study item evaluations show that such context information also affect the validity of the trained model in the deployment scenario. Thus such context information about the training dataset also needs to be recorded as part of the metadata, including:

        - Range of SNR/SINR

        - Range of Channel estimation error

        - Range of NW synchronization error

        - Range of UE/gNB RX and TX timing error

        - …

The training dataset is attached with metadata which describes the context information on the set of collected training data samples.

For training data collection of Part A, the metadata includes: Group 1 for assistance data, Group 2 for signal measurement configuration, and Group 3 for measurement environment condition and quality.

RAN1 to study the details of Group 3 metadata to attach to training dataset, including:

- Range of SNR/SINR

- Range of Channel estimation error

- Range of NW synchronization error

- Range of UE/gNB RX and TX timing error

The metadata of the training dataset is not only necessary for keeping track of the context and versions of the training dataset, it also provides context information of model(s) trained with the given training dataset. During model deployment, the context information of the model is needed to ensure consistency between model training and model inference. Further details are provided in section 7.1 and 7.2.

Furthermore, the AI/ML solution needs to secure a training dataset of adequate quality as a whole.

- Collect a sufficiently large size of training dataset.

- Collect data that fully reflect the UE distribution during operation, e.g., uniformly distributed over the entire floor.

- Collect data from a variety of UEs to generalize over different UE implementations, including different UE vendors, different UE capabilities, different UE orientation, etc.

While the training data collection procedure needs to pay attention to the above issues, they are expected to be taken care of via engineering effort and not explicit in the standard specification.

Collecting sufficient data in terms of training dataset size, UE distribution, diversity of UE sources, etc, is up to implementation.

### Assistance data for consistency between training and inference

Once the assistance data for data collection is established, the AI/ML positioning framework should also enable consistency between assistance data received during data collection and inference.  In our view, such consistency can be achieved to a large degree using a validity area attached to the training data set.  During RAN1#117, it was discussed whether a training data set validity area should be explicitly attached to a training data set, i.e via a validity area ID, or if the training data set should contain information sufficient for a node receiving the training data set to determine the validity area of the model being trained and leave the consistency check to implementation.  In our view, explicit definition of the model validity area is needed.

We see the need of validity area for the following reason:

- Consistency of assistance data between data collection and inference. In case 1 or 2a, the UE perform data collection with a given set of assistance data and train its model, then receive assistance data at the inference phase. The AD for inference must at least overlap with the AD used during data collection to train the model. The UE could of course get a set of AD and request what AD is missing via on-demand PRS request, but this is inefficient signalling, since it is possible for the LMF and the UE to handshake which AD is needed during inference thanks to an agreed upon validity area established during data collection, if a validity area ID is defined.

- For the LMF, knowledge of the validity area of a model can also be used to decide whether the model is compatible with the AD it intends to send. For example, the network may have changed the PRS configuration between data collection and inference, making the UE untrained for the new deployment. From the overhead perspective, is more efficient for the network to know in advance if the AD is useful and thus know that starting the procedure for AI use cases is worth it, or if legacy positioning should be used instead. Hence knowledge of validity area is also valuable at the LMF side.

Hence, a training validity area ID associated with the UE’s model training dataset validity area can reduce the amount of overhead to align assistance data between training and inference.

The validity area is clearly an important component of the assistance data. Considering the model generality issues studied in Rel-18 SI, finger-printing based AI/ML methods are very sensitive to the deployment area. A model trained for one deployment area is unlikely to work for a different deployment area without any change. For the training data collection stage, it is necessary to define a training dataset validity area Adataset. This describes the area that training dataset is collected, and can be defined as:

| Training dataset validity area | The network area for which the training dataset is valid. |
| --- | --- |

Similar to the existing 'AreaID-CellList', the validity area for AI/ML operation can be defined as a list of TRPs, where the TRPs transmits DL PRS and covers the surrounding area to support positioning. Checking the indication of the TRP ID in TS 37.355, it includes dl-PRS-ID-r16 in addition to the fields in NR-Cell-IDs-r17.  Nevertheless, this shows that an IE can be easily defined to provide area information using a list of TRP IDs.

TS 37.355:

– AreaID-CellList The IE AreaID-CellList provides the NR Cell-IDs of the TRPs belonging to a particular network area where the associated assistance data are valid. Each cell is included in only one area. -- ASN1START AreaID-CellList-r17 ::= SEQUENCE (SIZE(1..maxCellIDsPerArea-r17)) OF NR-Cell-IDs-r17 NR-Cell-IDs-r17 ::= SEQUENCE { nr-CellGlobalID-r17 NCGI-r15 OPTIONAL, -- Need ON nr-PhysCellID-r17 NR-PhysCellID-r16 OPTIONAL, -- Need ON nr-ARFCN-r17 ARFCN-ValueNR-r15 OPTIONAL, -- Need ON ... } -- ASN1STOP

TRP-LocationInfoElement-r16 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255), nr-PhysCellID-r16 NR-PhysCellID-r16 OPTIONAL, -- Need ON nr-CellGlobalID-r16 NCGI-r15 OPTIONAL, -- Need ON nr-ARFCN-r16 ARFCN-ValueNR-r15 OPTIONAL, -- Need ON associated-DL-PRS-ID-r16 INTEGER (0..255) OPTIONAL, -- Need OP trp-Location-r16 RelativeLocation-r16 OPTIONAL, -- Need OP … }

Therefore, the training dataset validity area can be defined using the following:

– AreaID-TRPList The IE AreaID-TRPList provides the NR TRP IDs of the TRPs belonging to a particular network area. For AI/ML training data collection, AreaID-TRPList provides the network area where the training data is collected. For a trained AI/ML model, AreaID-TRPList provides the network area where the AI/ML model is valid. A TRP ID can be included in different network areas described by AreaID-TRPList. -- ASN1START AreaID-TRPList-r19 ::= SEQUENCE (SIZE(1..maxTRPIDsPerArea-r19)) OF NR-TRP-IDs-r19 NR-TRP-IDs-r19 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255), nr-CellGlobalID-r17 NCGI-r15 OPTIONAL, -- Need ON nr-PhysCellID-r17 NR-PhysCellID-r16 OPTIONAL, -- Need ON nr-ARFCN-r17 ARFCN-ValueNR-r15 OPTIONAL, -- Need ON ... } -- ASN1STOP

The training dataset validity area can be recorded as a part of the meta data for the training dataset, where the metadata describes the model training context. With the list of TRPs recorded as a part of the meta data for the training dataset, then the measurements collected for each data sample can be organized with the same hierarchy as for legacy assistance data, i.e. per TRP/frequency layers for the UE, and per TRP for the gNB assistance data.

The training dataset validity area is recorded as a part of metadata of the training dataset. The validity area can be defined as a list of TRPs where data is collected.

Training data provided to UE/gNB is organized with the same hierarchy as for legacy assistance data, i.e. per TRP/frequency layers for the UE, and per TRP for the gNB assistance data.

If the UE collects measurement data based on PRS from a list of TRPs, then the UE record the associated TRP IDs as the validity area of the training data.  Assistance data on the PRS configuration, PRS index used in measurement, etc, are also recorded as metadata of training dataset as well.

Furthermore, while the validity area and the RS assistance data are very similar to the assistance data for legacy positioning methods, a new assistance data needs to be introduced for associated ID. For AI/ML based positioning, the associated ID indicates the additional network conditions, and provides information on the consistency between training data collection and model inference, see related discussion in section 7.2.3.

In summary, for Case 1/2a, the assistance data for training data collection includes:

  - Training dataset validity area,

  - The assistance data necessary for PRS reception, i.e., those marked as “E” (i.e., Explicit) in Table 23;

  - Associated ID to indicate the additional network conditions. The additional network conditions are those that are not explicitly signaled but may affect the consistency between training data collection and model inference. Details of associated ID are discussed further in section 7.1.

For Part A training data collection of Case 1/2a, the assistance data include: training dataset validity area, assistance data necessary for PRS reception, and associated ID for additional network conditions. Corresponding assistance data is sent during model inference as well.

The assistance data required for PRS/SRS reception is the same across all use cases for both direct and AI/ML assisted positioning.

# Model performance monitoring

## Principles for model performance monitoring

For AI/ML based positioning, model performance monitoring was studied in Rel-18 study item phase. Both label-based methods and label-free methods were explored. If true ground truth label can be found, it is preferred to use label-based method and monitor the accuracy of model output directly. However, for real life deployment, the problem is, ground truth label or its estimation is rarely available during model deployment.

For label-free model performance monitoring, as discussed in Rel-18 SI, the monitoring metrics can be various formats depending on the method:

- statistics of measurement(s) compared to the statistics associated with the training data,

- statistics associated with the model output,

- relative displacement,

- inference output inconsistency (e.g., approach 3 in section 11.1.3),

- …

There is no effort to acquire ground truth label or its approximation or substitution. The calculation of monitoring metric is up to implementation. It is not feasible to align the monitoring metric among the different monitoring methods, or align on a threshold for decision making at the end of model monitoring. Hence the entire model performance monitoring procedure is self-contained within the model inference entity, i.e., no need to receive assistance data on ground truth label; no need to send monitoring metric to another entity; no need to receive thresholds or criteria for decision making. The only potential specification impact is to notify other entities of the model monitoring decision, if necessary.

Label-free model performance monitoring is up to implementation of model inference entity. The only potential specification impact is to notify other entities of the model monitoring decision, if necessary.

For label-based model performance monitoring, as discussed in Rel-18 SI, the monitoring metric is:

- statistics of the difference between model output and provided ground truth label (or its approximation/substitution).

Thus there is a need to provide ground truth label (or its approximation/substitution).

- If certain information on the ground truth label can be generated by the model inference entity (e.g., a variant of approach 1 of section 11.1.1), then such label-based model performance monitoring is also self-contained and up to implementation of model inference entity.

- On the other hand, if the information on the ground truth label has to be provided by another entity, then signaling needs to be specified so that model output and ground truth label information are brought together to the model monitoring entity. If the model monitoring entity is different than the model inference entity, then signaling is also needed to send the monitoring decision.

  - Still, it is infeasible to align the exact monitoring metric calculation, or align on a threshold for decision making at the end of model monitoring. Thus a same entity should perform both monitoring metric calculation and monitoring decision making regarding how to handle the model.  Note that the network is still in charge of the positioning procedure at large, and may decide to interrupt the positioning procedure based on received information.

If certain information on the ground truth label can be generated by the model inference entity, then such label-based model performance monitoring is also self-contained and up to implementation of model inference entity. The only potential specification impact is to notify other entities of the model monitoring decision, if necessary.

Another consideration for model performance monitoring is, there are two levels of performance monitoring, decision making, and related management procedures. One is the model-level performance monitoring and management, the other is functionality-level performance monitoring and management.

For the model-level performance monitoring and management, this includes model performance monitoring metric calculation, the decision making on whether the model performs satisfactorily, and actions to take if the model no longer performs well (e.g., deactivate the current model, switch to an alternative model, trigger model update). For this set of procedure, only the model inference entity has the necessary information to carry out, for example, UE for Case 1, gNB for Case 3a. External entities (e.g., LMF) has no knowledge about the details on the model to judge. There are many detailed information on the model that can affect the performance monitoring and management decisions, including:

  - the type/size/granularity of channel measurements used as model input,

  - the model structure (e.g., distributed or centralized model),

  - how the model was trained (e.g., whether the model can accommodate a varying number of active TRPs),

  - the generalization capability of the trained model (e.g., whether/what mixed dataset was used in training),

  - pre-processing and feature engineering applied at model input,

  - postprocessing applied at model output,

  - whether an alternative model is available, and if so, the details of the alternative model,

  - whether a performance degradation is intentional and temporary due to runtime constraints of the model host, e.g., power consumption, memory, processing pipeline, coexistence with other functionalities, etc.

With the above considerations, it is clear that the model-level performance monitoring and management have to be handled by the model inference entity, i.e., UE for Case 1, gNB for Case 3a. In other words, self-monitoring should be supported as a baseline, with the detailed procedure up to the implementation.

For direct positioning, the model inference entity should act as the core entity that performs model monitoring. This includes calculation of model monitoring metrics and making (preliminary) model monitoring decision. To support the model monitoring procedure, assistance information may be provided between nodes, for example, LMF may send LOS/NLOS assistance information to UE for Case 1. This is much more efficient than UE sends raw information about the model input and/or model output to LMF, and LMF calculates the model monitoring metrics for the UE.

For Case 2b and 3b, LMF is the model inference entity. Thus there is no need to define signaling for reporting model performance monitoring decisions. There can be, however, additional signaling to support the LMF decision making in model monitoring. One need is assistance information from UE/gNB to LMF to provide LoS link information (see section 11.1.1). Also LMF could request support from UE/gNB to validate the AIML model input.

For assisted positioning, the model inference entity should also act as the core entity that performs model monitoring for itself. To support the calculation of model monitoring metrics, the LMF can help by providing the estimated ground truth to the model inference entity (i.e., UE for Case 2a, gNB for Case 3a), where the estimated ground truth is generated by the LMF using the associated conventional positioning methods.

For functionality-level performance monitoring and management, LMF should continue to be responsible. That is, with the introduction of AI/ML based positioning, LMF continues to maintain its role as described in TS 38.305:

“For positioning of a target UE, the LMF decides on the position methods to be used, based on factors that may include the LCS Client type, the required QoS, UE positioning capabilities, gNB positioning capabilities and ng-eNB positioning capabilities. The LMF then invokes these positioning methods in the UE, serving gNB and/or serving ng-eNB. The positioning methods may yield a location estimate for UE-based position methods and/or positioning measurements for UE-assisted and network-based position methods. The LMF may combine all the received results and determine a single location estimate for the target UE (hybrid positioning). Additional information like accuracy of the location estimate and velocity may also be determined.”

In general, LMF is responsible for selecting/activating the desired positioning methods, taking into account the positioning accuracy requirement. LMF manage the positioning functionality, but it does not have detailed information to manage individual models residing at gNB or UE.

For AI/ML based positioning, support self-monitoring by model inference entity as a baseline. Furthermore, self-contained monitoring is preferred, where no estimated label needs to be signaled from external entities. Only when necessary, introduce signaling to provide information on ground truth label to the model inference entity.

The model inference entity is responsible for model-level performance monitoring and management, including calculation of model monitoring metrics. The detailed algorithm and procedure are up to implementation.

For model inference at UE (Case 1, 2a) or gNB (Case 3a), the LMF can provide assistance information to support the calculation of model monitoring metrics.

For AI/ML based positioning, LMF is responsible for functionality-level performance monitoring and management.

## Model performance monitoring metric calculation for Case 1

Several label-based options for case 1 were proposed for further study in the last RAN1 meeting:

RAN1 #116-bis Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side. - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

In RAN1#118, it was concluded that Option A-4 does not need to be further discussed.

In our view these label-based performance monitoring proposals add complexity to the NR specifications and increase the implementation costs. Some of these proposals even elevates the costs of operating the positioning systems. These label-based monitoring proposals should be compared to label-free monitoring approaches in terms of complexity as well as effectiveness. That is, quantitative evaluation results should be provided for these label-based monitoring proposals, which can be compared to those using label-free monitoring.

Moreover, it is possible to perform label-based monitoring where certain information on the ground truth label can be obtained by the target UE itself. For example, as a variant of monitoring approach 1 of section 11.1.1, the target UE can use a mechanism outside of the Case 1 model to detect opportunistic LoS link(s) and estimate the corresponding distance between the target UE and the TRP, which can then be used as a ground truth label approximation/substitution to monitor Case 1 model.

The more complicated label-based monitoring solutions should be introduced to NR only if the solutions provide added values over those achievable with label-free monitoring solutions.

If the more complicated label-based monitoring is to be supported, then Option A-1 and/or A-2 is sufficient.

- In option A1, the LMF derives the ground truth label. Without knowing the UE model implementation, the only ground truth label the LMF can generate is an estimated UE location label. This can be already done using the existing specification, by setting up legacy positioning procedure in parallel to the ML positioning procedure where the UE request the LMF for its location.

- Similarly, option A2 can be achieved by setting up legacy UE based positioning in parallel to AIML positioning procedure.  In section 5.3.1 we propose to re-use the existing signaling for LOS indication to support model monitoring.

Option A-3, B-1, B-2 should not be supported.

- Option A-3 requires the presence of PRU to support model monitoring. This incurs high deployment cost and signaling overhead unnecessarily.

- Option B-1 and B-2 violate the principle of self-monitoring discussed in section 6. As discussed, LMF is responsible for functionality-level performance monitoring and management, but it does not have the requisite information to perform model-level management including monitoring metric calculation.

For model performance monitoring of AI/ML positioning Case 1, support Option A-1 and A-2 for model performance monitoring metric calculation, if estimated label is considered necessary to be provided by an external entity (which is LMF for Case 1).

## Signaling to support decision making related to model monitoring

During RAN1#118, we discussed which entity (LMF, gNB, TRP, UE) should make LCM decision, particularly whether entity beside the model hosting entity should make LCM decision for the model hosting entity:

Proposal 6.2.5-1 For label-based model performance monitoring of AI/ML positioning Case 1, study the following options: - Option 1: the target UE makes the functionality/model management decision/recommendation (e.g., activate or deactivate a model, update a model, fallback to legacy positioning methods) as a result of model performance monitoring. - The target UE notifies the LMF of its functionality/model management decision/recommendation. - Option 1 is at least applicable if Option A (The target UE side performs monitoring metric calculation) is supported. - Option 2: LMF makes the functionality/model management decision (e.g., functionality/model deactivation, fallback) as a result of model performance monitoring, and notifies the target UE. - Option 2 is at least applicable if Option B (The LMF performs monitoring metric calculation) is supported. Note: From RAN1 perspective, related to model inference and model monitoring of AI/ML based positioning, LMF makes the decision on whether to activate or deactivate positioning methods, including methods using AI/ML positioning. Proposal 6.3.4-1 For label-based model performance monitoring of AI/ML positioning Case 3a, study the following options: - Option 1: the gNB makes the functionality/model management decision/recommendation (e.g., activate or deactivate a model, update a model, fallback to legacy positioning methods) as a result of model performance monitoring. - The gNB notifies the LMF of its functionality/model management decision/recommendation. - Option 1 is at least applicable if Option A (NG-RAN node performs monitoring metric calculation for its own model) is supported. - Send an LS to RAN3 asking whether existing procedures are sufficient to enable this notification. - Option 2: LMF makes the functionality/model management decision (e.g., functionality/model deactivation, fallback) as a result of model performance monitoring, and notifies the gNB. - Option 2 is at least applicable if Option B (LMF performs monitoring metric calculation for the model located at the NG-RAN node) is supported.

For model-level decision making, the model inference entity is responsible, as discussed in section 6.1. For functionality-level decision making, the LMF is responsible.

For Case 1, beyond monitoring the deployed model by UE, some additional validation or supervision may be necessary to ensure a reliable system-level performance. Specifically, LMF may need to check from time to time whether the AI/ML model in UE works well or not. For this purpose, the UE can report model quality information to LMF periodically or when triggered by LMF. The model quality information may include: model monitoring decisions by UE, quality of the model output (e.g., uncertainty and confidence of model output). This allows the LMF to supervise the UE's AI/ML positioning functionality at a high level.

Similarly, for Case 3a, the LMF need to supervise gNB’s AI/ML positioning functionality at a high level. LMF can check the quality of model output reported by gNB. In addition, the gNB can report its model monitoring decisions to LMF periodically or when triggered by LMF. Specifically, the gNB can feedback model monitoring decision to the LMF using e.g. measurement error with cause value. The signaling details are up to RAN3.

As discussed in section 6.1, for both direct and assisted AI/ML positioning, the LMF is responsible for configuring the positioning method for the target UE as in legacy. Thus, the LMF can decide to deactivate the AI/ML based positioning methods, e.g., if the LMF decides that the model is no longer appropriate for the deployment. For example, for Case 1/2a/3a, it can be understood the model inference entity (UE, gNB) performs model monitoring and makes preliminary model monitoring decision, while LMF can make the final decision such as functionality deactivation, fallback to a legacy method.

For both direct and assisted AI/ML positioning, the model inference entity is the core entity that performs continuous model monitoring itself, including calculation of model monitoring metrics.

For Case 1/3a, the target UE (Case 1) and gNB (Case 3a) makes the model management decision and recommendation as a result of model performance monitoring. The target UE and gNB sends its model management decision and recommendation to LMF, e.g., failure cause values as in the positioning framework.

For Case 1/3a, LMF make functionality-level decisions, including whether to activate or deactivate AI/ML positioning, and which AI/ML positioning method to use.

# Model inference

## Signaling to support model inference

For the model inference stage of LCM, for an AI/ML model to work properly, the model inference context needs to be consistent with model training context. The model training context need to include all the settings that noticeably affect the model performance and cannot be ignored. A model cannot be activated for inference without checking the model inference context requirement. Specifically,

- If the model inference context requirement is satisfied, then the model inference can proceed.

  - This is the ideal case where the trained model is fully compatible with the deployment environment.

- If the model inference context requirement is not satisfied, then configuration can be updated to satisfy the model inference context requirement. After that, the model inference can proceed.

  - This may happen if the model is trained for the deployment area, but some of the parameters (e.g., PRS) are not configured the same as in the model training.

- If the model inference context requirement is not satisfied, and the configuration cannot be updated to satisfy the model inference context requirement, then model inference may not proceed. A notification message (e.g., an error message) can be sent on the failure of model inference due to mismatched context between model training and model inference.

  - This may happen if the deployment area is different from the validity area of the trained model.

Under the broad outline above, there can be further nuanced procedure, if the context requirement is not satisfied for a subset of the TRPs (TRPinconsistent), but satisfied for the remaining TRPs (TRPconsistent). For example, for TRPinconsistent, the PRS beam pattern is changed between training data collection phase and model inference phase. In this case, if the model has been trained to handle a varying number of TRPs and/or a varying set of TRPs, then the model inference can still proceed by including TRPconsistent in model inference, while excluding TRPinconsistent.

The model inference context requirement includes at least the following of an AI/ML model, which are directly related to the model input of the various positioning cases:

  - validity area of the model for inference, e.g., a list of TRPs that transmits PRS (for downlink based cases) or receives SRS (for uplink based bases),

  - configuration of reference signal transmission and measurement,

    - For models using DL PRS (Case 1, 2a, 2b), this refers to DL PRS configuration for transmission (Case 1, 2a, 2b), and measurement report on PRS (Case 2b).

    - For models using UL SRS (Case 3a, 3b), this refers to UL SRS configuration for transmission (Case 3a, 3b), and measurement report on SRS (Case 3b).

More context requirements need to be checked for compatibility, e.g., range of SNR/SINR. See the discussion on metadata of training data in section 5.2.

## Consistency aspects in general

In general, the observed fingerprint of a location is the composite picture including transmitter effect, radio channel, and receiver effect. To correctly determine a UE location based on fingerprinting, the features of the composite picture need to stay (approximately) the same for different UEs (e.g., PRU and normal UE) at different times (e.g., training data collection, model inference). Thus it is necessary to ensure consistency in any transmitter behavior (e.g., PRS configuration) and receiver behavior (e.g., measurement type and granularity, at least for Case 2b/3b) that affect the observed fingerprint.

Similar to the consistency issues discussed for the training data collection phase (see section 5.2), for UE-side model, at the following need to be checked for consistency between training and inference.

    - Validity area (section 7.2.1)

    - Assistance data. This includes:

      - Explicit configuration for RS reception (section 7.2.2)

      - Associated ID for network-side additional conditions (section 7.2.3)

    - Signal measurement configuration (section 7.2.4)

### Consistency of validity area

Regarding deployment environment identifiers, the model inference validity area Ainference needs to be recorded for an AI/ML model when model training is performed. When the trained model is checked for potential activation, the model inference validity area Ainference can be checked against the deployment environment. It can be defined as:

| Model inference validity area | The network area for which the model inference is valid. |
| --- | --- |

The model inference validity area Ainference may or may not be different from the training dataset validity area Adataset mentioned in section 5.2.2.

Training dataset validity area Adataset: this describes the area that training dataset is collected. Adataset is from the perspective of training data collection entity, as explained in section 5.1.2. For example, in Figure 22, the training data includes measurements associated with 18 TRPs, TRP {0,1,…,17}. Hence the training dataset validity area Adataset = TRP {0,1,…,17} covers the entire factory floor.

Model inference validity area Ainference: this describes the coverage area that the trained model is used to perform inference for. Ainference is from the perspective of model inference entity. Ainference can be the same as Adataset, or a subset of Adataset. For example, in Figure 22, the model inference entity may use measurements associated with 9 TRPs on the left hand side for inference, Ainference= TRP {0,1,…,8}, which is a sub-area of Adataset.

Figure 22. An example where the model inference validity area is a sub-area of training dataset validity area.

It should be clarified that the model inference validity area is a property of the trained model. It shall not be confused with the legacy IE assistanceDataValidityArea-r17, which is for the validity area of the assistance data for the associated legacy method, see below for DL-TDOA and DL-AoD.

assistanceDataValidityArea This field specifies the network area for which this NR-DL-TDOA-ProvideAssistanceData is valid.

assistanceDataValidityArea This field specifies the network area for which this NR-DL-AoD-ProvideAssistanceData is valid.

Another difference is, for model inference validity area, a cell or a TRP ID may belong to two different areas, i.e., two different areas can overlap. In contrast, two different assistanceDataValidityArea cannot overlap, i.e., a cell can appear in at most one assistanceDataValidityArea (see AreaID-CellList description: “Each cell is included in only one area.”). See explanation in section 5.2.2 as well.

In terms of signaling, the validity area does not require any new IE at model inference stage. The UE simply checks the observed TRP list against the model inference validity area Ainference, which has been stored as a part of metadata for the trained model.

The model training entity records the model inference validity area as a part of metadata for the trained model.

When preparing for model inference, the model inference validity area is checked for compatibility with the deployment area.

### Consistency of RS configuration between model inference and model training

For configuration of reference signal transmission and measurement, DL PRS and UL SRS need to be addressed for the relevant positioning cases. When the AI/ML model takes DL PRS measurement as input, for configuration of PRS transmission and measurement, it is necessary to ensure that the same configuration is used in training data collection stage and model inference stage. Also, the same configuration is used for all PRUs and UEs, for example, the IDs (e.g., TRP ID, dl-PRS-ID, DL PRS Resource Set ID, a DL PRS Resources ID) are assigned the same to all PRUs and UEs, and not varying in a UE-specific manner.

Similarly, when the AI/ML model takes UL SRS measurement as input, for configuration of SRS transmission and measurement, it is necessary to ensure that the same configuration is used in training data collection stage and model inference stage.

When preparing for model inference, configurations of reference signal transmission and measurement are checked for compatibility between the trained model and the deployment environment.

In terms of specification impact, for Case 2a/3a the model readiness for inference may be signaled from the model inference node (e.g., UE/gNB) to other nodes requesting the measurements, e.g. the LMF, in order to signal that AI/ML positioning is available.

For Case 2a/3a, support UE/gNB to send an indication to the LMF when the AI/ML model is ready for reporting measurements based on model inference. It is up to RAN2/3 to decide which procedure to use to convey the information.

Ideally, signals configured for inference are configured exactly the same for the training phase. For example, PRS configuration of bandwidth, comb offset, etc., should remain unchanged between inference and training. To make sure of this, the training data could be generated for all possible signal configurations, but this would lead to a massive training data set.  Thus some misalignment may be unavoidable. For example, the SRS resource used for training the gNB based model used in Case 3a may be already assigned to another UE at the time of inference, so that a resource with a different comb offset and or cyclic shift may be chosen to be configured.  It is however important that the measured channel response at inference is consistent with the one measured during training. This means that at least bandwidth, transmitted power and spatial properties of the reference signals stay the same between inference and training.

Before model inference can start, the UE/gNB should ensure the following stay unchanged between training and inference:

- PRS:

  - Positioning frequency layer configuration (bandwidth, comb size, SCS, starting PRB, CP)

  - Muting patterns

  - DL PRS spatial properties

  - Power over the measured bandwidth

- SRS:

  - Bandwidth configuration (starting PRB, bandwidth, comb size, SCS, CP)

  - UL SRS for positioning spatial properties

  - SRS transmission power

  - UL path loss reference

### Consistency of network-side additional conditions between model inference and model training

For AI/ML based positioning with UE-side model, to support verification of NW additional conditions, associated IDs can be defined to identify a given set of NW additional conditions. This has the benefit that the IE details do not need to be explicitly signaled, if they are not needed for performing model inference, and only needed for consistency verification. This avoids exposing unnecessary network condition details and reduces signaling overhead.

Specific for Case 1, the IEs marked as “I” (i.e., implicit”) in Table 23 can be covered by associated ID.

The benefit of associated ID is that the details for the information covered by associated ID do not need to be explicitly signaled, if they are not needed for performing model inference, and only needed for consistency verification.

Adopting associated ID for the positioning use case is similar to the agreement made in RAN1#118 for beam management (AI 9.1.1):

Agreement For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

In RAN1#118, the following working assumption was confirmed under agenda item 9.1.3.3.

Agreement Confirm the following Working assumption. Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

One difference between the positioning use case and beam management use case is, for BM the model covers the beams in a given cell only, while multiple cells/TRPs are involved in the model for positioning. Also, the number of TRPs that are needed for generating model input can vary, if a model for positioning is trained to support a varying number of TRPs.

Thus for the positioning use case, the coverage of associated ID should be discussed separately. In our view, the consistency between training and inference should be checked for each TRP separately, including the NW-side conditions explicitly signalled (e.g., PRS bandwidth) and the NW-side additional conditions which are covered by associated IDs.

Regarding the associated ID, it is to be defined per TRP, i.e., for a given TRP j, the UE assumes that the NW-side additional conditions with the same associated ID are consistent within the TRP j.

- If for a given TRP j, the same associated ID is used between training and inference, then TRP j may be included in model inference by the UE.

- If for a given TRP i, different associated IDs are used between training and inference, then TRP i should not be included in model inference by the UE.

RAN1 should further discuss the range of values for associated ID (hence the number of bits to represent the associated ID).

For Case 1, to ensure the consistency of NW-side additional condition  across training and inference, support associated ID for indicating NW conditions/configurations.

For AI/ML based positioning, the consistency between training and inference is checked for each TRP separately.

For a given TRP j, the UE assumes that the NW-side additional conditions with the same associated ID are consistent within the TRP j.

### Consistency of measurements reported between model inference and model training

For the consistency between the RSs used during training and inference, similarly it could be left to network deployment choice at least for cases 1/2a/3a. Although there could be several types of UE deployed in the network and each could be using e.g. a different bandwidth during training, parameters can be controlled by network implementation to remain static during both training and inference. The UE can also ensure the measurement bandwidth is consistent with the bandwidth used for training. For case 3b and 2b, the LMF gather the measurements and the gNB/UE may not have been involved with the model training.  It is thus important that the LMF signals what measurement bandwidth to use to provide model input.

For Cases with LMF-side model (Case 2b/3b), the measurements reported for model input must be consistent with the ones used with training data.

For measurements reported to the LMF in case 2b/3b, support the LMF to indicate the PRS/SRS measurement bandwidth.

## Consistency of network side conditions for Case 1

During RAN1#118b, the signalling of network side information was extensively discussed. One contentious point was to identify what information the UE should be entitled to for the purpose of model training, and thus should be received explicitly as e.g. a parameter value, and what information is only necessary to check for consistency between inference and training phases (hence sufficient to be covered by associated ID).

During UE based positioning, legacy methods support two types of assistance data:

- DL-PRS assistance data (nr-DL-PRS-AssistanceData), which is provided to receive the PRS signals.

- Calculation assistance data (nr-PositionCalculationAssistance), which is provided to enable the positioning method computation of the position.

The latter assistance data is heavily dependent on the positioning method. Case 1 is a UE based positioning case. For legacy positioning, so far DL-TDOA and DL-AOD can be supported, respectively, in UE-based manner. As discussed in section 1 of this contribution, only time-based methods have been discussed in the SI phase, and we thus assume only assistance data from time-based methods as baseline for Case 1.  Furthermore, one should consider what is strictly necessary for case 1, which is, due to its AI/ML nature,  a fingerprinting method. This most notably changes the need the UE has in terms of calculation assistance data.

For Case 1, the SI only considered time-based fingerprinting approaches.

Based on the available assistance data candidates in UE based DL-TDOA, we have the following observation:

IEs for which knowing the value of the parameter has unclear use for fingerprinting (hence consistency indication is enough):

- TRP location (nr-TRP-LocationInfo): Pre release-19 methods for DL-TDOA compute the time of arrival based on tri-alterations for which the coordinates of the TRPs transmitting the reference signal are needed to resolve the location equation.  For fingerprinting, the location coordinates of the TRP are not needed, since this information is implicitly imbedded in the label data of the training dataset, and never used explicitly in any calculation. Thus it is enough to know whether the TRP position has changed during the inference phase and during the model training phase.

- beam direction (NR-DL-PRS-BeamInfo): similar to TRP location, as long as the beam direction is consistent between training and inference, fingerprinting can be done.

- RTD information: for fingerprinting, the exact details of the RTD information between TRP are not needed, since this information is implicitly imbedded in the channel measurement data of the training dataset, and never used explicitly in any calculation. Moreover, the training dataset can include a mixture of training data with a range of RTD values (i.e., a range of network synchronization error), so that the model is generalized and robust under RTD variation. It should be enough to signal that RTD information has not changed above a certain threshold, e.g., beyond the range that the model has been trained for.

IEs not relevant to AI-ML positioning (not needed in Case 1 AD):

- TEG information: TEG was designed for allowing combining measurements that were time aligned within a certain error margin.  The use of TEGs in the AI/ML framework is not justified.

- Integrity parameters: these were added during Rel-18 while the SI was still ongoing. These parameters were designed to assist computation of an integrity metric for the location estimate in DL-AOD and DL-TDOA.  It is not clear how these parameters could be used by the fingerprinting methods in case1.

IEs outside of DL-TDOA, only for AOD:

- TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP)

- Expected Angle Assistance information

- PRS priority list

These IEs are only delivered for angle-based methods. As discussed earlier, all evaluations were done based on timing measurements, not angle measurements. Thus DL-AOD should not be taken as a baseline method for case 1, and the related IEs are not justified for Case 1.

For case 1, the following parameter only need to be known to be consistent:

- Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB

- Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs)

- Fine Timing relative to the serving (reference) TRP of candidate NR TRPs

For case 1, the following parameter are not required for time-based finger printing:

- The association information of DL-PRS resources with TRP Tx TEG ID

- Data facilitating the integrity results determination of the calculated location

- TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP)

- Expected Angle Assistance information

- PRS priority list

Based on the observations above, the following table is proposed for AD in case 1. A copy is attached as an excel document.

The legend for the rightmost column is:

E: include in the AD, using an existing IE

I: only indicate changes between training and inference within the consistency mechanism, e.g. associated ID.

O: out of scope for Case 1 AD, not included.

Table 23. Categorization of existing assistance data IEs for Case 1

|  | Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TdoA [1] or UE-based DL-AoD [2], as applicable | Name of IE in existing 37.355 specification | Signalled as Case 1 AD? |
| --- | --- | --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement | part of NR-DL-PRS-AssistanceDataPerTRP | E |
| 2 | Timing relative to the serving (reference) TRP of candidate NR TRPs | nr-DL-PRS-SFN0-Offset | E |
| 3 | DL-PRS configuration of candidate NR TRPs | NR-DL-PRS-Info | E |
| 4 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation | NR-DL-PRS-AggregationInfo-r18 | E |
| 5 | SSB information of the TRPs (the time/frequency occupancy of SSBs) | nr-SSB-Config-r16 (part of TRP AD) | E |
| 6 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB | NR-DL-PRS-BeamInfo-r16 | I |
| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) | nr-TRP-LocationInfo | I |
| 8 | Fine Timing relative to the serving (reference) TRP of candidate NR TRPs | NR-RTD-Info-r16 | I |
| 9 | PRS-only TP indication | prs-OnlyTP-r16 | E |
| 10 | The association information of DL-PRS resources with TRP Tx TEG ID | NR-DL-PRS-TRP-TEG-Info-r17 | I |
| 11 | LOS/NLOS indicators | LOS-NLOS-Indicator-r17 | E |
| 12 | On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation | nr-On-Demand-DL-PRS-Configurations-r17 | E |
| 13 | Validity Area of the Assistance Data | assistanceDataValidityArea-r17 | E |
| 14 | PRU measurements together with the location information of the PRU | NR-PRU-DL-Info | E |
| 15 | Data facilitating the integrity results determination of the calculated location | within NR-PositionCalculationAssistance nr-IntegrityServiceParameters nr-IntegrityServiceAlert nr-IntegrityRiskParameters nr-IntegrityParametersTRP-LocationInfo nr-IntegrityParametersDL-PRS-BeamInfo nr-IntegrityParametersRTD-Info nr-IntegrityParametersTRP-BeamAntennaInfo | O |
| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) | NR-TRP-BeamAntennaInfo-r17 | O |
| 17 | Expected Angle Assistance information | nr-DL-PRS-ExpectedAoD-or-AoA | O |
| 18 | PRS priority list | dl-PRS-ResourcePrioritySubset-r17 | O |

For Case 1, the IEs categorized as “E” (i.e., explicit) are provided as assistance data from LMF to UE.

For Case 1, associated ID is introduced to indicate whether the IEs categorized as “I” (i.e., implicit) are consistent between training and inference.

For Case 1, the IEs categorized as “O” are not provided from LMF to UE, neither explicitly nor implicitly via associated ID.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The template for specifying Case 1 should be decided by RAN WG, choosing between existing DL-TDOA, DL-AoD or a new procedure as the baseline procedure.

Observation 2	For Case 1/2a (i.e., UE-side model), channel measurement generation process is collocated in the same node with the AI/ML models. The exact details of the model inputs are implementation issues at least for model inference.

Observation 3	For Case 3b (1st priority) and Case 2b (2nd priority), due to the LMF sided model, there is a need to clarify the target model input formats to be supported for the AI/ML positioning models.

Observation 4	Option B and option C may not result in additional information available for reporting.

Observation 5	The positioning accuracy of the AI/ML model is not sensitive to channel estimation method when the timing of the first detected path is used to determine the start of Nt samples.

Observation 6	Setting d = 0, i.e., starting the Nt samples at the timing of the first detected path, provides the same or better accuracy compared to d = 3 , i.e., starting the Nt samples 3 samples before the timing of the first detected path.

Observation 7	The size Nt can be <=1/2 the measurement range Mt without significantly degrading the positioning accuracy.

Observation 8	Setting Nt=Nt' leads to clear performance degradation when the number of active TRPs is small (e.g., 4 TRPs), while performance degradation is minor when the number of active TRPs is adequate (e.g., 6 or 9 TRPs).

Observation 9	Unlike path-based measurement, sample-based measurement is robust to channel estimation algorithm, even though the start of Nt consecutive samples is determined using the first detected path.

Observation 10	Training/test timing error distributional mismatch has the same impact on both direct sample-based reporting and Rel-17 path-based reporting.

Observation 11	When compared to the total-power PDP input type, the 2-port PDP input type (1) doubles the signal sizes; (2) requires higher computational complexity; and (3) achieves marginal performance improvements.

Observation 12	When compared to the total-power PDP input type, the 1-port PDP input type (1) discards signal power and radio channel information that is readily available, and (2) achieves lower positioning accuracy.

Observation 13	While both PDP sub-samples from regular sampling grid points and legacy path-based reports are both sub-samples of the TD PDP samples, there are statistical differences between them that may impact AI/ML model performance. Most notably, the former is obtained via straightforward computation, but the latter depend heavily on the UE’s channel estimation and interpolation capabilities. The Rel-17 path-based reporting impose substantially higher computation complexity loads on the measurement nodes than the sample-based reporting approach.

Observation 14	There has been no systematic study on the use of (off-grid) RSRPP reports as AI/ML model inputs in 3GPP. Without correct modeling of UE channel estimation and interpolation capabilities, the performance of (off-grid) RSRPP reports as AI/ML model inputs cn be exaggerated.

Observation 15	If the UE receiver implementation issues are not studied, there will be model input distributional mismatch between training and inference phases, when different UEs with different receiver implementation are used in training vs inference.

Observation 16	Rel-17 path-based reporting requires much higher computational complexity for the measurement node than the sample-based reporting approach, particularly when more capable and accurate channel estimation algorithms are used.

Observation 17	Positioning accuracy of ML models using Rel-17 path-based reports, which are trained and tested with data generated by a specific measurement implementation, depends significantly on the signal processing capabilities of the measurement nodes.

Observation 18	Models trained using Rel-17 path-based reports prepared by one channel estimator do not generalize to reports from different channel estimators.

Observation 19	Given the large dimensions and ranges of possible channel estimation algorithm design tradeoffs and algorithm behaviors, fingerprinting type ML models do not generalize to measurement reports generated from different channel estimation implementations.

Observation 20	Models using path-based PDP and path-based CIR as model input are sensitive to channel estimator mismatch between training and inference phases, with higher sensitivity observed for path-based CIR.

Observation 21	On the basis of a same large RS bandwidth (e.g., 100 MHz), models using Rel-17 path-based measurements on average achieve similar positioning accuracy as models using sample-based measurements.

Observation 22	On the basis of a same small RS bandwidth (e.g., 25 MHz), ML models using Rel-17 path-based reporting on average have 26% higher 90%tile 2D positioning error than ML models using sample-based reporting.

Observation 23	Given the same receiver computational complexity, a measurement node can support a wider RS bandwidth for sample-based reporting than for Rel-17 path-based reporting. The wider RS bandwidth measurement reports enable better positioning accuracy for the ML models using sample-based reporting.

Observation 24	Sample-based reporting can achieve significantly better positioning accuracy compared to path-based reporting when fewer number of TRPs are used on smaller RS bandwidth (i.e., BW=50, 25 and 12.5 MHz).

Observation 25	Sample-based measurement allows a substantially smaller PRS bandwidth to be configured to achieve the same positioning accuracy as path-based measurement. The PRS resource allocation savings can be 50% or more, especially for smaller number of active TRP (e.g., 3-12 TRPs).

Observation 26	Sample-based measurement allows a substantially smaller number of reported TRPs to achieve the same positioning accuracy as path-based measurement. The reported TRP overhead savings can be 50% or more, especially for smaller BWs.

Observation 27	Sample-based signaling can achieve better positioning accuracy and lower signaling sizes than legacy path-based signaling using substantially simpler measurement node processing.

Observation 28	Rel-18 CPP measurements (DL RSCPD, DL RSCP, UL RSCP) assumes LoS environment, which is contradictory with the target use case of Rel-19 AI/ML based positioning.

Observation 29	Single phase value for first path/sample (e.g., DL RSCPD, DL RSCP, UL RSCP) brings no observable performance benefits to PDP, but incurs substantially high deployment cost and signalling overhead.

Observation 30	The initial phase values of measured CIR samples may be random and contains no useful spatial dependent information for an AI/ML model to learn the association between measured CIR samples and the target positions. Without addressing such spurious information during training and inference, fingerprinting ML models using the CIR inputs can produce inaccurate position estimates.

Observation 31	CIR samples containing relative phases can be used as inputs to AI/ML models to circumvent the impact of random initial phase values of measured CIR samples.

Observation 32	For the RSCPD measurement PRU assisted phase error compensation requires PRUs to be used during training and inference, further increasing the deployment cost and signaling overhead.

Observation 33	For inter-path phase measurements, the model input at training and inference must know the UE TX or RX antenna phase contribution, either by reporting the antenna phase contribution to the model, removing it, or assuming the same antenna pattern is used during inference and training.

Observation 34	It is unclear how a mobile UE would know the direction of departure / arrival and associated phase offset for each path toward a given TRP.

Observation 35	For small or moderate signaling sizes, PDP and DP samples can achieve better positioning accuracy than CIR samples at the same or smaller signaling size. Multi-port CIR samples can achieve higher positioning accuracy only with very large signaling requirements.

Observation 36	For small or moderate signaling sizes, PDP and DP samples can achieve better positioning accuracy than CIR samples at the same or smaller signaling size as well as with substantially lower AI/ML model complexity.

Observation 37	For most given 90%tile 2D UE positioning error requirements, the DP samples requires the smallest signaling sizes.

Observation 38	Multi-port complex-valued CIR samples for both Case 3b (1st priority) and Case 2b (2nd priority) require very large signaling sizes, which can cause significantly negative impacts on the radio and core networks.

Observation 39	With small to moderate number of samples available to an ML model, the total-power PDP input type is a very helpful induction bias to impose on the ML model based on human domain knowledge that the sample powers contain more important information about the UE positions. It’s only with very large number of samples that the model can start to tease out how to use the additional information in the sample phases on its own.

Observation 40	Dimension reduction techniques can be used to reduce the signal sizes of multi-port CIR samples. But the achievable positioning accuracy is also compromised. The overall accuracy vs overhead tradeoff situation of CIR samples is not improved by the two considered dimension reduction techniques.

Observation 41	For Case 3a, the location of the AI/ML model (e.g., gNB-DU vs gNB-CU) impacts the need to specify new measurements for model input.

Observation 42	The LOS/NLOS indicator in legacy positioning report is a measure of the reliability of the measurement report.

Observation 43	AI/ML assisted positioning can provide measurement report of the NLOS channel with high confidence.

Observation 44	The interpretation of a LOS indicator as reflecting the physical nature of the channel should be  the same in release 19 and legacy.

Observation 45	If the LMF is not involved in the model monitoring of the model generating the LOS indicator, the need for identifying a LOS indicator as generated by a model output is not clear.

Observation 46	Current specification only support LOS indication as additional information to another measurement.

Observation 47	For Case 2a/3a first physical path RSRPP is not an indication of quality/reliability for AI/ML generated measurement, since the model may be able to obtain accurate timing information even if the direct physical path is heavily obstructed.

Observation 48	From the RAN1 perspective, current specification support for Part A data collection in Case 1 is sufficient.

Observation 49	From the RAN1 perspective, Part A data collection in Case 2b and 3b is identical to measurement reporting for inference.

Observation 50	From the RAN1 perspective, Part A data collection in Case 3a is either up to the network implementation or identical to measurement reporting for inference.

Observation 51	Collecting sufficient data in terms of training dataset size, UE distribution, diversity of UE sources, etc, is up to implementation.

Observation 52	Label-free model performance monitoring is up to implementation of model inference entity. The only potential specification impact is to notify other entities of the model monitoring decision, if necessary.

Observation 53	If certain information on the ground truth label can be generated by the model inference entity, then such label-based model performance monitoring is also self-contained and up to implementation of model inference entity. The only potential specification impact is to notify other entities of the model monitoring decision, if necessary.

Observation 54	The benefit of associated ID is that the details for the information covered by associated ID do not need to be explicitly signaled, if they are not needed for performing model inference, and only needed for consistency verification.

Observation 55	For Cases with LMF-side model (Case 2b/3b), the measurements reported for model input must be consistent with the ones used with training data.

Observation 56	For Case 1, the SI only considered time-based fingerprinting approaches.

Observation 57	For case 1, the following parameter only need to be known to be consistent:

Observation 58	For case 1, the following parameter are not required for time-based finger printing:

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN1 assumes Case 1 is supported using UE based DL-TDOA procedures as a foundation. It is up to RAN2 to decide whether a set of new procedure is to be defined for Case 1, or extension to DL-TDOA suffices. RAN1 confirms the understanding with RAN2.

Proposal 2	RAN1 assumes Case 3a is supported using UL-TDOA or multi RTT  procedure as a foundation. It is up to RAN3 to decide whether a set of new procedure is to be defined for Case 3a, or extension to existing procedure suffices. RAN1 confirms the understanding with RAN2 and RAN3.

Proposal 3	RAN1 assumes Case 3b with sample-based reporting should be supported using a separate procedure from legacy UL RTOA and gNB RxTxTimeDiff reporting for measurement reporting for model input. RAN1 confirm the understanding with RAN2 and RAN3.

Proposal 4	For Rel-19 AI/ML based positioning Case 3b/2b, the measurement report of (PDP, DP) support sample-based measurement.

Proposal 5	For Rel-19 AI/ML based positioning Case 3b, LMF includes an indicator in the measurement request to gNB, where the indicator is on the LMF preferred measurement report: either (a) sample measurement, or (b) legacy path-based measurement.

Proposal 6	For the definition of sample-based measurement, the timing grid for the Nt consecutive samples is determined by the reference time Tref (seconds), together with the timing granularity T.

Proposal 7	The starting time of the Nt consecutive samples is the timing of the first detected path, quantized to the timing grid (option A in Proposal 2.1.1.4-1 from RAN1#118bis).

Proposal 8	For Case 3b (and 3a if needed), support {Nt, Nt'} parameter combinations: {9, 9}; {16, 16}, {16, 9}; {32, 16}, {32, 9}; {64, 32}, {64, 16}, {64, 9}.

Proposal 9	For Case 3b (and 3a if needed), a small number of k values, e.g., k={4}, k={3,4}, or k={2,3,4,5}, is supported for timing reporting granularity factor in Rel-19.

Proposal 10	For the model input types for Case 3b and 3a (if applicable), consider input based on DP or PDP samples containing sample powers summed over all receive antenna ports, i.e., total-power PDP.

Proposal 11	When studying the feasibility of (off-grid) RSRPP reports as AI/ML model inputs, the investigation should be based on at least realistic modeling of UE’s channel estimation capabilities. The reported per-path delays should be from an actual path delay estimator operating at the correct SNR.

Proposal 12	Do not support phase information for determining model input, including CIR and single phase value for first path/sample.

Proposal 13	Before CIR can be adopted as model input, RAN1 need to investigate whether fingerprinting ML models can handle CIR phase measurements, which vary not only with the radio channel environment but also with the transmitter/receiver circuits.

Proposal 14	To decide whether to support phase information to enable CIR as model input, RAN1 should weigh the small positioning accuracy improvement against the standardization effort, the signaling overhead, and the difficulty to align phase information between training and inference.

Proposal 15	If phase information is supported to enable CIR as model input, the phase values are reported in the format of relative phase.

Proposal 16	RAN1 to down-prioritize the signaling approach(es) and/or measurement definitions to support CIR model input types for Case 3b (1st priority) and Case 2b (2nd priority).

Proposal 17	Update the agreement for Case 3b channel measurement reference time by deleting the FFS.

Proposal 18	For AI/ML based positioning Case 3a, the reference time for uplink channel measurements is the same as that of Case 3b, i.e., the timing information of DP and PDP is represented relative to the UL RTOA reference time T0+tSRS as defined in TS 38.215.

Proposal 19	For Case 2b/3b, the measurement reported for input to the AI/ML model inference should include at least the following side information:

	Source of the measurement (TRP ID, PRS resource and resource set ID, PFL ID in the downlink, UL SRS resource and resource set ID in the uplink)

	Time stamp of the measurement

	Measurement quality indicator for the power information (for PDP) and timing information (for PDP, DP), for the reported path/samples.

Proposal 20	For Case 2b/3b measurements reported for input to the LMF-side AI/ML model, do not include TEG information.

Proposal 21	Send an LS to RAN3 to request feedback on whether the input to the AI/ML model for case 3a need to be specified.

Proposal 22	Support reporting (optionally) a LOS/NLOS indicator when timing information is reported for Rel-19 AI/ML positioning Case 3a.

Proposal 23	For AI/ML assisted positioning Case 3a, when the LOS/NLOS indicator is reported, from RAN1 perspective, the LOS/NLOS indicator provides information on the likelihood of a physical Line-of-Sight propagation path from the source to the receiver with a value of 1 corresponding to LOS and a value of 0 corresponding to NLOS; the LOS/NLOS indicator reuse the same format as the existing IE "LOS/NLOS Information" in 38.455.

Proposal 24	From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, one LOS/NLOS indicator is optionally reported per TRP in a measurement report from gNB to LMF, as in the existing specification.

Proposal 25	For case 3a and 2a, the timing measurement for the first path is associated with a virtual line of sight path.

	If a timing measurement is reported together with the LOS/NLOS indicator, the indicator status (for physical LOS/NLOS) is decoupled from the reported timing measurement (for virtual LOS)

	Note: when the TRP-UE link is LOS, the virtual LOS coincides with the physical LOS.

	Details of the association with virtual LOS is up to RAN2/3, e.g. indication in measurement report, or configuration in measurement request.

Proposal 26	The LOS indicator reported for Case 3a does not need to be differentiated based on how it is computed (i.e. model output)

Proposal 27	Reporting of LOS as a separate measurement is not supported.

Proposal 28	When timing information is reported for Rel-19 AI/ML positioning Case 3a, a LOS/NLOS indicator can be optionally reported, the LOS/NLOS indicator reuse the same format as the existing IE "LOS/NLOS Information" in 38.455, and its definition. If a timing measurement is reported together with the LOS/NLOS indicator, the indicator status (for physical LOS/NLOS) is decoupled from the reported timing measurement (for virtual LOS). Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

Proposal 29	(for conclusion) For AI/ML assisted positioning, when the LOS/NLOS indicator is reported for Rel-19 Case 3a, do not support an indication of whether the LOS/NLOS indicator is obtained based on a model output.

Proposal 30	(for conclusion) For AI/ML assisted positioning, when the LOS/NLOS indicator is reported for Rel-19 Case 3a, do not support reporting of the LOS/NLOS indicator as a separate measurement (i.e., without other associated measurement).

Proposal 31	Reuse the existing measurement reporting in LPP and NRPPa to report timing information for the model output of AI/ML assisted positioning.

Proposal 32	For AI/ML assisted positioning at gNB (Case 3a), the model output is uplink relative time of arrival (TUL-RTOA).

Proposal 33	For AI/ML assisted positioning at gNB (Case 3a), postprocessing is applied to generate gNB RxTxTimeDiff for measurement reporting of multi-RTT method.

Proposal 34	(for conclusion) For AI/ML assisted positioning at gNB (Case 3a), the gNB is not expected to report additional paths for AI/ML based measurement report enhancements.  Note: the network can simply omit the optional field for additional paths. It is not necessary to make specification changes to the measurement report IE.

Proposal 35	For AI/ML assisted positioning at gNB (Case 3a), power is optionally reported using UL SRS RSRP (Case 3a). The gNB is not expected to report per-path power.

Proposal 36	For AI/ML assisted positioning at gNB (Case 3a) and UE (Case 2a), the measurement report contains an indicator that AI/ML is used to produce the measurements. The signaling details are up to RAN3/RAN2.

Proposal 37	For Rel-19 AI/ML based positioning, support collecting labelled and optionally un-labelled training data samples.

Proposal 38	For training data collection of AI/ML based positioning, for both Part A and Part B, time stamp is mandatorily provided by the training data generation entity.

Proposal 39	For training data collection of AI/ML based positioning, the time stamp should allow unambiguous mapping between Part A and Part B.

	For channel measurement generated by TRP/gNB, at least when measurement periodicity exceeds the SFN/slot wrap around time (10.24 sec), field “Measurement time” is included in the time stamp.

	For channel measurement and label generated by PRU or non-PRU UE, UTC Time (e.g., for utc-time-r16) is included in the time stamp.

	Exactly how to specify time stamp for Part A and Part B is up to RAN2/RAN3.

Proposal 40	From RAN1 perspective, the existing IEs and their associated granularities can be reused for time stamp of Part A and Part B. It is acceptable to have different time stamp granularity for Part A and Part B. There is no need for RAN1 to discuss how to perform the pairing using the time stamps.

Proposal 41	RAN1 does not need to discuss further signalling to enable Part A data collection.  Measurement reporting is jointly discussed for training data collection and inference.

Proposal 42	For training data collection in all cases, the quality of power measurements is provided for PDP in Part A in units of dB. The candidate resolution includes 0.1 dB and 1 dB.

Proposal 43	Confirm the existing WAs on label generation for all cases and send the agreements to RAN2.

Proposal 44	For training data collection Part B, support only ground truth labels in the format of UE location coordinates (x, y, z).

Proposal 45	From RAN1 perspective, the same label (UE location) and label quality (UE location uncertainty) are collected, irrespective of the type of model architecture (e.g., assisted or direct positioning) that may be selected by the model training entity.

Proposal 46	For training data collection, the label quality for Part B reuse the existing uncertainty IEs for indicating UE location error, e.g., horizontalUncertainty-r16 and verticalUncertainty-r16.

Proposal 47	A label provided by a LMF, PRU, or non-PRU UE can be attached with one or more timestamp, or a span of time stamps, for which the label is valid.

Proposal 48	The training dataset is attached with metadata which describes the context information on the set of collected training data samples.

Proposal 49	For training data collection of Part A, the metadata includes: Group 1 for assistance data, Group 2 for signal measurement configuration, and Group 3 for measurement environment condition and quality.

Proposal 50	RAN1 to study the details of Group 3 metadata to attach to training dataset, including:

	Range of SNR/SINR

	Range of Channel estimation error

	Range of NW synchronization error

	Range of UE/gNB RX and TX timing error

Proposal 51	The training dataset validity area is recorded as a part of metadata of the training dataset. The validity area can be defined as a list of TRPs where data is collected.

Proposal 52	Training data provided to UE/gNB is organized with the same hierarchy as for legacy assistance data, i.e. per TRP/frequency layers for the UE, and per TRP for the gNB assistance data.

Proposal 53	For Part A training data collection of Case 1/2a, the assistance data include: training dataset validity area, assistance data necessary for PRS reception, and associated ID for additional network conditions. Corresponding assistance data is sent during model inference as well.

Proposal 54	The assistance data required for PRS/SRS reception is the same across all use cases for both direct and AI/ML assisted positioning.

Proposal 55	For AI/ML based positioning, support self-monitoring by model inference entity as a baseline. Furthermore, self-contained monitoring is preferred, where no estimated label needs to be signaled from external entities. Only when necessary, introduce signaling to provide information on ground truth label to the model inference entity.

Proposal 56	The model inference entity is responsible for model-level performance monitoring and management, including calculation of model monitoring metrics. The detailed algorithm and procedure are up to implementation.

Proposal 57	For model inference at UE (Case 1, 2a) or gNB (Case 3a), the LMF can provide assistance information to support the calculation of model monitoring metrics.

Proposal 58	For AI/ML based positioning, LMF is responsible for functionality-level performance monitoring and management.

Proposal 59	For model performance monitoring of AI/ML positioning Case 1, support Option A-1 and A-2 for model performance monitoring metric calculation, if estimated label is considered necessary to be provided by an external entity (which is LMF for Case 1).

Proposal 60	For both direct and assisted AI/ML positioning, the model inference entity is the core entity that performs continuous model monitoring itself, including calculation of model monitoring metrics.

Proposal 61	For Case 1/3a, the target UE (Case 1) and gNB (Case 3a) makes the model management decision and recommendation as a result of model performance monitoring. The target UE and gNB sends its model management decision and recommendation to LMF, e.g., failure cause values as in the positioning framework.

Proposal 62	For Case 1/3a, LMF make functionality-level decisions, including whether to activate or deactivate AI/ML positioning, and which AI/ML positioning method to use.

Proposal 63	The model training entity records the model inference validity area as a part of metadata for the trained model.

Proposal 64	When preparing for model inference, the model inference validity area is checked for compatibility with the deployment area.

Proposal 65	When preparing for model inference, configurations of reference signal transmission and measurement are checked for compatibility between the trained model and the deployment environment.

Proposal 66	For Case 2a/3a, support UE/gNB to send an indication to the LMF when the AI/ML model is ready for reporting measurements based on model inference. It is up to RAN2/3 to decide which procedure to use to convey the information.

Proposal 67	For Case 1, to ensure the consistency of NW-side additional condition  across training and inference, support associated ID for indicating NW conditions/configurations.

Proposal 68	For AI/ML based positioning, the consistency between training and inference is checked for each TRP separately.

Proposal 69	For a given TRP j, the UE assumes that the NW-side additional conditions with the same associated ID are consistent within the TRP j.

Proposal 70	For measurements reported to the LMF in case 2b/3b, support the LMF to indicate the PRS/SRS measurement bandwidth.

Proposal 71	For Case 1, the IEs categorized as “E” (i.e., explicit) are provided as assistance data from LMF to UE.

Proposal 72	For Case 1, associated ID is introduced to indicate whether the IEs categorized as “I” (i.e., implicit) are consistent between training and inference.

Proposal 73	For Case 1, the IEs categorized as “O” are not provided from LMF to UE, neither explicitly nor implicitly via associated ID.

# References

3GPP TS 38.305, Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN.

3GPP TS 37.355, LTE Positioning Protocol (LPP).

3GPP TS 38.455, NR Positioning Protocol A (NRPPa).

3GPP TR 38.843 V2.0.1 (2023-12), Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18).

RP-242399, "Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface," Qualcomm, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024.

R1-2306454, “Evaluation of AI/ML for Positioning Accuracy Enhancement,” Ericsson.

# Appendix A. High accuracy uncertainty and confidence as defined in TS 23.032

6.2a	High Accuracy Uncertainty

The high accuracy uncertainty r, expressed in metres, is mapped to a number K, with the following formula:

r=C1+xK-1

with C = 0.3 and x = 0.02. With 0  K  255, a suitably useful range between 0 and 46.49129 metres is achieved for the high accuracy uncertainty, while still being able to code down to values as small as 6 millimetre. The uncertainty can then be coded on 8 bits, as the binary encoding of K.

Table 6.2a-1: Example values for the high accuracy uncertainty function

| Value of K | Value of uncertainty |
| --- | --- |
| 0 | 0 m |
| 1 | 0.006 m |
| 2 | 0.01212 m |
| - | - |
| 20 | 0.14578 m |
| - | - |
| 40 | 0.36241 m |
| - | - |
| 60 | 0.68430 m |
| - | - |
| 80 | 1.16263 m |
| - | - |
| 100 | 1.87339 m |
| - | - |
| 120 | 2.92954 m |
| - | - |
| 127 | 3.40973 m |
| - | - |
| 255 | 46.49129 m |

6.5	Confidence

The confidence by which the position of a target entity is known to be within the shape description, (expressed as a percentage) is directly mapped from the 7 bit binary number K, except for K=0 which is used to indicate 'no information', and 100 < K ≤128 which should not be used but may be interpreted as "no information" if received.

# Appendix B. Model performance monitoring approaches

In the following, we review three highly effective performance monitoring solutions, which are applicable to Case 1 and Case 3a.

Case 1 is UE-based positioning with UE-side model, direct AI/ML positioning, where an AI/ML model taking multiple measurement inputs to produce an estimate of the UE position, as illustrated in Figure 23(a).

Case 3a NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning. An AI/ML model taking multiple measurement inputs to produce multiple model outputs such as the ToAs to multiple TRPs. The ToAs can then further processed to produce an estimate of the UE position using, for example, conventional triangulation optimization. This is illustrated in Figure 23(b).

(a) direct AI/ML positioning				(b) AI/ML assisted positioning

Figure 23. Two possible implementations of Case 1 UE-based positioning with UE-side model.

In Section 11.1.1 (label-based) and 11.1.3 (label-free), we provide two effective performance monitoring methods which can be applied for both the direct positioning (e.g., Case 1) and the assisted positioning (e.g., Case 3a). In Section 11.1.2, we provide an effective label-based performance monitoring for assisted positioning (e.g., Case 3a). The discussion shows that self-contained performance monitoring is possible for both Case 1 and Case 3a, with either (a) label-free monitoring methods or (b) label-based monitoring methods with certain label related information generated by the model inference entity itself.

Specific to Case 1, the method in Section 11.1.1 is an example of target UE perform label-based monitoring without label related info from other entities; the method in Section 11.1.2 is an example of label-free monitoring. From the discussion, we can conclude the following for Case 1:

Self-contained performance monitoring solutions for Case 1 have been shown to achieve high effectiveness in AI/ML model performance assessment. These performance results are to be used as baselines to evaluate the complexity vs added benefits tradeoff for the label-based performance monitoring proposals under consideration in RAN1.

### Monitoring approach 1 (Label-based): Opportunistic LoS links

In a cluttered environment, there is often a low probability of line-of-sight for a radio link between a UE and a TRP. For example, for InF-DH environment, Table 24 shows the LoS probabilities of a radio link between TRP and UE when assuming different InF-DH clutter parameter settings. It is observed that the LoS probability of a UE-TRP link ranges from 44.9% in a mildly cluttered environment to only 0.8% in a heavily cluttered environment. For InF-DH with clutter parameter {60%, 6m, 2m}, line-of-sight links are very rare (probability=0.8%) and this is the reason why conventional positioning methods fail.

Table 24. LoS probabilities of a radio link between TRP and UE when assuming different InF-DH clutter parameter settings.

| Environment clutter parameter setting | LoS Probability |
| --- | --- |
| {40%, 2m, 2m} | 0.449 |
| {50%, 2m, 2m} | 0.352 |
| {60%, 2m, 2m} | 0.268 |
| {40%, 6m, 2m} | 0.014 |
| {50%, 6m, 2m} | 0.025 |
| {60%, 6m, 2m} | 0.008 |

On the other hand, Table 24 shows that LoS link can still be encountered from time to time, even for the most cluttered environment of InF-DH with clutter parameter {60%, 6m, 2m}. In particular, considering that there are 18 TRPs that the UE monitors, the probability of observing at least one LoS link is actually great than 10%.

In Case 1, the target UE measures the PRS from the TRPs and determines an estimate, pUE, of its own positioning using a direct positioning AI/ML model. The target UE can also

- Identify the LoS links using either (1) existing LOS/NLOS classification implementation supporting Rel-17 positioning, or (2) an AI/ML model for LOS/NLOS classification.

- Estimate the ToA, tLOS, for the identified LoS link using existing LOS/NLOS classification implementation supporting Rel-17 positioning, or (2) an AI/ML model (which can be the same or different than the model for LOS/NLOS classification).

Rel-17 specs supports the network providing the TRP locations to the UE. Using the UE position estimates and the TRP location of the LoS link, the UE can calculate the distance to the LoS TRP.

The UE can form an AI/ML model performance metric on its own direct positioning AI/ML model as

distpUE, pTRPLOS-c⋅tLOS

Figure 24. An example of UE in a cluttered location where there is only one LoS link to TRP4.

In the following, we provide the evaluation results of such opportunistic LoS link based performance monitoring solution. Figure 25 shows the cumulative distribution function (CDF) of the AI/ML model performance metric described in the above.

- The direct positioning model is trained using the data collected from the indoor factor scenario with 60% clutter density, and clutter height and width of 6 m and 2 m, respectively (i.e., InF-DH with clutter parameter {60%, 6m, 2m}).

- The LOS/NLOS classification model is an with a computational complexity of less than 1M FLOPs. The model has been tuned to avoid false LoS classification and has a LoS miss probability of 27%. This model is used here since it was found to be effective working on a single TRP-UE link, where it generates LOS/NLOS classification, and ToA estimation if the link is classified as LoS.

Two curves are shown. One curve is for "same environment", which means that the inference environment is the same as the training environment for the inference model. The inference model performs normally in "same environment", i.e., no performance degradation. The other curve is for "new environment", which means that the inference environment is different from the training environment for the inference model, though both environments belong to InF-DH with clutter parameter {60%, 6m, 2m}, i.e., different random seeds were used. The inference model performs poorly in "new environment", i.e., performance degradation has occurred, and model performance monitoring method should detect it.

It can be observed that the CDF of the LoS link ToA difference of "same environment" is much smaller (more than an order of magnitude) and easily distinguishable from the CDF of "new environment". Thus model performance monitoring decision can be made based on the easily distinguishable behavior.

Since the model inference environment is the heavily NLoS environment of InF-DH with clutter parameter {60%, 6m, 2m}, the LoS probability is low and inadequate for conventional positioning methods, as expected. However, approximately 10% of positioning requests (containing measurement samples for 18 TRP links) can be opportunistically leveraged for performing model monitoring, i.e., at least one link out of the 18 is LoS. This is adequate for the purpose of model performance monitoring.

Figure 25. Cumulative distribution function of the difference of the ToA reports generated by the inference model and the monitoring model for the links classified as LoS by the monitoring model in two different realizations of InF-DH with clutter parameter {60%, 6m, 2m}.

This evaluation result demonstrates that it is effective to monitor AI/ML model performance using opportunistic LoS links even in a heavily NLoS environment like InF-DH with clutter parameter {60%, 6m, 2m}.

In general, the key is to provide reliable LoS link identification to the model inference entity, regardless of how such information is obtained. For instance, non-AI methods (e.g., legacy methods in existing specification) can also be used to provide the LOS/NLOS classification as long as false positive (i.e., incorrectly classify a link as LoS) probability is kept low.

In summary, for Case 1, the target UE can perform label-based model performance monitoring for its own model without assistance from other entities (e.g., LMF, PRU).

While the discussion above emphasized self-contained model performance monitoring by the target UE, the same principle can be applied if another entity (e.g., LMF) is willing to assist the target UE with label related information.

In general, the LoS identification can be provided by an entity different from the model inference entity to have independent information for model monitoring. Thus, for a model deployed in one node (i.e. gNB, LMF or the UE), the LoS state of a link can be signalled from another node (i.e. another gNB, UE). For example, a UE hosting the AI/ML model and measuring the DL PRS may also be configured to transmit SRS. Hence the LoS state of a given PRS resource may be known to gNB at a given time. To assist the model monitoring of the model hosted by the UE, the LMF could relay to the UE an assistance data, e.g., which PRS resource propagated via a LoS link and at what time.

As shown below, in legacy UE-based positioning methods, LMF can already inform the UE of LoS link state in assistance data, which helps the UE to calculate the location.

TS 37.355 v18.0.0 – NR-DL-PRS-ExpectedLOS-NLOS-Assistance The IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance is used by the location server to provide the expected likelihood of a LOS propagation path from a TRP to the target device, or for all DL-PRS Resources of the TRP to the target device. -- ASN1START NR-DL-PRS-ExpectedLOS-NLOS-Assistance-r17 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-DL-PRS-ExpectedLOS-NLOS-AssistancePerFreqLayer-r17 NR-DL-PRS-ExpectedLOS-NLOS-AssistancePerFreqLayer-r17 ::= SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-ExpectedLOS-NLOS-AssistancePerTRP-r17 NR-DL-PRS-ExpectedLOS-NLOS-AssistancePerTRP-r17 ::= SEQUENCE { dl-PRS-ID-r17 INTEGER (0..255), nr-PhysCellID-r17 NR-PhysCellID-r16 OPTIONAL, -- Need ON nr-CellGlobalID-r17 NCGI-r15 OPTIONAL, -- Need ON nr-ARFCN-r17 ARFCN-ValueNR-r15 OPTIONAL, -- Need ON nr-los-nlos-indicator-r17 CHOICE { perTrp-r17 LOS-NLOS-Indicator-r17, perResource-r17 SEQUENCE (SIZE (1..nrMaxSetsPerTrpPerFreqLayer-r16)) OF NR-DL-PRS-ExpectedLOS-NLOS-AssistancePerResource-r17 }, ... } NR-DL-PRS-ExpectedLOS-NLOS-AssistancePerResource-r17 ::= SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF LOS-NLOS-Indicator-r17

Thus, similar LOS/NLOS assistance data can be signaled to assist with model monitoring. With the assistance data on LoS links, model output accuracy can be calculated for model performance monitoring of both direct and assisted AI/ML positioning.

### Monitoring approach 2 (Label-based) for assisted AI/ML positioning

For assisted AI/ML positioning (Case 3a/2a), the gNB (Case 3a) or target UE (Case 2a) has an AI/ML model taking multiple measurement inputs to produce multiple model outputs such as the ToAs to multiple TRPs. The ToAs can then further processed by the LMF to produce an estimate of the UE position using, for example, conventional triangulation optimization.

By leveraging the associated triangulation optimization, the AI/ML model can be monitored without collecting new test samples. As shown in Figure 23 (b), the positioning algorithm takes the estimated time of arrivals into conventional triangulation-based error minimization framework to search and determine the UE position. It can be expected that the minimization outcome will have smaller residual losses when the AI/ML models are operating in the correct environment and are generating correct time of arrivals than when the models are applied to an environment different than the one used to train the models.

Therefore, the residual losses from conventional triangulation-based error minimization positioning algorithms

minpUEidpUE,pTRPi-c⋅ti

can be considered as an audit on the performance of the AI/ML model generating the ToA estimates.

In Figure 26, we provide the residual losses from conventional triangulation-based error minimization positioning algorithms. The AI/ML models for supplying the ToAs are trained with a dataset for the {60%, 6m, 2m} parameter.

- When the trained model is operating in the same environment of {60%, 6m, 2m}, the residual losses shown in blue line are below 0.77 with a probability of 99%.

- When the trained model is operating in the substantially different environment of {40%, 2m, 2m} where the model performs badly, the residual losses shown in orange line are above 0.77 with a probability of 99%.

- When the trained model is operating in a moderately different environment of {40%, 6m, 2m}, where the test environment has drifted from the training set environment, but the model is still performing well, the distribution of the residual losses shown in green remain quite similar to those for the {60%, 6m, 2m} environment (blue line).

From this analysis, it can be concluded that the residual losses from the conventional positioning algorithms can be used as a reliable metric to detect model/environment mismatch. For the example shown here, one could determine a threshold for flagging model refinement/re-training, for example, threshold = 1 considering both blue and green curves. If the positioning residual losses are above this threshold, there is a high chance that the environment has drifted too far from the training environment and the model will need to be replaced or adjusted.

Figure 26: Residual losses from conventional triangulation-based error minimization positioning algorithms. The ML model is trained in the {60%, 6m, 2m} environment and tested in three environments: {60%, 6m, 2m}, {40%, 6m, 2m} and {40%, 2m, 2m}.

Based on the investigation above, the intrinsic model monitoring for AI/ML assisted approaches is a valuable model monitoring method. The complexity, latency, and power consumption are negligible since the computation is already part of the conventional triangulation method.

In literature, the important model monitoring metrics include false alarm rate (FAR), missed detection rate (MDR), and detection delay. For the simplistic case evaluated in Figure 26, they can be estimated as follows. Assume the threshold for declare the model has drifted or not is set at residual loss =1. This corresponds to CDF=99.5% for the ‘no drift’ cases ({60%, 6m, 2m} and {40%, 2m, 2m}), and CDF=3% for the ‘drift’ case ({40%, 6m, 2m}).

The model monitoring algorithm can monitor m positioning requests to decide ‘drift’ or not. If for all m positioning requests, the residual loss is less than 1m, then ‘no drift’ is declared; otherwise, ‘drift’ is declared. In this case,

- FAR = 1-0.995m, the probability that the environment has not significantly changed (stayed at blue or green curve), but ‘drift’ is declared.

- MDR = 0.03m, the probability that the environment has changed significantly (i.e., changed to orange curve), but ‘no drift’ is declared.

The detection delay is the time to observe m positioning occasions. Plugging in m=2 shows that this simple method can achieve FAR=1% and MDR=0.09%.

### Monitoring approach 3 (Label-free): ML model self-monitoring

Data augmentation is an important and in fact indispensable component of training high performance deep learning models with limited data. In addition, data augmentation may also enable the model to self-monitor without collecting new test samples nor ground truth UE positions. One model-based self-monitoring method is described below.

- Model training with data augmentation

  - Given ground truth samples xi, yi

  - Train the model with a mixture of

    - Unmodified samples xi, yi

    - Data augmentation modified samples xi', yi

- Self-model monitoring at operation

  - Given positioning request data xj

  - Obtain a first target estimate yj using unmodified data xj

  - Obtain a second target estimate yj' using data augmentation modified data xj'

  - Check the difference between the two target estimates, dj=‖yj-yj'‖.

We investigate the effectiveness of this self-monitoring approach using the smallest model (0.73 M parameters) for the centralized direct positioning approach trained with {60%, 6m, 2m} dataset samples. The CDF of the test results are summarized in Figure 27. It can be observed that:

- When the model is operating in environments similar to that it was trained for (such as the {60%, 6m, 2m} and {40%, 6m, 2m} environments), the two UE position estimates do not differ by more than 1 m, i.e., yj-yj'<1 m.

- When the model is operating in environments which it was not trained for and hence is under-performing, the two UE position estimates differ by more than 1 m with substantially high probabilities. For instance, in the {60%, 2m, 2m} and {40%, 2m, 2m} environments, the two UE position estimates differ by more than 1 m with 20% and 33% probabilities, respectively.

- While observing one UE position estimate difference of above 1 m is not reliable enough to ensure model is still operating in the intended environment, checking the dj distribution in a small window of 10s-20s model inferences can achieve monitoring accuracy >99%.

  - Observing 14 UE position estimate differences of above 1 m is enough to ensure an environment change from {60%, 6m, 2m} to {60%, 2m, 2m} is identified with a probability > 95%.

    - Observing 21 UE position estimate differences of above 1 m is enough to ensure an environment change from {60%, 6m, 2m} to {60%, 2m, 2m} is identified with a probability > 99%.

  - Observing 8 UE position estimate differences of above 1 m is enough to ensure an environment change from {60%, 6m, 2m} to {40%, 2m, 2m} is identified with a probability > 96%.

    - Observing 12 UE position estimate differences of above 1 m is enough to ensure an environment change from {60%, 6m, 2m} to {40%, 2m, 2m} is identified with a probability > 99%.

Note that the need of observing multiple instances is no different than any other model monitoring methods, since poor performance at a single instance cannot tell whether the model has truly degraded, or it’s a random anomaly.

Figure 27. 2D position estimate difference using unmodified or modified positioning request data at production in different operating environments for a small centralized direct positioning model trained with {60%, 6m, 2m} dataset samples.

The curves Figure 27 can be used to devise a monitoring method and calculate false alarm rate (FAR), missed detection rate (MDR), and detection delay as well. Take the threshold of 2D position estimate difference dthresh= 1m. At this point, CDF=99.96% for {60%, 6m, 2m} and {40%, 6m, 2m}, CDF=80% for {60%, 2m, 2m}, and CDF= 67% for {40%, 2m, 2m}. The model monitoring protocol can be:

- Declare ‘no drift’ from {60%, 6m, 2m} to {60%, 2m, 2m} if the last m positioning instances have d<dthresh.

- Declare ‘no drift’ from {60%, 6m, 2m} to {40%, 2m, 2m} if the last m position instances have d<dthresh.

- Otherwise, declare ‘drift’ and the model trained for {60%, 6m, 2m} is no longer appropriate.

In this case, take m=21,

- FAR = 1 – 0.999621 = 0.8%

- Missed detection rate of drift from {60%, 6m, 2m} to {60%, 2m, 2m}= 0.821 = 0.9%

- Missed detection rate of drift from {60%, 6m, 2m} to {40%, 2m, 2m}= 0.6721 = 0.02%

The detection delay is the time to observe m positioning occasions.

Thus, compared with the label-based model monitoring methods (e.g., see analysis in section 11.1.2), the self-model monitoring method requires longer observation time. On the other hand, it has the advantage of being self-contained and label free, and requiring no external assistance or information exchange beyond the model and its own data augmentation.