3GPP TSG RAN WG1 #119	R1- 2409444

Orlando, USA, November 18th – November 22nd, 2024

Agenda item:		9.6.1

Title:			LP-WUS and LP-SS design

Source: 			Nokia

Document for:		Discussion and Decision

# Introduction

In this document, we will discuss in detail about our view and proposals on the design of LP-WUS and LP-SS design for wake-up signal. The proposal for both LP-SS and LP-WUS are made under two underlying assumptions, namely, the complexity of the gNB transmission and the low power nature of the receiving device, i.e., LR.

# Waveform Design

## Signal bandwidth and the location within carrier BW

Agreement From RAN1 perspective, support X=11 PRBs for LP-WUS and LP-SS with SCS 60kHz and 120kHz for FR2. Agreement From RAN1 perspective, support X=11 PRBs for LP-WUS and LP-SS with SCS 15kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5 MHz

One of the parameter for LP-WUS design is the bandwidth of the signal as it will affect many aspects of the LP-WUS operation and LR design. Assuming higher bandwidth would allow higher signal power, but also result more noise power for ED based receivers, so the receiver SNR may remain unchanged, thus the typically considered receivers may not benefit from this in terms of performance. However, by increasing the bandwidth, robustness against narrowband interference and fading can be improved.

Wider LP-WUS/LP-SS BW can offer robustness against fading and frequency offset but impacts the spectrum efficiency.

On other hand Rel-17 RedCap devices are restricted to a RF/BB bandwidth of 20MHz and introduction of enhanced support of RedCap devices with further restricted BW, down to 5MHz, has been specified for Rel-18. Furthermore, in Rel-18 the below 5MHz carrier operation was also introduced.  Thus, limited LP-WUS BW would also enable wider adoption of LP-WUS. The scalability of LP-WUS/LP-SS BW would benefit multitude of device types and deployments.

Consider a scalable design for LP-WUS/LP-SS to support multiple BW options that can fit within NR deployment scenario. Thus, consider the feasibility of LP-WUS BW equal to or below 5MHz.

The position of LP-WUS/LP-SS transmissions within the carrier should be flexible, i.e., can be configurable by the gNB, which can then be informed to the MR through unicast or broadcast messages. This way, the spectral utilization can be optimized by the NW.

The location of LP-WUS/LP-SS within carrier BW should be flexible and configurable by the NW.

## Modulation Type

In the last RAN1#118bis meeting, it was agreed that if ZC sequence is used as a overlay sequence, then the need for OOK1 can be ignored as ZC sequence in time domain is the same as ZC sequence in frequency domain but with different root index.

Agreement For overlaid OFDM sequences for LP-WUS, further down-selection between following two options for OOK-1 and OOK-4 with M=1(if supported) - Option 1-1: Overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing - Option 2: Overlaid sequence(s) are the sequence(s) of an OFDM symbol before IFFT processing - Note: Different options for OOK-1 and OOK-4 with M=1 (if supported) is not precluded – in which case, it should be deemed necessary Working Assumption For overlaid OFDM sequences for LP-WUS in time or frequency domain, for OOK-1 and OOK-4 M=1 - Option 1-1(compromised): overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing for OOK-4 with M=1. - ZC sequence in time domainàDFT/LSàIFFT to result in a frequency domain sequence that is ZC sequence or a sequence that is close to ZC sequence, no additional operation - It doesn’t preclude additional operation for OOK-4 with M>1. - DFT size is 2^n Note1: OOK-1 is considered as specific case of OOK-4 with M=1. Note: Concerns were raised by Ericsson and CATT that the above is not beneficial in terms of implementation complexity

### CP Handling0

As OOK4 is agreed as a way forward to perform wave generation, the presence of CP in the final time domain waveform with modulation order M≥2 poses a problem at the LR while calculating the average transmit power used for threshold. As the glitch due to CP depends on the last few samples of the equivalent NR CP-OFDM symbol, the CP samples may or may not contain a fraction of energy. Under the assumption that Manchester encoding will be used for LP-WUS, the threshold for detection can be evaluated by half of the total energy per OOK symbol in a simple implementation to average out the effect of any interference/noise.

| (a) Performance of handling CP at transmitter in AWGN | (b) Process of CP glitch removal |
| --- | --- |

Fig. 1 Handling CP glitch before DFT process and the BLER performance.

In such implementation, the threshold evaluation may suffer the impact of data dependent CP glitch, thus leading to a wrong evaluation of threshold value used to determine the OOK symbol polarity, i.e., 0/1. On the contrary, if the CP is either zero or one always, i.e., the dependency on the LP-WUS data is not there, the offset in the threshold can be determined apriori and can be corrected as a bias term. The impact of CP will be high when M=4 compared to M=2, as the OOK symbol duration will be smaller. Furthermore, by avoiding the CP glitch, the power spent on CP insertion can be distributed to the OOK ON duration symbols, thus increasing the OOK symbol energy marginally.

Fig. 1 illustrates the way to generate the OOK sequence in time domain using DFT process for M≥2 without CP glitch by inserting zeros in the last few samples corresponding to the length of CP in the final time domain sequence. This way, the energy can be distributed to the ON pulses within each CP-OFDM symbol and thus improving the signal energy and detection capability at the LR. The number of samples that are to be zeroed out will depend on the CP duration, i.e., TCP, NIFFT,NWUS. Fig. 1b demonstrates an approach to avoid the CP glitch by inserting zeros and

Fig. 1a demonstrates the performance of with and without CP glitch removal using zero filling of trailing samples before DFT processing in OOK4 waveform generation. The performance of with and without CP removal is comparable and in certain cases, better for ED receiver but has minor impact on the IQ based receivers. The reason for slight degradation in the IQ detection performance is the length of sequence, which affects the correlation gain. On the contrary, ED detector does not show any degradation in fact proves to gain in terms of detection due to the power pooling from the CP symbols to the actual OOK symbols. Thus, the CP glitch removal by zero filling provides benefit in addition to easing out the LR threshold process.

The effect of CP glitch can be avoided by inserting zeros in the trailing samples before DFT process, wherein the length of zeroed samples depends on the SCS and NIFFT and NWUS.

For a same sequence, PAPR of OOK4 based generation is noticeably lower than OOK1 style generation due to the mapping in time domain compared to frequency domain, respectively.

## Modulation order and Pulse shaping

### Choice of M value

Agreement [H][FL1] Proposal 3.1-1: Confirm the Working Assumption that OOK-4 with M=4 for 15KHz SCS is supported for LP-WUS. Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) - FFS M=1 for OOK-4

In RAN1#118bis meeting, the modulation order M considered for further study includes M={1,2,4} as they provide better robustness against time and frequency offset to certain extent. The maximum tolerable time offset by the OOK modulation scheme for M=4 was observed as ≈4μsec, beyond which, the degradation is quite significant. As M increases, the ON duration of OOK signal reduces noticeably, which will then be comparable to that of the CP duration. Thus, the CP insertion may cause misdetection at LR as the OOK ON duration is now comparable to the CP insertion and every NR CP-OFDM symbol boundary.

The performance of M=1 is inferior compared to M=2 while using the Manchester encoding for transmission unless power pooling across symbols is allowed. This is mainly attributed to the pulse duration used for transmitting OOK signal. In M=1, the envelope detector based LR collects not only the signal energy from the entire symbol comparable to NR OFDM symbol but also the noise. On the contrary, for M=2, the symbol duration is halved, i.e., the ON duration spans only half the NR OFDM symbol with the rest carrying no signal. Thus, power pooling can boost the ON duration by a factor of two. Furthermore, the noise is also collected along with the signal only for half the duration. Thus, the SNR observed by M=2 is better than M=1 by ≈3dB for coherent receiver but falls short for a non-coherent receivers.

The SNR observed by M=1 and M≥2 are roughly the same other than the noise averaging attributed to the LR type used, namely, envelope or sequence detector.

As the modulation order M increases, the ON duration of OOK symbol length is comparable to that of CP insertion, having M>4 impairs the detectability due to reduced ON duration to provide receive side SNR improvements.

As Manchester encoding is agreed for LP-WUS, consider only M={2,4} as the possible modulation orders for both envelope and sequence detectors.

### Effect of pulse shaping

To improve the detection performance of OOK with envelope detector (ED) pulse shaping can be applied to the ON duration of the transmission. When the applied pulse shaping is accounted in the receiver, the benefits of pulse shaping can improve the receiver SNR, and band limiting of the transmitted pulse in the ON duration, i.e., by restricting the spectral leakage of LP-WUS transmission to the adjacent NR transmission.

|  | M=1 | M=2 | M=4 w/o MC | M=4 with MC |
| --- | --- | --- | --- | --- |
| Rectangular pulse | 0.7dB | 6.9dB | 12.8dB | 6.9dB |
| Gaussian pulse | 6.2dB | 12.2dB | 18.3dB | 12.2dB |
| Tukey pulse | 2.6dB | 8.6dB | 14.8dB | 8.6dB |
| Hanning pulse | 6.1dB | 12.1dB | 18.3dB | 12.1dB |

Table 1 Worst case PAPR for pulse shaping over different modulation orders (with and without MC).

Even though pulse shaping has the above-mentioned benefits, namely, reducing the spectral leakage, improving receiver SNR even with non-coherent receivers such as ED, it has the following shortcomings as well. As the pulse shaping concentrates the total energy within the CP-OFDM symbol to multiple locations depending on M and the LP-WUS bits, the impact on PAPR of the overall gNB transmission could be impacted and should be analyzed. For instance, the worst case PAPR corresponding to the above listed pulse shapes are provided in Table 1. By worst case, we mean that the information bits can be {1,0,0,0} that leads to only one ON duration a NR CP-OFDM symbol.

| (a) Envelope detection performance after matched filtering | (b) Coherent detection performance after matched filtering |
| --- | --- |

Fig. 2 Effect of matched filtering on the auto-correlation properties/structure with various pulse shapes

Use of pulse shaping benefit the LR to obtain better receive SNR even with simple detectors by enabling using receiver side filter matched with the pulse shape used for transmission.

As the modulation order increases, i.e., M, takes the value of 4, the degradation in the performance is almost ≥5dB for a change of 1μsec in the time offset, irrespective of the pulse shaping used at the transmitter for ON duration. However, for lower modulation orders, the deviation is less than ≤2dB. Thus, the effect of timing offset reduces the ON duration length from which the energy is collected to verify against the threshold to determine whether the bit is one or zero. Therefore, the modulation order should be limited to M≤4 even with pulse shaping is used at both the transmitter and the corresponding matched filter is used at the receiver.

The use of pulse shaping in the ON duration can improve the detection performance against time offset compared to the rectangular pulse, i.e., without pulse shaping counterpart.

Pulse shaping should be considered for ON duration of OOK symbols to avoid power leakage from ON to OFF symbol duration.

The effect of combining the pulse shaping together with the overlay sequence does not affect the OOK performance but hampers the correlation gain depending on the shape of the pulse used for ON duration. The benefit of using pulse shaping overlaid with the sequence on coherent receiver is highlighted in Fig. . The sequence correlation provides sharper peaks due to coherent reception, which is highlighted in Fig. b for the corresponding pulse shapes considered in Fig. a.

### Design of Overlay sequence

Using the above-mentioned types of LR implementation architectures, let us discuss different possibilities of using sequences in the ON duration to provide the same information bits to sequence only detectors but using fewer LP-WUS symbols to reduce the power consumption.

Agreement For overlaid OFDM sequence at least for LP-WUS, - For OOK-4 with M>1, support ZC sequence for overlaid sequence. - FFS details Agreement In case of overlaid OFDM sequence not carrying information: - Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol. - Note 1: multiple overlaid OFDM sequences are specified. - Note 2: gNB can configure different overlaid OFDM sequence(s) for different cells.

The IQ based LRs, which uses the overlay sequences, receives the same information as that of the ED based LRs with fewer OOK symbols. Thus, the association of sequences to the information bits determines the LR detection complexity.

#### Single overlaid sequence

Overlaying a sequence in the ON duration is illustrated in Fig. 3. In this option, the sequence is used only to improve coverage of the LP-WUS detection, i.e., the sequence used in the ON duration is the same on all ON durations of the LP-WUS transmission. Thus, the SD based receiver can correlate with the same sequence and determine the peaks in every ON duration while evaluating zeros in the OFF duration as it contains only the receiver noise. This imposes the SD based LR to be simple and can be triggered only when the ED mode experiences coverage issue. It has the following benefits, which are listed below.

As the overlay sequence is based on cell-id, sequence correlation is simple at LR, i.e., it expects a peak at every ON duration of OOK transmission.

Since the whole ON duration is used for sequence transmission, depending on the modulation order M of the OOK sequence, the correlation gain and hence the coverage can change. When M=1, the LP-WUS signal can have the best coverage for SD.

This approach is beneficial for dual mode LRs, where ED is used when the signal conditions are good and when the signal quality deteriorates, SD mode can be opted to improve the coverage.

However, on the contrary, it has some drawbacks.

Since the ON duration carries the overlay sequence, the information transmitted for OOK based receivers are usually long. Thus, using the overlay sequence only in the ON duration may turn the SD to perform correlation for the entire LP-WUS duration, that is targeted for ED. It increases the power consumption at the SD type LRs.

Fig. 3 Overlay sequence type with Nosq=4.

Due to the cell-specific nature of the overlay sequence, this option is useful for LP-SS and for LRs equipped with ED and SD operating modes. That is, predominantly ED will be used for OOK sequence detection, and upon the detection failure, i.e., when the coverage is not sufficient for OOK detection, SD receiver will be activated for sequence detection. Upon detection failure by ED, SD receiver will be activated to perform sequence detection. However, due to the robustness of ED against frequency offset, SD receiver may incur huge frequency offset while performing sequence correlation.

The sequence overlaid in the ON duration of OOK signal must be robust against frequency offset as the accuracy of LO cannot be ensured for IQ receivers.

#### Multiple sequences in ON duration carrying more bits, i.e., Nosq>1

Agreement In case of overlaid OFDM sequence carrying information, support option 2: - Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options for potential down-selection. - Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK ‘ON’ symbols. - Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s) Note: the overlaid OFDM sequence in each OOK ‘ON’ symbol can be different according to information bits to be carried by the overlaid OFDM sequence within the LP-WUS. Agreement Regarding the maximum number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, down-select from the below: - 4 - 8 - 16 - 32 - 64 for M not larger than 2 - FFS whether the same or different set of candidate overlaid sequences are used for different cells

In the second case, where multiple sequences can be allocated to carry more than one bit of information, i.e., using Nosq ovelay sequences, we can convey Nosq bits depending on which sequence is used for transmission. This is addressed in this section. The size of the sequence remains the same as that of the OOK ON duration, i.e., M. The following are the merits of this approach, namely,

The number of overlay sequences can be configured based on the number of bits carried in each OOK ON duration.

As the OOK modulation order increases, the number of samples within ON duration reduces, thus imposing difficulty in finding a better set of Nosq sequences with cross correlation property.

Since multiple bits can be conveyed in each ON duration of LP-WUS, sequence detectors may not be active for the complete LP-WUS transmission. Thus, the SD can receive fewer ON duration of LP-WUS signal to recover the complete message carried by the overall LP-WUS, thereby, reducing the power consumption of SD based receivers.

Unlike the previous scheme, this has fewer drawbacks, namely,

As the number of sequences used for correlation increases with the number of bits carried within each ON duration of LP-WUS, the HW complexity and the processing complexity increases exponentially. Additionally, the SD based receivers may require buffering if the processing capability does not meet the real-time requirements of sequence detection.

Using multiple overlay sequences in the ON duration to convey more bits may increase the complexity of sequence detectors exponentially. Thus, limiting to fewer bits would provide a better trade-off between the complexity and the power consumption.

The correlation gains and cross-correlation between sequences diminish with the increase in M used by the underlying OOK sequence, since increasing M reduces the ON duration length, i.e., sequence length of embedded sequence itself.

The number of overlay sequences per each OOK signal should consider the underlying modulation order, i.e., M, used by OOK.

##### Relative phase rotation of consecutive overlay sequences

LP-WUS with overlaid sequences over OOK ON symbols allows to map each OOK ON symbol to bit patterns containing multiple bits and thereby such a sequence-based wakeup receiver will be able to receive the entire LP-WUS wakeup message in shorter time than the OOK receiver. However, since each possible bit pattern per ON symbol needs to be mapped to a sequence, then the Hamming distance will be reduced by 2k, where k is the number of bits per OOK ON symbol, i.e., if there is just 1 bit per ON symbol, then a single sequence is enough, but if there are 2 bits per ON symbol, then 4 sequences are needed, and likewise if there are 3 bits per ON symbol, then 8 sequences are needed. So, for a given sequence length, the Hamming distance will be reduced by 2k if k bits are carried by the sequence compared to the case where just 1 bit is carried by the sequence.

The Hamming distance will be reduced by 2kif k bits are carried by the sequence compared to the case where just 1 bit is carried by the sequence.

To mitigate the performance degradation by lowering the Hamming distance, we suggest adding redundancy by making relative phase rotation between consecutive sequences, such that the relative phase rotation can be used to identify the transmitted sequence in addition to the correlation itself.

A relationship between the different sequences used in neighbouring OOK ON symbols can be achieved by rotating the phase of the time domain samples of the sequence on symbol N relative to the phase rotation of the sequence on previous symbol N-1.

The phase rotation between the sequences used in successive ON symbols, N and N+1 is dictated by the symbol transmitted in symbol N.

The Manchester encoding ensures that there is a short maximum distance between consecutive OOK ON symbols and they are therefore considered to be within the channel coherency time, meaning that the relative phase rotation between consecutive sequences is not significantly impacted by the channel.

The Manchester encoding ensures short time between consecutive sequences, meaning that the relative phase rotation is not significantly impacted be the channel.

The correlation value between the complex I/Q samples and the different sequences will give complex correlation peaks for each sequence. In a noisy environment two sequences with low Hamming distance may be observed with similar correlation peak magnitudes. In this case, the UE may perform a consistency check between the relative phase rotation and the sequences with the strongest correlation peaks and select the most likely sequence.

The UE may perform a consistency check between the relative phase rotation and the sequences with the strongest correlation peaks and select the most likely sequence.

In the case that the WUS message contains a CRC checksum field, and that the checksum fails when the WUS message is checked, the UE may re-evaluate the detected sequences provided that the correlations peaks for each sequence is stored for each OOK ON symbol.

##### Subgroup specific overlay sequence, i.e., Nosq=|G|+1

Overlaying a sequence in the ON duration is illustrated in Fig. 3. Using multiple overlay sequences in the ON duration is beneficial as it provides additional information with smaller ON time for IQ based receivers. Even though, the RF power is reduced by a factor of ≈L/⌈log2Nosq⌉, where L denotes the length of LP-WUS payload. Thus, by capturing only a fraction of LP-WUS duration, the complexity involved in the correlation of multiple sequences at the LR cannot be ignored. The base-band power consumption due to intensive computations is comparable to that of RF reception power, if not equal. Thus, increasing the number of cross correlation sequences at the LR incurs significant power consumption and increases the memory footprint ∝Nosq as well.

Using multiple overlay sequences is not a bad option from the NW perspective, it is only the complexity involved with multiple correlations at the LR is problematic. Thus, to reduce the LR complexity, NW can assign a sequence for each subgroup that can be overlaid in the ON duration. The ordering or overlaying of sequences corresponding to the paged subgroups is up to NW implementation. However, the LR correlates for a single sequence that is assigned to the subgroup it belongs to and searches for an entire LP-WUS duration.

Fig. 4 LP-WUS overlay sequence mapping, one sequence per sub-group

In Fig. 4, the group specific overlay sequence is applied on every ON duration of OOK signal, i.e., z={zi} is chosen over an ON pulse depending on the subgroup that needs to be paged. At the receiver, the LR equipped with IQ receiver, perform correlation with the corresponding group specific sequence, say, {zg}, over all LP-WUS ON duration symbols. Thus, at each ON duration, LR correlates with only one sequence and that is assigned to the group it belongs to. In addition to the subgroup specific sequence, there will be a sequence to indicate all subgroups, which is like the codepoint based scheme used in idle operation.

To minimize the LR complexity due to multiple sequence correlations, a group specific sequence should be used as overlay sequence, and thus limiting the correlation sequence to only one.

Number of sequences can be made equal to that of number subgroups, which can then be overlaid on the ON duration of LP-WUS sequence.

A subgroup specific sequence should be considered for overlaying to limit the number of correlations to unity, i.e., LR uses only one sequence for correlation.

#### Simulation performance

In this section, we compare the above schemes with the envelope and sequence detectors without any additional receiver side impairments, i.e., by assuming perfect frequency and timing synchronization. The overlay sequence used for this evaluation is based on “m-sequence” and the generation is based on OOK4 style for M values. The comparison is made between three schemes in Fig. 5, namely,

- envelope detection,

- cell specific sequence detection, i.e., LR correlates with only one sequence,

- four sequences carrying two bits of information, thus requiring only half the number of ON symbols.

The comparison is drawn between different M={1,2,4} with rectangular or Gaussian pulse for each OOK symbols. It can be seen from both Fig. 5a, and Fig. 5b that the 1% and 10% BLER requirement for LP-WUS using Gaussian pulse shape outperforms the one with rectangular pulse at least by a margin of ≈2dB on all scenarios. Furthermore, note that as we increase M from M=1→4, the overall performance degrades, with M=1 being more robust and requires lower SNR to achieve the same target as others.

| (a) Gaussian pulse in TDLC 300ns | (b) Rectangular pulse in TDL-C 300ns |
| --- | --- |

Fig. 5 SNR required to achieve 1% and 10% BLER with LP-WUS payload of size 8.

Fig. 5 also draws the comparison between different receiver architectures as listed above. In particular, Fig. 5a outlines the performance with Gaussian pulse. Having a cell specific sequence or a single sequence for correlation provides the best performance followed by the envelope detector. The performance of having four overlay sequences providing two bits of information is comparable to single sequence only for M=1 scenario. As M↑, the performance of multiple sequences degrades due to the length reduction, thus showing inferior performance for M=2 and M=4.

The shape of the pulse does not impair the detection performance of IQ based LRs, contrastingly, it improves the detection performance and provides better spectral compactness.

The cross-correlation performance degrades drastically, if the same number of sequences is used irrespective of M used by underlying OOK transmission.

#### Design of ZC overlay sequences

The choice of overlay sequence used over the ON symbol of OOK is discussed in this section. We identified a set of possible root sequences that can be used for a given length, which is governed by M value used. Together with the root sequence list with utmost of four cyclic shifts per root sequence, we can create a set of ZC overlay sequences that can coexist to provide better auto-/cross- correlation properties, listed in Table 2.

| Sequence | ZC length | Root sequence IDs (u) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | u0 | u1 | u2 | u3 | u4 | u5 |
| ZC | M=1 NZC = 131 | 3 | 18 | 27 | 89 | 102 | 123 |
|  | M=2 (NZC = 61) | 13 | 19 | 44 | 49 | 51 | 53 |
|  | M=4 (NZC = 31) | 5 | 9 | 12 | 15 | 16 | 18 |

Table 2 ZC overlay sequence root IDs.

The ZC sequence, {zm} used to verify the above correlation properties is defined as follows.

{zm}=exp-jπu×m×(m+1)NZC.

Increasing the number of ZC overlay sequences for a fixed length with different root sequences and the associated cyclic shifts, the probability of false alarm increases linearly.

The number of overlay sequences must be scaled by the value of M used for OOK transmission, since the length of overlay sequence is proportional to the value of M.

#### Impact of QAM mapping of LP-WUS overlay sequence in frequency domain

| Type |  | MQAM=16 |  | MQAM=64 |  | MQAM=256 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PAPR Δ (dB) | MSE with actual (dB) | PAPR Δ (dB) | MSE with actual (dB) | PAPR Δ (dB) | MSE with actual (dB) |
| ZC | M=1 (PAPR = 6.64dB) | +1.55 | -52 | +0.74 | -57 | +0.39 | -63.7 |
|  | M=2 (PAPR = 7.14) | +2.28 | -45 | +0.54 | -51 | -0.29 | -57 |
|  | M=4 (PAPR = 8.15dB) | +1.38 | -46 | -0.3 | -51 | -0.4 | -57 |
| M-Sequence | M=1 (PAPR = 8.52dB) | +1.28 | -44 | +1.6 | -46 | +0.69 | -48.9 |
|  | M=2 (PAPR = 8.19dB) | +0.46 | -41 | +0.49 | -44 | +0.38 | -48.5 |
|  | M=4 (PAPR = 8.43dB) | -0.65 | -41 | -0.53 | -44 | +0.08 | -48.5 |

Table 3 Effect of QAM mapping on LP-WUS waveform on PAPR/MSE.

In this section, we discuss the impact of mapping sequences to QAM constellation before performing IFFT process to facilitate gNB implementation. However, the impact of doing that mapping introduces ripples/spurious peaks in the final time domain waveform, which may affect the detection capability of LP-WUS signal at the LR. The mapping of LP-WUS waveform after DFT processing to a QAM modulation with different MQAM orders, distortion to the time domain waveforms changes noticeably.

The effect of mapping to a QAM constellation of LP-WUS frequency domain signal is listed out in Table 3, where the impact on the PAPR of the resulting time domain waveform after IFFT process with respect to the unquantized signal is highlighted. Additionally, the MSE degradation is also listed in Table 3 with different sizes of QAM constellation sizes. It can be easily verified from Table 3 that by increasing the constellation size improves the fidelity of the signal.

It is worth noting that the degradation caused by the QAM mapping is not the same for the type of sequences used for overlaying and the OOK modulation order, i.e., M. The degradation is severe for m-sequence compared to the Zadoff-Chu sequence, which has variations only in the phase content. On the other hand, as M increases, i.e., from M=1 to M=4, the MSE increases significantly, and the mapping is not efficient enough to reproduce the ideal time domain waveform for lower order QAM constellations. Thus, to ensure efficient reproduction of the intended signal with distinct ON/OFF transitions, higher order QAM constellation mapping is imperative. Fig. 6 lists the additional required SNR with respect to unquantized version after performing mapping of LP-WUS contents with QAM constellation of different modulation orders using either ZC or m-sequence as overlay sequence. As discussed earlier, increasing M>1, QAM mapping degrades the performance significantly, whereas for M=1, even QAM16 mapping seems to be fine as the degradation is insignificant. Much of the degradation is observed when M=4 as expected, since the DFT spreading with four OOK segments gets distorted to the maximum when there is a QAM constellation mapping.

Quantizing LP-WUS frequency domain signal using QAM constellation impairs the time domain signal significantly as the OOK modulation order M increases.

The effect of quantizing with a QAM constellation impairs the time domain waveform differently for different family of overlaid sequences used over ON duration. In case of m-sequence, the degradation is much severe than for ZC sequence.

| (a) 1% BLER offset for WUS payload of 8 bits vs ideal with ZC overlay sequence | (b) 1% BLER offset for WUS payload of 16 bits vs ideal with M overlay sequence |
| --- | --- |

Fig. 6 Additional SNR requirement due to mapping frequency domain LP-WUS with QAM constellations.

Avoid mapping of LP-WUS frequency domain contents to a QAM constellation as it impairs the time domain OOK reconstruction depending on the size of constellation used.

If QAM mapping is needed for EVM requirements, then QAM256 shall be considered, since the degradation is limited to ≤0.5dB.

# LP-WUS Message Mapping

In this section, we will discuss the LP-WUS channel format, fields that can be present, and contents used for transmission. Based on the agreement in RAN1#118bis from 9.6.2, the analysis consider the set of MOs monitored by the LR before making the decision.

Agreement For the offset value(s) between an LO and a reference PO/PF, consider the following options: - Definition: The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay. - Option 1: gNB configures a single offset value. - If the gap between an LO and the PO associated with the offset is no less than the wake-up delay a UE supports, the UE monitors the PO associated with the offset after receiving a wake-up indication in a LP-WUS. - Otherwise, - Option 1-1: the UE follows the legacy paging monitoring procedure. - Option 1-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - Option 2: gNB configures one or multiple offset values. - For the same PO, each offset corresponds to a LO. - This does not preclude the possibility that the same LO may correspond to different POs with different offset values. - Option 2A: A UE does not expect that the gap between the LO associated with the largest offset and the corresponding PO is less than the wake-up delay the UE supports. The UE monitors the LO associated with one offset that has a gap between the LO and the corresponding PO no less than the wake-up delay. - This implies that the gNB needs to configure at least one offset value that is no less than the largest wake-up delay supported by the UEs. - FFS exactly how to choose the offset - Option 2B: - If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports, the UE monitors the LO associated with one offset that has a gap between the LO and the PO associated with the offset no less than the wake-up delay. - FFS exactly how to choose the offset - Otherwise, - Option 2B-1: the UE follows the legacy paging monitoring procedure. - Option 2B-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - FFS exactly how to choose the offset - FFS the UE shall monitor the LO associated with additional offset(s) Note: The PO mentioned above refers to legacy PO configured for the UE.

Using the Fig. 7 as reference, let us discuss each field used in the LP-WUS channel structure and identify the use case or merits in detail. In Fig. 7, two different types of channel structure are proposed, namely, payload based and sequence-based scheme. Let us discuss each of the fields in detail.

(a) Monitoring occasions within LP-WUS (LR monitors more all MOs, all subgroups paged, if more than M subgroups are to be paged)

(b) Sequence based scheme to address different subgroups in a MO

(c) Bitmap based scheme with CRC and Channel coding

Fig. 7 LP-WUS channel structure options

## Additional Synchronization Signals, e.g., Preamble

Upon detecting the LP-SS, LR may still experience certain level of uncertainty before receiving the LP-WUS. This includes both the following.

- Oscillator drift between LP-SS and LP-WUS

- Residual timing error after receiving LP-SS, which will be in the order of {±5,±2,±1}μsec depending on the modulation order M={1,2,4} used for LP-SS, respectively.

However, if the preamble is used before LP-WUS, due to the known pattern used, the LR can synchronize with the known pattern before receiving the LP-WUS. Even if the sequence is the same as LP-SS, we know that the residual error is still in the order of ±T μsec depending on the modulation order used. Thus, one cannot determine the ideal time with zero residual error after receiving preamble. Thus, LR must employ uncertainty window to receive LP-WUS irrespective of preamble presence or not.

Even after receiving the preamble before LP-WUS, the residual error cannot be zero as is the case with LP-SS. Therefore, LP-WUS detection still requires uncertainty window to perform detection.

Agreement For RRC idle/inactive state, support the following option for at least indicating subgroup information using LP-WUS: - Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs - UE monitoring one or more MOs (up to X MOs) for the same beam within an LO is supported - Value of X is larger than 1. FFS on additional details of X.

In RAN1#118, it has been agreed that codepoint based LP-WUS notification will be used for LP-WUS. Thus the LR is required to monitor only two codewords, namely, the subgroup specific codeword and all subgroup codeword. Hence the LP-WUS can be treated as a known sequence. Unlike preamble, which is a unique code sequence, LR will monitor two specific sequences associated with the paging of the respective subgroup. Thus, the need for preamble is not justified given the overhead and the need for uncertainty window even after receiving the preamble.

As codeword is used for idle mode LP-WUS, LR can use two known sequences, namely, the specific subgroup sequence and all subgroup sequence, to perform search with uncertainty window determined by timing error, thus avoiding the need for preamble.

As LR monitors only for two sequences, like preamble, detection can be performed with an uncertainty window proportional to timing inaccuracy.

Fig. 8 demonstrates the performance of using the two known codewords, i.e., group specific and all subgroup sequences, as preamble to do the timing estimation. Due to the uncertainty of the sequence presence, LR may need to perform two step approach, namely,

- Perform correlation with two known sequences, i.e., subgroup specific sequence and all subgroup sequence to determine the peak within certain uncertainty window ±25μsec.

- Upon detecting the peak, the LP-WUS message detection using adjacent OOK symbol energy comparison is performed to determine whether the LP-WUS codeword is indeed one of the two sequences used for search.

  - Even though the sequence correlation can provide the peak, which upon validation can provide the presence or absence of the sequence, due the Manchester encoding, the probability of false detection is highly likely.

  - However, if there is a peak that validated certain threshold, by checking the sequence using Manchester encoding reduces the false alarm.

Thus, the above procedure is used to identify the timing estimation and the detection capability. It is good to note that similar steps would be needed for preamble detection as it would not be always on signal.

In Fig. 8 we present the performance of timing estimation for different M values and the codeword detection accuracy after choosing a valid peak within the search window of detection region spanning ±25μsec. Fig. 8a illustrates the timing estimation accuracy of when the subgroup specific codeword is used as preamble for synchronization. Upon detecting the timing estimate, the sequence is verified for validation, i.e., checking whether the codeword after Manchester based decoding is indeed a valid codeword as illustrated in Fig. 8b.

| (a) Timing error using CW as preamble for different LP-WUS size | (b) Codeword detection accuracy in % with timing estimate (M=2) |
| --- | --- |

Fig. 8 Using desired CW as preamble with ±25μsec uncertainty window at SINR -4.76dB.

Even though Fig. 8b demonstrates ≈80% accuracy for detecting the desired codeword at -4.76dB SINR the 90%-ile timing accuracy achieves similar performance as shown for LP-SS in Section 4.2. The codeword for each subgroup is designed based on RS encoder, thus ensuring good Hamming distance separating each codeword, i.e., if the codeword length is n and the number of information bits is k, then from the Gray coded set, we choose every i×2n-k, ∀i∈{0,..2k-1} entry as a valid codeword for mapping. The detection performance could be improved if the message is decoded further from the coded bits to the information bits using block decoder or convolutional decoder, to increase the separation between the codewords. Thus, additional synchronization signal is not required for idle mode due to the reasons presented in this section.

Using LP-WUS codeword(s) as a preamble can obtain similar time synchronisation performance as LP-SS.

Do not support preamble/additional RS for LP-WUS.

Also show in Fig. 8b, the increase in the number of subgroups increases the false alarm for a fixed codeword length. Thus, increasing the information bits for a given codeword length, i.e., the number of subgroups per LP-WUS, increases the false alarm.

As the presence/absence of preamble depends on LP-WUS itself, detection uncertainty/accuracy is the same even with codeword is used as preamble.

As the number of subgroups or information bits increase, the probability of mistaking the codeword as some other valid code is high, which increases the miss detection probability.

The number of subgroups must consider the length of LP-WUS codeword, false alarm, miss detection probability together with false wake-up probability.

## LP-WUS Information Mapping

The LP-WUS information mapping for both connected mode and idle mode.

Agreement Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs (for non-CA case), select at least one from the following - Option 1: A bitmap with each bit corresponding to [one or more] UEs - Option 3: A codepoint value corresponding to [one or more] UEs - FFS details for extension of option 1 and/or 3 when UE is configured with CA Agreement For RRC idle/inactive state, support the following option for at least indicating subgroup information using LP-WUS: - Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs - UE monitoring one or more MOs (up to X MOs) for the same beam within an LO is supported - Value of X is larger than 1. FFS on additional details of X.

### Bitmap based message

An illustration of the payload type used for LP-WUS message is presented in Fig. 7a, where the LP-WUS message is formed by concatenating the bit fields corresponding to multiple POs, where each PO i has Gi groups with a total of NGi bits, i.e., each bit dictates whether the corresponding group is paged or not with a bit value of ONE or ZERO, respectively. Thus, the LP-WUS message contains NGi×NPO bits if the LP-WUS addresses NPO POs.

#### CRC

The CRC field can optionally be included in the LP-WUS message structure. The use of CRC facilitates the LR to verify the content of LP-WUS message is error free. However, if there is an error in either CRC or the message bit, LR must wake-up the MR to monitor paging PDCCH message, since there is no guarantee on the presence of paging message. Thus, the CRC in LP-WUS message becomes critical to identify whether the message is intact or not.

However, with the agreement on Manchester encoding on the LP-WUS payload, the necessity of CRC is reduced as the data integrity can be verified to a certain extent with the presence of ones and zeros due to Manchester encoding, i.e., if there is three consecutive ones or zeros, one can assure that the LP-WUS detection is in error. Additionally, by identifying the level difference between ON and OFF signal, LR can determine the quality of reception.

Data integrity can be verified for ½ rate Manchester encoding by checking for three consecutive zeros or ones in the LP-WUS payload. Furthermore, reliability of detection can also be determined based on the energy difference between ON and OFF OOK symbols.

A shorter length CRC is preferrable for LP-WUS message, since the data is encoded with Manchester encoding that is already capable of providing data integrity check to an extent.

#### Payload

The payload is a bit string which is constructed by concatenating multiple groups, each associated with different POs or a single PO depending on the design. A simple illustration is provided Fig. 7a, where we have NPO PO associated message, where each PO may contain different number of groups, which are addressed separately. Upon concatenating the bit string from multiple POs, FEC can be performed that expands the number of bits carried by the payload from Kwus,bits≈Khdr+Kcrc+i∈1,..,NPONGi to Nwus,bits=c×Kwus,bits, where Kcrc,Khdr and c denotes the number of bits used for CRC, number of bits used for header field, and the FEC code rate, respectively. Thus, the channel coded data stream is then Manchester encoded to increase the total number of bits to 2×Nwus,bits, which will then be transmitted after using desired modulation order M that is common for all the groups in each POs. Thus, the total number of CP-OFDM symbols used by the payload-based scheme is given by 2×Nwus,bitsM. The number of bits used as payload depends on the number of POs addressed by the LP-WUS and the groups within each PO.

### Sequence based message

This is presented in Fig. 7b, where there is no requirement of CRC bits to the LP-WUS message. However, header field can optionally be present, depending on the use case it serves.

#### Sequence message construction.

In this approach, whether LP-WUS addresses multiple POs or a single PO, the number of bits per PO can be fixed, which can either be the same or different. Let us assume that each PO i consists of Gi groups, thus requiring only ⌈log2NGi⌉ bits. However, in the sequence based mapping, the number of bits associated with each PO i is allocated with NPOi bits, i.e., NPOi>⌈log2NGi⌉, thus mapping a lower number of bits to higher number of bits per PO. This mapping increases the Hamming distance between the different bit words per PO, thus increasing the robustness. In one possible option, each group Gi associated with a PO looks for certain pattern inside the PO specific region, thus the detection robustness is proportional to the length of the sequence allocated to a PO segment. This is illustrated in Fig. 7b, where PO1 contains 2 groups, and PO2 contains 2 groups and PONPO contains 3 groups.

Unlike the payload-based approach, the use of Manchester encoding, if used, can be altered within each PO segment as each PO segment is independent of one another and the same goes for the modulation order as well. This is illustrated in Fig. 7b, where PO1 uses the modulation order M=2 with Manchester encoding, whereas M=1 without Manchester encoding is considered by PO2. Thus, the sequence-based mapping provides better flexibility and facilitates the grouping of UEs depending on the signal quality and conditions.

The performance of payload-based and sequence-based LP-WUS structure depends on the length of the message content, paging probability of users, and the number of users sharing the LP-WUS.

### Simulation Analysis

In this section, we compare both bitmap and codepoint scheme for total false wake-up probability by varying the number of UEs per PO, paging probabilities per UE, and the number of bits used for LP-WUS transmission. The detailed analysis of events corresponding to false subgroup wake-up and false alarm related wake-up is discussed in Section 6.1. The total false wake-up probability incurred by the LP-WUS for bitmap and code point based scheme is presented in Fig. 10. The total false wake-up probabilities presented in the tables is calculated by considering the contribution from (a) the false detection of the subgroup when there is not subgroup paging indicated by LP-WUS, then (b) a valid detection of LP-WUS indicating the subgroup but the indication is not for the UE.

### Connected mode design

In this section, we discuss the mapping that suits for connected mode, where the user traffic arrival rate is assumed to be pk≥2% together with the number of users 16 for a given LP-WUS payload of size of 16 bits. Fig. 9 describes the bitmap schemes, namely, one with CRC and each bit maps to a UE, while the second option is bitmap based on segmentation, i.e., more than one bits is used to map a group of UEs that are paged together if there is a message for one of them in the group. Thus, the redundancy provides robustness against noise and thus no separate CRC bits used.

Fig. 9 Bitmap schemes

Fig. 10 shows the total false wake-up probabilities of various mapping scheme by considering the bitmap schemes mentioned earlier with the codepoint approach for 16 bits. The legend shows the number of subgroups/CRC bits/Bits per segment for fair comparison. Note that the CRC bits are included in the total bits of 16, thus, the LP-WUS length is same across all schemes. If either the traffic arrival probability or the number of UEs per LP-WUS increases, the false wake-up probability scales up quickly for both code point and bitmap scheme. The X axis presents both UE traffic arrival rate and LP-WUS transmission rate, i.e.,  pk= 10% implies that on an average a UE is trigged to monitor PDCCH on every 10th monitored LP-WUS MO.

Fig. 10 Comparing RRC connected mode design against UE traffic arrival probability.

Comparing the bitmap scheme with and without CRC bits, the performance of CRC based scheme outperforms when the number of CRC bits is 3 and paging probability is lower. The need for CRC bits to ensure robustness against noise is needed only when there is no LP-WUS transmission at certain monitoring occasions. As the number of UE paging probability increases, LP-WUS arrival probability increases in each monitored occasion, thus the need for CRC becomes less relevant as the assumed probability for ‘flipping’ a bit is lower than receiving a bit from noise. Hence, if LP-WUS monitoring occasions are assumed to contain valid LP-WUS almost always (<90% of times), there would not be need for CRC due to the false wakeups. Also, for the increased traffic arrival probabilities, as number of bits with CRC>0 is now less than number of UEs, the rate of false wakeups is increased as UEs are ‘grouped’ to same bit. This may be less relevant in practical scenarios where UEs would remain in Active time monitoring PDCCH (and not monitor LP-WUS), probability of the false wakeups would be reduced.  In addition, comparing the bitmap approach with CRC to bitmap (wo CRC) with segmentation or to codepoint based approach, it can be observed that the bitmap with CRC can reach lowest false alarm probably for a given traffic arrival rate.

The ‘optimum’ size of CRC is dependent on the probability of LP-WUS transmission i.e. what is the ratio between LP-WUS monitoring rate and traffic arrival rate, and number of UEs grouped to one LP-WUS.

As LP-WUS is expected to be sent more frequently in connected mode, a shorter CRC can provide adequate protection with reduced overhead.

Consider bitmap scheme with 3-bit CRC for connected mode.

# LP-SS Design

As the LR is designed to reduce the power, the LP-WUS design considers the waveform using OOK with or without an embedded sequence. To integrate the OOK signal in the OFDM framework, to minimize impact on the legacy UEs, OOK scheme follows OFDM symbol structure and generation. Due to the inferior performance of OOK scheme under low S(I)NR cell-edge cases, additional sequence is embedded in the ON duration of the OOK signal, thus ensuring coverage of OOK receivers with additional sequence detection capabilities.

Agreement For OOK-based LP-WUR, the LP-WUS design assumes the following: - As starting point, the time error is based on the 5-10ppm residual frequency error after frequency error correction without considering impact of drift. - The frequency error: up to company report. - Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance - Note: Companies can assume other values outside of 5-10ppm as long as the values are justified Agreement As a starting point, for both time error and frequency error, the overlaid OFDM sequence design of LP-WUS assumes that the residual frequency error is 0.1-5ppm for OFDM-based LP-WUR after frequency error correction without considering impact of drift. - Note: Companies can use any value within the above range with justification on the value (including impact on power consumption) Agreement For LP-SS design from RAN1 perspective, consider at least the following as the design target: - For RRM measurement performed by LP-WUR based on LP-SS, UE can satisfy measurement accuracy based on X LP-SS samples within a period which is comparable to Y=the length of I-DRX cycle that is larger or equal to 1.28s. - FFS: X - Note: Y is chosen for evaluating LP-SS design. Network overhead and network power consumption are to be considered Agreement To determine the binary sequences for LP-SS, for each M value, down-select the sequence length L from the corresponding candidate values: - M=1, L= {4,6,8} - M=2, L= {8,12,16,24} - M=4, L= {16,24,32,56} - FFS whether one or more than one L value (s) applied to each of M values M=1,2,4 - FFS the L values applied to other applicable SCS(s) - FFS the L values if other M values in addition to M=1,2,4 are supported. - FFS whether the time estimation averaged across multiple LP-SS occasions is applied - Note 1: with sequence length L, it includes Manchester coding (if supported for LP-SS) - Note 2: This doesn’t preclude any of three agreed sequence types Above applies at least for both 15kHz and 30kHz

Despite limiting the usage of power consuming receiver blocks in the LR, it may not be effective for a LR to monitor LP-WUS continuously all the time, since the energy consumption is proportional to the ON duration. Thus, for an efficient operation, LRs are operated in a duty cycled manner, which turns on at discreet instants to reduce the power consumption. The OOK only receiver, i.e., using ED receiver, reduces the overall power consumption by restricting the use of IQ branches with higher sampling rate.  Due to the removal or trim-down of major power consuming components such as PLLs, LNAs, and clock sources or synthesizers, LRs may require periodic calibration of clocks before receiving the LP-WUS signal for improved detection performance, thus, mandating a periodic low-power synchronization signal (LP-SS) to be transmitted from a stable clock source, such as gNB.

## LP-SS waveform properties

### Modulation type and order

The modulation order used for LP-WUS could range from M∈{1,2,4}, with or without Manchester encoding, which dictates that LP-SS can have the same modulation range. Unlike LP-WUS that carries only the paging message with additional fine time and frequency synchronization fields as preamble, LP-SS is a periodic signal used to obtain coarse synchronization of both time and frequency. Furthermore, the design of LP-SS is important as it is the only physical channel that LRs can use to decide the coverage and the detectability of LP-WUS signal. Thus, to ensure better coverage and detectability of LP-SS, the modulation order should be limited to M∈{2,4} with or without Manchester encoding to ensure proper threshold determination.

#### Issues of non-uniform OOK symbol power on LP-SS waveform

In this section, we briefly highlight the issue of non-uniform power distribution of OOK symbol generated via OOK4 style processing with M>1. Due to the limitation of total transmit power on the underlying NR OFDM, the power of each OOK ON symbols depend on the number of ON symbols within each NR OFDM symbols. If the number of ON symbols within each NR OFDM symbols are not the same, even though the power within each NR OFDM symbol is same for all OOK ON symbols within that NR OFDM symbol, the power of each OOK ON symbols across NR OFDM symbols may not be the same as highlighted in Fig. 11.

Fig. 11 Time domain LP-SS observation with and without uniform power distribution over OOK ON duration.

Uniform distribution of power across all ON pulses ensures reliable RSRP estimation as the signal energy is evaluated by subtracting the respective OFF duration energy.

RAN1 to consider LP-SS with uniform power distribution over all ON duration pulses, i.e., equal number of ON/OFF pulses within each NR OFDM symbol.

### LP-SS Binary Sequences

Agreement For the binary LP-SS sequence type for the ‘ON-OFF’ pattern in a LP-SS, further down-selection from the following: - Gold sequence - M sequence - Computer searched sequence - FFS: the length of LP-SS sequence Agreement The number of binary LP-SS sequences is 4.

Even though LP-WUS/LP-SS target is MSG3 coverage, thus LP-WUS/LP-SS usage in the cell-edge is limited, since the MR based paging monitoring will be activated if the UE is in the cell-edge. Under such assumptions, the number of binary sequences can even be unity. However, due to the sector edge etc., it is better to consider higher number of binary sequences based on either Gold or m-sequences. In practical (heterogenous) deployments with different sizes of cells, which do not reflect the typical hexagonal grid assumed evaluations, there can be varying number of other sites/cells that may interfere. Therefore, higher number of LP-SS sequences is preferred. Having higher number of sequences would also simplify deployment, by giving flexibility to select the sequences.

The design of sequences should satisfy the following conditions to ensure robust timing and RRM estimation accuracy in addition to the detection of the intended sequence itself.

Good auto-correlation properties, i.e., the first maximum peak should be separated from the second maximum peak by at least three OOK symbols to ensure distinctiveness from the neighbouring peaks. The Gaussian pulse shaping is considered in this evaluation.

Good cross-correlation properties, i.e., at least first P peak locations obtained from two distinct sequences should be separated by at least three OOK symbols from the auto-correlation peak, thus false alarm can be avoided.

Depending on the choice of M, the number of ON/OFF within every M consecutive OOK bits should at least be M2 to ensure reliable RSRP measurements.

|  | Sequences choices |
| --- | --- |
| L=8, M=2 | [0 1 0 1 0 1 1 0;0 1 0 1 1 0 1 0;1 0 0 1 0 1 1 0;1 0 1 0 0 1 0 1] |
| L=12, M=2 | [0 1 0 1 1 0 1 0 1 0 0 1;1 0 0 1 0 1 1 0 0 1 0 1;1 0 1 0 0 1 1 0 1 0 0 1;1 0 1 0 1 0 1 0 0 1 1 0] [0 1 0 1 0 1 1 0 1 0 0 1;1 0 0 1 1 0 0 1 1 0 0 1;1 0 0 1 1 0 1 0 0 1 1 0;1 0 1 0 0 1 1 0 0 1 0 1] |
| L=12, M=4 | [0 0 1 1 1 0 0 1 1 1 0 0;0 1 1 0 0 1 1 0 0 0 1 1;1 0 0 1 1 0 0 1 0 0 1 1;1 1 0 0 0 0 1 1 1 0 0 1] [0 1 0 1 0 1 1 0 1 0 0 1;1 0 0 1 1 0 0 1 1 0 0 1;1 0 0 1 1 0 1 0 0 0 1 1;1 1 0 0 0 0 1 1 1 0 1 0] |
| L=16, M=4 | [0 0 1 1 0 1 1 0 0 0 1 1 1 0 0 1;0 0 1 1 1 0 0 1 1 0 1 0 0 0 1 1;0 1 0 1 0 1 0 1 1 1 0 0 1 0 0 1;0 1 1 0 0 0 1 1 1 0 1 0 1 0 1 0] [0 1 0 1 0 1 0 1 1 1 0 0 1 1 0 0;0 1 1 0 0 0 1 1 1 0 1 0 1 0 0 1;0 1 1 0 0 1 1 0 0 1 0 1 0 1 0 1;0 1 1 0 1 1 0 0 1 1 0 0 0 0 1 1] [0 1 0 1 1 1 0 0 0 1 1 0 1 1 0 0;0 1 1 0 0 0 1 1 1 0 0 1 0 1 0 1;0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1;0 1 1 0 1 1 0 0 1 1 0 0 0 0 1 1] |

Table 4 LP-SS sequences for different lengths.

The above choices in Table 4 satisfy the properties discussed above to facilitate both timing estimation accuracy and the RRM measurement accuracy. The sequence choice depends on the M used for the LP-SS transmission, since four consecutive zeros is still valid if two adjacent OFDM symbols carry [1 1 0 0], [0 0 1 1] with M=4 as the number of ON OOK symbols in each NR OFDM symbol is still two to qualify uniform power. However, the same is not valid for M=2, since the only approach to satisfy M=2 is to have a Manchester encoding like construction to ensure uniform power on each OOK symbols.

RAN1 should aim at designing LP-SS of length {12,16} with M={2,4} to ensure good auto/cross correlation properties together with uniform OOK energy distribution.

### Use of overlay sequence

The use of overlay sequence in the ON duration of LP-SS is left open in the WID. The main reason for not considering an overlay sequence is that SD receivers can utilize SSBs to obtain synchronization. However, it has following constraints.

Agreement For the overlaid OFDM sequence(s) for LP-SS, consider the following options for further down-selection: - Option 1: Do not specify the overlaid OFDM sequences(s) - Option 2: Specify the overlaid OFDM sequence(s) targeting for OOK waveform generation without targeting for sync and RRM measurement for OFDM-based LP-WUR using the overlaid sequence of LP-SS. - Option 3: Specify the overlaid OFDM sequence(s) targeting for OOK waveform generation and also targeting for sync and RRM measurement for OFDM-based LP-WUR using the overlaid sequence of LP-SS. - For Option 3, it is up to RAN4 to make decision on whether/how to define the RRM measurement requirement for OFDM-based LP-WUR using the overlaid sequence of LP-SS. Agreement Support overlaid OFDM sequence(s) for LP-SS: - LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. - Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable. - Applicable at least for OOK-1 and FFS for OOK-4 - From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

The SCS used by the NR data and LP-WUS message may be different than the one used by SSB, depending on the deployment scenario. This may impose additional burden on LRs to adjust the reception strategy, i.e., one for SSBs and another for LP-WUS, which may ruin the power saving benefit of SD based LRs.

The SSS sequence within SSB are based on m-sequence, which has very low processing gain (correlation gain) when the receiver is having a frequency offset comparable to 0.5× SCS. Such an accurate XOs cannot be assumed for LRs even with SD only mode. This may increase the initial synchronization of LRs unless there is an assistance from MR to synchronize the XO.

As the ED based LRs does not care about the content of ON duration, having a sequence overlaid in the ON duration benefits both type of LR implementations, i.e., ED and SD based receivers. Furthermore, the overlay sequence is used in the ON duration of LP-WUS, therefore, there is no harm in using an overlay sequence in the ON duration of LP-SS.

As the LP-WUS carries an overlay sequence in the ON duration, it would be beneficial to follow the same approach for LP-SS as well as it may avoid LRs with SD to receive SSBs.

The pulse shaping waveform for OOK signal can be considered as an overlaid sequence for the sequence-based detectors to perform coherent detection to improve the detection performance.

The LP-SS overlay sequence should assist frequency, time, and channel measurements, if SSBs and LP-SSs are not co-located in spectrum to experience similar frequency selectivity.

## LP-SS based timing synchronization

Agreement To determine the binary sequences for LP-SS, consider the following for performance comparison: - Sync accuracy T’: the achieved sync accuracy T’ for a set of 4 sequences with the same number of occupied OFDM symbols (timing estimation error smaller than ≤T’ us for P=90 % of the time for SNR=-3dB, -6dB (low priority)) - Assume the sliding window size is at least [-25us, 25us]. - RRM measurement accuracy X: the achieved measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s - Assume X= 3.5dB shall be satisfied for LP-RSRP and LP-RSRQ, based on Y=1, 4 and Z=1.28s for comparison - Cross correlation: maximum cross-cor value/ maximum auto-cor value - FFS how to use cross-correlation for comparison - Note: refer to RAN4 RRM assumption for the inter-cell interference

The primary aim LP-SS is to assist LR in maintaining synchronization of both time and frequency. Once the initial time and frequency is obtained, e.g., based on MR assistance or via LR, fine tuning can either be performed by LRs using subsequent LP-SSs itself. Additionally, the LP-SS is also used for RRM measurements, therefore, it should be cell-specific, i.e., upon receiving the LP-SS, a LR should identify the cell it is intended to synchronize with.

| Timing estimation error in μsec |  | Averaging over LP-SS occasions X (AWGN) |  |  |  | Averaging over LP-SS occasions X (TDL-C 300ns) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1/0 Detection |  | +1/-1 Detection |  | 1/0 Detection |  |  | +1/-1 Detection |  |
| Modulation M Sequence length L |  | X = 1 | X = 2 | X = 1 | X = 2 |  | X = 1 | X = 2 | X = 1 | X = 2 |
| M = 1 | L = 4 | 1.56 | 1.17 | 1.56 | 1.17 |  | 6.8 | 5.3 | 5.34 | 4.9 |
|  | L = 6 | 1.3 | 0.98 | 1.3 | 0.98 |  | 4.95 | 3.91 | 4.69 | 3.71 |
|  | L = 8 | 1.17 | 0.91 | 1.17 | 0.85 |  | 4.95 | 3.71 | 4.82 | 3.65 |
|  | L = 12 | 0.91 | 0.72 | 0.91 | 0.72 |  | 4.95 | 3.78 | 4.95 | 3.71 |
| M = 2 | L = 8 | 0.39 | 0.26 | 0.39 | 0.26 |  | 1.69 | 1.43 | 1.69 | 1.43 |
|  | L = 12 | 0.26 | 0.26 | 0.26 | 0.26 |  | 1.69 | 1.37 | 1.69 | 1.37 |
|  | L = 16 | 0.26 | 0.26 | 0.26 | 0.26 |  | 1.69 | 1.37 | 1.69 | 1.37 |
|  | L = 24 | 0.26 | 0.21 | 0.26 | 0.21 |  | 1.69 | 1.37 | 1.69 | 1.37 |
| M = 4 | L = 16 | 0.13 | 0.13 | 0.13 | 0.13 |  | 0.91 | 0.85 | 0.91 | 0.85 |
|  | L = 24 | 0.13 | 0.13 | 0.13 | 0.13 |  | 0.91 | 0.85 | 0.91 | 0.85 |
|  | L = 32 | 0.13 | 0.13 | 0.13 | 0.13 |  | 0.91 | 0.78 | 0.91 | 0.78 |
|  | L = 56 | 0.13 | 0.13 | 0.13 | 0.13 |  | 0.91 | 0.78 | 0.91 | 0.78 |

Table 5 Timing estimation error in μsec for different L using M={1,2,4} without inter-cell interference.

Thus, to achieve initial synchronization and to utilize the cell-specific information to perform reliable RRM measurements, the LP-SS channel structure must be designed to assist the LR to acquire both time and frequency synchronization with minimal complexity. The OOK signal is robust against the frequency offset but the timing accuracy is imperative as it demands ≈1μsec timing error tolerance to receive M=4 modulation order. Additionally, the same XO will be used to drive both RF and sampling clock, thus it is crucial for LRs with ED only receivers to estimate and correct the frequency offset to a certain extent.

In this section, we discuss the timing estimation using complete LP-SS sequence with Gaussian pulse shaping for OOK ON pulses. The evaluation is performed as agreed in RAN1#118bis and the comparison is drawn between different M values with chosen sequence lengths. The length of LP-SS sequence spans an equivalent of {4,8} NR symbols to ensure fair comparison for all M values. Furthermore, the use of Manchester encoding is not followed but the total number of ON and OFF symbols are maintained to satisfy ⌊M/2⌋, i.e., thus ensuring same power levels.

| Timing estimation error in μsec |  | Averaging over LP-SS occasions X (AWGN) |  |  |  | Averaging over LP-SS occasions X (TDL-C 300ns) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1/0 Detection |  | +1/-1 Detection |  | 1/0 Detection |  |  | +1/-1 Detection |  |
| Modulation M Sequence length L |  | X = 1 | X = 2 | X = 1 | X = 2 |  | X = 1 | X = 2 | X = 1 | X = 2 |
| M = 1 | L = 4 | 2.21 | 1.63 | 2.08 | 1.5 |  | 6.9 | 5.4 | 5.73 | 5.14 |
|  | L = 6 | 1.82 | 1.3 | 1.82 | 1.24 |  | 5.73 | 4.88 | 5.47 | 4.56 |
|  | L = 8 | 1.56 | 1.11 | 1.56 | 1.11 |  | 5.34 | 4.36 | 5.34 | 4.23 |
|  | L = 12 | 1.3 | 0.98 | 1.3 | 0.91 |  | 5.08 | 3.91 | 4.95 | 3.78 |
| M = 2 | L = 8 | 0.39 | 0.33 | 0.39 | 0.33 |  | 1.82 | 1.5 | 1.82 | 1.5 |
|  | L = 12 | 0.39 | 0.26 | 0.39 | 0.26 |  | 1.69 | 1.43 | 1.69 | 1.43 |
|  | L = 16 | 0.26 | 0.26 | 0.26 | 0.26 |  | 1.69 | 1.43 | 1.69 | 1.43 |
|  | L = 24 | 0.26 | 0.26 | 0.26 | 0.26 |  | 1.69 | 1.37 | 1.69 | 1.37 |
| M = 4 | L = 16 | 0.26 | 0.2 | 0.26 | 0.2 |  | 0.91 | 0.85 | 0.91 | 0.85 |
|  | L = 24 | 0.13 | 0.13 | 0.13 | 0.13 |  | 0.91 | 0.85 | 0.91 | 0.85 |
|  | L = 32 | 0.13 | 0.13 | 0.13 | 0.13 |  | 0.91 | 0.85 | 0.91 | 0.85 |
|  | L = 56 | 0.13 | 0.13 | 0.13 | 0.13 |  | 0.91 | 0.78 | 0.91 | 0.78 |

Table 6 Timing estimation error in μsec for different L using M={1,2,4} with inter-cell PDSCH interference of 0dB.

Table 5 and Table 6 illustrates the timing estimation performance with the SNR of -3dB without inter-cell interference and with inter-cell interference of 0dB using two different approaches, which depends on how the estimation is performed.

The first option is denoted as [1/0], which is the traditional envelope detector where the energy ranges between 0 and 1, which is the maximum.

An alternative approach is also evaluated, wherein the received signal is DC shifted to have [1/-1] variations by measuring the average energy over the capture buffer, which is given by 2×LU+LLPSS, where LU is the uncertainty length and LLPSS is the length of LP-SS sequence. The reference sequence used for correlation is modified to have zero DC component as well, i.e., |r(n)|2 - E{|r(n)|2}. Note that the removal of DC from the input signal is not efficient as it may have additional bias due to energy collection from uncertainty window as well.

Now, looking at both Table 5 and Table 6, we can easily observe that the estimation accuracy is better for M={2,4}, however, the difference between them is negligible. Furthermore, the use of pulse shaping improves the detection accuracy as can be seen from both Table 5 and Table 6, which shows that the use of Gaussian pulse improves the detection accuracy by the order of ≈1μsec on all cases. However, note that the presence of Manchester encoding impairs the detection performance of rectangular pulse shaping but had a marginal impact on the Gaussian pulse shaped waveforms. In Table 5 and Table 6, we also study the effect of averaging over multiple LP-SS that are time correlated, i.e., within coherent time of the fading channel. It is worth noting that by averaging over multiple LP-SS occasions that are separated by 320ms, one can improve the estimation accuracy significantly as it filters the noise even for the shorter LP-SS sequence.

The detection accuracy can be improved by averaging the timing estimates over multiple LP-SS monitoring occasions, while ensuring the coherency of the channel fading.

Increasing the length of sequence beyond {8,16} for M={2,4}, respectively, increases the overhead linearly without improving the estimation accuracy noticeably.

RAN1 should consider LP-SS length spanning L={8,16} for M={2,4}, respectively, with an optional averaging over LP-SS occasions to provide desired timing estimation accuracy.

### Clock calibration using LP-SS

If the clock source used to wake-up LR for monitoring LP-SS is drifting or having an offset, by periodically synchronizing LP-SS, the RTC can be calibrated accurately either with or without a numerically controlled oscillator (NCO), which is a simple counter. If the clock is running at optimal rate, i.e., with 0 ppm, let us assume that the LR may count T ticks between every LP-SS detection or occurrence. However, due to XO offset, let us assume it evaluates T' ticks, then upon detection of multiple LP-SSs, LR can estimate the number of number of ticks to the actual or nominal ticks and can evaluate the ppm offset as 106×1FCLK×T'TLPSS, where TLPSS is the LP-SS period and FCLK is the clock speed. By averaging the clock count T' over multiple LP-SS, LR can estimate the ppm offset and can perform calibration.

However, due to reduce clock speed of 32KHz in RTC, accuracy may not be achieved even with subsequent LP-SS timing corrections, which are in ≈μsec resolution. To overcome this issue, a simple NCO based on 32 bits counter can be used to ensure sufficient resolution. In this case, the counter can be incremented by a constant, say, C, which will add itself at every RTC clock tick and overflows when it is time to monitor LP-SS, i.e., the elapsed time is 320ms. If there is a timing correction estimated by LP-SS, that can be added to this number, which has better resolution to adjust the constant C' to ensure sufficient accuracy.

By utilizing the timing estimates from multiple LP-SSs, having a predefined periodicity, the RTC can assumed to be calibrated to an accuracy of ≈1ppm with the help of NCO.

## RRM Measurements

In the past RAN1 meetings, the LR based measurement metrics have been discussed and following agreements have been made:

Agreement From RAN1 perspective, at least the following metrics can be supported for RRM serving cell measurement performed by OOK-based receiver based on LP-SS: - LP-RSRP - LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols. - FFS: How to determine the received power of LP-SS in OOK ON symbols - LP-RSRQ - LP-RSRQ = LP-RSRP/LP-RSSI - For the definition of LP-RSSI for determination of LP-RSRQ, further consider the following options: - Option 1: LP-RSSI is the linear average of total received power in all LP-SS OOK symbols. - Option 2: LP-RSSI is the linear average of total received power in LP-SS OOK OFF symbols. - Option 3: LP-RSSI is the linear average of total received power in LP-SS OOK ON symbols. - FFS: LP-SINR, Power ratio of OOK-ON symbol and OOK-OFF symbol Note: RAN1 will send an LS to RAN2 and RAN4 on the measurement metrics that can be supported from RAN1 perspective, to facilitate RAN2/RAN4 discussions. The exact metrics for OOK-based receiver to be used and defined in the specifications depend on the outcome of [RAN1]/RAN2/RAN4 discussions.

We discuss the open aspects related to the LP-RSRQ definition in our companion paper [R1-2408538]. In following we consider the issues related to the periodicity of the LP-SS.

### LP-SS design impact on LP-RSRP measurement performance

Traditionally, RSRP is used as the measurement metric to determine the cell coverage/range. In the LP-WUS context, it is one of the possible reliable metric to determine the RRM relaxation thresholds. To study the LP-RSRP estimation accuracy, a time correlated fading of 1.28s/2.56s is used with 1e6 samples for the LP-RSRP evaluation under TDL-C 300ns channel. In this evaluation, we tried to determine the value of X, which represents the number of measurements needed within 1.28s that are spaced at equal interval. We have determined LP-RSRP based on the signal energy received during the ON duration corrected with the (noise and interference) energy received in the OFF duration of LP-SS, i.e., ΔE=EON-EOFF.

| RSRP estimation error in dB |  | Averaging over LP-SS occasions X |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AWGN |  |  | TDL-C 300ns |  |  |
| RSRP = ΔE |  | X = 2 | X = 4 | X = 8 | X = 2 | X = 4 | X = 8 |
| M = 1 | L = 4 | 1.37 | 1.04 | 0.82 | 6.22 | 4.16 | 2.83 |
|  | L = 6 | 1.16 | 0.9 | 0.73 | 6.27 | 4.19 | 2.87 |
|  | L = 8 | 1.04 | 0.82 | 0.68 | 6.31 | 4.22 | 2.9 |
|  | L = 12 | 0.91 | 0.74 | 0.62 | 6.21 | 4.17 | 2.88 |
| M = 2 | L = 8 | 0.92 | 0.74 | 0.62 | 6.74 | 4.45 | 3.06 |
|  | L = 12 | 0.81 | 0.67 | 0.57 | 6.67 | 4.43 | 3.03 |
|  | L = 16 | 0.75 | 0.62 | 0.54 | 6.71 | 4.46 | 3.05 |
|  | L = 24 | 0.67 | 0.57 | 0.5 | 6.66 | 4.44 | 3.04 |
| M = 4 | L = 16 | 1.08 | 0.9 | 0.78 | 7.86 | 5.25 | 3.67 |
|  | L = 24 | 0.95 | 0.81 | 0.72 | 7.77 | 5.2 | 3.64 |
|  | L = 32 | 0.89 | 0.77 | 0.69 | 7.78 | 5.22 | 3.64 |
|  | L = 56 | 0.79 | 0.7 | 0.64 | 7.76 | 5.2 | 3.64 |

Table 7 LP-RSRP for LP-SS periodicity of 320ms using X = {2,4,8} sample averages without inter-cell interference.

Table 7 and Table 8 compares the RSRP estimation with different values for M={1,2,4}. The comparison also includes the impact of pulse shaping and the use of equal energy over all OOK ON symbols. The RSRP measurement is performed by averaging LP-SS measurements over {0.64s,1.28s,2.56s} durations, i.e., by averaging over {2,4,8} LP-SS occasions, respectively. Even though, the sequence is generated with balanced number of zeros and ones using m-sequence, the concentration of ones and zeros in the resulting sequence introduces power variation at every NR CP-OFDM symbol, since the power is distributed only within a symbol.

| RSRP estimation error in dB |  | Averaging over LP-SS occasions X |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AWGN |  |  | TDL-C 300ns |  |  |
| RSRP = ΔE |  | X = 2 | X = 4 | X = 8 | X = 2 | X = 4 | X = 8 |
| M = 1 | L = 4 | 1.91 | 1.43 | 1.12 | 6.33 | 4.32 | 2.96 |
|  | L = 6 | 1.58 | 1.21 | 0.97 | 6.39 | 4.28 | 2.92 |
|  | L = 8 | 1.4 | 1.1 | 0.89 | 6.33 | 4.24 | 2.93 |
|  | L = 12 | 1.21 | 0.97 | 0.8 | 6.3 | 4.26 | 2.94 |
| M = 2 | L = 8 | 1.28 | 1.04 | 0.88 | 7.1 | 4.65 | 3.22 |
|  | L = 12 | 1.13 | 0.94 | 0.8 | 6.98 | 4.65 | 3.21 |
|  | L = 16 | 1.04 | 0.87 | 0.76 | 6.92 | 4.62 | 3.19 |
|  | L = 24 | 0.94 | 0.8 | 0.71 | 6.93 | 4.63 | 3.19 |
| M = 4 | L = 16 | 1.52 | 1.27 | 1.11 | 8.25 | 5.58 | 3.92 |
|  | L = 24 | 1.37 | 1.17 | 1.04 | 8.16 | 5.52 | 3.92 |
|  | L = 32 | 1.28 | 1.11 | 1.1 | 8.11 | 5.5 | 3.9 |
|  | L = 56 | 1.14 | 1.02 | 0.94 | 8.07 | 5.47 | 3.89 |

Table 8 LP-RSRP for LP-SS periodicity of 320ms using X = {2,4,8} sample averages under SIR of 3dB with PDSCH data.

The impact of such a clustered 1’s and 0’s is depicted in Table 7 and Table 8, where the signal power variation is large due to uneven concentration of ON and OFF locations, if Manchester encoding is not used. However, if Manchester encoding like restriction is used, then the signal power remains same and as observed in Table 7 and Table 8, the RSRP estimation using both S and D technique performs the same. Finally, by averaging over multiple LP-SS occasions, the accuracy of RSRP can be improved as shown in Table 7 and Table 8. The performance of M={2,4} with averaging performs similar or even better than M=1 due to power boosting within a symbol.

Averaging over multiple LP-SS MOs improves the RSRP estimation accuracy when employed with uniform power distribution together with or without pulse shaping.

RAN1 should ensure uniform power levels on all OOK ON symbols to ensure reliable LP-RSRP estimation by the LR.

The LP-SS sequence length shall have at least {12,16} for M={2,4}, respectively, together with the averaging over ≥4 LP-SS observations to ensure reliable LP-RSRP.

Using M=1 for LP-SS enforces LP-WUS to use M=1 unless additional synchronization signal, namely, preamble, is used to provide desired accuracy in timing to receive M>1.

# Conclusion

In this document, we presented our initial thoughts on the WI scope and the topics that are relevant to be considered and discussed during the WI phase.

## List of Proposals

Proposal 1:	Consider a scalable design for LP-WUS/LP-SS to support multiple BW options that can fit within NR deployment scenario. Thus, consider the feasibility of LP-WUS BW equal to or below 5MHz.

Proposal 2:	The location of LP-WUS/LP-SS within carrier BW should be flexible and configurable by the NW.

Proposal 3:	As Manchester encoding is agreed for LP-WUS, consider only M={2,4} as the possible modulation orders for both envelope and sequence detectors.

Proposal 4:	Pulse shaping should be considered for ON duration of OOK symbols to avoid power leakage from ON to OFF symbol duration.

Proposal 5:	The number of overlay sequences per each OOK signal should consider the underlying modulation order, i.e., M, used by OOK.

Proposal 6:	A relationship between the different sequences used in neighbouring OOK ON symbols can be achieved by rotating the phase of the time domain samples of the sequence on symbol N relative to the phase rotation of the sequence on previous symbol N-1.

Proposal 7:	The phase rotation between the sequences used in successive ON symbols, N and N+1 is dictated by the symbol transmitted in symbol N.

Proposal 8:	A subgroup specific sequence should be considered for overlaying to limit the number of correlations to unity, i.e., LR uses only one sequence for correlation.

Proposal 9:	The number of overlay sequences must be scaled by the value of M used for OOK transmission, since the length of overlay sequence is proportional to the value of M.

Proposal 10:	Avoid mapping of LP-WUS frequency domain contents to a QAM constellation as it impairs the time domain OOK reconstruction depending on the size of constellation used.

Proposal 11:	If QAM mapping is needed for EVM requirements, then QAM256 shall be considered, since the degradation is limited to ≤0.5dB.

Proposal 12:	Do not support preamble/additional RS for LP-WUS.

Proposal 13:	The number of subgroups must consider the length of LP-WUS codeword, false alarm, miss detection probability together with false wake-up probability.

Proposal 14:	A shorter length CRC is preferrable for LP-WUS message, since the data is encoded with Manchester encoding that is already capable of providing data integrity check to an extent.

Proposal 15:	Consider bitmap scheme with 3-bit CRC for connected mode.

Proposal 16:	RAN1 to consider LP-SS with uniform power distribution over all ON duration pulses, i.e., equal number of ON/OFF pulses within each NR OFDM symbol.

Proposal 17:	RAN1 should aim at designing LP-SS of length {12,16} with M={2,4} to ensure good auto/cross correlation properties together with uniform OOK energy distribution.

Proposal 18:	The LP-SS overlay sequence should assist frequency, time, and channel measurements, if SSBs and LP-SSs are not co-located in spectrum to experience similar frequency selectivity.

Proposal 19:	RAN1 should consider LP-SS length spanning L={8,16} for M={2,4}, respectively, with an optional averaging over LP-SS occasions to provide desired timing estimation accuracy.

Proposal 20:	By utilizing the timing estimates from multiple LP-SSs, having a predefined periodicity, the RTC can assumed to be calibrated to an accuracy of ≈1ppm with the help of NCO.

Proposal 21:	RAN1 should ensure uniform power levels on all OOK ON symbols to ensure reliable LP-RSRP estimation by the LR.

Proposal 22:	The LP-SS sequence length shall have at least {12,16} for M={2,4}, respectively, together with the averaging over ≥4 LP-SS observations to ensure reliable LP-RSRP.

Proposal 23:	Using M=1 for LP-SS enforces LP-WUS to use M=1 unless additional synchronization signal, namely, preamble, is used to provide desired accuracy in timing to receive M>1.

## List of Observations

Observation 1:	Wider LP-WUS/LP-SS BW can offer robustness against fading and frequency offset but impacts the spectrum efficiency.

Observation 2:	The effect of CP glitch can be avoided by inserting zeros in the trailing samples before DFT process, wherein the length of zeroed samples depends on the SCS and NIFFT and NWUS.

Observation 3:	For a same sequence, PAPR of OOK4 based generation is noticeably lower than OOK1 style generation due to the mapping in time domain compared to frequency domain, respectively.

Observation 4:	The SNR observed by M=1 and M≥2 are roughly the same other than the noise averaging attributed to the LR type used, namely, envelope or sequence detector.

Observation 5:	As the modulation order M increases, the ON duration of OOK symbol length is comparable to that of CP insertion, having M>4 impairs the detectability due to reduced ON duration to provide receive side SNR improvements.

Observation 6:	Use of pulse shaping benefit the LR to obtain better receive SNR even with simple detectors by enabling using receiver side filter matched with the pulse shape used for transmission.

Observation 7:	The use of pulse shaping in the ON duration can improve the detection performance against time offset compared to the rectangular pulse, i.e., without pulse shaping counterpart.

Observation 8:	The sequence overlaid in the ON duration of OOK signal must be robust against frequency offset as the accuracy of LO cannot be ensured for IQ receivers.

Observation 9:	Using multiple overlay sequences in the ON duration to convey more bits may increase the complexity of sequence detectors exponentially. Thus, limiting to fewer bits would provide a better trade-off between the complexity and the power consumption.

Observation 10:	The correlation gains and cross-correlation between sequences diminish with the increase in M used by the underlying OOK sequence, since increasing M reduces the ON duration length, i.e., sequence length of embedded sequence itself.

Observation 11:	The Hamming distance will be reduced by 2kif k bits are carried by the sequence compared to the case where just 1 bit is carried by the sequence.

Observation 12:	The Manchester encoding ensures short time between consecutive sequences, meaning that the relative phase rotation is not significantly impacted be the channel.

Observation 13:	The UE may perform a consistency check between the relative phase rotation and the sequences with the strongest correlation peaks and select the most likely sequence.

Observation 14:	To minimize the LR complexity due to multiple sequence correlations, a group specific sequence should be used as overlay sequence, and thus limiting the correlation sequence to only one.

Observation 15:	Number of sequences can be made equal to that of number subgroups, which can then be overlaid on the ON duration of LP-WUS sequence.

Observation 16:	The shape of the pulse does not impair the detection performance of IQ based LRs, contrastingly, it improves the detection performance and provides better spectral compactness.

Observation 17:	The cross-correlation performance degrades drastically, if the same number of sequences is used irrespective of M used by underlying OOK transmission.

Observation 18:	Increasing the number of ZC overlay sequences for a fixed length with different root sequences and the associated cyclic shifts, the probability of false alarm increases linearly.

Observation 19:	Quantizing LP-WUS frequency domain signal using QAM constellation impairs the time domain signal significantly as the OOK modulation order M increases.

Observation 20:	The effect of quantizing with a QAM constellation impairs the time domain waveform differently for different family of overlaid sequences used over ON duration. In case of m-sequence, the degradation is much severe than for ZC sequence.

Observation 21:	Even after receiving the preamble before LP-WUS, the residual error cannot be zero as is the case with LP-SS. Therefore, LP-WUS detection still requires uncertainty window to perform detection.

Observation 22:	As codeword is used for idle mode LP-WUS, LR can use two known sequences, namely, the specific subgroup sequence and all subgroup sequence, to perform search with uncertainty window determined by timing error, thus avoiding the need for preamble.

Observation 23:	As LR monitors only for two sequences, like preamble, detection can be performed with an uncertainty window proportional to timing inaccuracy.

Observation 24:	Using LP-WUS codeword(s) as a preamble can obtain similar time synchronisation performance as LP-SS.

Observation 25:	As the presence/absence of preamble depends on LP-WUS itself, detection uncertainty/accuracy is the same even with codeword is used as preamble.

Observation 26:	As the number of subgroups or information bits increase, the probability of mistaking the codeword as some other valid code is high, which increases the miss detection probability.

Observation 27:	Data integrity can be verified for ½ rate Manchester encoding by checking for three consecutive zeros or ones in the LP-WUS payload. Furthermore, reliability of detection can also be determined based on the energy difference between ON and OFF OOK symbols.

Observation 28:	The performance of payload-based and sequence-based LP-WUS structure depends on the length of the message content, paging probability of users, and the number of users sharing the LP-WUS.

Observation 29:	The ‘optimum’ size of CRC is dependent on the probability of LP-WUS transmission i.e. what is the ratio between LP-WUS monitoring rate and traffic arrival rate, and number of UEs grouped to one LP-WUS.

Observation 30:	As LP-WUS is expected to be sent more frequently in connected mode, a shorter CRC can provide adequate protection with reduced overhead.

Observation 31:	Uniform distribution of power across all ON pulses ensures reliable RSRP estimation as the signal energy is evaluated by subtracting the respective OFF duration energy.

Observation 32:	As the LP-WUS carries an overlay sequence in the ON duration, it would be beneficial to follow the same approach for LP-SS as well as it may avoid LRs with SD to receive SSBs.

Observation 33:	The pulse shaping waveform for OOK signal can be considered as an overlaid sequence for the sequence-based detectors to perform coherent detection to improve the detection performance.

Observation 34:	The detection accuracy can be improved by averaging the timing estimates over multiple LP-SS monitoring occasions, while ensuring the coherency of the channel fading.

Observation 35:	Increasing the length of sequence beyond {8,16} for M={2,4}, respectively, increases the overhead linearly without improving the estimation accuracy noticeably.

Observation 36:	Averaging over multiple LP-SS MOs improves the RSRP estimation accuracy when employed with uniform power distribution together with or without pulse shaping.

# Appendix

## Probability Analysis

In this section, we will discuss which option is beneficial in terms of information mapping to reduce the overall MR wake-up probability. The paging probability of user k is denoted as pk and the number of subgroups is denoted by G with total K users in a PO. Let pg and pgk be the subgroup paging probability of subgroup g and subgroup gk containing user k, respectively. Let B be the total number of bits used for LP-WUS and let C be the number of CRC bits in bit map scheme. Let KG=KG be the total number of users per each subgroup G and similarly qn and qb represents the bit error probability in the presence of noise and flipping probability when there is a signal, respectively.

### Codepoint or Sequence Approach

In this scheme, each UE k monitors two codeword, namely, a subgroup specific sequence and all UE wake-up sequence. If NMO is used for LP-WUS, then all codeword will be used only when there is at least NMO+1 subgroups are paged. Using this, we can evaluate the total false wake-up probability of a UE k by considering the following.

- Subgroup gk is paged but not due to the user k

  - If more users are assigned to a subgroup, subgroup paging probability will be high.

- No subgroup is paged but codeword is detected from noise at least in those MOs that does not carry any sequence

  - It is based on noise and since Manchester encoding is assumed, two bits should be considered

- Subgroup g≠gk is paged, but it is detected as the codeword corresponding to gk due to false alarm

  - The probability of flipping bits depends on the target BLER, which is considered as ≈1%

  - If more subgroups used, probability of all subgroups wake-up is highly likely.

- More than NMO subgroups paged, which initiates all subgroup paged

Thus, a balance needs to be identified between the number of subgroups G and the number of MOs NMO used for a given number of LP-WUS bits per MO, i.e., B. Let us consider each probabilities separately with

pg=1-(1-pk)KG

Using this, we will define the total false wake-up probability as A+B+C+D, where each of those are specified as below.

Subgroup gk is paged but not due to user k

  - The probability that a subgroup g is paged, but not due to user k∈g, which is given as

A⟹1-1-pkKG-1×(1-pk)

Probability of detecting codeword(s) over at least one MO due to noise

  - In this case, we evaluate the probability that there is a codeword detection, which can be either subgroup gk specific or all subgroup codeword. Depending on the number of subgroups paged, the noise can trigger the codeword detection on one or NMO-1 MOs.

B⟹1-pgk ×i=0NMO-1G-1i×1-pgG-1-i×pgi×1-1-2×qn2BNMO-i

Probability of detecting codeword when some other group is paged

  - Here, the probability that the codeword associated with the subgroup of user k is detected when some other subgroups codeword is transmitted. In this case, the Hamming distance between the codewords play a role in reducing the false wake-up probability.

C⟹1-pgk ×i=1NMOG-ii×1-pgG-i×pgi×1-1-2×qb(B-log2G) i

Probability of using all subgroup wake-up when more than NMO subgroups paged

  - Again, the subgroup corresponding to user k is not paged, but more than NMO subgroups paged. In this case, all subgroup wake-up code word will be sent.

D⟹1-pgk×1-i=0NMOG-1-ii×1-pgG-1-i×pgi

Assuming BLER of 1%, when there is a valid signal present, bit flipping probability can be calculated as qb≈(1-(1 -BLER)1/B. Finally, the miss detection probability, which depends on how densely the codeword is packed to support multiple subgroups can be evaluated depending on the Hamming distance, which can be given as follows.

E⟹pgk×1-i=0B-log2G-1Bi×1-qbB-i×qbi

### Bitmap Approach

In this scheme, the subgroups are denoted by a bitmap with the number of subgroups B=R+log2G+C, where C denotes the number of CRC bits and log2G represents the number of information bits. R in the above expression provides the redundancy or encoding to provide robustness against false alarm, which can be considered as FEC. The use of FEC is like the codepoint or sequence mapping discussed earlier. Using the above notations, let us define the probabilities of various events.

- The subgroup paging probability is given by pg=1-1-pkKG indicating that at least one UE in a subgroup G is paged.

- The probability that a group Gk containing user k is paged, however, not due to user k is given by A as in previous section.

- The false alarm due to successful CRC detection is given by qg=(1-pgk)×qnC, where C is the number of CRC bits and the successful wake-up of MR happens only when the subgroup bits contain a valid wake-up bit pattern in addition to CRC validity. The probability that a bit is detected as either one or zero is equally likely, and thus the bit error probability is considered as 0.5, wherein a bit is determined from noise or no transmission.

- The probability of false detection, which is nothing but a sequence of bits that has certain errors, but the bit indicates the wake-up.

- F⟹1-1-qbBG) i

- where the random error occurred in payload with BG denoting the number of bits per each subgroup, which can either be ≥1 to provide noise robustness.