3GPP TSG RAN WG1 #119		R1-2409596

Orlando, US, November 18th – 22nd, 2024

Agenda item:	9.4.1.1

Source:	Samsung

Title:	Considerations for evaluation assumptions and results

Document for:	Discussion and decision

- Introduction

In RAN#102, the SID on “Study on solutions for Ambient IoT (Internet of Things) in NR” was approved [1], where the objectives of this study are as follows:

4 Objective 4.1 Objective of SI or Core part WI or Testing part WI <The general scope is omitted> The following objectives are set, within the General Scope: - Evaluation assumptions - Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs - Clause 5.8: 2D distribution of devices - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4] - Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1] - Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology. NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically. NOTE: strive to minimize evaluation cases in RAN1. - Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination (coordination with SA3 is required for privacy aspects). - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1. <The rest is omitted>

In this contribution, we share our views on necessary further evaluation assumptions for coverage and evaluation methodologies including interference assumptions. In addition, we discuss how to evaluate other performance metrics other than coverage such as latency and inventory completion time.

- Evaluation Methodology for Coverage

Coverage evaluation assumptions

In this section, we focus on the link level simulation assumptions for A-IoT coverage evaluations.

    - Link-budget Template

      - Receiver Noise figure

Agreement (RAN1#117) - For R2D link in the coverage evaluation for device 2, - Budget-Alt1 is used if receiver architecture is RF ED - Budget-Alt2 is used if receiver architecture is IF/ZIF ED - Note1: this does not preclude to have LLS for device 1 and 2 R2D link with RF-ED if needed. - Note1b: For device 2 R2D link with RF-ED, Budget-Alt1 is mandatory, Budget-Alt2 is optional. - Note2: this does not imply all M values are achievable with the sensitivity given by Budget-Alt1 for RF ED - Note3: For device 2 with an RF ED-based receiver on the R2D link, if the receiver sensitivity derived from Budget-Alt2, assuming a noise figure of [X dB], exceeds the receiver sensitivity based on Budget-Alt1, then Budget-Alt2 is applied.

At RAN1#117, we discussed a value of X for the noise figure in Budget-Alt2. 20 dB is one candidate value for X, and this value comes from the discussions in the LP-WuS study as follows.

##### 7.1.1a.1.1 RF envelope detection For OOK-1/2/4 with RF envelope detection, some sources ([7A-1] [7A-2] [7A-3] [7A-4] [7A-5] [7A-6]) provided analysis, with relative power consumption and noise figure summarized in Table 7.1.1a-1. - - The relative power consumption for ON state is in the range of 0.01~0.2. - - The noise figure is in the range of 12~22 dB. Table 7.1.1a-1: Relative power consumption and noise figure for OOK-1/2/4 with RF envelope detection Source reference; [7A-1]; [7A-2]; [7A-3]; [7A-4]; [7A-5]; [7A-6]. Power consumption (ON state); 0.05 for single-branch, 0.01 for each additional branch; 0.01; 0.01~0.1; 0.01; 0.01~0.1; 0.05~0.2. Noise figure (dB); 20; 17~22; [12-18]; 20; 15; 20.

The noise figure of a device can vary significantly based on the performance of its components. It is influenced by the quality of the components and the specific implementation details. Consequently, even if both AIoT devices and LP-WuR are based on RF-ED based receiver, it is difficult to expect that they have identical hardware configurations in practice due to cost and size constraints of AIoT devices. It is worth noting that AIoT devices are designed to be much cheaper than LP-WuR, which suggests that their noise figure values are likely to be more degraded in comparison.

When there are N cascaded components, the noise figure, F [dB], can be calculated using Friis formula where FX and GX correspond to the noise figure and gain values for the X-th component, respectively. Using the formula, the noise figure of AIoT device 2 can be calculated theoretically. The calculation takes into account the architecture shown in Figure 1, and the LNA can be substituted with a reflection amplifier. For LNA, assume a gain of 10 dB and a noise figure of 1 dB. Therefore, G1 and F1 are respectively 10 and 1.26 on a linear scale. For the envelope detector and filter, assume a gain and noise figure of 0 dB. For ADC, it is possible to calculate the noise figure using the formula for ADC SNR. SNR of ADC [dB] is calculated as 6.02N+1.76, where N represents the number of bits in ADC. Assuming a 4-bit ADC for device 2, the resulting SNR value is 25.84 dB. Assuming an input power between -10 dBm and -40 dBm, the noise power for the ADC, N0 can be between -35.84 dBm and -65.84 dBm. Calculating PSD, No = N0 -10log(1 MHz), it is between -95.85 dBm/Hz and -125.85 dBm/Hz, which is based on a 1MHz bandwidth. Finally, F3 is calculated as log(No) + 174 dBm, resulting in a value between 48.16 to 78.16 dB. Thus, according to Friis’ formula, F can be calculated to be approximately between 36.16 dB to 48.16 dB.

Figure 1. N cascaded components

F= F1+F2-1G1+F3-1G1G2+…++Fn-1G1G2G3…Gn-1=SNRinSNRout

Formula 1. Friis’ Formula

Figure 2. AIoT device 2 receiver

Observation 1. AIoT devices are likely to have a much higher noise figure than LP-WuS since they are targeted to be significantly cheaper than LP-WuR.

Observation 2. Theoretically, the calculated noise figure for AIoT device 2 can be around 50 dB.

Proposal 1. NF value greater than 20 dB should be considered in the coverage evaluation such as 40 and 60 dB for the receiver noise figure of device 2.

    - Preliminary results

In this chapter, we discuss the preliminary results of the coverage evaluation. For the evaluation, the following cases are defined:

Table 1. Coverage evaluation cases for D1T1 scenarios

| No. | Scenario | CW case | Total Tx power (dB) | Tx antenna gain (dBi) | CW cancellation Capability (dBm) | BLER (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | D1T1-A1 | 1-1 | 33 | 2 | 140 | 10 |
| 2-A | D1T1-A2 | 1-1 | 33 | 2 | 120 | 10 |
| 2-B | D1T1-A2 | 1-1 | 33 | 2 | 130 | 10 |
| 3 | D1T1-B | 1-1 | 33 | 2 | 140 | 10 |
| 4 | D1T1-A1 | 1-1 | 24 | 2 | 140 | 10 |
| 5-A | D1T1-A2 | 1-1 | 24 | 2 | 120 | 10 |
| 5-B | D1T1-A2 | 1-1 | 24 | 2 | 130 | 10 |
| 6 | D1T1-B | 1-1 | 24 | 2 | 140 | 10 |
| 7 | D1T1-A1 | 1-4 | 23 | 2 | 140 | 10 |
| 8-A | D1T1-A2 | 1-4 | 23 | 2 | 120 | 10 |
| 8-B | D1T1-A2 | 1-4 | 23 | 2 | 130 | 10 |
| 9 | D1T1-B | 1-4 | 23 | 2 | 140 | 10 |
| 10 | D1T1-A1 | 1-1 | 33 | 2 | 140 | 1 |
| 11-A | D1T1-A2 | 1-1 | 33 | 2 | 120 | 1 |
| 11-B | D1T1-A2 | 1-1 | 33 | 2 | 130 | 1 |
| 12 | D1T1-B | 1-1 | 33 | 2 | 140 | 1 |
| 13 | D1T1-A1 | 1-1 | 24 | 2 | 140 | 1 |
| 14-A | D1T1-A2 | 1-1 | 24 | 2 | 120 | 1 |
| 14-B | D1T1-A2 | 1-1 | 24 | 2 | 130 | 1 |
| 15 | D1T1-B | 1-1 | 24 | 2 | 140 | 1 |
| 16 | D1T1-A1 | 1-4 | 23 | 2 | 140 | 1 |
| 17-A | D1T1-A2 | 1-4 | 23 | 2 | 120 | 1 |
| 17-B | D1T1-A2 | 1-4 | 23 | 2 | 130 | 1 |
| 18 | D1T1-B | 1-4 | 23 | 2 | 140 | 1 |
| 19 | D1T1-C | 1-1 | 33 | 2 | N/A | 10 |
| 20 | D1T1-C | 1-1 | 24 | 2 | N/A | 10 |
| 21 | D1T1-C | 1-4 | 23 | 2 | N/A | 10 |
| 22 | D1T1-C | 1-1 | 33 | 2 | N/A | 1 |
| 23 | D1T1-C | 1-1 | 24 | 2 | N/A | 1 |
| 24 | D1T1-C | 1-4 | 23 | 2 | N/A | 1 |

Table 2. CW interference cancellation capability for D1T1 scenarios

| spatial domain | analog domain | Digital Domain | Others | Total capability |
| --- | --- | --- | --- | --- |
| For D1T1-A2 |  |  |  |  |
| 80 or 90 dBm | 20 dBm | 20 dBm | N/A | 120 or 130 dBm |
| For D1T1-A1/B |  |  |  |  |
| 100 dBm | 20 dBm | 20 dBm | N/A | 140 dBm |

In this case, the evaluation cases X-A and X-B refer to CW self-interference cases (i.e., the ‘A2’ scenario), where CW cancellation capability is assumed to be 120 dBm and 130 dBm, respectively. Based on our observation, we considered CW cancellation capability as shown in Table 2. In the ‘A1/B’ scenarios, CW acts as cross-interference at the reader, whereas in the ‘A2’ scenario, CW functions as self-interference. Therefore, in the ‘A2’ scenario, we can expect worse CW cancellation capability compared to the ‘A1/B’ scenarios. To access the impact of CW cancellation capability on coverage, we considered values of 120 dBm and 130 dBm. For device 2, we calculated and compared Budget-Alt1 and Budget-Alt2 to evaluate R2D coverage. Note that device 2 may have lower receiver sensitivity compared to Device 1 in the case of Budget-Alt1. In the D1T1 scenario, we assumed NF = 60 dB for device 2a for Budget-Alt2. Figure 3 shows the BLER performance of device 2 for R2D coverage evaluation, with a data size of 20 bits, CRC6, and an exact data rate of 5.77 kbps. The remaining simulation assumptions are summarized in Table 3. The required CNR for 10% and 1% BLER targets are as follows:

- SFO = [0.1~1]*104 ppm, 10% BLER target: required CNR = 3.37 dB

- SFO = [0.1~1]*104 ppm, 1% BLER target: required CNR = 15.60 dB

Figure 3. BLER performance of R2D transmission

Figure 4 shows the BLER performance of device 1 and device 2 for D2R coverage evaluation, with data sizes of 20 and 96 bits, CRC6, and a reference data rate of 7kbps. In the figure, D1 and D2 represent device 1 and device 2, respectively. The remaining simulation assumptions are summarized in Table 3. In D2R reception, the reader estimates the device’s clock drift and adjusts its receiving clock to compensate for SFO accumulation in each OFDM symbol, while in R2D reception, where the receiver is a device, SFO accumulates across the entire received signal.

Figure 4. BLER performance of D2R transmission

The required CNR targets are as follows:

- Device 1, 20 bits payload: SFO = [0.1~1]*105 ppm, 10% BLER target: required CNR = 6 dB, 1% BLER target: required CNR = 8 dB

- Device 2, 20 bits payload: SFO = [0.1~1]*104 ppm, 10% BLER target: required CNR = 6 dB, 1% BLER target: required CNR = 8 dB

Table 3. Link level simulation assumption for the coverage evaluation

|  | Parameters |  | Assumptions |
| --- | --- | --- | --- |
|  | R2D/D2R common parameters |  |  |
| [0a] | Carrier frequency |  | 900 MHz |
| [0b] | SCS |  | 15 kHz |
| [0c] | Block structure |  | Data only |
| [0d] | Channel model |  | TDL-A for InF |
| [0e] | Delay spread |  | 30 ns |
| [0f] | Device velocity |  | 3 km/h |
| [0g] | Number of Tx/Rx chains for Ambient IoT device |  | 1 |
| [0h1] | BS | Number of antenna elements | 2 |
| [0h2] |  | Number of TXRUs | 2 |
| [0m] | Reference data rate |  | 5~7 kbps |
| [0n] | Message size |  | 20 bits |
| [0p] | BLER target |  | 1%, 10% |
| [0q] | Sampling frequency |  | 1.92Msps - Initial SFO: Randomly select a value from the range of [0.1 ~ 1] *10^5 ppm for device 1 and [0.1 ~ 1] *10^4 ppm for device 2 |
| [0r] | Device 1/2a/2b |  | RF-ED |
| R2D specific parameters |  |  |  |
| [1a] | Transmission bandwidth |  | 180 kHz |
| [1b] | ED bandwidth |  | 20 MHz Note: this does not imply that a A-IoT device supports sampling clock rate as large as RF ED bandwidth. |
| [1c] | BB LPF |  | 3-order Butterworth/RC filter with cutoff frequency at half of R2D transmission bandwidth. |
| [1d] | Waveform |  | OOK waveform generated by OFDM modulator |
| [1e] | Modulation |  | OOK-1 |
| [1f] | Line code |  | Manchester |
| [1g] | FEC |  | No FEC |
| [1h] | ADC bit width |  | 4-bit for device 2 |
| [1j] | Detection method for Line code |  | Edge detector |
| D2D specific parameters |  |  |  |
| [2a1] | Transmission bandwidth |  | [2a1]-Alt1 (M): - DSB 15 kHz considered for D2R transmission bandwidth |
| [2a3] | Receiver bandwidth |  | 15 kHz |
| [2b] | Waveform (CW) |  | Unmodulated single tone |
| [2d] | Modulation |  | OOK |
| [2e] | Line code |  | Manchester |
| [2g] | FEC |  | No FEC |
| [2h] | ADC bit width |  | ideal |
| [2j] | D2R receiver |  | Non-coherent receiver |

      - Evaluation results for Device 1 in D1T1

For device 1, the R2D coverage evaluation was performed using Budget-Alt1 with a receiver sensitivity of -30 dB. The coverage evaluation results are summarized in Table 3. From the results below, for device 1 in CW case 1-1, it is difficult to achieve 10-50m coverage with the transmission power of 24 dBm. Even with the transmission power of 33 dBm, it is challenging to achieve 10-50m coverage in scenario ‘A2’, where CW self-interference cancellation is required and lower interference cancellation capability is expected. In CW case 1-4, due to the low transmission power of 23 dBm, it is hard to achieve 10-50m coverage in all cases. The total transmission power and Tx antenna gain were assumed to be the same for both R2D and CW.

Table 3. Coverage evaluation results for device 1

| No. | R2D Coverage | D2R Coverage | Effective coverage |
| --- | --- | --- | --- |
| 1 | 11.69m | 19.76m | 11.69m |
| 2-A | 11.69m | 2.27m | 2.27m |
| 2-B | 11.69m | 6.50m | 6.50m |
| 3 | 11.69m | 19.76m | 11.69m |
| 4 | 4.54m | 19.69m | 4.54m |
| 5-A | 4.54m | 2.27m | 2.27m |
| 5-B | 4.54m | 6.49m | 4.54m |
| 6 | 4.54m | 19.69m | 4.54m |
| 7 | 4.09m | 19.61m | 4.09m |
| 8-A | 4.09m | 2.27m | 2.27m |
| 8-B | 4.09m | 6.49m | 4.09m |
| 9 | 4.09m | 19.61m | 4.09m |
| 10 | 11.69m | 17.79m | 11.69m |
| 11-A | 11.69m | 1.84m | 1.84m |
| 11-B | 11.69m | 5.27m | 5.27m |
| 12 | 11.69m | 17.79m | 11.69m |
| 13 | 4.54m | 17.73m | 4.54m |
| 14-A | 4.54m | 1.84m | 1.84m |
| 14-B | 4.54m | 5.26m | 4.54m |
| 15 | 4.54m | 17.73m | 4.54m |
| 16 | 4.09m | 17.65m | 4.09m |
| 17-A | 4.09m | 1.84m | 1.84m |
| 17-B | 4.09m | 5.26m | 4.09m |
| 18 | 4.09m | 17.65m | 4.09m |

Observation 3. For device 1, in CW Case 1-1, with the transmission power of 24 dBm and the antenna gain of 2 dBi, achieving coverage of 10 to 50m is challenging for all scenarios ‘A1/A2/B’.

Observation 4. For device 1, in CW Case 1-1, with the transmission power of 33 dBm and the antenna gain of 2 dBi, the coverage is 11.69m in scenario ‘A1/B’, but in scenario ‘A2’, due to lower CW interference cancellation capability, it is difficult to achieve 10 to 50m coverage.

Observation 5. For device 1, in CW Case 1-4, achieving coverage of 10 to 50m is challenging in all scenarios ‘A1/A2/B’ due to the low total transmission power of 23 dB.

      - Evaluation results for Device 2a in D1T1

For device 2a, the R2D coverage evaluation was performed by calculating and comparing the coverage results obtained from both Budget-alt1 and Budget-Alt2. A receiver sensitivity of -40 dB was assumed for Budget-Alt1. The coverage evaluation results are summarized in Table 4. At 10% BLER, coverage of 19.76m with 33 dBm transmission power and 2 dBi antenna gain, and 10.11m with 24 dBm transmission power, was achieved in the ‘A1/B’ scenarios. On the other hand, in the ‘A2’ scenario, where CW cancellation capability is lower, achieving 10 to 50 m coverages is challenging. In addition, in CW case 1-4, where the transmission power is limited to 23 dBm, it is difficult to achieve 10 to 50m coverage. The total transmission power and Tx antenna gain were assumed to be the same for both R2D and CW.

Table 4. Coverage evaluation results for device 2a

| No. | R2D Coverage | D2R Coverage | Effective coverage |
| --- | --- | --- | --- |
| 1 | 26.05 | 19.76 | 19.76 |
| 2-A | 26.05 | 2.27 | 2.27 |
| 2-B | 26.05 | 6.50 | 6.50 |
| 3 | 26.05 | 19.76 | 19.76 |
| 4 | 10.11 | 19.69 | 10.11 |
| 5-A | 10.11 | 2.27 | 2.27 |
| 5-B | 10.11 | 6.49 | 6.49 |
| 6 | 10.11 | 19.69 | 10.11 |
| 7 | 9.10 | 19.61 | 9.10 |
| 8-A | 9.10 | 2.27 | 2.27 |
| 8-B | 9.10 | 6.49 | 6.49 |
| 9 | 9.10 | 19.61 | 9.10 |
| 10 | 7.20 | 17.79 | 7.20 |
| 11-A | 7.20 | 1.84 | 1.84 |
| 11-B | 7.20 | 5.27 | 5.27 |
| 12 | 7.20 | 17.79 | 7.20 |
| 13 | 2.79 | 17.73 | 2.79 |
| 14-A | 2.79 | 1.84 | 1.84 |
| 14-B | 2.79 | 5.26 | 2.79 |
| 15 | 2.79 | 17.73 | 2.79 |
| 16 | 2.52 | 17.65 | 2.52 |
| 17-A | 2.52 | 1.84 | 1.84 |
| 17-B | 2.52 | 5.26 | 2.52 |
| 18 | 2.52 | 17.65 | 2.52 |

Observation 6. For device 2a, in CW case 1-1 at 1% BLER, achieving coverage of 10 to 50m is challenging for all scenarios ‘A1/A2/B’.

Observation 7. For device 2a, in CW Case 1-1 at 10% BLER, in scenarios ‘A1/B’, the total transmission power of 33 dB with an antenna gain of 2 dBi can achieve a coverage of 19.76m, while the transmission power of 24 dB can achieve a coverage of 10.11m. However, achieving a coverage of 10 to 50 meters is difficult for scenario ‘A2’ due to the lower CW cancellation capability of 120 dB or 130 dB.

Observation 8. For device 2a, in CW Case 1-4, achieving coverage of 10 to 50m is challenging for all scenarios ‘A1/A2/B’ due to the limited total transmission power of 23 dB.

      - Evaluation results for Device 2b in D1T1

For device 2b, the R2D coverage evaluation was performed using Budget-Alt2. Total Tx power for D2R is assumed to be -20 dBm. For the cases 22 to 24 at 1% BLER, achieving coverage of 10 to 50m is challenging. While achieving coverage of 10 to 50m is challenging in CW Case 1-4. In CW case 1-1, the R2D total transmission power of 33 dBm with the antenna gain of 2 dBi can achieve a coverage of 26.05m, while the R2D total transmission power 24 dBm can achieve a coverage of 10.11m.

Table 5. Coverage evaluation results for device 2b

| No. | R2D Coverage (InF-DH NLOS) | D2R Coverage (InF-DH NLOS) | Coverage |
| --- | --- | --- | --- |
| 19 | 26.05m | 992.07m | 26.05m |
| 20 | 10.11m | 992.07m | 10.11m |
| 21 | 9.10m | 992.07m | 9.10m |
| 22 | 7.20m | 803.93m | 7.20m |
| 23 | 2.79m | 803.93m | 2.79m |
| 24 | 2.04m | 803.93m | 2.04m |

Observation 9. For device 2b, in CW case 1-1 at 1% BLER, achieving coverage of 10 to 50m is challenging for scenario ‘C’.

Observation 10. For device 2b, in CW Case 1-1 at 10% BLER, the R2D total transmission power of 33 dB with an antenna gain of 2 dBi can achieve a coverage of 26.05m, while the R2D transmission power of 24 dB can achieve a coverage of 10.11m.

Observation 11. For device 2b, in CW Case 1-4, achieving coverage of 10 to 50m is challenging because of the limited total transmission power of 23 dBm.

Observation 12. At 10% BLER, when the EIRP at the Tx side is less than 25 dBm, the R2D link can act as a coverage bottleneck, making it challenging to achieve coverage of 10 to 50m.

Observation 13. At 10% BLER, when CW cancellation capability is less than 120 dBm for Device 1 and Device 2a, the D2R can be a coverage bottleneck, making it challenging to achieve coverage of 10 to 50m.

Proposal 2. Capture the following observations in the TR.

Observation X. At 10% BLER, when the EIRP at the Tx side is less than 25 dBm, the R2D link can act as a coverage bottleneck, making it challenging to achieve coverage of 10 to 50m.

Observation Y. At 10% BLER, when CW cancellation capability is less than 120 dBm for Device 1 and Device 2a, the D2R can be a coverage bottleneck, making it challenging to achieve coverage of 10 to 50m.

Table 6 summarizes the link budget values used in the above results.

Table 6. Link budget table for the coverage evaluation

| No. | Item | R2D | D2R |
| --- | --- | --- | --- |
|  | Cell area reliability (%) | 1% or 10% | 1% or 10% |
| (0) System configuration |  |  |  |
| 0A | Scenarios | Reported | Reported |
| 0A1 | CW case | N/A | 1-1 or 1-4 |
| 0B | Device type | Device 1 or 2a | Device 1 or 2a |
| 0C | Center frequency (GHz) | 0.90 | 0.90 |
| 0D | Topology/Pathloss model | InF-DH NLOS | InF-DH NLOS |
| (1) Transmitter |  |  |  |
| 1D | Number of Tx antenna elements / TxRU/ Tx chains modelled in LLS | 2 | 1 |
| 1E | Total Tx Power (dBm) | 33 or 24 or 23 | - Calculated for device 1/2a - -20 for device2b |
| 1E1 | CW Tx power (dBm) | N/A | 33 or 24 or 23 |
| 1E2 | CW Tx antenna gain (dBi) | N/A | 2 |
| 1E3 | CW2D distance (m) | N/A | Calculated for device 1/2a |
| 1E4 | CW2D pathloss (dB) | N/A | Calculated for device 1/2a |
| 1E5 | CW received power (dBm) | N/A | Calculated for device 1/2a |
| 1F | Transmission Bandwidth used for the evaluated channel (kHz) | 180 | 15 |
| 1G | Tx antenna gain (dBi) | 2 | 0 |
| 1H | Ambient IoT backscatter loss due to Modulation factor (dB) | N/A | 6 |
| 1J | Ambient IoT on-object antenna penalty (dB) | N/A | 0.9 |
| 1K | Ambient IoT backscatter amplifier gain (dB) | N/A | 10 |
| 1N | Cable, connector, combiner, body losses, etc. (dB) | 1 | N/A |
| 1M | EIRP (dBm) | Calculated | Calculated |
| (2) Receiver |  |  |  |
| 2A | Number of receive antenna elements / TxRU / chains modelled in LLS | 1 | 2 |
| 2B | Bandwidth used for the evaluated channel (Hz) | 20MHz | 15kHz |
| 2C | Receiver antenna gain (dBi) | 0 | 6 |
| 2X | Cable, connector, combiner, body losses, etc. (dB) | N/A | 1 |
| 2D | Receiver Noise Figure (dB) | 60 for Alt2 | 5 |
| 2E | Thermal Noise power spectrum density (dBm/Hz) | -174 for Alt2 | -174 |
| 2F | Noise Power (dBm) | Calculated for Alt 2 | Calculated |
| 2G | Required SNR | Reported for Alt2 | Reported |
| 2H | Ambient IoT on-object antenna penalty (dB) | 0.9 | N/A |
| 2J | Budget-Alt1/ Budget-Alt2 | - Alt1 for device 1 - Alt2 for device 2 | Alt2 |
| 2K | CW cancellation (dB) | N/A | - 140 dB for A1/B scenarios - 120 or 130 dB for A2 scenarios |
| 2K1 | Remaining CW interference (dB) | N/A | Calculated |
| 2K2 | Receiver Sensitivity loss (dB) | N/A | Calculated |
| 2L | Receiver Sensitivity (dBm) | - -30 for Alt1 - Calculated for Alt2 | Calculated |
| 3A | Shadow fading margin (dB) | 4 | 4 |
| 3B | polarization mismatching loss (dB) | 3 | 3 |
| 3C | BS selection/macro-diversity gain (dB) | 0 | 0 |
| 3D | Other gains (dB) (if any please specify) | 0 | 0 |
|  | (4) MPL / distance |  |  |
| 4A | MPL (dB) | Calculated | Calculated |
| 4B | Distance (m) | Calculated | Calculated |
| 5A | Other notes | - | - |

Considerations on Latency in A-IoT systems

    - Latency for a single A-IoT device

Agreement (RAN1#118) Definition of the latency is refined as follows, - For the use case of “inventory-only”: - The time interval between the time that the A-IoT paging is sent from reader to a A-IoT device and the time that the inventory report is successfully received at reader from the A-IoT device, i.e., for completing Step A and Step B (as per RAN2 agreements). - For the use case of “inventory and command”: - The time interval between the time that the A-IoT paging is sent from reader and the time that - the command is successfully received at A-IoT device, i.e., for completing Step A, Step B, Step C (as per RAN2 agreements), if Step D is optional and not used (note: pending RAN2 decision on optionality of Step D). - the response is successfully received at Reader, i.e., for completing Step A, Step B, Step C, and Step D (as per RAN2 agreements). - Note: the “successfully received” (i.e., decoding successfully) - Expected value of latency is calculated according a X% re-attempt probability to each attempt. - Note: the latency is evaluated for a single A-IoT device. - Note: Time for energy harvesting is not included in the definition of latency. Agreement (RAN1#118) Companies to report the calculation of single-device latency according to the follow tables Table XX: calculation of single-device latency for “inventory-only” Steps; Procedures; Time (us). Step A: A-IoT paging; A-IoT paging message. Gap A (if any). Step B: D2R data transmission.; A-IoT Random access procedure. Gap B1. D2R data transmission. Total time for one attempt, T1= Tstep_A+TGap_A+ Tstep_B. Average time considering all the re-attempts, e.g., T2 = T1 /(1-X). Note: the following is assumed - Data rate: 1kbps or 7kbps - The rate for re-attempt probability X = [10] % - Message size for each procedure: FFS. Table YY: calculation of single-device latency for “inventory and command” Steps; Procedures; Time (us). Step A: A-IoT paging; A-IoT paging message. Gap A (if any). Step B: D2R data transmission.; A-IoT Random access procedure. Gap B1. D2R data transmission. Gap B (if any). Step C: Data transmission; R2D data transmission. Gap C (if any). Step D (optional, pending RAN2 decision); D2R data transmission. Total time for one attempt, T1= Tstep_A+TGap_A+ Tstep_B+TGap_B+ Tstep_C+ +TGap_C +Tstep_D. Average time considering all the re-attempts, e.g., T2 = T1 / (1-X). Note: the following is assumed - Data rate: 1kbps or 7kbps - The rate for re-attempt probability X = [10] % - Message size for each procedure: FFS.

During RAN1#118, the above agreements were reached regarding the latency for a single A-IoT device. Figure 5 illustrates the A-IoT 4-step and 2-step random access (RA) procedures based on the RAN2#126 agreements. In case of 4-step RA, Step A, which refers to A-IoT paging message sent by the reader to the device, includes the transmission of Msg 0. Step B, where the device ID transmission occurs, includes the transmission of Msg 1, Msg 2 and Msg 3. Msg 0 is considered a simple triggering message, assumed to carry 20 information bits. Msg 1 and Msg 2, which involve the device ID transmission and echoing, are also assumed to carry 20 bits of information. For Msg 3, since upper layer data is transmitted, it is assumed to carry 96 information bits. In case of 2-step RA, Step A includes the transmission of Msg 0 and Step B includes the transmission of Msg 1. As with the 4-step RA, Msg 0 can be assumed to carry 20 bits of information. On the other hand, Msg 1 can be assumed to carry 96 bits of information, as it includes both the device ID and upper layer data. The time required to transmit each message can be calculated as ‘(information length) / (data rate)’.

Figure 5. 4-step and 2-step random access based on RAN2 #126 agreements

Based on the assumptions above, the calculated latency for a single A-IoT device is shown in Table 7-10 below. For Step C, it is assumed that 96 bits of data are transmitted. A gap of 0.2ms was assumed between all procedures. Note that this gap does not account for energy harvesting time, meaning any period where the device is not transmitting or receiving data for charging purpose is excluded.

Table 7. Latency of a single A-IoT device for “inventory-only” based on the 4-step RA

| Steps | Procedures |  | Time (ms) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1kbps | 7kbps |
| Step A: A-IoT paging | A-IoT paging message | T0 | 20 | 2.86 |
| Gap A (if any) |  | Tgap(0, 1) | 0.2 | 0.2 |
| Step B: D2R data transmission. | A-IoT Random access procedure | (T1+T2+Tgap(1,2)) | 40.2 | 5.91 |
|  | Gap B1 | Tgap(2, 3) | 0.2 | 0.2 |
|  | D2R data transmission | T3 | 96 | 13.71 |
| Total time for one attempt, T1= Tstep_A+Tgap_A+ Tstep_B |  |  | 156.6 | 22.88 |
| Average time considering all the re-attempts, e.g., T2 = T1 /(1-X), X = 10% |  |  | 174 | 25.42 |

Table 8. Latency of a single A-IoT device for “inventory-only” based on the 2-step RA

| Steps | Procedures |  | Time (ms) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1kbps | 7kbps |
| Step A: A-IoT paging | A-IoT paging message | T0 | 20 | 2.86 |
| Gap A (if any) |  | Tgap(0, 1) | 0.2 | 0.2 |
| Step B: D2R data transmission. | A-IoT Random access procedure | - | - | - |
|  | Gap B1 | - | - | - |
|  | D2R data transmission | T1 | 96 | 13.71 |
| Total time for one attempt, T1= Tstep_A+Tgap_A+ Tstep_B |  |  | 116.2 | 16.77 |
| Average time considering all the re-attempts, e.g., T2 = T1 /(1-X), X = 10% |  |  | 129.1 | 18.63 |

Table 9. Latency of a single A-IoT device for “inventory-command” based on the 4-step RA

| Steps | Procedures |  | Time (ms) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1kbps | 7kbps |
| Step A: A-IoT paging | A-IoT paging message | T0 | 20 | 2.86 |
| Gap A (if any) |  | Tgap(0, 1) | 0.2 | 0.2 |
| Step B: D2R data transmission. | A-IoT Random access procedure | (T1+T2+Tgap(1,2)) | 40.2 | 5.91 |
|  | Gap B1 | Tgap(2, 3) | 0.2 | 0.2 |
|  | D2R data transmission | T3 | 96 | 13.71 |
| Gap B (if any) |  | Tgap, command | 0.2 | 0.2 |
| Step C: data transmission. | R2D data transmission | Tcommand | 96 | 13.71 |
| Total time for one attempt, T1= Tstep_A+Tgap_A+ Tstep_B+Tgap_B+Tstep_C |  |  | 252.8 | 36.79 |
| Average time considering all the re-attempts, e.g., T2 = T1 /(1-X), X = 10% |  |  | 280.89 | 40.88 |

Table 10. Latency of a single A-IoT device for “inventory-command” based on the 2-step RA

| Steps | Procedures |  | Time (ms) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1kbps | 7kbps |
| Step A: A-IoT paging | A-IoT paging message | T0 | 20 | 2.86 |
| Gap A (if any) |  | Tgap(0, 1) | 0.2 | 0.2 |
| Step B: D2R data transmission. | A-IoT Random access procedure | - | - | - |
|  | Gap B1 | - | - | - |
|  | D2R data transmission | T1 | 96 | 13.71 |
| Gap B (if any) |  | Tgap, command | 0.2 | 0.2 |
| Step C: data transmission. | R2D data transmission | Tcommand | 96 | 13.71 |
| Total time for one attempt, T1= Tstep_A+Tgap_A+ Tstep_B+Tgap_B+Tstep_C |  |  | 212.4 | 30.68 |
| Average time considering all the re-attempts, e.g., T2 = T1 /(1-X), X = 10% |  |  | 236 | 34.1 |

Proposal 3. The following is assumed to calculate the latency for a single device.

|  | Data Size |  |
| --- | --- | --- |
|  | 4-step | 2-step |
| Msg 0 (A-IoT Paging) | 20 bits | 20 bits |
| Msg 1 | 20 bits | 96 bits |
| Msg 2 | 20 bits | - |
| Msg 3 | 96 bits | - |

Observation 14. The latency for a single device is calculated as follows:

Table A. Latency of a single A-IoT device for “inventory-only” based on the 4-step RA

| Steps | Procedures |  | Time (ms) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1kbps | 7kbps |
| Step A: A-IoT paging | A-IoT paging message | T0 | 20 | 2.86 |
| Gap A (if any) |  | Tgap(0, 1) | 0.2 | 0.2 |
| Step B: D2R data transmission. | A-IoT Random access procedure | (T1+T2+Tgap(1,2)) | 40.2 | 5.91 |
|  | Gap B1 | Tgap(2, 3) | 0.2 | 0.2 |
|  | D2R data transmission | T3 | 96 | 13.71 |
| Total time for one attempt, T1= Tstep_A+Tgap_A+ Tstep_B |  |  | 156.6 | 22.88 |
| Average time considering all the re-attempts, e.g., T2 = T1 /(1-X), X = 10% |  |  | 174 | 25.42 |

Table B. Latency of a single A-IoT device for “inventory-only” based on the 2-step RA

| Steps | Procedures |  | Time (ms) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1kbps | 7kbps |
| Step A: A-IoT paging | A-IoT paging message | T0 | 20 | 2.86 |
| Gap A (if any) |  | Tgap(0, 1) | 0.2 | 0.2 |
| Step B: D2R data transmission. | A-IoT Random access procedure | - | - | - |
|  | Gap B1 | - | - | - |
|  | D2R data transmission | T1 | 96 | 13.71 |
| Total time for one attempt, T1= Tstep_A+Tgap_A+ Tstep_B |  |  | 116.2 | 16.77 |
| Average time considering all the re-attempts, e.g., T2 = T1 /(1-X), X = 10% |  |  | 129.1 | 18.63 |

Table C. Latency of a single A-IoT device for “inventory-command” based on the 4-step RA

| Steps | Procedures |  | Time (ms) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1kbps | 7kbps |
| Step A: A-IoT paging | A-IoT paging message | T0 | 20 | 2.86 |
| Gap A (if any) |  | Tgap(0, 1) | 0.2 | 0.2 |
| Step B: D2R data transmission. | A-IoT Random access procedure | (T1+T2+Tgap(1,2)) | 40.2 | 5.91 |
|  | Gap B1 | Tgap(2, 3) | 0.2 | 0.2 |
|  | D2R data transmission | T3 | 96 | 13.71 |
| Gap B (if any) |  | Tgap, command | 0.2 | 0.2 |
| Step C: data transmission. | R2D data transmission | Tcommand | 96 | 13.71 |
| Total time for one attempt, T1= Tstep_A+Tgap_A+ Tstep_B+Tgap_B+Tstep_C |  |  | 252.8 | 36.79 |
| Average time considering all the re-attempts, e.g., T2 = T1 /(1-X), X = 10% |  |  | 280.89 | 40.88 |

Table D. Latency of a single A-IoT device for “inventory-command” based on the 2-step RA

| Steps | Procedures |  | Time (ms) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1kbps | 7kbps |
| Step A: A-IoT paging | A-IoT paging message | T0 | 20 | 2.86 |
| Gap A (if any) |  | Tgap(0, 1) | 0.2 | 0.2 |
| Step B: D2R data transmission. | A-IoT Random access procedure | - | - | - |
|  | Gap B1 | - | - | - |
|  | D2R data transmission | T1 | 96 | 13.71 |
| Gap B (if any) |  | Tgap, command | 0.2 | 0.2 |
| Step C: data transmission. | R2D data transmission | Tcommand | 96 | 13.71 |
| Total time for one attempt, T1= Tstep_A+Tgap_A+ Tstep_B+Tgap_B+Tstep_C |  |  | 212.4 | 30.68 |
| Average time considering all the re-attempts, e.g., T2 = T1 /(1-X), X = 10% |  |  | 236 | 34.1 |

- Conclusion

In this contribution, we made the following observations and proposals:

Observation 1. AIoT devices are likely to have a much higher noise figure than LP-WuS since they are targeted to be significantly cheaper than LP-WuR.

Observation 2. Theoretically, the calculated noise figure for AIoT device 2 can be around 50 dB.

Proposal 1. NF value greater than 20 dB should be considered in the coverage evaluation such as 40 and 60 dB for the receiver noise figure of device 2.

Observation 3. For device 1, in CW Case 1-1, with the transmission power of 24 dBm and the antenna gain of 2 dBi, achieving coverage of 10 to 50m is challenging for all scenarios ‘A1/A2/B’.

Observation 4. For device 1, in CW Case 1-1, with the transmission power of 33 dBm and the antenna gain of 2 dBi, the coverage is 11.69m in scenario ‘A1/B’, but in scenario ‘A2’, due to lower CW interference cancellation capability, it is difficult to achieve 10 to 50m coverage.

Observation 5. For device 1, in CW Case 1-4, achieving coverage of 10 to 50m is challenging in all scenarios ‘A1/A2/B’ due to the low total transmission power of 23 dB.

Observation 6. For device 2a, in CW case 1-1 at 1% BLER, achieving coverage of 10 to 50m is challenging for all scenarios ‘A1/A2/B’.

Observation 7. For device 2a, in CW Case 1-1 at 10% BLER, in scenarios ‘A1/B’, the total transmission power of 33 dB with an antenna gain of 2 dBi can achieve a coverage of 19.76m, while the transmission power of 24 dB can achieve a coverage of 10.11m. However, achieving a coverage of 10 to 50 meters is difficult for scenario ‘A2’ due to the lower CW cancellation capability of 120 dB or 130 dB.

Observation 8. For device 2a, in CW Case 1-4, achieving coverage of 10 to 50m is challenging for all scenarios ‘A1/A2/B’ due to the limited total transmission power of 23 dB.

Observation 9. For device 2b, in CW case 1-1 at 1% BLER, achieving coverage of 10 to 50m is challenging for scenario ‘C’.

Observation 10. For device 2b, in CW Case 1-1 at 10% BLER, the R2D total transmission power of 33 dB with an antenna gain of 2 dBi can achieve a coverage of 26.05m, while the R2D transmission power of 24 dB can achieve a coverage of 10.11m.

Observation 11. For device 2b, in CW Case 1-4, achieving coverage of 10 to 50m is challenging because of the limited total transmission power of 23 dBm.

Observation 12. At 10% BLER, when the EIRP at the Tx side is less than 25 dBm, the R2D link can act as a coverage bottleneck, making it challenging to achieve coverage of 10 to 50m.

Observation 13. At 10% BLER, when CW cancellation capability is less than 120 dBm for Device 1 and Device 2a, the D2R can be a coverage bottleneck, making it challenging to achieve coverage of 10 to 50m.

Proposal 2. Capture the following observations in the TR.

Observation X. At 10% BLER, when the EIRP at the Tx side is less than 25 dBm, the R2D link can act as a coverage bottleneck, making it challenging to achieve coverage of 10 to 50m.

Observation Y. At 10% BLER, when CW cancellation capability is less than 120 dBm for Device 1 and Device 2a, the D2R can be a coverage bottleneck, making it challenging to achieve coverage of 10 to 50m.

Proposal 3. The following is assumed to calculate the latency for a single device.

|  | Data Size |  |
| --- | --- | --- |
|  | 4-step | 2-step |
| Msg 0 (A-IoT Paging) | 20 bits | 20 bits |
| Msg 1 | 20 bits | 96 bits |
| Msg 2 | 20 bits | - |
| Msg 3 | 96 bits | - |

Observation 14. The latency for a single device is calculated as follows:

Table A. Latency of a single A-IoT device for “inventory-only” based on the 4-step RA

| Steps | Procedures |  | Time (ms) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1kbps | 7kbps |
| Step A: A-IoT paging | A-IoT paging message | T0 | 20 | 2.86 |
| Gap A (if any) |  | Tgap(0, 1) | 0.2 | 0.2 |
| Step B: D2R data transmission. | A-IoT Random access procedure | (T1+T2+Tgap(1,2)) | 40.2 | 5.91 |
|  | Gap B1 | Tgap(2, 3) | 0.2 | 0.2 |
|  | D2R data transmission | T3 | 96 | 13.71 |
| Total time for one attempt, T1= Tstep_A+Tgap_A+ Tstep_B |  |  | 156.6 | 22.88 |
| Average time considering all the re-attempts, e.g., T2 = T1 /(1-X), X = 10% |  |  | 174 | 25.42 |

Table B. Latency of a single A-IoT device for “inventory-only” based on the 2-step RA

| Steps | Procedures |  | Time (ms) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1kbps | 7kbps |
| Step A: A-IoT paging | A-IoT paging message | T0 | 20 | 2.86 |
| Gap A (if any) |  | Tgap(0, 1) | 0.2 | 0.2 |
| Step B: D2R data transmission. | A-IoT Random access procedure | - | - | - |
|  | Gap B1 | - | - | - |
|  | D2R data transmission | T1 | 96 | 13.71 |
| Total time for one attempt, T1= Tstep_A+Tgap_A+ Tstep_B |  |  | 116.2 | 16.77 |
| Average time considering all the re-attempts, e.g., T2 = T1 /(1-X), X = 10% |  |  | 129.1 | 18.63 |

Table C. Latency of a single A-IoT device for “inventory-command” based on the 4-step RA

| Steps | Procedures |  | Time (ms) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1kbps | 7kbps |
| Step A: A-IoT paging | A-IoT paging message | T0 | 20 | 2.86 |
| Gap A (if any) |  | Tgap(0, 1) | 0.2 | 0.2 |
| Step B: D2R data transmission. | A-IoT Random access procedure | (T1+T2+Tgap(1,2)) | 40.2 | 5.91 |
|  | Gap B1 | Tgap(2, 3) | 0.2 | 0.2 |
|  | D2R data transmission | T3 | 96 | 13.71 |
| Gap B (if any) |  | Tgap, command | 0.2 | 0.2 |
| Step C: data transmission. | R2D data transmission | Tcommand | 96 | 13.71 |
| Total time for one attempt, T1= Tstep_A+Tgap_A+ Tstep_B+Tgap_B+Tstep_C |  |  | 252.8 | 36.79 |
| Average time considering all the re-attempts, e.g., T2 = T1 /(1-X), X = 10% |  |  | 280.89 | 40.88 |

Table D. Latency of a single A-IoT device for “inventory-command” based on the 2-step RA

| Steps | Procedures |  | Time (ms) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1kbps | 7kbps |
| Step A: A-IoT paging | A-IoT paging message | T0 | 20 | 2.86 |
| Gap A (if any) |  | Tgap(0, 1) | 0.2 | 0.2 |
| Step B: D2R data transmission. | A-IoT Random access procedure | - | - | - |
|  | Gap B1 | - | - | - |
|  | D2R data transmission | T1 | 96 | 13.71 |
| Gap B (if any) |  | Tgap, command | 0.2 | 0.2 |
| Step C: data transmission. | R2D data transmission | Tcommand | 96 | 13.71 |
| Total time for one attempt, T1= Tstep_A+Tgap_A+ Tstep_B+Tgap_B+Tstep_C |  |  | 212.4 | 30.68 |
| Average time considering all the re-attempts, e.g., T2 = T1 /(1-X), X = 10% |  |  | 236 | 34.1 |

References:

RP-234058 “New SID: Study on solutions for Ambient IoT (Internet of Things) in NR”