3GPP TSG RAN WG1 #119		R1-2409604

Orlando, US, November 18th – 22nd, 2024

Agenda item:	9.5.3

Source:	Samsung

Title:	Adaptation of common signal/channel transmissions

Document for:	Discussion and decision

# Introduction

A WI on enhancements of network energy saving for NR was endorsed in [1], and revised in [2]. One objective in the revised WID is to specify adaptation of common signal/channel transmissions as following.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

This contribution discusses how to support adaptation of common signal/channel transmissions, including the adaptation of SSB in time domain, and adaptation of PRACH in time domain.

# Adaptation of SSB transmissions in time domain

The following agreement was made in RAN1#116 meeting regarding whether the SSB adaptation is triggered by the network or the UE [3].

Agreement For the adaptation mechanisms of SSB in time-domain, study further following mechanisms: - Adaptation mechanism indicated or configured by gNB without UE trigger - Adaptation triggered by UE (if any) FFS: Details of associated signaling/indication/configuration.

The following agreement was further made in RAN1#116bis meeting [4].

Agreement For indication of adaptation of SSB in time-domain, - Support at least SSB adaptation provided by gNB without UE trigger

In some situations, the gNB are not always aware of UE’s channel conditions, for example, beam degrading, out of synchronization, etc. UE triggered SSB adaptation can reduce the channel state report latency and therefore can be beneficial for the throughput as well as the transmission period. Reducing the transmission period is beneficial for the network energy saving, thus, it is necessary to support UE triggering the SSB adaptation for these cases.

Observation 1: UE triggered SSB adaptation is beneficial for network energy saving.

Proposal 1: For the adaptation mechanisms of SSB transmissions in time-domain, support adaptation mechanism indicated or configured by gNB with UE trigger.

The following agreement was made in RAN1#116bis meeting [4].

Agreement Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s): - For cell with both legacy UEs and Rel-19 NES-capable UEs - Rel-19 NES-capable UE’s PCell (Connected mode) - Study from the following options: - Option A1: adaptation for CD-SSB - Option A2: adaptation for SSB that is not CD-SSB - Option A3: adaptation for SSB not on sync raster - Rel-19 NES-capable UE’s SCell - Study from the following options: - Option B1: adaptation for CD-SSB - Option B2: adaptation for SSB that is not CD-SSB - Option B3: adaptation for SSB not on sync raster - FFS: Rel-19 NES-capable UE in idle/inactive mode - Note: Impact to idle/inactive UEs shall be minimized

The following agreement was further reached in RAN1#118 meeting [6].

Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

The following conclusion was made in RAN1#118bis meeting [7].

Conclusion There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19

The PCell of UE1 can be an SCell of UE2, there is no need to differentiate PCell and SCell for the adaptation of SSB transmission in time domain. A unified solution is preferred for both PCell and SCell. Meanwhile, considering the impact to legacy UEs is equally applicable for RRC idle and inactive UEs, the adaptation can only focus on RRC connected UEs.

Proposal 2: For the adaptation of SSB transmissions in time domain:

- The same types of SSB supported for SCell can be supported for PCell.

- Do not support RRC inactive UE.

Regarding the adaptation scheme for the periodicity of SSB transmission, the following agreement was agreed in [5].

Agreement For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB

Since cell DTX was not supported for adaptation of the SSB transmission, it is natural to support a group common DCI based adaptation wherein the applied periodicity of the SSB transmission can be indicated by the group common DCI from a set of one or multiple values configured by RRC. The DCI format can reuse an existing DCI format, and the associated RNTI can be further discussed to be a new RNTI or an existing RNTI.

Proposal 3: For the adaptation of SSB transmissions in time domain, support a group common DCI indicating an applicable periodicity from a set of one or multiple values configured by RRC.

For the SSB transmission before and after adaptation, RAN4 has a LS [8] regarding the time/frequency/spatial domain relationship in need of clarifications from RAN1. In general, as a time domain adaptation scheme, the frequency domain location and spatial domain information for the SSB transmission shall not be impacted due to the adaptation. Moreover, since there is no explicit indication of time domain location of the SSB burst, the UE can determine the SFN offset and half index for the SSB after adaptation based on the SFN offset and half index for the SSB before adaptation.

Proposal 4: For SSB before and after adaptation:

- In time domain, there is no explicit indication on the SFN offset and half frame index to determine the time location of the SSB burst, and the UE can determine the SFN offset and half index for the SSB after adaptation based on the SFN offset and half index for the SSB before adaptation;

- In frequency domain, the frequency location of SSB before and after adaptation maintains the same;

- In spatial domain, the actually transmitted SSB in a burst for SSB before and after adaptation maintains the same.

# Adaptation of PRACH transmissions in time domain

In RAN1#118bis meeting [7], the updated agreement on the PRACH configuration and PRACH adaption was reached as following:

Agreement For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - For Alt 1 (same PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-2a: up to N1 additional value(s) of (x, y) - FFS: value of N1(>=1) - Opt 1-2b: up to N2 additional value(s) of y - FFS: value of N2 (>=1) - Opt 1-3: Muting/masking ROs - Opt 1-4: - up to N4_1 additional timing offset(s) at frame-level - up to N4_2 additional timing offset(s) at slot-level - FFS: values of N4_1(>=1) and N4_2(>=1) - Opt 1-5: No additional mechanism is selected. - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211) - For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-2a: up to N1 additional value(s) of (x, y) - FFS: value of N1(>=1) - Opt 1-2b: up to N2 additional value(s) of y - FFS: value of N2 (>=1) - Opt 1-3: Muting/masking ROs - Opt 1-4: - up to N4_1 additional timing offset(s) at frame-level - up to N4_2 additional timing offset(s) at slot-level - FFS: values of N4_1(>=1) and N4_2(>=1) - Opt 1-5: No additional mechanism is selected. - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

The listed options for each alternative are on additional PRACH resource configuration, then for either alternative, the comparison on the options is given in below:

For Alt.1 in which the PRACH configuration index are the same for NES RACH and legacy RACH resource, which implies the configured time domain pattern are the same for these two. Then option 1-2a, 1-2b, 1-4 are used to adjust the periodicity, location of 10ms RACH frame, or some time domain level offset. It’s just matter of which level of the configuration flexibility that each option wants to give to network.

| Option | Level of flexibility |
| --- | --- |
| 1-2a | RACH configuration periodicity and/or RACH frame location (N1) |
| 1-2b | RACH frame location (N2) |
| 1-4 | RACH frame location (N4_1) and/or slot level offset (N4_2) |

However, any of the above options (1-2a, 1-2b, 1-4) will end up with having a new/additional PRACH configuration index comparing to existing table, which is not the intention of having this alternative at all. Thus, none of the options should be selected for this alternative.

Observation 2: Option 1-2a, 1-2b, 1-4 will equal to creating a new PRACH configuration index.

Then for option 1-3, which is only muting/masking a subset of the configured RO(s) for the NES RACH configuration (with expectation to have different frequency domain configuration). Although the legacy RACH configuration is always there, gNB could choose to mask/mute a certain RO in some time, so that the RACH resource could be denser in some other time. Since in reality, reducing the RF bandwidth could also help gNB to save energy, for alt.1, the option 1-3 could be supported.

Then for Alt. 2, in which a new PRACH configuration index is configured for NES RACH. It has similar issue but even less motivation for option 1-2a, 1-2b, 1-4 in Alt. 2. Given there is already a new configuration index, network is already able to choose a different x and/or y comparing the one configured to the legacy RACH. The effect seems to give more flexibility to such NES configuration, given there is no strong motivation for such new configuration row. These 3 options are less attractive in Alt.2 neither. Then coming to option 1-3, as already clarified during the discussion, it could be used in the case when have some overlap with legacy RO. However, there could be one different way to pursue energy saving, e.g., to mask/mute the non-overlapped RO, to make the RO(s) being more concentrated as well.

Proposal 5: For additional PRACH resources in time-domain:

- For both alternatives (i.e., same or different PRACH configuration index), support option 1-3 (i.e., Muting/masking ROs).

Agreement For DCI-based adaptation for additional PRACH resources, select only from the following alternatives - Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - FFS: details - Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not - FFS: whether the subset of the additional PRACH resources is in - Alt 2-1: RO level per SSB - Alt 2-2: SSB-to-RO mapping cycle level - Alt 2-3: PRACH association period level - Alt 2-4: PRACH association pattern period level - Alt 2-5: SFN level - Alt 3: DCI-based Enhanced/new Cell DRX to indicate whether the enhanced/new Cell DRX is activated or deactivated. - If activated, the additional configured PRACH provided by semi-static signalling within non-active period are not available. - FFS: whether Alt 1 and/or Alt 2 can be applied to the active period - FFS: details

In the above listed Alt 2, there are multiple levels of RO resource to be adapted. For the Alt 2-3/4, which are eventually the integer number of PRACH configuration period, i.e., in the level of 10ms, (which in this sense are also similar to Alt2-5) they can be regarded the same type of adaptation. Only difference is that the PRACH association period could change from time to time, e.g., sometimes it’s 10ms and some other times it becomes 20ms. Such uncertainty motivated the determination of association pattern period in Rel-15, so it did not fit to be the time domain adaptation method. So all these three can be regarded as time domain only based idea. On the other hand, the use of association pattern period has similar effect of having a cell DRX, which can be configured with many flexible time periodicity values. Thus, the Alt 3 have even better coverage of association pattern period. The first two resource levels, i.e., Alt 2-1/2, are not purely time domain concept. The RO level and SSB-RO mapping circle can have some different frequency resource occupation. For SSB-RO mapping circle, it is defined as the RO(s) can be mapped to all SSBs in one SSB period. Considering the SSB mapping is time-frequency dimension and it maps to certain ROs, such level is also considered as RO(s) level. The only difference from the RO level is that Alt 2-2 requires integer rounds of mapping all SSBs in a SSB period, which is an unnecessary limitation and also not convenient for network to design the adaptation pattern. As shown in following figure, the Alt 2-1 can achieve Alt 2-2 and be more flexible. Thus, RO level adaption will be a good choice. As aforementioned, the masking method (e.g., the PRACH mask index or ra-ssb-OccasionMaskIndex) can give very flexible adaptation on the RO pattern to be used.

Figure 1 Illustration of alternatives for PRACH configuration.

Proposal 6: RAN1 supports Alt 2-1 and Alt 3 adaptation mechanism for NES additional PRACH resources.

After getting NES additional RACH resource, the new Rel-19 NES UE can see actually two RACH configuration at least, i.e., the legacy one and the NES adapted one. Then for UE initiated RACH procedure, such two resources can be available for UE to use. Thus, there should be no prioritization between these resources. Following current behaviour, UE will determine the next available RO which comes from either NES RACH configuration or legacy RACH configuration.

Proposal 7: Either NES additional RACH or legacy RACH can be selected by Rel-19 NES UE during one RACH attempt.

Last meeting, there is one important working assumption made as following.

Working Assumption For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode. - P-RNTI is used

Since DCI format 1_0 with P-RNTI is selected to indicate the adaption information for NES configured RACH resource, the following aspect needs to be discussed and decided.

- The time point to apply the adapted RACH resource or the received RACH adaption

Figure 2 Illustration of different time instance for UE intending for RACH.

Given the fact that the NES RACH resource needs to serve the idle UE for initial access, and network has no idea when UE will initiate RACH transmission, thus, as shown in above figure, there are several time instances to be considered. Correspondingly, RAN1 needs to discuss the UE behaviour in different time instances:

  - Case 1: Time instance 1, in which the UE have RRC configuration but got no DCI adaptation yet. In this case, UE needs to know whether the RRC indicated resource can be directly used or it has to wait for the DCI information for adaptation if any. Thus, UE may assume:

    - Option 1-1: the RRC configured NES RACH resource is available.

    - Option 1-2: the RRC configured NES RACH resource is NOT directly available and UE needs to check the PO indicated adaption before actual using it.

  - Case 2: Time instance 2 & 3, in which different UE might get their adaptation DCI in different PO. In this case, different UE actually obtain the adaptation information in different time point, but all the UE should have the common understanding on which RACH resource can be used, or at least there should be no conflict if UE can see different available resources. The UE might:

    - Option 2-1: use its received PO carrying the adaption information as the reference.

For example, assuming ending time of such PO is N, then after a certain time X, the adapted RACH resource can be used. For this option, clearly different UE might have different “N” according to their own received PO. Then gNB needs to ensure the determined available adapted RACH resource by UE(s) are aligned. In addition, even though if the determined available adapted RACH resource is in somewhat “UE specific”, the fairness needs to maintain for different options. Such fairness can be reflected that, for different UE, for their own PO, they can get the resource to use in same/similar time delay. If one design gets the UE in PO 1 has resource in a really quick way, but the UE in PO 4 needs to wait another long time, it won’t be a fair design solution.

    - Option 2-2: use the Page cycle which contains the received PO carrying the adaption information as the reference.

This means, all the UE within the paging cycle will use the same time point of the paging cycle as the reference time to determine available adapted RACH resource. Thus, there is no issue of alignment.

  - Case 3: Time instance 1, 4 & 5, in which different/new UE might initiate RACH procedure and transmission in different time instances. This case is actually more related to the question on how often does the network inform such adaption information. UE might assume:

    - Option 3-1: the adaptation information is transmitted in every paging cycle.

This option could the best/lowest latency for UE to get the adaptation information but certainly, it will consume the largest network energy from network.

    - Option 3-2: the adaptation information is transmitted in a particular paging cycle, e.g., network indicates one paging cycle or pre-define a paging cycle to carry the adaptation.

This option can achieve some balance between indication latency and energy consumption. The cost is some signalling overhead or additional UE determination on the paging cycle for the adaption.

Proposal 8: Apply the adapted RACH resource or the received RACH adaption, RAN1 considers following:

- the RRC configured NES RACH resource is NOT directly available and UE needs to check the PO indicated adaption before actual using it;

- UE uses the Page cycle which contains the received PO carrying the adaption information as the reference;

- the adaptation information is transmitted in a particular paging cycle, e.g., network indicates one paging cycle or pre-define a paging cycle to carry the adaptation.

# Conclusion

The contribution discusses adaptation of common signal/channel transmissions, and the proposals and observations are summarized below.

Adaptation of SSB transmissions in time domain

Observation 1: UE triggered SSB adaptation is beneficial for network energy saving.

Proposal 1: For the adaptation mechanisms of SSB transmissions in time-domain, support adaptation mechanism indicated or configured by gNB with UE trigger.

Proposal 2: For the adaptation of SSB transmissions in time domain:

- The same types of SSB supported for SCell can be supported for PCell.

- Do not support RRC inactive UE.

Proposal 3: For the adaptation of SSB transmissions in time domain, support a group common DCI indicating an applicable periodicity from a set of one or multiple values configured by RRC.

Proposal 4: For SSB before and after adaptation:

- In time domain, there is no explicit indication on the SFN offset and half frame index to determine the time location of the SSB burst, and the UE can determine the SFN offset and half index for the SSB after adaptation based on the SFN offset and half index for the SSB before adaptation;

- In frequency domain, the frequency location of SSB before and after adaptation maintains the same;

- In spatial domain, the actually transmitted SSB in a burst for SSB before and after adaptation maintains the same.

Adaptation of PRACH transmissions in time domain

Observation 2: Option 1-2a, 1-2b, 1-4 will equal to creating a new PRACH configuration index.

Proposal 5: For additional PRACH resources in time-domain:

- For both alternatives (i.e., same or different PRACH configuration index), support option 1-3 (i.e., Muting/masking ROs).

Proposal 6: RAN1 supports Alt 2-1 and Alt 3 adaptation mechanism for NES additional PRACH resources.

Proposal 7: Either NES additional RACH or legacy RACH can be selected by Rel-19 NES UE during one RACH attempt.

Proposal 8: Apply the adapted RACH resource or the received RACH adaption, RAN1 considers following:

- the RRC configured NES RACH resource is NOT directly available and UE needs to check the PO indicated adaption before actual using it;

- UE uses the Page cycle which contains the received PO carrying the adaption information as the reference;

- the adaptation information is transmitted in a particular paging cycle, e.g., network indicates one paging cycle or pre-define a paging cycle to carry the adaptation.

# References

- RP-234065, “New WID: Enhancements of network energy savings for NR”

- RP-242354, “Revised WID: Enhancements of network energy savings for NR”

- RAN1#116 Chair’s notes

- RAN1#116bis Chair’s notes

- RAN1#117 Chair’s notes

- RAN1#118 Chair’s notes

- RAN1#118bis Chair’s notes

- R4-2416913, LS on SSB relation in On-demand SSB and SSB adaptation on SCell, RAN4.

# APPENDIX

## RAN1#116 agreements

Agreement

For adaptation of SSB in time-domain, consider the following adaptation mechanisms for further study

- Adaptation of SSB burst periodicity

- Adaptation based on two SSB configurations where up to two configurations can be active

- Adaptation based on skipping/transmitting some SSB bursts non-uniformly with single SSB configuration

- Adapting the transmitted number of SSBs within a SSB burst

- Cell DTX for SSB adaptation

- Whether to support new SSB burst periodicity value(s)

- Whether to support new SSB burst(s) (i.e. how SSB transmission is made within a burst)

  - New compact SSB burst(s)

  - Adapting the position of SSBs within a SSB burst

- Other mechanisms/combinations are not precluded

Agreement

For adaptation of PRACH in time-domain, consider the following adaptation mechanisms for further study

- Adaptation based on configuration of additional[/different] PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

  - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

- For the additional PRACH resources,

  - Adaptation of PRACH resource periodicity/PRACH occasion

  - Adaptation at PRACH configuration/association period/association pattern period level and SSB to RO mapping cycle

  - Adaptation based on extending cell DRX operation for PRACH

  - Concentrating ROs in time domain

- Other options are not precluded

Agreement

For adaptation of paging,

- Study further from RAN1 perspective, techniques for adaptation of paging occasions in time-domain and achievable network energy savings

- Note: Specification details for PO/PF determination and paging-related configuration/procedures to be handled by RAN2

Agreement

For the adaptation mechanisms of SSB in time-domain, study further applicable scenarios and associated legacy UE impact/handling (if any) based on the following:

- Applicability to UE in idle/inactive and/or connected mode

- Applicability to PCell and/or SCell(s)

Agreement

For the adaptation mechanisms of SSB in time-domain, study further following mechanisms:

- Adaptation mechanism indicated or configured by gNB without UE trigger

- Adaptation triggered by UE (if any)

FFS: Details of associated signaling/indication/configuration.

Agreement

For the adaptation mechanisms of PRACH in time-domain

- Support at least PRACH adaptation provided by gNB without UE trigger

  - FFS: PRACH adaptation with UE trigger

  - Note: UE trigger means UE requests adaptation of PRACH

- Study at least the following,

  - Dynamic signaling and/or semi-static signaling of PRACH adaptation

  - Adaptation of PRACH transmission according to certain condition

  - Applicability to idle/inactive and/or connected mode UEs

  - Which scenarios the adaptation mechanism is applicable to (e.g. cell with both legacy and Rel-19 UE, cell with only Rel-19 UEs)

## RAN1#116-bis agreements

Agreement

For indication of adaptation of SSB in time-domain,

- Support at least SSB adaptation provided by gNB without UE trigger

Agreement

For adaptation of PRACH in time-domain, support at least the following:

- Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

  - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

  - Configuration of additional PRACH resources is provided by semi-static signalling

    - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs

  - FFS: adaptation mechanism for additional PRACH resources

  - Note: No change to the existing PRACH configuration tables in 38.211

Agreement

For adaptation of PRACH in time-domain, support the following:

- SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs (if any)

  - FFS: whether/how to handle SSB-RO mapping if the additional PRACH resources overlap in both time and frequency with the PRACH resources for legacy UEs

  - Note: SSB-RO mapping of the PRACH resources for legacy UEs is not impacted if Rel-19 UE uses these PRACH resources

  - FFS: SSB-RO mapping for the additional PRACH resources

Agreement

Support adaptation mechanisms of PRACH in time-domain for following:

- UE in idle/inactive mode

- UE in connected mode

Agreement

Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s):

- For cell with both legacy UEs and Rel-19 NES-capable UEs

  - Rel-19 NES-capable UE’s PCell (Connected mode)

    - Study from the following options:

      - Option A1: adaptation for CD-SSB

      - Option A2: adaptation for SSB that is not CD-SSB

      - Option A3: adaptation for SSB not on sync raster

  - Rel-19 NES-capable UE’s SCell

    - Study from the following options:

      - Option B1: adaptation for CD-SSB

      - Option B2: adaptation for SSB that is not CD-SSB

      - Option B3: adaptation for SSB not on sync raster

  - FFS: Rel-19 NES-capable UE in idle/inactive mode

- Note: Impact to idle/inactive UEs shall be minimized

Agreement

For adaptation of PRACH in spatial domain,

- Study possibility of scenarios with non-uniform distribution of UEs in different beams

    - Note 6: Companies are encouraged to provide details on how they map UEs to different beams

- Study network energy savings gain achieved by non-uniform PRACH resource allocation across SSBs for scenarios with non-uniform distribution of UEs in different beams (if any),

    - Assume the following framework for network energy evaluation in FR1 and companies to report at least the below settings used in the evaluation/simulation

      - 20ms SSB period

      - 30kHz SCS, DDDSU TDD pattern

      - Setting A: SIB1 period (20ms/40ms/160ms)

      - Setting B1: Cell load (Empty/low/medium)

      - Setting B2: Traffic model

      - Setting C: SIB1 PDSCH time domain resource index in 38.214 Table 5.1.2.1.1-2

      - Setting D: CORESET0/SSB multiplexing pattern including controlResourceSetZero (index) in 38.213 Table 13-6, and searchSpaceZero (index) in 38.213 Table 13-11

      - Setting E1: PRACH configurations

        - (legacy) PRACH resources according to the following PRACH configuration for all transmitted SSBs

      - Case A1-1: PRACH configuration #5 (20ms)

      - Case A1-2: PRACH configuration #17 (10ms)

      - Case A2-1: PRACH configuration #0 (160ms)

        - (time-domain PRACH adaptation) Additional and legacy PRACH resources yielding total PRACH resources that are according to one of the following PRACH configuration for all transmitted SSBs

      - Case B1: PRACH configuration #17 (10ms)

      - Case B2: PRACH configuration #0 (160ms)

      - Companies to report details of assumed time domain adaptation mechanism

        - (spatial-domain PRACH adaptation) Additional and legacy PRACH resources yielding total PRACH resources that are according to one of the following PRACH configuration

      - Case C1: PRACH configuration #17 (10ms)

      - Case C2: PRACH configuration #0 (160ms)

      - Companies to report details of assumed spatial domain adaptation mechanism, including details of non-uniform PRACH resource allocation across SSBs

      - Setting F: Cat 1/Cat 2 BS as defined in TR38.864

      - Setting G1: Number of SSB beams: 4,8 SSBs in a SSB burst with SSB pattern case C

      - Note 1: Baseline to compare is Case C1 vs Case B1/A1-1/A1-2, Case C2 vs Case B2/A2-1

      - Note 2: It is up to company to report the SSB-RO mapping ratio and FDMed RO number, etc

      - Note 3: Other PRACH configuration index with different PRACH format other than format 0 is not precluded

      - Note 4: Other SSB/SIB1/RACH periodicity/PRACH resource/configuration assumptions are not precluded (up to companies to report)

    - Other frameworks for network energy evaluation are not precluded, e.g. including for FR2

## RAN1#117 agreements

Agreement

For the study of adaptation of PRACH in spatial domain, following network energy savings gains were reported by sources based on the evaluation framework agreed in RAN1#116bis:

- Two sources showed following NES gain for TDD, CAT1 BS power model, case C1 vs A1-1, zero load [R1-2404409, R1-2405107]

  - -4% ~ -45%

- Seven sources showed following NES gain for TDD, CAT1 BS power model, case C1 vs B1/A1-2, zero load [R1-2404225, R1-2404185, R1-2404334, R1-2404123, R1-2404562, R1-2405107, R1-2405163]

  - 0% ~ 31%

  - Note: Five sources assumed that case B1 has same PRACH resources as case A1-2. Remaining two sources evaluated only A1-2.

  - Note: Three sources showed NES gains 0% ~ 10% [R1-2404225, R1-2404185, R1-2404334]

- One source showed following NES gain for TDD, CAT1 BS power model, case C1 vs B1, zero load [R1-2404464]

  - 1.0%~8.8%

  - Note: The evaluation results provide the extra NES gain of spatial domain PRACH adaptation compared to time domain PRACH adaptation, where spatial domain and time domain PRACH adaptations are based on dynamic switching between PRACH resources according to two PRACH configuration indexes.

- One source showed following NES gain for TDD, CAT1 BS power model, case C1 vs B1, zero load [R1-2404626]

  - -48.41%~0%

  - Note: For B1, it was assumed that periodicity of PRACH resources can be adapted. For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- One source showed following NES gain for TDD, CAT1 BS power model, for case C1 vs A1-2, zero load [R1-2404626]

  - 4.59%~38.04%

  - Note: For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- Four sources showed following NES gain for TDD, CAT2 BS power model, case C1 vs B1/A1-2, zero load [R1-2404562, R1-2404225, R1-2403943, R1-2404626]

  - 0% ~ 3.5%

  - Note: Three sources assumed that case B1 has same PRACH resources as case A1-2. One source evaluated only A1-2.

- One source showed following NES gain for TDD, CAT2 BS power model, case C1 vs B1, zero load [R1-2404464]

  - 0%~0.2%

  - Note: The evaluation results provide the extra NES gain of spatial domain PRACH adaptation compared to time domain PRACH adaptation, where spatial domain and time domain PRACH adaptations are based on dynamic switching between PRACH resources according to two PRACH configuration indexes

- One source showed following NES gain for TDD, CAT2 BS power model, case C1 vs B1, zero load [R1-2404626]

  - -1.19%~0%

  - Note: For B1, it was assumed that periodicity of PRACH resources can be adapted. For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- Two sources showed following NES gain for TDD, CAT1 or CAT2 BS power model, case C2 vs B2, zero load [R1-2403943, R1-2405107]

  - Less than 0.2%

- One source showed following NES gain for TDD, CAT1 BS power model, (C1 vs A1-2 with changed PRACH format), PRACH format A, 10ms PRACH periodicity, different loads [R1-2403980]

  - 13.7%/8.7%/4.9%/2.6% for zero/low/light/medium cell load

- One source showed following NES gain for TDD, CAT1 BS power model, (C1 vs B1 with changed PRACH format), PRACH format A, 10ms PRACH periodicity, different loads [R1-2403980]

  - 8.03%/5.1%/3.06%/1.74% for zero/low/light/medium cell load

- One source showed following NES gain for TDD, C1 vs B1/A1-2, different loads [R1-2404562]

  - 16%/4.78% for light/medium cell load for CAT1 BS power model

  - 0.65%/0.29% for light/medium cell load for CAT2 BS power model

- One source showed following NES gain for TDD, C1 vs B1, different loads [R1-2404626]

  - -18.57%~0%/-2.52%~0% for low /medium cell load for CAT1 BS power model

  - -0.81%~0%/-0.42%~0% for low /medium cell load for CAT2 BS power model

  - Note: For B1, it was assumed that periodicity of PRACH resources can be adapted. For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- One source showed following NES gain for TDD, C1 vs A1-2, different loads [R1-2404626]

  - 3.67%~19.88%/2.29%~5.22% for low /medium cell load for CAT1 BS power model

  - 0.67%~1.75%/0.39%~0.91% for low /medium cell load for CAT2 BS power model

  - Note: For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- One source showed NES gain for FDD, C1 vs B1, zero load [R1-2404464]

  - 1.4%~7% for CAT1 BS power model

  - 0%~0.3% for CAT2 BS power model

  - Note: The evaluation results provide the extra NES gain of spatial domain PRACH adaptation compared to time domain PRACH adaptation, where spatial domain and time domain PRACH adaptations are based on dynamic switching between PRACH resources according to two PRACH configuration indexes

- One source showed NES gain for FR2, CAT1 BS power model, spatial domain adaptation of PRACH configuration index 75 vs a time domain adaptation of PRACH configuration index 75, zero load [R1-2405163]

  - 4%~7%

- Note 1: About possibility of scenarios with non-uniform distribution of UEs in different beams

  - Several companies indicated (and three companies showed data/analysis) that there can be scenarios with non-uniform distribution of UEs in different beams.

  - Several companies mentioned that for non-uniform UE distribution, it can be addressed by gNB implementation e.g. by adjusting SSB beamwidth, etc. Several companies also mentioned that it is not clear how gNB can predict the distribution of UEs in different beams, especially for Idle/Inactive UEs.

- Note 2: Most sources that showed the NES gains (if any) for adaptation of PRACH in spatial domain compared to A1-2/B1 observed that the gain would be due to reduction in the number of overall ROs in time domain in their evaluations. Most of these companies only accounted for ROs in time domain.

- Note 3: The evaluation results assumed the non-uniform distribution of UE is static during the evaluation time period.

Conclusion

There is no consensus in RAN1 on the support of PRACH adaptation in spatial domain

Agreement

For adaptation of PRACH in time-domain, support at least the following case(s)

- Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

- Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

- Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains

- FFS: whether additional conditions are needed to support the above cases

- FFS: Additional case whether full/partial overlap in both time and frequency is allowed

- Above does not preclude discussion for the case where the configuration for additional PRACH resources contains legacy PRACH resources

Agreement

At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule.

- Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes.

  - Note: This mapping is not impacted by time domain PRACH adaptation

- Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs.

Agreement

For adaptation of SSB in time-domain, Option 1 is supported

- Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s)

- Note: Using Option 2 to realize Option 1 is not precluded

  - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active]

    - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities

- FFS: Details including applicable scenarios

- FFS: Support of Cell DTX for connected mode UEs for SSB

Agreement

For adaptation of PRACH in time-domain, the additional PRACH resources are configured based on at least:

- a PRACH configuration index

- FFS: whether the PRACH configuration index is same and/or different from the PRACH configuration index for the legacy PRACH resources

Study further the following

- When the PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resource,

  - Additional parameter(s) for determining the additional PRACH resources e.g.

    - Scaled/adjusted PRACH configuration period

    - Additional timing offset

    - Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration

    - Muting/masking ROs

- When the PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resource

  - Additional mechanisms (if any) for determining the additional PRACH resources e.g.

    - Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources)

- Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed)

Agreement

For the adaptation mechanism for additional PRACH resources, study further the following:

- Option 1: Higher layer signalling (with potential enhancements) based PRACH resource adaptation

- Option 2: L1-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

  - Strive to re-use existing DCI format(s)

- Option 3: Adaptation of PRACH transmission according to predefined condition(s)

  - FFS: details

- Option 4-rev1: L1-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: whether the subset of the additional PRACH resources is in RO level / SSB-to-RO mapping cycle level/PRACH association period level/PRACH association pattern period level for time-domain PRACH adaptation

  - Strive to re-use existing DCI format(s)

- Option 5: Enhanced cell DRX

## RAN1#118 agreement

Agreement

- For adaptation of PRACH in time-domain, select at least one from the following alternatives for configuration of the additional PRACH resources

- Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources and

  - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g.

    - Opt 1-1: Scaled/adjusted PRACH configuration period

    - Opt 1-2: Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration

    - Opt 1-3: Muting/masking ROs

    - Opt 1-4: additional timing offset(s)

- Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources,

  - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g.

    - Opt 2-1: Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources)

    - Opt 2-2: Additional timing offset(s)

- FFS: Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed)

- FFS: Additional frequency domain parameter(s) (e.g., freq. starting offset)

Agreement

- Extend the RAN1#117 agreement on SSB-RO mapping rule for additional PRACH resources to Case 1

- Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

RAN1#117 Agreement

At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule.

- Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes.

  - Note: This mapping is not impacted by time domain PRACH adaptation

- Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs.

RAN1#117 Agreement

At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule.

- Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes.

  - Note: This mapping is not impacted by time domain PRACH adaptation

- Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs.

Agreement

- For SSB-RO mapping rule for additional PRACH resources for Case 2.

- Extend the RAN1#117 and RAN1#118 agreements on SSB-RO mapping

Agreement

For the adaptation mechanism for additional PRACH resources (for CONNECTED mode UE and IDLE/INACTIVE mode UE),

- At least DCI based adaptation is supported. No introduction of new DCI format.

R1-2407278	Summary#2 of AI 9.5.3 for R19 NES	Moderator (Ericsson)

Agreement

For adaptation mechanism(s) of SSB in time-domain,

- For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported

  - FFS: Adaptation for SSB that is not CD-SSB is supported (A2)

  - FFS: Adaptation for SSB not on sync raster is supported (A3)

- For Rel-19 NES-capable UE’s SCell

  - Adaptation of SSB configured for the SCell is supported for the following cases

    - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell

    - Adaptation for SSB that is not CD-SSB on sync raster (B2’)

    - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

Agreement

For DCI-based adaptation for additional PRACH resources,

- Select from the following DCI format(s) to carry the adaptation indication.

  - DCI format 1_0

  - DCI format 2_7

  - DCI format 2_9

- FFS: existing (P-RNTI, SI-RNTI, CellDTRX-RNTI, PEI-RNTI, C-RNTI) or new RNTI used for detecting the DCI format

Agreement

For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select from following options

- Option 1: One SSB burst periodicity is configured for the UE and UEs assumes SSB transmissions are not present during Cell DTX non-active period

- Option 2: UE assumes SSB transmission with different periodicities during Cell DTX non-active period and during Cell DTX active period

- Option 3: Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior) – no spec impact

Agreement

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives

- Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

- Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: whether the subset of the additional PRACH resources is in

    - Alt 2-1: RO level per SSB

    - Alt 2-2: SSB-to-RO mapping cycle level

    - Alt 2-3: PRACH association period level

    - Alt 2-4: PRACH association pattern period level

    - Alt 2-5: SFN level

- Alt 3: DCI-based Enhanced/new Cell DRX to indicate whether the enhanced/new Cell DRX is activated or deactivated.

  - If activated, the additional configured PRACH provided by semi-static signalling within non-active period are not available.

  - FFS: whether Alt 1 and/or Alt 2 can be applied to the active period

  - FFS: details

## RAN1#118-bis agreement

Agreement

- For adaptation of PRACH in time-domain, the same PRACH preamble format is used for the additional RACH resources and legacy PRACH resources.

- Agreement

For adaptation of PRACH in time-domain, support both of the following

- Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources

- Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources

- FFS: Additional details

Working Assumption

For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode.

- P-RNTI is used

Agreement

- For adaptation of PRACH in time-domain, the frequency domain resources for the additional PRACH resources and legacy PRACH resources can be same or different

- FFS: applicable case(s) (i.e. case(s) from the RAN1#117 agreement).

- Discuss further following options for signaling

- Option 1: at least the following parameter(s) can be configured separately for the additional PRACH resources.

  - msg1-FrequencyStart at least for 4-step RACH

  - FFS: other applicable legacy frequency domain parameter(s)

- Option 2: Offset to legacy frequency domain parameter(s) are configured for the additional PRACH resources

  - Note: Offset to legacy frequency domain parameter(s) is a new parameter

  - FFS: applicable legacy frequency domain parameter(s)

Conclusion

There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19

Agreement

For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select Option 3, i.e. Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior).

- No spec impact

Agreement

For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain,

- For Alt 1 (same PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - FFS: value of N1(>=1)

  - Opt 1-2b: up to N2 additional value(s) of y

    - FFS: value of N2 (>=1)

  - Opt 1-3: Muting/masking ROs

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - FFS: values of N4_1(>=1) and N4_2(>=1)

  - Opt 1-5: No additional mechanism is selected.

  - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

- For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - FFS: value of N1(>=1)

  - Opt 1-2b: up to N2 additional value(s) of y

    - FFS: value of N2 (>=1)

  - Opt 1-3: Muting/masking ROs

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - FFS: values of N4_1(>=1) and N4_2(>=1)

  - Opt 1-5: No additional mechanism is selected.

  - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)