3GPP TSG RAN WG1 #119		                                    R1-2409620

Orlando, US, November 18th – 22nd, 2024

Agenda item:		9.14.2

Source:			Samsung

Title:			UE features for NR MIMO Phase 5

Document for:		Discussion and decision

- Introduction

This contribution provides Samsung’s view regarding UE features for supporting Rel-19 MIMO functionality [1]. During Rel-19 MIMO discussion, RAN1 agreed on remarkable enhancements possibly requesting more complicated UE operation. Some of features are already agreed to bring new UE capabilities. Also, there are RAN1 agreements put FFS or working assumption on introduction of new UE capabilities, and some of the agreed or discussed UE parameters are aligned on their existence or configuration. In this contribution, we discuss all the Rel-19 MIMO related UE capabilities together, and propose whether and how to define new UE capabilities.

- Discussion

  - UE-initiated/event-driven beam management (UEI/ED BM)

In the following, we provide a list of UE capabilities that need to be supported for Rel-19 UEI/ED BM. In particular, our views on the UE capabilities are based on at least the following agreements/working assumption made in the agenda item 9.2.1 of Rel-19 MIMO.

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported: - Mode A (dynamically scheduling UCI by gNB): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report - FFS: Request format, e.g., SR or a new UCI type. - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. - Step 3: Beam report is transmitted in second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature). - No new DCI format is introduced. - Mode B (UCI in pre-configured resource(s) for second UL channel): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report - FFS: Notification format, e.g., SR or a new UCI type. - Step 2: UE transmits the beam report in the second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - The notification in Step1 is in a separate reporting instance from the beam report in Step 2. FFS: Whether UE receives acknowledge information with response to each step for all modes For above procedures, cross-CC beam reporting is supported for both modes. - FFS: Details. Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification. Agreement On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, at least Option-3 is supported - Option-3: N ≥ 1 beam(s) are reported in the report instance, - At least one of N reported beam(s) should satisfy the condition of Event-2 - N is configured by gNB - FFS: candidate value of ‘N’. - FFS: RRC can enable or disable whether current beam is always reported in addition to the N beams - FFS: Option-1/1a/1b/2. - Above applies at least for the single CC case Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, support the both schemes as follows. - Scheme-1: RS for current beam is the QCL RS in the indicated TCI state - FFS: Whether/How to handle the case if only one TRS is configured in the indicated TCI state. - Scheme-2: the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state. - Enabling one of either Scheme-1 or Scheme-2 is selected by NW. - FFS: The above selection is via an explicit RRC parameter or an implicit manner, e.g., if the RS(s) for new beam are CSI-RS, Scheme-1 is enabled; otherwise, Scheme-2 is enabled. - (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam. - The above QCL RS is the RS w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated TCI state. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-B, UEI beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the last symbol of report notification on the first PUCCH channel. Agreement On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the candidate value of ‘N’ at least comprises {1, 2, 3, 4} - FFS: additional candidate value(s) of {5, 6, 7, 8} - FFS: If ‘N’ is not RRC configured, only one L1-RSRP and CRI/SSBRI are reported by default. Agreement Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set. - FFS: The RS in the RS resource set can be updated by MAC-CE Agreement Regarding the triggering event determination for Event 2, the event instance(s) counting is per new beam. Further study candidate condition(s) of resetting the counting including whether resetting is needed. Working Assumption The following working assumption in RAN1#118 is revised in red. On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th Q-th best quality. - M Q is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2.

Proposal 1. Support the following list of UE capabilities for Rel-19 UEI/ED BM.

| Index | Feature group | Components | Prerequisite FGs | Consequence if the feature is not supported by the UE | Type | Need of FR1/FR2 differentiation | Note | Mandatory /Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X-1-1 | UEI/ED beam reporting | 1. Support of event-2 2. Support of determination of one event-2 instance within a time window 3. Supported candidate value(s) of the threshold for determining an event-2 instance 4. Support of determining event-2 based on determination of one event-2 instance 5. Support of Mode A 6. Supported candidate value(s) of reported beam(s) N in a reporting instance 7. Supported maximum number of candidate beam RS resources in a candidate beam RS resource set 8. Support of Scheme-1 and/or Scheme-2 for determining current beam RS 9. Supported candidate value(s) of time window duration | 2-24 | UEI/ED beam reporting is not supported | Per band | N/A | Component 2: one event-2 instance is determined when a candidate beam becomes a threshold value better than the current beam within a time window Component 3: candidate value(s) of the threshold is in dB range Component 6: candidate value(s): {2, 3, 4} Component 7: candidate value(s): {1, 2, …, 128} Component 8: {Scheme-1, Scheme-2, both} Component 9: candidate value(s) is in ms | Optional with capability signalling |
| X-1-2 | UEI/ED beam reporting Mode B | 1. Support of event-2. 2. Support of determination of one event-2 instance within a time window 3. Supported candidate value(s) of the threshold for determining an event-2 instance 4. Support of determining event-2 based on determination of one event instance 5. Supported candidate value(s) of reported beam(s) N in a reporting instance 6. Supported candidate value(s) of reported beam(s) in a reporting instance that satisfies the event-2 condition(s) 7. Supported maximum number of candidate beam RS resources in a candidate beam RS resource set 8. Supported candidate value(s) of X 9. Support of Scheme-1 and/or Scheme-2 for determining current beam RS 10. Supported candidate value(s) of time window duration | X-1-1 | UEI/ED beam reporting Mode B is not supported | Per band | N/A | Component 2: one event-2 instance is determined when a candidate beam becomes a threshold value better than the current beam within a time window Component 3: candidate value(s) of the threshold is in dB range Component 5: candidate value(s): {2, 3, 4} Component 6: candidate value(s): {1, 2, 3, 4} Component 7: candidate value(s): {1, 2, …, 128} Component 8: candidate value(s): {1, 2, …} symbols Note: UEI beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the last symbol of report notification on the first PUCCH channel Component 9: {Scheme-1, Scheme-2, both} Component 10: candidate value(s) is in ms | Optional with capability signalling |
| X-1-3 | Determination of event-2 based on more than one event-2 instances | 1. Support of determining event-2 based on determination of M>1 event-2 instances within a time window 2. Supported candidate value(s) of M 3. Support of per candidate beam event instance(s) counting | X-1-1 | UEI/ED beam reporting based on determination of more than one event-2 instances is not supported | Per band | N/A | Component 2: candidate value(s): { 2, 3, …} | Optional with capability signalling |
| X-1-4 | Cross-CC beam measurement and report | 1. Support of cross-CC beam measurement and report for Mode A and/or Mode B | X-1-1 | Cross-CC beam measurement and report for UEI/ED beam reporting is not supported | Per band | N/A |  | Optional with capability signalling |
| X-1-5 (subject to confirming the WA) | Additional events to event-2 | 1. Support of event-1 and event-7 2. Supported candidate value(s) of Q (i.e., the activated TCI state with the Q-th best quality) | X-1-1 | Additional events to event-2 for UEI/ED beam reporting are not supported | Per band | N/A | Component 1: candidate value(s): {event-1, event-7, both} Note: event-1 is when quality of the current beam is worse than a certain threshold. Note: event-7 is when quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. Component 2: candidate value(s): {1, 2, 3, 4} | Optional with capability signalling |

  - CSI enhancement

Type-I/II

Regarding Rel-19 Type-I single-panel (SP) codebook refinement for 48, 64, and 128 CSI-RS ports, two schemes were agreed as below:

Agreement For the Rel-19 Type-I single-panel (SP) codebook refinement for 48, 64, and 128 CSI-RS ports, for RI=1-4, support the following: - Scheme-A (based on Scheme1 in RAN1#116 agreement): Adding new (N1, N2) values for the Rel-15 Type-I single-panel codebook mode-1 (L=1) where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and for rank-3/4, follow legacy mechanisms for <16 ports - Scheme-B (based on Scheme2 in RAN1#116 agreement): Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and - W1 structure: - For each layer, reuse legacy Rel-16 eType-II SD basis with L=1 to determine the DFT-based SD basis candidates - For 1<RI≤4, L=1 SD basis vector is independently selected for different layers - The SD basis selection indication includes layer-common (q1,q2) and log2N1N2 bits for each layer - Note: This implies that each of the SD basis vectors is selected from a group of N1N2 orthogonal basis vectors - W2 structure: Layer-specific inter-polarization co-phasing with the alphabet {+1, +j, -1, -j} FFS (RAN1#116bis): For Rel-19 Type-I SP, whether to support Mode-C based on Scheme5 in RAN1#116 agreement with L=1 for RI=2-4 FFS (RAN1#116bis): For Rel-19 Type-I SP, whether inter-polarization amplitude for Mode-B can also be supported FFS: Discuss further if Rel-19 Type-I MP extension based on scheme 4 is needed

Although Scheme-A seems to simply follow legacy design, scaling up Rel-15 Type-I single-panel codebook with new (N1, N2) values would introduce significant implementation complexity at UE in addition to small performance gain. Scheme-B, on the other hand, can be implemented effortlessly reusing most of existing components of Rel-16 eType-II at UE, and it is also validated the UPT gain is significant over Scheme-A. Therefore, due to less implementation complexity, more flexibility of Scheme-B, and performance benefit, we propose Scheme-B as the basic feature.

Proposal 2: We propose Scheme-B as the basic feature for Enhanced Type-I SP codebook:

| Index | Feature group | Components | Pre-requisite | Reporting granularity | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| X-2-1 | Basic Features of Enhanced Type-I SP codebook | 1) Scheme-B 2) A list of supported combinations, up to 16, across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource b) Maximum total number of NZP CSI-RS resources aggregated c) Maximum total number of Tx ports of aggregated NZP CSI-RS resources 3) Support of rank 1,2,3,4 | 2-35 |  | Component 2 candidate values: 2) a) {16, 32} b) {[1], 2, 3, 4} c) {64} | Optional with capability signalling |
| X-2-2 | Total number of Tx ports of aggregated NZP CSI-RS resources is 48 for Type-I single-panel | 1) support of maximum total number of Tx ports of aggregated NZP CSI-RS resources is 48 |  |  |  |  |
| X-2-3 | Total number of Tx ports of aggregated NZP CSI-RS resources is 128 for Type-I single-panel | 1) support of maximum total number of Tx ports of aggregated NZP CSI-RS resources larger is 128 |  |  |  |  |

CRI-based CSI

For CRI- based up to 128 ports, based on the following agreements, some functionalities are subject to UE capability

Agreement For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, the supported combinations of KS value and the maximum number of ports per NZP CSI-RS resource are as follows: - FFS: UE capability on KS and the number of ports per resource KS; Maximum # ports per resource. 2, 3, 4; 32. 5, 6, 7, 8; 16. Agreement For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, - For Rel-15 Type-I Single Panel codebook, M is NW-configured via higher-layer (RRC) signaling with candidate value(s) of {1, …, min(4,KS)} - The maximum value of M is subject to UE capability - For Rel-16 eType-II, M=1 is supported - The maximum value of KS is {1,2,3,4} and subject to UE capability - The support for Rel-16 eType-II is a separate UE capability at least from the support for Rel-19 Type-I and Type-II codebook refinements - FFS (RAN1#116bis): The support for M=2, and if so, the value of M={1, 2} is NW-configured via higher-layer (RRC) signaling, and if additional restriction(s) are needed FFS: The determination of M reported beams Note: Selection algorithm of CRI(s) from measurement of KS>1 NZP-CSI-RS resources is up to UE implementation. Agreement For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, - When M>1, the M PMIs are independently calculated and indicated - with the Rel-16 eType-II codebook and KS={1,2,3,4}, support M=2 with a maximum of 16 ports per resource, R=1 only, and a maximum UCI payload of 1706 bits. - The value of M={1, 2} is NW-configured via higher-layer (RRC) signalling - The maximum value of M is subject to UE capability - on the configured KS>1 NZP CSI-RS resources, reuse the legacy IMR rule for the Rel-15 CRI-based reporting for NZP CSI-RS resource for interference measurement, i.e. only 1 NZP CSI-RS resource for interference measurement can be configured Agreement For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, for A-CSI only, the NW can configure MR (<M) of KS CSI-RS resources to be selected as part of reporting the M “quadruplets”: - (M–MR) CRIs, each with log2KS-MR bits are reported, along with the M sets of CQI/PMI/RI/(if applicable) LI - The value of MR is NW-configured via higher-layer (RRC) signaling - The MR selected resources are NW-configured via higher-layer (RRC) signaling - FFS: value of MR - This is an optional UE capability

Proposal 3: The following list of UE capabilities can be a starting point of further discussion

| Index | Feature group | Components | Prerequisite FGs | Consequence if the feature is not supported by the UE | Type | Need of FR1/FR2 differentiation | Note | Mandatory /Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X-2-4 | CRI based upto 128 ports with Rel-15 Type-I SP codebook | - Supported number of CRIs | 2-36 | Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook is not supported | Per Band | N/A | Component 4 candidate value: {1, 2, 3, 4} | Optional with capability signalling |
| X-2-5 | CRI based upto 128 ports with Rel-16 eType-II codebook | - Supported number of resources(Ks) - Supported number of CRIs (M) | 16-3a | Hybrid BF (CRI-based) with Rel-16 eType-II codebook is not supported | Per Band | N/A | Component 1 candidate value: {1, 2, 3, 4} Component 2 candidate value: {1, 2} | Optional with capability signalling |
| X-2-6 | Configuration of MR High priority resources | - Supported number of High priority Resources per CSI report with Rel-15 type-I SP codebook - Supported number of High priority Resources per CSI report with Rel-16 eType-II codebook | X-2-4, X-2-5 | Configuration of MR High priority resources is not supported | Per Band | N/A | Component 1 candidate value: {1, 2} Component 1 candidate value: {1} | Optional with capability signalling |

CJT calibration reporting

Based on agreement so far in Rel. 19 for CJT calibration reporting, we propose below new feature groups:

Proposal 4: The following list of UE capabilities can be a starting point of further discussion

| Index | Feature group | Components | Pre-requisite | Reporting granularity | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| X-2-7 | Basic Features of CJT calibration reporting | 1. support of CJT calibration reporting 2. supported report type 3. supported reporting mode 4. supported number of reported offsets 5. cjtc-P is reported wideband 6. value of X for CPU occupation (OCPU= X NTRP) |  |  | component 2 candidate values: {cjtc-Dd, cjtc-F, cjtc-P, cjtc-Dd-F} component 3 candidate values: {TRP selection, offset value, both} component 4 candidate values: {2,3,4} component 6 candidate values: {1,2} | Optional with capability signalling |
| X-2-8 | Joint delay and frequency offset CJT calibration reporting | 1. support of joint delay and frequency offset CJT calibration reporting in one report 2. value of X for CPU occupation (OCPU= X NTRP) |  |  | component 2 candidate values: {1,2} | Optional with capability signalling |
| X-2-9 | Maximum number of TRS resource sets in a report configuration | 1. supported maximum number of TRS resource sets in a report configuration |  |  | component 1 candidate values: {2,3,4} | Optional with capability signalling |
| X-2-10 | Sub-band phase offset CJT calibration reporting | 1. support of sub-band phase offset reporting 2. supported number of supported sub-bands in PRBs |  |  | component 2 candidate values: {1, 2, 4, 8, 16} | Optional with capability signalling |
| X-2-11 | Linking CJTC Dd and Rel-18 eType-II CJT CSI reports | 1. support of linking CJTC Dd and Rel-18 eType-II CJT CSI reports 2.supported triggering mode 3. value of X for CPU occupation (OCPU= X NTRP) |  |  | component 2 candidate values: {separate, joint, both} component 2 candidate values: {1,2} | Optional with capability signalling |

  - UL 3TX

For Rel-19 UL 3TX, there are some agreements as follows for certain functionalities by subject to UE capability.

Agreement For codebook-based uplink transmission by a 3TX UE, support full-power Mode 0, subject to UE capability. Agreement For codebook-based UL transmission by a 3TX UE, subject to its capability, - A 3TX UE may report a maximum number of 3 layers - A 3TX UE may report a maximum number of SRS ports of up to 3 Note: SRS resource definition is not changed nor the number of SRS ports in the SRS resource. Agreement To support non-codebook-based PUSCH transmission by a 3TX UE, - UE reports a capability of up to a maximum of 3 layers. Agreement For SRS configuration supporting codebook-based UL transmission by a 3TX UE, support Alt1, - Alt1: Support configuration of X 4-port SRS resources in a resource set where one the ports is muted - FFS muting mechanism where X can be up to 2, subject to UE capability. Agreement For codebook-based transmission by a 3TX UE, - Only PUSCH antenna ports 1000, 1001, 1002 are used - Option- 2: Subject to UE capability, up to 2 PTRS ports may be configured in PTRS-UplinkConfig, - FFS whether a single bit or 2 bits are used for PTRS-DMRS association indication. Above is only for single panel transmission.

Considering above agreements and some basic features which have been agreed in normative phase and/or Rel-19 MIMO work item description without explicit description on UE capability, the following list of UE capabilities can be a starting point of further discussion.

Proposal 5. Support the following list of UE capabilities for Rel-19 UL 3TX.

| Index | Feature group | Components | Prerequisite FGs | Consequence if the feature is not supported by the UE | Type | Need of FR1/FR2 differentiation | Note | Mandatory /Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X-3-1 | Codebook based 3TX PUSCH transmission | 1. Support non-coherent codebook subset only. 2. Support 4-port SRS resource with muting port 1003 3. Supported codebook based PUSCH MIMO with maximal number of supported layers 4. Supported maximum number of SRS resource per set (SRS set use is configured as for codebook). | 2-14 | Codebook based 3TX PUSCH transmission is not supported | Per FS | N/A | Component 3 candidate value: {no-codebook based MIMO, 1, 2, 3} Component 4 candidate value: {1, 2} | Optional with capability signalling |
| X-3-2 | Non-codebook based 3TX PUSCH transmission | 1. Maximal number of supported layers (non-codebook transmission scheme) 2. Supported max number of SRS resource per set (SRS set use is configured as for non-codebook transmission). 3. Maximum number of simultaneous transmitted SRS resources at one symbol | 2-15 | Non-codebook based 3TX PUSCH transmission is not supported | Per FSPC | N/A | Component 1 candidate value: {1, 2, 3} Component 2 candidate value: {1, 2, 3, 4} Component 3 candidate value: {1, 2, 3, 4} | Optional with capability signalling |
| X-3-1-1 | Multi-TRP PUSCH repetition type A – codebook based 3TX PUSCH transmission | Support of multi-TRP PUSCH repetition type A for codebook based 3TX PUSCH transmission | X-3-1, 23-3-1 | Multi-TRP PUSCH repetition type A is not supported for codebook based 3TX PUSCH transmission. | Per FS | N/A |  | Optional with capability signalling |
| X-3-1-2 | Multi-TRP PUSCH repetition type B – codebook based 3TX PUSCH transmission | Support of multi-TRP PUSCH repetition type B for codebook based 3TX PUSCH transmission | X-3-1, 23-3-1-1 | Multi-TRP PUSCH repetition type B is not supported for codebook based 3TX PUSCH transmission. | Per FSPC | N/A |  | Optional with capability signalling |
| X-3-2-1 | Multi-TRP PUSCH repetition type A – non-codebook based 3TX PUSCH transmission | Support of multi-TRP PUSCH repetition type A for non-codebook based 3TX PUSCH transmission | X-3-2, 23-3-1-2 | Multi-TRP PUSCH repetition type A is not supported for non-codebook based 3TX PUSCH transmission. | Per FS | N/A |  | Optional with capability signalling |
| X-3-2-2 | Multi-TRP PUSCH repetition type B – non-codebook based 3TX PUSCH transmission | Support of multi-TRP PUSCH repetition type B for non-codebook based 3TX PUSCH transmission | X-3-2, 23-3-1-3 | Multi-TRP PUSCH repetition type B is not supported for non-codebook based 3TX PUSCH transmission. | Per FSPC | N/A |  | Optional with capability signalling |
| X-3-3 | Two PT-RS ports for 3TX PUSCH | Support two PT-RS ports for 3TX PUSCH | X-3-1 or X-3-2 | Two PT-RS ports for 3TX PUSCH are not supported | Per Band | N/A |  | Optional with capability signalling |
| X-3-4 | Full-power mode 0 for codebook based 3TX PUSCH | Support full-power mode 0 for codebook based 3TX PUSCH transmission | X-3-1 | Full-power mode 0 for codebook based 3TX PUSCH transmission is not supported | Per FS | N/A |  | Optional with capability signalling |
| X-3-5 | SRS 3T6R antenna switching | 1. Support of 3T6R for antenna switching by using 4-port SRS resource with muting port 1003 2. Downgrade antenna switching configurations 3. Report the entry number of the first-listed band with UL in the band combination that affects this DL 4. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL |  | SRS 3T6R antenna switching is not supported | Per FS | N/A | Component 2 candidate value: combination (including empty) of {1T1R, 1T2R, 1T4R, 2T2R, 2T4R, 2T6R} Component 3 candidate value: {1,2,…,32} Component 4 candidate value: {1,2,…,32} | Optional with capability signalling |
| X-3-6 | SRS 3T3R antenna switching | Support of SRS 3T3R antenna switching |  | SRS 3T3R antenna switching is not supported | Per FS | N/A |  | Optional with capability signalling |
| X-3-7 | Maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for 3T6R antenna switching | X-3-5 | Maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS is supported | Per FS | N/A | Note: The two SP-SRS resource sets are not activated at the same time | Optional with capability signalling |

  - Asymmetric MTRP

For Rel-19 asymmetric MTRP, based on the following agreements for certain functionalities by subject to UE capability, we can start to define UE capability signaling structure.

Agreement Support applying PL offset on PDCCH-order PRACH towards a UL TRP in FR1. - Note: The DL reference timing determination for PDCCH-order PRACH transmission to an UL TRP is still based on the DL RS defined in current RAN4 specification - Above is subject to a separate UE capability signaling Agreement Support DCI format 1_1 to indicate TPC command for SRS CLPC adjustment state(s) separate from PUSCH: - (Working Assumption) Introduce a 2-bit TPC command field to indicate TPC command for SRS associated with separate SRS CLPC adjustment state where: - The 2-bit TPC command field is present if UE reports supporting a dedicated UE capability, and a corresponding RRC parameter is configured (which is a new RRC to enable this). - (Working Assumption) Introduce a 1-bit SRS close-loop indicator to indicate one of the two separate SRS CLPC adjustment states for the TPC command - The 1-bit SRS close-loop indicator is present if UE reports supporting another dedicated UE capability and a corresponding RRC parameter is configured (which is a new RRC to enable this) and two separate SRS CLPC adjustment states are configured. Agreement About the extended value range 1~X of starting bit of blocks in DCI format 2_3 in Rel-19, support Alt1: - Alt1: X = 45 (to be captured in RAN2 spec) - This feature is a separate UE capability and is appliable to any Rel-19 UE who supports this UE capability, regardless this UE supports two separate SRS CLPC adjustment states or not. - Note: X=45 can be used for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise Agreement Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

Considering above agreements and some basic features which have been agreed in normative phase and/or Rel-19 MIMO work item description without explicit description on UE capability, the following list of UE capabilities can be a starting point of further discussion.

Proposal 6. Support the following list of UE capabilities for Rel-19 asymmetric MTRP.

| Index | Feature group | Components | Prerequisite FGs | Consequence if the feature is not supported by the UE | Type | Need of FR1/FR2 differentiation | Note | Mandatory /Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X-4-1 | Pathloss offset with joint DL/UL TCI(s) | Support Pathloss offset for joint DL/UL TCI(s) | 23-1-1 and/or 40-1-1 | Pathloss offset for joint DL/UL TCI(s) is not supported | Per band | FR1 only |  | Optional with capability signalling |
| X-4-2 | Pathloss offset for separate UL TCI(s) | Support Pathloss offset for separate UL TCI(s) | 23-10-1 and/or 40-1-2 | Pathloss offset for separate UL TCI(s) is not supported | Per band | N/A |  | Optional with capability signalling |
| X-4-3 | Two SRS CLPC adjustment states separate from PUSCH | Support up to two SRS CLPC adjustment states separate from PUSCH |  | Two SRS CLPC adjustment states separate from PUSCH is not supported | Per band | N/A |  | Optional with capability signalling |
| X-4-4 | Two TA enhancements for asymmetric MTRP | Support two TA without restriction of multi-DCI based multi-TRP operation in asymmetric MTRP |  | Two TA enhancements for asymmetric MTRP is not supported | Per band | N/A |  | Optional with capability signalling |
| X-4-5 | Applying PL offset on PDCCH-order triggered PRACH towards a UL TRP | Support applying PL offset on PDCCH-order triggered PRACH towards a UL TRP | X-4-1 and/or X-4-2 | Applying PL offset on PDCCH-order triggered PRACH towards a UL TRP is not supported | Per band | FR1 only |  | Optional with capability signalling |
| X-4-6 | Extended value range of starting bit of blocks in DCI format 2_3 | Support of extended value range of starting bit of blocks in DCI format 2_3 from 1~31 to 1~45 |  | Extended value range of starting bit of blocks in DCI format 2_3 is not supported | Per band | N/A | Note: Extended value range of 1~45 can be used for operations in FR1 in shared spectrum or FR2-2 and 1~43 otherwise | Optional with capability signalling |
| X-4-7 | SRS TPC command field for SRS CLPC adjustment state(s) separate from PUSCH in DCI format 1_1 | Support SRS TPC command field for SRS CLPC adjustment state(s) separate from PUSCH in DCI format 1_1 |  | SRS TPC command field for SRS CLPC adjustment state separate from PUSCH in DCI format 1_1 is not supported | Per band | N/A |  | Optional with capability signalling |
| X-4-8 | SRS closed-loop indicator field for one of two separate SRS CLPC adjustment states in DCI format 1_1 | Support SRS closed-loop indicator field for one of two separate SRS CLPC adjustment states in DCI format 1_1 | X-4-3 | SRS closed-loop indicator field for one of two separate SRS CLPC adjustment states in DCI format 1_1 is not supported | Per band | N/A |  | Optional with capability signalling |
| X-4-9 | Overlapping UL transmission reduction for asymmetric MTRP | Support of reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions when the UE is not configured with UL STx2P for asymmetric MTRP operation with two TA enhancement. | X-4-4 | Reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions is not supported. | Per band | N/A | Note: If UE does not support this feature, UE does not expect the two UL transmissions associated with different TAGs to overlap (i.e., scheduling restriction is applied to avoid overlap between the two UL transmissions) | Optional with capability signalling |

- Conclusion

In this contribution, the following observation and proposals are given:

Proposal 1. Support the following list of UE capabilities for Rel-19 UEI/ED BM.

| Index | Feature group | Components | Prerequisite FGs | Consequence if the feature is not supported by the UE | Type | Need of FR1/FR2 differentiation | Note | Mandatory /Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X-1-1 | UEI/ED beam reporting | 1. Support of event-2 2. Support of determination of one event-2 instance within a time window 3. Supported candidate value(s) of the threshold for determining an event-2 instance 4. Support of determining event-2 based on determination of one event-2 instance 5. Support of Mode A 6. Supported candidate value(s) of reported beam(s) N in a reporting instance 7. Supported maximum number of candidate beam RS resources in a candidate beam RS resource set 8. Support of Scheme-1 and/or Scheme-2 for determining current beam RS 9. Supported candidate value(s) of time window duration | 2-24 | UEI/ED beam reporting is not supported | Per band | N/A | Component 2: one event-2 instance is determined when a candidate beam becomes a threshold value better than the current beam within a time window Component 3: candidate value(s) of the threshold is in dB range Component 6: candidate value(s): {2, 3, 4} Component 7: candidate value(s): {1, 2, …, 128} Component 8: {Scheme-1, Scheme-2, both} Component 9: candidate value(s) is in ms | Optional with capability signalling |
| X-1-2 | UEI/ED beam reporting Mode B | 1. Support of event-2. 2. Support of determination of one event-2 instance within a time window 3. Supported candidate value(s) of the threshold for determining an event-2 instance 4. Support of determining event-2 based on determination of one event instance 5. Supported candidate value(s) of reported beam(s) N in a reporting instance 6. Supported candidate value(s) of reported beam(s) in a reporting instance that satisfies the event-2 condition(s) 7. Supported maximum number of candidate beam RS resources in a candidate beam RS resource set 8. Supported candidate value(s) of X 9. Support of Scheme-1 and/or Scheme-2 for determining current beam RS 10. Supported candidate value(s) of time window duration | X-1-1 | UEI/ED beam reporting Mode B is not supported | Per band | N/A | Component 2: one event-2 instance is determined when a candidate beam becomes a threshold value better than the current beam within a time window Component 3: candidate value(s) of the threshold is in dB range Component 5: candidate value(s): {2, 3, 4} Component 6: candidate value(s): {1, 2, 3, 4} Component 7: candidate value(s): {1, 2, …, 128} Component 8: candidate value(s): {1, 2, …} symbols Note: UEI beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the last symbol of report notification on the first PUCCH channel Component 9: {Scheme-1, Scheme-2, both} Component 10: candidate value(s) is in ms | Optional with capability signalling |
| X-1-3 | Determination of event-2 based on more than one event-2 instances | 1. Support of determining event-2 based on determination of M>1 event-2 instances within a time window 2. Supported candidate value(s) of M 3. Support of per candidate beam event instance(s) counting | X-1-1 | UEI/ED beam reporting based on determination of more than one event-2 instances is not supported | Per band | N/A | Component 2: candidate value(s): { 2, 3, …} | Optional with capability signalling |
| X-1-4 | Cross-CC beam measurement and report | 1. Support of cross-CC beam measurement and report for Mode A and/or Mode B | X-1-1 | Cross-CC beam measurement and report for UEI/ED beam reporting is not supported | Per band | N/A |  | Optional with capability signalling |
| X-1-5 (subject to confirming the WA) | Additional events to event-2 | 1. Support of event-1 and event-7 2. Supported candidate value(s) of Q (i.e., the activated TCI state with the Q-th best quality) | X-1-1 | Additional events to event-2 for UEI/ED beam reporting are not supported | Per band | N/A | Component 1: candidate value(s): {event-1, event-7, both} Note: event-1 is when quality of the current beam is worse than a certain threshold. Note: event-7 is when quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. Component 2: candidate value(s): {1, 2, 3, 4} | Optional with capability signalling |

Proposal 2: We propose Scheme-B as the basic feature for Enhanced Type-I SP codebook:

| Index | Feature group | Components | Pre-requisite | Reporting granularity | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| X-2-1 | Basic Features of Enhanced Type-I SP codebook | 1) Scheme-B 2) A list of supported combinations, up to 16, across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource b) Maximum total number of NZP CSI-RS resources aggregated c) Maximum total number of Tx ports of aggregated NZP CSI-RS resources 3) Support of rank 1,2,3,4 | 2-35 |  | Component 2 candidate values: 2) a) {16, 32} b) {[1], 2, 3, 4} c) {64} | Optional with capability signalling |
| X-2-2 | Total number of Tx ports of aggregated NZP CSI-RS resources is 48 for Type-I single-panel | 1) support of maximum total number of Tx ports of aggregated NZP CSI-RS resources is 48 |  |  |  |  |
| X-2-3 | Total number of Tx ports of aggregated NZP CSI-RS resources is 128 for Type-I single-panel | 1) support of maximum total number of Tx ports of aggregated NZP CSI-RS resources larger is 128 |  |  |  |  |

Proposal 3: The following list of UE capabilities can be a starting point of further discussion

| Index | Feature group | Components | Prerequisite FGs | Consequence if the feature is not supported by the UE | Type | Need of FR1/FR2 differentiation | Note | Mandatory /Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X-2-4 | CRI based upto 128 ports with Rel-15 Type-I SP codebook | - Supported number of CRIs | 2-36 | Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook is not supported | Per Band | N/A | Component 4 candidate value: {1, 2, 3, 4} | Optional with capability signalling |
| X-2-5 | CRI based upto 128 ports with Rel-16 eType-II codebook | - Supported number of resources(Ks) - Supported number of CRIs (M) | 16-3a | Hybrid BF (CRI-based) with Rel-16 eType-II codebook is not supported | Per Band | N/A | Component 1 candidate value: {1, 2, 3, 4} Component 2 candidate value: {1, 2} | Optional with capability signalling |
| X-2-6 | Configuration of MR High priority resources | - Supported number of High priority Resources per CSI report with Rel-15 type-I SP codebook - Supported number of High priority Resources per CSI report with Rel-16 eType-II codebook | X-2-4, X-2-5 | Configuration of MR High priority resources is not supported | Per Band | N/A | Component 1 candidate value: {1, 2} Component 1 candidate value: {1} | Optional with capability signalling |

Proposal 4: The following list of UE capabilities can be a starting point of further discussion

| Index | Feature group | Components | Pre-requisite | Reporting granularity | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| X-2-7 | Basic Features of CJT calibration reporting | 1. support of CJT calibration reporting 2. supported report type 3. supported reporting mode 4. supported number of reported offsets 5. cjtc-P is reported wideband 6. value of X for CPU occupation (OCPU= X NTRP) |  |  | component 2 candidate values: {cjtc-Dd, cjtc-F, cjtc-P, cjtc-Dd-F} component 3 candidate values: {TRP selection, offset value, both} component 4 candidate values: {2,3,4} component 6 candidate values: {1,2} | Optional with capability signalling |
| X-2-8 | Joint delay and frequency offset CJT calibration reporting | 1. support of joint delay and frequency offset CJT calibration reporting in one report 2. value of X for CPU occupation (OCPU= X NTRP) |  |  | component 2 candidate values: {1,2} | Optional with capability signalling |
| X-2-9 | Maximum number of TRS resource sets in a report configuration | 1. supported maximum number of TRS resource sets in a report configuration |  |  | component 1 candidate values: {2,3,4} | Optional with capability signalling |
| X-2-10 | Sub-band phase offset CJT calibration reporting | 1. support of sub-band phase offset reporting 2. supported number of supported sub-bands in PRBs |  |  | component 2 candidate values: {1, 2, 4, 8, 16} | Optional with capability signalling |
| X-2-11 | Linking CJTC Dd and Rel-18 eType-II CJT CSI reports | 1. support of linking CJTC Dd and Rel-18 eType-II CJT CSI reports 2.supported triggering mode 3. value of X for CPU occupation (OCPU= X NTRP) |  |  | component 2 candidate values: {separate, joint, both} component 2 candidate values: {1,2} | Optional with capability signalling |

Proposal 5. Support the following list of UE capabilities for Rel-19 UL 3TX.

| Index | Feature group | Components | Prerequisite FGs | Consequence if the feature is not supported by the UE | Type | Need of FR1/FR2 differentiation | Note | Mandatory /Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X-3-1 | Codebook based 3TX PUSCH transmission | 1. Support non-coherent codebook subset only. 2. Support 4-port SRS resource with muting port 1003 3. Supported codebook based PUSCH MIMO with maximal number of supported layers 4. Supported maximum number of SRS resource per set (SRS set use is configured as for codebook). | 2-14 | Codebook based 3TX PUSCH transmission is not supported | Per FS | N/A | Component 3 candidate value: {no-codebook based MIMO, 1, 2, 3} Component 4 candidate value: {1, 2} | Optional with capability signalling |
| X-3-2 | Non-codebook based 3TX PUSCH transmission | 1. Maximal number of supported layers (non-codebook transmission scheme) 2. Supported max number of SRS resource per set (SRS set use is configured as for non-codebook transmission). 3. Maximum number of simultaneous transmitted SRS resources at one symbol | 2-15 | Non-codebook based 3TX PUSCH transmission is not supported | Per FSPC | N/A | Component 1 candidate value: {1, 2, 3} Component 2 candidate value: {1, 2, 3, 4} Component 3 candidate value: {1, 2, 3, 4} | Optional with capability signalling |
| X-3-1-1 | Multi-TRP PUSCH repetition type A – codebook based 3TX PUSCH transmission | Support of multi-TRP PUSCH repetition type A for codebook based 3TX PUSCH transmission | X-3-1, 23-3-1 | Multi-TRP PUSCH repetition type A is not supported for codebook based 3TX PUSCH transmission. | Per FS | N/A |  | Optional with capability signalling |
| X-3-1-2 | Multi-TRP PUSCH repetition type B – codebook based 3TX PUSCH transmission | Support of multi-TRP PUSCH repetition type B for codebook based 3TX PUSCH transmission | X-3-1, 23-3-1-1 | Multi-TRP PUSCH repetition type B is not supported for codebook based 3TX PUSCH transmission. | Per FSPC | N/A |  | Optional with capability signalling |
| X-3-2-1 | Multi-TRP PUSCH repetition type A – non-codebook based 3TX PUSCH transmission | Support of multi-TRP PUSCH repetition type A for non-codebook based 3TX PUSCH transmission | X-3-2, 23-3-1-2 | Multi-TRP PUSCH repetition type A is not supported for non-codebook based 3TX PUSCH transmission. | Per FS | N/A |  | Optional with capability signalling |
| X-3-2-2 | Multi-TRP PUSCH repetition type B – non-codebook based 3TX PUSCH transmission | Support of multi-TRP PUSCH repetition type B for non-codebook based 3TX PUSCH transmission | X-3-2, 23-3-1-3 | Multi-TRP PUSCH repetition type B is not supported for non-codebook based 3TX PUSCH transmission. | Per FSPC | N/A |  | Optional with capability signalling |
| X-3-3 | Two PT-RS ports for 3TX PUSCH | Support two PT-RS ports for 3TX PUSCH | X-3-1 or X-3-2 | Two PT-RS ports for 3TX PUSCH are not supported | Per Band | N/A |  | Optional with capability signalling |
| X-3-4 | Full-power mode 0 for codebook based 3TX PUSCH | Support full-power mode 0 for codebook based 3TX PUSCH transmission | X-3-1 | Full-power mode 0 for codebook based 3TX PUSCH transmission is not supported | Per FS | N/A |  | Optional with capability signalling |
| X-3-5 | SRS 3T6R antenna switching | 1. Support of 3T6R for antenna switching by using 4-port SRS resource with muting port 1003 2. Downgrade antenna switching configurations 3. Report the entry number of the first-listed band with UL in the band combination that affects this DL 4. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL |  | SRS 3T6R antenna switching is not supported | Per FS | N/A | Component 2 candidate value: combination (including empty) of {1T1R, 1T2R, 1T4R, 2T2R, 2T4R, 2T6R} Component 3 candidate value: {1,2,…,32} Component 4 candidate value: {1,2,…,32} | Optional with capability signalling |
| X-3-6 | SRS 3T3R antenna switching | Support of SRS 3T3R antenna switching |  | SRS 3T3R antenna switching is not supported | Per FS | N/A |  | Optional with capability signalling |
| X-3-7 | Maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for 3T6R antenna switching | X-3-5 | Maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS is supported | Per FS | N/A | Note: The two SP-SRS resource sets are not activated at the same time | Optional with capability signalling |

Proposal 6. Support the following list of UE capabilities for Rel-19 asymmetric MTRP.

| Index | Feature group | Components | Prerequisite FGs | Consequence if the feature is not supported by the UE | Type | Need of FR1/FR2 differentiation | Note | Mandatory /Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X-4-1 | Pathloss offset with joint DL/UL TCI(s) | Support Pathloss offset for joint DL/UL TCI(s) | 23-1-1 and/or 40-1-1 | Pathloss offset for joint DL/UL TCI(s) is not supported | Per band | FR1 only |  | Optional with capability signalling |
| X-4-2 | Pathloss offset for separate UL TCI(s) | Support Pathloss offset for separate UL TCI(s) | 23-10-1 and/or 40-1-2 | Pathloss offset for separate UL TCI(s) is not supported | Per band | N/A |  | Optional with capability signalling |
| X-4-3 | Two SRS CLPC adjustment states separate from PUSCH | Support up to two SRS CLPC adjustment states separate from PUSCH |  | Two SRS CLPC adjustment states separate from PUSCH is not supported | Per band | N/A |  | Optional with capability signalling |
| X-4-4 | Two TA enhancements for asymmetric MTRP | Support two TA without restriction of multi-DCI based multi-TRP operation in asymmetric MTRP |  | Two TA enhancements for asymmetric MTRP is not supported | Per band | N/A |  | Optional with capability signalling |
| X-4-5 | Applying PL offset on PDCCH-order triggered PRACH towards a UL TRP | Support applying PL offset on PDCCH-order triggered PRACH towards a UL TRP | X-4-1 and/or X-4-2 | Applying PL offset on PDCCH-order triggered PRACH towards a UL TRP is not supported | Per band | FR1 only |  | Optional with capability signalling |
| X-4-6 | Extended value range of starting bit of blocks in DCI format 2_3 | Support of extended value range of starting bit of blocks in DCI format 2_3 from 1~31 to 1~45 |  | Extended value range of starting bit of blocks in DCI format 2_3 is not supported | Per band | N/A | Note: Extended value range of 1~45 can be used for operations in FR1 in shared spectrum or FR2-2 and 1~43 otherwise | Optional with capability signalling |
| X-4-7 | SRS TPC command field for SRS CLPC adjustment state(s) separate from PUSCH in DCI format 1_1 | Support SRS TPC command field for SRS CLPC adjustment state(s) separate from PUSCH in DCI format 1_1 |  | SRS TPC command field for SRS CLPC adjustment state separate from PUSCH in DCI format 1_1 is not supported | Per band | N/A |  | Optional with capability signalling |
| X-4-8 | SRS closed-loop indicator field for one of two separate SRS CLPC adjustment states in DCI format 1_1 | Support SRS closed-loop indicator field for one of two separate SRS CLPC adjustment states in DCI format 1_1 | X-4-3 | SRS closed-loop indicator field for one of two separate SRS CLPC adjustment states in DCI format 1_1 is not supported | Per band | N/A |  | Optional with capability signalling |
| X-4-9 | Overlapping UL transmission reduction for asymmetric MTRP | Support of reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions when the UE is not configured with UL STx2P for asymmetric MTRP operation with two TA enhancement. | X-4-4 | Reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions is not supported. | Per band | N/A | Note: If UE does not support this feature, UE does not expect the two UL transmissions associated with different TAGs to overlap (i.e., scheduling restriction is applied to avoid overlap between the two UL transmissions) | Optional with capability signalling |

References

- RP-242394, WID revision: NR MIMO Phase 5, RAN#105.