3GPP TSG RAN WG1 #119			R1-2409663

Orlando, US, November 18th – 22nd, 2024

Title:	[Draft] Reply LS on Additional Measurements for SL-TDOA and SL-TOA

Response to:	LS on Additional Measurements for SL-TDOA and SL-TOA (R2-2409256)

Release:	Rel-18

Work Item:	NR_pos_enh2-Core

Source:	vivo

To:	RAN2

Cc:	RAN4

Contact person:

Name:

E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall description

RAN1 thanks RAN2 LS on additional Measurements for SL-TDOA and SL-TOA.

RAN1 would like to provide the response to the questions as follows.

Question 1: Is RAN2 requested to add signalling to enable a SL-TDOA/SL-TOA capable UE to report additional measurements (up to 3 additional SL-TDOA/SL-TOA measurements according to UE capabilities)? Question 2: If the answer to Question 1 is ‘yes’, what should the additional SL-TDOA/SL-TOA measurements comprise? Do the additional SL-TDOA/SL-TOA measurements include the SL-RSTD/SL-RTOA measurements only (as FGs 41-1-7a and FG 41-1-7b seems to imply), or can the additional SL-TDOA/SL-TOA measurements also include the SL-PRS RSRP/RSRPP measurements together with other currently reported information such as SL-PRS Resource ID (sl-PRS-ResourceId-r18), LOS/NLOS indicator (los-NLOS-Indicator-r18), additional paths measurements (sl-TDOA-AdditionalPathList-r18/sl-TOA-AdditionalPathList-r18), timing quality (sl-TimingQuality-r18), and/or time stamp (sl-TimeStamp-r18)? Question 3: If additional SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurement reporting is required, should the encoding of the additional SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurements be defined relative to the 1st/main SL-RSTD/SL-RTOA measurement (or 1st/main SL-PRS RSRP/RSRPP measurement)? I.e., analogous to LPP additional measurement reporting where the e.g., additional RSTD measurements are defined relative to the 1st/main RSTD measurement (e.g., nr-RSTD-ResultDiff in LPP). If so, what would be the value range of the additional SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurements relative to the 1st/main SL-RSTD/SL-RTOA (and probably SL-PRS RSRP/RSRPP) measurement?

For the above question, RAN1 would like to answer one by one.

For Q1, RAN1 has different views on whether to add signalling to enable a SL-TDOA/SL-TOA capable UE to report additional measurements (up to 3 additional SL-TDOA/SL-TOA measurements according to UE capabilities). So RAN1 added the note for the FGs 41-1-7a and FG 41-1-7b and up to RAN2 to decide whether to add signalling to enable a SL-TDOA/SL-TOA capable UE to report additional measurements (up to 3 additional SL-TDOA/SL-TOA measurements according to UE capabilities)

For Q2: If RAN2 decides to add the signalling, it can include the additional SL-TDOA/SL-TOA measurements, and can also include the SL-PRS RSRP/RSRPP measurements together with other currently reported information such as SL-PRS Resource ID (sl-PRS-ResourceId-r18),  LOS/NLOS indicator (los-NLOS-Indicator-r18), additional paths measurements (sl-TDOA-AdditionalPathList-r18/sl-TOA-AdditionalPathList-r18), timing quality (sl-TimingQuality-r18), and/or time stamp (sl-TimeStamp-r18)

For Q3: If RAN2 decides to add the signalling, for the SL-TDOA, the differential RSRP (e.g., DL-PRS-RSRP-ResultDiff), differential RSTD (e.g., nr-RSTD-ResultDiff in LPP) relative to the 1st/main SL-RSTD can be directly reused. In addition, the value range can be the same as the value range of additional measurements in LPP. However, differential reporting may not be supported for SL-TOA since differential reporting of additional measurement is not supported in NRPPa.

- Actions

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above response into consideration

- Date of next TSG-RAN WG1 meeting

TSG RAN WG1 Meeting #120		           17th February – 21st February	         	 Athens, Greece

TSG RAN WG1 Meeting #120-bis                                 7th April– 11th April                                 China