3GPP TSG RAN WG1 #119                                                                                             R1-2409698

Orlando, US, November 18th – 22nd, 2024

Source:	vivo

Title:	Discussions on NR-NTN downlink coverage enhancement

Agenda Item:	9.11.1

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the downlink coverage enhancement for NR NTN, including the analysis of the impacts of the SSB periodicity extension, wide/narrow beam footprint, and link level enhancements.

- Analysis of the impacts of SSB periodicity extension

The following observation were captured for the system level evaluation in RAN1#117 ~ #118bis:

Observation Based on the results of DL coverage ratio evaluation at system level collected from 7 sources for all the three LEO600km satellite parameter sets where the beam footprint diameter is 50 km: - For Set 1-1/1-3, the coverage ratio can be improved from 10% to 100% if the SSB periodicity is increased from 20ms to 80ms and beam hopping is applied - For Set 1-2, the coverage ratio can be improved from 1.5% to 96.8% if the SSB periodicity is increased from 20ms to 320ms and beam hopping is applied. - Note: coverage ratio is N2+N3/ total beam footprints - Note: the baseline assumes no beam hopping. TDM between SIB1 and SIB19 is assumed in those results, following current specs. Based on the results of DL coverage ratio evaluation at system level collected from 3 sources for a deployment scenario implementing wide beam footprint: - 1 source reports that with a deployment of wide beam covering 4 narrow (of 50km size) beams, which means Set 1-2 FR1 with additional EIRP reduction of 6dB, using SSB periodicity of 80 ms can provide coverage ratio of 96.8%, and Set 1-1/1-3 FR1 with additional EIRP reduction of 6dB, SSB periodicity of 80 ms can provide coverage of 100%. - 1 source observed that for Set 1-1, 1-2 and 1-3, the coverage ratio can be improved from 1.5% to 100% using the legacy default SSB periodicity of 20ms during initial access, by choosing a wide beam footprint with beam footprint sizes of 84 km and 56 km respectively. - Note: the PDCCH and the PDSCH for SIB19 is assumed to be transmitted within 2 OFDM symbols and 5 MHz bandwidth. the PDSCH for SIB1 is assumed to be transmitted within 3 OFDM symbols and 5 MHz bandwidth. This assumes no SIB1 and SIB19 transmission in N2 beam footprints. This assumes non-aligned SFN timing across different beams. - 1 source observed, for Set 1-1 with increased beam size, that the legacy SSB periodicity of 20ms during initial access is usable with NTN beam hopping, by choosing a deployment scenario implementing wide beam footprint with beam footprint sizes of 70.7 km and 86.6 km, leading to a total of 529 and 353 beam footprints within the satellite coverage area, respectively, and the coverage ratio is 80% and 90%, respectively, and a ratio of simultaneously active beam footprints to the total number of beam foot prints equal to 20% and 30%. - Note: Beam footprint size is increased by increasing only the adjacent beam spacing without increasing the 3dB beamwidth. Note: RAN1 will further investigate the impact of SSB periodicity extension Note: Any needed clarification “SSB channel enhancement is not considered” in the WID is up to RAN plenary Note: RAN1 will further investigate the impact of wider beam of SSB and/or other channels on performance (e.g. link budget, capacity...) Observation Based on the results of DL coverage evaluation at system level collected from different sources, it is observed that extending the default value of SSB periodicity (different from 20ms) in NTN with LEO600km satellite parameter sets where the beam footprint diameter is 50 km, is beneficial in terms of reduction of common control channel overhead, when targeting a full coverage of 1058 beam footprints: - With Set 1-1 FR1 and Set 1-3 FR1, the common messages (SSB, SIB1) overhead is around 40% assuming 5 MHz BW when SSB/SIB1 periodicity of 20ms is in use, this overhead ratio could be reduced to less than 14% when 160ms SSB/SIB1 periodicity is used. - With Set 1-2 FR1, the common message (SSB, SIB1) overhead is greater than 100% assuming 5 MHz BW when SSB/SIB1 periodicity of 20ms is in use, this overhead could be reduced to around 25.8% when 640ms SSB/SIB1 periodicity is used. - Note: the overhead of SIB19 was included in some of the results - Note: an observation when SSB/SIB1 periodicity is 320 ms will be discussed and added to the observation Agreement As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - Default value[s] with extended periodicity assumed by NTN UE for initial access can be: - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms} Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms

RAN #104 has made the following amendments to the objective of the downlink coverage enhancements in the WID:

- SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation

In this section, we analyze the impacts of SSB periodicity extension, including the UE overheads regarding the complexity, latency and success rate of initial access and the potential spec impacts.

2.1 UE’s overheads of initial access

During initial access, UE has no prior information about the time- and frequency-domain locations of the SSB. Thus, for any single candidate frequency on the sync raster, the UE has to sample the received signal continuously and to buffer the samples for whole duration of TTI of PBCH, which is 80ms and accounts for potential at most 4-shot transmissions of SSB. As the Figure 1 shows, the extension of SSB periodicity proportionally increases the duration for which the UE needs to keep listening and buffering. After that, the UE tries to blindly detect the SSB among the buffered samples, of which the computations also increase proportionally to the SSB periodicity. Consequently, it costs proportionally higher latency and energy consumption for the UE to acquire the SSB for initial cell selection and synchronization. This argument also holds for the memory requirements of the modem to buffer the samples. Table 1 gives a preliminary estimate of the overheads that the UE has to pay for the SSB periodicity extended to 320ms when compared with the overheads in the case of the SSB periodicity of 20ms.

Figure 1. Example of how the UE acquires the SSB during initial access

Table 1. A preliminary estimate of UE-side overheads with SSB periodicity extended to 320ms

| Items | Sources | Overheads (320ms) |
| --- | --- | --- |
| Latency/ Energy consumption | Signal sampling and buffering | ↑16 times |
|  | Blind detection | ↑16 times |
| Memory requirement | Signal buffering | ↑16 times |

Moreover, considering a number of frequencies on the sync raster, the UE has to sweep all of the frequencies in batches if the memory is not large enough to accommodate the samples for all of the frequencies on the sync raster. In this case, the latency and the energy consumption are increased by additionally Nsync/Nbatch times, where Nsync is the number of frequencies on the sync raster and Nbatch is the number of frequencies in a single batch. Alternatively, the UE can be equipped with a 16-times-higher memory to achieve a comparable latency as in the case of legacy SSB periodicity. Since Nsync depends on the density of the sync raster, a higher sync raster density leads to higher latency, energy consumption and the memory requirements.

Observation 1: If the SSB periodicity is extended to 320ms, the UE has to pay for 16-times higher latency, complexity, and power consumption for initial access, compared with the 20ms case.

Apart from initial access, SSB is an important reference signal used for many purposes, including inter-cell identification, cell reselection, handover, RLM, and BFD. The SSB with longer periodicity introduces higher latency for the UE to perform RRM, and makes the UE unable to timely respond to the events such as radio link failure and beam failure.

Observation 2: The extension of SSB periodicity also induces higher latency regarding RRM, RLM, BFD, etc.

Another aspect is the success rate of initial cell selection. Here we mainly focus on the success rate of the SSB reception. Figure 2 depict the CDF of the received SINR in a cell served by the satellite with parameter sets 1-1/1-2/1-3 FR1 respectively. The simulation parameters are shown in Appendix C. Considering that the SNR requirements of the 4-shot SSB reception is -11.26 dB, we can obtain the success possibilities of the 4-shot SSB reception accordingly for the three parameter sets, namely, 12.7% for set 1-3 FR1, and 100% for both sets 1-1 and 1-2 FR1. Since multi-shot SSB reception is possible, the SNR requirements can be relaxed by 3dB and 6dB respectively for the 2-shot and 1-shot SSB reception. We summarize the success possibilities of the 1/2/4-shot SSB reception of parameter Sets 1-1/1-2 and Set 1-3 FR1 in Table 2.

According to the analysis in Section 2.2, at most 2-shot or 4-shot SSB reception can be supported when SSB periodicity is 40ms or 20ms respectively. When SSB periodicity is larger or equal to 80ms, only one-shot SSB reception is available. Based on Table 2, we fix a given time window of 320ms and compare the success rates under different values of SSB periodicity. In principle, multiple SSB receptions occur within the time window, for example, 1-shot SSB receptions for 4 times when SSB periodicity is 80ms. A successful of initial cell selection is achieved if at least one of the SSB reception is successful. Following this idea, we can give the results in Table 3.

Observation 3: For parameter set 1-3 FR1, successful initial cell selection can not be achieved if the SSB periodicity is 320ms.

Observation 4: For parameter set 1-1 FR1, the success rates of initial cell selection are reduced from 68.7% to 44%, if the SSB periodicity is extended from 160ms to 320ms.

Figure 2. CDF of the received SINR in a cell for parameter sets 1-1/1-2/1-3 FR1

Table 2. The success probabilities of parameter sets 1-1/1-2/1-3 under 1/2/4-shot SSB reception

| Parameter Sets | 1-shot | 2-shot | 4-shot |
| --- | --- | --- | --- |
| Set 1-1 FR1 | 25.2% | 99.8% | 100% |
| Set 1-2 FR1 | 100% | 100% | 100% |
| Set 1-3 FR1 | 0% | 0% | 12.7% |

Table 3. The success rates of initial cell selection under different values of SSB periodicity

| Parameter Sets | 160ms | 320ms |
| --- | --- | --- |
| Set 1-1 FR1 | 68.7% | 44% |
| Set 1-2 FR1 | 100% | 100% |
| Set 1-3 FR1 | 0% | 0% |

2.2 Backward compatibility

To enable the SSB periodicity extension, significant changes are needed to be made on the spec, which inevitably results in a critical backward compatibility issue. Particularly, the impacts can be elaborated from the following several aspects:

- The procedures of PBCH generation and reception

- SMTC configurations

- RRM measurements

The first aspect is related to the generation and reception of PBCH. In the current TS 38.212, the PBCH is generated by assuming the TTI of 80ms and the periodicity of 20ms, where 8bits aA,aA+1,aA+2,aA+3,...,aA+7 including half frame indication, MSB of kSSB or 3 bits of SSB index, and 4th, 3rd, 2nd, and 1st LSB of SFN are generated from PHY layer and are attached to the MIB generated in higher-layer. In this way, the same MIB is preserved for all of the 4 SSB instances to be combined within the TTI. In addition, the bits corresponding to the 3rd and 2nd LSB of SFN are not scrambled in the first scrambling procedure prior to encoding, and are used to determine the sequence of the first scrambling. The reason for choosing the 3rd and 2nd LSB of SFN is that only the 2nd and 3rd LSB of the SFN vary for any given 4 SSB instances belonging to the given 80ms PBCH TTI and 20ms periodicity. The second scrambling based on PCI is applied to all PBCH payload bits, and can be de-scrambled by the UE according to the PCI. In summary, here is the legacy UE behavior of decoding the PBCH:

- For the single-shot transmission without SSB combination, the UE simply performs decoding of the encoded PBCH bits after the first de-scrambling based on PCI;

- For the multi-shot transmission with SSB combination, the UE performs soft-combining over multiple SSB instances under the hypotheses on {2nd, 3rd LSB SFN} after the first de-scrambling based on PCI.

However, if the SSB periodicity is extended to some values larger than 20ms, the varying bits of the SFNs carrying the multiple SSB instances within a PBCH TTI may be different. Figure 3 shows an example with the SSB periodicity extended to 40ms, where 4-shot transmission is assumed. For simplicity, the SFN starts with 0, but it can be proved to be true for any other value. From the figure, the varying bits become the 3rd and 4th LSB of the SFN, instead of the 2nd and 3rd bits in the legacy behavior. Consequently, the bits used to determine the sequence for the first scrambling and kept unscrambled in the first scrambling procedure should be the 3rd and 4th LSB of the SFN. Meanwhile, the UE performs soft-combining under the hypotheses on {3rd, 4th LSB SFN} instead.

Figure 3. An example with SSB periodicity of 40ms and the varying bits of the SFN.

Table 4 shows more a detailed analysis regarding the impacts on the PBCH generation, given different potential values of the extended SSB periodicity. Note that there is no change in the SFN for the one-shot case, and thus the legacy PBCH generation procedure can be completely reused without requiring modifications. Clearly from the table, if the 4-shot transmission is assumed, the PBCH generation needs significant modifications, with regards to the TTI of the PBCH, the SFN bits used for generating scrambling sequence and the bits of SFN delivered by higher layers (and the physical layer as well, since the remaining SFN bits are delivered by the physical layer). As per the amended WID, any modification to the SSB other than periodicity should not be considered. Alternatively, if the SSB generation is kept legacy with the SSB periodicity extended, the multi-shot transmission of the SSB may be unavailable and the gains by SSB combination are lost. Consequently, the link budget of the SSB may also be degraded.

Observation 5: The SSB periodicity is related closely to how to generate the PBCH, including the scrambling of PBCH and the bits of SFN delivered by the higher layer. When extending the SSB periodicity:

- Alternative 1: Considering potential modifications to the legacy SSB generation procedures to preserve the multi-shot SSB combination.

- Alternative 2: Keeping the legacy SSB generation procedures with only 1-shot SSB decoding available. In this case, the link budget of SSB would be degraded.

Observation 6: To avoid modifying the SSB generation procedures and preserve backward compatibility, the SSB periodicity should not be larger than 40ms or 160ms respectively if 4-shot or 1-shot combination is assumed.

TS 38.212 7.1.2 Scrambling For PBCH transmission in a frame, the bit sequence is scrambled into a bit sequence , where for and is generated according to the following: ; ; while if corresponds to any one of the bits belonging to the candidate SS/PBCH block index, the half frame index, and 2nd and 3rd least significant bits of the system frame number ; else ; ; end if ; end while The scrambling sequence is given by Clause 5.2.1of [4, TS38.211] and initialized with at the start of each SFN satisfying ; for Lmax=4 or Lmax=8, M=A-4 for Lmax=10, M=A-5 for Lmax=20, and for Lmax=64, where Lmax is the number of candidate SS/PBCH blocks in a half frame according to Clause 4.1 of [5, TS38.213]; and is determined according to Table 7.1.2-1 using the 3rd and 2nd LSB of the SFN in which the PBCH is transmitted. Table 7.1.2-1: Value of for PBCH scrambling (3rd LSB of SFN, 2nd LSB of SFN); Value of. (0, 0); 0. (0, 1); 1. (1, 0); 2. (1, 1); 3.

Table 4. Impacts on the PBCH generation with periodicity of 40/80/160ms and 1/4-shot combination

| Periodicity | 1 shot is assumed |  |  |  |
| --- | --- | --- | --- | --- |
|  | TTI of PBCH | Bits of SFN Varying during TTI | Bits of SFN Not Scrambled in PBCH | Bits of SFN Carried by Higher Layers |
| 40ms | / | / | / | / |
| 80ms | / | / | / | / |
| 160ms | / | / | / | / |
|  | 4 shots are assumed |  |  |  |
|  | TTI of PBCH | Bits of SFN Varying during TTI | Bits of SFN Not Scrambled in PBCH | Bits of SFN Carried by Higher Layers |
| 40ms | 160ms | 3rd and 4th LSB of SFN | Additionally include 4th LSB of SFN on top of the legacy behavior | Unchanged (i.e., 6 bits for the 5th -10th LSB of SFN) |
| 80ms | 320ms | 4th and 5th LSB of SFN | Additionally include 4th and 5th LSB of SFN on top of the legacy behavior | 1 less bit corresponding to the 5th LSB of SFN, resulting in one more reserved bits |
| 160ms | 640ms | 5th and 6th LSB of SFN | Additionally include 5th and 6th LSB of SFN on top of the legacy behavior | 2 less bits corresponding to the 5th and 6th LSB of SFN, resulting in two more reserved bits |

The second aspect is related to the SMTC configurations. In the current spec, the periodicity of SMTC carried by SSB-MTC can be at most 160 subframes and the ssb time offset between the SSB from source and target satellite indicated by SIB19 also has the range limited by 160 subframes. Clearly, the range of the above two parameters can no longer be sufficient if the SSB periodicity is extended beyond 160ms.

Observation 7: SMTC configurations are currently limited by 160ms. To preserve backward compatibility, the SSB periodicity should not be larger than 160ms.

The third aspect is related to the RRM measurements. As an important reference signal, SSB is crucial for RLM, BFD, intra-frequency measurement and inter-frequency measurements. For example, according to TS 38.133, we have

- For RLM: UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_SSB [ms] period becomes worse than the threshold Qout_SSB within TEvaluate_out_SSB [ms] evaluation period. The time duration specified by TEvaluate_out_SSB is based on the SSB periodicity if DRX is not configured, or the maximum between SSB perioidicity and DRX cycle if the DRX cycle less or equal to 320ms is configured, or the DRX cyle if the DRX cycle is larger than 320ms.

- For BFD: UE shall be able to evaluate whether the downlink radio link quality on the configured SSB resource in set q0 estimated over the last TEvaluate_BFD_SSB ms period becomes worse than the threshold Qout_LR_SSB within TEvaluate_BFD_SSB ms period. The time duration specified by TEvaluate_BFD_SSB is based on the SSB periodicity if DRX is not configured, or the maximum between SSB perioidicity and DRX cycle if the DRX cycle less or equal to 320ms is configured, or the DRX cyle if the DRX cycle is larger than 320ms.

- For intra-frequency measurements (without measurement gaps for example): The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index or within Tidentify_intra_with_index. Both the time duration specified by Tidentify_intra_without_index and Tidentify_intra_with_index are based on the SMTC period if DRX is not configured, or the maximum between SMTC period and DRX cycle if the DRX cycle less or equal to 320ms is configured, or the DRX cyle if the DRX cycle is larger than 320ms.

- For inter-frequency measurements (without measurement gaps for example): UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_without_index or within Tidentify_inter_with_index. Both the time duration specified by Tidentify_inter_without_index and Tidentify_inter_with_index are based on the SMTC period if DRX is not configured, or the maximum between SMTC period and DRX cycle if the DRX cycle less or equal to 320ms is configured, or the DRX cycle if the DRX cycle is larger than 320ms.

- For timing accuracy requirements: The UE shall meet the Te_NTN  requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms.

We can observe that, if the SSB periodicity is extended (so is the SMTC period) beyond 160ms and DRX is enabled, the measurement durations for the RLM, BFD, intra- and inter-frequency measurement durations are always limited by the DRX cycle but in fact SSB may not appear within those durations. To avoid this issue, the UE behavior of the RLM, BFD, intra- and inter-frequency measurements have to be adjusted at least for determining the measurement durations. Regarding the timing accuracy requirements, it is now assumed that SSB is always available at least every 160ms, which makes it questionable if the current requirements are still valid in the case with the SSB periodicity larger than 160ms. Meanwhile, any change to enlarge this value leads to different UE behavior from the legacy one.

Observation 8: The UE behavior of RRM measurement has to be modified if the SSB periodicity is extended beyond 160ms, otherwise the SSB may not appear within the determined RRM measurement durations. To preserve backward compatibility, the SSB periodicity should not be larger than 160ms.

Observation 9: The timing accuracy requirements are defined based on the appearance of the SSB at least every 160ms. It is questionable if the current requirements are still valid if the SSB periodicity is extended beyond 160ms, and meanwhile any change to enlarge this value leads to different UE behavior from the legacy one.

From the above observations, if the SSB periodicity is extended to 320ms, significant spec impacts can be foreseen and result in the backward compatibility issue even more significantly, including the new network indication and the new UE interpretation of the SMTC configuration, as well as the new behavior of RRM measurement.

Observation 10: If the SSB periodicity is extended further from 160ms to 320ms, significant spec impacts can be foreseen and result in the backward compatibility issue even more significantly, including the new network indication and the new UE interpretation of the SMTC configuration, as well as the new behavior of RRM measurement.

2.3 Impacts on signal receptions

According to Section 2.2, the SSB reception has to be compromised from 4-shot combination to 2-shot or even 1-shot combination if the SSB periodicity exceeds 20ms. The impacts on the SSB receptions can be elaborated from two perspectives:

- Link budget degradation with fewer combinations, such as 2-shot and 1-shot SSB reception respectively suffers 3dB and 6dB loss compared to the 4-shot case.

- Higher complexity for dealing with frequency shifts. SSB may suffer intense frequency offsets, particularly PSS, for which UEs needs to set up multiple searchers around the target frequencies on the sync raster. For the case with the frequency offset of 24ppm, the frequency shifts are around 48kHz at the center carrier frequency of 2GHz. If multi-shot SSB reception is available, UE can search the ±48kHz by multiple times, e.g., a range of around 25kHz for each SSB. UE can tune the center frequency to be the center of the target range, obtain the baseband samples, and use multiple searchers with a small step-size to search for the PSS in parallel. In contrast, for the single-shot SSB reception, UE has to receive all the baseband samples for a wider bandwidth and implement four times as many parallel searchers as for the 4-shot case. Both issues lead to higher complexity and/or implementation costs. Figure 4 gives an exemplary illustration.

Figure 4. An exemplary illustration of the frequency offset handling in 4-shot and 1-shot SSB reception

Observation 11: Compared with 4-shot SSB reception, the SSB reception with fewer shots not only leads to a worse SSB link budget, but also requires UEs to pay higher complexity and/or implementation costs for handling the frequency offset issue in the SSB reception.

Due to the satellite movements, the frequency shifts of the satellite keep changing at certain drifts. As SSB is an important reference signal for correcting the frequency shifts, a sparse SSB pattern may impact the performance of other physical channels or signals than SSB, due to the accumulation of the frequency offset drifts. We give the simulations results in Figure 5-7 of the PDCCH and PDSCH with SIB1/SIB19 when SSB periodicity is extended to 80ms, 160ms, 320ms and 640ms. The details of the simulation parameters are shown in Appendix B.

Figure 5. BLER of the PDCCH with different SSB periodicities

Figure 6. BLER of the SIB1 PDSCH with different SSB periodicities. Left: TBS of 800bits; Right: TBS of 1200 bits

Figure 7. BLER of the SIB19 PDSCH with different SSB periodicities.

From the simulations results, little impacts can be observed for the PDCCH reception. However, the PDSCH with SIB1 and SIB19 are sensitive to the choice of the SSB periodicity, regardless of whether repetitions are used or not. Particularly, the SSB periodicity of 320ms and 640ms can respectively lead to more than 2dB and 5dB loss for the SIB1 PDSCH reception, and more than 1.4dB and 4dB loss for the SIB19 PDSCH reception.

Observation 12: Apparent degradations can be observed towards the PDSCH reception performance when the SSB periodicity exceeds 160ms, because of the accumulative effects of frequency offset drifts.

Based on the observations above, we make the following proposals:

Proposal 1: Methods should be considered for alleviating the negative impacts of extended SSB periodicity on the UE, such that the UE overheads during/after initial access are not higher than those with the default periodicity of 20ms, in terms of the latency, complexity, and power consumption.

Proposal 2: Backward compatibility should be preserved for the system level design. Any potential extension to the SSB periodicity should be carefully designed to avoid raising any backward compatibility issue, at least including PBCH generation and reception, SMTC configurations, and RRM measurement.

Proposal 3: The SSB periodicity should not be extended beyond 160ms, considering the aspects of UE’s power consumption, implementation costs, link performance degradations from unreasonably explosive growth and backward compatibility issues.

- Wide/Narrow Beam Footprint

The following observation and agreements were captured for the system level evaluation in RAN1#117 and RAN1#118:

RAN1#117 Observation Based on the results of DL coverage ratio evaluation at system level collected from 7 sources for all the three LEO600km satellite parameter sets where the beam footprint diameter is 50 km: - For Set 1-1/1-3, the coverage ratio can be improved from 10% to 100% if the SSB periodicity is increased from 20ms to 80ms and beam hopping is applied - For Set 1-2, the coverage ratio can be improved from 1.5% to 96.8% if the SSB periodicity is increased from 20ms to 320ms and beam hopping is applied. - Note: coverage ratio is N2+N3/ total beam footprints - Note: the baseline assumes no beam hopping. TDM between SIB1 and SIB19 is assumed in those results, following current specs. Based on the results of DL coverage ratio evaluation at system level collected from 3 sources for a deployment scenario implementing wide beam footprint: - 1 source reports that with a deployment of wide beam covering 4 narrow (of 50km size) beams, which means Set 1-2 FR1 with additional EIRP reduction of 6dB, using SSB periodicity of 80 ms can provide coverage ratio of 96.8%, and Set 1-1/1-3 FR1 with additional EIRP reduction of 6dB, SSB periodicity of 80 ms can provide coverage of 100%. - 1 source observed that for Set 1-1, 1-2 and 1-3, the coverage ratio can be improved from 1.5% to 100% using the legacy default SSB periodicity of 20ms during initial access, by choosing a wide beam footprint with beam footprint sizes of 84 km and 56 km respectively. - Note: the PDCCH and the PDSCH for SIB19 is assumed to be transmitted within 2 OFDM symbols and 5 MHz bandwidth. the PDSCH for SIB1 is assumed to be transmitted within 3 OFDM symbols and 5 MHz bandwidth. This assumes no SIB1 and SIB19 transmission in N2 beam footprints. This assumes non-aligned SFN timing across different beams. - 1 source observed, for Set 1-1 with increased beam size, that the legacy SSB periodicity of 20ms during initial access is usable with NTN beam hopping, by choosing a deployment scenario implementing wide beam footprint with beam footprint sizes of 70.7 km and 86.6 km, leading to a total of 529 and 353 beam footprints within the satellite coverage area, respectively, and the coverage ratio is 80% and 90%, respectively, and a ratio of simultaneously active beam footprints to the total number of beam foot prints equal to 20% and 30%. - Note: Beam footprint size is increased by increasing only the adjacent beam spacing without increasing the 3dB beamwidth. Note: RAN1 will further investigate the impact of SSB periodicity extension Note: Any needed clarification “SSB channel enhancement is not considered” in the WID is up to RAN plenary Note: RAN1 will further investigate the impact of wider beam of SSB and/or other channels on performance (e.g. link budget, capacity...) RAN1#118 Agreement As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint

As captured in the observations in RAN1#117, an appropriate beam pattern can reduce the overheads for common signallings in the system level. In this part, we give an exemplary design of the beam hopping model, where the beam footprint adopted for broadcasting SSB is wider than that for data transmission. Figure 8 depicts an example with the ratio of the number of SSB transmission beams to that of data transmission beams to be 1:3, where the red and black circles are respectively the beams for SSB and the beams for data transmissions. Through the wide beam footprint, the number of total beams to be illuminated for SSB transmission is reduced to be 353.

Figure 8. Example of the beam hopping model with wide beam footprint for SSB

Similar to the gNB behavior in TN, gNB in NTN can transmit SSB by sweeping all beams in a single SSB burst within 5ms. Therefore, the dwell time for a beam to transmit SSB is set to be TSSB (such as 4 OFDM symbols for a single SSB), the total resources required for sweeping all of 353 beams for SSB is given by

353NActive*TSSB*25RB

every SSB periodicity, where the number of active beams NActive should be 106 for Set1-1/1-3 FR1, and 16 for Set 1-2 FR1. Assuming the SIB1 and SIB 19 are delivered at the same periodicity of SSB via the beams for data transmissions, and the CORESET, SIB1 and SIB19 respectively occupy 2, 3, and 2 OFDM symbols in the time domain, and 24 PRBs in the frequency domain, we have the following equation for estimating the systematic overheads for common signallings:

OH=NActive*TSSB*25RB*N2NActive+NActive*24RB*7symbol*N3NActiveNActive*25RB*1ms* TP,

where TP stands for the periodicity of SSB and SIB1/SIB19, in the unit of slot. In the above equation, all active beams are used for transmitting SSB over 353 wide beams, and for transmitting SIB1, SIB19 and data over 1058 narrow beams, that is, N1=0, N2=353, N3=1058. We summarize the estimates of the overheads of common signallings for parameter set 1-1/1-2/1-3 FR1 in Table 5, where N/A stands for the infeasible case

with the overhead over 100%. Besides, the corresponding system level simulation results for different TSSB and SSB/SIB1/SIB19 periodicity configurations are listed in Appendix A.

Table 5. Estimated overheads of common signallings (SSB, SIB1, and SIB19)

| TSSB | SSB/SIB1/SIB19 periodicity |  | 20ms | 40ms | 80ms | 160ms |
| --- | --- | --- | --- | --- | --- | --- |
| 4 Symbols for 1 SSB | Set 1-1/1-3 | OH w/o wide beam footprint | 38.21% | 19.11% | 9.55% | 4.78% |
|  |  | OH w/ wide beam footprint | 28.71% | 14.36% | 7.18% | 3.59% |
|  | Set 1-2 | OH w/o wide beam footprint | N/A | N/A | 63.29% | 31.65% |
|  |  | OH w/ wide beam footprint | N/A | 95.11% | 47.55% | 23.78% |
| 8 Symbols for 2 SSB | Set 1-1/1-3 | OH w/o wide beam footprint | 52.47% | 26.24% | 13.12% | 6.56% |
|  |  | OH w/ wide beam footprint | 33.47% | 16.73% | 8.37% | 4.18% |
|  | Set 1-2 | OH w/o wide beam footprint | N/A | N/A | 86.91% | 43.45% |
|  |  | OH w/ wide beam footprint | N/A | N/A | 55.43% | 27.72% |
| 16 Symbols for 4 SSB | Set 1-1/1-3 | OH w/o wide beam footprint | 81.00% | 40.50% | 20.25% | 10.12% |
|  |  | OH w/ wide beam footprint | 42.98% | 21.49% | 10.75% | 5.37% |
|  | Set 1-2 | OH w/o wide beam footprint | N/A | N/A | N/A | 67.07% |
|  |  | OH w/ wide beam footprint | N/A | N/A | 71.19% | 35.60% |

The dwell time for SSB is considered to be 4, 8, and 16 OFDM symbols to obtain the results in Table 3, which can respectively accommodate up to 1, 2 or 4 SSB. From the table, it can be seen that the overheads caused by common signallings can be reduced by around 50% if the proposed beam hopping model is adopted.

Notice that flexible power sharing among beams is in the scope of the WID, the link budget of the wide beams can be further boosted by dynamic power sharing, which can be left for the gNB implementation.

Observation 13: A proper beam hopping model can significantly reduce the overheads for common signallings, and reduce the need for a significant extension to the SSB periodicity.

Proposal 4: RAN1 to first clarify the beam hopping model adopted for SSB and data transmissions before analyzing the necessity of extending the periodicity of SSB.

- Link level enhancement

The following agreement was captured for the link level study in RAN1#118bis:

RAN1#117 Observation Based on LLS results on PDCCH coverage evaluation collected from different sources: - It is observed that the required SNR for PDCCH is in average equal to -6dB (17 sources) - With parameter LEO600km Set1-1 FR1 and 1-2 FR1: - 17 sources observed that there is no coverage gap with Set1-1/1-2 FR1. - The coverage margin is around 4 dB compared to CNR of -1.9 dB. - With parameter LEO600km Set1-3 FR1: - 15 sources observed that there is a PDCCH coverage gap of 3.9dB in average compared to CNR of -9.9 dB. - Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDCCH. Observation Based on LLS results on PDSCH Msg4 coverage evaluation collected from different sources: - It is observed that the required SNR for PDSCH carrying Msg4 is in average equal to – 5.2 dB (14 sources) - With parameter LEO600km Set1-1 FR1 and 1-2 FR1: - 14 sources observed that there is no coverage gap for PDSCH with Msg4: - The coverage margin is around 3.3 dB on average compared to CNR of -1.9 dB - With parameter LEO600km Set1-3 FR1: - 11 sources observed that there is a coverage gap for PDSCH with Msg4: - The coverage gap is around 4.7 dB on average compared to CNR of -9.9 dB - 1 source observed that there is no coverage gap for PDSCH with Msg4 with a coverage margin of 0.3 dB compared to CNR of -9.9 dB - Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH Msg4. Observation Based on LLS results on PDSCH SIB1 coverage evaluation collected from different sources: - For PDSCH carrying SIB1 option 1 (with a payload size of 800bits) it is observed that the required SNR is in average equal to – 5.8 dB (14 sources) - For PDSCH carrying SIB1 option 2 (with a payload size of 1280bits) it is observed that the required SNR is in average equal to – 3.4 dB (12 sources) - With parameter LEO600km Set1-1 FR1 and 1-2FR1: - 14 sources observed that there is no coverage gap for PDSCH with SIB1 option 1: - The coverage margin is around 3.9 dB on average compared to CNR of -1.9 dB - 12 sources observed that there is no coverage gap for PDSCH with SIB1 option 2: - The coverage margin is around 1.5 dB on average compared to CNR of -1.9 dB - With parameter LEO600km Set1-3 FR1: - 11 sources observed that there is a coverage gap for PDSCH with SIB1 option 1: - The coverage gap is around 4.1 dB on average compared to CNR of -9.9 dB - 1 source observed that there is no coverage gap for PDSCH with SIB1 option 1: - The coverage margin is 3.4 dB compared to CNR of -9.9 dB - 10 sources observed that there is a coverage gap for PDSCH with SIB1 option 2: - The coverage gap is around 6.5 dB on average compared to CNR of -9.9 dB - Note: some results assumed SIB1 combination (where SIB1 is repeated within 160 ms) and some results assumed no SIB1 combination - Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH carrying SIB1. RAN1#118bis Agreement Support PDCCH CSS Link level enhancement in Rel-19 for all CSS types except type 3. - The following techniques are for further study: - PDCCH repetition, including: - Option 1: Intra-slot PDCCH repetition - Option 2: Inter-slot PDCCH repetition - CORESET length (i.e. number of OFDM symbols) extension - DCI format optimization (e.g. size reduction, etc) - Note: the same technique is intended to apply to all search space types targeted for link level enhancements - For the above techniques, at least the following aspects should be discussed for the relevant candidate techniques: - Configuration - Backward compatibility and UE behaviour of legacy UE - Linked Search Space - Blocking probability - DCI format size budget - Resource overhead - Impact on CORESET0 - Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. - FFS: whether to apply the selected solution to PDCCH CSS type3 and PDCCH USS

In this section, we discuss the potential enhancements of PDCCH, SIB1 PDSCH and Msg4 PDSCH.

From the agreement of RAN1#118bis, the target CNR is set to be -8dB. Therefore, for Set1-3 FR1, according to the RAN1’s observations in RAN1#117, the link budget gaps of PDCCH, MSG4 PDSCH and SIB1 are 2dB, 2.8dB, and 1.5dB (payload size of 800bits) or 4.5dB (payload size of 1280bits), respectively.

Observation 14: The target of link level enhancements for PDCCH is 2dB.

Observation 15: The target of link level enhancements for Msg4 PDSCH is 2.8dB.

Observation 16: The target of link level enhancements for SIB1 PDSCH is 1.5dB for payload size of 800bits, or 4.5dB for payload size of 1280bits.

4.1 PDCCH Enhancements

There are three candidate solutions are considered for PDCCH link level enhancements, namely PDCCH repetition, DCI format optimization, and CORESET length extension.

4.1.1 PDCCH Repetition

NR already supports PDCCH repetitions for connected mode UE. The candidate PDCCH monitoring occasions are determined by the configuration of search spaces, including the periodicity and offset of the search spaces. According to the current PDCCH repetition scheme introduced by mTRP, up to two individual search spaces can be linked together, but they are required to have the same periodicities and to have their respective PDCCH monitoring occasions located within a slot. In other words, during each period, two monitoring occasions respectively determined by the two linked search spaces carry two copies of a same PDCCH.

Observation 17: The PDCCH repetition schemed introduced by mTRP is to linked up to two individual search spaces. The two search spaces are required to have the same periodicities and to have their respective PDCCH monitoring occasions located within a slot.

However, for idle and inactive mode UEs, before the acquisition of SIB1, UE can be informed only by the MIB with the Type0-PDCCH CSS configuration, with which the UE obtains the PDCCH scheduling the SIB1 PDSCH. Hence, if the mTRP PDCCH repetition is reused here, MIB is required to deliver the configurations of at least one additional search space, specifically, 4 bits for each search space. Clearly, MIB has no more space for those bits. Thus, the mTRP PDCCH repetition scheme may not be simply reused for the Type0-PDCCH CSS.

Observation 18: MIB has no more space to carry the additional bits to introduce an additional search spaces for search space linkage.

One possible solution is to link more than one monitoring occasion associated with the single Type0-PDCCH CSS for PDCCH repetition, instead of linking multiple search spaces. Current spec has defined two candidate monitoring occasions associated with each SSB index in two consecutive slots for search space zero, yet neither gNB nor UE assumes that both occasions are available and carry the same PDCCH. It is straightforward to specify the two monitoring occasions to be bound for repeating a same PDCCH, however, the possible repetition number is not larger than two. On the other hand, any other search space except Type0-PDCCH CSS is only associated with a single monitoring occasion, and thus requires separate design.

Observation 19: In current spec, Type0-PDCCH CSS specified two candidate monitoring occasions associated with each SSB index in two consecutive slots. The two monitoring occasions can be bound for PDCCH repetition, but the maximum repetition number is limited to two. Meanwhile, any other search space except Type0-PDCCH CSS is only associated with a single monitoring occasion, and thus needs separate design.

Alternatively, the additional monitoring occasions can be not limited to the two candidate monitoring occasions associated in two consecutive slots. In fact, multiple monitoring occasions can be defined according to the first monitoring occasion derived from the search space by legacy rules. For example, if the repeated monitoring occasions can be obtained by shifting the first monitoring occasion by a certain number of slots. Figure 9 depicts an exemplary case with the index 0 of the Table 13-11 in TS 38.213 (i.e., TDD pattern of Type0-PDCCH CSS), and three repetitions. As such, both Type0-PDCCH CSS and any other CSS or even USS can follow a unified design. For instance, regarding Type0-PDCCH CSS, the gap between the initial transmissions and the repetitions could be pre-defined to be a specific value, e.g., 1 slot, while the gap could be indicated by SIB or RRC configurations to be any other value for any other search space except Type0-PDCCH CSS.

Figure 9. Example of creating two additional monitoring occasions for PDCCH repetition

For Type0-PDCCH CSS, linking more than one monitoring occasion may result in the overlapping between the PDCCH monitoring occasions and the candidate SSB receptions. For example, in Figure 9, repeated monitoring occasions associated with SSB 0 may be overlapping with the original monitoring occasions associated with SSB 1 or SSB 2. Considering that gNB adopts different spatial filters (i.e., beams) to transmit the SSB with different indexes and their associated search spaces, a UE will not receive the overlapping SSB if the repeated monitoring occasions associated with a different SSB index. In other words, the overlapping issue does not impact the signal receptions for any given UEs.

Observation 20: Repeated PDCCH candidates can be transmitted through multiple monitoring occasions associated with the single type-0 search space, instead of linking multiple search spaces that are separately defined. Though repeated monitoring occasions may overlap with an SSB associated with a different SSB index, they have no impacts on the signal receptions for any given UE.

Proposal 5: RAN1 may further study to link multiple monitoring occasions associated with a single search space for PDCCH repetition, at least the following two options to deliver the repeated PDCCH

- Linking two candidate monitoring occasions associated with each SSB index specified by Type0-PDCCH CSS

- Linking additional monitoring occasions derived from the first monitoring occasion, e.g., shifting by some slots

Proposal 6: RAN1 to consider to a unified repetition design for all CSS types.

Regarding the PDSCH scheduled by the PDCCH, pre-R19 and R19 UE may have different interpretations of the scheduling timeline carried in the PDCCH, e.g., k0. Although multiple candidate monitoring occasions are defined for a configured search space, UE may receive the PDCCH candidates in only part of the monitoring occasions. For a R19 UE, it knows where the first and the last candidate monitoring occasion is, the scheduling timeline is clear. However, for a pre-R19 UE, it can only receive the PDCCH candidates in the legacy monitoring occasions, i.e., two monitoring occasions for Type0-PDCCH CSS, or only one monitoring occasion for any other CSS type. We analyze two potential cases:

- Case 1: for Type0-PDCCH CSS, if the network uses the two candidate monitoring occasions for repetition, pre-R19 UE may receive a PDCCH in either monitoring occasion. In this case, no matter how to indicate the scheduling timeline, there is a chance for a pre-R19 obtain an incorrect scheduling timeline. Alternatively, the network can mute either monitoring occasion and choose other additional monitoring occasions (where pre-R19 UE does not monitor) for repetition. Meanwhile, the network indicates the scheduling timeline based on the first monitoring occasion, and transmits a corresponding PDSCH according to the first monitoring occasion. An exemplary illustration is shown in Figure 10.

Figure 10. Illustration of case 1, where scheduling timeline is based on the first monitoring occasion

- Case 2: for other CSS than Type0-PDCCH CSS, pre-R19 UE can only have a single monitoring occasion. In this case, the network can also indicate the scheduling timeline based on the first monitoring occasion, and transmit a corresponding PDSCH according to the first monitoring occasion. An exemplary illustration is shown in Figure 11.

Figure 11. Illustration of case 2, where scheduling timeline is based on the last monitoring occasion

From the aforementioned analysis, backward compatibility can be achieved with some restrictions:

- For Type0-PDCCH CSS, network mutes a monitoring occasion (defined by pre-R19 rules) associated with the SSB index, and choose other additional monitoring occasions for repetition, where pre-R19 UE does not monitor.

- For all CSS types, network indicates timeline is based on the first PDCCH monitoring occasion, which can be determined either with or without repetition enabled.

- For all CSS types, a PDSCH needs to be transmitted corresponding to the first PDCCH monitoring occasion.

Note that, besides the PDSCH corresponding to the first PDCCH monitoring occasion, there should be another PDSCH scheduled after the last PDCCH monitoring occasion, which is for the R19 UEs only. For R19 UEs, they are aware of the two PDSCH candidate and can use them for joint decoding.

Proposal 7: RAN1 should further study how to schedules the PDSCH via the repeated PDCCH, e.g., the scheduling timeline, i.e., the k0, in the case of PDCCH repetition.

4.1.2 DCI format optimization

Another candidate solution is to reduce the DCI size, so that the required SNR of the PDCCH can be reduced accordingly. Table 6 presents the achievable gains by reducing DCI payload and CRC compared to the baseline design. The simulation assumptions to obtain the table is shown in Appendix D. Specifically, the gains generally marginal, unless the total number of payload bits and CRC bits is reduced to less than 32 bits. In that case, more than 2dB gain can be achieved, e.g., a payload of  8bits and a CRC of 24 bits, or a payload of  16bits and a CRC of 16 bits. If we follow the legacy DCI format and maintain the CRC to be 24 bits, the payload of the DCI has to be reduced to 8bits.

Table 6. Achievable gains by reducing DCI payload and CRC, compared to the baseline design

| Cases | 24-bit payload + 24-bit CRC | 16-bit payload + 24-bit CRC | 8-bit payload + 24-bit CRC | 16-bit payload + 16-bit CRC | 12-bit payload + 16-bit CRC | 8-bit payload + 16-bit CRC |
| --- | --- | --- | --- | --- | --- | --- |
| Gain relative to baseline (i.e., 40-bit payload+24-bit CRC) | 0.9dB | 1.51dB | 2.1dB | 2.1dB | 2.5dB | 2.93dB |

Observation 21: The gain brought by DCI size reduction is generally marginal. DCI size reduction may not fully bridge the PDCCH link budget gap, unless the total number of payload bits and CRC bits is reduced to less than 32 bits. If the legacy CRC design of 24 bits are inherited, the payload of the DCI has to be reduced to 8bits.

With at most 8 bits, the target DCI format can only deliver the most necessary information under many pre-defined assumptions. Taking the DCI for scheduling SIB as an example, we list some potential fields for considerations:

- FDRA: can be 0 bit if the FDRA is the same as CORESET#0

- TDRA: 1 to 4 bits with less possible entries in the TDRA table

- MCS: 3 to 4 bits with less possible entries in the MCS table, e.g., fixed modulation order (e.g., only QPSK)

- Indicator of SIB: 1 bit for indicating SIB1 or OSI, or 0 bit for SIB1 only

Observation 22: A compact DCI format (e.g., similar to LTE DCI format 1C) can meet the link level performance target, with 8 bits payload and 24 bits CRC, or 16bits payload with 16 bits CRC.

4.1.3 CORESET length extension

The third candidate solution is to enlarge the number of symbols for CORESET. Then, the available resources for PDCCH are increasing. However, firstly, it does not allow the legacy UE to receive the PDCCH in the extended CORESET. Secondly, it requires significant efforts to support CORESET beyond 3 symbols. For example, if four symbols CORESET are introduced, the configurable REG bundle size, the CCE size, and the mapping rule may all need to change. Thirdly, if more symbols are allocated to CORESET, the remaining resource for PDSCH is limited, which may degrade the performance of PDSCH.

Observation 23: The CORESET symbol extension has some issues, such as backward compatibility issue, and may degrade the performance of the scheduling PDSCH (e.g., SIB). Moreover, it requires significant efforts thus is challenging to be completed in Rel-19.

4.2 Msg4 enhancement

For PDSCH with Msg4, a straightforward solution is to enable retransmission via HARQ, and each retransmission can provide ~3dB gain. The RTT delay is around 25.77ms and 12.89ms for transparent and regenerative payload in LEO-600, respectively. Considering the ra-ContentionResolutionTimer can be configured up to 64 subframes, at most 1 and 3 potential retransmissions can be allowed for transparent payload and regenerative payload, respectively, before ra-ContentionResolutionTimer expires.

We give the simulations results in Figure 12 of the PDSCH with Msg4 for different retransmission times. From the simulations results, the SNRs are -7.7dB and -10.4dB for 2 and 4 times retransmissions, respectively. Hence, there are 0.3dB and 2.4dB converage margins for 2- and 4-times retransmissions, respectively, compared to the -8dB targert CNR.

Figure 12 The SNR-BLER of the PDSCH Msg4 with different retransmission times

Observation 24: Retransmission can solve the coverage issue of PDSCH with Msg4.

Proposal 8: Retransmission is can fulfil the coverage gap of PDSCH with Msg4, and no enhancements are needed.

4.3 SIB enhancements

Regarding the PDSCH with SIBs other than SIB1 (OSI), including SIB19, it can be transmitted within the SI window configured by the network. Within a SI window, the content of SIB remains unchanged, which means that gNB can repeat the OSI PDSCH within the SI window and the UE can combine the repetitions if necessary, where each repetition can provide ~3dB gain to fill the coverage gap.

Observation 25: NR already support repetition for OSI.

Proposal 9: No need to consider the enhancements for OSI.

As for SIB1, neither retransmission nor repetition is supported. One possible solution is to allow UE to receive multiple PDSCH respectively scheduled by multiple PDCCH candidates (in multiple monitoring occasions derived from Type0-PDCCH CSS) for joint decoding, similar to other SIs. How to define the monitoring occasions and their association with SSB index highly depends on the PDCCH and SSB design. The discussions on SIB1 enhancements should wait until the discussions on PDCCH and SSB enhancements are clear.

Proposal 10: The discussions on SIB1 enhancements should wait until the discussions on PDCCH and SSB enhancements are clear.

- Conclusion

In this contribution, we provide our views on downlink coverage enhancement for NR-NTN. According to the discussions, we have following observations and proposals:

Observation 1: If the SSB periodicity is extended to 320ms, the UE has to pay for 16-times higher latency, complexity, and power consumption for initial access, compared with the 20ms case.

Observation 2: The extension of SSB periodicity also induces higher latency regarding RRM, RLM, BFD, etc.

Observation 3: For parameter set 1-3 FR1, successful initial cell selection can not be achieved if the SSB periodicity is 320ms.

Observation 4: For parameter set 1-1 FR1, the success rates of initial cell selection are reduced from 68.7% to 44%, if the SSB periodicity is extended from 160ms to 320ms.

Observation 5: The SSB periodicity is related closely to how to generate the PBCH, including the scrambling of PBCH and the bits of SFN delivered by the higher layer. When extending the SSB periodicity:

- Alternative 1: Considering potential modifications to the legacy SSB generation procedures to preserve the multi-shot SSB combination.

- Alternative 2: Keeping the legacy SSB generation procedures with only 1-shot SSB decoding available. In this case, the link budget of SSB would be degraded.

Observation 6: To avoid modifying the SSB generation procedures and preserve backward compatibility, the SSB periodicity should not be larger than 40ms or 160ms respectively if 4-shot or 1-shot combination is assumed.

Observation 7: SMTC configurations are currently limited by 160ms. To preserve backward compatibility, the SSB periodicity should not be larger than 160ms.

Observation 8: The UE behavior of RRM measurement has to be modified if the SSB periodicity is extended beyond 160ms, otherwise the SSB may not appear within the determined RRM measurement durations. To preserve backward compatibility, the SSB periodicity should not be larger than 160ms.

Observation 9: The timing accuracy requirements are defined based on the appearance of the SSB at least every 160ms. It is questionable if the current requirements are still valid if the SSB periodicity is extended beyond 160ms, and meanwhile any change to enlarge this value leads to different UE behavior from the legacy one.

Observation 10: If the SSB periodicity is extended further from 160ms to 320ms, significant spec impacts can be foreseen and result in the backward compatibility issue even more significantly, including the new network indication and the new UE interpretation of the SMTC configuration, as well as the new behavior of RRM measurement.

Observation 11: Compared with 4-shot SSB reception, the SSB reception with fewer shots not only leads to a worse SSB link budget, but also requires UEs to pay higher complexity and/or implementation costs for handling the frequency offset issue in the SSB reception.

Observation 12: Apparent degradations can be observed towards the PDSCH reception performance when the SSB periodicity exceeds 160ms, because of the accumulative effects of frequency offset drifts.

Observation 13: A proper beam hopping model can significantly reduce the overheads for common signallings, and reduce the need for a significant extension to the SSB periodicity.

Observation 14: The target of link level enhancements for PDCCH is 2dB.

Observation 15: The target of link level enhancements for Msg4 PDSCH is 2.8dB.

Observation 16: The target of link level enhancements for SIB1 PDSCH is 1.5dB for payload size of 800bits, or 4.5dB for payload size of 1280bits.

Observation 17: The PDCCH repetition schemed introduced by mTRP is to linked up to two individual search spaces. The two search spaces are required to have the same periodicities and to have their respective PDCCH monitoring occasions located within a slot.

Observation 18: MIB has no more space to carry the additional bits to introduce an additional search spaces for search space linkage.

Observation 19: In current spec, Type0-PDCCH CSS specified two candidate monitoring occasions associated with each SSB index in two consecutive slots. The two monitoring occasions can be bound for PDCCH repetition, but the maximum repetition number is limited to two. Meanwhile, any other search space except Type0-PDCCH CSS is only associated with a single monitoring occasion, and thus needs separate design.

Observation 20: Repeated PDCCH candidates can be transmitted through multiple monitoring occasions associated with the single type-0 search space, instead of linking multiple search spaces that are separately defined. Though repeated monitoring occasions may overlap with an SSB associated with a different SSB index, they have no impacts on the signal receptions for any given UE.

Observation 21: The gain brought by DCI size reduction is generally marginal. DCI size reduction may not fully bridge the PDCCH link budget gap, unless the total number of payload bits and CRC bits is reduced to less than 32 bits. If the legacy CRC design of 24 bits are inherited, the payload of the DCI has to be reduced to 8bits.

Observation 22: A compact DCI format (e.g., similar to LTE DCI format 1C) can meet the link level performance target, with 8 bits payload and 24 bits CRC, or 16bits payload with 16 bits CRC.

Observation 23: The CORESET symbol extension has some issues, such as backward compatibility issue, and may degrade the performance of the scheduling PDSCH (e.g., SIB). Moreover, it requires significant efforts thus is challenging to be completed in Rel-19.

Observation 24: Retransmission can solve the coverage issue of PDSCH with Msg4.

Observation 25: NR already support repetition for OSI.

Proposal 1: Methods should be considered for alleviating the negative impacts of extended SSB periodicity on the UE, such that the UE overheads during/after initial access are not higher than those with the default periodicity of 20ms, in terms of the latency, complexity, and power consumption.

Proposal 2: Backward compatibility should be preserved for the system level design. Any potential extension to the SSB periodicity should be carefully designed to avoid raising any backward compatibility issue, at least including PBCH generation and reception, SMTC configurations, and RRM measurement.

Proposal 3: The SSB periodicity should not be extended beyond 160ms, considering the aspects of UE’s power consumption, implementation costs, link performance degradations from unreasonably explosive growth and backward compatibility issues.

Proposal 4: RAN1 to first clarify the beam hopping model adopted for SSB and data transmissions before analyzing the necessity of extending the periodicity of SSB.

Proposal 5: RAN1 may further study to link multiple monitoring occasions associated with a single search space for PDCCH repetition, at least the following two options to deliver the repeated PDCCH

- Linking two candidate monitoring occasions associated with each SSB index specified by Type0-PDCCH CSS

- Linking additional monitoring occasions derived from the first monitoring occasion, e.g., shifting by some slots

Proposal 6: RAN1 to consider to a unified repetition design for all CSS types.

Proposal 7: RAN1 should further study how to schedules the PDSCH via the repeated PDCCH, e.g., the scheduling timeline, i.e., the k0, in the case of PDCCH repetition.

Proposal 8: Retransmission is can fulfil the coverage gap of PDSCH with Msg4, and no enhancements are needed.

Proposal 9: No need to consider the enhancements for OSI.

Proposal 10: The discussions on SIB1 enhancements should wait until the discussions on PDCCH and SSB enhancements are clear.

References

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Huawei, 3GPP TSG RAN Meeting #102, December 11-15, 2023.

- R1-2206063, Discussions on coverage enhancement for NR NTN, vivo, 3GPP TSG RAN WG1 #110, Toulouse, France, August 22nd – 26th, 2022.

- TR 38.811, Study on New Radio (NR) to support non-terrestrial networks.

- R1-2402259, Discussions on NR-NTN downlink coverage enhancement, vivo.

- ITU-T G.711 PULSE CODE MODULATION (PCM) OF VOICE FREQUENCIES.

- TR 37.911, Study on self-evaluation towards the IMT-2020 submission of the 3GPP Satellite Radio Interface Technology (Release 18).

Appendix A. System level simulation parameters and results

| Parameters | Value |
| --- | --- |
| Scenario | RMA |
| Satellite orbit configuration | LEO-600 km altitude |
| Service link frequency | S-band (2GHz) |
| Channel bandwidth | 5MHz |
| 3 dB beam width | 6.06 deg for Set1-1 6.06 deg for Set1-2 5.04 deg for Set1-3 |
| Satellite EIRP density | 31dBW/MHz for Set1-1 31 dBW/MHz for Set1-2 31 dBW/MHz for Set1-3 |
| Satellite antenna gain | 30 dBi, consider 4dB steering loss |
| Satellite G/T | 1.1 dB/K |
| Satellite polarization configuration | Circular |
| Satellite antenna pattern | See section 6.4.1 in TR38.811: Bessel function |
| UE configuration | Handheld |
| UE Antenna type and configuration | Omni-directional; [M N P Mg Ng; Mp Np]=[1 1 2 1 1;1 1] |
| UE antenna polarisation | Linear: ±45°X-pol |
| UE Antenna gain (dBi) | -5.5dBi |
| UE Antenna temperature (K) | 290 |
| Noise figure (dB) | 7 |
| UE Tx transmit power | 200 mW (23 dBm) |
| UEs outdoor/indoor configuration | 100% outdoor distribution for UEs, X = 2,4,6,8,10,… UEs per beam with uniform distribution |
| Handover Margin | 0dB |
| UE attachment | RSRP |
| Channel model | Large scale model and small scale model |
| Propagation conditions | Clear Sky Line of sight |
| Receiver type | MMSE-IRC |
| Scheduler | SU |
| Traffic model | - FTP 3 (see TR 38.821) packet size = 0.5 Mbyte; 200ms as mean inter-arrival time; - FTP 3 packet size = 0.1Mbyte; 2s as mean inter-arrival time; - Voip AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) with 20 ms data arriving interval |
| Channel estimation | Realistic |
| Metrics for performance | - The dwell time and revisit time interval for each beam illumination across the coverage - The UE throughput - User perceived throughput (UPT) - Latency - Cell average throughput |

- Results for LEO600km Set1-1 FR1

For traffic FTP 3 with packet size = 0.5 Mbyte and 200ms as mean inter-arrival time, the results are listed below,

Average SE (bps/Hz/ beam footprint) for the proposed beam hopping design

| TSSB | SSB/SIB1/SIB19 periodicity |  |  |  |
| --- | --- | --- | --- | --- |
|  | 20ms | 40ms | 80ms | 160ms |
| 0.5ms | 0.0500 | 0.0618 | 0.0677 | 0.0706 |
| 1ms | 0.0437 | 0.0586 | 0.0661 | 0.0698 |
| 2m | 0.0314 | 0.0525 | 0.0630 | 0.0683 |

| TSSB | SSB/SIB1/SIB19 periodicity |  |  |  |
| --- | --- | --- | --- | --- |
|  | 20ms | 40ms | 80ms | 160ms |
| 4 Symbols for 1 SSB | 0.0526 | 0.0631 | 0.0684 | 0.0709 |
| 8 Symbols for 2 SSB | 0.0490 | 0.0612 | 0.0674 | 0.0705 |
| 16 Symbols for 4 SSB | 0.0419 | 0.0578 | 0.0656 | 0.0696 |

For traffic VoIP traffic (AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) and 20 ms data arriving interval), the results are listed below,

Average throughput performance (Kbps) of UE for the proposed beam hopping design

| TSSB | SSB/SIB1/SIB19 periodicity |  |  |  |
| --- | --- | --- | --- | --- |
|  | 20ms | 40ms | 80ms | 160ms |
| 0.5ms | 32.51 | 40.25 | 44.13 | 46.07 |
| 1ms | 28.51 | 38.25 | 43.13 | 45.56 |
| 2m | 20.52 | 34.26 | 41.13 | 44.56 |

| TSSB | SSB/SIB1/SIB19 periodicity |  |  |  |
| --- | --- | --- | --- | --- |
|  | 20ms | 40ms | 80ms | 160ms |
| 4 Symbols for 1 SSB | 34.22 | 41.10 | 44.56 | 46.28 |
| 8 Symbols for 2 SSB | 31.94 | 39.97 | 43.98 | 45.99 |
| 16 Symbols for 4 SSB | 27.38 | 37.69 | 42.84 | 45.42 |

- Results for LEO600km Set1-2 FR1

For traffic FTP 3 with packet size = 0.5 Mbyte and 200ms as mean inter-arrival time, the results are listed below,

Average SE (bps/Hz/ beam footprint) for the proposed beam hopping design

| TSSB | SSB/SIB1/SIB19 periodicity |  |  |  |
| --- | --- | --- | --- | --- |
|  | 20ms | 40ms | 80ms | 160ms |
| 0.5ms | / | / | 0.1164 | 0.1832 |
| 1ms | / | / | 0.0819 | 0.1659 |
| 2m | / | / | 0.0819 | 0.1659 |

| TSSB | SSB/SIB1/SIB19 periodicity |  |  |  |
| --- | --- | --- | --- | --- |
|  | 20ms | 40ms | 80ms | 160ms |
| 4 Symbols for 1 SSB | / | 0.0122 | 0.1312 | 0.1906 |
| 8 Symbols for 2 SSB | / | / | 0.1115 | 0.1807 |
| 16 Symbols for 4 SSB | / | / | 0.4564 | 0.2032 |

For traffic VoIP traffic (AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) and 20 ms data arriving interval), the results are listed below,

Average throughput performance (Kbps) of UE for the proposed beam hopping design

| TSSB | SSB/SIB1/SIB19 periodicity |  |  |  |
| --- | --- | --- | --- | --- |
|  | 20ms | 40ms | 80ms | 160ms |
| 0.5ms | / | / | 390.94 | 615.47 |
| 1ms | / | / | 275.10 | 557.51 |
| 2m | / | / | 275.10 | 557.51 |

| TSSB | SSB/SIB1/SIB19 periodicity |  |  |  |
| --- | --- | --- | --- | --- |
|  | 20ms | 40ms | 80ms | 160ms |
| 4 Symbols for 1 SSB | / | 41.08 | 440.58 | 640.25 |
| 8 Symbols for 2 SSB | / |  | 374.39 | 607.15 |
| 16 Symbols for 4 SSB | / |  | 1533.00 | 682.84 |

- Results for LEO600km Set1-3 FR1

For traffic FTP 3 with packet size = 0.5 Mbyte and 200ms as mean inter-arrival time, the results are listed below,

Average SE (bps/Hz/ beam footprint) for the proposed beam hopping design

| TSSB | SSB/SIB1/SIB19 periodicity |  |  |  |
| --- | --- | --- | --- | --- |
|  | 20ms | 40ms | 80ms | 160ms |
| 0.5ms | 0.0264 | 0.0327 | 0.0359 | 0.0264 |
| 1ms | 0.0232 | 0.0311 | 0.0351 | 0.0232 |
| 2m | 0.0167 | 0.0279 | 0.0334 | 0.0167 |

| TSSB | SSB/SIB1/SIB19 periodicity |  |  |  |
| --- | --- | --- | --- | --- |
|  | 20ms | 40ms | 80ms | 160ms |
| 4 Symbols for 1 SSB | 0.0278 | 0.0334 | 0.0362 | 0.0265 |
| 8 Symbols for 2 SSB | 0.0260 | 0.0325 | 0.0358 | 0.0234 |
| 16 Symbols for 4 SSB | 0.0223 | 0.0307 | 0.0348 | 0.0170 |

For traffic VoIP traffic (AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) and 20 ms data arriving interval), the results are listed below,

Average throughput performance (Kbps) of UE for the proposed beam hopping design

| TSSB | SSB/SIB1/SIB19 periodicity |  |  |  |
| --- | --- | --- | --- | --- |
|  | 20ms | 40ms | 80ms | 160ms |
| 0.5ms | 31.19 | 38.62 | 42.34 | 31.19 |
| 1ms | 27.35 | 36.70 | 41.38 | 27.35 |
| 2m | 19.68 | 32.87 | 39.46 | 19.68 |

| TSSB | SSB/SIB1/SIB19 periodicity |  |  |  |
| --- | --- | --- | --- | --- |
|  | 20ms | 40ms | 80ms | 160ms |
| 4 Symbols for 1 SSB | 32.83 | 39.44 | 42.75 | 31.33 |
| 8 Symbols for 2 SSB | 30.64 | 38.35 | 42.20 | 27.61 |
| 16 Symbols for 4 SSB | 26.26 | 36.16 | 41.10 | 20.06 |

Appendix B. Link level simulation parameters for Section 2.3

Table B1. Link-level simulation assumptions for PDCCH transmission

| Parameters | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| System bandwidth | 5 MHz, 25RBs |
| SCS | 15kHz |
| Antenna configuration | 1T2R |
| Aggregation level | 8 |
| CORESET size | 2 symbols, 24 PRBs |
| Channel model | NTN-TDL-C(LOS) |
| UE speed | 3km/h |
| Payload | 40/36/32/30/24/20 bits |
| Modulation order | QPSK |
| BLER | 1% BLER |
| Elevation angle | 30 for LEO-600 |
| Frequency offset | 0.1ppm |

Table B2. Link-level simulation assumptions for PDSCH Msg4 transmission

| Parameters | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| System bandwidth | 5 MHz, 24RBs |
| Waveform | CP-OFDM |
| SCS | 15kHz |
| Antenna configuration | 1T2R |
| Allocated RBs | 24 PRBs |
| Symbol allocation | 14 OS |
| DMRS configuration | 2 DMRS symbols for Msg4 |
| Mapping type | Mapping type A |
| Repetition number | Repetition type A, 1/2/4 repetition |
| Channel model | NTN-TDL-C(LOS) |
| HARQ | NO |
| UE speed | 3km/h |
| TBS/MCS | 1040 bits for Msg4 |
| Modulation order | QPSK |
| BLER | 10% iBLER |
| Elevation angle | 30 for LEO-600 |
| Frequency offset | 0.1ppm |

Appendix C. Simulation parameters for obtaining Figure 2

| Parameters | Value |
| --- | --- |
| Scenario | RMA |
| Satellite orbit configuration | LEO600km |
| Service link frequency | S-band (2GHz) |
| Channel bandwidth | 5MHz |
| 3 dB beam width | 4.4127 deg |
| Satellite power per simultaneously active beam | 21dBWfor Set1-1, 29dBW for Set1-2, 13dBWf or Set1-3 |
| Satellite antenna gain | 30 dBi, consider 4dB steering loss |
| Satellite G/T | 1.1 dB/K |
| Satellite polarization configuration | Circular |
| Satellite antenna pattern | See section 6.4.1 in TR38.811: Bessel function |
| UE configuration | Handheld |
| UE Antenna type and configuration | Omni-directional; [M N P Mg Ng; Mp Np]=[1 1 2 1 1;1 1] |
| UE antenna polarisation | Linear: ±45°X-pol |
| UE Antenna gain (dBi) | -5.5dBi |
| UE Antenna temperature (K) | 290 |
| Noise figure (dB) | 7 |
| UE Tx transmit power | 200 mW (23 dBm) |
| Handover Margin | 0dB |
| UE attachment | RSRP |
| Channel model | Large scale model and small scale model |
| Metrics | SINR |

Appendix D. Simulation parameters for obtaining Table y

Link-level simulation assumptions for PDCCH transmission

| Parameters | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| System bandwidth | 5 MHz, 25RBs |
| SCS | 15kHz |
| Antenna configuration | 1T2R |
| Aggregation level | 8 |
| CORESET size | 2 symbols, 24 PRBs |
| Channel model | NTN-TDL-C(LOS) |
| UE speed | 3km/h |
| Payload | 40/32/24/16/12/8 bits + CRC24 32/24/16/12/8 bits + CRC16 |
| Modulation order | QPSK |
| BLER | 1% BLER |
| Elevation angle | 30 for LEO-600 |
| Frequency offset | 0.1ppm |