3GPP TSG RAN WG1 #119		        R1-2409800

Orlando, US, November 18th – 22nd, 2024

Agenda Item:	9.4.1.2

Source:	Apple

Title:	On remaining details of device architecture for AIoT

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide our views on the remaining clock purposes and their corresponding details including their applicability to different device types.

# Discussion

## Clock Functions

In RAN1#118bis, following agreement related to clock functionalities in the device have been made in [1]:

Agreement

- Adopt following updated table format.

- One or more rows under a single purpose may exist to capture different options, if any.

| # | Purpose | Clock speed | Applicable Device types | Power consumption | Initial clock Accuracy (ppm) | Accuracy after clock sync / calibration at device side (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |  |

- Agreement

- For clock purpose #3, adopt following update.

| # | Purpose | Clock speed | Applicable Device types | Power consumption | Initial clock Accuracy (ppm) | Accuracy after clock sync / Calibration at device side (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| [ #3 ] | [ Time counting (if supported) ] | e.g. tens kHz | 1 (if applicable) 2a, 2b | <0.1uW | [10^4 - 10^5] | Calibration may not always be feasible for this clock purpose |
|  |  |  | 2a, 2b | <0.1uW | [10^3-10^4] | FFS N/A |

- Agreement

- For clock purpose #5, adopt following update.

| # | Purpose | Clock speed | Applicable Device types | Power consumption | Initial clock Accuracy (ppm) | Accuracy after clock sync / calibration at device side (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| #5 | LO for carrier frequency (for up/down conversion) | According to carrier frequency e.g., 900 MHz | 2b | 10s ~ 100s uW | [10^23 ~ 10^4] | Option 1: 10s (Note2) Option 2: 100s (Note2) |
| Note 2: Either internal clock counter for calibration clock counting based on at least R2D signals or clock counting based on transmission from reader Calibration is based on existing or new signal from reader. FFS how to calibrate (9.4.2.2). No drastic temperature change is assumed during calibration. |  |  |  |  |  |  |

- Further study above options and strive to down select considering its implication on performance, system design, necessity, etc.

- Note: study of whether additional signal is necessary or not for calibration is discussed under agenda 9.4.2.2.

- Agreement

- For clock purpose #4, adopt following update.

| # | Purpose | Clock speed | Applicable Device types | Power consumption | Initial clock Accuracy (ppm) | Accuracy after clock sync / calibration at device side (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| #4 | Large frequency shift (if supported for device 2a) | 10s MHz (e.g., 50MHz) | 2a | [10s] uW | [10^3-10^4] | [10^3] (Note 2) |
| Note2: Digital clock counter can be assumed for calibration clock counting based on R2D signals how to calibrate. No drastic temperature change is assumed during calibration. |  |  |  |  |  |  |

For clock purpose 1, for sampling, one key remaining issue is on the power consumption for such clocks and whether the same requirement can be applied for device 1, device 2a/2b. In our view, since the sampling function is commonly applied to all devices with similar clock speed of few MHz, therefore, the power consumption requirement could be same regardless of device type. However, this is also coupled with the initial clock accuracy. With more sophisticated clock and with potentially higher power consumption, i.e. > 1uW, better SFO, i.e. with lower PPM can be considered. As a reference, we can consider, as considered for evaluation purpose as well, 10^4-10^5 ppm for device 1 and 10^3-10^4 ppm for device 2a/2b. Based on some literature review, it is realistic to support sampling function with a clock speed of few MHz and a power consumption requirement well under 1uW , as shown in [2], where 1.52MHz clock speed is generated and a power consumption is 320nW is observed. Additionally, it needs to be discussed whether it is feasible to improve clock accuracy after clock sync/calibration. For device 1, it may not be a realistic assumption that there can be any significant improvement in the clock accuracy considering very low power consumption requirement. However, for device 2a/2b, we can assume an improvement, by a factor of 10.

Proposal 1: For purpose#1 of the clock, i.e. sampling, a single value of power consumption < 1uW can be supported considering the common requirement in terms of clock speed for all device types

Proposal 2: For purpose#1 of the clock, i.e. sampling, following initial clock accuracy can be considered:

- Device 1: 10^4-10^5 ppm

- Device 2a/2b: 10^3-10^4 ppm

Proposal 3: For purpose#1 of the clock, i.e. sampling, following clock accuracy after calibration can be considered:

- Device 1: Not applicable

- Device 2a/2b: 10^3 ppm

For clock purpose 2, first and foremost discussion is the applicability to device types. Since this is mainly meant for small frequency shift for backscattering devices, we think that this purpose is defined only for device 1 and device 2a. For device 2b, it has its own carrier frequency generator, therefore, it doesn’t need additional clock for the purpose for FDMA, for which small frequency shift is needed for device 1 and device 2a. Then in terms of the clock speed, the typical requirement for FDMA is only up to few 10s of kHz within the system bandwidth. In terms of power consumption, since the requirements are much less compared to the clock purpose#1, we can assume that the power consumption requirement in this case would also be less than 1uW. In terms of the clock accuracy, similar range as for clock purpose 1 can be assumed. However, for clock accuracy after calibration, we don’t think it is applicable in this scenario.

Proposal 4: For purpose#2 of the clock, i.e. small frequency shift, a single value of power consumption < 1uW can be supported considering the common requirement in terms of clock speed for all device types

Proposal 5: For purpose#2 of the clock, i.e. small frequency shift, following initial clock accuracy can be considered:

- Device 1: 10^4-10^5 ppm

- Device 2a: 10^3-10^4 ppm

Proposal 6: For purpose#2 of the clock, i.e. small frequency shift, following clock accuracy after calibration can be considered:

- Device 1: Not applicable

- Device 2a: Not applicable

# Conclusion

In this contribution, we made the following observations/proposals on clock purposes and their corresponding details

Proposal 1: For purpose#1 of the clock, i.e. sampling, a single value of power consumption < 1uW can be supported considering the common requirement in terms of clock speed for all device types

Proposal 2: For purpose#1 of the clock, i.e. sampling, following initial clock accuracy can be considered:

- Device 1: 10^4-10^5 ppm

- Device 2a/2b: 10^3-10^4 ppm

Proposal 3: For purpose#1 of the clock, i.e. sampling, following clock accuracy after calibration can be considered:

- Device 1: Not applicable

- Device 2a/2b: 10^3 ppm

Proposal 4: For purpose#2 of the clock, i.e. small frequency shift, a single value of power consumption < 1uW can be supported considering the common requirement in terms of clock speed for all device types

Proposal 5: For purpose#2 of the clock, i.e. small frequency shift, following initial clock accuracy can be considered:

- Device 1: 10^4-10^5 ppm

- Device 2a: 10^3-10^4 ppm

Proposal 6: For purpose#2 of the clock, i.e. small frequency shift, following clock accuracy after calibration can be considered:

- Device 1: Not applicable

- Device 2a: Not applicable

# References

- 3GPP RAN1#118bis, “Chairman Notes”

- R. Barnett and J. Liu, "A 0.8V 1.52MHz MSVC Relaxation Oscillator with Inverted Mirror Feedback Reference for UHF RFID," IEEE Custom Integrated Circuits Conference 2006, San Jose, CA, USA, 2006, pp. 769-772, doi: 10.1109/CICC.2006.320975.