3GPP TSG RAN WG1 #119			R1-2409827

Orlando, US, November 18th – 22nd, 2024

Agenda Item:	9.11.5

Source:	Apple

Title:	Discussion on IoT-NTN TDD mode

Document for:	Discussion/Decision

# Introduction

In the RAN #105 meeting, the WID on introduction of IoT-NTN TDD mode was agreed in [1]. The objective includes the periodic pattern impacts on current NB-IoT FDD NTN operation. The following agreements were made in RAN1#118bis meeting [2].

Agreement The target operating DL CNR of NB-IoT NTN TDD is obtained following the parameters in TR 36.763 and modifying the carrier frequency to 1.6GHz: - For LEO-600, the operating DL CNR is 4.91dB (i.e. with 0dBi UE antenna gain) - For LEO-1200, the operating DL CNR is 5.51dB (i.e. with 0dBi UE antenna gain) - When reporting link level simulation results, companies are encouraged to report the CNR margin with respect to the above operating DL CNRs. - Companies may report values for other scenarios (e.g. UE antenna gain of -5.5 dBi, LEO-800) with the corresponding assumptions. Agreement For the study phase, RAN1 assumes as a baseline for evaluations (which includes DL synchronization as per the WID) that downlink PHY channels and signals are transmitted on the anchor carrier following subframe index/location and SFN mapping of NB-IoT NTN FDD. - For the purpose of study, the DL subframes of the Rel-19 TDD pattern may not fully cover all the instances of the PHY channels and signals and eNB will not transmit the corresponding instances. - FFS: What is the TDD pattern Agreement RAN1 evaluates via link level simulations the following: - NPSS/NSSS detection - NPBCH decoding Agreement The impact of the TDD pattern at least on the following channels / signals are evaluated at least qualitatively (e.g. specification impact, number of available instances, etc.): - SIB1-NB - Other SIBs needed for basic NB-IoT NTN operation (e.g. SIB2, SIB31) - Paging - NPRACH / RAR - NPSS/NSSS/NPBCH - NPDCCH, NPUSCH, NPDSCH - Note: Study for the above channels / signals may continue during the normative phase. - It is up to companies to perform link level evaluations for some of the aspects above Agreement For link level simulations of NPSS / NSSS / NPBCH, RAN1 uses the following assumptions: Parameter; Value. Carrier frequency; 1.6GHz. Channel model; NTN TDL-C rural, 30 degrees. Frequency error / timing drift (including XO error); 34 ppm for NPSS / NSSS (24ppm Doppler + 10ppm XO error) 0.1ppm for NPBCH. Variation of frequency error / variation of timing drift; 0.27ppm/s for NPSS / NSSS 0 for NPBCH. UE velocity; 3km/h. Target performance; For NPSS/NSSS: - 99% detection probability with 0.1% false alarm rate - FFS: How to define correct detection, including successful timing / frequency estimation / cell ID detection. For NPBCH: - 1% BLER For the channels / signals above, acquisition delay shall be reported for the corresponding detection probability. Other assumptions (e.g. combining length, frequency error hypothesis, etc.); To be reported and justified by companies. Agreement For the study phase, RAN1 evaluates the following combination of N (in radio frames) and D (D = number of consecutive downlink subframes): - N = 9: - D = 8 - D = 20 - D = 30 - N = 8: - D = 20 - Companies to report which subframe indices are included in the set of consecutive downlink subframes. - Other combinations are not precluded to be reported by companies NOTE: This does not imply that the above combinations are the only ones to be considered during the normative phase. Agreement For the study phase, RAN1 evaluates the following combination of N (in radio frames) and U (U = number of consecutive uplink subframes): - N = 9: - U = 8 - U = 20 - U = 30 - N = 8: - U = 20 - Companies to report which subframe indices are included in the set of consecutive uplink subframes. - Other combinations are not precluded to be reported by companies - For evaluations including both uplink and downlink (if any), companies are encouraged to prioritize the case of D=U NOTE: This does not imply that the above combinations are the only ones to be considered during the normative phase. Agreement For the study phase, RAN1 assumes as a baseline for evaluations that uplink PHY channels are transmitted on the anchor carrier following subframe index/location and SFN mapping of NB-IoT NTN FDD. - For the purpose of study, the UL subframes of the Rel-19 TDD pattern may not fully cover all the instances of the PHY channels and eNB will not receive the corresponding instances. - FFS: What is the TDD pattern

In this contribution, we provide our considerations on this new IoT-NTN TDD mode.

# Discussion

According to the WI description, it targets for the NB-IoT FDD NTN operating in unpaired frequency band and tries to re-use existing IoT-NTN design. To reduce the power consumption from the satellite, half-duplex operation from network side is assumed. It’s evident that the DL and UL throughputs will decrease. For IoT-NTN TDD mode study phase, one aspect is the DL transmission pattern, especially the NPSS/NSSS/NPCH transmission periodicity, UE would monitor these signal/channel to get the DL synchronization and system frame number. According to the current NB-IoT specification,

- NPSS is transmitted every 10ms in subframe#5.

- NSSS is transmitted every 20ms in subframe#9. Four NSSS sequences with different cyclic shifts are transmitted in 80ms.

- NPBCH is transmitted every 10ms in subframe#0. NPBCH consists of 8 independently decodable blocks spanning 80ms, MIB content remains unchanged for 640ms.

- SIB1-NB is transmitted with a fixed periodicity of 2560ms, SIB1-NB transmission occurs in subframe#4 of every other frame in 16 continuous frames, and supports 4, 8, and 16 repetitions within 2560ms periodicity.

The NPSS, NSSS and NPBCH transmission pattern are shown in Figure 1 and Figure 2.

| Figure 1: NPSS/ NSSS/NPBCH/SIB1-NB transmission pattern | Figure 2: NPBCH transmission pattern |
| --- | --- |

As indicated by WID and the agreements in last RAN1 meeting, to be compatible with in orbit satellite system, there is one DL transmission opportunity every 90 subframes, i.e., N=9, for IoT-NTN. One DL transmission opportunity could be 8ms, 20ms or 30ms. The number of continuous DL subframe will impact the compatible design with satellite and the DL throughput. To compromise the DL overhead and throughput, it’s the middle way to allow NPBCH/NPSS/NSSS/SIB1-NB transmitted in 8 subframes, i.e., within one satellite slot, in every 90ms periodicity.

Figure 3: subframe indices within consecutive downlink subframes.

Proposal 1: NPBCH/NPSS/NSSS/SIB1-NB are transmitted in 8 continuous subframes in very 90ms periodicity.

After UE detects the NPBCH/NPSS/NSSS, it could synchronize to the network, then detects the SIB-NB information. To boost the DL performance and assign the UL resources, SIB1-NB could contain the information of additional DL subframe configuration and UL subframe configuration in a 90ms periodicity. The configuration shall be compatible with the with satellite frame structure. Thus, the UE could derive the Rel-19 TDD pattern implicitly.

Proposal 2: Additional DL subframes and UL subframes are configurable by SIB1-NB.

For NB-IoT downlink control channel, to support IoT-NTN TDD mode, NPDCCH search space configurations, e.g., repetition number and periodicity, are restricted to adapt the periodic transmission pattern.

For NB-IoT downlink data channel, the repetition number and scheduling delay could be impacted by the IoT NTN TDD mode. If the scheduled NPDSCH is out of downlink subframes indicated by the periodic transmission pattern, UE behavior needs to be specified.

For NB-IoT uplink data channel, 15kHz and 3.75kHz SCS are supported. For the typical single tone transmission, one RU occupies 32ms for 3.75KHz SCS and 8ms for 15kHz SCS. The NPUSCH transmission will take longer time considering repetition and scheduling delay. The IoT-NTN TDD mode will restrict the NPUSCH scheduling. Similar as downlink, the NPUSCH out of uplink subframes of periodic transmission pattern need to be considered.

Observation 1: IoT-NTN TDD mode impacts on DL and UL scheduling.

In current specification, NPRACH periodicity is in the range of {40, 80, 160, 240, 320, 640, 1280, 2560}ms. One PRACH transmission will take 6.4ms for PRACH format 0 and 5.6ms for PRACH format 1. These factors shall be considered in the periodic transmission pattern design.

After the PRACH preamble transmission, the UE would start to monitor RA response window from the end of the last preamble repetition plus a fix subframes plus UE-eNB RTT. Currently the window size is multiple of PDCCH period. It would require periodic transmission pattern to cover the RA response window.

Observation 2: IoT-NTN TDD mode impacts on random access.

# Conclusion

In this contribution, we provided our views on IoT-NTN TDD mode. Our observation and proposals are as follows:

Proposal 1: NPBCH/NPSS/NSSS/SIB1-NB are transmitted in 8 continuous subframes in very 90ms periodicity.

Proposal 2: Additional DL subframes and UL subframes are configurable by SIB1-NB.

Observation 1: IoT-NTN TDD mode impacts on DL and UL scheduling.

Observation 2: IoT-NTN TDD mode impacts on random access.

# References

- RP-242415, “New WID on introduction of IoT-NTN TDD mode” Sep. 2024

- R1-2409341, “Report of RAN1#118bis meeting”, Oct. 2024