3GPP TSG RAN WG1 #119			R1-249899

Orlando, US, November 18th – 22nd, 2024

Source: 	     Xiaomi

Title:	Discussion on downlink and uplink channel and signal aspects for Ambient IoT

Agenda item: 	9.4.2.3

Document for:  Decision

Introduction

In RAN#103 plenary meeting, the SID on solutions for Ambient IoT has been revised as [1], including the following objectives which are led by RAN1:

General Scope The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope: - The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices: - ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally. - X is to be decided in WGs. - Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”. - For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ. NOTE 1: It is to be understood that “≤ a few hundred µW” means WGs are not tasked with setting a particular value, and that it will be for WG discussions to determine if a presented design with corresponding power consumption satisfies the “≤ a few hundred µW” requirement. - Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848: - Deployment scenario 1 with Topology 1 - Basestation and coexistence characteristics: Micro-cell, co-site - Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control - Basestation and coexistence characteristics: Macro-cell, co-site - The location of intermediate node is indoor - FR1 licensed spectrum in FDD. - Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s). - Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command). - From RAN#104, the study will assess whether the harmonized air interface design (per bullet ‘A’ above) can address the DO-A (Device-originated autonomous) use case, only to identify which part(s) of the harmonized air interface design (per bullet ‘A’ above) is/are not sufficient for the DO-A use case. Transmission from Ambient IoT device (including backscattering when used) can occur at least in UL spectrum. The following objectives are set, within the General Scope: Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination, which is the determination of whether BS or intermediate UE and ambient IoT device are near each other or not (coordination with SA3 is required for privacy aspects). - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1.

This contribution discusses on further design of the A-IoT channel/signal including the aspects of general R2D and D2R channel/signal design, time/frequency domain resource.

R2D channel/signal

In RAN1#117 meeting, for R2D transmission, it was agreed that only one physical channel (PRDCH) is studied as follow.

Agreement For R2D, the only physical channel is PRDCH. - PRDCH carries any higher-layer payload - PRDCH carries L1 R2D control information (if defined) - FFS details of device behaviour(s) for receiving PRDCH

Additionally, in RAN1#118 meeting, the following agreements was achieved:

Agreement For D2R scheduling, the following information potentially can be explicitly/implicitly indicated to the device via corresponding PRDCH: - Time domain resources - Frequency domain resources - MCS-like information - Chip duration - ID associated with device(s) - Repetitions FFS: other information FFS: For each information, whether higher-layer signaling and/or L1 R2D control signaling is used Agreement For R2D reception, the following information potentially can be explicitly/implicitly indicated to the device via PRDCH: - ID associated with device(s) intended for the reception of R2D, potentially including all devices (if supported) - FFS: other information FFS: For each information, whether higher-layer signaling and/or L1 R2D control signaling is used

Regarding the FFS part, one further issue is how to convey the above control information in PRDCH, to avoid the cross-layer interaction and additional latency, we prefer to use L1 R2D control signalling. Moreover, it should be transmitted adjacently in front of the R2D data, and the payload size of control should be fixed and pre-defined by the specification to avoid high complexity of blind decoding.

Figure 1 Illustration of L1 R2D control signalling

Proposal 1:  L1 R2D control signalling is used to carry the information for D2R scheduling/R2D reception, which can be explicitly/implicitly indicated to the device via PRDCH.

Another aspect is whether other information is needed to be indicated to the device via corresponding PRDCH, as described in [2], we propose to support midamble. The location/length/pattern of midamble can be indicated dynamically by the control information carried by the corresponding R2D transmission.

Proposal 2: For D2R scheduling, the following information can also be explicitly/implicitly indicated to the device via corresponding PRDCH:

- The location/length/pattern of midamble.

In RAN1#118bis meeting, the following agreement was further reached w.r.t the PRDCH design details.

Agreement For the PRDCH design details, following is captured in the TR 38.769: Following design options have been studied for PRDCH: - Option 1: If there is no L1 R2D control information carried by PRDCH, then PRDCH transmission with only R2D data is considered - Note: In this option, R2D control information (if any) maybe carried by higher-layer signaling - Option 2: If any L1 R2D control information is supported & carried by PRDCH, then for the CRC attachment (appended if there is non-zero length CRC), following two options have been studied - Option 2-1: Joint CRC is attached to L1 R2D control information and R2D data - Option 2-2: Separate CRC is attached to L1 R2D control information and R2D data, respectively

According to the agreements above, there can be three formats of R2D transmission, format 0 only includes the L1 control information, e.g., when there is no corresponding R2D packet to be transmitted, and only a D2R scheduling would be performed; in format 1, there is only R2D data and no D2R scheduling would be performed (e.g., for R2D command use case), and the transmission parameters of the PRDCH are pre-defined or implicitly indicated (e.g., by postamble); format 2 includes both L1 control information and R2D data.

Proposal 3: For PRDCH, there may be three formats can be studied:

- Format 0: L1 control information only;

- Format 1: R2D data only;

- Format 2: L1 control information+ R2D data.

Based on the above analysis, device needs to know whether the PRDCH includes the L1 control information in advance before the decoding process, and then to decide whether it is necessary to decode the L1 control information first. As aforementioned, the control information is carried by a PHY header whose size is pre-defined. For this purpose, it can be considered that the first two bits are reserved for the PRDCH format indication.

Proposal 4: The first two bits of PRDCH transmission can be used for the PRDCH format indication.

Figure 2 Illustration of PRDCH format indicator

For R2D transmission, for the timing acquisition, a preamble is agreed to be added immediately before an R2D transmission, which is similar to RFID.  The following agreement has been achieved w.r.t the R2D preamble during RAN1#116bis meeting:

Agreement For the R2D timing acquisition signal immediately preceding the transmission of a physical channel, study a preamble with at least two parts which includes a start-indicator part and a clock-acquisition part, where the start-indicator part immediately precedes the clock-acquisition part: - Start-indicator part provides the start of the R2D transmission - FFS: Details of start-indicator part - Clock-acquisition part provides at least the chip synchronization of the subsequent physical channel transmission - FFS: Details of clock-acquisition part, e.g. structure, encoding, length, etc. - FFS: Methods to determine chip duration of the subsequent physical channel transmission - FFS: Other functionalities - Note: the preamble is considered not to be part of a physical channel - FFS: other part(s) of the preamble, if any - FFS: whether the above clock acquisition is sufficient for all devices QC - FFS: how to make the preamble compact

Based on the above agreement, from our perspective, an R2D preamble may include the following segments similar to RFID, and the details would be:

- Start-indicator part;

- Clock-acquisition part (including R2D time acquisition signal, and the potential D2R time acquisition signal).

Regarding the Start-indicator part, the following agreement has been achieved in previous meetings:

Agreement For the start-indicator part of the R2D time acquisition signal, ON/OFF pattern i.e. high/low voltage transmission is applied - FFS: length/pattern of ON/OFF. FFS: when TD2R_min is applicable, whether/how the start-indicator part is included in TD2R_min or not. To be discussed in 9.4.2.2 Agreement For the start-indicator part of the R2D time acquisition signal, for providing the start of the R2D transmission, following is captured in the TR 38.769: - Following options have been studied for the start-indicator part of the R2D time acquisition signal: - Option 1: ON-OFF transmission is considered based on energy/edge detection, and multiple alternatives have been studied including - Alt 1: A single ON-OFF transmission, i.e. one high-voltage transmission followed by one low-voltage transmission, where ON and OFF may have same or different durations - Alt 2: A multi-ON-OFF transmission, where different ON and different OFF may have same or different durations and different parts may have same or different duration - Option 2: ON-OFF sequence-based design is considered which consists of a pre-defined sequence for detection of start-indicator part based on digital correlation - For both the options, it is observed that a fixed duration for the start-indicator part can be considered, regardless of the value of M used for PRDCH transmissions.

A fixed length of high voltage followed by a fixed length of low voltage (i.e., Option 1) is feasible and preferrable because monitoring a decrease in voltage level consumes less power than monitoring a ON-OFF sequence-based pattern, which can provide more benefits on power consumption reduction. In addition, device (especially for device 1) may not be capable to support correlation operation, which makes Option 2 unfeasible while it is based on sequence detection.  Furthermore, b/w the two alternatives under Option 1, it may depend on deployment and scenario, in which device may or may not have the ability to distinguish the “high” and “low” voltage without an explicit example. More details under Option 1 can be further discussed and the final decision can be made in WI phase. Then, we propose:

Proposal 5: ON-OFF transmission based on energy/edge detection (i.e., Option 1) should be baseline of Start-indicator part of the R2D time acquisition signal of.

- More details under Option 1 can be further discussed and the final decision b/w Alt 1 and Alt 2 can be made during WI phase.

For the R2D Clock-acquisition part of the R2D time acquisition signal, more details are provided in last meeting.

Agreement For the clock-acquisition part of the R2D time acquisition signal, following is captured in the TR 38.769: - Clock-acquisition part is based on OOK without line coding and includes rising/falling edges, including at least two rising or at least two falling edges for the device to determine the OOK chip duration Agreement For the clock-acquisition part of the R2D time acquisition signal for OOK chip duration determination, following options are studied: - Option 1: Duration of the clock-acquisition part is variable for different M values, i.e. the duration becomes shorter with increasing value of M - Option 2: Duration of the clock-acquisition part is constant for different M values based on repetition, i.e. repetition factor is increased with increasing value of M to keep the duration constant - FFS: Whether/what restriction on M values for the clock-acquisition part Note: Other functionalities of clock-acquisition part is a separate discussion

In RFID, the chip number of clock-acquisition part is fixed, which is not variable along with the chip duration length changing. A-IOT devices should achieve similar performance as RFID, that is, it is unnecessary to increase the repetition factor as M increases. Moreover, the value of M should be consistent for the clock acquisition part and the subsequent PRDCH data part for more accurate clock calibration.

Proposal 6: For the clock-acquisition part of the R2D time acquisition signal, Option 1 should be supported in which duration of the clock-acquisition part is variable for different M values, i.e., the duration becomes shorter with increasing value of M.

- The value of M should be consistent for the clock acquisition part and the subsequent PRDCH data part.

Besides the above part, D2R Clock-acquisition part can also include D2R time acquisition signal, which is similar to TRcal in RFID. This signal can be used by the device to measure the length, to compute the D2R transmission frequency and data rate, and then adjust its D2R working frequency and data rate to be equal to the obtained value.

Proposal 7: Clock-acquisition part in the R2D preamble can additionally include D2R time acquisition signal for the device to compute its D2R transmission frequency and data rate.

In summary, the design details of R2D preamble can be illustrated by the following figure 3.

Figure 3 Illustration of preamble design for A-IoT R2D

During the RAN1#116bis meeting, postamble is agreed to be an option for determining or deriving the end of PRDCH, it can also be used for timing maintenance and SFO tracking. An R2D postamble cannot be decoded as data, and it can include the following segments:

- End-indicator part: It can be a certain time of contiguous logic high/low electrical level to indicate the R2D transmission which will be ended immediately when it has been identified by the device. To differentiate the preamble and postamble, the electrical level of preamble and postamble needs to be set as opposite.

Figure 4 Illustration of postamble design for A-IoT R2D

Proposal 8: R2D postamble is immediately transmitted after any PRDCH transmission.

- An End-indicator part (a certain time of contiguous logic high/low electrical level) may be included in a R2D postamble.

D2R channel/signal

During the RAN1#117 meeting, the following agreement has been achieved, in which the CSI feedback and Autonomous SR are not considered for the L1 D2R control information study.

Agreement For L1 D2R control information (if defined), the following are not considered for further study: - CSI feedback - Autonomous SR - FFS: Whether any other L1 D2R control information is needed or not

One remaining issue is that whether other D2R control information (if defined) is transmitted on the PDRCH. According to the SID, HARQ-ACK feedback for triggering R2D retransmission is not supported in A-IoT, which means that confirmatory response to R2D command is not conveyed as the physical layer HARQ-ACK feedback. Another way is to use L1 D2R control information to convey the confirmatory response for the R2D command, however, it requires much more cross-layer interactions and thereby extra delays, which is not preferred.

Proposal 9: HARQ-ACK for triggering R2D retransmission is not supported in A-IoT as L1 D2R control information.

Proposal 10: The confirmatory response to R2D command should be included in higher-layer payload.

Another aspect is whether the L1 D2R control information can be used for acquiring the end of PDRCH transmission, the related agreement has been shown as below:

Agreement For the reader to acquire the end of PDRCH transmission, study at least following options: - Option 1: D2R postamble immediately follows the PDRCH - Option 2: Based on control information

As stated in [2], postamble is needed due to beneficial for SFO mitigation and channel estimation, thus it is sufficient to indicate the end of PDRCH transmission by postamble. It is not necessary to carry the PDRCH end indicator in the L1 D2R control information regardless whether the reader can predict the payload size of the corresponding D2R response.

Proposal 11: It is not necessary to carry the PDRCH end indicator in the L1 D2R control information.

- The reader can acquire the end of PDRCH transmission by D2R postamble immediately following the PDRCH.

Based on the analysis above about PDRCH transmissions, the relative aspects of PDRCH can be indicated by sequences rather than explicit control information. Considering the discussion and study progress, any of L1 D2R control information has not been defined. It is not necessary to further discuss or define it.

Proposal 12: It is not necessary to introduce any L1 D2R control information in A-IoT.

In D2R direction, there can be two kinds of transmissions in A-IoT. A shorter one is used as temporary identification of a device (e.g., similar to RN16 in RFID); the other one is D2R data (such as device ID or the temperature in the use case of Sensor) transmission, which can be much longer than the former one. Whether separate PDRCH formats are needed to be designed for these two kinds of transmission can be further studied in RAN1.

Figure 5 Illustration of two types of D2R data channel formats

Proposal 13: One or two potential formats can be defined for PDRCH in A-IoT:

- If only one format is defined, it is used for both the transmission of random temporary binary stream (RN16 like) and other D2R data (e.g., device ID and the potential subsequent D2R data transmission);

- If two formats are defined, the following two formats can be considered:

  - A short format is used for the transmission of the random temporary binary stream (RN16 like);

  - A long format is used for the transmission of other D2R data (e.g., device ID and the potential subsequent D2R data transmission).

During the previous meetings, the following agreement has been reached in which the purpose for studying preamble preceding a PDRCH transmission has been listed:

Agreement For D2R transmission, preamble preceding the PDRCH is studied also for the potential additional functionalities: - SFO estimation - CFO estimation - Channel estimation - Interference estimation Note: this does not preclude studying the above functionalities by using a midamble and/or postamble, if supported FFS: Other functionalities, if any Agreement For the D2R preamble, binary signal is considered.

Besides functionalities, the sequence design for preamble also needs to be studied in RAN1. For instance, a sequence with good autocorrelation characteristics (e.g., Golay sequence or m sequence) can be considered for the preamble since gNB and intermediate UE has the capability perform sequence auto-correlation operation. In [2], Golay sequence is used in the simulation on SFO mitigation.

Proposal 14: For the D2R preamble which is immediately transmitted before any PDRCH transmission:

- Golay sequence can be used as baseline, other sequences, e.g., m sequence or Gold sequence can be further studied.

According to [2], midamble and postamble are proposed to be supported for PDRCH transmission, then the above functionalities can be studied for midamble and postamble as well.

Proposal 15: For D2R transmission, midamble/postamble are also studied for the potential additional functionalities:

- SFO estimation;

- CFO estimation;

- Channel estimation;

- Interference estimation.

Since Ambient IoT device may suffer from the sampling frequency offset (SFO), which may have negative impacts on the D2R transmission performance. As described in [2], midamble and postamble can be studied and the sequence design is also necessary to be discussed. Postamble should be differenciated with the preamble. For example, postamble can be a complementary Golay sequence of the preamble sequence. However, for midamble, it can be same or truncated sequence of preamble.

Figure 6 Illustration for mid-amble inserted in-between the segments of a long UL transmission

Proposal 16: D2R postamble can be a complementary Golay sequence of the preamble.

- Other sequence, e.g., m sequence or Gold sequence can be further studied.

Proposal 17: D2R midamble can be a same sequence as preamble.

Time and frequency resource for R2D/D2R transmission

In Rel-19 Ambient IoT, only FR1 licensed spectrum in FDD is deployed. In topology 1, for R2D transmission (mainly for R2D command) from gNB, the transmission spectrum should naturally be FDD DL spectrum; for D2R, as mentioned in the SID, the transmission from Ambient IoT device (including backscattering when used) can at least occur in UL spectrum. Whether the D2R backscattering transmission can additionally occur in FDD DL spectrum may depend on the spectrum used for CW transmission and the device has the capability or not to shift the operating frequency.

However, in topology 2, for R2D transmission from intermediate UE, the transmission spectrum should be FDD UL spectrum, the motivation is to fulfil the spectrum regulation for a UE like sidelink transmission where the UL transmission from Ambient IoT device should be carried on the UE’s UL spectrum.

Proposal 18: For R2D transmission in A-IoT:

- In topology 1, where the R2D transmission is from gNB, FDD DL spectrum is used;

- In topology 2, where the R2D transmission is from intermediate UE, FDD UL spectrum is used.

Proposal 19: For D2R transmission from Ambient IoT device (including backscattering when used):

- At least FDD UL spectrum can be used;

- FFS: Whether UL transmission from Ambient IoT device can occur in the FDD DL spectrum.

Considerations on topology 2

For resource allocation and/or controlling of intermediate UE in topology 2, the following agreement has been made in last meeting

Agreement For the topology 2, following is captured in the TR 38.769: For resource allocation and/or controlling of intermediate UE in topology 2 for R2D and D2R transmissions via the Uu interface between the BS and intermediate UE, following two options have been studied: - Option 1: Higher-layer signaling is used - Option 2: L1 and higher-layer signaling are used From RAN1 perspective, detailed aspects on the two options are left up to work item phase. Following observations related to the two options have been made by following companies: - Four sources [Ericsson, Continental, Vivo, Mediatek] observed that option 1 can save overhead compared to option 2. - One source [Huawei] observed that the timing gap between R2D message and D2R message is not extended, and inventory rate is comparable with UHF RFID and furthermore, it is observed that with option 1, the standard impact includes no or less impact on L1 signaling for resource allocation and controlling of intermediate UE - Six sources [Ericsson, Qualcomm, Oppo, Futurewei, IIT-K, Cewit] observed that option 2 with L1 signaling provides better resource utilization compared to option 1 For the purpose of the Rel-19 study, the study on resource allocation and/or controlling of intermediate UE in topology 2 in RAN1 is considered completed based on two options above. Additional observations can be captured at RAN1#119.

For resource allocation of intermediate UE, Option 1 can provide semi-static resources but some configured resources may be wasted since the reader and device may not have the requirement to perform transmission b/w each other when the starting time of the aforementioned resources are arrived; however, if we go with Option 2, in which the intermediate UE can trigger SR/BSR report to require the resource in a on-demand way, the resources wasting issue can be avoided but more frequent signaling exchanging is needed b/w gNB and intermediate UE. Therefore, in our point of view, using DCI to allocation more than one resources in one time can be a good trade-off b/w the two options.

Observation 1: For resource allocation of intermediate UE, Option 1 can save signaling overhead b/w gNB and intermediate UE compared to Option 2 but may cause resource waste issue.

Observation 2: For resource allocation of intermediate UE, Option 2 can provide better resource efficiency compared to Option 1 but requires more signaling overhead b/w gNB and intermediate UE.

Conclusion

In this contribution, views on A-IoT channel and signal aspects were further provided. Based on the discussion, the following observations and proposals are proposed:

Observation 1: For resource allocation of intermediate UE, Option 1 can save signaling overhead b/w gNB and intermediate UE compared to Option 2 but may cause resource waste issue.

Observation 2: For resource allocation of intermediate UE, Option 2 can provide better resource efficiency compared to Option 1 but requires more signaling overhead b/w gNB and intermediate UE.

Proposal 1:  L1 R2D control signalling is used to carry the information for D2R scheduling/R2D reception, which can be explicitly/implicitly indicated to the device via PRDCH.

Proposal 2: For D2R scheduling, the following information can also be explicitly/implicitly indicated to the device via corresponding PRDCH:

- The location/length/pattern of midamble.

Proposal 3: For PRDCH, there may be three formats can be studied:

- Format 0: L1 control information only;

- Format 1: R2D data only;

- Format 2: L1 control information+ R2D data.

Proposal 4: The first two bits of PRDCH transmission can be used for the PRDCH format indication.

Proposal 5: ON-OFF transmission based on energy/edge detection (i.e., Option 1) should be baseline of Start-indicator part of the R2D time acquisition signal of.

- More details under Option 1 can be further discussed and the final decision b/w Alt 1 and Alt 2 can be made during WI phase.

Proposal 6: For the clock-acquisition part of the R2D time acquisition signal, Option 1 should be selected, i.e., Duration of the clock-acquisition part is variable for different M values, i.e., the duration becomes shorter with increasing value of M.

- The value of M should be consistent for the clock acquisition part and the subsequent PRDCH data part.

Proposal 7: Clock-acquisition part in the R2D preamble can additionally include D2R time acquisition signal for the device to compute its D2R transmission frequency and data rate.

Proposal 8: R2D postamble is immediately transmitted after any PRDCH transmission.

- An End-indicator part (a certain time of contiguous logic high/low electrical level) may be included in a R2D postamble.

Proposal 9: HARQ-ACK for triggering R2D retransmission is not supported in A-IoT as L1 D2R control information.

Proposal 10: The confirmatory response to R2D command should be included in higher-layer payload.

Proposal 11: It is not necessary to carry the PDRCH end indicator in the L1 D2R control information.

- The reader can acquire the end of PDRCH transmission by D2R postamble immediately following the PDRCH.

Proposal 12: It is not necessary to introduce any L1 D2R control information in A-IoT.

Proposal 13: One or two potential formats can be defined for PDRCH in A-IoT:

- If only one format is defined, it is used for both the transmission of random temporary binary stream (RN16 like) and other D2R data (e.g., device ID and the potential subsequent D2R data transmission);

- If two formats are defined, the following two formats can be considered:

  - A short format is used for the transmission of the random temporary binary stream (RN16 like);

  - A long format is used for the transmission of other D2R data (e.g., device ID and the potential subsequent D2R data transmission).

Proposal 14: For the D2R preamble which is immediately transmitted before any PDRCH transmission:

- Golay sequence can be used as baseline, other sequences, e.g., m sequence or Gold sequence can be further studied.

Proposal 15: For D2R transmission, midamble/postamble are also studied for the potential additional functionalities:

- SFO estimation;

- CFO estimation;

- Channel estimation;

- Interference estimation.

Proposal 16: D2R postamble can be a complementary Golay sequence of the preamble.

- Other sequence, e.g., m sequence or Gold sequence can be further studied.

Proposal 17: D2R midamble can be a same sequence as preamble.

Proposal 18: For R2D transmission in A-IoT:

- In topology 1, where the R2D transmission is from gNB, FDD DL spectrum is used;

- In topology 2, where the R2D transmission is from intermediate UE, FDD UL spectrum is used.

Proposal 19: For D2R transmission from Ambient IoT device (including backscattering when used):

- At least FDD UL spectrum can be used;

- FFS: Whether UL transmission from Ambient IoT device can occur in the FDD DL spectrum.

Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, RANP#103, March, 2024.

[2] R1-2409898, Discussion on frame structure and timing aspects for Ambient IoT, Xiaomi, RAN1#118bis, October 14th-18th, 2024.