3GPP TSG RAN WG1 #119		    R1-2409956

Orlando, US, November 18th – 22nd, 2024

Source:	CATT

Title:	Discussion on measurements related enhancements for LTM

Agenda Item:	9.9.1

Document for:	Discussion and Decision

Introduction

In RAN1#118bis meeting, the following agreements were achieved for measurements related enhancements for LTM [1]:

Agreement The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows: - UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI. - Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18) - The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used - No L1 specified filtering for time and spatial domain is introduced - No enhancement on how to report L cells x M beams - Periodic reporting on PUCCH is supported - FFS: semi-persistent reporting on PUCCH/PUSCH, and aperiodic reporting on PUSCH Agreement From RAN1 perspective, there is no restriction with regards to the frequency location of CSI-RS used for L1-measurement Agreement For CSI-RS based L1-measurement report by gNB scheduled measurement reporting, semi-persistent reporting on PUCCH/PUSCH and aperiodic reporting on PUSCH are supported Agreement The serving cell RS for event evaluation is at least derived from QCL RS or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell - QCL RS above is the RS w.r.t. QCL-TypeD when the indicated joint/DL TCI state is configured with two QCL RSs - FFS: Details on determination of QCL RS or SSB QCLed with QCL RS Note: This does not imply the support of mTRP scenarios Agreement The following alternatives are further studied: - Alt-1: CSI-RS measurement and CSI reporting operations are performed before reception of LTM Cell Switch Command (CSC) MAC CE. - The report is sent to the serving cell and transferred to the candidate/target cell(s) - Alt-2: CSI-RS measurement can start before reception of LTM CSC MAC CE and CSI reporting operation is performed after reception of LTM CSC MAC CE. - The report is sent directly to target cell - Alt-3: CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE. - The report is sent directly to target cell Companies are requested to provide the details of exact report timing and triggering mechanism in the next meeting Working Assumption In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective - Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability. Final LS in R1-2409283.

In this contribution, our views on the CSI-RS for LTM procedures, event triggered L1 measurement reporting and conditional LTM are provided.

Discussion

  - CSI-RS measurements for LTM

    - Configuration of CSI-RS

CSI-RS parameters

In legacy MIMO CSI framework, CSI-RS parameters include the number of ports, CDM type, density, RE locations, frequency band, periodicity, power and so on. These parameters can be directly reused for supporting CSI-RS based measurement for LTM. According to the agreements in RAN1#118, at least CSI-RS for BM (beam management) is supported for L1-RSRP measurement for a candidate cell. CSI-RS for BM includes an additional parameter of ‘repetition’. In MIMO, if ‘repetition’ is set to ‘on’, all the CSI-RS resources within a resource set would be transmitted with the same beam direction, which is used for RX beam sweeping of UE. Otherwise, when ‘repetition’ is set to ‘off’, CSI-RS resources within the resource set could be transmitted with different beam directions, which implies TX beam sweeping of gNB. In our opinion, the parameter ‘repetition’ is also applicable for LTM.

Proposal 1: All the parameters of CSI-RS for BM in legacy CSI framework, including ’repetition’, can be directly reused for CSI-RS based measurement for LTM.

RRC structure

In Rel-18 LTM, TRS has been supported as the QCL source of the TCI state. Corresponding NZP CSI-RS resources are configured per candidate cell as shown in Figure 1. The parameters (e.g. density, RE locations, frequency band) of these NZP CSI-RS resources are not configured directly in candidate cell configuration. Instead, they are included in the serving cell config. In fact, such structure has been already supported the configuration of CSI-RS for L1-RSRP measurement, which should be reused in Rel-19 LTM. In addition, in Rel-18 LTM, a single SSB resource set associated with a CSI report-config includes SSBs of multiple candidate cells. Similarly, NZP CSI-RS resources of multiple candidate cells could also be configured within a resource set. Referring to CSI framework, a separate resource set should be employed for CSI-RS.

Figure 1: RRC structure of CSI-RS configuration

Proposal 2: The RRC structure of configuring TRS as QCL source for SSB based L1-RSRP measurement in Rel-18 LTM can be reused for the configuration of CSI-RS based L1-RSRP measurement in Rel-19 LTM.

Proposal 3: CSI-RS resource set for LTM should include NZP CSI-RS resource(s) of one or multiple candidate cells.

    - Type of CSI-RS

In legacy RRM procedure, CSI-RS for mobility of multiple neighboring cells can be optionally configured for a UE to evaluate the channel quality of these cells for reporting L3-RSRP measurements. In our opinion, the CSI-RS for mobility resources have similar functionality as that of CSI-RS for BM. If CSI-RS for mobility resources are configured for a UE, it is beneficial to reuse them also for L1-RSRP measurements.

Proposal 4: In addition to CSI-RS for beam management, CSI-RS for mobility, if configured, should also be used for L1-RSRP measurements.

    - Transmission of CSI-RS in time domain

For L1-RSRP measurement for candidate cells, it was agreed to support periodic CSI-RS. Considering the RS overhead of such always-on CSI-RS, the number periodic CSI-RS resources should be limited for each candidate cell. In the last meeting, as a working assumption, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurements for gNB scheduled reporting. Semi-persistent CSI-RS can be activated when it is required and be suspended when it is not needed, which requires adding the activating/deactivating signaling required for coordination between serving cell and candidate cell(s). Considering the purpose of introducing CSI-RS in Rel-19 is to address the limitation of SSB, the flexibility provided by semi-persistent CSI-RS seems more important. Therefore, the working assumption should be confirmed. For the same reason, aperiodic CSI-RS should also be supported.

Proposal 5: Confirm the working assumption that semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting.

Proposal 6: Aperiodic CSI-RS should also be supported for candidate cell L1-RSRP measurement for gNB scheduled reporting.

    - Procedure of CSI-RS measurement

In legacy RRM procedure based on CSI-RS for mobility, the CSI-RS for mobility of a candidate cell is measured according to the timing of the candidate cell, which is achieved by associating an SSB of the candidate cell with a CSI-RS resource for mobility, where the SSB serves as the QCL source RS of this CSI-RS resource. Therefore, UE needs to monitor SSB for synchronization before measuring CSI-RS for mobility. Such procedure should also be supported for CSI-RS based L1-RSRP measurement.

Proposal 7: In Rel-19 LTM, CSI-RS for L1-RSRP measurement of a candidate cell should be associated with SSB of the corresponding candidate cell for obtaining the timing of cell.

  - Event triggered L1 measurement reporting

    - L1 measurement resources

In RAN1 #118 meeting, it was agreed that at least periodic CSI-RS is supported for L1-RSRP measurement for candidate cell for both gNB scheduled reporting and event triggered reporting. Whether to support AP CSI-RS and SP CSI-RS for L1-RSRP measurement is still undecided. In general, supporting SP/AP CSI-RS provides more flexibility for LTM. For example, when UE reports the beam of serving cell becomes worse than absolute threshold in Event LTM2, gNB can trigger/activate AP/SP CSI-RS of the candidate cell to quickly find a new better beam, e.g., using Event LTM3, and prevent LTM failure. If AP CSI-RS and/or SP CSI-RS for L1-RSRP measurement is not supported, gNB needs to rely on periodic CSI-RS transmission to see if the condition in Event LTM3 is satisfied. It will lead to an increasing latency of the measurement reporting due to the periodicity of the CSI-RS.

Proposal 8: Support SP CSI-RS and AP CSI-RS for L1-RSRP measurement for event-triggered reporting.

For the identification of the serving cell RS for event evaluation, the following agreement was made in RAN1 #118 meeting.

Agreement - For the identification of the serving cell RS for event evaluation, - At least the following options are further studied in RAN1, where different options could apply to different LTM event - Option. 1: Derived from QCL (type-D) RS(s) of the indicated joint/DL TCI state for the serving cell - Option. 2: Derived from QCL RS(s) or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell - QCL RS or SSB is configured by the network - Option. 3: Measurement RS(s) is/are explicitly configured - Option. 4: Derived from QCL RSs of activated TCI states with the best quality, or SSB which is QCLed with the QCL RSs of activated TCI states with the best quality. - Option 6: Derived from QCL RSs of activated TCI states, or SSB which is QCLed with the QCL RSs of activated TCI states - The RSs of the candidate cell(s) for event evaluation are explicitly configure Note: Companies are encouraged to take into account the RAN2 agreement (i.e current beam rather than best beam) for their further study.

Option 2 was agreed to be supported in RAN1 #118bis meeting. In addition to Option 2, Option 3 should be supported to determine the serving cell RS for event evaluation. It has the benefits that the legacy LTM CSI report configuration framework can be leveraged as much as possible. For example, two LTM-CSI-ResourceSet(s), one is for serving cell RS measurement and another is for candidate cell RS measurement, can be associated with the same LTM-CSI-ResourceConfig.

Proposal 9: For the determination of the serving cell RS for event evaluation, support Option 3, i.e. Measurement RS(s) is/are explicitly configured.

Regarding Option 2 to determine the serving cell RS for event evaluation, one issue to be further discussed is how to determine QCL RS or SSB QCLed with QCL RS for the event evaluation of serving cell RS. In RAN2 #127bis meeting, following agreement was achieved:

Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5)

According to the RAN2 agreement, the serving cell RS for event evaluation should have the same type as those of the candidate cell for event LTM3 and event LTM5. It implies that if the measurement RS for candidate cell is configured as a set of SSB(s), the measurement RS for the serving cell should be SSB which is QCLed with the QCL RS in the indicated TCI state. Similarly, if the measurement RS for candidate cell is CSI-RS, the measurement RS for the serving cell should be QCL RS of the indicated TCI state.

Proposal 10: If the measurement RS for candidate cell is configured as a set of SSB(s), the measurement RS for the serving cell should be the SSB which is QCLed with the QCL RS in the indicated TCI state. When the measurement RS for candidate cell is CSI-RS, the measurement RS for serving cell should be the QCL RS of the indicated joint/DL TCI state.

Another issue is how to determine the measurement RS for serving cell when only one TRS is configured in the indicated TCI state, where the TRS can be the QCL-type D source RS of the indicated TCI state or QCL-type A+D source RS of the indicated TCI state. The issue needs to be solved considering that TRS cannot be used as the beam measurement RS in the current spec. In Rel-19 MIMO discussion, it has been agreed that when only one TRS is configured in the indicated TCI state, the UE assumes that the CSI-RS resource in the indicated TCI state is configured in a CSI-RS resource set configured with repetition. In our opinion, this agreement can be applied to NR Rel-19 LTM, i.e. if only one TRS is configured in the indicated TCI state, it is expected that UE can find a CSI-RS in the CSI-RS resource set configured with repetition which is QCLed with the TRS in the indicated TCI state.

Proposal 11: To determine the measurement RS for the serving cell, when only one TRS is configured in the indicated TCI state, the UE assumes that the CSI-RS resource in the indicated TCI state is configured in a CSI-RS resource set configured with repetition.

    - Uplink report contents

In RAN1 #118 meeting, the necessity of L1 specified filtering mechanism is discussed with no conclusion. The discussion mainly focused on two issues. 1) Filtering event evaluation and reporting. 2) Time domain and cell level filtering. In RAN2 #127bis meeting, following agreement was achieved:

TTT operates only based on a timer (like TTT used in L3 event triggered MR)

According to the agreement, TTT similar to those used in L3 event triggered MR has been introduced for LTM which can be seen as a kind of time-domain filtering, introducing another filtering for event evaluation is not necessary and brings additional implementation complexity. On the other hand, filtering for reporting quantity is needed to avoid ping-pong effect. For LTM, the reported quantity is used for handover. In our opinion, L1-RSRP is not robust enough due to the one-shot measurement. To determine an accurate target cell, the L1 cell-level measurement should also be reported to avoid the ping-pong effect. In the legacy Layer 3 RRM procedure, both beam-level measurement result (L3-RSRP) and cell-level measurement result (spatial filtered L3-RSRP) could be reported. Similarly, in addition to L1-RSRP, L1 cell-level measurement, i.e. spatial filtered L1-RSRP should also be supported for inter-cell mobility. The spatial filtered L1-RSRP could be calculated by linear averaging of those L1-RSRPs above a configured threshold within a cell. In addition, time domain filtering, e.g. first order IIR filtering can be also considered to avoid the ping-pong effect.

Proposal 12: To avoid the ping-pong effect, filtering for reporting is needed. L1 cell-level measurement results, i.e. spatial filtered L1-RSRP or first order IIR filtering should be supported in addition to the beam-level L1-RSRP.

  - Beam Management based on CSI-RS

    - QCL source of candidate TCI state

In Rel-18 LTM, both SSB and TRS have been supported as the QCL source RS of candidate TCI/TCI-UL state for beam indication. SSB can be used to achieve a coarse DL synchronization with the candidate cell, and TRS can be used for fine DL synchronization or beam refinement. It seems there is no need to further support CSI-RS for BM as the QCL source RS of candidate TCI/TCI-UL state.

Proposal 13: No need to support CSI-RS for BM as the QCL source RS of candidate TCI/TCI-UL state for beam indication.

    - Rx beam refinement

As discussed previously, if ‘repetition’ is set to ‘on’, all the CSI-RS resources within a resource set would be transmitted with the same beam direction, which is used for RX beam sweeping of UE. With the prior beam information, e.g., provided by SSB based L1-RSRP for a candidate cell or L3 beam reporting during RRM, the candidate beams may be acquired by the serving cell. Then the candidate cell could be activated/triggered to transmit semi-persistent/aperiodic CSI-RS with a fixed beam direction for Rx beam refinement. This could be applicable for either gNB scheduled reporting or event triggered reporting.

Proposal 14: Rx beam refinement with ‘repetition’ set to ‘on’ is supported for both gNB scheduled reporting and event triggered reporting.

  - CSI acquisition for candidate cell be before cell switch

    - Timing of measurement and reporting

In the last meeting, three alternatives on CSI acquisition framework were proposed for further study. The pros and cons of these three alternatives are analysed below:

Alt-1: CSI-RS measurement and CSI reporting operations are performed before CSC

Pros:

- Legacy CSI framework could be reused. Since CSI reporting is triggered by the serving cell, the triggering signalling and UL resource allocated for reporting can be flexibly controlled by the serving cell.

Cons:

- The overhead of RS measurement and CSI reporting may be larger than Alt-3. As target cell and corresponding target TCI have not been determined, UE needs to measure and report multiple CSI-RS resources of multiple candidate cells.

- Cooperation between serving cell and target cell is needed to inform the reported CSI, which would result in additional RAN3 workload.

- CSI may be out-dated if CSI reporting is triggered much earlier than CSC.

Alt-2: CSI-RS measurement starts before CSC and CSI reporting operations are performed after CSC

Pros:

- Smaller overhead of CSI reporting than Alt.1. Since CSC has indicated the target cell and corresponding target TCI, UE only needs to report single CSI related.

- CSI is reported directly to the target cell without RAN3 impact.

Cons:

- Larger overhead of RS measurement than Alt-3. As target cell and corresponding target TCI have not been determined, UE needs to measure multiple CSI-RS resources of multiple candidate cells.

- CSI may be out-dated if CSI measurement is early performed.

- New mechanism on CSI reporting, e.g. triggering signalling, UL resource determination, has to be designed.

Alt-3: CSI-RS measurement and CSI reporting operations are performed after CSC

Pros:

- Smaller overhead of RS measurement than Alt-1 and Alt-2. The same overhead of CSI reporting as Alt-2. Since CSC has indicated the target cell and corresponding target TCI, UE only needs to report single CSI related.

- CSI is reported directly to the target cell without RAN3 impact.

- Latest CSI could be achieved.

Cons:

- Cell switch delay is increased. Since the target cell and corresponding target LTM cell switch delay is defined as the delay from CSC until the time the UE transmits the first UL message on the target cell. If Alt-3 is supported, additional delay due to CSI-RS measurement and CSI calculation is required, according to the CSI processing timeline defined in MIMO.

- New mechanism on CSI reporting, e.g. triggering signalling, UL resource determination, has to be designed.

The analysis result is summarized in Table 1.

Table 1 Summary of the properties of the three alternatives on CSI acquisition framework

|  | RS overhead | CSI-RS measurement complexity | Reporting overhead | Coordination between serving cell and candidate cell | CSI framework | Validity of CSI | Cell switch delay |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Alt-1 | Large | Large | Large | Yes | Reuse | May be out-dated | No impact |
| Alt-2 | Large (as Alt-1) | Large (as Alt-1) | Small | No | New design | May be out-dated | No impact |
| Alt-3 | Small | Small | Small (as Alt-2) | No | New design | Valid | Increase |

For Alt-2/Alt-3, new triggering mechanism has to be designed, which would cause more RAN1 spec effort. In addition, since one of the main objectives of Rel-19 LTM is to reduce cell switch latency, Alt-3 seems not a proper choice to this objective. In our opinion, Alt-1 is slightly preferred with its less RAN1 impact. To deal with the overhead issue, aperiodic CSI-RS transmitted from the candidate cells and aperiodic CSI reporting could be triggered by the serving cell. The CSI-RS to be measured could be restricted within the cells/beams reported in the recent LTM L1 measurement reporting, or determined by the QCL source RS in the activated candidate TCI states. In addition, wideband CSI would not change quite frequently. If supported, it is feasible for the serving cell to determine an appropriate time to trigger a valid CSI reporting, e.g. just before the target cell and corresponding TCI state have been decided.

Proposal 15: CSI-RS measurement and CSI reporting operations performed before CSC (i.e., Alt-1) is slightly preferred among the alternatives.

Proposal 16: For Alt-1, the CSI-RS to be measured could be restricted within the cells/beams reported in the recent LTM L1 measurement reporting, or determined by the QCL source RS in the activated candidate TCI states.

    - Time domain property of CSI-RS transmission

As mentioned above, for Alt-1/Alt-2, multiple CSI-RS resources of multiple candidate cells need to be measured. Since each CSI-RS resource is configured with more than one port (e.g. up to 128 ports) for PMI calculation, if periodic CSI-RS is used, the CSI-RS overhead would be quite large. According to the method proposed previously in Section 2.5.1, aperiodic CSI-RS can be used instead. For Alt-3, it is straightforward for the target cell to transmit aperiodic CSI-RS with the target beam indicated in CSC. In this way, aperiodic CSI-RS is applicable to all the three alternatives, which should at least be supported.

Proposal 17: At least aperiodic CSI-RS is supported for CSI acquisition of candidate/target cell(s).

    - Time domain property of CSI reporting

According to the WID [2], before first data transmission with the target cell, accurate CSI reporting (RI/PMI/CQI) for the target cell is required to determine a suitable MCS. It enables greater throughput on the target cell immediately after cell switch. Therefore, only the latest CSI is useful. To avoid unnecessary reporting, only aperiodic report on PUSCH is supported for CSI acquisition of candidate/target cell(s).

Proposal 18: Only aperiodic report on PUSCH is supported for CSI acquisition of candidate/target cell(s).

    - Restrictions on the CSI configurations

In legacy MIMO, a number of report quantities are specified aiming different application scenarios. For LTM, the reported CSI is used for the first data transmission after cell switch. For that purpose CSI reporting with less computation complexity seems enough. In addition, considering the potential limited UL resource for CSI reporting, larger CSI payload size should be avoided. Therefore, the configuration of CSI report should be restricted to Type-I codebook with report quantity configured with ‘CRI-RI-PMI-CQI’.

Proposal 19: For CSI acquisition of candidate/target cell(s), support CSI report for Type-I codebook with report quantity configured with ‘CRI-RI-PMI-CQI’.

  - Conditional LTM

Following agreements on CLTM have been achieved in RAN2 #127bis [3]:

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions. - Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions. - Source cell and each candidate cell provides its own execution condition for conditional LTM. - It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used. - RACH-less Conditional intra-CU LTM is supported. - RACH based conditional intra-CU LTM is supported. - UE based TA measurement mechanism is supported for conditional LTM. - PDCCH ordered early TA acquisition is supported for conditional LTM. - Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM. - For RACH-less conditional LTM, CG-based first ULtransmission on target cell is supported. FFS on DG-based approach. - The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

From the above agreements, both RACH-based and RACH-less conditional intra-CU LTM are supported. For the RACH-less conditional intra-CU LTM, upon receiving the conditional LTM configuration via RRCReconfiguration, the UE obtains the LTM candidate cells for measurement. Then UE then performs downlink and uplink synchronization with these candidate cells. Afterward, UE checks whether the execution condition for conditional LTM is satisfied with the candidate cells and chooses one candidate cell as the target cell for cell switch. Subsequently, UE detaches from source and apply target cell configuration. UE sends RRCReconfigruationComplete message as the first UL data transmission, using the granted UL resource, to the target cell to signal the completion of the cell switch.

Performing downlink and uplink synchronization with all candidate cells will increase UE complexity. To solve this problem, two thresholds, T1 and T2, can be defined to choose the target cell from those candidate cells configured in RRCReconfiguration. The first threshold is used to choose some candidate cells for downlink and uplink synchronization. Suppose L1-RSRP = 6dB is defined for the first threshold T1, The UE measures the beam quality (e.g. L1-RSRP) of all the candidate cells configured in RRCReconfiguration and compares them with T1.  Then the UE chooses a set of the candidate cells, C1, with L1-RSRP larger than T1(6dB) from all the candidate cells configured to perform downlink and uplink synchronization. Similarly, the UE checks the candidate cell(s) in the subset against T2 and chooses the target cell which satisfies the condition in the trigger event for cell switch.

Proposal 20:  For Rel-19 CLTM, two thresholds can be defined to select target cell from candidate cells configured in RRCReconfiguration. The first threshold is used to select candidate cells for downlink and uplink synchronization, and the second threshold is used to determine the target cell for cell switch.

In RAN2 #127bis meeting, it was agreed that PDCCH ordered early TA acquisition is supported for CLTM. PDCCH ordered early TA acquisition was supported in NR Rel-18, where gNB controls the candidate cells for which a PRACH is triggered by a PDCCH order. In Rel-19 CLTM, UE performs the beam measurement to multiple candidate cells and chooses the candidate cell for downlink and uplink synchronization (TA acquisition). To accomplish this, UE can send a request to gNB, indicating the candidate cells for TA acquisition. The candidate cell index and the beam measurement results(CRI/SSBRI) can also be reported with the request. Upon reception of the indication, gNB will issue a PDCCH order to trigger the UE to send PRACH toward the indicated candidate cells included. Then the candidate cells will receive the preamble with the transmission beam of CRI/SSBRI reported by the UE to estimate TA.

Proposal 21: For PDCCH ordered RACH-based TA acquisition mechanism for CLTM, support a UE to send a request to indicate the candidate cells for TA acquisition.

Upon receiving the preamble from the UE, the candidate cell will derive the TA and deliver it to the UE. How to provide the estimated TA to UE is an issue to be resolved. For PDCCH ordered RACH in Rel-18 LTM, the TA is provided in the cell switch command. However, this method cannot be applied because in NR Rel-19 conditional LTM, as the UE autonomously evaluates the conditions for the conditional cell switch and performs the cell switch to the target cell without cell switch command. The estimated TA can be provided either directly by the candidate cell or via the current serving cell.  The TA can be included in RAR or a MAC CE, which needs further discussion.

Proposal 22: For PDCCH ordered RACH based TA acquisition mechanism for conditional LTM, following options are considered for TA delivery:

- RAR from the candidate cell

- RAR from the serving cell

- MAC CE from the serving cell

  - FFS: whether the MAC CE carries the TAs for multiple candidate cells.

When the UE receives the TA delivered by the network, it will perform the RACH-less cell switch to the target cell once it detects that any condition defined in the conditional cell switch is satisfied. If no condition defined in the conditional cell switch is satisfied, the UE is required to maintain the TA until the cell switch takes place. How to maintain the validity of the TA for UE until the cell switch requires further discussion.

In our opinion, the TA validity maintenance should be performed at the UE side since gNB doesn’t know the exact time when the condition for conditional cell switch is satisfied. To update the TA value received from gNB, a delta_TA can be used, which is obtained by UE-based TA acquisition. Specifically, UE measures the timing difference of the downlink reference signal from serving cell and candidate cell, which can be SSB or CSI-RS, to obtain delta_TA. Then TA value can be updated by using the delta_TA.

Proposal 23: For PDCCH ordered RACH-based TA acquisition mechanism for CLTM, TA validity maintenance is performed at the UE, where UE-based TA acquisition is applied to obtain the delta_TA which is used to update the TA on the TA acquired by PDCCH ordered RACH. Delta_TA is calculated by measuring the timing difference of SSB/CSI-RS received from serving cell and candidate cells.

Conclusions

In this contribution, we provide our views on the support of CSI-RS for LTM procedures, event triggered L1 measurement reporting and conditional LTM.  We have the following proposals:

Proposal 1: All the parameters of CSI-RS for BM in legacy CSI framework, including ’repetition’, can be directly reused for CSI-RS based measurement for LTM.

Proposal 2: The RRC structure of configuring TRS as QCL source for SSB based L1-RSRP measurement in Rel-18 LTM can be reused for the configuration of CSI-RS based L1-RSRP measurement in Rel-19 LTM.

Proposal 3: CSI-RS resource set for LTM should include NZP CSI-RS resource(s) of one or multiple candidate cells.

Proposal 4: In addition to CSI-RS for beam management, CSI-RS for mobility, if configured, should also be used for L1-RSRP measurements.

Proposal 5: Confirm the working assumption that semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting.

Proposal 6: Aperiodic CSI-RS should also be supported for candidate cell L1-RSRP measurement for gNB scheduled reporting.

Proposal 7: In Rel-19 LTM, CSI-RS for L1-RSRP measurement of a candidate cell should be associated with SSB of the corresponding candidate cell for obtaining the timing of cell.

Proposal 8: Support SP CSI-RS and AP CSI-RS for L1-RSRP measurement for event-triggered reporting.

Proposal 9: For the determination of the serving cell RS for event evaluation, support Option 3, i.e. Measurement RS(s) is/are explicitly configured.

Proposal 10: If the measurement RS for candidate cell is configured as a set of SSB(s), the measurement RS for the serving cell should be the SSB which is QCLed with the QCL RS in the indicated TCI state. When the measurement RS for candidate cell is CSI-RS, the measurement RS for serving cell should be the QCL RS of the indicated joint/DL TCI state.

Proposal 11: To determine the measurement RS for the serving cell, when only one TRS is configured in the indicated TCI state, the UE assumes that the CSI-RS resource in the indicated TCI state is configured in a CSI-RS resource set configured with repetition.

Proposal 12: To avoid the ping-pong effect, filtering for reporting is needed. L1 cell-level measurement results, i.e. spatial filtered L1-RSRP or first order IIR filtering should be supported in addition to the beam-level L1-RSRP.

Proposal 13: No need to support CSI-RS for BM as the QCL source RS of candidate TCI/TCI-UL state for beam indication.

Proposal 14: Rx beam refinement with ‘repetition’ set to ‘on’ is supported for both gNB scheduled reporting and event triggered reporting.

Proposal 15: CSI-RS measurement and CSI reporting operations performed before CSC (i.e., Alt-1) is slightly preferred among the alternatives.

Proposal 16: For Alt-1, the CSI-RS to be measured could be restricted within the cells/beams reported in the recent LTM L1 measurement reporting, or determined by the QCL source RS in the activated candidate TCI states.

Proposal 17: At least aperiodic CSI-RS is supported for CSI acquisition of candidate/target cell(s).

Proposal 18: Only aperiodic report on PUSCH is supported for CSI acquisition of candidate/target cell(s).

Proposal 19: For CSI acquisition of candidate/target cell(s), support CSI report for Type-I codebook with report quantity configured with ‘CRI-RI-PMI-CQI’.

Proposal 20:  For Rel-19 CLTM, two thresholds can be defined to select target cell from candidate cells configured in RRCReconfiguration. The first threshold is used to select candidate cells for downlink and uplink synchronization, and the second threshold is used to determine the target cell for cell switch.

Proposal 21: For PDCCH ordered RACH-based TA acquisition mechanism for CLTM, support a UE to send a request to indicate the candidate cells for TA acquisition.

Proposal 22: For PDCCH ordered RACH based TA acquisition mechanism for conditional LTM, following options are considered for TA delivery:

- RAR from the candidate cell

- RAR from the serving cell

- MAC CE from the serving cell

  - FFS: whether the MAC CE carries the TAs for multiple candidate cells.

Proposal 23: For PDCCH ordered RACH-based TA acquisition mechanism for CLTM, TA validity maintenance is performed at the UE, where UE-based TA acquisition is applied to obtain the delta_TA which is used to update the TA on the TA acquired by PDCCH ordered RACH. Delta_TA is calculated by measuring the timing difference of SSB/CSI-RS received from serving cell and candidate cells.

References

- Chair notes RAN1#118bis, 3GPP TSG RAN WG1 Meeting #118bis, Hefei, China, October 14th – 18th, 2024.

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, RAN meeting #105, Melbourne, Australia, Sep 9-12, 2024.

- R2-2406204，Report from session on V2X/SL, R18/19 MOB, and R19 NES. RAN2 meeting #127, Maastricht, Netherlands, Aug 19th – 23rd, 2024.