3GPP TSG RAN WG1 #119	R1-2410060

Orlando, USA, November 18th – 22nd, 2024

Agenda Item:	9.4.2.2

Source:	Fujitsu

Title:	Discussion on frame structure and timing aspects

Document for:	Discussion

Introduction

In Rel-18, as a promising new IoT type device/service, Ambient IoT was studied as a RAN SI. The study conclusions and observations on deployment scenarios, use cases, and RAN design targets are captured in TR 38.848 [1]. In RAN#102, Ambient IoT was successfully approved as a Rel-19 WG SI [2].

The scope of this sub-agenda is about frame structure and timing aspects. In this contribution, we provide our views about these aspects.

Discussion

Carrier frequency synchronization for device 2b

In RAN1-118bis meeting, the carrier frequency synchronization for device 2b was discussed as following proposal and agreement.

FL3 High Priority Proposal 3.2.2-2: For device 2b, RAN1 studies the following two options for carrier frequency synchronization: - Opt.1: Introduce dedicated synchronization signal for carrier frequency synchronization. - Opt.2: Enable synchronization for carrier frequency using R2D signal/channel. FL4: following agreement was made in AI 9.4.1.2 Agreement For clock purpose #5, adopt following update. #; Purpose; Clock speed; Applicable Device types; Power consumption; Initial clock Accuracy (ppm); Accuracy after clock sync / calibration at device side (ppm). #5; LO for carrier frequency (for up/down conversion); According to carrier frequency e.g., 900 MHz; 2b; 10s ~ 100s uW; [10^23 ~ 10^4]; Option 1: 10s (Note2) Option 2: 100s (Note2). Note 2: Either internal clock counter for calibration clock counting based on at least R2D signals or clock counting based on transmission from reader Calibration is based on existing or new signal from reader. FFS how to calibrate (9.4.2.2). No drastic temperature change is assumed during calibration.. - Further study above options and strive to down select considering its implication on performance, system design, necessity, etc. Note: study of whether additional signal is necessary or not for calibration is discussed under agenda 9.4.2.2

One of the intentions for the carrier frequency offset (CFO) calibration on device 2b is to improve the spectrum efficiency. The initial CFO on devices consumes too much guard-band for D2R transmission. For example, with 103 ppm of initial CFO, it would require a 1.8MHz guard-band on the two-sides of a D2R transmission in the carrier frequency of 900 MHz, which makes the D2R resource utilization efficiency very low.

Observation 1:

The initial CFO on device 2b makes D2R resource utilization efficiency very low.

Proposal 1:

The calibration of initial CFO on device 2b is necessary to reduce the occupied BW for D2R transmission which improves the spectrum efficiency.

For option 2 of FL proposal, it needs to study the feasibility for CFO calibration using R2D signal/channel, e.g., the R2D timing acquisition signal. The R2D timing acquisition signal is designed for SFO calibration of sampling clock. In our understanding, the local oscillator with FLL embedded in device 2b can be assumed as the common reference for different purposes of clocks considering the complexity and the cost of the device. If a device calibrates the SFO of the sampling clock by adjusting the frequency of local oscillator, the CFO of carrier frequency on the device is calibrated to some extent as well.

Observation 2:

It’s feasible to calibrate the CFO on device 2b using R2D signal/channel, e.g., the R2D timing acquisition signal.

- The local oscillator with FLL embedded in device 2b can be assumed as the common reference for different purposes of clocks considering the complexity and the cost of the device.

- The calibration of SFO for the sampling clock by adjusting the frequency of local oscillator calibrates the CFO of carrier frequency to some extent as well.

Proposal 2:

For device 2b, the carrier frequency synchronization with R2D signal/channel, e.g., the R2D timing acquisition signal, is considered as the starting point.

One critical factor is that the target accuracy of CFO calibration is much higher than that of SFO calibration. For example, unlike SFO calibration that targets <= 1%, CFO calibration would need to target < 0.1%. If the R2D signal/channel, e.g., R2D timing acquisition signal, is used to calibrate the CFO on device 2b, how to obtain < 0.1% calibration accuracy needs further consideration.

Proposal 3:

If the R2D signal/channel, e.g., R2D timing acquisition signal, is used for carrier frequency synchronization on device 2b, how to obtain < 0.1% accuracy needs further consideration.

For option 1 of FL proposal, it’s not clear the benefit to introduce a dedicated signal for CFO calibration. Some companies [4][5] propose a single-tone RF signal is transmitted from reader to calibrate the CFO in analog domain for device 2b. The analog domain calibrator will introduce additional complexity and cost for the device. And the analog domain calibration may not be robust due to the interference and fading.

Proposal 4:

For device 2b, introducing a dedicated signal for carrier frequency synchronization should be deprioritized.

D2R ambles

In RAN1 #118bis meeting, the following agreement was reached for the study of the ambles for the D2R transmission.

Agreement - The TR will capture the following options, and companies are encouraged to analyze the tradeoffs among the following D2R amble(s) options: - Option 1: D2R preamble only - Option 2: D2R preamble + X midamble(s), where X 1 - Option 3: D2R preamble + postamble - Option 4: D2R preamble + Y midamble(s) + postamble, where Y1 - For the above options, companies are encouraged to report at least the following: - Purpose(s) of the preamble, midamble and postamble - Whether companies assume multiple options can be supported

Following the agreements, we have the following observations on the potential usage of the preamble/midamble/postamble signal in D2R transmissions.

Observation 3:

The observations on the purposes to adopt preamble/midamble/postamble are listed in Table 1.

Table 1 Observations on the usage of the preamble/midamble/postamble signal in D2R transmissions

|  | Coherent detection at the reader | Non-coherent detection at the reader |  |
| --- | --- | --- | --- |
|  |  | Without line code | With line code 1 |
| Preamble | 1. To indicate the start of the subsequent D2R transmission. 2. To estimate the SFO occurring in the received D2R signal. 3. To estimate the CFO occurring in the received D2R signal (device 2b only). 4. To estimate channel. | 1. To indicate the start of the subsequent D2R transmission. 2. To estimate the SFO occurring in the received D2R signal. 3. To estimate the CFO occurring in the received D2R signal (device 2b only). | 1. To indicate the start of the subsequent D2R transmission. |
| Midamble(s) | 1. To estimate the SFO occurring in the received D2R signal. 2. To estimate the CFO occurring in the received D2R signal (device 2b only). 3. To estimate channel. | 1. To estimate the SFO occurring in the received D2R signal. 2. To estimate the CFO occurring in the received D2R signal (device 2b only). |  |
| Postamble | 1. To indicate the end of the D2R transmission 2, or to increase the probability of correctly determining the end of the D2R transmission 3. 2. To estimate the SFO occurring in the received D2R signal. 3. To estimate the CFO occurring in the received D2R signal (device 2b only). 4. To estimate channel. | 1. To indicate the end of the D2R transmission 2, or to increase the probability of correctly determining the end of the D2R transmission 3. 2. To estimate the SFO occurring in the received D2R signal. 3. To estimate the CFO occurring in the received D2R signal (device 2b only). | 1. To indicate the end of the D2R transmission 2, or to increase the probability of correctly determining the end of the D2R transmission 3. |
| Condition 1: If it is confirmed that line code can work well for synchronization/timing determination in the whole SNR region required for D2R transmissions. Condition 2: In the case the reader cannot always determine the end of the D2R transmissions, postabmle is needed to indicate the end of the transmissions to the reader. For example, the reader doesn’t know how long the D2R transmission is (the symbol number of the D2R transmission). Condition 3: In the case the reader knows where the end of the D2R transmissions is, but the practical determination of the end of the D2R transmissions could still be wrong due to some reasons, e.g., low SNR/low reception power of D2R signal. |  |  |  |

Besides, we think the options on the D2R ambles which are listed in the agreement are not mutually exclusive, at least, for now.

- For option 1 (preamble only):

  - It seems not clear yet whether option 1 can always work well for synchronization and channel estimation (when coherently detecting) in the whole SNR region required for D2R transmissions.

    - If the performance of the coherent detection based on preamble only is confirmed to be acceptable in the whole SNR region required for D2R transmissions, option 1 may be enough for the whole design.

  - Option 1 is a basic design and cannot be excluded from the final design. Option 1 is at least necessary for very short D2R transmissions, i.e., with a very small payload size, e.g., a few bits to about twenty bits.

    - When a D2R transmission is very short, the channel experienced by the D2R transmission would not obviously change, because the relative speed between the device and the reader is usually assumed to be very slow and the continuation time of the transmission is short. Moreover, the very short transmission would not experience large SFO change as well.

      - The synchronization and the SFO/CFO estimation based on Option 1 could be enough for a non-coherent detection.

      - The synchronization, the channel estimation and SFO/CFO estimation based on option 1 could be enough for coherent detection.

- For option 2 (preamble + midamble):

  - Option 2 may be necessary for D2R transmissions with large payload size.

    - The payload size of D2R transmissions could be very large, such as 1000 bits, which was described in the SID and confirmed in the last RAN1 meeting. When a transmission carries hundreds of bits or one thousand bits, the transmission may last tens ms or hundreds ms.  With such a long transmission time, the transmission would experience an obvious change of both the channel and the SFO at the transmitter side.

      - If coherent detection is adopted, some midamble could be necessary to guarantee the reader has enough reference to accurately estimate the channel and/or SFO/CFO.

      - If non-coherent detection and a line code are both adopted, theoretically, midamble(s) might be necessary as well, which depends on whether this scheme can work well in lower SNR region. If not, midamble(s) are necessary. Otherwise, midamble(s) are not necessary.

        - So far, there is still no common sense on the factor that how low SNR should be supported for D2R transmission yet. Some further discussion/study is needed, perhaps, in WI phase.

- For option 3: (preamble + postamble)

  - The need for option 3 mainly depends on the need for postamble.

    - The potential usages of postamble are listed in Table 1:

      - To indicate the end of the D2R transmission 2, or to increase the probability of correctly determining the end of the D2R transmission.

      - To be used for channel/SFO/CFO estimation.

    - The postamble may be needed in the following cases:

      - When the reader doesn’t always know where the end of the D2R transmissions is.

      - When the probability of correctly determining the end of the D2R transmissions is smaller than what is required.

      - When the accuracy of estimated channel/SFO/CFO only based on preamble/midamble(s) is not enough.

- For option 4 (preamble + midamble(s) + postamble):

  - Option 4 is needed if there is a case that the accuracy of estimated channel/SFO/CFO only based on option 1 and/or option 2 is really bad.

Based on the above considerations, we have the following observation:

Observation 4:

More than one amble option can be supported. More details rely on further discussion/consideration in WI phase.

Random access

Resource allocation for Msg. 1

In RAN1-118 meeting, the resource allocation about Msg. 1 is discussed as the following agreement [3].

Agreement RAN1 studies following: - A R2D transmission triggering random access determines X time domain resource(s) for D2R transmission(s) for Msg1, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s). - The study includes - Study X=1 and X>1 and X>=1, the maximum value of X>1 should be set considering the device implementation complexity, device power consumption, the resource usage efficiency affected at least by SFO, and inventory latency. - Size(s) for resource allocation in the time domain - Determination of the X time domain resource(s) by the device - Addressing timing errors for adjacent time domain resources due to residual SFO of the device

How the X>=1 time domain resources for Msg. 1 are allocated by a page information should be studied. From our understanding, the position of each resource in the time domain should be indicated by the page information. The resource position in the time domain can be indicated with a starting point and a duration with reference to the position of paging message. Figure 1 gives an example about two resources in the time domain allocated by a paging message. An offset is used to indicate the distance between the end of the paging message and the starting point of a time domain resource. A duration is used to describe the length of a time domain resource for Msg1. transmission.

Figure 1 Example about two resources in the time domain allocated by a paging message

Proposal 5:

For the resource allocation for Msg.1, the resource position in the time domain should be indicated in the paging message.

- The position of a time domain resource can be indicated by an offset and a duration with reference to the position of the paging message.

Regarding the resource allocation for Msg. 1 transmission, the duration of an allocated time domain resource could be longer than the actual duration of a time domain resource used for Msg. 1 transmission. Some margin should be reserved to conquer the timing error due to the inaccurate clock embedded in the device. Figure 2 shows an example. The reader monitors Msg. 1 on the indicated duration of a time domain resource. As long as the Msg. 1 from a device falls in the indicated duration, the reader can successfully detect the Msg. 1.

Figure 2 Example about the duration of a time domain resource for Msg.1 transmission

Proposal 6:

Regarding the duration of a time domain resource for Msg.1 indicated by the paging message, some margin should be reserved to conquer the potential timing error due to the inaccurate clock embedded in the device.

Msg.2 related aspects

In RAN1-118 meeting, the Msg. 2 related aspects have been discussed as following agreement [3].

Agreement RAN1 studies the following options for Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access. - Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device - Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices Agreement RAN1 studies the starting time and time duration for Msg2 monitoring for Msg2 reception.

For Msg.2 monitoring by a device, in the case that a PRDCH for Msg.2 transmission corresponds to multiple Msg.1 received from different devices, the Msg.2 can be monitored with reference to the time domain resource of the last Msg.1 allocated in paging message. Figure 3 shows an example. A device should know the position of the time domain resource for the last Msg.1 after receiving the paging message. The device can start monitoring Msg. 2 with time interval TD2R_min from the last Msg.1 time domain resource.

Figure 3 Example about Msg.2 monitoring

Proposal 7:

In case that a PRDCH for Msg.2 transmission corresponds to multiple Msg.1 received from different devices, the Msg.2 can be monitored with reference to the time domain resource of the last Msg. 1.

- The device starts monitoring the Msg.2 with time interval TD2R_min from the time domain resource of the last Msg.1.

In case that a PRDCH for Msg.2 transmission corresponds to a Msg.1 received from one device, a reader needs to transmit more than one Msg.2 in response to more than one Msg.1. A device may need to monitor more than one Msg.2 and identify which one is for itself. The time duration which a device needs to monitor is larger than the time duration used for its Msg.2 transmission. Figure 4 shows an example. Considering the limited energy stored on devices, a mechanism to facilitate a device to locate its Msg.2 may need to be considered to save the power of the device, e.g., based on some assistant signaling or predefined rule.

Figure 4 Example about Msg.2 monitoring

Observation 5:

In case that a PRDCH for Msg.2 transmission corresponds to a Msg.1 received from one device,

- The device may need to monitor more than one Msg.2 and identify which one is for itself.

Proposal 8:

In case that a PRDCH for Msg.2 transmission corresponds to a Msg.1 received from one device, a mechanism to facilitate a device to locate its Msg.2 may need to be considered to save the power of the device, e.g. based on some assistant signaling or predefined rules.

- Conclusion

In this contribution, we consider the frame structure and timing aspects for AIoT. We have following observations and proposals:

Observation 1:

The initial CFO on device 2b makes D2R resource utilization efficiency very low.

Proposal 1:

The calibration of initial CFO on device 2b is necessary to reduce the occupied BW for D2R transmission which improves the spectrum efficiency.

Observation 2:

It’s feasible to calibrate the CFO on device 2b using R2D signal/channel, e.g., the R2D timing acquisition signal.

- The local oscillator with FLL embedded in device 2b can be assumed as the common reference for different purposes of clocks considering the complexity and the cost of the device.

- The calibration of SFO for the sampling clock by adjusting the frequency of local oscillator calibrates the CFO of carrier frequency to some extent as well.

Proposal 2:

For device 2b, the carrier frequency synchronization with R2D signal/channel, e.g., the R2D timing acquisition signal, is considered as the starting point.

Proposal 3:

If the R2D signal/channel, e.g., R2D timing acquisition signal, is used for carrier frequency synchronization on device 2b, how to obtain < 0.1% accuracy needs further consideration.

Proposal 4:

For device 2b, introducing a dedicated signal for carrier frequency synchronization should be deprioritized.

Observation 3:

The observations on the purposes to adopt preamble/midamble/postamble are listed in Table 1.

Table 1 Observations on the usage of the preamble/midamble/postamble signal in D2R transmissions

|  | Coherent detection at the reader | Non-coherent detection at the reader |  |
| --- | --- | --- | --- |
|  |  | Without line code | With line code 1 |
| Preamble | 1. To indicate the start of the subsequent D2R transmission. 2. To estimate the SFO occurring in the received D2R signal. 3. To estimate the CFO occurring in the received D2R signal (device 2b only). 4. To estimate channel. | 1. To indicate the start of the subsequent D2R transmission. 2. To estimate the SFO occurring in the received D2R signal. 3. To estimate the CFO occurring in the received D2R signal (device 2b only). | 1. To indicate the start of the subsequent D2R transmission. |
| Midamble(s) | 1. To estimate the SFO occurring in the received D2R signal. 2. To estimate the CFO occurring in the received D2R signal (device 2b only). 3. To estimate channel. | 1. To estimate the SFO occurring in the received D2R signal. 2. To estimate the CFO occurring in the received D2R signal (device 2b only). |  |
| Postamble | 1. To indicate the end of the D2R transmission 2, or to increase the probability of correctly determining the end of the D2R transmission 3. 2. To estimate the SFO occurring in the received D2R signal. 3. To estimate the CFO occurring in the received D2R signal (device 2b only). 4. To estimate channel. | 1. To indicate the end of the D2R transmission 2, or to increase the probability of correctly determining the end of the D2R transmission 3. 2. To estimate the SFO occurring in the received D2R signal. 3. To estimate the CFO occurring in the received D2R signal (device 2b only). | 1. To indicate the end of the D2R transmission 2, or to increase the probability of correctly determining the end of the D2R transmission 3. |
| Condition 1: If it is confirmed that line code can work well for synchronization/timing determination in the whole SNR region required for D2R transmissions. Condition 2: In the case the reader cannot always determine the end of the D2R transmissions, postabmle is needed to indicate the end of the transmissions to the reader. For example, the reader doesn’t know how long the D2R transmission is (the symbol number of the D2R transmission). Condition 3: In the case the reader knows where the end of the D2R transmissions is, but the practical determination of the end of the D2R transmissions could still be wrong due to some reasons, e.g., low SNR/low reception power of D2R signal. |  |  |  |

Observation 4:

More than one amble option can be supported. More details rely on further discussion/consideration in WI phase.

Proposal 5:

For the resource allocation for Msg.1, the resource position in the time domain should be indicated in the paging message.

- The position of a time domain resource can be indicated by an offset and a duration with reference to the position of the paging message.

Proposal 6:

Regarding the duration of a time domain resource for Msg.1 indicated by the paging message, some margin should be reserved to conquer the potential timing error due to the inaccurate clock embedded in the device.

Proposal 7:

In case that a PRDCH for Msg.2 transmission corresponds to multiple Msg.1 received from different devices, the Msg.2 can be monitored with reference to the time domain resource of the last Msg. 1.

- The device starts monitoring the Msg.2 with time interval TD2R_min from the time domain resource of the last Msg.1.

Observation 5:

In case that a PRDCH for Msg.2 transmission corresponds to a Msg.1 received from one device,

- The device may need to monitor more than one Msg.2 and identify which one is for itself.

Proposal 8:

In case that a PRDCH for Msg.2 transmission corresponds to a Msg.1 received from one device, a mechanism to facilitate a device to locate its Msg.2 may need to be considered to save the power of the device, e.g. based on some assistant signaling or predefined rules.

- Reference

[1] TR 38.848 V18.0.0, Study on Ambient IoT (Internet of Things) in RAN, 2023-09.

[2] RP-240826, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR, 3GPP TSG RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024.

[3] R1-2409244, Final FL summary on frame structure and timing aspects for Rel-19 Ambient IoT, Hefei, China, October 14th – 18th, 2024.

[4] R1-2407671, Physical channels and signals for Ambient IoT, Hefei, China, October 14th – 18th, 2024. Huawei, HiSilicon.

[5] R1-2408853, Downlink and uplink channel/signal aspects, Hefei, China, October 14th – 18th, 2024. Qualcomm.