3GPP TSG RAN WG1 Meeting #119		R1-2410233

Orlando, U.S., 18th – 22nd November 2024

Agenda Item	: 	9.6.3

Source 	:	Sony

Title 	:	LP-WUS operation in CONNECTED mode

Document for 	:	Discussion and decision

# Introduction

A revised work item on Low-power Wake-up Signal and Receiver for NR (LP-WUS/WUR) was approved in RANP#103 [1]. The objective for the operation in connected mode is:

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

In RAN#118bis, the following agreements were made [2]:

Agreement For Option 1-1 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer - UE is configured with legacy C-DRX configurations as Rel-18 - UE behaviors related to CDRX active time triggered by all legacy DRX timers are not affected unless included in this proposal - No impact on RRM/RLM/BFD measurement requirements is assumed - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP) - Periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported during the time given by the configured drx-onDurationTimer regardless if UE is indicated to wake-up or not - Note: Periodic CSI/L1-RSRP is reported during CDRX active time as in legacy specification Confirm following working assumption Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions. For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported candidate value - UE can indicate its preference for configured time offset between LP-WUS and PDCCH reception using UAI mechanism. FFS details. UAI is optional. - FFS: A single time offset is configured for a UE at a given time LP-WUS is supported when UE is configured with NR-DC in RRC CONNECTED mode - Above is supported for the case PDCCH monitoring is triggered by LP-WUS in the same cell group - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable For LP-WUS CONNECTED mode operation, Option 1-2 can be configured separately from Option 1-1 - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Regarding the potential restriction for LP-WUS configuration in relation with C-DRX configuration for option 1-2 of LP-WUS CONNECTED mode operation, from RAN1 perspective, - AltOpt2: At least LP-WUS periodicity is no larger than long C-DRX cycle, FFS other restrictions - Introducing monitoring window is not precluded

In this document, we provide our views on LP-WUS operation in RRC CONNECTED mode.

# Discussion

The legacy UE operating in RRC connected mode and configured with C-DRX should keep the main receiver (MR) fully operated to monitor PDCCH in C-DRX active time. However, there are some occasions when there is nothing scheduled for the UE. Consequently, the UE uses unnecessary power consumption by monitoring PDCCH when the gNB does not transmit anything on the PDCCH. There have been some enhancements proposed in previous NR releases to reduce UE power consumption. These techniques are Rel-16 DCP on the usage of DCI with CRC scrambled by PS-RNTI and Rel-17 PDCCH monitoring adaptation. However, both techniques are based on PDCCH-based signalling. PDCCH decoding is known as a power-hungry and high computation operation as it involved blind detection process. LP-WUS can be used as an alternative to DCP so that the UE power consumption can be improved. LP-WUS can trigger the PDCCH monitoring. Several solutions/options were identified in the last few meetings. In RAN1#118, it was agreed to support Option 1-1 and Option 1-2. Our main preference is option 1-1 that the LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. The operation of option 1-1 is illustrated in Figure 1.

In our view, Option 1-2 is the extension of Option 1-1. It has more flexibility in comparison to Option 1-1. For example, the starting time for the UE to monitor the PDCCH, that is starting of the drx-onDurationTimer can be more flexible. There can be some other parameters that can be configurable. Supporting Option 1-2, in addition to Option 1-1 can be beneficial for the UE operation in RRC connected mode. The UE can be operated to carry different types of traffic, including XR traffic, IoT-type traffic, eMBB traffic, in which the data transmission can be tailored so that UE power consumption can be optimized. We support to confirming the working assumption made in RAN1#118.

Figure 1: Illustration of LP-WUS with Option 1-1 [3]

The discussion on support of multiple minimum time gaps (i.e., the time offset in Figure 1) was initiated in RAN1#117 and continued further in subsequent meetings. We consider the LP-WUS operation with multiple minimum time-gaps can be beneficial. Some of the UE operation may require a shorter gap (e.g., XR applications). Some others can be more predicted by the gNB, so that the minimum time gap can be longer. The supporting of multiple minimum time-gaps can be based on UE capability. One UE may have a longer gap while the others may have a shorter one. A UE that supports a shorter time-gap indirectly also supports longer time-gap or multiple time-gaps. For such UE, it can be operated such that depending on the need it can be operated with longer time gap and on another occasion, it can be operated with a shorter time-gap. At a given time, a UE is operating with a single time offset. It can be explicitly configured by the gNB or implicitly configured, such as based on configured rules at the UE side.

Proposal 1: Support LP-WUS in RRC connected mode with multiple minimum time-gaps depending on UE capability.

Proposal 2: Support a single time offset (FFS: explicit / implicit) that can be configured for a UE at a given time.

During study item phase, various traffic models when the UE is in RRC connected mode were evaluated. The traffic models were XR application, FTP application, and IM application. These applications have different behavior and characteristics, where XR application is one of the examples. The received XR traffic to the UE may experience jitter so that the actual received transmission can be delayed or advanced within certain window (e.g, +/- 4 ms). In this case, the gNB can indicate the UE that after reception of LP-WUS the UE shall advancing/delaying the PDCCH monitoring operation. In this example, the gNB indicates the subsequent action from the UE upon the reception of LP-WUS. Furthermore, when the UE performs the above applications, the UE is expected in performing unicast transmission/reception with the gNB. Subsequently, the LP-WUS transmission in RRC connected mode is a unicast transmission between the gNB and the target UE.

Proposal 3: At least for Option 1-2, LP-WUS transmission in RRC Connected mode indicates the UE on the subsequent action to be performed upon the reception of LP-WUS.

As described above, one example in the subsequent action to be performed by the UE upon the reception of LP-WUS is to temporarily adjust the start of DRX ON operation. This is particularly to deal with the time-jitter that occurs in delay-sensitive XR application. A UE can be configured with multiple sub LP-WUS occasions (sLOs) in which each occasion is associated with the starting time of DRX ON duration as shown in Figure 2. An LO can be divided into multiple sLOs. The LO can be the same LO as the LO for the option 1. In case of option 1, the gNB transmits LP-WUS at any time within an LO. This triggers the UE to start the DRX ON at the T_reference. T_reference is basically the legacy DRX ON starting time. In case of option 1-2, the gNB shall transmit LP-WUS in one of the sLO. The LP-WUS reception in the early sLO indicates the UE to start DRX_ON starting time earlier than the T_reference as shown in  Figure 2. Here, the starting time is T_offset earlier than the T_reference. The granularity of the T_offset can be in the range of number of slots and configured by gNB.

Figure 2: Example of the implementation Option 1-2

Proposal 4: For option 1-2, support a UE to monitor multiple sub-LP WUS occasions (sLOs) in which each sLO is associated with one or more DRX parameters (e.g., the starting time of DRX ON).

There have been various legacy techniques in triggering the UE for initiating data reception, including Downlink Control information of Power saving (DCP) in legacy NR. Apart from the DCP, another technique introduced in LTE-MTC and NB-IoT is on the usage of wake-up signal (WUS) so that the PDCCH decoding for paging channel can be avoided in case the UE is not being paged. The WUS in both MTC and NB-IoT is sequence-based WUS. We consider a similar approach can be applied for NR. In our companion technical contribution [5], we described the design of LP-WUS design for NR. One main consideration aspect that is described in the WID is to specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes. Based on our analysis described in [5], the sequence-based LP-WUS operation should also be applicable for the UE operating in RRC connected mode. Another alternative being discussed in RAN1#117 is to use encoded bits. While flexible, the usage of encoded bits, however, is challenging as it may be difficult to meet the required performance resulting in high miss and false-alarm rates and thereby unnecessary high system overhead at gNB and high power consumption at the UE, respectively.  Using bitmap/encoded bits is not our preferred solution as it results in high power consumption and high system overhead. Thus, reducing the UE power saving gain. It was suggested to discuss this issue in LP-WUS & LP-SS design agenda item (A.I 9.6.1). Hence, we consider there should be a common design of LP-WUS operated in all RRC states.

Proposal 5: RAN1 strives for the LP-WUS common design including supporting sequence-based LP-WUS operation in both RRC CONNECTED and RRC idle/inactive mode.

The information on the target UE and the subsequent action from the UE (upon the reception of LP-WUS) is carried by the transmission of LP-WUS. We consider the overall equivalent information bits can be associated to the property of the LP-WUS itself and the number of time/frequency resources of that LP-WUS. In RAN1#116, it was discussed the maximum number of LP-WUS information bits. We consider this as the information bits associated to the property of the LP-WUS. The LP-WUS reception for high number of information bits may increase UE power consumption. As we propose sequence-based LP-WUS, we consider maximum of 8 bits for the LP-WUS information bits is sufficient. Note that the total-length of the sequence-based LP-WUS is not affected by the size of the information bits. The total-length of the sequence based LP-WUS may vary, such as with a long transmission in order to obtain high processing gain at the receiver. The total information bits can be more than 8 bits by considering the time/frequency resources to carry LP-WUS. Hence, a gNB can have more flexibility for the LP-WUS operation. For example, if more UEs are required then more LP-WUS resources can be configured. In RAN1#117, it was suggested that the issue on the number of information bits is to be discussed in the other agenda item (LP-WUS & LP-SS design). Hence, we consider RAN1 should aim for the common design for the LP-WUS in RRC idle/inactive and connected mode.

Proposal 6: The maximum number of LP-WUS information bits for RRC connected mode is up to 8 bits

The LP-WUS resource is allocated prior to the PDCCH resource to be monitored by the UE. In case there are multiple UEs going to monitor the same timing of PDCCH resource, the LP-WUS resource allocation prior to the PDCCH resource can be multiplexed, such as in time and/or frequency. From the UE point of view, the UE is expected to monitor a given LP-WUS resource. The UE is not expected to monitor multiple LP-WUS resources as it will increase the UE power consumption.

Proposal 7: The UE operating in RRC connected mode and configured to receive LP-WUS is expected to monitor a given LP-WUS resource (i.e., LP-WUS occasion).

During study item phase, two modes of LP-WUS operation were discussed. It includes duty-cycled monitoring and continuous monitoring mode for LP-WUS. The pros and cons of each mode were also summarized in Section 7.3.2.2 in [3]. We consider both can be applied for different applications. The UE is expected to support various applications / traffic types in RRC connected mode. Some applications may be beneficial by using one of the monitoring modes. Furthermore, the monitoring mode can be expressed as a UE capability. The LP-WUS monitoring mode can be as the UE feature. At least one of the modes is the mandatory feature while the other can be optional / mandatory (to be further discussed).

Proposal 8: In addition to duty-cycled monitoring mode, continuous monitoring mode for LP-WUS in RRC-connected is also supported.

It was agreed that the PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB via RRC signaling [6]. Furthermore, this was further discussed where there is a need to have a similar mechanism but faster than RRC signalling, which is known as LP-WUS activating/deactivating. We consider the LP-WUS enabling/disabling via RRC signalling is sufficient. After the LP-WUS is enabled, it was discussed in RAN1#118bis on the fall-back option in case the LP-WUS has been activated by the gNB via RRC signalling. One of the fallback options is for the UE to operate with legacy DRX operation.

Proposal 9: Enabling/Disabling of LP-WUS monitoring by RRC signalling is sufficient. No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring (known as Option 1).

# Conclusion

This document has considered the LP-WUS operation in RRC CONNECTED mode. We made the following proposals:

Proposal 1: Support LP-WUS in RRC connected mode with multiple minimum time-gaps depending on UE capability.

Proposal 2: Support a single time offset (FFS: explicit / implicit) that can be configured for a UE at a given time.

Proposal 3: At least for Option 1-2, LP-WUS transmission in RRC Connected mode indicates the UE on the subsequent action to be performed upon the reception of LP-WUS.

Proposal 4: For option 1-2, support a UE to monitor multiple sub-LP WUS occasions (sLOs) in which each sLO is associated with one or more DRX parameters (e.g., the starting time of DRX ON).

Proposal 4: RAN1 strives for the LP-WUS common design including supporting sequence-based LP-WUS operation in both RRC CONNECTED and RRC idle/inactive mode.

Proposal 5: The maximum number of LP-WUS information bits for RRC connected mode is up to 8 bits

Proposal 6: The UE operating in RRC connected mode and configured to receive LP-WUS is expected to monitor a given LP-WUS resource (i.e., LP-WUS occasion).

Proposal 7: In addition to duty-cycled monitoring mode, continuous monitoring mode for LP-WUS in RRC-connected is also supported.

Proposal 8: Enabling/Disabling of LP-WUS monitoring by RRC signalling is sufficient. No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring (known as Option 1).

# References

- RP-241824, “Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, RANP#105, June 2024.

- Chairman’s notes, RAN1#118bis, Hefei – China, October 2024.

- 3GPP TR 38.869 v18.0.0, “Study on low-power Wake-up Signal and Receiver for NR”

- R1-2409266, FL summary #5 on LP-WUS operation in CONNECTED mode, NTT Docomo, 1#118b, Hefei – China, October 2024.

- R1-2410231, “LP-WUS and LP-SS design”, Sony, RAN1#119, Orlando - USA, November 2024.

- Chairman’s notes, RAN1#116, Athens – Greece, Feb 2024.